| 3GPP TS 38.133 V17.11.0 (2023-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	119

1	Scope	121

2	References	121

3	Definitions, symbols and abbreviations	122

3.1	Definitions	122

3.2	Symbols	123

3.3	Abbreviations	124

3.4	Test tolerances	127

3.5	Frequency bands grouping	127

3.5.1	Introduction	127

3.5.2	NR operating bands in FR1	127

3.5.2A	NR operating bands for satellite access in FR1	128

3.5.3	NR operating bands in FR2	128

3.6	Applicability of requirements in this specification version	129

3.6.1	RRC connected state requirements in DRX	130

3.6.2	Number of serving carriers	130

3.6.2.1	Number of serving carriers for SA	130

3.6.2.2	Number of serving carriers for EN-DC	130

3.6.2.3	Number of serving carriers for NE-DC	130

3.6.2.4	Number of serving carriers for NR-DC	131

3.6.3	Applicability for intra-band FR2	131

3.6.4	Applicability for FR2 UE power classes	131

3.6.5	Applicability for SDL bands	131

3.6.6	Applicability of requirements for NGEN-DC operation	131

3.6.7	Applicability of QCL	131

3.6.9	Applicability of requirements for scheduling availability	132

3.6.10	Applicability of requirements for measurement restrictions	132

3.6.11	Applicability of requirements for Redcap UEs	132

3.6.11.1	RRC connected state requirements in DRX	132

3.6.11.2	Applicability for FR2 Redcap UE power classes	132

3.6.11.3	Applicability of QCL	132

3.6.12	Applicability of requirements for Satellite Access	133

3.6.13	Applicability of requirements for FR2	133

3.6.14	Applicability of requirements for FR2 Power Class 6	133

3.6.15	Applicability of requirements for per-FR gap	133

4	SA: RRC_IDLE state mobility	133

4.1	Cell Selection	133

4.2	Cell Re-selection	134

4.2.1	Introduction	134

4.2.2	Requirements	134

4.2.2.1	UE measurement capability	134

4.2.2.2	Measurement and evaluation of serving cell	134

4.2.2.3	Measurements of intra-frequency NR cells	137

4.2.2.4	Measurements of inter-frequency NR cells	140

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	145

4.2.2.6	Maximum interruption in paging reception	147

4.2.2.7	General requirements	148

4.2.2.8	Minimum requirement at transitions	148

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	148

4.2.2.9.1	Introduction	148

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	148

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	151

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	153

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	154

4.2.2.10.1	Introduction	154

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	154

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	157

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	160

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	160

4.2.2.11.1	Introduction	160

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	161

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	162

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	164

4.2A	Cell Re-selection when subject to CCA	165

4.2A.1	Introduction	165

4.2A.2	Requirements	165

4.2A.2.1	UE measurement capability	165

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	165

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	166

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	168

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	169

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	170

4.2A.2.7	General requirements	170

4.2B	Cell Re-selection for RedCap	170

4.2B.1	Introduction	170

4.2B.2	Requirements	170

4.2B.2.1	UE measurement capability for RedCap	170

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	170

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	171

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	171

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	173

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	175

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	178

4.2B.2.6	Maximum interruption in paging reception for RedCap	180

4.2B.2.7	General requirements for RedCap	180

4.2B.2.8	Minimum requirement at transitions	181

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	181

4.2B.2.9.1	Introduction	181

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	182

4.2B.2.9.3	Measurements for a UE fulfilling stationary and not-at-cell edge criteria	185

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criteria	185

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	185

4.2B.2.9.5	Measurements for a UE fulfilling low mobility criterion and stationary and not-at-cell edge criteria	186

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary and not-at-cell edge criteria	186

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary and notatcell edge criteria	186

4.2B.2.9.8	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and stationary criterion	186

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	187

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	190

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	192

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	192

4.2B.2.10.1	Introduction	192

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	193

4.2B.2.10.3	Measurements for a UE fulfilling stationary not at cell edge criterion	195

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	196

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	196

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	197

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	197

4.2B.2.10.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	197

4.2B.2.10.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	197

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	198

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	200

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	203

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	203

4.2B.2.11.1	Introduction	203

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	204

4.2B.2.11.3	Measurements for a UE fulfilling stationary not at cell edge criterion	205

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and Rel-16 not at cell edge criterion	206

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	206

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and stationary not at cell edge criteria	207

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge criterion and stationary not at cell edge criteria	207

4.2B.2.11.7	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary not at cell edge criteria	207

4.2B.2.11.8	Measurements for a UE fulfilling low mobility not-at-cell edge criterion and stationary criteria	207

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	208

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	209

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	210

4.2C	Cell Re-selection for NR UE for Satellite Access	211

4.2C.1	Introduction	211

4.2C.2	Requirements	211

4.2C.2.1	UE measurement capability	211

4.2C.2.2	Measurement and evaluation of serving cell	211

4.2C.2.3	Measurements of intra-frequency NR cells	212

4.2C.2.4	Measurements of inter-frequency NR cells	214

4.2C.2.5	Maximum interruption in paging reception	218

4.2C.2.6	Minimum requirement at transitions	218

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	218

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	218

4.2C.2.9	General requirements	218

4.3	Minimization of Drive Tests (MDT)	219

4.3.1	Introduction	219

4.3.2	Measurement Requirements	219

4.3.3	Requirements for Relative Time Stamp Accuracy	219

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	220

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	220

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	220

4.3C.1	Introduction	220

4.3C.2	Measurement Requirements	221

4.3C.3	Requirements for Relative Time Stamp Accuracy	221

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	221

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	221

4.4	Idle Mode CA/DC Measurements	222

4.4.1	Introduction	222

4.4.2	Measurement Requirements	222

4.4.2.1	Detected cell requirement during state transition and Idle mode	222

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	223

4.4.2.3	Measurements on serving cell	224

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	224

5	SA: RRC_INACTIVE state mobility	226

5.1	Cell Re-selection	226

5.1.1	Introduction	226

5.1.2	Requirements	226

5.1.2.1	UE measurement capability	226

5.1.2.2	Measurement and evaluation of serving cell	226

5.1.2.3	Measurements of intra-frequency NR cells	226

5.1.2.4	Measurements of inter-frequency NR cells	226

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	226

5.1.2.6	Maximum interruption in paging reception	226

5.1.2.7	General requirements	227

5.1A	Cell Re-selection with CCA	227

5.1A.1	Introduction	227

5.1A.2	Requirements	227

5.1A.2.1	UE measurement capability	227

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	227

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	227

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	227

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	227

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	227

5.1A.2.7	General requirements	227

5.1B	Cell Re-selection for RedCap	228

5.1B.1	Introduction	228

5.1B.2	Requirements	228

5.1B.2.1	UE measurement capability	228

5.1B.2.2	Measurement and evaluation of serving cell	228

5.1B.2.3	Measurements of intra-frequency NR cells	229

5.1B.2.4	Measurements of inter-frequency NR cells	230

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	231

5.1B.2.6	Maximum interruption in paging reception	231

5.1B.2.7	General requirements	231

5.1B.2.8	Minimum requirement at transitions	231

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	231

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	232

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	232

5.1C	Cell Re-selection	232

5.1C.1	Introduction	232

5.1C.2	Requirements	232

5.1C.2.1	UE measurement capability	232

5.1C.2.2	Measurement and evaluation of serving cell	232

5.1C.2.3	Measurements of intra-frequency NR cells	232

5.1C.2.4	Measurements of inter-frequency NR cells	232

5.1C.2.5	Maximum interruption in paging reception	232

5.1C.2.6	General requirements	232

5.2	Void	233

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	233

5.2B.1	Introduction	233

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	233

5.2B.2.1	Void	233

5.2B.3	TA validation requirements for RedCap	233

5.2B.2.2	Void	234

5.2B.2.3	Void	234

5.2B.4	Scheduling restriction	234

5.2B.5	Applicability conditions for CG-SDT for RedCap	234

5.3	Minimization of Drive Tests (MDT)	235

5.3.1	Introduction	235

5.3.2	Measurement Requirements	235

5.3.3	Requirements for Relative Time Stamp Accuracy	235

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	235

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	235

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	235

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	236

5.3C.1	Introduction	236

5.3C.2	Measurement Requirements	236

5.3C.3	Requirements for Relative Time Stamp Accuracy	236

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	236

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	236

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	236

5.4	Idle Mode CA/DC Measurements	237

5.4.1	Introduction	237

5.4.2	Measurement Requirements	237

5.4.2.1	Detected cell requirement during state transition and Idle mode	237

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	237

5.4.2.3	Measurements on serving cell	237

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	237

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	237

5.5.1	Introduction	237

5.5.2	Requirements on UE synchronization for small data transmissions	237

5.5.3	TA validation requirements	238

5.5.4	Scheduling restriction	238

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	238

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	239

5.5.4.3	Scheduling availability of UE performing measurements on FR2	239

5.5.5	Applicability conditions for SDT	239

5.6	NR measurements for positioning	240

5.6.1	Introduction	240

5.6.2	RSTD measurements	240

5.6.2.1	Introduction	240

5.6.2.2	Requirements Applicability	240

5.6.2.3	Measurement Capability	241

5.6.2.5	Measurements Period Requirements	241

5.6.3	PRS-RSRP measurements	244

5.6.3.1	Introduction	244

5.6.3.2	Requirements applicability	244

5.6.3.3	Measurement Capability	244

5.6.3.4	Measurement Reporting Requirements	244

5.6.3.5	Measurement Period Requirements	245

5.6.4	UE Rx-Tx time difference measurements	247

5.6.4.1	Introduction	247

5.6.4.2	Requirements Applicability	247

5.6.4.3	Measurement Capability	247

5.6.4.4	Measurement Reporting Requirements	247

5.6.4.5	Measurement Period Requirements	248

5.6.5	PRS-RSRPP measurements	250

5.6.5.1	Introduction	250

5.6.5.2	Requirements Applicability	250

5.6.5.3	Measurement Capability	250

5.6.5.4	Measurement Reporting Requirements	251

5.6.5.5	Measurement Period Requirements	251

5.7	Random access based Small Data Transmissions (RA-SDT)	251

5.7.1	Introduction	251

5.7.2	Requirements for small data transmissions based on 2-step RA	251

5.7.3	Requirements for small data transmissions based on 4-step RA	251

5.7.4	Applicability conditions for SDT	251

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	252

5.7B.1	Introduction	252

5.7B.2	Requirements for small data transmissions based on 2-step RA	252

5.7B.3	Requirements for small data transmissions based on 4-step RA	252

5.7B.4	Applicability conditions for RA-SDT for RedCap	252

6	RRC_CONNECTED state mobility	252

6.1	Handover	252

6.1.1	NR Handover	252

6.1.1.1	Introduction	252

6.1.1.2	NR FR1 - NR FR1 Handover	252

6.1.1.2.1	Handover delay	253

6.1.1.2.2	Interruption time	253

6.1.1.3	NR FR2- NR FR1 Handover	253

6.1.1.3.1	Handover delay	253

6.1.1.3.2	Interruption time	254

6.1.1.4	NR FR2- NR FR2 Handover	254

6.1.1.4.1	Handover delay	254

6.1.1.4.2	Interruption time	254

6.1.1.5	NR FR1- NR FR2 Handover	255

6.1.1.5.1	Handover delay	255

6.1.1.5.2	Interruption time	256

6.1.2	NR Handover to other RATs	256

6.1.2.1	NR – E-UTRAN Handover	256

6.1.2.1.1	Introduction	256

6.1.2.1.2	Handover delay	257

6.1.2.1.3	Interruption time	257

6.1.2.2	NR – UTRAN Handover	257

6.1.2.2.1	Introduction	257

6.1.2.2.2	Handover delay	257

6.1.2.2.3	Interruption time	257

6.1.3	NR DAPS Handover	258

6.1.3.1	Introduction	258

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	258

6.1.3.2.1	DAPS handover delay	259

6.1.3.2.2	Interruption time	259

6.1.3.3	NR FR2- NR FR1 DAPS Handover	261

6.1.3.3.1	DAPS handover delay	261

6.1.3.3.2	Interruption time	262

6.1.3.4	NR FR1- NR FR2 DAPS Handover	262

6.1.3.4.1	DAPS handover delay	263

6.1.3.4.2	Interruption time	263

6.1.4	NR Conditional Handover	263

6.1.4.1	Introduction	263

6.1.4.2	NR FR1 – NR FR1 conditional handover	264

6.1.4.3	NR FR2 – NR FR1 conditional handover	265

6.1.4.4	NR FR2 – NR FR2 conditional handover	265

6.1.4.4.1	Handover delay	265

6.1.4.4.2	Measurement time	265

6.1.4.4.3	Preparation time	266

6.1.4.4.4	Interruption time	266

6.1.4.5	NR FR1 – NR FR2 conditional handover	266

6.1.5	NR Handover with PSCell	267

6.1.5.1	Introduction	267

6.1.5.2	Handover with PSCell from NR SA to EN-DC	267

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	267

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	267

6.1.5.3	HO with PSCell from NE-DC to NE-DC	268

6.1.5.3.1	Handover delay	268

6.1.5.3.2	HO with PSCell - PCell Interruption time	268

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	269

6.1.5.4	HO with PSCell from NR-DC to NR-DC	269

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	270

6.1.5.5.1	Introduction	270

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	270

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	271

6.1A	Void	272

6.1A.1	Void	272

6.1A.1.1	Void	272

6.1A.1.2	Void	272

6.1A.1.2.1	Void	272

6.1A.1.2.2	Void	272

6.1B	Handover to target cell using CCA	272

6.1B.1	NR Handover	272

6.1B.1.1	Introduction	272

6.1B.1.2	NR FR1 - NR FR1 Handover	272

6.1B.1.2.1	Handover delay	272

6.1B.1.2.2	Interruption time	273

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	273

6.1B.1.3.1	Handover delay	273

6.1B.1.3.2	Interruption time	274

6.1B.1.4	NR FR1- NR FR2-2 Handover	274

6.1B.1.4.1	Handover delay	275

6.1B.1.4.2	Interruption time	275

6.1C	Handover for SAN	276

6.1C.1	NR SAN Handover	276

6.1C.1.1	Introduction	276

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	276

6.1C.1.2.1	Handover delay	276

6.1C.1.2.2	Interruption time	276

6.1C.2	NR SAN Conditional Handover	277

6.1C.2.1	Introduction	277

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	277

6.1C.2.2.1	Handover delay	277

6.1C.2.2.2	Measurement time	278

6.1C.2.2.3	Preparation time	279

6.1C.2.2.4	Interruption time	279

6.1D	Handover for RedCap	279

6.1D.1	NR Handover	279

6.1D.1.1	Introduction	279

6.1D.1.2	NR FR1 - NR FR1 Handover	280

6.1D.1.2.1	Handover delay	280

6.1D.1.2.2	Interruption time	280

6.1D.1.3	NR FR2- NR FR2 Handover	281

6.1D.1.3.1	Handover delay	281

6.1D.1.3.2	Interruption time	281

6.1D.2	NR Handover to other RATs	283

6.1D.2.1	NR – E-UTRAN Handover	283

6.2	RRC Connection Mobility Control	283

6.2.1	SA: RRC Re-establishment	283

6.2.1.1	Introduction	283

6.2.1.2	Requirements	283

6.2.1.2.1	UE Re-establishment delay requirement	283

6.2.1A	RRC Re-establishment with CCA	285

6.2.1A.1	Introduction	285

6.2.1A.2	Requirements	285

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	286

6.2.1B	SA: RRC Re-establishment for RedCap	287

6.2.1B.1	Introduction	287

6.2.1B.2	Requirements	288

6.2.2	Random access	288

6.2.2.1	Introduction	288

6.2.2.2	Requirements for 4-step RA type	288

6.2.2.2.1	Contention based random access	289

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	289

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	289

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	289

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	289

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	289

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	289

6.2.2.2.2	Non-Contention based random access	289

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	289

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	290

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	290

6.2.2.2.3	UE behaviour when configured with supplementary UL	290

6.2.2.3	Requirements for 2-step RA type	290

6.2.2.3.1	Contention based random access	291

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	291

6.2.2.3.1.2	Correct behaviour when receiving MsgB	291

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	291

6.2.2.3.2	Non-Contention based random access	292

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	292

6.2.2.3.2.2	Correct behaviour when receiving MsgB	292

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	292

6.2.2.3.3	UE behaviour when configured with supplementary UL	292

6.2.2A	Random access when CCA is used on target frequency	292

6.2.2A.1	Introduction	292

6.2.2A.2	Requirements for 4-step RA type	293

6.2.2A.2.1	Contention based random access	293

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	293

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	293

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	293

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	294

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	294

6.2.2A.2.2	Non-Contention based random access	294

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	294

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	294

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	295

6.2.2A.3	Requirements for 2-step RA type	295

6.2.2A.3.1	Contention based random access	295

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	295

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	296

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	296

6.2.2A.3.2	Non-Contention based random access	296

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	296

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	297

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	297

6.2.2B	Random access for RedCap	297

6.2.2B.1	Introduction	297

6.2.2B.2	Requirements	298

6.2.3	SA: RRC Connection Release with Redirection	298

6.2.3.1	Introduction	298

6.2.3.2	Requirements	298

6.2.3.2.1	RRC connection release with redirection to NR	298

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	299

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	300

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	301

6.2.3A.1	Introduction	301

6.2.3A.2	Requirements	301

6.2.3A.2.1	RRC connection release with redirection to NR	301

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	301

6.2C	RRC Connection Mobility Control for Satellite Access	301

6.2C.1	SA: RRC Re-establishment for Satellite Access	301

6.2C.1.1	Introduction	301

6.2C.1.2	Requirements	302

6.2C.1.2.1	UE Re-establishment delay requirement	302

6.2C.2	Random access for satellite access	303

6.2C.2.1	Introduction	303

6.2C.2.2	Requirements for 4-step RA type	303

6.2C.2.2.1	Contention based random access	304

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	304

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	304

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	304

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	304

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	304

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	304

6.2C.2.2.2	Non-Contention based random access	304

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	304

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	305

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	305

6.2C.2.3	Requirements for 2-step RA type	305

6.2C.2.3.1	Contention based random access	306

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	306

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	306

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	306

6.2C.2.3.2	Non-Contention based random access	306

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	306

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	307

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	307

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	307

6.2C.3.1	Introduction	307

6.2C.3.2	Requirements	307

6.2C.3.2.1	RRC connection release with redirection to NR	307

7	Timing	308

7.1	UE transmit timing	308

7.1.1	Introduction	308

7.1.2	Requirements	308

7.1.2.1	Gradual timing adjustment	310

7.1.2.2	Void	311

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	311

7.1A	UE transmit timing for RedCap	311

7.1A.1	Introduction	311

7.1A.2	Requirements	312

7.1A.2.1	Gradual timing adjustment	313

7.1C	UE transmit timing for Satellite Access	313

7.1C.1	Introduction	313

7.1C.2	Requirements	313

7.1C.2.1	Gradual timing adjustment	314

7.2	UE timer accuracy	315

7.2.1	Introduction	315

7.2.2	Requirements	315

7.2A	UE timer accuracy for RedCap	315

7.2A.1	Introduction	315

7.2A.2	Requirements	315

7.2C	UE timer accuracy for satellite access	315

7.2C.1	Introduction	315

7.2C.2	Requirements	316

7.3	Timing advance	316

7.3.1	Introduction	316

7.3.2	Requirements	316

7.3.2.1	Timing Advance adjustment delay	316

7.3.2.2	Timing Advance adjustment accuracy	316

7.3A	Timing Advance for RedCap	316

7.3A.1	Introduction	316

7.3A.2	Requirements	317

7.3A.2.1	Timing Advance adjustment delay	317

7.3A.2.2	Timing Advance adjustment accuracy	317

7.3C	Timing advance for satellite access	317

7.3C.1	Introduction	317

7.3C.2	Requirements	317

7.3C.2.1	Timing Advance adjustment delay	317

7.3C.2.2	Timing Advance adjustment accuracy	317

7.4	Cell phase synchronization accuracy	318

7.4.1	Definition	318

7.4.2	Minimum requirements	318

7.5	Maximum Transmission Timing Difference	318

7.5.1	Introduction	318

7.5.2	Minimum Requirements for inter-band EN-DC	318

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	319

7.5.3	Minimum Requirements for intra-band EN-DC	319

7.5.4	Minimum Requirements for NR Carrier Aggregation	319

7.5.5	Minimum Requirements for inter-band NE-DC	320

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	320

7.5.6	Minimum Requirements for inter-band NR DC	321

7.6	Maximum Receive Timing Difference	321

7.6.1	Introduction	321

7.6.2	Minimum Requirements for inter-band EN-DC	322

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	322

7.6.3	Minimum Requirements for intra-band EN-DC	323

7.6.4	Minimum Requirements for NR Carrier Aggregation	323

7.6.5	Minimum Requirements for inter-band NE-DC	324

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	324

7.6.6	Minimum Requirements for inter-band NR DC	324

7.7	deriveSSB-IndexFromCell tolerance	325

7.7.1	Minimum requirements	325

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	326

7.7A.1	Minimum requirements	326

7.8	Void	326

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	326

7.9.1	Minimum requirements	326

8	Signalling characteristics	327

8.1	Radio Link Monitoring	327

8.1.1	Introduction	327

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	328

8.1.2	Requirements for SSB based radio link monitoring	328

8.1.2.1	Introduction	329

8.1.2.2	Minimum requirement	329

8.1.2.3	Measurement restrictions for SSB based RLM	333

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	333

8.1.3	Requirements for CSI-RS based radio link monitoring	334

8.1.3.1	Introduction	334

8.1.3.2	Minimum requirement	335

8.1.3.3	Measurement restrictions for CSI-RS based RLM	338

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	339

8.1.4	Minimum requirement at transitions	340

8.1.5	Minimum requirement for UE turning off the transmitter	340

8.1.6	Minimum requirement for L1 indication	340

8.1.7	Scheduling availability of UE during radio link monitoring	341

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	341

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	341

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	341

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	342

8.1A	Radio Link Monitoring with CCA on Target Frequency	342

8.1A.1	Introduction	342

8.1A.2	Requirements for SSB Based Radio Link Monitoring	343

8.1A.2.1	Introduction	343

8.1A.2.2	Minimum Requirement	344

8.1A.3	Minimum requirement at transitions	347

8.1A.4	Minimum requirement for UE turning off the transmitter	348

8.1A.5	Minimum requirement for L1 indication	348

8.1A.6	Scheduling availability of UE during radio link monitoring	348

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	348

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	349

8.1B	Radio Link Monitoring for RedCap	349

8.1B.1	Introduction	349

8.1B.2	Requirements for SSB based radio link monitoring	350

8.1B.2.1	Introduction	350

8.1B.2.2	Minimum requirement	351

8.1B.2.3	Measurement restrictions for SSB based RLM	353

8.1B.3	Requirements for CSI-RS based radio link monitoring	354

8.1B.3.1	Introduction	354

8.1B.3.2	Minimum requirement	354

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	357

8.1B.4	Minimum requirement at transitions	358

8.1B.5	Minimum requirement for UE turning off the transmitter	358

8.1B.6	Minimum requirement for L1 indication	358

8.1B.7	Scheduling availability of UE during radio link monitoring	358

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	359

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	359

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	359

8.1C	Radio Link Monitoring for Satellite Access	359

8.1C.1	Introduction	359

8.1C.2	Requirements for SSB based radio link monitoring	360

8.1C.2.1	Introduction	360

8.1C.2.2	Minimum requirement	361

8.1C.2.3	Measurement restrictions for SSB based RLM	362

8.1C.3	Requirements for CSI-RS based radio link monitoring	362

8.1C.3.1	Introduction	362

8.1C.3.2	Minimum requirement	363

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	364

8.1C.4	Minimum requirement at transitions	365

8.1C.5	Minimum requirement for UE turning off the transmitter	365

8.1C.6	Minimum requirement for L1 indication	365

8.1C.7	Scheduling availability of UE during radio link monitoring	365

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	366

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	366

8.2	Interruption	366

8.2.1	EN-DC Interruption	366

8.2.1.1	Introduction	366

8.2.1.2	Requirements	367

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	367

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	367

8.2.1.2.3	Interruptions at SCell addition/release	367

8.2.1.2.4	Interruptions at SCell activation/deactivation	368

8.2.1.2.5	Interruptions during measurements on SCC	370

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	370

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	370

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	370

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	371

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	371

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	372

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	373

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	373

8.2.1.2.8.2	Interruptions during direct SCell activation	373

8.2.1.2.9	Interruptions at SCell hibernation	373

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	374

8.2.1.2.11	Interruptions due to UE-specific CBW change	374

8.2.1.2.12	Interruptions at NR SRS carrier based switching	374

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	376

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	377

8.2.1.2.15	Interruptions due to SCell dormancy	377

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	377

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	377

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	378

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	378

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	378

8.2.1.2.18	Interruptions at NR SRS antenna port switching	379

8.2.1.2.19	Interruptions at fast SCell activation	380

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	381

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	381

8.2.2.1	Introduction	381

8.2.2.2	Requirements	382

8.2.2.2.1	Interruptions at SCell addition/release	382

8.2.2.2.2	Interruptions at SCell activation/deactivation	383

8.2.2.2.3	Interruptions during measurements on deactivated SCC	384

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	385

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	385

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	386

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	388

8.2.2.2.8	Interruptions due to UE-specific CBW change	388

8.2.2.2.9	Interruptions at NR SRS carrier based switching	388

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	390

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	390

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	391

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	391

8.2.2.2.11	Interruptions at direct SCell activation	391

8.2.2.2.12	Interruptions due to SCell dormancy	392

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	392

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	392

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	392

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	392

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	392

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	393

8.2.2.2.16	Interruptions at NR SRS antenna port switching	393

8.2.2.2.17	Interruptions at fast SCell activation	394

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	395

8.2.3	NE-DC Interruptions	395

8.2.3.1	Introduction	395

8.2.3.2	Requirements	396

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	396

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	396

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	396

8.2.3.2.4	Interruptions at SCell activation/deactivation	398

8.2.3.2.5	Interruptions during measurements on SCC	399

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	399

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	399

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	400

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	400

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	400

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	401

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	401

8.2.3.2.9	Interruptions at SCell hibernation	401

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	402

8.2.3.2.11	Interruptions at NR SRS carrier based switching	402

8.2.3.2.12	Interruptions at E-UTRA SRS carrier based switching	403

8.2.3.2.13	Interruptions due to SCell dormancy	404

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	405

8.2.3.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	405

8.2.3.2.17	Interruptions at fast SCell activation	407

8.2.3.2.18	Interruptions due to UE-specific CBW change	407

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	408

8.2.4	NR-DC: Interruptions	408

8.2.4.1	Introduction	408

8.2.4.2	Requirements	409

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	409

8.2.4.2.2	Interruptions at SCell activation/deactivation	410

8.2.4.2.3	Interruptions during measurements on SCC	411

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	411

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	412

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	412

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	412

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	413

8.2.4.2.9	Interruptions at NR SRS carrier based switching	413

8.2.4.2.10	Interruptions at direct SCell activation	414

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	415

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	415

8.2.4.2.13	Interruptions due to SCell dormancy	416

8.2.4.2.14	Interruptions at NR SRS antenna port switching	416

8.2.4.2.15	Interruptions at fast SCell activation	417

8.2.4.2.16	Interruptions at SCG activation/deactivation	418

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	418

8.2.4.2.18	Interruptions during RLM/BFD measurements on deactivated PSCell	419

8.2.4.2.19	Interruptions due to UE-specific CBW change	419

8.2.4.2A	Void	419

8.2.4.2A.1	Void	419

8.2.4.2A.2	Void	419

8.2.4.2A.3	Void	419

8.3	SCell Activation and Deactivation Delay	419

8.3.1	Introduction	419

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	419

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	424

8.3.4	Direct SCell Activation at SCell addition	424

8.3.5	Direct SCell Activation at Handover	426

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	427

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	431

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	432

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	433

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	434

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	437

8.3.13.1	Introduction	437

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	439

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	439

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	440

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	442

8.3A.1	Introduction	442

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	443

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	447

8.4	UE UL carrier RRC reconfiguration delay	447

8.4.1	Introduction	447

8.4.2	UE UL carrier configuration delay requirement	448

8.4.3	UE UL carrier deconfiguration delay requirement	448

8.5	Link Recovery Procedures	448

8.5.1	Introduction	448

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	449

8.5.2	Requirements for SSB based beam failure detection	450

8.5.2.1	Introduction	450

8.5.2.2	Minimum requirement	450

8.5.2.3	Measurement restriction for SSB based beam failure detection	453

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	454

8.5.3	Requirements for CSI-RS based beam failure detection	455

8.5.3.1	Introduction	455

8.5.3.2	Minimum requirement	455

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	459

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	460

8.5.4	Minimum requirement for L1 indication	460

8.5.5	Requirements for SSB based candidate beam detection	461

8.5.5.1	Introduction	461

8.5.5.2	Minimum requirement	461

8.5.5.3	Measurement restriction for SSB based candidate beam detection	464

8.5.6	Requirements for CSI-RS based candidate beam detection	464

8.5.6.1	Introduction	464

8.5.6.2	Minimum requirement	465

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	468

8.5.7	Scheduling availability of UE during beam failure detection	468

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	468

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	469

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	469

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	470

8.5.8	Scheduling availability of UE during candidate beam detection	470

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	470

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	470

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	470

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	471

8.5.9	Requirements for Beam Failure Recovery in SCell	471

8.5.9.1	Introduction	471

8.5.9.2	Requirement	471

8.5.10	Minimum requirement at transitions for beam failure detection	471

8.5A	Link Recovery Procedures when CCA is used on target frequency	472

8.5A.1	Introduction	472

8.5A.2	Requirements for SSB based beam failure detection	473

8.5A.2.1	Introduction	473

8.5A.2.2	Minimum requirement	473

8.5A.2.3	Measurement restriction for SSB based beam failure detection	475

8.5A.3	Void	475

8.5A.4	Minimum requirement for L1 indication	475

8.5A.5	Requirements for SSB based candidate beam detection	476

8.5A.5.1	Introduction	476

8.5A.5.2	Minimum requirement	476

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	478

8.5A.6	Void	479

8.5A.7	Scheduling availability of UE during beam failure detection	479

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	479

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	479

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	479

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	479

8.5A.8	Scheduling availability of UE during candidate beam detection	479

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	479

8.5A.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	480

8.5B	Link Recovery Procedures for Redcap	480

8.5B.1	Introduction	480

8.5B.2	Requirements for SSB based beam failure detection for Redcap	480

8.5B.2.1	Introduction	480

8.5B.2.2	Minimum requirement	481

8.5B.2.3	Measurement restriction for SSB based beam failure detection	482

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	483

8.5B.3.1	Introduction	483

8.5B.3.2	Minimum requirement	483

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	485

8.5B.4	Minimum requirement for L1 indication for Redcap	486

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	487

8.5B.5.1	Introduction	487

8.5B.5.2	Minimum requirement	487

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	489

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	489

8.5B.6.1	Introduction	489

8.5B.6.2	Minimum requirement	489

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	491

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	492

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	492

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	492

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	492

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	493

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	493

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	493

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	493

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	493

8.5C	Link Recovery Procedures for Satellite Access	494

8.5C.1	Introduction	494

8.5C.2	Requirements for SSB based beam failure detection	494

8.5C.2.1	Introduction	494

8.5C.2.2	Minimum requirement	495

8.5C.2.3	Measurement restriction for SSB based beam failure detection	495

8.5C.3	Requirements for CSI-RS based beam failure detection	496

8.5C.3.1	Introduction	496

8.5C.3.2	Minimum requirement	496

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	497

8.5C.4	Minimum requirement for L1 indication	498

8.5C.5	Requirements for SSB based candidate beam detection	498

8.5C.5.1	Introduction	498

8.5C.5.2	Minimum requirement	498

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	499

8.5C.6	Requirements for CSI-RS based candidate beam detection	500

8.5C.6.1	Introduction	500

8.5C.6.2	Minimum requirement	500

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	501

8.5C.7	Scheduling availability of UE during beam failure detection	501

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	501

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	501

8.5C.8	Scheduling availability of UE during candidate beam detection	502

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	502

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	502

8.5C.9	Minimum requirement at transitions for beam failure detection	502

8.6	Active BWP switch delay	502

8.6.1	Introduction	502

8.6.2	DCI and timer based BWP switch delay on a single CC	503

8.6.2A	DCI based BWP switch delay on multiple CCs	504

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	504

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	506

8.6.2B	Timer based BWP switch delay on multiple CCs	506

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	506

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	506

8.6.3	RRC based BWP switch delay on a single CC	507

8.6.3A	RRC based BWP switch delay on multiple CCs	508

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	508

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	508

8.6.4	BWP switch delay on Consistent UL CCA recovery	509

8.6A	Active BWP switch delay for RedCap	509

8.6A.1	Introduction	509

8.6A.2	DCI and timer based BWP switch delay on a single CC	509

8.6A.3	RRC based BWP switch delay on a single CC	511

8.6C	Active BWP switch delay for satellite access	511

8.6C.1	Introduction	511

8.6C.2	DCI and timer based BWP switch delay on a single CC	511

8.6C.3	RRC based BWP switch delay on a single CC	513

8.7	Void	513

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	513

8.8.1	Introduction	513

8.8.2	E-UTRAN PSCell Addition Delay Requirement	513

8.8.3	E-UTRAN PSCell Release Delay Requirement	514

8.9	NR-DC: PSCell Addition and Release Delay	514

8.9.1	Introduction	514

8.9.2	PSCell Addition Delay Requirement	514

8.9.3	PSCell Release Delay Requirement	515

8.9A	Conditional PSCell Addition Delay	515

8.9A.1	Introduction	515

8.9A.2	Conditional PSCell Addition Delay Requirement	515

8.9A.2.1	Measurement time	516

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	516

8.9B.1	Introduction	516

8.9B.2	PSCell Addition Delay Requirement	517

8.9B.3	PSCell Release Delay Requirement	517

8.10	Active TCI state switching delay	518

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	519

8.10.4	DCI based TCI state switch delay	519

8.10.5	RRC based TCI state switch delay	520

8.10.6	Active TCI state list update delay	520

8.10A	Active TCI state switching delay with CCA	520

8.10A.1	Introduction	520

8.10A.2	Known conditions for TCI state	521

8.10A.3	MAC-CE based TCI state switch delay	521

8.10A.4	DCI based TCI state switch delay	522

8.10A.5	RRC based TCI state switch delay	522

8.10A.6	Active TCI state list update delay	523

8.10B	Active TCI state switching delay for RedCap	523

8.10B.1	Introduction	523

8.10B.2	Known conditions for TCI state	523

8.10B.3	MAC-CE based TCI state switch delay	524

8.10B.4	DCI based TCI state switch delay	524

8.10B.5	RRC based TCI state switch delay	525

8.10B.6	Active TCI state list update delay	525

8.10C	Active TCI state switching delay for satellite access	526

8.10C.1	Introduction	526

8.10C.2	MAC-CE based TCI state switch delay	526

8.10C.4	DCI based TCI state switch delay	526

8.10C.5	RRC based TCI state switch delay	526

8.10C.6	Active TCI state list update delay	526

8.11	PSCell Change	527

8.11A	PSCell Change in Carriers with CCA	527

8.11B	Conditional PSCell Change	527

8.11B.1	Introduction	527

8.11B.2	Conditoinal PSCell Change delay	528

8.11B.2.1	Measurement time	528

8.11D	Conditional PSCell Change in Carriers with CCA	529

8.11D.1	Introduction	529

8.11D.2	Conditional PSCell Change delay	529

8.11D.2.1	Measurement time	530

8.12	Uplink spatial relation switch delay	530

8.12.1	Introduction	530

8.12.2	Known conditions for spatial relation when associated with DL-RS	530

8.12.3	MAC-CE based spatial relation switch delay	531

8.12.4	DCI based spatial relation switch delay	531

8.12.5	RRC based spatial relation switch delay	532

8.12A	Uplink spatial relation switch delay for RedCap	532

8.12A.1	Introduction	532

8.12A.2	Known conditions for spatial relation when associated with DL-RS	532

8.12A.3	MAC-CE based spatial relation switch delay	533

8.12A.4	DCI based spatial relation switch delay	533

8.12A.5	RRC based spatial relation switch delay	534

8.12C	Uplink spatial relation switch delay for satellite access	534

8.12C.1	Void	535

8.12C.2	Void	535

8.12C.3	Void	535

8.12C.4	Void	535

8.12C.5	Void	535

8.13	UE-specific CBW change	535

8.13.1	Introduction	535

8.13.2	UE-specific CBW change delay	535

8.13A	UE-specific CBW change for RedCap	535

8.13A.1	Introduction	535

8.13A.2	UE-specific CBW change delay	535

8.13C	UE-specific CBW change for satellite access	536

8.13C.1	Introduction	536

8.13C.2	UE-specific CBW change delay	536

8.14	Pathloss reference signal switching delay	536

8.14.1	Introduction	536

8.14.2	Known conditions for pathloss reference signal	537

8.14.3	MAC-CE based pathloss reference signal switch delay	537

8.14C	Pathloss reference signal switching delay for satellite access	538

8.14C.1	Introduction	538

8.14C.2	Known conditions for pathloss reference signal	538

8.14C.3	MAC-CE based pathloss reference signal switch delay	538

8.15	Active downlink TCI state switching delay for unified TCI	539

8.15.1	Introduction	539

8.15.4	DCI based downlink TCI state switch delay	541

8.15.5	Active Downlink TCI state list update delay	541

8.16	Active uplink TCI state switching delay for unified TCI	542

8.16.1	Introduction	542

8.16.4	DCI based uplink TCI state switch delay	544

8.16.5	Active Uplink TCI state list update delay	545

8.17	SCG Activation and Deactivation Delay	546

8.17.1	Introduction	546

8.17.2	SCG Activation Delay Requirement	546

8.17.3	SCG Deactivation Delay Requirement	547

8.18	TRP specific Link Recovery Procedures	548

8.18.1	Introduction	548

8.18.2	Requirements for TRP specific SSB based beam failure detection	548

8.18.2.1	Introduction	548

8.18.2.2	Minimum requirement	549

8.18.2.3	Measurement restriction for SSB based beam failure detection	551

8.18.3	Requirements for CSI-RS based beam failure detection	551

8.18.3.1	Introduction	551

8.18.3.2	Minimum requirement	552

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	554

8.18.4	Minimum requirement for L1 indication	555

8.18.5	Requirements for SSB based candidate beam detection	555

8.18.5.1	Introduction	555

8.18.5.2	Minimum requirement	555

8.18.5.3	Measurement restriction for SSB based candidate beam detection	557

8.18.6	Requirements for CSI-RS based candidate beam detection	558

8.18.6.1	Introduction	558

8.18.6.2	Minimum requirement	558

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	560

8.18.7	Requirements for TRP specific Beam Failure Recovery	561

8.18.7.1	Introduction	561

8.18.7.2	Requirement	561

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	561

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	561

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	562

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	562

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	563

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	563

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	563

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	563

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	563

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	564

8.19	Pre-configured measurement gap activation/deactivation delay	564

8.19.1	Introduction	564

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	564

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	564

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	564

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	565

9	Measurement Procedure	566

9.1	General measurement requirement	566

9.1.1	Introduction	566

9.1.2	Measurement gap	566

9.1.2.1	EN-DC: Measurement Gap Sharing	578

9.1.2.1a	SA: Measurement Gap Sharing	579

9.1.2.1b	NE-DC: Measurement Gap Sharing	580

9.1.2.1c	NR-DC: Measurement Gap Sharing	581

9.1.3	UE Measurement capability	582

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	582

9.1.3.1a	SA: Monitoring of multiple layers using gaps	582

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	583

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	583

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	584

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	585

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	585

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	586

9.1A.3.2	Void	587

9.1.3A	UE Measurement capability under operation mode with CCA	587

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	587

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	587

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	587

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	588

9.1.3C	UE Measurement capability under operation mode with satellite access	588

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	588

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	589

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	589

9.1.4.1	Introduction	589

9.1.4.2	Requirements	589

9.1.5	Carrier-specific scaling factor	593

9.1.5.1	Monitoring of multiple layers outside gaps	594

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	596

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	597

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	598

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	598

9.1.5.2	Monitoring of multiple layers within gaps	599

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	601

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	603

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	605

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	607

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	608

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	609

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	609

9.1.5.3	Monitoring of multiple layers within NCSG	609

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	610

9.1.6	Minimum requirement at transitions	610

9.1.7	Pre-configured measurement gap	611

9.1.7.1	Introduction	611

9.1.7.2	Requirements applicability	611

9.1.7.3	Requirements	611

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	612

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	612

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	613

9.1.8	Concurrent measurement gaps	613

9.1.8.1	Introduction	613

9.1.8.2	Requirements	613

9.1.8.3	Collision between concurrent measurement gaps	615

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	615

9.1.9	Network controlled small gap	615

9.1.9.1	Introduction	615

9.1.9.2	Requirements applicability	616

9.1.10	MUSIM gaps	618

9.1.11	UL gap for Tx power management	619

9.1A	General measurement requirement for RedCap	620

9.1A.1	Introduction	620

9.1A.2	Measurement gap	620

9.1A.2.1	SA: Measurement Gap Sharing	624

9.1A.3	UE Measurement capability	625

9.1A.3.1	SA: Monitoring of multiple layers using gaps	625

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	625

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	625

9.1A.4.1	Introduction	625

9.1A.4.2	Requirements	626

9.1A.5	Carrier-specific scaling factor	626

9.1A.5.1	Monitoring of multiple layers outside gaps	626

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	627

9.1A.5.2	Monitoring of multiple layers within gaps	627

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	627

9.1A.6	Minimum requirement at transitions	628

9.1C	General measurement requirement for SAN	629

9.1C.1	Introduction	629

9.1C.2	Measurement gap	629

9.1C.8	Concurrent measurement gaps for SAN	631

9.1C.8.1	Introduction	631

9.1C.8.2	Requirements	631

9.1C.8.3	Collision between concurrent measurement gaps	631

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	632

9.1C.9	Collision between SMTC and measurement gap for SAN	632

9.1C.9.1	Introduction	632

9.1C.9.2	Collision between SMTCs and measurement gap	632

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	633

9.2	NR intra-frequency measurements	633

9.2.1	Introduction	633

9.2.2	Requirements applicability	634

9.2.3	Number of cells and number of SSB	634

9.2.3.1	Requirements for FR1	634

9.2.3.2	Requirements for FR2	635

9.2.4	Measurement Reporting Requirements	635

9.2.4.1	Periodic Reporting	635

9.2.4.2	Event-triggered Periodic Reporting	635

9.2.4.3	Event Triggered Reporting	635

9.2.5	Intrafrequency measurements without measurement gaps	636

9.2.5.1	Intrafrequency cell identification	636

9.2.5.2	Measurement period	641

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	644

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	644

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	644

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	645

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	646

9.2.5.4	SFTD Measurements between PCell and PSCell	647

9.2.5.4.1	Introduction	647

9.2.5.4.2	SFTD Measurement delay	647

9.2.5.4.3	SFTD Measurement Reporting Delay	647

9.2.6	Intra-frequency measurements with measurement gaps	648

9.2.6.1	Void	648

9.2.6.2	Intra-frequency cell identification	648

9.2.6.3	Intrafrequency Measurement Period	650

9.2.7	Intra-frequency measurements with NCSG	652

9.2.7.1	Intra-frequency cell identification	652

9.2.7.2	Measurement period	654

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	655

9.2A	NR intra-frequency measurements with CCA	655

9.2A.1	Introduction	655

9.2A.2	Requirements applicability	656

9.2A.3	Number of cells and number of SSB	656

9.2A.3.1	Requirements for FR1	656

9.2A.3.2	Requirements for FR2-2	657

9.2A.4	Measurement Reporting Requirements	657

9.2A.5	Intra-frequency measurements without measurement gaps	658

9.2A.5.2	Measurement period	662

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	664

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	665

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	665

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	665

9.2A.6	Intra-frequency measurements with measurement gaps	665

9.2A.6.1	Intra-frequency cell identification	665

9.2A.6.2	Intra-frequency Measurement Period	668

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	669

9.2A.7.1	Intra-frequency RSSI measurements	669

9.2A.7.2	Intra-frequency Channel occupancy measurements	671

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	672

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	672

9.2B	NR intra-frequency measurements for RedCap	673

9.2B.1	Introduction	673

9.2B.2	Requirements applicability	673

9.2B.3	Number of cells and number of SSB	674

9.2B.3.1	Requirements for FR1	674

9.2B.3.2	Requirements for FR2	674

9.2B.4	Measurement Reporting Requirements	674

9.2B.4.1	Periodic Reporting	674

9.2B.4.2	Event-triggered Periodic Reporting	674

9.2B.4.3	Event Triggered Reporting	674

9.2B.5	Intra-frequency measurements without measurement gaps	675

9.2B.5.1	Intra-frequency cell identification	675

9.2B.5.2	Measurement period	677

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	678

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	678

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	678

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	679

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	679

9.2B.6	Intra-frequency measurements with measurement gaps	680

9.2B.6.1	Intra-frequency cell identification	680

9.2B.6.2	Intra-frequency Measurement Period	681

9.2C	NR intra-frequency measurements for SAN	682

9.2C.1	Introduction	682

9.2C.2	Requirements applicability	683

9.2C.3	Number of cells and number of SSB	683

9.2C.3.1	Requirements for FR1	683

9.2C.4	Measurement Reporting Requirements	683

9.2C.4.1	Periodic Reporting	683

9.2C.4.2	Event-triggered Periodic Reporting	684

9.2C.4.3	Event Triggered Reporting	684

9.2C.5	Intra frequency measurements without measurement gaps	684

9.2C.5.1	Intra frequency cell identification	684

9.2C.5.2	Measurement period	686

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	687

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	687

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	687

9.2C.6	Intra-frequency measurements with measurement gaps	688

9.2C.6.1	Intra-frequency cell identification	688

9.2C.6.3	Intrafrequency Measurement Period	689

9.3	NR inter-frequency measurements	689

9.3.1	Introduction	689

9.3.2	Requirements applicability	690

9.3.2.1	Void	691

9.3.2.2	Void	691

9.3.3	Number of cells and number of SSB	691

9.3.3.1	Requirements for FR1	691

9.3.3.2	Requirements for FR2	691

9.3.4	Inter-frequency measurement with measurement gaps	691

9.3.4.1	Void	695

9.3.4.2	Void	695

9.3.5	Inter-frequency measurements	695

9.3.5.1	Void	696

9.3.5.2	Void	696

9.3.5.3	Void	696

9.3.6	Inter-frequency measurements reporting requirements	696

9.3.6.1	Periodic Reporting	696

9.3.6.2	Event-triggered Periodic Reporting	696

9.3.6.3	Event-triggered Reporting	696

9.3.7	Void	697

9.3.8	Inter-frequency SFTD measurement requirements	697

9.3.8.1	Introduction	697

9.3.8.2	SFTD Measurement delay	697

9.3.8.3	SFTD Measurement reporting delay	698

9.3.9	Inter frequency measurements without measurement gaps	698

9.3.9.1	Inter frequency Cell identification	698

9.3.9.2	Measurement period	701

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	702

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	703

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	703

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	704

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	704

9.3.10	Inter-frequency measurement with NCSG	704

9.3.10.1	Inter-frequency cell identification	704

9.3.10.2	Measurement period	706

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	706

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	706

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	707

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	707

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	709

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	710

9.3A.1	Introduction	710

9.3A.2	Requirements applicability	710

9.3A.3	Number of cells and number of SSB	711

9.3A.3.1	Requirements for FR1	711

9.3A.3.2	Requirements for FR2-2	711

9.3A.4	Inter-frequency cell identification	711

9.3A.5	Inter-frequency measurements	713

9.3A.6	NR Inter-frequency measurements reporting requirements	715

9.3A.6.1	Periodic Reporting	715

9.3A.6.2	Event-triggered Periodic Reporting	715

9.3A.6.3	Event-triggered Reporting	715

9.3A.8	Inter-frequency RSSI measurements	715

9.3A.9	Inter-frequency channel occupancy measurements	716

9.3B	NR inter-frequency measurements for RedCap	717

9.3B.1	Introduction	717

9.3B.2	Requirements applicability	717

9.3B.3	Number of cells and number of SSB	718

9.3B.3.1	Requirements for FR1	718

9.3B.3.2	Requirements for FR2	718

9.3B.4	Inter-frequency measurement with measurement gaps	718

9.3B.5	Inter-frequency measurements	719

9.3B.6	Inter-frequency measurements reporting requirements	720

9.3B.6.1	Periodic Reporting	720

9.3B.6.2	Event-triggered Periodic Reporting	720

9.3B.6.3	Event-triggered Reporting	720

9.3B.7	Inter frequency measurements without measurement gaps	721

9.3B.7.1	Inter frequency Cell identification	721

9.3B.7.2	Measurement period	723

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	724

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	724

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	725

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	725

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	725

9.3C	NR inter-frequency measurements for SAN	726

9.3C.1	Introduction	726

9.3C.2	Requirements applicability	726

9.3C.3	Number of cells and number of SSB	726

9.3C.3.1	Requirements for FR1	726

9.3C.4	Inter-frequency measurement with measurement gaps	727

9.3C.5	Inter-frequency measurements	728

9.3C.6	Inter-frequency measurements reporting requirements	728

9.3C.6.1	Periodic Reporting	728

9.3C.6.2	Event-triggered Periodic Reporting	728

9.3C.6.3	Event-triggered Reporting	729

9.3C.7	Inter frequency measurements without measurement gaps	729

9.3C.7.1	Inter frequency Cell identification	729

9.3C.7.2	Measurement period	731

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	731

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	731

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	732

9.4	Inter-RAT measurements	732

9.4.1	Introduction	732

9.4.2	NR − E-UTRAN FDD measurements	734

9.4.2.1	Introduction	734

9.4.2.2	Requirements when no DRX is used	734

9.4.2.3	Requirements when DRX is used	735

9.4.2.4	Measurement reporting requirements	737

9.4.2.4.1	Periodic Reporting	737

9.4.2.4.2	Event-Triggered Periodic Reporting	737

9.4.2.4.3	Event-Triggered Reporting	738

9.4.3	NR − E-UTRAN TDD measurements	738

9.4.3.1	Introduction	738

9.4.3.2	Requirements when no DRX is used	738

9.4.3.3	Requirements when DRX is used	740

9.4.3.4	Measurement reporting requirements	741

9.4.3.4.1	Periodic Reporting	741

9.4.3.4.2	Event-Triggered Periodic Reporting	742

9.4.3.4.3	Event-Triggered Reporting	742

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	742

9.4.4	Inter-RAT RSTD measurements	742

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	742

9.4.4.1.1	Introduction	742

9.4.4.1.2	Requirements	743

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	746

9.4.4.2.1	Introduction	746

9.4.4.2.2	Requirements	747

9.4.5	Inter-RAT E-CID measurements	750

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	750

9.4.5.1.1	Introduction	750

9.4.5.1.2	Requirements	750

9.4.5.1.3	Measurement Reporting Delay	750

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	750

9.4.5.2.1	Introduction	750

9.4.5.2.2	Requirements	750

9.4.5.2.3	Measurement Reporting Delay	751

9.4.6	NR − UTRAN FDD measurements	751

9.4.6.1	Introduction	751

9.4.6.2	Requirements when no DRX is used	751

9.4.6.3	Requirements when DRX is used	752

9.4.7	NR – E-UTRAN measurements with autonomous gaps	754

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	754

9.4.7.2	CGI reporting delay	754

9.4A	Inter-RAT measurements for RedCap	755

9.4A.1	Introduction	755

9.4A.2	NR − E-UTRAN FDD measurements	756

9.4A.2.1	Introduction	756

9.4A.2.2	Requirements when no DRX is used	756

9.4A.2.3	Requirements when DRX is used	757

9.4A.2.4	Measurement reporting requirements	758

9.4A.2.4.1	Periodic Reporting	758

9.4A.2.4.2	Event-Triggered Periodic Reporting	758

9.4A.2.4.3	Event-Triggered Reporting	758

9.4A.3	NR − E-UTRAN TDD measurements	759

9.4A.3.1	Introduction	759

9.4A.3.2	Requirements when no DRX is used	759

9.4A.3.3	Requirements when DRX is used	761

9.4A.3.4	Measurement reporting requirements	762

9.4A.3.4.1	Periodic Reporting	762

9.4A.3.4.2	Event-Triggered Periodic Reporting	762

9.4A.3.4.3	Event-Triggered Reporting	762

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	763

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	763

9.4A.4.2	CGI reporting delay	763

9.4A.4.3	CGI reporting scheduling restriction	763

9.5	L1-RSRP measurements for Reporting	764

9.5.1	Introduction	764

9.5.2	Requirements applicability	764

9.5.3	Measurement Reporting Requirements	765

9.5.3.1	Periodic Reporting	765

9.5.3.2	Semi-Persistent Reporting	765

9.5.3.3	Aperiodic Reporting	765

9.5.4	L1-RSRP measurement requirements	766

9.5.4.1	SSB based L1-RSRP Reporting	766

9.5.4.2	CSI-RS based L1-RSRP Reporting	769

9.5.4A	Void	772

9.5.4A.1	Void	772

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	772

9.5.5.1	Measurement restriction for SSB based L1-RSRP	772

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	773

9.5.6	Scheduling availability of UE during L1-RSRP measurement	774

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	774

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	774

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	774

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	776

9.5A	L1-RSRP measurements for Reporting under CCA	776

9.5A.1	Introduction	776

9.5A.2	Requirements applicability	776

9.5A.3	Measurement Reporting Requirements	776

9.5A.3.1	Periodic Reporting	777

9.5A.3.2	Semi-Persistent Reporting	777

9.5A.3.3	Aperiodic Reporting	777

9.5A.4	L1-RSRP measurement requirements	777

9.5A.4.1	SSB based L1-RSRP Reporting	777

9.5A.5	Measurement restriction for L1-RSRP measurement	780

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	780

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	780

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	780

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	781

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	781

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	781

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	782

9.5B	L1-RSRP measurements for Reporting for RedCap	782

9.5B.1	Introduction	782

9.5B.2	Requirements applicability	782

9.5B.3	Measurement Reporting Requirements	783

9.5B.3.1	Periodic Reporting	783

9.5B.3.2	Semi-Persistent Reporting	783

9.5B.3.3	Aperiodic Reporting	783

9.5B.4	L1-RSRP measurement requirements	784

9.5B.4.1	SSB based L1-RSRP Reporting	784

9.5B.4.2	CSI-RS based L1-RSRP Reporting	785

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	788

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	788

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	789

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	789

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	789

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	789

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	790

9.5C	L1-RSRP measurements for Reporting for satellite access	790

9.5C.1	Introduction	790

9.5C.3	Measurement Reporting Requirements	791

9.5C.3.1	Periodic Reporting	791

9.5C.3.2	Semi-Persistent Reporting	791

9.5C.3.3	Aperiodic Reporting	792

9.5C.4	L1-RSRP measurement requirements	792

9.5C.4.1	SSB based L1-RSRP Reporting	792

9.5C.5	Measurement restriction for L1-RSRP measurement	794

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	794

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	794

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	795

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	795

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	795

9.6	NE-DC: Measurements	795

9.6.1	Introduction	795

9.6.2	SFTD Measurements	795

9.6.2.1	Introduction	795

9.6.2.2	SFTD Measurement requirements	795

9.7	Cross Link Interference measurements	796

9.7.1	Introduction	796

9.7.2	SRS-RSRP measurements	796

9.7.2.1	Introduction	796

9.7.2.2	Requirements applicability	797

9.7.2.3	Measurement Reporting Requirements	797

9.7.2.3.1	Periodic Reporting	797

9.7.2.3.2	Event-triggered Periodic Reporting	797

9.7.2.3.3	Event Triggered Reporting	797

9.7.2.4	Measurement capability	797

9.7.2.5	SRS-RSRP measurement period	798

9.7.3	CLI-RSSI measurements	798

9.7.3.1	Introduction	798

9.7.3.2	Requirements applicability	798

9.7.3.3	Measurement Reporting Requirements	798

9.7.3.3.1	Periodic Reporting	798

9.7.3.3.2	Event-triggered Periodic Reporting	798

9.7.3.3.3	Event Triggered Reporting	799

9.7.3.4	Measurement capability	799

9.7.3.5	CLI-RSSI measurement period	799

9.7.4	Scheduling availability of UE during CLI measurements	799

9.7.4.1	Scheduling availability of UE performing measurement on FR1	799

9.7.4.2	Scheduling availability of UE performing measurement on FR2	800

9.8	L1-SINR measurements for Reporting	800

9.8.1	Introduction	800

9.8.2	Requirements applicability	801

9.8.3	Measurement Reporting Requirements	801

9.8.3.1	Periodic Reporting	801

9.8.3.2	Semi-Persistent Reporting	802

9.8.4	L1-SINR measurement requirements	802

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	802

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	807

9.8.5	Measurement restriction for L1-SINR measurement	808

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	809

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	809

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	810

9.8.6	Scheduling availability of UE during L1-SINR measurement	810

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	810

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	810

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	811

9.9	NR measurements for positioning	812

9.9.1	Introduction	812

9.9.1.1	General Aspects of Gap-based Measurement	812

9.9.1.2	General Aspects of Gapless Measurement	812

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	813

9.9.2	RSTD measurements	814

9.9.2.1	Introduction	814

9.9.2.2	Requirements Applicability	814

9.9.2.3	Measurement Capability	814

9.9.2.4	Measurement Reporting Requirements	814

9.9.2.4.1	Void	815

9.9.2.4.2	Void	815

9.9.2.4.3	Void	815

9.9.2.5	Measurements Period Requirements	815

9.9.2.6	Void	818

9.9.2.7	Measurements Period Requirements without Measurement Gaps	818

9.9.2.8	Void	821

9.9.2.9	Measurements Period Requirements with both MG and PPW	821

9.9.3	PRS-RSRP measurements	821

9.9.3.1	Introduction	821

9.9.3.2	Requirements applicability	821

9.9.3.3	Measurement Capability	822

9.9.3.4	Measurement Reporting Requirements	822

9.9.3.5	Measurement Period Requirements	822

9.9.3.6	Measurement Period Requirements without Measurement Gaps	824

9.9.3.7	Void	827

9.9.3.8	Measurements Period Requirements with both MG and PPW	827

9.9.4	UE Rx-Tx time difference measurements	828

9.9.4.1	Introduction	828

9.9.4.2	Requirements Applicability	828

9.9.4.3	Measurement Capability	828

9.9.4.4	Measurement Reporting Requirements	828

9.9.4.5	Measurement Period Requirements	828

9.9.4.6	Measurement Period Requirements without Measurement Gaps	832

9.9.4.7	Void	835

9.9.4.8	Measurements Period Requirements with both MG and PPW	835

9.9.5	E-CID measurements	836

9.9.5.1	Introduction	836

9.9.5.2	Measurement Requirements	836

9.9.5.2.1	Intra-frequency Measurement Requirements	836

9.9.5.2.2	Inter-frequency Measurement Requirements	836

9.9.5.2.3	Measurement Reporting Delay	837

9.9.6	PRS-RSRPP measurements	837

9.9.6.1	Introduction	837

9.9.6.2	Requirements applicability	837

9.9.6.3	Measurement capability	837

9.9.6.4	Measurement reporting requirements	837

9.9.6.5	Measurement period requirements	837

9.9.6.6	Measurement Period Requirements without Measurement Gaps	838

9.9.6.7	Void	838

9.9.6.8	Measurements Period Requirements with both MG and PPW	838

9.10	CSI-RS based L3 measurements	838

9.10.1	Introduction	838

9.10.2	CSI-RS based intra-frequency measurements	838

9.10.2.1	Introduction	838

9.10.2.2	Requirements applicability	839

9.10.2.3	Number of cells and number of CSI-RS	840

9.10.2.3.1	Requirements for FR1	840

9.10.2.3.2	Requirements for FR2	840

9.10.2.4	Measurement Reporting Requirements	840

9.10.2.4.1	Periodic Reporting	840

9.10.2.4.2	Event-triggered Periodic Reporting	841

9.10.2.4.3	Event Triggered Reporting	841

9.10.2.5	Intra-frequency measurements without measurement gaps	841

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	842

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	843

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	843

9.10.3	CSI-RS based Inter-frequency measurements	843

9.10.3.1	Introduction	843

9.10.3.2	Requirements applicability	843

9.10.3.3	Number of cells and number of CSI-RS resources	844

9.10.3.3.1	Requirements for FR1	844

9.10.3.3.2	Requirements for FR2	844

9.10.3.4	Measurements reporting requirements	844

9.10.3.4.1	Periodic Reporting	845

9.10.3.4.2	Event-triggered Periodic Reporting	845

9.10.3.4.3	Event-triggered Reporting	845

9.10.3.5	Inter frequency measurements with measurement gaps	845

9.11	NR measurements with autonomous gaps	847

9.11.1	Introduction	847

9.11.2	CGI identification of an NR cell with autonomous gaps	847

9.11.3	CGI reporting delay	848

9.11A	NR measurements with autonomous gaps for RedCap	848

9.11A.1	Introduction	848

9.11A.2	CGI identification of an NR cell with autonomous gaps	848

9.11A.3	CGI reporting delay	849

9.11A.4	CGI reporting scheduling restriction	850

9.12	Measurement for Propagation Delay Compensation	850

9.12.1	Introduction	850

9.12.2	Requirements Applicability	850

9.12.3	Measurement Capability	851

9.12.4	Measurement period requirements	851

9.12.4.1	PRS Measurement Period	851

9.12.4.2	TRS Measurement Period	852

9.12.5	Measurement Reporting Requirements	853

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	853

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	853

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	853

9.13.1	Introduction	853

9.13.2	Requirements Applicability	854

9.13.3	Measurement Reporting Requirements	854

9.13.3.1	Periodic Reporting	854

9.13.3.2	Semi-Persistent Reporting	855

9.13.3.3	Aperiodic Reporting	855

9.13.4	L1-RSRP measurement requirements	855

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	855

9.13.5	Measurement restriction for L1-RSRP measurement	857

9.13.5.1	Measurement restriction for SSB based L1-RSRP	857

9.13.6	Scheduling availability of UE during L1-RSRP measurement	858

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	858

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	858

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	859

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	859

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	859

10	Measurement Performance requirements	860

10.1	NR measurements	860

10.1.1	Introduction	860

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	860

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	860

10.1.2.1.1	Absolute SS-RSRP Accuracy	860

10.1.2.1.2	Relative SS-RSRP Accuracy	861

10.1.2.2	Void	862

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	862

10.1.2.3.1	Absolute CSI-RSRP Accuracy	862

10.1.2.3.2	Relative CSI-RSRP Accuracy	863

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	864

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	864

10.1.2B.1.1	Absolute SS-RSRP Accuracy	864

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	865

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	865

10.1.2C.1.1	Absolute SS-RSRP Accuracy	865

10.1.2C.1.2	Relative SS-RSRP Accuracy	866

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	866

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	866

10.1.3.1.1	Absolute SS-RSRP Accuracy	866

10.1.3.1.2	Relative SS-RSRP Accuracy	867

10.1.3.2	Void	868

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	868

10.1.3.3.1	Absolute CSI-RSRP Accuracy	868

10.1.3.3.2	Relative CSI-RSRP Accuracy	869

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	870

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	870

10.1.3B.1.1	Absolute SS-RSRP Accuracy	870

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	871

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	871

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	871

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	872

10.1.4.2	Void	873

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	873

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	873

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	874

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	875

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	875

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	875

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	876

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	876

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	876

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	877

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	877

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	877

10.1.5.1.1	Absolute SS-RSRP Accuracy	877

10.1.5.1.2	Relative SS-RSRP Accuracy	878

10.1.5.2	Void	879

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	879

10.1.5.3.1	Absolute CSI-RSRP Accuracy	879

10.1.5.3.2	Relative CSI-RSRP Accuracy	880

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	881

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	881

10.1.5B.1.1	Absolute SS-RSRP Accuracy	881

10.1.6	RSRP Measurement Report Mapping	882

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	884

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	884

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	884

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	885

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	885

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	886

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	886

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	886

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	887

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	887

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	887

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	888

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	888

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	888

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	889

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	889

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	889

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	889

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	890

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	890

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	890

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	890

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	891

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	892

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	892

10.1.9.2.2	Relative CSI-RSRQ Accuracy	893

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	894

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	894

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	894

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	895

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	895

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	895

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	896

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	897

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	897

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	897

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	897

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	898

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	898

10.1.10.2.2	Relative CSI-RSRQ Accuracy	899

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	900

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	900

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	900

10.1.11	RSRQ report mapping	901

10.1.12	Intra-frequency SINR accuracy requirements for FR1	901

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	901

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	901

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	902

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	902

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	903

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	903

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	903

10.1.13	Intra-frequency SINR accuracy requirements for FR2	904

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	904

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	904

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	905

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	905

10.1.14	Inter-frequency SINR accuracy requirements for FR1	905

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	905

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	905

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	906

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	907

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	907

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	908

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	909

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	909

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	909

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	910

10.1.15	Inter-frequency SINR accuracy requirements for FR2	911

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	911

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	911

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	911

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	912

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	912

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	913

10.1.16	SINR report mapping	914

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	914

10.1.17	Power Headroom	915

10.1.17.1	Power Headroom Report	915

10.1.17.1.1	Power Headroom Report Mapping	915

10.1.18	PCMAX,c,f	916

10.1.18.1	Report Mapping	916

10.1.19	L1-RSRP accuracy requirements for FR1	916

10.1.19.1	SSB based L1-RSRP accuracy requirements	916

10.1.19.1.1	Absolute Accuracy	916

10.1.19.1.2	Relative Accuracy	917

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	918

10.1.19.2.1	Absolute Accuracy	918

10.1.19.2.2	Relative Accuracy	919

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	920

10.1.19C.1	SSB based L1-RSRP accuracy requirements	920

10.1.19C.1.1	Absolute Accuracy	920

10.1.19C.1.2	Relative Accuracy	921

10.1.20	L1-RSRP accuracy requirements for FR2	921

10.1.20.1	SSB based L1-RSRP accuracy requirements	921

10.1.20.1.1	Absolute Accuracy	921

10.1.20.1.2	Relative Accuracy	922

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	923

10.1.20.2.1	Absolute Accuracy	923

10.1.20.2.2	Relative Accuracy	924

10.1.21	SFTD accuracy requirements	925

10.1.21.1	SFTD acuracy requirements for NE-DC	925

10.1.21.2	SFTD acuracy requirements for NR-DC	927

10.1.21.3	Inter frequency SFTD acuracy requirements	928

10.1.22	CLI measurement accuracy requirements	929

10.1.22.1	SRS-RSRP	929

10.1.22.1.1	SRS-RSRP Accuracy	929

10.1.22.1.2	SRS-RSRP report mapping	931

10.1.22.2	CLI-RSSI	931

10.1.22.2.1	CLI-RSSI Accuracy	931

10.1.22.2.2	CLI-RSSI report mapping	932

10.1.23	RSTD Measurements	933

10.1.23.1	Introduction	933

10.1.23.2	Measurement Accuracy Requirements	933

10.1.23.3	Report mapping	943

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	943

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	945

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	947

10.1.24	PRS-RSRP Measurements	949

10.1.24.1	Introduction	949

10.1.24.2	Measurement Accuracy Requirements	949

10.1.24.2.1	Absolute PRS RSRP accuracy	949

10.1.24.2.2	Relative PRS RSRP accuracy	953

10.1.24.3	Report mapping	957

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	957

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	958

10.1.25	UE Rx-Tx Time Difference Measurements	960

10.1.25.1	Introduction	960

10.1.25.2	Measurement Accuracy Requirements	960

10.1.25.3	Report mapping	973

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	973

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	975

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	976

10.1.26	FR2 P-MPR report	978

10.1.26.1	Report mapping	978

10.1.27	L1-SINR accuracy requirements for FR1	978

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	978

10.1.27.1.1	Absolute Accuracy	978

10.1.27.1.2	Relative Accuracy	979

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	980

10.1.27.2.1	Absolute Accuracy	980

10.1.27.2.2	Relative Accuracy	981

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	983

10.1.27.3.1	Absolute Accuracy	983

10.1.27.3.2	Relative Accuracy	985

10.1.28	L1-SINR accuracy requirements for FR2	986

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	993

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	993

10.1.29.1.1	Absolute SS-RSRQ Accuracy	993

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	993

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	993

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	993

10.1.30.1.2	Relative Accuracy of SS-RSRQ	994

10.1.31	Intra-frequency SINR accuracy requirements under CCA	995

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	995

10.1.31.1.1	Absolute SS-SINR Accuracy	995

10.1.32	Inter-frequency SINR accuracy requirements under CCA	995

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	995

10.1.32.1.1	Aboslute Accuracy of SS-SINR	995

10.1.32.1.2	Relative Accuracy of SS-SINR	996

10.1.33	L1-RSRP accuracy requirements under CCA	997

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	997

10.1.33.1.1	Absolute Accuracy	997

10.1.33.1.2	Relative Accuracy	997

10.1.34	RSSI measurements under CCA	998

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	998

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	998

10.1.34.3	RSSI measurement report mapping	998

10.1.35	Channel occupancy measurements under CCA	999

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	999

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	999

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	999

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	999

10.1.36.1.1	Absolute SS-RSRP Accuracy	999

10.1.36.1.2	Relative SS-RSRP Accuracy	1000

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1001

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1001

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1001

10.1.37.1.2	Relative Accuracy of SS-RSRP	1001

10.1.38	PRS-RSRPP Measurements	1002

10.1.38.1	Introduction	1002

10.1.38.2	Measurement Accuracy Requirements	1002

10.1.38.2.1	Absolute PRS RSRPP accuracy	1002

10.1.38.3	Report mapping	1006

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1006

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1007

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1008

10.1.39.1	Introduction	1008

10.1.39.2	Measurement Accuracy Requirements for PRS	1008

10.1.39.3	 Measurement Accuracy Requirements for TRS	1012

10.1.40	Measurement Accuracy Requirements for Propagation Delay Compensation	1015

10.1.40.1	Measurement Accuracy Requirements for PRS	1015

10.1.40.2	Measurement Accuracy Requirements for TRS	1018

10.1A	NR measurements for RedCap	1021

10.1A.1	Introduction	1021

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1021

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1021

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1021

10.1A.2.1.2	Relative SS-RSRP Accuracy	1022

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1023

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1023

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1023

10.1A.3.1.2	Relative SS-RSRP Accuracy	1023

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1023

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1023

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1023

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1024

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1025

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1025

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1025

10.1A.5.1.2	Relative SS-RSRP Accuracy	1025

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1025

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1025

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1025

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1026

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1026

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1026

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1026

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1026

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1026

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1027

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1028

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1028

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1028

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1028

10.1A.10	Intra-frequency SINR accuracy requirements for FR1	1028

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1028

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1028

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1029

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1029

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1029

10.1A.12	Inter-frequency SINR accuracy requirements for FR1	1029

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1029

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1029

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1030

10.1A.13	Inter-frequency SINR accuracy requirements for FR2	1031

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1031

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1031

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1031

10.1A.14	L1-RSRP accuracy requirements for FR1	1031

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1031

10.1A.14.1.1	Absolute Accuracy	1031

10.1A.14.1.2	Relative Accuracy	1032

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1033

10.1A.14.2.1	Absolute Accuracy	1033

10.1A.14.2.2	Relative Accuracy	1034

10.1A.15	L1-RSRP accuracy requirements for FR2	1035

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1035

10.1A.15.1.1	Absolute Accuracy	1035

10.1A.15.1.2	Relative Accuracy	1035

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1035

10.1A.15.2.1	Absolute Accuracy	1035

10.1A.15.2.2	Relative Accuracy	1035

10.2	E-UTRAN measurements	1036

10.2.1	Introduction	1036

10.2.2	E-UTRAN RSRP measurements	1036

10.2.3	E-UTRAN RSRQ measurements	1036

10.2.4	E-UTRAN RSTD measurements	1036

10.2.5	E-UTRAN RS-SINR measurements	1037

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1037

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1037

10.2A	E-UTRAN measurements for RedCap	1038

10.2A.1	Introduction	1038

10.2A.2	E-UTRAN RSRP measurements	1038

10.2A.3	E-UTRAN RSRQ measurements	1038

10.2A.4	E-UTRAN RS-SINR measurements	1039

10.3	UTRAN FDD Measurements	1039

10.3.1	UTRAN FDD CPICH RSCP	1039

10.3.2	UTRAN FDD CPICH Ec/No	1040

10.4	V2X measurements	1040

10.4.1	Introduction	1040

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1040

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1040

10.4.2.2	PSBCH-RSRP Relative Accuracy	1041

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1042

10.4.3.1	Absolute SL-RSSI Accuracy	1042

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1042

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1042

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1043

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1043

11	Void	1044

12	V2X Requirements	1045

12.1	Introduction	1045

12.2	UE Transmit Timing	1045

12.2.1	Introduction	1045

12.2.2	GNSS as synchronization reference source	1045

12.2.3	NR Cell as synchronization reference source	1046

12.2.4	E-URTAN Cell as synchronization reference source	1046

12.2.5	SyncRef UE as synchronization reference source	1046

12.3	Initiation/Cease of SLSS Transmissions	1047

12.3.1	Introduction	1047

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1047

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1048

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1049

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1049

12.4	Selection / Reselection of V2X Synchronization Reference Source	1050

12.5	L1 SL-RSRP measurements	1051

12.5.1	Introduction	1051

12.5.2	SL-RSRP measurements	1051

12.6	Congestion Control measurements	1052

12.7	Interruption	1052

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1052

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1053

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1054

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1055

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1055

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1056

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1056

12.7.8	Interruptions at NR sidelink discovery configuration	1056

12.8	Reliability of GNSS signal	1056

12.9	Scheduling availability	1057

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1057

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1057

12.10	Selection / Reselection of relay UE	1058

12.10.1	Introduction	1058

12.10.2	Selection / Reselection of relay UE	1058

13	Measurement Performance Requirements for NR gNB	1058

13.1	UL-RTOA	1058

13.1.1	Report mapping	1058

13.1.1A	Additional Path Report Mapping for UL-RTOA	1060

13.2	gNB Rx-Tx time difference	1062

13.2.1	Report mapping	1062

13.2.1A	Additional Path Report Mapping for gNB Rx-Tx	1064

13.2.2	Measurement Accuracy Requirements	1066

13.2.2.1	Introduction	1066

13.3	UL SRS RSRP measurement	1067

13.3.1	Report mapping	1067

13.3.2	Measurement accuracy requirements	1068

13.3.2.1	Introduction	1068

13.3.2.2	Requirements	1068

13.4	AoA/ZoA	1069

13.4.1	Report mapping	1069

13.5	Timing advance (TADV)	1070

13.5.1	Report mapping	1070

13.6	UL SRS RSRPP measurement	1071

13.6.1	Report mapping	1071

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1073

13.7.1	Report mapping	1073

13.7.2	Measurement Accuracy Requirements	1073

13.7.2.1	Introduction	1073

13.7.2.2	Requirements	1073

Annex A (normative): Test Cases	1075

A.1	Purpose of annex	1075

A.2	Requirement classification for statistical testing	1075

A.2.1	Types of requirements in TS 38.133	1075

A.2.1.1	Time and delay requirements on UE higher layer actions	1075

A.2.1.2	Measurements of power levels, relative powers and time	1075

A.2.1.3	Implementation requirements	1076

A.2.1.4	Physical layer timing requirements	1076

A.2.1.5	Requirements under CCA	1076

A.3	RRM test configurations	1077

A.3.1	Reference measurement channels	1077

A.3.1.1	PDSCH	1077

A.3.1.1.1	FDD	1077

A.3.1.1.2	TDD	1078

A.3.1.2	CORESET for RMSI scheduling	1081

A.3.1.2.1	FDD	1081

A.3.1.2.2	TDD	1082

A.3.1.3	CORESET for RMC scheduling	1085

A.3.1.3.1	FDD	1085

A.3.1.3.2	TDD	1086

A.3.1.4	TDD UL/DL configuration	1089

A.3.1A	Reference measurement channels under CCA	1091

A.3.1A.1	PDSCH	1091

A.3.1A.1.1	TDD	1091

A.3.1A.2	CORESET for RMSI scheduling	1092

A.3.1A.2.1	TDD	1092

A.3.1A.3	CORESET for RMC scheduling	1093

A.3.1A.3.1	TDD	1093

A.3.1A.4	TDD UL/DL configuration	1094

A.3.1A.5	RMC burst transmission model	1094

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1094

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1095

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1095

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1096

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1096

A.3.2.2	Void	1097

A.3.3	Reference DRX configurations	1097

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1097

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1098

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1098

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1098

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1099

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1099

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1099

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1100

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1100

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1100

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1101

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1101

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1101

A.3.4	Test Cases with Different Channel Bandwidths	1102

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1102

A.3.4.1.1	Introduction	1102

A.3.4.1.2	Principle of testing	1102

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1102

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1102

A.3.5.1.1	Introduction	1102

A.3.5.1.2	Principle of Testing	1102

A.3.6	Antenna configurations	1102

A.3.6.1	Antenna configurations for FR1	1102

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1102

A.3.6.1.1.1	Introduction	1102

A.3.6.1.1.2	Principle of testing	1103

A.3.6.2	Antenna configurations for FR2	1106

A.3.6A	Antenna configurations with unlicensed bands	1106

A.3.6A.1	Antenna configurations for FR1	1106

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1106

A.3.6A.1.1.1	Introduction	1106

A.3.6A.1.1.2	Principle of testing	1106

A.3.7	EN-DC test setup	1108

A.3.7.1	Introduction	1108

A.3.7.2	E-UTRAN Serving Cell Parameters	1108

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1108

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1109

A.3.7A	NR FR1-FR2 test setup	1110

A.3.7B	EN-DC test setup with unlicensed bands	1111

A.3.7B.1	Introduction	1111

A.3.7B.2	E-UTRAN Serving Cell Parameters	1111

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1111

A.3.7C	LTE-FR1/FR2 test setup	1112

A.3.7D	NE-DC test setup	1112

A.3.7D.1	Introduction	1112

A.3.7D.2	E-UTRAN Serving Cell Parameters	1112

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1112

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1112

A.3.8	PRACH configurations	1112

A.3.8.1	Introduction	1112

A.3.8.2	PRACH configurations in FR1	1113

A.3.8.2.1	FR1 PRACH configuration 1	1113

A.3.8.2.2	FR1 PRACH configuration 2	1113

A.3.8.2.3	FR1 PRACH configuration 3	1114

A.3.8.2.4	FR1 PRACH configuration 4	1115

A.3.8.3	PRACH configurations in FR2	1116

A.3.8.3.1	FR2 PRACH configuration 1	1116

A.3.8.3.2	FR2 PRACH configuration 2	1117

A.3.8.3.3	FR2 PRACH configuration 3	1118

A.3.8.3.4	FR2 PRACH configuration 4	1119

A.3.8A	PRACH configurations under CCA	1120

A.3.8A.1	Introduction	1120

A.3.8A.2	PRACH configurations in FR1	1120

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1120

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1121

A.3.9	BWP configurations	1122

A.3.9.1	Introduction	1122

A.3.9.2	Downlink BWP configurations	1123

A.3.9.2.1	Initial BWP	1123

A.3.9.2.2	Dedicated BWP	1123

A.3.9.3	Uplink BWP configurations	1124

A.3.9.3.1	Initial BWP	1124

A.3.9.3.2	Dedicated BWP	1124

A.3.9A	BWP configurations for RedCap	1124

A.3.9A.1	Introduction	1124

A.3.9A.2	Downlink BWP configurations	1125

A.3.9A.2.1	Dedicated BWP	1125

A.3.9A.3	Uplink BWP configurations	1125

A.3.9A.3.2	Dedicated BWP	1125

A.3.10	SSB Configurations	1126

A.3.10.1	SSB Configurations for FR1	1126

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1126

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1128

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1128

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1129

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1129

A.3.10.2	SSB Configurations for FR2	1130

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1130

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1130

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1131

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1131

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1132

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1132

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1133

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1133

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1134

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1134

A.3.10A	SSB Configurations under CCA	1139

A.3.10A.1	SSB Configurations under CCA for FR1	1139

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1139

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1139

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1140

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1140

A.3.10B	SSB Configurations for RedCap	1141

A.3.10B.1	SSB Configurations for FR1	1141

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1141

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1141

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1142

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1142

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1143

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1143

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1144

A.3.10B.2	SSB Configurations for FR2	1144

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1144

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1145

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1145

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1146

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1146

A.3.11	SMTC Configurations	1147

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1147

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1147

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1147

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1147

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1148

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1148

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1148

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1148

A.3.11A	SMTC Configurations for RedCap	1149

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1149

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1149

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1149

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1149

A.3.12	Test Cases with Different CC Configurations	1149

A.3.12.1	EN-DC Test Cases with Different EN-DC Configurations	1149

A.3.12.1.1	Introduction	1149

A.3.12.1.2	Principle of testing	1150

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1150

A.3.12.2.1	Introduction	1150

A.3.12.2.2	Principle of testing	1150

A.3.13	Test Cases in SA and EN-DC Operations	1150

A.3.13.1	Introduction	1150

A.3.13.2	Principle of Testing	1150

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1151

A.3.13A.1	Introduction	1151

A.3.13A.2	Principle of Testing in EN-DC	1151

A.3.13A.3	Principle of Testing in SA	1151

A.3.13A.4	Principle of Testing in E-UTRA	1152

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1152

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1152

A.3.13B.1.1	Introduction	1152

A.3.13B.1.2	Principle of Testing	1152

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1152

A.3.13B.2.1	Introduction	1152

A.3.13B.2.2	Principle of Testing	1152

A.3.14	CSI-RS configurations	1153

A.3.14.1	FDD	1153

A.3.14.2	TDD	1155

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1161

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1161

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1161

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1161

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1162

A.3.15.3	Setup 3: 2 AoAs	1162

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1162

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1162

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1162

A.3.16	TCI State Configuration	1163

A.3.16.1	Introduction	1163

A.3.16.2	TCI states	1163

A.3.16A	Unified TCI State Configuration	1163

A.3.16A.1	Introduction	1163

A.3.16A.2	DLorJoint TCI states	1164

A.3.16A.3	UL TCI states	1164

A.3.17	Configurations of CSI-RS for tracking	1165

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1165

A.3.17.1.1	FDD	1165

A.3.17.1.2	TDD	1167

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1169

A.3.17.2.1	TDD	1169

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1170

A.3.18.1	Introduction	1170

A.3.18.2	PRACH Power Measurement	1170

A.3.19	Test applicability for DAPS handover	1170

A.3.19.1	Introduction	1170

A.3.19.2	Principle of testing	1170

A.3.20	MsgA configurations	1171

A.3.20.1	Introduction	1171

A.3.20.2	MsgA configurations in FR1	1171

A.3.20.2.1	FR1 MsgA configuration 1	1171

A.3.20.2.2	FR1 MsgA configuration 2	1172

A.3.20.3	MsgA configurations in FR2	1174

A.3.20.3.1	FR2 MsgA configuration 1	1174

A.3.20.3.2	FR2 MsgA configuration 2	1175

A.3.20A	MsgA configurations under CCA	1177

A.3.20A.1	Introduction	1177

A.3.20A.2	MsgA configurations in FR1	1177

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1177

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1178

A.3.21	V2X sidelink communication	1181

A.3.21.1	Introduction	1181

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1181

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1185

A.3.21.4	Reference SL-DRX configurations	1186

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1186

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1186

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1186

A.3.22	CSI-IM configurations	1186

A.3.22.1	FDD	1186

A.3.22.2	TDD	1187

A.3.23	Spatial Relation Configuration	1188

A.3.23.1	Introduction	1188

A.3.23.2	Spatial Relation	1189

A.3.24	SRS configuration	1190

A.3.25	Channel bandwidth (CBW) configurations	1192

A.3.25.1	DL UE specific CBW	1192

A.3.25.2	UL UE specific CBW	1193

A.3.26	CCA model	1193

A.3.26.1	Introduction	1193

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1193

A.3.26.2.1	DL CCA model	1193

A.3.26.2.2	UL CCA model	1195

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1195

A.3.26.3.1	DL CCA model	1195

A.3.26.3.2	UL CCA model	1196

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1196

A.3.27.1	Introduction	1196

A.3.27.2	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1196

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1197

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1197

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1197

A.3.28	Discovery Burst Transmission Window configuration under CCA	1197

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1197

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1197

A.3.29.1	Introduction	1197

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1197

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1198

A.3.30	CSI-RS configurations for RRM	1199

A.3.30.1	FDD	1199

A.3.30.2	TDD	1200

A.3.31	PRS Configurations	1202

A.3.31.1.	PRS Configurations for FR1	1202

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1202

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1203

A.3.31.2.	PRS Configurations for FR2	1203

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1203

A.3.32	NR sidelink discovery	1203

A.3.32.1	Introduction	1203

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1203

A.3.32.3	Principle of Testing	1204

A.3.33	PRS Processing Window (PPW) configurations	1204

A.3.34	Testing principles for test cases related to PRS measurements	1204

A.3.34.1	Introduction	1204

A.3.34.2	Test cases in RRC_INACTIVE state	1205

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1205

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1205

A.3.35	Testing principle for RedCap UE	1206

A.3.35.1	Introduction	1206

A.3.35.2	Principle of testing for FR1	1206

A.3.35.3	Principle of testing for FR2	1206

A.3.36	Testing related to Satellite access	1206

A.3.36.1	Introduction	1206

A.3.36.2	Principle of testing GSO and NGSO scenarios	1206

A.3.36.2	Principle of testing different RRM requirements	1207

A.3.36.3	Principle of testing different ephemeris formats	1207

A.3.36.4	General setup for SIB19	1209

A.3.36.5	Satellite specific parameters configuration	1209

A.3.36.5.1	Satellite specific configuration for serving cell	1209

A.3.36.5.2	Satellite specific configuration for neighbour cell	1210

A.4	EN-DC tests with all NR cells in FR1	1211

A.4.1	Void	1211

A.4.2	Void	1211

A.4.3	RRC_CONNECTED state mobility	1211

A.4.3.1	Void	1211

A.4.3.2	RRC Connection Mobility Control	1211

A.4.3.2.1	Void	1211

A.4.3.2.2	Random Access	1211

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1211

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1215

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1219

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1223

A.4.3.2.3	Void	1227

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1227

A.4.3.3.1	Test Purpose and Environment	1227

A.4.3.3.2	Test Requirements	1233

A.4.4	Timing	1234

A.4.4.1	UE transmit timing	1234

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1234

A.4.4.1.1.1	Test Purpose and environment	1234

A.4.4.1.1.2	Test requirements	1238

A.4.4.2	UE timer accuracy	1238

A.4.4.3	Timing advance	1238

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1238

A.4.4.3.1.1	Test Purpose and Environment	1238

A.4.4.3.1.2	Test Parameters	1238

A.4.4.3.1.3	Test Requirements	1242

A.4.5	Signaling characteristics	1242

A.4.5.1	Radio link Monitoring	1242

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1243

A.4.5.1.1.1	Test Purpose and Environment	1243

A.4.5.1.1.2	Test Requirements	1247

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1247

A.4.5.1.2.1	Test Purpose and Environment	1247

A.4.5.1.2.2	Test Requirements	1253

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1253

A.4.5.1.3.1	Test Purpose and Environment	1253

A.4.5.1.3.2	Test Requirements	1258

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1258

A.4.5.1.4.1	Test Purpose and Environment	1258

A.4.5.1.4.2	Test Requirements	1264

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1264

A.4.5.1.5.1	Test Purpose and Environment	1264

A.4.5.1.5.2	Test Requirements	1269

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1269

A.4.5.1.6.1	Test Purpose and Environment	1269

A.4.5.1.6.2	Test Requirements	1275

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1275

A.4.5.1.7.1	Test Purpose and Environment	1275

A.4.5.1.7.2	Test Requirements	1280

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1281

A.4.5.1.8.1	Test Purpose and Environment	1281

A.4.5.1.8.2	Test Requirements	1286

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1286

A.4.5.1.9.1	Test Purpose and Environment	1286

A.4.5.2	Interruption	1292

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1292

A.4.5.2.1.1	Test Purpose and Environment	1292

A.4.5.2.1.2	Test Requirements	1296

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1296

A.4.5.2.2.1	Test Purpose and Environment	1296

A.4.5.2.2.2	Test Requirements	1300

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1300

A.4.5.2.3.1	Test Purpose and Environment	1300

A.4.5.2.3.2	Test Requirements	1305

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1306

A.4.5.2.4.1	Test Purpose and Environment	1306

A.4.5.2.4.2	Test Requirements	1310

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1311

A.4.5.2.5.1	Test Purpose and Environment	1311

A.4.5.2.5.2	Test Requirements	1315

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1315

A.4.5.2.6.1	Test Purpose and Environment	1315

A.4.5.2.6.2	Test Requirements	1319

A.4.5.2.7	Void	1319

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1319

A.4.5.2.8.1	Test Purpose and Environment	1319

A.4.5.2.8.2	Test Requirements	1322

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1323

A.4.5.2.9.1	Test Purpose and Environment	1323

A.4.5.2.9.2	Test Requirements	1326

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1327

A.4.5.2.10.1	Test Purpose and Environment	1327

A.4.5.2.10.2	Test Requirements	1330

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1330

A.4.5.2.11.1	Test Purpose and Environment	1330

A.4.5.2.11.2	Test Requirements	1335

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1336

A.4.5.2.12.1	Test Purpose and Environment	1336

A.4.5.3	SCell Activation and Deactivation Delay	1343

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1343

A.4.5.3.1.1	Test Purpose and Environment	1343

A.4.5.3.1.2	Test Requirements	1349

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1350

A.4.5.3.2.1	Test Purpose and Environment	1350

A.4.5.3.2.2	Test Requirements	1350

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1350

A.4.5.3.3.1	Test Purpose and Environment	1350

A.4.5.3.3.2	Test Requirements	1351

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1352

A.4.5.3.4.1	Test Purpose and Environment	1352

A.4.5.3.3.2	Test Requirements	1354

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1354

A.4.5.3.5.1	Test Purpose and Environment	1354

A.4.5.3.5.2	Test Requirements	1362

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1363

A.4.5.3.6.1	Test Purpose and Environment	1363

A.4.5.3.6.2	Test Requirements	1367

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1368

A.4.5.3.7.1	Test Purpose and Environment	1368

A.4.5.3.7.2	Test Requirements	1368

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1369

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1369

A.4.5.4.1.1	Test Purpose and Environment	1369

A.4.5.4.1.2	Test Requirements	1379

A.4.5.5	Beam Failure Detection and Link recovery procedures	1379

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1379

A.4.5.5.1.1	Test Purpose and Environment	1379

A.4.5.5.1.2	Test Requirements	1385

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1386

A.4.5.5.2.1	Test Purpose and Environment	1386

A.4.5.5.2.2	Test Requirements	1391

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1392

A.4.5.5.3.1	Test Purpose and Environment	1392

A.4.5.5.3.2	Test Requirements	1397

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1398

A.4.5.5.4.1	Test Purpose and Environment	1398

A.4.5.5.4.2	Test Requirements	1403

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1404

A.4.5.5.5.1	Test Purpose and Environment	1404

A.4.5.5.5.2	Test Requirements	1409

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1410

A.4.5.5.6.1	Test Purpose and Environment	1410

A.4.5.5.6.2	Test Requirements	1416

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1417

A.4.5.5.7.1	Test Purpose and Environment	1417

A.4.5.5.7.2	Test Requirements	1422

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1423

A.4.5.5.8.1	Test Purpose and Environment	1423

A.4.5.5.8.2	Test Requirements	1429

A.4.5.6	Active BWP switch	1430

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1430

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1430

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1435

A.4.5.6.2	RRC-based Active BWP Switch	1441

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1445

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1445

A.4.5.6.3.1.1	Test Purpose and Environment	1445

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1451

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1451

A.4.5.6.4.1.1	Test Purpose and Environment	1451

A.4.5.6.4.1.2	Test Requirements	1458

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1458

A.4.5.6.4.2.1	Test Purpose and Environment	1458

A.4.5.6.4.2.2	Test Requirements	1468

A.4.5.6.5	SCell dormancy switch	1469

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1469

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1474

A.4.5.6.5.2.1	Test Purpose and Environment	1474

A.4.5.6.5.2.2	Test Requirements	1480

A.4.5.7	PSCell addition and release delay	1480

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1480

A.4.5.7.1.1	Test purpose and environment	1480

A.4.5.7.1.2	Test Requirements	1485

A.4.5.8	DL Interruptions at switching between two uplink carriers	1486

A.4.5.8.1	Test Purpose and Environment	1486

A.4.5.8.2	Test Requirements	1489

A.4.5.9	UE specific CBW change	1490

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1490

A.4.5.9.1.1	Test Purpose and Environment	1490

A.4.5.9.1.2	Test Requirements	1494

A.4.5.10	PSCell activation and deactivation delay	1494

A.4.5.10.1	PSCell activation and deactivation delay	1494

A.4.5.10.1.1	Test purpose and environment	1494

A.4.5.10.1.2	Test Requirements	1498

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1499

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1499

A.4.5.11.1.1	Test purpose and environment	1499

A.4.5.11.1.2	Test Parameters	1499

A.4.5.11.1.3	Test Requirements	1503

A.4.6	Measurement procedure	1503

A.4.6.1	Intra-frequency Measurements	1503

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1503

A.4.6.1.1.1	Test purpose and Environment	1503

A.4.6.1.1.2	Test parameters	1503

A.4.6.1.1.3	Test Requirements	1507

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1507

A.4.6.1.2.1	Test purpose and Environment	1507

A.4.6.1.2.2	Test parameters	1507

A.4.6.1.2.2	Test Requirements	1511

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1511

A.4.6.1.3.1	Test purpose and Environment	1511

A.4.6.1.3.2	Test parameters	1511

A.4.6.1.3.3	Test Requirements	1515

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1515

A.4.6.1.4.1	Test purpose and Environment	1515

A.4.6.1.4.2	Test parameters	1515

A.4.6.1.4.3	Test Requirements	1519

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1519

A.4.6.1.5.1	Test purpose and Environment	1519

A.4.6.1.5.2	Test parameters	1519

A.4.6.1.5.3	Test Requirements	1521

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1522

A.4.6.1.6.1	Test purpose and Environment	1522

A.4.6.1.6.2	Test parameters	1522

A.4.6.1.6.3	Test Requirements	1524

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1525

A.4.6.1.7.1	Test purpose and Environment	1525

A.4.6.1.7.2	Test parameters	1525

A.4.6.1.7.3	Test Requirements	1529

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1529

A.4.6.1.8.1	Test Purpose and Environment	1529

A.4.6.1.8.2	Test Requirements	1534

A.4.6.2	Inter-frequency Measurements	1535

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1535

A.4.6.2.1.1	Test Purpose and Environment	1535

A.4.6.2.1.2	Test Requirements	1539

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1539

A.4.6.2.2.1	Test Purpose and Environment	1539

A.4.6.2.2.2	Test Requirements	1543

A.4.6.2.3	Void	1543

A.4.6.2.4	Void	1543

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1543

A.4.6.2.5.1	Test Purpose and Environment	1543

A.4.6.2.5.2	Test Requirements	1548

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1548

A.4.6.2.6.1	Test Purpose and Environment	1548

A.4.6.2.6.2	Test Requirements	1553

A.4.6.2.7	Void	1553

A.4.6.2.8	Void	1553

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1554

A.4.6.2.9.1	Test Purpose and Environment	1554

A.4.6.2.9.2	Test Requirements	1557

A.4.6.3	Void	1558

A.4.6.4	L1-RSRP measurement for beam reporting	1558

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1558

A.4.6.4.1.1	Test Purpose and Environment	1558

A.4.6.4.1.2	Test parameters	1558

A.4.6.4.1.3	Test Requirements	1561

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1561

A.4.6.4.2.1	Test Purpose and Environment	1561

A.4.6.4.2.2	Test parameters	1562

A.4.6.4.2.3	Test Requirements	1565

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1565

A.4.6.4.3.1	Test Purpose and Environment	1565

A.4.6.4.3.2	Test parameters	1566

A.4.6.4.3.3	Test Requirements	1569

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1569

A.4.6.4.4.1	Test Purpose and Environment	1569

A.4.6.4.4.2	Test parameters	1570

A.4.6.4.4.3	Test Requirements	1572

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1572

A.4.6.4.5.1	Test Purpose and Environment	1572

A.4.6.4.5.2	Test parameters	1573

A.4.6.4.5.3	Test Requirements	1576

A.4.6.5	CLI measurements	1576

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1576

A.4.6.5.1.1	Test Purpose and Environment	1576

A.4.6.5.1.2	Test Parameters	1577

A.4.6.5.1.3	Test Requirements	1580

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1580

A.4.6.5.2.1	Test Purpose and Environment	1580

A.4.6.5.2.2	Test Parameters	1581

A.4.6.5.2.3	Test Requirements	1582

A.4.6.6.1.2	Test Requirements	1587

A.4.6.7	L1-SINR measurement for beam reporting	1587

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1590

A.4.6.7.2.1	Test Purpose and Environment	1590

A.4.6.7.2.2	Test parameters	1590

A.4.6.7.2.3	Test Requirements	1593

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1593

A.4.6.7.3.1	Test Purpose and Environment	1593

A.4.6.7.3.2	Test parameters	1593

A.4.6.7.3.3	Test Requirements	1595

A.4.6.8	CSI-RS based intra-frequency Measurement	1596

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1596

A.4.6.8.1.1	Test purpose and Environment	1596

A.4.6.8.1.2	Test Requirements	1600

A.4.6.9	CSI-RS based inter-frequency Measurement	1600

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1600

A.4.6.9.1.1	Test Purpose and Environment	1600

A.4.6.9.1.2	Test Requirements	1604

A.4.7	Measurement Performance requirements	1605

A.4.7.1	SS-RSRP	1605

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1605

A.4.7.1.1.1	Test Purpose and Environment	1605

A.4.7.1.1.2	Test parameters	1605

A.4.7.1.1.3	Test Requirements	1609

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1609

A.4.7.1.2.1	Test Purpose and Environment	1609

A.4.7.1.2.2	Test parameters	1610

A.4.7.1.2.3	Test Requirements	1614

A.4.7.1.3	Void	1614

A.4.7.2	SS-RSRQ	1614

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1614

A.4.7.2.1.1	Test Purpose and Environment	1614

A.4.7.2.1.2	Test Parameters	1615

A.4.7.2.1.3	Test Requirements	1619

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1619

A.4.7.2.2.1	Test Purpose and Environment	1619

A.4.7.2.2.2	Test Parameters	1619

A.4.7.2.2.3	Test Requirements	1624

A.4.7.3	SS-SINR	1624

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1624

A.4.7.3.1.1	Test Purpose and Environment	1624

A.4.7.3.1.2	Test Parameters	1624

A.4.7.3.1.3	Test Requirements	1627

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1627

A.4.7.3.2.1	Test Purpose and Environment	1627

A.4.7.3.2.2	Test Parameters	1627

A.4.7.3.2.3	Test Requirements	1632

A.4.7.4	L1-RSRP measurement for beam reporting	1632

A.4.7.4.1	SSB based L1-RSRP measurement	1632

A.4.7.4.1.1	Test Purpose and Environment	1632

A.4.7.4.1.2	Test parameters	1633

A.4.7.4.1.3	Test Requirements	1636

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1636

A.4.7.4.2.1	Test Purpose and Environment	1636

A.4.7.4.2.2	Test parameters	1637

A.4.7.4.2.3	Test Requirements	1640

A.4.7.5	SFTD accuracy	1640

A.4.7.5.1	SFTD accuracy	1640

A.4.7.5.1.1	Test Purpose and Environment	1640

A.4.7.5.1.2	Test Parameters	1640

A.4.7.5.1.3	Test Requirements	1645

A.4.7.5.2	Void	1645

A.4.7.5.3	Void	1645

A.4.7.6	CLI measurements	1645

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1645

A.4.7.6.1.1	Test Purpose and Environment	1645

A.4.7.6.1.2	Test parameters	1646

A.4.7.6.1.3	Test Requirements	1651

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1651

A.4.7.6.2.1	Test Purpose and Environment	1651

A.4.7.6.2.2	Test parameters	1652

A.4.7.6.2.3	Test Requirements	1655

A.4.7.7	L1-SINR measurement for beam reporting	1655

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1659

A.4.7.7.2.1	Test Purpose and Environment	1659

A.4.7.7.2.2	Test parameters	1659

A.4.7.7.2.3	Test Requirements	1663

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1663

A.4.7.7.3.1	Test Purpose and Environment	1663

A.4.7.7.3.2	Test parameters	1663

A.4.7.7.3.3	Test Requirements	1666

A.4.7.8	CSI-RSRP	1666

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1666

A.4.7.8.1.1	Test Purpose and Environment	1666

A.4.7.8.1.2	Test parameters	1667

A.4.7.8.1.3	Test Requirements	1672

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1672

A.4.7.8.2.1	Test Purpose and Environment	1672

A.4.7.8.2.2	Test parameters	1672

A.4.7.8.2.3	Test Requirements	1677

A.4.7.9	CSI-RSRQ	1677

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1677

A.4.7.9.1.1	Test Purpose and Environment	1677

A.4.7.9.1.2	Test Parameters	1677

A.4.7.9.1.3	Test Requirements	1682

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1682

A.4.7.9.2.1	Test Purpose and Environment	1682

A.4.7.9.2.2	Test Parameters	1682

A.4.7.9.2.3	Test Requirements	1687

A.4.7.10	CSI-SINR	1687

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1687

A.4.7.10.1.1	Test Purpose and Environment	1687

A.4.7.10.1.2	Test Parameters	1688

A.4.7.10.1.3	Test Requirements	1692

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1692

A.4.7.10.2.1	Test Purpose and Environment	1692

A.4.7.10.2.2	Test Parameters	1693

A.4.7.10.2.3	Test Requirements	1697

A.4.8	Void	1697

A.4A	NE-DC test with all NR cells in FR1	1698

A.4A.1	Signaling characteristics	1698

A.4A.1.1	E-UTRAN PSCell addition	1698

A.4A.1.1.1	Test purpose and environment	1698

A.4A.1.1.2	Test Requirements	1702

A.4A.1.2	Active BWP switch	1703

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1703

A.4A.1.2.1.1	Test Purpose and Environment	1703

A.4A.1.2.1.2	Test Requirements	1707

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1708

A.4A.1.3.1	Test purpose and environment	1708

A.4A.1.3.2	Test Requirements	1712

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1713

A.4A.1.4.1	Test Purpose and Environment	1713

A.4A.1.4.2	Test Parameters	1713

A.4A.1.4.3	Test Requirements	1718

A.4A.1.4.3.1	Test Requirements for NR HO	1718

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1718

A.4A.2	Measurement performance	1719

A.4A.2.1	SFTD accuracy	1719

A.4A.2.1.1	SFTD accuracy	1719

A.4A.2.1.1.1	Test Purpose	1719

A.4A.2.1.1.2	Test Environment	1719

A.4A.2.1.1.3	Test Requirements	1722

A.5	EN-DC tests with one or more NR cells in FR2	1723

A.5.1	Void	1723

A.5.2	Void	1723

A.5.3	RRC_CONNECTED state mobility	1723

A.5.3.1	Void	1723

A.5.3.2	RRC Connection Mobility Control	1723

A.5.3.2.1	Void	1723

A.5.3.2.2	Random Access	1723

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1723

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1726

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1732

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1735

A.5.3.2.3	Void	1739

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1739

A.5.3.3.1	Test purpose and environment	1739

A.5.3.3.2	Test Requirements	1743

A.5.5.3.3	PUCCH SCell Activation and deactivation of known SCell in FR2	1744

A.5.5.3.3.1	Test Purpose and Environment	1744

A.5.5.3.3.2	Test Requirements	1747

A.5.5.3.4	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1747

A.5.5.3.4.1	Test Purpose and Environment	1747

A.5.5.3.4.2	Test Requirements	1751

A.5.5.3.5	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1751

A.5.5.3.5.1	Test Purpose and Environment	1751

A.5.5.3.5.2	Test Requirements	1755

A.5.5.3.6	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1756

A.5.5.3.6.1	Test Purpose and Environment	1756

A.5.5.3.6.2	Test Requirements	1759

A.5.4	Timing	1760

A.5.4.1	UE transmit timing	1760

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1760

A.5.4.1.1.1	Test Purpose and environment	1760

A.5.4.1.1.2	Test requirements	1763

A.5.4.2	UE timer accuracy	1764

A.5.4.3	Timing advance	1764

A.5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	1764

A.5.4.3.1.1	Test Purpose and Environment	1764

A.5.4.3.1.2	Test Parameters	1764

A.5.4.3.1.3	Test Requirements	1768

A.5.5	Signaling characteristics	1768

A.5.5.1	Radio link Monitoring	1768

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1768

A.5.5.1.1.1	Test Purpose and Environment	1768

A.5.5.1.1.2	Test Requirements	1772

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1773

A.5.5.1.2.1	Test Purpose and Environment	1773

A.5.5.1.2.2	Test Requirements	1776

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1777

A.5.5.1.3.1	Test Purpose and Environment	1777

A.5.5.1.3.2	Test Requirements	1781

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1781

A.5.5.1.4.1	Test Purpose and Environment	1781

A.5.5.1.4.2	Test Requirements	1785

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1785

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1789

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1793

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1798

A.5.5.1.8.2	Test Requirements	1802

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1803

A.5.5.1.9.1	Test Purpose and Environment	1803

A.5.5.1.9.2	Test Requirements	1805

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1805

A.5.5.1.10.1	Test Purpose and Environment	1805

A.5.5.1.10.2	Test Requirements	1808

A.5.5.2	Interruption	1808

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1808

A.5.5.2.1.1	Test Purpose and Environment	1808

A.5.5.2.1.2	Test Requirements	1811

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1811

A.5.5.2.2.1	Test Purpose and Environment	1811

A.5.5.2.2.2	Test Requirements	1814

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1814

A.5.5.2.3.1	Test Purpose and Environment	1814

A.5.5.2.3.2	Test Requirements	1818

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1818

A.5.5.2.4.1	Test Purpose and Environment	1818

A.5.5.2.4.2	Test Requirements	1821

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1822

A.5.5.2.5.1	Test Purpose and Environment	1822

A.5.5.2.5.2	Test Requirements	1825

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1825

A.5.5.2.6.1	Test Purpose and Environment	1825

A.5.5.2.6.2	Test Requirements	1828

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1828

A.5.5.2.7.1	Test Purpose and Environment	1828

A.5.5.2.7.2	Test Requirements	1832

A.5.5.2.8	E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1832

A.5.5.2.8.1	Test Purpose and Environment	1832

A.5.5.2.8.3	Test Requirements	1834

A.5.5.3	SCell Activation and Deactivation Delay	1834

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1834

A.5.5.3.1.1	Test Purpose and Environment	1834

A.5.5.3.1.2	Test Requirements	1836

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1836

A.5.5.3.2.1	Test Purpose and Environment	1836

A.5.5.3.2.2	Test Requirements	1840

A.5.5.3.3	Void	1840

A.5.5.3.4	Void	1840

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1840

A.5.5.3.5.1	Test Purpose and Environment	1840

A.5.5.3.5.2	Test Requirements	1843

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1844

A.5.5.3.6.1	Test Purpose and Environment	1844

A.5.5.3.6.2	Test Requirements	1848

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1849

A.5.5.3.7.1	Test Purpose and Environment	1849

A.5.5.3.7.2	Test Requirements	1852

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1852

A.5.5.3.8.1	Test Purpose and Environment	1852

A.5.5.3.8.2	Test Requirements	1856

A.5.5.4	Void	1857

A.5.5.5	Beam Failure Detection and Link recovery procedures	1857

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1857

A.5.5.5.1.1	Test Purpose and Environment	1857

A.5.5.5.1.2	Test Requirements	1861

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1862

A.5.5.5.2.1	Test Purpose and Environment	1862

A.5.5.5.2.2	Test Requirements	1866

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1867

A.5.5.5.3.1	Test Purpose and Environment	1867

A.5.5.5.3.2	Test Requirements	1871

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1872

A.5.5.5.4.1	Test Purpose and Environment	1872

A.5.5.5.4.2	Test Requirements	1876

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1877

A.5.5.5.5.1	Test Purpose and Environment	1877

A.5.5.5.5.2	Test Requirements	1881

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1882

A.5.5.5.6.1	Test Purpose and Environment	1882

A.5.5.5.6.2	Test Requirements	1886

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1887

A.5.5.5.7.1	Test Purpose and Environment	1887

A.5.5.5.7.2	Test Requirements	1891

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1892

A.5.5.5.8.1	Test Purpose and Environment	1892

A.5.5.5.8.2	Test Requirements	1897

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	1897

A.5.5.5.9.1	Test Purpose and Environment	1897

A.5.5.5.9.2	Test Requirements	1901

A.5.5.6	Active BWP switch	1901

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1901

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1901

A.5.5.6.1.1.1	Test Purpose and Environment	1901

A.5.5.6.1.1.2	Test Requirements	1905

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1905

A.5.5.6.2	RRC-based Active BWP Switch	1910

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1910

A.5.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1914

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1914

A.5.5.6.4	SCell dormancy switch	1918

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1918

A.5.5.6.4.1.1	Test Purpose and Environment	1918

A.5.5.6.4.1.2	Test Requirements	1922

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1922

A.5.5.6.4.2.1	Test Purpose and Environment	1922

A.5.5.6.4.2.2	Test Requirements	1929

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1929

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	1929

A.5.5.7	PSCell addition and release delay	1932

A.5.5.7.1	Addition and Release Delay of NR PSCell	1932

A.5.5.7.1.1	Test purpose and environment	1932

A.5.5.7.1.2	Test Requirements	1935

A.5.5.8	Active TCI state switch delay	1936

A.5.5.8.1	MAC-CE based active TCI state switch	1936

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1936

A.5.5.8.1.1.1	Test Purpose and Environment	1936

A.5.5.8.1.1.2	Test Requirements	1939

A.5.5.8.2	RRC based active TCI state switch	1939

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1939

A.5.5.8.2.1.1	Test Purpose and Environment	1939

A.5.5.8.2.1.2	Test Requirements	1943

A.5.5.9	Uplink spatial relation switch delay	1943

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1943

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1943

A.5.5.9.1.1.1	Test Purpose and Environment	1943

A.5.5.9.1.1.2	Test Requirements	1946

A.5.5.9.2	RRC based spatial relation switch	1946

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1946

A.5.5.9.2.1.1	Test Purpose and Environment	1946

A.5.5.9.2.1.2	Test Requirements	1949

A.5.5.10	UE specific CBW change	1949

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1949

A.5.5.10.1.1	Test Purpose and Environment	1949

A.5.5.10.1.2	Test Requirements	1952

A.5.5.11	Unified TCI state switch delay	1953

A.5.5.11.1	MAC-CE based active joint TCI state switch	1953

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	1953

A.5.5.11.1.1.1	Test Purpose and Environment	1953

A.5.5.11.1.1.2	Test parameters	1953

A.5.5.11.1.1.3	Test Requirements	1956

A.5.5.11.2	MAC-CE based active uplink TCI state switch	1956

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	1956

A.5.5.11.2.1.1	Test Purpose and Environment	1956

A.5.5.11.2.1.2	Test parameters	1957

A.5.5.11.2.1.3	Test Requirements	1959

A.5.5.11.3	MAC-CE based active downlink TCI state switch	1959

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	1959

A.5.5.11.3.1.1	Test Purpose and Environment	1959

A.5.5.11.3.1.2	Test Parameters	1960

A.5.5.11.3.1.3	Test Requirements	1964

A.5.5.12	PSCell activation and deactivation delay	1964

A.5.5.12.1	PSCell activation and deactivation delay	1964

A.5.5.12.1.1	Test purpose and environment	1964

A.5.5.12.1.2	Test Requirements	1967

A.5.5.13	Conditional PSCell addition and release delay	1968

A.5.5.13.1	Addition and Release Delay of NR PSCell	1968

A.5.5.13.1.1	Test purpose and environment	1968

A.5.5.13.1.2	Test Requirements	1971

A.5.6	Measurement procedure	1972

A.5.6.1	Intra-frequency Measurements	1972

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1972

A.5.6.1.1.1	Test purpose and Environment	1972

A.5.6.1.1.2	Test Requirements	1975

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1975

A.5.6.1.2.1	Test purpose and Environment	1975

A.5.6.1.2.2	Test Requirements	1977

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1978

A.5.6.1.3.1	Test purpose and Environment	1978

A.5.6.1.3.2	Test Requirements	1982

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1982

A.5.6.1.4.1	Test purpose and Environment	1982

A.5.6.1.4.2	Test Requirements	1985

A.5.6.2	Inter-frequency Measurements	1986

A.5.6.2.1	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1986

A.5.6.2.1.1	Test Purpose and Environment	1986

A.5.6.2.1.2	Test Requirements	1989

A.5.6.2.2	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1989

A.5.6.2.2.1	Test Purpose and Environment	1989

A.5.6.2.2.2	Test Requirements	1993

A.5.6.2.3	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1993

A.5.6.2.3.1	Test Purpose and Environment	1993

A.5.6.2.3.2	Test Requirements	1998

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1998

A.5.6.2.4.1	Test Purpose and Environment	1998

A.5.6.2.4.2	Test Requirements	2002

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2002

A.5.6.2.5.1	Test Purpose and Environment	2002

A.5.6.2.5.2	Test Requirements	2007

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2007

A.5.6.2.6.1	Test Purpose and Environment	2007

A.5.6.2.6.2	Test Requirements	2011

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2011

A.5.6.2.7.1	Test Purpose and Environment	2011

A.5.6.2.7.2	Test Requirements	2016

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2016

A.5.6.2.8.1	Test Purpose and Environment	2016

A.5.6.2.8.2	Test Requirements	2021

A.5.6.3	L1-RSRP measurement for beam reporting	2022

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2022

A.5.6.3.1.1	Test Purpose and Environment	2022

A.5.6.3.1.2	Test parameters	2022

A.5.6.3.1.3	Test Requirements	2024

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2024

A.5.6.3.2.1	Test Purpose and Environment	2024

A.5.6.3.2.2	Test parameters	2025

A.5.6.3.2.3	Test Requirements	2027

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2027

A.5.6.3.3.1	Test Purpose and Environment	2027

A.5.6.3.3.2	Test parameters	2028

A.5.6.3.3.3	Test Requirements	2030

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2030

A.5.6.3.4.1	Test Purpose and Environment	2030

A.5.6.3.4.2	Test parameters	2031

A.5.6.3.4.3	Test Requirements	2033

A.5.6.4	CLI measurements	2034

A.5.6.4.1	SRS-RSRP measurement with DRX	2034

A.5.6.4.1.1	Test Purpose and Environment	2034

A.5.6.4.1.2	Test Parameters	2034

A.5.6.4.1.3	Test Requirements	2036

A.5.6.4.2	CLI-RSSI measurement with DRX	2036

A.5.6.4.2.1	Test Purpose and Environment	2036

A.5.6.4.2.2	Test Parameters	2037

A.5.6.4.2.3	Test Requirements	2039

A.5.6.5	Measurements with autonomous gaps	2039

A.5.6.5.1	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2039

A.5.6.5.1.1	Test Purpose and Environment	2039

A.5.6.5.1.2	Test Requirements	2042

A.5.6.6	L1-SINR measurement for beam reporting	2043

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2045

A.5.6.6.2.1	Test Purpose and Environment	2045

A.5.6.6.2.2	Test parameters	2046

A.5.6.6.2.3	Test Requirements	2049

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2049

A.5.6.6.3.1	Test Purpose and Environment	2049

A.5.6.6.3.2	Test parameters	2050

A.5.6.6.3.3	Test Requirements	2052

A.5.6.7	CSI-RS based Intra-frequency Measurements	2052

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2052

A.5.6.7.1.1	Test purpose and Environment	2052

A.5.6.7.1.2	Test Requirements	2055

A.5.6.8	CSI-RS based Inter-frequency Measurements	2055

A.5.6.8.1	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2055

A.5.6.8.1.1	Test Purpose and Environment	2055

A.5.6.8.1.2	Test Requirements	2060

A.5.7	Measurement Performance requirements	2060

A.5.7.1	SS-RSRP	2060

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2060

A.5.7.1.1.1	Test Purpose and Environment	2060

A.5.7.1.1.2	Test parameters	2061

A.5.7.1.1.3	Test Requirements	2063

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2064

A.5.7.1.2.1	Test Purpose and Environment	2064

A.5.7.1.2.2	Test parameters	2064

A.5.7.1.2.3	Test Requirements	2068

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2069

A.5.7.1.3.1	Test Purpose and Environment	2069

A.5.7.1.3.2	Test parameters	2069

A.5.7.1.3.3	Test Requirements	2072

A.5.7.2	SS-RSRQ	2073

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2073

A.5.7.2.1.1	Test Purpose and Environment	2073

A.5.7.2.1.2	Test Parameters	2073

A.5.7.2.1.3	Test Requirements	2076

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2076

A.5.7.2.2.1	Test Purpose and Environment	2076

A.5.7.2.2.2	Test Parameters	2076

A.5.7.2.2.3	Test Requirements	2078

A.5.7.3	SS-SINR	2079

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2079

A.5.7.3.1.1	Test Purpose and Environment	2079

A.5.7.3.1.2	Test Parameters	2079

A.5.7.3.1.3	Test Requirements	2082

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2082

A.5.7.3.2.1	Test Purpose and Environment	2082

A.5.7.3.2.2	Test Parameters	2082

A.5.7.3.2.3	Test Requirements	2084

A.5.7.4	L1-RSRP measurement for beam reporting	2084

A.5.7.4.1	SSB based L1-RSRP measurement	2084

A.5.7.4.1.1	Test Purpose and Environment	2084

A.5.7.4.1.2	Test parameters	2085

A.5.7.4.1.3	Test Requirements	2087

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2088

A.5.7.4.2.1	Test Purpose and Environment	2088

A.5.7.4.2.2	Test parameters	2088

A.5.7.4.2.3	Test Requirements	2090

A.5.7.5	CLI measurements	2091

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2091

A.5.7.5.1.1	Test Purpose and Environment	2091

A.5.7.5.1.2	Test parameters	2091

A.5.7.5.1.3	Test Requirements	2094

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2095

A.5.7.5.2.1	Test Purpose and Environment	2095

A.5.7.5.2.2	Test parameters	2095

A.5.7.5.2.3	Test Requirements	2097

A.5.7.6	L1-SINR measurement for beam reporting	2098

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2101

A.5.7.6.2.1	Test Purpose and Environment	2101

A.5.7.6.2.2	Test parameters	2101

A.5.7.6.2.3	Test Requirements	2103

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2104

A.5.7.6.3.1	Test Purpose and Environment	2104

A.5.7.6.3.2	Test parameters	2104

A.5.7.6.3.3	Test Requirements	2106

A.5.7.7	CSI-RSRP	2107

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2107

A.5.7.7.1.1	Test Purpose and Environment	2107

A.5.7.7.1.2	Test parameters	2107

A.5.7.7.1.3	Test Requirements	2109

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2110

A.5.7.7.2.1	Test Purpose and Environment	2110

A.5.7.7.2.2	Test parameters	2110

A.5.7.7.2.3	Test Requirements	2114

A.5.7.8	CSI-RSRQ	2115

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2115

A.5.7.8.1.1	Test Purpose and Environment	2115

A.5.7.8.1.2	Test Parameters	2115

A.5.7.8.1.3	Test Requirements	2117

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2117

A.5.7.8.2.1	Test Purpose and Environment	2117

A.5.7.8.2.2	Test Parameters	2117

A.5.7.8.2.3	Test Requirements	2119

A.5.7.9	CSI-SINR	2119

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2119

A.5.7.9.1.1	Test Purpose and Environment	2119

A.5.7.9.1.2	Test Parameters	2119

A.5.7.9.1.3	Test Requirements	2122

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2122

A.5.7.9.2.1	Test Purpose and Environment	2122

A.5.7.9.2.2	Test Parameters	2122

A.5.7.9.2.3	Test Requirements	2125

A.5.8	Void	2125

A.6	NR standalone tests with all NR cells in FR1	2173

A.6.1	SA: RRC_IDLE state mobility	2173

A.6.1.1	Cell re-selection to NR	2173

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2173

A.6.1.1.1.1	Test Purpose and Environment	2173

A.6.1.1.1.2	Test Parameters	2173

A.6.1.1.1.3	Test Requirements	2177

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2177

A.6.1.1.2.1	Test Purpose and Environment	2177

A.6.1.1.2.2	Test Parameters	2177

A.6.1.1.2.3	Test Requirements	2181

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2181

A.6.1.1.3.1	Test Purpose and Environment	2181

A.6.1.1.3.2	Test Parameters	2181

A.6.1.1.3.3	Test Requirements	2186

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2186

A.6.1.1.4.1	Test Purpose and Environment	2186

A.6.1.1.4.2	Test Parameters	2186

A.6.1.1.4.3	Test Requirements	2189

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2189

A.6.1.1.5.1	Test Purpose and Environment	2189

A.6.1.1.5.2	Test Parameters	2189

A.6.1.1.5.3	Test Requirements	2194

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2195

A.6.1.1.6.1	Test Purpose and Environment	2195

A.6.1.1.6.2	Test Parameters	2195

A.6.1.1.6.3	Test Requirements	2199

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2200

A.6.1.1.7.1	Test Purpose and Environment	2200

A.6.1.1.7.2	Test Parameters	2200

A.6.1.1.7.3	Test Requirements	2203

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2203

A.6.1.1.8.1	Test Purpose and Environment	2203

A.6.1.1.8.2	Test Parameters	2203

A.6.1.1.8.3	Test Requirements	2207

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2207

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2207

A.6.1.2.1.1	Test Purpose and Environment	2207

A.6.1.2.1.2	Test Parameters	2207

A.6.1.2.1.3	Test Requirements	2210

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2211

A.6.1.2.2.1	Test Purpose and Environment	2211

A.6.1.2.2.2	Test Parameters	2211

A.6.1.2.2.3	Test Requirements	2214

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2215

A.6.1.2.3.1	Test Purpose and Environment	2215

A.6.1.2.3.2	Test Parameters	2215

A.6.1.2.3.3	Test Requirements	2218

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2219

A.6.1.2.4.1	Test Purpose and Environment	2219

A.6.1.2.4.2	Test Parameters	2219

A.6.1.2.4.3	Test Requirements	2222

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2223

A.6.1.2.5.1	Test Purpose and Environment	2223

A.6.1.2.5.2	Test Parameters	2223

A.6.1.2.5.3	Test Requirements	2227

A.6.1.1.7	Void	2228

A.6.2	SA: RRC_INACTIVE state mobility	2228

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2228

A.6.2.1.1	Test purpose and Environment	2228

A.6.2.1.2	Test Parameters	2229

A.6.2.1.3	Test requirements	2231

A.6.3	RRC_CONNECTED state mobility	2231

A.6.3.1	Handover	2231

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2231

A.6.3.1.1.1	Test Purpose and Environment	2231

A.6.3.1.1.2	Test Parameters	2231

A.6.3.1.1.3	Test Requirements	2235

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2235

A.6.3.1.2.1	Test Purpose and Environment	2235

A.6.3.1.2.2	Test Parameters	2235

A.6.3.1.2.3	Test Requirements	2239

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2239

A.6.3.1.3.1	Test Purpose and Environment	2239

A.6.3.1.3.2	Test Parameters	2239

A.6.3.1.3.3	Test Requirements	2243

A.6.3.1.4	SA NR - E-UTRAN handover	2243

A.6.3.1.4.1	Test Purpose and Environment	2243

A.6.3.1.4.2	Test Requirements	2249

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2249

A.6.3.1.5.1	Test Purpose and Environment	2249

A.6.3.1.5.2	Test Requirements	2255

A.6.3.1.6	SA NR - UTRAN FDD handover	2255

A.6.3.1.6.1	Test Purpose and Environment	2255

A.6.3.1.6.2	Test Requirements	2259

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2259

A.6.3.1.7.1	Test Purpose and Environment	2259

A.6.3.1.7.2	Test Parameters	2259

A.6.3.1.7.3	Test Requirements	2263

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2264

A.6.3.1.8.1	Test Purpose and Environment	2264

A.6.3.1.8.2	Test Parameters	2264

A.6.3.1.8.3	Test Requirements	2268

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2269

A.6.3.1.9.1	Test Purpose and Environment	2269

A.6.3.1.9.2	Test Parameters	2269

A.6.3.1.9.3	Test Requirements	2273

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2273

A.6.3.1.10.1	Test Purpose and Environment	2273

A.6.3.1.10.2	Test Parameters	2273

A.6.3.1.10.3	Test Requirements	2276

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2276

A.6.3.1.11.1	Test Purpose and Environment	2276

A.6.3.1.11.2	Test Parameters	2276

A.6.3.1.11.3	Test Requirements	2283

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2283

A.6.3.1.12.1	Test Purpose and Environment	2283

A.6.3.1.12.2	Test Parameters	2283

A.6.3.1.12.3	Test Requirements	2291

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2291

A.6.3.1.13.1	Test Purpose and Environment	2291

A.6.3.1.13.2	Test Requirements	2300

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2300

A.6.3.1.14.1	Test Purpose and Environment	2300

A.6.3.1.14.2	Test Requirements	2309

A.6.3.2	RRC Connection Mobility Control	2310

A.6.3.2.1	SA: RRC Re-establishment	2310

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2310

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2314

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2318

A.6.3.2.2	Random Access	2321

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2321

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2326

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2330

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2335

A.6.3.2.3	SA: RRC Connection Release with Redirection	2339

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2339

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2343

A.6.3.3	Conditional handover	2350

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2350

A.6.3.3.1.1	Test Purpose and Environment	2350

A.6.3.3.1.2	Test Parameters	2350

A.6.3.3.1.3	Test Requirements	2354

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2354

A.6.3.3.2.1	Test Purpose and Environment	2354

A.6.3.3.2.2	Test Parameters	2354

A.6.3.3.2.3	Test Requirements	2358

A.6.4	Timing	2358

A.6.4.1	UE transmit timing	2358

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2358

A.6.4.1.1.1	Test Purpose and environment	2358

A.6.4.1.1.2	Test requirements	2362

A.6.4.2	UE timer accuracy	2362

A.6.4.3	Timing advance	2362

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2362

A.6.4.3.1.1	Test Purpose and Environment	2362

A.6.4.3.1.2	Test Parameters	2362

A.6.4.3.1.3	Test Requirements	2366

A.6.5	Signalling characteristics	2366

A.6.5.1	Radio link Monitoring	2366

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2367

A.6.5.1.1.1	Test Purpose and Environment	2367

A.6.5.1.1.2	Test Requirements	2372

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2372

A.6.5.1.2.1	Test Purpose and Environment	2372

A.6.5.1.2.2	Test Requirements	2378

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2378

A.6.5.1.3.1	Test Purpose and Environment	2378

A.6.5.1.3.2	Test Requirements	2384

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2384

A.6.5.1.4.1	Test Purpose and Environment	2384

A.6.5.1.4.2	Test Requirements	2390

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2390

A.6.5.1.5.1	Test Purpose and Environment	2390

A.6.5.1.5.2	Test Requirements	2396

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2396

A.6.5.1.6.1	Test Purpose and Environment	2396

A.6.5.1.6.2	Test Requirements	2401

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2401

A.6.5.1.7.1	Test Purpose and Environment	2401

A.6.5.1.7.2	Test Requirements	2406

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2406

A.6.5.1.8.1	Test Purpose and Environment	2406

A.6.5.1.8.2	Test Requirements	2411

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2411

A.6.5.1.9.1	Test Purpose and Environment	2411

A.6.5.1.9.2	Test Requirements	2417

A.6.5.2	Interruption	2417

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2417

A.6.5.2.1.2	Test Requirements	2421

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2422

A.6.5.2.2.1	Test Purpose and Environment	2422

A.6.5.2.2.2	Test Parameters	2422

A.6.5.2.2.3	Test Requirements	2424

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2425

A.6.5.2.3.1	Test Purpose and Environment	2425

A.6.5.2.3.2	Test Parameters	2425

A.6.5.2.3.3	Test Requirements	2427

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2428

A.6.5.2.4.1	Test Purpose and Environment	2428

A.6.5.2.4.2	Test Parameters	2428

A.6.5.2.4.3	Test Requirements	2430

A.6.5.3	SCell Activation and Deactivation Delay	2431

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2431

A.6.5.3.1.1	Test Purpose and Environment	2431

A.6.5.3.1.2	Test Requirements	2436

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2437

A.6.5.3.2.1	Test Purpose and Environment	2437

A.6.5.3.2.2	Test Requirements	2437

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2437

A.6.5.3.3.1	Test Purpose and Environment	2437

A.6.5.3.3.2	Test Requirements	2438

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2438

A.6.5.3.4.1	Test Purpose and Environment	2438

A.6.5.3.4.2	Test Requirements	2445

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2445

A.6.5.3.5.1	Test Purpose and Environment	2445

A.6.5.3.5.2	Test Requirements	2450

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2450

A.6.5.3.6.1	Test Purpose and Environment	2450

A.6.5.3.6.2	Test Requirements	2454

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2454

A.6.5.3.7.1	Test Purpose and Environment	2454

A.6.5.3.7.2	Test Requirements	2458

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2459

A.6.5.3.8.1	Test Purpose and Environment	2459

A.6.5.3.8.2	Test Requirements	2463

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2464

A.6.5.3.9.1	Test Purpose and Environment	2464

A.6.5.3.9.2	Test Requirements	2468

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2469

A.6.5.3.10.1	Test Purpose and Environment	2469

A.6.5.3.10.2	Test Requirements	2472

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2473

A.6.5.3.11.1	Test Purpose and Environment	2473

A.6.5.3.11.2	Test Requirements	2473

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2473

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2473

A.6.5.4.1.1	Test Purpose and Environment	2473

A.6.5.4.1.2	Test Requirements	2483

A.6.5.4.2	Void	2484

A.6.5.5	Beam Failure Detection and Link recovery procedures	2484

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2484

A.6.5.5.1.1	Test Purpose and Environment	2484

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2490

A.6.5.5.2.1	Test Purpose and Environment	2490

A.6.5.5.2.2	Test Requirements	2496

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2497

A.6.5.5.3.1	Test Purpose and Environment	2497

A.6.5.5.3.2	Test Requirements	2502

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2503

A.6.5.5.4.1	Test Purpose and Environment	2503

A.6.5.5.4.2	Test Requirements	2508

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2509

A.6.5.5.5.1	Test Purpose and Environment	2509

A.6.5.5.5.2	Test Requirements	2513

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2513

A.6.5.5.6.1	Test Purpose and Environment	2513

A.6.5.5.6.2	Test Requirements	2518

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2518

A.6.5.5.7.1	Test Purpose and Environment	2518

A.6.5.5.7.2	Test Requirements	2524

A.6.5.6	Active BWP switch	2525

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2525

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2525

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2531

A.6.5.6.2	RRC-based Active BWP Switch	2536

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2536

A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2540

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2540

A.6.5.6.4	SCell dormancy switch	2547

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2547

A.6.5.6.4.1.1	Test Purpose and Environment	2547

A.6.5.6.4.1.2	Test Requirements	2553

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2553

A.6.5.6.4.2.1	Test Purpose and Environment	2553

A.6.5.6.4.2.2	Test Requirements	2560

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2560

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2560

A.6.5.7	DL interruptions at switching between two uplink carriers	2565

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2565

A.6.5.7.1.1	Test Purpose and Environment	2565

A.6.5.7.1.2	Test Requirements	2569

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2569

A.6.5.7.2.1	Test Purpose and Environment	2569

A.6.5.7.2.2	Test Requirements	2573

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2573

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2573

A.6.5.7A.1.1	Test Purpose and Environment	2573

A.6.5.7A.1.2	Test Requirements	2577

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2577

A.6.5.7A.2.1	Test Purpose and Environment	2577

A.6.5.7A.2.2	Test Requirements	2581

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2581

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2581

A.6.5.7B.1.1	Test Purpose and Environment	2581

A.6.5.7B.1.2	Test Requirements	2585

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2585

A.6.5.7B.2.1	Test Purpose and Environment	2585

A.6.5.7B.2.2	Test Requirements	2589

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2589

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2589

A.6.5.7C.1.1	Test Purpose and Environment	2589

A.6.5.7C.1.2	Test Requirements	2593

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2594

A.6.5.7C.2.1	Test Purpose and Environment	2594

A.6.5.7C.2.2	Test Requirements	2598

A.6.5.8	UE specific CBW change	2599

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2599

A.6.5.8.1.1	Test Purpose and Environment	2599

A.6.5.8.1.2	Test Requirements	2603

A.6.5.9	Pathloss reference signal switching delay	2603

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2603

A.6.5.9.1.1	Test Purpose and Environment	2603

A.6.5.9.1.2	Test Requirements	2606

A.6.6	Measurement procedure	2607

A.6.6.1	Intra-frequency Measurements	2607

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2607

A.6.6.1.1.1	Test purpose and Environment	2607

A.6.6.1.1.2	Test parameters	2607

A.6.6.1.1.3	Test Requirements	2611

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2611

A.6.6.1.2.1	Test purpose and Environment	2611

A.6.6.1.2.2	Test parameters	2611

A.6.6.1.2.3	Test Requirements	2615

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2615

A.6.6.1.3.1	Test purpose and Environment	2615

A.6.6.1.3.2	Test parameters	2615

A.6.6.1.3.3	Test Requirements	2619

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2619

A.6.6.1.4.1	Test purpose and Environment	2619

A.6.6.1.4.2	Test parameters	2619

A.6.6.1.4.3	Test Requirements	2623

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2623

A.6.6.1.5.1	Test purpose and Environment	2623

A.6.6.1.5.2	Test parameters	2623

A.6.6.1.5.3	Test Requirements	2625

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2626

A.6.6.1.6.1	Test purpose and Environment	2626

A.6.6.1.6.2	Test parameters	2626

A.6.6.1.6.3	Test Requirements	2627

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2628

A.6.6.1.7.1	Test purpose and Environment	2628

A.6.6.1.7.2	Test parameters	2628

A.6.6.1.7.3	Test Requirements	2632

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2632

A.6.6.1.8.1	Test purpose and Environment	2632

A.6.6.1.8.2	Test parameters	2632

A.6.6.1.8.3	Test Requirements	2636

A.6.6.2	Inter-frequency Measurements	2636

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2636

A.6.6.2.1.1	Test Purpose and Environment	2636

A.6.6.2.1.2	Test Requirements	2640

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2640

A.6.6.2.2.1	Test Purpose and Environment	2640

A.6.6.2.2.2	Test Requirements	2644

A.6.6.2.3	Void	2645

A.6.6.2.4	Void	2645

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2645

A.6.6.2.5.1	Test Purpose and Environment	2645

A.6.6.2.5.2	Test Requirements	2649

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2649

A.6.6.2.6.1	Test Purpose and Environment	2649

A.6.6.2.6.2	Test Requirements	2653

A.6.6.2.7	Void	2653

A.6.6.2.8	Void	2653

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2653

A.6.6.2.9.1	Test Purpose and Environment	2653

A.6.6.2.9.2	Test Requirements	2657

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2657

A.6.6.2.10.1	Test Purpose and Environment	2657

A.6.6.2.10.2	Test Requirements	2661

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2665

A.6.6.2.12.1	Test Purpose and Environment	2665

A.6.6.2.12.2	Test Requirements	2669

A.6.6.3	Inter-RAT Measurements	2669

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2669

A.6.6.3.1.1	Test Purpose and Environment	2669

A.6.6.3.1.2	Test Requirements	2675

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2675

A.6.6.3.2.1	Test Purpose and Environment	2675

A.6.6.3.2.2	Test Requirements	2682

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2682

A.6.6.3.3.1	Test Purpose and Environment	2682

A.6.6.3.3.2	Test Requirements	2687

A.6.6.4	L1-RSRP measurement for beam reporting	2687

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2687

A.6.6.4.1.1	Test Purpose and Environment	2687

A.6.6.4.1.2	Test parameters	2688

A.6.6.4.1.3	Test Requirements	2691

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2691

A.6.6.4.2.1	Test Purpose and Environment	2691

A.6.6.4.2.2	Test parameters	2692

A.6.6.4.2.3	Test Requirements	2695

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2695

A.6.6.4.3.1	Test Purpose and Environment	2695

A.6.6.4.3.2	Test parameters	2696

A.6.6.4.3.3	Test Requirements	2699

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2699

A.6.6.4.4.1	Test Purpose and Environment	2699

A.6.6.4.4.2	Test parameters	2700

A.6.6.4.4.3	Test Requirements	2703

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2703

A.6.6.4.5.1	Test Purpose and Environment	2703

A.6.6.4.5.2	Test parameters	2704

A.6.6.4.5.3	Test Requirements	2707

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2707

A.6.6.4.6.1	Test Purpose and Environment	2707

A.6.6.4.6.2	Test parameters	2708

A.6.6.4.6.3	Test Requirements	2711

A.6.6.5	Inter-RAT UTRAN FDD measurements	2712

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2712

A.6.6.5.1.1	Test Purpose and Environment	2712

A.6.6.5.1.2	Test Requirements	2715

A.6.6.6	CLI measurements	2715

A.6.6.6.1	SRS-RSRP measurement with DRX	2715

A.6.6.6.1.1	Test Purpose and Environment	2715

A.6.6.6.1.2	Test Parameters	2716

A.6.6.6.1.3	Test Requirements	2719

A.6.6.6.2	CLI-RSSI measurement with DRX	2719

A.6.6.6.2.1	Test Purpose and Environment	2719

A.6.6.6.2.2	Test Parameters	2720

A.6.6.6.2.3	Test Requirements	2722

A.6.6.7	NR measurements with autonomous gaps	2722

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2722

A.6.6.7.1.1	Test Purpose and Environment	2722

A.6.6.7.1.2	Test Parameters	2722

A.6.6.7.1.3	Test Requirements	2725

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2725

A.6.6.7.2.1	Test Purpose and Environment	2725

A.6.6.7.2.2	Test Requirements	2728

A.6.6.8	L1-SINR measurement for beam reporting	2729

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2731

A.6.6.8.2.1	Test Purpose and Environment	2731

A.6.6.8.2.2	Test parameters	2731

A.6.6.8.2.3	Test Requirements	2735

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2735

A.6.6.8.3.1	Test Purpose and Environment	2735

A.6.6.8.3.2	Test parameters	2736

A.6.6.8.3.3	Test Requirements	2738

A.6.6.9	Idle Mode CA/DC Measurements	2738

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2738

A.6.6.9.1.1	Test Purpose and Environment	2738

A.6.6.9.1.2	Test Requirements	2745

A.6.6.10	CSI-RS based intra-frequency Measurements	2745

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2745

A.6.6.10.1.1	Test purpose and Environment	2745

A.6.6.10.1.2	Test Requirements	2748

A.6.6.11	CSI-RS based inter-frequency Measurements	2748

A.6.6.11.1	SA event triggered reporting tests with gap under DRX	2748

A.6.6.11.1.1	Test Purpose and Environment	2748

A.6.6.11.1.2	Test Requirements	2752

A.6.6.12	RSTD measurements	2752

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2752

A.6.6.12.1.1	Test Purpose and Environment	2752

A.6.6.12.1.2	Test Requirements	2759

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2760

A.6.6.12.2.1	Test Purpose and Environment	2760

A.6.6.12.2.2	Test Requirements	2766

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2766

A.6.6.12.3.1	Test Purpose and Environment	2766

A.6.6.12.3.2	Test Requirements	2772

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2772

A.6.6.12.4.1	Test Purpose and Environment	2772

A.6.6.12.4.2	Test Requirements	2776

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2777

A.6.6.12.5.1	Test Purpose and Environment	2777

A.6.6.12.5.2	Test Requirements	2781

A.6.6.13	PRS-RSRP measurements	2781

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2781

A.6.6.13.1.1	Test purpose and Environment	2781

A.6.6.13.1.2	Test Requirements	2785

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2785

A.6.6.13.2.1	Test purpose and Environment	2785

A.6.6.13.2.2	Test Requirements	2789

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2789

A.6.6.13.3.1	Test purpose and Environment	2789

A.6.6.13.3.2	Test Requirements	2793

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2793

A.6.6.13.4.1	Test purpose and Environment	2793

A.6.6.14	UE Rx-Tx time difference measurements	2797

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2797

A.6.6.14.1.1	Test purpose and environment	2797

A.6.6.14.1.2	Test requirements	2801

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2801

A.6.6.14.2.1	Test purpose and environment	2801

A.6.6.14.2.2	Test requirements	2805

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2805

A.6.6.14.3.1	Test purpose and environment	2805

A.6.6.14.3.2	Test requirements	2809

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2809

A.6.6.14.4.1	Test purpose and environment	2809

A.6.6.14.4.2	Test requirements	2813

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2813

A.6.6.14.4.1	Test purpose and environment	2813

A.6.6.14.4.2	Test requirements	2817

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2817

A.6.6.15.1	Test Purpose and Environment	2817

A.6.6.15.2	Test Requirements	2824

A.6.6.16	PRS-RSRPP measurements	2825

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2825

A.6.6.16.1.1	Test purpose and Environment	2825

A.6.6.16.1.2	Test Requirements	2827

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2827

A.6.6.16.2.1	Test purpose and Environment	2827

A.6.6.16.2.2	Test Requirements	2830

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2830

A.6.6.16.3.1	Test purpose and Environment	2830

A.6.6.16.3.2	Test Requirements	2834

A.6.6.17	SA event triggered reporting tests with Pre-MG	2834

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2834

A.6.6.17.1.1	Test purpose and Environment	2834

A.6.6.17.1.2	Test parameters	2834

A.6.6.17.1.3	Test Requirements	2838

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2838

A.6.6.17.2.1	Test purpose and Environment	2838

A.6.6.17.2.2	Test parameters	2838

A.6.6.17.2.3	Test Requirements	2843

A.6.6.17.3	Void	2843

A.6.6.17.3.1	Void	2843

A.6.6.17.3.2	Void	2843

A.6.6.17.3.3	Void	2843

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2843

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2843

A.6.6.18.1.1	Test Purpose and Environment	2843

A.6.6.18.1.2	Test Requirements	2847

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2847

A.6.6.18.2.1	Test Purpose and Environment	2847

A.6.6.18.2.2	Test Requirements	2851

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2851

A.6.6.18.3.1	Test Purpose and Environment	2851

A.6.6.18.3.2	Test Requirements	2858

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2858

A.6.6.18.4.1	Test Purpose and Environment	2858

A.6.6.18.4.2	Test Requirements	2862

A.6.6.19	SA event triggered reporting tests with NCSG	2862

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2862

A.6.6.19.1.1	Test purpose and Environment	2862

A.6.6.19.1.2	Test parameters	2863

A.6.6.19.1.3	Test Requirements	2867

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2867

A.6.6.19.2.1	Test Purpose and Environment	2867

A.6.6.19.2.2	Test parameters	2867

A.6.6.19.2.3	Test Requirements	2871

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2871

A.6.6.19.3.1	Test Purpose and Environment	2871

A.6.6.19.3.2	Test parameters	2872

A.6.6.19.3.3	Test Requirements	2877

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2877

A.6.6.19.4.1	Test purpose and Environment	2877

A.6.6.19.4.2	Test parameters	2877

A.6.6.19.4.3	Test Requirements	2880

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2880

A.6.6.20.1	Test purpose and environment	2880

A.6.6.20.2	Test requirements	2884

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2884

A.6.6.21.1	Test purpose and environment	2884

A.6.6.21.2	Test requirements	2886

A.6.7	Measurement Performance requirements	2887

A.6.7.1	SS-RSRP	2887

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2887

A.6.7.1.1.1	Test Purpose and Environment	2887

A.6.7.1.1.2	Test parameters	2887

A.6.7.1.1.3	Test Requirements	2892

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2892

A.6.7.1.2.1	Test Purpose and Environment	2892

A.6.7.1.2.2	Test parameters	2892

A.6.7.1.2.3	Test Requirements	2896

A.6.7.1.3	Void	2896

A.6.7.2	SS-RSRQ	2896

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2896

A.6.7.2.1.1	Test Purpose and Environment	2896

A.6.7.2.1.2	Test Parameters	2897

A.6.7.2.1.3	Test Requirements	2901

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2901

A.6.7.2.2.1	Test Purpose and Environment	2901

A.6.7.2.2.2	Test Parameters	2901

A.6.7.2.2.3	Test Requirements	2906

A.6.7.3	SS-SINR	2906

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2906

A.6.7.3.1.1	Test Purpose and Environment	2906

A.6.7.3.1.2	Test Parameters	2907

A.6.7.3.1.3	Test Requirements	2911

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2912

A.6.7.3.2.1	Test Purpose and Environment	2912

A.6.7.3.2.2	Test Parameters	2912

A.6.7.3.2.3	Test Requirements	2916

A.6.7.4	L1-RSRP measurement for beam reporting	2917

A.6.7.4.1	SSB based L1-RSRP measurement	2917

A.6.7.4.1.1	Test Purpose and Environment	2917

A.6.7.4.1.2	Test parameters	2917

A.6.7.4.1.3	Test Requirements	2921

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2921

A.6.7.4.2.1	Test Purpose and Environment	2921

A.6.7.4.2.2	Test parameters	2922

A.6.7.4.2.3	Test Requirements	2926

A.6.7.5	E-UTRAN RSRP	2926

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2926

A.6.7.5.1.1	Test Purpose and Environment	2926

A.6.7.5.1.2	Test parameters	2927

A.6.7.5.1.3	Test Requirements	2933

A.6.7.6	E-UTRAN RSRQ	2934

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2934

A.6.7.6.1.1	Test Purpose and Environment	2934

A.6.7.6.1.2	Test parameters	2934

A.6.7.6.1.3	Test Requirements	2939

A.6.7.7	E-UTRAN RS-SINR	2940

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	2940

A.6.7.7.1.1	Test Purpose and Environment	2940

A.6.7.7.1.2	Test parameters	2940

A.6.7.7.1.3	Test Requirements	2946

A.6.7.8	CLI measurements	2947

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	2947

A.6.7.8.1.1	Test Purpose and Environment	2947

A.6.7.8.1.2	Test parameters	2947

A.6.7.8.1.3	Test Requirements	2953

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	2953

A.6.7.8.2.1	Test Purpose and Environment	2953

A.6.7.8.2.2	Test parameters	2954

A.6.7.8.2.3	Test Requirements	2957

A.6.7.9	L1-SINR measurement for beam reporting	2958

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2962

A.6.7.9.2.1	Test Purpose and Environment	2962

A.6.7.9.2.2	Test parameters	2963

A.6.7.9.2.3	Test Requirements	2968

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2968

A.6.7.9.3.1	Test Purpose and Environment	2968

A.6.7.9.3.2	Test parameters	2968

A.6.7.9.3.3	Test Requirements	2973

A.6.7.10	CSI-RSRP	2973

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2973

A.6.7.9.10.1	Test Purpose and Environment	2973

A.6.7.9.10.2	Test parameters	2974

A.6.7.10.1.3	Test Requirements	2979

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2979

A.6.7.9.10.1	Test Purpose and Environment	2979

A.6.7.10.2.2	Test parameters	2979

A.6.7.10.2.3	Test Requirements	2984

A.6.7.11	CSI-RSRQ	2984

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2984

A.6.7.11.1.1	Test Purpose and Environment	2984

A.6.7.11.1.2	Test Parameters	2984

A.6.7.11.1.3	Test Requirements	2989

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2989

A.6.7.11.2.1	Test Purpose and Environment	2989

A.6.7.11.2.2	Test Parameters	2989

A.6.7.11.2.3	Test Requirements	2995

A.6.7.12	CSI-SINR	2996

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2996

A.6.7.12.1.1	Test Purpose and Environment	2996

A.6.7.12.1.2	Test Parameters	2996

A.6.7.12.1.3	Test Requirements	3000

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3001

A.6.7.12.2.1	Test Purpose and Environment	3001

A.6.7.12.2.2	Test Parameters	3001

A.6.7.12.2.3	Test Requirements	3007

A.6.7.13	RSTD measurements	3007

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	3007

A.6.7.13.1.1	Test purpose and Environment	3007

A.6.7.13.1.2	Test Requirements	3010

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	3010

A.6.7.13.2.1	Test purpose and Environment	3010

A.6.7.13.2.2	Test Requirements	3014

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3014

A.6.7.13.3.1	Test purpose and Environment	3014

A.6.7.13.3.2	Test Requirements	3016

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	3016

A.6.7.14	PRS-RSRP measurements	3020

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	3020

A.6.7.14.1.1	Test Purpose and Environment	3020

A.6.7.14.1.2	Test parameters	3021

A.6.7.14.1.3	Test Requirements	3025

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	3025

A.6.7.14.2.1	Test Purpose and Environment	3025

A.6.7.14.2.2	Test parameters	3025

A.6.7.14.2.3	Test Requirements	3028

A.6.7.14.3	Void	3029

A.6.7.14.3.1	Void	3029

A.6.7.14.3.2	Void	3029

A.6.7.14.3.3	Void	3029

A.6.7.15	UE Rx-Tx time difference measurements	3029

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	3029

A.6.7.15.1.1	Test purpose and environment	3029

A.6.7.15.1.2	Test parameters	3029

A.6.7.15.1.3	Test requirements	3032

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3033

A.6.7.15.2.1	Test purpose and environment	3033

A.6.7.15.2.2	Test parameters	3033

A.6.7.15.2.3	Test requirements	3036

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3036

A.6.7.15.3.1	Test purpose and environment	3036

A.6.7.15.3.2	Test parameters	3037

A.6.7.15.3.3	Test requirements	3040

A.6.7.16	PRS-RSRPP measurements	3040

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3040

A.6.7.16.1.1	Test Purpose and Environment	3040

A.6.7.16.1.2	Test parameters	3041

A.6.7.16.1.3	Test Requirements	3046

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3046

A.6.7.16.2.1	Test Purpose and Environment	3046

A.6.7.16.2.2	Test parameters	3046

A.6.8	Measurement procedure in RRC_INACTIVE	3050

A.6.8.1	RSTD measurements	3050

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3050

A.6.8.1.1.1	Test Purpose and Environment	3050

A.6.8.1.1.2	Test Requirements	3053

A.6.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3053

A.6.8.1.2.1	Test Purpose and Environment	3053

A.6.8.1.2.2	Test Requirements	3058

A.6.8.2	PRS-RSRP measurements	3058

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3058

A.6.8.2.1.1	Test purpose and Environment	3058

A.6.8.2.1.2	Test Requirements	3060

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3061

A.6.8.2.2.1	Test purpose and Environment	3061

A.6.8.2.2.2	Test Requirements	3064

A.6.8.3	UE Rx-Tx time difference measurements	3064

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3064

A.6.8.3.1.1	Test purpose and environment	3064

A.6.8.3.1.2	Test requirements	3068

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3068

A.6.8.3.2.1	Test purpose and environment	3068

A.6.8.3.2.2	Test requirements	3071

A.6.8.4	PRS-RSRPP measurements	3071

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3071

A.6.8.4.1.1	Test purpose and Environment	3071

A.6.8.4.1.2	Test Requirements	3074

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3074

A.6.8.4.2.1	Test purpose and Environment	3074

A.6.8.4.2.2	Test Requirements	3076

A.6.9	Measurement performance requirements in RRC_INACTIVE	3076

A.6.9.1	RSTD measurements	3076

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3076

A.6.9.1.1.1	Test purpose and Environment	3076

A.6.9.1.1.2	Test Requirements	3078

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3078

A.6.9.1.2.1	Test purpose and Environment	3078

A.6.9.1.2.2	Test Requirements	3080

A.6.9.2	PRS-RSRP measurements	3080

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3080

A.6.9.2.1.1	Test Purpose and Environment	3080

A.6.9.2.1.2	Test parameters	3081

A.6.9.2.1.3	Test Requirements	3086

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3086

A.6.9.2.2.1	Test Purpose and Environment	3086

A.6.9.2.2.2	Test parameters	3086

A.6.9.2.2.3	Test Requirements	3089

A.6.9.3	UE Rx-Tx time difference measurements	3090

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3090

A.6.9.3.1.1.1	Test purpose and environment	3090

A.6.9.3.1.1.2	Test parameters	3090

A.6.9.3.1.1.3	Test requirements	3093

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3093

A.6.9.3.2.1	Test purpose and environment	3093

A.6.9.3.2.2	Test parameters	3093

A.6.9.3.2.3	Test requirements	3096

A.6.9.4	PRS-RSRPP measurements	3096

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3096

A.6.9.4.1.1	Test Purpose and Environment	3096

A.6.9.4.1.2	Test parameters	3096

A.6.9.4.1.3	Test Requirements	3101

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3101

A.6.9.4.2.1	Test Purpose and Environment	3101

A.6.9.4.2.2	Test parameters	3101

A.6.9.4.2.3	Test Requirements	3104

A.7	NR standalone tests with one or more NR cells in FR2	3106

A.7.1	SA: RRC_IDLE state mobility	3106

A.7.1.1	Cell re-selection to NR	3106

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3106

A.7.1.1.1.1	Test Purpose and Environment	3106

A.7.1.1.1.2	Test Parameters	3106

A.7.1.1.1.3	Test Requirements	3109

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3109

A.7.1.1.2.1	Test Purpose and Environment	3109

A.7.1.1.2.2	Test Parameters	3110

A.7.1.1.2.3	Test Requirements	3112

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3112

A.7.1.1.3.1	Test Purpose and Environment	3112

A.7.1.1.3.2	Test Parameters	3112

A.7.1.1.3.3	Test Requirements	3115

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3115

A.7.1.1.4.1	Test Purpose and Environment	3115

A.7.1.1.4.2	Test Parameters	3115

A.7.1.1.4.3	Test Requirements	3118

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3118

A.7.1.1.5.1	Test Purpose and Environment	3118

A.7.1.1.5.2	Test Parameters	3118

A.7.1.1.5.3	Test Requirements	3121

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3121

A.7.1.1.6.1	Test Purpose and Environment	3121

A.7.1.1.6.2	Test Parameters	3121

A.7.1.1.6.3	Test Requirements	3124

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3124

A.7.1.1.7.1	Test Purpose and Environment	3124

A.7.1.1.7.2	Test Parameters	3124

A.7.1.1.7.3	Test Requirements	3128

A.7.2	SA: RRC_INACTIVE state mobility	3128

A.7.2.1	Small Data Transmission	3128

A.7.2.1.1	TA validation for CG-SDT in FR2	3128

A.7.2.1.1.1	Test Purpose and Environment	3128

A.7.2.1.1.2	Test Requirements	3132

A.7.3	RRC_CONNECTED state mobility	3132

A.7.3.1	Handover	3132

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3132

A.7.3.1.1.1	Test Purpose and Environment	3132

A.7.3.1.1.2	Test Parameters	3132

A.7.3.1.1.3	Test Requirements	3136

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3136

A.7.3.1.2.1	Test Purpose and Environment	3136

A.7.3.1.2.2	Test Parameters	3136

A.7.3.1.2.3	Test Requirements	3139

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3139

A.7.3.1.3.1	Test Purpose and Environment	3139

A.7.3.1.3.2	Test Parameters	3139

A.7.3.1.3.3	Test Requirements	3140

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3141

A.7.3.1.4.1	Test Purpose and Environment	3141

A.7.3.1.4.2	Test Parameters	3141

A.7.3.1.4.3 Test Requirements	3148

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3148

A.7.3.1.5.1	Test Purpose and Environment	3148

A.7.3.1.5.2	Test Parameters	3148

A.7.3.1.5.3 Test Requirements	3155

A.7.3.1.6	Handover with PSCell from SA to EN-DC; unknown FR2 target cell	3155

A.7.3.1.6.1	Test Purpose and Environment	3155

A.7.3.1.6.2	Test Parameters	3155

A.7.3.1.6.3	Test Requirements	3160

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3160

A.7.3.1.7.1	Test purpose and environment	3160

A.7.3.1.7.2	Test Requirements	3166

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3166

A.7.3.1.8.1	Test Purpose and Environment	3166

A.7.3.1.8.2	Test Requirements	3172

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3172

A.7.3.1.9.1	Test Purpose and Environment	3172

A.7.3.1.9.2	Test Parameters	3172

A.7.3.1.9.3	Test Requirements	3175

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3175

A.7.3.1.10.1	Test Purpose and Environment	3175

A.7.3.1.10.2	Test Parameters	3175

A.7.3.1.10.3	Test Requirements	3179

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3179

A.7.3.1.11.1	Test Purpose and Environment	3179

A.7.3.1.11.2	Test Parameters	3179

A.7.3.1.11.3	Test Requirements	3183

A.7.3.2	RRC Connection Mobility Control	3183

A.7.3.2.1	SA: RRC Re-establishment	3183

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3183

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3186

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3189

A.7.3.2.1.3.1	Test Purpose and Environment	3189

A.7.3.2.1.3.2	Test Requirements	3191

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3192

A.7.3.2.1.4.1	Test Purpose and Environment	3192

A.7.3.2.1.4.2	Test Requirements	3194

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3195

A.7.3.2.1.5.1	Test Purpose and Environment	3195

A.7.3.2.1.5.2	Test Requirements	3198

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3198

A.7.3.2.1.6.1	Test Purpose and Environment	3198

A.7.3.2.1.6.2	Test Requirements	3200

A.7.3.2.2	Random Access	3201

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3201

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3205

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3209

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3212

A.7.3.2.3	SA: RRC Connection Release with Redirection	3215

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3215

A.7.3.3 Conditional Handover	3219

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3219

A.7.3.3.1.1	Test Purpose and Environment	3219

A.7.3.3.1.2	Test Parameters	3219

A.7.3.3.1.2.3	Test Requirements	3222

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3222

A.7.3.3.2.1	Test Purpose and Environment	3222

A.7.3.3.2.2	Test Parameters	3222

A.7.3.3.2.3 Test Requirements	3224

A.7.4	Timing	3224

A.7.4.1	UE transmit timing	3224

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3224

A.7.4.1.1.1	Test Purpose and environment	3224

A.7.4.1.1.2	Test requirements	3227

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3227

A.7.4.1.2.1	Test Purpose and environment	3227

A.7.4.1.2.2	Test requirements	3232

A.7.4.2	UE timer accuracy	3233

A.7.4.3	Timing advance	3233

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3233

A.7.4.3.1.1	Test Purpose and Environment	3233

A.7.4.3.1.2	Test Parameters	3233

A.7.4.3.1.3 Test Requirements	3236

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3237

A.7.4.3.2.1	Test Purpose and Environment	3237

A.7.4.3.2.2	Test Parameters	3237

A.7.4.3.2.3	Test Requirements	3241

A.7.5	Signaling characteristics	3241

A.7.5.1	Radio link Monitoring	3241

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3241

A.7.5.1.1.1	Test Purpose and Environment	3241

A.7.5.1.1.2	Test Requirements	3244

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3245

A.7.5.1.2.1	Test Purpose and Environment	3245

A.7.5.1.2.2	Test Requirements	3249

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3250

A.7.5.1.3.1	Test Purpose and Environment	3250

A.7.5.1.3.2	Test Requirements	3254

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3254

A.7.5.1.4.1	Test Purpose and Environment	3254

A.7.5.1.4.2	Test Requirements	3259

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3259

A.7.5.1.5.1	Test Purpose and Environment	3259

A.7.5.1.5.2	Test Requirements	3264

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3264

A.7.5.1.6.1	Test Purpose and Environment	3264

A.7.5.1.6.2	Test Requirements	3268

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3268

A.7.5.1.7.1	Test Purpose and Environment	3268

A.7.5.1.7.2	Test Requirements	3272

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3272

A.7.5.1.8.1	Test Purpose and Environment	3272

A.7.5.1.8.2	Test Requirements	3277

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3277

A.7.5.1.9.1	Test Purpose and Environment	3277

A.7.5.1.9.2	Test Requirements	3280

A.7.5.2	Interruption	3280

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3280

A.7.5.2.1.1	Test Purpose and Environment	3280

A.7.5.2.1.2	Test Requirements	3283

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3284

A.7.5.2.2.1	Test Purpose and Environment	3284

A.7.5.2.2.2	Test Parameters	3284

A.7.5.2.2.3	Test Requirements	3286

A.7.5.3	SCell Activation and Deactivation Delay	3286

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3286

A.7.5.3.1.1	Test Purpose and Environment	3286

A.7.5.3.1.2	Test Requirements	3288

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3288

A.7.5.3.2.1	Test Purpose and Environment	3288

A.7.5.3.2.2	Test Requirements	3292

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3293

A.7.5.3.3.1	Test Purpose and Environment	3293

A.7.5.3.3.2	Test Requirements	3296

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3297

A.7.5.3.4.1	Test Purpose and Environment	3297

A.7.5.3.4.2	Test Requirements	3300

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3301

A.7.5.3.5.1	Test Purpose and Environment	3301

A.7.5.3.5.2	Test Requirements	3304

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3305

A.7.5.3.6.1	Test Purpose and Environment	3305

A.7.5.3.6.2	Test Requirements	3309

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3310

A.7.5.3.7.1	Test Purpose and Environment	3310

A.7.5.3.7.2	Test Requirements	3314

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3315

A.7.5.3.8.1	Test Purpose and Environment	3315

A.7.5.3.8.2	Test Requirements	3319

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3320

A.7.5.3.9.1	Test Purpose and Environment	3320

A.7.5.3.9.2	Test Requirements	3323

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3324

A.7.5.3.10.1	Test Purpose and Environment	3324

A.7.5.3.10.2	Test Requirements	3328

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3329

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3329

A.7.5.3.11.1.1	Test Purpose and Environment	3329

A.7.5.3.11.1.2	Test Requirements	3333

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3334

A.7.5.3.11.2.1	Test Purpose and Environment	3334

A.7.5.3.11.2.2	Test Requirements	3338

A.7.5.3.12	Void	3339

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3339

A.7.5.3.13.1	Test Purpose and Environment	3339

A.7.5.3.13.2	Test Requirements	3341

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3342

A.7.5.3.14.1	Test Purpose and Environment	3342

A.7.5.3.14.2	Test Requirements	3345

A.7.5.15	Void	3346

A.7.5.4	Void	3346

A.7.5.5	Beam Failure Detection and Link recovery procedures	3346

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3346

A.7.5.5.1.1	Test Purpose and Environment	3346

A.7.5.5.1.2	Test Requirements	3350

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3351

A.7.5.5.2.1	Test Purpose and Environment	3351

A.7.5.5.2.2	Test Requirements	3354

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3355

A.7.5.5.3.1	Test Purpose and Environment	3355

A.7.5.5.3.2	Test Requirements	3359

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3360

A.7.5.5.4.1	Test Purpose and Environment	3360

A.7.5.5.4.2	Test Requirements	3364

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3365

A.7.5.5.5.1	Test Purpose and Environment	3365

A.7.5.5.5.2	Test Requirements	3368

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3369

A.7.5.5.6.1	Test Purpose and Environment	3369

A.7.5.5.6.2	Test Requirements	3373

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3374

A.7.5.5.7.1	Test Purpose and Environment	3374

A.7.5.5.7.2	Test Requirements	3378

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3379

A.7.5.5.8.1	Test Purpose and Environment	3379

A.7.5.5.8.2	Test Requirements	3383

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3383

A.7.5.5.9.1	Test Purpose and Environment	3383

A.7.5.5.9.2	Test Requirements	3389

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3389

A.7.5.5.10.1	Test Purpose and Environment	3389

A.7.5.5.10.2	Test Requirements	3395

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3395

A.7.5.5.11.1	Test Purpose and Environment	3395

A.7.5.5.11.2	Test Requirements	3400

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3400

A.7.5.5.12.1	Test Purpose and Environment	3400

A.7.5.5.12.2	Test Requirements	3405

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3405

A.7.5.5.13.1	Test Purpose and Environment	3405

A.7.5.5.13.2	Test Requirements	3409

A.7.5.6	Active BWP switch	3409

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3409

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3409

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3414

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3419

A.7.5.6.1.3.1	Test Purpose and Environment	3419

A.7.5.6.1.3.2	Test Requirements	3422

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3422

A.7.5.6.1.4.1	Test Purpose and Environment	3422

A.7.5.6.1.4.2	Test Requirements	3427

A.7.5.6.2	RRC-based Active BWP Switch	3428

A.7.5.6.2.1.1	Test Purpose and Environment	3428

A.7.5.6.2.1.2	Test Requirements	3431

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3432

A.7.5.6.2.2.1	Test Purpose and Environment	3432

A.7.5.6.2.2.2	Test Requirements	3436

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3437

A.7.5.6.3.1.1	Test Purpose and Environment	3437

A.7.5.6.3.1.2	Test Requirements	3440

A.7.5.6.4	SCell dormancy switch	3440

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3440

A.7.5.6.4.1.1	Test Purpose and Environment	3440

A.7.5.6.4.1.2	Test Requirements	3445

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3445

A.7.5.6.4.2.1	 Test Purpose and Environment	3445

A.7.5.6.4.2.2	 Test Requirements	3450

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3450

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3450

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3453

A.7.5.6.5.2.1	Test Purpose and Environment	3453

A.7.5.6.5.2.2	Test Requirements	3457

A.7.5.7	PSCell addition and release delay	3458

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3458

A.7.5.7.1.1	Test Purpose and Environment	3458

A.7.5.7.1.2	Test Requirements	3461

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3461

A.7.5.7.2.1	Test Purpose and Environment	3461

A.7.5.7.2.2	Test Requirements	3464

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3464

A.7.5.7.3.1	Test Purpose and Environment	3464

A.7.5.7.3.2	Test Requirements	3467

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3467

A.7.5.7.4.1	Test Purpose and Environment	3467

A.7.5.7.4.2	Test Requirements	3470

A.7.5.8	Active TCI state switch delay	3470

A.7.5.8.1	MAC-CE based active TCI state switch	3470

A.7.5.8.2	RRC based active TCI state switch	3474

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3478

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3478

A.7.5.8.3.1.1	Test Purpose and Environment	3478

A.7.5.8.3.1.2	Test Requirements	3482

A.7.5.9	Uplink spatial relation switch delay	3483

A.7.5.9.1.1.1	Test Purpose and Environment	3483

A.7.5.9.1.1.2	Test Requirements	3486

A.7.5.9.2	RRC based spatial relation switch	3486

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3486

A.7.5.9.2.1.1	Test Purpose and Environment	3486

A.7.5.9.2.1.2	Test Requirements	3489

A.7.5.10	UE specific CBW change	3489

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3489

A.7.5.10.1.1	Test Purpose and Environment	3489

A.7.5.10.1.2	Test Requirements	3492

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3493

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3493

A.7.5.11.1.1	Test Purpose and Environment	3493

A.7.5.11.1.2	Test Requirements	3496

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3496

A.7.5.12.1	Addition and Release Delay of PSCell	3496

A.7.5.12.1.1	Test purpose and environment	3496

A.7.5.12.1.2	Test Parameters	3496

A.7.5.12.1.3	Test Requirements	3499

A.7.5.13	 Unified TCI state switching delay	3499

A.7.5.13.1	MAC-CE based active joint TCI state switching	3499

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3499

A.7.5.13.1.1.1	Test Purpose and Environment	3499

A.7.5.13.1.1.2	Test parameters	3500

A.7.5.13.1.1.3	Test Requirements	3502

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3502

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3502

A.7.5.13.2.1.1	Test Purpose and Environment	3502

A.7.5.13.2.1.2	Test parameters	3503

A.7.5.13.2.1.3	Test Requirements	3505

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3505

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3505

A.7.5.13.3.1.1	Test Purpose and Environment	3505

A.7.5.13.3.1.2	Test Parameters	3506

A.7.5.13.3.1.3	Test Requirements	3509

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3509

A.7.5.14.1	Test Purpose and Environment	3509

A.7.5.14.2	Test Requirements	3512

A.7.6	Measurement procedure	3513

A.7.6.1	Intra-frequency Measurements	3513

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3513

A.7.6.1.1.1	Test purpose and Environment	3513

A.7.6.1.1.2	Test Requirements	3516

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3516

A.7.6.1.2.1	Test purpose and Environment	3516

A.7.6.1.2.2	Test Requirements	3519

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3520

A.7.6.1.3.1	Test purpose and Environment	3520

A.7.6.1.3.2	Test Requirements	3523

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3523

A.7.6.1.4.1	Test purpose and Environment	3523

A.7.6.1.4.2	Test Requirements	3526

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasCA-Scell-r17	3527

A.7.6.1.5.1	Test purpose and Environment	3527

A.7.6.1.5.2	Test Requirements	3530

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3530

A.7.6.1.6.1	Test purpose and Environment	3530

A.7.6.1.6.2	Test Requirements	3533

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3534

A.7.6.1.7.1	Test purpose and Environment	3534

A.7.6.1.7.2	Test Requirements	3536

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3537

A.7.6.1.8.1	Test purpose and Environment	3537

A.7.6.1.8.2	Test Requirements	3540

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3541

A.7.6.1.9.1	Test purpose and Environment	3541

A.7.6.1.9.2	Test Requirements	3543

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3544

A.7.6.1.10.1	Test purpose and Environment	3544

A.7.6.1.10.2	Test Requirements	3547

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3548

A.7.6.1.11.1	Test purpose and Environment	3548

A.7.6.1.11.2	Test Requirements	3550

A.7.6.2	Inter-frequency Measurements	3551

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3551

A.7.6.2.1.1	Test Purpose and Environment	3551

A.7.6.2.1.2	Test Requirements	3554

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3554

A.7.6.2.2.1	Test Purpose and Environment	3554

A.7.6.2.2.2	Test Requirements	3558

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3558

A.7.6.2.3.1	Test Purpose and Environment	3558

A.7.6.2.3.2	Test Requirements	3562

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3562

A.7.6.2.4.1	Test Purpose and Environment	3562

A.7.6.2.4.2	Test Requirements	3566

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3566

A.7.6.2.5.1	Test Purpose and Environment	3566

A.7.6.2.5.2	Test Requirements	3570

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3571

A.7.6.2.6.1	Test Purpose and Environment	3571

A.7.6.2.6.2	Test Requirements	3575

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3576

A.7.6.2.7.1	Test Purpose and Environment	3576

A.7.6.2.7.2	Test Requirements	3580

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3581

A.7.6.2.8.1	Test Purpose and Environment	3581

A.7.6.2.8.2	Test Requirements	3585

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3586

A.7.6.2.9.1	Test Purpose and Environment	3586

A.7.6.2.9.2	Test Requirements	3590

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3590

A.7.6.2.10.1	Test Purpose and Environment	3590

A.7.6.2.10.2	Test Requirements	3592

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3592

A.7.6.2.11.1	Test Purpose and Environment	3592

A.7.6.2.11.2	Test Requirements	3595

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3595

A.7.6.2.12.1	Test Purpose and Environment	3595

A.7.6.2.12.2	Test Requirements	3599

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3600

A.7.6.2.13.1	Test Purpose and Environment	3600

A.7.6.2.13.2	Test Requirements	3604

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3605

A.7.6.2.14.1	Test Purpose and Environment	3605

A.7.6.2.14.2	Test Requirements	3609

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3610

A.7.6.2.15.1	Test Purpose and Environment	3610

A.7.6.2.15.2	Test Requirements	3614

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3615

A.7.6.2.16.1	Test Purpose and Environment	3615

A.7.6.2.16.2	Test Requirements	3621

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3621

A.7.6.2.17.1	Test Purpose and Environment	3621

A.7.6.2.17.2	Test Requirements	3628

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3629

A.7.6.2.18.1	Test Purpose and Environment	3629

A.7.6.2.18.2	Test Requirements	3634

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3635

A.7.6.2.19.1	Test Purpose and Environment	3635

A.7.6.2.19.2	Test Requirements	3643

A.7.6.3	L1-RSRP measurement for beam reporting	3644

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3644

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3644

A.7.6.3.1.1	Test Purpose and Environment	3644

A.7.6.3.1.2	Test parameters	3644

A.7.6.3.1.3	Test Requirements	3646

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3646

A.7.6.3.2.1	Test Purpose and Environment	3646

A.7.6.3.2.2	Test parameters	3647

A.7.6.3.2.3	Test Requirements	3649

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3649

A.7.6.3.3.1	Test Purpose and Environment	3649

A.7.6.3.3.2	Test parameters	3650

A.7.6.3.3.3	Test Requirements	3652

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3653

A.7.6.3.4.1	Test Purpose and Environment	3653

A.7.6.3.4.2	Test parameters	3653

A.7.6.3.3.3	Test Requirements	3655

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3656

A.7.6.3.5.1	Test Purpose and Environment	3656

A.7.6.3.5.2	Test parameters	3656

A.7.6.3.5.3	Test Requirements	3658

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3658

A.7.6.3.6.1	Test Purpose and Environment	3658

A.7.6.3.6.2	Test parameters	3659

A.7.6.3.6.3	Test Requirements	3662

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3662

A.7.6.3.7.1	Test Purpose and Environment	3662

A.7.6.3.7.2	Test parameters	3663

A.7.6.3.7.3	Test Requirements	3666

A.7.6.4	CLI measurements	3667

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3667

A.7.6.4.1.1	Test Purpose and Environment	3667

A.7.6.4.1.2	Test Parameters	3667

A.7.6.4.1.3	Test Requirements	3669

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3669

A.7.6.4.2.1	Test Purpose and Environment	3669

A.7.6.4.2.2	Test Parameters	3670

A.7.6.4.2.3	Test Requirements	3671

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3672

A.7.6.5.1.1	Test Purpose and Environment	3672

A.7.6.5.1.2	Test Requirements	3675

A.7.6.6	L1-SINR measurement for beam reporting	3675

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3677

A.7.6.6.2.1	Test Purpose and Environment	3677

A.7.6.6.2.2	Test parameters	3678

A.7.6.6.2.3	Test Requirements	3680

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3680

A.7.6.6.3.1	Test Purpose and Environment	3680

A.7.6.6.3.2	Test parameters	3681

A.7.6.6.3.3	Test Requirements	3683

A.7.6.7	CSI-RS based intra-frequency Measurements	3683

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3683

A.7.6.7.1.1	Test purpose and Environment	3683

A.7.6.7.1.2	Test Requirements	3686

A.7.6.8	CSI-RS based inter-frequency Measurements	3686

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3686

A.7.6.8.1.1	Test Purpose and Environment	3686

A.7.6.8.1.2	Test Requirements	3689

A.7.6.9	RSTD measurements	3689

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3689

A.7.6.9.1.1	Test Purpose and Environment	3689

A.7.6.9.1.2	Test Requirements	3697

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3697

A.7.6.9.2.1	Test Purpose and Environment	3697

A.7.6.9.2.2	Test Requirements	3705

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3705

A.7.6.9.3.1	Test Purpose and Environment	3705

A.7.6.9.3.2	Test Requirements	3711

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3712

A.7.6.9.4.1	Test Purpose and Environment	3712

A.7.6.9.4.2	Test Requirements	3715

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3715

A.7.6.9.5.1	Test Purpose and Environment	3715

A.7.6.9.5.2	Test Requirements	3719

A.7.6.10 PRS-RSRP measurements	3719

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3719

A.7.6.10.1.1	Test Purpose and Environment	3719

A.7.6.10.1.2	Test Requirements	3723

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3723

A.7.6.10.2.1	Test Purpose and Environment	3723

A.7.6.10.2.2	Test Requirements	3727

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3727

A.7.6.10.3.1	Test Purpose and Environment	3727

A.7.6.10.3.2	Test Requirements	3731

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3731

A.7.6.10.4.1	Test Purpose and Environment	3731

A.7.6.10.4.2	Test Requirements	3735

A.7.6.11	UE Rx-Tx time difference measurements	3735

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3735

A.7.6.11.1.1	Test purpose and environment	3735

A.7.6.11.1.2	Test requirements	3739

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3739

A.7.6.11.2.1	Test purpose and environment	3739

A.7.6.11.2.2	Test requirements	3743

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3743

A.7.6.11.3.1	Test purpose and environment	3743

A.7.6.11.3.2	Test requirements	3745

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3746

A.7.6.11.4.1	Test purpose and environment	3746

A.7.6.11.4.2	Test requirements	3748

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3749

A.7.6.11.5.1	Test purpose and environment	3749

A.7.6.11.5.2	Test requirements	3751

A.7.6.12	PRS-RSRPP measurements	3752

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3752

A.7.6.12.1.1	Test Purpose and Environment	3752

A.7.6.12.1.2	Test Requirements	3754

A.7.6.12.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3755

A.7.6.12.2.1	Test Purpose and Environment	3755

A.7.6.12.2.2	Test Requirements	3757

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3757

A.7.6.12.3.1	Test Purpose and Environment	3757

A.7.6.12.3.2	Test Requirements	3761

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3761

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3761

A.7.6.13.1.1	Test purpose and environment	3761

A.7.6.13.1.2	Test requirements	3763

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3764

A.7.6.13.2.1	Test purpose and environment	3764

A.7.6.13.2.2	Test requirements	3765

A.7.6.14	SA event triggered reporting tests with Pre-MG	3766

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3766

A.7.6.14.1.1	Test purpose and Environment	3766

A.7.6.14.1.2	Test parameters	3766

A.7.6.14.1.3	Test Requirements	3768

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3769

A.7.6.14.2.1	Test purpose and Environment	3769

A.7.6.14.2.2	Test parameters	3769

A.7.6.14.2.3	Test Requirements	3771

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3772

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3772

A.7.6.15.1.1	Test Purpose and Environment	3772

A.7.6.15.1.2	Test Requirements	3775

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3775

A.7.6.15.2.1	Test Purpose and Environment	3775

A.7.6.15.2.2	Test Requirements	3778

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overalpping scenario for SSB-based measurements and PRS-based measurement	3778

A.7.6.15.3.1	Test Purpose and Environment	3778

A.7.6.15.3.2	Test Requirements	3782

A.7.6.16	SA event triggered reporting tests with NCSG	3782

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3782

A.7.6.16.1.1	Test purpose and Environment	3782

A.7.6.16.1.2	Test Requirements	3786

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3786

A.7.6.16.2.1	Test Purpose and Environment	3786

A.7.6.16.2.2	Test Requirements	3789

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3789

A.7.6.16.3.1	Test Purpose and Environment	3789

A.7.6.16.3.2	Test Requirements	3792

A.7.7	Measurement Performance requirements	3792

A.7.7.1	SS-RSRP	3793

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3793

A.7.7.1.1.1	Test Purpose and Environment	3793

A.7.7.1.1.2	Test parameters	3793

A.7.7.1.1.3	Test Requirements	3797

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3798

A.7.7.1.2.1	Test Purpose and Environment	3798

A.7.7.1.2.2	Test parameters	3798

A.7.7.1.2.3	Test Requirements	3802

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3803

A.7.7.1.3.1	Test Purpose and Environment	3803

A.7.7.1.3.2	Test parameters	3803

A.7.7.1.3.3	Test Requirements	3807

A.7.7.2	SS-RSRQ	3807

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3807

A.7.7.2.1.1	Test Purpose and Environment	3807

A.7.7.2.1.2	Test Parameters	3807

A.7.7.2.1.3	Test Requirements	3809

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3809

A.7.7.2.2.1	Test Purpose and Environment	3809

A.7.7.2.2.2	Test Parameters	3809

A.7.7.2.2.3	Test Requirements	3811

A.7.7.3	SS-SINR	3811

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3811

A.7.7.3.1.1	Test Purpose and Environment	3811

A.7.7.3.1.2	Test Parameters	3811

A.7.7.3.1.3	Test Requirements	3813

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3813

A.7.7.3.2.1	Test Purpose and Environment	3813

A.7.7.3.2.2	Test Parameters	3813

A.7.7.3.2.3	Test Requirements	3815

A.7.7.4	L1-RSRP measurement for beam reporting	3815

A.7.7.4.1	SSB based L1-RSRP measurement	3815

A.7.7.4.1.1	Test Purpose and Environment	3815

A.7.7.4.1.2	Test parameters	3816

A.7.7.4.1.3	Test Requirements	3818

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3819

A.7.7.4.2.1	Test Purpose and Environment	3819

A.7.7.4.2.2	Test parameters	3819

A.7.7.4.2.3	Test Requirements	3821

A.7.7.5	CLI measurements	3822

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3822

A.7.7.5.1.1	Test Purpose and Environment	3822

A.7.7.5.1.2	Test parameters	3822

A.7.7.5.1.3	Test Requirements	3825

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3826

A.7.7.5.2.1	Test Purpose and Environment	3826

A.7.7.5.2.2	Test parameters	3826

A.7.7.5.2.3	Test Requirements	3828

A.7.7.6	L1-SINR measurement for beam reporting	3829

A.7.7.6.1.1	Test Purpose and Environment	3829

A.7.7.6.1.2	Test parameters	3829

A.7.7.6.1.3	Test Requirements	3831

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3832

A.7.7.6.2.1	Test Purpose and Environment	3832

A.7.7.6.2.2	Test parameters	3832

A.7.7.6.2.3	Test Requirements	3834

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3835

A.7.7.6.3.1	Test Purpose and Environment	3835

A.7.7.6.3.2	Test parameters	3835

A.7.7.6.3.3	Test Requirements	3837

A.7.7.7	CSI-RSRP	3838

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3838

A.7.7.7.1.1	Test Purpose and Environment	3838

A.7.7.7.1.2	Test parameters	3838

A.7.7.7.1.3	Test Requirements	3841

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3842

A.7.7.7.2.1	Test Purpose and Environment	3842

A.7.7.7.2.2	Test parameters	3842

A.7.7.7.2.3	Test Requirements	3845

A.7.7.8	CSI-RSRQ	3846

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3846

A.7.7.8.1.1	Test Purpose and Environment	3846

A.7.7.8.1.2	Test Parameters	3846

A.7.7.8.1.3	Test Requirements	3848

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3848

A.7.7.8.2.1	Test Purpose and Environment	3848

A.7.7.8.2.2	Test Parameters	3848

A.7.7.8.2.3	Test Requirements	3850

A.7.7.9	CSI-SINR	3850

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3850

A.7.7.9.1.1	Test Purpose and Environment	3850

A.7.7.9.1.2	Test Parameters	3850

A.7.7.9.1.3	Test Requirements	3852

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3852

A.7.7.9.2.1	Test Purpose and Environment	3852

A.7.7.9.2.2	Test Parameters	3853

A.7.7.9.2.3	Test Requirements	3855

A.7.7.10	RSTD measurements	3855

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3855

A.7.7.10.1.1	Test purpose and Environment	3855

A.7.7.10.1.2	Test Requirements	3858

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3858

A.7.7.10.2.1	Test purpose and Environment	3858

A.7.7.10.2.2	Test Requirements	3862

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3863

A.7.7.10.3.1	Test purpose and Environment	3863

A.7.7.10.3.2	Test Requirements	3864

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3865

A.7.7.10.4.1	Test purpose and Environment	3865

A.7.7.10.4.2	Test Requirements	3867

A.7.7.11	PRS-RSRP measurements	3867

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3867

A.7.7.11.1.1	Test Purpose and Environment	3867

A.7.7.11.1.2	Test parameters	3867

A.7.7.11.1.3	Test Requirements	3871

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3872

A.7.7.11.2.1	Test Purpose and Environment	3872

A.7.7.11.2.2	Test parameters	3872

A.7.7.11.2.3	Test Requirements	3874

A.7.7.12	UE Rx-Tx time difference measurements	3875

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3875

A.7.7.12.1.1	Test purpose and environment	3875

A.7.7.12.1.2	Test parameters	3875

A.7.7.12.1.3	Test requirements	3878

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3878

A.7.7.12.2.1	Test purpose and environment	3878

A.7.7.12.2.2	Test parameters	3879

A.7.7.12.2.3	Test requirements	3882

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3882

A.7.7.12.3.1	Test purpose and environment	3882

A.7.7.12.3.2	Test parameters	3882

A.7.7.12.3.3	Test requirements	3885

A.7.7.13	PRS-RSRPP measurements	3885

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3885

A.7.7.13.1.1	Test Purpose and Environment	3885

A.7.7.13.1.2	Test parameters	3885

A.7.7.13.1.3	Test Requirements	3888

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3889

A.7.7.13.2.1	Test Purpose and Environment	3889

A.7.7.13.2.2	Test parameters	3889

A.7.7.13.2.3	Test Requirements	3891

A.7.8	Measurement procedure in RRC_INACTIVE	3892

A.7.8.1	RSTD measurements	3892

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3892

A.7.8.1.1.1	Test Purpose and Environment	3892

A.7.8.1.1.2	Test Requirements	3895

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3895

A.7.8.1.2.1	Test Purpose and Environment	3895

A.7.8.1.2.2	Test Requirements	3901

A.7.8.2	PRS-RSRP measurements	3902

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3902

A.7.8.2.1.1	Test Purpose and Environment	3902

A.7.8.2.1.2	Test Requirements	3906

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3906

A.7.8.2.2.1	Test purpose and Environment	3906

A.7.8.2.2.2	Test Requirements	3910

A.7.8.3	UE Rx-Tx time difference measurements	3910

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3910

A.7.8.3.1.1	Test purpose and environment	3910

A.7.8.3.1.2	Test requirements	3914

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3914

A.7.8.3.2.1	Test purpose and environment	3914

A.7.8.3.2.2	Test requirements	3916

A.7.8.4	PRS-RSRPP measurements	3917

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3917

A.7.8.4.1.1	Test Purpose and Environment	3917

A.7.8.4.1.2	Test Requirements	3919

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3919

A.7.8.4.2.1	Test Purpose and Environment	3919

A.7.8.4.2.2	Test Requirements	3922

A.7.9	Measurement performance requirements in RRC_INACTIVE	3922

A.7.9.1	RSTD measurements	3922

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3922

A.7.9.1.1.1	Test purpose and Environment	3922

A.7.9.1.1.2	Test Requirements	3924

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3924

A.7.9.1.2.1	Test purpose and Environment	3924

A.7.9.1.2.2	Test Requirements	3926

A.7.9.2	PRS-RSRP measurements	3926

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	3926

A.7.9.2.1.1	Test Purpose and Environment	3926

A.7.9.2.1.2	Test parameters	3926

A.7.9.2.1.3	Test Requirements	3928

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	3929

A.7.9.2.2.1	Test Purpose and Environment	3929

A.7.9.2.2.2	Test parameters	3929

A.7.9.2.2.3	Test Requirements	3931

A.7.9.3	UE Rx-Tx time difference measurements	3932

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	3932

A.7.9.3.1.1	Test purpose and environment	3932

A.7.9.3.1.2	Test parameters	3932

A.7.9.3.1.3	Test requirements	3934

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3934

A.7.9.3.2.1	Test purpose and environment	3934

A.7.9.3.2.2	Test parameters	3934

A.7.9.3.2.3	Test requirements	3937

A.7.9.4	PRS-RSRPP measurements	3937

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	3937

A.7.9.4.1.1	Test Purpose and Environment	3937

A.7.9.4.1.2	Test parameters	3937

A.7.9.4.1.3	Test Requirements	3939

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	3940

A.7.9.4.2.1	Test Purpose and Environment	3940

A.7.9.4.2.2	Test parameters	3940

A.7.9.4.2.3	Test Requirements	3942

A.8	E-UTRA standalone tests for NR RRM	3944

A.8.1	Void	3944

A.8.2	RRC_IDLE state mobility	3944

A.8.2.1	Inter-RAT NR Cell re-selection	3944

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	3944

A.8.2.1.1.1	Test Purpose and Environment	3944

A.8.2.1.1.2	Test Requirements	3949

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3950

A.8.2.1.2.1	Test Purpose and Environment	3950

A.8.2.1.2.2	Test Requirements	3954

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	3955

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	3955

A.8.2.2.1.1	Test Purpose and Environment	3955

A.8.2.2.1.2	Test Requirements	3959

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	3959

A.8.2.2.2.1	Test Purpose and Environment	3959

A.8.2.2.2.2	Test Requirements	3962

A.8.3	RRC_CONNECTED state mobility	3963

A.8.3.1	Handover	3963

A.8.3.1.1	E-UTRAN - NR handover in FR1	3963

A.8.3.1.1.1	Test Purpose and Environment	3963

A.8.3.1.1.2	Test Requirements	3968

A.8.4	Measurement procedure	3968

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	3968

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	3968

A.8.4.1.1.1	Test Purpose and Environment	3968

A.8.4.1.1.2	Test Requirements	3970

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	3971

A.8.4.1.2.1	Test Purpose and Environment	3971

A.8.4.1.2.2	Test Requirements	3972

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	3972

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3972

A.8.4.2.1.1	Test Purpose and Environment	3972

A.8.4.2.1.2	Test Requirements	3977

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3977

A.8.4.2.2.1	Test Purpose and Environment	3977

A.8.4.2.2.2	Test Requirements	3981

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3981

A.8.4.2.3.1	Test Purpose and Environment	3981

A.8.4.2.3.2	Test Requirements	3986

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3986

A.8.4.2.4.1	Test Purpose and Environment	3986

A.8.4.2.4.2	Test Requirements	3990

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3990

A.8.4.2.5.1	Test Purpose and Environment	3990

A.8.4.2.5.2	Test Requirements	3992

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3993

A.8.4.2.6.1	Test Purpose and Environment	3993

A.8.4.2.6.2	Test Requirements	3995

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3996

A.8.4.2.7.1	Test Purpose and Environment	3996

A.8.4.2.7.2	Test Requirements	3998

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3999

A.8.4.2.8.1	Test Purpose and Environment	3999

A.8.4.2.8.2	Test Requirements	4001

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	4002

A.8.4.2.9.1	Test Purpose and Environment	4002

A.8.4.2.9.2	Test Requirements	4006

A.8.5	Measurement performance	4006

A.8.5.1	SFTD accuracy	4006

A.8.5.1.1	SFTD accuracy	4006

A.8.5.1.1.1	Test Purpose	4006

A.8.5.1.1.2	Test Environment	4007

A.8.5.1.1.3	Test Requirements	4013

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	4013

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4013

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4018

A.8.5.2.1.2.1	Test Purpose and Environment	4018

A.8.5.2.1.2.2	Test Parameters	4018

A.8.5.2.1.2.3	Test Requirements	4020

A.8.5.2.2	SS-RSRQ	4020

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4020

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4025

A.8.5.2.2.2.1	Test Purpose and Environment	4025

A.8.5.2.2.2.2	Test Parameters	4025

A.8.5.2.2.2.3	Test Requirements	4027

A.8.5.2.3	SS-SINR	4027

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4027

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4031

A.8.5.2.3.2.1	Test Purpose and Environment	4031

A.8.5.2.3.2.2	Test Parameters	4031

A.8.5.2.3.2.3	Test Requirements	4033

A.9	V2X Tests	4034

A.9.1	V2X Tests in FR1	4034

A.9.1.1	Test for V2X UE Transmit Timing	4034

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	4034

A.9.1.1.1.1	Test Purpose and Environment	4034

A.9.1.1.1.2	Test requirements	4034

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	4034

A.9.1.1.2.1	Test Purpose and Environment	4034

A.9.1.1.2.2	Test requirements	4035

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4035

A.9.1.1.3.1	Test Purpose and Environment	4035

A.9.1.1.3.2	Test requirements	4038

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4038

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4038

A.9.1.2.1.1	Test Purpose and Environment	4038

A.9.1.2.1.2	Test Requirements	4041

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4041

A.9.1.2.2.1	Test Purpose and Environment	4041

A.9.1.2.2.2	Test Requirements	4042

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4043

A.9.1.2.3.1	Test Purpose and Environment	4043

A.9.1.2.3.2	Test Requirements	4045

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4045

A.9.1.3.1	 Test for GNSS configured as the highest priority	4045

A.9.1.3.1.1	Test Purpose and Environment	4045

A.9.1.3.1.2	Test Requirements	4047

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4048

A.9.1.3.2.1	Test Purpose and Environment	4048

A.9.1.3.2.2	Test Requirements	4050

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4051

A.9.1.3.3.1	Test Purpose and Environment	4051

A.9.1.3.3.2	Test Requirements	4053

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4054

A.9.1.3.4.1	Test Purpose and Environment	4054

A.9.1.3.4.2	Test Requirements	4056

A.9.1.4	Test for L1 SL-RSRP Measurement	4057

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4057

A.9.1.4.1.1	Test Purpose and Environment	4057

A.9.1.4.1.2	Test Requirements	4060

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4060

A.9.1.4.2.1	Test Purpose and Environment	4060

A.9.1.4.2.2	Test Requirements	4064

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4064

A.9.1.4.3.1	Test Purpose and Environment	4064

A.9.1.4.3.2	Test Requirements	4071

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4071

A.9.1.4.4.1	Test Purpose and Environment	4071

A.9.1.4.4.2	Test Requirements	4075

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4075

A.9.1.4.5.1	Test Purpose and Environment	4075

A.9.1.4.5.2	Test Requirements	4078

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4078

A.9.1.4.6.1	Test Purpose and Environment	4078

A.9.1.4.6.2	Test Requirements	4082

A.9.1.5	Test for Congestion Control Measurement	4082

A.9.1.5.1	Test Purpose and Environment	4082

A.9.1.5.2	Test Requirements	4088

A.9.1.6	Test for Interruption	4088

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4088

A.9.1.6.1.1	Test Purpose and Environment	4088

A.9.1.6.1.2	Test Requirements	4092

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4092

A.9.1.6.2.1	Test Purpose and Environment	4092

A.9.1.6.2.2	Test Requirements	4096

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4096

A.9.1.6.3.1	Test Purpose and Environment	4096

A.9.1.6.3.2	Test Requirements	4099

A.9.1.7	Selection / Reselection of relay UE	4099

A.9.1.7.1	Test Purpose and Environment	4099

A.9.1.7.2	Test Requirements	4105

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4106

A.10.1	RRC_CONNECTED state mobility	4106

A.10.1.1	RRC connection mobility control	4106

A.10.1.1.1	Random Access	4106

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4106

A.10.1.1.1.1.1	Test Purpose and Environment	4106

A.10.1.1.1.1.2	Test Requirements	4109

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4109

A.10.1.1.1.1.2.2	Random Access Response Reception	4109

A.10.1.1.1.1.2.3	No Random Access Response Reception	4110

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4110

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4110

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4111

A.10.1.1.1.2.1	Test Purpose and Environment	4111

A.10.1.1.1.2.2	Test Requirements	4114

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4114

A.10.1.1.1.2.2.2	Random Access Response Reception	4115

A.10.1.1.1.2.2.3	No Random Access Response Reception	4115

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4115

A.10.1.1.1.3.1	Test Purpose and Environment	4115

A.10.1.1.1.3.2	Test Requirements	4119

A.10.1.1.1.3.2.1	MsgA Transmission	4119

A.10.1.1.1.3.2.2	MsgB Reception	4119

A.10.1.1.1.3.2.3	No MsgB Reception	4120

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4120

A.10.1.1.1.4.1	Test Purpose and Environment	4120

A.10.1.1.1.4.2	Test Requirements	4124

A.10.1.1.1.4.2.1	MsgA Transmission	4124

A.10.1.1.1.4.2.2	MsgB Reception	4125

A.10.1.1.1.4.2.3	No MsgB Reception	4125

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4125

A.10.1.2.1	Test Purpose and Environment	4125

A.10.1.2.2	Test Requirements	4131

A.10.2	Timing	4132

A.10.2.1	UE transmit timing	4132

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4132

A.10.2.1.1.1	Test Purpose and environment	4132

A.10.2.1.1.2	Test requirements	4135

A.10.2.2	UE timing advance	4136

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4136

A.10.2.2.1.1	Test Purpose and Environment	4136

A.10.2.2.1.2	Test Parameters	4136

A.10.2.2.1.3	Test Requirements	4140

A.10.3	Signalling characteristics	4140

A.10.3.1	Radio link monitoring	4140

A.10.3.1.1	Introduction	4140

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4141

A.10.3.1.2.1	Test purpose and environment	4141

A.10.3.1.2.2	Test requirements	4145

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4146

A.10.3.1.3.1	Test purpose and environment	4146

A.10.3.1.3.2	Test requirements	4151

A.10.3.1.4	Void	4151

A.10.3.1.4.1	Void	4151

A.10.3.1.4.2	Void	4151

A.10.3.1.5	Void	4151

A.10.3.1.5.1	Void	4151

A.10.3.1.5.2	Void	4151

A.10.3.2	Interruption	4151

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	4151

A.10.3.2.1.1	Test Purpose and Environment	4151

A.10.3.2.1.2	Test Requirements	4156

A.10.3.3	SCell activation and deactivation delay	4156

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4156

A.10.3.3.1.1	Test Purpose and Environment	4156

A.10.3.3.1.2	Test Requirements	4161

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4161

A.10.3.3.2.1	Test Purpose and Environment	4161

A.10.3.3.2.2	Test Requirements	4162

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4162

A.10.3.3.3.1	Test Purpose and Environment	4162

A.10.3.3.3.2	Test Requirements	4163

A.10.3.4	Beam failure detection and link recovery procedures	4163

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4163

A.10.3.4.1.1	Test Purpose and Environment	4163

A.10.3.4.1.2	Test Requirements	4168

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4168

A.10.3.4.2.1	Test Purpose and Environment	4168

A.10.3.4.2.2	Test Requirements	4174

A.10.3.5	Active BWP switching	4174

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4174

A.10.3.5.1.2	Test Requirements	4179

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4180

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4180

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4183

A.10.3.5.3	RRC-based Active BWP Switch	4187

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4187

A.10.3.6	PSCell addition and release delay	4191

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4191

A.10.3.6.1.1	Test purpose and environment	4191

A.10.3.6.1.2	Test Requirements	4196

A.10.3.7	Void	4197

A.10.4	Measurement procedure	4197

A.10.4.1	Intra-frequency measurements	4197

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4197

A.10.4.1.1.1	Test purpose and environment	4197

A.10.4.1.1.2	Test parameters	4197

A.10.4.1.1.3	Test Requirements	4197

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	4198

A.10.4.1.2.1	Test purpose and environment	4198

A.10.4.1.2.2	Test parameters	4198

A.10.4.1.2.3	Test Requirements	4198

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	4198

A.10.4.1.3.1	Test purpose and environment	4198

A.10.4.1.3.2	Test parameters	4198

A.10.4.1.3.3	Test Requirements	4198

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4199

A.10.4.1.4.1	Test purpose and environment	4199

A.10.4.1.4.2	Test parameters	4199

A.10.4.1.4.3	Test Requirements	4199

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4199

A.10.4.1.5.1	Test purpose and environment	4199

A.10.4.1.5.2	Test parameters	4199

A.10.4.1.5.3	Test Requirements	4199

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4200

A.10.4.1.6.1	Test purpose and environment	4200

A.10.4.1.6.2	Test parameters	4200

A.10.4.1.6.3	Test Requirements	4200

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4200

A.10.4.1.7.1	Test purpose and environment	4200

A.10.4.1.7.2	Test parameters	4200

A.10.4.1.7.3	Test Requirements	4200

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4201

A.10.4.1.8.1	Test purpose and environment	4201

A.10.4.1.8.2	Test parameters	4201

A.10.4.1.8.3	Test Requirements	4201

A.10.4.1.9	RSSI measurement reporting on PSCC	4201

A.10.4.1.9.1	Test purpose and environment	4201

A.10.4.1.9.2	Test parameters	4201

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	4202

A.10.4.1.10.1	Test purpose and environment	4202

A.10.4.1.10.2	Test parameters	4202

A.10.4.1.11	RSSI measurement reporting on SCC	4202

A.10.4.1.11.1	Test purpose and environment	4202

A.10.4.1.11.2	Test parameters	4202

A.10.4.1.12	Channel occupancy measurement reporting on SCC	4203

A.10.4.1.12.1	Test purpose and environment	4203

A.10.4.1.12.2	Test parameters	4203

A.10.4.2	Inter-frequency measurements	4203

A.10.4.2.1	RSSI measurement reporting	4203

A.10.4.2.1.1	Test purpose and environment	4203

A.10.4.2.1.2	Test parameters	4203

A.10.4.2.2	Channel occupancy measurement reporting	4204

A.10.4.2.2.1	Test purpose and environment	4204

A.10.4.2.2.2	Test parameters	4204

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4204

A.10.4.2.3.1	Test Purpose and Environment	4204

A.10.4.2.3.2	Test Requirements	4208

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4209

A.10.4.2.4.1	Test Purpose and Environment	4209

A.10.4.2.4.2	Test Requirements	4213

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4214

A.10.4.2.5.1	Test Purpose and Environment	4214

A.10.4.2.5.2	Test Requirements	4218

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4219

A.10.4.2.6.1	Test Purpose and Environment	4219

A.10.4.2.6.2	Test Requirements	4223

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4224

A.10.4.2.7.1	Test Purpose and Environment	4224

A.10.4.2.7.2	Test Requirements	4230

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4230

A.10.4.2.8.1	Test Purpose and Environment	4230

A.10.4.2.8.2	Test Requirements	4236

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4237

A.10.4.2.9.1	Test Purpose and Environment	4237

A.10.4.2.9.2	Test Requirements	4242

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4242

A.10.4.2.10.1	Test Purpose and Environment	4242

A.10.4.2.10.2	Test Requirements	4248

A.10.4.3	L1-RSRP measurements for beam reporting	4249

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4249

A.10.4.3.1.1	Test Purpose and Environment	4249

A.10.4.3.1.2	Test parameters	4249

A.10.4.3.1.3	Test Requirements	4251

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4252

A.10.4.3.2.1	Test Purpose and Environment	4252

A.10.4.3.2.2	Test parameters	4252

A.10.4.3.2.3	Test Requirements	4254

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4255

A.10.4.3.3.1	Test Purpose and Environment	4255

A.10.4.3.3.2	Test parameters	4255

A.10.4.3.3.3	Test Requirements	4258

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4259

A.10.4.3.4.1	Test Purpose and Environment	4259

A.10.4.3.4.2	Test parameters	4259

A.10.4.3.4.3	Test Requirements	4262

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4263

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4263

A.10.4.4.1.1	Test Purpose and Environment	4263

A.10.4.4.1.2	Test Requirements	4267

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4268

A.10.4.4.2.1	Test Purpose and Environment	4268

A.10.4.4.2.2	Test Requirements	4273

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4274

A.10.4.4.3.1	Test Purpose and Environment	4274

A.10.4.4.3.2	Test Requirements	4278

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4279

A.10.4.4.4.1	Test Purpose and Environment	4279

A.10.4.4.4.2	Test Requirements	4284

A.10.5	Measurement performance	4285

A.10.5.1	SS-RSRP	4285

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4285

A.10.5.1.1.1	Test Purpose and Environment	4285

A.10.5.1.1.2	Test parameters	4285

A.10.5.1.1.3	Test Requirements	4288

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4288

A.10.5.1.2.1	Test Purpose and Environment	4288

A.10.5.1.2.2	Test parameters	4288

A.10.5.1.2.3	Test Requirements	4291

A.10.5.2	SS-RSRQ	4291

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4291

A.10.5.2.1.1	Test Purpose and Environment	4291

A.10.5.2.1.2	Test Parameters	4291

A.10.5.2.1.3	Test Requirements	4294

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4294

A.10.5.2.2.1	Test Purpose and Environment	4294

A.10.5.2.2.2	Test Parameters	4294

A.10.5.2.2.3	Test Requirements	4297

A.10.5.3	SS-SINR	4297

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4297

A.10.5.3.1.1	Test Purpose and Environment	4297

A.10.5.3.1.2	Test Parameters	4297

A.10.5.3.1.3	Test Requirements	4300

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4300

A.10.5.3.2.1	Test Purpose and Environment	4300

A.10.5.3.2.2	Test Parameters	4300

A.10.5.3.2.3	Test Requirements	4303

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4303

A.10.5.3.3.1	Test Purpose and Environment	4303

A.10.5.3.3.2	Test Parameters	4303

A.10.5.3.3.3	Test Requirements	4306

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4306

A.10.5.4.1	SSB based L1-RSRP measurement	4306

A.10.5.4.1.1	Test Purpose and Environment	4306

A.10.5.4.1.2	Test parameters	4307

A.10.5.4.1.3	Test Requirements	4309

A.10.5.5	RSSI	4309

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4309

A.10.5.5.1.1	Test Purpose and Environment	4309

A.10.5.5.1.2	Test parameters	4309

A.10.5.5.1.3	Test Requirements	4312

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4312

A.10.5.5.2.1	Test Purpose and Environment	4312

A.10.5.5.2.2	Test parameters	4312

A.10.5.5.2.3	Test Requirements	4315

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4315

A.10.5.5.3.1	Test Purpose and Environment	4315

A.10.5.5.3.2	Test parameters	4315

A.10.5.5.3.3	Test Requirements	4318

A.10.5.6	Channel occupancy	4318

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4318

A.10.5.6.1.1	Test Purpose and Environment	4318

A.10.5.6.1.2	Test parameters	4318

A.10.5.6.1.3	Test Requirements	4322

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4322

A.10.5.6.2.1	Test Purpose and Environment	4322

A.10.5.6.2.2	Test parameters	4322

A.10.5.6.2.3	Test Requirements	4325

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4325

A.10.5.6.3.1	Test Purpose and Environment	4325

A.10.5.6.3.2	Test parameters	4325

A.10.5.6.3.3	Test Requirements	4329

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4332

A.11.1	RRC_IDLE state mobility	4332

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4332

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4332

A.11.1.1.1.1	Test Purpose and Environment	4332

A.11.1.1.1.2	Test Parameters	4332

A.11.1.1.1.3	Test Requirements	4335

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4335

A.11.1.1.2.1	Test Purpose and Environment	4335

A.11.1.1.2.2	Test Parameters	4335

A.11.1.1.2.3	Test Requirements	4339

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4339

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4339

A.11.1.2.1.1	Test Purpose and Environment	4339

A.11.1.2.1.2	Test Parameters	4340

A.11.1.2.1.3	Test Requirements	4346

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4347

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4347

A.11.1.3.1.1	Test Purpose and Environment	4347

A.11.1.3.1.2	Test Parameters	4347

A.11.1.3.1.3	Test Requirements	4353

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4354

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4354

A.11.1.4.1.1	Test Purpose and Environment	4354

A.11.1.4.1.2	Test Parameters	4354

A.11.1.4.1.3	Test Requirements	4357

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4358

A.11.1.4.2.1	Test Purpose and Environment	4358

A.11.1.4.2.2	Test Requirements	4361

A.11.2	RRC_CONNECTED state mobility	4362

A.11.2.1	Handover	4362

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4362

A.11.2.1.1.1	Test Purpose and Environment	4362

A.11.2.1.1.2	Test Parameters	4362

A.11.2.1.1.3 Test Requirements	4365

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4365

A.11.2.1.2.1	Test Purpose and Environment	4365

A.11.2.1.2.2	Test Parameters	4365

A.11.2.1.2.3	Test Requirements	4368

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4368

A.11.2.1.3.1	Test Purpose and Environment	4368

A.11.2.1.3.2	Test Parameters	4368

A.11.2.1.3.3 Test Requirements	4371

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4371

A.11.2.1.4.1	Test Purpose and Environment	4371

A.11.2.1.4.2	Test Parameters	4371

A.11.2.1.4.3 Test Requirements	4375

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4376

A.11.2.1.5.1	Test Purpose and Environment	4376

A.11.2.1.5.2	Test Parameters	4376

A.11.2.1.5.3 Test Requirements	4379

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4380

A.11.2.1.6.1	Test Purpose and Environment	4380

A.11.2.1.6.2	Test Parameters	4380

A.11.2.1.6.3 Test Requirements	4383

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4384

A.11.2.1.7.1	Test Purpose and Environment	4384

A.11.2.1.7.2	Test Requirements	4390

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4390

A.11.2.1.8.1	Test Purpose and Environment	4390

A.11.2.1.8.2	Test Requirements	4395

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4395

A.11.2.1.9.1	Test Purpose and Environment	4395

A.11.2.1.9.2	Test Requirements	4404

A.11.2.2	RRC connection mobility control	4405

A.11.2.2.1	RRC re-establishment	4405

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4405

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4408

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4415

A.11.2.2.2	Random Access	4420

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4420

A.11.2.2.2.1.1	Test Purpose and Environment	4420

A.11.2.2.2.1.2	Test Requirements	4423

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4423

A.11.2.2.2.1.2.2	Random Access Response Reception	4423

A.11.2.2.2.1.2.3	No Random Access Response Reception	4424

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4424

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4424

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4425

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4425

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4425

A.11.2.2.2.2.1	Test Purpose and Environment	4425

A.11.2.2.2.2.2	Test Requirements	4428

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4428

A.11.2.2.2.2.2.2	Random Access Response Reception	4429

A.11.2.2.2.2.2.3	No Random Access Response Reception	4429

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4429

A.11.2.2.2.3.1	Test Purpose and Environment	4429

A.11.2.2.2.3.2	Test Requirements	4433

A.11.2.2.2.3.2.1	MsgA Transmission	4433

A.11.2.2.2.3.2.2	MsgB Reception	4433

A.11.2.2.2.3.2.3	No MsgB Reception	4434

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4434

A.11.2.2.2.4.1	Test Purpose and Environment	4434

A.11.2.2.2.4.2	Test Requirements	4438

A.11.2.2.2.4.2.1	MsgA Transmission	4438

A.11.2.2.2.4.2.2	MsgB Reception	4439

A.11.2.2.2.4.2.3	No MsgB Reception	4439

A.11.2.2.3	RRC connection release with redirection	4440

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4440

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4444

A.11.3	Timing	4449

A.11.3.1	UE transmit timing	4449

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4449

A.11.3.1.1.1	Test Purpose and environment	4449

A.11.3.1.1.2	Test requirements	4452

A.11.3.2	UE timing advance	4453

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4453

A.11.3.2.1.1	Test Purpose and Environment	4453

A.11.3.2.1.2	Test Parameters	4453

A.11.3.2.1.3	Test Requirements	4457

A.11.4	Signalling characteristics	4457

A.11.4.1	Radio link monitoring	4457

A.11.4.1.1	Introduction	4457

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4458

A.11.4.1.2.1	Test purpose and environment	4458

A.11.4.1.2.2	Test requirements	4462

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4462

A.11.4.1.3.1	Test purpose and environment	4462

A.11.4.1.3.2	Test requirements	4468

A.11.4.1.4	Void	4468

A.11.4.1.4.1	Void	4468

A.11.4.1.4.2	Void	4468

A.11.4.1.5	Void	4468

A.11.4.1.5.1	Void	4468

A.11.4.1.5.2	Void	4468

A.11.4.2	Interruption	4469

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4469

A.11.4.2.1.1 Test Purpose and Environment	4469

A.11.4.2.1.2	Test Requirements	4473

A.11.4.3	SCell activation and deactivation delay	4473

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4473

A.11.4.3.1.1	Test Purpose and Environment	4473

A.11.4.3.1.2	Test Requirements	4478

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4478

A.11.4.3.2.1	Test Purpose and Environment	4478

A.11.4.3.2.2	Test Requirements	4479

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4479

A.11.4.3.3.1	Test Purpose and Environment	4479

A.11.4.3.3.2	Test Requirements	4480

A.11.4.4	Beam failure detection and link recovery procedures	4480

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4480

A.11.4.4.1.1	Test Purpose and Environment	4480

A.11.4.4.1.2	Test Requirements	4485

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4485

A.11.4.4.2.1	Test Purpose and Environment	4485

A.11.4.4.2.2	Test Requirements	4491

A.11.4.5	Active BWP switching	4491

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4491

A.11.4.5.1.1	Test Purpose and Environment	4491

A.11.4.5.1.2	Test Requirements	4495

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4496

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4496

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4499

A.11.4.5.3	RRC-based Active BWP Switch	4502

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4502

A.11.4.6	Void	4505

A.11.5	Measurement procedure	4505

A.11.5.1	Intra-frequency measurements	4505

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4505

A.11.5.1.1.1	Test purpose and environment	4505

A.11.5.1.1.2	Test parameters	4505

A.11.5.1.1.3	Test Requirements	4508

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4508

A.11.5.1.2.1	Test purpose and environment	4508

A.11.5.1.2.2	Test parameters	4508

A.11.5.1.2.3	Test Requirements	4511

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4511

A.11.5.1.3.1	Test purpose and environment	4511

A.11.5.1.3.2	Test parameters	4511

A.11.5.1.3.3	Test Requirements	4511

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4512

A.11.5.1.4.1	Test purpose and environment	4512

A.11.5.1.4.2	Test parameters	4512

A.11.5.1.4.3	Test Requirements	4512

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4512

A.11.5.1.5.1	Test purpose and environment	4512

A.11.5.1.5.2	Test parameters	4512

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4513

A.11.5.1.6.1	Test purpose and environment	4513

A.11.5.1.6.2	Test parameters	4513

A.11.5.1.6.3	Test Requirements	4513

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4513

A.11.5.1.7.1	Test purpose and environment	4513

A.11.5.1.7.2	Test parameters	4513

A.11.5.1.7.3	Test Requirements	4513

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4514

A.11.5.1.8.1	Test purpose and environment	4514

A.11.5.1.8.2	Test parameters	4514

A.11.5.1.8.3	Test Requirements	4514

A.11.5.1.9	RSSI measurement reporting on PCC	4514

A.11.5.1.9.1	Test purpose and environment	4514

A.11.5.1.9.2	Test parameters	4514

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4515

A.11.5.1.10.1	Test purpose and environment	4515

A.11.5.1.10.2	Test parameters	4515

A.11.5.1.11	RSSI measurement reporting on SCC	4515

A.11.5.1.11.1	Test purpose and environment	4515

A.11.5.1.11.2	Test parameters	4515

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4516

A.11.5.1.12.1	Test purpose and environment	4516

A.11.5.1.12.2	Test parameters	4516

A.11.5.2	Inter-frequency measurements	4516

A.11.5.2.1	RSSI measurement reporting	4516

A.11.5.2.1.1	Test purpose and environment	4516

A.11.5.2.1.2	Test parameters	4516

A.11.5.2.2	Channel occupancy measurement reporting	4517

A.11.5.2.2.1	Test purpose and environment	4517

A.11.5.2.2.2	Test parameters	4517

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4517

A.11.5.2.3.1	Test Purpose and Environment	4517

A.11.5.2.3.2	Test Requirements	4521

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4522

A.11.5.2.4.1	Test Purpose and Environment	4522

A.11.5.2.4.2	Test Requirements	4527

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4527

A.11.5.2.5.1	Test Purpose and Environment	4527

A.11.5.2.5.2	Test Requirements	4531

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4532

A.11.5.2.6.1	Test Purpose and Environment	4532

A.11.5.2.6.2	Test Requirements	4537

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4537

A.11.5.2.7.1	Test Purpose and Environment	4537

A.11.5.2.7.2	Test Requirements	4541

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4542

A.11.5.2.8.1	Test Purpose and Environment	4542

A.11.5.2.8.2	Test Requirements	4547

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4547

A.11.5.2.9.1	Test Purpose and Environment	4547

A.11.5.2.9.2	Test Requirements	4552

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4553

A.11.5.2.10.1	Test Purpose and Environment	4553

A.11.5.2.10.2	Test Requirements	4558

A.11.5.3	Inter-RAT E-UTRAN measurements	4559

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4559

A.11.5.3.1.1	Test Purpose and Environment	4559

A.11.5.3.1.2	Test Requirements	4563

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4563

A.11.5.3.2.1	Test Purpose and Environment	4563

A.11.5.3.2.2	Test Requirements	4568

A.11.5.4	L1-RSRP measurements for beam reporting	4568

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4568

A.11.5.4.1.1	Test Purpose and Environment	4568

A.11.5.4.1.2	Test parameters	4568

A.11.5.4.1.3	Test Requirements	4570

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4570

A.11.5.4.2.1	Test Purpose and Environment	4570

A.11.5.4.2.2	Test parameters	4571

A.11.5.4.2.3	Test Requirements	4573

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4573

A.11.5.4.3.1	Test Purpose and Environment	4573

A.11.5.4.3.2	Test parameters	4574

A.11.5.4.3.3	Test Requirements	4577

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4578

A.11.5.4.4.1	Test Purpose and Environment	4578

A.11.5.4.4.2	Test parameters	4578

A.11.5.4.4.3	Test Requirements	4581

A.11.6	Measurement performance	4582

A.11.6.1	SS-RSRP	4582

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4582

A.11.6.1.1.1	Test Purpose and Environment	4582

A.11.6.1.1.2	Test parameters	4582

A.11.6.1.1.3	Test Requirements	4584

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4584

A.11.6.1.2.1	Test Purpose and Environment	4584

A.11.6.1.2.2	Test parameters	4584

A.11.6.1.2.3	Test Requirements	4586

A.11.6.2	SS-RSRQ	4586

A.11.6.2.1	Intra-frequency measurement accuracy	4586

A.11.6.2.1.1	Test Purpose and Environment	4586

A.11.6.2.1.2	Test Parameters	4586

A.11.6.2.1.3	Test Requirements	4589

A.11.6.2.2	Inter-frequency measurement accuracy	4589

A.11.6.2.2.1	Test Purpose and Environment	4589

A.11.6.2.2.2	Test Parameters	4589

A.11.6.2.2.3	Test Requirements	4592

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4592

A.11.6.2.3.1	Test Purpose and Environment	4592

A.11.6.2.3.2	Test Parameters	4592

A.11.6.2.3.3	Test Requirements	4595

A.11.6.2.4	Inter-frequency measurement accuracy	4595

A.11.6.2.4.1	Test Purpose and Environment	4595

A.11.6.2.4.2	Test Parameters	4595

A.11.6.2.4.3	Test Requirements	4604

A.11.6.3	SS-SINR	4604

A.11.6.3.1	Intra-frequency measurement accuracy	4604

A.11.6.3.1.1	Test Purpose and Environment	4604

A.11.6.3.1.2	Test Parameters	4604

A.11.6.3.1.3	Test Requirements	4607

A.11.6.3.2	Inter-frequency measurement accuracy	4607

A.11.6.3.2.1	Test Purpose and Environment	4607

A.11.6.3.2.2	Test Parameters	4607

A.11.6.3.2.3	Test Requirements	4610

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4610

A.11.6.3.3.1	Test Purpose and Environment	4610

A.11.6.3.3.2	Test Parameters	4610

A.11.6.3.3.3	Test Requirements	4613

A.11.6.3.4	Inter-frequency measurement accuracy	4613

A.11.6.3.4.1	Test Purpose and Environment	4613

A.11.6.3.4.2	Test Parameters	4613

A.11.6.3.4.3	Test Requirements	4621

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4621

A.11.6.4.1	SSB based L1-RSRP measurement	4621

A.11.6.4.1.1	Test Purpose and Environment	4621

A.11.6.4.1.2	Test parameters	4622

A.11.6.4.1.3	Test Requirements	4625

A.11.6.5	RSSI	4625

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4625

A.11.6.5.1.1	Test Purpose and Environment	4625

A.11.6.5.1.2	Test parameters	4625

A.11.6.5.1.3	Test Requirements	4628

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4628

A.11.6.5.2.1	Test Purpose and Environment	4628

A.11.6.5.2.2	Test parameters	4628

A.11.6.5.2.3	Test Requirements	4631

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4631

A.11.6.5.3.1	Test Purpose and Environment	4631

A.11.6.5.3.2	Test parameters	4631

A.11.6.5.3.3	Test Requirements	4634

A.11.6.6	Channel occupancy	4634

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4634

A.11.6.6.1.1	Test Purpose and Environment	4634

A.11.6.6.1.2	Test parameters	4634

A.11.6.6.1.3	Test Requirements	4637

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4637

A.11.6.6.2.1	Test Purpose and Environment	4637

A.11.6.6.2.2	Test parameters	4637

A.11.6.6.2.3	Test Requirements	4640

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4640

A.11.6.6.3.1	Test Purpose and Environment	4640

A.11.6.6.3.2	Test parameters	4640

A.11.6.6.3.3	Test Requirements	4643

A.11.6.7	E-UTRAN RSRP	4643

A.11.6.8	E-UTRAN RSRQ	4643

A.11.6.9	E-UTRAN SINR	4643

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4643

A.12.1	RRC_IDLE state mobility	4643

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4643

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4643

A.12.1.1.1.1	Test Purpose and Environment	4643

A.12.1.1.1.2	Test Requirements	4648

A.12.2	RRC_CONNECTED state mobility	4649

A.12.2.1	Handover	4649

A.12.2.1.1	E-UTRAN - NR with CCA handover	4649

A.12.2.1.1.1	Test Purpose and Environment	4649

A.12.2.1.1.2	Test Requirements	4654

A.12.3	Signalling characteristics	4655

A.12.3.1	Interruptions	4655

A.12.4	Measurement procedure	4655

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4655

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4655

A.12.4.1.1.1	Test Purpose and Environment	4655

A.12.4.1.1.2	Test Requirements	4659

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4659

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4659

A.12.4.2.1.1	Test Purpose and Environment	4659

A.12.4.2.1.2	Test Requirements	4663

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4664

A.12.4.2.2.1	Test Purpose and Environment	4664

A.12.4.2.2.2	Test Requirements	4668

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4669

A.12.4.2.3.1	Test Purpose and Environment	4669

A.12.4.2.3.2	Test Requirements	4673

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4674

A.12.4.2.4.1	Test Purpose and Environment	4674

A.12.4.2.4.2	Test Requirements	4678

A.12.4.2.5	RSSI measurement reporting	4679

A.12.4.2.5.1	Test purpose and environment	4679

A.12.4.2.5.2	Test parameters	4679

A.12.4.2.5.3	Test Requirements	4682

A.12.4.2.6	Channel occupancy measurement reporting	4682

A.12.4.2.6.1	Test purpose and environment	4682

A.12.4.2.6.2	Test parameters	4682

A.12.5	Measurement performance	4685

A.12.5.1	E-UTRANNR SFTD	4685

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4685

A.12.5.1.1.1	Test Purpose	4685

A.12.5.1.1.2	Test Environment	4685

A.12.5.1.1.3	Test Requirements	4690

A.12.5.2	E-UTRANNR SS-RSRP	4690

A.12.5.3	E-UTRANNR SS-RSRQ	4690

A.12.5.4	E-UTRANNR SS-SINR	4690

A.12.5.5	E-UTRANNR RSSI	4690

A.12.5.6	E-UTRANNR channel occupancy	4690

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4691

A.13.1	Timing	4691

A.13.1.1	UE transmit timing	4691

A.13.1.2	Timing advance	4691

A.13.2	Signalling characteristics	4691

A.13.2.1	Interruption	4691

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4691

A.13.2.1.1.1	Test Purpose and Environment	4691

A.13.2.1.1.2	Test Requirements	4695

A.13.2.2	SCell activation and deactivation delay	4695

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4695

A.13.2.2.1.1	Test Purpose and Environment	4695

A.13.2.2.1.2	Test Requirements	4700

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4700

A.13.2.2.2.1	Test Purpose and Environment	4700

A.13.2.2.2.2	Test Requirements	4701

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4701

A.13.2.2.3.1	Test Purpose and Environment	4701

A.13.2.2.3.2	Test Requirements	4702

A.13.2.3	Void	4702

A.13.3	Measurement procedure	4702

A.13.3.1	Intra-frequency measurements	4702

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4702

A.13.3.1.1.1	Test purpose and environment	4702

A.13.3.1.1.2	Test parameters	4702

A.13.3.1.1.3	Test Requirements	4707

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4707

A.13.3.1.2.1	Test purpose and environment	4707

A.13.3.1.2.2	Test parameters	4707

A.13.3.1.2.3	Test Requirements	4712

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4712

A.13.3.1.3.1	Test purpose and environment	4712

A.13.3.1.3.2	Test parameters	4712

A.13.3.1.3.3	Test Requirements	4717

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4717

A.13.3.1.4.1	Test purpose and environment	4717

A.13.3.1.4.2	Test parameters	4717

A.13.3.1.4.3	Test Requirements	4722

A.13.3.1.5	RSSI measurement reporting on SCC	4722

A.13.3.1.5.1	Test purpose and environment	4722

A.13.3.1.5.2	Test parameters	4722

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4723

A.13.3.1.6.1	Test purpose and environment	4723

A.13.3.1.6.2	Test parameters	4724

A.13.3.2	Inter-frequency measurements	4724

A.13.3.2.1	RSSI measurement reporting	4724

A.13.3.2.1.1	Test purpose and environment	4724

A.13.3.2.1.2	Test parameters	4724

A.13.3.2.2	Channel occupancy measurement reporting	4725

A.13.3.2.2.1	Test purpose and environment	4725

A.13.3.2.2.2	Test parameters	4725

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4725

A.13.3.2.3.1	Test Purpose and Environment	4725

A.13.3.2.3.2	Test Requirements	4730

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4730

A.13.3.2.4.1	Test Purpose and Environment	4730

A.13.3.2.4.2	Test Requirements	4735

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4736

A.13.3.2.5.1	Test Purpose and Environment	4736

A.13.3.2.5.2	Test Requirements	4741

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4741

A.13.3.2.6.1	Test Purpose and Environment	4741

A.13.3.2.6.2	Test Requirements	4747

A.13.3.3	L1-RSRP measurements for beam reporting	4748

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4748

A.13.3.3.1.1	Test Purpose and Environment	4748

A.13.3.3.1.2	Test parameters	4748

A.13.3.3.1.3	Test Requirements	4752

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4752

A.13.3.3.2.1	Test Purpose and Environment	4752

A.13.3.3.2.2	Test parameters	4752

A.13.3.3.2.3	Test Requirements	4756

A.13.4	Measurement performance	4756

A.13.4.1	SS-RSRP	4756

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4756

A.13.4.1.1.1	Test Purpose and Environment	4756

A.13.4.1.1.2	Test parameters	4756

A.13.4.1.1.3	Test Requirements	4758

A.13.4.2	SS-RSRQ	4758

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4758

A.13.4.2.1.1	Test Purpose and Environment	4758

A.13.4.2.1.2	Test Parameters	4758

A.13.4.2.1.3	Test Requirements	4765

A.13.4.3	SS-SINR	4766

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4766

A.13.4.3.1.1	Test Purpose and Environment	4766

A.13.4.3.1.2	Test Parameters	4766

A.13.4.3.1.3	Test Requirements	4773

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4773

A.13.4.4.1	SSB based L1-RSRP measurement	4773

A.13.4.4.1.1	Test Purpose and Environment	4773

A.13.4.4.1.2	Test parameters	4773

A.13.4.4.1.3	Test Requirements	4777

A.13.4.5	RSSI	4777

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4777

A.13.4.5.1.1	Test Purpose and Environment	4777

A.13.4.5.1.2	Test parameters	4777

A.13.4.5.1.3	Test Requirements	4780

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4780

A.13.4.5.2.1	Test Purpose and Environment	4780

A.13.4.5.2.2	Test parameters	4780

A.13.4.5.2.3	Test Requirements	4784

A.13.4.6	Channel occupancy	4784

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4784

A.13.4.6.1.1	Test Purpose and Environment	4784

A.13.4.6.1.2	Test parameters	4784

A.13.4.6.1.3	Test Requirements	4788

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4788

A.13.4.6.2.1	Test Purpose and Environment	4788

A.13.4.6.2.2	Test parameters	4788

A.13.4.6.2.3	Test Requirements	4792

A.14	NR standalone tests for Satellite access	4792

A.14.1	RRC_IDLE state mobility	4792

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4792

A.14.1.1.1	Test Purpose and Environment	4792

A.14.1.1.2	Test Parameters	4792

A.14.1.1.3	Test Requirements	4794

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4795

A.14.1.2.1	Test Purpose and Environment	4795

A.14.1.2.2	Test Parameters	4795

A.14.1.2.3	Test Requirements	4799

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4800

A.14.1.3.1	Test Purpose and Environment	4800

A.14.1.3.2	Test Parameters	4800

A.14.1.3.3	Test Requirements	4801

A.14.1.4	Location-based cell measurement initiation to FR1 intra-frequency NR cell reselection	4802

A.14.1.4.1	Test Purpose and Environment	4802

A.14.1.4.2	Test Parameters	4802

A.14.1.4.3	Test Requirements	4804

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4805

A.14.1.5.1	Test Purpose and Environment	4805

A.14.1.5.2	Test Parameters	4805

A.14.1.5.3	Test Requirements	4807

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4808

A.14.1.6.1	Test Purpose and Environment	4808

A.14.1.6.2	Test Parameters	4808

A.14.1.6.3	Test Requirements	4810

A.14.1.7	Time-based Cell measurement initiation to FR1 inter-frequency cell reselection	4811

A.14.1.7.1	Test Purpose and Environment	4811

A.14.1.7.2	Test Parameters	4811

A.14.1.7.3	Test Requirements	4812

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4813

A.14.1.8.1	Test Purpose and Environment	4813

A.14.1.8.2	Test Parameters	4813

A.14.1.8.3	Test Requirements	4815

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4816

A.14.1.9.1	Test Purpose and Environment	4816

A.14.1.9.2	Test Parameters	4816

A.14.1.9.3	Test Requirements	4819

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4819

A.14.1.10.1	Test Purpose and Environment	4819

A.14.1.10.2	Test Parameters	4819

A.14.1.10.3	Test Requirements	4821

A.14.2	RRC_CONNECTED state mobility	4822

A.14.2.1	Handover	4822

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4822

A.14.2.1.1.1	Test Purpose and Environment	4822

A.14.2.1.1.2	Test Parameters	4822

A.14.2.1.1.3	Test Requirements	4824

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4825

A.14.2.1.2.1	Test Purpose and Environment	4825

A.14.2.1.2.2	Test Parameters	4825

A.14.2.1.2.3	Test Requirements	4827

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4827

A.14.2.1.3.1	Test Purpose and Environment	4827

A.14.2.1.3.2	Test Parameters	4827

A.14.2.1.3.3	Test Requirements	4830

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4830

A.14.2.1.4.1	Test Purpose and Environment	4830

A.14.2.1.4.2	Test Parameters	4830

A.14.2.1.4.3	Test Requirements	4832

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4832

A.14.2.1.5.1	Test Purpose and Environment	4832

A.14.2.1.5.2	Test Parameters	4833

A.14.2.1.5.3	Test Requirements	4836

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4836

A.14.2.1.6.1	Test Purpose and Environment	4836

A.14.2.1.6.2	Test Parameters	4836

A.14.2.1.6.3	Test Requirements	4840

A.14.2.2	RRC Connection Mobility Control	4840

A.14.2.2.1	SA: RRC Re-establishment for SAN	4840

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4840

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4843

A.14.2.2.2	Random Access	4846

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4846

A.14.2.2.2.1.1	Test Purpose and Environment	4846

A.14.2.2.2.1.2	Test Requirements	4849

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4850

A.14.2.2.2.2.1	Test Purpose and Environment	4850

A.14.2.2.2.2.2	Test Requirements	4854

A.14.2.2.3	RRC Connection Release with Redirection	4855

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4855

A.14.2.2.3.1.1	Test Purpose and Environment	4855

A.14.2.2.3.1.2	Test Parameters	4855

A.14.2.2.3.1.3	Test Requirements	4858

A.14.3	Timing for Satellite Access	4858

A.14.3.1	UE transmit timing for Satellite Access	4858

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4858

A.14.3.1.1.1	Test Purpose and environment	4858

A.14.3.1.1.2	Test requirements	4860

A.14.3.2	Timing advance for satellite access	4861

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4861

A.14.3.2.1.1	Test Purpose and Environment	4861

A.14.3.2.1.2	Test Parameters	4861

A.14.3.2.1.3	Test Requirements	4864

A.14.4	Signalling characteristics	4864

A.14.4.1	Radio link Monitoring	4864

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4864

A.14.4.1.1.1	Test Purpose and Environment	4864

A.14.4.1.1.2	Test Requirements	4869

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4869

A.14.4.1.2.1	Test Purpose and Environment	4869

A.14.4.1.2.2	Test Requirements	4875

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4875

A.14.4.1.3.1	Test Purpose and Environment	4875

A.14.4.1.3.2	Test Requirements	4879

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4879

A.14.4.1.4.1	Test Purpose and Environment	4879

A.14.4.1.4.2	Test Requirements	4884

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4884

A.14.4.1.5.1	Test Purpose and Environment	4884

A.14.4.1.5.2	Test Requirements	4889

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4889

A.14.4.1.6.1	Test Purpose and Environment	4889

A.14.4.1.6.2	Test Requirements	4894

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4894

A.14.4.1.7.1	Test Purpose and Environment	4894

A.14.4.1.7.2	Test Requirements	4899

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4899

A.14.4.1.8.1	Test Purpose and Environment	4899

A.14.4.1.8.2	Test Requirements	4904

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4904

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4904

A.14.4.2.1.1	Test Purpose and Environment	4904

A.14.4.2.1.2	Test Requirements	4910

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	4910

A.14.4.2.2.1	Test Purpose and Environment	4910

A.14.4.2.2.2	Test Requirements	4915

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	4915

A.14.4.2.3.1	Test Purpose and Environment	4915

A.14.4.2.3.2	Test Requirements	4921

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	4921

A.14.4.2.4.1	Test Purpose and Environment	4921

A.14.4.2.4.2	Test Requirements	4926

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	4926

A.14.4.2.5.1	Test Purpose and Environment	4926

A.14.4.2.5.2	Test Requirements	4931

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	4931

A.14.4.2.6.1	Test Purpose and Environment	4931

A.14.4.2.6.2	Test Requirements	4936

A.14.4.3	Active BWP switch for satellite access	4936

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	4936

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	4936

A.14.4.3.1.1.1	Test Purpose and Environment	4936

A.14.4.3.1.1.2	Test Requirements	4941

A.14.4.3.2	RRC-based Active BWP Switch	4941

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4941

A.14.4.3.2.1.1	Test Purpose and Environment	4941

A.14.4.3.2.1.2	Test Requirements	4945

A.14.4.4	UE specific CBW change for satellite access	4945

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	4945

A.14.4.4.1.1	Test Purpose and Environment	4945

A.14.4.4.1.2	Test Requirements	4949

A.14.4.5	Pathloss reference signal switching delay	4949

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	4949

A.14.4.5.1.1	Test Purpose and Environment	4949

A.14.4.5.1.2	Test Requirements	4951

A.14.5	Measurement procedure	4952

A.14.5.1	Intra-frequency Measurements	4952

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	4952

A.14.5.1.1.1	Test purpose and Environment	4952

A.14.5.1.1.2	Test parameters	4952

A.14.5.1.1.3	Test Requirements	4954

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	4955

A.14.5.1.2.1	Test purpose and Environment	4955

A.14.5.1.2.2	Test parameters	4955

A.14.5.1.2.3	Test Requirements	4957

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	4958

A.14.5.1.3.1	Test purpose and Environment	4958

A.14.5.1.3.2	Test parameters	4958

A.14.5.1.3.3	Test Requirements	4960

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	4961

A.14.5.1.4.1	Test purpose and Environment	4961

A.14.5.1.4.2	Test parameters	4961

A.14.5.1.4.3	Test Requirements	4963

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	4964

A.14.5.1.5.1	Test purpose and Environment	4964

A.14.5.1.5.2	Test parameters	4964

A.15.5.1.5.3	Test Requirements	4966

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	4967

A.14.5.1.6.1	Test purpose and Environment	4967

A.14.5.1.6.2	Test parameters	4967

A.14.5.1.6.3	Test Requirements	4969

A.14.5.2	Inter-frequency Measurements	4970

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	4970

A.14.5.2.1.1	Test Purpose and Environment	4970

A.14.5.2.1.2	Test Requirements	4974

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	4974

A.14.5.2.2.1	Test Purpose and Environment	4974

A.14.5.2.2.2	Test Requirements	4978

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	4978

A.14.5.2.3.1	Test Purpose and Environment	4978

A.14.5.2.3.2	Test Requirements	4982

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	4982

A.14.5.2.4.1	Test Purpose and Environment	4982

A.14.5.2.4.2	Test Requirements	4986

A.14.5.2.5	void	4986

A.14.5.2.5.1	void	4986

A.14.5.2.5.2	void	4986

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	4986

A.14.5.2.6.1	Test Purpose and Environment	4986

A.14.5.2.6.2	Test Requirements	4990

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	4990

A.14.5.2.7.1	Test purpose and Environment	4990

A.14.5.2.7.2	Test parameters	4990

A.14.5.2.7.3	Test Requirements	4992

A.14.5.2.8	Event triggered reporting tests without gap under DRX	4993

A.14.5.2.8.1	Test purpose and Environment	4993

A.14.5.2.8.2	Test parameters	4993

A.14.5.2.8.3	Test Requirements	4995

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	4996

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	4996

A.14.5.3.1.1	Test Purpose and Environment	4996

A.14.5.3.1.2	Test parameters	4996

A.14.5.3.1.3	Test Requirements	4999

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	4999

A.14.5.3.2.1	Test Purpose and Environment	4999

A.14.5.3.2.2	Test parameters	4999

A.14.5.3.2.3	Test Requirements	5003

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	5003

A.14.5.3.3.1	Test Purpose and Environment	5003

A.14.5.3.3.2	Test parameters	5003

A.14.5.3.3.3	Test Requirements	5007

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	5007

A.14.5.3.4.1	Test Purpose and Environment	5007

A.14.5.3.4.2	Test parameters	5008

A.14.5.3.4.3	Test Requirements	5011

A.14.6	Measurement Performance requirements	5011

A.14.6.1	 SS-RSRP for SAN	5011

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5011

A.14.6.1.1.1	Test Purpose and Environment	5011

A.14.6.1.1.2	Test parameters	5011

A.14.6.1.1.3	Test Requirements	5015

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5015

A.14.6.1.2.1	Test Purpose and Environment	5015

A.14.6.1.2.2	Test parameters	5015

A.14.6.1.2.3	Test Requirements	5018

A.14.6.2	SS-RSRQ	5018

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5018

A.14.6.2.1.1	Test Purpose and Environment	5018

A.14.6.2.1.2	Test Parameters	5018

A.14.6.2.1.3	Test Requirements	5021

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5021

A.14.6.2.2.1	Test Purpose and Environment	5021

A.14.6.2.2.2	Test Parameters	5021

A.14.6.2.2.3	Test Requirements	5024

A.14.6.3	SS-SINR	5024

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5024

A.14.6.3.1.1	Test Purpose and Environment	5024

A.14.6.3.1.2	Test Parameters	5024

A.14.6.3.1.3	Test Requirements	5028

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5028

A.14.6.3.2.1	Test Purpose and Environment	5028

A.14.6.3.2.2	Test Parameters	5028

A.14.6.3.2.3	Test Requirements	5032

A.14.6.4	L1-RSRP measurement for beam reporting	5032

A.14.6.4.1	SSB based L1-RSRP measurement	5032

A.14.6.4.1.1	Test Purpose and Environment	5032

A.14.6.4.1.2	Test parameters	5032

A.14.6.4.1.3	Test Requirements	5035

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5036

A.14.6.4.2.1	Test Purpose and Environment	5036

A.14.6.4.2.2	Test parameters	5036

A.14.6.4.2.3	Test Requirements	5039

A.15	NR standalone tests with one or more NR cells in FR2-2	5040

A.15.1	SA: RRC_IDLE state mobility	5040

A.15.1.1	Cell re-selection to NR	5040

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	5040

A.15.1.1.1.1	Test Purpose and Environment	5040

A.15.1.1.1.2	Test Parameters	5040

A.15.1.1.1.3	Test Requirements	5044

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5044

A.15.1.2.1	Test Purpose and Environment	5044

A.15.1.2.2	Test Parameters	5044

A.15.1.2.3	Test Requirements	5048

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5048

A.15.1.3.1	Test Purpose and Environment	5048

A.15.1.3.2	Test Parameters	5048

A.15.1.3.3	Test Requirements	5051

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5051

A.15.1.4.1	Test Purpose and Environment	5051

A.15.1.4.2	Test Parameters	5051

A.15.1.4.3	Test Requirements	5054

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5054

A.15.1.5.1	Test Purpose and Environment	5054

A.15.1.5.2	Test Parameters	5054

A.15.1.5.3	Test Requirements	5058

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5058

A.15.1.6.1	Test Purpose and Environment	5058

A.15.1.6.2	Test Parameters	5058

A.15.1.6.3	Test Requirements	5061

A.15.2	Signaling characteristics	5062

A.15.2.1	SCell Activation and Deactivation Delay	5062

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5062

A.15.2.1.1.1	Test Purpose and Environment	5062

A.15.2.1.1.2	Test Requirements	5064

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5064

A.15.2.1.2.1	Test Purpose and Environment	5064

A.15.2.1.2.2	Test Requirements	5069

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5070

A.15.2.1.3.1	Test Purpose and Environment	5070

A.15.2.1.3.2	Test Requirements	5073

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5074

A.15.2.1.4.1	Test Purpose and Environment	5074

A.15.2.1.4.2	Test Requirements	5077

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5078

A.15.2.1.5.1	Test Purpose and Environment	5078

A.15.2.1.5.2	Test Requirements	5082

A.15.3	RRC_CONNECTED state mobility	5083

A.15.3.1	Handover	5083

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5083

A.15.3.1.1.1	Test Purpose and Environment	5083

A.15.3.1.1.2	Test Parameters	5083

A.15.3.1.1.3	Test Requirements	5087

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5087

A.15.3.1.2.1	Test Purpose and Environment	5087

A.15.3.1.2.2	Test Parameters	5087

A.15.3.1.2.3	Test Requirements	5091

A.15.4	Measurement procedure	5092

A.15.4.1	Intra-frequency Measurements	5092

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5092

A.15.4.1.1.1	Test purpose and Environment	5092

A.15.4.1.1.2	Test Requirements	5094

A.15.4.2	Inter-frequency Measurements	5095

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5095

A.15.4.2.1.1	Test Purpose and Environment	5095

A.15.4.2.1.2	Test Requirements	5098

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5099

A.16.1	SA: RRC_IDLE state mobility for RedCap	5099

A.16.1.1	Cell re-selection to NR	5099

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5099

A.16.1.1.1.1	Test Purpose and Environment	5099

A.16.1.1.1.2	Test Parameters	5099

A.16.1.1.1.3	Test Requirements	5103

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5103

A.16.1.1.2.1	Test Purpose and Environment	5103

A.16.1.1.2.2	Test Parameters	5103

A.16.1.1.2.3	Test Requirements	5107

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5107

A.16.1.1.3.1	Test Purpose and Environment	5107

A.16.1.1.3.2	Test Parameters	5107

A.16.1.1.3.3	Test Requirements	5112

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5112

A.16.1.1.4.1	Test Purpose and Environment	5112

A.16.1.1.4.2	Test Parameters	5112

A.16.1.1.4.3	Test Requirements	5117

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5117

A.16.1.1.5.1	Test Purpose and Environment	5117

A.16.1.1.5.2	Test Parameters	5117

A.16.1.1.5.3	Test Requirements	5122

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5122

A.16.1.1.6.1	Test Purpose and Environment	5122

A.16.1.1.6.2	Test Parameters	5122

A.16.1.1.6.3	Test Requirements	5127

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5127

A.16.1.1.7.1	Test Purpose and Environment	5127

A.16.1.1.7.2	Test Parameters	5127

A.16.1.1.7.3	Test Requirements	5132

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5132

A.16.1.1.8.1	Test Purpose and Environment	5132

A.16.1.1.8.2	Test Parameters	5133

A.16.1.1.8.3	Test Requirements	5137

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5138

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5138

A.16.1.2.1.1	Test Purpose and Environment	5138

A.16.1.2.1.2	Test Parameters	5138

A.16.1.2.1.3	Test Requirements	5141

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5142

A.16.1.2.2.1	Test Purpose and Environment	5142

A.16.1.2.2.2	Test Parameters	5142

A.16.1.2.2.3	Test Requirements	5145

A.16.1.2.3.1	Test Purpose and Environment	5146

A. 16.1.2.3.2	Test Parameters	5146

A.16.1.2.3.3	Test Requirements	5149

A.16.1.2.4.1	Test Purpose and Environment	5150

A.16.1.2.4.2	Test Parameters	5150

A.16.1.3.1.3	Test Requirements	5153

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5154

A.16.1.2.5.1	Test Purpose and Environment	5154

A.16.1.2.5.2	Test Parameters	5154

A.16.1.2.5.3	Test Requirements	5157

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5158

A.16.1.2.6.1	Test Purpose and Environment	5158

A.16.1.2.6.2	Test Parameters	5158

A.16.1.2.6.3	Test Requirements	5161

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5162

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5162

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5162

A.16.2.1.1.1	Test purpose and Environment	5162

A.16.2.1.1.2	Test Parameters	5163

A.16.2.1.1.3	Test requirements	5166

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5166

A.16.2.1.2.1	Test purpose and Environment	5166

A.16.2.1.2.2	Test Parameters	5167

A.16.2.1.2.3	Test requirements	5170

A.16.3	RRC_CONNECTED state mobility for RedCap	5170

A.16.3.1	Handover	5170

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5170

A.16.3.1.1.1	Test Purpose and Environment	5170

A.16.3.1.1.2	Test Parameters	5170

A.16.3.1.1.3	Test Requirements	5174

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5174

A.16.3.1.2.1	Test Purpose and Environment	5174

A.16.3.1.2.2	Test Parameters	5174

A.16.3.1.2.3	Test Requirements	5178

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5178

A.16.3.1.3.1	Test Purpose and Environment	5178

A.16.3.1.3.2	Test Parameters	5178

A.16.3.1.3.3	Test Requirements	5182

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5182

A.16.3.1.4.1	Test Purpose and Environment	5182

A.16.3.1.4.2	Test Parameters	5182

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5186

A.16.3.1.5.1	Test Purpose and Environment	5186

A.16.3.1.5.2	Test Parameters	5186

A.16.3.1.5.3	Test Requirements	5190

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5190

A.16.3.1.6.1	Test Purpose and Environment	5190

A.16.3.1.6.2	Test Parameters	5190

A.16.3.1.6.3	Test Requirements	5194

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5194

A.16.3.1.7.1	Test Purpose and Environment	5194

A.16.3.1.7.2	Test Requirements	5200

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5200

A.16.3.1.8.1	Test Purpose and Environment	5200

A.16.3.1.8.2	Test Requirements	5206

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5206

A.16.3.1.9.1	Test Purpose and Environment	5206

A.16.3.1.9.2	Test Requirements	5213

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5213

A.16.3.1.10.1	Test Purpose and Environment	5213

A.16.3.1.10.2	Test Requirements	5219

A.16.3.2	RRC Connection Mobility Control	5219

A.16.3.2.1	SA: RRC Re-establishment	5219

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5219

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5223

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5227

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5232

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5237

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5241

A.16.3.2.2	Random Access	5245

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5245

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5251

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5256

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5266

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5270

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5274

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5278

A.16.3.2.3	SA: RRC Connection Release with Redirection	5282

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5282

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5286

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5290

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5297

A.16.4	Timing for RedCap	5304

A.16.4.1	UE transmit timing	5304

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5304

A.16.4.1.1.1	Test Purpose and environment	5304

A.16.4.1.1.2	Test requirements	5306

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5307

A.16.4.1.2.1	Test Purpose and environment	5307

A.16.4.1.2.2	Test requirements	5309

A.16.4.2	UE timer accuracy	5309

A.16.4.3	Timing advance	5309

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5309

A.16.4.3.1.1	Test Purpose and Environment	5309

A.16.4.3.1.2	Test Parameters	5309

A.16.4.3.1.3	Test Requirements	5313

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5313

A.16.4.3.2.1	Test Purpose and Environment	5313

A.16.4.3.2.2	Test Parameters	5314

A.16.4.3.2.3	Test Requirements	5317

A.16.5	Signalling characteristics for RedCap	5318

A.16.5.1	Radio link Monitoring	5318

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5318

A.16.5.1.1.1	Test Purpose and Environment	5318

A.16.5.1.1.2	Test Requirements	5323

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5323

A.16.5.1.2.1	Test Purpose and Environment	5323

A.16.5.1.2.2	Test Requirements	5329

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5329

A.16.5.1.3.1	Test Purpose and Environment	5329

A.16.5.1.3.2	Test Requirements	5335

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5335

A.16.5.1.4.1	Test Purpose and Environment	5335

A.16.5.1.4.2	Test Requirements	5341

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5341

A.16.5.1.5.1	Test Purpose and Environment	5341

A.16.5.1.5.2	Test Requirements	5347

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5347

A.16.5.1.6.1	Test Purpose and Environment	5347

A.16.5.1.6.2	Test Requirements	5352

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5352

A.16.5.1.7.1	Test Purpose and Environment	5352

A.16.5.1.7.2	Test Requirements	5357

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5357

A.16.5.1.8.1	Test Purpose and Environment	5357

A.16.5.1.8.2	Test Requirements	5363

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5363

A.16.5.1.9.1	Test Purpose and Environment	5363

A.16.5.1.9.2	Test Requirements	5369

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5369

A.16.5.1.10.1	Test Purpose and Environment	5369

A.16.5.1.10.2	Test Requirements	5375

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5375

A.16.5.1.11.1	Test Purpose and Environment	5375

A.16.5.1.11.2	Test Requirements	5381

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5381

A.16.5.1.12.1	Test Purpose and Environment	5381

A.16.5.1.12.2	Test Requirements	5386

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5386

A.16.5.1.13.1	Test Purpose and Environment	5386

A.16.5.1.13.2	Test Requirements	5392

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5392

A.16.5.1.14.1	Test Purpose and Environment	5392

A.16.5.1.14.2	Test Requirements	5398

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5398

A.16.5.1.15.1	Test Purpose and Environment	5398

A.16.5.1.15.2	Test Requirements	5404

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5404

A.16.5.1.16.1	Test Purpose and Environment	5404

A.16.5.1.16.2	Test Requirements	5410

A.16.5.2	Beam Failure Detection and Link recovery procedures	5410

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5410

A.16.5.2.1.1	Test Purpose and Environment	5410

A.16.5.2.1.2	Test Requirements	5416

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5416

A.16.5.2.2.1	Test Purpose and Environment	5416

A.16.5.2.2.2	Test Requirements	5422

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5422

A.16.5.2.3.1	Test Purpose and Environment	5422

A.16.5.2.3.2	Test Requirements	5429

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5429

A.16.5.2.4.1	Test Purpose and Environment	5429

A.16.5.2.4.2	Test Requirements	5435

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5435

A.16.5.2.5.1	Test Purpose and Environment	5435

A.16.5.2.5.2	Test Requirements	5441

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5441

A.16.5.2.6.1	Test Purpose and Environment	5441

A.16.5.2.6.2	Test Requirements	5447

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5447

A.16.5.2.7.1	Test Purpose and Environment	5447

A.16.5.2.7.2	Test Requirements	5453

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5453

A.16.5.2.8.1	Test Purpose and Environment	5453

A.16.5.2.8.2	Test Requirements	5459

A.16.5.3	Active BWP switch	5459

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5459

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5459

A.16.5.3.1.1.1	Test Purpose and Environment	5459

A.16.5.3.1.1.2	Test Requirements	5464

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5464

A.16.5.3.1.1.2	Test Purpose and Environment	5464

A.16.5.3.1.2.2	Test Requirements	5469

A.16.5.3.2	RRC-based Active BWP Switch	5469

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5469

A.16.5.3.2.1.1	Test Purpose and Environment	5469

A.16.5.3.2.1.2	Test Requirements	5473

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5473

A.16.5.3.2.2.1	Test Purpose and Environment	5473

A.16.5.3.2.2.2	Test Requirements	5477

A.16.5.4	UE specific CBW change	5477

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5477

A.16.5.4.1.1	Test Purpose and Environment	5477

A.16.5.4.1.2	Test Requirements	5481

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5481

A.16.5.4.2.1	Test Purpose and Environment	5481

A.16.5.4.2.2	Test Requirements	5485

A.16.6	Measurement procedure for RedCap	5485

A.16.6.1	Intra-frequency Measurements	5485

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5485

A.16.6.1.1.1	Test purpose and Environment	5485

A.16.6.1.1.2	Test parameters	5485

A.16.6.1.1.3	Test Requirements	5489

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5489

A.16.6.1.2.1	Test purpose and Environment	5489

A.16.6.1.2.2	Test parameters	5489

A.16.6.1.2.3	Test Requirements	5493

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5493

A.16.6.1.3.1	Test purpose and Environment	5493

A.16.6.1.3.2	Test parameters	5493

A.16.6.1.3.3	Test Requirements	5497

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5497

A.16.6.1.4.1	Test purpose and Environment	5497

A.16.6.1.4.2	Test parameters	5497

A.16.6.1.4.3	Test Requirements	5501

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5501

A.16.6.1.5.1	Test purpose and Environment	5501

A.16.6.1.5.2	Test parameters	5501

A.16.6.1.5.3	Test Requirements	5505

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5505

A.16.6.1.6.1	Test purpose and Environment	5505

A.16.6.1.6.2	Test parameters	5505

A.16.6.1.6.3	Test Requirements	5509

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5509

A.16.6.1.7.1	Test purpose and Environment	5509

A.16.6.1.7.2	Test parameters	5509

A.16.6.1.7.3	Test Requirements	5513

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5513

A.16.6.1.8.1	Test purpose and Environment	5513

A.16.6.1.8.2	Test parameters	5513

A.16.6.1.8.3	Test Requirements	5517

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5517

A.16.6.1.9.1	Test purpose and Environment	5517

A.16.6.1.9.2	Test parameters	5517

A.16.6.1.9.3	Test Requirements	5519

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5520

A.16.6.1.10.1	Test purpose and Environment	5520

A.16.6.1.10.2	Test parameters	5520

A.16.6.1.10.3	Test Requirements	5521

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5522

A.16.6.1.11.1	Test purpose and Environment	5522

A.16.6.1.11.2	Test parameters	5522

A.16.6.1.11.3	Test Requirements	5524

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5525

A.16.6.1.12.1	Test purpose and Environment	5525

A.16.6.1.12.2	Test parameters	5525

A.16.6.1.12.3	Test Requirements	5527

A.16.6.2	Inter-frequency Measurements	5528

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5528

A.16.6.2.1.1	Test Purpose and Environment	5528

A.16.6.2.1.2	Test Requirements	5533

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5533

A.16.6.2.2.1	Test Purpose and Environment	5533

A.16.6.2.2.2	Test Requirements	5538

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5538

A.16.6.2.3.1	Test Purpose and Environment	5538

A.16.6.2.3.2	Test Requirements	5541

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5542

A.16.6.2.4.1	Test Purpose and Environment	5542

A.16.6.2.4.2	Test Requirements	5545

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5546

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5546

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5546

A.16.6.2.7.1	Test Purpose and Environment	5546

A.16.6.2.7.2	Test Requirements	5550

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5551

A.16.6.2.8.1	Test Purpose and Environment	5551

A.16.6.2.8.2	Test Requirements	5555

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5556

A.16.6.2.9.1	Test Purpose and Environment	5556

A.16.6.2.9.2	Test Requirements	5558

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5559

A.16.6.2.10.1	Test Purpose and Environment	5559

A.16.6.2.10.2	Test Requirements	5561

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5562

A.16.6.2.11.1	Test Purpose and Environment	5562

A.16.6.2.11.2	Test Requirements	5565

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5565

A.16.6.2.12.1	Test Purpose and Environment	5565

A.16.6.2.12.2	Test Requirements	5569

A.16.6.3	Inter-RAT Measurements	5570

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5570

A.16.6.3.1.1	Test purpose and Environment	5570

A.16.6.3.1.2	Test Requirements	5575

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5575

A.16.6.3.2.1	Test purpose and Environment	5575

A.16.6.3.2.2	Test Requirements	5580

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5580

A.16.6.3.3.1	Test purpose and Environment	5580

A.16.6.3.3.2	Test Requirements	5585

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5585

A.16.6.3.4.1	Test purpose and Environment	5585

A.16.6.3.4.2	Test Requirements	5590

A.16.6.4	L1-RSRP measurement for beam reporting	5590

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5590

A.16.6.4.1.1	Test Purpose and Environment	5590

A.16.6.4.1.2	Test parameters	5590

A.16.6.4.1.3	Test Requirements	5593

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5593

A.16.6.4.2.1	Test Purpose and Environment	5593

A.16.6.4.2.2	Test parameters	5594

A.16.6.4.2.3	Test Requirements	5597

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5597

A.16.6.4.3.1	Test Purpose and Environment	5597

A.16.6.4.3.2	Test parameters	5598

A.16.6.4.3.3	Test Requirements	5601

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5601

A.16.6.4.4.1	Test Purpose and Environment	5601

A.16.6.4.4.2	Test parameters	5602

A.16.6.4.4.3	Test Requirements	5605

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5605

A.16.6.4.5.1	Test Purpose and Environment	5605

A.16.6.4.5.2	Test parameters	5606

A.16.6.4.5.3	Test Requirements	5609

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5609

A.16.6.4.6.1	Test Purpose and Environment	5609

A.16.6.4.6.2	Test parameters	5610

A.16.6.4.6.3	Test Requirements	5613

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5613

A.16.6.4.7.1	Test Purpose and Environment	5613

A.16.6.4.7.2	Test parameters	5614

A.16.6.4.7.3	Test Requirements	5617

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5617

A.16.6.4.8.1	Test Purpose and Environment	5617

A.16.6.4.8.2	Test parameters	5618

A.16.6.4.8.3	Test Requirements	5621

A.16.6.7	NR measurements with autonomous gaps	5621

A.16.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5621

A.16.6.7.1.1	Test Purpose and Environment	5621

A.16.6.7.1.2	Test Parameters	5621

A.16.6.7.1.3	Test Requirements	5624

A.16.6.7.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5624

A.16.6.7.2.1	Test Purpose and Environment	5624

A.16.6.7.2.2	Test Parameters	5624

A.16.6.7.2.3	Test Requirements	5627

A.16.6.7.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5627

A.16.6.7.3.1	Test Purpose and Environment	5627

A.16.6.7.3.2	Test Requirements	5634

A.16.6.7.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5634

A.16.6.7.4.1	Test Purpose and Environment	5634

A.16.6.7.4.2	Test Requirements	5641

A.16.7	Measurement Performance requirements for RedCap	5641

A.16.7.1	SS-RSRP	5641

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5641

A.16.7.1.1.1	Test Purpose and Environment	5641

A.16.7.1.1.2	Test parameters	5641

A.16.7.1.1.3	Test Requirements	5646

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5646

A.16.7.1.2.1	Test Purpose and Environment	5646

A.16.7.1.2.2	Test parameters	5646

A.16.7.1.2.3	Test Requirements	5651

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5651

A.16.7.1.3.1	Test Purpose and Environment	5651

A.16.7.1.3.2	Test parameters	5651

A.16.7.1.3.3	Test Requirements	5655

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5655

A.16.7.1.4.1	Test Purpose and Environment	5655

A.16.7.1.4.2	Test parameters	5656

A.16.7.1.4.3	Test Requirements	5660

A.16.7.2	SS-RSRQ	5660

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5660

A.16.7.2.1.1	Test Purpose and Environment	5660

A.16.7.2.1.2	Test Parameters	5661

A.16.7.2.1.3	Test Requirements	5666

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5666

A.16.7.2.2.1	Test Purpose and Environment	5666

A.16.7.2.2.2	Test Parameters	5666

A.16.7.2.2.3	Test Requirements	5671

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5671

A.16.7.2.3.1	Test Purpose and Environment	5671

A.16.7.2.3.2	Test parameters	5671

A.16.7.2.3.3	Test Requirements	5676

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5676

A.16.7.2.4.1	Test Purpose and Environment	5676

A.16.7.2.4.2	Test parameters	5676

A.16.7.2.4.3	Test Requirements	5682

A.16.7.3	SS-SINR	5682

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5682

A.16.7.3.1.1	Test Purpose and Environment	5682

A.16.7.3.1.2	Test parameters	5683

A.16.7.3.1.3	Test Requirements	5687

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5687

A.16.7.3.2.1	Test Purpose and Environment	5687

A.16.7.3.2.2	Test parameters	5688

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5692

A.16.7.3.3.1	Test Purpose and Environment	5692

A.16.7.3.3.2	Test parameters	5692

A.16.7.3.3.3	Test Requirements	5697

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5698

A.16.7.3.4.1	Test Purpose and Environment	5698

A.16.7.3.4.2	Test parameters	5698

A.16.7.3.4.3	Test Requirements	5702

A.16.7.4	L1-RSRP measurement for beam reporting	5703

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5703

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5703

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5703

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5703

A.16.7.5	E-UTRAN RSRP	5703

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5703

A.16.7.5.1.1	Test Purpose and Environment	5703

A.16.7.5.1.2	Test parameters	5703

A.16.7.5.1.3	Test Requirements	5709

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5710

A.16.7.5.2.1	Test Purpose and Environment	5710

A.16.7.5.2.2	Test parameters	5710

A.16.7.5.2.3	Test Requirements	5716

A.16.7.6	E-UTRAN RSRQ	5717

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5717

A.16.7.6.1.1	Test Purpose and Environment	5717

A.16.7.6.1.2	Test parameters	5717

A.16.7.6.1.3	Test Requirements	5723

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5724

A.16.7.6.2.1	Test Purpose and Environment	5724

A.16.7.6.2.2	Test parameters	5724

A.16.7.6.2.3	Test Requirements	5730

A.16.7.7	E-UTRAN RS-SINR	5731

A.16.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5731

A.16.7.7.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5731

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5372

A.17.1	SA: RRC_IDLE state mobility for RedCap	5372

A.17.1.1	Cell re-selection to NR	5372

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5372

A.17.1.1.1.1	Test Purpose and Environment	5372

A.17.1.1.1.2	Test Parameters	5372

A.17.1.1.1.3	Test Requirements	5376

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5376

A.17.1.1.2.1	Test Purpose and Environment	5376

A.17.1.1.2.2	Test Parameters	5376

A.17.1.1.2.3	Test Requirements	5380

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5380

A.17.1.1.3.1	Test Purpose and Environment	5380

A.17.1.1.3.2	Test Parameters	5380

A.17.1.1.3.3	Test Requirements	5383

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5383

A.17.1.1.4.1	Test Purpose and Environment	5383

A.17.1.1.4.2	Test Parameters	5383

A.17.1.1.4.3	Test Requirements	5386

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5386

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5386

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5386

A.17.2.1.1.1	Test Purpose and Environment	5386

A.17.2.1.1.2	Test Requirements	5390

A.17.3	RRC_CONNECTED state mobility for RedCap	5390

A.17.3.1	Handover for RedCap	5390

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5390

A.17.3.1.1.1	Test Purpose and Environment	5390

A.17.3.1.1.2	Test Parameters	5390

A.17.3.1.1.3	Test Requirements	5393

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5393

A.17.3.1.2.1	Test Purpose and Environment	5393

A.17.3.1.2.2	Test Parameters	5393

A.17.3.1.2.3 Test Requirements	5395

A.17.3.2	RRC Connection Mobility Control for RedCap	5395

A.17.3.2.1	SA: RRC Re-establishment	5395

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5395

A.17.3.2.1.1.1	Test Purpose and Environment	5395

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5398

A.17.3.2.1.2.1	Test Purpose and Environment	5398

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5401

A.17.3.2.1.3.1	Test Purpose and Environment	5401

A.17.3.2.1.3.2	Test Requirements	5403

A.17.3.2.2	Random Access	5404

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5404

A.17.3.2.2.1.1	Test Purpose and Environment	5404

A.17.3.2.2.1.2	Test Requirements	5406

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5408

A.17.3.2.2.2.1	Test Purpose and Environment	5408

A.17.3.2.2.2.2	Test Requirements	5410

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5412

A.17.3.2.2.3.1	Test Purpose and Environment	5412

A.17.3.2.2.3.2	Test Requirements	5414

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5415

A.17.3.2.2.4.1	Test Purpose and Environment	5415

A.17.3.2.2.4.2	Test Requirements	5417

A.17.3.2.3	SA: RRC Connection Release with Redirection	5418

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5418

A.17.3.2.3.1.1	Test Purpose and Environment	5418

A.17.3.2.3.1.2	Test Parameters	5418

A.17.3.2.3.1.3	Test Requirements	5422

A.17.4	Timing	5422

A.17.4.1	UE transmit timing	5422

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5422

A.17.4.1.1.1	Test Purpose and environment	5422

A.17.4.1.1.2	Test requirements	5425

A.17.4.2	UE timer accuracy	5426

A.17.4.3	Timing advance	5426

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5426

A.17.4.3.1.1	Test Purpose and Environment	5426

A.17.4.3.1.2	Test Parameters	5426

A.17.4.3.1.3 Test Requirements	5429

A.17.4.2	UE timer accuracy	5429

A.17.4.3	Timing advance	5429

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5429

A.17.5	Signaling characteristics for RedCap	5429

A.17.5.1	Radio link Monitoring for RedCap	5429

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5429

A.17.5.1.1.1	Test Purpose and Environment	5429

A.17.5.1.1.2	Test Requirements	5432

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5433

A.17.5.1.2.1	Test Purpose and Environment	5433

A.17.5.1.2.2	Test Requirements	5437

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5438

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5438

A.17.5.1.4.1	Test Purpose and Environment	5438

A.17.5.1.4.2	Test Requirements	5442

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5442

A.17.5.1.5.1	Test Purpose and Environment	5442

A.17.5.1.5.2	Test Requirements	5447

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5447

A.17.5.1.6.1	Test Purpose and Environment	5447

A.17.5.1.6.2	Test Requirements	5451

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5451

A.17.5.1.7.1	Test Purpose and Environment	5451

A.17.5.1.7.2	Test Requirements	5455

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5455

A.17.5.1.8.1	Test Purpose and Environment	5455

A.17.5.1.8.2	Test Requirements	5460

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5460

A.17.5.1.9.1	Test Purpose and Environment	5460

A.17.5.1.9.2	Test Requirements	5463

A.17.5.2	Beam Failure Detection and Link recovery procedures	5463

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5463

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5463

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5463

A.17.5.2.3.1	Test Purpose and Environment	5463

A.17.5.2.3.2	Test Requirements	5467

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5467

A.17.5.2.4.1	Test Purpose and Environment	5467

A.17.5.2.4.2	Test Requirements	5471

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5471

A.17.5.2.5.1	Test Purpose and Environment	5471

A.17.5.2.5.2	Test Requirements	5475

A.17.5.3	Active BWP switch for RedCap	5475

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5475

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5475

A.17.5.3.1.1.1	Test Purpose and Environment	5475

A.17.5.3.1.1.2	Test Requirements	5478

A.17.5.3.2	RRC-based Active BWP Switch	5478

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5478

A.17.5.3.2.1.1	Test Purpose and Environment	5478

A.17.5.3.2.1.2	Test Requirements	5481

A.17.5.4	Active TCI state switch delay	5482

A.17.5.4.1	MAC-CE based active TCI state switch	5482

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5482

A.17.5.4.1.1.1	Test Purpose and Environment	5482

A.17.5.4.1.1.2	Test Requirements	5485

A.17.5.4.2	RRC based active TCI state switch	5485

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5485

A.17.5.4.2.1.1	Test Purpose and Environment	5485

A.17.5.4.2.1.2	Test Requirements	5489

A.17.5.5	Uplink spatial relation switch delay	5489

A.17.5.5.1	MAC-CE based Spatial Relation switch	5489

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5489

A.17.5.5.1.1.1	Test Purpose and Environment	5489

A.17.5.5.1.1.2	Test Requirements	5492

A.17.5.5.2	RRC based spatial relation switch	5492

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5492

A.17.5.5.2.1.2	Test Requirements	5495

A.17.5.6	UE specific CBW change	5495

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5495

A.17.5.6.1.1	Test Purpose and Environment	5495

A.17.5.6.1.2	Test Requirements	5498

A.17.6	Measurement procedure for RedCap	5499

A.17.6.1	Intra-frequency Measurements	5499

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5499

A.17.6.1.1.1	Test purpose and Environment	5499

A.17.6.1.1.2	Test Requirements	5502

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5502

A.17.6.1.2.1	Test purpose and Environment	5502

A.7.6.1.2.2	Test Requirements	5502

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5503

A.17.6.1.3.1	Test purpose and Environment	5503

A.17.6.1.3.2	Test Requirements	5507

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5507

A.17.6.1.4.1	Test purpose and Environment	5507

A.17.6.1.4.2	Test Requirements	5510

A.17.6.2	Inter-frequency Measurements	5511

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5511

A.17.6.2.1.1	Test Purpose and Environment	5511

A.17.6.2.1.2	Test Requirements	5514

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5514

A.17.6.2.2.1	Test Purpose and Environment	5514

A.17.6.2.2.2	Test Requirements	5518

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5518

A.17.6.2.3.1	Test Purpose and Environment	5518

A.17.6.2.3.2	Test Requirements	5522

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	5522

A.17.6.2.4.1	Test Purpose and Environment	5522

A.17.6.2.4.2	Test Requirements	5526

A.17.6.3	L1-RSRP measurement for beam reporting	5526

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	5526

A.17.6.3.1.1	Test Purpose and Environment	5526

A.17.6.3.1.2	Test parameters	5527

A.17.6.3.1.3	Test Requirements	5527

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	5527

A.17.6.3.2.1	Test Purpose and Environment	5527

A.17.6.3.2.2	Test parameters	5527

A.17.6.3.2.3	Test Requirements	5529

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	5529

A.17.6.3.3.1	Test Purpose and Environment	5529

A.17.6.3.3.2	Test parameters	5530

A.17.6.3.3.3	Test Requirements	5532

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	5533

A.17.6.3.4.1	Test Purpose and Environment	5533

A.17.6.3.4.2	Test parameters	5533

A.7.6.3.3.3	Test Requirements	5535

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	5536

A.17.6.4.1.1	Test Purpose and Environment	5536

A.17.6.4.1.2	Test Requirements	5539

A.17.7	Measurement Performance requirements	5539

A.17.7.1	SS-RSRP	5539

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5539

A.17.7.1.1.1	Test Purpose and Environment	5539

A.17.7.1.1.2	Test parameters	5539

A.17.7.1.1.3	Test Requirements	5543

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	5544

A.17.7.1.2.1	Test Purpose and Environment	5544

A.17.7.1.2.2	Test parameters	5544

A.17.7.1.2.3	Test Requirements	5548

A.17.7.2	SS-RSRQ	5549

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	5549

A.17.7.2.1.1	Test Purpose and Environment	5549

A.17.7.2.1.2	Test Parameters	5549

A.17.7.2.1.3	Test Requirements	5551

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	5551

A.17.7.2.2.1	Test Purpose and Environment	5551

A.17.7.2.2.2	Test parameters	5551

A.17.7.2.2.3	Test Requirements	5553

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	5553

A.17.7.3	L1-RSRP measurement for beam reporting	5553

A.17.7.3.1	SSB based L1-RSRP measurement	5553

A.17.7.3.1.1	Test Purpose and Environment	5553

A.17.7.3.1.2	Test parameters	5554

A.17.7.3.1.3	Test Requirements	5554

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5554

A.17.7.3.2.1	Test Purpose and Environment	5554

A.17.7.3.2.2	Test parameters	5554

A.17.7.3.2.3	Test Requirements	5555

A.17.7.4	SS-SINR	5555

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	5555

A.17.7.4.1.1	Test Purpose and Environment	5555

A.17.7.4.1.2	Test parameters	5555

A.17.7.4.1.3	Test Requirements	5557

A.18	E-UTRA standalone tests for NR RRM for RedCap	5557

A.18.1	RRC_IDLE state mobility	5557

A.18.1.1	Inter-RAT NR Cell re-selection	5557

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	5557

A.18.1.1.1.1	Test Purpose and Environment	5557

A.18.2.1.1.2	Test Requirements	5562

A.18.2	RRC_CONNECTED state mobility	5562

A.18.2.1	Handover	5562

A.18.2.1.1	E-UTRAN - NR handover in FR1	5562

A.18.2.1.1.1	Test Purpose and Environment	5562

A.18.2.1.1.2	Test Requirements	5567

A.18.2.2	RRC connection release with redirection	5567

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	5567

A.18.2.2.1.1	Test Purpose and Environment	5567

A.18.2.2.1.2	Test Parameters	5567

A.18.2.2.1.3	Test Requirements	5574

A.18.3	Measurement procedure	5574

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	5574

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	5574

A.18.3.1.1.1	Test Purpose and Environment	5574

A.18.3.1.1.2	Test Requirements	5580

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	5580

A.18.3.1.2.1	Test Purpose and Environment	5580

A.18.3.1.2.2	Test Requirements	5586

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	5586

A.18.3.1.3.1	Test Purpose and Environment	5586

A.18.3.1.3.2	Test Requirements	5592

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	5592

A.18.3.1.4.1	Test Purpose and Environment	5592

A.18.3.1.4.2	Test Requirements	5598

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	5598

A.18.3.1.5.1	Test Purpose and Environment	5598

A.18.3.1.5.2	Test Requirements	5600

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	5601

A.18.3.1.6.1	Test Purpose and Environment	5601

A.18.3.1.6.2	Test Requirements	5603

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	5604

A.18.3.1.7.1	Test Purpose and Environment	5604

A.18.3.1.7.2	Test Requirements	5606

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	5607

A.18.3.1.8.1	Test Purpose and Environment	5607

A.18.3.1.8.2	Test Requirements	5609

Annex B (normative): Conditions for RRM requirements applicability for operating bands	5611

B.1	Conditions for NR RRC_IDLE state mobility	5611

B.1.1	Introduction	5611

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	5611

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	5612

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	5612

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	5614

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	5614

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	5615

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	5615

B.2.1	Introduction	5615

B.2.1.1	General	5615

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	5615

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	5615

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	5615

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	5616

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	5617

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	5617

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	5617

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	5618

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	5618

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	5618

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	5618

B.2.2	Conditions for NR intra-frequency measurements	5619

B.2.3	Conditions for NR inter-frequency measurements	5620

B.2.4	Conditions for NR L1-RSRP reporting	5621

B.2.4.1	Conditions for SSB based L1-RSRP reporting	5621

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	5622

B.2.5	Conditions for RRC connection release with redirection to NR	5624

B.2.6	Void	5625

B.2.6.1	Void	5625

B.2.6.2	Void	5625

B.2.7	Conditions for SRS-RSRP measurements	5625

B.2.8	Conditions for NR L1-SINR reporting	5626

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	5626

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	5627

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	5627

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	5627

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	5629

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	5629

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	5630

B.2.9	Conditions for NR intra-frequency measurements under CCA	5631

B.2.10	Conditions for NR inter-frequency measurements under CCA	5631

B.2.11	Conditions for NR L1-RSRP reporting under CCA	5631

B.2.11.1	Conditions for SSB based L1-RSRP reporting	5631

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	5632

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	5633

B.2.14	Conditions for NR PRS-based measurements	5635

B.2.15	Conditions for NR intra-frequency measurements	5637

B.2.16	Conditions for NR inter-frequency measurements	5638

B.2.17	Conditions for NR intra-frequency measurements for satellite access	5640

B.2.18	Conditions for NR inter-frequency measurements for satellite access	5641

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	5641

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	5641

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	5641

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	5642

B.3	RRM Requirements Exceptions	5642

B.3.1	Introduction	5642

B.3.2	Receiver sensitivity relaxation for CA	5642

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	5642

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	5642

B.3.2.2.1	Inter-band carrier aggregation	5642

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	5642

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	5643

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	5643

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	5643

B.3.2.4.1	Intra-band contiguous carrier aggregation	5643

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	5643

B.3.3	Receiver sensitivity relaxation for DC	5643

B.3.3.1	Receiver sensitivity relaxation for EN-DC	5643

B.3.3.2	Receiver sensitivity relaxation for NE-DC	5643

B.3.4	Receiver sensitivity relaxation for SUL	5643

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	5643

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	5644

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	5644

B.4	Conditions for V2X	5644

B.4.1	Test parameters for GNSS signals	5644

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	5644

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	5645

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	5645

B.5	High level test procedure for SAN RRM tests	5645

Annex C (informative): Change history	5646