| 3GPP TS 38.133 V18.4.0 (2023-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	124

1	Scope	126

2	References	126

3	Definitions, symbols and abbreviations	127

3.1	Definitions	127

3.2	Symbols	129

3.3	Abbreviations	130

3.4	Test tolerances	133

3.5	Frequency bands grouping	133

3.5.1	Introduction	133

3.5.2	NR operating bands in FR1	133

3.5.2A	NR operating bands for satellite access in FR1	134

3.5.3	NR operating bands in FR2	135

3.6	Applicability of requirements in this specification version	136

3.6.1	RRC connected state requirements in DRX	136

3.6.2	Number of serving carriers	136

3.6.2.1	Number of serving carriers for SA	136

3.6.2.2	Number of serving carriers for EN-DC	136

3.6.2.3	Number of serving carriers for NE-DC	137

3.6.2.4	Number of serving carriers for NR-DC	137

3.6.3	Applicability for intra-band FR2	137

3.6.4	Applicability for FR2 UE power classes	137

3.6.5	Applicability for SDL bands	137

3.6.6	Applicability of requirements for NGEN-DC operation	137

3.6.7	Applicability of QCL	137

3.6.9	Applicability of requirements for scheduling availability	138

3.6.10	Applicability of requirements for measurement restrictions	139

3.6.11	Applicability of requirements for Redcap UEs	139

3.6.11.1	RRC connected state requirements in DRX	139

3.6.11.2	Applicability for FR2 Redcap UE power classes	139

3.6.11.3	Applicability of QCL	139

3.6.12	Applicability of requirements for Satellite Access	139

3.6.13	Applicability of requirements for FR2	139

3.6.14	Applicability of requirements for FR2 Power Class 6	139

3.6.15	Applicability of requirements for per-FR gap	139

3.6.16	Applicability of requirements for ATG	140

3.6.17	Applicability of requirements for MUSIM gaps	140

4	SA: RRC_IDLE state mobility	140

4.1	Cell Selection	140

4.2	Cell Re-selection	140

4.2.1	Introduction	140

4.2.2	Requirements	140

4.2.2.1	UE measurement capability	140

4.2.2.2	Measurement and evaluation of serving cell	141

4.2.2.3	Measurements of intra-frequency NR cells	143

4.2.2.4	Measurements of inter-frequency NR cells	147

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	152

4.2.2.6	Maximum interruption in paging reception	155

4.2.2.7	General requirements	156

4.2.2.8	Minimum requirement at transitions	156

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	156

4.2.2.9.1	Introduction	156

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	157

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	159

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	161

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	162

4.2.2.10.1	Introduction	162

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	162

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	165

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	168

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	168

4.2.2.11.1	Introduction	168

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	169

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	170

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	172

4.2A	Cell Re-selection when subject to CCA	173

4.2A.1	Introduction	173

4.2A.2	Requirements	173

4.2A.2.1	UE measurement capability	173

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	173

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	174

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	176

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	177

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	178

4.2A.2.7	General requirements	178

4.2B	Cell Re-selection for RedCap	178

4.2B.1	Introduction	178

4.2B.2	Requirements	178

4.2B.2.1	UE measurement capability for RedCap	178

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	178

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	179

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	179

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	180

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	183

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	186

4.2B.2.6	Maximum interruption in paging reception for RedCap	188

4.2B.2.7	General requirements for RedCap	188

4.2B.2.8	Minimum requirement at transitions	188

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	189

4.2B.2.9.1	Introduction	189

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	190

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	193

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	193

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	194

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	194

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	194

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	194

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	194

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	195

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	198

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	200

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	200

4.2B.2.10.1	Introduction	200

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	201

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary  criterion	203

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	204

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	204

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	205

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	205

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	205

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	205

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	206

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	208

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	211

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	211

4.2B.2.11.1	Introduction	211

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	212

4.2B.2.11.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	213

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	213

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	214

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary  criteria	214

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	214

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	215

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	215

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	215

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	217

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	218

4.2C	Cell Re-selection for NR UE for Satellite Access	218

4.2C.1	Introduction	218

4.2C.2	Requirements	219

4.2C.2.1	UE measurement capability	219

4.2C.2.2	Measurement and evaluation of serving cell	219

4.2C.2.3	Measurements of intra-frequency NR cells	220

4.2C.2.4	Measurements of inter-frequency NR cells	222

4.2C.2.5	Maximum interruption in paging reception	225

4.2C.2.6	Minimum requirement at transitions	226

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	226

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	226

4.2C.2.9	General requirements	226

4.2D	Cell Re-selection for ATG	226

4.2D.1	Introduction	226

4.2D.2	Requirements	226

4.2D.2.1	UE measurement capability	226

4.2D.2.2	Measurement and evaluation of serving cell	227

4.2D.2.3	Measurements of intra-frequency NR cells	227

4.2D.2.4	Measurements of inter-frequency NR cells	228

4.2D.2.5	Maximum interruption in paging reception	230

4.2D.2.6	General requirements	230

4.3	Minimization of Drive Tests (MDT)	230

4.3.1	Introduction	230

4.3.2	Measurement Requirements	231

4.3.3	Requirements for Relative Time Stamp Accuracy	231

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	231

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	232

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	232

4.3C.1	Introduction	232

4.3C.2	Measurement Requirements	232

4.3C.3	Requirements for Relative Time Stamp Accuracy	233

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	233

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	233

4.4	Idle Mode CA/DC Measurements	234

4.4.1	Introduction	234

4.4.2	Measurement Requirements	234

4.4.2.1	Detected cell requirement during state transition and Idle mode	234

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	234

4.4.2.3	Measurements on serving cell	236

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	236

4.5	NR measurements for positioning	236

4.5.1	Introduction	236

4.5.2	RSTD measurements	237

4.5.2.1	Introduction	237

4.5.2.2	Requirements Applicability	237

4.5.2.3	Measurement Capability	237

4.5.2.4	Measurement Reporting Requirements	237

4.5.2.5	Measurements Period Requirements	238

4.5.3	PRS-RSRP measurements	241

4.5.3.1	Introduction	241

4.5.3.2	Requirements applicability	241

4.5.3.3	Measurement Capability	241

4.5.3.4	Measurement Reporting Requirements	241

4.5.3.5	Measurement Period Requirements	241

4.5.4	PRS-RSRPP measurements	244

4.5.4.1	Introduction	244

4.5.4.2	Requirements Applicability	244

4.5.4.3	Measurement Capability	244

4.5.4.4	Measurement Reporting Requirements	244

4.5.4.5	Measurement Period Requirements	244

4.5.5	Measurement requirements for DL RSCPD reported with RSTD	245

4.5.5.1	Introduction	245

4.5.5.2	Requirements Applicability	245

4.5.5.3	Measurement Capability	245

4.5.5.4	Measurement Reporting Requirements	245

4.5.5.5	Measurements Period Requirements	245

4.6	NR measurements for positioning for RedCap	248

4.6.1	Introduction	248

4.6.2	RSTD measurements for RedCap	249

4.6.2.1	Introduction	249

4.6.2.2	Requirements Applicability	249

4.6.2.3	Measurement Capability	249

4.6.2.4	Measurement Reporting Requirements	249

4.6.2.5	Measurements Period Requirements without RX FH	249

4.6.2.6	Measurement Period Requirements with RX FH	250

4.6.3	PRS-RSRP measurements for RedCap	250

4.6.3.1	Introduction	250

4.6.3.2	Requirements applicability	250

4.6.3.3	Measurement Capability	251

4.6.3.4	Measurement Reporting Requirements	251

4.6.3.5	Measurement Period Requirements without FH	251

4.6.4	PRS-RSRPP measurements for RedCap	254

4.6.4.1	Introduction	254

4.6.4.2	Requirements Applicability	254

4.6.4.3	Measurement Capability	254

4.6.4.4	Measurement Reporting Requirements	254

4.6.4.5	Measurement Period Requirements without FH	254

4.7	Measurement report for fast CA/DC setup	255

4.7.1	Introduction	255

4.7.2	Measurement Requirements	255

4.7.3	Measurement Report Requirements	255

5	SA: RRC_INACTIVE state mobility	256

5.1	Cell Re-selection	256

5.1.1	Introduction	256

5.1.2	Requirements	256

5.1.2.1	UE measurement capability	256

5.1.2.2	Measurement and evaluation of serving cell	256

5.1.2.3	Measurements of intra-frequency NR cells	257

5.1.2.4	Measurements of inter-frequency NR cells	258

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	259

5.1.2.6	Maximum interruption in paging reception	260

5.1.2.7	General requirements	260

5.1A	Cell Re-selection with CCA	260

5.1A.1	Introduction	260

5.1A.2	Requirements	260

5.1A.2.1	UE measurement capability	260

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	261

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	261

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	261

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	261

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	261

5.1A.2.7	General requirements	261

5.1B	Cell Re-selection for RedCap	261

5.1B.1	Introduction	261

5.1B.2	Requirements	261

5.1B.2.1	UE measurement capability	261

5.1B.2.2	Measurement and evaluation of serving cell	261

5.1B.2.3	Measurements of intra-frequency NR cells	264

5.1B.2.4	Measurements of inter-frequency NR cells	265

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	268

5.1B.2.6	Maximum interruption in paging reception	269

5.1B.2.7	General requirements	269

5.1B.2.8	Minimum requirement at transitions	269

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	269

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	269

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	269

5.1C	Cell Re-selection	269

5.1C.1	Introduction	269

5.1C.2	Requirements	269

5.1C.2.1	UE measurement capability	269

5.1C.2.2	Measurement and evaluation of serving cell	270

5.1C.2.3	Measurements of intra-frequency NR cells	270

5.1C.2.4	Measurements of inter-frequency NR cells	270

5.1C.2.5	Maximum interruption in paging reception	270

5.1C.2.6	General requirements	270

5.1D	Cell Re-selection for ATG	270

5.1D.1	Introduction	270

5.1D.2	Requirements	270

5.1D.2.1	UE measurement capability	270

5.1D.2.2	Measurement and evaluation of serving cell	270

5.1D.2.3	Measurements of intra-frequency NR cells	270

5.1D.2.4	Measurements of inter-frequency NR cells	270

5.1D.2.5	Maximum interruption in paging reception	270

5.1D.2.6	General requirements	271

5.2	Void	271

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	271

5.2B.1	Introduction	271

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	271

5.2B.2.1	Void5.2B.3 TA validation requirements for RedCap	271

5.2B.2.2	Void	272

5.2B.4	Scheduling restriction	272

5.2B.5	Applicability conditions for CG-SDT for RedCap	272

5.3	Minimization of Drive Tests (MDT)	273

5.3.1	Introduction	273

5.3.2	Measurement Requirements	273

5.3.3	Requirements for Relative Time Stamp Accuracy	273

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	273

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	273

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	273

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	274

5.3C.1	Introduction	274

5.3C.2	Measurement Requirements	274

5.3C.3	Requirements for Relative Time Stamp Accuracy	274

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	274

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	274

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	274

5.4	Idle Mode CA/DC Measurements	275

5.4.1	Introduction	275

5.4.2	Measurement Requirements	275

5.4.2.1	Detected cell requirement during state transition and Idle mode	275

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	275

5.4.2.3	Measurements on serving cell	275

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	275

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	275

5.5.1	Introduction	275

5.5.2	Requirements on UE synchronization for small data transmissions	275

5.5.3	TA validation requirements	276

5.5.4	Scheduling restriction	276

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	276

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	277

5.5.4.3	Scheduling availability of UE performing measurements on FR2	277

5.5.5	Applicability conditions for SDT	277

5.5D	Configured Grant based Small Data Transmissions (CG-SDT) for ATG	278

5.6	NR measurements for positioning	278

5.6.1	Introduction	278

5.6.1A	Cell re-selection for positioning	279

5.6.1A.1	Measurement and evaluation of serving cell	279

5.6.1A.2	Measurements of intra-frequency NR cells	280

5.6.2	RSTD measurements	281

5.6.2.1	Introduction	281

5.6.2.2	Requirements Applicability	281

5.6.2.3	Measurement Capability	281

5.6.2.5	Measurements Period Requirements	281

5.6.2.6	Measurements Period Requirements with Bandwidth Aggregation	284

5.6.3	PRS-RSRP measurements	287

5.6.3.1	Introduction	287

5.6.3.2	Requirements applicability	287

5.6.3.3	Measurement Capability	287

5.6.3.4	Measurement Reporting Requirements	287

5.6.3.5	Measurement Period Requirements	288

5.6.4	UE Rx-Tx time difference measurements	290

5.6.4.1	Introduction	290

5.6.4.2	Requirements Applicability	290

5.6.4.3	Measurement Capability	291

5.6.4.4	Measurement Reporting Requirements	291

5.6.4.5	Measurement Period Requirements	291

5.6.4.6	Measurement Period Requirements with Bandwidth Aggregation	294

5.6.5	PRS-RSRPP measurements	297

5.6.5.1	Introduction	297

5.6.5.2	Requirements Applicability	297

5.6.5.3	Measurement Capability	297

5.6.5.4	Measurement Reporting Requirements	297

5.6.5.5	Measurement Period Requirements	298

5.6.6	TA validation requirements for positioning	298

5.6.6.1	Introduction	298

5.6.6.2	TA validation requirements	298

5.6.6.3	TA validation requirements when configured with validity area	299

5.6.7	Measurement requirements for DL RSCPD reported with RSTD	300

5.6.7.1	Introduction	300

5.6.7.2	Requirements Applicability	300

5.6.7.3	Measurement Capability	300

5.6.7.4	Measurement Reporting Requirements	300

5.6.7.5	Measurements Period Requirements	300

5.6.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	301

5.6.8.1	Introduction	301

5.6.8.2	Requirements Applicability	301

5.6.8.3	Measurement Capability	302

5.6.8.4	Measurement Reporting Requirements	302

5.6.8.5	Measurement Period Requirements	302

5.6A	NR measurements for positioning for RedCap	305

5.6A.1	Introduction	305

5.6A.2	Cell re-selection for positioning	305

5.6A.2.1	Measurement and evaluation of serving cell	306

5.6A.2.2	Measurements of intra-frequency NR cells	307

5.6A.3	TA validation requirements for positioning SRS	307

5.6A.3.1	Introduction	307

5.6A.3.2	TA validation requirements	307

5.6A.3.3	TA validation requirements when configured with validity area	308

5.6A.4	RSTD measurements for RedCap	308

5.6A.4.1	 Introduction	308

5.6A.4.2	Requirements applicability	308

5.6A.4.3	Measurement Capability	308

5.6A.4.4	Measurement Reporting Requirements	308

5.6A.4.5	Measurement period requirement without FH	309

5.6A.4.6	Measurement period requirement with FH	312

5.6A.5	PRS-RSRP measurements for RedCap	312

5.6A.5.1	 Introduction	312

5.6A.5.2	Requirements applicability	313

5.6A.5.3	Measurement Capability	313

5.6A.5.4	Measurement Reporting Requirements	313

5.6A.5.5	Measurement Period Requirements	314

5.6A.5.6	Measurement period requirement with FH	316

5.6A.6	UE Rx-Tx time difference measurements for RedCap	317

5.6A.6.1	Introduction	317

5.6A.6.2	Requirements Applicability	317

5.6A.6.3	Measurement Capability	317

5.6A.6.4	Measurement Reporting Requirements	317

5.6A.6.5	Measurement Period Requirements without FH	318

5.6A.6.6	Measurement Period Requirements with FH	318

5.7	Random access based Small Data Transmissions (RA-SDT)	319

5.7.1	Introduction	319

5.7.2	Requirements for small data transmissions based on 2-step RA	319

5.7.3	Requirements for small data transmissions based on 4-step RA	319

5.7.4	Applicability conditions for SDT	319

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	319

5.7B.1	Introduction	319

5.7B.2	Requirements for small data transmissions based on 2-step RA	319

5.7B.3	Requirements for small data transmissions based on 4-step RA	319

5.7B.4	Applicability conditions for RA-SDT for RedCap	320

5.7D	Random access based Small Data Transmissions (RA-SDT) for ATG	320

5.8	Measurement report for fast CA/DC setup	320

5.8.1	Introduction	320

5.8.2	Measurement Requirements	320

5.8.3	Measurement Report Requirements	320

6	RRC_CONNECTED state mobility	320

6.1	Handover	320

6.1.1	NR Handover	320

6.1.1.1	Introduction	320

6.1.1.2	NR FR1 - NR FR1 Handover	321

6.1.1.2.1	Handover delay	321

6.1.1.2.2	Interruption time	321

6.1.1.3	NR FR2- NR FR1 Handover	322

6.1.1.3.1	Handover delay	322

6.1.1.3.2	Interruption time	322

6.1.1.4	NR FR2- NR FR2 Handover	323

6.1.1.4.1	Handover delay	323

6.1.1.4.2	Interruption time	323

6.1.1.5	NR FR1- NR FR2 Handover	324

6.1.1.5.1	Handover delay	324

6.1.1.5.2	Interruption time	325

6.1.2	NR Handover to other RATs	326

6.1.2.1	NR – E-UTRAN Handover	326

6.1.2.1.1	Introduction	326

6.1.2.1.2	Handover delay	326

6.1.2.1.3	Interruption time	326

6.1.2.2	NR – UTRAN Handover	327

6.1.2.2.1	Introduction	327

6.1.2.2.2	Handover delay	327

6.1.2.2.3	Interruption time	327

6.1.3	NR DAPS Handover	328

6.1.3.1	Introduction	328

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	328

6.1.3.2.1	DAPS handover delay	328

6.1.3.2.2	Interruption time	329

6.1.3.3	NR FR2- NR FR1 DAPS Handover	331

6.1.3.3.1	DAPS handover delay	331

6.1.3.3.2	Interruption time	331

6.1.3.4	NR FR1- NR FR2 DAPS Handover	332

6.1.3.4.1	DAPS handover delay	332

6.1.3.4.2	Interruption time	332

6.1.4	NR Conditional Handover	333

6.1.4.1	Introduction	333

6.1.4.2	NR FR1 – NR FR1 conditional handover	333

6.1.4.3	NR FR2 – NR FR1 conditional handover	335

6.1.4.4	NR FR2 – NR FR2 conditional handover	335

6.1.4.4.1	Handover delay	335

6.1.4.4.2	Measurement time	336

6.1.4.4.3	Preparation time	336

6.1.4.4.4	Interruption time	336

6.1.4.5	NR FR1 – NR FR2 conditional handover	337

6.1.5	NR Handover with PSCell	337

6.1.5.1	Introduction	337

6.1.5.2	Handover with PSCell from NR SA to EN-DC	337

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	338

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	338

6.1.5.3	HO with PSCell from NE-DC to NE-DC	338

6.1.5.3.1	Handover delay	339

6.1.5.3.2	HO with PSCell - PCell Interruption time	339

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	339

6.1.5.4	HO with PSCell from NR-DC to NR-DC	339

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	341

6.1.5.5.1	Introduction	341

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	341

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	342

6.1.6.1	Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC	343

6.1.6.1.1	CHO with PSCell – PCell Interruption time	343

6.1.6.1.2	CHO with PSCell – PSCell change delay	344

6.1.6.2	Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC	344

6.1.6.2.2	CHO with PSCell – PSCell change delay	345

6.1.7.1.2	PSCell conditional change delay	348

6.1.7.2	Conditional handover including target MCG in FR1 and Candidate SCG for CPC in FR2 in NR-DC	349

6.1.7.2.1	PCell handover delay	349

6.1.7.2.1.1	Measurement time	350

6.1.7.2.2	PSCell conditional change delay	350

6.1A	Void	351

6.1A.1	Void	351

6.1A.1.1	Void	351

6.1A.1.2	Void	351

6.1A.1.2.1	Void	351

6.1A.1.2.2	Void	351

6.1B	Handover to target cell using CCA	351

6.1B.1	NR Handover	351

6.1B.1.1	Introduction	351

6.1B.1.2	NR FR1 - NR FR1 Handover	352

6.1B.1.2.1	Handover delay	352

6.1B.1.2.2	Interruption time	352

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	353

6.1B.1.3.1	Handover delay	353

6.1B.1.3.2	Interruption time	353

6.1B.1.4	NR FR1- NR FR2-2 Handover	354

6.1B.1.4.1	Handover delay	354

6.1B.1.4.2	Interruption time	354

6.1C	Handover for SAN	355

6.1C.1	NR SAN Handover	355

6.1C.1.1	Introduction	355

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	355

6.1C.1.2.1	Handover delay	355

6.1C.1.2.2	Interruption time	356

6.1C.2	NR SAN Conditional Handover	356

6.1C.2.1	Introduction	356

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	356

6.1C.2.2.1	Handover delay	357

6.1C.2.2.2	Measurement time	357

6.1C.2.2.3	Preparation time	358

6.1C.2.2.4	Interruption time	358

6.1D	Handover for RedCap	359

6.1D.1	NR Handover	359

6.1D.1.1	Introduction	359

6.1D.1.2	NR FR1 - NR FR1 Handover	359

6.1D.1.2.1	Handover delay	359

6.1D.1.2.2	Interruption time	359

6.1D.1.3	NR FR2- NR FR2 Handover	361

6.1D.1.3.1	Handover delay	361

6.1D.1.3.2	Interruption time	361

6.1D.2	NR Handover to other RATs	362

6.1D.2.1	NR – E-UTRAN Handover	362

6.1E	Handover for ATG	362

6.1E.1	NR Handover	362

6.1E.1.1	Introduction	362

6.1E.1.2	NR FR1 - NR FR1 Handover	363

6.1E.1.2.1	Handover delay	363

6.1E.1.2.2	Interruption time	363

6.1E.2	NR Conditional Handover	364

6.1E.2.1	Introduction	364

6.1E.2.2	NR FR1 – NR FR1 conditional handover	364

6.1E.2.2.1	Handover delay	364

6.1E.2.2.2	Measurement time	364

6.1E.2.2.3	Preparation time	365

6.1E.2.2.4	Interruption time	365

6.2	RRC Connection Mobility Control	365

6.2.1	SA: RRC Re-establishment	365

6.2.1.1	Introduction	365

6.2.1.2	Requirements	365

6.2.1.2.1	UE Re-establishment delay requirement	366

6.2.1A	RRC Re-establishment with CCA	367

6.2.1A.1	Introduction	367

6.2.1A.2	Requirements	368

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	368

6.2.1B	SA: RRC Re-establishment for RedCap	370

6.2.1B.1	Introduction	370

6.2.1B.2	Requirements	370

6.2.2	Random access	370

6.2.2.1	Introduction	370

6.2.2.2	Requirements for 4-step RA type	371

6.2.2.2.1	Contention based random access	371

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	371

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	371

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	371

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	371

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	371

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	372

6.2.2.2.2	Non-Contention based random access	372

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	372

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	372

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	373

6.2.2.2.3	UE behaviour when configured with supplementary UL	373

6.2.2.3	Requirements for 2-step RA type	373

6.2.2.3.1	Contention based random access	373

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	373

6.2.2.3.1.2	Correct behaviour when receiving MsgB	374

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	374

6.2.2.3.2	Non-Contention based random access	374

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	374

6.2.2.3.2.2	Correct behaviour when receiving MsgB	374

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	374

6.2.2.3.3	UE behaviour when configured with supplementary UL	375

6.2.2A	Random access when CCA is used on target frequency	375

6.2.2A.1	Introduction	375

6.2.2A.2	Requirements for 4-step RA type	375

6.2.2A.2.1	Contention based random access	375

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	375

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	376

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	376

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	376

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	376

6.2.2A.2.2	Non-Contention based random access	376

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	376

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	377

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	377

6.2.2A.3	Requirements for 2-step RA type	377

6.2.2A.3.1	Contention based random access	378

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	378

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	378

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	379

6.2.2A.3.2	Non-Contention based random access	379

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	379

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	379

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	379

6.2.2B	Random access for RedCap	380

6.2.2B.1	Introduction	380

6.2.2B.2	Requirements	380

6.2.3	SA: RRC Connection Release with Redirection	380

6.2.3.1	Introduction	380

6.2.3.2	Requirements	381

6.2.3.2.1	RRC connection release with redirection to NR	381

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	381

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	382

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	383

6.2.3A.1	Introduction	383

6.2.3A.2	Requirements	383

6.2.3A.2.1	RRC connection release with redirection to NR	383

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	384

6.2C	RRC Connection Mobility Control for Satellite Access	384

6.2C.1	SA: RRC Re-establishment for Satellite Access	384

6.2C.1.1	Introduction	384

6.2C.1.2	Requirements	384

6.2C.1.2.1	UE Re-establishment delay requirement	384

6.2C.2	Random access for satellite access	385

6.2C.2.1	Introduction	385

6.2C.2.2	Requirements for 4-step RA type	386

6.2C.2.2.1	Contention based random access	386

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	386

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	386

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	386

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	386

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	386

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	387

6.2C.2.2.2	Non-Contention based random access	387

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	387

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	387

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	388

6.2C.2.3	Requirements for 2-step RA type	388

6.2C.2.3.1	Contention based random access	388

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	388

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	388

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	389

6.2C.2.3.2	Non-Contention based random access	389

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	389

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	389

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	389

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	389

6.2C.3.1	Introduction	389

6.2C.3.2	Requirements	390

6.2C.3.2.1	RRC connection release with redirection to NR	390

6.2D	RRC Connection Mobility Control for ATG	390

6.2D.1	SA: RRC Re-establishment	390

6.2D.1.1	Introduction	390

6.2D.1.2	Requirements	391

6.2D.1.2.1	UE Re-establishment delay requirement	391

6.2D.2	Random access	392

6.2D.2.1	Introduction	392

6.2D.2.2	Requirements for 4-step RA type	392

6.2D.2.3	Requirements for 2-step RA type	392

6.2D.3	SA: RRC Connection Release with Redirection	393

6.2D.3.1	Introduction	393

6.2D.3.2	Requirements	393

6.2D.3.2.1	RRC connection release with redirection to NR	393

6.3	L1/L2-Triggered Mobility	394

6.3.1	PCell Cell Switch	394

6.3.1.1	Introduction	394

6.3.1.2	LTM Cell Switch delay	394

6.3.1.3	Interruption time	395

7	Timing	396

7.1	UE transmit timing	396

7.1.1	Introduction	396

7.1.2	Requirements	397

7.1.2.1	Gradual timing adjustment	399

7.1.2.2	Void	399

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	399

7.1.2.4	UE transmit timing for positioning measurements	400

7.1A	UE transmit timing for RedCap	400

7.1A.1	Introduction	400

7.1A.2	Requirements	401

7.1A.2.1	Gradual timing adjustment	402

7.1C	UE transmit timing for Satellite Access	402

7.1C.1	Introduction	402

7.1C.2	Requirements	402

7.1C.2.1	Gradual timing adjustment	403

7.1D	UE transmit timing for ATG	404

7.1D.1	Introduction	404

7.1D.2	Requirements	404

7.1D.2.1	Gradual timing adjustment	404

7.2	UE timer accuracy	405

7.2.1	Introduction	405

7.2.2	Requirements	405

7.2A	UE timer accuracy for RedCap	405

7.2A.1	Introduction	405

7.2A.2	Requirements	406

7.2C	UE timer accuracy for satellite access	406

7.2C.1	Introduction	406

7.2C.2	Requirements	406

7.2D	UE timer accuracy for ATG	406

7.2D.1	Introduction	406

7.2D.2	Requirements	406

7.3	Timing advance	407

7.3.1	Introduction	407

7.3.2	Requirements	407

7.3.2.1	Timing Advance adjustment delay	407

7.3.2.2	Timing Advance adjustment accuracy	407

7.3A	Timing Advance for RedCap	407

7.3A.1	Introduction	407

7.3A.2	Requirements	407

7.3A.2.1	Timing Advance adjustment delay	407

7.3A.2.2	Timing Advance adjustment accuracy	408

7.3C	Timing advance for satellite access	408

7.3C.1	Introduction	408

7.3C.2	Requirements	408

7.3C.2.1	Timing Advance adjustment delay	408

7.3C.2.2	Timing Advance adjustment accuracy	408

7.3D	Timing advance for ATG	409

7.3D.1	Introduction	409

7.3D.2	Requirements	409

7.3D.2.1	Timing Advance adjustment delay	409

7.3D.2.2	Timing Advance adjustment accuracy	409

7.4	Cell phase synchronization accuracy	409

7.4.1	Definition	409

7.4.2	Minimum requirements	409

7.5	Maximum Transmission Timing Difference	410

7.5.1	Introduction	410

7.5.2	Minimum Requirements for inter-band EN-DC	410

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	410

7.5.3	Minimum Requirements for intra-band EN-DC	411

7.5.4	Minimum Requirements for NR Carrier Aggregation	411

7.5.5	Minimum Requirements for inter-band NE-DC	412

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	412

7.5.6	Minimum Requirements for inter-band NR DC	413

7.5.7	Minimum Requirements for multi-TRP	413

7.6	Maximum Receive Timing Difference	414

7.6.1	Introduction	414

7.6.2	Minimum Requirements for inter-band EN-DC	414

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	415

7.6.3	Minimum Requirements for intra-band EN-DC	415

7.6.4	Minimum Requirements for NR Carrier Aggregation	416

7.6.5	Minimum Requirements for inter-band NE-DC	417

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	417

7.6.6	Minimum Requirements for inter-band NR DC	418

7.6.7	Minimum Requirements for PC6 UE in FR2	419

7.6.8	Minimum Requirements for Multi-TRPs	419

7.7	deriveSSB-IndexFromCell tolerance	419

7.7.1	Minimum requirements	419

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	420

7.7A.1	Minimum requirements	420

7.7D	DeriveSSB-IndexFromCell tolerance for ATG	420

7.7D.1	Minimum requirements	420

7.8	Void	420

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	420

7.9.1	Minimum requirements	420

7.9D	DeriveSSB-IndexFromCellInter-r17 tolerance for ATG	420

7.9D.1	Minimum requirements	420

8	Signalling characteristics	429

8.1	Radio Link Monitoring	429

8.1.1	Introduction	429

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	430

8.1.2	Requirements for SSB based radio link monitoring	431

8.1.2.1	Introduction	431

8.1.2.2	Minimum requirement	432

8.1.2.3	Measurement restrictions for SSB based RLM	436

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	437

8.1.3	Requirements for CSI-RS based radio link monitoring	437

8.1.3.1	Introduction	437

8.1.3.2	Minimum requirement	438

8.1.3.3	Measurement restrictions for CSI-RS based RLM	442

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	443

8.1.4	Minimum requirement at transitions	444

8.1.5	Minimum requirement for UE turning off the transmitter	444

8.1.6	Minimum requirement for L1 indication	444

8.1.7	Scheduling availability of UE during radio link monitoring	445

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	445

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	445

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	445

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	446

8.1.8	Minimum requirement under IDC Interference	447

8.1A	Radio Link Monitoring with CCA on Target Frequency	447

8.1A.1	Introduction	447

8.1A.2	Requirements for SSB Based Radio Link Monitoring	448

8.1A.2.1	Introduction	448

8.1A.2.2	Minimum Requirement	448

8.1A.3	Minimum requirement at transitions	452

8.1A.4	Minimum requirement for UE turning off the transmitter	452

8.1A.5	Minimum requirement for L1 indication	452

8.1A.6	Scheduling availability of UE during radio link monitoring	452

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	453

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	454

8.1B	Radio Link Monitoring for RedCap	454

8.1B.1	Introduction	454

8.1B.2	Requirements for SSB based radio link monitoring	455

8.1B.2.1	Introduction	455

8.1B.2.2	Minimum requirement	456

8.1B.2.3	Measurement restrictions for SSB based RLM	458

8.1B.3	Requirements for CSI-RS based radio link monitoring	458

8.1B.3.1	Introduction	458

8.1B.3.2	Minimum requirement	459

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	462

8.1B.4	Minimum requirement at transitions	462

8.1B.5	Minimum requirement for UE turning off the transmitter	463

8.1B.6	Minimum requirement for L1 indication	463

8.1B.7	Scheduling availability of UE during radio link monitoring	463

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	463

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	463

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	464

8.1C	Radio Link Monitoring for Satellite Access	464

8.1C.1	Introduction	464

8.1C.2	Requirements for SSB based radio link monitoring	465

8.1C.2.1	Introduction	465

8.1C.2.2	Minimum requirement	466

8.1C.2.3	Measurement restrictions for SSB based RLM	467

8.1C.3	Requirements for CSI-RS based radio link monitoring	467

8.1C.3.1	Introduction	467

8.1C.3.2	Minimum requirement	468

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	469

8.1C.4	Minimum requirement at transitions	469

8.1C.5	Minimum requirement for UE turning off the transmitter	470

8.1C.6	Minimum requirement for L1 indication	470

8.1C.7	Scheduling availability of UE during radio link monitoring	470

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	470

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	470

8.1D	Radio Link Monitoring for ATG	471

8.1D.1	Introduction	471

8.1D.2	Requirements for SSB based radio link monitoring	471

8.1D.2.1	Introduction	471

8.1D.2.2	Minimum requirement	472

8.1D.2.3	Measurement restrictions for SSB based RLM	473

8.1D.3	Requirements for CSI-RS based radio link monitoring	474

8.1D.3.1	Introduction	474

8.1D.3.2	Minimum requirement	474

8.1D.3.3	Measurement restrictions for CSI-RS based RLM	475

8.1D.4	Minimum requirement at transitions	475

8.1D.5	Minimum requirement for UE turning off the transmitter	476

8.1D.6	Minimum requirement for L1 indication	476

8.1D.7	Scheduling availability of UE during radio link monitoring	476

8.1D.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	476

8.1D.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	476

8.2	Interruption	476

8.2.1	EN-DC Interruption	476

8.2.1.1	Introduction	476

8.2.1.2	Requirements	477

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	477

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	477

8.2.1.2.3	Interruptions at SCell addition/release	478

8.2.1.2.4	Interruptions at SCell activation/deactivation	479

8.2.1.2.5	Interruptions during measurements on SCC	481

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	481

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	481

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	482

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	482

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	482

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	483

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	484

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	484

8.2.1.2.8.2	Interruptions during direct SCell activation	484

8.2.1.2.9	Interruptions at SCell hibernation	485

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	485

8.2.1.2.11	Interruptions due to UE-specific CBW change	485

8.2.1.2.12	Interruptions at NR SRS carrier based switching	485

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	487

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	488

8.2.1.2.15	Interruptions due to SCell dormancy	488

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	488

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	488

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	489

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	489

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	489

8.2.1.2.18	Interruptions at NR SRS antenna port switching	490

8.2.1.2.19	Interruptions at fast SCell activation	491

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	492

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	492

8.2.2.1	Introduction	492

8.2.2.2	Requirements	493

8.2.2.2.1	Interruptions at SCell addition/release	493

8.2.2.2.2	Interruptions at SCell activation/deactivation	494

8.2.2.2.3	Interruptions during measurements on deactivated SCC	495

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	496

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	497

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	498

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	499

8.2.2.2.8	Interruptions due to UE-specific CBW change	499

8.2.2.2.9	Interruptions at NR SRS carrier based switching	499

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	501

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	502

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	502

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	503

8.2.2.2.10D	DL Interruptions at UE switching across three or four uplink bands	503

8.2.2.2.11	Interruptions at direct SCell activation	504

8.2.2.2.12	Interruptions due to SCell dormancy	504

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	504

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	504

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	505

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	505

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	505

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	505

8.2.2.2.16	Interruptions at NR SRS antenna port switching	506

8.2.2.2.17	Interruptions at fast SCell activation	507

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	508

8.2.2.2.19	Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]	508

8.2.2.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	509

8.2.3	NE-DC Interruptions	509

8.2.3.1	Introduction	509

8.2.3.2	Requirements	510

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	510

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	510

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	511

8.2.3.2.4	Interruptions at SCell activation/deactivation	512

8.2.3.2.5	Interruptions during measurements on SCC	513

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	513

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	513

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	514

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	514

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	514

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	515

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	515

8.2.3.2.9	Interruptions at SCell hibernation	515

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	516

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	516

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	517

8.2.3.2.13	Interruptions due to SCell dormancy	518

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	519

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	519

8.2.3.2.17	Interruptions at fast SCell activation	521

8.2.3.2.18	Interruptions due to UE-specific CBW change	521

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	522

8.2.4	NR-DC: Interruptions	522

8.2.4.1	Introduction	522

8.2.4.2	Requirements	523

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	523

8.2.4.2.2	Interruptions at SCell activation/deactivation	524

8.2.4.2.3	Interruptions during measurements on SCC	525

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	525

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	526

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	526

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	526

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	527

8.2.4.2.9	Interruptions at NR SRS carrier based switching	527

8.2.4.2.10	Interruptions at direct SCell activation	528

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	529

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	529

8.2.4.2.13	 Interruptions due to SCell dormancy	530

8.2.4.2.14	Interruptions at NR SRS antenna port switching	530

8.2.4.2.15	Interruptions at fast SCell activation	531

8.2.4.2.16	Interruptions at SCG activation/deactivation	532

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	532

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	533

8.2.4.2.19	Interruptions due to UE-specific CBW change	533

8.2.4.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	533

8.2.4.2A	Void	534

8.2.4.2A.1	Void	534

8.2.4.2A.2	Void	534

8.2.4.2A.3	Void	534

8.3	SCell Activation and Deactivation Delay	534

8.3.1	Introduction	534

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	534

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	540

8.3.4	Direct SCell Activation at SCell addition	540

8.3.5	Direct SCell Activation at Handover	542

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	544

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	548

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	548

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	549

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	551

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	553

8.3.13.1	Introduction	553

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	556

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	556

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	556

8.3.17	SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.	559

8.3.18	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting	562

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	565

8.3A.1	Introduction	565

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	565

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	569

8.4	UE UL carrier RRC reconfiguration delay	570

8.4.1	Introduction	570

8.4.2	UE UL carrier configuration delay requirement	570

8.4.3	UE UL carrier deconfiguration delay requirement	570

8.5	Link Recovery Procedures	570

8.5.1	Introduction	570

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	572

8.5.2	Requirements for SSB based beam failure detection	572

8.5.2.1	Introduction	572

8.5.2.2	Minimum requirement	573

8.5.2.3	Measurement restriction for SSB based beam failure detection	577

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	577

8.5.3	Requirements for CSI-RS based beam failure detection	578

8.5.3.1	Introduction	578

8.5.3.2	Minimum requirement	579

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	582

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	584

8.5.4	Minimum requirement for L1 indication	585

8.5.5	Requirements for SSB based candidate beam detection	585

8.5.5.1	Introduction	585

8.5.5.2	Minimum requirement	585

8.5.5.3	Measurement restriction for SSB based candidate beam detection	588

8.5.6	Requirements for CSI-RS based candidate beam detection	589

8.5.6.1	Introduction	589

8.5.6.2	Minimum requirement	589

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	593

8.5.7	Scheduling availability of UE during beam failure detection	594

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	594

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	594

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	594

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	595

8.5.8	Scheduling availability of UE during candidate beam detection	596

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	596

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	596

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	596

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	597

8.5.9	Requirements for Beam Failure Recovery in SCell	597

8.5.9.1	Introduction	597

8.5.9.2	Requirement	597

8.5.10	Minimum requirement at transitions for beam failure detection	597

8.5.11	Minimum requirement under IDC Interference	598

8.5A	Link Recovery Procedures when CCA is used on target frequency	598

8.5A.1	Introduction	598

8.5A.2	Requirements for SSB based beam failure detection	599

8.5A.2.1	Introduction	599

8.5A.2.2	Minimum requirement	599

8.5A.2.3	Measurement restriction for SSB based beam failure detection	601

8.5A.3	Void	602

8.5A.4	Minimum requirement for L1 indication	602

8.5A.5	Requirements for SSB based candidate beam detection	602

8.5A.5.1	Introduction	602

8.5A.5.2	Minimum requirement	602

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	605

8.5A.6	Void	605

8.5A.7	Scheduling availability of UE during beam failure detection	605

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	605

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	606

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	606

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	606

8.5A.8	Scheduling availability of UE during candidate beam detection	606

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	606

8.5.8A.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	606

8.5B	Link Recovery Procedures for Redcap	606

8.5B.1	Introduction	606

8.5B.2	Requirements for SSB based beam failure detection for Redcap	607

8.5B.2.1	Introduction	607

8.5B.2.2	Minimum requirement	607

8.5B.2.3	Measurement restriction for SSB based beam failure detection	609

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	609

8.5B.3.1	Introduction	609

8.5B.3.2	Minimum requirement	610

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	612

8.5B.4	Minimum requirement for L1 indication for Redcap	613

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	613

8.5B.5.1	Introduction	613

8.5B.5.2	Minimum requirement	613

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	615

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	615

8.5B.6.1	Introduction	615

8.5B.6.2	Minimum requirement	616

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	618

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	618

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	618

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	618

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	618

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	619

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	619

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	619

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	619

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	620

8.5C	Link Recovery Procedures for Satellite Access	620

8.5C.1	Introduction	620

8.5C.2	Requirements for SSB based beam failure detection	621

8.5C.2.1	Introduction	621

8.5C.2.2	Minimum requirement	621

8.5C.2.3	Measurement restriction for SSB based beam failure detection	622

8.5C.3	Requirements for CSI-RS based beam failure detection	622

8.5C.3.1	Introduction	622

8.5C.3.2	Minimum requirement	623

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	624

8.5C.4	Minimum requirement for L1 indication	624

8.5C.5	Requirements for SSB based candidate beam detection	625

8.5C.5.1	Introduction	625

8.5C.5.2	Minimum requirement	625

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	626

8.5C.6	Requirements for CSI-RS based candidate beam detection	626

8.5C.6.1	Introduction	626

8.5C.6.2	Minimum requirement	626

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	627

8.5C.7	Scheduling availability of UE during beam failure detection	628

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	628

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	628

8.5C.8	Scheduling availability of UE during candidate beam detection	628

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	628

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	628

8.5C.9	Minimum requirement at transitions for beam failure detection	629

8.5D	Link Recovery Procedures for ATG	629

8.5D.1	Introduction	629

8.5D.2	Requirements for SSB based beam failure detection	630

8.5D.2.1	Introduction	630

8.5D.2.2	Minimum requirement	630

8.5D.2.3	Measurement restriction for SSB based beam failure detection	631

8.5D.3	Requirements for CSI-RS based beam failure detection	631

8.5D.3.1	Introduction	631

8.5D.3.2	Minimum requirement	631

8.5D.3.3	Measurement restrictions for CSI-RS beam failure detection	632

8.5D.4	Minimum requirement for L1 indication	633

8.5D.5	Requirements for SSB based candidate beam detection	633

8.5D.5.1	Introduction	633

8.5D.5.2	Minimum requirement	633

8.5D.5.3	Measurement restriction for SSB based candidate beam detection	634

8.5D.6	Requirements for CSI-RS based candidate beam detection	634

8.5D.6.1	Introduction	634

8.5D.6.2	Minimum requirement	634

8.5D.6.3	Measurement restriction for CSI-RS based candidate beam detection	635

8.5D.7	Scheduling availability of UE during beam failure detection	636

8.5D.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	636

8.5D.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	636

8.5D.8	Scheduling availability of UE during candidate beam detection	636

8.5D.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	636

8.5D.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	636

8.5D.9	Minimum requirement at transitions for beam failure detection	637

8.6	Active BWP switch delay	637

8.6.1	Introduction	637

8.6.2	DCI and timer based BWP switch delay on a single CC	637

8.6.2A	DCI based BWP switch delay on multiple CCs	639

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	639

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	640

8.6.2B	Timer based BWP switch delay on multiple CCs	640

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	640

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	641

8.6.3	RRC based BWP switch delay on a single CC	642

8.6.3A	RRC based BWP switch delay on multiple CCs	642

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	642

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	643

8.6.4	BWP switch delay on Consistent UL CCA recovery	643

8.6A	Active BWP switch delay for RedCap	644

8.6A.1	Introduction	644

8.6A.2	DCI and timer based BWP switch delay on a single CC	644

8.6A.3	RRC based BWP switch delay on a single CC	645

8.6C	Active BWP switch delay for satellite access	646

8.6C.1	Introduction	646

8.6C.2	DCI and timer based BWP switch delay on a single CC	646

8.6C.3	RRC based BWP switch delay on a single CC	647

8.6D	Active BWP switch delay for ATG	648

8.6D.1	Introduction	648

8.6D.2	DCI and timer based BWP switch delay	648

8.6D.3	RRC based BWP switch delay on a single CC	649

8.7	Void	650

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	650

8.8.1	Introduction	650

8.8.2	E-UTRAN PSCell Addition Delay Requirement	650

8.8.3	E-UTRAN PSCell Release Delay Requirement	650

8.9	NR-DC: PSCell Addition and Release Delay	651

8.9.1	Introduction	651

8.9.2	PSCell Addition Delay Requirement	651

8.9.3	PSCell Release Delay Requirement	651

8.9A	Conditional PSCell Addition Delay	652

8.9A.1	Introduction	652

8.9A.2	Conditional PSCell Addition Delay Requirement	652

8.9A.2.1	Measurement time	652

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	653

8.9B.1	Introduction	653

8.9B.2	PSCell Addition Delay Requirement	653

8.9B.3	PSCell Release Delay Requirement	654

8.9C	Subsequent Conditional PSCell Addition Delay	654

8.9C.1	Introduction	654

8.9C.2	Subsequent Conditional PSCell Addition Delay Requirement	654

8.9C.2.1	Measurement time	655

8.10	Active TCI state switching delay	655

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	656

8.10.4	DCI based TCI state switch delay	657

8.10.5	RRC based TCI state switch delay	657

8.10.6	Active TCI state list update delay	658

8.10A	Active TCI state switching delay with CCA	658

8.10A.1	Introduction	658

8.10A.2	Known conditions for TCI state	658

8.10A.3	MAC-CE based TCI state switch delay	658

8.10A.4	DCI based TCI state switch delay	659

8.10A.5	RRC based TCI state switch delay	660

8.10A.6	Active TCI state list update delay	661

8.10B	Active TCI state switching delay for RedCap	661

8.10B.1	Introduction	661

8.10B.2	Known conditions for TCI state	661

8.10B.3	MAC-CE based TCI state switch delay	661

8.10B.4	DCI based TCI state switch delay	662

8.10B.5	RRC based TCI state switch delay	662

8.10B.6	Active TCI state list update delay	663

8.10C	Active TCI state switching delay for satellite access	663

8.10C.1	Introduction	663

8.10C.2	MAC-CE based TCI state switch delay	663

8.10C.4	DCI based TCI state switch delay	663

8.10C.5	RRC based TCI state switch delay	664

8.10C.6	Active TCI state list update delay	664

8.10D	Active TCI state switching delay for ATG	664

8.10D.2	Void	664

8.10D.6	Active TCI state list update delay	665

8.10E	Active TCI state switching delay for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	665

8.10E.1	Introduction	665

8.10E.2	Known conditions for TCI state	665

8.10E.3	MAC-CE based dual DL TCI state switch delay	666

8.10E.3.1	MAC-CE based dual DL TCI state switching delay for sDCI	666

8.10E.3.2	MAC-CE based dual DL TCI state switching delay for mDCI	666

8.10E.4	DCI based dual DL TCI state switch delay for sDCI and mDCI	666

8.10E.5	RRC based dual DL TCI state switch delay	667

8.10E.6	Active DL TCI state list update delay	667

8.11	PSCell Change	667

8.11A	PSCell Change in Carriers with CCA	668

8.11B	Conditional PSCell Change	668

8.11B.1	Introduction	668

8.11B.2	Conditoinal PSCell Change delay	668

8.11B.2.1	Measurement time	669

8.11D	Conditional PSCell Change in Carriers with CCA	669

8.11D.1	Introduction	669

8.11D.2	Conditional PSCell Change delay	669

8.11D.2.1	Measurement time	670

8.11E	Subsequent Conditional PSCell Change	671

8.11E.1	Introduction	671

8.11E.2	Subsequent Conditoinal PSCell Change delay	671

8.11E.2.1	Measurement time	671

8.12	Uplink spatial relation switch delay	672

8.12.1	Introduction	672

8.12.2	Known conditions for spatial relation when associated with DL-RS	672

8.12.3	MAC-CE based spatial relation switch delay	672

8.12.4	DCI based spatial relation switch delay	673

8.12.5	RRC based spatial relation switch delay	673

8.12A	Uplink spatial relation switch delay for RedCap	674

8.12A.1	Introduction	674

8.12A.2	Known conditions for spatial relation when associated with DL-RS	674

8.12A.3	MAC-CE based spatial relation switch delay	674

8.12A.4	DCI based spatial relation switch delay	675

8.12A.5	RRC based spatial relation switch delay	675

8.12C	Uplink spatial relation switch delay for satellite access	676

8.12C.1	Void	676

8.12C.2	Void	676

8.12C.3	Void	676

8.12C.4	Void	676

8.12C.5	Void	676

8.13	UE-specific CBW change	676

8.13.1	Introduction	676

8.13.2	UE-specific CBW change delay	676

8.13A	UE-specific CBW change for RedCap	677

8.13A.1	Introduction	677

8.13A.2	UE-specific CBW change delay	677

8.13C	UE-specific CBW change for satellite access	677

8.13C.1	Introduction	677

8.13C.2	UE-specific CBW change delay	678

8.13D	UE-specific CBW change for ATG	678

8.13D.1	Introduction	678

8.13D.2	UE-specific CBW change delay	678

8.14	Pathloss reference signal switching delay	678

8.14.1	Introduction	678

8.14.2	Known conditions for pathloss reference signal	678

8.14.3	MAC-CE based pathloss reference signal switch delay	679

8.14C	Pathloss reference signal switching delay for satellite access	679

8.14C.1	Introduction	680

8.14C.2	Known conditions for pathloss reference signal	680

8.14C.3	MAC-CE based pathloss reference signal switch delay	680

8.14D	Pathloss reference signal switching delay for ATG	681

8.14D.1	Introduction	681

8.14D.2	Known conditions for pathloss reference signal	681

8.14D.3	MAC-CE based pathloss reference signal switch delay	681

8.15	Active downlink TCI state switching delay for unified TCI	681

8.15.1	Introduction	681

8.15.4	DCI based downlink TCI state switch delay	683

8.15.5	Active Downlink TCI state list update delay	683

8.15D	Active downlink TCI state switching delay for unified TCI for ATG	684

8.15D.1	Introduction	684

8.15D.2	Void	684

8.15D.4	DCI based downlink TCI state switch delay	685

8.15D.5	Active Downlink TCI state list update delay	685

8.16	Active uplink TCI state switching delay for unified TCI	685

8.16.1	Introduction	685

8.16.4	DCI based uplink TCI state switch delay	688

8.16.5	Active Uplink TCI state list update delay	688

8.16D	Active uplink TCI state switching delay for unified TCI for ATG	690

8.16D.1	Introduction	690

8.16D.2	Void	690

8.16D.3	MAC-CE based uplink TCI state switch delay	690

8.16D.4	DCI based uplink TCI state switch delay	691

8.16D.5	Active Uplink TCI state list update delay	691

8.17	SCG Activation and Deactivation Delay	692

8.17.1	Introduction	692

8.17.2	SCG Activation Delay Requirement	692

8.17.3	SCG Deactivation Delay Requirement	693

8.18	TRP specific Link Recovery Procedures	693

8.18.1	Introduction	693

8.18.2	Requirements for TRP specific SSB based beam failure detection	694

8.18.2.1	Introduction	694

8.18.2.2	Minimum requirement	694

8.18.2.3	Measurement restriction for SSB based beam failure detection	696

8.18.3	Requirements for CSI-RS based beam failure detection	697

8.18.3.1	Introduction	697

8.18.3.2	Minimum requirement	697

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	700

8.18.4	Minimum requirement for L1 indication	701

8.18.5	Requirements for SSB based candidate beam detection	701

8.18.5.1	Introduction	701

8.18.5.2	Minimum requirement	701

8.18.5.3	Measurement restriction for SSB based candidate beam detection	703

8.18.6	Requirements for CSI-RS based candidate beam detection	704

8.18.6.1	Introduction	704

8.18.6.2	Minimum requirement	704

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	706

8.18.7	Requirements for TRP specific Beam Failure Recovery	707

8.18.7.1	Introduction	707

8.18.7.2	Requirement	708

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	708

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	708

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	708

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	708

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	709

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	709

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	710

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	710

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	710

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	710

8.19	Pre-configured measurement gap activation/deactivation delay	711

8.19.1	Introduction	711

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	711

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	711

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	711

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	711

8.19.5	Activation/deactivation delay requirements for multiple pre-configured measurement gaps	712

8.19.5,1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of multiple pre-configured measurement gaps	712

8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of multiple pre-configured measurement gaps	712

8.19.5.3	Pre-configured measurement gap activation/deactivation delay when colliding with a measurement gap	712

8.19D	Pre-configured measurement gap activation/deactivation delay for ATG	712

8.19D.1	Introduction	712

8.19D.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	713

8.19D.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	713

8.19D.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	713

8.20	PSCell Cell Switch	713

8.20.1	Introduction	713

8.20.2	LTM Cell Switch delay	714

8.20.3	Interruption time	714

8.21	Active downlink TCI state switching delay for unified TCI for single-DCI mTRP	714

8.21.1	Introduction	714

8.21.2	Known conditions for downlink TCI state	714

8.21.3	MAC-CE based downlink TCI state switch delay	715

8.21.4	DCI based downlink TCI state switch delay	716

8.21.5	Active Downlink TCI state list update delay	716

8.22	Active downlink TCI state switching delay for unified TCI for multi-DCI mTRP	717

8.22.1	Introduction	717

8.22.2	Known conditions for downlink TCI state	718

8.22.3	MAC-CE based downlink TCI state switch delay	718

8.22.4	DCI based downlink TCI state switch delay	719

8.22.5	Active Downlink TCI state list update delay	719

8.23	Active uplink TCI state switching delay for unified TCI for single-DCI mTRP	720

8.23.1	Introduction	720

8.23.2	Known conditions for uplink TCI state	720

8.23.3	MAC-CE based uplink TCI state switch delay	721

8.23.4	DCI based uplink TCI state switch delay	722

8.23.5	Active uplink TCI state list update delay	722

8.24	Active uplink TCI state switching delay for unified TCI for multi-DCI mTRP	723

8.24.1	Introduction	723

8.24.2	Known conditions for uplink TCI state	724

8.24.3	MAC-CE based uplink TCI state switch delay	724

8.24.4	DCI based uplink TCI state switch delay	726

8.24.5	Active Uplink TCI state list update delay	726

9	Measurement Procedure	733

9.1	General measurement requirement	733

9.1.1	Introduction	733

9.1.2	Measurement gap	733

9.1.2.1	EN-DC: Measurement Gap Sharing	745

9.1.2.1a	SA: Measurement Gap Sharing	746

9.1.2.1b	NE-DC: Measurement Gap Sharing	747

9.1.2.1c	NR-DC: Measurement Gap Sharing	748

9.1.3	UE Measurement capability	749

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	749

9.1.3.1a	SA: Monitoring of multiple layers using gaps	749

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	750

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	750

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	751

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	752

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	752

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	753

9.1A.3.2	Void	754

9.1.3A	UE Measurement capability under operation mode with CCA	754

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	754

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	754

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	754

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	755

9.1.3C	UE Measurement capability under operation mode with satellite access	755

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	755

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	756

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	756

9.1.4.1	Introduction	756

9.1.4.2	Requirements	756

9.1.5	Carrier-specific scaling factor	760

9.1.5.1	Monitoring of multiple layers outside gaps	761

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	764

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	765

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	766

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	767

9.1.5.2	Monitoring of multiple layers within gaps	768

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	771

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	772

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	775

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	777

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.3	Monitoring of multiple layers within NCSG	779

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	780

9.1.5.4	L1-RSRP measurements within measurement gap	781

9.1.5.4.1	SA mode: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	783

9.1.5.4.2	NR-DC: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	784

9.1.6	Minimum requirement at transitions	786

9.1.7	Pre-configured measurement gap	786

9.1.7.1	Introduction	786

9.1.7.2	Requirements applicability	786

9.1.7.3	Requirements	787

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	787

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	788

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	788

9.1.8	Concurrent measurement gaps	788

9.1.8.1	Introduction	788

9.1.8.2	Requirements	789

9.1.8.3	Collision between concurrent measurement gaps	790

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	790

9.1.9	Network controlled small gap	791

9.1.9.1	Introduction	791

9.1.9.2	Requirements applicability	792

9.1.10	MUSIM gaps	794

9.1.10.1	Introduction	795

9.1.10.2	Priorities for MUSIM gaps	796

9.1.10.3	Keep solution for MUSIM gaps	796

9.1.10.4	Collisions between different MUSIM gaps	796

9.1.10.5	Collisions between MUSIM gaps and measurement gaps	796

9.1.10.6	Measurement gap related requirements of MUSIM gaps	797

9.1.11	UL gap for Tx power management	797

9.1.12	Concurrent measurement gaps with Pre-MG	797

9.1.12.1	Introduction	797

9.1.12.2	Requirements	797

9.1.12.3	Collisions involving Pre-MG(s)	798

9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap	799

9.1.13	Concurrent measurement gaps with NCSG	799

9.1.13.1	Introduction	799

9.1.13.2	Requirements	799

9.1.13.3	Collision involving NCSGs	801

9.1A	General measurement requirement for RedCap	801

9.1A.1	Introduction	801

9.1A.2	Measurement gap	801

9.1A.2.1	SA: Measurement Gap Sharing	805

9.1A.3	UE Measurement capability	806

9.1A.3.1	SA: Monitoring of multiple layers using gaps	806

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	806

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	806

9.1A.4.1	Introduction	806

9.1A.4.2	Requirements	807

9.1A.5	Carrier-specific scaling factor	807

9.1A.5.1	Monitoring of multiple layers outside gaps	807

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	808

9.1A.5.2	Monitoring of multiple layers within gaps	808

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	808

9.1A.5.2.2	SA mode: carrier-specific scaling factor for PRS measurements performed within gaps	809

9.1A.6	Minimum requirement at transitions	809

9.1C	General measurement requirement for SAN	810

9.1C.1	Introduction	810

9.1C.2	Measurement gap	810

9.1C.8	Concurrent measurement gaps for SAN	812

9.1C.8.1	Introduction	812

9.1C.8.2	Requirements	812

9.1C.8.3	Collision between concurrent measurement gaps	812

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	813

9.1C.9	Collision between SMTC and measurement gap for SAN	813

9.1C.9.1	Introduction	813

9.1C.9.2	Collision between SMTCs and measurement gap	813

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	814

9.1D	General measurement requirement for ATG	814

9.1D.1	Introduction	814

9.1D.2	Measurement gap	814

9.1D.2.1a	SA: Measurement Gap Sharing	816

9.1D.3	UE Measurement capability	817

9.1D.3.1	SA: Monitoring of multiple layers using gaps	817

9.1D.3.2	SA: Maximum allowed layers for multiple monitoring	817

9.1D.4	Void	817

9.1D.5	Carrier-specific scaling factor	817

9.1D.5.1	Monitoring of multiple layers outside gaps	817

9.1D.5.1.1	Void	818

9.1D.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements performed outside gaps	818

9.1D.5.2	Monitoring of multiple layers within gaps	818

9.1D.5.2.1	Void	819

9.1D.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements performed within gaps	819

9.1D.6	Void	820

9.1D.7	Pre-configured measurement gap	820

9.1D.7.1	Introduction	820

9.1D.7.2	Requirements applicability	820

9.1D.7.3	Requirements	821

9.1D.7.3.1	Requirements for autonomous activation/deactivation mechanism	821

9.1D.7.3.2	Requirements for network-controlled activation/deactivation mechanism	821

9.1D.7.3.3	Requirements for reception/transmission during activation/deactivation	822

9.1D.8	Capabilities for Support of Event Triggering and Reporting Criteria	822

9.1D.8.1	Introduction	822

9.1D.8.2	Requirements	822

9.1D.9	Minimum requirement at transitions	823

9.2	NR intra-frequency measurements	823

9.2.1	Introduction	823

9.2.2	Requirements applicability	825

9.2.3	Number of cells and number of SSB	826

9.2.3.1	Requirements for FR1	826

9.2.3.2	Requirements for FR2	826

9.2.4	Measurement Reporting Requirements	826

9.2.4.1	Periodic Reporting	826

9.2.4.2	Event-triggered Periodic Reporting	826

9.2.4.3	Event Triggered Reporting	827

9.2.4.4	SCell activation Triggered Reporting	827

9.2.5	Intrafrequency measurements without measurement gaps	828

9.2.5.1	Intrafrequency cell identification	828

9.2.5.2	Measurement period	836

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	840

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	841

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	841

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	842

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	843

9.2.5.4	SFTD Measurements between PCell and PSCell	843

9.2.5.4.1	Introduction	843

9.2.5.4.2	SFTD Measurement delay	844

9.2.5.4.3	SFTD Measurement Reporting Delay	844

9.2.6	Intra-frequency measurements with measurement gaps	844

9.2.6.1	Void	844

9.2.6.2	Intra-frequency cell identification	844

9.2.6.3	Intrafrequency Measurement Period	848

9.2.7	Intra-frequency measurements with NCSG	849

9.2.7.1	Intra-frequency cell identification	849

9.2.7.2	Measurement period	851

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	853

9.2A	NR intra-frequency measurements with CCA	853

9.2A.1	Introduction	853

9.2A.2	Requirements applicability	854

9.2A.3	Number of cells and number of SSB	854

9.2A.3.1	Requirements for FR1	854

9.2A.3.2	Requirements for FR2-2	854

9.2A.4	Measurement Reporting Requirements	855

9.2A.5	Intra-frequency measurements without measurement gaps	855

9.2A.5.2	Measurement period	860

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	862

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	863

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	863

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	863

9.2A.6	Intra-frequency measurements with measurement gaps	863

9.2A.6.1	Intra-frequency cell identification	863

9.2A.6.2	Intra-frequency Measurement Period	866

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	867

9.2A.7.1	Intra-frequency RSSI measurements	867

9.2A.7.2	Intra-frequency Channel occupancy measurements	869

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	870

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	870

9.2B	NR intra-frequency measurements for RedCap	871

9.2B.1	Introduction	871

9.2B.2	Requirements applicability	871

9.2B.3	Number of cells and number of SSB	872

9.2B.3.1	Requirements for FR1	872

9.2B.3.2	Requirements for FR2	872

9.2B.4	Measurement Reporting Requirements	872

9.2B.4.1	Periodic Reporting	872

9.2B.4.2	Event-triggered Periodic Reporting	872

9.2B.4.3	Event Triggered Reporting	872

9.2B.5	Intra-frequency measurements without measurement gaps	873

9.2B.5.1	Intra-frequency cell identification	873

9.2B.5.2	Measurement period	875

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	876

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	876

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	876

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	877

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	877

9.2B.6	Intra-frequency measurements with measurement gaps	878

9.2B.6.1	Intra-frequency cell identification	878

9.2B.6.2	Intra-frequency Measurement Period	879

9.2C	NR intra-frequency measurements for SAN	880

9.2C.1	Introduction	880

9.2C.2	Requirements applicability	881

9.2C.3	Number of cells and number of SSB	881

9.2C.3.1	Requirements for FR1	881

9.2C.4	Measurement Reporting Requirements	881

9.2C.4.1	Periodic Reporting	881

9.2C.4.2	Event-triggered Periodic Reporting	882

9.2C.4.3	Event Triggered Reporting	882

9.2C.5	Intra frequency measurements without measurement gaps	882

9.2C.5.1	Intra frequency cell identification	882

9.2C.5.2	Measurement period	884

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	885

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	885

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	885

9.2C.6	Intra-frequency measurements with measurement gaps	886

9.2C.6.1	Intra-frequency cell identification	886

9.2C.6.3	Intrafrequency Measurement Period	887

9.2D	NR intra-frequency measurements for ATG	887

9.2D.1	Introduction	887

9.2D.2	Requirements applicability	888

9.2D.3	Number of cells and number of SSB	888

9.2D.3.1	Requirements for FR1	888

9.2D.4	Measurement Reporting Requirements	888

9.2D.4.1	Periodic Reporting	888

9.2D.4.2	Event-triggered Periodic Reporting	888

9.2D.4.3	Event Triggered Reporting	888

9.2D.5	Intra-frequency measurements without measurement gaps	889

9.2D.5.1	Intra-frequency cell identification	889

9.2D.5.2	Measurement period	891

9.2D.5.3	Scheduling availability of UE during intra-frequency measurements	892

9.2D.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	892

9.2D.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	893

9.2D.6	Intra-frequency measurements with measurement gaps	893

9.2D.6.1	Void	893

9.2D.6.2	Intra-frequency cell identification	893

9.2D.6.3	Intra-frequency Measurement Period	894

9.3	NR inter-frequency measurements	895

9.3.1	Introduction	895

9.3.2	Requirements applicability	897

9.3.2.1	Void	898

9.3.2.2	Void	898

9.3.3	Number of cells and number of SSB	898

9.3.3.1	Requirements for FR1	898

9.3.3.2	Requirements for FR2	898

9.3.4	Inter-frequency measurement with measurement gaps	898

9.3.4.1	Void	902

9.3.4.2	Void	902

9.3.5	Inter-frequency measurements	902

9.3.5.1	Void	904

9.3.5.2	Void	904

9.3.5.3	Void	904

9.3.6	Inter-frequency measurements reporting requirements	904

9.3.6.1	Periodic Reporting	904

9.3.6.2	Event-triggered Periodic Reporting	904

9.3.6.3	Event-triggered Reporting	904

9.3.7	Void	905

9.3.8	Inter-frequency SFTD measurement requirements	905

9.3.8.1	Introduction	905

9.3.8.2	SFTD Measurement delay	905

9.3.8.3	SFTD Measurement reporting delay	906

9.3.9	Inter frequency measurements without measurement gaps	906

9.3.9.1	Inter frequency Cell identification	906

9.3.9.2	Measurement period	911

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	913

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	914

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	914

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	915

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	915

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	915

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	916

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	916

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	917

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	919

9.3.10	Inter-frequency measurement with NCSG	919

9.3.10.1	Inter-frequency cell identification	919

9.3.10.2	Measurement period	921

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	921

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	921

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	922

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	923

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	925

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	925

9.3A.1	Introduction	925

9.3A.2	Requirements applicability	925

9.3A.3	Number of cells and number of SSB	926

9.3A.3.1	Requirements for FR1	926

9.3A.3.2	Requirements for FR2-2	926

9.3A.4	Inter-frequency cell identification	926

9.3A.5	Inter-frequency measurements	928

9.3A.6	NR Inter-frequency measurements reporting requirements	930

9.3A.6.1	Periodic Reporting	930

9.3A.6.2	Event-triggered Periodic Reporting	930

9.3A.6.3	Event-triggered Reporting	930

9.3A.8	Inter-frequency RSSI measurements	930

9.3A.9	Inter-frequency channel occupancy measurements	931

9.3B	NR inter-frequency measurements for RedCap	932

9.3B.1	Introduction	932

9.3B.2	Requirements applicability	932

9.3B.3	Number of cells and number of SSB	933

9.3B.3.1	Requirements for FR1	933

9.3B.3.2	Requirements for FR2	933

9.3B.4	Inter-frequency measurement with measurement gaps	933

9.3B.5	Inter-frequency measurements	934

9.3B.6	Inter-frequency measurements reporting requirements	935

9.3B.6.1	Periodic Reporting	935

9.3B.6.2	Event-triggered Periodic Reporting	935

9.3B.6.3	Event-triggered Reporting	935

9.3B.7	Inter frequency measurements without measurement gaps	936

9.3B.7.1	Inter frequency Cell identification	936

9.3B.7.2	Measurement period	938

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	939

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	939

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	940

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	940

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	940

9.3C	NR inter-frequency measurements for SAN	941

9.3C.1	Introduction	941

9.3C.2	Requirements applicability	941

9.3C.3	Number of cells and number of SSB	941

9.3C.3.1	Requirements for FR1	941

9.3C.4	Inter-frequency measurement with measurement gaps	942

9.3C.5	Inter-frequency measurements	943

9.3C.6	Inter-frequency measurements reporting requirements	943

9.3C.6.1	Periodic Reporting	943

9.3C.6.2	Event-triggered Periodic Reporting	943

9.3C.6.3	Event-triggered Reporting	944

9.3C.7	Inter frequency measurements without measurement gaps	944

9.3C.7.1	Inter frequency Cell identification	944

9.3C.7.2	Measurement period	946

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	946

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	946

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	947

9.3D	NR inter-frequency measurements for ATG	947

9.3D.1	Introduction	947

9.3D.2	Requirements applicability	947

9.3D.3	Number of cells and number of SSB	948

9.3D.3.1	Requirements for FR1	948

9.3D.4	Inter-frequency measurement with measurement gaps	948

9.3D.5	Inter-frequency measurements	949

9.3D.6	Inter-frequency measurements reporting requirements	950

9.3D.6.1	Periodic Reporting	950

9.3D.6.2	Event-triggered Periodic Reporting	950

9.3D.6.3	Event-triggered Reporting	950

9.3D.7	Void	950

9.3D.8	Void	950

9.3D.9	Inter frequency measurements without measurement gaps	950

9.3D.9.1	Inter frequency Cell identification	950

9.3D.9.2	Measurement period	952

9.3D.9.3	Scheduling availability of UE during inter-frequency measurements	953

9.3D.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	953

9.3D.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	954

9.4	Inter-RAT measurements	954

9.4.1	Introduction	954

9.4.2	NR − E-UTRAN FDD measurements	956

9.4.2.1	Introduction	956

9.4.2.2	Requirements when no DRX is used	956

9.4.2.3	Requirements when DRX is used	958

9.4.2.4	Measurement reporting requirements	959

9.4.2.4.1	Periodic Reporting	959

9.4.2.4.2	Event-Triggered Periodic Reporting	959

9.4.2.4.3	Event-Triggered Reporting	960

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	960

9.4.3	NR − E-UTRAN TDD measurements	960

9.4.3.1	Introduction	960

9.4.3.2	Requirements when no DRX is used	960

9.4.3.3	Requirements when DRX is used	962

9.4.3.4	Measurement reporting requirements	964

9.4.3.4.1	Periodic Reporting	964

9.4.3.4.2	Event-Triggered Periodic Reporting	964

9.4.3.4.3	Event-Triggered Reporting	964

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	965

9.4.4	Inter-RAT RSTD measurements	965

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	965

9.4.4.1.1	Introduction	965

9.4.4.1.2	Requirements	966

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	968

9.4.4.2.1	Introduction	968

9.4.4.2.2	Requirements	969

9.4.5	Inter-RAT E-CID measurements	972

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	972

9.4.5.1.1	Introduction	972

9.4.5.1.2	Requirements	972

9.4.5.1.3	Measurement Reporting Delay	973

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	973

9.4.5.2.1	Introduction	973

9.4.5.2.2	Requirements	973

9.4.5.2.3	Measurement Reporting Delay	973

9.4.6	NR − UTRAN FDD measurements	973

9.4.6.1	Introduction	973

9.4.6.2	Requirements when no DRX is used	973

9.4.6.3	Requirements when DRX is used	975

9.4.7	NR – E-UTRAN measurements with autonomous gaps	977

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	977

9.4.7.2	CGI reporting delay	977

9.4.8	NR – E-UTRAN measurements without measurement gaps	977

9.4.8.1	Introduction	977

9.4.8.2	Collision handling between EMW and MG/SMTC/SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement	977

9.4.8.3	NR − E-UTRAN FDD measurements	978

9.4.8.3.1	Introduction	978

9.4.8.3.2	Requirements when no DRX is used	978

9.4.8.3.3	Requirements when DRX is used	979

9.4.8.3.4	Measurement reporting requirements	980

9.4.8.3.4.1	Periodic Reporting	980

9.4.8.3.4.2	Event-Triggered Periodic Reporting	980

9.4.8.3.4.3	Event-Triggered Reporting	980

9.4.8.3.5	Scheduling availability during NR − E-UTRAN FDD measurements	981

9.4.8.3.5.1	Scheduling availability of UE performing inter-RAT measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	981

9.4.8.4	NR − E-UTRAN TDD measurements	981

9.4.8.4.1	Introduction	981

9.4.8.4.2	Requirements when no DRX is used	981

9.4.8.4.3	Requirements when DRX is used	983

9.4.8.4.4	Measurement reporting requirements	984

9.4.8.4.5	Scheduling availability during NR − E-UTRAN TDD measurements	984

9.4A	Inter-RAT measurements for RedCap	985

9.4A.1	Introduction	985

9.4A.2	NR − E-UTRAN FDD measurements	986

9.4A.2.1	Introduction	986

9.4A.2.2	Requirements when no DRX is used	987

9.4A.2.3	Requirements when DRX is used	988

9.4A.2.4	Measurement reporting requirements	989

9.4A.2.4.1	Periodic Reporting	989

9.4A.2.4.2	Event-Triggered Periodic Reporting	989

9.4A.2.4.3	Event-Triggered Reporting	989

9.4A.3	NR − E-UTRAN TDD measurements	989

9.4A.3.1	Introduction	989

9.4A.3.2	Requirements when no DRX is used	990

9.4A.3.3	Requirements when DRX is used	992

9.4A.3.4	Measurement reporting requirements	993

9.4A.3.4.1	Periodic Reporting	993

9.4A.3.4.2	Event-Triggered Periodic Reporting	993

9.4A.3.4.3	Event-Triggered Reporting	993

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	993

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	993

9.4A.4.2	CGI reporting delay	994

9.4A.4.3	CGI reporting scheduling restriction	994

9.5	L1-RSRP measurements for Reporting	995

9.5.1	Introduction	995

9.5.2	Requirements applicability	995

9.5.3	Measurement Reporting Requirements	996

9.5.3.1	Periodic Reporting	996

9.5.3.2	Semi-Persistent Reporting	996

9.5.3.3	Aperiodic Reporting	997

9.5.4	L1-RSRP measurement requirements	997

9.5.4.1	SSB based L1-RSRP Reporting	997

9.5.4.2	CSI-RS based L1-RSRP Reporting	1003

9.5.4A	Void	1007

9.5.4A.1	Void	1007

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1007

9.5.5.1	Measurement restriction for SSB based L1-RSRP	1007

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	1008

9.5.6	Scheduling availability of UE during L1-RSRP measurement	1009

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1009

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1009

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1009

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1011

9.5A	L1-RSRP measurements for Reporting under CCA	1011

9.5A.1	Introduction	1011

9.5A.2	Requirements applicability	1012

9.5A.3	Measurement Reporting Requirements	1012

9.5A.3.1	Periodic Reporting	1012

9.5A.3.2	Semi-Persistent Reporting	1012

9.5A.3.3	Aperiodic Reporting	1013

9.5A.4	L1-RSRP measurement requirements	1013

9.5A.4.1	SSB based L1-RSRP Reporting	1013

9.5A.5	Measurement restriction for L1-RSRP measurement	1015

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	1016

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	1016

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1016

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1016

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	1016

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	1016

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1017

9.5B	L1-RSRP measurements for Reporting for RedCap	1017

9.5B.1	Introduction	1017

9.5B.2	Requirements applicability	1017

9.5B.3	Measurement Reporting Requirements	1018

9.5B.3.1	Periodic Reporting	1018

9.5B.3.2	Semi-Persistent Reporting	1019

9.5B.3.3	Aperiodic Reporting	1019

9.5B.4	L1-RSRP measurement requirements	1019

9.5B.4.1	SSB based L1-RSRP Reporting	1019

9.5B.4.2	CSI-RS based L1-RSRP Reporting	1021

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1024

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	1024

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	1024

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	1025

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1025

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1025

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1025

9.5C	L1-RSRP measurements for Reporting for satellite access	1026

9.5C.1	Introduction	1026

9.5C.3	Measurement Reporting Requirements	1027

9.5C.3.1	Periodic Reporting	1027

9.5C.3.2	Semi-Persistent Reporting	1027

9.5C.3.3	Aperiodic Reporting	1027

9.5C.4	L1-RSRP measurement requirements	1027

9.5C.4.1	SSB based L1-RSRP Reporting	1027

9.5C.5	Measurement restriction for L1-RSRP measurement	1029

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	1029

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	1030

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	1030

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1030

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1030

9.5D	L1-RSRP measurements for Reporting for ATG	1030

9.5D.1	Introduction	1030

9.5D.2	Requirements applicability	1031

9.5D.3	Measurement Reporting Requirements	1031

9.5D.3.1	Periodic Reporting	1031

9.5D.3.2	Semi-Persistent Reporting	1031

9.5D.3.3	Aperiodic Reporting	1032

9.5D.4	L1-RSRP measurement requirements	1032

9.5D.4.1	SSB based L1-RSRP Reporting	1032

9.5D.4.2	CSI-RS based L1-RSRP Reporting	1034

9.5D.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1035

9.5D.5.1	Measurement restriction for SSB based L1-RSRP	1035

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	1035

9.5.6	Scheduling availability of UE during L1-RSRP measurement	1036

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1036

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1036

9.6	NE-DC: Measurements	1036

9.6.1	Introduction	1036

9.6.2	SFTD Measurements	1036

9.6.2.1	Introduction	1036

9.6.2.2	SFTD Measurement requirements	1036

9.7	Cross Link Interference measurements	1037

9.7.1	Introduction	1037

9.7.2	SRS-RSRP measurements	1038

9.7.2.1	Introduction	1038

9.7.2.2	Requirements applicability	1038

9.7.2.3	Measurement Reporting Requirements	1038

9.7.2.3.1	Periodic Reporting	1038

9.7.2.3.2	Event-triggered Periodic Reporting	1038

9.7.2.3.3	Event Triggered Reporting	1038

9.7.2.4	Measurement capability	1039

9.7.2.5	SRS-RSRP measurement period	1039

9.7.3	CLI-RSSI measurements	1039

9.7.3.1	Introduction	1039

9.7.3.2	Requirements applicability	1039

9.7.3.3	Measurement Reporting Requirements	1039

9.7.3.3.1	Periodic Reporting	1039

9.7.3.3.2	Event-triggered Periodic Reporting	1040

9.7.3.3.3	Event Triggered Reporting	1040

9.7.3.4	Measurement capability	1040

9.7.3.5	CLI-RSSI measurement period	1040

9.7.4	Scheduling availability of UE during CLI measurements	1040

9.7.4.1	Scheduling availability of UE performing measurement on FR1	1040

9.7.4.2	Scheduling availability of UE performing measurement on FR2	1041

9.8	L1-SINR measurements for Reporting	1042

9.8.1	Introduction	1042

9.8.2	Requirements applicability	1042

9.8.3	Measurement Reporting Requirements	1043

9.8.3.1	Periodic Reporting	1043

9.8.3.2	Semi-Persistent Reporting	1043

9.8.4	L1-SINR measurement requirements	1043

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1043

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1048

9.8.5	Measurement restriction for L1-SINR measurement	1050

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1050

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1051

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1052

9.8.6	Scheduling availability of UE during L1-SINR measurement	1053

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1053

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1053

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1055

9.8D	L1-SINR measurements for Reporting for ATG	1055

9.8D.1	Introduction	1055

9.8D.2	Requirements applicability	1055

9.8D.3	Measurement Reporting Requirements	1056

9.8D.3.1	Periodic Reporting	1056

9.8D.3.2	Semi-Persistent Reporting	1056

9.8D.3.3	Aperiodic Reporting	1056

9.8D.4	L1-SINR measurement requirements	1057

9.8D.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1057

9.8D.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1058

9.8D.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1059

9.9	NR measurements for positioning	1060

9.9.1	Introduction	1060

9.9.1.1	General Aspects of Gap-based Measurement	1060

9.9.1.2	General Aspects of Gapless Measurement	1061

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1062

9.9.2	RSTD measurements	1063

9.9.2.1	Introduction	1063

9.9.2.2	Requirements Applicability	1063

9.9.2.3	Measurement Capability	1063

9.9.2.4	Measurement Reporting Requirements	1063

9.9.2.4.1	Void	1064

9.9.2.4.2	Void	1064

9.9.2.4.3	Void	1064

9.9.2.5	Measurements Period Requirements	1064

9.9.2.6	Void	1067

9.9.2.7	Measurements Period Requirements without Measurement Gaps	1067

9.9.2.8	Void	1070

9.9.2.9	Measurements Period Requirements with both MG and PPW	1070

9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation	1070

9.9.3	PRS-RSRP measurements	1074

9.9.3.1	Introduction	1074

9.9.3.2	Requirements applicability	1074

9.9.3.3	Measurement Capability	1074

9.9.3.4	Measurement Reporting Requirements	1074

9.9.3.5	Measurement Period Requirements	1074

9.9.3.6	Measurement Period Requirements without Measurement Gaps	1077

9.9.3.7	Void	1079

9.9.3.8	Measurements Period Requirements with both MG and PPW	1079

9.9.4	UE Rx-Tx time difference measurements	1080

9.9.4.1 Introduction	1080

9.9.4.2 Requirements Applicability	1080

9.9.4.3 Measurement Capability	1080

9.9.4.4 Measurement Reporting Requirements	1080

9.9.4.5	Measurement Period Requirements	1081

9.9.4.6	Measurement Period Requirements without Measurement Gaps	1084

9.9.4.7	Void	1087

9.9.4.8	Measurements Period Requirements with both MG and PPW	1087

9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation	1088

9.9.5	E-CID measurements	1092

9.9.5.1	Introduction	1092

9.9.5.2	Measurement Requirements	1092

9.9.5.2.1	Intra-frequency Measurement Requirements	1092

9.9.5.2.2	Inter-frequency Measurement Requirements	1092

9.9.5.2.3	Measurement Reporting Delay	1093

9.9.6	PRS-RSRPP measurements	1093

9.9.6.1	Introduction	1093

9.9.6.2	Requirements applicability	1093

9.9.6.3	Measurement capability	1093

9.9.6.4	Measurement reporting requirements	1093

9.9.6.5	Measurement period requirements	1093

9.9.6.6	Measurement Period Requirements without Measurement Gaps	1094

9.9.6.7	Void	1094

9.9.6.8	Measurements Period Requirements with both MG and PPW	1094

9.9.7	Measurement requirements for DL RSCPD reported with RSTD	1094

9.9.7.1	Introduction	1094

9.9.7.2	Requirements Applicability	1094

9.9.7.3	Measurement Capability	1094

9.9.7.4	Measurement Reporting Requirements	1094

9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD	1095

9.9.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	1096

9.9.8.1	Introduction	1096

9.9.8.2	Requirements Applicability	1096

9.9.8.3	Measurement Capability	1096

9.9.8.4	Measurement Reporting Requirements	1096

9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference	1096

9.9A	NR measurements for positioning for RedCap	1100

9.9A.1	Introduction	1100

9.9A.1.1	General Aspects of Gap-based Measurement	1100

9.9A.1.2	General Aspects of Gapless Measurement for RedCap positioning without FH	1101

9.9A.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps for RedCap positioning without FH	1102

9.9A.2	RSTD measurements for RedCap	1103

9.9A.2.1	Introduction	1103

9.9A.2.2	Requirements Applicability	1103

9.9A.2.3	Measurement Capability	1103

9.9A.2.4	Measurement Reporting Requirements	1103

9.9A.2.5	Measurements Period Requirements without FH	1103

9.9A.2.5.1	Measurements Period Requirements without FH with MG	1103

9.9A.2.5.2	Measurements Period Requirements without FH without MG	1106

9.9A.2.5.3	Measurements Period Requirements without FH with both MG and PPW	1109

9.9A.2.6	Measurements Period Requirements with FH	1110

9.9A.2.6.1	Measurements Period Requirements with FH with MG	1110

9.9A.3	PRS-RSRP measurements for RedCap	1110

9.9A.3.1	Introduction	1110

9.9A.3.2	Requirements applicability	1110

9.9A.3.3	Measurement Capability	1111

9.9A.3.4	Measurement Reporting Requirements	1111

9.9A.3.5	Measurements Period Requirements without FH	1111

9.9A.3.5.1	Measurement Period Requirements without FH with MG	1111

9.9A.3.5.2	Measurement Period Requirements without FH without MG	1113

9.9A.3.5.3	Measurements Period Requirements without FH with both MG and PPW	1116

9.9A.3.6	Measurements Period Requirements with FH	1116

9.9A.3.6.1	Measurements Period Requirements with FH with MG	1116

9.9A.4	UE Rx-Tx time difference measurements for RedCap	1117

9.9A.4.1	Introduction	1117

9.9A.4.2	Requirements Applicability	1117

9.9A.4.3	Measurement Capability	1117

9.9A.4.4 Measurement Reporting Requirements	1117

9.9A.4.5	Measurement Period Requirements without FH with MG	1117

9.9A.4.6	Measurement Period Requirements without FH without MG	1117

9.9A.4.7	Measurements Period Requirements without FH with both MG and PPW	1118

9.9A.4.8	Measurements Period Requirements with FH	1118

9.9A.5	PRS-RSRPP measurements for RedCap	1118

9.9A.5.1	Introduction	1118

9.9A.5.2	Requirements Applicability	1118

9.9A.5.3	Measurement Capability	1118

9.9A.5.4	Measurement Reporting Requirements	1118

9.9A.5.5	Measurement Period Requirements without FH with MG	1119

9.9A.5.6	Measurement Period Requirements without FH without MG	1119

9.9A.5.7	Measurements Period Requirements without FH with both MG and PPW	1119

9.9A.5.8	Measurements Period Requirements with FH	1119

9.10	CSI-RS based L3 measurements	1119

9.10.1	Introduction	1119

9.10.2	CSI-RS based intra-frequency measurements	1119

9.10.2.1	Introduction	1119

9.10.2.2	Requirements applicability	1120

9.10.2.3	Number of cells and number of CSI-RS	1121

9.10.2.3.1	Requirements for FR1	1121

9.10.2.3.2	Requirements for FR2	1121

9.10.2.4	Measurement Reporting Requirements	1121

9.10.2.4.1	Periodic Reporting	1122

9.10.2.4.2	Event-triggered Periodic Reporting	1122

9.10.2.4.3	Event Triggered Reporting	1122

9.10.2.5	Intra-frequency measurements without measurement gaps	1122

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1124

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1124

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	1124

9.10.3	CSI-RS based Inter-frequency measurements	1125

9.10.3.1	Introduction	1125

9.10.3.2	Requirements applicability	1125

9.10.3.3	Number of cells and number of CSI-RS resources	1126

9.10.3.3.1	Requirements for FR1	1126

9.10.3.3.2	Requirements for FR2	1126

9.10.3.4	Measurements reporting requirements	1126

9.10.3.4.1	Periodic Reporting	1126

9.10.3.4.2	Event-triggered Periodic Reporting	1126

9.10.3.4.3	Event-triggered Reporting	1126

9.10.3.5	Inter frequency measurements with measurement gaps	1127

9.10D	CSI-RS based L3 measurements for ATG	1128

9.10D.1	Introduction	1128

9.10D.2	CSI-RS based intra-frequency measurements	1129

9.10D.2.1	Introduction	1129

9.10D.2.2	Requirements applicability	1129

9.10D.2.3	Number of cells and number of CSI-RS	1130

9.10D.2.3.1	Requirements for FR1	1130

9.10D.2.4	Measurement Reporting Requirements	1130

9.10D.2.4.1	Periodic Reporting	1130

9.10D.2.4.2	Event-triggered Periodic Reporting	1130

9.10D.2.4.3	Event Triggered Reporting	1131

9.10D.2.5	Intra-frequency measurements without measurement gaps	1131

9.10D.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1132

9.10D.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1132

9.10D.3	CSI-RS based Inter-frequency measurements	1133

9.10D.3.1	Introduction	1133

9.10D.3.2	Requirements applicability	1133

9.10D.3.3	Number of cells and number of CSI-RS resources	1133

9.10D.3.3.1	Requirements for FR1	1133

9.10D.3.4	Measurements reporting requirements	1134

9.10D.3.4.1	Periodic Reporting	1134

9.10D.3.4.2	Event-triggered Periodic Reporting	1134

9.10D.3.4.3	Event-triggered Reporting	1134

9.10D.3.5	Inter frequency measurements with measurement gaps	1134

9.11	NR measurements with autonomous gaps	1135

9.11.1	Introduction	1135

9.11.2	CGI identification of an NR cell with autonomous gaps	1136

9.11.3	CGI reporting delay	1137

9.11A	NR measurements with autonomous gaps for RedCap	1137

9.11A.1	Introduction	1137

9.11A.2	CGI identification of an NR cell with autonomous gaps	1137

9.11A.3	CGI reporting delay	1138

9.11A.4	CGI reporting scheduling restriction	1138

9.11D	NR measurements with autonomous gaps for ATG	1139

9.11D.1	Introduction	1139

9.11D.2	CGI identification of an NR cell with autonomous gaps	1139

9.11D.3	CGI reporting delay	1140

9.12	Measurement for Propagation Delay Compensation	1140

9.12.1	Introduction	1140

9.12.2	Requirements Applicability	1140

9.12.3	Measurement Capability	1140

9.12.4	Measurement period requirements	1141

9.12.4.1	PRS Measurement Period	1141

9.12.4.2	TRS Measurement Period	1142

9.12.5	Measurement Reporting Requirements	1142

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	1143

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	1143

9.12.8	Measurement requirement for Propagation Delay Compensation with MUSIM gaps	1143

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	1143

9.13.1	Introduction	1143

9.13.2	Requirements Applicability	1144

9.13.3	Measurement Reporting Requirements	1144

9.13.3.1	Periodic Reporting	1145

9.13.3.2	Semi-Persistent Reporting	1145

9.13.3.3	Aperiodic Reporting	1145

9.13.4	L1-RSRP measurement requirements	1145

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	1145

9.13.5	Measurement restriction for L1-RSRP measurement	1148

9.13.5.1	Measurement restriction for SSB based L1-RSRP	1148

9.13.6	Scheduling availability of UE during L1-RSRP measurement	1149

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1149

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1149

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1149

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1150

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1150

9.14	Intra-frequency L1-RSRP measurements for neighbor cell	1150

9.14.1	Introduction	1150

9.14.2	Requirements Applicability	1151

9.14.3	Measurement Reporting Requirements	1151

9.14.3.1	Periodic Reporting	1151

9.14.3.2	Semi-Persistent Reporting	1151

9.14.3.3	Aperiodic Reporting	1152

9.14.4	Number of cells and number of SSB	1152

9.14.5	L1-RSRP measurement requirements without measurement gaps	1152

9.14.5.1	SSB based L1-RSRP Reporting	1152

9.14.6	Measurement restriction for L1-RSRP measurement	1155

9.14.6.1	Measurement restriction for SSB based L1-RSRP	1155

9.14.7	Scheduling availability of UE during L1-RSRP measurement	1156

9.14.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1156

9.14.7.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1156

9.14.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1156

9.14.7.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1157

9.14.7.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1157

9.15	NR inter-frequency L1 measurement	1157

9.15.1	Introduction	1157

9.15.2	Requirements Applicability	1157

9.15.3	Measurement Reporting Requirements	1158

9.15.3.1	Periodic Reporting	1158

9.15.3.2	Semi-Persistent Reporting	1158

9.15.3.3	Aperiodic Reporting	1159

9.15.4	Number of cells and number of SSB	1159

9.15.5	inter-frequency L1-RSRP with MG	1159

9.15.5.1	Inter-frequency SSB based L1-RSRP Reporting	1159

9.15.6	Inter frequency L1-RSRP measurement without measurement gaps	1160

9.15.6.1	Inter frequency L1-RSRP measurement requirements	1160

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement	1160

9.15.6.2	Measurement restriction for inter frequency L1-RSRP measurement	1162

9.15.6.2.1	Measurement restriction for SSB based L1-RSRP	1163

9.15.6.3	Scheduling availability of UE during inter-frequency L1-RSRP measurements	1164

9.15.6.3.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1164

9.15.6.3.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1164

9.15.6.3.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1164

9.15.6.3.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1165

9.15.6.3.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1165

10	Measurement Performance requirements	1177

10.1	NR measurements	1177

10.1.1	Introduction	1177

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	1177

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	1177

10.1.2.1.1	Absolute SS-RSRP Accuracy	1177

10.1.2.1.2	Relative SS-RSRP Accuracy	1178

10.1.2.2	Void	1179

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	1179

10.1.2.3.1	Absolute CSI-RSRP Accuracy	1179

10.1.2.3.2	Relative CSI-RSRP Accuracy	1180

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1181

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	1181

10.1.2B.1.1	Absolute SS-RSRP Accuracy	1181

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	1182

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	1182

10.1.2C.1.1	Absolute SS-RSRP Accuracy	1182

10.1.2C.1.2	Relative SS-RSRP Accuracy	1183

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	1183

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	1183

10.1.3.1.1	Absolute SS-RSRP Accuracy	1183

10.1.3.1.2	Relative SS-RSRP Accuracy	1184

10.1.3.2	Void	1185

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	1185

10.1.3.3.1	Absolute CSI-RSRP Accuracy	1185

10.1.3.3.2	Relative CSI-RSRP Accuracy	1186

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1187

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	1187

10.1.3B.1.1	Absolute SS-RSRP Accuracy	1187

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	1188

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	1188

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1188

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	1189

10.1.4.2	Void	1190

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	1190

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	1190

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	1191

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1192

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	1192

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	1192

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	1193

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	1193

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	1193

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	1194

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	1194

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	1194

10.1.5.1.1	Absolute SS-RSRP Accuracy	1194

10.1.5.1.2	Relative SS-RSRP Accuracy	1195

10.1.5.2	Void	1196

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	1196

10.1.5.3.1	Absolute CSI-RSRP Accuracy	1196

10.1.5.3.2	Relative CSI-RSRP Accuracy	1197

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1198

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	1198

10.1.5B.1.1	Absolute SS-RSRP Accuracy	1198

10.1.6	RSRP Measurement Report Mapping	1199

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	1201

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1201

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	1201

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	1202

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	1202

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1203

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1203

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	1203

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	1204

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1204

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	1204

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	1205

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1205

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	1205

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	1206

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	1206

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1206

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1206

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	1207

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	1207

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1207

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	1207

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	1208

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	1209

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	1209

10.1.9.2.2	Relative CSI-RSRQ Accuracy	1210

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1211

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1211

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	1211

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	1212

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1212

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	1212

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	1213

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	1214

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1214

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	1214

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	1214

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	1215

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	1215

10.1.10.2.2	Relative CSI-RSRQ Accuracy	1216

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1217

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1217

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	1217

10.1.11	RSRQ report mapping	1218

10.1.12	Intra-frequency SINR accuracy requirements for FR1	1218

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	1218

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	1218

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	1219

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	1219

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	1220

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	1220

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	1220

10.1.13	Intra-frequency SINR accuracy requirements for FR2	1221

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	1221

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	1221

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	1222

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	1222

10.1.14	Inter-frequency SINR accuracy requirements for FR1	1222

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	1222

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	1222

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	1223

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	1224

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	1224

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	1225

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	1226

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	1226

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	1226

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	1227

10.1.15	Inter-frequency SINR accuracy requirements for FR2	1228

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	1228

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	1228

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	1228

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	1229

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	1229

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	1230

10.1.16	SINR report mapping	1231

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	1231

10.1.17	Power Headroom	1232

10.1.17.1	Power Headroom Report	1232

10.1.17.1.1	Power Headroom Report Mapping	1232

10.1.18	PCMAX,c,f	1233

10.1.18.1	Report Mapping	1233

10.1.19	L1-RSRP accuracy requirements for FR1	1233

10.1.19.1	SSB based L1-RSRP accuracy requirements	1233

10.1.19.1.1	Absolute Accuracy	1233

10.1.19.1.2	Relative Accuracy	1234

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	1235

10.1.19.2.1	Absolute Accuracy	1235

10.1.19.2.2	Relative Accuracy	1236

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	1237

10.1.19C.1	SSB based L1-RSRP accuracy requirements	1237

10.1.19C.1.1	Absolute Accuracy	1237

10.1.19C.1.2	Relative Accuracy	1238

10.1.20	L1-RSRP accuracy requirements for FR2	1238

10.1.20.1	SSB based L1-RSRP accuracy requirements	1238

10.1.20.1.1	Absolute Accuracy	1238

10.1.20.1.2	Relative Accuracy	1239

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	1240

10.1.20.2.1	Absolute Accuracy	1240

10.1.20.2.2	Relative Accuracy	1241

10.1.21	SFTD accuracy requirements	1242

10.1.21.1	SFTD acuracy requirements for NE-DC	1242

10.1.21.2	SFTD acuracy requirements for NR-DC	1244

10.1.21.3	Inter frequency SFTD acuracy requirements	1245

10.1.22	CLI measurement accuracy requirements	1246

10.1.22.1	SRS-RSRP	1246

10.1.22.1.1	SRS-RSRP Accuracy	1246

10.1.22.1.2	SRS-RSRP report mapping	1248

10.1.22.2	CLI-RSSI	1248

10.1.22.2.1	CLI-RSSI Accuracy	1248

10.1.22.2.2	CLI-RSSI report mapping	1249

10.1.23	RSTD Measurements	1250

10.1.23.1	Introduction	1250

10.1.23.2	Measurement Accuracy Requirements	1250

10.1.23.3	Report mapping	1260

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	1260

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	1262

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	1264

10.1.24	PRS-RSRP Measurements	1266

10.1.24.1	Introduction	1266

10.1.24.2	Measurement Accuracy Requirements	1266

10.1.24.2.1	Absolute PRS RSRP accuracy	1266

10.1.24.2.2	Relative PRS RSRP accuracy	1270

10.1.24.3	Report mapping	1274

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	1274

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	1275

10.1.25	UE Rx-Tx Time Difference Measurements	1277

10.1.25.1	Introduction	1277

10.1.25.2	Measurement Accuracy Requirements	1277

10.1.25.3	Report mapping	1294

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1294

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	1296

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1297

10.1.26	FR2 P-MPR report	1299

10.1.26.1	Report mapping	1299

10.1.27	L1-SINR accuracy requirements for FR1	1299

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1299

10.1.27.1.1	Absolute Accuracy	1299

10.1.27.1.2	Relative Accuracy	1300

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1301

10.1.27.2.1	Absolute Accuracy	1301

10.1.27.2.2	Relative Accuracy	1303

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1304

10.1.27.3.1	Absolute Accuracy	1304

10.1.27.3.2	Relative Accuracy	1306

10.1.28	L1-SINR accuracy requirements for FR2	1308

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1315

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1315

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1315

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1315

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1315

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	1315

10.1.30.1.2	Relative Accuracy of SS-RSRQ	1316

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1317

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1317

10.1.31.1.1	Absolute SS-SINR Accuracy	1317

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1317

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1317

10.1.32.1.1	Aboslute Accuracy of SS-SINR	1317

10.1.32.1.2	Relative Accuracy of SS-SINR	1318

10.1.33	L1-RSRP accuracy requirements under CCA	1319

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1319

10.1.33.1.1	Absolute Accuracy	1319

10.1.33.1.2	Relative Accuracy	1319

10.1.34	RSSI measurements under CCA	1320

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1320

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1320

10.1.34.3	RSSI measurement report mapping	1320

10.1.35	Channel occupancy measurements under CCA	1321

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1321

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1321

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1321

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1321

10.1.36.1.1	Absolute SS-RSRP Accuracy	1321

10.1.36.1.2	Relative SS-RSRP Accuracy	1322

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1323

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1323

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1323

10.1.37.1.2	Relative Accuracy of SS-RSRP	1323

10.1.38	PRS-RSRPP Measurements	1324

10.1.38.1	Introduction	1324

10.1.38.2	Measurement Accuracy Requirements	1324

10.1.38.2.1	Absolute PRS RSRPP accuracy	1324

10.1.38.3	Report mapping	1328

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1328

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1329

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1330

10.1.39.1	Void	1330

10.1.39.2	 Measurement Accuracy Requirements for PRS	1330

10.1.39.3	 Measurement Accuracy Requirements for TRS	1334

10.1.40	Void	1337

10.1A	NR measurements for RedCap	1337

10.1A.1	Introduction	1337

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1337

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1337

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1337

10.1A.2.1.2	Relative SS-RSRP Accuracy	1338

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1339

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1339

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1339

10.1A.3.1.2	Relative SS-RSRP Accuracy	1339

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1339

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1339

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1339

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1340

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1341

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1341

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1341

10.1A.5.1.2	Relative SS-RSRP Accuracy	1341

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1341

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1341

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1341

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1342

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1342

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1342

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1342

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1342

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1342

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1343

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1344

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1344

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1344

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1344

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1344

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1344

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1344

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1345

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1345

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1345

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1345

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1345

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1345

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1346

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1347

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1347

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1347

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1347

10.1A.14	 L1-RSRP accuracy requirements for FR1	1347

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1347

10.1A.14.1.1	Absolute Accuracy	1347

10.1A.14.1.2	Relative Accuracy	1348

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1349

10.1A.14.2.1	Absolute Accuracy	1349

10.1A.14.2.2	Relative Accuracy	1350

10.1A.15	 L1-RSRP accuracy requirements for FR2	1351

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1351

10.1A.15.1.1	Absolute Accuracy	1351

10.1A.15.1.2	Relative Accuracy	1351

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1351

10.1A.15.2.1	Absolute Accuracy	1351

10.1A.15.2.2	Relative Accuracy	1351

10.2	E-UTRAN measurements	1352

10.2.1	Introduction	1352

10.2.2	E-UTRAN RSRP measurements	1352

10.2.3	E-UTRAN RSRQ measurements	1352

10.2.4	E-UTRAN RSTD measurements	1352

10.2.5	E-UTRAN RS-SINR measurements	1353

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1353

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1353

10.2A	E-UTRAN measurements for RedCap	1354

10.2A.1	Introduction	1354

10.2A.2	E-UTRAN RSRP measurements	1354

10.2A.3	E-UTRAN RSRQ measurements	1354

10.2A.4	E-UTRAN RS-SINR measurements	1355

10.3	UTRAN FDD Measurements	1355

10.3.1	UTRAN FDD CPICH RSCP	1355

10.3.2	UTRAN FDD CPICH Ec/No	1356

10.4	V2X measurements	1356

10.4.1	Introduction	1356

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1356

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1356

10.4.2.2	PSBCH-RSRP Relative Accuracy	1357

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1358

10.4.3.1	Absolute SL-RSSI Accuracy	1358

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1358

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1358

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1359

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1359

11	Void	1360

12	V2X Requirements	1139

12.1	Introduction	1139

12.2	UE Transmit Timing	1139

12.2.1	Introduction	1139

12.2.2	GNSS as synchronization reference source	1140

12.2.3	NR Cell as synchronization reference source	1140

12.2.4	E-URTAN Cell as synchronization reference source	1140

12.2.5	SyncRef UE as synchronization reference source	1141

12.3	Initiation/Cease of SLSS Transmissions	1141

12.3.1	Introduction	1141

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1141

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1142

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1143

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1143

12.3A	Initiation/Cease of SLSS Transmissions with CCA	1144

12.3A.1	Introduction	1144

12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1144

12.3A.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1144

12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1144

12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1145

12.4	Selection / Reselection of V2X Synchronization Reference Source	1145

12.4A	Selection / Reselection of Sidelink Synchronization Reference Source with CCA	1147

12.5	L1 SL-RSRP measurements	1149

12.5.1	Introduction	1149

12.5.2	SL-RSRP measurements	1150

12.6	Congestion Control measurements	1150

12.7	Interruption	1150

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1150

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1151

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1152

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1153

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1153

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1154

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1154

12.7.8	Interruptions at NR sidelink discovery configuration	1154

12.7.9	Interruptions to WAN due to sidelink carrier addition/release	1155

12.8	Reliability of GNSS signal	1155

12.9	Scheduling availability	1155

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1155

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1156

12.10	Selection / Reselection of relay UE	1156

12.10.1	Introduction	1156

12.10.2	Selection / Reselection of relay UE	1156

12.11	Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation	1157

12.12	Selection / Reselection of Synchronization Reference Source for NR SL Carrier Aggregation	1157

12A	NR Sidelink Measurements for Positioning	1159

12A.1	Introduction	1159

12A.2	SL-RSTD measurements	1159

12A.2.1	Introduction	1159

12A.2.3	Measurement Capability	1159

12A.2.4	Measurement Reporting Requirements	1159

12A.2.5	Measurements Period Requirements	1160

12A.3	SL-RSRP measurements	1161

12A.3.1	Introduction	1161

12A.3.2	Requirements Applicability	1161

12A.3.4	Measurement Reporting Requirements	1161

12A.3.5	Measurements Period Requirements	1161

12A.4	SL-Rx-Tx measurements	1162

12A.4.1	Introduction	1162

12A.4.2	Requirements Applicability	1162

12A.4.3	Measurement Capability	1162

12A.4.4	Measurement Reporting Requirements	1162

12A.4.5	Measurement Period Requirements	1163

12A.5	SL-RSRPP measurements	1164

12A.5.1	Introduction	1164

12A.5.2	Requirements Applicability	1164

12A.5.3	Measurement Capability	1164

12A.5.4	Measurement Reporting Requirements	1164

12A.5.5	Measurement Period Requirements	1164

12A.6	SL-AoA measurements	1165

12A.6.1	Introduction	1165

12A.6.2	Requirements Applicable	1165

12A.6.3	Measurement Capability	1165

12A.6.4	Measurement Reporting Requirements	1165

12A.6.5	Measurement Period Requirements	1165

12A.7	SL-RTOA measurements	1166

12A.7.1	Introduction	1166

12A.7.2	Requirements Applicable	1166

12A.7.3	Measurement Capability	1166

12A.7.4	Measurement Reporting Requirements	1166

12A.7.5	Measurement Period Requirements	1167

13	Measurement Performance Requirements for NR gNB	1167

13.1	UL-RTOA	1167

13.1.1	Report mapping	1167

13.1.1A	Additional Path Report Mapping for UL-RTOA	1169

13.2	gNB Rx-Tx time difference	1171

13.2.1	Report mapping	1171

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	1173

13.2.2	Measurement Accuracy Requirements	1175

13.2.2.1	Introduction	1175

13.3	UL SRS RSRP measurement	1176

13.3.1	Report mapping	1176

13.3.2	Measurement accuracy requirements	1177

13.3.2.1	Introduction	1177

13.3.2.2	Requirements	1177

13.4	AoA/ZoA	1178

13.4.1	Report mapping	1178

13.5	Timing advance (TADV)	1179

13.5.1	Report mapping	1179

13.6	UL SRS RSRPP measurement	1180

13.6.1	Report mapping	1180

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1182

13.7.1	Report mapping	1182

13.7.2	Measurement Accuracy Requirements	1182

13.7.2.1	Introduction	1182

13.7.2.2	Requirements	1182

Annex A (normative): Test Cases	1184

A.1	Purpose of annex	1184

A.2	Requirement classification for statistical testing	1184

A.2.1	Types of requirements in TS 38.133	1184

A.2.1.1	Time and delay requirements on UE higher layer actions	1184

A.2.1.2	Measurements of power levels, relative powers and time	1184

A.2.1.3	Implementation requirements	1185

A.2.1.4	Physical layer timing requirements	1185

A.2.1.5	Requirements under CCA	1185

A.3	RRM test configurations	1186

A.3.1	Reference measurement channels	1186

A.3.1.1	PDSCH	1186

A.3.1.1.1	FDD	1186

A.3.1.1.2	TDD	1187

A.3.1.2	CORESET for RMSI scheduling	1190

A.3.1.2.1	FDD	1190

A.3.1.2.2	TDD	1191

A.3.1.3	CORESET for RMC scheduling	1194

A.3.1.3.1	FDD	1194

A.3.1.3.2	TDD	1195

A.3.1.4	TDD UL/DL configuration	1198

A.3.1A	Reference measurement channels under CCA	1200

A.3.1A.1	PDSCH	1200

A.3.1A.1.1	TDD	1200

A.3.1A.2	CORESET for RMSI scheduling	1201

A.3.1A.2.1	TDD	1201

A.3.1A.3	CORESET for RMC scheduling	1202

A.3.1A.3.1	TDD	1202

A.3.1A.4	TDD UL/DL configuration	1203

A.3.1A.5	RMC burst transmission model	1203

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1203

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1204

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1204

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1205

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1205

A.3.2.2	Void	1206

A.3.3	Reference DRX configurations	1206

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1206

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1207

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1207

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1207

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1208

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1208

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1208

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1209

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1209

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1209

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1210

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1210

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1210

A.3.4	Test Cases with Different Channel Bandwidths	1211

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1211

A.3.4.1.1	Introduction	1211

A.3.4.1.2	Principle of testing	1211

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1211

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1211

A.3.5.1.1	Introduction	1211

A.3.5.1.2	Principle of Testing	1211

A.3.6	Antenna configurations	1211

A.3.6.1	Antenna configurations for FR1	1211

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1211

A.3.6.1.1.1	Introduction	1211

A.3.6.1.1.2	Principle of testing	1212

A.3.6.2	Antenna configurations for FR2	1215

A.3.6A	Antenna configurations with unlicensed bands	1215

A.3.6A.1	Antenna configurations for FR1	1215

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1215

A.3.6A.1.1.1	Introduction	1215

A.3.6A.1.1.2	Principle of testing	1215

A.3.7	EN-DC test setup	1217

A.3.7.1	Introduction	1217

A.3.7.2	E-UTRAN Serving Cell Parameters	1217

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1217

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1218

A.3.7A	NR FR1-FR2 test setup	1219

A.3.7B	EN-DC test setup with unlicensed bands	1220

A.3.7B.1	Introduction	1220

A.3.7B.2	E-UTRAN Serving Cell Parameters	1220

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1220

A.3.7C	LTE-FR1/FR2 test setup	1221

A.3.7D	NE-DC test setup	1221

A.3.7D.1	Introduction	1221

A.3.7D.2	E-UTRAN Serving Cell Parameters	1221

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1221

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1221

A.3.8	PRACH configurations	1221

A.3.8.1	Introduction	1221

A.3.8.2	PRACH configurations in FR1	1222

A.3.8.2.1	FR1 PRACH configuration 1	1222

A.3.8.2.2	FR1 PRACH configuration 2	1222

A.3.8.2.3	FR1 PRACH configuration 3	1223

A.3.8.2.4	FR1 PRACH configuration 4	1224

A.3.8.3	PRACH configurations in FR2	1225

A.3.8.3.1	FR2 PRACH configuration 1	1225

A.3.8.3.2	FR2 PRACH configuration 2	1226

A.3.8.3.3	FR2 PRACH configuration 3	1227

A.3.8.3.4	FR2 PRACH configuration 4	1228

A.3.8A	PRACH configurations under CCA	1229

A.3.8A.1	Introduction	1229

A.3.8A.2	PRACH configurations in FR1	1229

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1229

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1230

A.3.9	BWP configurations	1231

A.3.9.1	Introduction	1231

A.3.9.2	Downlink BWP configurations	1232

A.3.9.2.1	Initial BWP	1232

A.3.9.2.2	Dedicated BWP	1232

A.3.9.3	Uplink BWP configurations	1233

A.3.9.3.1	Initial BWP	1233

A.3.9.3.2	Dedicated BWP	1233

A.3.9A	BWP configurations for RedCap	1233

A.3.9A.1	Introduction	1233

A.3.9A.2	Downlink BWP configurations	1234

A.3.9A.2.1	Dedicated BWP	1234

A.3.9A.3	Uplink BWP configurations	1234

A.3.9A.3.1	Dedicated BWP	1234

A.3.10	SSB Configurations	1235

A.3.10.1	SSB Configurations for FR1	1235

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1235

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1237

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1237

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1238

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1238

A.3.10.2	SSB Configurations for FR2	1239

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1239

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1239

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1240

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1240

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1241

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1241

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1242

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1242

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1243

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1243

A.3.10A	SSB Configurations under CCA	1248

A.3.10A.1	SSB Configurations under CCA for FR1	1248

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1248

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1248

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1249

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1249

A.3.10B	SSB Configurations for RedCap	1250

A.3.10B.1	SSB Configurations for FR1	1250

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1250

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1250

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1251

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1251

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1252

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1252

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1253

A.3.10B.2	SSB Configurations for FR2	1253

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1253

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1254

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1254

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1255

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1255

A.3.11	SMTC Configurations	1256

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1256

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1256

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1256

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1256

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1257

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1257

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1257

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1257

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1257

A.3.11A	SMTC Configurations for RedCap	1258

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1258

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1258

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1258

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1258

A.3.12	Test Cases with Different CC Configurations	1258

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1258

A.3.12.1.1	Introduction	1258

A.3.12.1.2	Principle of testing	1259

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1259

A.3.12.2.1	Introduction	1259

A.3.12.2.2	Principle of testing	1259

A.3.13	Test Cases in SA and EN-DC Operations	1259

A.3.13.1	Introduction	1259

A.3.13.2	Principle of Testing	1259

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1260

A.3.13A.1	Introduction	1260

A.3.13A.2	Principle of Testing in EN-DC	1260

A.3.13A.3	Principle of Testing in SA	1260

A.3.13A.4	Principle of Testing in E-UTRA	1261

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1261

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1261

A.3.13B.1.1	Introduction	1261

A.3.13B.1.2	Principle of Testing	1261

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1261

A.3.13B.2.1	Introduction	1261

A.3.13B.2.2	Principle of Testing	1261

A.3.14	CSI-RS configurations	1262

A.3.14.1	FDD	1262

A.3.14.2	TDD	1264

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1270

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1270

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1270

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1270

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1271

A.3.15.3	Setup 3: 2 AoAs	1271

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1271

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1271

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1271

A.3.16	TCI State Configuration	1272

A.3.16.1	Introduction	1272

A.3.16.2	TCI states	1272

A.3.16A	Unified TCI State Configuration	1272

A.3.16A.1	Introduction	1272

A.3.16A.2	DLorJoint TCI states	1273

A.3.16A.3	UL TCI states	1273

A.3.17	Configurations of CSI-RS for tracking	1274

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1274

A.3.17.1.1	FDD	1274

A.3.17.1.2	TDD	1276

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1278

A.3.17.2.1	TDD	1278

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1279

A.3.18.1	Introduction	1279

A.3.18.2	PRACH Power Measurement	1279

A.3.19	Test applicability for DAPS handover	1279

A.3.19.1	Introduction	1279

A.3.19.2	Principle of testing	1279

A.3.20	MsgA configurations	1280

A.3.20.1	Introduction	1280

A.3.20.2	MsgA configurations in FR1	1280

A.3.20.2.1	FR1 MsgA configuration 1	1280

A.3.20.2.2	FR1 MsgA configuration 2	1281

A.3.20.3	MsgA configurations in FR2	1283

A.3.20.3.1	FR2 MsgA configuration 1	1283

A.3.20.3.2	FR2 MsgA configuration 2	1284

A.3.20A	MsgA configurations under CCA	1286

A.3.20A.1	Introduction	1286

A.3.20A.2	MsgA configurations in FR1	1286

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1286

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1287

A.3.21	V2X sidelink communication	1290

A.3.21.1	Introduction	1290

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1290

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1294

A.3.21.4	Reference SL-DRX configurations	1295

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1295

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1295

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1295

A.3.22	CSI-IM configurations	1295

A.3.22.1	FDD	1295

A.3.22.2	TDD	1296

A.3.23	Spatial Relation Configuration	1297

A.3.23.1	Introduction	1297

A.3.23.2	Spatial Relation	1298

A.3.24	SRS configuration	1299

A.3.25	Channel bandwidth (CBW) configurations	1301

A.3.25.1	DL UE specific CBW	1301

A.3.25.2	UL UE specific CBW	1302

A.3.26	CCA model	1302

A.3.26.1	Introduction	1302

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1302

A.3.26.2.1	DL CCA model	1302

A.3.26.2.2	UL CCA model	1304

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1304

A.3.26.3.1	DL CCA model	1304

A.3.26.3.2	UL CCA model	1305

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1305

A.3.27.1	Introduction	1305

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1305

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1306

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1306

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1306

A.3.28	Discovery Burst Transmission Window configuration under CCA	1306

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1306

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1306

A.3.29.1	Introduction	1306

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1306

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1307

A.3.30	CSI-RS configurations for RRM	1308

A.3.30.1	FDD	1308

A.3.30.2	TDD	1309

A.3.31	PRS Configurations	1311

A.3.31.1.	PRS Configurations for FR1	1311

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1311

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1312

A.3.31.2.	PRS Configurations for FR2	1312

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1312

A.3.32	NR sidelink discovery	1312

A.3.32.1	Introduction	1312

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1312

A.3.32.3	Principle of Testing	1313

A.3.33	PRS Processing Window (PPW) configurations	1313

A.3.34	Testing principles for test cases related to PRS measurements	1313

A.3.34.1	Introduction	1313

A.3.34.2	Test cases in RRC_INACTIVE state	1314

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1314

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1314

A.3.35	Testing principle for RedCap UE	1315

A.3.35.1	Introduction	1315

A.3.35.2	Principle of testing for FR1	1315

A.3.35.3	Principle of testing for FR2	1315

A.3.36	Testing related to Satellite access	1315

A.3.36.1	Introduction	1315

A.3.36.1	Introduction	1315

A.3.36.2	Principle of testing GSO and NGSO scenarios	1315

A.3.36.2	Principle of testing different RRM requirements	1316

A.3.36.3	Principle of testing different ephemeris formats	1316

A.3.36.4	General setup for SIB19	1318

A.3.36.5	Satellite specific parameters configuration	1319

A.3.36.5.1	Satellite specific configuration for serving cell	1319

A.3.36.5.2	Satellite specific configuration for neighbour cell	1319

A.4	EN-DC tests with all NR cells in FR1	1321

A.4.1	Void	1321

A.4.2	Void	1321

A.4.3	RRC_CONNECTED state mobility	1321

A.4.3.1	Void	1321

A.4.3.2	RRC Connection Mobility Control	1321

A.4.3.2.1	Void	1321

A.4.3.2.2	Random Access	1321

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1321

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1325

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1329

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1333

A.4.3.2.3	Void	1337

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1337

A.4.3.3.1	Test Purpose and Environment	1337

A.4.3.3.2	Test Requirements	1343

A.4.4	Timing	1344

A.4.4.1	UE transmit timing	1344

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1344

A.4.4.1.1.1	Test Purpose and environment	1344

A.4.4.1.1.2	Test requirements	1348

A.4.4.2	UE timer accuracy	1348

A.4.4.3	Timing advance	1348

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1348

A.4.4.3.1.1	Test Purpose and Environment	1348

A.4.4.3.1.2	Test Parameters	1348

A.4.4.3.1.3	Test Requirements	1352

A.4.5	Signaling characteristics	1352

A.4.5.1	Radio link Monitoring	1352

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1353

A.4.5.1.1.1	Test Purpose and Environment	1353

A.4.5.1.1.2	Test Requirements	1357

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1357

A.4.5.1.2.1	Test Purpose and Environment	1357

A.4.5.1.2.2	Test Requirements	1363

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1363

A.4.5.1.3.1	Test Purpose and Environment	1363

A.4.5.1.3.2	Test Requirements	1368

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1368

A.4.5.1.4.1	Test Purpose and Environment	1368

A.4.5.1.4.2	Test Requirements	1374

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1374

A.4.5.1.5.1	Test Purpose and Environment	1374

A.4.5.1.5.2	Test Requirements	1379

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1379

A.4.5.1.6.1	Test Purpose and Environment	1379

A.4.5.1.6.2	Test Requirements	1385

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1385

A.4.5.1.7.1	Test Purpose and Environment	1385

A.4.5.1.7.2	Test Requirements	1390

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1391

A.4.5.1.8.1	Test Purpose and Environment	1391

A.4.5.1.8.2	Test Requirements	1396

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1396

A.4.5.1.9.1	Test Purpose and Environment	1396

A.4.5.2	Interruption	1402

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1402

A.4.5.2.1.1	Test Purpose and Environment	1402

A.4.5.2.1.2	Test Requirements	1406

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1406

A.4.5.2.2.1	Test Purpose and Environment	1406

A.4.5.2.2.2	Test Requirements	1410

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1410

A.4.5.2.3.1	Test Purpose and Environment	1410

A.4.5.2.3.2	Test Requirements	1415

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1416

A.4.5.2.4.1	Test Purpose and Environment	1416

A.4.5.2.4.2	Test Requirements	1420

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1421

A.4.5.2.5.1	Test Purpose and Environment	1421

A.4.5.2.5.2	Test Requirements	1425

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1425

A.4.5.2.6.1	Test Purpose and Environment	1425

A.4.5.2.6.2	Test Requirements	1429

A.4.5.2.7	Void	1429

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1429

A.4.5.2.8.1	Test Purpose and Environment	1429

A.4.5.2.8.2	Test Requirements	1432

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1433

A.4.5.2.9.1	Test Purpose and Environment	1433

A.4.5.2.9.2	Test Requirements	1437

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1438

A.4.5.2.10.1	Test Purpose and Environment	1438

A.4.5.2.10.2	Test Requirements	1441

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1441

A.4.5.2.11.1	Test Purpose and Environment	1441

A.4.5.2.11.2	Test Requirements	1446

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1447

A.4.5.2.12.1	Test Purpose and Environment	1447

A.4.5.3	SCell Activation and Deactivation Delay	1455

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1455

A.4.5.3.1.1	Test Purpose and Environment	1455

A.4.5.3.1.2	Test Requirements	1461

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1462

A.4.5.3.2.1	Test Purpose and Environment	1462

A.4.5.3.2.2	Test Requirements	1462

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1462

A.4.5.3.3.1	Test Purpose and Environment	1462

A.4.5.3.3.2	Test Requirements	1463

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1464

A.4.5.3.4.1	Test Purpose and Environment	1464

A.4.5.3.3.2	Test Requirements	1466

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1466

A.4.5.3.5.1	Test Purpose and Environment	1466

A.4.5.3.5.2	Test Requirements	1474

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1475

A.4.5.3.6.1	Test Purpose and Environment	1475

A.4.5.3.6.2	Test Requirements	1479

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1480

A.4.5.3.7.1	Test Purpose and Environment	1480

A.4.5.3.7.2	Test Requirements	1480

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1481

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1481

A.4.5.4.1.1 Test Purpose and Environment	1481

A.4.5.4.1.2	Test Requirements	1491

A.4.5.5	Beam Failure Detection and Link recovery procedures	1491

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1491

A.4.5.5.1.1	Test Purpose and Environment	1491

A.4.5.5.1.2	Test Requirements	1497

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1498

A.4.5.5.2.1	Test Purpose and Environment	1498

A.4.5.5.2.2	Test Requirements	1503

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1504

A.4.5.5.3.1	Test Purpose and Environment	1504

A.4.5.5.3.2	Test Requirements	1509

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1510

A.4.5.5.4.1	Test Purpose and Environment	1510

A.4.5.5.4.2	Test Requirements	1515

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1516

A.4.5.5.5.1	Test Purpose and Environment	1516

A.4.5.5.5.2	Test Requirements	1521

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1522

A.4.5.5.6.1	Test Purpose and Environment	1522

A.4.5.5.6.2	Test Requirements	1528

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1529

A.4.5.5.7.1	Test Purpose and Environment	1529

A.4.5.5.7.2	Test Requirements	1534

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1535

A.4.5.5.8.1	Test Purpose and Environment	1535

A.4.5.5.8.2	Test Requirements	1541

A.4.5.6	Active BWP switch	1542

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1542

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1542

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1547

A.4.5.6.2	RRC-based Active BWP Switch	1553

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1557

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1557

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1563

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1563

A.4.5.6.4.1.1	Test Purpose and Environment	1563

A.4.5.6.4.1.2	Test Requirements	1570

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1570

A.4.5.6.4.2.1	Test Purpose and Environment	1570

A.4.5.6.4.2.2	Test Requirements	1580

A.4.5.6.5	SCell dormancy switch	1581

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1581

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1586

A.4.5.6.5.2.1	Test Purpose and Environment	1586

A.4.5.6.5.2.2	Test Requirements	1592

A.4.5.7	PSCell addition and release delay	1592

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1592

A.4.5.7.1.1	Test purpose and environment	1592

A.4.5.7.1.2	Test Requirements	1597

A.4.5.8	DL Interruptions at switching between two uplink carriers	1598

A.4.5.8.1	Test Purpose and Environment	1598

A.4.5.8.2	Test Requirements	1601

A.4.5.9	UE specific CBW change	1602

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1602

A.4.5.9.1.1	Test Purpose and Environment	1602

A.4.5.9.1.2	Test Requirements	1606

A.4.5.10	PSCell activation and deactivation delay	1606

A.4.5.10.1	PSCell activation and deactivation delay	1606

A.4.5.10.1.1	Test purpose and environment	1606

A.4.5.10.1.2	Test Requirements	1610

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1611

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1611

A.4.5.11.1.1	Test purpose and environment	1611

A.4.5.11.1.2	Test Parameters	1611

A.4.5.11.1.3	Test Requirements	1615

A.4.6	Measurement procedure	1615

A.4.6.1	Intra-frequency Measurements	1615

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1615

A.4.6.1.1.1	Test purpose and Environment	1615

A.4.6.1.1.2	Test parameters	1615

A.4.6.1.1.3	Test Requirements	1619

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1619

A.4.6.1.2.1	Test purpose and Environment	1619

A.4.6.1.2.2	Test parameters	1619

A.4.6.1.2.2	Test Requirements	1623

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1623

A.4.6.1.3.1	Test purpose and Environment	1623

A.4.6.1.3.2	Test parameters	1623

A.4.6.1.3.3	Test Requirements	1627

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1627

A.4.6.1.4.1	Test purpose and Environment	1627

A.4.6.1.4.2	Test parameters	1627

A.4.6.1.4.3	Test Requirements	1631

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1631

A.4.6.1.5.1	Test purpose and Environment	1631

A.4.6.1.5.2	Test parameters	1631

A.4.6.1.5.3	Test Requirements	1633

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1634

A.4.6.1.6.1	Test purpose and Environment	1634

A.4.6.1.6.2	Test parameters	1634

A.4.6.1.6.3	Test Requirements	1636

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1637

A.4.6.1.7.1	Test purpose and Environment	1637

A.4.6.1.7.2	Test parameters	1637

A.4.6.1.7.3	Test Requirements	1641

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1641

A.4.6.1.8.1	Test Purpose and Environment	1641

A.4.6.1.8.2	Test Requirements	1646

A.4.6.2	Inter-frequency Measurements	1647

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1647

A.4.6.2.1.1	Test Purpose and Environment	1647

A.4.6.2.1.2	Test Requirements	1651

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1651

A.4.6.2.2.1	Test Purpose and Environment	1651

A.4.6.2.2.2	Test Requirements	1655

A.4.6.2.3	Void	1655

A.4.6.2.4	Void	1655

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1655

A.4.6.2.5.1	Test Purpose and Environment	1655

A.4.6.2.5.2	Test Requirements	1660

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1660

A.4.6.2.6.1	Test Purpose and Environment	1660

A.4.6.2.6.2	Test Requirements	1665

A.4.6.2.7	Void	1665

A.4.6.2.8	Void	1665

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1666

A.4.6.2.9.1	Test Purpose and Environment	1666

A.4.6.2.9.2	Test Requirements	1669

A.4.6.3	Void	1669

A.4.6.4	L1-RSRP measurement for beam reporting	1669

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1669

A.4.6.4.1.1	Test Purpose and Environment	1669

A.4.6.4.1.2	Test parameters	1670

A.4.6.4.1.3	Test Requirements	1673

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1673

A.4.6.4.2.1	Test Purpose and Environment	1673

A.4.6.4.2.2	Test parameters	1674

A.4.6.4.2.3	Test Requirements	1677

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1677

A.4.6.4.3.1	Test Purpose and Environment	1677

A.4.6.4.3.2	Test parameters	1678

A.4.6.4.3.3	Test Requirements	1681

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1681

A.4.6.4.4.1	Test Purpose and Environment	1681

A.4.6.4.4.2	Test parameters	1682

A.4.6.4.4.3	Test Requirements	1684

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1684

A.4.6.4.5.1	Test Purpose and Environment	1684

A.4.6.4.5.2	Test parameters	1685

A.4.6.4.5.3	Test Requirements	1688

A.4.6.5	CLI measurements	1688

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1688

A.4.6.5.1.1	Test Purpose and Environment	1688

A.4.6.5.1.2	Test Parameters	1689

A.4.6.5.1.3	Test Requirements	1692

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1692

A.4.6.5.2.1	Test Purpose and Environment	1692

A.4.6.5.2.2	Test Parameters	1693

A.4.6.5.2.3	Test Requirements	1694

A.4.6.6.1.2	Test Requirements	1699

A.4.6.7	L1-SINR measurement for beam reporting	1699

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1702

A.4.6.7.2.1	Test Purpose and Environment	1702

A.4.6.7.2.2	Test parameters	1702

A.4.6.7.2.3	Test Requirements	1705

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1705

A.4.6.7.3.1	Test Purpose and Environment	1705

A.4.6.7.3.2	Test parameters	1705

A.4.6.7.3.3	Test Requirements	1707

A.4.6.8	CSI-RS based intra-frequency Measurement	1708

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1708

A.4.6.8.1.1	Test purpose and Environment	1708

A.4.6.8.1.2	Test Requirements	1712

A.4.6.9	CSI-RS based inter-frequency Measurement	1712

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1712

A.4.6.9.1.1	Test Purpose and Environment	1712

A.4.6.9.1.2	Test Requirements	1716

A.4.7	Measurement Performance requirements	1717

A.4.7.1	SS-RSRP	1717

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1717

A.4.7.1.1.1	Test Purpose and Environment	1717

A.4.7.1.1.2	Test parameters	1717

A.4.7.1.1.3	Test Requirements	1721

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1721

A.4.7.1.2.1	Test Purpose and Environment	1721

A.4.7.1.2.2	Test parameters	1722

A.4.7.1.2.3	Test Requirements	1726

A.4.7.1.3	Void	1727

A.4.7.2	SS-RSRQ	1727

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1727

A.4.7.2.1.1	Test Purpose and Environment	1727

A.4.7.2.1.2	Test Parameters	1727

A.4.7.2.1.3	Test Requirements	1732

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1732

A.4.7.2.2.1	Test Purpose and Environment	1732

A.4.7.2.2.2	Test Parameters	1732

A.4.7.2.2.3	Test Requirements	1738

A.4.7.3	SS-SINR	1738

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1738

A.4.7.3.1.1	Test Purpose and Environment	1738

A.4.7.3.1.2	Test Parameters	1738

A.4.7.3.1.3	Test Requirements	1742

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1743

A.4.7.3.2.1	Test Purpose and Environment	1743

A.4.7.3.2.2	Test Parameters	1743

A.4.7.3.2.3	Test Requirements	1747

A.4.7.4	L1-RSRP measurement for beam reporting	1748

A.4.7.4.1	SSB based L1-RSRP measurement	1748

A.4.7.4.1.1	Test Purpose and Environment	1748

A.4.7.4.1.2	Test parameters	1748

A.4.7.4.1.3	Test Requirements	1751

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1751

A.4.7.4.2.1	Test Purpose and Environment	1751

A.4.7.4.2.2	Test parameters	1752

A.4.7.4.2.3	Test Requirements	1755

A.4.7.5	SFTD accuracy	1756

A.4.7.5.1	SFTD accuracy	1756

A.4.7.5.1.1	Test Purpose and Environment	1756

A.4.7.5.1.2	Test Parameters	1756

A.4.7.5.1.3	Test Requirements	1760

A.4.7.5.2	Void	1760

A.4.7.5.3	Void	1760

A.4.7.6	CLI measurements	1760

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1760

A.4.7.6.1.1	Test Purpose and Environment	1760

A.4.7.6.1.2	Test parameters	1761

A.4.7.6.1.3	Test Requirements	1766

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1766

A.4.7.6.2.1	Test Purpose and Environment	1766

A.4.7.6.2.2	Test parameters	1767

A.4.7.6.2.3	Test Requirements	1770

A.4.7.7	L1-SINR measurement for beam reporting	1770

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1774

A.4.7.7.2.1	Test Purpose and Environment	1774

A.4.7.7.2.2	Test parameters	1774

A.4.7.7.2.3	Test Requirements	1778

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1778

A.4.7.7.3.1	Test Purpose and Environment	1778

A.4.7.7.3.2	Test parameters	1778

A.4.7.7.3.3	Test Requirements	1781

A.4.7.8	CSI-RSRP	1782

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1782

A.4.7.8.1.1	Test Purpose and Environment	1782

A.4.7.8.1.2	Test parameters	1782

A.4.7.8.1.3	Test Requirements	1787

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1787

A.4.7.8.2.1	Test Purpose and Environment	1787

A.4.7.8.2.2	Test parameters	1787

A.4.7.8.2.3	Test Requirements	1792

A.4.7.9	CSI-RSRQ	1792

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1792

A.4.7.9.1.1	Test Purpose and Environment	1792

A.4.7.9.1.2	Test Parameters	1792

A.4.7.9.1.3	Test Requirements	1798

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1798

A.4.7.9.2.1	Test Purpose and Environment	1798

A.4.7.9.2.2	Test Parameters	1798

A.4.7.9.2.3	Test Requirements	1803

A.4.7.10	CSI-SINR	1804

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1804

A.4.7.10.1.1	Test Purpose and Environment	1804

A.4.7.10.1.2	Test Parameters	1804

A.4.7.10.1.3	Test Requirements	1808

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1809

A.4.7.10.2.1	Test Purpose and Environment	1809

A.4.7.10.2.2	Test Parameters	1809

A.4.7.10.2.3	Test Requirements	1814

A.4.8	Void	1814

A.4A	NE-DC test with all NR cells in FR1	1814

A.4A.1	Signaling characteristics	1814

A.4A.1.1	E-UTRAN PSCell addition	1814

A.4A.1.1.1	Test purpose and environment	1814

A.4A.1.1.2	Test Requirements	1818

A.4A.1.2	Active BWP switch	1819

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1819

A.4A.1.2.1.1	Test Purpose and Environment	1819

A.4A.1.2.1.2	Test Requirements	1823

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1824

A.4A.1.3.1	Test purpose and environment	1824

A.4A.1.3.2	Test Requirements	1828

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1829

A.4A.1.4.1	Test Purpose and Environment	1829

A.4A.1.4.2	Test Parameters	1829

A.4A.1.4.3	Test Requirements	1834

A.4A.1.4.3.1	Test Requirements for NR HO	1834

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1834

A.4A.2	Measurement performance	1835

A.4A.2.1	SFTD accuracy	1835

A.4A.2.1.1	SFTD accuracy	1835

A.4A.2.1.1.1	Test Purpose	1835

A.4A.2.1.1.2	Test Environment	1835

A.4A.2.1.1.3	Test Requirements	1839

A.5	EN-DC tests with one or more NR cells in FR2	1839

A.5.1	Void	1839

A.5.2	Void	1839

A.5.3	RRC_CONNECTED state mobility	1839

A.5.3.1	Void	1839

A.5.3.2	RRC Connection Mobility Control	1839

A.5.3.2.1	Void	1839

A.5.3.2.2	Random Access	1839

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1839

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1843

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1849

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1852

A.5.3.2.3	Void	1856

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1856

A.5.3.3.1	Test purpose and environment	1856

A.5.3.3.2	Test Requirements	1860

A.5.5.3.3	PUCCH SCell Activation and deactivation of known SCell in FR2	1861

A.5.5.3.3.1	Test Purpose and Environment	1861

A.5.5.3.3.2	Test Requirements	1864

A.5.5.3.4	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1865

A.5.5.3.4.1	Test Purpose and Environment	1865

A.5.5.3.4.2	Test Requirements	1868

A.5.5.3.5	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1869

A.5.5.3.5.1	Test Purpose and Environment	1869

A.5.5.3.5.2	Test Requirements	1872

A.5.5.3.6	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1873

A.5.5.3.6.1	Test Purpose and Environment	1873

A.5.5.3.6.2	Test Requirements	1876

A.5.4	Timing	1877

A.5.4.1	UE transmit timing	1877

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1877

A.5.4.1.1.1	Test Purpose and environment	1877

A.5.4.1.1.2	Test requirements	1880

A.5.4.2	UE timer accuracy	1881

A.5.4.3	Timing advance	1881

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1881

A.5.4.3.1.1 Test Purpose and Environment	1881

A.5.4.3.1.2 Test Parameters	1881

A.5.4.3.1.3	Test Requirements	1885

A.5.5	Signaling characteristics	1885

A.5.5.1	Radio link Monitoring	1885

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1885

A.5.5.1.1.1	Test Purpose and Environment	1885

A.5.5.1.1.2	Test Requirements	1889

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1890

A.5.5.1.2.1	Test Purpose and Environment	1890

A.5.5.1.2.2	Test Requirements	1893

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1894

A.5.5.1.3.1	Test Purpose and Environment	1894

A.5.5.1.3.2	Test Requirements	1898

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1898

A.5.5.1.4.1	Test Purpose and Environment	1898

A.5.5.1.4.2	Test Requirements	1902

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1902

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1906

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1910

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1915

A.5.5.1.8.2	Test Requirements	1919

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1920

A.5.5.1.9.1	Test Purpose and Environment	1920

A.5.5.1.9.2	Test Requirements	1922

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1922

A.5.5.1.10.1	Test Purpose and Environment	1922

A.5.5.1.10.2	Test Requirements	1925

A.5.5.2	Interruption	1925

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1925

A.5.5.2.1.1	Test Purpose and Environment	1925

A.5.5.2.1.2	Test Requirements	1928

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1928

A.5.5.2.2.1	Test Purpose and Environment	1928

A.5.5.2.2.2	Test Requirements	1931

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1931

A.5.5.2.3.1	Test Purpose and Environment	1931

A.5.5.2.3.2	Test Requirements	1935

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1935

A.5.5.2.4.1	Test Purpose and Environment	1935

A.5.5.2.4.2	Test Requirements	1938

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1939

A.5.5.2.5.1	Test Purpose and Environment	1939

A.5.5.2.5.2	Test Requirements	1942

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1942

A.5.5.2.6.1	Test Purpose and Environment	1942

A.5.5.2.6.2	Test Requirements	1945

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1945

A.5.5.2.7.1	Test Purpose and Environment	1945

A.5.5.2.7.2	Test Requirements	1949

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1949

A.5.5.2.8.1 Test Purpose and Environment	1949

A.5.5.2.8.3	Test Requirements	1951

A.5.5.3	SCell Activation and Deactivation Delay	1951

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1951

A.5.5.3.1.1	Test Purpose and Environment	1951

A.5.5.3.1.2	Test Requirements	1953

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1953

A.5.5.3.2.1	Test Purpose and Environment	1953

A.5.5.3.2.2	Test Requirements	1957

A.5.5.3.3	Void	1957

A.5.5.3.4	Void	1957

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1957

A.5.5.3.5.1	Test Purpose and Environment	1957

A.5.5.3.5.2	Test Requirements	1960

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1961

A.5.5.3.6.1	Test Purpose and Environment	1961

A.5.5.3.6.2	Test Requirements	1965

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1966

A.5.5.3.7.1	Test Purpose and Environment	1966

A.5.5.3.7.2	Test Requirements	1969

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1969

A.5.5.3.8.1	Test Purpose and Environment	1969

A.5.5.3.8.2	Test Requirements	1973

A.5.5.4	Void	1974

A.5.5.5	Beam Failure Detection and Link recovery procedures	1974

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1974

A.5.5.5.1.1	Test Purpose and Environment	1974

A.5.5.5.1.2	Test Requirements	1978

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1979

A.5.5.5.2.1	Test Purpose and Environment	1979

A.5.5.5.2.2	Test Requirements	1983

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1984

A.5.5.5.3.1	Test Purpose and Environment	1984

A.5.5.5.3.2	Test Requirements	1988

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1989

A.5.5.5.4.1	Test Purpose and Environment	1989

A.5.5.5.4.2	Test Requirements	1993

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1994

A.5.5.5.5.1	Test Purpose and Environment	1994

A.5.5.5.5.2	Test Requirements	1998

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1999

A.5.5.5.6.1	Test Purpose and Environment	1999

A.5.5.5.6.2	Test Requirements	2003

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	2004

A.5.5.5.7.1	Test Purpose and Environment	2004

A.5.5.5.7.2	Test Requirements	2008

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2009

A.5.5.5.8.1	Test Purpose and Environment	2009

A.5.5.5.8.2	Test Requirements	2014

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	2014

A.5.5.5.9.1	Test Purpose and Environment	2014

A.5.5.5.9.2	Test Requirements	2018

A.5.5.6	Active BWP switch	2018

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	2018

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2018

A.5.5.6.1.1.1	Test Purpose and Environment	2018

A.5.5.6.1.1.2	Test Requirements	2022

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	2022

A.5.5.6.2	RRC-based Active BWP Switch	2027

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2027

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2031

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	2031

A.5.5.6.4	SCell dormancy switch	2035

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	2035

A.5.5.6.4.1.1	Test Purpose and Environment	2035

A.5.5.6.4.1.2	Test Requirements	2039

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	2039

A.5.5.6.4.2.1	Test Purpose and Environment	2039

A.5.5.6.4.2.2	Test Requirements	2046

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2046

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	2046

A.5.5.7	PSCell addition and release delay	2049

A.5.5.7.1	Addition and Release Delay of NR PSCell	2049

A.5.5.7.1.1	Test purpose and environment	2049

A.5.5.7.1.2	Test Requirements	2052

A.5.5.8	Active TCI state switch delay	2053

A.5.5.8.1	MAC-CE based active TCI state switch	2053

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2053

A.5.5.8.1.1.1	Test Purpose and Environment	2053

A.5.5.8.1.1.2	Test Requirements	2056

A.5.5.8.2	RRC based active TCI state switch	2056

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2056

A.5.5.8.2.1.1	Test Purpose and Environment	2056

A.5.5.8.2.1.2	Test Requirements	2060

A.5.5.9	Uplink spatial relation switch delay	2060

A.5.5.9.1	MAC-CE based uplink spatial relation switch	2060

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	2060

A.5.5.9.1.1.1	Test Purpose and Environment	2060

A.5.5.9.1.1.2	Test Requirements	2063

A.5.5.9.2	RRC based spatial relation switch	2063

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	2063

A.5.5.9.2.1.1	Test Purpose and Environment	2063

A.5.5.9.2.1.2	Test Requirements	2066

A.5.5.10	UE specific CBW change	2066

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	2066

A.5.5.10.1.1	Test Purpose and Environment	2066

A.5.5.10.1.2	Test Requirements	2069

A.5.5.11	Unified TCI state switch delay	2070

A.5.5.11.1	MAC-CE based active joint TCI state switch	2070

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	2070

A.5.5.11.1.1.1	Test Purpose and Environment	2070

A.5.5.11.1.1.2	Test parameters	2070

A.5.5.11.1.1.3	Test Requirements	2073

A.5.5.11.2	MAC-CE based active uplink TCI state switch	2073

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	2073

A.5.5.11.2.1.1	Test Purpose and Environment	2073

A.5.5.11.2.1.2	Test parameters	2074

A.5.5.11.2.1.3	Test Requirements	2076

A.5.5.11.3	MAC-CE based active downlink TCI state switch	2076

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	2076

A.5.5.11.3.1.1	Test Purpose and Environment	2076

A.5.5.11.3.1.2	Test Parameters	2077

A.5.5.11.3.1.3	Test Requirements	2081

A.5.5.12	PSCell activation and deactivation delay	2081

A.5.5.12.1	PSCell activation and deactivation delay	2081

A.5.5.12.1.1	Test purpose and environment	2081

A.5.5.12.1.2	Test Requirements	2084

A.5.5.13	Conditional PSCell addition and release delay	2085

A.5.5.13.1	Addition and Release Delay of NR PSCell	2085

A.5.5.13.1.1	Test purpose and environment	2085

A.5.5.13.1.2	Test Requirements	2088

A.5.6	Measurement procedure	2089

A.5.6.1	Intra-frequency Measurements	2089

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	2089

A.5.6.1.1.1	Test purpose and Environment	2089

A.5.6.1.1.2	Test Requirements	2092

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	2092

A.5.6.1.2.1	Test purpose and Environment	2092

A.5.6.1.2.2	Test Requirements	2094

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	2095

A.5.6.1.3.1	Test purpose and Environment	2095

A.5.6.1.3.2	Test Requirements	2099

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	2099

A.5.6.1.4.1	Test purpose and Environment	2099

A.5.6.1.4.2	Test Requirements	2102

A.5.6.2	Inter-frequency Measurements	2103

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2103

A.5.6.2.1.1	Test Purpose and Environment	2103

A.5.6.2.1.2	Test Requirements	2106

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2106

A.5.6.2.2.1	Test Purpose and Environment	2106

A.5.6.2.2.2	Test Requirements	2110

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2110

A.5.6.2.3.1	Test Purpose and Environment	2110

A.5.6.2.3.2	Test Requirements	2114

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2114

A.5.6.2.4.1	Test Purpose and Environment	2114

A.5.6.2.4.2	Test Requirements	2118

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2118

A.5.6.2.5.1	Test Purpose and Environment	2118

A.5.6.2.5.2	Test Requirements	2123

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2123

A.5.6.2.6.1	Test Purpose and Environment	2123

A.5.6.2.6.2	Test Requirements	2127

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2127

A.5.6.2.7.1	Test Purpose and Environment	2127

A.5.6.2.7.2	Test Requirements	2132

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2132

A.5.6.2.8.1	Test Purpose and Environment	2132

A.5.6.2.8.2	Test Requirements	2137

A.5.6.3	L1-RSRP measurement for beam reporting	2138

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2138

A.5.6.3.1.1	Test Purpose and Environment	2138

A.5.6.3.1.2	Test parameters	2138

A.5.6.3.1.3	Test Requirements	2140

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2140

A.5.6.3.2.1	Test Purpose and Environment	2140

A.5.6.3.2.2	Test parameters	2141

A.5.6.3.2.3	Test Requirements	2143

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2143

A.5.6.3.3.1	Test Purpose and Environment	2143

A.5.6.3.3.2	Test parameters	2144

A.5.6.3.3.3	Test Requirements	2146

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2146

A.5.6.3.4.1	Test Purpose and Environment	2146

A.5.6.3.4.2	Test parameters	2147

A.5.6.3.4.3	Test Requirements	2149

A.5.6.4	CLI measurements	2150

A.5.6.4.1	SRS-RSRP measurement with DRX	2150

A.5.6.4.1.1	Test Purpose and Environment	2150

A.5.6.4.1.2	Test Parameters	2150

A.5.6.4.1.3	Test Requirements	2152

A.5.6.4.2	CLI-RSSI measurement with DRX	2152

A.5.6.4.2.1	Test Purpose and Environment	2152

A.5.6.4.2.2	Test Parameters	2153

A.5.6.4.2.3	Test Requirements	2155

A.5.6.5	Measurements with autonomous gaps	2155

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2155

A.5.6.5.1.1	Test Purpose and Environment	2155

A.5.6.5.1.2	Test Requirements	2158

A.5.6.6	L1-SINR measurement for beam reporting	2159

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2161

A.5.6.6.2.1	Test Purpose and Environment	2161

A.5.6.6.2.2	Test parameters	2162

A.5.6.6.2.3	Test Requirements	2165

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2165

A.5.6.6.3.1	Test Purpose and Environment	2165

A.5.6.6.3.2	Test parameters	2166

A.5.6.6.3.3	Test Requirements	2168

A.5.6.7	CSI-RS based Intra-frequency Measurements	2168

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2168

A.5.6.7.1.1	Test purpose and Environment	2168

A.5.6.7.1.2	Test Requirements	2171

A.5.6.8	CSI-RS based Inter-frequency Measurements	2171

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2171

A.5.6.8.1.1	Test Purpose and Environment	2171

A.5.6.8.1.2	Test Requirements	2176

A.5.7	Measurement Performance requirements	2176

A.5.7.1	SS-RSRP	2176

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2176

A.5.7.1.1.1	Test Purpose and Environment	2176

A.5.7.1.1.2	Test parameters	2177

A.5.7.1.1.3	Test Requirements	2179

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2180

A.5.7.1.2.1	Test Purpose and Environment	2180

A.5.7.1.2.2	Test parameters	2180

A.5.7.1.2.3	Test Requirements	2184

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2185

A.5.7.1.3.1	Test Purpose and Environment	2185

A.5.7.1.3.2	Test parameters	2185

A.5.7.1.3.3	Test Requirements	2188

A.5.7.2	SS-RSRQ	2189

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2189

A.5.7.2.1.1	Test Purpose and Environment	2189

A.5.7.2.1.2	Test Parameters	2189

A.5.7.2.1.3	Test Requirements	2192

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2192

A.5.7.2.2.1	Test Purpose and Environment	2192

A.5.7.2.2.2	Test Parameters	2192

A.5.7.2.2.3	Test Requirements	2194

A.5.7.3	SS-SINR	2195

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2195

A.5.7.3.1.1	Test Purpose and Environment	2195

A.5.7.3.1.2	Test Parameters	2195

A.5.7.3.1.3	Test Requirements	2198

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2198

A.5.7.3.2.1	Test Purpose and Environment	2198

A.5.7.3.2.2	Test Parameters	2198

A.5.7.3.2.3	Test Requirements	2200

A.5.7.4	L1-RSRP measurement for beam reporting	2200

A.5.7.4.1	SSB based L1-RSRP measurement	2200

A.5.7.4.1.1	Test Purpose and Environment	2200

A.5.7.4.1.2	Test parameters	2201

A.5.7.4.1.3	Test Requirements	2203

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2204

A.5.7.4.2.1	Test Purpose and Environment	2204

A.5.7.4.2.2	Test parameters	2204

A.5.7.4.2.3	Test Requirements	2206

A.5.7.5	CLI measurements	2207

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2207

A.5.7.5.1.1	Test Purpose and Environment	2207

A.5.7.5.1.2	Test parameters	2207

A.5.7.5.1.3	Test Requirements	2210

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2211

A.5.7.5.2.1	Test Purpose and Environment	2211

A.5.7.5.2.2	Test parameters	2211

A.5.7.5.2.3	Test Requirements	2213

A.5.7.6	L1-SINR measurement for beam reporting	2214

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2217

A.5.7.6.2.1	Test Purpose and Environment	2217

A.5.7.6.2.2	Test parameters	2217

A.5.7.6.2.3	Test Requirements	2219

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2220

A.5.7.6.3.1	Test Purpose and Environment	2220

A.5.7.6.3.2	Test parameters	2220

A.5.7.6.3.3	Test Requirements	2222

A.5.7.7	CSI-RSRP	2223

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2223

A.5.7.7.1.1	Test Purpose and Environment	2223

A.5.7.7.1.2	Test parameters	2223

A.5.7.7.1.3	Test Requirements	2225

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2226

A.5.7.7.2.1	Test Purpose and Environment	2226

A.5.7.7.2.2	Test parameters	2226

A.5.7.7.2.3	Test Requirements	2230

A.5.7.8	CSI-RSRQ	2231

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2231

A.5.7.8.1.1	Test Purpose and Environment	2231

A.5.7.8.1.2	Test Parameters	2231

A.5.7.8.1.3	Test Requirements	2233

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2233

A.5.7.8.2.1	Test Purpose and Environment	2233

A.5.7.8.2.2	Test Parameters	2233

A.5.7.8.2.3	Test Requirements	2235

A.5.7.9	CSI-SINR	2235

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2235

A.5.7.9.1.1	Test Purpose and Environment	2235

A.5.7.9.1.2	Test Parameters	2235

A.5.7.9.1.3	Test Requirements	2238

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2238

A.5.7.9.2.1	Test Purpose and Environment	2238

A.5.7.9.2.2	Test Parameters	2238

A.5.7.9.2.3	Test Requirements	2241

A.5.8	Void	2241

A.6	NR standalone tests with all NR cells in FR1	2242

A.6.1	SA: RRC_IDLE state mobility	2242

A.6.1.1	Cell re-selection to NR	2242

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2242

A.6.1.1.1.1	Test Purpose and Environment	2242

A.6.1.1.1.2	Test Parameters	2242

A.6.1.1.1.3	Test Requirements	2246

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2246

A.6.1.1.2.1	Test Purpose and Environment	2246

A.6.1.1.2.2	Test Parameters	2246

A.6.1.1.2.3	Test Requirements	2250

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2250

A.6.1.1.3.1	Test Purpose and Environment	2250

A.6.1.1.3.2	Test Parameters	2250

A.6.1.1.3.3	Test Requirements	2255

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2255

A.6.1.1.4.1	Test Purpose and Environment	2255

A.6.1.1.4.2	Test Parameters	2255

A.6.1.1.4.3	Test Requirements	2258

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2258

A.6.1.1.5.1	Test Purpose and Environment	2258

A.6.1.1.5.2	Test Parameters	2258

A.6.1.1.5.3	Test Requirements	2263

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2263

A.6.1.1.6.1	Test Purpose and Environment	2263

A.6.1.1.6.2	Test Parameters	2264

A.6.1.1.6.3	Test Requirements	2268

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2268

A.6.1.1.7.1	Test Purpose and Environment	2268

A.6.1.1.7.2	Test Parameters	2269

A.6.1.1.7.3	Test Requirements	2272

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2272

A.6.1.1.8.1	Test Purpose and Environment	2272

A.6.1.1.8.2	Test Parameters	2272

A.6.1.1.8.3	Test Requirements	2276

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2276

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2276

A.6.1.2.1.1	Test Purpose and Environment	2276

A.6.1.2.1.2	Test Parameters	2276

A.6.1.2.1.3	Test Requirements	2279

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2280

A.6.1.2.2.1	Test Purpose and Environment	2280

A.6.1.2.2.2	Test Parameters	2280

A.6.1.2.2.3	Test Requirements	2283

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2284

A.6.1.2.3.1	Test Purpose and Environment	2284

A.6.1.2.3.2	Test Parameters	2284

A.6.1.2.3.3	Test Requirements	2287

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2288

A.6.1.2.4.1	Test Purpose and Environment	2288

A.6.1.2.4.2	Test Parameters	2288

A.6.1.2.4.3	Test Requirements	2291

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2292

A.6.1.2.5.1	Test Purpose and Environment	2292

A.6.1.2.5.2	Test Parameters	2292

A.6.1.2.5.3	Test Requirements	2296

A.6.1.1.7	Void	2297

A.6.2	SA: RRC_INACTIVE state mobility	2297

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2297

A.6.2.1.1	Test purpose and Environment	2297

A.6.2.1.2	Test Parameters	2299

A.6.2.1.3	Test requirements	2301

A.6.3	RRC_CONNECTED state mobility	2301

A.6.3.1	Handover	2301

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2301

A.6.3.1.1.1	Test Purpose and Environment	2301

A.6.3.1.1.2	Test Parameters	2301

A.6.3.1.1.3 Test Requirements	2305

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2305

A.6.3.1.2.1	Test Purpose and Environment	2305

A.6.3.1.2.2	Test Parameters	2305

A.6.3.1.2.3	Test Requirements	2309

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2309

A.6.3.1.3.1	Test Purpose and Environment	2309

A.6.3.1.3.2	Test Parameters	2309

A.6.3.1.3.3	Test Requirements	2313

A.6.3.1.4	SA NR - E-UTRAN handover	2313

A.6.3.1.4.1	Test Purpose and Environment	2313

A.6.3.1.4.2	Test Requirements	2319

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2319

A.6.3.1.5.1	Test Purpose and Environment	2319

A.6.3.1.5.2	Test Requirements	2325

A.6.3.1.6	 SA NR - UTRAN FDD handover	2325

A.6.3.1.6.1	Test Purpose and Environment	2325

A.6.3.1.6.2	Test Requirements	2329

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2329

A.6.3.1.7.1	Test Purpose and Environment	2329

A.6.3.1.7.2	Test Parameters	2329

A.6.3.1.7.3	Test Requirements	2333

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2334

A.6.3.1.8.1	Test Purpose and Environment	2334

A.6.3.1.8.2	Test Parameters	2334

A.6.3.1.8.3	Test Requirements	2338

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2339

A.6.3.1.9.1	Test Purpose and Environment	2339

A.6.3.1.9.2	Test Parameters	2339

A.6.3.1.9.3	Test Requirements	2343

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2343

A.6.3.1.10.1	Test Purpose and Environment	2343

A.6.3.1.10.2	Test Parameters	2343

A.6.3.1.10.3	Test Requirements	2346

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2346

A.6.3.1.11.1	Test Purpose and Environment	2346

A.6.3.1.11.2	Test Parameters	2346

A.6.3.1.11.3 Test Requirements	2353

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2353

A.6.3.1.12.1	Test Purpose and Environment	2353

A.6.3.1.12.2	Test Parameters	2353

A.6.3.1.12.3 Test Requirements	2361

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2361

A.6.3.1.13.1	Test Purpose and Environment	2361

A.6.3.1.13.2	Test Requirements	2370

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2370

A.6.3.1.14.1	Test Purpose and Environment	2370

A.6.3.1.14.2	Test Requirements	2379

A.6.3.2	RRC Connection Mobility Control	2380

A.6.3.2.1	SA: RRC Re-establishment	2380

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2380

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2384

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2388

A.6.3.2.2	Random Access	2391

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2391

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2395

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2400

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2405

A.6.3.2.3	SA: RRC Connection Release with Redirection	2409

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2409

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2413

A.6.3.3 Conditional handover	2420

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2420

A.6.3.3.1.1	Test Purpose and Environment	2420

A.6.3.3.1.2	Test Parameters	2420

A.6.3.3.1.3 Test Requirements	2424

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2424

A.6.3.3.2.1	Test Purpose and Environment	2424

A.6.3.3.2.2	Test Parameters	2424

A.6.3.3.2.3 Test Requirements	2428

A.6.4	Timing	2428

A.6.4.1	UE transmit timing	2428

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2428

A.6.4.1.1.1	Test Purpose and environment	2428

A.6.4.1.1.2	Test requirements	2432

A.6.4.2	UE timer accuracy	2432

A.6.4.3	Timing advance	2432

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2432

A.6.4.3.1.1	Test Purpose and Environment	2432

A.6.4.3.1.2	Test Parameters	2432

A.6.4.3.1.3	Test Requirements	2436

A.6.5	Signalling characteristics	2436

A.6.5.1	Radio link Monitoring	2436

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2437

A.6.5.1.1.1	Test Purpose and Environment	2437

A.6.5.1.1.2	Test Requirements	2442

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2442

A.6.5.1.2.1	Test Purpose and Environment	2442

A.6.5.1.2.2	Test Requirements	2448

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2448

A.6.5.1.3.1	Test Purpose and Environment	2448

A.6.5.1.3.2	Test Requirements	2454

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2454

A.6.5.1.4.1	Test Purpose and Environment	2454

A.6.5.1.4.2	Test Requirements	2460

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2460

A.6.5.1.5.1	Test Purpose and Environment	2460

A.6.5.1.5.2	Test Requirements	2466

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2466

A.6.5.1.6.1	Test Purpose and Environment	2466

A.6.5.1.6.2	Test Requirements	2471

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2471

A.6.5.1.7.1	Test Purpose and Environment	2471

A.6.5.1.7.2	Test Requirements	2476

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2476

A.6.5.1.8.1	Test Purpose and Environment	2476

A.6.5.1.8.2	Test Requirements	2481

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2481

A.6.5.1.9.1	Test Purpose and Environment	2481

A.6.5.1.9.2	Test Requirements	2487

A.6.5.2	Interruption	2487

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2487

A.6.5.2.1.2	Test Requirements	2491

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2492

A.6.5.2.2.1	Test Purpose and Environment	2492

A.6.5.2.2.2	Test Parameters	2492

A.6.5.2.2.3	Test Requirements	2494

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2495

A.6.5.2.3.1	Test Purpose and Environment	2495

A.6.5.2.3.2	Test Parameters	2495

A.6.5.2.3.3	Test Requirements	2497

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2498

A.6.5.2.4.1	Test Purpose and Environment	2498

A.6.5.2.4.2	Test Parameters	2498

A.6.5.2.4.3	Test Requirements	2500

A.6.5.3	SCell Activation and Deactivation Delay	2501

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2501

A.6.5.3.1.1	Test Purpose and Environment	2501

A.6.5.3.1.2	Test Requirements	2506

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2507

A.6.5.3.2.1	Test Purpose and Environment	2507

A.6.5.3.2.2	Test Requirements	2507

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2507

A.6.5.3.3.1	Test Purpose and Environment	2507

A.6.5.3.3.2	Test Requirements	2508

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2508

A.6.5.3.4.1	Test Purpose and Environment	2508

A.6.5.3.4.2	Test Requirements	2516

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2516

A.6.5.3.5.1	Test Purpose and Environment	2516

A.6.5.3.5.2	Test Requirements	2522

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2523

A.6.5.3.6.1	Test Purpose and Environment	2523

A.6.5.3.6.2	Test Requirements	2527

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2527

A.6.5.3.7.1	Test Purpose and Environment	2527

A.6.5.3.7.2	Test Requirements	2531

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2532

A.6.5.3.8.1	Test Purpose and Environment	2532

A.6.5.3.8.2	Test Requirements	2536

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2537

A.6.5.3.9.1	Test Purpose and Environment	2537

A.6.5.3.9.2	Test Requirements	2541

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2542

A.6.5.3.10.1	Test Purpose and Environment	2542

A.6.5.3.10.2	Test Requirements	2545

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2545

A.6.5.3.11.1	Test Purpose and Environment	2545

A.6.5.3.11.2	Test Requirements	2546

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2546

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2546

A.6.5.4.1.1	Test Purpose and Environment	2546

A.6.5.4.1.2	Test Requirements	2556

A.6.5.4.2	Void	2557

A.6.5.5	Beam Failure Detection and Link recovery procedures	2557

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2557

A.6.5.5.1.1	Test Purpose and Environment	2557

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2563

A.6.5.5.2.1	Test Purpose and Environment	2563

A.6.5.5.2.2	Test Requirements	2569

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2570

A.6.5.5.3.1	Test Purpose and Environment	2570

A.6.5.5.3.2	Test Requirements	2575

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2576

A.6.5.5.4.1	Test Purpose and Environment	2576

A.6.5.5.4.2	Test Requirements	2581

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2582

A.6.5.5.5.1	Test Purpose and Environment	2582

A.6.5.5.5.2	Test Requirements	2586

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2586

A.6.5.5.6.1	Test Purpose and Environment	2586

A.6.5.5.6.2	Test Requirements	2591

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2591

A.6.5.5.7.1	Test Purpose and Environment	2591

A.6.5.5.7.2	Test Requirements	2597

A.6.5.6	Active BWP switch	2598

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2598

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2598

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2604

A.6.5.6.2	RRC-based Active BWP Switch	2609

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2609

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2613

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2613

A.6.5.6.4	SCell dormancy switch	2620

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2620

A.6.5.6.4.1.1	Test Purpose and Environment	2620

A.6.5.6.4.1.2	Test Requirements	2626

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2626

A.6.5.6.4.2.1	 Test Purpose and Environment	2626

A.6.5.6.4.2.2	Test Requirements	2633

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2633

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2633

A.6.5.7	DL interruptions at switching between two uplink carriers	2638

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2638

A.6.5.7.1.1	Test Purpose and Environment	2638

A.6.5.7.1.2	Test Requirements	2642

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2642

A.6.5.7.2.1	Test Purpose and Environment	2642

A.6.5.7.2.2	Test Requirements	2646

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2646

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2646

A.6.5.7A.1.1	Test Purpose and Environment	2646

A.6.5.7A.1.2	Test Requirements	2650

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2650

A.6.5.7A.2.1	Test Purpose and Environment	2650

A.6.5.7A.2.2	Test Requirements	2654

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2654

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2654

A.6.5.7B.1.1	Test Purpose and Environment	2654

A.6.5.7B.1.2	Test Requirements	2658

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2658

A.6.5.7B.2.1	Test Purpose and Environment	2658

A.6.5.7B.2.2	Test Requirements	2662

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2662

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2662

A.6.5.7C.1.1	Test Purpose and Environment	2662

A.6.5.7C.1.2	Test Requirements	2666

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2667

A.6.5.7C.2.1	Test Purpose and Environment	2667

A.6.5.7C.2.2	Test Requirements	2671

A.6.5.8	UE specific CBW change	2672

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2672

A.6.5.8.1.1	Test Purpose and Environment	2672

A.6.5.8.1.2	Test Requirements	2676

A.6.5.9	Pathloss reference signal switching delay	2676

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2676

A.6.5.9.1.1	Test Purpose and Environment	2676

A.6.5.9.1.2	Test Requirements	2679

A.6.6	Measurement procedure	2680

A.6.6.1	Intra-frequency Measurements	2680

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2680

A.6.6.1.1.1	Test purpose and Environment	2680

A.6.6.1.1.2	Test parameters	2680

A.6.6.1.1.3	Test Requirements	2684

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2684

A.6.6.1.2.1	Test purpose and Environment	2684

A.6.6.1.2.2	Test parameters	2684

A.6.6.1.2.3	Test Requirements	2688

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2688

A.6.6.1.3.1	Test purpose and Environment	2688

A.6.6.1.3.2	Test parameters	2688

A.6.6.1.3.3	Test Requirements	2692

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2692

A.6.6.1.4.1	Test purpose and Environment	2692

A.6.6.1.4.2	Test parameters	2692

A.6.6.1.4.3	Test Requirements	2696

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2696

A.6.6.1.5.1	Test purpose and Environment	2696

A.6.6.1.5.2	Test parameters	2696

A.6.6.1.5.3	Test Requirements	2698

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2699

A.6.6.1.6.1	Test purpose and Environment	2699

A.6.6.1.6.2	Test parameters	2699

A.6.6.1.6.3	Test Requirements	2700

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2701

A.6.6.1.7.1	Test purpose and Environment	2701

A.6.6.1.7.2	Test parameters	2701

A.6.6.1.7.3	Test Requirements	2705

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2705

A.6.6.1.8.1	Test purpose and Environment	2705

A.6.6.1.8.2	Test parameters	2705

A.6.6.1.8.3	Test Requirements	2709

A.6.6.2	Inter-frequency Measurements	2709

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2709

A.6.6.2.1.1	Test Purpose and Environment	2709

A.6.6.2.1.2	Test Requirements	2713

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2713

A.6.6.2.2.1	Test Purpose and Environment	2713

A.6.6.2.2.2	Test Requirements	2717

A.6.6.2.3	Void	2718

A.6.6.2.4	Void	2718

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2718

A.6.6.2.5.1	Test Purpose and Environment	2718

A.6.6.2.5.2	Test Requirements	2722

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2722

A.6.6.2.6.1	Test Purpose and Environment	2722

A.6.6.2.6.2	Test Requirements	2726

A.6.6.2.7	Void	2726

A.6.6.2.8	Void	2726

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2726

A.6.6.2.9.1	Test Purpose and Environment	2726

A.6.6.2.9.2	Test Requirements	2730

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2730

A.6.6.2.10.1	Test Purpose and Environment	2730

A.6.6.2.10.2	Test Requirements	2734

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2738

A.6.6.2.12.1	Test Purpose and Environment	2738

A.6.6.2.12.2	Test Requirements	2742

A.6.6.3	Inter-RAT Measurements	2742

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2742

A.6.6.3.1.1	Test Purpose and Environment	2742

A.6.6.3.1.2	Test Requirements	2748

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2748

A.6.6.3.2.1	Test Purpose and Environment	2748

A.6.6.3.2.2	Test Requirements	2755

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2755

A.6.6.3.3.1	Test Purpose and Environment	2755

A.6.6.3.3.2	Test Requirements	2760

A.6.6.4	L1-RSRP measurement for beam reporting	2760

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2760

A.6.6.4.1.1	Test Purpose and Environment	2760

A.6.6.4.1.2	Test parameters	2761

A.6.6.4.1.3	Test Requirements	2764

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2764

A.6.6.4.2.1	Test Purpose and Environment	2764

A.6.6.4.2.2	Test parameters	2765

A.6.6.4.2.3	Test Requirements	2768

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2768

A.6.6.4.3.1	Test Purpose and Environment	2768

A.6.6.4.3.2	Test parameters	2769

A.6.6.4.3.3	Test Requirements	2772

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2772

A.6.6.4.4.1	Test Purpose and Environment	2772

A.6.6.4.4.2	Test parameters	2773

A.6.6.4.4.3	Test Requirements	2776

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2776

A.6.6.4.5.1	Test Purpose and Environment	2776

A.6.6.4.5.2	Test parameters	2777

A.6.6.4.5.3	Test Requirements	2780

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2780

A.6.6.4.6.1	Test Purpose and Environment	2780

A.6.6.4.6.2	Test parameters	2781

A.6.6.4.6.3	Test Requirements	2784

A.6.6.5	Inter-RAT UTRAN FDD measurements	2785

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2785

A.6.6.5.1.1	Test Purpose and Environment	2785

A.6.6.5.1.2	Test Requirements	2788

A.6.6.6	CLI measurements	2788

A.6.6.6.1	SRS-RSRP measurement with DRX	2788

A.6.6.6.1.1	Test Purpose and Environment	2788

A.6.6.6.1.2	Test Parameters	2789

A.6.6.6.1.3	Test Requirements	2792

A.6.6.6.2	CLI-RSSI measurement with DRX	2792

A.6.6.6.2.1	Test Purpose and Environment	2792

A.6.6.6.2.2	Test Parameters	2793

A.6.6.6.2.3	Test Requirements	2795

A.6.6.7	NR measurements with autonomous gaps	2795

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2795

A.6.6.7.1.1	Test Purpose and Environment	2795

A.6.6.7.1.2	Test Parameters	2795

A.6.6.7.1.3	Test Requirements	2798

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2798

A.6.6.7.2.1	Test Purpose and Environment	2798

A.6.6.7.2.2	Test Requirements	2801

A.6.6.8	L1-SINR measurement for beam reporting	2802

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2804

A.6.6.8.2.1	Test Purpose and Environment	2804

A.6.6.8.2.2	Test parameters	2804

A.6.6.8.2.3	Test Requirements	2808

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2808

A.6.6.8.3.1	Test Purpose and Environment	2808

A.6.6.8.3.2	Test parameters	2809

A.6.6.8.3.3	Test Requirements	2811

A.6.6.9	Idle Mode CA/DC Measurements	2811

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2811

A.6.6.9.1.1	Test Purpose and Environment	2811

A.6.6.9.1.2	Test Requirements	2818

A.6.6.10	CSI-RS based intra-frequency Measurements	2818

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2818

A.6.6.10.1.1	Test purpose and Environment	2818

A.6.6.10.1.2	Test Requirements	2821

A.6.6.11	CSI-RS based inter-frequency Measurements	2821

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2821

A.6.6.11.1.1	Test Purpose and Environment	2821

A.6.6.11.1.2	Test Requirements	2825

A.6.6.12	RSTD measurements	2825

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2825

A.6.6.12.1.1	Test Purpose and Environment	2825

A.6.6.12.1.2	Test Requirements	2832

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2833

A.6.6.12.2.1	Test Purpose and Environment	2833

A.6.6.12.2.2	Test Requirements	2839

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2839

A.6.6.12.3.1	Test Purpose and Environment	2839

A.6.6.12.3.2	Test Requirements	2845

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2845

A.6.6.12.4.1	Test Purpose and Environment	2845

A.6.6.12.4.2	Test Requirements	2849

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2850

A.6.6.12.5.1	Test Purpose and Environment	2850

A.6.6.12.5.2	Test Requirements	2854

A.6.6.13 PRS-RSRP measurements	2854

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2854

A.6.6.13.1.1	Test purpose and Environment	2854

A.6.6.13.1.2	Test Requirements	2858

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2858

A.6.6.13.2.1	Test purpose and Environment	2858

A.6.6.13.2.2	Test Requirements	2862

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2862

A.6.6.13.3.1	Test purpose and Environment	2862

A.6.6.13.3.2	Test Requirements	2866

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2866

A.6.6.13.4.1	Test purpose and Environment	2866

A.6.6.14	UE Rx-Tx time difference measurements	2870

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2870

A.6.6.14.1.1	Test purpose and environment	2870

A.6.6.14.1.2	Test requirements	2874

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2874

A.6.6.14.2.1	Test purpose and environment	2874

A.6.6.14.2.2	Test requirements	2878

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2878

A.6.6.14.3.1	Test purpose and environment	2878

A.6.6.14.3.2	Test requirements	2882

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2882

A.6.6.14.4.1	Test purpose and environment	2882

A.6.6.14.4.2	Test requirements	2886

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2886

A.6.6.14.4.1	Test purpose and environment	2886

A.6.6.14.4.2	Test requirements	2890

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2890

A.6.6.15.1	Test Purpose and Environment	2890

A.6.6.15.2	Test Requirements	2897

A.6.6.16	PRS-RSRPP measurements	2898

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2898

A.6.6.16.1.1	Test purpose and Environment	2898

A.6.6.16.1.2	Test Requirements	2900

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2900

A.6.6.16.2.1	Test purpose and Environment	2900

A.6.6.16.2.2	Test Requirements	2903

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2903

A.6.6.16.3.1	Test purpose and Environment	2903

A.6.6.16.3.2	Test Requirements	2907

A.6.6.17	SA event triggered reporting tests with Pre-MG	2907

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2907

A.6.6.17.1.1	Test purpose and Environment	2907

A.6.6.17.1.2	Test parameters	2907

A.6.6.17.1.3	Test Requirements	2911

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2911

A.6.6.17.2.1	Test purpose and Environment	2911

A.6.6.17.2.2	Test parameters	2911

A.6.6.17.2.3	Test Requirements	2916

A.6.6.17.3	Void	2916

A.6.6.17.3.1	Void	2916

A.6.6.17.3.2	Void	2916

A.6.6.17.3.3	Void	2916

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2916

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2916

A.6.6.18.1.1	Test Purpose and Environment	2916

A.6.6.18.1.2	Test Requirements	2920

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2920

A.6.6.18.2.1	Test Purpose and Environment	2920

A.6.6.18.2.2	Test Requirements	2924

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2924

A.6.6.18.3.1	Test Purpose and Environment	2924

A.6.6.18.3.2	Test Requirements	2931

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2931

A.6.6.18.4.1	Test Purpose and Environment	2931

A.6.6.18.4.2	Test Requirements	2935

A.6.6.19	SA event triggered reporting tests with NCSG	2936

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2936

A.6.6.19.1.1	Test purpose and Environment	2936

A.6.6.19.1.2	Test parameters	2936

A.6.6.19.1.3	Test Requirements	2940

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2940

A.6.6.19.2.1	Test Purpose and Environment	2940

A.6.6.19.2.2	Test parameters	2940

A.6.6.19.2.3	Test Requirements	2944

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2944

A.6.6.19.3.1	Test Purpose and Environment	2944

A.6.6.19.3.2	Test parameters	2945

A.6.6.19.3.3	Test Requirements	2950

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2950

A.6.6.19.4.1	Test purpose and Environment	2950

A.6.6.19.4.2	Test parameters	2950

A.6.6.19.4.3	Test Requirements	2953

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2953

A.6.6.20.1	Test purpose and environment	2953

A.6.6.20.2	Test requirements	2957

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2957

A.6.6.21.1	Test purpose and environment	2957

A.6.6.21.2	Test requirements	2959

A.6.7	Measurement Performance requirements	2960

A.6.7.1	SS-RSRP	2960

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2960

A.6.7.1.1.1	Test Purpose and Environment	2960

A.6.7.1.1.2	Test parameters	2960

A.6.7.1.1.3	Test Requirements	2965

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2965

A.6.7.1.2.1	Test Purpose and Environment	2965

A.6.7.1.2.2	Test parameters	2965

A.6.7.1.2.3	Test Requirements	2969

A.6.7.1.3	Void	2970

A.6.7.2	SS-RSRQ	2970

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2970

A.6.7.2.1.1	Test Purpose and Environment	2970

A.6.7.2.1.2	Test Parameters	2970

A.6.7.2.1.3	Test Requirements	2975

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2975

A.6.7.2.2.1	Test Purpose and Environment	2975

A.6.7.2.2.2	Test Parameters	2975

A.6.7.2.2.3	Test Requirements	2980

A.6.7.3	SS-SINR	2980

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2980

A.6.7.3.1.1	Test Purpose and Environment	2980

A.6.7.3.1.2	Test Parameters	2981

A.6.7.3.1.3	Test Requirements	2985

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2986

A.6.7.3.2.1	Test Purpose and Environment	2986

A.6.7.3.2.2	Test Parameters	2986

A.6.7.3.2.3	Test Requirements	2991

A.6.7.4	L1-RSRP measurement for beam reporting	2991

A.6.7.4.1	SSB based L1-RSRP measurement	2991

A.6.7.4.1.1	Test Purpose and Environment	2991

A.6.7.4.1.2	Test parameters	2991

A.6.7.4.1.3	Test Requirements	2995

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2995

A.6.7.4.2.1	Test Purpose and Environment	2995

A.6.7.4.2.2	Test parameters	2996

A.6.7.4.2.3	Test Requirements	3000

A.6.7.5	E-UTRAN RSRP	3001

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3001

A.6.7.5.1.1	Test Purpose and Environment	3001

A.6.7.5.1.2	Test parameters	3001

A.6.7.5.1.3	Test Requirements	3007

A.6.7.6	E-UTRAN RSRQ	3008

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3008

A.6.7.6.1.1	Test Purpose and Environment	3008

A.6.7.6.1.2	Test parameters	3008

A.6.7.6.1.3	Test Requirements	3013

A.6.7.7	E-UTRAN RS-SINR	3014

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3014

A.6.7.7.1.1	Test Purpose and Environment	3014

A.6.7.7.1.2	Test parameters	3014

A.6.7.7.1.3	Test Requirements	3020

A.6.7.8	CLI measurements	3021

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	3021

A.6.7.8.1.1	Test Purpose and Environment	3021

A.6.7.8.1.2	Test parameters	3021

A.6.7.8.1.3	Test Requirements	3027

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	3027

A.6.7.8.2.1	Test Purpose and Environment	3027

A.6.7.8.2.2	Test parameters	3028

A.6.7.8.2.3	Test Requirements	3031

A.6.7.9	L1-SINR measurement for beam reporting	3032

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3037

A.6.7.9.2.1	Test Purpose and Environment	3037

A.6.7.9.2.2	Test parameters	3037

A.6.7.9.2.3	Test Requirements	3042

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3042

A.6.7.9.3.1	Test Purpose and Environment	3042

A.6.7.9.3.2	Test parameters	3043

A.6.7.9.3.3	Test Requirements	3047

A.6.7.10	CSI-RSRP	3048

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3048

A.6.7.9.10.1	Test Purpose and Environment	3048

A.6.7.9.10.2	Test parameters	3048

A.6.7.10.1.3	Test Requirements	3053

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3053

A.6.7.9.10.1	Test Purpose and Environment	3053

A.6.7.10.2.2	Test parameters	3054

A.6.7.10.2.3	Test Requirements	3059

A.6.7.11	CSI-RSRQ	3059

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3059

A.6.7.11.1.1	Test Purpose and Environment	3059

A.6.7.11.1.2	Test Parameters	3059

A.6.7.11.1.3	Test Requirements	3064

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3064

A.6.7.11.2.1	Test Purpose and Environment	3064

A.6.7.11.2.2	Test Parameters	3064

A.6.7.11.2.3	Test Requirements	3070

A.6.7.12	CSI-SINR	3071

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3071

A.6.7.12.1.1	Test Purpose and Environment	3071

A.6.7.12.1.2	Test Parameters	3071

A.6.7.12.1.3	Test Requirements	3076

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3076

A.6.7.12.2.1	Test Purpose and Environment	3076

A.6.7.12.2.2	Test Parameters	3076

A.6.7.12.2.3	Test Requirements	3082

A.6.7.13	RSTD measurements	3082

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	3082

A.6.7.13.1.1	Test purpose and Environment	3082

A.6.7.13.1.2	Test Requirements	3085

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	3085

A.6.7.13.2.1	Test purpose and Environment	3085

A.6.7.13.2.2	Test Requirements	3089

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3089

A.6.7.13.3.1	Test purpose and Environment	3089

A.6.7.13.3.2	Test Requirements	3091

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	3091

A.6.7.14	PRS-RSRP measurements	3095

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	3095

A.6.7.14.1.1	Test Purpose and Environment	3095

A.6.7.14.1.2	Test parameters	3096

A.6.7.14.1.3	Test Requirements	3100

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	3100

A.6.7.14.2.1	Test Purpose and Environment	3100

A.6.7.14.2.2	Test parameters	3100

A.6.7.14.2.3	Test Requirements	3103

A.6.7.14.3	Void	3104

A.6.7.14.3.1	Void	3104

A.6.7.14.3.2	Void	3104

A.6.7.14.3.3	Void	3104

A.6.7.15	UE Rx-Tx time difference measurements	3104

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	3104

A.6.7.15.1.1	Test purpose and environment	3104

A.6.7.15.1.2	Test parameters	3104

A.6.7.15.1.3	Test requirements	3107

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3108

A.6.7.15.2.1	Test purpose and environment	3108

A.6.7.15.2.2	Test parameters	3108

A.6.7.15.2.3	Test requirements	3111

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3111

A.6.7.15.3.1	Test purpose and environment	3111

A.6.7.15.3.2	Test parameters	3112

A.6.7.15.3.3	Test requirements	3115

A.6.7.16	PRS-RSRPP measurements	3115

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3115

A.6.7.16.1.1	Test Purpose and Environment	3115

A.6.7.16.1.2	Test parameters	3116

A.6.7.16.1.3	Test Requirements	3121

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3121

A.6.7.16.2.1	Test Purpose and Environment	3121

A.6.7.16.2.2	Test parameters	3121

A.6.8	Measurement procedure in RRC_INACTIVE	3125

A.6.8.1	RSTD measurements	3125

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3125

A.6.8.1.1.1	Test Purpose and Environment	3125

A.6.8.1.1.2	Test Requirements	3128

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3128

A.6.8.1.2.1	Test Purpose and Environment	3128

A.6.8.1.2.2	Test Requirements	3133

A.6.8.2	PRS-RSRP measurements	3133

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3133

A.6.8.2.1.1	Test purpose and Environment	3133

A.6.8.2.1.2	Test Requirements	3135

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3136

A.6.8.2.2.1	Test purpose and Environment	3136

A.6.8.2.2.2	Test Requirements	3139

A.6.8.3	UE Rx-Tx time difference measurements	3139

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3139

A.6.8.3.1.1	Test purpose and environment	3139

A.6.8.3.1.2	Test requirements	3143

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3143

A.6.8.3.2.1	Test purpose and environment	3143

A.6.8.3.2.2	Test requirements	3146

A.6.8.4	PRS-RSRPP measurements	3146

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3146

A.6.8.4.1.1	Test purpose and Environment	3146

A.6.8.4.1.2	Test Requirements	3149

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3149

A.6.8.4.2.1	Test purpose and Environment	3149

A.6.8.4.2.2	Test Requirements	3151

A.6.9	Measurement performance requirements in RRC_INACTIVE	3151

A.6.9.1	RSTD measurements	3151

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3151

A.6.9.1.1.1	Test purpose and Environment	3151

A.6.9.1.1.2	Test Requirements	3153

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3153

A.6.9.1.2.1	Test purpose and Environment	3153

A.6.9.1.2.2	Test Requirements	3155

A.6.9.2	PRS-RSRP measurements	3155

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3155

A.6.9.2.1.1	Test Purpose and Environment	3155

A.6.9.2.1.2	Test parameters	3156

A.6.9.2.1.3	Test Requirements	3161

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3161

A.6.9.2.2.1	Test Purpose and Environment	3161

A.6.9.2.2.2	Test parameters	3161

A.6.9.2.2.3	Test Requirements	3164

A.6.9.3	UE Rx-Tx time difference measurements	3165

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3165

A.6.9.3.1.1.1	Test purpose and environment	3165

A.6.9.3.1.1.2 Test parameters	3165

A.6.9.3.1.1.3	Test requirements	3168

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3168

A.6.9.3.2.1	Test purpose and environment	3168

A.6.9.3.2.2	Test parameters	3168

A.6.9.3.2.3	Test requirements	3171

A.6.9.4	PRS-RSRPP measurements	3171

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3171

A.6.9.4.1.1	Test Purpose and Environment	3171

A.6.9.4.1.2	Test parameters	3171

A.6.9.4.1.3	Test Requirements	3176

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3176

A.6.9.4.2.1	Test Purpose and Environment	3176

A.6.9.4.2.2	Test parameters	3176

A.6.9.4.2.3	Test Requirements	3179

A.7	NR standalone tests with one or more NR cells in FR2	3183

A.7.1	SA: RRC_IDLE state mobility	3183

A.7.1.1	Cell re-selection to NR	3183

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3183

A.7.1.1.1.1	Test Purpose and Environment	3183

A.7.1.1.1.2	Test Parameters	3183

A.7.1.1.1.3	Test Requirements	3186

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3186

A.7.1.1.2.1	Test Purpose and Environment	3186

A.7.1.1.2.2	Test Parameters	3187

A.7.1.1.2.3	Test Requirements	3189

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3189

A.7.1.1.3.1	Test Purpose and Environment	3189

A.7.1.1.3.2	Test Parameters	3189

A.7.1.1.3.3	Test Requirements	3192

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3192

A.7.1.1.4.1	Test Purpose and Environment	3192

A.7.1.1.4.2	Test Parameters	3192

A.7.1.1.4.3	Test Requirements	3195

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3195

A.7.1.1.5.1	Test Purpose and Environment	3195

A.7.1.1.5.2	Test Parameters	3195

A.7.1.1.5.3	Test Requirements	3198

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3198

A.7.1.1.6.1	Test Purpose and Environment	3198

A.7.1.1.6.2	Test Parameters	3198

A.7.1.1.6.3	Test Requirements	3201

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3201

A.7.1.1.7.1	Test Purpose and Environment	3201

A.7.1.1.7.2	Test Parameters	3201

A.7.1.1.7.3	Test Requirements	3205

A.7.2	SA: RRC_INACTIVE state mobility	3205

A.7.2.1	Small Data Transmission	3205

A.7.2.1.1	TA validation for CG-SDT in FR2	3205

A.7.2.1.1.1	Test Purpose and Environment	3205

A.7.2.1.1.2	Test Requirements	3209

A.7.3	RRC_CONNECTED state mobility	3209

A.7.3.1	Handover	3209

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3209

A.7.3.1.1.1	Test Purpose and Environment	3209

A.7.3.1.1.2	Test Parameters	3209

A.7.3.1.1.3	Test Requirements	3213

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3213

A.7.3.1.2.1	Test Purpose and Environment	3213

A.7.3.1.2.2	Test Parameters	3213

A.7.3.1.2.3	Test Requirements	3216

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3216

A.7.3.1.3.1	Test Purpose and Environment	3216

A.7.3.1.3.2	Test Parameters	3216

A.7.3.1.3.3	Test Requirements	3217

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3218

A.7.3.1.4.1	Test Purpose and Environment	3218

A.7.3.1.4.2	Test Parameters	3218

A.7.3.1.4.3 Test Requirements	3225

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3225

A.7.3.1.5.1	Test Purpose and Environment	3225

A.7.3.1.5.2	Test Parameters	3225

A.7.3.1.5.3 Test Requirements	3232

A.7.3.1.6	Handover with PSCell from SA to EN-DC with unknown FR2 target PScell	3232

A.7.3.1.6.1	Test Purpose and Environment	3232

A.7.3.1.6.2	Test Parameters	3232

A.7.3.1.6.3	Test Requirements	3240

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3241

A.7.3.1.7.1	Test purpose and environment	3241

A.7.3.1.7.2	Test Requirements	3246

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3247

A.7.3.1.8.1	Test Purpose and Environment	3247

A.7.3.1.8.2	Test Requirements	3252

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3252

A.7.3.1.9.1	Test Purpose and Environment	3252

A.7.3.1.9.2	Test Parameters	3252

A.7.3.1.9.3	Test Requirements	3255

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3255

A.7.3.1.10.1	Test Purpose and Environment	3255

A.7.3.1.10.2	Test Parameters	3255

A.7.3.1.10.3	Test Requirements	3259

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3259

A.7.3.1.11.1	Test Purpose and Environment	3259

A.7.3.1.11.2	Test Parameters	3259

A.7.3.1.11.3	Test Requirements	3263

A.7.3.2	RRC Connection Mobility Control	3263

A.7.3.2.1	SA: RRC Re-establishment	3263

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3263

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3266

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3269

A.7.3.2.1.3.1	Test Purpose and Environment	3269

A.7.3.2.1.3.2	Test Requirements	3271

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3272

A.7.3.2.1.4.1	Test Purpose and Environment	3272

A.7.3.2.1.4.2	Test Requirements	3274

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3275

A.7.3.2.1.5.1	Test Purpose and Environment	3275

A.7.3.2.1.5.2	Test Requirements	3278

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3278

A.7.3.2.1.6.1	Test Purpose and Environment	3278

A.7.3.2.1.6.2	Test Requirements	3280

A.7.3.2.2	Random Access	3281

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3281

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3285

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3289

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3292

A.7.3.2.3	SA: RRC Connection Release with Redirection	3295

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3295

A.7.3.3 Conditional Handover	3299

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3299

A.7.3.3.1.1	Test Purpose and Environment	3299

A.7.3.3.1.2	Test Parameters	3299

A.7.3.3.1.2.3	Test Requirements	3302

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3302

A.7.3.3.2.1	Test Purpose and Environment	3302

A.7.3.3.2.2	Test Parameters	3302

A.7.3.3.2.3 Test Requirements	3304

A.7.4	Timing	3304

A.7.4.1	UE transmit timing	3304

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3304

A.7.4.1.1.1	Test Purpose and environment	3304

A.7.4.1.1.2	Test requirements	3307

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3308

A.7.4.1.2.1	Test Purpose and environment	3308

A.7.4.1.2.2	Test requirements	3312

A.7.4.2	UE timer accuracy	3313

A.7.4.3	Timing advance	3313

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3313

A.7.4.3.1.1	Test Purpose and Environment	3313

A.7.4.3.1.2	Test Parameters	3313

A.7.4.3.1.3 Test Requirements	3316

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3317

A.7.4.3.2.1	Test Purpose and Environment	3317

A.7.4.3.2.2	Test Parameters	3317

A.7.4.3.2.3	Test Requirements	3321

A.7.5	Signaling characteristics	3321

A.7.5.1	Radio link Monitoring	3321

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3321

A.7.5.1.1.1	Test Purpose and Environment	3321

A.7.5.1.1.2	Test Requirements	3324

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3325

A.7.5.1.2.1	Test Purpose and Environment	3325

A.7.5.1.2.2	Test Requirements	3329

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3330

A.7.5.1.3.1	Test Purpose and Environment	3330

A.7.5.1.3.2	Test Requirements	3334

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3334

A.7.5.1.4.1	Test Purpose and Environment	3334

A.7.5.1.4.2	Test Requirements	3339

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3339

A.7.5.1.5.1	Test Purpose and Environment	3339

A.7.5.1.5.2	Test Requirements	3344

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3344

A.7.5.1.6.1	Test Purpose and Environment	3344

A.7.5.1.6.2	Test Requirements	3348

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3348

A.7.5.1.7.1	Test Purpose and Environment	3348

A.7.5.1.7.2	Test Requirements	3352

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3352

A.7.5.1.8.1	Test Purpose and Environment	3352

A.7.5.1.8.2	Test Requirements	3357

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3357

A.7.5.1.9.1	Test Purpose and Environment	3357

A.7.5.1.9.2	Test Requirements	3360

A.7.5.2	Interruption	3360

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3360

A.7.5.2.1.1	Test Purpose and Environment	3360

A.7.5.2.1.2	Test Requirements	3363

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3364

A.7.5.2.2.1	Test Purpose and Environment	3364

A.7.5.2.2.2	Test Parameters	3364

A.7.5.2.2.3	Test Requirements	3366

A.7.5.3	SCell Activation and Deactivation Delay	3366

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3366

A.7.5.3.1.1	Test Purpose and Environment	3366

A.7.5.3.1.2	Test Requirements	3368

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3368

A.7.5.3.2.1	Test Purpose and Environment	3368

A.7.5.3.2.2	Test Requirements	3372

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3373

A.7.5.3.3.1	Test Purpose and Environment	3373

A.7.5.3.3.2	Test Requirements	3376

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3377

A.7.5.3.4.1	Test Purpose and Environment	3377

A.7.5.3.4.2	Test Requirements	3380

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3381

A.7.5.3.5.1	Test Purpose and Environment	3381

A.7.5.3.5.2	Test Requirements	3384

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3385

A.7.5.3.6.1	Test Purpose and Environment	3385

A.7.5.3.6.2	Test Requirements	3389

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3390

A.7.5.3.7.1	Test Purpose and Environment	3390

A.7.5.3.7.2	Test Requirements	3394

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3395

A.7.5.3.8.1	Test Purpose and Environment	3395

A.7.5.3.8.2	Test Requirements	3399

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3400

A.7.5.3.9.1	Test Purpose and Environment	3400

A.7.5.3.9.2	Test Requirements	3403

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3404

A.7.5.3.10.1	Test Purpose and Environment	3404

A.7.5.3.10.2	Test Requirements	3408

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3409

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3409

A.7.5.3.11.1.1	Test Purpose and Environment	3409

A.7.5.3.11.1.2	Test Requirements	3413

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3414

A.7.5.3.11.2.1	Test Purpose and Environment	3414

A.7.5.3.11.2.2	Test Requirements	3418

A.7.5.3.12	Void	3419

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3419

A.7.5.3.13.1	Test Purpose and Environment	3419

A.7.5.3.13.2	Test Requirements	3421

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3422

A.7.5.3.14.1	Test Purpose and Environment	3422

A.7.5.3.14.2	Test Requirements	3425

A.7.5.4	Void	3426

A.7.5.5	Beam Failure Detection and Link recovery procedures	3426

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3426

A.7.5.5.1.1	Test Purpose and Environment	3426

A.7.5.5.1.2	Test Requirements	3429

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3430

A.7.5.5.2.1	Test Purpose and Environment	3430

A.7.5.5.2.2	Test Requirements	3433

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3434

A.7.5.5.3.1	Test Purpose and Environment	3434

A.7.5.5.3.2	Test Requirements	3438

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3439

A.7.5.5.4.1	Test Purpose and Environment	3439

A.7.5.5.4.2	Test Requirements	3443

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3444

A.7.5.5.5.1	Test Purpose and Environment	3444

A.7.5.5.5.2	Test Requirements	3447

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3448

A.7.5.5.6.1	Test Purpose and Environment	3448

A.7.5.5.6.2	Test Requirements	3452

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3452

A.7.5.5.7.1	Test Purpose and Environment	3452

A.7.5.5.7.2	Test Requirements	3456

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3457

A.7.5.5.8.1	Test Purpose and Environment	3457

A.7.5.5.8.2	Test Requirements	3461

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3461

A.7.5.5.9.1	Test Purpose and Environment	3461

A.7.5.5.9.2	Test Requirements	3467

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3467

A.7.5.5.10.1	Test Purpose and Environment	3467

A.7.5.5.10.2	Test Requirements	3473

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3473

A.7.5.5.11.1	Test Purpose and Environment	3473

A.7.5.5.11.2	Test Requirements	3478

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3478

A.7.5.5.12.1	Test Purpose and Environment	3478

A.7.5.5.12.2	Test Requirements	3483

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3483

A.7.5.5.13.1	Test Purpose and Environment	3483

A.7.5.5.13.2	Test Requirements	3487

A.7.5.6	Active BWP switch	3487

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3487

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3487

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3492

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3497

A.7.5.6.1.3.1	Test Purpose and Environment	3497

A.7.5.6.1.3.2	Test Requirements	3500

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3500

A.7.5.6.1.4.1	Test Purpose and Environment	3500

A.7.5.6.1.4.2	Test Requirements	3505

A.7.5.6.2	RRC-based Active BWP Switch	3506

A.7.5.6.2.1.1	Test Purpose and Environment	3506

A.7.5.6.2.1.2	Test Requirements	3509

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3510

A.7.5.6.2.2.1	Test Purpose and Environment	3510

A.7.5.6.2.2.2	Test Requirements	3514

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3515

A.7.5.6.3.1.1	Test Purpose and Environment	3515

A.7.5.6.3.1.2	Test Requirements	3518

A.7.5.6.4	SCell dormancy switch	3518

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3518

A.7.5.6.4.1.1	Test Purpose and Environment	3518

A.7.5.6.4.1.2	Test Requirements	3523

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3523

A.7.5.6.4.2.1	 Test Purpose and Environment	3523

A.7.5.6.4.2.2	 Test Requirements	3528

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3528

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3528

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3531

A.7.5.6.5.2.1	Test Purpose and Environment	3531

A.7.5.6.5.2.2	Test Requirements	3535

A.7.5.7	PSCell addition and release delay	3536

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3536

A.7.5.7.1.1	Test Purpose and Environment	3536

A.7.5.7.1.2	Test Requirements	3539

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3539

A.7.5.7.2.1	Test Purpose and Environment	3539

A.7.5.7.2.2	Test Requirements	3542

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3542

A.7.5.7.3.1	Test Purpose and Environment	3542

A.7.5.7.3.2	Test Requirements	3545

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3545

A.7.5.7.4.1	Test Purpose and Environment	3545

A.7.5.7.4.2	Test Requirements	3548

A.7.5.8	Active TCI state switch delay	3548

A.7.5.8.1	MAC-CE based active TCI state switch	3548

A.7.5.8.2	RRC based active TCI state switch	3552

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3556

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3556

A.7.5.8.3.1.1	Test Purpose and Environment	3556

A.7.5.8.3.1.2	Test Requirements	3560

A.7.5.9	Uplink spatial relation switch delay	3561

A.7.5.9.1.1.1	Test Purpose and Environment	3561

A.7.5.9.1.1.2	Test Requirements	3564

A.7.5.9.2	RRC based spatial relation switch	3564

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3564

A.7.5.9.2.1.1	Test Purpose and Environment	3564

A.7.5.9.2.1.2	Test Requirements	3567

A.7.5.10	UE specific CBW change	3567

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3567

A.7.5.10.1.1	Test Purpose and Environment	3567

A.7.5.10.1.2	Test Requirements	3570

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3571

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3571

A.7.5.11.1.1	Test Purpose and Environment	3571

A.7.5.11.1.2	Test Requirements	3574

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3574

A.7.5.12.1	Addition and Release Delay of PSCell	3574

A.7.5.12.1.1	Test purpose and environment	3574

A.7.5.12.1.2	Test Parameters	3574

A.7.5.12.1.3	Test Requirements	3577

A.7.5.13	 Unified TCI state switching delay	3577

A.7.5.13.1	MAC-CE based active joint TCI state switching	3577

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3577

A.7.5.13.1.1.1	Test Purpose and Environment	3577

A.7.5.13.1.1.2	Test parameters	3578

A.7.5.13.1.1.3	Test Requirements	3580

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3580

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3580

A.7.5.13.2.1.1	Test Purpose and Environment	3580

A.7.5.13.2.1.2	Test parameters	3581

A.7.5.13.2.1.3	Test Requirements	3583

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3583

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3583

A.7.5.13.3.1.1	Test Purpose and Environment	3583

A.7.5.13.3.1.2	Test Parameters	3584

A.7.5.13.3.1.3	Test Requirements	3587

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3587

A.7.5.14.1	Test Purpose and Environment	3587

A.7.5.14.2	Test Requirements	3590

A.7.5.15	Void	3591

A.7.6	Measurement procedure	3591

A.7.6.1	Intra-frequency Measurements	3591

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3591

A.7.6.1.1.1	Test purpose and Environment	3591

A.7.6.1.1.2	Test Requirements	3594

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3594

A.7.6.1.2.1	Test purpose and Environment	3594

A.7.6.1.2.2	Test Requirements	3597

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3598

A.7.6.1.3.1	Test purpose and Environment	3598

A.7.6.1.3.2	Test Requirements	3601

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3601

A.7.6.1.4.1	Test purpose and Environment	3601

A.7.6.1.4.2	Test Requirements	3604

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3605

A.7.6.1.5.1	Test purpose and Environment	3605

A.7.6.1.5.2	Test Requirements	3608

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3608

A.7.6.1.6.1	Test purpose and Environment	3608

A.7.6.1.6.2	Test Requirements	3611

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3612

A.7.6.1.7.1	Test purpose and Environment	3612

A.7.6.1.7.2	Test Requirements	3614

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3615

A.7.6.1.8.1	Test purpose and Environment	3615

A.7.6.1.8.2	Test Requirements	3618

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3619

A.7.6.1.9.1	Test purpose and Environment	3619

A.7.6.1.9.2	Test Requirements	3621

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3622

A.7.6.1.10.1	Test purpose and Environment	3622

A.7.6.1.10.2	Test Requirements	3625

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3626

A.7.6.1.11.1	Test purpose and Environment	3626

A.7.6.1.11.2	Test Requirements	3628

A.7.6.2	Inter-frequency Measurements	3629

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3629

A.7.6.2.1.1	Test Purpose and Environment	3629

A.7.6.2.1.2	Test Requirements	3632

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3632

A.7.6.2.2.1	Test Purpose and Environment	3632

A.7.6.2.2.2	Test Requirements	3636

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3636

A.7.6.2.3.1	Test Purpose and Environment	3636

A.7.6.2.3.2	Test Requirements	3640

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3640

A.7.6.2.4.1	Test Purpose and Environment	3640

A.7.6.2.4.2	Test Requirements	3644

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3644

A.7.6.2.5.1	Test Purpose and Environment	3644

A.7.6.2.5.2	Test Requirements	3648

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3649

A.7.6.2.6.1	Test Purpose and Environment	3649

A.7.6.2.6.2	Test Requirements	3653

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3654

A.7.6.2.7.1	Test Purpose and Environment	3654

A.7.6.2.7.2	Test Requirements	3658

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3659

A.7.6.2.8.1	Test Purpose and Environment	3659

A.7.6.2.8.2	Test Requirements	3663

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3664

A.7.6.2.9.1	Test Purpose and Environment	3664

A.7.6.2.9.2	Test Requirements	3668

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3668

A.7.6.2.10.1	Test Purpose and Environment	3668

A.7.6.2.10.2	Test Requirements	3670

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3670

A.7.6.2.11.1	Test Purpose and Environment	3670

A.7.6.2.11.2	Test Requirements	3673

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3673

A.7.6.2.12.1	Test Purpose and Environment	3673

A.7.6.2.12.2	Test Requirements	3677

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3678

A.7.6.2.13.1	Test Purpose and Environment	3678

A.7.6.2.13.2	Test Requirements	3682

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3683

A.7.6.2.14.1	Test Purpose and Environment	3683

A.7.6.2.14.2	Test Requirements	3687

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3688

A.7.6.2.15.1	Test Purpose and Environment	3688

A.7.6.2.15.2	Test Requirements	3692

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3693

A.7.6.2.16.1	Test Purpose and Environment	3693

A.7.6.2.16.2	Test Requirements	3699

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3699

A.7.6.2.17.1	Test Purpose and Environment	3699

A.7.6.2.17.2	Test Requirements	3706

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3707

A.7.6.2.18.1	Test Purpose and Environment	3707

A.7.6.2.18.2	Test Requirements	3713

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3713

A.7.6.2.19.1	Test Purpose and Environment	3713

A.7.6.2.19.2	Test Requirements	3721

A.7.6.3	L1-RSRP measurement for beam reporting	3722

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3722

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3722

A.7.6.3.1.1	Test Purpose and Environment	3722

A.7.6.3.1.2	Test parameters	3722

A.7.6.3.1.3	Test Requirements	3724

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3724

A.7.6.3.2.1	Test Purpose and Environment	3724

A.7.6.3.2.2	Test parameters	3725

A.7.6.3.2.3	Test Requirements	3727

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3727

A.7.6.3.3.1	Test Purpose and Environment	3727

A.7.6.3.3.2	Test parameters	3728

A.7.6.3.3.3	Test Requirements	3730

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3731

A.7.6.3.4.1	Test Purpose and Environment	3731

A.7.6.3.4.2	Test parameters	3731

A.7.6.3.3.3	Test Requirements	3733

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3734

A.7.6.3.5.1	Test Purpose and Environment	3734

A.7.6.3.5.2	Test parameters	3734

A.7.6.3.5.3	Test Requirements	3736

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3736

A.7.6.3.6.1	Test Purpose and Environment	3736

A.7.6.3.6.2	Test parameters	3737

A.7.6.3.6.3	Test Requirements	3740

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3740

A.7.6.3.7.1	Test Purpose and Environment	3740

A.7.6.3.7.2	Test parameters	3741

A.7.6.3.7.3	Test Requirements	3744

A.7.6.4	CLI measurements	3745

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3745

A.7.6.4.1.1	Test Purpose and Environment	3745

A.7.6.4.1.2	Test Parameters	3745

A.7.6.4.1.3	Test Requirements	3747

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3747

A.7.6.4.2.1	Test Purpose and Environment	3747

A.7.6.4.2.2	Test Parameters	3748

A.7.6.4.2.3	Test Requirements	3749

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3750

A.7.6.5.1.1	Test Purpose and Environment	3750

A.7.6.5.1.2	Test Requirements	3753

A.7.6.6	L1-SINR measurement for beam reporting	3753

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3755

A.7.6.6.2.1	Test Purpose and Environment	3755

A.7.6.6.2.2	Test parameters	3756

A.7.6.6.2.3	Test Requirements	3758

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3758

A.7.6.6.3.1	Test Purpose and Environment	3758

A.7.6.6.3.2	Test parameters	3759

A.7.6.6.3.3	Test Requirements	3761

A.7.6.7	CSI-RS based intra-frequency Measurements	3761

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3761

A.7.6.7.1.1	Test purpose and Environment	3761

A.7.6.7.1.2	Test Requirements	3764

A.7.6.8	CSI-RS based inter-frequency Measurements	3764

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3764

A.7.6.8.1.1	Test Purpose and Environment	3764

A.7.6.8.1.2	Test Requirements	3767

A.7.6.9	RSTD measurements	3767

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3767

A.7.6.9.1.1	Test Purpose and Environment	3767

A.7.6.9.1.2	Test Requirements	3775

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3775

A.7.6.9.2.1	Test Purpose and Environment	3775

A.7.6.9.2.2	Test Requirements	3782

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3782

A.7.6.9.3.1	Test Purpose and Environment	3782

A.7.6.9.3.2	Test Requirements	3788

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3789

A.7.6.9.4.1	Test Purpose and Environment	3789

A.7.6.9.4.2	Test Requirements	3792

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3792

A.7.6.9.5.1	Test Purpose and Environment	3792

A.7.6.9.5.2	Test Requirements	3796

A.7.6.10 PRS-RSRP measurements	3796

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3796

A.7.6.10.1.1	Test Purpose and Environment	3796

A.7.6.10.1.2	Test Requirements	3800

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3800

A.7.6.10.2.1	Test Purpose and Environment	3800

A.7.6.10.2.2	Test Requirements	3804

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3804

A.7.6.10.3.1	Test Purpose and Environment	3804

A.7.6.10.3.2	Test Requirements	3808

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3808

A.7.6.10.4.1	Test Purpose and Environment	3808

A.7.6.10.4.2	Test Requirements	3812

A.7.6.11	UE Rx-Tx time difference measurements	3812

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3812

A.7.6.11.1.1	Test purpose and environment	3812

A.7.6.11.1.2	Test requirements	3816

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3816

A.7.6.11.2.1	Test purpose and environment	3816

A.7.6.11.2.2	Test requirements	3820

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3820

A.7.6.11.3.1	Test purpose and environment	3820

A.7.6.11.3.2	Test requirements	3822

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3823

A.7.6.11.4.1	Test purpose and environment	3823

A.7.6.11.4.2	Test requirements	3825

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3826

A.7.6.11.5.1	Test purpose and environment	3826

A.7.6.11.5.2	Test requirements	3828

A.7.6.12	PRS-RSRPP measurements	3829

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3829

A.7.6.12.1.1	Test Purpose and Environment	3829

A.7.6.12.1.2	Test Requirements	3831

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3832

A.7.6.12.2.1	Test Purpose and Environment	3832

A.7.6.12.2.2	Test Requirements	3834

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3835

A.7.6.12.3.1	Test Purpose and Environment	3835

A.7.6.12.3.2	Test Requirements	3838

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3838

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3838

A.7.6.13.1.1	Test purpose and environment	3838

A.7.6.13.1.2	Test requirements	3840

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3841

A.7.6.13.2.1	Test purpose and environment	3841

A.7.6.13.2.2	Test requirements	3843

A.7.6.14	SA event triggered reporting tests with Pre-MG	3844

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3844

A.7.6.14.1.1	Test purpose and Environment	3844

A.7.6.14.1.2	Test parameters	3844

A.7.6.14.1.3	Test Requirements	3846

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3847

A.7.6.14.2.1	Test purpose and Environment	3847

A.7.6.14.2.2	Test parameters	3847

A.7.6.14.2.3	Test Requirements	3849

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3850

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3850

A.7.6.15.1.1	Test Purpose and Environment	3850

A.7.6.15.1.2	Test Requirements	3853

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3853

A.7.6.15.2.1	Test Purpose and Environment	3853

A.7.6.15.2.2	Test Requirements	3856

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overalpping scenario for SSB-based measurements and PRS-based measurement	3856

A.7.6.15.3.1	Test Purpose and Environment	3856

A.7.6.15.3.2	Test Requirements	3860

A.7.6.16	SA event triggered reporting tests with NCSG	3860

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3860

A.7.6.16.1.1	Test purpose and Environment	3860

A.7.6.16.1.2	Test Requirements	3864

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3864

A.7.6.16.2.1	Test Purpose and Environment	3864

A.7.6.16.2.2	Test Requirements	3867

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3867

A.7.6.16.3.1	Test Purpose and Environment	3867

A.7.6.16.3.2	Test Requirements	3870

A.7.7	Measurement Performance requirements	3870

A.7.7.1	SS-RSRP	3871

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3871

A.7.7.1.1.1	Test Purpose and Environment	3871

A.7.7.1.1.2	Test parameters	3871

A.7.7.1.1.3	Test Requirements	3875

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3875

A.7.7.1.2.1	Test Purpose and Environment	3875

A.7.7.1.2.2	Test parameters	3876

A.7.7.1.2.3	Test Requirements	3880

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3881

A.7.7.1.3.1	Test Purpose and Environment	3881

A.7.7.1.3.2	Test parameters	3881

A.7.7.1.3.3	Test Requirements	3885

A.7.7.2	SS-RSRQ	3885

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3885

A.7.7.2.1.1	Test Purpose and Environment	3885

A.7.7.2.1.2	Test Parameters	3885

A.7.7.2.1.3	Test Requirements	3887

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3887

A.7.7.2.2.1	Test Purpose and Environment	3887

A.7.7.2.2.2	Test Parameters	3887

A.7.7.2.2.3	Test Requirements	3889

A.7.7.3	SS-SINR	3889

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3889

A.7.7.3.1.1	Test Purpose and Environment	3889

A.7.7.3.1.2	Test Parameters	3889

A.7.7.3.1.3	Test Requirements	3891

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3891

A.7.7.3.2.1	Test Purpose and Environment	3891

A.7.7.3.2.2	Test Parameters	3891

A.7.7.3.2.3	Test Requirements	3893

A.7.7.4	L1-RSRP measurement for beam reporting	3893

A.7.7.4.1	SSB based L1-RSRP measurement	3893

A.7.7.4.1.1	Test Purpose and Environment	3893

A.7.7.4.1.2	Test parameters	3894

A.7.7.4.1.3	Test Requirements	3896

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3897

A.7.7.4.2.1	Test Purpose and Environment	3897

A.7.7.4.2.2	Test parameters	3897

A.7.7.4.2.3	Test Requirements	3899

A.7.7.5	CLI measurements	3900

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3900

A.7.7.5.1.1	Test Purpose and Environment	3900

A.7.7.5.1.2	Test parameters	3900

A.7.7.5.1.3	Test Requirements	3903

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3904

A.7.7.5.2.1	Test Purpose and Environment	3904

A.7.7.5.2.2	Test parameters	3904

A.7.7.5.2.3	Test Requirements	3906

A.7.7.6	L1-SINR measurement for beam reporting	3907

A.7.7.6.1.1	Test Purpose and Environment	3907

A.7.7.6.1.2	Test parameters	3907

A.7.7.6.1.3	Test Requirements	3909

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3910

A.7.7.6.2.1	Test Purpose and Environment	3910

A.7.7.6.2.2	Test parameters	3910

A.7.7.6.2.3	Test Requirements	3912

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3913

A.7.7.6.3.1	Test Purpose and Environment	3913

A.7.7.6.3.2	Test parameters	3913

A.7.7.6.3.3	Test Requirements	3915

A.7.7.7	CSI-RSRP	3916

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3916

A.7.7.7.1.1	Test Purpose and Environment	3916

A.7.7.7.1.2	Test parameters	3916

A.7.7.7.1.3	Test Requirements	3919

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3920

A.7.7.7.2.1	Test Purpose and Environment	3920

A.7.7.7.2.2	Test parameters	3920

A.7.7.7.2.3	Test Requirements	3923

A.7.7.8	CSI-RSRQ	3924

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3924

A.7.7.8.1.1	Test Purpose and Environment	3924

A.7.7.8.1.2	Test Parameters	3924

A.7.7.8.1.3	Test Requirements	3926

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3926

A.7.7.8.2.1	Test Purpose and Environment	3926

A.7.7.8.2.2	Test Parameters	3926

A.7.7.8.2.3	Test Requirements	3928

A.7.7.9	CSI-SINR	3928

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3928

A.7.7.9.1.1	Test Purpose and Environment	3928

A.7.7.9.1.2	Test Parameters	3928

A.7.7.9.1.3	Test Requirements	3930

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3930

A.7.7.9.2.1	Test Purpose and Environment	3930

A.7.7.9.2.2	Test Parameters	3931

A.7.7.9.2.3	Test Requirements	3933

A.7.7.10	RSTD measurements	3933

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3933

A.7.7.10.1.1	Test purpose and Environment	3933

A.7.7.10.1.2	Test Requirements	3936

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3936

A.7.7.10.2.1	Test purpose and Environment	3936

A.7.7.10.2.2	Test Requirements	3940

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3941

A.7.7.10.3.1	Test purpose and Environment	3941

A.7.7.10.3.2	Test Requirements	3942

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3943

A.7.7.10.4.1	Test purpose and Environment	3943

A.7.7.10.4.2	Test Requirements	3945

A.7.7.11	PRS-RSRP measurements	3945

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3945

A.7.7.11.1.1	Test Purpose and Environment	3945

A.7.7.11.1.2	Test parameters	3945

A.7.7.11.1.3	Test Requirements	3949

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3950

A.7.7.11.2.1	Test Purpose and Environment	3950

A.7.7.11.2.2	Test parameters	3950

A.7.7.11.2.3	Test Requirements	3952

A.7.7.12	UE Rx-Tx time difference measurements	3953

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3953

A.7.7.12.1.1	Test purpose and environment	3953

A.7.7.12.1.2	Test parameters	3953

A.7.7.12.1.3	Test requirements	3956

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3957

A.7.7.12.2.1	Test purpose and environment	3957

A.7.7.12.2.2	Test parameters	3957

A.7.7.12.2.3	Test requirements	3960

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3960

A.7.7.12.3.1	Test purpose and environment	3960

A.7.7.12.3.2	Test parameters	3961

A.7.7.12.3.3	Test requirements	3964

A.7.7.13	PRS-RSRPP measurements	3964

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3964

A.7.7.13.1.1	Test Purpose and Environment	3964

A.7.7.13.1.2	Test parameters	3965

A.7.7.13.1.3	Test Requirements	3967

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3968

A.7.7.13.2.1	Test Purpose and Environment	3968

A.7.7.13.2.2	Test parameters	3968

A.7.7.13.2.3	Test Requirements	3970

A.7.8	Measurement procedure in RRC_INACTIVE	3971

A.7.8.1	RSTD measurements	3971

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3971

A.7.8.1.1.1	Test Purpose and Environment	3971

A.7.8.1.1.2	Test Requirements	3974

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3974

A.7.8.1.2.1	Test Purpose and Environment	3974

A.7.8.1.2.2	Test Requirements	3980

A.7.8.2	PRS-RSRP measurements	3981

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3981

A.7.8.2.1.1	Test Purpose and Environment	3981

A.7.8.2.1.2	Test Requirements	3985

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3985

A.7.8.2.2.1	Test purpose and Environment	3985

A.7.8.2.2.2	Test Requirements	3989

A.7.8.3	UE Rx-Tx time difference measurements	3989

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3989

A.7.8.3.1.1	Test purpose and environment	3989

A.7.8.3.1.2	Test requirements	3993

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3993

A.7.8.3.2.1	Test purpose and environment	3993

A.7.8.3.2.2	Test requirements	3995

A.7.8.4	PRS-RSRPP measurements	3996

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	3996

A.7.8.4.1.1	Test Purpose and Environment	3996

A.7.8.4.1.2	Test Requirements	3998

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	3998

A.7.8.4.2.1	Test Purpose and Environment	3998

A.7.8.4.2.2	Test Requirements	4001

A.7.9	Measurement performance requirements in RRC_INACTIVE	4001

A.7.9.1	RSTD measurements	4001

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4001

A.7.9.1.1.1	Test purpose and Environment	4001

A.7.9.1.1.2	Test Requirements	4003

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4003

A.7.9.1.2.1	Test purpose and Environment	4003

A.7.9.1.2.2	Test Requirements	4005

A.7.9.2	PRS-RSRP measurements	4005

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	4005

A.7.9.2.1.1	Test Purpose and Environment	4005

A.7.9.2.1.2	Test parameters	4005

A.7.9.2.1.3	Test Requirements	4007

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	4008

A.7.9.2.2.1	Test Purpose and Environment	4008

A.7.9.2.2.2	Test parameters	4008

A.7.9.2.2.3	Test Requirements	4010

A.7.9.3	UE Rx-Tx time difference measurements	4011

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	4011

A.7.9.3.1.1	Test purpose and environment	4011

A.7.9.3.1.2	Test parameters	4011

A.7.9.3.1.3	Test requirements	4014

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	4014

A.7.9.3.2.1	Test purpose and environment	4014

A.7.9.3.2.2	Test parameters	4014

A.7.9.3.2.3	Test requirements	4017

A.7.9.4	PRS-RSRPP measurements	4017

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	4017

A.7.9.4.1.1	Test Purpose and Environment	4017

A.7.9.4.1.2	Test parameters	4017

A.7.9.4.1.3	Test Requirements	4019

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	4020

A.7.9.4.2.1	Test Purpose and Environment	4020

A.7.9.4.2.2	Test parameters	4020

A.7.9.4.2.3	Test Requirements	4022

A.8	E-UTRA standalone tests for NR RRM	4024

A.8.1	Void	4024

A.8.2	RRC_IDLE state mobility	4024

A.8.2.1	Inter-RAT NR Cell re-selection	4024

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	4024

A.8.2.1.1.1	Test Purpose and Environment	4024

A.8.2.1.1.2	Test Requirements	4029

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	4030

A.8.2.1.2.1	Test Purpose and Environment	4030

A.8.2.1.2.2	Test Requirements	4034

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	4035

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	4035

A.8.2.2.1.1	Test Purpose and Environment	4035

A.8.2.2.1.2	Test Requirements	4039

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	4039

A.8.2.2.2.1	Test Purpose and Environment	4039

A.8.2.2.2.2	Test Requirements	4042

A.8.3	RRC_CONNECTED state mobility	4043

A.8.3.1	Handover	4043

A.8.3.1.1	E-UTRAN - NR handover in FR1	4043

A.8.3.1.1.1	Test Purpose and Environment	4043

A.8.3.1.1.2	Test Requirements	4048

A.8.4	Measurement procedure	4048

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	4048

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	4048

A.8.4.1.1.1	Test Purpose and Environment	4048

A.8.4.1.1.2	Test Requirements	4050

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	4051

A.8.4.1.2.1	Test Purpose and Environment	4051

A.8.4.1.2.2	Test Requirements	4052

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	4052

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4052

A.8.4.2.1.1	Test Purpose and Environment	4052

A.8.4.2.1.2	Test Requirements	4057

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4057

A.8.4.2.2.1	Test Purpose and Environment	4057

A.8.4.2.2.2	Test Requirements	4061

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4061

A.8.4.2.3.1	Test Purpose and Environment	4061

A.8.4.2.3.2	Test Requirements	4066

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4066

A.8.4.2.4.1	Test Purpose and Environment	4066

A.8.4.2.4.2	Test Requirements	4070

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4070

A.8.4.2.5.1	Test Purpose and Environment	4070

A.8.4.2.5.2	Test Requirements	4072

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4073

A.8.4.2.6.1	Test Purpose and Environment	4073

A.8.4.2.6.2	Test Requirements	4075

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4076

A.8.4.2.7.1	Test Purpose and Environment	4076

A.8.4.2.7.2	Test Requirements	4078

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4079

A.8.4.2.8.1	Test Purpose and Environment	4079

A.8.4.2.8.2	Test Requirements	4081

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	4082

A.8.4.2.9.1	Test Purpose and Environment	4082

A.8.4.2.9.2	Test Requirements	4086

A.8.5	Measurement performance	4086

A.8.5.1	SFTD accuracy	4086

A.8.5.1.1	SFTD accuracy	4086

A.8.5.1.1.1	Test Purpose	4086

A.8.5.1.1.2	Test Environment	4087

A.8.5.1.1.3	Test Requirements	4093

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	4093

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4093

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4098

A.8.5.2.1.2.1	Test Purpose and Environment	4098

A.8.5.2.1.2.2	Test Parameters	4098

A.8.5.2.1.2.3	Test Requirements	4100

A.8.5.2.2	SS-RSRQ	4100

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4100

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4105

A.8.5.2.2.2.1	Test Purpose and Environment	4105

A.8.5.2.2.2.2	Test Parameters	4105

A.8.5.2.2.2.3	Test Requirements	4107

A.8.5.2.3	SS-SINR	4107

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4107

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4111

A.8.5.2.3.2.1	Test Purpose and Environment	4111

A.8.5.2.3.2.2	Test Parameters	4111

A.8.5.2.3.2.3	Test Requirements	4113

A.9	V2X Tests	4114

A.9.1	V2X Tests in FR1	4114

A.9.1.1	Test for V2X UE Transmit Timing	4114

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	4114

A.9.1.1.1.1	Test Purpose and Environment	4114

A.9.1.1.1.2	Test requirements	4114

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	4114

A.9.1.1.2.1	Test Purpose and Environment	4114

A.9.1.1.2.2	Test requirements	4115

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4115

A.9.1.1.3.1	Test Purpose and Environment	4115

A.9.1.1.3.2	Test requirements	4118

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4118

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4118

A.9.1.2.1.1	Test Purpose and Environment	4118

A.9.1.2.1.2	Test Requirements	4121

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4121

A.9.1.2.2.1	Test Purpose and Environment	4121

A.9.1.2.2.2	Test Requirements	4123

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4123

A.9.1.2.3.1	Test Purpose and Environment	4123

A.9.1.2.3.2	Test Requirements	4125

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4125

A.9.1.3.1	 Test for GNSS configured as the highest priority	4125

A.9.1.3.1.1	Test Purpose and Environment	4125

A.9.1.3.1.2	Test Requirements	4127

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4128

A.9.1.3.2.1	Test Purpose and Environment	4128

A.9.1.3.2.2	Test Requirements	4130

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4131

A.9.1.3.3.1	Test Purpose and Environment	4131

A.9.1.3.3.2	Test Requirements	4133

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4134

A.9.1.3.4.1	Test Purpose and Environment	4134

A.9.1.3.4.2	Test Requirements	4136

A.9.1.4	Test for L1 SL-RSRP Measurement	4137

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4137

A.9.1.4.1.1	Test Purpose and Environment	4137

A.9.1.4.1.2	Test Requirements	4140

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4140

A.9.1.4.2.1	Test Purpose and Environment	4140

A.9.1.4.2.2	Test Requirements	4144

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4144

A.9.1.4.3.1	Test Purpose and Environment	4144

A.9.1.4.3.2	Test Requirements	4151

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4151

A.9.1.4.4.1	Test Purpose and Environment	4151

A.9.1.4.4.2	Test Requirements	4155

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4155

A.9.1.4.5.1	Test Purpose and Environment	4155

A.9.1.4.5.2	Test Requirements	4158

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4158

A.9.1.4.6.1	Test Purpose and Environment	4158

A.9.1.4.6.2	Test Requirements	4162

A.9.1.5	Test for Congestion Control Measurement	4162

A.9.1.5.1	Test Purpose and Environment	4162

A.9.1.5.2	Test Requirements	4168

A.9.1.6	Test for Interruption	4168

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4168

A.9.1.6.1.1	Test Purpose and Environment	4168

A.9.1.6.1.2	Test Requirements	4171

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4171

A.9.1.6.2.1	Test Purpose and Environment	4171

A.9.1.6.2.2	Test Requirements	4175

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4175

A.9.1.6.3.1	Test Purpose and Environment	4175

A.9.1.6.3.2	Test Requirements	4178

A.9.1.7	Selection / Reselection of relay UE	4178

A.9.1.7.1	Test Purpose and Environment	4178

A.9.1.7.2	Test Requirements	4184

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4185

A.10.1	RRC_CONNECTED state mobility	4185

A.10.1.1	RRC connection mobility control	4185

A.10.1.1.1	Random Access	4185

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4185

A.10.1.1.1.1.1	Test Purpose and Environment	4185

A.10.1.1.1.1.2	Test Requirements	4188

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4188

A.10.1.1.1.1.2.2	Random Access Response Reception	4188

A.10.1.1.1.1.2.3	No Random Access Response Reception	4189

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4189

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4189

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4190

A.10.1.1.1.2.1	Test Purpose and Environment	4190

A.10.1.1.1.2.2	Test Requirements	4193

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4193

A.10.1.1.1.2.2.2	Random Access Response Reception	4194

A.10.1.1.1.2.2.3	No Random Access Response Reception	4194

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4194

A.10.1.1.1.3.1	Test Purpose and Environment	4194

A.10.1.1.1.3.2	Test Requirements	4198

A.10.1.1.1.3.2.1	MsgA Transmission	4198

A.10.1.1.1.3.2.2	MsgB Reception	4198

A.10.1.1.1.3.2.3	No MsgB Reception	4199

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4199

A.10.1.1.1.4.1	Test Purpose and Environment	4199

A.10.1.1.1.4.2	Test Requirements	4203

A.10.1.1.1.4.2.1	MsgA Transmission	4203

A.10.1.1.1.4.2.2	MsgB Reception	4204

A.10.1.1.1.4.2.3	No MsgB Reception	4204

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4204

A.10.1.2.1	Test Purpose and Environment	4204

A.10.1.2.2	Test Requirements	4210

A.10.2	Timing	4211

A.10.2.1	UE transmit timing	4211

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4211

A.10.2.1.1.1	Test Purpose and environment	4211

A.10.2.1.1.2	Test requirements	4214

A.10.2.2	UE timing advance	4215

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4215

A.10.2.2.1.1	Test Purpose and Environment	4215

A.10.2.2.1.2	Test Parameters	4215

A.10.2.2.1.3	Test Requirements	4219

A.10.3	Signalling characteristics	4219

A.10.3.1	Radio link monitoring	4219

A.10.3.1.1	Introduction	4219

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4220

A.10.3.1.2.1	Test purpose and environment	4220

A.10.3.1.2.2	Test requirements	4225

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4225

A.10.3.1.3.1	Test purpose and environment	4225

A.10.3.1.3.2	Test requirements	4231

A.10.3.1.4	Void	4231

A.10.3.1.4.1	Void	4231

A.10.3.1.4.2	Void	4231

A.10.3.1.5	Void	4231

A.10.3.1.5.1	Void	4231

A.10.3.1.5.2	Void	4231

A.10.3.2	Interruption	4231

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	4231

A.10.3.2.1.1	Test Purpose and Environment	4231

A.10.3.2.1.2	Test Requirements	4236

A.10.3.3	SCell activation and deactivation delay	4236

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4236

A.10.3.3.1.1	Test Purpose and Environment	4236

A.10.3.3.1.2	Test Requirements	4241

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4241

A.10.3.3.2.1	Test Purpose and Environment	4241

A.10.3.3.2.2	Test Requirements	4242

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4242

A.10.3.3.3.1	Test Purpose and Environment	4242

A.10.3.3.3.2	Test Requirements	4243

A.10.3.4	Beam failure detection and link recovery procedures	4243

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4243

A.10.3.4.1.1	Test Purpose and Environment	4243

A.10.3.4.1.2	Test Requirements	4248

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4248

A.10.3.4.2.1	Test Purpose and Environment	4248

A.10.3.4.2.2	Test Requirements	4254

A.10.3.5	Active BWP switching	4254

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4254

A.10.3.5.1.2	Test Requirements	4259

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4260

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4260

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4263

A.10.3.5.3	RRC-based Active BWP Switch	4267

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4267

A.10.3.6	PSCell addition and release delay	4271

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4271

A.10.3.6.1.1	Test purpose and environment	4271

A.10.3.6.1.2	Test Requirements	4276

A.10.3.7	Void	4277

A.10.4	Measurement procedure	4277

A.10.4.1	Intra-frequency measurements	4277

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4277

A.10.4.1.1.1	Test purpose and environment	4277

A.10.4.1.1.2	Test parameters	4277

A.10.4.1.1.3	Test Requirements	4281

A.10.4.1.2	Void	4281

A.10.4.1.3	Void	4281

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4282

A.10.4.1.4.1	Test purpose and environment	4282

A.10.4.1.4.2	Test parameters	4282

A.10.4.1.4.3	Test Requirements	4286

A.10.4.1.5	Void	4286

A.10.4.1.6	Void	4286

A.10.4.1.7	Void	4287

A.10.4.1.8	Void	4287

A.10.4.1.9	RSSI measurement reporting on PSCC	4287

A.10.4.1.9.1	Test purpose and environment	4287

A.10.4.1.9.2	Test parameters	4287

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	4287

A.10.4.1.10.1	Test purpose and environment	4287

A.10.4.1.10.2	Test parameters	4287

A.10.4.1.11	RSSI measurement reporting on SCC	4288

A.10.4.1.11.1	Test purpose and environment	4288

A.10.4.1.11.2	Test parameters	4288

A.10.4.1.12	Channel occupancy measurement reporting on SCC	4288

A.10.4.1.12.1	Test purpose and environment	4288

A.10.4.1.12.2	Test parameters	4288

A.10.4.2	Inter-frequency measurements	4289

A.10.4.2.1	RSSI measurement reporting	4289

A.10.4.2.1.1	Test purpose and environment	4289

A.10.4.2.1.2	Test parameters	4289

A.10.4.2.2	Channel occupancy measurement reporting	4289

A.10.4.2.2.1	Test purpose and environment	4289

A.10.4.2.2.2	Test parameters	4289

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4290

A.10.4.2.3.1	Test Purpose and Environment	4290

A.10.4.2.3.2	Test Requirements	4294

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4295

A.10.4.2.4.1	Test Purpose and Environment	4295

A.10.4.2.4.2	Test Requirements	4299

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4300

A.10.4.2.5.1	Test Purpose and Environment	4300

A.10.4.2.5.2	Test Requirements	4304

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4305

A.10.4.2.6.1	Test Purpose and Environment	4305

A.10.4.2.6.2	Test Requirements	4309

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4310

A.10.4.2.7.1	Test Purpose and Environment	4310

A.10.4.2.7.2	Test Requirements	4316

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4316

A.10.4.2.8.1	Test Purpose and Environment	4316

A.10.4.2.8.2	Test Requirements	4322

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4323

A.10.4.2.9.1	Test Purpose and Environment	4323

A.10.4.2.9.2	Test Requirements	4328

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4328

A.10.4.2.10.1	Test Purpose and Environment	4328

A.10.4.2.10.2	Test Requirements	4334

A.10.4.3	L1-RSRP measurements for beam reporting	4335

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4335

A.10.4.3.1.1	Test Purpose and Environment	4335

A.10.4.3.1.2	Test parameters	4335

A.10.4.3.1.3	Test Requirements	4337

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4338

A.10.4.3.2.1	Test Purpose and Environment	4338

A.10.4.3.2.2	Test parameters	4338

A.10.4.3.2.3	Test Requirements	4340

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4341

A.10.4.3.3.1	Test Purpose and Environment	4341

A.10.4.3.3.2	Test parameters	4341

A.10.4.3.3.3	Test Requirements	4344

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4345

A.10.4.3.4.1	Test Purpose and Environment	4345

A.10.4.3.4.2	Test parameters	4345

A.10.4.3.4.3	Test Requirements	4348

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4349

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4349

A.10.4.4.1.1	Test Purpose and Environment	4349

A.10.4.4.1.2	Test Requirements	4353

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4354

A.10.4.4.2.1	Test Purpose and Environment	4354

A.10.4.4.2.2	Test Requirements	4359

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4360

A.10.4.4.3.1	Test Purpose and Environment	4360

A.10.4.4.3.2	Test Requirements	4364

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4365

A.10.4.4.4.1	Test Purpose and Environment	4365

A.10.4.4.4.2	Test Requirements	4370

A.10.5	Measurement performance	4371

A.10.5.1	SS-RSRP	4371

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4371

A.10.5.1.1.1	Test Purpose and Environment	4371

A.10.5.1.1.2	Test parameters	4371

A.10.5.1.1.3	Test Requirements	4374

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4374

A.10.5.1.2.1	Test Purpose and Environment	4374

A.10.5.1.2.2	Test parameters	4374

A.10.5.1.2.3	Test Requirements	4377

A.10.5.2	SS-RSRQ	4377

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4377

A.10.5.2.1.1	Test Purpose and Environment	4377

A.10.5.2.1.2	Test Parameters	4377

A.10.5.2.1.3	Test Requirements	4380

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4380

A.10.5.2.2.1	Test Purpose and Environment	4380

A.10.5.2.2.2	Test Parameters	4380

A.10.5.2.2.3	Test Requirements	4383

A.10.5.3	SS-SINR	4383

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4383

A.10.5.3.1.1	Test Purpose and Environment	4383

A.10.5.3.1.2	Test Parameters	4383

A.10.5.3.1.3	Test Requirements	4386

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4386

A.10.5.3.2.1	Test Purpose and Environment	4386

A.10.5.3.2.2	Test Parameters	4386

A.10.5.3.2.3	Test Requirements	4389

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4389

A.10.5.3.3.1	Test Purpose and Environment	4389

A.10.5.3.3.2	Test Parameters	4389

A.10.5.3.3.3	Test Requirements	4392

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4392

A.10.5.4.1	SSB based L1-RSRP measurement	4392

A.10.5.4.1.1	Test Purpose and Environment	4392

A.10.5.4.1.2	Test parameters	4393

A.10.5.4.1.3	Test Requirements	4395

A.10.5.5	RSSI	4395

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4395

A.10.5.5.1.1	Test Purpose and Environment	4395

A.10.5.5.1.2	Test parameters	4395

A.10.5.5.1.3	Test Requirements	4398

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4398

A.10.5.5.2.1	Test Purpose and Environment	4398

A.10.5.5.2.2	Test parameters	4398

A.10.5.5.2.3	Test Requirements	4401

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4401

A.10.5.5.3.1	Test Purpose and Environment	4401

A.10.5.5.3.2	Test parameters	4401

A.10.5.5.3.3	Test Requirements	4404

A.10.5.6	Channel occupancy	4404

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4404

A.10.5.6.1.1	Test Purpose and Environment	4404

A.10.5.6.1.2	Test parameters	4404

A.10.5.6.1.3	Test Requirements	4408

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4408

A.10.5.6.2.1	Test Purpose and Environment	4408

A.10.5.6.2.2	Test parameters	4408

A.10.5.6.2.3	Test Requirements	4411

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4411

A.10.5.6.3.1	Test Purpose and Environment	4411

A.10.5.6.3.2	Test parameters	4411

A.10.5.6.3.3	Test Requirements	4415

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4420

A.11.1	RRC_IDLE state mobility	4420

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4420

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4420

A.11.1.1.1.1	Test Purpose and Environment	4420

A.11.1.1.1.2	Test Parameters	4420

A.11.1.1.1.3	Test Requirements	4423

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4423

A.11.1.1.2.1	Test Purpose and Environment	4423

A.11.1.1.2.2	Test Parameters	4423

A.11.1.1.2.3	Test Requirements	4427

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4427

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4427

A.11.1.2.1.1	Test Purpose and Environment	4427

A.11.1.2.1.2	Test Parameters	4428

A.11.1.2.1.3	Test Requirements	4434

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4435

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4435

A.11.1.3.1.1	Test Purpose and Environment	4435

A.11.1.3.1.2	Test Parameters	4435

A.11.1.3.1.3	Test Requirements	4441

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4442

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4442

A.11.1.4.1.1	Test Purpose and Environment	4442

A.11.1.4.1.2	Test Parameters	4442

A.11.1.4.1.3	Test Requirements	4445

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4446

A.11.1.4.2.1	Test Purpose and Environment	4446

A.11.1.4.2.2	Test Requirements	4449

A.11.2	RRC_CONNECTED state mobility	4450

A.11.2.1	Handover	4450

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4450

A.11.2.1.1.1	Test Purpose and Environment	4450

A.11.2.1.1.2	Test Parameters	4450

A.11.2.1.1.3 Test Requirements	4453

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4453

A.11.2.1.2.1	Test Purpose and Environment	4453

A.11.2.1.2.2	Test Parameters	4453

A.11.2.1.2.3	Test Requirements	4456

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4456

A.11.2.1.3.1	Test Purpose and Environment	4456

A.11.2.1.3.2	Test Parameters	4456

A.11.2.1.3.3 Test Requirements	4459

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4459

A.11.2.1.4.1	Test Purpose and Environment	4459

A.11.2.1.4.2	Test Parameters	4459

A.11.2.1.4.3 Test Requirements	4463

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4464

A.11.2.1.5.1	Test Purpose and Environment	4464

A.11.2.1.5.2	Test Parameters	4464

A.11.2.1.5.3 Test Requirements	4467

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4468

A.11.2.1.6.1	Test Purpose and Environment	4468

A.11.2.1.6.2	Test Parameters	4468

A.11.2.1.6.3 Test Requirements	4471

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4472

A.11.2.1.7.1	Test Purpose and Environment	4472

A.11.2.1.7.2	Test Requirements	4478

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4478

A.11.2.1.8.1	Test Purpose and Environment	4478

A.11.2.1.8.2	Test Requirements	4483

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4483

A.11.2.1.9.1	Test Purpose and Environment	4483

A.11.2.1.9.2	Test Requirements	4492

A.11.2.2	RRC connection mobility control	4493

A.11.2.2.1	RRC re-establishment	4493

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4493

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4496

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4503

A.11.2.2.2	Random Access	4508

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4508

A.11.2.2.2.1.1	Test Purpose and Environment	4508

A.11.2.2.2.1.2	Test Requirements	4511

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4511

A.11.2.2.2.1.2.2	Random Access Response Reception	4511

A.11.2.2.2.1.2.3	No Random Access Response Reception	4512

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4512

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4512

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4513

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4513

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4513

A.11.2.2.2.2.1	Test Purpose and Environment	4513

A.11.2.2.2.2.2	Test Requirements	4516

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4516

A.11.2.2.2.2.2.2	Random Access Response Reception	4517

A.11.2.2.2.2.2.3	No Random Access Response Reception	4517

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4517

A.11.2.2.2.3.1	Test Purpose and Environment	4517

A.11.2.2.2.3.2	Test Requirements	4521

A.11.2.2.2.3.2.1	MsgA Transmission	4521

A.11.2.2.2.3.2.2	MsgB Reception	4521

A.11.2.2.2.3.2.3	No MsgB Reception	4522

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4522

A.11.2.2.2.4.1	Test Purpose and Environment	4522

A.11.2.2.2.4.2	Test Requirements	4526

A.11.2.2.2.4.2.1	MsgA Transmission	4526

A.11.2.2.2.4.2.2	MsgB Reception	4527

A.11.2.2.2.4.2.3	No MsgB Reception	4527

A.11.2.2.3	RRC connection release with redirection	4528

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4528

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4532

A.11.3	Timing	4537

A.11.3.1	UE transmit timing	4537

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4537

A.11.3.1.1.1	Test Purpose and environment	4537

A.11.3.1.1.2	Test requirements	4540

A.11.3.2	UE timing advance	4541

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4541

A.11.3.2.1.1	Test Purpose and Environment	4541

A.11.3.2.1.2	Test Parameters	4541

A.11.3.2.1.3	Test Requirements	4545

A.11.4	Signalling characteristics	4545

A.11.4.1	Radio link monitoring	4545

A.11.4.1.1	Introduction	4545

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4546

A.11.4.1.2.1	Test purpose and environment	4546

A.11.4.1.2.2	Test requirements	4550

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4550

A.11.4.1.3.1	Test purpose and environment	4550

A.11.4.1.3.2	Test requirements	4556

A.11.4.1.4	Void	4556

A.11.4.1.4.1	Void	4556

A.11.4.1.4.2	Void	4556

A.11.4.1.5	Void	4556

A.11.4.1.5.1	Void	4556

A.11.4.1.5.2	Void	4556

A.11.4.2	Interruption	4557

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4557

A.11.4.2.1.1 Test Purpose and Environment	4557

A.11.4.2.1.2	Test Requirements	4561

A.11.4.3	SCell activation and deactivation delay	4561

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4561

A.11.4.3.1.1	Test Purpose and Environment	4561

A.11.4.3.1.2	Test Requirements	4566

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4566

A.11.4.3.2.1	Test Purpose and Environment	4566

A.11.4.3.2.2	Test Requirements	4567

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4567

A.11.4.3.3.1	Test Purpose and Environment	4567

A.11.4.3.3.2	Test Requirements	4568

A.11.4.4	Beam failure detection and link recovery procedures	4568

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4568

A.11.4.4.1.1	Test Purpose and Environment	4568

A.11.4.4.1.2	Test Requirements	4573

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4573

A.11.4.4.2.1	Test Purpose and Environment	4573

A.11.4.4.2.2	Test Requirements	4579

A.11.4.5	Active BWP switching	4579

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4579

A.11.4.5.1.1	Test Purpose and Environment	4579

A.11.4.5.1.2	Test Requirements	4583

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4584

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4584

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4587

A.11.4.5.3	RRC-based Active BWP Switch	4590

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4590

A.11.4.6	Void	4593

A.11.5	Measurement procedure	4593

A.11.5.1	Intra-frequency measurements	4593

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4593

A.11.5.1.1.1	Test purpose and environment	4593

A.11.5.1.1.2	Test parameters	4593

A.11.5.1.1.3	Test Requirements	4597

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4597

A.11.5.1.2.1	Test purpose and environment	4597

A.11.5.1.2.2	Test parameters	4597

A.11.5.1.2.3	Test Requirements	4601

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4601

A.11.5.1.3.1	Test purpose and environment	4601

A.11.5.1.3.2	Test parameters	4601

A.11.5.1.3.3	Test Requirements	4601

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4602

A.11.5.1.4.1	Test purpose and environment	4602

A.11.5.1.4.2	Test parameters	4602

A.11.5.1.4.3	Test Requirements	4602

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4602

A.11.5.1.5.1	Test purpose and environment	4602

A.11.5.1.5.2	Test parameters	4602

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4603

A.11.5.1.6.1	Test purpose and environment	4603

A.11.5.1.6.2	Test parameters	4603

A.11.5.1.6.3	Test Requirements	4603

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4603

A.11.5.1.7.1	Test purpose and environment	4603

A.11.5.1.7.2	Test parameters	4603

A.11.5.1.7.3	Test Requirements	4603

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4604

A.11.5.1.8.1	Test purpose and environment	4604

A.11.5.1.8.2	Test parameters	4604

A.11.5.1.8.3	Test Requirements	4604

A.11.5.1.9	RSSI measurement reporting on PCC	4604

A.11.5.1.9.1	Test purpose and environment	4604

A.11.5.1.9.2	Test parameters	4604

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4605

A.11.5.1.10.1	Test purpose and environment	4605

A.11.5.1.10.2	Test parameters	4605

A.11.5.1.11	RSSI measurement reporting on SCC	4605

A.11.5.1.11.1	Test purpose and environment	4605

A.11.5.1.11.2	Test parameters	4605

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4606

A.11.5.1.12.1	Test purpose and environment	4606

A.11.5.1.12.2	Test parameters	4606

A.11.5.2	Inter-frequency measurements	4606

A.11.5.2.1	RSSI measurement reporting	4606

A.11.5.2.1.1	Test purpose and environment	4606

A.11.5.2.1.2	Test parameters	4606

A.11.5.2.2	Channel occupancy measurement reporting	4607

A.11.5.2.2.1	Test purpose and environment	4607

A.11.5.2.2.2	Test parameters	4607

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4607

A.11.5.2.3.1	Test Purpose and Environment	4607

A.11.5.2.3.2	Test Requirements	4611

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4612

A.11.5.2.4.1	Test Purpose and Environment	4612

A.11.5.2.4.2	Test Requirements	4617

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4617

A.11.5.2.5.1	Test Purpose and Environment	4617

A.11.5.2.5.2	Test Requirements	4621

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4622

A.11.5.2.6.1	Test Purpose and Environment	4622

A.11.5.2.6.2	Test Requirements	4627

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4627

A.11.5.2.7.1	Test Purpose and Environment	4627

A.11.5.2.7.2	Test Requirements	4631

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4632

A.11.5.2.8.1	Test Purpose and Environment	4632

A.11.5.2.8.2	Test Requirements	4637

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4637

A.11.5.2.9.1	Test Purpose and Environment	4637

A.11.5.2.9.2	Test Requirements	4642

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4643

A.11.5.2.10.1	Test Purpose and Environment	4643

A.11.5.2.10.2	Test Requirements	4648

A.11.5.3	Inter-RAT E-UTRAN measurements	4649

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4649

A.11.5.3.1.1	Test Purpose and Environment	4649

A.11.5.3.1.2	Test Requirements	4653

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4653

A.11.5.3.2.1	Test Purpose and Environment	4653

A.11.5.3.2.2	Test Requirements	4658

A.11.5.4	L1-RSRP measurements for beam reporting	4658

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4658

A.11.5.4.1.1	Test Purpose and Environment	4658

A.11.5.4.1.2	Test parameters	4658

A.11.5.4.1.3	Test Requirements	4660

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4661

A.11.5.4.2.1	Test Purpose and Environment	4661

A.11.5.4.2.2	Test parameters	4661

A.11.5.4.2.3	Test Requirements	4663

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4664

A.11.5.4.3.1	Test Purpose and Environment	4664

A.11.5.4.3.2	Test parameters	4664

A.11.5.4.3.3	Test Requirements	4667

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4668

A.11.5.4.4.1	Test Purpose and Environment	4668

A.11.5.4.4.2	Test parameters	4668

A.11.5.4.4.3	Test Requirements	4671

A.11.6	Measurement performance	4672

A.11.6.1	SS-RSRP	4672

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4672

A.11.6.1.1.1	Test Purpose and Environment	4672

A.11.6.1.1.2	Test parameters	4672

A.11.6.1.1.3	Test Requirements	4674

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4674

A.11.6.1.2.1	Test Purpose and Environment	4674

A.11.6.1.2.2	Test parameters	4674

A.11.6.1.2.3	Test Requirements	4676

A.11.6.2	SS-RSRQ	4676

A.11.6.2.1	Intra-frequency measurement accuracy	4676

A.11.6.2.1.1	Test Purpose and Environment	4676

A.11.6.2.1.2	Test Parameters	4676

A.11.6.2.1.3	Test Requirements	4679

A.11.6.2.2	Inter-frequency measurement accuracy	4679

A.11.6.2.2.1	Test Purpose and Environment	4679

A.11.6.2.2.2	Test Parameters	4679

A.11.6.2.2.3	Test Requirements	4682

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4682

A.11.6.2.3.1	Test Purpose and Environment	4682

A.11.6.2.3.2	Test Parameters	4682

A.11.6.2.3.3	Test Requirements	4685

A.11.6.2.4	Inter-frequency measurement accuracy	4685

A.11.6.2.4.1	Test Purpose and Environment	4685

A.11.6.2.4.2	Test Parameters	4685

A.11.6.2.4.3	Test Requirements	4693

A.11.6.3	SS-SINR	4693

A.11.6.3.1	Intra-frequency measurement accuracy	4693

A.11.6.3.1.1	Test Purpose and Environment	4693

A.11.6.3.1.2	Test Parameters	4693

A.11.6.3.1.3	Test Requirements	4697

A.11.6.3.2	Inter-frequency measurement accuracy	4697

A.11.6.3.2.1	Test Purpose and Environment	4697

A.11.6.3.2.2	Test Parameters	4697

A.11.6.3.2.3	Test Requirements	4700

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4700

A.11.6.3.3.1	Test Purpose and Environment	4700

A.11.6.3.3.2	Test Parameters	4700

A.11.6.3.3.3	Test Requirements	4703

A.11.6.3.4	Inter-frequency measurement accuracy	4703

A.11.6.3.4.1	Test Purpose and Environment	4703

A.11.6.3.4.2	Test Parameters	4703

A.11.6.3.4.3	Test Requirements	4711

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4712

A.11.6.4.1	SSB based L1-RSRP measurement	4712

A.11.6.4.1.1	Test Purpose and Environment	4712

A.11.6.4.1.2	Test parameters	4712

A.11.6.4.1.3	Test Requirements	4715

A.11.6.5	RSSI	4715

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4715

A.11.6.5.1.1	Test Purpose and Environment	4715

A.11.6.5.1.2	Test parameters	4715

A.11.6.5.1.3	Test Requirements	4718

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4718

A.11.6.5.2.1	Test Purpose and Environment	4718

A.11.6.5.2.2	Test parameters	4718

A.11.6.5.2.3	Test Requirements	4721

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4721

A.11.6.5.3.1	Test Purpose and Environment	4721

A.11.6.5.3.2	Test parameters	4721

A.11.6.5.3.3	Test Requirements	4724

A.11.6.6	Channel occupancy	4724

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4724

A.11.6.6.1.1	Test Purpose and Environment	4724

A.11.6.6.1.2	Test parameters	4724

A.11.6.6.1.3	Test Requirements	4727

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4727

A.11.6.6.2.1	Test Purpose and Environment	4727

A.11.6.6.2.2	Test parameters	4727

A.11.6.6.2.3	Test Requirements	4730

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4730

A.11.6.6.3.1	Test Purpose and Environment	4730

A.11.6.6.3.2	Test parameters	4730

A.11.6.6.3.3	Test Requirements	4733

A.11.6.7	E-UTRAN RSRP	4733

A.11.6.8	E-UTRAN RSRQ	4733

A.11.6.9	E-UTRAN SINR	4733

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4733

A.12.1	RRC_IDLE state mobility	4733

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4733

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4733

A.12.1.1.1.1	Test Purpose and Environment	4733

A.12.1.1.1.2	Test Requirements	4738

A.12.2	RRC_CONNECTED state mobility	4739

A.12.2.1	Handover	4739

A.12.2.1.1	E-UTRAN - NR with CCA handover	4739

A.12.2.1.1.1	Test Purpose and Environment	4739

A.12.2.1.1.2	Test Requirements	4744

A.12.3	Signalling characteristics	4745

A.12.3.1	Interruptions	4745

A.12.4	Measurement procedure	4745

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4745

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4745

A.12.4.1.1.1	Test Purpose and Environment	4745

A.12.4.1.1.2	Test Requirements	4749

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4749

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4749

A.12.4.2.1.1	Test Purpose and Environment	4749

A.12.4.2.1.2	Test Requirements	4753

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4754

A.12.4.2.2.1	Test Purpose and Environment	4754

A.12.4.2.2.2	Test Requirements	4758

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4759

A.12.4.2.3.1	Test Purpose and Environment	4759

A.12.4.2.3.2	Test Requirements	4763

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4764

A.12.4.2.4.1	Test Purpose and Environment	4764

A.12.4.2.4.2	Test Requirements	4768

A.12.4.2.5	RSSI measurement reporting	4769

A.12.4.2.5.1	Test purpose and environment	4769

A.12.4.2.5.2	Test parameters	4769

A.12.4.2.5.3	Test Requirements	4772

A.12.4.2.6	Channel occupancy measurement reporting	4772

A.12.4.2.6.1	Test purpose and environment	4772

A.12.4.2.6.2	Test parameters	4772

A.12.4.2.6.3	Test Requirements	4776

A.12.5	Measurement performance	4776

A.12.5.1	E-UTRANNR SFTD	4776

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4776

A.12.5.1.1.1	Test Purpose	4776

A.12.5.1.1.2	Test Environment	4776

A.12.5.1.1.3	Test Requirements	4782

A.12.5.2	E-UTRANNR SS-RSRP	4782

A.12.5.3	E-UTRANNR SS-RSRQ	4782

A.12.5.4	E-UTRANNR SS-SINR	4782

A.12.5.5	E-UTRANNR RSSI	4782

A.12.5.6	E-UTRANNR channel occupancy	4782

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4783

A.13.1	Timing	4783

A.13.1.1	UE transmit timing	4783

A.13.1.2	Timing advance	4783

A.13.2	Signalling characteristics	4783

A.13.2.1	Interruption	4783

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4783

A.13.2.1.1.1	Test Purpose and Environment	4783

A.13.2.1.1.2	Test Requirements	4787

A.13.2.2	SCell activation and deactivation delay	4787

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4787

A.13.2.2.1.1	Test Purpose and Environment	4787

A.13.2.2.1.2	Test Requirements	4792

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4792

A.13.2.2.2.1	Test Purpose and Environment	4792

A.13.2.2.2.2	Test Requirements	4793

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4793

A.13.2.2.3.1	Test Purpose and Environment	4793

A.13.2.2.3.2	Test Requirements	4794

A.13.2.3	Void	4794

A.13.3	Measurement procedure	4794

A.13.3.1	Intra-frequency measurements	4794

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4794

A.13.3.1.1.1	Test purpose and environment	4794

A.13.3.1.1.2	Test parameters	4794

A.13.3.1.1.3	Test Requirements	4799

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4799

A.13.3.1.2.1	Test purpose and environment	4799

A.13.3.1.2.2	Test parameters	4799

A.13.3.1.2.3	Test Requirements	4804

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4804

A.13.3.1.3.1	Test purpose and environment	4804

A.13.3.1.3.2	Test parameters	4804

A.13.3.1.3.3	Test Requirements	4809

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4809

A.13.3.1.4.1	Test purpose and environment	4809

A.13.3.1.4.2	Test parameters	4809

A.13.3.1.4.3	Test Requirements	4814

A.13.3.1.5	RSSI measurement reporting on SCC	4814

A.13.3.1.5.1	Test purpose and environment	4814

A.13.3.1.5.2	Test parameters	4814

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4815

A.13.3.1.6.1	Test purpose and environment	4815

A.13.3.1.6.2	Test parameters	4816

A.13.3.2	Inter-frequency measurements	4816

A.13.3.2.1	RSSI measurement reporting	4816

A.13.3.2.1.1	Test purpose and environment	4816

A.13.3.2.1.2	Test parameters	4816

A.13.3.2.2	Channel occupancy measurement reporting	4817

A.13.3.2.2.1	Test purpose and environment	4817

A.13.3.2.2.2	Test parameters	4817

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4817

A.13.3.2.3.1	Test Purpose and Environment	4817

A.13.3.2.3.2	Test Requirements	4822

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4822

A.13.3.2.4.1	Test Purpose and Environment	4822

A.13.3.2.4.2	Test Requirements	4827

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4828

A.13.3.2.5.1	Test Purpose and Environment	4828

A.13.3.2.5.2	Test Requirements	4833

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4833

A.13.3.2.6.1	Test Purpose and Environment	4833

A.13.3.2.6.2	Test Requirements	4839

A.13.3.3	L1-RSRP measurements for beam reporting	4840

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4840

A.13.3.3.1.1	Test Purpose and Environment	4840

A.13.3.3.1.2	Test parameters	4840

A.13.3.3.1.3	Test Requirements	4844

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4844

A.13.3.3.2.1	Test Purpose and Environment	4844

A.13.3.3.2.2	Test parameters	4844

A.13.3.3.2.3	Test Requirements	4848

A.13.4	Measurement performance	4848

A.13.4.1	SS-RSRP	4848

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4848

A.13.4.1.1.1	Test Purpose and Environment	4848

A.13.4.1.1.2	Test parameters	4848

A.13.4.1.1.3	Test Requirements	4850

A.13.4.2	SS-RSRQ	4850

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4850

A.13.4.2.1.1	Test Purpose and Environment	4850

A.13.4.2.1.2	Test Parameters	4850

A.13.4.2.1.3	Test Requirements	4857

A.13.4.3	SS-SINR	4857

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4857

A.13.4.3.1.1	Test Purpose and Environment	4857

A.13.4.3.1.2	Test Parameters	4857

A.13.4.3.1.3	Test Requirements	4865

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4865

A.13.4.4.1	SSB based L1-RSRP measurement	4865

A.13.4.4.1.1	Test Purpose and Environment	4865

A.13.4.4.1.2	Test parameters	4865

A.13.4.4.1.3	Test Requirements	4869

A.13.4.5	RSSI	4869

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4869

A.13.4.5.1.1	Test Purpose and Environment	4869

A.13.4.5.1.2	Test parameters	4869

A.13.4.5.1.3	Test Requirements	4873

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4873

A.13.4.5.2.1	Test Purpose and Environment	4873

A.13.4.5.2.2	Test parameters	4873

A.13.4.5.2.3	Test Requirements	4877

A.13.4.6	Channel occupancy	4877

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4877

A.13.4.6.1.1	Test Purpose and Environment	4877

A.13.4.6.1.2	Test parameters	4877

A.13.4.6.1.3	Test Requirements	4881

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4881

A.13.4.6.2.1	Test Purpose and Environment	4881

A.13.4.6.2.2	Test parameters	4881

A.13.4.6.2.3	Test Requirements	4885

A.14	NR standalone tests for Satellite access	4885

A.14.1	RRC_IDLE state mobility	4885

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4885

A.14.1.1.1	Test Purpose and Environment	4885

A.14.1.1.2	Test Parameters	4885

A.14.1.1.3	Test Requirements	4887

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4888

A.14.1.2.1	Test Purpose and Environment	4888

A.14.1.2.2	Test Parameters	4888

A.14.1.2.3	Test Requirements	4892

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4893

A.14.1.3.1	Test Purpose and Environment	4893

A.14.1.3.2	Test Parameters	4893

A.14.1.3.3	Test Requirements	4894

A.14.1.4	Location-based measurement initiation to FR1 intra-frequency NR cell reselection	4895

A.14.1.4.1	Test Purpose and Environment	4895

A.14.1.4.2	Test Parameters	4895

A.14.1.4.3	Test Requirements	4897

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4898

A.14.1.5.1	Test Purpose and Environment	4898

A.14.1.5.2	Test Parameters	4898

A.14.1.5.3	Test Requirements	4900

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4901

A.14.1.6.1	Test Purpose and Environment	4901

A.14.1.6.2	Test Parameters	4901

A.14.1.6.3	Test Requirements	4903

A.14.1.7	Time-based measurement initiation to FR1 inter-frequency cell reselection	4904

A.14.1.7.1	Test Purpose and Environment	4904

A.14.1.7.2	Test Parameters	4904

A.14.1.7.3	Test Requirements	4905

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4906

A.14.1.8.1	Test Purpose and Environment	4906

A.14.1.8.2	Test Parameters	4906

A.14.1.8.3	Test Requirements	4908

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4909

A.14.1.9.1	Test Purpose and Environment	4909

A.14.1.9.2	Test Parameters	4909

A.14.1.9.3	Test Requirements	4912

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4912

A.14.1.10.1	Test Purpose and Environment	4912

A.14.1.10.2	Test Parameters	4912

A.14.1.10.3	Test Requirements	4914

A.14.2	RRC_CONNECTED state mobility	4915

A.14.2.1	Handover	4915

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4915

A.14.2.1.1.1	Test Purpose and Environment	4915

A.14.2.1.1.2	Test Parameters	4915

A.14.2.1.1.3	Test Requirements	4917

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4918

A.14.2.1.2.1	Test Purpose and Environment	4918

A.14.2.1.2.2	Test Parameters	4918

A.14.2.1.2.3	Test Requirements	4920

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4920

A.14.2.1.3.1	Test Purpose and Environment	4920

A.14.2.1.3.2	Test Parameters	4920

A.14.2.1.3.3	Test Requirements	4923

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4923

A.14.2.1.4.1	Test Purpose and Environment	4923

A.14.2.1.4.2	Test Parameters	4923

A.14.2.1.4.3	Test Requirements	4925

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4925

A.14.2.1.5.1	Test Purpose and Environment	4925

A.14.2.1.5.2	Test Parameters	4926

A.14.2.1.5.3	Test Requirements	4929

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4929

A.14.2.1.6.1	Test Purpose and Environment	4929

A.14.2.1.6.2	Test Parameters	4929

A.14.2.1.6.3	Test Requirements	4933

A.14.2.2	RRC Connection Mobility Control	4933

A.14.2.2.1	SA: RRC Re-establishment for SAN	4933

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4933

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4936

A.14.2.2.2	Random Access	4939

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4939

A.14.2.2.2.1.1	Test Purpose and Environment	4939

A.14.2.2.2.1.2	Test Requirements	4942

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4943

A.14.2.2.2.2.1	Test Purpose and Environment	4943

A.14.2.2.2.2.2	Test Requirements	4947

A.14.2.2.3	RRC Connection Release with Redirection	4948

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4948

A.14.2.2.3.1.1	Test Purpose and Environment	4948

A.14.2.2.3.1.2	Test Parameters	4948

A.14.2.2.3.1.3	Test Requirements	4951

A.14.3	Timing for Satellite Access	4951

A.14.3.1	UE transmit timing for Satellite Access	4951

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4951

A.14.3.1.1.1	Test Purpose and environment	4951

A.14.3.1.1.2	Test requirements	4953

A.14.3.2	Timing advance for satellite access	4954

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4954

A.14.3.2.1.1	Test Purpose and Environment	4954

A.14.3.2.1.2	Test Parameters	4954

A.14.3.2.1.3	Test Requirements	4957

A.14.4	Signalling characteristics	4957

A.14.4.1	Radio link Monitoring	4957

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4957

A.14.4.1.1.1	Test Purpose and Environment	4957

A.14.4.1.1.2	Test Requirements	4962

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4962

A.14.4.1.2.1	Test Purpose and Environment	4962

A.14.4.1.2.2	Test Requirements	4968

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4968

A.14.4.1.3.1	Test Purpose and Environment	4968

A.14.4.1.3.2	Test Requirements	4972

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4972

A.14.4.1.4.1	Test Purpose and Environment	4972

A.14.4.1.4.2	Test Requirements	4977

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4977

A.14.4.1.5.1	Test Purpose and Environment	4977

A.14.4.1.5.2	Test Requirements	4982

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4982

A.14.4.1.6.1	Test Purpose and Environment	4982

A.14.4.1.6.2	Test Requirements	4986

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4986

A.14.4.1.7.1	Test Purpose and Environment	4986

A.14.4.1.7.2	Test Requirements	4991

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4991

A.14.4.1.8.1	Test Purpose and Environment	4991

A.14.4.1.8.2	Test Requirements	4996

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4996

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4996

A.14.4.2.1.1	Test Purpose and Environment	4996

A.14.4.2.1.2	Test Requirements	5002

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	5002

A.14.4.2.2.1	Test Purpose and Environment	5002

A.14.4.2.2.2	Test Requirements	5007

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	5007

A.14.4.2.3.1	Test Purpose and Environment	5007

A.14.4.2.3.2	Test Requirements	5013

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	5013

A.14.4.2.4.1	Test Purpose and Environment	5013

A.14.4.2.4.2	Test Requirements	5018

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	5018

A.14.4.2.5.1	Test Purpose and Environment	5018

A.14.4.2.5.2	Test Requirements	5023

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	5023

A.14.4.2.6.1	Test Purpose and Environment	5023

A.14.4.2.6.2	Test Requirements	5028

A.14.4.3	Active BWP switch for satellite access	5028

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	5028

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	5028

A.14.4.3.1.1.1	Test Purpose and Environment	5028

A.14.4.3.1.1.2	Test Requirements	5033

A.14.4.3.2	RRC-based Active BWP Switch	5033

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	5033

A.14.4.3.2.1.1	Test Purpose and Environment	5033

A.14.4.3.2.1.2	Test Requirements	5037

A.14.4.4	UE specific CBW change for satellite access	5037

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	5037

A.14.4.4.1.1	Test Purpose and Environment	5037

A.14.4.4.1.2	Test Requirements	5041

A.14.4.5	Pathloss reference signal switching delay	5041

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	5041

A.14.4.5.1.1	Test Purpose and Environment	5041

A.14.4.5.1.2	Test Requirements	5043

A.14.5	Measurement procedure	5044

A.14.5.1	Intra-frequency Measurements	5044

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	5044

A.14.5.1.1.1	Test purpose and Environment	5044

A.14.5.1.1.2	Test parameters	5044

A.14.5.1.1.3	Test Requirements	5046

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	5047

A.14.5.1.2.1	Test purpose and Environment	5047

A.14.5.1.2.2	Test parameters	5047

A.14.5.1.2.3	Test Requirements	5048

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	5049

A.14.5.1.3.1	Test purpose and Environment	5049

A.14.5.1.3.2	Test parameters	5049

A.14.5.1.3.3	Test Requirements	5051

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	5052

A.14.5.1.4.1	Test purpose and Environment	5052

A.14.5.1.4.2	Test parameters	5052

A.14.5.1.4.3	Test Requirements	5054

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	5055

A.14.5.1.5.1	Test purpose and Environment	5055

A.14.5.1.5.2	Test parameters	5055

A.15.5.1.5.3	Test Requirements	5057

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	5058

A.14.5.1.6.1	Test purpose and Environment	5058

A.14.5.1.6.2	Test parameters	5058

A.14.5.1.6.3	Test Requirements	5060

A.14.5.2	Inter-frequency Measurements	5061

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	5061

A.14.5.2.1.1	Test Purpose and Environment	5061

A.14.5.2.1.2	Test Requirements	5065

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	5065

A.14.5.2.2.1	Test Purpose and Environment	5065

A.14.5.2.2.2	Test Requirements	5069

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	5069

A.14.5.2.3.1	Test Purpose and Environment	5069

A.14.5.2.3.2	Test Requirements	5073

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	5073

A.14.5.2.4.1	Test Purpose and Environment	5073

A.14.5.2.4.2	Test Requirements	5077

A.14.5.2.5	void	5077

A.14.5.2.5.1	void	5077

A.14.5.2.5.2	void	5077

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	5077

A.14.5.2.6.1	Test Purpose and Environment	5077

A.14.5.2.6.2	Test Requirements	5081

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	5081

A.14.5.2.7.1	Test purpose and Environment	5081

A.14.5.2.7.2	Test parameters	5081

A.14.5.2.7.3	Test Requirements	5083

A.14.5.2.8	Event triggered reporting tests without gap under DRX	5084

A.14.5.2.8.1	Test purpose and Environment	5084

A.14.5.2.8.2	Test parameters	5084

A.14.5.2.8.3	Test Requirements	5085

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	5086

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	5086

A.14.5.3.1.1	Test Purpose and Environment	5086

A.14.5.3.1.2	Test parameters	5086

A.14.5.3.1.3	Test Requirements	5089

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	5089

A.14.5.3.2.1	Test Purpose and Environment	5089

A.14.5.3.2.2	Test parameters	5089

A.14.5.3.2.3	Test Requirements	5093

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	5093

A.14.5.3.3.1	Test Purpose and Environment	5093

A.14.5.3.3.2	Test parameters	5093

A.14.5.3.3.3	Test Requirements	5097

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	5097

A.14.5.3.4.1	Test Purpose and Environment	5097

A.14.5.3.4.2	Test parameters	5098

A.14.5.3.4.3	Test Requirements	5101

A.14.6	Measurement Performance requirements	5101

A.14.6.1	 SS-RSRP for SAN	5101

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5101

A.14.6.1.1.1	Test Purpose and Environment	5101

A.14.6.1.1.2	Test parameters	5101

A.14.6.1.1.3	Test Requirements	5105

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5105

A.14.6.1.2.1	Test Purpose and Environment	5105

A.14.6.1.2.2	Test parameters	5105

A.14.6.1.2.3	Test Requirements	5108

A.14.6.2	SS-RSRQ	5108

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5108

A.14.6.2.1.1	Test Purpose and Environment	5108

A.14.6.2.1.2	Test Parameters	5108

A.14.6.2.1.3	Test Requirements	5111

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5111

A.14.6.2.2.1	Test Purpose and Environment	5111

A.14.6.2.2.2	Test Parameters	5111

A.14.6.2.2.3	Test Requirements	5114

A.14.6.3	SS-SINR	5114

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5114

A.14.6.3.1.1	Test Purpose and Environment	5114

A.14.6.3.1.2	Test Parameters	5114

A.14.6.3.1.3	Test Requirements	5118

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5118

A.14.6.3.2.1	Test Purpose and Environment	5118

A.14.6.3.2.2	Test Parameters	5118

A.14.6.3.2.3	Test Requirements	5122

A.14.6.4	L1-RSRP measurement for beam reporting	5122

A.14.6.4.1	SSB based L1-RSRP measurement	5122

A.14.6.4.1.1	Test Purpose and Environment	5122

A.14.6.4.1.2	Test parameters	5122

A.14.6.4.1.3	Test Requirements	5125

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5126

A.14.6.4.2.1	Test Purpose and Environment	5126

A.14.6.4.2.2	Test parameters	5126

A.14.6.4.2.3	Test Requirements	5129

A.15	NR standalone tests with one or more NR cells in FR2-2	5122

A.15.1	SA: RRC_IDLE state mobility	5122

A.15.1.1	Cell re-selection to NR	5122

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	5122

A.15.1.1.1.1	Test Purpose and Environment	5122

A.15.1.1.1.2	Test Parameters	5122

A.15.1.1.1.3	Test Requirements	5126

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5126

A.15.1.2.1	Test Purpose and Environment	5126

A.15.1.2.2	Test Parameters	5126

A.15.1.2.3	Test Requirements	5130

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5130

A.15.1.3.1	Test Purpose and Environment	5130

A.15.1.3.2	Test Parameters	5130

A.15.1.3.3	Test Requirements	5133

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5133

A.15.1.4.1	Test Purpose and Environment	5133

A.15.1.4.2	Test Parameters	5133

A.15.1.4.3	Test Requirements	5136

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5136

A.15.1.5.1	Test Purpose and Environment	5136

A.15.1.5.2	Test Parameters	5136

A.15.1.5.3	Test Requirements	5140

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5140

A.15.1.6.1	Test Purpose and Environment	5140

A.15.1.6.2	Test Parameters	5140

A.15.1.6.3	Test Requirements	5143

A.15.2	Signaling characteristics	5144

A.15.2.1	SCell Activation and Deactivation Delay	5144

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5144

A.15.2.1.1.1	Test Purpose and Environment	5144

A.15.2.1.1.2	Test Requirements	5146

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5146

A.15.2.1.2.1	Test Purpose and Environment	5146

A.15.2.1.2.2	Test Requirements	5151

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5152

A.15.2.1.3.1	Test Purpose and Environment	5152

A.15.2.1.3.2	Test Requirements	5155

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5156

A.15.2.1.4.1	Test Purpose and Environment	5156

A.15.2.1.4.2	Test Requirements	5159

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5160

A.15.2.1.5.1	Test Purpose and Environment	5160

A.15.2.1.5.2	Test Requirements	5164

A.15.3	RRC_CONNECTED state mobility	5165

A.15.3.1	Handover	5165

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5165

A.15.3.1.1.1	Test Purpose and Environment	5165

A.15.3.1.1.2	Test Parameters	5165

A.15.3.1.1.3	Test Requirements	5169

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5169

A.15.3.1.2.1	Test Purpose and Environment	5169

A.15.3.1.2.2	Test Parameters	5169

A.15.3.1.2.3	Test Requirements	5173

A.15.4	Measurement procedure	5174

A.15.4.1	Intra-frequency Measurements	5174

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5174

A.15.4.1.1.1	Test purpose and Environment	5174

A.15.4.1.1.2	Test Requirements	5176

A.15.4.2	Inter-frequency Measurements	5177

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5177

A.15.4.2.1.1	Test Purpose and Environment	5177

A.15.4.2.1.2	Test Requirements	5180

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5181

A.16.1	SA: RRC_IDLE state mobility for RedCap	5181

A.16.1.1	Cell re-selection to NR	5181

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5181

A.16.1.1.1.1	Test Purpose and Environment	5181

A.16.1.1.1.2	Test Parameters	5181

A.16.1.1.1.3	Test Requirements	5185

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5185

A.16.1.1.2.1	Test Purpose and Environment	5185

A.16.1.1.2.2	Test Parameters	5185

A.16.1.1.2.3	Test Requirements	5189

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5189

A.16.1.1.3.1	Test Purpose and Environment	5189

A.16.1.1.3.2	Test Parameters	5189

A.16.1.1.3.3	Test Requirements	5194

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5194

A.16.1.1.4.1	Test Purpose and Environment	5194

A.16.1.1.4.2	Test Parameters	5194

A.16.1.1.4.3	Test Requirements	5199

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5199

A.16.1.1.5.1	Test Purpose and Environment	5199

A.16.1.1.5.2	Test Parameters	5199

A.16.1.1.5.3	Test Requirements	5204

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5204

A.16.1.1.6.1	Test Purpose and Environment	5204

A.16.1.1.6.2	Test Parameters	5204

A.16.1.1.6.3	Test Requirements	5209

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5209

A.16.1.1.7.1	Test Purpose and Environment	5209

A.16.1.1.7.2	Test Parameters	5209

A.16.1.1.7.3	Test Requirements	5214

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5214

A.16.1.1.8.1	Test Purpose and Environment	5214

A.16.1.1.8.2	Test Parameters	5215

A.16.1.1.8.3	Test Requirements	5219

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5220

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5220

A.16.1.2.1.1	Test Purpose and Environment	5220

A.16.1.2.1.2	Test Parameters	5220

A.16.1.2.1.3	Test Requirements	5223

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5224

A.16.1.2.2.1	Test Purpose and Environment	5224

A.16.1.2.2.2	Test Parameters	5224

A.16.1.2.2.3	Test Requirements	5227

A.16.1.2.3.1	Test Purpose and Environment	5228

A. 16.1.2.3.2	Test Parameters	5228

A.16.1.2.3.3	Test Requirements	5231

A.16.1.2.4.1	Test Purpose and Environment	5232

A.16.1.2.4.2	Test Parameters	5232

A.16.1.3.1.3	Test Requirements	5235

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5236

A.16.1.2.5.1	Test Purpose and Environment	5236

A.16.1.2.5.2	Test Parameters	5236

A.16.1.2.5.3	Test Requirements	5239

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5240

A.16.1.2.6.1	Test Purpose and Environment	5240

A.16.1.2.6.2	Test Parameters	5240

A.16.1.2.6.3	Test Requirements	5243

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5244

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5244

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5244

A.16.2.1.1.1	Test purpose and Environment	5244

A.16.2.1.1.2	Test Parameters	5245

A.16.2.1.1.3	Test requirements	5248

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5248

A.16.2.1.2.1	Test purpose and Environment	5248

A.16.2.1.2.2	Test Parameters	5250

A.16.2.1.2.3	Test requirements	5253

A.16.3	RRC_CONNECTED state mobility for RedCap	5253

A.16.3.1	Handover	5253

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5253

A.16.3.1.1.1	Test Purpose and Environment	5253

A.16.3.1.1.2	Test Parameters	5254

A.16.3.1.1.3	Test Requirements	5257

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5257

A.16.3.1.2.1	Test Purpose and Environment	5257

A.16.3.1.2.2	Test Parameters	5257

A.16.3.1.2.3	Test Requirements	5261

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5261

A.16.3.1.3.1	Test Purpose and Environment	5261

A.16.3.1.3.2	Test Parameters	5261

A.16.3.1.3.3	Test Requirements	5265

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5265

A.16.3.1.4.1	Test Purpose and Environment	5265

A.16.3.1.4.2	Test Parameters	5265

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5269

A.16.3.1.5.1	Test Purpose and Environment	5269

A.16.3.1.5.2	Test Parameters	5269

A.16.3.1.5.3	Test Requirements	5273

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5273

A.16.3.1.6.1	Test Purpose and Environment	5273

A.16.3.1.6.2	Test Parameters	5273

A.16.3.1.6.3	Test Requirements	5277

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5277

A.16.3.1.7.1	Test Purpose and Environment	5277

A.16.3.1.7.2	Test Requirements	5283

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5283

A.16.3.1.8.1	Test Purpose and Environment	5283

A.16.3.1.8.2	Test Requirements	5289

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5289

A.16.3.1.9.1	Test Purpose and Environment	5289

A.16.3.1.9.2	Test Requirements	5296

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5296

A.16.3.1.10.1	Test Purpose and Environment	5296

A.16.3.1.10.2	Test Requirements	5302

A.16.3.2	RRC Connection Mobility Control	5302

A.16.3.2.1	SA: RRC Re-establishment	5302

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5302

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5306

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5310

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5315

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5320

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5324

A.16.3.2.2	Random Access	5328

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5328

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5334

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5339

A.16.3.2.2.4	4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	5344

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5349

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5353

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5357

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5361

A.16.3.2.3	SA: RRC Connection Release with Redirection	5365

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5365

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5369

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5373

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5380

A.16.4	Timing for RedCap	5387

A.16.4.1	UE transmit timing	5387

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5387

A.16.4.1.1.1	Test Purpose and environment	5387

A.16.4.1.1.2	Test requirements	5390

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5390

A.16.4.1.2.1	Test Purpose and environment	5390

A.16.4.1.2.2	Test requirements	5392

A.16.4.2	UE timer accuracy	5393

A.16.4.3	Timing advance	5393

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5393

A.16.4.3.1.1	Test Purpose and Environment	5393

A.16.4.3.1.2	Test Parameters	5393

A.16.4.3.1.3	Test Requirements	5397

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5397

A.16.4.3.2.1	Test Purpose and Environment	5397

A.16.4.3.2.2	Test Parameters	5398

A.16.4.3.2.3	Test Requirements	5401

A.16.5	Signalling characteristics for RedCap	5402

A.16.5.1	Radio link Monitoring	5402

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5402

A.16.5.1.1.1	Test Purpose and Environment	5402

A.16.5.1.1.2	Test Requirements	5407

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5407

A.16.5.1.2.1	Test Purpose and Environment	5407

A.16.5.1.2.2	Test Requirements	5413

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5413

A.16.5.1.3.1	Test Purpose and Environment	5413

A.16.5.1.3.2	Test Requirements	5419

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5419

A.16.5.1.4.1	Test Purpose and Environment	5419

A.16.5.1.4.2	Test Requirements	5425

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5425

A.16.5.1.5.1	Test Purpose and Environment	5425

A.16.5.1.5.2	Test Requirements	5431

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5431

A.16.5.1.6.1	Test Purpose and Environment	5431

A.16.5.1.6.2	Test Requirements	5436

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5436

A.16.5.1.7.1	Test Purpose and Environment	5436

A.16.5.1.7.2	Test Requirements	5441

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5441

A.16.5.1.8.1	Test Purpose and Environment	5441

A.16.5.1.8.2	Test Requirements	5447

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5447

A.16.5.1.9.1	Test Purpose and Environment	5447

A.16.5.1.9.2	Test Requirements	5453

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5453

A.16.5.1.10.1	Test Purpose and Environment	5453

A.16.5.1.10.2	Test Requirements	5459

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5459

A.16.5.1.11.1	Test Purpose and Environment	5459

A.16.5.1.11.2	Test Requirements	5465

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5465

A.16.5.1.12.1	Test Purpose and Environment	5465

A.16.5.1.12.2	Test Requirements	5471

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5471

A.16.5.1.13.1	Test Purpose and Environment	5471

A.16.5.1.13.2	Test Requirements	5477

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5477

A.16.5.1.14.1	Test Purpose and Environment	5477

A.16.5.1.14.2	Test Requirements	5483

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5483

A.16.5.1.15.1	Test Purpose and Environment	5483

A.16.5.1.15.2	Test Requirements	5489

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5489

A.16.5.1.16.1	Test Purpose and Environment	5489

A.16.5.1.16.2	Test Requirements	5495

A.16.5.2	Beam Failure Detection and Link recovery procedures	5495

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5495

A.16.5.2.1.1	Test Purpose and Environment	5495

A.16.5.2.1.2	Test Requirements	5501

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5501

A.16.5.2.2.1	Test Purpose and Environment	5501

A.16.5.2.2.2	Test Requirements	5507

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5507

A.16.5.2.3.1	Test Purpose and Environment	5507

A.16.5.2.3.2	Test Requirements	5514

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5514

A.16.5.2.4.1	Test Purpose and Environment	5514

A.16.5.2.4.2	Test Requirements	5520

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5520

A.16.5.2.5.1	Test Purpose and Environment	5520

A.16.5.2.5.2	Test Requirements	5526

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5526

A.16.5.2.6.1	Test Purpose and Environment	5526

A.16.5.2.6.2	Test Requirements	5532

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5532

A.16.5.2.7.1	Test Purpose and Environment	5532

A.16.5.2.7.2	Test Requirements	5538

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5538

A.16.5.2.8.1	Test Purpose and Environment	5538

A.16.5.2.8.2	Test Requirements	5544

A.16.5.3	Active BWP switch	5544

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5544

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5544

A.16.5.3.1.1.1	Test Purpose and Environment	5544

A.16.5.3.1.1.2	Test Requirements	5549

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5549

A.16.5.3.1.2.1	Test Purpose and Environment	5549

A.16.5.3.1.2.2	Test Requirements	5554

A.16.5.3.2	RRC-based Active BWP Switch	5554

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5554

A.16.5.3.2.1.1	Test Purpose and Environment	5554

A.16.5.3.2.1.2	Test Requirements	5558

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5558

A.16.5.3.2.2.1	Test Purpose and Environment	5558

A.16.5.3.2.2.2	Test Requirements	5562

A.16.5.4	UE specific CBW change	5562

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5562

A.16.5.4.1.1	Test Purpose and Environment	5562

A.16.5.4.1.2	Test Requirements	5566

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5566

A.16.5.4.2.1	Test Purpose and Environment	5566

A.16.5.4.2.2	Test Requirements	5570

A.16.6	Measurement procedure for RedCap	5570

A.16.6.1	Intra-frequency Measurements	5570

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5570

A.16.6.1.1.1	Test purpose and Environment	5570

A.16.6.1.1.2	Test parameters	5570

A.16.6.1.1.3	Test Requirements	5574

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5574

A.16.6.1.2.1	Test purpose and Environment	5574

A.16.6.1.2.2	Test parameters	5574

A.16.6.1.2.3	Test Requirements	5578

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5578

A.16.6.1.3.1	Test purpose and Environment	5578

A.16.6.1.3.2	Test parameters	5578

A.16.6.1.3.3	Test Requirements	5582

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5582

A.16.6.1.4.1	Test purpose and Environment	5582

A.16.6.1.4.2	Test parameters	5582

A.16.6.1.4.3	Test Requirements	5586

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5586

A.16.6.1.5.1	Test purpose and Environment	5586

A.16.6.1.5.2	Test parameters	5586

A.16.6.1.5.3	Test Requirements	5590

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5590

A.16.6.1.6.1	Test purpose and Environment	5590

A.16.6.1.6.2	Test parameters	5590

A.16.6.1.6.3	Test Requirements	5594

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5594

A.16.6.1.7.1	Test purpose and Environment	5594

A.16.6.1.7.2	Test parameters	5594

A.16.6.1.7.3	Test Requirements	5598

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5598

A.16.6.1.8.1	Test purpose and Environment	5598

A.16.6.1.8.2	Test parameters	5598

A.16.6.1.8.3	Test Requirements	5602

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5602

A.16.6.1.9.1	Test purpose and Environment	5602

A.16.6.1.9.2	Test parameters	5602

A.16.6.1.9.3	Test Requirements	5604

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5605

A.16.6.1.10.1	Test purpose and Environment	5605

A.16.6.1.10.2	Test parameters	5605

A.16.6.1.10.3	Test Requirements	5606

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5607

A.16.6.1.11.1	Test purpose and Environment	5607

A.16.6.1.11.2	Test parameters	5607

A.16.6.1.11.3	Test Requirements	5609

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5610

A.16.6.1.12.1	Test purpose and Environment	5610

A.16.6.1.12.2	Test parameters	5610

A.16.6.1.12.3	Test Requirements	5612

A.16.6.2	Inter-frequency Measurements	5613

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5613

A.16.6.2.1.1	Test Purpose and Environment	5613

A.16.6.2.1.2	Test Requirements	5618

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5618

A.16.6.2.2.1	Test Purpose and Environment	5618

A.16.6.2.2.2	Test Requirements	5623

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5623

A.16.6.2.3.1	Test Purpose and Environment	5623

A.16.6.2.3.2	Test Requirements	5626

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5627

A.16.6.2.4.1	Test Purpose and Environment	5627

A.16.6.2.4.2	Test Requirements	5630

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5631

A.16.6.2.5.1	Test Purpose and Environment	5631

A.16.6.2.5.2	Test Requirements	5634

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5635

A.16.6.2.6.1	Test Purpose and Environment	5635

A.16.6.2.6.2	Test Requirements	5638

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5639

A.16.6.2.7.1	Test Purpose and Environment	5639

A.16.6.2.7.2	Test Requirements	5643

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5644

A.16.6.2.8.1	Test Purpose and Environment	5644

A.16.6.2.8.2	Test Requirements	5648

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5649

A.16.6.2.9.1	Test Purpose and Environment	5649

A.16.6.2.9.2	Test Requirements	5652

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5652

A.16.6.2.10.1	Test Purpose and Environment	5652

A.16.6.2.10.2	Test Requirements	5655

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5655

A.16.6.2.11.1	Test Purpose and Environment	5655

A.16.6.2.11.2	Test Requirements	5658

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5658

A.16.6.2.12.1	Test Purpose and Environment	5658

A.16.6.2.12.2	Test Requirements	5662

A.16.6.3	Inter-RAT Measurements	5663

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5663

A.16.6.3.1.1	Test purpose and Environment	5663

A.16.6.3.1.2	Test Requirements	5668

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5668

A.16.6.3.2.1	Test purpose and Environment	5668

A.16.6.3.2.2	Test Requirements	5673

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5673

A.16.6.3.3.1	Test purpose and Environment	5673

A.16.6.3.3.2	Test Requirements	5678

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5678

A.16.6.3.4.1	Test purpose and Environment	5678

A.16.6.3.4.2	Test Requirements	5683

A.16.6.4	L1-RSRP measurement for beam reporting	5683

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5683

A.16.6.4.1.1	Test Purpose and Environment	5683

A.16.6.4.1.2	Test parameters	5683

A.16.6.4.1.3	Test Requirements	5686

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5686

A.16.6.4.2.1	Test Purpose and Environment	5686

A.16.6.4.2.2	Test parameters	5687

A.16.6.4.2.3	Test Requirements	5690

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5690

A.16.6.4.3.1	Test Purpose and Environment	5690

A.16.6.4.3.2	Test parameters	5691

A.16.6.4.3.3	Test Requirements	5694

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5694

A.16.6.4.4.1	Test Purpose and Environment	5694

A.16.6.4.4.2	Test parameters	5695

A.16.6.4.4.3	Test Requirements	5698

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5698

A.16.6.4.5.1	Test Purpose and Environment	5698

A.16.6.4.5.2	Test parameters	5699

A.16.6.4.5.3	Test Requirements	5702

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5702

A.16.6.4.6.1	Test Purpose and Environment	5702

A.16.6.4.6.2	Test parameters	5703

A.16.6.4.6.3	Test Requirements	5706

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5706

A.16.6.4.7.1	Test Purpose and Environment	5706

A.16.6.4.7.2	Test parameters	5707

A.16.6.4.7.3	Test Requirements	5710

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5710

A.16.6.4.8.1	Test Purpose and Environment	5710

A.16.6.4.8.2	Test parameters	5711

A.16.6.4.8.3	Test Requirements	5714

A.16.6.5	NR measurements with autonomous gaps	5714

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5714

A.16.6.5.1.1	Test Purpose and Environment	5714

A.16.6.5.1.2	Test Parameters	5714

A.16.6.5.1.3	Test Requirements	5717

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5717

A.16.6.5.2.1	Test Purpose and Environment	5717

A.16.6.5.2.2	Test Parameters	5717

A.16.6.5.2.3	Test Requirements	5720

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5720

A.16.6.5.3.1	Test Purpose and Environment	5720

A.16.6.5.3.2	Test Requirements	5727

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5727

A.16.6.5.4.1	Test Purpose and Environment	5727

A.16.6.5.4.2	Test Requirements	5734

A.16.7	Measurement Performance requirements for RedCap	5734

A.16.7.1	SS-RSRP	5734

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5734

A.16.7.1.1.1	Test Purpose and Environment	5734

A.16.7.1.1.2	Test parameters	5734

A.16.7.1.1.3	Test Requirements	5739

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5739

A.16.7.1.2.1	Test Purpose and Environment	5739

A.16.7.1.2.2	Test parameters	5739

A.16.7.1.2.3	Test Requirements	5744

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5744

A.16.7.1.3.1	Test Purpose and Environment	5744

A.16.7.1.3.2	Test parameters	5744

A.16.7.1.3.3	Test Requirements	5748

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5749

A.16.7.1.4.1	Test Purpose and Environment	5749

A.16.7.1.4.2	Test parameters	5749

A.16.7.1.4.3	Test Requirements	5753

A.16.7.2	SS-RSRQ	5754

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5754

A.16.7.2.1.1	Test Purpose and Environment	5754

A.16.7.2.1.2	Test Parameters	5754

A.16.7.2.1.3	Test Requirements	5759

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5759

A.16.7.2.2.1	Test Purpose and Environment	5759

A.16.7.2.2.2	Test Parameters	5759

A.16.7.2.2.3	Test Requirements	5764

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5764

A.16.7.2.3.1	Test Purpose and Environment	5764

A.16.7.2.3.2	Test parameters	5764

A.16.7.2.3.3	Test Requirements	5769

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5769

A.16.7.2.4.1	Test Purpose and Environment	5769

A.16.7.2.4.2	Test parameters	5770

A.16.7.2.4.3	Test Requirements	5775

A.16.7.3	SS-SINR	5775

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5775

A.16.7.3.1.1	Test Purpose and Environment	5775

A.16.7.3.1.2	Test parameters	5776

A.16.7.3.1.3	Test Requirements	5780

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5781

A.16.7.3.2.1	Test Purpose and Environment	5781

A.16.7.3.2.2	Test parameters	5781

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5786

A.16.7.3.3.1	Test Purpose and Environment	5786

A.16.7.3.3.2	Test parameters	5786

A.16.7.3.3.3	Test Requirements	5791

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5791

A.16.7.3.4.1	Test Purpose and Environment	5791

A.16.7.3.4.2	Test parameters	5791

A.16.7.3.4.3	Test Requirements	5796

A.16.7.4	L1-RSRP measurement for beam reporting	5796

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5796

A.16.7.4.1.1	Test Purpose and Environment	5796

A.16.7.4.1.2	Test parameters	5797

A.16.7.4.1.3	Test Requirements	5801

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5801

A.16.7.4.2.1	Test Purpose and Environment	5801

A.16.7.4.2.2	Test parameters	5801

A.16.7.4.2.3	Test Requirements	5802

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5802

A.16.7.4.3.1	Test Purpose and Environment	5802

A.16.7.4.3.2	Test parameters	5802

A.16.7.4.3.3	Test Requirements	5806

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5806

A.16.7.4.4.1	Test Purpose and Environment	5806

A.16.7.4.4.2	Test parameters	5807

A.16.7.4.4.3	Test Requirements	5807

A.16.7.5	E-UTRAN RSRP	5807

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5807

A.16.7.5.1.1	Test Purpose and Environment	5807

A.16.7.5.1.2	Test parameters	5807

A.16.7.5.1.3	Test Requirements	5813

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5814

A.16.7.5.2.1	Test Purpose and Environment	5814

A.16.7.5.2.2	Test parameters	5814

A.16.7.5.2.3	Test Requirements	5820

A.16.7.6	E-UTRAN RSRQ	5821

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5821

A.16.7.6.1.1	Test Purpose and Environment	5821

A.16.7.6.1.2	Test parameters	5821

A.16.7.6.1.3	Test Requirements	5827

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5828

A.16.7.6.2.1	Test Purpose and Environment	5828

A.16.7.6.2.2	Test parameters	5828

A.16.7.6.2.3	Test Requirements	5834

A.16.7.7	E-UTRAN RS-SINR	5835

A.16.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5835

A.16.7.7.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5835

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5837

A.17.1	SA: RRC_IDLE state mobility for RedCap	5837

A.17.1.1	Cell re-selection to NR	5837

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5837

A.17.1.1.1.1	Test Purpose and Environment	5837

A.17.1.1.1.2	Test Parameters	5837

A.17.1.1.1.3	Test Requirements	5841

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5841

A.17.1.1.2.1	Test Purpose and Environment	5841

A.17.1.1.2.2	Test Parameters	5841

A.17.1.1.2.3	Test Requirements	5845

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5845

A.17.1.1.3.1	Test Purpose and Environment	5845

A.17.1.1.3.2	Test Parameters	5845

A.17.1.1.3.3	Test Requirements	5848

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5848

A.17.1.1.4.1	Test Purpose and Environment	5848

A.17.1.1.4.2	Test Parameters	5848

A.17.1.1.4.3	Test Requirements	5852

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5852

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5852

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5852

A.17.2.1.1.1	Test Purpose and Environment	5852

A.17.2.1.1.2	Test Requirements	5856

A.17.3	RRC_CONNECTED state mobility for RedCap	5856

A.17.3.1	Handover for RedCap	5856

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5856

A.17.3.1.1.1	Test Purpose and Environment	5856

A.17.3.1.1.2	Test Parameters	5856

A.17.3.1.1.3	Test Requirements	5859

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5859

A.17.3.1.2.1	Test Purpose and Environment	5859

A.17.3.1.2.2	Test Parameters	5859

A.17.3.1.2.3 Test Requirements	5861

A.17.3.2	RRC Connection Mobility Control for RedCap	5861

A.17.3.2.1	SA: RRC Re-establishment	5861

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5861

A.17.3.2.1.1.1	Test Purpose and Environment	5861

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5864

A.17.3.2.1.2.1	Test Purpose and Environment	5864

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5867

A.17.3.2.1.3.1	Test Purpose and Environment	5867

A.17.3.2.1.3.2	Test Requirements	5869

A.17.3.2.2	Random Access	5870

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5870

A.17.3.2.2.1.1	Test Purpose and Environment	5870

A.17.3.2.2.1.2	Test Requirements	5872

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5874

A.17.3.2.2.2.1	Test Purpose and Environment	5874

A.17.3.2.2.2.2	Test Requirements	5876

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5878

A.17.3.2.2.3.1	Test Purpose and Environment	5878

A.17.3.2.2.3.2	Test Requirements	5880

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5881

A.17.3.2.2.4.1	Test Purpose and Environment	5881

A.17.3.2.2.4.2	Test Requirements	5883

A.17.3.2.3	SA: RRC Connection Release with Redirection	5884

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5884

A.17.3.2.3.1.1	Test Purpose and Environment	5884

A.17.3.2.3.1.2	Test Parameters	5884

A.17.3.2.3.1.3	Test Requirements	5888

A.17.4	Timing	5888

A.17.4.1	UE transmit timing	5888

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5888

A.17.4.1.1.1	Test Purpose and environment	5888

A.17.4.1.1.2	Test requirements	5891

A.17.4.2	UE timer accuracy	5892

A.17.4.3	Timing advance	5892

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5892

A.17.4.3.1.1	Test Purpose and Environment	5892

A.17.4.3.1.2	Test Parameters	5892

A.17.4.3.1.3 Test Requirements	5895

A.17.4.2	UE timer accuracy	5895

A.17.4.3	Timing advance	5895

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5895

A.17.5	Signaling characteristics for RedCap	5895

A.17.5.1	Radio link Monitoring for RedCap	5895

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5895

A.17.5.1.1.1	Test Purpose and Environment	5895

A.17.5.1.1.2	Test Requirements	5898

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5899

A.17.5.1.2.1	Test Purpose and Environment	5899

A.17.5.1.2.2	Test Requirements	5903

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5904

A.17.5.1.3.1	Test Purpose and Environment	5904

A.17.5.1.3.2	Test Requirements	5907

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5907

A.17.5.1.4.1	Test Purpose and Environment	5907

A.17.5.1.4.2	Test Requirements	5912

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5912

A.17.5.1.5.1	Test Purpose and Environment	5912

A.17.5.1.5.2	Test Requirements	5917

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5917

A.17.5.1.6.1	Test Purpose and Environment	5917

A.17.5.1.6.2	Test Requirements	5921

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5921

A.17.5.1.7.1	Test Purpose and Environment	5921

A.17.5.1.7.2	Test Requirements	5925

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5925

A.17.5.1.8.1	Test Purpose and Environment	5925

A.17.5.1.8.2	Test Requirements	5930

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5930

A.17.5.1.9.1	Test Purpose and Environment	5930

A.17.5.1.9.2	Test Requirements	5933

A.17.5.2	Beam Failure Detection and Link recovery procedures	5933

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5933

A.17.5.2.1.1	Test Purpose and Environment	5933

A.17.5.2.1.2	Test Requirements	5938

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5939

A.17.5.2.2.1	Test Purpose and Environment	5939

A.17.5.2.2.2	Test Requirements	5943

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5944

A.17.5.2.3.1	Test Purpose and Environment	5944

A.17.5.2.3.2	Test Requirements	5948

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5948

A.17.5.2.4.1	Test Purpose and Environment	5948

A.17.5.2.4.2	Test Requirements	5952

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5952

A.17.5.2.5.1	Test Purpose and Environment	5952

A.17.5.2.5.2	Test Requirements	5956

A.17.5.3	Active BWP switch for RedCap	5956

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5956

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5956

A.17.5.3.1.1.1	Test Purpose and Environment	5956

A.17.5.3.1.1.2	Test Requirements	5959

A.17.5.3.2	RRC-based Active BWP Switch	5959

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5959

A.17.5.3.2.1.1	Test Purpose and Environment	5959

A.17.5.3.2.1.2	Test Requirements	5962

A.17.5.4	Active TCI state switch delay	5963

A.17.5.4.1	MAC-CE based active TCI state switch	5963

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5963

A.17.5.4.1.1.1	Test Purpose and Environment	5963

A.17.5.4.1.1.2	Test Requirements	5966

A.17.5.4.2	RRC based active TCI state switch	5966

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5966

A.17.5.4.2.1.1	Test Purpose and Environment	5966

A.17.5.4.2.1.2	Test Requirements	5970

A.17.5.5	Uplink spatial relation switch delay	5970

A.17.5.5.1	MAC-CE based Spatial Relation switch	5970

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5970

A.17.5.5.1.1.1	Test Purpose and Environment	5970

A.17.5.5.1.1.2	Test Requirements	5973

A.17.5.5.2	RRC based spatial relation switch	5973

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5973

A.17.5.5.2.1.2	Test Requirements	5976

A.17.5.6	UE specific CBW change	5976

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5976

A.17.5.6.1.1	Test Purpose and Environment	5976

A.17.5.6.1.2	Test Requirements	5979

A.17.6	Measurement procedure for RedCap	5980

A.17.6.1	Intra-frequency Measurements	5980

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5980

A.17.6.1.1.1	Test purpose and Environment	5980

A.17.6.1.1.2	Test Requirements	5983

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5983

A.17.6.1.2.1	Test purpose and Environment	5983

A.7.6.1.2.2	Test Requirements	5983

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5984

A.17.6.1.3.1	Test purpose and Environment	5984

A.17.6.1.3.2	Test Requirements	5988

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5988

A.17.6.1.4.1	Test purpose and Environment	5988

A.17.6.1.4.2	Test Requirements	5991

A.17.6.2	Inter-frequency Measurements	5992

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5992

A.17.6.2.1.1	Test Purpose and Environment	5992

A.17.6.2.1.2	Test Requirements	5995

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5995

A.17.6.2.2.1	Test Purpose and Environment	5995

A.17.6.2.2.2	Test Requirements	5999

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	5999

A.17.6.2.3.1	Test Purpose and Environment	5999

A.17.6.2.3.2	Test Requirements	6003

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	6003

A.17.6.2.4.1	Test Purpose and Environment	6003

A.17.6.2.4.2	Test Requirements	6007

A.17.6.3	L1-RSRP measurement for beam reporting	6007

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	6007

A.17.6.3.1.1	Test Purpose and Environment	6007

A.17.6.3.1.2	Test parameters	6008

A.17.6.3.1.3	Test Requirements	6008

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	6008

A.17.6.3.2.1	Test Purpose and Environment	6008

A.17.6.3.2.2	Test parameters	6008

A.17.6.3.2.3	Test Requirements	6010

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	6010

A.17.6.3.3.1	Test Purpose and Environment	6010

A.17.6.3.3.2	Test parameters	6011

A.17.6.3.3.3	Test Requirements	6013

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	6014

A.17.6.3.4.1	Test Purpose and Environment	6014

A.17.6.3.4.2	Test parameters	6014

A.7.6.3.3.3	Test Requirements	6016

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	6017

A.17.6.4.1.1	Test Purpose and Environment	6017

A.17.6.4.1.2	Test Requirements	6020

A.17.7	Measurement Performance requirements	6020

A.17.7.1	SS-RSRP	6020

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6020

A.17.7.1.1.1	Test Purpose and Environment	6020

A.17.7.1.1.2	Test parameters	6020

A.17.7.1.1.3	Test Requirements	6024

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6025

A.17.7.1.2.1	Test Purpose and Environment	6025

A.17.7.1.2.2	Test parameters	6025

A.17.7.1.2.3	Test Requirements	6029

A.17.7.2	SS-RSRQ	6030

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	6030

A.17.7.2.1.1	Test Purpose and Environment	6030

A.17.7.2.1.2	Test Parameters	6030

A.17.7.2.1.3	Test Requirements	6032

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	6032

A.17.7.2.2.1	Test Purpose and Environment	6032

A.17.7.2.2.2	Test parameters	6032

A.17.7.2.2.3	Test Requirements	6034

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	6034

A.17.7.3	L1-RSRP measurement for beam reporting	6034

A.17.7.3.1	SSB based L1-RSRP measurement	6034

A.17.7.3.1.1	Test Purpose and Environment	6034

A.17.7.3.1.2	Test parameters	6035

A.17.7.3.1.3	Test Requirements	6035

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	6035

A.17.7.3.2.1	Test Purpose and Environment	6035

A.17.7.3.2.2	Test parameters	6035

A.17.7.3.2.3	Test Requirements	6036

A.17.7.4	SS-SINR	6036

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	6036

A.17.7.4.1.1	Test Purpose and Environment	6036

A.17.7.4.1.2	Test parameters	6036

A.17.7.4.1.3	Test Requirements	6038

A.18	E-UTRA standalone tests for NR RRM for RedCap	6038

A.18.1	RRC_IDLE state mobility	6038

A.18.1.1	Inter-RAT NR Cell re-selection	6038

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	6038

A.18.1.1.1.1	Test Purpose and Environment	6038

A.18.2.1.1.2	Test Requirements	6043

A.18.2	RRC_CONNECTED state mobility	6043

A.18.2.1	Handover	6043

A.18.2.1.1	E-UTRAN - NR handover in FR1	6043

A.18.2.1.1.1	Test Purpose and Environment	6043

A.18.2.1.1.2	Test Requirements	6048

A.18.2.2	RRC connection release with redirection	6048

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	6048

A.18.2.2.1.1	Test Purpose and Environment	6048

A.18.2.2.1.2	Test Parameters	6048

A.18.2.2.1.3	Test Requirements	6055

A.18.3	Measurement procedure	6055

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	6055

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	6055

A.18.3.1.1.1	Test Purpose and Environment	6055

A.18.3.1.1.2	Test Requirements	6061

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	6061

A.18.3.1.2.1	Test Purpose and Environment	6061

A.18.3.1.2.2	Test Requirements	6067

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	6067

A.18.3.1.3.1	Test Purpose and Environment	6067

A.18.3.1.3.2	Test Requirements	6073

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	6073

A.18.3.1.4.1	Test Purpose and Environment	6073

A.18.3.1.4.2	Test Requirements	6079

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	6079

A.18.3.1.5.1	Test Purpose and Environment	6079

A.18.3.1.5.2	Test Requirements	6081

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	6082

A.18.3.1.6.1	Test Purpose and Environment	6082

A.18.3.1.6.2	Test Requirements	6084

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	6085

A.18.3.1.7.1	Test Purpose and Environment	6085

A.18.3.1.7.2	Test Requirements	6087

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	6088

A.18.3.1.8.1	Test Purpose and Environment	6088

A.18.3.1.8.2	Test Requirements	6090

Annex B (normative): Conditions for RRM requirements applicability for operating bands	6094

B.1	Conditions for NR RRC_IDLE state mobility	6094

B.1.1	Introduction	6094

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	6094

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	6095

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	6095

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	6097

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	6097

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	6098

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	6098

B.2.1	Introduction	6098

B.2.1.1	General	6098

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	6098

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	6098

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	6098

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	6099

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	6100

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	6100

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	6100

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	6101

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	6101

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	6101

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	6101

B.2.2	Conditions for NR intra-frequency measurements	6102

B.2.3	Conditions for NR inter-frequency measurements	6103

B.2.4	Conditions for NR L1-RSRP reporting	6104

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6104

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6105

B.2.5	Conditions for RRC connection release with redirection to NR	6107

B.2.6	Void	6108

B.2.6.1	Void	6108

B.2.6.2	Void	6108

B.2.7	Conditions for SRS-RSRP measurements	6108

B.2.8	Conditions for NR L1-SINR reporting	6109

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	6109

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	6110

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	6110

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	6110

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	6112

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	6112

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	6113

B.2.9	Conditions for NR intra-frequency measurements under CCA	6114

B.2.10	Conditions for NR inter-frequency measurements under CCA	6114

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6114

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6114

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	6115

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	6116

B.2.14	Conditions for NR PRS-based measurements	6118

B.2.15	Conditions for NR intra-frequency measurements	6120

B.2.16	Conditions for NR inter-frequency measurements	6121

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6123

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6124

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6124

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6124

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6124

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6125

B.3	RRM Requirements Exceptions	6125

B.3.1	Introduction	6125

B.3.2	Receiver sensitivity relaxation for CA	6125

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6125

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6125

B.3.2.2.1	Inter-band carrier aggregation	6125

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6125

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6126

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6126

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6126

B.3.2.4.1	Intra-band contiguous carrier aggregation	6126

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6126

B.3.3	Receiver sensitivity relaxation for DC	6126

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6126

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6126

B.3.4	Receiver sensitivity relaxation for SUL	6126

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6126

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6127

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6127

B.4	Conditions for V2X	6127

B.4.1	Test parameters for GNSS signals	6127

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6127

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6128

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6128

B.5	High level test procedure for SAN RRM tests	6128

Annex C (informative): Change history	6130