| 3GPP TS 38.133 V18.5.0 (2024-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	124

1	Scope	126

2	References	126

3	Definitions, symbols and abbreviations	127

3.1	Definitions	127

3.2	Symbols	129

3.3	Abbreviations	130

3.4	Test tolerances	133

3.5	Frequency bands grouping	133

3.5.1	Introduction	133

3.5.2	NR operating bands in FR1	133

3.5.2A	NR operating bands for satellite access in FR1	134

3.5.3	NR operating bands in FR2	135

3.6	Applicability of requirements in this specification version	136

3.6.1	RRC connected state requirements in DRX	136

3.6.2	Number of serving carriers	136

3.6.2.1	Number of serving carriers for SA	136

3.6.2.2	Number of serving carriers for EN-DC	136

3.6.2.3	Number of serving carriers for NE-DC	137

3.6.2.4	Number of serving carriers for NR-DC	137

3.6.3	Applicability for intra-band FR2	137

3.6.4	Applicability for FR2 UE power classes	137

3.6.5	Applicability for SDL bands	137

3.6.6	Applicability of requirements for NGEN-DC operation	137

3.6.7	Applicability of QCL	137

3.6.9	Applicability of requirements for scheduling availability	138

3.6.10	Applicability of requirements for measurement restrictions	139

3.6.11	Applicability of requirements for Redcap UEs	139

3.6.11.1	RRC connected state requirements in DRX	139

3.6.11.2	Applicability for FR2 Redcap UE power classes	139

3.6.11.3	Applicability of QCL	139

3.6.12	Applicability of requirements for Satellite Access	139

3.6.13	Applicability of requirements for FR2	139

3.6.14	Applicability of requirements for FR2 Power Class 6	139

3.6.15	Applicability of requirements for per-FR gap	140

3.6.16	Applicability of requirements for ATG	140

3.6.17	Applicability of requirements for MUSIM gaps	140

3.6.18	Applicability of requirements for a UE operating on a cell with less than 5MHz BW]	140

4	SA: RRC_IDLE state mobility	140

4.1	Cell Selection	140

4.2	Cell Re-selection	140

4.2.1	Introduction	140

4.2.2	Requirements	141

4.2.2.1	UE measurement capability	141

4.2.2.2	Measurement and evaluation of serving cell	141

4.2.2.3	Measurements of intra-frequency NR cells	143

4.2.2.4	Measurements of inter-frequency NR cells	147

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	152

4.2.2.6	Maximum interruption in paging reception	155

4.2.2.7	General requirements	156

4.2.2.8	Minimum requirement at transitions	156

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	156

4.2.2.9.1	Introduction	156

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	157

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	159

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	161

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	162

4.2.2.10.1	Introduction	162

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	162

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	165

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	168

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	168

4.2.2.11.1	Introduction	168

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	169

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	170

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	172

4.2A	Cell Re-selection when subject to CCA	173

4.2A.1	Introduction	173

4.2A.2	Requirements	173

4.2A.2.1	UE measurement capability	173

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	173

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	174

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	176

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	177

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	178

4.2A.2.7	General requirements	178

4.2B	Cell Re-selection for RedCap	178

4.2B.1	Introduction	178

4.2B.2	Requirements	178

4.2B.2.1	UE measurement capability for RedCap	178

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	178

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	179

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	179

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	180

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	183

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	186

4.2B.2.6	Maximum interruption in paging reception for RedCap	188

4.2B.2.7	General requirements for RedCap	188

4.2B.2.8	Minimum requirement at transitions	188

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	189

4.2B.2.9.1	Introduction	189

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	190

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	193

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	193

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	194

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	194

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	194

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	194

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	194

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	195

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	198

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	200

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	200

4.2B.2.10.1	Introduction	200

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	201

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary  criterion	203

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	204

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	204

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	205

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	205

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	205

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	205

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	206

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	208

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	211

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	211

4.2B.2.11.1	Introduction	211

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	212

4.2B.2.11.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	213

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	213

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	214

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary  criteria	214

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	214

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	215

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	215

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	215

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	217

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	218

4.2C	Cell Re-selection for NR UE for Satellite Access	218

4.2C.1	Introduction	218

4.2C.2	Requirements	219

4.2C.2.1	UE measurement capability	219

4.2C.2.2	Measurement and evaluation of serving cell	219

4.2C.2.3	Measurements of intra-frequency NR cells	220

4.2C.2.4	Measurements of inter-frequency NR cells	222

4.2C.2.5	Maximum interruption in paging reception	225

4.2C.2.6	Minimum requirement at transitions	226

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	226

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	226

4.2C.2.9	General requirements	226

4.2D	Cell Re-selection for ATG	226

4.2D.1	Introduction	226

4.2D.2	Requirements	226

4.2D.2.1	UE measurement capability	226

4.2D.2.2	Measurement and evaluation of serving cell	227

4.2D.2.3	Measurements of intra-frequency NR cells	227

4.2D.2.4	Measurements of inter-frequency NR cells	228

4.2D.2.5	Maximum interruption in paging reception	230

4.2D.2.6	General requirements	230

4.3	Minimization of Drive Tests (MDT)	231

4.3.1	Introduction	231

4.3.2	Measurement Requirements	231

4.3.3	Requirements for Relative Time Stamp Accuracy	231

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	232

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	232

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	232

4.3C.1	Introduction	232

4.3C.2	Measurement Requirements	233

4.3C.3	Requirements for Relative Time Stamp Accuracy	233

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	233

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	233

4.4	Idle Mode CA/DC Measurements	234

4.4.1	Introduction	234

4.4.2	Measurement Requirements	234

4.4.2.1	Detected cell requirement during state transition and Idle mode	234

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	235

4.4.2.3	Measurements on serving cell	236

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	236

4.5	NR measurements for positioning	237

4.5.1	Introduction	237

4.5.2	RSTD measurements	237

4.5.2.1	Introduction	237

4.5.2.2	Requirements Applicability	237

4.5.2.3	Measurement Capability	238

4.5.2.4	Measurement Reporting Requirements	238

4.5.2.5	Measurements Period Requirements	238

4.5.2.6	Measurements Period Requirements with Bandwidth Aggregation	241

4.5.3	PRS-RSRP measurements	244

4.5.3.1	Introduction	244

4.5.3.2	Requirements applicability	244

4.5.3.3	Measurement Capability	244

4.5.3.4	Measurement Reporting Requirements	244

4.5.3.5	Measurement Period Requirements	245

4.5.4	PRS-RSRPP measurements	247

4.5.4.1	Introduction	247

4.5.4.2	Requirements Applicability	247

4.5.4.3	Measurement Capability	248

4.5.4.4	Measurement Reporting Requirements	248

4.5.4.5	Measurement Period Requirements	248

4.5.5	Measurement requirements for DL RSCPD reported with RSTD	248

4.5.5.1	Introduction	248

4.5.5.2	Requirements Applicability	248

4.5.5.3	Measurement Capability	248

4.5.5.4	Measurement Reporting Requirements	249

4.5.5.5	Measurements Period Requirements	249

4.6	NR measurements for positioning for RedCap	252

4.6.1	Introduction	252

4.6.2	RSTD measurements for RedCap	253

4.6.2.1	Introduction	253

4.6.2.2	Requirements Applicability	253

4.6.2.3	Measurement Capability	253

4.6.2.4	Measurement Reporting Requirements	253

4.6.2.5	Measurements Period Requirements without RX FH	253

4.6.2.6	Measurement Period Requirements with RX FH	254

4.6.3	PRS-RSRP measurements for RedCap	254

4.6.3.1	Introduction	254

4.6.3.2	Requirements applicability	254

4.6.3.3	Measurement Capability	254

4.6.3.4	Measurement Reporting Requirements	254

4.6.3.5	Measurement Period Requirements without RX FH	255

4.6.3.6	Measurement Period Requirements with RX FH	257

4.6.4	PRS-RSRPP measurements for RedCap	257

4.6.4.1	Introduction	257

4.6.4.2	Requirements Applicability	258

4.6.4.3	Measurement Capability	258

4.6.4.4	Measurement Reporting Requirements	258

4.6.4.5	Measurement Period Requirements without RX FH	258

4.6.4.6	Measurement Period Requirements with RX FH	258

4.7	Measurement report for fast CA/DC setup	259

4.7.1	Introduction	259

4.7.2	Void	259

4.7.3	Measurement Report Requirements	259

5	SA: RRC_INACTIVE state mobility	260

5.1	Cell Re-selection	260

5.1.1	Introduction	260

5.1.2	Requirements	260

5.1.2.1	UE measurement capability	260

5.1.2.2	Measurement and evaluation of serving cell	260

5.1.2.3	Measurements of intra-frequency NR cells	262

5.1.2.4	Measurements of inter-frequency NR cells	265

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	267

5.1.2.6	Maximum interruption in paging reception	268

5.1.2.7	General requirements	268

5.1A	Cell Re-selection with CCA	268

5.1A.1	Introduction	268

5.1A.2	Requirements	269

5.1A.2.1	UE measurement capability	269

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	269

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	269

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	269

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	269

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	269

5.1A.2.7	General requirements	269

5.1B	Cell Re-selection for RedCap	269

5.1B.1	Introduction	269

5.1B.2	Requirements	269

5.1B.2.1	UE measurement capability	269

5.1B.2.2	Measurement and evaluation of serving cell	269

5.1B.2.3	Measurements of intra-frequency NR cells	272

5.1B.2.4	Measurements of inter-frequency NR cells	274

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	276

5.1B.2.6	Maximum interruption in paging reception	277

5.1B.2.7	General requirements	277

5.1B.2.8	Minimum requirement at transitions	277

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	277

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	278

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	279

5.1C	Cell Re-selection	280

5.1C.1	Introduction	280

5.1C.2	Requirements	280

5.1C.2.1	UE measurement capability	280

5.1C.2.2	Measurement and evaluation of serving cell	280

5.1C.2.3	Measurements of intra-frequency NR cells	280

5.1C.2.4	Measurements of inter-frequency NR cells	280

5.1C.2.5	Maximum interruption in paging reception	280

5.1C.2.6	General requirements	280

5.1D	Cell Re-selection for ATG	280

5.1D.1	Introduction	280

5.1D.2	Requirements	281

5.1D.2.1	UE measurement capability	281

5.1D.2.2	Measurement and evaluation of serving cell	281

5.1D.2.3	Measurements of intra-frequency NR cells	281

5.1D.2.4	Measurements of inter-frequency NR cells	281

5.1D.2.5	Maximum interruption in paging reception	281

5.1D.2.6	General requirements	281

5.2	Void	281

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	281

5.2B.1	Introduction	281

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	281

5.2B.2.1	Void5.2B.3 TA validation requirements for RedCap	282

5.2B.2.2	Void	283

5.2B.4	Scheduling restriction	283

5.2B.5	Applicability conditions for CG-SDT for RedCap	283

5.3	Minimization of Drive Tests (MDT)	283

5.3.1	Introduction	283

5.3.2	Measurement Requirements	283

5.3.3	Requirements for Relative Time Stamp Accuracy	283

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	284

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	284

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	284

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	284

5.3C.1	Introduction	284

5.3C.2	Measurement Requirements	284

5.3C.3	Requirements for Relative Time Stamp Accuracy	284

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	285

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	285

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	285

5.4	Inactive Mode CA/DC Measurements	285

5.4.1	Introduction	285

5.4.2	Measurement Requirements	285

5.4.2.1	Detected cell requirement during state transition and inactive mode	285

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	285

5.4.2.3	Measurements on serving cell	285

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	285

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	286

5.5.1	Introduction	286

5.5.2	Requirements on UE synchronization for small data transmissions	286

5.5.3	TA validation requirements	286

5.5.4	Scheduling restriction	287

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	287

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	287

5.5.4.3	Scheduling availability of UE performing measurements on FR2	287

5.5.5	Applicability conditions for SDT	288

5.5D	Configured Grant based Small Data Transmissions (CG-SDT) for ATG	288

5.6	NR measurements for positioning	288

5.6.1	Introduction	288

5.6.1A	Cell re-selection for positioning	289

5.6.1A.1	Measurement and evaluation of serving cell	290

5.6.1A.2	Measurements of intra-frequency NR cells	290

5.6.2	RSTD measurements	291

5.6.2.1	Introduction	291

5.6.2.2	Requirements Applicability	291

5.6.2.3	Measurement Capability	291

5.6.2.5	Measurements Period Requirements	292

5.6.2.6	Measurements Period Requirements with Bandwidth Aggregation	295

5.6.3	PRS-RSRP measurements	298

5.6.3.1	Introduction	298

5.6.3.2	Requirements applicability	298

5.6.3.3	Measurement Capability	298

5.6.3.4	Measurement Reporting Requirements	298

5.6.3.5	Measurement Period Requirements	299

5.6.4	UE Rx-Tx time difference measurements	302

5.6.4.1	Introduction	302

5.6.4.2	Requirements Applicability	302

5.6.4.3	Measurement Capability	302

5.6.4.4	Measurement Reporting Requirements	302

5.6.4.5	Measurement Period Requirements	302

5.6.4.6	Measurement Period Requirements with Bandwidth Aggregation	306

5.6.5	PRS-RSRPP measurements	309

5.6.5.1	Introduction	309

5.6.5.2	Requirements Applicability	309

5.6.5.3	Measurement Capability	309

5.6.5.4	Measurement Reporting Requirements	309

5.6.5.5	Measurement Period Requirements	310

5.6.6	TA validation requirements for positioning	310

5.6.6.1	Introduction	310

5.6.6.2	TA validation requirements	310

5.6.6.3	TA validation requirements when configured with validity area	311

5.6.7	Measurement requirements for DL RSCPD reported with RSTD	312

5.6.7.1	Introduction	312

5.6.7.2	Requirements Applicability	312

5.6.7.3	Measurement Capability	312

5.6.7.4	Measurement Reporting Requirements	312

5.6.7.5	Measurements Period Requirements	312

5.6.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	314

5.6.8.1	Introduction	314

5.6.8.2	Requirements Applicability	314

5.6.8.3	Measurement Capability	314

5.6.8.4	Measurement Reporting Requirements	314

5.6.8.5	Measurement Period Requirements	315

5.6A	NR measurements for positioning for RedCap	316

5.6A.1	Introduction	316

5.6A.2	Cell re-selection for positioning	317

5.6A.2.1	Measurement and evaluation of serving cell	317

5.6A.2.2	Measurements of intra-frequency NR cells	318

5.6A.3	TA validation requirements for positioning SRS	319

5.6A.3.1	Introduction	319

5.6A.3.2	TA validation requirements	319

5.6A.3.3	TA validation requirements when configured with validity area	319

5.6A.4	RSTD measurements for RedCap	319

5.6A.4.1	 Introduction	319

5.6A.4.2	Requirements applicability	319

5.6A.4.3	Measurement Capability	320

5.6A.4.4	Measurement Reporting Requirements	320

5.6A.4.5	Measurement Period Requirement without RX FH	320

5.6A.4.6	Measurement Period Requirement with RX FH	323

5.6A.5	PRS-RSRP measurements for RedCap	324

5.6A.5.1	 Introduction	324

5.6A.5.2	Requirements applicability	324

5.6A.5.3	Measurement Capability	324

5.6A.5.4	Measurement Reporting Requirements	324

5.6A.5.5	Measurement Period Requirements without RX FH	325

5.6A.5.6	Measurement Period Requirement with RX FH	328

5.6A.6	UE Rx-Tx time difference measurements for RedCap	328

5.6A.6.1	Introduction	328

5.6A.6.2	Requirements Applicability	328

5.6A.6.3	Measurement Capability	329

5.6A.6.4	Measurement Reporting Requirements	329

5.6A.6.5	Measurement Period Requirements without RX FH	329

5.6A.6.6	Measurement Period Requirements with RX FH	330

5.6A.7	PRS-RSRPP measurements for RedCap	330

5.6A.7.1	 Introduction	330

5.6A.7.2	Requirements applicability	330

5.6A.7.3	Measurement Capability	331

5.6A.7.4	Measurement Reporting Requirements	331

5.6A.7.5	Measurement Period Requirements without FH	331

5.6A.7.6	Measurement period requirement with FH	331

5.7	Random access based Small Data Transmissions (RA-SDT)	331

5.7.1	Introduction	331

5.7.2	Requirements for small data transmissions based on 2-step RA	332

5.7.3	Requirements for small data transmissions based on 4-step RA	332

5.7.4	Applicability conditions for SDT	332

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	332

5.7B.1	Introduction	332

5.7B.2	Requirements for small data transmissions based on 2-step RA	332

5.7B.3	Requirements for small data transmissions based on 4-step RA	332

5.7B.4	Applicability conditions for RA-SDT for RedCap	332

5.7D	Random access based Small Data Transmissions (RA-SDT) for ATG	333

5.8	Measurement report for fast CA/DC setup	333

5.8.1	Introduction	333

5.8.2	Void	333

5.8.3	Measurement Report Requirements	333

6	RRC_CONNECTED state mobility	333

6.1	Handover	333

6.1.1	NR Handover	333

6.1.1.1	Introduction	333

6.1.1.2	NR FR1 - NR FR1 Handover	334

6.1.1.2.1	Handover delay	334

6.1.1.2.2	Interruption time	334

6.1.1.3	NR FR2- NR FR1 Handover	335

6.1.1.3.1	Handover delay	335

6.1.1.3.2	Interruption time	335

6.1.1.4	NR FR2- NR FR2 Handover	336

6.1.1.4.1	Handover delay	336

6.1.1.4.2	Interruption time	336

6.1.1.5	NR FR1- NR FR2 Handover	337

6.1.1.5.1	Handover delay	337

6.1.1.5.2	Interruption time	338

6.1.2	NR Handover to other RATs	339

6.1.2.1	NR – E-UTRAN Handover	339

6.1.2.1.1	Introduction	339

6.1.2.1.2	Handover delay	339

6.1.2.1.3	Interruption time	339

6.1.2.2	NR – UTRAN Handover	340

6.1.2.2.1	Introduction	340

6.1.2.2.2	Handover delay	340

6.1.2.2.3	Interruption time	340

6.1.3	NR DAPS Handover	341

6.1.3.1	Introduction	341

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	341

6.1.3.2.1	DAPS handover delay	341

6.1.3.2.2	Interruption time	342

6.1.3.3	NR FR2- NR FR1 DAPS Handover	344

6.1.3.3.1	DAPS handover delay	344

6.1.3.3.2	Interruption time	344

6.1.3.4	NR FR1- NR FR2 DAPS Handover	345

6.1.3.4.1	DAPS handover delay	345

6.1.3.4.2	Interruption time	345

6.1.4	NR Conditional Handover	346

6.1.4.1	Introduction	346

6.1.4.2	NR FR1 – NR FR1 conditional handover	346

6.1.4.2.2	Measurement time	347

6.1.4.3	NR FR2 – NR FR1 conditional handover	348

6.1.4.4	NR FR2 – NR FR2 conditional handover	348

6.1.4.4.1	Handover delay	348

6.1.4.4.2	Measurement time	349

6.1.4.4.3	Preparation time	349

6.1.4.4.4	Interruption time	350

6.1.4.5	NR FR1 – NR FR2 conditional handover	350

6.1.5	NR Handover with PSCell	350

6.1.5.1	Introduction	350

6.1.5.2	Handover with PSCell from NR SA to EN-DC	350

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	351

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	351

6.1.5.3	HO with PSCell from NE-DC to NE-DC	352

6.1.5.3.1	Handover delay	352

6.1.5.3.2	HO with PSCell - PCell Interruption time	352

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	352

6.1.5.4	HO with PSCell from NR-DC to NR-DC	352

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	354

6.1.5.5.1	Introduction	354

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	354

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	355

6.1.6.1	Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC	356

6.1.6.1.1	CHO with PSCell – PCell Interruption time	356

6.1.6.1.2	CHO with PSCell – PSCell change delay	357

6.1.6.2	Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC	357

6.1.6.2.2	CHO with PSCell – PSCell change delay	358

6.1.7.1.2	PSCell conditional change delay	361

6.1.7.2	Conditional handover including target MCG in FR1 and Candidate SCG for CPC in FR2 in NR-DC	362

6.1.7.2.1	PCell handover delay	362

6.1.7.2.1.1	Measurement time	363

6.1.7.2.2	PSCell conditional change delay	363

6.1A	Void	364

6.1A.1	Void	364

6.1A.1.1	Void	364

6.1A.1.2	Void	364

6.1A.1.2.1	Void	364

6.1A.1.2.2	Void	364

6.1B	Handover to target cell using CCA	364

6.1B.1	NR Handover	364

6.1B.1.1	Introduction	364

6.1B.1.2	NR FR1 - NR FR1 Handover	365

6.1B.1.2.1	Handover delay	365

6.1B.1.2.2	Interruption time	365

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	366

6.1B.1.3.1	Handover delay	366

6.1B.1.3.2	Interruption time	366

6.1B.1.4	NR FR1- NR FR2-2 Handover	367

6.1B.1.4.1	Handover delay	367

6.1B.1.4.2	Interruption time	367

6.1C	Handover for SAN	368

6.1C.1	NR SAN Handover	368

6.1C.1.1	Introduction	368

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	368

6.1C.1.2.1	Handover delay	368

6.1C.1.2.2	Interruption time	369

6.1C.2	NR SAN Conditional Handover	369

6.1C.2.1	Introduction	369

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	369

6.1C.2.2.1	Handover delay	370

6.1C.2.2.2	Measurement time	370

6.1C.2.2.3	Preparation time	371

6.1C.2.2.4	Interruption time	371

6.1D	Handover for RedCap	372

6.1D.1	NR Handover	372

6.1D.1.1	Introduction	372

6.1D.1.2	NR FR1 - NR FR1 Handover	372

6.1D.1.2.1	Handover delay	372

6.1D.1.2.2	Interruption time	372

6.1D.1.3	NR FR2- NR FR2 Handover	374

6.1D.1.3.1	Handover delay	374

6.1D.1.3.2	Interruption time	374

6.1D.2	NR Handover to other RATs	375

6.1D.2.1	NR – E-UTRAN Handover	375

6.1E	Handover for ATG	376

6.1E.1	NR Handover	376

6.1E.1.1	Introduction	376

6.1E.1.2	NR FR1 - NR FR1 Handover	376

6.1E.1.2.1	Handover delay	376

6.1E.1.2.2	Interruption time	376

6.1E.2	NR Conditional Handover	377

6.1E.2.1	Introduction	377

6.1E.2.2	NR FR1 – NR FR1 conditional handover	377

6.1E.2.2.1	Handover delay	377

6.1E.2.2.2	Measurement time	377

6.1E.2.2.3	Preparation time	378

6.1E.2.2.4	Interruption time	378

6.2	RRC Connection Mobility Control	378

6.2.1	SA: RRC Re-establishment	378

6.2.1.1	Introduction	378

6.2.1.2	Requirements	379

6.2.1.2.1	UE Re-establishment delay requirement	379

6.2.1A	RRC Re-establishment with CCA	380

6.2.1A.1	Introduction	380

6.2.1A.2	Requirements	381

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	381

6.2.1B	SA: RRC Re-establishment for RedCap	383

6.2.1B.1	Introduction	383

6.2.1B.2	Requirements	383

6.2.2	Random access	383

6.2.2.1	Introduction	383

6.2.2.2	Requirements for 4-step RA type	384

6.2.2.2.1	Contention based random access	384

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	384

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	384

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	384

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	384

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	384

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	385

6.2.2.2.2	Non-Contention based random access	385

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	385

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	385

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	386

6.2.2.2.3	UE behaviour when configured with supplementary UL	386

6.2.2.3	Requirements for 2-step RA type	386

6.2.2.3.1	Contention based random access	386

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	386

6.2.2.3.1.2	Correct behaviour when receiving MsgB	387

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	387

6.2.2.3.2	Non-Contention based random access	387

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	387

6.2.2.3.2.2	Correct behaviour when receiving MsgB	387

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	387

6.2.2.3.3	UE behaviour when configured with supplementary UL	388

6.2.2A	Random access when CCA is used on target frequency	388

6.2.2A.1	Introduction	388

6.2.2A.2	Requirements for 4-step RA type	388

6.2.2A.2.1	Contention based random access	388

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	388

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	389

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	389

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	389

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	389

6.2.2A.2.2	Non-Contention based random access	389

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	389

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	390

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	390

6.2.2A.3	Requirements for 2-step RA type	390

6.2.2A.3.1	Contention based random access	391

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	391

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	391

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	392

6.2.2A.3.2	Non-Contention based random access	392

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	392

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	392

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	392

6.2.2B	Random access for RedCap	393

6.2.2B.1	Introduction	393

6.2.2B.2	Requirements	393

6.2.3	SA: RRC Connection Release with Redirection	393

6.2.3.1	Introduction	393

6.2.3.2	Requirements	394

6.2.3.2.1	RRC connection release with redirection to NR	394

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	394

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	395

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	396

6.2.3A.1	Introduction	396

6.2.3A.2	Requirements	396

6.2.3A.2.1	RRC connection release with redirection to NR	396

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	397

6.2C	RRC Connection Mobility Control for Satellite Access	397

6.2C.1	SA: RRC Re-establishment for Satellite Access	397

6.2C.1.1	Introduction	397

6.2C.1.2	Requirements	397

6.2C.1.2.1	UE Re-establishment delay requirement	397

6.2C.2	Random access for satellite access	398

6.2C.2.1	Introduction	398

6.2C.2.2	Requirements for 4-step RA type	399

6.2C.2.2.1	Contention based random access	399

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	399

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	399

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	399

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	399

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	399

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	400

6.2C.2.2.2	Non-Contention based random access	400

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	400

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	400

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	401

6.2C.2.3	Requirements for 2-step RA type	401

6.2C.2.3.1	Contention based random access	401

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	401

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	401

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	402

6.2C.2.3.2	Non-Contention based random access	402

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	402

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	402

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	402

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	402

6.2C.3.1	Introduction	402

6.2C.3.2	Requirements	403

6.2C.3.2.1	RRC connection release with redirection to NR	403

6.2D	RRC Connection Mobility Control for ATG	403

6.2D.1	SA: RRC Re-establishment	403

6.2D.1.1	Introduction	403

6.2D.1.2	Requirements	404

6.2D.1.2.1	UE Re-establishment delay requirement	404

6.2D.2	Random access	405

6.2D.2.1	Introduction	405

6.2D.2.2	Requirements for 4-step RA type	405

6.2D.2.3	Requirements for 2-step RA type	405

6.2D.3	SA: RRC Connection Release with Redirection	406

6.2D.3.1	Introduction	406

6.2D.3.2	Requirements	406

6.2D.3.2.1	RRC connection release with redirection to NR	406

6.3	L1/L2-Triggered Mobility	407

6.3.1	PCell Cell Switch	407

6.3.1.1	Introduction	407

6.3.1.2	LTM Cell Switch delay	407

6.3.1.3	Interruption time	408

7	Timing	409

7.1	UE transmit timing	409

7.1.1	Introduction	409

7.1.2	Requirements	410

7.1.2.1	Gradual timing adjustment	412

7.1.2.2	Void	413

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	413

7.1.2.4	UE transmit timing for positioning measurements	413

7.1A	UE transmit timing for RedCap	414

7.1A.1	Introduction	414

7.1A.2	Requirements	414

7.1A.2.1	Gradual timing adjustment	415

7.1C	UE transmit timing for Satellite Access	416

7.1C.1	Introduction	416

7.1C.2	Requirements	416

7.1C.2.1	Gradual timing adjustment	417

7.1D	UE transmit timing for ATG	417

7.1D.1	Introduction	417

7.1D.2	Requirements	417

7.1D.2.1	Gradual timing adjustment	418

7.2	UE timer accuracy	419

7.2.1	Introduction	419

7.2.2	Requirements	419

7.2A	UE timer accuracy for RedCap	419

7.2A.1	Introduction	419

7.2A.2	Requirements	419

7.2C	UE timer accuracy for satellite access	420

7.2C.1	Introduction	420

7.2C.2	Requirements	420

7.2D	UE timer accuracy for ATG	420

7.2D.1	Introduction	420

7.2D.2	Requirements	420

7.3	Timing advance	421

7.3.1	Introduction	421

7.3.2	Requirements	421

7.3.2.1	Timing Advance adjustment delay	421

7.3.2.2	Timing Advance adjustment accuracy	421

7.3A	Timing Advance for RedCap	421

7.3A.1	Introduction	421

7.3A.2	Requirements	421

7.3A.2.1	Timing Advance adjustment delay	421

7.3A.2.2	Timing Advance adjustment accuracy	421

7.3C	Timing advance for satellite access	422

7.3C.1	Introduction	422

7.3C.2	Requirements	422

7.3C.2.1	Timing Advance adjustment delay	422

7.3C.2.2	Timing Advance adjustment accuracy	422

7.3D	Timing advance for ATG	422

7.3D.1	Introduction	422

7.3D.2	Requirements	423

7.3D.2.1	Timing Advance adjustment delay	423

7.3D.2.2	Timing Advance adjustment accuracy	423

7.4	Cell phase synchronization accuracy	423

7.4.1	Definition	423

7.4.2	Minimum requirements	423

7.5	Maximum Transmission Timing Difference	423

7.5.1	Introduction	423

7.5.2	Minimum Requirements for inter-band EN-DC	424

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	424

7.5.3	Minimum Requirements for intra-band EN-DC	425

7.5.4	Minimum Requirements for NR Carrier Aggregation	425

7.5.5	Minimum Requirements for inter-band NE-DC	426

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	426

7.5.6	Minimum Requirements for inter-band NR DC	427

7.5.7	Minimum Requirements for multi-TRP	427

7.6	Maximum Receive Timing Difference	428

7.6.1	Introduction	428

7.6.2	Minimum Requirements for inter-band EN-DC	429

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	429

7.6.3	Minimum Requirements for intra-band EN-DC	430

7.6.4	Minimum Requirements for NR Carrier Aggregation	430

7.6.5	Minimum Requirements for inter-band NE-DC	431

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	431

7.6.6	Minimum Requirements for inter-band NR DC	432

7.6.7	Minimum Requirements for PC6 UE in FR2	433

7.6.8	Minimum Requirements for Multi-TRPs	433

7.7	deriveSSB-IndexFromCell tolerance	434

7.7.1	Minimum requirements	434

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	434

7.7A.1	Minimum requirements	434

7.7D	DeriveSSB-IndexFromCell tolerance for ATG	434

7.7D.1	Minimum requirements	434

7.8	Void	435

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	435

7.9.1	Minimum requirements	435

7.9D	DeriveSSB-IndexFromCellInter-r17 tolerance for ATG	435

7.9D.1	Minimum requirements	435

8	Signalling characteristics	429

8.1	Radio Link Monitoring	429

8.1.1	Introduction	429

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	430

8.1.2	Requirements for SSB based radio link monitoring	431

8.1.2.1	Introduction	431

8.1.2.2	Minimum requirement	432

8.1.2.3	Measurement restrictions for SSB based RLM	436

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	437

8.1.3	Requirements for CSI-RS based radio link monitoring	438

8.1.3.1	Introduction	438

8.1.3.2	Minimum requirement	439

8.1.3.3	Measurement restrictions for CSI-RS based RLM	442

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	444

8.1.4	Minimum requirement at transitions	444

8.1.5	Minimum requirement for UE turning off the transmitter	445

8.1.6	Minimum requirement for L1 indication	445

8.1.7	Scheduling availability of UE during radio link monitoring	445

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	446

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	446

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	446

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	447

8.1.8	Minimum requirement under IDC Interference	447

8.1A	Radio Link Monitoring with CCA on Target Frequency	447

8.1A.1	Introduction	447

8.1A.2	Requirements for SSB Based Radio Link Monitoring	448

8.1A.2.1	Introduction	448

8.1A.2.2	Minimum Requirement	449

8.1A.3	Minimum requirement at transitions	452

8.1A.4	Minimum requirement for UE turning off the transmitter	453

8.1A.5	Minimum requirement for L1 indication	453

8.1A.6	Scheduling availability of UE during radio link monitoring	453

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	453

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	454

8.1B	Radio Link Monitoring for RedCap	454

8.1B.1	Introduction	454

8.1B.2	Requirements for SSB based radio link monitoring	455

8.1B.2.1	Introduction	455

8.1B.2.2	Minimum requirement	456

8.1B.2.3	Measurement restrictions for SSB based RLM	458

8.1B.3	Requirements for CSI-RS based radio link monitoring	459

8.1B.3.1	Introduction	459

8.1B.3.2	Minimum requirement	459

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	462

8.1B.4	Minimum requirement at transitions	463

8.1B.5	Minimum requirement for UE turning off the transmitter	463

8.1B.6	Minimum requirement for L1 indication	463

8.1B.7	Scheduling availability of UE during radio link monitoring	463

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	464

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	464

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	464

8.1C	Radio Link Monitoring for Satellite Access	464

8.1C.1	Introduction	464

8.1C.2	Requirements for SSB based radio link monitoring	465

8.1C.2.1	Introduction	465

8.1C.2.2	Minimum requirement	466

8.1C.2.3	Measurement restrictions for SSB based RLM	467

8.1C.3	Requirements for CSI-RS based radio link monitoring	467

8.1C.3.1	Introduction	467

8.1C.3.2	Minimum requirement	468

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	469

8.1C.4	Minimum requirement at transitions	470

8.1C.5	Minimum requirement for UE turning off the transmitter	470

8.1C.6	Minimum requirement for L1 indication	470

8.1C.7	Scheduling availability of UE during radio link monitoring	470

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	471

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	471

8.1D	Radio Link Monitoring for ATG	471

8.1D.1	Introduction	471

8.1D.2	Requirements for SSB based radio link monitoring	472

8.1D.2.1	Introduction	472

8.1D.2.2	Minimum requirement	473

8.1D.2.3	Measurement restrictions for SSB based RLM	474

8.1D.3	Requirements for CSI-RS based radio link monitoring	474

8.1D.3.1	Introduction	474

8.1D.3.2	Minimum requirement	474

8.1D.3.3	Measurement restrictions for CSI-RS based RLM	475

8.1D.4	Minimum requirement at transitions	476

8.1D.5	Minimum requirement for UE turning off the transmitter	476

8.1D.6	Minimum requirement for L1 indication	476

8.1D.7	Scheduling availability of UE during radio link monitoring	476

8.1D.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	476

8.1D.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	476

8.2	Interruption	476

8.2.1	EN-DC Interruption	476

8.2.1.1	Introduction	476

8.2.1.2	Requirements	477

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	477

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	478

8.2.1.2.3	Interruptions at SCell addition/release	478

8.2.1.2.4	Interruptions at SCell activation/deactivation	479

8.2.1.2.5	Interruptions during measurements on SCC	481

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	481

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	481

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	482

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	482

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	483

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	483

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	484

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	484

8.2.1.2.8.2	Interruptions during direct SCell activation	485

8.2.1.2.9	Interruptions at SCell hibernation	485

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	485

8.2.1.2.11	Interruptions due to UE-specific CBW change	485

8.2.1.2.12	Interruptions at NR SRS carrier based switching	486

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	487

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	488

8.2.1.2.15	Interruptions due to SCell dormancy	488

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	488

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	489

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	489

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	489

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	489

8.2.1.2.18	Interruptions at NR SRS antenna port switching	490

8.2.1.2.19	Interruptions at fast SCell activation	491

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	492

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	492

8.2.2.1	Introduction	492

8.2.2.2	Requirements	493

8.2.2.2.1	Interruptions at SCell addition/release	493

8.2.2.2.2	Interruptions at SCell activation/deactivation	494

8.2.2.2.3	Interruptions during measurements on deactivated SCC	496

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	496

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	497

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	498

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	499

8.2.2.2.8	Interruptions due to UE-specific CBW change	499

8.2.2.2.9	Interruptions at NR SRS carrier based switching	499

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	501

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	502

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	502

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	503

8.2.2.2.10D	DL Interruptions at UE switching across three or four uplink bands	503

8.2.2.2.11	Interruptions at direct SCell activation	504

8.2.2.2.12	Interruptions due to SCell dormancy	504

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	504

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	504

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	505

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	505

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	505

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	505

8.2.2.2.16	Interruptions at NR SRS antenna port switching	506

8.2.2.2.17	Interruptions at fast SCell activation	507

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	508

8.2.2.2.19	Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]	508

8.2.2.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	509

8.2.3	NE-DC Interruptions	509

8.2.3.1	Introduction	509

8.2.3.2	Requirements	510

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	510

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	510

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	510

8.2.3.2.4	Interruptions at SCell activation/deactivation	512

8.2.3.2.5	Interruptions during measurements on SCC	513

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	513

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	513

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	514

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	514

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	514

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	515

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	515

8.2.3.2.9	Interruptions at SCell hibernation	515

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	516

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	516

8.2.3.2.12	Interruptions at E-UTRA SRS carrier based switching	517

8.2.3.2.13	Interruptions due to SCell dormancy	518

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	519

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	519

8.2.3.2.17	Interruptions at fast SCell activation	521

8.2.3.2.18	Interruptions due to UE-specific CBW change	521

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	522

8.2.4	NR-DC: Interruptions	522

8.2.4.1	Introduction	522

8.2.4.2	Requirements	523

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	523

8.2.4.2.2	Interruptions at SCell activation/deactivation	524

8.2.4.2.3	Interruptions during measurements on SCC	525

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	525

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	526

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	526

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	526

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	527

8.2.4.2.9	Interruptions at NR SRS carrier based switching	527

8.2.4.2.10	Interruptions at direct SCell activation	528

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	529

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	529

8.2.4.2.13	 Interruptions due to SCell dormancy	530

8.2.4.2.14	Interruptions at NR SRS antenna port switching	530

8.2.4.2.15	Interruptions at fast SCell activation	531

8.2.4.2.16	Interruptions at SCG activation/deactivation	532

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	532

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	533

8.2.4.2.19	Interruptions due to UE-specific CBW change	533

8.2.4.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	533

8.2.4.2A	Void	534

8.2.4.2A.1	Void	534

8.2.4.2A.2	Void	534

8.2.4.2A.3	Void	534

8.3	SCell Activation and Deactivation Delay	534

8.3.1	Introduction	534

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	534

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	541

8.3.4	Direct SCell Activation at SCell addition	541

8.3.5	Direct SCell Activation at Handover	543

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	544

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	548

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	549

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	550

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	551

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	555

8.3.13.1	Introduction	555

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	557

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	557

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	558

8.3.17	SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.	560

8.3.18	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting	564

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	567

8.3A.1	Introduction	567

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	567

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	571

8.4	UE UL carrier RRC reconfiguration delay	572

8.4.1	Introduction	572

8.4.2	UE UL carrier configuration delay requirement	572

8.4.3	UE UL carrier deconfiguration delay requirement	572

8.5	Link Recovery Procedures	572

8.5.1	Introduction	572

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	573

8.5.2	Requirements for SSB based beam failure detection	574

8.5.2.1	Introduction	574

8.5.2.2	Minimum requirement	575

8.5.2.3	Measurement restriction for SSB based beam failure detection	579

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	580

8.5.3	Requirements for CSI-RS based beam failure detection	581

8.5.3.1	Introduction	581

8.5.3.2	Minimum requirement	581

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	585

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	587

8.5.4	Minimum requirement for L1 indication	587

8.5.5	Requirements for SSB based candidate beam detection	588

8.5.5.1	Introduction	588

8.5.5.2	Minimum requirement	588

8.5.5.3	Measurement restriction for SSB based candidate beam detection	592

8.5.6	Requirements for CSI-RS based candidate beam detection	592

8.5.6.1	Introduction	592

8.5.6.2	Minimum requirement	593

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	596

8.5.7	Scheduling availability of UE during beam failure detection	597

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	597

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	597

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	598

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	599

8.5.8	Scheduling availability of UE during candidate beam detection	599

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	599

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	599

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	600

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	600

8.5.9	Requirements for Beam Failure Recovery in SCell	600

8.5.9.1	Introduction	600

8.5.9.2	Requirement	601

8.5.10	Minimum requirement at transitions for beam failure detection	601

8.5.11	Minimum requirement under IDC Interference	601

8.5A	Link Recovery Procedures when CCA is used on target frequency	602

8.5A.1	Introduction	602

8.5A.2	Requirements for SSB based beam failure detection	602

8.5A.2.1	Introduction	602

8.5A.2.2	Minimum requirement	603

8.5A.2.3	Measurement restriction for SSB based beam failure detection	605

8.5A.3	Void	605

8.5A.4	Minimum requirement for L1 indication	605

8.5A.5	Requirements for SSB based candidate beam detection	606

8.5A.5.1	Introduction	606

8.5A.5.2	Minimum requirement	606

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	609

8.5A.6	Void	609

8.5A.7	Scheduling availability of UE during beam failure detection	609

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	609

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	609

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	609

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	609

8.5A.8	Scheduling availability of UE during candidate beam detection	609

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	610

8.5.8A.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	610

8.5B	Link Recovery Procedures for Redcap	610

8.5B.1	Introduction	610

8.5B.2	Requirements for SSB based beam failure detection for Redcap	610

8.5B.2.1	Introduction	610

8.5B.2.2	Minimum requirement	611

8.5B.2.3	Measurement restriction for SSB based beam failure detection	612

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	613

8.5B.3.1	Introduction	613

8.5B.3.2	Minimum requirement	613

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	615

8.5B.4	Minimum requirement for L1 indication for Redcap	616

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	617

8.5B.5.1	Introduction	617

8.5B.5.2	Minimum requirement	617

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	619

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	619

8.5B.6.1	Introduction	619

8.5B.6.2	Minimum requirement	619

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	621

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	622

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	622

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	622

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	622

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	623

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	623

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	623

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	623

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	623

8.5C	Link Recovery Procedures for Satellite Access	624

8.5C.1	Introduction	624

8.5C.2	Requirements for SSB based beam failure detection	624

8.5C.2.1	Introduction	624

8.5C.2.2	Minimum requirement	625

8.5C.2.3	Measurement restriction for SSB based beam failure detection	625

8.5C.3	Requirements for CSI-RS based beam failure detection	626

8.5C.3.1	Introduction	626

8.5C.3.2	Minimum requirement	626

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	627

8.5C.4	Minimum requirement for L1 indication	628

8.5C.5	Requirements for SSB based candidate beam detection	628

8.5C.5.1	Introduction	628

8.5C.5.2	Minimum requirement	628

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	629

8.5C.6	Requirements for CSI-RS based candidate beam detection	630

8.5C.6.1	Introduction	630

8.5C.6.2	Minimum requirement	630

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	631

8.5C.7	Scheduling availability of UE during beam failure detection	631

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	631

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	631

8.5C.8	Scheduling availability of UE during candidate beam detection	632

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	632

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	632

8.5C.9	Minimum requirement at transitions for beam failure detection	632

8.5D	Link Recovery Procedures for ATG	632

8.5D.1	Introduction	632

8.5D.2	Requirements for SSB based beam failure detection	633

8.5D.2.1	Introduction	633

8.5D.2.2	Minimum requirement	633

8.5D.2.3	Measurement restriction for SSB based beam failure detection	634

8.5D.3	Requirements for CSI-RS based beam failure detection	634

8.5D.3.1	Introduction	634

8.5D.3.2	Minimum requirement	635

8.5D.3.3	Measurement restrictions for CSI-RS beam failure detection	636

8.5D.4	Minimum requirement for L1 indication	636

8.5D.5	Requirements for SSB based candidate beam detection	636

8.5D.5.1	Introduction	636

8.5D.5.2	Minimum requirement	636

8.5D.5.3	Measurement restriction for SSB based candidate beam detection	637

8.5D.6	Requirements for CSI-RS based candidate beam detection	638

8.5D.6.1	Introduction	638

8.5D.6.2	Minimum requirement	638

8.5D.6.3	Measurement restriction for CSI-RS based candidate beam detection	639

8.5D.7	Scheduling availability of UE during beam failure detection	639

8.5D.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	639

8.5D.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	639

8.5D.8	Scheduling availability of UE during candidate beam detection	640

8.5D.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	640

8.5D.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	640

8.5D.9	Minimum requirement at transitions for beam failure detection	640

8.6	Active BWP switch delay	640

8.6.1	Introduction	640

8.6.2	DCI and timer based BWP switch delay on a single CC	640

8.6.2A	DCI based BWP switch delay on multiple CCs	642

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	642

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	643

8.6.2B	Timer based BWP switch delay on multiple CCs	644

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	644

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	644

8.6.3	RRC based BWP switch delay on a single CC	645

8.6.3A	RRC based BWP switch delay on multiple CCs	646

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	646

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	646

8.6.4	BWP switch delay on Consistent UL CCA recovery	647

8.6A	Active BWP switch delay for RedCap	647

8.6A.1	Introduction	647

8.6A.2	DCI and timer based BWP switch delay on a single CC	647

8.6A.3	RRC based BWP switch delay on a single CC	648

8.6C	Active BWP switch delay for satellite access	649

8.6C.1	Introduction	649

8.6C.2	DCI and timer based BWP switch delay on a single CC	649

8.6C.3	RRC based BWP switch delay on a single CC	651

8.6D	Active BWP switch delay for ATG	651

8.6D.1	Introduction	651

8.6D.2	DCI and timer based BWP switch delay	651

8.6D.3	RRC based BWP switch delay on a single CC	652

8.7	Void	653

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	653

8.8.1	Introduction	653

8.8.2	E-UTRAN PSCell Addition Delay Requirement	653

8.8.3	E-UTRAN PSCell Release Delay Requirement	653

8.9	NR-DC: PSCell Addition and Release Delay	654

8.9.1	Introduction	654

8.9.2	PSCell Addition Delay Requirement	654

8.9.3	PSCell Release Delay Requirement	655

8.9A	Conditional PSCell Addition Delay	655

8.9A.1	Introduction	655

8.9A.2	Conditional PSCell Addition Delay Requirement	655

8.9A.2.1	Measurement time	656

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	656

8.9B.1	Introduction	656

8.9B.2	PSCell Addition Delay Requirement	656

8.9B.3	PSCell Release Delay Requirement	657

8.9C	Subsequent Conditional PSCell Addition Delay	657

8.9C.1	Introduction	657

8.9C.2	Subsequent Conditional PSCell Addition Delay Requirement	657

8.9C.2.1	Measurement time	658

8.10	Active TCI state switching delay	658

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	659

8.10.4	DCI based TCI state switch delay	660

8.10.5	RRC based TCI state switch delay	660

8.10.6	Active TCI state list update delay	661

8.10A	Active TCI state switching delay with CCA	661

8.10A.1	Introduction	661

8.10A.2	Known conditions for TCI state	661

8.10A.3	MAC-CE based TCI state switch delay	662

8.10A.4	DCI based TCI state switch delay	663

8.10A.5	RRC based TCI state switch delay	663

8.10A.6	Active TCI state list update delay	664

8.10B	Active TCI state switching delay for RedCap	664

8.10B.1	Introduction	664

8.10B.2	Known conditions for TCI state	664

8.10B.3	MAC-CE based TCI state switch delay	664

8.10B.4	DCI based TCI state switch delay	665

8.10B.5	RRC based TCI state switch delay	665

8.10B.6	Active TCI state list update delay	666

8.10C	Active TCI state switching delay for satellite access	666

8.10C.1	Introduction	666

8.10C.2	MAC-CE based TCI state switch delay	666

8.10C.4	DCI based TCI state switch delay	666

8.10C.5	RRC based TCI state switch delay	667

8.10C.6	Active TCI state list update delay	667

8.10D	Active TCI state switching delay for ATG	667

8.10D.2	Void	667

8.10D.6	Active TCI state list update delay	668

8.10E	Active TCI state switching delay for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	668

8.10E.1	Introduction	668

8.10E.2	Known conditions for TCI state	668

8.10E.3	MAC-CE based dual DL TCI state switch delay	669

8.10E.3.1	MAC-CE based dual DL TCI state switching delay for sDCI	669

8.10E.3.2	MAC-CE based dual DL TCI state switching delay for mDCI	669

8.10E.4	DCI based dual DL TCI state switch delay for sDCI and mDCI	669

8.10E.5	RRC based dual DL TCI state switch delay	670

8.10E.6	Active DL TCI state list update delay	670

8.11	PSCell Change	670

8.11A	PSCell Change in Carriers with CCA	671

8.11B	Conditional PSCell Change	671

8.11B.1	Introduction	671

8.11B.2	Conditoinal PSCell Change delay	671

8.11B.2.1	Measurement time	672

8.11D	Conditional PSCell Change in Carriers with CCA	672

8.11D.1	Introduction	672

8.11D.2	Conditional PSCell Change delay	672

8.11D.2.1	Measurement time	673

8.11E	Subsequent Conditional PSCell Change	674

8.11E.1	Introduction	674

8.11E.2	Subsequent Conditoinal PSCell Change delay	674

8.11E.2.1	Measurement time	674

8.12	Uplink spatial relation switch delay	675

8.12.1	Introduction	675

8.12.2	Known conditions for spatial relation when associated with DL-RS	675

8.12.3	MAC-CE based spatial relation switch delay	675

8.12.4	DCI based spatial relation switch delay	676

8.12.5	RRC based spatial relation switch delay	676

8.12A	Uplink spatial relation switch delay for RedCap	677

8.12A.1	Introduction	677

8.12A.2	Known conditions for spatial relation when associated with DL-RS	677

8.12A.3	MAC-CE based spatial relation switch delay	677

8.12A.4	DCI based spatial relation switch delay	678

8.12A.5	RRC based spatial relation switch delay	678

8.12C	Uplink spatial relation switch delay for satellite access	679

8.12C.1	Void	679

8.12C.2	Void	679

8.12C.3	Void	679

8.12C.4	Void	679

8.12C.5	Void	679

8.13	UE-specific CBW change	679

8.13.1	Introduction	679

8.13.2	UE-specific CBW change delay	679

8.13A	UE-specific CBW change for RedCap	680

8.13A.1	Introduction	680

8.13A.2	UE-specific CBW change delay	680

8.13C	UE-specific CBW change for satellite access	680

8.13C.1	Introduction	680

8.13C.2	UE-specific CBW change delay	681

8.13D	UE-specific CBW change for ATG	681

8.13D.1	Introduction	681

8.13D.2	UE-specific CBW change delay	681

8.14	Pathloss reference signal switching delay	681

8.14.1	Introduction	681

8.14.2	Known conditions for pathloss reference signal	681

8.14.3	MAC-CE based pathloss reference signal switch delay	682

8.14C	Pathloss reference signal switching delay for satellite access	682

8.14C.1	Introduction	683

8.14C.2	Known conditions for pathloss reference signal	683

8.14C.3	MAC-CE based pathloss reference signal switch delay	683

8.14D	Pathloss reference signal switching delay for ATG	684

8.14D.1	Introduction	684

8.14D.2	Known conditions for pathloss reference signal	684

8.14D.3	MAC-CE based pathloss reference signal switch delay	684

8.15	Active downlink TCI state switching delay for unified TCI	684

8.15.1	Introduction	684

8.15.4	DCI based downlink TCI state switch delay	686

8.15.5	Active Downlink TCI state list update delay	686

8.15D	Active downlink TCI state switching delay for unified TCI for ATG	687

8.15D.1	Introduction	687

8.15D.2	Void	687

8.15D.4	DCI based downlink TCI state switch delay	688

8.15D.5	Active Downlink TCI state list update delay	688

8.16	Active uplink TCI state switching delay for unified TCI	688

8.16.1	Introduction	688

8.16.4	DCI based uplink TCI state switch delay	691

8.16.5	Active Uplink TCI state list update delay	691

8.16D	Active uplink TCI state switching delay for unified TCI for ATG	693

8.16D.1	Introduction	693

8.16D.2	Void	693

8.16D.3	MAC-CE based uplink TCI state switch delay	693

8.16D.4	DCI based uplink TCI state switch delay	694

8.16D.5	Active Uplink TCI state list update delay	694

8.17	SCG Activation and Deactivation Delay	695

8.17.1	Introduction	695

8.17.2	SCG Activation Delay Requirement	695

8.17.3	SCG Deactivation Delay Requirement	696

8.18	TRP specific Link Recovery Procedures	696

8.18.1	Introduction	696

8.18.2	Requirements for TRP specific SSB based beam failure detection	697

8.18.2.1	Introduction	697

8.18.2.2	Minimum requirement	697

8.18.2.3	Measurement restriction for SSB based beam failure detection	699

8.18.3	Requirements for CSI-RS based beam failure detection	700

8.18.3.1	Introduction	700

8.18.3.2	Minimum requirement	701

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	704

8.18.4	Minimum requirement for L1 indication	706

8.18.5	Requirements for SSB based candidate beam detection	706

8.18.5.1	Introduction	706

8.18.5.2	Minimum requirement	706

8.18.5.3	Measurement restriction for SSB based candidate beam detection	708

8.18.6	Requirements for CSI-RS based candidate beam detection	709

8.18.6.1	Introduction	709

8.18.6.2	Minimum requirement	709

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	712

8.18.7	Requirements for TRP specific Beam Failure Recovery	713

8.18.7.1	Introduction	713

8.18.7.2	Requirement	713

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	714

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	714

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	714

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	714

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	715

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	715

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	716

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	716

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	716

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	716

8.19	Pre-configured measurement gap activation/deactivation delay	717

8.19.1	Introduction	717

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	717

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	717

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	717

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	717

8.19.5	Activation/deactivation delay requirements for concurrent measurement gaps with Pre-MG	718

8.19.5.1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of concurrent measurement gaps with Pre-MG	718

8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of concurrent measurement gaps with Pre-MG	718

8.19.5.3	Pre-configured measurement gap activation/deactivation delay when colliding with a concurrent measurement gap	718

8.19D	Pre-configured measurement gap activation/deactivation delay for ATG	718

8.19D.1	Introduction	718

8.19D.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	719

8.19D.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	719

8.19D.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	719

8.20	PSCell Cell Switch	719

8.20.1	Introduction	719

8.20.2	LTM Cell Switch delay	720

8.20.3	Interruption time	720

8.21	Active downlink TCI state switching delay for unified TCI for single-DCI mTRP	720

8.21.1	Introduction	720

8.21.2	Known conditions for downlink TCI state	720

8.21.3	MAC-CE based downlink TCI state switch delay	721

8.21.4	DCI based downlink TCI state switch delay	722

8.21.5	Active Downlink TCI state list update delay	722

8.22	Active downlink TCI state switching delay for unified TCI for multi-DCI mTRP	723

8.22.1	Introduction	723

8.22.2	Known conditions for downlink TCI state	724

8.22.3	MAC-CE based downlink TCI state switch delay	724

8.22.4	DCI based downlink TCI state switch delay	725

8.22.5	Active Downlink TCI state list update delay	725

8.23	Active uplink TCI state switching delay for unified TCI for single-DCI mTRP	726

8.23.1	Introduction	726

8.23.2	Known conditions for uplink TCI state	726

8.23.3	MAC-CE based uplink TCI state switch delay	727

8.23.4	DCI based uplink TCI state switch delay	728

8.23.5	Active uplink TCI state list update delay	729

8.24	Active uplink TCI state switching delay for unified TCI for multi-DCI mTRP	729

8.24.1	Introduction	729

8.24.2	Known conditions for uplink TCI state	730

8.24.3	MAC-CE based uplink TCI state switch delay	730

8.24.4	DCI based uplink TCI state switch delay	732

8.24.5	Active Uplink TCI state list update delay	732

9	Measurement Procedure	733

9.1	General measurement requirement	733

9.1.1	Introduction	733

9.1.2	Measurement gap	733

9.1.2.1	EN-DC: Measurement Gap Sharing	745

9.1.2.1a	SA: Measurement Gap Sharing	746

9.1.2.1b	NE-DC: Measurement Gap Sharing	747

9.1.2.1c	NR-DC: Measurement Gap Sharing	748

9.1.3	UE Measurement capability	749

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	749

9.1.3.1a	SA: Monitoring of multiple layers using gaps	749

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	750

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	750

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	751

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	752

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	752

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	753

9.1A.3.2	Void	754

9.1.3A	UE Measurement capability under operation mode with CCA	754

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	754

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	754

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	754

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	755

9.1.3C	UE Measurement capability under operation mode with satellite access	755

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	755

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	756

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	756

9.1.4.1	Introduction	756

9.1.4.2	Requirements	756

9.1.5	Carrier-specific scaling factor	760

9.1.5.1	Monitoring of multiple layers outside gaps	761

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	764

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	765

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	766

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	767

9.1.5.2	Monitoring of multiple layers within gaps	768

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	771

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	772

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	775

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	777

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.3	Monitoring of multiple layers within NCSG	779

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	780

9.1.5.4	L1-RSRP measurements within measurement gap	781

9.1.5.4.1	SA mode: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	783

9.1.5.4.2	NR-DC: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	784

9.1.6	Minimum requirement at transitions	786

9.1.7	Pre-configured measurement gap	786

9.1.7.1	Introduction	786

9.1.7.2	Requirements applicability	786

9.1.7.3	Requirements	787

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	787

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	788

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	788

9.1.8	Concurrent measurement gaps	788

9.1.8.1	Introduction	788

9.1.8.2	Requirements	789

9.1.8.3	Collision between concurrent measurement gaps	790

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	790

9.1.9	Network controlled small gap	791

9.1.9.1	Introduction	791

9.1.9.2	Requirements applicability	792

9.1.10	MUSIM gaps	794

9.1.10.1	Introduction	795

9.1.10.2	Priorities for MUSIM gaps	796

9.1.10.3	Keep solution for MUSIM gaps	796

9.1.10.4	Collisions between different MUSIM gaps	796

9.1.10.5	Collisions between MUSIM gaps and measurement gaps	796

9.1.10.6	Measurement gap related requirements of MUSIM gaps	797

9.1.11	UL gap for Tx power management	797

9.1.12	Concurrent measurement gaps with Pre-MG	797

9.1.12.1	Introduction	797

9.1.12.2	Requirements	797

9.1.12.3	Collisions involving Pre-MG(s)	798

9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap	799

9.1.13	Concurrent measurement gaps with NCSG	799

9.1.13.1	Introduction	799

9.1.13.2	Requirements	799

9.1.13.3	Collision involving NCSGs	801

9.1A	General measurement requirement for RedCap	801

9.1A.1	Introduction	801

9.1A.2	Measurement gap	801

9.1A.2.1	SA: Measurement Gap Sharing	805

9.1A.3	UE Measurement capability	806

9.1A.3.1	SA: Monitoring of multiple layers using gaps	806

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	806

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	806

9.1A.4.1	Introduction	806

9.1A.4.2	Requirements	807

9.1A.5	Carrier-specific scaling factor	807

9.1A.5.1	Monitoring of multiple layers outside gaps	807

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	808

9.1A.5.2	Monitoring of multiple layers within gaps	808

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	808

9.1A.6	Minimum requirement at transitions	810

9.1C	General measurement requirement for SAN	810

9.1C.1	Introduction	811

9.1C.2	Measurement gap	811

9.1C.8	Concurrent measurement gaps for SAN	812

9.1C.8.1	Introduction	812

9.1C.8.2	Requirements	812

9.1C.8.3	Collision between concurrent measurement gaps	813

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	813

9.1C.9	Collision between SMTC and measurement gap for SAN	813

9.1C.9.1	Introduction	813

9.1C.9.2	Collision between SMTCs and measurement gap	814

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	814

9.1D	General measurement requirement for ATG	814

9.1D.1	Introduction	814

9.1D.2	Measurement gap	814

9.1D.2.1a	SA: Measurement Gap Sharing	817

9.1D.3	UE Measurement capability	817

9.1D.3.1	SA: Monitoring of multiple layers using gaps	817

9.1D.3.2	SA: Maximum allowed layers for multiple monitoring	817

9.1D.4	Void	818

9.1D.5	Carrier-specific scaling factor	818

9.1D.5.1	Monitoring of multiple layers outside gaps	818

9.1D.5.1.1	Void	819

9.1D.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements performed outside gaps	819

9.1D.5.2	Monitoring of multiple layers within gaps	819

9.1D.5.2.1	Void	820

9.1D.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements performed within gaps	820

9.1D.6	Void	821

9.1D.7	Pre-configured measurement gap	821

9.1D.7.1	Introduction	821

9.1D.7.2	Requirements applicability	821

9.1D.7.3	Requirements	821

9.1D.7.3.1	Requirements for autonomous activation/deactivation mechanism	821

9.1D.7.3.2	Requirements for network-controlled activation/deactivation mechanism	822

9.1D.7.3.3	Requirements for reception/transmission during activation/deactivation	822

9.1D.8	Capabilities for Support of Event Triggering and Reporting Criteria	822

9.1D.8.1	Introduction	822

9.1D.8.2	Requirements	823

9.1D.9	Minimum requirement at transitions	823

9.2	NR intra-frequency measurements	824

9.2.1	Introduction	824

9.2.2	Requirements applicability	826

9.2.3	Number of cells and number of SSB	827

9.2.3.1	Requirements for FR1	827

9.2.3.2	Requirements for FR2	827

9.2.4	Measurement Reporting Requirements	827

9.2.4.1	Periodic Reporting	827

9.2.4.2	Event-triggered Periodic Reporting	827

9.2.4.3	Event Triggered Reporting	827

9.2.4.4	SCell activation Triggered Reporting	828

9.2.5	Intrafrequency measurements without measurement gaps	829

9.2.5.1	Intrafrequency cell identification	829

9.2.5.2	Measurement period	837

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	841

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	841

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	842

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	842

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	844

9.2.5.4	SFTD Measurements between PCell and PSCell	844

9.2.5.4.1	Introduction	844

9.2.5.4.2	SFTD Measurement delay	844

9.2.5.4.3	SFTD Measurement Reporting Delay	845

9.2.6	Intra-frequency measurements with measurement gaps	845

9.2.6.1	Void	845

9.2.6.2	Intra-frequency cell identification	845

9.2.6.3	Intrafrequency Measurement Period	848

9.2.7	Intra-frequency measurements with NCSG	850

9.2.7.1	Intra-frequency cell identification	850

9.2.7.2	Measurement period	852

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	853

9.2A	NR intra-frequency measurements with CCA	853

9.2A.1	Introduction	853

9.2A.2	Requirements applicability	854

9.2A.3	Number of cells and number of SSB	855

9.2A.3.1	Requirements for FR1	855

9.2A.3.2	Requirements for FR2-2	855

9.2A.4	Measurement Reporting Requirements	855

9.2A.5	Intra-frequency measurements without measurement gaps	856

9.2A.5.2	Measurement period	860

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	862

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	863

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	863

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	863

9.2A.6	Intra-frequency measurements with measurement gaps	863

9.2A.6.1	Intra-frequency cell identification	863

9.2A.6.2	Intra-frequency Measurement Period	866

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	867

9.2A.7.1	Intra-frequency RSSI measurements	867

9.2A.7.2	Intra-frequency Channel occupancy measurements	869

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	870

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	870

9.2B	NR intra-frequency measurements for RedCap	871

9.2B.1	Introduction	871

9.2B.2	Requirements applicability	871

9.2B.3	Number of cells and number of SSB	872

9.2B.3.1	Requirements for FR1	872

9.2B.3.2	Requirements for FR2	872

9.2B.4	Measurement Reporting Requirements	872

9.2B.4.1	Periodic Reporting	872

9.2B.4.2	Event-triggered Periodic Reporting	872

9.2B.4.3	Event Triggered Reporting	872

9.2B.5	Intra-frequency measurements without measurement gaps	873

9.2B.5.1	Intra-frequency cell identification	873

9.2B.5.2	Measurement period	875

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	876

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	876

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	876

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	877

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	877

9.2B.6	Intra-frequency measurements with measurement gaps	878

9.2B.6.1	Intra-frequency cell identification	878

9.2B.6.2	Intra-frequency Measurement Period	879

9.2C	NR intra-frequency measurements for SAN	880

9.2C.1	Introduction	880

9.2C.2	Requirements applicability	881

9.2C.3	Number of cells and number of SSB	881

9.2C.3.1	Requirements for FR1	881

9.2C.4	Measurement Reporting Requirements	881

9.2C.4.1	Periodic Reporting	881

9.2C.4.2	Event-triggered Periodic Reporting	882

9.2C.4.3	Event Triggered Reporting	882

9.2C.5	Intra frequency measurements without measurement gaps	882

9.2C.5.1	Intra frequency cell identification	882

9.2C.5.2	Measurement period	884

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	885

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	885

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	885

9.2C.6	Intra-frequency measurements with measurement gaps	886

9.2C.6.1	Intra-frequency cell identification	886

9.2C.6.3	Intrafrequency Measurement Period	887

9.2D	NR intra-frequency measurements for ATG	887

9.2D.1	Introduction	887

9.2D.2	Requirements applicability	888

9.2D.3	Number of cells and number of SSB	888

9.2D.3.1	Requirements for FR1	888

9.2D.4	Measurement Reporting Requirements	888

9.2D.4.1	Periodic Reporting	888

9.2D.4.2	Event-triggered Periodic Reporting	888

9.2D.4.3	Event Triggered Reporting	888

9.2D.5	Intra-frequency measurements without measurement gaps	889

9.2D.5.1	Intra-frequency cell identification	889

9.2D.5.2	Measurement period	891

9.2D.5.3	Scheduling availability of UE during intra-frequency measurements	892

9.2D.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	892

9.2D.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	893

9.2D.6	Intra-frequency measurements with measurement gaps	893

9.2D.6.1	Void	893

9.2D.6.2	Intra-frequency cell identification	893

9.2D.6.3	Intra-frequency Measurement Period	894

9.3	NR inter-frequency measurements	895

9.3.1	Introduction	895

9.3.2	Requirements applicability	897

9.3.2.1	Void	898

9.3.2.2	Void	898

9.3.3	Number of cells and number of SSB	898

9.3.3.1	Requirements for FR1	898

9.3.3.2	Requirements for FR2	898

9.3.4	Inter-frequency measurement with measurement gaps	898

9.3.4.1	Void	903

9.3.4.2	Void	903

9.3.5	Inter-frequency measurements	903

9.3.5.1	Void	904

9.3.5.2	Void	904

9.3.5.3	Void	904

9.3.6	Inter-frequency measurements reporting requirements	904

9.3.6.1	Periodic Reporting	904

9.3.6.2	Event-triggered Periodic Reporting	904

9.3.6.3	Event-triggered Reporting	905

9.3.7	Void	905

9.3.8	Inter-frequency SFTD measurement requirements	905

9.3.8.1	Introduction	905

9.3.8.2	SFTD Measurement delay	905

9.3.8.3	SFTD Measurement reporting delay	906

9.3.9	Inter frequency measurements without measurement gaps	907

9.3.9.1	Inter frequency Cell identification	907

9.3.9.2	Measurement period	912

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	914

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	914

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	915

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	915

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	916

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	916

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	916

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	917

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	917

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	919

9.3.10	Inter-frequency measurement with NCSG	919

9.3.10.1	Inter-frequency cell identification	919

9.3.10.2	Measurement period	921

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	922

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	922

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	923

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	923

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	925

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	925

9.3A.1	Introduction	925

9.3A.2	Requirements applicability	926

9.3A.3	Number of cells and number of SSB	926

9.3A.3.1	Requirements for FR1	926

9.3A.3.2	Requirements for FR2-2	926

9.3A.4	Inter-frequency cell identification	927

9.3A.5	Inter-frequency measurements	929

9.3A.6	NR Inter-frequency measurements reporting requirements	930

9.3A.6.1	Periodic Reporting	930

9.3A.6.2	Event-triggered Periodic Reporting	930

9.3A.6.3	Event-triggered Reporting	930

9.3A.8	Inter-frequency RSSI measurements	931

9.3A.9	Inter-frequency channel occupancy measurements	932

9.3B	NR inter-frequency measurements for RedCap	932

9.3B.1	Introduction	932

9.3B.2	Requirements applicability	933

9.3B.3	Number of cells and number of SSB	933

9.3B.3.1	Requirements for FR1	933

9.3B.3.2	Requirements for FR2	933

9.3B.4	Inter-frequency measurement with measurement gaps	934

9.3B.5	Inter-frequency measurements	935

9.3B.6	Inter-frequency measurements reporting requirements	936

9.3B.6.1	Periodic Reporting	936

9.3B.6.2	Event-triggered Periodic Reporting	936

9.3B.6.3	Event-triggered Reporting	936

9.3B.7	Inter frequency measurements without measurement gaps	937

9.3B.7.1	Inter frequency Cell identification	937

9.3B.7.2	Measurement period	939

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	940

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	940

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	940

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	941

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	941

9.3C	NR inter-frequency measurements for SAN	941

9.3C.1	Introduction	941

9.3C.2	Requirements applicability	942

9.3C.3	Number of cells and number of SSB	942

9.3C.3.1	Requirements for FR1	942

9.3C.4	Inter-frequency measurement with measurement gaps	942

9.3C.5	Inter-frequency measurements	944

9.3C.6	Inter-frequency measurements reporting requirements	944

9.3C.6.1	Periodic Reporting	944

9.3C.6.2	Event-triggered Periodic Reporting	944

9.3C.6.3	Event-triggered Reporting	944

9.3C.7	Inter frequency measurements without measurement gaps	945

9.3C.7.1	Inter frequency Cell identification	945

9.3C.7.2	Measurement period	947

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	947

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	947

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	947

9.3D	NR inter-frequency measurements for ATG	948

9.3D.1	Introduction	948

9.3D.2	Requirements applicability	948

9.3D.3	Number of cells and number of SSB	949

9.3D.3.1	Requirements for FR1	949

9.3D.4	Inter-frequency measurement with measurement gaps	949

9.3D.5	Inter-frequency measurements	950

9.3D.6	Inter-frequency measurements reporting requirements	950

9.3D.6.1	Periodic Reporting	950

9.3D.6.2	Event-triggered Periodic Reporting	951

9.3D.6.3	Event-triggered Reporting	951

9.3D.7	Void	951

9.3D.8	Void	951

9.3D.9	Inter frequency measurements without measurement gaps	951

9.3D.9.1	Inter frequency Cell identification	951

9.3D.9.2	Measurement period	953

9.3D.9.3	Scheduling availability of UE during inter-frequency measurements	954

9.3D.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	954

9.3D.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	955

9.4	Inter-RAT measurements	955

9.4.1	Introduction	955

9.4.2	NR − E-UTRAN FDD measurements	957

9.4.2.1	Introduction	957

9.4.2.2	Requirements when no DRX is used	957

9.4.2.3	Requirements when DRX is used	959

9.4.2.4	Measurement reporting requirements	960

9.4.2.4.1	Periodic Reporting	960

9.4.2.4.2	Event-Triggered Periodic Reporting	960

9.4.2.4.3	Event-Triggered Reporting	961

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	961

9.4.3	NR − E-UTRAN TDD measurements	961

9.4.3.1	Introduction	961

9.4.3.2	Requirements when no DRX is used	961

9.4.3.3	Requirements when DRX is used	963

9.4.3.4	Measurement reporting requirements	965

9.4.3.4.1	Periodic Reporting	965

9.4.3.4.2	Event-Triggered Periodic Reporting	965

9.4.3.4.3	Event-Triggered Reporting	965

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	966

9.4.4	Inter-RAT RSTD measurements	966

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	966

9.4.4.1.1	Introduction	966

9.4.4.1.2	Requirements	966

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	969

9.4.4.2.1	Introduction	969

9.4.4.2.2	Requirements	970

9.4.5	Inter-RAT E-CID measurements	973

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	973

9.4.5.1.1	Introduction	973

9.4.5.1.2	Requirements	973

9.4.5.1.3	Measurement Reporting Delay	974

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	974

9.4.5.2.1	Introduction	974

9.4.5.2.2	Requirements	974

9.4.5.2.3	Measurement Reporting Delay	974

9.4.6	NR − UTRAN FDD measurements	974

9.4.6.1	Introduction	974

9.4.6.2	Requirements when no DRX is used	974

9.4.6.3	Requirements when DRX is used	976

9.4.7	NR – E-UTRAN measurements with autonomous gaps	978

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	978

9.4.7.2	CGI reporting delay	978

9.4.8	NR – E-UTRAN measurements without measurement gaps	978

9.4.8.1	Introduction	978

9.4.8.2	General requirements	978

9.4.8.3	NR − E-UTRAN FDD measurements	979

9.4.8.3.1	Introduction	979

9.4.8.3.2	Requirements when no DRX is used	979

9.4.8.3.3	Requirements when DRX is used	980

9.4.8.3.4	Measurement reporting requirements	981

9.4.8.3.4.1	Periodic Reporting	981

9.4.8.3.4.2	Event-Triggered Periodic Reporting	981

9.4.8.3.4.3	Event-Triggered Reporting	981

9.4.8.3.5	Scheduling availability during NR − E-UTRAN FDD measurements	982

9.4.8.3.5.1	Scheduling availability of UE performing inter-RAT measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	982

9.4.8.4	NR − E-UTRAN TDD measurements	982

9.4.8.4.1	Introduction	982

9.4.8.4.2	Requirements when no DRX is used	982

9.4.8.4.3	Requirements when DRX is used	984

9.4.8.4.4	Measurement reporting requirements	985

9.4.8.4.5	Scheduling availability during NR − E-UTRAN TDD measurements	985

9.4A	Inter-RAT measurements for RedCap	986

9.4A.1	Introduction	986

9.4A.2	NR − E-UTRAN FDD measurements	987

9.4A.2.1	Introduction	987

9.4A.2.2	Requirements when no DRX is used	988

9.4A.2.3	Requirements when DRX is used	989

9.4A.2.4	Measurement reporting requirements	990

9.4A.2.4.1	Periodic Reporting	990

9.4A.2.4.2	Event-Triggered Periodic Reporting	990

9.4A.2.4.3	Event-Triggered Reporting	990

9.4A.3	NR − E-UTRAN TDD measurements	990

9.4A.3.1	Introduction	990

9.4A.3.2	Requirements when no DRX is used	991

9.4A.3.3	Requirements when DRX is used	993

9.4A.3.4	Measurement reporting requirements	994

9.4A.3.4.1	Periodic Reporting	994

9.4A.3.4.2	Event-Triggered Periodic Reporting	994

9.4A.3.4.3	Event-Triggered Reporting	994

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	994

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	994

9.4A.4.2	CGI reporting delay	995

9.4A.4.3	CGI reporting scheduling restriction	995

9.5	L1-RSRP measurements for Reporting	996

9.5.1	Introduction	996

9.5.2	Requirements applicability	996

9.5.3	Measurement Reporting Requirements	997

9.5.3.1	Periodic Reporting	997

9.5.3.2	Semi-Persistent Reporting	997

9.5.3.3	Aperiodic Reporting	998

9.5.4	L1-RSRP measurement requirements	998

9.5.4.1	SSB based L1-RSRP Reporting	998

9.5.4.2	CSI-RS based L1-RSRP Reporting	1004

9.5.4A	Void	1007

9.5.4A.1	Void	1007

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1008

9.5.5.1	Measurement restriction for SSB based L1-RSRP	1008

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	1008

9.5.6	Scheduling availability of UE during L1-RSRP measurement	1010

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1010

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1010

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1010

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1012

9.5A	L1-RSRP measurements for Reporting under CCA	1012

9.5A.1	Introduction	1012

9.5A.2	Requirements applicability	1013

9.5A.3	Measurement Reporting Requirements	1013

9.5A.3.1	Periodic Reporting	1013

9.5A.3.2	Semi-Persistent Reporting	1014

9.5A.3.3	Aperiodic Reporting	1014

9.5A.4	L1-RSRP measurement requirements	1014

9.5A.4.1	SSB based L1-RSRP Reporting	1014

9.5A.5	Measurement restriction for L1-RSRP measurement	1017

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	1017

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	1017

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1017

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1018

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	1018

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	1018

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1019

9.5B	L1-RSRP measurements for Reporting for RedCap	1019

9.5B.1	Introduction	1019

9.5B.2	Requirements applicability	1019

9.5B.3	Measurement Reporting Requirements	1020

9.5B.3.1	Periodic Reporting	1020

9.5B.3.2	Semi-Persistent Reporting	1020

9.5B.3.3	Aperiodic Reporting	1020

9.5B.4	L1-RSRP measurement requirements	1021

9.5B.4.1	SSB based L1-RSRP Reporting	1021

9.5B.4.2	CSI-RS based L1-RSRP Reporting	1023

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1025

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	1025

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	1026

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	1026

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1026

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1027

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1027

9.5C	L1-RSRP measurements for Reporting for satellite access	1028

9.5C.1	Introduction	1028

9.5C.3	Measurement Reporting Requirements	1028

9.5C.3.1	Periodic Reporting	1028

9.5C.3.2	Semi-Persistent Reporting	1029

9.5C.3.3	Aperiodic Reporting	1029

9.5C.4	L1-RSRP measurement requirements	1029

9.5C.4.1	SSB based L1-RSRP Reporting	1029

9.5C.5	Measurement restriction for L1-RSRP measurement	1031

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	1031

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	1031

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	1032

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1032

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1032

9.5D	L1-RSRP measurements for Reporting for ATG	1032

9.5D.1	Introduction	1032

9.5D.2	Requirements applicability	1032

9.5D.3	Measurement Reporting Requirements	1033

9.5D.3.1	Periodic Reporting	1033

9.5D.3.2	Semi-Persistent Reporting	1033

9.5D.3.3	Aperiodic Reporting	1033

9.5D.4	L1-RSRP measurement requirements	1033

9.5D.4.1	SSB based L1-RSRP Reporting	1033

9.5D.4.2	CSI-RS based L1-RSRP Reporting	1035

9.5D.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1036

9.5D.5.1	Measurement restriction for SSB based L1-RSRP	1036

9.5D.5.2	Measurement restriction for CSI-RS based L1-RSRP	1037

9.5D.6	Scheduling availability of UE during L1-RSRP measurement	1037

9.5D.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1037

9.5D.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1037

9.6	NE-DC: Measurements	1037

9.6.1	Introduction	1037

9.6.2	SFTD Measurements	1038

9.6.2.1	Introduction	1038

9.6.2.2	SFTD Measurement requirements	1038

9.7	Cross Link Interference measurements	1038

9.7.1	Introduction	1038

9.7.2	SRS-RSRP measurements	1039

9.7.2.1	Introduction	1039

9.7.2.2	Requirements applicability	1039

9.7.2.3	Measurement Reporting Requirements	1039

9.7.2.3.1	Periodic Reporting	1039

9.7.2.3.2	Event-triggered Periodic Reporting	1039

9.7.2.3.3	Event Triggered Reporting	1039

9.7.2.4	Measurement capability	1040

9.7.2.5	SRS-RSRP measurement period	1040

9.7.3	CLI-RSSI measurements	1040

9.7.3.1	Introduction	1040

9.7.3.2	Requirements applicability	1040

9.7.3.3	Measurement Reporting Requirements	1040

9.7.3.3.1	Periodic Reporting	1041

9.7.3.3.2	Event-triggered Periodic Reporting	1041

9.7.3.3.3	Event Triggered Reporting	1041

9.7.3.4	Measurement capability	1041

9.7.3.5	CLI-RSSI measurement period	1041

9.7.4	Scheduling availability of UE during CLI measurements	1041

9.7.4.1	Scheduling availability of UE performing measurement on FR1	1041

9.7.4.2	Scheduling availability of UE performing measurement on FR2	1042

9.8	L1-SINR measurements for Reporting	1043

9.8.1	Introduction	1043

9.8.2	Requirements applicability	1043

9.8.3	Measurement Reporting Requirements	1044

9.8.3.1	Periodic Reporting	1044

9.8.3.2	Semi-Persistent Reporting	1044

9.8.4	L1-SINR measurement requirements	1045

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1045

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1050

9.8.5	Measurement restriction for L1-SINR measurement	1051

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1052

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1052

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1054

9.8.6	Scheduling availability of UE during L1-SINR measurement	1054

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1054

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1054

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1056

9.8D	L1-SINR measurements for Reporting for ATG	1056

9.8D.1	Introduction	1056

9.8D.2	Requirements applicability	1056

9.8D.3	Measurement Reporting Requirements	1057

9.8D.3.1	Periodic Reporting	1057

9.8D.3.2	Semi-Persistent Reporting	1057

9.8D.3.3	Aperiodic Reporting	1058

9.8D.4	L1-SINR measurement requirements	1058

9.8D.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1058

9.8D.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1059

9.8D.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1060

9.9	NR measurements for positioning	1061

9.9.1	Introduction	1061

9.9.1.1	General Aspects of Gap-based Measurement	1061

9.9.1.2	General Aspects of Gapless Measurement	1062

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1063

9.9.2	RSTD measurements	1064

9.9.2.1	Introduction	1064

9.9.2.2	Requirements Applicability	1064

9.9.2.3	Measurement Capability	1064

9.9.2.4	Measurement Reporting Requirements	1064

9.9.2.4.1	Void	1065

9.9.2.4.2	Void	1065

9.9.2.4.3	Void	1065

9.9.2.5	Measurements Period Requirements	1065

9.9.2.6	Void	1068

9.9.2.7	Measurements Period Requirements without Measurement Gaps	1068

9.9.2.8	Void	1071

9.9.2.9	Measurements Period Requirements with both MG and PPW	1071

9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation	1071

9.9.3	PRS-RSRP measurements	1075

9.9.3.1	Introduction	1075

9.9.3.2	Requirements applicability	1075

9.9.3.3	Measurement Capability	1075

9.9.3.4	Measurement Reporting Requirements	1075

9.9.3.5	Measurement Period Requirements	1075

9.9.3.6	Measurement Period Requirements without Measurement Gaps	1078

9.9.3.7	Void	1080

9.9.3.8	Measurements Period Requirements with both MG and PPW	1080

9.9.4	UE Rx-Tx time difference measurements	1081

9.9.4.1 Introduction	1081

9.9.4.2 Requirements Applicability	1081

9.9.4.3 Measurement Capability	1081

9.9.4.4 Measurement Reporting Requirements	1081

9.9.4.5	Measurement Period Requirements	1082

9.9.4.6	Measurement Period Requirements without Measurement Gaps	1085

9.9.4.7	Void	1088

9.9.4.8	Measurements Period Requirements with both MG and PPW	1089

9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation	1089

9.9.5	E-CID measurements	1093

9.9.5.1	Introduction	1093

9.9.5.2	Measurement Requirements	1093

9.9.5.2.1	Intra-frequency Measurement Requirements	1093

9.9.5.2.2	Inter-frequency Measurement Requirements	1093

9.9.5.2.3	Measurement Reporting Delay	1094

9.9.6	PRS-RSRPP measurements	1094

9.9.6.1	Introduction	1094

9.9.6.2	Requirements applicability	1094

9.9.6.3	Measurement capability	1094

9.9.6.4	Measurement reporting requirements	1094

9.9.6.5	Measurement period requirements	1095

9.9.6.6	Measurement Period Requirements without Measurement Gaps	1095

9.9.6.7	Void	1095

9.9.6.8	Measurements Period Requirements with both MG and PPW	1095

9.9.7	Measurement requirements for DL RSCPD reported with RSTD	1095

9.9.7.1	Introduction	1095

9.9.7.2	Requirements Applicability	1095

9.9.7.3	Measurement Capability	1095

9.9.7.4	Measurement Reporting Requirements	1095

9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD	1096

9.9.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	1098

9.9.8.1	Introduction	1098

9.9.8.2	Requirements Applicability	1098

9.9.8.3	Measurement Capability	1098

9.9.8.4	Measurement Reporting Requirements	1098

9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference	1099

9.9A	NR measurements for positioning for RedCap	1101

9.9A.1	Introduction	1101

9.9A.1.1	General Aspects of Gap-based Measurement	1101

9.9A.1.2	General Aspects of Gapless Measurement for RedCap positioning without FH	1102

9.9A.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps for RedCap positioning without FH	1103

9.9A.2	RSTD measurements for RedCap	1104

9.9A.2.1	Introduction	1104

9.9A.2.2	Requirements Applicability	1104

9.9A.2.3	Measurement Capability	1104

9.9A.2.4	Measurement Reporting Requirements	1105

9.9A.2.5	Measurements Period Requirements without FH	1105

9.9A.2.5.1	Measurements Period Requirements without FH with MG	1105

9.9A.2.5.2	Measurements Period Requirements without FH without MG	1108

9.9A.2.5.3	Measurements Period Requirements without FH with both MG and PPW	1111

9.9A.2.6	Measurements Period Requirements with FH	1111

9.9A.2.6.1	Measurements Period Requirements with FH with MG	1111

9.9A.3	PRS-RSRP measurements for RedCap	1112

9.9A.3.1	Introduction	1112

9.9A.3.2	Requirements applicability	1112

9.9A.3.3	Measurement Capability	1113

9.9A.3.4	Measurement Reporting Requirements	1113

9.9A.3.5	Measurements Period Requirements without FH	1113

9.9A.3.5.1	Measurement Period Requirements without FH with MG	1113

9.9A.3.5.2	Measurement Period Requirements without FH without MG	1115

9.9A.3.5.3	Measurements Period Requirements without FH with both MG and PPW	1118

9.9A.3.6	Measurements Period Requirements with FH	1119

9.9A.3.6.1	Measurements Period Requirements with FH with MG	1119

9.9A.4	UE Rx-Tx time difference measurements for RedCap	1120

9.9A.4.1	Introduction	1120

9.9A.4.2	Requirements Applicability	1120

9.9A.4.3	Measurement Capability	1120

9.9A.4.4	Measurement Reporting Requirements	1120

9.9A.4.5	Measurement Period Requirements without FH with MG	1120

9.9A.4.6	Measurement Period Requirements without FH without MG	1120

9.9A.4.7	Measurements Period Requirements without FH with both MG and PPW	1120

9.9A.4.8	Measurements Period Requirements with FH	1121

9.9A.5	PRS-RSRPP measurements for RedCap	1122

9.9A.5.1	Introduction	1122

9.9A.5.2	Requirements Applicability	1122

9.9A.5.3	Measurement Capability	1122

9.9A.5.4	Measurement Reporting Requirements	1122

9.9A.5.5	Measurement Period Requirements without FH with MG	1122

9.9A.5.6	Measurement Period Requirements without FH without MG	1122

9.9A.5.7	Measurements Period Requirements without FH with both MG and PPW	1123

9.9A.5.8	Measurements Period Requirements with FH	1123

9.10	CSI-RS based L3 measurements	1123

9.10.1	Introduction	1123

9.10.2	CSI-RS based intra-frequency measurements	1123

9.10.2.1	Introduction	1123

9.10.2.2	Requirements applicability	1124

9.10.2.3	Number of cells and number of CSI-RS	1125

9.10.2.3.1	Requirements for FR1	1125

9.10.2.3.2	Requirements for FR2	1125

9.10.2.4	Measurement Reporting Requirements	1125

9.10.2.4.1	Periodic Reporting	1125

9.10.2.4.2	Event-triggered Periodic Reporting	1125

9.10.2.4.3	Event Triggered Reporting	1126

9.10.2.5	Intra-frequency measurements without measurement gaps	1126

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1128

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1128

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	1128

9.10.3	CSI-RS based Inter-frequency measurements	1129

9.10.3.1	Introduction	1129

9.10.3.2	Requirements applicability	1129

9.10.3.3	Number of cells and number of CSI-RS resources	1130

9.10.3.3.1	Requirements for FR1	1130

9.10.3.3.2	Requirements for FR2	1130

9.10.3.4	Measurements reporting requirements	1130

9.10.3.4.1	Periodic Reporting	1130

9.10.3.4.2	Event-triggered Periodic Reporting	1130

9.10.3.4.3	Event-triggered Reporting	1130

9.10.3.5	Inter frequency measurements with measurement gaps	1131

9.10D	CSI-RS based L3 measurements for ATG	1132

9.10D.1	Introduction	1132

9.10D.2	CSI-RS based intra-frequency measurements	1133

9.10D.2.1	Introduction	1133

9.10D.2.2	Requirements applicability	1133

9.10D.2.3	Number of cells and number of CSI-RS	1134

9.10D.2.3.1	Requirements for FR1	1134

9.10D.2.4	Measurement Reporting Requirements	1134

9.10D.2.4.1	Periodic Reporting	1134

9.10D.2.4.2	Event-triggered Periodic Reporting	1134

9.10D.2.4.3	Event Triggered Reporting	1135

9.10D.2.5	Intra-frequency measurements without measurement gaps	1135

9.10D.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1137

9.10D.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1137

9.10D.3	CSI-RS based Inter-frequency measurements	1137

9.10D.3.1	Introduction	1137

9.10D.3.2	Requirements applicability	1137

9.10D.3.3	Number of cells and number of CSI-RS resources	1138

9.10D.3.3.1	Requirements for FR1	1138

9.10D.3.4	Measurements reporting requirements	1138

9.10D.3.4.1	Periodic Reporting	1138

9.10D.3.4.2	Event-triggered Periodic Reporting	1138

9.10D.3.4.3	Event-triggered Reporting	1138

9.10D.3.5	Inter frequency measurements with measurement gaps	1139

9.11	NR measurements with autonomous gaps	1140

9.11.1	Introduction	1140

9.11.2	CGI identification of an NR cell with autonomous gaps	1140

9.11.3	CGI reporting delay	1141

9.11A	NR measurements with autonomous gaps for RedCap	1142

9.11A.1	Introduction	1142

9.11A.2	CGI identification of an NR cell with autonomous gaps	1142

9.11A.3	CGI reporting delay	1143

9.11A.4	CGI reporting scheduling restriction	1143

9.11D	NR measurements with autonomous gaps for ATG	1143

9.11D.1	Introduction	1144

9.11D.2	CGI identification of an NR cell with autonomous gaps	1144

9.11D.3	CGI reporting delay	1145

9.12	Measurement for Propagation Delay Compensation	1145

9.12.1	Introduction	1145

9.12.2	Requirements Applicability	1145

9.12.3	Measurement Capability	1145

9.12.4	Measurement period requirements	1145

9.12.4.1	PRS Measurement Period	1145

9.12.4.2	TRS Measurement Period	1146

9.12.5	Measurement Reporting Requirements	1147

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	1147

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	1148

9.12.8	Measurement requirement for Propagation Delay Compensation with MUSIM gaps	1148

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	1148

9.13.1	Introduction	1148

9.13.2	Requirements Applicability	1148

9.13.3	Measurement Reporting Requirements	1149

9.13.3.1	Periodic Reporting	1149

9.13.3.2	Semi-Persistent Reporting	1149

9.13.3.3	Aperiodic Reporting	1150

9.13.4	L1-RSRP measurement requirements	1150

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	1150

9.13.5	Measurement restriction for L1-RSRP measurement	1153

9.13.5.1	Measurement restriction for SSB based L1-RSRP	1153

9.13.6	Scheduling availability of UE during L1-RSRP measurement	1154

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1154

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1154

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1155

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1155

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1155

9.14	Intra-frequency L1-RSRP measurements for neighbor cell	1156

9.14.1	Introduction	1156

9.14.2	Requirements Applicability	1156

9.14.3	Measurement Reporting Requirements	1156

9.14.3.1	Periodic Reporting	1157

9.14.3.2	Semi-Persistent Reporting	1157

9.14.3.3	Aperiodic Reporting	1157

9.14.4	Number of cells and number of SSB	1157

9.14.5	L1-RSRP measurement requirements without measurement gaps	1157

9.14.5.1	SSB based L1-RSRP Reporting	1157

9.14.6	Measurement restriction for L1-RSRP measurement	1160

9.14.6.1	Measurement restriction for SSB based L1-RSRP	1160

9.14.7	Scheduling availability of UE during L1-RSRP measurement	1161

9.14.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1161

9.14.7.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1161

9.14.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1161

9.14.7.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1162

9.14.7.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1162

9.15	NR inter-frequency L1 measurement	1163

9.15.1	Introduction	1163

9.15.2	Requirements Applicability	1163

9.15.3	Measurement Reporting Requirements	1163

9.15.3.1	Periodic Reporting	1164

9.15.3.2	Semi-Persistent Reporting	1164

9.15.3.3	Aperiodic Reporting	1164

9.15.4	Number of cells and number of SSB	1164

9.15.5	inter-frequency L1-RSRP with MG	1164

9.15.5.1	Inter-frequency SSB based L1-RSRP Reporting	1164

9.15.6	Inter frequency L1-RSRP measurement without measurement gaps	1165

9.15.6.1	Inter frequency L1-RSRP measurement requirements	1165

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement	1165

9.15.6.2	Measurement restriction for inter frequency L1-RSRP measurement	1168

9.15.6.2.1	Measurement restriction for SSB based L1-RSRP	1168

9.15.6.3	Scheduling availability of UE during inter-frequency L1-RSRP measurements	1169

9.15.6.3.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1169

9.15.6.3.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1169

9.15.6.3.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1170

9.15.6.3.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1170

9.15.6.3.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1170

10	Measurement Performance requirements	1177

10.1	NR measurements	1177

10.1.1	Introduction	1177

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	1177

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	1177

10.1.2.1.1	Absolute SS-RSRP Accuracy	1177

10.1.2.1.2	Relative SS-RSRP Accuracy	1178

10.1.2.2	Void	1179

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	1179

10.1.2.3.1	Absolute CSI-RSRP Accuracy	1179

10.1.2.3.2	Relative CSI-RSRP Accuracy	1180

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1181

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	1181

10.1.2B.1.1	Absolute SS-RSRP Accuracy	1181

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	1182

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	1182

10.1.2C.1.1	Absolute SS-RSRP Accuracy	1182

10.1.2C.1.2	Relative SS-RSRP Accuracy	1183

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	1183

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	1183

10.1.3.1.1	Absolute SS-RSRP Accuracy	1183

10.1.3.1.2	Relative SS-RSRP Accuracy	1184

10.1.3.2	Void	1185

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	1185

10.1.3.3.1	Absolute CSI-RSRP Accuracy	1185

10.1.3.3.2	Relative CSI-RSRP Accuracy	1186

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1187

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	1187

10.1.3B.1.1	Absolute SS-RSRP Accuracy	1187

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	1188

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	1188

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1188

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	1189

10.1.4.2	Void	1190

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	1190

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	1190

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	1191

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1192

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	1192

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	1192

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	1193

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	1193

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	1193

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	1194

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	1194

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	1194

10.1.5.1.1	Absolute SS-RSRP Accuracy	1194

10.1.5.1.2	Relative SS-RSRP Accuracy	1195

10.1.5.2	Void	1196

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	1196

10.1.5.3.1	Absolute CSI-RSRP Accuracy	1196

10.1.5.3.2	Relative CSI-RSRP Accuracy	1197

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1198

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	1198

10.1.5B.1.1	Absolute SS-RSRP Accuracy	1198

10.1.6	RSRP Measurement Report Mapping	1199

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	1201

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1201

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	1201

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	1202

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	1202

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1203

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1203

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	1203

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	1204

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1204

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	1204

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	1205

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1205

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	1205

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	1206

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	1206

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1206

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1206

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	1207

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	1207

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1207

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	1207

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	1208

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	1209

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	1209

10.1.9.2.2	Relative CSI-RSRQ Accuracy	1210

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1211

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1211

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	1211

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	1212

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1212

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	1212

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	1213

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	1214

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1214

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	1214

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	1214

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	1215

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	1215

10.1.10.2.2	Relative CSI-RSRQ Accuracy	1216

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1217

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1217

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	1217

10.1.11	RSRQ report mapping	1218

10.1.12	Intra-frequency SINR accuracy requirements for FR1	1218

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	1218

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	1218

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	1219

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	1219

10.1.12C	Intra-frequency SINR accuracy requirements for FR1 SAN	1220

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	1220

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	1220

10.1.13	Intra-frequency SINR accuracy requirements for FR2	1221

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	1221

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	1221

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	1222

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	1222

10.1.14	Inter-frequency SINR accuracy requirements for FR1	1222

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	1222

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	1222

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	1223

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	1224

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	1224

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	1225

10.1.14C	Inter-frequency SINR accuracy requirements for FR1 SAN	1226

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	1226

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	1226

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	1227

10.1.15	Inter-frequency SINR accuracy requirements for FR2	1228

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	1228

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	1228

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	1228

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	1229

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	1229

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	1230

10.1.16	SINR report mapping	1231

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	1231

10.1.17	Power Headroom	1232

10.1.17.1	Power Headroom Report	1232

10.1.17.1.1	Power Headroom Report Mapping	1232

10.1.18	PCMAX,c,f	1233

10.1.18.1	Report Mapping	1233

10.1.19	L1-RSRP accuracy requirements for FR1	1233

10.1.19.1	SSB based L1-RSRP accuracy requirements	1233

10.1.19.1.1	Absolute Accuracy	1233

10.1.19.1.2	Relative Accuracy	1234

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	1235

10.1.19.2.1	Absolute Accuracy	1235

10.1.19.2.2	Relative Accuracy	1236

10.1.19C	L1-RSRP accuracy requirements for FR1 SAN	1237

10.1.19C.1	SSB based L1-RSRP accuracy requirements	1237

10.1.19C.1.1	Absolute Accuracy	1237

10.1.19C.1.2	Relative Accuracy	1238

10.1.20	L1-RSRP accuracy requirements for FR2	1238

10.1.20.1	SSB based L1-RSRP accuracy requirements	1238

10.1.20.1.1	Absolute Accuracy	1238

10.1.20.1.2	Relative Accuracy	1239

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	1240

10.1.20.2.1	Absolute Accuracy	1240

10.1.20.2.2	Relative Accuracy	1241

10.1.21	SFTD accuracy requirements	1242

10.1.21.1	SFTD acuracy requirements for NE-DC	1242

10.1.21.2	SFTD acuracy requirements for NR-DC	1244

10.1.21.3	Inter frequency SFTD acuracy requirements	1245

10.1.22	CLI measurement accuracy requirements	1246

10.1.22.1	SRS-RSRP	1246

10.1.22.1.1	SRS-RSRP Accuracy	1246

10.1.22.1.2	SRS-RSRP report mapping	1248

10.1.22.2	CLI-RSSI	1248

10.1.22.2.1	CLI-RSSI Accuracy	1248

10.1.22.2.2	CLI-RSSI report mapping	1249

10.1.23	RSTD Measurements	1250

10.1.23.1	Introduction	1250

10.1.23.2	Measurement Accuracy Requirements	1250

10.1.23.3	Report mapping	1260

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	1260

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	1262

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	1264

10.1.24	PRS-RSRP Measurements	1266

10.1.24.1	Introduction	1266

10.1.24.2	Measurement Accuracy Requirements	1266

10.1.24.2.1	Absolute PRS RSRP accuracy	1266

10.1.24.2.2	Relative PRS RSRP accuracy	1270

10.1.24.3	Report mapping	1274

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	1274

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	1275

10.1.25	UE Rx-Tx Time Difference Measurements	1277

10.1.25.1	Introduction	1277

10.1.25.2	Measurement Accuracy Requirements	1277

10.1.25.3	Report mapping	1294

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1294

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	1296

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1297

10.1.26	FR2 P-MPR report	1299

10.1.26.1	Report mapping	1299

10.1.27	L1-SINR accuracy requirements for FR1	1299

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1299

10.1.27.1.1	Absolute Accuracy	1299

10.1.27.1.2	Relative Accuracy	1300

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1301

10.1.27.2.1	Absolute Accuracy	1301

10.1.27.2.2	Relative Accuracy	1303

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1304

10.1.27.3.1	Absolute Accuracy	1304

10.1.27.3.2	Relative Accuracy	1306

10.1.28	L1-SINR accuracy requirements for FR2	1308

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1315

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1315

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1315

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1315

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1315

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	1315

10.1.30.1.2	Relative Accuracy of SS-RSRQ	1316

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1317

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1317

10.1.31.1.1	Absolute SS-SINR Accuracy	1317

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1317

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1317

10.1.32.1.1	Aboslute Accuracy of SS-SINR	1317

10.1.32.1.2	Relative Accuracy of SS-SINR	1318

10.1.33	L1-RSRP accuracy requirements under CCA	1319

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1319

10.1.33.1.1	Absolute Accuracy	1319

10.1.33.1.2	Relative Accuracy	1319

10.1.34	RSSI measurements under CCA	1320

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1320

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1320

10.1.34.3	RSSI measurement report mapping	1320

10.1.35	Channel occupancy measurements under CCA	1321

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1321

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1321

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1321

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1321

10.1.36.1.1	Absolute SS-RSRP Accuracy	1321

10.1.36.1.2	Relative SS-RSRP Accuracy	1322

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1323

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1323

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1323

10.1.37.1.2	Relative Accuracy of SS-RSRP	1323

10.1.38	PRS-RSRPP Measurements	1324

10.1.38.1	Introduction	1324

10.1.38.2	Measurement Accuracy Requirements	1324

10.1.38.2.1	Absolute PRS RSRPP accuracy	1324

10.1.38.3	Report mapping	1328

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1328

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1329

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1330

10.1.39.1	Void	1330

10.1.39.2	 Measurement Accuracy Requirements for PRS	1330

10.1.39.3	 Measurement Accuracy Requirements for TRS	1334

10.1.40	Void	1337

10.1A	NR measurements for RedCap	1337

10.1A.1	Introduction	1337

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1337

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1337

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1337

10.1A.2.1.2	Relative SS-RSRP Accuracy	1338

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1339

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1339

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1339

10.1A.3.1.2	Relative SS-RSRP Accuracy	1339

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1339

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1339

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1339

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1340

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1341

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1341

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1341

10.1A.5.1.2	Relative SS-RSRP Accuracy	1341

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1341

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1341

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1341

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1342

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1342

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1342

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1342

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1342

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1342

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1343

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1344

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1344

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1344

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1344

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1344

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1344

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1344

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1345

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1345

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1345

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1345

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1345

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1345

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1346

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1347

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1347

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1347

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1347

10.1A.14	 L1-RSRP accuracy requirements for FR1	1347

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1347

10.1A.14.1.1	Absolute Accuracy	1347

10.1A.14.1.2	Relative Accuracy	1348

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1349

10.1A.14.2.1	Absolute Accuracy	1349

10.1A.14.2.2	Relative Accuracy	1350

10.1A.15	 L1-RSRP accuracy requirements for FR2	1351

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1351

10.1A.15.1.1	Absolute Accuracy	1351

10.1A.15.1.2	Relative Accuracy	1351

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1351

10.1A.15.2.1	Absolute Accuracy	1351

10.1A.15.2.2	Relative Accuracy	1351

10.2	E-UTRAN measurements	1352

10.2.1	Introduction	1352

10.2.2	E-UTRAN RSRP measurements	1352

10.2.3	E-UTRAN RSRQ measurements	1352

10.2.4	E-UTRAN RSTD measurements	1352

10.2.5	E-UTRAN RS-SINR measurements	1353

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1353

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1353

10.2A	E-UTRAN measurements for RedCap	1354

10.2A.1	Introduction	1354

10.2A.2	E-UTRAN RSRP measurements	1354

10.2A.3	E-UTRAN RSRQ measurements	1354

10.2A.4	E-UTRAN RS-SINR measurements	1355

10.3	UTRAN FDD Measurements	1355

10.3.1	UTRAN FDD CPICH RSCP	1355

10.3.2	UTRAN FDD CPICH Ec/No	1356

10.4	V2X measurements	1356

10.4.1	Introduction	1356

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1356

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1356

10.4.2.2	PSBCH-RSRP Relative Accuracy	1357

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1358

10.4.3.1	Absolute SL-RSSI Accuracy	1358

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1358

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1358

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1359

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1359

11	Void	1360

12	V2X Requirements	1139

12.1	Introduction	1139

12.2	UE Transmit Timing	1139

12.2.1	Introduction	1139

12.2.2	GNSS as synchronization reference source	1140

12.2.3	NR Cell as synchronization reference source	1140

12.2.4	E-URTAN Cell as synchronization reference source	1140

12.2.5	SyncRef UE as synchronization reference source	1141

12.3	Initiation/Cease of SLSS Transmissions	1141

12.3.1	Introduction	1141

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1141

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1142

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1143

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1143

12.3A	Initiation/Cease of SLSS Transmissions with CCA	1144

12.3A.1	Introduction	1144

12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1144

12.3A.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1144

12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1144

12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1145

12.4	Selection / Reselection of V2X Synchronization Reference Source	1145

12.4A	Selection / Reselection of Sidelink Synchronization Reference Source with CCA	1147

12.5	L1 SL-RSRP measurements	1149

12.5.1	Introduction	1149

12.5.2	SL-RSRP measurements	1149

12.6	Congestion Control measurements	1150

12.7	Interruption	1150

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1150

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1150

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1152

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1153

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1153

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1153

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1154

12.7.8	Interruptions at NR sidelink discovery configuration	1154

12.7.9	Interruptions to WAN due to sidelink carrier addition/release	1154

12.8	Reliability of GNSS signal	1155

12.9	Scheduling availability	1155

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1155

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1156

12.10	Selection / Reselection of relay UE	1156

12.10.1	Introduction	1156

12.10.2	Selection / Reselection of relay UE	1156

12.11	Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation	1157

12.12	Selection / Reselection of Synchronization Reference Source for NR SL Carrier Aggregation	1157

12A	NR Sidelink Measurements for Positioning	1159

12A.1	Introduction	1159

12A.2	SL-RSTD measurements	1159

12A.2.1	Introduction	1159

12A.2.3	Measurement Capability	1159

12A.2.4	Measurement Reporting Requirements	1159

12A.2.5	Measurements Period Requirements	1160

12A.3	SL-RSRP measurements	1161

12A.3.1	Introduction	1161

12A.3.2	Requirements Applicability	1161

12A.3.4	Measurement Reporting Requirements	1161

12A.3.5	Measurements Period Requirements	1161

12A.4	SL-Rx-Tx measurements	1162

12A.4.1	Introduction	1162

12A.4.2	Requirements Applicability	1162

12A.4.3	Measurement Capability	1163

12A.4.4	Measurement Reporting Requirements	1163

12A.4.5	Measurement Period Requirements	1163

12A.5	SL-RSRPP measurements	1164

12A.5.1	Introduction	1164

12A.5.2	Requirements Applicability	1164

12A.5.3	Measurement Capability	1164

12A.5.4	Measurement Reporting Requirements	1164

12A.5.5	Measurement Period Requirements	1165

12A.6	SL-AoA measurements	1165

12A.6.1	Introduction	1165

12A.6.2	Requirements Applicability	1165

12A.6.3	Measurement Capability	1165

12A.6.4	Measurement Reporting Requirements	1165

12A.6.5	Measurement Period Requirements	1166

12A.7	SL-RTOA measurements	1166

12A.7.1	Introduction	1166

12A.7.2	Requirements Applicability	1166

12A.7.3	Measurement Capability	1167

12A.7.4	Measurement Reporting Requirements	1167

12A.7.5	Measurement Period Requirements	1167

13	Measurement Performance Requirements for NR gNB	1168

13.1	UL-RTOA	1168

13.1.1	Report mapping	1168

13.1.1A	Additional Path Report Mapping for UL-RTOA	1170

13.2	gNB Rx-Tx time difference	1172

13.2.1	Report mapping	1172

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	1174

13.2.2	Measurement Accuracy Requirements	1176

13.2.2.1	Introduction	1176

13.3	UL SRS RSRP measurement	1177

13.3.1	Report mapping	1177

13.3.2	Measurement accuracy requirements	1178

13.3.2.1	Introduction	1178

13.3.2.2	Requirements	1178

13.4	AoA/ZoA	1179

13.4.1	Report mapping	1179

13.5	Timing advance (TADV)	1180

13.5.1	Report mapping	1180

13.6	UL SRS RSRPP measurement	1181

13.6.1	Report mapping	1181

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1183

13.7.1	Report mapping	1183

13.7.2	Measurement Accuracy Requirements	1183

13.7.2.1	Introduction	1183

13.7.2.2	Requirements	1183

Annex A (normative): Test Cases	1184

A.1	Purpose of annex	1184

A.2	Requirement classification for statistical testing	1184

A.2.1	Types of requirements in TS 38.133	1184

A.2.1.1	Time and delay requirements on UE higher layer actions	1184

A.2.1.2	Measurements of power levels, relative powers and time	1184

A.2.1.3	Implementation requirements	1185

A.2.1.4	Physical layer timing requirements	1185

A.2.1.5	Requirements under CCA	1185

A.3	RRM test configurations	1186

A.3.1	Reference measurement channels	1186

A.3.1.1	PDSCH	1186

A.3.1.1.1	FDD	1186

A.3.1.1.2	TDD	1187

A.3.1.2	CORESET for RMSI scheduling	1190

A.3.1.2.1	FDD	1190

A.3.1.2.2	TDD	1191

A.3.1.3	CORESET for RMC scheduling	1194

A.3.1.3.1	FDD	1194

A.3.1.3.2	TDD	1195

A.3.1.4	TDD UL/DL configuration	1198

A.3.1A	Reference measurement channels under CCA	1200

A.3.1A.1	PDSCH	1200

A.3.1A.1.1	TDD	1200

A.3.1A.2	CORESET for RMSI scheduling	1201

A.3.1A.2.1	TDD	1201

A.3.1A.3	CORESET for RMC scheduling	1202

A.3.1A.3.1	TDD	1202

A.3.1A.4	TDD UL/DL configuration	1203

A.3.1A.5	RMC burst transmission model	1203

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1203

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1204

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1204

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1205

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1205

A.3.2.2	Void	1206

A.3.3	Reference DRX configurations	1206

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1206

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1207

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1207

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1207

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1208

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1208

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1208

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1209

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1209

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1209

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1210

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1210

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1210

A.3.4	Test Cases with Different Channel Bandwidths	1211

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1211

A.3.4.1.1	Introduction	1211

A.3.4.1.2	Principle of testing	1211

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1211

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1211

A.3.5.1.1	Introduction	1211

A.3.5.1.2	Principle of Testing	1211

A.3.6	Antenna configurations	1211

A.3.6.1	Antenna configurations for FR1	1211

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1211

A.3.6.1.1.1	Introduction	1211

A.3.6.1.1.2	Principle of testing	1212

A.3.6.1.2	Antenna connection for 8 Rx capable UEs	1215

A.3.6.1.2.1	Introduction	1215

A.3.6.1.2.2	Principle of testing	1215

A.3.6.2	Antenna configurations for FR2	1217

A.3.6A	Antenna configurations with unlicensed bands	1217

A.3.6A.1	Antenna configurations for FR1	1217

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1217

A.3.6A.1.1.1	Introduction	1217

A.3.6A.1.1.2	Principle of testing	1217

A.3.7	EN-DC test setup	1219

A.3.7.1	Introduction	1219

A.3.7.2	E-UTRAN Serving Cell Parameters	1219

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1219

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1220

A.3.7A	NR FR1-FR2 test setup	1221

A.3.7B	EN-DC test setup with unlicensed bands	1222

A.3.7B.1	Introduction	1222

A.3.7B.2	E-UTRAN Serving Cell Parameters	1222

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1222

A.3.7C	LTE-FR1/FR2 test setup	1223

A.3.7D	NE-DC test setup	1223

A.3.7D.1	Introduction	1223

A.3.7D.2	E-UTRAN Serving Cell Parameters	1223

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1223

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1223

A.3.8	PRACH configurations	1223

A.3.8.1	Introduction	1223

A.3.8.2	PRACH configurations in FR1	1224

A.3.8.2.1	FR1 PRACH configuration 1	1224

A.3.8.2.2	FR1 PRACH configuration 2	1224

A.3.8.2.3	FR1 PRACH configuration 3	1225

A.3.8.2.4	FR1 PRACH configuration 4	1226

A.3.8.3	PRACH configurations in FR2	1227

A.3.8.3.1	FR2 PRACH configuration 1	1227

A.3.8.3.2	FR2 PRACH configuration 2	1228

A.3.8.3.3	FR2 PRACH configuration 3	1229

A.3.8.3.4	FR2 PRACH configuration 4	1230

A.3.8A	PRACH configurations under CCA	1231

A.3.8A.1	Introduction	1231

A.3.8A.2	PRACH configurations in FR1	1231

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1231

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1232

A.3.9	BWP configurations	1233

A.3.9.1	Introduction	1233

A.3.9.2	Downlink BWP configurations	1234

A.3.9.2.1	Initial BWP	1234

A.3.9.2.2	Dedicated BWP	1234

A.3.9.3	Uplink BWP configurations	1235

A.3.9.3.1	Initial BWP	1235

A.3.9.3.2	Dedicated BWP	1235

A.3.9A	BWP configurations for RedCap	1235

A.3.9A.1	Introduction	1235

A.3.9A.2	Downlink BWP configurations	1236

A.3.9A.2.1	Dedicated BWP	1236

A.3.9A.3	Uplink BWP configurations	1236

A.3.9A.3.1	Dedicated BWP	1236

A.3.10	SSB Configurations	1237

A.3.10.1	SSB Configurations for FR1	1237

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1237

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1239

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1239

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1240

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1240

A.3.10.2	SSB Configurations for FR2	1241

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1241

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1241

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1242

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1242

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1243

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1243

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1244

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1244

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1245

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1245

A.3.10A	SSB Configurations under CCA	1250

A.3.10A.1	SSB Configurations under CCA for FR1	1250

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1250

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1250

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1251

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1251

A.3.10B	SSB Configurations for RedCap	1252

A.3.10B.1	SSB Configurations for FR1	1252

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1252

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1252

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1253

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1253

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1254

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1254

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1255

A.3.10B.2	SSB Configurations for FR2	1255

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1255

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1256

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1256

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1257

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1257

A.3.11	SMTC Configurations	1258

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1258

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1258

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1258

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1258

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1259

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1259

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1259

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1259

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1259

A.3.11A	SMTC Configurations for RedCap	1260

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1260

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1260

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1260

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1260

A.3.12	Test Cases with Different CC Configurations	1260

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1260

A.3.12.1.1	Introduction	1260

A.3.12.1.2	Principle of testing	1261

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1261

A.3.12.2.1	Introduction	1261

A.3.12.2.2	Principle of testing	1261

A.3.13	Test Cases in SA and EN-DC Operations	1261

A.3.13.1	Introduction	1261

A.3.13.2	Principle of Testing	1261

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1262

A.3.13A.1	Introduction	1262

A.3.13A.2	Principle of Testing in EN-DC	1262

A.3.13A.3	Principle of Testing in SA	1262

A.3.13A.4	Principle of Testing in E-UTRA	1263

A.3.13B	Test Cases for EN-DC and NE-DC Operations	1263

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1263

A.3.13B.1.1	Introduction	1263

A.3.13B.1.2	Principle of Testing	1264

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1264

A.3.13B.2.1	Introduction	1264

A.3.13B.2.2	Principle of Testing	1264

A.3.14	CSI-RS configurations	1264

A.3.14.1	FDD	1264

A.3.14.2	TDD	1266

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1272

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1272

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1272

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1272

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1273

A.3.15.3	Setup 3: 2 AoAs	1273

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1273

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1273

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1273

A.3.16	TCI State Configuration	1274

A.3.16.1	Introduction	1274

A.3.16.2	TCI states	1274

A.3.16A	Unified TCI State Configuration	1274

A.3.16A.1	Introduction	1274

A.3.16A.2	DLorJoint TCI states	1275

A.3.16A.3	UL TCI states	1275

A.3.17	Configurations of CSI-RS for tracking	1276

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1276

A.3.17.1.1	FDD	1276

A.3.17.1.2	TDD	1278

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1280

A.3.17.2.1	TDD	1280

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1281

A.3.18.1	Introduction	1281

A.3.18.2	PRACH Power Measurement	1281

A.3.19	Test applicability for DAPS handover	1281

A.3.19.1	Introduction	1281

A.3.19.2	Principle of testing	1281

A.3.20	MsgA configurations	1282

A.3.20.1	Introduction	1282

A.3.20.2	MsgA configurations in FR1	1282

A.3.20.2.1	FR1 MsgA configuration 1	1282

A.3.20.2.2	FR1 MsgA configuration 2	1283

A.3.20.3	MsgA configurations in FR2	1285

A.3.20.3.1	FR2 MsgA configuration 1	1285

A.3.20.3.2	FR2 MsgA configuration 2	1286

A.3.20A	MsgA configurations under CCA	1288

A.3.20A.1	Introduction	1288

A.3.20A.2	MsgA configurations in FR1	1288

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1288

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1289

A.3.21	V2X sidelink communication	1292

A.3.21.1	Introduction	1292

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1292

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1296

A.3.21.4	Reference SL-DRX configurations	1297

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1297

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1297

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1297

A.3.22	CSI-IM configurations	1297

A.3.22.1	FDD	1297

A.3.22.2	TDD	1298

A.3.23	Spatial Relation Configuration	1299

A.3.23.1	Introduction	1299

A.3.23.2	Spatial Relation	1300

A.3.24	SRS configuration	1301

A.3.25	Channel bandwidth (CBW) configurations	1303

A.3.25.1	DL UE specific CBW	1303

A.3.25.2	UL UE specific CBW	1304

A.3.26	CCA model	1304

A.3.26.1	Introduction	1304

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1304

A.3.26.2.1	DL CCA model	1304

A.3.26.2.2	UL CCA model	1306

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1306

A.3.26.3.1	DL CCA model	1306

A.3.26.3.2	UL CCA model	1307

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1307

A.3.27.1	Introduction	1307

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1307

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1308

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1308

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1308

A.3.28	Discovery Burst Transmission Window configuration under CCA	1308

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1308

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1308

A.3.29.1	Introduction	1308

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1308

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1309

A.3.30	CSI-RS configurations for RRM	1310

A.3.30.1	FDD	1310

A.3.30.2	TDD	1311

A.3.31	PRS Configurations	1313

A.3.31.1.	PRS Configurations for FR1	1313

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1313

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1314

A.3.31.2.	PRS Configurations for FR2	1314

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1314

A.3.32	NR sidelink discovery	1314

A.3.32.1	Introduction	1314

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1314

A.3.32.3	Principle of Testing	1315

A.3.33	PRS Processing Window (PPW) configurations	1315

A.3.34	Testing principles for test cases related to PRS measurements	1315

A.3.34.1	Introduction	1315

A.3.34.2	Test cases in RRC_INACTIVE state	1316

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1316

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1316

A.3.35	Testing principle for RedCap UE	1317

A.3.35.1	Introduction	1317

A.3.35.2	Principle of testing for FR1	1317

A.3.35.3	Principle of testing for FR2	1317

A.3.36	Testing related to Satellite access	1317

A.3.36.1	Introduction	1317

A.3.36.1	Introduction	1317

A.3.36.2	Principle of testing GSO and NGSO scenarios	1317

A.3.36.2	Principle of testing different RRM requirements	1318

A.3.36.3	Principle of testing different ephemeris formats	1318

A.3.36.4	General setup for SIB19	1320

A.3.36.5	Satellite specific parameters configuration	1321

A.3.36.5.1	Satellite specific configuration for serving cell	1321

A.3.36.5.2	Satellite specific configuration for neighbour cell	1321

A.4	EN-DC tests with all NR cells in FR1	1321

A.4.1	Void	1321

A.4.2	Void	1321

A.4.3	RRC_CONNECTED state mobility	1321

A.4.3.1	Void	1321

A.4.3.2	RRC Connection Mobility Control	1321

A.4.3.2.1	Void	1321

A.4.3.2.2	Random Access	1321

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1321

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1325

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1329

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1333

A.4.3.2.3	Void	1337

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1337

A.4.3.3.1	Test Purpose and Environment	1337

A.4.3.3.2	Test Requirements	1343

A.4.4	Timing	1344

A.4.4.1	UE transmit timing	1344

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1344

A.4.4.1.1.1	Test Purpose and environment	1344

A.4.4.1.1.2	Test requirements	1348

A.4.4.2	UE timer accuracy	1348

A.4.4.3	Timing advance	1348

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1348

A.4.4.3.1.1	Test Purpose and Environment	1348

A.4.4.3.1.2	Test Parameters	1348

A.4.4.3.1.3	Test Requirements	1352

A.4.5	Signaling characteristics	1352

A.4.5.1	Radio link Monitoring	1352

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1353

A.4.5.1.1.1	Test Purpose and Environment	1353

A.4.5.1.1.2	Test Requirements	1357

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1357

A.4.5.1.2.1	Test Purpose and Environment	1357

A.4.5.1.2.2	Test Requirements	1363

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1363

A.4.5.1.3.1	Test Purpose and Environment	1363

A.4.5.1.3.2	Test Requirements	1368

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1368

A.4.5.1.4.1	Test Purpose and Environment	1368

A.4.5.1.4.2	Test Requirements	1374

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1374

A.4.5.1.5.1	Test Purpose and Environment	1374

A.4.5.1.5.2	Test Requirements	1379

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1379

A.4.5.1.6.1	Test Purpose and Environment	1379

A.4.5.1.6.2	Test Requirements	1385

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1385

A.4.5.1.7.1	Test Purpose and Environment	1385

A.4.5.1.7.2	Test Requirements	1390

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1391

A.4.5.1.8.1	Test Purpose and Environment	1391

A.4.5.1.8.2	Test Requirements	1396

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1396

A.4.5.1.9.1	Test Purpose and Environment	1396

A.4.5.2	Interruption	1402

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1402

A.4.5.2.1.1	Test Purpose and Environment	1402

A.4.5.2.1.2	Test Requirements	1406

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1406

A.4.5.2.2.1	Test Purpose and Environment	1406

A.4.5.2.2.2	Test Requirements	1410

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1410

A.4.5.2.3.1	Test Purpose and Environment	1410

A.4.5.2.3.2	Test Requirements	1415

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1416

A.4.5.2.4.1	Test Purpose and Environment	1416

A.4.5.2.4.2	Test Requirements	1420

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1421

A.4.5.2.5.1	Test Purpose and Environment	1421

A.4.5.2.5.2	Test Requirements	1425

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1425

A.4.5.2.6.1	Test Purpose and Environment	1425

A.4.5.2.6.2	Test Requirements	1429

A.4.5.2.7	Void	1429

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1429

A.4.5.2.8.1	Test Purpose and Environment	1429

A.4.5.2.8.2	Test Requirements	1432

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1433

A.4.5.2.9.1	Test Purpose and Environment	1433

A.4.5.2.9.2	Test Requirements	1437

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1438

A.4.5.2.10.1	Test Purpose and Environment	1438

A.4.5.2.10.2	Test Requirements	1441

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1441

A.4.5.2.11.1	Test Purpose and Environment	1441

A.4.5.2.11.2	Test Requirements	1446

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1447

A.4.5.2.12.1	Test Purpose and Environment	1447

A.4.5.3	SCell Activation and Deactivation Delay	1455

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1455

A.4.5.3.1.1	Test Purpose and Environment	1455

A.4.5.3.1.2	Test Requirements	1461

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1462

A.4.5.3.2.1	Test Purpose and Environment	1462

A.4.5.3.2.2	Test Requirements	1462

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1462

A.4.5.3.3.1	Test Purpose and Environment	1462

A.4.5.3.3.2	Test Requirements	1463

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1464

A.4.5.3.4.1	Test Purpose and Environment	1464

A.4.5.3.3.2	Test Requirements	1466

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1467

A.4.5.3.5.1	Test Purpose and Environment	1467

A.4.5.3.5.2	Test Requirements	1474

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1475

A.4.5.3.6.1	Test Purpose and Environment	1475

A.4.5.3.6.2	Test Requirements	1479

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1480

A.4.5.3.7.1	Test Purpose and Environment	1480

A.4.5.3.7.2	Test Requirements	1480

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1481

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1481

A.4.5.4.1.1 Test Purpose and Environment	1481

A.4.5.4.1.2	Test Requirements	1491

A.4.5.5	Beam Failure Detection and Link recovery procedures	1491

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1491

A.4.5.5.1.1	Test Purpose and Environment	1491

A.4.5.5.1.2	Test Requirements	1497

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1498

A.4.5.5.2.1	Test Purpose and Environment	1498

A.4.5.5.2.2	Test Requirements	1503

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1504

A.4.5.5.3.1	Test Purpose and Environment	1504

A.4.5.5.3.2	Test Requirements	1509

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1510

A.4.5.5.4.1	Test Purpose and Environment	1510

A.4.5.5.4.2	Test Requirements	1515

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1516

A.4.5.5.5.1	Test Purpose and Environment	1516

A.4.5.5.5.2	Test Requirements	1521

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1522

A.4.5.5.6.1	Test Purpose and Environment	1522

A.4.5.5.6.2	Test Requirements	1528

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1529

A.4.5.5.7.1	Test Purpose and Environment	1529

A.4.5.5.7.2	Test Requirements	1534

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1535

A.4.5.5.8.1	Test Purpose and Environment	1535

A.4.5.5.8.2	Test Requirements	1541

A.4.5.6	Active BWP switch	1542

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1542

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1542

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1547

A.4.5.6.2	RRC-based Active BWP Switch	1553

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1557

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1557

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1563

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1563

A.4.5.6.4.1.1	Test Purpose and Environment	1563

A.4.5.6.4.1.2	Test Requirements	1570

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1570

A.4.5.6.4.2.1	Test Purpose and Environment	1570

A.4.5.6.4.2.2	Test Requirements	1580

A.4.5.6.5	SCell dormancy switch	1581

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1581

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1586

A.4.5.6.5.2.1	Test Purpose and Environment	1586

A.4.5.6.5.2.2	Test Requirements	1592

A.4.5.7	PSCell addition and release delay	1592

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1592

A.4.5.7.1.1	Test purpose and environment	1592

A.4.5.7.1.2	Test Requirements	1597

A.4.5.8	DL Interruptions at switching between two uplink carriers	1598

A.4.5.8.1	Test Purpose and Environment	1598

A.4.5.8.2	Test Requirements	1601

A.4.5.9	UE specific CBW change	1602

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1602

A.4.5.9.1.1	Test Purpose and Environment	1602

A.4.5.9.1.2	Test Requirements	1606

A.4.5.10	PSCell activation and deactivation delay	1606

A.4.5.10.1	PSCell activation and deactivation delay	1606

A.4.5.10.1.1	Test purpose and environment	1606

A.4.5.10.1.2	Test Requirements	1610

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1611

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1611

A.4.5.11.1.1	Test purpose and environment	1611

A.4.5.11.1.2	Test Parameters	1611

A.4.5.11.1.3	Test Requirements	1615

A.4.6	Measurement procedure	1615

A.4.6.1	Intra-frequency Measurements	1615

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1615

A.4.6.1.1.1	Test purpose and Environment	1615

A.4.6.1.1.2	Test parameters	1615

A.4.6.1.1.3	Test Requirements	1619

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1619

A.4.6.1.2.1	Test purpose and Environment	1619

A.4.6.1.2.2	Test parameters	1619

A.4.6.1.2.2	Test Requirements	1623

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1623

A.4.6.1.3.1	Test purpose and Environment	1623

A.4.6.1.3.2	Test parameters	1623

A.4.6.1.3.3	Test Requirements	1627

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1627

A.4.6.1.4.1	Test purpose and Environment	1627

A.4.6.1.4.2	Test parameters	1627

A.4.6.1.4.3	Test Requirements	1631

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1631

A.4.6.1.5.1	Test purpose and Environment	1631

A.4.6.1.5.2	Test parameters	1631

A.4.6.1.5.3	Test Requirements	1633

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1634

A.4.6.1.6.1	Test purpose and Environment	1634

A.4.6.1.6.2	Test parameters	1634

A.4.6.1.6.3	Test Requirements	1636

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1637

A.4.6.1.7.1	Test purpose and Environment	1637

A.4.6.1.7.2	Test parameters	1637

A.4.6.1.7.3	Test Requirements	1641

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1641

A.4.6.1.8.1	Test Purpose and Environment	1641

A.4.6.1.8.2	Test Requirements	1646

A.4.6.2	Inter-frequency Measurements	1647

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1647

A.4.6.2.1.1	Test Purpose and Environment	1647

A.4.6.2.1.2	Test Requirements	1651

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1651

A.4.6.2.2.1	Test Purpose and Environment	1651

A.4.6.2.2.2	Test Requirements	1655

A.4.6.2.3	Void	1655

A.4.6.2.4	Void	1655

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1655

A.4.6.2.5.1	Test Purpose and Environment	1655

A.4.6.2.5.2	Test Requirements	1660

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1660

A.4.6.2.6.1	Test Purpose and Environment	1660

A.4.6.2.6.2	Test Requirements	1665

A.4.6.2.7	Void	1665

A.4.6.2.8	Void	1665

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1666

A.4.6.2.9.1	Test Purpose and Environment	1666

A.4.6.2.9.2	Test Requirements	1669

A.4.6.3	Void	1669

A.4.6.4	L1-RSRP measurement for beam reporting	1669

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1669

A.4.6.4.1.1	Test Purpose and Environment	1669

A.4.6.4.1.2	Test parameters	1670

A.4.6.4.1.3	Test Requirements	1673

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1673

A.4.6.4.2.1	Test Purpose and Environment	1673

A.4.6.4.2.2	Test parameters	1674

A.4.6.4.2.3	Test Requirements	1677

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1677

A.4.6.4.3.1	Test Purpose and Environment	1677

A.4.6.4.3.2	Test parameters	1678

A.4.6.4.3.3	Test Requirements	1681

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1681

A.4.6.4.4.1	Test Purpose and Environment	1681

A.4.6.4.4.2	Test parameters	1682

A.4.6.4.4.3	Test Requirements	1684

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1684

A.4.6.4.5.1	Test Purpose and Environment	1684

A.4.6.4.5.2	Test parameters	1685

A.4.6.4.5.3	Test Requirements	1688

A.4.6.5	CLI measurements	1688

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1688

A.4.6.5.1.1	Test Purpose and Environment	1688

A.4.6.5.1.2	Test Parameters	1689

A.4.6.5.1.3	Test Requirements	1692

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1692

A.4.6.5.2.1	Test Purpose and Environment	1692

A.4.6.5.2.2	Test Parameters	1693

A.4.6.5.2.3	Test Requirements	1694

A.4.6.6.1.2	Test Requirements	1699

A.4.6.7	L1-SINR measurement for beam reporting	1699

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1702

A.4.6.7.2.1	Test Purpose and Environment	1702

A.4.6.7.2.2	Test parameters	1702

A.4.6.7.2.3	Test Requirements	1705

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1705

A.4.6.7.3.1	Test Purpose and Environment	1705

A.4.6.7.3.2	Test parameters	1705

A.4.6.7.3.3	Test Requirements	1707

A.4.6.8	CSI-RS based intra-frequency Measurement	1708

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1708

A.4.6.8.1.1	Test purpose and Environment	1708

A.4.6.8.1.2	Test Requirements	1712

A.4.6.9	CSI-RS based inter-frequency Measurement	1712

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1712

A.4.6.9.1.1	Test Purpose and Environment	1712

A.4.6.9.1.2	Test Requirements	1716

A.4.7	Measurement Performance requirements	1718

A.4.7.1	SS-RSRP	1718

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1718

A.4.7.1.1.1	Test Purpose and Environment	1718

A.4.7.1.1.2	Test parameters	1718

A.4.7.1.1.3	Test Requirements	1722

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1722

A.4.7.1.2.1	Test Purpose and Environment	1722

A.4.7.1.2.2	Test parameters	1723

A.4.7.1.2.3	Test Requirements	1727

A.4.7.1.3	Void	1728

A.4.7.2	SS-RSRQ	1728

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1728

A.4.7.2.1.1	Test Purpose and Environment	1728

A.4.7.2.1.2	Test Parameters	1728

A.4.7.2.1.3	Test Requirements	1733

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1733

A.4.7.2.2.1	Test Purpose and Environment	1733

A.4.7.2.2.2	Test Parameters	1733

A.4.7.2.2.3	Test Requirements	1739

A.4.7.3	SS-SINR	1739

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1739

A.4.7.3.1.1	Test Purpose and Environment	1739

A.4.7.3.1.2	Test Parameters	1739

A.4.7.3.1.3	Test Requirements	1743

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1744

A.4.7.3.2.1	Test Purpose and Environment	1744

A.4.7.3.2.2	Test Parameters	1744

A.4.7.3.2.3	Test Requirements	1748

A.4.7.4	L1-RSRP measurement for beam reporting	1749

A.4.7.4.1	SSB based L1-RSRP measurement	1749

A.4.7.4.1.1	Test Purpose and Environment	1749

A.4.7.4.1.2	Test parameters	1749

A.4.7.4.1.3	Test Requirements	1752

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1752

A.4.7.4.2.1	Test Purpose and Environment	1752

A.4.7.4.2.2	Test parameters	1753

A.4.7.4.2.3	Test Requirements	1756

A.4.7.5	SFTD accuracy	1757

A.4.7.5.1	SFTD accuracy	1757

A.4.7.5.1.1	Test Purpose and Environment	1757

A.4.7.5.1.2	Test Parameters	1757

A.4.7.5.1.3	Test Requirements	1761

A.4.7.5.2	Void	1761

A.4.7.5.3	Void	1761

A.4.7.6	CLI measurements	1761

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1761

A.4.7.6.1.1	Test Purpose and Environment	1761

A.4.7.6.1.2	Test parameters	1762

A.4.7.6.1.3	Test Requirements	1767

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1767

A.4.7.6.2.1	Test Purpose and Environment	1767

A.4.7.6.2.2	Test parameters	1768

A.4.7.6.2.3	Test Requirements	1771

A.4.7.7	L1-SINR measurement for beam reporting	1771

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1775

A.4.7.7.2.1	Test Purpose and Environment	1775

A.4.7.7.2.2	Test parameters	1775

A.4.7.7.2.3	Test Requirements	1779

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1779

A.4.7.7.3.1	Test Purpose and Environment	1779

A.4.7.7.3.2	Test parameters	1779

A.4.7.7.3.3	Test Requirements	1782

A.4.7.8	CSI-RSRP	1783

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1783

A.4.7.8.1.1	Test Purpose and Environment	1783

A.4.7.8.1.2	Test parameters	1783

A.4.7.8.1.3	Test Requirements	1788

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1788

A.4.7.8.2.1	Test Purpose and Environment	1788

A.4.7.8.2.2	Test parameters	1788

A.4.7.8.2.3	Test Requirements	1793

A.4.7.9	CSI-RSRQ	1793

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1793

A.4.7.9.1.1	Test Purpose and Environment	1793

A.4.7.9.1.2	Test Parameters	1793

A.4.7.9.1.3	Test Requirements	1799

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1799

A.4.7.9.2.1	Test Purpose and Environment	1799

A.4.7.9.2.2	Test Parameters	1799

A.4.7.9.2.3	Test Requirements	1804

A.4.7.10	CSI-SINR	1805

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1805

A.4.7.10.1.1	Test Purpose and Environment	1805

A.4.7.10.1.2	Test Parameters	1805

A.4.7.10.1.3	Test Requirements	1809

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1810

A.4.7.10.2.1	Test Purpose and Environment	1810

A.4.7.10.2.2	Test Parameters	1810

A.4.7.10.2.3	Test Requirements	1815

A.4.8	Void	1815

A.4A	NE-DC test with all NR cells in FR1	1815

A.4A.1	Signaling characteristics	1815

A.4A.1.1	E-UTRAN PSCell addition	1815

A.4A.1.1.1	Test purpose and environment	1815

A.4A.1.1.2	Test Requirements	1819

A.4A.1.2	Active BWP switch	1820

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1820

A.4A.1.2.1.1	Test Purpose and Environment	1820

A.4A.1.2.1.2	Test Requirements	1824

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1825

A.4A.1.3.1	Test purpose and environment	1825

A.4A.1.3.2	Test Requirements	1829

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1830

A.4A.1.4.1	Test Purpose and Environment	1830

A.4A.1.4.2	Test Parameters	1830

A.4A.1.4.3	Test Requirements	1835

A.4A.1.4.3.1	Test Requirements for NR HO	1835

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1835

A.4A.2	Measurement performance	1836

A.4A.2.1	SFTD accuracy	1836

A.4A.2.1.1	SFTD accuracy	1836

A.4A.2.1.1.1	Test Purpose	1836

A.4A.2.1.1.2	Test Environment	1836

A.4A.2.1.1.3	Test Requirements	1840

A.5	EN-DC tests with one or more NR cells in FR2	1840

A.5.1	Void	1840

A.5.2	Void	1840

A.5.3	RRC_CONNECTED state mobility	1840

A.5.3.1	Void	1840

A.5.3.2	RRC Connection Mobility Control	1840

A.5.3.2.1	Void	1840

A.5.3.2.2	Random Access	1840

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1840

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1844

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1850

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1853

A.5.3.2.3	Void	1857

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1857

A.5.3.3.1	Test purpose and environment	1857

A.5.3.3.2	Test Requirements	1861

A.5.3.3.3	Void	1862

A.5.3.3.4	Void	1862

A.5.3.3.5	Void	1862

A.5.3.3.6	Void	1862

A.5.4	Timing	1862

A.5.4.1	UE transmit timing	1862

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1862

A.5.4.1.1.1	Test Purpose and environment	1862

A.5.4.1.1.2	Test requirements	1865

A.5.4.2	UE timer accuracy	1866

A.5.4.3	Timing advance	1866

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1866

A.5.4.3.1.1 Test Purpose and Environment	1866

A.5.4.3.1.2 Test Parameters	1866

A.5.4.3.1.3	Test Requirements	1870

A.5.5	Signaling characteristics	1870

A.5.5.1	Radio link Monitoring	1870

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1870

A.5.5.1.1.1	Test Purpose and Environment	1870

A.5.5.1.1.2	Test Requirements	1874

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1875

A.5.5.1.2.1	Test Purpose and Environment	1875

A.5.5.1.2.2	Test Requirements	1878

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1879

A.5.5.1.3.1	Test Purpose and Environment	1879

A.5.5.1.3.2	Test Requirements	1883

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1883

A.5.5.1.4.1	Test Purpose and Environment	1883

A.5.5.1.4.2	Test Requirements	1887

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1887

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1891

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1895

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1900

A.5.5.1.8.2	Test Requirements	1904

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1905

A.5.5.1.9.1	Test Purpose and Environment	1905

A.5.5.1.9.2	Test Requirements	1907

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1907

A.5.5.1.10.1	Test Purpose and Environment	1907

A.5.5.1.10.2	Test Requirements	1910

A.5.5.2	Interruption	1910

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1910

A.5.5.2.1.1	Test Purpose and Environment	1910

A.5.5.2.1.2	Test Requirements	1913

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1913

A.5.5.2.2.1	Test Purpose and Environment	1913

A.5.5.2.2.2	Test Requirements	1916

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1916

A.5.5.2.3.1	Test Purpose and Environment	1916

A.5.5.2.3.2	Test Requirements	1920

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1920

A.5.5.2.4.1	Test Purpose and Environment	1920

A.5.5.2.4.2	Test Requirements	1923

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1924

A.5.5.2.5.1	Test Purpose and Environment	1924

A.5.5.2.5.2	Test Requirements	1927

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1927

A.5.5.2.6.1	Test Purpose and Environment	1927

A.5.5.2.6.2	Test Requirements	1930

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1930

A.5.5.2.7.1	Test Purpose and Environment	1930

A.5.5.2.7.2	Test Requirements	1934

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1934

A.5.5.2.8.1 Test Purpose and Environment	1934

A.5.5.2.8.3	Test Requirements	1936

A.5.5.2.9	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell	1936

A.5.5.2.9.1	Test Purpose and Environment	1936

A.5.5.2.9.2	Test Requirements	1941

A.5.5.3	SCell Activation and Deactivation Delay	1941

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1941

A.5.5.3.1.1	Test Purpose and Environment	1941

A.5.5.3.1.2	Test Requirements	1943

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1943

A.5.5.3.2.1	Test Purpose and Environment	1943

A.5.5.3.2.2	Test Requirements	1947

A.5.5.3.3	Void	1947

A.5.5.3.4	Void	1947

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1947

A.5.5.3.5.1	Test Purpose and Environment	1947

A.5.5.3.5.2	Test Requirements	1950

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1951

A.5.5.3.6.1	Test Purpose and Environment	1951

A.5.5.3.6.2	Test Requirements	1955

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1956

A.5.5.3.7.1	Test Purpose and Environment	1956

A.5.5.3.7.2	Test Requirements	1959

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1959

A.5.5.3.8.1	Test Purpose and Environment	1959

A.5.5.3.8.2	Test Requirements	1963

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	1964

A.5.5.3.9.1	Test Purpose and Environment	1964

A.5.5.3.9.2	Test Requirements	1968

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1969

A.5.5.3.10.1	Test Purpose and Environment	1969

A.5.5.3.10.2	Test Requirements	1972

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1973

A.5.5.3.11.1	Test Purpose and Environment	1973

A.5.5.3.11.2	Test Requirements	1977

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1978

A.5.5.3.12.1	Test Purpose and Environment	1978

A.5.5.3.12.2	Test Requirements	1981

A.5.5.4	Void	1982

A.5.5.5	Beam Failure Detection and Link recovery procedures	1982

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1982

A.5.5.5.1.1	Test Purpose and Environment	1982

A.5.5.5.1.2	Test Requirements	1986

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1987

A.5.5.5.2.1	Test Purpose and Environment	1987

A.5.5.5.2.2	Test Requirements	1991

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1992

A.5.5.5.3.1	Test Purpose and Environment	1992

A.5.5.5.3.2	Test Requirements	1996

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1997

A.5.5.5.4.1	Test Purpose and Environment	1997

A.5.5.5.4.2	Test Requirements	2001

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	2002

A.5.5.5.5.1	Test Purpose and Environment	2002

A.5.5.5.5.2	Test Requirements	2006

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	2007

A.5.5.5.6.1	Test Purpose and Environment	2007

A.5.5.5.6.2	Test Requirements	2011

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	2012

A.5.5.5.7.1	Test Purpose and Environment	2012

A.5.5.5.7.2	Test Requirements	2016

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2017

A.5.5.5.8.1	Test Purpose and Environment	2017

A.5.5.5.8.2	Test Requirements	2022

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	2022

A.5.5.5.9.1	Test Purpose and Environment	2022

A.5.5.5.9.2	Test Requirements	2026

A.5.5.6	Active BWP switch	2026

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	2026

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2026

A.5.5.6.1.1.1	Test Purpose and Environment	2026

A.5.5.6.1.1.2	Test Requirements	2030

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	2030

A.5.5.6.2	RRC-based Active BWP Switch	2035

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2035

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2039

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	2039

A.5.5.6.4	SCell dormancy switch	2043

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	2043

A.5.5.6.4.1.1	Test Purpose and Environment	2043

A.5.5.6.4.1.2	Test Requirements	2047

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	2047

A.5.5.6.4.2.1	Test Purpose and Environment	2047

A.5.5.6.4.2.2	Test Requirements	2054

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2054

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	2054

A.5.5.7	PSCell addition and release delay	2057

A.5.5.7.1	Addition and Release Delay of NR PSCell	2057

A.5.5.7.1.1	Test purpose and environment	2057

A.5.5.7.1.2	Test Requirements	2060

A.5.5.8	Active TCI state switch delay	2061

A.5.5.8.1	MAC-CE based active TCI state switch	2061

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2061

A.5.5.8.1.1.1	Test Purpose and Environment	2061

A.5.5.8.1.1.2	Test Requirements	2065

A.5.5.8.2	RRC based active TCI state switch	2065

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2065

A.5.5.8.2.1.1	Test Purpose and Environment	2065

A.5.5.8.2.1.2	Test Requirements	2069

A.5.5.9	Uplink spatial relation switch delay	2069

A.5.5.9.1	MAC-CE based uplink spatial relation switch	2069

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	2069

A.5.5.9.1.1.1	Test Purpose and Environment	2069

A.5.5.9.1.1.2	Test Requirements	2072

A.5.5.9.2	RRC based spatial relation switch	2072

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	2072

A.5.5.9.2.1.1	Test Purpose and Environment	2072

A.5.5.9.2.1.2	Test Requirements	2075

A.5.5.10	UE specific CBW change	2075

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	2075

A.5.5.10.1.1	Test Purpose and Environment	2075

A.5.5.10.1.2	Test Requirements	2078

A.5.5.11	Unified TCI state switch delay	2079

A.5.5.11.1	MAC-CE based active joint TCI state switch	2079

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	2079

A.5.5.11.1.1.1	Test Purpose and Environment	2079

A.5.5.11.1.1.2	Test parameters	2079

A.5.5.11.1.1.3	Test Requirements	2082

A.5.5.11.2	MAC-CE based active uplink TCI state switch	2082

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	2082

A.5.5.11.2.1.1	Test Purpose and Environment	2082

A.5.5.11.2.1.2	Test parameters	2083

A.5.5.11.2.1.3	Test Requirements	2085

A.5.5.11.3	MAC-CE based active downlink TCI state switch	2085

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	2085

A.5.5.11.3.1.1	Test Purpose and Environment	2085

A.5.5.11.3.1.2	Test Parameters	2086

A.5.5.11.3.1.3	Test Requirements	2090

A.5.5.12	PSCell activation and deactivation delay	2090

A.5.5.12.1	PSCell activation and deactivation delay	2090

A.5.5.12.1.1	Test purpose and environment	2090

A.5.5.12.1.2	Test Requirements	2093

A.5.5.13	Conditional PSCell addition and release delay	2094

A.5.5.13.1	Addition and Release Delay of NR PSCell	2094

A.5.5.13.1.1	Test purpose and environment	2094

A.5.5.13.1.2	Test Requirements	2097

A.5.6	Measurement procedure	2098

A.5.6.1	Intra-frequency Measurements	2098

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	2098

A.5.6.1.1.1	Test purpose and Environment	2098

A.5.6.1.1.2	Test Requirements	2101

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	2101

A.5.6.1.2.1	Test purpose and Environment	2101

A.5.6.1.2.2	Test Requirements	2103

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	2104

A.5.6.1.3.1	Test purpose and Environment	2104

A.5.6.1.3.2	Test Requirements	2108

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	2108

A.5.6.1.4.1	Test purpose and Environment	2108

A.5.6.1.4.2	Test Requirements	2111

A.5.6.2	Inter-frequency Measurements	2112

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2112

A.5.6.2.1.1	Test Purpose and Environment	2112

A.5.6.2.1.2	Test Requirements	2115

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2115

A.5.6.2.2.1	Test Purpose and Environment	2115

A.5.6.2.2.2	Test Requirements	2119

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2119

A.5.6.2.3.1	Test Purpose and Environment	2119

A.5.6.2.3.2	Test Requirements	2123

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2123

A.5.6.2.4.1	Test Purpose and Environment	2123

A.5.6.2.4.2	Test Requirements	2127

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2127

A.5.6.2.5.1	Test Purpose and Environment	2127

A.5.6.2.5.2	Test Requirements	2132

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2132

A.5.6.2.6.1	Test Purpose and Environment	2132

A.5.6.2.6.2	Test Requirements	2136

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2136

A.5.6.2.7.1	Test Purpose and Environment	2136

A.5.6.2.7.2	Test Requirements	2141

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2141

A.5.6.2.8.1	Test Purpose and Environment	2141

A.5.6.2.8.2	Test Requirements	2146

A.5.6.3	L1-RSRP measurement for beam reporting	2147

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2147

A.5.6.3.1.1	Test Purpose and Environment	2147

A.5.6.3.1.2	Test parameters	2147

A.5.6.3.1.3	Test Requirements	2149

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2149

A.5.6.3.2.1	Test Purpose and Environment	2149

A.5.6.3.2.2	Test parameters	2150

A.5.6.3.2.3	Test Requirements	2152

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2152

A.5.6.3.3.1	Test Purpose and Environment	2152

A.5.6.3.3.2	Test parameters	2153

A.5.6.3.3.3	Test Requirements	2155

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2155

A.5.6.3.4.1	Test Purpose and Environment	2155

A.5.6.3.4.2	Test parameters	2156

A.5.6.3.4.3	Test Requirements	2158

A.5.6.4	CLI measurements	2159

A.5.6.4.1	SRS-RSRP measurement with DRX	2159

A.5.6.4.1.1	Test Purpose and Environment	2159

A.5.6.4.1.2	Test Parameters	2159

A.5.6.4.1.3	Test Requirements	2161

A.5.6.4.2	CLI-RSSI measurement with DRX	2161

A.5.6.4.2.1	Test Purpose and Environment	2161

A.5.6.4.2.2	Test Parameters	2162

A.5.6.4.2.3	Test Requirements	2164

A.5.6.5	Measurements with autonomous gaps	2164

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2164

A.5.6.5.1.1	Test Purpose and Environment	2164

A.5.6.5.1.2	Test Requirements	2167

A.5.6.6	L1-SINR measurement for beam reporting	2168

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2170

A.5.6.6.2.1	Test Purpose and Environment	2170

A.5.6.6.2.2	Test parameters	2171

A.5.6.6.2.3	Test Requirements	2174

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2174

A.5.6.6.3.1	Test Purpose and Environment	2174

A.5.6.6.3.2	Test parameters	2175

A.5.6.6.3.3	Test Requirements	2177

A.5.6.7	CSI-RS based Intra-frequency Measurements	2177

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2177

A.5.6.7.1.1	Test purpose and Environment	2177

A.5.6.7.1.2	Test Requirements	2180

A.5.6.8	CSI-RS based Inter-frequency Measurements	2180

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2180

A.5.6.8.1.1	Test Purpose and Environment	2180

A.5.6.8.1.2	Test Requirements	2185

A.5.7	Measurement Performance requirements	2185

A.5.7.1	SS-RSRP	2185

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2185

A.5.7.1.1.1	Test Purpose and Environment	2185

A.5.7.1.1.2	Test parameters	2186

A.5.7.1.1.3	Test Requirements	2188

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2189

A.5.7.1.2.1	Test Purpose and Environment	2189

A.5.7.1.2.2	Test parameters	2189

A.5.7.1.2.3	Test Requirements	2193

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2194

A.5.7.1.3.1	Test Purpose and Environment	2194

A.5.7.1.3.2	Test parameters	2194

A.5.7.1.3.3	Test Requirements	2197

A.5.7.2	SS-RSRQ	2198

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2198

A.5.7.2.1.1	Test Purpose and Environment	2198

A.5.7.2.1.2	Test Parameters	2198

A.5.7.2.1.3	Test Requirements	2201

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2201

A.5.7.2.2.1	Test Purpose and Environment	2201

A.5.7.2.2.2	Test Parameters	2201

A.5.7.2.2.3	Test Requirements	2203

A.5.7.3	SS-SINR	2204

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2204

A.5.7.3.1.1	Test Purpose and Environment	2204

A.5.7.3.1.2	Test Parameters	2204

A.5.7.3.1.3	Test Requirements	2207

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2207

A.5.7.3.2.1	Test Purpose and Environment	2207

A.5.7.3.2.2	Test Parameters	2207

A.5.7.3.2.3	Test Requirements	2209

A.5.7.4	L1-RSRP measurement for beam reporting	2209

A.5.7.4.1	SSB based L1-RSRP measurement	2209

A.5.7.4.1.1	Test Purpose and Environment	2209

A.5.7.4.1.2	Test parameters	2210

A.5.7.4.1.3	Test Requirements	2212

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2213

A.5.7.4.2.1	Test Purpose and Environment	2213

A.5.7.4.2.2	Test parameters	2213

A.5.7.4.2.3	Test Requirements	2215

A.5.7.5	CLI measurements	2216

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2216

A.5.7.5.1.1	Test Purpose and Environment	2216

A.5.7.5.1.2	Test parameters	2216

A.5.7.5.1.3	Test Requirements	2219

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2220

A.5.7.5.2.1	Test Purpose and Environment	2220

A.5.7.5.2.2	Test parameters	2220

A.5.7.5.2.3	Test Requirements	2222

A.5.7.6	L1-SINR measurement for beam reporting	2223

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2226

A.5.7.6.2.1	Test Purpose and Environment	2226

A.5.7.6.2.2	Test parameters	2226

A.5.7.6.2.3	Test Requirements	2228

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2229

A.5.7.6.3.1	Test Purpose and Environment	2229

A.5.7.6.3.2	Test parameters	2229

A.5.7.6.3.3	Test Requirements	2231

A.5.7.7	CSI-RSRP	2232

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2232

A.5.7.7.1.1	Test Purpose and Environment	2232

A.5.7.7.1.2	Test parameters	2232

A.5.7.7.1.3	Test Requirements	2234

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2235

A.5.7.7.2.1	Test Purpose and Environment	2235

A.5.7.7.2.2	Test parameters	2235

A.5.7.7.2.3	Test Requirements	2239

A.5.7.8	CSI-RSRQ	2240

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2240

A.5.7.8.1.1	Test Purpose and Environment	2240

A.5.7.8.1.2	Test Parameters	2240

A.5.7.8.1.3	Test Requirements	2242

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2242

A.5.7.8.2.1	Test Purpose and Environment	2242

A.5.7.8.2.2	Test Parameters	2242

A.5.7.8.2.3	Test Requirements	2244

A.5.7.9	CSI-SINR	2244

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2244

A.5.7.9.1.1	Test Purpose and Environment	2244

A.5.7.9.1.2	Test Parameters	2244

A.5.7.9.1.3	Test Requirements	2247

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2247

A.5.7.9.2.1	Test Purpose and Environment	2247

A.5.7.9.2.2	Test Parameters	2247

A.5.7.9.2.3	Test Requirements	2250

A.5.8	Void	2250

A.6	NR standalone tests with all NR cells in FR1	2244

A.6.1	SA: RRC_IDLE state mobility	2244

A.6.1.1	Cell re-selection to NR	2244

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2244

A.6.1.1.1.1	Test Purpose and Environment	2244

A.6.1.1.1.2	Test Parameters	2244

A.6.1.1.1.3	Test Requirements	2248

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2248

A.6.1.1.2.1	Test Purpose and Environment	2248

A.6.1.1.2.2	Test Parameters	2248

A.6.1.1.2.3	Test Requirements	2252

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2252

A.6.1.1.3.1	Test Purpose and Environment	2252

A.6.1.1.3.2	Test Parameters	2252

A.6.1.1.3.3	Test Requirements	2257

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2257

A.6.1.1.4.1	Test Purpose and Environment	2257

A.6.1.1.4.2	Test Parameters	2257

A.6.1.1.4.3	Test Requirements	2260

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2260

A.6.1.1.5.1	Test Purpose and Environment	2260

A.6.1.1.5.2	Test Parameters	2260

A.6.1.1.5.3	Test Requirements	2265

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2265

A.6.1.1.6.1	Test Purpose and Environment	2265

A.6.1.1.6.2	Test Parameters	2266

A.6.1.1.6.3	Test Requirements	2270

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2270

A.6.1.1.7.1	Test Purpose and Environment	2270

A.6.1.1.7.2	Test Parameters	2271

A.6.1.1.7.3	Test Requirements	2274

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2274

A.6.1.1.8.1	Test Purpose and Environment	2274

A.6.1.1.8.2	Test Parameters	2274

A.6.1.1.8.3	Test Requirements	2278

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2278

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2278

A.6.1.2.1.1	Test Purpose and Environment	2278

A.6.1.2.1.2	Test Parameters	2278

A.6.1.2.1.3	Test Requirements	2281

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2282

A.6.1.2.2.1	Test Purpose and Environment	2282

A.6.1.2.2.2	Test Parameters	2282

A.6.1.2.2.3	Test Requirements	2285

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2286

A.6.1.2.3.1	Test Purpose and Environment	2286

A.6.1.2.3.2	Test Parameters	2286

A.6.1.2.3.3	Test Requirements	2289

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2290

A.6.1.2.4.1	Test Purpose and Environment	2290

A.6.1.2.4.2	Test Parameters	2290

A.6.1.2.4.3	Test Requirements	2293

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2294

A.6.1.2.5.1	Test Purpose and Environment	2294

A.6.1.2.5.2	Test Parameters	2294

A.6.1.2.5.3	Test Requirements	2298

A.6.1.1.7	Void	2299

A.6.2	SA: RRC_INACTIVE state mobility	2299

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2299

A.6.2.1.1	Test purpose and Environment	2299

A.6.2.1.2	Test Parameters	2300

A.6.2.1.3	Test requirements	2302

A.6.3	RRC_CONNECTED state mobility	2302

A.6.3.1	Handover	2302

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2302

A.6.3.1.1.1	Test Purpose and Environment	2302

A.6.3.1.1.2	Test Parameters	2302

A.6.3.1.1.3 Test Requirements	2306

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2306

A.6.3.1.2.1	Test Purpose and Environment	2306

A.6.3.1.2.2	Test Parameters	2306

A.6.3.1.2.3	Test Requirements	2310

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2310

A.6.3.1.3.1	Test Purpose and Environment	2310

A.6.3.1.3.2	Test Parameters	2310

A.6.3.1.3.3	Test Requirements	2314

A.6.3.1.4	SA NR - E-UTRAN handover	2314

A.6.3.1.4.1	Test Purpose and Environment	2314

A.6.3.1.4.2	Test Requirements	2320

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2320

A.6.3.1.5.1	Test Purpose and Environment	2320

A.6.3.1.5.2	Test Requirements	2326

A.6.3.1.6	 SA NR - UTRAN FDD handover	2326

A.6.3.1.6.1	Test Purpose and Environment	2326

A.6.3.1.6.2	Test Requirements	2330

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2330

A.6.3.1.7.1	Test Purpose and Environment	2330

A.6.3.1.7.2	Test Parameters	2330

A.6.3.1.7.3	Test Requirements	2334

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2335

A.6.3.1.8.1	Test Purpose and Environment	2335

A.6.3.1.8.2	Test Parameters	2335

A.6.3.1.8.3	Test Requirements	2339

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2340

A.6.3.1.9.1	Test Purpose and Environment	2340

A.6.3.1.9.2	Test Parameters	2340

A.6.3.1.9.3	Test Requirements	2344

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2344

A.6.3.1.10.1	Test Purpose and Environment	2344

A.6.3.1.10.2	Test Parameters	2344

A.6.3.1.10.3	Test Requirements	2347

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2347

A.6.3.1.11.1	Test Purpose and Environment	2347

A.6.3.1.11.2	Test Parameters	2347

A.6.3.1.11.3 Test Requirements	2354

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2354

A.6.3.1.12.1	Test Purpose and Environment	2354

A.6.3.1.12.2	Test Parameters	2354

A.6.3.1.12.3 Test Requirements	2362

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2362

A.6.3.1.13.1	Test Purpose and Environment	2362

A.6.3.1.13.2	Test Requirements	2371

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2371

A.6.3.1.14.1	Test Purpose and Environment	2371

A.6.3.1.14.2	Test Requirements	2380

A.6.3.2	RRC Connection Mobility Control	2381

A.6.3.2.1	SA: RRC Re-establishment	2381

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2381

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2385

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2389

A.6.3.2.2	Random Access	2392

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2392

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2396

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2401

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2406

A.6.3.2.3	SA: RRC Connection Release with Redirection	2410

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2410

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2414

A.6.3.3 Conditional handover	2421

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2421

A.6.3.3.1.1	Test Purpose and Environment	2421

A.6.3.3.1.2	Test Parameters	2421

A.6.3.3.1.3 Test Requirements	2425

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2425

A.6.3.3.2.1	Test Purpose and Environment	2425

A.6.3.3.2.2	Test Parameters	2425

A.6.3.3.2.3 Test Requirements	2429

A.6.4	Timing	2429

A.6.4.1	UE transmit timing	2429

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2429

A.6.4.1.1.1	Test Purpose and environment	2429

A.6.4.1.1.2	Test requirements	2433

A.6.4.2	UE timer accuracy	2433

A.6.4.3	Timing advance	2433

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2433

A.6.4.3.1.1	Test Purpose and Environment	2433

A.6.4.3.1.2	Test Parameters	2433

A.6.4.3.1.3	Test Requirements	2437

A.6.5	Signalling characteristics	2437

A.6.5.1	Radio link Monitoring	2437

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2438

A.6.5.1.1.1	Test Purpose and Environment	2438

A.6.5.1.1.2	Test Requirements	2443

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2443

A.6.5.1.2.1	Test Purpose and Environment	2443

A.6.5.1.2.2	Test Requirements	2449

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2449

A.6.5.1.3.1	Test Purpose and Environment	2449

A.6.5.1.3.2	Test Requirements	2455

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2455

A.6.5.1.4.1	Test Purpose and Environment	2455

A.6.5.1.4.2	Test Requirements	2461

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2461

A.6.5.1.5.1	Test Purpose and Environment	2461

A.6.5.1.5.2	Test Requirements	2467

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2467

A.6.5.1.6.1	Test Purpose and Environment	2467

A.6.5.1.6.2	Test Requirements	2472

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2472

A.6.5.1.7.1	Test Purpose and Environment	2472

A.6.5.1.7.2	Test Requirements	2476

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2476

A.6.5.1.8.1	Test Purpose and Environment	2476

A.6.5.1.8.2	Test Requirements	2482

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2482

A.6.5.1.9.1	Test Purpose and Environment	2482

A.6.5.1.9.2	Test Requirements	2488

A.6.5.2	Interruption	2488

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2488

A.6.5.2.1.2	Test Requirements	2492

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2493

A.6.5.2.2.1	Test Purpose and Environment	2493

A.6.5.2.2.2	Test Parameters	2493

A.6.5.2.2.3	Test Requirements	2495

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2496

A.6.5.2.3.1	Test Purpose and Environment	2496

A.6.5.2.3.2	Test Parameters	2496

A.6.5.2.3.3	Test Requirements	2498

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2499

A.6.5.2.4.1	Test Purpose and Environment	2499

A.6.5.2.4.2	Test Parameters	2499

A.6.5.2.4.3	Test Requirements	2501

A.6.5.3	SCell Activation and Deactivation Delay	2502

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2502

A.6.5.3.1.1	Test Purpose and Environment	2502

A.6.5.3.1.2	Test Requirements	2507

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2508

A.6.5.3.2.1	Test Purpose and Environment	2508

A.6.5.3.2.2	Test Requirements	2508

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2508

A.6.5.3.3.1	Test Purpose and Environment	2508

A.6.5.3.3.2	Test Requirements	2509

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2509

A.6.5.3.4.1	Test Purpose and Environment	2509

A.6.5.3.4.2	Test Requirements	2517

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2517

A.6.5.3.5.1	Test Purpose and Environment	2517

A.6.5.3.5.2	Test Requirements	2523

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2524

A.6.5.3.6.1	Test Purpose and Environment	2524

A.6.5.3.6.2	Test Requirements	2528

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2528

A.6.5.3.7.1	Test Purpose and Environment	2528

A.6.5.3.7.2	Test Requirements	2532

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2533

A.6.5.3.8.1	Test Purpose and Environment	2533

A.6.5.3.8.2	Test Requirements	2537

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2538

A.6.5.3.9.1	Test Purpose and Environment	2538

A.6.5.3.9.2	Test Requirements	2542

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2543

A.6.5.3.10.1	Test Purpose and Environment	2543

A.6.5.3.10.2	Test Requirements	2546

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2546

A.6.5.3.11.1	Test Purpose and Environment	2546

A.6.5.3.11.2	Test Requirements	2547

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2547

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2547

A.6.5.4.1.1	Test Purpose and Environment	2547

A.6.5.4.1.2	Test Requirements	2557

A.6.5.4.2	Void	2558

A.6.5.5	Beam Failure Detection and Link recovery procedures	2558

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2558

A.6.5.5.1.1	Test Purpose and Environment	2558

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2564

A.6.5.5.2.1	Test Purpose and Environment	2564

A.6.5.5.2.2	Test Requirements	2570

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2571

A.6.5.5.3.1	Test Purpose and Environment	2571

A.6.5.5.3.2	Test Requirements	2576

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2577

A.6.5.5.4.1	Test Purpose and Environment	2577

A.6.5.5.4.2	Test Requirements	2582

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2583

A.6.5.5.5.1	Test Purpose and Environment	2583

A.6.5.5.5.2	Test Requirements	2587

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2587

A.6.5.5.6.1	Test Purpose and Environment	2587

A.6.5.5.6.2	Test Requirements	2592

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2592

A.6.5.5.7.1	Test Purpose and Environment	2592

A.6.5.5.7.2	Test Requirements	2598

A.6.5.6	Active BWP switch	2599

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2599

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2599

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2605

A.6.5.6.2	RRC-based Active BWP Switch	2610

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2610

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2614

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2614

A.6.5.6.4	SCell dormancy switch	2621

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2621

A.6.5.6.4.1.1	Test Purpose and Environment	2621

A.6.5.6.4.1.2	Test Requirements	2627

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2627

A.6.5.6.4.2.1	 Test Purpose and Environment	2627

A.6.5.6.4.2.2	Test Requirements	2634

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2634

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2634

A.6.5.7	DL interruptions at switching between two uplink carriers	2639

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2639

A.6.5.7.1.1	Test Purpose and Environment	2639

A.6.5.7.1.2	Test Requirements	2643

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2643

A.6.5.7.2.1	Test Purpose and Environment	2643

A.6.5.7.2.2	Test Requirements	2647

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2647

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2647

A.6.5.7A.1.1	Test Purpose and Environment	2647

A.6.5.7A.1.2	Test Requirements	2651

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2651

A.6.5.7A.2.1	Test Purpose and Environment	2651

A.6.5.7A.2.2	Test Requirements	2655

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2655

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2655

A.6.5.7B.1.1	Test Purpose and Environment	2655

A.6.5.7B.1.2	Test Requirements	2659

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2659

A.6.5.7B.2.1	Test Purpose and Environment	2659

A.6.5.7B.2.2	Test Requirements	2663

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2663

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2663

A.6.5.7C.1.1	Test Purpose and Environment	2663

A.6.5.7C.1.2	Test Requirements	2667

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2668

A.6.5.7C.2.1	Test Purpose and Environment	2668

A.6.5.7C.2.2	Test Requirements	2672

A.6.5.7D	DL interruptions at UE switching across three or four uplink bands	2673

A.6.5.7D.1	DL interruptions at switching across three uplink bands in TDD-TDD CA	2673

A.6.5.7D.1.1	Test Purpose and Environment	2673

A.6.5.7D.1.2	Test Requirements	2677

A.6.5.7D.2	DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG	2678

A.6.5.7D.2.1	Test Purpose and Environment	2678

A.6.5.7D.2.2	Test Requirements	2682

A.6.5.7D.3	DL interruptions at Tx switching across three uplink bands in FDD-TDD CA in SA for 2 TAGs	2682

A.6.5.7D.3.1	Test Purpose and Environment	2682

A.6.5.7D.3.2	Test Requirements	2686

A.6.5.7D.4	DL interruptions at Tx switching across four uplink bands in TDD-TDD CA with different UL/DL pattern in SA for two TAGs	2687

A.6.5.7D.4.1	Test Purpose and Environment	2687

A.6.5.7D.7.2	Test Requirements	2692

A.6.5.8	UE specific CBW change	2692

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2692

A.6.5.8.1.1	Test Purpose and Environment	2692

A.6.5.8.1.2	Test Requirements	2696

A.6.5.9	Pathloss reference signal switching delay	2696

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2696

A.6.5.9.1.1	Test Purpose and Environment	2696

A.6.5.9.1.2	Test Requirements	2699

A.6.6	Measurement procedure	2683

A.6.6.1	Intra-frequency Measurements	2683

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2683

A.6.6.1.1.1	Test purpose and Environment	2683

A.6.6.1.1.2	Test parameters	2683

A.6.6.1.1.3	Test Requirements	2686

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2686

A.6.6.1.2.1	Test purpose and Environment	2686

A.6.6.1.2.2	Test parameters	2686

A.6.6.1.2.3	Test Requirements	2690

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2690

A.6.6.1.3.1	Test purpose and Environment	2690

A.6.6.1.3.2	Test parameters	2690

A.6.6.1.3.3	Test Requirements	2694

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2694

A.6.6.1.4.1	Test purpose and Environment	2694

A.6.6.1.4.2	Test parameters	2694

A.6.6.1.4.3	Test Requirements	2698

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2698

A.6.6.1.5.1	Test purpose and Environment	2698

A.6.6.1.5.2	Test parameters	2698

A.6.6.1.5.3	Test Requirements	2700

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2701

A.6.6.1.6.1	Test purpose and Environment	2701

A.6.6.1.6.2	Test parameters	2701

A.6.6.1.6.3	Test Requirements	2702

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2703

A.6.6.1.7.1	Test purpose and Environment	2703

A.6.6.1.7.2	Test parameters	2703

A.6.6.1.7.3	Test Requirements	2707

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2707

A.6.6.1.8.1	Test purpose and Environment	2707

A.6.6.1.8.2	Test parameters	2707

A.6.6.1.8.3	Test Requirements	2711

A.6.6.2	Inter-frequency Measurements	2711

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2711

A.6.6.2.1.1	Test Purpose and Environment	2711

A.6.6.2.1.2	Test Requirements	2715

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2715

A.6.6.2.2.1	Test Purpose and Environment	2715

A.6.6.2.2.2	Test Requirements	2719

A.6.6.2.3	Void	2720

A.6.6.2.4	Void	2720

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2720

A.6.6.2.5.1	Test Purpose and Environment	2720

A.6.6.2.5.2	Test Requirements	2724

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2724

A.6.6.2.6.1	Test Purpose and Environment	2724

A.6.6.2.6.2	Test Requirements	2728

A.6.6.2.7	Void	2728

A.6.6.2.8	Void	2728

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2728

A.6.6.2.9.1	Test Purpose and Environment	2728

A.6.6.2.9.2	Test Requirements	2732

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2732

A.6.6.2.10.1	Test Purpose and Environment	2732

A.6.6.2.10.2	Test Requirements	2736

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2740

A.6.6.2.12.1	Test Purpose and Environment	2740

A.6.6.2.12.2	Test Requirements	2744

A.6.6.3	Inter-RAT Measurements	2744

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2744

A.6.6.3.1.1	Test Purpose and Environment	2744

A.6.6.3.1.2	Test Requirements	2750

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2750

A.6.6.3.2.1	Test Purpose and Environment	2750

A.6.6.3.2.2	Test Requirements	2757

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2757

A.6.6.3.3.1	Test Purpose and Environment	2757

A.6.6.3.3.2	Test Requirements	2762

A.6.6.4	L1-RSRP measurement for beam reporting	2762

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2762

A.6.6.4.1.1	Test Purpose and Environment	2762

A.6.6.4.1.2	Test parameters	2763

A.6.6.4.1.3	Test Requirements	2766

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2766

A.6.6.4.2.1	Test Purpose and Environment	2766

A.6.6.4.2.2	Test parameters	2767

A.6.6.4.2.3	Test Requirements	2770

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2770

A.6.6.4.3.1	Test Purpose and Environment	2770

A.6.6.4.3.2	Test parameters	2771

A.6.6.4.3.3	Test Requirements	2774

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2774

A.6.6.4.4.1	Test Purpose and Environment	2774

A.6.6.4.4.2	Test parameters	2775

A.6.6.4.4.3	Test Requirements	2778

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2778

A.6.6.4.5.1	Test Purpose and Environment	2778

A.6.6.4.5.2	Test parameters	2779

A.6.6.4.5.3	Test Requirements	2782

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2782

A.6.6.4.6.1	Test Purpose and Environment	2782

A.6.6.4.6.2	Test parameters	2783

A.6.6.4.6.3	Test Requirements	2786

A.6.6.5	Inter-RAT UTRAN FDD measurements	2787

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2787

A.6.6.5.1.1	Test Purpose and Environment	2787

A.6.6.5.1.2	Test Requirements	2790

A.6.6.6	CLI measurements	2790

A.6.6.6.1	SRS-RSRP measurement with DRX	2790

A.6.6.6.1.1	Test Purpose and Environment	2790

A.6.6.6.1.2	Test Parameters	2791

A.6.6.6.1.3	Test Requirements	2794

A.6.6.6.2	CLI-RSSI measurement with DRX	2794

A.6.6.6.2.1	Test Purpose and Environment	2794

A.6.6.6.2.2	Test Parameters	2795

A.6.6.6.2.3	Test Requirements	2797

A.6.6.7	NR measurements with autonomous gaps	2797

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2797

A.6.6.7.1.1	Test Purpose and Environment	2797

A.6.6.7.1.2	Test Parameters	2797

A.6.6.7.1.3	Test Requirements	2800

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2800

A.6.6.7.2.1	Test Purpose and Environment	2800

A.6.6.7.2.2	Test Requirements	2803

A.6.6.8	L1-SINR measurement for beam reporting	2804

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	2804

A.6.6.8.1.1	Test Purpose and Environment	2804

A.6.6.8.1.2	Test parameters	2804

A.6.6.8.1.3	Test Requirements	2806

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2806

A.6.6.8.2.1	Test Purpose and Environment	2806

A.6.6.8.2.2	Test parameters	2806

A.6.6.8.2.3	Test Requirements	2811

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2811

A.6.6.8.3.1	Test Purpose and Environment	2811

A.6.6.8.3.2	Test parameters	2812

A.6.6.8.3.3	Test Requirements	2814

A.6.6.9	Idle Mode CA/DC Measurements	2814

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2814

A.6.6.9.1.1	Test Purpose and Environment	2814

A.6.6.9.1.2	Test Requirements	2821

A.6.6.10	CSI-RS based intra-frequency Measurements	2821

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2821

A.6.6.10.1.1	Test purpose and Environment	2821

A.6.6.10.1.2	Test Requirements	2824

A.6.6.11	CSI-RS based inter-frequency Measurements	2824

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2824

A.6.6.11.1.1	Test Purpose and Environment	2824

A.6.6.11.1.2	Test Requirements	2828

A.6.6.12	RSTD measurements	2828

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2828

A.6.6.12.1.1	Test Purpose and Environment	2828

A.6.6.12.1.2	Test Requirements	2836

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2836

A.6.6.12.2.1	Test Purpose and Environment	2836

A.6.6.12.2.2	Test Requirements	2844

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2844

A.6.6.12.3.1	Test Purpose and Environment	2844

A.6.6.12.3.2	Test Requirements	2850

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2851

A.6.6.12.4.1	Test Purpose and Environment	2851

A.6.6.12.4.2	Test Requirements	2855

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2855

A.6.6.12.5.1	Test Purpose and Environment	2855

A.6.6.12.5.2	Test Requirements	2859

A.6.6.13 PRS-RSRP measurements	2860

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2860

A.6.6.13.1.1	Test purpose and Environment	2860

A.6.6.13.1.2	Test Requirements	2864

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2864

A.6.6.13.2.1	Test purpose and Environment	2864

A.6.6.13.2.2	Test Requirements	2868

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2868

A.6.6.13.3.1	Test purpose and Environment	2868

A.6.6.13.3.2	Test Requirements	2872

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2872

A.6.6.13.4.1	Test purpose and Environment	2872

A.6.6.14	UE Rx-Tx time difference measurements	2876

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2876

A.6.6.14.1.1	Test purpose and environment	2876

A.6.6.14.1.2	Test requirements	2880

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2880

A.6.6.14.2.1	Test purpose and environment	2880

A.6.6.14.2.2	Test requirements	2884

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2884

A.6.6.14.3.1	Test purpose and environment	2884

A.6.6.14.3.2	Test requirements	2888

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2888

A.6.6.14.4.1	Test purpose and environment	2888

A.6.6.14.4.2	Test requirements	2892

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2892

A.6.6.14.4.1	Test purpose and environment	2892

A.6.6.14.4.2	Test requirements	2896

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2896

A.6.6.15.1	Test Purpose and Environment	2896

A.6.6.15.2	Test Requirements	2903

A.6.6.16	PRS-RSRPP measurements	2904

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2904

A.6.6.16.1.1	Test purpose and Environment	2904

A.6.6.16.1.2	Test Requirements	2906

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2906

A.6.6.16.2.1	Test purpose and Environment	2906

A.6.6.16.2.2	Test Requirements	2909

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2909

A.6.6.16.3.1	Test purpose and Environment	2909

A.6.6.16.3.2	Test Requirements	2913

A.6.6.17	SA event triggered reporting tests with Pre-MG	2913

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2913

A.6.6.17.1.1	Test purpose and Environment	2913

A.6.6.17.1.2	Test parameters	2913

A.6.6.17.1.3	Test Requirements	2918

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2918

A.6.6.17.2.1	Test purpose and Environment	2918

A.6.6.17.2.2	Test parameters	2918

A.6.6.17.2.3	Test Requirements	2923

A.6.6.17.3	Void	2923

A.6.6.17.3.1	Void	2923

A.6.6.17.3.2	Void	2923

A.6.6.17.3.3	Void	2923

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2923

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2923

A.6.6.18.1.1	Test Purpose and Environment	2923

A.6.6.18.1.2	Test Requirements	2927

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2927

A.6.6.18.2.1	Test Purpose and Environment	2927

A.6.6.18.2.2	Test Requirements	2931

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2931

A.6.6.18.3.1	Test Purpose and Environment	2931

A.6.6.18.3.2	Test Requirements	2938

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2938

A.6.6.18.4.1	Test Purpose and Environment	2938

A.6.6.18.4.2	Test Requirements	2942

A.6.6.19	SA event triggered reporting tests with NCSG	2943

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2943

A.6.6.19.1.1	Test purpose and Environment	2943

A.6.6.19.1.2	Test parameters	2943

A.6.6.19.1.3	Test Requirements	2947

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2947

A.6.6.19.2.1	Test Purpose and Environment	2947

A.6.6.19.2.2	Test parameters	2947

A.6.6.19.2.3	Test Requirements	2951

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2951

A.6.6.19.3.1	Test Purpose and Environment	2951

A.6.6.19.3.2	Test parameters	2952

A.6.6.19.3.3	Test Requirements	2957

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2957

A.6.6.19.4.1	Test purpose and Environment	2957

A.6.6.19.4.2	Test parameters	2957

A.6.6.19.4.3	Test Requirements	2960

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2960

A.6.6.20.1	Test purpose and environment	2960

A.6.6.20.2	Test requirements	2964

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	2964

A.6.6.21.1	Test purpose and environment	2964

A.6.6.21.2	Test requirements	2966

A.6.7	Measurement Performance requirements	2967

A.6.7.1	SS-RSRP	2967

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2967

A.6.7.1.1.1	Test Purpose and Environment	2967

A.6.7.1.1.2	Test parameters	2967

A.6.7.1.1.3	Test Requirements	2972

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	2972

A.6.7.1.2.1	Test Purpose and Environment	2972

A.6.7.1.2.2	Test parameters	2972

A.6.7.1.2.3	Test Requirements	2976

A.6.7.1.3	Void	2977

A.6.7.2	SS-RSRQ	2977

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2977

A.6.7.2.1.1	Test Purpose and Environment	2977

A.6.7.2.1.2	Test Parameters	2977

A.6.7.2.1.3	Test Requirements	2982

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2982

A.6.7.2.2.1	Test Purpose and Environment	2982

A.6.7.2.2.2	Test Parameters	2982

A.6.7.2.2.3	Test Requirements	2987

A.6.7.3	SS-SINR	2987

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2987

A.6.7.3.1.1	Test Purpose and Environment	2987

A.6.7.3.1.2	Test Parameters	2988

A.6.7.3.1.3	Test Requirements	2992

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2993

A.6.7.3.2.1	Test Purpose and Environment	2993

A.6.7.3.2.2	Test Parameters	2993

A.6.7.3.2.3	Test Requirements	2998

A.6.7.4	L1-RSRP measurement for beam reporting	2998

A.6.7.4.1	SSB based L1-RSRP measurement	2998

A.6.7.4.1.1	Test Purpose and Environment	2998

A.6.7.4.1.2	Test parameters	2998

A.6.7.4.1.3	Test Requirements	3002

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3002

A.6.7.4.2.1	Test Purpose and Environment	3002

A.6.7.4.2.2	Test parameters	3003

A.6.7.4.2.3	Test Requirements	3007

A.6.7.5	E-UTRAN RSRP	3008

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3008

A.6.7.5.1.1	Test Purpose and Environment	3008

A.6.7.5.1.2	Test parameters	3008

A.6.7.5.1.3	Test Requirements	3014

A.6.7.6	E-UTRAN RSRQ	3015

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3015

A.6.7.6.1.1	Test Purpose and Environment	3015

A.6.7.6.1.2	Test parameters	3015

A.6.7.6.1.3	Test Requirements	3020

A.6.7.7	E-UTRAN RS-SINR	3021

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3021

A.6.7.7.1.1	Test Purpose and Environment	3021

A.6.7.7.1.2	Test parameters	3021

A.6.7.7.1.3	Test Requirements	3027

A.6.7.8	CLI measurements	3028

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	3028

A.6.7.8.1.1	Test Purpose and Environment	3028

A.6.7.8.1.2	Test parameters	3028

A.6.7.8.1.3	Test Requirements	3034

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	3034

A.6.7.8.2.1	Test Purpose and Environment	3034

A.6.7.8.2.2	Test parameters	3035

A.6.7.8.2.3	Test Requirements	3038

A.6.7.9	L1-SINR measurement for beam reporting	3039

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3044

A.6.7.9.2.1	Test Purpose and Environment	3044

A.6.7.9.2.2	Test parameters	3044

A.6.7.9.2.3	Test Requirements	3049

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3049

A.6.7.9.3.1	Test Purpose and Environment	3049

A.6.7.9.3.2	Test parameters	3050

A.6.7.9.3.3	Test Requirements	3054

A.6.7.10	CSI-RSRP	3055

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3055

A.6.7.9.10.1	Test Purpose and Environment	3055

A.6.7.9.10.2	Test parameters	3055

A.6.7.10.1.3	Test Requirements	3060

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3060

A.6.7.9.10.1	Test Purpose and Environment	3060

A.6.7.10.2.2	Test parameters	3061

A.6.7.10.2.3	Test Requirements	3066

A.6.7.11	CSI-RSRQ	3066

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3066

A.6.7.11.1.1	Test Purpose and Environment	3066

A.6.7.11.1.2	Test Parameters	3066

A.6.7.11.1.3	Test Requirements	3071

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3071

A.6.7.11.2.1	Test Purpose and Environment	3071

A.6.7.11.2.2	Test Parameters	3071

A.6.7.11.2.3	Test Requirements	3077

A.6.7.12	CSI-SINR	3078

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3078

A.6.7.12.1.1	Test Purpose and Environment	3078

A.6.7.12.1.2	Test Parameters	3078

A.6.7.12.1.3	Test Requirements	3083

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3083

A.6.7.12.2.1	Test Purpose and Environment	3083

A.6.7.12.2.2	Test Parameters	3083

A.6.7.12.2.3	Test Requirements	3089

A.6.7.13	RSTD measurements	3089

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	3089

A.6.7.13.1.1	Test purpose and Environment	3089

A.6.7.13.1.2	Test Requirements	3092

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	3092

A.6.7.13.2.1	Test purpose and Environment	3092

A.6.7.13.2.2	Test Requirements	3096

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3096

A.6.7.13.3.1	Test purpose and Environment	3096

A.6.7.13.3.2	Test Requirements	3098

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	3098

A.6.7.14	PRS-RSRP measurements	3102

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	3102

A.6.7.14.1.1	Test Purpose and Environment	3102

A.6.7.14.1.2	Test parameters	3103

A.6.7.14.1.3	Test Requirements	3107

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	3107

A.6.7.14.2.1	Test Purpose and Environment	3107

A.6.7.14.2.2	Test parameters	3107

A.6.7.14.2.3	Test Requirements	3109

A.6.7.14.3	Void	3109

A.6.7.14.3.1	Void	3109

A.6.7.14.3.2	Void	3109

A.6.7.14.3.3	Void	3109

A.6.7.15	UE Rx-Tx time difference measurements	3109

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	3109

A.6.7.15.1.1	Test purpose and environment	3109

A.6.7.15.1.2	Test parameters	3110

A.6.7.15.1.3	Test requirements	3113

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3114

A.6.7.15.2.1	Test purpose and environment	3114

A.6.7.15.2.2	Test parameters	3114

A.6.7.15.2.3	Test requirements	3117

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3117

A.6.7.15.3.1	Test purpose and environment	3117

A.6.7.15.3.2	Test parameters	3118

A.6.7.15.3.3	Test requirements	3121

A.6.7.16	PRS-RSRPP measurements	3121

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3121

A.6.7.16.1.1	Test Purpose and Environment	3121

A.6.7.16.1.2	Test parameters	3122

A.6.7.16.1.3	Test Requirements	3125

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3126

A.6.7.16.2.1	Test Purpose and Environment	3126

A.6.7.16.2.2	Test parameters	3126

A.6.8	Measurement procedure in RRC_INACTIVE	3130

A.6.8.1	RSTD measurements	3130

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3130

A.6.8.1.1.1	Test Purpose and Environment	3130

A.6.8.1.1.2	Test Requirements	3133

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3134

A.6.8.1.2.1	Test Purpose and Environment	3134

A.6.8.1.2.1	Test Purpose and Environment	3134

A.6.8.1.2.2	Test Requirements	3138

A.6.8.2	PRS-RSRP measurements	3138

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3138

A.6.8.2.1.1	Test purpose and Environment	3138

A.6.8.2.1.2	Test Requirements	3140

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3141

A.6.8.2.2.1	Test purpose and Environment	3141

A.6.8.2.2.2	Test Requirements	3144

A.6.8.3	UE Rx-Tx time difference measurements	3144

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3144

A.6.8.3.1.1	Test purpose and environment	3144

A.6.8.3.1.2	Test requirements	3148

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3148

A.6.8.3.2.1	Test purpose and environment	3148

A.6.8.3.2.2	Test requirements	3151

A.6.8.4	PRS-RSRPP measurements	3151

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3151

A.6.8.4.1.1	Test purpose and Environment	3151

A.6.8.4.1.2	Test Requirements	3154

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3154

A.6.8.4.2.1	Test purpose and Environment	3154

A.6.8.4.2.2	Test Requirements	3156

A.6.9	Measurement performance requirements in RRC_INACTIVE	3156

A.6.9.1	RSTD measurements	3156

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3156

A.6.9.1.1.1	Test purpose and Environment	3156

A.6.9.1.1.2	Test Requirements	3158

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3159

A.6.9.1.2.1	Test purpose and Environment	3159

A.6.9.1.2.2	Test Requirements	3161

A.6.9.2	PRS-RSRP measurements	3161

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3161

A.6.9.2.1.1	Test Purpose and Environment	3161

A.6.9.2.1.2	Test parameters	3161

A.6.9.2.1.3	Test Requirements	3163

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3164

A.6.9.2.2.1	Test Purpose and Environment	3164

A.6.9.2.2.2	Test parameters	3164

A.6.9.2.2.3	Test Requirements	3167

A.6.9.3	UE Rx-Tx time difference measurements	3167

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3167

A.6.9.3.1.1.1	Test purpose and environment	3167

A.6.9.3.1.1.2 Test parameters	3167

A.6.9.3.1.1.3	Test requirements	3170

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3170

A.6.9.3.2.1	Test purpose and environment	3170

A.6.9.3.2.2	Test parameters	3170

A.6.9.3.2.3	Test requirements	3173

A.6.9.4	PRS-RSRPP measurements	3173

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3173

A.6.9.4.1.1	Test Purpose and Environment	3173

A.6.9.4.1.2	Test parameters	3173

A.6.9.4.1.3	Test Requirements	3176

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3177

A.6.9.4.2.1	Test Purpose and Environment	3177

A.6.9.4.2.2	Test parameters	3177

A.6.9.4.2.3	Test Requirements	3179

A.7	NR standalone tests with one or more NR cells in FR2	3185

A.7.1	SA: RRC_IDLE state mobility	3185

A.7.1.1	Cell re-selection to NR	3185

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3185

A.7.1.1.1.1	Test Purpose and Environment	3185

A.7.1.1.1.2	Test Parameters	3185

A.7.1.1.1.3	Test Requirements	3188

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3188

A.7.1.1.2.1	Test Purpose and Environment	3188

A.7.1.1.2.2	Test Parameters	3189

A.7.1.1.2.3	Test Requirements	3191

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3191

A.7.1.1.3.1	Test Purpose and Environment	3191

A.7.1.1.3.2	Test Parameters	3191

A.7.1.1.3.3	Test Requirements	3194

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3194

A.7.1.1.4.1	Test Purpose and Environment	3194

A.7.1.1.4.2	Test Parameters	3194

A.7.1.1.4.3	Test Requirements	3197

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3197

A.7.1.1.5.1	Test Purpose and Environment	3197

A.7.1.1.5.2	Test Parameters	3197

A.7.1.1.5.3	Test Requirements	3200

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3200

A.7.1.1.6.1	Test Purpose and Environment	3200

A.7.1.1.6.2	Test Parameters	3200

A.7.1.1.6.3	Test Requirements	3203

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3203

A.7.1.1.7.1	Test Purpose and Environment	3203

A.7.1.1.7.2	Test Parameters	3203

A.7.1.1.7.3	Test Requirements	3207

A.7.2	SA: RRC_INACTIVE state mobility	3207

A.7.2.1	Small Data Transmission	3207

A.7.2.1.1	TA validation for CG-SDT in FR2	3207

A.7.2.1.1.1	Test Purpose and Environment	3207

A.7.2.1.1.2	Test Requirements	3211

A.7.3	RRC_CONNECTED state mobility	3211

A.7.3.1	Handover	3211

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3211

A.7.3.1.1.1	Test Purpose and Environment	3211

A.7.3.1.1.2	Test Parameters	3211

A.7.3.1.1.3	Test Requirements	3215

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3215

A.7.3.1.2.1	Test Purpose and Environment	3215

A.7.3.1.2.2	Test Parameters	3215

A.7.3.1.2.3	Test Requirements	3218

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3218

A.7.3.1.3.1	Test Purpose and Environment	3218

A.7.3.1.3.2	Test Parameters	3218

A.7.3.1.3.3	Test Requirements	3219

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3220

A.7.3.1.4.1	Test Purpose and Environment	3220

A.7.3.1.4.2	Test Parameters	3220

A.7.3.1.4.3 Test Requirements	3227

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3227

A.7.3.1.5.1	Test Purpose and Environment	3227

A.7.3.1.5.2	Test Parameters	3227

A.7.3.1.5.3 Test Requirements	3234

A.7.3.1.6	Handover with PSCell from SA to EN-DC with unknown FR2 target PScell	3234

A.7.3.1.6.1	Test Purpose and Environment	3234

A.7.3.1.6.2	Test Parameters	3234

A.7.3.1.6.3	Test Requirements	3242

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3243

A.7.3.1.7.1	Test purpose and environment	3243

A.7.3.1.7.2	Test Requirements	3248

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3249

A.7.3.1.8.1	Test Purpose and Environment	3249

A.7.3.1.8.2	Test Requirements	3254

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3254

A.7.3.1.9.1	Test Purpose and Environment	3254

A.7.3.1.9.2	Test Parameters	3254

A.7.3.1.9.3	Test Requirements	3257

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3257

A.7.3.1.10.1	Test Purpose and Environment	3257

A.7.3.1.10.2	Test Parameters	3257

A.7.3.1.10.3	Test Requirements	3261

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3261

A.7.3.1.11.1	Test Purpose and Environment	3261

A.7.3.1.11.2	Test Parameters	3261

A.7.3.1.11.3	Test Requirements	3265

A.7.3.2	RRC Connection Mobility Control	3265

A.7.3.2.1	SA: RRC Re-establishment	3265

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3265

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3268

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3271

A.7.3.2.1.3.1	Test Purpose and Environment	3271

A.7.3.2.1.3.2	Test Requirements	3273

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3274

A.7.3.2.1.4.1	Test Purpose and Environment	3274

A.7.3.2.1.4.2	Test Requirements	3276

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3277

A.7.3.2.1.5.1	Test Purpose and Environment	3277

A.7.3.2.1.5.2	Test Requirements	3280

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3280

A.7.3.2.1.6.1	Test Purpose and Environment	3280

A.7.3.2.1.6.2	Test Requirements	3282

A.7.3.2.2	Random Access	3283

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3283

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3287

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3291

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3294

A.7.3.2.3	SA: RRC Connection Release with Redirection	3297

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3297

A.7.3.3 Conditional Handover	3301

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3301

A.7.3.3.1.1	Test Purpose and Environment	3301

A.7.3.3.1.2	Test Parameters	3301

A.7.3.3.1.2.3	Test Requirements	3304

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3304

A.7.3.3.2.1	Test Purpose and Environment	3304

A.7.3.3.2.2	Test Parameters	3304

A.7.3.3.2.3 Test Requirements	3306

A.7.4	Timing	3306

A.7.4.1	UE transmit timing	3306

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3306

A.7.4.1.1.1	Test Purpose and environment	3306

A.7.4.1.1.2	Test requirements	3309

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3310

A.7.4.1.2.1	Test Purpose and environment	3310

A.7.4.1.2.2	Test requirements	3314

A.7.4.2	UE timer accuracy	3315

A.7.4.3	Timing advance	3315

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3315

A.7.4.3.1.1	Test Purpose and Environment	3315

A.7.4.3.1.2	Test Parameters	3315

A.7.4.3.1.3 Test Requirements	3318

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3319

A.7.4.3.2.1	Test Purpose and Environment	3319

A.7.4.3.2.2	Test Parameters	3319

A.7.4.3.2.3	Test Requirements	3323

A.7.5	Signaling characteristics	3323

A.7.5.1	Radio link Monitoring	3323

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3323

A.7.5.1.1.1	Test Purpose and Environment	3323

A.7.5.1.1.2	Test Requirements	3326

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3327

A.7.5.1.2.1	Test Purpose and Environment	3327

A.7.5.1.2.2	Test Requirements	3331

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3332

A.7.5.1.3.1	Test Purpose and Environment	3332

A.7.5.1.3.2	Test Requirements	3336

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3336

A.7.5.1.4.1	Test Purpose and Environment	3336

A.7.5.1.4.2	Test Requirements	3341

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3341

A.7.5.1.5.1	Test Purpose and Environment	3341

A.7.5.1.5.2	Test Requirements	3346

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3346

A.7.5.1.6.1	Test Purpose and Environment	3346

A.7.5.1.6.2	Test Requirements	3350

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3350

A.7.5.1.7.1	Test Purpose and Environment	3350

A.7.5.1.7.2	Test Requirements	3354

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3354

A.7.5.1.8.1	Test Purpose and Environment	3354

A.7.5.1.8.2	Test Requirements	3359

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3359

A.7.5.1.9.1	Test Purpose and Environment	3359

A.7.5.1.9.2	Test Requirements	3362

A.7.5.2	Interruption	3362

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3362

A.7.5.2.1.1	Test Purpose and Environment	3362

A.7.5.2.1.2	Test Requirements	3365

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3366

A.7.5.2.2.1	Test Purpose and Environment	3366

A.7.5.2.2.2	Test Parameters	3366

A.7.5.2.2.3	Test Requirements	3368

A.7.5.3	SCell Activation and Deactivation Delay	3368

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3368

A.7.5.3.1.1	Test Purpose and Environment	3368

A.7.5.3.1.2	Test Requirements	3370

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3370

A.7.5.3.2.1	Test Purpose and Environment	3370

A.7.5.3.2.2	Test Requirements	3374

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3375

A.7.5.3.3.1	Test Purpose and Environment	3375

A.7.5.3.3.2	Test Requirements	3378

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3379

A.7.5.3.4.1	Test Purpose and Environment	3379

A.7.5.3.4.2	Test Requirements	3382

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3383

A.7.5.3.5.1	Test Purpose and Environment	3383

A.7.5.3.5.2	Test Requirements	3386

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3387

A.7.5.3.6.1	Test Purpose and Environment	3387

A.7.5.3.6.2	Test Requirements	3391

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3392

A.7.5.3.7.1	Test Purpose and Environment	3392

A.7.5.3.7.2	Test Requirements	3396

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3397

A.7.5.3.8.1	Test Purpose and Environment	3397

A.7.5.3.8.2	Test Requirements	3401

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3402

A.7.5.3.9.1	Test Purpose and Environment	3402

A.7.5.3.9.2	Test Requirements	3405

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3406

A.7.5.3.10.1	Test Purpose and Environment	3406

A.7.5.3.10.2	Test Requirements	3410

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3411

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3411

A.7.5.3.11.1.1	Test Purpose and Environment	3411

A.7.5.3.11.1.2	Test Requirements	3415

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3416

A.7.5.3.11.2.1	Test Purpose and Environment	3416

A.7.5.3.11.2.2	Test Requirements	3420

A.7.5.3.12	Void	3421

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3421

A.7.5.3.13.1	Test Purpose and Environment	3421

A.7.5.3.13.2	Test Requirements	3423

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3424

A.7.5.3.14.1	Test Purpose and Environment	3424

A.7.5.3.14.2	Test Requirements	3427

A.7.5.4	Void	3428

A.7.5.5	Beam Failure Detection and Link recovery procedures	3428

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3428

A.7.5.5.1.1	Test Purpose and Environment	3428

A.7.5.5.1.2	Test Requirements	3431

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3432

A.7.5.5.2.1	Test Purpose and Environment	3432

A.7.5.5.2.2	Test Requirements	3435

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3436

A.7.5.5.3.1	Test Purpose and Environment	3436

A.7.5.5.3.2	Test Requirements	3440

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3441

A.7.5.5.4.1	Test Purpose and Environment	3441

A.7.5.5.4.2	Test Requirements	3445

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3446

A.7.5.5.5.1	Test Purpose and Environment	3446

A.7.5.5.5.2	Test Requirements	3449

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3450

A.7.5.5.6.1	Test Purpose and Environment	3450

A.7.5.5.6.2	Test Requirements	3454

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3454

A.7.5.5.7.1	Test Purpose and Environment	3454

A.7.5.5.7.2	Test Requirements	3458

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3459

A.7.5.5.8.1	Test Purpose and Environment	3459

A.7.5.5.8.2	Test Requirements	3463

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3463

A.7.5.5.9.1	Test Purpose and Environment	3463

A.7.5.5.9.2	Test Requirements	3469

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3469

A.7.5.5.10.1	Test Purpose and Environment	3469

A.7.5.5.10.2	Test Requirements	3475

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3475

A.7.5.5.11.1	Test Purpose and Environment	3475

A.7.5.5.11.2	Test Requirements	3480

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3480

A.7.5.5.12.1	Test Purpose and Environment	3480

A.7.5.5.12.2	Test Requirements	3485

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3485

A.7.5.5.13.1	Test Purpose and Environment	3485

A.7.5.5.13.2	Test Requirements	3489

A.7.5.6	Active BWP switch	3489

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3489

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3489

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3494

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3499

A.7.5.6.1.3.1	Test Purpose and Environment	3499

A.7.5.6.1.3.2	Test Requirements	3502

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3502

A.7.5.6.1.4.1	Test Purpose and Environment	3502

A.7.5.6.1.4.2	Test Requirements	3507

A.7.5.6.2	RRC-based Active BWP Switch	3508

A.7.5.6.2.1.1	Test Purpose and Environment	3508

A.7.5.6.2.1.2	Test Requirements	3511

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3512

A.7.5.6.2.2.1	Test Purpose and Environment	3512

A.7.5.6.2.2.2	Test Requirements	3516

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3517

A.7.5.6.3.1.1	Test Purpose and Environment	3517

A.7.5.6.3.1.2	Test Requirements	3520

A.7.5.6.4	SCell dormancy switch	3520

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3520

A.7.5.6.4.1.1	Test Purpose and Environment	3520

A.7.5.6.4.1.2	Test Requirements	3525

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3525

A.7.5.6.4.2.1	 Test Purpose and Environment	3525

A.7.5.6.4.2.2	 Test Requirements	3530

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3530

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3530

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3533

A.7.5.6.5.2.1	Test Purpose and Environment	3533

A.7.5.6.5.2.2	Test Requirements	3537

A.7.5.7	PSCell addition and release delay	3538

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3538

A.7.5.7.1.1	Test Purpose and Environment	3538

A.7.5.7.1.2	Test Requirements	3541

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3541

A.7.5.7.2.1	Test Purpose and Environment	3541

A.7.5.7.2.2	Test Requirements	3544

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3544

A.7.5.7.3.1	Test Purpose and Environment	3544

A.7.5.7.3.2	Test Requirements	3547

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3547

A.7.5.7.4.1	Test Purpose and Environment	3547

A.7.5.7.4.2	Test Requirements	3550

A.7.5.8	Active TCI state switch delay	3550

A.7.5.8.1	MAC-CE based active TCI state switch	3550

A.7.5.8.2	RRC based active TCI state switch	3554

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3558

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3558

A.7.5.8.3.1.1	Test Purpose and Environment	3558

A.7.5.8.3.1.2	Test Requirements	3562

A.7.5.9	Uplink spatial relation switch delay	3563

A.7.5.9.1.1.1	Test Purpose and Environment	3563

A.7.5.9.1.1.2	Test Requirements	3565

A.7.5.9.2	RRC based spatial relation switch	3565

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3565

A.7.5.9.2.1.1	Test Purpose and Environment	3565

A.7.5.9.2.1.2	Test Requirements	3568

A.7.5.10	UE specific CBW change	3568

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3568

A.7.5.10.1.1	Test Purpose and Environment	3568

A.7.5.10.1.2	Test Requirements	3571

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3572

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3572

A.7.5.11.1.1	Test Purpose and Environment	3572

A.7.5.11.1.2	Test Requirements	3575

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3575

A.7.5.12.1	Addition and Release Delay of PSCell	3575

A.7.5.12.1.1	Test purpose and environment	3575

A.7.5.12.1.2	Test Parameters	3575

A.7.5.12.1.3	Test Requirements	3578

A.7.5.13	 Unified TCI state switching delay	3578

A.7.5.13.1	MAC-CE based active joint TCI state switching	3578

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3578

A.7.5.13.1.1.1	Test Purpose and Environment	3578

A.7.5.13.1.1.2	Test parameters	3579

A.7.5.13.1.1.3	Test Requirements	3581

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3581

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3581

A.7.5.13.2.1.1	Test Purpose and Environment	3581

A.7.5.13.2.1.2	Test parameters	3582

A.7.5.13.2.1.3	Test Requirements	3584

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3584

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3584

A.7.5.13.3.1.1	Test Purpose and Environment	3584

A.7.5.13.3.1.2	Test Parameters	3585

A.7.5.13.3.1.3	Test Requirements	3588

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3588

A.7.5.14.1	Test Purpose and Environment	3588

A.7.5.14.2	Test Requirements	3591

A.7.5.15	Void	3592

A.7.6	Measurement procedure	3594

A.7.6.1	Intra-frequency Measurements	3594

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3594

A.7.6.1.1.1	Test purpose and Environment	3594

A.7.6.1.1.2	Test Requirements	3597

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3597

A.7.6.1.2.1	Test purpose and Environment	3597

A.7.6.1.2.2	Test Requirements	3600

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3601

A.7.6.1.3.1	Test purpose and Environment	3601

A.7.6.1.3.2	Test Requirements	3604

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3604

A.7.6.1.4.1	Test purpose and Environment	3604

A.7.6.1.4.2	Test Requirements	3607

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3608

A.7.6.1.5.1	Test purpose and Environment	3608

A.7.6.1.5.2	Test Requirements	3611

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3611

A.7.6.1.6.1	Test purpose and Environment	3611

A.7.6.1.6.2	Test Requirements	3614

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3615

A.7.6.1.7.1	Test purpose and Environment	3615

A.7.6.1.7.2	Test Requirements	3617

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3618

A.7.6.1.8.1	Test purpose and Environment	3618

A.7.6.1.8.2	Test Requirements	3621

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3622

A.7.6.1.9.1	Test purpose and Environment	3622

A.7.6.1.9.2	Test Requirements	3624

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3625

A.7.6.1.10.1	Test purpose and Environment	3625

A.7.6.1.10.2	Test Requirements	3629

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3629

A.7.6.1.11.1	Test purpose and Environment	3629

A.7.6.1.11.2	Test Requirements	3632

A.7.6.2	Inter-frequency Measurements	3632

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3632

A.7.6.2.1.1	Test Purpose and Environment	3632

A.7.6.2.1.2	Test Requirements	3636

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3636

A.7.6.2.2.1	Test Purpose and Environment	3636

A.7.6.2.2.2	Test Requirements	3640

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3640

A.7.6.2.3.1	Test Purpose and Environment	3640

A.7.6.2.3.2	Test Requirements	3644

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3644

A.7.6.2.4.1	Test Purpose and Environment	3644

A.7.6.2.4.2	Test Requirements	3648

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3648

A.7.6.2.5.1	Test Purpose and Environment	3648

A.7.6.2.5.2	Test Requirements	3652

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3653

A.7.6.2.6.1	Test Purpose and Environment	3653

A.7.6.2.6.2	Test Requirements	3657

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3658

A.7.6.2.7.1	Test Purpose and Environment	3658

A.7.6.2.7.2	Test Requirements	3662

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3663

A.7.6.2.8.1	Test Purpose and Environment	3663

A.7.6.2.8.2	Test Requirements	3667

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3668

A.7.6.2.9.1	Test Purpose and Environment	3668

A.7.6.2.9.2	Test Requirements	3672

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3672

A.7.6.2.10.1	Test Purpose and Environment	3672

A.7.6.2.10.2	Test Requirements	3674

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3674

A.7.6.2.11.1	Test Purpose and Environment	3674

A.7.6.2.11.2	Test Requirements	3677

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3677

A.7.6.2.12.1	Test Purpose and Environment	3677

A.7.6.2.12.2	Test Requirements	3681

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3682

A.7.6.2.13.1	Test Purpose and Environment	3682

A.7.6.2.13.2	Test Requirements	3686

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3687

A.7.6.2.14.1	Test Purpose and Environment	3687

A.7.6.2.14.2	Test Requirements	3691

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3692

A.7.6.2.15.1	Test Purpose and Environment	3692

A.7.6.2.15.2	Test Requirements	3696

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3697

A.7.6.2.16.1	Test Purpose and Environment	3697

A.7.6.2.16.2	Test Requirements	3703

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3703

A.7.6.2.17.1	Test Purpose and Environment	3703

A.7.6.2.17.2	Test Requirements	3710

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3711

A.7.6.2.18.1	Test Purpose and Environment	3711

A.7.6.2.18.2	Test Requirements	3717

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3717

A.7.6.2.19.1	Test Purpose and Environment	3717

A.7.6.2.19.2	Test Requirements	3725

A.7.6.3	L1-RSRP measurement for beam reporting	3726

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3726

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3726

A.7.6.3.1.1	Test Purpose and Environment	3726

A.7.6.3.1.2	Test parameters	3726

A.7.6.3.1.3	Test Requirements	3728

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3728

A.7.6.3.2.1	Test Purpose and Environment	3728

A.7.6.3.2.2	Test parameters	3729

A.7.6.3.2.3	Test Requirements	3731

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3731

A.7.6.3.3.1	Test Purpose and Environment	3731

A.7.6.3.3.2	Test parameters	3732

A.7.6.3.3.3	Test Requirements	3734

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3735

A.7.6.3.4.1	Test Purpose and Environment	3735

A.7.6.3.4.2	Test parameters	3735

A.7.6.3.3.3	Test Requirements	3737

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3738

A.7.6.3.5.1	Test Purpose and Environment	3738

A.7.6.3.5.2	Test parameters	3738

A.7.6.3.5.3	Test Requirements	3740

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3740

A.7.6.3.6.1	Test Purpose and Environment	3740

A.7.6.3.6.2	Test parameters	3741

A.7.6.3.6.3	Test Requirements	3744

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3744

A.7.6.3.7.1	Test Purpose and Environment	3744

A.7.6.3.7.2	Test parameters	3745

A.7.6.3.7.3	Test Requirements	3748

A.7.6.4	CLI measurements	3749

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3749

A.7.6.4.1.1	Test Purpose and Environment	3749

A.7.6.4.1.2	Test Parameters	3749

A.7.6.4.1.3	Test Requirements	3751

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3751

A.7.6.4.2.1	Test Purpose and Environment	3751

A.7.6.4.2.2	Test Parameters	3752

A.7.6.4.2.3	Test Requirements	3753

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3754

A.7.6.5.1.1	Test Purpose and Environment	3754

A.7.6.5.1.2	Test Requirements	3757

A.7.6.6	L1-SINR measurement for beam reporting	3757

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3759

A.7.6.6.2.1	Test Purpose and Environment	3759

A.7.6.6.2.2	Test parameters	3760

A.7.6.6.2.3	Test Requirements	3762

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3762

A.7.6.6.3.1	Test Purpose and Environment	3762

A.7.6.6.3.2	Test parameters	3763

A.7.6.6.3.3	Test Requirements	3765

A.7.6.7	CSI-RS based intra-frequency Measurements	3765

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3765

A.7.6.7.1.1	Test purpose and Environment	3765

A.7.6.7.1.2	Test Requirements	3768

A.7.6.8	CSI-RS based inter-frequency Measurements	3768

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3768

A.7.6.8.1.1	Test Purpose and Environment	3768

A.7.6.8.1.2	Test Requirements	3771

A.7.6.9	RSTD measurements	3771

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3771

A.7.6.9.1.1	Test Purpose and Environment	3771

A.7.6.9.1.2	Test Requirements	3779

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3779

A.7.6.9.2.1	Test Purpose and Environment	3779

A.7.6.9.2.2	Test Requirements	3787

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3787

A.7.6.9.3.1	Test Purpose and Environment	3787

A.7.6.9.3.2	Test Requirements	3793

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3794

A.7.6.9.4.1	Test Purpose and Environment	3794

A.7.6.9.4.2	Test Requirements	3797

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3798

A.7.6.9.5.1	Test Purpose and Environment	3798

A.7.6.9.5.2	Test Requirements	3801

A.7.6.10 PRS-RSRP measurements	3802

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3802

A.7.6.10.1.1	Test Purpose and Environment	3802

A.7.6.10.1.2	Test Requirements	3806

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3806

A.7.6.10.2.1	Test Purpose and Environment	3806

A.7.6.10.2.2	Test Requirements	3810

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3810

A.7.6.10.3.1	Test Purpose and Environment	3810

A.7.6.10.3.2	Test Requirements	3814

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3814

A.7.6.10.4.1	Test Purpose and Environment	3814

A.7.6.10.4.2	Test Requirements	3818

A.7.6.11	UE Rx-Tx time difference measurements	3818

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3818

A.7.6.11.1.1	Test purpose and environment	3818

A.7.6.11.1.2	Test requirements	3822

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3822

A.7.6.11.2.1	Test purpose and environment	3822

A.7.6.11.2.2	Test requirements	3826

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3826

A.7.6.11.3.1	Test purpose and environment	3826

A.7.6.11.3.2	Test requirements	3828

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3829

A.7.6.11.4.1	Test purpose and environment	3829

A.7.6.11.4.2	Test requirements	3831

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3832

A.7.6.11.5.1	Test purpose and environment	3832

A.7.6.11.5.2	Test requirements	3834

A.7.6.12	PRS-RSRPP measurements	3835

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3835

A.7.6.12.1.1	Test Purpose and Environment	3835

A.7.6.12.1.2	Test Requirements	3837

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3838

A.7.6.12.2.1	Test Purpose and Environment	3838

A.7.6.12.2.2	Test Requirements	3840

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3841

A.7.6.12.3.1	Test Purpose and Environment	3841

A.7.6.12.3.2	Test Requirements	3844

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3844

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3844

A.7.6.13.1.1	Test purpose and environment	3844

A.7.6.13.1.2	Test requirements	3846

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3847

A.7.6.13.2.1	Test purpose and environment	3847

A.7.6.13.2.2	Test requirements	3849

A.7.6.14	SA event triggered reporting tests with Pre-MG	3850

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3850

A.7.6.14.1.1	Test purpose and Environment	3850

A.7.6.14.1.2	Test parameters	3850

A.7.6.14.1.3	Test Requirements	3852

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3853

A.7.6.14.2.1	Test purpose and Environment	3853

A.7.6.14.2.2	Test parameters	3853

A.7.6.14.2.3	Test Requirements	3855

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3856

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3856

A.7.6.15.1.1	Test Purpose and Environment	3856

A.7.6.15.1.2	Test Requirements	3859

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3859

A.7.6.15.2.1	Test Purpose and Environment	3859

A.7.6.15.2.2	Test Requirements	3862

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3862

A.7.6.15.3.1	Test Purpose and Environment	3862

A.7.6.15.3.2	Test Requirements	3866

A.7.6.16	SA event triggered reporting tests with NCSG	3866

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3866

A.7.6.16.1.1	Test purpose and Environment	3866

A.7.6.16.1.2	Test Requirements	3870

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3870

A.7.6.16.2.1	Test Purpose and Environment	3870

A.7.6.16.2.2	Test Requirements	3873

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3873

A.7.6.16.3.1	Test Purpose and Environment	3873

A.7.6.16.3.2	Test Requirements	3876

A.7.7	Measurement Performance requirements	3876

A.7.7.1	SS-RSRP	3877

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3877

A.7.7.1.1.1	Test Purpose and Environment	3877

A.7.7.1.1.2	Test parameters	3877

A.7.7.1.1.3	Test Requirements	3881

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3881

A.7.7.1.2.1	Test Purpose and Environment	3881

A.7.7.1.2.2	Test parameters	3882

A.7.7.1.2.3	Test Requirements	3886

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3887

A.7.7.1.3.1	Test Purpose and Environment	3887

A.7.7.1.3.2	Test parameters	3887

A.7.7.1.3.3	Test Requirements	3891

A.7.7.2	SS-RSRQ	3891

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3891

A.7.7.2.1.1	Test Purpose and Environment	3891

A.7.7.2.1.2	Test Parameters	3891

A.7.7.2.1.3	Test Requirements	3893

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3893

A.7.7.2.2.1	Test Purpose and Environment	3893

A.7.7.2.2.2	Test Parameters	3893

A.7.7.2.2.3	Test Requirements	3895

A.7.7.3	SS-SINR	3895

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3895

A.7.7.3.1.1	Test Purpose and Environment	3895

A.7.7.3.1.2	Test Parameters	3895

A.7.7.3.1.3	Test Requirements	3897

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3897

A.7.7.3.2.1	Test Purpose and Environment	3897

A.7.7.3.2.2	Test Parameters	3897

A.7.7.3.2.3	Test Requirements	3899

A.7.7.4	L1-RSRP measurement for beam reporting	3899

A.7.7.4.1	SSB based L1-RSRP measurement	3899

A.7.7.4.1.1	Test Purpose and Environment	3899

A.7.7.4.1.2	Test parameters	3900

A.7.7.4.1.3	Test Requirements	3902

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3903

A.7.7.4.2.1	Test Purpose and Environment	3903

A.7.7.4.2.2	Test parameters	3903

A.7.7.4.2.3	Test Requirements	3905

A.7.7.5	CLI measurements	3906

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3906

A.7.7.5.1.1	Test Purpose and Environment	3906

A.7.7.5.1.2	Test parameters	3906

A.7.7.5.1.3	Test Requirements	3909

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3910

A.7.7.5.2.1	Test Purpose and Environment	3910

A.7.7.5.2.2	Test parameters	3910

A.7.7.5.2.3	Test Requirements	3912

A.7.7.6	L1-SINR measurement for beam reporting	3913

A.7.7.6.1.1	Test Purpose and Environment	3913

A.7.7.6.1.2	Test parameters	3913

A.7.7.6.1.3	Test Requirements	3915

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3916

A.7.7.6.2.1	Test Purpose and Environment	3916

A.7.7.6.2.2	Test parameters	3916

A.7.7.6.2.3	Test Requirements	3918

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3919

A.7.7.6.3.1	Test Purpose and Environment	3919

A.7.7.6.3.2	Test parameters	3919

A.7.7.6.3.3	Test Requirements	3921

A.7.7.7	CSI-RSRP	3922

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3922

A.7.7.7.1.1	Test Purpose and Environment	3922

A.7.7.7.1.2	Test parameters	3922

A.7.7.7.1.3	Test Requirements	3925

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3926

A.7.7.7.2.1	Test Purpose and Environment	3926

A.7.7.7.2.2	Test parameters	3926

A.7.7.7.2.3	Test Requirements	3929

A.7.7.8	CSI-RSRQ	3930

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3930

A.7.7.8.1.1	Test Purpose and Environment	3930

A.7.7.8.1.2	Test Parameters	3930

A.7.7.8.1.3	Test Requirements	3932

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3932

A.7.7.8.2.1	Test Purpose and Environment	3932

A.7.7.8.2.2	Test Parameters	3932

A.7.7.8.2.3	Test Requirements	3934

A.7.7.9	CSI-SINR	3934

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3934

A.7.7.9.1.1	Test Purpose and Environment	3934

A.7.7.9.1.2	Test Parameters	3934

A.7.7.9.1.3	Test Requirements	3936

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3936

A.7.7.9.2.1	Test Purpose and Environment	3936

A.7.7.9.2.2	Test Parameters	3937

A.7.7.9.2.3	Test Requirements	3939

A.7.7.10	RSTD measurements	3939

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	3939

A.7.7.10.1.1	Test purpose and Environment	3939

A.7.7.10.1.2	Test Requirements	3942

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	3942

A.7.7.10.2.1	Test purpose and Environment	3942

A.7.7.10.2.2	Test Requirements	3945

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3945

A.7.7.10.3.1	Test purpose and Environment	3945

A.7.7.10.3.2	Test Requirements	3947

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	3947

A.7.7.10.4.1	Test purpose and Environment	3947

A.7.7.10.4.2	Test Requirements	3950

A.7.7.11	PRS-RSRP measurements	3950

A.7.7.11.1	SA measurement accuracy with PRS in FR2	3950

A.7.7.11.1.1	Test Purpose and Environment	3950

A.7.7.11.1.2	Test parameters	3950

A.7.7.11.1.3	Test Requirements	3954

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	3955

A.7.7.11.2.1	Test Purpose and Environment	3955

A.7.7.11.2.2	Test parameters	3955

A.7.7.11.2.3	Test Requirements	3957

A.7.7.12	UE Rx-Tx time difference measurements	3958

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	3958

A.7.7.12.1.1	Test purpose and environment	3958

A.7.7.12.1.2	Test parameters	3958

A.7.7.12.1.3	Test requirements	3961

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	3962

A.7.7.12.2.1	Test purpose and environment	3962

A.7.7.12.2.2	Test parameters	3962

A.7.7.12.2.3	Test requirements	3965

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3965

A.7.7.12.3.1	Test purpose and environment	3965

A.7.7.12.3.2	Test parameters	3966

A.7.7.12.3.3	Test requirements	3969

A.7.7.13	PRS-RSRPP measurements	3969

A.7.7.13.1	SA measurement accuracy with PRS in FR2	3969

A.7.7.13.1.1	Test Purpose and Environment	3969

A.7.7.13.1.2	Test parameters	3970

A.7.7.13.1.3	Test Requirements	3972

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	3972

A.7.7.13.2.1	Test Purpose and Environment	3972

A.7.7.13.2.2	Test parameters	3973

A.7.7.13.2.3	Test Requirements	3975

A.7.8	Measurement procedure in RRC_INACTIVE	3976

A.7.8.1	RSTD measurements	3976

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	3976

A.7.8.1.1.1	Test Purpose and Environment	3976

A.7.8.1.1.2	Test Requirements	3979

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3979

A.7.8.1.2.1	Test Purpose and Environment	3979

A.7.8.1.2.2	Test Requirements	3985

A.7.8.2	PRS-RSRP measurements	3986

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3986

A.7.8.2.1.1	Test Purpose and Environment	3986

A.7.8.2.1.2	Test Requirements	3990

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3990

A.7.8.2.2.1	Test purpose and Environment	3990

A.7.8.2.2.2	Test Requirements	3994

A.7.8.3	UE Rx-Tx time difference measurements	3994

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3994

A.7.8.3.1.1	Test purpose and environment	3994

A.7.8.3.1.2	Test requirements	3998

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	3998

A.7.8.3.2.1	Test purpose and environment	3998

A.7.8.3.2.2	Test requirements	4000

A.7.8.4	PRS-RSRPP measurements	4001

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4001

A.7.8.4.1.1	Test Purpose and Environment	4001

A.7.8.4.1.2	Test Requirements	4003

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4004

A.7.8.4.2.1	Test Purpose and Environment	4004

A.7.8.4.2.2	Test Requirements	4006

A.7.9	Measurement performance requirements in RRC_INACTIVE	4007

A.7.9.1	RSTD measurements	4007

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4007

A.7.9.1.1.1	Test purpose and Environment	4007

A.7.9.1.1.2	Test Requirements	4009

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4009

A.7.9.1.2.1	Test purpose and Environment	4009

A.7.9.1.2.2	Test Requirements	4011

A.7.9.2	PRS-RSRP measurements	4011

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	4011

A.7.9.2.1.1	Test Purpose and Environment	4011

A.7.9.2.1.2	Test parameters	4011

A.7.9.2.1.3	Test Requirements	4013

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	4014

A.7.9.2.2.1	Test Purpose and Environment	4014

A.7.9.2.2.2	Test parameters	4014

A.7.9.2.2.3	Test Requirements	4016

A.7.9.3	UE Rx-Tx time difference measurements	4017

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	4017

A.7.9.3.1.1	Test purpose and environment	4017

A.7.9.3.1.2	Test parameters	4017

A.7.9.3.1.3	Test requirements	4020

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	4020

A.7.9.3.2.1	Test purpose and environment	4020

A.7.9.3.2.2	Test parameters	4020

A.7.9.3.2.3	Test requirements	4023

A.7.9.4	PRS-RSRPP measurements	4023

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	4023

A.7.9.4.1.1	Test Purpose and Environment	4023

A.7.9.4.1.2	Test parameters	4023

A.7.9.4.1.3	Test Requirements	4025

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	4026

A.7.9.4.2.1	Test Purpose and Environment	4026

A.7.9.4.2.2	Test parameters	4026

A.7.9.4.2.3	Test Requirements	4028

A.8	E-UTRA standalone tests for NR RRM	4024

A.8.1	Void	4024

A.8.2	RRC_IDLE state mobility	4024

A.8.2.1	Inter-RAT NR Cell re-selection	4024

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	4024

A.8.2.1.1.1	Test Purpose and Environment	4024

A.8.2.1.1.2	Test Requirements	4029

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	4030

A.8.2.1.2.1	Test Purpose and Environment	4030

A.8.2.1.2.2	Test Requirements	4034

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	4035

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	4035

A.8.2.2.1.1	Test Purpose and Environment	4035

A.8.2.2.1.2	Test Requirements	4039

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	4039

A.8.2.2.2.1	Test Purpose and Environment	4039

A.8.2.2.2.2	Test Requirements	4042

A.8.3	RRC_CONNECTED state mobility	4043

A.8.3.1	Handover	4043

A.8.3.1.1	E-UTRAN - NR handover in FR1	4043

A.8.3.1.1.1	Test Purpose and Environment	4043

A.8.3.1.1.2	Test Requirements	4048

A.8.4	Measurement procedure	4048

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	4048

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	4048

A.8.4.1.1.1	Test Purpose and Environment	4048

A.8.4.1.1.2	Test Requirements	4050

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	4051

A.8.4.1.2.1	Test Purpose and Environment	4051

A.8.4.1.2.2	Test Requirements	4052

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	4052

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4052

A.8.4.2.1.1	Test Purpose and Environment	4052

A.8.4.2.1.2	Test Requirements	4057

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4057

A.8.4.2.2.1	Test Purpose and Environment	4057

A.8.4.2.2.2	Test Requirements	4061

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4061

A.8.4.2.3.1	Test Purpose and Environment	4061

A.8.4.2.3.2	Test Requirements	4066

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4066

A.8.4.2.4.1	Test Purpose and Environment	4066

A.8.4.2.4.2	Test Requirements	4070

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4070

A.8.4.2.5.1	Test Purpose and Environment	4070

A.8.4.2.5.2	Test Requirements	4072

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4073

A.8.4.2.6.1	Test Purpose and Environment	4073

A.8.4.2.6.2	Test Requirements	4075

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4076

A.8.4.2.7.1	Test Purpose and Environment	4076

A.8.4.2.7.2	Test Requirements	4078

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4079

A.8.4.2.8.1	Test Purpose and Environment	4079

A.8.4.2.8.2	Test Requirements	4081

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	4082

A.8.4.2.9.1	Test Purpose and Environment	4082

A.8.4.2.9.2	Test Requirements	4086

A.8.5	Measurement performance	4086

A.8.5.1	SFTD accuracy	4086

A.8.5.1.1	SFTD accuracy	4086

A.8.5.1.1.1	Test Purpose	4086

A.8.5.1.1.2	Test Environment	4087

A.8.5.1.1.3	Test Requirements	4093

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	4093

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4093

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4098

A.8.5.2.1.2.1	Test Purpose and Environment	4098

A.8.5.2.1.2.2	Test Parameters	4098

A.8.5.2.1.2.3	Test Requirements	4100

A.8.5.2.2	SS-RSRQ	4100

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4100

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4105

A.8.5.2.2.2.1	Test Purpose and Environment	4105

A.8.5.2.2.2.2	Test Parameters	4105

A.8.5.2.2.2.3	Test Requirements	4107

A.8.5.2.3	SS-SINR	4107

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4107

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4111

A.8.5.2.3.2.1	Test Purpose and Environment	4111

A.8.5.2.3.2.2	Test Parameters	4111

A.8.5.2.3.2.3	Test Requirements	4113

A.9	V2X Tests	4114

A.9.1	V2X Tests in FR1	4114

A.9.1.1	Test for V2X UE Transmit Timing	4114

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	4114

A.9.1.1.1.1	Test Purpose and Environment	4114

A.9.1.1.1.2	Test requirements	4114

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	4114

A.9.1.1.2.1	Test Purpose and Environment	4114

A.9.1.1.2.2	Test requirements	4115

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4115

A.9.1.1.3.1	Test Purpose and Environment	4115

A.9.1.1.3.2	Test requirements	4118

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4118

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4118

A.9.1.2.1.1	Test Purpose and Environment	4118

A.9.1.2.1.2	Test Requirements	4121

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4121

A.9.1.2.2.1	Test Purpose and Environment	4121

A.9.1.2.2.2	Test Requirements	4123

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4123

A.9.1.2.3.1	Test Purpose and Environment	4123

A.9.1.2.3.2	Test Requirements	4125

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4125

A.9.1.3.1	 Test for GNSS configured as the highest priority	4125

A.9.1.3.1.1	Test Purpose and Environment	4125

A.9.1.3.1.2	Test Requirements	4127

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4128

A.9.1.3.2.1	Test Purpose and Environment	4128

A.9.1.3.2.2	Test Requirements	4130

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4131

A.9.1.3.3.1	Test Purpose and Environment	4131

A.9.1.3.3.2	Test Requirements	4133

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4134

A.9.1.3.4.1	Test Purpose and Environment	4134

A.9.1.3.4.2	Test Requirements	4136

A.9.1.4	Test for L1 SL-RSRP Measurement	4137

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4137

A.9.1.4.1.1	Test Purpose and Environment	4137

A.9.1.4.1.2	Test Requirements	4140

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4140

A.9.1.4.2.1	Test Purpose and Environment	4140

A.9.1.4.2.2	Test Requirements	4144

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4144

A.9.1.4.3.1	Test Purpose and Environment	4144

A.9.1.4.3.2	Test Requirements	4151

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4151

A.9.1.4.4.1	Test Purpose and Environment	4151

A.9.1.4.4.2	Test Requirements	4155

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4155

A.9.1.4.5.1	Test Purpose and Environment	4155

A.9.1.4.5.2	Test Requirements	4158

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4158

A.9.1.4.6.1	Test Purpose and Environment	4158

A.9.1.4.6.2	Test Requirements	4162

A.9.1.5	Test for Congestion Control Measurement	4162

A.9.1.5.1	Test Purpose and Environment	4162

A.9.1.5.2	Test Requirements	4168

A.9.1.6	Test for Interruption	4168

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4168

A.9.1.6.1.1	Test Purpose and Environment	4168

A.9.1.6.1.2	Test Requirements	4171

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4171

A.9.1.6.2.1	Test Purpose and Environment	4171

A.9.1.6.2.2	Test Requirements	4175

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4175

A.9.1.6.3.1	Test Purpose and Environment	4175

A.9.1.6.3.2	Test Requirements	4178

A.9.1.7	Selection / Reselection of relay UE	4178

A.9.1.7.1	Test Purpose and Environment	4178

A.9.1.7.2	Test Requirements	4184

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4185

A.10.1	RRC_CONNECTED state mobility	4185

A.10.1.1	RRC connection mobility control	4185

A.10.1.1.1	Random Access	4185

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4185

A.10.1.1.1.1.1	Test Purpose and Environment	4185

A.10.1.1.1.1.2	Test Requirements	4188

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4188

A.10.1.1.1.1.2.2	Random Access Response Reception	4188

A.10.1.1.1.1.2.3	No Random Access Response Reception	4189

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4189

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4189

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4190

A.10.1.1.1.2.1	Test Purpose and Environment	4190

A.10.1.1.1.2.2	Test Requirements	4193

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4193

A.10.1.1.1.2.2.2	Random Access Response Reception	4194

A.10.1.1.1.2.2.3	No Random Access Response Reception	4194

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4194

A.10.1.1.1.3.1	Test Purpose and Environment	4194

A.10.1.1.1.3.2	Test Requirements	4198

A.10.1.1.1.3.2.1	MsgA Transmission	4198

A.10.1.1.1.3.2.2	MsgB Reception	4198

A.10.1.1.1.3.2.3	No MsgB Reception	4199

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4199

A.10.1.1.1.4.1	Test Purpose and Environment	4199

A.10.1.1.1.4.2	Test Requirements	4203

A.10.1.1.1.4.2.1	MsgA Transmission	4203

A.10.1.1.1.4.2.2	MsgB Reception	4204

A.10.1.1.1.4.2.3	No MsgB Reception	4204

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4204

A.10.1.2.1	Test Purpose and Environment	4204

A.10.1.2.2	Test Requirements	4210

A.10.2	Timing	4211

A.10.2.1	UE transmit timing	4211

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4211

A.10.2.1.1.1	Test Purpose and environment	4211

A.10.2.1.1.2	Test requirements	4214

A.10.2.2	UE timing advance	4215

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4215

A.10.2.2.1.1	Test Purpose and Environment	4215

A.10.2.2.1.2	Test Parameters	4215

A.10.2.2.1.3	Test Requirements	4219

A.10.3	Signalling characteristics	4219

A.10.3.1	Radio link monitoring	4219

A.10.3.1.1	Introduction	4219

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4220

A.10.3.1.2.1	Test purpose and environment	4220

A.10.3.1.2.2	Test requirements	4225

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4225

A.10.3.1.3.1	Test purpose and environment	4225

A.10.3.1.3.2	Test requirements	4231

A.10.3.1.4	Void	4231

A.10.3.1.4.1	Void	4231

A.10.3.1.4.2	Void	4231

A.10.3.1.5	Void	4231

A.10.3.1.5.1	Void	4231

A.10.3.1.5.2	Void	4231

A.10.3.2	Interruption	4231

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	4231

A.10.3.2.1.1	Test Purpose and Environment	4231

A.10.3.2.1.2	Test Requirements	4236

A.10.3.3	SCell activation and deactivation delay	4236

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4236

A.10.3.3.1.1	Test Purpose and Environment	4236

A.10.3.3.1.2	Test Requirements	4241

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4241

A.10.3.3.2.1	Test Purpose and Environment	4241

A.10.3.3.2.2	Test Requirements	4242

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4242

A.10.3.3.3.1	Test Purpose and Environment	4242

A.10.3.3.3.2	Test Requirements	4243

A.10.3.4	Beam failure detection and link recovery procedures	4243

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4243

A.10.3.4.1.1	Test Purpose and Environment	4243

A.10.3.4.1.2	Test Requirements	4248

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4248

A.10.3.4.2.1	Test Purpose and Environment	4248

A.10.3.4.2.2	Test Requirements	4254

A.10.3.5	Active BWP switching	4254

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4254

A.10.3.5.1.2	Test Requirements	4259

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4260

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4260

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4263

A.10.3.5.3	RRC-based Active BWP Switch	4267

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4267

A.10.3.6	PSCell addition and release delay	4271

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4271

A.10.3.6.1.1	Test purpose and environment	4271

A.10.3.6.1.2	Test Requirements	4276

A.10.3.7	Void	4277

A.10.4	Measurement procedure	4277

A.10.4.1	Intra-frequency measurements	4277

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4277

A.10.4.1.1.1	Test purpose and environment	4277

A.10.4.1.1.2	Test parameters	4277

A.10.4.1.1.3	Test Requirements	4281

A.10.4.1.2	Void	4281

A.10.4.1.3	Void	4281

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4282

A.10.4.1.4.1	Test purpose and environment	4282

A.10.4.1.4.2	Test parameters	4282

A.10.4.1.4.3	Test Requirements	4286

A.10.4.1.5	Void	4286

A.10.4.1.6	Void	4286

A.10.4.1.7	Void	4287

A.10.4.1.8	Void	4287

A.10.4.1.9	RSSI measurement reporting on PSCC	4287

A.10.4.1.9.1	Test purpose and environment	4287

A.10.4.1.9.2	Test parameters	4287

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	4287

A.10.4.1.10.1	Test purpose and environment	4287

A.10.4.1.10.2	Test parameters	4287

A.10.4.1.11	RSSI measurement reporting on SCC	4288

A.10.4.1.11.1	Test purpose and environment	4288

A.10.4.1.11.2	Test parameters	4288

A.10.4.1.12	Channel occupancy measurement reporting on SCC	4288

A.10.4.1.12.1	Test purpose and environment	4288

A.10.4.1.12.2	Test parameters	4288

A.10.4.2	Inter-frequency measurements	4289

A.10.4.2.1	RSSI measurement reporting	4289

A.10.4.2.1.1	Test purpose and environment	4289

A.10.4.2.1.2	Test parameters	4289

A.10.4.2.2	Channel occupancy measurement reporting	4289

A.10.4.2.2.1	Test purpose and environment	4289

A.10.4.2.2.2	Test parameters	4289

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4290

A.10.4.2.3.1	Test Purpose and Environment	4290

A.10.4.2.3.2	Test Requirements	4294

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4295

A.10.4.2.4.1	Test Purpose and Environment	4295

A.10.4.2.4.2	Test Requirements	4299

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4300

A.10.4.2.5.1	Test Purpose and Environment	4300

A.10.4.2.5.2	Test Requirements	4304

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4305

A.10.4.2.6.1	Test Purpose and Environment	4305

A.10.4.2.6.2	Test Requirements	4309

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4310

A.10.4.2.7.1	Test Purpose and Environment	4310

A.10.4.2.7.2	Test Requirements	4316

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4316

A.10.4.2.8.1	Test Purpose and Environment	4316

A.10.4.2.8.2	Test Requirements	4322

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4323

A.10.4.2.9.1	Test Purpose and Environment	4323

A.10.4.2.9.2	Test Requirements	4328

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4328

A.10.4.2.10.1	Test Purpose and Environment	4328

A.10.4.2.10.2	Test Requirements	4334

A.10.4.3	L1-RSRP measurements for beam reporting	4335

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4335

A.10.4.3.1.1	Test Purpose and Environment	4335

A.10.4.3.1.2	Test parameters	4335

A.10.4.3.1.3	Test Requirements	4337

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4338

A.10.4.3.2.1	Test Purpose and Environment	4338

A.10.4.3.2.2	Test parameters	4338

A.10.4.3.2.3	Test Requirements	4340

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4341

A.10.4.3.3.1	Test Purpose and Environment	4341

A.10.4.3.3.2	Test parameters	4341

A.10.4.3.3.3	Test Requirements	4344

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4345

A.10.4.3.4.1	Test Purpose and Environment	4345

A.10.4.3.4.2	Test parameters	4345

A.10.4.3.4.3	Test Requirements	4348

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4349

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4349

A.10.4.4.1.1	Test Purpose and Environment	4349

A.10.4.4.1.2	Test Requirements	4353

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4354

A.10.4.4.2.1	Test Purpose and Environment	4354

A.10.4.4.2.2	Test Requirements	4359

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4360

A.10.4.4.3.1	Test Purpose and Environment	4360

A.10.4.4.3.2	Test Requirements	4364

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4365

A.10.4.4.4.1	Test Purpose and Environment	4365

A.10.4.4.4.2	Test Requirements	4370

A.10.5	Measurement performance	4371

A.10.5.1	SS-RSRP	4371

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4371

A.10.5.1.1.1	Test Purpose and Environment	4371

A.10.5.1.1.2	Test parameters	4371

A.10.5.1.1.3	Test Requirements	4374

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4374

A.10.5.1.2.1	Test Purpose and Environment	4374

A.10.5.1.2.2	Test parameters	4374

A.10.5.1.2.3	Test Requirements	4377

A.10.5.2	SS-RSRQ	4377

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4377

A.10.5.2.1.1	Test Purpose and Environment	4377

A.10.5.2.1.2	Test Parameters	4377

A.10.5.2.1.3	Test Requirements	4380

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4380

A.10.5.2.2.1	Test Purpose and Environment	4380

A.10.5.2.2.2	Test Parameters	4380

A.10.5.2.2.3	Test Requirements	4383

A.10.5.3	SS-SINR	4383

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4383

A.10.5.3.1.1	Test Purpose and Environment	4383

A.10.5.3.1.2	Test Parameters	4383

A.10.5.3.1.3	Test Requirements	4386

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4386

A.10.5.3.2.1	Test Purpose and Environment	4386

A.10.5.3.2.2	Test Parameters	4386

A.10.5.3.2.3	Test Requirements	4389

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4389

A.10.5.3.3.1	Test Purpose and Environment	4389

A.10.5.3.3.2	Test Parameters	4389

A.10.5.3.3.3	Test Requirements	4392

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4392

A.10.5.4.1	SSB based L1-RSRP measurement	4392

A.10.5.4.1.1	Test Purpose and Environment	4392

A.10.5.4.1.2	Test parameters	4393

A.10.5.4.1.3	Test Requirements	4395

A.10.5.5	RSSI	4395

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4395

A.10.5.5.1.1	Test Purpose and Environment	4395

A.10.5.5.1.2	Test parameters	4395

A.10.5.5.1.3	Test Requirements	4398

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4398

A.10.5.5.2.1	Test Purpose and Environment	4398

A.10.5.5.2.2	Test parameters	4398

A.10.5.5.2.3	Test Requirements	4401

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4401

A.10.5.5.3.1	Test Purpose and Environment	4401

A.10.5.5.3.2	Test parameters	4401

A.10.5.5.3.3	Test Requirements	4405

A.10.5.6	Channel occupancy	4405

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4405

A.10.5.6.1.1	Test Purpose and Environment	4405

A.10.5.6.1.2	Test parameters	4405

A.10.5.6.1.3	Test Requirements	4409

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4409

A.10.5.6.2.1	Test Purpose and Environment	4409

A.10.5.6.2.2	Test parameters	4409

A.10.5.6.2.3	Test Requirements	4412

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4412

A.10.5.6.3.1	Test Purpose and Environment	4412

A.10.5.6.3.2	Test parameters	4412

A.10.5.6.3.3	Test Requirements	4416

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4420

A.11.1	RRC_IDLE state mobility	4420

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4420

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4420

A.11.1.1.1.1	Test Purpose and Environment	4420

A.11.1.1.1.2	Test Parameters	4420

A.11.1.1.1.3	Test Requirements	4423

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4423

A.11.1.1.2.1	Test Purpose and Environment	4423

A.11.1.1.2.2	Test Parameters	4423

A.11.1.1.2.3	Test Requirements	4427

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4427

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4427

A.11.1.2.1.1	Test Purpose and Environment	4427

A.11.1.2.1.2	Test Parameters	4428

A.11.1.2.1.3	Test Requirements	4434

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4435

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4435

A.11.1.3.1.1	Test Purpose and Environment	4435

A.11.1.3.1.2	Test Parameters	4435

A.11.1.3.1.3	Test Requirements	4441

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4442

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4442

A.11.1.4.1.1	Test Purpose and Environment	4442

A.11.1.4.1.2	Test Parameters	4442

A.11.1.4.1.3	Test Requirements	4445

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4446

A.11.1.4.2.1	Test Purpose and Environment	4446

A.11.1.4.2.2	Test Requirements	4449

A.11.2	RRC_CONNECTED state mobility	4450

A.11.2.1	Handover	4450

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4450

A.11.2.1.1.1	Test Purpose and Environment	4450

A.11.2.1.1.2	Test Parameters	4450

A.11.2.1.1.3 Test Requirements	4453

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4453

A.11.2.1.2.1	Test Purpose and Environment	4453

A.11.2.1.2.2	Test Parameters	4453

A.11.2.1.2.3	Test Requirements	4456

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4456

A.11.2.1.3.1	Test Purpose and Environment	4456

A.11.2.1.3.2	Test Parameters	4456

A.11.2.1.3.3 Test Requirements	4459

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4459

A.11.2.1.4.1	Test Purpose and Environment	4459

A.11.2.1.4.2	Test Parameters	4459

A.11.2.1.4.3 Test Requirements	4463

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4464

A.11.2.1.5.1	Test Purpose and Environment	4464

A.11.2.1.5.2	Test Parameters	4464

A.11.2.1.5.3 Test Requirements	4467

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4468

A.11.2.1.6.1	Test Purpose and Environment	4468

A.11.2.1.6.2	Test Parameters	4468

A.11.2.1.6.3 Test Requirements	4471

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4472

A.11.2.1.7.1	Test Purpose and Environment	4472

A.11.2.1.7.2	Test Requirements	4478

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4478

A.11.2.1.8.1	Test Purpose and Environment	4478

A.11.2.1.8.2	Test Requirements	4483

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4483

A.11.2.1.9.1	Test Purpose and Environment	4483

A.11.2.1.9.2	Test Requirements	4492

A.11.2.2	RRC connection mobility control	4493

A.11.2.2.1	RRC re-establishment	4493

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4493

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4496

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4503

A.11.2.2.2	Random Access	4508

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4508

A.11.2.2.2.1.1	Test Purpose and Environment	4508

A.11.2.2.2.1.2	Test Requirements	4511

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4511

A.11.2.2.2.1.2.2	Random Access Response Reception	4511

A.11.2.2.2.1.2.3	No Random Access Response Reception	4512

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4512

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4512

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4513

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4513

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4513

A.11.2.2.2.2.1	Test Purpose and Environment	4513

A.11.2.2.2.2.2	Test Requirements	4516

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4516

A.11.2.2.2.2.2.2	Random Access Response Reception	4517

A.11.2.2.2.2.2.3	No Random Access Response Reception	4517

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4517

A.11.2.2.2.3.1	Test Purpose and Environment	4517

A.11.2.2.2.3.2	Test Requirements	4521

A.11.2.2.2.3.2.1	MsgA Transmission	4521

A.11.2.2.2.3.2.2	MsgB Reception	4521

A.11.2.2.2.3.2.3	No MsgB Reception	4522

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4522

A.11.2.2.2.4.1	Test Purpose and Environment	4522

A.11.2.2.2.4.2	Test Requirements	4526

A.11.2.2.2.4.2.1	MsgA Transmission	4526

A.11.2.2.2.4.2.2	MsgB Reception	4527

A.11.2.2.2.4.2.3	No MsgB Reception	4527

A.11.2.2.3	RRC connection release with redirection	4528

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4528

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4532

A.11.3	Timing	4537

A.11.3.1	UE transmit timing	4537

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4537

A.11.3.1.1.1	Test Purpose and environment	4537

A.11.3.1.1.2	Test requirements	4540

A.11.3.2	UE timing advance	4541

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4541

A.11.3.2.1.1	Test Purpose and Environment	4541

A.11.3.2.1.2	Test Parameters	4541

A.11.3.2.1.3	Test Requirements	4545

A.11.4	Signalling characteristics	4545

A.11.4.1	Radio link monitoring	4545

A.11.4.1.1	Introduction	4545

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4546

A.11.4.1.2.1	Test purpose and environment	4546

A.11.4.1.2.2	Test requirements	4550

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4550

A.11.4.1.3.1	Test purpose and environment	4550

A.11.4.1.3.2	Test requirements	4556

A.11.4.1.4	Void	4556

A.11.4.1.4.1	Void	4556

A.11.4.1.4.2	Void	4556

A.11.4.1.5	Void	4556

A.11.4.1.5.1	Void	4556

A.11.4.1.5.2	Void	4556

A.11.4.2	Interruption	4557

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	4557

A.11.4.2.1.1 Test Purpose and Environment	4557

A.11.4.2.1.2	Test Requirements	4561

A.11.4.3	SCell activation and deactivation delay	4561

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4561

A.11.4.3.1.1	Test Purpose and Environment	4561

A.11.4.3.1.2	Test Requirements	4566

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4566

A.11.4.3.2.1	Test Purpose and Environment	4566

A.11.4.3.2.2	Test Requirements	4567

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4567

A.11.4.3.3.1	Test Purpose and Environment	4567

A.11.4.3.3.2	Test Requirements	4568

A.11.4.4	Beam failure detection and link recovery procedures	4568

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4568

A.11.4.4.1.1	Test Purpose and Environment	4568

A.11.4.4.1.2	Test Requirements	4573

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4573

A.11.4.4.2.1	Test Purpose and Environment	4573

A.11.4.4.2.2	Test Requirements	4579

A.11.4.5	Active BWP switching	4579

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4579

A.11.4.5.1.1	Test Purpose and Environment	4579

A.11.4.5.1.2	Test Requirements	4583

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4584

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4584

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4587

A.11.4.5.3	RRC-based Active BWP Switch	4590

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4590

A.11.4.6	Void	4593

A.11.5	Measurement procedure	4593

A.11.5.1	Intra-frequency measurements	4593

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4593

A.11.5.1.1.1	Test purpose and environment	4593

A.11.5.1.1.2	Test parameters	4593

A.11.5.1.1.3	Test Requirements	4597

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4597

A.11.5.1.2.1	Test purpose and environment	4597

A.11.5.1.2.2	Test parameters	4597

A.11.5.1.2.3	Test Requirements	4601

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4601

A.11.5.1.3.1	Test purpose and environment	4601

A.11.5.1.3.2	Test parameters	4601

A.11.5.1.3.3	Test Requirements	4601

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4602

A.11.5.1.4.1	Test purpose and environment	4602

A.11.5.1.4.2	Test parameters	4602

A.11.5.1.4.3	Test Requirements	4602

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4602

A.11.5.1.5.1	Test purpose and environment	4602

A.11.5.1.5.2	Test parameters	4602

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4603

A.11.5.1.6.1	Test purpose and environment	4603

A.11.5.1.6.2	Test parameters	4603

A.11.5.1.6.3	Test Requirements	4603

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4603

A.11.5.1.7.1	Test purpose and environment	4603

A.11.5.1.7.2	Test parameters	4603

A.11.5.1.7.3	Test Requirements	4603

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4604

A.11.5.1.8.1	Test purpose and environment	4604

A.11.5.1.8.2	Test parameters	4604

A.11.5.1.8.3	Test Requirements	4604

A.11.5.1.9	RSSI measurement reporting on PCC	4604

A.11.5.1.9.1	Test purpose and environment	4604

A.11.5.1.9.2	Test parameters	4604

A.11.5.1.10	Channel occupancy measurement reporting on PCC	4605

A.11.5.1.10.1	Test purpose and environment	4605

A.11.5.1.10.2	Test parameters	4605

A.11.5.1.11	RSSI measurement reporting on SCC	4605

A.11.5.1.11.1	Test purpose and environment	4605

A.11.5.1.11.2	Test parameters	4605

A.11.5.1.12	Channel occupancy measurement reporting on SCC	4606

A.11.5.1.12.1	Test purpose and environment	4606

A.11.5.1.12.2	Test parameters	4606

A.11.5.2	Inter-frequency measurements	4606

A.11.5.2.1	RSSI measurement reporting	4606

A.11.5.2.1.1	Test purpose and environment	4606

A.11.5.2.1.2	Test parameters	4606

A.11.5.2.2	Channel occupancy measurement reporting	4607

A.11.5.2.2.1	Test purpose and environment	4607

A.11.5.2.2.2	Test parameters	4607

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4607

A.11.5.2.3.1	Test Purpose and Environment	4607

A.11.5.2.3.2	Test Requirements	4611

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4612

A.11.5.2.4.1	Test Purpose and Environment	4612

A.11.5.2.4.2	Test Requirements	4617

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4617

A.11.5.2.5.1	Test Purpose and Environment	4617

A.11.5.2.5.2	Test Requirements	4621

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4622

A.11.5.2.6.1	Test Purpose and Environment	4622

A.11.5.2.6.2	Test Requirements	4627

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4627

A.11.5.2.7.1	Test Purpose and Environment	4627

A.11.5.2.7.2	Test Requirements	4631

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4632

A.11.5.2.8.1	Test Purpose and Environment	4632

A.11.5.2.8.2	Test Requirements	4637

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4637

A.11.5.2.9.1	Test Purpose and Environment	4637

A.11.5.2.9.2	Test Requirements	4642

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4643

A.11.5.2.10.1	Test Purpose and Environment	4643

A.11.5.2.10.2	Test Requirements	4648

A.11.5.3	Inter-RAT E-UTRAN measurements	4649

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4649

A.11.5.3.1.1	Test Purpose and Environment	4649

A.11.5.3.1.2	Test Requirements	4654

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4654

A.11.5.3.2.1	Test Purpose and Environment	4654

A.11.5.3.2.2	Test Requirements	4660

A.11.5.4	L1-RSRP measurements for beam reporting	4660

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4660

A.11.5.4.1.1	Test Purpose and Environment	4660

A.11.5.4.1.2	Test parameters	4660

A.11.5.4.1.3	Test Requirements	4662

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4663

A.11.5.4.2.1	Test Purpose and Environment	4663

A.11.5.4.2.2	Test parameters	4663

A.11.5.4.2.3	Test Requirements	4665

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4666

A.11.5.4.3.1	Test Purpose and Environment	4666

A.11.5.4.3.2	Test parameters	4666

A.11.5.4.3.3	Test Requirements	4669

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4670

A.11.5.4.4.1	Test Purpose and Environment	4670

A.11.5.4.4.2	Test parameters	4670

A.11.5.4.4.3	Test Requirements	4673

A.11.6	Measurement performance	4674

A.11.6.1	SS-RSRP	4674

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4674

A.11.6.1.1.1	Test Purpose and Environment	4674

A.11.6.1.1.2	Test parameters	4674

A.11.6.1.1.3	Test Requirements	4676

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4676

A.11.6.1.2.1	Test Purpose and Environment	4676

A.11.6.1.2.2	Test parameters	4676

A.11.6.1.2.3	Test Requirements	4678

A.11.6.2	SS-RSRQ	4678

A.11.6.2.1	Intra-frequency measurement accuracy	4678

A.11.6.2.1.1	Test Purpose and Environment	4678

A.11.6.2.1.2	Test Parameters	4678

A.11.6.2.1.3	Test Requirements	4681

A.11.6.2.2	Inter-frequency measurement accuracy	4681

A.11.6.2.2.1	Test Purpose and Environment	4681

A.11.6.2.2.2	Test Parameters	4681

A.11.6.2.2.3	Test Requirements	4684

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4684

A.11.6.2.3.1	Test Purpose and Environment	4684

A.11.6.2.3.2	Test Parameters	4684

A.11.6.2.3.3	Test Requirements	4687

A.11.6.2.4	Inter-frequency measurement accuracy	4687

A.11.6.2.4.1	Test Purpose and Environment	4687

A.11.6.2.4.2	Test Parameters	4687

A.11.6.2.4.3	Test Requirements	4695

A.11.6.3	SS-SINR	4695

A.11.6.3.1	Intra-frequency measurement accuracy	4695

A.11.6.3.1.1	Test Purpose and Environment	4695

A.11.6.3.1.2	Test Parameters	4695

A.11.6.3.1.3	Test Requirements	4699

A.11.6.3.2	Inter-frequency measurement accuracy	4699

A.11.6.3.2.1	Test Purpose and Environment	4699

A.11.6.3.2.2	Test Parameters	4699

A.11.6.3.2.3	Test Requirements	4702

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4702

A.11.6.3.3.1	Test Purpose and Environment	4702

A.11.6.3.3.2	Test Parameters	4702

A.11.6.3.3.3	Test Requirements	4705

A.11.6.3.4	Inter-frequency measurement accuracy	4705

A.11.6.3.4.1	Test Purpose and Environment	4705

A.11.6.3.4.2	Test Parameters	4705

A.11.6.3.4.3	Test Requirements	4713

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4714

A.11.6.4.1	SSB based L1-RSRP measurement	4714

A.11.6.4.1.1	Test Purpose and Environment	4714

A.11.6.4.1.2	Test parameters	4714

A.11.6.4.1.3	Test Requirements	4717

A.11.6.5	RSSI	4717

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4717

A.11.6.5.1.1	Test Purpose and Environment	4717

A.11.6.5.1.2	Test parameters	4717

A.11.6.5.1.3	Test Requirements	4720

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4720

A.11.6.5.2.1	Test Purpose and Environment	4720

A.11.6.5.2.2	Test parameters	4720

A.11.6.5.2.3	Test Requirements	4723

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4723

A.11.6.5.3.1	Test Purpose and Environment	4723

A.11.6.5.3.2	Test parameters	4723

A.11.6.5.3.3	Test Requirements	4726

A.11.6.6	Channel occupancy	4726

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4726

A.11.6.6.1.1	Test Purpose and Environment	4726

A.11.6.6.1.2	Test parameters	4726

A.11.6.6.1.3	Test Requirements	4729

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4729

A.11.6.6.2.1	Test Purpose and Environment	4729

A.11.6.6.2.2	Test parameters	4729

A.11.6.6.2.3	Test Requirements	4732

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4732

A.11.6.6.3.1	Test Purpose and Environment	4732

A.11.6.6.3.2	Test parameters	4732

A.11.6.6.3.3	Test Requirements	4735

A.11.6.7	E-UTRAN RSRP	4735

A.11.6.8	E-UTRAN RSRQ	4735

A.11.6.9	E-UTRAN SINR	4735

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4735

A.12.1	RRC_IDLE state mobility	4735

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4735

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4735

A.12.1.1.1.1	Test Purpose and Environment	4735

A.12.1.1.1.2	Test Requirements	4740

A.12.2	RRC_CONNECTED state mobility	4741

A.12.2.1	Handover	4741

A.12.2.1.1	E-UTRAN - NR with CCA handover	4741

A.12.2.1.1.1	Test Purpose and Environment	4741

A.12.2.1.1.2	Test Requirements	4746

A.12.3	Signalling characteristics	4747

A.12.3.1	Interruptions	4747

A.12.4	Measurement procedure	4747

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4747

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4747

A.12.4.1.1.1	Test Purpose and Environment	4747

A.12.4.1.1.2	Test Requirements	4751

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4751

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4751

A.12.4.2.1.1	Test Purpose and Environment	4751

A.12.4.2.1.2	Test Requirements	4755

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4756

A.12.4.2.2.1	Test Purpose and Environment	4756

A.12.4.2.2.2	Test Requirements	4760

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4761

A.12.4.2.3.1	Test Purpose and Environment	4761

A.12.4.2.3.2	Test Requirements	4765

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4766

A.12.4.2.4.1	Test Purpose and Environment	4766

A.12.4.2.4.2	Test Requirements	4770

A.12.4.2.5	RSSI measurement reporting	4771

A.12.4.2.5.1	Test purpose and environment	4771

A.12.4.2.5.2	Test parameters	4771

A.12.4.2.5.3	Test Requirements	4774

A.12.4.2.6	Channel occupancy measurement reporting	4774

A.12.4.2.6.1	Test purpose and environment	4774

A.12.4.2.6.2	Test parameters	4774

A.12.4.2.6.3	Test Requirements	4778

A.12.5	Measurement performance	4778

A.12.5.1	E-UTRANNR SFTD	4778

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4778

A.12.5.1.1.1	Test Purpose	4778

A.12.5.1.1.2	Test Environment	4778

A.12.5.1.1.3	Test Requirements	4784

A.12.5.2	E-UTRANNR SS-RSRP	4784

A.12.5.3	E-UTRANNR SS-RSRQ	4784

A.12.5.4	E-UTRANNR SS-SINR	4784

A.12.5.5	E-UTRANNR RSSI	4784

A.12.5.6	E-UTRANNR channel occupancy	4784

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4783

A.13.1	Timing	4783

A.13.1.1	UE transmit timing	4783

A.13.1.2	Timing advance	4783

A.13.2	Signalling characteristics	4783

A.13.2.1	Interruption	4783

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	4783

A.13.2.1.1.1	Test Purpose and Environment	4783

A.13.2.1.1.2	Test Requirements	4787

A.13.2.2	SCell activation and deactivation delay	4787

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4787

A.13.2.2.1.1	Test Purpose and Environment	4787

A.13.2.2.1.2	Test Requirements	4792

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4792

A.13.2.2.2.1	Test Purpose and Environment	4792

A.13.2.2.2.2	Test Requirements	4793

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4793

A.13.2.2.3.1	Test Purpose and Environment	4793

A.13.2.2.3.2	Test Requirements	4794

A.13.2.3	Void	4794

A.13.3	Measurement procedure	4794

A.13.3.1	Intra-frequency measurements	4794

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4794

A.13.3.1.1.1	Test purpose and environment	4794

A.13.3.1.1.2	Test parameters	4794

A.13.3.1.1.3	Test Requirements	4799

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4799

A.13.3.1.2.1	Test purpose and environment	4799

A.13.3.1.2.2	Test parameters	4799

A.13.3.1.2.3	Test Requirements	4804

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4804

A.13.3.1.3.1	Test purpose and environment	4804

A.13.3.1.3.2	Test parameters	4804

A.13.3.1.3.3	Test Requirements	4809

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4809

A.13.3.1.4.1	Test purpose and environment	4809

A.13.3.1.4.2	Test parameters	4809

A.13.3.1.4.3	Test Requirements	4814

A.13.3.1.5	RSSI measurement reporting on SCC	4814

A.13.3.1.5.1	Test purpose and environment	4814

A.13.3.1.5.2	Test parameters	4814

A.13.3.1.6	Channel occupancy measurement reporting on SCC	4815

A.13.3.1.6.1	Test purpose and environment	4815

A.13.3.1.6.2	Test parameters	4816

A.13.3.2	Inter-frequency measurements	4816

A.13.3.2.1	RSSI measurement reporting	4816

A.13.3.2.1.1	Test purpose and environment	4816

A.13.3.2.1.2	Test parameters	4816

A.13.3.2.2	Channel occupancy measurement reporting	4817

A.13.3.2.2.1	Test purpose and environment	4817

A.13.3.2.2.2	Test parameters	4817

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4817

A.13.3.2.3.1	Test Purpose and Environment	4817

A.13.3.2.3.2	Test Requirements	4822

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4822

A.13.3.2.4.1	Test Purpose and Environment	4822

A.13.3.2.4.2	Test Requirements	4827

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4828

A.13.3.2.5.1	Test Purpose and Environment	4828

A.13.3.2.5.2	Test Requirements	4833

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4833

A.13.3.2.6.1	Test Purpose and Environment	4833

A.13.3.2.6.2	Test Requirements	4839

A.13.3.3	L1-RSRP measurements for beam reporting	4840

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4840

A.13.3.3.1.1	Test Purpose and Environment	4840

A.13.3.3.1.2	Test parameters	4840

A.13.3.3.1.3	Test Requirements	4844

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4844

A.13.3.3.2.1	Test Purpose and Environment	4844

A.13.3.3.2.2	Test parameters	4844

A.13.3.3.2.3	Test Requirements	4848

A.13.4	Measurement performance	4848

A.13.4.1	SS-RSRP	4848

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4848

A.13.4.1.1.1	Test Purpose and Environment	4848

A.13.4.1.1.2	Test parameters	4848

A.13.4.1.1.3	Test Requirements	4850

A.13.4.2	SS-RSRQ	4850

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4850

A.13.4.2.1.1	Test Purpose and Environment	4850

A.13.4.2.1.2	Test Parameters	4850

A.13.4.2.1.3	Test Requirements	4857

A.13.4.3	SS-SINR	4857

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4857

A.13.4.3.1.1	Test Purpose and Environment	4857

A.13.4.3.1.2	Test Parameters	4857

A.13.4.3.1.3	Test Requirements	4865

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4865

A.13.4.4.1	SSB based L1-RSRP measurement	4865

A.13.4.4.1.1	Test Purpose and Environment	4865

A.13.4.4.1.2	Test parameters	4865

A.13.4.4.1.3	Test Requirements	4869

A.13.4.5	RSSI	4869

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4869

A.13.4.5.1.1	Test Purpose and Environment	4869

A.13.4.5.1.2	Test parameters	4869

A.13.4.5.1.3	Test Requirements	4873

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4873

A.13.4.5.2.1	Test Purpose and Environment	4873

A.13.4.5.2.2	Test parameters	4873

A.13.4.5.2.3	Test Requirements	4877

A.13.4.6	Channel occupancy	4877

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4877

A.13.4.6.1.1	Test Purpose and Environment	4877

A.13.4.6.1.2	Test parameters	4877

A.13.4.6.1.3	Test Requirements	4881

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4881

A.13.4.6.2.1	Test Purpose and Environment	4881

A.13.4.6.2.2	Test parameters	4881

A.13.4.6.2.3	Test Requirements	4885

A.14	NR standalone tests for Satellite access	4885

A.14.1	RRC_IDLE state mobility	4885

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4885

A.14.1.1.1	Test Purpose and Environment	4885

A.14.1.1.2	Test Parameters	4885

A.14.1.1.3	Test Requirements	4887

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4888

A.14.1.2.1	Test Purpose and Environment	4888

A.14.1.2.2	Test Parameters	4888

A.14.1.2.3	Test Requirements	4892

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4893

A.14.1.3.1	Test Purpose and Environment	4893

A.14.1.3.2	Test Parameters	4893

A.14.1.3.3	Test Requirements	4894

A.14.1.4	Location-based measurement initiation to FR1 intra-frequency NR cell reselection	4895

A.14.1.4.1	Test Purpose and Environment	4895

A.14.1.4.2	Test Parameters	4895

A.14.1.4.3	Test Requirements	4897

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4898

A.14.1.5.1	Test Purpose and Environment	4898

A.14.1.5.2	Test Parameters	4898

A.14.1.5.3	Test Requirements	4900

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4901

A.14.1.6.1	Test Purpose and Environment	4901

A.14.1.6.2	Test Parameters	4901

A.14.1.6.3	Test Requirements	4903

A.14.1.7	Time-based measurement initiation to FR1 inter-frequency cell reselection	4904

A.14.1.7.1	Test Purpose and Environment	4904

A.14.1.7.2	Test Parameters	4904

A.14.1.7.3	Test Requirements	4905

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4906

A.14.1.8.1	Test Purpose and Environment	4906

A.14.1.8.2	Test Parameters	4906

A.14.1.8.3	Test Requirements	4908

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4909

A.14.1.9.1	Test Purpose and Environment	4909

A.14.1.9.2	Test Parameters	4909

A.14.1.9.3	Test Requirements	4912

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4912

A.14.1.10.1	Test Purpose and Environment	4912

A.14.1.10.2	Test Parameters	4912

A.14.1.10.3	Test Requirements	4914

A.14.2	RRC_CONNECTED state mobility	4915

A.14.2.1	Handover	4915

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4915

A.14.2.1.1.1	Test Purpose and Environment	4915

A.14.2.1.1.2	Test Parameters	4915

A.14.2.1.1.3	Test Requirements	4917

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4918

A.14.2.1.2.1	Test Purpose and Environment	4918

A.14.2.1.2.2	Test Parameters	4918

A.14.2.1.2.3	Test Requirements	4920

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4920

A.14.2.1.3.1	Test Purpose and Environment	4920

A.14.2.1.3.2	Test Parameters	4920

A.14.2.1.3.3	Test Requirements	4923

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4923

A.14.2.1.4.1	Test Purpose and Environment	4923

A.14.2.1.4.2	Test Parameters	4923

A.14.2.1.4.3	Test Requirements	4925

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4925

A.14.2.1.5.1	Test Purpose and Environment	4925

A.14.2.1.5.2	Test Parameters	4926

A.14.2.1.5.3	Test Requirements	4929

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4929

A.14.2.1.6.1	Test Purpose and Environment	4929

A.14.2.1.6.2	Test Parameters	4929

A.14.2.1.6.3	Test Requirements	4933

A.14.2.2	RRC Connection Mobility Control	4933

A.14.2.2.1	SA: RRC Re-establishment for SAN	4933

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4933

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4936

A.14.2.2.2	Random Access	4939

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4939

A.14.2.2.2.1.1	Test Purpose and Environment	4939

A.14.2.2.2.1.2	Test Requirements	4942

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4943

A.14.2.2.2.2.1	Test Purpose and Environment	4943

A.14.2.2.2.2.2	Test Requirements	4947

A.14.2.2.3	RRC Connection Release with Redirection	4948

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4948

A.14.2.2.3.1.1	Test Purpose and Environment	4948

A.14.2.2.3.1.2	Test Parameters	4948

A.14.2.2.3.1.3	Test Requirements	4951

A.14.3	Timing for Satellite Access	4951

A.14.3.1	UE transmit timing for Satellite Access	4951

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4951

A.14.3.1.1.1	Test Purpose and environment	4951

A.14.3.1.1.2	Test requirements	4953

A.14.3.2	Timing advance for satellite access	4954

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4954

A.14.3.2.1.1	Test Purpose and Environment	4954

A.14.3.2.1.2	Test Parameters	4954

A.14.3.2.1.3	Test Requirements	4957

A.14.4	Signalling characteristics	4957

A.14.4.1	Radio link Monitoring	4957

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4957

A.14.4.1.1.1	Test Purpose and Environment	4957

A.14.4.1.1.2	Test Requirements	4962

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4962

A.14.4.1.2.1	Test Purpose and Environment	4962

A.14.4.1.2.2	Test Requirements	4968

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4968

A.14.4.1.3.1	Test Purpose and Environment	4968

A.14.4.1.3.2	Test Requirements	4972

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4972

A.14.4.1.4.1	Test Purpose and Environment	4972

A.14.4.1.4.2	Test Requirements	4977

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4977

A.14.4.1.5.1	Test Purpose and Environment	4977

A.14.4.1.5.2	Test Requirements	4982

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	4982

A.14.4.1.6.1	Test Purpose and Environment	4982

A.14.4.1.6.2	Test Requirements	4986

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4986

A.14.4.1.7.1	Test Purpose and Environment	4986

A.14.4.1.7.2	Test Requirements	4991

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	4991

A.14.4.1.8.1	Test Purpose and Environment	4991

A.14.4.1.8.2	Test Requirements	4996

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	4996

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	4996

A.14.4.2.1.1	Test Purpose and Environment	4996

A.14.4.2.1.2	Test Requirements	5002

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	5002

A.14.4.2.2.1	Test Purpose and Environment	5002

A.14.4.2.2.2	Test Requirements	5007

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	5007

A.14.4.2.3.1	Test Purpose and Environment	5007

A.14.4.2.3.2	Test Requirements	5013

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	5013

A.14.4.2.4.1	Test Purpose and Environment	5013

A.14.4.2.4.2	Test Requirements	5018

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	5018

A.14.4.2.5.1	Test Purpose and Environment	5018

A.14.4.2.5.2	Test Requirements	5023

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	5023

A.14.4.2.6.1	Test Purpose and Environment	5023

A.14.4.2.6.2	Test Requirements	5028

A.14.4.3	Active BWP switch for satellite access	5028

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	5028

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	5028

A.14.4.3.1.1.1	Test Purpose and Environment	5028

A.14.4.3.1.1.2	Test Requirements	5033

A.14.4.3.2	RRC-based Active BWP Switch	5033

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	5033

A.14.4.3.2.1.1	Test Purpose and Environment	5033

A.14.4.3.2.1.2	Test Requirements	5037

A.14.4.4	UE specific CBW change for satellite access	5037

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	5037

A.14.4.4.1.1	Test Purpose and Environment	5037

A.14.4.4.1.2	Test Requirements	5041

A.14.4.5	Pathloss reference signal switching delay	5041

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	5041

A.14.4.5.1.1	Test Purpose and Environment	5041

A.14.4.5.1.2	Test Requirements	5043

A.14.5	Measurement procedure	5044

A.14.5.1	Intra-frequency Measurements	5044

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	5044

A.14.5.1.1.1	Test purpose and Environment	5044

A.14.5.1.1.2	Test parameters	5044

A.14.5.1.1.3	Test Requirements	5046

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	5047

A.14.5.1.2.1	Test purpose and Environment	5047

A.14.5.1.2.2	Test parameters	5047

A.14.5.1.2.3	Test Requirements	5048

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	5049

A.14.5.1.3.1	Test purpose and Environment	5049

A.14.5.1.3.2	Test parameters	5049

A.14.5.1.3.3	Test Requirements	5051

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	5052

A.14.5.1.4.1	Test purpose and Environment	5052

A.14.5.1.4.2	Test parameters	5052

A.14.5.1.4.3	Test Requirements	5054

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	5055

A.14.5.1.5.1	Test purpose and Environment	5055

A.14.5.1.5.2	Test parameters	5055

A.15.5.1.5.3	Test Requirements	5057

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	5058

A.14.5.1.6.1	Test purpose and Environment	5058

A.14.5.1.6.2	Test parameters	5058

A.14.5.1.6.3	Test Requirements	5060

A.14.5.2	Inter-frequency Measurements	5061

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	5061

A.14.5.2.1.1	Test Purpose and Environment	5061

A.14.5.2.1.2	Test Requirements	5065

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	5065

A.14.5.2.2.1	Test Purpose and Environment	5065

A.14.5.2.2.2	Test Requirements	5069

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	5069

A.14.5.2.3.1	Test Purpose and Environment	5069

A.14.5.2.3.2	Test Requirements	5073

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	5073

A.14.5.2.4.1	Test Purpose and Environment	5073

A.14.5.2.4.2	Test Requirements	5077

A.14.5.2.5	void	5077

A.14.5.2.5.1	void	5077

A.14.5.2.5.2	void	5077

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	5077

A.14.5.2.6.1	Test Purpose and Environment	5077

A.14.5.2.6.2	Test Requirements	5081

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	5081

A.14.5.2.7.1	Test purpose and Environment	5081

A.14.5.2.7.2	Test parameters	5081

A.14.5.2.7.3	Test Requirements	5083

A.14.5.2.8	Event triggered reporting tests without gap under DRX	5084

A.14.5.2.8.1	Test purpose and Environment	5084

A.14.5.2.8.2	Test parameters	5084

A.14.5.2.8.3	Test Requirements	5085

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	5086

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	5086

A.14.5.3.1.1	Test Purpose and Environment	5086

A.14.5.3.1.2	Test parameters	5086

A.14.5.3.1.3	Test Requirements	5089

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	5089

A.14.5.3.2.1	Test Purpose and Environment	5089

A.14.5.3.2.2	Test parameters	5089

A.14.5.3.2.3	Test Requirements	5093

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	5093

A.14.5.3.3.1	Test Purpose and Environment	5093

A.14.5.3.3.2	Test parameters	5093

A.14.5.3.3.3	Test Requirements	5097

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	5097

A.14.5.3.4.1	Test Purpose and Environment	5097

A.14.5.3.4.2	Test parameters	5098

A.14.5.3.4.3	Test Requirements	5101

A.14.6	Measurement Performance requirements	5101

A.14.6.1	 SS-RSRP for SAN	5101

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5101

A.14.6.1.1.1	Test Purpose and Environment	5101

A.14.6.1.1.2	Test parameters	5101

A.14.6.1.1.3	Test Requirements	5105

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5105

A.14.6.1.2.1	Test Purpose and Environment	5105

A.14.6.1.2.2	Test parameters	5105

A.14.6.1.2.3	Test Requirements	5108

A.14.6.2	SS-RSRQ	5108

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5108

A.14.6.2.1.1	Test Purpose and Environment	5108

A.14.6.2.1.2	Test Parameters	5108

A.14.6.2.1.3	Test Requirements	5111

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5111

A.14.6.2.2.1	Test Purpose and Environment	5111

A.14.6.2.2.2	Test Parameters	5111

A.14.6.2.2.3	Test Requirements	5114

A.14.6.3	SS-SINR	5114

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5114

A.14.6.3.1.1	Test Purpose and Environment	5114

A.14.6.3.1.2	Test Parameters	5114

A.14.6.3.1.3	Test Requirements	5118

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5118

A.14.6.3.2.1	Test Purpose and Environment	5118

A.14.6.3.2.2	Test Parameters	5118

A.14.6.3.2.3	Test Requirements	5122

A.14.6.4	L1-RSRP measurement for beam reporting	5122

A.14.6.4.1	SSB based L1-RSRP measurement	5122

A.14.6.4.1.1	Test Purpose and Environment	5122

A.14.6.4.1.2	Test parameters	5122

A.14.6.4.1.3	Test Requirements	5125

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5126

A.14.6.4.2.1	Test Purpose and Environment	5126

A.14.6.4.2.2	Test parameters	5126

A.14.6.4.2.3	Test Requirements	5129

A.15	NR standalone tests with one or more NR cells in FR2-2	5122

A.15.1	SA: RRC_IDLE state mobility	5122

A.15.1.1	Cell re-selection to NR	5122

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	5122

A.15.1.1.1.1	Test Purpose and Environment	5122

A.15.1.1.1.2	Test Parameters	5122

A.15.1.1.1.3	Test Requirements	5126

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5126

A.15.1.2.1	Test Purpose and Environment	5126

A.15.1.2.2	Test Parameters	5126

A.15.1.2.3	Test Requirements	5130

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5130

A.15.1.3.1	Test Purpose and Environment	5130

A.15.1.3.2	Test Parameters	5130

A.15.1.3.3	Test Requirements	5133

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5133

A.15.1.4.1	Test Purpose and Environment	5133

A.15.1.4.2	Test Parameters	5133

A.15.1.4.3	Test Requirements	5136

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5136

A.15.1.5.1	Test Purpose and Environment	5136

A.15.1.5.2	Test Parameters	5136

A.15.1.5.3	Test Requirements	5140

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5140

A.15.1.6.1	Test Purpose and Environment	5140

A.15.1.6.2	Test Parameters	5140

A.15.1.6.3	Test Requirements	5143

A.15.2	Signaling characteristics	5144

A.15.2.1	SCell Activation and Deactivation Delay	5144

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5144

A.15.2.1.1.1	Test Purpose and Environment	5144

A.15.2.1.1.2	Test Requirements	5146

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5146

A.15.2.1.2.1	Test Purpose and Environment	5146

A.15.2.1.2.2	Test Requirements	5151

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5152

A.15.2.1.3.1	Test Purpose and Environment	5152

A.15.2.1.3.2	Test Requirements	5155

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5156

A.15.2.1.4.1	Test Purpose and Environment	5156

A.15.2.1.4.2	Test Requirements	5159

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5160

A.15.2.1.5.1	Test Purpose and Environment	5160

A.15.2.1.5.2	Test Requirements	5164

A.15.3	RRC_CONNECTED state mobility	5165

A.15.3.1	Handover	5165

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5165

A.15.3.1.1.1	Test Purpose and Environment	5165

A.15.3.1.1.2	Test Parameters	5165

A.15.3.1.1.3	Test Requirements	5169

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5169

A.15.3.1.2.1	Test Purpose and Environment	5169

A.15.3.1.2.2	Test Parameters	5169

A.15.3.1.2.3	Test Requirements	5173

A.15.4	Measurement procedure	5174

A.15.4.1	Intra-frequency Measurements	5174

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5174

A.15.4.1.1.1	Test purpose and Environment	5174

A.15.4.1.1.2	Test Requirements	5176

A.15.4.2	Inter-frequency Measurements	5177

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5177

A.15.4.2.1.1	Test Purpose and Environment	5177

A.15.4.2.1.2	Test Requirements	5180

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5181

A.16.1	SA: RRC_IDLE state mobility for RedCap	5181

A.16.1.1	Cell re-selection to NR	5181

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5181

A.16.1.1.1.1	Test Purpose and Environment	5181

A.16.1.1.1.2	Test Parameters	5181

A.16.1.1.1.3	Test Requirements	5185

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5185

A.16.1.1.2.1	Test Purpose and Environment	5185

A.16.1.1.2.2	Test Parameters	5185

A.16.1.1.2.3	Test Requirements	5189

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5189

A.16.1.1.3.1	Test Purpose and Environment	5189

A.16.1.1.3.2	Test Parameters	5189

A.16.1.1.3.3	Test Requirements	5194

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5194

A.16.1.1.4.1	Test Purpose and Environment	5194

A.16.1.1.4.2	Test Parameters	5194

A.16.1.1.4.3	Test Requirements	5199

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5199

A.16.1.1.5.1	Test Purpose and Environment	5199

A.16.1.1.5.2	Test Parameters	5199

A.16.1.1.5.3	Test Requirements	5204

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5204

A.16.1.1.6.1	Test Purpose and Environment	5204

A.16.1.1.6.2	Test Parameters	5204

A.16.1.1.6.3	Test Requirements	5209

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5209

A.16.1.1.7.1	Test Purpose and Environment	5209

A.16.1.1.7.2	Test Parameters	5209

A.16.1.1.7.3	Test Requirements	5214

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5214

A.16.1.1.8.1	Test Purpose and Environment	5214

A.16.1.1.8.2	Test Parameters	5215

A.16.1.1.8.3	Test Requirements	5219

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5220

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5220

A.16.1.2.1.1	Test Purpose and Environment	5220

A.16.1.2.1.2	Test Parameters	5220

A.16.1.2.1.3	Test Requirements	5223

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5224

A.16.1.2.2.1	Test Purpose and Environment	5224

A.16.1.2.2.2	Test Parameters	5224

A.16.1.2.2.3	Test Requirements	5227

A.16.1.2.3.1	Test Purpose and Environment	5228

A. 16.1.2.3.2	Test Parameters	5228

A.16.1.2.3.3	Test Requirements	5231

A.16.1.2.4.1	Test Purpose and Environment	5232

A.16.1.2.4.2	Test Parameters	5232

A.16.1.3.1.3	Test Requirements	5235

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5236

A.16.1.2.5.1	Test Purpose and Environment	5236

A.16.1.2.5.2	Test Parameters	5236

A.16.1.2.5.3	Test Requirements	5239

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5240

A.16.1.2.6.1	Test Purpose and Environment	5240

A.16.1.2.6.2	Test Parameters	5240

A.16.1.2.6.3	Test Requirements	5243

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5244

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5244

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5244

A.16.2.1.1.1	Test purpose and Environment	5244

A.16.2.1.1.2	Test Parameters	5245

A.16.2.1.1.3	Test requirements	5248

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5248

A.16.2.1.2.1	Test purpose and Environment	5248

A.16.2.1.2.2	Test Parameters	5250

A.16.2.1.2.3	Test requirements	5253

A.16.3	RRC_CONNECTED state mobility for RedCap	5253

A.16.3.1	Handover	5253

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5253

A.16.3.1.1.1	Test Purpose and Environment	5253

A.16.3.1.1.2	Test Parameters	5254

A.16.3.1.1.3	Test Requirements	5257

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5257

A.16.3.1.2.1	Test Purpose and Environment	5257

A.16.3.1.2.2	Test Parameters	5257

A.16.3.1.2.3	Test Requirements	5261

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5261

A.16.3.1.3.1	Test Purpose and Environment	5261

A.16.3.1.3.2	Test Parameters	5261

A.16.3.1.3.3	Test Requirements	5265

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5265

A.16.3.1.4.1	Test Purpose and Environment	5265

A.16.3.1.4.2	Test Parameters	5265

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5269

A.16.3.1.5.1	Test Purpose and Environment	5269

A.16.3.1.5.2	Test Parameters	5269

A.16.3.1.5.3	Test Requirements	5273

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5273

A.16.3.1.6.1	Test Purpose and Environment	5273

A.16.3.1.6.2	Test Parameters	5273

A.16.3.1.6.3	Test Requirements	5277

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5277

A.16.3.1.7.1	Test Purpose and Environment	5277

A.16.3.1.7.2	Test Requirements	5283

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5283

A.16.3.1.8.1	Test Purpose and Environment	5283

A.16.3.1.8.2	Test Requirements	5289

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5289

A.16.3.1.9.1	Test Purpose and Environment	5289

A.16.3.1.9.2	Test Requirements	5296

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5296

A.16.3.1.10.1	Test Purpose and Environment	5296

A.16.3.1.10.2	Test Requirements	5302

A.16.3.2	RRC Connection Mobility Control	5302

A.16.3.2.1	SA: RRC Re-establishment	5302

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5302

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5306

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5310

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5315

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5320

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5324

A.16.3.2.2	Random Access	5328

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5328

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5334

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5339

A.16.3.2.2.4	4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	5344

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5349

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5353

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5357

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5361

A.16.3.2.3	SA: RRC Connection Release with Redirection	5365

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5365

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5369

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5373

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5380

A.16.4	Timing for RedCap	5387

A.16.4.1	UE transmit timing	5387

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5387

A.16.4.1.1.1	Test Purpose and environment	5387

A.16.4.1.1.2	Test requirements	5390

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5390

A.16.4.1.2.1	Test Purpose and environment	5390

A.16.4.1.2.2	Test requirements	5393

A.16.4.2	UE timer accuracy	5393

A.16.4.3	Timing advance	5393

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5393

A.16.4.3.1.1	Test Purpose and Environment	5393

A.16.4.3.1.2	Test Parameters	5393

A.16.4.3.1.3	Test Requirements	5397

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5397

A.16.4.3.2.1	Test Purpose and Environment	5397

A.16.4.3.2.2	Test Parameters	5398

A.16.4.3.2.3	Test Requirements	5401

A.16.5	Signalling characteristics for RedCap	5402

A.16.5.1	Radio link Monitoring	5402

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5402

A.16.5.1.1.1	Test Purpose and Environment	5402

A.16.5.1.1.2	Test Requirements	5407

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5407

A.16.5.1.2.1	Test Purpose and Environment	5407

A.16.5.1.2.2	Test Requirements	5413

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5413

A.16.5.1.3.1	Test Purpose and Environment	5413

A.16.5.1.3.2	Test Requirements	5419

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5419

A.16.5.1.4.1	Test Purpose and Environment	5419

A.16.5.1.4.2	Test Requirements	5425

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5425

A.16.5.1.5.1	Test Purpose and Environment	5425

A.16.5.1.5.2	Test Requirements	5431

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5431

A.16.5.1.6.1	Test Purpose and Environment	5431

A.16.5.1.6.2	Test Requirements	5436

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5436

A.16.5.1.7.1	Test Purpose and Environment	5436

A.16.5.1.7.2	Test Requirements	5441

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5441

A.16.5.1.8.1	Test Purpose and Environment	5441

A.16.5.1.8.2	Test Requirements	5447

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5447

A.16.5.1.9.1	Test Purpose and Environment	5447

A.16.5.1.9.2	Test Requirements	5453

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5453

A.16.5.1.10.1	Test Purpose and Environment	5453

A.16.5.1.10.2	Test Requirements	5459

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5459

A.16.5.1.11.1	Test Purpose and Environment	5459

A.16.5.1.11.2	Test Requirements	5465

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5465

A.16.5.1.12.1	Test Purpose and Environment	5465

A.16.5.1.12.2	Test Requirements	5471

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5471

A.16.5.1.13.1	Test Purpose and Environment	5471

A.16.5.1.13.2	Test Requirements	5476

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5476

A.16.5.1.14.1	Test Purpose and Environment	5476

A.16.5.1.14.2	Test Requirements	5482

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5482

A.16.5.1.15.1	Test Purpose and Environment	5482

A.16.5.1.15.2	Test Requirements	5488

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5488

A.16.5.1.16.1	Test Purpose and Environment	5488

A.16.5.1.16.2	Test Requirements	5494

A.16.5.2	Beam Failure Detection and Link recovery procedures	5494

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5494

A.16.5.2.1.1	Test Purpose and Environment	5494

A.16.5.2.1.2	Test Requirements	5500

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5500

A.16.5.2.2.1	Test Purpose and Environment	5500

A.16.5.2.2.2	Test Requirements	5506

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5506

A.16.5.2.3.1	Test Purpose and Environment	5506

A.16.5.2.3.2	Test Requirements	5513

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5513

A.16.5.2.4.1	Test Purpose and Environment	5513

A.16.5.2.4.2	Test Requirements	5519

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5519

A.16.5.2.5.1	Test Purpose and Environment	5519

A.16.5.2.5.2	Test Requirements	5525

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5525

A.16.5.2.6.1	Test Purpose and Environment	5525

A.16.5.2.6.2	Test Requirements	5531

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5531

A.16.5.2.7.1	Test Purpose and Environment	5531

A.16.5.2.7.2	Test Requirements	5537

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5537

A.16.5.2.8.1	Test Purpose and Environment	5537

A.16.5.2.8.2	Test Requirements	5543

A.16.5.3	Active BWP switch	5543

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5543

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5543

A.16.5.3.1.1.1	Test Purpose and Environment	5543

A.16.5.3.1.1.2	Test Requirements	5548

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5548

A.16.5.3.1.2.1	Test Purpose and Environment	5548

A.16.5.3.1.2.2	Test Requirements	5553

A.16.5.3.2	RRC-based Active BWP Switch	5553

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5553

A.16.5.3.2.1.1	Test Purpose and Environment	5553

A.16.5.3.2.1.2	Test Requirements	5557

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5557

A.16.5.3.2.2.1	Test Purpose and Environment	5557

A.16.5.3.2.2.2	Test Requirements	5561

A.16.5.4	UE specific CBW change	5561

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5561

A.16.5.4.1.1	Test Purpose and Environment	5561

A.16.5.4.1.2	Test Requirements	5565

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5565

A.16.5.4.2.1	Test Purpose and Environment	5565

A.16.5.4.2.2	Test Requirements	5569

A.16.6	Measurement procedure for RedCap	5569

A.16.6.1	Intra-frequency Measurements	5569

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5569

A.16.6.1.1.1	Test purpose and Environment	5569

A.16.6.1.1.2	Test parameters	5569

A.16.6.1.1.3	Test Requirements	5573

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5573

A.16.6.1.2.1	Test purpose and Environment	5573

A.16.6.1.2.2	Test parameters	5573

A.16.6.1.2.3	Test Requirements	5577

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5577

A.16.6.1.3.1	Test purpose and Environment	5577

A.16.6.1.3.2	Test parameters	5577

A.16.6.1.3.3	Test Requirements	5581

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5581

A.16.6.1.4.1	Test purpose and Environment	5581

A.16.6.1.4.2	Test parameters	5581

A.16.6.1.4.3	Test Requirements	5585

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5585

A.16.6.1.5.1	Test purpose and Environment	5585

A.16.6.1.5.2	Test parameters	5585

A.16.6.1.5.3	Test Requirements	5589

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5589

A.16.6.1.6.1	Test purpose and Environment	5589

A.16.6.1.6.2	Test parameters	5589

A.16.6.1.6.3	Test Requirements	5593

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5593

A.16.6.1.7.1	Test purpose and Environment	5593

A.16.6.1.7.2	Test parameters	5593

A.16.6.1.7.3	Test Requirements	5597

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5597

A.16.6.1.8.1	Test purpose and Environment	5597

A.16.6.1.8.2	Test parameters	5597

A.16.6.1.8.3	Test Requirements	5601

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5601

A.16.6.1.9.1	Test purpose and Environment	5601

A.16.6.1.9.2	Test parameters	5601

A.16.6.1.9.3	Test Requirements	5603

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5604

A.16.6.1.10.1	Test purpose and Environment	5604

A.16.6.1.10.2	Test parameters	5604

A.16.6.1.10.3	Test Requirements	5605

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5606

A.16.6.1.11.1	Test purpose and Environment	5606

A.16.6.1.11.2	Test parameters	5606

A.16.6.1.11.3	Test Requirements	5608

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5609

A.16.6.1.12.1	Test purpose and Environment	5609

A.16.6.1.12.2	Test parameters	5609

A.16.6.1.12.3	Test Requirements	5611

A.16.6.2	Inter-frequency Measurements	5612

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5612

A.16.6.2.1.1	Test Purpose and Environment	5612

A.16.6.2.1.2	Test Requirements	5617

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5617

A.16.6.2.2.1	Test Purpose and Environment	5617

A.16.6.2.2.2	Test Requirements	5622

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5622

A.16.6.2.3.1	Test Purpose and Environment	5622

A.16.6.2.3.2	Test Requirements	5625

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5626

A.16.6.2.4.1	Test Purpose and Environment	5626

A.16.6.2.4.2	Test Requirements	5629

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5630

A.16.6.2.5.1	Test Purpose and Environment	5630

A.16.6.2.5.2	Test Requirements	5633

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5634

A.16.6.2.6.1	Test Purpose and Environment	5634

A.16.6.2.6.2	Test Requirements	5637

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5638

A.16.6.2.7.1	Test Purpose and Environment	5638

A.16.6.2.7.2	Test Requirements	5642

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5643

A.16.6.2.8.1	Test Purpose and Environment	5643

A.16.6.2.8.2	Test Requirements	5646

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5647

A.16.6.2.9.1	Test Purpose and Environment	5647

A.16.6.2.9.2	Test Requirements	5650

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5650

A.16.6.2.10.1	Test Purpose and Environment	5650

A.16.6.2.10.2	Test Requirements	5653

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5653

A.16.6.2.11.1	Test Purpose and Environment	5653

A.16.6.2.11.2	Test Requirements	5656

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5656

A.16.6.2.12.1	Test Purpose and Environment	5656

A.16.6.2.12.2	Test Requirements	5660

A.16.6.3	Inter-RAT Measurements	5661

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5661

A.16.6.3.1.1	Test purpose and Environment	5661

A.16.6.3.1.2	Test Requirements	5666

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5666

A.16.6.3.2.1	Test purpose and Environment	5666

A.16.6.3.2.2	Test Requirements	5671

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5671

A.16.6.3.3.1	Test purpose and Environment	5671

A.16.6.3.3.2	Test Requirements	5676

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5676

A.16.6.3.4.1	Test purpose and Environment	5676

A.16.6.3.4.2	Test Requirements	5681

A.16.6.4	L1-RSRP measurement for beam reporting	5681

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5681

A.16.6.4.1.1	Test Purpose and Environment	5681

A.16.6.4.1.2	Test parameters	5681

A.16.6.4.1.3	Test Requirements	5684

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5684

A.16.6.4.2.1	Test Purpose and Environment	5684

A.16.6.4.2.2	Test parameters	5685

A.16.6.4.2.3	Test Requirements	5688

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5688

A.16.6.4.3.1	Test Purpose and Environment	5688

A.16.6.4.3.2	Test parameters	5689

A.16.6.4.3.3	Test Requirements	5692

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5692

A.16.6.4.4.1	Test Purpose and Environment	5692

A.16.6.4.4.2	Test parameters	5693

A.16.6.4.4.3	Test Requirements	5696

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5696

A.16.6.4.5.1	Test Purpose and Environment	5696

A.16.6.4.5.2	Test parameters	5697

A.16.6.4.5.3	Test Requirements	5700

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5700

A.16.6.4.6.1	Test Purpose and Environment	5700

A.16.6.4.6.2	Test parameters	5701

A.16.6.4.6.3	Test Requirements	5704

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5704

A.16.6.4.7.1	Test Purpose and Environment	5704

A.16.6.4.7.2	Test parameters	5705

A.16.6.4.7.3	Test Requirements	5708

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5708

A.16.6.4.8.1	Test Purpose and Environment	5708

A.16.6.4.8.2	Test parameters	5709

A.16.6.4.8.3	Test Requirements	5712

A.16.6.5	NR measurements with autonomous gaps	5712

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5712

A.16.6.5.1.1	Test Purpose and Environment	5712

A.16.6.5.1.2	Test Parameters	5712

A.16.6.5.1.3	Test Requirements	5715

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5715

A.16.6.5.2.1	Test Purpose and Environment	5715

A.16.6.5.2.2	Test Parameters	5715

A.16.6.5.2.3	Test Requirements	5718

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5718

A.16.6.5.3.1	Test Purpose and Environment	5718

A.16.6.5.3.2	Test Requirements	5723

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5724

A.16.6.5.4.1	Test Purpose and Environment	5724

A.16.6.5.4.2	Test Requirements	5729

A.16.7	Measurement Performance requirements for RedCap	5730

A.16.7.1	SS-RSRP	5730

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5730

A.16.7.1.1.1	Test Purpose and Environment	5730

A.16.7.1.1.2	Test parameters	5730

A.16.7.1.1.3	Test Requirements	5735

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5735

A.16.7.1.2.1	Test Purpose and Environment	5735

A.16.7.1.2.2	Test parameters	5735

A.16.7.1.2.3	Test Requirements	5740

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5740

A.16.7.1.3.1	Test Purpose and Environment	5740

A.16.7.1.3.2	Test parameters	5740

A.16.7.1.3.3	Test Requirements	5744

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5745

A.16.7.1.4.1	Test Purpose and Environment	5745

A.16.7.1.4.2	Test parameters	5745

A.16.7.1.4.3	Test Requirements	5749

A.16.7.2	SS-RSRQ	5750

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5750

A.16.7.2.1.1	Test Purpose and Environment	5750

A.16.7.2.1.2	Test Parameters	5750

A.16.7.2.1.3	Test Requirements	5755

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5755

A.16.7.2.2.1	Test Purpose and Environment	5755

A.16.7.2.2.2	Test Parameters	5755

A.16.7.2.2.3	Test Requirements	5760

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5760

A.16.7.2.3.1	Test Purpose and Environment	5760

A.16.7.2.3.2	Test parameters	5760

A.16.7.2.3.3	Test Requirements	5765

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5765

A.16.7.2.4.1	Test Purpose and Environment	5765

A.16.7.2.4.2	Test parameters	5766

A.16.7.2.4.3	Test Requirements	5771

A.16.7.3	SS-SINR	5771

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5771

A.16.7.3.1.1	Test Purpose and Environment	5771

A.16.7.3.1.2	Test parameters	5772

A.16.7.3.1.3	Test Requirements	5776

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5777

A.16.7.3.2.1	Test Purpose and Environment	5777

A.16.7.3.2.2	Test parameters	5777

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5781

A.16.7.3.3.1	Test Purpose and Environment	5781

A.16.7.3.3.2	Test parameters	5782

A.16.7.3.3.3	Test Requirements	5787

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5787

A.16.7.3.4.1	Test Purpose and Environment	5787

A.16.7.3.4.2	Test parameters	5787

A.16.7.3.4.3	Test Requirements	5792

A.16.7.4	L1-RSRP measurement for beam reporting	5792

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5792

A.16.7.4.1.1	Test Purpose and Environment	5792

A.16.7.4.1.2	Test parameters	5793

A.16.7.4.1.3	Test Requirements	5797

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5797

A.16.7.4.2.1	Test Purpose and Environment	5797

A.16.7.4.2.2	Test parameters	5797

A.16.7.4.2.3	Test Requirements	5798

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5798

A.16.7.4.3.1	Test Purpose and Environment	5798

A.16.7.4.3.2	Test parameters	5798

A.16.7.4.3.3	Test Requirements	5802

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5802

A.16.7.4.4.1	Test Purpose and Environment	5802

A.16.7.4.4.2	Test parameters	5803

A.16.7.4.4.3	Test Requirements	5803

A.16.7.5	E-UTRAN RSRP	5803

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5803

A.16.7.5.1.1	Test Purpose and Environment	5803

A.16.7.5.1.2	Test parameters	5803

A.16.7.5.1.3	Test Requirements	5809

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5810

A.16.7.5.2.1	Test Purpose and Environment	5810

A.16.7.5.2.2	Test parameters	5810

A.16.7.5.2.3	Test Requirements	5816

A.16.7.6	E-UTRAN RSRQ	5817

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5817

A.16.7.6.1.1	Test Purpose and Environment	5817

A.16.7.6.1.2	Test parameters	5817

A.16.7.6.1.3	Test Requirements	5823

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5824

A.16.7.6.2.1	Test Purpose and Environment	5824

A.16.7.6.2.2	Test parameters	5824

A.16.7.6.2.3	Test Requirements	5830

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5837

A.17.1	SA: RRC_IDLE state mobility for RedCap	5837

A.17.1.1	Cell re-selection to NR	5837

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5837

A.17.1.1.1.1	Test Purpose and Environment	5837

A.17.1.1.1.2	Test Parameters	5837

A.17.1.1.1.3	Test Requirements	5841

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5841

A.17.1.1.2.1	Test Purpose and Environment	5841

A.17.1.1.2.2	Test Parameters	5841

A.17.1.1.2.3	Test Requirements	5845

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5845

A.17.1.1.3.1	Test Purpose and Environment	5845

A.17.1.1.3.2	Test Parameters	5845

A.17.1.1.3.3	Test Requirements	5848

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5848

A.17.1.1.4.1	Test Purpose and Environment	5848

A.17.1.1.4.2	Test Parameters	5848

A.17.1.1.4.3	Test Requirements	5852

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5852

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5852

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5852

A.17.2.1.1.1	Test Purpose and Environment	5852

A.17.2.1.1.2	Test Requirements	5856

A.17.3	RRC_CONNECTED state mobility for RedCap	5856

A.17.3.1	Handover for RedCap	5856

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5856

A.17.3.1.1.1	Test Purpose and Environment	5856

A.17.3.1.1.2	Test Parameters	5856

A.17.3.1.1.3	Test Requirements	5859

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5859

A.17.3.1.2.1	Test Purpose and Environment	5859

A.17.3.1.2.2	Test Parameters	5859

A.17.3.1.2.3 Test Requirements	5862

A.17.3.2	RRC Connection Mobility Control for RedCap	5862

A.17.3.2.1	SA: RRC Re-establishment	5862

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5862

A.17.3.2.1.1.1	Test Purpose and Environment	5862

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5865

A.17.3.2.1.2.1	Test Purpose and Environment	5865

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5868

A.17.3.2.1.3.1	Test Purpose and Environment	5868

A.17.3.2.1.3.2	Test Requirements	5870

A.17.3.2.2	Random Access	5871

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5871

A.17.3.2.2.1.1	Test Purpose and Environment	5871

A.17.3.2.2.1.2	Test Requirements	5873

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5875

A.17.3.2.2.2.1	Test Purpose and Environment	5875

A.17.3.2.2.2.2	Test Requirements	5877

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5879

A.17.3.2.2.3.1	Test Purpose and Environment	5879

A.17.3.2.2.3.2	Test Requirements	5881

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5882

A.17.3.2.2.4.1	Test Purpose and Environment	5882

A.17.3.2.2.4.2	Test Requirements	5884

A.17.3.2.3	SA: RRC Connection Release with Redirection	5885

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5885

A.17.3.2.3.1.1	Test Purpose and Environment	5885

A.17.3.2.3.1.2	Test Parameters	5885

A.17.3.2.3.1.3	Test Requirements	5889

A.17.4	Timing	5889

A.17.4.1	UE transmit timing	5889

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5889

A.17.4.1.1.1	Test Purpose and environment	5889

A.17.4.1.1.2	Test requirements	5892

A.17.4.2	UE timer accuracy	5893

A.17.4.3	Timing advance	5893

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5893

A.17.4.3.1.1	Test Purpose and Environment	5893

A.17.4.3.1.2	Test Parameters	5893

A.17.4.3.1.3 Test Requirements	5896

A.17.5	Signaling characteristics for RedCap	5896

A.17.5.1	Radio link Monitoring for RedCap	5896

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5896

A.17.5.1.1.1	Test Purpose and Environment	5896

A.17.5.1.1.2	Test Requirements	5899

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5900

A.17.5.1.2.1	Test Purpose and Environment	5900

A.17.5.1.2.2	Test Requirements	5904

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5905

A.17.5.1.3.1	Test Purpose and Environment	5905

A.17.5.1.3.2	Test Requirements	5908

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5908

A.17.5.1.4.1	Test Purpose and Environment	5908

A.17.5.1.4.2	Test Requirements	5913

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5913

A.17.5.1.5.1	Test Purpose and Environment	5913

A.17.5.1.5.2	Test Requirements	5918

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5918

A.17.5.1.6.1	Test Purpose and Environment	5918

A.17.5.1.6.2	Test Requirements	5922

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5922

A.17.5.1.7.1	Test Purpose and Environment	5922

A.17.5.1.7.2	Test Requirements	5926

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5926

A.17.5.1.8.1	Test Purpose and Environment	5926

A.17.5.1.8.2	Test Requirements	5931

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5931

A.17.5.1.9.1	Test Purpose and Environment	5931

A.17.5.1.9.2	Test Requirements	5934

A.17.5.2	Beam Failure Detection and Link recovery procedures	5934

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5934

A.17.5.2.1.1	Test Purpose and Environment	5934

A.17.5.2.1.2	Test Requirements	5939

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5940

A.17.5.2.2.1	Test Purpose and Environment	5940

A.17.5.2.2.2	Test Requirements	5944

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5945

A.17.5.2.3.1	Test Purpose and Environment	5945

A.17.5.2.3.2	Test Requirements	5949

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5949

A.17.5.2.4.1	Test Purpose and Environment	5949

A.17.5.2.4.2	Test Requirements	5953

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5953

A.17.5.2.5.1	Test Purpose and Environment	5953

A.17.5.2.5.2	Test Requirements	5957

A.17.5.3	Active BWP switch for RedCap	5957

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5957

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5957

A.17.5.3.1.1.1	Test Purpose and Environment	5957

A.17.5.3.1.1.2	Test Requirements	5960

A.17.5.3.2	RRC-based Active BWP Switch	5960

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5960

A.17.5.3.2.1.1	Test Purpose and Environment	5960

A.17.5.3.2.1.2	Test Requirements	5963

A.17.5.4	Active TCI state switch delay	5964

A.17.5.4.1	MAC-CE based active TCI state switch	5964

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5964

A.17.5.4.1.1.1	Test Purpose and Environment	5964

A.17.5.4.1.1.2	Test Requirements	5967

A.17.5.4.2	RRC based active TCI state switch	5967

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5967

A.17.5.4.2.1.1	Test Purpose and Environment	5967

A.17.5.4.2.1.2	Test Requirements	5971

A.17.5.5	Uplink spatial relation switch delay	5971

A.17.5.5.1	MAC-CE based Spatial Relation switch	5971

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5971

A.17.5.5.1.1.1	Test Purpose and Environment	5971

A.17.5.5.1.1.2	Test Requirements	5974

A.17.5.5.2	RRC based spatial relation switch	5974

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5974

A.17.5.5.2.1.2	Test Requirements	5977

A.17.5.6	UE specific CBW change	5977

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5977

A.17.5.6.1.1	Test Purpose and Environment	5977

A.17.5.6.1.2	Test Requirements	5980

A.17.6	Measurement procedure for RedCap	5981

A.17.6.1	Intra-frequency Measurements	5981

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5981

A.17.6.1.1.1	Test purpose and Environment	5981

A.17.6.1.1.2	Test Requirements	5984

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5984

A.17.6.1.2.1	Test purpose and Environment	5984

A.7.6.1.2.2	Test Requirements	5984

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5985

A.17.6.1.3.1	Test purpose and Environment	5985

A.17.6.1.3.2	Test Requirements	5989

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5989

A.17.6.1.4.1	Test purpose and Environment	5989

A.17.6.1.4.2	Test Requirements	5992

A.17.6.2	Inter-frequency Measurements	5993

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5993

A.17.6.2.1.1	Test Purpose and Environment	5993

A.17.6.2.1.2	Test Requirements	5996

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5996

A.17.6.2.2.1	Test Purpose and Environment	5996

A.17.6.2.2.2	Test Requirements	6000

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	6000

A.17.6.2.3.1	Test Purpose and Environment	6000

A.17.6.2.3.2	Test Requirements	6004

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	6004

A.17.6.2.4.1	Test Purpose and Environment	6004

A.17.6.2.4.2	Test Requirements	6008

A.17.6.3	L1-RSRP measurement for beam reporting	6009

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	6009

A.17.6.3.1.1	Test Purpose and Environment	6009

A.17.6.3.1.2	Test parameters	6009

A.17.6.3.1.3	Test Requirements	6009

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	6009

A.17.6.3.2.1	Test Purpose and Environment	6009

A.17.6.3.2.2	Test parameters	6009

A.17.6.3.2.3	Test Requirements	6011

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	6011

A.17.6.3.3.1	Test Purpose and Environment	6011

A.17.6.3.3.2	Test parameters	6012

A.17.6.3.3.3	Test Requirements	6014

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	6015

A.17.6.3.4.1	Test Purpose and Environment	6015

A.17.6.3.4.2	Test parameters	6015

A.7.6.3.3.3	Test Requirements	6017

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	6018

A.17.6.4.1.1	Test Purpose and Environment	6018

A.17.6.4.1.2	Test Requirements	6021

A.17.7	Measurement Performance requirements	6021

A.17.7.1	SS-RSRP	6021

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6021

A.17.7.1.1.1	Test Purpose and Environment	6021

A.17.7.1.1.2	Test parameters	6021

A.17.7.1.1.3	Test Requirements	6025

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6026

A.17.7.1.2.1	Test Purpose and Environment	6026

A.17.7.1.2.2	Test parameters	6026

A.17.7.1.2.3	Test Requirements	6030

A.17.7.2	SS-RSRQ	6031

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	6031

A.17.7.2.1.1	Test Purpose and Environment	6031

A.17.7.2.1.2	Test Parameters	6031

A.17.7.2.1.3	Test Requirements	6033

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	6033

A.17.7.2.2.1	Test Purpose and Environment	6033

A.17.7.2.2.2	Test parameters	6033

A.17.7.2.2.3	Test Requirements	6035

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	6035

A.17.7.3	L1-RSRP measurement for beam reporting	6035

A.17.7.3.1	SSB based L1-RSRP measurement	6035

A.17.7.3.1.1	Test Purpose and Environment	6035

A.17.7.3.1.2	Test parameters	6036

A.17.7.3.1.3	Test Requirements	6036

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	6036

A.17.7.3.2.1	Test Purpose and Environment	6036

A.17.7.3.2.2	Test parameters	6036

A.17.7.3.2.3	Test Requirements	6037

A.17.7.4	SS-SINR	6037

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	6037

A.17.7.4.1.1	Test Purpose and Environment	6037

A.17.7.4.1.2	Test parameters	6037

A.17.7.4.1.3	Test Requirements	6039

A.18	E-UTRA standalone tests for NR RRM for RedCap	6039

A.18.1	RRC_IDLE state mobility	6039

A.18.1.1	Inter-RAT NR Cell re-selection	6039

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	6039

A.18.1.1.1.1	Test Purpose and Environment	6039

A.18.2.1.1.2	Test Requirements	6044

A.18.2	RRC_CONNECTED state mobility	6044

A.18.2.1	Handover	6044

A.18.2.1.1	E-UTRAN - NR handover in FR1	6044

A.18.2.1.1.1	Test Purpose and Environment	6044

A.18.2.1.1.2	Test Requirements	6049

A.18.2.2	RRC connection release with redirection	6049

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	6049

A.18.2.2.1.1	Test Purpose and Environment	6049

A.18.2.2.1.2	Test Parameters	6049

A.18.2.2.1.3	Test Requirements	6056

A.18.3	Measurement procedure	6056

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	6056

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	6056

A.18.3.1.1.1	Test Purpose and Environment	6056

A.18.3.1.1.2	Test Requirements	6062

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	6062

A.18.3.1.2.1	Test Purpose and Environment	6062

A.18.3.1.2.2	Test Requirements	6068

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	6068

A.18.3.1.3.1	Test Purpose and Environment	6068

A.18.3.1.3.2	Test Requirements	6074

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	6074

A.18.3.1.4.1	Test Purpose and Environment	6074

A.18.3.1.4.2	Test Requirements	6080

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	6080

A.18.3.1.5.1	Test Purpose and Environment	6080

A.18.3.1.5.2	Test Requirements	6082

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	6083

A.18.3.1.6.1	Test Purpose and Environment	6083

A.18.3.1.6.2	Test Requirements	6085

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	6086

A.18.3.1.7.1	Test Purpose and Environment	6086

A.18.3.1.7.2	Test Requirements	6088

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	6089

A.18.3.1.8.1	Test Purpose and Environment	6089

A.18.3.1.8.2	Test Requirements	6091

A.19	NR standalone tests for ATG	6092

A.19.1	RRC_IDLE state mobility	6092

A.19.1.1	Cell reselection to FR1 intra-frequency NR case	6092

A.19.1.1.1	Test Purpose and Environment	6092

A.19.1.1.2	Test Parameters	6092

A.19.1.1.3	Test Requirements	6093

A.19.1.2	Cell reselection to FR1 inter-frequency NR case	6094

A.19.1.2.1	Test Purpose and Environment	6094

A.19.1.2.2	Test Parameters	6094

A.19.1.2.3	Test Requirements	6097

A.19.1.3	Cell reselection to FR1 inter-frequency NR case for UE configured with hs-ATG-cellReselectionSet-r18	6097

A.19.1.3.1	Test Purpose and Environment	6097

A.19.1.3.2	Test Parameters	6097

A.19.1.2.3	Test Requirements	6101

A.19.2	RRC_CONNECTED state mobility	6101

A.19.2.1	Handover	6101

A.19.2.1.1	Intra-frequency handover from FR1 to FR1; known target cell	6101

A19.2.1.1.1	Test Purpose and Environment	6101

A.19.2.1.1.2	Test Parameters	6101

A.19.2.1.2.3	Test Requirements	6102

A.19.2.1.2	Inter-frequency handover from FR1 to FR1; unknown target cell	6102

A.19.2.1.2.1	Test Purpose and Environment	6102

A.19.2.1.2.2	Test Parameters	6102

A.19.2.1.2.3	Test Requirements	6103

A.19.2.2	Conditional Handover	6104

A.19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1	6104

A.19.2.2.1.1	Test Purpose and Environment	6104

A.19.2.2.1.2	Test Parameters	6104

A.19.2.2.1.3	Test Requirements	6108

A.19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1	6109

A.19.2.2.2.1	Test Purpose and Environment	6109

A.19.2.2.2.2	Test Parameters	6109

A.19.2.2.2.3	Test Requirements	6113

A.19.2.3	RRC Connection Mobility Control	6114

A.19.2.3.1	SA: RRC Re-establishment	6114

A.19.2.3.1.1	Intra-frequency RRC Re-establishment in FR1 for ATG	6114

A.19.2.3.1.1.1	Test Purpose and Environment	6114

A.19.2.3.1.1.2 Test Requirements	6115

A.19.2.3.1.2	Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG	6115

A.19.2.3.1.2.1	Test Purpose and Environment	6115

A.19.2.3.1.2.2	Test Requirements	6117

A.19.2.3.2	Random Access for ATG UE	6118

A.19.2.3.2.1.1	Test Purpose and Environment	6118

A.19.2.3.2.1.2	Test Requirements	6119

A.19.2.3.2.2.1	Test Purpose and Environment	6119

A.19.2.3.2.2.2	Test Requirements	6120

A.19.2.3.2.3	2-step RA type contention based random access test in FR1 for NR standalone	6120

A.19.2.3.2.3.1	Test Purpose and Environment	6120

A.19.2.3.2.3.2	Test Requirements	6120

A.19.2.3.2.4	2-step RA type non-contention based test in FR1 for NR standalone	6121

A.19.2.3.2.4.1	Test Purpose and Environment	6121

A.19.2.3.2.4.2	Test Requirements	6121

A.19.2.3.3.1.1	Test Purpose and Environment	6121

A.19.2.3.3.1.2	Test Parameters	6121

A.19.2.3.3.1.3	Test Requirements	6122

A.19.3	Timing	6123

A.19.3.1	UE transmit timing	6123

A.19.3.1.1	ATG UE Transmit Timing Test for FR1	6123

A.19.3.1.1.1	Test Purpose and environment	6123

A.19.3.1.1.2	Test requirements	6126

A.19.3.2	UE timer accuracy	6127

A.19.3.3	Timing advance	6127

A.19.3.3.1	SA FR1 timing advance adjustment accuracy	6127

A.19.3.3.1.1	Test Purpose and Environment	6127

A.19.3.3.1.2	Test Parameters	6127

A.19.3.3.1.3	Test Requirements	6128

A.19.4	Signalling characteristics	6128

A.19.4.1	Radio link Monitoring	6128

A.19.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6128

A.19.4.1.1.1	Test Purpose and Environment	6128

A.19.4.1.1.2	Test Requirements	6134

A.19.4.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6134

A.19.4.1.2.1	Test Purpose and Environment	6134

A.19.4.1.2.2	Test Requirements	6140

A.19.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	6140

A.19.4.1.3.1	Test Purpose and Environment	6140

A.19.4.1.3.2	Test Requirements	6146

A.19.4.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	6146

A.19.4.1.4.1	Test Purpose and Environment	6146

A.19.4.1.4.2	Test Requirements	6152

A.19.4.2	Beam Failure Detection and Link recovery procedures	6152

A.19.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	6152

A.19.4.2.1.1	Test Purpose and Environment	6152

A.19.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	6160

A.19.4.2.2.1	Test Purpose and Environment	6160

A.19.4.2.2.2	Test Requirements	6165

A.19.4.3	Active BWP switch	6166

A.19.4.3.1	DCI-based and Timer-based Active BWP Switch	6166

A.19.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	6166

A.19.4.3.2	RRC-based Active BWP Switch	6170

A.19.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	6170

A.19.4.4	UE specific CBW change	6175

A19.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	6175

A19.4.4.1.1	Test Purpose and Environment	6175

A.19.4.4.1.2	Test Requirements	6180

A.19.4.5	Pathloss reference signal switching delay	6180

A.19.4.5.1	MAC-CE based pathloss reference signal switch delay	6180

A.19.4.5.1.1	Test Purpose and Environment	6180

A.19.4.5.1.2	Test Requirements	6183

A.19.5	Measurement procedure	6184

A.19.5.1	Intra-frequency Measurements	6184

A.19.5.1.1	SA event triggered reporting tests without gap without SSB index reading under non-DRX	6184

A.19.5.1.1.1	Test purpose and Environment	6184

A.19.5.1.1.2	Test parameters	6184

A.19.5.1.1.3	Test Requirements	6185

A.19.5.1.2	SA event triggered reporting tests with per-UE gaps under non-DRX	6185

A.19.5.1.2.1	Test purpose and Environment	6185

A.19.5.1.2.2	Test parameters	6185

A.19.5.1.2.3	Test Requirements	6185

A.19.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	6186

A.19.5.1.3.1	Test purpose and Environment	6186

A.19.5.1.3.2	Test parameters	6186

A.19.5.1.3.3	Test Requirements	6186

A.19.5.1.4	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	6187

A.19.5.1.4.1	Test purpose and Environment	6187

A.19.5.1.4.2	Test parameters	6187

A.19.5.1.4.3	Test Requirements	6187

A.19.5.2	Inter-frequency Measurements	6188

A.19.5.2.1.2	Test parameters	6188

A.19.5.2.1.3	Test Requirements	6190

A.19.5.2.2.2	Test parameters	6190

A.19.5.2.3.2	Test parameters	6191

A.19.5.2.3.3	Test Requirements	6192

A.19.5.3	L1-RSRP measurement for beam reporting for ATG	6192

A.19.5.3.1	SSB based L1-RSRP measurement when DRX is not used	6192

A.19.5.3.1.1	Test Purpose and Environment	6192

A.19.5.3.1.2	Test parameters	6192

A.19.5.3.1.3	Test Requirements	6193

A.19.5.3.2	CSI-RS based L1-RSRP measurement when DRX is not used	6193

A.19.5.3.2.1	Test Purpose and Environment	6193

A.19.5.3.2.2	Test parameters	6193

A.19.5.3.2.3	Test Requirements	6193

A.19.5.4	L1-SINR measurement for beam reporting for ATG	6194

A.19.5.4.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	6194

A.19.5.4.1.3	Test Requirements	6194

A.19.5.4.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	6194

A.19.5.4.2.1	Test Purpose and Environment	6194

A.19.5.4.2.2	Test parameters	6195

A.19.5.4.2.3	Test Requirements	6195

A.19.5.4.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	6195

A.19.5.4.3.1	Test Purpose and Environment	6195

A.19.5.4.3.2	Test parameters	6196

A.19.5.4.3.3	Test Requirements	6196

A.19.5.5	NR measurements with autonomous gaps for ATG	6196

A.19.5.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	6196

A.19.5.5.1.1	Test Purpose and Environment	6196

A.19.5.5.1.2	Test Parameters	6196

A.19.5.5.1.3	Test Requirements	6197

A.19.6	Measurement Performance requirements	6197

A.19.6.1	SS-RSRP for ATG UE	6197

A.19.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6197

A.19.6.1.1.1	Test Purpose and Environment	6197

A.19.6.1.1.2	Test parameters	6197

A.19.6.1.1.3	Test Requirements	6198

A.19.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6198

A.19.6.1.2.1	Test Purpose and Environment	6198

A.19.6.1.2.2	Test parameters	6198

A.19.6.1.2.3	Test Requirements	6199

A.19.6.2	SS-RSRQ for ATG UE	6199

A.19.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6199

A.19.6.2.1.1	Test Purpose and Environment	6199

A.19.6.2.1.2	Test Parameters	6199

A.19.6.2.1.3	Test Requirements	6199

A.19.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6200

A.19.6.2.2.1	Test Purpose and Environment	6200

A.19.6.2.2.2	Test Parameters	6200

A.19.6.2.2.3	Test Requirements	6200

A.19.6.3	SS-SINR for ATG UE	6200

A.19.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6200

A.19.6.3.1.1	Test Purpose and Environment	6200

A.19.6.3.1.2	Test Parameters	6201

A.19.6.3.1.3	Test Requirements	6201

A.19.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6201

A.19.6.3.2.1	Test Purpose and Environment	6201

A.19.6.3.2.2	Test Parameters	6201

A.19.6.3.2.3	Test Requirements	6202

A.19.6.4	L1-RSRP measurement for beam reporting for ATG UE	6202

A.19.6.4.1	SSB based L1-RSRP measurement	6202

A.19.6.4.1.1	Test Purpose and Environment	6202

A.19.6.4.1.2	Test parameters	6202

A.19.6.4.1.3	Test Requirements	6203

A.19.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	6203

A.19.6.4.2.1	Test Purpose and Environment	6203

A.19.6.4.2.2	Test parameters	6203

A.19.6.4.2.3	Test Requirements	6203

A.19.6.5	L1-SINR measurement for beam reporting based CMR for ATG UE	6204

A.19.6.5.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	6204

A.19.6.5.1.1	Test Purpose and Environment	6204

A.19.6.5.1.2	Test parameters	6204

A.19.6.5.1.3	Test Requirements	6204

A.19.6.5.2	L1-SINR measurement with SSB based CMR and dedicated IMR	6205

A.19.6.5.2.1	Test Purpose and Environment	6205

A.19.6.5.2.2	Test parameters	6205

A.19.6.5.2.3	Test Requirements	6205

A.19.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	6205

A.19.6.5.3.1	Test Purpose and Environment	6205

A.19.6.5.3.2	Test parameters	6206

A.19.6.5.3.3	Test Requirements	6206

A.19.6.6	CSI-RSRP for ATG UE	6206

A.19.6.6.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6206

A.19.6.6.1.1	Test Purpose and Environment	6206

A.19.6.6.1.2	Test parameters	6206

A.19.6.6.1.3	Test Requirements	6207

A.19.6.6.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6207

A.19.6.6.2.1	Test Purpose and Environment	6207

A.19.6.6.2.2	Test parameters	6207

A.19.6.6.2.3	Test Requirements	6208

A.19.6.7	CSI-RSRQ for ATG UE	6208

A.19.6.7.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6208

A.19.6.7.1.1	Test Purpose and Environment	6208

A.19.6.7.1.2	Test Parameters	6208

A.19.6.7.1.3	Test Requirements	6209

A.19.6.7.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6209

A.19.6.7.2.1	Test Purpose and Environment	6209

A.19.6.7.2.2	Test Parameters	6209

A.19.6.7.2.3	Test Requirements	6209

A.19.6.8	CSI-SINR for ATG UE	6210

A.19.6.8.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6210

A.19.6.8.1.1	Test Purpose and Environment	6210

A.19.6.8.1.2	Test Parameters	6210

A.19.6.8.1.3	Test Requirements	6210

A.19.6.8.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6210

A.19.6.8.2.1	Test Purpose and Environment	6210

A.19.6.8.2.2	Test Parameters	6210

A.19.6.8.2.3	Test Requirements	6211

Annex B (normative): Conditions for RRM requirements applicability for operating bands	6212

B.1	Conditions for NR RRC_IDLE state mobility	6212

B.1.1	Introduction	6212

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	6212

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	6213

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	6213

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	6215

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	6215

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	6216

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	6216

B.2.1	Introduction	6216

B.2.1.1	General	6216

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	6216

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	6216

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	6216

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	6217

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	6218

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	6218

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	6218

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	6219

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	6219

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	6219

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	6219

B.2.2	Conditions for NR intra-frequency measurements	6220

B.2.3	Conditions for NR inter-frequency measurements	6221

B.2.4	Conditions for NR L1-RSRP reporting	6222

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6222

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6094

B.2.5	Conditions for RRC connection release with redirection to NR	6095

B.2.6	Void	6096

B.2.6.1	Void	6096

B.2.6.2	Void	6096

B.2.7	Conditions for SRS-RSRP measurements	6096

B.2.8	Conditions for NR L1-SINR reporting	6097

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	6097

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	6098

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	6098

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	6098

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	6100

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	6100

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	6101

B.2.9	Conditions for NR intra-frequency measurements under CCA	6102

B.2.10	Conditions for NR inter-frequency measurements under CCA	6102

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6102

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6102

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	6103

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	6104

B.2.14	Conditions for NR PRS-based measurements	6106

B.2.15	Conditions for NR intra-frequency measurements for RedCap	6108

B.2.16	Conditions for NR inter-frequency measurements for RedCap	6109

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6110

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6111

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6111

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6111

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6111

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6112

B.3	RRM Requirements Exceptions	6112

B.3.1	Introduction	6112

B.3.2	Receiver sensitivity relaxation for CA	6112

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6112

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6112

B.3.2.2.1	Inter-band carrier aggregation	6112

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6112

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6113

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6113

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6113

B.3.2.4.1	Intra-band contiguous carrier aggregation	6113

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6113

B.3.3	Receiver sensitivity relaxation for DC	6113

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6113

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6113

B.3.4	Receiver sensitivity relaxation for SUL	6113

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6113

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6114

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6114

B.4	Conditions for V2X	6114

B.4.1	Test parameters for GNSS signals	6114

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6114

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6115

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6115

B.5	High level test procedure for SAN RRM tests	6115

Annex C (informative): Change history	6117