| 3GPP TS 38.133 V18.6.0 (2024-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	124

1	Scope	126

2	References	126

3	Definitions, symbols and abbreviations	127

3.1	Definitions	127

3.2	Symbols	129

3.3	Abbreviations	130

3.4	Test tolerances	133

3.5	Frequency bands grouping	133

3.5.1	Introduction	133

3.5.2	NR operating bands in FR1	133

3.5.2A	NR operating bands for satellite access in FR1	134

3.5.3	NR operating bands in FR2	135

3.6	Applicability of requirements in this specification version	136

3.6.1	RRC connected state requirements in DRX	136

3.6.2	Number of serving carriers	136

3.6.2.1	Number of serving carriers for SA	136

3.6.2.2	Number of serving carriers for EN-DC	136

3.6.2.3	Number of serving carriers for NE-DC	137

3.6.2.4	Number of serving carriers for NR-DC	137

3.6.3	Applicability for intra-band FR2	137

3.6.4	Applicability for FR2 UE power classes	137

3.6.5	Applicability for SDL bands	137

3.6.6	Applicability of requirements for NGEN-DC operation	137

3.6.7	Applicability of QCL	137

3.6.9	Applicability of requirements for scheduling availability	138

3.6.10	Applicability of requirements for measurement restrictions	139

3.6.11	Applicability of requirements for Redcap UEs	139

3.6.11.1	RRC connected state requirements in DRX	139

3.6.11.2	Applicability for FR2 Redcap UE power classes	139

3.6.11.3	Applicability of QCL	139

3.6.12	Applicability of requirements for Satellite Access	139

3.6.13	Applicability of requirements for FR2	139

3.6.14	Applicability of requirements for FR2 Power Class 6	139

3.6.15	Applicability of requirements for per-FR gap	140

3.6.16	Applicability of requirements for ATG	140

3.6.17	Applicability of requirements for MUSIM gaps	140

3.6.18	Applicability of requirements for a UE operating on a cell with less than 5MHz BW]	140

3.6.19	Applicability of requirements for multi-Rx operation in FR2-1	140

4	SA: RRC_IDLE state mobility	140

4.1	Cell Selection	140

4.2	Cell Re-selection	140

4.2.1	Introduction	140

4.2.2	Requirements	141

4.2.2.1	UE measurement capability	141

4.2.2.2	Measurement and evaluation of serving cell	141

4.2.2.3	Measurements of intra-frequency NR cells	143

4.2.2.4	Measurements of inter-frequency NR cells	147

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	152

4.2.2.6	Maximum interruption in paging reception	155

4.2.2.7	General requirements	156

4.2.2.8	Minimum requirement at transitions	156

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	156

4.2.2.9.1	Introduction	156

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	157

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	159

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	161

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	162

4.2.2.10.1	Introduction	162

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	162

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	165

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	168

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	168

4.2.2.11.1	Introduction	168

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	169

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	170

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	172

4.2.2.12	 Measurements of inter-frequency NR cells with NTN carrier	172

4.2A	Cell Re-selection when subject to CCA	175

4.2A.1	Introduction	175

4.2A.2	Requirements	176

4.2A.2.1	UE measurement capability	176

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	176

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	177

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	178

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	180

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	180

4.2A.2.7	General requirements	180

4.2B	Cell Re-selection for RedCap	181

4.2B.1	Introduction	181

4.2B.2	Requirements	181

4.2B.2.1	UE measurement capability for RedCap	181

4.2B.2.1.1	UE measurement capability for 1 Rx RedCap	181

4.2B.2.1.2	UE measurement capability for 2 Rx RedCap	181

4.2B.2.2	Measurement and evaluation of serving cell for RedCap UE	181

4.2B.2.3	Measurements of intra-frequency NR cells for RedCap UE	183

4.2B.2.4	Measurements of inter-frequency NR cells for RedCap UE	186

4.2B.2.5	Measurements of inter-RAT E-UTRAN cells for RedCap UE	189

4.2B.2.6	Maximum interruption in paging reception for RedCap	191

4.2B.2.7	General requirements for RedCap	191

4.2B.2.8	Minimum requirement at transitions	191

4.2B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion for RedCap	192

4.2B.2.9.1	Introduction	192

4.2B.2.9.2	Measurements for UE fulfilling stationary criterion	193

4.2B.2.9.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	196

4.2B.2.9.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criteria	196

4.2B.2.9.4	Measurements for a UE fulfilling low mobility and stationary criteria	197

4.2B.2.9.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	197

4.2B.2.9.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	197

4.2B.2.9.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	197

4.2B.2.9.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary criterion	197

4.2B.2.9.9	Measurements for UE fulfilling low mobility criterion	198

4.2B.2.9.10	Measurements for UE fulfilling not-at-cell edge criterion	201

4.2B.2.9.11	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	203

4.2B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	203

4.2B.2.10.1	Introduction	203

4.2B.2.10.2	Measurements for UE fulfilling stationary criterion	204

4.2B.2.10.3	Measurements for a UE fulfilling not-at-cell edge while stationary  criterion	206

4.2B.2.10.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	207

4.2B.2.10.4	Measurements for a UE fulfilling low mobility and stationary criteria	207

4.2B.2.10.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary criteria	208

4.2B.2.10.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	208

4.2B.2.10.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	208

4.2B.2.10.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	208

4.2B.2.10.9	Measurements for UE fulfilling low mobility criterion	209

4.2B.2.10.10	Measurements for UE fulfilling not-at-cell edge criterion	211

4.2B.2.10.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	214

4.2B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	214

4.2B.2.11.1	Introduction	214

4.2B.2.11.2	Measurements for UE fulfilling stationary criterion	215

4.2B.2.11.3	Measurements for a UE fulfilling not-at-cell edge while stationary criterion	216

4.2B.2.11.3A	Measurements for a UE fulfilling stationary and not-at-cell-edge criterion	217

4.2B.2.11.4	Measurements for a UE fulfilling low mobility and stationary criteria	217

4.2B.2.11.5	Measurements for a UE fulfilling low mobility and not-at-cell-edge while stationary  criteria	217

4.2B.2.11.6	Measurements for a UE fulfilling not-at-cell edge and not-at-cell edge while stationary criteria	217

4.2B.2.11.7	Measurements for a UE fulfilling low mobility and not-at-cell edge criteria and not-at-cell-edge while stationary criteria	218

4.2B.2.11.8	Measurements for a UE fulfilling low mobility, not-at-cell edge and stationary  criteria	218

4.2B.2.11.9	Measurements for UE fulfilling low mobility criterion	218

4.2B.2.11.10	Measurements for UE fulfilling with not-at-cell edge criterion	220

4.2B.2.11.11	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	221

4.2C	Cell Re-selection for NR UE for Satellite Access	221

4.2C.1	Introduction	221

4.2C.2	Requirements	222

4.2C.2.1	UE measurement capability	222

4.2C.2.2	Measurement and evaluation of serving cell	222

4.2C.2.3	Measurements of intra-frequency NR cells	223

4.2C.2.4	Measurements of inter-frequency NR cells	226

4.2C.2.5	Maximum interruption in paging reception	229

4.2C.2.6	Minimum requirement at transitions	229

4.2C.2.7	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	229

4.2C.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	229

4.2C.2.9	General requirements	230

4.2C.2.10	Measurements of inter-frequency NR cells with TN carrier	230

4.2C.2.11	Measurements of inter-RAT E-UTRAN cells with TN carrier	233

4.2C.3	Requirements from NTN to TN	234

4.2C.4	Requirements from TN to NTN	234

4.2D	Cell Re-selection for ATG	235

4.2D.1	Introduction	235

4.2D.2	Requirements	235

4.2D.2.1	UE measurement capability	235

4.2D.2.2	Measurement and evaluation of serving cell	235

4.2D.2.3	Measurements of intra-frequency NR cells	236

4.2D.2.4	Measurements of inter-frequency NR cells	237

4.2D.2.5	Maximum interruption in paging reception	239

4.2D.2.6	General requirements	239

4.3	Minimization of Drive Tests (MDT)	239

4.3.1	Introduction	239

4.3.2	Measurement Requirements	240

4.3.3	Requirements for Relative Time Stamp Accuracy	240

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	240

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	241

4.3C	Minimization of Drive Tests (MDT) for Satellite Access	241

4.3C.1	Introduction	241

4.3C.2	Measurement Requirements	241

4.3C.3	Requirements for Relative Time Stamp Accuracy	242

4.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	242

4.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	242

4.4	Idle Mode CA/DC Measurements	243

4.4.1	Introduction	243

4.4.2	Measurement Requirements	243

4.4.2.1	Detected cell requirement during state transition and Idle mode	243

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	243

4.4.2.3	Measurements on serving cell	245

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	245

4.5	NR measurements for positioning	245

4.5.1	Introduction	245

4.5.2	RSTD measurements	246

4.5.2.1	Introduction	246

4.5.2.2	Requirements Applicability	246

4.5.2.3	Measurement Capability	246

4.5.2.4	Measurement Reporting Requirements	246

4.5.2.5	Measurements Period Requirements	247

4.5.2.6	Measurements Period Requirements with Bandwidth Aggregation	250

4.5.3	PRS-RSRP measurements	253

4.5.3.1	Introduction	253

4.5.3.2	Requirements applicability	253

4.5.3.3	Measurement Capability	253

4.5.3.4	Measurement Reporting Requirements	254

4.5.3.5	Measurement Period Requirements	254

4.5.4	PRS-RSRPP measurements	256

4.5.4.1	Introduction	256

4.5.4.2	Requirements Applicability	257

4.5.4.3	Measurement Capability	257

4.5.4.4	Measurement Reporting Requirements	257

4.5.4.5	Measurement Period Requirements	257

4.5.5	Measurement requirements for DL RSCPD reported with RSTD	257

4.5.5.1	Introduction	257

4.5.5.2	Requirements Applicability	257

4.5.5.3	Measurement Capability	258

4.5.5.4	Measurement Reporting Requirements	258

4.5.5.5	Measurements Period Requirements	258

4.6	NR measurements for positioning for RedCap	262

4.6.1	Introduction	262

4.6.2	RSTD measurements for RedCap	262

4.6.2.1	Introduction	262

4.6.2.2	Requirements Applicability	262

4.6.2.3	Measurement Capability	262

4.6.2.4	Measurement Reporting Requirements	263

4.6.2.5	Measurements Period Requirements without RX FH	263

4.6.2.6	Measurement Period Requirements with RX FH	264

4.6.3	PRS-RSRP measurements for RedCap	265

4.6.3.1	Introduction	265

4.6.3.2	Requirements applicability	265

4.6.3.3	Measurement Capability	266

4.6.3.4	Measurement Reporting Requirements	266

4.6.3.5	Measurement Period Requirements without RX FH	266

4.6.3.6	Measurement Period Requirements with RX FH	269

4.6.4	PRS-RSRPP measurements for RedCap	270

4.6.4.1	Introduction	270

4.6.4.2	Requirements Applicability	270

4.6.4.3	Measurement Capability	270

4.6.4.4	Measurement Reporting Requirements	270

4.6.4.5	Measurement Period Requirements without RX FH	271

4.6.4.6	Measurement Period Requirements with RX FH	271

4.7	Measurement report for fast CA/DC setup	271

4.7.1	Introduction	271

4.7.2	Void	271

4.7.3	Measurement Report Requirements	271

5	SA: RRC_INACTIVE state mobility	273

5.1	Cell Re-selection	273

5.1.1	Introduction	273

5.1.2	Requirements	273

5.1.2.1	UE measurement capability	273

5.1.2.2	Measurement and evaluation of serving cell	273

5.1.2.3	Measurements of intra-frequency NR cells	275

5.1.2.4	Measurements of inter-frequency NR cells	278

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	280

5.1.2.6	Maximum interruption in paging reception	281

5.1.2.7	General requirements	281

5.1.2.8	Measurement of inter-frequency NR cells with NTN carrier	282

5.1.2.9	Minimum requirement at transitions	282

5.1.2.10	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	282

5.1.2.11	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	283

5.1.2.12	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	284

5.1A	Cell Re-selection with CCA	284

5.1A.1	Introduction	284

5.1A.2	Requirements	284

5.1A.2.1	UE measurement capability	284

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	284

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	285

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	285

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	285

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	285

5.1A.2.7	General requirements	285

5.1B	Cell Re-selection for RedCap	285

5.1B.1	Introduction	285

5.1B.2	Requirements	285

5.1B.2.1	UE measurement capability	285

5.1B.2.2	Measurement and evaluation of serving cell	285

5.1B.2.3	Measurements of intra-frequency NR cells	288

5.1B.2.4	Measurements of inter-frequency NR cells	290

5.1B.2.5	Measurements of inter-RAT E-UTRAN cells	292

5.1B.2.6	Maximum interruption in paging reception	293

5.1B.2.7	General requirements	293

5.1B.2.8	Minimum requirement at transitions	293

5.1B.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	293

5.1B.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	295

5.1B.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	297

5.1C	Cell Re-selection for Satellite Access	299

5.1C.1	Introduction	299

5.1C.2	Requirements	299

5.1C.2.1	UE measurement capability	299

5.1C.2.2	Measurement and evaluation of serving cell	299

5.1C.2.3	Measurements of intra-frequency NR cells	299

5.1C.2.4	Measurements of inter-frequency NR cells	299

5.1C.2.5	Maximum interruption in paging reception	299

5.1C.2.6	General requirements	299

5.1C.2.7	Measurements of inter-frequency NR cells with TN carrier	299

5.1C.2.8	Measurements of inter-RAT E-UTRAN cells with TN carrier	299

5.1C.3	Requirements from NTN to TN	299

5.1C.4	Requirements from TN to NTN	300

5.1D	Cell Re-selection for ATG	300

5.1D.1	Introduction	300

5.1D.2	Requirements	300

5.1D.2.1	UE measurement capability	300

5.1D.2.2	Measurement and evaluation of serving cell	300

5.1D.2.3	Measurements of intra-frequency NR cells	300

5.1D.2.4	Measurements of inter-frequency NR cells	300

5.1D.2.5	Maximum interruption in paging reception	300

5.1D.2.6	General requirements	300

5.2	Void	300

5.2B	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	300

5.2B.1	Introduction	300

5.2B.2	Requirements on UE synchronization for small data transmissions for RedCap	301

5.2B.2.1	Void5.2B.3 TA validation requirements for RedCap	301

5.2B.2.2	Void	302

5.2B.4	Scheduling restriction	302

5.2B.5	Applicability conditions for CG-SDT for RedCap	302

5.3	Minimization of Drive Tests (MDT)	302

5.3.1	Introduction	302

5.3.2	Measurement Requirements	303

5.3.3	Requirements for Relative Time Stamp Accuracy	303

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	303

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	303

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	303

5.3C	Minimization of Drive Tests (MDT) for Satellite Access	303

5.3C.1	Introduction	303

5.3C.2	Measurement Requirements	304

5.3C.3	Requirements for Relative Time Stamp Accuracy	304

5.3C.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	304

5.3C.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	304

5.3C.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	304

5.4	Inactive Mode CA/DC Measurements	304

5.4.1	Introduction	304

5.4.2	Measurement Requirements	304

5.4.2.1	Detected cell requirement during state transition and inactive mode	304

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	305

5.4.2.3	Measurements on serving cell	305

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	305

5.5	Configured Grant based Small Data Transmissions (CG-SDT)	305

5.5.1	Introduction	305

5.5.2	Requirements on UE synchronization for small data transmissions	305

5.5.3	TA validation requirements	305

5.5.4	Scheduling restriction	306

5.5.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	306

5.5.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	307

5.5.4.3	Scheduling availability of UE performing measurements on FR2	307

5.5.5	Applicability conditions for SDT	307

5.5D	Configured Grant based Small Data Transmissions (CG-SDT) for ATG	307

5.6	NR measurements for positioning	308

5.6.1	Introduction	308

5.6.1A	Cell re-selection for positioning	308

5.6.1A.1	Measurement and evaluation of serving cell	309

5.6.1A.2	Measurements of intra-frequency NR cells	310

5.6.2	RSTD measurements	311

5.6.2.1	Introduction	311

5.6.2.2	Requirements Applicability	311

5.6.2.3	Measurement Capability	311

5.6.2.5	Measurements Period Requirements	311

5.6.2.6	Measurements Period Requirements with Bandwidth Aggregation	314

5.6.3	PRS-RSRP measurements	318

5.6.3.1	Introduction	318

5.6.3.2	Requirements applicability	318

5.6.3.3	Measurement Capability	318

5.6.3.4	Measurement Reporting Requirements	318

5.6.3.5	Measurement Period Requirements	319

5.6.4	UE Rx-Tx time difference measurements	321

5.6.4.1	Introduction	321

5.6.4.2	Requirements Applicability	321

5.6.4.3	Measurement Capability	321

5.6.4.4	Measurement Reporting Requirements	322

5.6.4.5	Measurement Period Requirements	322

5.6.4.6	Measurement Period Requirements with Bandwidth Aggregation	325

5.6.5	PRS-RSRPP measurements	328

5.6.5.1	Introduction	328

5.6.5.2	Requirements Applicability	329

5.6.5.3	Measurement Capability	329

5.6.5.4	Measurement Reporting Requirements	329

5.6.5.5	Measurement Period Requirements	329

5.6.6	TA validation requirements for positioning	329

5.6.6.1	Introduction	329

5.6.6.2	TA validation requirements	329

5.6.6.3	TA validation requirements when configured with validity area	330

5.6.7	Measurement requirements for DL RSCPD reported with RSTD	331

5.6.7.1	Introduction	331

5.6.7.2	Requirements Applicability	331

5.6.7.3	Measurement Capability	331

5.6.7.4	Measurement Reporting Requirements	332

5.6.7.5	Measurements Period Requirements	332

5.6.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	333

5.6.8.1	Introduction	333

5.6.8.2	Requirements Applicability	333

5.6.8.3	Measurement Capability	333

5.6.8.4	Measurement Reporting Requirements	334

5.6.8.5	Measurement Period Requirements	334

5.6A	NR measurements for positioning for RedCap	336

5.6A.1	Introduction	336

5.6A.2	Cell re-selection for positioning	336

5.6A.2.1	Measurement and evaluation of serving cell	337

5.6A.2.2	Measurements of intra-frequency NR cells	337

5.6A.3	TA validation requirements for positioning SRS	338

5.6A.3.1	Introduction	338

5.6A.3.2	TA validation requirements	338

5.6A.3.3	TA validation requirements when configured with validity area	338

5.6A.4	RSTD measurements for RedCap	339

5.6A.4.1	 Introduction	339

5.6A.4.2	Requirements applicability	339

5.6A.4.3	Measurement Capability	339

5.6A.4.4	Measurement Reporting Requirements	339

5.6A.4.5	Measurement Period Requirement without RX FH	340

5.6A.4.6	Measurement Period Requirement with RX FH	343

5.6A.5	PRS-RSRP measurements for RedCap	343

5.6A.5.1	 Introduction	343

5.6A.5.2	Requirements applicability	343

5.6A.5.3	Measurement Capability	344

5.6A.5.4	Measurement Reporting Requirements	344

5.6A.5.5	Measurement Period Requirements without RX FH	344

5.6A.5.6	Measurement Period Requirement with RX FH	347

5.6A.6	UE Rx-Tx time difference measurements for RedCap	348

5.6A.6.1	Introduction	348

5.6A.6.2	Requirements Applicability	348

5.6A.6.3	Measurement Capability	348

5.6A.6.4	Measurement Reporting Requirements	348

5.6A.6.5	Measurement Period Requirements without RX FH	349

5.6A.6.6	Measurement Period Requirements with RX FH	349

5.6A.7	PRS-RSRPP measurements for RedCap	350

5.6A.7.1	 Introduction	350

5.6A.7.2	Requirements applicability	350

5.6A.7.3	Measurement Capability	350

5.6A.7.4	Measurement Reporting Requirements	350

5.6A.7.5	Measurement Period Requirements without FH	351

5.6A.7.6	Measurement period requirement with FH	351

5.7	Random access based Small Data Transmissions (RA-SDT)	351

5.7.1	Introduction	351

5.7.2	Requirements for small data transmissions based on 2-step RA	351

5.7.3	Requirements for small data transmissions based on 4-step RA	351

5.7.4	Applicability conditions for SDT	351

5.7B	Random access based Small Data Transmissions (RA-SDT) for RedCap	352

5.7B.1	Introduction	352

5.7B.2	Requirements for small data transmissions based on 2-step RA	352

5.7B.3	Requirements for small data transmissions based on 4-step RA	352

5.7B.4	Applicability conditions for RA-SDT for RedCap	352

5.7D	Random access based Small Data Transmissions (RA-SDT) for ATG	352

5.8	Measurement report for fast CA/DC setup	352

5.8.1	Introduction	352

5.8.2	Void	353

5.8.3	Measurement Report Requirements	353

6	RRC_CONNECTED state mobility	353

6.1	Handover	353

6.1.1	NR Handover	353

6.1.1.1	Introduction	353

6.1.1.2	NR FR1 - NR FR1 Handover	353

6.1.1.2.1	Handover delay	353

6.1.1.2.2	Interruption time	354

6.1.1.3	NR FR2- NR FR1 Handover	355

6.1.1.3.1	Handover delay	355

6.1.1.3.2	Interruption time	355

6.1.1.4	NR FR2- NR FR2 Handover	356

6.1.1.4.1	Handover delay	356

6.1.1.4.2	Interruption time	356

6.1.1.5	NR FR1- NR FR2 Handover	357

6.1.1.5.1	Handover delay	357

6.1.1.5.2	Interruption time	357

6.1.2	NR Handover to other RATs	358

6.1.2.1	NR – E-UTRAN Handover	358

6.1.2.1.1	Introduction	358

6.1.2.1.2	Handover delay	359

6.1.2.1.3	Interruption time	359

6.1.2.2	NR – UTRAN Handover	359

6.1.2.2.1	Introduction	359

6.1.2.2.2	Handover delay	359

6.1.2.2.3	Interruption time	360

6.1.3	NR DAPS Handover	360

6.1.3.1	Introduction	360

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	360

6.1.3.2.1	DAPS handover delay	361

6.1.3.2.2	Interruption time	362

6.1.3.3	NR FR2- NR FR1 DAPS Handover	363

6.1.3.3.1	DAPS handover delay	363

6.1.3.3.2	Interruption time	364

6.1.3.4	NR FR1- NR FR2 DAPS Handover	364

6.1.3.4.1	DAPS handover delay	365

6.1.3.4.2	Interruption time	365

6.1.4	NR Conditional Handover	365

6.1.4.1	Introduction	365

6.1.4.2	NR FR1 – NR FR1 conditional handover	366

6.1.4.2.2	Measurement time	366

6.1.4.3	NR FR2 – NR FR1 conditional handover	367

6.1.4.4	NR FR2 – NR FR2 conditional handover	368

6.1.4.4.1	Handover delay	368

6.1.4.4.2	Measurement time	368

6.1.4.4.3	Preparation time	369

6.1.4.4.4	Interruption time	369

6.1.4.5	NR FR1 – NR FR2 conditional handover	369

6.1.5	NR Handover with PSCell	370

6.1.5.1	Introduction	370

6.1.5.2	Handover with PSCell from NR SA to EN-DC	370

6.1.5.2.1	Interruption time for inter-RAT HO from NR to E-UTRAN	370

6.1.5.2.2	PSCell addition in HO with PSCell for NR SA to EN-DC	371

6.1.5.3	HO with PSCell from NE-DC to NE-DC	371

6.1.5.3.1	Handover delay	371

6.1.5.3.2	HO with PSCell - PCell Interruption time	371

6.1.5.3.3	PSCell addition/change in NE-DC to NE-DC HO with PSCell	372

6.1.5.4	HO with PSCell from NR-DC to NR-DC	372

6.1.5.5	Handover with PSCell from NR SA to EN-DC with PSCell using CCA	373

6.1.5.5.1	Introduction	373

6.1.5.5.2	NR SA to EN-DC HO with PSCell- NR to E-UTRA HO Interruption time	374

6.1.5.5.3	NR SA to EN-DC HO with PSCell - NR PSCell Addition Delay requirements	374

6.1.6.1	Conditional handover including target MCG in FR1 and target SCG in FR1 in NR-DC	375

6.1.6.1.1	CHO with PSCell – PCell Interruption time	376

6.1.6.1.2	CHO with PSCell – PSCell change delay	376

6.1.6.2	Conditional handover including target MCG in FR1 and target SCG in FR2 in NR-DC	377

6.1.6.2.2	CHO with PSCell – PSCell change delay	378

6.1.7.1.2	PSCell conditional change delay	380

6.1.7.2	Conditional handover including target MCG in FR1 and Candidate SCG for CPC in FR2 in NR-DC	381

6.1.7.2.1	PCell handover delay	382

6.1.7.2.1.1	Measurement time	382

6.1.7.2.2	PSCell conditional change delay	383

6.1A	Void	384

6.1A.1	Void	384

6.1A.1.1	Void	384

6.1A.1.2	Void	384

6.1A.1.2.1	Void	384

6.1A.1.2.2	Void	384

6.1B	Handover to target cell using CCA	384

6.1B.1	NR Handover	384

6.1B.1.1	Introduction	384

6.1B.1.2	NR FR1 - NR FR1 Handover	384

6.1B.1.2.1	Handover delay	384

6.1B.1.2.2	Interruption time	384

6.1B.1.3	NR FR2-2 NR FR2-2 Handover	385

6.1B.1.3.1	Handover delay	385

6.1B.1.3.2	Interruption time	385

6.1B.1.4	NR FR1- NR FR2-2 Handover	386

6.1B.1.4.1	Handover delay	386

6.1B.1.4.2	Interruption time	387

6.1C	Handover for SAN	388

6.1C.1	NR SAN Handover	388

6.1C.1.1	Introduction	388

6.1C.1.2	NR SAN FR1 – NR SAN FR1 Handover	388

6.1C.1.2.1	Handover delay	388

6.1C.1.2.2	Interruption time	388

6.1C.1.3	NR SAN FR2-NTN – NR SAN FR2-NTN Handover	389

6.1C.1.3.1	Handover delay	389

6.1C.1.3.2	Interruption time	390

6.1C.2	NR SAN Conditional Handover	390

6.1C.2.1	Introduction	390

6.1C.2.2	NR SAN FR1 – NR SAN FR1 conditional handover	390

6.1C.2.2.1	Handover delay	391

6.1C.2.2.2	Measurement time	391

6.1C.2.2.3	Preparation time	392

6.1C.2.2.4	Interruption time	393

6.1C.2.3	NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurement criteria	393

6.1C.2.3.1	Handover delay	393

6.1C.2.3.2	Preparation time	394

6.1C.2.3.3	Interruption time	394

6.1C.2.4	NR SAN FR2-NTN – NR SAN FR2-NTN conditional handover	394

6.1C.3	NR SAN Satellite switching with re-synchronization	395

6.1C.3.1	Introduction	395

6.1C.3.2	NR SAN FR1 – NR SAN FR1 Satellite switching with re-synchronization	395

6.1C.3.2.1	Satellite switching delay	395

6.1C.3.2.2	Interruption time for hard satellite switch with re-sync	395

6.1D	Handover for RedCap	397

6.1D.1	NR Handover	397

6.1D.1.1	Introduction	397

6.1D.1.2	NR FR1 - NR FR1 Handover	397

6.1D.1.2.1	Handover delay	397

6.1D.1.2.2	Interruption time	397

6.1D.1.3	NR FR2- NR FR2 Handover	399

6.1D.1.3.1	Handover delay	399

6.1D.1.3.2	Interruption time	399

6.1D.2	NR Handover to other RATs	400

6.1D.2.1	NR – E-UTRAN Handover	400

6.1E	Handover for ATG	400

6.1E.1	NR Handover	400

6.1E.1.1	Introduction	400

6.1E.1.2	NR FR1 - NR FR1 Handover	400

6.1E.1.2.1	Handover delay	401

6.1E.1.2.2	Interruption time	401

6.1E.2	NR Conditional Handover	402

6.1E.2.1	Introduction	402

6.1E.2.2	NR FR1 – NR FR1 conditional handover	402

6.1E.2.2.1	Handover delay	402

6.1E.2.2.2	Measurement time	402

6.1E.2.2.3	Preparation time	403

6.1E.2.2.4	Interruption time	403

6.2	RRC Connection Mobility Control	403

6.2.1	SA: RRC Re-establishment	403

6.2.1.1	Introduction	403

6.2.1.2	Requirements	403

6.2.1.2.1	UE Re-establishment delay requirement	404

6.2.1A	RRC Re-establishment with CCA	405

6.2.1A.1	Introduction	405

6.2.1A.2	Requirements	406

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	406

6.2.1B	SA: RRC Re-establishment for RedCap	408

6.2.1B.1	Introduction	408

6.2.1B.2	Requirements	408

6.2.2	Random access	408

6.2.2.1	Introduction	408

6.2.2.2	Requirements for 4-step RA type	409

6.2.2.2.1	Contention based random access	409

6.2.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	409

6.2.2.2.1.2	Correct behaviour when receiving Random Access Response	409

6.2.2.2.1.3	Correct behaviour when not receiving Random Access Response	409

6.2.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	409

6.2.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	409

6.2.2.2.1.6	Correct behaviour when contention Resolution timer expires	410

6.2.2.2.2	Non-Contention based random access	410

6.2.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	410

6.2.2.2.2.2	Correct behaviour when receiving Random Access Response	410

6.2.2.2.2.3	Correct behaviour when not receiving Random Access Response	411

6.2.2.2.3	UE behaviour when configured with supplementary UL	411

6.2.2.3	Requirements for 2-step RA type	411

6.2.2.3.1	Contention based random access	411

6.2.2.3.1.1	Correct behaviour when transmitting MsgA	411

6.2.2.3.1.2	Correct behaviour when receiving MsgB	412

6.2.2.3.1.3	Correct behaviour when not receiving MsgB	412

6.2.2.3.2	Non-Contention based random access	412

6.2.2.3.2.1	Correct behaviour when transmitting MsgA	412

6.2.2.3.2.2	Correct behaviour when receiving MsgB	412

6.2.2.3.2.3	Correct behaviour when not receiving MsgB	412

6.2.2.3.3	UE behaviour when configured with supplementary UL	413

6.2.2A	Random access when CCA is used on target frequency	413

6.2.2A.1	Introduction	413

6.2.2A.2	Requirements for 4-step RA type	413

6.2.2A.2.1	Contention based random access	413

6.2.2A.2.1.1	Correct behaviour when transmitting Random Access Preamble	413

6.2.2A.2.1.2	Correct behaviour when receiving Random Access Response	414

6.2.2A.2.1.3	Correct behaviour when not receiving Random Access Response	414

6.2.2A.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	414

6.2.2A.2.1.6	Correct behaviour when contention Resolution timer expires	414

6.2.2A.2.2	Non-Contention based random access	414

6.2.2A.2.2.1	Correct behaviour when transmitting Random Access Preamble	414

6.2.2A.2.2.2	Correct behaviour when receiving Random Access Response	415

6.2.2A.2.2.3	Correct behaviour when not receiving Random Access Response	415

6.2.2A.3	Requirements for 2-step RA type	415

6.2.2A.3.1	Contention based random access	416

6.2.2A.3.1.1	Correct behaviour when transmitting MsgA	416

6.2.2A.3.1.2	Correct behaviour when receiving MsgB	416

6.2.2A.3.1.3	Correct behaviour when not receiving MsgB	417

6.2.2A.3.2	Non-Contention based random access	417

6.2.2A.3.2.1	Correct behaviour when transmitting MsgA	417

6.2.2A.3.2.2	Correct behaviour when receiving MsgB	417

6.2.2A.3.2.3	Correct behaviour when not receiving MsgB	417

6.2.2B	Random access for RedCap	418

6.2.2B.1	Introduction	418

6.2.2B.2	Requirements	418

6.2.2C	PDCCH ordered Random Access for LTM	418

6.2.2C.1	Introduction	418

6.2.2C.2	PDCCH ordered Random Access delay	418

6.2.3	SA: RRC Connection Release with Redirection	419

6.2.3.1	Introduction	419

6.2.3.2	Requirements	419

6.2.3.2.1	RRC connection release with redirection to NR	419

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	420

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	421

6.2.3A	SA: RRC Connection Release with Redirection for RedCap	422

6.2.3A.1	Introduction	422

6.2.3A.2	Requirements	422

6.2.3A.2.1	RRC connection release with redirection to NR	422

6.2.3A.2.2	RRC connection release with redirection to E-UTRAN	422

6.2C	RRC Connection Mobility Control for Satellite Access	423

6.2C.1	SA: RRC Re-establishment for Satellite Access	423

6.2C.1.1	Introduction	423

6.2C.1.2	Requirements	423

6.2C.1.2.1	UE Re-establishment delay requirement	423

6.2C.1.2.2	UE Re-establishment delay requirement for VSAT	424

6.2C.2	Random access for satellite access	425

6.2C.2.1	Introduction	425

6.2C.2.2	Requirements for 4-step RA type	425

6.2C.2.2.1	Contention based random access	425

6.2C.2.2.1.1	Correct behaviour when transmitting Random Access Preamble	425

6.2C.2.2.1.2	Correct behaviour when receiving Random Access Response	425

6.2C.2.2.1.3	Correct behaviour when not receiving Random Access Response	425

6.2C.2.2.1.4	Correct behaviour when receiving an UL grant for msg3 retransmission	426

6.2C.2.2.1.5	SA: Correct behaviour when receiving a message over Temporary C-RNTI	426

6.2C.2.2.1.6	Correct behaviour when contention Resolution timer expires	426

6.2C.2.2.2	Non-Contention based random access	426

6.2C.2.2.2.1	Correct behaviour when transmitting Random Access Preamble	426

6.2C.2.2.2.2	Correct behaviour when receiving Random Access Response	426

6.2C.2.2.2.3	Correct behaviour when not receiving Random Access Response	427

6.2C.2.3	Requirements for 2-step RA type	427

6.2C.2.3.1	Contention based random access	427

6.2C.2.3.1.1	Correct behaviour when transmitting MsgA	427

6.2C.2.3.1.2	Correct behaviour when receiving MsgB	428

6.2C.2.3.1.3	Correct behaviour when not receiving MsgB	428

6.2C.2.3.2	Non-Contention based random access	428

6.2C.2.3.2.1	Correct behaviour when transmitting MsgA	428

6.2C.2.3.2.2	Correct behaviour when receiving MsgB	428

6.2C.2.3.2.3	Correct behaviour when not receiving MsgB	428

6.2C.3	SA: RRC Connection Release with Redirection for Satellite Access	429

6.2C.3.1	Introduction	429

6.2C.3.2	Requirements	429

6.2C.3.2.1	RRC connection release with redirection to NR	429

6.2D	RRC Connection Mobility Control for ATG	430

6.2D.1	SA: RRC Re-establishment	430

6.2D.1.1	Introduction	430

6.2D.1.2	Requirements	430

6.2D.1.2.1	UE Re-establishment delay requirement	430

6.2D.2	Random access	431

6.2D.2.1	Introduction	431

6.2D.2.2	Requirements for 4-step RA type	432

6.2D.2.3	Requirements for 2-step RA type	432

6.2D.3	SA: RRC Connection Release with Redirection	432

6.2D.3.1	Introduction	432

6.2D.3.2	Requirements	432

6.2D.3.2.1	RRC connection release with redirection to NR	432

6.3	L1/L2-Triggered Mobility	433

6.3.1	LTM PCell Cell Switch	433

6.3.1.1	Introduction	433

6.3.1.2	LTM Cell Switch delay	434

6.3.1.3	Interruption time	435

7	Timing	436

7.1	UE transmit timing	436

7.1.1	Introduction	436

7.1.2	Requirements	437

7.1.2.1	Gradual timing adjustment	439

7.1.2.2	Void	439

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE	439

7.1.2.4	UE transmit timing for positioning measurements	440

7.1A	UE transmit timing for RedCap	441

7.1A.1	Introduction	441

7.1A.2	Requirements	441

7.1A.2.1	Gradual timing adjustment	442

7.1C	UE transmit timing for Satellite Access	442

7.1C.1	Introduction	442

7.1C.2	Requirements	443

7.1C.2.1	Gradual timing adjustment	444

7.1D	UE transmit timing for ATG	445

7.1D.1	Introduction	445

7.1D.2	Requirements	445

7.1D.2.1	Gradual timing adjustment	446

7.2	UE timer accuracy	446

7.2.1	Introduction	446

7.2.2	Requirements	446

7.2A	UE timer accuracy for RedCap	447

7.2A.1	Introduction	447

7.2A.2	Requirements	447

7.2C	UE timer accuracy for satellite access	447

7.2C.1	Introduction	447

7.2C.2	Requirements	447

7.2D	UE timer accuracy for ATG	448

7.2D.1	Introduction	448

7.2D.2	Requirements	448

7.3	Timing advance	448

7.3.1	Introduction	448

7.3.2	Requirements	448

7.3.2.1	Timing Advance adjustment delay	448

7.3.2.2	Timing Advance adjustment accuracy	448

7.3A	Timing Advance for RedCap	449

7.3A.1	Introduction	449

7.3A.2	Requirements	449

7.3A.2.1	Timing Advance adjustment delay	449

7.3A.2.2	Timing Advance adjustment accuracy	449

7.3C	Timing advance for satellite access	449

7.3C.1	Introduction	449

7.3C.2	Requirements	449

7.3C.2.1	Timing Advance adjustment delay	449

7.3C.2.2	Timing Advance adjustment accuracy	450

7.3D	Timing advance for ATG	450

7.3D.1	Introduction	450

7.3D.2	Requirements	450

7.3D.2.1	Timing Advance adjustment delay	450

7.3D.2.2	Timing Advance adjustment accuracy	450

7.4	Cell phase synchronization accuracy	451

7.4.1	Definition	451

7.4.2	Minimum requirements	451

7.5	Maximum Transmission Timing Difference	451

7.5.1	Introduction	451

7.5.2	Minimum Requirements for inter-band EN-DC	451

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	452

7.5.3	Minimum Requirements for intra-band EN-DC	452

7.5.4	Minimum Requirements for NR Carrier Aggregation	453

7.5.5	Minimum Requirements for inter-band NE-DC	453

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	454

7.5.6	Minimum Requirements for inter-band NR DC	454

7.5.7	Minimum Requirements for multi-TRP	455

7.6	Maximum Receive Timing Difference	455

7.6.1	Introduction	455

7.6.2	Minimum Requirements for inter-band EN-DC	456

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	456

7.6.3	Minimum Requirements for intra-band EN-DC	457

7.6.4	Minimum Requirements for NR Carrier Aggregation	457

7.6.5	Minimum Requirements for inter-band NE-DC	458

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	459

7.6.6	Minimum Requirements for inter-band NR DC	459

7.6.7	Minimum Requirements for PC6 UE in FR2	460

7.6.8	Minimum Requirements for Multi-TRPs	460

7.7	deriveSSB-IndexFromCell tolerance	461

7.7.1	Minimum requirements	461

7.7A	deriveSSB-IndexFromCell tolerance for RedCap	461

7.7A.1	Minimum requirements	461

7.7D	DeriveSSB-IndexFromCell tolerance for ATG	462

7.7D.1	Minimum requirements	462

7.8	Void	462

7.9	deriveSSB-IndexFromCellInter-r17 tolerance	462

7.9.1	Minimum requirements	462

7.9D	DeriveSSB-IndexFromCellInter-r17 tolerance for ATG	462

7.9D.1	Minimum requirements	462

8	Signalling characteristics	429

8.1	Radio Link Monitoring	429

8.1.1	Introduction	429

8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria	430

8.1.2	Requirements for SSB based radio link monitoring	431

8.1.2.1	Introduction	431

8.1.2.2	Minimum requirement	432

8.1.2.3	Measurement restrictions for SSB based RLM	436

8.1.2.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	437

8.1.3	Requirements for CSI-RS based radio link monitoring	438

8.1.3.1	Introduction	438

8.1.3.2	Minimum requirement	439

8.1.3.3	Measurement restrictions for CSI-RS based RLM	442

8.1.3.4	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	444

8.1.4	Minimum requirement at transitions	444

8.1.5	Minimum requirement for UE turning off the transmitter	445

8.1.6	Minimum requirement for L1 indication	445

8.1.7	Scheduling availability of UE during radio link monitoring	445

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	446

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	446

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	446

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	447

8.1.8	Minimum requirement under IDC Interference	447

8.1A	Radio Link Monitoring with CCA on Target Frequency	447

8.1A.1	Introduction	447

8.1A.2	Requirements for SSB Based Radio Link Monitoring	448

8.1A.2.1	Introduction	448

8.1A.2.2	Minimum Requirement	449

8.1A.3	Minimum requirement at transitions	452

8.1A.4	Minimum requirement for UE turning off the transmitter	453

8.1A.5	Minimum requirement for L1 indication	453

8.1A.6	Scheduling availability of UE during radio link monitoring	453

8.1A.6.3	Scheduling availability of UE performing radio link monitoring on FR2-2	453

8.1A.6.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	454

8.1B	Radio Link Monitoring for RedCap	454

8.1B.1	Introduction	454

8.1B.2	Requirements for SSB based radio link monitoring	455

8.1B.2.1	Introduction	455

8.1B.2.2	Minimum requirement	456

8.1B.2.3	Measurement restrictions for SSB based RLM	458

8.1B.3	Requirements for CSI-RS based radio link monitoring	459

8.1B.3.1	Introduction	459

8.1B.3.2	Minimum requirement	459

8.1B.3.3	Measurement restrictions for CSI-RS based RLM	462

8.1B.4	Minimum requirement at transitions	463

8.1B.5	Minimum requirement for UE turning off the transmitter	463

8.1B.6	Minimum requirement for L1 indication	463

8.1B.7	Scheduling availability of UE during radio link monitoring	463

8.1B.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	464

8.1B.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	464

8.1B.7.3	Scheduling availability of UE performing radio link monitoring on FR2	464

8.1C	Radio Link Monitoring for Satellite Access	464

8.1C.1	Introduction	464

8.1C.2	Requirements for SSB based radio link monitoring	465

8.1C.2.1	Introduction	465

8.1C.2.2	Minimum requirement	466

8.1C.2.3	Measurement restrictions for SSB based RLM	467

8.1C.3	Requirements for CSI-RS based radio link monitoring	468

8.1C.3.1	Introduction	468

8.1C.3.2	Minimum requirement	468

8.1C.3.3	Measurement restrictions for CSI-RS based RLM	470

8.1C.4	Minimum requirement at transitions	470

8.1C.5	Minimum requirement for UE turning off the transmitter	470

8.1C.6	Minimum requirement for L1 indication	470

8.1C.7	Scheduling availability of UE during radio link monitoring	471

8.1C.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1 and [FR2-NTN]	471

8.1C.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1 and [FR2-NTN]	471

8.1D	Radio Link Monitoring for ATG	471

8.1D.1	Introduction	471

8.1D.2	Requirements for SSB based radio link monitoring	472

8.1D.2.1	Introduction	472

8.1D.2.2	Minimum requirement	473

8.1D.2.3	Measurement restrictions for SSB based RLM	474

8.1D.3	Requirements for CSI-RS based radio link monitoring	474

8.1D.3.1	Introduction	474

8.1D.3.2	Minimum requirement	474

8.1D.3.3	Measurement restrictions for CSI-RS based RLM	476

8.1D.4	Minimum requirement at transitions	476

8.1D.5	Minimum requirement for UE turning off the transmitter	476

8.1D.6	Minimum requirement for L1 indication	476

8.1D.7	Scheduling availability of UE during radio link monitoring	476

8.1D.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	476

8.1D.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	477

8.2	Interruption	477

8.2.1	EN-DC Interruption	477

8.2.1.1	Introduction	477

8.2.1.2	Requirements	478

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	478

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	478

8.2.1.2.3	Interruptions at SCell addition/release	478

8.2.1.2.4	Interruptions at SCell activation/deactivation	480

8.2.1.2.5	Interruptions during measurements on SCC	481

8.2.1.2.5.1	Interruptions during measurements on deactivated NR SCC	481

8.2.1.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	482

8.2.1.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCell	482

8.2.1.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	483

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	483

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	483

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	485

8.2.1.2.8.1	Interruptions during direct SCell activation and hibernation of E-UTRA SCell	485

8.2.1.2.8.2	Interruptions during direct SCell activation	485

8.2.1.2.9	Interruptions at SCell hibernation	485

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	486

8.2.1.2.11	Interruptions due to UE-specific CBW change	486

8.2.1.2.12	Interruptions at NR SRS carrier based switching	486

8.2.1.2.13	Interruptions at E-UTRA SRS carrier based switching	488

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	488

8.2.1.2.15	Interruptions due to SCell dormancy	489

8.2.1.2.15.1	Interruptions due to SCell dormancy switch	489

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy	489

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy	489

8.2.1.2.16	Interruptions when identifying CGI of an NR cell with autonomous gaps	489

8.2.1.2.17	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	490

8.2.1.2.18	Interruptions at NR SRS antenna port switching	491

8.2.1.2.19	Interruptions at fast SCell activation	492

8.2.1.2.20	Interruptions due to PUCCH SCell activation/deactivation	493

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	493

8.2.2.1	Introduction	493

8.2.2.2	Requirements	494

8.2.2.2.1	Interruptions at SCell addition/release	494

8.2.2.2.2	Interruptions at SCell activation/deactivation	495

8.2.2.2.3	Interruptions during measurements on deactivated SCC	496

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	497

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	498

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	499

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	500

8.2.2.2.8	Interruptions due to UE-specific CBW change	500

8.2.2.2.9	Interruptions at NR SRS carrier based switching	500

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	502

8.2.2.2.10A	DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors	503

8.2.2.2.10B	DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	503

8.2.2.2.10C	DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors	504

8.2.2.2.10D	DL Interruptions at UE switching across three or four uplink bands	504

8.2.2.2.11	Interruptions at direct SCell activation	505

8.2.2.2.12	Interruptions due to SCell dormancy	505

8.2.2.2.12.1	Interruptions due to SCell dormancy switch	505

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy	505

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy	506

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	506

8.2.2.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	506

8.2.2.2.15	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	506

8.2.2.2.16	Interruptions at NR SRS antenna port switching	507

8.2.2.2.17	Interruptions at fast SCell activation	508

8.2.2.2.18	Interruptions due to PUCCH SCell activation/deactivation	509

8.2.2.2.19	Interruptions due to measurements without gap carried out by UE supporting NeedForInterruptionInfoNR	509

8.2.2.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	510

8.2.3	NE-DC Interruptions	510

8.2.3.1	Introduction	510

8.2.3.2	Requirements	511

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	511

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	511

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	512

8.2.3.2.4	Interruptions at SCell activation/deactivation	513

8.2.3.2.5	Interruptions during measurements on SCC	514

8.2.3.2.5.1	Interruptions during measurements on deactivated NR SCC	514

8.2.3.2.5.2	Interruptions during measurements on deactivated E-UTRAN SCC	514

8.2.3.2.5.3	Interruptions during CQI measurements on dormant E-UTRAN SCC	515

8.2.3.2.5.4	Interruptions during RRM measurements on dormant E-UTRAN SCC	515

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	515

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	516

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	516

8.2.3.2.9	Interruptions at SCell hibernation	516

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	517

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	517

8.2.3.2.12	Interruptions at E-UTRA SRS carrier based switching	518

8.2.3.2.13	Interruptions due to SCell dormancy	519

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	520

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	520

8.2.3.2.17	Interruptions at fast SCell activation	522

8.2.3.2.18	Interruptions due to UE-specific CBW change	522

8.2.3.2.19	Interruptions due to PUCCH SCell activation/deactivation	523

8.2.4	NR-DC: Interruptions	523

8.2.4.1	Introduction	523

8.2.4.2	Requirements	524

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	524

8.2.4.2.2	Interruptions at SCell activation/deactivation	525

8.2.4.2.3	Interruptions during measurements on SCC	526

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	526

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	527

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	527

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	527

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	528

8.2.4.2.9	Interruptions at NR SRS carrier based switching	528

8.2.4.2.10	Interruptions at direct SCell activation	529

8.2.4.2.11	Interruptions when identifying CGI of an NR cell with autonomous gaps	530

8.2.4.2.12	Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	530

8.2.4.2.13	 Interruptions due to SCell dormancy	531

8.2.4.2.14	Interruptions at NR SRS antenna port switching	531

8.2.4.2.15	Interruptions at fast SCell activation	532

8.2.4.2.16	Interruptions at SCG activation/deactivation	533

8.2.4.2.17	Interruptions due to RRM measurements on deactivated SCG	533

8.2.4.2.18 Interruptions during RLM/BFD measurements on deactivated PScell	534

8.2.4.2.19	Interruptions due to UE-specific CBW change	534

8.2.4.2.20	Interruptions due to PDCCH ordered RACH on target LTM cell	534

8.2.4.2.21	Interruptions at PSCell Cell switch	534

8.2.4.2A	Void	535

8.2.4.2A.1	Void	535

8.2.4.2A.2	Void	535

8.2.4.2A.3	Void	535

8.3	SCell Activation and Deactivation Delay	535

8.3.1	Introduction	535

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	535

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	542

8.3.4	Direct SCell Activation at SCell addition	542

8.3.5	Direct SCell Activation at Handover	544

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	546

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	550

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	550

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	551

8.3.12	SCell Activation Delay Requirement for Deactivated PUCCH SCell	553

8.3.13	SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells	556

8.3.13.1	Introduction	556

8.3.14	SCell Deactivation Delay Requirement for Activated PUCCH SCell	558

8.3.15	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple Downlink SCells	559

8.3.16	Fast SCell Activation Delay Requirement for Deactivated SCell	559

8.3.17	SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation.	562

8.3.18	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting	565

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	568

8.3A.1	Introduction	568

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	568

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	573

8.4	UE UL carrier RRC reconfiguration delay	573

8.4.1	Introduction	573

8.4.2	UE UL carrier configuration delay requirement	573

8.4.3	UE UL carrier deconfiguration delay requirement	573

8.5	Link Recovery Procedures	574

8.5.1	Introduction	574

8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria	575

8.5.2	Requirements for SSB based beam failure detection	575

8.5.2.1	Introduction	575

8.5.2.2	Minimum requirement	576

8.5.2.3	Measurement restriction for SSB based beam failure detection	580

8.5.2.4	Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria	581

8.5.3	Requirements for CSI-RS based beam failure detection	582

8.5.3.1	Introduction	582

8.5.3.2	Minimum requirement	582

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	586

8.5.3.4	Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria	588

8.5.4	Minimum requirement for L1 indication	588

8.5.5	Requirements for SSB based candidate beam detection	589

8.5.5.1	Introduction	589

8.5.5.2	Minimum requirement	589

8.5.5.3	Measurement restriction for SSB based candidate beam detection	593

8.5.6	Requirements for CSI-RS based candidate beam detection	593

8.5.6.1	Introduction	593

8.5.6.2	Minimum requirement	594

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	597

8.5.7	Scheduling availability of UE during beam failure detection	598

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	598

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	598

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	599

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	600

8.5.8	Scheduling availability of UE during candidate beam detection	600

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	600

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	600

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	600

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	601

8.5.9	Requirements for Beam Failure Recovery in SCell	601

8.5.9.1	Introduction	601

8.5.9.2	Requirement	601

8.5.10	Minimum requirement at transitions for beam failure detection	602

8.5.11	Minimum requirement under IDC Interference	602

8.5A	Link Recovery Procedures when CCA is used on target frequency	602

8.5A.1	Introduction	602

8.5A.2	Requirements for SSB based beam failure detection	603

8.5A.2.1	Introduction	603

8.5A.2.2	Minimum requirement	603

8.5A.2.3	Measurement restriction for SSB based beam failure detection	606

8.5A.3	Void	606

8.5A.4	Minimum requirement for L1 indication	606

8.5A.5	Requirements for SSB based candidate beam detection	606

8.5A.5.1	Introduction	606

8.5A.5.2	Minimum requirement	606

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	609

8.5A.6	Void	610

8.5A.7	Scheduling availability of UE during beam failure detection	610

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	610

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	610

8.5A.7.3	Scheduling availability of UE performing beam failure detection on FR2-2	610

8.5A.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR DC	610

8.5A.8	Scheduling availability of UE during candidate beam detection	610

8.5A.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	610

8.5.8A.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2-2 in case of FR1-FR2-2 inter-band CA and NR-DC	610

8.5B	Link Recovery Procedures for Redcap	611

8.5B.1	Introduction	611

8.5B.2	Requirements for SSB based beam failure detection for Redcap	611

8.5B.2.1	Introduction	611

8.5B.2.2	Minimum requirement	612

8.5B.2.3	Measurement restriction for SSB based beam failure detection	613

8.5B.3	Requirements for CSI-RS based beam failure detection for Redcap	614

8.5B.3.1	Introduction	614

8.5B.3.2	Minimum requirement	614

8.5B.3.3	Measurement restrictions for CSI-RS beam failure detection	616

8.5B.4	Minimum requirement for L1 indication for Redcap	617

8.5B.5	Requirements for SSB based candidate beam detection for Redcap	618

8.5B.5.1	Introduction	618

8.5B.5.2	Minimum requirement	618

8.5B.5.3	Measurement restriction for SSB based candidate beam detection	620

8.5B.6	Requirements for CSI-RS based candidate beam detection for Redcap	620

8.5B.6.1	Introduction	620

8.5B.6.2	Minimum requirement	620

8.5B.6.3	Measurement restriction for CSI-RS based candidate beam detection	622

8.5B.7	Scheduling availability of UE during beam failure detection for Redcap	623

8.5B.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	623

8.5B.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	623

8.5B.7.3	Scheduling availability of UE performing beam failure detection on FR2	623

8.5B.8	Scheduling availability of UE during candidate beam detection for Redcap	624

8.5B.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	624

8.5B.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	624

8.5B.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	624

8.5B.9	Minimum requirement at transitions for beam failure detection for Redcap	624

8.5C	Link Recovery Procedures for Satellite Access	625

8.5C.1	Introduction	625

8.5C.2	Requirements for SSB based beam failure detection	625

8.5C.2.1	Introduction	625

8.5C.2.2	Minimum requirement	626

8.5C.2.3	Measurement restriction for SSB based beam failure detection	626

8.5C.3	Requirements for CSI-RS based beam failure detection	627

8.5C.3.1	Introduction	627

8.5C.3.2	Minimum requirement	627

8.5C.3.3	Measurement restrictions for CSI-RS beam failure detection	628

8.5C.4	Minimum requirement for L1 indication	629

8.5C.5	Requirements for SSB based candidate beam detection	629

8.5C.5.1	Introduction	629

8.5C.5.2	Minimum requirement	629

8.5C.5.3	Measurement restriction for SSB based candidate beam detection	630

8.5C.6	Requirements for CSI-RS based candidate beam detection	631

8.5C.6.1	Introduction	631

8.5C.6.2	Minimum requirement	631

8.5C.6.3	Measurement restriction for CSI-RS based candidate beam detection	632

8.5C.7	Scheduling availability of UE during beam failure detection	632

8.5C.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	632

8.5C.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	632

8.5C.8	Scheduling availability of UE during candidate beam detection	633

8.5C.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	633

8.5C.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	633

8.5C.9	Minimum requirement at transitions for beam failure detection	633

8.5D	Link Recovery Procedures for ATG	633

8.5D.1	Introduction	633

8.5D.2	Requirements for SSB based beam failure detection	634

8.5D.2.1	Introduction	634

8.5D.2.2	Minimum requirement	634

8.5D.2.3	Measurement restriction for SSB based beam failure detection	635

8.5D.3	Requirements for CSI-RS based beam failure detection	635

8.5D.3.1	Introduction	635

8.5D.3.2	Minimum requirement	636

8.5D.3.3	Measurement restrictions for CSI-RS beam failure detection	637

8.5D.4	Minimum requirement for L1 indication	637

8.5D.5	Requirements for SSB based candidate beam detection	637

8.5D.5.1	Introduction	637

8.5D.5.2	Minimum requirement	637

8.5D.5.3	Measurement restriction for SSB based candidate beam detection	638

8.5D.6	Requirements for CSI-RS based candidate beam detection	639

8.5D.6.1	Introduction	639

8.5D.6.2	Minimum requirement	639

8.5D.6.3	Measurement restriction for CSI-RS based candidate beam detection	640

8.5D.7	Scheduling availability of UE during beam failure detection	640

8.5D.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	640

8.5D.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	640

8.5D.8	Scheduling availability of UE during candidate beam detection	641

8.5D.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	641

8.5D.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	641

8.5D.9	Minimum requirement at transitions for beam failure detection	641

8.6	Active BWP switch delay	641

8.6.1	Introduction	641

8.6.2	DCI and timer based BWP switch delay on a single CC	641

8.6.2A	DCI based BWP switch delay on multiple CCs	643

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	643

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	644

8.6.2B	Timer based BWP switch delay on multiple CCs	645

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	645

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	645

8.6.3	RRC based BWP switch delay on a single CC	646

8.6.3A	RRC based BWP switch delay on multiple CCs	647

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	647

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	647

8.6.4	BWP switch delay on Consistent UL CCA recovery	648

8.6A	Active BWP switch delay for RedCap	648

8.6A.1	Introduction	648

8.6A.2	DCI and timer based BWP switch delay on a single CC	648

8.6A.3	RRC based BWP switch delay on a single CC	649

8.6C	Active BWP switch delay for satellite access	650

8.6C.1	Introduction	650

8.6C.2	DCI and timer based BWP switch delay on a single CC	650

8.6C.3	RRC based BWP switch delay on a single CC	652

8.6D	Active BWP switch delay for ATG	652

8.6D.1	Introduction	652

8.6D.2	DCI and timer based BWP switch delay	652

8.6D.3	RRC based BWP switch delay on a single CC	653

8.7	Void	654

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	654

8.8.1	Introduction	654

8.8.2	E-UTRAN PSCell Addition Delay Requirement	654

8.8.3	E-UTRAN PSCell Release Delay Requirement	654

8.9	NR-DC: PSCell Addition and Release Delay	655

8.9.1	Introduction	655

8.9.2	PSCell Addition Delay Requirement	655

8.9.3	PSCell Release Delay Requirement	656

8.9A	Conditional PSCell Addition Delay	656

8.9A.1	Introduction	656

8.9A.2	Conditional PSCell Addition Delay Requirement	656

8.9A.2.1	Measurement time	657

8.9B	NR-DC: PSCell Addition and Release Delay in Carriers with CCA	657

8.9B.1	Introduction	657

8.9B.2	PSCell Addition Delay Requirement	657

8.9B.3	PSCell Release Delay Requirement	658

8.9C	Subsequent Conditional PSCell Addition Delay	658

8.9C.1	Introduction	658

8.9C.2	Subsequent Conditional PSCell Addition Delay Requirement	658

8.9C.2.1	Measurement time	659

8.10	Active TCI state switching delay	659

8.10.3A	MAC-CE based TCI state switch delay in HST FR2 scenarios	660

8.10.4	DCI based TCI state switch delay	661

8.10.5	RRC based TCI state switch delay	661

8.10.6	Active TCI state list update delay	662

8.10A	Active TCI state switching delay with CCA	662

8.10A.1	Introduction	662

8.10A.2	Known conditions for TCI state	662

8.10A.3	MAC-CE based TCI state switch delay	663

8.10A.4	DCI based TCI state switch delay	664

8.10A.5	RRC based TCI state switch delay	664

8.10A.6	Active TCI state list update delay	665

8.10B	Active TCI state switching delay for RedCap	665

8.10B.1	Introduction	665

8.10B.2	Known conditions for TCI state	665

8.10B.3	MAC-CE based TCI state switch delay	665

8.10B.4	DCI based TCI state switch delay	666

8.10B.5	RRC based TCI state switch delay	666

8.10B.6	Active TCI state list update delay	667

8.10C	Active TCI state switching delay for satellite access	667

8.10C.1	Introduction	667

8.10C.2	MAC-CE based TCI state switch delay	667

8.10C.4	DCI based TCI state switch delay	668

8.10C.5	RRC based TCI state switch delay	668

8.10C.6	Active TCI state list update delay	668

8.10D	Active TCI state switching delay for ATG	668

8.10D.2	Void	668

8.10D.6	Active TCI state list update delay	669

8.10E	Active TCI state switching delay for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17	669

8.10E.1	Introduction	669

8.10E.2	Known conditions for TCI state	670

8.10E.3	MAC-CE based dual DL TCI state switch delay	670

8.10E.3.1	MAC-CE based dual DL TCI state switching delay for sDCI	670

8.10E.3.2	MAC-CE based dual DL TCI state switching delay for mDCI	670

8.10E.4	DCI based dual DL TCI state switch delay for sDCI and mDCI	671

8.10E.4.1	DCI based dual DL TCI state switching delay for sDCI	671

8.10E.4.2	DCI based dual DL TCI state switching delay for mDCI	671

8.10E.5	RRC based dual DL TCI state switch delay	671

8.10E.6	Active DL TCI state list update delay	671

8.10E.6.1	Active DL TCI state list update delay for sDCI	671

8.10E.6.2	Active DL TCI state list update delay for mDCI	672

8.11	PSCell Change	672

8.11A	PSCell Change in Carriers with CCA	672

8.11B	Conditional PSCell Change	673

8.11B.1	Introduction	673

8.11B.2	Conditoinal PSCell Change delay	673

8.11B.2.1	Measurement time	673

8.11D	Conditional PSCell Change in Carriers with CCA	674

8.11D.1	Introduction	674

8.11D.2	Conditional PSCell Change delay	674

8.11D.2.1	Measurement time	675

8.11E	Subsequent Conditional PSCell Change	675

8.11E.1	Introduction	675

8.11E.2	Subsequent Conditoinal PSCell Change delay	675

8.11E.2.1	Measurement time	676

8.12	Uplink spatial relation switch delay	676

8.12.1	Introduction	676

8.12.2	Known conditions for spatial relation when associated with DL-RS	677

8.12.3	MAC-CE based spatial relation switch delay	677

8.12.4	DCI based spatial relation switch delay	678

8.12.5	RRC based spatial relation switch delay	678

8.12A	Uplink spatial relation switch delay for RedCap	678

8.12A.1	Introduction	678

8.12A.2	Known conditions for spatial relation when associated with DL-RS	678

8.12A.3	MAC-CE based spatial relation switch delay	679

8.12A.4	DCI based spatial relation switch delay	680

8.12A.5	RRC based spatial relation switch delay	680

8.12C	Uplink spatial relation switch delay for satellite access	680

8.12C.1	Void	681

8.12C.2	Void	681

8.12C.3	Void	681

8.12C.4	Void	681

8.12C.5	Void	681

8.13	UE-specific CBW change	681

8.13.1	Introduction	681

8.13.2	UE-specific CBW change delay	681

8.13A	UE-specific CBW change for RedCap	681

8.13A.1	Introduction	681

8.13A.2	UE-specific CBW change delay	681

8.13C	UE-specific CBW change for satellite access	682

8.13C.1	Introduction	682

8.13C.2	UE-specific CBW change delay	682

8.13D	UE-specific CBW change for ATG	682

8.13D.1	Introduction	682

8.13D.2	UE-specific CBW change delay	683

8.14	Pathloss reference signal switching delay	683

8.14.1	Introduction	683

8.14.2	Known conditions for pathloss reference signal	683

8.14.3	MAC-CE based pathloss reference signal switch delay	683

8.14C	Pathloss reference signal switching delay for satellite access	684

8.14C.1	Introduction	684

8.14C.2	Known conditions for pathloss reference signal	684

8.14C.3	MAC-CE based pathloss reference signal switch delay	685

8.14D	Pathloss reference signal switching delay for ATG	685

8.14D.1	Introduction	685

8.14D.2	Known conditions for pathloss reference signal	685

8.14D.3	MAC-CE based pathloss reference signal switch delay	685

8.15	Active downlink TCI state switching delay for unified TCI	686

8.15.1	Introduction	686

8.15.4	DCI based downlink TCI state switch delay	687

8.15.5	Active Downlink TCI state list update delay	688

8.15D	Active downlink TCI state switching delay for unified TCI for ATG	689

8.15D.1	Introduction	689

8.15D.2	Void	689

8.15D.4	DCI based downlink TCI state switch delay	689

8.15D.5	Active Downlink TCI state list update delay	689

8.16	Active uplink TCI state switching delay for unified TCI	690

8.16.1	Introduction	690

8.16.4	DCI based uplink TCI state switch delay	692

8.16.5	Active Uplink TCI state list update delay	693

8.16D	Active uplink TCI state switching delay for unified TCI for ATG	694

8.16D.1	Introduction	694

8.16D.2	Void	694

8.16D.3	MAC-CE based uplink TCI state switch delay	694

8.16D.4	DCI based uplink TCI state switch delay	695

8.16D.5	Active Uplink TCI state list update delay	695

8.17	SCG Activation and Deactivation Delay	696

8.17.1	Introduction	696

8.17.2	SCG Activation Delay Requirement	696

8.17.3	SCG Deactivation Delay Requirement	697

8.18	TRP specific Link Recovery Procedures	698

8.18.1	Introduction	698

8.18.2	Requirements for TRP specific SSB based beam failure detection	698

8.18.2.1	Introduction	698

8.18.2.2	Minimum requirement	699

8.18.2.3	Measurement restriction for SSB based beam failure detection	701

8.18.3	Requirements for CSI-RS based beam failure detection	702

8.18.3.1	Introduction	702

8.18.3.2	Minimum requirement	702

8.18.3.3	Measurement restrictions for CSI-RS beam failure detection	705

8.18.4	Minimum requirement for L1 indication	707

8.18.5	Requirements for SSB based candidate beam detection	707

8.18.5.1	Introduction	707

8.18.5.2	Minimum requirement	707

8.18.5.3	Measurement restriction for SSB based candidate beam detection	710

8.18.6	Requirements for CSI-RS based candidate beam detection	710

8.18.6.1	Introduction	710

8.18.6.2	Minimum requirement	710

8.18.6.3	Measurement restriction for CSI-RS based candidate beam detection	713

8.18.7	Requirements for TRP specific Beam Failure Recovery	714

8.18.7.1	Introduction	714

8.18.7.2	Requirement	715

8.18.8	Scheduling availability of UE during TRP specific beam failure detection	715

8.18.8.1	Scheduling availability of UE performing TRP specific beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	715

8.18.8.2	Scheduling availability of UE performing TRP specific beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	715

8.18.8.3	Scheduling availability of UE performing TRP specific beam failure detection on FR2	715

8.18.8.4	Scheduling availability of UE performing TRP specific beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	716

8.18.9	Scheduling availability of UE during TRP specific candidate beam detection	716

8.18.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	717

8.18.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	717

8.18.9.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	717

8.18.9.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	718

8.19	Pre-configured measurement gap activation/deactivation delay	718

8.19.1	Introduction	718

8.19.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	718

8.19.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	718

8.19.3	Pre-configured measurement gap activation/deactivation upon SCell activation/deactivation	718

8.19.4	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	719

8.19.5	Activation/deactivation delay requirements for concurrent measurement gaps with Pre-MG	719

8.19.5.1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of concurrent measurement gaps with Pre-MGs	719

8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of concurrent measurement gaps with Pre-MGs	719

8.19.5.3	Pre-configured measurement gap activation/deactivation delay when colliding with a concurrent measurement gap	719

8.19D	Pre-configured measurement gap activation/deactivation delay for ATG	720

8.19D.1	Introduction	720

8.19D.2	Pre-configured measurement gap activation/deactivation upon DCI/timer-based BWP switch	720

8.19D.2.1	Activation/deactivation upon DCI/timer-based BWP switch delay on a single CC	720

8.19D.3	Pre-configured measurement gap activation/deactivation upon RRC reconfiguration	720

8.20	LTM PSCell Cell Switch	720

8.20.1	Introduction	720

8.20.2	LTM Cell Switch delay	721

8.20.3	Void	721

8.21	Active downlink TCI state switching delay for unified TCI for single-DCI mTRP	721

8.21.1	Introduction	721

8.21.2	Known conditions for downlink TCI state	721

8.21.3	MAC-CE based downlink TCI state switch delay	722

8.21.4	DCI based downlink TCI state switch delay	723

8.21.5	Active Downlink TCI state list update delay	723

8.22	Active downlink TCI state switching delay for unified TCI for multi-DCI mTRP	724

8.22.1	Introduction	724

8.22.2	Known conditions for downlink TCI state	725

8.22.3	MAC-CE based downlink TCI state switch delay	725

8.22.4	DCI based downlink TCI state switch delay	726

8.22.5	Active Downlink TCI state list update delay	726

8.23	Active uplink TCI state switching delay for unified TCI for single-DCI mTRP	727

8.23.1	Introduction	727

8.23.2	Known conditions for uplink TCI state	727

8.23.3	MAC-CE based uplink TCI state switch delay	727

8.23.4	DCI based uplink TCI state switch delay	729

8.23.5	Active uplink TCI state list update delay	729

8.24	Active uplink TCI state switching delay for unified TCI for multi-DCI mTRP	730

8.24.1	Introduction	730

8.24.2	Known conditions for uplink TCI state	731

8.24.3	MAC-CE based uplink TCI state switch delay	731

8.24.4	DCI based uplink TCI state switch delay	732

8.24.5	Active Uplink TCI state list update delay	732

8.25	TCI state activation for LTM candidate cell	734

8.25.1	Introduction	734

8.25.2	Known TCI state conditions	734

8.25.3	SSB based TCI state activation delay	734

9	Measurement Procedure	733

9.1	General measurement requirement	733

9.1.1	Introduction	733

9.1.2	Measurement gap	733

9.1.2.1	EN-DC: Measurement Gap Sharing	745

9.1.2.1a	SA: Measurement Gap Sharing	746

9.1.2.1b	NE-DC: Measurement Gap Sharing	747

9.1.2.1c	NR-DC: Measurement Gap Sharing	748

9.1.3	UE Measurement capability	749

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	749

9.1.3.1a	SA: Monitoring of multiple layers using gaps	749

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	750

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	750

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	751

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	752

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	752

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	753

9.1A.3.2	Void	754

9.1.3A	UE Measurement capability under operation mode with CCA	754

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	754

9.1.3A.1a	SA: Monitoring of multiple layers using gaps under CCA	754

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	754

9.1.3A.2a	SA: Maximum allowed layers for multiple monitoring under CCA	755

9.1.3C	UE Measurement capability under operation mode with satellite access	755

9.1.3C.1a	SA: Monitoring of multiple layers using gaps under satellite access	755

9.1.3C.2a	SA: Maximum allowed layers for multiple monitoring for SAN	756

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	756

9.1.4.1	Introduction	756

9.1.4.2	Requirements	756

9.1.5	Carrier-specific scaling factor	760

9.1.5.1	Monitoring of multiple layers outside gaps	761

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	764

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	765

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	766

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	767

9.1.5.2	Monitoring of multiple layers within gaps	768

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	771

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	773

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	775

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	777

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	779

9.1.5.3	Monitoring of multiple layers within NCSG	779

9.1.5.3.1	SA mode: carrier-specific scaling factor for measurements performed within NCSG	781

9.1.5.4	L1-RSRP measurements within measurement gap	781

9.1.5.4.1	SA mode: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	783

9.1.5.4.2	NR-DC: carrier-specific scaling factor for L1-RSRP measurements performed within measurement gap	784

9.1.6	Minimum requirement at transitions	786

9.1.7	Pre-configured measurement gap	786

9.1.7.1	Introduction	786

9.1.7.2	Requirements applicability	786

9.1.7.3	Requirements	787

9.1.7.3.1	Requirements for autonomous activation/deactivation mechanism	787

9.1.7.3.2	Requirements for network-controlled activation/deactivation mechanism	788

9.1.7.3.3	Requirements for reception/transmission during activation/deactivation	788

9.1.8	Concurrent measurement gaps	789

9.1.8.1	Introduction	789

9.1.8.2	Requirements	789

9.1.8.3	Collision between concurrent measurement gaps	790

9.1.8.4	Measurement gap related requirements of concurrent measurement gaps	791

9.1.9	Network controlled small gap	791

9.1.9.1	Introduction	791

9.1.9.2	Requirements applicability	792

9.1.10	MUSIM gaps	794

9.1.10.1	Introduction	795

9.1.10.2	Priorities for MUSIM gaps	796

9.1.10.3	Keep solution for MUSIM gaps	796

9.1.10.4	Collisions between different MUSIM gaps	796

9.1.10.5	Collisions between MUSIM gaps and measurement gaps	796

9.1.10.6	MUSIM gap related requirements	797

9.1.11	UL gap for Tx power management	797

9.1.12	Concurrent measurement gaps with Pre-MG	797

9.1.12.1	Introduction	797

9.1.12.2	Requirements	797

9.1.12.3	Collisions involving Pre-MG(s)	799

9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap	799

9.1.13	Concurrent measurement gaps with NCSG	799

9.1.13.1	Introduction	799

9.1.13.2	Requirements	800

9.1.13.3	Collision involving NCSGs	801

9.1A	General measurement requirement for RedCap	802

9.1A.1	Introduction	802

9.1A.2	Measurement gap	802

9.1A.2.1	SA: Measurement Gap Sharing	806

9.1A.3	UE Measurement capability	807

9.1A.3.1	SA: Monitoring of multiple layers using gaps	807

9.1A.3.2	SA: Maximum allowed layers for multiple monitoring	807

9.1A.4	Capabilities for Support of Event Triggering and Reporting Criteria	807

9.1A.4.1	Introduction	807

9.1A.4.2	Requirements	808

9.1A.5	Carrier-specific scaling factor	808

9.1A.5.1	Monitoring of multiple layers outside gaps	808

9.1A.5.1.1	SA mode: carrier-specific scaling factor for SSB-based measurements performed outside gaps	809

9.1A.5.2	Monitoring of multiple layers within gaps	809

9.1A.5.2.1	SA mode: carrier-specific scaling factor for SSB measurements performed within gaps	809

9.1A.6	Minimum requirement at transitions	811

9.1C	General measurement requirement for SAN	811

9.1C.1	Introduction	811

9.1C.2	Measurement gap	812

9.1C.8	Concurrent measurement gaps for SAN	814

9.1C.8.1	Introduction	814

9.1C.8.2	Requirements	814

9.1C.8.3	Collision between concurrent measurement gaps	815

9.1C.8.4	Measurement gap related requirements of concurrent measurement gaps	815

9.1C.9	Collision between SMTC and measurement gap for SAN	815

9.1C.9.1	Introduction	815

9.1C.9.2	Collision between SMTCs and measurement gap	815

9.1C.9.3	Collision between multiple SMTCs on a SAN carrier	816

9.1D	General measurement requirement for ATG	816

9.1D.1	Introduction	816

9.1D.2	Measurement gap	816

9.1D.2.1a	SA: Measurement Gap Sharing	819

9.1D.3	UE Measurement capability	819

9.1D.3.1	SA: Monitoring of multiple layers using gaps	819

9.1D.3.2	SA: Maximum allowed layers for multiple monitoring	819

9.1D.4	Void	820

9.1D.5	Carrier-specific scaling factor	820

9.1D.5.1	Monitoring of multiple layers outside gaps	820

9.1D.5.1.1	Void	821

9.1D.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements performed outside gaps	821

9.1D.5.2	Monitoring of multiple layers within gaps	821

9.1D.5.2.1	Void	822

9.1D.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements performed within gaps	822

9.1D.6	Void	823

9.1D.7	Pre-configured measurement gap	823

9.1D.7.1	Introduction	823

9.1D.7.2	Requirements applicability	823

9.1D.7.3	Requirements	823

9.1D.7.3.1	Requirements for autonomous activation/deactivation mechanism	823

9.1D.7.3.2	Requirements for network-controlled activation/deactivation mechanism	824

9.1D.7.3.3	Requirements for reception/transmission during activation/deactivation	824

9.1D.8	Capabilities for Support of Event Triggering and Reporting Criteria	824

9.1D.8.1	Introduction	824

9.1D.8.2	Requirements	825

9.1D.9	Minimum requirement at transitions	825

9.2	NR intra-frequency measurements	826

9.2.1	Introduction	826

9.2.2	Requirements applicability	829

9.2.3	Number of cells and number of SSB	829

9.2.3.1	Requirements for FR1	829

9.2.3.2	Requirements for FR2	829

9.2.4	Measurement Reporting Requirements	830

9.2.4.1	Periodic Reporting	830

9.2.4.2	Event-triggered Periodic Reporting	830

9.2.4.3	Event Triggered Reporting	830

9.2.4.4	SCell activation Triggered Reporting	831

9.2.5	Intrafrequency measurements without measurement gaps	831

9.2.5.1	Intrafrequency cell identification	831

9.2.5.2	Measurement period	839

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	843

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	844

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	844

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	845

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	846

9.2.5.4	SFTD Measurements between PCell and PSCell	846

9.2.5.4.1	Introduction	846

9.2.5.4.2	SFTD Measurement delay	847

9.2.5.4.3	SFTD Measurement Reporting Delay	847

9.2.6	Intra-frequency measurements with measurement gaps	847

9.2.6.1	Void	847

9.2.6.2	Intra-frequency cell identification	847

9.2.6.3	Intrafrequency Measurement Period	851

9.2.7	Intra-frequency measurements with NCSG	852

9.2.7.1	Intra-frequency cell identification	852

9.2.7.2	Measurement period	854

9.2.7.3	Scheduling availability during intra-frequency measurement with NCSG	856

9.2A	NR intra-frequency measurements with CCA	856

9.2A.1	Introduction	856

9.2A.2	Requirements applicability	857

9.2A.3	Number of cells and number of SSB	857

9.2A.3.1	Requirements for FR1	857

9.2A.3.2	Requirements for FR2-2	857

9.2A.4	Measurement Reporting Requirements	858

9.2A.5	Intra-frequency measurements without measurement gaps	858

9.2A.5.2	Measurement period	863

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	865

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	865

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	866

9.2A.5.3.3	Scheduling availability of UE performing measurements in TDD bands on FR2-2	866

9.2A.6	Intra-frequency measurements with measurement gaps	866

9.2A.6.1	Intra-frequency cell identification	866

9.2A.6.2	Intra-frequency Measurement Period	868

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	869

9.2A.7.1	Intra-frequency RSSI measurements	869

9.2A.7.2	Intra-frequency Channel occupancy measurements	871

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements in FR1	873

9.2A.7.4	Scheduling restriction during RSSI measurements in FR2-2	873

9.2B	NR intra-frequency measurements for RedCap	873

9.2B.1	Introduction	873

9.2B.2	Requirements applicability	874

9.2B.3	Number of cells and number of SSB	874

9.2B.3.1	Requirements for FR1	874

9.2B.3.2	Requirements for FR2	874

9.2B.4	Measurement Reporting Requirements	875

9.2B.4.1	Periodic Reporting	875

9.2B.4.2	Event-triggered Periodic Reporting	875

9.2B.4.3	Event Triggered Reporting	875

9.2B.5	Intra-frequency measurements without measurement gaps	875

9.2B.5.1	Intra-frequency cell identification	875

9.2B.5.2	Measurement period	878

9.2B.5.3	Scheduling availability of UE during intra-frequency measurements	879

9.2B.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	879

9.2B.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	879

9.2B.5.3.3	Scheduling availability of UE performing measurements on FR2	880

9.2B.5.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	880

9.2B.6	Intra-frequency measurements with measurement gaps	881

9.2B.6.1	Intra-frequency cell identification	881

9.2B.6.2	Intra-frequency Measurement Period	882

9.2C	NR intra-frequency measurements for SAN	883

9.2C.1	Introduction	883

9.2C.2	Requirements applicability	883

9.2C.3	Number of cells and number of SSB	884

9.2C.3.1	Requirements for FR1	884

9.2C.4	Measurement Reporting Requirements	884

9.2C.4.1	Periodic Reporting	884

9.2C.4.2	Event-triggered Periodic Reporting	884

9.2C.4.3	Event Triggered Reporting	884

9.2C.5	Intra frequency measurements without measurement gaps	885

9.2C.5.1	Intra frequency cell identification	885

9.2C.5.2	Measurement period	887

9.2C.5.3	Scheduling availability of UE during intra-frequency measurements	887

9.2C.5.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	888

9.2C.5.3.2	Scheduling availability of UE performing measurements on a neighbor cell served by a different satellite in LEO	888

9.2C.6	Intra-frequency measurements with measurement gaps	888

9.2C.6.1	Intra-frequency cell identification	888

9.2C.6.3	Intrafrequency Measurement Period	890

9.2C.7	Intra frequency measurements without measurement gaps for NTN band above 10GHz	890

9.2C.7.1	Intra frequency cell identification	890

9.2C.7.2	Measurement period	892

9.2C.7.3	Scheduling availability of UE during intra-frequency measurements	892

9.2C.8	Intra-frequency measurements with measurement gaps for NTN band above 10GHz	892

9.2C.8.1	Intra-frequency cell identification	892

9.2C.8.3	Intrafrequency Measurement Period	893

9.2D	NR intra-frequency measurements for ATG	893

9.2D.1	Introduction	893

9.2D.2	Requirements applicability	894

9.2D.3	Number of cells and number of SSB	894

9.2D.3.1	Requirements for FR1	894

9.2D.4	Measurement Reporting Requirements	894

9.2D.4.1	Periodic Reporting	894

9.2D.4.2	Event-triggered Periodic Reporting	894

9.2D.4.3	Event Triggered Reporting	895

9.2D.5	Intra-frequency measurements without measurement gaps	895

9.2D.5.1	Intra-frequency cell identification	895

9.2D.5.2	Measurement period	897

9.2D.5.3	Scheduling availability of UE during intra-frequency measurements	898

9.2D.5.3.1	Scheduling availability of UE performing measurements on FR1	898

9.2D.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	899

9.2D.6	Intra-frequency measurements with measurement gaps	899

9.2D.6.1	Void	899

9.2D.6.2	Intra-frequency cell identification	899

9.2D.6.3	Intra-frequency Measurement Period	901

9.3	NR inter-frequency measurements	901

9.3.1	Introduction	901

9.3.2	Requirements applicability	904

9.3.2.1	Void	904

9.3.2.2	Void	904

9.3.3	Number of cells and number of SSB	904

9.3.3.1	Requirements for FR1	904

9.3.3.2	Requirements for FR2	904

9.3.4	Inter-frequency measurement with measurement gaps	905

9.3.4.1	Void	909

9.3.4.2	Void	909

9.3.5	Inter-frequency measurements	909

9.3.5.1	Void	911

9.3.5.2	Void	911

9.3.5.3	Void	911

9.3.6	Inter-frequency measurements reporting requirements	911

9.3.6.1	Periodic Reporting	911

9.3.6.2	Event-triggered Periodic Reporting	911

9.3.6.3	Event-triggered Reporting	912

9.3.7	Void	912

9.3.8	Inter-frequency SFTD measurement requirements	912

9.3.8.1	Introduction	912

9.3.8.2	SFTD Measurement delay	912

9.3.8.3	SFTD Measurement reporting delay	913

9.3.9	Inter frequency measurements without measurement gaps	914

9.3.9.1	Inter frequency Cell identification	914

9.3.9.2	Measurement period	919

9.3.9.3	Scheduling availability of UE during inter-frequency measurements when the SSB is completely contained in the active BWP of the UE	921

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	921

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	922

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	922

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	923

9.3.9.4	Scheduling availability of UE during inter-frequency measurements when the SSB is not completely contained in the active BWP of the UE	923

9.3.9.4.1	Scheduling availability of UE performing measurements in TDD bands on FR1	923

9.3.9.4.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	924

9.3.9.4.3	Scheduling availability of UE performing measurements on FR2	924

9.3.9.4.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	926

9.3.10	Inter-frequency measurement with NCSG	926

9.3.10.1	Inter-frequency cell identification	926

9.3.10.2	Measurement period	928

9.3.10.3	Scheduling availability during inter-frequency measurement with NCSG	928

9.3.10.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	929

9.3.10.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	929

9.3.10.3.3	Scheduling availability of UE performing measurements on FR2	930

9.3.10.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	932

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	932

9.3A.1	Introduction	932

9.3A.2	Requirements applicability	932

9.3A.3	Number of cells and number of SSB	933

9.3A.3.1	Requirements for FR1	933

9.3A.3.2	Requirements for FR2-2	933

9.3A.4	Inter-frequency cell identification	933

9.3A.5	Inter-frequency measurements	935

9.3A.6	NR Inter-frequency measurements reporting requirements	937

9.3A.6.1	Periodic Reporting	937

9.3A.6.2	Event-triggered Periodic Reporting	937

9.3A.6.3	Event-triggered Reporting	937

9.3A.8	Inter-frequency RSSI measurements	937

9.3A.9	Inter-frequency channel occupancy measurements	938

9.3B	NR inter-frequency measurements for RedCap	939

9.3B.1	Introduction	939

9.3B.2	Requirements applicability	939

9.3B.3	Number of cells and number of SSB	940

9.3B.3.1	Requirements for FR1	940

9.3B.3.2	Requirements for FR2	940

9.3B.4	Inter-frequency measurement with measurement gaps	940

9.3B.5	Inter-frequency measurements	941

9.3B.6	Inter-frequency measurements reporting requirements	942

9.3B.6.1	Periodic Reporting	942

9.3B.6.2	Event-triggered Periodic Reporting	942

9.3B.6.3	Event-triggered Reporting	942

9.3B.7	Inter frequency measurements without measurement gaps	943

9.3B.7.1	Inter frequency Cell identification	943

9.3B.7.2	Measurement period	945

9.3B.7.3	Scheduling availability of UE during inter-frequency measurements	946

9.3B.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	946

9.3B.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	947

9.3B.7.3.3	Scheduling availability of UE performing measurements on FR2	947

9.3B.7.3.4	Scheduling availability of UE performing measurements in HD-FDD bands on FR1	947

9.3C	NR inter-frequency measurements for SAN	948

9.3C.1	Introduction	948

9.3C.2	Requirements applicability	948

9.3C.3	Number of cells and number of SSB	948

9.3C.3.1	Requirements for FR1	948

9.3C.4	Inter-frequency measurement with measurement gaps	949

9.3C.5	Inter-frequency measurements	950

9.3C.6	Inter-frequency measurements reporting requirements	951

9.3C.6.1	Periodic Reporting	951

9.3C.6.2	Event-triggered Periodic Reporting	951

9.3C.6.3	Event-triggered Reporting	951

9.3C.7	Inter frequency measurements without measurement gaps	951

9.3C.7.1	Inter frequency Cell identification	951

9.3C.7.2	Measurement period	953

9.3C.7.3	Scheduling availability of UE during inter-frequency measurements	953

9.3C.7.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	953

9.3C.7.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	954

9.3C.8	Inter-frequency measurement with measurement gaps for NTN band above 10GHz	954

9.3C.9	Inter-frequency measurements for NTN band above 10GHz	955

9.3C.10	Inter frequency measurements without measurement gaps for NTN band above 10GHz	956

9.3C.10.1	Inter frequency Cell identification	956

9.3C.10.2	Measurement period	957

9.3C.10.3	Scheduling availability of UE during inter-frequency measurements	957

9.3C.10.3.1	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10GHz	957

9.3D	NR inter-frequency measurements for ATG	958

9.3D.1	Introduction	958

9.3D.2	Requirements applicability	958

9.3D.3	Number of cells and number of SSB	958

9.3D.3.1	Requirements for FR1	958

9.3D.4	Inter-frequency measurement with measurement gaps	959

9.3D.5	Inter-frequency measurements	960

9.3D.6	Inter-frequency measurements reporting requirements	960

9.3D.6.1	Periodic Reporting	960

9.3D.6.2	Event-triggered Periodic Reporting	960

9.3D.6.3	Event-triggered Reporting	960

9.3D.7	Void	961

9.3D.8	Void	961

9.3D.9	Inter frequency measurements without measurement gaps	961

9.3D.9.1	Inter frequency Cell identification	961

9.3D.9.2	Measurement period	963

9.3D.9.3	Scheduling availability of UE during inter-frequency measurements	964

9.3D.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	964

9.3D.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	964

9.4	Inter-RAT measurements	965

9.4.1	Introduction	965

9.4.2	NR − E-UTRAN FDD measurements	967

9.4.2.1	Introduction	967

9.4.2.2	Requirements when no DRX is used	967

9.4.2.3	Requirements when DRX is used	969

9.4.2.4	Measurement reporting requirements	970

9.4.2.4.1	Periodic Reporting	970

9.4.2.4.2	Event-Triggered Periodic Reporting	970

9.4.2.4.3	Event-Triggered Reporting	971

9.4.2.5	Scheduling Availability During NR − E-UTRAN FDD measurements with NCSG	971

9.4.3	NR − E-UTRAN TDD measurements	971

9.4.3.1	Introduction	971

9.4.3.2	Requirements when no DRX is used	971

9.4.3.3	Requirements when DRX is used	973

9.4.3.4	Measurement reporting requirements	975

9.4.3.4.1	Periodic Reporting	975

9.4.3.4.2	Event-Triggered Periodic Reporting	975

9.4.3.4.3	Event-Triggered Reporting	975

9.4.3.5	Scheduling Availability During NR − E-UTRAN TDD measurements with NCSG	976

9.4.4	Inter-RAT RSTD measurements	976

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	976

9.4.4.1.1	Introduction	976

9.4.4.1.2	Requirements	976

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	979

9.4.4.2.1	Introduction	979

9.4.4.2.2	Requirements	980

9.4.5	Inter-RAT E-CID measurements	983

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	983

9.4.5.1.1	Introduction	983

9.4.5.1.2	Requirements	983

9.4.5.1.3	Measurement Reporting Delay	984

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	984

9.4.5.2.1	Introduction	984

9.4.5.2.2	Requirements	984

9.4.5.2.3	Measurement Reporting Delay	984

9.4.6	NR − UTRAN FDD measurements	984

9.4.6.1	Introduction	984

9.4.6.2	Requirements when no DRX is used	984

9.4.6.3	Requirements when DRX is used	986

9.4.7	NR – E-UTRAN measurements with autonomous gaps	988

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	988

9.4.7.2	CGI reporting delay	988

9.4.8	NR – E-UTRAN measurements without measurement gaps	988

9.4.8.1	Introduction	988

9.4.8.2	General requirements	988

9.4.8.3	NR − E-UTRAN FDD measurements	989

9.4.8.3.1	Introduction	989

9.4.8.3.2	Requirements when no DRX is used	989

9.4.8.3.3	Requirements when DRX is used	990

9.4.8.3.4	Measurement reporting requirements	991

9.4.8.3.4.1	Periodic Reporting	991

9.4.8.3.4.2	Event-Triggered Periodic Reporting	991

9.4.8.3.4.3	Event-Triggered Reporting	991

9.4.8.3.5	Scheduling availability during NR − E-UTRAN FDD measurements	992

9.4.8.3.5.1	Scheduling availability of UE performing inter-RAT measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	992

9.4.8.4	NR − E-UTRAN TDD measurements	992

9.4.8.4.1	Introduction	992

9.4.8.4.2	Requirements when no DRX is used	992

9.4.8.4.3	Requirements when DRX is used	994

9.4.8.4.4	Measurement reporting requirements	994

9.4.8.4.5	Scheduling availability during NR − E-UTRAN TDD measurements	995

9.4A	Inter-RAT measurements for RedCap	996

9.4A.1	Introduction	996

9.4A.2	NR − E-UTRAN FDD measurements	997

9.4A.2.1	Introduction	997

9.4A.2.2	Requirements when no DRX is used	997

9.4A.2.3	Requirements when DRX is used	998

9.4A.2.4	Measurement reporting requirements	999

9.4A.2.4.1	Periodic Reporting	999

9.4A.2.4.2	Event-Triggered Periodic Reporting	999

9.4A.2.4.3	Event-Triggered Reporting	999

9.4A.3	NR − E-UTRAN TDD measurements	1000

9.4A.3.1	Introduction	1000

9.4A.3.2	Requirements when no DRX is used	1000

9.4A.3.3	Requirements when DRX is used	1002

9.4A.3.4	Measurement reporting requirements	1003

9.4A.3.4.1	Periodic Reporting	1003

9.4A.3.4.2	Event-Triggered Periodic Reporting	1003

9.4A.3.4.3	Event-Triggered Reporting	1003

9.4A.4	NR – E-UTRAN measurements with autonomous gaps	1004

9.4A.4.1	CGI identification of an E-UTRA cell with autonomous gaps	1004

9.4A.4.2	CGI reporting delay	1004

9.4A.4.3	CGI reporting scheduling restriction	1004

9.5	L1-RSRP measurements for Reporting	1005

9.5.1	Introduction	1005

9.5.2	Requirements applicability	1005

9.5.3	Measurement Reporting Requirements	1006

9.5.3.1	Periodic Reporting	1006

9.5.3.2	Semi-Persistent Reporting	1006

9.5.3.3	Aperiodic Reporting	1007

9.5.4	L1-RSRP measurement requirements	1007

9.5.4.1	SSB based L1-RSRP Reporting	1007

9.5.4.2	CSI-RS based L1-RSRP Reporting	1013

9.5.4A	Void	1016

9.5.4A.1	Void	1016

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1017

9.5.5.1	Measurement restriction for SSB based L1-RSRP	1017

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	1017

9.5.6	Scheduling availability of UE during L1-RSRP measurement	1019

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1019

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1019

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1020

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1021

9.5A	L1-RSRP measurements for Reporting under CCA	1022

9.5A.1	Introduction	1022

9.5A.2	Requirements applicability	1022

9.5A.3	Measurement Reporting Requirements	1022

9.5A.3.1	Periodic Reporting	1023

9.5A.3.2	Semi-Persistent Reporting	1023

9.5A.3.3	Aperiodic Reporting	1023

9.5A.4	L1-RSRP measurement requirements	1023

9.5A.4.1	SSB based L1-RSRP Reporting	1023

9.5A.5	Measurement restriction for L1-RSRP measurement	1026

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	1026

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	1026

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1026

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1027

9.5A.6.3	Void	1027

9.5A.6.3A	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	1027

9.5A.6.3B	Scheduling availability of UE performing L1-RSRP measurement on FR2-2	1027

9.5A.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1028

9.5B	L1-RSRP measurements for Reporting for RedCap	1028

9.5B.1	Introduction	1028

9.5B.2	Requirements applicability	1028

9.5B.3	Measurement Reporting Requirements	1029

9.5B.3.1	Periodic Reporting	1029

9.5B.3.2	Semi-Persistent Reporting	1029

9.5B.3.3	Aperiodic Reporting	1030

9.5B.4	L1-RSRP measurement requirements	1030

9.5B.4.1	SSB based L1-RSRP Reporting	1030

9.5B.4.2	CSI-RS based L1-RSRP Reporting	1032

9.5B.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1035

9.5B.5.1	Measurement restriction for SSB based L1-RSRP	1035

9.5B.5.2	Measurement restriction for CSI-RS based L1-RSRP	1035

9.5B.6	Scheduling availability of UE during L1-RSRP measurement	1035

9.5B.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1036

9.5B.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1036

9.5B.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1036

9.5C	L1-RSRP measurements for Reporting for satellite access	1037

9.5C.1	Introduction	1037

9.5C.3	Measurement Reporting Requirements	1037

9.5C.3.1	Periodic Reporting	1038

9.5C.3.2	Semi-Persistent Reporting	1038

9.5C.3.3	Aperiodic Reporting	1038

9.5C.4	L1-RSRP measurement requirements	1038

9.5C.4.1	SSB based L1-RSRP Reporting	1038

9.5C.5	Measurement restriction for L1-RSRP measurement	1040

9.5C.5.1	Measurement restriction for SSB based L1-RSRP	1040

9.5C.5.2	Measurement restriction for CSI-RS based L1-RSRP	1040

9.5C.6	Scheduling availability of UE during L1-RSRP measurement	1041

9.5C.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1041

9.5C.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1041

9.5C.7	L1-RSRP measurement requirements for NTN band above 10GHz	1041

9.5C.7.1	SSB based L1-RSRP Reporting	1041

9.5C.7.2	CSI-RS based L1-RSRP Reporting	1042

9.5C.8	Measurement restriction for L1-RSRP measurement for NTN band above 10GHz	1043

9.5C.8.1	Measurement restriction for SSB based L1-RSRP	1043

9.5C.8.2	Measurement restriction for CSI-RS based L1-RSRP	1043

9.5C.9	Scheduling availability of UE during L1-RSRP measurement for NTN band above 10GHz	1044

9.5C.9.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on NTN bands above 10GHz	1044

9.5C.9.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on NTN bands above 10GHz	1044

9.5D	L1-RSRP measurements for Reporting for ATG	1044

9.5D.1	Introduction	1044

9.5D.2	Requirements applicability	1044

9.5D.3	Measurement Reporting Requirements	1045

9.5D.3.1	Periodic Reporting	1045

9.5D.3.2	Semi-Persistent Reporting	1045

9.5D.3.3	Aperiodic Reporting	1045

9.5D.4	L1-RSRP measurement requirements	1046

9.5D.4.1	SSB based L1-RSRP Reporting	1046

9.5D.4.2	CSI-RS based L1-RSRP Reporting	1047

9.5D.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	1048

9.5D.5.1	Measurement restriction for SSB based L1-RSRP	1048

9.5D.5.2	Measurement restriction for CSI-RS based L1-RSRP	1049

9.5D.6	Scheduling availability of UE during L1-RSRP measurement	1049

9.5D.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1049

9.5D.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1049

9.6	NE-DC: Measurements	1049

9.6.1	Introduction	1049

9.6.2	SFTD Measurements	1050

9.6.2.1	Introduction	1050

9.6.2.2	SFTD Measurement requirements	1050

9.7	Cross Link Interference measurements	1050

9.7.1	Introduction	1050

9.7.2	SRS-RSRP measurements	1051

9.7.2.1	Introduction	1051

9.7.2.2	Requirements applicability	1051

9.7.2.3	Measurement Reporting Requirements	1051

9.7.2.3.1	Periodic Reporting	1051

9.7.2.3.2	Event-triggered Periodic Reporting	1051

9.7.2.3.3	Event Triggered Reporting	1052

9.7.2.4	Measurement capability	1052

9.7.2.5	SRS-RSRP measurement period	1052

9.7.3	CLI-RSSI measurements	1052

9.7.3.1	Introduction	1052

9.7.3.2	Requirements applicability	1052

9.7.3.3	Measurement Reporting Requirements	1053

9.7.3.3.1	Periodic Reporting	1053

9.7.3.3.2	Event-triggered Periodic Reporting	1053

9.7.3.3.3	Event Triggered Reporting	1053

9.7.3.4	Measurement capability	1053

9.7.3.5	CLI-RSSI measurement period	1053

9.7.4	Scheduling availability of UE during CLI measurements	1053

9.7.4.1	Scheduling availability of UE performing measurement on FR1	1053

9.7.4.2	Scheduling availability of UE performing measurement on FR2	1054

9.8	L1-SINR measurements for Reporting	1055

9.8.1	Introduction	1055

9.8.2	Requirements applicability	1055

9.8.3	Measurement Reporting Requirements	1056

9.8.3.1	Periodic Reporting	1056

9.8.3.2	Semi-Persistent Reporting	1056

9.8.4	L1-SINR measurement requirements	1057

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1057

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1062

9.8.5	Measurement restriction for L1-SINR measurement	1064

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	1064

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	1064

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	1066

9.8.6	Scheduling availability of UE during L1-SINR measurement	1067

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1067

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1067

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1068

9.8D	L1-SINR measurements for Reporting for ATG	1068

9.8D.1	Introduction	1068

9.8D.2	Requirements applicability	1069

9.8D.3	Measurement Reporting Requirements	1069

9.8D.3.1	Periodic Reporting	1069

9.8D.3.2	Semi-Persistent Reporting	1070

9.8D.3.3	Aperiodic Reporting	1070

9.8D.4	L1-SINR measurement requirements	1070

9.8D.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1070

9.8D.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1071

9.8D.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1072

9.9	NR measurements for positioning	1073

9.9.1	Introduction	1073

9.9.1.1	General Aspects of Gap-based Measurement	1073

9.9.1.2	General Aspects of Gapless Measurement	1074

9.9.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps	1075

9.9.2	RSTD measurements	1076

9.9.2.1	Introduction	1076

9.9.2.2	Requirements Applicability	1076

9.9.2.3	Measurement Capability	1077

9.9.2.4	Measurement Reporting Requirements	1077

9.9.2.4.1	Void	1077

9.9.2.4.2	Void	1077

9.9.2.4.3	Void	1077

9.9.2.5	Measurements Period Requirements	1077

9.9.2.6	Void	1080

9.9.2.7	Measurements Period Requirements without Measurement Gaps	1080

9.9.2.8	Void	1083

9.9.2.9	Measurements Period Requirements with both MG and PPW	1083

9.9.2.10	Measurements Period Requirements with Bandwidth Aggregation	1084

9.9.3	PRS-RSRP measurements	1087

9.9.3.1	Introduction	1087

9.9.3.2	Requirements applicability	1087

9.9.3.3	Measurement Capability	1087

9.9.3.4	Measurement Reporting Requirements	1087

9.9.3.5	Measurement Period Requirements	1088

9.9.3.6	Measurement Period Requirements without Measurement Gaps	1090

9.9.3.7	Void	1093

9.9.3.8	Measurements Period Requirements with both MG and PPW	1093

9.9.4	UE Rx-Tx time difference measurements	1093

9.9.4.1 Introduction	1093

9.9.4.2 Requirements Applicability	1093

9.9.4.3 Measurement Capability	1094

9.9.4.4 Measurement Reporting Requirements	1094

9.9.4.5	Measurement Period Requirements	1094

9.9.4.6	Measurement Period Requirements without Measurement Gaps	1098

9.9.4.7	Void	1101

9.9.4.8	Measurements Period Requirements with both MG and PPW	1101

9.9.4.9	Measurements Period Requirements with Bandwidth Aggregation	1102

9.9.5	E-CID measurements	1106

9.9.5.1	Introduction	1106

9.9.5.2	Measurement Requirements	1106

9.9.5.2.1	Intra-frequency Measurement Requirements	1106

9.9.5.2.2	Inter-frequency Measurement Requirements	1106

9.9.5.2.3	Measurement Reporting Delay	1106

9.9.6	PRS-RSRPP measurements	1107

9.9.6.1	Introduction	1107

9.9.6.2	Requirements applicability	1107

9.9.6.3	Measurement capability	1107

9.9.6.4	Measurement reporting requirements	1107

9.9.6.5	Measurement period requirements	1107

9.9.6.6	Measurement Period Requirements without Measurement Gaps	1107

9.9.6.7	Void	1108

9.9.6.8	Measurements Period Requirements with both MG and PPW	1108

9.9.7	Measurement requirements for DL RSCPD reported with RSTD	1108

9.9.7.1	Introduction	1108

9.9.7.2	Requirements Applicability	1108

9.9.7.3	Measurement Capability	1108

9.9.7.4	Measurement Reporting Requirements	1108

9.9.7.5	Measurements Period Requirements for DL RSCPD reported with RSTD	1108

9.9.8	Measurement requirements for DL RSCP reported with UE Rx-Tx time difference	1111

9.9.8.1	Introduction	1111

9.9.8.2	Requirements Applicability	1111

9.9.8.3	Measurement Capability	1111

9.9.8.4	Measurement Reporting Requirements	1111

9.9.8.5	Measurement Period Requirements for DL RSCP and UE Rx-Tx time difference	1112

9.9A	NR measurements for positioning for RedCap	1114

9.9A.1	Introduction	1114

9.9A.1.1	General Aspects of Gap-based Measurement	1115

9.9A.1.2	General Aspects of Gapless Measurement for RedCap positioning without FH	1115

9.9A.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps for RedCap positioning without FH	1116

9.9A.2	RSTD measurements for RedCap	1117

9.9A.2.1	Introduction	1117

9.9A.2.2	Requirements Applicability	1117

9.9A.2.3	Measurement Capability	1118

9.9A.2.4	Measurement Reporting Requirements	1118

9.9A.2.5	Measurements Period Requirements without FH	1118

9.9A.2.5.1	Measurements Period Requirements without FH with MG	1118

9.9A.2.5.2	Measurements Period Requirements without FH without MG	1121

9.9A.2.5.3	Measurements Period Requirements without FH with both MG and PPW	1124

9.9A.2.6	Measurements Period Requirements with FH	1124

9.9A.2.6.1	Measurements Period Requirements with FH with MG	1124

9.9A.3	PRS-RSRP measurements for RedCap	1126

9.9A.3.1	Introduction	1126

9.9A.3.2	Requirements applicability	1126

9.9A.3.3	Measurement Capability	1127

9.9A.3.4	Measurement Reporting Requirements	1127

9.9A.3.5	Measurements Period Requirements without FH	1127

9.9A.3.5.1	Measurement Period Requirements without FH with MG	1127

9.9A.3.5.2	Measurement Period Requirements without FH without MG	1129

9.9A.3.5.3	Measurements Period Requirements without FH with both MG and PPW	1132

9.9A.3.6	Measurements Period Requirements with FH	1132

9.9A.3.6.1	Measurements Period Requirements with FH with MG	1132

9.9A.4	UE Rx-Tx time difference measurements for RedCap	1134

9.9A.4.1	Introduction	1134

9.9A.4.2	Requirements Applicability	1134

9.9A.4.3	Measurement Capability	1135

9.9A.4.4	Measurement Reporting Requirements	1135

9.9A.4.5	Measurement Period Requirements without FH with MG	1135

9.9A.4.6	Measurement Period Requirements without FH without MG	1135

9.9A.4.7	Measurements Period Requirements without FH with both MG and PPW	1135

9.9A.4.8	Measurements Period Requirements with FH	1135

9.9A.5	PRS-RSRPP measurements for RedCap	1137

9.9A.5.1	Introduction	1137

9.9A.5.2	Requirements Applicability	1137

9.9A.5.3	Measurement Capability	1137

9.9A.5.4	Measurement Reporting Requirements	1137

9.9A.5.5	Measurement Period Requirements without FH with MG	1137

9.9A.5.6	Measurement Period Requirements without FH without MG	1137

9.9A.5.7	Measurements Period Requirements without FH with both MG and PPW	1137

9.9A.5.8	Measurements Period Requirements with FH	1138

9.9C	NR measurements for positioning in Satellite Access	1138

9.9C.1	Introduction	1138

9.9C.1.1	General Aspects of Gap-based Measurement	1138

9.9C.1.2	General Aspects of Gapless Measurement	1138

9.9C.2	RSTD measurements	1138

9.9C.3	PRS-RSRP measurements	1139

9.9C.4	UE Rx-Tx time difference measurements	1139

9.9C.4.1 Introduction	1139

9.9C.4.2 Requirements Applicability	1139

9.9C.4.3	Measurement Capability	1139

9.9C.4.4	Measurement Reporting Requirements	1139

9.9C.4.5	Measurement Period Requirements	1139

9.9C.4.6	Measurement Period Requirements without Measurement Gaps	1142

9.10	CSI-RS based L3 measurements	1144

9.10.1	Introduction	1144

9.10.2	CSI-RS based intra-frequency measurements	1144

9.10.2.1	Introduction	1144

9.10.2.2	Requirements applicability	1145

9.10.2.3	Number of cells and number of CSI-RS	1146

9.10.2.3.1	Requirements for FR1	1146

9.10.2.3.2	Requirements for FR2	1146

9.10.2.4	Measurement Reporting Requirements	1146

9.10.2.4.1	Periodic Reporting	1146

9.10.2.4.2	Event-triggered Periodic Reporting	1146

9.10.2.4.3	Event Triggered Reporting	1147

9.10.2.5	Intra-frequency measurements without measurement gaps	1147

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1149

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1149

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	1149

9.10.3	CSI-RS based Inter-frequency measurements	1150

9.10.3.1	Introduction	1150

9.10.3.2	Requirements applicability	1150

9.10.3.3	Number of cells and number of CSI-RS resources	1151

9.10.3.3.1	Requirements for FR1	1151

9.10.3.3.2	Requirements for FR2	1151

9.10.3.4	Measurements reporting requirements	1151

9.10.3.4.1	Periodic Reporting	1151

9.10.3.4.2	Event-triggered Periodic Reporting	1151

9.10.3.4.3	Event-triggered Reporting	1151

9.10.3.5	Inter frequency measurements with measurement gaps	1152

9.10D	CSI-RS based L3 measurements for ATG	1153

9.10D.1	Introduction	1153

9.10D.2	CSI-RS based intra-frequency measurements	1154

9.10D.2.1	Introduction	1154

9.10D.2.2	Requirements applicability	1154

9.10D.2.3	Number of cells and number of CSI-RS	1155

9.10D.2.3.1	Requirements for FR1	1155

9.10D.2.4	Measurement Reporting Requirements	1155

9.10D.2.4.1	Periodic Reporting	1155

9.10D.2.4.2	Event-triggered Periodic Reporting	1155

9.10D.2.4.3	Event Triggered Reporting	1156

9.10D.2.5	Intra-frequency measurements without measurement gaps	1156

9.10D.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	1158

9.10D.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	1158

9.10D.3	CSI-RS based Inter-frequency measurements	1158

9.10D.3.1	Introduction	1158

9.10D.3.2	Requirements applicability	1158

9.10D.3.3	Number of cells and number of CSI-RS resources	1159

9.10D.3.3.1	Requirements for FR1	1159

9.10D.3.4	Measurements reporting requirements	1159

9.10D.3.4.1	Periodic Reporting	1159

9.10D.3.4.2	Event-triggered Periodic Reporting	1159

9.10D.3.4.3	Event-triggered Reporting	1159

9.10D.3.5	Inter frequency measurements with measurement gaps	1160

9.11	NR measurements with autonomous gaps	1161

9.11.1	Introduction	1161

9.11.2	CGI identification of an NR cell with autonomous gaps	1161

9.11.3	CGI reporting delay	1162

9.11A	NR measurements with autonomous gaps for RedCap	1163

9.11A.1	Introduction	1163

9.11A.2	CGI identification of an NR cell with autonomous gaps	1163

9.11A.3	CGI reporting delay	1164

9.11A.4	CGI reporting scheduling restriction	1164

9.11D	NR measurements with autonomous gaps for ATG	1164

9.11D.1	Introduction	1165

9.11D.2	CGI identification of an NR cell with autonomous gaps	1165

9.11D.3	CGI reporting delay	1166

9.12	Measurement for Propagation Delay Compensation	1166

9.12.1	Introduction	1166

9.12.2	Requirements Applicability	1166

9.12.3	Measurement Capability	1166

9.12.4	Measurement period requirements	1166

9.12.4.1	PRS Measurement Period	1166

9.12.4.2	TRS Measurement Period	1167

9.12.5	Measurement Reporting Requirements	1168

9.12.6	Scheduling availability during measurement for Propagation Delay Compensation	1168

9.12.7	Measurement restriction for measurement for Propagation Delay Compensation	1169

9.12.8	Measurement requirement for Propagation Delay Compensation with MUSIM gaps	1169

9.13	L1-RSRP measurements for a cell with different PCI from serving cell	1169

9.13.1	Introduction	1169

9.13.2	Requirements Applicability	1169

9.13.3	Measurement Reporting Requirements	1170

9.13.3.1	Periodic Reporting	1170

9.13.3.2	Semi-Persistent Reporting	1170

9.13.3.3	Aperiodic Reporting	1171

9.13.4	L1-RSRP measurement requirements	1171

9.13.4.1	Inter-cell SSB based L1-RSRP Reporting	1171

9.13.5	Measurement restriction for L1-RSRP measurement	1174

9.13.5.1	Measurement restriction for SSB based L1-RSRP	1174

9.13.6	Scheduling availability of UE during L1-RSRP measurement	1175

9.13.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1175

9.13.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1175

9.13.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1176

9.13.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1176

9.13.6.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1176

9.14	Intra-frequency L1-RSRP measurements for neighbor cell	1177

9.14.1	Introduction	1177

9.14.2	Requirements Applicability	1177

9.14.3	Measurement Reporting Requirements	1178

9.14.3.1	Periodic Reporting	1178

9.14.3.2	Semi-Persistent Reporting	1178

9.14.3.3	Aperiodic Reporting	1178

9.14.4	Number of SSB frequency layers, number of cells and number of SSBs	1178

9.14.5	L1-RSRP measurement requirements without measurement gaps	1178

9.14.5.1	SSB based L1-RSRP Reporting	1178

9.14.6	Measurement restriction for L1-RSRP measurement	1181

9.14.6.1	Measurement restriction for SSB based L1-RSRP	1181

9.14.7	Scheduling availability of UE during L1-RSRP measurement	1182

9.14.7.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1182

9.14.7.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1182

9.14.7.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1183

9.14.7.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1183

9.14.7.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1183

9.15	NR inter-frequency L1 measurement	1184

9.15.1	Introduction	1184

9.15.2	Requirements Applicability	1184

9.15.3	Measurement Reporting Requirements	1184

9.15.3.1	Periodic Reporting	1185

9.15.3.2	Semi-Persistent Reporting	1185

9.15.3.3	Aperiodic Reporting	1185

9.15.4	Number of SSB frequency layers, number of cells and number of SSBs	1185

9.15.5	inter-frequency L1-RSRP with MG	1185

9.15.5.1	Inter-frequency SSB based L1-RSRP Reporting	1185

9.15.6	Inter frequency L1-RSRP measurement without measurement gaps	1186

9.15.6.1	Inter frequency L1-RSRP measurement requirements	1186

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement	1186

9.15.6.2	Measurement restriction for inter frequency L1-RSRP measurement	1189

9.15.6.2.1	Measurement restriction for SSB based L1-RSRP	1189

9.15.6.3	Scheduling availability of UE during inter-frequency L1-RSRP measurements	1190

9.15.6.3.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	1190

9.15.6.3.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	1190

9.15.6.3.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	1191

9.15.6.3.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	1191

9.15.6.3.5	Scheduling availability of UE performing L1-RSRP measurement in TDD bands on FR1	1191

10	Measurement Performance requirements	1177

10.1	NR measurements	1177

10.1.1	Introduction	1177

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	1177

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	1177

10.1.2.1.1	Absolute SS-RSRP Accuracy	1177

10.1.2.1.2	Relative SS-RSRP Accuracy	1178

10.1.2.2	Void	1179

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	1179

10.1.2.3.1	Absolute CSI-RSRP Accuracy	1179

10.1.2.3.2	Relative CSI-RSRP Accuracy	1180

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1181

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	1181

10.1.2B.1.1	Absolute SS-RSRP Accuracy	1181

10.1.2C	Intra-frequency RSRP accuracy requirements for FR1 SAN	1182

10.1.2C.1	Intra-frequency SS-RSRP accuracy requirements	1182

10.1.2C.1.1	Absolute SS-RSRP Accuracy	1182

10.1.2C.1.2	Relative SS-RSRP Accuracy	1183

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	1183

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	1183

10.1.3.1.1	Absolute SS-RSRP Accuracy	1183

10.1.3.1.2	Relative SS-RSRP Accuracy	1184

10.1.3.2	Void	1185

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	1185

10.1.3.3.1	Absolute CSI-RSRP Accuracy	1185

10.1.3.3.2	Relative CSI-RSRP Accuracy	1186

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1187

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	1187

10.1.3B.1.1	Absolute SS-RSRP Accuracy	1187

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	1188

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	1188

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1188

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	1189

10.1.4.2	Void	1190

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	1190

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	1190

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	1191

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1192

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	1192

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	1192

10.1.4C	Inter-frequency RSRP accuracy requirements for FR1 SAN	1193

10.1.4C.1	Inter-frequency SS-RSRP accuracy requirements	1193

10.1.4C.1.1	Absolute Accuracy of SS-RSRP in FR1	1193

10.1.4C.1.2	Relative Accuracy of SS-RSRP in FR1	1194

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	1194

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	1194

10.1.5.1.1	Absolute SS-RSRP Accuracy	1194

10.1.5.1.2	Relative SS-RSRP Accuracy	1195

10.1.5.2	Void	1196

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	1196

10.1.5.3.1	Absolute CSI-RSRP Accuracy	1196

10.1.5.3.2	Relative CSI-RSRP Accuracy	1197

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1198

10.1.5B.1	Inter-frequency SS-RSRP accuracy requirements	1198

10.1.5B.1.1	Absolute SS-RSRP Accuracy	1198

10.1.6	RSRP Measurement Report Mapping	1199

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	1201

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1201

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	1201

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	1202

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	1202

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1203

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1203

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	1203

10.1.7C	Intra-frequency RSRQ accuracy requirements for FR1 SAN	1204

10.1.7C.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1204

10.1.7C.1.1	Absolute SS-RSRQ Accuracy in FR1	1204

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	1205

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1205

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	1205

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	1206

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	1206

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1206

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1206

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	1207

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	1207

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1207

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	1207

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	1208

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	1209

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	1209

10.1.9.2.2	Relative CSI-RSRQ Accuracy	1210

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	1211

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1211

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	1211

10.1.9C	Inter-frequency RSRQ accuracy requirements for FR1 SAN	1212

10.1.9C.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1212

10.1.9C.1.1	Absolute Accuracy of SS-RSRQ in FR1	1212

10.1.9C.1.2	Relative Accuracy of SS-RSRQ in FR1	1213

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	1214

10.1.10.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1214

10.1.10.1.1	Absolute Accuracy of SS-RSRQ in FR2	1214

10.1.10.1.2	Relative Accuracy of SS-RSRQ in FR2	1214

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	1215

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	1215

10.1.10.2.2	Relative CSI-RSRQ Accuracy	1216

10.1.10B	 Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	1217

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1217

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	1217

10.1.11	RSRQ report mapping	1218

10.1.12	Intra-frequency SINR accuracy requirements for FR1	1218

10.1.12.1	Intra-frequency SS-SINR accuracy requirements in FR1	1218

10.1.12.1.1	Absolute SS-SINR Accuracy in FR1	1218

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	1219

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	1219

10.1.12C	 Intra-frequency SINR accuracy requirements for FR1 SAN	1220

10.1.12C.1	Intra-frequency SS-SINR accuracy requirements in FR1	1220

10.1.12C.1.1	Absolute SS-SINR Accuracy in FR1	1220

10.1.13	Intra-frequency SINR accuracy requirements for FR2	1221

10.1.13.1	Intra-frequency SS-SINR accuracy requirements in FR2	1221

10.1.13.1.1	Absolute SS-SINR Accuracy in FR2	1221

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	1222

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	1222

10.1.14	Inter-frequency SINR accuracy requirements for FR1	1222

10.1.14.1	Inter-frequency SS-SINR accuracy requirements in FR1	1222

10.1.14.1.1	Aboslute Accuracy of SS-SINR in FR1	1222

10.1.14.1.2	Relative Accuracy of SS-SINR in FR1	1223

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	1224

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	1224

10.1.14.2.2	Relative Accuracy of CSI-SINR in FR1	1225

10.1.14C	 Inter-frequency SINR accuracy requirements for FR1 SAN	1226

10.1.14C.1	Inter-frequency SS-SINR accuracy requirements in FR1	1226

10.1.14C.1.1	Aboslute Accuracy of SS-SINR in FR1	1226

10.1.14C.1.2	Relative Accuracy of SS-SINR in FR1	1227

10.1.15	Inter-frequency SINR accuracy requirements for FR2	1228

10.1.15.1	Inter-frequency SS-SINR accuracy requirements in FR2	1228

10.1.15.1.1	Aboslute Accuracy of SS-SINR in FR2	1228

10.1.15.1.2	Relative Accuracy of SS-SINR in FR2	1228

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	1229

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	1229

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	1230

10.1.16	SINR report mapping	1231

10.1.16.1	SS-SINR and CSI-SINR measurement report mapping	1231

10.1.17	Power Headroom	1232

10.1.17.1	Power Headroom Report	1232

10.1.17.1.1	Power Headroom Report Mapping	1232

10.1.18	PCMAX,c,f	1233

10.1.18.1	Report Mapping	1233

10.1.19	L1-RSRP accuracy requirements for FR1	1233

10.1.19.1	SSB based L1-RSRP accuracy requirements	1233

10.1.19.1.1	Absolute Accuracy	1233

10.1.19.1.2	Relative Accuracy	1234

10.1.19.2	CSI-RS based L1-RSRP accuracy requirements	1235

10.1.19.2.1	Absolute Accuracy	1235

10.1.19.2.2	Relative Accuracy	1236

10.1.19C	 L1-RSRP accuracy requirements for FR1 SAN	1237

10.1.19C.1	SSB based L1-RSRP accuracy requirements	1237

10.1.19C.1.1	Absolute Accuracy	1237

10.1.19C.1.2	Relative Accuracy	1238

10.1.19D	LTM Intra-frequency L1-RSRP accuracy requirements for FR1	1238

10.1.19D.1	SSB based intra-frequency L1-RSRP accuracy requirements	1238

10.1.19D.1.1	Absolute Accuracy	1238

10.1.19D.1.2	Relative Accuracy	1239

10.1.19E	  LTM Inter-frequency L1-RSRP accuracy requirements for FR1	1240

10.1.19E.1	SSB based Inter-frequency L1-RSRP accuracy requirements	1240

10.1.19E.1.1	Absolute Accuracy	1240

10.1.19E.1.2	Relative Accuracy	1241

10.1.20	L1-RSRP accuracy requirements for FR2	1242

10.1.20.1	SSB based L1-RSRP accuracy requirements	1242

10.1.20.1.1	Absolute Accuracy	1242

10.1.20.1.2	Relative Accuracy	1243

10.1.20.2	CSI-RS based L1-RSRP accuracy requirements	1244

10.1.20.2.1	Absolute Accuracy	1244

10.1.20.2.2	Relative Accuracy	1245

10.1.20A	  LTM Intra-frequency L1-RSRP accuracy requirements for FR2	1246

10.1.20A.1	SSB based intra-frequency L1-RSRP accuracy requirements	1246

10.1.20A.1.1	Absolute Accuracy	1246

10.1.20A.1.2	Relative Accuracy	1247

10.1.20B	  LTM Inter-frequency L1-RSRP accuracy requirements for FR2	1248

10.1.20B.1	SSB based inter-frequency L1-RSRP accuracy requirements	1248

10.1.20B.1.1	Absolute Accuracy	1248

10.1.20B.1.2	Relative Accuracy	1248

10.1.21	SFTD accuracy requirements	1249

10.1.21.1	SFTD acuracy requirements for NE-DC	1249

10.1.21.2	SFTD acuracy requirements for NR-DC	1251

10.1.21.3	Inter frequency SFTD acuracy requirements	1252

10.1.22	CLI measurement accuracy requirements	1253

10.1.22.1	SRS-RSRP	1253

10.1.22.1.1	SRS-RSRP Accuracy	1253

10.1.22.1.2	SRS-RSRP report mapping	1255

10.1.22.2	CLI-RSSI	1255

10.1.22.2.1	CLI-RSSI Accuracy	1255

10.1.22.2.2	CLI-RSSI report mapping	1256

10.1.23	RSTD Measurements	1257

10.1.23.1	Introduction	1257

10.1.23.2	Measurement Accuracy Requirements	1257

10.1.23.3	Report mapping	1267

10.1.23.3.1	Absolute DL RSTD Measurement Reporting	1267

10.1.23.3.2	Differential Reporting for DL RSTD Measurement	1269

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	1271

10.1.24	PRS-RSRP Measurements	1273

10.1.24.1	Introduction	1273

10.1.24.2	Measurement Accuracy Requirements	1273

10.1.24.2.1	Absolute PRS RSRP accuracy	1273

10.1.24.2.2	Relative PRS RSRP accuracy	1277

10.1.24.3	Report mapping	1281

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	1281

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	1282

10.1.25	UE Rx-Tx Time Difference Measurements	1284

10.1.25.1	Introduction	1284

10.1.25.2	Measurement Accuracy Requirements	1284

10.1.25.3	Report mapping	1301

10.1.25.3.1	Absolute UE Rx-Tx Measurement Report Mapping	1301

10.1.25.3.2	Differential UE Rx-Tx Measurement Report Mapping	1303

10.1.25.3.3	Additional Path Report Mapping for UE Rx-Tx Time Difference	1304

10.1.25C	UE Rx-Tx Time Difference Measurements in Satellite Accesss	1306

10.1.25C.1	Introduction	1306

10.1.25C.2	Measurement Accuracy Requirements	1306

10.1.25C.3	Report mapping	1307

10.1.26	FR2 P-MPR report	1307

10.1.26.1	Report mapping	1308

10.1.27	L1-SINR accuracy requirements for FR1	1308

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1308

10.1.27.1.1	Absolute Accuracy	1308

10.1.27.1.2	Relative Accuracy	1309

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1310

10.1.27.2.1	Absolute Accuracy	1310

10.1.27.2.2	Relative Accuracy	1312

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1313

10.1.27.3.1	Absolute Accuracy	1313

10.1.27.3.2	Relative Accuracy	1315

10.1.28	L1-SINR accuracy requirements for FR2	1317

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	1324

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1324

10.1.29.1.1	Absolute SS-RSRQ Accuracy	1324

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	1324

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1324

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	1324

10.1.30.1.2	Relative Accuracy of SS-RSRQ	1325

10.1.31	Intra-frequency SINR accuracy requirements under CCA	1326

10.1.31.1	Intra-frequency SS-SINR accuracy requirements in FR1	1326

10.1.31.1.1	Absolute SS-SINR Accuracy	1326

10.1.32	Inter-frequency SINR accuracy requirements under CCA	1326

10.1.32.1	Inter-frequency SS-SINR accuracy requirements in FR1	1326

10.1.32.1.1	Aboslute Accuracy of SS-SINR	1326

10.1.32.1.2	Relative Accuracy of SS-SINR	1327

10.1.33	L1-RSRP accuracy requirements under CCA	1328

10.1.33.1	SSB based L1-RSRP accuracy requirements in FR1	1328

10.1.33.1.1	Absolute Accuracy	1328

10.1.33.1.2	Relative Accuracy	1328

10.1.34	RSSI measurements under CCA	1329

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	1329

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	1329

10.1.34.3	RSSI measurement report mapping	1329

10.1.35	Channel occupancy measurements under CCA	1330

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	1330

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	1330

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	1330

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	1330

10.1.36.1.1	Absolute SS-RSRP Accuracy	1330

10.1.36.1.2	Relative SS-RSRP Accuracy	1331

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	1332

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	1332

10.1.37.1.1	Absolute Accuracy of SS-RSRP	1332

10.1.37.1.2	Relative Accuracy of SS-RSRP	1332

10.1.38	PRS-RSRPP Measurements	1333

10.1.38.1	Introduction	1333

10.1.38.2	Measurement Accuracy Requirements	1333

10.1.38.2.1	Absolute PRS RSRPP accuracy	1333

10.1.38.3	Report mapping	1337

10.1.38.3.1	Absolute PRS-RSRPP Measurement Report Mapping	1337

10.1.38.3.2	Differential Report Mapping for PRS-RSRPP Measurement	1338

10.1.39	UE Rx-Tx time difference measurements for RTT-based PDC	1339

10.1.39.1	Void	1339

10.1.39.2	 Measurement Accuracy Requirements for PRS	1339

10.1.39.3	 Measurement Accuracy Requirements for TRS	1343

10.1.40	Void	1346

10.1.41	FR1 DPC report	1346

10.1.41.1	Report mapping	1346

10.1.42	TDCP Measurement Report Mapping	1346

10.1A	NR measurements for RedCap	1348

10.1A.1	Introduction	1348

10.1A.2	Intra-frequency RSRP accuracy requirements for FR1	1348

10.1A.2.1	Intra-frequency SS-RSRP accuracy requirements	1348

10.1A.2.1.1	Absolute SS-RSRP Accuracy	1348

10.1A.2.1.2	Relative SS-RSRP Accuracy	1349

10.1A.3	Intra-frequency RSRP accuracy requirements for FR2	1350

10.1A.3.1	Intra-frequency SS-RSRP accuracy requirements	1350

10.1A.3.1.1	Absolute SS-RSRP Accuracy	1350

10.1A.3.1.2	Relative SS-RSRP Accuracy	1350

10.1A.4	Inter-frequency RSRP accuracy requirements for FR1	1350

10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements	1350

10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1	1350

10.1A.4.1.2	Relative Accuracy of SS-RSRP in FR1	1351

10.1A.5	Inter-frequency RSRP accuracy requirements for FR2	1352

10.1A.5.1	Inter-frequency SS-RSRP accuracy requirements	1352

10.1A.5.1.1	Absolute SS-RSRP Accuracy	1352

10.1A.5.1.2	Relative SS-RSRP Accuracy	1352

10.1A.6	Intra-frequency RSRQ accuracy requirements for FR1	1352

10.1A.6.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	1352

10.1A.6.1.1	Absolute SS-RSRQ Accuracy in FR1	1352

10.1A.7	Intra-frequency RSRQ accuracy requirements for FR2	1353

10.1A.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	1353

10.1A.7.1.1	Absolute SS-RSRQ Accuracy in FR2	1353

10.1A.8	Inter-frequency RSRQ accuracy requirements for FR1	1353

10.1A.8.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	1353

10.1A.8.1.1	Absolute Accuracy of SS-RSRQ in FR1	1353

10.1A.8.1.2	Relative Accuracy of SS-RSRQ in FR1	1354

10.1A.9	Inter-frequency RSRQ accuracy requirements for FR2	1355

10.1A.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	1355

10.1A.9.1.1	Absolute Accuracy of SS-RSRQ in FR2	1355

10.1A.9.1.2	Relative Accuracy of SS-RSRQ in FR2	1355

10.1A.10	 Intra-frequency SINR accuracy requirements for FR1	1355

10.1A.10.1	Intra-frequency SS-SINR accuracy requirements in FR1	1355

10.1A.10.1.1	Absolute SS-SINR Accuracy in FR1	1355

10.1A.11	Intra-frequency SINR accuracy requirements for FR2	1356

10.1A.11.1	Intra-frequency SS-SINR accuracy requirements in FR2	1356

10.1A.11.1.1	Absolute SS-SINR Accuracy in FR2	1356

10.1A.12 	Inter-frequency SINR accuracy requirements for FR1	1356

10.1A.12.1	Inter-frequency SS-SINR accuracy requirements in FR1	1356

10.1A.12.1.1	Aboslute Accuracy of SS-SINR in FR1	1356

10.1A.12.1.2	Relative Accuracy of SS-SINR in FR1	1357

10.1A.13	 Inter-frequency SINR accuracy requirements for FR2	1358

10.1A.13.1	Inter-frequency SS-SINR accuracy requirements in FR2	1358

10.1A.13.1.1	Aboslute Accuracy of SS-SINR in FR2	1358

10.1A.13.1.2	Relative Accuracy of SS-SINR in FR2	1358

10.1A.14	 L1-RSRP accuracy requirements for FR1	1358

10.1A.14.1	SSB based L1-RSRP accuracy requirements	1358

10.1A.14.1.1	Absolute Accuracy	1358

10.1A.14.1.2	Relative Accuracy	1359

10.1A.14.2	CSI-RS based L1-RSRP accuracy requirements	1360

10.1A.14.2.1	Absolute Accuracy	1360

10.1A.14.2.2	Relative Accuracy	1361

10.1A.15	 L1-RSRP accuracy requirements for FR2	1362

10.1A.15.1	SSB based L1-RSRP accuracy requirements	1362

10.1A.15.1.1	Absolute Accuracy	1362

10.1A.15.1.2	Relative Accuracy	1362

10.1A.15.2	CSI-RS based L1-RSRP accuracy requirements	1362

10.1A.15.2.1	Absolute Accuracy	1362

10.1A.15.2.2	Relative Accuracy	1362

10.2	E-UTRAN measurements	1363

10.2.1	Introduction	1363

10.2.2	E-UTRAN RSRP measurements	1363

10.2.3	E-UTRAN RSRQ measurements	1363

10.2.4	E-UTRAN RSTD measurements	1363

10.2.5	E-UTRAN RS-SINR measurements	1364

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	1364

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	1364

10.2A	E-UTRAN measurements for RedCap	1365

10.2A.1	Introduction	1365

10.2A.2	E-UTRAN RSRP measurements	1365

10.2A.3	E-UTRAN RSRQ measurements	1365

10.2A.4	E-UTRAN RS-SINR measurements	1366

10.3	UTRAN FDD Measurements	1366

10.3.1	UTRAN FDD CPICH RSCP	1366

10.3.2	UTRAN FDD CPICH Ec/No	1367

10.4	V2X measurements	1367

10.4.1	Introduction	1367

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	1367

10.4.2.1	PSBCH-RSRP Absolute Accuracy	1367

10.4.2.2	PSBCH-RSRP Relative Accuracy	1368

10.4.2A	Intra-frequency PSBCH-RSRP accuracy requirements for FR1 under CCA	1369

10.4.2A.1	PSBCH-RSRP Absolute Accuracy	1369

10.4.2A.2	PSBCH-RSRP Relative Accuracy	1369

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	1370

10.4.3.1	Absolute SL-RSSI Accuracy	1370

10.4.3A	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1 under CCA	1370

10.4.3A.1	Absolute SL-RSSI Accuracy	1370

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	1371

10.4.4.1	Absolute L1 SL-RSRP Accuracy	1371

10.4.4A	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1 under CCA	1372

10.4.4A.1	Absolute L1 SL-RSRP Accuracy	1372

10.4.5	Intra-Frequency Discovery Signal Measurement Accuracy Requirements	1372

10.4.5.1	Absolute Discovery Signal Measurement Accuracy	1373

11	Void	1373

12	V2X Requirements	1139

12.1	Introduction	1139

12.2	UE Transmit Timing	1139

12.2.1	Introduction	1139

12.2.2	GNSS as synchronization reference source	1140

12.2.3	NR Cell as synchronization reference source	1140

12.2.4	E-URTAN Cell as synchronization reference source	1140

12.2.5	SyncRef UE as synchronization reference source	1141

12.3	Initiation/Cease of SLSS Transmissions	1141

12.3.1	Introduction	1141

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1141

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1142

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1143

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1143

12.3A	Initiation/Cease of SLSS Transmissions with CCA	1144

12.3A.1	Introduction	1144

12.3A.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	1144

12.3A.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	1144

12.3A.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1144

12.3A.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1145

12.4	Selection / Reselection of V2X Synchronization Reference Source	1145

12.4A	Selection / Reselection of Sidelink Synchronization Reference Source with CCA	1147

12.5	L1 SL-RSRP measurements	1149

12.5.1	Introduction	1149

12.5.2	SL-RSRP measurements	1149

12.6	Congestion Control measurements	1150

12.7	Interruption	1150

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	1150

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	1150

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	1152

12.7.4	Interruptions to WAN at transitions between active and non-active during SL-DRX	1153

12.7.5	Interruptions to V2X sidelink at transitions between active and non-active during DRX	1153

12.7.6	Interruptions to V2X sidelink due to Active BWP switching Requirement	1153

12.7.7	Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration	1154

12.7.8	Interruptions at NR sidelink discovery configuration	1154

12.7.9	Interruptions to WAN due to sidelink carrier addition/release	1154

12.8	Reliability of GNSS signal	1155

12.9	Scheduling availability	1155

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	1155

12.9.2	Scheduling availability of UE switching between Uu uplink  and V2X sidelink	1156

12.10	Selection / Reselection of relay UE	1156

12.10.1	Introduction	1156

12.10.2	Selection / Reselection of relay UE	1156

12.11	Component Carrier Addition and Release Delay for Sidelink Carrier Aggregation	1157

12.12	Selection / Reselection of Synchronization Reference Source for NR SL Carrier Aggregation	1157

12A	NR Sidelink Measurements for Positioning	1159

12A.1	Introduction	1159

12A.2	SL RSTD measurements	1159

12A.2.1	Introduction	1159

12A.2.3	Measurement Capability	1159

12A.2.4	Measurement Reporting Requirements	1160

12A.2.5	Measurements Period Requirements	1160

12A.3	SL PRS-RSRP measurements	1161

12A.3.1	Introduction	1161

12A.3.2	Requirements Applicability	1161

12A.3.4	Measurement Reporting Requirements	1161

12A.3.5	Measurements Period Requirements	1162

12A.4	SL Rx-Tx measurements	1163

12A.4.1	Introduction	1163

12A.4.2	Requirements Applicability	1163

12A.4.3	Measurement Capability	1163

12A.4.4	Measurement Reporting Requirements	1163

12A.4.5	Measurement Period Requirements	1163

12A.5	SL PRS-RSRPP measurements	1165

12A.5.1	Introduction	1165

12A.5.2	Requirements Applicability	1165

12A.5.3	Measurement Capability	1165

12A.5.4	Measurement Reporting Requirements	1165

12A.5.5	Measurement Period Requirements	1165

12A.6	SL AoA measurements	1166

12A.6.1	Introduction	1166

12A.6.2	Requirements Applicability	1166

12A.6.3	Measurement Capability	1166

12A.6.4	Measurement Reporting Requirements	1166

12A.6.5	Measurement Period Requirements	1166

12A.7	SL RTOA measurements	1167

12A.7.1	Introduction	1167

12A.7.2	Requirements Applicability	1167

12A.7.3	Measurement Capability	1167

12A.7.4	Measurement Reporting Requirements	1168

12A.7.5	Measurement Period Requirements	1168

13	Measurement Performance Requirements for NR gNB	1169

13.1	UL-RTOA	1169

13.1.1	Report mapping	1169

13.1.1A	Additional Path Report Mapping for UL-RTOA	1171

13.2	gNB Rx-Tx time difference	1173

13.2.1	Report mapping	1173

13.2.1A	 Additional Path Report Mapping for gNB Rx-Tx	1175

13.2.2	Measurement Accuracy Requirements	1177

13.2.2.1	Introduction	1177

13.3	UL SRS RSRP measurement	1178

13.3.1	Report mapping	1178

13.3.2	Measurement accuracy requirements	1179

13.3.2.1	Introduction	1179

13.3.2.2	Requirements	1179

13.4	AoA/ZoA	1180

13.4.1	Report mapping	1180

13.5	Timing advance (TADV)	1181

13.5.1	Report mapping	1181

13.6	UL SRS RSRPP measurement	1182

13.6.1	Report mapping	1182

13.7	gNB Rx-Tx time difference measurements for RTT-based PDC	1184

13.7.1	Report mapping	1184

13.7.2	Measurement Accuracy Requirements	1184

13.7.2.1	Introduction	1184

13.7.2.2	Requirements	1184

Annex A (normative): Test Cases	1184

A.1	Purpose of annex	1184

A.2	Requirement classification for statistical testing	1184

A.2.1	Types of requirements in TS 38.133	1184

A.2.1.1	Time and delay requirements on UE higher layer actions	1184

A.2.1.2	Measurements of power levels, relative powers and time	1184

A.2.1.3	Implementation requirements	1185

A.2.1.4	Physical layer timing requirements	1185

A.2.1.5	Requirements under CCA	1185

A.3	RRM test configurations	1186

A.3.1	Reference measurement channels	1186

A.3.1.1	PDSCH	1186

A.3.1.1.1	FDD	1186

A.3.1.1.2	TDD	1188

A.3.1.2	CORESET for RMSI scheduling	1191

A.3.1.2.1	FDD	1191

A.3.1.2.2	TDD	1192

A.3.1.3	CORESET for RMC scheduling	1195

A.3.1.3.1	FDD	1195

A.3.1.3.2	TDD	1197

A.3.1.4	TDD UL/DL configuration	1200

A.3.1A	Reference measurement channels under CCA	1202

A.3.1A.1	PDSCH	1202

A.3.1A.1.1	TDD	1202

A.3.1A.2	CORESET for RMSI scheduling	1203

A.3.1A.2.1	TDD	1203

A.3.1A.3	CORESET for RMC scheduling	1204

A.3.1A.3.1	TDD	1204

A.3.1A.4	TDD UL/DL configuration	1205

A.3.1A.5	RMC burst transmission model	1205

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	1205

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	1206

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	1206

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	1207

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	1207

A.3.2.2	Void	1208

A.3.3	Reference DRX configurations	1208

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	1208

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	1209

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	1209

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	1209

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	1210

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	1210

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	1210

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	1211

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	1211

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	1211

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	1212

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	1212

A.3.3.13	DRX Configuration X1: DRX cycle = 80 ms and TAT = Infinity	1212

A.3.3.14	DRX Configuration 14: DRX cycle = 160 ms and TAT = Infinity	1213

A.3.4	Test Cases with Different Channel Bandwidths	1213

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	1213

A.3.4.1.1	Introduction	1213

A.3.4.1.2	Principle of testing	1213

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	1213

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	1213

A.3.5.1.1	Introduction	1213

A.3.5.1.2	Principle of Testing	1213

A.3.6	Antenna configurations	1214

A.3.6.1	Antenna configurations for FR1	1214

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	1214

A.3.6.1.1.1	Introduction	1214

A.3.6.1.1.2	Principle of testing	1214

A.3.6.1.2	Antenna connection for 8 Rx capable UEs	1217

A.3.6.1.2.1	Introduction	1217

A.3.6.1.2.2	Principle of testing	1217

A.3.6.2	Antenna configurations for FR2	1219

A.3.6A	Antenna configurations with unlicensed bands	1219

A.3.6A.1	Antenna configurations for FR1	1219

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	1219

A.3.6A.1.1.1	Introduction	1219

A.3.6A.1.1.2	Principle of testing	1219

A.3.7	EN-DC test setup	1221

A.3.7.1	Introduction	1221

A.3.7.2	E-UTRAN Serving Cell Parameters	1221

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1221

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1222

A.3.7A	NR FR1-FR2 test setup	1223

A.3.7B	EN-DC test setup with unlicensed bands	1224

A.3.7B.1	Introduction	1224

A.3.7B.2	E-UTRAN Serving Cell Parameters	1224

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	1224

A.3.7C	LTE-FR1/FR2 test setup	1225

A.3.7D	NE-DC test setup	1225

A.3.7D.1	Introduction	1225

A.3.7D.2	E-UTRAN Serving Cell Parameters	1225

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	1225

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	1225

A.3.8	PRACH configurations	1225

A.3.8.1	Introduction	1225

A.3.8.2	PRACH configurations in FR1	1226

A.3.8.2.1	FR1 PRACH configuration 1	1226

A.3.8.2.2	FR1 PRACH configuration 2	1226

A.3.8.2.3	FR1 PRACH configuration 3	1227

A.3.8.2.4	FR1 PRACH configuration 4	1228

A.3.8.2.5	FR1 PRACH configuration 5	1229

A.3.8.2.6	FR1 PRACH configuration 6	1230

A.3.8.3	PRACH configurations in FR2	1230

A.3.8.3.1	FR2 PRACH configuration 1	1230

A.3.8.3.2	FR2 PRACH configuration 2	1231

A.3.8.3.3	FR2 PRACH configuration 3	1232

A.3.8.3.4	FR2 PRACH configuration 4	1233

A.3.8.3.5	FR2 PRACH configuration 5	1234

A.3.8.3.6	FR2 PRACH configuration 6	1235

A.3.8A	PRACH configurations under CCA	1235

A.3.8A.1	Introduction	1235

A.3.8A.2	PRACH configurations in FR1	1235

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	1235

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	1236

A.3.9	BWP configurations	1237

A.3.9.1	Introduction	1237

A.3.9.2	Downlink BWP configurations	1238

A.3.9.2.1	Initial BWP	1238

A.3.9.2.2	Dedicated BWP	1238

A.3.9.3	Uplink BWP configurations	1239

A.3.9.3.1	Initial BWP	1239

A.3.9.3.2	Dedicated BWP	1239

A.3.9A	BWP configurations for RedCap	1239

A.3.9A.1	Introduction	1239

A.3.9A.2	Downlink BWP configurations	1240

A.3.9A.2.1	Dedicated BWP	1240

A.3.9A.3	Uplink BWP configurations	1240

A.3.9A.3.1	Dedicated BWP	1240

A.3.10	SSB Configurations	1241

A.3.10.1	SSB Configurations for FR1	1241

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1241

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	1243

A.3.10.1.6	SSB pattern 6 in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 40 MHz	1243

A.3.10.1.7	SSB pattern 7 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1244

A.3.10.1.8	SSB pattern 8 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1244

A.3.10.1.9	SSB pattern 9 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1245

A.3.10.1.10	SSB pattern 10 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1245

A.3.10.1.11	SSB pattern 11 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1246

A.3.10.1.12	SSB pattern 12 in FR1: SSB allocation for SSB SCS=30 kHz in 40 MHz	1246

A.3.10.1.13	SSB pattern 13 in FR1: SSB allocation for SSB SCS=15 kHz in 3 MHz	1247

A.3.10.2	SSB Configurations for FR2	1247

A.3.10.2.1	SSB pattern 1 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1247

A.3.10.2.2	SSB pattern 2 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1248

A.3.10.2.3	SSB pattern 3 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1248

A.3.10.2.4	SSB pattern 4 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1249

A.3.10.2.5	SSB pattern 5 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1249

A.3.10.2.6	SSB pattern 6 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1250

A.3.10.2.7	SSB pattern 7 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1250

A.3.10.2.8	SSB pattern 8 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1251

A.3.10.2.9	SSB pattern 9 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1251

A.3.10.2.10	SSB pattern 10 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1252

A.3.10.2.19	SSB pattern 19 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1256

A.3.10.2.20	SSB pattern 20 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1257

A.3.10.2.21	SSB pattern 21 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1257

A.3.10.2.22	SSB pattern 22 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1258

A.3.10.2.23	SSB pattern 23 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1258

A.3.10.2.24	SSB pattern 24 in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1259

A.3.10.2.25	SSB pattern 25 in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1259

A.3.10A	SSB Configurations under CCA	1260

A.3.10A.1	SSB Configurations under CCA for FR1	1260

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1260

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	1260

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1261

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	1261

A.3.10B	SSB Configurations for RedCap	1262

A.3.10B.1	SSB Configurations for FR1	1262

A.3.10B.1.1	SSB pattern 1 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1262

A.3.10B.1.2	SSB pattern 2 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1262

A.3.10B.1.3	SSB pattern 3 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz starting from odd SFN in 20 MHz	1263

A.3.10B.1.4	SSB pattern 4 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1263

A.3.10B.1.5	SSB pattern 5 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1264

A.3.10B.1.6	SSB pattern 6 for RedCap in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	1264

A.3.10B.1.7	SSB pattern 7 for RedCap in FR1: SSB allocation for SSB SCS=30 kHz in 20 MHz	1265

A.3.10B.2	SSB Configurations for FR2	1265

A.3.10B.2.1	SSB pattern 1 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1265

A.3.10B.2.2	SSB pattern 2 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1266

A.3.10B.2.3	SSB pattern 3 for RedCap in FR2: SSB allocation for SSB SCS=120 kHz in 100 MHz	1266

A.3.10B.2.4	SSB pattern 4 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1267

A.3.10B.2.5	SSB pattern 5 for RedCap in FR2: SSB allocation for SSB SCS=240 kHz in 100 MHz	1267

A.3.11	SMTC Configurations	1268

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	1268

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	1268

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	1268

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	1268

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	1269

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	1269

A.3.11.7	SMTC pattern 7: SMTC period = 20 ms with SMTC duration = 5 ms	1269

A.3.11.8	SMTC pattern 8: SMTC period = 10 ms with SMTC duration = 1 ms	1269

A.3.11.9	SMTC pattern 9: SMTC period = 20 ms with SMTC duration = 1 ms	1269

A.3.11.10	SMTC pattern 10: SMTC period = 80 ms with SMTC duration = 1 ms	1270

A.3.11.11	SMTC pattern 11: SMTC period = 80 ms with SMTC duration = 5 ms	1270

A.3.11A	SMTC Configurations for RedCap	1270

A.3.11A.0	Introduction	1270

A.3.11A.1	SMTC pattern 1 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1270

A.3.11A.2	SMTC pattern 2 for RedCap: SMTC period = 80 ms with SMTC duration = 1 ms	1270

A.3.11A.3	SMTC pattern 3 for RedCap: SMTC period = 40 ms with SMTC duration = 1 ms	1271

A.3.11A.4	SMTC pattern 4 for RedCap: SMTC period = 80 ms with SMTC duration = 5 ms	1271

A.3.12	Test Cases with Different CC Configurations	1271

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	1271

A.3.12.1.1	Introduction	1271

A.3.12.1.2	Principle of testing	1271

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	1271

A.3.12.2.1	Introduction	1271

A.3.12.2.2	Principle of testing	1271

A.3.13	Test Cases in SA and EN-DC Operations	1272

A.3.13.1	Introduction	1272

A.3.13.2	Principle of Testing	1272

A.3.13A	Test Cases involving E-UTRA/FR1 and FR2 carriers	1272

A.3.13A.1	Introduction	1272

A.3.13A.2	Principle of Testing in EN-DC	1272

A.3.13A.3	Principle of Testing in SA	1273

A.3.13A.4	Principle of Testing in E-UTRA	1273

A.3.13A.5	Principle of Testing in NR-DC	1274

A.3.13B	 Test Cases for EN-DC and NE-DC Operations	1274

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	1274

A.3.13B.1.1	Introduction	1274

A.3.13B.1.2	Principle of Testing	1274

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	1274

A.3.13B.2.1	Introduction	1274

A.3.13B.2.2	Principle of Testing	1275

A.3.14	CSI-RS configurations	1275

A.3.14.1	FDD	1275

A.3.14.2	TDD	1277

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	1284

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	1284

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	1284

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	1284

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	1285

A.3.15.3	Setup 3: 2 AoAs	1285

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	1285

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	1285

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	1285

A.3.15.4.3	Setup 4c: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak for power class 6 UE supporting simultaneous reception from multiple directions	1286

A.3.15.5	Setup 5: 2 AoAs for simultaneous reception with QCL Type-D	1286

A.3.15.6	Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D	1286

A.3.15.7	Setup 7: 3 AoAs	1287

A.3.16	TCI State Configuration	1287

A.3.16.1	Introduction	1287

A.3.16.2	TCI states	1287

A.3.16A	Unified TCI State Configuration	1287

A.3.16A.1	Introduction	1287

A.3.16A.2	DLorJoint TCI states	1288

A.3.16A.3	UL TCI states	1289

A.3.16B	 LTM Candidate TCI State Configuration	1289

A.3.16B.1	Introduction	1289

A.3.16B.2	LTM candidate DLorJoint TCI states	1290

A.3.16B.3	LTM candidate UL TCI states	1290

A.3.17	Configurations of CSI-RS for tracking	1291

A.3.17.1	Configuration of CSI-RS for tracking for FR1	1291

A.3.17.1.1	FDD	1291

A.3.17.1.2	TDD	1294

A.3.17.2	Configuration of CSI-RS for tracking for FR2	1299

A.3.17.2.1	TDD	1299

A.3.17.2.2	FDD	1301

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	1301

A.3.18.1	Introduction	1301

A.3.18.2	PRACH Power Measurement	1302

A.3.19	Test applicability for DAPS handover	1302

A.3.19.1	Introduction	1302

A.3.19.2	Principle of testing	1302

A.3.20	MsgA configurations	1302

A.3.20.1	Introduction	1302

A.3.20.2	MsgA configurations in FR1	1302

A.3.20.2.1	FR1 MsgA configuration 1	1302

A.3.20.2.2	FR1 MsgA configuration 2	1303

A.3.20.3	MsgA configurations in FR2	1305

A.3.20.3.1	FR2 MsgA configuration 1	1305

A.3.20.3.2	FR2 MsgA configuration 2	1306

A.3.20A	MsgA configurations under CCA	1308

A.3.20A.1	Introduction	1308

A.3.20A.2	MsgA configurations in FR1	1308

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	1308

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	1309

A.3.21	V2X sidelink communication	1312

A.3.21.1	Introduction	1312

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	1312

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	1316

A.3.21.4	Reference SL-DRX configurations	1317

A.3.21.4.1	SL-DRX Configuration 1: SL-DRX cycle = 40 ms	1317

A.3.21.4.2	SL-DRX Configuration 2: SL-DRX cycle = 320 ms	1317

A.3.21.4.3	SL-DRX Configuration 3: SL-DRX cycle = 640 ms	1317

A.3.22	CSI-IM configurations	1317

A.3.22.1	FDD	1317

A.3.22.2	TDD	1318

A.3.23	Spatial Relation Configuration	1319

A.3.23.1	Introduction	1319

A.3.23.2	Spatial Relation	1320

A.3.24	SRS configuration	1321

A.3.25	Channel bandwidth (CBW) configurations	1323

A.3.25.1	DL UE specific CBW	1323

A.3.25.2	UL UE specific CBW	1324

A.3.26	CCA model	1324

A.3.26.1	Introduction	1324

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	1324

A.3.26.2.1	DL CCA model	1324

A.3.26.2.2	UL CCA model	1326

A.3.26.3	CCA model for operation on a carrier frequency with CCA in FR2-2	1326

A.3.26.3.1	DL CCA model	1326

A.3.26.3.2	UL CCA model	1327

A.3.26.4	CCA model for operation on a sidelink carrier frequency with CCA	1327

A.3.26.4.1	CCA model for SyncRef UE	1327

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	1328

A.3.27.1	Introduction	1328

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	1328

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	1328

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	1329

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	1329

A.3.28	Discovery Burst Transmission Window configuration under CCA	1329

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	1329

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	1329

A.3.29.1	Introduction	1329

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	1329

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	1330

A.3.30	CSI-RS configurations for RRM	1330

A.3.30.1	FDD	1330

A.3.30.2	TDD	1331

A.3.31	PRS Configurations	1333

A.3.31.1.	PRS Configurations for FR1	1333

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	1333

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	1334

A.3.31.2.	PRS Configurations for FR2	1334

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	1334

A.3.32	NR sidelink discovery	1334

A.3.32.1	Introduction	1334

A.3.32.2	Reference resource pool configurations for NR Sidelink Discovery	1334

A.3.32.3	Principle of Testing	1335

A.3.33	PRS Processing Window (PPW) configurations	1335

A.3.34	Testing principles for test cases related to PRS measurements	1335

A.3.34.1	Introduction	1335

A.3.34.2	Test cases in RRC_INACTIVE state	1336

A.3.34.3	Test cases for PRS measurements with gaps in RRC_CONNECTED state	1336

A.3.34.4	Test cases for PRS measurements without gaps in RRC_CONNECTED state	1336

A.3.35	Testing principle for RedCap UE	1337

A.3.35.1	Introduction	1337

A.3.35.2	Principle of testing for FR1	1337

A.3.35.3	Principle of testing for FR2	1337

A.3.36	Testing related to Satellite access	1337

A.3.36.1	Introduction	1337

A.3.36.1	Introduction	1337

A.3.36.2	Principle of testing GSO and NGSO scenarios	1337

A.3.36.2	Principle of testing different RRM requirements	1338

A.3.36.3	Principle of testing different ephemeris formats	1338

A.3.36.4	General setup for SIB19	1340

A.3.36.5	Satellite specific parameters configuration	1341

A.3.36.5.1	Satellite specific configuration for serving cell	1341

A.3.36.5.2	Satellite specific configuration for neighbour cell	1341

A.3.37	Reference Cell DTX configurations	1342

A.3.37.1	Cell DTX Configuration 1: Cell DTX cycle = 640 ms and TAT = Infinity	1342

A.4	EN-DC tests with all NR cells in FR1	1321

A.4.1	Void	1321

A.4.2	Void	1321

A.4.3	RRC_CONNECTED state mobility	1321

A.4.3.1	Void	1321

A.4.3.2	RRC Connection Mobility Control	1321

A.4.3.2.1	Void	1321

A.4.3.2.2	Random Access	1321

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	1321

A.4.3.2.2.2	4-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1325

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	1329

A.4.3.2.2.4	2-step RA type n on-contention based random access test in FR1 for PSCell in EN-DC	1333

A.4.3.2.3	Void	1337

A.4.3.3	Handover with PSCell from EN-DC to EN-DC with known target PSCell in FR1	1337

A.4.3.3.1	Test Purpose and Environment	1337

A.4.3.3.2	Test Requirements	1343

A.4.4	Timing	1344

A.4.4.1	UE transmit timing	1344

A.4.4.1.1	NR UE Transmit Timing Test for FR1	1344

A.4.4.1.1.1	Test Purpose and environment	1344

A.4.4.1.1.2	Test requirements	1348

A.4.4.1.2	NR UE Transmit Timing Test for two TRPs in FR1	1348

A.4.4.1.2.1	Test Purpose and environment	1348

A.4.4.1.2.2	Test requirements	1353

A.4.4.2	UE timer accuracy	1354

A.4.4.3	Timing advance	1354

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	1354

A.4.4.3.1.1	Test Purpose and Environment	1354

A.4.4.3.1.2	Test Parameters	1354

A.4.4.3.1.3	Test Requirements	1357

A.4.5	Signaling characteristics	1357

A.4.5.1	Radio link Monitoring	1357

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1358

A.4.5.1.1.1	Test Purpose and Environment	1358

A.4.5.1.1.2	Test Requirements	1362

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	1362

A.4.5.1.2.1	Test Purpose and Environment	1362

A.4.5.1.2.2	Test Requirements	1368

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1368

A.4.5.1.3.1	Test Purpose and Environment	1368

A.4.5.1.3.2	Test Requirements	1373

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	1373

A.4.5.1.4.1	Test Purpose and Environment	1373

A.4.5.1.4.2	Test Requirements	1379

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1379

A.4.5.1.5.1	Test Purpose and Environment	1379

A.4.5.1.5.2	Test Requirements	1384

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	1384

A.4.5.1.6.1	Test Purpose and Environment	1384

A.4.5.1.6.2	Test Requirements	1390

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1390

A.4.5.1.7.1	Test Purpose and Environment	1390

A.4.5.1.7.2	Test Requirements	1395

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	1396

A.4.5.1.8.1	Test Purpose and Environment	1396

A.4.5.1.8.2	Test Requirements	1401

A.4.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1401

A.4.5.1.9.1	Test Purpose and Environment	1401

A.4.5.1.10	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	1407

A.4.5.1.10.1	Test Purpose and Environment	1407

A.4.5.1.11	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	1407

A.4.5.1.11.1	Test Purpose and Environment	1407

A.4.5.1.11.2	Test Requirements	1408

A.4.5.1.12	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting [FG 53-3]	1408

A.4.5.1.12.1	Test Purpose and Environment	1408

A.4.5.1.12.2	Test Requirements	1412

A.4.5.2	Interruption	1412

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1412

A.4.5.2.1.1	Test Purpose and Environment	1412

A.4.5.2.1.2	Test Requirements	1416

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1416

A.4.5.2.2.1	Test Purpose and Environment	1416

A.4.5.2.2.2	Test Requirements	1420

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1420

A.4.5.2.3.1	Test Purpose and Environment	1420

A.4.5.2.3.2	Test Requirements	1425

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1426

A.4.5.2.4.1	Test Purpose and Environment	1426

A.4.5.2.4.2	Test Requirements	1430

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1431

A.4.5.2.5.1	Test Purpose and Environment	1431

A.4.5.2.5.2	Test Requirements	1435

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1435

A.4.5.2.6.1	Test Purpose and Environment	1435

A.4.5.2.6.2	Test Requirements	1439

A.4.5.2.7	Void	1439

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	1439

A.4.5.2.8.1	Test Purpose and Environment	1439

A.4.5.2.8.2	Test Requirements	1442

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	1443

A.4.5.2.9.1	Test Purpose and Environment	1443

A.4.5.2.9.2	Test Requirements	1447

A.4.5.2.10	E-UTRAN – NR FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	1448

A.4.5.2.10.1	Test Purpose and Environment	1448

A.4.5.2.10.2	Test Requirements	1451

A.4.5.2.11	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in synchronous EN-DC	1451

A.4.5.2.11.1	Test Purpose and Environment	1451

A.4.5.2.11.2	Test Requirements	1456

A.4.5.2.12	E-UTRAN - NR FR1 interruptions at NR SRS antenna port switching in asynchronous EN-DC	1457

A.4.5.2.12.1	Test Purpose and Environment	1457

A.4.5.3	SCell Activation and Deactivation Delay	1465

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1465

A.4.5.3.1.1	Test Purpose and Environment	1465

A.4.5.3.1.2	Test Requirements	1471

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 640ms SCell measurement cycle	1472

A.4.5.3.2.1	Test Purpose and Environment	1472

A.4.5.3.2.2	Test Requirements	1472

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	1472

A.4.5.3.3.1	Test Purpose and Environment	1472

A.4.5.3.3.2	Test Requirements	1473

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	1474

A.4.5.3.4.1	Test Purpose and Environment	1474

A.4.5.3.3.2	Test Requirements	1476

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	1477

A.4.5.3.5.1	Test Purpose and Environment	1477

A.4.5.3.5.2	Test Requirements	1484

A.4.5.3.6	Fast SCell Activation of known SCell in FR1 for 160ms SCell measurement cycle	1485

A.4.5.3.6.1	Test Purpose and Environment	1485

A.4.5.3.6.2	Test Requirements	1489

A.4.5.3.7	Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle	1490

A.4.5.3.7.1	Test Purpose and Environment	1490

A.4.5.3.7.2	Test Requirements	1490

A.4.5.3.8	SCell Activation and deactivation of unknown SCell in FR1 for UE capable of short measurement interval	1490

A.4.5.3.8.1	Test Purpose and Environment	1490

A.4.5.3.8.2	Test Requirements	1491

A.4.5.3.9	SCell Activation of unknown SCell with valid L3 measurement results in FR1 for 160ms SCell measurement cycle	1492

A.4.5.3.9.1	Test Purpose and Environment	1492

A.4.5.3.9.2	Test Requirements	1498

A.4.5.3.10	SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation command in non-DRX	1499

A.4.5.3.10.1	Test Purpose and Environment	1499

A.4.5.3.10.2	Test Requirements	1503

A.4.5.3.11	TRS-based SCell Activation of SSB-less SCell in FR1 collocated inter-band	1503

A.4.5.3.11.1	Test Purpose and Environment	1503

A.4.5.3.11.2	Test Requirements	1508

A.4.5.3.12	Inter-band SSB-less Scell activation using A-TRS	1508

A.4.5.3.12.1	Test Purpose and Environment	1508

A.4.5.3.12.2	Test Requirements	1513

A.4.5.4	UE UL carrier RRC reconfiguration Delay	1514

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	1514

A.4.5.4.1.1 Test Purpose and Environment	1514

A.4.5.4.1.2	Test Requirements	1521

A.4.5.5	Beam Failure Detection and Link recovery procedures	1521

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1521

A.4.5.5.1.1	Test Purpose and Environment	1521

A.4.5.5.1.2	Test Requirements	1526

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	1527

A.4.5.5.2.1	Test Purpose and Environment	1527

A.4.5.5.2.2	Test Requirements	1532

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1533

A.4.5.5.3.1	Test Purpose and Environment	1533

A.4.5.5.3.2	Test Requirements	1538

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1539

A.4.5.5.4.1	Test Purpose and Environment	1539

A.4.5.5.4.2	Test Requirements	1544

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1545

A.4.5.5.5.1	Test Purpose and Environment	1545

A.4.5.5.5.2	Test Requirements	1550

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1551

A.4.5.5.6.1	Test Purpose and Environment	1551

A.4.5.5.6.2	Test Requirements	1557

A.4.5.5.7	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	1558

A.4.5.5.7.1	Test Purpose and Environment	1558

A.4.5.5.7.2	Test Requirements	1563

A.4.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1564

A.4.5.5.8.1	Test Purpose and Environment	1564

A.4.5.5.8.2	Test Requirements	1570

A.4.5.6	Active BWP switch	1571

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	1571

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	1571

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	1576

A.4.5.6.2	RRC-based Active BWP Switch	1582

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1586

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	1586

A.4.5.6.4	Simultaneous RRC-based Active BWP Switch on multiple CCs	1592

A.4.5.6.4.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	1592

A.4.5.6.4.1.1	Test Purpose and Environment	1592

A.4.5.6.4.1.2	Test Requirements	1599

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1599

A.4.5.6.4.2.1	Test Purpose and Environment	1599

A.4.5.6.4.2.2	Test Requirements	1609

A.4.5.6.5	SCell dormancy switch	1610

A.4.5.6.5.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	1610

A.4.5.6.5.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	1615

A.4.5.6.5.2.1	Test Purpose and Environment	1615

A.4.5.6.5.2.2	Test Requirements	1621

A.4.5.7	PSCell addition and release delay	1621

A.4.5.7.1	Addition and Release Delay of known NR PSCell	1621

A.4.5.7.1.1	Test purpose and environment	1621

A.4.5.7.1.2	Test Requirements	1626

A.4.5.8	DL Interruptions at switching between two uplink carriers	1627

A.4.5.8.1	Test Purpose and Environment	1627

A.4.5.8.2	Test Requirements	1630

A.4.5.9	UE specific CBW change	1631

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	1631

A.4.5.9.1.1	Test Purpose and Environment	1631

A.4.5.9.1.2	Test Requirements	1635

A.4.5.10	PSCell activation and deactivation delay	1635

A.4.5.10.1	PSCell activation and deactivation delay	1635

A.4.5.10.1.1	Test purpose and environment	1635

A.4.5.10.1.2	Test Requirements	1639

A.4.5.11	Conditional PSCell addition and release delay (FR1 EN-DC)	1640

A.4.5.11.1	Conditional PSCell Addition and Release Delay	1640

A.4.5.11.1.1	Test purpose and environment	1640

A.4.5.11.1.2	Test Parameters	1640

A.4.5.11.1.3	Test Requirements	1644

A.4.6	Measurement procedure	1644

A.4.6.1	Intra-frequency Measurements	1644

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	1644

A.4.6.1.1.1	Test purpose and Environment	1644

A.4.6.1.1.2	Test parameters	1644

A.4.6.1.1.3	Test Requirements	1648

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	1648

A.4.6.1.2.1	Test purpose and Environment	1648

A.4.6.1.2.2	Test parameters	1648

A.4.6.1.2.2	Test Requirements	1652

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	1652

A.4.6.1.3.1	Test purpose and Environment	1652

A.4.6.1.3.2	Test parameters	1652

A.4.6.1.3.3	Test Requirements	1656

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	1656

A.4.6.1.4.1	Test purpose and Environment	1656

A.4.6.1.4.2	Test parameters	1656

A.4.6.1.4.3	Test Requirements	1660

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	1660

A.4.6.1.5.1	Test purpose and Environment	1660

A.4.6.1.5.2	Test parameters	1660

A.4.6.1.5.3	Test Requirements	1662

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	1663

A.4.6.1.6.1	Test purpose and Environment	1663

A.4.6.1.6.2	Test parameters	1663

A.4.6.1.6.3	Test Requirements	1665

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1666

A.4.6.1.7.1	Test purpose and Environment	1666

A.4.6.1.7.2	Test parameters	1666

A.4.6.1.7.3	Test Requirements	1670

A.4.6.1.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasCA-Scell-r17	1670

A.4.6.1.8.1	Test Purpose and Environment	1670

A.4.6.1.8.2	Test Requirements	1675

A.4.6.1.9	EN-DC event triggered reporting tests without gap under non-DRX with NCD-SSB	1676

A.4.6.1.9.1	Test purpose and Environment	1676

A.4.6.1.9.2	Test parameters	1676

A.4.6.1.9.3	Test Requirements	1680

A.4.6.1.10	EN-DC event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP	1680

A.4.6.110.1	Test purpose and Environment	1680

A.4.6.1.10.2	Test Requirements	1680

A.4.6.2	Inter-frequency Measurements	1680

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	1680

A.4.6.2.1.1	Test Purpose and Environment	1680

A.4.6.2.1.2	Test Requirements	1685

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	1685

A.4.6.2.2.1	Test Purpose and Environment	1685

A.4.6.2.2.2	Test Requirements	1689

A.4.6.2.3	Void	1689

A.4.6.2.4	Void	1689

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	1689

A.4.6.2.5.1	Test Purpose and Environment	1689

A.4.6.2.5.2	Test Requirements	1694

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	1694

A.4.6.2.6.1	Test Purpose and Environment	1694

A.4.6.2.6.2	Test Requirements	1699

A.4.6.2.7	Void	1699

A.4.6.2.8	Void	1699

A.4.6.2.9	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	1700

A.4.6.2.9.1	Test Purpose and Environment	1700

A.4.6.2.9.2	Test Requirements	1703

A.4.6.3	Void	1703

A.4.6.4	L1-RSRP measurement for beam reporting	1703

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1703

A.4.6.4.1.1	Test Purpose and Environment	1703

A.4.6.4.1.2	Test parameters	1704

A.4.6.4.1.3	Test Requirements	1707

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	1707

A.4.6.4.2.1	Test Purpose and Environment	1707

A.4.6.4.2.2	Test parameters	1708

A.4.6.4.2.3	Test Requirements	1711

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1711

A.4.6.4.3.1	Test Purpose and Environment	1711

A.4.6.4.3.2	Test parameters	1712

A.4.6.4.3.3	Test Requirements	1715

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1715

A.4.6.4.4.1	Test Purpose and Environment	1715

A.4.6.4.4.2	Test parameters	1716

A.4.6.4.4.3	Test Requirements	1718

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1718

A.4.6.4.5.1	Test Purpose and Environment	1718

A.4.6.4.5.2	Test parameters	1719

A.4.6.4.5.3	Test Requirements	1722

A.4.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	1722

A.4.6.4.6.1	Test Purpose and Environment	1722

A.4.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	1723

A.4.6.4.7.1	Test Purpose and Environment	1723

A.4.6.4.7.2	Test Requirements	1723

A.4.6.4.8	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	1723

A.4.6.4.8.1	Test Purpose and Environment	1723

A.4.6.4.8.2	Test parameters	1723

A.4.6.4.8.3	Test Requirements	1727

A.4.6.5	CLI measurements	1727

A.4.6.5.1	SRS-RSRP measurement with non-DRX	1727

A.4.6.5.1.1	Test Purpose and Environment	1727

A.4.6.5.1.2	Test Parameters	1728

A.4.6.5.1.3	Test Requirements	1731

A.4.6.5.2	CLI-RSSI measurement with non-DRX	1731

A.4.6.5.2.1	Test Purpose and Environment	1731

A.4.6.5.2.2	Test Parameters	1732

A.4.6.5.2.3	Test Requirements	1733

A.4.6.6.1.2	Test Requirements	1738

A.4.6.7	L1-SINR measurement for beam reporting	1738

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1741

A.4.6.7.2.1	Test Purpose and Environment	1741

A.4.6.7.2.2	Test parameters	1741

A.4.6.7.2.3	Test Requirements	1744

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1744

A.4.6.7.3.1	Test Purpose and Environment	1744

A.4.6.7.3.2	Test parameters	1744

A.4.6.7.3.3	Test Requirements	1746

A.4.6.8	CSI-RS based intra-frequency Measurement	1747

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	1747

A.4.6.8.1.1	Test purpose and Environment	1747

A.4.6.8.1.2	Test Requirements	1751

A.4.6.9	CSI-RS based inter-frequency Measurement	1751

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	1751

A.4.6.9.1.1	Test Purpose and Environment	1751

A.4.6.9.1.2	Test Requirements	1755

A.4.7	Measurement Performance requirements	1718

A.4.7.1	SS-RSRP	1718

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1718

A.4.7.1.1.1	Test Purpose and Environment	1718

A.4.7.1.1.2	Test parameters	1718

A.4.7.1.1.3	Test Requirements	1722

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1722

A.4.7.1.2.1	Test Purpose and Environment	1722

A.4.7.1.2.2	Test parameters	1723

A.4.7.1.2.3	Test Requirements	1727

A.4.7.1.3	Void	1728

A.4.7.2	SS-RSRQ	1728

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1728

A.4.7.2.1.1	Test Purpose and Environment	1728

A.4.7.2.1.2	Test Parameters	1728

A.4.7.2.1.3	Test Requirements	1733

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1733

A.4.7.2.2.1	Test Purpose and Environment	1733

A.4.7.2.2.2	Test Parameters	1733

A.4.7.2.2.3	Test Requirements	1739

A.4.7.3	SS-SINR	1739

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1739

A.4.7.3.1.1	Test Purpose and Environment	1739

A.4.7.3.1.2	Test Parameters	1739

A.4.7.3.1.3	Test Requirements	1743

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1744

A.4.7.3.2.1	Test Purpose and Environment	1744

A.4.7.3.2.2	Test Parameters	1744

A.4.7.3.2.3	Test Requirements	1748

A.4.7.4	L1-RSRP measurement for beam reporting	1749

A.4.7.4.1	SSB based L1-RSRP measurement	1749

A.4.7.4.1.1	Test Purpose and Environment	1749

A.4.7.4.1.2	Test parameters	1749

A.4.7.4.1.3	Test Requirements	1752

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1752

A.4.7.4.2.1	Test Purpose and Environment	1752

A.4.7.4.2.2	Test parameters	1753

A.4.7.4.2.3	Test Requirements	1756

A.4.7.5	SFTD accuracy	1757

A.4.7.5.1	SFTD accuracy	1757

A.4.7.5.1.1	Test Purpose and Environment	1757

A.4.7.5.1.2	Test Parameters	1757

A.4.7.5.1.3	Test Requirements	1761

A.4.7.5.2	Void	1761

A.4.7.5.3	Void	1761

A.4.7.6	CLI measurements	1761

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	1761

A.4.7.6.1.1	Test Purpose and Environment	1761

A.4.7.6.1.2	Test parameters	1762

A.4.7.6.1.3	Test Requirements	1767

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	1767

A.4.7.6.2.1	Test Purpose and Environment	1767

A.4.7.6.2.2	Test parameters	1768

A.4.7.6.2.3	Test Requirements	1771

A.4.7.7	L1-SINR measurement for beam reporting	1771

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1775

A.4.7.7.2.1	Test Purpose and Environment	1775

A.4.7.7.2.2	Test parameters	1775

A.4.7.7.2.3	Test Requirements	1779

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1779

A.4.7.7.3.1	Test Purpose and Environment	1779

A.4.7.7.3.2	Test parameters	1779

A.4.7.7.3.3	Test Requirements	1782

A.4.7.8	CSI-RSRP	1783

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1783

A.4.7.8.1.1	Test Purpose and Environment	1783

A.4.7.8.1.2	Test parameters	1783

A.4.7.8.1.3	Test Requirements	1788

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1788

A.4.7.8.2.1	Test Purpose and Environment	1788

A.4.7.8.2.2	Test parameters	1788

A.4.7.8.2.3	Test Requirements	1793

A.4.7.9	CSI-RSRQ	1793

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1793

A.4.7.9.1.1	Test Purpose and Environment	1793

A.4.7.9.1.2	Test Parameters	1793

A.4.7.9.1.3	Test Requirements	1799

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1799

A.4.7.9.2.1	Test Purpose and Environment	1799

A.4.7.9.2.2	Test Parameters	1799

A.4.7.9.2.3	Test Requirements	1804

A.4.7.10	CSI-SINR	1805

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1805

A.4.7.10.1.1	Test Purpose and Environment	1805

A.4.7.10.1.2	Test Parameters	1805

A.4.7.10.1.3	Test Requirements	1809

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1810

A.4.7.10.2.1	Test Purpose and Environment	1810

A.4.7.10.2.2	Test Parameters	1810

A.4.7.10.2.3	Test Requirements	1815

A.4.7.11	TDCP amplitude measurement accuracy	1815

A.4.7.11.1	TDCP amplitude measurement accuracy in EN-DC	1815

A.4.7.11.1.1	Test Purpose and Environment	1815

A.4.7.11.1.2	Test parameters	1815

A.4.7.11.1.3	Test Requirements	1818

A.4.8	Void	1818

A.4A	NE-DC test with all NR cells in FR1	1818

A.4A.1	Signaling characteristics	1818

A.4A.1.1	E-UTRAN PSCell addition	1818

A.4A.1.1.1	Test purpose and environment	1818

A.4A.1.1.2	Test Requirements	1823

A.4A.1.2	Active BWP switch	1824

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	1824

A.4A.1.2.1.1	Test Purpose and Environment	1824

A.4A.1.2.1.2	Test Requirements	1828

A.4A.1.3	Intra-frequency handover with E-UTRAN PSCell	1829

A.4A.1.3.1	Test purpose and environment	1829

A.4A.1.3.2	Test Requirements	1833

A.4A.1.4	Handover with PSCell from NE-DC to NE-DC with unknown target PSCell	1834

A.4A.1.4.1	Test Purpose and Environment	1834

A.4A.1.4.2	Test Parameters	1834

A.4A.1.4.3	Test Requirements	1839

A.4A.1.4.3.1	Test Requirements for NR HO	1839

A.4A.1.4.3.2	Test Requirements for LTE PSCell Change	1839

A.4A.2	Measurement performance	1840

A.4A.2.1	SFTD accuracy	1840

A.4A.2.1.1	SFTD accuracy	1840

A.4A.2.1.1.1	Test Purpose	1840

A.4A.2.1.1.2	Test Environment	1840

A.4A.2.1.1.3	Test Requirements	1844

A.5	EN-DC tests with one or more NR cells in FR2	1844

A.5.1	Void	1844

A.5.2	Void	1844

A.5.3	RRC_CONNECTED state mobility	1844

A.5.3.1	Void	1844

A.5.3.2	RRC Connection Mobility Control	1844

A.5.3.2.1	Void	1844

A.5.3.2.2	Random Access	1844

A.5.3.2.2.1	4-step RA type c ontention based random access test in FR2 for PSCell/SCell in EN-DC	1844

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1848

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	1854

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	1857

A.5.3.2.3	Void	1861

A.5.3.3	Handover with PSCell with known FR2 target PSCell	1861

A.5.3.3.1	Test purpose and environment	1861

A.5.3.3.2	Test Requirements	1866

A.5.3.3.3	Void	1867

A.5.3.3.4	Void	1867

A.5.3.3.5	Void	1867

A.5.3.3.6	Void	1867

A.5.4	Timing	1867

A.5.4.1	UE transmit timing	1867

A.5.4.1.1	NR UE Transmit Timing Test for FR2	1867

A.5.4.1.1.1	Test Purpose and environment	1867

A.5.4.1.1.2	Test requirements	1870

A.5.4.1.2	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	1871

A.5.4.1.2.1	Test Purpose and environment	1871

A.5.4.1.2.2	Test requirements	1875

A.5.4.2	UE timer accuracy	1876

A.5.4.3	Timing advance	1876

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	1876

A.5.4.3.1.1 Test Purpose and Environment	1876

A.5.4.3.1.2 Test Parameters	1876

A.5.4.3.1.3	Test Requirements	1880

A.5.5	Signaling characteristics	1880

A.5.5.1	Radio link Monitoring	1880

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1880

A.5.5.1.1.1	Test Purpose and Environment	1880

A.5.5.1.1.2	Test Requirements	1884

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	1885

A.5.5.1.2.1	Test Purpose and Environment	1885

A.5.5.1.2.2	Test Requirements	1888

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1889

A.5.5.1.3.1	Test Purpose and Environment	1889

A.5.5.1.3.2	Test Requirements	1893

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	1893

A.5.5.1.4.1	Test Purpose and Environment	1893

A.5.5.1.4.2	Test Requirements	1897

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1897

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	1901

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1905

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1910

A.5.5.1.8.2	Test Requirements	1914

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1915

A.5.5.1.9.1	Test Purpose and Environment	1915

A.5.5.1.9.2	Test Requirements	1917

A.5.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	1917

A.5.5.1.10.1	Test Purpose and Environment	1917

A.5.5.1.10.2	Test Requirements	1920

A.5.5.1.11	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	1920

A.5.5.1.11.1	Test Purpose and Environment	1920

A.5.5.1.11.2	Test Requirements	1925

A.5.5.1.12	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	1926

A.5.5.1.12.1	Test Purpose and Environment	1926

A.5.5.1.13	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	1926

A.5.5.1.13.1	Test Purpose and Environment	1926

A.5.5.1.13.2	Test Requirements	1926

A.5.5.1.14	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode for UE supporting [FG 53-3]	1927

A.5.5.1.14.1	Test Purpose and Environment	1927

A.5.5.1.14.2	Test Requirements	1931

A.5.5.2	Interruption	1931

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1931

A.5.5.2.1.1	Test Purpose and Environment	1931

A.5.5.2.1.2	Test Requirements	1934

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1934

A.5.5.2.2.1	Test Purpose and Environment	1934

A.5.5.2.2.2	Test Requirements	1937

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1937

A.5.5.2.3.1	Test Purpose and Environment	1937

A.5.5.2.3.2	Test Requirements	1941

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1941

A.5.5.2.4.1	Test Purpose and Environment	1941

A.5.5.2.4.2	Test Requirements	1944

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1945

A.5.5.2.5.1	Test Purpose and Environment	1945

A.5.5.2.5.2	Test Requirements	1948

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1948

A.5.5.2.6.1	Test Purpose and Environment	1948

A.5.5.2.6.2	Test Requirements	1951

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1951

A.5.5.2.7.1	Test Purpose and Environment	1951

A.5.5.2.7.2	Test Requirements	1955

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1955

A.5.5.2.8.1 Test Purpose and Environment	1955

A.5.5.2.8.3	Test Requirements	1957

A.5.5.2.9	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR PSCell	1957

A.5.5.2.9.1	Test Purpose and Environment	1957

A.5.5.2.9.2	Test Requirements	1962

A.5.5.3	SCell Activation and Deactivation Delay	1962

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1962

A.5.5.3.1.1	Test Purpose and Environment	1962

A.5.5.3.1.2	Test Requirements	1964

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1964

A.5.5.3.2.1	Test Purpose and Environment	1964

A.5.5.3.2.2	Test Requirements	1968

A.5.5.3.3	Void	1968

A.5.5.3.4	Void	1968

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1968

A.5.5.3.5.1	Test Purpose and Environment	1968

A.5.5.3.5.2	Test Requirements	1971

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1972

A.5.5.3.6.1	Test Purpose and Environment	1972

A.5.5.3.6.2	Test Requirements	1976

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1977

A.5.5.3.7.1	Test Purpose and Environment	1977

A.5.5.3.7.2	Test Requirements	1980

A.5.5.3.8	Fast SCell Activation of SCell in FR2 intra-band	1980

A.5.5.3.8.1	Test Purpose and Environment	1980

A.5.5.3.8.2	Test Requirements	1984

A.5.5.3.9	PUCCH SCell Activation and deactivation of known SCell in FR2	1985

A.5.5.3.9.1	Test Purpose and Environment	1985

A.5.5.3.9.2	Test Requirements	1989

A.5.5.3.10	PUCCH SCell Activation and deactivation of unknown SCell in FR2	1990

A.5.5.3.10.1	Test Purpose and Environment	1990

A.5.5.3.10.2	Test Requirements	1993

A.5.5.3.11	Multiple SCell activation and deactivation of one known PUCCH SCell and one unknown SCell in FR2	1994

A.5.5.3.11.1	Test Purpose and Environment	1994

A.5.5.3.11.2	Test Requirements	1998

A.5.5.3.12	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR2 in non-DRX	1999

A.5.5.3.12.1	Test Purpose and Environment	1999

A.5.5.3.12.2	Test Requirements	2002

A.5.5.3.13	SCell Activation and deactivation of unknown SCell in FR2 for UE in DRX, capable of small beam sweeping factors and/or short measurement interval	2003

A.5.5.3.13.1	Test Purpose and Environment	2003

A.5.5.3.13.2	Test Requirements	2008

A.5.5.3.14	PUCCH SCell activation and deactivation with FR1 PSCell based on L3 reporting after SCell activation command	2010

A.5.5.3.14.1	Test Purpose and Environment	2010

A.5.5.3.14.2	Test Requirements	2016

A.5.5.3.15	SCell Activation of unknown SCell in FR2 in non-DRX for 160ms SCell measurement cycle with the L3 reporting during activation	2017

A.5.5.3.15.1	Test Purpose and Environment	2017

A.5.5.3.15.2	Test Requirements	2021

A.5.5.4	Void	2022

A.5.5.5	Beam Failure Detection and Link recovery procedures	2022

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	2022

A.5.5.5.1.1	Test Purpose and Environment	2022

A.5.5.5.1.2	Test Requirements	2026

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	2027

A.5.5.5.2.1	Test Purpose and Environment	2027

A.5.5.5.2.2	Test Requirements	2031

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	2032

A.5.5.5.3.1	Test Purpose and Environment	2032

A.5.5.5.3.2	Test Requirements	2036

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2037

A.5.5.5.4.1	Test Purpose and Environment	2037

A.5.5.5.4.2	Test Requirements	2041

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	2042

A.5.5.5.5.1	Test Purpose and Environment	2042

A.5.5.5.5.2	Test Requirements	2046

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	2047

A.5.5.5.6.1	Test Purpose and Environment	2047

A.5.5.5.6.2	Test Requirements	2051

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	2052

A.5.5.5.7.1	Test Purpose and Environment	2052

A.5.5.5.7.2	Test Requirements	2056

A.5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	2057

A.5.5.5.8.1	Test Purpose and Environment	2057

A.5.5.5.8.2	Test Requirements	2062

A.5.5.5.9	Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	2062

A.5.5.5.9.1	Test Purpose and Environment	2062

A.5.5.5.9.2	Test Requirements	2066

A.5.5.6	Active BWP switch	2066

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	2066

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2066

A.5.5.6.1.1.1	Test Purpose and Environment	2066

A.5.5.6.1.1.2	Test Requirements	2070

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	2070

A.5.5.6.2	RRC-based Active BWP Switch	2075

A.5.5.6.2.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	2075

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2079

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	2079

A.5.5.6.4	SCell dormancy switch	2083

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	2083

A.5.5.6.4.1.1	Test Purpose and Environment	2083

A.5.5.6.4.1.2	Test Requirements	2087

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	2087

A.5.5.6.4.2.1	Test Purpose and Environment	2087

A.5.5.6.4.2.2	Test Requirements	2094

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2094

A.5.5.6.5.1	E-UTRAN – NR PSCell FR2  and NR SCell FR2 DL active BWP switch on multiple CCs with non-DRX in synchronous EN-DC	2094

A.5.5.7	PSCell addition and release delay	2097

A.5.5.7.1	Addition and Release Delay of NR PSCell	2097

A.5.5.7.1.1	Test purpose and environment	2097

A.5.5.7.1.2	Test Requirements	2100

A.5.5.8	Active TCI state switch delay	2101

A.5.5.8.1	MAC-CE based active TCI state switch	2101

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2101

A.5.5.8.1.1.1	Test Purpose and Environment	2101

A.5.5.8.1.1.2	Test Requirements	2105

A.5.5.8.2	RRC based active TCI state switch	2105

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	2105

A.5.5.8.2.1.1	Test Purpose and Environment	2105

A.5.5.8.2.1.2	Test Requirements	2109

A.5.5.9	Uplink spatial relation switch delay	2109

A.5.5.9.1	MAC-CE based uplink spatial relation switch	2109

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	2109

A.5.5.9.1.1.1	Test Purpose and Environment	2109

A.5.5.9.1.1.2	Test Requirements	2112

A.5.5.9.2	RRC based spatial relation switch	2112

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	2112

A.5.5.9.2.1.1	Test Purpose and Environment	2112

A.5.5.9.2.1.2	Test Requirements	2115

A.5.5.10	UE specific CBW change	2115

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	2115

A.5.5.10.1.1	Test Purpose and Environment	2115

A.5.5.10.1.2	Test Requirements	2118

A.5.5.11	Unified TCI state switch delay	2119

A.5.5.11.1	MAC-CE based active joint TCI state switch	2119

A.5.5.11.1.1	E-UTRAN – NR PSCell FR2 active joint TCI state switch for a known TCI state	2119

A.5.5.11.1.1.1	Test Purpose and Environment	2119

A.5.5.11.1.1.2	Test parameters	2119

A.5.5.11.1.1.3	Test Requirements	2122

A.5.5.11.2	MAC-CE based active uplink TCI state switch	2122

A.5.5.11.2.1	E-UTRAN – NR PSCell FR2 active uplink TCI state switch for a known TCI state	2122

A.5.5.11.2.1.1	Test Purpose and Environment	2122

A.5.5.11.2.1.2	Test parameters	2123

A.5.5.11.2.1.3	Test Requirements	2125

A.5.5.11.3	MAC-CE based active downlink TCI state switch	2125

A.5.5.11.3.1	E-UTRAN – NR PSCell FR2 downlink TCI state switch to cell with additional PCI for a known TCI state	2125

A.5.5.11.3.1.1	Test Purpose and Environment	2125

A.5.5.11.3.1.2	Test Parameters	2126

A.5.5.11.3.1.3	Test Requirements	2130

A.5.5.12	PSCell activation and deactivation delay	2130

A.5.5.12.1	PSCell activation and deactivation delay	2130

A.5.5.12.1.1	Test purpose and environment	2130

A.5.5.12.1.2	Test Requirements	2133

A.5.5.13	Conditional PSCell addition and release delay	2133

A.5.5.13.1	Addition and Release Delay of NR PSCell	2133

A.5.5.13.1.1	Test purpose and environment	2133

A.5.5.13.1.2	Test Requirements	2136

A.5.6	Measurement procedure	2137

A.5.6.1	Intra-frequency Measurements	2137

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	2137

A.5.6.1.1.1	Test purpose and Environment	2137

A.5.6.1.1.2	Test Requirements	2140

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	2140

A.5.6.1.2.1	Test purpose and Environment	2140

A.5.6.1.2.2	Test Requirements	2142

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	2143

A.5.6.1.3.1	Test purpose and Environment	2143

A.5.6.1.3.2	Test Requirements	2147

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	2147

A.5.6.1.4.1	Test purpose and Environment	2147

A.5.6.1.4.2	Test Requirements	2150

A.5.6.1.5	EN-DC event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP	2151

A.5.6.1.5.1	Test purpose and Environment	2151

A.5.6.1.5.2	Test Requirements	2151

A.5.6.1.6	EN-DC event triggered reporting test without gap under non-DRX	2151

A.5.6.1.6.1	Test purpose and Environment	2151

A.5.6.1.6.2	Test Requirements	2155

A.5.6.2	Inter-frequency Measurements	2155

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2155

A.5.6.2.1.1	Test Purpose and Environment	2155

A.5.6.2.1.2	Test Requirements	2158

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2158

A.5.6.2.2.1	Test Purpose and Environment	2158

A.5.6.2.2.2	Test Requirements	2162

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2162

A.5.6.2.3.1	Test Purpose and Environment	2162

A.5.6.2.3.2	Test Requirements	2166

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2166

A.5.6.2.4.1	Test Purpose and Environment	2166

A.5.6.2.4.2	Test Requirements	2170

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	2170

A.5.6.2.5.1	Test Purpose and Environment	2170

A.5.6.2.5.2	Test Requirements	2175

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	2175

A.5.6.2.6.1	Test Purpose and Environment	2175

A.5.6.2.6.2	Test Requirements	2179

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	2179

A.5.6.2.7.1	Test Purpose and Environment	2179

A.5.6.2.7.2	Test Requirements	2184

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	2184

A.5.6.2.8.1	Test Purpose and Environment	2184

A.5.6.2.8.2	Test Requirements	2189

A.5.6.3	L1-RSRP measurement for beam reporting	2190

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	2190

A.5.6.3.1.1	Test Purpose and Environment	2190

A.5.6.3.1.2	Test parameters	2190

A.5.6.3.1.3	Test Requirements	2192

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	2192

A.5.6.3.2.1	Test Purpose and Environment	2192

A.5.6.3.2.2	Test parameters	2193

A.5.6.3.2.3	Test Requirements	2195

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	2195

A.5.6.3.3.1	Test Purpose and Environment	2195

A.5.6.3.3.2	Test parameters	2196

A.5.6.3.3.3	Test Requirements	2198

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	2198

A.5.6.3.4.1	Test Purpose and Environment	2198

A.5.6.3.4.2	Test parameters	2199

A.5.6.3.4.3	Test Requirements	2201

A.5.6.3.5	CSI-RS based L1-RSRP measurement when DRX is not used and when CD-SSB is outside active BWP	2202

A.5.6.3.5.1	Test Purpose and Environment	2202

A.5.6.3.6	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2202

A.5.6.3.6.1	Test Purpose and Environment	2202

A.5.6.3.6.2	Test Requirements	2202

A.5.6.3.7	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	2202

A.5.6.3.7.1	Test Purpose and Environment	2202

A.5.6.3.7.2	Test parameters	2203

A.5.6.3.7.3	Test Requirements	2205

A.5.6.4	CLI measurements	2205

A.5.6.4.1	SRS-RSRP measurement with DRX	2205

A.5.6.4.1.1	Test Purpose and Environment	2205

A.5.6.4.1.2	Test Parameters	2206

A.5.6.4.1.3	Test Requirements	2208

A.5.6.4.2	CLI-RSSI measurement with DRX	2208

A.5.6.4.2.1	Test Purpose and Environment	2208

A.5.6.4.2.2	Test Parameters	2209

A.5.6.4.2.3	Test Requirements	2211

A.5.6.5	Measurements with autonomous gaps	2211

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	2211

A.5.6.5.1.1	Test Purpose and Environment	2211

A.5.6.5.1.2	Test Requirements	2214

A.5.6.6	L1-SINR measurement for beam reporting	2215

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2217

A.5.6.6.2.1	Test Purpose and Environment	2217

A.5.6.6.2.2	Test parameters	2218

A.5.6.6.2.3	Test Requirements	2221

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2221

A.5.6.6.3.1	Test Purpose and Environment	2221

A.5.6.6.3.2	Test parameters	2222

A.5.6.6.3.3	Test Requirements	2224

A.5.6.7	CSI-RS based Intra-frequency Measurements	2224

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	2224

A.5.6.7.1.1	Test purpose and Environment	2224

A.5.6.7.1.2	Test Requirements	2227

A.5.6.8	CSI-RS based Inter-frequency Measurements	2227

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	2227

A.5.6.8.1.1	Test Purpose and Environment	2227

A.5.6.8.1.2	Test Requirements	2232

A.5.7	Measurement Performance requirements	2232

A.5.7.1	SS-RSRP	2232

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2232

A.5.7.1.1.1	Test Purpose and Environment	2232

A.5.7.1.1.2	Test parameters	2233

A.5.7.1.1.3	Test Requirements	2235

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2236

A.5.7.1.2.1	Test Purpose and Environment	2236

A.5.7.1.2.2	Test parameters	2236

A.5.7.1.2.3	Test Requirements	2240

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2241

A.5.7.1.3.1	Test Purpose and Environment	2241

A.5.7.1.3.2	Test parameters	2241

A.5.7.1.3.3	Test Requirements	2244

A.5.7.2	SS-RSRQ	2245

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2245

A.5.7.2.1.1	Test Purpose and Environment	2245

A.5.7.2.1.2	Test Parameters	2245

A.5.7.2.1.3	Test Requirements	2248

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2248

A.5.7.2.2.1	Test Purpose and Environment	2248

A.5.7.2.2.2	Test Parameters	2248

A.5.7.2.2.3	Test Requirements	2250

A.5.7.3	SS-SINR	2251

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2251

A.5.7.3.1.1	Test Purpose and Environment	2251

A.5.7.3.1.2	Test Parameters	2251

A.5.7.3.1.3	Test Requirements	2254

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2254

A.5.7.3.2.1	Test Purpose and Environment	2254

A.5.7.3.2.2	Test Parameters	2254

A.5.7.3.2.3	Test Requirements	2256

A.5.7.4	L1-RSRP measurement for beam reporting	2256

A.5.7.4.1	SSB based L1-RSRP measurement	2256

A.5.7.4.1.1	Test Purpose and Environment	2256

A.5.7.4.1.2	Test parameters	2257

A.5.7.4.1.3	Test Requirements	2259

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2260

A.5.7.4.2.1	Test Purpose and Environment	2260

A.5.7.4.2.2	Test parameters	2260

A.5.7.4.2.3	Test Requirements	2262

A.5.7.5	CLI measurements	2263

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	2263

A.5.7.5.1.1	Test Purpose and Environment	2263

A.5.7.5.1.2	Test parameters	2263

A.5.7.5.1.3	Test Requirements	2266

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	2267

A.5.7.5.2.1	Test Purpose and Environment	2267

A.5.7.5.2.2	Test parameters	2267

A.5.7.5.2.3	Test Requirements	2269

A.5.7.6	L1-SINR measurement for beam reporting	2270

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2273

A.5.7.6.2.1	Test Purpose and Environment	2273

A.5.7.6.2.2	Test parameters	2273

A.5.7.6.2.3	Test Requirements	2275

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2276

A.5.7.6.3.1	Test Purpose and Environment	2276

A.5.7.6.3.2	Test parameters	2276

A.5.7.6.3.3	Test Requirements	2278

A.5.7.7	CSI-RSRP	2279

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2279

A.5.7.7.1.1	Test Purpose and Environment	2279

A.5.7.7.1.2	Test parameters	2279

A.5.7.7.1.3	Test Requirements	2281

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2282

A.5.7.7.2.1	Test Purpose and Environment	2282

A.5.7.7.2.2	Test parameters	2282

A.5.7.7.2.3	Test Requirements	2286

A.5.7.8	CSI-RSRQ	2287

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2287

A.5.7.8.1.1	Test Purpose and Environment	2287

A.5.7.8.1.2	Test Parameters	2287

A.5.7.8.1.3	Test Requirements	2289

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2289

A.5.7.8.2.1	Test Purpose and Environment	2289

A.5.7.8.2.2	Test Parameters	2289

A.5.7.8.2.3	Test Requirements	2291

A.5.7.9	CSI-SINR	2291

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2291

A.5.7.9.1.1	Test Purpose and Environment	2291

A.5.7.9.1.2	Test Parameters	2291

A.5.7.9.1.3	Test Requirements	2294

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2294

A.5.7.9.2.1	Test Purpose and Environment	2294

A.5.7.9.2.2	Test Parameters	2294

A.5.7.9.2.3	Test Requirements	2297

A.5.8	Void	2297

A.6	NR standalone tests with all NR cells in FR1	2244

A.6.1	SA: RRC_IDLE state mobility	2244

A.6.1.1	Cell re-selection to NR	2244

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	2244

A.6.1.1.1.1	Test Purpose and Environment	2244

A.6.1.1.1.2	Test Parameters	2244

A.6.1.1.1.3	Test Requirements	2248

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	2248

A.6.1.1.2.1	Test Purpose and Environment	2248

A.6.1.1.2.2	Test Parameters	2248

A.6.1.1.2.3	Test Requirements	2252

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2252

A.6.1.1.3.1	Test Purpose and Environment	2252

A.6.1.1.3.2	Test Parameters	2252

A.6.1.1.3.3	Test Requirements	2257

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2257

A.6.1.1.4.1	Test Purpose and Environment	2257

A.6.1.1.4.2	Test Parameters	2257

A.6.1.1.4.3	Test Requirements	2260

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	2260

A.6.1.1.5.1	Test Purpose and Environment	2260

A.6.1.1.5.2	Test Parameters	2260

A.6.1.1.5.3	Test Requirements	2265

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	2265

A.6.1.1.6.1	Test Purpose and Environment	2265

A.6.1.1.6.2	Test Parameters	2266

A.6.1.1.6.3	Test Requirements	2270

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	2271

A.6.1.1.7.1	Test Purpose and Environment	2271

A.6.1.1.7.2	Test Parameters	2271

A.6.1.1.7.3	Test Requirements	2274

A.6.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE configured with highSpeedMeasInterFreq-r17	2274

A.6.1.1.8.1	Test Purpose and Environment	2274

A.6.1.1.8.2	Test Parameters	2274

A.6.1.1.8.3	Test Requirements	2278

A.6.1.1.9	Cell reselection to FR1 intra-frequency NR case for UE operating on a cell with less than 5MHz BW	2278

A.6.1.1.9.1	Test Purpose and Environment	2278

A.6.1.1.9.3	Test Requirements	2279

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	2280

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	2280

A.6.1.2.1.1	Test Purpose and Environment	2280

A.6.1.2.1.2	Test Parameters	2280

A.6.1.2.1.3	Test Requirements	2283

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	2284

A.6.1.2.2.1	Test Purpose and Environment	2284

A.6.1.2.2.2	Test Parameters	2284

A.6.1.2.2.3	Test Requirements	2287

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	2288

A.6.1.2.3.1	Test Purpose and Environment	2288

A.6.1.2.3.2	Test Parameters	2288

A.6.1.2.3.3	Test Requirements	2291

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	2292

A.6.1.2.4.1	Test Purpose and Environment	2292

A.6.1.2.4.2	Test Parameters	2292

A.6.1.2.4.3	Test Requirements	2295

A.6.1.2.5	Cell reselection to lower priority E-UTRAN cell for UE configured with highSpeedMeasFlag-r16	2296

A.6.1.2.5.1	Test Purpose and Environment	2296

A.6.1.2.5.2	Test Parameters	2296

A.6.1.2.5.3	Test Requirements	2300

A.6.1.1.7	Void	2301

A.6.2	SA: RRC_INACTIVE state mobility	2301

A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)	2301

A.6.2.1.1	Test purpose and Environment	2301

A.6.2.1.2	Test Parameters	2302

A.6.2.1.3	Test requirements	2304

A.6.3	RRC_CONNECTED state mobility	2304

A.6.3.1	Handover	2304

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	2304

A.6.3.1.1.1	Test Purpose and Environment	2304

A.6.3.1.1.2	Test Parameters	2304

A.6.3.1.1.3 Test Requirements	2308

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	2308

A.6.3.1.2.1	Test Purpose and Environment	2308

A.6.3.1.2.2	Test Parameters	2308

A.6.3.1.2.3	Test Requirements	2312

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	2312

A.6.3.1.3.1	Test Purpose and Environment	2312

A.6.3.1.3.2	Test Parameters	2312

A.6.3.1.3.3	Test Requirements	2316

A.6.3.1.4	SA NR - E-UTRAN handover	2316

A.6.3.1.4.1	Test Purpose and Environment	2316

A.6.3.1.4.2	Test Requirements	2322

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	2322

A.6.3.1.5.1	Test Purpose and Environment	2322

A.6.3.1.5.2	Test Requirements	2328

A.6.3.1.6	 SA NR - UTRAN FDD handover	2328

A.6.3.1.6.1	Test Purpose and Environment	2328

A.6.3.1.6.2	Test Requirements	2332

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	2332

A.6.3.1.7.1	Test Purpose and Environment	2332

A.6.3.1.7.2	Test Parameters	2332

A.6.3.1.7.3	Test Requirements	2336

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	2337

A.6.3.1.8.1	Test Purpose and Environment	2337

A.6.3.1.8.2	Test Parameters	2337

A.6.3.1.8.3	Test Requirements	2341

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	2342

A.6.3.1.9.1	Test Purpose and Environment	2342

A.6.3.1.9.2	Test Parameters	2342

A.6.3.1.9.3	Test Requirements	2346

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	2346

A.6.3.1.10.1	Test Purpose and Environment	2346

A.6.3.1.10.2	Test Parameters	2346

A.6.3.1.10.3	Test Requirements	2349

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	2349

A.6.3.1.11.1	Test Purpose and Environment	2349

A.6.3.1.11.2	Test Parameters	2349

A.6.3.1.11.3 Test Requirements	2356

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	2356

A.6.3.1.12.1	Test Purpose and Environment	2356

A.6.3.1.12.2	Test Parameters	2356

A.6.3.1.12.3 Test Requirements	2364

A.6.3.1.13	SA NR - E-UTRAN with NR PSCell addition in FR1	2364

A.6.3.1.13.1	Test Purpose and Environment	2364

A.6.3.1.13.2	Test Requirements	2373

A.6.3.1.14	SA NR - E-UTRAN handover with NR FR1 PScell addition	2373

A.6.3.1.14.1	Test Purpose and Environment	2373

A.6.3.1.14.2	Test Requirements	2382

A.6.3.1.15	Intra-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB	2383

A.6.3.1.15.1	Test Purpose and Environment	2383

A.6.3.1.15.2	Test Parameters	2383

A.6.3.1.15.3	Test Requirements	2386

A.6.3.1.16	Inter-frequency handover from FR1 to FR1; known target cell configured with NCD-SSB	2386

A.6.3.1.16.1	Test Purpose and Environment	2386

A.6.3.1.16.2	Test Parameters	2386

A.6.3.1.16.3 Test Requirements	2390

A.6.3.1.17	Handover with PSCell change delay from NR-DC (FR1-FR1) to NR-DC (FR1-FR1)	2390

A.6.3.1.17.1	Test Purpose and Environment	2390

A.6.3.1.17.2	Test Requirements	2397

A.6.3.1.18	Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth	2398

A.6.3.1.18.2	Test Parameters	2398

A.6.3.1.18.3	Test Requirements	2399

A.6.3.2	RRC Connection Mobility Control	2399

A.6.3.2.1	SA: RRC Re-establishment	2399

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	2399

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	2403

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	2407

A.6.3.2.2	Random Access	2410

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	2410

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	2414

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	2419

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	2424

A.6.3.2.3	SA: RRC Connection Release with Redirection	2428

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	2428

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	2432

A.6.3.2.4	LTM PDCCH-order Random Access	2439

A.6.3.2.4.1	PDCCH-order RACH on neighbor cell in FR1 when RACH BW is within active UL BWP	2439

A.6.3.2.4.2	PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 for LTM	2444

A.6.3.2.4.3	PDCCH-order RACH on neighbor cell without L1-RSRP measurement in FR1 when RACH BW is within active UL BWP	2449

A.6.3.3	Conditional handover	2454

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	2454

A.6.3.3.1.1	Test Purpose and Environment	2454

A.6.3.3.1.2	Test Parameters	2455

A.6.3.3.1.3 Test Requirements	2458

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	2458

A.6.3.3.2.1	Test Purpose and Environment	2458

A.6.3.3.2.2	Test Parameters	2458

A.6.3.3.2.3 Test Requirements	2462

A.6.3.3.3	NR conditional handover including target MCG and target SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2462

A.6.3.3.3.1	Test Purpose and Environment	2462

A.6.3.3.3.2	Test Requirements	2469

A.6.3.3.4	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2469

A.6.3.3.4.1	Test Purpose and Environment	2469

A.6.3.3.4.2	Test Parameters	2470

A.6.3.3.4.3	Test Requirements	2476

A.6.3.3.5	NR conditional handover including target MCG and candidate SCG from FR1-FR1 NR-DC to FR1-FR1 NR-DC with complementary conditional handover configuration	2477

A.6.3.3.5.1	Test Purpose and Environment	2477

A.6.3.3.5.2	Test Parameters	2477

A.6.3.3.5.3	Test Requirements	2484

A.6.3.3.6	NES triggering intra-frequency conditional handover from FR1 to FR1	2484

A.6.3.3.6.1	Test Purpose and Environment	2484

A.6.3.3.6.2	Test Parameters	2484

A.6.3.3.6.3 Test Requirements	2488

A.6.3.3.7	NES-based Inter-frequency conditional handover from FR1 to FR1	2488

A.6.3.3.7.1	Test Purpose and Environment	2488

A.6.3.3.7.2	Test Parameters	2488

A.6.3.3.7.3 Test Requirements	2492

A.6.3.4	LTM PCell Switch	2492

A.6.3.4.1	RACH-based Intra-frequency PCell switch from FR1 to FR1	2492

A.6.3.4.1.1	Test Purpose and Environment	2492

A.6.3.4.1.2	Test Parameters	2492

A.6.3.4.1.3	Test Requirements	2497

A.6.3.4.2	RACH based Inter-frequency LTM PCell switch from FR1 to FR1	2497

A.6.3.4.2.1	Test Purpose and Environment	2497

A.6.3.4.2.2	Test Parameters	2497

A.6.3.4.2.3 Test Requirements	2502

A.6.3.4.3	RACH-less Intra-frequency PCell switch from FR1 to FR1	2502

A.6.3.4.3.1	Test Purpose and Environment	2502

A.6.3.4.3.2	Test Parameters	2502

A.6.3.4.3.3	Test Requirements	2508

A.6.3.4.4	RACH-less Intra-frequency PCell switch from FR1 to FR1 with L1-RSRP measurement	2508

A.6.3.4.4.1	Test Purpose and Environment	2508

A.6.3.4.4.2	Test Parameters	2508

A.6.3.4.4.3	Test Requirements	2514

A.6.3.5	LTM PSCell Switch	2514

A.6.3.5.1 RACH-based intra-frequency LTM PSCell switch from FR1 to FR1	2514

A.6.3.5.1.1	Test Purpose and Environment	2514

A.6.3.5.1.2	Test Parameters	2514

A.6.3.5.1.3	Test Requirements	2520

A.6.4	Timing	2520

A.6.4.1	UE transmit timing	2520

A.6.4.1.1	NR UE Transmit Timing Test for FR1	2520

A.6.4.1.1.1	Test Purpose and environment	2520

A.6.4.1.1.2	Test requirements	2525

A.6.4.1.2	NR UE Transmit Timing Test for two TRPs in FR1	2525

A.6.4.1.2.1	Test Purpose and environment	2525

A.6.4.1.2.2	Test requirements	2530

A.6.4.2	UE timer accuracy	2531

A.6.4.3	Timing advance	2531

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	2531

A.6.4.3.1.1	Test Purpose and Environment	2531

A.6.4.3.1.2	Test Parameters	2531

A.6.4.3.1.3	Test Requirements	2535

A.6.5	Signalling characteristics	2535

A.6.5.1	Radio link Monitoring	2535

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2536

A.6.5.1.1.1	Test Purpose and Environment	2536

A.6.5.1.1.2	Test Requirements	2541

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	2541

A.6.5.1.2.1	Test Purpose and Environment	2541

A.6.5.1.2.2	Test Requirements	2547

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2547

A.6.5.1.3.1	Test Purpose and Environment	2547

A.6.5.1.3.2	Test Requirements	2553

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	2553

A.6.5.1.4.1	Test Purpose and Environment	2553

A.6.5.1.4.2	Test Requirements	2559

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2559

A.6.5.1.5.1	Test Purpose and Environment	2559

A.6.5.1.5.2	Test Requirements	2565

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	2565

A.6.5.1.6.1	Test Purpose and Environment	2565

A.6.5.1.6.2	Test Requirements	2570

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2570

A.6.5.1.7.1	Test Purpose and Environment	2570

A.6.5.1.7.2	Test Requirements	2574

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	2574

A.6.5.1.8.1	Test Purpose and Environment	2574

A.6.5.1.8.2	Test Requirements	2580

A.6.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	2580

A.6.5.1.9.1	Test Purpose and Environment	2580

A.6.5.1.9.2	Test Requirements	2586

A.6.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	2586

A.6.5.1.10.1	Test Purpose and Environment	2586

A.6.5.1.11	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	2587

A.6.5.1.11.1	Test Purpose and Environment	2587

A.6.5.1.11.2	Test Requirements	2587

A.6.5.1.12	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting [FG 53-3]	2587

A.6.5.1.12.1	Test Purpose and Environment	2587

A.6.5.1.12.2	Test Requirements	2591

A.6.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for UE operating on a cell with less than 5MHz BW	2591

A.6.5.1.13.1	Test Purpose and Environment	2591

A.6.5.1.13.2	Test Requirements	2592

A.6.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW	2593

A.6.5.1.14.1	Test Purpose and Environment	2593

A.6.5.1.14.2	Test Requirements	2594

A.6.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode	2594

A.6.5.1.15.1	Test Purpose and Environment	2594

A.6.5.1.15.2	Test Requirements	2595

A.6.5.2	Interruption	2595

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	2595

A.6.5.2.1.2	Test Requirements	2599

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	2600

A.6.5.2.2.1	Test Purpose and Environment	2600

A.6.5.2.2.2	Test Parameters	2600

A.6.5.2.2.3	Test Requirements	2602

A.6.5.2.3	SA interruptions at NR SRS antenna port switching with 1 SRS symbol in a slot in NR-CA	2603

A.6.5.2.3.1	Test Purpose and Environment	2603

A.6.5.2.3.2	Test Parameters	2603

A.6.5.2.3.3	Test Requirements	2605

A.6.5.2.4	SA interruptions at NR SRS antenna port switching	2606

A.6.5.2.4.1	Test Purpose and Environment	2606

A.6.5.2.4.2	Test Parameters	2606

A.6.5.2.4.3	Test Requirements	2608

A.6.5.3	SCell Activation and Deactivation Delay	2609

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2609

A.6.5.3.1.1	Test Purpose and Environment	2609

A.6.5.3.1.2	Test Requirements	2614

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2615

A.6.5.3.2.1	Test Purpose and Environment	2615

A.6.5.3.2.2	Test Requirements	2615

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2615

A.6.5.3.3.1	Test Purpose and Environment	2615

A.6.5.3.3.2	Test Requirements	2616

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	2616

A.6.5.3.4.1	Test Purpose and Environment	2616

A.6.5.3.4.2	Test Requirements	2624

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	2624

A.6.5.3.5.1	Test Purpose and Environment	2624

A.6.5.3.5.2	Test Requirements	2630

A.6.5.3.6	PUCCH SCell Activation and deactivation of known SCell in FR1	2631

A.6.5.3.6.1	Test Purpose and Environment	2631

A.6.5.3.6.2	Test Requirements	2635

A.6.5.3.7	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	2635

A.6.5.3.7.1	Test Purpose and Environment	2635

A.6.5.3.7.2	Test Requirements	2639

A.6.5.3.8	SCell Activation and Deactivation of one FR1 known PUCCH SCell and one FR1 unknown SCell with single activation/deactivation command	2640

A.6.5.3.8.1	Test Purpose and Environment	2640

A.6.5.3.8.2	Test Requirements	2644

A.6.5.3.9	SCell Activation and deactivation of unknown PUCCH SCell and unknown DL SCell in FR1 in non-DRX	2645

A.6.5.3.9.1	Test Purpose and Environment	2645

A.6.5.3.9.2	Test Requirements	2649

A.6.5.3.10	Fast SCell Activation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	2650

A.6.5.3.10.1	Test Purpose and Environment	2650

A.6.5.3.10.2	Test Requirements	2653

A.6.5.3.11	SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle	2653

A.6.5.3.11.1	Test Purpose and Environment	2653

A.6.5.3.11.2	Test Requirements	2654

A.6.5.3.12	SCell Activation and deactivation of unknown SCell in FR1 in DRX for UE capable of short measurement interval	2654

A.6.5.3.12.1	Test Purpose and Environment	2654

A.6.5.3.12.2	Test Requirements	2658

A.6.5.3.13	SCell Activation of multiple unknown SCells in FR1 with L3 reporting with single activation/deactivation commandin non-DRX	2658

A.6.5.3.13.1	Test Purpose and Environment	2658

A.6.5.3.13.2	Test Requirements	2663

A.6.5.3.14	SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160ms SCell measurement cycle	2664

A.6.5.3.14.1	Test Purpose and Environment	2664

A.6.5.3.14.2	Test Requirements	2669

A.6.5.3.15	TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX	2670

A.6.5.3.15.1	Test Purpose and Environment	2670

A.6.5.3.15.2	Test Requirements	2675

A.6.5.3.16	Inter-band SSB-less SCell Activation based on A-TRS	2676

A.6.5.3.16.1	Test Purpose and Environment	2676

A.6.5.3.16.2	Test Requirements	2681

A.6.5.4	UE UL carrier RRC reconfiguration Delay	2682

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	2682

A.6.5.4.1.1	Test Purpose and Environment	2682

A.6.5.4.1.2	Test Requirements	2689

A.6.5.4.2	Void	2690

A.6.5.5	Beam Failure Detection and Link recovery procedures	2690

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2690

A.6.5.5.1.1	Test Purpose and Environment	2690

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2696

A.6.5.5.2.1	Test Purpose and Environment	2696

A.6.5.5.2.2	Test Requirements	2702

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	2703

A.6.5.5.3.1	Test Purpose and Environment	2703

A.6.5.5.3.2	Test Requirements	2708

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2709

A.6.5.5.4.1	Test Purpose and Environment	2709

A.6.5.5.4.2	Test Requirements	2714

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	2715

A.6.5.5.5.1	Test Purpose and Environment	2715

A.6.5.5.5.2	Test Requirements	2719

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	2719

A.6.5.5.6.1	Test Purpose and Environment	2719

A.6.5.5.6.2	Test Requirements	2724

A.6.5.5.7	TRP Specific Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	2724

A.6.5.5.7.1	Test Purpose and Environment	2724

A.6.5.5.7.2	Test Requirements	2730

A.6.5.5.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for a UE operating on a cell with less than 5 MHz BW	2731

A.6.5.5.8.1	Test Purpose and Environment	2731

A.6.5.5.8.2	Test Requirements	2732

A.6.5.6	Active BWP switch	2732

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	2732

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	2732

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	2738

A.6.5.6.2	RRC-based Active BWP Switch	2743

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	2743

A.6.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	2747

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2747

A.6.5.6.4	SCell dormancy switch	2754

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	2754

A.6.5.6.4.1.1	Test Purpose and Environment	2754

A.6.5.6.4.1.2	Test Requirements	2760

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	2760

A.6.5.6.4.2.1	 Test Purpose and Environment	2760

A.6.5.6.4.2.2	Test Requirements	2767

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	2767

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	2767

A.6.5.7	DL interruptions at switching between two uplink carriers	2772

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2772

A.6.5.7.1.1	Test Purpose and Environment	2772

A.6.5.7.1.2	Test Requirements	2776

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2776

A.6.5.7.2.1	Test Purpose and Environment	2776

A.6.5.7.2.2	Test Requirements	2780

A.6.5.7A	DL interruptions at switching between two uplink carriers with two transmit antenna connectors	2780

A.6.5.7A.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	2780

A.6.5.7A.1.1	Test Purpose and Environment	2780

A.6.5.7A.1.2	Test Requirements	2784

A.6.5.7A.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	2784

A.6.5.7A.2.1	Test Purpose and Environment	2784

A.6.5.7A.2.2	Test Requirements	2788

A.6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	2788

A.6.5.7B.1	DL interruptions at switching between two uplink bands in FDD-TDD CA	2788

A.6.5.7B.1.1	Test Purpose and Environment	2788

A.6.5.7B.1.2	Test Requirements	2792

A.6.5.7B.2	DL interruptions at switching between two uplink bands in TDD-TDD CA	2792

A.6.5.7B.2.1	Test Purpose and Environment	2792

A.6.5.7B.2.2	Test Requirements	2796

A.6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	2796

A.6.5.7C.1	DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	2796

A.6.5.7C.1.1	Test Purpose and Environment	2796

A.6.5.7C.1.2	Test Requirements	2800

A.6.5.7C.2	DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	2801

A.6.5.7C.2.1	Test Purpose and Environment	2801

A.6.5.7C.2.2	Test Requirements	2805

A.6.5.7D	DL interruptions at UE switching across three or four uplink bands	2806

A.6.5.7D.1	DL interruptions at switching across three uplink bands in TDD-TDD CA for single TAG	2806

A.6.5.7D.1.1	Test Purpose and Environment	2806

A.6.5.7D.1.2	Test Requirements	2810

A.6.5.7D.2	DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG	2811

A.6.5.7D.2.1	Test Purpose and Environment	2811

A.6.5.7D.2.2	Test Requirements	2815

A.6.5.7D.3	DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs	2815

A.6.5.7D.3.1	Test Purpose and Environment	2815

A.6.5.7D.3.2	Test Requirements	2819

A.6.5.7D.4	DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs	2820

A.6.5.7D.4.1	Test Purpose and Environment	2820

A.6.5.7D.7.2	Test Requirements	2825

A.6.5.8	UE specific CBW change	2825

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	2825

A.6.5.8.1.1	Test Purpose and Environment	2825

A.6.5.8.1.2	Test Requirements	2829

A.6.5.9	Pathloss reference signal switching delay	2829

A.6.5.9.1	MAC-CE based pathloss reference signal switch delay	2829

A.6.5.9.1.1	Test Purpose and Environment	2829

A.6.5.9.1.2	Test Requirements	2832

A.6.5.10	Conditional PSCell addition and release delay (FR1 NR-DC)	2833

A.6.5.10.1	Conditional PSCell Addition and Release Delay	2833

A.6.5.10.1.1	Test purpose and environment	2833

A.6.5.10.1.2	Test Parameters	2833

A.6.5.10.1.3	Test Requirements	2836

A.6.5.11	PSCell addition and release delay	2836

A.6.5.11.1	Addition and Release Delay of unknown NR FR1 PSCell	2836

A.6.5.11.1.1	Test purpose and environment	2836

A.6.5.11.1.2	Test Requirements	2838

A.6.5.12	Subsequent conditional PSCell addition/change	2839

A.6.5.12.1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2839

A.6.5.12.1.1	Test purpose and environment	2839

A.6.5.12.1.2	Test Parameters	2839

A.6.5.12.1.3	Test Requirements	2842

A.6.5.12.2	Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC	2842

A.6.5.12.2.1	Test purpose and environment	2842

A.6.5.12.2.2	Test Parameters	2842

A.6.5.12.2.3	Test Requirements	2847

A.6.5.13	Active TCI state switch delay	2847

A.6.5.13.1	MAC-CE based joint TCI state switch for mDCI with two TA when RTD is larger than CP	2847

A.6.5.13.1.1	Test Purpose and Environment	2847

A.6.5.13.1.2	Test Requirements	2850

A.6.6	Measurement procedure	2683

A.6.6.1	Intra-frequency Measurements	2683

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	2683

A.6.6.1.1.1	Test purpose and Environment	2683

A.6.6.1.1.2	Test parameters	2683

A.6.6.1.1.3	Test Requirements	2686

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	2686

A.6.6.1.2.1	Test purpose and Environment	2686

A.6.6.1.2.2	Test parameters	2686

A.6.6.1.2.3	Test Requirements	2690

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	2690

A.6.6.1.3.1	Test purpose and Environment	2690

A.6.6.1.3.2	Test parameters	2690

A.6.6.1.3.3	Test Requirements	2694

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	2694

A.6.6.1.4.1	Test purpose and Environment	2694

A.6.6.1.4.2	Test parameters	2694

A.6.6.1.4.3	Test Requirements	2698

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	2698

A.6.6.1.5.1	Test purpose and Environment	2698

A.6.6.1.5.2	Test parameters	2698

A.6.6.1.5.3	Test Requirements	2700

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	2701

A.6.6.1.6.1	Test purpose and Environment	2701

A.6.6.1.6.2	Test parameters	2701

A.6.6.1.6.3	Test Requirements	2702

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	2703

A.6.6.1.7.1	Test purpose and Environment	2703

A.6.6.1.7.2	Test parameters	2703

A.6.6.1.7.3	Test Requirements	2707

A.6.6.1.8	SA event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasCA-Scell-r17	2707

A.6.6.1.8.1	Test purpose and Environment	2707

A.6.6.1.8.2	Test parameters	2707

A.6.6.1.8.3	Test Requirements	2712

A.6.6.1.9	SA event triggered reporting tests with MUSIM gap configured	2712

A.6.6.1.9.1	Test purpose and Environment	2712

A.6.6.1.9.2	Test parameters	2712

A.6.6.1.9.3	Test requirements	2716

A.6.6.1.10	SA event triggered reporting tests without gap under non-DRX when CD-SSB is outside active BWP	2716

A.6.6.1.10.1	Test purpose and Environment	2716

A.6.6.1.10.2	Test Requirements	2716

A.6.6.1.11	SA event triggered reporting tests without gap under non-DRX with NCD-SSB	2716

A.6.6.1.11.1	Test purpose and Environment	2716

A.6.6.1.11.2	Test parameters	2717

A.6.6.1.11.3	Test Requirements	2719

A.6.6.1.12	SA event triggered reporting tests without gap under non-DRX with SSB index reading and 12 PRB SSB	2719

A.6.6.1.12.1	Test purpose and Environment	2719

A.6.6.1.12.2	Test parameters	2719

A.6.6.1.12.3	Test Requirements	2720

A.6.6.1.13	SA event triggered reporting tests without gap under Cell DTX	2720

A.6.6.1.13.1	Test purpose and Environment	2720

A.6.6.1.13.2	Test parameters	2720

A.6.6.1.13.3	Test Requirements	2724

A.6.6.2	Inter-frequency Measurements	2724

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2724

A.6.6.2.1.1	Test Purpose and Environment	2724

A.6.6.2.1.2	Test Requirements	2728

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2728

A.6.6.2.2.1	Test Purpose and Environment	2728

A.6.6.2.2.2	Test Requirements	2732

A.6.6.2.3	Void	2733

A.6.6.2.4	Void	2733

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2733

A.6.6.2.5.1	Test Purpose and Environment	2733

A.6.6.2.5.2	Test Requirements	2737

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2737

A.6.6.2.6.1	Test Purpose and Environment	2737

A.6.6.2.6.2	Test Requirements	2741

A.6.6.2.7	Void	2741

A.6.6.2.8	Void	2741

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	2741

A.6.6.2.9.1	Test Purpose and Environment	2741

A.6.6.2.9.2	Test Requirements	2745

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	2745

A.6.6.2.10.1	Test Purpose and Environment	2745

A.6.6.2.10.2	Test Requirements	2749

A.6.6.2.12	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for UE configured with highSpeedMeasInterFreq-r17	2753

A.6.6.2.12.1	Test Purpose and Environment	2753

A.6.6.2.12.2	Test Requirements	2757

A.6.6.2.13	SA event triggered reporting tests for FR1 with MUSIM gap configured	2757

A.6.6.2.13.1	Test Purpose and Environment	2757

A.6.6.2.13.2	Test Requirements	2761

A.6.6.2.14	SA event triggered reporting tests for FR1 measurement gap and periodic MUSIM gap with partially partial overlapping scenario for SSB-based measurements in inter-frequency layers	2762

A.6.6.2.14.1	Test Purpose and Environment	2762

A.6.6.2.14.2	Test Requirements	2766

A.6.6.2.15	SA event triggered reporting tests for FR1 with 3MHz Channel Bandwidth configured without SSB time index detection when DRX is used	2766

A.6.6.2.15.1	Test Purpose and Environment	2766

A.6.6.2.15.2	Test Requirements	2767

A.6.6.3	Inter-RAT Measurements	2767

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	2767

A.6.6.3.1.1	Test Purpose and Environment	2767

A.6.6.3.1.2	Test Requirements	2773

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	2773

A.6.6.3.2.1	Test Purpose and Environment	2773

A.6.6.3.2.2	Test Requirements	2780

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	2780

A.6.6.3.3.1	Test Purpose and Environment	2780

A.6.6.3.3.2	Test Requirements	2785

A.6.6.4	L1-RSRP measurement for beam reporting	2785

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	2785

A.6.6.4.1.1	Test Purpose and Environment	2785

A.6.6.4.1.2	Test parameters	2786

A.6.6.4.1.3	Test Requirements	2789

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	2789

A.6.6.4.2.1	Test Purpose and Environment	2789

A.6.6.4.2.2	Test parameters	2790

A.6.6.4.2.3	Test Requirements	2793

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	2793

A.6.6.4.3.1	Test Purpose and Environment	2793

A.6.6.4.3.2	Test parameters	2794

A.6.6.4.3.3	Test Requirements	2797

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	2797

A.6.6.4.4.1	Test Purpose and Environment	2797

A.6.6.4.4.2	Test parameters	2798

A.6.6.4.4.3	Test Requirements	2801

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	2801

A.6.6.4.5.1	Test Purpose and Environment	2801

A.6.6.4.5.2	Test parameters	2802

A.6.6.4.5.3	Test Requirements	2805

A.6.6.4.6	Inter-cell SSB based L1-RSRP measurements on FR1 PCell when DRX is used	2805

A.6.6.4.6.1	Test Purpose and Environment	2805

A.6.6.4.6.2	Test parameters	2806

A.6.6.4.6.3	Test Requirements	2809

A.6.6.4.7	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2810

A.6.6.4.7.1	Test Purpose and Environment	2810

A.6.6.4.7.2	Test Requirements	2810

A.6.6.4.8	CSI-RS based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	2810

A.6.6.4.8.1	Test Purpose and Environment	2810

A.6.6.4.9	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	2810

A.6.6.4.9.1	Test Purpose and Environment	2810

A.6.6.4.9.2	Test parameters	2811

A.6.6.4.9.3	Test Requirements	2814

A.6.6.5	Inter-RAT UTRAN FDD measurements	2814

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	2814

A.6.6.5.1.1	Test Purpose and Environment	2814

A.6.6.5.1.2	Test Requirements	2818

A.6.6.6	CLI measurements	2818

A.6.6.6.1	SRS-RSRP measurement with DRX	2818

A.6.6.6.1.1	Test Purpose and Environment	2818

A.6.6.6.1.2	Test Parameters	2819

A.6.6.6.1.3	Test Requirements	2822

A.6.6.6.2	CLI-RSSI measurement with DRX	2822

A.6.6.6.2.1	Test Purpose and Environment	2822

A.6.6.6.2.2	Test Parameters	2823

A.6.6.6.2.3	Test Requirements	2825

A.6.6.7	NR measurements with autonomous gaps	2825

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	2825

A.6.6.7.1.1	Test Purpose and Environment	2825

A.6.6.7.1.2	Test Parameters	2825

A.6.6.7.1.3	Test Requirements	2828

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	2828

A.6.6.7.2.1	Test Purpose and Environment	2828

A.6.6.7.2.2	Test Requirements	2831

A.6.6.8	L1-SINR measurement for beam reporting	2832

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	2832

A.6.6.8.1.1	Test Purpose and Environment	2832

A.6.6.8.1.2	Test parameters	2832

A.6.6.8.1.3	Test Requirements	2834

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	2834

A.6.6.8.2.1	Test Purpose and Environment	2834

A.6.6.8.2.2	Test parameters	2834

A.6.6.8.2.3	Test Requirements	2839

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	2839

A.6.6.8.3.1	Test Purpose and Environment	2839

A.6.6.8.3.2	Test parameters	2840

A.6.6.8.3.3	Test Requirements	2842

A.6.6.9	Idle Mode CA/DC Measurements	2842

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	2842

A.6.6.9.1.1	Test Purpose and Environment	2842

A.6.6.9.1.2	Test Requirements	2849

A.6.6.9.2	Test case for Idle mode fast CA/DC eEMR measurement for FR1	2849

A.6.6.9.2.1	Test Purpose and Environment	2849

A.6.6.9.2.2	Test Requirements	2853

A.6.6.9.3	Test case for Idle mode fast CA/DC cell reselection measurement for FR1 without valid reporting	2853

A.6.6.9.3.1	Test Purpose and Environment	2853

A.6.6.9.3.2	Test Requirements	2858

A.6.6.9.4	Test case for Idle mode fast CA/DC cell reselection measurement for FR1 with valid reporting	2859

A.6.6.9.4.1	Test Purpose and Environment	2859

A.6.6.9.4.2	Test Requirements	2863

A.6.6.10	CSI-RS based intra-frequency Measurements	2864

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	2864

A.6.6.10.1.1	Test purpose and Environment	2864

A.6.6.10.1.2	Test Requirements	2866

A.6.6.11	CSI-RS based inter-frequency Measurements	2866

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	2866

A.6.6.11.1.1	Test Purpose and Environment	2866

A.6.6.11.1.2	Test Requirements	2870

A.6.6.12	RSTD measurements	2870

A.6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	2870

A.6.6.12.1.1	Test Purpose and Environment	2870

A.6.6.12.1.2	Test Requirements	2878

A.6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	2878

A.6.6.12.2.1	Test Purpose and Environment	2878

A.6.6.12.2.2	Test Requirements	2886

A.6.6.12.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR1 SA	2886

A.6.6.12.3.1	Test Purpose and Environment	2886

A.6.6.12.3.2	Test Requirements	2892

A.6.6.12.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA without measurement gap	2893

A.6.6.12.4.1	Test Purpose and Environment	2893

A.6.6.12.4.2	Test Requirements	2897

A.6.6.12.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_CONNECTED state with Rx TEG	2897

A.6.6.12.5.1	Test Purpose and Environment	2897

A.6.6.12.5.2	Test Requirements	2901

A.6.6.13 PRS-RSRP measurements	2902

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	2902

A.6.6.13.1.1	Test purpose and Environment	2902

A.6.6.13.1.2	Test Requirements	2906

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	2906

A.6.6.13.2.1	Test purpose and Environment	2906

A.6.6.13.2.2	Test Requirements	2910

A.6.6.13.3	PRS-RSRP reporting delay test case for reduced number of samples	2910

A.6.6.13.3.1	Test purpose and Environment	2910

A.6.6.13.3.2	Test Requirements	2914

A.6.6.13.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	2914

A.6.6.13.4.1	Test purpose and Environment	2914

A.6.6.14	UE Rx-Tx time difference measurements	2918

A.6.6.14.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	2918

A.6.6.14.1.1	Test purpose and environment	2918

A.6.6.14.1.2	Test requirements	2922

A.6.6.14.2	UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	2922

A.6.6.14.2.1	Test purpose and environment	2922

A.6.6.14.2.2	Test requirements	2926

A.6.6.14.3	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with reduced sample number	2926

A.6.6.14.3.1	Test purpose and environment	2926

A.6.6.14.3.2	Test requirements	2930

A.6.6.14.4	UE Rx-Tx time difference measurement without gaps in FR1 SA	2930

A.6.6.14.4.1	Test purpose and environment	2930

A.6.6.14.4.2	Test requirements	2934

A.6.6.14.5	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA with multiple RxTx TEGs	2934

A.6.6.14.4.1	Test purpose and environment	2934

A.6.6.14.4.2	Test requirements	2938

A.6.6.15	Idle Mode measurements of inter-RAT DC candidate cells for early reporting	2938

A.6.6.15.1	Test Purpose and Environment	2938

A.6.6.15.2	Test Requirements	2945

A.6.6.16	PRS-RSRPP measurements	2946

A.6.6.16.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state	2946

A.6.6.16.1.1	Test purpose and Environment	2946

A.6.6.16.1.2	Test Requirements	2948

A.6.6.16.2	PRS-RSRPP reporting delay test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	2948

A.6.6.16.2.1	Test purpose and Environment	2948

A.6.6.16.2.2	Test Requirements	2951

A.6.6.16.3	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_CONNECTED state without measurement gap	2951

A.6.6.16.3.1	Test purpose and Environment	2951

A.6.6.16.3.2	Test Requirements	2955

A.6.6.17	SA event triggered reporting tests with Pre-MG	2955

A.6.6.17.1	SA event triggered reporting tests with autonomous activation/deactivation Pre-MG	2955

A.6.6.17.1.1	Test purpose and Environment	2955

A.6.6.17.1.2	Test parameters	2955

A.6.6.17.1.3	Test Requirements	2960

A.6.6.17.2	SA event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2960

A.6.6.17.2.1	Test purpose and Environment	2960

A.6.6.17.2.2	Test parameters	2960

A.6.6.17.2.3	Test Requirements	2965

A.6.6.17.3	Void	2965

A.6.6.17.3.1	Void	2965

A.6.6.17.3.2	Void	2965

A.6.6.17.3.3	Void	2965

A.6.6.18	SA event triggered reporting tests with concurrent gaps	2965

A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers	2965

A.6.6.18.1.1	Test Purpose and Environment	2965

A.6.6.18.1.2	Test Requirements	2969

A.6.6.18.2	SA event triggered reporting tests for FR1 concurrent gap with partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	2969

A.6.6.18.2.1	Test Purpose and Environment	2969

A.6.6.18.2.2	Test Requirements	2973

A.6.6.18.3	SA NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2973

A.6.6.18.3.1	Test Purpose and Environment	2973

A.6.6.18.3.2	Test Requirements	2980

A.6.6.18.4	SA event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2980

A.6.6.18.4.1	Test Purpose and Environment	2980

A.6.6.18.4.2	Test Requirements	2984

A.6.6.19	SA event triggered reporting tests with NCSG	2985

A.6.6.19.1	SA event triggered reporting tests with NCSG under non-DRX in FR1	2985

A.6.6.19.1.1	Test purpose and Environment	2985

A.6.6.19.1.2	Test parameters	2985

A.6.6.19.1.3	Test Requirements	2989

A.6.6.19.2	SA event triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2989

A.6.6.19.2.1	Test Purpose and Environment	2989

A.6.6.19.2.2	Test parameters	2989

A.6.6.19.2.3	Test Requirements	2993

A.6.6.19.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2993

A.6.6.19.3.1	Test Purpose and Environment	2993

A.6.6.19.3.2	Test parameters	2994

A.6.6.19.3.3	Test Requirements	2999

A.6.6.19.4	Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2999

A.6.6.19.4.1	Test purpose and Environment	2999

A.6.6.19.4.2	Test parameters	2999

A.6.6.19.4.3	Test Requirements	3002

A.6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	3002

A.6.6.20.1	Test purpose and environment	3002

A.6.6.20.2	Test requirements	3006

A.6.6.21	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	3006

A.6.6.21.1	Test purpose and environment	3006

A.6.6.21.2	Test requirements	3008

A.6.6.22	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG	3009

A.6.6.22.1	SA event triggered reporting tests for FR1 concurrent gap with Pre-MG with partially partial overalpping scenario for SSB-based measurements in both intra-frequency and inter-frequency layers	3009

A.6.6.22.1.1	Test Purpose and Environment	3009

A.6.6.22.1.2	Test Requirements	3012

A.6.6.22.2	SA event triggered reporting tests for concurrent gap with pre-configured gaps and network-controlled activation/deactivation	3013

A.6.6.22.2.1	Test purpose and Environment	3013

A.6.6.22.2.2	Test parameters	3013

A.6.6.22.2.3	Test Requirements	3017

A.6.6.23	SA event triggered reporting tests for concurrent measurement gaps with NCSG	3018

A.6.6.23.1	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	3018

A.6.6.23.1.1	Test Purpose and Environment	3018

A.6.6.23.1.2	Test Requirements	3021

A.6.6.23.2	SA event triggered reporting tests for FR1 concurrent gaps with NCSG for partially partial overalpping scenario for SSB-based measurements in both inter-frequency layers	3022

A.6.6.23.2.1	Test Purpose and Environment	3022

A.6.6.23.2.2	Test Requirements	3025

A.6.6.23.3	Event triggered reporting on SCC with deactivated SCell test with per-UE Con-NCSG under non-DRX	3026

A.6.6.23.3.1	Test purpose and Environment	3026

A.6.6.23.3.2	Test parameters	3026

A.6.6.23.3.3	Test Requirements	3029

A.6.6.24	SA event triggered reporting tests with NeedForGap in FR1	3029

A.6.6.24.1	SA event triggered reporting tests without gaps, with interruptions, under non-DRX	3029

A.6.6.24.1.1	Test purpose and Environment	3029

A.6.6.24.1.2	Test parameters	3029

A.6.6.24.1.3	Test Requirements	3032

A.6.6.24.2	SA event triggered reporting tests for FR1 without gap with interruption for inter-frequency measurement with SSB time index detection when DRX is not used	3033

A.6.6.24.2.1	Test Purpose and Environment	3033

A.6.6.24.2.2	Test parameters	3033

A.6.6.24.2.3	Test Requirements	3037

A.6.6.24.3	SA event triggered reporting tests for FR1 with ‘no-gap-with-interruption’, without measurement gap or DRX	3037

A.6.6.24.3.1	Test Purpose and Environment	3037

A.6.6.24.3.2	Test Requirements	3041

A.6.6.24.4	SA event triggered reporting tests for FR1 NeedForGaps without gap without interruption when DRX is not used	3041

A.6.6.24.4.1	Test Purpose and Environment	3041

A.6.6.24.4.2	Test parameters	3042

A.6.6.24.4.2	Test Requirements	3045

A.6.6.24.5	SA event triggered reporting tests without gap under non-DRX for UE indicating no-gap-no-interruption	3045

A.6.6.24.5.1	Test purpose and Environment	3045

A.6.6.24.5.2	Test parameters	3045

A.6.6.24.5.3	Test Requirements	3049

A.6.6.25	SA NR - E-UTRAN event-triggered without measurement gaps	3049

A.6.6.25.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	3049

A.6.6.25.1.1	Test Purpose and Environment	3049

A.6.6.25.1.2	Test Requirements	3054

A.6.6.25.2	SA NR - E-UTRAN event-triggered reporting without gap under non-DRX in FR1	3054

A.6.6.25.2.1	Test Purpose and Environment	3054

A.6.6.25.2.2	Test parameters	3054

A.6.6.25.2.3	Test Requirements	3055

A.6.6.25.3	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for UE capable of inter-RAT EUTRAN measurement without gap when CRS is contained within UE’s active DL BWP	3055

A.6.6.25.3.1	Test Purpose and Environment	3055

A.6.6.25.3.2	Test Requirements	3060

A.6.6.26	LTM Intra-frequency L1-RSRP measurement	3060

A.6.6.26.1	Intra-frequency SSB based L1-RSRP measurement in FR1	3060

A.6.6.26.1.1	Test Purpose and Environment	3060

A.6.6.26.1.2	Test Parameters	3060

A.6.6.26.1.3 Test Requirements	3063

A.6.6.27	LTM Inter-frequency L1-RSRP measurement with measurement gap	3063

A.6.6.27.1	Inter-frequency SSB based L1-RSRP measurement with measurement gap	3063

A.6.6.27.1.1	Test Purpose and Environment	3063

A.6.6.27.1.2	Test parameters	3063

A.6.6.27.1.3	Test Requirements	3067

A.6.6.28	LTM Inter-frequency L1-RSRP measurement without measurement gap	3067

A.6.6.28.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap	3067

A.6.6.28.1.1	Test Purpose and Environment	3067

A.6.6.28.1.2	Test parameters	3067

A.6.6.28.1.3	Test Requirements	3070

A.6.7	Measurement Performance requirements	3071

A.6.7.1	SS-RSRP	3071

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3071

A.6.7.1.1.1	Test Purpose and Environment	3071

A.6.7.1.1.2	Test parameters	3071

A.6.7.1.1.3	Test Requirements	3076

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3076

A.6.7.1.2.1	Test Purpose and Environment	3076

A.6.7.1.2.2	Test parameters	3076

A.6.7.1.2.3	Test Requirements	3080

A.6.7.1.3	Void	3081

A.6.7.2	SS-RSRQ	3081

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3081

A.6.7.2.1.1	Test Purpose and Environment	3081

A.6.7.2.1.2	Test Parameters	3081

A.6.7.2.1.3	Test Requirements	3086

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3086

A.6.7.2.2.1	Test Purpose and Environment	3086

A.6.7.2.2.2	Test Parameters	3086

A.6.7.2.2.3	Test Requirements	3091

A.6.7.3	SS-SINR	3091

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3091

A.6.7.3.1.1	Test Purpose and Environment	3091

A.6.7.3.1.2	Test Parameters	3092

A.6.7.3.1.3	Test Requirements	3096

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3097

A.6.7.3.2.1	Test Purpose and Environment	3097

A.6.7.3.2.2	Test Parameters	3097

A.6.7.3.2.3	Test Requirements	3102

A.6.7.4	L1-RSRP measurement for beam reporting	3102

A.6.7.4.1	SSB based L1-RSRP measurement	3102

A.6.7.4.1.1	Test Purpose and Environment	3102

A.6.7.4.1.2	Test parameters	3102

A.6.7.4.1.3	Test Requirements	3106

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3106

A.6.7.4.2.1	Test Purpose and Environment	3106

A.6.7.4.2.2	Test parameters	3107

A.6.7.4.2.3	Test Requirements	3111

A.6.7.5	E-UTRAN RSRP	3112

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3112

A.6.7.5.1.1	Test Purpose and Environment	3112

A.6.7.5.1.2	Test parameters	3112

A.6.7.5.1.3	Test Requirements	3118

A.6.7.6	E-UTRAN RSRQ	3119

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3119

A.6.7.6.1.1	Test Purpose and Environment	3119

A.6.7.6.1.2	Test parameters	3119

A.6.7.6.1.3	Test Requirements	3124

A.6.7.7	E-UTRAN RS-SINR	3125

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	3125

A.6.7.7.1.1	Test Purpose and Environment	3125

A.6.7.7.1.2	Test parameters	3125

A.6.7.7.1.3	Test Requirements	3131

A.6.7.8	CLI measurements	3132

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	3132

A.6.7.8.1.1	Test Purpose and Environment	3132

A.6.7.8.1.2	Test parameters	3132

A.6.7.8.1.3	Test Requirements	3138

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	3138

A.6.7.8.2.1	Test Purpose and Environment	3138

A.6.7.8.2.2	Test parameters	3139

A.6.7.8.2.3	Test Requirements	3142

A.6.7.9	L1-SINR measurement for beam reporting	3143

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3148

A.6.7.9.2.1	Test Purpose and Environment	3148

A.6.7.9.2.2	Test parameters	3148

A.6.7.9.2.3	Test Requirements	3153

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3153

A.6.7.9.3.1	Test Purpose and Environment	3153

A.6.7.9.3.2	Test parameters	3154

A.6.7.9.3.3	Test Requirements	3158

A.6.7.10	CSI-RSRP	3159

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3159

A.6.7.9.10.1	Test Purpose and Environment	3159

A.6.7.9.10.2	Test parameters	3159

A.6.7.10.1.3	Test Requirements	3164

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	3164

A.6.7.9.10.1	Test Purpose and Environment	3164

A.6.7.10.2.2	Test parameters	3165

A.6.7.10.2.3	Test Requirements	3170

A.6.7.11	CSI-RSRQ	3170

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3170

A.6.7.11.1.1	Test Purpose and Environment	3170

A.6.7.11.1.2	Test Parameters	3170

A.6.7.11.1.3	Test Requirements	3175

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3175

A.6.7.11.2.1	Test Purpose and Environment	3175

A.6.7.11.2.2	Test Parameters	3175

A.6.7.11.2.3	Test Requirements	3181

A.6.7.12	CSI-SINR	3182

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3182

A.6.7.12.1.1	Test Purpose and Environment	3182

A.6.7.12.1.2	Test Parameters	3182

A.6.7.12.1.3	Test Requirements	3187

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	3187

A.6.7.12.2.1	Test Purpose and Environment	3187

A.6.7.12.2.2	Test Parameters	3187

A.6.7.12.2.3	Test Requirements	3193

A.6.7.13	RSTD measurements	3193

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	3193

A.6.7.13.1.1	Test purpose and Environment	3193

A.6.7.13.1.2	Test Requirements	3196

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	3196

A.6.7.13.2.1	Test purpose and Environment	3196

A.6.7.13.2.2	Test Requirements	3200

A.6.7.13.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_CONNECTED state	3200

A.6.7.13.3.1	Test purpose and Environment	3200

A.6.7.13.3.2	Test Requirements	3202

A.6.7.13.4	RSTD measurement accuracy test case with Rx TEG	3202

A.6.7.14	PRS-RSRP measurements	3206

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	3206

A.6.7.14.1.1	Test Purpose and Environment	3206

A.6.7.14.1.2	Test parameters	3207

A.6.7.14.1.3	Test Requirements	3211

A.6.7.14.2	SA: measurement accuracy with PRS in FR1 with reduced sample number	3211

A.6.7.14.2.1	Test Purpose and Environment	3211

A.6.7.14.2.2	Test parameters	3211

A.6.7.14.2.3	Test Requirements	3213

A.6.7.14.3	Void	3213

A.6.7.14.3.1	Void	3213

A.6.7.14.3.2	Void	3213

A.6.7.14.3.3	Void	3213

A.6.7.15	UE Rx-Tx time difference measurements	3213

A.6.7.15.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	3213

A.6.7.15.1.1	Test purpose and environment	3213

A.6.7.15.1.2	Test parameters	3214

A.6.7.15.1.3	Test requirements	3217

A.6.7.15.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR1 SA	3218

A.6.7.15.2.1	Test purpose and environment	3218

A.6.7.15.2.2	Test parameters	3218

A.6.7.15.2.3	Test requirements	3221

A.6.7.15.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	3221

A.6.7.15.3.1	Test purpose and environment	3221

A.6.7.15.3.2	Test parameters	3222

A.6.7.15.3.3	Test requirements	3225

A.6.7.16	PRS-RSRPP measurements	3225

A.6.7.16.1	SA: measurement accuracy with PRS in FR1	3225

A.6.7.16.1.1	Test Purpose and Environment	3225

A.6.7.16.1.2	Test parameters	3226

A.6.7.16.1.3	Test Requirements	3229

A.6.7.16.2	SA: measurement accuracy with reduced PRS samples in FR1	3230

A.6.7.16.2.1	Test Purpose and Environment	3230

A.6.7.16.2.2	Test parameters	3230

A.6.7.17	LTM L1-RSRP measurement	3233

A.6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1	3233

A.6.7.17.1.1	Test Purpose and Environment	3233

A.6.7.17.1.2	Test parameters	3234

A.6.7.17.1.3	Test Requirements	3239

A.6.7.18	TDCP amplitude measurement accuracy	3239

A.6.7.18.1	TDCP amplitude measurement accuracy in FR1	3239

A.6.7.18.1.1	Test Purpose and Environment	3239

A.6.7.18.1.2	Test parameters	3239

A.6.7.18.1.3	Test Requirements	3242

A.6.8	Measurement procedure in RRC_INACTIVE	3242

A.6.8.1	RSTD measurements	3242

A.6.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA in RRC_INACTIVE state	3242

A.6.8.1.1.1	Test Purpose and Environment	3242

A.6.8.1.1.2	Test Requirements	3246

A.6.8.1.2	 NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	3246

A.6.8.1.2.1	Test Purpose and Environment	3246

A.6.8.1.2.1	Test Purpose and Environment	3246

A.6.8.1.2.2	Test Requirements	3251

A.6.8.2	PRS-RSRP measurements	3251

A.6.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	3251

A.6.8.2.1.1	Test purpose and Environment	3251

A.6.8.2.1.2	Test Requirements	3253

A.6.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	3254

A.6.8.2.2.1	Test purpose and Environment	3254

A.6.8.2.2.2	Test Requirements	3257

A.6.8.3	UE Rx-Tx time difference measurements	3257

A.6.8.3.1	UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	3257

A.6.8.3.1.1	Test purpose and environment	3257

A.6.8.3.1.2	Test requirements	3261

A.6.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR1 SA	3261

A.6.8.3.2.1	Test purpose and environment	3261

A.6.8.3.2.2	Test requirements	3264

A.6.8.4	PRS-RSRPP measurements	3264

A.6.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3264

A.6.8.4.1.1	Test purpose and Environment	3264

A.6.8.4.1.2	Test Requirements	3267

A.6.8.4.2	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR1 in RRC_INACTIVE state for reduced number of samples	3267

A.6.8.4.2.1	Test purpose and Environment	3267

A.6.8.4.2.2	Test Requirements	3269

A.6.9	Measurement performance requirements in RRC_INACTIVE	3269

A.6.9.1	RSTD measurements	3269

A.6.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR1 in RRC_INACTIVE state	3269

A.6.9.1.1.1	Test purpose and Environment	3269

A.6.9.1.1.2	Test Requirements	3271

A.6.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR1 in RRC_INACTIVE state	3272

A.6.9.1.2.1	Test purpose and Environment	3272

A.6.9.1.2.2	Test Requirements	3274

A.6.9.2	PRS-RSRP measurements	3274

A.6.9.2.1	SA: measurement accuracy with PRS in FR1 in RRC_INACTIVE	3274

A.6.9.2.1.1	Test Purpose and Environment	3274

A.6.9.2.1.2	Test parameters	3274

A.6.9.2.1.3	Test Requirements	3276

A.6.9.2.2	SA: measurement accuracy with PRS in FR1 with reduced number of samples in RRC_INACTIVE state	3277

A.6.9.2.2.1	Test Purpose and Environment	3277

A.6.9.2.2.2	Test parameters	3277

A.6.9.2.2.3	Test Requirements	3280

A.6.9.3	UE Rx-Tx time difference measurements	3280

A.6.9.3.1.1	UE Rx-Tx time difference measurement accuracy in FR1 SA	3280

A.6.9.3.1.1.1	Test purpose and environment	3280

A.6.9.3.1.1.2 Test parameters	3280

A.6.9.3.1.1.3	Test requirements	3283

A.6.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples	3283

A.6.9.3.2.1	Test purpose and environment	3283

A.6.9.3.2.2	Test parameters	3283

A.6.9.3.2.3	Test requirements	3286

A.6.9.4	PRS-RSRPP measurements	3286

A.6.9.4.1	SA: PRS-RSRPP measurement accuracy in FR1 in RRC INACTIVE	3286

A.6.9.4.1.1	Test Purpose and Environment	3286

A.6.9.4.1.2	Test parameters	3286

A.6.9.4.1.3	Test Requirements	3289

A.6.9.4.2	SA: measurement accuracy with reduced PRS samples in FR1 in RRC INACTIVE	3290

A.6.9.4.2.1	Test Purpose and Environment	3290

A.6.9.4.2.2	Test parameters	3290

A.6.9.4.2.3	Test Requirements	3292

A.7	NR standalone tests with one or more NR cells in FR2	3185

A.7.1	SA: RRC_IDLE state mobility	3185

A.7.1.1	Cell re-selection to NR	3185

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	3185

A.7.1.1.1.1	Test Purpose and Environment	3185

A.7.1.1.1.2	Test Parameters	3185

A.7.1.1.1.3	Test Requirements	3188

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	3188

A.7.1.1.2.1	Test Purpose and Environment	3188

A.7.1.1.2.2	Test Parameters	3189

A.7.1.1.2.3	Test Requirements	3191

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3191

A.7.1.1.3.1	Test Purpose and Environment	3191

A.7.1.1.3.2	Test Parameters	3191

A.7.1.1.3.3	Test Requirements	3194

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3194

A.7.1.1.4.1	Test Purpose and Environment	3194

A.7.1.1.4.2	Test Parameters	3194

A.7.1.1.4.3	Test Requirements	3197

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	3197

A.7.1.1.5.1	Test Purpose and Environment	3197

A.7.1.1.5.2	Test Parameters	3197

A.7.1.1.5.3	Test Requirements	3200

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	3200

A.7.1.1.6.1	Test Purpose and Environment	3200

A.7.1.1.6.2	Test Parameters	3200

A.7.1.1.6.3	Test Requirements	3203

A.7.1.1.7	Cell reselection to FR2 intra-frequency NR case for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3203

A.7.1.1.7.1	Test Purpose and Environment	3203

A.7.1.1.7.2	Test Parameters	3203

A.7.1.1.7.3	Test Requirements	3207

A.7.1.1.8	Cell reselection to FR2 inter-frequency NR case for UE configured with highSpeedMeasFlagFR2-r17	3207

A.7.1.1.8.1	Test Purpose and Environment	3207

A.7.1.1.8.2	Test Parameters	3207

A.7.1.1.8.3	Test Requirements	3211

A.7.2	SA: RRC_INACTIVE state mobility	3211

A.7.2.1	Small Data Transmission	3211

A.7.2.1.1	TA validation for CG-SDT in FR2	3211

A.7.2.1.1.1	Test Purpose and Environment	3211

A.7.2.1.1.2	Test Requirements	3215

A.7.3	RRC_CONNECTED state mobility	3215

A.7.3.1	Handover	3215

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	3215

A.7.3.1.1.1	Test Purpose and Environment	3215

A.7.3.1.1.2	Test Parameters	3215

A.7.3.1.1.3	Test Requirements	3219

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	3219

A.7.3.1.2.1	Test Purpose and Environment	3219

A.7.3.1.2.2	Test Parameters	3219

A.7.3.1.2.3	Test Requirements	3222

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	3222

A.7.3.1.3.1	Test Purpose and Environment	3222

A.7.3.1.3.2	Test Parameters	3222

A.7.3.1.3.3	Test Requirements	3223

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	3224

A.7.3.1.4.1	Test Purpose and Environment	3224

A.7.3.1.4.2	Test Parameters	3224

A.7.3.1.4.3 Test Requirements	3231

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	3231

A.7.3.1.5.1	Test Purpose and Environment	3231

A.7.3.1.5.2	Test Parameters	3231

A.7.3.1.5.3 Test Requirements	3238

A.7.3.1.6	Handover with PSCell from SA to EN-DC with unknown FR2 target PScell	3238

A.7.3.1.6.1	Test Purpose and Environment	3238

A.7.3.1.6.2	Test Parameters	3238

A.7.3.1.6.3	Test Requirements	3246

A.7.3.1.7	HO with PSCell from FR1 NR-SA to EN-DC with known E-UTRA PCell and known FR2 PSCell	3247

A.7.3.1.7.1	Test purpose and environment	3247

A.7.3.1.7.2	Test Requirements	3252

A.7.3.1.8	NR PSCell change delay in HO with PSCell from NR-DC to NR-DC	3253

A.7.3.1.8.1	Test Purpose and Environment	3253

A.7.3.1.8.2	Test Requirements	3258

A.7.3.1.9	Intra-frequency handover from FR2-2 to FR2-2; unknown target cell	3258

A.7.3.1.9.1	Test Purpose and Environment	3258

A.7.3.1.9.2	Test Parameters	3258

A.7.3.1.9.3	Test Requirements	3261

A.7.3.1.10	Inter-frequency handover from FR2-2 to FR2-2; unknown target cell	3261

A.7.3.1.10.1	Test Purpose and Environment	3261

A.7.3.1.10.2	Test Parameters	3261

A.7.3.1.10.3	Test Requirements	3265

A.7.3.1.11	Inter-frequency handover from FR1 to FR2-2; unknown target cell	3265

A.7.3.1.11.1	Test Purpose and Environment	3265

A.7.3.1.11.2	Test Parameters	3265

A.7.3.1.11.3	Test Requirements	3269

A.7.3.1.12	Intra-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB	3269

A.7.3.1.12.1	Test Purpose and Environment	3269

A.7.3.1.12.2	Test Parameters	3269

A.7.3.1.12.3	Test Requirements	3273

A.7.3.1.13	Inter-frequency handover from FR2 to FR2; known target cell configured with NCD-SSB	3273

A.7.3.1.13.1	Test Purpose and Environment	3273

A.7.3.1.13.2	Test Parameters	3273

A.7.3.1.13.3	Test Requirements	3276

A.7.3.1.14	Handover with PSCell from FR1-FR2 NR-DC to FR1-FR1 NR-DC with target PSCell in FR1	3276

A.7.3.1.14.1	Test Purpose and Environment	3276

A.7.3.1.14.2	Test Requirements	3280

A.7.3.1.15	HO with PSCell from FR1-FR1 NR-DC to FR1-FR2 NR-DC	3281

A.7.3.1.15.1	Test Purpose and Environment	3281

A.7.3.1.15.2	Test Requirements	3286

A.7.3.2	RRC Connection Mobility Control	3286

A.7.3.2.1	SA: RRC Re-establishment	3286

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	3286

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	3289

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	3292

A.7.3.2.1.3.1	Test Purpose and Environment	3292

A.7.3.2.1.3.2	Test Requirements	3294

A.7.3.2.1.4	Intra-frequency RRC Re-establishment in FR2-2	3295

A.7.3.2.1.4.1	Test Purpose and Environment	3295

A.7.3.2.1.4.2	Test Requirements	3297

A.7.3.2.1.5	Inter-frequency RRC Re-establishment in FR2-2	3298

A.7.3.2.1.5.1	Test Purpose and Environment	3298

A.7.3.2.1.5.2	Test Requirements	3301

A.7.3.2.1.6	Intra-frequency RRC Re-establishment in FR2-2 without serving cell timing	3301

A.7.3.2.1.6.1	Test Purpose and Environment	3301

A.7.3.2.1.6.2	Test Requirements	3303

A.7.3.2.2	Random Access	3304

A.7.3.2.2.1	4-step RA type c ontention based random access test in FR2 for NR Standalone	3304

A.7.3.2.2.2	4-step RA type n on-contention based random access test in FR2 for NR Standalone	3308

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	3312

A.7.3.2.2.4	2-step RA type n on-contention based random access test in FR2 for NR Standalone	3315

A.7.3.2.3	SA: RRC Connection Release with Redirection	3318

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	3318

A.7.3.3 Conditional Handover	3322

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	3322

A.7.3.3.1.1	Test Purpose and Environment	3322

A.7.3.3.1.2	Test Parameters	3322

A.7.3.3.1.2.3	Test Requirements	3325

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	3325

A.7.3.3.2.1	Test Purpose and Environment	3325

A.7.3.3.2.2	Test Parameters	3325

A.7.3.3.2.3Test Requirements	3327

A.7.3.3.3	NES triggering intra-frequency target CHO delay From FR2 to FR2	3327

A.7.3.3.3.1	Test Purpose and Environment	3327

A.7.3.3.3.2	Test Parameters	3327

A.7.3.3.3.2.3	Test Requirements	3330

A.7.3.3.4	NES triggering inter-frequency conditional handover from FR2 to FR1	3330

A.7.3.3.4.1	Test Purpose and Environment	3330

A.7.3.3.4.2	Test Parameters	3330

A.7.3.3.4.3	Test Requirements	3333

A.7.3.4	LTM PCell Switch	3333

A.7.3.4.1	RACH based Intra-frequency PCell switch from FR2 to FR2	3333

A.7.3.4.1.1	Test Purpose and Environment	3333

A.7.3.4.1.2	Test Parameters	3333

A.7.3.4.1.3 Test Requirements	3337

A.7.3.4.2	RACH-less Intra-frequency PCell switch from FR2 to FR2	3338

A.7.3.4.2.1	Test Purpose and Environment	3338

A.7.3.4.2.2	Test Parameters	3338

A.7.3.4.2.3 Test Requirements	3342

A.7.3.4.3	RACH-based Inter-frequency LTM PCell switch from FR2 to FR2	3343

A.7.3.4.3.1	Test Purpose and Environment	3343

A.7.3.4.3.2	Test Parameters	3343

A.7.3.4.3.3 Test Requirements	3347

A.7.3.5	LTM PSCell Switch	3347

A.7.3.5.1	RACH-based Intra-frequency LTM PSCell switch from FR2 to FR2	3347

A.7.3.5.1.1	Test Purpose and Environment	3347

A.7.3.5.1.2	Test Parameters	3347

A.7.3.5.1.3	Test Requirements	3352

A.7.4	Timing	3353

A.7.4.1	UE transmit timing	3353

A.7.4.1.1	NR UE Transmit Timing Test for FR2	3353

A.7.4.1.1.1	Test Purpose and environment	3353

A.7.4.1.1.2	Test requirements	3356

A.7.4.1.2	NR UE Transmit Timing Test for FR2-2	3357

A.7.4.1.2.1	Test Purpose and environment	3357

A.7.4.1.2.2	Test requirements	3361

A.7.4.1.3	NR UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18	3362

A.7.4.1.3.1	Test Purpose and environment	3362

A.7.4.1.3.2	Test requirements	3366

A.7.4.2	UE timer accuracy	3367

A.7.4.3	Timing advance	3367

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	3367

A.7.4.3.1.1	Test Purpose and Environment	3367

A.7.4.3.1.2	Test Parameters	3367

A.7.4.3.1.3 Test Requirements	3370

A.7.4.3.2	SA FR2-2 timing advance adjustment accuracy	3371

A.7.4.3.2.1	Test Purpose and Environment	3371

A.7.4.3.2.2	Test Parameters	3371

A.7.4.3.2.3	Test Requirements	3375

A.7.5	Signaling characteristics	3375

A.7.5.1	Radio link Monitoring	3375

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3375

A.7.5.1.1.1	Test Purpose and Environment	3375

A.7.5.1.1.2	Test Requirements	3378

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3379

A.7.5.1.2.1	Test Purpose and Environment	3379

A.7.5.1.2.2	Test Requirements	3383

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3384

A.7.5.1.3.1	Test Purpose and Environment	3384

A.7.5.1.3.2	Test Requirements	3388

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3388

A.7.5.1.4.1	Test Purpose and Environment	3388

A.7.5.1.4.2	Test Requirements	3393

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3393

A.7.5.1.5.1	Test Purpose and Environment	3393

A.7.5.1.5.2	Test Requirements	3398

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	3398

A.7.5.1.6.1	Test Purpose and Environment	3398

A.7.5.1.6.2	Test Requirements	3402

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3402

A.7.5.1.7.1	Test Purpose and Environment	3402

A.7.5.1.7.2	Test Requirements	3406

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	3406

A.7.5.1.8.1	Test Purpose and Environment	3406

A.7.5.1.8.2	Test Requirements	3411

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	3411

A.7.5.1.9.1	Test Purpose and Environment	3411

A.7.5.1.9.2	Test Requirements	3414

A.7.5.1.10	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting fast beam sweeping in multi-Rx	3414

A.7.5.1.10.1	Test Purpose and Environment	3414

A.7.5.1.10.2	Test Requirements	3418

A.7.5.1.11	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode when CD-SSB is outside active BWP	3419

A.7.5.1.11.1	Test Purpose and Environment	3419

A.7.5.1.12	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode when CD-SSB is outside active BWP	3419

A.7.5.1.12.1	Test Purpose and Environment	3419

A.7.5.1.12.2	Test Requirements	3419

A.7.5.1.13	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode for UE supporting [FG 53-3]	3419

A.7.5.1.13.1	Test Purpose and Environment	3419

A.7.5.1.13.2	Test Requirements	3423

A.7.5.2	Interruption	3423

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	3423

A.7.5.2.1.1	Test Purpose and Environment	3423

A.7.5.2.1.2	Test Requirements	3425

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	3426

A.7.5.2.2.1	Test Purpose and Environment	3426

A.7.5.2.2.2	Test Parameters	3426

A.7.5.2.2.3	Test Requirements	3428

A.7.5.3	SCell Activation and Deactivation Delay	3428

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	3428

A.7.5.3.1.1	Test Purpose and Environment	3428

A.7.5.3.1.2	Test Requirements	3430

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	3430

A.7.5.3.2.1	Test Purpose and Environment	3430

A.7.5.3.2.2	Test Requirements	3434

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	3435

A.7.5.3.3.1	Test Purpose and Environment	3435

A.7.5.3.3.2	Test Requirements	3438

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	3439

A.7.5.3.4.1	Test Purpose and Environment	3439

A.7.5.3.4.2	Test Requirements	3442

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	3443

A.7.5.3.5.1	Test Purpose and Environment	3443

A.7.5.3.5.2	Test Requirements	3446

A.7.5.3.6	PUCCH SCell activation and deactivation for FR1+FR2 inter-band with target SCell in FR2 and known	3447

A.7.5.3.6.1	Test Purpose and Environment	3447

A.7.5.3.6.2	Test Requirements	3451

A.7.5.3.7	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR1 PCell	3452

A.7.5.3.7.1	Test Purpose and Environment	3452

A.7.5.3.7.2	Test Requirements	3456

A.7.5.3.8	SCell Activation and deactivation for known PUCCH SCell in FR2 inter-band in non-DRX	3457

A.7.5.3.8.1	Test Purpose and Environment	3457

A.7.5.3.8.2	Test Requirements	3461

A.7.5.3.9	PUCCH SCell Activation and deactivation of unknown SCell in FR2	3462

A.7.5.3.9.1	Test Purpose and Environment	3462

A.7.5.3.9.2	Test Requirements	3465

A.7.5.3.10	SCell Activation and deactivation of FR2 known PUCCH SCell and one FR2 unknown SCell with FR2 PCell	3466

A.7.5.3.10.1	Test Purpose and Environment	3466

A.7.5.3.10.2	Test Requirements	3470

A.7.5.3.11	PUCCH SCell activation and deactivation delay requirements of FR2 unknown cell with FR2 PCell	3471

A.7.5.3.11.1	PUCCH SCell activation with non-PUCCH SCell in a secondary PUCCH Group	3471

A.7.5.3.11.1.1	Test Purpose and Environment	3471

A.7.5.3.11.1.2	Test Requirements	3475

A.7.5.3.11.2	PUCCH SCell activation with non-PUCCH SCell in a primary PUCCH Group	3476

A.7.5.3.11.2.1	Test Purpose and Environment	3476

A.7.5.3.11.2.2	Test Requirements	3480

A.7.5.3.12	Void	3481

A.7.5.3.13	SCell Activation for SCell in FR2 intra-band in non-DRX	3481

A.7.5.3.13.1	Test Purpose and Environment	3481

A.7.5.3.13.2	Test Requirements	3483

A.7.5.3.14	SCell Activation for known SCell in FR2 inter-band	3484

A.7.5.3.14.1	Test Purpose and Environment	3484

A.7.5.3.14.2	Test Requirements	3487

A.7.5.3.15	PUCCH SCell activation and deactivation with FR1 PCell based on L3 reporting after SCell activation command	3487

A.7.5.3.15.1	Test Purpose and Environment	3487

A.7.5.3.15.2	Test Requirements	3492

A.7.5.3.16	PUCCH SCell activation and deactivation with FR2 PCell based on L3 reporting after SCell activation command	3493

A.7.5.3.16.1	Test Purpose and Environment	3493

A.7.5.3.16.2	Test Requirements	3496

A.7.5.3.17	SCell Activation and deactivation for SCell in FR2 inter-band in DRX for UE capable of small beam sweeping factors and/or short measurement interval	3497

A.7.5.3.17.1	Test Purpose and Environment	3497

A.7.5.3.17.2	Test Requirements	3500

A.7.5.3.18	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2, in DRX, for UE capable of small beam sweeping factors and/or short measurement interval	3502

A.7.5.3.18.1	Test Purpose and Environment	3502

A.7.5.3.18.2	Test Requirements	3506

A.7.5.4	Void	3508

A.7.5.5	Beam Failure Detection and Link recovery procedures	3508

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3508

A.7.5.5.1.1	Test Purpose and Environment	3508

A.7.5.5.1.2	Test Requirements	3512

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	3513

A.7.5.5.2.1	Test Purpose and Environment	3513

A.7.5.5.2.2	Test Requirements	3516

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3517

A.7.5.5.3.1	Test Purpose and Environment	3517

A.7.5.5.3.2	Test Requirements	3521

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3522

A.7.5.5.4.1	Test Purpose and Environment	3522

A.7.5.5.4.2	Test Requirements	3526

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3527

A.7.5.5.5.1	Test Purpose and Environment	3527

A.7.5.5.5.2	Test Requirements	3530

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	3531

A.7.5.5.6.1	Test Purpose and Environment	3531

A.7.5.5.6.2	Test Requirements	3535

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3535

A.7.5.5.7.1	Test Purpose and Environment	3535

A.7.5.5.7.2	Test Requirements	3539

A.7.5.5.8	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode for UE fulfilling relaxed measurement criterion	3540

A.7.5.5.8.1	Test Purpose and Environment	3540

A.7.5.5.8.2	Test Requirements	3544

A.7.5.5.9	TRP specific Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	3544

A.7.5.5.9.1	Test Purpose and Environment	3544

A.7.5.5.9.2	Test Requirements	3550

A.7.5.5.10	TRP specific Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3550

A.7.5.5.10.1	Test Purpose and Environment	3550

A.7.5.5.10.2	Test Requirements	3556

A.7.5.5.11	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3556

A.7.5.5.11.1	Test Purpose and Environment	3556

A.7.5.5.11.2	Test Requirements	3561

A.7.5.5.12	Beam Failure Detection and Link Recovery Test for FR2-2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3561

A.7.5.5.12.1	Test Purpose and Environment	3561

A.7.5.5.12.2	Test Requirements	3566

A.7.5.5.13	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2-2 PCell configured with SSB-based BFD and LR in non-DRX mode	3566

A.7.5.5.13.1	Test Purpose and Environment	3566

A.7.5.5.13.2	Test Requirements	3570

A.7.5.5.14	TRP specific Beam Failure Detection and Link Recovery for FR2 PCell configured with CSI-RS-based BFD and LR and multi-Rx operation in DRX mode	3570

A.7.5.5.14.1	Test Purpose and Environment	3570

A.7.5.5.14.2	Test Requirements	3574

A.7.5.6	Active BWP switch	3575

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	3575

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3575

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA	3579

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	3584

A.7.5.6.1.3.1	Test Purpose and Environment	3584

A.7.5.6.1.3.2	Test Requirements	3587

A.7.5.6.1.4	NR FR2-2- NR FR2-2 DL active BWP switch of SCell with non-DRX in SA	3587

A.7.5.6.1.4.1	Test Purpose and Environment	3587

A.7.5.6.1.4.2	Test Requirements	3592

A.7.5.6.2	RRC-based Active BWP Switch	3593

A.7.5.6.2.1.1	Test Purpose and Environment	3593

A.7.5.6.2.1.2	Test Requirements	3596

A.7.5.6.2.2	NR FR2-2 DL active BWP switch of PCell with non-DRX in SA	3597

A.7.5.6.2.2.1	Test Purpose and Environment	3597

A.7.5.6.2.2.2	Test Requirements	3601

A.7.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	3602

A.7.5.6.3.1.1	Test Purpose and Environment	3602

A.7.5.6.3.1.2	Test Requirements	3605

A.7.5.6.4	SCell dormancy switch	3605

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	3605

A.7.5.6.4.1.1	Test Purpose and Environment	3605

A.7.5.6.4.1.2	Test Requirements	3610

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	3610

A.7.5.6.4.2.1	 Test Purpose and Environment	3610

A.7.5.6.4.2.2	 Test Requirements	3615

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	3615

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	3615

A.7.5.6.5.2	NR FR2-2 Active BWP switch on multiple SCells with non-DRX in SA	3618

A.7.5.6.5.2.1	Test Purpose and Environment	3618

A.7.5.6.5.2.2	Test Requirements	3622

A.7.5.7	PSCell addition and release delay	3623

A.7.5.7.1	Addition and Release Delay of known NR PSCell	3623

A.7.5.7.1.1	Test Purpose and Environment	3623

A.7.5.7.1.2	Test Requirements	3626

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell in	3626

A.7.5.7.2.1	Test Purpose and Environment	3626

A.7.5.7.2.2	Test Requirements	3629

A.7.5.7.3	Addition and Release Delay of known NR PSCell in FR2-2	3629

A.7.5.7.3.1	Test Purpose and Environment	3629

A.7.5.7.3.2	Test Requirements	3632

A.7.5.7.4	Addition and Release Delay of unknown NR PSCell in FR2-2	3632

A.7.5.7.4.1	Test Purpose and Environment	3632

A.7.5.7.4.2	Test Requirements	3635

A.7.5.8	Active TCI state switch delay	3635

A.7.5.8.1	MAC-CE based active TCI state switch	3635

A.7.5.8.2	RRC based active TCI state switch	3639

A.7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	3643

A.7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	3643

A.7.5.8.3.1.1	Test Purpose and Environment	3643

A.7.5.8.3.1.2	Test Requirements	3647

A.7.5.8.3.2	NR PCell FR2 HST active TCI state switch for PC6 UE supporting tci-StateSwitchInd-r18 for a known TCI state	3648

A.7.5.8.3.2.1	Test Purpose and Environment	3648

A.7.5.8.3.2.2	Test Requirements	3651

A.7.5.8.4	DCI based active TCI state switch with m-DCI for simultaneous reception	3652

A.7.5.8.4.1	Test Purpose and Environment	3652

A.7.5.8.4.2	Test Requirements	3654

A.7.5.8.5	Single-DCI FR2 DCI based active TCI state switch with known target TCI states for simultaneous reception	3654

A.7.5.8.5.1	Test Purpose and Environment	3654

A.7.5.8.5.1.2	Test Requirements	3657

A.7.5.9	Uplink spatial relation switch delay	3657

A.7.5.9.1.1.1	Test Purpose and Environment	3657

A.7.5.9.1.1.2	Test Requirements	3660

A.7.5.9.2	RRC based spatial relation switch	3660

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	3660

A.7.5.9.2.1.1	Test Purpose and Environment	3660

A.7.5.9.2.1.2	Test Requirements	3663

A.7.5.10	UE specific CBW change	3663

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	3663

A.7.5.10.1.1	Test Purpose and Environment	3663

A.7.5.10.1.2	Test Requirements	3666

A.7.5.11	UE UL carrier RRC reconfiguration Delay	3667

A.7.5.11.1	UE UL carrier RRC reconfiguration Delay	3667

A.7.5.11.1.1	Test Purpose and Environment	3667

A.7.5.11.1.2	Test Requirements	3670

A.7.5.12	Conditional PSCell addition and release delay (FR2 SA)	3670

A.7.5.12.1	Addition and Release Delay of PSCell	3670

A.7.5.12.1.1	Test purpose and environment	3670

A.7.5.12.1.2	Test Parameters	3670

A.7.5.12.1.3	Test Requirements	3673

A.7.5.13	 Unified TCI state switching delay	3673

A.7.5.13.1	MAC-CE based active joint TCI state switching	3673

A.7.5.13.1.1	NR PCell FR2 active joint TCI state switch for a known TCI state	3673

A.7.5.13.1.1.1	Test Purpose and Environment	3673

A.7.5.13.1.1.2	Test parameters	3674

A.7.5.13.1.1.3	Test Requirements	3676

A.7.5.13.2	 MAC-CE based active uplink TCI state switch	3676

A.7.5.13.2.1	 NR FR2 PCell uplink TCI state switch for a known TCI state	3676

A.7.5.13.2.1.1	Test Purpose and Environment	3676

A.7.5.13.2.1.2	Test parameters	3677

A.7.5.13.2.1.3	Test Requirements	3679

A.7.5.13.3	MAC-CE based active downlink TCI state switch	3679

A.7.5.13.3.1	NR PCell FR2 active downlink TCI state switch to cell with additional PCI for a known TCI state	3679

A.7.5.13.3.1.1	Test Purpose and Environment	3679

A.7.5.13.3.1.2	Test Parameters	3680

A.7.5.13.3.1.3	Test Requirements	3683

A.7.5.13.4	sDCI MAC-CE based joint TCI state switching	3683

A.7.5.13.4.1	NR PCell FR2 dual downlink and uplink TCI state switch in sDCI for known case	3683

A.7.5.13.4.1.1	Test Purpose and Environment	3683

A.7.5.13.4.1.2	Test parameters	3684

A.7.5.13.4.1.3	Test Requirements	3688

A.7.5.13.5	MAC-CE based dual downlink TCI state switching delay for unified TCI for single-DCI mTRP	3688

A.7.5.13.5.1	NR PCell FR2 dual downlink TCI state switch in sDCI for known case	3688

A.7.5.13.5.1.1	Test Purpose and Environment	3688

A.7.5.13.5.1.2	Test Parameters	3689

A.7.5.13.5.1.3	Test Requirements	3691

A.7.5.13.6	 MAC-CE based active uplink TCI state switch for single-DCI mTRP	3691

A.7.5.13.6.1	 NR FR2 PCell uplink TCI state switch for two known TCI states	3691

A.7.5.13.6.1.1	Test Purpose and Environment	3691

A.7.5.13.6.1.2	Test parameters	3692

A.7.5.13.6.1.3	Test Requirements	3695

A.7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	3695

A.7.5.14.1	Test Purpose and Environment	3695

A.7.5.14.2	Test Requirements	3698

A.7.5.15	Void	3699

A.7.5.16	UE L1-RSRP Scheduling and Measureme t Restrictions on FR2-1	3699

A.7.5.16.1	Test Purpose and Environment	3699

A.7.5.16.2	Test Requirements	3703

A.7.5.17	SCG Activation and deactivation for FR1+FR1 inter-band with target PSCell in FR1	3703

A.7.5.17.1	Test Purpose and Environment	3703

A.7.5.17.2	Test Requirements	3707

A.7.5.18	Subsequent conditional PSCell addition/change	3708

A.7.5.18.1	Intra-frequency subsequent CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3708

A.7.5.18.1.1	Test purpose and environment	3708

A.7.5.18.1.2	Test Requirements	3711

A.7.5.18.2	Inter-frequency subsequent CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC	3712

A.7.5.18.2.1	Test Purpose and Environment	3712

A.7.5.18.2.2	Test Requirements	3715

A.7.6	Measurement procedure	3594

A.7.6.1	Intra-frequency Measurements	3594

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	3594

A.7.6.1.1.1	Test purpose and Environment	3594

A.7.6.1.1.2	Test Requirements	3597

A.7.6.1.2	SA event triggered reporting test without gap under DRX	3597

A.7.6.1.2.1	Test purpose and Environment	3597

A.7.6.1.2.2	Test Requirements	3600

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	3601

A.7.6.1.3.1	Test purpose and Environment	3601

A.7.6.1.3.2	Test Requirements	3604

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	3604

A.7.6.1.4.1	Test purpose and Environment	3604

A.7.6.1.4.2	Test Requirements	3607

A.7.6.1.5	SA event triggered reporting test without gap under non-DRX for UE configured with highSpeedMeasFlagFR2-r17	3608

A.7.6.1.5.1	Test purpose and Environment	3608

A.7.6.1.5.2	Test Requirements	3611

A.7.6.1.6	SA event triggered reporting test without gap under non-DRX for FR2-2	3611

A.7.6.1.6.1	Test purpose and Environment	3611

A.7.6.1.6.2	Test Requirements	3614

A.7.6.1.7	SA event triggered reporting test without gap under DRX for FR2-2	3615

A.7.6.1.7.1	Test purpose and Environment	3615

A.7.6.1.7.2	Test Requirements	3617

A.7.6.1.8	SA event triggered reporting test with per-UE gaps under non-DRX for FR2-2	3618

A.7.6.1.8.1	Test purpose and Environment	3618

A.7.6.1.8.2	Test Requirements	3621

A.7.6.1.9	SA event triggered reporting test with per-UE gaps under DRX for FR2-2	3622

A.7.6.1.9.1	Test purpose and Environment	3622

A.7.6.1.9.2	Test Requirements	3624

A.7.6.1.10	SA event triggered reporting test with SSB time index detection without gap under non-DRX for FR2-2	3625

A.7.6.1.10.1	Test purpose and Environment	3625

A.7.6.1.10.2	Test Requirements	3629

A.7.6.1.11	SA event triggered reporting test with SSB time index detection with per-UE gaps under non-DRX for FR2-2	3629

A.7.6.1.11.1	Test purpose and Environment	3629

A.7.6.1.11.2	Test Requirements	3632

A.7.6.1.12	SA event triggered reporting test without gap under non-DRX when CD-SSB is outside active BWP	3632

A.7.6.1.12.1	Test purpose and Environment	3632

A.7.6.1.12.2	Test Requirements	3633

A.7.6.1.13	SA event triggered reporting test without gap under non-DRX with NCD-SSB	3633

A.7.6.1.13.1	Test purpose and Environment	3633

A.7.6.1.13.2	Test Requirements	3637

A.7.6.1.14	SA event triggered reporting test without gap under non-DRX for power class 6 UE supporting measEnhCAInterFreqFR2-r18	3637

A.7.6.1.14.1	Test Purpose and Environment	3637

A.7.6.1.14.2	Test Requirements	3639

A.7.6.2	Inter-frequency Measurements	3640

A.7.6.2.1	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR2)	3640

A.7.6.2.1.1	Test Purpose and Environment	3640

A.7.6.2.1.2	Test Requirements	3643

A.7.6.2.2	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR2)	3643

A.7.6.2.2.1	Test Purpose and Environment	3643

A.7.6.2.2.2	Test Requirements	3647

A.7.6.2.3	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR2)	3647

A.7.6.2.3.1	Test Purpose and Environment	3647

A.7.6.2.3.2	Test Requirements	3651

A.7.6.2.4	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (Pcell in FR2)	3651

A.7.6.2.4.1	Test Purpose and Environment	3651

A.7.6.2.4.2	Test Requirements	3655

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	3655

A.7.6.2.5.1	Test Purpose and Environment	3655

A.7.6.2.5.2	Test Requirements	3659

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (Pcell in FR1)	3660

A.7.6.2.6.1	Test Purpose and Environment	3660

A.7.6.2.6.2	Test Requirements	3664

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	3665

A.7.6.2.7.1	Test Purpose and Environment	3665

A.7.6.2.7.2	Test Requirements	3669

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	3670

A.7.6.2.8.1	Test Purpose and Environment	3670

A.7.6.2.8.2	Test Requirements	3674

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	3675

A.7.6.2.9.1	Test Purpose and Environment	3675

A.7.6.2.9.2	Test Requirements	3679

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	3679

A.7.6.2.10.1	Test Purpose and Environment	3679

A.7.6.2.10.2	Test Requirements	3681

A.7.6.2.11	SA event triggered reporting test without gap under DRX	3682

A.7.6.2.11.1	Test Purpose and Environment	3682

A.7.6.2.11.2	Test Requirements	3684

A.7.6.2.12	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR2-2)	3685

A.7.6.2.12.1	Test Purpose and Environment	3685

A.7.6.2.12.2	Test Requirements	3688

A.7.6.2.13	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR2-2)	3689

A.7.6.2.13.1	Test Purpose and Environment	3689

A.7.6.2.13.2	Test Requirements	3693

A.7.6.2.14	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR2-2)	3694

A.7.6.2.14.1	Test Purpose and Environment	3694

A.7.6.2.14.2	Test Requirements	3698

A.7.6.2.15	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR2-2)	3699

A.7.6.2.15.1	Test Purpose and Environment	3699

A.7.6.2.15.2	Test Requirements	3703

A.7.6.2.16	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is not used (PCell in FR1)	3704

A.7.6.2.16.1	Test Purpose and Environment	3704

A.7.6.2.16.2	Test Requirements	3710

A.7.6.2.17	SA event triggered reporting tests for FR2-2 without SSB time index detection when DRX is used (PCell in FR1)	3710

A.7.6.2.17.1	Test Purpose and Environment	3710

A.7.6.2.17.2	Test Requirements	3717

A.7.6.2.18	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is not used (PCell in FR1)	3718

A.7.6.2.18.1	Test Purpose and Environment	3718

A.7.6.2.18.2	Test Requirements	3724

A.7.6.2.19	SA event triggered reporting tests for FR2-2 with SSB time index detection when DRX is used (PCell in FR1)	3724

A.7.6.2.19.1	Test Purpose and Environment	3724

A.7.6.2.19.2	Test Requirements	3732

A.7.6.2.20	SA event triggered reporting tests for FR2 with a measurement gap and two periodic MUSIM gaps configured	3733

A.7.6.2.20.1	Test Purpose and Environment	3733

A.7.6.2.20.2	Test Requirements	3736

A.7.6.2.21	SA event triggered reporting tests for FR2 with MUSIM gap partially partial overlapping scenario configured (PCell in FR2)	3736

A.7.6.2.21.1	Test Purpose and Environment	3736

A.7.6.2.21.2	Test Requirements	3739

A.7.6.2.22	SA event triggered reporting tests with SSB time index detection when DRX is not used (PCell in FR2) for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3739

A.7.6.2.22.1	Test Purpose and Environment	3739

A.7.6.2.22.2	Test Requirements	3743

A.7.6.2.23	SA event triggered reporting tests without SSB time index detection when DRX is not used (PCell in FR2) for FR2 power class 6 UE configured with highSpeedMeasFlagFR2-r17	3743

A.7.6.2.23.1	Test Purpose and Environment	3743

A.7.6.2.23.2	Test Requirements	3745

A.7.6.3	L1-RSRP measurement for beam reporting	3746

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3746

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	3746

A.7.6.3.1.1	Test Purpose and Environment	3746

A.7.6.3.1.2	Test parameters	3746

A.7.6.3.1.3	Test Requirements	3748

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	3748

A.7.6.3.2.1	Test Purpose and Environment	3748

A.7.6.3.2.2	Test parameters	3749

A.7.6.3.2.3	Test Requirements	3751

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	3751

A.7.6.3.3.1	Test Purpose and Environment	3751

A.7.6.3.3.2	Test parameters	3752

A.7.6.3.3.3	Test Requirements	3754

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	3755

A.7.6.3.4.1	Test Purpose and Environment	3755

A.7.6.3.4.2	Test parameters	3755

A.7.6.3.3.3	Test Requirements	3757

A.7.6.3.5	SSB based L1-RSRP measurement when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	3758

A.7.6.3.5.1	Test Purpose and Environment	3758

A.7.6.3.5.2	Test parameters	3758

A.7.6.3.5.3	Test Requirements	3760

A.7.6.3.6	Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	3760

A.7.6.3.6.1	Test Purpose and Environment	3760

A.7.6.3.6.2	Test parameters	3761

A.7.6.3.6.3	Test Requirements	3764

A.7.6.3.7	SSB based L1-RSRP measurement for FR2-2 when DRX is used	3764

A.7.6.3.7.1	Test Purpose and Environment	3764

A.7.6.3.7.2	Test parameters	3765

A.7.6.3.7.3	Test Requirements	3768

A.7.6.3.8	CSI-RS based L1-RSRP measurement when DRX is not used and when CD-SSB is outside active BWP	3769

A.7.6.3.8.1	Test Purpose and Environment	3769

A.7.6.3.9	SSB based L1-RSRP measurement when DRX is not used when CD-SSB is outside active BWP	3769

A.7.6.3.9.1	Test Purpose and Environment	3769

A.7.6.3.9.2	Test Requirements	3769

A.7.6.3.10	SSB based L1-RSRP measurement for UE supporting NCD-SSB based L1 measurement outside active BWP when DRX is not used	3769

A.7.6.3.10.1	Test Purpose and Environment	3769

A.7.6.3.10.2	Test parameters	3770

A.7.6.3.10.3	Test Requirements	3772

A.7.6.3.11	SSB based L1-RSRP measurement when DRX is used for power class 6 UE supporting simultaneousReceptionTwoQCL-r18	3772

A.7.6.3.11.1	Test Purpose and Environment	3772

A.7.6.3.11.2	Test parameters	3773

A.7.6.3.11.3	Test Requirements	3775

A.7.6.4	CLI measurements	3775

A.7.6.4.1	SRS-RSRP measurement with non-DRX	3775

A.7.6.4.1.1	Test Purpose and Environment	3775

A.7.6.4.1.2	Test Parameters	3776

A.7.6.4.1.3	Test Requirements	3778

A.7.6.4.2	CLI-RSSI measurement with non-DRX	3778

A.7.6.4.2.1	Test Purpose and Environment	3778

A.7.6.4.2.2	Test Parameters	3779

A.7.6.4.2.3	Test Requirements	3780

A.7.6.5.1   SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	3781

A.7.6.5.1.1	Test Purpose and Environment	3781

A.7.6.5.1.2	Test Requirements	3784

A.7.6.6	L1-SINR measurement for beam reporting	3784

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	3786

A.7.6.6.2.1	Test Purpose and Environment	3786

A.7.6.6.2.2	Test parameters	3787

A.7.6.6.2.3	Test Requirements	3789

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	3789

A.7.6.6.3.1	Test Purpose and Environment	3789

A.7.6.6.3.2	Test parameters	3790

A.7.6.6.3.3	Test Requirements	3792

A.7.6.7	CSI-RS based intra-frequency Measurements	3792

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	3792

A.7.6.7.1.1	Test purpose and Environment	3792

A.7.6.7.1.2	Test Requirements	3795

A.7.6.8	CSI-RS based inter-frequency Measurements	3795

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	3795

A.7.6.8.1.1	Test Purpose and Environment	3795

A.7.6.8.1.2	Test Requirements	3798

A.7.6.9	RSTD measurements	3798

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	3798

A.7.6.9.1.1	Test Purpose and Environment	3798

A.7.6.9.1.2	Test Requirements	3806

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	3806

A.7.6.9.2.1	Test Purpose and Environment	3806

A.7.6.9.2.2	Test Requirements	3814

A.7.6.9.3	NR RSTD measurement reporting delay test case for single positioning frequency layer with reduced number of samples in FR2 SA	3814

A.7.6.9.3.1	Test Purpose and Environment	3814

A.7.6.9.3.2	Test Requirements	3820

A.7.6.9.4	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA without measurement gap	3821

A.7.6.9.4.1	Test Purpose and Environment	3821

A.7.6.9.4.2	Test Requirements	3824

A.7.6.9.5	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_CONNECTED state with Rx TEG	3825

A.7.6.9.5.1	Test Purpose and Environment	3825

A.7.6.9.5.2	Test Requirements	3828

A.7.6.10 PRS-RSRP measurements	3829

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	3829

A.7.6.10.1.1	Test Purpose and Environment	3829

A.7.6.10.1.2	Test Requirements	3833

A.7.6.10.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	3833

A.7.6.10.2.1	Test Purpose and Environment	3833

A.7.6.10.2.2	Test Requirements	3837

A.7.6.10.3	PRS-RSRP reporting delay test case for reduced number of samples	3837

A.7.6.10.3.1	Test Purpose and Environment	3837

A.7.6.10.3.2	Test Requirements	3841

A.7.6.10.4	PRS-RSRP reporting delay test case for single positioning frequency layer outside MG	3841

A.7.6.10.4.1	Test Purpose and Environment	3841

A.7.6.10.4.2	Test Requirements	3845

A.7.6.11	UE Rx-Tx time difference measurements	3845

A.7.6.11.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	3845

A.7.6.11.1.1	Test purpose and environment	3845

A.7.6.11.1.2	Test requirements	3849

A.7.6.11.2	UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	3849

A.7.6.11.2.1	Test purpose and environment	3849

A.7.6.11.2.2	Test requirements	3853

A.7.6.11.3	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with reduced sample number	3853

A.7.6.11.3.1	Test purpose and environment	3853

A.7.6.11.3.2	Test requirements	3855

A.7.6.11.4	UE Rx-Tx time difference measurements without gaps in FR2 SA	3856

A.7.6.11.4.1	Test purpose and environment	3856

A.7.6.11.4.2	Test requirements	3858

A.7.6.11.5	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA with RxTx TEG	3859

A.7.6.11.5.1	Test purpose and environment	3859

A.7.6.11.5.2	Test requirements	3861

A.7.6.12	PRS-RSRPP measurements	3862

A.7.6.12.1 PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_CONNECTED state	3862

A.7.6.12.1.1	Test Purpose and Environment	3862

A.7.6.12.1.2	Test Requirements	3864

A.7.6.12.2	PRS-RSRPP reporting delay test case for reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	3865

A.7.6.12.2.1	Test Purpose and Environment	3865

A.7.6.12.2.2	Test Requirements	3867

A.7.6.12.3	PRS-RSRPP reporting delay test case for gapless measurement in FR2	3868

A.7.6.12.3.1	Test Purpose and Environment	3868

A.7.6.12.3.2	Test Requirements	3871

A.7.6.13	UE Rx-Tx time difference measurements for PDC	3871

A.7.6.13.1	UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2	3871

A.7.6.13.1.1	Test purpose and environment	3871

A.7.6.13.1.2	Test requirements	3873

A.7.6.13.2	UE Rx-Tx time difference measurement for propagation delay compensation using TRS in FR2	3874

A.7.6.13.2.1	Test purpose and environment	3874

A.7.6.13.2.2	Test requirements	3876

A.7.6.14	SA event triggered reporting tests with Pre-MG	3877

A.7.6.14.1	Intra-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MG in FR2	3877

A.7.6.14.1.1	Test purpose and Environment	3877

A.7.6.14.1.2	Test parameters	3877

A.7.6.14.1.3	Test Requirements	3880

A.7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	3880

A.7.6.14.2.1	Test purpose and Environment	3880

A.7.6.14.2.2	Test parameters	3880

A.7.6.14.2.3	Test Requirements	3883

A.7.6.15	SA event triggered reporting tests with concurrent gaps	3883

A.7.6.15.1	SA event triggered reporting tests For FR2 with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	3883

A.7.6.15.1.1	Test Purpose and Environment	3883

A.7.6.15.1.2	Test Requirements	3886

A.7.6.15.2	SA event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	3886

A.7.6.15.2.1	Test Purpose and Environment	3886

A.7.6.15.2.2	Test Requirements	3889

A.7.6.15.3	SA event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	3889

A.7.6.15.3.1	Test Purpose and Environment	3889

A.7.6.15.3.2	Test Requirements	3893

A.7.6.16	SA event triggered reporting tests with NCSG	3894

A.7.6.16.1	SA event triggered reporting test with per-UE NCSG under non-DRX	3894

A.7.6.16.1.1	Test purpose and Environment	3894

A.7.6.16.1.2	Test Requirements	3898

A.7.6.16.2	SA event triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used (PCell in FR2)	3898

A.7.6.16.2.1	Test Purpose and Environment	3898

A.7.6.16.2.2	Test Requirements	3901

A.7.6.16.3	Event triggered reporting test on deactivated Scell measurement via NCSG in FR2 in non-DRX	3901

A.7.6.16.3.1	Test Purpose and Environment	3901

A.7.6.16.3.2	Test Requirements	3904

A.7.6.17	SA event triggered reporting tests for concurrent measurement gaps with Pre-MG in FR2	3905

A.7.6.17.1	SA event triggered reporting test for FR2 with one pre-configured gap and one measurement gap	3905

A.7.6.17.1.1	Test Purpose and Environment	3905

A.7.6.17.1.2	Test Requirements	3908

A.7.6.18.1	Inter-frequency measurement test with SA event triggered reporting tests: with autonomous activation/deactivation of Pre-MGs in FR2	3908

A.7.6.18.1.1	Test purpose and Environment	3908

A.7.6.18.1.2	Test parameters	3909

A.7.6.18.1.3	Test Requirements	3911

A.7.6.18	SA event triggered reporting tests with concurrent gaps and NCSG	3912

A.7.6.18.1	SA event triggered reporting tests For FR2 with concurrent measurement gaps and NCSG without SSB time index detection when DRX is not used (PCell in FR2)	3912

A.7.6.18.1.1	Test Purpose and Environment	3912

A.7.6.18.1.2	Test Requirements	3915

A.7.6.19	SA event triggered reporting tests with NeedForGap in FR2	3915

A.7.6.19.1	SA event triggered reporting test for UE indicating NeedforInterruptionInfoNR under non-DRX and no interruption outside configured measurement gaps	3915

A.7.6.19.1.1	Test purpose and Environment	3915

A.7.6.19.1.2	Test Requirements	3919

A.7.6.19.2	SA event triggered reporting test without gap under non-DRX	3919

A.7.6.19.2.1	Test purpose and Environment	3919

A.7.6.19.2.2	Test Requirements	3922

A.7.6.20	LTM Intra-frequency L1-RSRP measurement	3922

A.7.6.20.1	Intra-frequency SSB based L1-RSRP measurement in FR2	3922

A.7.6.20.1.1	Test Purpose and Environment	3922

A.7.6.20.1.2	Test parameters	3922

A.7.6.20.1.3	Test Requirements	3925

A.7.6.21	LTM Inter-frequency L1-RSRP measurement with measurement gap	3925

A.7.6.21.1	Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM	3925

A.7.6.21.1.1	Test Purpose and Environment	3925

A.7.6.21.1.2	Test parameters	3926

A.7.6.21.1.3	Test Requirements	3928

A.7.6.22	LTM Inter-frequency L1-RSRP measurement without measurement gap	3928

A.7.6.22.1	Inter-frequency SSB based L1-RSRP measurement without measurement gap in FR2	3928

A.7.6.22.1.1	Test Purpose and Environment	3928

A.7.6.22.1.2	Test parameters	3929

A.7.6.22.1.3	Test Requirements	3932

A.7.6.23	Idle Mode CA/DC Measurements	3932

A.7.6.23.1	Test case for Idle mode fast CA/DC eEMR measurement for FR2	3932

A.7.6.23.1.1	Test Purpose and Environment	3932

A.7.6.23.1.2	Test Requirements	3937

A.7.6.23.2	Test case for Idle mode fast CA/DC cell reselection measurement for FR2 without valid reporting	3937

A.7.6.23.2.1	Test Purpose and Environment	3937

A.7.6.23.2.2	Test Requirements	3943

A.7.6.23.3 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 with valid reporting	3943

A.7.6.23.3.1	Test Purpose and Environment	3943

A.7.6.23.3.2	Test Requirements	3949

A.7.7	Measurement Performance requirements	3949

A.7.7.1	SS-RSRP	3950

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3950

A.7.7.1.1.1	Test Purpose and Environment	3950

A.7.7.1.1.2	Test parameters	3950

A.7.7.1.1.3	Test Requirements	3954

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3955

A.7.7.1.2.1	Test Purpose and Environment	3955

A.7.7.1.2.2	Test parameters	3955

A.7.7.1.2.3	Test Requirements	3959

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	3960

A.7.7.1.3.1	Test Purpose and Environment	3960

A.7.7.1.3.2	Test parameters	3960

A.7.7.1.3.3	Test Requirements	3964

A.7.7.2	SS-RSRQ	3964

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3964

A.7.7.2.1.1	Test Purpose and Environment	3964

A.7.7.2.1.2	Test Parameters	3964

A.7.7.2.1.3	Test Requirements	3966

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3966

A.7.7.2.2.1	Test Purpose and Environment	3966

A.7.7.2.2.2	Test Parameters	3966

A.7.7.2.2.3	Test Requirements	3968

A.7.7.3	SS-SINR	3968

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3968

A.7.7.3.1.1	Test Purpose and Environment	3968

A.7.7.3.1.2	Test Parameters	3968

A.7.7.3.1.3	Test Requirements	3970

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3970

A.7.7.3.2.1	Test Purpose and Environment	3970

A.7.7.3.2.2	Test Parameters	3970

A.7.7.3.2.3	Test Requirements	3972

A.7.7.4	L1-RSRP measurement for beam reporting	3972

A.7.7.4.1	SSB based L1-RSRP measurement	3972

A.7.7.4.1.1	Test Purpose and Environment	3972

A.7.7.4.1.2	Test parameters	3973

A.7.7.4.1.3	Test Requirements	3975

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	3976

A.7.7.4.2.1	Test Purpose and Environment	3976

A.7.7.4.2.2	Test parameters	3976

A.7.7.4.2.3	Test Requirements	3978

A.7.7.5	CLI measurements	3979

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	3979

A.7.7.5.1.1	Test Purpose and Environment	3979

A.7.7.5.1.2	Test parameters	3979

A.7.7.5.1.3	Test Requirements	3982

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	3983

A.7.7.5.2.1	Test Purpose and Environment	3983

A.7.7.5.2.2	Test parameters	3983

A.7.7.5.2.3	Test Requirements	3985

A.7.7.6	L1-SINR measurement for beam reporting	3986

A.7.7.6.1.1	Test Purpose and Environment	3986

A.7.7.6.1.2	Test parameters	3986

A.7.7.6.1.3	Test Requirements	3988

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	3989

A.7.7.6.2.1	Test Purpose and Environment	3989

A.7.7.6.2.2	Test parameters	3989

A.7.7.6.2.3	Test Requirements	3991

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	3992

A.7.7.6.3.1	Test Purpose and Environment	3992

A.7.7.6.3.2	Test parameters	3992

A.7.7.6.3.3	Test Requirements	3994

A.7.7.7	CSI-RSRP	3995

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3995

A.7.7.7.1.1	Test Purpose and Environment	3995

A.7.7.7.1.2	Test parameters	3995

A.7.7.7.1.3	Test Requirements	3998

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3999

A.7.7.7.2.1	Test Purpose and Environment	3999

A.7.7.7.2.2	Test parameters	3999

A.7.7.7.2.3	Test Requirements	4002

A.7.7.8	CSI-RSRQ	4003

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4003

A.7.7.8.1.1	Test Purpose and Environment	4003

A.7.7.8.1.2	Test Parameters	4003

A.7.7.8.1.3	Test Requirements	4005

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4005

A.7.7.8.2.1	Test Purpose and Environment	4005

A.7.7.8.2.2	Test Parameters	4005

A.7.7.8.2.3	Test Requirements	4007

A.7.7.9	CSI-SINR	4007

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	4007

A.7.7.9.1.1	Test Purpose and Environment	4007

A.7.7.9.1.2	Test Parameters	4007

A.7.7.9.1.3	Test Requirements	4009

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	4009

A.7.7.9.2.1	Test Purpose and Environment	4009

A.7.7.9.2.2	Test Parameters	4010

A.7.7.9.2.3	Test Requirements	4012

A.7.7.10	RSTD measurements	4012

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	4012

A.7.7.10.1.1	Test purpose and Environment	4012

A.7.7.10.1.2	Test Requirements	4015

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	4015

A.7.7.10.2.1	Test purpose and Environment	4015

A.7.7.10.2.2	Test Requirements	4018

A.7.7.10.3	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_CONNECTED state	4018

A.7.7.10.3.1	Test purpose and Environment	4018

A.7.7.10.3.2	Test Requirements	4020

A.7.7.10.4	RSTD measurement accuracy test case with Rx TEG	4020

A.7.7.10.4.1	Test purpose and Environment	4020

A.7.7.10.4.2	Test Requirements	4023

A.7.7.11	PRS-RSRP measurements	4023

A.7.7.11.1	SA measurement accuracy with PRS in FR2	4023

A.7.7.11.1.1	Test Purpose and Environment	4023

A.7.7.11.1.2	Test parameters	4023

A.7.7.11.1.3	Test Requirements	4027

A.7.7.11.2	SA measurement accuracy with PRS in FR2 with reduced sample number	4028

A.7.7.11.2.1	Test Purpose and Environment	4028

A.7.7.11.2.2	Test parameters	4028

A.7.7.11.2.3	Test Requirements	4030

A.7.7.12	UE Rx-Tx time difference measurements	4031

A.7.7.12.1	UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR2 SA	4031

A.7.7.12.1.1	Test purpose and environment	4031

A.7.7.12.1.2	Test parameters	4031

A.7.7.12.1.3	Test requirements	4034

A.7.7.12.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	4035

A.7.7.12.2.1	Test purpose and environment	4035

A.7.7.12.2.2	Test parameters	4035

A.7.7.12.2.3	Test requirements	4038

A.7.7.12.3	UE Rx-Tx time difference measurement accuracy with RxTx TEG	4038

A.7.7.12.3.1	Test purpose and environment	4038

A.7.7.12.3.2	Test parameters	4039

A.7.7.12.3.3	Test requirements	4042

A.7.7.13	PRS-RSRPP measurements	4042

A.7.7.13.1	SA measurement accuracy with PRS in FR2	4042

A.7.7.13.1.1	Test Purpose and Environment	4042

A.7.7.13.1.2	Test parameters	4043

A.7.7.13.1.3	Test Requirements	4045

A.7.7.13.2	SA measurement accuracy with reduced PRS samples in FR2	4045

A.7.7.13.2.1	Test Purpose and Environment	4045

A.7.7.13.2.2	Test parameters	4046

A.7.7.13.2.3	Test Requirements	4048

A.7.7.14	L1-RSRP measurement for group-based beam reporting	4049

A.7.7.14.1	SSB based L1-RSRP measurement	4049

A.7.7.14.1.1	Test Purpose and Environment	4049

A.7.7.14.1.2	Test parameters	4049

A.7.7.14.1.3	Test Requirements	4051

A.7.7.14.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	4052

A.7.7.14.2.1	Test Purpose and Environment	4052

A.7.7.14.2.2	Test parameters	4052

A.7.7.14.2.3	Test Requirements	4054

A.7.7.15	LTM L1-RSRP measurement	4055

A.7.7.15.1	SSB based inter-frequency L1-RSRP measurement	4055

A.7.7.15.1.1	Test Purpose and Environment	4055

A.7.7.15.1.2	Test parameters	4055

A.7.7.15.1.3	Test Requirements	4057

A.7.8	Measurement procedure in RRC_INACTIVE	4058

A.7.8.1	RSTD measurements	4058

A.7.8.1.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA in RRC_INACTIVE state	4058

A.7.8.1.1.1	Test Purpose and Environment	4058

A.7.8.1.1.2	Test Requirements	4061

A.7.8.1.2	NR RSTD measurement reporting delay test case with reduced number of samples in RRC_INACTIVE, FR1 SA	4062

A.7.8.1.2.1	Test Purpose and Environment	4062

A.7.8.1.2.2	Test Requirements	4067

A.7.8.2	PRS-RSRP measurements	4068

A.7.8.2.1	PRS-RSRP reporting delay test case for single positioning frequency layer in RRC_INACTIVE	4068

A.7.8.2.1.1	Test Purpose and Environment	4068

A.7.8.2.1.2	Test Requirements	4072

A.7.8.2.2	PRS-RSRP reporting delay test case with reduced number of samples in RRC_INACTIVE	4072

A.7.8.2.2.1	Test purpose and Environment	4072

A.7.8.2.2.2	Test Requirements	4076

A.7.8.3	UE Rx-Tx time difference measurements	4076

A.7.8.3.1	UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	4076

A.7.8.3.1.1	Test purpose and environment	4076

A.7.8.3.1.2	Test requirements	4080

A.7.8.3.2	UE Rx-Tx time difference measurement with reduced number of samples in RRC_INACTIVE, FR2 SA	4080

A.7.8.3.2.1	Test purpose and environment	4080

A.7.8.3.2.2	Test requirements	4082

A.7.8.4	PRS-RSRPP measurements	4083

A.7.8.4.1	PRS-RSRPP reporting delay test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4083

A.7.8.4.1.1	Test Purpose and Environment	4083

A.7.8.4.1.2	Test Requirements	4085

A.7.8.4.2	PRS-RSRPP reporting delay test with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4086

A.7.8.4.2.1	Test Purpose and Environment	4086

A.7.8.4.2.2	Test Requirements	4088

A.7.9	Measurement performance requirements in RRC_INACTIVE	4089

A.7.9.1	RSTD measurements	4089

A.7.9.1.1	RSTD measurement accuracy test case for single positioning frequency layer in FR2 in RRC_INACTIVE state	4089

A.7.9.1.1.1	Test purpose and Environment	4089

A.7.9.1.1.2	Test Requirements	4091

A.7.9.1.2	RSTD measurement accuracy test case with reduced number of samples for single positioning frequency layer in FR2 in RRC_INACTIVE state	4091

A.7.9.1.2.1	Test purpose and Environment	4091

A.7.9.1.2.2	Test Requirements	4093

A.7.9.2	PRS-RSRP measurements	4093

A.7.9.2.1	SA measurement accuracy with PRS in FR2 in RRC_INACTIVE	4093

A.7.9.2.1.1	Test Purpose and Environment	4093

A.7.9.2.1.2	Test parameters	4093

A.7.9.2.1.3	Test Requirements	4095

A.7.9.2.2	PRS-RSRP measurements with reduced number of sample in RRC_INACTIVE	4096

A.7.9.2.2.1	Test Purpose and Environment	4096

A.7.9.2.2.2	Test parameters	4096

A.7.9.2.2.3	Test Requirements	4098

A.7.9.3	UE Rx-Tx time difference measurements	4099

A.7.9.3.1	UE Rx-Tx time difference measurements in RRC_INACTIVE	4099

A.7.9.3.1.1	Test purpose and environment	4099

A.7.9.3.1.2	Test parameters	4099

A.7.9.3.1.3	Test requirements	4102

A.7.9.3.2	UE Rx-Tx time difference measurement accuracy with reduced number of samples in FR2 SA	4102

A.7.9.3.2.1	Test purpose and environment	4102

A.7.9.3.2.2	Test parameters	4102

A.7.9.3.2.3	Test requirements	4105

A.7.9.4	PRS-RSRPP measurements	4105

A.7.9.4.1	SA measurement accuracy in FR2 in RRC INACTIVE	4105

A.7.9.4.1.1	Test Purpose and Environment	4105

A.7.9.4.1.2	Test parameters	4105

A.7.9.4.1.3	Test Requirements	4107

A.7.9.4.2	SA measurement accuracy with reduced PRS samples in FR2 in RRC INACTIVE	4108

A.7.9.4.2.1	Test Purpose and Environment	4108

A.7.9.4.2.2	Test parameters	4108

A.7.9.4.2.3	Test Requirements	4110

A.8	E-UTRA standalone tests for NR RRM	4024

A.8.1	Void	4024

A.8.2	RRC_IDLE state mobility	4024

A.8.2.1	Inter-RAT NR Cell re-selection	4024

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	4024

A.8.2.1.1.1	Test Purpose and Environment	4024

A.8.2.1.1.2	Test Requirements	4029

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	4030

A.8.2.1.2.1	Test Purpose and Environment	4030

A.8.2.1.2.2	Test Requirements	4034

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	4035

A.8.2.2.1	E-UTRA – NR Early Measurement Reporting for NR in FR1	4035

A.8.2.2.1.1	Test Purpose and Environment	4035

A.8.2.2.1.2	Test Requirements	4039

A.8.2.2.2	E-UTRA – NR Early Measurement Reporting for NR in FR2	4039

A.8.2.2.2.1	Test Purpose and Environment	4039

A.8.2.2.2.2	Test Requirements	4042

A.8.3	RRC_CONNECTED state mobility	4043

A.8.3.1	Handover	4043

A.8.3.1.1	E-UTRAN - NR handover in FR1	4043

A.8.3.1.1.1	Test Purpose and Environment	4043

A.8.3.1.1.2	Test Requirements	4048

A.8.4	Measurement procedure	4048

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	4048

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	4048

A.8.4.1.1.1	Test Purpose and Environment	4048

A.8.4.1.1.2	Test Requirements	4050

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	4051

A.8.4.1.2.1	Test Purpose and Environment	4051

A.8.4.1.2.2	Test Requirements	4052

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	4052

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4052

A.8.4.2.1.1	Test Purpose and Environment	4052

A.8.4.2.1.2	Test Requirements	4057

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4057

A.8.4.2.2.1	Test Purpose and Environment	4057

A.8.4.2.2.2	Test Requirements	4061

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4061

A.8.4.2.3.1	Test Purpose and Environment	4061

A.8.4.2.3.2	Test Requirements	4065

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4065

A.8.4.2.4.1	Test Purpose and Environment	4065

A.8.4.2.4.2	Test Requirements	4069

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4069

A.8.4.2.5.1	Test Purpose and Environment	4069

A.8.4.2.5.2	Test Requirements	4071

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4072

A.8.4.2.6.1	Test Purpose and Environment	4072

A.8.4.2.6.2	Test Requirements	4074

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4075

A.8.4.2.7.1	Test Purpose and Environment	4075

A.8.4.2.7.2	Test Requirements	4077

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4078

A.8.4.2.8.1	Test Purpose and Environment	4078

A.8.4.2.8.2	Test Requirements	4080

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	4081

A.8.4.2.9.1	Test Purpose and Environment	4081

A.8.4.2.9.2	Test Requirements	4085

A.8.4.3	E-UTRAN - NR Inter-RAT event-triggered without measurement gaps	4085

A.8.4.3.1	NR Inter-RAT event triggered reporting tests for FR2 without MG nor DRX	4085

A.8.4.3.1.1	Test Purpose and Environment	4085

A.8.4.3.1.2	Test Requirements	4087

A.8.4.3.2	NR Inter-RAT event triggered reporting tests for FR1 without gaps when DRX is not used	4088

A.8.4.3.2.1	Test Purpose and Environment	4088

A.8.4.3.2.2	Test Requirements	4091

A.8.5	Measurement performance	4092

A.8.5.1	SFTD accuracy	4092

A.8.5.1.1	SFTD accuracy	4092

A.8.5.1.1.1	Test Purpose	4092

A.8.5.1.1.2	Test Environment	4092

A.8.5.1.1.3	Test Requirements	4098

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	4098

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4098

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4103

A.8.5.2.1.2.1	Test Purpose and Environment	4103

A.8.5.2.1.2.2	Test Parameters	4103

A.8.5.2.1.2.3	Test Requirements	4105

A.8.5.2.2	SS-RSRQ	4105

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4105

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4110

A.8.5.2.2.2.1	Test Purpose and Environment	4110

A.8.5.2.2.2.2	Test Parameters	4110

A.8.5.2.2.2.3	Test Requirements	4112

A.8.5.2.3	SS-SINR	4112

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	4112

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	4116

A.8.5.2.3.2.1	Test Purpose and Environment	4116

A.8.5.2.3.2.2	Test Parameters	4116

A.8.5.2.3.2.3	Test Requirements	4118

A.9	V2X Tests	4119

A.9.1	V2X Tests in FR1	4119

A.9.1.1	Test for V2X UE Transmit Timing	4119

A.9.1.1.1 Test for GNSS as Synchronization Reference Source	4119

A.9.1.1.1.1	Test Purpose and Environment	4119

A.9.1.1.1.2	Test requirements	4119

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	4119

A.9.1.1.2.1	Test Purpose and Environment	4119

A.9.1.1.2.2	Test requirements	4120

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	4120

A.9.1.1.3.1	Test Purpose and Environment	4120

A.9.1.1.3.2	Test requirements	4123

A.9.1.2	Test for Initiation/Cease of S-SSB Transmission with V2X Sidelink Communication	4123

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	4123

A.9.1.2.1.1	Test Purpose and Environment	4123

A.9.1.2.1.2	Test Requirements	4126

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	4126

A.9.1.2.2.1	Test Purpose and Environment	4126

A.9.1.2.2.2	Test Requirements	4128

A.9.1.2.3	Test for SyncRef UE as synchronization reference source when SL-DRX is used	4128

A.9.1.2.3.1	Test Purpose and Environment	4128

A.9.1.2.3.2	Test Requirements	4130

A.9.1.2.4	Test for SyncRef UE as synchronization reference source with CCA	4130

A.9.1.2.4.1	Test Purpose and Environment	4130

A.9.1.2.4.2	Test Requirements	4132

A.9.1.3	 Test for V2X Synchronization Reference Selection/Reselection	4132

A.9.1.3.1	 Test for GNSS configured as the highest priority	4132

A.9.1.3.1.1	Test Purpose and Environment	4132

A.9.1.3.1.2	Test Requirements	4134

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	4135

A.9.1.3.2.1	Test Purpose and Environment	4135

A.9.1.3.2.2	Test Requirements	4137

A.9.1.3.3	Test for GNSS configured as the highest priority under SL-DRX	4138

A.9.1.3.3.1	Test Purpose and Environment	4138

A.9.1.3.3.2	Test Requirements	4140

A.9.1.3.4	Test for FR1 NR Cell configured as the highest priority under SL-DRX	4141

A.9.1.3.4.1	Test Purpose and Environment	4141

A.9.1.3.4.2	Test Requirements	4143

A.9.1.4	Test for L1 SL-RSRP Measurement	4144

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	4144

A.9.1.4.1.1	Test Purpose and Environment	4144

A.9.1.4.1.2	Test Requirements	4147

A.9.1.4.2	Test for V2X UE Resource Pre-emption	4147

A.9.1.4.2.1	Test Purpose and Environment	4147

A.9.1.4.2.2	Test Requirements	4151

A.9.1.4.3	 Test for V2X UE Resource Re-evaluation	4151

A.9.1.4.3.1	Test Purpose and Environment	4151

A.9.1.4.3.2	Test Requirements	4158

A.9.1.4.4	Test for V2X UE Autonomous Resource Selection/Reselection with Periodic Sensing	4158

A.9.1.4.4.1	Test Purpose and Environment	4158

A.9.1.4.4.2	Test Requirements	4162

A.9.1.4.5	Test for V2X UE Autonomous Resource Selection/Reselection with Contiguous Sensing	4162

A.9.1.4.5.1	Test Purpose and Environment	4162

A.9.1.4.5.2	Test Requirements	4165

A.9.1.4.6	Test for V2X UE Autonomous Resource Selection/Reselection in SL-DRX	4165

A.9.1.4.6.1	Test Purpose and Environment	4165

A.9.1.4.6.2	Test Requirements	4169

A.9.1.5	Test for Congestion Control Measurement	4169

A.9.1.5.1	Test Purpose and Environment	4169

A.9.1.5.2	Test Requirements	4175

A.9.1.6	Test for Interruption	4175

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	4175

A.9.1.6.1.1	Test Purpose and Environment	4175

A.9.1.6.1.2	Test Requirements	4178

A.9.1.6.2	Test for interruption to WAN at transitions between active and non-active during SL-DRX in asynchronous case	4178

A.9.1.6.2.1	Test Purpose and Environment	4178

A.9.1.6.2.2	Test Requirements	4182

A.9.1.6.3	Test for Interruption at NR Sidelink Diccovery Configuration	4182

A.9.1.6.3.1	Test Purpose and Environment	4182

A.9.1.6.3.2	Test Requirements	4185

A.9.1.7	Selection / Reselection of relay UE	4185

A.9.1.7.1	Test Purpose and Environment	4185

A.9.1.7.2	Test Requirements	4191

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	4185

A.10.1	RRC_CONNECTED state mobility	4185

A.10.1.1	RRC connection mobility control	4185

A.10.1.1.1	Random Access	4185

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	4185

A.10.1.1.1.1.1	Test Purpose and Environment	4185

A.10.1.1.1.1.2	Test Requirements	4188

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	4188

A.10.1.1.1.1.2.2	Random Access Response Reception	4188

A.10.1.1.1.1.2.3	No Random Access Response Reception	4189

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	4189

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	4189

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	4190

A.10.1.1.1.2.1	Test Purpose and Environment	4190

A.10.1.1.1.2.2	Test Requirements	4193

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	4193

A.10.1.1.1.2.2.2	Random Access Response Reception	4194

A.10.1.1.1.2.2.3	No Random Access Response Reception	4194

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	4194

A.10.1.1.1.3.1	Test Purpose and Environment	4194

A.10.1.1.1.3.2	Test Requirements	4198

A.10.1.1.1.3.2.1	MsgA Transmission	4198

A.10.1.1.1.3.2.2	MsgB Reception	4198

A.10.1.1.1.3.2.3	No MsgB Reception	4199

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	4199

A.10.1.1.1.4.1	Test Purpose and Environment	4199

A.10.1.1.1.4.2	Test Requirements	4203

A.10.1.1.1.4.2.1	MsgA Transmission	4203

A.10.1.1.1.4.2.2	MsgB Reception	4204

A.10.1.1.1.4.2.3	No MsgB Reception	4204

A.10.1.2	Handover with PSCell from EN-DC to EN-DC with known target PSCell using CCA	4204

A.10.1.2.1	Test Purpose and Environment	4204

A.10.1.2.2	Test Requirements	4210

A.10.2	Timing	4211

A.10.2.1	UE transmit timing	4211

A.10.2.1.1	UE Transmit Timing Test with PSCell under DL CCA	4211

A.10.2.1.1.1	Test Purpose and environment	4211

A.10.2.1.1.2	Test requirements	4214

A.10.2.2	UE timing advance	4215

A.10.2.2.1	UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	4215

A.10.2.2.1.1	Test Purpose and Environment	4215

A.10.2.2.1.2	Test Parameters	4215

A.10.2.2.1.3	Test Requirements	4219

A.10.3	Signalling characteristics	4219

A.10.3.1	Radio link monitoring	4219

A.10.3.1.1	Introduction	4219

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4220

A.10.3.1.2.1	Test purpose and environment	4220

A.10.3.1.2.2	Test requirements	4225

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	4225

A.10.3.1.3.1	Test purpose and environment	4225

A.10.3.1.3.2	Test requirements	4231

A.10.3.1.4	Void	4231

A.10.3.1.4.1	Void	4231

A.10.3.1.4.2	Void	4231

A.10.3.1.5	Void	4231

A.10.3.1.5.1	Void	4231

A.10.3.1.5.2	Void	4231

A.10.3.2	Void	4231

A.10.3.3	SCell activation and deactivation delay	4232

A.10.3.3.1	SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	4232

A.10.3.3.1.1	Test Purpose and Environment	4232

A.10.3.3.1.2	Test Requirements	4236

A.10.3.3.2 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	4236

A.10.3.3.2.1	Test Purpose and Environment	4236

A.10.3.3.2.2	Test Requirements	4237

A.10.3.3.3	SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	4237

A.10.3.3.3.1	Test Purpose and Environment	4237

A.10.3.3.3.2	Test Requirements	4238

A.10.3.4	Beam failure detection and link recovery procedures	4238

A.10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	4238

A.10.3.4.1.1	Test Purpose and Environment	4238

A.10.3.4.1.2	Test Requirements	4243

A.10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	4243

A.10.3.4.2.1	Test Purpose and Environment	4243

A.10.3.4.2.2	Test Requirements	4249

A.10.3.5	Active BWP switching	4249

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	4249

A.10.3.5.1.2	Test Requirements	4254

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	4255

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4255

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	4258

A.10.3.5.3	RRC-based Active BWP Switch	4262

A.10.3.5.3.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	4262

A.10.3.6	PSCell addition and release delay	4266

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	4266

A.10.3.6.1.1	Test purpose and environment	4266

A.10.3.6.1.2	Test Requirements	4271

A.10.3.7	Void	4272

A.10.4	Measurement procedure	4272

A.10.4.1	Intra-frequency measurements	4272

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	4272

A.10.4.1.1.1	Test purpose and environment	4272

A.10.4.1.1.2	Test parameters	4272

A.10.4.1.1.3	Test Requirements	4276

A.10.4.1.2	Void	4276

A.10.4.1.3	Void	4276

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	4277

A.10.4.1.4.1	Test purpose and environment	4277

A.10.4.1.4.2	Test parameters	4277

A.10.4.1.4.3	Test Requirements	4281

A.10.4.1.5	Void	4282

A.10.4.1.6	Void	4282

A.10.4.1.7	Void	4282

A.10.4.1.8	Void	4282

A.10.4.1.9	Void	4282

A.10.4.1.10	Void	4282

A.10.4.1.11	Void	4282

A.10.4.1.12	Void	4282

A.10.4.2	Inter-frequency measurements	4282

A.10.4.2.1	Void	4282

A.10.4.2.2	Void	4282

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	4282

A.10.4.2.3.1	Test Purpose and Environment	4282

A.10.4.2.3.2	Test Requirements	4286

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	4287

A.10.4.2.4.1	Test Purpose and Environment	4287

A.10.4.2.4.2	Test Requirements	4291

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	4292

A.10.4.2.5.1	Test Purpose and Environment	4292

A.10.4.2.5.2	Test Requirements	4296

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	4297

A.10.4.2.6.1	Test Purpose and Environment	4297

A.10.4.2.6.2	Test Requirements	4301

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	4302

A.10.4.2.7.1	Test Purpose and Environment	4302

A.10.4.2.7.2	Test Requirements	4308

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	4308

A.10.4.2.8.1	Test Purpose and Environment	4308

A.10.4.2.8.2	Test Requirements	4314

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	4315

A.10.4.2.9.1	Test Purpose and Environment	4315

A.10.4.2.9.2	Test Requirements	4320

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	4320

A.10.4.2.10.1	Test Purpose and Environment	4320

A.10.4.2.10.2	Test Requirements	4326

A.10.4.3	L1-RSRP measurements for beam reporting	4327

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	4327

A.10.4.3.1.1	Test Purpose and Environment	4327

A.10.4.3.1.2	Test parameters	4327

A.10.4.3.1.3	Test Requirements	4329

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	4330

A.10.4.3.2.1	Test Purpose and Environment	4330

A.10.4.3.2.2	Test parameters	4330

A.10.4.3.2.3	Test Requirements	4332

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4333

A.10.4.3.3.1	Test Purpose and Environment	4333

A.10.4.3.3.2	Test parameters	4333

A.10.4.3.3.3	Test Requirements	4336

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	4337

A.10.4.3.4.1	Test Purpose and Environment	4337

A.10.4.3.4.2	Test parameters	4337

A.10.4.3.4.3	Test Requirements	4340

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4341

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4341

A.10.4.4.1.1	Test Purpose and Environment	4341

A.10.4.4.1.2	Test Requirements	4345

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4346

A.10.4.4.2.1	Test Purpose and Environment	4346

A.10.4.4.2.2	Test Requirements	4351

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4352

A.10.4.4.3.1	Test Purpose and Environment	4352

A.10.4.4.3.2	Test Requirements	4356

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4357

A.10.4.4.4.1	Test Purpose and Environment	4357

A.10.4.4.4.2	Test Requirements	4362

A.10.5	Measurement performance	4363

A.10.5.1	SS-RSRP	4363

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	4363

A.10.5.1.1.1	Test Purpose and Environment	4363

A.10.5.1.1.2	Test parameters	4363

A.10.5.1.1.3	Test Requirements	4366

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4366

A.10.5.1.2.1	Test Purpose and Environment	4366

A.10.5.1.2.2	Test parameters	4366

A.10.5.1.2.3	Test Requirements	4369

A.10.5.2	SS-RSRQ	4369

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4369

A.10.5.2.1.1	Test Purpose and Environment	4369

A.10.5.2.1.2	Test Parameters	4369

A.10.5.2.1.3	Test Requirements	4372

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	4372

A.10.5.2.2.1	Test Purpose and Environment	4372

A.10.5.2.2.2	Test Parameters	4372

A.10.5.2.2.3	Test Requirements	4375

A.10.5.3	SS-SINR	4375

A.10.5.3.1	Intra-frequency measurement accuracy on PSCC	4375

A.10.5.3.1.1	Test Purpose and Environment	4375

A.10.5.3.1.2	Test Parameters	4375

A.10.5.3.1.3	Test Requirements	4378

A.10.5.3.2	Inter-frequency measurement accuracy on PSCC	4378

A.10.5.3.2.1	Test Purpose and Environment	4378

A.10.5.3.2.2	Test Parameters	4378

A.10.5.3.2.3	Test Requirements	4381

A.10.5.3.3	Intra-frequency measurement accuracy on SCC	4381

A.10.5.3.3.1	Test Purpose and Environment	4381

A.10.5.3.3.2	Test Parameters	4381

A.10.5.3.3.3	Test Requirements	4384

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	4384

A.10.5.4.1	SSB based L1-RSRP measurement	4384

A.10.5.4.1.1	Test Purpose and Environment	4384

A.10.5.4.1.2	Test parameters	4385

A.10.5.4.1.3	Test Requirements	4387

A.10.5.5	RSSI	4387

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	4387

A.10.5.5.1.1	Test Purpose and Environment	4387

A.10.5.5.1.2	Test parameters	4387

A.10.5.5.1.3	Test Requirements	4390

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	4390

A.10.5.5.2.1	Test Purpose and Environment	4390

A.10.5.5.2.2	Test parameters	4390

A.10.5.5.2.3	Test Requirements	4393

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4393

A.10.5.5.3.1	Test Purpose and Environment	4393

A.10.5.5.3.2	Test parameters	4393

A.10.5.5.3.3	Test Requirements	4397

A.10.5.6	Channel occupancy	4397

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	4397

A.10.5.6.1.1	Test Purpose and Environment	4397

A.10.5.6.1.2	Test parameters	4397

A.10.5.6.1.3	Test Requirements	4401

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	4401

A.10.5.6.2.1	Test Purpose and Environment	4401

A.10.5.6.2.2	Test parameters	4401

A.10.5.6.2.3	Test Requirements	4404

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4404

A.10.5.6.3.1	Test Purpose and Environment	4404

A.10.5.6.3.2	Test parameters	4404

A.10.5.6.3.3	Test Requirements	4408

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	4420

A.11.1	RRC_IDLE state mobility	4420

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	4420

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	4420

A.11.1.1.1.1	Test Purpose and Environment	4420

A.11.1.1.1.2	Test Parameters	4420

A.11.1.1.1.3	Test Requirements	4424

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	4425

A.11.1.1.2.1	Test Purpose and Environment	4425

A.11.1.1.2.2	Test Parameters	4425

A.11.1.1.2.3	Test Requirements	4429

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	4429

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	4429

A.11.1.2.1.1	Test Purpose and Environment	4429

A.11.1.2.1.2	Test Parameters	4430

A.11.1.2.1.3	Test Requirements	4436

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	4437

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	4437

A.11.1.3.1.1	Test Purpose and Environment	4437

A.11.1.3.1.2	Test Parameters	4437

A.11.1.3.1.3	Test Requirements	4443

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	4444

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	4444

A.11.1.4.1.1	Test Purpose and Environment	4444

A.11.1.4.1.2	Test Parameters	4444

A.11.1.4.1.3	Test Requirements	4447

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	4448

A.11.1.4.2.1	Test Purpose and Environment	4448

A.11.1.4.2.2	Test Requirements	4451

A.11.2	RRC_CONNECTED state mobility	4452

A.11.2.1	Handover	4452

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	4452

A.11.2.1.1.1	Test Purpose and Environment	4452

A.11.2.1.1.2	Test Parameters	4452

A.11.2.1.1.3 Test Requirements	4455

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4455

A.11.2.1.2.1	Test Purpose and Environment	4455

A.11.2.1.2.2	Test Parameters	4455

A.11.2.1.2.3	Test Requirements	4458

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	4458

A.11.2.1.3.1	Test Purpose and Environment	4458

A.11.2.1.3.2	Test Parameters	4458

A.11.2.1.3.3 Test Requirements	4461

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	4461

A.11.2.1.4.1	Test Purpose and Environment	4461

A.11.2.1.4.2	Test Parameters	4461

A.11.2.1.4.3 Test Requirements	4465

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	4466

A.11.2.1.5.1	Test Purpose and Environment	4466

A.11.2.1.5.2	Test Parameters	4466

A.11.2.1.5.3 Test Requirements	4469

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	4470

A.11.2.1.6.1	Test Purpose and Environment	4470

A.11.2.1.6.2	Test Parameters	4470

A.11.2.1.6.3 Test Requirements	4473

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	4474

A.11.2.1.7.1	Test Purpose and Environment	4474

A.11.2.1.7.2	Test Requirements	4480

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	4480

A.11.2.1.8.1	Test Purpose and Environment	4480

A.11.2.1.8.2	Test Requirements	4485

A.11.2.1.9	Handover with PSCell from NR SA to EN-DC with known target PSCell using CCA	4485

A.11.2.1.9.1	Test Purpose and Environment	4485

A.11.2.1.9.2	Test Requirements	4494

A.11.2.2	RRC connection mobility control	4495

A.11.2.2.1	RRC re-establishment	4495

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	4495

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	4498

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	4505

A.11.2.2.2	Random Access	4510

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	4510

A.11.2.2.2.1.1	Test Purpose and Environment	4510

A.11.2.2.2.1.2	Test Requirements	4513

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	4513

A.11.2.2.2.1.2.2	Random Access Response Reception	4513

A.11.2.2.2.1.2.3	No Random Access Response Reception	4514

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	4514

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	4514

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	4515

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	4515

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	4515

A.11.2.2.2.2.1	Test Purpose and Environment	4515

A.11.2.2.2.2.2	Test Requirements	4518

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	4518

A.11.2.2.2.2.2.2	Random Access Response Reception	4519

A.11.2.2.2.2.2.3	No Random Access Response Reception	4519

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	4519

A.11.2.2.2.3.1	Test Purpose and Environment	4519

A.11.2.2.2.3.2	Test Requirements	4523

A.11.2.2.2.3.2.1	MsgA Transmission	4523

A.11.2.2.2.3.2.2	MsgB Reception	4523

A.11.2.2.2.3.2.3	No MsgB Reception	4524

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	4524

A.11.2.2.2.4.1	Test Purpose and Environment	4524

A.11.2.2.2.4.2	Test Requirements	4528

A.11.2.2.2.4.2.1	MsgA Transmission	4528

A.11.2.2.2.4.2.2	MsgB Reception	4529

A.11.2.2.2.4.2.3	No MsgB Reception	4529

A.11.2.2.3	RRC connection release with redirection	4530

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	4530

A.11.2.2.3.2	Redirection from NR FR1 carrier without CCA to NR FR1 carrier with CCA	4534

A.11.3	Timing	4539

A.11.3.1	UE transmit timing	4539

A.11.3.1.1	UE Transmit Timing Test with PCell under DL CCA	4539

A.11.3.1.1.1	Test Purpose and environment	4539

A.11.3.1.1.2	Test requirements	4542

A.11.3.2	UE timing advance	4543

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	4543

A.11.3.2.1.1	Test Purpose and Environment	4543

A.11.3.2.1.2	Test Parameters	4543

A.11.3.2.1.3	Test Requirements	4547

A.11.4	Signalling characteristics	4547

A.11.4.1	Radio link monitoring	4547

A.11.4.1.1	Introduction	4547

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4548

A.11.4.1.2.1	Test purpose and environment	4548

A.11.4.1.2.2	Test requirements	4552

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	4552

A.11.4.1.3.1	Test purpose and environment	4552

A.11.4.1.3.2	Test requirements	4558

A.11.4.1.4	Void	4558

A.11.4.1.4.1	Void	4558

A.11.4.1.4.2	Void	4558

A.11.4.1.5	Void	4558

A.11.4.1.5.1	Void	4558

A.11.4.1.5.2	Void	4558

A.11.4.2	Void	4558

A.11.4.3	SCell activation and deactivation delay	4559

A.11.4.3.1	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 160 ms SCell measurement cycle	4559

A.11.4.3.1.1	Test Purpose and Environment	4559

A.11.4.3.1.2	Test Requirements	4563

A.11.4.3.2	SCell Activation and Deactivation of known SCell with PCell and SCell under CCA, 640 ms SCell measurement cycle	4563

A.11.4.3.2.1	Test Purpose and Environment	4563

A.11.4.3.2.2	Test Requirements	4564

A.11.4.3.3	SCell Activation and Deactivation of unknown SCell with PCell and SCell under CCA	4564

A.11.4.3.3.1	Test Purpose and Environment	4564

A.11.4.3.3.2	Test Requirements	4565

A.11.4.4	Beam failure detection and link recovery procedures	4565

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	4565

A.11.4.4.1.1	Test Purpose and Environment	4565

A.11.4.4.1.2	Test Requirements	4570

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	4570

A.11.4.4.2.1	Test Purpose and Environment	4570

A.11.4.4.2.2	Test Requirements	4576

A.11.4.5	Active BWP switching	4576

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	4576

A.11.4.5.1.1	Test Purpose and Environment	4576

A.11.4.5.1.2	Test Requirements	4580

A.11.4.5.2	DCI-based and Timer-based Active BWP Switch	4581

A.11.4.5.2.1	NR FR1- NR FR1 DL active BWP switch of PCell with non-DRX in SA	4581

A.11.4.5.2.2	NR FR1 DL active BWP switch with non-DRX in SA	4584

A.11.4.5.3	RRC-based Active BWP Switch	4587

A.11.4.5.3.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	4587

A.11.4.6	Void	4590

A.11.5	Measurement procedure	4590

A.11.5.1	Intra-frequency measurements	4590

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	4590

A.11.5.1.1.1	Test purpose and environment	4590

A.11.5.1.1.2	Test parameters	4590

A.11.5.1.1.3	Test Requirements	4594

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	4594

A.11.5.1.2.1	Test purpose and environment	4594

A.11.5.1.2.2	Test parameters	4594

A.11.5.1.2.3	Test Requirements	4598

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	4598

A.11.5.1.3.1	Test purpose and environment	4598

A.11.5.1.3.2	Test parameters	4598

A.11.5.1.3.3	Test Requirements	4598

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	4599

A.11.5.1.4.1	Test purpose and environment	4599

A.11.5.1.4.2	Test parameters	4599

A.11.5.1.4.3	Test Requirements	4599

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	4599

A.11.5.1.5.1	Test purpose and environment	4599

A.11.5.1.5.2	Test parameters	4599

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	4600

A.11.5.1.6.1	Test purpose and environment	4600

A.11.5.1.6.2	Test parameters	4600

A.11.5.1.6.3	Test Requirements	4600

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4600

A.11.5.1.7.1	Test purpose and environment	4600

A.11.5.1.7.2	Test parameters	4600

A.11.5.1.7.3	Test Requirements	4600

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4601

A.11.5.1.8.1	Test purpose and environment	4601

A.11.5.1.8.2	Test parameters	4601

A.11.5.1.8.3	Test Requirements	4601

A.11.5.1.9	Void	4601

A.11.5.1.10	Void	4601

A.11.5.1.11	Void	4601

A.11.5.1.12	Void	4601

A.11.5.2	Inter-frequency measurements	4601

A.11.5.2.1	Void	4601

A.11.5.2.2	Void	4601

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4601

A.11.5.2.3.1	Test Purpose and Environment	4601

A.11.5.2.3.2	Test Requirements	4605

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4606

A.11.5.2.4.1	Test Purpose and Environment	4606

A.11.5.2.4.2	Test Requirements	4611

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4611

A.11.5.2.5.1	Test Purpose and Environment	4611

A.11.5.2.5.2	Test Requirements	4615

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4616

A.11.5.2.6.1	Test Purpose and Environment	4616

A.11.5.2.6.2	Test Requirements	4621

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4621

A.11.5.2.7.1	Test Purpose and Environment	4621

A.11.5.2.7.2	Test Requirements	4625

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4626

A.11.5.2.8.1	Test Purpose and Environment	4626

A.11.5.2.8.2	Test Requirements	4631

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4631

A.11.5.2.9.1	Test Purpose and Environment	4631

A.11.5.2.9.2	Test Requirements	4636

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4637

A.11.5.2.10.1	Test Purpose and Environment	4637

A.11.5.2.10.2	Test Requirements	4642

A.11.5.3	Inter-RAT E-UTRAN measurements	4643

A.11.5.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	4643

A.11.5.3.1.1	Test Purpose and Environment	4643

A.11.5.3.1.2	Test Requirements	4648

A.11.5.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	4648

A.11.5.3.2.1	Test Purpose and Environment	4648

A.11.5.3.2.2	Test Requirements	4654

A.11.5.4	L1-RSRP measurements for beam reporting	4654

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	4654

A.11.5.4.1.1	Test Purpose and Environment	4654

A.11.5.4.1.2	Test parameters	4654

A.11.5.4.1.3	Test Requirements	4656

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	4657

A.11.5.4.2.1	Test Purpose and Environment	4657

A.11.5.4.2.2	Test parameters	4657

A.11.5.4.2.3	Test Requirements	4659

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	4660

A.11.5.4.3.1	Test Purpose and Environment	4660

A.11.5.4.3.2	Test parameters	4660

A.11.5.4.3.3	Test Requirements	4663

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	4664

A.11.5.4.4.1	Test Purpose and Environment	4664

A.11.5.4.4.2	Test parameters	4664

A.11.5.4.4.3	Test Requirements	4667

A.11.6	Measurement performance	4668

A.11.6.1	SS-RSRP	4668

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4668

A.11.6.1.1.1	Test Purpose and Environment	4668

A.11.6.1.1.2	Test parameters	4668

A.11.6.1.1.3	Test Requirements	4670

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	4670

A.11.6.1.2.1	Test Purpose and Environment	4670

A.11.6.1.2.2	Test parameters	4670

A.11.6.1.2.3	Test Requirements	4672

A.11.6.2	SS-RSRQ	4672

A.11.6.2.1	Intra-frequency measurement accuracy	4672

A.11.6.2.1.1	Test Purpose and Environment	4672

A.11.6.2.1.2	Test Parameters	4672

A.11.6.2.1.3	Test Requirements	4675

A.11.6.2.2	Inter-frequency measurement accuracy	4675

A.11.6.2.2.1	Test Purpose and Environment	4675

A.11.6.2.2.2	Test Parameters	4675

A.11.6.2.2.3	Test Requirements	4678

A.11.6.2.3	Intra-frequency measurement accuracy on SCC	4678

A.11.6.2.3.1	Test Purpose and Environment	4678

A.11.6.2.3.2	Test Parameters	4678

A.11.6.2.3.3	Test Requirements	4681

A.11.6.2.4	Inter-frequency measurement accuracy	4681

A.11.6.2.4.1	Test Purpose and Environment	4681

A.11.6.2.4.2	Test Parameters	4681

A.11.6.2.4.3	Test Requirements	4689

A.11.6.3	SS-SINR	4689

A.11.6.3.1	Intra-frequency measurement accuracy	4689

A.11.6.3.1.1	Test Purpose and Environment	4689

A.11.6.3.1.2	Test Parameters	4689

A.11.6.3.1.3	Test Requirements	4693

A.11.6.3.2	Inter-frequency measurement accuracy	4693

A.11.6.3.2.1	Test Purpose and Environment	4693

A.11.6.3.2.2	Test Parameters	4693

A.11.6.3.2.3	Test Requirements	4696

A.11.6.3.3	Intra-frequency measurement accuracy on SCC	4696

A.11.6.3.3.1	Test Purpose and Environment	4696

A.11.6.3.3.2	Test Parameters	4696

A.11.6.3.3.3	Test Requirements	4699

A.11.6.3.4	Inter-frequency measurement accuracy	4699

A.11.6.3.4.1	Test Purpose and Environment	4699

A.11.6.3.4.2	Test Parameters	4699

A.11.6.3.4.3	Test Requirements	4707

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	4708

A.11.6.4.1	SSB based L1-RSRP measurement	4708

A.11.6.4.1.1	Test Purpose and Environment	4708

A.11.6.4.1.2	Test parameters	4708

A.11.6.4.1.3	Test Requirements	4711

A.11.6.5	RSSI	4711

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	4711

A.11.6.5.1.1	Test Purpose and Environment	4711

A.11.6.5.1.2	Test parameters	4711

A.11.6.5.1.3	Test Requirements	4714

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	4714

A.11.6.5.2.1	Test Purpose and Environment	4714

A.11.6.5.2.2	Test parameters	4714

A.11.6.5.2.3	Test Requirements	4717

A.11.6.5.3	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4717

A.11.6.5.3.1	Test Purpose and Environment	4717

A.11.6.5.3.2	Test parameters	4717

A.11.6.5.3.3	Test Requirements	4720

A.11.6.6	Channel occupancy	4720

A.11.6.6.1	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	4720

A.11.6.6.1.1	Test Purpose and Environment	4720

A.11.6.6.1.2	Test parameters	4720

A.11.6.6.1.3	Test Requirements	4723

A.11.6.6.2	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4723

A.11.6.6.2.1	Test Purpose and Environment	4723

A.11.6.6.2.2	Test parameters	4723

A.11.6.6.2.3	Test Requirements	4726

A.11.6.6.3	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4726

A.11.6.6.3.1	Test Purpose and Environment	4726

A.11.6.6.3.2	Test parameters	4726

A.11.6.6.3.3	Test Requirements	4729

A.11.6.7	E-UTRAN RSRP	4729

A.11.6.8	E-UTRAN RSRQ	4729

A.11.6.9	E-UTRAN SINR	4729

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	4729

A.12.1	RRC_IDLE state mobility	4729

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	4729

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	4729

A.12.1.1.1.1	Test Purpose and Environment	4729

A.12.1.1.1.2	Test Requirements	4734

A.12.2	RRC_CONNECTED state mobility	4735

A.12.2.1	Handover	4735

A.12.2.1.1	E-UTRAN - NR with CCA handover	4735

A.12.2.1.1.1	Test Purpose and Environment	4735

A.12.2.1.1.2	Test Requirements	4740

A.12.3	Void	4741

A.12.3.1	Void	4741

A.12.4	Measurement procedure	4741

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	4741

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	4741

A.12.4.1.1.1	Test Purpose and Environment	4741

A.12.4.1.1.2	Test Requirements	4745

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	4745

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4745

A.12.4.2.1.1	Test Purpose and Environment	4745

A.12.4.2.1.2	Test Requirements	4749

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4750

A.12.4.2.2.1	Test Purpose and Environment	4750

A.12.4.2.2.2	Test Requirements	4754

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4755

A.12.4.2.3.1	Test Purpose and Environment	4755

A.12.4.2.3.2	Test Requirements	4759

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4760

A.12.4.2.4.1	Test Purpose and Environment	4760

A.12.4.2.4.2	Test Requirements	4764

A.12.4.2.5	RSSI measurement reporting	4765

A.12.4.2.5.1	Test purpose and environment	4765

A.12.4.2.5.2	Test parameters	4765

A.12.4.2.5.3	Test Requirements	4768

A.12.4.2.6	Channel occupancy measurement reporting	4768

A.12.4.2.6.1	Test purpose and environment	4768

A.12.4.2.6.2	Test parameters	4768

A.12.4.2.6.3	Test Requirements	4772

A.12.5	Measurement performance	4772

A.12.5.1	E-UTRANNR SFTD	4772

A.12.5.1.1	Inter-RAT SFTD accuracy with NR target cell under CCA	4772

A.12.5.1.1.1	Test Purpose	4772

A.12.5.1.1.2	Test Environment	4772

A.12.5.1.1.3	Test Requirements	4778

A.12.5.2	E-UTRANNR SS-RSRP	4778

A.12.5.3	E-UTRANNR SS-RSRQ	4778

A.12.5.4	E-UTRANNR SS-SINR	4778

A.12.5.5	E-UTRANNR RSSI	4778

A.12.5.6	E-UTRANNR channel occupancy	4778

A.13	NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	4783

A.13.1	Void	4783

A.13.1.1	Void	4783

A.13.1.2	Void	4783

A.13.2	Signalling characteristics	4783

A.13.2.1	Void	4783

A.13.2.2	SCell activation and deactivation delay	4783

A.13.2.2.1	SCell Activation and Deactivation of known SCell under CCA, 160 ms SCell measurement cycle	4783

A.13.2.2.1.1	Test Purpose and Environment	4783

A.13.2.2.1.2	Test Requirements	4787

A.13.2.2.2 SCell Activation and Deactivation of known SCell under CCA, 640 ms SCell measurement cycle	4787

A.13.2.2.2.1	Test Purpose and Environment	4787

A.13.2.2.2.2	Test Requirements	4788

A.13.2.2.3	SCell Activation and Deactivation of unknown SCell under CCA	4788

A.13.2.2.3.1	Test Purpose and Environment	4788

A.13.2.2.3.2	Test Requirements	4789

A.13.2.3	Void	4789

A.13.3	Measurement procedure	4789

A.13.3.1	Intra-frequency measurements	4789

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	4789

A.13.3.1.1.1	Test purpose and environment	4789

A.13.3.1.1.2	Test parameters	4789

A.13.3.1.1.3	Test Requirements	4794

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	4794

A.13.3.1.2.1	Test purpose and environment	4794

A.13.3.1.2.2	Test parameters	4794

A.13.3.1.2.3	Test Requirements	4799

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	4799

A.13.3.1.3.1	Test purpose and environment	4799

A.13.3.1.3.2	Test parameters	4799

A.13.3.1.3.3	Test Requirements	4804

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	4804

A.13.3.1.4.1	Test purpose and environment	4804

A.13.3.1.4.2	Test parameters	4804

A.13.3.1.4.3	Test Requirements	4809

A.13.3.1.5	Void	4809

A.13.3.1.6	Void	4809

A.13.3.2	Inter-frequency measurements	4809

A.13.3.2.1	Void	4809

A.13.3.2.2	Void	4809

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	4809

A.13.3.2.3.1	Test Purpose and Environment	4809

A.13.3.2.3.2	Test Requirements	4814

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	4814

A.13.3.2.4.1	Test Purpose and Environment	4814

A.13.3.2.4.2	Test Requirements	4819

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	4820

A.13.3.2.5.1	Test Purpose and Environment	4820

A.13.3.2.5.2	Test Requirements	4825

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	4825

A.13.3.2.6.1	Test Purpose and Environment	4825

A.13.3.2.6.2	Test Requirements	4831

A.13.3.3	L1-RSRP measurements for beam reporting	4832

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	4832

A.13.3.3.1.1	Test Purpose and Environment	4832

A.13.3.3.1.2	Test parameters	4832

A.13.3.3.1.3	Test Requirements	4836

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	4836

A.13.3.3.2.1	Test Purpose and Environment	4836

A.13.3.3.2.2	Test parameters	4836

A.13.3.3.2.3	Test Requirements	4840

A.13.4	Measurement performance	4840

A.13.4.1	SS-RSRP	4840

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	4840

A.13.4.1.1.1	Test Purpose and Environment	4840

A.13.4.1.1.2	Test parameters	4840

A.13.4.1.1.3	Test Requirements	4842

A.13.4.2	SS-RSRQ	4842

A.13.4.2.1	Intra-frequency measurement accuracy on SCC	4842

A.13.4.2.1.1	Test Purpose and Environment	4842

A.13.4.2.1.2	Test Parameters	4842

A.13.4.2.1.3	Test Requirements	4849

A.13.4.3	SS-SINR	4849

A.13.4.3.1	Intra-frequency measurement accuracy on SCC	4849

A.13.4.3.1.1	Test Purpose and Environment	4849

A.13.4.3.1.2	Test Parameters	4849

A.13.4.3.1.3	Test Requirements	4857

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	4857

A.13.4.4.1	SSB based L1-RSRP measurement	4857

A.13.4.4.1.1	Test Purpose and Environment	4857

A.13.4.4.1.2	Test parameters	4857

A.13.4.4.1.3	Test Requirements	4861

A.13.4.5	RSSI	4861

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	4861

A.13.4.5.1.1	Test Purpose and Environment	4861

A.13.4.5.1.2	Test parameters	4861

A.13.4.5.1.3	Test Requirements	4865

A.13.4.5.2	Inter-frequency RSSI measurement accuracy on a carrier with CCA	4865

A.13.4.5.2.1	Test Purpose and Environment	4865

A.13.4.5.2.2	Test parameters	4865

A.13.4.5.2.3	Test Requirements	4869

A.13.4.6	Channel occupancy	4869

A.13.4.6.1	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	4869

A.13.4.6.1.1	Test Purpose and Environment	4869

A.13.4.6.1.2	Test parameters	4869

A.13.4.6.1.3	Test Requirements	4873

A.13.4.6.2	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	4873

A.13.4.6.2.1	Test Purpose and Environment	4873

A.13.4.6.2.2	Test parameters	4873

A.13.4.6.2.3	Test Requirements	4877

A.14	NR standalone tests for Satellite access	4877

A.14.1	RRC_IDLE state mobility	4877

A.14.1.1	Cell reselection to FR1 intra-frequency NR case	4877

A.14.1.1.1	Test Purpose and Environment	4877

A.14.1.1.2	Test Parameters	4877

A.14.1.1.3	Test Requirements	4879

A.14.1.2	Cell reselection to FR1 intra-frequency NR cell for UE configured with the feature for enhanced requirements	4880

A.14.1.2.1	Test Purpose and Environment	4880

A.14.1.2.2	Test Parameters	4880

A.14.1.2.3	Test Requirements	4884

A.14.1.3	Time-based measurement initiation to FR1 intra-frequency NR cell reselection	4885

A.14.1.3.1	Test Purpose and Environment	4885

A.14.1.3.2	Test Parameters	4885

A.14.1.3.3	Test Requirements	4886

A.14.1.4	Location-based measurement initiation to FR1 intra-frequency NR cell reselection	4887

A.14.1.4.1	Test Purpose and Environment	4887

A.14.1.4.2	Test Parameters	4887

A.14.1.4.3	Test Requirements	4889

A.14.1.5	Cell reselection to FR1 inter-frequency NR case	4890

A.14.1.5.1	Test Purpose and Environment	4890

A.14.1.5.2	Test Parameters	4890

A.14.1.5.3	Test Requirements	4892

A.14.1.6	Cell reselection to FR1 inter-frequency NR cell for UE configured with [capability for enhanced requirements]	4893

A.14.1.6.1	Test Purpose and Environment	4893

A.14.1.6.2	Test Parameters	4893

A.14.1.6.3	Test Requirements	4895

A.14.1.7	Time-based measurement initiation to FR1 inter-frequency cell reselection	4896

A.14.1.7.1	Test Purpose and Environment	4896

A.14.1.7.2	Test Parameters	4896

A.14.1.7.3	Test Requirements	4897

A.14.1.8	Location-based measurement initiation to FR1 inter-frequency NR cell reselection	4898

A.14.1.8.1	Test Purpose and Environment	4898

A.14.1.8.2	Test Parameters	4898

A.14.1.8.3	Test Requirements	4900

A.14.1.9	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	4901

A.14.1.9.1	Test Purpose and Environment	4901

A.14.1.9.2	Test Parameters	4901

A.14.1.9.3	Test Requirements	4904

A.14.1.10	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	4904

A.14.1.10.1	Test Purpose and Environment	4904

A.14.1.10.2	Test Parameters	4904

A.14.1.10.3	Test Requirements	4906

A.14.1.11	Cell reselection to FR1 inter-RAT for NR NTN carrier	4907

A.14.1.11.1	Test purpose and Environment	4907

A.14.1.11.2	Test parameters	4907

A.14.1.11.3	Test requirements	4910

A.14.1.12	Cell re-selection to FR1 inter-frequency NR case with TN carrier	4910

A.14.1.12.1	Test purpose and Environment	4910

A.14.1.12.2	Test parameters	4910

A.14.1.12.3	Test requirements	4912

A.14.2	RRC_CONNECTED state mobility	4913

A.14.2.1	Handover	4913

A.14.2.1.1	Intra-frequency SAN Handover from FR1 to FR1	4913

A.14.2.1.1.1	Test Purpose and Environment	4913

A.14.2.1.1.2	Test Parameters	4913

A.14.2.1.1.3	Test Requirements	4915

A.14.2.1.2	Inter-frequency SAN Handover from FR1 to FR1	4915

A.14.2.1.2.1	Test Purpose and Environment	4915

A.14.2.1.2.2	Test Parameters	4915

A.14.2.1.2.3	Test Requirements	4917

A.14.2.1.3	Intra-frequency SAN time-based conditional Handover from FR1 to FR1	4918

A.14.2.1.3.1	Test Purpose and Environment	4918

A.14.2.1.3.2	Test Parameters	4918

A.14.2.1.3.3	Test Requirements	4920

A.14.2.1.4	Inter-frequency SAN time-based conditional Handover from FR1 to FR1	4920

A.14.2.1.4.1	Test Purpose and Environment	4920

A.14.2.1.4.2	Test Parameters	4920

A.14.2.1.4.3	Test Requirements	4922

A.14.2.1.5	Intra-frequency SAN distance-based conditional Handover from FR1 to FR1	4922

A.14.2.1.5.1	Test Purpose and Environment	4922

A.14.2.1.5.2	Test Parameters	4923

A.14.2.1.5.3	Test Requirements	4926

A.14.2.1.6	Inter-frequency SAN distance-based conditional Handover from FR1 to FR1	4926

A.14.2.1.6.1	Test Purpose and Environment	4926

A.14.2.1.6.2	Test Parameters	4926

A.14.2.1.6.3	Test Requirements	4930

A.14.2.1.7	Intra-frequency intra-satellite Handover from FR2-NTN to FR2-NTN	4930

A.14.2.1.7.1	Test Purpose and Environment	4930

A.14.2.1.7.2	Test Parameters	4930

A.14.2.1.7.3	Test Requirements	4932

A.14.2.1.8	Intra-frequency SAN Handover from FR1 to FR1	4933

A.14.2.1.8.1	Test Purpose and Environment	4933

A.14.2.1.8.2	Test Parameters	4933

A.14.2.1.8.3	Test Requirements	4935

A.14.2.1.9	Intra-frequency inter-satellite handover from FR2-NTN to FR2-NTN	4935

A.14.2.1.9.1	Test Purpose and Environment	4935

A.14.2.1.9.2	Test Parameters	4935

A.14.2.1.9.3	Test Requirements	4938

A.14.2.2	RRC Connection Mobility Control	4938

A.14.2.2.1	SA: RRC Re-establishment for SAN	4938

A.14.2.2.1.1	Intra-frequency RRC Re-establishment in FR1	4938

A.14.2.2.1.2	Inter-frequency RRC Re-establishment in FR1	4941

A.14.2.2.2	Random Access	4944

A.14.2.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	4944

A.14.2.2.2.1.1	Test Purpose and Environment	4944

A.14.2.2.2.1.2	Test Requirements	4947

A.14.2.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	4948

A.14.2.2.2.2.1	Test Purpose and Environment	4948

A.14.2.2.2.2.2	Test Requirements	4952

A.14.2.2.3	RRC Connection Release with Redirection	4953

A.14.2.2.3.1	Redirection from NR in FR1 to NR in FR1	4953

A.14.2.2.3.1.1	Test Purpose and Environment	4953

A.14.2.2.3.1.2	Test Parameters	4953

A.14.2.2.3.1.3	Test Requirements	4956

A.14.2.2.4	RACH-based Hard Satellite switching with re-synchronization from FR1 to FR1	4956

A.14.2.2.4.1	Test Purpose and Environment	4956

A.14.2.2.4.2	Test Parameters	4956

A.14.2.2.4.3	Test Requirements	4959

A.14.2.2.5	RACH-less Soft Satellite switching with re-synchronization from FR1 to FR1	4959

A.14.2.2.5.1	Test Purpose and Environment	4959

A.14.2.2.5.2	Test Parameters	4959

A.14.2.2.5.3	Test Requirements	4961

A.14.2.3	Intra-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1	4962

A.14.2.3.1	Test Purpose and Environment	4962

A.14.2.3.2	Test Parameters	4962

A.14.2.3.3	Test Requirements	4964

A.14.2.4	Inter-frequency SAN time-based conditional Handover without L3 measurement criteria from FR1 to FR1	4964

A.14.2.4.1	Test Purpose and Environment	4964

A.14.2.4.2	Test Parameters	4964

A.14.2.4.3	Test Requirements	4966

A.14.3	Timing for Satellite Access	4966

A.14.3.1	UE transmit timing for Satellite Access	4966

A.14.3.1.1	NR UE Transmit Timing Test for FR1	4966

A.14.3.1.1.1	Test Purpose and environment	4966

A.14.3.1.1.2	Test requirements	4969

A.14.3.1.2	NR UE Transmit Timing Test for FR2-NTN	4970

A.14.3.1.2.1	Test Purpose and environment	4970

A.14.3.1.2.2	Test requirements	4974

A.14.3.2	Timing advance for satellite access	4975

A.14.3.2.1	SA FR1 timing advance adjustment accuracy	4975

A.14.3.2.1.1	Test Purpose and Environment	4975

A.14.3.2.1.2	Test Parameters	4975

A.14.3.2.1.3	Test Requirements	4978

A.14.3.2.3	SA FR2-NTN timing advance adjustment accuracy	4978

A.14.3.2.3.1	Test Purpose and Environment	4978

A.14.3.2.3.2	Test Parameters	4978

A.14.3.2.1.3	Test Requirements	4982

A.14.4	Signalling characteristics	4983

A.14.4.1	Radio link Monitoring	4983

A.14.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4983

A.14.4.1.1.1	Test Purpose and Environment	4983

A.14.4.1.1.2	Test Requirements	4987

A.14.4.1.2	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in non-DRX mode	4987

A.14.4.1.2.1	Test Purpose and Environment	4987

A.14.4.1.2.2	Test Requirements	4993

A.14.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4993

A.14.4.1.3.1	Test Purpose and Environment	4993

A.14.4.1.3.2	Test Requirements	4997

A.14.4.1.4	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with SSB-based RLM RS in DRX mode	4997

A.14.4.1.4.1	Test Purpose and Environment	4997

A.14.4.1.4.2	Test Requirements	5002

A.14.4.1.5	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	5002

A.14.4.1.5.1	Test Purpose and Environment	5002

A.14.4.1.5.2	Test Requirements	5007

A.14.4.1.6	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in non-DRX mode	5007

A.14.4.1.6.1	Test Purpose and Environment	5007

A.14.4.1.6.2	Test Requirements	5011

A.14.4.1.7	Radio Link Monitoring Out-of-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	5011

A.14.4.1.7.1	Test Purpose and Environment	5011

A.14.4.1.7.2	Test Requirements	5016

A.14.4.1.8	Radio Link Monitoring In-sync Test for FR1 SAN PCell configured with CSI-RS-based RLM in DRX mode	5016

A.14.4.1.8.1	Test Purpose and Environment	5016

A.14.4.1.8.2	Test Requirements	5021

A.14.4.1.9	Radio Link Monitoring Out-of-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode	5021

A.14.4.1.9.1	Test Purpose and Environment	5021

A.14.4.1.9.2	Test Requirements	5026

A.14.4.1.10	Radio Link Monitoring In-sync Test for FR2 SAN PCell configured with SSB-based RLM RS in non-DRX mode	5026

A.14.4.1.10.1	Test Purpose and Environment	5026

A.14.4.1.10.2	Test Requirements	5032

A.14.4.2	Beam Failure Detection and Link recovery procedures for satellite access	5032

A.14.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in non-DRX mode	5032

A.14.4.2.1.1	Test Purpose and Environment	5032

A.14.4.2.1.2	Test Requirements	5037

A.14.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with SSB-based BFD and LR in DRX mode	5037

A.14.4.2.2.1	Test Purpose and Environment	5037

A.14.4.2.2.2	Test Requirements	5042

A.14.4.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in non-DRX mode	5042

A.14.4.2.3.1	Test Purpose and Environment	5042

A.14.4.2.3.2	Test Requirements	5048

A.14.4.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell for satellite access configured with CSI-RS-based BFD and LR in DRX mode	5048

A.14.4.2.4.1	Test Purpose and Environment	5048

A.14.4.2.4.2	Test Requirements	5053

A.14.4.2.5	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	5053

A.14.4.2.5.1	Test Purpose and Environment	5053

A.14.4.2.5.2	Test Requirements	5058

A.14.4.2.6	Beam Failure Detection and Link Recovery Test for FR1 SCell for satellite access configured with CSI-RS-based BFD and SSB-based LR in DRX mode	5058

A.14.4.2.6.1	Test Purpose and Environment	5058

A.14.4.2.6.2	Test Requirements	5063

A.14.4.3	Active BWP switch for satellite access	5063

A.14.4.3.1	DCI-based and Timer-based Active BWP Switch	5063

A.14.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	5063

A.14.4.3.1.1.1	Test Purpose and Environment	5063

A.14.4.3.1.1.2	Test Requirements	5068

A.14.4.3.2	RRC-based Active BWP Switch	5068

A.14.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	5068

A.14.4.3.2.1.1	Test Purpose and Environment	5068

A.14.4.3.2.1.2	Test Requirements	5072

A.14.4.4	UE specific CBW change for satellite access	5072

A.14.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	5072

A.14.4.4.1.1	Test Purpose and Environment	5072

A.14.4.4.1.2	Test Requirements	5076

A.14.4.5	Pathloss reference signal switching delay	5076

A.14.4.5.1	MAC-CE based pathloss reference signal switch delay	5076

A.14.4.5.1.1	Test Purpose and Environment	5076

A.14.4.5.1.2	Test Requirements	5078

A.14.5	Measurement procedure	5079

A.14.5.1	Intra-frequency Measurements	5079

A.14.5.1.1	SA event triggered reporting tests without gap under non-DRX	5079

A.14.5.1.1.1	Test purpose and Environment	5079

A.14.5.1.1.2	Test parameters	5079

A.14.5.1.1.3	Test Requirements	5081

A.14.5.1.2	SA event triggered reporting tests without gap under DRX	5082

A.14.5.1.2.1	Test purpose and Environment	5082

A.14.5.1.2.2	Test parameters	5082

A.14.5.1.2.3	Test Requirements	5083

A.14.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	5084

A.14.5.1.3.1	Test purpose and Environment	5084

A.14.5.1.3.2	Test parameters	5084

A.14.5.1.3.3	Test Requirements	5086

A.14.5.1.4	SA event triggered reporting tests with single measurement gap under non-DRX for satellite access	5087

A.14.5.1.4.1	Test purpose and Environment	5087

A.14.5.1.4.2	Test parameters	5087

A.14.5.1.4.3	Test Requirements	5089

A.14.5.1.5	SA event triggered reporting tests with FNO concurrent gaps under DRX for satellite access	5090

A.14.5.1.5.1	Test purpose and Environment	5090

A.14.5.1.5.2	Test parameters	5090

A.15.5.1.5.3	Test Requirements	5092

A.14.5.1.6	SA event triggered reporting tests with PPO concurrent gaps under non-DRX with SSB index reading for satellite access	5093

A.14.5.1.6.1	Test purpose and Environment	5093

A.14.5.1.6.2	Test parameters	5093

A.14.5.1.6.3	Test Requirements	5095

A.14.5.1.7	SA event triggered reporting test with SSB time index reading without gap under non-DRX for FR2-NTN	5096

A.14.5.1.7.1	Test purpose and Environment	5096

A.14.5.1.7.2	Test parameters	5096

A.14.5.1.7.3	Test Requirements	5098

A.14.5.2	Inter-frequency Measurements	5098

A.14.5.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with single gap for satellite access	5098

A.14.5.2.1.1	Test Purpose and Environment	5098

A.14.5.2.1.2	Test Requirements	5102

A.14.5.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access	5102

A.14.5.2.2.1	Test Purpose and Environment	5102

A.14.5.2.2.2	Test Requirements	5106

A.14.5.2.3	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used with single gap for satellite access	5106

A.14.5.2.3.1	Test Purpose and Environment	5106

A.14.5.2.3.2	Test Requirements	5110

A.14.5.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in fully non-overlapped for satellite access	5110

A.14.5.2.4.1	Test Purpose and Environment	5110

A.14.5.2.4.2	Test Requirements	5114

A.14.5.2.5	void	5114

A.14.5.2.5.1	void	5114

A.14.5.2.5.2	void	5114

A.14.5.2.6	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used with two gaps in partially partial overalpping for satellite access	5114

A.14.5.2.6.1	Test Purpose and Environment	5114

A.14.5.2.6.2	Test Requirements	5118

A.14.5.2.7	Event triggered reporting test without gap under non-DRX	5118

A.14.5.2.7.1	Test purpose and Environment	5118

A.14.5.2.7.2	Test parameters	5118

A.14.5.2.7.3	Test Requirements	5120

A.14.5.2.8	Event triggered reporting tests without gap under DRX	5121

A.14.5.2.8.1	Test purpose and Environment	5121

A.14.5.2.8.2	Test parameters	5121

A.14.5.2.8.3	Test Requirements	5122

A.14.5.3	L1-RSRP measurement for beam reporting for satellite access	5123

A.14.5.3.1	SSB based L1-RSRP measurement for satellite access when DRX is not used	5123

A.14.5.3.1.1	Test Purpose and Environment	5123

A.14.5.3.1.2	Test parameters	5123

A.14.5.3.1.3	Test Requirements	5126

A.14.5.3.2	SSB based L1-RSRP measurement for satellite access when DRX is used	5126

A.14.5.3.2.1	Test Purpose and Environment	5126

A.14.5.3.2.2	Test parameters	5126

A.14.5.3.2.3	Test Requirements	5130

A.14.5.3.3	CSI-RS based L1-RSRP measurement for satellite access when DRX is not used	5130

A.14.5.3.3.1	Test Purpose and Environment	5130

A.14.5.3.3.2	Test parameters	5130

A.14.5.3.3.3	Test Requirements	5134

A.14.5.3.4	CSI-RS based L1-RSRP measurement for satellite access when DRX is used	5134

A.14.5.3.4.1	Test Purpose and Environment	5134

A.14.5.3.4.2	Test parameters	5135

A.14.5.3.4.3	Test Requirements	5138

A.14.6	Measurement Performance requirements	5138

A.14.6.1	 SS-RSRP for SAN	5138

A.14.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5138

A.14.6.1.1.1	Test Purpose and Environment	5138

A.14.6.1.1.2	Test parameters	5138

A.14.6.1.1.3	Test Requirements	5142

A.14.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	5142

A.14.6.1.2.1	Test Purpose and Environment	5142

A.14.6.1.2.2	Test parameters	5142

A.14.6.1.2.3	Test Requirements	5145

A.14.6.2	SS-RSRQ	5145

A.14.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5145

A.14.6.2.1.1	Test Purpose and Environment	5145

A.14.6.2.1.2	Test Parameters	5145

A.14.6.2.1.3	Test Requirements	5148

A.14.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for satellite access	5148

A.14.6.2.2.1	Test Purpose and Environment	5148

A.14.6.2.2.2	Test Parameters	5148

A.14.6.2.2.3	Test Requirements	5151

A.14.6.3	SS-SINR	5151

A.14.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5151

A.14.6.3.1.1	Test Purpose and Environment	5151

A.14.6.3.1.2	Test Parameters	5151

A.14.6.3.1.3	Test Requirements	5155

A.14.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	5155

A.14.6.3.2.1	Test Purpose and Environment	5155

A.14.6.3.2.2	Test Parameters	5155

A.14.6.3.2.3	Test Requirements	5159

A.14.6.4	L1-RSRP measurement for beam reporting	5159

A.14.6.4.1	SSB based L1-RSRP measurement	5159

A.14.6.4.1.1	Test Purpose and Environment	5159

A.14.6.4.1.2	Test parameters	5159

A.14.6.4.1.3	Test Requirements	5162

A.14.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	5163

A.14.6.4.2.1	Test Purpose and Environment	5163

A.14.6.4.2.2	Test parameters	5163

A.14.6.4.2.3	Test Requirements	5166

A.14.6.4.3	SSB based L1-RSRP measurement for VSAT UE in FR2-NTN when DRX is not used	5167

A.14.6.4.3.1	Test Purpose and Environment	5167

A.14.6.4.3.2	Test parameters	5167

A.14.6.4.3.3	Test Requirements	5169

A.15	NR standalone tests with one or more NR cells in FR2-2	5122

A.15.1	SA: RRC_IDLE state mobility	5122

A.15.1.1	Cell re-selection to NR	5122

A.15.1.1.1	Cell reselection to FR2-2 intra-frequency NR case	5122

A.15.1.1.1.1	Test Purpose and Environment	5122

A.15.1.1.1.2	Test Parameters	5122

A.15.1.1.1.3	Test Requirements	5126

A.15.1.2	Cell reselection to FR2-2 inter-frequency NR case	5126

A.15.1.2.1	Test Purpose and Environment	5126

A.15.1.2.2	Test Parameters	5126

A.15.1.2.3	Test Requirements	5130

A.15.1.3	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5130

A.15.1.3.1	Test Purpose and Environment	5130

A.15.1.3.2	Test Parameters	5130

A.15.1.3.3	Test Requirements	5133

A.15.1.4	Cell reselection to FR2-2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5133

A.15.1.4.1	Test Purpose and Environment	5133

A.15.1.4.2	Test Parameters	5133

A.15.1.4.3	Test Requirements	5136

A.15.1.5	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	5136

A.15.1.5.1	Test Purpose and Environment	5136

A.15.1.5.2	Test Parameters	5136

A.15.1.5.3	Test Requirements	5140

A.15.1.6	Cell reselection to FR2-2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	5140

A.15.1.6.1	Test Purpose and Environment	5140

A.15.1.6.2	Test Parameters	5140

A.15.1.6.3	Test Requirements	5143

A.15.2	Signaling characteristics	5144

A.15.2.1	SCell Activation and Deactivation Delay	5144

A.15.2.1.1	SCell Activation and deactivation for SCell in FR2-2 intra-band in non-DRX	5144

A.15.2.1.1.1	Test Purpose and Environment	5144

A.15.2.1.1.2	Test Requirements	5146

A.15.2.1.2	SCell Activation and deactivation for FR1+FR2-2 inter-band with target SCell in FR2-2	5146

A.15.2.1.2.1	Test Purpose and Environment	5146

A.15.2.1.2.2	Test Requirements	5151

A.15.2.1.3	SCell Activation and deactivation for SCell in FR2-2 inter-band in non-DRX	5152

A.15.2.1.3.1	Test Purpose and Environment	5152

A.15.2.1.3.2	Test Requirements	5155

A.15.2.1.4	Direct SCell activation at SCell addition of known SCell in FR2-2	5156

A.15.2.1.4.1	Test Purpose and Environment	5156

A.15.2.1.4.2	Test Requirements	5159

A.15.2.1.5	Direct SCell activation at handover with known SCell in FR2-2	5160

A.15.2.1.5.1	Test Purpose and Environment	5160

A.15.2.1.5.2	Test Requirements	5164

A.15.3	RRC_CONNECTED state mobility	5165

A.15.3.1	Handover	5165

A.15.3.1.1	Intra-frequency handover from FR2-2 carrier with CCA to FR2-2 carrier with CCA; unknown target cell	5165

A.15.3.1.1.1	Test Purpose and Environment	5165

A.15.3.1.1.2	Test Parameters	5165

A.15.3.1.1.3	Test Requirements	5169

A.15.3.1.2	Inter-frequency handover from FR1 to FR2-2 carrier with CCA; unknown target cell	5169

A.15.3.1.2.1	Test Purpose and Environment	5169

A.15.3.1.2.2	Test Parameters	5169

A.15.3.1.2.3	Test Requirements	5173

A.15.4	Measurement procedure	5174

A.15.4.1	Intra-frequency Measurements	5174

A.15.4.1.1	SA event triggered reporting test without gap under non-DRX for FR2-2 with CCA	5174

A.15.4.1.1.1	Test purpose and Environment	5174

A.15.4.1.1.2	Test Requirements	5176

A.15.4.2	Inter-frequency Measurements	5177

A.15.4.2.1	SA event triggered reporting tests for FR2-2 with CCA without SSB time index detection when DRX is not used (PCell in FR2-2)	5177

A.15.4.2.1.1	Test Purpose and Environment	5177

A.15.4.2.1.2	Test Requirements	5180

A.16	NR standalone tests with all NR cells in FR1 for RedCap	5181

A.16.1	SA: RRC_IDLE state mobility for RedCap	5181

A.16.1.1	Cell re-selection to NR	5181

A.16.1.1.1	Cell reselection to FR1 intra-frequency NR case for 1 Rx UE	5181

A.16.1.1.1.1	Test Purpose and Environment	5181

A.16.1.1.1.2	Test Parameters	5181

A.16.1.1.1.3	Test Requirements	5185

A.16.1.1.2	Cell reselection to FR1 intra-frequency NR case for 2 Rx UE	5185

A.16.1.1.2.1	Test Purpose and Environment	5185

A.16.1.1.2.2	Test Parameters	5185

A.16.1.1.2.3	Test Requirements	5189

A.16.1.1.3	Cell reselection to FR1 inter-frequency NR case for 1 Rx UE	5189

A.16.1.1.3.1	Test Purpose and Environment	5189

A.16.1.1.3.2	Test Parameters	5189

A.16.1.1.3.3	Test Requirements	5194

A.16.1.1.4	Cell reselection to FR1 inter-frequency NR case for 2 Rx UE	5194

A.16.1.1.4.1	Test Purpose and Environment	5194

A.16.1.1.4.2	Test Parameters	5194

A.16.1.1.4.3	Test Requirements	5199

A.16.1.1.5	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5199

A.16.1.1.5.1	Test Purpose and Environment	5199

A.16.1.1.5.2	Test Parameters	5199

A.16.1.1.5.3	Test Requirements	5204

A.16.1.1.6	Cell reselection to FR1 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5204

A.16.1.1.6.1	Test Purpose and Environment	5204

A.16.1.1.6.2	Test Parameters	5204

A.16.1.1.6.3	Test Requirements	5209

A.16.1.1.7	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5209

A.16.1.1.7.1	Test Purpose and Environment	5209

A.16.1.1.7.2	Test Parameters	5209

A.16.1.1.7.3	Test Requirements	5214

A.16.1.1.8	Cell reselection to FR1 inter-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5214

A.16.1.1.8.1	Test Purpose and Environment	5214

A.16.1.1.8.2	Test Parameters	5215

A.16.1.1.8.3	Test Requirements	5219

A.16.1.2	Inter-RAT E-UTRAN cell re-selection for RedCap	5220

A.16.1.2.1	Cell reselection to higher priority E-UTRAN for 1RX	5220

A.16.1.2.1.1	Test Purpose and Environment	5220

A.16.1.2.1.2	Test Parameters	5220

A.16.1.2.1.3	Test Requirements	5223

A.16.1.2.2	Cell reselection to higher priority E-UTRAN for 2RX	5224

A.16.1.2.2.1	Test Purpose and Environment	5224

A.16.1.2.2.2	Test Parameters	5224

A.16.1.2.2.3	Test Requirements	5227

A.16.1.2.3.1	Test Purpose and Environment	5228

A. 16.1.2.3.2	Test Parameters	5228

A.16.1.2.3.3	Test Requirements	5231

A.16.1.2.4.1	Test Purpose and Environment	5232

A.16.1.2.4.2	Test Parameters	5232

A.16.1.3.1.3	Test Requirements	5235

A.16.1.2.5	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	5236

A.16.1.2.5.1	Test Purpose and Environment	5236

A.16.1.2.5.2	Test Parameters	5236

A.16.1.2.5.3	Test Requirements	5239

A.16.1.2.6	Cell reselection to lower priority E-UTRAN for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5240

A.16.1.2.6.1	Test Purpose and Environment	5240

A.16.1.2.6.2	Test Parameters	5240

A.16.1.2.6.3	Test Requirements	5243

A.16.2	SA: RRC_INACTIVE state mobility for RedCap	5244

A.16.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5244

A.16.2.1.1	NR UE CG-SDT Test in FR1 for 1Rx RedCap UE	5244

A.16.2.1.1.1	Test purpose and Environment	5244

A.16.2.1.1.2	Test Parameters	5246

A.16.2.1.1.3	Test requirements	5249

A.16.2.1.2	NR UE CG-SDT Test in FR1 for 2Rx RedCap UE	5249

A.16.2.1.2.1	Test purpose and Environment	5249

A.16.2.1.2.2	Test Parameters	5251

A.16.2.1.2.3	Test requirements	5254

A.16.3	RRC_CONNECTED state mobility for RedCap	5254

A.16.3.1	Handover	5254

A.16.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell for 1 Rx UE	5254

A.16.3.1.1.1	Test Purpose and Environment	5254

A.16.3.1.1.2	Test Parameters	5255

A.16.3.1.1.3	Test Requirements	5258

A.16.3.1.2	Intra-frequency handover from FR1 to FR1; known target cell for 2 Rx UE	5258

A.16.3.1.2.1	Test Purpose and Environment	5258

A.16.3.1.2.2	Test Parameters	5258

A.16.3.1.2.3	Test Requirements	5262

A.16.3.1.3	Intra-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5262

A.16.3.1.3.1	Test Purpose and Environment	5262

A.16.3.1.3.2	Test Parameters	5262

A.16.3.1.3.3	Test Requirements	5266

A.16.3.1.4	Intra-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5266

A.16.3.1.4.1	Test Purpose and Environment	5266

A.16.3.1.4.2	Test Parameters	5266

A.16.3.1.5	Inter-frequency handover from FR1 to FR1; unknown target cell for 1 Rx UE	5270

A.16.3.1.5.1	Test Purpose and Environment	5270

A.16.3.1.5.2	Test Parameters	5270

A.16.3.1.5.3	Test Requirements	5274

A.16.3.1.6	Inter-frequency handover from FR1 to FR1; unknown target cell for 2 Rx UE	5274

A.16.3.1.6.1	Test Purpose and Environment	5274

A.16.3.1.6.2	Test Parameters	5274

A.16.3.1.6.3	Test Requirements	5278

A.16.3.1.7	SA NR - E-UTRAN handover for 1Rx UE	5278

A.16.3.1.7.1	Test Purpose and Environment	5278

A.16.3.1.7.2	Test Requirements	5284

A.16.3.1.8	 SA NR - E-UTRAN handover for 2Rx UE	5284

A.16.3.1.8.1	Test Purpose and Environment	5284

A.16.3.1.8.2	Test Requirements	5290

A.16.3.1.9	SA NR - E-UTRAN handover with unknown target cell for 1 Rx UE	5290

A.16.3.1.9.1	Test Purpose and Environment	5290

A.16.3.1.9.2	Test Requirements	5297

A.16.3.1.10	SA NR - E-UTRAN handover with unknown target cell for 2 Rx UE	5297

A.16.3.1.10.1	Test Purpose and Environment	5297

A.16.3.1.10.2	Test Requirements	5303

A.16.3.2	RRC Connection Mobility Control	5303

A.16.3.2.1	SA: RRC Re-establishment	5303

A.16.3.2.1.1	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	5303

A.16.3.2.1.2	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	5307

A.16.3.2.1.3	Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	5311

A.16.3.2.1.4	Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	5316

A.16.3.2.1.5	Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	5321

A.16.3.2.1.6	Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	5325

A.16.3.2.2	Random Access	5329

A.16.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5329

A.16.3.2.2.2	4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5335

A.16.3.2.2.3	4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	5340

A.16.3.2.2.4	4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	5345

A.16.3.2.2.5	2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	5350

A.16.3.2.2.6	2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	5354

A.16.3.2.2.7	2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	5358

A.16.3.2.2.8	2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	5362

A.16.3.2.3	SA: RRC Connection Release with Redirection	5366

A.16.3.2.3.1	Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	5366

A.16.3.2.3.2	Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	5370

A.16.3.2.3.3	Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	5374

A.16.3.2.3.4	Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	5381

A.16.4	Timing for RedCap	5388

A.16.4.1	UE transmit timing	5388

A.16.4.1.1	NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	5388

A.16.4.1.1.1	Test Purpose and environment	5388

A.16.4.1.1.2	Test requirements	5391

A.16.4.1.2	NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	5391

A.16.4.1.2.1	Test Purpose and environment	5391

A.16.4.1.2.2	Test requirements	5394

A.16.4.2	UE timer accuracy	5394

A.16.4.3	Timing advance	5394

A.16.4.3.1	SA FR1 timing advance adjustment accuracy for 1 Rx UE	5394

A.16.4.3.1.1	Test Purpose and Environment	5394

A.16.4.3.1.2	Test Parameters	5394

A.16.4.3.1.3	Test Requirements	5398

A.16.4.3.2	SA FR1 timing advance adjustment accuracy for 2 Rx UE	5398

A.16.4.3.2.1	Test Purpose and Environment	5398

A.16.4.3.2.2	Test Parameters	5399

A.16.4.3.2.3	Test Requirements	5402

A.16.5	Signalling characteristics for RedCap	5403

A.16.5.1	Radio link Monitoring	5403

A.16.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5403

A.16.5.1.1.1	Test Purpose and Environment	5403

A.16.5.1.1.2	Test Requirements	5408

A.16.5.1.2	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5408

A.16.5.1.2.1	Test Purpose and Environment	5408

A.16.5.1.2.2	Test Requirements	5414

A.16.5.1.3	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	5414

A.16.5.1.3.1	Test Purpose and Environment	5414

A.16.5.1.3.2	Test Requirements	5420

A.16.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	5420

A.16.5.1.4.1	Test Purpose and Environment	5420

A.16.5.1.4.2	Test Requirements	5426

A.16.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5426

A.16.5.1.5.1	Test Purpose and Environment	5426

A.16.5.1.5.2	Test Requirements	5432

A.16.5.1.6	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5432

A.16.5.1.6.1	Test Purpose and Environment	5432

A.16.5.1.6.2	Test Requirements	5437

A.16.5.1.7	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	5437

A.16.5.1.7.1	Test Purpose and Environment	5437

A.16.5.1.7.2	Test Requirements	5442

A.16.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	5442

A.16.5.1.8.1	Test Purpose and Environment	5442

A.16.5.1.8.2	Test Requirements	5448

A.16.5.1.9	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5448

A.16.5.1.9.1	Test Purpose and Environment	5448

A.16.5.1.9.2	Test Requirements	5454

A.16.5.1.10	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5454

A.16.5.1.10.1	Test Purpose and Environment	5454

A.16.5.1.10.2	Test Requirements	5460

A.16.5.1.11	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	5460

A.16.5.1.11.1	Test Purpose and Environment	5460

A.16.5.1.11.2	Test Requirements	5466

A.16.5.1.12	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	5466

A.16.5.1.12.1	Test Purpose and Environment	5466

A.16.5.1.12.2	Test Requirements	5472

A.16.5.1.13	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5472

A.16.5.1.13.1	Test Purpose and Environment	5472

A.16.5.1.13.2	Test Requirements	5477

A.16.5.1.14	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5477

A.16.5.1.14.1	Test Purpose and Environment	5477

A.16.5.1.14.2	Test Requirements	5483

A.16.5.1.15	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	5483

A.16.5.1.15.1	Test Purpose and Environment	5483

A.16.5.1.15.2	Test Requirements	5489

A.16.5.1.16	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	5489

A.16.5.1.16.1	Test Purpose and Environment	5489

A.16.5.1.16.2	Test Requirements	5495

A.16.5.2	Beam Failure Detection and Link recovery procedures	5495

A.16.5.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	5495

A.16.5.2.1.1	Test Purpose and Environment	5495

A.16.5.2.1.2	Test Requirements	5501

A.16.5.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5501

A.16.5.2.2.1	Test Purpose and Environment	5501

A.16.5.2.2.2	Test Requirements	5507

A.16.5.2.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	5507

A.16.5.2.3.1	Test Purpose and Environment	5507

A.16.5.2.3.2	Test Requirements	5514

A.16.5.2.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	5514

A.16.5.2.4.1	Test Purpose and Environment	5514

A.16.5.2.4.2	Test Requirements	5520

A.16.5.2.5	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	5520

A.16.5.2.5.1	Test Purpose and Environment	5520

A.16.5.2.5.2	Test Requirements	5526

A.16.5.2.6	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	5526

A.16.5.2.6.1	Test Purpose and Environment	5526

A.16.5.2.6.2	Test Requirements	5532

A.16.5.2.7	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	5532

A.16.5.2.7.1	Test Purpose and Environment	5532

A.16.5.2.7.2	Test Requirements	5538

A.16.5.2.8	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	5538

A.16.5.2.8.1	Test Purpose and Environment	5538

A.16.5.2.8.2	Test Requirements	5544

A.16.5.3	Active BWP switch	5544

A.16.5.3.1	DCI-based and Timer-based Active BWP Switch	5544

A.16.5.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA for 1 Rx UE	5544

A.16.5.3.1.1.1	Test Purpose and Environment	5544

A.16.5.3.1.1.2	Test Requirements	5549

A.16.5.3.1.2	NR FR1 DL active BWP switch with non-DRX in SA for 2 Rx UE	5549

A.16.5.3.1.2.1	Test Purpose and Environment	5549

A.16.5.3.1.2.2	Test Requirements	5554

A.16.5.3.2	RRC-based Active BWP Switch	5554

A.16.5.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 1 Rx UE	5554

A.16.5.3.2.1.1	Test Purpose and Environment	5554

A.16.5.3.2.1.2	Test Requirements	5558

A.16.5.3.2.2	NR FR1 DL active BWP switch of Cell with non-DRX in SA for 2 Rx UE	5558

A.16.5.3.2.2.1	Test Purpose and Environment	5558

A.16.5.3.2.2.2	Test Requirements	5562

A.16.5.4	UE specific CBW change	5562

A.16.5.4.1	UE specific CBW change on PCell in FR1 in non-DRX for 1 Rx UE	5562

A.16.5.4.1.1	Test Purpose and Environment	5562

A.16.5.4.1.2	Test Requirements	5566

A.16.5.4.2	UE specific CBW change on PCell in FR1 in non-DRX for 2 Rx UE	5566

A.16.5.4.2.1	Test Purpose and Environment	5566

A.16.5.4.2.2	Test Requirements	5570

A.16.6	Measurement procedure for RedCap	5570

A.16.6.1	Intra-frequency Measurements	5570

A.16.6.1.1	SA event triggered reporting tests without gap under non-DRX for 1 Rx UE	5570

A.16.6.1.1.1	Test purpose and Environment	5570

A.16.6.1.1.2	Test parameters	5570

A.16.6.1.1.3	Test Requirements	5574

A.16.6.1.2	SA event triggered reporting tests without gap under non-DRX for 2 Rx UE	5574

A.16.6.1.2.1	Test purpose and Environment	5574

A.16.6.1.2.2	Test parameters	5574

A.16.6.1.2.3	Test Requirements	5578

A.16.6.1.3	SA event triggered reporting tests without gap under DRX for 1 Rx UE	5578

A.16.6.1.3.1	Test purpose and Environment	5578

A.16.6.1.3.2	Test parameters	5578

A.16.6.1.3.3	Test Requirements	5582

A.16.6.1.4	SA event triggered reporting tests without gap under DRX for 2 Rx UE	5582

A.16.6.1.4.1	Test purpose and Environment	5582

A.16.6.1.4.2	Test parameters	5582

A.16.6.1.4.3	Test Requirements	5586

A.16.6.1.5	SA event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	5586

A.16.6.1.5.1	Test purpose and Environment	5586

A.16.6.1.5.2	Test parameters	5586

A.16.6.1.5.3	Test Requirements	5590

A.16.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	5590

A.16.6.1.6.1	Test purpose and Environment	5590

A.16.6.1.6.2	Test parameters	5590

A.16.6.1.6.3	Test Requirements	5594

A.16.6.1.7	SA event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	5594

A.16.6.1.7.1	Test purpose and Environment	5594

A.16.6.1.7.2	Test parameters	5594

A.16.6.1.7.3	Test Requirements	5598

A.16.6.1.8	SA event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	5598

A.16.6.1.8.1	Test purpose and Environment	5598

A.16.6.1.8.2	Test parameters	5598

A.16.6.1.8.3	Test Requirements	5602

A.16.6.1.9	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	5602

A.16.6.1.9.1	Test purpose and Environment	5602

A.16.6.1.9.2	Test parameters	5602

A.16.6.1.9.3	Test Requirements	5604

A.16.6.1.10	SA event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	5605

A.16.6.1.10.1	Test purpose and Environment	5605

A.16.6.1.10.2	Test parameters	5605

A.16.6.1.10.3	Test Requirements	5606

A.16.6.1.11	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	5607

A.16.6.1.11.1	Test purpose and Environment	5607

A.16.6.1.11.2	Test parameters	5607

A.16.6.1.11.3	Test Requirements	5609

A.16.6.1.12	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2 Rx UE	5610

A.16.6.1.12.1	Test purpose and Environment	5610

A.16.6.1.12.2	Test parameters	5610

A.16.6.1.12.3	Test Requirements	5612

A.16.6.2	Inter-frequency Measurements	5613

A.16.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	5613

A.16.6.2.1.1	Test Purpose and Environment	5613

A.16.6.2.1.2	Test Requirements	5618

A.16.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	5618

A.16.6.2.2.1	Test Purpose and Environment	5618

A.16.6.2.2.2	Test Requirements	5623

A.16.6.2.3	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	5623

A.16.6.2.3.1	Test Purpose and Environment	5623

A.16.6.2.3.2	Test Requirements	5626

A.16.6.2.4	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	5627

A.16.6.2.4.1	Test Purpose and Environment	5627

A.16.6.2.4.2	Test Requirements	5630

A.16.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	5631

A.16.6.2.5.1	Test Purpose and Environment	5631

A.16.6.2.5.2	Test Requirements	5634

A.16.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	5635

A.16.6.2.6.1	Test Purpose and Environment	5635

A.16.6.2.6.2	Test Requirements	5638

A.16.6.2.7	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	5639

A.16.6.2.7.1	Test Purpose and Environment	5639

A.16.6.2.7.2	Test Requirements	5643

A.16.6.2.8	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	5644

A.16.6.2.8.1	Test Purpose and Environment	5644

A.16.6.2.8.2	Test Requirements	5647

A.16.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	5648

A.16.6.2.9.1	Test Purpose and Environment	5648

A.16.6.2.9.2	Test Requirements	5651

A.16.6.2.10	SA event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	5651

A.16.6.2.10.1	Test Purpose and Environment	5651

A.16.6.2.10.2	Test Requirements	5654

A.16.6.2.11	SA event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	5654

A.16.6.2.11.1	Test Purpose and Environment	5654

A.16.6.2.11.2	Test Requirements	5657

A.16.6.2.12	SA event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	5657

A.16.6.2.12.1	Test Purpose and Environment	5657

A.16.6.2.12.2	Test Requirements	5661

A.16.6.3	Inter-RAT Measurements	5662

A.16.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 1 Rx UE	5662

A.16.6.3.1.1	Test purpose and Environment	5662

A.16.6.3.1.2	Test Requirements	5667

A.16.6.3.2	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1 for 2 Rx UE	5667

A.16.6.3.2.1	Test purpose and Environment	5667

A.16.6.3.2.2	Test Requirements	5672

A.16.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 1 Rx UE	5672

A.16.6.3.3.1	Test purpose and Environment	5672

A.16.6.3.3.2	Test Requirements	5677

A.16.6.3.4	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for 2 Rx UE	5677

A.16.6.3.4.1	Test purpose and Environment	5677

A.16.6.3.4.2	Test Requirements	5682

A.16.6.4	L1-RSRP measurement for beam reporting	5682

A.16.6.4.1	SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	5682

A.16.6.4.1.1	Test Purpose and Environment	5682

A.16.6.4.1.2	Test parameters	5682

A.16.6.4.1.3	Test Requirements	5686

A.16.6.4.2	SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	5686

A.16.6.4.2.1	Test Purpose and Environment	5686

A.16.6.4.2.2	Test parameters	5687

A.16.6.4.2.3	Test Requirements	5690

A.16.6.4.3	SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	5690

A.16.6.4.3.1	Test Purpose and Environment	5690

A.16.6.4.3.2	Test parameters	5691

A.16.6.4.3.3	Test Requirements	5694

A.16.6.4.4	SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	5694

A.16.6.4.4.1	Test Purpose and Environment	5694

A.16.6.4.4.2	Test parameters	5695

A.16.6.4.4.3	Test Requirements	5698

A.16.6.4.5	CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	5698

A.16.6.4.5.1	Test Purpose and Environment	5698

A.16.6.4.5.2	Test parameters	5699

A.16.6.4.5.3	Test Requirements	5702

A.16.6.4.6	CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	5702

A.16.6.4.6.1	Test Purpose and Environment	5702

A.16.6.4.6.2	Test parameters	5703

A.16.6.4.6.3	Test Requirements	5706

A.16.6.4.7	CSI-RS based L1-RSRP measurement when DRX is used for 1 Rx UE	5706

A.16.6.4.7.1	Test Purpose and Environment	5706

A.16.6.4.7.2	Test parameters	5707

A.16.6.4.7.3	Test Requirements	5710

A.16.6.4.8	CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	5710

A.16.6.4.8.1	Test Purpose and Environment	5710

A.16.6.4.8.2	Test parameters	5711

A.16.6.4.8.3	Test Requirements	5714

A.16.6.5	NR measurements with autonomous gaps	5714

A.16.6.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 1 Rx UE	5714

A.16.6.5.1.1	Test Purpose and Environment	5714

A.16.6.5.1.2	Test Parameters	5714

A.16.6.5.1.3	Test Requirements	5717

A.16.6.5.2	SA intra-frequency CGI identification of NR neighbor cell in FR1 for 2 Rx UE	5717

A.16.6.5.2.1	Test Purpose and Environment	5717

A.16.6.5.2.2	Test Parameters	5717

A.16.6.5.2.3	Test Requirements	5720

A.16.6.5.3	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	5720

A.16.6.5.3.1	Test Purpose and Environment	5720

A.16.6.5.3.2	Test Requirements	5725

A.16.6.5.4	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	5726

A.16.6.5.4.1	Test Purpose and Environment	5726

A.16.6.5.4.2	Test Requirements	5731

A.16.7	Measurement Performance requirements for RedCap	5732

A.16.7.1	SS-RSRP	5732

A.16.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5732

A.16.7.1.1.1	Test Purpose and Environment	5732

A.16.7.1.1.2	Test parameters	5732

A.16.7.1.1.3	Test Requirements	5737

A.16.7.1.2	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2Rx UE	5737

A.16.7.1.2.1	Test Purpose and Environment	5737

A.16.7.1.2.2	Test parameters	5737

A.16.7.1.2.3	Test Requirements	5742

A.16.7.1.3	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5742

A.16.7.1.3.1	Test Purpose and Environment	5742

A.16.7.1.3.2	Test parameters	5742

A.16.7.1.3.3	Test Requirements	5746

A.16.7.1.4	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5747

A.16.7.1.4.1	Test Purpose and Environment	5747

A.16.7.1.4.2	Test parameters	5747

A.16.7.1.4.3	Test Requirements	5751

A.16.7.2	SS-RSRQ	5752

A.16.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5752

A.16.7.2.1.1	Test Purpose and Environment	5752

A.16.7.2.1.2	Test Parameters	5752

A.16.7.2.1.3	Test Requirements	5757

A.16.7.2.2	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5757

A.16.7.2.2.1	Test Purpose and Environment	5757

A.16.7.2.2.2	Test Parameters	5757

A.16.7.2.2.3	Test Requirements	5762

A.16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5762

A.16.7.2.3.1	Test Purpose and Environment	5762

A.16.7.2.3.2	Test parameters	5762

A.16.7.2.3.3	Test Requirements	5767

A.16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5767

A.16.7.2.4.1	Test Purpose and Environment	5767

A.16.7.2.4.2	Test parameters	5768

A.16.7.2.4.3	Test Requirements	5773

A.16.7.3	SS-SINR	5773

A.16.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5773

A.16.7.3.1.1	Test Purpose and Environment	5773

A.16.7.3.1.2	Test parameters	5774

A.16.7.3.1.3	Test Requirements	5778

A.16.7.3.2	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5779

A.16.7.3.2.1	Test Purpose and Environment	5779

A.16.7.3.2.2	Test parameters	5779

A.16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	5783

A.16.7.3.3.1	Test Purpose and Environment	5783

A.16.7.3.3.2	Test parameters	5784

A.16.7.3.3.3	Test Requirements	5789

A.16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	5789

A.16.7.3.4.1	Test Purpose and Environment	5789

A.16.7.3.4.2	Test parameters	5789

A.16.7.3.4.3	Test Requirements	5794

A.16.7.4	L1-RSRP measurement for beam reporting	5794

A.16.7.4.1	SSB based L1-RSRP measurement for 1 Rx UE	5794

A.16.7.4.1.1	Test Purpose and Environment	5794

A.16.7.4.1.2	Test parameters	5795

A.16.7.4.1.3	Test Requirements	5799

A.16.7.4.2	SSB based L1-RSRP measurement for 2 Rx UE	5799

A.16.7.4.2.1	Test Purpose and Environment	5799

A.16.7.4.2.2	Test parameters	5799

A.16.7.4.2.3	Test Requirements	5800

A.16.7.4.3	CSI-RS based L1-RSRP measurement on resource set with repetition off for 1 Rx UE	5800

A.16.7.4.3.1	Test Purpose and Environment	5800

A.16.7.4.3.2	Test parameters	5800

A.16.7.4.3.3	Test Requirements	5804

A.16.7.4.4	CSI-RS based L1-RSRP measurement on resource set with repetition off for 2 Rx UE	5804

A.16.7.4.4.1	Test Purpose and Environment	5804

A.16.7.4.4.2	Test parameters	5805

A.16.7.4.4.3	Test Requirements	5805

A.16.7.5	E-UTRAN RSRP	5805

A.16.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5805

A.16.7.5.1.1	Test Purpose and Environment	5805

A.16.7.5.1.2	Test parameters	5805

A.16.7.5.1.3	Test Requirements	5811

A.16.7.5.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5812

A.16.7.5.2.1	Test Purpose and Environment	5812

A.16.7.5.2.2	Test parameters	5812

A.16.7.5.2.3	Test Requirements	5818

A.16.7.6	E-UTRAN RSRQ	5819

A.16.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell for 1 Rx UE	5819

A.16.7.6.1.1	Test Purpose and Environment	5819

A.16.7.6.1.2	Test parameters	5819

A.16.7.6.1.3	Test Requirements	5825

A.16.7.6.2	SA: inter-RAT measurement accuracy with FR1 serving cell for 2 Rx UE	5826

A.16.7.6.2.1	Test Purpose and Environment	5826

A.16.7.6.2.2	Test parameters	5826

A.16.7.6.2.3	Test Requirements	5832

A.17	NR standalone tests with one or more NR cells in FR2 for RedCap	5837

A.17.1	SA: RRC_IDLE state mobility for RedCap	5837

A.17.1.1	Cell re-selection to NR	5837

A.17.1.1.1	Cell reselection to FR2 intra-frequency NR case for 2 Rx	5837

A.17.1.1.1.1	Test Purpose and Environment	5837

A.17.1.1.1.2	Test Parameters	5837

A.17.1.1.1.3	Test Requirements	5841

A.17.1.1.2	Cell reselection to FR2 inter-frequency NR case	5841

A.17.1.1.2.1	Test Purpose and Environment	5841

A.17.1.1.2.2	Test Parameters	5841

A.17.1.1.2.3	Test Requirements	5845

A.17.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	5845

A.17.1.1.3.1	Test Purpose and Environment	5845

A.17.1.1.3.2	Test Parameters	5845

A.17.1.1.3.3	Test Requirements	5848

A.17.1.1.4	Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	5848

A.17.1.1.4.1	Test Purpose and Environment	5848

A.17.1.1.4.2	Test Parameters	5848

A.17.1.1.4.3	Test Requirements	5852

A.17.2	SA: RRC_INACTIVE state mobility for RedCap	5852

A.17.2.1	Configured Grant based Small Data Transmissions (CG-SDT) for RedCap	5852

A.17.2.1.1	TA validation for CG-SDT in FR2 for RedCap	5852

A.17.2.1.1.1	Test Purpose and Environment	5852

A.17.2.1.1.2	Test Requirements	5856

A.17.3	RRC_CONNECTED state mobility for RedCap	5856

A.17.3.1	Handover for RedCap	5856

A.17.3.1.1	Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5856

A.17.3.1.1.1	Test Purpose and Environment	5856

A.17.3.1.1.2	Test Parameters	5856

A.17.3.1.1.3	Test Requirements	5859

A.17.3.1.2	Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	5859

A.17.3.1.2.1	Test Purpose and Environment	5859

A.17.3.1.2.2	Test Parameters	5859

A.17.3.1.2.3 Test Requirements	5862

A.17.3.2	RRC Connection Mobility Control for RedCap	5862

A.17.3.2.1	SA: RRC Re-establishment	5862

A.17.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	5862

A.17.3.2.1.1.1	Test Purpose and Environment	5862

A.17.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	5865

A.17.3.2.1.2.1	Test Purpose and Environment	5865

A.17.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	5868

A.17.3.2.1.3.1	Test Purpose and Environment	5868

A.17.3.2.1.3.2	Test Requirements	5870

A.17.3.2.2	Random Access	5871

A.17.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	5871

A.17.3.2.2.1.1	Test Purpose and Environment	5871

A.17.3.2.2.1.2	Test Requirements	5873

A.17.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	5875

A.17.3.2.2.2.1	Test Purpose and Environment	5875

A.17.3.2.2.2.2	Test Requirements	5877

A.17.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	5879

A.17.3.2.2.3.1	Test Purpose and Environment	5879

A.17.3.2.2.3.2	Test Requirements	5881

A.17.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	5882

A.17.3.2.2.4.1	Test Purpose and Environment	5882

A.17.3.2.2.4.2	Test Requirements	5884

A.17.3.2.3	SA: RRC Connection Release with Redirection	5885

A.17.3.2.3.1	Redirection from NR in FR2 to NR in FR2	5885

A.17.3.2.3.1.1	Test Purpose and Environment	5885

A.17.3.2.3.1.2	Test Parameters	5885

A.17.3.2.3.1.3	Test Requirements	5889

A.17.4	Timing	5889

A.17.4.1	UE transmit timing	5889

A.17.4.1.1	NR UE Transmit Timing Test for FR2	5889

A.17.4.1.1.1	Test Purpose and environment	5889

A.17.4.1.1.2	Test requirements	5892

A.17.4.2	UE timer accuracy	5893

A.17.4.3	Timing advance	5893

A.17.4.3.1	SA FR2 timing advance adjustment accuracy	5893

A.17.4.3.1.1	Test Purpose and Environment	5893

A.17.4.3.1.2	Test Parameters	5893

A.17.4.3.1.3 Test Requirements	5896

A.17.5	Signaling characteristics for RedCap	5896

A.17.5.1	Radio link Monitoring for RedCap	5896

A.17.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5896

A.17.5.1.1.1	Test Purpose and Environment	5896

A.17.5.1.1.2	Test Requirements	5899

A.17.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	5900

A.17.5.1.2.1	Test Purpose and Environment	5900

A.17.5.1.2.2	Test Requirements	5904

A.17.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5905

A.17.5.1.3.1	Test Purpose and Environment	5905

A.17.5.1.3.2	Test Requirements	5908

A.17.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	5908

A.17.5.1.4.1	Test Purpose and Environment	5908

A.17.5.1.4.2	Test Requirements	5913

A.17.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5913

A.17.5.1.5.1	Test Purpose and Environment	5913

A.17.5.1.5.2	Test Requirements	5918

A.17.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	5918

A.17.5.1.6.1	Test Purpose and Environment	5918

A.17.5.1.6.2	Test Requirements	5922

A.17.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5922

A.17.5.1.7.1	Test Purpose and Environment	5922

A.17.5.1.7.2	Test Requirements	5926

A.17.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	5926

A.17.5.1.8.1	Test Purpose and Environment	5926

A.17.5.1.8.2	Test Requirements	5931

A.17.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	5931

A.17.5.1.9.1	Test Purpose and Environment	5931

A.17.5.1.9.2	Test Requirements	5934

A.17.5.2	Beam Failure Detection and Link recovery procedures	5934

A.17.5.2.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	5934

A.17.5.2.1.1	Test Purpose and Environment	5934

A.17.5.2.1.2	Test Requirements	5939

A.17.5.2.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	5940

A.17.5.2.2.1	Test Purpose and Environment	5940

A.17.5.2.2.2	Test Requirements	5944

A.17.5.2.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	5945

A.17.5.2.3.1	Test Purpose and Environment	5945

A.17.5.2.3.2	Test Requirements	5949

A.17.5.2.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	5949

A.17.5.2.4.1	Test Purpose and Environment	5949

A.17.5.2.4.2	Test Requirements	5953

A.17.5.2.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	5953

A.17.5.2.5.1	Test Purpose and Environment	5953

A.17.5.2.5.2	Test Requirements	5957

A.17.5.3	Active BWP switch for RedCap	5957

A.17.5.3.1	DCI-based and Timer-based Active BWP Switch	5957

A.17.5.3.1.1	NR FR2 DL active BWP switch with non-DRX in SA	5957

A.17.5.3.1.1.1	Test Purpose and Environment	5957

A.17.5.3.1.1.2	Test Requirements	5960

A.17.5.3.2	RRC-based Active BWP Switch	5960

A.17.5.3.2.1	NR FR2 DL active BWP switch of PCell with non-DRX in SA	5960

A.17.5.3.2.1.1	Test Purpose and Environment	5960

A.17.5.3.2.1.2	Test Requirements	5963

A.17.5.4	Active TCI state switch delay	5964

A.17.5.4.1	MAC-CE based active TCI state switch	5964

A.17.5.4.1.1	NR PCell FR2 active TCI state switch for a known TCI state	5964

A.17.5.4.1.1.1	Test Purpose and Environment	5964

A.17.5.4.1.1.2	Test Requirements	5967

A.17.5.4.2	RRC based active TCI state switch	5967

A.17.5.4.2.1	NR PCell FR2 active TCI state switch for a known TCI state	5967

A.17.5.4.2.1.1	Test Purpose and Environment	5967

A.17.5.4.2.1.2	Test Requirements	5971

A.17.5.5	Uplink spatial relation switch delay	5971

A.17.5.5.1	MAC-CE based Spatial Relation switch	5971

A.17.5.5.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	5971

A.17.5.5.1.1.1	Test Purpose and Environment	5971

A.17.5.5.1.1.2	Test Requirements	5974

A.17.5.5.2	RRC based spatial relation switch	5974

A.17.5.5.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	5974

A.17.5.5.2.1.2	Test Requirements	5977

A.17.5.6	UE specific CBW change	5977

A.17.5.6.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	5977

A.17.5.6.1.1	Test Purpose and Environment	5977

A.17.5.6.1.2	Test Requirements	5980

A.17.6	Measurement procedure for RedCap	5981

A.17.6.1	Intra-frequency Measurements	5981

A.17.6.1.1	SA event triggered reporting test without gap under non-DRX	5981

A.17.6.1.1.1	Test purpose and Environment	5981

A.17.6.1.1.2	Test Requirements	5984

A.17.6.1.2	SA event triggered reporting test without gap under DRX	5984

A.17.6.1.2.1	Test purpose and Environment	5984

A.7.6.1.2.2	Test Requirements	5984

A.17.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	5985

A.17.6.1.3.1	Test purpose and Environment	5985

A.17.6.1.3.2	Test Requirements	5989

A.17.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	5989

A.17.6.1.4.1	Test purpose and Environment	5989

A.17.6.1.4.2	Test Requirements	5992

A.17.6.2	Inter-frequency Measurements	5993

A.17.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	5993

A.17.6.2.1.1	Test Purpose and Environment	5993

A.17.6.2.1.2	Test Requirements	5996

A.17.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	5996

A.17.6.2.2.1	Test Purpose and Environment	5996

A.17.6.2.2.2	Test Requirements	6000

A.17.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	6000

A.17.6.2.3.1	Test Purpose and Environment	6000

A.17.6.2.3.2	Test Requirements	6004

A.17.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2) for 2 RX UE	6004

A.17.6.2.4.1	Test Purpose and Environment	6004

A.17.6.2.4.2	Test Requirements	6008

A.17.6.3	L1-RSRP measurement for beam reporting	6009

A.17.6.3.1	SSB based L1-RSRP measurement when DRX is not used	6009

A.17.6.3.1.1	Test Purpose and Environment	6009

A.17.6.3.1.2	Test parameters	6009

A.17.6.3.1.3	Test Requirements	6009

A.17.6.3.2	SSB based L1-RSRP measurement when DRX is used	6009

A.17.6.3.2.1	Test Purpose and Environment	6009

A.17.6.3.2.2	Test parameters	6009

A.17.6.3.2.3	Test Requirements	6011

A.17.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	6011

A.17.6.3.3.1	Test Purpose and Environment	6011

A.17.6.3.3.2	Test parameters	6012

A.17.6.3.3.3	Test Requirements	6014

A.17.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	6015

A.17.6.3.4.1	Test Purpose and Environment	6015

A.17.6.3.4.2	Test parameters	6015

A.7.6.3.3.3	Test Requirements	6017

A.17.6.4.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2) for 2 RX UE	6018

A.17.6.4.1.1	Test Purpose and Environment	6018

A.17.6.4.1.2	Test Requirements	6021

A.17.7	Measurement Performance requirements	6021

A.17.7.1	SS-RSRP	6021

A.17.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6021

A.17.7.1.1.1	Test Purpose and Environment	6021

A.17.7.1.1.2	Test parameters	6021

A.17.7.1.1.3	Test Requirements	6025

A.17.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	6026

A.17.7.1.2.1	Test Purpose and Environment	6026

A.17.7.1.2.2	Test parameters	6026

A.17.7.1.2.3	Test Requirements	6030

A.17.7.2	SS-RSRQ	6031

A.17.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	6031

A.17.7.2.1.1	Test Purpose and Environment	6031

A.17.7.2.1.2	Test Parameters	6031

A.17.7.2.1.3	Test Requirements	6033

A.17.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell for 2 Rx UE	6033

A.17.7.2.2.1	Test Purpose and Environment	6033

A.17.7.2.2.2	Test parameters	6033

A.17.7.2.2.3	Test Requirements	6035

A.17.7.2.3	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	6035

A.17.7.3	L1-RSRP measurement for beam reporting	6035

A.17.7.3.1	SSB based L1-RSRP measurement	6035

A.17.7.3.1.1	Test Purpose and Environment	6035

A.17.7.3.1.2	Test parameters	6036

A.17.7.3.1.3	Test Requirements	6036

A.17.7.3.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	6036

A.17.7.3.2.1	Test Purpose and Environment	6036

A.17.7.3.2.2	Test parameters	6036

A.17.7.3.2.3	Test Requirements	6037

A.17.7.4	SS-SINR	6037

A.17.7.4	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell for 2Rx UE	6037

A.17.7.4.1.1	Test Purpose and Environment	6037

A.17.7.4.1.2	Test parameters	6037

A.17.7.4.1.3	Test Requirements	6039

A.18	E-UTRA standalone tests for NR RRM for RedCap	6039

A.18.1	RRC_IDLE state mobility	6039

A.18.1.1	Inter-RAT NR Cell re-selection	6039

A.18.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	6039

A.18.1.1.1.1	Test Purpose and Environment	6039

A.18.2.1.1.2	Test Requirements	6044

A.18.2	RRC_CONNECTED state mobility	6044

A.18.2.1	Handover	6044

A.18.2.1.1	E-UTRAN - NR handover in FR1	6044

A.18.2.1.1.1	Test Purpose and Environment	6044

A.18.2.1.1.2	Test Requirements	6049

A.18.2.2	RRC connection release with redirection	6049

A.18.2.2.1	Redirection from E-UTRA to NR FR1 for redcap UE	6049

A.18.2.2.1.1	Test Purpose and Environment	6049

A.18.2.2.1.2	Test Parameters	6049

A.18.2.2.1.3	Test Requirements	6056

A.18.3	Measurement procedure	6056

A.18.3.1	E-UTRA – NR Inter-RAT Measurements	6056

A.18.3.1.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	6056

A.18.3.1.1.1	Test Purpose and Environment	6056

A.18.3.1.1.2	Test Requirements	6062

A.18.3.1.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	6062

A.18.3.1.2.1	Test Purpose and Environment	6062

A.18.3.1.2.2	Test Requirements	6068

A.18.3.1.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	6068

A.18.3.1.3.1	Test Purpose and Environment	6068

A.18.3.1.3.2	Test Requirements	6074

A.18.3.1.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	6074

A.18.3.1.4.1	Test Purpose and Environment	6074

A.18.3.1.4.2	Test Requirements	6080

A.18.3.1.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	6080

A.18.3.1.5.1	Test Purpose and Environment	6080

A.18.3.1.5.2	Test Requirements	6082

A.18.3.1.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	6083

A.18.3.1.6.1	Test Purpose and Environment	6083

A.18.3.1.6.2	Test Requirements	6085

A.18.3.1.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	6086

A.18.3.1.7.1	Test Purpose and Environment	6086

A.18.3.1.7.2	Test Requirements	6088

A.18.3.1.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	6089

A.18.3.1.8.1	Test Purpose and Environment	6089

A.18.3.1.8.2	Test Requirements	6091

A.19	NR standalone tests for ATG	6092

A.19.1	RRC_IDLE state mobility	6092

A.19.1.1	Cell reselection to FR1 intra-frequency NR case	6092

A.19.1.1.1	Test Purpose and Environment	6092

A.19.1.1.2	Test Parameters	6092

A.19.1.1.3	Test Requirements	6093

A.19.1.2	Cell reselection to FR1 inter-frequency NR case	6094

A.19.1.2.1	Test Purpose and Environment	6094

A.19.1.2.2	Test Parameters	6094

A.19.1.2.3	Test Requirements	6097

A.19.1.3	Cell reselection to FR1 inter-frequency NR case for UE configured with hs-ATG-cellReselectionSet-r18	6097

A.19.1.3.1	Test Purpose and Environment	6097

A.19.1.3.2	Test Parameters	6097

A.19.1.2.3	Test Requirements	6101

A.19.2	RRC_CONNECTED state mobility	6101

A.19.2.1	Handover	6101

A.19.2.1.1	Intra-frequency handover from FR1 to FR1; known target cell	6101

A19.2.1.1.1	Test Purpose and Environment	6101

A.19.2.1.1.2	Test Parameters	6101

A.19.2.1.2.3	Test Requirements	6102

A.19.2.1.2	Inter-frequency handover from FR1 to FR1; unknown target cell	6102

A.19.2.1.2.1	Test Purpose and Environment	6102

A.19.2.1.2.2	Test Parameters	6102

A.19.2.1.2.3	Test Requirements	6103

A.19.2.2	Conditional Handover	6104

A.19.2.2.1	Intra-frequency distance-based conditional Handover from FR1 to FR1	6104

A.19.2.2.1.1	Test Purpose and Environment	6104

A.19.2.2.1.2	Test Parameters	6104

A.19.2.2.1.3	Test Requirements	6108

A.19.2.2.2	Inter-frequency distance-based conditional Handover from FR1 to FR1	6109

A.19.2.2.2.1	Test Purpose and Environment	6109

A.19.2.2.2.2	Test Parameters	6109

A.19.2.2.2.3	Test Requirements	6113

A.19.2.3	RRC Connection Mobility Control	6114

A.19.2.3.1	SA: RRC Re-establishment	6114

A.19.2.3.1.1	Intra-frequency RRC Re-establishment in FR1 for ATG	6114

A.19.2.3.1.1.1	Test Purpose and Environment	6114

A.19.2.3.1.1.2 Test Requirements	6115

A.19.2.3.1.2	Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG	6115

A.19.2.3.1.2.1	Test Purpose and Environment	6115

A.19.2.3.1.2.2	Test Requirements	6117

A.19.2.3.2	Random Access for ATG UE	6118

A.19.2.3.2.1.1	Test Purpose and Environment	6118

A.19.2.3.2.1.2	Test Requirements	6119

A.19.2.3.2.2.1	Test Purpose and Environment	6119

A.19.2.3.2.2.2	Test Requirements	6120

A.19.2.3.2.3	2-step RA type contention based random access test in FR1 for NR standalone	6120

A.19.2.3.2.3.1	Test Purpose and Environment	6120

A.19.2.3.2.3.2	Test Requirements	6120

A.19.2.3.2.4	2-step RA type non-contention based test in FR1 for NR standalone	6121

A.19.2.3.2.4.1	Test Purpose and Environment	6121

A.19.2.3.2.4.2	Test Requirements	6121

A.19.2.3.3.1.1	Test Purpose and Environment	6121

A.19.2.3.3.1.2	Test Parameters	6121

A.19.2.3.3.1.3	Test Requirements	6122

A.19.3	Timing	6123

A.19.3.1	UE transmit timing	6123

A.19.3.1.1	ATG UE Transmit Timing Test for FR1	6123

A.19.3.1.1.1	Test Purpose and environment	6123

A.19.3.1.1.2	Test requirements	6126

A.19.3.2	UE timer accuracy	6127

A.19.3.3	Timing advance	6127

A.19.3.3.1	SA FR1 timing advance adjustment accuracy	6127

A.19.3.3.1.1	Test Purpose and Environment	6127

A.19.3.3.1.2	Test Parameters	6127

A.19.3.3.1.3	Test Requirements	6128

A.19.4	Signalling characteristics	6128

A.19.4.1	Radio link Monitoring	6128

A.19.4.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6128

A.19.4.1.1.1	Test Purpose and Environment	6128

A.19.4.1.1.2	Test Requirements	6134

A.19.4.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	6134

A.19.4.1.2.1	Test Purpose and Environment	6134

A.19.4.1.2.2	Test Requirements	6140

A.19.4.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	6140

A.19.4.1.3.1	Test Purpose and Environment	6140

A.19.4.1.3.2	Test Requirements	6146

A.19.4.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	6146

A.19.4.1.4.1	Test Purpose and Environment	6146

A.19.4.1.4.2	Test Requirements	6152

A.19.4.2	Beam Failure Detection and Link recovery procedures	6152

A.19.4.2.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	6152

A.19.4.2.1.1	Test Purpose and Environment	6152

A.19.4.2.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	6159

A.19.4.2.2.1	Test Purpose and Environment	6159

A.19.4.2.2.2	Test Requirements	6164

A.19.4.3	Active BWP switch	6165

A.19.4.3.1	DCI-based and Timer-based Active BWP Switch	6165

A.19.4.3.1.1	NR FR1 DL active BWP switch with non-DRX in SA	6165

A.19.4.3.2	RRC-based Active BWP Switch	6169

A.19.4.3.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	6169

A.19.4.4	UE specific CBW change	6174

A19.4.4.1	UE specific CBW change on PCell in FR1 in non-DRX	6174

A19.4.4.1.1	Test Purpose and Environment	6174

A.19.4.4.1.2	Test Requirements	6179

A.19.4.5	Pathloss reference signal switching delay	6179

A.19.4.5.1	MAC-CE based pathloss reference signal switch delay	6179

A.19.4.5.1.1	Test Purpose and Environment	6179

A.19.4.5.1.2	Test Requirements	6182

A.19.5	Measurement procedure	6183

A.19.5.1	Intra-frequency Measurements	6183

A.19.5.1.1	SA event triggered reporting tests without gap without SSB index reading under non-DRX	6183

A.19.5.1.1.1	Test purpose and Environment	6183

A.19.5.1.1.2	Test parameters	6183

A.19.5.1.1.3	Test Requirements	6184

A.19.5.1.2	SA event triggered reporting tests with per-UE gaps under non-DRX	6184

A.19.5.1.2.1	Test purpose and Environment	6184

A.19.5.1.2.2	Test parameters	6184

A.19.5.1.2.3	Test Requirements	6184

A.19.5.1.3	SA event triggered reporting tests without gap under non-DRX with SSB index reading	6185

A.19.5.1.3.1	Test purpose and Environment	6185

A.19.5.1.3.2	Test parameters	6185

A.19.5.1.3.3	Test Requirements	6185

A.19.5.1.4	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	6186

A.19.5.1.4.1	Test purpose and Environment	6186

A.19.5.1.4.2	Test parameters	6186

A.19.5.1.4.3	Test Requirements	6186

A.19.5.2	Inter-frequency Measurements	6187

A.19.5.2.1.2	Test parameters	6187

A.19.5.2.1.3	Test Requirements	6189

A.19.5.2.2.2	Test parameters	6189

A.19.5.2.3.2	Test parameters	6190

A.19.5.2.3.3	Test Requirements	6191

A.19.5.3	L1-RSRP measurement for beam reporting for ATG	6191

A.19.5.3.1	SSB based L1-RSRP measurement when DRX is not used	6191

A.19.5.3.1.1	Test Purpose and Environment	6191

A.19.5.3.1.2	Test parameters	6191

A.19.5.3.1.3	Test Requirements	6192

A.19.5.3.2	CSI-RS based L1-RSRP measurement when DRX is not used	6192

A.19.5.3.2.1	Test Purpose and Environment	6192

A.19.5.3.2.2	Test parameters	6192

A.19.5.3.2.3	Test Requirements	6192

A.19.5.4	L1-SINR measurement for beam reporting for ATG	6193

A.19.5.4.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	6193

A.19.5.4.1.3	Test Requirements	6193

A.19.5.4.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	6193

A.19.5.4.2.1	Test Purpose and Environment	6193

A.19.5.4.2.2	Test parameters	6194

A.19.5.4.2.3	Test Requirements	6194

A.19.5.4.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	6194

A.19.5.4.3.1	Test Purpose and Environment	6194

A.19.5.4.3.2	Test parameters	6195

A.19.5.4.3.3	Test Requirements	6195

A.19.5.5	NR measurements with autonomous gaps for ATG	6195

A.19.5.5.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	6195

A.19.5.5.1.1	Test Purpose and Environment	6195

A.19.5.5.1.2	Test Parameters	6195

A.19.5.5.1.3	Test Requirements	6196

A.19.6	Measurement Performance requirements	6196

A.19.6.1	SS-RSRP for ATG UE	6196

A.19.6.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6196

A.19.6.1.1.1	Test Purpose and Environment	6196

A.19.6.1.1.2	Test parameters	6196

A.19.6.1.1.3	Test Requirements	6197

A.19.6.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6197

A.19.6.1.2.1	Test Purpose and Environment	6197

A.19.6.1.2.2	Test parameters	6197

A.19.6.1.2.3	Test Requirements	6198

A.19.6.2	SS-RSRQ for ATG UE	6198

A.19.6.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6198

A.19.6.2.1.1	Test Purpose and Environment	6198

A.19.6.2.1.2	Test Parameters	6198

A.19.6.2.1.3	Test Requirements	6198

A.19.6.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6199

A.19.6.2.2.1	Test Purpose and Environment	6199

A.19.6.2.2.2	Test Parameters	6199

A.19.6.2.2.3	Test Requirements	6199

A.19.6.3	SS-SINR for ATG UE	6199

A.19.6.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6199

A.19.6.3.1.1	Test Purpose and Environment	6199

A.19.6.3.1.2	Test Parameters	6200

A.19.6.3.1.3	Test Requirements	6200

A.19.6.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6200

A.19.6.3.2.1	Test Purpose and Environment	6200

A.19.6.3.2.2	Test Parameters	6200

A.19.6.3.2.3	Test Requirements	6201

A.19.6.4	L1-RSRP measurement for beam reporting for ATG UE	6201

A.19.6.4.1	SSB based L1-RSRP measurement	6201

A.19.6.4.1.1	Test Purpose and Environment	6201

A.19.6.4.1.2	Test parameters	6201

A.19.6.4.1.3	Test Requirements	6202

A.19.6.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	6202

A.19.6.4.2.1	Test Purpose and Environment	6202

A.19.6.4.2.2	Test parameters	6202

A.19.6.4.2.3	Test Requirements	6202

A.19.6.5	L1-SINR measurement for beam reporting based CMR for ATG UE	6203

A.19.6.5.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	6203

A.19.6.5.1.1	Test Purpose and Environment	6203

A.19.6.5.1.2	Test parameters	6203

A.19.6.5.1.3	Test Requirements	6203

A.19.6.5.2	L1-SINR measurement with SSB based CMR and dedicated IMR	6204

A.19.6.5.2.1	Test Purpose and Environment	6204

A.19.6.5.2.2	Test parameters	6204

A.19.6.5.2.3	Test Requirements	6204

A.19.6.5.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	6204

A.19.6.5.3.1	Test Purpose and Environment	6204

A.19.6.5.3.2	Test parameters	6205

A.19.6.5.3.3	Test Requirements	6205

A.19.6.6	CSI-RSRP for ATG UE	6205

A.19.6.6.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6205

A.19.6.6.1.1	Test Purpose and Environment	6205

A.19.6.6.1.2	Test parameters	6205

A.19.6.6.1.3	Test Requirements	6206

A.19.6.6.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	6206

A.19.6.6.2.1	Test Purpose and Environment	6206

A.19.6.6.2.2	Test parameters	6206

A.19.6.6.2.3	Test Requirements	6207

A.19.6.7	CSI-RSRQ for ATG UE	6207

A.19.6.7.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6207

A.19.6.7.1.1	Test Purpose and Environment	6207

A.19.6.7.1.2	Test Parameters	6207

A.19.6.7.1.3	Test Requirements	6208

A.19.6.7.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6208

A.19.6.7.2.1	Test Purpose and Environment	6208

A.19.6.7.2.2	Test Parameters	6208

A.19.6.7.2.3	Test Requirements	6208

A.19.6.8	CSI-SINR for ATG UE	6209

A.19.6.8.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6209

A.19.6.8.1.1	Test Purpose and Environment	6209

A.19.6.8.1.2	Test Parameters	6209

A.19.6.8.1.3	Test Requirements	6209

A.19.6.8.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	6209

A.19.6.8.2.1	Test Purpose and Environment	6209

A.19.6.8.2.2	Test Parameters	6209

A.19.6.8.2.3	Test Requirements	6210

Annex B (normative): Conditions for RRM requirements applicability for operating bands	6212

B.1	Conditions for NR RRC_IDLE state mobility	6212

B.1.1	Introduction	6212

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	6212

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	6213

B.1.4	Conditions for measurements on NR intra-frequency cells for cell re-selection for RedCap	6213

B.1.5	Conditions for measurements on NR inter-frequency cells for cell re-selection for RedCap	6215

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	6215

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	6216

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	6216

B.2.1	Introduction	6216

B.2.1.1	General	6216

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	6216

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	6216

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	6216

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	6217

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	6218

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	6218

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	6218

B.2.1.5.2	Gain to SS-RSRP measurement point for different frequency	6219

B.2.1.5.3	Alignment of Rough beam to Rx beam Peak	6219

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	6219

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	6219

B.2.2	Conditions for NR intra-frequency measurements	6220

B.2.3	Conditions for NR inter-frequency measurements	6221

B.2.4	Conditions for NR L1-RSRP reporting	6224

B.2.4.1	Conditions for SSB based L1-RSRP reporting	6224

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	6226

B.2.5	Conditions for RRC connection release with redirection to NR	6227

B.2.6	Void	6228

B.2.6.1	Void	6228

B.2.6.2	Void	6228

B.2.7	Conditions for SRS-RSRP measurements	6228

B.2.8	Conditions for NR L1-SINR reporting	6229

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	6229

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	6230

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	6230

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	6230

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	6232

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	6232

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	6233

B.2.9	Conditions for NR intra-frequency measurements under CCA	6234

B.2.10	Conditions for NR inter-frequency measurements under CCA	6234

B.2.11	Conditions for NR L1-RSRP reporting under CCA	6234

B.2.11.1	Conditions for SSB based L1-RSRP reporting	6234

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	6235

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	6236

B.2.14	Conditions for NR PRS-based measurements	6238

B.2.15	Conditions for NR intra-frequency measurements for RedCap	6240

B.2.16	Conditions for NR inter-frequency measurements for RedCap	6241

B.2.17	Conditions for NR intra-frequency measurements for satellite access	6242

B.2.18	Conditions for NR inter-frequency measurements for satellite access	6243

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	6243

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	6243

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	6243

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	6244

B.3	RRM Requirements Exceptions	6244

B.3.1	Introduction	6244

B.3.2	Receiver sensitivity relaxation for CA	6244

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	6244

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	6244

B.3.2.2.1	Inter-band carrier aggregation	6244

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	6244

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	6245

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	6245

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	6245

B.3.2.4.1	Intra-band contiguous carrier aggregation	6245

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	6245

B.3.3	Receiver sensitivity relaxation for DC	6245

B.3.3.1	Receiver sensitivity relaxation for EN-DC	6245

B.3.3.2	Receiver sensitivity relaxation for NE-DC	6245

B.3.4	Receiver sensitivity relaxation for SUL	6245

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	6245

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	6246

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	6246

B.4	Conditions for V2X	6246

B.4.1	Test parameters for GNSS signals	6246

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	6246

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	6247

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	6247

B.4.5	Conditions for PSBCH-RSRP Accuracy Requirements under CCA	6247

B.4.6	Conditions for Selection/Reselection to Intra-frequency SyncRef UE under CCA	6248

B.4.7	Conditions for L1 SL-RSRP Accuracy Requirements under CCA	6248

B.5	High level test procedure for SAN RRM tests	6248

Annex C (informative): Change history	6250