3GPP TS 38.212 V15.11.0 (2021-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Multiplexing and channel coding

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, New Radio, Layer 1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Mapping to physical channels	7

4.1	Uplink	7

4.2	Downlink	8

5	General procedures	8

5.1	CRC calculation	8

5.2	Code block segmentation and code block CRC attachment	9

5.2.1	Polar coding	9

5.2.2	Low density parity check coding	9

5.3	Channel coding	11

5.3.1	Polar coding	11

5.3.1.1	Interleaving	12

5.3.1.2	Polar encoding	13

5.3.2	Low density parity check coding	17

5.3.3	Channel coding of small block lengths	24

5.3.3.1	Encoding of 1-bit information	24

5.3.3.2	Encoding of 2-bit information	24

5.3.3.3	Encoding of other small block lengths	24

5.4	Rate matching	25

5.4.1	Rate matching for Polar code	25

5.4.1.1	Sub-block interleaving	25

5.4.1.2	Bit selection	26

5.4.1.3	Interleaving of coded bits	27

5.4.2	Rate matching for LDPC code	28

5.4.2.1	Bit selection	28

5.4.2.2	Bit interleaving	30

5.4.3	Rate matching for channel coding of small block lengths	31

5.5	Code block concatenation	31

6	Uplink transport channels and control information	31

6.1	Random access channel	31

6.2	Uplink shared channel	32

6.2.1	Transport block CRC attachment	32

6.2.2	LDPC base graph selection	32

6.2.3	Code block segmentation and code block CRC attachment	32

6.2.4	Channel coding of UL-SCH	32

6.2.5	Rate matching	32

6.2.6	Code block concatenation	33

6.2.7	Data and control multiplexing	33

6.3	Uplink control information	43

6.3.1	Uplink control information on PUCCH	43

6.3.1.1	UCI bit sequence generation	43

6.3.1.1.1	HARQ-ACK/SR only	43

6.3.1.1.2	CSI only	43

6.3.1.1.3	HARQ-ACK/SR and CSI	50

6.3.1.2	Code block segmentation and CRC attachment	51

6.3.1.2.1	UCI encoded by Polar code	51

6.3.1.2.2	UCI encoded by channel coding of small block lengths	52

6.3.1.3	Channel coding of UCI	52

6.3.1.3.1	UCI encoded by Polar code	52

6.3.1.3.2	UCI encoded by channel coding of small block lengths	52

6.3.1.4	Rate matching	52

6.3.1.4.1	UCI encoded by Polar code	53

6.3.1.4.2	UCI encoded by channel coding of small block lengths	53

6.3.1.5	Code block concatenation	54

6.3.1.6	Multiplexing of coded UCI bits to PUCCH	54

6.3.2	Uplink control information on PUSCH	56

6.3.2.1	UCI bit sequence generation	56

6.3.2.1.1	HARQ-ACK	56

6.3.2.1.2	CSI	57

6.3.2.2	Code block segmentation and CRC attachment	60

6.3.2.2.1	UCI encoded by Polar code	60

6.3.2.2.2	UCI encoded by channel coding of small block lengths	60

6.3.2.3	Channel coding of UCI	60

6.3.2.3.1	UCI encoded by Polar code	60

6.3.2.3.2	UCI encoded by channel coding of small block lengths	60

6.3.2.4	Rate matching	61

6.3.2.4.1	UCI encoded by Polar code	61

6.3.2.4.1.1	HARQ-ACK	61

6.3.2.4.1.2	CSI part 1	62

6.3.2.4.1.3	CSI part 2	65

6.3.2.4.2	UCI encoded by channel coding of small block lengths	66

6.3.2.4.2.1	HARQ-ACK	66

6.3.2.4.2.2	CSI part 1	67

6.3.2.4.2.3	CSI part 2	67

6.3.2.5	Code block concatenation	67

6.3.2.6	Multiplexing of coded UCI bits to PUSCH	67

7	Downlink transport channels and control information	67

7.1	Broadcast channel	67

7.1.1	PBCH payload generation	68

7.1.2	Scrambling	69

7.1.3	Transport block CRC attachment	70

7.1.4	Channel coding	70

7.1.5	Rate matching	70

7.2	Downlink shared channel and paging channel	70

7.2.1	Transport block CRC attachment	70

7.2.2	LDPC base graph selection	70

7.2.3	Code block segmentation and code block CRC attachment	71

7.2.4	Channel coding	71

7.2.5	Rate matching	71

7.2.6	Code block concatenation	71

7.3	Downlink control information	71

7.3.1	DCI formats	71

7.3.1.0	DCI size alignment	72

7.3.1.1	DCI formats for scheduling of PUSCH	74

7.3.1.1.1	Format 0_0	74

7.3.1.1.2	Format 0_1	76

7.3.1.2	DCI formats for scheduling of PDSCH	88

7.3.1.2.1	Format 1_0	88

7.3.1.2.2	Format 1_1	91

7.3.1.3	DCI formats for other purposes	98

7.3.1.3.1	Format 2_0	98

7.3.1.3.2	Format 2_1	98

7.3.1.3.3	Format 2_2	98

7.3.1.3.4	Format 2_3	99

7.3.2	CRC attachment	99

7.3.3	Channel coding	100

7.3.4	Rate matching	100

Annex <A> (informative): Change history	101

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the coding, multiplexing and mapping to physical channels for 5G NR.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.201: "NR; Physical Layer – General Description"

[3]	3GPP TS 38.202: "NR; Services provided by the physical layer"

[4]	3GPP TS 38.211: "NR; Physical channels and modulation"

[5]	3GPP TS 38.213: "NR; Physical layer procedures for control"

[6]	3GPP TS 38.214: "NR; Physical layer procedures for data"

[7]	3GPP TS 38.215: "NR; Physical layer measurements"

[8]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BCH	Broadcast channel

CBG	Code block group

CBGTI	Code block group transmission information

CORESET	Control resource set

CQI	Channel quality indicator

CRC	Cyclic redundancy check

CRI	CSI-RS resource indicator

CSI	Channel state information

CSI-RS	CSI reference signal

DAI	Downlink assignment index

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

DMRS	Dedicated demodulation reference signal

HARQ	Hybrid automatic repeat request

HARQ-ACK	Hybrid automatic repeat request acknowledgement

LDPC	Low density parity check

LI	Layer indicator

MCS	Modulation and coding scheme

OFDM	Orthogonal frequency division multiplex

PBCH	Physical broadcast channel

PCH	Paging channel

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PMI	Precoding matrix indicator

PRB	Physical resource block

PRACH	Physical random access channel

PTRS	Phase-tracking reference signal

PUCCH	Physical uplink control channel

PUSCH	Physical uplink shared channel

RACH	Random access channel

RI	Rank indicator

RSRP	Reference signal received power

SFN	System frame number

SR	Scheduling request

SRS	Sounding reference signal

SS	Synchronisation signal

SUL	Supplementary uplink

TPC	Transmit power control

TrCH	Transport channel

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

VRB	Virtual resource block

ZP CSI-RS	Zero power CSI-RS

# 4	Mapping to physical channels

## 4.1	Uplink

Table 4.1-1 specifies the mapping of the uplink transport channels to their corresponding physical channels. Table 4.1-2 specifies the mapping of the uplink control channel information to its corresponding physical channel.

Table 4.1-1

| TrCH | Physical Channel |
| --- | --- |
| UL-SCH | PUSCH |
| RACH | PRACH |

Table 4.1-2

| Control information | Physical Channel |
| --- | --- |
| UCI | PUCCH, PUSCH |

## 4.2	Downlink

Table 4.2-1 specifies the mapping of the downlink transport channels to their corresponding physical channels. Table 4.2-2 specifies the mapping of the downlink control channel information to its corresponding physical channel.

Table 4.2-1

| TrCH | Physical Channel |
| --- | --- |
| DL-SCH | PDSCH |
| BCH | PBCH |
| PCH | PDSCH |

Table 4.2-2

| Control information | Physical Channel |
| --- | --- |
| DCI | PDCCH |

# 5	General procedures

Data and control streams from/to MAC layer are encoded /decoded to offer transport and control services over the radio transmission link. Channel coding scheme is a combination of error detection, error correcting, rate matching, interleaving and transport channel or control information mapping onto/splitting from physical channels.

## 5.1	CRC calculation

Denote the input bits to the CRC computation by , and the parity bits by , where  is the size of the input sequence and  is the number of parity bits. The parity bits are generated by one of the following cyclic generator polynomials:

-	 for a CRC length ;

-	 for a CRC length ;

-	 for a CRC length ;

-	 for a CRC length ;

-	 for a CRC length ;

-	 for a CRC length .

The encoding is performed in a systematic form, which means that in GF(2), the polynomial:

yields a remainder equal to 0 when divided by the corresponding CRC generator polynomial.

The bits after CRC attachment are denoted by , where . The relation between  and  is:

for

for .

## 5.2	Code block segmentation and code block CRC attachment

### 5.2.1	Polar coding

The input bit sequence to the code block segmentation is denoted by , where .

if

Number of code blocks: ;

else

Number of code blocks:

end if

;

for  to

;

end for

for  to

;

end for

;

for  to

for  to

;

;

end for

The sequence  is used to calculate the CRC parity bits  according to Clause 5.1 with a generator polynomial of length .

for  to

;

end for

end for

The value of  is no larger than 1706.

### 5.2.2	Low density parity check coding

The input bit sequence to the code block segmentation is denoted by , where . If  is larger than the maximum code block size , segmentation of the input bit sequence is performed and an additional CRC sequence of  bits is attached to each code block.

For LDPC base graph 1, the maximum code block size is:

-	.

For LDPC base graph 2, the maximum code block size is:

-	.

Total number of code blocks C is determined by:

if

Number of code blocks:

else

Number of code blocks: .

end if

The bits output from code block segmentation are denoted by , where  is the code block number, and  is the number of bits for the code block number .

The number of bits  in each code block is calculated as:

;

For LDPC base graph 1,

.

For LDPC base graph 2,

if

;

elseif

;

elseif

;

else

;

end if

find the minimum value of  in all sets of lifting sizes in Table 5.3.2-1, denoted as , such that , and set  for LDPC base graph 1 and  for LDPC base graph 2;

The bit sequence  is calculated as:

;

for  to

for  to

;

;

end for

if

The sequence  is used to calculate the CRC parity bits  according to Clause 5.1 with the generator polynomial .

for  to

;

end for

end if

for  to 	-- Insertion of filler bits

;

end for

end for

## 5.3	Channel coding

Usage of coding scheme for the different types of TrCH is shown in table 5.3-1. Usage of coding scheme for the different control information types is shown in table 5.3-2.

Table 5.3-1: Usage of channel coding scheme for TrCHs

| TrCH | Coding scheme |
| --- | --- |
| UL-SCH | LDPC |
| DL-SCH |  |
| PCH |  |
| BCH | Polar code |

Table 5.3-2: Usage of channel coding scheme for control information

| Control Information | Coding scheme |
| --- | --- |
| DCI | Polar code |
| UCI | Block code |
|  | Polar code |

### 5.3.1	Polar coding

The bit sequence input for a given code block to channel coding is denoted by, where  is the number of bits to encode. After encoding the bits are denoted by, where  and the value of  is determined by the following:

Denote by  the rate matching output sequence length as given in Clause 5.4.1;

If  and

;

else

;

end if

;

;

where .

UE is not expected to be configured with , where   is the number of parity check bits defined in Clause 5.3.1.2.

#### 5.3.1.1	Interleaving

The bit sequence  is interleaved into bit sequence  as follows:

,

where the interleaving pattern  is given by the following:

if

,

else

;

for  to

if

;

;

end if

end for

end if

where  is given by Table 5.3.1.1-1 and .

Table 5.3.1.1-1: Interleaving pattern

| 0 | 0 | 28 | 67 | 56 | 122 | 84 | 68 | 112 | 33 | 140 | 38 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 29 | 69 | 57 | 123 | 85 | 73 | 113 | 36 | 141 | 144 |
| 2 | 4 | 30 | 70 | 58 | 126 | 86 | 78 | 114 | 44 | 142 | 39 |
| 3 | 7 | 31 | 71 | 59 | 127 | 87 | 84 | 115 | 47 | 143 | 145 |
| 4 | 9 | 32 | 72 | 60 | 129 | 88 | 90 | 116 | 64 | 144 | 40 |
| 5 | 14 | 33 | 76 | 61 | 132 | 89 | 92 | 117 | 74 | 145 | 146 |
| 6 | 19 | 34 | 77 | 62 | 134 | 90 | 94 | 118 | 79 | 146 | 41 |
| 7 | 20 | 35 | 81 | 63 | 138 | 91 | 96 | 119 | 85 | 147 | 147 |
| 8 | 24 | 36 | 82 | 64 | 139 | 92 | 99 | 120 | 97 | 148 | 148 |
| 9 | 25 | 37 | 83 | 65 | 140 | 93 | 102 | 121 | 100 | 149 | 149 |
| 10 | 26 | 38 | 87 | 66 | 1 | 94 | 105 | 122 | 103 | 150 | 150 |
| 11 | 28 | 39 | 88 | 67 | 3 | 95 | 107 | 123 | 117 | 151 | 151 |
| 12 | 31 | 40 | 89 | 68 | 5 | 96 | 109 | 124 | 125 | 152 | 152 |
| 13 | 34 | 41 | 91 | 69 | 8 | 97 | 112 | 125 | 131 | 153 | 153 |
| 14 | 42 | 42 | 93 | 70 | 10 | 98 | 114 | 126 | 136 | 154 | 154 |
| 15 | 45 | 43 | 95 | 71 | 15 | 99 | 116 | 127 | 142 | 155 | 155 |
| 16 | 49 | 44 | 98 | 72 | 21 | 100 | 121 | 128 | 12 | 156 | 156 |
| 17 | 50 | 45 | 101 | 73 | 27 | 101 | 124 | 129 | 17 | 157 | 157 |
| 18 | 51 | 46 | 104 | 74 | 29 | 102 | 128 | 130 | 23 | 158 | 158 |
| 19 | 53 | 47 | 106 | 75 | 32 | 103 | 130 | 131 | 37 | 159 | 159 |
| 20 | 54 | 48 | 108 | 76 | 35 | 104 | 133 | 132 | 48 | 160 | 160 |
| 21 | 56 | 49 | 110 | 77 | 43 | 105 | 135 | 133 | 75 | 161 | 161 |
| 22 | 58 | 50 | 111 | 78 | 46 | 106 | 141 | 134 | 80 | 162 | 162 |
| 23 | 59 | 51 | 113 | 79 | 52 | 107 | 6 | 135 | 86 | 163 | 163 |
| 24 | 61 | 52 | 115 | 80 | 55 | 108 | 11 | 136 | 137 |  |  |
| 25 | 62 | 53 | 118 | 81 | 57 | 109 | 16 | 137 | 143 |  |  |
| 26 | 65 | 54 | 119 | 82 | 60 | 110 | 22 | 138 | 13 |  |  |
| 27 | 66 | 55 | 120 | 83 | 63 | 111 | 30 | 139 | 18 |  |  |

#### 5.3.1.2	Polar encoding

The Polar sequence  is given by Table 5.3.1.2-1, where  denotes a bit index before Polar encoding for  and . The Polar sequence  is in ascending order of reliability , where  denotes the reliability of bit index .

For any code block encoded to  bits, a same Polar sequence  is used. The Polar sequence  is a subset of Polar sequence  with all elements  of values less than , ordered in ascending order of reliability .

Denote  as a set of bit indices in Polar sequence , and  as the set of other bit indices in Polar sequence , where  and  are given in Clause 5.4.1.1, , , and  is the number of parity check bits.

Denote  as the -th Kronecker power of matrix , where .

For a bit index  with , denote  as the -th row of  and  as the row weight of , where  is the number of ones in . Denote the set of bit indices for parity check bits as , where . A number of  parity check bits are placed in the  least reliable bit indices in . A number of  other parity check bits are placed in the bit indices of minimum row weight in , where  denotes the  most reliable bit indices in ; if there are more than  bit indices of the same minimum row weight in , the  other parity check bits are placed in the  bit indices of the highest reliability and the minimum row weight in .

Generate  according to the following:

;

if

; ; ; ; ;

for  to

; ; ; ; ; ;

if

if

;

else

;

;

;

end if

else

;

end if

end for

else

for  to

if

;

;

else

;

end if

end for

end if

The output after encoding  is obtained by . The encoding is performed in GF(2).

Table 5.3.1.2-1: Polar sequence  and its corresponding reliability

| 0 | 0 | 128 | 518 | 256 | 94 | 384 | 214 | 512 | 364 | 640 | 414 | 768 | 819 | 896 | 966 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 129 | 54 | 257 | 204 | 385 | 309 | 513 | 654 | 641 | 223 | 769 | 814 | 897 | 755 |
| 2 | 2 | 130 | 83 | 258 | 298 | 386 | 188 | 514 | 659 | 642 | 663 | 770 | 439 | 898 | 859 |
| 3 | 4 | 131 | 57 | 259 | 400 | 387 | 449 | 515 | 335 | 643 | 692 | 771 | 929 | 899 | 940 |
| 4 | 8 | 132 | 521 | 260 | 608 | 388 | 217 | 516 | 480 | 644 | 835 | 772 | 490 | 900 | 830 |
| 5 | 16 | 133 | 112 | 261 | 352 | 389 | 408 | 517 | 315 | 645 | 619 | 773 | 623 | 901 | 911 |
| 6 | 32 | 134 | 135 | 262 | 325 | 390 | 609 | 518 | 221 | 646 | 472 | 774 | 671 | 902 | 871 |
| 7 | 3 | 135 | 78 | 263 | 533 | 391 | 596 | 519 | 370 | 647 | 455 | 775 | 739 | 903 | 639 |
| 8 | 5 | 136 | 289 | 264 | 155 | 392 | 551 | 520 | 613 | 648 | 796 | 776 | 916 | 904 | 888 |
| 9 | 64 | 137 | 194 | 265 | 210 | 393 | 650 | 521 | 422 | 649 | 809 | 777 | 463 | 905 | 479 |
| 10 | 9 | 138 | 85 | 266 | 305 | 394 | 229 | 522 | 425 | 650 | 714 | 778 | 843 | 906 | 946 |
| 11 | 6 | 139 | 276 | 267 | 547 | 395 | 159 | 523 | 451 | 651 | 721 | 779 | 381 | 907 | 750 |
| 12 | 17 | 140 | 522 | 268 | 300 | 396 | 420 | 524 | 614 | 652 | 837 | 780 | 497 | 908 | 969 |
| 13 | 10 | 141 | 58 | 269 | 109 | 397 | 310 | 525 | 543 | 653 | 716 | 781 | 930 | 909 | 508 |
| 14 | 18 | 142 | 168 | 270 | 184 | 398 | 541 | 526 | 235 | 654 | 864 | 782 | 821 | 910 | 861 |
| 15 | 128 | 143 | 139 | 271 | 534 | 399 | 773 | 527 | 412 | 655 | 810 | 783 | 726 | 911 | 757 |
| 16 | 12 | 144 | 99 | 272 | 537 | 400 | 610 | 528 | 343 | 656 | 606 | 784 | 961 | 912 | 970 |
| 17 | 33 | 145 | 86 | 273 | 115 | 401 | 657 | 529 | 372 | 657 | 912 | 785 | 872 | 913 | 919 |
| 18 | 65 | 146 | 60 | 274 | 167 | 402 | 333 | 530 | 775 | 658 | 722 | 786 | 492 | 914 | 875 |
| 19 | 20 | 147 | 280 | 275 | 225 | 403 | 119 | 531 | 317 | 659 | 696 | 787 | 631 | 915 | 862 |
| 20 | 256 | 148 | 89 | 276 | 326 | 404 | 600 | 532 | 222 | 660 | 377 | 788 | 729 | 916 | 758 |
| 21 | 34 | 149 | 290 | 277 | 306 | 405 | 339 | 533 | 426 | 661 | 435 | 789 | 700 | 917 | 948 |
| 22 | 24 | 150 | 529 | 278 | 772 | 406 | 218 | 534 | 453 | 662 | 817 | 790 | 443 | 918 | 977 |
| 23 | 36 | 151 | 524 | 279 | 157 | 407 | 368 | 535 | 237 | 663 | 319 | 791 | 741 | 919 | 923 |
| 24 | 7 | 152 | 196 | 280 | 656 | 408 | 652 | 536 | 559 | 664 | 621 | 792 | 845 | 920 | 972 |
| 25 | 129 | 153 | 141 | 281 | 329 | 409 | 230 | 537 | 833 | 665 | 812 | 793 | 920 | 921 | 761 |
| 26 | 66 | 154 | 101 | 282 | 110 | 410 | 391 | 538 | 804 | 666 | 484 | 794 | 382 | 922 | 877 |
| 27 | 512 | 155 | 147 | 283 | 117 | 411 | 313 | 539 | 712 | 667 | 430 | 795 | 822 | 923 | 952 |
| 28 | 11 | 156 | 176 | 284 | 212 | 412 | 450 | 540 | 834 | 668 | 838 | 796 | 851 | 924 | 495 |
| 29 | 40 | 157 | 142 | 285 | 171 | 413 | 542 | 541 | 661 | 669 | 667 | 797 | 730 | 925 | 703 |
| 30 | 68 | 158 | 530 | 286 | 776 | 414 | 334 | 542 | 808 | 670 | 488 | 798 | 498 | 926 | 935 |
| 31 | 130 | 159 | 321 | 287 | 330 | 415 | 233 | 543 | 779 | 671 | 239 | 799 | 880 | 927 | 978 |
| 32 | 19 | 160 | 31 | 288 | 226 | 416 | 555 | 544 | 617 | 672 | 378 | 800 | 742 | 928 | 883 |
| 33 | 13 | 161 | 200 | 289 | 549 | 417 | 774 | 545 | 604 | 673 | 459 | 801 | 445 | 929 | 762 |
| 34 | 48 | 162 | 90 | 290 | 538 | 418 | 175 | 546 | 433 | 674 | 622 | 802 | 471 | 930 | 503 |
| 35 | 14 | 163 | 545 | 291 | 387 | 419 | 123 | 547 | 720 | 675 | 627 | 803 | 635 | 931 | 925 |
| 36 | 72 | 164 | 292 | 292 | 308 | 420 | 658 | 548 | 816 | 676 | 437 | 804 | 932 | 932 | 878 |
| 37 | 257 | 165 | 322 | 293 | 216 | 421 | 612 | 549 | 836 | 677 | 380 | 805 | 687 | 933 | 735 |
| 38 | 21 | 166 | 532 | 294 | 416 | 422 | 341 | 550 | 347 | 678 | 818 | 806 | 903 | 934 | 993 |
| 39 | 132 | 167 | 263 | 295 | 271 | 423 | 777 | 551 | 897 | 679 | 461 | 807 | 825 | 935 | 885 |
| 40 | 35 | 168 | 149 | 296 | 279 | 424 | 220 | 552 | 243 | 680 | 496 | 808 | 500 | 936 | 939 |
| 41 | 258 | 169 | 102 | 297 | 158 | 425 | 314 | 553 | 662 | 681 | 669 | 809 | 846 | 937 | 994 |
| 42 | 26 | 170 | 105 | 298 | 337 | 426 | 424 | 554 | 454 | 682 | 679 | 810 | 745 | 938 | 980 |
| 43 | 513 | 171 | 304 | 299 | 550 | 427 | 395 | 555 | 318 | 683 | 724 | 811 | 826 | 939 | 926 |
| 44 | 80 | 172 | 296 | 300 | 672 | 428 | 673 | 556 | 675 | 684 | 841 | 812 | 732 | 940 | 764 |
| 45 | 37 | 173 | 163 | 301 | 118 | 429 | 583 | 557 | 618 | 685 | 629 | 813 | 446 | 941 | 941 |
| 46 | 25 | 174 | 92 | 302 | 332 | 430 | 355 | 558 | 898 | 686 | 351 | 814 | 962 | 942 | 967 |
| 47 | 22 | 175 | 47 | 303 | 579 | 431 | 287 | 559 | 781 | 687 | 467 | 815 | 936 | 943 | 886 |
| 48 | 136 | 176 | 267 | 304 | 540 | 432 | 183 | 560 | 376 | 688 | 438 | 816 | 475 | 944 | 831 |
| 49 | 260 | 177 | 385 | 305 | 389 | 433 | 234 | 561 | 428 | 689 | 737 | 817 | 853 | 945 | 947 |
| 50 | 264 | 178 | 546 | 306 | 173 | 434 | 125 | 562 | 665 | 690 | 251 | 818 | 867 | 946 | 507 |
| 51 | 38 | 179 | 324 | 307 | 121 | 435 | 557 | 563 | 736 | 691 | 462 | 819 | 637 | 947 | 889 |
| 52 | 514 | 180 | 208 | 308 | 553 | 436 | 660 | 564 | 567 | 692 | 442 | 820 | 907 | 948 | 984 |
| 53 | 96 | 181 | 386 | 309 | 199 | 437 | 616 | 565 | 840 | 693 | 441 | 821 | 487 | 949 | 751 |
| 54 | 67 | 182 | 150 | 310 | 784 | 438 | 342 | 566 | 625 | 694 | 469 | 822 | 695 | 950 | 942 |
| 55 | 41 | 183 | 153 | 311 | 179 | 439 | 316 | 567 | 238 | 695 | 247 | 823 | 746 | 951 | 996 |
| 56 | 144 | 184 | 165 | 312 | 228 | 440 | 241 | 568 | 359 | 696 | 683 | 824 | 828 | 952 | 971 |
| 57 | 28 | 185 | 106 | 313 | 338 | 441 | 778 | 569 | 457 | 697 | 842 | 825 | 753 | 953 | 890 |
| 58 | 69 | 186 | 55 | 314 | 312 | 442 | 563 | 570 | 399 | 698 | 738 | 826 | 854 | 954 | 509 |
| 59 | 42 | 187 | 328 | 315 | 704 | 443 | 345 | 571 | 787 | 699 | 899 | 827 | 857 | 955 | 949 |
| 60 | 516 | 188 | 536 | 316 | 390 | 444 | 452 | 572 | 591 | 700 | 670 | 828 | 504 | 956 | 973 |
| 61 | 49 | 189 | 577 | 317 | 174 | 445 | 397 | 573 | 678 | 701 | 783 | 829 | 799 | 957 | 1000 |
| 62 | 74 | 190 | 548 | 318 | 554 | 446 | 403 | 574 | 434 | 702 | 849 | 830 | 255 | 958 | 892 |
| 63 | 272 | 191 | 113 | 319 | 581 | 447 | 207 | 575 | 677 | 703 | 820 | 831 | 964 | 959 | 950 |
| 64 | 160 | 192 | 154 | 320 | 393 | 448 | 674 | 576 | 349 | 704 | 728 | 832 | 909 | 960 | 863 |
| 65 | 520 | 193 | 79 | 321 | 283 | 449 | 558 | 577 | 245 | 705 | 928 | 833 | 719 | 961 | 759 |
| 66 | 288 | 194 | 269 | 322 | 122 | 450 | 785 | 578 | 458 | 706 | 791 | 834 | 477 | 962 | 1008 |
| 67 | 528 | 195 | 108 | 323 | 448 | 451 | 432 | 579 | 666 | 707 | 367 | 835 | 915 | 963 | 510 |
| 68 | 192 | 196 | 578 | 324 | 353 | 452 | 357 | 580 | 620 | 708 | 901 | 836 | 638 | 964 | 979 |
| 69 | 544 | 197 | 224 | 325 | 561 | 453 | 187 | 581 | 363 | 709 | 630 | 837 | 748 | 965 | 953 |
| 70 | 70 | 198 | 166 | 326 | 203 | 454 | 236 | 582 | 127 | 710 | 685 | 838 | 944 | 966 | 763 |
| 71 | 44 | 199 | 519 | 327 | 63 | 455 | 664 | 583 | 191 | 711 | 844 | 839 | 869 | 967 | 974 |
| 72 | 131 | 200 | 552 | 328 | 340 | 456 | 624 | 584 | 782 | 712 | 633 | 840 | 491 | 968 | 954 |
| 73 | 81 | 201 | 195 | 329 | 394 | 457 | 587 | 585 | 407 | 713 | 711 | 841 | 699 | 969 | 879 |
| 74 | 50 | 202 | 270 | 330 | 527 | 458 | 780 | 586 | 436 | 714 | 253 | 842 | 754 | 970 | 981 |
| 75 | 73 | 203 | 641 | 331 | 582 | 459 | 705 | 587 | 626 | 715 | 691 | 843 | 858 | 971 | 982 |
| 76 | 15 | 204 | 523 | 332 | 556 | 460 | 126 | 588 | 571 | 716 | 824 | 844 | 478 | 972 | 927 |
| 77 | 320 | 205 | 275 | 333 | 181 | 461 | 242 | 589 | 465 | 717 | 902 | 845 | 968 | 973 | 995 |
| 78 | 133 | 206 | 580 | 334 | 295 | 462 | 565 | 590 | 681 | 718 | 686 | 846 | 383 | 974 | 765 |
| 79 | 52 | 207 | 291 | 335 | 285 | 463 | 398 | 591 | 246 | 719 | 740 | 847 | 910 | 975 | 956 |
| 80 | 23 | 208 | 59 | 336 | 232 | 464 | 346 | 592 | 707 | 720 | 850 | 848 | 815 | 976 | 887 |
| 81 | 134 | 209 | 169 | 337 | 124 | 465 | 456 | 593 | 350 | 721 | 375 | 849 | 976 | 977 | 985 |
| 82 | 384 | 210 | 560 | 338 | 205 | 466 | 358 | 594 | 599 | 722 | 444 | 850 | 870 | 978 | 997 |
| 83 | 76 | 211 | 114 | 339 | 182 | 467 | 405 | 595 | 668 | 723 | 470 | 851 | 917 | 979 | 986 |
| 84 | 137 | 212 | 277 | 340 | 643 | 468 | 303 | 596 | 790 | 724 | 483 | 852 | 727 | 980 | 943 |
| 85 | 82 | 213 | 156 | 341 | 562 | 469 | 569 | 597 | 460 | 725 | 415 | 853 | 493 | 981 | 891 |
| 86 | 56 | 214 | 87 | 342 | 286 | 470 | 244 | 598 | 249 | 726 | 485 | 854 | 873 | 982 | 998 |
| 87 | 27 | 215 | 197 | 343 | 585 | 471 | 595 | 599 | 682 | 727 | 905 | 855 | 701 | 983 | 766 |
| 88 | 97 | 216 | 116 | 344 | 299 | 472 | 189 | 600 | 573 | 728 | 795 | 856 | 931 | 984 | 511 |
| 89 | 39 | 217 | 170 | 345 | 354 | 473 | 566 | 601 | 411 | 729 | 473 | 857 | 756 | 985 | 988 |
| 90 | 259 | 218 | 61 | 346 | 211 | 474 | 676 | 602 | 803 | 730 | 634 | 858 | 860 | 986 | 1001 |
| 91 | 84 | 219 | 531 | 347 | 401 | 475 | 361 | 603 | 789 | 731 | 744 | 859 | 499 | 987 | 951 |
| 92 | 138 | 220 | 525 | 348 | 185 | 476 | 706 | 604 | 709 | 732 | 852 | 860 | 731 | 988 | 1002 |
| 93 | 145 | 221 | 642 | 349 | 396 | 477 | 589 | 605 | 365 | 733 | 960 | 861 | 823 | 989 | 893 |
| 94 | 261 | 222 | 281 | 350 | 344 | 478 | 215 | 606 | 440 | 734 | 865 | 862 | 922 | 990 | 975 |
| 95 | 29 | 223 | 278 | 351 | 586 | 479 | 786 | 607 | 628 | 735 | 693 | 863 | 874 | 991 | 894 |
| 96 | 43 | 224 | 526 | 352 | 645 | 480 | 647 | 608 | 689 | 736 | 797 | 864 | 918 | 992 | 1009 |
| 97 | 98 | 225 | 177 | 353 | 593 | 481 | 348 | 609 | 374 | 737 | 906 | 865 | 502 | 993 | 955 |
| 98 | 515 | 226 | 293 | 354 | 535 | 482 | 419 | 610 | 423 | 738 | 715 | 866 | 933 | 994 | 1004 |
| 99 | 88 | 227 | 388 | 355 | 240 | 483 | 406 | 611 | 466 | 739 | 807 | 867 | 743 | 995 | 1010 |
| 100 | 140 | 228 | 91 | 356 | 206 | 484 | 464 | 612 | 793 | 740 | 474 | 868 | 760 | 996 | 957 |
| 101 | 30 | 229 | 584 | 357 | 95 | 485 | 680 | 613 | 250 | 741 | 636 | 869 | 881 | 997 | 983 |
| 102 | 146 | 230 | 769 | 358 | 327 | 486 | 801 | 614 | 371 | 742 | 694 | 870 | 494 | 998 | 958 |
| 103 | 71 | 231 | 198 | 359 | 564 | 487 | 362 | 615 | 481 | 743 | 254 | 871 | 702 | 999 | 987 |
| 104 | 262 | 232 | 172 | 360 | 800 | 488 | 590 | 616 | 574 | 744 | 717 | 872 | 921 | 1000 | 1012 |
| 105 | 265 | 233 | 120 | 361 | 402 | 489 | 409 | 617 | 413 | 745 | 575 | 873 | 501 | 1001 | 999 |
| 106 | 161 | 234 | 201 | 362 | 356 | 490 | 570 | 618 | 603 | 746 | 913 | 874 | 876 | 1002 | 1016 |
| 107 | 576 | 235 | 336 | 363 | 307 | 491 | 788 | 619 | 366 | 747 | 798 | 875 | 847 | 1003 | 767 |
| 108 | 45 | 236 | 62 | 364 | 301 | 492 | 597 | 620 | 468 | 748 | 811 | 876 | 992 | 1004 | 989 |
| 109 | 100 | 237 | 282 | 365 | 417 | 493 | 572 | 621 | 655 | 749 | 379 | 877 | 447 | 1005 | 1003 |
| 110 | 640 | 238 | 143 | 366 | 213 | 494 | 219 | 622 | 900 | 750 | 697 | 878 | 733 | 1006 | 990 |
| 111 | 51 | 239 | 103 | 367 | 568 | 495 | 311 | 623 | 805 | 751 | 431 | 879 | 827 | 1007 | 1005 |
| 112 | 148 | 240 | 178 | 368 | 832 | 496 | 708 | 624 | 615 | 752 | 607 | 880 | 934 | 1008 | 959 |
| 113 | 46 | 241 | 294 | 369 | 588 | 497 | 598 | 625 | 684 | 753 | 489 | 881 | 882 | 1009 | 1011 |
| 114 | 75 | 242 | 93 | 370 | 186 | 498 | 601 | 626 | 710 | 754 | 866 | 882 | 937 | 1010 | 1013 |
| 115 | 266 | 243 | 644 | 371 | 646 | 499 | 651 | 627 | 429 | 755 | 723 | 883 | 963 | 1011 | 895 |
| 116 | 273 | 244 | 202 | 372 | 404 | 500 | 421 | 628 | 794 | 756 | 486 | 884 | 747 | 1012 | 1006 |
| 117 | 517 | 245 | 592 | 373 | 227 | 501 | 792 | 629 | 252 | 757 | 908 | 885 | 505 | 1013 | 1014 |
| 118 | 104 | 246 | 323 | 374 | 896 | 502 | 802 | 630 | 373 | 758 | 718 | 886 | 855 | 1014 | 1017 |
| 119 | 162 | 247 | 392 | 375 | 594 | 503 | 611 | 631 | 605 | 759 | 813 | 887 | 924 | 1015 | 1018 |
| 120 | 53 | 248 | 297 | 376 | 418 | 504 | 602 | 632 | 848 | 760 | 476 | 888 | 734 | 1016 | 991 |
| 121 | 193 | 249 | 770 | 377 | 302 | 505 | 410 | 633 | 690 | 761 | 856 | 889 | 829 | 1017 | 1020 |
| 122 | 152 | 250 | 107 | 378 | 649 | 506 | 231 | 634 | 713 | 762 | 839 | 890 | 965 | 1018 | 1007 |
| 123 | 77 | 251 | 180 | 379 | 771 | 507 | 688 | 635 | 632 | 763 | 725 | 891 | 938 | 1019 | 1015 |
| 124 | 164 | 252 | 151 | 380 | 360 | 508 | 653 | 636 | 482 | 764 | 698 | 892 | 884 | 1020 | 1019 |
| 125 | 768 | 253 | 209 | 381 | 539 | 509 | 248 | 637 | 806 | 765 | 914 | 893 | 506 | 1021 | 1021 |
| 126 | 268 | 254 | 284 | 382 | 111 | 510 | 369 | 638 | 427 | 766 | 752 | 894 | 749 | 1022 | 1022 |
| 127 | 274 | 255 | 648 | 383 | 331 | 511 | 190 | 639 | 904 | 767 | 868 | 895 | 945 | 1023 | 1023 |

### 5.3.2	Low density parity check coding

The bit sequence input for a given code block to channel coding is denoted by , where  is the number of bits to encode as defined in Clause 5.2.2. After encoding the bits are denoted by , where  for LDPC base graph 1 and  for LDPC base graph 2, and the value of  is given in Clause 5.2.2.

For a code block encoded by LDPC, the following encoding procedure applies:

1)	Find the set with index  in Table 5.3.2-1 which contains .

2)	for  to

if

;

else

;

;

end if

end for

3)	Generate  parity bits  such that , where ;  is a column vector of all elements equal to 0. The encoding is performed in GF(2).

For LDPC base graph 1, a matrix of  has 46 rows with row indices and 68 columns with column indices . For LDPC base graph 2, a matrix of  has 42 rows with row indices and 52 columns with column indices . The elements in  with row and column indices given in Table 5.3.2-2 (for LDPC base graph 1) and Table 5.3.2-3 (for LDPC base graph 2) are of value 1, and all other elements in  are of value 0.

The matrix  is obtained by replacing each element of  with a  matrix, according to the following:

-	Each element of value 0 in  is replaced by an all zero matrix of size ;

-	Each element of value 1 in  is replaced by a circular permutation matrix  of size , where  and  are the row and column indices of the element, and  is obtained by circularly shifting the identity matrix of size  to the right  times. The value of  is given by . The value of  is given by Tables 5.3.2-2 and 5.3.2-3 according to the set index  and LDPC base graph.

4)	for  to

;

end for

Table 5.3.2-1: Sets of LDPC lifting size

| Set index () | Set of lifting sizes () |
| --- | --- |
| 0 | {2, 4, 8, 16, 32, 64, 128, 256} |
| 1 | {3, 6, 12, 24, 48, 96, 192, 384} |
| 2 | {5, 10, 20, 40, 80, 160, 320} |
| 3 | {7, 14, 28, 56, 112, 224} |
| 4 | {9, 18, 36, 72, 144, 288} |
| 5 | {11, 22, 44, 88, 176, 352} |
| 6 | {13, 26, 52, 104, 208} |
| 7 | {15, 30, 60, 120, 240} |

Table 5.3.2-2: LDPC base graph 1 () and its parity check matrices ()

| Row index | Column index | Set index |  |  |  |  |  |  |  | Row index | Column index | Set index |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 250 | 307 | 73 | 223 | 211 | 294 | 0 | 135 | 15 | 1 | 96 | 2 | 290 | 120 | 0 | 348 | 6 | 138 |
|  | 1 | 69 | 19 | 15 | 16 | 198 | 118 | 0 | 227 |  | 10 | 65 | 210 | 60 | 131 | 183 | 15 | 81 | 220 |
|  | 2 | 226 | 50 | 103 | 94 | 188 | 167 | 0 | 126 |  | 13 | 63 | 318 | 130 | 209 | 108 | 81 | 182 | 173 |
|  | 3 | 159 | 369 | 49 | 91 | 186 | 330 | 0 | 134 |  | 18 | 75 | 55 | 184 | 209 | 68 | 176 | 53 | 142 |
|  | 5 | 100 | 181 | 240 | 74 | 219 | 207 | 0 | 84 |  | 25 | 179 | 269 | 51 | 81 | 64 | 113 | 46 | 49 |
|  | 6 | 10 | 216 | 39 | 10 | 4 | 165 | 0 | 83 |  | 37 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 9 | 59 | 317 | 15 | 0 | 29 | 243 | 0 | 53 | 16 | 1 | 64 | 13 | 69 | 154 | 270 | 190 | 88 | 78 |
|  | 10 | 229 | 288 | 162 | 205 | 144 | 250 | 0 | 225 |  | 3 | 49 | 338 | 140 | 164 | 13 | 293 | 198 | 152 |
|  | 11 | 110 | 109 | 215 | 216 | 116 | 1 | 0 | 205 |  | 11 | 49 | 57 | 45 | 43 | 99 | 332 | 160 | 84 |
|  | 12 | 191 | 17 | 164 | 21 | 216 | 339 | 0 | 128 |  | 20 | 51 | 289 | 115 | 189 | 54 | 331 | 122 | 5 |
|  | 13 | 9 | 357 | 133 | 215 | 115 | 201 | 0 | 75 |  | 22 | 154 | 57 | 300 | 101 | 0 | 114 | 182 | 205 |
|  | 15 | 195 | 215 | 298 | 14 | 233 | 53 | 0 | 135 |  | 38 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 16 | 23 | 106 | 110 | 70 | 144 | 347 | 0 | 217 | 17 | 0 | 7 | 260 | 257 | 56 | 153 | 110 | 91 | 183 |
|  | 18 | 190 | 242 | 113 | 141 | 95 | 304 | 0 | 220 |  | 14 | 164 | 303 | 147 | 110 | 137 | 228 | 184 | 112 |
|  | 19 | 35 | 180 | 16 | 198 | 216 | 167 | 0 | 90 |  | 16 | 59 | 81 | 128 | 200 | 0 | 247 | 30 | 106 |
|  | 20 | 239 | 330 | 189 | 104 | 73 | 47 | 0 | 105 |  | 17 | 1 | 358 | 51 | 63 | 0 | 116 | 3 | 219 |
|  | 21 | 31 | 346 | 32 | 81 | 261 | 188 | 0 | 137 |  | 21 | 144 | 375 | 228 | 4 | 162 | 190 | 155 | 129 |
|  | 22 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |  | 39 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 18 | 1 | 42 | 130 | 260 | 199 | 161 | 47 | 1 | 183 |
| 1 | 0 | 2 | 76 | 303 | 141 | 179 | 77 | 22 | 96 |  | 12 | 233 | 163 | 294 | 110 | 151 | 286 | 41 | 215 |
|  | 2 | 239 | 76 | 294 | 45 | 162 | 225 | 11 | 236 |  | 13 | 8 | 280 | 291 | 200 | 0 | 246 | 167 | 180 |
|  | 3 | 117 | 73 | 27 | 151 | 223 | 96 | 124 | 136 |  | 18 | 155 | 132 | 141 | 143 | 241 | 181 | 68 | 143 |
|  | 4 | 124 | 288 | 261 | 46 | 256 | 338 | 0 | 221 |  | 19 | 147 | 4 | 295 | 186 | 144 | 73 | 148 | 14 |
|  | 5 | 71 | 144 | 161 | 119 | 160 | 268 | 10 | 128 |  | 40 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 7 | 222 | 331 | 133 | 157 | 76 | 112 | 0 | 92 | 19 | 0 | 60 | 145 | 64 | 8 | 0 | 87 | 12 | 179 |
|  | 8 | 104 | 331 | 4 | 133 | 202 | 302 | 0 | 172 |  | 1 | 73 | 213 | 181 | 6 | 0 | 110 | 6 | 108 |
|  | 9 | 173 | 178 | 80 | 87 | 117 | 50 | 2 | 56 |  | 7 | 72 | 344 | 101 | 103 | 118 | 147 | 166 | 159 |
|  | 11 | 220 | 295 | 129 | 206 | 109 | 167 | 16 | 11 |  | 8 | 127 | 242 | 270 | 198 | 144 | 258 | 184 | 138 |
|  | 12 | 102 | 342 | 300 | 93 | 15 | 253 | 60 | 189 |  | 10 | 224 | 197 | 41 | 8 | 0 | 204 | 191 | 196 |
|  | 14 | 109 | 217 | 76 | 79 | 72 | 334 | 0 | 95 |  | 41 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 15 | 132 | 99 | 266 | 9 | 152 | 242 | 6 | 85 | 20 | 0 | 151 | 187 | 301 | 105 | 265 | 89 | 6 | 77 |
|  | 16 | 142 | 354 | 72 | 118 | 158 | 257 | 30 | 153 |  | 3 | 186 | 206 | 162 | 210 | 81 | 65 | 12 | 187 |
|  | 17 | 155 | 114 | 83 | 194 | 147 | 133 | 0 | 87 |  | 9 | 217 | 264 | 40 | 121 | 90 | 155 | 15 | 203 |
|  | 19 | 255 | 331 | 260 | 31 | 156 | 9 | 168 | 163 |  | 11 | 47 | 341 | 130 | 214 | 144 | 244 | 5 | 167 |
|  | 21 | 28 | 112 | 301 | 187 | 119 | 302 | 31 | 216 |  | 22 | 160 | 59 | 10 | 183 | 228 | 30 | 30 | 130 |
|  | 22 | 0 | 0 | 0 | 0 | 0 | 0 | 105 | 0 |  | 42 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 21 | 1 | 249 | 205 | 79 | 192 | 64 | 162 | 6 | 197 |
|  | 24 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 5 | 121 | 102 | 175 | 131 | 46 | 264 | 86 | 122 |
| 2 | 0 | 106 | 205 | 68 | 207 | 258 | 226 | 132 | 189 |  | 16 | 109 | 328 | 132 | 220 | 266 | 346 | 96 | 215 |
|  | 1 | 111 | 250 | 7 | 203 | 167 | 35 | 37 | 4 |  | 20 | 131 | 213 | 283 | 50 | 9 | 143 | 42 | 65 |
|  | 2 | 185 | 328 | 80 | 31 | 220 | 213 | 21 | 225 |  | 21 | 171 | 97 | 103 | 106 | 18 | 109 | 199 | 216 |
|  | 4 | 63 | 332 | 280 | 176 | 133 | 302 | 180 | 151 |  | 43 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 5 | 117 | 256 | 38 | 180 | 243 | 111 | 4 | 236 | 22 | 0 | 64 | 30 | 177 | 53 | 72 | 280 | 44 | 25 |
|  | 6 | 93 | 161 | 227 | 186 | 202 | 265 | 149 | 117 |  | 12 | 142 | 11 | 20 | 0 | 189 | 157 | 58 | 47 |
|  | 7 | 229 | 267 | 202 | 95 | 218 | 128 | 48 | 179 |  | 13 | 188 | 233 | 55 | 3 | 72 | 236 | 130 | 126 |
|  | 8 | 177 | 160 | 200 | 153 | 63 | 237 | 38 | 92 |  | 17 | 158 | 22 | 316 | 148 | 257 | 113 | 131 | 178 |
|  | 9 | 95 | 63 | 71 | 177 | 0 | 294 | 122 | 24 |  | 44 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 10 | 39 | 129 | 106 | 70 | 3 | 127 | 195 | 68 | 23 | 1 | 156 | 24 | 249 | 88 | 180 | 18 | 45 | 185 |
|  | 13 | 142 | 200 | 295 | 77 | 74 | 110 | 155 | 6 |  | 2 | 147 | 89 | 50 | 203 | 0 | 6 | 18 | 127 |
|  | 14 | 225 | 88 | 283 | 214 | 229 | 286 | 28 | 101 |  | 10 | 170 | 61 | 133 | 168 | 0 | 181 | 132 | 117 |
|  | 15 | 225 | 53 | 301 | 77 | 0 | 125 | 85 | 33 |  | 18 | 152 | 27 | 105 | 122 | 165 | 304 | 100 | 199 |
|  | 17 | 245 | 131 | 184 | 198 | 216 | 131 | 47 | 96 |  | 45 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 18 | 205 | 240 | 246 | 117 | 269 | 163 | 179 | 125 | 24 | 0 | 112 | 298 | 289 | 49 | 236 | 38 | 9 | 32 |
|  | 19 | 251 | 205 | 230 | 223 | 200 | 210 | 42 | 67 |  | 3 | 86 | 158 | 280 | 157 | 199 | 170 | 125 | 178 |
|  | 20 | 117 | 13 | 276 | 90 | 234 | 7 | 66 | 230 |  | 4 | 236 | 235 | 110 | 64 | 0 | 249 | 191 | 2 |
|  | 24 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 11 | 116 | 339 | 187 | 193 | 266 | 288 | 28 | 156 |
|  | 25 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 22 | 222 | 234 | 281 | 124 | 0 | 194 | 6 | 58 |
| 3 | 0 | 121 | 276 | 220 | 201 | 187 | 97 | 4 | 128 |  | 46 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 89 | 87 | 208 | 18 | 145 | 94 | 6 | 23 | 25 | 1 | 23 | 72 | 172 | 1 | 205 | 279 | 4 | 27 |
|  | 3 | 84 | 0 | 30 | 165 | 166 | 49 | 33 | 162 |  | 6 | 136 | 17 | 295 | 166 | 0 | 255 | 74 | 141 |
|  | 4 | 20 | 275 | 197 | 5 | 108 | 279 | 113 | 220 |  | 7 | 116 | 383 | 96 | 65 | 0 | 111 | 16 | 11 |
|  | 6 | 150 | 199 | 61 | 45 | 82 | 139 | 49 | 43 |  | 14 | 182 | 312 | 46 | 81 | 183 | 54 | 28 | 181 |
|  | 7 | 131 | 153 | 175 | 142 | 132 | 166 | 21 | 186 |  | 47 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 8 | 243 | 56 | 79 | 16 | 197 | 91 | 6 | 96 | 26 | 0 | 195 | 71 | 270 | 107 | 0 | 325 | 21 | 163 |
|  | 10 | 136 | 132 | 281 | 34 | 41 | 106 | 151 | 1 |  | 2 | 243 | 81 | 110 | 176 | 0 | 326 | 142 | 131 |
|  | 11 | 86 | 305 | 303 | 155 | 162 | 246 | 83 | 216 |  | 4 | 215 | 76 | 318 | 212 | 0 | 226 | 192 | 169 |
|  | 12 | 246 | 231 | 253 | 213 | 57 | 345 | 154 | 22 |  | 15 | 61 | 136 | 67 | 127 | 277 | 99 | 197 | 98 |
|  | 13 | 219 | 341 | 164 | 147 | 36 | 269 | 87 | 24 |  | 48 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 14 | 211 | 212 | 53 | 69 | 115 | 185 | 5 | 167 | 27 | 1 | 25 | 194 | 210 | 208 | 45 | 91 | 98 | 165 |
|  | 16 | 240 | 304 | 44 | 96 | 242 | 249 | 92 | 200 |  | 6 | 104 | 194 | 29 | 141 | 36 | 326 | 140 | 232 |
|  | 17 | 76 | 300 | 28 | 74 | 165 | 215 | 173 | 32 |  | 8 | 194 | 101 | 304 | 174 | 72 | 268 | 22 | 9 |
|  | 18 | 244 | 271 | 77 | 99 | 0 | 143 | 120 | 235 |  | 49 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 20 | 144 | 39 | 319 | 30 | 113 | 121 | 2 | 172 | 28 | 0 | 128 | 222 | 11 | 146 | 275 | 102 | 4 | 32 |
|  | 21 | 12 | 357 | 68 | 158 | 108 | 121 | 142 | 219 |  | 4 | 165 | 19 | 293 | 153 | 0 | 1 | 1 | 43 |
|  | 22 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |  | 19 | 181 | 244 | 50 | 217 | 155 | 40 | 40 | 200 |
|  | 25 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 21 | 63 | 274 | 234 | 114 | 62 | 167 | 93 | 205 |
| 4 | 0 | 157 | 332 | 233 | 170 | 246 | 42 | 24 | 64 |  | 50 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 102 | 181 | 205 | 10 | 235 | 256 | 204 | 211 | 29 | 1 | 86 | 252 | 27 | 150 | 0 | 273 | 92 | 232 |
|  | 26 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 14 | 236 | 5 | 308 | 11 | 180 | 104 | 136 | 32 |
| 5 | 0 | 205 | 195 | 83 | 164 | 261 | 219 | 185 | 2 |  | 18 | 84 | 147 | 117 | 53 | 0 | 243 | 106 | 118 |
|  | 1 | 236 | 14 | 292 | 59 | 181 | 130 | 100 | 171 |  | 25 | 6 | 78 | 29 | 68 | 42 | 107 | 6 | 103 |
|  | 3 | 194 | 115 | 50 | 86 | 72 | 251 | 24 | 47 |  | 51 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 12 | 231 | 166 | 318 | 80 | 283 | 322 | 65 | 143 | 30 | 0 | 216 | 159 | 91 | 34 | 0 | 171 | 2 | 170 |
|  | 16 | 28 | 241 | 201 | 182 | 254 | 295 | 207 | 210 |  | 10 | 73 | 229 | 23 | 130 | 90 | 16 | 88 | 199 |
|  | 21 | 123 | 51 | 267 | 130 | 79 | 258 | 161 | 180 |  | 13 | 120 | 260 | 105 | 210 | 252 | 95 | 112 | 26 |
|  | 22 | 115 | 157 | 279 | 153 | 144 | 283 | 72 | 180 |  | 24 | 9 | 90 | 135 | 123 | 173 | 212 | 20 | 105 |
|  | 27 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 52 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 6 | 0 | 183 | 278 | 289 | 158 | 80 | 294 | 6 | 199 | 31 | 1 | 95 | 100 | 222 | 175 | 144 | 101 | 4 | 73 |
|  | 6 | 22 | 257 | 21 | 119 | 144 | 73 | 27 | 22 |  | 7 | 177 | 215 | 308 | 49 | 144 | 297 | 49 | 149 |
|  | 10 | 28 | 1 | 293 | 113 | 169 | 330 | 163 | 23 |  | 22 | 172 | 258 | 66 | 177 | 166 | 279 | 125 | 175 |
|  | 11 | 67 | 351 | 13 | 21 | 90 | 99 | 50 | 100 |  | 25 | 61 | 256 | 162 | 128 | 19 | 222 | 194 | 108 |
|  | 13 | 244 | 92 | 232 | 63 | 59 | 172 | 48 | 92 |  | 53 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 17 | 11 | 253 | 302 | 51 | 177 | 150 | 24 | 207 | 32 | 0 | 221 | 102 | 210 | 192 | 0 | 351 | 6 | 103 |
|  | 18 | 157 | 18 | 138 | 136 | 151 | 284 | 38 | 52 |  | 12 | 112 | 201 | 22 | 209 | 211 | 265 | 126 | 110 |
|  | 20 | 211 | 225 | 235 | 116 | 108 | 305 | 91 | 13 |  | 14 | 199 | 175 | 271 | 58 | 36 | 338 | 63 | 151 |
|  | 28 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 24 | 121 | 287 | 217 | 30 | 162 | 83 | 20 | 211 |
| 7 | 0 | 220 | 9 | 12 | 17 | 169 | 3 | 145 | 77 |  | 54 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 44 | 62 | 88 | 76 | 189 | 103 | 88 | 146 | 33 | 1 | 2 | 323 | 170 | 114 | 0 | 56 | 10 | 199 |
|  | 4 | 159 | 316 | 207 | 104 | 154 | 224 | 112 | 209 |  | 2 | 187 | 8 | 20 | 49 | 0 | 304 | 30 | 132 |
|  | 7 | 31 | 333 | 50 | 100 | 184 | 297 | 153 | 32 |  | 11 | 41 | 361 | 140 | 161 | 76 | 141 | 6 | 172 |
|  | 8 | 167 | 290 | 25 | 150 | 104 | 215 | 159 | 166 |  | 21 | 211 | 105 | 33 | 137 | 18 | 101 | 92 | 65 |
|  | 14 | 104 | 114 | 76 | 158 | 164 | 39 | 76 | 18 |  | 55 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 29 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 34 | 0 | 127 | 230 | 187 | 82 | 197 | 60 | 4 | 161 |
| 8 | 0 | 112 | 307 | 295 | 33 | 54 | 348 | 172 | 181 |  | 7 | 167 | 148 | 296 | 186 | 0 | 320 | 153 | 237 |
|  | 1 | 4 | 179 | 133 | 95 | 0 | 75 | 2 | 105 |  | 15 | 164 | 202 | 5 | 68 | 108 | 112 | 197 | 142 |
|  | 3 | 7 | 165 | 130 | 4 | 252 | 22 | 131 | 141 |  | 17 | 159 | 312 | 44 | 150 | 0 | 54 | 155 | 180 |
|  | 12 | 211 | 18 | 231 | 217 | 41 | 312 | 141 | 223 |  | 56 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 16 | 102 | 39 | 296 | 204 | 98 | 224 | 96 | 177 | 35 | 1 | 161 | 320 | 207 | 192 | 199 | 100 | 4 | 231 |
|  | 19 | 164 | 224 | 110 | 39 | 46 | 17 | 99 | 145 |  | 6 | 197 | 335 | 158 | 173 | 278 | 210 | 45 | 174 |
|  | 21 | 109 | 368 | 269 | 58 | 15 | 59 | 101 | 199 |  | 12 | 207 | 2 | 55 | 26 | 0 | 195 | 168 | 145 |
|  | 22 | 241 | 67 | 245 | 44 | 230 | 314 | 35 | 153 |  | 22 | 103 | 266 | 285 | 187 | 205 | 268 | 185 | 100 |
|  | 24 | 90 | 170 | 154 | 201 | 54 | 244 | 116 | 38 |  | 57 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 36 | 0 | 37 | 210 | 259 | 222 | 216 | 135 | 6 | 11 |
| 9 | 0 | 103 | 366 | 189 | 9 | 162 | 156 | 6 | 169 |  | 14 | 105 | 313 | 179 | 157 | 16 | 15 | 200 | 207 |
|  | 1 | 182 | 232 | 244 | 37 | 159 | 88 | 10 | 12 |  | 15 | 51 | 297 | 178 | 0 | 0 | 35 | 177 | 42 |
|  | 10 | 109 | 321 | 36 | 213 | 93 | 293 | 145 | 206 |  | 18 | 120 | 21 | 160 | 6 | 0 | 188 | 43 | 100 |
|  | 11 | 21 | 133 | 286 | 105 | 134 | 111 | 53 | 221 |  | 58 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 13 | 142 | 57 | 151 | 89 | 45 | 92 | 201 | 17 | 37 | 1 | 198 | 269 | 298 | 81 | 72 | 319 | 82 | 59 |
|  | 17 | 14 | 303 | 267 | 185 | 132 | 152 | 4 | 212 |  | 13 | 220 | 82 | 15 | 195 | 144 | 236 | 2 | 204 |
|  | 18 | 61 | 63 | 135 | 109 | 76 | 23 | 164 | 92 |  | 23 | 122 | 115 | 115 | 138 | 0 | 85 | 135 | 161 |
|  | 20 | 216 | 82 | 209 | 218 | 209 | 337 | 173 | 205 |  | 59 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 38 | 0 | 167 | 185 | 151 | 123 | 190 | 164 | 91 | 121 |
| 10 | 1 | 98 | 101 | 14 | 82 | 178 | 175 | 126 | 116 |  | 9 | 151 | 177 | 179 | 90 | 0 | 196 | 64 | 90 |
|  | 2 | 149 | 339 | 80 | 165 | 1 | 253 | 77 | 151 |  | 10 | 157 | 289 | 64 | 73 | 0 | 209 | 198 | 26 |
|  | 4 | 167 | 274 | 211 | 174 | 28 | 27 | 156 | 70 |  | 12 | 163 | 214 | 181 | 10 | 0 | 246 | 100 | 140 |
|  | 7 | 160 | 111 | 75 | 19 | 267 | 231 | 16 | 230 |  | 60 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 8 | 49 | 383 | 161 | 194 | 234 | 49 | 12 | 115 | 39 | 1 | 173 | 258 | 102 | 12 | 153 | 236 | 4 | 115 |
|  | 14 | 58 | 354 | 311 | 103 | 201 | 267 | 70 | 84 |  | 3 | 139 | 93 | 77 | 77 | 0 | 264 | 28 | 188 |
|  | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 7 | 149 | 346 | 192 | 49 | 165 | 37 | 109 | 168 |
| 11 | 0 | 77 | 48 | 16 | 52 | 55 | 25 | 184 | 45 |  | 19 | 0 | 297 | 208 | 114 | 117 | 272 | 188 | 52 |
|  | 1 | 41 | 102 | 147 | 11 | 23 | 322 | 194 | 115 |  | 61 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 12 | 83 | 8 | 290 | 2 | 274 | 200 | 123 | 134 | 40 | 0 | 157 | 175 | 32 | 67 | 216 | 304 | 10 | 4 |
|  | 16 | 182 | 47 | 289 | 35 | 181 | 351 | 16 | 1 |  | 8 | 137 | 37 | 80 | 45 | 144 | 237 | 84 | 103 |
|  | 21 | 78 | 188 | 177 | 32 | 273 | 166 | 104 | 152 |  | 17 | 149 | 312 | 197 | 96 | 2 | 135 | 12 | 30 |
|  | 22 | 252 | 334 | 43 | 84 | 39 | 338 | 109 | 165 |  | 62 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 23 | 22 | 115 | 280 | 201 | 26 | 192 | 124 | 107 | 41 | 1 | 167 | 52 | 154 | 23 | 0 | 123 | 2 | 53 |
|  | 33 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 3 | 173 | 314 | 47 | 215 | 0 | 77 | 75 | 189 |
| 12 | 0 | 160 | 77 | 229 | 142 | 225 | 123 | 6 | 186 |  | 9 | 139 | 139 | 124 | 60 | 0 | 25 | 142 | 215 |
|  | 1 | 42 | 186 | 235 | 175 | 162 | 217 | 20 | 215 |  | 18 | 151 | 288 | 207 | 167 | 183 | 272 | 128 | 24 |
|  | 10 | 21 | 174 | 169 | 136 | 244 | 142 | 203 | 124 |  | 63 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 11 | 32 | 232 | 48 | 3 | 151 | 110 | 153 | 180 | 42 | 0 | 149 | 113 | 226 | 114 | 27 | 288 | 163 | 222 |
|  | 13 | 234 | 50 | 105 | 28 | 238 | 176 | 104 | 98 |  | 4 | 157 | 14 | 65 | 91 | 0 | 83 | 10 | 170 |
|  | 18 | 7 | 74 | 52 | 182 | 243 | 76 | 207 | 80 |  | 24 | 137 | 218 | 126 | 78 | 35 | 17 | 162 | 71 |
|  | 34 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 64 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 13 | 0 | 177 | 313 | 39 | 81 | 231 | 311 | 52 | 220 | 43 | 1 | 151 | 113 | 228 | 206 | 52 | 210 | 1 | 22 |
|  | 3 | 248 | 177 | 302 | 56 | 0 | 251 | 147 | 185 |  | 16 | 163 | 132 | 69 | 22 | 243 | 3 | 163 | 127 |
|  | 7 | 151 | 266 | 303 | 72 | 216 | 265 | 1 | 154 |  | 18 | 173 | 114 | 176 | 134 | 0 | 53 | 99 | 49 |
|  | 20 | 185 | 115 | 160 | 217 | 47 | 94 | 16 | 178 |  | 25 | 139 | 168 | 102 | 161 | 270 | 167 | 98 | 125 |
|  | 23 | 62 | 370 | 37 | 78 | 36 | 81 | 46 | 150 |  | 65 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 35 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 44 | 0 | 139 | 80 | 234 | 84 | 18 | 79 | 4 | 191 |
| 14 | 0 | 206 | 142 | 78 | 14 | 0 | 22 | 1 | 124 |  | 7 | 157 | 78 | 227 | 4 | 0 | 244 | 6 | 211 |
|  | 12 | 55 | 248 | 299 | 175 | 186 | 322 | 202 | 144 |  | 9 | 163 | 163 | 259 | 9 | 0 | 293 | 142 | 187 |
|  | 15 | 206 | 137 | 54 | 211 | 253 | 277 | 118 | 182 |  | 22 | 173 | 274 | 260 | 12 | 57 | 272 | 3 | 148 |
|  | 16 | 127 | 89 | 61 | 191 | 16 | 156 | 130 | 95 |  | 66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 17 | 16 | 347 | 179 | 51 | 0 | 66 | 1 | 72 | 45 | 1 | 149 | 135 | 101 | 184 | 168 | 82 | 181 | 177 |
|  | 21 | 229 | 12 | 258 | 43 | 79 | 78 | 2 | 76 |  | 6 | 151 | 149 | 228 | 121 | 0 | 67 | 45 | 114 |
|  | 36 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 10 | 167 | 15 | 126 | 29 | 144 | 235 | 153 | 93 |
| 15 | 0 | 40 | 241 | 229 | 90 | 170 | 176 | 173 | 39 |  | 67 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 5.3.2-3: LDPC base graph 2 () and its parity check matrices ()

| Row index | Column index | Set index |  |  |  |  |  |  |  | Row index | Column index | Set index |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 9 | 174 | 0 | 72 | 3 | 156 | 143 | 145 | 16 | 26 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 117 | 97 | 0 | 110 | 26 | 143 | 19 | 131 | 17 | 1 | 254 | 158 | 0 | 48 | 120 | 134 | 57 | 196 |
|  | 2 | 204 | 166 | 0 | 23 | 53 | 14 | 176 | 71 |  | 5 | 124 | 23 | 24 | 132 | 43 | 23 | 201 | 173 |
|  | 3 | 26 | 66 | 0 | 181 | 35 | 3 | 165 | 21 |  | 11 | 114 | 9 | 109 | 206 | 65 | 62 | 142 | 195 |
|  | 6 | 189 | 71 | 0 | 95 | 115 | 40 | 196 | 23 |  | 12 | 64 | 6 | 18 | 2 | 42 | 163 | 35 | 218 |
|  | 9 | 205 | 172 | 0 | 8 | 127 | 123 | 13 | 112 |  | 27 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 10 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 18 | 0 | 220 | 186 | 0 | 68 | 17 | 173 | 129 | 128 |
|  | 11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 6 | 194 | 6 | 18 | 16 | 106 | 31 | 203 | 211 |
| 1 | 0 | 167 | 27 | 137 | 53 | 19 | 17 | 18 | 142 |  | 7 | 50 | 46 | 86 | 156 | 142 | 22 | 140 | 210 |
|  | 3 | 166 | 36 | 124 | 156 | 94 | 65 | 27 | 174 |  | 28 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 4 | 253 | 48 | 0 | 115 | 104 | 63 | 3 | 183 | 19 | 0 | 87 | 58 | 0 | 35 | 79 | 13 | 110 | 39 |
|  | 5 | 125 | 92 | 0 | 156 | 66 | 1 | 102 | 27 |  | 1 | 20 | 42 | 158 | 138 | 28 | 135 | 124 | 84 |
|  | 6 | 226 | 31 | 88 | 115 | 84 | 55 | 185 | 96 |  | 10 | 185 | 156 | 154 | 86 | 41 | 145 | 52 | 88 |
|  | 7 | 156 | 187 | 0 | 200 | 98 | 37 | 17 | 23 |  | 29 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 8 | 224 | 185 | 0 | 29 | 69 | 171 | 14 | 9 | 20 | 1 | 26 | 76 | 0 | 6 | 2 | 128 | 196 | 117 |
|  | 9 | 252 | 3 | 55 | 31 | 50 | 133 | 180 | 167 |  | 4 | 105 | 61 | 148 | 20 | 103 | 52 | 35 | 227 |
|  | 11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 11 | 29 | 153 | 104 | 141 | 78 | 173 | 114 | 6 |
|  | 12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 30 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 81 | 25 | 20 | 152 | 95 | 98 | 126 | 74 | 21 | 0 | 76 | 157 | 0 | 80 | 91 | 156 | 10 | 238 |
|  | 1 | 114 | 114 | 94 | 131 | 106 | 168 | 163 | 31 |  | 8 | 42 | 175 | 17 | 43 | 75 | 166 | 122 | 13 |
|  | 3 | 44 | 117 | 99 | 46 | 92 | 107 | 47 | 3 |  | 13 | 210 | 67 | 33 | 81 | 81 | 40 | 23 | 11 |
|  | 4 | 52 | 110 | 9 | 191 | 110 | 82 | 183 | 53 |  | 31 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 8 | 240 | 114 | 108 | 91 | 111 | 142 | 132 | 155 | 22 | 1 | 222 | 20 | 0 | 49 | 54 | 18 | 202 | 195 |
|  | 10 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |  | 2 | 63 | 52 | 4 | 1 | 132 | 163 | 126 | 44 |
|  | 12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 32 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 23 | 0 | 23 | 106 | 0 | 156 | 68 | 110 | 52 | 5 |
| 3 | 1 | 8 | 136 | 38 | 185 | 120 | 53 | 36 | 239 |  | 3 | 235 | 86 | 75 | 54 | 115 | 132 | 170 | 94 |
|  | 2 | 58 | 175 | 15 | 6 | 121 | 174 | 48 | 171 |  | 5 | 238 | 95 | 158 | 134 | 56 | 150 | 13 | 111 |
|  | 4 | 158 | 113 | 102 | 36 | 22 | 174 | 18 | 95 |  | 33 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 5 | 104 | 72 | 146 | 124 | 4 | 127 | 111 | 110 | 24 | 1 | 46 | 182 | 0 | 153 | 30 | 113 | 113 | 81 |
|  | 6 | 209 | 123 | 12 | 124 | 73 | 17 | 203 | 159 |  | 2 | 139 | 153 | 69 | 88 | 42 | 108 | 161 | 19 |
|  | 7 | 54 | 118 | 57 | 110 | 49 | 89 | 3 | 199 |  | 9 | 8 | 64 | 87 | 63 | 101 | 61 | 88 | 130 |
|  | 8 | 18 | 28 | 53 | 156 | 128 | 17 | 191 | 43 |  | 34 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 9 | 128 | 186 | 46 | 133 | 79 | 105 | 160 | 75 | 25 | 0 | 228 | 45 | 0 | 211 | 128 | 72 | 197 | 66 |
|  | 10 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |  | 5 | 156 | 21 | 65 | 94 | 63 | 136 | 194 | 95 |
|  | 13 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 35 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 0 | 179 | 72 | 0 | 200 | 42 | 86 | 43 | 29 | 26 | 2 | 29 | 67 | 0 | 90 | 142 | 36 | 164 | 146 |
|  | 1 | 214 | 74 | 136 | 16 | 24 | 67 | 27 | 140 |  | 7 | 143 | 137 | 100 | 6 | 28 | 38 | 172 | 66 |
|  | 11 | 71 | 29 | 157 | 101 | 51 | 83 | 117 | 180 |  | 12 | 160 | 55 | 13 | 221 | 100 | 53 | 49 | 190 |
|  | 14 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 13 | 122 | 85 | 7 | 6 | 133 | 145 | 161 | 86 |
| 5 | 0 | 231 | 10 | 0 | 185 | 40 | 79 | 136 | 121 |  | 36 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 41 | 44 | 131 | 138 | 140 | 84 | 49 | 41 | 27 | 0 | 8 | 103 | 0 | 27 | 13 | 42 | 168 | 64 |
|  | 5 | 194 | 121 | 142 | 170 | 84 | 35 | 36 | 169 |  | 6 | 151 | 50 | 32 | 118 | 10 | 104 | 193 | 181 |
|  | 7 | 159 | 80 | 141 | 219 | 137 | 103 | 132 | 88 |  | 37 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 11 | 103 | 48 | 64 | 193 | 71 | 60 | 62 | 207 | 28 | 1 | 98 | 70 | 0 | 216 | 106 | 64 | 14 | 7 |
|  | 15 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 2 | 101 | 111 | 126 | 212 | 77 | 24 | 186 | 144 |
| 6 | 0 | 155 | 129 | 0 | 123 | 109 | 47 | 7 | 137 |  | 5 | 135 | 168 | 110 | 193 | 43 | 149 | 46 | 16 |
|  | 5 | 228 | 92 | 124 | 55 | 87 | 154 | 34 | 72 |  | 38 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 7 | 45 | 100 | 99 | 31 | 107 | 10 | 198 | 172 | 29 | 0 | 18 | 110 | 0 | 108 | 133 | 139 | 50 | 25 |
|  | 9 | 28 | 49 | 45 | 222 | 133 | 155 | 168 | 124 |  | 4 | 28 | 17 | 154 | 61 | 25 | 161 | 27 | 57 |
|  | 11 | 158 | 184 | 148 | 209 | 139 | 29 | 12 | 56 |  | 39 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 16 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 30 | 2 | 71 | 120 | 0 | 106 | 87 | 84 | 70 | 37 |
| 7 | 1 | 129 | 80 | 0 | 103 | 97 | 48 | 163 | 86 |  | 5 | 240 | 154 | 35 | 44 | 56 | 173 | 17 | 139 |
|  | 5 | 147 | 186 | 45 | 13 | 135 | 125 | 78 | 186 |  | 7 | 9 | 52 | 51 | 185 | 104 | 93 | 50 | 221 |
|  | 7 | 140 | 16 | 148 | 105 | 35 | 24 | 143 | 87 |  | 9 | 84 | 56 | 134 | 176 | 70 | 29 | 6 | 17 |
|  | 11 | 3 | 102 | 96 | 150 | 108 | 47 | 107 | 172 |  | 40 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 13 | 116 | 143 | 78 | 181 | 65 | 55 | 58 | 154 | 31 | 1 | 106 | 3 | 0 | 147 | 80 | 117 | 115 | 201 |
|  | 17 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 13 | 1 | 170 | 20 | 182 | 139 | 148 | 189 | 46 |
| 8 | 0 | 142 | 118 | 0 | 147 | 70 | 53 | 101 | 176 |  | 41 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 94 | 70 | 65 | 43 | 69 | 31 | 177 | 169 | 32 | 0 | 242 | 84 | 0 | 108 | 32 | 116 | 110 | 179 |
|  | 12 | 230 | 152 | 87 | 152 | 88 | 161 | 22 | 225 |  | 5 | 44 | 8 | 20 | 21 | 89 | 73 | 0 | 14 |
|  | 18 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 12 | 166 | 17 | 122 | 110 | 71 | 142 | 163 | 116 |
| 9 | 1 | 203 | 28 | 0 | 2 | 97 | 104 | 186 | 167 |  | 42 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 8 | 205 | 132 | 97 | 30 | 40 | 142 | 27 | 238 | 33 | 2 | 132 | 165 | 0 | 71 | 135 | 105 | 163 | 46 |
|  | 10 | 61 | 185 | 51 | 184 | 24 | 99 | 205 | 48 |  | 7 | 164 | 179 | 88 | 12 | 6 | 137 | 173 | 2 |
|  | 11 | 247 | 178 | 85 | 83 | 49 | 64 | 81 | 68 |  | 10 | 235 | 124 | 13 | 109 | 2 | 29 | 179 | 106 |
|  | 19 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 43 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 10 | 0 | 11 | 59 | 0 | 174 | 46 | 111 | 125 | 38 | 34 | 0 | 147 | 173 | 0 | 29 | 37 | 11 | 197 | 184 |
|  | 1 | 185 | 104 | 17 | 150 | 41 | 25 | 60 | 217 |  | 12 | 85 | 177 | 19 | 201 | 25 | 41 | 191 | 135 |
|  | 6 | 0 | 22 | 156 | 8 | 101 | 174 | 177 | 208 |  | 13 | 36 | 12 | 78 | 69 | 114 | 162 | 193 | 141 |
|  | 7 | 117 | 52 | 20 | 56 | 96 | 23 | 51 | 232 |  | 44 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 20 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 35 | 1 | 57 | 77 | 0 | 91 | 60 | 126 | 157 | 85 |
| 11 | 0 | 11 | 32 | 0 | 99 | 28 | 91 | 39 | 178 |  | 5 | 40 | 184 | 157 | 165 | 137 | 152 | 167 | 225 |
|  | 7 | 236 | 92 | 7 | 138 | 30 | 175 | 29 | 214 |  | 11 | 63 | 18 | 6 | 55 | 93 | 172 | 181 | 175 |
|  | 9 | 210 | 174 | 4 | 110 | 116 | 24 | 35 | 168 |  | 45 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 13 | 56 | 154 | 2 | 99 | 64 | 141 | 8 | 51 | 36 | 0 | 140 | 25 | 0 | 1 | 121 | 73 | 197 | 178 |
|  | 21 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 2 | 38 | 151 | 63 | 175 | 129 | 154 | 167 | 112 |
| 12 | 1 | 63 | 39 | 0 | 46 | 33 | 122 | 18 | 124 |  | 7 | 154 | 170 | 82 | 83 | 26 | 129 | 179 | 106 |
|  | 3 | 111 | 93 | 113 | 217 | 122 | 11 | 155 | 122 |  | 46 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 11 | 14 | 11 | 48 | 109 | 131 | 4 | 49 | 72 | 37 | 10 | 219 | 37 | 0 | 40 | 97 | 167 | 181 | 154 |
|  | 22 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 13 | 151 | 31 | 144 | 12 | 56 | 38 | 193 | 114 |
| 13 | 0 | 83 | 49 | 0 | 37 | 76 | 29 | 32 | 48 |  | 47 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 1 | 2 | 125 | 112 | 113 | 37 | 91 | 53 | 57 | 38 | 1 | 31 | 84 | 0 | 37 | 1 | 112 | 157 | 42 |
|  | 8 | 38 | 35 | 102 | 143 | 62 | 27 | 95 | 167 |  | 5 | 66 | 151 | 93 | 97 | 70 | 7 | 173 | 41 |
|  | 13 | 222 | 166 | 26 | 140 | 47 | 127 | 186 | 219 |  | 11 | 38 | 190 | 19 | 46 | 1 | 19 | 191 | 105 |
|  | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  | 48 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 14 | 1 | 115 | 19 | 0 | 36 | 143 | 11 | 91 | 82 | 39 | 0 | 239 | 93 | 0 | 106 | 119 | 109 | 181 | 167 |
|  | 6 | 145 | 118 | 138 | 95 | 51 | 145 | 20 | 232 |  | 7 | 172 | 132 | 24 | 181 | 32 | 6 | 157 | 45 |
|  | 11 | 3 | 21 | 57 | 40 | 130 | 8 | 52 | 204 |  | 12 | 34 | 57 | 138 | 154 | 142 | 105 | 173 | 189 |
|  | 13 | 232 | 163 | 27 | 116 | 97 | 166 | 109 | 162 |  | 49 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 24 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 40 | 2 | 0 | 103 | 0 | 98 | 6 | 160 | 193 | 78 |
| 15 | 0 | 51 | 68 | 0 | 116 | 139 | 137 | 174 | 38 |  | 10 | 75 | 107 | 36 | 35 | 73 | 156 | 163 | 67 |
|  | 10 | 175 | 63 | 73 | 200 | 96 | 103 | 108 | 217 |  | 13 | 120 | 163 | 143 | 36 | 102 | 82 | 179 | 180 |
|  | 11 | 213 | 81 | 99 | 110 | 128 | 40 | 102 | 157 |  | 50 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 25 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 41 | 1 | 129 | 147 | 0 | 120 | 48 | 132 | 191 | 53 |
| 16 | 1 | 203 | 87 | 0 | 75 | 48 | 78 | 125 | 170 |  | 5 | 229 | 7 | 2 | 101 | 47 | 6 | 197 | 215 |
|  | 9 | 142 | 177 | 79 | 158 | 9 | 158 | 31 | 23 |  | 11 | 118 | 60 | 55 | 81 | 19 | 8 | 167 | 230 |
|  | 11 | 8 | 135 | 111 | 134 | 28 | 17 | 54 | 175 |  | 51 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | 12 | 242 | 64 | 143 | 97 | 8 | 165 | 176 | 202 |  |  |  |  |  |  |  |  |  |  |

### 5.3.3	Channel coding of small block lengths

The bit sequence input for a given code block to channel coding is denoted by , where  is the number of bits to encode. After encoding the bits are denoted by .

#### 5.3.3.1	Encoding of 1-bit information

For , the code block is encoded according to Table 5.3.3.1-1, where  and  is the modulation order for the code block.

Table 5.3.3.1-1: Encoding of 1-bit information

|  | Encoded bits |
| --- | --- |
| 1 |  |
| 2 |  |
| 4 |  |
| 6 |  |
| 8 |  |

The "x" and "y" in Table 5.3.3.1-1 are placeholders for Clause 6.3.1.1 of [4, TS 38.211] to scramble the information bits in a way that maximizes the Euclidean distance of the modulation symbols carrying the information bits.

#### 5.3.3.2	Encoding of 2-bit information

For , the code block is encoded according to Table 5.3.3-2, where , , and  is the modulation order for the code block.

Table 5.3.3.2-1: Encoding of 2-bit information

|  | Encoded bits |
| --- | --- |
| 1 |  |
| 2 |  |
| 4 |  |
| 6 |  |
| 8 |  |

The "x" in Table 5.3.3.2-1 are placeholders for Clause 6.3.1.1 of [4, TS 38.211] to scramble the information bits in a way that maximizes the Euclidean distance of the modulation symbols carrying the information bits.

#### 5.3.3.3	Encoding of other small block lengths

For , the code block is encoded by , where , , and  represents the basis sequences as defined in Table 5.3.3.3-1.

Table 5.3.3.3-1: Basis sequences for (32, ) code

| i | Mi,0 | Mi,1 | Mi,2 | Mi,3 | Mi,4 | Mi,5 | Mi,6 | Mi,7 | Mi,8 | Mi,9 | Mi,10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 2 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |
| 3 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |
| 4 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 5 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 6 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |
| 7 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 8 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |
| 9 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 10 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 |
| 11 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 12 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 13 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| 14 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 15 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 16 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 17 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 18 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 19 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 20 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 21 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 22 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 23 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 24 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 |
| 25 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 26 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |
| 27 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 28 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 29 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |
| 30 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 31 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

## 5.4	Rate matching

### 5.4.1	Rate matching for Polar code

The rate matching for Polar code is defined per coded block and consists of sub-block interleaving, bit collection, and bit interleaving. The input bit sequence to rate matching is . The output bit sequence after rate matching is denoted as .

#### 5.4.1.1	Sub-block interleaving

The bits input to the sub-block interleaver are the coded bits . The coded bits  are divided into 32 sub-blocks. The bits output from the sub-block interleaver are denoted as , generated as follows:

for  to

;

;

;

end for

where the sub-block interleaver pattern  is given by Table 5.4.1.1-1.

Table 5.4.1.1-1: Sub-block interleaver pattern

| 0 | 0 | 4 | 3 | 8 | 8 | 12 | 10 | 16 | 12 | 20 | 14 | 24 | 24 | 28 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 5 | 5 | 9 | 16 | 13 | 18 | 17 | 20 | 21 | 22 | 25 | 25 | 29 | 29 |
| 2 | 2 | 6 | 6 | 10 | 9 | 14 | 11 | 18 | 13 | 22 | 15 | 26 | 26 | 30 | 30 |
| 3 | 4 | 7 | 7 | 11 | 17 | 15 | 19 | 19 | 21 | 23 | 23 | 27 | 28 | 31 | 31 |

The sets of bit indices  and  are determined as follows, where , , and  are defined in Clause 5.3.1

if

if 	-- puncturing

for  to

;

end for

if

;

else

;

end if

else	-- shortening

for  to

;

end for

end if

end if

;

comprises  most reliable bit indices in ;

;

#### 5.4.1.2	Bit selection

The bit sequence after the sub-block interleaver  from Clause 5.4.1.1 is written into a circular buffer of length .

Denoting by  the rate matching output sequence length, the bit selection output bit sequence , , is generated as follows:

if 	-- repetition

for  to

;

end for

else

if 	-- puncturing

for  to

;

end for

else	-- shortening

for  to

;

end for

end if

end if

#### 5.4.1.3	Interleaving of coded bits

The bit sequence  is interleaved into bit sequence , as follows:

If

Denote  as the smallest integer such that ;

;

for  to

for  to

if

;

else

;

end if

;

end for

end for

;

for  to

for  to

if

;

end if

end for

end for

else

for  to

;

end for

end if

The value of  is no larger than 8192.

### 5.4.2	Rate matching for LDPC code

The rate matching for LDPC code is defined per coded block and consists of bit selection and bit interleaving. The input bit sequence to rate matching is . The output bit sequence after rate matching is denoted as .

#### 5.4.2.1	Bit selection

The bit sequence after encoding  from Clause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Clause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

-	maximum number of layers for one TB for UL-SCH is given by X, where

-	if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell

-	otherwise, X is given by the maximum number of layers for PUSCH supported by the UE for the serving cell

-	maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where

-	if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	otherwise, X is given by the maximum number of layers for PDSCH supported by the UE for the serving cell

-	if the higher layer parameter mcs-Table given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for DL-SCH; otherwise a maximum modulation order  is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table or mcs-TableTransformPrecoder given by a pusch-Config or configuredGrantConfig for at least one UL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for UL-SCH; otherwise a maximum modulation order  is assumed for UL-SCH

-	maximum coding rate of 948/1024;

-	 is given by Table 5.4.2.1-1, where the value of  for DL-SCH is determined according to the initial downlink bandwidth part if there is no other downlink bandwidth part configured to the UE;

-	;

-	 is the number of code blocks of the transport block determined according to Clause 5.2.2.

Table 5.4.2.1-1: Value of

| Maximum number of PRBs across all configured DL BWPs and UL BWPs of a carrier for DL-SCH and UL-SCH, respectively |  |
| --- | --- |
| Less than 33 | 32 |
| 33 to 66 | 66 |
| 67 to 107 | 107 |
| 108 to 135 | 135 |
| 136 to 162 | 162 |
| 163 to 217 | 217 |
| Larger than 217 | 273 |

Denoting by  the rate matching output sequence length for the -th coded block, where the value of  is determined as follows:

Set

for  to

if the -th coded block is not scheduled for transmission as indicated by CBGTI according to Clause 5.1.7.2 for DL-SCH and 6.1.5.2 for UL-SCH in [6, TS 38.214]

;

else

if

;

else

;

end if

;

end if

end for

where

-	 is the number of transmission layers that the transport block is mapped onto;

-	 is the modulation order;

-	 is the total number of coded bits available for transmission of the transport block;

-	 if CBGTI is not present in the DCI scheduling the transport block and  is the number of scheduled code blocks of the transport block if CBGTI is present in the DCI scheduling the transport block.

Denote by  the redundancy version number for this transmission ( = 0, 1, 2 or 3), the rate matching output bit sequence , , is generated as follows, where  is given by Table 5.4.2.1-2 according to the value of  and LDPC base graph:

;

;

while

if

;

;

end if

;

end while

Table 5.4.2.1-2: Starting position of different redundancy versions,

|  | LDPC base graph 1 | LDPC base graph 2 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

#### 5.4.2.2	Bit interleaving

The bit sequence  is interleaved to bit sequence , according to the following, where the value of  is the modulation order.

for  to

for  to

;

end for

end for

### 5.4.3	Rate matching for channel coding of small block lengths

The input bit sequence to rate matching is . The output bit sequence after rate matching is denoted as , where  is the rate matching output sequence length. The bit sequence  is obtained by the following:

for  to

;

end for

## 5.5	Code block concatenation

The input bit sequence for the code block concatenation block are the sequences , for  and , where  is the number of rate matched bits for the -th code block. The output bit sequence from the code block concatenation block is the sequence  for .

The code block concatenation consists of sequentially concatenating the rate matching outputs for the different code blocks. Therefore,

Set  and

while

Set

while

end while

end while

# 6	Uplink transport channels and control information

## 6.1	Random access channel

The sequence index for the random access channel is received from higher layers and is processed according to [4, TS 38.211].

## 6.2	Uplink shared channel

### 6.2.1	Transport block CRC attachment

Error detection is provided on each UL-SCH transport block through a Cyclic Redundancy Check (CRC).

The entire transport block is used to calculate the CRC parity bits. Denote the bits in a transport block delivered to layer 1 by, and the parity bits by, where  is the payload size and  is the number of parity bits. The lowest order information bit  is mapped to the most significant bit of the transport block as defined in Clause 6.1.1 of [TS38.321].

The parity bits are computed and attached to the UL-SCH transport block according to Clause 5.1, by setting  to 24 bits and using the generator polynomial  if ; and by setting  to 16 bits and using the generator polynomial  otherwise.

The bits after CRC attachment are denoted by , where .

### 6.2.2	LDPC base graph selection

For initial transmission of a transport block with coding rate  indicated by the MCS index according to Clause 6.1.4.1 in [6, TS 38.214] and subsequent re-transmission of the same transport block, each code block of the transport block is encoded with either LDPC base graph 1 or 2 according to the following:

-	if , or if  and , or if , LDPC base graph 2 is used;

-	otherwise, LDPC base graph 1 is used,

where  is the payload size as described in Clause 6.2.1.

### 6.2.3	Code block segmentation and code block CRC attachment

The bits input to the code block segmentation are denoted by  where  is the number of bits in the transport block (including CRC).

Code block segmentation and code block CRC attachment are performed according to Clause 5.2.2.

The bits after code block segmentation are denoted by, where  is the code block number and  is the number of bits for code block number  according to Clause 5.2.2.

### 6.2.4	Channel coding of UL-SCH

Code blocks are delivered to the channel coding block. The bits in a code block are denoted by , where  is the code block number, and  is the number of bits in code block number . The total number of code blocks is denoted by  and each code block is individually LDPC encoded according to Clause 5.3.2.

After encoding the bits are denoted by , where the values of  is given in Clause 5.3.2.

### 6.2.5	Rate matching

Coded bits for each code block, denoted as , are delivered to the rate match block, where  is the code block number, and  is the number of encoded bits in code block number . The total number of code blocks is denoted by  and each code block is individually rate matched according to Clause 5.4.2 by setting  if higher layer parameter rateMatching is set to limitedBufferRM and by setting  otherwise.

After rate matching, the bits are denoted by, where is the number of rate matched bits for code block number .

### 6.2.6	Code block concatenation

The input bit sequence for the code block concatenation block are the sequences , for  and where  is the number of rate matched bits for the -th code block.

Code block concatenation is performed according to Clause 5.5.

The bits after code block concatenation are denoted by, where  is the total number of coded bits for transmission.

### 6.2.7	Data and control multiplexing

Denote the coded bits for UL-SCH as .

Denote the coded bits for HARQ-ACK, if any, as .

Denote the coded bits for CSI part 1, if any, as .

Denote the coded bits for CSI part 2, if any, as .

Denote the multiplexed data and control coded bit sequence as .

Denote  as the OFDM symbol index of the scheduled PUSCH, starting from 0 to , where  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS.

Denote  as the subcarrier index of the scheduled PUSCH, starting from 0 to , where  is expressed as a number of subcarriers.

Denote  as the set of resource elements, in ascending order of indices , available for transmission of data in OFDM symbol , for .

Denote  as the number of elements in set . Denote  as the -th element in .

Denote  as the set of resource elements, in ascending order of indices , available for transmission of UCI in OFDM symbol , for . Denote  as the number of elements in set . Denote  as the -th element in . For any OFDM symbol that carriers DMRS of the PUSCH, . For any OFDM symbol that does not carry DMRS of the PUSCH, .

If frequency hopping is configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the second hop.

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the second hop;

-	if HARQ-ACK is present for transmission on the PUSCH with UL-SCH, let

-	 and ;

-	if CSI is present for transmission on the PUSCH with UL-SCH, let

-	;

-	;

-	; and

-	;

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2,; otherwise,

-	;

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise; and

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise;

-	if CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	let , and denote ,  as the number of OFDM symbols of the PUSCH in the first and second hop, respectively;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	;

-

-	.

If frequency hopping is not configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS;

-	if HARQ-ACK is present for transmission on the PUSCH, let ;

-	if CSI is present for transmission on the PUSCH, let  and ;

-	let  and .

The multiplexed data and control coded bit sequence  is obtained according to the following:

Step 1:

Set  for ;

Set  for ;

Set  for ;

Set  for ;

if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits

the number of reserved resource elements for potential HARQ-ACK transmission is calculated according to Clause 6.3.2.4.2.1, by setting ;

denote  as the number of coded bits for potential HARQ-ACK transmission using the reserved resource elements;

if frequency hopping is configured for the PUSCH, let  and ;

if frequency hopping is not configured for the PUSCH, let ;

denote  as the set of reserved resource elements for potential HARQ-ACK transmission, in OFDM symbol , for ;

Set ;

Set ;

for ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

end for

end if

;

end while

end for

else

for ;

end if

Denote  as the number of elements in .

Step 2:

if HARQ-ACK is present for transmission on the PUSCH and the number of HARQ-ACK information bits is more than 2,

Set ;

Set ;

Set ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

end if

Step 3:

if CSI is present for transmission on the PUSCH,

Set ;

Set ;

Set ;

for  to

;

while

;

end while

while

if

if

;

;

end if

if

;

;

end if

;

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

Set ;

Set ;

Set ;

for  to

;

while

;

end while

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

;

for  to

;

end for

;

;

;

;

end if

;

end while

end for

end if

Step 4:

if UL-SCH is present for transmission on the PUSCH,

Set ;

for  to

if

for  to

;

for  to

;

;

end for

end for

end if

end for

end if

Step 5:

if HARQ-ACK is present for transmission on the PUSCH and the number of HARQ-ACK information bits is no more than 2,

Set ;

Set ;

Set ;

for  to

;

while

if

if

;

;

end if

if

;

;

end if

for  to

;

for  to

;

;

;

end for

end for

end if

;

end while

end for

end if

Step 6:

Set ;

for  to

for  to

;

for  to

;

;

end for

end for

end for

## 6.3	Uplink control information

### 6.3.1	Uplink control information on PUCCH

The procedure in this clause applies to PUCCH formats 2/3/4.

#### 6.3.1.1	UCI bit sequence generation

##### 6.3.1.1.1	HARQ-ACK/SR only

If only HARQ-ACK bits are transmitted on a PUCCH, the UCI bit sequence  is determined by setting  for  and , where the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213].

If only HARQ-ACK and SR bits are transmitted on a PUCCH, the UCI bit sequence  is determined by setting  for ,  for , and , where the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS 38.213], and the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213].

##### 6.3.1.1.2	CSI only

The bitwidth for PMI of codebookType=typeI-SinglePanel with 2 CSI-RS ports is 2 for Rank=1 and 1 for Rank=2, according to Clause 5.2.2.2.1 in [6, TS 38.214].

The bitwidth for PMI of codebookType=typeI-SinglePanel with more than 2 CSI-RS ports is provided in Tables 6.3.1.1.2-1, where the values of and  are given by Clause 5.2.2.2.1 in [6, TS 38.214].

Table 6.3.1.1.2-1: PMI of codebookType=typeI-SinglePanel

|  | Information field for wideband PMI |  |  | Information field for wideband PMI or per subband PMI |  |
| --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |
|  | codebookMode=1 | codebookMode=2 |  | codebookMode=1 | codebookMode=2 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (,) | N/A | 2 | 4 |
| Rank = 1 with >2 CSI-RS ports, | (,) | (, 0) | N/A | 2 | 4 |
| Rank=2 with 4 CSI-RS ports, | (,) | (, 0) | 1 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (,) | 2 | 1 | 3 |
| Rank=2 with >4 CSI-RS ports, | (,) | (, 0) | 2 | 1 | 3 |
| Rank=3 or 4, with 4 CSI-RS ports | (,) |  | 0 | 1 |  |
| Rank=3 or 4, with 8 or 12 CSI-RS ports | (,) |  | 2 | 1 |  |
| Rank=3 or 4 , with >=16 CSI-RS ports | (, ) |  | 2 | 1 |  |
| Rank=5 or 6 | (,) |  | N/A | 1 |  |
| Rank=7 or 8, | (, ) |  | N/A | 1 |  |
| Rank=7 or 8, | (,) |  | N/A | 1 |  |
| Rank=7 or 8, with or or | (,) |  | N/A | 1 |  |

The bitwidth for PMI of codebookType= typeI-MultiPanel is provided in Tables 6.3.1.1.2-2, where the values of and  are given by Clause 5.2.2.2.2 in [6, TS 38.214].

Table 6.3.1.1.2-2: PMI of codebookType= typeI-MultiPanel

|  | Information fields for wideband |  |  |  |  | Information fields for wideband or per subband |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (,) |  |  |  |  |  |  |  |  |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | N/A | N/A | 2 | N/A | N/A | N/A |
| Rank=1 with codebookMode=1 | (,) | N/A | 2 | 2 | 2 | 2 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | N/A | N/A | 1 | N/A | N/A | N/A |
| Rank=2 with , codebookMode=1 | (,) | 1 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=3 or 4 with , codebookMode=1 | (,) | 0 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=2 or 3 or 4 with , codebookMode=1 | (,) | 2 | 2 | 2 | 2 | 1 | N/A | N/A | N/A |
| Rank=1 with codebookMode=2 | (,) | N/A | 2 | 2 | N/A | N/A | 2 | 1 | 1 |
| Rank=2 with , codebookMode=2 | (,) | 1 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=3 or 4 with , codebookMode=2 | (,) | 0 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |
| Rank=2 or 3 or 4 with , codebookMode=2 | (,) | 2 | 2 | 2 | N/A | N/A | 1 | 1 | 1 |

The bitwidth for PMI with 1 CSI-RS port is 0.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel is provided in Tables 6.3.1.1.2-3.

Table 6.3.1.1.2-3: RI, LI, CQI, and CRI of codebookType=typeI-SinglePanel

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator | 0 |  |  |  |  |
| Layer Indicator | 0 |  |  |  |  |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2 |
| CRI |  |  |  |  |  |

in Table 6.3.1.1.2-3 is the number of allowed rank indicator values according to Clause 5.2.2.2.1 [6, TS 38.214].  is the value of the rank. The value of  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The bitwidth for RI/LI/CQI/CRI of codebookType= typeI-MultiPanel is provided in Table 6.3.1.1.2-4.

Table 6.3.1.1.2-4: RI, LI, CQI, and CRI of codebookType=typeI-MultiPanel

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| CRI |  |

where  is the number of allowed rank indicator values according to Clause 5.2.2.2.2 [6, TS 38.214],  is the value of the rank, and  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The bitwidth for RI/LI/CQI of codebookType= typeII or codebookType=typeII-PortSelection is provided in Table 6.3.1.1.2-5.

Table 6.3.1.1.2-5: RI, LI, and CQI of codebookType=typeII or typeII-PortSelection

| Field | Bitwidth |
| --- | --- |
| Rank Indicator |  |
| Layer Indicator |  |
| Wide-band CQI | 4 |
| Subband differential CQI | 2 |
| Indicator of the number of non-zero wideband amplitude coefficients for layer |  |

where  is the number of allowed rank indicator values according to Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and  is the value of the rank. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value.

The bitwidth for CRI, SSBRI, RSRP, and differential RSRP are provided in Table 6.3.1.1.2-6.

Table 6.3.1.1.2-6: CRI, SSBRI, and RSRP

| Field | Bitwidth |
| --- | --- |
| CRI |  |
| SSBRI |  |
| RSRP | 7 |
| Differential RSRP | 4 |

where  is the number of CSI-RS resources in the corresponding resource set, and  is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP'.

Table 6.3.1.1.2-7: Mapping order of CSI fields of one CSI report, pmi-FormatIndicator=widebandPMI and cqi-FormatIndicator=widebandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4, if reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4, if reported |
|  | Zero padding bits , if needed |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4, if reported |
|  | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4, if reported |

The number of zero padding bits  in Table 6.3.1.1.2-7 is 0 for 1 CSI-RS port and  for more than 1 CSI-RS port, where

-	 and  is the set of rank values  that are allowed to be reported;

-	, where  is the reported rank;

-	For 2 CSI-RS ports, ;

-	For more than 2 CSI-RS ports, ;

-	if PMI is reported,  and ; otherwise, ;

-	if PMI  is reported,  is obtained according to Tables 6.3.1.1.2-1/2; otherwise, ;

-	if PMI  is reported,  is obtained according to Tables 6.3.1.1.2-1/2; otherwise, ;

-	if CQI is reported,  is obtained according to Tables 6.3.1.1.2-3/4; otherwise, ;

-	if LI is reported,  is obtained according to Tables 6.3.1.1.2-3/4; otherwise, .

Table 6.3.1.1.2-8: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CRI or SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #3 as in Table 6.3.1.1.2-6, if reported |
|  | CRI or SSBRI #4 as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #3 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #4 as in Table 6.3.1.1.2-6, if reported |

Table 6.3.1.1.2-9: Mapping order of CSI fields of one CSI report, CSI part 1, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.1.1.2-10: Mapping order of CSI fields of one CSI report, CSI part 2 wideband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.1.1.2-11: Mapping order of CSI fields of one CSI report, CSI part 2 subband, pmi-FormatIndicator= subbandPMI or cqi-FormatIndicator=subbandCQI

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |

Note:	Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0.

If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to.

Table 6.3.1.1.2-12: Mapping order of CSI reports to UCI bit sequence , without two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1 as in Table 6.3.1.1.2-7/8 |
|  | CSI report #2 as in Table 6.3.1.1.2-7/8 |
|  | … |
|  | CSI report #n as in Table 6.3.1.1.2-7/8 |

If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence  starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the length of UCI bit sequence  is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3.

Table 6.3.1.1.2-13: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1 if CSI report #1 is not of two parts, or CSI report #1, CSI part 1, if CSI report #1 is of two parts, as in Table 6.3.1.1.2-7/8/9 |
|  | CSI report #2 if CSI report #2 is not of two parts, or CSI report #2, CSI part 1, if CSI report #2 is of two parts, as in Table 6.3.1.1.2-7/8/9 |
|  | … |
|  | CSI report #n if CSI report #n is not of two parts, or CSI report #n, CSI part 1, if CSI report #n is of two parts, as in Table 6.3.1.1.2-7/8/9 |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-13 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

Table 6.3.1.1.2-14: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1, CSI part 2 wideband, as in Table 6.3.1.1.2-10 if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 wideband, as in Table 6.3.1.1.2-10 if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 wideband, as in Table 6.3.1.1.2-10 if CSI part 2 exists for CSI report #n |
|  | CSI report #1, CSI part 2 subband, as in Table 6.3.1.1.2-11 if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 subband, as in Table 6.3.1.1.2-11 if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 subband, as in Table 6.3.1.1.2-11 if CSI part 2 exists for CSI report #n |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.1.1.2-14 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

##### 6.3.1.1.3	HARQ-ACK/SR and CSI

If none of the CSI reports for transmission on a PUCCH is of two parts, the UCI bit sequence  is generated according to the following, where :

-	if there is HARQ-ACK for transmission on the PUCCH, the HARQ-ACK bits are mapped to the UCI bit sequence , where  for , the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213], and  is number of HARQ-ACK bits; if there is no HARQ-ACK for transmission on the PUCCH, set ;

-	if there is SR for transmission on the PUCCH, set  for , where the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213]; if there is no SR for transmission on the PUCCH, set ;

-	the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence  starting with , where  is the number of CSI bits.

If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated,  and , according to the following, where  and :

-	if there is HARQ-ACK for transmission on the PUCCH, the HARQ-ACK bits are mapped to the UCI bit sequence , where  for , the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS38.213], and  is number of HARQ-ACK bits; if there is no HARQ-ACK for transmission on the PUCCH, set ;

-	if there is SR for transmission on the PUCCH, set  for , where the SR bit sequence  is given by Clause 9.2.5.1 of [5, TS 38.213]; if there is no SR for transmission on the PUCCH, set ;

-	the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence  starting with , where  is the number of CSI bits in CSI part 1 of all CSI reports;

-	the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence  starting with , where  is the number of CSI bits in CSI part 2 of all CSI reports. If the length of UCI bit sequence  is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3.

#### 6.3.1.2	Code block segmentation and CRC attachment

The UCI bit sequence from clause 6.3.1.1 is denoted by , where  is the payload size. The procedure in 6.3.1.2.1 applies for  and the procedure in Clause 6.3.1.2.2 applies for .

##### 6.3.1.2.1	UCI encoded by Polar code

If the payload size , code block segmentation and CRC attachment is performed according to Clause 5.2.1. If ( and ) or if , ; otherwise , where  is the rate matching output sequence length as given in Clause 6.3.1.4.1.

If , the parity bits  in Clause 5.2.1 are computed by setting  to 6 bits and using the generator polynomial  in Clause 5.1, resulting in the sequence  where  is the code block number and  is the number of bits for code block number .

If , the parity bits  in Clause 5.2.1 are computed by setting  to 11 bits and using the generator polynomial  in Clause 5.1, resulting in the sequence  where  is the code block number and  is the number of bits for code block number .

##### 6.3.1.2.2	UCI encoded by channel coding of small block lengths

If the payload size , CRC bits are not attached.

The output bit sequence is denoted by , where  for  and .

#### 6.3.1.3	Channel coding of UCI

##### 6.3.1.3.1	UCI encoded by Polar code

Information bits are delivered to the channel coding block. They are denoted by  , where  is the code block number, and  is the number of bits in code block number . The total number of code blocks is denoted by  and each code block is individually encoded by the following:

If , the information bits are encoded via Polar coding according to Clause 5.3.1, by setting , , ,  if  and  if , where  is the rate matching output sequence length as given in Clause 6.3.1.4.1.

If , the information bits are encoded via Polar coding according to Clause 5.3.1, by setting , , , and .

After encoding the bits are denoted by , where  is the number of coded bits in code block number .

##### 6.3.1.3.2	UCI encoded by channel coding of small block lengths

Information bits are delivered to the channel coding block. They are denoted by , where  is the number of bits.

The information bits are encoded according to Clause 5.3.3.

After encoding the bits are denoted by , where  is the number of coded bits.

#### 6.3.1.4	Rate matching

For PUCCH formats 2/3/4, the total rate matching output sequence length  is given by Table 6.3.1.4-1, where  , , and  are the number of symbols carrying UCI for PUCCH formats 2/3/4 respectively;  and  are the number of PRBs that are determined by the UE for PUCCH formats 2/3 transmission respectively according to Clause 9.2 of [5, TS38.213]; and  is the spreading factor for PUCCH format 4.

Table 6.3.1.4-1: Total rate matching output sequence length

| PUCCH format | Modulation order |  |
| --- | --- | --- |
|  | QPSK | π/2-BPSK |
| PUCCH format 2 |  | N/A |
| PUCCH format 3 |  |  |
| PUCCH format 4 |  |  |

##### 6.3.1.4.1	UCI encoded by Polar code

The input bit sequence to rate matching is  where  is the code block number, and  is the number of coded bits in code block number .

Table 6.3.1.4.1-1: Rate matching output sequence length

| UCI(s) for transmission on a PUCCH | UCI for encoding | Value of |
| --- | --- | --- |
| HARQ-ACK | HARQ-ACK |  |
| HARQ-ACK, SR | HARQ-ACK, SR |  |
| CSI (CSI not of two parts) | CSI |  |
| HARQ-ACK, CSI (CSI not of two parts) | HARQ-ACK, CSI |  |
| HARQ-ACK, SR, CSI (CSI not of two parts) | HARQ-ACK, SR, CSI |  |
| CSI (CSI of two parts) | CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, CSI (CSI of two parts) | HARQ-ACK, CSI part 1 |  |
|  | CSI part 2 |  |
| HARQ-ACK, SR, CSI (CSI of two parts) | HARQ-ACK, SR, CSI part 1 |  |
|  | CSI part 2 |  |

Rate matching is performed according to Clause 5.4.1 by setting  and the rate matching output sequence length to , where  is the number of code blocks for UCI determined according to Clause 6.3.1.2.1 and the value of  is given by Table 6.3.1.4.1-1:

-	 is the number of bits for HARQ-ACK for transmission on the current PUCCH;

-	 is the number of bits for SR for transmission on the current PUCCH;

-	 is the number of bits for CSI part 1 for transmission on the current PUCCH;

-	 is the number of bits for CSI part 2 for transmission on the current PUCCH;

-	if ,  ; otherwise,  is the number of CRC bits determined according to clause 6.3.1.2.1, where  equals  for "CSI (CSI of two parts)", equals  for "HARQ-ACK, CSI (CSI of two parts)", and equals  for "HARQ-ACK, SR, CSI (CSI of two parts)" respectively in Table 6.3.1.4.1-1;;

-	 is the configured maximum PUCCH coding rate;

-	 is given by Table 6.3.1.4-1.

The output bit sequence after rate matching is denoted as  where  is the length of rate matching output sequence in code block number .

##### 6.3.1.4.2	UCI encoded by channel coding of small block lengths

The input bit sequence to rate matching is .

The value of  is determined according to Table 6.3.1.4.1-1 by setting .

Rate matching is performed according to Clause 5.4.3 by setting the rate matching output sequence length .

The output bit sequence after rate matching is denoted as .

#### 6.3.1.5	Code block concatenation

The input bit sequence for the code block concatenation block are the sequences, for  and where  is the number of rate matched bits for the -th code block.

Code block concatenation is performed according to Clause 5.5.

The bits after code block concatenation are denoted by, where  with the values of  and  given in Clause 6.3.1.4.1. Let  be the total number of coded bits for transmission and . Set  for .

#### 6.3.1.6	Multiplexing of coded UCI bits to PUCCH

If CSI of two parts are transmitted on a PUCCH, the coded bits corresponding to UCI bit sequence  is denoted by and the coded bits corresponding to UCI bit sequence  is denoted by . The coded bit sequence , where , is generated according to the following.

Table 6.3.1.6-1: PUCCH DMRS and UCI symbols

| PUCCH duration (symbols) | PUCCH DMRS symbol indices | Number of UCI symbol indices sets | 1st UCI symbol indices set | 2nd UCI symbol indices set | 3rd UCI symbol indices set |
| --- | --- | --- | --- | --- | --- |
| 4 | {1} | 2 | {0,2} | {3} | - |
| 4 | {0,2} | 1 | {1,3} | - | - |
| 5 | {0, 3} | 1 | {1, 2, 4} | - | - |
| 6 | {1, 4} | 1 | {0, 2, 3, 5} | - | - |
| 7 | {1, 4} | 2 | {0, 2, 3, 5} | {6} | - |
| 8 | {1, 5} | 2 | {0, 2, 4, 6} | {3, 7} | - |
| 9 | {1, 6} | 2 | {0, 2, 5, 7} | {3, 4, 8} | - |
| 10 | {2, 7} | 2 | {1, 3, 6, 8} | {0, 4, 5, 9} | - |
| 10 | {1, 3, 6, 8} | 1 | {0,2,4,5,7,9} | - | - |
| 11 | {2, 7} | 3 | {1,3,6,8} | {0,4,5,9} | {10} |
| 11 | {1,3,6,9} | 1 | {0,2,4,5,7,8,10} | - | - |
| 12 | {2, 8} | 3 | {1,3,7,9} | {0,4,6,10} | {5, 11} |
| 12 | {1,4,7,10} | 1 | {0,2,3,5,6,8,9,11} | - | - |
| 13 | {2, 9} | 3 | {1,3,8,10} | {0,4,7,11} | {5,6,12} |
| 13 | {1,4,7,11} | 2 | {0,2,3,5,6,8,10,12} | {9} | - |
| 14 | {3, 10} | 3 | {2,4,9,11} | {1,5,8,12} | {0,6,7,13} |
| 14 | {1,5,8,12} | 2 | {0,2,4,6,7,9,11,13} | {3, 10} | - |

Denote  as UCI OFDM symbol index. Denote  as the number of elements in UCI symbol indices set  for , where  and  are given by Table 6.3.1.6-1 according to the PUCCH duration and the PUCCH DMRS configuration. Denote  as the number of OFDM symbols carrying UCI in the PUCCH. Denote  as the modulation order of the PUCCH.

For PUCCH format 3, set  , where  is the number of PRBs that is determined by the UE for PUCCH format 3 transmission according to Clause 9.2 of [5, TS 38.213].

For PUCCH format 4, set , where  is the spreading factor for PUCCH format 4.

Find the smallest such that .

Set ;

Set ;

Set ;

Set ;

for  to

if

for  to

for  to

;

;

end for

end for

elseif

if

;

else

;

end if

;

for  to

for  to

;

;

end for

end for

for  to

for  to

;

;

end for

end for

else

for  to

for  to

;

;

end for

end for

end if

end for

Set

for  to

for  to

for  to

;

;

end for

end for

end for

### 6.3.2	Uplink control information on PUSCH

#### 6.3.2.1	UCI bit sequence generation

##### 6.3.2.1.1	HARQ-ACK

If HARQ-ACK bits are transmitted on a PUSCH, the UCI bit sequence  is determined as follows:

-	If UCI is transmitted on PUSCH without UL-SCH and the UCI includes CSI part 1 without CSI part 2,

-	if there is no HARQ-ACK bit given by Clause 9.1 of [5, TS 38.213], set , , and ;

-	if there is only one HARQ-ACK bit  given by Clause 9.1 of [5, TS 38.213], set , , and ;

-	otherwise, set  for  and , where the HARQ-ACK bit sequence  is given by Clause 9.1 of [5, TS 38.213].

##### 6.3.2.1.2	CSI

The bitwidth for PMI of codebookType=typeI-SinglePanel and codebookType=typeI-MultiPanel is specified in Clause 6.3.1.1.2.

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel and codebookType=typeI-MultiPanel is specified in Clause 6.3.1.1.2.

The bitwidth for PMI of codebookType=typeII is provided in Tables 6.3.2.1.2-1, where the values of , , , , , , and  are given by Clause 5.2.2.2.3 in [6, TS 38.214].

Table 6.3.2.1.2-1: PMI of codebookType= typeII

|  | Information fields for wideband PMI |  |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |  |

The bitwidth for PMI of codebookType= typeII-PortSelection is provided in Tables 6.3.2.1.2-2, where the values of , , , , , , and  are given by Clause 5.2.2.2.4 in [6, TS 38.214].

Table 6.3.2.1.2-2: PMI of codebookType= typeII-PortSelection

|  | Information fields for wideband PMI |  |  |  |  | Information fields for wideband PMI or per subband PMI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rank=1 SBAmp off |  |  |  | N/A | N/A |  | N/A | N/A | N/A |
| Rank=2 SBAmp off |  |  |  |  |  |  |  | N/A | N/A |
| Rank=1 SBAmp on |  |  |  | N/A | N/A |  | N/A |  | N/A |
| Rank=2 SBAmp on |  |  |  |  |  |  |  |  |  |

For CSI on PUSCH, two UCI bit sequences are generated,  and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-6, are mapped to the UCI bit sequence  starting with . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.2.1.2-7, are mapped to the UCI bit sequence  starting with .

The mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting is provided in Table 6.3.1.1.2-8. The procedure in clause 6.3.2 described for CSI part 1 is also applicable for one report for CRI/RSRP or SSBRI/RSRP reporting.

Table 6.3.2.1.2-3: Mapping order of CSI fields of one CSI report, CSI part 1

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 1 | CRI as in Tables 6.3.1.1.2-3/4/6, if reported |
|  | Rank Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Wideband CQI for the first TB as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Subband differential CQI for the first TB with increasing order of subband number as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M0 for layer 0 as in Table 6.3.1.1.2-5, if reported |
|  | Indicator of the number of non-zero wideband amplitude coefficients M1 for layer 1 as in Table 6.3.1.1.2-5 (if the rank according to the reported RI is equal to one, this field is set to all zeros), if 2-layer PMI reporting is allowed according to the rank restriction in Clauses 5.2.2.2.3 and 5.2.2.2.4 [6, TS 38.214] and if reported |
| Note: Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0. |  |

Table 6.3.2.1.2-4: Mapping order of CSI fields of one CSI report, CSI part 2 wideband

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2 wideband | Wideband CQI for the second TB as in Tables 6.3.1.1.2-3/4/5, if present and reported |
|  | Layer Indicator as in Tables 6.3.1.1.2-3/4/5, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, if reported |
|  | PMI wideband information fields , from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214], if pmi-FormatIndicator= widebandPMI and if reported |

Table 6.3.2.1.2-5: Mapping order of CSI fields of one CSI report, CSI part 2 subband

| CSI report #n Part 2 subband | Subband differential CQI for the second TB of all even subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
| --- | --- |
|  | PMI subband information fields of all even subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all even subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |
|  | Subband differential CQI for the second TB of all odd subbands with increasing order of subband number, as in Tables 6.3.1.1.2-3/4/5, if cqi-FormatIndicator=subbandCQI and if reported |
|  | PMI subband information fields of all odd subbands with increasing order of subband number, from left to right as in Tables 6.3.1.1.2-1/2 or 6.3.2.1.2-1/2, or codebook index for 2 antenna ports according to Clause 5.2.2.2.1 in [6, TS38.214] of all odd subbands with increasing order of subband number, if pmi-FormatIndicator= subbandPMI and if reported |

Note:	Subbands for given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in the increasing order with the lowest subband of csi-ReportingBand as subband 0.

Table 6.3.2.1.2-6: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI part 1 of CSI report #1 as in Table 6.3.2.1.2-3 or Table 6.3.1.1.2-8 |
|  | CSI part 1 of CSI report #2 as in Table 6.3.2.1.2-3 or Table 6.3.1.1.2-8 |
|  | … |
|  | CSI part 1 of CSI report #n as in Table 6.3.2.1.2-3 or Table 6.3.1.1.2-8 |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.2.1.2-6 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

Table 6.3.2.1.2-7: Mapping order of CSI reports to UCI bit sequence , 
with two-part CSI report(s)

| UCI bit sequence | CSI report number |
| --- | --- |
|  | CSI report #1, CSI part 2 wideband, as in Table 6.3.2.1.2-4 if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 wideband, as in Table 6.3.2.1.2-4 if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 wideband, as in Table 6.3.2.1.2-4 if CSI part 2 exists for CSI report #n |
|  | CSI report #1, CSI part 2 subband, as in Table 6.3.2.1.2-5 if CSI part 2 exists for CSI report #1 |
|  | CSI report #2, CSI part 2 subband, as in Table 6.3.2.1.2-5 if CSI part 2 exists for CSI report #2 |
|  | … |
|  | CSI report #n, CSI part 2 subband, as in Table 6.3.2.1.2-5 if CSI part 2 exists for CSI report #n |

where CSI report #1, CSI report #2, …, CSI report #n in Table 6.3.2.1.2-7 correspond to the CSI reports in increasing order of CSI report priority values according to Clause 5.2.5 of [6, TS38.214].

#### 6.3.2.2	Code block segmentation and CRC attachment

Denote the bits of the payload by , where  is the payload size. The procedure in 6.3.2.2.1 applies for  and the procedure in Clause 6.3.2.2.2 applies for .

##### 6.3.2.2.1	UCI encoded by Polar code

Code block segmentation and CRC attachment is performed according to Clause 6.3.1.2.1.

##### 6.3.2.2.2	UCI encoded by channel coding of small block lengths

The procedure in Clause 6.3.1.2.2 applies.

#### 6.3.2.3	Channel coding of UCI

##### 6.3.2.3.1	UCI encoded by Polar code

Channel coding is performed according to Clause 6.3.1.3.1, except that the rate matching output sequence length   is given in Clause 6.3.2.4.1.

##### 6.3.2.3.2	UCI encoded by channel coding of small block lengths

Information bits are delivered to the channel coding block. They are denoted by , where  is the number of bits.

The information bits are encoded according to Clause 5.3.3.

After encoding the bits are denoted by , where  is the number of coded bits.

#### 6.3.2.4	Rate matching

##### 6.3.2.4.1	UCI encoded by Polar code

###### 6.3.2.4.1.1	HARQ-ACK

For HARQ-ACK transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows:

where

-	 is the number of HARQ-ACK bits;

-	if , ; otherwise  is the number of CRC bits for HARQ-ACK determined according to Clause 6.3.1.2.1;

-	;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise,  is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is configured by higher layer parameter scaling;

-	 is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission.

For HARQ-ACK transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows:

where

-	 is the number of HARQ-ACK bits;

-	if , ; otherwise  is the number of CRC bits for HARQ-ACK defined according to Clause 6.3.1.2.1;;

-	;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is the symbol index of the first OFDM symbol that does not carry DMRS of the PUSCH, after the first DMRS symbol(s), in the PUSCH transmission;

-	 is the code rate of the PUSCH, determined according to Clause 6.1.4.1 of [6, TS38.214];

-	 is the modulation order of the PUSCH;

-	 is configured by higher layer parameter scaling.

The input bit sequence to rate matching is  where  is the code block number, and  is the number of coded bits in code block number .

Rate matching is performed according to Clause 5.4.1 by setting  and the rate matching output sequence length to , where

-	 is the number of code blocks for UCI determined according to Clause 5.2.1;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	.

The output bit sequence after rate matching is denoted as  where  is the length of rate matching output sequence in code block number .

###### 6.3.2.4.1.2	CSI part 1

For CSI part 1 transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as , is determined as follows:

where

-	 is the number of bits for CSI part 1;

-	if , ; otherwise  is the number of CRC bits for CSI part 1 determined according to Clause 6.3.1.2.1;

-	;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise, is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is no more than 2 bits, where  is the number of reserved resource elements for potential HARQ-ACK transmission in OFDM symbol , for , in the PUSCH transmission, defined in Clause 6.2.7;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is configured by higher layer parameter scaling.

For CSI part 1 transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as , is determined as follows:

if there is CSI part 2 to be transmitted on the PUSCH,

else

end if

where

-	 is the number of bits for CSI part 1;

-	if , ; otherwise  is the number of CRC bits for CSI part 1 determined according to Clause 6.3.1.2.1;

-	;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is no more than 2 bits, where  is the number of reserved resource elements for potential HARQ-ACK transmission in OFDM symbol , for , in the PUSCH transmission, defined in Clause 6.2.7;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, ;

-	 is the code rate of the PUSCH, determined according to Clause 6.1.4.1 of [6, TS38.214];

-	 is the modulation order of the PUSCH.

The input bit sequence to rate matching is  where  is the code block number, and  is the number of coded bits in code block number .

Rate matching is performed according to Clause 5.4.1 by setting  and the rate matching output sequence length to , where

-	 is the number of code blocks for UCI determined according to Clause 5.2.1;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	.

The output bit sequence after rate matching is denoted as  where  is the length of rate matching output sequence in code block number .

###### 6.3.2.4.1.3	CSI part 2

For CSI part 2 transmission on PUSCH with UL-SCH, the number of coded modulation symbols per layer for CSI part 2 transmission, denoted as , is determined as follows:

where

-	 is the number of bits for CSI part 2;

-	if , ; otherwise  is the number of CRC bits for CSI part 2 determined according to Clause 6.3.1.2.1;

-	;

-	 is the number of code blocks for UL-SCH of the PUSCH transmission;

-	if the DCI format scheduling the PUSCH transmission includes a CBGTI field indicating that the UE shall not transmit the -th code block, =0; otherwise, is the -th code block size for UL-SCH of the PUSCH transmission;

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is 1 or 2 bits;

-	 is the number of coded modulation symbols per layer for CSI part 1 transmitted on the PUSCH;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, .

-	 is configured by higher layer parameter scaling.

For CSI part 2 transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for CSI part 2 transmission, denoted as , is determined as follows:

where

-	 is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	 is the number of subcarriers in OFDM symbol  that carries PTRS, in the PUSCH transmission;

-	 is the number of coded modulation symbols per layer for HARQ-ACK transmitted on the PUSCH if number of HARQ-ACK information bits is more than 2, and  if the number of HARQ-ACK information bits is 1 or 2 bits;

-	 is the number of coded modulation symbols per layer for CSI part 1 transmitted on the PUSCH;

-	 is the number of resource elements that can be used for transmission of UCI in OFDM symbol , for , in the PUSCH transmission and  is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS;

-	for any OFDM symbol that carries DMRS of the PUSCH, ;

-	for any OFDM symbol that does not carry DMRS of the PUSCH, .

The input bit sequence to rate matching is  where  is the code block number, and  is the number of coded bits in code block number .

Rate matching is performed according to Clause 5.4.1 by setting  and the rate matching output sequence length to , where

-	 is the number of code blocks for UCI determined according to Clause 5.2.1;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	.

The output bit sequence after rate matching is denoted as  where  is the length of rate matching output sequence in code block number .

##### 6.3.2.4.2	UCI encoded by channel coding of small block lengths

###### 6.3.2.4.2.1	HARQ-ACK

For HARQ-ACK transmission on PUSCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined according to Clause 6.3.2.4.1.1, by setting the number of CRC bits .

The input bit sequence to rate matching is .

Rate matching is performed according to Clause 5.4.3, by setting the rate matching output sequence length , where

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as .

###### 6.3.2.4.2.2	CSI part 1

For CSI part 1 transmission on PUSCH, the number of coded modulation symbols per layer for CSI part 1 transmission, denoted as , is determined according to Clause 6.3.2.4.1.2, by setting the number of CRC bits .

Rate matching is performed according to Clause 5.4.3, by setting the rate matching output sequence length , where

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as .

###### 6.3.2.4.2.3	CSI part 2

For CSI part 2 transmission on PUSCH, the number of coded modulation symbols per layer for CSI part 2 transmission, denoted as , is determined according to Clause 6.3.2.4.1.3, by setting the number of CRC bits .

Rate matching is performed according to Clause 5.4.3, by setting the rate matching output sequence length , where

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH.

The output bit sequence after rate matching is denoted as .

#### 6.3.2.5	Code block concatenation

Code block concatenation is performed according to Clause 6.3.1.5, except that the values of  and  given in Clause 6.3.2.4.1.

#### 6.3.2.6	Multiplexing of coded UCI bits to PUSCH

The coded UCI bits are multiplexed onto PUSCH according to the procedures in Clause 6.2.7.

# 7	Downlink transport channels and control information

## 7.1	Broadcast channel

Data arrives to the coding unit in the form of a maximum of one transport block every 80ms. The following coding steps can be identified:

-	Payload generation

-	Scrambling

-	Transport block CRC attachment

-	Channel coding

-	Rate matching

### 7.1.1	PBCH payload generation

Denote the bits in a transport block delivered to layer 1 by , where  is the payload size generated by higher layers. The lowest order information bit  is mapped to the most significant bit of the transport block as defined in Clause 6.1.1 of [8, TS 38.321].

Generate the following additional timing related PBCH payload bits , where:

-	 are the 4th, 3rd, 2nd, and 1st LSB of SFN, respectively;

-	 is the half frame bit ;

-	if

are the 6th, 5th, and 4th bits of SS/PBCH block index, respectively.

else

is the MSB of  as defined in Clause 7.4.3.1 of [4, TS 38.211].

are reserved.

end if

Let ; ; ; ; ;

for  to

if  is an SFN bit

;

;

elseif  is the half radio frame bit

elseif

;

;

else

;

;

end if

end for

where  is the number of candidate SS/PBCH blocks in a half frame according to Clause 4.1 of [5, TS38.213], and the value of  is given by Table 7.1.1-1.

Table 7.1.1-1: Value of PBCH payload interleaver pattern

| 0 | 16 | 4 | 8 | 8 | 24 | 12 | 3 | 16 | 9 | 20 | 14 | 24 | 21 | 28 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 5 | 30 | 9 | 7 | 13 | 2 | 17 | 11 | 21 | 15 | 25 | 22 | 29 | 28 |
| 2 | 18 | 6 | 10 | 10 | 0 | 14 | 1 | 18 | 12 | 22 | 19 | 26 | 25 | 30 | 29 |
| 3 | 17 | 7 | 6 | 11 | 5 | 15 | 4 | 19 | 13 | 23 | 20 | 27 | 26 | 31 | 31 |

### 7.1.2	Scrambling

For PBCH transmission in a frame, the bit sequence  is scrambled into a bit sequence , where  for  and  is generated according to the following:

;

;

while

if  corresponds to any one of the bits belonging to the SS/PBCH block index, the half frame index, and 2nd and 3rd least significant bits of the system frame number

;

else

;

;

end if

;

end while

The scrambling sequence  is given by Clause 5.2.1of [4, TS38.211] and initialized with  at the start of each SFN satisfying ;  for  or , and  for , where  is the number of candidate SS/PBCH blocks in a half frame according to Clause 4.1 of [5, TS38.213]; and  is determined according to Table 7.1.2-1 using the 3rd and 2nd LSB of the SFN in which the PBCH is transmitted.

Table 7.1.2-1: Value of  for PBCH scrambling

| (3rd LSB of SFN, 2nd LSB of SFN) | Value of |
| --- | --- |
| (0, 0) | 0 |
| (0, 1) | 1 |
| (1, 0) | 2 |
| (1, 1) | 3 |

### 7.1.3	Transport block CRC attachment

Error detection is provided on BCH transport blocks through a Cyclic Redundancy Check (CRC).

The entire transport block is used to calculate the CRC parity bits. The input bit sequence is denoted by , and the parity bits by, where  is the payload size and  is the number of parity bits.

The parity bits are computed and attached to the BCH transport block according to Clause 5.1 by setting  to 24 bits and using the generator polynomial , resulting in the sequence, where .

The bit sequence  is the input bit sequence  to the channel encoder, where  for  and .

### 7.1.4	Channel coding

Information bits are delivered to the channel coding block. They are denoted by  , where  is the number of bits, and they are encoded via Polar coding according to Clause 5.3.1, by setting , , , and .

After encoding the bits are denoted by , where  is the number of coded bits.

### 7.1.5	Rate matching

The input bit sequence to rate matching is .

The rate matching output sequence length .

Rate matching is performed according to Clause 5.4.1 by setting .

The output bit sequence after rate matching is denoted as .

## 7.2	Downlink shared channel and paging channel

### 7.2.1	Transport block CRC attachment

Error detection is provided on each transport block through a Cyclic Redundancy Check (CRC).

The entire transport block is used to calculate the CRC parity bits. Denote the bits in a transport block delivered to layer 1 by, and the parity bits by, where  is the payload size and  is the number of parity bits. The lowest order information bit  is mapped to the most significant bit of the transport block as defined in Clause 6.1.1 of [TS38.321].

The parity bits are computed and attached to the DL-SCH transport block according to Clause 5.1, by setting  to 24 bits and using the generator polynomial  if ; and by setting  to 16 bits and using the generator polynomial  otherwise.

The bits after CRC attachment are denoted by , where .

### 7.2.2	LDPC base graph selection

For initial transmission of a transport block with coding rate  indicated by the MCS index according to Clause 5.1.3.1 in [6, TS 38.214] and subsequent re-transmission of the same transport block, each code block of the transport block is encoded with either LDPC base graph 1 or 2 according to the following:

-	if , or if  and , or if , LDPC base graph 2 is used;

-	otherwise, LDPC base graph 1 is used,

where  is the payload size in Clause 7.2.1.

### 7.2.3	Code block segmentation and code block CRC attachment

The bits input to the code block segmentation are denoted by  where  is the number of bits in the transport block (including CRC).

Code block segmentation and code block CRC attachment are performed according to Clause 5.2.2.

The bits after code block segmentation are denoted by, where  is the code block number and  is the number of bits for code block number  according to Clause 5.2.2.

### 7.2.4	Channel coding

Code blocks are delivered to the channel coding block. The bits in a code block are denoted by  , where  is the code block number, and  is the number of bits in code block number . The total number of code blocks is denoted by  and each code block is individually LDPC encoded according to Clause 5.3.2.

After encoding the bits are denoted by , where the values of  is given in Clause 5.3.2.

### 7.2.5	Rate matching

Coded bits for each code block, denoted as , are delivered to the rate match block, where  is the code block number, and  is the number of encoded bits in code block number . The total number of code blocks is denoted by  and each code block is individually rate matched according to Clause 5.4.2 by setting .

After rate matching, the bits are denoted by, where is the number of rate matched bits for code block number .

### 7.2.6	Code block concatenation

The input bit sequence for the code block concatenation block are the sequences , for  and where  is the number of rate matched bits for the -th code block.

Code block concatenation is performed according to Clause 5.5.

The bits after code block concatenation are denoted by, where  is the total number of coded bits for transmission.

## 7.3	Downlink control information

A DCI transports downlink control information for one or more cells with one RNTI.

The following coding steps can be identified:

-	Information element multiplexing

-	CRC attachment

-	Channel coding

-	Rate matching

### 7.3.1	DCI formats

The DCI formats defined in table 7.3.1-1 are supported.

Table 7.3.1-1: DCI formats

| DCI format | Usage |
| --- | --- |
| 0_0 | Scheduling of PUSCH in one cell |
| 0_1 | Scheduling of PUSCH in one cell |
| 1_0 | Scheduling of PDSCH in one cell |
| 1_1 | Scheduling of PDSCH in one cell |
| 2_0 | Notifying a group of UEs of the slot format |
| 2_1 | Notifying a group of UEs of the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE |
| 2_2 | Transmission of TPC commands for PUCCH and PUSCH |
| 2_3 | Transmission of a group of TPC commands for SRS transmissions by one or more UEs |

The fields defined in the DCI formats below are mapped to the information bits  to  as follows.

Each field is mapped in the order in which it appears in the description, including the zero-padding bit(s), if any, with the first field mapped to the lowest order information bit  and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to .

If the number of information bits in a DCI format is less than 12 bits, zeros shall be appended to the DCI format until the payload size equals 12.

The size of each DCI format is determined by the configuration of the corresponding active bandwidth part of the scheduled cell and shall be adjusted as described in clause 7.3.1.0 if necessary.

#### 7.3.1.0	DCI size alignment

If necessary, padding or truncation shall be applied to the DCI formats according to the following steps executed in the order below:

Step 0:

-	Determine DCI format 0_0 monitored in a common search space according to clause 7.3.1.1.1 where  is the size of the initial UL bandwidth part.

-	Determine DCI format 1_0 monitored in a common search space according to clause 7.3.1.2.1 where  is given by

-	the size of CORESET 0 if CORESET 0 is configured for the cell; and

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

-	If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

-	If DCI format 0_0 is monitored in common search space and if the number of information bits in the DCI format 0_0 prior to truncation is larger than the payload size of the DCI format 1_0 monitored in common search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 equals the size of the DCI format 1_0.

Step 1:

-	Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where  is the size of the active UL bandwidth part.

-	Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where  is the size of the active DL bandwidth part.

-	For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in DCI format 0_0 in UE-specific search space for the SUL is not equal to the number of information bits in DCI format 0_0 in UE-specific search space for the non-SUL, a number of zero padding bits are generated for the smaller DCI format 0_0 until the payload size equals that of the larger DCI format 0_0.

-	If DCI format 0_0 is monitored in UE-specific search space and if the number of information bits in the DCI format 0_0 prior to padding is less than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 until the payload size equals that of the DCI format 1_0.

-	If DCI format 1_0 is monitored in UE-specific search space and if the number of information bits in the DCI format 1_0 prior to padding is less than the payload size of the DCI format 0_0 monitored in UE-specific search space for scheduling the same serving cell, zeros shall be appended to the DCI format 1_0 until the payload size equals that of the DCI format 0_0

Step 2:

-	For a UE configured with supplementaryUplink in ServingCellConfig in a cell, if PUSCH is configured to be transmitted on both the SUL and the non-SUL of the cell and if the number of information bits in format 0_1 for the SUL is not equal to the number of information bits in format 0_1 for the non-SUL, zeros shall be appended to smaller format 0_1 until the payload size equals that of the larger format 0_1.

-	If the size of DCI format 0_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 0_1.

-	If the size of DCI format 1_1 monitored in a UE-specific search space equals that of a DCI format 0_0/1_0 monitored in another UE-specific search space, one bit of zero padding shall be appended to DCI format 1_1.

Step 3:

-	If both of the following conditions are fulfilled the size alignment procedure is complete

-	the total number of different DCI sizes configured to monitor is no more than 4 for the cell

-	the total number of different DCI sizes with C-RNTI configured to monitor is no more than 3 for the cell

Step 4:

-	Otherwise

-	Remove the padding bit (if any) introduced in step 2 above.

-	Determine DCI format 1_0 monitored in a UE-specific search space according to clause 7.3.1.2.1 where  is given by

-	the size of CORESET 0 if CORESET 0 is configured for the cell; and

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

-	Determine DCI format 0_0 monitored in a UE-specific search space according to clause 7.3.1.1.1 where  is the size of the initial UL bandwidth part.

-	If the number of information bits in the DCI format 0_0 monitored in a UE-specific search space prior to padding is less than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, a number of zero padding bits are generated for the DCI format 0_0 monitored in a UE-specific search space until the payload size equals that of the DCI format 1_0 monitored in a UE-specific search space.

-	If the number of information bits in the DCI format 0_0 monitored in a UE-specific search space prior to truncation is larger than the payload size of the DCI format 1_0 monitored in UE-specific search space for scheduling the same serving cell, the bitwidth of the frequency domain resource assignment field in the DCI format 0_0 is reduced by truncating the first few most significant bits such that the size of DCI format 0_0 monitored in a UE-specific search space equals the size of the DCI format 1_0 monitored in a UE-specific search space.

The UE is not expected to handle a configuration that, after applying the above steps, results in

-	the total number of different DCI sizes configured to monitor is more than 4 for the cell; or

-	the total number of different DCI sizes with C-RNTI configured to monitor is more than 3 for the cell; or

-	the size of DCI format 0_0 in a UE-specific search space is equal to DCI format 0_1 in another UE-specific search space; or

-	the size of DCI format 1_0 in a UE-specific search space is equal to DCI format 1_1 in another UE-specific search space

#### 7.3.1.1	DCI formats for scheduling of PUSCH

##### 7.3.1.1.1	Format 0_0

DCI format 0_0 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Frequency domain resource assignment –  bits where   is defined in clause 7.3.1.0

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214]

-	Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214]

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213]

-	Padding bits, if required.

-	UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured;

-	If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured.

-	If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Frequency domain resource assignment –bits where

-	 is the size of the initial UL bandwidth part.

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Clause 8.3 of [5, TS 38.213], where  if  and  otherwise

-	 bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 4 bits as defined in Clause 6.1.2.1 of [6, TS 38.214]

-	Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214]

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit, reserved

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits, reserved

-	TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213]

-	Padding bits, if required.

-	UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s).

-	If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB

Table 7.3.1.1.1-1: UL/SUL indicator

| Value of UL/SUL indicator | Uplink |
| --- | --- |
| 0 | The non-supplementary uplink |
| 1 | The supplementary uplink |

Table 7.3.1.1.1-2: Redundancy version

| Value of the Redundancy version field | Value of to be applied |
| --- | --- |
| 00 | 0 |
| 01 | 1 |
| 10 | 2 |
| 11 | 3 |

Table 7.3.1.1.1-3: Frequency hopping indication

| Bit field mapped to index | PUSCH frequency hopping |
| --- | --- |
| 0 | Disabled |
| 1 | Enabled |

##### 7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Carrier indicator – 0 or 3 bits, as defined in Clause 10.1 of [5, TS38.213].

-	UL/SUL indicator – 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only one carrier in the cell is configured for PUSCH transmission; otherwise, 1 bit as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active UL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Clause 6.1.2.2.1 of [6, TS 38.214],

-	bits if only resource allocation type 1 is configured, or  bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the  LSBs provide the resource allocation as defined in Clause 6.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as follows:

-	For PUSCH hopping with resource allocation type 1:

-	 MSB bits are used to indicate the frequency offset according to Clause 6.3 of [6, TS 38.214], where  if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and  if the higher layer parameter frequencyHoppingOffsetLists contains four offset values

-	 bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

-	For non-PUSCH hopping with resource allocation type 1:

-	 bits provides the frequency domain resource allocation according to Clause 6.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment"  field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Clause 6.1.2.1 of [6, TS38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pusch-TimeDomainAllocationList if the higher layer parameter is configured; otherwise I is the number of entries in the default table.

-	Frequency hopping flag – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if the higher layer parameter frequencyHopping is not configured;

-	1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214].

-	Modulation and coding scheme – 5 bits as defined in Clause 6.1.4.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	1st downlink assignment index – 1 or 2 bits:

-	1 bit for semi-static HARQ-ACK codebook;

-	2 bits for dynamic HARQ-ACK codebook.

-	2nd downlink assignment index – 0 or 2 bits:

-	2 bits for dynamic HARQ-ACK codebook with two HARQ-ACK sub-codebooks;

-	0 bit otherwise.

-	TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS38.213]

-	SRS resource indicator – or  bits, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook',

-	 bits according to Tables 7.3.1.1.2-28/29/30/31 if the higher layer parameter txConfig = nonCodebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'nonCodeBook' and

-	if UE supports operation with maxMIMO-Layers and the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, Lmax is given by that parameter

-	otherwise, Lmax is given by the maximum number of layers for PUSCH supported by the UE for the serving cell for non-codebook based operation.

-	 bits according to Tables 7.3.1.1.2-32 if the higher layer parameter txConfig = codebook, where  is the number of configured SRS resources in the SRS resource set associated with the higher layer parameter usage of value 'codeBook'.

-	Precoding information and number of layers – number of bits determined by the following:

-	0 bits if the higher layer parameter txConfig = nonCodeBook;

-	0 bits for 1 antenna port and if the higher layer parameter txConfig = codebook;

-	4, 5, or 6 bits according to Table 7.3.1.1.2-2 for 4 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset;

-	2, 4, or 5 bits according to Table 7.3.1.1.2-3 for 4 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank, and codebookSubset;

-	2 or 4 bits according to Table7.3.1.1.2-4 for 2 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset;

-	1 or 3 bits according to Table7.3.1.1.2-5 for 2 antenna ports, if txConfig = codebook, and according to whether transform precoder is enabled or disabled, and the values of higher layer parameters maxRank and codebookSubset.

-	Antenna ports – number of bits determined by the following

-	2 bits as defined by Tables 7.3.1.1.2-6, if transform precoder is enabled, dmrs-Type=1, and maxLength=1;

-	4 bits as defined by Tables 7.3.1.1.2-7, if transform precoder is enabled, dmrs-Type=1, and maxLength=2;

-	3 bits as defined by Tables 7.3.1.1.2-8/9/10/11, if transform precoder is disabled, dmrs-Type=1, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-12/13/14/15, if transform precoder is disabled, dmrs-Type=1, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	4 bits as defined by Tables 7.3.1.1.2-16/17/18/19, if transform precoder is disabled, dmrs-Type=2, and maxLength=1, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook;

-	5 bits as defined by Tables 7.3.1.1.2-20/21/22/23, if transform precoder is disabled, dmrs-Type=2, and maxLength=2, and the value of rank is determined according to the SRS resource indicator field if the higher layer parameter txConfig = nonCodebook and according to the Precoding information and number of layers field if the higher layer parameter txConfig = codebook.

where the number of CDM groups without data of values 1, 2, and 3 in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively.

If a UE is configured with both dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-UplinkForPUSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PUSCH corresponds to the smaller value of  and .

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	CSI request – 0, 1, 2, 3, 4, 5, or 6 bits determined by higher layer parameter reportTriggerSize.

-	CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PUSCH is not configured, otherwise, 2, 4, 6, or 8 bits determined by higher layer parameter maxCodeBlockGroupsPerTransportBlock for PUSCH.

-	PTRS-DMRS association – number of bits determined as follows

-	0 bit if PTRS-UplinkConfig is not configured and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1;

-	2 bits otherwise, where Table 7.3.1.1.2-25 and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field.

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the "PTRS-DMRS association" field is present for the indicated bandwidth part but not present for the active bandwidth part, the UE assumes the "PTRS-DMRS association" field is not present for the indicated bandwidth part.

-	beta_offset indicator – 0 if the higher layer parameter betaOffsets = semiStatic; otherwise 2 bits as defined by Table 9.3-3 in [5, TS 38.213].

-	DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled.

-	UL-SCH indicator – 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. Except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s).

A UE does not expect that the bit width of a field in DCI format 0_1 with CRC scrambled by CS-RNTI is larger than corresponding bit width of same field in DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell. If the bit width of a field in the DCI format 0_1 with CRC scrambled by CS-RNTI is not equal to that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell, a number of most significant bits with value set to '0' are inserted to the field in DCI format 0_1 with CRC scrambled by CS-RNTI until the bit width equals that of the corresponding field in the DCI format 0_1 with CRC scrambled by C-RNTI for the same serving cell.

Table 7.3.1.1.2-1: Bandwidth part indicator

| Value of BWP indicator field | Bandwidth part |
| --- | --- |
| 2 bits |  |
| 00 | Configured BWP with BWP-Id = 1 |
| 01 | Configured BWP with BWP-Id = 2 |
| 10 | Configured BWP with BWP-Id = 3 |
| 11 | Configured BWP with BWP-Id = 4 |

Table 7.3.1.1.2-2: Precoding information and number of layers, for 4 antenna ports, if transform precoder is disabled and maxRank = 2 or 3 or 4

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 | 4 | 2 layers: TPMI=0 |
| … | … | … | … | … | … |
| 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 | 9 | 2 layers: TPMI=5 |
| 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 | 10 | 3 layers: TPMI=0 |
| 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 | 11 | 4 layers: TPMI=0 |
| 12 | 1 layer: TPMI=4 | 12 | 1 layer: TPMI=4 | 12-15 | reserved |
| … | … | … | … |  |  |
| 19 | 1 layer: TPMI=11 | 19 | 1 layer: TPMI=11 |  |  |
| 20 | 2 layers: TPMI=6 | 20 | 2 layers: TPMI=6 |  |  |
| … | … | … | … |  |  |
| 27 | 2 layers: TPMI=13 | 27 | 2 layers: TPMI=13 |  |  |
| 28 | 3 layers: TPMI=1 | 28 | 3 layers: TPMI=1 |  |  |
| 29 | 3 layers: TPMI=2 | 29 | 3 layers: TPMI=2 |  |  |
| 30 | 4 layers: TPMI=1 | 30 | 4 layers: TPMI=1 |  |  |
| 31 | 4 layers: TPMI=2 | 31 | 4 layers: TPMI=2 |  |  |
| 32 | 1 layers: TPMI=12 |  |  |  |  |
| … | … |  |  |  |  |
| 47 | 1 layers: TPMI=27 |  |  |  |  |
| 48 | 2 layers: TPMI=14 |  |  |  |  |
| … | … |  |  |  |  |
| 55 | 2 layers: TPMI=21 |  |  |  |  |
| 56 | 3 layers: TPMI=3 |  |  |  |  |
| … | … |  |  |  |  |
| 59 | 3 layers: TPMI=6 |  |  |  |  |
| 60 | 4 layers: TPMI=3 |  |  |  |  |
| 61 | 4 layers: TPMI=4 |  |  |  |  |
| 62-63 | reserved |  |  |  |  |

Table 7.3.1.1.2-3: Precoding information and number of layers for 4 antenna ports, if transform precoder is enabled, or if transform precoder is disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset= partialAndNonCoherent | Bit field mapped to index | codebookSubset= nonCoherent |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| … | … | … | … | … | … |
| 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 | 3 | 1 layer: TPMI=3 |
| 4 | 1 layer: TPMI=4 | 4 | 1 layer: TPMI=4 |  |  |
| … | … | … | … |  |  |
| 11 | 1 layer: TPMI=11 | 11 | 1 layer: TPMI=11 |  |  |
| 12 | 1 layers: TPMI=12 | 12-15 | reserved |  |  |
| … | … |  |  |  |  |
| 27 | 1 layers: TPMI=27 |  |  |  |  |
| 28-31 | reserved |  |  |  |  |

Table 7.3.1.1.2-4: Precoding information and number of layers, for 2 antenna ports, if transform precoder is disabled and maxRank = 2

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 2 layers: TPMI=0 | 2 | 2 layers: TPMI=0 |
| 3 | 1 layer: TPMI=2 | 3 | reserved |
| 4 | 1 layer: TPMI=3 |  |  |
| 5 | 1 layer: TPMI=4 |  |  |
| 6 | 1 layer: TPMI=5 |  |  |
| 7 | 2 layers: TPMI=1 |  |  |
| 8 | 2 layers: TPMI=2 |  |  |
| 9-15 | reserved |  |  |

Table 7.3.1.1.2-5: Precoding information and number of layers, for 2 antenna ports, if transform precoder is enabled, or if transform precoder is disabled and maxRank = 1

| Bit field mapped to index | codebookSubset = fullyAndPartialAndNonCoherent | Bit field mapped to index | codebookSubset = nonCoherent |
| --- | --- | --- | --- |
| 0 | 1 layer: TPMI=0 | 0 | 1 layer: TPMI=0 |
| 1 | 1 layer: TPMI=1 | 1 | 1 layer: TPMI=1 |
| 2 | 1 layer: TPMI=2 |  |  |
| 3 | 1 layer: TPMI=3 |  |  |
| 4 | 1 layer: TPMI=4 |  |  |
| 5 | 1 layer: TPMI=5 |  |  |
| 6-7 | reserved |  |  |

Table 7.3.1.1.2-6: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0 |
| 1 | 2 | 1 |
| 2 | 2 | 2 |
| 3 | 2 | 3 |

Table 7.3.1.1.2-7: Antenna port(s), transform precoder is enabled, dmrs-Type=1, maxLength=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0 | 1 |
| 1 | 2 | 1 | 1 |
| 2 | 2 | 2 | 1 |
| 3 | 2 | 3 | 1 |
| 4 | 2 | 0 | 2 |
| 5 | 2 | 1 | 2 |
| 6 | 2 | 2 | 2 |
| 7 | 2 | 3 | 2 |
| 8 | 2 | 4 | 2 |
| 9 | 2 | 5 | 2 |
| 10 | 2 | 6 | 2 |
| 11 | 2 | 7 | 2 |
| 12-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-8: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6-7 | Reserved | Reserved |

Table 7.3.1.1.2-9: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4-7 | Reserved | Reserved |

Table 7.3.1.1.2-10: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 2-7 | Reserved | Reserved |

Table 7.3.1.1.2-11: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 2-7 | Reserved | Reserved |

Table 7.3.1.1.2-12: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-17: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7-15 | Reserved | Reserved |

Table 7.3.1.1.2-18: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank =3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3-15 | Reserved | Reserved |

Table 7.3.1.1.2-19: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=1, rank =4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2-15 | Reserved | Reserved |

Table 7.3.1.1.2-20: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23: Antenna port(s), transform precoder is disabled, dmrs-Type=2, maxLength=2, rank=4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-24: SRS request

| Value of SRS request field | Triggered aperiodic SRS resource set(s) for DCI format 0_1, 1_1, and 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' | Triggered aperiodic SRS resource set(s) for DCI format 2_3 configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA' |
| --- | --- | --- |
| 00 | No aperiodic SRS resource set triggered | No aperiodic SRS resource set triggered |
| 01 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 1 or an entry in aperiodicSRS-ResourceTriggerList set to 1 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 1st set of serving cells configured by higher layers |
| 10 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 2 or an entry in aperiodicSRS-ResourceTriggerList set to 2 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 2nd set of serving cells configured by higher layers |
| 11 | SRS resource set(s) configured with higher layer parameter aperiodicSRS-ResourceTrigger set to 3 or an entry in aperiodicSRS-ResourceTriggerList set to 3 | SRS resource set(s) configured with higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching' and resourceType in SRS-ResourceSet set to 'aperiodic' for a 3rd set of serving cells configured by higher layers |

Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port |
| 1 | 2nd scheduled DMRS port |
| 2 | 3rd scheduled DMRS port |
| 3 | 4th scheduled DMRS port |

Table 7.3.1.1.2-26: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port |  | Value of LSB | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 |  | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 |  | 1 | 2nd DMRS port which shares PTRS port 1 |

Table 7.3.1.1.2-27: void

Table 7.3.1.1.2-28: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 2 | 2 | 2 | 2 |
|  |  | 3 | reserved | 3 | 3 |

Table 7.3.1.1.2-29: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6-7 | reserved | 6 | 0,3 |
|  |  |  |  | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10-15 | reserved |

Table 7.3.1.1.2-30: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14-15 | reserved |

Table 7.3.1.1.2-31: SRI indication for non-codebook based PUSCH transmission,

| Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), | Bit field mapped to index | SRI(s), |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0,1 | 2 | 2 | 2 | 2 |
| 3 | reserved | 3 | 0,1 | 3 | 3 |
|  |  | 4 | 0,2 | 4 | 0,1 |
|  |  | 5 | 1,2 | 5 | 0,2 |
|  |  | 6 | 0,1,2 | 6 | 0,3 |
|  |  | 7 | reserved | 7 | 1,2 |
|  |  |  |  | 8 | 1,3 |
|  |  |  |  | 9 | 2,3 |
|  |  |  |  | 10 | 0,1,2 |
|  |  |  |  | 11 | 0,1,3 |
|  |  |  |  | 12 | 0,2,3 |
|  |  |  |  | 13 | 1,2,3 |
|  |  |  |  | 14 | 0,1,2,3 |
|  |  |  |  | 15 | reserved |

Table 7.3.1.1.2-32: SRI indication for codebook based PUSCH transmission

| Bit field mapped to index | SRI(s), |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

Table 7.3.1.1.2-33: Void

#### 7.3.1.2	DCI formats for scheduling of PDSCH

##### 7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment –  bits where  is given by clause 7.3.1.0

If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows:

-	Random Access Preamble index – 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321]

-	UL/SUL indicator – 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved

-	SS/PBCH index – 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved.

-	PRACH Mask index – 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved

-	Reserved bits – 10 bits

Otherwise, all remaining fields are set as follows:

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	TPC command for scheduled PUCCH – 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI:

-	Short Messages Indicator – 2 bits according to Table 7.3.1.2.1-1.

-	Short Messages – 8 bits, according to Clause 6.5 of [9, TS38.331]. If only the scheduling information for Paging is carried, this bit field is reserved.

-	Frequency domain resource assignment – bits.  If only the short message is carried, this bit field is reserved.

-	 is the size of CORESET 0

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]. If only the short message is carried, this bit field is reserved.

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5. If only the short message is carried, this bit field is reserved.

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1. If only the short message is carried, this bit field is reserved.

-	TB scaling – 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]. If only the short message is carried, this bit field is reserved.

-	Reserved bits – 6 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by SI-RNTI:

-	Frequency domain resource assignment – bits

-	 is the size of CORESET 0

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	System information indicator – 1 bit as defined in Table 7.3.1.2.1-2

-	Reserved bits – 15 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by RA-RNTI:

-	Frequency domain resource assignment – bits

-	 is the size of CORESET 0 if CORESET 0 is configured for the cell and  is the size of initial DL bandwidth part if CORESET 0 is not configured for the cell

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	TB scaling – 2 bits as defined in Clause 5.1.3.2 of [6, TS38.214]

-	Reserved bits – 16 bits

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI:

-	Identifier for DCI formats – 1 bit

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Frequency domain resource assignment – bits

-	 is the size of CORESET 0

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits, reserved

-	TPC command for scheduled PUCCH – 2 bits as defined in Clause 7.2.1 of [5, TS38.213]

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

Table 7.3.1.2.1-1: Short Message indicator

| Bit field | Short Message indicator |
| --- | --- |
| 00 | Reserved |
| 01 | Only scheduling information for Paging is present in the DCI |
| 10 | Only short message is present in the DCI |
| 11 | Both scheduling information for Paging and short message are present in the DCI |

Table 7.3.1.2.1-2: System information indicator

| Bit field | System information indicator |
| --- | --- |
| 0 | SIB1 [9, TS38.331, Clause 5.2.1] |
| 1 | SI message [9, TS38.331, Clause 5.2.1] |

##### 7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats – 1 bits

-	The value of this bit field is always set to 1, indicating a DL DCI format

-	Carrier indicator – 0 or 3 bits as defined in Clause 10.1 of [5, TS 38.213].

-	Bandwidth part indicator – 0, 1 or 2 bits as determined by the number of DL BWPs  configured by higher layers, excluding the initial DL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

-	Frequency domain resource assignment – number of bits determined by the following, where  is the size of the active DL bandwidth part:

-	 bits if only resource allocation type 0 is configured, where  is defined in Clause 5.1.2.2.1 of [6, TS38.214],

-	bits if only resource allocation type 1 is configured, or

-	 bits if both resource allocation type 0 and 1 are configured.

-	If both resource allocation type 0 and 1 are configured, the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the  LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and if both resource allocation type 0 and 1 are configured for the indicated bandwidth part, the UE assumes resource allocation type 0 for the indicated bandwidth part if the bitwidth of the "Frequency domain resource assignment" field of the active bandwidth part is smaller than the bitwidth of the "Frequency domain resource assignment" field of the indicated bandwidth part.

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter is configured; otherwise I is the number of entries in the default table.

-	VRB-to-PRB mapping – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if interleaved VRB-to-PRB mapping is not configured by high layers;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6  of [4, TS 38.211].

-	PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingType is not configured or is set to 'staticBundling', or 1 bit if the higher layer parameter prb-BundlingType is set to 'dynamicBundling' according to Clause 5.1.2.3 of [6, TS 38.214].

-	Rate matching indicator – 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

-	ZP CSI-RS trigger – 0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as bits, where  is the number of aperiodic ZP CSI-RS resource sets configured by higher layer.

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2):

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part and the value of maxNrofCodeWordsScheduledByDCI for the indicated bandwidth part equals 2 and the value of maxNrofCodeWordsScheduledByDCI for the active bandwidth part equals 1, the UE assumes zeros are padded when interpreting the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 according to Clause 12 of [5, TS38.213], and the UE ignores the "Modulation and coding scheme", "New data indicator", and "Redundancy version" fields of transport block 2 for the indicated bandwidth part.

-	HARQ process number – 4 bits

-	Downlink assignment index – number of bits as defined in the following

-	4 bits if more than one serving cell are configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 MSB bits are the counter DAI and the 2 LSB bits are the total DAI;

-	2 bits if only one serving cell is configured in the DL and the higher layer parameter pdsch-HARQ-ACK-Codebook=dynamic, where the 2 bits are the counter DAI;

-	0 bits otherwise.

If the UE is configured with a PUCCH-SCell, the number of serving cells is determined within a PUCCH group.

-	TPC command for scheduled PUCCH – 2 bits as defined in Clause 7.2.1 of [5, TS 38.213]

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK.

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports  shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals , where  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and  is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of  zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of  and .

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI is not enabled; otherwise 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

If "Bandwidth part indicator" field indicates a bandwidth part other than the active bandwidth part,

-	if the higher layer parameter tci-PresentInDCI is not enabled for the CORESET used for the PDCCH carrying the DCI format 1_1,

-	the UE assumes tci-PresentInDCI is not enabled for all CORESETs in the indicated bandwidth part;

-	otherwise,

-	the UE assumes tci-PresentInDCI is enabled for all CORESETs in the indicated bandwidth part.

-	SRS request – 2 bits as defined by Table 7.3.1.1.2-24 for UEs not configured with supplementaryUplink in ServingCellConfig in the cell; 3 bits for UEs configured with supplementaryUplink in ServingCellConfig in the cell where the first bit is the non-SUL/SUL indicator as defined in Table 7.3.1.1.1-1 and the second and third bits are defined by Table 7.3.1.1.2-24. This bit field may also indicate the associated CSI-RS according to Clause 6.1.1.2 of [6, TS 38.214].

-	CBG transmission information (CBGTI) – 0 bit if higher layer parameter codeBlockGroupTransmission for PDSCH is not configured, otherwise, 2, 4, 6, or 8 bits as defined in Clause 5.1.7 of [6, TS38.214], determined by the higher layer parameters maxCodeBlockGroupsPerTransportBlock and maxNrofCodeWordsScheduledByDCI for the PDSCH.

-	CBG flushing out information (CBGFI) – 1 bit if higher layer parameter codeBlockGroupFlushIndicator is configured as "TRUE", 0 bit otherwise.

-	DMRS sequence initialization – 1 bit.

If DCI formats 1_1 are monitored in multiple search spaces associated with multiple CORESETs in a BWP for scheduling the same serving cell, zeros shall be appended until the payload size of the DCI formats 1_1 monitored in the multiple search spaces equal to the maximum payload size of the DCI format 1_1 monitored in the multiple search spaces.

Table 7.3.1.2.2-1: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 1 | 0,1 |
| 3 | 2 | 0 |
| 4 | 2 | 1 |
| 5 | 2 | 2 |
| 6 | 2 | 3 |
| 7 | 2 | 0,1 |
| 8 | 2 | 2,3 |
| 9 | 2 | 0-2 |
| 10 | 2 | 0-3 |
| 11 | 2 | 0,2 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.2.2-2: Antenna port(s) (1000 + DMRS port), dmrs-Type=1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 2 | 0-4 | 2 |
| 1 | 1 | 1 | 1 | 1 | 2 | 0,1,2,3,4,6 | 2 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,4,5,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 4 | 2 | 1 | 1 | 4-31 | reserved | reserved | reserved |
| 5 | 2 | 2 | 1 |  |  |  |  |
| 6 | 2 | 3 | 1 |  |  |  |  |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 2 | 0,2 | 1 |  |  |  |  |
| 12 | 2 | 0 | 2 |  |  |  |  |
| 13 | 2 | 1 | 2 |  |  |  |  |
| 14 | 2 | 2 | 2 |  |  |  |  |
| 15 | 2 | 3 | 2 |  |  |  |  |
| 16 | 2 | 4 | 2 |  |  |  |  |
| 17 | 2 | 5 | 2 |  |  |  |  |
| 18 | 2 | 6 | 2 |  |  |  |  |
| 19 | 2 | 7 | 2 |  |  |  |  |
| 20 | 2 | 0,1 | 2 |  |  |  |  |
| 21 | 2 | 2,3 | 2 |  |  |  |  |
| 22 | 2 | 4,5 | 2 |  |  |  |  |
| 23 | 2 | 6,7 | 2 |  |  |  |  |
| 24 | 2 | 0,4 | 2 |  |  |  |  |
| 25 | 2 | 2,6 | 2 |  |  |  |  |
| 26 | 2 | 0,1,4 | 2 |  |  |  |  |
| 27 | 2 | 2,3,6 | 2 |  |  |  |  |
| 28 | 2 | 0,1,4,5 | 2 |  |  |  |  |
| 29 | 2 | 2,3,6,7 | 2 |  |  |  |  |
| 30 | 2 | 0,2,4,6 | 2 |  |  |  |  |
| 31 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-3: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | 2-31 | reserved | reserved |
| 3 | 2 | 0 |  |  |  |
| 4 | 2 | 1 |  |  |  |
| 5 | 2 | 2 |  |  |  |
| 6 | 2 | 3 |  |  |  |
| 7 | 2 | 0,1 |  |  |  |
| 8 | 2 | 2,3 |  |  |  |
| 9 | 2 | 0-2 |  |  |  |
| 10 | 2 | 0-3 |  |  |  |
| 11 | 3 | 0 |  |  |  |
| 12 | 3 | 1 |  |  |  |
| 13 | 3 | 2 |  |  |  |
| 14 | 3 | 3 |  |  |  |
| 15 | 3 | 4 |  |  |  |
| 16 | 3 | 5 |  |  |  |
| 17 | 3 | 0,1 |  |  |  |
| 18 | 3 | 2,3 |  |  |  |
| 19 | 3 | 4,5 |  |  |  |
| 20 | 3 | 0-2 |  |  |  |
| 21 | 3 | 3-5 |  |  |  |
| 22 | 3 | 0-3 |  |  |  |
| 23 | 2 | 0,2 |  |  |  |
| 24-31 | Reserved | Reserved |  |  |  |

Table 7.3.1.2.2-4: Antenna port(s) (1000 + DMRS port), dmrs-Type=2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | 0 | 3 | 0-4 | 1 |
| 1 | 1 | 1 | 1 | 1 | 3 | 0-5 | 1 |
| 2 | 1 | 0,1 | 1 | 2 | 2 | 0,1,2,3,6 | 2 |
| 3 | 2 | 0 | 1 | 3 | 2 | 0,1,2,3,6,8 | 2 |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,6,7,8 | 2 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 6 | 2 | 3 | 1 | 6-63 | Reserved | Reserved | Reserved |
| 7 | 2 | 0,1 | 1 |  |  |  |  |
| 8 | 2 | 2,3 | 1 |  |  |  |  |
| 9 | 2 | 0-2 | 1 |  |  |  |  |
| 10 | 2 | 0-3 | 1 |  |  |  |  |
| 11 | 3 | 0 | 1 |  |  |  |  |
| 12 | 3 | 1 | 1 |  |  |  |  |
| 13 | 3 | 2 | 1 |  |  |  |  |
| 14 | 3 | 3 | 1 |  |  |  |  |
| 15 | 3 | 4 | 1 |  |  |  |  |
| 16 | 3 | 5 | 1 |  |  |  |  |
| 17 | 3 | 0,1 | 1 |  |  |  |  |
| 18 | 3 | 2,3 | 1 |  |  |  |  |
| 19 | 3 | 4,5 | 1 |  |  |  |  |
| 20 | 3 | 0-2 | 1 |  |  |  |  |
| 21 | 3 | 3-5 | 1 |  |  |  |  |
| 22 | 3 | 0-3 | 1 |  |  |  |  |
| 23 | 2 | 0,2 | 1 |  |  |  |  |
| 24 | 3 | 0 | 2 |  |  |  |  |
| 25 | 3 | 1 | 2 |  |  |  |  |
| 26 | 3 | 2 | 2 |  |  |  |  |
| 27 | 3 | 3 | 2 |  |  |  |  |
| 28 | 3 | 4 | 2 |  |  |  |  |
| 29 | 3 | 5 | 2 |  |  |  |  |
| 30 | 3 | 6 | 2 |  |  |  |  |
| 31 | 3 | 7 | 2 |  |  |  |  |
| 32 | 3 | 8 | 2 |  |  |  |  |
| 33 | 3 | 9 | 2 |  |  |  |  |
| 34 | 3 | 10 | 2 |  |  |  |  |
| 35 | 3 | 11 | 2 |  |  |  |  |
| 36 | 3 | 0,1 | 2 |  |  |  |  |
| 37 | 3 | 2,3 | 2 |  |  |  |  |
| 38 | 3 | 4,5 | 2 |  |  |  |  |
| 39 | 3 | 6,7 | 2 |  |  |  |  |
| 40 | 3 | 8,9 | 2 |  |  |  |  |
| 41 | 3 | 10,11 | 2 |  |  |  |  |
| 42 | 3 | 0,1,6 | 2 |  |  |  |  |
| 43 | 3 | 2,3,8 | 2 |  |  |  |  |
| 44 | 3 | 4,5,10 | 2 |  |  |  |  |
| 45 | 3 | 0,1,6,7 | 2 |  |  |  |  |
| 46 | 3 | 2,3,8,9 | 2 |  |  |  |  |
| 47 | 3 | 4,5,10,11 | 2 |  |  |  |  |
| 48 | 1 | 0 | 2 |  |  |  |  |
| 49 | 1 | 1 | 2 |  |  |  |  |
| 50 | 1 | 6 | 2 |  |  |  |  |
| 51 | 1 | 7 | 2 |  |  |  |  |
| 52 | 1 | 0,1 | 2 |  |  |  |  |
| 53 | 1 | 6,7 | 2 |  |  |  |  |
| 54 | 2 | 0,1 | 2 |  |  |  |  |
| 55 | 2 | 2,3 | 2 |  |  |  |  |
| 56 | 2 | 6,7 | 2 |  |  |  |  |
| 57 | 2 | 8,9 | 2 |  |  |  |  |
| 58-63 | Reserved | Reserved | Reserved |  |  |  |  |

Table 7.3.1.2.2-5: VRB-to-PRB mapping

| Bit field mapped to index | VRB-to-PRB mapping |
| --- | --- |
| 0 | Non-interleaved |
| 1 | Interleaved |

#### 7.3.1.3	DCI formats for other purposes

##### 7.3.1.3.1	Format 2_0

DCI format 2_0 is used for notifying the slot format.

The following information is transmitted by means of the DCI format 2_0 with CRC scrambled by SFI-RNTI:

-	Slot format indicator 1, Slot format indicator 2, …, Slot format indicator N.

The size of DCI format 2_0 is configurable by higher layers up to 128 bits, according to Clause 11.1.1 of [5, TS 38.213].

##### 7.3.1.3.2	Format 2_1

DCI format 2_1 is used for notifying the PRB(s) and OFDM symbol(s) where UE may assume no transmission is intended for the UE.

The following information is transmitted by means of the DCI format 2_1 with CRC scrambled by INT-RNTI:

-	Pre-emption indication 1, Pre-emption indication 2, …, Pre-emption indication N.

The size of DCI format 2_1 is configurable by higher layers up to 126 bits, according to Clause 11.2 of [5, TS 38.213]. Each pre-emption indication is 14 bits.

##### 7.3.1.3.3	Format 2_2

DCI format 2_2 is used for the transmission of TPC commands for PUCCH and PUSCH.

The following information is transmitted by means of the DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI or TPC-PUCCH-RNTI:

-	block number 1, block number 2,…, block number N

The parameter tpc-PUSCH or tpc-PUCCH  provided by higher layers determines the index to the block number for an UL of a cell, with the following fields defined for each block:

-	Closed loop indicator   – 0 or 1 bit.

-	For DCI format 2_2 with TPC-PUSCH-RNTI, 0 bit if the UE is not configured with high layer parameter twoPUSCH-PC-AdjustmentStates, in which case UE assumes each block in the DCI format 2_2 is of 2 bits; 1 bit otherwise, in which case UE assumes each block in the DCI format 2_2 is of 3 bits;

-	For DCI format 2_2 with TPC-PUCCH-RNTI, 0 bit if the UE is not configured with high layer parameter twoPUCCH-PC-AdjustmentStates, in which case UE assumes each block in the DCI format 2_2 is of 2 bits; 1 bit otherwise, in which case UE assumes each block in the DCI format 2_2 is of 3 bits;

-	TPC command –2 bits

The number of information bits in format 2_2 shall be equal to or less than the payload size of format 1_0 monitored in common search space in the same serving cell. If the number of information bits in format 2_2 is less than the payload size of format 1_0 monitored in common search space in the same serving cell, zeros shall be appended to format 2_2 until the payload size equals that of format 1_0 monitored in common search space in the same serving cell.

##### 7.3.1.3.4	Format 2_3

DCI format 2_3 is used for the transmission of a group of TPC commands for SRS transmissions by one or more UEs. Along with a TPC command, a SRS request may also be transmitted.

The following information is transmitted by means of the DCI format 2_3 with CRC scrambled by TPC-SRS-RNTI:

-	block number 1, block number 2, …, block number

where the starting position of a block is determined by the parameter startingBitOfFormat2-3 or startingBitOfFormat2-3SUL-v1530 provided by higher layers for the UE configured with the block.

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeA for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block is configured for the UE by higher layers, with the following fields defined for the block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Clause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24.

-	TPC command number 1, TPC command number 2, ..., TPC command number N, where each TPC command applies to a respective UL carrier provided by higher layer parameter cc-IndexInOneCC-Set

If the UE is configured with higher layer parameter srs-TPC-PDCCH-Group = typeB for an UL without PUCCH and PUSCH or an UL on which the SRS power control is not tied with PUSCH power control, one block or more blocks is configured for the UE by higher layers where each block applies to an UL carrier, with the following fields defined for each block:

-	SRS request – 0 or 2 bits. The presence of this field is according to the definition in Clause 11.4 of [5, TS38.213]. If present, this field is interpreted as defined by Table 7.3.1.1.2-24.

-	TPC command –2 bits

The number of information bits in format 2_3 shall be equal to or less than the payload size of format 1_0 monitored in common search space in the same serving cell. If the number of information bits in format 2_3 is less than the payload size of format 1_0 monitored in common search space in the same serving cell, zeros shall be appended to format 2_3 until the payload size equals that of format 1_0 monitored in common search space in the same serving cell.

### 7.3.2	CRC attachment

Error detection is provided on DCI transmissions through a Cyclic Redundancy Check (CRC).

The entire payload is used to calculate the CRC parity bits. Denote the bits of the payload by, and the parity bits by, where  is the payload size and  is the number of parity bits. Let  be a bit sequence such that  for  and  for . The parity bits are computed with input bit sequence  and attached according to Clause 5.1 by setting  to 24 bits and using the generator polynomial . The output bit  is

for

for ,

where .

After attachment, the CRC parity bits are scrambled with the corresponding RNTI  , where  corresponds to the MSB of the RNTI, to form the sequence of bits . The relation between ck and bk is:

for k = 0, 1, 2, …,

for k = , ,,..., .

### 7.3.3	Channel coding

Information bits are delivered to the channel coding block. They are denoted by  , where  is the number of bits, and they are encoded via Polar coding according to Clause 5.3.1, by setting , , , and .

After encoding the bits are denoted by , where  is the number of coded bits.

### 7.3.4	Rate matching

The input bit sequence to rate matching is .

Rate matching is performed according to Clause 5.4.1 by setting .

The output bit sequence after rate matching is denoted as .

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-05 | RAN1#89 | R1-1707082 |  |  |  | Draft skeleton | 0.0.0 |
| 2017-07 | AH_NR2 | R1-1712014 |  |  |  | Inclusion of LDPC related agreements | 0.0.1 |
| 2017-08 | RAN1#90 | R1-1714564 |  |  |  | Inclusion of Polar coding related agreements | 0.0.2 |
| 2017-08 | RAN1#90 | R1-1714659 |  |  |  | Endorsed version by RAN1#90 as basis for further updates | 0.1.0 |
| 2017-09 | RAN1#90 | R1-1715322 |  |  |  | Capturing additional agreements on LDPC and Polar code from RAN1 #90 | 0.1.1 |
| 2017-09 | RAN#77 | RP-171991 |  |  |  | For information to plenary | 1.0.0 |
| 2017-09 | RAN1#90b | R1-1716928 |  |  |  | Capturing additional agreements on LDPC and Polar code from RAN1 NR AH#3 | 1.0.1 |
| 2017-10 | RAN1#90b | R1-1719106 |  |  |  | Endorsed as v1.1.0 | 1.1.0 |
| 2017-11 | RAN1#91 | R1-1719225 |  |  |  | Capturing additional agreements on channel coding, etc. | 1.1.1 |
| 2017-11 | RAN1#91 | R1-1719245 |  |  |  | Capturing additional agreements on DCI format, channel coding, etc. | 1.1.2 |
| 2017-11 | RAN1#91 | R1-1721049 |  |  |  | Endorsed as v1.2.0 | 1.2.0 |
| 2017-12 | RAN1#91 | R1-1721342 |  |  |  | Capturing additional agreements on UCI, DCI, channel coding, etc. | 1.2.1 |
| 2017-12 | RAN#78 | RP-172668 |  |  |  | Endorsed version for approval by plenary. | 2.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2018-03 | RAN#79 | RP-180200 | 0001 | - | F | CR capturing the Jan18 ad-hoc and RAN1#92 meeting agreements | 15.1.0 |
| 2018-04 | RAN#79 |  |  |  |  | MCC: correction of typo in DCI format 0_1 (time domain resource assignment) – higher layer parameter should be pusch-AllocationList | 15.1.1 |
| 2018-06 | RAN#80 | RP-181172 | 0002 | 1 | F | CR to 38.212 capturing the RAN1#92bis and RAN1#93 meeting agreements | 15.2.0 |
| 2018-06 | RAN#80 | RP-181257 | 0003 | - | B | CR to 38.212 capturing the RAN1#92bis and RAN1#93 meeting agreements related to URLLC | 15.2.0 |
| 2018-09 | RAN#81 | RP-181789 | 0004 | - | F | CR to 38.212 capturing the RAN1#94 meeting agreements | 15.3.0 |
| 2018-12 | RAN#82 | RP-182523 | 0005 | 3 | F | Combined CR of all essential corrections to 38.212 from RAN1#94bis and RAN1#95 | 15.4.0 |
| 2019-03 | RAN#83 | RP-190448 | 0006 | - | F | Correction of wrong implementation on frequency domain resource assignment bitwidth | 15.5.0 |
| 2019-03 | RAN#83 | RP-190448 | 0008 | - | F | Correction to UCI multiplexing | 15.5.0 |
| 2019-03 | RAN#83 | RP-190448 | 0009 | - | F | Correction on DCI format 2_3 for SUL cell in TS 38.212 | 15.5.0 |
| 2019-03 | RAN#83 | RP-190448 | 0010 | - | F | Corrections to TS38.212 | 15.5.0 |
| 2019-03 | RAN#83 | RP-190448 | 0011 | - | F | On bitwidth calculation for DCI fields using RRC parameter indicating maximum number of MIMO layers per serving cell | 15.5.0 |
| 2019-03 | RAN#83 | RP-190448 | 0012 | - | F | CR on zero-padding of DCI 1_1 in cross-carrier scheduling case | 15.5.0 |
| 2019-03 | RAN#83 | RP-190448 | 0013 | - | F | Clarification on UL_SUL indicator field and SRS request field | 15.5.0 |
| 2019-06 | RAN#84 | RP-191282 | 0014 | - | F | CR on correction to bitwidth of NNZC indicator | 15.6.0 |
| 2019-06 | RAN#84 | RP-191282 | 0015 | - | F | Correction on DCI size alignment in TS 38.212 | 15.6.0 |
| 2019-06 | RAN#84 | RP-191282 | 0016 | - | F | Correction on UL/SUL indicator in DCI format 0_0 | 15.6.0 |
| 2019-06 | RAN#84 | RP-191282 | 0017 | - | F | Corrections to 38.212 including alignment of terminology across specifications | 15.6.0 |
| 2019-06 | RAN#84 | RP-191282 | 0018 | - | F | CR on maximum modulation order configured for serving cell | 15.6.0 |
| 2019-06 | RAN#84 | RP-191282 | 0019 | 1 | F | Corrections to 38.212 including alignment of terminology across specifications from RAN1#97 | 15.6.0 |
| 2019-09 | RAN#85 | RP-191941 | 0020 | - | F | Corrections to 38.212 including alignment of terminology across specifications in RAN1#98 | 15.7.0 |
| 2019-12 | RAN#86 | RP-192625 | 0021 | - | F | CR on UL/SUL indicator in DCI format 0_1 | 15.8.0 |
| 2019-12 | RAN#86 | RP-192625 | 0022 | - | F | Corrections to 38.212 including alignment of terminology across specifications in RAN1#98bis and RAN1#99 | 15.8.0 |
| 2020-06 | RAN#88-e | RP-200683 | 0037 | - | F | CR on L1-RSRP report on PUSCH | 15.9.0 |
| 2020-09 | RAN#89-e | RP-201803 | 0048 | - | F | CR on PTRS for TS 38.212 | 15.10.0 |
| 2021-06 | RAN#92-e | RP-211233 | 0065 | - | F | 38.212 CR on DAI size determination for DCI format 1_1 in CA | 15.11.0 |