3GPP TS 38.214 V16.3.0 (2020-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Physical layer procedures for data

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, New Radio, Layer 1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions, symbols and abbreviations	8

3.1	Definitions	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Power control	9

4.1	Power allocation for downlink	9

5	Physical downlink shared channel related procedures	10

5.1	UE procedure for receiving the physical downlink shared channel	10

5.1.1	Transmission schemes	13

5.1.1.1	Transmission scheme 1	13

5.1.2	Resource allocation	13

5.1.2.1	Resource allocation in time domain	13

5.1.2.1.1	Determination of the resource allocation table to be used for PDSCH	16

5.1.2.2	Resource allocation in frequency domain	19

5.1.2.2.1	Downlink resource allocation type 0	20

5.1.2.2.2	Downlink resource allocation type 1	20

5.1.2.3	Physical resource block (PRB) bundling	21

5.1.3	Modulation order, target code rate, redundancy version and transport block size determination	23

5.1.3.1	Modulation order and target code rate determination	25

5.1.3.2	Transport block size determination	29

5.1.4	PDSCH resource mapping	32

5.1.4.1	PDSCH resource mapping with RB symbol level granularity	32

5.1.4.2	PDSCH resource mapping with RE level granularity	33

5.1.5	Antenna ports quasi co-location	35

5.1.6	UE procedure for receiving reference signals	38

5.1.6.1	CSI-RS reception procedure	38

5.1.6.1.1	CSI-RS for tracking	38

5.1.6.1.2	CSI-RS for L1-RSRP and L1-SINR computation	40

5.1.6.1.3	CSI-RS for mobility	40

5.1.6.2	DM-RS reception procedure	41

5.1.6.3	PT-RS reception procedure	43

5.1.6.4	SRS reception procedure for CLI	45

5.1.6.5	PRS reception procedure	45

5.1.7	Code block group based PDSCH transmission	48

5.1.7.1	UE procedure for grouping of code blocks to code block groups	48

5.1.7.2	UE procedure for receiving code block group based transmissions	48

5.2	UE procedure for reporting channel state information (CSI)	49

5.2.1	Channel state information framework	49

5.2.1.1	Reporting settings	49

5.2.1.2	Resource settings	50

5.2.1.3	(void)	51

5.2.1.4	Reporting configurations	51

5.2.1.4.1	Resource Setting configuration	53

5.2.1.4.2	Report Quantity Configurations	54

5.2.1.4.3	L1-RSRP Reporting	56

5.2.1.4.4	L1-SINR Reporting	56

5.2.1.5	Triggering/activation of CSI Reports and CSI-RS	57

5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology	57

5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies	59

5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS	60

5.2.1.6	CSI processing criteria	61

5.2.2	Channel state information	63

5.2.2.1	Channel quality indicator (CQI)	63

5.2.2.1.1	(void)	65

5.2.2.2	Precoding matrix indicator (PMI)	65

5.2.2.2.1	Type I Single-Panel Codebook	65

5.2.2.2.2	Type I Multi-Panel Codebook	71

5.2.2.2.3	Type II Codebook	75

5.2.2.2.4	Type II Port Selection Codebook	81

5.2.2.2.5	Enhanced Type II Codebook	84

5.2.2.2.6	Enhanced Type II Port Selection Codebook	91

5.2.2.3	Reference signal (CSI-RS)	92

5.2.2.3.1	NZP CSI-RS	92

5.2.2.4	Channel State Information – Interference Measurement (CSI-IM)	93

5.2.2.5	CSI reference resource definition	94

5.2.3	CSI reporting using PUSCH	96

5.2.4	CSI reporting using PUCCH	99

5.2.5	Priority rules for CSI reports	100

5.3	UE PDSCH processing procedure time	100

5.3.1	Application delay of the minimum scheduling offset restriction	102

5.4	UE CSI computation time	103

5.5	UE PDSCH reception preparation time with cross carrier scheduling with different subcarrier spacings for PDCCH and PDSCH	104

6	Physical uplink shared channel related procedure	105

6.1	UE procedure for transmitting the physical uplink shared channel	105

6.1.1	Transmission schemes	106

6.1.1.1	Codebook based UL transmission	106

6.1.1.2	Non-Codebook based UL transmission	108

6.1.2	Resource allocation	109

6.1.2.1	Resource allocation in time domain	109

6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH	113

6.1.2.2	Resource allocation in frequency domain	115

6.1.2.2.1	Uplink resource allocation type 0	116

6.1.2.2.2	Uplink resource allocation type 1	117

6.1.2.2.3	Uplink resource allocation type 2	118

6.1.2.3	Resource allocation for uplink transmission with configured grant	119

6.1.3	UE procedure for applying transform precoding on PUSCH	122

6.1.4	Modulation order, redundancy version and transport block size determination	122

6.1.4.1	Modulation order and target code rate determination	123

6.1.4.2	Transport block size determination	127

6.1.5	Code block group based PUSCH transmission	128

6.1.5.1	UE procedure for grouping of code blocks to code block groups	128

6.1.5.2	UE procedure for transmitting code block group based transmissions	129

6.1.6	Uplink switching	129

6.1.6.1	Uplink switching for EN-DC	129

6.1.6.2	Uplink switching for carrier aggregation	130

6.1.6.3	Uplink switching for supplementary uplink	131

6.2	UE reference signal (RS) procedure	131

6.2.1	UE sounding procedure	131

6.2.1.1	UE SRS frequency hopping procedure	136

6.2.1.2	UE sounding procedure for DL CSI acquisition	137

6.2.1.3	UE sounding procedure between component carriers	138

6.2.1.4	UE sounding procedure for positioning purposes	139

6.2.2	UE DM-RS transmission procedure	139

6.2.3	UE PT-RS transmission procedure	141

6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled	141

6.2.3.2	UE PT-RS transmission procedure when transform precoding is enabled	143

6.3	UE PUSCH frequency hopping procedure	144

6.3.1	Frequency hopping for PUSCH repetition Type A	144

6.3.2	Frequency hopping for PUSCH repetition Type B	145

6.4	UE PUSCH preparation procedure time	146

7	UE procedures for transmitting and receiving on a carrier with intra-cell guard bands	147

8	Physical sidelink shared channel related procedures	148

8.1	UE procedure for transmitting the physical sidelink shared channel	149

8.1.1	Transmission schemes	150

8.1.2	Resource allocation	150

8.1.2.1	Resource allocation in time domain	150

8.1.2.2	Resource allocation in frequency domain	150

8.1.3	Modulation order, target code rate, redundancy version and transport block size determination	151

8.1.3.1	Modulation order and target code rate determination	151

8.1.3.2	Transport block size determination	151

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2	152

8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A	155

8.1.6	Sidelink congestion control in sidelink resource allocation mode 2	156

8.1.7	UE procedure for determining the number of logical slots for a reservation period	156

8.2	UE procedure for transmitting sidelink reference signals	156

8.2.1	CSI-RS transmission procedure	156

8.2.2	PSSCH DM-RS transmission procedure	157

8.2.3	PT-RS transmission procedure	157

8.3	UE procedure for receiving the physical sidelink shared channel	157

8.4	UE procedure for receiving reference signals	158

8.4.1	CSI-RS reception procedure	158

8.4.2	DM-RS reception procedure for RSRP computation	158

8.4.3	PT-RS reception procedure	158

8.5	UE procedure for reporting channel state information (CSI)	158

8.5.1	Channel state information framework	158

8.5.1.1	Reporting configurations	158

8.5.1.2	Triggering of sidelink CSI reports	158

8.5.2	Channel state information	159

8.5.2.1	CSI reporting quantities	159

8.5.2.1.1	Channel quality indicator (CQI)	159

8.5.2.2	Reference signal (CSI-RS)	159

8.5.2.3	CSI reference resource definition	159

8.5.3	CSI reporting	160

8.6	UE PSSCH preparation procedure time	160

Annex A (informative): Change history	162

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies and establishes the characteristics of the physicals layer procedures of data channels for 5G-NR.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.201: " NR; Physical Layer – General Description"

[3]	3GPP TS 38.202: "NR; Services provided by the physical layer"

[4]	3GPP TS 38.211: "NR; Physical channels and modulation"

[5]	3GPP TS 38.212: "NR; Multiplexing and channel coding"

[6]	3GPP TS 38.213: "NR; Physical layer procedures for control"

[7]	3GPP TS 38.215: "NR; Physical layer measurements"

[8]	3GPP TS 38.101: "NR; User Equipment (UE) radio transmission and reception"

[9]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception"

[10]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification"

[11]	3GPP TS 38.133: "NR; Requirements for support of radio resource management"

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification"

[13]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities"

[14]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)"

[15]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation"

[16]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access"

[17]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)"

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BWP	Bandwidth part

CBG	Code block group

CLI	Cross Link Interference

CP	Cyclic prefix

CQI	Channel quality indicator

CPU	CSI processing unit

CRB	Common resource block

CRC	Cyclic redundancy check

CRI	CSI-RS Resource Indicator

CSI	Channel state information

CSI-RS	Channel state information reference signal

CSI-RSRP	CSI reference signal received power

CSI-RSRQ	CSI reference signal received quality

CSI-SINR	CSI signal-to-noise and interference ratio

CW	Codeword

DCI	Downlink control information

DL	Downlink

DM-RS	Dedicated demodulation reference signals

EPRE	Energy per resource element

IAB-MT	Integrated Access and Backhaul – Mobile Terminal

L1-RSRP	Layer 1 reference signal received power

LI	Layer Indicator

MCS	Modulation and coding scheme

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PSS	Primary Synchronisation signal

PUCCH	Physical uplink control channel

QCL	Quasi co-location

PMI	Precoding Matrix Indicator

PRB	Physical resource block

PRG	Precoding resource block group

PRS	Positioning reference signal

PT-RS	Phase-tracking reference signal

RB	Resource block

RBG	Resource block group

RI	Rank Indicator

RIV	Resource indicator value

RS	Reference signal

SLIV	Start and length indicator value

SR	Scheduling Request

SRS	Sounding reference signal

SS	Synchronisation signal

SSS	Secondary Synchronisation signal

SS-RSRP	SS reference signal received power

SS-RSRQ	SS reference signal received quality

SS-SINR	SS signal-to-noise and interference ratio

TB	Transport Block

TCI	Transmission Configuration Indicator

TDM	Time division multiplexing

UE	User equipment

UL	Uplink

# 4	Power control

Throughout this specification, unless otherwise noted, statements using the term "UE" in clauses 4, 5, or 6 are equally applicable to the IAB-MT part of an IAB node.

## 4.1	Power allocation for downlink

The gNB determines the downlink transmit EPRE.

For the purpose of SS-RSRP, SS-RSRQ and SS-SINR measurements, the UE may assume downlink EPRE is constant across the bandwidth. For the purpose of SS-RSRP, SS-RSRQ and SS-SINR measurements, the UE may assume downlink EPRE is constant over SSS carried in different SS/PBCH blocks. For the purpose of SS-RSRP, SS-RSRQ and SS-SINR measurements, the UE may assume that the ratio of SSS EPRE to PBCH DM-RS EPRE is 0 dB.

For the purpose of CSI-RSRP, CSI-RSRQ and CSI-SINR measurements, the UE may assume downlink EPRE of a port of CSI-RS resource configuration is constant across the configured downlink bandwidth and constant across all configured OFDM symbols.

The downlink SS/PBCH SSS EPRE can be derived from the SS/PBCH downlink transmit power given by the parameter ss-PBCH-BlockPower provided by higher layers. The downlink SSS transmit power is defined as the linear average over the power contributions (in [W]) of all resource elements that carry the SSS within the operating system bandwidth.

The downlink CSI-RS EPRE can be derived from the SS/PBCH block downlink transmit power given by the parameter ss-PBCH-BlockPower and CSI-RS power offset given by the parameter powerControlOffsetSS provided by higher layers. The downlink reference-signal transmit power is defined as the linear average over the power contributions (in [W]) of the resource elements that carry the configured CSI-RS within the operating system bandwidth.

For downlink DM-RS associated with PDSCH, the UE may assume the ratio of PDSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 4.1-1 according to the number of DM-RS CDM groups without data as described in Clause 5.1.6.2. The DM-RS scaling factor  specified in Clause 7.4.1.1.2 of [4, TS 38.211] is given by .

Table 4.1-1: The ratio of PDSCH EPRE to DM-RS EPRE

| Number of DM-RS CDM groups without data | DM-RS configuration type 1 | DM-RS configuration type 2 |
| --- | --- | --- |
| 1 | 0 dB | 0 dB |
| 2 | -3 dB | -3 dB |
| 3 | - | -4.77 dB |

When the UE is scheduled with one or two PT-RS ports associated with the PDSCH,

-	if the UE is configured with the higher layer parameter epre-Ratio, the ratio of PT-RS EPRE to PDSCH EPRE per layer per RE for each PT-RS port () is given by Table 4.1-2 according to the epre-Ratio, the PT-RS scaling factor specified in clause 7.4.1.2.2 of [4, TS 38.211] is given by.

-	otherwise, the UE shall assume epre-Ratio is set to state '0' in Table 4.1-2 if not configured.

Table 4.1-2: PT-RS EPRE to PDSCH EPRE per layer per RE ()

| epre-Ratio | The number of PDSCH layers with DM-RS associated to the PT-RS port |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| 0 | 0 | 3 | 4.77 | 6 | 7 | 7.78 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | reserved |  |  |  |  |  |
| 3 | reserved |  |  |  |  |  |

For link recovery, as described in clause 6 of [6, TS 38.213] the ratio of the PDCCH EPRE to NZP CSI-RS EPRE is assumed as 0 dB.

# 5	Physical downlink shared channel related procedures

## 5.1	UE procedure for receiving the physical downlink shared channel

For downlink, a maximum of 16 HARQ processes per cell is supported by the UE. The number of processes the UE may assume will at most be used for the downlink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPDSCH, and when no configuration is provided the UE may assume a default number of 8 processes.

A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1 or 1_2 decode the corresponding PDSCHs as indicated by that DCI. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH. The UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6]. In a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH-r16 if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. In a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration , with N=13 for =0, N=13 for =1, N=20 for =2, and N=24 for =3.

When receiving PDSCH scheduled with SI-RNTI or P-RNTI, the UE may assume that the DM-RS port of PDSCH is quasi co-located with the associated SS/PBCH block with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

When receiving PDSCH scheduled with RA-RNTI, or MsgB-RNTI, the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE used for RACH association as applicable, and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. When receiving a PDSCH scheduled with RA-RNTI in response to a random access procedure triggered by a PDCCH order which triggers contention-free random access procedure for the SpCell [10, TS 38.321], the UE may assume that the DM-RS port of the received PDCCH order and the DM-RS ports of the corresponding PDSCH scheduled with RA-RNTI are quasi co-located with the same SS/PBCH block or CSI-RS with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

When receiving PDSCH in response to a PUSCH transmission scheduled by a RAR UL grant or corresponding PUSCH retransmission, or when receiving PDSCH in response to a PUSCH for Type-2 random access procedure, or a PUSCH scheduled by a  fallbackRAR UL grant or corresponding PUSCH retransmission, the UE may assume that the DM-RS port of PDSCH is quasi co-located with the SS/PBCH block the UE selected for RACH association and transmission with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable.

If the UE is not configured for PUSCH/PUCCH transmission for at least one serving cell configured with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on serving cell c1 and serving cell c2, the UE is not expected to receive PDSCH on serving cell c1 if the PDSCH overlaps in time with SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) on serving cell c2 not configured for PUSCH/PUCCH transmission.

The UE is not expected to decode a PDSCH in a serving cell scheduled by a PDCCH with C-RNTI, CS-RNTI or MCS-C-RNTI and one or multiple PDSCH(s) required to be received according to this Clause in the same serving cell without a corresponding PDCCH transmission if the PDSCHs partially or fully overlap in time except if the PDCCH scheduling the PDSCH ends at least 14 symbols before the earliest starting symbol of the PDSCH(s) without the corresponding PDCCH transmission, where the symbol duration is based on the smallest numerology between the scheduling PDCCH and the PDSCH, in which case the UE shall decode the PDSCH scheduled by the PDCCH.

The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MsgB-RNTI partially or fully overlap in time.

The UE in RRC_IDLE and RRC_INACTIVE modes shall be able to decode two PDSCHs each scheduled with SI-RNTI, P-RNTI, RA-RNTI or TC-RNTI, with the two PDSCHs partially or fully overlapping in time in non-overlapping PRBs.

On a frequency range 1 cell, the UE shall be able to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI and, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI that partially or fully overlap in time in non-overlapping PRBs, unless the PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI requires Capability 2 processing time according to clause 5.3 in which case the UE may skip decoding of the scheduled PDSCH with C-RNTI, MCS-C-RNTI, or CS-RNTI.

On a frequency range 2 cell, the UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if in the same cell, during a process of P-RNTI triggered SI acquisition, another PDSCH scheduled with SI-RNTI partially or fully overlap in time.

The UE is expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI during a process of autonomous SI acquisition.

If the UE is configured by higher layers to decode a PDCCH with its CRC scrambled by a CS-RNTI, the UE shall receive PDSCH transmissions without corresponding PDCCH transmissions using the higher-layer-provided PDSCH configuration for those PDSCHs.

If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE may expect to receive multiple PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain. The UE may expect the reception of full/partially-overlapped PDSCHs in time only when PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex. For a ControlResourceSet without CORESETPoolIndex, the UE may assume that the ControlResourceSet is assigned with CORESETPoolIndex as 0. When the UE is scheduled with full/partially/non-overlapped PDSCHs in time and frequency domain, the full scheduling information for receiving a PDSCH is indicated and carried only by the corresponding PDCCH, the UE is expected to be scheduled with the same active BWP and the same SCS. When the UE is scheduled with full/partially-overlapped PDSCHs in time and frequency domain, the UE can be scheduled with at most two codewords simultaneously. When PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex, the following operations are allowed:

-	For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i.

-	In a given scheduled cell, the UE can receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH associated with a value of CORESETpoolindex different from that of the first PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j.

If PDCCHs that schedule corresponding PDSCHs are associated to the same or different ControlResourceSets having the same value of CORESETPoolIndex, the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1.

A UE does not expect to be configured with repetitionScheme-r16 if the UE is configured with higher layer parameter repetitionNumber-r16.

When a UE is configured by higher layer parameter RepetitionScheme-r16 set to one of 'FDMSchemeA', 'FDMSchemeB', 'TDMSchemeA', if the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)".

-	When two TCI states are indicated in a DCI and the UE is set to 'FDMSchemeA', the UE shall receive a single PDSCH transmission occasion of the TB with each TCI state associated to a non-overlapping frequency domain resource allocation as described in Clause 5.1.2.3.

-	When two TCI states are indicated in a DCI and the UE is set to 'FDMSchemeB', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping frequency domain resource allocation with respect to the other PDSCH transmission occasion as described in Clause 5.1.2.3.

-	When two TCI states are indicated in a DCI and the UE is set to 'TDMSchemeA', the UE shall receive two PDSCH transmission occasions of the same TB with each TCI state associated to a PDSCH transmission occasion which has non-overlapping time domain resource allocation with respect to the other PDSCH transmission occasion and both PDSCH transmission occasions shall be received within a given slot as described in Clause 5.1.2.1.

When a UE is configured by the higher layer parameter repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, the UE may expect to be indicated with one or two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' together with the DCI field "Time domain resource assignment' indicating an entry which contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16 and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)".

-	When two TCI states are indicated in a DCI with 'Transmission Configuration Indication' field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with two TCI states used across multiple PDSCH transmission occasions in the repetitionNumber-r16 consecutive slots as defined in Clause 5.1.2.1.

-	When one TCI state is indicated in a DCI with 'Transmission Configuration Indication' field, the UE may expect to receive multiple slot level PDSCH transmission occasions of the same TB with one TCI state used across multiple PDSCH transmission occasions in the repetitionNumber-r16 consecutive slots as defined in Clause 5.1.2.1.

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM groups in the DCI field "Antenna Port(s)", the UE may expect to receive a single PDSCH where the association between the DM-RS ports and the TCI states are as defined in Clause 5.1.6.2.

When a UE is not indicated with a DCI that DCI field 'Time domain resource assignment' indicating an entry which contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, and it is indicated with one TCI states in a codepoint of the DCI field 'Transmission Configuration Indication', the UE procedure for receiving the PDSCH upon detection of a PDCCH follows Clause 5.1.

If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below.

‒	Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot

‒	Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH.

‒	Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q.

‒	Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast PDSCHs in a slot supported by the UE

### 5.1.1	Transmission schemes

Only one transmission scheme is defined for the PDSCH, and is used for all PDSCH transmissions.

#### 5.1.1.1	Transmission scheme 1

For transmission scheme 1 of the PDSCH, the UE may assume that a gNB transmission on the PDSCH would be performed with up to 8 transmission layers on antenna ports 1000-1011 as defined in Clause 7.3.1.4 of [4, TS 38.211], subject to the DM-RS reception procedures in Clause 5.1.6.2.

### 5.1.2	Resource allocation

#### 5.1.2.1	Resource allocation in time domain

When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception.

Given the parameter values of the indexed row:

-	The slot allocated for the PDSCH is Ks, where , if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and  and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211].

-	The reference point S0 for starting symbol S is defined as:

-	if configured with referenceOfSLIVForDCI-Format1-2-r16, and when receiving PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0, and PDSCH mapping Type B, the starting symbol S is relative to the starting symbol S0 of the PDCCH monitoring occasion where DCI format 1_2 is detected;

-	otherwise, the starting symbol S is relative to the start of the slot using S0=0.

-	The number of consecutive symbols L counting from the starting symbol S allocated for the PDSCH are determined from the start and length indicator SLIV:

if  then

else

where, and

-	the PDSCH mapping type is set to Type A or Type B as defined in Clause 7.4.1.1.2 of [4, TS 38.211].

The UE shall consider the S and L combinations defined in table 5.1.2.1-1 satisfying  for normal cyclic prefix and  for extended cyclic prefix as valid PDSCH allocations:

Table 5.1.2.1-1: Valid S and L combinations

| PDSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A | {0,1,2,3} (Note 1) | {3,…,14} | {3,…,14} | {0,1,2,3} (Note 1) | {3,…,12} | {3,…,12} |
| Type B | {0,…,12} | {2,…,13} | {2,…,14} | {0,…,10} | {2,4,6} | {2,…,12} |
| Note 1: S = 3 is applicable only if dmrs-TypeA-Position = 3 |  |  |  |  |  |  |

When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, if the UE is configured with pdsch-AggregationFactor in pdsch-config, the same symbol allocation is applied across the pdsch-AggregationFactor consecutive slots. When receiving PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the same symbol allocation is applied across the pdsch-AggregationFactor, in sps-Config if configured or in pdsch-config otherwise, consecutive slots. The UE may expect that the TB is repeated within each symbol allocation among each of the pdsch-AggregationFactor consecutive slots and the PDSCH is limited to a single transmission layer. For PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the UE is not expected to be configured with the time duration for the reception of pdsch-AggregationFactor repetitions, in sps-Config if configured or in pdsch-config otherwise, larger than the time duration derived by the periodicity P obtained from the corresponding sps-Config. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, …pdsch-AggregationFactor -1, is determined according to table 5.1.2.1-2 and "rvid indicated by the DCI scheduling the PDSCH" in table 5.1.2.1-2 is assumed to be 0 for PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2.

If a UE is configured with higher layer parameter repetitionNumber-r16 or if the UE is configured by repetitionScheme-r16 set to one of 'FDMSchemeA', 'FDMSchemeB' and 'TDMSchemeA', the UE does not expect to be configured with pdsch-AggregationFactor or pdsch-AggregationFactor-r16.

Table 5.1.2.1-2: Applied redundancy version when pdsch-AggregationFactor is present

| rvid indicated by the DCI scheduling the PDSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

A PDSCH reception in a slot of a multi-slot PDSCH reception is omitted according to the conditions in Clause 11.1 of [6, TS38.213].

The UE is not expected to receive a PDSCH with mapping type A in a slot, if the PDCCH scheduling the PDSCH was received in the same slot and was not contained within the first three symbols of the slot.

The UE is not expected to receive a PDSCH with mapping type B in a slot, if the first symbol of the PDCCH scheduling the PDSCH was received in a later symbol than the first symbol indicated in the PDSCH time domain resource allocation.

When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 1_1 or DCI format 0_1 if the same field is available. When the UE is configured with minimumSchedulingOffsetK0 in an active DL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than  K0min⋅2μ'2μ, where K0min and μ are the applied minimum scheduling offset restriction and the numerology of the active DL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active DL BWP in case of active DL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used, in the search space set provided by recoverySearchSpaceId when monitoring PDCCH as described in [6, TS 38.213] or when PDSCH transmission is scheduled with SI-RNTI, MsgB-RNTI or RA-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

The UE is not expected to be configured with referenceOfSLIVForDCI-Format1-2-r16 for serving cells configured for cross-carrier scheduling with a scheduling cell of a different downlink SCS configuration.

When a UE is configured by the higher layer parameter RepetitionScheme-r16 set to 'TDMSchemeA' and indicated DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the number of PDSCH transmission occasions is derived by the number of TCI states indicated by the DCI field 'Transmission Configuration Indication' of the scheduling DCI.

-	If two TCI states are indicated by the DCI field 'Transmission Configuration Indication', the UE is expected to receive two PDSCH transmission occasions, where the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1. The second TCI state is applied to the second PDSCH transmission occasion, and the second PDSCH transmission occasion shall have the same number of symbols as the first PDSCH transmission occasion. If the UE is configured by the higher layers with a value K in StartingSymbolOffsetK, it shall determine that the first symbol of the second PDSCH transmission occasion starts after K symbols from the last symbol of the first PDSCH transmission occasion. If the value K is not configured via the higher layer parameter StartingSymbolOffsetK, K  = 0 shall be assumed by the UE. The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 applied respectively to the first and second TCI state. The UE expects the PDSCH mapping type indicated by DCI field "Time domain resource assignment' to be mapping type B, and the indicated PDSCH mapping type is applied to both PDSCH transmission occasions.

-	Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1.

When a UE configured by the higher layer parameter PDSCH-config that indicates at least one entry contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16,

-	If two TCI states are indicated by the DCI field 'Transmission Configuration Indication' together with the DCI field "Time domain resource assignment' indicating an entry which contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16 and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber-r16  consecutive slots, the first TCI state is applied to the first PDSCH transmission occasion and resource allocation in time domain for the first PDSCH transmission occasion follows Clause 5.1.2.1.

When the value indicated by repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16 equals to two, the second TCI state is applied to the second PDSCH transmission occasion. When the value indicated by repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16 is larger than two, the UE may be further configured to enable cyclicMapping or sequenticalMapping in tciMapping-r16.

-	When cyclicMapping is enabled, the first and second TCI states are applied to the first and second PDSCH transmission occasions, respectively, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions.

-	When sequenticalMapping is enabled, first TCI state is applied to the first and second PDSCH transmissions, and the second TCI state is applied to the third and fourth PDSCH transmissions, and the same TCI mapping pattern continues to the remaining PDSCH transmission occasions.

The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions associated with the first TCI state, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted only considering PDSCH transmission occasions associated with the first TCI state. The redundancy version for PDSCH transmission occasions associated with the second TCI state is derived according to Table 5.1.2.1-3, where additional shifting operation for each redundancy version rvs is configured by higher layer parameter sequenceOffsetforRV-r16 and n is counted only considering PDSCH transmission occasions associated with the second TCI state.

Table 5.1.2.1-3: Applied redundancy version for the second TCI state when sequenceOffsetforRV-r16 is present

| rvid indicated by the DCI scheduling the PDSCH | rvid to be applied to nth transmission occasion with second TCI state |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 |
| 2 | (2+ rvs) mod 4 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 |
| 3 | (3+ rvs) mod 4 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 |
| 1 | (1+ rvs) mod 4 | (0+ rvs) mod 4 | (2+ rvs) mod 4 | (3+ rvs) mod 4 |

-	If one TCI state is indicated by the DCI field 'Transmission Configuration Indication' together with the DCI field "Time domain resource assignment' indicating an entry which contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16 and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the same SLIV is applied for all PDSCH transmission occasions across the repetitionNumber-r16  consecutive slots, the first PDSCH transmission occasion follows Clause 5.1.2.1, the same TCI state is applied to all PDSCH transmission occasions. The UE may expect that each PDSCH transmission occasion is limited to two transmission layers. For all PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n is counted considering PDSCH transmission occasions.

-	Otherwise, the UE is expected to receive a single PDSCH transmission occasion, and the resource allocation in the time domain follows Clause 5.1.2.1.

##### 5.1.2.1.1	Determination of the resource allocation table to be used for PDSCH

Table 5.1.2.1.1-1 and Table 5.1.2.1.1-1A define which PDSCH time domain resource allocation configuration to apply. Either a default PDSCH time domain allocation A, B or C according to tables 5.1.2.1.1-2, 5.1.2.1.1-3, 5.1.2.1.1-4 and 5.1.2.1.1-5 is applied, or the higher layer configured pdsch-TimeDomainAllocationList or pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 is applied. For operation with shared spectrum channel access, as described in [16, TS 37.213], UE reinterprets S and L in row 9 of Table 5.1.2.1.1-2 as S=6 and L=7.

Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0 and 1_1

| RNTI | PDCCH search space | SS/PBCH block and CORESET multiplexing pattern | pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- | --- |
| SI-RNTI | Type0 common | 1 | - | - | Default A for normal CP |
|  |  | 2 | - | - | Default B |
|  |  | 3 | - | - | Default C |
| SI-RNTI | Type0A common | 1 | No | - | Default A |
|  |  | 2 | No | - | Default B |
|  |  | 3 | No | - | Default C |
|  |  | 1,2,3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| RA-RNTI, MsgB-RNTI, TC-RNTI | Type1 common | 1, 2, 3 | No | - | Default A |
|  |  | 1, 2, 3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| P-RNTI | Type2 common | 1 | No | - | Default A |
|  |  | 2 | No | - | Default B |
|  |  | 3 | No | - | Default C |
|  |  | 1,2,3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | 1, 2, 3 | No | - | Default A |
|  |  | 1, 2, 3 | Yes | - | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, CS-RNTI | Any common search space not associated with CORESET 0 UE specific search space | 1,2,3 | No | No | Default A |
|  |  | 1,2,3 | Yes | No | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
|  |  | 1,2,3 | No/Yes | Yes | pdsch-TimeDomainAllocationList provided in pdsch-Config |

Table 5.1.2.1.1-1A: Applicable PDSCH time domain resource allocation for DCI format 1_2

| pdsch-ConfigCommon includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationList | pdsch-Config includes pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 | PDSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pdsch-TimeDomainAllocationList provided in pdsch-ConfigCommon |
| No/Yes | Yes | No | pdsch-TimeDomainAllocationList provided in pdsch-Config |
| No/Yes | No/Yes | Yes | pdsch-TimeDomainAllocationListForDCI-Format1-2-r16 provided in pdsch-Config |

Table 5.1.2.1.1-2: Default PDSCH time domain resource allocation A for normal CP

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 2 | 2 | Type A | 0 | 2 | 10 |
|  | 3 | Type A | 0 | 3 | 9 |
| 3 | 2 | Type A | 0 | 2 | 9 |
|  | 3 | Type A | 0 | 3 | 8 |
| 4 | 2 | Type A | 0 | 2 | 7 |
|  | 3 | Type A | 0 | 3 | 6 |
| 5 | 2 | Type A | 0 | 2 | 5 |
|  | 3 | Type A | 0 | 3 | 4 |
| 6 | 2 | Type B | 0 | 9 | 4 |
|  | 3 | Type B | 0 | 10 | 4 |
| 7 | 2 | Type B | 0 | 4 | 4 |
|  | 3 | Type B | 0 | 6 | 4 |
| 8 | 2,3 | Type B | 0 | 5 | 7 |
| 9 | 2,3 | Type B | 0 | 5 | 2 |
| 10 | 2,3 | Type B | 0 | 9 | 2 |
| 11 | 2,3 | Type B | 0 | 12 | 2 |
| 12 | 2,3 | Type A | 0 | 1 | 13 |
| 13 | 2,3 | Type A | 0 | 1 | 6 |
| 14 | 2,3 | Type A | 0 | 2 | 4 |
| 15 | 2,3 | Type B | 0 | 4 | 7 |
| 16 | 2,3 | Type B | 0 | 8 | 4 |

Table 5.1.2.1.1-3: Default PDSCH time domain resource allocation A for extended CP

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | Type A | 0 | 2 | 6 |
|  | 3 | Type A | 0 | 3 | 5 |
| 2 | 2 | Type A | 0 | 2 | 10 |
|  | 3 | Type A | 0 | 3 | 9 |
| 3 | 2 | Type A | 0 | 2 | 9 |
|  | 3 | Type A | 0 | 3 | 8 |
| 4 | 2 | Type A | 0 | 2 | 7 |
|  | 3 | Type A | 0 | 3 | 6 |
| 5 | 2 | Type A | 0 | 2 | 5 |
|  | 3 | Type A | 0 | 3 | 4 |
| 6 | 2 | Type B | 0 | 6 | 4 |
|  | 3 | Type B | 0 | 8 | 2 |
| 7 | 2 | Type B | 0 | 4 | 4 |
|  | 3 | Type B | 0 | 6 | 4 |
| 8 | 2,3 | Type B | 0 | 5 | 6 |
| 9 | 2,3 | Type B | 0 | 5 | 2 |
| 10 | 2,3 | Type B | 0 | 9 | 2 |
| 11 | 2,3 | Type B | 0 | 10 | 2 |
| 12 | 2,3 | Type A | 0 | 1 | 11 |
| 13 | 2,3 | Type A | 0 | 1 | 6 |
| 14 | 2,3 | Type A | 0 | 2 | 4 |
| 15 | 2,3 | Type B | 0 | 4 | 6 |
| 16 | 2,3 | Type B | 0 | 8 | 4 |

Table 5.1.2.1.1-4: Default PDSCH time domain resource allocation B

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 | 2,3 | Type B | 0 | 2 | 2 |
| 2 | 2,3 | Type B | 0 | 4 | 2 |
| 3 | 2,3 | Type B | 0 | 6 | 2 |
| 4 | 2,3 | Type B | 0 | 8 | 2 |
| 5 | 2,3 | Type B | 0 | 10 | 2 |
| 6 | 2,3 | Type B | 1 | 2 | 2 |
| 7 | 2,3 | Type B | 1 | 4 | 2 |
| 8 | 2,3 | Type B | 0 | 2 | 4 |
| 9 | 2,3 | Type B | 0 | 4 | 4 |
| 10 | 2,3 | Type B | 0 | 6 | 4 |
| 11 | 2,3 | Type B | 0 | 8 | 4 |
| 12 (Note 1) | 2,3 | Type B | 0 | 10 | 4 |
| 13 (Note 1) | 2,3 | Type B | 0 | 2 | 7 |
| 14 (Note 1) | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 15 | 2,3 | Type B | 1 | 2 | 4 |
| 16 | Reserved |  |  |  |  |
| Note 1: If the PDSCH was scheduled with SI-RNTI in PDCCH Type0 common search space, the UE may assume that this PDSCH resource allocation is not applied |  |  |  |  |  |

Table 5.1.2.1.1-5: Default PDSCH time domain resource allocation C

| Row index | dmrs-TypeA-Position | PDSCH mapping type | K0 | S | L |
| --- | --- | --- | --- | --- | --- |
| 1 (Note 1) | 2,3 | Type B | 0 | 2 | 2 |
| 2 | 2,3 | Type B | 0 | 4 | 2 |
| 3 | 2,3 | Type B | 0 | 6 | 2 |
| 4 | 2,3 | Type B | 0 | 8 | 2 |
| 5 | 2,3 | Type B | 0 | 10 | 2 |
| 6 | Reserved |  |  |  |  |
| 7 | Reserved |  |  |  |  |
| 8 | 2,3 | Type B | 0 | 2 | 4 |
| 9 | 2,3 | Type B | 0 | 4 | 4 |
| 10 | 2,3 | Type B | 0 | 6 | 4 |
| 11 | 2,3 | Type B | 0 | 8 | 4 |
| 12 | 2,3 | Type B | 0 | 10 | 4 |
| 13 (Note 1) | 2,3 | Type B | 0 | 2 | 7 |
| 14 (Note 1) | 2 | Type A | 0 | 2 | 12 |
|  | 3 | Type A | 0 | 3 | 11 |
| 15 (Note 1) | 2,3 | Type A | 0 | 0 | 6 |
| 16 (Note 1) | 2,3 | Type A | 0 | 2 | 6 |
| Note 1: The UE may assume that this PDSCH resource allocation is not used, if the PDSCH was scheduled with SI-RNTI in PDCCH Type0 common search space |  |  |  |  |  |

#### 5.1.2.2	Resource allocation in frequency domain

Two downlink resource allocation schemes, type 0 and type 1, are supported. The UE shall assume that when the scheduling grant is received with DCI format 1_0, then downlink resource allocation type 1 is used.

If the scheduling DCI is configured to indicate the downlink resource allocation type as part of the Frequency domain resource assignment field by setting a higher layer parameter resourceAllocation in pdsch-Config to 'dynamicswitch', for DCI format 1_1 or setting a higher layer parameter resourceAllocationForDCI-Format1-2-r16 in pdsch-Config to 'dynamicswitch' for DCI format 1_2, the UE shall use downlink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the downlink frequency resource allocation type as defined by the higher layer parameter resourceAllocation for DCI format 1_1 or by the higher layer parameter resourceAllocationForDCI-Format1-2-r16 for DCI format 1_2.

If a bandwidth part indicator field is not configured in the scheduling DCI or the UE does not support active BWP change via DCI, the RB indexing for downlink type 0 and type 1 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI and the UE supports active BWP change via DCI, the RB indexing for downlink type 0 and type 1 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI. The UE shall upon detection of PDCCH intended for the UE determine first the downlink bandwidth part and then the resource allocation within the bandwidth part.

For a PDSCH scheduled with a DCI format 1_0 in any type of PDCCH common search space, regardless of which bandwidth part is the active bandwidth part, RB numbering starts from the lowest RB of the CORESET in which the DCI was received; otherwise RB numbering starts from the lowest RB in the determined downlink bandwidth part.

##### 5.1.2.2.1	Downlink resource allocation type 0

In downlink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured by PDSCH-Config and the size of the bandwidth part as defined in Table 5.1.2.2.1-1.

Table 5.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs () for a downlink bandwidth part i of size PRBs is given by , where

-	the size of the first RBG is ,

-	the size of last RBG is if  and P otherwise,

-	the size of all other RBGs is P.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency and starting at the lowest frequency of the bandwidth part. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

##### 5.1.2.2.2	Downlink resource allocation type 1

In downlink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved or interleaved virtual resource blocks within the active bandwidth part of size  PRBs except for the case when DCI format 1_0 is decoded in any common search space in which case the size of CORESET 0 shall be used if CORESET 0 is configured for the cell and the size of initial DL bandwidth part shall be used if CORESET 0 is not configured for the cell.

A downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

When the DCI size for DCI format 1_0 in USS is derived from the size of DCI format 1_0 in CSS but applied to an active BWP with size of , a downlink type 1 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block and a length in terms of virtually contiguously allocated resource blocks , where  is given by

-	the size of CORESET 0 if CORESET 0 is configured for the cell;

-	the size of initial DL bandwidth part if CORESET 0 is not configured for the cell.

The resource indication value is defined by:

if  then

else

where, and where shall not exceed .

If , K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1.

When the scheduling grant is received with DCI format 1_2, a downlink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting resource block group RBGstart=0, 1, …, NRBG-1 and a length in terms of virtually contiguously allocated resource block groups LRBGs=1, …, NRBG, where the resource block groups are defined as in 5.1.2.2.1 with P defined by resourceAllocationType1GranularityForDCI-Format1-2-r16 if the UE is configured with higher layer parameter resourceAllocationType1GranularityForDCI-Format1-2-r16, and P=1 otherwise. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

#### 5.1.2.3	Physical resource block (PRB) bundling

The PRB bundling procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of prb-BundlingTypeForDCI-Format1-2-r16 instead of prb-BundlingType as well as vrb-ToPRB-InterleaverForDCI-Format1-2-r16 instead of vrb-ToPRB-Interleaver.

A UE may assume that precoding granularity is  consecutive resource blocks in the frequency domain.  can be equal to one of the values among {2, 4, wideband}.

If  is determined as "wideband", the UE is not expected to be scheduled with non-contiguous PRBs and the UE may assume that the same precoding is applied to the allocated resource associated with a same TCI state or a same QCL assumption.

If  is determined as one of the values among {2, 4}, Precoding Resource Block Group (PRGs) partitions the bandwidth part i with  consecutive PRBs. Actual number of consecutive PRBs in each PRG could be one or more.

The first PRG size is given by  and the last PRG size given by  if , and the last PRG size is if .

The UE may assume the same precoding is applied for any downlink contiguous allocation of PRBs in a PRG.

For PDSCH carrying SIB1 scheduled by PDCCH with CRC scrambled by SI-RNTI, a PRG is partitioned from the lowest numbered resource block of CORESET 0 if the corresponding PDCCH is associated with CORESET 0 and Type0-PDCCH common search space and is addressed to SI-RNTI; otherwise, a PRG is partitioned from common resource block 0.

If a UE is scheduled a PDSCH with DCI format 1_0, the UE shall assume that  is equal to 2 PRBs.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI,  for bandwidth part is equal to 2 PRBs unless configured by the higher layer parameter prb-BundlingType given by PDSCH-Config.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'dynamicBundling', the higher layer parameters bundleSizeSet1 and bundleSizeSet2 configure two sets of  values, the first set can take one or two  values among {2, 4, wideband}, and the second set can take one  value among {2, 4, wideband}.

If the PRB bundling size indicator signalled in DCI format 1_1 as defined in Clause 7.3.1.2.2 of [5, TS 38.212]

-	is set to '0', the UE shall use the  value from the second set of  values when receiving PDSCH scheduled by the same DCI.

-	is set to '1' and one value is configured for the first set of  values, the UE shall use this  value when receiving PDSCH scheduled by the same DCI

-	is set to '1' and two values are configured for the first set of  values as 'n2-wideband' (corresponding to two  values 2 and wideband) or 'n4-wideband' (corresponding to two  values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows:

-	If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than ,  is the same as the scheduled bandwidth, otherwise  is set to the remaining configured value of 2 or 4, respectively.

When receiving PDSCH scheduled by PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI, if the higher layer parameter prb-BundlingType is set to 'staticBundling', the  value is configured with the single value indicated by the higher layer parameter bundleSize.

When a UE is configured with nominal RBG size P=2 for bandwidth part i according to Clause 5.1.2.2.1, or when a UE is configured with interleaving unit of 2 for VRB to PRB mapping provided by the higher layer parameter vrb-ToPRB-Interleaver given by PDSCH-Config for bandwidth part i, the UE is not expected to be configured with = 4.

For a UE configured by the higher layer parameter RepetitionScheme-r16 set to 'FDMSchemeA' or 'FDMSchemeB', and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)",

-	If  is determined as "wideband", the first nPRB2 PRBs are assigned to the first TCI state and the remaining nPRB2 PRBs are assigned to the second TCI state, where nPRB is the total number of allocated PRBs for the UE.

-	If  is determined as one of the values among {2, 4}, even PRGs within the allocated frequency domain resources are assigned to the first TCI state and odd PRGs within the allocated frequency domain resources are assigned to the second TCI state.

-	The UE is not expected to receive more than two PDSCH transmission layers for each PDSCH transmission occasion.

For a UE configured by the higher layer parameter RepetitionScheme-r16 set to 'FDMSchemeB', and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", each PDSCH transmission occasion shall follow the Clause 7.3.1 of [4, TS 38.211] with the mapping to resource elements determined by the assigned PRBs for corresponding TCI state of the PDSCH transmission occasion, and the UE shall only expect at most two code blocks per PDSCH transmission occasion when a single transmission layer is scheduled and a single code block per PDSCH transmission occasion when two transmission layers are scheduled. For two PDSCH transmission occasions, the redundancy version to be applied is derived according to Table 5.1.2.1-2, where n=0, 1 are applied to the first and second TCI state, respectively.

### 5.1.3	Modulation order, target code rate, redundancy version and transport block size determination

To determine the modulation order, target code rate, and transport block size(s) in the physical downlink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field (IMCS) in the DCI to determine the modulation order (Qm) and target code rate (R) based on the procedure defined in Clause 5.1.3.1, and

-	read redundancy version field (rv) in the DCI to determine the redundancy version.

and second

-	the UE shall use the number of layers (ʋ), the total number of allocated PRBs before rate matching (nPRB) to determine to the transport block size based on the procedure defined in Clause 5.1.3.2.

The UE may skip decoding a transport block in an initial transmission if the effective channel code rate is higher than 0.95, where the effective channel code rate is defined as the number of downlink information bits (including CRC bits) divided by the number of physical channel bits on PDSCH.

The UE is not expected to handle any transport blocks (TBs) in a 14 consecutive-symbol duration for normal CP (or 12 for extended CP) ending at the last symbol of the latest PDSCH transmission within an active BWP on a serving cell whenever

2max⁡(0,μ-μ').i∈SCi'Lixi.Fi>X4.1RLBRM.TBSLBRM

where, for the serving cell,

-	S is the set of TBs belonging to PDSCH(s) that are partially or fully contained in the consecutive-symbol duration

-	for the ith TB

-	Ci' is the number of scheduled code blocks for as defined in [5, 38.212].

-	Li is the number of OFDM symbols assigned to the PDSCH

-	xi is the number of OFDM symbols of the PDSCH contained in the consecutive-symbol duration

-	Fi=maxj=0,...,J-1(min(k0,ij+Eij, Ncb,i)) based on the values defined in Clause 5.4.2.1 [5, TS 38.212]

-	k0,ij is the starting location of RV for the jth transmission

-	Ei j=min(Er) of the scheduled code blocks for the jth transmission

-	Ncb,i is the circular buffer length

-	J-1 is the current (re)transmission for the ith TB

-	μ' corresponds to the subcarrier spacing of the BWP (across all configured BWPs of a carrier) that has the largest configured number of PRBs

-	in case there is more than one BWP corresponding to the largest configured number of PRBs, µ' follows the BWP with the largest subcarrier spacing.

-	μ corresponds to the subcarrier spacing of the active BWP

-	RLBRM = 2/3 as defined in Clause 5.4.2.1 [5, TS 38.212]

-	TBSLBRM as defined in Clause 5.4.2.1 [5, TS 38.212]

-	X as defined for downlink in Clause 5.4.2.1 [5, TS 38.212].

If the UE skips decoding, the physical layer indicates to higher layer that the transport block is not successfully decoded.

Within a cell group, a UE is not required to handle PDSCH(s) transmissions in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate

where,

-	J is the number of configured serving cells belonging to a frequency range

-	for the j-th serving cell,

-	M is the number of TB(s) transmitted in slot sj.

-	Tslot(j) =10-3/2(j), where (j) is the numerology for PDSCH(s) in slot sj of the j-th serving cell.

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 7.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212].

-	C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212]

-	DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i).

For a j-th serving cell, if higher layer parameter processingType2Enabled of PDSCH-ServingCellConfig is configured for the serving cell and set to enable, or if at least one IMCS > W for a PDSCH, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS table 5.1.3.1-2, the UE is not required to handle PDSCH transmissions, if the following condition is not satisfied:

m=0M-1Vj,mL×Tsμ≤DataRateCC

where

-	L is the number of symbols assigned to the PDSCH

-	M is the number of TB(s) in the PDSCH

-	Tsμ=10-32μ∙Nsymbslot where  is the numerology of the PDSCH

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 7.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212]

-	C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212]

-	DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i).

#### 5.1.3.1	Modulation order and target code rate determination

For the PDSCH scheduled by a PDCCH with DCI format 1_0, format 1_1 or format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI, or for the PDSCH scheduled without corresponding PDCCH transmissions using the higher-layer-provided PDSCH configuration SPS-Config,

if the higher layer parameter mcs-TableForDCI-Format1-2-r16 given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with MCS-C-RNTI, the higher layer parameter mcs-TableForDCI-Format1-2-r16 given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with DCI format 1_2 scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256', and the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with MCS-C-RNTI, the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam64LowSE', and the PDSCH is scheduled by a PDCCH with a DCI format other than DCI format 1_2 in a UE-specific search space with CRC scrambled by C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with MCS-C-RNTI, and the PDSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-config, and the higher layer parameter mcs-TableForDCI-Format1-2-r16 given by PDSCH-Config is set to 'qam256',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_2 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_2 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is not configured with the higher layer parameter mcs-Table given by SPS-Config, and the higher layer parameter mcs-Table given by PDSCH-Config is set to 'qam256',

-	if the PDSCH is scheduled by a PDCCH with DCI format 1_1 with CRC scrambled by CS-RNTI or

-	if the PDSCH with SPS activated by DCI format 1_1 is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

elseif the UE is configured with the higher layer parameter mcs-Table given by SPS-Config set to 'qam64LowSE'

-	if the PDSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if the PDSCH is scheduled without corresponding PDCCH transmission using SPS-Config,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel.

end

The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, SI-RNTI and Qm > 2

For a UE configured with FDMSchemeB, and when the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the determined modulation order of PDSCH transmission occasion associated with the first TCI state is applied to the PDSCH transmission occasion associated with the second TCI state.

Table 5.1.3.1-1: MCS index table 1 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 157 | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 18 | 6 | 466 | 2.7305 |
| 19 | 6 | 517 | 3.0293 |
| 20 | 6 | 567 | 3.3223 |
| 21 | 6 | 616 | 3.6094 |
| 22 | 6 | 666 | 3.9023 |
| 23 | 6 | 719 | 4.2129 |
| 24 | 6 | 772 | 4.5234 |
| 25 | 6 | 822 | 4.8164 |
| 26 | 6 | 873 | 5.1152 |
| 27 | 6 | 910 | 5.3320 |
| 28 | 6 | 948 | 5.5547 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Table 5.1.3.1-2: MCS index table 2 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 120 | 0.2344 |
| 1 | 2 | 193 | 0.3770 |
| 2 | 2 | 308 | 0.6016 |
| 3 | 2 | 449 | 0.8770 |
| 4 | 2 | 602 | 1.1758 |
| 5 | 4 | 378 | 1.4766 |
| 6 | 4 | 434 | 1.6953 |
| 7 | 4 | 490 | 1.9141 |
| 8 | 4 | 553 | 2.1602 |
| 9 | 4 | 616 | 2.4063 |
| 10 | 4 | 658 | 2.5703 |
| 11 | 6 | 466 | 2.7305 |
| 12 | 6 | 517 | 3.0293 |
| 13 | 6 | 567 | 3.3223 |
| 14 | 6 | 616 | 3.6094 |
| 15 | 6 | 666 | 3.9023 |
| 16 | 6 | 719 | 4.2129 |
| 17 | 6 | 772 | 4.5234 |
| 18 | 6 | 822 | 4.8164 |
| 19 | 6 | 873 | 5.1152 |
| 20 | 8 | 682.5 | 5.3320 |
| 21 | 8 | 711 | 5.5547 |
| 22 | 8 | 754 | 5.8906 |
| 23 | 8 | 797 | 6.2266 |
| 24 | 8 | 841 | 6.5703 |
| 25 | 8 | 885 | 6.9141 |
| 26 | 8 | 916.5 | 7.1602 |
| 27 | 8 | 948 | 7.4063 |
| 28 | 2 | reserved |  |
| 29 | 4 | reserved |  |
| 30 | 6 | reserved |  |
| 31 | 8 | reserved |  |

Table 5.1.3.1-3: MCS index table 3 for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | 2 | 30 | 0.0586 |
| 1 | 2 | 40 | 0.0781 |
| 2 | 2 | 50 | 0.0977 |
| 3 | 2 | 64 | 0.1250 |
| 4 | 2 | 78 | 0.1523 |
| 5 | 2 | 99 | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 4 | 340 | 1.3281 |
| 16 | 4 | 378 | 1.4766 |
| 17 | 4 | 434 | 1.6953 |
| 18 | 4 | 490 | 1.9141 |
| 19 | 4 | 553 | 2.1602 |
| 20 | 4 | 616 | 2.4063 |
| 21 | 6 | 438 | 2.5664 |
| 22 | 6 | 466 | 2.7305 |
| 23 | 6 | 517 | 3.0293 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 719 | 4.2129 |
| 28 | 6 | 772 | 4.5234 |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

#### 5.1.3.2	Transport block size determination

In case the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled, then one of the two transport blocks is disabled by DCI format 1_1 if IMCS = 26 and if rvid = 1 for the corresponding transport block. If both transport blocks are enabled, transport block 1 and 2 are mapped to codeword 0 and 1 respectively. If only one transport block is enabled, then the enabled transport block is always mapped to the first codeword.

For the PDSCH assigned by a PDCCH with DCI format 1_0, format 1_1 or format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or SI-RNTI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall, except if the transport block is disabled in DCI format 1_1, first determine the TBS as specified below:

1)	The UE shall first determine the number of REs (NRE) NRE)within the slot.

-	A UE first determines the number of REs allocated for PDSCH within a PRB () by , where is the number of subcarriers in a physical resource block, Nsymbslot is the number of symbols of the PDSCH allocation within the slot, NDMRSPRB is the number of REs for DM-RS per PRB in the scheduled duration including the overhead of the DM-RS CDM groups without data, as indicated by DCI format 1_1 or format 1_2 or as described for format 1_0 in Clause 5.1.6.2, and  NohPRBis the overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig. If the xOverhead in PDSCH-ServingCellconfig is not configured (a value from 0, 6, 12, or 18), the  is set to 0. If the PDSCH is scheduled by PDCCH with a CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI or P-RNTI,  is assumed to be 0.

-	A UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs for the UE.

2)	Unquantized intermediate variable (Ninfo) TBStemp) is obtained by TBStemp= NRE*R*Qm*ʋ.

If

Use step 3 as the next step of the TBS determination

else

Use step 4 as the next step of the TBS determination

end if

3)	When , TBS is determined as follows

-	quantized intermediate number of information bits , where .

-	use Table 5.1.3.2-1 find the closest TBS that is not less than .

Table 5.1.3.2-1: TBS for

| Index | TBS | Index | TBS | Index | TBS | Index | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 24 | 31 | 336 | 61 | 1288 | 91 | 3624 |
| 2 | 32 | 32 | 352 | 62 | 1320 | 92 | 3752 |
| 3 | 40 | 33 | 368 | 63 | 1352 | 93 | 3824 |
| 4 | 48 | 34 | 384 | 64 | 1416 |  |  |
| 5 | 56 | 35 | 408 | 65 | 1480 |  |  |
| 6 | 64 | 36 | 432 | 66 | 1544 |  |  |
| 7 | 72 | 37 | 456 | 67 | 1608 |  |  |
| 8 | 80 | 38 | 480 | 68 | 1672 |  |  |
| 9 | 88 | 39 | 504 | 69 | 1736 |  |  |
| 10 | 96 | 40 | 528 | 70 | 1800 |  |  |
| 11 | 104 | 41 | 552 | 71 | 1864 |  |  |
| 12 | 112 | 42 | 576 | 72 | 1928 |  |  |
| 13 | 120 | 43 | 608 | 73 | 2024 |  |  |
| 14 | 128 | 44 | 640 | 74 | 2088 |  |  |
| 15 | 136 | 45 | 672 | 75 | 2152 |  |  |
| 16 | 144 | 46 | 704 | 76 | 2216 |  |  |
| 17 | 152 | 47 | 736 | 77 | 2280 |  |  |
| 18 | 160 | 48 | 768 | 78 | 2408 |  |  |
| 19 | 168 | 49 | 808 | 79 | 2472 |  |  |
| 20 | 176 | 50 | 848 | 80 | 2536 |  |  |
| 21 | 184 | 51 | 888 | 81 | 2600 |  |  |
| 22 | 192 | 52 | 928 | 82 | 2664 |  |  |
| 23 | 208 | 53 | 984 | 83 | 2728 |  |  |
| 24 | 224 | 54 | 1032 | 84 | 2792 |  |  |
| 25 | 240 | 55 | 1064 | 85 | 2856 |  |  |
| 26 | 256 | 56 | 1128 | 86 | 2976 |  |  |
| 27 | 272 | 57 | 1160 | 87 | 3104 |  |  |
| 28 | 288 | 58 | 1192 | 88 | 3240 |  |  |
| 29 | 304 | 59 | 1224 | 89 | 3368 |  |  |
| 30 | 320 | 60 | 1256 | 90 | 3496 |  |  |

4)	When , TBS is determined as follows.

-	quantized intermediate number of information bits , where and ties in the round function are broken towards the next largest integer.

-	if

, where

else

if

, where

else

end if

end if

else if Table 5.1.3.1-2 is used and ,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤27. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤27, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using 0 ≤ IMCS ≤28. If there is no PDCCH for the same transport block using 0 ≤ IMCS ≤28, and if the initial PDSCH for the same transport block is semi-persistently scheduled, the TBS shall be determined from the most recent semi-persistent scheduling assignment PDCCH.

The UE is not expected to receive a PDSCH assigned by a PDCCH with CRC scrambled by SI-RNTI with a TBS exceeding 2976 bits.

For a UE configured with FDMSchemeB and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the TBS determination follows the steps 1-4 with the following modification in step 1: a UE determines the total number of REs allocated for PDSCH ()NRE) by NRE= NRE'* nPRB, where nPRB is the total number of allocated PRBs corresponding to the first TCI state, and the determined TBS of PDSCH transmission occasion associated with the first TCI state is also applied to the PDSCH transmission occasion associated with the second TCI state. For a UE configured with TDMSchemeA and indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the TBS determination follows the steps 1-4 with the following modification in step 1: a UE determines the number of REs allocated for PDSCH within a PRB () by , where  is the number of symbols of the PDSCH allocation within the slot corresponding to the first TCI state, and the determined TBS of PDSCH transmission occasion associated with the first TCI state is also applied to the PDSCH transmission occasion associated with the second TCI state.

For the PDSCH assigned by a PDCCH with DCI format 1_0 with CRC scrambled by P-RNTI, or RA-RNTI, MsgB-RNTI, TBS determination follows the steps 1-4 with the following modification in step 2: a scaling  is applied in the calculation of Ninfo, where the scaling factor is determined based on the TB scaling field in the DCI as in Table 5.1.3.2-2.

Table 5.1.3.2-2: Scaling factor of Ninfo for P-RNTI, RA-RNTI and MsgB-RNTI

| TB scaling field | Scaling factor S |
| --- | --- |
| 00 | 1 |
| 01 | 0.5 |
| 10 | 0.25 |
| 11 |  |

The NDI and HARQ process ID, as signalled on PDCCH, and the TBS, as determined above, shall be reported to higher layers.

### 5.1.4	PDSCH resource mapping

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH.

For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents all of the candidate SS/PBCH blocks corresponding to SS/PBCH block indices provided by ssb-PositionsInBurst as described in Clause 4.1 of [6, TS 38.213].

#### 5.1.4.1	PDSCH resource mapping with RB symbol level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of rateMatchPatternGroup1ForDCI-Format1-2-r16, rateMatchPatternGroup2ForDCI-Format1-2-r16 instead of rateMatchPatternGroup1 and rateMatchPatternGroup2.

A UE may be configured with any of the following higher layer parameters indicating REs declared as not available for PDSCH:

-	rateMatchPatternToAddModList given by PDSCH-Config, by ServingCellConfig or by ServingCellConfigCommon and configuring up to 4 RateMatchPattern(s) per BWP and up to 4 RateMatchPattern(s) per serving-cell. A RateMatchPattern may contain:

-	within a BWP, when provided by PDSCH-Config or within a serving cell when provided by ServingCellConfig or ServingCellConfigCommon, a pair of reserved resources with numerology provided by higher layer parameter subcarrierSpacing given by RateMatchPattern when configured per serving cell or by numerology of associated BWP when configured per BWP .The pair of reserved resources are respectively indicated by an RB level bitmap (higher layer parameter resourceBlocks given by RateMatchPattern ) with 1RB granularity and a symbol level bitmap spanning one or two slots (higher layer parameters symbolsInResourceBlock given by RateMatchPattern ) for which the reserved RBs apply. A bit value equal to 1 in the RB and symbol level bitmaps indicates that the corresponding resource is not available for PDSCH. For each pair of RB and symbol level bitmaps, a UE may be configured with a time-domain pattern (higher layer parameter periodicityAndPattern given by RateMatchPattern ), where each bit of periodicityAndPattern corresponds to a unit equal to a duration of the symbol level bitmap, and a bit value equal to 1 indicates that the pair is present in the unit. The periodicityAndPattern can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40ms. The first symbol of periodicityAndPattern every 40ms/P periods is a first symbol in frame nf mod 4 = 0, where P is the duration of periodicityAndPattern in units of ms. When periodicityAndPattern is not configured for a pair, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. The pair can be included in one or two groups of resource sets (higher layer parameters rateMatchPatternGroup1and rateMatchPatternGroup2). The rateMatchPatternToAddModList given by ServingCellConfig or ServingCellConfigCommon configuration in numerology µ applies only to PDSCH of the same numerology µ.

-	within a BWP, a frequency domain resource of a CORESET configured by ControlResourceSet with controlResourceSetId or ControlResourceSetZero and time domain resource determined by the higher layer parameters monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot of all search-space-sets configured by SearchSpace and time domain resource of search-space-set zero configured by searchSpaceZero associated with the CORESET as well as CORESET duration configured by ControlResourceSet with controlResourceSetId or ControlResourceSetZero. This resource not available for PDSCH can be included in one or two groups of resource sets (higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2).

A configured group rateMatchPatternGroup1 or rateMatchPatternGroup2 contains a list of indices of RateMatchPattern(s) forming a union of resource-sets not available for a PDSCH dynamically if a corresponding bit of the Rate matching indicator field of the DCI format 1_1 scheduling the PDSCH is equal to 1. The REs corresponding to the union of resource-sets configured by RateMatchPattern(s) that are not included in either of the two groups are not available for a PDSCH scheduled by a DCI format 1_0, a PDSCH scheduled by a DCI format 1_1, and PDSCHs with SPS. When receiving aPDSCH scheduled by a DCI format 1_0 or PDSCHs with SPS activated by a DCI format 1_0, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for the scheduled PDSCH.

For a bitmap pair included in one or two groups of resource sets, the dynamic indication of availability for PDSCH applies to a set of slot(s) where the rateMatchPatternToAddModList is present among the slots of scheduled PDSCH.

If a UE monitors PDCCH candidates of aggregation levels 8 and 16 with the same starting CCE index in non-interleaved CORESET spanning one OFDM symbol and if a detected PDCCH scheduling the PDSCH has aggregation level 8, the resources corresponding to the aggregation level 16 PDCCH candidate are not available for the PDSCH.

If a PDSCH scheduled by a PDCCH would overlap with resources in the CORESET containing the PDCCH, the resources corresponding to a union of the detected PDCCH that scheduled the PDSCH and associated PDCCH DM-RS are not available for the PDSCH. When precoderGranularity configured in a CORESET where the PDCCH was detected is equal to allContiguousRBs, the associated PDCCH DM-RS are DM-RS in all REGs of the CORESET. Otherwise, the associated DM-RS are the DM-RS in REGs of the PDCCH or the activated PDSCHs with SPS. When receiving PDSCHs with SPS activated by a DCI format 1_1, the REs corresponding to configured resources in rateMatchPatternGroup1 or rateMatchPatternGroup2 are not available for the PDSCHs with SPS if a corresponding bit of the Rate matching indicator field of the DCI format 1_1 activating the PDSCHs with SPS is equal to 1.

#### 5.1.4.2	PDSCH resource mapping with RE level granularity

The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListForDCI-Format1-2-r16 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList.

A UE may be configured with any of the following higher layer parameters:

-	REs indicated by the RateMatchingPatternLTE-CRS in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring common RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	REs indicated by RateMatchingPatternLTE-CRS in lte-CRS-PatternList-r16 in ServingCellConfig configuring common RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH.

-	Each RateMatchingPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe.

-	If the UE is configured by higher layer parameter PDCCH-Config with two different values of CORESETPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH:

-	if the UE is configured with crs-RateMatch-PerCORESETPoolIndex-r16, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with CORESETPoolIndex = 0, or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with CORESETPoolIndex = 1;

-	otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

-	Within a BWP, the UE can be configured with one or more ZP CSI-RS resource set configuration(s) for aperiodic, semi-persistent and periodic time-domain behaviours (higher layer parameters aperiodic-ZP-CSI-RS-ResourceSetsToAddModList,  sp-ZP-CSI-RS-ResourceSetsToAddModList and p-ZP-CSI-RS-ResourceSet respectively comprised in PDSCH-Config), with each ZP-CSI-RS resource set consisting of at most 16 ZP CSI-RS resources (higher layer parameter ZP-CSI-RS-Resource) in numerology of the BWP. The REs indicated by p-ZP-CSI-RS-ResourceSet are declared as not available for PDSCH. The REs indicated by sp-ZP-CSI-RS-ResourceSetsToAddModList and aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are declared as not available for PDSCH when their triggering and activation are applied, respectively. The following parameters are configured via higher layer signaling for each ZP CSI-RS resource configuration:

-	zp-CSI-RS-ResourceId in ZP-CSI-RS-Resource determines ZP CSI-RS resource configuration identity.

-	nrofPorts defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	cdm-Type defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	resourceMapping given by ZP-CSI-RS-Resource defines the OFDM symbol and subcarrier occupancy of the ZP-CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211].

-	periodicityAndOffset in ZP-CSI-RS-Resource defines the ZP-CSI-RS periodicity and slot offset for periodic/semi-persistent ZP-CSI-RS.

The UE may be configured with a DCI field for triggering the aperiodic ZP-CSI-RS. A list of ZP-CSI-RS-ResourceSet(s), provided by higher layer parameter aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config , is configured for aperiodic triggering. The maximum number of aperiodic ZP-CSI-RS-ResourceSet(s) configured per BWP is 3. The bit-length of DCI field ZP CSI-RS trigger depends on the number of aperiodic ZP-CSI-RS-ResourceSet(s)configured (up to 2 bits). Each non-zero codepoint of ZP CSI-RS trigger in DCI format 1_1 triggers one aperiodic ZP-CSI-RS-ResourceSet in the list aperiodic-ZP-CSI-RS-ResourceSetsToAddModList by indicating the aperiodic ZP CSI-RS resource set ID. The DCI codepoint '01' triggers the resource set with ZP-CSI-RS-ResourceSetIds = 1, the DCI codepoint '10' triggers the resource set with ZP-CSI-RS-ResourceSetIds = 2, and the DCI codepoint '11' triggers the resource set with ZP-CSI-RS-ResourceSetIds = 3. Codepoint '00' is reserved for not triggering aperiodic ZP CSI-RS. When receiving PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0, the REs corresponding to configured resources in aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are available for PDSCH.

When the UE is configured with multi-slot and single-slot PDSCH scheduling, the triggered aperiodic ZP CSI-RS is applied to all the slot(s) of the PDSCH scheduled or the PDSCHs with SPS activated by the PDCCH containing the trigger.

For a UE configured with a list of ZP-CSI-RS-ResourceSet(s) provided by higher layer parameter sp-ZP-CSI-RS-ResourceSetsToAddModList:

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, as described in clause 6.1.3.19 of [10, TS 38.321], for ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on the PDSCH RE mapping corresponding to the activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, as described in clause 6.1.3.19 of [10, TS 38.321], for activated ZP CSI-RS resource(s), the corresponding action in [10, TS 38.321] and the UE assumption on cessation of the PDSCH RE mapping corresponding to the de-activated ZP CSI-RS resource(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

### 5.1.5	Antenna ports quasi co-location

The UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability maxNumberConfiguredTCIstatesPerCC. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH, the DM-RS port of PDCCH or the CSI-RS port(s) of a CSI-RS resource. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values:

-	'QCL-TypeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'QCL-TypeB': {Doppler shift, Doppler spread}

-	'QCL-TypeC': {Doppler shift, average delay}

-	'QCL-TypeD': {Spatial Rx parameter}

The UE receives an activation command, as described in clause 6.1.3.14 of [10, TS 38.321], used to map up to 8 TCI states to the codepoints of the DCI field 'Transmission Configuration Indication' in one CC/DL BWP or in a set of CCs/DL BWPs, respectively. When a set of TCI state IDs are activated for a set of CCs/DL BWPs, where the applicable list of CCs is determined by indicated CC in the activation command, the same set of TCI state IDs are applied for all DL BWPs in the indicated CCs.

When a UE supports two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' the UE may receive an activation command, as described in clause 6.1.3.24 of [10, TS 38.321], the activation command is used to map up to 8 combinations of one or two TCI states to the codepoints of the DCI field 'Transmission Configuration Indication'. The UE is not expected to receive more than 8 TCI states in the activation command.

When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated mapping between TCI states and codepoints of the DCI field 'Transmission Configuration Indication' should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. If tci-PresentInDCI is set to "enabled" or tci-PresentForDCI-Format1-2-r16 is configured for the CORESET scheduling the PDSCH, and the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than timeDurationForQCL if applicable, after a UE receives an initial higher layer configuration of TCI states and before reception of the activation command, the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the SS/PBCH block determined in the initial access procedure with respect to 'QCL-TypeA', and when applicable, also with respect to'QCL-TypeD'.

If a UE is configured with the higher layer parameter tci-PresentInDCI that is set as 'enabled' for the CORESET scheduling the PDSCH, the UE assumes that the TCI field is present in the DCI format 1_1 of the PDCCH transmitted on the CORESET. If a UE is configured with the higher layer parameter tci-PresentForDCI-Format1-2-r16 for the CORESET scheduling the PDSCH, the UE assumes that the TCI field with a DCI field size indicated by tci-PresentForDCI-Format1-2-r16 is present in the DCI format 1_2 of the PDCCH transmitted on the CORESET. If the PDSCH is scheduled by a DCI format not having the TCI field present, and the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, where the threshold is based on reported UE capability [13, TS 38.306], for determining PDSCH antenna port quasi co-location, the UE assumes that the TCI state or the QCL assumption for the PDSCH is identical to the TCI state or QCL assumption whichever is applied for the CORESET used for the PDCCH transmission within the active BWP of the serving cell.

If the PDSCH is scheduled by a DCI format having the TCI field present, the TCI field in DCI in the scheduling component carrier points to the activated TCI states in the scheduled component carrier or DL BWP, the UE shall use the TCI-State according to the value of the 'Transmission Configuration Indication' field in the detected PDCCH with DCI for determining PDSCH antenna port quasi co-location. The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH. When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with [enableDefaultBeamForCSS], the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentForDCI-Format1-2-r16 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains 'QCL-TypeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and the corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL.

Independent of the configuration of tci-PresentInDCI and tci-PresentForDCI-Format1-2-r16 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains the 'QCL-TypeD',

-	the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. In this case, if the 'QCL-TypeD' of the PDSCH DM-RS is different from that of the PDCCH DM-RS with which they overlap in at least one symbol, the UE is expected to prioritize the reception of PDCCH associated with that CORESET. This also applies to the intra-band CA case (when PDSCH and the CORESET are in different component carriers).

-	If a UE is configured with enableDefaultTCIStatePerCoresetPoolIndex-r16 and the UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in different ControlResourceSets,

-	the UE may assume that the DM-RS ports of PDSCH associated with a value of CORESETPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH, in the latest slot in which one or more CORESETs associated with the same value of CORESETPoolIndex as the PDCCH scheduling that PDSCH within the active BWP of the serving cell are monitored by the UE.

-	If a UE is configured with enableTwoDefaultTCIStates-r16, and at least one TCI codepoint indicates two TCI states, the UE may assume that the DM-RS ports of PDSCH or PDSCH transmission occasions of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states. When the UE is configured by higher layer parameter repetitionScheme-r16 set to 'TDMSchemeA' or is configured with higher layer parameter repetitionNumber-r16, the mapping of the TCI states to PDSCH transmission occasions is determined according to clause 5.1.2.1 by replacing the indicated TCI states with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states based on the activated TCI states in the slot with the first PDSCH transmission occasion.

-	In all cases above, if none of configured TCI states for the serving cell of scheduled PDSCH contains 'QCL-TypeD', the UE shall obtain the other QCL assumptions from the indicated TCI states for its scheduled PDSCH irrespective of the time offset between the reception of the DL DCI and the corresponding PDSCH.

If the PDCCH carrying the scheduling DCI is received on one component carrier, and the PDSCH scheduled by that DCI is on another component carrier and the UE is configured with [enableDefaultBeamForCCS]:

-	The timeDurationForQCL is determined based on the subcarrier spacing of the scheduled PDSCH. If µPDCCH < µPDSCH an additional timing delay d2μPDSCH2μPDCCH is added to the timeDurationForQCL, where d is defined in 5.2.1.5.1a-1, otherwise d is zero;

-	For both the cases, when the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, and when the DL DCI does not have the TCI field present, the UE obtains its QCL assumption for the scheduled PDSCH from the activated TCI state with the lowest ID applicable to PDSCH in the active BWP of the scheduled cell.

For a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'QCL-TypeC' with an SS/PBCH block and, when applicable, 'QCL-TypeD' with the same SS/PBCH block, or

-	'QCL-TypeC' with an SS/PBCH block and, when applicable,'QCL-TypeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

For an aperiodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, the UE shall expect that a TCI-State indicates 'QCL-TypeA' with a periodic CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable,'QCL-TypeD' with the same periodic CSI-RS resource.

For a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with the same CSI-RS resource, or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with an SS/PBCH block , or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'QCL-TypeB' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info when 'QCL-TypeD' is not applicable.

For a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with the same CSI-RS resource, or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'QCL-TypeC' with an SS/PBCH block and, when applicable, 'QCL-TypeD' with the same SS/PBCH block.

For the DM-RS of PDCCH, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with the same CSI-RS resource, or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'QCL-TypeD' with the same CSI-RS resource.

For the DM-RS of PDSCH, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with the same CSI-RS resource, or

-	'QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'QCL-TypeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition,or

-	QCL-TypeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'QCL-TypeD' with the same CSI-RS resource.

### 5.1.6	UE procedure for receiving reference signals

#### 5.1.6.1	CSI-RS reception procedure

The CSI-RS defined in Clause 7.4.1.5 of [4, TS 38.211], may be used for time/frequency tracking, CSI computation, L1-RSRP computation, L1-SINR computation and mobility.

For a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'on', the UE shall not expect to be configured with CSI-RS over the symbols during which the UE is also configured to monitor the CORESET, while for other NZP-CSI-RS-ResourceSet configurations, if the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that the CSI-RS and a PDCCH DM-RS transmitted in all the search space sets associated with CORESET are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable. This also applies to the case when CSI-RS and the CORESET are in different intra-band component carriers, if 'QCL-TypeD' is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s).

The UE is not expected to receive CSI-RS and SIB1 message in the overlapping PRBs in the OFDM symbols where SIB1 is transmitted.

If the UE is configured with DRX,

-	if  the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported;

-	if the UE is configured to monitor DCI format 2_6 and configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to cri-RSRP when drx-onDurationTimer is not started, the most recent CSI measurement occasion occurs in DRX active time or during the time duration indicated by drx-onDurationTimer also outside DRX active time for CSI to be reported;

-	otherwise, the most recent CSI measurement occasion occurs in DRX active time for CSI to be reported.

##### 5.1.6.1.1	CSI-RS for tracking

A UE in RRC connected mode is expected to receive the higher layer UE specific configuration of a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info.

For a NZP-CSI-RS-ResourceSet configured with the higher layer parameter trs-Info, the UE shall assume the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same.

-	For frequency range 1, the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot. If no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, then the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic NZP CSI-RS resources in one slot.

-	For frequency range 2 the UE may be configured with one or more NZP CSI-RS set(s), where a NZP-CSI-RS-ResourceSet consists of two periodic CSI-RS resources in one slot or with a NZP-CSI-RS-ResourceSet of four periodic NZP CSI-RS resources in two consecutive slots with two periodic NZP CSI-RS resources in each slot.

A UE configured with NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-Info may have the CSI-RS resources configured as:

-	Periodic, with the CSI-RS resources in the NZP-CSI-RS-ResourceSet configured with same periodicity, bandwidth and subcarrier location

-	Periodic CSI-RS resource in one set and aperiodic CSI-RS resources in a second set, with the aperiodic CSI-RS and periodic CSI-RS resource having the same bandwidth (with same RB location) and the aperiodic CSI-RS being 'QCL-Type-A' and 'QCL-TypeD', where applicable, with the periodic CSI-RS resources. For frequency range 2, the UE does not expect that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is smaller than the UE reported beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}. The UE shall expect that the periodic CSI-RS resource set and aperiodic CSI-RS resource set are configured with the same number of CSI-RS resources and with the same number of CSI-RS resources in a slot. For the aperiodic CSI-RS resource set if triggered, and if the associated periodic CSI-RS resource set is configured with four periodic CSI-RS resources with two consecutive slots with two periodic CSI-RS resources in each slot, the higher layer parameter aperiodicTriggeringOffset indicates the triggering offset for the first slot for the first two CSI-RS resources in the set.

A UE does not expect to be configured with a CSI-ReportConfig that is linked to a CSI-ResourceConfig containing an NZP-CSI-RS-ResourceSet configured with trs-Info and with the CSI-ReportConfig configured with the higher layer parameter timeRestrictionForChannelMeasurements set to 'configured'.

A UE does not expect to be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to other than 'none' for aperiodic NZP CSI-RS resource set configured with trs-Info.

A UE does not expect to be configured with a CSI-ReportConfig for periodic NZP CSI-RS resource set configured with trs-Info.

A UE does not expect to be configured with a NZP-CSI-RS-ResourceSet configured both with trs-Info and repetition.

Each CSI-RS resource, defined in Clause 7.4.1.5.3 of [4, TS 38.211], is configured by the higher layer parameter NZP-CSI-RS-Resource with the following restrictions:

-	the time-domain locations of the two CSI-RS resources in a slot, or of the four CSI-RS resources in two consecutive slots (which are the same across two consecutive slots), as defined by higher layer parameter CSI-RS-resourceMapping, is given by one of

-	, , or for frequency range 1 and frequency range 2,

-	, , , , ,  or  for frequency range 2.

-	a single port CSI-RS resource with density  given by Table 7.4.1.5.3-1 from [4, TS 38.211] and higher layer parameter density configured by CSI-RS-ResourceMapping.

-	if carrier Ngridsize,μ=52, NBWP,isize=52, μ=0 and the carrier is configured in paired spectrum, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is X resource blocks, where X ≥ 28 resources if the UE indicates trs-AddBW-Set1 for the trs-AdditionalBandwidth-r16 capability and X ≥ 32 if the UE indicates trs-AddBW-Set2 for the AdditionalBandwidth-r16 capability; in these cases, if the UE is configured with CSI-RS comprising X<52 resource blocks, the UE does not expect that the total number of PRBs allocated for DL transmissions but not overlapped with the PRBs carrying CSI-RS for tracking is more than 4, where all CSI-RS resource configurations shall span the same set of resource blocks; otherwise, the bandwidth of the CSI-RS resource, as given by the higher layer parameter freqBand configured by CSI-RS-ResourceMapping, is the minimum of 52 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks. For operation with shared spectrum channel access, freqBand configured by CSI-RS-ResourceMapping, is the minimum of 48 and NBWP,isize resource blocks, or is equal to NBWP,isize resource blocks.

-	the UE is not expected to be configured with the periodicity of  slots if the bandwidth of CSI-RS resource is larger than 52 resource blocks.

-	the periodicity and slot offset for periodic NZP CSI-RS resources, as given by the higher layer parameter periodicityAndOffset configured by NZP-CSI-RS-Resource, is one of slots where 10, 20, 40, or 80 and where µ is defined in Clause 4.3 of [4, TS 38.211].

-	same powerControlOffset and powerControlOffsetSS given by NZP-CSI-RS-Resource value across all resources.

##### 5.1.6.1.2	CSI-RS for L1-RSRP and L1-SINR computation

If a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources, described in Clause 5.2.2.3.1, within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter, where the CSI-RS resources in the NZP-CSI-RS-ResourceSet are transmitted in different OFDM symbols. If repetition is set to 'off', the UE shall not assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter.

If the UE is configured with a CSI-ReportConfig with reportQuantity set to "cri-RSRP", "cri-SINR" or "none" and if the CSI-ResourceConfig for channel measurement (higher layer parameter resourcesForChannelMeasurement) contains a NZP-CSI-RS-ResourceSet that is configured with the higher layer parameter repetition and without the higher layer parameter trs-Info, the UE can only be configured with the same number (1 or 2) of ports with the higher layer parameter nrofPorts for all CSI-RS resources within the set. If the UE is configured with the CSI-RS resource in the same OFDM symbol(s) as an SS/PBCH block, the UE may assume that the CSI-RS and the SS/PBCH block are quasi co-located with 'QCL-TypeD' if 'QCL-TypeD' is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap with those of the SS/PBCH block, and the UE shall expect that the same subcarrier spacing is used for both the CSI-RS and the SS/PBCH block.

##### 5.1.6.1.3	CSI-RS for mobility

If a UE is configured with the higher layer parameter CSI-RS-Resource-Mobility and the higher layer parameter associatedSSB is not configured, the UE shall perform measurements based on CSI-RS-Resource-Mobility and the UE may base the timing of the CSI-RS resource on the timing of the serving cell.

If a UE is configured with the higher layer parameters CSI-RS-Resource-Mobility and associatedSSB, the UE may base the timing of the CSI-RS resource on the timing of the cell given by the cellId of the CSI-RS resource configuration. Additionally, for a given CSI-RS resource, if the associated SS/PBCH block is configured but not detected by the UE, the UE is not required to monitor the corresponding CSI-RS resource. The higher layer parameter isQuasiColocated indicates whether the associated SS/PBCH block given by the associatedSSB and the CSI-RS resource(s) are quasi co-located with respect to 'QCL-TypeD', when applicable.

If a UE is configured with the higher layer parameter CSI-RS-Resource-Mobility and with periodicity greater than 10 ms in paired spectrum, the UE may assume the absolute value of the time difference between radio frame i between any two cells, listed in the configuration with the higher layer parameter CSI-RS-CellMobility and with same refFreqCSI-RS, is less than 153600 Ts.

If the UE is configured with DRX, the UE is not required to perform measurement of CSI-RS resources other than during the active time for measurements based on CSI-RS-Resource-Mobility. When the UE is configured to monitor DCI format 2_6, the UE is not required to perform measurements other than during the active time and during the timer duration indicated by drx-onDurationTimer also outside active time based on CSI-RS-Resource-Mobility.

If the UE is configured with DRX and DRX cycle in use is larger than 80 ms, the UE may not expect CSI-RS resources are available other than during the active time for measurements based on CSI-RS-Resource-Mobility. If the UE is configured with DRX and configured to monitor DCI format 2_6 and DRX cycle in use is larger than 80ms, the UE may not expect that the CSI-RS resources are available other than during the active time and during the time duration indicated by drx-onDurationTimer also outside active time for measurements based on CSI-RS-Resource-Mobility. Otherwise, the UE may assume CSI-RS are available for measurements based on CSI-RS-Resource-Mobility.

A UE configured with the higher layer parameters CSI-RS-Resource-Mobility may expect to be configured

-	with no more than 96 CSI-RS resources per higher layer parameter MeasObjectNR when all CSI-RS resources configured by the same higher layer parameter MeasObjectNR have been configured with associatedSSB, or,

-	with no more than 64 CSI-RS resources per higher layer parameter MeasObjectNR when all CSI-RS resources have been configured without associatedSSB or when only some of the CSI-RS resources have been configured with associatedSSB by the same higher layer parameter MeasObjectNR

-	For frequency range 1 the associatedSSB is optionally present for each CSI-RS resource

-	For frequency range 2 the associatedSSB is either present for all configured CSI-RS resources or not present for any configured CSI-RS resource per higher layer parameter MeasObjectNR.

For any CSI-RS resource configuration, the UE shall assume that the value for parameter cdm-Type is 'noCDM', and there is only one antenna port.

#### 5.1.6.2	DM-RS reception procedure

The DM-RS reception procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of dmrs-DownlinkForPDSCH-MappingTypeA-ForDCI-Format1-2-r16 and dmrs-DownlinkForPDSCH-MappingTypeB-ForDCI-Format1-2-r16 instead of dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB.

When receiving PDSCH scheduled by DCI format 1_0 or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in clause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition

-	For PDSCH with mapping type A, the UE shall assume dmrs-AdditionalPosition='pos2' and up to two additional single-symbol DM-RS present in a slot according to the PDSCH duration indicated in the DCI as defined in Clause 7.4.1.1 of [4, TS 38.211], and

-	For PDSCH with allocation duration of 7 symbols for normal CP or 6 symbols for extended CP with mapping type B, the UE shall assume one additional single-symbol DM-RS present in the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol respectively of the PDSCH allocation duration, otherwise the UE shall assume that the additional DM-RS symbol is not present, and

-	For PDSCH with allocation duration of 4 symbols with mapping type B, the UE shall assume that no additional DM-RS are present, and

-	For PDSCH with allocation duration of 2 symbols with mapping type B, the UE shall assume that no additional DM-RS are present, and the UE shall assume that the PDSCH is present in the symbol carrying DM-RS.

When receiving PDSCH scheduled by DCI format 1_1 by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI,

-	the UE may be configured with the higher layer parameter dmrs-Type, and the configured DM-RS configuration type is used for receiving PDSCH in as defined in Clause 7.4.1.1 of [4, TS 38.211].

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PDSCH by higher layer parameter maxLength given by DMRS-DownlinkConfig..

-	if maxLength is set to 'len1', single-symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be set to 'pos0', 'pos1', 'pos2' or 'pos3'.

-	if maxLength is set to 'len2', both single-symbol DM-RS and double symbol DM-RS can be scheduled for the UE by DCI, and the UE can be configured with a number of additional DM-RS for PDSCH by higher layer parameter dmrs-AdditionalPosition, which can be set to 'pos0' or 'pos1'.

-	and the UE shall assume to receive additional DM-RS as specified in Table 7.4.1.1.2-3 and Table 7.4.1.1.2-4 as described in Clause 7.4.1.1.2 of [4, TS 38.211].

For the UE-specific reference signals generation as defined in Clause 7.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s), nIDDMRS,i i = 0,1 which are the same for both PDSCH mapping Type A and Type B.

A UE may be scheduled with a number of DM-RS ports by the antenna port index in DCI format 1_1 as described in Clause 7.3.1.2 of [5, TS 38.212].

For DM-RS configuration type 1,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

For DM-RS configuration type 2,

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10 or 23} in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-4 of Clause 7.3.1.2 of [5, TS38.212], or

-	if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10, 23 or 24} in Table 7.3.1.2.2-3A and {2, 10, 23 or 58} in Table 7.3.1.2.2-4A of Clause 7.3.1.2 of [5, TS 38.212], or

-	if a UE is scheduled with two codewords,

the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

If a UE receiving PDSCH scheduled by DCI format 1_2 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCI-Format1-2-r16 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCI-Format1-2-r16 or a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 is configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB, the UE may assume that the following configurations are not occurring simultaneously for the received PDSCH:

-	any DM-RS ports among 1004-1007 or 1006-1011 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and the other UE(s) sharing the DM-RS REs on the same CDM group(s), and

-	PT-RS is transmitted to the UE.

The UE is not expected to simultaneously be configured with the maximum number of front-loaded DM-RS symbols for PDSCH by higher layer parameter maxLength being set equal to 'len2' and more than one additional DM-RS symbol as given by the higher layer parameter dmrs-AdditionalPosition.

The UE is not expected to assume co-scheduled UE(s) with different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the actual number of additional DM-RS, the DM-RS symbol location, and DM-RS configuration type as described in Clause 7.4.1.1 of [4, TS 38.211].

The UE does not expect the precoding of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be different in the PRG-level grid configured to this UE with PRG =2 or 4.

The UE does not expect the resource allocation of the potential co-scheduled UE(s) in other DM-RS ports of the same CDM group to be misaligned in the PRG-level grid to this UE with PRG=2 or 4.

When receiving PDSCH scheduled by DCI format 1_1, the UE shall assume that the CDM groups indicated in the configured index from Tables 7.3.1.2.2-1, 7.3.1.2.2-1A, 7.3.1.2.2-2, 7.3.1.2.2-2A, 7.3.1.2.2-3, 7.3.1.2.2-3A, 7.3.1.2.2-4, 7.3.1.2.2-4A of [5, TS. 38.212] contain potential co-scheduled downlink DM-RS and are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) in Tables 7.3.1.2.2-1, 7.3.1.2.2-1A, 7.3.1.2.2-2, 7.3.1.2.2-3, 7.3.1.2.2-3A, 7.3.1.2.2-4, 7.3.1.2.2-4A of [5, TS. 38.212] correspond to CDM group 0, {0,1}, {0,1,2}, respectively.

When receiving PDSCH scheduled by DCI format 1_0, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PDSCH with allocation duration of 2 symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases.

The UE is not expected to receive PDSCH scheduling DCI which indicates CDM group(s) with potential DM-RS ports which overlap with any configured CSI-RS resource(s) for that UE.

If the UE receives the DM-RS for PDSCH and an SS/PBCH block in the same OFDM symbol(s), then the UE may assume that the DM-RS and SS/PBCH block are quasi co-located with 'QCL-TypeD', if 'QCL-TypeD' is applicable. Furthermore, the UE shall not expect to receive DM-RS in resource elements that overlap with those of the SS/PBCH block, and the UE can expect that the same or different subcarrier spacing is configured for the DM-RS and SS/PBCH block in a CC except for the case of 240 kHz where only different subcarrier spacing is supported.

If a UE is configured by the higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet, the UE may be scheduled with fully or partially overlapping PDSCHs in the time and frequency domain by multiple PDCCHs with the following restrictions,

-	the UE is not expected to assume different DM-RS configuration with respect to the actual number of front-loaded DM-RS symbol(s), the actual number of additional DM-RS symbol(s), the actual DM-RS symbol location, and DM-RS configuration type.

-	the UE is not expected to assume DM-RS ports in a CDM group indicated by two TCI states.

When a UE is not indicated with a DCI that DCI field "Time domain resource assignment' indicating an entry which contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16 and it is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM groups in the DCI field "Antenna Port(s)",

-	the first TCI state corresponds to the CDM group of the first antenna port indicated by the antenna port indication table, and the second TCI state corresponds to the other CDM group.

#### 5.1.6.3	PT-RS reception procedure

The procedures on PT-RS reception described in this clause apply to a UE receiving PDSCH scheduled by DCI format 1_2 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA-ForDCI-Format1-2-r16 or dmrs-DownlinkForPDSCH-MappingTypeB-ForDCI-Format1-2-r16 and to a UE receiving PDSCH scheduled by DCI format 1_0 or DCI format 1_1 configured with the higher layer parameter phaseTrackingRS in dmrs-DownlinkForPDSCH-MappingTypeA or dmrs-DownlinkForPDSCH-MappingTypeB.

A UE shall report the preferred MCS and bandwidth thresholds based on the UE capability at a given carrier frequency, for each subcarrier spacing applicable to data channel at this carrier frequency, assuming the MCS table with the maximum Modulation Order as it reported to support.

If a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig,

-	the higher layer parameters timeDensity and frequencyDensity in PTRS-DownlinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, respectively.

-	if either or both of the additional higher layer parameters timeDensity and frequencyDensity are configured, and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS antenna port' presence and pattern is a function of the corresponding scheduled MCS of the corresponding codeword and scheduled bandwidth in corresponding bandwidth part as shown in Table 5.1.6.3-1 and Table 5.1.6.3-2,

-	if the higher layer parameter timeDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume LPT-RS = 1.

-	if the higher layer parameter frequencyDensity given by PTRS-DownlinkConfig is not configured, the UE shall assume KPT-RS = 2.

-	otherwise, if neither of the additional higher layer parameters timeDensity and frequencyDensity are configured and the RNTI equals MCS-C-RNTI, C-RNTI or CS-RNTI, the UE shall assume the PT-RS is present with LPT-RS = 1, KPT-RS = 2, and the UE shall assume PT-RS is not present when

-	the scheduled MCS from Table 5.1.3.1-1 is smaller than 10, or

-	the scheduled MCS from Table 5.1.3.1-2 is smaller than 5, or

-	the scheduled MCS from Table 5.1.3.1-3 is smaller than 15, or

-	the number of scheduled RBs is smaller than 3, or

-	otherwise, if the RNTI equals RA-RNTI, [MsgB-RNTI], SI-RNTI, or P-RNTI, the UE shall assume PT-RS is not present

Table 5.1.6.3-1: Time density of PT-RS as a function of scheduled MCS

| Scheduled MCS | Time density () |
| --- | --- |
| IMCS < ptrs-MCS1 | PT-RS is not present |
| ptrs-MCS1 IMCS < ptrs-MCS2 | 4 |
| ptrs-MCS2 IMCS < ptrs-MCS3 | 2 |
| ptrs-MCS3 IMCS < ptrs-MCS4 | 1 |

Table 5.1.6.3-2: Frequency density of PT-RS as a function of scheduled bandwidth

| Scheduled bandwidth | Frequency density () |
| --- | --- |
| NRB < NRB0 | PT-RS is not present |
| NRB0 NRB < NRB1 | 2 |
| NRB1 NRB | 4 |

If a UE is not configured with the higher layer parameter phaseTrackingRS in DMRS-DownlinkConfig, the UE assumes PT-RS is not present.

The higher layer parameter PTRS-DownlinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in range 0-29 when MCS Table 5.1.3.1-1 or MCS Table 5.1.3.1-3 is used and 0-28 when MCS Table 5.1.3.1-2 is used, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 or MCS Table 5.1.3.1-3 is used and 28 when MCS Table 5.1.3.1-2 is used, respectively. The higher layer parameter frequencyDensity in PTRS-DownlinkConfig provides the parameters NRBi i=0,1 with values in range 1-276.

If the higher layer parameter PTRS-DownlinkConfig indicates that the time density thresholds ptrs-MCSi = ptrs-MCSi+1, then the time density LPT-RS of the associated row where both these thresholds appear in Table 5.1.6.3-1 is disabled. If the higher layer parameter PTRS-DownlinkConfig indicates that the frequency density thresholds NRBi = NRBi +1, then the frequency density KPTRS of the associated row where both these thresholds appear in Table 5.1.6.3-2 is disabled.

If either or both of the parameters PT-RS time density (LPT-RS) and PT-RS frequency density (KPT-RS), shown in Table 5.1.6.3-1 and Table 5.1.6.3-2, indicates that 'PT-RS not present', the UE shall assume that PT-RS is not present.

When the UE is receiving a PDSCH with allocation duration of 2 symbols as defined in Clause 7.4.1.1.2 of [4, TS 38.211] and if LPT-RS is set to 2 or 4, the UE shall assume PT-RS is not transmitted.

When the UE is receiving a PDSCH with allocation duration of 4 symbols and if LPT-RS is set to 4, the UE shall assume PT-RS is not transmitted.

When a UE is receiving PDSCH for retransmission, if the UE is scheduled with an MCS index greater than V, where V=28 for MCS Table 5.1.3.1-1 and Table 5.1.3.1-3, and V=27 for MCS Table 5.1.3.1-2 respectively, the MCS for the PT-RS time-density determination is obtained from the DCI received for the same transport block in the initial transmission, which is smaller than or equal to V.

The DL DM-RS port(s) associated with a PT-RS port are assumed to be quasi co-located with respect to {'QCL-TypeA' and 'QCL-TypeD'}.If a UE is scheduled with one codeword, the PT-RS antenna port is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH.

If a UE is scheduled with two codewords, the PT-RS antenna port is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the codeword with the higher MCS. If the MCS indices of the two codewords are the same, the PT-RS antenna port is associated with the lowest indexed DM-RS antenna port assigned for codeword 0.

When a UE is not indicated with a DCI that DCI field "Time domain resource assignment' indicating an entry which contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, and if the UE is configured with the higher layer parameter maxNrofPorts equal to n2,  and if the UE is indicated with two TCI states by the codepoints of the DCI field 'Transmission Configuration Indication' and DM-RS port(s) within two CDM groups in the DCI field "Antenna Port(s)", the UE shall receive two PT-RS ports which are associated to the lowest indexed DM-RS port among the DM-RS ports corresponding to the first/second indicated TCI state, respectively.

When a UE configured by the higher layer parameter RepetitionScheme-r16 set to 'FDMSchemeA' or  'FDMSchemeB', and the UE is indicated with two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)", the UE shall receive a single PT-RS port which is associated with the lowest indexed DM-RS antenna port among the DM-RS antenna ports assigned for the PDSCH, a PT-RS frequency density is determined by the number of PRBs associated to each TCI state, and a PT-RS resource element mapping is associated to the allocated PRBs for each TCI state.

#### 5.1.6.4	SRS reception procedure for CLI

The SRS resources defined in Clause 6.4.1.4 of [4, TS 38.211] may be configured for SRS-RSRP measurement for CLI, as defined in Clause 5.1.19 of [7, TS 38.215]. The UE is not expected to measure SRS-RSRP with a subcarrier spacing other than the one configured for the active BWP confining the SRS resource. The UE is not expected to measure SRS-RSRP using the SRS-RSRP measurement resource which is not fully confined within the DL active BWP. The UE is not expected to measure more than 32 SRS resources, and the UE is not expected to receive more than 8 SRS resources in a slot.

#### 5.1.6.5	PRS reception procedure

The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters nr-DL-PRS-ResourceSet-r16 and nr-DL-PRS-Resource-r16. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer parameter nr-DL-PRS-PositioningFrequencyLayer-r16. A DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by nr-DL-PRS-PositioningFrequencyLayer-r16.

The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters nr-DL-PRS-PositioningFrequencyLayer-r16, nr-DL-PRS-ResourceSet-r16 and nr-DL-PRS-Resource-r16 defined by Clause 6.4.2.1 [17, TS 37.355].

A positioning frequency layer consists of one or more DL PRS resource sets and it is defined by Clause 6.4.2.1 [17, TS 37.355]:

-	dl-PRS-SubcarrierSpacing-r16 defines the subcarrier spacing for the DL PRS resource. All DL PRS resources and DL PRS resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-SubcarrierSpacing-r16. The supported values of dl-PRS-SubcarrierSpacing-r16 are given in Table 4.2-1 of [4, TS38.211].

-	DL-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-CyclicPrefix. The supported values of DL-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211].

-	dl-PRS-PointA-r16 defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS resource set have common Point A and all DL PRS resources sets belonging to the same DL PRS positioning frequency layer have a common Point A.

The UE expects that it will be configured with dl-PRS-ID-r16 each of which is defined such that it is associated with multiple DL PRS resource sets from the same cell. The UE expects that one of these dl-PRS-ID-r16 along with a nr-DL-PRS-ResourceSetId-r16 and a nr-DL-PRS-ResourceId-r16 can be used to uniquely identify a DL PRS resource.

A DL PRS resource set consists of one or more DL PRS resources and it is defined by Clause 6.4.2.1 [TS 37.355]:

-	nr-DL-PRS-ResourceSetId-r16 defines the identity of the DL PRS resource set configuration.

-	dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing-r16=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity.

-	dl-PRS-ResourceRepetitionFactor-r16 defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32,. All the DL PRS resources within one resource set have the same resource repetition factor.

-	dl-PRS-ResourceTimeGap-r16 defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceSetId-r16 within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap-r16 if dl-PRS-ResourceRepetitionFactor-r16 is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet-r16 is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap-r16.

-	dl-PRS-MutingOption1-r16 and dl-PRS-MutingOption2-r16 define the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. If mutingOption1 is configured, each bit in the bitmap of mutingOption1 corresponds to a configurable number provided by higher layer parameter dl-PRS-MutingBitRepetitionFactor-r16 of consecutive instances of a DL PRS resource set where all the DL PRS resources within the set are muted for the instance that is indicated to be muted. The length of the bitmap can be {2, 4, 6, 8, 16, 32} bits. If mutingOption2 is configured each bit in the bitmap of mutingOption2 corresponds to a single repetition index for each of the DL PRS resources within each instance of a nr-DL-PRS-ResourceSet-r16 and the length of the bitmap is equal to the values of dl-PRS-ResourceRepetitionFactor-r16. Both mutingOption1 and mutingOption2 may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in Clause 7.4.1.7.4 of [4, TS 38.211].

-	nr-DL-PRS-SFN0-Offset-r16 defines the time offset of the SFN0 slot 0 for the transmitting cell with respect to SFN0 slot 0 of reference cell.

-	dl-PRS-CombSizeN-r16 defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.1 of [TS38.211]. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-CombSizeN-r16.

-	dl-PRS-ResourceBandwidth-r16 defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a positioning frequency layer have the same value of dl-PRS-ResourceBandwidth-r16.

-	dl-PRS-StartPRB-r16 defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter dl-PRS-PointA-r16. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of dl-PRS-StartPRB-r16.

A DL PRS resource is defined by:

-	dl-PRS-ResourceList-r16 determines the DL PRS resources that are contained within one DL PRS resource set.

-	nr-DL-PRS-ResourceId-r16 determines the DL PRS resource configuration identity. All DL PRS resource IDs are locally defined within a DL PRS resource set.

-	dl-PRS-SequenceId-r16 is used to initialize cinit value used in pseudo random generator [4, TS38.211, 7.4.1.7.2] for generation of DL PRS sequence for a given DL PRS resource.

-	dl-PRS-CombSizeN-and-ReOffset-r16 defines the starting RE offset of the first symbol within a DL PRS resource in frequency. The relative RE offsets of the remaining symbols within a DL PRS resource are defined based on the initial offset and the rule described in Clause 7.4.1.7.3 of [4, TS38.211].

-	dl-PRS-ResourceSlotOffset-r16 determines the starting slot of the DL PRS resource with respect to corresponding DL PRS resource set slot offset

-	dl-PRS-ResourceSymbolOffset-r16 determines the starting symbol of a slot configured with the DL PRS resource.

-	dl-PRS-NumSymbols-r16 defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.1 of [4, TS38.211].

-	dl-PRS-QCL-Info-r16 defines any quasi-colocation information of the DL PRS resource with other reference signals. The DL PRS may be configured to be 'QCL-Type-D' with a DL PRS or SS/PBCH Block from a serving cell or a non-serving cell. The DL PRS may be configured to be 'QCL-Type-C' with a SS/PBCH Block from a serving or non-serving cell. If the DL PRS is configured as both 'QCL-Type-C' and 'QCL-Type-D' with a SS/PBCH Block then the SSB index indicated should be the same.

The UE assumes constant EPRE is used for all REs of a given DL PRS resource.

The UE may be indicated by the network that a DL PRS resources can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter nr-DL-PRS-ReferenceInfo-r16. The reference indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters nr-DL-PRS-expectedRSTD-r16 and nr-DL-PRS-expectedRSTD-uncerainty-r16. The UE expects the reference to be indicated whenever it is expected to receive the DL PRS. This reference provided by nr-DL-PRS-ReferenceInfo-r16 may include an dl-PRS-ID-r16, a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of DL PRS resource IDs. The UE may use different DL PRS resources or a different DL PRS resource set to determine the reference for the RSTD measurement as long as the condition that the DL PRS resources used belong to a single DL PRS resource set is met. If the UE chooses to use a different reference than indicated by the network, then it is expected to report the dl-PRS-ID-r16, the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference.

The UE may be configured to report quality metrics corresponding to the DL RSTD and UE Rx-Tx time difference measurements which include the following fields:

-	timingQualityValue-r16 which provides the best estimate of the uncertainty of the measurement

-	timingQualityResolution-r16 which specifies the resolution levels used in the timingQualityValue-r16 field.

The UE expects to be configured with higher layer parameter nr-DL-PRS-expectedRSTD-r16, which defines the time difference with respect to the received DL subframe timing the UE is expected to receive DL PRS, and DL-PRS-expectedRSTD-uncertainty-r16, which defines a search window around the nr-DL-PRS-expectedRSTD-r16.

For DL UE positioning measurement reporting in higher layer parameters NR-DL-TDOA-SignalMeasurementInformation or NR-Multi-RTT-SignalMeasurementInformation the UE can be configured to report the DL PRS resource ID(s) or the DL PRS resource set ID(s) associated with the DL PRS resource(s) or the DL PRS resource set(s) which are used in determining the UE measurements DL RSTD, UE Tx-Rx time difference.

For the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements the UE can report an associated higher layer parameter nr-TimeStamp-r16. The nr-TimeStamp-r16 can include the SFN and the slot number for a subcarrier spacing. These values correspond to the reference which is provided by nr-DL-PRS-ReferenceInfo-r16.

The UE is expected to measure the DL PRS resource outside the active DL BWP or with a numerology different from the numerology of the active DL BWP if the measurement is made during a configured measurement gap. When the UE is expected to measure the DL PRS resource outside the active DL BWP it may request a measurement gap in higher layer parameter measGapConfig.

The UE assumes that the DL PRS from the serving cell is not mapped to any symbol that contains SS/PBCH block from the serving cell. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that the DL PRS from a non-serving cell is not mapped to any symbol that contains the SS/PBCH block of the same non-serving cell.

The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of cells with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those cells. The up to 4 measurements being performed on the same pair of cells and all DL RSTD measurements in the same report use a single reference timing.

The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set..

The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers.

The UE may be configured to measure and report, subject to UE capability, the timing and the quality metrics of up to 2 additional detected paths that are associated with each RSTD or UE Rx – Tx time difference. The timing of each additional path is reported relative to the path timing used for determining nr-RSTD-r16 or nr-UE-RxTxTimeDiff-r16.

If the UE is configured with dl-PRS-QCL-Info-r16 and the QCL relation is between two DL PRS resources, then the UE assumes those DL PRS resources are from the same cell. If dl-PRS-QCL-Info-r16 is configured to the UE with 'QCL-Type-D' with a source DL-PRS-Resource then the nr-DL-PRS-ResourceSetId-r16 and the nr-DL-PRS-ResourceId-r16 of the source DL PRS resource are expected to be indicated to the UE.

UE is not expected to process DL PRS without configuration of measurement gap.

For the case when measurement gap is configured, the UE DL PRS processing capability is defined in [TS 38.306 Clause 4.2.7.2]. For the purpose of DL PRS processing capability, the duration K ms of DL PRS symbols within any P ms window, is calculated by

-	Type 1 duration calculation with UE symbol level buffering capability

K=s∈SKs
Ks=Tsend-Tsstart

-	Type 2 duration calculation with UE slot level buffering capability

K=12μS

-	S is the set of slots based on the numerology of the DL PRS of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets.

-	For Type 1, Tsstart, Tsend is the smallest interval in ms within slot s corresponding to an integer number of OFDM symbols based on the numerology of the DL PRS of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s, where the interval Tsstart, Tsend considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS resource sets (target and reference).

-	For Type 2, μ is the numerology of the DL PRS, and S is the cardinality of the set S.

### 5.1.7	Code block group based PDSCH transmission

#### 5.1.7.1	UE procedure for grouping of code blocks to code block groups

If a UE is configured to receive code block group (CBG) based transmissions by receiving the higher layer parameter codeBlockGroupTransmission for PDSCH, the UE shall determine the number of CBGs for atransport block reception as

,

where N is the maximum number of CBGs per transport block as configured by maxCodeBlockGroupsPerTransportBlock for PDSCH, and C is the number of code blocks in the transport block according to the procedure defined in Clause 7.2.3 of [5, TS 38.212].

Define , , and .

If , CBG m, , consists of code blocks with indices . CBG m, , consists of code blocks with indices .

#### 5.1.7.2	UE procedure for receiving code block group based transmissions

If a UE is configured to receive code block group based transmissions by receiving the higher layer parameter codeBlockGroupTransmission for PDSCH,

-	The CBG transmission information (CBGTI) field of DCI format 1_1 is of length NTB∙N  bits, where NTB is the value of the higher layer parameter maxNrofCodeWordsScheduledByDCI. If NTB=2 the CBGTI field bits are mapped such that the first set of N  bits starting from the MSB corresponds to the first TB while the second set of N  bits corresponds to a second TB, if scheduled. The first M bits of each set of N  bits in the CBGTI field have an in-order one-to-one mapping with the M CBGs of the TB, with the MSB mapped to CBG#0.

-	For initial transmission of a TB as indicated by the New Data Indicator field of the scheduling DCI, the UE may assume that all the code block groups of the TB are present.

-	For a retransmission of a TB as indicated by the New Data Indicator field of the scheduling DCI, the UE may assume that

-	The CBGTI field of the scheduling DCI indicates which CBGs of the TB are present in the transmission. A bit value of 0' in the CBGTI field indicates that the corresponding CBG is not transmitted and 1' indicates that it is transmitted.

-	If the CBG flushing out information (CBGFI) field of the scheduling DCI is present, CBGFI set to 0' indicates that the earlier received instances of the same CBGs being transmitted may be corrupted, and CBGFI set to 1' indicates that the CBGs being retransmitted are combinable with the earlier received instances of the same CBGs.

-	A CBG contains the same CBs as in the initial transmission of the TB.

## 5.2	UE procedure for reporting channel state information (CSI)

### 5.2.1	Channel state information framework

The procedures on aperiodic CSI reporting described in this clause assume that the CSI reporting is triggered by DCI format 0_1, but they equally apply to CSI reporting triggered by DCI format 0_2, by applying the higher layer parameter reportTriggerSizeForDCI-Format0-2-r16 instead of reportTriggerSize.

The time and frequency resources that can be used by the UE to report CSI are controlled by the gNB. CSI may consist of Channel Quality Indicator (CQI), precoding matrix indicator (PMI), CSI-RS resource indicator (CRI), SS/PBCH Block Resource indicator (SSBRI), layer indicator (LI), rank indicator (RI), L1-RSRP or L1-SINR.

For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, L1-SINR a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings, M≥1 CSI-ResourceConfig Resource Settings, and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs indicating the Resource Set IDs for channel and optionally for interference. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig.

#### 5.2.1.1	Reporting settings

Each Reporting Setting CSI-ReportConfig is associated with a single downlink BWP (indicated by higher layer parameter BWP-Id) given in the associated CSI-ResourceConfig for channel measurement and contains the parameter(s) for one CSI reporting band:codebook configuration including codebook subset restriction, time-domain behavior, frequency granularity for CQI and PMI, measurement restriction configurations, and the CSI-related quantities to be reported by the UE such as the layer indicator (LI), L1-RSRP, L1-SINR, CRI, and SSBRI (SSB Resource Indicator).

The time domain behavior of the CSI-ReportConfig is indicated by the higher layer parameter reportConfigType and can be set to 'aperiodic', 'semiPersistentOnPUCCH', 'semiPersistentOnPUSCH', or 'periodic'. For periodic and semiPersistentOnPUCCH/semiPersistentOnPUSCH CSI reporting, the configured periodicity and slot offset applies in the numerology of the UL BWP in which the CSI report is configured to be transmitted on. The higher layer parameter reportQuantity indicates the CSI-related, L1-RSRP-related or L1-SINR-related quantities to report. The reportFreqConfiguration indicates the reporting granularity in the frequency domain, including the CSI reporting band and if PMI/CQI reporting is wideband or sub-band. The timeRestrictionForChannelMeasurements parameter in CSI-ReportConfig can be configured to enable time domain restriction for channel measurements and timeRestrictionForInterferenceMeasurements can be configured to enable time domain restriction for interference measurements. The CSI-ReportConfig can also contain CodebookConfig, which contains configuration parameters for Type-I, Type II or Enhanced Type II CSI including codebook subset restriction, and configurations of group based reporting.

#### 5.2.1.2	Resource settings

Each CSI Resource Setting CSI-ResourceConfig contains a configuration of a list of S≥1 CSI Resource Sets (given by higher layer parameter csi-RS-ResourceSetList), where the list is comprised of references to either or both of NZP CSI-RS resource set(s) and SS/PBCH block set(s) or the list is comprised of references to CSI-IM resource set(s). Each CSI Resource Setting is located in the DL BWP identified by the higher layer parameter BWP-id, and all CSI Resource Settings linked to a CSI Report Setting have the same DL BWP.

The time domain behavior of the CSI-RS resources within a CSI Resource Setting are indicated by the higher layer parameter resourceType and can be set to aperiodic, periodic, or semi-persistent. For periodic and semi-persistent CSI Resource Settings, the number of CSI-RS Resource Sets configured is limited to S=1. For periodic and semi-persistent CSI Resource Settings, the configured periodicity and slot offset is given in the numerology of its associated DL BWP, as given by BWP-id. When a UE is configured with multiple CSI-ResourceConfigs consisting the same NZP CSI-RS resource ID, the same time domain behavior shall be configured for the CSI-ResourceConfigs. When a UE is configured with multiple CSI-ResourceConfigs consisting the same CSI-IM resource ID, the same time-domain behavior shall be configured for the CSI-ResourceConfigs. All CSI Resource Settings linked to a CSI Report Setting shall have the same time domain behavior.

The following are configured via higher layer signaling for one or more CSI Resource Settings for channel and interference measurement:

-	CSI-IM resource for interference measurement as described in Clause 5.2.2.4.

-	NZP CSI-RS resource for interference measurement as described in Clause 5.2.2.3.1.

-	NZP CSI-RS resource for channel measurement as described in Clause 5.2.2.3.1.

The UE may assume that the NZP CSI-RS resource(s) for channel measurement and the CSI-IM resource(s) for interference measurement configured for one CSI reporting are resource-wise QCLed with respect to 'QCL-TypeD'. When NZP CSI-RS resource(s) is used for interference measurement, the UE may assume that the NZP CSI-RS resource for channel measurement and the CSI- IM resource or NZP CSI-RS resource(s) for interference measurement configured for one CSI reporting are QCLed with respect to 'QCL-TypeD'.

For L1-SINR measurement:

-	When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel and interference measurement on NZP CSI-RS for L1-SINR computation. UE may assume that same 1 port NZP CSI-RS resource(s) with density 3 REs/RB is used for both channel and interference measurements.

-	When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement on SSB or NZP CSI-RS and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on 1 port NZP CSI-RS with density 3 REs/RB, where each SSB or NZP CSI-RS resource for channel measurement is associated with one CSI-IM resource or one NZP CSI-RS resource for interference measurement by the ordering of the SSB or NZP CSI-RS resource for channel measurement and CSI-IM resource or NZP CSI-RS resource for interference measurement in the corresponding resource sets. The number of SSB(s) or CSI-RS resources for channel measurement equals to the number of CSI-IM resources or the number of NZP CSI-RS resource for interference measurement.

-	UE may apply the SSB, or 'QCL-TypeD' RS configured to the NZP CSI-RS resource for channel measurement, as the reference RS for determining 'QCL-TypeD' assumption for the corresponding CSI-IM resource or the corresponding NZP CSI-RS resource for interference measurement configured for one CSI reporting.

-	UE may expect that the NZP CSI-RS resource set for channel measurement and the NZP-CSI-RS resource set for interference measurement, if any, are configured with the higher layer parameter repetition.

-	[When three Resource Settings are configured, the first one Resource Setting (given by higher layer parameterresourcesForChannelMeasurement) is for channel measurement on SSB or NZP CSI-RS. The second one (given by either higher layer parameter csi-IM-ResourcesForInterference) is for interference measurement performed on CSI-IM, where each NZP CSI-RS resource set for channel measurement is associated with one CSI-IM resource for interference measurement. The Third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on 1 port NZP CSI-RS with density 3 REs/RB.]

#### 5.2.1.3	(void)

#### 5.2.1.4	Reporting configurations

The UE shall calculate CSI parameters (if reported) assuming the following dependencies between CSI parameters (if reported)

-	LI shall be calculated conditioned on the reported CQI, PMI, RI and CRI

-	CQI shall be calculated conditioned on the reported PMI, RI and CRI

-	PMI shall be calculated conditioned on the reported RI and CRI

-	RI shall be calculated conditioned on the reported CRI.

The Reporting configuration for CSI can be aperiodic (using PUSCH), periodic (using PUCCH) or semi-persistent (using PUCCH, and DCI activated PUSCH). The CSI-RS Resources can be periodic, semi-persistent, or aperiodic. Table 5.2.1.4-1 shows the supported combinations of CSI Reporting configurations and CSI-RS Resource configurations and how the CSI Reporting is triggered for each CSI-RS Resource configuration. Periodic CSI-RS is configured by higher layers. Semi-persistent CSI-RS is activated and deactivated as described in Clause 5.2.1.5.2. Aperiodic CSI-RS is configured and triggered/activated as described in Clause 5.2.1.5.1.

Table 5.2.1.4-1: Triggering/Activation of CSI Reporting for the possible CSI-RS Configurations.

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

When the UE is configured with higher layer parameter NZP-CSI-RS-ResourceSet and when the higher layer parameter repetition is set to 'off', the UE shall determine a CRI from the supported set of CRI values as defined in Clause 6.3.1.1.2 of [5, TS 38.212] and report the number in each CRI report. When the higher layer parameter repetition is set to 'on', CRI is not reported. CRI reporting is not supported when the higher layer parameter codebookType is set to either 'typeII', 'typeII-PortSelection', 'typeII-r16' or to 'typeII-PortSelection-r16'.

For a periodic or semi-persistent CSI report on PUCCH, the periodicity TCSI (measured in slots) and the slot offset Toffset are configured by the higher layer parameter reportSlotConfig. Unless specified otherwise, the UE shall transmit the CSI report in frames with SFN nf and slot number within the frame ns,fμ satisfying

Nslotframe,μnf+ns,fμ-Toffsetmod TCSI=0

where  μ is the SCS configuration of the UL BWP the CSI report is transmitted on.

For a semi-persistent CSI report on PUSCH, the periodicity TCSI (measured in slots) is configured by the higher layer parameter reportSlotConfig. Unless specified otherwise, the UE shall transmit the CSI report in frames with SFN nf and slot number within the frame ns,fμ satisfying

Nslotframe,μ(nf-nfstart) +ns,fμ-ns,fstartmod TCSI=0

where nfstart and ns,fstart are the SFN and slot number within the frame respectively of the initial semi-persistent PUSCH transmission according to the activating DCI.

For a semi-persistent or aperiodic CSI report on PUSCH, the allowed slot offsets are configured by the following higher layer parameters:

-	if triggered/activated by DCI format 0_2 and the higher layer parameter reportSlotOffsetListForDCI-Format0-2-r16 is configured, the allowed slot offsets are configured by reportSlotOffsetListForDCI-Format0-2-r16, and

-	if triggered/activated by DCI format 0_1 and the higher layer parameter reportSlotOffsetListForDCI-Format0-1-r16 is configured, the allowed slot offsets are configured by reportSlotOffsetListForDCI-Format0-1-r16, and

-	otherwise, the allowed slot offsets are configured by the higher layer parameter reportSlotOffsetList.

The offset is selected in the activating/triggering DCI.

For CSI reporting, a UE can be configured via higher layer signaling with one out of two possible subband sizes, where a subband is defined as  contiguous PRBs and depends on the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2.

Table 5.2.1.4-2: Configurable subband sizes

| Bandwidth part (PRBs) | Subband size (PRBs) |
| --- | --- |
| 24 – 72 | 4, 8 |
| 73 – 144 | 8, 16 |
| 145 – 275 | 16, 32 |

The reportFreqConfiguration contained in a CSI-ReportConfig indicates the frequency granularity of the CSI Report. A CSI Reporting Setting configuration defines a CSI reporting band as a subset of subbands of the bandwidth part, where the reportFreqConfiguration indicates:

-	the csi-ReportingBand as a contiguous or non-contiguous subset of subbands in the bandwidth part for which CSI shall be reported.

-	A UE is not expected to be configured with csi-ReportingBand which contains a subband where a CSI-RS resource linked to the CSI Report setting has the frequency density of each CSI-RS port per PRB in the subband less than the configured density of the CSI-RS resource.

-	If a CSI-IM resource is linked to the CSI Report Setting, a UE is not expected to be configured with csi-ReportingBand which contains a subband where not all PRBs in the subband have the CSI-IM REs present.

-	wideband CQI or subband CQI reporting, as configured by the higher layer parameter cqi-FormatIndicator. When wideband CQI reporting is configured, a wideband CQI is reported for each codeword for the entire CSI reporting band. When subband CQI reporting is configured, one CQI for each codeword is reported for each subband in the CSI reporting band.

-	wideband PMI or subband PMI reporting as configured by the higher layer parameter pmi-FormatIndicator. When wideband PMI reporting is configured, a wideband PMI is reported for the entire CSI reporting band. When subband PMI reporting is configured, except with 2 antenna ports, a single wideband indication (i1 in Clause 5.2.2.2) is reported for the entire CSI reporting band and one subband indication (i2 in clause 5.2.2.2) is reported for each subband in the CSI reporting band. When subband PMIs are configured with 2 antenna ports, a PMI is reported for each subband in the CSI reporting band.

-	a UE is not expected to be configured with pmi-FormatIndicator if codebookConfig is set to 'typeII-r16' or 'typeII-PortSelection-r16'.

A CSI Reporting Setting is said to have a wideband frequency-granularity if

-	reportQuantity is set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', cqi-FormatIndicator is set to 'widebandCQI' and pmi-FormatIndicator is set to 'widebandPMI', or

-	reportQuantity is set to 'cri-RI-i1' or

-	reportQuantity is set to 'cri-RI-CQI' or 'cri-RI-i1-CQI' and cqi-FormatIndicator is set to 'widebandCQI', or

-	reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR'

otherwise, the CSI Reporting Setting is said to have a subband frequency-granularity.

If the UE is configured with a CSI Reporting Setting for a bandwidth part with fewer than 24 PRBs, the CSI reporting setting is expected to have a wideband frequency-granularity, and, if applicable, the higher layer parameter codebookType is set to 'typeI-SinglePanel'.

The first subband size is given by  and the last subband size given by  if  and if

If a UE is configured with semi-persistent CSI reporting, the UE shall report CSI when both CSI-IM and NZP CSI-RS resources are configured as periodic or semi-persistent. If a UE is configured with aperiodic CSI reporting, the UE shall report CSI when both CSI-IM and NZP CSI-RS resources are configured as periodic, semi-persistent or aperiodic.

A UE configured with DCI format 0_1 or 0_2 does not expect to be triggered with multiple CSI reports with the same CSI-ReportConfigId.

##### 5.2.1.4.1	Resource Setting configuration

For aperiodic CSI, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where each CSI-ReportConfig is linked to periodic, or semi-persistent, or aperiodic resource setting(s):

-	When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation.

-	When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on NZP CSI-RS.

-	When three Resource Settings are configured, the first Resource Setting (higher layer parameter resourcesForChannelMeasurement) is for channel measurement, the second one (given by higher layer parameter csi-IM-ResourcesForInterference) is for CSI-IM based interference measurement and the third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for NZP CSI-RS based interference measurement.

For semi-persistent or periodic CSI, each CSI-ReportConfig is linked to periodic or semi-persistent Resource Setting(s):

-	When one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is configured, the Resource Setting is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation.

-	When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference) is used for interference measurement performed on CSI-IM. For L1-SINR computation, the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourceForInterference) is used for interference measurement performed on CSI-IM or on NZP CSI-RS.

A UE is not expected to be configured with more than one CSI-RS resource in resource set for channel measurement for a CSI-ReportConfig with the higher layer parameter codebookType set to either 'typeII', 'typeII-PortSelection', 'typeII-r16' or to 'typeII-PortSelection-r16'. A UE is not expected to be configured with more than 64 NZP CSI-RS resources and/or SS/PBCH block resources in resource setting for channel measurement for a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', 'cri-RI-CQI', 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR'. If interference measurement is performed on CSI-IM, each CSI-RS resource for channel measurement is resource-wise associated with a CSI-IM resource by the ordering of the CSI-RS resource and CSI-IM resource in the corresponding resource sets. The number of CSI-RS resources for channel measurement equals to the number of CSI-IM resources.

Except for L1-SINR, if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement. Except for L1-SINR, the UE configured with the higher layer parameter nzp-CSI-RS-ResourcesForInterference may expect no more than 18 NZP CSI-RS ports configured in a NZP CSI-RS resource set.

For CSI measurement(s) other than L1-SINR, a UE assumes:

-	each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer.

-	all interference transmission layers on NZP CSI-RS ports for interference measurement take into account the associated EPRE ratios configured in 5.2.2.3.1;

-	other interference signal on REs of NZP CSI-RS resource for channel measurement, NZP CSI-RS resource for interference measurement, or CSI-IM resource for interference measurement.

For L1-SINR measurement with dedicated interference measurement resources, a UE assumes:

-	the total received power on dedicated NZP CSI-RS resource for interference measurement [and/] or dedicated CSI-IM resource for interference measurement corresponds to interference and noise.

##### 5.2.1.4.2	Report Quantity Configurations

A UE may be configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to either 'none', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RSRP', 'cri-SINR', 'ssb-Index-RSRP', 'ssb-Index-SINR' or 'cri-RI-LI-PMI-CQI'.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none', then the UE shall not report any quantity for the CSI-ReportConfig.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', the UE shall report a preferred precoder matrix for the entire reporting band, or a preferred precoder matrix per subband, according to Clause 5.2.2.2.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-i1',

-	the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel' and pmi-FormatIndicator set to 'widebandPMI' and,

-	the UE shall report a PMI consisting of a single wideband indication ( in Clause 5.2.2.2.1) for the entire CSI reporting band.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-i1-CQI',

-	the UE expects, for that CSI-ReportConfig, to be configured with higher layer parameter codebookType set to 'typeI-SinglePanel' and pmi-FormatIndicator set to 'widebandPMI' and,

-	the UE shall report a PMI consisting of a single wideband indication ( in Clause 5.2.2.2.1) for the entire CSI reporting band. The CQI is calculated conditioned on the reported assuming PDSCH transmission with  precoders (corresponding to the same but different  in Clause 5.2.2.2.1), where the UE assumes that one precoder is randomly selected from the set of  precoders for each PRG on PDSCH, where the PRG size for CQI calculation is configured by the higher layer parameter pdsch-BundleSizeForCSI.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-CQI',

-	if the UE is configured with higher layer parameter non-PMI-PortIndication contained in a CSI-ReportConfig, r ports are indicated in the order of layer ordering for rank r and each CSI-RS resource in the CSI resource setting is linked to the CSI-ReportConfig based on the order of the associated NZP-CSI-RS-ResourceId in the linked CSI resource setting for channel measurement given by higher layer parameter resourcesForChannelMeasurement. The configured higher layer parameter non-PMI-PortIndication contains a sequence  of port indices, where  are the CSI-RS port indices associated with rank ν and  where is the number of ports in the CSI-RS resource. The UE shall only report RI corresponding to the configured fields of PortIndexFor8Ranks.

-	if the UE is not configured with higher layer parameter non-PMI-PortIndication, the UE assumes, for each CSI-RS resource in the CSI resource setting linked to the CSI-ReportConfig, that the CSI-RS port indices  are associated with ranks  where  is the number of ports in the CSI-RS resource.

-	When calculating the CQI for a rank, the UE shall use the ports indicated for that rank for the selected CSI-RS resource. The precoder for the indicated ports shall be assumed to be the identity matrix scaled by .

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single report nrofReportedRS (higher layer configured) different CRI or SSBRI for each report setting.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-SINR' or 'ssb-Index-SINR',

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'disabled', the UE shall report in a single report nrofReportedRSForSINR (higher layer configured) different CRI or SSBRI for each report setting.

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', 'cri-RI-LI-PMI-CQI', or 'cri-SINR',and Ks>1 resources are configured in the corresponding resource set for channel measurement, then the UE shall derive the CSI parameters other than CRI conditioned on the reported CRI, where CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of associated nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated  nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet (if configured for CSI-ReportConfig with reportQuantity set to 'cri-SINR') for interference measurement. If Ks=2 CSI-RS resources are configured, each resource shall contain at most 16 CSI-RS ports. If 2<Ks≤8 CSI-RS resources are configured, each resource shall contain at most 8 CSI-RS ports.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-RSRP', the UE shall report SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-SINR', the UE shall derive L1-SINR conditioned on the reported SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList  in the corresponding CSI-SSB-ResourceSet for channel measurement, and (k+1)-th entry of associated csi-IM-Resource in the corresponding csi-IM-ResourceSet (if configured) or (k+1)-th entry of associated  nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet (if configured) for interference measurement.

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', ' cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', then the UE is not expected to be configured with more than 8 CSI-RS resources in a CSI-RS resource set contained within a resource setting that is linked to the CSI-ReportConfig.

If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR' or 'none' and the CSI-ReportConfig is linked to a resource setting configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within the resource setting.

The LI indicates which column of the precoder matrix of the reported PMI corresponds to the strongest layer of the codeword corresponding to the largest reported wideband CQI. If two wideband CQIs are reported and have equal value, the LI corresponds to strongest layer of the first codeword.

For operation with shared spectrum channel access, if the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', the UE shall derive:

-	the CSI parameters without averaging instances of any nzp-CSI-RSResource in the corresponding nzp-CSI-RS-ResourceSet for channel measurement located in different DL transmission bursts (defined in [16, TS 37.213]).

-	the interference measurements for computing CSI value based on periodic/semi-persistent CSI-IM measured only in OFDM symbol(s) that fulfill the same conditions under which the UE is expected to receive periodic/semi-persistent CSI-RS as described in Clause 11.1 and Clause 11.1.1 of [6, TS 38.213].

##### 5.2.1.4.3	L1-RSRP Reporting

For L1-RSRP computation

-	the UE may be configured with CSI-RS resources, SS/PBCH Block resources or both CSI-RS and SS/PBCH block resources, when resource-wise quasi co-located with 'QCL-Type C' and 'QCL-TypeD' when applicable.

-	the UE may be configured with CSI-RS resource setting up to 16 CSI-RS resource sets having up to 64 resources within each set. The total number of different CSI-RS resources over all resource sets is no more than 128.

For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in [11, TS 38.133].

If a UE is not configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting.

If a UE is configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

##### 5.2.1.4.4	L1-SINR Reporting

For L1-SINR computation, for channel measurement the UE may be configured with NZP CSI-RS resources and/or SS/PBCH Block resources, for interference measurement the UE may be configured with NZP CSI-RS or CSI-IM resources.

-	for channel measurement, the UE may be configured with CSI-RS resource setting with up to 16 resource sets, with a total of up to 64 CSI-RS resources or up to 64 SS/PBCH Block resources.

For L1-SINR reporting, if the higher layer parameter nrofReportedRSForSINR in CSI-ReportConfig is configured to be one, the reported L1-SINR value is defined by a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and if the higher layer parameter nrofReportedRSForSINR is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', the UE shall use differential L1-SINR based reporting, where the largest measured value of L1-SINR is quantized to a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and the differential L1-SINR is quantized to a 4-bit value. The differential L1-SINR is computed with 1 dB step size with a reference to the largest measured L1-SINR value which is part of the same L1-SINR reporting instance. When NZP CSI-RS is configured for channel measurement and/or interference measurement, the reported L1-SINR values should not be compensated by the power offset(s) given by higher layer parameter powerControOffsetSS or powerControlOffset.

#### 5.2.1.5	Triggering/activation of CSI Reports and CSI-RS

##### 5.2.1.5.1	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology

For CSI-RS resource sets associated with Resource Settings configured with the higher layer parameter resourceType set to 'aperiodic', 'periodic', or 'semi-persistent', trigger states for Reporting Setting(s) (configured with the higher layer parameter reportConfigType set to 'aperiodic') and/or Resource Setting for channel and/or interference measurement on one or more component carriers are configured using the higher layer parameter CSI-AperiodicTriggerStateList. For aperiodic CSI report triggering, a single set of CSI triggering states are higher layer configured, wherein the CSI triggering states can be associated with any candidate DL BWP. A UE is not expected to receive more than one DCI with non-zero CSI request per slot. A UE is not expected to be configured with different TCI-StateId's for the same aperiodic CSI-RS resource ID configured in multiple aperiodic CSI-RS resource sets with the same triggering offset in the same aperiodic trigger state. A UE is not expected to receive more than one aperiodic CSI report request for transmission in a given slot. If a UE does not indicate its capability of CSItriggerStateContainingNonactiveBWP the UE is not expected to be triggered with a CSI report for a non-active DL BWP. Otherwise, when a UE is triggered with a CSI report for a DL BWP that is non-active when expecting to receive the most recent occasion, no later than the CSI reference resource, of the associated NZP CSI-RS, the UE is not expected to report the CSI for the non-active DL BWP and the CSI report associated with that BWP is omitted. When a UE is triggered with aperiodic NZP CSI-RS in a DL BWP that is non-active when expecting to receive the NZP CSI-RS, the UE is not expected to measure the aperiodic CSI-RS. In the carrier of the serving cell expecting to receive that associated NZP CSI-RS, if the active DL BWP when receiving the NZP CSI-RS is different from the active DL BWP when receiving the triggering DCI,

-	the last symbol of the PDCCH span of the DCI carrying the BWP switching shall be no later than the last symbol of the PDCCH span of the DCI carrying the CSI trigger, irrespective of whether they are in the same carrier of a serving cell or not and irrespective of whether they are in the same SCS or not;

-	the UE is not expected to have any other BWP switching in that carrier after the last symbol of the PDCCH span covering the DCI carrying the CSI trigger and before the first symbol of the triggered NZP CSI-RS or CSI-IM.

A trigger state is initiated using the CSI request field in DCI.

-	When all the bits of CSI request field in DCI are set to zero, no CSI is requested.

-	When the number of configured CSI triggering states in CSI-AperiodicTriggerStateList is greater than , where  is the number of bits in the DCI CSI request field, the UE receives a subselection indication, as described in clause 6.1.3.13 of [10, TS 38.321], used to map up to  trigger states to the codepoints of the CSI request field in DCI.  is configured by the higher layer parameter reportTriggerSize where . When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the subselection indication, the corresponding action in [10, TS 38.321] and UE assumption on the mapping of the selected CSI trigger state(s) to the codepoint(s) of DCI CSI request field shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	When the number of CSI triggering states in CSI-AperiodicTriggerStateList is less than or equal to , the CSI request field in DCI directly indicates the triggering state.

-	For each aperiodic CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated the quasi co-location configuration of quasi co-location RS source(s) and quasi co-location type(s), as described in Clause 5.1.5, through higher layer signaling of qcl-info which contains a list of references to TCI-State's for the aperiodic CSI-RS resources associated with the CSI triggering state. If a State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic or semi-persistent located in the same or different CC/DL BWP.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided.

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS;

-	else if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored;

-	else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or is equal to or greater than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

-	A non-zero codepoint of the CSI request field in the DCI is mapped to a CSI triggering state according to the order of the associated positions of the up to 2NTS-1 trigger states in CSI-AperiodicTriggerStateList with codepoint '1' mapped to the triggering state in the first position.

For a UE configured with the higher layer parameter CSI-AperiodicTriggerStateList, if a Resource Setting linked to a CSI-ReportConfig has multiple aperiodic resource sets, only one of the aperiodic CSI-RS resource sets from the Resource Setting is associated with the trigger state, and the UE is higher layer configured per trigger state per Resource Setting to select the one CSI-IM/NZP CSI-RS resource set from the Resource Setting.

When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states , the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement.

The UE does not expect that aperiodic CSI-RS is transmitted before the OFDM symbol(s) carrying its triggering DCI. When the minimum scheduling offset restriction is applied, UE is not expected to be triggered by CSI triggering state indicated by the CSI request field in DCI in which CSI-RS triggering offset is smaller than the currently applicable minimum scheduling offset restriction K0min.

If interference measurement is performed on aperiodic NZP CSI-RS, a UE is not expected to be configured with a different aperiodic triggering offset of the NZP CSI-RS for interference measurement from the associated NZP CSI-RS for channel measurement.

If the UE is configured with a single carrier for uplink, the UE is not expected to transmit more than one aperiodic CSI report triggered by different DCIs on overlapping OFDM symbols.

##### 5.2.1.5.1a	Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions:

Beam switch timing:

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48} and enableBeamSwitchTiming-r16 is not provided, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero

-	if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD',

-	if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48} and enableBeamSwitchTiming-r16 is not provided, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming-r16 is one of the values {224, 336} and enableBeamSwitchTiming-r16 is provided, periodic CSI-RS, semi-persistent CSI-RS;

-	else,

-	if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored.

-	else if the UE is configured with [enableDefaultBeamForCCS], when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.

-	If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d ∙2μCSIRS/2μPDCCH in CSI-RS symbols, when the reported value is one of the values of {14,28,48} and enableBeamSwitchTiming-r16 is not provided, or is equal to or greater than 48+d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336} and enableBeamSwitchTiming-r16 is provided, where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

Table 5.2.1.5.1a-1: Additional beam switching timing delay d

| µPDCCH | d [PDCCH symbols] |
| --- | --- |
| 0 | 8 |
| 1 | 8 |
| 2 | 14 |

Aperiodic CSI-RS timing:

-	When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16, including the case that the UE is not configured with minimumSchedulingOffsetK0-r16 for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, …, 31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16,

-	μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and  μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5

-	If the µPDCCH < µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than the first symbol of the CSI-RS carrier's slot that starts at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

-	If the µPDCCH > µCSIRS, the UE is expected to be able to measure the aperiodic CSI RS, if the CSI-RS starts no earlier than at least Ncsirs PDCCH symbols after the end of the PDCCH triggering the aperiodic CSI-RS.

Table 5.2.1.5.1a: Ncsirs as a function of the subcarrier spacing of the triggering PDCCH

| µPDCCH | Ncsirs [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | [14] |

##### 5.2.1.5.2	Semi-persistent CSI/Semi-persistent CSI-RS

For semi-persistent reporting on PUSCH, a set of trigger states are higher layer configured by CSI-SemiPersistentOnPUSCH-TriggerStateList, where the CSI request field in DCI scrambled with SP-CSI-RNTI activates one of the trigger states. A UE is not expected to receive a DCI scrambled with SP-CSI-RNTI activating one semi-persistent CSI report with the same CSI-ReportConfigId as in a semi-persistent CSI report which is activated by a previously received DCI scrambled with SP-CSI-RNTI.

For semi-persistent reporting on PUCCH, the PUCCH resource used for transmitting the CSI report are configured by reportConfigType. Semi-persistent reporting on PUCCH is activated by an activation command as described in clause 6.1.3.16 of [10, TS 38.321], which selects one of the semi-persistent Reporting Settings for use by the UE on the PUCCH. When the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command, the indicated semi-persistent Reporting Setting should be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

For a UE configured with CSI resource setting(s) where the higher layer parameter resourceType set to 'semiPersistent'.

-	when a UE receives an activation command, as described in clause 6.1.3.12 of [10, TS 38.321], for CSI-RS resource set(s) for channel measurement and CSI-IM/NZP CSI-RS resource set(s) for interference measurement associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the selection command, the corresponding actions in [10, TS 38.321] and the UE assumptions (including QCL assumptions provided by a list of reference to TCI-State's, one per activated resource) on CSI-RS/CSI-IM transmission corresponding to the configured CSI-RS/CSI-IM resource configuration(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. If a TCI-State referred to in the list is configured with a reference to an RS associated with 'QCL-TypeD', that RS can be an SS/PBCH block, periodic or semi-persistent CSI-RS located in same or different CC/DL BWP.

-	when a UE receives a deactivation command, as described in clause 6.1.3.12 of [10, TS 38.321], for activated CSI-RS/CSI-IM resource set(s) associated with configured CSI resource setting(s), and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of CSI-RS/CSI-IM transmission corresponding to the deactivated CSI-RS/CSI-IM resource set(s) shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

A codepoint of the CSI request field in the DCI is mapped to a SP-CSI triggering state according to the order of the positions of the configured trigger states in CSI-SemiPersistentOnPUSCH-TriggerStateList, with codepoint '0' mapped to the triggering state in the first position. A UE validates, for semi-persistent CSI activation or release, a DL semi-persistent assignment PDCCH on a DCI only if the following conditions are met:

-	the CRC parity bits of the DCI format are scrambled with a SP-CSI-RNTI provided by higher layer parameter sp-CSI-RNTI

-	Special fields for the DCI format are set according to Table 5.2.1.5.2-1 or Table 5.2.1.5.2-2.

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of semi-persistent CSI transmission on PUSCH, and the UE activates or deactivates a CSI Reporting Setting indicated by CSI request field in the DCI. If validation is not achieved, the UE considers the DCI format as having been detected with a non-matching CRC.

Table 5.2.1.5.2-1: Special fields for semi-persistent CSI activation PDCCH validation

|  | DCI format 0_1/0_2 |
| --- | --- |
| HARQ process number | set to all '0's |
| Redundancy version | set to all '0's |

Table 5.2.1.5.2-2: Special fields for semi-persistent CSI deactivation PDCCH validation

|  | DCI format 0_1/0_2 |
| --- | --- |
| HARQ process number | set to all '0's |
| Modulation and coding scheme | set to all '1's |
| Resource block assignment | If higher layer configures RA type 0 only, set to all '0's; If higher layer configures RA type 1 only, set to all '1's; If higher layer configures dynamic switch between RA type 0 and 1, then if MSB is'0', set to all '0's; else, set to all '1's For DCI 0_1, if higher layer configures RA type 2, set to all '1's if µ = 0; set to all '0's if µ = 1 |
| Redundancy version | set to all '0's |

If the UE has an active semi-persistent CSI-RS/CSI-IM resource configuration, or an active semi-persistent ZP CSI-RS resource set configuration, and has not received a deactivation command, the activated semi-persistent CSI-RS/CSI-IM resource set or the activated semi-persistent ZP CSI-RS resource set configurations are considered to be active when the corresponding DL BWP is active, otherwise they are considered suspended.

If the UE is configured with carrier deactivation, the following configurations in the carrier in activated state would also be deactivated and need re-activation configuration(s): semi-persistent CSI-RS/CSI- IM resource, semi-persistent CSI reporting on PUCCH, semi-persistent SRS, semi-persistent ZP CSI-RS resource set.

#### 5.2.1.6	CSI processing criteria

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L  holds.

A UE is not expected to be configured with an aperiodic CSI trigger state containing more than NCPU Reporting Settings.Processing of a CSI report occupies a number of CPUs for a number of symbols as follows:

-	OCPU=0  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info configured

-	OCPU=1  for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR', 'ssb-Index-SINR' or 'none' (and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured)

-	for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-LI-PMI-CQI',

-	if a CSI report is aperiodically triggered without transmitting a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied, where the CSI corresponds to a single CSI with wideband frequency-granularity and to at most 4 CSI-RS ports in a single resource without CRI report and where codebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', OCPU=NCPUOCPU=NCPU,

-	otherwise, OCPU=KsOCPU=KS, where Ks Ks is the number of CSI-RS resources in the CSI-RS resource set for channel measurement.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

-	An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info is not configured, the CPU(s) are occupied for a number of OFDM symbols as follows:

-	A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion.

-	An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation.

where (Z3,Z3') are defined in the table 5.4-2.

In any slot, the UE is not expected to have more active CSI-RS ports or active CSI-RS resources in active BWPs than reported as capability. NZP CSI-RS resource is active in a duration of time defined as follows. For aperiodic CSI-RS, starting from the end of the PDCCH containing the request and ending at the end of the PUSCH containing the report associated with this aperiodic CSI-RS. For semi-persistent CSI-RS, starting from the end of when the activation command is applied, and ending at the end of when the deactivation command is applied. For periodic CSI-RS, starting when the periodic CSI-RS is configured by higher layer signalling, and ending when the periodic CSI-RS configuration is released. If a CSI-RS resource is referred N times by one or more CSI Reporting Settings, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted N times.

### 5.2.2	Channel state information

#### 5.2.2.1	Channel quality indicator (CQI)

The CQI indices and their interpretations are given in Table 5.2.2.1-2 or Table 5.2.2.1-4 for reporting CQI based on QPSK, 16QAM and 64QAM. The CQI indices and their interpretations are given in Table 5.2.2.1-3 for reporting CQI based on QPSK, 16QAM, 64QAM and 256QAM.

Based on an unrestricted observation interval in time unless specified otherwise in this Clause, and an unrestricted observation interval in frequency, the UE shall derive for each CQI value reported in uplink slot n the highest CQI index which satisfies the following condition:

-	A single PDSCH transport block with a combination of modulation scheme, target code rate and transport block size corresponding to the CQI index, and occupying a group of downlink physical resource blocks termed the CSI reference resource, could be received with a transport block error probability not exceeding:

-	0.1, if the higher layer parameter cqi-Table in CSI-ReportConfig configures 'table1' (corresponding to Table 5.2.2.1-2), or 'table2' (corresponding to Table 5.2.2.1-3), or

-	0.00001, if the higher layer parameter cqi-Table in CSI-ReportConfig configures 'table3' (corresponding to Table 5.2.2.1-4).

If a UE is not configured with higher layer parameter timeRestrictionForChannelMeasurements, the UE shall derive the channel measurements for computing CSI value reported in uplink slot n based on only the NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting.

If a UE is configured with higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig, the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

If a UE is not configured with higher layer parameter timeRestrictionForInterferenceMeasurements, the UE shall derive the interference measurements for computing CSI value reported in uplink slot n based on only the CSI-IM and/or NZP CSI-RS for interference measurement no later than the CSI reference resource associated with the CSI resource setting.

If a UE is configured with higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig, the UE shall derive the interference measurements for computing the CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting.

For each sub-band index s, a 2-bit sub-band differential CQI is defined as:

-	Sub-band Offset level (s) = sub-band CQI index (s) - wideband CQI index.

The mapping from the 2-bit sub-band differential CQI values to the offset level is shown in Table 5.2.2.1-1

Table 5.2.2.1-1: Mapping sub-band differential CQI value to offset level

| Sub-band differential CQI value | Offset level |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | ≥ 2 |
| 3 | ≤-1 |

A combination of modulation scheme and transport block size corresponds to a CQI index if:

-	the combination could be signaled for transmission on the PDSCH in the CSI reference resource according to the Transport Block Size determination described in Clause 5.1.3.2, and

-	the modulation scheme is indicated by the CQI index, and

-	the combination of transport block size and modulation scheme when applied to the reference resource results in the effective channel code rate which is the closest possible to the code rate indicated by the CQI index. If more than one combination of transport block size and modulation scheme results in an effective channel code rate equally close to the code rate indicated by the CQI index, only the combination with the smallest of such transport block sizes is relevant.

Table 5.2.2.1-2: 4-bit CQI Table

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 120 | 0.2344 |
| 3 | QPSK | 193 | 0.3770 |
| 4 | QPSK | 308 | 0.6016 |
| 5 | QPSK | 449 | 0.8770 |
| 6 | QPSK | 602 | 1.1758 |
| 7 | 16QAM | 378 | 1.4766 |
| 8 | 16QAM | 490 | 1.9141 |
| 9 | 16QAM | 616 | 2.4063 |
| 10 | 64QAM | 466 | 2.7305 |
| 11 | 64QAM | 567 | 3.3223 |
| 12 | 64QAM | 666 | 3.9023 |
| 13 | 64QAM | 772 | 4.5234 |
| 14 | 64QAM | 873 | 5.1152 |
| 15 | 64QAM | 948 | 5.5547 |

Table 5.2.2.1-3: 4-bit CQI Table 2

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 78 | 0.1523 |
| 2 | QPSK | 193 | 0.3770 |
| 3 | QPSK | 449 | 0.8770 |
| 4 | 16QAM | 378 | 1.4766 |
| 5 | 16QAM | 490 | 1.9141 |
| 6 | 16QAM | 616 | 2.4063 |
| 7 | 64QAM | 466 | 2.7305 |
| 8 | 64QAM | 567 | 3.3223 |
| 9 | 64QAM | 666 | 3.9023 |
| 10 | 64QAM | 772 | 4.5234 |
| 11 | 64QAM | 873 | 5.1152 |
| 12 | 256QAM | 711 | 5.5547 |
| 13 | 256QAM | 797 | 6.2266 |
| 14 | 256QAM | 885 | 6.9141 |
| 15 | 256QAM | 948 | 7.4063 |

Table 5.2.2.1-4: 4-bit CQI Table 3

| CQI index | modulation | code rate x 1024 | efficiency |
| --- | --- | --- | --- |
| 0 | out of range |  |  |
| 1 | QPSK | 30 | 0.0586 |
| 2 | QPSK | 50 | 0.0977 |
| 3 | QPSK | 78 | 0.1523 |
| 4 | QPSK | 120 | 0.2344 |
| 5 | QPSK | 193 | 0.3770 |
| 6 | QPSK | 308 | 0.6016 |
| 7 | QPSK | 449 | 0.8770 |
| 8 | QPSK | 602 | 1.1758 |
| 9 | 16QAM | 378 | 1.4766 |
| 10 | 16QAM | 490 | 1.9141 |
| 11 | 16QAM | 616 | 2.4063 |
| 12 | 64QAM | 466 | 2.7305 |
| 13 | 64QAM | 567 | 3.3223 |
| 14 | 64QAM | 666 | 3.9023 |
| 15 | 64QAM | 772 | 4.5234 |

##### 5.2.2.1.1	(void)

#### 5.2.2.2	Precoding matrix indicator (PMI)

##### 5.2.2.2.1	Type I Single-Panel Codebook

For 2 antenna ports {3000, 3001} and the UE configured with higher layer parameter codebookType set to 'typeI-SinglePanel' each PMI value corresponds to a codebook index given in Table 5.2.2.2.1-1. The UE is configured with the higher layer parameter twoTX-CodebookSubsetRestriction. The bitmap parameter twoTX-CodebookSubsetRestriction forms the bit sequence  where  is the LSB and  is the MSB and where a bit value of zero indicates that PMI reporting is not allowed to correspond to the precoder associated with the bit. Bits 0 to 3 are associated respectively with the codebook indices 0 to 3 for  layer, and bits 4 and 5 are associated respectively with the codebook indices 0 and 1 for  layers.

Table 5.2.2.2.1-1: Codebooks for 1-layer and 2-layer CSI reporting using antenna ports 3000 to 3001

| Codebook index | Number of layers |  |
| --- | --- | --- |
|  | 1 | 2 |
| 0 |  |  |
| 1 |  |  |
| 2 |  | - |
| 3 |  | - |

For 4 antenna ports {3000, 3001, 3002, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and the UE configured with higher layer parameter codebookType set to 'typeI-SinglePanel', except when the number of layers (where  is the associated RI value), each PMI value corresponds to three codebook indices ,,. When the number of layers , each PMI value corresponds to four codebook indices ,,,. The composite codebook index  is defined by

The codebooks for 1-8 layers are given respectively in Tables 5.2.2.2.1-5, 5.2.2.2.1-6, 5.2.2.2.1-7, 5.2.2.2.1-8, 5.2.2.2.1-9, 5.2.2.2.1-10, 5.2.2.2.1-11, and 5.2.2.2.1-12. The mapping from  to  and  for 2-layer reporting is given in Table 5.2.2.2.1-3. The mapping from  to  and  for 3-layer and 4-layer reporting when  is given in Table 5.2.2.2.1-4. The quantities , , , , and  are given by

-	The values of and  are configured with the higher layer parameter n1-n2, respectively. The supported configurations of  for a given number of CSI-RS ports and the corresponding values of  are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, , is.

-	UE shall only use  and shall not report  if the value of N2 is 1.

The bitmap parameter n1-n2 forms the bit sequence  where  is the LSB and  is the MSB and where a bit value of zero indicates that PMI reporting is not allowed to correspond to any precoder associated with the bit. The number of bits is given by . Except when the number of layers and the number of antenna ports is 16, 24, or 32, bit  is associated with all precoders based on the quantity , , . When the number of layers and the number of antenna ports is 16, 24, or 32,

-	bits , , and  are each associated with all precoders based on the quantity , , ;

-	if one or more of the associated bits is zero, then PMI reporting is not allowed to correspond to any precoder based on .

For UE configured with higher layer parameter codebookType set to 'typeI-SinglePanel', the bitmap parameter typeI-SinglePanel-ri‑Restriction forms the bit sequence  where  is the LSB and  is the MSB. When  is zero, , PMI and RI reporting are not allowed to correspond to any precoder associated with  layers.

For UE configured with higher layer parameter reportQuantity set to 'cri-RI-i1-CQI', the bitmap parameter typeI-SinglePanel-codebookSubsetRestriction-i2 forms the bit sequence b15,…,b1, b0 where b0 is the LSB and b15 is the MSB. The bit bi is associated with precoders corresponding to codebook index i2=i. When bi is zero, the randomly selected precoder for CQI calculation is not allowed to correspond to any precoder associated with the bit bi.

Table 5.2.2.2.1-2: Supported configurations of and

| Number of CSI-RS antenna ports, |  |  |
| --- | --- | --- |
| 4 | (2,1) | (4,1) |
| 8 | (2,2) | (4,4) |
|  | (4,1) | (4,1) |
| 12 | (3,2) | (4,4) |
|  | (6,1) | (4,1) |
| 16 | (4,2) | (4,4) |
|  | (8,1) | (4,1) |
| 24 | (4,3) | (4,4) |
|  | (6,2) | (4,4) |
|  | (12,1) | (4,1) |
| 32 | (4,4) | (4,4) |
|  | (8,2) | (4,4) |
|  | (16,1) | (4,1) |

Table 5.2.2.2.1-3: Mapping of  to  and  for 2-layer CSI reporting

| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 0 |  | 0 |  | 0 |  | 0 |
| 2 | 0 |  | 0 |  |  |  |  | 0 |
| 3 |  | 0 |  |  |  |  |  | 0 |

Table 5.2.2.2.1-4: Mapping of  to  and  for 3-layer and 4-layer CSI reporting when

| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | 0 |  | 0 | 0 |  | 0 |  |
| 2 |  |  |  | 0 |  | 0 |  |  |  |  |
| 3 |  |  |  |  |  | 0 |  |  |  | 0 |

Table 5.2.2.2.1-5: Codebook for 1-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1 |  |  |  |
| --- | --- | --- | --- |
| where . |  |  |  |

| codebookMode = 2, |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 |
|  |  | 4 | 5 | 6 | 7 |
|  |  | 8 | 9 | 10 | 11 |
|  |  | 12 | 13 | 14 | 15 |
| where . |  |  |  |  |  |

| codebookMode = 2, |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 |
|  |  | 4 | 5 | 6 | 7 |
|  |  | 8 | 9 | 10 | 11 |
|  |  | 12 | 13 | 14 | 15 |
| where . |  |  |  |  |  |

Table 5.2.2.2.1-6: Codebook for 2-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1 |  |  |  |
| --- | --- | --- | --- |
| where . and the mapping from to and is given in Table 5.2.2.2.1-3. |  |  |  |

| codebookMode = 2, |  |  |  |
| --- | --- | --- | --- |
|  |  | 0 | 1 |
|  |  | 2 | 3 |
|  |  | 4 | 5 |
|  |  | 6 | 7 |
| where . and the mapping from to and is given in Table 5.2.2.2.1-3. |  |  |  |

| codebookMode = 2, |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 |
|  |  | 4 | 5 | 6 | 7 |
| where . and the mapping from to is given in Table 5.2.2.2.1-3. |  |  |  |  |  |

Table 5.2.2.2.1-7: Codebook for 3-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1-2, |  |  |  |
| --- | --- | --- | --- |
| where . and the mapping from to and is given in Table 5.2.2.2.1-4. |  |  |  |

| codebookMode = 1-2, |  |  |  |  |
| --- | --- | --- | --- | --- |
| where . |  |  |  |  |

Table 5.2.2.2.1-8: Codebook for 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1-2, |  |  |  |
| --- | --- | --- | --- |
| where . and the mapping from to and is given in Table 5.2.2.2.1-4. |  |  |  |

| codebookMode = 1-2, |  |  |  |  |
| --- | --- | --- | --- | --- |
| where . |  |  |  |  |

Table 5.2.2.2.1-9: Codebook for 5-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| where . |  |  |  |  |

Table 5.2.2.2.1-10: Codebook for 6-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| where . |  |  |  |  |

Table 5.2.2.2.1-11: Codebook for 7-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| where . |  |  |  |  |

Table 5.2.2.2.1-12: Codebook for 8-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| where . |  |  |  |  |

##### 5.2.2.2.2	Type I Multi-Panel Codebook

For 8 antenna ports {3000, 3001, …, 3007}, 16 antenna ports {3000, 3001, …, 3015}, and 32 antenna ports {3000, 3001, …, 3031}, and the UE configured with higher layer parameter codebookType set to 'typeI-MultiPanel',

-	The values of , and  are configured with the higher layer parameters ng-n1-n2. The supported configurations of  for a given number of CSI-RS ports and the corresponding values of  are given in Table 5.2.2.2.2-1. The number of CSI-RS ports, , is .

-	When , codebookMode shall be set to either '1' or '2'. When , codebookMode shall be set to '1'.

The bitmap parameter ng-n1-n2 forms the bit sequence  where  is the LSB and  is the MSB and where a bit value of zero indicates that PMI reporting is not allowed to correspond to any precoder associated with the bit. The number of bits is given by . Bit  is associated with all precoders based on the quantity , , , as defined below. The bitmap parameter ri‑Restriction forms the bit sequence  where  is the LSB and  is the MSB. When  is zero, , PMI and RI reporting are not allowed to correspond to any precoder associated with  layers.

Table 5.2.2.2.2-1: Supported configurations of  and

| Number of CSI-RS antenna ports, |  |  |
| --- | --- | --- |
| 8 | (2,2,1) | (4,1) |
| 16 | (2,4,1) | (4,1) |
|  | (4,2,1) | (4,1) |
|  | (2,2,2) | (4,4) |
| 32 | (2,8,1) | (4,1) |
|  | (4,4,1) | (4,1) |
|  | (2,4,2) | (4,4) |
|  | (4,2,2) | (4,4) |

Each PMI value corresponds to the codebook indices  and , where  is the vector

and  is the associated RI value. When codebookMode is set to '1',  is

.

When codebookMode is set to '2',  and  are

.

The mapping from  to  and  for 2-layer reporting is given in Table 5.2.2.2.1-3. The mapping from  to  and  for 3-layer and 4-layer reporting is given in Table 5.2.2.2.2-2.

-	UE shall only use  and shall not report  if the value of N2 is 1.

Table 5.2.2.2.2-2: Mapping of  to  and  for 3-layer and 4-layer CSI reporting

| 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | 0 |  | 0 | 0 |  | 0 |  |
| 2 |  |  |  | 0 |  | 0 |  |  |  |  |
| 3 |  |  |  |  |  | 0 |  |  |  | 0 |

Several quantities are used to define the codebook elements. The quantities , , , , and  are given by

Furthermore, the quantities  and  () are given by

where

,

and the quantities  and  () are given by

where

.

The codebooks for 1-4 layers are given respectively in Tables 5.2.2.2.2-3, 5.2.2.2.2-4, 5.2.2.2.2-5, and 5.2.2.2.2-6.

Table 5.2.2.2.2-3: Codebook for 1-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1, |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | , |  |  |
| where . |  |  |  |  |

| codebookMode = 2, |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| where . |  |  |  |  |  |

Table 5.2.2.2.2-4: Codebook for 2-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1, |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | , |  |  |
| where and the mapping from to and is given in Table 5.2.2.2.1-3. |  |  |  |  |

| codebookMode = 2, |  |  |  |  |
| --- | --- | --- | --- | --- |
| where and the mapping from to and is given in Table 5.2.2.2.1-3. |  |  |  |  |

Table 5.2.2.2.2-5: Codebook for 3-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1, |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | , |  |  |
| where and the mapping from to and is given in Table 5.2.2.2.2-2. |  |  |  |  |

| codebookMode = 2, |  |  |  |  |
| --- | --- | --- | --- | --- |
| where and the mapping from to and is given in Table 5.2.2.2.2-2. |  |  |  |  |

Table 5.2.2.2.2-6: Codebook for 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| codebookMode = 1, |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | , |  |  |
| where and the mapping from to and is given in Table 5.2.2.2.2-2. |  |  |  |  |

| codebookMode = 2, |  |  |  |  |
| --- | --- | --- | --- | --- |
| where and the mapping from to and is given in Table 5.2.2.2.2-2. |  |  |  |  |

##### 5.2.2.2.3	Type II Codebook

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and the UE configured with higher layer parameter codebookType set to 'typeII'

-	The values of and  are configured with the higher layer parameter n1-n2-codebookSubsetRestriction. The supported configurations of  for a given number of CSI-RS ports and the corresponding values of  are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, , is .

-	The value of  is configured with the higher layer parameter numberOfBeams, where  when  and  when .

-	The value of  is configured with the higher layer parameter phaseAlphabetSize, where .

-	The UE is configured with the higher layer parameter subbandAmplitude set to 'true' or 'false'.

-	The UE shall not report RI > 2.

When , where  is the associated RI value, each PMI value corresponds to the codebook indices  and  where

..

The  vectors combined by the codebook are identified by the indices  and , where

.

Let

and

.

where the values of  are given in Table 5.2.2.2.3-1.

Then the elements of  and  are found from using the algorithm:

for

Find the largest  in Table 5.2.2.2.3-1 such that

When  and  are known,  is found using:

where the indices  are assigned such that  increases as  increases

, where  is given in Table 5.2.2.2.3-1.

-	If ,  and  for , and  is not reported.

-	When ,  and , and  is not reported.

-	When  and ,  and , and  is not reported.

-	When  and ,  and, and  is not reported.

Table 5.2.2.2.3-1: Combinatorial coefficients

|  | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 2 | 2 | 1 | 0 | 0 |
| 3 | 3 | 3 | 1 | 0 |
| 4 | 4 | 6 | 4 | 1 |
| 5 | 5 | 10 | 10 | 5 |
| 6 | 6 | 15 | 20 | 15 |
| 7 | 7 | 21 | 35 | 35 |
| 8 | 8 | 28 | 56 | 70 |
| 9 | 9 | 36 | 84 | 126 |
| 10 | 10 | 45 | 120 | 210 |
| 11 | 11 | 55 | 165 | 330 |
| 12 | 12 | 66 | 220 | 495 |
| 13 | 13 | 78 | 286 | 715 |
| 14 | 14 | 91 | 364 | 1001 |
| 15 | 15 | 105 | 455 | 1365 |

The strongest coefficient on layer  is identified by .

The amplitude coefficient indicators  and  are

for . The mapping from  to the amplitude coefficient  is given in Table 5.2.2.2.3-2 and the mapping from  to the amplitude coefficient  is given in Table 5.2.2.2.3-3. The amplitude coefficients are represented by

for .

Table 5.2.2.2.3-2: Mapping of elements of :  to

| 0 |  |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |

Table 5.2.2.2.3-3: Mapping of elements of :  to

| 0 |  |
| --- | --- |
| 1 |  |

The phase coefficient indicators are

for .

The amplitude and phase coefficient indicators are reported as follows:

-	The indicators , , and  (). , , and  are not reported for .

-	The remaining  elements of  () are reported, where . Let  () be the number of elements of  that satisfy .

-	The remaining  elements of  and  () are reported as follows:

-	When subbandAmplitude is set to 'false',

-	 for , and .  is not reported for .

-	For , the elements of  corresponding to the coefficients that satisfy , , as determined by the reported elements of , are reported, where  and the remaining  elements of  are not reported and are set to .

-	When subbandAmplitude is set to 'true',

-	For , the elements of  and  corresponding to the  strongest coefficients (excluding the strongest coefficient indicated by), as determined by the corresponding reported elements of , are reported, where and . The values of  are given in Table 5.2.2.2.3-4. The remaining  elements of  are not reported and are set to . The elements of  corresponding to the  weakest non-zero coefficients are reported, where . The remaining  elements of  are not reported and are set to .

-	When two elements,  and , of the reported elements of  are identical (), then element  is prioritized to be included in the set of the  strongest coefficients for  and  () reporting.

Table 5.2.2.2.3-4: Full resolution subband coefficients when subbandAmplitude is set to 'true'

| 2 | 4 |
| --- | --- |
| 3 | 4 |
| 4 | 6 |

The codebooks for 1-2 layers are given in Table 5.2.2.2.3-5, where the indices  and  are given by

for , and the quantities , , and  are given by

.

Table 5.2.2.2.3-5: Codebook for 1-layer and 2-layer CSI reporting using antenna ports 3000 to 2999+PCSI‑RS

| Layers |  |
| --- | --- |
| where , and the mappings from to , , , , , and , and from to , , and are as described above, including the ranges of the constituent indices of and . |  |

When the UE is configured with higher layer parameter codebookType set to 'typeII', thebitmap parameter typeII-RI‑Restriction forms the bit sequence  where  is the LSB and  is the MSB. When  is zero, , PMI and RI reporting are not allowed to correspond to any precoder associated with  layers. The bitmap parameter n1-n2‑codebookSubsetRestriction forms the bit sequence where bit sequences , and are concatenated to form . To define  and , first define the  vector groups  as

for

.

The UE shall be configured with restrictions for 4 vector groups indicated by  for  and identified by the group indices

for , where the indices are assigned such that  increases as  increases. The remaining vector groups are not restricted.

-	If ,  for , and  is empty.

-	If ,  is the binary representation of the integer  where  is the MSB and  is the LSB. is found using:

,

where  is defined in Table 5.2.2.2.3-1. The group indices  and indicators  for  may be found from  using the algorithm:

for

Find the largest  such that

The bit sequence  is the concatenation of the bit sequences  for , corresponding to the group indices . The bit sequence  is defined as

Bits  indicate the maximum allowed amplitude coefficient  for the vector in group  indexed by , where the maximum amplitude coefficients are given in Table 5.2.2.2.3-6. A UE that does not report parameter amplitudeSubsetRestriction = 'supported' in its capability signaling is not expected to be configured with  = 01 or 10.

Table 5.2.2.2.3-6: Maximum allowed amplitude coefficients for restricted vectors

| Bits | Maximum Amplitude Coefficient |
| --- | --- |
| 00 |  |
| 01 |  |
| 10 |  |
| 11 |  |

##### 5.2.2.2.4	Type II Port Selection Codebook

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and the UE configured with higher layer parameter codebookType set to 'typeII-PortSelection'

-	The number of CSI-RS ports is given by as configured by higher layer parameter nrofPorts.

-	The value of  is configured with the higher layer parameter numberOfBeams , where  when  and  when .

-	The value of  is configured with the higher layer parameter portSelectionSamplingSize, where  and .

-	The value of  is configured with the higher layer parameter phaseAlphabetSize, where .

-	The UE is configured with the higher layer parameter subbandAmplitude set to 'true' or 'false'.

-	The UE shall not report RI > 2.

The UE is also configured with the higher layer parameter typeII-PortSelectionRI‑Restriction. The bitmap parameter typeII-PortSelectionRI‑Restriction forms the bit sequence  where  is the LSB and  is the MSB. When  is zero, , PMI and RI reporting are not allowed to correspond to any precoder associated with  layers.

When , where  is the associated RI value, each PMI value corresponds to the codebook indices  and  where

.

The  antenna ports per polarization are selected by the index , where

.

The strongest coefficient on layer  is identified by .

The amplitude coefficient indicators  and  are

for . The mapping from  to the amplitude coefficient  is given in Table 5.2.2.2.3-2 and the mapping from  to the amplitude coefficient  is given in Table 5.2.2.2.3-3. The amplitude coefficients are represented by

for .

The phase coefficient indicators are

for .

The amplitude and phase coefficient indicators are reported as follows:

-	The indicators , , and  (). , , and  are not reported for .

-	The remaining  elements of  () are reported, where . Let  () be the number of elements of that satisfy .

-	The remaining  elements of  and  () are reported as follows:

-	When subbandAmplitude is set to 'false',

-	 for , and .  is not reported for .

-	For , the  elements of  corresponding to the coefficients that satisfy , , as determined by the reported elements of , are reported, where  and the remaining  elements of  are not reported and are set to .

-	When subbandAmplitude is set to 'true',

-	For , the elements of  and  corresponding to the  strongest coefficients (excluding the strongest coefficient indicated by), as determined by the corresponding reported elements of , are reported, where and . The values of  are given in Table 5.2.2.2.3-4. The remaining  elements of  are not reported and are set to . The elements of  corresponding to the  weakest non-zero coefficients are reported, where . The remaining  elements of  are not reported and are set to .

-	When two elements,  and , of the reported elements of  are identical (), then element  is prioritized to be included in the set of the  strongest coefficients for  and  () reporting.

The codebooks for 1-2 layers are given in Table 5.2.2.2.4-1, where the quantity  is given by

and  is a -element column vector containing a value of 1 in element  and zeros elsewhere (where the first element is element 0).

Table 5.2.2.2.4-1: Codebook for 1-layer and 2-layer CSI reporting using antenna ports 3000 to 2999+PCSI‑RS

| Layers |  |
| --- | --- |
| where , and the mappings from to , , and and from to , , , and are as described above, including the ranges of the constituent indices of and . |  |

##### 5.2.2.2.5	Enhanced Type II Codebook

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and UE configured with higher layer parameter codebookType set to ' typeII-r16'

-	The values of N1 and N2 are configured with the higher layer parameter n1-n2-codebookSubsetRestriction-r16. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2  are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2.

-	The values of L, β and pυ are determined by the higher layer parameter paramCombination-r16, where the mapping is given in Table 5.2.2.2.5-1.

-	The UE is not expected to be configured with paramCombination-r16 equal to

-	3, 4, 5, 6, 7, or 8 when PCSI-RS=4,

-	7 or 8 when PCSI-RS<32

-	7 or 8 when higher layer parameter typeII-RI-Restriction-r16 is configured with ri=1 for any i>1.

-	7 or 8 when R=2.

-	The parameter R is configured with the higher-layer parameter numberOfPMISubbandsPerCQISubband-r16. This parameter controls the total number of precoding matrices N3 indicated by the PMI as a function of the number of configured subbands in csi-ReportingBand, the subband size configured by the higher-level parameter subbandSize and of the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2, as follows:

-	When R=1:

-	One precoding matrix is indicated by the PMI for each subband in csi-ReportingBand.

-	When R=2:

-	For each subband in csi-ReportingBand that is not the first or last subband of a BWP, two precoding matrices are indicated by the PMI: the first precoding matrix corresponds to the first NPRBSB/2 PRBs of the subband and the second precoding matrix corresponds to the last NPRBSB/2 PRBs of the subband.

-	For each subband in csi-ReportingBand that is the first or last subband of a BWP

-	If NBWP,istart mod NPRBSB≥NPRBSB2, one precoding matrix is indicated by the PMI corresponding to the first subband. If NBWP,istart mod NPRBSB<NPRBSB2, two precoding matrices are indicated by the PMI corresponding to the first subband: the first precoding matrix corresponds to the first NPRBSB2-NBWP,istart mod NPRBSB PRBs of the first subband and the second precoding matrix corresponds to the last NPRBSB2 PRBs of the first subband.

-	If 1+NBWP,istart+NBWP,isize-1mod NPRBSB≤NPRBSB2, one precoding matrix is indicated by the PMI corresponding to the last subband. If 1+NBWP,istart+NBWP,isize-1mod NPRBSB>NPRBSB2, two precoding matrices are indicated by the PMI corresponding to the last subband: the first precoding matrix corresponds to the first NPRBSB2 PRBs of the last subband and the second precoding matrix corresponds to the last  1+NBWP,istart+NBWP,isize-1mod NPRBSB-NPRBSB2  PRBs of the last subband.

Table 5.2.2.2.5-1: Codebook parameter configurations for L,  β and pυ

| paramCombination-r16 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | ¼ | 1/8 | ¼ |
| 2 | 2 | ¼ | 1/8 | ½ |
| 3 | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | 1/8 | ½ |
| 5 | 4 | ¼ | ¼ | ¾ |
| 6 | 4 | ½ | ¼ | ½ |
| 7 | 6 | ¼ | - | ½ |
| 8 | 6 | ¼ | - | ¾ |

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-RI-Restriction-r16. The UE shall not report υ>4.

The PMI value corresponds to the codebook indices of  and  where

i1=i1,1i1,2i1,5i1,6,1i1,7,1i1,8,1                                                                                 υ=1i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2     i1,7,2     i1,8,2υ=2i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2     i1,7,2     i1,8,2     i1,6,3     i1,7,3     i1,8,3   υ=3i1,1i1,2     i1,5i1,6,1i1,7,1    i1,8,1i1,6,2i1,7,2     i1,8,2     i1,6,3i1,7,3i1,8,3     i1,6,4i1,7,4     i1,8,4υ=4i2=i2,3,1i2,4,1i2,5,1                                                                                                                 υ=1i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2υ=2i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3                                   υ=3i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3     i2,3,4i2,4,4i2,5,4υ=4

The precoding matrices indicated by the PMI are determined from L+Mυ vectors.

L vectors, vm1i,m2(i),i=0,1,…,L-1, are indentified by the indices q1,q2, n1, n2, indicated by i1,1, i1,2, obtained as in 5.2.2.2.3, where the values of Cx,y are given in Table 5.2.2.2.5-4.

Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, are identified by Minitial (for N3>19) and n3,l l=1,…, υ where

Minitial∈-2Mυ+1,-2Mυ+2,…,0

n3,l=n3,l(0),…,n3,l(Mυ-1)

n3,lf∈0,1,…,N3-1

which are indicated by means of the indices i1,5 (for N3>19) and i1,6,l for Mυ>1 and l=1,…, υ, where

i1,5∈0,1,…,2Mυ-1

i1,6,l∈0,1,…,N3-1Mυ-1-1N3≤190,1,…,2Mυ-1Mυ-1-1N3>19

The amplitude coefficient indicators i2,3,l and i2,4,l are

i2,3,l=kl,01kl,11

i2,4,l=kl,0(2)…kl,Mυ-1(2)

kl,f(2)=kl,0,f(2)…kl,2L-1,f(2)

kl,p(1)∈1,…,15

kl,i,f(2)∈0,…,7

for l=1,…,υ.

The phase coefficient indicator i2,5,l is

i2,5,l=cl,0…cl,Mυ-1

cl,f=cl,0,f…cl,2L-1,f

cl,i,f∈0,…,15

for l=1,…,υ.

Let K0=β2LM1. The bitmap whose nonzero bits identify which coefficients in i2,4,l and i2,5,l are reported, is indicated by i1,7,l

i1,7,l=kl,03…kl,Mυ-13

kl,f3=kl,0,f3…kl,2L-1,f3

kl,i,f(3)∈0,1

for l=1,…,υ, such that KlNZ=i=02L-1f=0Mυ-1kl,i,f(3)≤K0 is the number of nonzero coefficients for layer l=1,…,υ and KNZ=l=1υKlNZ≤2K0 is the total number of nonzero coefficients.

The indices of i2,4,l, i2,5,l and i1,7,l are associated to the Mυ codebook indices in n3,l.

The mapping from kl,p(1) to the amplitude coefficient pl,p(1) is given in Table 5.2.2.2.5-2 and the mapping from kl,i,f(2) to the amplitude coefficient pl,i,f(2) is given in Table 5.2.2.2.5-3. The amplitude coefficients are represented by

pl1=pl,01pl,11

pl(2)=pl,0(2)…pl,Mυ-1(2)

pl,f(2)=pl,0,f(2)…pl,2L-1,f(2)

for l=1,…,υ.

Let fl*∈0,1,…,Mυ-1 be the index of i2,4,l and il*∈0,1,…,2L-1 be the index of kl,fl*(2) which identify the strongest coefficient of layer l, i.e., the element kl,il*,fl*(2) of i2,4,l, for  l=1,…,υ. The codebook indices of n3,lkmi are remapped with respect to n3,l(fl*) as n3,l(f)=n3,l(f)-n3,l(fl*)mod N3kmi*=0, such that n3,l(fl*)=0, after remapping. The index f is remapped with respect to fl* as f=f-fl*mod Mυ, such that the index of the strongest coefficient is fl*=0 (l=1,…,υ), after remapping. The indices of i2,4,l, i2,5,l and i1,7,l indicate amplitude coefficients, phase coefficients and bitmap after remapping.

The strongest coefficient of layer l is identified by i1,8,l∈0,1,…,2L-1, which is obtained as follows

i1,8,l=i=0i1*k1,i,0(3)-1υ=1il*1<υ≤4

for l=1,…, υ.

Table 5.2.2.2.5-2: Mapping of elements of i2,3,l: kl,p(1) to pl,p(1)

| kl,p(1); pl,p(1). 0; Reserved. 1; 1128. 2; 181921/4. 3; 18. | kl,p(1); pl,p(1). 4; 120481/4. 5; 128. 6; 15121/4. 7; 14. | kl,p(1); pl,p(1). 8; 11281/4. 9; 18. 10; 1321/4. 11; 12. | kl,p(1); pl,p(1). 12; 181/4. 13; 12. 14; 121/4. 15; 1. |
| --- | --- | --- | --- |

The amplitude and phase coefficient indicators are reported as follows:

-	kl,il*L(1)=15, kl,il*,0(2)=7, kl,il*,0(3)=1 and cl,il*,0=0  l=1,…,υ. The indicators kl,il*L(1), kl,il*,0(2) and cl,il*,0 are not reported for l=1,…,υ.

-	The indicator kl,il*L+1 mod 2(1) is reported for l=1,…,υ.

-	The KNZ-υ indicators kl,i,f(2) for which kl,i,f(3)=1, i≠il*, f≠0 are reported.

-	The KNZ-υ indicators cl,i,f for which kl,i,f(3)=1, i≠il*, f≠0 are reported.

-	The remaining 2L⋅Mν⋅ν-KNZ indicators kl,i,f(2) are not reported.

-	The remaining 2L⋅Mν⋅ν-KNZ indicators cl,i,f are not reported.

Table 5.2.2.2.5-3: Mapping of elements of i2,4,l: kl,i,f(2) to pl,i,f(2)

| kl,i,f(2) | pl,i,f(2) |
| --- | --- |
| 0 | 182 |
| 1 | 18 |
| 2 | 142 |
| 3 | 14 |
| 4 | 122 |
| 5 | 12 |
| 6 | 12 |
| 7 | 1 |

The elements of n1 and n2 are found from i1,2 using the algorithm described in 5.2.2.2.3, where the values of Cx,y are given in Table 5.2.2.2.5-4.

For N3>19, Minitial is identified by i1,5.

For all values of N3, n3,l(0)=0 for l=1,…,υ. If Mυ>1, the nonzero elements of n3,l, identified by n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using  Cx,y as defined in Table 5.2.2.2.5-4 and the algorithm:

s0=0

for f=1,…,Mυ-1

Find the largest x*∈Mυ-1-f,…,N3-1-f in Table 5.2.2.2.5-4 such that

i1,6,l -sf-1≥Cx*,Mυ-f

ef=Cx*,Mυ-f

sf=sf-1+ef

if N3≤19

n3,l(f)=N3-1-x*

else

nl(f)=2Mυ-1-x*

if nl(f)≤Minitial+2Mυ-1

n3,l(f)=nl(f)

else

n3,l(f)=nl(f)+(N3-2Mυ)

end if

end if

Table 5.2.2.2.5-4: Combinatorial coefficients Cx,y

|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 3 | 3 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 4 | 4 | 6 | 4 | 1 | 0 | 0 | 0 | 0 | 0 |
| 5 | 5 | 10 | 10 | 5 | 1 | 0 | 0 | 0 | 0 |
| 6 | 6 | 15 | 20 | 15 | 6 | 1 | 0 | 0 | 0 |
| 7 | 7 | 21 | 35 | 35 | 21 | 7 | 1 | 0 | 0 |
| 8 | 8 | 28 | 56 | 70 | 56 | 28 | 8 | 1 | 0 |
| 9 | 9 | 36 | 84 | 126 | 126 | 84 | 36 | 9 | 1 |
| 10 | 10 | 45 | 120 | 210 | 252 | 210 | 120 | 45 | 10 |
| 11 | 11 | 55 | 165 | 330 | 462 | 462 | 330 | 165 | 55 |
| 12 | 12 | 66 | 220 | 495 | 792 | 924 | 792 | 495 | 220 |
| 13 | 13 | 78 | 286 | 715 | 1287 | 1716 | 1716 | 1287 | 715 |
| 14 | 14 | 91 | 364 | 1001 | 2002 | 3003 | 3432 | 3003 | 2002 |
| 15 | 15 | 105 | 455 | 1365 | 3003 | 5005 | 6435 | 6435 | 5005 |
| 16 | 16 | 120 | 560 | 1820 | 4368 | 8008 | 11440 | 12870 | 11440 |
| 17 | 17 | 136 | 680 | 2380 | 6188 | 12376 | 19448 | 24310 | 24310 |
| 18 | 18 | 153 | 816 | 3060 | 8568 | 18564 | 31824 | 43758 | 48620 |

When n3,l and Minitial are known, i1,5 and i1,6,l (l=1,…,υ) are found as follows:

-	If  N3≤19, i1,5=0 and is not reported. If Mν=1, i1,6,l=0, for l=1,…,ν, and is not reported. If Mν>1, i1,6,l=f=1Mυ-1CN3-1-n3,lf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4 and where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases.

-	If  N3>19, Minitial is indicated by i1,5, which is reported and given by

i1,5=MinitialMinitial=0 Minitial+2MυMinitial<0

Only the nonzero indices n3,l(f)∈IntS, where IntS={Minitial+i mod N3, i=0,1,…,2Mυ-1}, are reported, where the indices f=1,…,Mυ-1 are assigned such that n3,l(f) increases as f increases. Let

nl(f)=n3,lf n3,lf≤Minitial+2Mυ-1 n3,lf-(N3-2Mυ)n3,lf>Minitial+N3- 1,

then i1,6,l=f=1Mυ-1C2Mυ-1-nlf,Mυ-f, where Cx,y is given in Table 5.2.2.2.5-4.

The codebooks for 1-4 layers are given in Table 5.2.2.2.5-5, where m1(i), m2(i), for i=0,1,…,L-1, vm1i,m2(i)  are obtained as in clause 5.2.2.2.3, and the quantities φl,i,f and yt,l are given by

φl,i,f= ej2πcl,i,f16

yt,l=yt,l(0)    yt,l(1)   …    yt,l(Mυ-1)

where t=0,1,…,N3-1, is the index associated with the precoding matrix, l=1,…, υ, and with

yt,l(f)=ej2πtn3,l(f)N3

for f=0,1,…,Mυ-1.

Table 5.2.2.2.5-5: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| Layers |  |
| --- | --- |
| υ=1 | Wq1,q2, n1, n2,n3,1,p1(1),p12,i2,5,1,t(1)=Wq1,q2, n1, n2,n3,1,p1(1),p12,i2,5,1,t1 |
| υ=2 | Wq1,q2, n1, n2,n3,1,p1(1),p12,i2,5,1, n3,2,p2(1),p22,i2,5,2,t(2)=12Wq1,q2, n1, n2,n3,1,p1(1),p12,i2,5,1,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,i2,5,2,t2 |
| υ=3 | Wq1,q2, n1, n2,n3,1,p1(1),p12,i2,5,1,n3,2,p2(1),p22,i2,5,2,n3,3,p3(1),p32,i2,5,3,t(3)=13Wq1,q2, n1, n2,n3,1,p1(1),p12,i2,5,1,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,i2,5,2,t2 Wq1,q2, n1, n2,n3,3,p3(1),p32,i2,5,3,t3 |
| υ=4 | Wq1,q2, n1, n2,n3,1,p1(1),p12,i2,5,1,n3,2,p2(1),p22,i2,5,2,n3,3,p3(1),p32,i2,5,3,n3,4,p4(1),p42,i2,5,4,t(4)=12Wq1,q2, n1, n2,n3,1,p1(1),p12,i2,5,1,t1 Wq1,q2, n1, n2,n3,2,p2(1),p22,i2,5,2,t2 Wq1,q2, n1, n2,n3,3,p3(1),p32,i2,5,3,t3 Wq1,q2, n1, n2,n3,4,p4(1),p42,i2,5,4,t4 |
| Where Wq1,q2, n1, n2,n3,pl(1),pl2,i2,5,l,tl=1N1N2γt,li=0L-1vm1i,m2(i)pl,0(1)f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,fi=0L-1vm1i,m2(i)pl,1(1)f=0Mυ-1yt,l(f)pl,i+L,f(2)φl,i+L,f, l=1,2,3,4, γt,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,f2 and the mappings from i1 to q1, q2, n1, n2, n3,1, n3,2, n3,3, n3,4, and from i2 to i2,5,1, i2,5,2, i2,5,3, i2,5,4, p1(1), p2(1), p3(1)and p4(1), p1(2), p2(2), p3(2) and p4(2) are as described above, including the ranges of the constituent indices of i1 and i2. |  |

For coefficients with kl,i,f(3)=0, amplitude and phase are set to zero, i.e., pl,i,f(2)=0 and φl,i,f=0.

The bitmap parameter typeII-RI-Restriction-r16 forms the bit sequence r3,r2,r1,r0 where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…,3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

The bitmap parameter n1-n2-codebookSubsetRestriction-r16 forms the bit sequence B=B1B2 and configures the vector group indices g(k) as in clause 5.2.2.2.3. Bits b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) indicate the maximum allowed average amplitude, γi+pL (p=0,1), with i∈{0,1,…,L-1}, of the coefficients associated with the vector in group g(k) indexed by x1,x2, where the maximum amplitudes are given in Table 5.2.2.2.5-6 and the average coefficient amplitude is restricted as follows

1f=0Mυ-1kl,i+pL,f(3)f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL

for l=1,…,υ, and p=0,1. A UE that does not report the parameter amplitudeSubsetRestriction='supported' in its capability signaling is not expected to be configured with b2k,2(N1x2+x1+1)b2k,2(N1x2+x1)=01 or 10.

Table 5.2.2.2.5-6: Maximum allowed average coefficient amplitudes for restricted vectors

| Bit b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) | Maximum Average Coefficient Amplitude γi+pL |
| --- | --- |
| 00 | 0 |
| 01 | 1/4 |
| 10 | 1/2 |
| 11 | 1 |

##### 5.2.2.2.6	Enhanced Type II Port Selection Codebook

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and the UE configured with higher layer parameter codebookType set to ' typeII-PortSelection-r16 '

-	The number of CSI-RS ports is configured as in Clause 5.2.2.2.4

-	The value of d is configured with the higher layer parameter portSelectionSamplingSize-r16, where d∈1,2,3,4 and d≤L.

-	The values L, β and pυ are configured as in Clause 5.2.2.2.5, where the supported configurations are given in Table 5.2.2.2.6-1.

Table 5.2.2.2.6-1: Codebook parameter configurations for L, β andpυ

| paramCombination-r16 | L | pυ |  | β |
| --- | --- | --- | --- | --- |
|  |  | υ ∈1,2 | υ ∈3,4 |  |
| 1 | 2 | ¼ | 1/8 | ¼ |
| 2 | 2 | ¼ | 1/8 | ½ |
| 3 | 4 | ¼ | 1/8 | ¼ |
| 4 | 4 | ¼ | 1/8 | ½ |
| 5 | 4 | ¼ | ¼ | ¾ |
| 6 | 4 | ½ | ¼ | ½ |

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-PortSelectionRI-Restriction-r16. The UE shall not report υ>4.

-	The values of R is configured as in Clause 5.2.2.2.5.

The UE is also configured with the higher layer bitmap parameter typeII-PortSelectionRI-Restriction-r16, which forms the bit sequence r3,r2,r1,r0, where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…, 3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

The PMI value corresponds to the codebook indices i1 and i2 where

i1=i1,1     i1,5i1,6,1i1,7,1i1,8,1                                                                                             υ=1i1,1i1,5     i1,6,1i1,7,1i1,8,1    i1,6,2i1,7,2     i1,8,2 υ=2i1,1i1,5     i1,6,1i1,7,1i1,8,1   i1,6,2i1,7,2     i1,8,2     i1,6,3     i1,7,3     i1,8,3           υ=3i1,1i1,5     i1,6,1i1,7,1i1,8,1   i1,6,2i1,7,2i1,8,2     i1,6,3 i1,7,3 i1,8,3     i1,6,4i1,7,4     i1,8,4υ=4i2=i2,3,1i2,4,1i2,5,1                                                                                                                   υ=1i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2υ=2i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3                                   υ=3i2,3,1i2,4,1i2,5,1    i2,3,2i2,4,2i2,5,2    i2,3,3i2,4,3i2,5,3     i2,3,4i2,4,4i2,5,4υ=4

The 2L antenna ports are selected by the index i1,1 as in clause 5.2.2.2.4.

Parameters N3, Mυ, Minitial (for N3>19) and K0 are defined as in clause 5.2.2.2.5.

For layer l, l=1,…,υ, the strongest coefficient i1,8,l, the amplitude coefficient indicators i2,3,l and i2,4,l, the phase coefficient indicator i2,5,l and the bitmap indicator i1,7,l are defined and indicated as in clause 5.2.2.2.5, where the mapping from kl,p(1) to the amplitude coefficient pl,p(1) is given in Table 5.2.2.2.5-2 and the mapping from kl,i,f(2) to the amplitude coefficient pl,i,f(2) is given inTable 5.2.2.2.5-3.

The number of nonzero coefficients for layer l, KlNZ, and the total number of nonzero coefficients KNZ are defined as in Clause 5.2.2.2.5.

The amplitude coefficients pl(1) and pl(2) l=1,…,υ  are represented as in clause 5.2.2.2.5.

The amplitude and phase coefficient indicators are reported as in clause 5.2.2.2.5.

Codebook indicators i1,5 and i1,6,l (l=1,…,υ) are found as in clause 5.2.2.2.5.

The codebooks for 1-4 layers are given in Table 5.2.2.2.6-2, where vm is a PCSI-RS/2 -element column vector containing a value of 1 in element m mod PCSI-RS/2  and zeros elsewhere (where the first element is element 0), and the quantities φl,i,f and yt,l  are defined as in clause 5.2.2.2.5.

Table 5.2.2.2.6-2: Codebook for 1-layer. 2-layer, 3-layer and 4-layer CSI reporting using antenna ports 3000 to 2999+PCSI-RS

| Layers |  |
| --- | --- |
| υ=1 | Wi1,1,n3,1,p1(1),p12,i2,5,1,t(1)=Wi1,1,n3,1,p1(1),p12,i2,5,1,t1 |
| υ=2 | Wi1,1,n3,1,p1(1),p12,i2,5,1, n3,2,p2(1),p22,i2,5,2,t(2)=12Wi1,1,n3,1,p1(1),p12,i2,5,1,t1 Wi1,1,n3,2,p2(1),p22,i2,5,2,t2 |
| υ=3 | Wi1,1,n3,1,p1(1),p12,i2,5,1,n3,2,p2(1),p22,i2,5,2,n3,3,p3(1),p32,i2,5,3,t(3)=13Wi1,1,n3,1,p1(1),p12,i2,5,1,t1 Wi1,1,n3,2,p2(1),p22,i2,5,2,t2 Wi1,1,n3,3,p3(1),p32,i2,5,3,t3 |
| υ=4 | Wi1,1,n3,1,p1(1),p12,i2,5,1,n3,2,p2(1),p22,i2,5,2,n3,3,p3(1),p32,i2,5,3,n3,4,p4(1),p42,i2,5,4,t(4)=12Wi1,1,n3,1,p1(1),p12,i2,5,1,t1 Wi1,1,n3,2,p2(1),p22,i2,5,2,t2 Wi1,1,n3,3,p3(1),p32,i2,5,3,t3 Wi1,1,n3,4,p4(1),p42,i2,5,4,t4 |
| Where Wi1,1,n3,pl(1),pl2,i2,5,l,tl=1γt,li=0L-1vi1,1d+ipl,0(1)f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,fi=0L-1vi1,1d+ipl,1(1)f=0Mυ-1yt,l(f)pl,i+L,f(2)φl,i+L,f, l=1,2,3,4, γt,l=i=02L-1pl,iL12f=0Mυ-1yt,l(f)pl,i,f(2)φl,i,f2 and the mappings from i1 to i1,1, n3,1, n3,2, n3,3, n3,4, and from i2 to i2,5,1, i2,5,2, i2,5,3, i2,5,4, p1(1), p2(1), p3(1)and p4(1), p1(2), p2(2), p3(2) and p4(2) are as described above, including the ranges of the constituent indices of i1 and i2. |  |

For coefficients with kl,i,f(3)=0, amplitude and phase are set to zero, i.e., pl,i,f(2)=0 and φl,i,f=0.

#### 5.2.2.3	Reference signal (CSI-RS)

##### 5.2.2.3.1	NZP CSI-RS

The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s).

The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource, CSI-ResourceConfig and NZP-CSI-RS-ResourceSet for each CSI-RS resource configuration:

-	nzp-CSI-RS-ResourceId determines CSI-RS resource configuration identity.

-	periodicityAndOffset defines the CSI-RS periodicity and slot offset for periodic/semi-persistent CSI-RS. All the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources.

-	resourceMapping defines the number of ports, CDM-type, and OFDM symbol and subcarrier occupancy of the CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211].

-	nrofPorts in resourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	density in resourceMapping defines CSI-RS frequency density of each CSI-RS port per PRB, and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. For density 1/2, the odd/even PRB allocation indicated in density is with respect to the common resource block grid.

-	cdm-Type in resourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211].

-	powerControlOffset: which is the assumed ratio of PDSCH EPRE to NZP CSI-RS EPRE when UE derives CSI feedback and takes values in the range of [-8, 15] dB with 1 dB step size.

-	powerControlOffsetSS: which is the assumed ratio of NZP CSI-RS EPRE to SS/PBCH block EPRE.

-	scramblingID defines scrambling ID of CSI-RS with length of 10 bits.

-	BWP-Id in CSI-ResourceConfig defines which bandwidth part the configured CSI-RS is located in.

-	repetition in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and defines whether UE can assume the CSI-RS resources within the NZP CSI-RS Resource Set are transmitted with the same downlink spatial domain transmission filter or not as described in Clause 5.1.6.1.2. and can be configured only when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'cri-RSRP', 'cri-SINR' or 'none'.

-	qcl-InfoPeriodicCSI-RS contains a reference to a TCI-State indicating QCL source RS(s) and QCL type(s). If the TCI-State is configured with a reference to an RS with 'QCL-TypeD' association, that RS may be an SS/PBCH block located in the same or different CC/DL BWP or a CSI-RS resource configured as periodic located in the same or different CC/DL BWP.

-	trs-Info in NZP-CSI-RS-ResourceSet is associated with a CSI-RS resource set and for which the UE can assume that the antenna port with the same port index of the configured NZP CSI-RS resources in the NZP-CSI-RS-ResourceSet is the same as described in Clause 5.1.6.1.1 and can be configured when reporting setting is not configured or when the higher layer parameter reportQuantity associated with all the reporting settings linked with the CSI-RS resource set is set to 'none'.

All CSI-RS resources within one set are configured with same density and same nrofPorts, except for the NZP CSI-RS resources used for interference measurement.

The UE expects that all the CSI-RS resources of a resource set are configured with the same starting RB and number of RBs and the same cdm-type.

The bandwidth and initial common resource block (CRB) index of a CSI-RS resource within a BWP, as defined in Clause 7.4.1.5 of [4, TS 38.211], are determined based on the higher layer parameters nrofRBs and startingRB, respectively, within the CSI-FrequencyOccupation IE configured by the higher layer parameter freqBand within the CSI-RS-ResourceMapping IE. Both nrofRBs and startingRB are configured as integer multiples of 4 RBs, and the reference point for startingRB is CRB 0 on the common resource block grid. If startingRB<NBWPstart, the UE shall assume that the initial CRB index of the CSI-RS resource is Ninitial RB=NBWPstart, otherwise Ninitial RB=startingRB. If nrofRBs>NBWPsize+NBWPstart-Ninitial RB, the UE shall assume that the bandwidth of the CSI-RS resource is NCSI-RSBW=NBWPsize+NBWPstart-Ninitial RB, otherwise NCSI-RSBW=nrofRBs. In all cases, the UE shall expect that NCSI-RSBW≥min⁡(24, NBWPsize).

#### 5.2.2.4	Channel State Information – Interference Measurement (CSI-IM)

The UE can be configured with one or more CSI-IM resource set configuration(s) as indicated by the higher layer parameter CSI-IM-ResourceSet. Each CSI-IM resource set consists of K≥1 CSI-IM resource(s).

The following parameters are configured via higher layer parameter CSI-IM-Resource for each CSI-IM resource configuration:

-	csi-IM-ResourceId determines CSI-IM resource configuration identity

-	subcarrierLocation-p0 or subcarrierLocation-p1 defines subcarrier occupancy of the CSI-IM resource within a slot for csi-IM-ResourceElementPattern set to 'pattern0' or 'pattern1', respectively.

-	symbolLocation-p0 or symbolLocation-p1 defines OFDM symbol location of the CSI-IM resource within a slot for csi-IM-ResourceElementPattern set to 'pattern0' or 'pattern1', respectively.

-	periodicityAndOffset defines the CSI-IM periodicity and slot offset for periodic/semi-persistent CSI-IM.

-	freqBand includes parameters to enable configuration of frequency-occupancy of CSI-IM

In each of the PRBs configured by freqBand, the UE shall assume each CSI-IM resource is located in,

-	resource elements , ,  and , if csi-IM-ResourceElementPattern is set to 'pattern0',

-	resource elements , ,  and  if csi-IM-ResourceElementPattern is set to 'pattern1',

where and  are the configured frequency-domain location and time-domain location, respectively, given by the higher layer parameters in the above list.

#### 5.2.2.5	CSI reference resource definition

The CSI reference resource for a serving cell is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of downlink physical resource blocks corresponding to the band to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in uplink slot n' is defined by a single downlink slot n-nCSI_ref,

-	where  and and  are the subcarrier spacing configurations for DL and UL, respectively

-	where for periodic and semi-persistent CSI reporting

-	if a single CSI-RS/SSB resource is configured for channel measurement nCSI_ref is the smallest value greater than or equal to 4⋅2µDL, such that it corresponds to a valid downlink slot, or

-	if multiple CSI-RS/SSB resources are configured for channel measurement nCSI_ref is the smallest value greater than or equal to , such that it corresponds to a valid downlink slot.

-	where for aperiodic CSI reporting, if the UE is indicated by the DCI to report CSI in the same slot as the CSI request, nCSI_ref is such that the reference resource is in the same valid downlink slot as the corresponding CSI request, otherwise nCSI_ref is the smallest value greater than or equal to , such that slot n- nCSI_ref corresponds to a valid downlink slot, where Z' corresponds to the delay requirement as defined in Clause 5.4.

-	when periodic or semi-persistent CSI-RS/CSI-IM or SSB is used for channel/interference measurements, the UE is not expected to measure channel/interference on the CSI-RS/CSI-IM/SSB whose last OFDM symbol is received up to Z' symbols before transmission time of the first OFDM symbol of the aperiodic CSI reporting.

A slot in a serving cell shall be considered to be a valid downlink slot if:

-	it comprises at least one higher layer configured downlink or flexible symbol, and

-	it does not fall within a configured measurement gap for that UE

If there is no valid downlink slot for the CSI reference resource corresponding to a CSI Report Setting in a serving cell, CSI reporting is omitted for the serving cell in uplink slot n'.

After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise.

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

When deriving CSI feedback, the UE is not expected that a NZP CSI -RS resource for channel measurement overlaps with CSI-IM resource for interference measurement or NZP CSI -RS resource for interference measurement.

If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI:

-	The first 2 OFDM symbols are occupied by control signaling.

-	The number of PDSCH and DM-RS symbols is equal to 12.

-	The same bandwidth part subcarrier spacing configured as for the PDSCH reception

-	The bandwidth as configured for the corresponding CQI report.

-	The reference resource uses the CP length and subcarrier spacing configured for PDSCH reception

-	No resource elements used by primary or secondary synchronization signals or PBCH.

-	Redundancy Version 0.

-	The ratio of PDSCH EPRE to CSI-RS EPRE is as given in Clause 5.2.2.3.1.

-	Assume no REs allocated for NZP CSI-RS and ZP CSI-RS.

-	Assume the same number of front loaded DM-RS symbols as the maximum front-loaded symbols configured by the higher layer parameter maxLength in DMRS-DownlinkConfig.

-	Assume the same number of additional DM-RS symbols as the additional symbols configured by the higher layer parameter dmrs-AdditionalPosition.

-	Assume the PDSCH symbols are not containing DM-RS.

-	Assume PRB bundling size of 2 PRBs.

-	The PDSCH transmission scheme where the UE may assume that PDSCH transmission would be performed with up to 8 transmission layers as defined in Clause 7.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PDSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by

y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i)

where  is a vector of PDSCH symbols from the layer mapping defined in Clause 7.3.1.4 of [4, TS 38.211],  is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to either 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI', W(i) is the precoding matrix corresponding to the reported PMI applicable to x(i). If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-CQI', W(i) is the precoding matrix corresponding to the procedure described in Clause 5.2.1.4.2. If the higher layer parameter reportQuantity in CSI-ReportConfig for which the CQI is reported is set to 'cri-RI-i1-CQI', W(i) is the precoding matrix corresponding to the reported i1 according to the procedure described in Clause 5.2.1.4.2.The corresponding PDSCH signals transmitted on antenna ports [3000,…,3000 + P - 1] would have a ratio of EPRE to CSI-RS EPRE equal to the ratio given in Clause 5.2.2.3.1.

### 5.2.3	CSI reporting using PUSCH

A UE shall perform aperiodic CSI reporting using PUSCH on serving cell c upon successful decoding of a DCI format 0_1 or DCI format 0_2 which triggers an aperiodic CSI trigger state.

When a DCI format 0_1 schedules two PUSCH allocations, the aperiodic CSI report is carried on the second scheduled PUSCH. When a DCI format 0_1 schedules more than two PUSCH allocations, the aperiodic CSI report is carried on the penultimate scheduled PUSCH.

An aperiodic CSI report carried on the PUSCH supports wideband, and sub-band frequency granularities. An aperiodic CSI report carried on the PUSCH supports Type I, Type II and Enhanced Type II CSI.

A UE shall perform semi-persistent CSI reporting on the PUSCH upon successful decoding of a DCI format 0_1 or DCI format 0_2 which activates a semi-persistent CSI trigger state. DCI format 0_1 and DCI format 0_2 contains a CSI request field which indicates the semi-persistent CSI trigger state to activate or deactivate. Semi-persistent CSI reporting on the PUSCH supports Type I, Type II with wideband, and sub-band frequency granularities and Enhanced Type II CSI. The PUSCH resources and MCS shall be allocated semi-persistently by an uplink DCI.

CSI reporting on PUSCH can be multiplexed with uplink data on PUSCH except that semi-persistent CSI reporting on PUSCH activated by a DCI format is not expected to be multiplexed with uplink data on the PUSCH. CSI reporting on PUSCH can also be performed without any multiplexing with uplink data from the UE.

Type I CSI feedback is supported for CSI Reporting on PUSCH. Type I wideband and sub-band CSI is supported for CSI Reporting on the PUSCH. Type II CSI is supported for CSI Reporting on the PUSCH.

For Type I, Type II and Enhanced Type II CSI feedback on PUSCH, a CSI report comprises of two parts. Part 1 has a fixed payload size and is used to identify the number of information bits in Part 2. Part 1 shall be transmitted in its entirety before Part 2.

-	For Type I CSI feedback, Part 1 contains RI (if reported), CRI (if reported), CQI for the first codeword (if reported). Part 2 contains PMI (if reported) and contains the CQI for the second codeword (if reported) when RI (if reported) is larger than 4.

-	For Type II CSI feedback, Part 1 contains RI (if reported), CQI, and an indication of the number of non-zero wideband amplitude coefficients per layer for the Type II CSI (see Clause 5.2.2.2.3). The fields of Part 1 – RI (if reported), CQI, and the indication of the number of non-zero wideband amplitude coefficients for each layer – are separately encoded. Part 2 contains the PMI of the Type II CSI. Part 1 and 2 are separately encoded.

-	For Enhanced Type II CSI feedback, Part 1 contains RI, CQI, and an indication of the overall number of non-zero amplitude coefficients across layers for the Enhanced Type II CSI (see Clause 5.2.2.2.5). The fields of Part 1 – RI, CQI, and the indication of the overall number of non-zero amplitude coefficients across layers – are separately encoded. Part 2 contains the PMI of the Enhanced Type II CSI. Part 1 and 2 are separately encoded.

A Type II CSI report that is carried on the PUSCH shall be computed independently from any Type II CSI report that is carried on the PUCCH formats 3 or 4 (see Clause 5.2.4 and 5.2.2).

When the higher layer parameter reportQuantity is configured with one of the values 'cri-RSRP', 'ssb-Index-RSRP', 'cri-SINR' or 'ssb-Index-SINR', the CSI feedback consists of a single part.

For both Type I and Type II reports configured for PUCCH but transmitted on PUSCH, the determination of the payload for CSI part 1 and CSI part 2 follows that of PUCCH as described in Clause 5.2.4.

When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where  is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority  is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the  CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in increasing order with the lowest subband of csi-ReportingBand as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level.

-	For Enhanced Type II reports, for a given CSI report n, each reported element of indices  i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1, i1,2 and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l (if reported), the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the  KNZ/2-υ highest priority elements of i2,4,l and the  KNZ/2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the KNZ/2 lowest priority elements of i2,4,l and the KNZ/2 lowest priority elements of i2,5,l (l=1,…,υ).

Table 5.2.3-1: Priority reporting levels for Part 2 CSI

Priority 0: For CSI reports 1 to NRep, Group 0 CSI for CSI reports configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 wideband CSI for CSI reports configured otherwise

Priority 1: Group 1 CSI for CSI report 1, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of even subbands for CSI report 1, if configured otherwise

Priority 2: Group 2 CSI for CSI report 1, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of odd subbands for CSI report 1, if configured otherwise

Priority 3: Group 1 CSI for CSI report 2, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of even subbands for CSI report 2, if configured otherwise

Priority 4: Group 2 CSI for CSI report 2, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'. Part 2 subband CSI of odd subbands for CSI report 2, if configured otherwise

⁞

Priority 2NRep-1: Group 1 CSI for CSI report NRep, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of even subbands for CSI report NRep, if configured otherwise

Priority 2NRep: Group 2 CSI for CSI report NRep, if configured as 'typeII-r16' or 'typeII-PortSelection-r16'; Part 2 subband CSI of odd subbands for CSI report NRep, if configured otherwise

When the UE is scheduled to transmit a transport block on PUSCH multiplexed with a CSI report(s), Part 2 CSI is omitted only when  is larger than , where parameters , , , , , , , ,  and are defined in Clause 6.3.2.4 of [5, TS 38.212].

Part 2 CSI is omitted level by level, beginning with the lowest priority level until the lowest priority level is reached which causes the  to be less than or equal to .

When part 2 CSI is transmitted on PUSCH with no transport block, lower priority bits are omitted until Part 2 CSI code rate, which is given by OCSI-2+LCSI-2/(NL⋅Q'CSI,2⋅Qm) where OCSI-2, LCSI-2, NL,Q'CSI,2,Qm are given in clause 6.3.2.4 of [5, 38.212] before HARQ-ACK puncturing part 2 CSI if any, is below a threshold code rate lower than one, where

-	is the CSI offset value from Table 9.3-2 of [6, TS 38.213]

-	R is signaled code rate in DCI

If the UE is in an active semi-persistent CSI reporting configuration on PUSCH, the CSI reporting is deactivated when either the downlink BWP or the uplink BWP is changed. Another activation command is required to enable the semi-persistent CSI reporting.

### 5.2.4	CSI reporting using PUCCH

A UE is semi-statically configured by higher layers to perform periodic CSI Reporting on the PUCCH. A UE can be configured by higher layers for multiple periodic CSI Reports corresponding to multiple higher layer configured CSI Reporting Settings, where the associated CSI Resource Settings are higher layer configured. Periodic CSI reporting on PUCCH formats 2, 3, 4 supports Type I CSI with wideband granularity.

A UE shall perform semi-persistent CSI reporting on the PUCCH applied starting from the first slot that is after slot n+3Nslotsubframe,µ when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command described in clause 6.1.3.16 of [10, TS 38.321] where  is the SCS configuration for the PUCCH. The activation command will contain one or more Reporting Settings where the associated CSI Resource Settings are configured. Semi-persistent CSI reporting on the PUCCH supports Type I CSI. Semi-persistent CSI reporting on the PUCCH format 2 supports Type I CSI with wideband frequency granularity. Semi-persistent CSI reporting on PUCCH formats 3 or 4 supports Type I CSI with wideband and sub-band frequency granularities and Type II CSI Part 1.

When the PUCCH carry Type I CSI with wideband frequency granularity, the CSI payload carried by the PUCCH format 2 and PUCCH formats 3, or 4 are identical and the same irrespective of RI (if reported), CRI (if reported). For type I CSI sub-band reporting on PUCCH formats 3, or 4, the payload is split into two parts. The first part contains RI (if reported), CRI (if reported), CQI for the first codeword. The second part contains PMI and contains the CQI for the second codeword when RI > 4.

A semi-persistent report carried on the PUCCH formats 3 or 4 supports Type II CSI feedback, but only Part 1 of Type II CSI feedback (See Clause 5.2.2 and 5.2.3). Supporting Type II CSI reporting on the PUCCH formats 3 or 4 is a UE capability type2-SP-CSI-Feedback-LongPUCCH. A Type II CSI report (Part 1 only) carried on PUCCH formats 3 or 4 shall be calculated independently of any Type II CSI reports carried on the PUSCH (see Clause 5.2.3).

When the UE is configured with CSI Reporting on PUCCH formats 2, 3 or 4, each PUCCH resource is configured for each candidate UL BWP.

If the UE is in an active semi-persistent CSI reporting configuration on PUCCH and has not received a deactivation command, the CSI reporting takes place when the BWP in which the reporting is configured to take place is the active BWP, otherwise the CSI reporting is suspended.

A UE is not expected to report CSI with a total number of UCI bits and CRC bits larger than 115 bits when configured with PUCCH format 4. For CSI reports transmitted on a PUCCH, if all CSI reports consist of one part, the UE may omit a portion of CSI reports. Omission of CSI is according to the priority order determined from the Prii,CSI(y,k,c,s) value as defined in Clause 5.2.5. CSI report is omitted beginning with the lowest priority level until the CSI report code rate is less or equal to the one configured by the higher layer parameter maxCodeRate.

If any of the CSI reports consist of two parts, the UE may omit a portion of Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1. Part 2 CSI is omitted beginning with the lowest priority level until the Part 2 CSI code rate is less or equal to the one configured by higher layer parameter maxCodeRate.

### 5.2.5	Priority rules for CSI reports

CSI reports are associated with a priority value PriiCSIy,k,c,s=2∙Ncells∙Ms∙y+Ncells∙Ms∙k+Ms∙c+s where

-	 for aperiodic CSI reports to be carried on PUSCH  for semi-persistent CSI reports to be carried on PUSCH,  for semi-persistent CSI reports to be carried on PUCCH and  for periodic CSI reports to be carried on PUCCH;

-	 for CSI reports carrying L1-RSRP or L1-SINR and  for CSI reports not carrying L1-RSRP or L1-SINR;

-	c is the serving cell index and Ncells is the value of the higher layer parameter maxNrofServingCells;

-	s is the reportConfigID and is the value of the higher layer parameter maxNrofCSI-ReportConfigurations.

A first CSI report is said to have priority over second CSI report if the associated  value is lower for the first report than for the second report.

Two CSI reports are said to collide if the time occupancy of the physical channels scheduled to carry the CSI reports overlap in at least one OFDM symbol and are transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports,

-	if y values are different between the two CSI reports, the following rules apply except for the case when one of the y value is 2 and the other y value is 3 (for CSI reports transmitted on PUSCH, as described in Clause 5.2.3; for CSI reports transmitted on PUCCH, as described in Clause 5.2.4):

-	The CSI report with higher  value shall not be sent by the UE.

-	otherwise, the two CSI reports are multiplexed or either is dropped based on the priority values, as described in Clause 9.2.5.2 in [6, TS 38.213].

If a semi-persistent CSI report to be carried on PUSCH overlaps in time with PUSCH data transmission in one or more symbols on the same carrier, and if the earliest symbol of these PUSCH channels starts no earlier than N2+d2,1 symbols after the last symbol of the DCI scheduling the PUSCH where d2,1 is the maximum of the d2,1 associated with the PUSCH carrying semi-persistent CSI report and the PUSCH with data transmission, the CSI report shall not be transmitted by the UE. Otherwise, if the timeline requirement is not satisfied this is an error case.

If a UE would transmit a first PUSCH that includes semi-persistent CSI reports and a second PUSCH that includes an UL-SCH and the first PUSCH transmission would overlap in time with the second PUSCH transmission, the UE does not transmit the first PUSCH and transmits the second PUSCH. The UE expects that the first and second PUSCH transmissions satisfy the above timing conditions for PUSCH transmissions that overlap in time when at least one of the first or second PUSCH transmissions is in response to a DCI format detection by the UE.

## 5.3	UE PDSCH processing procedure time

If the first uplink symbol of the PUCCH which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1, where L1 is defined as the next uplink symbol with its CP starting after  after the end of the last symbol of the PDSCH carrying the TB being acknowledged, then the UE shall provide a valid HARQ-ACK message.

-	N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	For operation with shared spectrum channel access, is calculated according to [4, TS 38.211], otherwise =0.

-	If the PDSCH DM-RS position l1 for the additional DM-RS in Table 7.4.1.1.2-3 in clause 7.4.1.1.2 of [4, TS 38.211] is l1=12 then N1,0=14 in Table 5.3-1, otherwise N1,0=13.

-	If the UE is configured with multiple active component carriers, the first uplink symbol which carries the HARQ-ACK information further includes the effect of timing difference between the component carriers as given in [11, TS 38.133].

-	For the PDSCH mapping type A as given in clause 7.4.1.1 of [4, TS 38.211]: if the last symbol of PDSCH is on the i-th symbol of the slot where i < 7, then d1,1 = 7 - i, otherwise d1,1 = 0

-	If a PUCCH of a larger priority index would overlap with PUCCH/PUSCH of a smaller priority index, d2 for the PUCCH of a larger priority is set as reported by the UE; otherwise d2 = 0.

-	For UE processing capability 1: If the PDSCH is mapping type B as given in clause 7.4.1.1 of [4, TS 38.211], and

-	if the number of PDSCH symbols allocated is L ≥ 7, then d1,1 = 0,

-	if the number of PDSCH symbols allocated is L ≥ 4 and L ≤ 6, then d1,1 = 7- L.

-	if the number of PDSCH symbols allocated is L = 3 then d1,1 = 3 + min (d,1), where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

-	if the number of PDSCH symbols allocated is 2, then d1,1 = 3+d, where d is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

-	For UE processing capability 2: If the PDSCH is mapping type B as given in clause 7.4.1.1 of [4, TS 38.211],

-	if the number of PDSCH symbols allocated is L ≥ 7, then d1,1 = 0,

-	if the number of PDSCH symbols allocated is L ≥ 3 and L ≤ 6, then d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH,

-	if the number of PDSCH symbols allocated is 2,

-	if the scheduling PDCCH was in a 3-symbol CORESET and the CORESET and the PDSCH had the same starting symbol, then d1,1 = 3,

-	otherwise d1,1 is the number of overlapping symbols of the scheduling PDCCH and the scheduled PDSCH.

-	For UE processing capability 2 with scheduling limitation when µPDSCH = 1, if the scheduled RB allocation exceeds 136 RBs, the UE defaults to capability 1 processing time. The UE may skip decoding a number of PDSCHs with last symbol within 10 symbols before the start of a PDSCH that is scheduled to follow Capability 2, if any of those PDSCHs are scheduled with more than 136 RBs with 30kHz SCS and following Capability 1 processing time.

-	For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PDSCH-ServingCellConfig is configured for the cell and set to enable.

-	If this PUCCH resource is overlapping with another PUCCH or PUSCH resource, then HARQ-ACK is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the HARQ-ACK message is transmitted on PUCCH.

Otherwise the UE may not provide a valid HARQ-ACK corresponding to the scheduled PDSCH. The value of Tproc,1 is used both in the case of normal and extended cyclic prefix.

For a PDSCH that consists of two PDSCH transmission occasions in time domain in one slot, d1,1 is calculated based on the first PDSCH transmission occasion in the slot, and as described above.

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ pos0 in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |

### 5.3.1	Application delay of the minimum scheduling offset restriction

When the UE is scheduled with DCI format 0_1 or 1_1 with a 'Minimum applicable scheduling offset indicator' field in slot n, it shall determine the K0min and K2min values, if configured respectively, to be applied, while the previously applied K0min and/or K2min values are applied until the new values take effect. If the DCI in slot n also indicates an active DL (UL) BWP change for a serving cell, the indicated K0min (K2min) value in the new active DL (UL) BWP, if configured, is applied from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI. Otherwise, change of applied minimum scheduling offset restriction indication carried by DCI in slot n, shall be applied in slot n+X of the scheduling cell. The UE does not expect to be scheduled with DCI format 0_1 or 1_1 with 'Minimum applicable scheduling offset indicator' field indicating another change to K0min or K2min for the same active BWP of the scheduled cell before slot n+X of the scheduling cell.

When the DCI format 0_1 or 1_1 with 'Minimum applicable scheduling offset indicator' field indicating a change to the applied K0min or K2min is contained within the first three symbols of slot n, the value of application delay X is determined by, X=maxK0minOld∙2μPDCCH2μPDSCH,Zμ where  K0minOld is the currently applied K0min value of the active DL BWP in the scheduled cell and is zero, if minimumSchedulingOffsetK0 is not configured for the active DL BWP in the scheduled cell, Zµ is determined by the subcarrier spacing of the active DL BWP in the scheduling cell in slot n, and given in Table 5.3.1-1, and µPDCCH and µPDSCH are the sub-carrier spacing configurations for PDCCH of the active DL BWP in the scheduling cell and PDSCH of the active DL BWP in the scheduled cell, respectively, in slot n. After indication of a change to the applied K0min or K2min of the scheduled cell in slot n of the scheduling cell, if there is an active DL BWP change in the scheduling cell before slot n+X, the new K0min and/or K2min values are applied from the first slot no earlier than the start of slot n+X based on the sub-carrier spacing configuration of the active DL BWP in the scheduling cell in slot n.

When the DCI format 0_1 or 1_1 with 'Minimum applicable scheduling offset indicator' field is received outside the first three symbols of the slot, value of Zµ from Table 5.3.1-1 is incremented by one before determining the application delay X.

Table 5.3.1-1: Definition of Zµ

| µ | Zµ |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 2 |

## 5.4	UE CSI computation time

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide a valid CSI report for the n-th triggered report,

-	if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref, and

-	if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref(n),

where Zref is defined as the next uplink symbol with its CP starting  after the end of the last symbol of the PDCCH triggering the CSI report(s), and where Z'ref(n), is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of: aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement, when aperiodic CSI-RS is used for channel measurement for the n-th triggered CSI report, and where Tswitch is defined in clause 6.4 and is applied only if Z1 of table 5.4-1 is applied.

If the PUSCH indicated by the DCI is overlapping with another PUCCH or PUSCH, then the CSI report(s) are multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213] and clause 5.2.5 when applicable, otherwise the CSI report(s) are transmitted on the PUSCH indicated by the DCI.

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts earlier than at symbol Zref,

-	the UE may ignore the scheduling DCI if no HARQ-ACK or transport block is multiplexed on the PUSCH.

When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts earlier than at symbol Z'ref(n),

-	the UE may ignore the scheduling DCI if the number of triggered reports is one and no HARQ-ACK or transport block is multiplexed on the PUSCH

-	Otherwise, the UE is not required to update the CSI for the n-th triggered CSI report.

Z, Z' and µ are defined as:

Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as

-	(Z1,Z1') of the table 5.4-1 if the CSI is triggered without a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied (according to Clause 5.2.1.6) and the CSI to be transmitted is a single CSI and corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or

-	(Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or

-	(Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the reportQuantity is set to 'ssb-Index-SINR', or reportQuantity is set to 'cri-SINR', or

-	(Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP',  where Xµ is according to UE reported capability beamReportTiming  and KBl is according to UE reported capability beamSwitchTiming as defined in [13, TS 38.306], or

-	(Z2,Z2') of table 5.4-2 otherwise.

-	µ of table 5.4-1 and table 5.4-2 corresponds to the min (µPDCCH, µCSI-RS, µUL) where the µPDCCH corresponds to the subcarrier spacing of the PDCCH with which the DCI was transmitted and µUL corresponds to the subcarrier spacing of the PUSCH with which the CSI report is to be transmitted and µCSI-RS corresponds to the minimum subcarrier spacing of the aperiodic CSI-RS triggered by the DCI

Table 5.4-1: CSI computation delay requirement 1

|  | Z1 [symbols] |  |
| --- | --- | --- |
|  | Z1 | Z'1 |
| 0 | 10 | 8 |
| 1 | 13 | 11 |
| 2 | 25 | 21 |
| 3 | 43 | 36 |

Table 5.4-2: CSI computation delay requirement 2

|  | Z1 [symbols] |  | Z2 [symbols] |  | Z3 [symbols] |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Z1 | Z'1 | Z2 | Z'2 | Z3 | Z'3 |
| 0 | 22 | 16 | 40 | 37 | 22 | X0 |
| 1 | 33 | 30 | 72 | 69 | 33 | X1 |
| 2 | 44 | 42 | 141 | 140 | min(44, X3X2+ KB1) | X2 |
| 3 | 97 | 85 | 152 | 140 | min(97, X3+ KB2) | X3 |

## 5.5	UE PDSCH reception preparation time with cross carrier scheduling with different subcarrier spacings for PDCCH and PDSCH

This clause applies only if the PDCCH carrying the scheduling DCI is received on one carrier with one OFDM subcarrier spacing (µPDCCH), and the PDSCH scheduled to be received by the DCI is on another carrier with another OFDM subcarrier spacing (µPDCCH).

If the µPDCCH < µPDSCH, the UE is expected to receive the scheduled PDSCH, if the first symbol in the PDSCH allocation, including the DM-RS, as defined by the slot offset K0 and the start and length indicator SLIV of the scheduling DCI starts no earlier than the first symbol of the slot of the PDSCH reception starting at least Npdsch PDCCH symbols after the end of the PDCCH scheduling the PDSCH, not taking into account the effect of receive timing difference between the scheduling cell and the scheduled cell.

If the µPDCCH > µPDSCH, the UE is expected to receive the scheduled PDSCH, if the first symbol in the PDSCH allocation, including the DM-RS, as defined by the slot offset K0 and the start and length indicator SLIV of the scheduling DCI starts no earlier than Npdsch PDCCH symbols after the end of the PDCCH scheduling the PDSCH, not taking into account the effect of receive timing difference between the scheduling cell and the scheduled cell.

Table 5.5-1: Npdsch as a function of the subcarrier spacing of the scheduling PDCCH

| µPDCCH | Npdsch [symbols] |
| --- | --- |
| 0 | 4 |
| 1 | 5 |
| 2 | 10 |
| 3 | 14 |

# 6	Physical uplink shared channel related procedure

## 6.1	UE procedure for transmitting the physical uplink shared channel

PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to Clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList-r16 is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell.

For the PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant activated by DCI format 0_0 or 0_1, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubset, maxRank, scaling of UCI-OnPUSCH, which are provided by pusch-Config. For the PUSCH transmission corresponding to a Type 2 configured grant activated by DCI format 0_2, the parameters applied for the transmission are provided by configuredGrantConfig except for dataScramblingIdentityPUSCH, txConfig, codebookSubsetForDCI-Format0-2-r16, maxRankForDCI-Format0-2-r16, scaling of UCI-OnPUSCH, resourceAllocationType1GranularityForDCI-Format0-2-r16 provided by pusch-Config. If the UE is provided with transformPrecoder in configuredGrantConfig, the UE applies the higher layer parameter tp-pi2BPSK, if provided in pusch-Config, according to the procedure described in Clause 6.1.4 for the PUSCH transmission corresponding to a configured grant.

For the PUSCH retransmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, the parameters in pusch-Config are applied for the PUSCH transmission except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex described in Clause 7.1 of [6, TS 38.213], mcs-Table, mcs-TableTransformPrecoder described in Clause 6.1.4.1 and transformPrecoder described in Clause 6.1.3.

For a UE configured with two uplinks in a serving cell, PUSCH retransmission for a TB on the serving cell is not expected to be on a different uplink than the uplink used for the PUSCH initial transmission of that TB.

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI. Upon detection of a DCI format 0_1 or 0_2  with "UL-SCH indicator" set to "0" and with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the "CSI request" and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH, as described in clause 6.1.2.1, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo 16 operation applied. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1 or 0_2 scrambled by C-RNTI or MCS-C-RNTI for a given HARQ process until after the end of the expected transmission of the last PUSCH for that HARQ process.

If a UE is configured by higher layer parameter PDCCH-Config that contains two different values of CORESETPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two non-overlapping in time domain PUSCHs are associated to different ControlResourceSets having different values of CORESETPoolIndex, for any two HARQ process IDs  in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH associated with a different value of CORESETpoolIndex  that ends later than symbol i.

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

If a UE receives an ACK for a given HARQ process in CG-DFI in a PDCCH ending in symbol i to terminate a transport block repetition in a PUSCH transmission with a configured grant on a given serving cell with the same HARQ process after symbol i, the UE is expected to terminate the repetition of the transport block in a PUSCH transmission starting from a symbol j if the gap between the end of PDCCH of symbol i and the start of the PUSCH transmission in symbol j is equal to or more than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH and the subcarrier spacing of the PDCCH indicating CG-DFI.

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213].

For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-r16 is set 'enabled', the UE is not configured with PUCCH resources on the active UL BWP and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS with 'QCL-TypeD' corresponding to the QCL assumption of the CORESET with the lowest ID on the active DL BWP of the cell.

For PUSCH scheduled by DCI format 0_0 on a cell and if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-r16 is set 'enabled', the UE is configured with PUCCH resources on the active UL BWP where all the PUCCH resource(s) are not configured with any spatial relation and the UE is in RRC connected mode, the UE shall transmit PUSCH according to the spatial relation, if applicable, with a reference to the RS with 'QCL-TypeD' corresponding to the QCL assumption of the CORESET with the lowest ID on the active DL BWP of the cell in case CORESET(s) are configured on the cell.

For uplink, 16 HARQ processes per cell is supported by the UE.

### 6.1.1	Transmission schemes

Two transmission schemes are supported for PUSCH: codebook based transmission and non-codebook based transmission. The UE is configured with codebook based transmission when the higher layer parameter txConfig in pusch-Config is set to 'codebook', the UE is configured non-codebook based transmission when the higher layer parameter txConfig is set to 'nonCodebook'. If the higher layer parameter txConfig is not configured, the UE is not expected to be scheduled by DCI format 0_1 or 0_2. If PUSCH is scheduled by DCI format 0_0, the PUSCH transmission is based on a single antenna port. Except if the higher layer parameter enableDefaultBeamPL-ForPUSCH0-r16 is set 'enabled', the UE shall not expect PUSCH scheduled by DCI format 0_0 in a BWP without configured PUCCH resource with PUCCH-SpatialRelationInfo in frequency range 2 in RRC connected mode.

#### 6.1.1.1	Codebook based UL transmission

For codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE determines its PUSCH transmission precoder based on SRI, TPMI and the transmission rank, where the SRI, TPMI and the transmission rank are given by DCI fields of SRS resource indicator and Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2  or given by srs-ResourceIndicator and precodingAndNumberOfLayers according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListForDCI-Format0-2-r16 in SRS-config, respectively. The TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource selected by the SRI when multiple SRS resources are configured, or if a single SRS resource is configured TPMI is used to indicate the precoder to be applied over the layers {0…ν-1} and that corresponds to the SRS resource. The transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to higher layer parameter nrofSRS-Ports in SRS-Config, as defined in Clause 6.3.1.5 of [4, TS 38.211]. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI.

For codebook based transmission, the UE determines its codebook subsets based on TPMI and upon the reception of higher layer parameter codebookSubset in pusch-Config for PUSCH associated with DCI format 0_1 and codebookSubsetForDCI-Format0-2-r16 in pusch-Config for PUSCH associated with DCI format 0_2 which may be configured with 'fullyAndPartialAndNonCoherent', or 'partialAndNonCoherent', or 'nonCoherent' depending on the UE capability. When higher layer parameter ul-FullPowerTransmission-r16 is set to 'fullpowerMode2' and the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetForDCI-Format0-2-r16 is set to 'partialAndNonCoherent', and when the SRS-resourceSet with usage set to "codebook" includes at least one SRS resource with 4 ports and one SRS resource with 2 ports, the codebookSubset associated with the 2-port SRS resource is 'nonCoherent'. The maximum transmission rank may be configured by the higher layer parameter maxRank in pusch-Config for PUSCH scheduled with DCI format 0_1 and maxRank-ForDCIFormat0_2 for PUSCH scheduled with DCI format 0_2.

A UE reporting its UE capability of 'partialAndNonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetForDCI-Format0-2-r16 with 'fullyAndPartialAndNonCoherent'.

A UE reporting its UE capability of 'nonCoherent' transmission shall not expect to be configured by either codebookSubset or codebookSubsetForDCI-Format0-2-r16 with 'fullyAndPartialAndNonCoherent' or with 'partialAndNonCoherent'.

A UE shall not expect to be configured with the higher layer parameter codebookSubset or the higher layer parameter codebookSubsetForDCI-Format0-2-r16 set to 'partialAndNonCoherent' when higher layer parameter nrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates that the maximum number of the configured SRS antenna ports in the SRS-ResourceSet is two.

For codebook based transmission, the UE may be configured with a single SRS-ResourceSet with usage set to 'codebook' and only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ul-FullPowerTransmission-r16 is set to 'fullpowerMode2', the maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources.

A UE shall not expect to be configured with higher layer parameter ul-FullPowerTransmission-r16 set to 'fullpowerMode1' and codebookSubset or codebookSubsetForDCI-Format0-2-r16 set to 'fullAndPartialAndNonCoherent' simultaneously.

The UE shall transmit PUSCH using the same antenna port(s) as the SRS port(s) in the SRS resource indicated by the DCI format 0_1 or 0_2 or by configuredGrantConfig according to clause 6.1.2.3.

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

Except when higher layer parameter ul-FullPowerTransmission-r16 is set to 'fullpowerMode2', when multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-Resource in SRS-ResourceSet shall be configured with the same value for all these SRS resources.

When higher layer parameter ul-FullPowerTransmission-r16 is set to 'fullpowerMode2',

-	the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to 'codebook'.

-	up to 2 different spatial relations can be configured for all SRS resources in the SRS resource set with usage set to 'codebook' when multiple SRS resources are configured in the SRS resource set.

-	subject to UE capability, a maximum of 2 or 4 SRS resources are supported in an SRS resource set with usage set to 'codebook'

#### 6.1.1.2	Non-Codebook based UL transmission

For non-codebook based transmission, PUSCH can be scheduled by DCI format 0_0, DCI format 0_1, DCI format 0_2 or semi-statically configured to operate according to Clause 6.1.2.3. If this PUSCH is scheduled by DCI format 0_1, DCI format 0_2, or semi-statically configured to operate according to Clause 6.1.2.3, the UE can determine its PUSCH precoder and transmission rank based on the SRI when multiple SRS resources are configured, where the SRI is given by the SRS resource indicator in DCI according to clause 7.3.1.1.2 and 7.3.1.1.3 of [5, 38.212] for DCI format 0_1 and DCI format 0_2, or the SRI is given by srs-ResourceIndicator according to clause 6.1.2.3. The SRS-ResourceSet(s) applicable for PUSCH scheduled by DCI format 0_1 and DCI format 0_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListForDCI-Format0-2-r16 in SRS-config, respectively. The UE shall use one or multiple SRS resources for SRS transmission, where, in a SRS resource set, the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities. The SRS resources transmitted simultaneously occupy the same RBs. Only one SRS port for each SRS resource is configured. Only one SRS resource set can be configured with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook'. The maximum number of SRS resources that can be configured for non-codebook based uplink transmission is 4. The indicated SRI in slot n is associated with the most recent transmission of SRS resource(s) identified by the SRI, where the SRS transmission is prior to the PDCCH carrying the SRI.

For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' if configured.

-	If aperiodic SRS resource set is configured, the associated NZP-CSI-RS is indicated via SRS request field in DCI format 0_1 and 1_1, as well as DCI format 0_2 (if SRS request field is present) and DCI format 1_2 (if SRS request field is present), where AperiodicSRS-ResourceTrigger and AperiodicSRS-ResourceTriggerList (indicating the association between aperiodic SRS triggering state(s) and SRS resource sets), triggered SRS resource(s) srs-ResourceSetId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId) are higher layer configured in SRS-ResourceSet. The SRS-ResourceSet(s) associated with the SRS request by DCI format 0_1 and 1_1 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and the SRS-ResourceSet(s) associated with the SRS request by DCI format 0_2 and 1_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModListForDCI-Format0-2-r16. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the aperiodic NZP-CSI-RS resource and the first symbol of the aperiodic SRS transmission is less than 42 OFDM symbols.

-	If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, the presence of the associated CSI-RS is indicated by the SRS request field if the value of the SRS request field is not '00' as in Table 7.3.1.1.2-24 of [5, TS 38.212] and if the scheduling DCI is not used for cross carrier or cross bandwidth part scheduling. The CSI-RS is located in the same slot as the SRS request field. If the UE configured with aperiodic SRS associated with aperiodic NZP CSI-RS resource, any of the TCI states configured in the scheduled CC shall not be configured with 'QCL-TypeD'.

-	If periodic or semi-persistent SRS resource set is configured, the NZP-CSI-RS-ResourceId for measurement is indicated via higher layer parameter associatedCSI-RS in SRS-ResourceSet.

The UE shall perform one-to-one mapping from the indicated SRI(s) to the indicated DM-RS ports(s) and their corresponding PUSCH layers {0 … ν-1} given by DCI format 0_1 or by configuredGrantConfig according to clause 6.1.2.3 in increasing order.

The UE shall transmit PUSCH using the same antenna ports as the SRS port(s) in the SRS resource(s) indicated by SRI(s) given by DCI format 0_1 or by configuredGrantConfig according to clause 6.1.2.3, where the SRS port in (i+1)-th SRS resource in the SRS resource set is indexed as .

The DM-RS antenna ports  in Clause 6.4.1.1.3 of [4, TS 38.211] are determined according to the ordering of DM-RS port(s) given by Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 in Clause 7.3.1.1.2 of [5, TS 38.212].

For non-codebook based transmission, the UE does not expect to be configured with both spatialRelationInfo for SRS resource and associatedCSI-RS in SRS-ResourceSet for SRS resource set.

For non-codebook based transmission, the UE can be scheduled with DCI format 0_1 when at least one SRS resource is configured in SRS-ResourceSet with usage set to 'nonCodebook'.

### 6.1.2	Resource allocation

#### 6.1.2.1	Resource allocation in time domain

When the UE is scheduled to transmit a transport block and no CSI report, or the UE is scheduled to transmit a transport block and a CSI report(s) on PUSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocated table. The determination of the used resource allocation table is defined in Clause 6.1.2.1.1. The indexed row defines the slot offset K2, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, the PUSCH mapping type, and the number of repetitions (if numberOfRepetitions-r16 is present in the resource allocation table) to be applied in the PUSCH transmission.

When the UE is scheduled to transmit a PUSCH with no transport block and with a CSI report(s) by a CSI request field on a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to the allocated table as defined in Clause 6.1.2.1.1. The indexed row defines the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PUSCH mapping type to be applied in the PUSCH transmission and the K2 value is determined as , where  are the corresponding list entries of the higher layer parameter

-	reportSlotOffsetListForDCI-Format0-2-r16, if PUSCH is scheduled by DCI format 0_2 and reportSlotOffsetListForDCI-Format0-2-r16 is configured;

-	reportSlotOffsetListForDCI-Format0-1-r16, if PUSCH is scheduled by DCI format 0_1 and reportSlotOffsetListForDCI-Format0-1-r16 is configured;

-	reportSlotOffsetList, otherwise;

in CSI-ReportConfig for the  triggered CSI Reporting Settings and  is the (m+1)th entry of .

-	The slot Ks where the UE shall transmit the PUSCH is determined by K2 as Ks =, if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, Ks =, otherwise, and where n is the slot with the scheduling DCI, K2 is based on the numerology of PUSCH, and  and  are the subcarrier spacing configurations for PUSCH and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot, offset, PUSCHCA and μoffset,PUSCH are the Nslot, offsetCA and the,respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the PUSCH, as defined in clause 4.5 of [4, TS 38.211], and

-	for PUSCH scheduled by DCI format 0_1, if pusch-RepTypeIndicatorForDCI-Format0-1-r16 is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. For PUSCH scheduled by DCI format 0_2, if pusch-RepTypeIndicatorForDCI-Format0-2-r16 is set to 'pusch-RepTypeB', the UE applies PUSCH repetition Type B procedure when determining the time domain resource allocation. Otherwise, the UE applies PUSCH repetition Type A procedure when determining the time domain resource allocation for PUSCH scheduled by PDCCH.

-	For PUSCH repetition Type A, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are determined from the start and length indicator SLIV of the indexed row:

if  then

else

where, and

-	For PUSCH repetition Type B, the starting symbol S relative to the start of the slot, and the number of consecutive symbols L counting from the symbol S allocated for the PUSCH are provided by startSymbol-r16 and length-r16 of the indexed row of the resource allocation table, respectively.

-	For PUSCH repetition Type A, the PUSCH mapping type is set to Type A or Type B as defined in Clause 6.4.1.1.3 of [4, TS 38.211] as given by the indexed row.

-	For PUSCH repetition Type B, the PUSCH mapping type is set to Type B.

The UE shall consider the S and L combinations defined in table 6.1.2.1-1 as valid PUSCH allocations

Table 6.1.2.1-1: Valid S and L combinations

| PUSCH mapping type | Normal cyclic prefix |  |  | Extended cyclic prefix |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | S | L | S+L | S | L | S+L |
| Type A (repetition Type A only) | 0 | {4,…,14} | {4,…,14} | 0 | {4,…,12} | {4,…,12} |
| Type B | {0,…,13} | {1,…,14} | {1,…,14} for repetition Type A, {1,…,27} for repetition Type B | {0,…, 11} | {1,…,12} | {1,…,12} for repetition Type A, {1,…,23} for repetition Type B |

For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as

-	if numberOfRepetitions-r16 is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions-r16;

-	elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor;

-	otherwise K=1.

For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2.

Table 6.1.2.1-2: Redundancy version for PUSCH transmission

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) or nth actual repetition (repetition Type B) |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

When transmitting MsgA PUSCH on a non-initial UL BWP, if the UE is configured with startSymbolAndLengthMsgA-PO, the UE shall determine the S and L from startSymbolAndLengthMsgA-PO.

When transmitting MsgA PUSCH, if the UE is not configured with startSymbolAndLengthMsgA-PO, and if the TDRA list PUSCH-TimeDomainResourceAllocationList is provided in PUSCH-ConfigCommon, the UE shall use msgA-PUSCH-TimeDomainAllocation to indicate which values are used in the list. If PUSCH-TimeDomainResourceAllocationList is not provided in PUSCH-ConfigCommon, the UE shall use parameters S and L from table 6.1.2.1.1-2 or table 6.1.2.1.1-3 where msgA-PUSCH-TimeDomainAllocation indicates which values are used in the list. The time offset for PUSCH transmission is described in [6, TS38.213].

For PUSCH repetition Type A, a PUSCH transmission in a slot of a multi-slot PUSCH transmission is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].

For PUSCH repetition Type B, except for PUSCH transmitting CSI report(s) with no transport block, the number of nominal repetitions is given by numberOfRepetitions-r16. For the n-th nominal repetition, n = 0, …, numberOfRepetitions-r16 - 1,

-	The slot where the nominal repetition starts is given by , and the starting symbol relative to the start of the slot is given by .

-	The slot where the nominal repetition ends is given by , and the ending symbol relative to the start of the slot is given by .

Here is the slot where the PUSCH transmission starts, and is the number of symbols per slot as defined in Clause 4.3.2 of [4, TS38.211].

For PUSCH repetition Type B, the UE determines invalid symbol(s) for PUSCH repetition Type B transmission as follows:

-	A symbol that is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated is considered as an invalid symbol for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s) for PUSCH repetition Type B transmission.

-	For operation in unpaired spectrum, if numberOfInvalidSymbolsForDL-UL-Switching-r16 is configured, numberOfInvalidSymbolsForDL-UL-Switching-r16 symbol(s) after the last symbol that is indicated as downlink in each consecutive set of all symbols that are indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated are considered as invalid symbol(s) for PUSCH repetition Type B transmission. The symbol(s) given by numberOfInvalidSymbolsForDL-UL-Switching-r16 are defined using the reference SCS configuration referenceSubcarrierSpacing provided in tdd-UL-DL-ConfigurationCommon.

-	The UE may be configured with the higher layer parameter invalidSymbolPattern-r16, which provides a symbol level bitmap spanning one or two slots (higher layer parameter symbols-r16 given by invalidSymbolPattern-r16). A bit value equal to 1 in the symbol level bitmap symbols-r16 indicates that the corresponding symbol is an invalid symbol for PUSCH repetition Type B transmission. The UE may be additionally configured with a time-domain pattern (higher layer parameter periodicityAndPattern-r16 given byinvalidSymbolPattern-r16), where each bit of periodicityAndPattern-r16 corresponds to a unit equal to a duration of the symbol level bitmap symbols-r16, and a bit value equal to 1 indicates that the symbol level bitmap symbols-r16 is present in the unit. The periodicityAndPattern-r16 can be {1, 2, 4, 5, 8, 10, 20 or 40} units long, but maximum of 40ms. The first symbol of periodicityAndPattern-r16 every 40ms/P periods is a first symbol in frame 𝑛𝑓 mod 4 = 0, where P is the duration of periodicityAndPattern-r16 in units of ms. When periodicityAndPattern-r16 is not configured, for a symbol level bitmap spanning two slots, the bits of the first and second slots correspond respectively to even and odd slots of a radio frame, and for a symbol level bitmap spanning one slot, the bits of the slot correspond to every slot of a radio frame. If invalidSymbolPattern-r16 is configured, when the UE applies the invalid symbol pattern is determined as follows:

-	if the PUSCH is scheduled by DCI format 0_1, or corresponds to a Type 2 configured grant activated by DCI format 0_1, and if invalidSymbolPatternIndicatorForDCI-Format0-1-r16 is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	if the PUSCH is scheduled by DCI format 0_2, or corresponds to a Type 2 configured grant activated by DCI format 0_2, and if invalidSymbolPatternIndicatorForDCI-Format0-2-r16 is configured,

-	if invalid symbol pattern indicator field is set 1, the UE applies the invalid symbol pattern;

-	otherwise, the UE does not apply the invalid symbol pattern;

-	otherwise, the UE applies the invalid symbol pattern.

-	If the UE

-	is configured with multiple serving cells and is provided half-duplex-behavior-r16 = 'enable', and

-	is not capable of simultaneous transmission and reception on any of the multiple serving cells, and

-	indicates support of capability for half-duplex operation in CA with unpaired spectrum, and

-	is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells,

-	a symbol is considered as an invalid symbol in any of the multiple serving cells for PUSCH repetition Type B transmission if the symbol is indicated to the UE for reception of SS/PBCH blocks in any of the multiple serving cells by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, and

a symbol is considered as an invalid symbol in any of the multiple serving cells for PUSCH repetition Type B transmission with Type 1 or Type 2 configured grant except for the first Type 2 PUSCH transmission (including all repetitions) after activation if the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on the reference cell, or the UE is configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on the reference cell in the symbol.

For PUSCH repetition Type B, after determining the invalid symbol(s) for PUSCH repetition type B transmission for each of the K nominal repetitions, the remaining symbols are considered as potentially valid symbols for PUSCH repetition Type B transmission. If the number of potentially valid symbols for PUSCH repetition type B transmission is greater than zero for a nominal repetition, the nominal repetition consists of one or more actual repetitions, where each actual repetition consists of a consecutive set of all potentially valid symbols that can be used for PUSCH repetition Type B transmission within a slot. An actual repetition with a single symbol is omitted except for the case of L=1. An actual repetition is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213]. The redundancy version to be applied on the nth actual repetition (with the counting including the actual repetitions that are omitted) is determined according to table 6.1.2.1-2.

For PUSCH repetition Type B, when a UE receives a DCI that schedules aperiodic CSI report(s) or activates semi-persistent CSI report(s) on PUSCH with no transport block by a CSI request field on a DCI, the number of nominal repetitions is always assumed to be 1, regardless of the value of numberOfRepetitions-r16. When the UE is scheduled to transmit a PUSCH repetition Type B with no transport block and with aperiodic or semi-persistent CSI report(s) by a CSI request field on a DCI, the first nominal repetition is expected to be the same as the first actual repetition. For PUSCH repetition Type B carrying semi-persistent CSI report(s) without a corresponding PDCCH after being activated on PUSCH by a CSI request field on a DCI, if the first nominal repetition is not the same as the first actual repetition, the first nominal repetition is omitted; otherwise, the first nominal repetition is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].

For PUSCH repetition Type B, when a UE is scheduled to transmit a transport block and aperiodic CSI report(s) on PUSCH by a CSI request field on a DCI, the CSI report(s) is multiplexed only on the first actual repetition. The UE does not expect that the first actual repetition has a single symbol duration.

If pusch-TimeDomainAllocationList in pusch-Config contains row indicating resource allocation for two to eight contiguous PUSCHs, K2 indicates the slot where UE shall transmit the first PUSCH of the multiple PUSCHs. Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is signalled by the number of indicated valid SLIVs in the row of the pusch-TimeDomainAllocationList signalled in DCI format 0_1.

When the UE is configured with minimumSchedulingOffsetK2 in an active UL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or DCI format 1_1 if the same field is available. When the UE configured with minimumSchedulingOffsetK2 in an active UL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to transmit a PUSCH scheduled with C-RNTI, CS-RNTI, MCS-C-RNTI or SP-CSI-RNTI with K2 smaller than K2min⋅2μ'2μ, where K2min and μ are the applied minimum scheduling offset restriction and the numerology of the active UL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active UL BWP in case of active UL BWP change in the scheduled cell and is equal to μ, otherwise. The minimum scheduling restriction is not applied when PUSCH transmission is scheduled by RAR UL grant or fallbackRAR UL grant for RACH procedure, or when PUSCH is scheduled with TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

##### 6.1.2.1.1	Determination of the resource allocation table to be used for PUSCH

Table 6.1.2.1.1-1, Table 6.1.2.1.1-1A and Table 6.1.2.1.1-1B define which PUSCH time domain resource allocation configuration to apply.

Table 6.1.2.1.1-4 defines the subcarrier spacing specific values j. j is used in determination of K2 in conjunction to table 6.1.2.1.1-2, for normal CP or table 6.1.2.1.1.-3 for extended CP, where µPUSCH is the subcarrier spacing configurations for PUSCH.

Table 6.1.2.1.1-5 defines the additional subcarrier spacing specific slot delay value for the first transmission of PUSCH scheduled by the RAR or by the fallback RAR. When the UE transmits a PUSCH scheduled by RAR or by the fallback RAR, the Δ value specific to the PUSCH subcarrier spacing µPUSCH is applied in addition to the K2 value.

Table 6.1.2.1.1-1: Applicable PUSCH time domain resource allocation for common search space and DCI format 0_0 in UE specific search space

| RNTI | PDCCH search space | pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in clause 8.2 of [6, TS 38.213] or MAC fallback RAR as described in clause 8.2A of [6, 38.213] or for MsgA PUSCH transmission |  | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SP-CSI-RNTI | Any common search space not associated with CORESET 0, DCI format 0_0 in UE specific search space | No | No | Default A |
|  |  | Yes | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
|  |  | No/Yes | Yes | pusch-TimeDomainAllocationList provided in pusch-Config |

Table 6.1.2.1.1-1A: Applicable PUSCH time domain resource allocation for DCI format 0_1 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListForDCI-Format0-1-r16 | pusch-Config includes pusch-TimeDomainAllocationList-ForMultiPUSCH | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| No | No | No | No | Default A |
| Yes | No | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No/Yes | Yes | - | pusch-TimeDomainAllocationListForDCI-Format0-1-r16 provided in pusch-Config |
| No/Yes | No/Yes | - | Yes | pusch-TimeDomainAllocationList-ForMultiPUSCH provided in pusch-Config |

Table 6.1.2.1.1-1B: Applicable PUSCH time domain resource allocation for DCI format 0_2 in UE specific search space scrambled with C-RNTI, MCS-C-RNTI, CS-RNTI or SP-CSI-RNTI

| pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationListForDCI-Format0-2-r16 | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- |
| No | No | No | Default A |
| Yes | No | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| No/Yes | Yes | No | pusch-TimeDomainAllocationList provided in pusch-Config |
| No/Yes | No/Yes | Yes | pusch-TimeDomainAllocationListForDCI-Format0-2-r16 provided in pusch-Config |

Table 6.1.2.1.1-2: Default PUSCH time domain resource allocation A for normal CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 14 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 10 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 14 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 14 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 6 |
| 15 | Type A | j+3 | 0 | 14 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-3: Default PUSCH time domain resource allocation A for extended CP

| Row index | PUSCH mapping type | K2 | S | L |
| --- | --- | --- | --- | --- |
| 1 | Type A | j | 0 | 8 |
| 2 | Type A | j | 0 | 12 |
| 3 | Type A | j | 0 | 10 |
| 4 | Type B | j | 2 | 10 |
| 5 | Type B | j | 4 | 4 |
| 6 | Type B | j | 4 | 8 |
| 7 | Type B | j | 4 | 6 |
| 8 | Type A | j+1 | 0 | 8 |
| 9 | Type A | j+1 | 0 | 12 |
| 10 | Type A | j+1 | 0 | 10 |
| 11 | Type A | j+2 | 0 | 6 |
| 12 | Type A | j+2 | 0 | 12 |
| 13 | Type A | j+2 | 0 | 10 |
| 14 | Type B | j | 8 | 4 |
| 15 | Type A | j+3 | 0 | 8 |
| 16 | Type A | j+3 | 0 | 10 |

Table 6.1.2.1.1-4: Definition of value j

| µPUSCH | j |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 3 |

Table 6.1.2.1.1-5: Definition of value Δ

| µPUSCH | Δ |
| --- | --- |
| 0 | 2 |
| 1 | 3 |
| 2 | 4 |
| 3 | 6 |

#### 6.1.2.2	Resource allocation in frequency domain

The UE shall determine the resource block assignment in frequency domain using the resource allocation field in the detected PDCCH DCI except for a PUSCH transmission scheduled by a RAR UL grant or fallbackRAR UL grant, in which case the frequency domain resource allocation is determined according to clause 8.3 of [6, 38.213] or a MsgA PUSCH transmission with frequency domain resource allocation determined according to clause 8.1A of [6, 38.213]. Three uplink resource allocation schemes type 0, type 1 and type 2 are supported. Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. Uplink resource allocation scheme type 1 and type 2 are supported for PUSCH for both cases when transform precoding is enabled or disabled.

If the scheduling DCI is configured to indicate the uplink resource allocation type as part of the Frequency domain resource assignment field by setting a higher layer parameter resourceAllocation in pusch-Config to 'dynamicSwitch', for DCI format 0_1 or setting a higher layer parameter resourceAllocationForDCI-Format0-2-r16 in pusch-Config to 'dynamicswitch' for DCI format 0_2, the UE shall use uplink resource allocation type 0 or type 1 as defined by this DCI field. Otherwise the UE shall use the uplink frequency resource allocation type as defined by the higher layer parameter resourceAllocation for DCI format 0_1 or the higher layer parameter resourceAllocationForDCI-Format0-2-r16 for DCI format 0_2. The UE shall assume that when the scheduling PDCCH is received with DCI format 0_1 and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, uplink type 2 resource allocation is used.

The UE shall assume that when the scheduling PDCCH is received with DCI format 0_0, then uplink resource allocation type 1 is used, except when any of the higher layer parameters useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured in which case uplink resource allocation type 2 is used.

The UE expects that either none or both of useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured..

If a bandwidth part indicator field is not configured in the scheduling DCI or the UE does not support active bandwidth part change via DCI, the RB indexing for uplink type 0, type 1 and type 2 resource allocation is determined within the UE's active bandwidth part. If a bandwidth part indicator field is configured in the scheduling DCI and the UE supports active bandwidh part change via DCI, the RB indexing for uplink type 0, type 1, type 2 resource allocation is determined within the UE's bandwidth part indicated by bandwidth part indicator field value in the DCI. The UE shall upon detection of PDCCH intended for the UE determine first the uplink bandwidth part and then the resource allocation within the bandwidth part. RB numbering starts from the lowest RB in the determined uplink bandwidth part.

##### 6.1.2.2.1	Uplink resource allocation type 0

In uplink resource allocation of type 0, the resource block assignment information includes a bitmap indicating the Resource Block Groups (RBGs) that are allocated to the scheduled UE where a RBG is a set of consecutive virtual resource blocks defined by higher layer parameter rbg-Size configured in pusch-Config and the size of the bandwidth part as defined in Table 6.1.2.2.1-1.

Table 6.1.2.2.1-1: Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

The total number of RBGs () for a uplink bandwidth part i of sizePRBs is given by  where

-	the size of the first RBG is ,

-	the size of the last RBG is if and P otherwise.

-	the size of all other RBG is P.

The bitmap is of size bits with one bitmap bit per RBG such that each RBG is addressable. The RBGs shall be indexed in the order of increasing frequency of the bandwidth part and starting at the lowest frequency. The order of RBG bitmap is such that RBG 0 to RBG are mapped from MSB to LSB of the bitmap. The RBG is allocated to the UE if the corresponding bit value in the bitmap is 1, the RBG is not allocated to the UE otherwise.

In frequency range 1, only 'almost contiguous allocation' defined in [8, TS 38.101-1] is allowed as non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM.

In frequency range 2, non-contiguous allocation per component carrier for UL RB allocation for CP-OFDM is not supported.

##### 6.1.2.2.2	Uplink resource allocation type 1

In uplink resource allocation of type 1, the resource block assignment information indicates to a scheduled UE a set of contiguously allocated non-interleaved virtual resource blocks within the active bandwidth part of size  PRBs except for the case when DCI format 0_0 is decoded in any common search space in which case the size of the initial UL bandwidth part  shall be used.

An uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting virtual resource block () and a length in terms of contiguously allocated resource blocks. The resource indication value is defined by

if  then

else

where 1 and shall not exceed.

When the DCI size for DCI format 0_0 in USS is derived from the initial UL BWP with size  but applied to another active BWP with size of , an uplink type 1 resource block assignment field consists of a resource indication value (RIV) corresponding to a starting resource block and a length in terms of virtually contiguously allocated resource blocks .

The resource indication value is defined by

if  then

else

where, and where shall not exceed .

If , K is the maximum value from set {1, 2, 4, 8} which satisfies ; otherwise K = 1.

When the scheduling grant is received with DCI format 0_2, an uplink type 1 resource allocation field consists of a resource indication value (RIV) corresponding to a starting resource block group RBGstart=0, 1, …, NRBG-1 and a length in terms of virtually contiguously allocated resource block groups LRBGs=1, …, NRBG, where the resource block groups are defined as in 6.1.2.2.1 with P defined by resourceAllocationType1GranularityForDCI-Format0-2-r16 if the UE is configured with higher layer parameter resourceAllocationType1GranularityForDCI-Format0-2-r16, and P=1 otherwise. The resource indication value is defined by

if  then

else

where 1 and shall not exceed .

##### 6.1.2.2.3	Uplink resource allocation type 2

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_0 monitored in a UE-specific search space and DCI 0_1 a set of up to  NRB-set,ULBWP contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. Within the active UL BWP, the assigned physical resource block n is mapped to virtual resource block n. For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the union of the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. For DCI 0_0 monitored in a common search space, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP. For DCI 0_0 monitored in a CSS with CRC scrambled by an RNTI other than TC-RNTI, the uplink RB set is the lowest indexed one amongst uplink RB set(s) that intersects the lowest-indexed CCE of the PDCCH in which the UE detects the DCI 0_0 in the active downlink BWP. If there is no intersection, the uplink RB set is RB set 0 in the active uplink BWP. For DCI 0_0 with CRC scrambled by TC-RNTI, the uplink RB set is the same one in which the UE transmits the PRACH associated with the RAR UL grant, in which case the UE assumes that the uplink RB set is defined as when the UE is not configured with intraCellGuardBandUL-r16 (see Clause 7).

For µ=0, the X=6 MSBs of the resource block assignment information indicates to a UE a set of allocated interlace indices  m0+l, where the indication consists of a resource indication value (RIV). For 0≤RIV<M(M+1)/2 , l=0,1,⋯L-1 the resource indication value corresponds to the starting interlace index m0 and the number of contiguous interlace indices (). The resource indication value is defined by:

if (L-1)≤M/2 then

RIV=M(L-1)+m0

else

RIV=M(M-L+1)+(M-1-m0)

For RIV≥M(M+1)/2 , the resource indication value corresponds to the starting interlace index m0 and the set of values  according to Table 6.1.2.2.3-1.

Table 6.1.2.2.3-1: m0  and  for RIV≥M(M+1)/2.

| RIV-M(M+1)/2 | m0 |  |
| --- | --- | --- |
| 0 | 0 | {0, 5} |
| 1 | 0 | {0, 1, 5, 6} |
| 2 | 1 | {0, 5} |
| 3 | 1 | {0, 1, 2, 3, 5, 6, 7, 8} |
| 4 | 2 | {0, 5} |
| 5 | 2 | {0, 1, 2, 5, 6, 7} |
| 6 | 3 | {0, 5} |
| 7 | 4 | {0, 5} |

For µ=1, the X=5 MSBs of the resource block assignment information comprise a bitmap indicating the interlaces that are allocated to the scheduled UE. The bitmap is of size M bits with one bitmap bit per interlace such that each interlace is addressable, where M and interlace indexing is defined in Clause 4.4.4.6 in [4, TS 38.211]. The order of interlace bitmap is such that interlace 0 to interlace M-1 are mapped from MSB to LSB of the bitmap. An interlace is allocated to the UE if the corresponding bit value in the bitmap is 1; otherwise the interlace is not allocated to the UE.

For DCI 0_0 monitored in a UE-specific search space and DC 0_1 for both µ=0 and µ=1, the Y=log2NRB-set,ULBWPNRB-set,ULBWP+12LSBs of  the resource block assignment information indicate to a UE a set of contiguously allocated RB sets for PUSCH scheduled by DCI 0_0 monitored in a UE-specific search space, DCI 0_1 and Type 1 and Type 2 configured grant. The resource allocation field consists of a resource indication value (RIVRB-set). For 0≤RIVRB-set<NRB-set,ULBWP(NRB-set,ULBWP+1)/2 , l=0,1,⋯LRBset-1 the resource indication value corresponds to the starting RB set index NRB-set,ULstart and the number of contiguous RB sets  LRB-set . The resource indication value is defined by;

if (LRB-set-1)≤NRB-set,ULBWP/2 then

RIVRB-set=NRB-set,ULBWP(LRB-set-1)+NRB-set,ULstart

else

RIVRB-set=NRB-set,ULBWP(NRB-set,ULBWP-LRB-set+1)+(NRB-set,ULBWP-1-NRB-set,ULstart)

where  NRB-set,ULstart=0,1,⋯NRB-set,ULBWP-1, LRB-set≥1 and shall not exceed NRB-set,ULBWP-NRB-set,ULstart

If transform precoding is enabled according to the procedure in Clause 6.1.3, then the UE transmits PUSCH on the lowest-indexed MRBPUSCH PRBs amongst the PRBs indicated by the frequency domain resource assignment information. MRBPUSCH is the largest integer not greater than the number of RBs indicated by the frequency domain resource assignment information that fulfils the conditions in Clause 6.3.1.4 of [4, TS 38.211].

#### 6.1.2.3	Resource allocation for uplink transmission with configured grant

When PUSCH resource allocation is semi-statically configured by higher layer parameter configuredGrantConfig in BWP-UplinkDedicated information element, and the PUSCH transmission corresponding to a configured grant, the following higher layer parameters are applied in the transmission:

-	For Type 1 PUSCH transmissions with a configured grant, the following parameters are given in configuredGrantConfig unless mentioned otherwise:

-	For the determination of the PUSCH repetition type, if the higher layer parameter pusch-RepTypeIndicator-r16 in rrc-ConfiguredUplinkGrant is configured and set to 'pusch-RepTypeB', PUSCH repetition type B is applied; otherwise, PUSCH repetition type A is applied;

-	For PUSCH repetition type A, the selection of the time domain resource allocation table follows the rules for DCI format 0_0 on UE specific search space, as defined in Clause 6.1.2.1.1.

-	For PUSCH repetition type B, the selection of the time domain resource allocation table is as follows:

-	If pusch-RepTypeIndicatorForDCI-Format0-1-r16 in pusch-Config is configured and set to 'pusch-RepTypeB', PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_1 in pusch-Config is used;

-	Otherwise, PUSCH-TimeDomainResourceAllocationList-ForDCIformat0_2 in pusch-Config is used.

-	It is not expected that pusch-RepTypeIndicator-r16 in rrc-ConfiguredUplinkGrant is configured with 'pusch-RepTypeB' when none of pusch-RepTypeIndicatorForDCI-Format0-1-r16 and pusch-RepTypeIndicatorForDCI-Format0-2-r16 in pusch-Config is set to 'pusch-RepTypeB'.

-	The higher layer parameter timeDomainAllocation value m provides a row index m+1 pointing to the determined time domain resource allocation table, where the start symbol and length are determined following the procedure defined in Clause 6.1.2.1;

-	Frequency domain resource allocation is determined by the N LSB bits in the higher layer parameter frequencyDomainAllocation, forming a bit sequence f17, …, f1, f0, where f0 is the LSB, according to the procedure in Clause 6.1.2.2 and N is determined as the size of frequency domain resource assignment field in DCI format 0_1 for a given resource allocation type indicated by resourceAllocation, except if useInterlacePUCCH-PUSCH in BWP-UplinkDedicated is configured, in which case uplink type 2 resource allocation is used wherein the UE interprets the LSB bits in the higher layer parameter frequencyDomainAllocation as for the frequency domain resource assignment field of DCI 0_1 according to the procedure in Clause 6.1.2.2.3;

-	The IMCS is provided by higher layer parameter mcsAndTBS;

-	Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Clause 7.3.1.1.2 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively;

-	When frequency hopping is enabled, the frequency offset between two frequency hops can be configured by higher layer parameter frequencyHoppingOffset.

-	For Type 2 PUSCH transmissions with a configured grant: the resource allocation follows the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI.

-	The PUSCH repetition type and the time domain resource allocation table are determined by the PUSCH repetition type and the time domain resource allocation table associated with the UL grant received on the DCI, respectively, as defined in Clause 6.1.2.1.

For PUSCH transmissions with a Type 1 or Type 2 configured grant, the number of (nominal) repetitions K to be applied to the transmitted transport block is provided by the indexed row in the time domain resource allocation table if numberOfRepetitions-r16 is present in the table; otherwise K is provided by the higher layer configured parameters repK.

The UE shall not transmit anything on the resources configured by configuredGrantConfig if the higher layers did not deliver a transport block to transmit on the resources allocated for uplink transmission without grant.

A set of allowed periodicities P are defined in [12, TS 38.331]. The higher layer parameter cg-nrofSlots-r16, provides the number of consecutive slots allocated within a configured grant period. The higher layer parameter cg-nrofPUSCH-InSlot-r16 provides the number of consecutive PUSCH allocations within a slot, where the first PUSCH allocation follows the higher layer parameter timeDomainAllocation for Type 1 PUSCH transmission or the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI for Type 2 PUSCH transmissions, and the remaining PUSCH allocations have the same length and PUSCH mapping type, and are appended following the previous allocations without any gaps. The same combination of start symbol and length and PUSCH mapping type repeats over the consecutively allocated slots.

For operation with shared spectrum channel access where a UE is performing uplink transmission with configured grants in contiguous OFDM symbols on all resource blocks of an RB set, for the first such UL transmission the UE determines a duration of a cyclic prefix extension Text to be applied for transmission according to [4, TS 38.211] where the index for Δi [4, TS 38.211] is chosen randomly from a set of values configured by higher layers according to the following rule:

-	If the first such UL transmission is within a channel occupancy initiated by the gNB (defined in Clause 4 of [16, TS 37.213]), the set of values is determined by cg-StartingFullBW-InsideCOT-r16;

-	otherwise, the set of values is determined by cg-StartingFullBW-OutsideCOT-r16.

For operation with shared spectrum channel access where a UE is performing uplink transmission with configured grants in contiguous OFDM symbols on fewer than all resource blocks of an RB set, for the first such UL transmission the UE determines a duration of a cyclic prefix extension Text to be applied for transmission according to [4, TS 38.211] according to the following rule:

-	If the first such UL transmission is within a channel occupancy initiated by the gNB (defined in Clause 4 of [16, TS 37.213]), the index for Δi [4, TS 38.211]  is equal to cg-StartingPartialBW-InsideCOT-r16;

-	otherwise, the index for Δi [4, TS 38.211]  is equal to cg-StartingPartialBW-OutsideCOT-r16.

##### 6.1.2.3.1	Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

The procedures described in this clause apply to PUSCH transmissions of PUSCH repetition Type A with a Type 1 or Type 2 configured grant.

The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If cg-RetransmissionTimer is provided, the redundancy version for uplink transmission with a configured grant is determined by the UE. If the parameter repK-RV is not provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig and cg-RetransmissionTimer is not provided, for the nth transmission occasion among K repetitions, n=1, 2, …, K, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with startingFromRV0-r16 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the K repetitions. Otherwise, the initial transmission of a transport block may start at

-	the first transmission occasion of the K repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the K repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the K repetitions if the configured RV sequence is {0,0,0,0}, except the last transmission occasion when K≥8.

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1 or 0_2, whichever is reached first. In addition, the UE shall terminate the repetition of a transport block in a PUSCH transmission if the UE receives a DCI format 0_1 with DFI flag provided and set to '1', and if in this DCI the UE detects ACK for the HARQ process corresponding to that transport block.

The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available for the PUSCH transmission in a slot is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion.

For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, the UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 9, Clause 11.1 and Clause 11.2A of [6, TS38.213].

##### 6.1.2.3.2	Transport Block repetition for uplink transmissions of PUSCH repetition Type B with a configured grant

The procedures described in this Clause apply to PUSCH transmissions of PUSCH repetition type B with a Type 1 or Type 2 configured grant.

For PUSCH transmissions with a Type 1 or Type 2 configured grant, the nominal repetitions and the actual repetitions are determined according to the procedures for PUSCH repetition Type B defined in Clause 6.1.2.1. The higher layer configured parameters repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for each actual repetition with a configured grant shall be set to 0. Otherwise, for the nth transmission occasion among all the actual repetitions (including the actual repetitions that are omitted) of the K nominal repetitions, it is associated with (mod(n-1,4)+1)th value in the configured RV sequence. If a configured grant configuration is configured with startingFromRV0-r16 set to 'off', the initial transmission of a transport block may only start at the first transmission occasion of the actual repetitions. Otherwise, the initial transmission of a transport block may start at

-	the first transmission occasion of the actual repetitions if the configured RV sequence is {0,2,3,1},

-	any of the transmission occasions of the actual repetitions that are associated with RV=0 if the configured RV sequence is {0,3,0,3},

-	any of the transmission occasions of the actual repetitions if the configured RV sequence is {0,0,0,0}, except the actual repetitions within the last nominal repetition when K≥8.

For any RV sequence, the repetitions shall be terminated after transmitting K nominal repetitions, or at the last transmission occasion among the K nominal repetitions within the period P, or from the starting symbol of a repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1 or 0_2, whichever is reached first. The UE is not expected to be configured with the time duration for the transmission of K nominal repetitions larger than the time duration derived by the periodicity P.

### 6.1.3	UE procedure for applying transform precoding on PUSCH

For a PUSCH scheduled by RAR UL grant, or for a PUSCH scheduled by fallbackRAR UL grant, or for a PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

For a MsgA PUSCH, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msgA-TransformPrecoder. If higher layer parameter msgA-TransformPrecoder is not configured, the UE shall consider the transform precoding either 'enabled' or 'disabled' according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI:

-	If the DCI with the scheduling grant was received with DCI format 0_0, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

-	If the DCI with the scheduling grant was not received with DCI format 0_0

-	If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission with a configured grant

-	If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

### 6.1.4	Modulation order, redundancy version and transport block size determination

To determine the modulation order, target code rate, redundancy version and transport block size for the physical uplink shared channel, the UE shall first

-	read the 5-bit modulation and coding scheme field in the DCI scheduling PUSCH or provided in a DCI activating a configured grant Type 2 PUSCH, or as provided by mcsAndTBS as described in Clause 6.1.2.3 for a configured grant Type 1 PUSCH to determine the modulation order  and target code rate (R) based on the procedure defined in Clause 6.1.4.1

-	read redundancy version field (rv) in the DCI to determine the redundancy version for PUSCH scheduled by DCI, or determine the redundancy version according to Clause 6.1.2.3.1 for configured grant Type 1 and Type 2 PUSCH,

and second

-	use the number of layers , the total number of allocated PRBs  to determine the transport block size based on the procedure defined in Clause 6.1.4.2.

When the UE is scheduled with multiple PUSCHs by a DCI, as described in clause 6.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one to one mapped to the scheduled PUSCH(s) with the corresponding transport block(s) in the scheduled order where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PUSCH.

Within a cell group, a UE is not required to handle PUSCH(s) transmissions in slot sj in serving cell-j, and for j = 0,1,2.. J-1, slot sj overlapping with any given point in time, if the following condition is not satisfied at that point in time:

j=0J-1m=0M-1Vj,mTslotμ(j)≤DataRate,

where

-	J is the number of configured serving cells belong to a frequency range

-	for the j-th serving cell,

-	M is the number of TB(s) transmitted in slot-sj.

-	Tslot(j) =10-3/2(j), where (j) is the numerology for PUSCH(s) in slot sj of the j-th serving cell.

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 6.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212].

-	C'is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5,38.212]

-	DataRate [Mbps] is computed as the maximum data rate summed over all the carriers in the frequency range for any signaled band combination and feature set consistent with the configured servings cells, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i).

For a j-th serving cell, if higher layer parameter processingType2Enabled of PUSCH-ServingCellConfig is configured for the serving cell and set to enable, or if at least one IMCS > W for a PUSCH, where W = 28 for MCS tables 5.1.3.1-1 and 5.1.3.1-3, and W = 27 for MCS tables 5.1.3.1-2, 6.1.4.1-1, and 6.1.4.1-2, or if it is an actual repetition for PUSCH repetition Type B, the UE is not required to handle PUSCH transmissions, if the following condition is not satisfied:

m=0M-1Vj,mL×Tsμ≤DataRateCC

where

-	L is the number of symbols assigned to the PUSCH

-	M is the number of TB in the PUSCH

-	Tsμ=10-32μ∙Nsymbslot where  is the numerology of the PUSCH

-	for the m-th TB, Vj,m=C'∙AC

-	A is the number of bits in the transport block as defined in Clause 6.2.1 [5, TS 38.212]

-	C is the total number of code blocks for the transport block defined in Clause 5.2.2 [5, TS 38.212]

-	C' is the number of scheduled code blocks for the transport block as defined in Clause 5.4.2.1 [5, TS 38.212]

-	DataRateCC [Mbps] is computed as the maximum data rate for a carrier in the frequency band of the serving cell for any signaled band combination and feature set consistent with the serving cell, where the data rate value is given by the formula in Clause 4.1.2 in [13, TS 38.306], including the scaling factor f(i)

-	each actual repetition for PUSCH repetition type B is treated as one PUSCH.

#### 6.1.4.1	Modulation order and target code rate determination

For a PUSCH scheduled by RAR UL grant or

for a PUSCH scheduled by a fallbackRAR UL grant or

for a MsgA PUSCH transmission, or

for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, SP-CSI-RNTI, or

for a PUSCH with configured grant using CS-RNTI, and

if transform precoding is disabled for this PUSCH transmission according to Clause 6.1.3

-	if mcs-TableForDCI-Format0-2-r16 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel;

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableForDCI-Format0-2-r16  in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-Table in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in configuredGrantConfig is set to 'qam256',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-Table in configuredGrantConfig is set to 'qam64LowSE',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 5.1.3.1-3 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif for a MsgA PUSCH transmission,

-	the UE shall use higher layer parameter msgA-MCS for IMCS and Table 5.1.3.1-1 to determine the Target code rate (R) used in the physical uplink shared channel.

-	else

-	the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

else

-	if mcs-TableTransformPrecoderForDCI-Format0-2-r16 in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoderForDCI-Format0-2-r16 in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with DCI format 0_2 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in pusch-Config is set to 'qam256', and PUSCH is scheduled by a PDCCH with DCI format 0_1 with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 5.1.3.1.-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is not configured with MCS-C-RNTI, mcs-TableTransformPrecoder in pusch-Config is set to 'qam64LowSE', and the PUSCH is scheduled by a PDCCH with a DCI format other than DCI format 0_2 in a UE-specific search space with CRC scrambled by C-RNTI or SP-CSI-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif the UE is configured with MCS-C-RNTI, and the PUSCH is scheduled by a PDCCH with CRC scrambled by MCS-C-RNTI,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam256',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif mcs-TableTransformPrecoder in configuredGrantConfig is set to 'qam64LowSE',

-	if PUSCH is scheduled by a PDCCH with CRC scrambled by CS-RNTI or

-	if PUSCH is transmitted with configured grant,

-	the UE shall use IMCS and Table 6.1.4.1-2 to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

-	elseif for a MsgA PUSCH transmission,

-	the UE shall use higher layer parameter MsgA-MCS for IMCS and Table 6.1.4.1-1 to determine the Target code rate (R) used in the physical uplink shared channel.

-	the UE shall use q=2 for determining modulation order Qm in Table 6.1.4.1-1.

-	else

-	the UE shall use IMCS and Table 6.1.4.1-1to determine the modulation order (Qm) and Target code rate (R) used in the physical uplink shared channel.

end

For Table 6.1.4.1-1 and Table 6.1.4.1-2, if higher layer parameter tp-pi2BPSK is configured, q = 1 otherwise q=2.

Table 6.1.4.1-1: MCS index table for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 240/ q | 0.2344 |
| 1 | q | 314/ q | 0.3066 |
| 2 | 2 | 193 | 0.3770 |
| 3 | 2 | 251 | 0.4902 |
| 4 | 2 | 308 | 0.6016 |
| 5 | 2 | 379 | 0.7402 |
| 6 | 2 | 449 | 0.8770 |
| 7 | 2 | 526 | 1.0273 |
| 8 | 2 | 602 | 1.1758 |
| 9 | 2 | 679 | 1.3262 |
| 10 | 4 | 340 | 1.3281 |
| 11 | 4 | 378 | 1.4766 |
| 12 | 4 | 434 | 1.6953 |
| 13 | 4 | 490 | 1.9141 |
| 14 | 4 | 553 | 2.1602 |
| 15 | 4 | 616 | 2.4063 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 466 | 2.7305 |
| 18 | 6 | 517 | 3.0293 |
| 19 | 6 | 567 | 3.3223 |
| 20 | 6 | 616 | 3.6094 |
| 21 | 6 | 666 | 3.9023 |
| 22 | 6 | 719 | 4.2129 |
| 23 | 6 | 772 | 4.5234 |
| 24 | 6 | 822 | 4.8164 |
| 25 | 6 | 873 | 5.1152 |
| 26 | 6 | 910 | 5.3320 |
| 27 | 6 | 948 | 5.5547 |
| 28 | q | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

Table 6.1.4.1-2: MCS index table 2 for PUSCH with transform precoding and 64QAM

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x 1024 | Spectral efficiency |
| --- | --- | --- | --- |
| 0 | q | 60/q | 0.0586 |
| 1 | q | 80/q | 0.0781 |
| 2 | q | 100/q | 0.0977 |
| 3 | q | 128/q | 0.1250 |
| 4 | q | 156/q | 0.1523 |
| 5 | q | 198/q | 0.1934 |
| 6 | 2 | 120 | 0.2344 |
| 7 | 2 | 157 | 0.3066 |
| 8 | 2 | 193 | 0.3770 |
| 9 | 2 | 251 | 0.4902 |
| 10 | 2 | 308 | 0.6016 |
| 11 | 2 | 379 | 0.7402 |
| 12 | 2 | 449 | 0.8770 |
| 13 | 2 | 526 | 1.0273 |
| 14 | 2 | 602 | 1.1758 |
| 15 | 2 | 679 | 1.3262 |
| 16 | 4 | 378 | 1.4766 |
| 17 | 4 | 434 | 1.6953 |
| 18 | 4 | 490 | 1.9141 |
| 19 | 4 | 553 | 2.1602 |
| 20 | 4 | 616 | 2.4063 |
| 21 | 4 | 658 | 2.5703 |
| 22 | 4 | 699 | 2.7305 |
| 23 | 4 | 772 | 3.0156 |
| 24 | 6 | 567 | 3.3223 |
| 25 | 6 | 616 | 3.6094 |
| 26 | 6 | 666 | 3.9023 |
| 27 | 6 | 772 | 4.5234 |
| 28 | q | reserved |  |
| 29 | 2 | reserved |  |
| 30 | 4 | reserved |  |
| 31 | 6 | reserved |  |

#### 6.1.4.2	Transport block size determination

For a PUSCH scheduled by RAR UL grant or

for a PUSCH scheduled by fallbackRAR UL grant or

for a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, or

for a PUSCH scheduled by a DCI format 0_1 or DCI format 0_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or

for a PUSCH transmission with configured grant, or

for a MsgA PUSCH transmission,

if

-	and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is disabled and a table other than Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot:

-	A UE first determines the number of REs allocated for PUSCH within a PRB  by

-	, where is the number of subcarriers in the frequency domain in a physical resource block,  Nsymbslotis the number of symbols L of the PUSCH allocation according to Clause 6.1.2.1 for scheduled PUSCH or Clause 6.1.2.3 for configured PUSCH,  is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PUSCH with a configured grant in Clause 6.1.2.3 or as indicated by DCI format 0_1 or DCI format 0_2 or as described for DCI format 0_0 in Clause 6.2.2, and  is the overhead configured by higher layer parameter xOverhead in PUSCH-ServingCellConfig. If the  is not configured (a value from 6, 12, or 18), the  is assumed to be 0. For Msg3 or MsgA PUSCH transmission the  is always set to 0. In case of PUSCH repetition Type B,  is determined assuming a nominal repetition with the duration of L symbols without segmentation.

-	A UE determines the total number of REs allocated for PUSCH  by where  is the total number of allocated PRBs for the UE.

-	Next, proceed with steps 2-4 as defined in Clause 5.1.3.2

-	For a PUSCH scheduled by fallbackRAR UL grant, UE assumes the TB size determined by the UL grant in the fallback RAR shall be the same as the TB size used in the corresponding MsgA PUSCH transmission.

else if

-	 and transform precoding is disabled and Table 5.1.3.1-2 is used, or

-	 and transform precoding is enabled,

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant,

-	the TBS shall be determined from configuredGrantConfig for a configured grant Type 1 PUSCH.

-	the TBS shall be determined from the most recent PDCCH scheduling a configured grant Type 2 PUSCH.

else

-	the TBS is assumed to be as determined from the DCI transported in the latest PDCCH for the same transport block using . If there is no PDCCH for the same transport block using , and if the initial PUSCH for the same transport block is transmitted with configured grant,

-	the TBS shall be determined from configuredGrantConfig for a configured grant Type 1 PUSCH.

-	the TBS shall be determined from the most recent PDCCH scheduling a configured grant Type 2 PUSCH.

### 6.1.5	Code block group based PUSCH transmission

#### 6.1.5.1	UE procedure for grouping of code blocks to code block groups

If a UE is configured to transmit code block group (CBG) based transmissions by receiving the higher layer parameter codeBlockGroupTransmission in PUSCH-ServingCellConfig, the UE shall determine the number of CBGs for a PUSCH transmission as

,

where N is the maximum number of CBGs per transport block as configured by maxCodeBlockGroupsPerTransportBlock in PUSCH-ServingCellConfig, and C is the number of code blocks in the PUSCH according to the procedure defined in Clause 6.2.3 of [5, TS 38.212].

Define , , and .

If , CBG m, , consists of code blocks with indices . CBG m, , consists of code blocks with indices .

#### 6.1.5.2	UE procedure for transmitting code block group based transmissions

If a UE is configured to transmit code block group based transmissions by receiving the higher layer parameter codeBlockGroupTransmission in PUSCH-ServingCellConfig,

-	For an initial transmission of a TB as indicated by the New Data Indicator field of the scheduling DCI, the UE may expect that the CBGTI field indicates all the CBGs of the TB are to be transmitted, and the UE shall include all the code block groups of the TB.

-	For a retransmission of a TB as indicated by the New Data Indicator field of the scheduling DCI, the UE shall include only the CBGs indicated by the CBGTI field of the scheduling DCI.

A bit value of 0' in the CBGTI field indicates that the corresponding CBG is not to be transmitted and 1' indicates that it is to be transmitted. The order of CBGTI field bits is such that the CBGs are mapped in order from CBG#0 onwards starting from the MSB.

### 6.1.6	Uplink switching

The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching-r16. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod-r16:

-	If a UE indicated a capability for uplink switching with uplinkTxSwitchRequested-r16 for a band combination, and if it is for that band combination

-	Configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), or

-	Configured with uplink carrier aggregation, or

-	Configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink.

the conditions under which the switching gap may be present and the location of the switching gap are defined for each of the cases in clauses 6.1.6.1, 6.1.6.2, and 6.1.6.3 respectively.

If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213].

The UE does not expect to perform more than one uplink switching in a slot with µUL = max(µUL,carrier1, µUL,carrier2), where the µUL,carrier1 corresponds to the subcarrier spacing of the uplink transmitted before the switching gap and the µUL,carrier2 corresponds to the subcarrier spacing of the uplink transmitted after the switching gap.

#### 6.1.6.1	Uplink switching for EN-DC

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch-r16 for a band combination, and if it is for that band combination configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), if the UE is configured with uplink switching with parameter uplinkTxSwitching-r16,

-	for the UE configured with switchedUL by the parameter uplinkTxSwitchingOption-r16, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	when the UE is to transmit an NR uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	when the UE is to transmit an E-UTRA uplink that takes place after an NR uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	the UE is not expected to transmit simultaneously on the NR uplink and the E-UTRA uplink. If the UE is scheduled or configured to transmit any NR uplink transmission overlapping with an E-UTRA uplink transmission, the NR uplink transmission is dropped,

-	for the UE configured with dualUL by the parameter uplinkTxSwitchingOption-r16, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	when the UE is to transmit an NR two-port uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	when the UE is to transmit an E-UTRA uplink that takes place after an NR two-port uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	the UE is not expected to transmit simultaneously a two- port transmission on the NR uplink and the E-UTRA uplink.

-	in all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

-	when the UE is configured with tdm-PatternConfig-r15 or by tdm-PatternConfig-r16

-	for the E-UTRA subframes designated as uplink by the configuration, the UE assumes the operation state in which one-port E-UTRA uplink can be transmitted.

-	for the E-UTRA subframes other than the ones designated as uplink by the configuration, the UE assumes the operation state in which two-port NR uplink can be transmitted.

#### 6.1.6.2	Uplink switching for carrier aggregation

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch-r16 for a band combination, and if it is for that band combination configured with uplink carrier aggregation:

-	If the UE is configured with uplink switching with parameter uplinkTxSwitching-r16, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 1-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	When the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	For the UE configured with switchedUL by the parameter uplinkTxSwitchingOption-r16, when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier, then the UE is not expected to  transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	For the UE configured with dualUL by the parameter uplinkTxSwitchingOption-r16, when the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on the same uplink carrier and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	For the UE configured with dualUL by the parameter uplinkTxSwitchingOption-r16, when the UE is to transmit a 1-port transmission on one uplink carrier and if the preceding uplink transmission was a 1-port transmission on another uplink carrier and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers.

-	The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier, and any transmission on another uplink carrier.

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

#### 6.1.6.3	Uplink switching for supplementary uplink

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch-r16 for a band combination, and if it is for that band combination configured in a serving cell with two uplink carriers with higher layer parameter supplementaryUplink:

-	If the UE is configured with uplink switching with parameter uplinkTxSwitching-r16,

-	If the UE is to transmit any uplink channel or signal on a different uplink from the preceding transmission occasion based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s), then the UE assumes that an uplink switching is triggered in a duration of switching gap NTx1-Tx2, where T0 is the start time of the first symbol of the transmission occasion of the uplink channel or signal and Toffset is the preparation procedure time of the transmission occasion of the uplink channel or signal given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213], respectively. During the switching gap NTx1-Tx2, the UE is not expected to transmit on any of the two uplinks.

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

## 6.2	UE reference signal (RS) procedure

### 6.2.1	UE sounding procedure

The UE may be configured with one or more Sounding Reference Signal (SRS) resource sets as configured by the higher layer parameter SRS-ResourceSet or SRS-PosResourceSet-r16. For each SRS resource set configured by SRS-ResourceSet, a UE may be configured with SRS resources (higher layer parameter SRS-Resource), where the maximum value of K is indicated by UE capability [13, 38.306]. When SRS is configured with the higher layer parameter SRS-PosResourceSet-r16, a UE may be configured with SRS resources (higher layer parameter SRS-PosResource-r16), where the maximum value of K is 16. The SRS resource set applicability is configured by the higher layer parameter usage in SRS-ResourceSet. When the higher layer parameter usage is set to 'beamManagement', only one SRS resource in each of multiple SRS sets may be transmitted at a given time instant, but the SRS resources in different SRS resource sets with the same time domain behaviour in the same BWP may be transmitted simultaneously.

For aperiodic SRS at least one state of the DCI field is used to select at least one out of the configured SRS resource set(s).

The following SRS parameters are semi-statically configurable by higher layer parameter SRS-Resource or SRS-PosResource-r16.

-	srs-ResourceId or SRS-PosResourceId-r16 determines SRS resource configuration identity.

-	Number of SRS ports as defined by the higher layer parameter nrofSRS-Ports and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, nrofSRS-Ports is 1.

-	Time domain behaviour of SRS resource configuration as indicated by the higher layer parameter resourceType or resourceType-r16, which may be periodic, semi-persistent, aperiodic SRS transmission as defined in Clause 6.4.1.4 of [4, TS 38.211].

-	Slot level periodicity and slot level offset as defined by the higher layer parameters periodicityAndOffset-p or periodicityAndOffset-sp for an SRS resource of type periodic or semi-persistent, which is configured by SRS-Resource, and periodicityAndOffset-p-r16 or periodicityAndOffset-sp-r16 for an SRS resource of type periodic or semi-persistent, which is configured by SRS-PosResource-r16. The UE is not expected to be configured with SRS resources in the same SRS resource set SRS-ResourceSet or SRS-PosResourceSet-r16 with different slot level periodicities. For an SRS-ResourceSet configured with higher layer parameter resourceType set to 'aperiodic', a slot level offset is defined by the higher layer parameter slotOffset. For an SRS-PosResourceSet-r16 configured with higher layer parameter resourceType-r16 set to 'aperiodic', the slot level offset is defined by the higher layer parameter slotOffset-r16 for each SRS resource.

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping or resourceMapping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, then R is equal to the number of OFDM symbols in the SRS resource.

-	SRS bandwidth and , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then= 0.

-	Frequency hopping bandwidth, , as defined by the higher layer parameter freqHopping or freqHopping-r16 and described in Clause 6.4.1.4 of [4, TS 38.211]. If not configured, then = 0.

-	Defining frequency domain position and configurable shift, as defined by the higher layer parameters freqDomainPosition and freqDomainShift or freqDomainShift-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211]. If freqDomainPosition is not configured, freqDomainPosition is zero.

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2 or cyclicShift-n4 for transmission comb value 2 or 4for an SRS configured by SRS-Resource respectively, and defined by the higher layer parameter cyclicShift-n2-r16, cyclicShift-n4-r16, or cyclicShift-n8-r16 for transmission comb value 2, 4 or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb value as defined by the higher layer parameter transmissionComb described in Clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb offset as defined by the higher layer parameter combOffset-n2 or combOffset-n4 for transmission comb value 2 or 4for an SRS configured by SRS-Resource respectively, and defined by the higher layer parameter combOffset-n2-r16, combOffset-n4-r16, or combOffset-n8-r16 for transmission comb value 2, 4, or 8 for an SRS configured by SRS-PosResource-r16, respectively, and described in Clause 6.4.1.4 of [4, TS 38.211].

-	SRS sequence ID as defined by the higher layer parameter sequenceId or sequenceId-r16 in Clause 6.4.1.4 of [4].

-	The configuration of the spatial relation between a reference RS and the target SRS, where the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16, if configured, contains the ID of the reference RS. The reference RS may be an SS/PBCH block, CSI-RS configured on serving cell indicated by higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise, or an SRS configured on uplink BWP indicated by the higher layer parameter uplinkBWP or uplinkBWP-r16, and serving cell indicated by the higher layer parameter servingCellId if present, same serving cell as the target SRS otherwise. When an SRS is configured by the higher layer parameter SRS-PosResourceSet-r16 the reference RS may also be a DL PRS configured on a serving cell, an SS/PBCH block or a DL PRS of a non-serving cell indicated by a higher layer parameter.

The UE may be configured by the higher layer parameter resourceMapping in SRS-Resource with an SRS resource occupying  adjacent OFDM symbols at any symbol location within the slot, where all antenna ports of the SRS resources are mapped to each symbol of the resource. When the SRS is configured with the higher layer parameter SRS-PosResourceSet-r16 the higher layer parameter resourceMapping in SRS-PosResource-r16 with an SRS resource occupying NS∈1,2,4,8,12 adjacent symbols anywhere within the slot.

If a PUSCH with a priority index 0 and SRS configured by SRS-Resource are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS.

If a PUSCH transmission with a priority index 1 or a PUCCH transmission with a priority index 1 would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s).

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource-r16 is set to 'periodic':

-	if the UE is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 containing the ID of a reference 'ssb-Index', 'ssb-IndexServing-r16', or 'ssb-IndexNcell-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 containing the ID of a reference 'srs' or 'srs-spatialRelation-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS. When the SRS is configured by the higher layer parameter SRS-PosResource-r16 and if the higher layer parameter spatialRelationInfoPos-r16 contains the ID of a reference 'dl-PRS-ResourceId-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource-r16 is set to 'semi-persistent':

-	when a UE receives an activation command, as described in clause 6.1.3.17 or 6.1.3.36 of [10, TS 38.321], for an SRS resource, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission corresponding to the configured SRS resource set shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH. The activation command also contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the activated SRS resource set. When the SRS is configured with the higher layer parameter SRS-ResourceSet, each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the activation command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the activation command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the SRS is configured with the higher layer parameter SRS-PosResourceSet-r16, each ID in the list of reference signal IDs may refer to a reference SS/PBCH block on a serving or non-serving cell indicated by PCI field in the activation command, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the activation command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the activation command if present, same serving cell and bandwidth part as the SRS resource set otherwise, or DL PRS of a serving or non-serving cell indicated by a higher layer parameter.

-	if an SRS resource in the activated resource set is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16, the UE shall assume that the ID of the reference signal in the activation command overrides the one configured in spatialRelationInfo or spatialRelationInfoPos-r16.

-	when a UE receives a deactivation command [10, TS 38.321] for an activated SRS resource set, and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the deactivation command, the corresponding actions in [10, TS 38.321] and UE assumption on cessation of SRS transmission corresponding to the deactivated SRS resource set shall apply starting from the first slot that is after slot n+3Nslotsubframe,µ where  is the SCS configuration for the PUCCH.

-	if the UE is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 containing the ID of a reference 'ssb-Index', 'ssb-IndexServing-r16', or 'ssb-IndexNcell-r16' the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 contains the ID of a reference 'srs' or 'srs-SpatialRelation-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS. When the SRS is configured by the higher layer parameter SRS-PosResourceSet and if the higher layer parameter spatialRelationInfoPos-r16 contains the ID of a reference 'dl-PRS-ResourceId-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

If the UE has an active semi-persistent SRS resource configuration and has not received a deactivation command, the semi-persistent SRS configuration is considered to be active in the UL BWP which is active, otherwise it is considered suspended.

For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource-r16 is set to 'aperiodic':

-	the UE receives a configuration of SRS resource sets,

-	the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). For SRS in a resource set with usage set to 'codebook' or 'antennaSwitching', the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2  symbols and an additional time duration  Tswitch. Otherwise, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 +14 symbols and an additional time duration Tswitch. The minimal time interval unit of OFDM symbol is counted based on the minimum subcarrier spacing given by min(µPDCCH, µUL) where µUL is given by min(µUL,carrier1, µUL,carrier2, µSRS) when the UE is configured with dualUL by the parameter uplinkTxSwitchingOption-r16 for uplink carrier aggregation, and by µSRS otherwise. µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively.

-	Tswitch, µUL,carrier1 and µUL,carrier2 are defined in clause 6.4.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and except when SRS is configured with the higher layer parameter SRS-PosResource-r16, the UE transmits aperiodic SRS in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell,  Ks =, otherwise, and where

-	k is configured via higher layer parameter slotOffset for each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	If the UE receives the DCI triggering aperiodic SRS in slot n and when SRS is configured with the higher layer parameter SRS-PosResource-r16, the UE transmits every aperiodic SRS resource in each of the triggered SRS resource set(s) in slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, Ks =, otherwise, and where

-	k is configured via higher layer parameter slotOffset for each aperiodic SRS resource in each triggered SRS resources set and is based on the subcarrier spacing of the triggered SRS transmission, µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively;

-	Nslot, offset, PDCCHCA and μoffset,PDCCH are the  Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH, Nslot,offset,SRSCA and μoffset,SRS are the  and the , respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the SRS, as defined in [4, TS 38.211] clause 4.5.

-	if the UE is configured with the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 containing the ID of a reference 'ssb-Index', 'ssb-IndexServing-r16' or 'ssb-IndexNcell-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference SS/PBCH block, if the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 contains the ID of a reference 'csi-RS-Index' or 'csi-RS-IndexServing-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference periodic CSI-RS or of the reference semi-persistent CSI-RS, or of the latest reference aperiodic CSI-RS. If the higher layer parameter spatialRelationInfo or spatialRelationInfoPos-r16 contains the ID of a reference 'srs' or 'srs-SpatialRelation-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the transmission of the reference periodic SRS or of the reference semi-persistent SRS or of the reference aperiodic SRS. When the SRS is configured by the higher layer parameter SRS-PosResourceSet-r16 and if the higher layer parameter spatialRelationInfoPos-r16 contains the ID of a reference 'dl-PRS-ResourceId-r16', the UE shall transmit the target SRS resource with the same spatial domain transmission filter used for the reception of the reference DL PRS.

-	when a UE receives an spatial relation update command, as described in clause 6.1.3.26 of [10, TS 38.321], for an SRS resource configured with the higher layer parameter SRS-Resource, and when the HARQ-ACK corresponding to the PDSCH carrying the update command is transmitted in slot n, the corresponding actions in [10, TS 38.321] and the UE assumptions on updating spatial relation for the SRS resource shall be applied for SRS transmission starting from the first slot that is after slot n+3Nslotsubframe,µ.  The update command contains spatial relation assumptions provided by a list of references to reference signal IDs, one per element of the updated SRS resource set. Each ID in the list refers to a reference SS/PBCH block, NZP CSI-RS resource configured on serving cell indicated by Resource Serving Cell ID field in the update command if present, same serving cell as the SRS resource set otherwise, or SRS resource configured on serving cell and uplink bandwidth part indicated by Resource Serving Cell ID field and Resource BWP ID field in the update command if present, same serving cell and bandwidth part as the SRS resource set otherwise. When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', the UE shall not expect to be configured with different spatial relations for SRS resources in the same SRS resource set.

The UE is not expected to be configured with different time domain behavior for SRS resources in the same SRS resource set. The UE is also not expected to be configured with different time domain behavior between SRS resource and associated SRS resources set.

For operation in the same carrier, the UE is not expected to be configured on overlapping symbols with a SRS resource configured by the higher layer parameter SRS-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as 'periodic'.

For operation in the same carrier, the UE is not expected to be triggered to transmit SRS on overlapping symbols with a SRS resource configured by the higher layer parameter SRS-PosResource-r16 and a SRS resource configured by the higher layer parameter SRS-Resource with resourceType of both SRS resources as 'semi-persistent' or 'aperiodic'.

For operations in the same carrier, the UE is not expected to be configured on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource-r16 with resourceType of the SRS resources as 'periodic'.

For operations in the same carrier, the UE is not expected to be triggered to transmit SRS on overlapping symbols with more than one SRS resources configured by the higher layer parameter SRS-PosResource-r16 with resourceType of the SRS resources as 'semi-persistent' or 'aperiodic'.

For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource-r16 on different CCs, subject to UE's capability

For intra-band and inter-band CA operations, a UE can simultaneously transmit more than one SRS resource configured by SRS-PosResource-r16 and SRS-Resource on different CCs, subject to UE's capability.

The SRS request field [5, TS38.212] in DCI format 0_1, 1_1, 0_2 (if SRS request field is present), 1_2 (if SRS request field is present) indicates the triggered SRS resource set given in Table 7.3.1.1.2-24 of [5, TS 38.212]. The 2-bit SRS request field [5, TS38.212] in DCI format 2_3 indicates the triggered SRS resource set given in Clause 7.3 of [5, TS 38.212] if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB', or indicates the SRS transmission on a set of serving cells configured by higher layers if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA'.

For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent and periodic SRS are configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s).

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PUCCH/PUSCH transmissions are not supported by UE, the UE is not expected to be configured with SRS from a carrier and PUSCH/UL DM-RS/UL PT-RS/PUCCH formats from a different carrier in the same symbol.

In case of intra-band carrier aggregation or in inter-band CA band combination if simultaneous SRS and PRACH transmissions are not supported by UE, the UE shall not transmit simultaneously SRS resource(s) from a carrier and PRACH from a different carrier.

In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted.

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set to 'antennaSwitching', and a guard period of Y symbols is configured according to Clause 6.2.1.2, the UE shall use the same priority rules as defined above during the guard period as if SRS was configured.

When a spatialRelationInfo is activated/updated for a semi-persistent or aperiodic SRS resource configured by the higher layer parameter SRS-Resource by a MAC CE for a set of CCs/BWPs, where the applicable list of CCs is indicated by higher layer parameter simultaneousSpatial-UpdatedList-r16 or simultaneousSpatial-UpdatedListSecond-r16,, the spatialRelationInfo is applied for the semi-persistent or aperiodic SRS resource(s) with the same SRS resource ID for all the BWPs in the indicated CCs.

When the higher layer parameter enableDefaultBeamPL-ForSRS-r16 is set 'enabled', and if the higher layer parameter spatialRelationInfo for the SRS resource, except for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'beamManagement' or for the SRS resource with the higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook' with configuration of associatedCSI-RS or for the SRS resource configured by the higher layer parameter SRS-PosResourceSet-r16, is not configured in FR2 and if the UE is not configured with higher layer parameter(s) pathlossReferenceRS, and if the UE is not configured with different values of CORESETPoolIndex in ControlResourceSets, and is not provided at least one TCI codepoint mapped with two TCI states, the UE shall transmit the target SRS resource in an active UL BWP of a CC,

-	according to the spatial relation, if applicable, with a reference to the RS with 'QCL-TypeD' corresponding to the QCL assumption of the CORESET with the lowest controlResourceSetId in the active DL BWP in the CC.

-	according to the spatial relation, if applicable, with a reference to the RS with 'QCL-TypeD' in the activated TCI state with the lowest ID applicable to PDSCH in the active DL BWP of the CC if the UE is not configured with any CORESET in the active DL BWP of the CC

#### 6.2.1.1	UE SRS frequency hopping procedure

For a given SRS resource, the UE is configured with repetition factor R∈{1,2,4} by higher layer parameter resourceMapping in SRS-Resource where R≤Ns. When frequency hopping within an SRS resource in each slot is not configured (R=Ns), each of the antenna ports of the SRS resource in each slot is mapped in all the  symbols to the same set of subcarriers in the same set of PRBs. When frequency hopping within an SRS resource in each slot is configured without repetition (R=1), according to the SRS hopping parameters , and defined in Clause 6.4.1.4 of [4, TS 38.211], each of the antenna ports of the SRS resource in each slot is mapped to different sets of subcarriers in each OFDM symbol, where the same transmission comb value is assumed for different sets of subcarriers. When both frequency hopping and repetition within an SRS resource in each slot are configured (Ns=4, R=2), each of the antenna ports of the SRS resource in each slot is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols, and frequency hopping across the two pairs is according to the SRS hopping parameters , and .

A UE may be configured  adjacent symbol aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across  symbols when frequency hopping is configured with R=1. A UE may be configured  adjacent symbols aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across two pairs of R adjacent OFDM symbols, when frequency hopping is configured with R=2. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols of the resource.

A UE may be configured symbol periodic or semi-persistent SRS resource with inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location in each slot. A UE may be configured symbol periodic or semi-persistent SRS resource with intra-slot and inter-slot hopping within a bandwidth part, where the N-symbol SRS resource occupies the same symbol location(s) in each slot. For Ns=4, when frequency hopping is configured with R=2, intra-slot and inter-slot hopping is supported with each of the antenna ports of the SRS resource mapped to different sets of subcarriers across two pairs of R adjacent OFDM symbol(s) of the resource in each slot. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols of the resource in each slot. For Ns= R, when frequency hopping is configured, inter-slot frequency hopping is supported with each of the antenna ports of the SRS resource mapped to the same set of subcarriers in R adjacent OFDM symbol(s) of the resource in each slot.

#### 6.2.1.2	UE sounding procedure for DL CSI acquisition

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R):

-	For 1T2R, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of the second resource in the set is associated with a different UE antenna port than the SRS port of the first resource in the same set, or

-	For 2T4R, up to two SRS resource sets configured with a different value for the higher layer parameter resourceType in SRS-ResourceSet set, where each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of two SRS ports, and the SRS port pair of the second resource is associated with a different UE antenna port pair than the SRS port pair of the first resource, or

-	For 1T4R, zero or one SRS resource set configured with higher layer parameter resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' with four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a single SRS port, and the SRS port of each resource is associated with a different UE antenna port, and

-	For 1T4R, zero or two SRS resource sets each configured with higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic' and with a total of four SRS resources transmitted in different symbols of two different slots, and where the SRS port of each SRS resource in the given two sets is associated with a different UE antenna port. The two sets are each configured with two SRS resources, or one set is configured with one SRS resource and the other set is configured with three SRS resources. The UE shall expect that the two sets are both configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. The UE shall expect that the value of the higher layer parameter aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of the higher layer parameter slotOffset in each SRS-ResourceSet is different. Or,

-	For 1T=1R, or 2T=2R, or 4T=4R, up to two SRS resource sets each with one SRS resource, where the number of SRS ports for each resource is equal to 1, 2, or 4.

The UE is configured with a guard period of Y symbols, in which the UE does not transmit any other signal, in the case the SRS resources of a set are transmitted in the same slot. The guard period is in-between the SRS resources of the set.

The UE shall expect to be configured with the same number of SRS ports for all SRS resources in the SRS resource set(s) with higher layer parameter usage set as 'antennaSwitching'.

For 1T2R, 1T4R or 2T4R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R or 4T=4R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

The value of Y is defined by Table 6.2.1.2-1.

Table 6.2.1.2-1: The minimum guard period between two SRS resources of an SRS resource set for antenna switching

|  |  | Y [symbol] |
| --- | --- | --- |
| 0 | 15 | 1 |
| 1 | 30 | 1 |
| 2 | 60 | 1 |
| 3 | 120 | 2 |

#### 6.2.1.3	UE sounding procedure between component carriers

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI, and the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2.

If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by MIB or SIB1 on serving cell c2.

For n-th (n ≥ 1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided

-	it is no earlier than the summation of

-	the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and

-	the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of srs-SwitchingTimeNR,

-	it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time.

otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission.

In case of inter-band carrier aggregation, a UE can simultaneously transmit SRS and PUCCH/PUSCH across component carriers in different bands subject to the UE's capability.

In case of inter-band carrier aggregation, a UE can simultaneously transmit PRACH and SRS across component carriers in different bands subject to UE's capability.

#### 6.2.1.4	UE sounding procedure for positioning purposes

When the SRS is configured by the higher layer parameter SRS-PosResource-r16 and if the higher layer parameter spatialRelationInfoPos-r16 is configured, it contains the ID of the configuration fields of a reference RS according to Clause 6.3.2 of [TS 38.331]. The reference RS can be an SRS configured by the higher layer parameter SRS-Resource or SRS-PosResource-r16, CSI-RS, SS/PBCH block, or a DL PRS configured on a serving cell or a SS/PBCH block or a DL PRS configured on a non-serving cell.

The UE is not expected to transmit multiple SRS resources with different spatial relations in the same OFDM symbol.

If the UE is not configured with the higher layer parameter spatialRelationInfoPos-r16 the UE may use a fixed spatial domain transmission filter for transmissions of the SRS configured by the higher layer parameter SRS-PosResource-r16 across multiple SRS resources or it may use a different spatial domain transmission filter across multiple SRS resources.

The UE is only expected to transmit an SRS configured the by the higher layer parameter SRS-PosResource-r16 within the active UL BWP of the UE.

When the configuration of SRS is done by the higher layer parameter SRS-PosResource-r16, the UE can only be provided with a single RS source in spatialRelationInfoPos-r16 per SRS resource for positioning.

For operation on the same carrier, if an SRS configured by the higher parameter SRS-PosResource-r16 collides with a scheduled PUSCH, the SRS is dropped in the symbols where the collision occurs.

The UE does not expect to be configured with SRS-PosResource-r16 on a BWP not configured with PUSCH/PUCCH transmission.

### 6.2.2	UE DM-RS transmission procedure

The DM-RS transmission procedures for PUSCH scheduled by PDCCH with DCI format 0_1 described in this clause equally apply to PUSCH scheduled by PDCCH with DCI format 0_2, by applying the parameters of dmrs-UplinkForPUSCH-MappingTypeA-ForDCI-Format0-2-r16 and dmrs-UplinkForPUSCH-MappingTypeB-ForDCI-Format0-2-r16 instead of dmrs-UplinkForPUSCH-MappingTypeA and dmrs-UplinkForPUSCH-MappingTypeB.

When transmitted PUSCH is neither scheduled by DCI format 0_1/0_2 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedure, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and

If frequency hopping is disabled:

-	The UE shall assume dmrs-AdditionalPosition equals to 'pos2' and up to two additional DM-RS can be transmitted according to PUSCH duration, or

If frequency hopping is enabled:

-	The UE shall assume dmrs-AdditionalPosition equals to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration.

When transmitted PUSCH is scheduled by activation DCI format 0_0 with CRC scrambled by CS-RNTI, the UE shall use single symbol front-loaded DM-RS of configuration type provided by higher layer parameter dmrs-Type in configuredGrantConfig on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, and additional DM-RS with dmrs-AdditionalPosition  from configuredGrantConfig can be transmitted according to the scheduling type and the PUSCH duration as specified  in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled.

For the UE-specific reference signals generation as defined in Clause 6.4.1.1 of [4, TS 38.211], a UE can be configured by higher layers with one or two scrambling identity(s), nIDDMRS,i i = 0,1 which are the same for both PUSCH mapping Type A and Type B.

When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, or corresponding to a configured grant, or being a PUSCH for Type-2 random access procedure,

-	the UE may be configured with higher layer parameter dmrs-Type in DMRS-UplinkConfig, and the configured DM-RS configuration type is used for transmitting PUSCH in as defined in Clause 6.4.1.1 of [4, TS 38.211]. For MsgA PUSCH transmissions, dmrs-Type is type 1.

-	the UE may be configured with the maximum number of front-loaded DM-RS symbols for PUSCH by higher layer parameter maxLength in DMRS-UplinkConfig, or by higher layer parameter msgA-MaxLength in msgA-DMRS-Config,

-	if maxLength is not configured, single-symbol DM-RS can be scheduled for the UE by DCI or configured by the configured grant configuration, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3'.

-	if maxLength is configured, either single-symbol DM-RS or double symbol DM-RS can be scheduled for the UE by DCI or configured by the configured grant configuration, and the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter dmrs-AdditionalPosition, which can be 'pos0' or 'pos1'.

-	for msgA PUSCH for Type-2 random access procedure the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter msgA-DMRS-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3' for single-symbol DM-RS or 'pos0', 'pos1' for double-symbol DM-RS.

-	and, the UE shall transmit a number of additional DM-RS as specified in Table 6.4.1.1.3-3 and Table 6.4.1.1.3-4 in -Clause 6.4.1.1.3 of [4, TS 38.211].

If a UE transmitting PUSCH is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig, the UE may assume that the following configurations are not occurring simultaneously for the transmitted PUSCH

-	any DM-RS ports among 4-7 or 6-11 for DM-RS configurations type 1 and type 2, respectively are scheduled for the UE and PT-RS is transmitted from the UE.

For PUSCH scheduled by DCI format 0_1, by activation DCI format 0_1 with CRC scrambled by CS-RNTI, or configured by configured grant Type 1 configuration, the UE shall assume the DM-RS CDM groups indicated in Tables 7.3.1.1.2-6 to 7.3.1.1.2-23 of Clause 7.3.1.1 of [5, TS38.212] are not used for data transmission, where "1", "2" and "3" for the number of DM-RS CDM group(s) correspond to CDM group 0, {0,1}, {0,1,2}, respectively.

For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0 with CRC scrambled by CS-RNTI, the UE shall assume the number of DM-RS CDM groups without data is 1 which corresponds to CDM group 0 for the case of PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, the UE shall assume that the number of DM-RS CDM groups without data is 3 which corresponds to CDM group {0,1,2} for the case of PUSCH scheduled by activation DCI format 0_0 and the dmrs-Type in cg-DMRS-Configuration equal to 'type2' and the PUSCH allocation duration being more than 2 OFDM symbols, and the UE shall assume that the number of DM-RS CDM groups without data is 2 which corresponds to CDM group {0,1} for all other cases.

For MsgA PUSCH transmission, if the UE is not configured with msgA-PUSCH-DMRS-CDM-group, the UE shall assume that 2 DM-RS CDM groups are configured. Otherwise, msgA-PUSCH-DMRS-CDM-group indicates which DM-RS CDM group to use from the set of {0,1}.

For MsgA PUSCH transmission, if the UE is not configured with msgA-PUSCH-NrofPort, the UE shall assume that 4 ports are configured per DM-RS CDM groups. Otherwise, msgA-PUSCH-NrofPort with value of 0 indicates the first port per DM-RS CDM group, while a value of 1 indicates the first two ports per DM-RS CDM group.

For uplink DM-RS with PUSCH, the UE may assume the ratio of PUSCH EPRE to DM-RS EPRE ( [dB]) is given by Table 6.2.2-1 according to the number of DM-RS CDM groups without data. The DM-RS scaling factor  specified in clause 6.4.1.1.3 of [4, TS 38.211] is given by .

Table 6.2.2-1: The ratio of PUSCH EPRE to DM-RS EPRE

| Number of DM-RS CDM groups without data | DM-RS configuration type 1 | DM-RS configuration type 2 |
| --- | --- | --- |
| 1 | 0 dB | 0 dB |
| 2 | -3 dB | -3 dB |
| 3 | - | -4.77 dB |

For PUSCH repetition Type B, the DM-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition. A UE is not expected to be indicated with an antenna port configuration that is invalid for the allocated duration of any actual repetition.

### 6.2.3	UE PT-RS transmission procedure

The procedures on PT-RS transmission described in this clause as well as clauses 6.2.3.1 and 6.2.3.2 apply to a UE PUSCH transmission scheduled by DCI format 0_2 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-ForDCI-Format0-2-r16 or dmrs-UplinkForPUSCH-MappingTypeB-ForDCI-Format0-2-r16 is configured, to PUSCH transmissions scheduled by DCI format 0_0 or format 0_1 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB is configured and PUSCH transmissions corresponding to a configured grant if the higher layer parameter phaseTrackingRS in cg-DMRS-Configuration is configured. If a UE is not configured with the higher layer parameter phaseTrackingRS in the respective DMRS-UplinkConfig, the UE shall not transmit PT-RS. The PTRS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant. For PUSCH repetition Type B, the PT-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition.

#### 6.2.3.1	UE PT-RS transmission procedure when transform precoding is not enabled

When transform precoding is not enabled and if a UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig,

-	the higher layer parameters timeDensity and frequencyDensity in PTRS-UplinkConfig indicate the threshold values ptrs-MCSi, i=1,2,3 and NRB,i , i=0,1, as shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, respectively.

-	if either or both higher layer parameters timeDensity and/or frequencyDensity in PTRS-UplinkConfig are configured, the UE shall assume the PT-RS antenna ports' presence and pattern are a function of the corresponding scheduled MCS and scheduled bandwidth in a corresponding bandwidth part as shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, respectively,

-	if the higher layer parameter timeDensity is not configured, the UE shall assume LPT-RS = 1.

-	if the higher layer parameter frequencyDensity is not configured, the UE shall assume KPT-RS = 2.

-	if none of the higher layer parameters timeDensity and frequencyDensity in PTRS-UplinkConfig are configured, the UE shall assume LPT-RS = 1 and KPT-RS = 2.

Table 6.2.3.1-1: Time density of PT-RS as a function of scheduled MCS

| Scheduled MCS | Time density() |
| --- | --- |
| IMCS < ptrs-MCS1 | PT-RS is not present |
| ptrs-MCS1 IMCS < ptrs-MCS2 | 4 |
| ptrs-MCS2 IMCS < ptrs-MCS3 | 2 |
| ptrs-MCS3 IMCS < ptrs-MCS4 | 1 |

Table 6.2.3.1-2: Frequency density of PT-RS as a function of scheduled bandwidth

| Scheduled bandwidth | Frequency density () |
| --- | --- |
| NRB < NRB0 | PT-RS is not present |
| NRB0 NRB < NRB1 | 2 |
| NRB1 NRB | 4 |

The higher layer parameter PTRS-UplinkConfig provides the parameters ptrs-MCSi, i=1,2,3 and with values in 0-29 when MCS Table 5.1.3.1-1 is used and 0-28 when MCS Table 5.1.3.1-2 is used, respectively. ptrs-MCS4 is not explicitly configured by higher layers but assumed 29 when MCS Table 5.1.3.1-1 is used and 28 when MCS Table 5.1.3.1-2 is used. The higher layer parameter PTRS-UplinkConfig provides the parameters NRBi i=0,1 with values in range 1-276.

If the higher layer parameter PTRS-UplinkConfig indicates that the time density thresholds ptrs-MCSi = ptrs-MCSi+1, then the time density LPTRS of the associated row where both these thresholds appear in Table 6.2.3.1-1 is disabled. If the higher layer parameter frequencyDensity in PTRS-UplinkConfig indicates that the frequency density thresholds NRB,i = NRB,i+1, then the frequency density KPTRS of the associated row where both these thresholds appear in Table 6.2.3.1-2 is disabled.

If either or both of the parameters PT-RS time density (LPT-RS) and PT-RS frequency density (KPT-RS), shown in Table 6.2.3.1-1 and Table 6.2.3.1-2, indicates that are configured as 'PT-RS not present', the UE shall assume that PT-RS is not present.

When a UE is scheduled to transmit PUSCH with allocation duration of 2 symbols or less, and if LPT-RS is set to 2 or 4, the UE shall not transmit PT-RS. When a UE is scheduled to transmit PUSCH with allocation duration of 4 symbols or less, and if LPT-RS is set to 4, the UE shall not transmit PT-RS.

When a UE is scheduled to transmit PUSCH for retransmission, if the UE is scheduled with IMCS > V, where V = 28 for MCS Table 5.1.3.1-1 and MCS Table 5.1.3.1-3 and V = 27 for MCS Table 5.1.3.1-2, respectively, the MCS for PT-RS time-density determination is obtained from the DCI for the same transport block in the initial transmission, which is smaller than or equal to V.

The maximum number of configured PT-RS ports is given by the higher layer parameter maxNrofPorts in PTRS-UplinkConfig. The UE is not expected to be configured with a larger number of UL PT-RS ports than it has reported need for.

If a UE has reported the capability of supporting full-coherent UL transmission, the UE shall expect the number of UL PT-RS ports to be configured as one if UL-PTRS is configured.

For codebook or non-codebook based UL transmission, the association between UL PT-RS port(s) and DM-RS port(s) is signalled by PTRS-DMRS association field in DCI format 0_1 and DCI format 0_2. For a PUSCH corresponding to a configured grant Type 1 transmission, the UE may assume the association between UL PT-RS port(s) and DM-RS port(s) defined by value 0 in Table 7.3.1.1.2-25 or value "00" in Table 7.3.1.1.1.2-26 described in Clause 7.3.1 of [5, TS38.212].

For PUSCH scheduled by DCI format 0_0 or by activation DCI format 0_0, the UL PT-RS port is associated to DM-RS port 0.

For non-codebook based UL transmission, the actual number of UL PT-RS port(s) to transmit is determined based on SRI(s) in DCI format 0_1 and DCI format 0_2 or higher layer parameter sri-ResourceIndicator in rrc-ConfiguredUplinkGrant. A UE is configured with the PT-RS port index for each configured SRS resource by the higher layer parameter ptrs-PortIndex configured by SRS-Config if the UE is configured with the higher layer parameter phaseTrackingRS in DMRS-UplinkConfig. If the PT-RS port index associated with different SRIs are the same, the corresponding UL DM-RS ports are associated to the one UL PT-RS port.

For partial-coherent and non-coherent codebook-based UL transmission, the actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers which are indicated by Precoding information and number of layers field in DCI format 0_1 and DCI format 0_2 or configured by higher layer parameter precodingAndNnumberOfLayers:

-	if the UE is configured with the higher layer parameter maxNrofPorts in PTRS-UplinkConfig set to 'n2', the actual UL PT-RS port(s) and the associated transmission layer(s) are derived from indicated TPMI as:

-	PUSCH antenna port 1000 and 1002 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI share PT-RS port 1.

-	UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000 and PUSCH antenna port 1002 in indicated TPMI, and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1001 and PUSCH antenna port 1003 in indicated TPMI, where 'x' and/or 'y' are given by DCI parameter PTRS-DMRS association as shown in DCI format 0_1 and DCI format 0_2 described in Clause 7.3.1 of [5, TS38.212].

When the UE is scheduled with Qp={1,2} PT-RS port(s) in uplink and the number of scheduled layers is ,

-	If the UE is configured with higher layer parameter ptrs-Power, the PUSCH to PT-RS power ratio per layer per RE  is given by , where  is shown in the Table 6.2.3.1-3 according to the higher layer parameter ptrs-Power, the PT-RS scaling factor  specified in clause 6.4.1.2.2.1 of [4, TS 38.211] is given by and also on the Precoding Information and Number of Layers field in DCI.

-	The UE shall assume ptrs-Power in PTRS-UplinkConfig is set to state "00" in Table 6.2.3.1-3 if not configured or in case of non-codebook based PUSCH.

Table 6.2.3.1-3: Factor related to PUSCH to PT-RS power ratio per layer per RE

| UL-PTRS-power / |  | The number of PUSCH layers ( ) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 |  | 3 |  | 4 |  |  |
|  | All cases | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial and non- coherent and non-codebook based | Full coherent | Partial coherent | Non-coherent and non-codebook based |
| 00 | 0 | 3 | 3Qp-3 | 4.77 | 3Qp-3 | 6 | 3Qp | 3Qp-3 |
| 01 | 0 | 3 | 3 | 4.77 | 4.77 | 6 | 6 | 6 |
| 10 | Reserved |  |  |  |  |  |  |  |
| 11 | Reserved |  |  |  |  |  |  |  |

#### 6.2.3.2	UE PT-RS transmission procedure when transform precoding is enabled

When transform precoding is enabled and if a UE is configured with the higher layer parameter transformPrecoderEnabled in PTRS-UplinkConfig,

-	the UE shall be configured with the higher layer parameters sampleDensity and the UE shall assume the PT-RS antenna ports' presence and PT-RS group pattern are a function of the corresponding scheduled bandwidth in a corresponding bandwidth part, as shown in Table 6.2.3.2-1. The UE shall assume no PT-RS is present when the number of scheduled RBs is less than NRB0 if NRB0 > 1 or if the RNTI equals TC-RNTI.

-	and the UE may be configured PT-RS time density LPT-RS = 2 with the higher layer parameter timeDensityTransformPrecoding. Otherwise, the UE shall assume LPT-RS = 1.

-	if the higher layer parameter sampleDensity indicates that the sample density thresholds NRB,i = NRB,i+1, then the associated row where both these thresholds appear in Table 6.2.3.2-1 is disabled.

Table 6.2.3.2-1: PT-RS group pattern as a function of scheduled bandwidth

| Scheduled bandwidth | Number of PT-RS groups | Number of samples per PT-RS group |
| --- | --- | --- |
| NRB0 NRB < NRB1 | 2 | 2 |
| NRB1 NRB < NRB2 | 2 | 4 |
| NRB2 NRB < NRB3 | 4 | 2 |
| NRB3 NRB < NRB4 | 4 | 4 |
| NRB4 NRB | 8 | 4 |

When transform precoding is enabled and if a UE is configured with the higher layer parameter transformPrecoderEnabled in PTRS-UplinkConfig, the PT-RS scaling factor β' specified in Clause 6.4.1.2.2.2 of [4, TS 38.211] is determined by the scheduled modulation order as shown in table 6.2.3.2-2.

Table 6.2.3.2-2: PT-RS scaling factor (β') when transform precoding enabled.

| Scheduled modulation | PT-RS scaling factor (β') |
| --- | --- |
| π/2-BPSK | 1 |
| QPSK | 1 |
| 16QAM |  |
| 64QAM |  |
| 256QAM |  |

## 6.3	UE PUSCH frequency hopping procedure

### 6.3.1	Frequency hopping for PUSCH repetition Type A

For PUSCH repetition Type A (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingForDCI-Format0-2-r16 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, and by frequencyHopping provided in pusch-Config for PUSCH transmission scheduled by a DCI format other than 0_2, and by frequencyHopping provided in configuredGrantConfig for configured PUSCH transmission. One of two frequency hopping modes can be configured:

-	Intra-slot frequency hopping, applicable to single slot and multi-slot PUSCH transmission.

-	Inter-slot frequency hopping, applicable to multi-slot PUSCH transmission.

In case of resource allocation type 2, the UE transmits PUSCH without frequency hopping.

In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format or in a random access response UL grant is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingOffset is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211].

For a PUSCH scheduled by RAR UL grant, fallbackRAR UL grant, or by DCI format 0_0 with CRC scrambled by TC-RNTI, frequency offsets are obtained as described in clause 8.3 of [6, TS 38.213]. For a PUSCH scheduled by DCI format 0_0/0_1 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_0/0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsForDCI-Format0-2-r16 in pusch-Config.

-	When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant.

-	When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

For PUSCH based on a Type1 configured UL grant the frequency offset is provided by the higher layer parameter frequencyHoppingOffset in rrc-ConfiguredUplinkGrant.

For a MsgA PUSCH the frequency offset is provided by the higher layer parameter as described in [6, TS 38.213.

In case of intra-slot frequency hopping, the starting RB in each hop is given by:

,

where i=0 and i=1 are the first hop and the second hop respectively, and  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) or as calculated from the resource assignment for MsgA PUSCH (described in [6, TS 38.213]) and is the frequency offset in RBs between the two frequency hops. The number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot.

In case of inter-slot frequency hopping, the starting RB during slot  is given by:

,

where  is the current slot number within a radio frame, where a multi-slot PUSCH transmission can take place,  is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops.

### 6.3.2	Frequency hopping for PUSCH repetition Type B

For PUSCH repetition Type B (as determined according to procedures defined in Clause 6.1.2.1 for scheduled PUSCH, or Clause 6.1.2.3 for configured PUSCH), a UE is configured for frequency hopping by the higher layer parameter frequencyHoppingForDCI-Format0-2-r16 in pusch-Config for PUSCH transmission scheduled by DCI format 0_2, by frequencyHoppingForDCI-Format0-1-r16 provided in pusch-Config for PUSCH transmission scheduled by DCI format 0_1, and by frequencyHoppingPUSCH-RepTypeB-r16 provided in rrc-ConfiguredUplinkGrant for Type 1 configured PUSCH transmission. The frequency hopping mode for Type 2 configured PUSCH transmission follows the configuration of the activating DCI format. One of two frequency hopping modes can be configured:

-	Inter-repetition frequency hopping

-	Inter-slot frequency hopping

In case of resource allocation type 1, whether or not transform precoding is enabled for PUSCH transmission, the UE may perform PUSCH frequency hopping, if the frequency hopping field in a corresponding detected DCI format is set to 1, or if for a Type 1 PUSCH transmission with a configured grant the higher layer parameter frequencyHoppingPUSCH-RepTypeB-r16 is provided, otherwise no PUSCH frequency hopping is performed. When frequency hopping is enabled for PUSCH, the RE mapping is defined in clause 6.3.1.6 of [4, TS 38.211].

For a PUSCH scheduled by DCI format 0_1 or a PUSCH based on a Type 2 configured UL grant activated by DCI format 0_1 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetLists in pusch-Config. For a PUSCH scheduled by DCI format 0_2 or a PUSCH based on a Type 2 configured UL grant activated by DCI format 0_2 and for resource allocation type 1, frequency offsets are configured by higher layer parameter frequencyHoppingOffsetListsForDCI-Format0-2-r16 in pusch-Config.

-	When the size of the active BWP is less than 50 PRBs, one of two higher layer configured offsets is indicated in the UL grant.

-	When the size of the active BWP is equal to or greater than 50 PRBs, one of four higher layer configured offsets is indicated in the UL grant.

For PUSCH based on a Type1 configured UL grant the frequency offset is provided by the higher layer parameter frequencyHoppingOffset in rrc-ConfiguredUplinkGrant.

In case of inter-repetition frequency hopping, the starting RB for an actual repetition within the n-th nominal repetition (as defined in Clause 6.1.2.1) is given by:

,

where RBstart is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and RBoffset is the frequency offset in RBs between the two frequency hops.

In case of inter-slot frequency hopping, the starting RB during slot  follows that of inter-slot frequency hopping for PUSCH Repetition Type A in Clause 6.3.1.

## 6.4	UE PUSCH preparation procedure time

If the first uplink symbol in the PUSCH allocation for a transport block, including the DM-RS, as defined by the slot offset K2 and the start and length indicator SLIV of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the transport block.

-	N2 is based on µ of Table 6.4-1 and Table 6.4-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µDL, µUL) resulting with the largest Tproc,2, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PUSCH was transmitted and µUL corresponds to the subcarrier spacing of the uplink channel with which the PUSCH is to be transmitted, and κ is defined in clause 4.1 of [4, TS 38.211].

-	For operation with shared spectrum channel access, is calculated according to [4, TS 38.211], otherwise =0.

-	If the first symbol of the PUSCH allocation consists of DM-RS only, then d2,1 = 0, otherwise d2,1 = 1.

-	If the UE is configured with multiple active component carriers, the first uplink symbol in the PUSCH allocation further includes the effect of timing difference between component carriers as given in [11, TS 38.133].

-	If the scheduling DCI triggered a switch of BWP, d2,2 equals to the switching time as defined in [11, TS 38.133], otherwise d2,2=0.

-	If a PUSCH of a larger priority index would overlap with PUCCH of a smaller priority index, d2 for the PUSCH of a larger priority is set as reported by the UE; otherwise d2 = 0.

-	For a UE that supports capability 2 on a given cell, the processing time according to UE processing capability 2 is applied if the high layer parameter processingType2Enabled in PUSCH-ServingCellConfig is configured for the cell and set to enable,

-	If the PUSCH indicated by the DCI is overlapping with one or more PUCCH channels, then the transport block is multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213], otherwise the transport block is transmitted on the PUSCH indicated by the DCI.

-	If uplink switching gap is triggered as defined in clause 6.1.6,  equals to the switching gap duration and for the UE configured with dualUL by the parameter uplinkTxSwitchingOption-r16 for uplink carrier aggregation µUL=min(µUL,carrier1, µUL,carrier2), otherwise .

Otherwise the UE may ignore the scheduling DCI.

The value of  is used both in the case of normal and extended cyclic prefix.

Table 6.4-1: PUSCH preparation time for PUSCH timing capability 1

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Table 6.4-2: PUSCH preparation time for PUSCH timing capability 2

|  | PUSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 5 |
| 1 | 5.5 |
| 2 | 11 for frequency range 1 |

# 7	UE procedures for transmitting and receiving on a carrier with intra-cell guard bands

For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with  NRB-set,x-1    intra-cell guard bands on a carrier, each defined by start CRB and size in number of CRBs, GB s,xstart,μ  and GB s,xstart,μ , provided by higher layer parameters startCRB-r16 and nrofCRBs-r16, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. The UE does not expect that nrofCRBs-r16 is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as

RB s,xstart,μ=Ngrid,xstart,μ+0s=0GB s-1,xstart,μ+GB s-1,xsize,μotherwise

and

RB s,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1s=NRB-set,x-1GB s,xstart,μ-1otherwise

The RB set with index s consists of RBs,xsize,μ resource blocks where  RBs,xsize,μ=RB s,xend,μ-RB s,xstart,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines the CRB indices for the intra-cell guard band(s), if any, and corresponding RB set(s) according to the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. For either or both DL and UL, if the nominal intra-cell guard band and RB set pattern as specified in [8, TS 38.101-1] contains no intra-cell guard bands, the number of RB sets for the carrier is NRB-set,x=1.

For a carrier, the UE expects   N BWP,istart,μ=RB s0,xstart,μ and N BWP,isize,μ=RB s1,xend,μ-RB s0,xstart,μ+1  where 0≤s0≤s1≤NRB-set,x-1 for a BWP i configured by BWP-DownlinkCommon or BWP-DownlinkDedicated for the DL BWP, or BWP-UplinkCommon or BWP-UplinkDedicated for the UL BWP.  Within the BWP i, RB sets are numbered in increasing order from 0 to NRB-set,xBWP-1  where NRB-set,xBWP  is the number of RB sets contained in the BWP i and RB set 0 within the BWP i corresponds to RB set s0 in the carrier and RB set NRB-set,xBWP-1 within the BWP i corresponds to RB set s1 in the carrier.

When a UE is provided with nrofCRBs-r16=0 for all intra-cell guard band(s) on a carrier, the UE is indicated that no intra-cell guard-bands are configured for the carrier and expects NRB-set,x>1. For μ=0, the UE expects the number of RBs within a RB set is between 100 and 110. For μ=1, the UE expects the number of RBs within a RB set is between 50 and 55 except for at most one RB set which may contain 56 RBs.

# 8	Physical sidelink shared channel related procedures

A UE can be configured by higher layers with one or more sidelink resource pools. A sidelink resource pool can be for transmission of PSSCH, as described in Clause 8.1, or for reception of PSSCH, as described in Clause 8.3 and can be associated with either sidelink resource allocation mode 1 or sidelink resource allocation mode 2.

In the frequency domain, a sidelink resource pool consists of sl-NumSubchannel contiguous sub-channels. A sub-channel consists of sl-SubchannelSize contiguous PRBs, where sl-NumSubchannel and sl-SubchannelSize are higher layer parameters.

The set of slots that may belong to a sidelink resource pool is denoted by (t0SL,t1SL,⋯,tTmax-1SL) where

-	0≤tiSL<10240×2μ, 0≤i<Tmax,

-	the slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell or DFN 0,

-	the set includes all the slots except the following slots,

-	NS_SSB slots in which S-SS/PSBCH block (S-SSB) is configured,

-	NnonSL slots in each of which at least one of Y-th, (Y+1)-th, …, (Y+X-1)-th OFDM symbols are not semi-statically configured as UL as per the higher layer parameter tdd-UL-DL-ConfigurationCommon or sl-TDD-Configuration, where Y and X are set by the higher layer parameters sl-StartSymbol and sl-LengthSymbols, respectively.

-	The reserved slots which are determined by the following steps.

1)	the remaining slots excluding NS_SSB slots and NnonSL slots from the set of all the slots are denoted by (l0,l1,⋯,l(10240×2μ-NSSSB-NnonSL-1)) arranged in increasing order of slot index.

2)	a slot lr (0≤r<10240×2μ-NSSSB-NnonSL) belongs to the reserved slots if r=m∙(10240×2μ-NSSSB-NnonSL)Nreserved, here m=0,1,⋯,Nreserved-1 and Nreserved=10240×2μ-NSSSB-NnonSL mod Lbitmap where Lbitmap denotes the length of bitmap configured by higher layers.

-	The slots in the set are arranged in increasing order of slot index.

The UE determines the set of slots assigned to a sidelink resource pool as follows:

-	a bitmap b0,b1,…,bLbitmap-1 associated with the resource pool is used where Lbitmap the length of the bitmap is configured by higher layers.

-	a slot tkSL (0≤k<10240×2μ-NSSSB-NnonSL-Nreserved) belongs to the set if bk'=1 where k'=k mod Lbitmap.

-	The slots in the set are re-indexed such that the subscripts i of the remaining slots  t'iSL are successive {0, 1, …, T'max-1} where T'max is the number of the slots remaining in the set.

The UE determines the set of resource blocks assigned to a sidelink resource pool as follows:

-	The resource block pool consists of NPRB PRBs.

-	The sub-channel m for m=0,1,⋯,numSubchannel-1 consists of a set of nsubCHsize contiguous resource blocks with the physical resource block number nPRB=nsubCHRBstart+m∙nsubCHsize+j for j=0,1,⋯,nsubCHsize-1, where nsubCHRBstart and nsubCHsize are given by higher layer parameters sl-StartRB-Subchannel and sl-SubchannelSize, respectively

A UE is not expected to use the last NPRB mod nsubCHsize PRBs in the resource pool.

## 8.1	UE procedure for transmitting the physical sidelink shared channel

Each PSSCH transmission is associated with an PSCCH transmission.

That PSCCH transmission carries the 1st stage of the SCI associated with the PSSCH transmission; the 2nd stage of the associated SCI is carried within the resource of the PSSCH.

If the UE transmits SCI format 1-A on PSCCH according to a PSCCH resource configuration in slot n and PSCCH resource m, then for the associated PSSCH transmission in the same slot

-	one transport block is transmitted with up to two layers;

-	The number of layers (ʋ) is determined according to the "Number of DMRS port" field in the SCI

-	The set of consecutive symbols within the slot for transmission of the PSSCH is determined according to clause 8.1.2.1;

-	The set of contiguous resource blocks for transmission of the PSSCH is determined according to clause 8.1.2.2;

Transform precoding is not supported for PSSCH transmission.

Only wideband precoding is supported for PSSCH transmission.

The DM-RS antenna ports  in Clause 8.4.1.1.1 of [4, TS38.211] are determined according to the ordering of DM-RS port(s) given by Tables 8.3.1.1-1 in Clause 8.3.1.1 of [5, TS 38.212].

The UE shall set the contents of the SCI format 2-A as follows:

-	the UE shall set value of the "HARQ process number" field as indicated by higher layers.

-	the UE shall set value of the "NDI" field as indicated by higher layers.

-	the UE shall set value of the "Source ID" field as indicated by higher layers.

-	the UE shall set value of the "Destination ID" field as indicated by higher layers.

-	the UE shall set value of the "HARQ feedback enabled/disabled indicator" field as indicated by higher layers.

-	the UE shall set value of the "Cast type indicator" field as indicated by higher layers.

-	the UE shall set value of the "CSI request" field as indicated by higher layers.

The UE shall set the contents of the SCI formats 2-B as follows:

-	the UE shall set value of the "HARQ process number" field as indicated by higher layers.

-	the UE shall set value of the "NDI" field as indicated by higher layers.

-	the UE shall set value of the "Source ID" field as indicated by higher layers.

-	the UE shall set value of the "Destination ID" field as indicated by higher layers.

-	the UE shall set value of the "HARQ feedback enabled/disabled indicator" field as indicated by higher layers.

-	the UE shall set value of the "Zone ID" field as indicated by higher layers.

-	the UE shall set the "Communication range requirement" field as indicated by higher layers.

### 8.1.1	Transmission schemes

Only one transmission scheme is defined for the PSSCH and is used for all PSSCH transmissions.

PSSCH transmission is performed with up to two antenna ports, with antenna ports 1000-1001 as defined in clause 8.2.4 of [4, TS 38.211].

### 8.1.2	Resource allocation

In sidelink resource allocation mode 1:

-	for PSSCH and PSCCH transmission, dynamic grant, configured grant type 1 and configured grant type 2 are supported. The configured grant Type 2 sidelink transmission is semi-persistently scheduled by a SL grant in a valid activation DCI according to Clause 10.3 of [6, TS 38.213].

#### 8.1.2.1	Resource allocation in time domain

The UE shall transmit the PSSCH in the same slot as the associated PSCCH.

The minimum resource allocation unit in the time domain is a slot.

The UE shall transmit the PSSCH in consecutive symbols within the slot, subject to the following restrictions:

-	The UE shall not transmit PSSCH in symbols which are not configured for sidelink. A symbol is configured for sidelink, according to higher layer parameters startSLsymbols and lengthSLsymbols, where startSLsymbols is the symbol index of the first symbol of lengthSLsymbols consecutive symbols configured for sidelink.

-	Within the slot, PSSCH resource allocation starts at symbol startSLsymbols+1.

-	The UE shall not transmit PSSCH in symbols which are configured for use by PSFCH, if PSFCH is configured in this slot.

-	The UE shall not transmit PSSCH in the last symbol configured for sidelink.

-	The UE shall not transmit PSSCH in the symbol immediately preceding the symbols which are configured for use by PSFCH, if PSFCH is configured in this slot.

In sidelink resource allocation mode 1:

-	For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0.

-	For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0.

-	For sidelink dynamic grant and sidelink configured grant type 2:

-	The "Time gap" field value m of the DCI format 3_0 provides an index m + 1 into a slot offset table. That table is given by higher layer parameter timeGapFirstSidelinkTransmission and the table value at index m + 1 will be referred to as slot offset KSL.

-	The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than  TDL-TTA2+KSL×Tslot where TDL is starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value and KSL is the slot offset between the slot DCI and the first sidelink transmission scheduled by DCI, Tc is as defined in 38.211, and Tslott is the SL slot duration.

-	For sidelink configured grant type 1:

-	The slot of the first sidelink transmissions follows the higher layer configuration according to [10, TS 38.321].

#### 8.1.2.2	Resource allocation in frequency domain

The resource allocation unit in the frequency domain is the sub-channel.

The sub-channel assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI.

The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted.

If a PSSCH scheduled by a PSCCH would overlap with resources containing the PSCCH, the resources corresponding to a union of the PSCCH that scheduled the PSSCH and associated PSCCH DM-RS are not available for the PSSCH.

### 8.1.3	Modulation order, target code rate, redundancy version and transport block size determination

#### 8.1.3.1	Modulation order and target code rate determination

IMCS is given by the "Modulation and coding scheme" field in SCI format 1-A.

The MCS table is determined as follows: Table 5.1.3.1-1 is used if no additional MCS table is configured by higher layer parameter sl-MCS-Table; otherwise an MCS table is determined according to Table 8.1.3.1-1 or Table 8.1.3.1-2 and "MCS table indicator" field in SCI format 1-A.

Table 8.1.3.1-1: Mapping of one bit of MCS table indicator to MCS table

| MCS table indicator | MCS table |
| --- | --- |
| '0' | Table 5.1.3.1-1 |
| '1' | 1st table provided by higher layer parameter sl-Additional-MCS-Table |

Table 8.1.3.1-2: Mapping of two bits of MCS table indicator to MCS table

| MCS table indicator | MCS table |
| --- | --- |
| '00' | Table 5.1.3.1-1 |
| '01' | 1st table provided by higher layer parameter sl-Additional-MCS-Table |
| '10' | 2nd table provided by higher layer parameter sl-Additional-MCS-Table |
| '11' | reserved |

The UE shall use IMCS and the MCS table determined according to the previous step to determine the modulation order (Qm) and Target code rate (R) used in the physical sidelink shared channel.

#### 8.1.3.2	Transport block size determination

For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤27, or a table other than Table 5.1.3.1-2 is used and 0 ≤ IMCS ≤28, the UE shall first determine the TBS as specified below:

The UE shall first determine the number of REs (NRE) within the slot.

-	A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where

-	NscRB=12 is the number of subcarriers in a physical resource block,

-	NsymbshNsymbslot = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers,

-	NsymbPSFCH = 3 if "PSFCH overhead indication" field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3.

-	NohPRB is the overhead given by higher layer parameter sl-X-Overhead,

-	NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePattern.

Table 8.1.3.2-1: NREDMRS according to higher layer parameter sl-PSSCH-DMRS-TimePattern

| sl-PSSCH-DMRS-TimePattern | NREDMRS |
| --- | --- |
| {2} | 12 |
| {3} | 18 |
| {4} | 24 |
| {2,3} | 15 |
| {2,4} | 18 |
| {3,4} | 21 |
| {2,3,4} | 18 |

-	A UE determines the total number of REs allocated for PSSCH ()NRE) by NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2NRE= NRE'* nPRB, where

-	nPRB is the total number of allocated PRBs for the PSSCH,

-	NRESCI,1 is the total number of REs occupied by the PSCCH and PSCCH DMRS.

-	NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [5, TS 38.212], with the assumption of γ=0.

The UE determines TBS according to Steps 2), 3), and 4) in clause 5.1.3.2.

A UE is not expected to receive an SCI indicating 28≤IMCS≤31 if Table 5.1.3.1-2 is used, or 29≤IMCS≤31 otherwise.

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	optionally, the resource reservation interval, Prsvp_TX, in units of ms.

-	if the higher layer requests the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission as part of re-evaluation or pre-emption procedure, the higher layer provides a set of resources (r0,r1,r2,…) which may be subject to re-evaluation and a set of resources (r0',r1',r2',…) which may be subject to pre-emption.

-	it is up to UE implementation to determine the subset of resources as requested by higher layers before or after the slot ri'' - T3, where ri'' is the slot with the smallest slot index among (r0,r1,r2,…) and (r0',r1',r2',…) , and T3 is equal to Tproc,1SL, where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP.

The following higher layer parameters affect this procedure:

-	sl-SelectionWindowList: internal parameter T2min is set to the corresponding value from higher layer parameter sl-SelectionWindowList for the given value of prioTX.

-	sl-ThresPSSCH-RSRP-List: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 1-A and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	sl-RS-ForSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1.

-	sl-ResourceReservePeriodList-r16

-	sl-SensingWindow: internal parameter T0 is defined as the number of slots corresponding to sl-SensingWindow ms.

-	sl-TxPercentageList: internal parameter X for a given prioTX is defined as sl-TxPercentageList (prioTX) converted from percentage to ratio

-	sl-PreemptionEnable: if sl-PreemptionEnable-r16 is provided, and if it is not equal to 'enabled', internal parameter priopre is set to the higher layer provided parameter sl-PreemptionEnable

The resource reservation interval, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7.

Notation:

t0SL,t1SL,t2SL,... denotes the set of slots which can belong to a sidelink resource pool and is defined in Clause 8.

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot tySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL  , where Tproc,1SL  is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL) where T0 is defined above and Tproc,0SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which can belong to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

3)	The internal parameter Th(pi,pj)  is set to the corresponding value of RSRP threshold indicated by the i-th field in sl-ThresPSSCH-RSRP-List-r16, where i=pi+pj-1*8.

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX;

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

The UE shall report set SA to higher layers.

If a resource ri from the set (r0,r1,r2,…) is not a member of SA, then the UE shall report re-evaluation of the resource ri to higher layers.

If a resource ri' from the set (r0',r1',r2',…) is not a member of SA due to exclusion in step 6 above by comparison with the RSRP measurement for the received SCI format 1-A with an associated priority prioRX, and satisfy one of the following conditions, then the UE shall report pre-emption of the resource ri' to higher layers.

-	sl-PreemptionEnable is provided and is equal to 'enabled' and prioTX>prioRX

-	sl-PreemptionEnable is provided and is not equal to 'enabled', and prioRX<priopre and prioTX>prioRX

Table 8.1.4-1: Tproc,0SL depending on sub-carrier spacing

| μSL | Tproc,0SL [slots] |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 4 |

Table 8.1.4-2: Tproc,1SL depending on sub-carrier spacing

| μSL | Tproc,1SL [slots] |
| --- | --- |
| 0 | 3 |
| 1 | 5 |
| 2 | 9 |
| 3 | 17 |

### 8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields "Frequency resource assignment", "Time resource assignment" of the associated SCI format 1-A as described below.

"Time resource assignment" carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=t1

else

if t2-t1-1≤15

TRIV=30t2-t1-1+t1+31

else

TRIV=3031-t2+t1+62-t1

end if

end if

where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31.

The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where.

If sl-MaxNumPerReserve is 2 then

FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i

If sl-MaxNumPerReserve is 3 then

FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2

where

-	nsubCH,1start denotes the starting sub-channel index for the second resource

-	nsubCH,2start denotes the starting sub-channel index for the third resource

-	N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter sl-NumSubchannel

If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.The number of slots in one set of the time and frequency resources for transmission opportunities of PSSCH is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1.

If a set of sub-channels in slot tmSL is determined as the time and frequency resource for PSSCH transmission corresponding to the configured sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots tm+j×Prsvp_TX'SL  are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t0SL,t1SL,t2SL,... is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

### 8.1.6	Sidelink congestion control in sidelink resource allocation mode 2

If a UE is configured with higher layer parameter sl-CR-Limit and transmits PSSCH in slot n, the UE shall ensure the following limits for any priority value k;

i≥kCRi≤CRLimitk

where CRi is the CR evaluated in slot n-N for the PSSCH transmissions with "Priority" field in the SCI set to i, and CRLimitk corresponds to the high layer parameter sl-CR-Limit that is associated with the priority value k and the CBR range which includes the CBR measured in slot n-N, where N is the congestion control processing time.

The congestion control processing time N is based on µ of Table 8.1.6-1 and Table 8.1.6-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH is to be transmitted. A UE shall only apply a single processing time capability in sidelink congestion control.

Table 8.1.6-1: Congestion control processing time for processing timing capability 1

| µ | Congestion control processing time N [slots] |
| --- | --- |
| 0 | 2 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |

Table 8.1.6-2: Congestion control processing time for processing timing capability 2

| µ | Congestion control processing time N [slots] |
| --- | --- |
| 0 | 2 |
| 1 | 4 |
| 2 | 8 |
| 3 | 16 |

It is up to UE implementation how to meet the above limits, including dropping the transmissions in slot n.

### 8.1.7	UE procedure for determining the number of logical slots for a reservation period

A given resource reservation period Prsvp in milliseconds is converted to a period Prsvp' in logical slots as:

Prsvp'=N20 ms×Prsvp

where N is the number of slots that can be used for SL transmission within 20 ms of the configured UL-DL configuration.

## 8.2	UE procedure for transmitting sidelink reference signals

### 8.2.1	CSI-RS transmission procedure

A UE transmits sidelink CSI-RS within a unicast PSSCH transmission if the following conditions hold:

-	CSI reporting is enabled by higher layer parameter sl-CSI-Acquisition; and

-	the "CSI request" field in the corresponding SCI format 0-2 is set to 1.

The following parameters for CSI-RS transmission are configured via the higher layer parameter [TBD] for each CSI-RS configuration:

-	nrofPortsCSIRS-SL indicates the number of ports for SL CSI-RS.

-	firstSymbolInTimeDomainCSIRS-SL indicates the first OFDM symbol in a PRB used for SL CSI-RS

-	frequDomainAllocationCSIRS-SL indicates the frequency domain allocation for SL CSI-RS.

When the UE is configured with Qp={1,2} CSI-RS port(s) in sidelink and the number of scheduled layers is nlayerPSSCH,

-	The CSI-RS scaling factor βCSIRS specified in clause 8.4.1.5.3 of [4, TS 38.211] is given by βCSIRS=βDM-RSPSSCH∙nlayerPSSCHQp where βDM-RSPSSCH is the scaling factor for the corresponding PSSCH specified in clause 8.3.1.5 of [4, TS 38.211].

### 8.2.2	PSSCH DM-RS transmission procedure

The UE selects the DM-RS time domain pattern out of the patterns configured using the higher layer parameter TimePatternPsschDmrs for the resource pool on which the PSSCH is to be transmitted. If more than one DM-RS time domain pattern is configured, the selected pattern is indicated by the "DMRS pattern" field in the SCI format 0-1 associated with the PSSCH transmission.

If PSSCH DMRS and PSCCH are mapped to the same OFDM symbol, then this mapping within a single sub-channel is only supported if higher layer parameter sl-SubchannelSize >= 20, i.e. the sub-channel size is at least 20 PRBs.

When a sub-channel size is less than 20 PRBs and the size of PSCCH is less than the sub-channel size, a UE is not expected to choose a PSSCH DMRS pattern to be transmitted in the same OFDM symbol with PSCCH.

### 8.2.3	PT-RS transmission procedure

Transmission of PT-RS is only supported in FR2.

The UE PT-RS transmission procedure specified in clause 6.2.3.1 applies for derivation of the PT-RS parameters LPT-RS and, KPT-RS  and for determination of PT-RS presence, with the following changes:

-	timeDensity and frequencyDensity in PTRS-UplinkConfig are replaced by sl-PTRS-TimeDensity and sl-PTRS-FreqDensity in SL-PTRS-Config respectively, and SL-PTRS-Config is (pre)configured per resource pool;

-	the number of antenna ports is the same as the number of PSSCH DM-RS antenna ports and the association between a PT-RS antenna port and a PSSCH DM-RS antenna port is fixed.

## 8.3	UE procedure for receiving the physical sidelink shared channel

For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A and 2-B, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A and 2-B, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate.

A UE is required to decode neither the corresponding SCI formats 2-A and 2-B nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support.

## 8.4	UE procedure for receiving reference signals

### 8.4.1	CSI-RS reception procedure

The CSI-RS defined in Clause 8.4.1.5 of [4, TS 38.211] may be used for CSI computation.

### 8.4.2	DM-RS reception procedure for RSRP computation

#### 8.4.2.1	RSRP for resource selection in sidelink resource allocation mode 2

In sidelink resource allocation mode 2, the UE measures RSRP for resource selection as follows:

-	PSSCH-RSRP over the DM-RS resource elements for the PSSCH according to the received SCI format 1-A if higher layer parameter sl-RS-ForSensing is set to "pssch", and

-	PSCCH-RSRP over the DM-RS resource elements for the PSCCH carrying to the received SCI format 1-A if higher layer parameter sl-RS-ForSensing is set to "pscch".

### 8.4.3	PT-RS reception procedure

Reception of PT-RS is only supported in FR2.

The UE PT-RS reception procedure specified in clause 5.1.6.3 applies for derivation of the PT-RS parameters LPT-RS and KPT-RS and for determination of PT-RS presence, with the following changes:

-	timeDensity and frequencyDensity in PTRS-DownlinkConfig are replaced by sl-PTRS-TimeDensity and sl-PTRS-FreqDensity in SL-PTRS-Config respectively, and SL-PTRS-Config is (pre)configured per resource pool;

-	the number of antenna ports is the same as the number of PSSCH DMRS antenna ports and the association between a PT-RS antenna port and a PSSCH DMRS antenna port is fixed.

## 8.5	UE procedure for reporting channel state information (CSI)

### 8.5.1	Channel state information framework

CSI consists of Channel Quality Indicator (CQI) and Rank Indicator (RI). The CQI and RI are always reported together.

#### 8.5.1.1	Reporting configurations

The UE shall calculate CSI parameters (if reported) assuming the following dependencies between CSI parameters (if reported)

-	CQI shall be calculated conditioned on the reported RI

The CSI reporting can be aperiodic (using [10, TS 38.321]). Table 8.5.1.1-1 shows the supported combinations of CSI reporting configurations and CSI-RS configurations and how the CSI reporting is triggered for CSI-RS configuration. Aperiodic CSI-RS is configured and triggered/activated as described in Clause 8.5.1.2.

Table 8.5.1.1-1 Triggering/Activation of CSI reporting for the possible CSI-RS Configurations.

| CSI-RS Configuration | Aperiodic CSI Reporting |
| --- | --- |
| Aperiodic CSI-RS | Triggered by SCI. |

For CSI reporting, wideband CQI reporting is supported. A wideband CQI is reported [for each codeword] for the entire CSI reporting band.

#### 8.5.1.2	Triggering of sidelink CSI reports

The CSI-triggering UE is not allowed to trigger another aperiodic CSI report for the same UE before the last slot of the expected reception or completion of the ongoing aperiodic CSI report associated with the SCI format 2-A with the "CSI request" field set to 1, where the last slot of the expected reception of the ongoing aperiodic CSI report is given by [10, TS38.321].

An aperiodic CSI report is triggered by an SCI format 2-A with the "CSI request" field set to 1.

A UE is not expected to transmit a sidelink CSI-RS and a sidelink PT-RS which overlap.

### 8.5.2	Channel state information

#### 8.5.2.1	CSI reporting quantities

#### 8.5.2.1.1	Channel quality indicator (CQI)

The UE shall derive CQI as specified in clause 5.2.2.1, with the following changes

-	PDSCH replaced by PSSCH

-	uplink slot replaced by sidelink slot

-	downlink physical resource blocks replaced by sidelink physical resource blocks

-	Transport Block Size determination according to Clause 8.1.3.2

-	CSI reference resource according to the Clause 8.5.2.3

-	interference measurements are not supported

-	sub-band differential CQI is not supported

-	cqi-Table is determined as follows

-	cqi-Table = 'table1' if Table 5.1.3.1-1 is determined as the MCS table according to Clause 8.1.3.1 of [6, 38.214],

-	cqi-Table = 'table2' if Table 5.1.3.1-2 is determined as the MCS table according to Clause 8.1.3.1 of [6, 38.214],

-	cqi-Table = 'table3' if Table 5.1.3.1-3 is determined as the MCS table according to Clause 8.1.3.1 of [6, 38.214]

#### 8.5.2.2	Reference signal (CSI-RS)

The UE can be configured with one CSI-RS pattern as indicated by the higher layer parameters sl-CSI-RS-FreqAllocation, sl-OneAntennaPort, sl-CSI-RS-FirstSymbol in SL-CSI-RS-Config.

Parameters for which the UE shall assume non-zero transmission power for CSI-RS are configured according to clause 8.2.1.

A UE is not expected to be configured such that a CSI-RS and the corresponding PSCCH can be mapped to the same resource element.A UE is not expected to receive sidelink CSI-RS and PSSCH DMRS, nor CSI-RS and 2nd-stage SCI, on the same symbol.

Sidelink CSI-RS shall be transmitted according to [4, TS 38.211] in the resource blocks used for the PSSCH associated with the SCI format 2-A triggering a report.

#### 8.5.2.3	CSI reference resource definition

The CSI reference resource in sidelink is defined as follows:

-	In the frequency domain, the CSI reference resource is defined by the group of sidelink physical resource blocks containing the sidelink CSI-RS to which the derived CSI relates.

-	In the time domain, the CSI reference resource for a CSI reporting in sidelink slot n is defined by a single sidelink slot nCSI_ref where nCSI_ref is the same sidelink slot as the corresponding CSI request.

If configured to report CQI index and RI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index and RI index:

-	The reference resource uses the CP length and subcarrier spacing configured for the SL BWP.

-	Redundancy Version 0.

-	PSCCH occupies 2 OFDM symbols.

-	The number of PSSCH and DMRS symbols is equal to sl-LenghSymbols‒2.

-	Assume no REs allocated for sidelink CSI-RS.

-	Assume no REs allocated SCI format 2-A or SCI format 2-B.

-	Assume the same number of DM-RS symbols as the smallest one configured by the higher layer parameter sl-PSSCH-DMRS-TimePatternList.

-	Assume no REs allocated for sidelink PT-RS.

-	Assume sidelink CSI-RS RE power is the same as PSSCH RE power.

-	The PSSCH transmission scheme where the UE may assume that PSSCH transmission would be performed with up to 2 transmission layers as defined in Clause 8.3.1.4 of [4, TS 38.211]. For CQI calculation, the UE should assume that PSSCH signals on antenna ports in the set [1000,…, 1000+ν-1] for ν layers would result in signals equivalent to corresponding symbols transmitted on antenna ports [3000,…, 3000+P-1], as given by

y3000(i)⋯y3000+P-1(i)=W(i)x0(i)⋯xν-1(i)

where  is a vector of PSSCH symbols from the layer mapping defined in Clause 8.3.1.4 of [4, TS 38.211], P∈1,2 is the number of CSI-RS ports. If only one CSI-RS port is configured, W(i) is 1. Otherwise, W(i) is the identity matrix.

### 8.5.3	CSI reporting

The UE can be configured with one CSI reporting latency bound as indicated by the higher layer parameter sl-LatencyBound-CSI-Report. CSI reporting is aperiodic and is described in [10, TS 38.321].

## 8.6	UE PSSCH preparation procedure time

For sidelink dynamic grant and for SL configured grant type 2 activation, if the first sidelink symbol in the sidelink allocation for a PSSCH for a transport block and the associated PSCCH, including the DM-RS and the duplicated symbol, as defined by the slot offset KSL of the scheduling DCI for dynamic grant or the activating DCI for SL configured grant type 2, is no earlier than at symbol L, where L is defined as the next sidelink symbol with its CP starting Tproc=(N2+d2,1)(2048+144)⋅κ2-μ⋅TC after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the sidelink transmissions for dynamic grant or activating the SL configured grant type 2, then the UE shall transmit the PSSCH and the associated PSCCH.

-	N2 is based on µ of Table 8.6-1, where µ corresponds to the one of (µDL, µSL) resulting with the largest Tproc, where the µDL corresponds to the subcarrier spacing of the downlink with which the PDCCH carrying the DCI scheduling the PSSCH for dynamic grant or activating the SL configured grant type 2 was transmitted and µSL corresponds to the subcarrier spacing of the sidelink channel with which the PSSCH and the associated PSCCH are to be transmitted, and κ is defined in Clause 4.1 of [4, TS 38.211].

-	d2,1 = 1.

Otherwise the UE may ignore the scheduling DCI for dynamic grant or the activating DCI for SL configured grant type 2.

The value of Tproc is used both in the case of normal and extended cyclic prefix.

Table 8.6-1: PSSCH preparation time

|  | PSSCH preparation time N2 [symbols] |
| --- | --- |
| 0 | 10 |
| 1 | 12 |
| 2 | 23 |
| 3 | 36 |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-05 | RAN1#89 | R1-1708892 | - | - | - | Draft skeleton | 0.0.0 |
| 2017-07 | AH_1706 | R1-1712016 |  |  |  | Inclusion of agreements up to and including RAN1#AH2 | 0.0.1 |
| 2017-08 | AH_1706 | R1-1714234 |  |  |  | Inclusion of agreements up to and including RAN1#AH2 | 0.0.2 |
| 2017-08 | RAN1#90 | R1-1714596 |  |  |  | Updated editor's version | 0.0.3 |
| 2017-08 | RAN1#90 | R1-1714626 |  |  |  | Updated editor's version | 0.0.4 |
| 2017-08 | RAN1#90 | R1-1715077 |  |  |  | Endorsed version by RAN1#90 | 0.1.0 |
| 2017-08 | RAN1#90 | R1-1715324 |  |  |  | Inclusion of agreements up to and including RAN1#90 | 0.1.1 |
| 2017-08 | RAN1#90 | R1-1715331 |  |  |  | Updated editor's version | 0.1.2 |
| 2017-09 | RAN#77 | RP-172001 |  |  |  | For information to plenary | 1.0.0 |
| 2017-09 | AH_1709 | R1-1716930 |  |  |  | Inclusion of agreements up to and including RAN1#AH3 | 1.0.1 |
| 2017-10 | RAN1#90bis | R1-1718808 |  |  |  | Updated editor's version | 1.0.2 |
| 2017-10 | RAN1#90bis | R1-1718819 |  |  |  | Endorsed version by RAN1#90bis | 1.1.0 |
| 2017-10 | RAN1#90bis | R1-1719227 |  |  |  | Inclusion of agreements up to and including RAN1#90bis | 1.1.1 |
| 2017-11 | RAN1#90bis | R1-1720113 |  |  |  | Inclusion of agreements up to and including RAN1#90bis | 1.1.2 |
| 2017-11 | RAN1#90bis | R1-1720114 |  |  |  | Inclusion of agreements up to and including RAN1#90bis | 1.1.3 |
| 2017-11 | RAN1#90bis | R1-1721051 |  |  |  | Endorsed version | 1.2.0 |
| 2017-12 | RAN1#91 | R1-1721344 |  |  |  | Inclusion of agreements up to and including RAN1#91 | 1.3.0 |
| 2017-12 | RAN#78 | RP-172416 |  |  |  | Endorsed version for approval by plenary | 2.0.0 |
| 2017-12 | RAN#78 |  |  |  |  | Approved by plenary – Rel-15 spec under change control | 15.0.0 |
| 2018-03 | RAN#79 | RP-180200 | 0001 |  | F | CR capturing the Jan18 ad-hoc and RAN1#92 meeting agreements | 15.1.0 |
| 2018-06 | RAN#80 | RP-181172 | 0002 | 1 | F | CR to 38.214 capturing the RAN1#92bis and RAN1#93 meeting agreements | 15.2.0 |
| 2018-06 | RAN#80 | RP-181257 | 0003 | - | B | CR to 38.214 capturing the RAN1#92bis and RAN1#93 meeting agreements related to URLLC | 15.2.0 |
| 2018-06 | RAN#80 | RP-181172 | 0004 | - | F | CR to 38.214: maintenance according to agreed Rel 15 features | 15.2.0 |
| 2018-09 | RAN#81 | RP-181789 | 0005 | - | F | CR to 38.214 capturing the RAN1#94 meeting agreements | 15.3.0 |
| 2018-12 | RAN#82 | RP-182523 | 0006 | 2 | F | Combined CR of all essential corrections to 38.214 from RAN1#94bis and RAN1#95 | 15.4.0 |
| 2019-03 | RAN#83 | RP-190632 | 0007 | 3 | F | Correction to aperiodic CSI-RS triggering with different numerology between PDCCH and CSI-RS | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0009 | - | F | Correction on CSI-RS configuration in 38.214 | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0010 | - | F | Correction on uplink resource allocation type 1 | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0011 | - | F | Correction on determination of the resource allocation table for PUSCH with SP CSI | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0012 | - | F | Correction on PUSCH resource allocation | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0013 | - | F | Change Request for alignment of frequency domain resource allocation with 38.213 for a PUSCH transmission scheduled by a RAR UL grant | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0014 | - | F | CR on sequential PDSCH and PUSCH scheduling | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0015 | - | F | CR on out of HARQ order with multiple PDSCHs within one slot | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0016 | - | F | Correction to LBRM restriction | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0017 | - | F | CR on PDSCH beam indication | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0018 | - | F | Correction on TCI indication for multi-slot PDSCH | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0019 | - | F | Clarifications on CSI reporting on PUSCH | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0020 | - | F | QCL properties of Msg4 in CONNECTED Mode | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0022 | 1 | F | CR on dynamic grant overriding configured grant | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0023 | - | F | Correction on PUSCH with configured grant | 15.5.0 |
| 2019-03 | RAN#83 | RP-190450 | 0024 | - | F | Corrections to 38.214 | 15.5.0 |
| 2019-03 | RAN#83 | RP-190425 | 0025 | 1 | F | CR on QCL assumption for receiving PDSCH for RAR | 15.5.0 |
| 2019-06 | RAN#84 | RP-191552 | 0035 | - | F | Removal of "Correction to aperiodic CSI-RS triggering with different numerology between PDCCH and CSI-RS" | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0026 | - | F | Corrections on non-codebook based UL transmission to TS 38.214 | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0027 | - | F | CR on UE procedure for PDSCH and PUSCH | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0028 | - | F | Correction on configured scheduling PUSCH repetition with dynamic SFI | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0029 | - | F | CR on rate matching for PDSCH scheduled by DCI format 1_0 | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0030 | - | F | Clarification on CG transmission opportunities | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0031 | 5 | F | Corrections to 38.214 including alignment of terminology across specifications | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0032 | - | F | CR to 38.214 clarifying calculation of DataRate and DataRateCC | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0033 | - | F | Correction on PUSCH retransmission for a serving cell configured with two uplinks | 15.6.0 |
| 2019-06 | RAN#84 | RP-191284 | 0034 | - | F | Corrections to 38.214 regarding the MAC CE activation/deactivation timing in mixed numerology scenario | 15.6.0 |
| 2019-09 | RAN#85 | RP-191943 | 0037 | - | F | Correction on UE receiving PDSCH procedure in FR2 | 15.7.0 |
| 2019-09 | RAN#85 | RP-191943 | 0038 | - | F | Correction on slot aggregation | 15.7.0 |
| 2019-09 | RAN#85 | RP-191943 | 0039 | - | F | CR on grant-based PDSCH overlapping with SPS PDSCH | 15.7.0 |
| 2019-09 | RAN#85 | RP-191943 | 0040 | - | F | Correction on the resource mapping of PDSCH in TS 38.214 | 15.7.0 |
| 2019-09 | RAN#85 | RP-191943 | 0041 | 1 | F | Corrections to 38.214 including alignment of terminology across specifications in RAN1#98 | 15.7.0 |
| 2019-09 | RAN#85 | RP-191943 | 0042 | - | F | Clarification of PUSCH with SP-CSI overlapping with PUSCH with data | 15.7.0 |
| 2019-12 | RAN#86 | RP-192627 | 0045 | - | F | Correction on resource allocation for uplink transmission with configured grant Type 1 | 15.8.0 |
| 2019-12 | RAN#86 | RP-192627 | 0046 | - | F | Clarification to the dynamically scheduled PDSCH collision with SPS-PDSCH | 15.8.0 |
| 2019-12 | RAN#86 | RP-192627 | 0047 | - | F | Correction on rate-matching for LTE-CRS-toMatchAround | 15.8.0 |
| 2019-12 | RAN#86 | RP-192627 | 0048 | - | F | Correction on timing for MAC CE applicability in 38.214 | 15.8.0 |
| 2019-12 | RAN#86 | RP-192627 | 0049 | - | F | Corrections to 38.214 including alignment of terminology across specifications in RAN1#98bis and RAN1#99 | 15.8.0 |
| 2019-12 | RAN#86 | RP-192634 | 0043 | 1 | B | Introduction of UE behaviour for SRS measurements for CLI | 16.0.0 |
| 2019-12 | RAN#86 | RP-192635 | 0050 | - | B | Introduction of two-step RACH | 16.0.0 |
| 2019-12 | RAN#86 | RP-192636 | 0051 | - | B | Introduction of NR - U | 16.0.0 |
| 2019-12 | RAN#86 | RP-192637 | 0052 | - | B | Introduction of integrated access and backhaul for NR | 16.0.0 |
| 2019-12 | RAN#86 | RP-192638 | 0053 | - | B | Introduction of NR V2X | 16.0.0 |
| 2019-12 | RAN#86 | RP-192639 | 0054 | - | B | Introduction of NR URLLC support | 16.0.0 |
| 2019-12 | RAN#86 | RP-192641 | 0055 | - | B | Introduction of NR enhanced MIMO | 16.0.0 |
| 2019-12 | RAN#86 | RP-192642 | 0056 | - | B | Introduction of cross-slot scheduling restriction | 16.0.0 |
| 2019-12 | RAN#86 | RP-192643 | 0057 | - | B | Introduction of NR positioning support | 16.0.0 |
| 2019-12 | RAN#86 | RP-192645 | 0058 | - | B | Introduction of Cross-carrier Scheduling with Different Numerologies | 16.0.0 |
| 2019-12 | RAN#86 | RP-192646 | 0059 | - | B | Introduction of multiple LTE CRS rate matching patterns | 16.0.0 |
| 2019-12 | RAN#86 | RP-192646 | 0060 | - | B | Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336 | 16.0.0 |
| 2019-12 | RAN#86 | RP-192646 | 0061 | - | B | Behaviour for triggered with a CSI report for non-active BWP | 16.0.0 |
| 2019-12 | RAN#86 | RP-192646 | 0062 | - | B | Introduction of downgraded configurations for SRS antenna switching | 16.0.0 |
| 2019-12 | RAN#86 | RP-192646 | 0063 | - | B | Introduction of one-slot periodic TRS configuration for FR1 under a certain condition | 16.0.0 |
| 2019-12 | RAN#86 | RP-192640 | 0064 | - | B | Introduction of Industrial IoT | 16.0.0 |
| 2020-03 | RAN#87-e | RP-200185 | 0068 | - | F | Corrections on NR - U | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200187 | 0069 | - | F | Corrections on NR V2X | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200483 | 0070 | 1 | F | Corrections on Cross-carrier Scheduling with Different Numerologies | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200188 | 0071 | - | F | Corrections on NR URLLC support | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200190 | 0072 | - | F | Corrections on NR enhanced MIMO | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200189 | 0073 | - | F | Corrections on Industrial IoT | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200191 | 0074 | - | F | Corrections of cross-slot scheduling restriction and CSI/L1-RSRP measurement outside active time | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200184 | 0075 | - | F | Corrections on two-step RACH after RAN1#100-e | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200192 | 0076 | - | F | Corrections of NR positioning support | 16.1.0 |
| 2020-03 | RAN#87-e | RP-200448 | 0078 | - | A | CR on UL PTRS density selection | 16.1.0 |
| 2020-06 | RAN#88-e | RP-200683 | 0080 | - | A | CR on CSI reporting for BWP size < 24 PRBs | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200683 | 0082 | - | A | CR on 38.214 PDSCH resource mapping | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200683 | 0084 | - | A | CR to 38.214 clarification on resource and port occupation of duplicate CSI-RS resources | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200693 | 0085 | 1 | F | Corrections of cross-slot scheduling restriction | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200686 | 0086 | 1 | F | Correction of two-step RACH | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200690 | 0087 | 1 | F | Corrections on NR URLLC support | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200691 | 0088 | 1 | F | Corrections on Industrial IoT | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200694 | 0089 | 1 | F | Corrections of NR positioning support | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200687 | 0090 | 1 | F | Corrections on NR - U | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200689 | 0091 | 1 | F | Corrections on NR V2X | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200696 | 0092 | 1 | F | Corrections on Cross-carrier Scheduling with Different Numerologies | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200692 | 0093 | 1 | F | Corrections on NR enhanced MIMO | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200685 | 0094 | - | F | Correction on SRS-RSRP reception procedure for CLI | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200683 | 0097 | - | A | CR on SRS for 38.214 | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200683 | 0099 | - | A | CR on 38.214 rate-matching for PDSCH with SPS | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200683 | 0101 | - | A | Correction for SP-CSI reporting on PUSCH | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200683 | 0103 | - | A | CR on port and CSI-RS resource counting | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200705 | 0106 | - | B | Introduction of switched uplink operation | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200697 | 0107 | - | F | Correction on aperiodic CSI-RS triggering with beam switching timing of 224 and 336 and on CSI reporting | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200697 | 0108 | - | F | Correction to TBS determination when 3824<Ninfo<3825 | 16.2.0 |
| 2020-06 | RAN#88-e | RP-200697 | 0109 | - | D | Editorial corrections | 16.2.0 |
| 2020-09 | RAN#89-e | RP-201803 | 0111 | - | A | CR on Measurement Restriction for L1-RSRP | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201803 | 0113 | - | A | Correction on sounding procedure between component carriers | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201803 | 0115 | - | A | Clarification on which UE capability component indicates the number of supported simultaneous CSI calculations NCPU | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201814 | 0116 | - | F | Corrections to PDSCH PRB bundling notation (Rel-15 origin) | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201804 | 0117 | - | F | CR on 2-step RACH for 38.214 | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201813 | 0118 | - | F | Corrections on Cross-carrier Scheduling with Different Numerologies | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201809 | 0120 | - | F | Correction on SRS carrier switching | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201814 | 0121 | - | F | Correction on aperiodic CSI-RS triggering with beam switching timing of 224 and 336 | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201814 | 0122 | - | B | Introduction of flexible TRS bandwidth for BWP of 52 RBs | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201814 | 0123 | - | F | Correction on 38.214 for PUSCH with UL skipping (Note: CR was not implementable because "not based on the latest version of the specification") | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201810 | 0124 | - | F | RRM measurements when drx-onDurationTimer does not start | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201820 | 0125 | - | F | Correction on uplink Tx switching | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201807 | 0126 | 1 | F | Corrections on 5G V2X sidelink features | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201809 | 0127 | - | F | Corrections to MIMO enhancements | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201811 | 0128 | - | F | Corrections to NR positioning support | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201805 | 0129 | - | F | Corrections to NR-based access to unlicensed spectrum | 16.3.0 |
| 2020-09 | RAN#89-e | RP-201808 | 0130 | - | F | Corrections on NR URLLC support | 16.3.0 |