Annex A (informative):
Connection Diagrams

# A.1	Definition of Terms

System Simulator or SS – A device or system, that is capable of generating simulated Node B signalling and analysing UE signalling responses on one or more RF channels, in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Measurement and control of the UE Tx output power through TPC commands

2.	Measurement of Throughput

3.	Measurement of signalling timing and delays

4.	Ability to simulate UTRAN and/or E-UTRAN and/or GERAN signalling

Test System – A combination of devices brought together into a system for the purpose of making one or more measurements on a UE in accordance with the test case requirements. A test system may include one or more System Simulators if additional signalling is required for the test case. The following diagrams are all examples of Test Systems.

NOTE 1:	The above terms are logical definitions to be used to describe the test methods used in the documents TS 38.521-1, TS 38.521-2, TS 38.521-3, TS 38.523-1 and TS 38.533 in practice, real devices called 'System Simulators' may also include additional measurement capabilities or may only support those features required for the test cases they are designed to perform.

NOTE 2:	Components in the connection diagrams:
The components in the connection diagrams represent ideal components. They are intended to display the wanted signal flow. They don’t mandate real implementations.

Connection: Each connection is displayed as a one or two sided arrow, showing the intended signal flow. In some cases, for some tests, some connections shown may not be necessary (for example UL RX connection for a second cell).

Circulator: The signal, entering one port, is conducted to the adjacent port, indicated by the arrow. The attenuation among the above mentioned ports is ideally 0 and the isolation among the other ports is ideally ∞.

Splitter: a splitter has one input and 2 or more outputs. The signal at the input is equally divided to the outputs. The attenuation from input to the outputs is ideally 0 and the isolation between the outputs is ideally ∞.

Combiner: a combiner has one output and 2 or more inputs. The signals at the inputs are conducted to the output, all with the same, ideally 0 attenuation. The isolation between the inputs is ideally ∞.

Switch: contacts a sink (or source) alternatively to two or more sources (or sinks).

Fader: The fader has one input and one output. The MIMO fading channel is represented by several single faders (e.g. 8 in case of a MIMO antenna configuration 4x2) The correlation among the faders is described in TS 36.521-1 clause B.2.2. In some cases, for some tests, diagrams with fader(s) are referenced when no fading is required; in this case the fader(s) is omitted.

Attenuator: TBD

Test Equipment Part (TE): is the section of the connection diagram focused including a combination of devices to perform one or several measurements on a UE depending on the test requirements specified in 3GPP TS 38.101-1 [7], 3GPP TS 38.101-2 [8] and 3GPP TS 38.101-3 [9]. The basic TE is the system simulator to enable the connection between the gNB (and the eNB, if NSA mode) and the DUT. The number of cells, the number of streams per cell and how to combine them, channel and propagations conditions, etc. are also part of the TE. Other instruments as external spectrum analyser, interferer generators, external faders or external AWGN generators can be also considered part of the TE, as these instruments allow to measure a test requirement or to set the UE under certain conditions.

DUT Part (UE): for conducted measurement this section is focused on the number of physical antenna connectors and how to combine in the DUT. For radiated measurement this section shows the connections needed to translate the UL/DL streams to the radiated part.

GNSS System Simulator or GSS: A device or system, that is capable of generating simulated GNSS satellite transmissions in order to create the required test environment for the UE under test. It will also include the following capabilities:

1.	Control of the output power of individual satellites and the simulation of atmospheric delays.

2.	Ability to synchronize with NR timing in the SS.

# A.2	General Considerations on Connections Diagram

In order to improve the maintainability and the readability of this section and to make easy to identify the whole connection diagram to use per each test case, several considerations have been used for this section:

-	The whole connection diagram to use for a specific test has been split in Test Equipment (TE) and User Equipment (UE) parts.

-	The same connection diagram will be used for SA and NSA, where the LTE link is specified in each connection diagram (TE and UE) with a dashed line (and this part will be only used for NSA).

-	To obtain the whole connection diagram required per each test case is necessary to specify the TE part required for each measurement and the UE part will depend on the UE antenna implementation.

# A.3	Setup Diagrams

## A.3.1	Test Equipment Parts for Conducted Measurements

### A.3.1.1	Basic Transmitter/Receiver tests

Figure A.3.1.1.1: Test Equipment connection for basic single cell, RX and TX tests

Figure A.3.1.1.2: Test Equipment connection for single cell, RX and TX tests for NR UL MIMO or NR Tx diversity

Figure A.3.1.1.3: Test Equipment connection for NR CA and NR-DC, basic RX and TX tests

Figure A.3.1.1.4: Test Equipment connection for NR SUL, basic RX and TX tests

Figure A.3.1.1.5: Test Equipment connection for UL MIMO on SUL band, basic RX and TX tests

### A.3.1.2	Transmitter tests using Spectrum Analyser

Figure A.3.1.2.1: Test Equipment connection for TX-tests with additional Spectrum Analyzer

Figure A.3.1.2.2: Test Equipment connection for TX-tests for UL MIMO with additional Spectrum Analyser

Figure A.3.1.2.3: Test Equipment connection for NR CA TX-tests with additional Spectrum Analyser

Figure A.3.1.2.4: Test Equipment connection for NR SUL TX-tests with additional Spectrum Analyzer

### A.3.1.3	Transmitter tests using Spectrum Analyser and Signal Generator

Figure A.3.1.3.1: Test Equipment connection for Transmitter tests with CW Interference and spectrum analyser

Figure A.3.1.3.2: Test Equipment connection for Transmitter tests for UL MIMO with CW Interference and spectrum analyser

Figure A.3.1.3.3: Test Equipment connection for NR CA Transmitter tests with CW Interference and spectrum analyser

Figure A.3.1.3.4: Test Equipment connection for Transmitter tests for SUL with CW Interference and spectrum analyzer

### A.3.1.4	Receiver tests using Signal Generator

Figure A.3.1.4.1: Test Equipment connection for Receiver tests with Modulated Interference

Figure A.3.1.4.2: Test Equipment connection for Receiver tests with CW Interference

Figure A.3.1.4.3: Test Equipment connection for Receiver tests both Modulated and additional CW Interference signal

Figure A.3.1.4.4: Test Equipment connection for Receiver tests for UL MIMO with Modulated Interference

Figure A.3.1.4.5: Test Equipment connection for Receiver tests for UL MIMO with CW Interference

Figure A.3.1.4.6: Test Equipment connection for Receiver tests for UL MIMO with both Modulated and additional CW Interference signal

Figure A.3.1.4.7: Test Equipment connection for NR CA Receiver tests with additional Modulated Interference signal and/or CW Interference signal

Figure A.3.1.4.8: Test Equipment connection for NR SUL Receiver tests with Modulated Interference

Figure A.3.1.4.9: Test Equipment connection for NR SUL Receiver tests with CW Interference

Figure A.3.1.4.10: Test Equipment connection for NR SUL with DL CA Receiver tests with additional Modulated Interference and/or CW Interference signal

### A.3.1.5	Receiver tests using Spectrum Analyser

Figure A.3.1.5.1: Test Equipment connection for RX-tests with additional Spectrum Analyzer

Figure A.3.1.5.2: Test Equipment connection for NR CA RX-tests with additional Spectrum Analyzer

### A.3.1.6	Receiver Performance tests

Figure A.3.1.6.1: Test Equipment connection for Receiver Performance tests with antenna configuration 1x2

### A.3.1.7	Demodulation Performance and CSI reporting tests

Figure A.3.1.7.0: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 2x1

Figure A.3.1.7.1: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 2x2

Figure A.3.1.7.2: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 1x2

Figure A.3.1.7.2A: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 1x2 for nDLCA

Figure A.3.1.7.3: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 1x4

Note: LTE can be 2Rx or 4Rx and not dependent on NR #Rx

Figure A.3.1.7.4: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 2x4

Note 1: LTE can be 2Rx or 4Rx and not dependent on NR #Rx

Note 2: NR may be 2Rx on some of the CCs, in that case TE NR TX3 and TE NR TRX4 are not used

Figure A.3.1.7.5: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 4x4

Note 1: LTE can be 2Rx or 4Rx and not dependent on NR #Rx

Note: 2 NR may be 2Rx on some of the CCs, in that case TE NR TX3 and TE NR TX4 are not used

Figure A.3.1.7.6: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 2x4 for nDL CA

Note 1: LTE can be 2Rx or 4Rx and not dependent on NR #Rx

Note 2: NR may be 2Rx on some of the CCs, in that case TE NR TX3 and TE NR TX4 are not used

Figure A.3.1.7.7: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 4x4 for nDL CA

Note 1: LTE can be 2Rx or 4Rx and not dependent on NR #Rx

Note 2: NR may be 2Rx on some of the CCs, in that case TE NR TX3 and TE NR TX4 are not used

Figure A.3.1.7.8: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration 4x2

Note: LTE can be 2Rx or 4Rx and not dependent on NR #Rx

Figure A.3.1.7.9: Test Equipment connection for Demodulation Performance and CSI reporting tests with 2 carriers or 2 TRPs (2x2 or 2x4)

Note 1: LTE can be 2Rx or 4Rx and not dependent on NR #Rx

Note 2: NR may be 2Rx on some of the CCs, in that case TE NR TX3 and TE NR TX4 are not used

Figure A.3.1.7.10: Test Equipment connection for Demodulation Performance and CSI reporting tests with antenna configuration Nx2 and Nx4, N=16 or N=32

Note 1: LTE can be 2Rx or 4Rx and not dependent on NR #Rx

Note 2: NR may be 2Rx on some of the CCs, in that case TE NR TX3 and TE NR TX4 are not used

### A.3.1.8	RRM tests with more than one NR cell

The figures in this section represent connection diagrams for test cases with more than one NR cell. The parameters in the connection diagram, e.g. the number of cells n or the value of the phase rotator φi shall be defined by the test cases.

Figure A.3.1.8.1: Test Equipment connection for tests with more than one NR cell and antenna configuration 1x1

Figure A.3.1.8.2: Test Equipment connection for tests with more than one NR cell and antenna configuration 1x2

Figure A.3.1.8.2a: Test Equipment connection for tests with more than one NR cell and antenna configuration 2x2

Figure A.3.1.8.3: Test Equipment connection for tests with more than one NR cell and antenna configuration 1x2 and fading

Figure A.3.1.8.4: Test Equipment connection for tests with more than one NR cell for 4Rx capable UEs with fading

Figure A.3.1.8.5: Test Equipment connection for tests with more than one NR cell and antenna configuration 1x4

### A.3.1.9	Test Equipment supporting NR Sidelink

Figure A.3.1.9.1: Test Equipment connection for NR sidelink operation non-concurrent with NR UL/DL transmission

Figure A.3.1.9.2: Test Equipment connection for NR sidelink operation non-concurrent with NR UL/DL transmission SL-MIMO

Figure A.3.1.9.3: Test Equipment connection for inter-band con-current NR V2X operation

Figure A.3.1.9.4: Test Equipment connection for con-current E-UTRA Uu and NR Sidelink operation

Figure A.3.1.9.5: Test Equipment connection for con-current E-UTRA V2X sidelink and NR Uu operation

## A.3.2	User Equipment Parts for Conducted Measurements

### A.3.2.1	General

The User Equipment part is focused on the number of physical antenna connectors and how to combine in the DUT. Depending on the DUT implementation only one of the following connection diagrams applies. These connection diagrams are examples of User equipment parts.

### A.3.2.2	One Antenna Connector

Figure A.3.2.2.1: User Equipment connection for single basic cell

### A.3.2.3	Two Antenna Connectors

Figure A.3.2.3.1: User Equipment connection for single basic cell with NR and LTE cells at different separated connectors

Figure A.3.2.3.2: User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells

Figure A.3.2.3.3: 2 Tx User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells and 2TX UL MIMO or Tx diversity supported

Figure A.3.2.3.4: User Equipment connection for UEs with NR and LTE RxTx and Rx antenna at same connectors

Figure A.3.2.3.5: User Equipment connection for single basic cell with NR and LTE cells at different separated connectors with NR SUL and NR NUL transmitted on separate antenna connectors

Figure A.3.2.3.6: User Equipment connection for single basic cell with NR and LTE cells at different separated connectors with NR SUL and NR NUL transmitted on the same antenna connector

Figure A.3.2.3.7: User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells with NR SUL and NR NUL transmitted on separate antenna connectors

Figure A.3.2.3.8: User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells with NR SUL and NR NUL transmitted on the same antenna connector

Figure A.3.2.3.9: User Equipment connection for single basic cell with NR and LTE cells at different separated connectors with UE supporting UL MIMO on SUL band

Figure A.3.2.3.10: User Equipment connection for single basic cell with NR and LTE cells at the same connectors for both cells with UE supporting UL MIMO on SUL band

### A.3.2.4	Three Antenna Connectors

Figure A.3.2.4.1: 2Tx User Equipment connection for single basic cell with NR and LTE cells at different separated connectors and 2TX UL MIMO supported

### A.3.2.5	Four Antenna Connectors

Figure A.3.2.5.1: User Equipment connection for UEs with NR and LTE RxTx and Rx antenna at different separated connectors

Figure A.3.2.5.2: User Equipment connection for 4Rx capable UEs without any 2Rx RF bands (NR and LTE at same connectors)

Figure A.3.2.5.3: User Equipment connection for UEs with NR CA (component carriers on common connector) and LTE

Figure A.3.2.5.4: User Equipment connection for UEs with NR SUL and DL CA (component carriers on common connector) and LTE

### A.3.2.6	Over Four Antenna Connectors

Figure A.3.2.6.1: User Equipment connection for UEs with NR CA (component carriers on separated connectors) and LTE

Figure A.3.2.6.2: User Equipment connection for UEs with NR CA and NR 4Rx (component carriers on separated connectors) and LTE

Note: NR may be 2Rx on some of the CCs, in that case RX3 and RX4 are not used

### A.3.2.7	User Equipment supporting NR Sidelink

Figure A.3.2.7.1: User Equipment connection for NR sidelink operation non-concurrent with NR UL/DL transmission

Figure A.3.2.7.2: User Equipment connection for NR sidelink operation non-concurrent with NR UL/DL transmission with SL-MIMO

Figure A.3.2.7.3: User Equipment connection for inter-band con-current NR V2X operation

Figure A.3.2.7.4: User Equipment connection for con-current E-UTRA Uu and NR Sidelink operation

Figure A.3.2.7.5: User Equipment connection for con-current E-UTRA V2X sidelink and NR Uu operation

## A.3.3	Test Equipment Parts for Radiated Measurements

### A.3.3.1	Transmitter/Receiver tests

The Test Equipment part is focused on logical representation of TE measurement and link antenna(s) and positioner controller. The Test Equipment connection diagram below is applicable for NR radiated RX and TX tests, including CA and UL MIMO tests.

Figure A.3.3.1.1: Basic TE diagram for radiated RX and TX tests

For NR radiated RX tests requiring to simulate a modulated interference, connection diagram defined in figure A.3.3.1.2 will apply.

Figure A.3.3.1.2: TE diagram for radiated RX tests with Modulated Interference

For NR radiated spurious emission test cases, connection diagram defined in figure A.3.3.1.3 will apply.

Measurement antenna(s)                                        (dual or single polarized)

SS NR

Link antenna(s)

RX

DUT

Positioner System for link and/or measurement antenna(s)

LTE TX/RX antenna(s)

SS LTE

Measurement
antenna(s)

Spectrum Analyzer

Figure A.3.3.1.3: TE diagram for spurious emission tests

### A.3.3.2	Demodulation and CSI tests

Figure A.3.3.2.1: Demodulation and CSI tests

Figures A.3.3.2.1-1 and A.3.3.2.1-1 show the connection diagram inside SS NR of Figure A.3.3.2.1 for downlink signal path.

Figure A.3.3.2.1-1: TE diagram for Demodulation and CSI tests (1x2)

Figure A.3.3.2.1-2: TE diagram for Demodulation and CSI tests (2x2)

### A.3.3.3	RRM tests

The Test Equipment part is focused on logical representation of TE antenna(s) and positioner. The Test Equipment connection diagram below is applicable for NR radiated RRM tests. SS NR uses several antennas to cover all required AoA offsets. The actual number of antennas is not determined and depends on the TE implementation. Positioner in the TE part is optional.

Figure A.3.3.3.1: TE diagram for radiated RRM tests

Figure A.3.3.3.1-1 shows the connection diagram inside the SS NR of Figure A.3.3.3.1 for downlink signal path (for single probe). 1x2 without fading in FR2 RRM test case represents the scenario with single antenna transmission from TE side and 2 antenna receptions at UE side, which is equivalent to 1x1 in conducted test case from test equipment perspective.

Figure A.3.3.3.1-1: TE diagram for radiated 1AoA RRM tests (1x2 without fading)

## A.3.4	User Equipment Parts for Radiated Measurements

### A.3.4.1	Basic Transmitter/Receiver tests

The User Equipment part is focused on logical representation of UE antenna(s), DUT positioner and positioner controller. The UE connection diagram below is applicable for NR radiated RX and TX tests, including CA and UL MIMO tests.

Figure A.3.4.1.1: UE diagram for radiated RX and TX tests

### A.3.4.2	Demodulation and CSI tests

Same as Figure A.3.4.1.1.

### A.3.4.3	RRM tests

Same as Figure A.3.4.1.1.

Annex B (normative):
Permitted test methods For OTA Testing

# B.1	General

Editor's Note: The working assumption is that the DFF or IFF: CATR based OTA test methodologies defined in Annexes B.2.2 and B.2.4 respectively should be used for Signalling test.

The applicability of the permitted test methods herein is defined by the appropriate references within clauses 5, 6, and 7. A summary of the applicability is shown in Table B.1-1.

Table B.1-1: Permitted Test Methods Applicability Summary

| Permitted Test Methods | UE RF | Demodulation | RRM |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 AoA | 2 AoA |
| DFF | yes | yes | yes | yes |
| DFF Simplification | yes | yes | yes | N/A |
| IFF | yes | yes | yes | N/A |
| NFTF | yes (Note 1) | N/A | N/A | N/A |
| Enhanced IFF | yes | yes | yes | yes |
| IFF+DFF | yes | yes | yes | yes |
| Note 1: Not applicable for EIS, Frequency Error, EVM, Carrier Leakage, In-Band Emission, EVM SF, OBW as defined in Table J-1 of TS38.521-2 [15] |  |  |  |  |

# B.2	Permitted Test Methods

## B.2.1	General

The main objective of this annex is to specify basic parameters of permitted OTA test methods suitable for RF Tx and Rx, Performance, and RRM measurements and Signalling Conformance tests performed at high frequency in the FR2 operating bands defined in clause 4.3.1.2. The applicability of each OTA test method is summarized in Table B.1-1.

## B.2.2	Direct far field (DFF)

### B.2.2.1	Description

The DFF measurement setup for FR2 is capable of centre and off-centre of beam measurements and is shown in Figure B.2.2.1-1 below.

Figure B.2.2.1-1: DFF measurement setup

The key aspects of the DFF setup are:

-	Far-field measurement system in an anechoic chamber

-	The criterion for determining the far-field distance is described in B.2.2.4.

-	A positioning system such that the angle between the dual-polarized measurement antenna and the DUT has at least two axes of freedom and maintains a polarization reference.

-	A positioning system such that the angle between the link antenna and the DUT has at least two axes of freedom and maintains a polarization reference; this positioning system for the link antenna is in addition to the positioning system for the measurement antenna and provides for an angular relationship independently controllable from the measurement antenna.

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1 UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

The applicability criteria of the DFF setup are:

-	The DUT radiating aperture is D ≤ 5 cm

-	Either a single radiating aperture, multiple non-coherent apertures, or multiple coherent apertures DUTs can be tested

-	If multiple antenna panels that are phase coherent are defined as a single array, the criterion on DUT radiating aperture applies to this single array

-	D is based on the MU assessment in Annex B.1.1.3 of TR 38.810 [24]

-	A measurement distance larger than the far-field criteria defined in B.2.2.4 is not precluded

-	If the uncertainties can be further optimized, the MU may be reduced or D may be increased

-	A manufacturer declaration on the following elements is needed unless the entire DUT size is contained in a sphere of diameter of ≤ 5 cm:

-	Manufacturer declares antenna array size

For RRM testing, an example baseline system with two simultaneously active AoA (NMAX_AoAs = 2) as defined in Clause 7.1.3 using a DFF setup is illustrated in Figure 2.2.1-2. Implementations of the RRM baseline system with only a subset of the probes are possible as long as the system can satisfy the relative angular relationships outlined in Clause 7.1.3.2.1.

Figure B.2.2.1-2: Example RRM baseline system with two simultaneously active AoA using a DFF setup.

### B.2.2.2	Quiet zone dimension

In order to allow testing of DUTs of different sizes and to allow for flexibility in test chamber implementations, there will be various defined quiet zone dimensions. The smallest quiet zone shall have a radius of 100mm to accommodate DUTs such as smartphones. The next larger quiet zone shall have a radius of 150mm to accommodate larger DUTs such as tablets. To test even larger devices, e.g., larger tablets and laptops, quiet zones with 200mm and 275mm are defined The device types are listed as examples and other device types are not precluded.

The radiating portions of the device have to be fully enclosed within the quiet zone, but the non-radiating portions of the device can be located/placed outside the quiet zone if a vendor declaration with positioning reference points and the minimum QZ required to contain all active antennas within the quiet zone (per band) is provided. This grey-box testing approach where the declared reference point is aligned with the centre of the QZ is further illustrated in Figure B.2.2.2-1.

Figure B.2.2.2-1: Grey-box test approach

In the absence of a vendor declaration, the geometric centre of the DUT shall be aligned with the centre of the QZ and the DUT shall be fully contained within the QZ. This black-box testing approach is further illustrated in Figure B.2.2.2-2.

Figure B.2.2.2-2: Black-box test approach

### B.2.2.3	Quality of the quiet zone

The quality of the quiet zone shall be measured for the frequencies defined in FFS. The measured quality of the quiet zone performance is used in uncertainty calculations for the appropriate quality of the quiet zone dimension utilized for the DUT.

### B.2.2.4	Measurement Distance

For far-field measurements, the distance R between the DUT and the measurement antenna shall be calculated by the following equation.

where  is the largest wavelength within the frequency band of interest and D is the diameter of the smallest sphere that encloses the radiating parts of the DUT.

For DFF, free space path loss is calculated by applying the Free Space Loss formula with R equal to the far field distance: .

The minimum range length of a DFF system, i.e., the minimum distance between the centre of the quiet zone and the measurement antenna, needs to take into account the unknown offset of the antenna aperture from the centre of quiet zone in order to guarantee far-field conditions for any antenna array integrated inside the DUT. The distance between the centre of the quiet zone to the measurement antenna is referenced as RDFF, while the radius of the quiet zone is RQZ as illustrated in Figure B.2.2.4-1. The minimum distance between the antenna array integrated anywhere within the DUT and the measurement antenna needs to meet the far-field distance, RFF = 2D2/.

Figure B.2.2.4-1: Illustration of DFF System for range length definition

The setup in Figure B.2.2.4-2 is used to derive the minimum range length for NR FR2 DFF systems where the sphere enclosing the DUT matches the QZ and the DUT antenna with radiating aperture diameter D located in the corner of the DUT. With this setup, the minimum range length, RDFF, can be determined as

RDFF = RQZ – D/2 + RFF = RQZ – D/2 + 2D2/

which is tabulated in Table B.2.2.4-1 for two different QZ sizes assuming D=5cm.

Figure B.2.2.4-2: Illustration of DFF System for minimum range length definition

Table B.2.2.4-1: Minimum Range Length of DFF System for D = 5cm

| f [GHz] QZ [cm] | 24.25 | 30 | 40 | 50 | 52.6 |
| --- | --- | --- | --- | --- | --- |
| 20 | 0.48 | 0.58 | 0.74 | 0.91 | 0.95 |
| 30 | 0.53 | 0.63 | 0.79 | 0.96 | 1.00 |

The influence of measurement distance on measurement uncertainty is discussed in Annex B.2.1 of TR 38.903 [XX].

## B.2.3	Direct far field (DFF) setup simplification for centre of beam measurements

### B.2.3.1	Description

The DFF setup in Annex B.2.2 can be simplified in the following way to perform centre of the beam measurements:

-	The measurement and the link antenna can be combined so that the single antenna is used to steer the beam and to perform UE measurements.

The measurement setup for FR2 capable of centre of beam measurements is shown in Figure B.2.3.1-1 below.

Figure B.2.3.1-1: DFF simplification for centre of beam measurement setup

The applicability criteria of the simplified DFF setup for centre of beam measurements are defined in B.2.2.1.

### B.2.3.2	Quiet zone dimension

Same as Annex B.2.2.2.

### B.2.3.3	Quality of the quiet zone

Same as Annex B.2.2.3.

### B.2.3.4	Measurement Distance

Same as Annex B.2.2.4.

## B.2.4	Indirect far field (IFF): Compact Antenna Test Range (CATR)

### B.2.4.1	Description

The IFF method utilizing a compact antenna test range (CATR) creates the far field environment using a transformation with a parabolic reflector.

The IFF CATR measurement setup for FR2 is capable of centre and off-centre of beam measurements and an example setup is shown in Figure B.2.4.1-1 below. The relative orientation of the coordinate system with respect to the reflector and the axes of rotation apply to any CATR measurement setup.

Figure B.2.4.1-1: Example of IFF: CATR measurement setup

The key aspects of this test method setup are:

-	Indirect Far Field using Compact Antenna Test Range as described in TR 38.810 [24] with quiet zone diameter that meets the requirements of B.2.4.2.

-	A positioning system such that the angle between the dual-polarized measurement antenna and the DUT has at least two axes of freedom and maintains a polarization reference.

-	Before performing the UE Beamlock Test Function as defined in clause 4.9.2, the measurement probe acts as a link antenna maintaining polarization reference with respect to the DUT. Once the beam is locked then the link is to be passed to the link antenna which maintains reliable signal level with respect to the DUT.

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

The applicability criteria of this test method are:

-	The total test volume, i.e., the quiet zone is defined as a sphere with radius R.

-	DUT must fit within the quiet zone for the entire duration of the test.

-	Either a single radiating aperture, multiple non-coherent apertures or multiple coherent apertures DUTs can be tested.

-	No manufacturer declaration of the antenna array size is needed.

### B.2.4.2	Quiet zone dimension

Same as Annex B.2.2.2.

### B.2.4.3	Quality of the quiet zone

Same as Annex B.2.2.3.

### B.2.4.4	Measurement Distance

The CATR system does not require a measurement distance of  to achieve a plane wave as in a standard far field range.

For the CATR system, the far-field distance is seen as the focal length. The focal length is the distance between the feed and the reflector of the CATR. Further information on the focal length of a CATR system can be found in clause 5.2.3.2 of TR 38.810 [24].

The measurement distance for any CATR system implementation shall be adequate to meet the quiet zone dimensions defined in B.2.4.2.

In a CATR, from the reflector to the quiet zone, there is a plane wave with no free space path loss.

For CATR, free space path loss is calculated by applying the Free Space Loss formula with R equal to the far field distance based on the focal length: .

A summary of the comparison of path losses which can be expected for the CATR compared to a Fraunhofer limit distance () for different antenna sizes and frequencies can be found in clause 5.2.3.2 of TR 38.810 [24].

The influence of measurement distance on measurement uncertainty can be considered as zero as defined in Annex B.2.2 of TR 38.903 [XX].

## B.2.5	Near field to far field transform (NFTF)

### B.2.5.1	Description

The NFTF method computes the metrics defined in Far Field by using the Near Field to Far Field transformation.

The NFTF measurement setup of UE RF characteristics for FR2 is capable of centre and off centre of beam measurements and an example setup is shown in Figure B.2.5.1-1:

Figure B.2.5.1-1: Example of NFTF measurement setup

The key aspects of the Near Field test range are:

-	Radiated Near Field UE beam pattern is measured and based on the NFTF mathematical transform, the final metric such as EIRP is the same as the metric for the baseline setup

-	A positioning system such as the angle between the dual-polarized measurement/link antenna and the DUT has at least two axes of freedom and maintains a polarization reference

-	For setups intended for measurements of UE RF characteristics in non-standalone (NSA) mode with 1UL configuration, an LTE link antenna is used to provide the LTE link to the DUT. The LTE link antenna provides a stable LTE signal without precise path loss or polarization control.

-	For setups intended for measurements in NR CA mode with FR1 and FR2 inter-band NR CA, test setup provides NR FR1 link to the DUT. The NR FR1 link has a stable and noise-free signal without precise path loss or polarization control.

The applicability criteria of the NFTF setup are:

-	The DUT radiating aperture is D ≤ 5 cm

-	Either a single radiating aperture, multiple non-coherent apertures or multiple coherent apertures DUTs can be tested

-	If multiple antenna panels that are phase coherent are defined as a single array, the criterion on DUT radiating aperture applies to this single array

-	D is based on the MU assessment in Annex B.1.4.3 of TR 38.810 [24]

-	If the uncertainties can be further optimized, the MU may be reduced or D may be increased

-	A manufacturer declaration on the following elements is needed unless the entire DUT size is contained in a sphere of diameter of ≤ 5 cm:

-	Manufacturer declares antenna array size

-	EIRP, TRP, and spurious emissions metrics can be tested.

### B.2.5.2	Quiet zone dimension

Same as Annex B.2.2.2.

### B.2.5.3	Quality of the quiet zone

Same as Annex B.2.2.3.

### B.2.5.4	Measurement Distance

The NFTF system does not require a measurement distance of  as in a standard far field range due to the use of the Near Field to Far Field transformation.

The measurement distance for any NFTF system implementation shall ensure that the DUT is not measured in the reactive near-field region and is adequate to meet the quiet zone dimensions defined in B.2.5.2.

## B.2.6	Enhanced IFF

### B.2.6.1	Description

The Enhanced IFF method utilizing multiple compact antenna test ranges (CATRs) creates the far field environment using a transformation with 2 or more parabolic reflectors for RRM testing with two simultaneously active AoA (NMAX_AoAs = 2) as defined in Clause 7.1.3.

An example RRM baseline system using an Enhanced IFF setup are shown in Figure B.2.6.1-1. Implementations of the RRM baseline system with only a subset of the reflectors/probes are possible as long as the system can satisfy the relative angular relationships outlined in Clause 7.1.3.2.1.

Figure B.2.6.1-1: Example RRM baseline system with two simultaneously active AoA using an Enhanced IFF setup

The key aspects of this test method setup are the same as the IFF setup, outlined in Clause B.2.4.1.

### B.2.6.2	Quiet zone dimension

Same as Clause B.2.2.2.

### B.2.6.3	Quality of the quiet zone

Same as Clause B.2.2.3.

### B.2.6.4	Measurement Distance

Same as Clause B.2.4.4.

## B.2.7	IFF+DFF

### B.2.7.1	Description

The IFF+DFF method is utilizing a combination of compact antenna test ranges (CATRs) and DFF probes for RRM testing with two simultaneously active AoA (NMAX_AoAs = 2) as defined in Clause 7.1.3.

An example RRM baseline system using the IFF+DFF setup is shown in Figure B.2.7.1-1. Implementations of the RRM baseline system with only a subset of the probes are possible as long as the system can satisfy the relative angular relationships outlined in Clause 7.1.3.2.1.

Figure B.2.7.1-1: Example RRM baseline system with two simultaneously active AoA using an IFF+DFF setup

The key aspects of this test method setup are the same as the IFF setup for the probes based on the IFF method, outlined in B.2.4.1 and the DFF setup for the probes based on the DFF method, outlined in Clause B.2.2.1.

### B.2.7.2	Quiet zone dimension

Same as Clause B.2.2.2.

### B.2.7.3	Quality of the quiet zone

Same as Clause B.2.2.3.

### B.2.7.4	Measurement Distance

Same as Clause B.2.4.4 for the IFF setup for the probes based on the IFF method and Clause B.2.2.4 for the DFF setup for the probes based on the DFF method.

Annex C (informative):
Calculation of test frequencies

# C.0	General

Test frequencies are defined in clause 4.3.1 with extensions for signalling test cases in clause 6.2.3. This annex gives a guideline to determine these test frequencies and the associated signalling parameters for a given NR band, NR CA or NR DC band combination.

Clause C.1 describes definitions and parameters used by the procedures to determine test frequencies, SS/PBCH Block (SSB) and CORESET#0 configuration parameters.

Clause C.2 describes how to calculate test frequencies for symmetric NR bands, asymmetric NR bands, NR CA and NR DC configurations.

Clause C.3.2 describes how to determine the SSB, CORESET#0 and signalling parameters for FR1 carriers with SCS=15 kHz or SCS=30 kHz, and for FR2 carriers with SCS=60 kHz or SCS=120 kHz. CORESET#0 is required for a carrier to be used as a Pcell.

Clause C.3.3 describes how to determine the SSB and signalling parameters for a carriers without CORESET#0.

Clause C.5 describes how to calculate test frequencies for NR V2X bands.

# C.1	Definitions and Parameters

Figure C.1-1 shows SSB and CORESET#0 and related parameters.CORESET#0 is required for a carrier to be used as a PCell.

Figure C.1-1: location of SSB and CORESET#0 within a channel

The parameters referenced in figure C.1-1 are defined in Table C.1-1.

Table C.1-1: Definition of parameters in Figure C.1-1 used in Annex C

| Parameter | Description |
| --- | --- |
| FPointA | Reference Point A frequency. |
| Fcarrier | Fcarrier is the centre frequency of a carrier corresponding to its NR-ARFCN value. |
| FcarrierLow | FcarrierLow is the centre frequency of lowest subcarrier of the carrier. FcarrierLow = Fcarrier - 12 * SCSCarrier * (NRB / 2) with NRB according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8] for the channel bandwidth of the carrier. |
| ΔFcarrierBandwidth | ΔFcarrierBandwidth is the carrier’s channel bandwidth as provided in carrierBandwidth to the UE (SCS-SpecificCarrier). |
| ΔFoffsetToCarrier | ΔFoffsetToCarrier is the frequency offset between Point A and the lower edge of the carrier. FoffsetToCarrier = offsetToCarrier * PRB size, where PRB size according to the subcarrier spacing of the carrier. offsetToCarrier is signalled to the UE (SCS-SpecificCarrier). |
| FSSref | Centre frequency of SSB. For a cell selectable as PCell the FSSref corresponds to a valid GSCN value according to clause 5.4.3.1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFOffset(RBs) | ΔFOffset(RBs) = 12* Offset(RBs) * subCarrierSpacingCommon, where Offset(RBs) is given in tables 13-1 to 13-10 of TS 38.213 [22]. |
| ΔFkSSB | ΔFKSSB = kSSB * {15 kHz for FR1, subCarrierSpacingCommon (MIB) for FR2} (TS 38.211 [3], clause 7.4.3.1) |
| ΔFOffsetSSB-CORESET0 | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of CORESET#0. ΔFOffsetSSB-CORESET0 = ΔFOffset(RBs) + ΔFkSSB. |
| ΔFOffsetCORESET-0-Carrier | Frequency offset, FOffsetCORESET#0-Carrier, between the lowest subcarrier of CORESET#0 and the lowest subcarrier of the carrier. |
| ΔFOffsetSSB-Carrier | Frequency offset between the lowest subcarrier of the SSB and the lowest subcarrier of the carrier. |
| FCORESET0Low | Centre frequency of subcarrier 0 of CORESET#0. |
| FOffsetToPointA | Frequency of the lowest subcarrier of the lowest resource block, which has the subcarrier spacing provided by the higher-layer parameter subCarrierSpacingCommon and overlaps with the SS/PBCH block used by the UE for initial cell selection, expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2 (TS 38.211 [29] clause 4.4.4.2). |
| ΔFOffsetToPointA | Frequency offset between FOffsetToPointA and point A. ΔFOffsetToPointA = offsetToPointA * {15 kHz for FR1; 60 kHz for FR2} (TS 38.211 [29] clause 4.4.4.2). |

Additional parameters used in this annex are defined in Table C.1-2.

Table C.1-2: Definition of additional parameters used in Annex C.

| kSSB | as defined in TS 38.211 [29] clause 7.4.3.1 |
| --- | --- |
| SCSCarrier | subcarrier spacing for the carrier (SCS-SpecificCarrier): FR1: 15kHz, 30kHz or 60kHz according to TS 38.101-1 [7] Table 5.3.5-1 FR2: 60kHz or 120kHz according to TS 38.101-2 [8] Table 5.3.5-1 |
| SCSSSB | SS/PBCH block subcarrier spacing FR1: 15kHz or 30kHz according to TS 38.101-1 [7] Table 5.4.3.3-1 FR2: 120kHz or 240kHz according to TS 38.101-2 [8] Table 5.4.3.3-1 NOTE: According to the tables in clause 13 of TS 38.213 [22] not all combinations of SCSSSB and SCSCarrier are applicable. |
| SCScommon | Subcarrier spacing for SIB1, Msg.2/4 for initial access, paging and broadcast SI-messages. Provided to the UE in the MIB in IE subCarrierSpacingCommon. |
| PRBsize | Physical Resource Block size of the carrier = 12 * SCSCarrier. |
| CRBsize | Common Resource Block size = 12 * SCScommon. |
| FDL_Low, FUL_Low | Lowest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FDL_High, FUL_High | Highest frequency of the downlink and uplink frequency range of the band as defined in clause 5.2 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| ΔFRaster | Frequency raster of the band as defined in clause 5.4.2.3 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| BWDL | Bandwidth of downlink frequency range of the band. |
| BWUL | Bandwidth of uplink frequency range of the band. |
| CBWDL | Downlink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| CBWUL | Uplink channel bandwidth (MHz) of the carrier according to Table 5.3.2-1 of TS 38.101-1 [7] and TS 38.101-2 [8]. |
| FTx-Rx_separation | Default Tx – Rx carrier centre frequency separation of the band as defined in clause 5.4.4 of TS 38.101-1 [7]. For TDD bands FTx-Rx_separation = 0. |
| ΔFTx-Rx_separation | ΔFTx-Rx = ¦ (BWDL – BWUL)/2 ¦ is the deviation to the default Tx-Rx carrier centre frequency separation (FTx-Rx_separation) for FDD FR1 bands supporting asymmetric channel bandwidths as defined in clause 5.3.6 of TS 38.101-1 [7]. |
| BWSSB | Bandwidth of the SSB. BWSSB = 12 * SCSSSB * 20 |
| ΔGSCN, GSCNMIN, GSCNMAX | GSCN step size, GSCN minimum and GSCN maximum values for the NR band according to table 5.4.3.3-1 of TS 38.101-1 [7] and TS 38.101-2 [8] |
| OffsetRBs | Offset (RBs) according to the applicable table 13-1 to 13-10 in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier. |
| OffsetRBs,max | Maximum value for Offset (RBs) according to the applicable table 13-1 to 13-1o in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier limited to the table indexes with number of RBs equal to the minimum value of in the table. |
| OffsetRBs,min | Minimum value for Offset (RBs) according to the applicable table 13-1 to 13-1o in TS 38.213 [22] for the given band and {SCSSSB, SCSCarrier} combination of the carrier limited to the table indexes with number of RBs equal to the minimum value of in the table. |

# C.2	Determination of test frequencies

## C.2.0	General

Test frequencies are determined as:

For symmetric NR bands (supporting same bandwidth in UL and DL):

-	test frequencies for the supported symmetric channel bandwidth combinations are determined as described in clause C.2.1; and

-	the test frequencies for the supported asymmetric channel bandwidth combinations are determined as described in clause C.2.3.

For asymmetric NR bands (supporting different bandwidth in UL and DL):

-	the test frequencies for the supported symmetric channel bandwidth combinations are determined as described in clause C.2.2; and

-	the test frequencies for the supported asymmetric channel bandwidth combinations are determined as described in clause C.2.3.

For NR CA and NR DC:

-	the test frequencies are determined as described in the relevant subclause in C.2.4 depending to the type of configuration.

The carrier test frequencies are determined considering the channel raster according to clause 5.4.2.3 in TS 38.101-1 [7] for FR1 and in TS 38.101-2 [8] for FR2.

## C.2.1	Determination of test frequencies for symmetric NR bands and symmetric uplink and downlink channel bandwidth combinations

### C.2.1.1	Determination of test frequencies for Low-, Mid- and High-Range

Downlink:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq1 |
| --- | --- |
| FDL_MidRange = Round((FDL_Low + BWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq2 |
| FDL_HighRange = Floor((FDL_High - CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.1.1-Eq3 |

FDL_LowRange is rounded up and FDL_HighRange is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

Uplink:

| FUL_LowRange = FDL_LowRange - FTx-Rx_separation | C.2.1.1-Eq4 |
| --- | --- |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation | C.2.1.1-Eq5 |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation | C.2.1.1-Eq6 |

For those FDD frequency bands with two UL sub-bands, FTx-Rx_separation shall be chosen using the following criteria:

- For Low-Range and Mid-Range, use FTx-Rx_separation for lower UL sub-band.

- For High-Range, use FTx-Rx_separation for higher UL sub-band.

### C.2.1.2	Determination test frequencies for of Mid-Low and Mid-High-Range for signalling tests

| FMid-LowRange = Round((FLowRange+ (FHighRange - FLowRange )/3) / ΔFRaster) * ΔFRaster | C.2.1.2-Eq1 |
| --- | --- |
| FMid-HighRange = Round((FLowRange + 2*(FHighRange - FLowRange )/3) / ΔFRaster) * ΔFRaster | C.2.1.2-Eq2 |

## C.2.2	Determination of test frequencies for asymmetric NR bands and symmetric uplink and downlink channel bandwidth combinations

Determination of test frequencies for asymmetric NR bands, and symmetric uplink and downlink channel bandwidth combinations are determined using the procedure in clause C.2.3 with ΔFTx-Rx = 0.

## C.2.3	Determination of test frequencies for asymmetric uplink and downlink channel bandwidth combinations

### C.2.3.1	General

The following procedure is used to calculate test frequencies for NR bands supporting asymmetric UL and DL channel bandwidths as described below, where CBWUL and CBWDL refer to the carrier’s UL and DL channel bandwidths; and BWUL and BWDL refer to the band’s total UL and DL bandwidths.

The procedure is also used to calculate test frequencies for symmetric UL and DL bandwidth combinations for asymmetric NR bands.

For FDD bands supporting asymmetric uplink and downlink bandwidth combinations a deviation of ΔFTX-RX (C.2.3.1-Eq1) is to be added to the default Tx-Rx carrier centre frequency separation, FTx-Rx_separation (TS 38.101-1 [7] clause 5.3.6).

| ΔFTX-RX = ¦(CBWDL – CBWUL)/2¦ | C.2.3.1-Eq1 |
| --- | --- |

For the case of asymmetric NR bands and symmetric UL and DL bandwidth combinations ΔFTX-RX = 0.To meet the Tx-Rx frequency separation requirement for asymmetric NR bands were the supported overall UL bandwidth is smaller than the supported overall DL bandwidth it may not be possible to cover the full DL frequency range for all UL and DL channel bandwidth combinations. For CA when such band is only used for DL CC the full range can be used for all DL channel bandwidths.

To maximize the tested frequency range for the non-CA case the UL frequency range, as being smaller than the DL frequency range, need to be used as the starting point to calculate the uplink and downlink test frequencies.

### C.2.3.2	Determination of Low-, Mid- and High-Range for asymmetric uplink and downlink bandwidth combinations

The following procedure is used to determine the test frequencies for Low-, Mid- and High-Range for bands supporting asymmetric UL and DL bandwidth combinations.

1.	Calculate uplink carrier centre frequencies:

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq1 |
| --- | --- |
| FUL_MidRange = Round((FUL_Low + BWUL_Band/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq2 |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq3 |

For those FDD frequency bands with two UL sub-bands, for Low- and Mid-Range, use lower sub-band and for High-Range use higher sub-band. In these cases, BWUL_Band refers to the sub-band.

2.	Calculate the downlink frequencies:

Calculate the DL carrier centre frequencies from the UL frequencies in step 1.

| FDL_LowRange = FUL_LowRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq4 |
| --- | --- |
| FDL_MidRange = FUL_MidRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq5 |
| FDL_HighRange = FUL_HighRange + FTx-Rx_separation + ΔFTx-Rx | C.2.3.2-Eq6 |

For those FDD frequency bands where TX-RX carrier centre frequency separation can have multiple values, FTx-Rx_separation shall be chosen using the following criteria:

- For Low-Range, use FTx-Rx_separation that minimizes FDL_LowRange.for each SCS.

- For Mid-Range, use FTx-Rx_separation closer to FDL_LowRange + (FDL_HighRange – FDL_LowRange)/2.

- For High-Range, use FTx-Rx_separation that maximizes FDL_HighRange for each SCS.

3.	Check that the calculated centre test frequencies in step 2 for the BWDL fits within the bands DL frequency range:

If FDL_LowRange is smaller than the lowest frequency of the band then recalculate the minimum FDL_LowRange and modify the associated FUL_LowRange, FDL_MidRange and FUL_MidRange as:

| FDL_LowRange = Ceil((FDL_Low + CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq7 |
| --- | --- |
| FUL_LowRange = FDL_LowRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq8a |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq8b |

If FDL_HighRange is larger than the higher frequency of the band then recalculate the maximum FDL_HighRange and modify the associated FUL_HighRange, FDL_MidRange and FUL_MidRange as:

| FDL_HighRange = Floor((FDL_Low + BWDL_Band – CBWDL/2) / ΔFRaster) * ΔFRaster | C.2.3.2-Eq9 |
| --- | --- |
| FUL_HighRange = FDL_HighRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq10 |
| FDL_MidRange = Round((FDL_LowRange + FDL_HighRange)/2 / ΔFRaster))* ΔFRaster | C.2.3.2-Eq11 |
| FUL_MidRange = FDL_MidRange - FTx-Rx_separation - ΔFTx-Rx | C.2.3.2-Eq12 |

### C.2.3.3	Determination of test frequencies for a Mid range adjacent inter-frequency cell for FR2 RRM multicell testing

The following procedure is used to determine the test frequencies for Mid-Range adjacent inter-frequency cell used for RRM FR2 NR multi-cell in NR SA and EN-DC test cases. The reason for using an adjacent inter-frequency cell to the Mid-range cell for FR2 instead of using Low- or High- Range test frequencies as used for FR1 is to reduce test system complexity.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate the test frequencies for the Mid adjacent inter-frequency cell:

| Parameter | Description |
| --- | --- |
| FMid | Carrier centre frequency (MHz) of the Mid-range cell |
| CBWMid | Channel bandwidth (MHz) of the Mid-range cell |
| CBWAdjecent | Channel bandwidth (MHz) of the adjacent cell |

1.	Calculate the Mid-range adjacent cell carrier centre frequencies:

| FMidRangeAdjecentCell = Ceil((FMid + (CBWMid + CBWAdjecent)/2) / ΔFRaster) * ΔFRaster | C.2.3.3-Eq1 |
| --- | --- |

2.	Calculate SSB and CORESET#0 parameters as described in clause C.4.

## C.2.4	Frequency determination for NR CA and NR DC configurations

### C.2.4.1	Determination of test frequencies for NR Inter-band CA and NR DC

Test frequencies for NR Inter-band CA configurations and NR DC use the single carrier test frequencies for each of the included NR bands in the configuration as specified in clause 4.3.1.1.1 for FR1 bands and in clause 4.3.1.2.1 for FR2 bands.

### C.2.4.2	Determination of test frequencies for NR Intra-band Contiguous CA

#### C.2.4.2.1	General

By default, test frequencies for NR Intra-band Contiguous CA in clause 4.3.1 are specified using the nominal channel spacing between the carrier components as specified in TS 38.101-1 [7] clause 5.4A.1 for FR1 and TS 38.101-2 [8] clause 5.4A.1 for FR2. In addition, some NR bands may have test frequencies specified based on an adjusted channel spacing as specified in in TS 38.101-1 [7] clause 5.4A.1 for FR1 and TS 38.101-2 [8] clause 5.4A.1 for FR2.

The test frequencies for NR Intra-band Contiguous CA with SCS=15kHz or SCS=30 kHz for FR1 and with SCS=60 kHZ or SCS=120 kHz for FR2 is calculated for each CC such that the specific test cases can decide which CC is used as PCell. This means that all CC test frequencies is calculated with a CORESET#0 as specified in clause C.3.2.

The test frequencies for CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 is calculated without CORESET#0 as specified in C.3.3. CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 can only be used for NR Intra-band Contiguous CA configurations with mixed numerologies.

Note:	For NR Intra-band Contiguous CA configurations for bands where Tx frequency range is lower than Rx frequency range the RAN4 requirements for reference sensitivity testing is specified having the PCC frequency lower than the SCC frequencies such that UL PRB maximise the Tx-Rx separation. This means that CC1 shall be used as PCell in the reference test case for bands where Tx frequency range is lower than Rx frequency range; and highest CC shall be used as PCell in the reference test case for bands where Tx frequency range is higher than Rx frequency range.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate carrier components (CC) test frequencies for NR Intra-band Contiguous and Non-contiguous CA configurations:

| Parameter | Description |
| --- | --- |
| NCC | Number of CCs in the for NR Intra-band configuration |
| CCBWDL(i) | Channel bandwidth (MHz) of downlink CC(i), where i = 1 to NCC |
| FChannel_Spacing(i) | Channel spacing between CC(i) and CC(i+1), where i = 1 to (NCC-1) |

#### C.2.4.2.2	Determination of test frequencies for Low-, Mid- and High-Range

Downlink CC(1), lowest frequency CC:

FDL_LowRange_CC(1) is rounded up and FDL_HighRange_CC(1) is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

| FDL_LowRange_CC(1) = Ceil((FDL_Low + CCBWDL(1) / 2) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq1 |
| --- | --- |
| FDL_MidRange_CC(1) = Round((FDL_Low + BWDL/2 – ∑k=1 to (Ncc) CCBWDL (k)/2 + CCBWDL(1)/2) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq2 |
| FDL_HighRange_CC(1) = Floor((FDL_High - CCBWDL(NCC)/2– ∑k=1 to (Ncc-1) FChannel_Spacing(k)) / ΔFRaster) * ΔFRaster | C.2.4.2.2-Eq3 |

Downlink CC(2) to CC(NCC), in increasing frequency order:

| FDL_LowRange_CC(i) = FDL_LowRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq4 |
| --- | --- |
| FDL_MidRange_CC(i) = FDL_MidRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq5 |
| FDL_HighRange_CC(i) = FDL_HighRange_CC(i-1) + FChannel_Spacing(i)), i=2 to NCC | C.2.4.2.2-Eq6 |

Uplink CC(i), i=1 to NCC:

| FUL_LowRange_CC(i) = FDL_LowRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq7 |
| --- | --- |
| FUL_MidRange_CC(i) = FDL_MidRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq8 |
| FUL_HighRange_CC(i) = FDL_HighRange_CC(i) - FTx-Rx_separation | C.2.4.2.2-Eq9 |

### C.2.4.2A	Determination of test frequencies for FR1 NR Intra-band Contiguous CA without UL CA for bands with uplink bandwidth less than downlink bandwidth

#### C.2.4.2A.1	General

By default, test frequencies for FR1 NR Intra-band Contiguous CA for bands with uplink bandwidth less than downlink bandwidth in clause 4.3.1 (e.g. n66 and n70) are specified with CC1 used as PCC and all additional CCs are specified as SCCs without UL to enable the SCCs for High range to extend into the upper DL BW beyond the UL BW. The nominal channel spacing between the carrier components is calculated as specified in TS 38.101-1 [7] clause 5.4A.1.

In addition to the definition of parameters in clause C.1 the defintion of parameters NCC, CCBWDL and FChannel_Spacing in clause C.2.4.2.1, and ΔFTX-RX in clause C.2.3.1 are used to calculate the test frequencies.

#### C.2.4.2A.2	Determination of test frequencies for Low-, Mid- and High-Range

1.	Calculate UL carrier centre frequencies for Low and High ranges:

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.4.2A.2-Eq1 |
| --- | --- |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.4.2A.2-Eq2 |

2.	Calculate the DL CC(1) carrier centre frequencies from the UL frequencies in step 1 for Low and High ranges:

| ΔFTX-RX = ¦(CBWDL(1) – CBWUL)/2¦ | C.2.4.2A.2-Eq3 |
| --- | --- |
| s = +1 if FUL_Low <= FDL_Low else -1 | C.2.4.2A.2-Eq4 |
| FDL_LowRange_CC(1) = FUL_LowRange + s*(FTx-Rx_separation + ΔFTx-Rx) | C.2.4.2A.2-Eq5 |
| FDL_HighRange_CC(1) = FUL_HighRange + s*(FTx-Rx_separation + ΔFTx-Rx) | C.2.4.2A.2-Eq6 |

3.	Check that DL aggregated CBW for the High range fits into the DL bandwidth.

| FDL_HighRange_max = Floor((FDL_HighRange_CC(1) + ∑k=1 to Ncc-1 FChannel_Spacing(k)+ CCBWDL(NCC)/2) | C.2.4.2A.2-Eq7 |
| --- | --- |

If FDL_HighRange_max is less or equal to FDL_High then goto step 4 else modify FDL_HighRange_CC1 such that the full aggregated CBW is located at the DL bandwidth high edge and recalculate FUL_HighRange.

| FDL_HighRange_CC(1) = FDL_High - CCBWDL(NCC)/2 - ∑k=1 to (Ncc-2) FChannel_Spacing(k) | C.2.4.2A.2-Eq8 |
| --- | --- |
| FUL_HighRange = FDL_HighRange_CC(1) – s*(FTx-Rx_separation + ΔFTx-Rx) | C.2.4.2A.2-Eq9 |

4.	Calculate the FUL_MidRange and FDL_MidRange_CC(1):

| FDL_MidRange_CC(1) = Round((FDL_LowRange_CC(1) + FDL_HighRange_CC(1))/2) / ΔFRaster) * ΔFRaster | C.2.4.2A.2-Eq10 |
| --- | --- |
| FUL_MidRange = FUDL_MidRange_CC(1) – s*(FTx-Rx_separation + ΔFTx-Rx) | C.2.4.2A.2-Eq11 |

5.	Calulate DL CC(2) to CC(NCC), in increasing frequency order:

| FDL_LowRange_CC(i) = FDL_LowRange_CC(i-1) + FChannel_Spacing(i), i=2 to NCC | C.2.4.2A.2-Eq12 |
| --- | --- |
| FDL_MidRange_CC(i) = FDL_MidRange_CC(i-1) + FChannel_Spacing(i), i=2 to NCC | C.2.4.2A.2-Eq13 |
| FDL_HighRange_CC(i) = FDL_HighRange_CC(i-1) + FChannel_Spacing(i), i=2 to NCC | C.2.4.2A.2-Eq14 |

### C.2.4.3	Determination of test frequencies for NR Intra-band Non-Contiguous CA

#### C.2.4.3.1	General

The default test frequencies in clause 4.3.1 for NR Intra-band Non-Contiguous CA are based on maximum Wgap between the carrier components of the different bands taking the UE capability of maximum supported frequency separation between the lower edge of lowest component carrier and upper edge of highest component carrier for UL for FR1, and UL and DL for FR2.

Test frequencies with Wgap different from maximum Wgap are specified in the specific test cases using them.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate carrier components (CC) test frequencies for NR Intra-band Contiguous and Non-contiguous CA configurations:

| Parameter | Description |
| --- | --- |
| NSB | Number of sub-block in the for NR Intra-band non-contiguous configuration |
| SBCBWDL(i) | Downlink channel bandwidth (MHz) of sub-block SB(i), where i = 1 to NSB |
| maxFsBW | Maximum frequency separation bandwidth between the lower edge of lowest component carrier and upper edge of highest component carrier. |
| maxWgap | maxWgap is the maximum separation in MHz between each sub-block in the NR Intra-band non-contiguous configuration within maxFsBW for a given sub-block combination. |

#### C.2.4.3.1A	Selection of maximum frequency separation for FR1

Select the maxFsBW dependent on the type of configuration and aggregated CBW for the sub-block combination in Table C.2.4.3.1A-1:

Table C.2.4.3.1A-1: Selecting maxFsBW for FR1

| Type of configuration | Aggregated CBW | maxFsBW |
| --- | --- | --- |
| Without UL CA | All | Full NR bandwidth |
| With UL CA | <100 MHz | Min(Full NR bandwidth,100 MHz) |
|  | >=100 MHz and <200 MHz | Min(Full NR bandwidth,200 MHz) |
|  | >=200 MHz and <600 MHz | Min(Full NR bandwidth,600 MHz= |
|  | =600 MHz | Full NR bandwidth |

#### C.2.4.3.1B	Selection of maximum frequency separation for FR2

Select the maxFsBW dependent on the type of configuration and aggregated CBW for the sub-block combination in Table C.2.4.3.1B-1:

Table C.2.4.3.1B-1: Selecting maxFsBW for FR2

| Type of configuration | Aggregated CBW | maxFsBW (Note 1) |
| --- | --- | --- |
| With non-contiguous UL CA | <400 MHz | 400 MHz |
|  | >=400 MHz and <600 MHz | 600 MHz |
|  | >=600 MHz and <800 MHz | 800 MHz |
|  | >=800 MHz and <1000 MHz | 1000 MHz |
|  | >=1000 MHz and <1200 MHz | 1200 MHz |
|  | >=1200 MHz and <1400 MHz | 1400 MHz |
|  | >=1400 MHz | N/A |
| Without non-contiguous UL CA | <400 MHz | 400 MHz |
|  | >=400 MHz and <600 MHz | 600 MHz |
|  | >=600 MHz and <800 MHz | 800 MHz |
|  | >=800 MHz and <1000 MHz | 1000 MHz |
|  | >=1000 MHz and <1200 MHz | 1200 MHz |
|  | >=1200 MHz and <1400 MHz | 1400 MHz |
|  | >=1400 MHz and <1600 MHz | 1600 MHz |
|  | >=1600 MHz and <1800 MHz | 1800 MHz |
|  | >=1800 MHz and <2000 MHz | 2000 MHz |
|  | >=2000 MHz and <2200 MHz | 2200 MHz |
|  | >=2200 MHz and <2400 MHz | 2400 MHz |
| Note 1: For FR2 intra-band non-contiguous CA configurations with non-contiguous UL CA the selected value of maxFsBW is based on applicable frequency separation classes for UL non-contiguous operation in TS 38.101-2 [8],Table 5.3A.4-2. For FR2 intra-band non-contiguous CA configurations without non-contiguous UL CA the selected value of maxFsBW is based on applicable frequency separation classes for DL non-contiguous operation in TS 38.101-2 [8],Table 5.3A.4-2. |  |  |

#### C.2.4.3.2	Determination of test frequencies for a sub-block combination

Editor's note: The number of test points for FR2 intra-band non-contiguous CA configurations is under investigation, e.g. “Low” and “High”, or “Mid”.

1.	Calculate the maxWgap value between sub-blocks for the sub-block combination:

| maxWgap = (maxFsBW – ∑k=1 to NSB SBCBWDL(k)) / (NSB-1) | C.2.4.3.3-Eq1 |
| --- | --- |

2.	Calculate test frequencies for all sub-blocks in the sub-block combination:

If the maxFsBW is smaller than the full bandwidth of the NR band then calculate test frequencies for both Low and High range. Else only for the Low range.

For Low range the test frequencies are calculated such that the lower edge of the lowest component carrier of the lowest frequency sub-block is located at the lower edge of the NR bandwidth. The sub-blocks are separated by the calculated maxWgap in step 1.

For High range the test frequencies are calculated such that the upper edge of the highest component carrier of the highest frequency sub-block is located at the upper edge frequency of the NR bandwidth. The sub-blocks are separated by the calculated maxWgap in step 1.

Within each sub-block the test frequencies and parameters of the sub-block are calculated based on the location of the sub-block and the relevant principles in clause C.2, C3 and C4 for the type of component carriers in the sub-block.

The test frequencies for CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 is calculated without CORESET#0 as specified in C.3.3. CCs with SCS=60 kHz for FR1 and with SCS=240 kHz for FR2 can only be used for NR Intra-band Non-Contiguous CA configurations with mixed numerologies.

#### C.2.4.3.3	Void

## C.2.5	Frequency determination for supplemental uplink

### C.2.5.1	General

The following procedure is used to calculate test frequencies for NR supplemental uplink as described below, where CBWUL refers to the carrier’s UL channel bandwidths and BWUL refers to the band’s total UL bandwidths.

### C.2.5.2	Determination of Low-, Mid- and High-Range for supplemental uplink bands

The following procedure is used to determine the uplink carrier centre frequencies for Low-, Mid- and High-Range for supplemental uplink bands.

| FUL_LowRange = Ceil((FUL_Low + CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.5.2-Eq1 |
| --- | --- |
| FUL_MidRange = Round((FUL_Low + BWUL_Band/2) / ΔFRaster) * ΔFRaster | C.2.5.2-Eq2 |
| FUL_HighRange = Floor((FUL_Low + BWUL_Band – CBWUL/2) / ΔFRaster) * ΔFRaster | C.2.5.2-Eq3 |

## C.2.6	Frequency determination for EN-DC configurations

### C.2.6.1	Determination of test frequencies for EN-DC Inter-band

Test frequencies for EN-DC Intra-band non-contiguous configurations use the Low and High ranges test frequencies for each of the included single carrier E-UTRA and NR bands, and E-UTRA and NR CA configurations in the configuration as specified E-UTRA in TS 36.508 [2], clause 4.3.1 and for NR in clause 4.3.1.1.1 for FR1 bands and in clause 4.3.1.2.1 for FR2 bands.

### C.2.6.2	Determination of test frequencies for EN_DC Intra-band Contiguous CA

#### C.2.6.2.1	General

By default, test frequencies for EN-DC Intra-band Contiguous CA in clause 4.3.1 are specified using the nominal channel spacing between the E-UTRA and NR carrier components as specified in TS 38.101-3 [9], clause 5.4B.1.

The test frequencies for EN_DC Intra-band Contiguous CA is calculated for Low and High ranges for the following two cases:

-	with NR CC at the band edge; and

-	 with E-UTRA CC at the band edge.

In addition to the definition of parameters in clause C.1 the following parameters are used to calculate carrier components (CC) test frequencies for EN_DC Intra-band Contiguous:

| Parameter | Description |
| --- | --- |
| NNR_CC | Number of NR CCs in the EN-DC Intra-band configuration |
| NEUTRA_CC | Number of E-UTRA CCs in the EN-DC Intra-band configuration |
| CCBWNR_DL(i) | Channel bandwidth (MHz) of downlink NR CC(i), where i = 1 to NNR_CC |
| CCBWEUTRA_DL(m) | Channel bandwidth (MHz) of downlink E-UTRA CC(m), where m = 1 to NEUTRA_CC |
| FNR_EUTRA_Channel_Spacing | Nominal channel spacing between adjacent NR and E-UTRA CCs as defined in TS 38.101-3 [9], 5.4B.1. |
| FEUTRA_Channel_Spacing(m) | Nominal channel spacing between E-UTRA adjacent contiguous CC(m) and CC(m+1), where m = 1 to (NEUTRA_CC-1) as defined in TS 36.101 [48], 5.7.1A. |
| FNR_Channel_Spacing(i) | Nominal channel spacing between NR adjacent contiguous CC(i) and CC(i+1), where i = 1 to (NNR_CC-1) as defined in TS 38.101-1 [7] clause 5.4A.1 for FR1 and TS 38.101-2 [8] clause 5.4A.1 for FR2 |
| LCMΔFRaster | Least Common Multiple of NR ΔFRaster and E-UTRA ΔFRaster equals to 300 kHz for E-UTRA ΔFRaster = 100 kHz and NR ΔFRaster = 15kHz, 30kHz and 60kHz. |

#### C.2.6.2.2	Determination of test frequencies for Low-, Mid- and High-Range with NR at band edges

Downlink NR CC(1), lowest frequency CC:

FNR_DL_LowRange_CC(1) is rounded up and FNR_DL_HighRange_CC(1) is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

The NR test frequencies are calculated such that both the NR CC and E-UTRA CC adjacent to each other are located at the NR and E-UTRA frequency raster respectively.

| FNR_DL_LowRange_CC(1) = Ceil((FDL_Low + CCBWNR_DL(1) / 2 + FNR_EUTRA_Channel_Spacing) / LCMΔFRaster) * LCMΔFRaster - FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(1) + CCBWNR_DL(NNR_CC))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.2-Eq1 |
| --- | --- |
| FNR_DL_MidRange_CC(1) = Round((FDL_Low + BWDL/2 – (∑i=1 to NNR_CC CCBWNR_DL (i) + ∑m=1 to NEUTRA_CC CCBWEUTRA_DL(m))/2 + CCBWNR_DL(1))/2) + FNR_EUTRA_Channel_Spacing / LCMΔFRaster) * LCMΔFRaster - FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(1) + CCBWNR_DL(NNR_CC))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.2-Eq2 |
| FNR_DL_HighRange_CC(1) = Floor((FDL_High - CCBWNR_DL(NNR_CC)/2– ∑i=1 to (NNR_CC-1) FNR_Channel_Spacing(i) - FNR_EUTRA_Channel_Spacing) / LCMΔFRaster) * LCMΔFRaster + FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(NEUTRA_CC) + CCBWNR_DL(1))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.2-Eq3 |

Downlink NR CC(2) to CC(NNR_CC), in increasing frequency order:

| FNR_DL_LowRange_CC(k) = FDL_LowRange_CC(k-1) + ∑FNR_Channel_Spacing(k), k=2 to NDL_CC | C.2.6.2.2-Eq4 |
| --- | --- |
| FNR_DL_MidRange_CC(k) = FDL_MidRange_CC(k-1) + ∑FNR_Channel_Spacing(k), k=2 to NDL_CC | C.2.6.2.2-Eq5 |
| FNR_DL_HighRange_CC(k) = FDL_HighRange_CC(k-1) + ∑FNR_Channel_Spacing(k), k=2 to NDL_CC | C.2.6.2.2-Eq6 |

Uplink NR CC(k), k=1 to NCC:

| FNR_UL_LowRange_CC(k) = FNR_DL_LowRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq7 |
| --- | --- |
| FNR_UL_MidRange_CC(k) = FNR_DL_MidRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq8 |
| FNR_UL_HighRange_CC(k) = FNR_DL_HighRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq9 |

Downlink E-UTRA CC(1), lowest frequency CC:

ΔFNR_EUTRA_Channel_Spacing is selected in each formula C.2.6.2.2-Eq10, C.2.6.2.2-Eq11 and C.2.6.2.2-Eq12 selected such that FEUTRA_DL_LowRange_CC(1), FEUTRA_DL_MidRange_CC(1) and FEUTRA_DL_HighRange_CC(1) are located on the E-UTRA band frequency raster.

| FEUTRA_DL_LowRange_CC(1) = FNR_DL_LowRange_CC(NDL_CC) + FNR_EUTRA_Channel_Spacing | C.2.6.2.2-Eq10 |
| --- | --- |
| FEUTRA_DL_MidRange_CC(1) = FNR_DL_MidRange_CC(NDL_CC) + FNR_EUTRA_Channel_Spacing | C.2.6.2.2-Eq11 |
| FEUTRA_DL_HighRange_CC(1) = FNR_DL_HighRange_CC(NDL_CC) - ∑i=1 to (NNR_CC-1) FNR_Channel_Spacing(i) - FNR_EUTRA_Channel_Spacing - ∑i=1 to (NEUTRA_CC-1) FEUTRA_Channel_Spacing(i) | C.2.6.2.2-Eq12 |

Downlink E-UTRA CC(2) to CC(NEUTRA_CC), in increasing frequency order:

| FEUTRA_DL_LowRange_CC(k) = FEUTRA_DL_LowRange_CC(k-1) + ∑FEUTRA_Channel_Spacing(k), k=2 to NEUTRA_CC | C.2.6.2.2-Eq13 |
| --- | --- |
| FEUTRA_DL_MidRange_CC(k) = FEUTRA_DL_MidRange_CC(k-1) + ∑FEUTRA_Channel_Spacing(k), k=2 to NEUTRA_CC | C.2.6.2.2-Eq14 |
| FEUTRA_DL_HighRange_CC(k) = FEUTRA_DL_HighRange_CC(k-1) + ∑FEUTRA_Channel_Spacing(k), k=2 to NEUTRA_CC | C.2.6.2.2-Eq15 |

Uplink E-UTRA CC(k), k=1 to NEUTRA_CC:

| FEUTRA_UL_LowRange_CC(k) = FEUTRA_DL_LowRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq16 |
| --- | --- |
| FEUTRA_UL_MidRange_CC(k) = FEUTRA_DL_MidRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq17 |
| FEUTRA_UL_HighRange_CC(k) = FEUTRA_DL_HighRange_CC(k) - FTx-Rx_separation | C.2.6.2.2-Eq18 |

#### C.2.6.2.3	Determination of test frequencies for Low-, Mid- and High-Range with E-UTRA at band edges

To get the NR carrier on the synchcronisation raster the calculations of the E-UTRA carrier components needs to be based on the location of the NR carrier.

Downlink NR CC(1), lowest frequency CC:

FNR_DL_LowRange_CC(1) is rounded up and FNR_DL_HighRange_CC(1) is rounded down to obey to the minimum guard band according to clause 5.3.3 of TS 38.101-1 [7] and TS 38.101-2 [8].

| FNR_DL_LowRange_CC(1) = Ceil((FDL_Low + ∑m=1 to NEUTRA_CC CCBWEUTRA_DL(m)+CCBWNR_DL(1) / 2 - FNR_EUTRA_Channel_Spacing) / LCMΔFRaster) * LCMΔFRaster + FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(NEUTRA_CC) + CCBWNR_DL(1))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.3-Eq1 |
| --- | --- |
| FNR_DL_MidRange_CC(1) = same formula as C.2.6.2.2-Eq2 | C.2.6.2.3-Eq2 |
| FNR_DL_HighRange_CC(1) = Floor((FDL_High – (∑m=1 to NEUTRA_CC CCBWEUTRA_DL(m) + ∑i=1 to NNR_CC CCBWNR_DL(i)) +CCBWNR_DL(1) / 2 + FNR_EUTRA_Channel_Spacing) / LCMΔFRaster) * LCMΔFRaster - FNR_EUTRA_Channel_Spacing, where FNR_EUTRA_Channel_Spacing = Round((CCBWEUTRA_DL(1) + CCBWNR_DL(NNR_CC))/(2*ΔFRaster))* ΔFRaster | C.2.6.2.3-Eq3 |

Downlink NR CC(2) to CC(NNR_CC), in increasing frequency order:

| FNR_DL_LowRange_CC(k) = same formula as C.2.6.2.2-Eq4 | C.2.6.2.3-Eq4 |
| --- | --- |
| FNR_DL_MidRange_CC(k) = same formula as C.2.6.2.2-Eq5 | C.2.6.2.3-Eq5 |
| FNR_DL_HighRange_CC(k) = same formula as C.2.6.2.2-Eq6 | C.2.6.2.3-Eq6 |

Uplink NR CC(k), k=1 to NCC:

| FNR_UL_LowRange_CC(k) = same formula as C.2.6.2.2-Eq7 | C.2.6.2.3-Eq7 |
| --- | --- |
| FNR_UL_MidRange_CC(k) = same formula as C.2.6.2.2-Eq8 | C.2.6.2.3-Eq8 |
| FNR_UL_HighRange_CC(k) = same formula as C.2.6.2.2-Eq9 | C.2.6.2.3-Eq9 |

Downlink E-UTRA CC(1), lowest frequency CC:

| FEUTRA_DL_LowRange_CC(1) = FNR_DL_LowRange_CC(1) - FNR_EUTRA_Channel_Spacing - ∑m=1 to (NEUTRA_CC - 1) FEUTRA_Channel_Spacing(m) | C.2.6.2.3-Eq10 |
| --- | --- |
| FEUTRA_DL_MidRange_CC(1) = same formula as C.2.6.2.2-Eq11 | C.2.6.2.3-Eq11 |
| FEUTRA_DL_HighRange_CC(1) = FNR_DL_HighRange_CC(NDL_CC) + ∑i=1 to (NNR_CC-1) FNR_Channel_Spacing(i) + FNR_EUTRA_Channel_Spacing + ∑i=1 to (NEUTRA_CC-1) FEUTRA_Channel_Spacing(i) | C.2.6.2.3-Eq12 |

Downlink E-UTRA CC(2) to CC(NEUTRA_CC), in increasing frequency order:

| FEUTRA_DL_LowRange_CC(k) = same formula as C.2.6.2.2-Eq13 | C.2.6.2.3-Eq13 |
| --- | --- |
| FEUTRA_DL_MidRange_CC(k) = same formula as C.2.6.2.2-Eq14 | C.2.6.2.3-Eq14 |
| FEUTRA_DL_HighRange_CC(k) = same formula as C.2.6.2.2-Eq15 | C.2.6.2.3-Eq15 |

Uplink E-UTRA CC(k), k=1 to NEUTRA_CC:

| FEUTRA_UL_LowRange_CC(k) = same formula as C.2.6.2.2-Eq16 | C.2.6.2.3-Eq16 |
| --- | --- |
| FEUTRA_UL_MidRange_CC(k) = same formula as C.2.6.2.2-Eq17 | C.2.6.2.3-Eq17 |
| FEUTRA_UL_HighRange_CC(k) = same formula as C.2.6.2.2-Eq18 | C.2.6.2.3-Eq18 |

### C.2.6.3	Determination of test frequencies for EN-DC Intra-band non-contiguous

Test frequencies for EN-DC Intra-band non-contiguous configurations use the Low and High ranges test frequencies for each of the included single carrier E-UTRA and NR bands, and E-UTRA and NR CA configurations in the configuration as specified E-UTRA in TS 36.508 [2], clause 4.3.1 and for NR in clause 4.3.1.1.1 for FR1 bands and in clause 4.3.1.2.1 for FR2 bands.

The following cases of test frequencies are specified for relevant E-UTRA and NR CBW combinations, and NR SCS:

-	Low with maxWgap (NR - E-UTRA): NR Low range and E-UTRA High range

-	High with maxWgap (E-UTRA - NR): NR High range and E-UTRA Low range

# C.3	Determination of SSB and CORESET#0

## C.3.1	General

The requirements to be met and the principles used for determining the SSB and CORESET#0 for a PCell are:

1.	The complete SSB and CORESET#0 shall be within the carrier’s channel bandwidth.

2.	The SSB centre frequency (SSref) shall be on the synchronisation raster.

3.	The SSB shall be kept as close as possible to the carrier’s lower edge centre frequency.

4.	CORESET#0 configuration is selected using lowest number of RBs and symbols in applicable table in TS 38.213 [22], clause 13.

5.	The first SSB subcarrier shall be aligned with the defined resource grid given by SCS indicated by subCarrierSpacingCommon in the MIB.

## C.3.2	Determination of SSB, CORESET#0 and signalling parameters for a PCell

Calculation of SSB and CORESET#0 parameters is limited to FR1 carriers with SCS=15 kHz or SCS=30 kHz, and to FR2 carriers with SCS=60 kHz or SCS=120 kHz. CORESET#0 is required for a carrier to be used as a PCell.

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	Determine SSB and CORESET#0:

1a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Min

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

1b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-1 [7] and TS 38.101-2 [8].

1c.	Calculate the FSSref for the selected GSCN value in step 1b in accordance to TS 38.101-1 [7], clause 5.4.3.1 for FR1 and TS 38.101-2 [7], clause 5.4.3.1 for FR2.

1d.	Calculate the frequency FoffsetToPointA, which is the lowest subcarrier of the lowest resource block with the subcarrier spacing being a multiple of resource blocks expressed in terms of common PRB size and overlaps with the SS/PBCH block subcarrier 0 of the first resource block of the SS/PBCH block, FSSBlow (TS 38.211 [3], clause 7.4.3.1):

FSSBlow = FSSref - BWSSB / 2

FoffsetToPointA = CRBsize * Floor((FSSBlow - FcarrierLow) / CRBsize) + FcarrierLow

1e.	Calculate the maximum OffsetRBs value with FCORESET0Low >= FcarrierLow:

Max_OffsetRBs = (FoffsetToPointA - FcarrierLow) / CRBsize

1f.	Select the largest valid OffsetRBs value equal or smaller than the calculated max value, Max_OffsetRBs in step 1e within the applicable values for the carrier in TS 38.213 [4], table 13-1 to 13-10 limited to the table indexes with number of RBs and number of symbols  equal to the minimum value of in the table and minimum value of  for the selected .If a valid OffsetRBs value is found, then continue from step 1g.

If no valid OffsetRBs value is found, then select the next valid GSCN with FSSref <= FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1f.

If no valid OffsetRBs value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.3 and the procedure is completed.

1g.	Calculate kSSB

kSSB = (FSSBlow - FoffsetToPointA) / {15 kHz for FR1, subCarrierSpacingCommon (MIB) for FR2} (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / {15 kHz for FR1; subCarrierSpacingCommon (MIB) for FR2}. kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 2.

If kSSB is not an integer value or kSSB MOD N <> 0, then select the next valid GSCN with FSSref < FSSref_Max within the valid GSCN range for the carrier and repeat steps 1b to 1g. 

If N > 1 and no valid kSSB value found within the valid GSCN range for the currently selected carrier frequency Fcarrier then shift Fcarrier up by ΔFRaster for Low range; or down by ΔFRaster for Mid, Mid-Low, Mid-High and High ranges and repeat steps 1a to 1g for a maximum shift of 3 * ΔFRaster (see clause C.3.1, Note 1). Cases requiring more than 3*ΔFRaster shifts shall be analyzed individually and, if eventually agreed, marked in section 4.3 to have been obtained applying certain exceptions over this section C.3.2. In case of an exception, for Mid, Mid-Low and Mid-High, shifting frequency up can also be considered to achieve the frequency closer to the ideal case.

If no valid kSSB value found within the valid GSCN range then will the carrier not be possible to use as PCell and FSSref, kSSB, FPointA, OffsetToCarrier and OffsetToPointA are calculated as described in clause C.3.2 and the procedure is completed.

2.	Determine OffsetToCarrier

Select offsetToCarrier value for the carrier in accordance to Table C.3.2-1.

Table C.3.2-1: Downlink and uplink offsetToCarrier default values for different frequency ranges

| Frequency range | Downlink offsetToCarrier | Uplink offsetToCarrier |
| --- | --- | --- |
| Low range | 0 | 0 |
| Mid range | 102 | 504 |
| High range | 504 | 6 |
| Mid-Low range | 12 | 36 |
| Mid-High range | 24 | 114 |
| Note: Different values of offsetToCarrier have been selected for Low, Mid-Low, Mid, Mid-High and High ranges to achieve enhanced test coverage of the offsetToCarrier range of values. |  |  |

In case low, mid and high range are exactly the same frequency, use offsetToCarrier associated to low range.

2b.	Determine FpointA:

FPointA = FcarrierLow - offsetToCarrier * PRBsize

3.	Calculate ΔFOffsetCORESET-0-Carrier:

The ΔFOffsetCORESET-0-Carrier value is used to calculate the Offset Carrier CORESET#0 parameter included in the test frequency tables in sub-clauses 4.3.1 and 6.2.3.

ΔFOffsetCORESET-0-Carrier = FoffsetToPointA - OffsetRB * CRBsize - FcarrierLow

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to the index associated with the selected OffsetRBs value in the applicable table, 13-1 to 13-10, in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | (FOffsetToPointA - FPointA) / (12*{15 kHz for FR1; 60 kHz for FR2}). The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | Set to value calculated in step 2a. The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

## C.3.3	Determination of SSB and signalling parameters for a carrier without CORESET#0

The following procedure is used for calculation of SSB and signalling parameters for a carrier without a CORESET#0.

1. 	Calculate FSSref, kSSB and FPointA with the SSB lowest subcarrier at the carrier’s lowest subcarrier (ΔFOffsetSSB-Carrier in Figure C.1-1 = 0):

FSSref = FcarrierLow + BWSSB / 2

kSSB = {31 for FR1; 15 for FR2} indicating that no CORESET#0 is present for the carrier (TS 38.213 [4], clause 13).

offsetToCarrier = target value for offsetToCarrier dependent on frequency range as specified in Table C.3.1-1.

FPointA = FcarrierLow - offsetToCarrier * PRBsize

2.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. For the case kSSB > 15 the extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [22]. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | Set to 0 indicating that no CORESET#0 exist (TS 38.213 [22], clause 13). The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| searchSpaceZero | Set to 0 indicating that no CORESET#0 exist (TS 38.213 [22], clause 13). The IE field searchSpaceZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToCarrier | Set to offsetToCarrier target value selected in step 1. |

# C.4	Determination of SSB and CORESET#0 for RRM testing with SSB SCS 120 kHz and 240 kHz

## C.4.1	General

The requirements to be met and the principles used for determining the SSB and CORESET#0 for a PCell used in RRM test cases are:

1.	The complete SSB and CORESET#0 shall be within the carrier’s channel bandwidth.

2.	The SSB centre frequency (SSref) shall be on the synchronisation raster.

3.	The SSB shall be kept as close as possible to the carrier’s lower edge centre frequency.

4.	The CORESET#0 configuration is selected using 24 RBs and OffsetRBs = 0 according to Table 13-8 and Index 0 for SCSSSB =120 KHz and Table 13-10 and Index 0 for SCSSSB =240 kHz.

5.	The first SSB subcarrier shall be aligned with the defined resource grid given by SCS indicated by subCarrierSpacingCommon in the MIB.

## C.4.2	Determination of SSB, CORESET#0 and signalling parameters

The following procedure is used to determine an SSB on the synchronisation raster (GSCN) and a CORESET#0 configuration (kSSB, OffsetRBs = 0 and OffsetToPointA) as close as possible to the carrier’s lower edge. See figure C1-1 and clause C.1 for definition of parameters referenced in the procedure.

1.	The target test frequencies for Low, Mid and High ranges are calculated as described in clause C.2.1.1.

For each of Low, Mid and High ranges do:

2.	Determine SSB and CORESET#0:

2a.	Calculate the lower of FSSref, FSSref_Min, correspondent to SSB lowest subcarrier being at the same frequency as the carrier’s lowest subcarrier; and the higher limit of FSSref, FSSref_Max, correspondent to SSB highest subcarrier being at the same frequency as the carrier’s highest subcarrier FSSref_Min

FcarrierLow = see formula for FcarrierLow in Table C.1-1

FSSref_Min = FcarrierLow + CRBsize * OffsetRBs,min + BWSSB / 2

FSSref_Max = FcarrierLow + ΔFcarrierBandwidth - BWSSB / 2

2b.	Calculate GSCNMIN correspondent to FSSref_Min in accordance to TS 38.101-2 [7], clause 5.4.3.1 and select the closest valid GSCN value with GSCN >= GSCNMIN for the carrier in according to the carrier’s synchronisation raster as specified in clause 5.4.3.3 in TS 38.101-2 [8].

2c.	Calculate the FSSref for the selected GSCN value in step 2b in accordance to TS 38.101-2 [7], clause 5.4.3.1 for FR2.

2d.	Calculate the frequency FSSBlow and shift the carrier frequency to achieve FcarrierLow equal or as close as possible FSSBlow on the carrier’s frequency raster.

FSSBlow = FSSref - BWSSB / 2

Fcarrier = calculated using the formula in clause C.2.1.1 with FDL_Low = FSSBlow

FcarrierLow = see formula for FcarrierLow in Table C.1-1 with new value of Fcarrier

2e.	Calculate kSSB

kSSB = (FSSBlow - FcarrierLow) / subCarrierSpacingCommon (MIB, FR2) (TS 38.211 [3], clause 7.4.3.1).

N = SCSSSB / subCarrierSpacingCommon (MIB, FR2). kSSB MOD N <> 0 indicates that the SSB subcarriers are not aligned with the resource grid given by the SCS indicated by subCarrierSpacingCommon in the MIB.

If kSSB is an integer and kSSB MOD N = 0, then continue from step 3 else modify the carrier frequency to get valid value of kSSB and kSSB MOD N = 0.

3.	Calculate Point A frequency, ΔFoffsetToCarrier and ΔFOffsetToPointA:
The CORESET#0 configuration is selected using 24 RBs and OffsetRBs = 0 according to Table 13-8 and Index 0 for SCSSSB =120 KHz and Table 13-10 and Index 0 for SCSSSB =240 kHz (see C.4.1). This means that FOffsetToPointA = FcarrierLow. By selecting Point A equal to FcarrierLow this gives:

FPointA = FcarrierLow

ΔFoffsetToCarrier = 0

ΔFOffsetToPointA = 0

4.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| ssb-SubcarrierOffset | Set to the 4 least significant bits of kSSB. The IE field ssb-SubcarrierOffset is signalled in the MIB. |
| controlResourceSetZero | 0 (Index=0 in table 13-8 for SCSSSB =120 KHz and table 13-10 for SCSSSB =240 KHz in TS 38.213 [22]. The IE field controlResourceSetZero is signalled in the IE pdcch-ConfigSIB1 in the MIB. |
| absoluteFrequencySSB | Set to FSSref expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| absoluteFrequencyPointA | Set to FPointA expressed in ARFCN as defined in TS 38.101-1 [15] and TS 38.101-2 [39], clause 5.4.2. |
| offsetToPointA | 0 The IE field offsetToPointA is signalled in IE FrequencyInfoDL-SIB. |
| offsetToCarrier | 0 The IE field offsetToCarrier is signalled in IE SCS-SpecificCarrier. |

# C.5	Determination of test frequencies and S-SSB for V2X bands

## C.5.1	General

Figure C.5.1-1 shows carrier and S-SSB on V2X bands and related parameters.

Figure C.5.1-1: location of S-SSB within a channel

The parameters referenced in Figure C.5.1-1 are defined in Table C.5.1-1.

Table C.5.1-1: Definition of parameters for V2X bands

| Parameter | Description |
| --- | --- |
| FSL_PointA | Reference Point A frequency. |
| FSL_carrier | FSL_carrier is the centre frequency of a carrier corresponding to its NR-ARFCN value. |
| FSL_carrierLow | FSL_carrierLow is the centre frequency of lowest subcarrier of the carrier. FSL_carrierLow = FSL_carrier - 12 * SCSSL_Carrier * (NRB / 2) with NRB according to section 5.3E.1 of TS 38.101-1 for the channel bandwidth of the carrier. |
| ΔFSL_carrierBandwidth | ΔFSL_carrierBandwidth is the carrier’s channel bandwidth as provided in carrierBandwidth to the UE (sl-SCS-SpecificCarrierList-r16). |
| ΔFSL_OffsetToCarrier | ΔFSL_OffsetToCarrier is the frequency offset between Point A and the lower edge of the carrier. ΔFSL_OffsetToCarrier = offsetToCarrier * CRBsize, where CRBsize according to the subcarrier spacing of the carrier. offsetToCarrier is signalled to the UE (sl-SCS-SpecificCarrierList-r16). |
| FSL_SSref | Centre frequency of subcarrier with index 66 in the S-SS/PSBCH block, corresponding to NR-ARFCN value signalled to the UE by sl-AbsoluteFrequencySSB-r16. |

Additional parameters used in this annex are defined in Table C.5.1-2.

Table C.5.1-2: Definition of additional parameters used in section C.5.2

| SCSSL_Carrier | subcarrier spacing for the carrier (sl-SCS-SpecificCarrierList-r16): FR1: 15kHz, 30kHz or 60kHz according to TS 38.101-1 [7] Table 5.3.5-1 |
| --- | --- |
| CRBsize | Common Resource Block size = 12 * SCSSL_Carrier. |
| FSL_Low | Lowest frequency of the frequency range of the V2X band as defined in clause 5.2E.1 of TS 38.101-1 [7]. |
| FSL_High | Highest frequency of the frequency range of the V2X band as defined in clause 5.2E.1 of TS 38.101-1 [7]. |
| ΔFSL_Raster | Frequency raster of the band as defined in clause 5.4E.2.3 of TS 38.101-1 [7]. |
| BWSL | Bandwidth of V2X operation frequency range of the band. |
| CBWSL | UE V2X operation channel bandwidth (MHz) of the carrier according to section 5.3E.1 of TS 38.101-1 [7]. |
| BWSL_SSB | Bandwidth of the SSB. BWSL_SSB = 11 * SCSSL_Carrier * 20 |

## C.5.2	Determination of test frequencies and S-SSB for V2X bands

The carrier test frequencies are determined considering the channel raster according to clause 5.4.2.3 in TS 38.101-1 [7] for FR1.

The complete S-SSB shall be within a bandwidth of the SL BWP. The subcarrier with index 0 in the S-SSB shall be aligned with a subcarrier with index 0 in an RB of the SL BWP.

1.	Determine sidelink carrier centre frequencies and the frequency of the carrier’s lowest subcarrier:

| FSL_LowRange = Ceil((FSL_Low + CBWSL/2) / ΔFSL_Raster) * ΔFSL_Raster | C.5.2-Eq1 |
| --- | --- |
| FSL_MidRange = Round((FSL_Low + BWSL/2) / ΔFSL_Raster_) * ΔFSL_Raster | C.5.2-Eq2 |
| FSL_HighRange = Floor((FSL_High – CBWSL/2) / ΔFSL_Raster) * ΔFSL_Raster | C.5.2-Eq3 |
| FSL_carrierLow = see formula for FSL_carrierLow in Table C.1-1 |  |

2.	Determine OffsetToCarrier

Select offsetToCarrier value for the carrier in accordance to Table C.5.2-1.

Table C.5.2-1: Sidelink offsetToCarrier default values for different frequency ranges

| Frequency range | Sidelink offsetToCarrier |
| --- | --- |
| Low range | 0 |
| Mid range | 504 |
| High range | 6 |
| Mid-Low range | 36 |
| Mid-High range | 114 |
| Note: Different values of offsetToCarrier have been selected for Low, Mid-Low, Mid, Mid-High and High ranges to achieve enhanced test coverage of the offsetToCarrier range of values. |  |

3.	Determine FSL_PointA:

FSL_PointA = FSL_carrierLow - offsetToCarrier * CRBsize

4.	Determine the centre frequencies of S-SSB for the lowest, mid and highest possible location of FSL_SSref:

FSL_SSref_Low = FcarrierLow + BWSL_SSB / 2

FSL_SSref_Mid = FcarrierLow + (Floor(NRB/2) – 5) * CRBsize + BWSSB / 2

FSL_SSref_High = FcarrierLow + (NRB – 11) * CRBsize + BWSSB / 2

5.	Calculate signalling parameters:

| IE field | Value |
| --- | --- |
| sl-AbsoluteFrequencySSB-r16 | Set to FSL_SSref expressed in ARFCN as defined in TS 38.101-1 [15] clause 5.4E.2. |
| sl-AbsoluteFrequencyPointA-r16 | Set to FSL_PointA expressed in ARFCN as defined in TS 38.101-1 [15] clause 5.4E.2. |
| offsetToCarrier | Set to value calculated in step 2. The IE field offsetToCarrier is signalled in IE sl-SCS-SpecificCarrierList-r16. |

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-12 | RAN5#77 | R5-176995 | - | - | - | TP on clauses of test equipment requirement in 38.508-1 | 0.1.0 |
| 2017-12 | RAN5#77 | R5-176779 | - | - | - | Add references | 0.1.0 |
| 2017-12 | RAN5#77 | R5-176917 | - | - | - | Introduce general chapter for generic procedures | 0.1.0 |
| 2017-12 | RAN5#77 | R5-176918 | - | - | - | Add generic procedures RRC_IDLE and RRC_CONNECTED | 0.1.0 |
| 2017-12 | RAN5#77 | R5-176920 | - | - | - | Introduce RRC chapters | 0.1.0 |
| 2018-01 | RAN5#1-5G-NR Adhoc | R5-180066 | - | - | - | Definition of downlink physical layer parameters for NR | 0.2.0 |
| 2018-03 | RAN5#78 | R5-181697 | - | - | - | Addition of the environmental information into TS 38.508-1 | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180265 | - | - | - | Introduce chapter for reference configurations | 0.3.0 |
| 2018-03 | RAN5#78 | R5-181311 | - | - | - | Update the general chapter | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180382 | - | - | - | Update RRCReconfiguration | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180383 | - | - | - | Add draft RRC messages | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180577 | - | - | - | Update chapter for test frequencies | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180709 | - | - | - | Add CellGroupConfig | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180773 | - | - | - | Add radioBearerConfig | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180775 | - | - | - | Add draft Radio resource control information elements | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180966 | - | - | - | Update RRC Connected state | 0.3.0 |
| 2018-03 | RAN5#78 | R5-181035 | - | - | - | Update RRC IDLE state | 0.3.0 |
| 2018-03 | RAN5#78 | R5-180253 | - | - | - | Revised WID on: UE Conformance Test Aspects - 5G system with NR and LTE | 0.3.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181812 | - | - | - | Update Radio resource control information elements | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182109 | - | - | - | Update CellGroupConfig | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182064 | - | - | - | Update radioBearerConfig | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182062 | - | - | - | Update MIB | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182063 | - | - | - | Introduce radio conditions | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181786 | - | - | - | Update RRCReconfiguration | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181971 | - | - | - | Add Other information elements | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182065 | - | - | - | Update chapter 4.5.1 General | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181813 | - | - | - | Update RRC IDLE state | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182066 | - | - | - | Update RRC CONNECTED state | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182110 | - | - | - | Text proposal to add clause 4.4 reference system configurations to TS 38.508-1 | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182067 | - | - | - | TP for definition of physical channel allocations in 38.508-1 | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182091 | - | - | - | TP for clauses of signal level | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181972 | - | - | - | TP for updating of Downlink physical layer parameters | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181893 | - | - | - | Addition of UE capability information elements | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181973 | - | - | - | TP for adding Mid channel BW definition in TS 38.508-1 | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-181974 | - | - | - | Addition of SRB3 | 0.4.0 |
| 2018-04 | RAN5#1-5G-NR Adhoc | R5-182068 | - | - | - | Update MeasConfig information elements | 0.4.0 |
| 2018-05 | RAN5#79 | R5-183082 | - | - | - | Update radio resource control information elements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182288 | - | - | - | TP for updating of downlink physical layer parameters in 38.508-1 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182349 | - | - | - | Corrections to clause 4.4 reference system configurations | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182792 | - | - | - | TP for clauses of Supported Channels for a NR cell | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183218 | - | - | - | pCR update chapter for test frequencies - EN-DC | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183234 | - | - | - | TP for updating of physical channel allocation part in 38.508-1 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183256 | - | - | - | pCR update chapter for test frequencies - FR1 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183916 | - | - | - | TP for Annex A in TS 38.508-1 and adding a set of Connection Diagrams | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183920 | - | - | - | Introduction of Environmental conditions for FR1 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182249 | - | - | - | Add reference to NR cell table | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183210 | - | - | - | Update PDCCH | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182312 | - | - | - | Update chapter 4.5.1 General | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182313 | - | - | - | Update RRC CONNECTED state | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183087 | - | - | - | Addition of new RRCReconfiguration definition for AM/UM bearers | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183088 | - | - | - | Updates to UE capability information elements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183250 | - | - | - | Updates to UE capability information elements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183083 | - | - | - | Update RACH | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183084 | - | - | - | Update ARFCN | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183211 | - | - | - | Update BWP-UplinkDedicated | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183212 | - | - | - | Update serving cell | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183214 | - | - | - | Update RadioBearerConfig | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183215 | - | - | - | Update RRCReconfiguration | 1.0.0 |
| 2018-05 | RAN5#79 | R5-182381 | - | - | - | Update MIB | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183090 | - | - | - | Update RRCReconfiguration for measurements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183264 | - | - | - | Corrections to clause 4.5 | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183249 | - | - | - | Correction to the Table CellGroupConfig | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183255 | - | - | - | Update of FR1 signal levels | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183216 | - | - | - | Update CellGroupConfig and some related information elements | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183086 | - | - | - | Update CSI-MeasConfig | 1.0.0 |
| 2018-05 | RAN5#79 | R5-183260 | - | - | - | Update some information elements related to MeasConfig | 1.0.0 |
| 2018-06 | RAN#80 | RP-181207 | - | - | - | put under revision control as v15.0.0 with small editorial changes | 15.0.0 |
| 2018-09 | RAN#81 | R5-184087 | 0004 | - | F | Update chapter 3 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184297 | 0012 | - | F | Addition of Mid channel bandwidth definition for several missing bands | 15.1.0 |
| 2018-09 | RAN#81 | R5-184327 | 0014 | - | F | Adding condition for CP-OFDM waveform | 15.1.0 |
| 2018-09 | RAN#81 | R5-184347 | 0019 | - | F | Modified RRC_IDLE procedure to allow multi PDN configuration throughout the test case | 15.1.0 |
| 2018-09 | RAN#81 | R5-184471 | 0044 | - | F | Introduction of test frequencies for NR band n77 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184472 | 0045 | - | F | Introduction of test frequencies for NR band n78 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184473 | 0046 | - | F | Introduction of test frequencies for NR band n79 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184474 | 0047 | - | F | Introduction of test frequencies for NR band n257 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184475 | 0048 | - | F | Introduction of test frequencies for NR band n258 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184476 | 0049 | - | F | Introduction of test frequencies for NR band n260 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184477 | 0050 | - | F | Introduction of test frequencies for NR band n261 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184599 | 0056 | - | F | Add IE SS-RSSI-Measurement | 15.1.0 |
| 2018-09 | RAN#81 | R5-184617 | 0059 | - | F | Update MIB | 15.1.0 |
| 2018-09 | RAN#81 | R5-184630 | 0072 | - | F | Editorial Update in clause 4.6.3 | 15.1.0 |
| 2018-09 | RAN#81 | R5-184783 | 0079 | - | F | Introduce 5GMM messages | 15.1.0 |
| 2018-09 | RAN#81 | R5-184785 | 0080 | - | F | Introduce 5GSM messages | 15.1.0 |
| 2018-09 | RAN#81 | R5-184806 | 0081 | - | F | Mid test CH BW for n71 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185028 | 0002 | 1 | F | Add SRB1 and SRB2 with NR PDCP | 15.1.0 |
| 2018-09 | RAN#81 | R5-185029 | 0003 | 1 | F | Update serving cell | 15.1.0 |
| 2018-09 | RAN#81 | R5-185030 | 0005 | 1 | F | Introduce SA RRC messages | 15.1.0 |
| 2018-09 | RAN#81 | R5-185031 | 0006 | 1 | F | Correct IE FrequencyInfoDL | 15.1.0 |
| 2018-09 | RAN#81 | R5-185032 | 0007 | 1 | F | Introduce SA system information blocks | 15.1.0 |
| 2018-09 | RAN#81 | R5-185033 | 0008 | 1 | F | Introduce SA other information elements | 15.1.0 |
| 2018-09 | RAN#81 | R5-185035 | 0013 | 1 | F | Correct IE GSCN-ValueNR | 15.1.0 |
| 2018-09 | RAN#81 | R5-185036 | 0017 | 1 | F | Update of FR1 signal levels | 15.1.0 |
| 2018-09 | RAN#81 | R5-185037 | 0022 | 1 | F | Addition of IP Connectivity check procedure | 15.1.0 |
| 2018-09 | RAN#81 | R5-185038 | 0053 | 1 | F | Introduce SA radio resource control information elements | 15.1.0 |
| 2018-09 | RAN#81 | R5-185039 | 0054 | 1 | F | Update IE PhysicalCellGroupConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185040 | 0055 | 1 | F | Introduce cell configurations and timer tolerances chapter headers | 15.1.0 |
| 2018-09 | RAN#81 | R5-185041 | 0057 | 1 | F | Add IE SSB-MTC | 15.1.0 |
| 2018-09 | RAN#81 | R5-185042 | 0058 | 1 | F | Update BWP | 15.1.0 |
| 2018-09 | RAN#81 | R5-185043 | 0060 | 1 | F | Update PDSCH-Config | 15.1.0 |
| 2018-09 | RAN#81 | R5-185044 | 0062 | 1 | F | Update PUCCH and PUSCH configuration | 15.1.0 |
| 2018-09 | RAN#81 | R5-185045 | 0063 | 1 | F | Update RACH configuration | 15.1.0 |
| 2018-09 | RAN#81 | R5-185046 | 0065 | 1 | F | Update CellGroupConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185047 | 0066 | 1 | F | Update CSI-MeasConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185048 | 0067 | 1 | F | Update MeasConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185049 | 0068 | 1 | F | Update other information elements | 15.1.0 |
| 2018-09 | RAN#81 | R5-185050 | 0070 | 1 | F | Update RadioBearerConfig | 15.1.0 |
| 2018-09 | RAN#81 | R5-185051 | 0073 | 1 | F | Specifying content for MeasResultSCG-Failure | 15.1.0 |
| 2018-09 | RAN#81 | R5-185052 | 0075 | 1 | F | Editorial correction to band representation of non-contiguous EN-DC band combination | 15.1.0 |
| 2018-09 | RAN#81 | R5-185053 | 0076 | 1 | F | Correction to RLC-Config IE | 15.1.0 |
| 2018-09 | RAN#81 | R5-185054 | 0077 | 1 | F | Correction to RadioBearerConfig-DRB | 15.1.0 |
| 2018-09 | RAN#81 | R5-185055 | 0078 | 1 | F | Corrections and updates to BandCombinationList and Feature Set IEs | 15.1.0 |
| 2018-09 | RAN#81 | R5-185056 | 0084 | 1 | F | Corrections and updates to UE Capability IEs | 15.1.0 |
| 2018-09 | RAN#81 | R5-185085 | 0087 | - | F | Addition of UM condition to RLC-Bearer-Config IE | 15.1.0 |
| 2018-09 | RAN#81 | R5-185133 | 0086 | 1 | F | Correction of clause 4.3.3.2.3 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185163 | 0018 | 1 | F | Modified RRC_Connected procedure for Multi PDN throughout the test case. | 15.1.0 |
| 2018-09 | RAN#81 | R5-185165 | 0020 | 1 | F | Update EN-DC Generic Procedure Parameter for Multi-PDN addition throughout Test Case | 15.1.0 |
| 2018-09 | RAN#81 | R5-185168 | 0082 | 1 | F | Introduction of OTA signalling test environment | 15.1.0 |
| 2018-09 | RAN#81 | R5-185171 | 0009 | 2 | F | Updates to PDCCH and SearchSpace configurations | 15.1.0 |
| 2018-09 | RAN#81 | R5-185173 | 0016 | 1 | F | Test Frequencies | 15.1.0 |
| 2018-09 | RAN#81 | R5-185177 | 0051 | 1 | F | Introduction of test frequencies for signalling testing in clause 6 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185250 | 0023 | 1 | F | Introduction of test frequencies for NR band n1 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185251 | 0024 | 1 | F | Introduction of test frequencies for NR band n2 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185252 | 0025 | 1 | F | Introduction of test frequencies for NR band n3 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185253 | 0026 | 1 | F | Introduction of test frequencies for NR band n5 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185254 | 0027 | 1 | F | Introduction of test frequencies for NR band n7 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185255 | 0028 | 1 | F | Introduction of test frequencies for NR band n8 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185256 | 0029 | 1 | F | Introduction of test frequencies for NR band n12 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185257 | 0030 | 1 | F | Introduction of test frequencies for NR band n20 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185258 | 0031 | 1 | F | Introduction of test frequencies for NR band n25 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185259 | 0032 | 1 | F | Introduction of test frequencies for NR band n28 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185260 | 0033 | 1 | F | Introduction of test frequencies for NR band n34 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185261 | 0034 | 1 | F | Introduction of test frequencies for NR band n38 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185262 | 0035 | 1 | F | Introduction of test frequencies for NR band n39 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185263 | 0036 | 1 | F | Introduction of test frequencies for NR band n40 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185264 | 0037 | 1 | F | Update of test frequencies for NR band n41 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185265 | 0038 | 1 | F | Introduction of test frequencies for NR band n51 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185266 | 0039 | 1 | F | Introduction of test frequencies for NR band n66 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185267 | 0040 | 1 | F | Introduction of test frequencies for NR band n70 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185268 | 0041 | 1 | F | Update of test frequencies for NR band n71 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185269 | 0042 | 1 | F | Introduction of test frequencies for NR band n75 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185270 | 0043 | 1 | F | Introduction of test frequencies for NR band n76 | 15.1.0 |
| 2018-09 | RAN#81 | R5-185443 | 0052 | 1 | F | Correction to power level for FR1 RF tests | 15.1.0 |
| 2018-09 | RAN#81 | R5-185557 | 0085 | 1 | F | FR2_UE_BeamlockProcedure_38.508-1 | 15.1.0 |
| 2018-12 | RAN#82 | R5-186453 | 0239 | - | F | Updates to clause 4.3.3, physical channel allocations | 15.2.0 |
| 2018-12 | RAN#82 | R5-186457 | 0240 | - | F | Correction to E-UTRA test frequency for intra-band contiguous configuration for band 41 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186468 | 0241 | - | F | E-UTRA test frequencies for EN-DC intra-band contiguous configurations for band 71 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186491 | 0245 | - | F | Update chapter 4.5 for RF connected procedure | 15.2.0 |
| 2018-12 | RAN#82 | R5-186508 | 0249 | - | F | FR2 UE and TE radiated connection diagram | 15.2.0 |
| 2018-12 | RAN#82 | R5-186575 | 0251 | - | F | Update IE ServingCellConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186612 | 0252 | - | F | Add CounterCheck | 15.2.0 |
| 2018-12 | RAN#82 | R5-186613 | 0253 | - | F | Update DLInformationTransfer | 15.2.0 |
| 2018-12 | RAN#82 | R5-186641 | 0255 | - | F | Update IE SchedulingRequestResourceConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186665 | 0258 | - | F | Update LocationMeasurementIndication | 15.2.0 |
| 2018-12 | RAN#82 | R5-186666 | 0259 | - | F | Update MeasurementReport | 15.2.0 |
| 2018-12 | RAN#82 | R5-186677 | 0261 | - | F | Resubmission of update to 38.508 for mid channel bandwidth | 15.2.0 |
| 2018-12 | RAN#82 | R5-186682 | 0262 | - | F | Update MobilityFromNRCommand | 15.2.0 |
| 2018-12 | RAN#82 | R5-186691 | 0264 | - | F | Update Paging | 15.2.0 |
| 2018-12 | RAN#82 | R5-186692 | 0265 | - | F | Update RRCReestablishment | 15.2.0 |
| 2018-12 | RAN#82 | R5-186714 | 0267 | - | F | Update RRCReject | 15.2.0 |
| 2018-12 | RAN#82 | R5-186719 | 0268 | - | F | Updates related to introduction of test frequencies | 15.2.0 |
| 2018-12 | RAN#82 | R5-186722 | 0271 | - | F | Update SecurityAlgorithmConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186723 | 0272 | - | F | Updates to MeasResults | 15.2.0 |
| 2018-12 | RAN#82 | R5-186734 | 0273 | - | F | Update RRCRelease | 15.2.0 |
| 2018-12 | RAN#82 | R5-186744 | 0274 | - | F | Update RRCResume | 15.2.0 |
| 2018-12 | RAN#82 | R5-186825 | 0279 | - | F | Correction of test frequencies for NR band n1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186826 | 0280 | - | F | Correction of test frequencies for NR band n2 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186827 | 0281 | - | F | Correction of test frequencies for NR band n3 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186828 | 0282 | - | F | Correction of test frequencies for NR band n5 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186829 | 0283 | - | F | Correction of test frequencies for NR band n7 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186830 | 0284 | - | F | Correction of test frequencies for NR band n8 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186831 | 0285 | - | F | Correction of test frequencies for NR band n12 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186832 | 0286 | - | F | Correction of test frequencies for NR band n20 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186833 | 0287 | - | F | Correction of test frequencies for NR band n25 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186834 | 0288 | - | F | Correction of test frequencies for NR band n28 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186835 | 0289 | - | F | Correction of test frequencies for NR band n34 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186836 | 0290 | - | F | Correction of test frequencies for NR band n38 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186837 | 0291 | - | F | Correction of test frequencies for NR band n39 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186838 | 0292 | - | F | Correction of test frequencies for NR band n40 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186839 | 0293 | - | F | Correction of test frequencies for NR band n41 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186840 | 0294 | - | F | Correction of test frequencies for NR band n51 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186841 | 0295 | - | F | Introduction of test frequencies for NR band n66 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186842 | 0296 | - | F | Introduction of test frequencies for NR band n70 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186844 | 0298 | - | F | Correction of test frequencies for NR band n75 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186845 | 0299 | - | F | Correction of test frequencies for NR band n76 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186846 | 0300 | - | F | Correction of test frequencies for NR band n77 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186847 | 0301 | - | F | Correction of test frequencies for NR band n78 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186848 | 0302 | - | F | Correction of test frequencies for NR band n79 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186850 | 0304 | - | F | Correction of test frequencies for NR band n258 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186851 | 0305 | - | F | Correction of test frequencies for NR band n260 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186852 | 0306 | - | F | Correction of test frequencies for NR band n261 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186855 | 0309 | - | F | Introduction of preamble test states | 15.2.0 |
| 2018-12 | RAN#82 | R5-186857 | 0311 | - | F | Introduction DCI format 1_0 for paging, SI and random access | 15.2.0 |
| 2018-12 | RAN#82 | R5-186858 | 0312 | - | F | Correction to DCI format 1_1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186859 | 0313 | - | F | Update IE RateMatchPattern | 15.2.0 |
| 2018-12 | RAN#82 | R5-186861 | 0315 | - | F | Correction of generic procedure parameter naming for test loop function | 15.2.0 |
| 2018-12 | RAN#82 | R5-186862 | 0316 | - | F | Correction of test procedures to activate and deactivate UE Beamlock Function | 15.2.0 |
| 2018-12 | RAN#82 | R5-186893 | 0318 | - | F | Corrections to the notes in the OTA signal level tables | 15.2.0 |
| 2018-12 | RAN#82 | R5-186911 | 0320 | - | F | Add RRCSetupComplete | 15.2.0 |
| 2018-12 | RAN#82 | R5-186912 | 0321 | - | F | Add RRCSetupRequest | 15.2.0 |
| 2018-12 | RAN#82 | R5-186913 | 0322 | - | F | Add RRCSystemInfoRequest | 15.2.0 |
| 2018-12 | RAN#82 | R5-186916 | 0323 | - | F | Add SecurityModeCommand | 15.2.0 |
| 2018-12 | RAN#82 | R5-186918 | 0324 | - | F | Update SystemInformation | 15.2.0 |
| 2018-12 | RAN#82 | R5-186920 | 0325 | - | F | Add UEAssistanceInformation | 15.2.0 |
| 2018-12 | RAN#82 | R5-186921 | 0326 | - | F | Update UECapabilityEnquiry | 15.2.0 |
| 2018-12 | RAN#82 | R5-186922 | 0327 | - | F | Update ULInformationTransfer | 15.2.0 |
| 2018-12 | RAN#82 | R5-186923 | 0328 | - | F | Update IE PTRS-UplinkConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186925 | 0330 | - | F | Update RRCResumeRequest | 15.2.0 |
| 2018-12 | RAN#82 | R5-186929 | 0331 | - | F | Update PTRS-DownlinkConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-186936 | 0335 | - | F | Update PUCCH-SpatialRelationInfo | 15.2.0 |
| 2018-12 | RAN#82 | R5-186987 | 0342 | - | F | Addition of SIB3 message_Resubmission of 185792 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186988 | 0343 | - | F | Addition of SIB5 message_Resubmission of 186054 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186989 | 0344 | - | F | Addition of SIB6 - SIB8 message_Resubmission of 186055 | 15.2.0 |
| 2018-12 | RAN#82 | R5-186990 | 0345 | - | F | Addition of SIB9 message_Resubmission of 186056 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187026 | 0348 | - | F | Addition of P-Max in Test environment for RF test | 15.2.0 |
| 2018-12 | RAN#82 | R5-187028 | 0350 | - | F | Addition of test frequencies for SUL band n80 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187030 | 0352 | - | F | Addition of test frequencies for SUL band n82 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187031 | 0353 | - | F | Addition of test frequencies for SUL band n83 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187032 | 0354 | - | F | Addition of test frequencies for SUL band n84 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187033 | 0355 | - | F | Addition of test frequencies for SUL band n86 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187110 | 0358 | - | F | Correction to default message contents for SRB3 configuration | 15.2.0 |
| 2018-12 | RAN#82 | R5-187159 | 0361 | - | F | Updates to Configuration Update 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187160 | 0362 | - | F | Updates to De-registration 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187161 | 0363 | - | F | Updates to Identity 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187162 | 0364 | - | F | Updates to NAS Transport 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187163 | 0365 | - | F | Updates to Notification 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187164 | 0366 | - | F | Updates to PDU session authentication 5GSM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187166 | 0368 | - | F | Updates to PDU session modification 5GSM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187172 | 0374 | - | F | Removal of Editor’s Notes in section 4.6.3 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187175 | 0377 | - | F | Addition and updates to Information Elements in section 4.6.5 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187270 | 0381 | - | F | Updating 4.2.1 General functional requirements | 15.2.0 |
| 2018-12 | RAN#82 | R5-187271 | 0382 | - | F | Update the section for test equipment requirements for TRx | 15.2.0 |
| 2018-12 | RAN#82 | R5-187272 | 0383 | - | F | FR2 downlink signal level(38.508-1) | 15.2.0 |
| 2018-12 | RAN#82 | R5-187413 | 0389 | - | F | Uplink RNTI to valid value in TS 38.508-1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187415 | 0390 | - | F | Update maxPayloadMinus1 in PUCCH config in TS 38.508-1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187420 | 0393 | - | F | Addition of connection diagram for 2 TX UL MIMO | 15.2.0 |
| 2018-12 | RAN#82 | R5-187557 | 0396 | - | F | Addition of low and high test channel bandwidth in 38.508 | 15.2.0 |
| 2018-12 | RAN#82 | R5-188205 | 0397 | 1 | F | Updates to Annex B to add Permitted OTA Test Methods | 15.2.0 |
| 2018-12 | RAN#82 | R5-187610 | 0398 | - | F | Corrections to IEs part of PDSCH-ServingCellConfig, ServingCellConfig and ServingCellConfigCommon | 15.2.0 |
| 2018-12 | RAN#82 | R5-187659 | 0243 | 1 | F | Wordings for Uplink NAS messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187660 | 0247 | 1 | F | Default cell configurations for NAS | 15.2.0 |
| 2018-12 | RAN#82 | R5-187661 | 0248 | 1 | F | Update IE SI-SchedulingInfo | 15.2.0 |
| 2018-12 | RAN#82 | R5-187662 | 0349 | 1 | F | Addition of Combinations of system information blocks in 4.4.3.1.2 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187664 | 0263 | 1 | F | Correction to various Radio resource control IEs | 15.2.0 |
| 2018-12 | RAN#82 | R5-187665 | 0308 | 1 | F | Correction to DCI formats 0_0 and 0_1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187666 | 0310 | 1 | F | Introduction of SDL and SUL cells in simulated cells in clause 4.4.2 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187667 | 0314 | 1 | F | Correction to RRC_IDLE procedure | 15.2.0 |
| 2018-12 | RAN#82 | R5-187668 | 0332 | 1 | F | Update CSI related information elements | 15.2.0 |
| 2018-12 | RAN#82 | R5-187669 | 0333 | 1 | F | Update ServingCellConfigCommon and TDD-UL-DL-Config | 15.2.0 |
| 2018-12 | RAN#82 | R5-187670 | 0334 | 1 | F | Update SRS-Config | 15.2.0 |
| 2018-12 | RAN#82 | R5-187671 | 0336 | 1 | F | Update some information elements for measurements | 15.2.0 |
| 2018-12 | RAN#82 | R5-187672 | 0337 | 1 | F | Update CellGroupConfig and related information elements | 15.2.0 |
| 2018-12 | RAN#82 | R5-187673 | 0338 | 1 | F | CR of NR 508-1 clause 4.6.2_SIB2, SIB4 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187674 | 0339 | 1 | F | CR of NR 508-1 Table 4.4.2-2_Default NR Cells parameters | 15.2.0 |
| 2018-12 | RAN#82 | R5-187675 | 0341 | 1 | F | Update RLC-Config | 15.2.0 |
| 2018-12 | RAN#82 | R5-187676 | 0357 | 1 | F | Specifying Test procedure to check that UE is camped on a new NR cell belonging to a new TA | 15.2.0 |
| 2018-12 | RAN#82 | R5-187677 | 0360 | 1 | F | Updates to Authentication 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187678 | 0369 | 1 | F | Updates to PDU session release 5GSM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187679 | 0371 | 1 | F | Updates to Security mode 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187680 | 0375 | 1 | F | Addition of new Information Elements in section 4.6.3 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187681 | 0379 | 1 | F | Updates to SIG OTA Calibration for FR2 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187682 | 0394 | 1 | F | Addition of default QoS configurations | 15.2.0 |
| 2018-12 | RAN#82 | R5-187720 | 0319 | 2 | F | Uplink PTRS disable for RF testing | 15.2.0 |
| 2018-12 | RAN#82 | R5-188238 | 0242 | 2 | F | Addition to E-UTRA test frequencies for intra-band contiguous configuration for band 41 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187723 | 0303 | 1 | F | Correction of test frequencies for NR band n257 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187724 | 0269 | 1 | F | New annex for NR test frequency calculations | 15.2.0 |
| 2018-12 | RAN#82 | R5-187725 | 0297 | 1 | F | Correction of test frequencies for NR band n71 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187745 | 0238 | 1 | F | Update SIB1 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187747 | 0257 | 1 | F | Correction to Signal levels for conducted testing | 15.2.0 |
| 2018-12 | RAN#82 | R5-187748 | 0270 | 1 | F | Updates to E-UTRA RRC_CONNECTED generic procedure | 15.2.0 |
| 2018-12 | RAN#82 | R5-187750 | 0275 | 1 | F | Add RRCResumeComplete | 15.2.0 |
| 2018-12 | RAN#82 | R5-187751 | 0278 | 1 | F | Update chapter 4.5.3 RRC_INACTIVE | 15.2.0 |
| 2018-12 | RAN#82 | R5-187752 | 0307 | 1 | F | Correction of test frequencies for signalling testing in clause 6 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187753 | 0317 | 1 | F | Specifying Test procedure to check that UE is in RRC_IDLE state on a certain NR cell | 15.2.0 |
| 2018-12 | RAN#82 | R5-187754 | 0329 | 1 | F | Update IE RLF-TimersAndConstants | 15.2.0 |
| 2018-12 | RAN#82 | R5-187755 | 0346 | 1 | F | Add RRCSetup | 15.2.0 |
| 2018-12 | RAN#82 | R5-187756 | 0347 | 1 | F | Update RRCReconfiguration | 15.2.0 |
| 2018-12 | RAN#82 | R5-187757 | 0356 | 1 | F | Update IE RadioBearerConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-187759 | 0370 | 1 | F | Updates to Registration 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187760 | 0372 | 1 | F | Updates to Security protected 5GS NAS and 5GMM status messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187761 | 0373 | 1 | F | Updates to Service Request 5GMM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-187762 | 0376 | 1 | F | Addition and updates to Information Elements in section 4.6.4 | 15.2.0 |
| 2018-12 | RAN#82 | R5-187763 | 0388 | 1 | F | Addition of 5GS related new EFs to Test UICC definition | 15.2.0 |
| 2018-12 | RAN#82 | R5-187764 | 0395 | 1 | F | Update IE CellGroupConfig | 15.2.0 |
| 2018-12 | RAN#82 | R5-187802 | 0384 | 1 | F | Updating power levels for LTE Anchor Link | 15.2.0 |
| 2018-12 | RAN#82 | R5-187887 | 0351 | 1 | F | Addition of test frequencies for SUL band n81 | 15.2.0 |
| 2018-12 | RAN#82 | R5-188031 | 0391 | 1 | F | Addition of 2TX_UL_MIMO condition | 15.2.0 |
| 2018-12 | RAN#82 | R5-188107 | 0367 | 2 | F | Updates to PDU session establishment 5GSM messages | 15.2.0 |
| 2018-12 | RAN#82 | R5-188122 | 0260 | 2 | F | Update chapter 4.5.2 RRC_IDLE | 15.2.0 |
| 2018-12 | RAN#82 | R5-188123 | 0250 | 1 | F | Update chapter 4.5.4 RRC_CONNECTED | 15.2.0 |
| 2019-03 | RAN#83 | R5-191047 | 0526 | - | F | Update IE PDCCH-ConfigCommon | 15.3.0 |
| 2019-03 | RAN#83 | R5-191048 | 0527 | - | F | Update IE RadioBearerConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-191094 | 0529 | - | F | Updates of test channel bandwidth in TS 38.508-1 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191129 | 0530 | - | F | Update IE SDAP-Config | 15.3.0 |
| 2019-03 | RAN#83 | R5-191145 | 0531 | - | F | Update IE CellGroupId | 15.3.0 |
| 2019-03 | RAN#83 | R5-191155 | 0532 | - | F | Correction to temperature and voltage of Common test environments | 15.3.0 |
| 2019-03 | RAN#83 | R5-191187 | 0534 | - | F | Updates for Other SI support | 15.3.0 |
| 2019-03 | RAN#83 | R5-191189 | 0536 | - | F | Correction to RadioBearerConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-191191 | 0538 | - | F | Correction to SystemInformation | 15.3.0 |
| 2019-03 | RAN#83 | R5-191192 | 0539 | - | F | Correction to PUCCH-Config | 15.3.0 |
| 2019-03 | RAN#83 | R5-191193 | 0540 | - | F | Correction to SIB3 and SIB4 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191194 | 0541 | - | F | Correction of PUSCH-TimeDomainResourceAllocationList | 15.3.0 |
| 2019-03 | RAN#83 | R5-191195 | 0542 | - | F | Corrections and clarifications regarding DCI formats 0_1 and 1_1 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191219 | 0545 | - | F | Updates to Authentication 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191220 | 0546 | - | F | Updates to Configuration Update 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191221 | 0547 | - | F | Updates to De-registration 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191222 | 0548 | - | F | Updates to NAS transport 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191223 | 0549 | - | F | Updates to PDU session establishment 5GSM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191224 | 0550 | - | F | Updates to PDU session modification 5GSM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191225 | 0551 | - | F | Updates to PDU session release 5GSM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191226 | 0552 | - | F | Updates to Registration 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191227 | 0553 | - | F | Updates to Security Mode 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191228 | 0554 | - | F | Updates to Security Protected 5GS NAS message | 15.3.0 |
| 2019-03 | RAN#83 | R5-191229 | 0555 | - | F | Updates to Service Request 5GMM messages | 15.3.0 |
| 2019-03 | RAN#83 | R5-191233 | 0556 | - | F | Update IE BWP-Id | 15.3.0 |
| 2019-03 | RAN#83 | R5-191234 | 0557 | - | F | Add IE RejectWaitTime | 15.3.0 |
| 2019-03 | RAN#83 | R5-191235 | 0558 | - | F | Update IE ShortMAC-I | 15.3.0 |
| 2019-03 | RAN#83 | R5-191236 | 0559 | - | F | Update IE UE-TimersAndConstants | 15.3.0 |
| 2019-03 | RAN#83 | R5-191237 | 0560 | - | F | Update IE PUCCH-ConfigCommon | 15.3.0 |
| 2019-03 | RAN#83 | R5-191242 | 0561 | - | F | Addition of Positioning specifications | 15.3.0 |
| 2019-03 | RAN#83 | R5-191243 | 0562 | - | F | Update AS security Algorithm for RF testing | 15.3.0 |
| 2019-03 | RAN#83 | R5-191274 | 0563 | - | F | Update of structure of test frequency clauses | 15.3.0 |
| 2019-03 | RAN#83 | R5-191280 | 0564 | - | F | Correction to UL configuration | 15.3.0 |
| 2019-03 | RAN#83 | R5-191281 | 0565 | - | F | Correction to default value of IE’s in PDSCH-Config in Table 4.6.3-75 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191301 | 0568 | - | F | Correction of test frequencies for signalling testing in clause 6 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191302 | 0569 | - | F | Correction of test frequencies for EN-DC configuration DC_(n)41 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191304 | 0571 | - | F | Correction of test frequencies for NR band n1 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191305 | 0572 | - | F | Correction of test frequencies for NR band n2 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191306 | 0573 | - | F | Correction of test frequencies for NR band n3 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191307 | 0574 | - | F | Correction of test frequencies for NR band n5 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191308 | 0575 | - | F | Correction of test frequencies for NR band n7 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191309 | 0576 | - | F | Correction of test frequencies for NR band n8 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191310 | 0577 | - | F | Correction of test frequencies for NR band n12 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191311 | 0578 | - | F | Correction of test frequencies for NR band n20 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191312 | 0579 | - | F | Correction of test frequencies for NR band n25 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191313 | 0580 | - | F | Correction of test frequencies for NR band n28 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191314 | 0581 | - | F | Correction of test frequencies for NR band n34 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191315 | 0582 | - | F | Correction of test frequencies for NR band n38 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191316 | 0583 | - | F | Correction of test frequencies for NR band n39 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191317 | 0584 | - | F | Correction of test frequencies for NR band n40 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191318 | 0585 | - | F | Correction of test frequencies for NR band n41 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191319 | 0586 | - | F | Introduction of test frequencies for NR band n50 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191320 | 0587 | - | F | Correction of test frequencies for NR band n51 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191321 | 0588 | - | F | Correction of test frequencies for NR band n66 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191322 | 0589 | - | F | Correction of test frequencies for NR band n70 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191323 | 0590 | - | F | Correction of test frequencies for NR band n71 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191324 | 0591 | - | F | Introduction of test frequencies for NR band n74 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191325 | 0592 | - | F | Correction of test frequencies for NR band n75 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191326 | 0593 | - | F | Correction of test frequencies for NR band n76 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191327 | 0594 | - | F | Correction of test frequencies for NR band n77 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191328 | 0595 | - | F | Correction of test frequencies for NR band n78 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191329 | 0596 | - | F | Correction of test frequencies for NR band n79 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191330 | 0597 | - | F | Correction of test frequencies for NR band n257 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191331 | 0598 | - | F | Correction of test frequencies for NR band n258 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191332 | 0599 | - | F | Correction of test frequencies for NR band n260 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191333 | 0600 | - | F | Correction of test frequencies for NR band n261 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191334 | 0601 | - | F | Correction of DCI format 1_0 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191352 | 0603 | - | F | Update CounterCheckResponse | 15.3.0 |
| 2019-03 | RAN#83 | R5-191354 | 0604 | - | F | Add FailureInformation | 15.3.0 |
| 2019-03 | RAN#83 | R5-191355 | 0605 | - | F | Update LocationMeasurementIndication | 15.3.0 |
| 2019-03 | RAN#83 | R5-191356 | 0606 | - | F | Updates to section 4.8.3 (test USIM parameters) | 15.3.0 |
| 2019-03 | RAN#83 | R5-191360 | 0607 | - | F | Update MeasurementReport | 15.3.0 |
| 2019-03 | RAN#83 | R5-191361 | 0608 | - | F | Update MobilityFromNRCommand | 15.3.0 |
| 2019-03 | RAN#83 | R5-191364 | 0609 | - | F | Update Paging | 15.3.0 |
| 2019-03 | RAN#83 | R5-191366 | 0610 | - | F | Update RRCSetupComplete | 15.3.0 |
| 2019-03 | RAN#83 | R5-191368 | 0611 | - | F | Update SecurityModeComplete | 15.3.0 |
| 2019-03 | RAN#83 | R5-191370 | 0612 | - | F | Update SecurityModeFailure | 15.3.0 |
| 2019-03 | RAN#83 | R5-191371 | 0613 | - | F | Update UEAssistanceInformation | 15.3.0 |
| 2019-03 | RAN#83 | R5-191372 | 0614 | - | F | Update UECapabilityInformation | 15.3.0 |
| 2019-03 | RAN#83 | R5-191384 | 0616 | - | F | Correction to SecurityConfig of RadioBearerConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-191385 | 0617 | - | F | Correction to SIB9 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191386 | 0618 | - | F | Correction to SRS-Config of BWP-UplinkDedicated | 15.3.0 |
| 2019-03 | RAN#83 | R5-191446 | 0620 | - | F | Correction of default configuration of RRC IEs in 38.508-1 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191450 | 0621 | - | F | Addition of NR system information combination SIB6, SIB7 | 15.3.0 |
| 2019-03 | RAN#83 | R5-191538 | 0624 | - | F | Update ULInformationTransfer | 15.3.0 |
| 2019-03 | RAN#83 | R5-191539 | 0625 | - | F | Update IE QuantityConfig and CSI-ReportConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-191620 | 0629 | - | F | Clarification for NR inter-band measurement test case configuration | 15.3.0 |
| 2019-03 | RAN#83 | R5-191762 | 0637 | - | F | Editorial update in MeasObjectNR and ReportConfigNR | 15.3.0 |
| 2019-03 | RAN#83 | R5-191763 | 0638 | - | F | Update ReportConfigNR and TimeToTrigger | 15.3.0 |
| 2019-03 | RAN#83 | R5-192271 | 0570 | 1 | F | Correction of test frequencies for EN-DC configuration DC_(n)71 | 15.3.0 |
| 2019-03 | RAN#83 | R5-192272 | 0602 | 1 | F | Update chapter 4.5 RRC Connected initiation | 15.3.0 |
| 2019-03 | RAN#83 | R5-192273 | 0626 | 1 | F | Update RRCRelease | 15.3.0 |
| 2019-03 | RAN#83 | R5-192274 | 0615 | 1 | F | Correction to NR SchedulingRequestResourceConfig | 15.3.0 |
| 2019-03 | RAN#83 | R5-192275 | 0627 | 1 | F | Update IE I-RNTI-Value | 15.3.0 |
| 2019-03 | RAN#83 | R5-192276 | 0628 | 1 | F | Update IE ShortI-RNTI-Value | 15.3.0 |
| 2019-03 | RAN#83 | R5-192277 | 0630 | 1 | F | Updates to test environments for Signalling test | 15.3.0 |
| 2019-03 | RAN#83 | R5-192278 | 0633 | 1 | F | Addition of USIM Profiles for Signaling TC | 15.3.0 |
| 2019-03 | RAN#83 | R5-192279 | 0636 | 1 | F | Update QoS Configuration | 15.3.0 |
| 2019-03 | RAN#83 | R5-192280 | 0643 | 1 | F | Update to of Generic procedure E-UTRA RRC_IDLE | 15.3.0 |
| 2019-03 | RAN#83 | R5-192281 | 0644 | 1 | F | Introduction of EAP AKA | 15.3.0 |
| 2019-03 | RAN#83 | R5-192290 | 0655 | - | F | Update chapter 4.5 RRC_INACTIVE | 15.3.0 |
| 2019-03 | RAN#83 | R5-192363 | 0631 | 1 | F | Updating P-Max IE | 15.3.0 |
| 2019-03 | RAN#83 | R5-192364 | 0632 | 2 | F | Updating IEs part of SearchSpace | 15.3.0 |
| 2019-03 | RAN#83 | R5-192400 | 0528 | 1 | F | Setup diagram for receiver test using spectrum analyzer | 15.3.0 |
| 2019-03 | RAN#83 | R5-192541 | 0622 | 1 | F | Connection diagrams for RRM tests | 15.3.0 |
| 2019-03 | RAN#83 | R5-192542 | 0646 | 1 | F | Antenna Connection diagram for UE part for RRM | 15.3.0 |
| 2019-03 | RAN#83 | R5-192543 | 0649 | 1 | F | Connection diagram for FR1 demod test cases | 15.3.0 |
| 2019-03 | RAN#83 | R5-192705 | 0645 | 1 | F | Introduction of Non 3GPP Access over WLAN | 15.3.0 |
| 2019-03 | RAN#83 | R5-192735 | 0533 | 1 | F | Correction to PUSCH-Config | 15.3.0 |
| 2019-03 | RAN#83 | R5-192736 | 0535 | 1 | F | Addition of details on Test State 0 | 15.3.0 |
| 2019-03 | RAN#83 | R5-192737 | 0537 | 1 | F | Correction of CellGroupConfig tables and logical channel identities | 15.3.0 |
| 2019-03 | RAN#83 | R5-192738 | 0543 | 1 | F | Additions and updates to UE capability Information Elements | 15.3.0 |
| 2019-03 | RAN#83 | R5-192739 | 0544 | 1 | F | Updates and additions of default QoS configurations | 15.3.0 |
| 2019-03 | RAN#83 | R5-192740 | 0566 | 1 | F | Update chapter 4.5 General for PDUs | 15.3.0 |
| 2019-03 | RAN#83 | R5-192741 | 0567 | 1 | F | Update of Annex C on calculation of test frequencies | 15.3.0 |
| 2019-03 | RAN#83 | R5-192742 | 0619 | 1 | F | Correction to schedulingRequestID Configuration | 15.3.0 |
| 2019-03 | RAN#83 | R5-192743 | 0639 | 1 | F | Addition of Switch/Power UE procedures | 15.3.0 |
| 2019-03 | RAN#83 | R5-192744 | 0640 | 1 | F | Update to Test procedure to check that UE is camped on a new cell belonging to a new TA | 15.3.0 |
| 2019-03 | RAN#83 | R5-192745 | 0641 | 1 | F | Update to Test procedure to check that UE is in state 5GC RRC_IDLE on a certain cell | 15.3.0 |
| 2019-03 | RAN#83 | R5-192846 | 0648 | 1 | F | Updates to Annex B to add Permitted OTA Test Methods | 15.3.0 |
| 2019-03 | RAN#83 | - | - | - | - | Editorial updates of table numbering | 15.3.0 |
| 2019-06 | RAN#84 | R5-193537 | 0680 | - | F | Remove unused DCI formats from 38.508-1 | 15.4.0 |
| 2019-06 | RAN#84 | R5-193540 | 0681 | - | F | Adding setup diagram for Receiver performance tests 2x2 | 15.4.0 |
| 2019-06 | RAN#84 | R5-193542 | 0682 | - | F | Remove brackets from parameters for DCI formats for scheduling | 15.4.0 |
| 2019-06 | RAN#84 | R5-193613 | 0691 | - | F | Update default configuration of QuantityConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-193681 | 0693 | - | F | Update chapter 4.5.3 RRC_INACTIVE procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-193682 | 0694 | - | F | Update chapter 4.5.4 RRC_CONNECTED procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-193683 | 0695 | - | F | Update chapter 4.5.5 SWITCHED_OFF procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-193690 | 0696 | - | F | Resubmission: Connection diagram for 1x2 Demod test cases | 15.4.0 |
| 2019-06 | RAN#84 | R5-193734 | 0701 | - | F | Update IE I-RNTI-Value | 15.4.0 |
| 2019-06 | RAN#84 | R5-193735 | 0702 | - | F | Update IE ShortI-RNTI-Value | 15.4.0 |
| 2019-06 | RAN#84 | R5-193746 | 0710 | - | F | Update IE SubcarrierSpacing | 15.4.0 |
| 2019-06 | RAN#84 | R5-193813 | 0711 | - | F | Update of USIM EF5GS3GPPLOCI & EF5GSN3GPPLOCI | 15.4.0 |
| 2019-06 | RAN#84 | R5-193828 | 0713 | - | F | Add IE MultiFrequencyBandListNR-SIB | 15.4.0 |
| 2019-06 | RAN#84 | R5-193829 | 0714 | - | F | Add IE NR-NS-PmaxList | 15.4.0 |
| 2019-06 | RAN#84 | R5-193843 | 0716 | - | F | Update IE ServingCellConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-193862 | 0717 | - | F | Corrections to References | 15.4.0 |
| 2019-06 | RAN#84 | R5-193980 | 0725 | - | F | New test procedure for Registration Reject | 15.4.0 |
| 2019-06 | RAN#84 | R5-193981 | 0726 | - | F | Updates to test procedure 4.9.1 | 15.4.0 |
| 2019-06 | RAN#84 | R5-194038 | 0728 | - | F | Editorial Correction - USIM Profiles for Signaling TC | 15.4.0 |
| 2019-06 | RAN#84 | R5-194040 | 0729 | - | F | Correction to QoS Configuration | 15.4.0 |
| 2019-06 | RAN#84 | R5-194086 | 0733 | - | F | Update K2 value to align with RF DL RMC | 15.4.0 |
| 2019-06 | RAN#84 | R5-194087 | 0734 | - | F | Update aggregationlevel2 in SearchSpace IE | 15.4.0 |
| 2019-06 | RAN#84 | R5-194303 | 0740 | - | F | TDD-UL-DL-Config for FR1 SCS 60kHz | 15.4.0 |
| 2019-06 | RAN#84 | R5-194359 | 0742 | - | F | Removal of column for Number of PDU sessions established from tables for Test States | 15.4.0 |
| 2019-06 | RAN#84 | R5-194362 | 0743 | - | F | Editorial correction to test frequency clauses | 15.4.0 |
| 2019-06 | RAN#84 | R5-194364 | 0744 | - | F | Update of test frequencies for EN-DC combination DC_41A_n41A | 15.4.0 |
| 2019-06 | RAN#84 | R5-194367 | 0745 | - | F | Common procedure to configure SCC for CA RF testing | 15.4.0 |
| 2019-06 | RAN#84 | R5-194369 | 0746 | - | F | Introduction of test frequencies for inter-band Rel-15 EN-DC two bands configurations | 15.4.0 |
| 2019-06 | RAN#84 | R5-194420 | 0751 | - | F | Update IE BWP-Downlink | 15.4.0 |
| 2019-06 | RAN#84 | R5-194435 | 0752 | - | F | Update IE BWP-Id | 15.4.0 |
| 2019-06 | RAN#84 | R5-194438 | 0755 | - | F | Updates to UE 4.6.5 Other Information Elements | 15.4.0 |
| 2019-06 | RAN#84 | R5-194441 | 0757 | - | F | Update IE BWP-Uplink | 15.4.0 |
| 2019-06 | RAN#84 | R5-194479 | 0758 | - | F | Editorial updates to 4.7.1 Contents of 5GMM messages | 15.4.0 |
| 2019-06 | RAN#84 | R5-194480 | 0759 | - | F | Editorial updates to 4.7.2 Contents of 5GSM messages | 15.4.0 |
| 2019-06 | RAN#84 | R5-194510 | 0762 | - | F | Update of Switch off - Power off procedure in RRC_CONNECTED | 15.4.0 |
| 2019-06 | RAN#84 | R5-194539 | 0767 | - | F | Introduction of test frequencies for EN-DC CA configuration DC_30A_n260(A-I) | 15.4.0 |
| 2019-06 | RAN#84 | R5-194541 | 0768 | - | F | Antenna Connection diagram for TE part for RRM | 15.4.0 |
| 2019-06 | RAN#84 | R5-194709 | 0785 | - | F | Update 38.508 RF and RRM clauses with agreed recommendation to configure UE as non-IMS | 15.4.0 |
| 2019-06 | RAN#84 | R5-194783 | 0774 | - | F | Introduction of test frequencies for NR band n50 and signalling testing | 15.4.0 |
| 2019-06 | RAN#84 | R5-194784 | 0775 | - | F | Introduction of test frequencies for NR band n74 and signalling testing | 15.4.0 |
| 2019-06 | RAN#84 | R5-194790 | 0778 | - | F | Updates to power allocations | 15.4.0 |
| 2019-06 | RAN#84 | R5-194791 | 0779 | - | F | Update of DownlinkConfigCommonSIB | 15.4.0 |
| 2019-06 | RAN#84 | R5-194794 | 0684 | 1 | F | Update IE PDSCH-Config | 15.4.0 |
| 2019-06 | RAN#84 | R5-194795 | 0687 | 1 | F | Update NR MeasObjectNR | 15.4.0 |
| 2019-06 | RAN#84 | R5-194796 | 0690 | 1 | F | Update default configuration of ReportConfigNR | 15.4.0 |
| 2019-06 | RAN#84 | R5-194797 | 0692 | 1 | F | Update chapter 4.5.2 RRC_IDLE procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-194798 | 0704 | 1 | F | Correction to the note associated to the Table 4.7.1-2 | 15.4.0 |
| 2019-06 | RAN#84 | R5-194800 | 0708 | 1 | F | Update IE MIB | 15.4.0 |
| 2019-06 | RAN#84 | R5-194801 | 0709 | 1 | F | Update IE SchedulingRequestResourceConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-194802 | 0712 | 1 | F | Correct clause numbers in 4.5A | 15.4.0 |
| 2019-06 | RAN#84 | R5-194803 | 0718 | 1 | F | Update IE ServingCellConfigCommon | 15.4.0 |
| 2019-06 | RAN#84 | R5-194804 | 0721 | 1 | F | Update IE FrequencyInfoUL | 15.4.0 |
| 2019-06 | RAN#84 | R5-194805 | 0722 | 1 | F | Update IE FrequencyInfoUL-SIB | 15.4.0 |
| 2019-06 | RAN#84 | R5-194806 | 0723 | 1 | F | Update generic procedures chapter general | 15.4.0 |
| 2019-06 | RAN#84 | R5-194807 | 0724 | 1 | F | Update chapter 4.5.2 RRC_IDLE Initiation | 15.4.0 |
| 2019-06 | RAN#84 | R5-194808 | 0730 | 1 | F | Updates to RadioBearerConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-194809 | 0732 | 1 | F | Updates to PhysicalCellGroupConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-194810 | 0739 | 1 | F | New test procedure for RRC_CONNECTED | 15.4.0 |
| 2019-06 | RAN#84 | R5-194811 | 0741 | 1 | F | Updated IE MeasObjectEUTRA and ReportConfigInterRAT | 15.4.0 |
| 2019-06 | RAN#84 | R5-194812 | 0753 | 1 | F | Updates to Registration 5GMM messages | 15.4.0 |
| 2019-06 | RAN#84 | R5-194813 | 0754 | 1 | F | Updates to UE 4.6.4 UE Capability Information Elements | 15.4.0 |
| 2019-06 | RAN#84 | R5-194814 | 0760 | 1 | F | New Test procedure for UE for Tracking area updating / inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode | 15.4.0 |
| 2019-06 | RAN#84 | R5-194817 | 0777 | 1 | F | New Test procedure for UE for Tracking area updating / inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 15.4.0 |
| 2019-06 | RAN#84 | R5-194821 | 0780 | - | F | Introducing conditions for Handover in RRCReconfiguration and RadioBearerConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-194824 | 0781 | - | F | Updates to Service Request 5GMM message | 15.4.0 |
| 2019-06 | RAN#84 | R5-194879 | 0735 | 1 | F | Updates to Multi-Cell SIG OTA testing for FR2 | 15.4.0 |
| 2019-06 | RAN#84 | R5-194881 | 0763 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n257B | 15.4.0 |
| 2019-06 | RAN#84 | R5-194882 | 0764 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260B | 15.4.0 |
| 2019-06 | RAN#84 | R5-194883 | 0765 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260I | 15.4.0 |
| 2019-06 | RAN#84 | R5-194884 | 0766 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n261B | 15.4.0 |
| 2019-06 | RAN#84 | R5-194885 | 0782 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260(A-I) | 15.4.0 |
| 2019-06 | RAN#84 | R5-194889 | 0737 | 1 | F | corrections to Non 3GPP Access over WLAN procedures | 15.4.0 |
| 2019-06 | RAN#84 | R5-194894 | 0783 | - | F | Update FFS in ResumeCause | 15.4.0 |
| 2019-06 | RAN#84 | R5-194896 | 0784 | - | F | Updates to reference QoS configurations for EPS interworking | 15.4.0 |
| 2019-06 | RAN#84 | R5-194902 | 0685 | 1 | F | Correction of Setup Diagrams for Receiver tests using Signal Generator in 38.508-1 | 15.4.0 |
| 2019-06 | RAN#84 | R5-195095 | 0750 | 1 | F | Introduction of Connection diagram for 2x4 and 4x4 Demod test cases | 15.4.0 |
| 2019-06 | RAN#84 | R5-195322 | 0686 | 1 | F | Update NR SIB1 | 15.4.0 |
| 2019-06 | RAN#84 | R5-195323 | 0703 | 1 | F | Update IE CommonCellGroupConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-195324 | 0715 | 1 | F | Update default configuration of MeasGapConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-195325 | 0719 | 1 | F | Addition of Switch off / Power off procedure in RRC_INACTIVE | 15.4.0 |
| 2019-06 | RAN#84 | R5-195326 | 0720 | 1 | F | Update of SIB5 | 15.4.0 |
| 2019-06 | RAN#84 | R5-195327 | 0731 | 1 | F | Updates to RLC-BearerConfig | 15.4.0 |
| 2019-06 | RAN#84 | R5-195328 | 0756 | 1 | F | Updates to PDU session establishment 5GSM messages | 15.4.0 |
| 2019-06 | RAN#84 | R5-195329 | 0773 | 1 | F | Introduction of test frequencies for inter-RAT signalling testing | 15.4.0 |
| 2019-06 | RAN#84 | R5-195330 | 0776 | 1 | F | Correction to PUSCH-Config | 15.4.0 |
| 2019-06 | RAN#84 | R5-195426 | 0727 | 2 | F | 38.508-1 implementation of FR2 UL demod OTA tests using single pol Rx TE | 15.4.0 |
| 2019-06 | RAN#84 | R5-195427 | 0772 | 2 | F | Addition of message contents needed for DEMOD test cases | 15.4.0 |
| 2019-06 | RAN#84 | R5-194370 | 0747 | - | F | Introduction of test frequencies for inter-band Rel-16 EN-DC two bands configurations | 16.0.0 |
| 2019-06 | RAN#84 | R5-194371 | 0748 | - | F | Introduction of test frequencies for inter-band Rel-16 EN-DC five bands configurations | 16.0.0 |
| 2019-06 | RAN#84 | R5-194373 | 0749 | - | F | Introduction of test frequencies for NR CA configuration CA_n41C | 16.0.0 |
| 2019-09 | RAN#85 | R5-195696 | 0795 | - | F | Update IE PDCP-Config | 16.1.0 |
| 2019-09 | RAN#85 | R5-195711 | 0797 | - | F | Add IE CGI-InfoEUTRA | 16.1.0 |
| 2019-09 | RAN#85 | R5-195729 | 0798 | - | F | Update IE CGI-Info | 16.1.0 |
| 2019-09 | RAN#85 | R5-195730 | 0799 | - | F | Update IE MeasResults | 16.1.0 |
| 2019-09 | RAN#85 | R5-195731 | 0800 | - | F | Update of 4.3.1.0A mid test CBW in 38.508-1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-195747 | 0803 | - | F | Update IE MeasResultCellListSFTD | 16.1.0 |
| 2019-09 | RAN#85 | R5-195748 | 0804 | - | F | Add IE MeasResultCellListSFTD-EUTRA | 16.1.0 |
| 2019-09 | RAN#85 | R5-195749 | 0805 | - | F | Add IE MeasResult2EUTRA | 16.1.0 |
| 2019-09 | RAN#85 | R5-195750 | 0806 | - | F | Add IE MeasResult2NR | 16.1.0 |
| 2019-09 | RAN#85 | R5-195751 | 0807 | - | F | Add IE SK-Counter | 16.1.0 |
| 2019-09 | RAN#85 | R5-195752 | 0808 | - | F | Update IE SS-RSSI-Measurement | 16.1.0 |
| 2019-09 | RAN#85 | R5-195792 | 0811 | - | F | Update MeasurementReport | 16.1.0 |
| 2019-09 | RAN#85 | R5-195885 | 0814 | - | F | Update RRCResume | 16.1.0 |
| 2019-09 | RAN#85 | R5-195886 | 0815 | - | F | Editorial update RRCReconfigurationComplete | 16.1.0 |
| 2019-09 | RAN#85 | R5-195887 | 0816 | - | F | Editorial update RRCReject | 16.1.0 |
| 2019-09 | RAN#85 | R5-195888 | 0817 | - | F | Editorial update RRCRelease | 16.1.0 |
| 2019-09 | RAN#85 | R5-195889 | 0818 | - | F | Add SCGFailureInformation | 16.1.0 |
| 2019-09 | RAN#85 | R5-195890 | 0819 | - | F | Add SCGFailureInformationEUTRA | 16.1.0 |
| 2019-09 | RAN#85 | R5-195895 | 0820 | - | F | Update UECapabilityEnquiry | 16.1.0 |
| 2019-09 | RAN#85 | R5-195909 | 0821 | - | F | Editorial update UECapabilityInformation | 16.1.0 |
| 2019-09 | RAN#85 | R5-195910 | 0822 | - | F | Add ULInformationTransferMRDC | 16.1.0 |
| 2019-09 | RAN#85 | R5-195926 | 0823 | - | F | Editorial update RRC IEs | 16.1.0 |
| 2019-09 | RAN#85 | R5-195927 | 0824 | - | F | Editorial update S-NSSAI | 16.1.0 |
| 2019-09 | RAN#85 | R5-195944 | 0826 | - | F | Correction to ReportConfigNR | 16.1.0 |
| 2019-09 | RAN#85 | R5-195945 | 0827 | - | F | Updates to default configurations for 5GC NAS test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196030 | 0829 | - | F | Handling of thresholds in FR2 when Events A3 and A6 are inter-frequency | 16.1.0 |
| 2019-09 | RAN#85 | R5-196031 | 0830 | - | F | Adding references to TS 38.508-1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196136 | 0836 | - | F | Addition new NR cell for SS-RSRP RRM tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-196148 | 0837 | - | F | Update of Annex C for selecting SSB location for cells not selectable as PCell | 16.1.0 |
| 2019-09 | RAN#85 | R5-196158 | 0838 | - | F | Correction of references to test frequency tables | 16.1.0 |
| 2019-09 | RAN#85 | R5-196159 | 0839 | - | F | Correction of clause numbers for test frequencies for Non-3GPP Access | 16.1.0 |
| 2019-09 | RAN#85 | R5-196168 | 0840 | - | F | Correction of test frequency parameters for SSB location for NR band n1 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196169 | 0841 | - | F | Correction of test frequency parameters for SSB location for NR band n2 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196170 | 0842 | - | F | Correction of test frequency parameters for SSB location for NR band n3 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196171 | 0843 | - | F | Correction of test frequency parameters for SSB location for NR band n7 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196172 | 0844 | - | F | Correction of test frequency parameters for SSB location for NR band n25 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196174 | 0846 | - | F | Correction of test frequency parameters for SSB location for NR band n38 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196175 | 0847 | - | F | Correction of test frequency parameters for SSB location for NR band n39 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196176 | 0848 | - | F | Correction of test frequency parameters for SSB location for NR band n40 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196177 | 0849 | - | F | Correction of test frequency parameters for SSB location for NR band n41 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196178 | 0850 | - | F | Correction of test frequency parameters for SSB location for NR band n50 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196179 | 0851 | - | F | Correction of test frequency parameters for SSB location for NR band n66 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196180 | 0852 | - | F | Correction of test frequency parameters for SSB location for NR band n70 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196181 | 0853 | - | F | Correction of test frequency parameters for SSB location for NR band n74 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196182 | 0854 | - | F | Correction of test frequency parameters for SSB location for NR band n75 and SCS 15kHz and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196183 | 0855 | - | F | Correction of test frequency parameters for SSB location for NR band n77 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196184 | 0856 | - | F | Correction of test frequency parameters for SSB location for NR band n78 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196185 | 0857 | - | F | Correction of test frequency parameters for SSB location for NR band n79 and SCS 60kHz | 16.1.0 |
| 2019-09 | RAN#85 | R5-196197 | 0860 | - | F | Update IE ServingCellConfigCommon | 16.1.0 |
| 2019-09 | RAN#85 | R5-196198 | 0861 | - | F | Update IE SubcarrierSpacing | 16.1.0 |
| 2019-09 | RAN#85 | R5-196262 | 0863 | - | F | Editorial update IE RLC-BearerConfig | 16.1.0 |
| 2019-09 | RAN#85 | R5-196289 | 0864 | - | F | Update chapter 4.5A.2 UE-requested PDU session establishment procedure | 16.1.0 |
| 2019-09 | RAN#85 | R5-196310 | 0867 | - | F | Addition of SUL bands for protocol testing in clause 6.2.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196311 | 0868 | - | F | Update of test frequency parameters for NR CA configuration CA_n41C | 16.1.0 |
| 2019-09 | RAN#85 | R5-196315 | 0872 | - | F | Update of test frequency parameters for NR CA configuration CA_n260(A-I) | 16.1.0 |
| 2019-09 | RAN#85 | R5-196316 | 0873 | - | F | Introduction of test frequencies for NR CA configuration CA_n261B and CA_n261C | 16.1.0 |
| 2019-09 | RAN#85 | R5-196317 | 0874 | - | F | Update of test frequency table for EN-DC configuration DC_41A_n41A for BCS1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196318 | 0875 | - | F | Correction of test frequency parameters for EN-DC configuration DC_(n)41AA | 16.1.0 |
| 2019-09 | RAN#85 | R5-196319 | 0876 | - | F | Correction of test frequency parameters for EN-DC configuration DC_(n)71AA | 16.1.0 |
| 2019-09 | RAN#85 | R5-196468 | 0885 | - | F | Introduction of test frequencies for NR CA configuration CA_n258B and CA_n258C | 16.1.0 |
| 2019-09 | RAN#85 | R5-196469 | 0886 | - | F | Introduction of test frequencies for NR CA configuration CA_n258G to CA_n258M | 16.1.0 |
| 2019-09 | RAN#85 | R5-196470 | 0887 | - | F | Introduction of test frequencies for NR CA configuration CA_n260G to CA_n260I | 16.1.0 |
| 2019-09 | RAN#85 | R5-196472 | 0889 | - | F | Introduction of test frequencies for NR CA configuration CA_n261G to CA_n261I | 16.1.0 |
| 2019-09 | RAN#85 | R5-196473 | 0890 | - | F | Introduction of test frequencies for NR CA configuration CA_n261O to CA_n261Q | 16.1.0 |
| 2019-09 | RAN#85 | R5-196490 | 0894 | - | F | Introduction of test frequencies for NR CA configuration CA_n78C | 16.1.0 |
| 2019-09 | RAN#85 | R5-196539 | 0895 | - | F | Update to 38.508-1 for Demod specific message contents | 16.1.0 |
| 2019-09 | RAN#85 | R5-196581 | 0897 | - | F | Removing brackets from values for DCI formats | 16.1.0 |
| 2019-09 | RAN#85 | R5-196597 | 0899 | - | F | Cleanup of editor note of EFOPL5G | 16.1.0 |
| 2019-09 | RAN#85 | R5-196637 | 0900 | - | F | Update of default messages for EMERGENCY services test scenarios | 16.1.0 |
| 2019-09 | RAN#85 | R5-196641 | 0904 | - | F | Update of Test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 16.1.0 |
| 2019-09 | RAN#85 | R5-196654 | 0905 | - | F | Editorial correction of reference test conditions | 16.1.0 |
| 2019-09 | RAN#85 | R5-196751 | 0911 | - | F | AP#82.01: Update default DCI format to 0_1 / 1_1 in TS 38.508-1 for SIG test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196824 | 0917 | - | F | Updates to UE 4.6.4 UE Capability Information Elements | 16.1.0 |
| 2019-09 | RAN#85 | R5-196825 | 0918 | - | F | Addition of default test control messages | 16.1.0 |
| 2019-09 | RAN#85 | R5-196873 | 0922 | - | F | Introduction of test frequencies for NR CA configuration CA_n258D to CA_n258F | 16.1.0 |
| 2019-09 | RAN#85 | R5-196874 | 0923 | - | F | Introduction of test frequencies for NR CA configuration CA_n260D to CA_n260F | 16.1.0 |
| 2019-09 | RAN#85 | R5-196875 | 0924 | - | F | Introduction of test frequencies for NR CA configuration CA_n260O to CA_n260Q | 16.1.0 |
| 2019-09 | RAN#85 | R5-196942 | 0927 | - | F | Correction of clause 2 and 4.3 in 38.508-1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196980 | 0786 | 1 | F | Using generic procedure for IMS registration to 5GS | 16.1.0 |
| 2019-09 | RAN#85 | R5-196981 | 0788 | 1 | F | Update of SIB2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196982 | 0790 | 1 | F | Update of SIB5 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196984 | 0792 | 1 | F | Update of frequency definition for Inter-RAT test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196985 | 0793 | 1 | F | Update IE CellGroupConfig | 16.1.0 |
| 2019-09 | RAN#85 | R5-196986 | 0858 | 1 | F | Update MIB | 16.1.0 |
| 2019-09 | RAN#85 | R5-196987 | 0865 | 1 | F | Update chapter 4.5.4 RRC_CONNECTED procedures | 16.1.0 |
| 2019-09 | RAN#85 | R5-196988 | 0831 | 1 | F | Addition of IE MasterKeyUpdate | 16.1.0 |
| 2019-09 | RAN#85 | R5-196990 | 0833 | 1 | F | USIM Configuration for Signalling Test Cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196991 | 0835 | 1 | F | Correction to SIG OTA UE Orientation procedure | 16.1.0 |
| 2019-09 | RAN#85 | R5-196992 | 0878 | 1 | F | Addition of New Test Procedure - Response\No response to Paging for 5GC NAS testing | 16.1.0 |
| 2019-09 | RAN#85 | R5-196994 | 0913 | 1 | F | Update IE ServingCellConfig | 16.1.0 |
| 2019-09 | RAN#85 | R5-196995 | 0910 | 1 | F | Corrections to DCI_1_0 configuration | 16.1.0 |
| 2019-09 | RAN#85 | R5-196996 | 0919 | 1 | F | Updates to generic procedure using SERVICE REQUEST procedure | 16.1.0 |
| 2019-09 | RAN#85 | R5-196997 | 0901 | 1 | F | Introduction of Test Procedure for IMS Emergency call establishment in 5GC NORMAL-SERVICE | 16.1.0 |
| 2019-09 | RAN#85 | R5-196998 | 0903 | 1 | F | Update of Test procedure for UE for Tracking area updating / Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode | 16.1.0 |
| 2019-09 | RAN#85 | R5-197014 | 0928 | - | F | Addition of NR CA test frequencies for protocol testing in clause 6.2.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197099 | 0929 | - | F | Correction to Switch off-Power off procedure in RRC_CONNECTED | 16.1.0 |
| 2019-09 | RAN#85 | R5-197104 | 0884 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n257G to CA_n257M | 16.1.0 |
| 2019-09 | RAN#85 | R5-197106 | 0892 | 1 | F | Update of EN-DC inter-band configurations in clause 4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197139 | 0891 | 2 | F | Update of NR CA inter-band configurations in clause 4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197226 | 0915 | 1 | F | changes for Non 3GPP Access over WLAN | 16.1.0 |
| 2019-09 | RAN#85 | R5-197230 | 0883 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n257D to CA_n257F | 16.1.0 |
| 2019-09 | RAN#85 | R5-197231 | 0869 | 1 | F | Update of test frequency parameters for NR CA configuration CA_n257B and CA_n257C | 16.1.0 |
| 2019-09 | RAN#85 | R5-197232 | 0870 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260B and CA_n260C | 16.1.0 |
| 2019-09 | RAN#85 | R5-197233 | 0888 | 1 | F | Introduction of test frequencies for NR CA configuration CA_n260J to CA_n260M | 16.1.0 |
| 2019-09 | RAN#85 | R5-197234 | 0813 | 1 | F | Update RRCReconfiguration | 16.1.0 |
| 2019-09 | RAN#85 | R5-197235 | 0796 | 1 | F | Update RadioBearerConfig-DRB | 16.1.0 |
| 2019-09 | RAN#85 | R5-197236 | 0825 | 1 | F | Update RRCReconfiguration-HO | 16.1.0 |
| 2019-09 | RAN#85 | R5-197241 | 0791 | 1 | F | Update of EUTRA-AllowedMeasBandwidth | 16.1.0 |
| 2019-09 | RAN#85 | R5-197243 | 0809 | 1 | F | Addition of Delta to signalling threshold in System Information in FR2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197244 | 0866 | 1 | F | Correction to REGISTRATION REJECT message | 16.1.0 |
| 2019-09 | RAN#85 | R5-197246 | 0902 | 1 | F | Introduction of Test Procedure for IMS Emergency call establishment in 5GC LIMITED-SERVICE or NO-SUPI | 16.1.0 |
| 2019-09 | RAN#85 | R5-197296 | 0898 | 2 | F | Update of PHR-Config | 16.1.0 |
| 2019-09 | RAN#85 | R5-197300 | 0787 | 1 | F | 4x2 Connection Diagram for demodulation tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-197301 | 0862 | 1 | F | Correction to Section 5.4.2 Message definition for Performance Test | 16.1.0 |
| 2019-09 | RAN#85 | R5-197302 | 0882 | 1 | F | Addition of FR2 CA connection diagram | 16.1.0 |
| 2019-09 | RAN#85 | R5-197303 | 0920 | 1 | F | Corrections to test frequencies and formulas | 16.1.0 |
| 2019-09 | RAN#85 | R5-197304 | 0896 | 1 | F | Removing IOT bit information from test channel bandwidth tables | 16.1.0 |
| 2019-09 | RAN#85 | R5-197305 | 0908 | 1 | F | Addition of SMTC Configuration for RRM test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197507 | 0906 | 1 | F | Addition of TDD UL DL Config for RRM test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197508 | 0907 | 1 | F | Addition of FilterCoefficient configuration for RRM test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197638 | 0881 | 2 | F | Addition of FR1 CA connection diagram | 16.1.0 |
| 2019-10 | RAN#85 | - | - | - | - | Implementation fixes | 16.1.1 |
| 2019-12 | RAN#86 | R5-197727 | 0932 | - | F | Editorial update IE BWP-Id | 16.2.0 |
| 2019-12 | RAN#86 | R5-197751 | 0933 | - | F | Editorial update IE PDSCH-TimeDomainResourceAllocationList | 16.2.0 |
| 2019-12 | RAN#86 | R5-197835 | 0937 | - | F | Correction to IE ReportConfigNR | 16.2.0 |
| 2019-12 | RAN#86 | R5-197897 | 0940 | - | F | Editorial update IE CodebookConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-197932 | 0946 | - | F | Editorial update IE PDSCH-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-197967 | 0948 | - | F | Update of Annex C on calculation of test frequencies and parameters | 16.2.0 |
| 2019-12 | RAN#86 | R5-197968 | 0949 | - | F | Correction of test frequency parameters for NR band n1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197969 | 0950 | - | F | Correction of test frequency parameters for NR band n2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197970 | 0951 | - | F | Correction of test frequency parameters for NR band n3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197971 | 0952 | - | F | Correction of test frequency parameters for NR band n5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197972 | 0953 | - | F | Correction of test frequency parameters for NR band n7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197973 | 0954 | - | F | Correction of test frequency parameters for NR band n8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197974 | 0955 | - | F | Correction of test frequency parameters for NR band n12 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197975 | 0956 | - | F | Correction of test frequency parameters for NR band n20 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197976 | 0957 | - | F | Correction of test frequency parameters for NR band n25 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197977 | 0958 | - | F | Correction of test frequency parameters for NR band n28 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197978 | 0959 | - | F | Correction of test frequency parameters for NR band n34 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197979 | 0960 | - | F | Correction of test frequency parameters for NR band n38 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197980 | 0961 | - | F | Correction of test frequency parameters for NR band n39 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197981 | 0962 | - | F | Correction of test frequency parameters for NR band n40 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197982 | 0963 | - | F | Correction of test frequency parameters for NR band n41 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197983 | 0964 | - | F | Correction of test frequency parameters for NR band n50 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197984 | 0965 | - | F | Correction of test frequency parameters for NR band n51 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197985 | 0966 | - | F | Correction of test frequency parameters for NR band n66 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197986 | 0967 | - | F | Correction of test frequency parameters for NR band n70 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197987 | 0968 | - | F | Correction of test frequency parameters for NR band n71 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197988 | 0969 | - | F | Correction of test frequency parameters for NR band n74 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197989 | 0970 | - | F | Correction of test frequency parameters for NR band n75 (SDL) | 16.2.0 |
| 2019-12 | RAN#86 | R5-197990 | 0971 | - | F | Correction of test frequency parameters for NR band n76 (SDL) | 16.2.0 |
| 2019-12 | RAN#86 | R5-197991 | 0972 | - | F | Correction of test frequency parameters for NR band n77 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197992 | 0973 | - | F | Correction of test frequency parameters for NR band n78 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197993 | 0974 | - | F | Correction of test frequency parameters for NR band n79 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197994 | 0975 | - | F | Editorial correction to note 1 in frequency tables for NR bands n257, n258, n260 and n261 | 16.2.0 |
| 2019-12 | RAN#86 | R5-197997 | 0978 | - | F | Introduction of test frequencies for NR CA configuration CA_n261D to CA_n261F | 16.2.0 |
| 2019-12 | RAN#86 | R5-197998 | 0979 | - | F | Introduction of test frequencies for NR CA configuration CA_n261J to CA_n261M | 16.2.0 |
| 2019-12 | RAN#86 | R5-198016 | 0983 | - | F | Introduction of test frequencies parameters for Rel-16 NR CA configuration CA_n66B | 16.2.0 |
| 2019-12 | RAN#86 | R5-198017 | 0984 | - | F | Introduction of test frequencies parameters for Rel-16 NR CA configuration CA_n66(2A) | 16.2.0 |
| 2019-12 | RAN#86 | R5-198018 | 0985 | - | F | Introduction of test frequencies and parameters for NR band n29 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198019 | 0986 | - | F | Introduction of test frequencies and parameters for NR band n65 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198028 | 0988 | - | F | Add 4Rx connection diagram for RRM measurement tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198057 | 0993 | - | F | Editorial update IE RateMatchPattern | 16.2.0 |
| 2019-12 | RAN#86 | R5-198058 | 0994 | - | F | Editorial update IE SchedulingRequestResourceConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-198082 | 0999 | - | F | Introduce general chapter in 4.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198120 | 1004 | - | F | Correction of test frequencies parameters for Rel-15 EN-DC configuration DC_(n)41AA | 16.2.0 |
| 2019-12 | RAN#86 | R5-198121 | 1005 | - | F | Correction of test frequencies parameters for Rel-15 EN-DC configuration DC_(n)71AA | 16.2.0 |
| 2019-12 | RAN#86 | R5-198125 | 1009 | - | F | Introduction of test frequencies and parameters for NR band n48 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198126 | 1010 | - | F | Introduction of test frequencies for NR band b41 and CBW 30MHz | 16.2.0 |
| 2019-12 | RAN#86 | R5-198131 | 1012 | - | F | Update of USIM Configuration 15 for forbidden PLMN | 16.2.0 |
| 2019-12 | RAN#86 | R5-198133 | 1014 | - | F | Update IE ServingCellConfigCommonSIB | 16.2.0 |
| 2019-12 | RAN#86 | R5-198141 | 1015 | - | F | Clarification on default radio configuration of NAS cells | 16.2.0 |
| 2019-12 | RAN#86 | R5-198217 | 1019 | - | F | Correction of test frequency parameters for protocol testing and NR bands with scs=15kHz | 16.2.0 |
| 2019-12 | RAN#86 | R5-198223 | 1021 | - | F | Add IE BetaOffsets | 16.2.0 |
| 2019-12 | RAN#86 | R5-198224 | 1022 | - | F | Update IE PUSCH-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198250 | 1023 | - | F | Update IE CSI-FrequencyOccupation | 16.2.0 |
| 2019-12 | RAN#86 | R5-198251 | 1024 | - | F | Update IE PHR-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198258 | 1026 | - | F | Editorial update IE DRX-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198282 | 1027 | - | F | Update to Connection diagram for 2x4 and 4x4 Demod test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-198286 | 1028 | - | F | Correction of mapping of frequency ranges to NR test frequencies for NR SA | 16.2.0 |
| 2019-12 | RAN#86 | R5-198304 | 1029 | - | F | Editorial update IE LogicalChannelConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-198370 | 1035 | - | F | Connection diagram for FR2 Demod and CSI test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-198480 | 1039 | - | F | Editorial update IE PDCCH-ConfigCommon | 16.2.0 |
| 2019-12 | RAN#86 | R5-198485 | 1041 | - | F | Editorial update IE PDCP-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198506 | 1044 | - | F | Addition of RRCReconfiguration for Speech call setup | 16.2.0 |
| 2019-12 | RAN#86 | R5-198507 | 1045 | - | F | Editorial updates to section 4.7.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198508 | 1046 | - | F | New reference QoS configurations for IMS voice and video | 16.2.0 |
| 2019-12 | RAN#86 | R5-198509 | 1047 | - | F | Updates to REGISTRATION ACCEPT 5GMM message | 16.2.0 |
| 2019-12 | RAN#86 | R5-198510 | 1048 | - | F | Updates to test control messages | 16.2.0 |
| 2019-12 | RAN#86 | R5-198540 | 1051 | - | F | Update of REGISTRATION ACCEPT for IMS emergency support | 16.2.0 |
| 2019-12 | RAN#86 | R5-198544 | 1053 | - | F | Update of Table 4.6.3-162 SearchSpace in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198638 | 1058 | - | F | Corrections on test frequencies for NR CA band n260 in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198649 | 1059 | - | F | Corrections on test frequencies for NR CA band n261 in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198659 | 1062 | - | F | Update TCI State Cell parameter in Demod section | 16.2.0 |
| 2019-12 | RAN#86 | R5-198718 | 1067 | - | F | Updates to RSRP-Range, RSRQ-Range and SINR-Range | 16.2.0 |
| 2019-12 | RAN#86 | R5-198847 | 0941 | 1 | F | Corrections to DCI_1_1 configuration | 16.2.0 |
| 2019-12 | RAN#86 | R5-198848 | 0931 | 1 | F | Update IE PUSCH-TimeDomainResourceAllocationList | 16.2.0 |
| 2019-12 | RAN#86 | R5-198850 | 0934 | 1 | F | Correction to IE MasterKeyUpdate | 16.2.0 |
| 2019-12 | RAN#86 | R5-198851 | 0935 | 1 | F | Update of NR SIBs | 16.2.0 |
| 2019-12 | RAN#86 | R5-198852 | 0936 | 1 | F | Correction to USIM configuration | 16.2.0 |
| 2019-12 | RAN#86 | R5-198853 | 0938 | 1 | F | Correction to IE ReportConfigInterRAT | 16.2.0 |
| 2019-12 | RAN#86 | R5-198854 | 0942 | 1 | F | Correction to Table 4.9.9.2.3-1 for Inter-system change from S1 mode to N1 mode in 5GMM-IDLE mode | 16.2.0 |
| 2019-12 | RAN#86 | R5-198856 | 0982 | 1 | F | Addition of frequency configurations for NR MFBI testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-198857 | 0992 | 1 | F | Editorial update IE CSI-AperiodicTriggerStateList | 16.2.0 |
| 2019-12 | RAN#86 | R5-198858 | 0998 | 1 | F | Editorial update IE ServingCellConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-198859 | 1000 | 1 | F | Editorial update IE SecurityAlgorithmConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-198860 | 1002 | 1 | F | Update IE SRS-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198861 | 1001 | 1 | F | Update of Generic Test Procedures for IMS Emergency call establishment 4.9.11 and 4.9.12 to reflect the fact that they can be used in multiple states and scenarios | 16.2.0 |
| 2019-12 | RAN#86 | R5-198862 | 1013 | 1 | F | Update IE ServingCellConfigCommon | 16.2.0 |
| 2019-12 | RAN#86 | R5-198863 | 1016 | 1 | F | Update IE CSI-RS-ResourceMapping | 16.2.0 |
| 2019-12 | RAN#86 | R5-198865 | 1032 | 1 | F | Update RRCReconfiguration | 16.2.0 |
| 2019-12 | RAN#86 | R5-198866 | 1033 | 1 | F | Update chapter 4.5.1 General | 16.2.0 |
| 2019-12 | RAN#86 | R5-198867 | 1034 | 1 | F | Update to PDU SESSION ESTABLISHMENT ACCEPT and Reference QoS flow descriptions to align EPS bearer id format | 16.2.0 |
| 2019-12 | RAN#86 | R5-198869 | 1061 | 1 | F | New Test Procedures for IMS Emergency call release | 16.2.0 |
| 2019-12 | RAN#86 | R5-198870 | 1063 | 1 | F | Update chapter 4.5.2 RRC_IDLE | 16.2.0 |
| 2019-12 | RAN#86 | R5-198871 | 1072 | 1 | F | Update of RRCReconfiguration for measurement configuration | 16.2.0 |
| 2019-12 | RAN#86 | R5-198955 | 0995 | 1 | F | Update procedure for NR RF CA testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-198956 | 0996 | 1 | F | Update procedure for EN-DC RF CA testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-198957 | 1036 | 1 | F | Update to 38.508-1 for DEMOD message contents | 16.2.0 |
| 2019-12 | RAN#86 | R5-198958 | 0991 | 1 | F | Update IE PUCCH-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-198959 | 0976 | 1 | F | Introduction of test frequencies for Rel-15 EN-DC inter-band configurations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198960 | 0980 | 1 | F | Introduction of test frequencies for Rel-16 NR inter-band CA configurations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198961 | 0987 | 1 | F | Introduction of test frequencies for NR configuration CA_n29A-n66A | 16.2.0 |
| 2019-12 | RAN#86 | R5-198962 | 1003 | 1 | F | Introduction of test frequencies for Rel-15 NR DC configurations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198997 | 1068 | 1 | F | Introduction of test frequencies and parameters for NR bands n29, n48 and n65 for protocol testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-199008 | 0997 | 1 | F | Editorial update WLAN table 4.5.2.2-3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199013 | 0943 | 1 | F | Correction to SMTC and GAP for inter frequency cell | 16.2.0 |
| 2019-12 | RAN#86 | R5-199015 | 0981 | 1 | F | Correction of test frequencies for NR CA and EN-DC protocol testing | 16.2.0 |
| 2019-12 | RAN#86 | R5-199016 | 1049 | 1 | F | Updates to Test Procedure 4.9.11 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199017 | 1050 | 1 | F | Updates to Test Procedure 4.9.12 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199020 | 1079 | - | F | Update default setting of deriveSSB-IndexFromCell | 16.2.0 |
| 2019-12 | RAN#86 | R5-199021 | 1011 | 1 | F | Update IE TDD-UL-DL-Config | 16.2.0 |
| 2019-12 | RAN#86 | R5-199022 | 1070 | 1 | F | Updates to Signalling Reference test conditions | 16.2.0 |
| 2019-12 | RAN#86 | R5-199026 | 1025 | 2 | F | Update IE CellGroupConfig | 16.2.0 |
| 2019-12 | RAN#86 | R5-199071 | 0944 | 2 | F | Update of SUL related messages | 16.2.0 |
| 2019-12 | RAN#86 | R5-199075 | 1080 | 1 | F | Correction to NR RRC_IDLE mode procedure | 16.2.0 |
| 2019-12 | RAN#86 | R5-199093 | 1065 | 2 | F | Update chapter 4.5.4 RRC_CONNECTED | 16.2.0 |
| 2019-12 | RAN#86 | R5-199094 | 1069 | 1 | F | Updates for handling of Multiple PDU sessions / Multiple DRBs | 16.2.0 |
| 2019-12 | RAN#86 | R5-199103 | 1076 | 2 | F | Adding new generic procedure for UE-requested PDU session modification after the first S1 to N1 mode change | 16.2.0 |
| 2019-12 | RAN#86 | R5-199300 | 1042 | 1 | F | Corrections on category of EN-DC configurations for test frequencies in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199301 | 1054 | 1 | F | Addition of ServingCellConfigCommon for RRM tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-199302 | 1057 | 1 | F | Corrections on test frequencies for NR CA band n257 in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199303 | 1060 | 1 | F | Corrections on test frequencies for NR CA band n258 in 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199304 | 1066 | 1 | F | Update Radio resource control information elements for RRM to add CSI-RS for Tracking | 16.2.0 |
| 2019-12 | RAN#86 | R5-199423 | 1077 | - | F | Update ra-responseWindow in TS 38.508-1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199481 | 0989 | 1 | F | Addition of FR1 NR CA and NR 4Rx connection diagrams | 16.2.0 |
| 2019-12 | RAN#86 | R5-199511 | 1078 | - | F | Update of quiet zone size | 16.2.0 |
| 2019-12 | RAN#86 | R5-199545 | 1020 | 1 | F | Addition of multi-AoA capabilities for IFF | 16.2.0 |
| 2020-03 | RAN#87 | R5-200135 | 1120 |  | F | Removal of Correction to SIG OTA UE Orientation procedure | 16.3.0 |
| 2020-03 | RAN#87 | R5-200147 | 1130 |  | F | Update to USIM config 6.4.1-11 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200244 | 1133 |  | F | Correction to nAndPagingFrameOffset | 16.3.0 |
| 2020-03 | RAN#87 | R5-200296 | 1136 |  | F | Addition of generic procedure for IMS MO speech setup | 16.3.0 |
| 2020-03 | RAN#87 | R5-200297 | 1137 |  | F | Addition of generic procedure for IMS MT speech setup | 16.3.0 |
| 2020-03 | RAN#87 | R5-200298 | 1138 |  | F | Addition of generic procedure for IMS MO call release | 16.3.0 |
| 2020-03 | RAN#87 | R5-200299 | 1139 |  | F | Addition of generic procedure for IMS MT call release | 16.3.0 |
| 2020-03 | RAN#87 | R5-200349 | 1142 |  | F | Correction to frequencyBandList in SIB4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200431 | 1146 |  | F | Correction to CSI-FrequencyOccupation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200432 | 1147 |  | F | Correction to default setting of additionalPmax | 16.3.0 |
| 2020-03 | RAN#87 | R5-200433 | 1148 |  | F | Correction to powerControlOffset for performance tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200434 | 1149 |  | F | Correction to RACH configuration for RRM tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200435 | 1150 |  | F | Correction to TDD UL-DL Config for performance test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200477 | 1154 |  | F | Update to Registration REQ and Authentication Response message | 16.3.0 |
| 2020-03 | RAN#87 | R5-200499 | 1157 |  | F | Correction of test frequency tables for NR band n1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200500 | 1158 |  | F | Correction of test frequency tables for NR band n2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200501 | 1159 |  | F | Correction of test frequency tables for NR band n3 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200502 | 1160 |  | F | Correction of test frequency tables for NR band n7 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200503 | 1161 |  | F | Correction of test frequency tables for NR band n25 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200504 | 1162 |  | F | Correction of test frequency tables for NR band n28 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200505 | 1163 |  | F | Correction of test frequency tables for NR band n34 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200506 | 1164 |  | F | Correction of test frequency tables for NR band n38 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200507 | 1165 |  | F | Correction of test frequency tables for NR band n39 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200508 | 1166 |  | F | Correction of test frequency tables for NR band n40 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200510 | 1168 |  | F | Correction of test frequency tables for NR band n50 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200511 | 1169 |  | F | Correction of test frequency tables for NR band n66 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200512 | 1170 |  | F | Correction of test frequency tables for NR band n70 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200513 | 1171 |  | F | Correction of test frequency tables for NR band n71 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200514 | 1172 |  | F | Correction of test frequency tables for NR band n74 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200515 | 1173 |  | F | Correction of test frequency tables for NR band n75 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200531 | 1189 |  | F | Correction of test frequency tables for NR band n29 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200532 | 1190 |  | F | Correction of test frequency tables for NR band n48 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200533 | 1191 |  | F | Correction of test frequency tables for NR band n65 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200597 | 1200 |  | F | Introduction of test frequencies for inter-band Rel-16 EN-DC configurations in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200605 | 1202 |  | F | Addition of test frequencies for n95 SUL band | 16.3.0 |
| 2020-03 | RAN#87 | R5-200645 | 1206 |  | F | Updates to 4.6.4 UE Capability Information Elements | 16.3.0 |
| 2020-03 | RAN#87 | R5-200646 | 1207 |  | F | Correction to QoS rule number 7 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200647 | 1208 |  | F | Correction to IMS emergency call release procedures | 16.3.0 |
| 2020-03 | RAN#87 | R5-201246 | 1209 | 1 | F | TRS configuration messages definition for RF in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200678 | 1210 |  | F | Update of IE ControlResourceSet to introduce band and channel bandwidth specific values for frequencyDomainResources | 16.3.0 |
| 2020-03 | RAN#87 | R5-200703 | 1213 |  | F | Correction to IE BeamFailureRecoveryConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200774 | 1215 |  | F | Editorial update IE MeasConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200775 | 1216 |  | F | Editorial update IE radioLinkMonitoringRS-Id | 16.3.0 |
| 2020-03 | RAN#87 | R5-200804 | 1218 |  | F | Correction of test frequency tables for NR band n5 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200805 | 1219 |  | F | Correction of test frequency tables for NR band n8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200806 | 1220 |  | F | Correction of test frequency tables for NR band n12 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200807 | 1221 |  | F | Correction of test frequency tables for NR band n20 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200808 | 1222 |  | F | Correction of test frequency tables for NR band n51 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200900 | 1197 | 1 | F | Corrections on test frequencies for EN-DC band combinations including FR1 and FR2 in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200901 | 1198 | 1 | F | Corrections on test frequencies for EN-DC band combinations including FR2 in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200902 | 1199 | 1 | F | Corrections on uplink EN-DC configurations for test frequencies in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200921 | 1132 | 1 | F | Addition of Rel-16 inter-band CA and EN-DC FR1 two bands test configurations | 16.3.0 |
| 2020-03 | RAN#87 | R5-200930 | 1081 | 1 | F | Update SIB1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200931 | 1082 | 1 | F | Update CounterCheck | 16.3.0 |
| 2020-03 | RAN#87 | R5-200932 | 1083 | 1 | F | Editorial update DLInformationTransfer | 16.3.0 |
| 2020-03 | RAN#87 | R5-200933 | 1084 | 1 | F | Editorial update FailureInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200934 | 1085 | 1 | F | Editorial update MeasurementReport | 16.3.0 |
| 2020-03 | RAN#87 | R5-200935 | 1086 | 1 | F | Editorial update MobilityFromNRCommand | 16.3.0 |
| 2020-03 | RAN#87 | R5-200936 | 1087 | 1 | F | Editorial update Paging | 16.3.0 |
| 2020-03 | RAN#87 | R5-200937 | 1088 | 1 | F | Editorial update RRCReestablishment | 16.3.0 |
| 2020-03 | RAN#87 | R5-200938 | 1090 | 1 | F | Editorial update RRCReconfigurationComplete | 16.3.0 |
| 2020-03 | RAN#87 | R5-200939 | 1091 | 1 | F | Editorial update RRCReject | 16.3.0 |
| 2020-03 | RAN#87 | R5-200940 | 1092 | 1 | F | Editorial update RRCRelease | 16.3.0 |
| 2020-03 | RAN#87 | R5-200941 | 1093 | 1 | F | Editorial update RRCResumeComplete | 16.3.0 |
| 2020-03 | RAN#87 | R5-200942 | 1094 | 1 | F | Editorial update RRCSetup | 16.3.0 |
| 2020-03 | RAN#87 | R5-200943 | 1096 | 1 | F | Editorial update SCGFailureInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200944 | 1097 | 1 | F | Editorial update SecurityMode | 16.3.0 |
| 2020-03 | RAN#87 | R5-200945 | 1098 | 1 | F | Update SystemInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200946 | 1099 | 1 | F | Editorial update UEAssistanceInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200947 | 1100 | 1 | F | Editorial update UECapability | 16.3.0 |
| 2020-03 | RAN#87 | R5-200948 | 1101 | 1 | F | Editorial update ULInformation | 16.3.0 |
| 2020-03 | RAN#87 | R5-200949 | 1103 | 1 | F | Editorial update IE RLC-BearerConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200951 | 1108 | 1 | F | Add IE TDD-UL-DL-ConfigDedicated | 16.3.0 |
| 2020-03 | RAN#87 | R5-200952 | 1111 | 1 | F | Update IE ServingCellConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200953 | 1112 | 1 | F | Update IE ServingCellConfigCommonSIB | 16.3.0 |
| 2020-03 | RAN#87 | R5-200954 | 1113 | 1 | F | Update IE DMRS-DownlinkConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-200955 | 1114 | 1 | F | Update IE FrequencyInfoUL | 16.3.0 |
| 2020-03 | RAN#87 | R5-200956 | 1118 | 1 | F | Update chapter 4.5.1 General | 16.3.0 |
| 2020-03 | RAN#87 | R5-200957 | 1122 | 1 | F | Update chapter 4.5.4 RRC_CONNECTED | 16.3.0 |
| 2020-03 | RAN#87 | R5-200958 | 1125 | 1 | F | Update IE CellGroupId | 16.3.0 |
| 2020-03 | RAN#87 | R5-200959 | 1126 | 1 | F | Update IE ServCellIndex | 16.3.0 |
| 2020-03 | RAN#87 | R5-200960 | 1127 | 1 | F | Update IE SK-Counter | 16.3.0 |
| 2020-03 | RAN#87 | R5-200961 | 1128 | 1 | F | Update IE SDAP-Config | 16.3.0 |
| 2020-03 | RAN#87 | R5-200965 | 1145 | 1 | F | Correction to CORESET and search space configuration | 16.3.0 |
| 2020-03 | RAN#87 | R5-200966 | 1193 | 1 | F | Addition of NR SUL connection diagrams | 16.3.0 |
| 2020-03 | RAN#87 | R5-200967 | 1201 | 1 | F | Clarification to high test channel bandwidth table | 16.3.0 |
| 2020-03 | RAN#87 | R5-200968 | 1203 | 1 | F | Addition of missing EN-DC test frequencies | 16.3.0 |
| 2020-03 | RAN#87 | R5-200996 | 1124 | 1 | F | Correction to PUCCH-Config for Format1 and Format2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201005 | 1131 | 1 | F | Update of Annex C on calculation of test frequencies to achieve full bandwidth testing of NR bands | 16.3.0 |
| 2020-03 | RAN#87 | R5-201020 | 1155 | 1 | F | Update SIG test frequencies in clause 6.2.3.x | 16.3.0 |
| 2020-03 | RAN#87 | R5-201021 | 1167 | 1 | F | Correction of test frequency tables for NR band n41 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201022 | 1174 | 1 | F | Correction of test frequency tables for NR band n77 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201023 | 1175 | 1 | F | Correction of test frequency tables for NR band n78 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201024 | 1176 | 1 | F | Correction of test frequency tables for NR band n79 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201025 | 1177 | 1 | F | Correction of test frequency tables for NR band n257 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201026 | 1178 | 1 | F | Correction of test frequency tables for NR band n258 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201027 | 1179 | 1 | F | Correction of test frequency tables for NR band n260 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201028 | 1180 | 1 | F | Correction of test frequency tables for NR band n261 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201029 | 1192 | 1 | F | Update of clause 4.4.2 on simulated cells | 16.3.0 |
| 2020-03 | RAN#87 | R5-201061 | 1153 | 1 | F | Addition of a few R16s inter-band EN-DC FR1 test configurations | 16.3.0 |
| 2020-03 | RAN#87 | R5-201065 | 1194 | 1 | F | Addition of test channel bandwidth for NR bands in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201092 | 1123 | 1 | F | Updates to NR FR1 and LTE Power levels in OTA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201093 | 1224 | 1 | F | Message content Updates for Carrier Aggregation | 16.3.0 |
| 2020-03 | RAN#87 | R5-201108 | 1143 | 1 | F | Correction to EUTRA-AllowedMeasBandwidth | 16.3.0 |
| 2020-03 | RAN#87 | R5-201116 | 1204 | 1 | F | Updates to 4.7.3 Contents of EAP-AKA messages in 38.508-1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201148 | 1134 | 1 | F | Updates to default SSB index of intra-frequency NR cells | 16.3.0 |
| 2020-03 | RAN#87 | R5-201159 | 1151 | 1 | F | Correction to test frequencies for n257 intra-band contiguous CA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201173 | 1117 | 1 | F | Update IE TDD-UL-DL-Config | 16.3.0 |
| 2020-03 | RAN#87 | R5-201174 | 1095 | 1 | F | Update RRCSystemInfoRequest | 16.3.0 |
| 2020-03 | RAN#87 | R5-201175 | 1106 | 1 | F | Update IE RLF-TimersAndConstants | 16.3.0 |
| 2020-03 | RAN#87 | R5-201176 | 1107 | 1 | F | Update IE SCS-SpecificCarrier | 16.3.0 |
| 2020-03 | RAN#87 | R5-201177 | 1109 | 1 | F | Update chapter 4.6.0 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201179 | 1116 | 1 | F | Update IE MeasObjectNR | 16.3.0 |
| 2020-03 | RAN#87 | R5-201189 | 1214 | 1 | F | Addition of IFF DFF Hybrid Setup for FR2 2AoA RRM test | 16.3.0 |
| 2020-03 | RAN#87 | R5-201194 | 1141 | 1 | F | Update to Common Coreset RB IE and section 5-6 Demod message contents | 16.3.0 |
| 2020-03 | RAN#87 | R5-201195 | 1144 | 1 | F | Update of DCI 1_0 and DCI_1_1 configuration | 16.3.0 |
| 2020-03 | RAN#87 | R5-201197 | 1152 | 1 | F | Correction to TRS configuration for RRM tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-201202 | 1205 | 1 | F | Update to Switch Off/ Power off procedure in RRC_CONNECTED mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201203 | 1129 | 1 | F | Update to PDCP-Config | 16.3.0 |
| 2020-03 | RAN#87 | R5-201217 | 1217 | 1 | F | Updates to PsDU session modification procedures | 16.3.0 |
| 2020-03 | RAN#87 | R5-201221 | 1089 | 1 | F | Update RRCReconfiguration | 16.3.0 |
| 2020-03 | RAN#87 | R5-201222 | 1121 | 1 | F | Update IE CellGroupConfig | 16.3.0 |
| 2020-03 | RAN#87 | R5-201232 | 1140 | 2 | F | CR to 38.508-1 to introduce DFF Range Length | 16.3.0 |
| 2020-03 | RAN#87 | R5-201234 | 1110 | 2 | F | Update IE ServingCellConfigCommon | 16.3.0 |
| 2020-03 | RAN#87 | R5-201148 | 1134 | 1 | F | Add new missing column of Table 4.4.2-2 | 16.3.1 |
| 2020-06 | RAN#88 | R5-201320 | 1225 | - | F | Update IE CellGroupConfig | 16.4.0 |
| 2020-06 | RAN#88 | R5-201322 | 1227 | - | F | Update of default value of frequencyDomainResources in ControlResourceSet IE | 16.4.0 |
| 2020-06 | RAN#88 | R5-201331 | 1228 | - | F | Correction to Table 4.9.6.1-1-Switch off in Idle | 16.4.0 |
| 2020-06 | RAN#88 | R5-201333 | 1230 | - | F | Addition of 4.9.6.3A Switch off Power off procedure in RRC_CONNECTED with T3540 started | 16.4.0 |
| 2020-06 | RAN#88 | R5-201335 | 1232 | - | F | Update to USIM config 6.4.1-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201336 | 1233 | - | F | Update to USIM Table 6.4.1-10 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201337 | 1234 | - | F | Correction to Table 7.3.1-7 NZP-CSI-RS-Resource for TRS | 16.4.0 |
| 2020-06 | RAN#88 | R5-201446 | 1235 | - | F | Fixing wrong reference for RRC_CONNECTED state on WLAN access | 16.4.0 |
| 2020-06 | RAN#88 | R5-201572 | 1253 | - | F | Corrections to default content of DCI messages | 16.4.0 |
| 2020-06 | RAN#88 | RP-201138 | 1258 | 1 | F | Correction to IE SearchSpace | 16.4.0 |
| 2020-06 | RAN#88 | R5-201731 | 1265 | - | F | Addition of NR SUL connection diagram in A.3.1.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201800 | 1267 | - | F | Addition of USIM configuration for TC 6.3.1.8 and TC 6.3.1.9 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201837 | 1268 | - | F | Update of test channel bandwidths for band n48 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201932 | 1271 | - | F | Removing brackets from mid test channel BWs for FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201956 | 1273 | - | F | Correction of clause 4.4.2 on simulated cells | 16.4.0 |
| 2020-06 | RAN#88 | R5-201958 | 1275 | - | F | Correction to Annex C on calculation of kSSB to align SSB and RMSI subcarriers | 16.4.0 |
| 2020-06 | RAN#88 | R5-201959 | 1276 | - | F | Removal of definition of frequencyDomainResources value dependent on CORESET#0 configuration in Annex C. | 16.4.0 |
| 2020-06 | RAN#88 | R5-201961 | 1278 | - | F | Correction of test frequency tables for NR band n1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201962 | 1279 | - | F | Correction of test frequency tables for NR band n2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201963 | 1280 | - | F | Correction of test frequency tables for NR band n3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201964 | 1281 | - | F | Correction of test frequency tables for NR band n5 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201965 | 1282 | - | F | Correction of test frequency tables for NR band n7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201966 | 1283 | - | F | Correction of test frequency tables for NR band n8 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201967 | 1284 | - | F | Correction of test frequency tables for NR band n12 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201968 | 1285 | - | F | Correction of test frequency tables for NR band n20 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201969 | 1286 | - | F | Correction of test frequency tables for NR band n25 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201970 | 1287 | - | F | Correction of test frequency tables for NR band n28 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201971 | 1288 | - | F | Correction of test frequency tables for NR band n34 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201972 | 1289 | - | F | Correction of test frequency tables for NR band n38 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201973 | 1290 | - | F | Correction of test frequency tables for NR band n39 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201974 | 1291 | - | F | Correction of test frequency tables for NR band n40 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201975 | 1292 | - | F | Correction of test frequency tables for NR band n41 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201976 | 1293 | - | F | Correction of test frequency tables for NR band n50 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201977 | 1294 | - | F | Correction of test frequency tables for NR band n51 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201978 | 1295 | - | F | Correction of test frequency tables for NR band n66 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201979 | 1296 | - | F | Correction of test frequency tables for NR band n70 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201980 | 1297 | - | F | Correction of test frequency tables for NR band n71 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201981 | 1298 | - | F | Correction of test frequency tables for NR band n74 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201982 | 1299 | - | F | Correction of test frequency tables for NR band n77 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201983 | 1300 | - | F | Correction of test frequency tables for NR band n78 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201984 | 1301 | - | F | Correction of test frequency tables for NR band n79 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201989 | 1306 | - | F | Corrections of test frequency tables for CA_n41C | 16.4.0 |
| 2020-06 | RAN#88 | R5-201990 | 1307 | - | F | Corrections of test frequency tables for CA_n78C | 16.4.0 |
| 2020-06 | RAN#88 | R5-201991 | 1308 | - | F | Editorial correction to test frequency clause numbering | 16.4.0 |
| 2020-06 | RAN#88 | R5-201995 | 1312 | - | F | Correction of test frequency tables for CA_n66B | 16.4.0 |
| 2020-06 | RAN#88 | R5-201997 | 1314 | - | F | Introduction of test frequencies for NR band n26 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201998 | 1315 | - | F | Introduction of test frequencies for NR band 26 for protocol testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-201999 | 1316 | - | F | Correction of test frequency tables for NR band n29 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202000 | 1317 | - | F | Correction of test frequency tables for NR band n48 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202001 | 1318 | - | F | Correction of test frequency tables for NR band n65 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202024 | 1327 | - | F | Update IE PDCP-Config | 16.4.0 |
| 2020-06 | RAN#88 | R5-202123 | 1330 | - | F | CR to 38.508-1 to clarify the test zone/quiet zone | 16.4.0 |
| 2020-06 | RAN#88 | R5-202186 | 1333 | - | F | Addition of locationAndBandwidth in BWP for FR1 in 38.508-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202210 | 1335 | - | F | Corrections on test frequencies for inter-band EN-DC configurations within FR1 for five bands in 38.508-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202212 | 1336 | - | F | Corrections of test frequency tables for CA_n257x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202213 | 1337 | - | F | Corrections of test frequency tables for CA_n258x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202214 | 1338 | - | F | Corrections of test frequency tables for CA_n260x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202215 | 1339 | - | F | Corrections of test frequency tables for CA_n261x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202244 | 1343 | - | F | Correction to nrofRBs under TRS CSI-FrequencyOccupation for Demod test cases with 10 MHz CBW | 16.4.0 |
| 2020-06 | RAN#88 | R5-202284 | 1351 | - | F | Correction to configuration bwp-id parameter in TCI-State IE | 16.4.0 |
| 2020-06 | RAN#88 | R5-202409 | 1354 | - | F | Update PDCCH-ControlResourceSet for RRM testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202410 | 1355 | - | F | Addition Physical Layer Parameter section for RRM testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202449 | 1356 | - | F | Correction of test frequencies for DC n71AA | 16.4.0 |
| 2020-06 | RAN#88 | R5-202486 | 1358 | - | F | Correction to content of EF5GSN3GPPLOCI | 16.4.0 |
| 2020-06 | RAN#88 | R5-202525 | 1360 | - | F | Correction to System Information Combination for PWS | 16.4.0 |
| 2020-06 | RAN#88 | R5-202531 | 1353 | 1 | F | Addition of R16 new channel bandwidths for n1 in 38.508-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202534 | 1266 | 1 | F | Removal of USIM configuration 14 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202549 | 1350 | 1 | F | Aligning the tabular representation of ASN.1 with PRD13 sections 4.8 and 5 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202554 | 1252 | 1 | F | Updates to PDCCH-ConfigCommon | 16.4.0 |
| 2020-06 | RAN#88 | R5-202561 | 1226 | 1 | F | Update IE default content for control resource set establishment and common search space mapping | 16.4.0 |
| 2020-06 | RAN#88 | R5-202562 | 1229 | 1 | F | Update to 4.9.6.3 Switch off Power off procedure in RRC_CONNECTED | 16.4.0 |
| 2020-06 | RAN#88 | R5-202563 | 1231 | 1 | F | Correction to Table 4.9.7.2.3-1-Tracking Area Update Request | 16.4.0 |
| 2020-06 | RAN#88 | R5-202564 | 1236 | 1 | F | Correction to Table 4.5.2.2-2-Adding second SMC procedure for Selected EPS NAS security algorithms IE | 16.4.0 |
| 2020-06 | RAN#88 | R5-202565 | 1237 | 1 | F | Clarification to ROUND for negative Threshold values in SIB1 and SIB4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202573 | 1251 | 1 | F | Updates to test frequency definitions for SDL NR bands | 16.4.0 |
| 2020-06 | RAN#88 | R5-202574 | 1257 | 1 | F | Correction to condition SRB_NR_PDCP in RadioBearerConfig | 16.4.0 |
| 2020-06 | RAN#88 | R5-202575 | 1260 | 1 | F | Correction to UECapabilityEnquiry in case of EN-DC interband CA | 16.4.0 |
| 2020-06 | RAN#88 | R5-202576 | 1262 | 1 | F | Updates to PDCP-Config | 16.4.0 |
| 2020-06 | RAN#88 | R5-202577 | 1274 | 1 | F | Clarifications of Annex C on calculation of test frequencies | 16.4.0 |
| 2020-06 | RAN#88 | R5-202578 | 1277 | 1 | F | Update SIG test frequencies in clause 6.2.3.x | 16.4.0 |
| 2020-06 | RAN#88 | R5-202579 | 1302 | 1 | F | Correction of test frequency tables for NR band n257 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202580 | 1303 | 1 | F | Correction of test frequency tables for NR band n258 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202581 | 1304 | 1 | F | Correction of test frequency tables for NR band n260 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202582 | 1305 | 1 | F | Correction of test frequency tables for NR band n261 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202583 | 1322 | 1 | F | Introduction of protocol testing applicability for EN-DC inter-band, NR-CA inter-band and NR DC test frequency tables | 16.4.0 |
| 2020-06 | RAN#88 | R5-202585 | 1331 | 1 | F | Correction to Reference QoS rules | 16.4.0 |
| 2020-06 | RAN#88 | R5-202586 | 1346 | 1 | F | Updates to Generic Test Procedure for IMS MT speech call establishment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202587 | 1347 | 1 | F | Updates to Generic Test Procedure for IMS MO call release | 16.4.0 |
| 2020-06 | RAN#88 | R5-202588 | 1349 | 1 | F | Aligning the tabular representation of ASN.1 with PRD13 section 4.6 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202589 | 1359 | 1 | F | Update the default USIM configurations | 16.4.0 |
| 2020-06 | RAN#88 | R5-202590 | 1361 | 1 | F | Addition of Generic procedure to check user plane connectivity for CA tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202703 | 1255 | 1 | F | Clarifications on the QoQZ validation procedure for RRM | 16.4.0 |
| 2020-06 | RAN#88 | R5-202708 | 1254 | 1 | F | TRS - PowerControlOffset correction for UE RF testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202820 | 1352 | 1 | F | Correction to PRB-Id for secondHopPRB | 16.4.0 |
| 2020-06 | RAN#88 | R5-202859 | 1329 | 1 | F | Updates on FR2 inter-band EN-DC configurations for test frequencies in 38.508-1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202879 | 1332 | 1 | F | Addition of BW to Table 4.6.3-33 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202880 | 1334 | 1 | F | Clarification of disabling Tx diversity for FR2 UE | 16.4.0 |
| 2020-06 | RAN#88 | R5-202881 | 1341 | 1 | F | Restructuring 38.508-1 message contents for Demod and CSI reporting test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202882 | 1342 | 1 | F | Update of PUCCH-ResourceId for Demod test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202883 | 1348 | 1 | F | Configuration of p-ZP-CSI-RS-ResourceSet for PDSCH Demod test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202956 | 1270 | 1 | F | Update of default test channel BW | 16.4.0 |
| 2020-06 | RAN#88 | R5-202962 | 1269 | 2 | F | Updating DCI related messages | 16.4.0 |
| 2020-06 | RAN#88 | R5-202967 | 1344 | 1 | F | Update to default value of PDSCH-to-HARQ_feedback timing indicator (k1) | 16.4.0 |
| 2020-06 | RAN#88 | R5-203056 | 1313 | 1 | F | Introduction of test frequencies for Rel-16 NR CA configuration CA_n66B and CA_n66(2A) in cl 6.2.3.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-203057 | 1319 | 1 | F | Addition of test frequencies for additional channel bandwidths for NR band n66 | 16.4.0 |
| 2020-06 | RAN#88 | R5-203078 | 1325 | 1 | F | Updates to Generic Test Procedure for IMS MO speech call establishment | 16.4.0 |
| 2020-06 | RAN#88 | R5-203079 | 1326 | 2 | F | Update NR-DC in chapter 4 | 16.4.0 |
| 2020-09 | RAN#89 | R5-203275 | 1364 | - | F | Editorial update IE ARFCN-ValueNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-203277 | 1366 | - | F | Add IEs ARFCN-ValueUTRA-FDD, AvailabilityCombinationsPerCell, AvailabilityIndicator and BAP-Routing-ID | 16.5.0 |
| 2020-09 | RAN#89 | R5-203278 | 1367 | - | F | n26 Default CH BW in 38.508-1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203287 | 1368 | - | F | Correction PRB-Id for PUCCH secondHopPRB | 16.5.0 |
| 2020-09 | RAN#89 | R5-203320 | 1369 | - | F | Add IE BeamFailureRecoverySCellConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203339 | 1372 | - | F | Add IE CGI-InfoEUTRALogging | 16.5.0 |
| 2020-09 | RAN#89 | R5-203340 | 1373 | - | F | Add IEs CGI-Info-Logging and CLI-RSSI-Range | 16.5.0 |
| 2020-09 | RAN#89 | R5-203341 | 1374 | - | F | Add IEs CommonLocationInfo, CondReconfigId, CondReconfigToAddModList and ConditionalReconfiguration | 16.5.0 |
| 2020-09 | RAN#89 | R5-203342 | 1375 | - | F | Add IEs ConfiguredGrantConfigIndex and ConfiguredGrantConfigIndexMAC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203343 | 1376 | - | F | Add IE DRX-ConfigSecondaryGroup | 16.5.0 |
| 2020-09 | RAN#89 | R5-203344 | 1377 | - | F | Add IE HighSpeedConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203345 | 1378 | - | F | Add IE InvalidSymbolPattern | 16.5.0 |
| 2020-09 | RAN#89 | R5-203346 | 1379 | - | F | Add IEs LBT-FailureRecoveryConfig and LocationInfo | 16.5.0 |
| 2020-09 | RAN#89 | R5-203347 | 1380 | - | F | Add IE MeasIdleConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203348 | 1381 | - | F | Add IE MeasObjectCLI | 16.5.0 |
| 2020-09 | RAN#89 | R5-203349 | 1382 | - | F | Add IE MeasObjectNR-SL | 16.5.0 |
| 2020-09 | RAN#89 | R5-203350 | 1383 | - | F | Add IE MeasObjectUTRA-FDD | 16.5.0 |
| 2020-09 | RAN#89 | R5-203351 | 1384 | - | F | Add IEs MeasResultIdleEUTRA and MeasResultIdleNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-203355 | 1387 | - | F | Add IEs MsgA-ConfigCommon and MsgA-PUSCH-Config | 16.5.0 |
| 2020-09 | RAN#89 | R5-203356 | 1388 | - | F | Add IEs NeedForGapsConfigNR and NeedForGapsInfoNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-203357 | 1389 | - | F | Correction to Table 4.5.2.2-2-second SMC procedure for Selected EPS NAS security algorithms IE | 16.5.0 |
| 2020-09 | RAN#89 | R5-203359 | 1391 | - | F | Correction to Table 4.6.3-141 ReportConfigInterRAT | 16.5.0 |
| 2020-09 | RAN#89 | R5-203446 | 1395 | - | F | Add IEs NPN-Identity and NPN-IdentityInfoList | 16.5.0 |
| 2020-09 | RAN#89 | R5-203449 | 1396 | - | F | Add IE PLMN-IdentityList2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203450 | 1397 | - | F | Add IE PUCCH-ConfigurationList | 16.5.0 |
| 2020-09 | RAN#89 | R5-203451 | 1398 | - | F | Add IE PUCCH-SpatialRelationInfo-Id | 16.5.0 |
| 2020-09 | RAN#89 | R5-203455 | 1399 | - | F | Corrections to 4.5.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203456 | 1400 | - | F | Updating usages of TS 34.229-1 to TS 34.229-5 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203467 | 1401 | - | F | Add IE RACH-ConfigCommonTwoStepRA | 16.5.0 |
| 2020-09 | RAN#89 | R5-203470 | 1402 | - | F | Add IE RACH-ConfigGenericTwoStepRA | 16.5.0 |
| 2020-09 | RAN#89 | R5-203471 | 1403 | - | F | Add IE ReferenceTimeInfo | 16.5.0 |
| 2020-09 | RAN#89 | R5-203472 | 1404 | - | F | Add IE RepetitionSchemeConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203476 | 1405 | - | F | Add IE ReportConfigNR-SL | 16.5.0 |
| 2020-09 | RAN#89 | R5-203500 | 1410 | - | F | Update to Table 4.6.3-74: MeasObjectEUTRA | 16.5.0 |
| 2020-09 | RAN#89 | R5-203506 | 1411 | - | F | Add IE RSSI-Range | 16.5.0 |
| 2020-09 | RAN#89 | R5-203507 | 1412 | - | F | Add IEs SemiStaticChannelAccessConfig and Sensor-LocationInfo | 16.5.0 |
| 2020-09 | RAN#89 | R5-203508 | 1413 | - | F | Add IE SI-RequestConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203509 | 1414 | - | F | Add IEs SPS-ConfigIndex, SPS-PUCCH-AN and SPS-PUCCH-AN-List | 16.5.0 |
| 2020-09 | RAN#89 | R5-203510 | 1415 | - | F | Add IE SRS-RSRP-Range | 16.5.0 |
| 2020-09 | RAN#89 | R5-203533 | 1417 | - | F | Update to PDSCH-ServingCellConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203534 | 1418 | - | F | Updates to CellGroupConfig and RNTI-Value for NR-DC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203557 | 1420 | - | F | Add IEs UL-DelayValueConfig and UplinkCancellation | 16.5.0 |
| 2020-09 | RAN#89 | R5-203575 | 1422 | - | F | Add chapter Positioning System information blocks | 16.5.0 |
| 2020-09 | RAN#89 | R5-203577 | 1423 | - | F | Add IEs SIB10, SIB11, SIB12, SIB13 and SIB14 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203582 | 1424 | - | F | Add messages DedicatedSIBRequest, DLDedicatedMessageSegment, DLInformationTransferMRDC and IABOtherInformation | 16.5.0 |
| 2020-09 | RAN#89 | R5-203634 | 1429 | - | F | Introduction of test frequencies for additional Rel-16 EN-DC inter-band configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-203662 | 1432 | - | F | Removal of USIM profile #16 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203671 | 1433 | - | F | Update of PDSCH-to-HARQ_feedback timing indicator (k1) value | 16.5.0 |
| 2020-09 | RAN#89 | R5-203681 | 1436 | - | F | Editorial correction typos in annex C.2.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203704 | 1438 | - | F | Correction to Table 4.6.3-87 NZP-CSI-RS-ResourceSet | 16.5.0 |
| 2020-09 | RAN#89 | R5-203719 | 1441 | - | F | Add messages LoggedMeasurementConfiguration, MCGFailureInformation and SidelinkUEInformationNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-203725 | 1442 | - | F | Add messages UEInformationRequest, UEInformationResponse, ULDedicatedMessageSegment and ULInformationTransferIRAT | 16.5.0 |
| 2020-09 | RAN#89 | R5-203729 | 1443 | - | F | Update IE RACH-ConfigGeneric | 16.5.0 |
| 2020-09 | RAN#89 | R5-203730 | 1444 | - | F | Scheduling Request Resource config for RRM test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203731 | 1445 | - | F | OSI scheduling config for RRM test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203755 | 1447 | - | F | Update IE SchedulingRequestResourceConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-203767 | 1449 | - | F | Addition of test frequencies for new Rel-16 CBW for NR band n77 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203768 | 1450 | - | F | Addition of test frequencies for new Rel-16 CBW for NR band n78 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203769 | 1451 | - | F | Introduction of test frequencies for Rel-16 NR band n30 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203793 | 1454 | - | F | Correction of n29 test frequencies for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203794 | 1455 | - | F | Introduction of n30 test frequencies for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203796 | 1456 | - | F | Correction of EN-DC test frequency information for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203813 | 1457 | - | F | Correction to PUCCH-Config | 16.5.0 |
| 2020-09 | RAN#89 | R5-203815 | 1458 | - | F | FR2 PUSCH K2 values alignment to TS 38.214 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203908 | 1463 | - | F | Corrections on test frequencies for NR FR2 CA band n261 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203998 | 1467 | - | F | Addition of test frequencies for new Rel-16 CBW 25 and 50 MHz for NR band n1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204021 | 1469 | - | F | Correction of n51 and n76 test frequencies for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-204032 | 1470 | - | F | Introduction of n259 test frequencies for protocol testing | 16.5.0 |
| 2020-09 | RAN#89 | R5-204038 | 1471 | - | F | Corrections of test frequency tables for EN-DC configuration DC_(n)41AA | 16.5.0 |
| 2020-09 | RAN#89 | R5-204039 | 1472 | - | F | Corrections of test frequency tables for EN-DC configuration DC_(n)71AA | 16.5.0 |
| 2020-09 | RAN#89 | R5-204041 | 1473 | - | F | Addition of test channel bandwidths for n1 new CBW in 38.508-1 R16 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204049 | 1475 | - | F | Correction to the procedure for determination of SSB and CORESET0 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204052 | 1478 | - | F | PUCCH Resource ID for CSI TCs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204053 | 1479 | - | F | Correction to PDCCH-ConfigCommon for performance tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204150 | 1484 | - | F | Update Table 5.4.2.0-2: ServingCellConfigCommon | 16.5.0 |
| 2020-09 | RAN#89 | R5-204167 | 1488 | - | F | Corrections of test frequency tables for CA_n260(A-I) | 16.5.0 |
| 2020-09 | RAN#89 | R5-204168 | 1489 | - | F | Update Table 7.3.1-4: ServingCellConfigCommon | 16.5.0 |
| 2020-09 | RAN#89 | R5-204223 | 1491 | - | F | Update missing SMTC configurations in RRM message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204238 | 1493 | - | F | Correction to default contents of RRCReestablishmentRequest message | 16.5.0 |
| 2020-09 | RAN#89 | R5-204325 | 1495 | - | F | Adding procedure for establishment of multiple additional PDN connections in EPS (S1 mode) | 16.5.0 |
| 2020-09 | RAN#89 | R5-204327 | 1497 | - | F | Updates to Test procedure for UE for Tracking area updating / Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode | 16.5.0 |
| 2020-09 | RAN#89 | R5-204329 | 1499 | - | F | Update of 4.5A.2 UE-requested PDU session establishment procedure | 16.5.0 |
| 2020-09 | RAN#89 | R5-204330 | 1500 | - | F | Updates Procedure to UE-requested PDU session modification after the first S1 to N1 mode change | 16.5.0 |
| 2020-09 | RAN#89 | R5-204331 | 1501 | - | F | Void 4.9.14 Procedure for UE-requested PDU session modification after the first S1 to N1 mode change | 16.5.0 |
| 2020-09 | RAN#89 | R5-204346 | 1502 | - | F | Introduction of test frequencies for Rel-16 NR band n259 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204378 | 1427 | 1 | F | Correction to NR inter-band CA configurations in FR1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204384 | 1409 | 1 | F | Correction to test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 16.5.0 |
| 2020-09 | RAN#89 | R5-204386 | 1390 | 1 | F | Correction to Table 4.6.1-17 RRCResume | 16.5.0 |
| 2020-09 | RAN#89 | R5-204387 | 1406 | 1 | F | Corrections to generic procedures for MO and MT speech call establishment | 16.5.0 |
| 2020-09 | RAN#89 | R5-204388 | 1407 | 1 | F | Correction to USIM configurations 7 and 13 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204389 | 1408 | 1 | F | Correction to switch off / power off procedures for IMS | 16.5.0 |
| 2020-09 | RAN#89 | R5-204390 | 1421 | 1 | F | Correction of description of NGEN-DC in table 4.5.1-1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204391 | 1425 | 1 | F | Correction to UE-CapabilityRAT-RequestList and UE-CapabilityRequestFilterNR | 16.5.0 |
| 2020-09 | RAN#89 | R5-204392 | 1439 | 1 | F | Addition of Generic Test Procedure for IMS MO SMS in 5GC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204393 | 1440 | 1 | F | Addition of Generic Test Procedure for IMS MT SMS in 5GC | 16.5.0 |
| 2020-09 | RAN#89 | R5-204394 | 1446 | 1 | F | Update IE RLC-BearerConfig | 16.5.0 |
| 2020-09 | RAN#89 | R5-204395 | 1481 | 1 | F | Update IE SIB2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204396 | 1482 | 1 | F | New procedure for PDU Session Release | 16.5.0 |
| 2020-09 | RAN#89 | R5-204397 | 1483 | 1 | F | Update to FreqBandList | 16.5.0 |
| 2020-09 | RAN#89 | R5-204398 | 1490 | 1 | F | Update IE ServingCellConfigCommon | 16.5.0 |
| 2020-09 | RAN#89 | R5-204399 | 1494 | 1 | F | Adding generic procedure E-UTRA RRC_IDLE with unrestricted number of PDN connections | 16.5.0 |
| 2020-09 | RAN#89 | R5-204400 | 1496 | 1 | F | Update of PDU SESSION ESTABLISHMENT ACCEPT for multi PDU-PDN handling | 16.5.0 |
| 2020-09 | RAN#89 | R5-204401 | 1498 | 1 | F | Updates of Test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 16.5.0 |
| 2020-09 | RAN#89 | R5-204507 | 1371 | 1 | F | Add IEs BH-RLC-ChannelConfig, BH-LogicalChannelIdentity, BH-LogicalChannelIdentity-Ext and BH-RLC-ChannelID | 16.5.0 |
| 2020-09 | RAN#89 | R5-204508 | 1386 | 1 | F | Add IEs MeasResultsSL and MeasTriggerQuantityEUTRA | 16.5.0 |
| 2020-09 | RAN#89 | R5-204509 | 1416 | 1 | F | Modification to InterRAT-Parameters to add the UE capability nr-HO-ToEN-DC-r16 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204510 | 1419 | 1 | F | Add IE SSB-PositionQCL-Relation | 16.5.0 |
| 2020-09 | RAN#89 | R5-204704 | 1503 | - | F | Adding the test frequency for DC_3A-7A_n78A | 16.5.0 |
| 2020-09 | RAN#89 | R5-204706 | 1504 | - | F | Adding the test frequency for DC_28A_n3A | 16.5.0 |
| 2020-09 | RAN#89 | R5-204708 | 1435 | 1 | F | Updating indicator for SUL FR1 test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204709 | 1437 | 1 | F | Update frequencyDomainResources and nrofcandidates | 16.5.0 |
| 2020-09 | RAN#89 | R5-204751 | 1428 | 1 | F | Introduction of test frequencies for additional Rel-15 band EN-DC inter-band configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-204752 | 1460 | 1 | F | Correction to TCI-state related configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-204753 | 1461 | 1 | F | Correction to CSI-RS related configurations | 16.5.0 |
| 2020-09 | RAN#89 | R5-204754 | 1468 | 1 | F | Update on test frequencies for EN-DC configurations including FR2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204755 | 1485 | 1 | F | Corrections of test frequency tables for CA_n258x | 16.5.0 |
| 2020-09 | RAN#89 | R5-204756 | 1486 | 1 | F | Corrections of test frequency tables for CA_n260x | 16.5.0 |
| 2020-09 | RAN#89 | R5-204757 | 1487 | 1 | F | Corrections of test frequency tables for CA_n261x | 16.5.0 |
| 2020-09 | RAN#89 | R5-204758 | 1492 | 1 | F | Add SSB subcarrier spacing configurations in RRM message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204800 | 1362 | 1 | F | Introduction of test frequencies for Rel-16 inter-band EN-DC combinations within FR1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204852 | 1434 | 1 | F | Jumbo CR for update to Demod message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204896 | 1370 | 1 | F | Addition of test frequencies for n28 with CBW of 30MHz | 16.5.0 |
| 2020-09 | RAN#89 | R5-204899 | 1448 | 1 | F | Adding default value for IE rbg-Size for demodulation and CSI reporting tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204900 | 1462 | 1 | F | Introduction of definition of Mid, Low, High test channel bandwidth and removal of NOTEs that incorrectly permit UE not to support mandatory BWs | 16.5.0 |
| 2020-09 | RAN#89 | R5-204901 | 1476 | 1 | F | Correction to message configuration for performance tests | 16.5.0 |
| 2020-10 | RAN#89 | R5-204325 | 1495 | - | F | Addition of missing Table 4.5A.2B.2.2-2 and specific message contents of R5-204325 | 16.5.1 |
| 2020-12 | RAN#90 | R5-205093 | 1507 | - | F | Add IE BandCombinationListSidelink | 16.6.0 |
| 2020-12 | RAN#90 | R5-205096 | 1508 | - | F | Add IE CarrierAggregationVariant | 16.6.0 |
| 2020-12 | RAN#90 | R5-205104 | 1509 | - | F | Add IEs FreqSeparationClassDL-Only and HighSpeedParameters | 16.6.0 |
| 2020-12 | RAN#90 | R5-205130 | 1514 | - | F | Add IE PowSav-Parameters | 16.6.0 |
| 2020-12 | RAN#90 | R5-205167 | 1519 | - | F | Add IE OLPC-SRS-Pos | 16.6.0 |
| 2020-12 | RAN#90 | R5-205172 | 1521 | - | F | Add IEs SidelinkParameters, SON-Parameters and SpatialRelationsSRS-Pos | 16.6.0 |
| 2020-12 | RAN#90 | R5-205178 | 1522 | - | F | Add IE UE-BasedPerfMeas-Parameters | 16.6.0 |
| 2020-12 | RAN#90 | R5-205181 | 1523 | - | F | Add IE SharedSpectrumChAccessParamsPerBand | 16.6.0 |
| 2020-12 | RAN#90 | R5-205186 | 1524 | - | F | Add IEs AbsoluteTimeInfo, AreaConfiguration and BT-NameList | 16.6.0 |
| 2020-12 | RAN#90 | R5-205187 | 1525 | - | F | Add IEs IAB-IP-Address, IAB-IP-AddressIndex and IAB-IP-Usage | 16.6.0 |
| 2020-12 | RAN#90 | R5-205188 | 1526 | - | F | Add IEs LoggingDuration, LoggingInterval, LogMeasResultListBT and LogMeasResultListWLAN | 16.6.0 |
| 2020-12 | RAN#90 | R5-205189 | 1527 | - | F | Add IE PhysCellIdUTRA-FDD | 16.6.0 |
| 2020-12 | RAN#90 | R5-205190 | 1528 | - | F | Add IEs Sensor-NameList, TraceReference and UE-MeasurementsAvailable-r16 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205191 | 1529 | - | F | Add IEs UTRA-FDD-Q-OffsetRange, VisitedCellInfoList and WLAN-NameList | 16.6.0 |
| 2020-12 | RAN#90 | R5-205215 | 1533 | - | F | Update chapter 4.5.1 General | 16.6.0 |
| 2020-12 | RAN#90 | R5-205249 | 1537 | - | F | Introduction of test frequencies for NR Band n53 signalling testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-205250 | 1538 | - | F | Introduction of test channel BWs for NR Band n53 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205288 | 1539 | - | F | Addition of IE DCP-Config-r16 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205332 | 1542 | - | F | Updates to generic procedure NR-DC RRC_CONNECTED | 16.6.0 |
| 2020-12 | RAN#90 | R5-205334 | 1544 | - | F | Updates to RadioBearerConfig in Table 4.6.3-132 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205336 | 1546 | - | F | Updates to RRCReconfiguration in Table 4.6.1-13 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205339 | 1549 | - | F | Updates to RRCReconfiguration-NR-DC in Table 4.8.1-1CA | 16.6.0 |
| 2020-12 | RAN#90 | R5-205403 | 1554 | - | F | Addition of PC5 RRC messages for sidelink communication | 16.6.0 |
| 2020-12 | RAN#90 | R5-205404 | 1555 | - | F | Addition of sidelink IEs for Uu RRC and PC5 RRC | 16.6.0 |
| 2020-12 | RAN#90 | R5-205532 | 1573 | - | F | Clarifications to Annex C and CORESET1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205661 | 1580 | - | F | Update of Annex C on calculating test frequencies for RRM testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-205701 | 1588 | - | F | Update RF test channel bandwidths for n14 and n30 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205725 | 1594 | - | F | Correction to test channel bandwidth for NR band n40 and n50 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205728 | 1597 | - | F | Adding test frequencies for CA_n78B | 16.6.0 |
| 2020-12 | RAN#90 | R5-205771 | 1602 | - | F | Addition of test frequencies for a few Rel-16 EN-DC configurations | 16.6.0 |
| 2020-12 | RAN#90 | R5-205832 | 1603 | - | F | Connection diagrams for radiated RRM Tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-205874 | 1607 | - | F | Correction of test frequency of CA_n41C | 16.6.0 |
| 2020-12 | RAN#90 | R5-205875 | 1608 | - | F | Correction of test frequency of CA_n66B | 16.6.0 |
| 2020-12 | RAN#90 | R5-205881 | 1610 | - | F | Addition of test frequency for 40MHz of band n38 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205926 | 1612 | - | F | Correction to nrofRBs IE for CSI-FrequencyOccupation | 16.6.0 |
| 2020-12 | RAN#90 | R5-205932 | 1614 | - | F | Addition of IE configuration for ULFPTx to clause 5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205937 | 1615 | - | F | Update of 4.9.12 Generic Test Procedure for IMS Emergency call establishment in 5GC without IMS emergency registration and editorials | 16.6.0 |
| 2020-12 | RAN#90 | R5-205939 | 1617 | - | F | Update for Flexible PDU-PDN - Default messages | 16.6.0 |
| 2020-12 | RAN#90 | R5-205940 | 1618 | - | F | Update for Flexible PDU-PDN - DNN Configurations | 16.6.0 |
| 2020-12 | RAN#90 | R5-205997 | 1621 | - | F | Correction of test frequencies for NR band n1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205998 | 1622 | - | F | Editorial correction to NR-DC test frequency clause 4.3.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206002 | 1626 | - | F | Correction of FR1 NR band test frequency tables for protocol testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206003 | 1627 | - | F | Correction of FR2 NR band test frequency tables for protocol testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206004 | 1628 | - | F | Change of default SCS for NR CA test frequencies for FR2 protocol testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206008 | 1632 | - | F | Editorial correction to NR CA test frequencies for FR1 protocol testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206019 | 1637 | - | F | Correction of test frequencies for CA_n260 of intra-band non-contiguous A-I | 16.6.0 |
| 2020-12 | RAN#90 | R5-206046 | 1642 | - | F | Clarify usage of SSB-Ids for RRM test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206048 | 1644 | - | F | Clarification on the conditions in DCI format 1_1 table for RRM | 16.6.0 |
| 2020-12 | RAN#90 | R5-206060 | 1645 | - | F | Correction to 4.9.17 IMS MO release | 16.6.0 |
| 2020-12 | RAN#90 | R5-206061 | 1646 | - | F | Correction to 4.9.18 IMS MT release | 16.6.0 |
| 2020-12 | RAN#90 | R5-206065 | 1650 | - | F | Alignment of Rel-16 5GSM messages | 16.6.0 |
| 2020-12 | RAN#90 | R5-206066 | 1651 | - | F | Addition of new SSTs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206067 | 1652 | - | F | Update IE SIB2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206078 | 1654 | - | F | Update IE SIB4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206086 | 1655 | - | F | Addition of common message contents for sustained downlink data rate tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206087 | 1656 | - | F | Correction to Default RRM TRS qcl-info and PDCCH TCI State | 16.6.0 |
| 2020-12 | RAN#90 | R5-206112 | 1657 | - | F | Update requirements of test equipment for RF test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206113 | 1658 | - | F | Update requirements of test equipment for RRM tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206115 | 1659 | - | F | Update requirements of reference test conditions for RRM tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206157 | 1663 | - | F | CSI-measConfig applicable for RRM testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206158 | 1664 | - | F | Editorial update IE CellAccessRelatedInfo-EUTRA-5GC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206164 | 1666 | - | F | Editorial update IE CellAccessRelatedInfo-EUTRA-EPC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206266 | 1671 | - | F | Corrections to test procedures in subclause 4.9 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206288 | 1510 | 1 | F | Corrections to UE-requested PDU session establishment procedure | 16.6.0 |
| 2020-12 | RAN#90 | R5-206289 | 1515 | 1 | F | Update of 4.9.7 Test procedure for UE for Tracking area updating / Inter-system change from N1 mode to S1 mode in 5GMM/EMM-IDLE mode | 16.6.0 |
| 2020-12 | RAN#90 | R5-206290 | 1516 | 1 | F | Update of 4.9.9 Test procedure for UE for Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 16.6.0 |
| 2020-12 | RAN#90 | R5-206291 | 1517 | 1 | F | Update of 4.5.2 RRC_IDLE | 16.6.0 |
| 2020-12 | RAN#90 | R5-206292 | 1532 | 1 | F | Correction to Test procedure 4.9.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206293 | 1534 | 1 | F | Corrections to generic procedures regarding IMS usage | 16.6.0 |
| 2020-12 | RAN#90 | R5-206294 | 1540 | 1 | F | Update Generic Test Procedures for IMS MO, MT speech call | 16.6.0 |
| 2020-12 | RAN#90 | R5-206295 | 1543 | 1 | F | Updates to generic procedure parameters in Table 4.5.1-1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206296 | 1616 | 1 | F | Update for Flexible PDU-PDN - Session-Connection establishment | 16.6.0 |
| 2020-12 | RAN#90 | R5-206297 | 1541 | 1 | F | Updates to CellGroupConfig in Table 4.6.3-19 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206298 | 1545 | 1 | F | Updates to RadioBearerConfig in Table 4.6.3-132 for NR-DC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206299 | 1547 | 1 | F | Updates to RRCReconfiguration in Table 4.6.1-13 for NR-DC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206300 | 1548 | 1 | F | Updates to RRCReconfigurationComplete | 16.6.0 |
| 2020-12 | RAN#90 | R5-206301 | 1586 | 1 | F | Update IE SSB-ToMeasure | 16.6.0 |
| 2020-12 | RAN#90 | R5-206302 | 1620 | 1 | F | Correction to PDCCH-ConfigCommon | 16.6.0 |
| 2020-12 | RAN#90 | R5-206303 | 1665 | 1 | F | Messages Exceptions corrections for SUL cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206304 | 1670 | 1 | F | Update to ims-EmergencySupport indication of SIB1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206305 | 1595 | 1 | F | Correction to test frequencies for signalling testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206306 | 1625 | 1 | F | Introducing test frequencies for CA_n261(2A) for protocol testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206307 | 1629 | 1 | F | Introduction of NR-DC test frequencies for protocol testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206308 | 1661 | 1 | F | Update requirements of test equipment for Signalling test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206309 | 1531 | 1 | F | Correction to Table 4.8.1-1A RRCReconfiguration-HO | 16.6.0 |
| 2020-12 | RAN#90 | R5-206388 | 1631 | 1 | F | Introduction of n14 test frequencies for protocol testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206394 | 1552 | 1 | F | Adding ReferenceTimeInfo IE config for IIoT test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206403 | 1559 | 1 | F | Update to RRC messages and IEs for R16 Mobility Enhancement | 16.6.0 |
| 2020-12 | RAN#90 | R5-206407 | 1553 | 1 | F | Correction to Uu RRC messages and SIBs for sidelink communication | 16.6.0 |
| 2020-12 | RAN#90 | R5-206408 | 1556 | 1 | F | Addition of V2X default configuration_USIM | 16.6.0 |
| 2020-12 | RAN#90 | R5-206409 | 1557 | 1 | F | Addition of V2X default configuration_NAS Messages | 16.6.0 |
| 2020-12 | RAN#90 | R5-206419 | 1575 | 1 | F | Updates to default contents of NAS messages for Rel-16 RACS | 16.6.0 |
| 2020-12 | RAN#90 | R5-206420 | 1576 | 1 | F | Updates to default contents of RRC messages for Rel-16 RACS | 16.6.0 |
| 2020-12 | RAN#90 | R5-206426 | 1649 | 1 | F | Alignment of Rel-16 5GMM messages | 16.6.0 |
| 2020-12 | RAN#90 | R5-206427 | 1668 | 1 | F | Updates to DLDedicatedMessageSegment message | 16.6.0 |
| 2020-12 | RAN#90 | R5-206621 | 1589 | 1 | F | Correction to test frequencies for NR band n34 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206622 | 1590 | 1 | F | Correction to test frequencies for NR band n38 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206623 | 1591 | 1 | F | Correction to test frequencies for NR band n39 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206624 | 1592 | 1 | F | Correction to test frequencies for NR band n40 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206625 | 1593 | 1 | F | Correction to test frequencies for NR band n50 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206626 | 1609 | 1 | F | Correction of test frequency of CA_n78C | 16.6.0 |
| 2020-12 | RAN#90 | R5-206628 | 1611 | 1 | F | Update to DEMOD message contents | 16.6.0 |
| 2020-12 | RAN#90 | R5-206629 | 1669 | 1 | F | Single PDN and PDU configuration for EN-DC RF testing | 16.6.0 |
| 2020-12 | RAN#90 | R5-206630 | 1619 | 1 | F | Addition of aperiodic CSI-RS reference configuration for RRM test | 16.6.0 |
| 2020-12 | RAN#90 | R5-206631 | 1623 | 1 | F | Introduction of test frequencies for SCS=60 kHz and EN-DC configurations DC_41X_n41A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206632 | 1505 | 1 | F | Message contents for iRAT periodical measurements | 16.6.0 |
| 2020-12 | RAN#90 | R5-206633 | 1638 | 1 | F | Minor corrections of 4.1 for test environment conditions | 16.6.0 |
| 2020-12 | RAN#90 | R5-206712 | 1579 | 1 | F | Addition of UL and DL inter-band CA configurations for several FR1 bands | 16.6.0 |
| 2020-12 | RAN#90 | R5-206713 | 1587 | 1 | F | Update to reference test conditions for R16 EN-DC configuration | 16.6.0 |
| 2020-12 | RAN#90 | R5-206714 | 1639 | 1 | F | Introduction of test frequencies for additional Rel-16 EN-DC inter-band configurations | 16.6.0 |
| 2020-12 | RAN#90 | R5-206715 | 1660 | 1 | F | Introduction of test frequencies for additional Rel-16 EN-DC inter-band configurations | 16.6.0 |
| 2020-12 | RAN#90 | R5-206736 | 1536 | 1 | F | Introduction of test frequencies for NR Band n53 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206737 | 1572 | 1 | F | Addition of R16 new channel bandwidths for n3 in 38.508-1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206738 | 1630 | 1 | F | Introduction of test frequencies for n14 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206739 | 1636 | 1 | F | Correction of 4.3.1 for test channel bandwidth of NR bands | 16.6.0 |
| 2020-12 | RAN#90 | R5-206757 | 1596 | 1 | F | Adding test frequencies for CA_n40B | 16.6.0 |
| 2020-12 | RAN#90 | R5-206758 | 1598 | 1 | F | Adding test frequencies for CA_n77A-n77A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206759 | 1599 | 1 | F | Adding test frequencies for CA_n78A-n78A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206760 | 1633 | 1 | F | Updating message contents for Uplink carrier switching | 16.6.0 |
| 2020-12 | RAN#90 | R5-206790 | 1581 | 1 | F | Introduction of test frequencies for RRM and NR band n257 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206791 | 1582 | 1 | F | Introduction of test frequencies for RRM and NR band n258 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206792 | 1583 | 1 | F | Introduction of test frequencies for RRM and NR band n260 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206793 | 1584 | 1 | F | Introduction of test frequencies for RRM and NR band n261 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206794 | 1585 | 1 | F | Introduction of test frequencies for RRM and NR band n259 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206820 | 1667 | 1 | F | Update to quality of quiet zone validation rule for IFF DFF hybrid setup | 16.6.0 |
| 2020-12 | RAN#90 | R5-206860 | 1506 | 1 | F | SSB bitmap correction for RRM test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206861 | 1624 | 1 | F | Introducing test frequencies for CA_n261(2A) | 16.6.0 |
| 2020-12 | RAN#90 | R5-206862 | 1643 | 1 | F | Clarify the RF / RRM conditions for default messages | 16.6.0 |
| 2021-03 | RAN#91 | R5-210186 | 1678 | - | F | Update global conditions | 16.7.0 |
| 2021-03 | RAN#91 | R5-210327 | 1688 | - | F | Update FailureInformation message | 16.7.0 |
| 2021-03 | RAN#91 | R5-210328 | 1689 | - | F | Editorial update RRCReconfiguration message | 16.7.0 |
| 2021-03 | RAN#91 | R5-210359 | 1692 | - | F | Editorial update SidelinkUEInformationNR message | 16.7.0 |
| 2021-03 | RAN#91 | R5-210394 | 1697 | - | F | Editorial update UEAssistanceInformation message | 16.7.0 |
| 2021-03 | RAN#91 | R5-210414 | 1699 | - | F | Update UECapabilityEnquiry message | 16.7.0 |
| 2021-03 | RAN#91 | R5-210468 | 1704 | - | F | Add new SIB combination for RRM tests with single cell | 16.7.0 |
| 2021-03 | RAN#91 | R5-210599 | 1711 | - | F | Editorial correction on numbering of several Tables in 38.508-1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210616 | 1713 | - | F | Editorial update DLDedicatedMessageSegment message | 16.7.0 |
| 2021-03 | RAN#91 | R5-210623 | 1714 | - | F | Correction to Table 4.6.1-13 RRCReconfiguration | 16.7.0 |
| 2021-03 | RAN#91 | R5-210626 | 1717 | - | F | Correction to Table 6.4.1-11 USIM Configuration 11 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210627 | 1718 | - | F | Correction to Table 4.8.2.1-7 Reference QoS rule 7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210687 | 1719 | - | F | Correction to Table 4.6.3-25B CondReconfigId | 16.7.0 |
| 2021-03 | RAN#91 | R5-210688 | 1720 | - | F | Correction to Table 4.6.3-25C CondReconfigToAddModList | 16.7.0 |
| 2021-03 | RAN#91 | R5-210689 | 1721 | - | F | Correction to Table 4.6.3-25D ConditionalReconfiguration | 16.7.0 |
| 2021-03 | RAN#91 | R5-210698 | 1723 | - | F | Addition of IE SL-PreconfigurationNR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210699 | 1724 | - | F | Addition of V2X NAS IEs | 16.7.0 |
| 2021-03 | RAN#91 | R5-210701 | 1726 | - | F | Correction of NR SL IE SL-BWP-ConfigCommon | 16.7.0 |
| 2021-03 | RAN#91 | R5-210703 | 1728 | - | F | Correction of NR SL IE SL-ConfigDedicatedNR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210704 | 1729 | - | F | Correction of NR SL IE SL-FreqConfigCommon | 16.7.0 |
| 2021-03 | RAN#91 | R5-210705 | 1730 | - | F | Correction of NR SL IE SL-LogicalChannelConfig | 16.7.0 |
| 2021-03 | RAN#91 | R5-210706 | 1731 | - | F | Correction of NR SL IE SL-MeasConfigInfo | 16.7.0 |
| 2021-03 | RAN#91 | R5-210707 | 1732 | - | F | Correction of NR SL IE SL-PDCP-Config | 16.7.0 |
| 2021-03 | RAN#91 | R5-210708 | 1733 | - | F | Correction of NR SL IE SL-RadioBearerConfig | 16.7.0 |
| 2021-03 | RAN#91 | R5-210709 | 1734 | - | F | Correction of NR SL IE SL-ResourcePool | 16.7.0 |
| 2021-03 | RAN#91 | R5-210710 | 1735 | - | F | Correction of NR SL IE SL-RLC-BearerConfig | 16.7.0 |
| 2021-03 | RAN#91 | R5-210711 | 1736 | - | F | Correction of NR SL IE SL-RLC-Config | 16.7.0 |
| 2021-03 | RAN#91 | R5-210713 | 1738 | - | F | Correction to NR Uu IE ARFCN-ValueNR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210714 | 1739 | - | F | Correction to NR Uu IE SCS-SpecificCarrier | 16.7.0 |
| 2021-03 | RAN#91 | R5-210771 | 1744 | - | F | Correction in CodebookConfig for 4Tx RI Demod test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210772 | 1745 | - | F | Alignment xOverhead setting with PDSCH RMCs for Demod FR2 testing | 16.7.0 |
| 2021-03 | RAN#91 | R5-210805 | 1746 | - | F | Update IE SemiStaticChannelAccessConfig | 16.7.0 |
| 2021-03 | RAN#91 | R5-210812 | 1747 | - | F | Update IE ServingCellConfig | 16.7.0 |
| 2021-03 | RAN#91 | R5-210824 | 1748 | - | F | Number of control symbols for RRM tests with 240kHz SSB SCS | 16.7.0 |
| 2021-03 | RAN#91 | R5-210826 | 1750 | - | F | Editorial update IE ServingCellConfigCommon | 16.7.0 |
| 2021-03 | RAN#91 | R5-210873 | 1752 | - | F | Correction of aperiodic CSI-RS reference configuration for RRM tests | 16.7.0 |
| 2021-03 | RAN#91 | R5-210897 | 1753 | - | F | Correction to test frequency parameters for band n83 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210898 | 1754 | - | F | Correction to test frequency parameters for band n84 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211032 | 1759 | - | F | Correction test frequencies for CA_n261(2A) | 16.7.0 |
| 2021-03 | RAN#91 | R5-211033 | 1760 | - | F | Correction test frequencies for CA_n261(2A) for protocol testing | 16.7.0 |
| 2021-03 | RAN#91 | R5-211034 | 1761 | - | F | Correction of protocol applicability for test frequencies for DC_xA_n261(2A) configurations | 16.7.0 |
| 2021-03 | RAN#91 | R5-211107 | 1770 | - | F | Corrections to subclauses in 38.508-1 with appropriate subclause level and heading styles | 16.7.0 |
| 2021-03 | RAN#91 | R5-211116 | 1772 | - | F | Update of 4.3.1.1.3.41.1 for test frequency of NR intra-band contiguous CA_n41C | 16.7.0 |
| 2021-03 | RAN#91 | R5-211117 | 1773 | - | F | Update of 4.3.1.1.3.66.1 for test frequency of NR intra-band contiguous CA_n66B | 16.7.0 |
| 2021-03 | RAN#91 | R5-211118 | 1774 | - | F | Update of 4.3.1.1.3.78.1 for test frequency of NR intra-band contiguous CA_n78C | 16.7.0 |
| 2021-03 | RAN#91 | R5-211121 | 1777 | - | F | Update of 4.3.1.3.2.1 for test frequencies for NR-DC configurations between FR1 and FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211124 | 1780 | - | F | Update of 4.3.1.6.1.3 for test frequencies for EN-DC band combinations including FR1 and FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211168 | 1784 | - | F | Updates to PDU SESSION ESTABLISHMENT ACCEPT message | 16.7.0 |
| 2021-03 | RAN#91 | R5-211170 | 1786 | - | F | Editorial update to BandCombinationListSidelink IE | 16.7.0 |
| 2021-03 | RAN#91 | R5-211171 | 1787 | - | F | Update to RRCReconfiguration-Speech IE | 16.7.0 |
| 2021-03 | RAN#91 | R5-211204 | 1791 | - | F | Editorial update IE BWP | 16.7.0 |
| 2021-03 | RAN#91 | R5-211328 | 1796 | - | F | Correction to frequency parameters for band n53 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211335 | 1727 | 1 | F | Correction of NR SL IE SL-BWP-PoolConfigCommon | 16.7.0 |
| 2021-03 | RAN#91 | R5-211336 | 1737 | 1 | F | Correction of NR SL IE SL-SDAP-Config | 16.7.0 |
| 2021-03 | RAN#91 | R5-211337 | 1740 | 1 | F | Correction to PC5-RRC message RRCReconfigurationSidelink | 16.7.0 |
| 2021-03 | RAN#91 | R5-211338 | 1741 | 1 | F | Correction to PC5-RRC message UECapabilityEnquirySidelink | 16.7.0 |
| 2021-03 | RAN#91 | R5-211339 | 1742 | 1 | F | Correction to PC5-RRC message UECapabilityInformationSidelink | 16.7.0 |
| 2021-03 | RAN#91 | R5-211369 | 1673 | 1 | F | Corrections to generic test procedures for IMS | 16.7.0 |
| 2021-03 | RAN#91 | R5-211370 | 1676 | 1 | F | Correction to generic procedure for UE-requested PDU session modification after S1 to N1 change | 16.7.0 |
| 2021-03 | RAN#91 | R5-211371 | 1680 | 1 | F | Correction to test procedure 4.9.7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211372 | 1698 | 1 | F | Correction to RRC IDLE procedures | 16.7.0 |
| 2021-03 | RAN#91 | R5-211374 | 1712 | 1 | F | Update IE PDCCH-ConfigCommon | 16.7.0 |
| 2021-03 | RAN#91 | R5-211375 | 1715 | 1 | F | Correction to Table 4.6.3-185 SSB-MTC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211456 | 1722 | 1 | F | Correction to Table 4.6.3-142 ReportConfigNR | 16.7.0 |
| 2021-03 | RAN#91 | R5-211462 | 1725 | 1 | F | Addition of SI combination for NR SL | 16.7.0 |
| 2021-03 | RAN#91 | R5-211465 | 1693 | 1 | F | Updates to SIB1 and SIB10 for Rel-16 NPN | 16.7.0 |
| 2021-03 | RAN#91 | R5-211466 | 1694 | 1 | F | Addition of System information combination for Rel-16 NPN | 16.7.0 |
| 2021-03 | RAN#91 | R5-211467 | 1684 | 1 | F | Introduction of definition of common environment for R16 NR Immediate MDT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211468 | 1685 | 1 | F | Updating Contents of RRC messages for Logged MDT test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211491 | 1700 | 1 | F | Addition of Cell configurations for 5G-SRVCC from NG-RAN to UTRAN | 16.7.0 |
| 2021-03 | RAN#91 | R5-211497 | 1682 | 1 | F | Editorial update IE PhysicalCellGroupConfig | 16.7.0 |
| 2021-03 | RAN#91 | R5-211498 | 1781 | 1 | F | Introduction of support for URLLC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211499 | 1782 | 1 | F | Addition of QoS for URLLC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211548 | 1695 | 1 | F | Addition of NID information for Rel-16 NPN | 16.7.0 |
| 2021-03 | RAN#91 | R5-211604 | 1794 | - | F | Introduction of test frequencies for CBW 70 MHz for n77 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211605 | 1795 | - | F | Introduction of test frequencies for CBW 70 MHz for n78 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211660 | 1677 | 1 | F | Update of EN-DC inter-band configurations in clause 4.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211661 | 1690 | 1 | F | Addition of 3 band EN-DC Test Frequency (DC_1A-8A_n78A, DC_3A-8A_n78A) | 16.7.0 |
| 2021-03 | RAN#91 | R5-211662 | 1691 | 1 | F | Addition of 4 band EN-DC Test Frequency (DC_1A-3A-8A_n78A) | 16.7.0 |
| 2021-03 | RAN#91 | R5-211663 | 1743 | 1 | F | Update PDSCH-TimeDomainResourceAllocationList to consider coreset0 for Demod FR2 test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211664 | 1763 | 1 | F | Update message content for PMI reporting test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211665 | 1705 | 1 | F | Changes to RRM default message contents | 16.7.0 |
| 2021-03 | RAN#91 | R5-211666 | 1706 | 1 | F | Add SSB Index table for RRM with SECOND_SSB condition | 16.7.0 |
| 2021-03 | RAN#91 | R5-211667 | 1751 | 1 | F | Addition of default configuration of CSI-IM for RRM tests | 16.7.0 |
| 2021-03 | RAN#91 | R5-211668 | 1790 | 1 | F | Specify CSI-SSB-ResourceSet for RRM | 16.7.0 |
| 2021-03 | RAN#91 | R5-211669 | 1702 | 1 | F | Editorial rework of the conditions for CSI-FrequencyOccupation | 16.7.0 |
| 2021-03 | RAN#91 | R5-211670 | 1703 | 1 | F | Align TDD UL DL Common for RRM with TS 38.533 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211671 | 1707 | 1 | F | Correct reportOffsetList in CSI-ReportConfig | 16.7.0 |
| 2021-03 | RAN#91 | R5-211672 | 1708 | 1 | F | Specify CSI-SSB-ResourceSet | 16.7.0 |
| 2021-03 | RAN#91 | R5-211673 | 1764 | 1 | F | Clarification on the connection diagram for FR2 demod and RRM test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211762 | 1778 | 1 | F | Update of 4.3.1.4.1 for test frequencies for EN-DC band combinations within FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211763 | 1779 | 1 | F | Update of 4.3.1.5.1 for test frequencies for EN-DC band combinations including FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211784 | 1776 | 1 | F | Update of 4.3.1.0A for mid test channel bandwidth | 16.7.0 |
| 2021-03 | RAN#91 | R5-211785 | 1792 | 1 | F | Correction of test frequencies for NR band n48 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211855 | 1758 | 1 | F | Updating the value of P-Max for EN-DC and NR SA test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211856 | 1768 | 1 | F | Correction to the message contents for CQI reporting tests in 5.4.2.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211857 | 1769 | 1 | F | Correction to the message contents for PMI reporting tests in 5.4.2.5 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210927 | 1755 | - | F | Updating Rel-17 mid and highest channel bandwidth for n83 and n84 | 17.0.0 |
| 2021-03 | RAN#91 | R5-210928 | 1756 | - | F | Adding Rel-17 CBW 30MHz test frequencies for n83 | 17.0.0 |
| 2021-03 | RAN#91 | R5-210929 | 1757 | - | F | Updating test frequencies for Rel-17 new CBWs for band n84 | 17.0.0 |
| 2021-03 | RAN#91 | R5-211836 | 1766 | 1 | F | Introduction of test frequencies for n48 adding CBW 70 MHz - DL only | 17.0.0 |
| 2021-06 | RAN#92 | R5-212203 | 1804 | - | F | Resubmission of Addition of SI combination for NR SL | 17.1.0 |
| 2021-06 | RAN#92 | R5-212213 | 1807 | - | F | Add IE Phy-ParametersSharedSpectrumChAccess | 17.1.0 |
| 2021-06 | RAN#92 | R5-212248 | 1811 | - | F | Correction to TCI stated of CSI-RS for TRS | 17.1.0 |
| 2021-06 | RAN#92 | R5-212249 | 1812 | - | F | Correction to physical layer parameters for demodulation tests | 17.1.0 |
| 2021-06 | RAN#92 | R5-212250 | 1813 | - | F | Update of TE diagram for FR2 RRM tests with multiple NR cells | 17.1.0 |
| 2021-06 | RAN#92 | R5-212409 | 1817 | - | F | Correction to IE BWP-DownlinkDedicated | 17.1.0 |
| 2021-06 | RAN#92 | R5-212455 | 1822 | - | F | Correction of NR SL IE SL-BWP-Config | 17.1.0 |
| 2021-06 | RAN#92 | R5-212456 | 1823 | - | F | Correction of NR SL IE SL-BWP-PoolConfig | 17.1.0 |
| 2021-06 | RAN#92 | R5-212457 | 1824 | - | F | Correction of NR SL IE SL-CBR-CommonTxConfigList | 17.1.0 |
| 2021-06 | RAN#92 | R5-212458 | 1825 | - | F | Correction of NR SL IE SL-CBR-PriorityTxConfigList | 17.1.0 |
| 2021-06 | RAN#92 | R5-212459 | 1826 | - | F | Correction of NR SL IE SL-ConfiguredGrantConfig | 17.1.0 |
| 2021-06 | RAN#92 | R5-212460 | 1827 | - | F | Correction of NR SL IE SL-DestinationIdentity | 17.1.0 |
| 2021-06 | RAN#92 | R5-212461 | 1828 | - | F | Correction of NR SL IE SL-FreqConfig | 17.1.0 |
| 2021-06 | RAN#92 | R5-212462 | 1829 | - | F | Correction of NR SL IE SL-MeasConfigCommon | 17.1.0 |
| 2021-06 | RAN#92 | R5-212463 | 1830 | - | F | Correction of NR SL IE SL-MeasIdList | 17.1.0 |
| 2021-06 | RAN#92 | R5-212464 | 1831 | - | F | Correction of NR SL IE SL-MeasObjectList | 17.1.0 |
| 2021-06 | RAN#92 | R5-212465 | 1832 | - | F | Correction of NR SL IE SL-PSBCH-Config | 17.1.0 |
| 2021-06 | RAN#92 | R5-212468 | 1835 | - | F | Correction of NR SL IE SL-QoS-Profile | 17.1.0 |
| 2021-06 | RAN#92 | R5-212469 | 1836 | - | F | Correction of NR SL IE SL-QuantityConfig | 17.1.0 |
| 2021-06 | RAN#92 | R5-212473 | 1840 | - | F | Correction of NR SL IE SL-ScheduledConfig | 17.1.0 |
| 2021-06 | RAN#92 | R5-212476 | 1843 | - | F | Correction of NR SL IE SL-TxPower | 17.1.0 |
| 2021-06 | RAN#92 | R5-212478 | 1845 | - | F | Correction of NR SL IE SL-UE-SelectedConfig | 17.1.0 |
| 2021-06 | RAN#92 | R5-212479 | 1846 | - | F | Correction of NR SL IE SL-ZoneConfig | 17.1.0 |
| 2021-06 | RAN#92 | R5-212616 | 1859 | - | F | Correction to PUCCH resource indicator value for PMI reporting requirements | 17.1.0 |
| 2021-06 | RAN#92 | R5-212642 | 1861 | - | F | Editorial correction of header level in clause 5.4.2.0 in 38.508-1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-212688 | 1865 | - | F | Correction of nominal channel spacing in test frequencies for CA_n257x | 17.1.0 |
| 2021-06 | RAN#92 | R5-212689 | 1866 | - | F | Correction of nominal channel spacing in test frequencies for CA_n258x | 17.1.0 |
| 2021-06 | RAN#92 | R5-212690 | 1867 | - | F | Correction of nominal channel spacing in test frequencies for CA_n260x | 17.1.0 |
| 2021-06 | RAN#92 | R5-212691 | 1868 | - | F | Correction of nominal channel spacing in test frequencies for CA_n261x | 17.1.0 |
| 2021-06 | RAN#92 | R5-212692 | 1869 | - | F | Correction of test frequencies for CA_n41C | 17.1.0 |
| 2021-06 | RAN#92 | R5-212693 | 1870 | - | F | Correction of test frequencies for CA_n78C | 17.1.0 |
| 2021-06 | RAN#92 | R5-212694 | 1871 | - | F | Correction of test frequencies for DC_(n)41AA | 17.1.0 |
| 2021-06 | RAN#92 | R5-212695 | 1872 | - | F | Editorial correction of test frequencies for protocol testing | 17.1.0 |
| 2021-06 | RAN#92 | R5-212698 | 1875 | - | F | Introduction of test frequencies for n41 adding CBW 70 MHz | 17.1.0 |
| 2021-06 | RAN#92 | R5-212699 | 1876 | - | F | Introduction of test frequencies for n48 adding CBW 30 MHz | 17.1.0 |
| 2021-06 | RAN#92 | R5-212702 | 1879 | - | F | Introduction of principles for calculating test frequencies for NR Intra-band Contiguous CA for asymmetric bands in Annex C | 17.1.0 |
| 2021-06 | RAN#92 | R5-212703 | 1880 | - | F | Correction of test frequencies for CA_n66B | 17.1.0 |
| 2021-06 | RAN#92 | R5-212705 | 1881 | - | F | Add message contents for RRM FR2 tests with reduced RB allocation | 17.1.0 |
| 2021-06 | RAN#92 | R5-212724 | 1885 | - | F | Correct number of HARQ processes for PDSCH | 17.1.0 |
| 2021-06 | RAN#92 | R5-212819 | 1892 | - | F | Correction of 4.3.1.0D for bandwidth part | 17.1.0 |
| 2021-06 | RAN#92 | R5-212820 | 1893 | - | F | Correction of 4.3.1.1.2 for test frequencies for NR FR1 inter-band CA configurations | 17.1.0 |
| 2021-06 | RAN#92 | R5-212886 | 1897 | - | F | Update of default SCS for n48 for protocol testing | 17.1.0 |
| 2021-06 | RAN#92 | R5-212890 | 1898 | - | F | Test frequencies definition for EN-DC band 41CA | 17.1.0 |
| 2021-06 | RAN#92 | R5-212892 | 1900 | - | F | Correction to NZP CSI-RS default configuration for RRM test | 17.1.0 |
| 2021-06 | RAN#92 | R5-212921 | 1901 | - | F | Addition of GNSS requirements for NR sidelink | 17.1.0 |
| 2021-06 | RAN#92 | R5-212924 | 1904 | - | F | Addition of connection diagram of NR sidelink testing | 17.1.0 |
| 2021-06 | RAN#92 | R5-212946 | 1905 | - | F | Updating IEs for URLLC | 17.1.0 |
| 2021-06 | RAN#92 | R5-212986 | 1906 | - | F | Introducing Rel-16 CA configuration CA_n28A-n41A to clause 4.3.1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213003 | 1907 | - | F | Adding test frequency description for SUL configuration | 17.1.0 |
| 2021-06 | RAN#92 | R5-213004 | 1908 | - | F | Adding connection diagrams for SUL configuration with DL CA | 17.1.0 |
| 2021-06 | RAN#92 | R5-213054 | 1910 | - | F | Correction to PDU Session Authentication Command, PDU Session Authentication Complete and PDU Session Authentication Result messages. | 17.1.0 |
| 2021-06 | RAN#92 | R5-213057 | 1912 | - | F | Introduction of test frequencies for CA_n48(2A) | 17.1.0 |
| 2021-06 | RAN#92 | R5-213150 | 1917 | - | F | Updates to global conditions | 17.1.0 |
| 2021-06 | RAN#92 | R5-213153 | 1920 | - | F | Updates to PDU SESSION ESTABLISHMENT REJECT message | 17.1.0 |
| 2021-06 | RAN#92 | R5-213154 | 1921 | - | F | Updates to PDU SESSION MODIFICATION REJECT message | 17.1.0 |
| 2021-06 | RAN#92 | R5-213155 | 1922 | - | F | Updates to QoS flows | 17.1.0 |
| 2021-06 | RAN#92 | R5-213180 | 1939 | - | F | Update chapter 4.5.2 RRC_IDLE | 17.1.0 |
| 2021-06 | RAN#92 | R5-213192 | 1940 | - | F | Update chapter 4.5.4 RRC_CONNECTED | 17.1.0 |
| 2021-06 | RAN#92 | R5-213304 | 1953 | - | F | Correction of test frequencies for NR band n66 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213307 | 1954 | - | F | Correction of common default messages for demod FR2 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213339 | 1956 | - | F | Update PUCCH-ConfigCommon for Demod testing | 17.1.0 |
| 2021-06 | RAN#92 | R5-213340 | 1957 | - | F | Update message content for subband CQI reporting test cases | 17.1.0 |
| 2021-06 | RAN#92 | R5-213400 | 1961 | - | F | Inclusion of additional P-CSCF IP address in PDU session establishment | 17.1.0 |
| 2021-06 | RAN#92 | R5-213409 | 1955 | 1 | F | Editorial updates to test procedure titles | 17.1.0 |
| 2021-06 | RAN#92 | R5-213416 | 1838 | 1 | F | Correction of NR SL IE SL-ReportConfigList | 17.1.0 |
| 2021-06 | RAN#92 | R5-213417 | 1841 | 1 | F | Correction of NR SL IE SL-SyncConfig | 17.1.0 |
| 2021-06 | RAN#92 | R5-213418 | 1848 | 1 | F | Correction to PC5-RRC message MeasurementReportSidelink | 17.1.0 |
| 2021-06 | RAN#92 | R5-213419 | 1850 | 1 | F | Correction to PC5-RRC message RRCReconfigurationFailureSidelink | 17.1.0 |
| 2021-06 | RAN#92 | R5-213423 | 1857 | 1 | F | Updates to NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND and NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE messages | 17.1.0 |
| 2021-06 | RAN#92 | R5-213424 | 1858 | 1 | F | Updates to NETWORK SLICE-SPECIFIC AUTHENTICATION RESULT message | 17.1.0 |
| 2021-06 | RAN#92 | R5-213425 | 1924 | 1 | F | Introduction of Always-On indication for URLLC | 17.1.0 |
| 2021-06 | RAN#92 | R5-213439 | 1816 | 1 | F | Correction to IMS call release sequences | 17.1.0 |
| 2021-06 | RAN#92 | R5-213440 | 1886 | 1 | F | Addition of Generic Test procedure for IMS MO Video call establishment in 5GC | 17.1.0 |
| 2021-06 | RAN#92 | R5-213441 | 1913 | 1 | F | Correction to Table 4.9.12.2.2-1 for IMS Emergency call establishment in 5GC without IMS emergency registration | 17.1.0 |
| 2021-06 | RAN#92 | R5-213442 | 1914 | 1 | F | Correction to Procedure for UE-requested PDU session modification after the first S1 to N1 mode change / Single-registration mode with N26 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213443 | 1941 | 1 | F | Update chapter 4.5.1 General | 17.1.0 |
| 2021-06 | RAN#92 | R5-213444 | 1887 | 1 | F | Addition of default contents for RRCReconfiguration-Video | 17.1.0 |
| 2021-06 | RAN#92 | R5-213445 | 1888 | 1 | F | Corrections to PDCP config | 17.1.0 |
| 2021-06 | RAN#92 | R5-213446 | 1798 | 1 | F | Update of USIM Configurations 4, 7, 8, 10, 12 and 21 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213447 | 1854 | 1 | F | Correction to Table 6.4.1-12 USIM Configuration 12 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213448 | 1873 | 1 | F | Update of default SCS for n38, n39, n40 and n50 for protocol testing | 17.1.0 |
| 2021-06 | RAN#92 | R5-213449 | 1942 | 1 | F | Update maximum number of simultaneous configured cells for FR1 and FR2 in OTA | 17.1.0 |
| 2021-06 | RAN#92 | R5-213450 | 1951 | 1 | F | Updates to FR1 and E-UTRA OTA signal level testing | 17.1.0 |
| 2021-06 | RAN#92 | R5-213451 | 1802 | 1 | F | Correction and editorials to default message content specification | 17.1.0 |
| 2021-06 | RAN#92 | R5-213452 | 1915 | 1 | F | Correction to Combinations of system information blocks | 17.1.0 |
| 2021-06 | RAN#92 | R5-213557 | 1819 | 1 | F | Addition of general procedures for NR sidelink | 17.1.0 |
| 2021-06 | RAN#92 | R5-213558 | 1820 | 1 | F | Addition of test procedure to establish sidelink unicast mode | 17.1.0 |
| 2021-06 | RAN#92 | R5-213559 | 1821 | 1 | F | Addition of test state for NR sidelink | 17.1.0 |
| 2021-06 | RAN#92 | R5-213560 | 1833 | 1 | F | Correction of NR SL IE SL-PSSCH-TxConfigList | 17.1.0 |
| 2021-06 | RAN#92 | R5-213561 | 1834 | 1 | F | Correction of NR SL IE SL-QoS-FlowIdentity | 17.1.0 |
| 2021-06 | RAN#92 | R5-213562 | 1837 | 1 | F | Correction of NR SL IE SLRB-Uu-ConfigIndex | 17.1.0 |
| 2021-06 | RAN#92 | R5-213563 | 1839 | 1 | F | Correction of NR SL IE SL-RLC-BearerConfigIndex | 17.1.0 |
| 2021-06 | RAN#92 | R5-213564 | 1844 | 1 | F | Correction of NR SL IE SL-TypeTxSync | 17.1.0 |
| 2021-06 | RAN#92 | R5-213565 | 1847 | 1 | F | Correction to PC5-RRC message MasterInformationBlockSidelink | 17.1.0 |
| 2021-06 | RAN#92 | R5-213566 | 1849 | 1 | F | Correction to PC5-RRC message RRCReconfigurationCompleteSidelink | 17.1.0 |
| 2021-06 | RAN#92 | R5-213567 | 1856 | 1 | F | Update IE SL-Thres-RSRP-List | 17.1.0 |
| 2021-06 | RAN#92 | R5-213573 | 1890 | 1 | F | Updates to NPN-Identity for Rel-16 NPN | 17.1.0 |
| 2021-06 | RAN#92 | R5-213574 | 1891 | 1 | F | Addition of System information combination for Rel-16 NPN | 17.1.0 |
| 2021-06 | RAN#92 | R5-213580 | 1889 | 1 | F | Updates to default contents of UECapabilityEnquiry message | 17.1.0 |
| 2021-06 | RAN#92 | R5-213601 | 1855 | 1 | F | Introduction of definition of common environment for R16 NR SON and MDT | 17.1.0 |
| 2021-06 | RAN#92 | R5-213643 | 1918 | 1 | F | Addition of DNN configurations for new SSTs | 17.1.0 |
| 2021-06 | RAN#92 | R5-213644 | 1919 | 1 | F | Updates to UE Policy Delivery messages | 17.1.0 |
| 2021-06 | RAN#92 | R5-213645 | 1923 | 1 | F | Updates to REGISTRATION ACCEPT message | 17.1.0 |
| 2021-06 | RAN#92 | R5-213675 | 1960 | 1 | F | Correction to procedure 4.9.9 Tracking area updating / Inter-system change from S1 mode to N1 mode in 5GMM/EMM-IDLE mode | 17.1.0 |
| 2021-06 | RAN#92 | R5-213833 | 1926 | 1 | F | Testing frequencies update for band n3 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213834 | 1950 | 1 | F | CR to 38.508-1 on larger quiet zone with grey-box approach | 17.1.0 |
| 2021-06 | RAN#92 | R5-213853 | 1864 | 1 | F | Correction of test frequencies for CA_n260(A-I) | 17.1.0 |
| 2021-06 | RAN#92 | R5-213854 | 1862 | 1 | F | Introduction of principles for calculating test frequencies for NR Intra-band Non-Contiguous CA in Annex C | 17.1.0 |
| 2021-06 | RAN#92 | R5-213858 | 1896 | 1 | F | Update Note about n28 Test frequency Mid range and CBW 30 MHz | 17.1.0 |
| 2021-06 | RAN#92 | R5-213859 | 1925 | 1 | F | Mid range test frequencies update in case asymmetric bandwidths | 17.1.0 |
| 2021-06 | RAN#92 | R5-213860 | 1927 | 1 | F | Testing frequencies update for band n34 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213861 | 1928 | 1 | F | Testing frequencies update for band n39 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213862 | 1929 | 1 | F | Testing frequencies update for band n53 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213863 | 1930 | 1 | F | Testing frequencies update for band n66 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213864 | 1931 | 1 | F | Testing frequencies update for band n70 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213865 | 1932 | 1 | F | Testing frequencies update for band n80 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213866 | 1933 | 1 | F | Testing frequencies update for band n81 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213867 | 1934 | 1 | F | Testing frequencies update for band n82 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213868 | 1935 | 1 | F | Testing frequencies update for band n86 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213869 | 1937 | 1 | F | OffsetToCarrier alignment for cases with equal low, mid and high frequency range (n30, n39, n51, n70) and editorial corrections in annex C.3. | 17.1.0 |
| 2021-06 | RAN#92 | R5-213870 | 1884 | 1 | F | Align Chapter 7 of TS 38.508-1 with Annex H of TS 38.533 | 17.1.0 |
| 2021-06 | RAN#92 | R5-213871 | 1944 | 1 | F | Introduction of test frequencies for n257 for RRM Inter-freq adjacent cell | 17.1.0 |
| 2021-06 | RAN#92 | R5-213872 | 1945 | 1 | F | Introduction of test frequencies for n258 for RRM Inter-freq adjacent cell | 17.1.0 |
| 2021-06 | RAN#92 | R5-213873 | 1947 | 1 | F | Introduction of test frequencies for n260 for RRM Inter-freq adjacent cell | 17.1.0 |
| 2021-06 | RAN#92 | R5-213874 | 1948 | 1 | F | Introduction of test frequencies for n261 for RRM Inter-freq adjacent cell | 17.1.0 |
| 2021-06 | RAN#92 | R5-213875 | 1863 | 1 | F | Introduction of principles for calculating test frequencies for EN-DC configurations in Annex C | 17.1.0 |
| 2021-06 | RAN#92 | R5-213876 | 1882 | 1 | F | Add locationAndBandwidth for RRM FR2 tests with reduced RB allocation | 17.1.0 |
| 2021-06 | RAN#92 | R5-213877 | 1936 | 1 | F | Annex C: Clarifications to maximum and minimum offsetRBs | 17.1.0 |
| 2021-06 | RAN#92 | R5-213878 | 1938 | 1 | F | Annex C update to add SUL test frequencies calculation | 17.1.0 |
| 2021-06 | RAN#92 | R5-213879 | 1949 | 1 | F | Determination of test frequencies for a Mid range adjacent inter-frequency cell for FR2 RRM multicell testing in Annex C | 17.1.0 |
| 2021-06 | RAN#92 | R5-213964 | 1877 | 1 | F | Correction of test frequencies for CA_n66(2A) | 17.1.0 |
| 2021-06 | RAN#92 | R5-213977 | 1946 | 1 | F | Introduction of test frequencies for n259 for RRM Inter-freq adjacent cell | 17.1.0 |
| 2021-06 | RAN#92 | R5-213996 | 1902 | 1 | F | Addition of calculation method of NR sidelink test frequencies | 17.1.0 |
| 2021-06 | RAN#92 | R5-213997 | 1903 | 1 | F | Addition of V2X test frequencies of band n47 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214017 | 1799 | 1 | F | Updating test frequencies for Rel-17 EN-DC band combinations within FR1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214018 | 1801 | 1 | F | Add test frequencies for R17 NR inter-band CA configurations in FR1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214019 | 1909 | 1 | F | Introducing Rel-17 CA configuration CA_n28A-n79A to clause 4.3.1 | 17.1.0 |
| 2021-06 | RAN#92 | R5-214026 | 1809 | 1 | F | RRC signalling for UL power boosting via suspended IBE requirements | 17.1.0 |
| 2021-06 | RAN#92 | R5-214047 | 1959 | 1 | F | Clarification on PDU configuration for RF, Demod and RRM tests | 17.1.0 |
| 2021-06 | RAN#92 | R5-214076 | 1860 | 1 | F | Update IE PDCCH-ConfigCommon for additional BWP | 17.1.0 |
| 2021-06 | RAN#92 | R5-214107 | 1883 | 1 | F | Align RRM CSI-ResourcePeriodicityAndOffset to TS 38.133 | 17.1.0 |
| 2021-09 | RAN#93 | R5-214381 | 1963 | - | F | Correct dl_DataToUL_ACK for short DCI test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-214436 | 1967 | - | F | Correction to 38.508 Table 4.8.2.3-2: Reference QoS flow #2 | 17.2.0 |
| 2021-09 | RAN#93 | R5-214554 | 1968 | - | F | Correction of default test frequencies for bands n38, n39, n40 and n50 and protocol testing | 17.2.0 |
| 2021-09 | RAN#93 | R5-214611 | 1970 | - | F | Correction of default test frequencies for band n48 and protocol testing | 17.2.0 |
| 2021-09 | RAN#93 | R5-214622 | 1971 | - | F | Editorial updates to test procedure titles | 17.2.0 |
| 2021-09 | RAN#93 | R5-214678 | 1972 | - | F | Correction to k1 setting for FR2 RRM | 17.2.0 |
| 2021-09 | RAN#93 | R5-214727 | 1974 | - | F | Introduction of test frequencies for CA_n48B | 17.2.0 |
| 2021-09 | RAN#93 | R5-214728 | 1975 | - | F | Corrections to UEInformationRequest and UEInformationResponse | 17.2.0 |
| 2021-09 | RAN#93 | R5-214755 | 1976 | - | F | Updates to System information combination for NR-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-214800 | 1979 | - | F | Correction to IEs for UE policy part | 17.2.0 |
| 2021-09 | RAN#93 | R5-214808 | 1987 | - | F | Correction to NR V2X USIM configuration | 17.2.0 |
| 2021-09 | RAN#93 | R5-214853 | 1989 | - | F | Introduction of test frequencies for CA_n48B and protocol testing | 17.2.0 |
| 2021-09 | RAN#93 | R5-214900 | 1992 | - | F | Editorial Updates to Clause. 4.4.3.1.2 for System information combination | 17.2.0 |
| 2021-09 | RAN#93 | R5-214929 | 1994 | - | F | Introduction of test frequencies for CA_n71(2A) for protocol testing | 17.2.0 |
| 2021-09 | RAN#93 | R5-214947 | 1997 | - | F | Correction to Table 6.4.1-8 USIM Configuration 8 | 17.2.0 |
| 2021-09 | RAN#93 | R5-214959 | 1998 | - | F | Correction of test frequencies for CA_n66(2A) for protocol testing | 17.2.0 |
| 2021-09 | RAN#93 | R5-214962 | 1999 | - | F | Alignment of test frequency tables for CA_n48(2A), CA_n66(2A), CA_n77(2A) and CA_n78(2A) | 17.2.0 |
| 2021-09 | RAN#93 | R5-214977 | 2003 | - | F | Correction to default configuration-ControlResourceSet | 17.2.0 |
| 2021-09 | RAN#93 | R5-214978 | 2004 | - | F | Correction to default configuration-SCell CSI on PCell | 17.2.0 |
| 2021-09 | RAN#93 | R5-215071 | 2006 | - | F | Update to Out of Coverage procedure to trigger SL-MIMO transmission | 17.2.0 |
| 2021-09 | RAN#93 | R5-215308 | 2012 | - | F | Adding test frequencies for SUL band n97 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215341 | 2015 | - | F | Correction to TRS configuration for RF test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-215456 | 2019 | - | F | Update of 4.3.1.4.1 for test frequencies for EN-DC configurations within FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215462 | 2021 | - | F | Correction of 4.3.1.0D for locationAndBandwidth in BWP | 17.2.0 |
| 2021-09 | RAN#93 | R5-215499 | 2023 | - | F | Introduction of V2X SST | 17.2.0 |
| 2021-09 | RAN#93 | R5-215504 | 2028 | - | F | Updates to Table 4.4A.5-2 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215518 | 2029 | - | F | Editorial correction: channel bandwidth and RB allocation revision in Test frequencies for CA_n260(A-I) | 17.2.0 |
| 2021-09 | RAN#93 | R5-215530 | 2030 | - | F | Correction on Test frequencies for DC_(n)41CA | 17.2.0 |
| 2021-09 | RAN#93 | R5-215541 | 2031 | - | F | Test frequencies update for CA_ n257G, CA_ n257H and CA_ n257I | 17.2.0 |
| 2021-09 | RAN#93 | R5-215612 | 2032 | - | F | Correction RF E-UTRA CONNECTED state | 17.2.0 |
| 2021-09 | RAN#93 | R5-215678 | 2034 | - | F | RRC and NAS message handling in uplink in case of simultaneous RRC and NAS procedures | 17.2.0 |
| 2021-09 | RAN#93 | R5-215679 | 2035 | - | F | Enquiry of Capability and checking of UeCapabilityInformation contents for NR-DC | 17.2.0 |
| 2021-09 | RAN#93 | R5-215689 | 2039 | - | F | Correction to USIM Configuration 18 and 19 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215691 | 2040 | - | F | Update chapter 4.5.4 RRC_CONNECTED | 17.2.0 |
| 2021-09 | RAN#93 | R5-215835 | 1964 | 1 | F | Correct CSI-MeasConfig for test cases with 1SSB | 17.2.0 |
| 2021-09 | RAN#93 | R5-215836 | 1965 | 1 | F | Complete CSI-ReportConfig for RRM | 17.2.0 |
| 2021-09 | RAN#93 | R5-215837 | 2014 | 1 | F | Correction to CSI report configurations | 17.2.0 |
| 2021-09 | RAN#93 | R5-215936 | 2005 | 1 | F | Update of SIB2 to add messages for relaxed RRM measurement | 17.2.0 |
| 2021-09 | RAN#93 | R5-215969 | 1962 | 1 | F | Updating Test Frequencies for Rel-17 CA,DC band combinations within FR1 into TS 38.508-1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215970 | 1993 | 1 | F | Introduction of test frequencies for CA_n71(2A) | 17.2.0 |
| 2021-09 | RAN#93 | R5-215971 | 2008 | 1 | F | Addition of R17 CADC configuration into 38.508-1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-215972 | 2020 | 1 | F | Update of 4.3.1.1.2 for NR inter-band CA configurations in FR1 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216027 | 2016 | 1 | F | Addition of Perf RI FR2 message contents | 17.2.0 |
| 2021-09 | RAN#93 | R5-216070 | 2007 | 1 | F | Adding connection diagram for eMIMO multi-TRP demod test cases | 17.2.0 |
| 2021-09 | RAN#93 | R5-216079 | 1973 | 1 | F | Updating test frequencies for Rel-17 inter-band EN-DC configurations | 17.2.0 |
| 2021-09 | RAN#93 | R5-216116 | 2009 | 1 | F | Updates to Test Equipment connection for Demodulation Performance and CSI reporting tests | 17.2.0 |
| 2021-09 | RAN#93 | R5-216122 | 2010 | 1 | B | Introduction of test frequencies for n24 and n99 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216150 | 1977 | 1 | F | Correction to Test Procedure for IMS MO and MT call release in 5GC | 17.2.0 |
| 2021-09 | RAN#93 | R5-216151 | 2017 | 1 | F | Correction to introduce Handling of PDU Session Release during switch off/Power off procedures | 17.2.0 |
| 2021-09 | RAN#93 | R5-216152 | 2026 | 1 | F | Addition of UE Configuration Update procedure | 17.2.0 |
| 2021-09 | RAN#93 | R5-216153 | 2037 | 1 | F | Corrections for IMS video call signalling | 17.2.0 |
| 2021-09 | RAN#93 | R5-216154 | 2041 | 1 | F | Correction to Table 4.6.3-142 and Table 4.6.3-79 for SFTD measurement reporting | 17.2.0 |
| 2021-09 | RAN#93 | R5-216155 | 2024 | 1 | F | Introduction of PS Data Off | 17.2.0 |
| 2021-09 | RAN#93 | R5-216156 | 2025 | 1 | F | Introduction of URSP | 17.2.0 |
| 2021-09 | RAN#93 | R5-216157 | 2027 | 1 | F | Updates to REGISTRATION messages | 17.2.0 |
| 2021-09 | RAN#93 | R5-216158 | 2036 | 1 | F | Correction to Table 4.8.2.2-1 for default Packet filter ID | 17.2.0 |
| 2021-09 | RAN#93 | R5-216159 | 1969 | 1 | F | Correction for USIM configurations | 17.2.0 |
| 2021-09 | RAN#93 | R5-216160 | 2018 | 1 | F | Updates to NR cell configurations for SIG | 17.2.0 |
| 2021-09 | RAN#93 | R5-216161 | 2038 | 1 | F | Correction to reference configurations for IMS video call signalling | 17.2.0 |
| 2021-09 | RAN#93 | R5-216243 | 1991 | 1 | F | Correction of test frequencies for CA_n66B for protocol testing | 17.2.0 |
| 2021-09 | RAN#93 | R5-216256 | 2022 | 1 | F | Introduction of MIoT SST | 17.2.0 |
| 2021-09 | RAN#93 | R5-216263 | 1980 | 1 | F | Correction to IEs for V2XP info | 17.2.0 |
| 2021-09 | RAN#93 | R5-216264 | 1981 | 1 | F | Correction to IEs for Served by E-UTRA or served by NR | 17.2.0 |
| 2021-09 | RAN#93 | R5-216265 | 1982 | 1 | F | Correction to IEs for Not served by E-UTRA and not served by NR | 17.2.0 |
| 2021-09 | RAN#93 | R5-216266 | 1983 | 1 | F | Correction to IEs for V2X service identifier to PC5 RAT and Tx profiles mapping rules | 17.2.0 |
| 2021-09 | RAN#93 | R5-216267 | 1984 | 1 | F | Correction to IEs for Privacy config | 17.2.0 |
| 2021-09 | RAN#93 | R5-216268 | 1985 | 1 | F | Correction to IEs for V2X communication over PC5 in E-UTRA-PC5 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216269 | 1986 | 1 | F | Correction to IEs for V2X communication over PC5 in NR-PC5 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216270 | 1988 | 1 | F | Correction to UE Policy Delivery msg | 17.2.0 |
| 2021-09 | RAN#93 | R5-216284 | 1990 | 1 | F | Update default message contents of LoggedMeasurementConfiguration | 17.2.0 |
| 2021-09 | RAN#93 | R5-216320 | 2011 | 1 | B | Introduction of signalling test frequencies for n24 and n99 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216321 | 2013 | 1 | F | Adding signalling test frequencies for SUL band n97 | 17.2.0 |
| 2021-09 | RAN#93 | R5-216327 | 2000 | 1 | F | Default message content update for NR EIEI | 17.2.0 |
| 2021-09 | RAN#93 | R5-216328 | 2001 | 1 | F | Generic procedure for eCall over IMS establishment in 5GS Normal Service | 17.2.0 |
| 2021-09 | RAN#93 | R5-216329 | 2002 | 1 | F | USIM configuration for NR EIEI | 17.2.0 |
| 2021-12 | RAN#94 | R5-216510 | 2047 | - | F | Updating Test Frequencies for Rel-16 CA,DC band combinations within FR1 into TS 38.508-1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-216530 | 2048 | - | F | Addition of test frequencies for asymmetric channel bandwidths for n24 | 17.3.0 |
| 2021-12 | RAN#94 | R5-216603 | 2051 | - | F | Addition of NR band n95 | 17.3.0 |
| 2021-12 | RAN#94 | R5-216636 | 2052 | - | F | Updates on simultaneous co-existence of NR cells | 17.3.0 |
| 2021-12 | RAN#94 | R5-216761 | 2053 | - | F | Addition of PDCCH Search Space Ext configuration in 38.508-1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-216767 | 2054 | - | F | Updates to NETWORK SLICE-SPECIFIC AUTHENTICATION COMMAND message | 17.3.0 |
| 2021-12 | RAN#94 | R5-216768 | 2055 | - | F | Updates to NETWORK SLICE-SPECIFIC AUTHENTICATION COMPLETE message | 17.3.0 |
| 2021-12 | RAN#94 | R5-216773 | 2059 | - | F | Update IE SSB-PositionQCL-Relation | 17.3.0 |
| 2021-12 | RAN#94 | R5-216829 | 2061 | - | F | Correction to IMS MO emergency call release procedure | 17.3.0 |
| 2021-12 | RAN#94 | R5-216851 | 2062 | - | F | Correction to IE Table 7.3.1-3 - SSB-MTC | 17.3.0 |
| 2021-12 | RAN#94 | R5-216895 | 2063 | - | F | Correction to IE Table 4.6.3-77A - MeasObjectUTRA-FDD | 17.3.0 |
| 2021-12 | RAN#94 | R5-216896 | 2064 | - | F | Correction to IE Table 4.6.3-79 - MeasResults | 17.3.0 |
| 2021-12 | RAN#94 | R5-216898 | 2066 | - | F | Correction to IE Table 4.6.5-16 - UTRA-FDD-Q-OffsetRange | 17.3.0 |
| 2021-12 | RAN#94 | R5-216899 | 2067 | - | F | Correction to RRC message Table 4.6.1-8 - MobilityFromNRCommand | 17.3.0 |
| 2021-12 | RAN#94 | R5-216905 | 2068 | - | F | Correction to IE Table 4.6.3-62A - HighSpeedConfig | 17.3.0 |
| 2021-12 | RAN#94 | R5-216922 | 2097 | - | F | Update of Table 4.7.1-7-Registration Accept | 17.3.0 |
| 2021-12 | RAN#94 | R5-217024 | 2101 | - | F | Add Reference file | 17.3.0 |
| 2021-12 | RAN#94 | R5-217025 | 2102 | - | F | Correction to DIRECT LINK ESTABLISHMENT REQUEST msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217026 | 2103 | - | F | Correction to DIRECT LINK ESTABLISHMENT ACCEPT msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217027 | 2104 | - | F | Correction to DIRECT LINK MODIFICATION REQUEST msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217028 | 2105 | - | F | Correction to DIRECT LINK MODIFICATION ACCEPT msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217029 | 2106 | - | F | Correction to DIRECT LINK RELEASE REQUEST msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217030 | 2107 | - | F | Correction to DIRECT LINK RELEASE ACCEPT msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217032 | 2109 | - | F | Correction to DIRECT LINK KEEPALIVE RESPONSE msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217035 | 2112 | - | F | Correction to DIRECT LINK AUTHENTICATION REJECT msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217038 | 2115 | - | F | Correction to DIRECT LINK SECURITY MODE REJECT msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217039 | 2116 | - | F | Correction to DIRECT LINK REKEYING REQUEST msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217040 | 2117 | - | F | Correction to DIRECT LINK REKEYING RESPONSE msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217044 | 2121 | - | F | Correction to DIRECT LINK IDENTIFIER UPDATE REJECT msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217045 | 2122 | - | F | Correction to DIRECT LINK MODIFICATION REJECT msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217095 | 2125 | - | F | Update chapter 4.8.1 for NE-DC | 17.3.0 |
| 2021-12 | RAN#94 | R5-217118 | 2128 | - | F | Meas exception for TRS config in RF test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217243 | 2133 | - | F | Correction to IE Table 4.6.3-127 QuantityConfig | 17.3.0 |
| 2021-12 | RAN#94 | R5-217246 | 2134 | - | F | Correction of spec number and addition of uncertainty value for OTA testing in section 5.2.1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217259 | 2135 | - | F | Addition of 13 NR CA combinations to FR1 inter-band configurations table | 17.3.0 |
| 2021-12 | RAN#94 | R5-217286 | 2137 | - | F | Correction to IMS Emergency call establishment test procedure with IMS emergency registration | 17.3.0 |
| 2021-12 | RAN#94 | R5-217298 | 2138 | - | F | Addition of test frequencies for Rel-15 EN-DC configurations | 17.3.0 |
| 2021-12 | RAN#94 | R5-217340 | 2139 | - | F | Correction of 4.3.1.2.4.4.2 for test frequency for intra-band non-contiguous CA_n260 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217366 | 2140 | - | F | Addition of default message for 16Tx PMI reporting test cases in 5.4.2.5 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217379 | 2141 | - | F | Addition of default DCI_0_2 for URLLC | 17.3.0 |
| 2021-12 | RAN#94 | R5-217390 | 2143 | - | F | Update to SIG test frequencies for V2X | 17.3.0 |
| 2021-12 | RAN#94 | R5-217430 | 2144 | - | F | Correction to the periodicity of CSI-RS for tracking | 17.3.0 |
| 2021-12 | RAN#94 | R5-217453 | 2146 | - | F | Updates to 4.8.4 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217527 | 2148 | - | F | message content update for HST single tap 1Tx PDSCH test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217555 | 2153 | - | F | Editorial correction in channel bandwidth clause | 17.3.0 |
| 2021-12 | RAN#94 | R5-217587 | 2155 | - | F | Addition of CA_n1A-n3A into TS 38.508-1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217588 | 2156 | - | F | Introduction of test frequencies for CA_n48(2A) for protocol testing | 17.3.0 |
| 2021-12 | RAN#94 | R5-217589 | 2157 | - | F | Introduction of test frequencies for n38 and CBWs 25 MHz and 30 MHz | 17.3.0 |
| 2021-12 | RAN#94 | R5-217682 | 2160 | - | F | Editorial update to clause 4.5A.2 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217735 | 2163 | - | F | Introduction of test frequencies for CA_n260K, CA_n260L and CA_n260M | 17.3.0 |
| 2021-12 | RAN#94 | R5-217766 | 2165 | - | F | Correction to NR V2X default configuration | 17.3.0 |
| 2021-12 | RAN#94 | R5-217792 | 2043 | 1 | F | Update chapter 4.5.4 RRC_CONNECTED | 17.3.0 |
| 2021-12 | RAN#94 | R5-217793 | 2044 | 1 | F | Update chapter 4.5.1 General | 17.3.0 |
| 2021-12 | RAN#94 | R5-217794 | 2045 | 1 | F | Update chapter 4.5.2 RRC_IDLE | 17.3.0 |
| 2021-12 | RAN#94 | R5-217795 | 2060 | 1 | F | Addition of new generic procedure for IMS MT video call establishment | 17.3.0 |
| 2021-12 | RAN#94 | R5-217796 | 2130 | 1 | F | Adding test procedure for adding video to a speech call | 17.3.0 |
| 2021-12 | RAN#94 | R5-217797 | 2136 | 1 | F | Correction to IMS MO Video call establishment test procedure | 17.3.0 |
| 2021-12 | RAN#94 | R5-217798 | 2147 | 1 | F | Adding test procedure for removing video from an ongoing call | 17.3.0 |
| 2021-12 | RAN#94 | R5-217799 | 2042 | 1 | F | Update RRCReconfiguration | 17.3.0 |
| 2021-12 | RAN#94 | R5-217800 | 2046 | 1 | F | Update IE CellAccessRelatedInfo | 17.3.0 |
| 2021-12 | RAN#94 | R5-217801 | 2127 | 1 | F | Update RRCReconfigurationComplete | 17.3.0 |
| 2021-12 | RAN#94 | R5-217877 | 2098 | 1 | F | Correction to default configuration of SIB12 | 17.3.0 |
| 2021-12 | RAN#94 | R5-217878 | 2099 | 1 | F | Addition of default configuration of DCI and SCI for NR SL test | 17.3.0 |
| 2021-12 | RAN#94 | R5-217879 | 2100 | 1 | F | Addition of power level setting for NR SL test | 17.3.0 |
| 2021-12 | RAN#94 | R5-217880 | 2108 | 1 | F | Correction to DIRECT LINK KEEPALIVE REQUEST msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217881 | 2110 | 1 | F | Correction to DIRECT LINK AUTHENTICATION REQUEST msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217882 | 2111 | 1 | F | Correction to DIRECT LINK AUTHENTICATION RESPONSE msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217883 | 2113 | 1 | F | Correction to DIRECT LINK SECURITY MODE COMMAND msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217884 | 2114 | 1 | F | Correction to DIRECT LINK SECURITY MODE COMPLETE msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217885 | 2118 | 1 | F | Correction to DIRECT LINK IDENTIFIER UPDATE REQUEST msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217886 | 2119 | 1 | F | Correction to DIRECT LINK IDENTIFIER UPDATE ACCEPT msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217887 | 2120 | 1 | F | Correction to DIRECT LINK IDENTIFIER UPDATE ACK msg | 17.3.0 |
| 2021-12 | RAN#94 | R5-217888 | 2123 | 1 | F | Correction to DIRECT LINK ESTABLISHMENT REJECT | 17.3.0 |
| 2021-12 | RAN#94 | R5-217933 | 2050 | 1 | F | Updates to UTRA signal levels in FR1 and OTA environment | 17.3.0 |
| 2021-12 | RAN#94 | R5-217943 | 2124 | 1 | F | Update Radio resource control information elements for NR 2-step RACH test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-217944 | 2161 | 1 | F | Test Procedure for eCall over IMS establishment in 5GS eCall only Support | 17.3.0 |
| 2021-12 | RAN#94 | R5-217949 | 2056 | 1 | F | Update IE MeasObjectNR | 17.3.0 |
| 2021-12 | RAN#94 | R5-217950 | 2057 | 1 | F | Update IE ServingCellConfigCommon | 17.3.0 |
| 2021-12 | RAN#94 | R5-217951 | 2058 | 1 | F | Update IE ServingCellConfigCommonSIB | 17.3.0 |
| 2021-12 | RAN#94 | R5-218220 | 2158 | 1 | F | Correction of test frequencies for CA_n257x | 17.3.0 |
| 2021-12 | RAN#94 | R5-218221 | 2129 | 1 | F | Add meas objects and report config for inter-RAT | 17.3.0 |
| 2021-12 | RAN#94 | R5-218267 | 2049 | 1 | F | Addition of NR inter-band CA configurations for CA_n260-n261 in FR2 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218268 | 2151 | 1 | F | Addition of R16 FR1+FR2 CADC configuration into 38.508-1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218269 | 2159 | 1 | F | Addition of test frequencies for R16 EN-DC FR2 configurations with n260 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218335 | 2065 | 1 | F | Correction to IE Table 4.6.3-141 - ReportConfigInterRAT | 17.3.0 |
| 2021-12 | RAN#94 | R5-218361 | 2152 | 1 | F | Addition of test frequencies for n3 CBW 50MHz | 17.3.0 |
| 2021-12 | RAN#94 | R5-218414 | 2166 | 1 | F | Update to demod parameter CSI-RS-ResourceMapping to add 1Tx condition | 17.3.0 |
| 2021-12 | RAN#94 | R5-218415 | 2164 | 1 | F | Update to n71 test frequencies for LTE-NR coex test cases | 17.3.0 |
| 2021-12 | RAN#94 | R5-218451 | 2132 | 1 | F | Introduction_of_test_frequencies_for_new_EN-DC_comb_within_FR1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218452 | 2149 | 1 | F | Introduction of test frequencies for CA_n48B BCS1 | 17.3.0 |
| 2021-12 | RAN#94 | R5-218470 | 2150 | 1 | F | Introduction of test frequencies for CA_n48B BCS2 | 17.3.0 |
| 2022-03 | RAN#95 | R5-220090 | 2171 | - | F | Correction of clause title tyles of 4.3.1.1.1.x | 17.4.0 |
| 2022-03 | RAN#95 | R5-220094 | 2172 | - | F | Correction of test channel bandwidth for n38 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220186 | 2186 | - | F | Editorial update of test procedure 4.9.15 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220206 | 2188 | - | F | Addition of test frequencies for CA_n3A-n41A with and without UL configuration | 17.4.0 |
| 2022-03 | RAN#95 | R5-220240 | 2189 | - | F | Corrections to 4.9.17 on IMS MO call release | 17.4.0 |
| 2022-03 | RAN#95 | R5-220248 | 2191 | - | F | Correct TDD pattern for FR2 RF 60kHz SCS | 17.4.0 |
| 2022-03 | RAN#95 | R5-220271 | 2192 | - | F | Addition of test frequencies for CA_n41A-n79A with UL configuration | 17.4.0 |
| 2022-03 | RAN#95 | R5-220308 | 2195 | - | F | Introduction of test frequencies for CA_n261M | 17.4.0 |
| 2022-03 | RAN#95 | R5-220309 | 2196 | - | F | Correction of NR inter-band CA configurations for CA_n260-n261 in FR2 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220311 | 2197 | - | F | Introduction of test frequencies for Rel-16 inter-band EN-DC two band combinations within FR1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220374 | 2198 | - | F | Introduction of test frequencies for additional Rel-17 EN-DC inter-band configurations | 17.4.0 |
| 2022-03 | RAN#95 | R5-220449 | 2199 | - | F | Update of test frequencies for protocol testing and NR inter-band CA | 17.4.0 |
| 2022-03 | RAN#95 | R5-220452 | 2200 | - | F | Correction to RF E-UTRA RRC_CONNECTED procedure | 17.4.0 |
| 2022-03 | RAN#95 | R5-220541 | 2204 | - | F | Introduction of test frequencies for n25 adding CBWs 25MHz, 30MHz, 40MHz | 17.4.0 |
| 2022-03 | RAN#95 | R5-220567 | 2206 | - | F | Addition of default AT command and information element for NR SL test | 17.4.0 |
| 2022-03 | RAN#95 | R5-220569 | 2208 | - | F | Correction to test procedures for establishing unicast link | 17.4.0 |
| 2022-03 | RAN#95 | R5-220579 | 2218 | - | F | Correction to SIB12 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220580 | 2219 | - | F | Correction to IE SL-BWP-Config and SL-BWP-ConfigCommon | 17.4.0 |
| 2022-03 | RAN#95 | R5-220582 | 2221 | - | F | Correction to IE SL-ResourcePool | 17.4.0 |
| 2022-03 | RAN#95 | R5-220590 | 2229 | - | F | Correction to V2X service identifier to PC5 RAT and Tx profiles mapping rule | 17.4.0 |
| 2022-03 | RAN#95 | R5-220591 | 2230 | - | F | Correction to V2X frequencies | 17.4.0 |
| 2022-03 | RAN#95 | R5-220632 | 2231 | - | F | Introduction_of_test_frequencies_for_new_EN-DC_comb_within_FR1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220643 | 2232 | - | F | Introduction of test frequencies for n2 adding CBWs 25MHz, 30MHz, 40MHz | 17.4.0 |
| 2022-03 | RAN#95 | R5-220653 | 2234 | - | F | Addition of test frequencies for CA_n41C-n79A with and without UL configuration | 17.4.0 |
| 2022-03 | RAN#95 | R5-220725 | 2237 | - | F | Addition of default configuration for NR SL RRM test | 17.4.0 |
| 2022-03 | RAN#95 | R5-220760 | 2238 | - | F | Introduction of test frequencies for n5 adding CBW 25MHz | 17.4.0 |
| 2022-03 | RAN#95 | R5-220761 | 2239 | - | F | Editorial corrections for NR CA configuration CA_n48B | 17.4.0 |
| 2022-03 | RAN#95 | R5-220771 | 2242 | - | F | Introduction of test frequencies for CA_n66(2A) BCS1 and BCS2 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220774 | 2243 | - | F | Corrections to test procedures 4.9.26 and 4.9.27 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220779 | 2245 | - | F | Update of test frequencies for n66 and asymmetric channel bandwidth combination set 1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220788 | 2246 | - | F | Addition of V2X connection diagram | 17.4.0 |
| 2022-03 | RAN#95 | R5-220797 | 2249 | - | F | Addition of connection diagram for 16Tx | 17.4.0 |
| 2022-03 | RAN#95 | R5-220808 | 2250 | - | F | Update to MAC-CellGroupConfig | 17.4.0 |
| 2022-03 | RAN#95 | R5-220847 | 2253 | - | F | Addition of test frequency n53 in Table 6.2.3.1-4 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220860 | 2254 | - | F | Introducing Rel-17 2 band CA configurations for n24 and n41 to clause 4.3.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220861 | 2255 | - | F | Introducing Rel-17 2 band CA configurations for n24 and n48 to clause 4.3.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220862 | 2256 | - | F | Introducing Rel-17 2 band CA configurations for n24 and n77 to clause 4.3.1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-220916 | 2259 | - | F | Addition of PRB-Id setting for RRM test cases | 17.4.0 |
| 2022-03 | RAN#95 | R5-220955 | 2262 | - | F | Updates to IE UE Route Selection Policy Rules | 17.4.0 |
| 2022-03 | RAN#95 | R5-220960 | 2264 | - | F | Addition of test frequency for DC_7C_n78A | 17.4.0 |
| 2022-03 | RAN#95 | R5-220972 | 2265 | - | F | Addition of test frequencies for Rel-16 EN-DC configurations | 17.4.0 |
| 2022-03 | RAN#95 | R5-220974 | 2266 | - | F | Correction to applicability for protocol testing for inter-band EN-DC configurations | 17.4.0 |
| 2022-03 | RAN#95 | R5-221070 | 2269 | - | F | USIM configurations for NR EIEI test cases | 17.4.0 |
| 2022-03 | RAN#95 | R5-221104 | 2270 | - | F | Addition of new USIM configuration for RACS test case | 17.4.0 |
| 2022-03 | RAN#95 | R5-221132 | 2271 | - | F | Updating channel bandwidths for NR band n97 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221133 | 2272 | - | F | Updating test frequencies for NR band n97 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221167 | 2275 | - | F | Update to Ciphering algo IE for FR1 NSA SDR | 17.4.0 |
| 2022-03 | RAN#95 | R5-221176 | 2276 | - | F | Introduction of test frequencies for NR CA configurations CA_n5A-n7A, CA_n5A-n78A and CA_n7A-n78A | 17.4.0 |
| 2022-03 | RAN#95 | R5-221186 | 2278 | - | F | Introduction of test frequencies for DC_1A_n5A, DC_1A_n7A, DC_3A_n5A, DC_7A_n5A, DC_28A_n7A | 17.4.0 |
| 2022-03 | RAN#95 | R5-221255 | 2282 | - | F | Update K1 value for RRM TDD FR1 30kHz RMC | 17.4.0 |
| 2022-03 | RAN#95 | R5-221264 | 2284 | - | F | Correction of csi-ResourceConfigId | 17.4.0 |
| 2022-03 | RAN#95 | R5-221265 | 2285 | - | F | Correction of DMRS-DownlinkConfig | 17.4.0 |
| 2022-03 | RAN#95 | R5-221292 | 2287 | - | F | Addition of Setup Diagram for RRM multicell 2x2 test cases | 17.4.0 |
| 2022-03 | RAN#95 | R5-221369 | 2289 | - | F | Update to PUCCH resource configuration for Scell CSI | 17.4.0 |
| 2022-03 | RAN#95 | R5-221377 | 2290 | - | F | Addition of Test frequencies for NE-DC band configurations for signalling testing | 17.4.0 |
| 2022-03 | RAN#95 | R5-221389 | 2291 | - | F | Correction to SIG test frequencies for intra-band non-contiguous CA | 17.4.0 |
| 2022-03 | RAN#95 | R5-221413 | 2293 | - | F | Update IE SDAP-Config | 17.4.0 |
| 2022-03 | RAN#95 | R5-221419 | 2173 | 1 | F | Update chapter 4.5.1 General | 17.4.0 |
| 2022-03 | RAN#95 | R5-221420 | 2233 | 1 | F | Correction to RRCReconfiguration message with condition REEST | 17.4.0 |
| 2022-03 | RAN#95 | R5-221421 | 2277 | 1 | F | Correction to the BWP-DownlinkDedicated | 17.4.0 |
| 2022-03 | RAN#95 | R5-221422 | 2280 | 1 | F | NE-DC support for UECapabilityEnquiry and UECapabilityInformation messages | 17.4.0 |
| 2022-03 | RAN#95 | R5-221423 | 2194 | 1 | F | Correction to IMS MO speech call establishment generic procedure | 17.4.0 |
| 2022-03 | RAN#95 | R5-221424 | 2176 | 1 | F | Correction to test procedure 4.9.11 IMS Emergency call or eCall over IMS establishment in 5GC with IMS emergency registration | 17.4.0 |
| 2022-03 | RAN#95 | R5-221425 | 2177 | 1 | F | Correction to test procedure 4.9.12 IMS Emergency call or eCall over IMS establishment in 5GC without IMS emergency registration | 17.4.0 |
| 2022-03 | RAN#95 | R5-221426 | 2263 | 1 | F | Updates to IE Route Selection Descriptors | 17.4.0 |
| 2022-03 | RAN#95 | R5-221491 | 2168 | 1 | F | Update of NR CA configurations for Protocol testing with NR CA 3CC | 17.4.0 |
| 2022-03 | RAN#95 | R5-221492 | 2169 | 1 | F | Update of inter-band cell environment for Protocol testing with NR CA 3CC | 17.4.0 |
| 2022-03 | RAN#95 | R5-221493 | 2286 | 1 | F | Modification of common test environment for EHC testing | 17.4.0 |
| 2022-03 | RAN#95 | R5-221499 | 2207 | 1 | F | Correction to generic test procedures for NR SL MIMO tests | 17.4.0 |
| 2022-03 | RAN#95 | R5-221500 | 2209 | 1 | F | Addition of test procedures for releasing unicast link | 17.4.0 |
| 2022-03 | RAN#95 | R5-221501 | 2210 | 1 | F | Addition of test procedures for data exchanging on unicast link | 17.4.0 |
| 2022-03 | RAN#95 | R5-221502 | 2211 | 1 | F | Correction to PC5 RRC message MasterInformationBlockSidelink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221503 | 2212 | 1 | F | Correction to PC5 RRC message MeasurementReportSidelink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221504 | 2213 | 1 | F | Correction to PC5 RRC message RRCReconfigurationSidelink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221505 | 2214 | 1 | F | Correction to PC5 RRC message RRCReconfigurationCompleteSidelink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221506 | 2215 | 1 | F | Correction to PC5 RRC message RRCReconfigurationFailureSidelink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221507 | 2216 | 1 | F | Correction to PC5 RRC message UECapabilityEnquirySidelink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221508 | 2217 | 1 | F | Correction to PC5 RRC message UECapabilityInformationSidelink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221509 | 2222 | 1 | F | Correction to V2X message DIRECT LINK ESTABLISHMENT REQUEST | 17.4.0 |
| 2022-03 | RAN#95 | R5-221510 | 2223 | 1 | F | Correction to V2X message DIRECT LINK ESTABLISHMENT ACCEPT | 17.4.0 |
| 2022-03 | RAN#95 | R5-221511 | 2224 | 1 | F | Correction to V2X message DIRECT LINK RELEASE REQUEST | 17.4.0 |
| 2022-03 | RAN#95 | R5-221512 | 2225 | 1 | F | Correction to V2X message DIRECT LINK RELEASE ACCEPT | 17.4.0 |
| 2022-03 | RAN#95 | R5-221513 | 2226 | 1 | F | Correction to V2X message DIRECT LINK KEEPALIVE REQUEST | 17.4.0 |
| 2022-03 | RAN#95 | R5-221514 | 2227 | 1 | F | Correction to V2X message DIRECT LINK SECURITY MODE COMMAND | 17.4.0 |
| 2022-03 | RAN#95 | R5-221515 | 2228 | 1 | F | Correction to V2X message DIRECT LINK SECURITY MODE COMPLETE | 17.4.0 |
| 2022-03 | RAN#95 | R5-221584 | 2251 | 1 | F | Addition of default DCI_1_2 for URLLC | 17.4.0 |
| 2022-03 | RAN#95 | R5-221591 | 2203 | 1 | F | Addition of positioning system information blocks associated parameters | 17.4.0 |
| 2022-03 | RAN#95 | R5-221667 | 2258 | 1 | F | Correction to test frequency range for n14 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221668 | 2268 | 1 | F | Correction of 4.3.1.2.4.4.2 for test frequencies for CA_n260_A-I | 17.4.0 |
| 2022-03 | RAN#95 | R5-221669 | 2236 | 1 | F | Correction to default RRC IEs for RRM | 17.4.0 |
| 2022-03 | RAN#95 | R5-221670 | 2167 | 1 | F | Updated the related RRC information for DSS | 17.4.0 |
| 2022-03 | RAN#95 | R5-221671 | 2190 | 1 | F | Added FR2 connection diagram using modulated interferer | 17.4.0 |
| 2022-03 | RAN#95 | R5-221672 | 2244 | 1 | F | Correction of 4.1.1 on removal of lower humidity limit in NR test environment | 17.4.0 |
| 2022-03 | RAN#95 | R5-221757 | 2267 | 1 | F | Correction of 4.3.1.1.5.48 for test frequencies of CA_n48_2A | 17.4.0 |
| 2022-03 | RAN#95 | R5-221787 | 2283 | 1 | F | Update of mid test channel bandwidth for band n25 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221815 | 2247 | 1 | F | Update to NR sidelink preconfiguration | 17.4.0 |
| 2022-03 | RAN#95 | R5-221816 | 2248 | 1 | F | Update to GNSS configuration for NR sidelink | 17.4.0 |
| 2022-03 | RAN#95 | R5-221869 | 2273 | 1 | F | Updating test frequencies for NR band n1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221871 | 2185 | 1 | F | Add test frequencies for R17 NR inter-band CA configurations in FR1 | 17.4.0 |
| 2022-03 | RAN#95 | R5-221872 | 2187 | 1 | F | Addition of test frequencies for NE-DC configurations DC_n28A_3A, DC_n28A_3C, DC_n28A_39A, DC_n28A_39C | 17.4.0 |
| 2022-03 | RAN#95 | R5-221873 | 2202 | 1 | F | Update of protocol testing applicability for 3CC inter-band NR DC configurations | 17.4.0 |
| 2022-03 | RAN#95 | R5-221874 | 2261 | 1 | F | Addition of several NR CA combinations to FR1 inter-band configurations table | 17.4.0 |
| 2022-03 | RAN#95 | R5-222039 | 2205 | 1 | F | Correction to cl 4.5.3 RRC_INACTIVE generic procedure | 17.4.0 |
| 2022-06 | RAN#96 | R5-222122 | 2294 | - | F | Correction for Procedure for UE-requested PDU session modification after the first S1 to N1 mode change | 17.5.0 |
| 2022-06 | RAN#96 | R5-222173 | 2296 | - | F | Introduction of test frequencies for CA_n77C BCS0 and BCS1 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222175 | 2297 | - | F | Introduction of test frequencies for CA_n77C for protocol testing | 17.5.0 |
| 2022-06 | RAN#96 | R5-222283 | 2300 | - | F | Introduction of test frequencies for Rel-16 inter-band EN-DC three band combinations within FR1 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222308 | 2302 | - | F | Introduction of NR-DC in FR1 for test setup diagrams | 17.5.0 |
| 2022-06 | RAN#96 | R5-222329 | 2303 | - | F | Correction of test frequencies for CA_n66(2A) BCS1 and BCS2 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222380 | 2308 | - | F | Updating RRCReconfiguration and RadioBearerConfig for NR-DC and NE-DC | 17.5.0 |
| 2022-06 | RAN#96 | R5-222431 | 2310 | - | F | Correction to message contents for CQI reporting | 17.5.0 |
| 2022-06 | RAN#96 | R5-222460 | 2312 | - | F | Updates to REGISTRATION ACCEPT message | 17.5.0 |
| 2022-06 | RAN#96 | R5-222461 | 2313 | - | F | Updates to Configuration Update Command message | 17.5.0 |
| 2022-06 | RAN#96 | R5-222462 | 2314 | - | F | Updates to Registration Reject message | 17.5.0 |
| 2022-06 | RAN#96 | R5-222463 | 2315 | - | F | Updates to De-registration Request message | 17.5.0 |
| 2022-06 | RAN#96 | R5-222464 | 2316 | - | F | Update of Combinations of system information blocks for NE-DC | 17.5.0 |
| 2022-06 | RAN#96 | R5-222502 | 2317 | - | F | Addition of test frequency for performance test cases | 17.5.0 |
| 2022-06 | RAN#96 | R5-222503 | 2318 | - | F | Addition of locationAndBandwidth for BW 45 MHz | 17.5.0 |
| 2022-06 | RAN#96 | R5-222512 | 2319 | - | F | Correction to generic procedure 4.9.28 | 17.5.0 |
| 2022-06 | RAN#96 | R5-222513 | 2320 | - | F | Editorial update RRCReconfiguration | 17.5.0 |
| 2022-06 | RAN#96 | R5-222537 | 2321 | - | F | Corrections to Table 7.3.1-12G | 17.5.0 |
| 2022-06 | RAN#96 | R5-222565 | 2324 | - | F | Update IE P-Max | 17.5.0 |
| 2022-06 | RAN#96 | R5-222566 | 2325 | - | F | Editorial update IE FreqBandList | 17.5.0 |
| 2022-06 | RAN#96 | R5-222567 | 2326 | - | F | Editorial update IE CellGroupConfig | 17.5.0 |
| 2022-06 | RAN#96 | R5-222568 | 2327 | - | F | Editorial update IE CellGroupId | 17.5.0 |
| 2022-06 | RAN#96 | R5-222570 | 2328 | - | F | Editorial update IE PDCCH-ConfigCommon | 17.5.0 |
| 2022-06 | RAN#96 | R5-222572 | 2329 | - | F | Addition of CA configuration for CA_n29A-n71A | 17.5.0 |
| 2022-06 | RAN#96 | R5-222617 | 2331 | - | F | Addition of default message contents for NR SL Demod | 17.5.0 |
| 2022-06 | RAN#96 | R5-222647 | 2338 | - | F | Correction to sidelink IE SL-ReportConfigList | 17.5.0 |
| 2022-06 | RAN#96 | R5-222650 | 2341 | - | F | Correction to general functional requirements for RedCap test | 17.5.0 |
| 2022-06 | RAN#96 | R5-222657 | 2344 | - | F | Editorial update IE SCellIndex | 17.5.0 |
| 2022-06 | RAN#96 | R5-222818 | 2346 | - | F | Correction to V2X message | 17.5.0 |
| 2022-06 | RAN#96 | R5-222836 | 2354 | - | F | Clarification of Annex C for calculation of SSB and CORESET#0 for PCells | 17.5.0 |
| 2022-06 | RAN#96 | R5-222876 | 2370 | - | F | Removing redundant ciphering algorithm for SDR testing | 17.5.0 |
| 2022-06 | RAN#96 | R5-222917 | 2372 | - | F | Connection diagram for 1x2 nDLCA Demodulation and CSI cases | 17.5.0 |
| 2022-06 | RAN#96 | R5-222924 | 2373 | - | F | Addition of connection diagram for Tx Diversity support | 17.5.0 |
| 2022-06 | RAN#96 | R5-222933 | 2374 | - | F | Update of auxiliary procedure 4.5A.2B | 17.5.0 |
| 2022-06 | RAN#96 | R5-223025 | 2376 | - | F | Update of NR inter-band CA configurations in FR1 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223067 | 2378 | - | F | Addition of test frequency for NR inter-band CA configurations including n1 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223072 | 2379 | - | F | Introduction of test frequencies for CA_n258G for protocol testing | 17.5.0 |
| 2022-06 | RAN#96 | R5-223084 | 2382 | - | F | Corrections to usages of Annex A.6 of TS 34.229-5 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223126 | 2397 | - | F | Introducing band configuration DC_20A_n257A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223158 | 2399 | - | F | Editorial update IE ServCellIndex | 17.5.0 |
| 2022-06 | RAN#96 | R5-223197 | 2401 | - | F | Introduction of test frequencies for additional Rel-17 NR CA and EN-DC inter-band configurations | 17.5.0 |
| 2022-06 | RAN#96 | R5-223222 | 2403 | - | F | Correction to 4.3.1.1.2.1 on test frequencies for NR inter-band CA configurations in FR1 with two bands | 17.5.0 |
| 2022-06 | RAN#96 | R5-223223 | 2404 | - | F | Correction to 4.3.1.1.2.2 on test frequencies for NR inter-band CA configurations in FR1 with three bands | 17.5.0 |
| 2022-06 | RAN#96 | R5-223224 | 2405 | - | F | Correction to 4.3.1.1.5.66 on test frequencies for NR intra-band non-contiguous CA configurations of CA_n66 with class 2A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223225 | 2406 | - | F | Correction to 4.3.1.1.5.71 on test frequencies for NR intra-band non-contiguous CA configurations of CA_n71 with class 2A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223229 | 2410 | - | F | Correction to 4.3.1.4.1.3 on test frequencies for inter-band EN-DC R17 configurations with three bands | 17.5.0 |
| 2022-06 | RAN#96 | R5-223234 | 2411 | - | F | Editorial correction to 4.3.1.2.2 on test frequencies for NR inter-band CA configurations in FR2 for CA_n260-n261 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223250 | 2412 | - | F | Hardcoding USIM configurations | 17.5.0 |
| 2022-06 | RAN#96 | R5-223341 | 2375 | 1 | F | Update of Test procedure for IMS MO Emergency call release | 17.5.0 |
| 2022-06 | RAN#96 | R5-223359 | 2332 | 1 | F | Addition of test frequency for NR SL concurrent | 17.5.0 |
| 2022-06 | RAN#96 | R5-223360 | 2333 | 1 | F | Correction to default configuration of SCI | 17.5.0 |
| 2022-06 | RAN#96 | R5-223361 | 2334 | 1 | F | Correction to sidelink IE SL-BWP-PoolConfig | 17.5.0 |
| 2022-06 | RAN#96 | R5-223362 | 2335 | 1 | F | Correction to sidelink IE SL-BWP-PoolConfigCommon | 17.5.0 |
| 2022-06 | RAN#96 | R5-223363 | 2336 | 1 | F | Correction to sidelink IE SL-FreqConfig | 17.5.0 |
| 2022-06 | RAN#96 | R5-223364 | 2337 | 1 | F | Correction to sidelink IE SL-FreqConfigCommon | 17.5.0 |
| 2022-06 | RAN#96 | R5-223365 | 2339 | 1 | F | Correction to test procedures for unicast link establishment | 17.5.0 |
| 2022-06 | RAN#96 | R5-223387 | 2330 | 1 | F | Addition of scheduling information for positioning system information blocks | 17.5.0 |
| 2022-06 | RAN#96 | R5-223399 | 2295 | 1 | F | Addition of SIB11 to common environment for early measurements | 17.5.0 |
| 2022-06 | RAN#96 | R5-223400 | 2414 | 1 | F | Modification of SIB1 in common environment for idle/inactive measurements | 17.5.0 |
| 2022-06 | RAN#96 | R5-223410 | 2340 | 1 | F | Addition of abbreviations for RedCap test | 17.5.0 |
| 2022-06 | RAN#96 | R5-223413 | 2380 | 1 | F | Updates to Test procedure 4.9.15 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223414 | 2307 | 1 | F | Editorial updates to SIBs | 17.5.0 |
| 2022-06 | RAN#96 | R5-223415 | 2381 | 1 | F | Updates to Data-off condition for PDU SESSION ESTABLISHMENT REQUEST message | 17.5.0 |
| 2022-06 | RAN#96 | R5-223416 | 2299 | 1 | F | Resolving test frequency for n53 10 Mhz CBW | 17.5.0 |
| 2022-06 | RAN#96 | R5-223417 | 2353 | 1 | F | Correction to Combinations of system information blocks | 17.5.0 |
| 2022-06 | RAN#96 | R5-223480 | 2402 | 1 | F | Corrections on mandatory channel bandwidths after Rel-15 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223602 | 2415 | - | F | Correction to 4.3.1.4.1.3 on test frequencies for DC_1A-28A_n78C | 17.5.0 |
| 2022-06 | RAN#96 | R5-223649 | 2398 | 1 | F | Introduction of test frequencies for 3 band EN-DC configurations | 17.5.0 |
| 2022-06 | RAN#96 | R5-223650 | 2400 | 1 | F | Introduction of test frequencies for additional Rel-16 NR CA DC and EN-DC inter-band configurations | 17.5.0 |
| 2022-06 | RAN#96 | R5-223651 | 2407 | 1 | F | Correction to 4.3.1.1.5.77 on test frequencies for NR intra-band non-contiguous CA configurations of CA_n77 with class 2A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223652 | 2408 | 1 | F | Correction to 4.3.1.1.5.78 on test frequencies for NR intra-band non-contiguous CA configurations of CA_n78 with class 2A | 17.5.0 |
| 2022-06 | RAN#96 | R5-223653 | 2409 | 1 | F | Correction to 4.3.1.4.1.3 on test frequencies for inter-band EN-DC R16 configurations | 17.5.0 |
| 2022-06 | RAN#96 | R5-223755 | 2301 | 1 | F | Introduction of test frequencies for NR-DC in FR1 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223784 | 2371 | 1 | F | Addition of RedCap default test channel bandwidth | 17.5.0 |
| 2022-06 | RAN#96 | R5-223792 | 2322 | 1 | F | Correction to test frequency for n53 | 17.5.0 |
| 2022-06 | RAN#96 | R5-223793 | 2369 | 1 | F | Clarification of PCC and SCC configuration for CA test cases | 17.5.0 |
| 2022-06 | RAN#96 | R5-223795 | 2323 | 1 | F | CR on Permitted Methodologies and Applicability | 17.5.0 |
| 2022-06 | RAN#96 | R5-223796 | 2377 | 1 | F | Add new messages and procedure for test function to limit Pcell Power | 17.5.0 |
| 2022-09 | RAN#97 | R5-223987 | 2416 | - | F | Correction to test procedure 4.9.11 IMS Emergency call or eCall over IMS establishment in 5GC with IMS emergency registration | 17.6.0 |
| 2022-09 | RAN#97 | R5-224033 | 2417 | - | F | Introduction of test frequencies for NR-U band n46 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224034 | 2418 | - | F | Introduction of test frequencies for NR-U band n96 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224094 | 2429 | - | F | Update of MDT default RRC messages and IEs | 17.6.0 |
| 2022-09 | RAN#97 | R5-224139 | 2434 | - | F | Add IE BeamFailureRecoveryServingCellConfig | 17.6.0 |
| 2022-09 | RAN#97 | R5-224140 | 2435 | - | F | Add IE BetaOffsetsCrossPri | 17.6.0 |
| 2022-09 | RAN#97 | R5-224149 | 2436 | - | F | Introduction of test frequencies for Rel-16 inter-band DC_3A-7A-20A_n8A within FR1 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224166 | 2437 | - | F | Correction to Table 4.7.2-2 PDU Session Establishment Accept | 17.6.0 |
| 2022-09 | RAN#97 | R5-224179 | 2438 | - | F | Test frequencies for NR DC configurations in FR1 for signalling testing | 17.6.0 |
| 2022-09 | RAN#97 | R5-224235 | 2443 | - | F | Correction of the styles of the subclauses in 4.3.1.1 and 4.3.1.2 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224276 | 2445 | - | F | Add IE CandidateBeamRS | 17.6.0 |
| 2022-09 | RAN#97 | R5-224284 | 2446 | - | F | Add IE CFR-ConfigMulticast | 17.6.0 |
| 2022-09 | RAN#97 | R5-224290 | 2447 | - | F | Add IEs DL-PRS-ProcessingWindowPreConfig, DMRS-BundlingPUCCH-Config and DMRS-BundlingPUSCH-Config | 17.6.0 |
| 2022-09 | RAN#97 | R5-224380 | 2459 | - | F | Add IE FreqPriorityListNRSlicing | 17.6.0 |
| 2022-09 | RAN#97 | R5-224458 | 2465 | - | F | Alignment CSI-ResourcePeriodicityAndOffset to CSI-RS.3.2 TDD | 17.6.0 |
| 2022-09 | RAN#97 | R5-224459 | 2466 | - | F | Correction derivation path Table 7.3.1-35 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224460 | 2467 | - | F | Correction to 7.3.1-12G | 17.6.0 |
| 2022-09 | RAN#97 | R5-224462 | 2469 | - | F | Definition of ZP-CSI-RS-ResourceSetId | 17.6.0 |
| 2022-09 | RAN#97 | R5-224638 | 2478 | - | F | Clarification to Antenna ports of DCI format 1_1 for RF TCs | 17.6.0 |
| 2022-09 | RAN#97 | R5-224639 | 2479 | - | F | Correction to CSI-MeasConfig for RRM | 17.6.0 |
| 2022-09 | RAN#97 | R5-224658 | 2482 | - | F | Add IE GapPriority | 17.6.0 |
| 2022-09 | RAN#97 | R5-224662 | 2484 | - | F | Add IE HysteresisLocation | 17.6.0 |
| 2022-09 | RAN#97 | R5-224685 | 2485 | - | F | Correction to IMS MO Video call establishment test procedure in 5GC | 17.6.0 |
| 2022-09 | RAN#97 | R5-224686 | 2486 | - | F | Correction to IMS MT Video call establishment test procedure in 5GC | 17.6.0 |
| 2022-09 | RAN#97 | R5-224689 | 2489 | - | F | Correction to IMS MT speech call establishment test procedure in 5GC | 17.6.0 |
| 2022-09 | RAN#97 | R5-224725 | 2491 | - | F | Update the SN-FieldLength of PDCP-Config and RLC-Config for RedCap test | 17.6.0 |
| 2022-09 | RAN#97 | R5-224740 | 2495 | - | F | Add Default configuration for DCI format 4_1 scheduling MBS Multicast test | 17.6.0 |
| 2022-09 | RAN#97 | R5-224741 | 2496 | - | F | Add Default configuration for DCI format 4_2 scheduling MBS Multicast test | 17.6.0 |
| 2022-09 | RAN#97 | R5-224762 | 2508 | - | F | Add IE MeasGapId | 17.6.0 |
| 2022-09 | RAN#97 | R5-224763 | 2509 | - | F | Add IE MeasObjectRxTxDiff | 17.6.0 |
| 2022-09 | RAN#97 | R5-224816 | 2511 | - | F | Add IE MeasResultRxTxTimeDiff | 17.6.0 |
| 2022-09 | RAN#97 | R5-224817 | 2512 | - | F | Add IE MRB-Identity | 17.6.0 |
| 2022-09 | RAN#97 | R5-224818 | 2513 | - | F | Add IEs MUSIM-GapConfig, MUSIM-GapID and MUSIM-GapInfo | 17.6.0 |
| 2022-09 | RAN#97 | R5-224819 | 2514 | - | F | Add IEs NeedForGapNCSG-ConfigEUTRA, NeedForGapNCSG-ConfigNR, NeedForGapNCSG-InfoEUTRA and NeedForGapNCSG-InfoNR | 17.6.0 |
| 2022-09 | RAN#97 | R5-224820 | 2515 | - | F | Add IE NonCellDefiningSSB | 17.6.0 |
| 2022-09 | RAN#97 | R5-224821 | 2516 | - | F | Add IE NR-DL-PRS-PDC-Info | 17.6.0 |
| 2022-09 | RAN#97 | R5-224822 | 2517 | - | F | Add IEs NSAG-IdentityInfo and NTN-Config | 17.6.0 |
| 2022-09 | RAN#97 | R5-224824 | 2519 | - | F | Add IEs RxTxTimeDiff, SCellActivationRS-Config and SCellActivationRS-ConfigId | 17.6.0 |
| 2022-09 | RAN#97 | R5-224825 | 2520 | - | F | Add IE SemiStaticChannelAccessConfigUE | 17.6.0 |
| 2022-09 | RAN#97 | R5-224859 | 2521 | - | F | Add IE TCI-Info | 17.6.0 |
| 2022-09 | RAN#97 | R5-224860 | 2522 | - | F | Add IE UE-TimersAndConstantsRemoteUE | 17.6.0 |
| 2022-09 | RAN#97 | R5-224861 | 2523 | - | F | Add IEs UL-ExcessDelayConfig and UE UL-GapFR2-Config | 17.6.0 |
| 2022-09 | RAN#97 | R5-224862 | 2524 | - | F | Add IEs Uplink-PowerControl, Uu-RelayRLC-ChannelConfig and Uu-RelayRLC-ChannelID | 17.6.0 |
| 2022-09 | RAN#97 | R5-224863 | 2525 | - | F | Add IE AppLayerMeasParameters | 17.6.0 |
| 2022-09 | RAN#97 | R5-224864 | 2526 | - | F | Add IE FR2-2-AccessParamsPerBand | 17.6.0 |
| 2022-09 | RAN#97 | R5-224877 | 2528 | - | F | Add IE NTN-Parameters | 17.6.0 |
| 2022-09 | RAN#97 | R5-224878 | 2529 | - | F | Add IE PosSRS-RRC-Inactive-OutsideInitialUL-BWP-r17 | 17.6.0 |
| 2022-09 | RAN#97 | R5-224880 | 2531 | - | F | Add IE RedCapParameters | 17.6.0 |
| 2022-09 | RAN#97 | R5-224881 | 2532 | - | F | Add IE SRS-AllPosResourcesRRC-Inactive | 17.6.0 |
| 2022-09 | RAN#97 | R5-224882 | 2533 | - | F | Add IE UE-RadioPagingInfo | 17.6.0 |
| 2022-09 | RAN#97 | R5-224883 | 2534 | - | F | Add IE AppLayerMeasConfig | 17.6.0 |
| 2022-09 | RAN#97 | R5-224909 | 2535 | - | F | Update IE DownlinkConfigCommonSIB | 17.6.0 |
| 2022-09 | RAN#97 | R5-224912 | 2537 | - | F | Update IE MeasObjectNR | 17.6.0 |
| 2022-09 | RAN#97 | R5-224913 | 2538 | - | F | Add IE MeasResultForRSSI | 17.6.0 |
| 2022-09 | RAN#97 | R5-224914 | 2539 | - | F | Update IE MeasResults | 17.6.0 |
| 2022-09 | RAN#97 | R5-224915 | 2540 | - | F | Add IE MeasRSSI-ReportConfig | 17.6.0 |
| 2022-09 | RAN#97 | R5-224916 | 2541 | - | F | Update IE ReportConfigNR | 17.6.0 |
| 2022-09 | RAN#97 | R5-224917 | 2542 | - | F | Add IE RMTC Config | 17.6.0 |
| 2022-09 | RAN#97 | R5-224992 | 2545 | - | F | Add IE DedicatedInfoF1c | 17.6.0 |
| 2022-09 | RAN#97 | R5-225007 | 2546 | - | F | Add IE MeasConfigAppLayerId | 17.6.0 |
| 2022-09 | RAN#97 | R5-225092 | 2550 | - | F | Adding new connection diagram for SUL with UL MIMO test case | 17.6.0 |
| 2022-09 | RAN#97 | R5-225177 | 2553 | - | F | Introduction of new default 5GMM messages | 17.6.0 |
| 2022-09 | RAN#97 | R5-225178 | 2554 | - | F | Introduction of new default 5GSM messages | 17.6.0 |
| 2022-09 | RAN#97 | R5-225180 | 2556 | - | F | Updates to default 5GSM messages | 17.6.0 |
| 2022-09 | RAN#97 | R5-225188 | 2557 | - | F | CR to Correct Permitted Methodologies and Applicability | 17.6.0 |
| 2022-09 | RAN#97 | R5-225212 | 2558 | - | F | Update nrofRB for 200 MHz ChBw 120 kHz SCS | 17.6.0 |
| 2022-09 | RAN#97 | R5-225215 | 2559 | - | F | Update number of HARQ processes for RRM test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-225271 | 2555 | 1 | F | Updates to default 5GMM messages | 17.6.0 |
| 2022-09 | RAN#97 | R5-225280 | 2494 | 1 | F | Add Default configuration for DCI format 4_0 scheduling MBS Broadcast test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225290 | 2471 | 1 | F | Correction of NR SL message contents | 17.6.0 |
| 2022-09 | RAN#97 | R5-225291 | 2472 | 1 | F | Correction of test procedure 4.9.31 - Connectivity check | 17.6.0 |
| 2022-09 | RAN#97 | R5-225299 | 2448 | 1 | F | Updates to Table 4.6.3-16: CellAccessRelatedInfo for NPN | 17.6.0 |
| 2022-09 | RAN#97 | R5-225300 | 2456 | 1 | F | Addition of Test Environment for legacy test cases applicable to SNPN Only UE | 17.6.0 |
| 2022-09 | RAN#97 | R5-225310 | 2544 | 1 | F | Update of protocol applicability for DC_19A_n77(2A), DC_19A_n78(2A), DC_21A_n77(2A) and DC_21A_n78(2A) | 17.6.0 |
| 2022-09 | RAN#97 | R5-225311 | 2462 | 1 | F | Correction of the scheduling information for combination NR-15 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225312 | 2463 | 1 | F | Addition of message contents for DedicatedSIBRequest | 17.6.0 |
| 2022-09 | RAN#97 | R5-225316 | 2425 | 1 | F | Update default message contents of RACH-ConfigDedicated | 17.6.0 |
| 2022-09 | RAN#97 | R5-225321 | 2536 | 1 | F | Update IE LBT FailureRecoveryConfig | 17.6.0 |
| 2022-09 | RAN#97 | R5-225325 | 2497 | 1 | F | Add test procedures for MBS Multicast test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225326 | 2502 | 1 | F | Add SIB20 for MBS Broadcast test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225327 | 2503 | 1 | F | Add SIB21 for MBS Broadcast test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225328 | 2504 | 1 | F | Add MBSBroadcastConfiguration for MBS Broadcast test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225329 | 2505 | 1 | F | Add MBSInterestIndication for MBS Broadcast test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225330 | 2506 | 1 | F | Add MBS information elements for MBS test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225333 | 2475 | 1 | F | Update of MDT message and information element | 17.6.0 |
| 2022-09 | RAN#97 | R5-225342 | 2492 | 1 | F | Addition of RedCap default test channel bandwidth for signaling MFBI test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225343 | 2493 | 1 | F | Addition of RedCap default test channel bandwidth for signaling test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225352 | 2449 | 1 | F | Update of Table 4.6.1-30 to add message contents values for SDT in UEAssistanceInformation | 17.6.0 |
| 2022-09 | RAN#97 | R5-225353 | 2450 | 1 | F | Update of Table 4.6.1-28 to add message contents values for SDT in SIB1 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225354 | 2451 | 1 | F | Update of Table 4.6.1-16 to add message contents values for SDT in RRCRelease | 17.6.0 |
| 2022-09 | RAN#97 | R5-225355 | 2453 | 1 | F | Update of Table 4.6.3-96 to add message contents values for SDT in PDCCH-ConfigCommon | 17.6.0 |
| 2022-09 | RAN#97 | R5-225356 | 2473 | 1 | F | Addition of SIB16 for slice based cell reselection | 17.6.0 |
| 2022-09 | RAN#97 | R5-225357 | 2474 | 1 | F | Updates to message contents for slice specific RACH configuration | 17.6.0 |
| 2022-09 | RAN#97 | R5-225358 | 2464 | 1 | F | Add UplinkDataCompression into PDCP-Config for NR UDC test | 17.6.0 |
| 2022-09 | RAN#97 | R5-225362 | 2483 | 1 | F | Updates to test procedures 4.9.12A and 4.9.12B | 17.6.0 |
| 2022-09 | RAN#97 | R5-225363 | 2458 | 1 | F | Add IEs DRX-ConfigSL, EphemerisInfo, FeatureCombination and FeatureCombinationPreambles | 17.6.0 |
| 2022-09 | RAN#97 | R5-225364 | 2530 | 1 | F | Editorial update IE RAT-Type | 17.6.0 |
| 2022-09 | RAN#97 | R5-225365 | 2549 | 1 | F | Updating the values for IE P-Max | 17.6.0 |
| 2022-09 | RAN#97 | R5-225366 | 2428 | 1 | F | Updates on UE Power Limit Messages: Option 2 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225367 | 2548 | 1 | F | Introducing FR2 signal test frequencies for intra-band contiguous 3CA | 17.6.0 |
| 2022-09 | RAN#97 | R5-225681 | 2441 | 1 | F | Correction of test channel bandwidth for R16 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225682 | 2442 | 1 | F | Correction of test channel bandwidth for n79 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225684 | 2440 | 1 | F | Correction of test channel bandwidth for R15 | 17.6.0 |
| 2022-09 | RAN#97 | R5-225686 | 2439 | 1 | F | Update UE capability information elements for PC1.5 duty cycle | 17.6.0 |
| 2022-09 | RAN#97 | R5-225730 | 2476 | 1 | F | Addition of test frequencies for DC_21A_n28A | 17.6.0 |
| 2022-09 | RAN#97 | R5-225731 | 2527 | 1 | F | Addition of configurations for many 4CA NR combinations | 17.6.0 |
| 2022-09 | RAN#97 | R5-225766 | 2543 | 1 | F | Corrections on Mid test channel bandwidth for band n66 for Redcap | 17.6.0 |
| 2022-09 | RAN#97 | R5-225780 | 2468 | 1 | F | Correction quantity config for inter-RAT to UTRA | 17.6.0 |
| 2022-09 | RAN#97 | R5-225781 | 2477 | 1 | F | Correction to connection diagram for DL CA Demodulation and CSI test cases | 17.6.0 |
| 2022-09 | RAN#97 | R5-225782 | 2481 | 1 | F | Correction to RRC message for uplink CA | 17.6.0 |
| 2022-12 | RAN#98 | R5-225952 | 2563 |  | F | Introduction of test channel bandwidths for new NR bands n91, n92, n93 and n94 | 17.7.0 |
| 2022-12 | RAN#98 | R5-225953 | 2564 |  | F | Introduction of test frequencies for new NR bands n91, n92, n93 and n94 | 17.7.0 |
| 2022-12 | RAN#98 | R5-225970 | 2566 |  | F | Inclusive Language Review and update of IE SIB3 | 17.7.0 |
| 2022-12 | RAN#98 | R5-225971 | 2567 |  | F | Inclusive Language Review and update of IE SIB4 | 17.7.0 |
| 2022-12 | RAN#98 | R5-225972 | 2568 |  | F | Inclusive Language Review and update of IE SIB5 | 17.7.0 |
| 2022-12 | RAN#98 | R5-225973 | 2569 |  | F | Inclusive Language Review and update of IE MeasObjectEUTRA | 17.7.0 |
| 2022-12 | RAN#98 | R5-225974 | 2570 |  | F | Inclusive Language Review and update of IE MeasObjectNR | 17.7.0 |
| 2022-12 | RAN#98 | R5-225975 | 2571 |  | F | Inclusive Language Review and update of IE ReportConfigNR | 17.7.0 |
| 2022-12 | RAN#98 | R5-226020 | 2572 |  | F | Update of test procedures 4.9.12A and 4.9.12B | 17.7.0 |
| 2022-12 | RAN#98 | R5-226040 | 2573 |  | F | Correction to test procedure 4.9.7 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226041 | 2574 |  | F | Updates to SysInfo for SCells operating on NR CA bands with no UL frequency | 17.7.0 |
| 2022-12 | RAN#98 | R5-226110 | 2575 |  | F | Correction to default configurations for RedCap RRM TCs | 17.7.0 |
| 2022-12 | RAN#98 | R5-226247 | 2577 |  | F | Add IE SIB17 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226248 | 2578 |  | F | Add IE SIB18 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226249 | 2579 |  | F | Add IE SIB19 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226250 | 2580 |  | F | Update IE BeamFailureRecoveryRSConfig | 17.7.0 |
| 2022-12 | RAN#98 | R5-226251 | 2581 |  | F | Update IE BeamFailureRecoveryServingCellConfig | 17.7.0 |
| 2022-12 | RAN#98 | R5-226253 | 2583 |  | F | Update IE DRX-ConfigSecondaryGroup | 17.7.0 |
| 2022-12 | RAN#98 | R5-226254 | 2584 |  | F | Update IE FreqPriorityListSlicing | 17.7.0 |
| 2022-12 | RAN#98 | R5-226255 | 2585 |  | F | Add IE FreqPriorityListDedicatedSlicing | 17.7.0 |
| 2022-12 | RAN#98 | R5-226257 | 2587 |  | F | Update IE MUSIM-GapId | 17.7.0 |
| 2022-12 | RAN#98 | R5-226259 | 2589 |  | F | Add IEs PCI-ARFCN-EUTRA and PCI-ARFCN-NR | 17.7.0 |
| 2022-12 | RAN#98 | R5-226260 | 2590 |  | F | Add IE ReferenceLocation | 17.7.0 |
| 2022-12 | RAN#98 | R5-226261 | 2591 |  | F | Update IE RMTC-Config | 17.7.0 |
| 2022-12 | RAN#98 | R5-226262 | 2592 |  | F | Add IE ServingCellAndBWP-Id | 17.7.0 |
| 2022-12 | RAN#98 | R5-226263 | 2593 |  | F | Update IE TCI-ActivatedConfig | 17.7.0 |
| 2022-12 | RAN#98 | R5-226264 | 2594 |  | F | Add IE TAR-Config | 17.7.0 |
| 2022-12 | RAN#98 | R5-226265 | 2595 |  | F | Add IEs TCI-UL-State and TCI-UL-StateId | 17.7.0 |
| 2022-12 | RAN#98 | R5-226266 | 2596 |  | F | Add IE TimeAlignmentTimer | 17.7.0 |
| 2022-12 | RAN#98 | R5-226268 | 2598 |  | F | Update IE PosSRS-RRC-Inactive-OutsideInitialUL-BWP | 17.7.0 |
| 2022-12 | RAN#98 | R5-226269 | 2599 |  | F | Add IE PRS-ProcessingCapabilityOutsideMGinPPWperType | 17.7.0 |
| 2022-12 | RAN#98 | R5-226270 | 2600 |  | F | Add IE SimultaneousRxTxPerBandPair | 17.7.0 |
| 2022-12 | RAN#98 | R5-226271 | 2601 |  | F | Update IE UE-MeasurementsAvailable | 17.7.0 |
| 2022-12 | RAN#98 | R5-226352 | 2605 |  | F | Addition of CGI specific information elements for R15 SON_MDT | 17.7.0 |
| 2022-12 | RAN#98 | R5-226366 | 2607 |  | F | Removal of Test freqs for R16 pending CADC configs from cl 4.3.1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226367 | 2608 |  | F | Removal of Test freqs for R17 pending CADC configs from cl 4.3.1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226390 | 2613 |  | F | Editorial correction for test procedure sequence in 4.7.2-2 on 38.508-1 in rel17 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226400 | 2615 |  | F | Updates for NR CA_n5A-n77A, CA_n66A-n77A | 17.7.0 |
| 2022-12 | RAN#98 | R5-226494 | 2616 |  | F | Correct typo in Table 4.6.3-68 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226495 | 2617 |  | F | Corrections to Table 7.3.1-22 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226496 | 2618 |  | F | Corrections to Table 7.3.1-31 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226669 | 2625 |  | F | Update to K1 value for FDD RRM test cases | 17.7.0 |
| 2022-12 | RAN#98 | R5-226684 | 2626 |  | F | Addition of 2x1 antenna connection for RedCap | 17.7.0 |
| 2022-12 | RAN#98 | R5-226691 | 2627 |  | F | n66_2A frequency correction | 17.7.0 |
| 2022-12 | RAN#98 | R5-226693 | 2628 |  | F | Corrections to ssbPositionInBurst for 7.3.1-x | 17.7.0 |
| 2022-12 | RAN#98 | R5-226694 | 2629 |  | F | Corrections to RACHConfigCommon for RRM | 17.7.0 |
| 2022-12 | RAN#98 | R5-226695 | 2630 |  | F | Removal of Table 7.3.1-33 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226718 | 2638 |  | F | Add low-mid-high ch BWs for NR-U | 17.7.0 |
| 2022-12 | RAN#98 | R5-226727 | 2639 |  | F | Update of test frequency for n3 R17 CBW | 17.7.0 |
| 2022-12 | RAN#98 | R5-226731 | 2641 |  | F | Update IE Phy-ParametersSharedSpectrumChAccess | 17.7.0 |
| 2022-12 | RAN#98 | R5-226863 | 2644 |  | F | Updating signalling test frequencies for n79 20MHz CBW | 17.7.0 |
| 2022-12 | RAN#98 | R5-226864 | 2645 |  | F | Updating test channel bandwidths for n79 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226894 | 2646 |  | F | Update of n28 test frequencies for DC_21A_n28A in clause 6.2.3.1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-226934 | 2647 |  | F | Updates to default 5GMM messages | 17.7.0 |
| 2022-12 | RAN#98 | R5-226935 | 2648 |  | F | Updates to default 5GSM messages | 17.7.0 |
| 2022-12 | RAN#98 | R5-227084 | 2650 |  | F | Update to offsetToCarrier for DC_21A_n28A | 17.7.0 |
| 2022-12 | RAN#98 | R5-227087 | 2651 |  | F | Addition of locationAndBandwidth for BW 35 MHz | 17.7.0 |
| 2022-12 | RAN#98 | R5-227101 | 2653 |  | F | Correction to definition of SRB_NR_PDCP | 17.7.0 |
| 2022-12 | RAN#98 | R5-227108 | 2655 |  | F | Test frequencies updates for band n7 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227110 | 2657 |  | F | Test frequencies updates for band n25 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227111 | 2658 |  | F | Test frequencies updates for band n66 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227138 | 2660 |  | F | Update of configuration of system information blocks for MBS Broadcast TC | 17.7.0 |
| 2022-12 | RAN#98 | R5-227139 | 2661 |  | F | Update of MBS IE CFR-ConfigMCCH-MTCH | 17.7.0 |
| 2022-12 | RAN#98 | R5-227142 | 2664 |  | F | Editoral correction of physical layer parameters for scheduling of MBS | 17.7.0 |
| 2022-12 | RAN#98 | R5-227174 | 2671 |  | F | Update PC5 message for LINK IDENTIFIER UPDATE procedure | 17.7.0 |
| 2022-12 | RAN#98 | R5-227251 | 2674 |  | F | Correction of test frequencies | 17.7.0 |
| 2022-12 | RAN#98 | R5-227275 | 2677 |  | F | Introduction of CA_n48A-n77A and CA_n71A-n77A configurations | 17.7.0 |
| 2022-12 | RAN#98 | R5-227406 | 2614 | 1 | F | Addition of Test procedure for deleting configured S-NSSAI, default configured S-NSSAI and allowed S-NSSAI | 17.7.0 |
| 2022-12 | RAN#98 | R5-227407 | 2582 | 1 | F | Update IE DL-PPW-PreConfig | 17.7.0 |
| 2022-12 | RAN#98 | R5-227408 | 2586 | 1 | F | Add IE LTE-NeighCellsCRS-AssistInfoList | 17.7.0 |
| 2022-12 | RAN#98 | R5-227409 | 2597 | 1 | F | Add IEs UplinkTxDirectCurrentMoreCarrierList and UplinkTxDirectCurrentTwoCarrierList | 17.7.0 |
| 2022-12 | RAN#98 | R5-227410 | 2676 | 1 | F | Updates to connectivity options | 17.7.0 |
| 2022-12 | RAN#98 | R5-227449 | 2619 | 1 | F | Update to cl 4.5B.1 for SNPN-only UEs | 17.7.0 |
| 2022-12 | RAN#98 | R5-227473 | 2649 | 1 | F | Correction to EHC parametrization in 38.508-1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227476 | 2678 | 1 | F | Addition of USIM configuration for MUSIM test cases | 17.7.0 |
| 2022-12 | RAN#98 | R5-227477 | 2679 | 1 | F | Addition of test procedure for registration of a MUSIM UE | 17.7.0 |
| 2022-12 | RAN#98 | R5-227480 | 2620 | 1 | F | Update of Table 4.6.1-28 to add message contents values for NTN in SIB1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227499 | 2672 | 1 | F | Updating SIB1 content with Paging Early Indication configuration | 17.7.0 |
| 2022-12 | RAN#98 | R5-227507 | 2659 | 1 | F | Update of CLOSE UE TEST LOOP message | 17.7.0 |
| 2022-12 | RAN#98 | R5-227508 | 2662 | 1 | F | Update of MBS IE TMGI | 17.7.0 |
| 2022-12 | RAN#98 | R5-227509 | 2663 | 1 | F | Update of Radio resource control information elements for Broadcast MBS TC | 17.7.0 |
| 2022-12 | RAN#98 | R5-227510 | 2665 | 1 | F | Update of DNN configuration for PDU session associated with MBS Multicast session | 17.7.0 |
| 2022-12 | RAN#98 | R5-227511 | 2666 | 1 | F | Update of Procedure for MBS Multicast session join and session establishment | 17.7.0 |
| 2022-12 | RAN#98 | R5-227512 | 2667 | 1 | F | Update of Radio resource control information elements for Multicast MBS TC | 17.7.0 |
| 2022-12 | RAN#98 | R5-227518 | 2670 | 1 | F | Update common configurations for RedCap UE | 17.7.0 |
| 2022-12 | RAN#98 | R5-227542 | 2609 | 1 | F | Update of information elements for slice specific RACH configuration | 17.7.0 |
| 2022-12 | RAN#98 | R5-227544 | 2610 | 1 | F | Update of MDT information element for SON_MDT | 17.7.0 |
| 2022-12 | RAN#98 | R5-227587 | 2654 | 1 | F | Update TS 38.508-1 clause 4.5B for RedCap UE | 17.7.0 |
| 2022-12 | RAN#98 | R5-227593 | 2611 | 1 | F | Addition of parameter related to SOR-CMCI for DL NAS transport and UL NAS transport message in 38.508-1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227700 | 2606 | 1 | F | Removal of Test freqs for R15 pending CADC configs from cl 4.3.1 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227701 | 2640 | 1 | F | Update of test frequency for n8 R17 CBW 35MHz | 17.7.0 |
| 2022-12 | RAN#98 | R5-227702 | 2643 | 1 | F | Adding test frequencies for n79 new CBW 10MHz and 20MHz | 17.7.0 |
| 2022-12 | RAN#98 | R5-227703 | 2602 | 1 | F | Update to IE Table 4.6.3-62A - HighSpeedConfig | 17.7.0 |
| 2022-12 | RAN#98 | R5-227639 | 2603 | 2 | F | Update IE ServingCellConfigCommon | 17.7.0 |
| 2022-12 | RAN#98 | R5-227705 | 2604 | 1 | F | Update IE ServingCellConfigCommonSIB | 17.7.0 |
| 2022-12 | RAN#98 | R5-227706 | 2673 | 1 | F | Disabling of PUCCH intra-slot frequency hopping for RF | 17.7.0 |
| 2022-12 | RAN#98 | R5-227840 | 2565 | 1 | F | Introduction of test frequencies for signalling testing for new NR bands n91, n92, n93 and n94 | 17.7.0 |
| 2022-12 | RAN#98 | R5-227892 | 2675 | 1 | F | Updates to determination of test frequencies for NR Intra-band Non-Contiguous CA | 17.7.0 |
| 2023-03 | RAN#99 | R5-230065 | 2680 | - | F | Introduction of test channel bandwidths for new NR bands n100, n101 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230066 | 2681 | - | F | Introduction of test frequencies for new NR bands n100, n101 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230067 | 2682 | - | F | Introduction of test frequencies for signalling testing for new NR bands n100, n101 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230093 | 2683 | - | F | Addition of test frequencies for new EN-DC comb within FR1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230103 | 2684 | - | F | Updates to clause 4.5B.2 for RedCap test environment | 17.8.0 |
| 2023-03 | RAN#99 | R5-230231 | 2692 | - | F | Update IEs SIB16, CellReselectionPriority, FreqPriorityListSlicing, NSAG-ID and NSAG-IdentityInfo | 17.8.0 |
| 2023-03 | RAN#99 | R5-230279 | 2696 | - | F | Corrections to Clause 6.2.3.7 Test frequencies for NR sidelink configurations for signalling testing | 17.8.0 |
| 2023-03 | RAN#99 | R5-230287 | 2697 | - | F | Update inter-band NR CA configurations of three bands CA_n2A-n5A-n77A, CA_n2A-n66A-n77A, and CA_n5A-n66A-n77A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230457 | 2700 | - | F | Correction to default configuration of RRC IEs for RedCap | 17.8.0 |
| 2023-03 | RAN#99 | R5-230587 | 2703 | - | F | Update IE BWP-UplinkDedicated | 17.8.0 |
| 2023-03 | RAN#99 | R5-230588 | 2704 | - | F | Update IE LBT-FailureRecoveryConfig | 17.8.0 |
| 2023-03 | RAN#99 | R5-230601 | 2705 | - | F | update default message contents of ReportConfigInterRAT | 17.8.0 |
| 2023-03 | RAN#99 | R5-230675 | 2713 | - | F | Correction to Test Procedures for Switch off/Power off | 17.8.0 |
| 2023-03 | RAN#99 | R5-230753 | 2719 | - | F | Introduction or test frequencies for n46 and n96 in clause 6.2.3.1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230754 | 2720 | - | F | Corrections to Annex C for test frequency calculations | 17.8.0 |
| 2023-03 | RAN#99 | R5-230755 | 2721 | - | F | Update IE DownlinkConfigCommonSIB | 17.8.0 |
| 2023-03 | RAN#99 | R5-230882 | 2723 | - | F | NTN test channel bandwidths for n256 and n255 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230884 | 2724 | - | F | NR NTN test frequencies for n256 | 17.8.0 |
| 2023-03 | RAN#99 | R5-230897 | 2726 | - | F | Addition of test frequency for DC_71A_n2A | 17.8.0 |
| 2023-03 | RAN#99 | R5-230966 | 2728 | - | F | Correction to PUCCH secondHopPRB for RF condition | 17.8.0 |
| 2023-03 | RAN#99 | R5-230999 | 2729 | - | F | Update IE BWP-UplinkCommon | 17.8.0 |
| 2023-03 | RAN#99 | R5-231002 | 2731 | - | F | Corrections to RRC Reconfiguration for SCell addition | 17.8.0 |
| 2023-03 | RAN#99 | R5-231023 | 2733 | - | F | Addition of default RRC message configuration for measurement gap enhancements | 17.8.0 |
| 2023-03 | RAN#99 | R5-231079 | 2735 | - | F | Updating test frequencies for n79 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231185 | 2736 | - | F | Correction to PDU SESSION ESTABLISHMENT ACCEPT message | 17.8.0 |
| 2023-03 | RAN#99 | R5-231218 | 2737 | - | F | Add condition to activate dedicated BWP to ServingCellConfig | 17.8.0 |
| 2023-03 | RAN#99 | R5-231219 | 2738 | - | F | Addition of scheduling information for high accuracy GNSS posSibTypes | 17.8.0 |
| 2023-03 | RAN#99 | R5-231227 | 2739 | - | F | Introduction of CA_n41A-n71A configuration. | 17.8.0 |
| 2023-03 | RAN#99 | R5-231236 | 2744 | - | F | Update IE NPN-IdentityInfoList | 17.8.0 |
| 2023-03 | RAN#99 | R5-231243 | 2747 | - | F | Test frequencies update for bands n8 and n25 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231399 | 2722 | 1 | F | Add IEs PathlossReferenceRS and PathlossReferenceRS-Id | 17.8.0 |
| 2023-03 | RAN#99 | R5-231423 | 2701 | 1 | F | Correction to PHY parameters for SL mode 1 transmission | 17.8.0 |
| 2023-03 | RAN#99 | R5-231424 | 2702 | 1 | F | Correction to RRC IEs for SL mode 1 transmission | 17.8.0 |
| 2023-03 | RAN#99 | R5-231447 | 2695 | 1 | F | Addition of CG SDT Configuration message contents for 3GPP SDT | 17.8.0 |
| 2023-03 | RAN#99 | R5-231448 | 2727 | 1 | F | Update of the contents of RRC messages for L2 U2N relay related operation | 17.8.0 |
| 2023-03 | RAN#99 | R5-231453 | 2750 | 1 | F | Adding default contents for SIB17 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231461 | 2716 | 1 | F | Addition of System information combination for Rel-17 eNPN | 17.8.0 |
| 2023-03 | RAN#99 | R5-231467 | 2707 | 1 | F | Addition of Procedure for MBS Multicast session release | 17.8.0 |
| 2023-03 | RAN#99 | R5-231468 | 2708 | 1 | F | Update of Contents of Paging for Multicast MBS TC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231469 | 2709 | 1 | F | Correction of CLOSE UE TEST LOOP message for Loop Mode C | 17.8.0 |
| 2023-03 | RAN#99 | R5-231470 | 2710 | 1 | F | Correction of PDCP-Config for MBS TC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231471 | 2711 | 1 | F | Correction of RadioBearerConfig for MBS TC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231472 | 2712 | 1 | F | Correction of CellGroupConfig for MBS TC | 17.8.0 |
| 2023-03 | RAN#99 | R5-231560 | 2715 | 1 | F | Updates to SIB1 and SIB18 for Rel-17 Enpn | 17.8.0 |
| 2023-03 | RAN#99 | R5-231570 | 2690 | 1 | F | Update IE SIB2 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231571 | 2706 | 1 | F | update default message contents of MeasResults | 17.8.0 |
| 2023-03 | RAN#99 | R5-231577 | 2691 | 1 | F | Update IEs SIB11, ARFCN-ValueEUTRA, MeasIdleConfig and EUTRA-PhysCellIdRange | 17.8.0 |
| 2023-03 | RAN#99 | R5-231603 | 2717 | 1 | F | Correction of test frequencies for n46 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231604 | 2718 | 1 | F | Correction of test frequencies for n96 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231634 | 2694 | 1 | F | Introduction of CA_n41A-n66A configuration. | 17.8.0 |
| 2023-03 | RAN#99 | R5-231792 | 2686 | 1 | F | Update of Propagation Delay Compensation tables for UE Rx-Tx measurements | 17.8.0 |
| 2023-03 | RAN#99 | R5-231793 | 2725 | 1 | F | Addition of test frequencies for R16 combos | 17.8.0 |
| 2023-03 | RAN#99 | R5-231867 | 2693 | 1 | F | Correction to high range reference test frequency for n66 DL CA | 17.8.0 |
| 2023-03 | RAN#99 | R5-231874 | 2698 | 1 | F | Addition of test frequencies for new 3CC EN-DC comb within FR1 | 17.8.0 |
| 2023-03 | RAN#99 | R5-231902 | 2751 | 1 | F | Correction to introduce search space configuration changes for DCI_2-6 transmission | 17.8.0 |
| 2023-06 | RAN#100 | R5-232056 | 2752 | - | F | Correction to test procedure 4.9.9 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232337 | 2756 | - | F | Addition of test frequencies for new 3CC EN-DC comb within FR2 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232338 | 2757 | - | F | Addition of test frequencies for new EN-DC comb within FR2 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232350 | 2758 | - | F | RF message exceptions for K1 and number of HARQ processes in CA | 17.9.0 |
| 2023-06 | RAN#100 | R5-232441 | 2762 | - | F | Introduction of test channel bandwidths for new NR band n13 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232442 | 2763 | - | F | Introduction of test frequencies for new NR band n13 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232443 | 2764 | - | F | Introduction of test frequencies for signalling testing for new NR band n13 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232497 | 2765 | - | F | Correction to default configuration of RRC IEs for NR cov enh test | 17.9.0 |
| 2023-06 | RAN#100 | R5-232520 | 2766 | - | F | NR NTN test frequencies for n255 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232576 | 2767 | - | F | Correction to RRC IEs for NR sidelink test | 17.9.0 |
| 2023-06 | RAN#100 | R5-232654 | 2770 | - | F | Update of NR inter-band CA configurations in FR1 for CA_n3A-n8A | 17.9.0 |
| 2023-06 | RAN#100 | R5-232693 | 2774 | - | F | Addition of NTN freq bands to clause 6.2.3 for Default test frequencies | 17.9.0 |
| 2023-06 | RAN#100 | R5-232712 | 2776 | - | F | Correction NZP-CSI-RS-ResourceSet for FR1 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232723 | 2781 | - | F | Correction to default P-Max value for Power Class 1.5 UEs | 17.9.0 |
| 2023-06 | RAN#100 | R5-232743 | 2782 | - | F | Update of the contents of RRC messages for L2 U2N relay related operation | 17.9.0 |
| 2023-06 | RAN#100 | R5-232746 | 2783 | - | F | Update of the contents of Sidelink information elements | 17.9.0 |
| 2023-06 | RAN#100 | R5-232790 | 2786 | - | F | Addition of test frequencies of CA_n39A-n41A config TC 4.3.1.1.2.1 | 17.9.0 |
| 2023-06 | RAN#100 | R5-232877 | 2787 | - | F | Addition of new CA configuration CA-n41A-n66A-n71A | 17.9.0 |
| 2023-06 | RAN#100 | R5-232934 | 2789 | - | F | Update CSI-ReportConfig IE content for RRM testing | 17.9.0 |
| 2023-06 | RAN#100 | R5-232946 | 2793 | - | F | Delete NR-19 for MBS in the Common configurations of system information blocks | 17.9.0 |
| 2023-06 | RAN#100 | R5-232970 | 2794 | - | F | Update Service accept NAS message | 17.9.0 |
| 2023-06 | RAN#100 | R5-232971 | 2795 | - | F | Update RadioBearerConfig-SRB2-DRB message | 17.9.0 |
| 2023-06 | RAN#100 | R5-232991 | 2797 | - | F | Correction to RRM PDCCH TCI-State | 17.9.0 |
| 2023-06 | RAN#100 | R5-232992 | 2798 | - | F | Correction to RRM TRS CSI-ResourceConfig | 17.9.0 |
| 2023-06 | RAN#100 | R5-233091 | 2799 | - | F | Adding uplink CA test frequencies for CA_n77(2A) | 17.9.0 |
| 2023-06 | RAN#100 | R5-233135 | 2804 | - | F | Correction to RF or RRM condition for default messages | 17.9.0 |
| 2023-06 | RAN#100 | R5-233142 | 2805 | - | F | Update to test procedure for registration of a MUSIM UE | 17.9.0 |
| 2023-06 | RAN#100 | R5-233195 | 2806 | - | F | Corrections to Test frequencies for NR CA configurations for signalling testing | 17.9.0 |
| 2023-06 | RAN#100 | R5-233317 | 2769 | 1 | F | Correction to introduce search space configuration changes for DCI_2-6 transmission | 17.9.0 |
| 2023-06 | RAN#100 | R5-233369 | 2775 | 1 | F | Addition of new clause for UE Position Requirements for NR NTN testing | 17.9.0 |
| 2023-06 | RAN#100 | R5-233379 | 2785 | 1 | F | Addition of a new combination of system information block for SIB16 | 17.9.0 |
| 2023-06 | RAN#100 | R5-233382 | 2791 | 1 | F | Addition of Procedure to check TMGI and associated MRB reception in a multicast MBS session | 17.9.0 |
| 2023-06 | RAN#100 | R5-233383 | 2792 | 1 | F | Update PDCCH-Config for MSS condition | 17.9.0 |
| 2023-06 | RAN#100 | R5-233406 | 2790 | 1 | F | Update general configuration parameter for HD-FDD UE | 17.9.0 |
| 2023-06 | RAN#100 | R5-233423 | 2777 | 1 | F | Addition of PC5 RRC message uuMessageTransferSidelink | 17.9.0 |
| 2023-06 | RAN#100 | R5-233424 | 2778 | 1 | F | Addition of PC5 RRC RemoteUEInformationSidelink message | 17.9.0 |
| 2023-06 | RAN#100 | R5-233425 | 2780 | 1 | F | Update of PC5 RRC message Uu-RelayRLC-ChannelConfig | 17.9.0 |
| 2023-06 | RAN#100 | R5-233426 | 2779 | 1 | F | Update of PC5 RRC message SL-L2RelayUE-Config | 17.9.0 |
| 2023-06 | RAN#100 | R5-233429 | 2755 | 1 | F | Add generic procedure for Switch off / Power off procedure in MA PDU session Established on NR and WLAN | 17.9.0 |
| 2023-06 | RAN#100 | R5-233434 | 2771 | 1 | F | Updates to SIB19 for NR NTN | 17.9.0 |
| 2023-06 | RAN#100 | R5-233435 | 2772 | 1 | F | Update IE ServingCellConfigCommon for NR NTN | 17.9.0 |
| 2023-06 | RAN#100 | R5-233436 | 2773 | 1 | F | Update to clause 4.4.3 Common parameters for NR NTN | 17.9.0 |
| 2023-06 | RAN#100 | R5-233474 | 2768 | 1 | F | Correction to switch off test procedure | 17.9.0 |
| 2023-06 | RAN#100 | R5-233500 | 2808 | 1 | F | Correction of 38.508-1 4.1.1 on lower humidity limit in temperature test environment | 17.9.0 |
| 2023-06 | RAN#100 | R5-233674 | 2754 | 1 | F | Addition and update of tables for HST FR2 scenario | 17.9.0 |
| 2023-06 | RAN#100 | R5-233675 | 2807 | 1 | F | Update to TS 38.508-1 clause 4.6.3-200BB for FR2 UL Gaps IE | 17.9.0 |
| 2023-06 | RAN#100 | R5-233676 | 2809 | - | F | Correction to frequency range for ssb-PositionsInBurst and SSB-ToMeasure | 17.9.0 |
| 2023-06 | RAN#100 | R5-233699 | 2759 | 1 | F | Correction of test frequency parameters for n79 | 17.9.0 |
| 2023-06 | RAN#100 | R5-233700 | 2802 | 1 | F | Updating frequency calculation in Annex C.3.2 | 17.9.0 |
| 2023-06 | RAN#100 | R5-233734 | 2788 | 2 | F | Addition of test frequency for new 3/4 band EN-DC comb | 17.9.0 |
| 2023-09 | RAN#101 | R5-233931 | 2812 | - | F | Clarify default ASN.1 contents | 17.10.0 |
| 2023-09 | RAN#101 | R5-233932 | 2813 | - | F | Update IE MAC-CellGroupConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-233944 | 2814 | - | F | Introduction of Inter-band NR CA configuration CA_n20A-n78A | 17.10.0 |
| 2023-09 | RAN#101 | R5-233987 | 2815 | - | F | Adding 5GS registration type for Rel-17 eNPN onboarding service. | 17.10.0 |
| 2023-09 | RAN#101 | R5-234038 | 2817 | - | F | Update IE PUSCH-TimeDomainResourceAllocationList | 17.10.0 |
| 2023-09 | RAN#101 | R5-234056 | 2819 | - | F | Add DC_48A_n46A configuration to 38.508-1 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234058 | 2820 | - | F | Update IE PDSCH-Config | 17.10.0 |
| 2023-09 | RAN#101 | R5-234076 | 2821 | - | F | Addition of test frequencies for new EN-DC combos within FR2 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234145 | 2826 | - | F | Update LoggedMeasurementConfiguration | 17.10.0 |
| 2023-09 | RAN#101 | R5-234146 | 2827 | - | F | Update IE CellGroupConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234147 | 2828 | - | F | Update IE ConfiguredGrantConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234148 | 2829 | - | F | Update IE ControlResourceSet | 17.10.0 |
| 2023-09 | RAN#101 | R5-234149 | 2830 | - | F | Update IE CSI-ReportConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234159 | 2831 | - | F | Test frequencies for band n70 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-234160 | 2832 | - | F | Test frequency corrections for n77 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234161 | 2833 | - | F | Low and High test channel BW correction in 38.508-1 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234162 | 2834 | - | F | Test frequency corrections for n66 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234191 | 2835 | - | F | Relative Tx Power Allocation to PRS | 17.10.0 |
| 2023-09 | RAN#101 | R5-234295 | 2837 | - | F | Addition of test frequencies for new NRCA comb within FR1 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234303 | 2838 | - | F | Update IE LogicalChannelConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234305 | 2840 | - | F | Update IE RadioBearerConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234306 | 2841 | - | F | Update IE RLC-BearerConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234307 | 2842 | - | F | Update IE RLC-Config | 17.10.0 |
| 2023-09 | RAN#101 | R5-234308 | 2843 | - | F | Update IE SRB-Identity | 17.10.0 |
| 2023-09 | RAN#101 | R5-234310 | 2844 | - | F | Add radio configuration for RRCReconfiguration and SRB4 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234447 | 2849 | - | F | Addition of several NR CA combination to inter-band configurations table | 17.10.0 |
| 2023-09 | RAN#101 | R5-234457 | 2850 | - | F | Correction of DCI 4-0 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234459 | 2852 | - | F | Update PDSCH-ConfigBroadcast-r17 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234476 | 2854 | - | F | Correction of DCI 1-0 DCI 1-1 and DCI 1-2 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234513 | 2856 | - | F | Update IE PDCP-Config | 17.10.0 |
| 2023-09 | RAN#101 | R5-234514 | 2857 | - | F | Update IE PDSCH-ServingCellConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234515 | 2858 | - | F | Update IE PUCCH-Config | 17.10.0 |
| 2023-09 | RAN#101 | R5-234516 | 2859 | - | F | Update IE PUSCH-Config | 17.10.0 |
| 2023-09 | RAN#101 | R5-234520 | 2860 | - | F | Update IE PUSCH-ServingCellConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234521 | 2861 | - | F | Update IE RACH-ConfigCommon | 17.10.0 |
| 2023-09 | RAN#101 | R5-234522 | 2862 | - | F | Update IE RACH-ConfigDedicated | 17.10.0 |
| 2023-09 | RAN#101 | R5-234523 | 2863 | - | F | Update IE ReportConfigInterRAT | 17.10.0 |
| 2023-09 | RAN#101 | R5-234527 | 2864 | - | F | Update IE SCS-SpecificCarrier | 17.10.0 |
| 2023-09 | RAN#101 | R5-234528 | 2865 | - | F | Update IE ServingCellConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-234529 | 2866 | - | F | Update IE ServingCellConfigCommon | 17.10.0 |
| 2023-09 | RAN#101 | R5-234530 | 2867 | - | F | Update IE SRS-Config | 17.10.0 |
| 2023-09 | RAN#101 | R5-234538 | 2868 | - | F | Addition of test freqs for CA_n258E. | 17.10.0 |
| 2023-09 | RAN#101 | R5-234539 | 2869 | - | F | Addition of test freqs for CA_n258F. | 17.10.0 |
| 2023-09 | RAN#101 | R5-234540 | 2870 | - | F | Addition of test freqs for CA_n258I. | 17.10.0 |
| 2023-09 | RAN#101 | R5-234541 | 2871 | - | F | Addition of test freqs for CA_n258J. | 17.10.0 |
| 2023-09 | RAN#101 | R5-234542 | 2872 | - | F | Addition of test freqs for CA_n258K. | 17.10.0 |
| 2023-09 | RAN#101 | R5-234543 | 2873 | - | F | Addition of test freqs for CA_n258L. | 17.10.0 |
| 2023-09 | RAN#101 | R5-234544 | 2874 | - | F | Addition of test freqs for CA_n258M. | 17.10.0 |
| 2023-09 | RAN#101 | R5-234689 | 2879 | - | F | Addition of test frequencies for new EN-DC configurations within FR1 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234731 | 2885 | - | F | Introduction of test frequency of CA_n28A-n41A-n79A | 17.10.0 |
| 2023-09 | RAN#101 | R5-234905 | 2893 | - | F | Correction to frequency range for ssb-PositionsInBurst and SSB-ToMeasure | 17.10.0 |
| 2023-09 | RAN#101 | R5-234906 | 2894 | - | F | Correction to default value of csi-SSB-ResourceToAddModList for RRM test cases | 17.10.0 |
| 2023-09 | RAN#101 | R5-234935 | 2895 | - | F | Annex C Test frequencies calculations corrections | 17.10.0 |
| 2023-09 | RAN#101 | R5-234936 | 2896 | - | F | Test frequencies for band n2: Addition of 35 MHz channel bandwidth | 17.10.0 |
| 2023-09 | RAN#101 | R5-234941 | 2899 | - | F | Test frequencies for band n5 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-234942 | 2900 | - | F | Test frequencies for band n8 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-234944 | 2902 | - | F | Test frequencies for band n25 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-234947 | 2905 | - | F | Test frequencies for band n71 and asymmetric channel bandwidths in Rel-15 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234948 | 2906 | - | F | Test frequencies for band n71 and asymmetric channel bandwidths in Rel-17 | 17.10.0 |
| 2023-09 | RAN#101 | R5-234949 | 2907 | - | F | Test frequencies for band n91 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-234950 | 2908 | - | F | Test frequencies for band n92 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-234951 | 2909 | - | F | Test frequencies for band n93 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-234952 | 2910 | - | F | Test frequencies for band n94 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-235084 | 2911 | - | F | Updates on Dedicated coreset configuration for RF and RRM testing | 17.10.0 |
| 2023-09 | RAN#101 | R5-235249 | 2922 | - | F | Update MIB for NCD-SSB | 17.10.0 |
| 2023-09 | RAN#101 | R5-235277 | 2825 | 1 | F | Correct Editorial mistake in default message contents of DLInformationTransfer | 17.10.0 |
| 2023-09 | RAN#101 | R5-235279 | 2855 | 1 | F | Update SchedulingRequestResourceConfig | 17.10.0 |
| 2023-09 | RAN#101 | R5-235280 | 2886 | 1 | F | Updates to default 5GMM messages | 17.10.0 |
| 2023-09 | RAN#101 | R5-235306 | 2884 | 1 | F | Correction to test procedure 4.9.31_SL DRB connectivity check | 17.10.0 |
| 2023-09 | RAN#101 | R5-235313 | 2851 | 1 | F | Correction of DCI 4-2 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235336 | 2810 | 1 | F | Addition of NTN freq band n255 to clause 6.2.3 for Default test frequencies | 17.10.0 |
| 2023-09 | RAN#101 | R5-235349 | 2822 | 1 | F | Update default message contents of RRCReconfiguration for MDT enhance | 17.10.0 |
| 2023-09 | RAN#101 | R5-235350 | 2823 | 1 | F | Update default message contents of UEInformationRequest for MDT enhance | 17.10.0 |
| 2023-09 | RAN#101 | R5-235369 | 2875 | 1 | F | Addition of default ProSe PC5 signalling messages | 17.10.0 |
| 2023-09 | RAN#101 | R5-235370 | 2876 | 1 | F | Addition of spec reference of 24.554 for ProSe PC5 signalling messages | 17.10.0 |
| 2023-09 | RAN#101 | R5-235371 | 2881 | 1 | F | Addition of test procedure for establishing unicast mode ProSe Direct communication / Peer UE side | 17.10.0 |
| 2023-09 | RAN#101 | R5-235372 | 2882 | 1 | F | Addition of test procedure for registration of NR sidelink U2N Relay / Relay UE side | 17.10.0 |
| 2023-09 | RAN#101 | R5-235373 | 2883 | 1 | F | Addition of Generic procedures for NR sidelink U2N Relay | 17.10.0 |
| 2023-09 | RAN#101 | R5-235375 | 2912 | 1 | F | Updating PDCCH-ConfigCommon content | 17.10.0 |
| 2023-09 | RAN#101 | R5-235376 | 2839 | 1 | F | Update IE LogicalChannelIdentity | 17.10.0 |
| 2023-09 | RAN#101 | R5-235377 | 2845 | 1 | F | Add radio configuration for RadioBearerConfig and SRB4 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235378 | 2846 | 1 | F | Add radio configuration for CellGroupConfig and SRB4 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235391 | 2880 | 1 | F | Addition of test procedure for establishing unicast mode ProSe Direct communication / Initiating UE side | 17.10.0 |
| 2023-09 | RAN#101 | R5-235414 | 2887 | 1 | F | Addition of signaling test frequency for CA_n77(2A) | 17.10.0 |
| 2023-09 | RAN#101 | R5-235415 | 2853 | 1 | F | Add si-SchedulingInfo-v1700 for the TC scheduling SIB15 to SIB21 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235600 | 2901 | 1 | F | Test frequencies for band n24 and asymmetric channel bandwidths | 17.10.0 |
| 2023-09 | RAN#101 | R5-235601 | 2897 | 1 | F | Test frequencies for band n18 | 17.10.0 |
| 2023-09 | RAN#101 | R5-235602 | 2836 | 1 | F | Update inter-band NR CA 3DL configurations with additional band combos | 17.10.0 |
| 2023-09 | RAN#101 | R5-235603 | 2921 | 1 | F | Addition of CA_n2(2A)-n66A frequency | 17.10.0 |
| 2023-09 | RAN#101 | R5-235604 | 2916 | 1 | F | Rel-16 Corrections to Table 4.3.1.0A-1 (mid test channel bandwidths) for unlicensed bands | 17.10.0 |
| 2023-09 | RAN#101 | R5-235605 | 2918 | 1 | F | Rel-17 Corrections to Table 4.3.1.0A-1 (mid test channel bandwidths) | 17.10.0 |
| 2023-09 | RAN#101 | R5-235606 | 2913 | 1 | F | Addition of connection diagram for FR2 spurious emissions | 17.10.0 |
| 2023-09 | RAN#101 | R5-235780 | 2889 | 1 | F | Addition of default message for unified TCI state | 17.10.0 |
| 2023-09 | RAN#101 | R5-235781 | 2891 | 1 | F | Correction to SIB4 redCapAccessAllowed-r17 for RedCap testing | 17.10.0 |
| 2023-09 | RAN#101 | R5-235782 | 2892 | 1 | F | Correction to c-SRS default value for FR2 multiple BWP case | 17.10.0 |
| 2023-09 | RAN#101 | R5-235822 | 2917 | 1 | F | Rel-16 Corrections to Table 4.3.1.0A-1 (mid test channel bandwidths) for sidelink | 17.10.0 |
| 2023-09 | RAN#101 | R5-235823 | 2919 | 1 | F | Correction to Table 4.3.1.0A-1 (mid test channel bandwidths) for RedCap Ues | 17.10.0 |
| 2023-09 | RAN#101 | R5-235841 | 2890 | 1 | F | Correction to notes for RedCap CBW | 17.10.0 |
| 2023-09 | RAN#101 | R5-235843 | 2914 | 1 | F | Rel-15 Corrections to Table 4.3.1.0A-1 (mid test channel bandwidths) | 17.10.0 |
| 2023-09 | RAN#101 | R5-235844 | 2915 | 1 | F | Rel-16 Corrections to Table 4.3.1.0A-1 (mid test channel bandwidths) | 17.10.0 |
| 2023-09 | RAN#101 | R5-235460 | 2920 | 2 | F | Common test environments for NR-NTN testing | 17.10.0 |