# 6	NR standalone in FR1

This clause contains test scenarios for NR standalone. This configuration is also known as SA Option 2. All NR cells are in Frequency Range 1.

## 6.1	RRC_IDLE state mobility

### 6.1.1	NR cell re-selection

#### 6.1.1.0	Minimum conformance requirements

##### 6.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection

The cell re-selection delay shall be less than Tevaluate NR_Intra + TSI-NR in RRC_IDLE state.

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [30] within Tdetect,NR_Intra as defined in table 4.2.2.3-1 of TS 38.133 [6] when that Treselection= 0. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 of TS 38.133 [6] for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1 of TS 38.133 [6]) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1 of TS 38.133 [6] provided that the cell has at least [3]dB better ranked.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non-zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [30], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.2 and 4.2.2.3.

##### 6.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection

The cell re-selection delay shall be less than Tevaluate NR_Intra + TSI-NR in RRC_IDLE state.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 of TS 38.133 [6].

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [30] within Kcarrier * Tdetect,NR_Inter if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least [5] dB for reselections based on ranking or [6]dB for SS-RSRP reselections based on absolute priorities or [4]dB for SS-RSRQ reselections based on absolute priorities. The parameter Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.3 of TS 38.133 [6] for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter (see table 4.2.2.4-1 of TS 38.133 [6]) for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [30] within Kcarrier * Tevaluate,NR_Inter when Treselection = 0 as specified in table 4.2.2.4-1 of TS 38.133 [6] provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and the cell has at least [5]dB better ranked

-	[6]dB for SS-RSRP reselections based on absolute priorities or

-	[4]dB for SS-RSRQ reselections based on absolute priorities.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non-zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 of TS 38.133 [6] under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively, and

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to TBD ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to TBD ms before the start or up to TBD ms after the end of the paging occasion [1].

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.4.

##### 6.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with highSpeedMeasFlag-r16

The cell re-selection delay shall be less than Tevaluate NR_Intra + TSI-NR in RRC_IDLE state.

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [30] within Tdetect,NR_Intra as defined in table 4.2.2.3-2 of TS 38.133 [6] when that Treselection= 0. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 of TS 38.133 [6] for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-2 of TS 38.133 [6]) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-2 of TS 38.133 [6] provided that the cell has at least 3dB better ranked.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non-zero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [30], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.2 and 4.2.2.3.

#### 6.1.1.1	NR SA FR1 cell re-selection

6.1.1.1.1	Test purpose

The purpose of this test is to verify that when the current and target cell operates on the same carrier frequency the UE is able to search and measure cells to meet the intra-frequency NR cell re-selection requirements.

6.1.1.1.2	Test applicability

This test applies to all types of NR UE from release 15 onwards.

6.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.1.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.1.1.1.

6.1.1.1.4	Test description

6.1.1.1.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 6.1.1.1.4.1-1.

Table 6.1.1.1.4.1-1: Supported test configurations for NR SA FR1 cell re-selection

| Configuration | Description |
| --- | --- |
| 6.1.1.1-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.1.1.1-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.1.1.1-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.1.1.1.4.1-2.

Table 6.1.1.1.4.1-2: Initial conditions for NR SA FR1 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.1.1.1.4.1-3.

2. Message contents are defined in clause 6.1.1.1.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 6.1.1.1.4.1-3: General test parameters for NR SA FR1 cell re-selection

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

6.1.1.1.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5. Set Cell 2 physical cell identity = initial cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 6.1.1.1.5-1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.1.1.1.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 34 seconds from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 8.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.

7a	The SS shall send an RRCRelease to ensure that the UE is in state RRC_IDLE.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 6.1.1.1.5-1. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 8 seconds from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, skip to step 13.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5 in Cell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

6.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.1.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-1 with Condition SMTC.2 and Asynchronous cells for configuration 6.1.1.1-1 and Cell 1 Table H.2.1-1 with Condition SMTC.6 and Asynchronous cells for configuration 6.1.1.1-1 and Cell 2 Table H.2.1-1 with Condition SMTC.1 and synchronous cells for configuration 6.1.1.1-2 Table H.2.1-1 with Condition SMTC.1 and synchronous cells for configuration 6.1.1.1-3 Table H.2.1-2 Table H.2.1-3 |
| Default RRC messages and information elements contents exceptions |  |

6.1.1.1.5	Test requirement

Tables 6.1.1.1.4.1-3 and 6.1.1.1.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case.

Table 6.1.1.1.5-1: Cell specific test parameters for NR SA FR1 cell re-selection

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -130 |  |  | -130 |  |  |
|  |  | 3 | -127 |  |  | -127 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 16 | -3.55 | 3.24 | -infinity | 3.24 | -3.55 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16.45 | -infinity | 16.45 | 13 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -81.55 | -infinity | -81.55 | -85 |
|  |  | 2 | -82 | -85 | -81.55 | -infinity | -81.55 | -85 |
|  |  | 3 | -79 | -82 | -78.54 | -infinity | -78.54 | -82 |
| Io | dBm/9.36 MHz | 1 | -53.94 | -51.91 | -51.91 | Specified in Cell 1 columns |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -51.91 | -51.91 |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85 | -45.81 | -45.81 |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  |  | 60 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The cell re-selection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell test requirement in this case is expressed as:

Cell re-selection delay to a newly detectable cell = Tdetect,NR_Intra + TSI-NR

Tdetect,NR_Intra = 32 s; as specified in TS 38.133 [6] clause 4.2.2.3.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a newly detectable cell shall be less than a total of 33.28 seconds in this test case (note: this gives a total of 33.28 seconds but the test allows 34 seconds).

The cell re-selection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 1.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 6.4 s; as specified in TS 38.133 [6] clause 4.2.2.3.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to an already detected cell shall be less than a total of 7.68 seconds in this test case (note: this gives a total of 7.68 seconds but the test allows 8 seconds).

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 6.1.1.2	NR SA FR1-FR1 cell re-selection

6.1.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection.

6.1.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.1.1.2.

6.1.1.2.4	Test description

6.1.1.2.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 6.1.1.2.4.1-1.

Table 6.1.1.2.4.1-1: Supported test configurations for NR SA FR1-FR1 cell re-selection

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 6.1.1.2-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.1.1.2-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.1.1.2-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 6.1.1.2.4.1-2.

Table 6.1.1.2.4.1-2: Initial conditions for NR SA FR1-FR1 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.1.1.2.4.1-3.

2. Message contents are defined in clause 6.1.1.2.4.3.

3. There is two NR carrier and 2 NR Cells specified in the test. Cell 2 is the PCell and Cell 1 is the neighbour cell in a different carrier than cell 2. Cell 1 and Cell 2 are configured according to Annex C.1.2.

Table 6.1.1.2.4.1-3: General test parameters for NR SA FR1-FR1 cell re-selection

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2, 3 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC 2 | Configured in SIB4 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB4 of Cell 2 |
|  |  |  | 2 | SMTC 1 |  |
|  |  |  | 3 | SMTC 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

6.1.1.2.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.1.1.2.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, Cell 1.

4.	If the UE responds on lower priority cell, Cell 1 during time duration T1 within 8 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”.

5.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 11.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 6.1.1.2.5-1. T2 starts. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE.

7.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 6.1.1.2.5-1. T3 starts.

7a	The SS shall send an RRCRelease to ensure that the UE is in state RRC_IDLE.

8.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, Cell 2.

9.	If the UE responds on higher priority cell, Cell 2 during time duration T3 within 68 seconds from the beginning of time period T3, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”.

10.	If the UE has re-selected Cell 2 within T3, after the re-selection or when T3 expires, skip to step 12.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE.

11.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5 in Cell 2.

12.	Repeat step 3-11 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

6.1.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.1.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 6.1.1.2-1 and Cell 1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for configuration 6.1.1.2-1 and Cell 2 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.1.1.2-2 Table H.2.2-2 with Condition SSB.2 FR1, SMTC.1 and synchronous cells for configuration 6.1.1.2-3 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions |  |

6.1.1.2.5	Test requirement

Tables 6.1.1.2.4.1-3 and 6.1.1.2.5-1 define the primary level settings including test tolerances for inter frequency NR cell re-selection test case.

Table 6.1.1.2.5-1: Cell specific test parameters for NR SA FR1-FR1 cell re-selection

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 15.6 | 15.6 | 15.6 | -3.6 | -infinity | 13.6 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  | -100 | -98 | -98 |
|  |  | 2 | -98 |  |  | -100 | -98 | -98 |
|  |  | 3 | -95 |  |  | -97 | -95 | -95 |
| Note2 | dBm/15 kHz | 1 | -98 |  |  | -100 | -98 | -98 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 15.6 | 15.6 | 15.6 | -3.6 | -infinity | 13.6 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82.4 | -82.4 | -82.4 | -103.6 | -infinity | -84.4 |
|  |  | 2 | -82.4 | -82.4 | -82.4 | -103.6 | -infinity | -84.4 |
|  |  | 3 | -79.39 | -79.39 | -79.39 | -100.59 | -infinity | -81.39 |
| Io | dBm/9.36 MHz | 1 | -54.33 | -54.33 | -54.33 | -70.46 | -infinity | -56.26 |
|  | dBm/9.36 MHz | 2 | -54.33 | -54.33 | -54.33 | -70.46 | -infinity | -56.26 |
|  | dBm/38.16 MHz | 3 | -48.23 | -48.23 | -48.23 | -67.37 | -infinity | -53.17 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration updateon cell 1.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_ inter + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2.7 of TS 38.133 [6]

Tevaluate, NR_ inter	See Table 4.2.2.4-1 in clause 4.2.2.4 of TS 38.133 [6]

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 6.1.1.3 to 6.1.1.6

#### 6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

6.1.1.7.1	Test purpose

The purpose of this test is to verify that when the current and target cell operates on the same carrier frequency the UE configured with highSpeedMeasFlag-r16 is able to search and measure cells to meet the intra-frequency NR cell re-selection requirements.

6.1.1.7.2	Test applicability

This test applies to all types of NR UE from release 16 onwards that supports measurement enhancements in high speed scenario

6.1.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.1.1.7.

6.1.1.7.4	Test description

6.1.1.7.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 6.1.1.7.4.1-1.

Table 6.1.1.7.4.1-1: Supported test configurations for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 6.1.1.7-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.1.1.7-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.1.1.7-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.1.1.7.4.1-2.

Table 6.1.1.7.4.1-2: Initial conditions for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.1.1.7.4.1-3.

2. Message contents are defined in clause 6.1.1.7.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 6.1.1.7.4.1-3: General test parameters for NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial | Active cell |  | 1, 2, 3 | Cell1 |  |
| condition | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.32 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

6.1.1.7.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. highSpeedMeasFlag-r16 is broadcasted to UE.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5. Set Cell 2 physical cell identity = initial cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 6.1.1.7.5-1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.1.1.7.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 34 seconds from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 8.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 12.

7a	The SS shall send an RRCRelease to ensure that the UE is in state RRC_IDLE.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 6.1.1.7.5-1. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 8 seconds from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, skip to step 13.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5 in Cell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

6.1.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.1.1.7.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks on condition of HighSpeedMeas with exceptions | Table H.2.1-1 with Condition SMTC.2 and Asynchronous cells for configuration 6.1.1.7-1 Table H.2.1-1 with Condition SMTC.1 and synchronous cells for configuration 6.1.1.7-2 Table H.2.1-1 with Condition SMTC.1 and synchronous cells for configuration 6.1.1.7-3 Table H.2.1-2 Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions |  |

6.1.1.7.5	Test requirement

Tables 6.1.1.7.4.1-3 and 6.1.1.7.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16.

Table 6.1.1.7.5-1: Cell specific test parameters for intra frequency NR cell re-selection test case for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 |  | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.2.1-1 |  |  | OP.1 defined in A.2.1-1 |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |  |
|  | dB | 1 | 16+TT | -3.11+ TT | 2.79+TT | -infinity |  | 2.79+TT | -3.11+ TT |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98+TT |  |  |  |  |  |  |
|  |  | 2 | -98+TT |  |  |  |  |  |  |
|  |  | 3 | -95+TT |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98+TT |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
|  | dB | 1 | 16+TT | 13+TT | 16+TT | -infinity |  | 16+TT | 13+TT |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82+TT | -85+TT | -82+TT | -infinity |  | -82+TT | -85+TT |
|  |  | 2 | -82+TT | -85+TT | -82+TT | -infinity |  | -82+TT | -85+TT |
|  |  | 3 | -79+TT | -82+TT | -79+TT | -infinity |  | -79+TT | -82+TT |
| Io | dBm/9.36 MHz | 1 | -53.94+ TT | -52.21+ TT | -52.21+ TT | specified in Cell 1 columns- |  |  |  |
|  | dBm/9.36 MHz | 2 | -53.94+ TT | -52.21+ TT | -52.21+ TT |  |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85+ TT | -46.12+ TT | -46.12+ TT |  |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |  | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | N50 |  |  | N50 |  |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  | AWGN 1944Hz Note4 |  |  |
| Propagation Condition |  | 3 | AWGN |  |  |  | AWGN 3334Hz Note5 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a newly detectable cell test requirement in this case is expressed as:

Cell re-selection delay to a newly detectable cell = Tdetect,NR_Intra + TSI-NR

Tdetect,NR_Intra = 2.56 s; as specified in Table 4.2.2.3-2 in clause 4.2.2.3 TS 38.133 [6].

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a newly detectable cell shall be less than a total of 3.84 seconds in this test case (note: this gives a total of 3.84 seconds but the test allows 4 seconds).

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 0.96 s; as specified in Table 4.2.2.3-2 in clause 4.2.2.3 TS 38.133 [6].

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to an already detected cell shall be less than a total of 2.24 seconds in this test case (note: this gives a total of 2.24 seconds but the test allows 3 seconds).

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

### 6.1.2	NR – E-UTRA cell re-selection

#### 6.1.2.0	Minimum conformance requirements

##### 6.1.2.0.1	Minimum conformance requirements for NR – E-UTRA cell re-selection

The cell re-selection delay to a higher priority cell shall be less than Thigher_priority_search + Tevaluate, EUTRAN + TSI-E-UTRA in RRC_IDLE state.

The cell re-selection delay to a lower priority E-UTRA cell shall be less than Tevaluate, E-UTRAN + TSI-E-UTRA in RRC_IDLE state.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2 of TS 38.133 [6].

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this section apply for inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this section. The parameter NEUTRA_carrier is the total number of configured E-UTRA carriers in the neighbour frequency list. The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,EUTRAN/2.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band.

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS38.304 [30] within (NEUTRA_carrier) * Tdetect,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

Cells which have been detected shall be measured at least every (NEUTRA_carrier) * Tmeasure,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT E-UTRAN carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [30] within (NEUTRA_carrier) * Tevaluate,EUTRAN when Treselection = 0 as speficied in table 4.2.2.5-1 of TS 38.133 [6] provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

If Treselection timer has a non zero value and the inter-RAT E-UTRA cell is satisfied with the reselection criteria which are defined in TS 38.304 [30], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.5.

#### 6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA

6.1.2.1.1	Test purpose

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2.2.5 of TS 38.133 [6] when the E-UTRAN cell is of higher priority.

6.1.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.1.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.1.2.1.

6.1.2.1.4	Test description

6.1.2.1.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.1.2.1.4.1-1.

Table 6.1.2.1.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 6.1.2.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | LTE 10MHz bandwidth, TDD duplex mode |
| 6.1.2.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | LTE 10MHz bandwidth, TDD duplex mode |
| 6.1.2.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | LTE 10MHz bandwidth, TDD duplex mode |
| 6.1.2.1-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | LTE 10MHz bandwidth, FDD duplex mode |
| 6.1.2.1-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | LTE 10MHz bandwidth, FDD duplex mode |
| 6.1.2.1-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | LTE 10MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equirement and the DUT according to the parameters in Table 6.1.2.1.4.1-2.

Table 6.1.2.1.4.1-2: Initial conditions for NR SA FR1 – E-URTA cell re-selection to higher priority E-UTRA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 6.1.2.1.4.1-3.

2.	Message contents are defined in clause 6.1.2.1.4.3.

3.	The test scenario comprises of one NR cell and one E-UTRAN cell. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 6.1.2.1.4.1-3: General test parameters for NR to higher priority E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase and during T2 period the UE reselects to cell 2. |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T2. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T3 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T3 for iteration of the tests. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | >7 | During T1, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

6.1.2.1.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an E-UTRA neighbour cell (Cell 2). The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. NR cell 1 is already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of higher priority than cell 1.

Before T1 the UE is camped on to cell 1. During T1, cell 2 shall be powered off. At the start of T2 the UE is expected to detect cell 2, send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2. At the start of T3 cell 2 becomes weaker than cell 1, and the UE reselects to Cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.1.2.1.5-1 and 6.1.2.1.5-2. T1 starts.

3.	During T1, Cell 2 shall be powered off and set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.1.2.1.5-1 and 6.1.2.1.5-2. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a higher priority cell, Cell 2.

6.	If the UE responds on Cell 2 during time duration T2 within 68 seconds from the beginning of time period T2, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 8.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 11.

7a	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 6.1.2.1.5-1 and 6.1.2.1.5-2. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to a lower priority cell, Cell 1.

10.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, skip to step 12.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE.

11.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5.

12.	Repeat step 2-11 until a test verdict has been achieved.

6.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.1.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with Condition SMTC 1 and higher priority for configuration 6.1.2.1-2, 6.1.2.1-3, 6.1.2.1-5 and 6.1.2.1-6 Table H.2.3-2 with Condition SMTC 2 and higher priority for configuration 6.1.2.1-1 and 6.1.2.1-4 Table H.2.3-3 with Condition higher priority |
| Default RRC messages and information elements contents exceptions |  |

Table 6.1.2.1.4.3-2: SystemInformationBlockType3

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 22 | Actual value = 44 dB |  |
| cellReselectionPriority | 5 |  | higher priority E-UTRA |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.1.4.3-3: SystemInformationBlockType24

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority-r15[n] | 4 |  | higher priority E-UTRA |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| } |  |  |  |

6.1.2.1.5	Test requirement

Tables 6.1.2.1.4.1-3, 6.1.2.1.5-1 and 6.1.2.1.5-2 define the primary level settings including test tolerances for higher priority E-UTRA cell re-selection test case.

Table 6.1.2.1.5-1: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH parameters |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| Dedicated CORESET parameters |  | 1, 4 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| SSB parameters |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.2 FR1 |  |  |
| NR SMTC parameters |  | 1, 4 | SMTC 2 |  |  |
|  |  | 2, 5 | SMTC 1 |  |  |
|  |  | 3, 6 | SMTC 1 |  |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.2.1-1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
|  | dBm/SCS | 1, 4 | -98 |  |  |
|  |  | 2, 5 | -98 |  |  |
|  |  | 3, 6 | -95 |  |  |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| SS-RSRP | dBm/SCS | 1, 4 | -84 | -82.4 | -82.4 |
|  |  | 2, 5 | -84 | -82.4 | -82.4 |
|  |  | 3, 6 | -81 | -79.39 | -79.39 |
|  | dB | 1, 4 | 14 | 15.6 | 15.6 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4 | 14 | 15.6 | 15.6 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -55.88 | -54.33 | -54.33 |
|  | dBm/9.36 MHz | 2, 5 | -55.88 | -54.33 | -54.33 |
|  | dBm/38.16 MHz | 3, 6 | -49.79 | -48.23 | -48.23 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Threshx, highP (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of threshX-High which is included in NR system information, and is a threshold for the E-UTRA target cell. |  |  |  |  |  |

Table 6.1.2.1.5-2: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 |  | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |  |
| BWchannel | MHz | 10 |  |  |  |
| OCNG Patterns defined in TS 36.133 clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |  |
| PBCH_RA | dB | 0 |  |  |  |
| PBCH_RB | dB |  |  |  |  |
| PSS_RA | dB |  |  |  |  |
| SSS_RA | dB |  |  |  |  |
| PCFICH_RB | dB |  |  |  |  |
| PHICH_RA | dB |  |  |  |  |
| PHICH_RB | dB |  |  |  |  |
| PDCCH_RA | dB |  |  |  |  |
| PDCCH_RB | dB |  |  |  |  |
| PDSCH_RA | dB |  |  |  |  |
| PDSCH_RB | dB |  |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |  |
|  | dBm/15 kHz | -98 |  | -98 | -100 |
| RSRP | dBm/15 KHz | -infinity |  | -84.4 | -103.6 |
|  | dB | -infinity |  | 13.6 | -3.6 |
|  | dB | -infinity |  | 13.6 | -3.6 |
| TreselectionEUTRAN | S | 0 |  |  |  |
| SnonintrasearchP | dB | Not sent |  |  |  |
| Threshx, highP | dB | 48 |  |  |  |
| Threshserving, lowP | dB | 44 |  |  |  |
| Threshx, lowP (Note 2) | dB | 50 |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of threshX-Low-r15 which is included in E-UTRA system information, and is a threshold for the NR target cell. |  |  |  |  |  |

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Thigher_priority_search	See clause 4.2.2.7 of TS 38.133 [6]

Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5 of TS 38.133 [6]

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority E-UTRAN cell.

#### 6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA

6.1.2.2.1	Test purpose

This test is to verify the requirement for the NR to E-UTRAN inter-RAT cell reselection requirements specified in clause 4.2.2.5 of TS 38.133 [6] when the E-UTRAN cell is of lower priority.

6.1.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.1.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.1.2.2.

6.1.2.2.4	Test description

6.1.2.2.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.1.2.2.4.1-1.

Table 6.1.2.2.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 6.1.2.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | LTE 10MHz bandwidth, TDD duplex mode |
| 6.1.2.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | LTE 10MHz bandwidth, TDD duplex mode |
| 6.1.2.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | LTE 10MHz bandwidth, TDD duplex mode |
| 6.1.2.2-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | LTE 10MHz bandwidth, FDD duplex mode |
| 6.1.2.2-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | LTE 10MHz bandwidth, FDD duplex mode |
| 6.1.2.2-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | LTE 10MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equirement and the DUT according to the parameters in Table 6.1.2.2.4.1-2.

Table 6.1.2.2.4.1-2: Initial conditions for NR SA FR1 – E-URTA cell re-selection to lower priority E-UTRA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.1.2.2.4.1-3.

2. Message contents are defined in clause 6.1.2.2.4.3.

3. The test scenario comprises of one NR cell and one E-UTRAN cell. Cell 1 is the NR PCell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 6.1.2.2.4.1-3: General test parameters for NR to lower priority E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE camps on cell 1 in the initial phase. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T1. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell1 | The UE shall perform reselection to cell 1 during T2 for iteration of the tests. |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell2 |  |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1, 2, 3 | 53 | As specified in table 5.7.1-2 in TS 36.211 |
|  |  |  | 4, 5, 6 | 4 |  |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

6.1.2.2.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR PCell (Cell 1) and an E-UTRA neighbour cell (Cell 2). The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both NR cell 1 and E-UTRAN cell 2 are already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of lower priority than cell 1.

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.1.2.2.5-1 and 6.1.2.2.5-2. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

4.	If the UE responds on Cell 2 during time duration T1 within 8 seconds from the beginning of time period T1 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

5.	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, continue with step 6.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 9.

5a	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 6.1.2.2.5-1 and 6.1.2.2.5-2. T2 starts.

7.	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 1.

8.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, skip to step 10.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch on and off the UE.

9.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.1.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.1.2.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with Condition SMTC 1 and lower priority for configuration 6.1.2.2-2, 6.1.2.2-3, 6.1.2.2-5 and 6.1.2.2-6 Table H.2.3-2 with Condition SMTC 2 and lower priority for configuration 6.1.2.2-1 and 6.1.2.2-4 Table H.2.3-3 with Condition lower priority |
| Default RRC messages and information elements contents exceptions |  |

Table 6.1.2.2.4.3-2: SystemInformationBlockType3

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| threshServingLow | 22 | Actual value = 44 dB |  |
| cellReselectionPriority | 4 |  | lower priority E-UTRA |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.2.4.3-3: SystemInformationBlockType24

| Derivation Path: TS 36.508 [25] Table 4.4.3.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| cellReselectionPriority-r15[n] | 5 |  | lower priority E-UTRA |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| } |  |  |  |

6.1.2.2.5	Test requirement

Tables 6.1.2.2.4.1-3, 6.1.2.2.5-1 and 6.1.2.2.5-2 define the primary level settings including test tolerances for lower priority E-UTRA cell re-selection test case.

Table 6.1.2.2.5-1: Cell specific test parameters for NR cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1, 4 | N/A |  |
|  |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1, 4 | SR.1.1 FDD |  |
|  |  | 2, 5 | SR.1.1 TDD |  |
|  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1, 4 | CR.1.1 FDD |  |
|  |  | 2, 5 | CR.1.1 TDD |  |
|  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 4 | CCR.1.1 FDD |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |
|  |  | 2, 5 | SSB.1 FR1 |  |
|  |  | 3, 6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1, 4 | SMTC pattern 2 |  |
|  |  | 2, 5 | SMTC pattern 1 |  |
|  |  | 3, 6 | SMTC pattern 1 |  |
| OCNG Pattern |  | 1, 2, 3, 4, 5, 6 | OP.1 defined in A.2.1-1 |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |
|  |  | 3, 6 | -137 |  |
|  | dBm/SCS | 1, 4 | -100 | -98 |
|  |  | 2, 5 | -100 | -98 |
|  |  | 3, 6 | -97 | -95 |
|  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -100 | -98 |
| SS-RSRP | dBm/SCS | 1, 4 | -103.6 | -84.4 |
|  |  | 2, 5 | -103.6 | -84.4 |
|  |  | 3, 6 | -100.59 | -81.39 |
|  | dB | 1, 4 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
|  | dB | 1, 4 | -3.6 | 13.6 |
|  |  | 2, 5 |  |  |
|  |  | 3, 6 |  |  |
| Io | dBm/9.36 MHz | 1, 4 | -70.46 | -56.26 |
|  | dBm/9.36 MHz | 2, 5 | -70.46 | -56.26 |
|  | dBm/38.16 MHz | 3, 6 | -64.38 | -50.16 |
| Treselection | S | 1, 2, 3, 4, 5, 6 | 0 |  |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |
| Threshx, low P (Note 2) | dB | 1, 2, 3, 4, 5, 6 | 50 |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, LowP which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table 6.1.2.2.5-2: Cell specific test parameters for E-UTRA cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 T3 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -82.4 | -84 |
|  | dB | 15.6 | 14 |
|  | dB | 15.6 | 14 |
| TreselectionEUTRAN | S | 0 |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP (Note 2) | dB | 48 |  |
| Threshserving, lowP | dB | 44 |  |
| Threshx, lowP | dB | 50 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: This refers to the value of Threshx, highP which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

The cell reselection delay to a lower priority E-UTRAN cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure on cell 2.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Tevaluate, E-UTRAN	See Table 4.2.2.5-1 in clause 4.2.2.5 of TS 38.133 [6]

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 7.68 s, allow 8 s for the cell re-selection delay to a lower priority E-UTRAN cell.

## 6.2	RRC_INACTIVE state mobility

## 6.3	RRC_CONNECTED state mobility

### 6.3.1	Handover

#### 6.3.1.0	Minimum conformance requirements

##### 6.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover

When the UE receives a RRC message implying handover to E-UTRAN the UE shall be ready to start the transmission of the uplink PRACH channel in E-UTRA within Dhandover seconds from the end of the last TTI containing the RRC command. Dhandover is defined as

Dhandover = TRRC_procedure_delay + Tinterruption

Where:

TRRC_procedure_delay: it is the RRC procedure delay, which is 50ms

Tinterruption: it is the time between end of the last TTI containing the RRC command on the NR PDSCH and the time the UE starts transmission of the PRACH in E-UTRAN, excluding TRRC_procedure_delay.

When the inter-RAT handover to E-UTRAN is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20 ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is unknown and signal quality is sufficient for successful cell detection on the first attempt, then Tsearch = 80 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is know if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in TS 36.133 [6] clause [9.4.1].

The normative reference for this requirement is TS 38.133 [6] clause 6.1.2.1.

##### 6.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover

When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the end of the last TTI containing the RRC command.

Where:

Dhandover equals the maximum RRC procedure delay to be defined in clause12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.2.2.

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + 20+ T∆ ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥[-2] dB, then Tsearch = Trs + 2 ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥[-2] dB, then Tsearch = [3* Trs + 2] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs=[5]ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

NOTE 1:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in Clause [9.2.5] for intra-frequency handover and Clause [9.3.1] for inter-frequency handover.

The normative reference for this requirement is TS 38.133 [6] clause 6.1.1.2.

##### 6.3.1.0.3	Minimum conformance requirements for NR – UTRAN handover

When the UE receives a RRC message implying handover to UTRAN the UE shall be ready to start the transmission of the new UTRA uplink DPCCH within Dhandover msfrom the end of the last NR TTI containing the RRC MobilityfromNRCommand command.

where:

-	Dhandover equals the RRC procedure delay, which is 50 ms plus the interruption time stated in TS 38.133 [6] clause 6.1.2.2.3.

The interruption time is the time between the end of the last TTI containing the RRC command on the NR PDSCH and the time the UE starts transmission on the uplink DPCCH in UTRAN, excluding the RRC procedure delay. The interruption time depends on whether the target cell is known to the UE or not.

The target cell is known if it has been measured by the UE during the last 5 seconds otherwise it is unknown. The UE shall always perform a UTRA synchronisation procedure as part of the handover procedure.

If the target cell is known the interruption time shall be less than Tinterrupt1

Tinterrupt1 = TIU+Tsync+50+ 10*Fmax + TMC ms

If the target cell is unknown the interruption time shall be less than Tinterrupt2

Tinterrupt2 = TIU+Tsync+150 + 10*Fmax + TMC ms

This requirement shall be met, provided that there is one target cell in the MobilityfromNRCommand command. Performance requirements for E-UTRA to UTRA soft handover are not specified. When UE is connected to an NR cell, UTRA SFN timing measurements are not reported. This implies that the timing of the DPCH of the UTRA target cells in the active set cannot be configured by UTRAN to guarantee that all target cells fall within the UE reception window of T0 +/- 148 chips.

Where:

-	TIU is the interruption uncertainty when changing the timing from the NR to the new UTRAN cell. TIU can be up to one UTRA frame (10 ms).

-	Fmax denotes the maximum number of radio frames within the transmission time intervals of all transport channels that are multiplexed into the same CCTrCH on the UTRA target cell. If HS-PDSCH is configured in the UTRA target cell, Fmax is 4 radio frames.

-	Tsync is the time required for measuring the downlink DPCCH channel as stated in TS 25.214, clause 4.3.1.2. In case higher layers indicate the usage of a post-verification period Tsync=0 ms. Otherwise Tsync=40 ms.

-	TMC is 0ms if a single UTRA cell is configured as the handover target, otherwise 20ms if handover to UTRA with 1, 2 or 3 UTRA carriers with secondary HS-PDSCH is configured.

The phase reference is the primary CPICH.

The requirements in this clause assume that N312 has the smallest possible value i.e. only one insync is required.

#### 6.3.1.1	NR SA FR1 handover with known target cell

6.3.1.1.1	Test purpose

To verify the UE's ability to perform NR FR1-NR FR1 intra frequency handover in RRC_CONNECTED state by meeting the delay requirement and interruption length for handover to a known target cell.

6.3.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.1.1.

6.3.1.1.4	Test description

6.3.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.3.1.1.4.1-1.

Table 6.3.1.1.4.1-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.1.1.4.1-2

Table 6.3.1.1.4.1-2: Initial conditions for NR SA FR1 handover with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.1, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.3.1.1.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 6.3.1.1.4.1-3 below, with A3-Offset modified by Test Tolerance.

Table 6.3.1.1.4.1-3: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | -1 | Original Value 0dB, post TT Analysis, its -1 dB |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

6.3.1.1.4.2	Test procedure

The test scenario comprises of two NR carriers and one cell on each carrier as given in tables 6.3.1.1.4.1-3 and 6.3.1.1.5-1. No gap patterns are configured in the test case. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send measurement report A3. An RRC message implying handover shall be sent to the UE during period T2, after the UE has reported Event A3. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.1.1.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	SS shall transmit an RRCReconfiguration message, configuring measurement object.

4.	UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.3.1.15-1.

6.	UE shall transmit a MeasurementReport message triggered by Event A3.

7.	SS shall transmit the RRCReconfiguration message implying handover is sent to the UE, at that instant the SS shall switch the power settings from T2 to T3 as specified in Table 6.3.1.1.5-1. T3 starts.

8.	The UE shall transmit RRCReconfigurationComplete message.

9.	If the UE transmits the uplink PRACH channel to Cell 2 less than 220 ms from the beginning of time period T3 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

10.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

11.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

12.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.3.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and no GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC pattern 1 and Synchronous cells for Config 1 and 2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC pattern 1 and Synchronous cells for Config 3 Table H.3.1-4 with A3-offset = -1dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.2-2 with Condition RBConfig_KeyChange |

6.3.1.1.5	Test requirements

Table 6.3.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.1.5-1: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  |  | Unit | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 |  | T2 |  | T3 |  | T1 |  | T2 |  | T3 |
| NR RF Channel Number |  |  |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| DRx Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |  |  |  |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| SMTC Configuration |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| SSB Configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |  |  |  |
| PRACH configuration |  |  |  |  | PRACH.1 FR1 |  |  |  |  |  |  |  |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | dBm/15kHz | -98 |  |  |  |  | -98 |  |  |  |  |  |
| Note2 | Config 1,2 |  |  | dBm/SCS | -98 |  |  |  |  | -98 |  |  |  |  |  |
|  | Config 3 |  |  |  | -95 |  |  |  |  | -95 |  |  |  |  |  |
|  | Config 1,2 |  |  | dB | 8 | -2.41 |  | -2.41 |  | -Infinity |  | 1.36 |  | 1.36 |  |
|  | Config 3 |  |  | dB | 8 | -2.41 |  | -2.41 |  | -Infinity |  | 1.36 |  | 1.36 |  |
|  | Config 1,2 |  |  | dB | 8 | 8 |  | 8 |  | -Infinity |  | 10 |  | 10 |  |
|  | Config 3 |  |  | dB | 8 | 8 |  | 8 |  | -Infinity |  | 10 |  | 10 |  |
| SSB_RP |  | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -90 |  | Infinity |  | -88 |  | -88 |  |
|  |  | Config 3 |  | dBm/SCS | -87 | -87 |  | -87 |  | Infinity |  | -84.99 |  | -84.99 |  |
| IoNote3 | Config 1,2 |  |  | dBm/ 9.36MHz | -61.41 | -57.67 |  | -57.67 |  | -61.41 |  | -57.67 |  | -57.67 |  |
|  | Config 3 |  |  | dBm/ 38.16MHz | -55.31 | -51.56 |  | -51.56 |  | -55.31 |  | -51.56 |  | -51.56 |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 220 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 210 ms in the test. Tinterrupt is defined in clause 6.3.1.0.2.

This gives a total of 220 ms.

#### 6.3.1.2	NR SA FR1 handover with unknown target cell

6.3.1.2.1	Test purpose

To verify the UE’s ability to perform NR FR1-NR FR1 intra frequency handover in RRC_CONNECTED state by meeting the delay requirement and interruption length for handover to an unknown target cell.

6.3.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.1.2.

6.3.1.2.4	Test description

6.3.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.3.1.2.4.1-1.

Table 6.3.1.2.4.1-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.1.2.4.1-2

Table 6.3.1.2.4.1-2: Initial conditions for NR SA FR1 handover with unknown target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.1, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.3.1.2.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Table 6.3.1.2.4.1-4 below.

Table 6.3.1.2.4.1-3: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

6.3.1.2.4.2	Test procedure

The test scenario comprises of two NR carriers and one cell on each carrier as given in tables 6.3.1.2.4.1-3 and 6.3.1.2.5-1. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1and T2 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and start to transmit the PRACH to Cell 2. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.1.2.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	SS shall transmit an RRCReconfiguration message implying handover to Cell 2.

4.	The start of T2 is the instant when the last TTI containing the RRC reconfiguration message implying handover is sent to the UE, at that instant the SS shall switch the power settings from T1 to T2 as specified in Table 6.3.1.2.5-1. T2 starts.

5.	The UE shall transmit RRCReconfigurationComplete message.

6.	If the UE transmits the uplink PRACH channel to Cell 2 less than 282 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.3.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.2-2 with Condition RBConfig_KeyChange |

6.3.1.2.5	Test requirements

Table 6.3.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.2.5-1: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf. 2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 FR1 |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | PRACH.1 FR1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |  |
|  |  |  | dB | 8 | -0.64 |  | -Infinity |  | -0.64 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 8 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -Infinity |  | -90 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -87 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.95 |  | -61.41 |  | -57.95 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -51.84 |  | -55.31 |  | -51.84 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 282 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 232 ms in the test. Tinterrupt is defined in clause 6.3.1.0.2.

This gives a total of 282 ms.

#### 6.3.1.3	NR SA FR1-FR1 handover with unknown target cell

6.3.1.3.1	Test purpose

To verify the UE’s ability to perform NR FR1-NR FR1 inter frequency handover in RRC_CONNECTED state by meeting the delay requirement and interruption length for handover to an unknown target cell.

6.3.1.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.1.3.

6.3.1.3.4	Test description

6.3.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.3.1.3.4-1.

Table 6.3.1.3.4.1-1: Inter-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.1.3.4-2

Table 6.3.1.3.4.1-2: Initial conditions for NR SA FR1 handover with unknown target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.1, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.1.3.4-3 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.3.1.3.4.3.

2.	The power levels and settings for NR Cell 1 are set according to Annex C.1.2 and C.1.3. Cell 2 is NR FR1 target Cell, and its power levels and settings are also set according to Annex C.1.2 and C.1.3.

3.	The test parameters are given in Tables 6.3.1.3.4.1-3 and 6.3.1.3.5-1 below.

Table 6.3.1.3.4.1-3: General test parameters Inter-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset |  | dB | -4 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

6.3.1.3.4.2	Test procedure

The test scenario comprises of two NR carriers and one cell on each carrier as given in tables 6.3.1.3.4-3 and 6.3.1.3.5-1. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1and T2 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and start to transmit the PRACH to Cell 2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.1.3.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	SS shall transmit an RRCReconfiguration message implying handover to Cell 2.

4.	The start of T2 is the instant when the last TTI containing the RRC reconfiguration message implying handover is sent to the UE, at that instant the SS shall switch the power settings from T1 to T2 as specified in Table 6.3.1.3.5-1. T2 starts.

5.	The UE shall transmit RRCReconfigurationComplete message.

6.	If the UE transmits the uplink PRACH channel to Cell 2 less than 282 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.3.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.1.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ and no GAP NEEDED Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1, SMTC pattern 1 and Synchronous cells for Config 1 and 2 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1, SMTC pattern 2 and Synchronous cells for Config 3 Table H.3.1-4 with A3-offset = -4dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.2-2 with Condition RBConfig_KeyChange |

6.3.1.3.5	Test requirements

Table 6.3.1.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.3.1.3.5-1: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  | -98 |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  | -98 |  |  |  |
|  | Config 3 |  |  | -95 |  |  | -95 |  |  |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 6.7 |  |
|  |  |  | dB | 4 | 4 |  | -Infinity |  | 6.7 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -94 | -94 |  | -Infinity |  | -91.3 |  |
|  | Config 3 |  | dBm/SCS | -91 | -91 |  | -Infinity |  | -88.3 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -62.51 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -56.40 |  |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 282 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 272 ms in the test. Tinterrupt is defined in clause 6.3.1.0.2.

This gives a total of 282 ms.

#### 6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell

6.3.1.4.1	Test purpose

To verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1.

6.3.1.4.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.1.4.

6.3.1.4.4	Test description

6.3.1.4.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.3.1.4.4.1-1.

Table 6.3.1.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.3.1.4-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.3.1.4-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.3.1.4-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.3.1.4-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.3.1.4-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.3.1.4-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.3.1.4.4.1-2.

Table 6.3.1.4.4.1-2: Initial conditions for NR SA FR1 – E-UTRA handover with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.3.1.4.4.1-3.

2. Message contents are defined in clause 6.3.1.4.4.3.

3. The test comprises of one NR carrier and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 6.3.1.4.4.1-3: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | As specified in Table 6.3.1.4.5-1 | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2EUTRAN |  | dBm | -98 | Absolute E-UTRAN RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in TS 38.133 [6], table 9.1.2-1 started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | ≤5 |  |
| T3 |  | s | 1 |  |

6.3.1.4.4.2	Test procedure

The test comprises of one NR carrier and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 of TS 38.133 [6] is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.1.4.5-1 and 6.3.1.4.5-2. T1 starts.

3.	The SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.3.1.4.5-1 and 6.3.1.4.5-2.

6.	UE shall transmit a MeasurementReport message triggered by Event B2.

7.	SS shall transmit an RRCReconfiguration message implying handover to Cell 2.

8.	The start of T3 is the instant when the last TTI containing the RRCReconfiguration message implying handover is sent to the UE, at that instant the SS shall switch the power setting from T2 to T3 as specified in Table 6.3.1.4.5-1 and 6.3.1.4.5-2.

9.	The UE shall transmit RRCConnectionReconfigurationComplete message on Cell 2.

10.	If the UE transmits the uplink PRACH channel to Cell 2 less than 85 ms from the beginning of time period T3 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

11.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

12.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.3.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.3.1.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-RAT and GAP NEEDED Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT Table H.3.3-1 |
| Specific message contents exceptions for Test Configuration 6.3.1.4-1, 6.3.1.4-2, 6.3.1.4-4 and 6.3.1.4-5 | Table H.3.1-3 with Condition SSB.1 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.3.1.4-3 and 6.3.1.4-6 | Table H.3.1-3 with Condition SSB.2 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.3.1.4.4.3-2: PRACH-Config-DEFAULT: Inter-RAT handover to E-UTRAN known cell

| Derivation Path: TS 36.508 [25], Table 4.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 4 |  | Config 1, 2, 3 |
|  | 53 |  | Config 4, 5, 6 |
| } |  |  |  |
| } |  |  |  |

6.3.1.4.5	Test requirement

Tables 6.3.1.4.4.1-3, 6.3.1.4.5-1 and 6.3.1.4.5-2 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case.

Table 6.3.1.4.5-1: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T3 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |  |
| Duplex mode |  |  | 1, 4 | FDD |  |  |  |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |  |  |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |  |  |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |  |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |  |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |  |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |  |  |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |  |  |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |  |  |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |  |  |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |  |  |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |  |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DL BWP.0.1 |  |  |  |  |
|  | Dedicated DL BWP |  |  | DL BWP.1.1 |  |  |  |  |
|  | Initial UL BWP |  |  | UL BWP.0.1 |  |  |  |  |
|  | Dedicated UL BWP |  |  | UL BWP.1.1 |  |  |  |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |  |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |  |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |  |  |  |
| b2-Threshold1 |  | dBm | 1, 2, 4, 5 | -96 |  |  |  |  |
|  |  |  | 3, 6 | -93 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -100 | -104 |  | -100 |  |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -100 | -104 |  | -100 |  |
|  |  |  | 3, 6 | -97 | -101 |  | -97 |  |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | 13.55 |  | -1.55 |  | -5.55 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | 13.55 |  | -1.55 |  | -5.55 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -86.45 |  | -105.55 |  | -105.55 |
|  |  |  | 3, 6 | -83.44 |  | -102.54 |  | -102.54 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.31 |  | -73.04 |  | -70.98 |
|  |  | dBm/38.16 MHz | 3, 6 | -52.21 |  | -66.94 |  | -64.88 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table 6.3.1.4.5-2: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |  |
|  |  | 4, 5, 6 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 9.55 | 9.55 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 9.55 | 9.55 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -88.45 | -88.45 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -88.45 | -88.45 |
| IoNote6 | dBm/Ch BW | 1, 2, 3, 4, 5, 6 | -67.21 +10log (NRB,c /100) | -57.20 +10log (NRB,c /100) | -57.20 +10log (NRB,c /100) |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211. Note 3: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 85 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in section 6.1.2.1 of TS 38.133 [6].

Tinterrupt = 35 ms in the test; Tinterrupt is defined in section 6.1.2.1 of TS 38.133 [6].

This gives a total of 85 ms.

#### 6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell

6.3.1.5.1	Test purpose

To verify that the UE can make correct inter-RAT E-UTRAN handover when operating in standalone (SA) operation with PCell in FR1. This test shall verify the NR to E-UTRAN handover requirements for the case when the target E-UTRAN cell is unknown as specified in section 6.1.2.1 of TS 38.133 [6].

6.3.1.5.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.1.2.

6.3.1.5.4	Test description

6.3.1.5.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.3.1.5.4.1-1.

Table 6.3.1.5.4.1-1: Supported test configurations for SA inter-RAT E-UTRAN handover tests

| Configuration | Description |
| --- | --- |
| 6.3.1.5-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.3.1.5-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.3.1.5-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.3.1.5-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.3.1.5-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.3.1.5-6 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.3.1.5.4.1-2.

Table 6.3.1.5.4.1-2: Initial conditions for NR SA FR1 – E-UTRA handover with unknown target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.3.1.5.4.1-3.

2. Message contents are defined in clause 6.3.1.5.4.3.

3. The test comprises of one NR carrier and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is the E-UTRAN neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2. Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 6.3.1.5.4.1-3: General test parameters for SA inter-RAT E-UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | E-UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

6.3.1.5.4.2	Test procedure

The test comprises of one NR carrier and one E-UTRA carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN neighbour cell. The test consists of two successive time periods, with time durations of T1 and T2. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable. No Gap pattern shall be configured.

An RRC message implying handover shall be sent to the UE during period T1. The start of T2 is the instant when the last subframe containing the RRC message implying handover is sent to the UE. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and start to transmit the PRACH to Cell 2.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.1.5.5-1 and 6.3.1.5.5-2. T1 starts.

3.	SS shall transmit an RRCReconfiguration message implying handover to Cell 2.

4.	The start of T2 is the instant when the last subframe containing the RRC Reconfiguration message implying handover is sent to the UE, at that instant the SS shall switch the power settings from T1 to T2 as specified in Table 6.3.1.5.5-1 and 6.3.1.5.5-2. T2 starts.

5.	The UE shall transmit RRCConnectionReconfigurationComplete message on Cell 2.

6.	If the UE transmits the uplink PRACH channel to Cell 2 less than 165 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	After T2 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	Repeat step 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.3.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.1.5.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.3-1 |

Table 6.3.1.5.4.3-2: PRACH-Config-DEFAULT: Inter-RAT handover to E-UTRAN known cell

| Derivation Path: TS 36.508 [25], Table 4.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 4 |  | Config 1, 2, 3 |
|  | 53 |  | Config 4, 5, 6 |
| } |  |  |  |
| } |  |  |  |

6.3.1.5.5	Test requirement

Tables 6.3.1.5.4.1-3, 6.3.1.5.5-1 and 6.3.1.5.5-2 define the primary level settings including test tolerances for inter-RAT E-UTRAN handover: unknown target cell test.

Table 6.3.1.5.5-1: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  | 1, 4 | FDD |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |
|  |  |  | 3, 6 | TDDConf. 2.1 |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  |  | 3, 6 | -95 |  |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | 0 | 0 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | 0 | 0 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -98 | -98 |
|  |  |  | 3, 6 | -95 | -95 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -67.04 | -67.04 |
|  |  | dBm/38.16 MHz | 3, 6 | -60.94 | -60.94 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 6.3.1.5.5-2: Cell specific test parameters for SA inter-RAT E-UTRA handover (Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |
|  |  | 4, 5, 6 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -91 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -91 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -70.22 | -62.43 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211. Note 3: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 165 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms and is specified in section 6.1.2.1 of TS 38.133 [6].

Tinterrupt = 115 ms in the test; Tinterrupt is defined in section 6.1.2.1 of TS 38.133 [6].

This gives a total of 165 ms.

#### 6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell

Editor's note: Following part of this test case are still incomplete:

-	Connect diagram is missing;

-	Message contents is missing;

-	TT analysis is missing

6.3.1.6.1	Test purpose

To verify that the UE can make correct inter-RAT UTRAN handover when operating in standalone (SA) operation with PCell in FR1.

6.3.1.6.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards.

6.3.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.1.6.

6.3.1.6.4	Test description

6.3.1.6.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.3.1.6.4.1-1.

Table 6.3.1.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.3.1.6-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, UTRAN FDD |
| 6.3.1.6-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, UTRAN FDD |
| 6.3.1.6-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, UTRAN FDD |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.3.1.6.4.1-2.

Table 6.3.1.6.4.1-2: Initial conditions for NR SA FR1 – UTRAN handover with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table FFS and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part | FFS |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.3.1.6.4.1-3.

2. Message contents are defined in clause 6.3.1.6.4.3.

3. The test comprises of one NR carrier and one UTRAN carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 Annex C.0 and C.1.

Table 6.3.1.6.4.1-3: General test parameters for SA inter-RAT UTRAN handover

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  |  | 2 | 1 UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | NR cell |
|  | Neighbouring cell |  | Cell 2 | UTRAN cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| Inter-RAT (UTRAN FDD) measurement quantity |  |  | CPICH Ec/N0 |  |
| b2-Threshold1 |  | dBm | As specified in Table FFS | Absolute NR SS-RSRP threshold for event B2 |
| b2-Threshold2-UTRA |  | dB | -18 | Absolute UTRAN CPICH Ec/Io threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table FFS started before T2 starts |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

6.3.1.6.4.2	Test procedure

The test comprises of one NR carrier and one UTRAN carrier. There are two cells and one cell on each carrier. Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 9.1.2-1 of TS 38.133 [6] is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

1.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.1.6.5-1 and 6.3.1.6.5-2. T1 starts.

3.	The SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.3.1.6.5-1 and 6.3.1.6.5-2.

6.	UE shall transmit a MeasurementReport message triggered by Event B2.

7.	SS shall transmit an MobilityFromNRCommand message implying handover to Cell 2.

8.	The start of T3 is the instant when the last TTI containing the MobilityFromNRCommand message implying handover is sent to the UE, at that instant the SS shall switch the power setting from T2 to T3 as specified in Table 6.3.1.6.5-1 and 6.3.1.6.5-2.

9.	The UE shall transmit HANDOVER TO UTRAN COMMAND message.

10.	If the UE transmits the uplink PRACH channel to Cell 2 less than 85 ms from the beginning of time period T3 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

11.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

12.	The SS shall set Cell 2 primary scrambling code = ((current cell 2 primary scrambling code - 50) mod 200 + 100) for next iteration of the test procedure loop.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.3.1.6.4.3	Message contents

FFS

6.3.1.6.5	Test requirement

Tables 6.3.1.6.5-1 and 6.3.1.6.5-2 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case.

Table 6.3.1.6.5-1: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 1)

| Parameter |  | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3 | 1 |  |  |
| Duplex mode |  |  | 1 | FDD |  |  |
|  |  |  | 2, 3 | TDD |  |  |
| TDD Configuration |  |  | 2 | TDDConf.1.1 |  |  |
|  |  |  | 3 | TDDConf.1.2 |  |  |
| BWchannel |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1 | SR.1.1 FDD |  |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1 | CR.1.1 FDD |  |  |
|  |  |  | 2 | CR.1.1 TDD |  |  |
|  |  |  | 3 | CR.2.1 TDD |  |  |
| TRS configuration |  |  | 1 | TRS.1.1 FDD |  |  |
|  |  |  | 2 | TRS.1.1 TDD |  |  |
|  |  |  | 3 | TRS.1.2 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  |  |
|  |  |  | 3 | SSB.2 FR1 |  |  |
| b2-Threshold1 |  | dBm | 1, 2 | -96 |  |  |
|  |  |  | 3 | -93 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3 | -100 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2, | -100 |  |  |
|  |  |  | 3 | -97 |  |  |
| Ês/Noc |  | dB | 1, 2, 3 | 012 | 0-4 | 0-4 |
| Ês/IotNote3 |  | dB | 1, 2, 3 | 012 | 0-4 | 0-4 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2 | -88 | -104 | -104 |
|  |  |  | 3 | -85 | -101 | -101 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2 | -59.78 | -70.59 | -70.59 |
|  |  | dBm/38.16 MHz | 3 | -53.68 | -64.49 | -64.49 |
| Propagation condition |  |  | 1, 2, 3 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.3.1.6.5-2: Cell specific test parameters for SA inter-RAT UTRAN FDD handover (Cell 2)

| Parameter | Unit | Cell 2 (UTRA) |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| CPICH_Ec/Ior | dB | -10 |  |  |
| PCCPCH_Ec/Ior | dB | -12 |  |  |
| SCH_Ec/Ior | dB | -12 |  |  |
| PICH_Ec/Ior | dB | -15 |  |  |
| DCH_Ec/Ior | dB | N/A | N/A | Note 1 |
| OCNS_Ec/Ior | dB | -0.941 | 0.941 | Note 2 |
|  | dB | -infinity | -1.8 | -1.8 |
|  | dBm/3,84 MHz | -70 | -70 | -70 |
| CPICH_Ec/Io | dB | -infinity | -14 | -14 |
| Propagation Condition |  | AWGN |  |  |
| Note 1: The DPCH level is controlled by the power control loop Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior . |  |  |  |  |

The UE shall start to transmit the UL DPCCH to Cell 2 less than 190 ms from the beginning of time period T3.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 50 ms, which is specified in clause 5.3.1.1.1.

Tinterrupt = 140 ms in the test; Tinterrupt is defined in clause 5.3.1.1.2. 	This gives a total of 190 ms.

### 6.3.2	RRC connection mobility control

#### 6.3.2.1	RRC re-establishment

##### 6.3.2.1.0	Minimum conformance requirements

6.3.2.1.0.1	Minimum conformance requirements for FR1 RRC re-establishment

In RRC connected mode the UE shall be capable of sending RRCReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection. The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establish_delay=TUE_re-establish_delay+TUL_grant

TUL_grant: It is the time required to acquire and process uplink grant from the target PCell. The uplink grant is required to transmit RRCReestablishmentRequest message.

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 38.331 [2] is detected by the UE and when the UE sends PRACH to the target PCell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

The intra-frequency target NR cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in Section 10.1.2 and 10.1.3 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding NR Band.

The inter-frequency target NR cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in Section 10.1.4 and 10.1.5 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding NR Band.

Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in table 6.3.2.1.0.1-1.

Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. Tidentify_inter_NR,i shall not exceed the values defined in table 6.3.2.1.0.1-2.

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i.

TSI-NR  = It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target NR cell.

TPRACH = It is the delay caused due to the random access procedure when sending random access to the target NR cell. The delay depends on the PRACH configuration defined in Table 6.3.3.2-2 [6] or Table 6.3.3.2-3 [6] for FR1 and in Table 6.3.3.2-4 [6] for FR2.

Nfreq: It is the total number of NR frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target intra-frequency NR cell is known, else Nfreq = 2 and Tidentify_intra_NR = 0 if the target inter-frequency NR cell is known.

There is no requirement if the target cell does not contain the UE context.

In the requirement defined in the below tables, the target FR1 cell is known if it has been meeting the relevant cell identification requirement during the last [5] seconds otherwise it is unknown.

Table 6.3.2.1.0.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ [-8] | FR1 | MAX (200 ms, [5] x TSMTC) | MAX (800 ms, [10] x TSMTC) |
| ≥ [-8] | FR2 | N/A | MAX (1000 ms, [80] x TSMTC)) |
| < [-8] | FR1 | N/A | 800Note1 |
| < [-8] | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < [-8] dB. |  |  |  |

Table 6.3.2.1.0.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ [-8] | FR1 | MAX (200 ms, [6] x TSMTC, i) | MAX (800 ms, [13] x TSMTC, i) |
| ≥ [-8] | FR2 | N/A | MAX (1000 ms, [104] x TSMTC, i)) |
| < [-8] | FR1 | N/A | 800Note1 |
| < [-8] | FR2 | N/A | 4000Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < [-8] dB. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.1.2.1.

##### 6.3.2.1.1	NR SA FR1 RRC re-establishment

6.3.2.1.1.1	Test purpose

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 with known target cell is within the specified limits.

6.3.2.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.2.1.1.

6.3.2.1.1.4	Test description

6.3.2.1.1.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.3.2.1.1.4.1-1

Table 6.3.2.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.3.2.1.1-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.3.2.1.1-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.3.2.1.1-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equirement and the DUT according to the parameters in Table 6.3.2.1.1.4.1-2

Table 6.3.2.1.1.4.1-2: Initial conditions for NR Intra-frequency RRC Re-establishment in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 6.3.2.1.1.4.1-3.

2.	Message contents are defined in clause 6.3.2.1.1.4.3.

There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.3.2.1.1.4.1-3: General test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1, 2, 3 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 200 | Time for the UE to detect RLF |
| T3 |  | s | 1, 2, 3 | 2 |  |

6.3.2.1.1.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	Set the parameters according to T1 in Table 6.3.2.1.1.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.3.2.1.1.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 6.3.2.1.1.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within 1.6 s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

6.3.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3with the following exceptions:

Table 6.3.2.1.1.4.3-1: Common Exception messages intra-frequency RRC re-establishment

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Specific message contents exceptions for Test Configuration 6.3.2.1.1-1 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.3.2.1.1-2 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1 and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.3.2.1.1-3 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1 and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.3.2.1.1.4.3-2: RLF-TimersAndConstants for intra-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| t311 | ms3000 |  |  |
| } |  |  |  |

Table 6.3.2.1.1.4.3-3: MeasConfig for intra-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig::= SEQUENCE { |  |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| } |  |  |  |

6.3.2.1.1.5	Test requirement

Table 6.3.2.1.1.5-1 defines the primary level settings including test tolerances for NR Intra-frequency RRC Re-establishment in FR1 test case.

Table 6.3.2.1.1.5-1: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 | T1 | T2 |  | T3 |
| TDD configuration |  | 1 | N/A |  |  |  |  | N/A |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  |  | N/A |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  | N/A |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  |  |  | ULBWP.0.1 |  |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |  | N/A |  | N/A | N/A | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |  | N/A |  | N/A | N/A | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  |  | SSB |  |  |  |
|  | dB | 1 | 1.54 |  | -infinity |  | -infinity | -3.79 | 4 |  | 4 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 7 |  | -infinity |  | -infinity | 4 | 4 |  | 4 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -91 |  | -infinity |  | -infinity | -94 | -94 |  | -94 |
|  |  | 2 | -91 |  | -infinity |  | -infinity | -94 | -94 |  | -94 |
|  |  | 3 | -88 |  | -infinity |  | -infinity | -91 | -91 |  | -91 |
| Io | dBm/9.36 MHz | 1 | -60.74 |  | -64.59 |  | -64.59 | -60.74 | -64.59 |  | -64.59 |
|  | dBm/9.36 MHz | 2 | -60.74 |  | -64.59 |  | -64.59 | -60.74 | -64.59 |  | -64.59 |
|  | dBm/38.16 MHz | 3 | -54.65 |  | -58.50 |  | -58.50 | -54.65 | -58.50 |  | -58.50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to a known NR intra frequency cell shall be less than 1.6 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 200 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 1545 ms, allow 1.6 s in the test case.

##### 6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment

6.3.2.1.2.1	Test purpose

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR1 without known target cell is within the specified limits.

6.3.2.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] A.6.3.2.1.2.

6.3.2.1.2.4	Test description

6.3.2.1.2.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.3.2.1.2.4.1-1

Table 6.3.2.1.2.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 6.3.2.1.2-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.3.2.1.2-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.3.2.1.2-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equirement and the DUT according to the parameters in Table 6.3.2.1.2.4.1-2

Table 6.3.2.1.2.4.1-2: Initial conditions for NR Inter-frequency RRC Re-establishment in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 6.3.2.1.2.4.1-3.

2.	Message contents are defined in clause 6.3.2.1.2.4.3.

There are two NR carriers and two cells specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.3.2.1.2.4.1-3: General test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1, 2, 3 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | PRACH.1 FR1 | Table A.7.1-1 |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | ms | 1, 2, 3 | 200 | Time for the UE to detect RLF |
| T3 |  | s | 1, 2, 3 | 5 |  |

6.3.2.1.2.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	Set the parameters according to T1 in Table 6.3.2.1.2.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.3.2.1.2.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 6.3.2.1.2.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within 3 s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

6.3.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3with the following exceptions:

Table 6.3.2.1.2.4.3-1: Common Exception messages inter-frequency RRC re-establishment

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Specific message contents exceptions for Test Configuration 6.3.2.1.2-1 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.3.2.1.2-2 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.3.2.1.2-3 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.3.2.1.2.4.3-2: RLF-TimersAndConstants for inter-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| t311 | ms5000 |  |  |
| } |  |  |  |

Table 6.3.2.1.2.4.3-3: MeasConfig for inter-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig::= SEQUENCE { |  |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| } |  |  |  |

6.3.2.1.2.5	Test requirement

Table 6.3.2.1.2.5-1 defines the primary level settings including test tolerances for NR Inter-frequency RRC Re-establishment in FR1 test case.

Table 6.3.2.1.2.5-1: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 | T1 |  | T2 | T3 |
| RF Channel Number |  | 1, 2, 3 | 1 |  |  |  |  | 2 |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  |  | N/A |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  |  | N/A |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  | N/A |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  |  |  |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0 |  |  |  |  | DLBWP.0 |  |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0 |  |  |  |  | ULBWP.0 |  |  |  |
| Active DL BWP confgiuration |  | 1, 2, 3 | DLBWP.1.1 | N/A |  | N/A |  | N/A | N/A |  | DLBWP.1.1 |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 | N/A |  | N/A |  | N/A | N/A |  | ULBWP.1.1 |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  |  | SSB |  |  |  |
|  | dB | 1 | 4 |  | -infinity |  | -infinity | -infinity |  | -infinity | 7 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
|  | dB | 1 | 4 |  | -infinity |  | -infinity | -infinity |  | -infinity | 7 |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -94 |  | -infinity |  | -infinity | -infinity |  | -infinity | -91 |
|  |  | 2 | -94 |  | -infinity |  | -infinity | -infinity |  | -infinity | -91 |
|  |  | 3 | -91 |  | -infinity |  | -infinity | -infinity |  | -infinity | -88 |
| Io | dBm/9.36 MHz | 1 | -64.59 |  | -70.05 |  | -70.05 | -70.05 |  | -70.05 | -62.26 |
|  | dBm/9.36 MHz | 2 | -64.59 |  | -70.05 |  | -70.05 | -70.05 |  | -70.05 | -62.26 |
|  | dBm/38.16 MHz | 3 | -58.50 |  | -63.94 |  | -63.94 | -63.94 |  | -63.94 | -56.15 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 3 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2

Tidentify_intra_NR = 800 ms

Tidentify_inter_NR = 800 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 2945 ms, allow 3 s in the test case.

##### 6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing

6.3.2.1.3.1	Test purpose

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR1 without serving cell timing is within the specified limits and to verify the requirements in TS 38.133 [6] clause 6.2.1

6.3.2.1.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.2.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.3.2.1.3.

6.3.2.1.3.4	Test description

6.3.2.1.3.4.1	Initial conditions

The test shall be tested using any of the test configuration in Table 6.3.2.1.3.4.1-1

Table 6.3.2.1.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.3.2.1.3-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.3.2.1.3-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.3.2.1.3-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.3.2.1.3.4.1-2

Table 6.3.2.1.3.4.1-2: Initial conditions for NR SA FR1 RRC re-establishment without serving cell timing

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 6.3.2.1.3.4.1-3.

2.	Message contents are defined in clause 6.3.2.1.3.4.3.

3.	There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.3.2.1.3.4.1-3: General test parameters for NR SA FR1 RRC re-establishment without serving cell timing

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| N310 |  | - | 1, 2, 3 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1, 2, 3 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1, 2, 3 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1, 2, 3 | 3000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2, 3 | OFF |  |
| PRACH configuration |  |  | 1, 2, 3 | PRACH.1 FR1 |  |
| T1 |  | s | 1, 2, 3 | 5 |  |
| T2 |  | s | 1, 2, 3 | 6 | Time for the UE to detect RLF |
| T3 |  | s | 1, 2, 3 | 3 |  |

6.3.2.1.3.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, is deactivated. The time period T3 starts after the occurrence of the radio link failure.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	Set the parameters according to T1 in Table 6.3.2.1.3.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.3.2.1.3.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 6.3.2.1.3.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within 2.2 s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

6.3.2.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.3.2.1.3.4.3-1: Common Exception messages for NR SA FR1 RRC re-establishment without serving cell timing

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Specific message contents exceptions for Test Configuration 6.3.2.1.3-1 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.3.2.1.3-2 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1 and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.3.2.1.3-3 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1 and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.3.2.1.3.4.3-2: RLF-TimersAndConstants for intra-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms6000 |  |  |
| t311 | ms3000 |  |  |
| } |  |  |  |

Table 6.3.2.1.3.4.3-3: MeasConfig for intra-frequency RRC re-establishment

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig::= SEQUENCE { |  |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | Not present |  |  |
| } |  |  |  |

6.3.2.1.3.5	Test requirement

Table 6.3.2.1.3.5-1 defines the primary level settings including test tolerances for NR SA FR1 RRC re-establishment without serving cell timing test case.

Table 6.3.2.1.3.5-1: Cell specific test parameters for NR SA FR1 RRC re-establishment without serving cell timing

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | -infinity | -infinity | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 2 | -94 | -infinity | -infinity | -infinity | -infinity | -94 |
|  |  | 3 | -91 | -infinity | -infinity | -infinity | -infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.59 | -70.05 | -70.05 | -70.05 | -70.05 | -64.59 |
|  | dBm/9.36 MHz | 2 | -64.59 | -70.05 | -70.05 | -70.05 | -70.05 | -64.59 |
|  | dBm/38.16 MHz | 3 | -58.50 | -63.94 | -63.94 | -63.94 | -63.94 | -58.50 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 2.2 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 800 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 [13] for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 2145 ms, allow 2.2 s in the test case.

#### 6.3.2.2	Random access

##### 6.3.2.2.0	Minimum conformance requirements

6.3.2.2.0.1	Minimum conformance requirements for Contention based random access

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-1 [2] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-1 [2] Table 6.3.4.3-1.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB if the association between Random Access Preambles and SS blocks is configured, as specified in clause 5.1.2 in TS 38.321 [12].

With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, if the association between PRACH occasions and SSBs is configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [12].

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

The UE shall send ACK if the Contention Resolution is successful.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2.

6.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-1 [2] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-1 [2] Table 6.3.4.3-1.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs is configured, with the UE selected CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs, UE shall have the capability to select the Random Access Preamble corresponding to the selected CSI-RS, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS, and PRACH occasion shall be randomly selected with equal probability amongst the selected CSI-RS associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s), if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, unless the random access procedure is initialized for Other SI request from UE.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.2.

Non-contention based random access procedure is not initialized for Other SI requested from UE or for beam failure recovery, so the requirements related to those features are omitted.

##### 6.3.2.2.1	NR SA FR1 contention based random access

6.3.2.2.1.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

6.3.2.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.2.2.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clauses A.6.3.2.2.1.

6.3.2.2.1.4	Test description

6.3.2.2.1.4.1	Initial conditions

This test can be run in the configurations defined in Table 6.3.2.2.1.4.1-1.

Table 6.3.2.2.1.4.1-1: Contention based random access test in FR1 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 6.3.2.2.1-1 | 1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.3.2.2.1-2 | 2 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.2.1.4.1-2.

Table 6.3.2.2.1.4.1-2: Initial conditions for Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.3.2.2.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell). The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.3.2.2.1.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR1. The System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 6.3.2.2.1.5-1.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when transmitting Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB.

5.	Test 2: Correct behaviour when receiving Random Access Response

5.1. Repeat steps 1-3.

5.2. The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

5.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

5.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

5.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

5.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 6.3.2.2.1.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in 6.3.2.2.1.5.

6.	Test 3: Correct behaviour when not receiving Random Access Response

6.1. Repeat steps 1-3.

6.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

6.3. As no Random Access Response was received within the RA Response window, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 6.3.2.2.1.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in 6.3.2.2.1.5.

7.	Test 4: Correct behaviour when receiving an UL grant for msg3 retransmission

7.1. Repeat steps 1-3.

7.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

7.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

7.4. The System Simulator shall send PDCCH addressed to the Temporary C-RNTI after receiving the msg3.

7.5. The UE shall re-transmit the msg3.

7.6. The System Simulator shall check if UE re-transmit the msg3.

8.	Test 5: Correct behaviour when receiving an unsuccessful UE Contention Resolution

8.1. Repeat steps 1-3.

8.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

8.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

8.4. The System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

8.5. As the UE Contention Resolution Identity included in the MAC control element did not match the CCCH SDU transmitted in the uplink message, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

8.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 6.3.2.2.1.5.

9.	Test 6: Correct behaviour when receiving a successful UE Contention Resolution

9.1. Repeat steps 1-3.

9.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

9.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

9.4. The System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in msg3 uplink message.

9.5. As the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU, the Contention Resolution is successful and the UE shall send ACK.

10.	Test 7: Correct behaviour when contention Resolution timer expires

10.1. Repeat steps 1-3.

10.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

10.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

10.4. The System Simulator shall not send a response.

10.5. As there was no response, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the Contention Resolution Timer expires and then after the backoff timer expires.

10.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 6.3.2.2.1.5.

6.3.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.2.2.1.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

Table 6.3.2.2.1.4.3-1: FrequencyInfoUL-SIB for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 6.3.2.2.1.4.3-2: RACH-ConfigCommon for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n48 |  | FR1 |
| } |  |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| rsrp-ThresholdSSB | RSRP_51 |  |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 15 |  |  |
|  | kHz 30 |  |  |
| } |  |  |  |

Table 6.3.2.2.1.4.3-3: RACH-ConfigGeneric for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  | FR1 |
| msg1-FDM | one |  | FR1 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 6.3.2.2.1.4.3-4: ServingCellConfigCommonSIB for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | '1100 0000'B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | -5 |  |  |
| } |  |  |  |

6.3.2.2.1.5	Test requirement

Table 6.3.2.2.1.5-1 defines the primary level settings for contention based random access test in FR1 for NR Standalone. Tables 6.3.2.2.1.5-2, 6.3.2.2.1.5-3 and 6.3.2.2.1.5-4 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 6.3.2.2.1.5-1: General test parameters for contention based random access test in FR1 for NR Standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 | As defined in A.3.1, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB.2 FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3.1 |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3.1 |
| Duplex Mode for Cell 2 |  | Config 1 |  | FDD |  |
|  |  | Config 2 |  | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.1.1 FDD | As defined in A.1.1 |
|  |  | Config 2 |  | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |
| SSB with index 0 |  |  | dB | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | 3 |  |
|  | SS-RSRP | Config 1 | dBm/15kHz | -95 |  |
|  |  | Config 2 |  | -98 |  |
|  | SS-RSRP |  | dBm/ SCS | -95 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 |  |
|  |  | Config 2 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP | Config 1 | dBm/15kHz | -115 |  |
|  |  | Config 2 |  | -118 |  |
|  | SS-RSRP |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | FR1 PRACH configuration 1 | As defined in A.3.x. |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Es/Iot, SS-RSRP and Io level have been derived from other parameters for information purpose. They are not settable parameters. Note 3: Void. Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Test 1: Correct behaviour when transmitting Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

Test 2: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.1.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.1.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.1.5-4.

Test 3: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.1.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.1.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.1.5-4.

Test 4: Correct behaviour when receiving an UL grant for msg3 retransmission

-	The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

Test 5: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires.

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.1.5-2.

-	The transmit timing of the PRACH transmission shall be within the accuracy specified in Table 6.3.2.2.1.5-4.

Test 6: Correct behaviour when receiving a correct message over Temporary C-RNTI

-	The UE shall send ACK if the contention resolution is successful.

Test 7: Correct behaviour when contention resolution timer expires

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires if the contention resolution timer expires.

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.1.5-2.

-	The transmit timing of the PRACH transmission shall be within the accuracy specified in Table 6.3.2.2.1.5-4.

Table 6.3.2.2.1.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 6.3.2.2.1.5-3 Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 |

Table 6.3.2.2.1.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 6.3.2.2.2	NR SA FR1 non-contention based random access

6.3.2.2.2.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

6.3.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Additionally Test 2 is applicable to UE that supports CSI-RS based Random Access Preamble which requires UE to support csi-RSRP-AndRSRQ-MeasWithSSB or csi-RSRP-AndRSRQ-MeasWithoutSSB.

6.3.2.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clauses A.6.3.2.2.2.

6.3.2.2.2.4	Test description

6.3.2.2.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 6.3.2.2.2.4.1-1.

Table 6.3.2.2.2.4.1-1: Non-Contention based random access test in FR1 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 6.3.2.2.2-1 | 1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.3.2.2.2-2 | 2 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.2.2.4.1-2.

Table 6.3.2.2.2.4.1-2: Initial conditions for Non-Contention based random access test in FR1 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.3.2.2.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell). The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.3.2.2.2.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR1. The System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink. There are two subtests, to test both SSB-based non-contention based random access (subtest 1) and CSI-RS-based non-contention based random access (subtest 2).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	Set the parameters according to Table 6.3.2.2.2.5-1 Subtest 1.

3.	The SS shall signal a Random Access Preamble ID via a RRCReconfiguration message to the UE and initiate a Non-contention based Random Access procedure according to Table 6.3.2.2.2.4.3-3 and Table 6.3.2.2.2.4.3-4.

4.	Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

5.	Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

5.1. Set the parameters according to Table 6.3.2.2.2.5-1 Subtest 2.

5.2. Repeat steps 1-3.

5.3. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the CSI-RS configured, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

6.	Test 3: Correct behaviour when receiving Random Access Response

6.1. Repeat steps 1-3.

6.2. The UE shall send preambles to the System Simulator. In response to the first 4 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

6.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 6.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in 6.3.2.2.2.5.

7.	Test 4: Correct behaviour when not receiving Random Access Response

7.1. Repeat steps 1-3.

7.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 4 preambles.

7.3. As no Random Access Response was received within the RA Response window configured in RACH-ConfigCommon, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

7.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 5 preambles have been received by the System Simulator.

7.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

7.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 6.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 4 preambles) and it shall not exceed the values specified in 6.3.2.2.2.5.

6.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.2.2.2.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Common exceptions to the contents of TS 38.508-1 [14] | Table 4.6.3-115 with SSB-Index 0 Table 4.6.3-120 with SSB-Index 0 |

Table 6.3.2.2.2.4.3-1: FrequencyInfoUL-SIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-2: RACH-ConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| rsrp-ThresholdSSB | RSRP_51 |  | Subtest 1 |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 15 |  |  |
|  | kHz 30 |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-3: CellGroupConfig for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| newUE-Identity | 1 |  |  |
| t304 | ms1000 |  |  |
| rach-ConfigDedicted |  |  |  |
| uplink | RACH-ConfigDedicated |  |  |
| } |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP SEQUENCE { |  |  |  |
| srs-Config | SRS-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-4: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 1 entry |  |  |
| ssb[1] | 0 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_51 |  | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-5: RACH-ConfigGeneric for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 102 |  |  |
| msg1-FDM | one |  |  |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -120 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 6.3.2.2.2.4.3-6: ServingCellConfigCommonSIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | -5 |  |  |
| } |  |  |  |

6.3.2.2.2.5	Test requirement

Table 6.3.2.2.2.5-1 defines the primary level settings for non-contention based random access test in FR1 for NR Standalone. Tables 6.3.2.2.2.5-2, 6.3.2.2.2.5-3 and 6.3.2.2.2.5-4 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 6.3.2.2.2.5-1: General test parameters for non-contention based random access test in FR1 for NR Standalone

| Parameter |  |  | Unit | Subtest 1 | Subtest 2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | As defined in A.3.10, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 2 |  | SSB.2 FR1 | SSB.2 FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | 2 | Different from the definition in A.3.10 |
| SS/PBCH block index |  |  |  | 0,1 | 0,1 | Different from the definition in A.3.10 |
| CSI-RS Configuration |  | Config 1 |  | N/A | CSI-RS.1.1 FDD | As defined in A.3.1.4 |
|  |  | Config 2 |  |  | CSI-RS.2.1 TDD |  |
| Duplex Mode for Cell 2 |  | Config 1 |  | FDD | FDD |  |
|  |  | Config 2 |  | TDD | TDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.2.1 | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OCNG pattern 1 | OCNG pattern 1 | As defined in A.3.2.1. |
| PDSCH parameters Note 4 |  | Config 1 |  | SR.1.1 FDD | SR.1.1 FDD | As defined in A.3.1.1. |
|  |  | Config 2 |  | SR.2.1 TDD | SR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  | dB |  |  |  |
| SSB with index 0 |  |  | dB | 3 | 3 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 2 |  | -101 | -101 |  |
|  |  |  | dB | 3 | 3 |  |
|  | SS-RSRP Note 3 | Config 1 | dBm/15kHz | -95 | -95 |  |
|  |  | Config 2 |  | -98 | -98 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -95 | -95 |  |
| SSB with index 1 |  |  | dB | -17 | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | Config 1 | dBm/15kHz | -98 | -98 |  |
|  |  | Config 2 |  | -101 | -101 |  |
|  |  |  | dB | -17 | -17 |  |
|  | SS-RSRP Note 3 | Config 1 | dBm/15kHz | -115 | -115 |  |
|  |  | Config 2 |  | -118 | -118 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 | -115 |  |
| Io Note 2 |  | Config 1 | dBm | -65.3/9.36MHz | -65.3/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 2 |  | -62.2/38.16MHz | -62.2/38.16MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | FR1 PRACH configuration 2 | FR1 PRACH configuration 3 | As defined in A.3.8. |
| Propagation Condition |  |  | - | AWGN | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: Es/Iot, SS-RSRP and Io level have been derived from other parameters for information purpose. They are not settable parameters. Note 3: Void. Note 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |  |

Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

-	The Random Access Preamble shall have the Preamble Index associated with the CSI-RS configured.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured.

-	The selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

Test 3: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.2.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.2.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.2.5-4.

Test 4: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be -22 dBm within the accuracy specified in Table 6.3.2.2.2.5-2.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 6.3.2.2.2.5-3.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 6.3.2.2.2.5-4.

Table 6.3.2.2.2.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 11.1 dB |

Table 6.3.2.2.2.5-3 Relative power tolerance Test requirements

| Power step P (Up or down) (dB) | PRACH (dB) |
| --- | --- |
| 2 ≤ ΔP < 3 | ± 3.2 |

Table 6.3.2.2.2.5-4: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 880*Tc |
|  | 30 | 30 | 624*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

#### 6.3.2.3	RRC connection release with redirection

##### 6.3.2.3.0	Minimum conformance requirements

6.3.2.3.0.1	Minimum conformance requirements for FR1 RRC connection release with redirection

The UE shall be capable of performing the RRC connection release with redirection to the target NR cell within Tconnection_release_redirect_NR.

The time delay (Tconnection_release_redirect_NR) is the time between the end of the last slot containing the RRC command, “RRCRelease” (TS 38.331 [2]) on the NR PDSCH and the time the UE starts to send random access to the target NR cell. The time delay (Tconnection_release_redirect_NR) shall be less than:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH

The target NR cell shall be considered detetable when for each relevant SSB, the side conditions should be met that,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.5 for a corresponding NR Band.

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCRelease” as defined in clause 6.2.2 of TS 38.331 [2].

Tidentify-NR: It is the time to identify the target NR cell and depend on the frequency range (FR) of the target NR cell. It is defined in table 6.3.2.3.0.1-1. Note that Tidentify-NR = TPSS/SSS-sync + Tmeas, in which TPSS/SSS-sync is the cell search time and Tmeas is the measurement time due to cell selection criteria evaluation.

TSI-NR: It is the time required for acquiring all the relevant system information of the target NR cell. This time depends upon whether the UE is provided with the relevant system information of the target NR cell or not by the old NR cell before the RRC connection is released. TSI-NR = 0 provided the UE is provided with the SI (including MIB and all relevant SIBs) of the target NR cell before the RRC connection is released by the old NR cell.

TRACH: It is the delay caused due to the random access procedure when sending random access to the target NR cell. This delay depends on the PRACH configuration defined in Table 6.3.3.2-2 [6] or Table 6.3.3.2-3 [6] for FR1 and in Table 6.3.3.2-4 [6] for FR2.

Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then:

-	the requirement in this section is applied with Trs = 20 ms assuming the SSB transmission periodicity is not larger than 20 ms,

-	there is no requirement if the SSB transmission periodicity is larger than 20ms.

Table 6.3.2.3.0.1-1: Time to identify target NR cell for RRC connection release with redirection to NR

| Frequency range (FR) of target NR cell | Tidentify-NR |
| --- | --- |
| FR1 | MAX (680 ms, [11] x Trs) |
| FR2 | MAX (880 ms, 8x[11] x Trs) |
| Note : If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the redirection command, SMTC follows smtc1 or smtc2 according to the physical cell ID of the target cell. |  |

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.3.2.1.

6.3.2.3.0.2	Minimum conformance requirements for FR1 – E-UTRAN RRC connection release with redirection

The UE shall be capable of performing the RRC connection release with redirection to the target E-UTRAN cell within Tconnection_release_redirect_E-UTRA.

The time delay (Tconnection_release_redirect_E-UTRA) is the time between the end of the last slot containing the RRC command, “RRCRelease” (TS 38.331 [2]) on the PDSCH and the time the UE starts to send random access to the target E-UTRA cell. The time delay (Tconnection_release_redirect_E-UTRA) shall be less than:

Tconnection_release_redirect_E-UTRA = TRRC_procedure_delay + Tidentify-E-UTRA + TSI-E-UTRA + TRACH

The target E-UTRA FDD or TDD cell shall be considered detectable when for each relevant SSB:

-	RSRP related conditions in the accuracy requirements in Section 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2 and Annex B.3 of TS 36.133 [15],

-	RSRQ related conditions in the accuracy requirements in Section 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2 and Annex B.3 of TS 36.133 [15],

-	RS-SINR related conditions in the accuracy requirements in Section 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2 and Annex B.3 of TS 36.133 [15].

TRRC_procedure_delay: It is the RRC procedure delay for processing the received message “RRCRelease” as defined in clause 6.2.2 of TS 38.331 [2].

Tidentify-E-UTRA: It is the time to identify the target E-UTRA cell. It shall be less than 320 ms.

TSI-E-UTRA: It is the time required for acquiring all the relevant system information of the target E-UTRA cell. This time depends upon whether the UE is provided with the relevant system information (SI) of the target E-UTRA cell or not by the old NR cell before the RRC connection is released. TSI-E-UTRA = 0 provided the UE is provided with the SI (including MIB and all relevant SIBs) of the target E-UTRA cell before the RRC connection is released by the old NR cell.

TRACH: It is the delay caused due to the random access procedure when sending random access to the target E-UTRA cell.

The normative reference for this requirement is TS 38.133 [6] clauses 6.2.3.2.2.

##### 6.3.2.3.1	NR SA FR1 RRC connection release with redirection

6.3.2.3.1.1	Test purpose

This test is to verify RRC connection release with redirection from NR to NR.

6.3.2.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.2.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.3.0.1.

The normative reference for this requirement is TS 38.133 [6] A.6.3.2.3.1.

6.3.2.3.1.4	Test description

6.3.2.3.1.4.1	Initial conditions

This test can be run in the configurations defined in Table 6.3.2.3.1.4.1-1.

Table 6.3.2.3.1.4.1-1: Redirection from NR to NR test configurations

| Configuration | Description |
| --- | --- |
| 6.3.2.3.1-1 | Source cell: NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.3.2.3.1-2 | Source cell: NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.3.2.3.1-3 | Source cell: NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.3.1.4.1-2.

Table 6.3.2.3.1.4.1-2: Initial conditions for Redirection from NR to NR test case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameters for PCell and neighbour cell are given in Table 6.3.2.3.1.4.1-3 below.

2. Message contents are defined in clause 6.3.2.3.1.4.3.

3. There are two carriers and two NR cells specified in the test. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 6.3.2.3.1.4.1-3: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

6.3.2.3.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The “RRCRelease” message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.2.3.1.5-1. T1 starts.

3.	SS shall transmit an RRCRelease during period T1.

4.	The SS shall switch the power setting from T1 to T2 as specified in Table 6.3.2.3.1.5-1. When the last TTI containing the RRCRelease message is sent to UE, T2 starts.

5.	If the UE transmits the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

6.	After T2 expires, the UE shall be switched off. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell and Cell 2 shall be powered OFF.

7.	The SS shall set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

8.	Repeat step 2-7 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

6.3.2.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.3.2.3.1.4.3-1: RRCRelease for NR RRC redirection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |  |
| nr SEQUENCE { |  |  |  |  |
| carrierFreq |  | ARFCN-ValueNR | Frequency of Cell 2 |  |
| ssbSubcarrierSpacing |  | kHz15 |  | Config 1,2 |
|  |  | kHz30 |  | Config 3 |
| smtc SEQUENCE { |  |  |  |  |
| duration |  | sf1 |  | SMTC.1 |
|  |  | sf5 |  | SMTC.2 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.3.2.3.1.4.3-2: SIB2

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |

6.3.2.3.1.5	Test requirement

Table 6.3.2.3.1.5-1 defines the primary level settings for Redirection from NR to NR test case.

Table 6.3.2.3.1.5-1: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR 2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR 2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -64.59 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 2240 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = 110 ms and is specified in clause 12 in TS 38.331 [13].

Tidentify-NR = 680 ms in the test.

TSI-NR = 1280 ms is assumed, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH = 170 ms in the test.

This gives a total of 2240 ms.

##### 6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection

6.3.2.3.2.1	Test purpose

This test is to verify RRC connection release with redirection from NR to E-UTRAN.

6.3.2.3.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.3.2.3.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.3.2.3.0.2.

The normative reference for this requirement is TS 38.133 [6] A.6.3.2.3.2.

6.3.2.3.2.4	Test description

6.3.2.3.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 6.3.2.3.2.4.1-1.

Table 6.3.2.3.2.4.1-1: Redirection from NR to E-UTRAN test configurations

| Configuration | Description |
| --- | --- |
| 6.3.2.3.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.3.2.3.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.3.2.3.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.3.2.3.2-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.3.2.3.2-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.3.2.3.2-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.3.2.3.2.4.1-2.

Table 6.3.2.3.2.4.1-2: Initial conditions for Redirection from NR to E-UTRAN test case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.2.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameters settings are given in Table 6.3.2.3.2.4.1-3 below.

2. Message contents are defined in clause 6.3.2.3.2.4.3.

3. There are two cells specified in the test. Cell 1 is the NR PCell and Cell 2 is the E-UTRAN neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 6.3.2.3.2.4.1-3: General test parameters for Redirection from NR to E-UTRAN test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2.3 |  |

6.3.2.3.2.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The “RRCRelease” message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

2.	Set the parameters according to T1 in Table 6.3.2.3.2.5-1 and 6.3.2.3.2.5-2. T1 starts.

3.	SS shall transmit an RRCRelease during period T1.

4.	The SS shall start T2 timer when the last TTI containing the RRCRelease message is sent to UE.

5.	When T2 starts, the SS shall switch the power setting from T1 to T2 as specified in Table 6.3.2.3.2.5-1 for Cell 1 and Table 6.3.2.3.2.5-2 for Cell 2.

6.	The UE shall transmit RRCConnectionRequest message.

7.	If the UE transmits the PRACH to Cell 2 less than 2205 ms from the beginning of time period T2 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T2 expires, the UE shall be switched off. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell and Cell 2 shall be powered OFF.

9.	The SS shall set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14+2) for next iteration of the test procedure loop.

10.	Repeat step 2-9 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

6.3.2.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.3.2.3.2.4.3-1: RRCRelease for Inter-RAT RRC redirection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |  |
| eutra SEQUENCE { |  |  |  |  |
| eutraFrequency |  | ARFCN-ValueEUTRA | Frequency of Cell 2 |  |
| cnType |  | epc |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.3.2.3.2.5	Test requirement

Table 6.3.2.3.2.5-1 and 6.3.2.3.2.5-2 define the primary level settings for Redirection from NR to E-UTRAN test case.

Table 6.3.2.3.2.5-1: Cell specific test parameters for Redirection from NR to E-UTRAN test case(Cell 1)

| Parameter |  |  | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |
| RF Channel Number |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |
| BWchannel |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |
| BWP BW |  | Config 1,4 | MHz | 10: NRB,c = 52 |  |  |
|  |  | Config 2,5 |  | 10: NRB,c = 52 |  |  |
|  |  | Config 3,6 |  | 40: NRB,c = 106 |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD |  |  |
|  |  | Config 2,5 |  | SR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |  |
|  |  | Config 2,5 |  | CR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | CR2.1 TDD |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |
| SMTC configuration |  | Config 1,2,4,5 |  | SMTC.1 FR1 |  |  |
|  |  | Config 3,6 |  | SMTC.2 FR1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 kHz |  |  |
|  |  | Config 3,6 |  | 30 kHz |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 kHz |  |  |
|  |  | Config 3,6 |  | 30 kHz |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -98 |  |  |
|  | Config 3,6 |  |  | -95 |  |  |
|  |  |  | dB | 4 | 4 |  |
|  |  |  | dB | 4 | 4 |  |
| IoNote3 | Config 1,2,4,5 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  |
|  | Config 3,6 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.3.2.3.2.5-1: Cell specific test parameters for Redirection from NR to E-UTRAN test case(Cell 2)

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |
|  |  | 4, 5, 6 | 53 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 4 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 4 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -94 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -94 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -70.22 | -64.76 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211. Note 3: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 2205 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to E-UTRAN observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_E-UTRA = TRRC_procedure_delay + Tidentify-E-UTRA + TSI-E-UTRA + TRACH,

where:

TRRC_procedure_delay = 110 ms and is specified in clause 12 in TS 38.331 [2].

Tidentify- E-UTRA = 800 ms in the test.

TSI- E-UTRA = 1280 ms is assumed, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target E-UTRAN cell.

TRACH = 15 ms in the test.

This gives a total of 2205 ms.

## 6.4	Timing

### 6.4.1	UE transmit timing

#### 6.4.1.0	Minimum conformance requirements

#### 6.4.1.0	Minimum conformance requirements

##### 6.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 6.4.1.0.1-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS or it is the PRACH transmission.

The UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table 6.4.1.0.1-2.

Table 6.4.1.0.1-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 6.4.1.0.1-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD band without LTE-NR coexistence case or FR1 TDD band without LTE-NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with LTE-NR coexistence case | 0 (Note 1) |
| FR1 TDD band with LTE-NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies based on the information n-TimingAdvanceOffset according to [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to section 4.2 in [3] and the value 39936 of can also be provided for a FDD serving cell. |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied.

When the transmission timing error between the UE and the reference timing exceeds Te, the UE is required to adjust its timing to within Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp  per second.

3)	The maximum aggregate adjustment rate shall be Tq per [200]ms.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 6.4.1.0.1-3.

Table 6.4.1.0.1-3: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (KHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2 | 60 | 2.5*64*Tc | 2.5*64*Tc |
|  | 120 | 2.5*64*Tc | 2.5*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

The normative reference for this requirement is TS.38.133 [6] clause 7.1.2.

#### 6.4.1.1	NR SA FR1 UE transmit timing accuracy

6.4.1.1.1	Test purpose

The purpose of this test is to verify that the UE can follow frame timing change of the connected gNodeb and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in TS 38.133 [6] clause 7.1.2.

6.4.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.4.1.0.1.

The normative reference for this requirement is TS.38.133 [6] clause A.6.4.1.1

6.4.1.1.4	Test Description

6.4.1.1.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1 [17].

This test can be run in one of the configurations defined in Table 6.4.1.1.4.1-1.

Table 6.4.1.1.4.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 6.4.1.1-1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 6.4.1.1-2 | NR TDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 6.4.1.1-3 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations in FR1 depending on UE capability. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.4.1.1.4.1-2

Table 6.4.1.1.4.1-2: Initial conditions for NR SA FR1 transmit timing accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.4.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without LTE link |  |  |

1. Message contents are defined in clause 6 6.4.1.1.4.3.

2. A single NR cell is used. The connection setup is done according to the settings in Annex C.1.3, and the downlink signal levels as per Annex C.1. The general test parameters are given in Table 6.4.1.1.5-1 below.

6.4.1.1.4.2	Test procedure

The test consists of a single NR cell (PCell). The downlink timing of the NR Cell is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting SRS used as a measurement reference facilitating the SS timing estimation.

The test sequence shall be carried out in RRC_CONNECTED for every test case.

Following will be the test sequence for this test

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [6] clause 4.5.

2.	Set up NR Cell according to parameters given in Table 6.4.1.1.4.1-1.

3.	The SS shall transmit an RRCReconfiguration message configuring the UE with the message content defined in clause 6.4.1.1.4.3.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	After connection set up with the cell and during 2 seconds before DL timing adjustment, the test equipment shall monitor all SRS transmisisons and verify that, for each received SRS, the timing of the NR cell is within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB.

a.	The NTA offset value (in Tc units) is 25600 for FR1

b.	The Te values depend on the DL and UL SCS for which the test is being run and are given in Table 6.4.1.1.5-4

6.	The test system shall adjust the timing of the DL path by values given in Table 6.4.1.1.4.2-1. For Test 2, the DL timing change shall be applied within the first half of the DRX cycle upon expiration of the preceding DRX ON duration.

Table 6.4.1.1.4.2-1: Adjustment Value for DL Timing

| SCS of SSB signals (KHz) | Adjustment Value |  |
| --- | --- | --- |
|  | Test1 | Test2 |
| 15 | +64*64Tc | +32*64Tc |
| 30 | +32*64Tc | +16*64Tc |

7.	The test system shall verify that the adjustment step size and the adjustment rate shall be according to requirements specified in Table 6.4.1.1.5-5. This will only be done for Test1. The test system samples the UE Transmit Timing once per SRS transmission (as per configured SRS periodicity). To check Rule 1, the SS shall check that the maximum time adjustment step size Tq between one SRS transmission to next consecutive SRS transmission of a valid UL slot is within Rule 1 as specified in clause 6.4.1.0.1 and Table 6.4.1.0.1-3. To check that the minimum adjustment rate is within Rule 2 as specified in clause 6.4.1.0.1 and Table 6.4.1.0.1-3, the SS shall measure the change in SRS transmission timing over a 1 + offset seconds sliding window (offset in ms to the next consecutive SRS transmission), with step size p (where p is the periodicity of SRS) , as long as the resulting slot is a valid UL slot. To check that the maximum adjustment rate is within Rule 3 as specified in clause 6.4.1.0.1 and Table 6.4.1.0.1-3, the SS shall measure the change in SRS transmission timing over a 200ms – offset sliding window of previous SRS transmission, with step size p (where p is the periodicity of SRS) , as long as the resulting slot is a valid UL slot. The three rules apply until the UE transmit timing offset is within the limits specified in 6.4.1.0.1 and Table 6.4.1.0.1-3 with respect to the first detected path (in time) of the corresponding downlink frame of Cell 1. The test system will wait till evaluation interval of T seconds is met to ensure UE transmit timing is stable at the end of the step, where T=.DL_timing_change[Ts]/5.5Ts and DL_timing_change is specified in Table 6.4.1.1.4.2-1.

8.	After the UE transmit timing is within the limits specified in step 7, and during 2 seconds, the test system shall monitor all SRS transmissions and verify that, for each received SRS, the UE transmit timing offset stays within (NTA + NTA_offset) ×Tc ± Te of the first detected path of DL SSB. For Test 2 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

6.4.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.4.1.1.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.9-1 |

Table 6.4.1.1.4.3-1: SRS-Config : Additional test requirement for UE transmit timing accuracy for NR SA FR1 UE

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-182 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SEQUENCE { |  |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SEQUENCE { |  |  |  |
| SRS-Resource[1] SEQUENCE { |  | entry 1 |  |
| freqHopping SEQUENCE { |  |  |  |
| c-SRS | 1 |  |  |
| } |  |  |  |
| groupOrSequenceHopping | neither |  |  |
| resourceType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| periodicityAndOffset-p CHOICE { |  |  |  |
| sl1 | 0 |  | Test 1 |
| sl320 | 3 |  | Test 2 and SCS15 |
| sl640 | 5 |  | Test 2 and SCS30 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.4.1.1.4.3-2: DRX-Config : Additional test requirement for UE transmit timing accuracy Test 2 for NR SA FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= CHOICE { |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms320 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

6.4.1.1.5	Test Requirements

Table 6.4.1.1.5-1: Cell Specific Test Parameters for UL Transmit Timing test

| Parameter | Unit | Config | Test1 |  | Test2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2,3 | Freq1 |  | Freq1 |  |
| NR Channel Number |  | 1,2,3 | 2 |  | 2 |  |
| TDD configuration |  | 1 | Not Applicable |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |  |  |  |
|  |  | 2 | 10: NRB,c = 52 |  |  |  |
|  |  | 3 | 40: NRB,c = 106 |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| DRx Cycle | ms | 1,2,3 | N/A |  |  | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |  |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  |
| SMTC |  | 1 | SMTC.1 |  |  |  |
|  |  | 3 | SMTC.2 |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3 | -98 |  | -98 |  |
| Note2 | dBm/SCS | 1,2 | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  |
|  |  | 1,2,3 | 3.3 |  | 3.3 |  |
|  |  | 1,2,3 | 3.3 |  | 3.3 |  |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -95 |  | -95 |  |
|  |  | 3 | -92 |  | -92 |  |
| IoNote3 | dBm/9.36MHz | 1,2 | -65.08 |  | -65.08 |  |
|  | dBm/38.1MHz | 3 | -61.99 |  | -61.99 |  |
| Propagation condition |  | 1,2,3 | AWGN |  |  |  |
| SRS Config |  | 1 | SRSConf.1Note6 | SRSConf.3Note6 |  |  |
|  |  | 2,3 | SRSConf.1Note6 | SRSConf.2Note6 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRx related parameters are given in Table 6.4.1.1.5-3 or Table A.5.-1 Note 6: SRS configs are given in Table 6.4.1.1.5-2 |  |  |  |  |  |  |

Table 6.4.1.1.5-2: SRS Configuration for Timing Accuracy Test

|  | Field | SRSConf.1 | SRSConf.2 | SRSConf.3 | Comments |
| --- | --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 | 0 |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook | Codebook |  |
|  | SRS-ResourceSetId | 0 | 0 | 0 |  |
| SRS-Resource | nrofSRS-Ports | Port1 | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 | 0 |  |
|  | freqHopping c-SRS | sl1 | sl1 | sl1 |  |
|  | freqHopping b-SRS | 0 | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither | Neither |  |
|  | resourceType | Periodic | Periodic | Periodic |  |
|  | periodicityAndOffset-p | sl1 | sl640,5 | sl320,3 | Offset to align with DRx periodicity |
|  | sequenceId | 0 | 0 | 0 | Any 10 bit number |

Table 6.4.1.1.5-3: DRX-Configuration for UL Timing Tests

| Field | Test 2 |
| --- | --- |
|  | Value |
| drx-onDurationTimer | 6 ms |
| drx-InactivityTimer | 1 ms |
| drx-RetransmissionTimerDL | 1 slot |
| drx-RetransmissionTimerUL | 1 slot |
| longDRX-CycleStartOffset | 320 ms |
| shortDRX | disable |
| TimeAlignmentTimer | Infinity |
| Note: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [13] |  |

Table 6.4.1.1.5-4: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (KHz) | SCS of uplink signals (KHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 13.75*64*Tc |
|  |  | 30 | 11.75*64*Tc |
|  |  | 60 | 11.75*64*Tc |
|  | 30 | 15 | 9.75*64*Tc |
|  |  | 30 | 9.75*64*Tc |
|  |  | 60 | 8.75*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 6.4.1.1.5-5: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (KHz) | Tq | Tp | Maximum Adjustement Rate |
| --- | --- | --- | --- | --- |
| 1 | 15 | 6.0*64*Tc | 1.9*64*Tc | 6.6*64*Tc |
|  | 30 | 6.0*64*Tc | 1.9*64*Tc | 6.6*64*Tc |
|  | 60 | 6.0*64*Tc | 1.9*64*Tc | 6.6*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |  |

### 6.4.2	UE timer accuracy

### 6.4.3	Timing advance

#### 6.4.3.0	Minimum conformance requirement

##### 6.4.3.0.1	Minimum conformance requirement for timing advance adjustment

The UE shall adjust the timing of its transmissions with a relative accuracy better than or equal to the UE Timing Advance adjustment accuracy requirement in Table 6.4.3.1.3-1, to the signalled timing advance value compared to the timing of preceding uplink transmission. The timing advance command step is defined in TS38.213 [8].

Table 6.4.3.1.3-1: UE Timing Advance adjustment accuracy

| Sub Carrier Spacing, SCS kHz | 15 | 30 | 60 | 120 |
| --- | --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±256 Tc | ±256 Tc | ±128 Tc | ±32 Tc |

#### 6.4.3.1	NR SA FR1 timing advance adjustment accuracy

6.4.3.1.1	Test purpose

The purpose of the test is to verify UE timing advance adjustment delay and accuracy requirement defined in clause 7.3 of TS 38.133 [6].

6.4.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.4.3.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.4.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.4.3.1.

6.4.3.1.4	Test description

6.4.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.4.3.1.4.1-1.

Table 6.4.3.1.4.1-1: NR SA FR1 timing advance adjustment accuracy supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.4.3.1.4.1-2

Table 6.4.3.1.4.1-2: Initial conditions for NR SA FR1 timing advance adjustment accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.4.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without LTE Link |  |  |

Table 6.4.3.1.4.1-3: General test parameters for timing advance

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF channel number |  | 1 |  |
| Initial DL BWP |  | DLBWP.0.1 | As specified in Table A.8.1-1 |
| Dedicated DL BWP |  | DLBWP.1.1 | As specified in Table A.8.1-2 |
| Initial UL BWP |  | ULBWP.0.1 | As specified in Table A.8.2-1 |
| Dedicated UL BWP |  | ULBWP.1.1 | As specified in Table A.8.2-2 |
| Timing Advance Command (TA) value during T1 |  | 31 | NTA_new = NTA_old for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  | 39 | For SCS = 15kHz : NTA_new = NTA_old + 8192*Tc (based on equation in TS38.213 section 4.2) For SCS = 30kHz : NTA_new = NTA_old + 4096*Tc (based on equation in TS38.213 section 4.2) |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

1. Message contents are defined in clause 6.4.3.1.4.3.

2. Single Cell is used, which is NR FR1 PCell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

3. The test parameters are given in Table 6.4.3.1.4.1-3 above.

4. Downlink signals for NR cell are initially set up according to Annex C.1.2 and C.1.3.

5. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [10] clause 4.5.

6.4.3.1.4.2	Test Procedure

The test consists of single cell. The test consists of two successive time periods, with time durations of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and Sounding Reference Signals (SRS), as specified in table 6.4.3.1.5-1 and table 6.4.3.1.4.1-3, are sent from the UE and received by the test equipment. By measuring the reception of the SRS, the transmit timing, and hence the timing advance adjustment accuracy, can be measured. The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321, shall be configured so that it does not expire in the duration of the test.

1. Set the parameters according to values in Tables 6.4.3.1.4.1-3.

2. SS shall transmit an RRCReconfiguration message.

3. The UE shall transmit RRCReconfigurationComplete message.

4. During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.4 in TS 38.321. The Timing Advance Command value shall be set to 31, which according to Clause 4.2 in TS 38.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

5. During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value 39 as specified in table 6.4.3.1.4.1-3. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the SRS sent from the UE.

6. As specified in Clause 7.3.2.1 of TS 38.133 [6], the UE adjusts its uplink timing at slot n+k+1 for a timing advance command received in slot n. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the SRS sent from the UE.

7. The UE Time Alignment Timer, described in Clause 5.2 in TS 38.321, shall be configured so that it does not expire in the duration of the test.

8. The result from the SRS and adjustment of the timing advance in step 7) is used to measure that the UE adjusts the timing of its transmission with a relative accuracy better than or equal to value specified in Table 6.4.3.1.3-1 to the signalled timing advance value compared to the timing of preceding uplink transmission.

9. If the UE adjust the timing of its transmission within a relative accuracy greater than or equal to value specified in Table 6.4.3.1.3-1 to the signalled timing advance value compared to the timing of preceding uplink transmission then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

10. The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

11. After the RRC connection release, the SS transmits in Cell a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in State RRC_CONNECTED according to TS 38.508-1 [14] clause 4.5.4 or if the paging fails, switch off and on the UE and ensure the UE is in RRC_CONNECTED according to TS 38.508-1 [14] clause 4.5.4.

12. Repeat step 2-11 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.4.3.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 7.3, with the following exceptions:

Table 6.4.3.1.4.3-0: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.9-1 |

Table 6.4.3.1.4.3-1: srs-Config setup

| Derivation Path: TS 38.508-1, Table 4.6.3-182 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment |  | Condition |
| SRS-Config ::= SEQUENCE { |  |  |  |  |
| srs-ResourceSetToAddModList SEQUENCE (SIZE(0..maxNrofSRS-ResourceSets)) OF SEQUENCE { | 1 entry |  |  |  |
| SRS-ResourceSet[1] SEQUENCE { |  |  | entry 1 |  |
| resourceType CHOICE { |  |  |  |  |
| periodic SEQUENCE { |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| srs-ResourceToAddModList SEQUENCE (SIZE(1..maxNrofSRS-Resources)) OF SEQUENCE { | 1 entry |  |  |  |
| SRS-Resource[1] SEQUENCE { |  |  | entry 1 |  |
| freqHopping SEQUENCE { |  |  |  |  |
| c-SRS | 12 | Config 1,2 |  |  |
|  | 24 | Config 3 |  |  |
| } |  |  |  |  |
| groupOrSequenceHopping | neither |  |  |  |
| resourceType CHOICE { |  |  |  |  |
| periodic SEQUENCE { | periodic |  |  |  |
| periodicityAndOffset-p CHOICE { |  |  |  |  |
| sl5 | 2 | Once every 5 Slots |  | SCS15 |
| sl5 | 4 | Once every 5 Slots |  | SCS30 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.4.3.1.5	Test Requirement

The UE shall apply the signalled Timing Advance value to the transmission timing at the designated activation time i.e. k slots after the reception of the timing advance command, where:

k = 5 for Config 1, 2, and 3

The Timing Advance adjustment accuracy shall be within the limits specified in Table 6.4.3.1.5-3.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

Table 6.4.3.1.5-1 and Table 6.4.3.1.5-2 define the primary level settings.

Table 6.4.3.1.5-1: Cell specific test parameters for timing advance

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| Duplex mode |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DRx Cycle |  |  | ms | Not Applicable |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |
|  |  | Config 3 |  | 30 kHz |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |
|  |  | Config 3 |  | 30 kHz |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |
|  | Config 3 |  |  | -95 |  |
|  |  |  | dB | 3 |  |
|  |  |  | dB | 3 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -67.57 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -62.58 |  |
| Propagation condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 6.4.3.1.5-2: Sounding Reference Symbol Configuration for timing advance

| Field |  | Value | Comment |
| --- | --- | --- | --- |
| c-SRS | Config 1,2 | 12 | Frequency hopping is disabled |
|  | Config 3 | 24 |  |
| b-SRS |  | 0 |  |
| b-hop |  | 0 |  |
| freqDomainPosition |  | 0 | Frequency domain position of SRS |
| freqDomainShift |  | 0 |  |
| groupOrSequenceHopping |  | neither | No group or sequence hopping |
| SRS-PeriodicityAndOffset |  | sl5@2 for SCS 15kHz sl5@4 for SCS 30kHz | Once every 5 slots |
| pathlossReferenceRS |  | ssb-Index=0 | SSB #0 is used for SRS path loss estimation |
| usage |  | Codebook | Codebook based UL transmission |
| startPosition |  | 0 | resourceMapping setting. SRS on last symbol of slot, and 1symbols for SRS without repetition. |
| nrofSymbols |  | n1 |  |
| repetitionFactor |  | n1 |  |
| combOffset-n2 |  | 0 | transmissionComb setting |
| cyclicShift-n2 |  | 0 |  |
| nrofSRS-Ports |  | port1 | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 38.331. |  |  |  |

Table 6.4.3.1.5-3: UE Timing Advance adjustment accuracy

| Sub Carrier Spacing, SCS kHz | 15 | 30 | 60 |
| --- | --- | --- | --- |
| UE Timing Advance adjustment accuracy | ±344 Tc | ±344 Tc | ±216 Tc |

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

## 6.5	Signaling characteristics

### 6.5.1	Radio link monitoring

The UE shall monitor the downlink link quality based on the reference signal in the configured RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell and PCell as specified in [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP.

On each RLM-RS resource, the UE shall estimate the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell.

The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate (BLERout) as defined in Table 6.5.1-1.

The threshold Qin is defined as the level at which the downlink radio link quality can be significantly more reliably received than at Qout and shall correspond to the in-sync block error rate (BLERin) as defined in Table 6.5.1-1.

The out-of-sync block error rate (BLERout) and in-sync block error rate (BLERin) are determined from the network configuration via parameter rlmInSyncOutOfSyncThreshold signalled by higher layers. When UE is not configured with RLM-IS-OOS-thresholdConfig from the network, UE determines out-of-sync and in-sync block error rates from Configuration #0 in Table 6.5.1-1 as default. All requirements here are applicable for BLER Configuration #0 in Table 6.5.1-1.

Table 6.5.1-1: Out-of-sync and in-sync block error rates

| Configuration | BLERout | BLERin |
| --- | --- | --- |
| 0 | 10% | 2% |

UE shall be able to monitor up to XRLM-RS RLM-RS resources of the same or different types in each corresponding carrier frequency range, where XRLM-RS is specified in Table 6.5.1-2, and meet the requirements as specified in this section.

Table 6.5.1-2: Maximum number of RLM-RS resources XRLM-RS

| Maximum number of RLM-RS resources, XRLM-RS | Carrier frequency range of PCell/Pcell |
| --- | --- |
| 2 | FR1, ≤ 3 GHz |
| 4 | FR1, > 3 GHz |
| 8 | FR2 |

If different SCS is used for CSI-RS based RLM-RS and SSB, then CSI-RS based RLM-RS and SSB shall be TDMed. If same SCS is used for CSI-RS based RLM-RS and SSB, then CSI-RS based RLM-RS and SSB can be FDMed or TDMed.

Any uplink signal transmitted by the UE is used for detecting the In-/Out-of-Sync state of the UE. In terms of measurement, the uplink signal is verified on the basis of the UE output power:

For intra-band contiguous carrier aggregation, transmit OFF power is measured as the mean power per component carrier.

For UE with multiple transmit antennas, transmit OFF power is measured as the mean power at each transmit connector.

-	UE output power higher than Transmit OFF power -50 dBm (as defined in TS 38.101-3 [4]) means uplink signal

-	UE output power equal to or less than Transmit OFF power -50 dBm (as defined in TS 38.101-3 [4]) means no uplink signal.

#### 6.5.1.0	Minimum conformance requirements

##### 6.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period. The requirements in this section apply for each SSB based RLM-RS resource configured for PSCell, provided that the SSB configured for RLM is transmitted within UE active DL BWP during the entire evaluation period defined in Table 6.5.1.0.1-1.

TEvaluate_out_SSB is defined in Table 6.5.1.0.1-1 for FR1.

Table 6.5.1.0.1-1: Evaluation period TEvaluate_out for FR1

| Configuration | TEvaluate_out_SSB (ms) |
| --- | --- |
| no DRX | max(200,ceil(10*P)*TSSB) |
| DRX cycle≤320 | max(200,ceil(15*P)*max(TDRX,TSSB)) |
| DRX cycle>320 | ceil(10*P)*TDRX |
| NOTE: TSSB is the periodicity of SSB configured for RLM. |  |

For FR1,

-	P=1/(1 – TSSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

If the high layer in TS 38.331 [13] signaling of smtc2 is present, TSMTCperiod follows smtc2; otherwise TSMTCperiod follows smtc1.

Longer evaluation period would be expected if the combination of RLM-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, Layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers. A Layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [13].

The out-of-sync and in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 in TS 38.213 [8]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval.

If DRX is used, when the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each RLM-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each RLM-RS resource. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell.

When

- the UE transitions from a first configuration of RLM-RS resources to a second configuration of RLM-RS resources that is different from the first configuration,

or

- the UE transitions between DRX and no DRX or DRX cycle periodicity changes,

for each RLM-RS resource, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each RLM-RS resource present in the second configuration. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell.

When DRX is not used TIndication_interval is max(10ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in section 8.1.2 if the RLM-RS resource is SSB.

When DRX is used, TIndication_interval is max(10ms, 1.5*DRX_cycle_length, 1.5*TRLM-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms. Upon start of T310 timer as specified in TS 38.331 [13], the UE shall monitor the configured RLM-RS resources for recovery using the evaluation period and Layer 1 indication interval corresponding to the no DRX mode until the expiry or stop of T310 timer.

The transmitter power of the UE in the monitored cell shall be turned off within 40ms after expiry of T310 timer as specified in TS 38.331 [13].

There are no scheduling restrictions due to radio link monitoring performed with a same subcarrier spacing as PDSCH/PDCCH on FR1.

For UE which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UE which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to radio link monitoring based on SSB as RLM-RS.

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured for radio link monitoring.

The normative reference for this requirement is TS 38.133 [6] clauses 8.1.2, 8.1.4, 8.1.5, 8.1.6 and 8.1.7.

##### 6.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 6.5.1.0.2-1 for FR1.

For FR1,

-	P=1/(1 – TSSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Note: The overlap between CSI-RS RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.Longer evaluation period would be expected if the combination of RLM-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

The values of Mout and Min used in Table 6.5.1.2.3-1 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with Density =3.

Table 6.5.1.0.2-1: Evaluation period TEvaluate_out and TEvaluate_in for FR1

| Configuration | TEvaluate_out (ms) | TEvaluate_in (ms) |
| --- | --- | --- |
| no DRX | max(200, ceil(Mout×P)×TCSI-RS) | max(100, ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | max(200, ceil(1.5×Mout×P)× max(TDRX, TCSI-RS)) | max(100, ceil(1.5×Min×P)× max(TDRX, TCSI-RS)) |
| DRX > 320ms | ceil(Mout×P) × TDRX | ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource configured for RLM. TDRX is the DRX cycle length. |  |  |

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin, Layer 1 of the UE shall send an in-sync indication for the cell to the higher layers. A Layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [2].

The in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 in TS 38.213 [3]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval.

When DRX is not used TIndication_interval is max(10ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in section 8.1.2 of TS 38.133 [6] if the RLM-RS resource is SSB, or TCSI-RS specified later in this if the RLM-RS resource is CSI-RS.

When DRX is used, TIndication_interval is max(10ms, 1.5*DRX_cycle_length, 1.5*TRLM-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms. Upon start of T310 timer as specified in TS 38.331 [2], the UE shall monitor the configured RLM-RS resources for recovery using the evaluation period and Layer 1 indication interval corresponding to the no DRX mode until the expiry or stop of T310 timer.

When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each RLM-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each RLM-RS resource. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell.

When the UE transitions from a first configuration of RLM-RS resources to a second configuration of RLM-RS resources that is different from the first configuration, for each RLM-RS resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each RLM-RS resource present in the second configuration. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell.

The transmitter power of the UE in the monitored cell shall be turned off within 40ms after expiry of T310 timer as specified in TS 38.331 [13].

There are no scheduling restrictions due to radio link monitoring performed with a same subcarrier spacing as PDSCH/PDCCH on FR1.

For UE which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UE which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to radio link monitoring based on SSB as RLM-RS.

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on SSB symbols to be measured for radio link monitoring.

The normative reference for this requirement is TS 38.133 [6] clauses 8.1.2, 8.1.4, 8.1.5, 8.1.6, 8.1.7 and A.7.5.1.

##### 6.5.1.0.3	Minimum conformance requirements for out-of-sync and in-sync CSI-RS based RLM

[TS 38.133 clause 8.1.3.2]

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS [ms] period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS [ms] evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS [ms] period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS [ms] evaluation period.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in Table 8.1.3.2-1 for FR1.

For FR1,

-	P=1/(1 – TCSI-RS/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

If the high layer in TS 38.331 [2] signaling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

Note: The overlap between CSI-RS RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration. Longer evaluation period would be expected if the combination of RLM-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

The values of Mout and Min used in Table 8.1.3.2-1 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with Density =3.

Table 6.5.1.0.3-1: Evaluation period TEvaluate_out and TEvaluate_in for FR1

| Configuration | TEvaluate_out (ms) | TEvaluate_in (ms) |
| --- | --- | --- |
| no DRX | max(200, ceil(Mout×P)×TCSI-RS) | max(100, ceil(Min×P) × TCSI-RS) |
| DRX ≤ 320ms | max(200, ceil(1.5×Mout×P)× max(TDRX, TCSI-RS)) | max(100, ceil(1.5×Min×P)× max(TDRX, TCSI-RS)) |
| DRX > 320ms | ceil(Mout×P) × TDRX | ceil(Min×P) × TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource configured for RLM. TDRX is the DRX cycle length. |  |  |

[TS 38.133 clause 8.1.6]

When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, Layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers. A Layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [2].

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin, Layer 1 of the UE shall send an in-sync indication for the cell to the higher layers. A Layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [2].

The out-of-sync and in-sync evaluations for the configured RLM-RS resources shall be performed as specified in clause 5 in TS 38.213 [3]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval.

When DRX is not used TIndication_interval is max(10ms, TRLM-RS,M), where TRLM,M is the shortest periodicity of all configured RLM-RS resources for the monitored cell, which corresponds to TSSB specified in section 8.1.2 if the RLM-RS resource is SSB, or TCSI-RS specified in section 8.1.3 if the RLM-RS resource is CSI-RS.

In case DRX is used, TIndication_interval is max(10ms, 1.5*DRX_cycle_length, 1.5*TRLM-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms. Upon start of T310 timer as specified in TS 38.331 [2], the UE shall monitor the configured RLM-RS resources for recovery using the evaluation period and Layer 1 indication interval corresponding to the no DRX mode until the expiry or stop of T310 timer.

[TS 38.133 clause 8.1.5]

The transmitter power of the UE in the monitored cell shall be turned off within 40ms after expiry of T310 timer as specified in TS 38.331 [2].

The normative reference for this requirement is TS 38.133 [6] clause 8.1.3.2, 8.1.6 and 8.1.5.

#### 6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode

6.5.1.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell configured with SSB-based RLM RS in non-DRX mode. This test will partly verify the NR cell radio link monitoring requirements in TS 38.133 [6] section 8.1.2.

6.5.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.5.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.1.

6.5.1.1.4	Test description

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 6.5.1.1.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 1.

Figure 6.5.1.1.4-1: SNR variation for out-of-sync testing

6.5.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.1.1.4.1-1.

Table 6.5.1.1.4.1-1: NA SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 2 | TDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 3 | TDD, SSB SCS 30 KHz, data SCS 30KHz, BW 40MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.1.4.1-2.

Table 6.5.1.1.4.1-2: Initial conditions for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.7.4 for TE Part |  |  |

PDCCH transmission parameters are given in Table 6.5.1.1.4.1-3.

Table 6.5.1.1.4.1-3: PDCCH transmission parameters for out-of-sync

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (MHz) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

1. Message contents are defined in clause 6.5.1.1.4.3.

2. Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

3. The test parameters are given in Table 6.5.1.1.4.1-4 below.

4. Downlink signals for NR cell are initially set up according to Annex C.1.2 and C.1.3.

Table 6.5.1.1.4.1-4: General test parameters for FR1 out-of-sync testing in non-DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 KHz |
|  |  | Config 3 |  | 30 KHz |
| PRACH Configuration |  | Config 1, 2 |  | Table A.7.1-1, PRACH.1 FR1 |
|  |  | Config 3 |  | Table A.7.1-1, PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX |  |  |  | OFF |
| Gap pattern ID |  |  |  | gp0 |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1, 4 |  | CSI-RS.1.1 FDD |
|  |  | Config 2, 5 |  | CSI-RS.1.1 TDD |
|  |  | Config 3, 6 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  | TRS.1.1 FDD |
|  |  | Config 2, 5 |  | TRS.1.1 TDD |
|  |  | Config 3, 6 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.48 |
| T3 |  |  | s | 0.48 |
| D1 |  |  | s | 0.44 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

6.5.1.1.4.2	Test Procedure

There is one cell (Cell 1), which is the active NR cell, in the test. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. The UE shall be configured for periodic CQI reporting in PUCCH [format 1] with a reporting periodicity as mentioned in the above table 6.5.1.1.4.1-4.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

2.	The SS shall transmit an RRCReconfiguration message configuring the UE for inter-frequency measurements.

3.	The UE shall transmit RRCReconfigurationComplete message.

4.	Set the parameters according to T1 in Table 6.5.1.1.5-1. Propagation conditions are set according to Annex C.2.2. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.1.1.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.1.5-1. T3 starts.

7.	If the SS:
a) detects uplink power equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each subframe configured for CQI transmission (according to configured CQI periodicity on PUCCH [format 1]) during the period from time point A to time point B

and
b) does not detect any uplink power higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C (D1 after the start of T3) until T3 expires,
the number of successful tests is increased by one.

8.	Otherwise the number of failed tests is increased by one.

9.	When T3 expires the SS shall change the SNR value to T1 as specified in Table 6.5.1.1.5-1.

10.	If the UE has not re-established the connection in at least 1s, the UE is switched off and then on. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [10] clause 4.5.

11.	Repeat steps 4-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.1.1.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 and 7.3.1 with condition “Short_DCI” and with the following exceptions:

Table 6.5.1.1.4.3-0: Common Exception messages for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, GAP_NEEDED Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier center of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-1 Table H.3.1-6 Table H.3.5-4 Table H.3.5-9 |

Table 6.5.1.1.4.3-1: Void

Table 6.5.1.1.4.3-2: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

Table 6.5.1.1.4.3-3: Void

Table 6.5.1.1.4.3-4 SIB1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -53 | -106 is actual value in dBm (-53 * 2 dBm) | dBm/15kHz |
|  | -51 | -102 is actual value in dBm (-51 * 2 dBm) | dBm/30kHz |
| } |  |  |  |
| } |  |  |  |

6.5.1.1.5	Test Requirement

Table 6.5.1.1.5-1 defines the cell specific primary level settings.

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

Table 6.5.1.1.5-1: Cell specific test parameters for FR1 (Cell 2) for out-of-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1.9 | -6.1 | -15.9 |
|  | Config 2 |  | 1.9 | -6.1 | -15.9 |
|  | Config 3 |  | 1.9 | -6.1 | -15.9 |
| SNR on other channels and signals | Config 1, 2, 3 | dB | 1 |  |  |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.1.4-1. Note 5: The SNR values are specified for a UE with 2RX antennas connected under test. For a UE with 4RX antennas connected under test, the SNR during T3 from D.4.1.1, is -18dB-TT = -18.9dB (including test tolerances). |  |  |  |  |  |

Table 6.5.1.1.5-2: Measurement gap configuration for out-of-sync tests in non-DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note Ensure that RLM RS is partially overlapped with measurement gap |  |

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode

6.5.1.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects in sync for the purpose of monitoring downlink radio link quality of the PCell, when DRX is not used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.2.

6.5.1.2.2	Test applicability

This test applies to all types of NR UEs supporting Release 15 and forwared

6.5.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.2.

6.5.1.2.4	Test Description

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.1.2.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms.

Figure 6.5.1.2.4-1 - SNR variation for in-sync testing

6.5.1.2.4	Test Requirements

6.5.1.2.4.1	Initial Conditions

This test shall be tested using any of the test configurations in Table 6.5.1.2.4.1-1.

Table 6.5.1.2.4.1-1: Supported test configurations for FR1 PSCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 2 | TDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 3 | TDD, SSB SCS 30 KHz, data SCS 30KHz, BW 40MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.2.4.1-2.

Table 6.5.1.2.4.1-2: Initial conditions for SA FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.1, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

PDCCH transmission parameters are given in Table 6.5.1.2.4.1-3.

Table 6.5.1.2.4.1-3: PDCCH transmission parameters for in-sync

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (MHz) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

1. Message contents are defined in clause 6.5.1.2.4.3.

2. The power levels and settings for Cell 1 are set according to Annex A.6, Table A.6.1.1-1. The connection setup is done according to the settings in Annex C.1.3, and the downlink signal levels as per Annex C.1.2

3. The general test parameters are given in Table 6.5.1.2.4.1-4 below.

4. Downlink signals for NR cell are initially set up according to Annex C.1.

Table 6.5.1.2.4.1-4: General test parameters for FR1 in-sync testing in non-DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration |  |  | Config 1 |  | SSB.1 FR1 |
|  |  |  | Config 2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |
|  |  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |
|  |  |  | Config 3 |  | 30 KHz |
| PRACH Configuration |  |  | Config 1, 2 |  | Table A.7.1-1, PRACH.1 FR1 |
|  |  |  | Config 3 |  | Table A.7.1-1, PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX |  |  |  |  | OFF |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 1000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  |  | TRS.1.1 FDD |
|  |  | Config 2, 5 |  |  | TRS.1.1 TDD |
|  |  | Config 3, 6 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 0.24 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 0.88 |
| D1 |  |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

6.5.1.2.4.2	Test Procedure

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.1.2.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states.. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. The UE shall be configured for periodic CQI reporting in PUCCH [format 1] with a reporting periodicity as mentioned in the above table 6.5.1.2.4.1-4.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

2. Set the parameters according to T1 in Table 6.5.1.2.5-1 for subtest 1 and 2. Propagation conditions are set according to Annex C.2.2. T1 starts.

3. When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.1.2.5-1. T2 starts.

4. When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.2.5-1. T3 starts.

5. When T3 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.2.5-1. T4 starts.

6. When T4 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.2.5-1. T5 starts.

7. If the SS detects uplink power equal to or higher than the minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in the subframe according the configured CQI reporting mode (PUCCH 1-0) during the period from time point A to time point F (D1 after the start of time duration T5) the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

9. Repeat steps 2-7 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.1.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 and 7.3.1 with condition “Short_DCI” and with the following exceptions:

Table 6.5.1.2.4.3-0: Common Exception messages for NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.5-4 Table H.3.5-9 |

Table 6.5.1.2.4.3-1: Void

Table 6.5.1.2.4.3-2: RLF-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms1000 |  |  |
| n310 | n1 |  |  |
| t311 | ms1000 |  |  |
| n311 | n1 |  |  |
| } |  |  |  |

Table 6.5.1.2.4.3-3: Void

6.5.1.2.5	Test Requirement

The requirements in this section apply for each SSB based RLM-RS resource configured for the PCell, provided that the SSB configured for RLM are actually transmitted within UE active DL BWP during the entire evaluation period specified in section 6.5.1.2.3.

Table 6.5.1.2.5-1 defines the cell specific primary level settings.

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence interval of 95%.

Table 6.5.1.2.5-1: Cell specific test parameters for FR1 for in-sync radio link monitoring tests in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1.8 | -6.2 | -15.8 | -5.3e | 1.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
| SNR on other channels and signals | Config 1, 2, 3 | dB | 1 |  |  |  |  |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure 6.5.1.2.4-1. Note 5: The SNR values are specified for a UE with 2RX antennas connected under test. For a UE wiht 4RX antennas connected under test, the SNR during T3 and T4 from D.4.1.1 are -18.0-TT and -8.0-TT, which are -18.8dB and -8.8dB(including test tolerances) |  |  |  |  |  |  |  |

#### 6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode

6.5.1.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink radio link quality of the PCell configured with SSB-based RLM RS when DRX is used. This test will partly verify the NR cell radio link monitoring requirements in TS 38.133 [6] section 8.1.2.

6.5.1.3.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.5.1.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 6.5.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.3.

6.5.1.3.4	Test description

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 6.5.1.3.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms).

Figure 6.5.1.3.4-1: SNR variation for out-of-sync testing

6.5.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.1.3.4.1-1.

Table 6.5.1.3.4.1-1: NA SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 2 | TDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 3 | TDD, SSB SCS 30 KHz, data SCS 30KHz, BW 40MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.3.4.1-2.

Table 6.5.1.3.4.1-2: Initial conditions for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.3.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.7.4 for TE Part |  |  |

PDCCH transmission parameters are given in Table 6.5.1.3.4.1-3..

Table 6.5.1.3.4.1-3: PDCCH transmission parameters for out-of-sync

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI format | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (MHz) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

1. Message contents are defined in clause 6.5.1.3.4.3.

2. Single Cell is used, which is NR FR1 PCell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

3. The test parameters are given in Table 6.5.1.3.4.1-4 below.

4. Downlink signals for NR cell are initially set up according to Annex C.1.2 and C.1.3.

Table 6.5.1.3.4.1-4: General test parameters for FR1 out-of-sync testing in DRX mode

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |
| Active PCell |  |  |  | Cell 1 |
| RF Channel Number |  |  |  | 1 |
| Duplex mode |  | Config 1 |  | FDD |
|  |  | Config 2, 3 |  | TDD |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  | Config 1 |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.2.1 |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration |  | Config 1 |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  | Config 1, 2 |  | SMTC.1 |
|  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  | 15 KHz |
|  |  | Config 3 |  | 30 KHz |
| PRACH Configuration |  | Config 1, 2 |  | Table A.7.1-1, PRACH.1 FR1 |
|  |  | Config 3 |  | Table A.7.1-1, PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  | 0 |
| OCNG parameters |  |  |  | OP.1 |
| CP length |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  | 2 |
|  | Aggregation level |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  | dB | 4 |
|  | DMRS precoder granularity |  |  | REG bundle size |
|  | REG bundle size |  |  | 6 |
| DRX Configuration |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  | N.A. |
| Layer 3 filtering |  |  |  | Enabled |
| T310 timer |  |  | ms | 0 |
| T311 timer |  |  | ms | 1000 |
| N310 |  |  |  | 1 |
| N311 |  |  |  | 1 |
| CSI-RS configuration |  | Config 1 |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1, 4 |  | TRS.1.1 FDD |
|  |  | Config 2, 5 |  | TRS.1.1 TDD |
|  |  | Config 3, 6 |  | TRS.1.2 TDD |
| T1 |  |  | s | 0.2 |
| T2 |  |  | s | 0.68 |
| T3 |  |  | s | 0.68 |
| D1 |  |  | s | 0.64 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

6.5.1.3.4.2	Test Procedure

There is one cell (Cell 1), which is the active NR cell, in the test. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. The UE shall be configured for periodic CQI reporting in PUCCH [format 1] with a reporting periodicity as mentioned in the above table 6.5.1.3.4.1-4.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

2.	Set the parameters according to T1 in Table 6.5.1.3.5-1. Propagation conditions are set according to Annex C.2.2. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.1.3.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.3.5-1. T3 starts.

5.	If the SS:
a) detects uplink power equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each subframe configured for CQI transmission (according to configured CQI periodicity on PUCCH [format 1]) during the period from time point A to time point B

and
b) does not detect any uplink power higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C (D1 after the start of T3) until T3 expires,
the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

6.	When T3 expires the SS shall change the SNR value to T1 as specified in Table 6.5.1.3.5-1 for subtests 1 and 2.

7.	If the UE has not re-established the connection in at least 1s, the UE is switched off and then on. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [10] clause 4.5.

8.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.1.3.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 and 7.3.1 with condition “Short_DCI” and with the following exceptions:

Table 6.5.1.3.4.3-0: Common Exception messages for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.5-4 Table H.3.5-9 Table H.3.7-1 with condition DRX.3 |

Table 6.5.1.3.4.3-1: Void

Table 6.5.1.3.4.3-2: RLF-TimersAndConstant

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| n310 | n1 |  |  |
| n311 | n1 |  |  |
| t311-v1530 | ms1000 |  |  |
| } |  |  |  |

Table 6.5.1.3.4.3-3: Void

Table 6.5.1.3.4.3-4 SIB1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfoSEQUENCE { |  |  |  |
| q-RxLevMin | -53 | -106 is actual value in dBm (-53 * 2 dBm) | dBm/15kHz |
|  | -51 | -102 is actual value in dBm (-51 * 2 dBm) | dBm/30kHz |
| } |  |  |  |
| } |  |  |  |

6.5.1.3.5	Test Requirement

Table 6.5.1.3.5-1 defines the cell specific primary level settings.

The UE behavior in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

Table 6.5.1.3.5-1: Cell specific test parameters for FR1 (Cell 1) for out-of-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1.8 | -6.2 | -15.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 |
| SNR on other channels and signals | Config 1, 2, 3 | dB | 1 |  |  |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -95 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in Figure 6.5.1.3.4-1. Note 5: The SNR values are specified for a UE with 2RX antennas connected under test. For a UE with 4RX antennas connected under test, the SNR during T3 from D.4.1.1, is -18dB-TT = -18.9dB (including test tolerances).. |  |  |  |  |  |

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode

6.5.1.4.1	Test purpose

The purpose of this test is to verify that the UE properly detects in sync for the purpose of monitoring downlink radio link quality of the Pcell when DRX is used. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.2.

6.5.1.4.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.5.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.4.

6.5.1.4.4	Test Description

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.1.4.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CSI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.Editor note: whether to revise power level to be gradually changed

Figure 6.5.1.4.4-1 - SNR variation for in-sync testing

6.5.1.4.4.1	Initial Conditions

This test shall be tested using any of the test configurations in Table 6.5.1.4.4.1-1.

Table 6.5.1.4.4.1-1: Supported test configurations for NR FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 2 | TDD, SSB SCS 15 KHz, data SCS 15KHz, BW 10MHz |
| 3 | TDD, SSB SCS 30 KHz, data SCS 30KHz, BW 40MHz |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.4.4.1-2.

Table 6.5.1.4.4.1-2: Initial conditions for SA FR1 radio link monitoring in-sync test for NR PCell configured with SSB-based RLM RS in DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.4.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.7.4 for TE Part |  |  |

PDCCH transmission parameters are given in Table 6.5.1.4.4.1-3.

Table 6.5.1.4.4.1-3: PDCCH transmission parameters for in-sync

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| DCI payload size | 1-0 |
| Number of control OFDM symbols | 2 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (MHz) | 24 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Distributed |

1. Message contents are defined in clause 6.5.1.4.4.3.

2. There is one cell (Cell 1), which is the active NR cell, in the test. The power levels and settings are set according to Annex A.6, Table A.6.1.1-1. The connection setup is done according to the settings in Annex C.1.3, and the downlink signal levels as per Annex C.1.2.

3. The general test parameters are given in Table 6.5.1.4.4.1-4 below.

4. Downlink signals for NR cell are initially set up according to Annex C.1.

Table 6.5.1.4.4.1-4: General test parameters for FR1 in-sync testing in DRX mode

| Parameter |  |  |  | Unit | Value |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |
| Active PCell |  |  |  |  | Cell 1 |
| RF Channel Number |  |  |  |  | 1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2, 3 |  | TDD |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration |  |  | Config 1 |  | SSB.1 FR1 |
|  |  |  | Config 2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |
|  |  |  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |
|  |  |  | Config 3 |  | 30 KHz |
| PRACH Configuration |  |  | Config 1, 2 |  | Table A.7.1-1, PRACH.1 FR1 |
|  |  |  | Config 3 |  | Table A.7.1-1, PRACH.1 FR1 |
| SSB index assigned as RLM RS |  |  |  |  | 0 |
| OCNG parameters |  |  |  |  | OP.1 |
| CP length |  |  |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |
| In sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 0 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| Out of sync transmission parameters | DCI format |  |  |  | 1-0 |
|  | Number of Control OFDM symbols |  |  |  | 2 |
|  | Aggregation level |  |  | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy |  |  | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy |  |  | dB | 4 |
|  | DMRS precoder granularity |  |  |  | REG bundle size |
|  | REG bundle size |  |  |  | 6 |
| DRX Configuration |  |  |  |  | DRX.3 |
| Gap pattern ID |  |  |  |  | N.A. |
| Layer 3 filtering |  |  |  |  | Enabled |
| T310 timer |  |  |  | ms | 2000 |
| T311 timer |  |  |  | ms | 1000 |
| N310 |  |  |  |  | 1 |
| N311 |  |  |  |  | 1 |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |
|  |  | Config 2 |  |  | TRS.1.1 TDD |
|  |  | Config 3 |  |  | TRS.1.2 TDD |
| T1 |  |  |  | s | 0.2 |
| T2 |  |  |  | s | 0.2 |
| T3 |  |  |  | s | 0.64 |
| T4 |  |  |  | s | 0.2 |
| T5 |  |  |  | s | 0.88 |
| D1 |  |  |  | s | 0.84 |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

6.5.1.4.4.2	Test Procedure

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.1.4.4-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. The UE shall be configured for periodic CQI reporting in PUCCH [format 1] with a reporting periodicity as mentioned in the above table 6.5.1.4.4.1-4.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

2. Set the parameters according to T1 in Table 6.5.1.4.5-1 for subtest 1 and 2. Propagation conditions are set according to Annex C.2.2. T1 starts.

3. When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.1.4.5-1. T2 starts.

4. When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.4.5-1. T3 starts.

5. When T3 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.4.5-1. T4 starts.

6. When T4 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.4.5-1. T5 starts.

7.	If the SS detects uplink power equal to or higher than the minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in the On-duration part of every DRX cycle in the subframe according the configured CQI reporting mode (PUCCH 1-0) during the period from time point A to time point F (D1 second after the start of time duration T5) the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [6] clause 4.5.

9. Repeat steps 2-7 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.1.4.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 4.6.1 and 7.3.1 with condition “Short_DCI” and with the following exceptions:

Table 6.5.1.4.4.3-0: Common Exception messages for NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.5-4 Table H.3.5-9 Table H.3.7-1 with condition DRX.3 |

Table 6.5.1.4.4.3-1: Void

Table 6.5.1.4.4.3-2: RLF-TimersAndConstant

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms2000 |  |  |
| n310 | n1 |  |  |
| n311 | n1 |  |  |
| t311-v1530 | ms1000 |  |  |
| } |  |  |  |

Table 6.5.1.4.4.3-3: Void

6.5.1.4.5	Test Requirement

The requirements in this section apply for each SSB based RLM-RS resource configured for the PCell, provided that the SSB configured for RLM are actually transmitted within UE active DL BWP during the entire evaluation period specified in section 6.5.1.4.3.

Table 6.5.1.4.5-1 defines the cell specific primary level settings.

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (D1 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence interval of 95%.

Table 6.5.1.4.5-1: Cell specific test parameters for FR1 for in-sync radio link monitoring tests in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
| SNR on other channels and signals | Config 1, 2, 3 | dB | 1 |  |  |  |  |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
|  | Config 1 | dBm/SCS | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -95 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 3: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 4: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in Figure 6.5.1.4.4-1. Note 5: The SNR values are specified for a UE with 2RX antennas connected under test. For a UE with 4RX antennas connected under test, the SNR during T3 and T4 from D.4.1.1 are -18.0-TT and -8.0-TT, which are -18.8dB and -8.8dB(including test tolerances). |  |  |  |  |  |  |  |

#### 6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

6.5.1.5.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1.

6.5.1.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting CSI-RS based RLM.

6.5.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.5.

6.5.1.5.4	Test description

The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 6.5.1.5.4-1 shows the three different time durations and the corresponding variation of the downlink SNR in the active cell to emulate out-of-sync states.

Figure 6.5.1.5.4-1: SNR variation for out-of-sync testing

6.5.1.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.1.5.4.1-1.

Table 6.5.1.5.4.1-1: Supported test configurations for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX

| Configuration | Description |
| --- | --- |
| 6.5.1.5-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.1.5-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.1.5-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.5.4.1-2.

Table 6.5.1.5.4.1-2: Initial conditions for NR SA radio link monitoring NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.5.1.5.4.1-3. The measurement gap configuration is according to Table 6.5.1.5.4.1-4.

2. Message contents are defined in clause 6.5.1.5.4.3.

3. There are one cell in the test, where Cell 1 is the NR PCell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 6.5.1.5.5-1 for this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

Table 6.5.1.5.4.1-3: General test parameters for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State.2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.48 |
| T3 |  | s | 0.48 |
| D1 |  | s | 0.44 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table 6.5.1.5.4.1-4: Measurement gap configuration for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |

6.5.1.5.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test. In the test, SSB0 is configured as the BFD-RS.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2.	The SS shall transmit an RRCReconfiguration message configuring the UE for inter-frequency measurements.

3.	The UE shall transmit RRCReconfigurationComplete message.

4.	Set the parameters of Cell 1 according to T1 in Table 6.5.1.5.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.1.5.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.5.5-1. T3 starts.

7.	If the SS:

a) detects uplink power equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C (D1 after the start of T3) until T3 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T3 expires the SS shall change the SNR value to T1 as specified in Table 6.5.1.5.5-1.

9.	If the UE has not re-established the connection in at least 1s, the UE is switched off and then on. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat steps 4-9 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and 7.3.1 with the following exceptions:

Table 6.5.1.5.4.3-1: Common Exception messages for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, GAP_NEEDED Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier center of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-9 Table H.3.5-4 Table H.3.5-9 with Condition CSI-RS RLM |

Table 6.5.1.5.4.3-2 SIB1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfoSEQUENCE { |  |  |  |
| q-RxLevMin | -46 | -92 is actual value in dBm (-46 * 2 dBm) | dBm/15kHz or dBm/30kHz |
| } |  |  |  |
| } |  |  |  |

6.5.1.5.5	Test requirement

Tables 6.5.1.5.4.1-3 and 6.5.1.5.5-1 define the primary level settings including test tolerances for Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode.

Table 6.5.1.5.5-1: Cell specific test parameters for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSSSS_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSOCNG_beta |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1.8 | -6.2 | -15.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 |
| SNR on other channels and signals | Config 1 | dB | 1 |  |  |
|  | Config 2 |  | 1 |  |  |
|  | Config 3 |  | 1 |  |  |
|  | Config 1 | dBm/15KHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.1.5.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 from D.4.1.1 is -18 -TT, which is -18.8dB (including test tolerances). |  |  |  |  |  |

The UE behaviour during time durations T1, T2 and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 no later than time point C (D1 ms after the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

#### 6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

6.5.1.6.1	Test purpose

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when no DRX is used. This test will partly verify the FR1 PCell CSI-RS in-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1.

6.5.1.6.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting CSI-RS based RLM.

6.5.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.6.

6.5.1.6.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.1.6.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the Pcell to emulate out-of-sync and in-sync states.

Figure 6.5.1.6.4-1: SNR variation for In-sync testing

6.5.1.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.1.6.4.1-1.

Table 6.5.1.6.4.1-1: Supported test configurations for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

| Configuration | Description |
| --- | --- |
| 6.5.1.6-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.1.6-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.1.6-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.6.4.1-2.

Table 6.5.1.6.4.1-2: Initial conditions for for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.5.1.6.4.1-3.

2. Message contents are defined in clause 6.5.1.6.4.3.

3. There is one cell in the test, where Cell 1 is the NR PCell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 6.5.1.6.5-1 for this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

Table 6.5.1.6.4.1-3: General test parameters for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State.2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 1000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 0.44 |
| T4 |  | s | 0.2 |
| T5 |  | s | 0.88 |
| D1 |  | s | 0.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

6.5.1.6.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is not enabled. In the test, SSB0 is configured as the BFD-RS.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters of Cell 1 according to T1 in Table 6.5.1.6.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3. When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.1.6.5-1. T2 starts.

4. When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.6.5-1. T3 starts.

5. When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.5.1.6.5-1. T4 starts.

6. When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.5.1.6.5-1. T5 starts.

7. If the SS detects uplink power equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point F (T6 after the start of time duration T5) the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8. If the UE has not re-established the connection in at least 1s, the UE is switched off and then on. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

9. After T5 expires, repeat steps 2-7 for both subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and 7.3.1 with the following exceptions:

Table 6.5.1.6.4.3-1: Common Exception messages for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.5-4 Table H.3.5-9 with Condition CSI-RS RLM |

6.5.1.6.5	Test requirement

Tables 6.5.1.6.4.1-3 and 6.5.1.6.5-1 define the primary level settings including test tolerances for Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode.

Table 6.5.1.6.5-1: Cell specific test parameters for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSSSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSOCNG_beta |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
| SNR on other channels and signals | Config 1 | dB | 1 |  |  |  |  |
|  | Config 2 |  | 1 |  |  |  |  |
|  | Config 3 |  | 1 |  |  |  |  |
|  | Config 1 | dBm/15kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 6.5.1.6.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 from D.4.1.1 are -18.0-TT and -8.0-TT, which are -18.8dB and -8.8dB(including test tolerances). |  |  |  |  |  |  |  |

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

#### 6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

6.5.1.7.1	Test purpose

The purpose of this test is to verify that the UE properly detects the out of sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS Out-of-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1.

6.5.1.7.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, CSI-RS based RLM and long DRX cycle.

6.5.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.7.

6.5.1.7.4	Test description

The test consists three successive time periods, with time duration of T1, T2 and T3 respectively. Figure 6.5.1.7.4-1 shows the three different time durations and the corresponding variation of the downlink SNR in the active cell to emulate out-of-sync states.

Figure 6.5.1.7.4-1: SNR variation for out-of-sync testing

6.5.1.7.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 6.5.1.7.4.1-1.

Table 6.5.1.7.4.1-1: Supported test configurations for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Configuration | Description |
| --- | --- |
| 6.5.1.7-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.1.7-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.1.7-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.7.4.1-2.

Table 6.5.1.7.4.1-2: Initial conditions for NR SA radio link monitoring for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.5.1.7.4.1-3.

2. Message contents are defined in clause 6.5.1.7.4.3.

3. There is one cell in the test, where Cell 1 is the NR PCell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 6.5.1.7.5-1 for this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

Table 6.5.1.7.4.1-3: General test parameters for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State.2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | N.A. |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 1.28 |
| T3 |  | s | 1.28 |
| D1 |  | s | 1.24 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

6.5.1.7.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. In the test, SSB0 is configured as the BFD-RS.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Establish SRB2 and DRB in the RRC Reconfiguration message.

2. Set the parameters of Cell 1 according to T1 in Table 6.5.1.7.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3. When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.1.7.5-1. T2 starts.

4. When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.7.5-1. T3 starts.

5. If the SS:

a) detects uplink power equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in the On-duration part of every DRX cycle in the slots configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C (D1 after the start of T3) until T3 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

6.	When T3 expires the SS shall change the SNR value to T1 as specified in Table 6.5.1.7.5-1.

7.	If the UE has not re-established the connection in at least 1s, the UE is switched off and then on. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and 7.3.1 with the following exceptions:

Table 6.5.1.7.4.3-1: Common Exception messages for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-9 Table H.3.5-4 Table H.3.5-9 with Condition CSI-RS RLM Table H.3.7-1 with condition DRX.3 |

Table 6.5.1.7.4.3-2 SIB1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellSelectionInfoSEQUENCE { |  |  |  |
| q-RxLevMin | -46 | -92 is actual value in dBm (-46 * 2 dBm) | dBm/15kHz or dBm/30kHz |
| } |  |  |  |
| } |  |  |  |

Table 6.5.1.7.4.3-3: RLF-TimersAndConstant

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| } |  |  |  |

6.5.1.7.5	Test requirement

Tables 6.5.1.7.4.1-3 and 6.5.1.7.5-1 define the primary level settings including test tolerances for Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode.

Table 6.5.1.7.5-1: Cell specific test parameters for FR1 for NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSSSS_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSOCNG_beta |  | dB |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1.8 | -6.2 | -15.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 |
| SNR on other channels and signals | Config 1 | dB | 1 |  |  |
|  | Config 2 |  | 1 |  |  |
|  | Config 3 |  | 1 |  |  |
|  | Config 1 | dBm/15kHz | -98 |  |  |
|  | Config 2 |  | -98 |  |  |
|  | Config 3 |  | -98 |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2 and T3 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.1.7.1-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 from D.4.1.1 is -18 -TT, which is -18.8dB (including test tolerances). |  |  |  |  |  |

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During time durations T1, T2 and T3, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on PCell.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 (PCell) at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

The UE shall stop transmitting uplink signal in Cell 1 (PCell) no later than time point C (D1 ms after the start of the time duration T3) on the PCell.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

#### 6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

6.5.1.8.1	Test purpose

The purpose of this test is to verify that the UE properly detects the in sync for the purpose of monitoring downlink CSI-RS based radio link quality of the PCell when DRX is used. This test will partly verify the FR1 PCell CSI-RS in-sync radio link monitoring requirements in TS 38.133 [6] clause 8.1.

6.5.1.8.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, CSI-RS based RLM and long DRX cycle.

6.5.1.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.1.8.

6.5.1.8.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.1.8.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate in-sync states.

Figure 6.5.1.8.4-1: SNR variation for In-sync testing

6.5.1.8.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 6.5.1.8.4.1-1.

Table 6.5.1.8.4.1-1: Supported test configurations for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Configuration | Description |
| --- | --- |
| 6.5.1.8-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.1.8-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.1.8-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.1.8.4.1-2.

Table 6.5.1.8.4.1-2: Initial conditions for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.5.1.8.4.1-3. The measurement gap configuration for subtest 2 is according to Table 6.5.1.8.4.1-4.

2. Message contents are defined in clause 6.5.1.8.4.3.

3. There are one cell in the test, where Cell 1 is the NR PCell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table A.6.1.1-1 for this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

Table 6.5.1.8.4.1-3: General test parameters for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode | Config 1 |  | FDD |
|  | Config 2, 3 |  | TDD |
| TDD Configuration | Config 1 |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.2.1 |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |
|  | Config 3 |  | CR.2.1 TDD |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |
|  | Config 2 |  | SSB.1 FR1 |
|  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |
|  | Config 3 |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |
|  | Config 3 |  | 30 kHz |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |
|  | Config 2 |  | TRS.1.1 TDD |
|  | Config 3 |  | TRS.1.2 TDD |
| CSI-RS for RLM | Config 1 |  | Resource #4 in TRS.1.1 FDD |
|  | Config 2 |  | Resource #4 in TRS.1.1 TDD |
|  | Config 3 |  | Resource #4 in TRS.1.2 TDD |
| TCI configuration for PDCCH/PDSCH |  |  | TCI.State.2 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| Out of sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 8 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 4 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 4 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| In sync transmission parameters | DCI format |  | 1-0 |
|  | Number of Control OFDM symbols |  | 2 |
|  | Aggregation level | CCE | 4 |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |
|  | DMRS precoder granularity |  | REG bundle size |
|  | REG bundle size |  | 6 |
| DRX |  |  | DRX.3 |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 2000 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.1.1 FDD |
|  | Config 2 |  | CSI-RS.1.1 TDD |
|  | Config 3 |  | CSI-RS.2.1 TDD |
| T1 |  | s | 0.2 |
| T2 |  | s | 0.2 |
| T3 |  | s | 1.24 |
| T4 |  |  | 0.2 |
| T5 |  |  | 1.88 |
| T6 |  | s | 1.84 |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table 6.5.1.8.4.1-4: Measurement gap configuration for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Field | Test 1 |
| --- | --- |
|  | Value |
| gapOffset | 0 |
| Note 1: Void |  |

6.5.1.8.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms). In the test, SSB0 is configured as the BFD-RS.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall transmit an RRCReconfiguration message configuring the UE for inter-frequency measurements.

3.	The UE shall transmit RRCReconfigurationComplete message.

4.	Set the parameters of Cell 1 according to T1 in Table 6.5.1.8.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.1.8.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.1.8.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.5.1.8.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.5.1.8.5-1. T5 starts.

9.	If the SS detects uplink power equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in the On-duration part of every DRX cycle in the configured slots for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point F (T6 after the start of time duration T5) the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

10.	If the UE has not re-established the connection in at least 1s, the UE is switched off and then on. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 4-10 for both subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.1.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and 7.3.1 with the following exceptions:

Table 6.5.1.8.4.3-1: Common Exception messages for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, GAP_NEEDED Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier center of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.5-4 Table H.3.5-9 with Condition CSI-RS RLM Table H.3.7-1 with condition DRX.3 and Gap |

Table 6.5.1.8.4.3-2: RLF-TimersAndConstant

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms2000 |  |  |
| } |  |  |  |

6.5.1.8.5	Test requirement

Tables 6.5.1.8.4.1-3 and 6.5.1.8.5-1 define the primary level settings including test tolerances for Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode.

Table 6.5.1.8.5-1: Cell specific test parameters for NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSSPDCCH_beta |  | dB | 4 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRSPDCCH_DMRS_beta |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSSPBCH_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSPSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRSSSS_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS PDSCH_beta |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRSOCNG_beta |  | dB |  |  |  |  |  |
| SNR on RLM-RS | Config 1 | dB | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 2 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
|  | Config 3 |  | 1.8 | -6.2 | -15.8 | -5.3 | 1.8 |
| SNR on other channels and signals | Config 1 | dB | 1 |  |  |  |  |
|  | Config 2 |  | 1 |  |  |  |  |
|  | Config 3 |  | 1 |  |  |  |  |
|  | Config 1 | dBm/15kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1.. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2, SNR3, SNR4 and SNR5 respectively in figure 6.5.1.8.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 from D.4.1.1 are -18.0-TT and -8.0-TT, which are -18.8dB and -8.8dB(including test tolerances) |  |  |  |  |  |  |  |

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the period from time point A to time point F (T6 second after the start of time duration T5) the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting on the PCell.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

### 6.5.2	Interruption

#### 6.5.2.0	Minimum conformance requirements

##### 6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC

Interruptions on PCell due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [13] is 640 ms or longer.

If the PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on PCell immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 6.5.2.0.1-1

If the PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by Table 6.5.2.0.1-2. The interruption shall not exceed requirements in Table 6.5.2.0.1-2.

Interruptions on active SCells due to measurements when an SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell [13] is 640 ms or longer.

If the activated SCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on an activated SCell immediately before and immediately after an SMTC. Each interruption shall not exceed requirement in Table 6.5.2.0.1-1

If the activated SCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on the activated SCell no earlier than X slots before TSMTC_duration and no later than X slots after TSMTC_duration, provided the cell specific reference signals from the active serving cells and the deactivated SCell are available in the same slot, where X and TSMTC_duration are given by Table 6.5.2.0.1-2. The interruption shall not exceed requirements in Table 6.5.2.0.1-2.

Table 6.5.2.0.1-1: Interruption duration for SCell activation/deactivation for inter-band CA

|  | NR Slot length (ms) of victim cell | Interruption length (slot) |  |
| --- | --- | --- | --- |
| 0 | 1 |  | 1 |
| 1 | 0.5 |  | 1 |
| 2 | 0.25 | Both aggressor cell and victim cell are on FR2 | 2 |
|  |  | Either aggressor cell or victim cell is on FR1 | 3 |
| 3 | 0.125 | Aggressor cell is on FR2 | 4 |
|  |  | Aggressor cell is on FR1 | 5 |

Table 6.5.2.0.1-2: Interruption duration for SCell activation/deactivation for intra-band CA

|  | NR Slot length (ms) | X (slots) | Interruption length (slots) |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 1 | 0.5 | 1 | 2 + TSMTC_duration * Nslotsubframe,μ |
| 2 | 0.25 | 2 | 4 + TSMTC_duration * Nslotsubframe,μ |
| 3 | 0.125 | 4 | 8 + TSMTC_duration * Nslotsubframe,μ |
| NOTE 1: TSMTC_duration measured in subframes is the longest SMTC duration among all above active serving cells and the deactivated SCell to be measured; NOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [7]. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.2.2.2.

#### 6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC

6.5.2.1.1	Test purpose

To verify UE’s ability to complete  NR PCell interruptions during the measurement on the deactivated NR SCC within the missed ACK/NACK rate in standalone NR requirements.

6.5.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 2DL CA.

6.5.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.2.1.

6.5.2.1.4	Test description

6.5.2.1.4.1	Initial conditions

Test 6.5.2.1 can be run in one of the configurations defined in Table 6.5.2.1.4.1-1.

Table 6.5.2.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.5.2.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD – FDD duplex mode |
| 6.5.2.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD – TDD duplex mode |
| 6.5.2.1-3 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD – FDD duplex mode |
| 6.5.2.1-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD – TDD duplex mode |
| 6.5.2.1-5 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD – TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.2.1.4.1-2.

Table 6.5.2.1.4.1-2: Initial conditions for NR SA FR1 interruptions during measurements on deactivated NR SCC

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.5.2.1.4.1-3

2. Message contents are defined in clause 6.5.2.1.4.3.

3. There are two NR carriers and two cells specified in the test. Cell 1 is the PCell on one NR carrier, Cell 2 is the SCell on the other NR carrier. Cell 1 and Cell 2 shall be configured according to Annex C.1.1 and C.1.2.

Table 6.5.2.1.4.1-3: General test parameters for NR SA FR1 interruptions during measurements on deactivated NR SCC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR RF channels |
| Active PCell |  | Cell1 | PCell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell2 | Deactivated SCell on NR RF channel number 2. |
| CP length |  | Normal | Applicable to Cell1 and Cell 2 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

6.5.2.1.4.2	Test procedure

The test consists of two cells: Cell1 and Cell2. Cell1 is PCell and Cell2 is deactivated SCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. Cell1 shall be configured as PCell and Cell2 shall be configured as SCell.. During T1, PCell is continuously scheduled in DL.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall transmit an RRCReconfiguration message including measCycleSCell or allowInterruptions for the deactivated NR SCell.

3.	The UE shall transmit RRCReconfigurationComplete message.

4.	Set the parameters according to Table 6.5.2.1.5-1. Propagation conditions are set according to Annex C.2.1. T1 starts.

5.	SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

6.	If more than 99.5% of uplink transmissions are received by SS then count a success for the event “ACK/NACK”. Otherwise count a fail for the event “ACK/NACK”.

7.	If no longer than X consecutive DTX is observed by the SS, then count a success for the event “DTX”. Otherwise count a fail for the event “DTX”. Where

-	For test configuration 6.5.2.1-1 and 6.5.2.1-3,

-	X = interruption length+k1 if k1 ≤ interruption length

-	X = interruption length if k1 > interruption length

-	For test configuration 6.5.2.1-2, 6.5.2.1-4 and 6.5.2.1-5,

-	X = interruption length.

-	interruption length is given in table 6.5.2.1.5-2 for inter-band case and in 6.5.2.1.5-3 for intra-band case.

8.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED TS 38.508-1 [14] clause 4.5),

or

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat step 2-8 until a test verdict has been achieved.

Each of the events "ACK/NACK" and "DTX" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

6.5.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition Deactivated SCell; |
| Specific message contents exceptions for Test Configuration 6.5.2.1-1, 6.5.2.1-2, 6.5.2.1-3 and 6.5.2.1-4 | Table H.3.1-3 with Condition Deactivated SCell and SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.5.2.1-5 | Table H.3.1-3 with Condition Deactivated SCell and SSB.2 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

6.5.2.1.5	Test requirement

Table 6.5.2.1.5-1 defines the primary level settings including test tolerances for NR SA FR1 interruptions during measurements on deactivated NR SCC.

Table 6.5.2.1.5-1: NR cell specific test parameters for NR SA FR1 interruptions during measurements on deactivated NR SCC

| Parameter |  | Unit | Cell1 |  | Cell2 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  | FR1 |  | FR1 |
| Duplex mode | Config 1 |  | FDD |  | FDD |
|  | Config 2,5 |  | TDD |  | TDD |
|  | Confiq 3 |  | TDD |  | FDD |
|  | Confiq 4 |  | FDD |  | TDD |
| TDD configuration | Config 1 |  | Not Applicable |  | Not Applicable |
|  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |
|  | Config 3 |  | TDDConf.1.1 |  | Not Applicable |
|  | Confiq 4 |  | Not Applicable |  | TDDConf.1.1 |
|  | Confiq 5 |  | TDDConf.1.2 |  | TDDConf.1.2 |
| BWchannel | Config 1,2,3,4 |  | 10 MHz: NRB,c = 52 |  | 10 MHz: NRB,c = 52 |
|  | Config 5 |  | 40 MHz: NRB,c = 106 |  | 40 MHz: NRB,c = 106 |
| Initial DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.0.1 | DLBWP.0.1 |  |
|  | Config 5 |  | DLBWP.0.1 | DLBWP.0.1 |  |
| Dedicated DL BWP Configuration | Config 1,2,3,4 |  | DLBWP.1.1 | DLBWP.1.1 |  |
|  | Config 5 |  | DLBWP.1.1 | DLBWP.1.1 |  |
| Initial UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.0.1 | ULBWP.0.1 |  |
|  | Config 5 |  | ULBWP.0.1 | ULBWP.0.1 |  |
| Dedicated UL BWP Configuration | Config 1,2,3,4 |  | ULBWP.1.1 | ULBWP.1.1 |  |
|  | Config 5 |  | ULBWP.1.1 | ULBWP.1.1 |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  | SR.1.1 FDD |
|  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |
|  | Config 3 |  | SR.1.1 TDD |  | SR.1.1 FDD |
|  | Confiq 4 |  | SR.1.1 FDD |  | SR.1.1 TDD |
|  | Confiq 5 |  | SR.2.1 TDD |  | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  | CR.1.1 FDD |
|  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |
|  | Config 3 |  | CR.1.1 TDD |  | CR.1.1 FDD |
|  | Confiq 4 |  | CR.1.1 FDD |  | CR.1.1 TDD |
|  | Confiq 5 |  | CR.2.1 TDD |  | CR.2.1 TDD |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  | CCR.1.1 FDD |
|  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |
|  | Config 3 |  | CCR.1.1 TDD |  | CCR.1.1 FDD |
|  | Config 4 |  | CCR.1.1 FDD |  | CCR.1.1 TDD |
|  | Config 5 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |
| OCNG Patterns |  |  | OP.1 |  | OP.1 |
| SMTC Configuration |  |  | SMTC.1 |  | SMTC.1 |
| SSB Configuration | Config 1,2,3,4 |  | SSB.1 FR1 |  | SSB.1 FR1 |
|  | Config 5 |  | SSB.2 FR1 |  | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 |  | dBm/15 kHz | -104 |  | -104 |
| SS-RSRP Note 3 |  | dBm/15 kHz | -87 |  | -87 |
| Ês/Iot |  | dB | 17 |  | 17 |
| Ês/Noc |  | dB | 17 |  | 17 |
| NocNote 2 | Config 1,2,3,4 | dBm/SCS | -104 |  | -104 |
|  | Config 5 |  | -101 |  | -101 |
| IoNote3 | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 |  | -58.96 |
|  | Config 5 | dBm/ 38.16MHz | -52.86 |  | -52.86 |
| Time offset to Cell1 Note 5 |  | s | - |  | 3 |
| Propagation Condition |  |  | AWGN |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Receive time difference between slot boundaries of signals received from the two cells at the UE antenna connector including time alignment error between the two cells. Note 6: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2 defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |

The UE shall be continuously scheduled on PCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on PCell.

If the NR PCell is not in the same band as the deactivated SCell, the UE is only allowed to cause interruptions on NR PCell immediately before and immediately after an SMTC. Each interruption on NR PCell shall not exceed the value defined in Table 6.5.2.1.5-2.

If the NR PCell is in the same band as the deactivated SCell, the UE is only allowed to cause an interruption on PCell no earlier than 1 slot before an SMTC and no later than 1 slot after the SMTC. the interruption on NR PCell shall not exceed the value defined in Table 6.5.2.1.5-3.

Table 6.5.2.1.5-2: Interruption duration if the PCell is not in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |

Table 6.5.2.1.5-3: Interruption duration if the PCell is in the same band as the deactivated SCell

|  | NR Slot length (ms) | Interruption length |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

The rate of correct events observed during repeated tests shall be at least 90%.

### 6.5.3	SCell activation and deactivation delay

#### 6.5.3.0	Minimum conformance requirements

##### 6.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay

Same as in clause 4.5.3.0.1.

#### 6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle

6.5.3.1.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements in TS 38.133 [6] clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

6.5.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting 2DL CA.

6.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.3.1.

6.5.3.1.4	Test description

6.5.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.3.1.4.1-1.

Table 6.5.3.1.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.5.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.5.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.5.3.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.3.1.4.1-2 and Table 6.5.3.1.4.1-3.

Table 6.5.3.1.4.1-2: Initial conditions for known FR1 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.3.1.5-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.1 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

Table 6.5.3.1.4.1-3: General test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] depends on UE’s capability |
| TCSI_Reporting | ms | 10+5⋅2µDL | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13]. µDL is the subcarrier spacing configuration for DL. |

1.	Message contents are defined in clause 6.5.3.1.4.3.

2.	There are two NR carriers, each with one cell. Cell 1 is NR FR1 PCell and Cell 2 is the deactivated SCell. Cell 1 and Cell 2 are configured according to Annex C.1.2 and C.1.3.

6.5.3.1.4.2	Test procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. Before the test starts the UE is connected to Cell 1, but is not aware of Cell2. The UE is only monitoring the PCC. The UE shall be continuously scheduled in the PCell throughout the whole test.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T2. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in TS 38.133 [6] clause 8.3. The UE shall start reporting CSI in PCell in slot n+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] clause 8.3, where Ninterruption is the interruption length given in TS 38.133 [6] clause 8.2.

Time period T3 starts when a MAC message for deactivation of SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell in a slot m+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in TS 38.133 [6] clause 8.3.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Tables 6.5.3.1.4.1-3 and 6.5.3.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2. T1 starts.

3.	The SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1.

4.	The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted n. If the SS receives ACK for MAC-CE sent by the UE, T2 starts in slot m, and the test proceeds to step 6, otherwise go to step 9.

6.	The UE shall start sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PCell during SCell activation.

-	If the first CSI report for SCell is received by the SS in a slot n+1+THARQ+3msNR slot length,

-	or slot n+1+THARQ+3ms+TXNR slot length+Ninterruption+1 if the slot n+1+THARQ+3msNR slot length was subject to interruption,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length,

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length

-	and DTX is not observed by the SS outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T2

-	Then the number of successes for the event “Activation” is increased by one. Otherwise, count a fail for the event “Activation” and go to step 9.

7.	When T2 expires, the SS deactivate SCC by sending the deactivation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted m. If the SS receives ACK for MAC-CE sent by the UE, T3 starts in slot m, and the test proceeds to step 8, otherwise go to step 9.

8.	The UE shall stop sending CSI reports for SCell and the SS shall monitor CSI reports for SCell sent from the UE and ACK/NACK sent in PCell during SCell deactivation.

-	If the last CSI report is received by the SS earlier than or equal to slot m+THARQ+3msNR slot length

-	and DTX is not observed by the SS outside the slot m+1+THARQNR slot length to m+1+THARQ+3ms+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T3,

-	Then the number of successes for the event “Deactivation” is increased by one. Otherwise, count a fail for the event “Deactivation”.

9.	When T3 expires, or Activation in step 5 was not acknowledged, or a fail was counted for the event “Activation” in step 6, or Deactivation in step 7 was not acknowledged, the SS shall transmit a RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

10.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

11.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

13.	Repeat steps 2-11 until a test verdict has been achieved.

Each of the events “Activation” and “Deactivation” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.
If all events pass, the test passes. If one event fails, the test fails.

6.5.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition Deactivated SCell; Table H.3.1-7 with Condition Deactivated SCell; |
| Specific message contents exceptions for Test Configuration 6.5.3.1-1, 6.5.3.1-2 | Table H.3.1-3 with Condition Deactivated SCell and SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.5.3.1-3 | Table H.3.1-3 with Condition Deactivated SCell and SSB.2 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.5.3.1.4.3-2: RRCReconfiguration: SCell addition

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig-DEFAULT | Measurements configuration | NR_MEAS |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-SCell(n) | n is number of SCC to be added | SCell_add |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.3.1.4.3-3: MeasObjectNR for SCell measurement

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| measCycleSCell-v1530 | sf160 |  |  |
| } |  |  |  |

6.5.3.1.5	Test requirement

Table 6.5.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.5.3.1.5-1: Cell specific test parameters for known FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter |  |  |  |  | Unit | T1 |  |  | T2 |  | T3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| Duplex mode |  | Config 1 |  |  |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  |  | Not applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  | TDDConf.1.2 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1,2 |  |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| Initial BWP configuration |  |  |  |  |  | DLBWP.0.2 |  |  |  |  |  |  |  |
| TCI state |  |  |  |  |  | TCI.State.0 |  |  |  |  |  |  |  |
| TRS Configuration |  |  |  |  |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  |  | SR.1.1 FDD |  | - | SR.1.1 FDD | - | SR.1.1 FDD |  | - |
|  |  | Config 2 |  |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| Dedicated CORESET parameters |  | Config 1 |  |  |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  |  |  | CCR2.3 TDD |  |  | CCR2.3 TDD |  | CCR2.3 TDD |  |  |
| RMSI CORESET parameters |  | Config 1 |  |  |  | CR.1.1 FDD |  | - | CR.1.1 FDD | - | CR.1.1 FDD |  | - |
|  |  | Config 2 |  |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  |  |  | CR2.1 TDD |  |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  |  |  |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  | Config 3 |  |  |  |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting | Config 1 |  |  |  |  | CSI-RS.1.1 FDD |  |  |  |  |  |  |  |
|  | Config 2 |  |  |  |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |  |
|  | Config 3 |  |  |  |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |  |
| SMTC configuration |  |  |  |  |  | SMTC.1 |  |  |  |  |  |  |  |
| CSI reporting periodicity |  |  |  |  | ms | 5 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  | Config 1,2,4,5 | dBm/15kHz | -104 |  |  |  |  |  |  |  |
|  |  |  |  | Config 3,6 |  | -101 |  |  |  |  |  |  |  |
|  |  |  |  |  | dB | 17 |  |  |  |  |  |  |  |
|  |  |  |  |  | dB | 17 |  |  |  |  |  |  |  |
| SS-RSRPNote3 |  |  |  | Config 1,2,4,5 | dBm/SCS | -87 |  |  |  |  |  |  |  |
|  |  |  |  | Config 3,6 |  | -84 |  |  |  |  |  |  |  |
| SCH_RP Note 3 |  |  |  |  | dBm/15 kHz | -87 |  |  |  |  |  |  |  |
| Io Note3 |  |  | Config 1,2,4,5 |  | dBm/ 9.36MHz | -58.96 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  | dBm/ 38.16MHz | -52.87 |  |  |  |  |  |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. |  |  |  |  |  |  |  |  |  |  |  |  |  |

During T2 the UE shall send the first CSI report for SCell in a slot n+1+THARQ+3msNR slot length, UE is allowed to postpone CSI report to next available uplink resource if an available uplink resource is subject to interruption.

During T2 the UE shall start sending CSI reports for SCell with non-zero CQI index at latest in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length, Tactivation_time = TFirstSSB+ 5ms, as defined in TS 38.133 [6] section 8.3.

During T2 interruption of PCell/PSCell during SCell activation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in TS 38.133 [6] section 8.3.

Figures 6.5.3.1.5-1 shows the derivation of the Test procedure requirement for DTX during T2, based on the core requirements for interruption.

Figure 6.5.3.1.5-1: Procedure derivation for Activation

1)	Activation command for SCell

2)	ACK for MAC-CE for SCell1 activation

3)	First CSI report timing (could be invalid CQI)

During T3 the UE shall stop sending CSI reports for SCell at latest in a slot m+THARQ+3msNR slot length, as defined in TS 38.133 [6] section 8.3.

During T3 interruption of PCell during SCell deactivation shall not happen outside the slot m+1+THARQNR slot length to m+1+THARQ+3msNR slot length, as defined in TS 38.133 [6] section 8.3.

Figures 6.5.3.1.5-2 shows the derivation of the Test procedure requirement for NR PSCell DTX during T3, based on the core requirements for interruption.

Figure 6.5.3.1.5-2: Procedure derivation for Deactivation

1)	Deactivation command for SCell

2)	Latest slot stop sending CSI reports for SCell

The interruption on any activated serving cell shall not be more than the values specified for SA in TS 38.133 [6] clause 8.2.2.2.2.

All of the above test requirements shall be fulfilled in order for the observed SCell activation delay and SCell deactivation delay to be counted as correct. The rate of correct observed SCell activation delay and SCell deactivation delay during repeated tests shall be at least 90%.

NOTE:	During T2 if there are no uplink resources for reporting the valid CSI in a slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in TS 38.133 [6] section 8.3 then the UE shall use the next available uplink resource for reporting the corresponding valid CSI.

#### 6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle

6.5.3.2.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements in TS 38.133 [6] clause 8.3, when the SCell in FR1 is known by the UE at the time of activation.

6.5.3.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting 2DL CA.

6.5.3.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.3.2.

6.5.3.2.4	Test description

6.5.3.2.4.1	Initial conditions

Same initial conditions as described in section 6.5.3.1.4.1 with following exception:

-	The supported test configurations is replaced by Table 6.5.3.2.4.1-1.

-	The listed parameter values in Tables 6.5.3.2.4.1-2 will replace the values of corresponding parameters in Tables 6.5.3.1.4.1-3.

Table 6.5.3.2.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.5.3.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.5.3.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.5.3.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.5.3.2.4.1-2: General test parameters for known FR1 SCell activation case, 320ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| SCell measurement cycle (measCycleSCell) | ms | 320 |  |

6.5.3.2.4.2	Test procedure

Same test procedure as described in section 6.5.3.1.4.2.

6.5.3.2.4.3	Message contents

Same message contents as described in section 6.5.3.1.4.3 with following exception:

Table 6.5.3.2.4.3-1: MeasObjectNR for SCell measurement

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| measCycleSCell-v1530 | sf320 |  |  |
| } |  |  |  |

6.5.3.2.5	Test requirement

Same test requirement as described in section 6.5.3.1.5, except Tactivation_time will be replaced with the value TFirstSSB+5msTFirstSSB_MAX + Trs + 5ms.

#### 6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX

6.5.3.3.1	Test purpose

This test is to verify that the SCell activation and deactivation times are within the requirements stated in TS 38.133 [6] clause 8.3, when the SCell in FR1 is unknown by the UE at the time of activation.

6.5.3.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting 2DL CA.

6.5.3.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.3.3.

6.5.3.3.4	Test description

6.5.3.3.4.1	Initial conditions

Same initial conditions as described in section 6.5.3.1.4.1 with following exception:

-	The supported test configurations is replaced by Table 6.5.3.3.4.1-1.

-	The listed parameter values in Tables 6.5.3.3.4.1-2 will replace the values of corresponding parameters in Tables 6.5.3.1.4.1-3.

Table 6.5.3.3.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.5.3.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.5.3.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.5.3.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.5.3.3.4.1-2: General test parameters for unknown FR1 SCell activation case, 160ms SCell measurement cycle

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| T1 | ms | 100 | During this time the PSCell shall be known and the SCell configured, but not detected. |

6.5.3.3.4.2	Test procedure

Same test procedure as described in section 6.5.3.1.4.2, except step3 and step 5:

3.	The SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1. The SCell (Cell 2) shall be powered OFF till T2 starts.

5.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10) in a slot # denoted n. If the SS receives ACK for MAC-CE sent by the UE, power ON the SCell (Cell2), T2 starts in slot n, and the test proceeds to step 6, otherwise go to step 9.

6.5.3.3.4.3	Message contents

Same message contents as described in section 6.5.3.1.4.3

6.5.3.3.5	Test requirement

Same test requirement as described in section 6.5.3.1.5, except Tactivation_time will be replaced with the valueTFirstSSB+5msTFirstSSB_MAX+TSMTC_MAX+2*Trs+5ms TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms

### 6.5.4	UE UL carrier RRC reconfiguration delay

#### 6.5.4.0	Minimum conformance requirements

##### 6.5.4.0.1	Minimum conformance requirements for UL carrier RRC reconfiguration delay

[TS 38.133, clause 8.4.2]

When the UE receives a RRC message implying NR UL or Supplementary UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within TUL_carrier_config from the end of the last slot containing the RRC command.

TUL_carrier_config equals the maximum RRC procedure delay defined in clause x.y in TS 38.331 [2] plus the interruption time specified in TS 38.133 [6] section 8.2.1.2.6.

[TS 38.133, clause 8.4.3]

When the UE receives a RRC message implying NR UL or Supplementary UL carrier deconfiguration RRC signalling, the UE shall stop UL signalling on the deconfigured UL carrier within TUL_carrier_deconfig from the end of the last slot containing the RRC command.

TUL_carrier_deconfig equals the maximum RRC procedure delay defined in clause x.y in TS 38.331 [2].

The normative reference for this requirement is TS 38.133 [6] clause 8.4.

#### 6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay

6.5.4.1.1	Test purpose

To verify that when the UE receives a RRC message implying NR UL or Supplementary UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in TS 38.133 [6] section 8.4.2 and 8.4.3 for configuring and deconfiguring, respectively.

6.5.4.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting SUL.

6.5.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.4.1.

6.5.4.1.4	Test description

6.5.4.1.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.5.4.1.4.1-1.

Table 6.5.4.1.4.1-1: Supported test configurations

| Configuration | PSCell (Cell 1) | SCell (Cell 2) |
| --- | --- | --- |
| 6.5.4.1-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | DL and UL: 15kHz SSB SCS, 10MHz bandwidth, FDD duplex mode; SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-2 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | DL and UL: 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode; SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-3 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | DL and UL: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, 40MHz bandwidth, SUL duplex mode |
| 6.5.4.1-4 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | DL and UL: 15kHz SSB SCS, 10MHz bandwidth, FDD duplex mode; SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-5 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | DL and UL: 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode; SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-6 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, 40MHz bandwidth, SUL duplex mode |
| 6.5.4.1-7 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | DL and UL: 15kHz SSB SCS, 10MHz bandwidth, FDD duplex mode; SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-8 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | DL and UL: 15kHz SSB SCS, 10MHz bandwidth, TDD duplex mode; SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-9 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | DL and UL: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode; SUL: 30kHz SCS, 40MHz bandwidth, SUL duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Configure the test requirement and the DUT according to the parameters in Table 6.5.4.1.4.1-2.

Table 6.5.4.1.4.1-2: Initial conditions for NR SA FR1 UE UL carrier RRC reconfiguration delay

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 6.5.4.1.4.1-3.

2.	Message contents are defined in clause 6.5.4.1.4.3.

3.	There are two NR FR1 carriers and two cells in the test. Cell 1 is PCell on the primary component carrier, Cell 2 is SCell on the secondary component carrier. Cell 1 is the cell used for connection setup with the power levels set according to Table A.6.5.4.1.5-1 for this test. Cell 2 is configured according to Annex C.1.1 and C.1.2.

Table 6.5.4.1.4.1-3: General test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on Pcell

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 1, 2 | Two radio channels are used for these two tests. |
| Active cell |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Cell 1: FR1 PCell Cell 2: FR1 SCell | FR1 PCell on RF channel number 1 FR1 SCell on RF channel number 2 |
| CP length |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | Normal |  |
| DRX |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Measurement gap pattern Id |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | OFF |  |
| Filter coefficient |  | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 0 | L3 filtering is not used |
| T1 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T2 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |
| T3 | s | Config 1,2,3, 4, 5, 6, 7, 8, 9 | 5 |  |

6.5.4.1.4.2	Test procedure

There are two cells: FR1 PCell (cell 1) and FR1 SCell (cell 2). Both NR uplink and supplementary uplink are broadcast by ServingCellConfigCommonSIB. In test 1, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, NR uplink of cell 2 is configured to UE. At the start of T2, a supplementary uplink of cell 2 is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the supplementary uplink is released through RRCReconfiguration.

In test 2, the test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, supplementary uplink on cell 2 is configured to UE. At the start of T2, a NR uplink is configured to UE through RRCReconfiguration, then UE shall start transmission both on the NR uplink and supplementary uplink. At the start of T3, the NR uplink is released through RRCReconfiguration.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Setup PCell (Cell 1) according to parameters given in Table 6.5.4.1.5-1. Propagation conditions are set according to Annex C clause C.2.2.

3.	For SCell (Cell 2), both NR uplink and supplementary uplink are broadcast by ServingCellConfigCommonSIB.

4.	For Test 1: NR uplink of SCell is configured to UE during T1

4.1 During time duration T1, NR uplink of SCell is configured to UE. Setup SCell (Cell 2) according to parameters given in Table 6.5.4.1.5-2

4.2 At the start of T2, a supplementary uplink of SCell (Cell 2) is configured to UE through RRCReconfiguration, then UE shall start transmission on both the NR uplink and supplementary uplink on SCell (Cell 2) within 20ms. If UE transmits data on both the NR uplink and supplementary uplink on SCell (Cell 2) within 20ms from the start of T2, then count a success for the event “reconfiguration” otherwise count a failure for event “reconfiguration”

4.3 At the start of T3, the supplementary uplink is released through RRCReconfiguration, then UE shall transmit data only on the NR uplink carrier on SCell (Cell 2) within 20ms. If UE stop transmitting data on supplementary uplink carrier on SCell (Cell 2) within 20ms from the start of T3, then count a success for the event “deconfiguration” otherwise count a failure for event “deconfiguration”.

5.	For Test 2: Supplementary uplink of SCell is configured to UE during T1

5.1 During time duration T1, Supplementary uplink of SCell is configured to UE. Setup SCell (Cell 2) according to parameters given in Table 6.5.4.1.5-2

5.2 At the start of T2, a NR uplink of SCell (Cell 2) is configured to UE through RRCReconfiguration, then UE shall start transmission on both the NR uplink and supplementary uplink on SCell (Cell 2) within 20ms. If UE transmits data on both the NR uplink and supplementary uplink on SCell (Cell 2) within 20ms from the start of T2, then count a success for the event “reconfiguration” otherwise count a failure for event “reconfiguration”

5.3 At the start of T3, the NR uplink is released through RRCReconfiguration, then UE shall transmit data only on the Supplementary uplink carrier on SCell (Cell 2) within 20ms. If UE stop transmitting data on NR uplink carrier on SCell (Cell 2) within 20ms from the start of T3, then count a success for the event “deconfiguration” otherwise count a failure for event “deconfiguration”.

6.	Repeat steps 1-5 until a test verdict has been achieved.

Each of the events “reconfiguration” and “deconfiguration” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1in Annex G.2 is achieved. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

6.5.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.5.4.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.8-1 Table H.3.8-2 |

6.5.4.1.5	Test requirement

Table 6.5.4.1.5.1-1 and 6.5.4.1.5-2 define the primary level settings including test tolerances for UE UL carrier RRC reconfiguration delay test.

Table 6.5.4.1.5-1: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on PCell (Cell 1)

| Parameter | Unit | Test Configuration | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 |  |  | 1 |  |  |
| TDD configuration |  | Conf 1, 2, 3 | N/A |  |  | N/A |  |  |
|  |  | Conf 4, 5, 6 | TDD Conf.1.1 |  |  | TDD Conf.1.1 |  |  |
|  |  | Conf 7, 8, 9 | TDD Conf.2.1 |  |  | TDD Conf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 2, 3 | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | Conf 4, 5, 6 | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | Conf 7, 8, 9 | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| PDSCH reference measurement channel as defined in A.3.1.1 |  | Conf 1, 2, 3 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | SR 2.1 TDD |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  | Conf 1, 2, 3 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  | Conf 1, 2, 3 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | Conf 4, 5, 6 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Conf 7, 8, 9 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | OP.1 |  |  | OP.1 |  |  |
| SSB configuration |  | Conf 1, 2, 3, 4, 5, 6 | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 7, 8, 9 | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  | SMTC.1 |  |  |
| CSI-RS for tracking |  | Conf 1 | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 2 | TRS.1.1 FDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 3 | TRS.1.1 FDD |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 4 | TRS.1.1 TDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 5 | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 6 | TRS.1.1 TDD |  |  | TRS.1.2 TDD |  |  |
|  |  | Conf 7 | TRS.1.2 TDD |  |  | TRS.1.1 FDD |  |  |
|  |  | Conf 8 | TRS.1.2 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Conf 9 | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1,2,3,4,5,6 | -102 |  |  | -102 |  |  |
|  |  | Conf 7,8,9 | -99 |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1,2,3,4,5,6 | -86 | -86 | -86 | -86 | -86 | -86 |
|  |  | Conf 7,8,9 | -83 | -83 | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1,2,3,4,5,6 | -57.9 | -57.9 | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 7,8,9 | -51.8 | -51.8 | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table 6.5.4.1.5-2 : NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on SCell (Cell 2)

| Parameter | Unit | Test Configuration | Test 1 |  |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T3 | T1 | T2 | T3 |
| Channel number |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 2 |  |  |  | 2 |  |  |
| TDD configuration |  | Conf 1, 4, 7 | N/A |  |  |  | N/A |  |  |
|  |  | Conf 2, 5, 8 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | Conf 3, 6, 9 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| BWchannel | MHz | Conf 1, 4, 7 | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | Conf 2, 5, 8 | 10: NRB,c = 52 |  |  |  | 10: NRB,c = 52 |  |  |
|  |  | Conf 3, 6, 9 | 40: NRB,c = 106 |  |  |  | 40: NRB,c = 106 |  |  |
| PUSCH parameters for NR UL carrier |  | Conf 1, 4, 7 | G-FR1-A3-10 in [28] | G-FR1-A3-10 in [28] | G-FR1-A3-10 in [28] |  | N/A | G-FR1-A3-10 in [28] | N/A |
|  |  | Conf 2, 5, 8 | G-FR1-A3-10 in [28] | G-FR1-A3-10 in [28] | G-FR1-A3-10 in [28] |  | N/A | G-FR1-A3-10 in [28] | N/A |
|  |  | Conf 3, 6, 9 | G-FR1-A3-14 in [28] | G-FR1-A3-14 in [28] | G-FR1-A3-14 in [28] |  | N/A | G-FR1-A3-14 in [28] | N/A |
| PUCCH parameters For NR UL carrier |  | Conf 1, 4, 7 | Table 8.3.3.1.2-1 in [28] | Table 8.3.3.1.2-1 in [28] | Table 8.3.3.1.2-1 in [28] |  | N/A | N/A | N/A |
|  |  | Conf 2, 5, 8 | Table 8.3.3.1.2-1 in [28] | Table 8.3.3.1.2-1 in [28] | Table 8.3.3.1.2-1 in [28] |  | N/A | N/A | N/A |
|  |  | Conf 3, 6, 9 | Table 8.3.3.1.2-2 in [28] | Table 8.3.3.1.2-2 in [28] | Table 8.3.3.1.2-2 in [28] |  | N/A | N/A | N/A |
| PUSCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | G-FR1-A3-10 in [28] | N/A |  | G-FR1-A3-10 in [28] | G-FR1-A3-10 in [28] | G-FR1-A3-10 in [28] |
|  |  | Conf 2, 5, 8 | N/A | G-FR1-A3-10 in [28] | N/A |  | G-FR1-A3-10 in [28] | G-FR1-A3-10 in [28] | G-FR1-A3-10 in [28] |
|  |  | Conf 3, 6, 9 | N/A | G-FR1-A3-14 in [28] | N/A |  | G-FR1-A3-14 in [28] | G-FR1-A3-14 in [28] | G-FR1-A3-14 in [28] |
| PUCCH parameters for supplementary UL |  | Conf 1, 4, 7 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [28] | Table 8.3.3.1.2-1 in [28] | Table 8.3.3.1.2-1 in [28] |
|  |  | Conf 2, 5, 8 | N/A | N/A | N/A |  | Table 8.3.3.1.2-1 in [28] | Table 8.3.3.1.2-1 in [28] | Table 8.3.3.1.2-1 in [28] |
|  |  | Conf 3, 6, 9 | N/A | N/A | N/A |  | Table 8.3.3.1.2-2 in [28] | Table 8.3.3.1.2-2 in [28] | Table 8.3.3.1.2-2 in [28] |
| PDSCH reference measurement channel as defined in A.3.1.1 |  | Conf 1, 4, 7 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | SR 2.1 TDD |  |  |  | SR 2.1 TDD |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  | Conf 1, 4, 7 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  | Conf 1, 4, 7 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |
|  |  | Conf 2, 5, 8 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |
|  |  | Conf 3, 6, 9 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern Note 1 |  | Conf 1, 2, 3 | OP.1 |  |  |  | OP.1 |  |  |
| SSB configuration |  | Conf 1, 2, 4, 5, 7,8 | SSB.1 FR1 |  |  |  | SSB.1 FR1 |  |  |
|  |  | Conf 3, 6, 9 | SSB.2 FR1 |  |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | SMTC.1 |  |  |  | SMTC.1 |  |  |
| DL initial BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |
| DL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |
| UL dedicated BWP configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |
| EPRE ratio of PSS to SSS | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 0 |  |  |  | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | -102 |  |  |  | -102 |  |  |
|  | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -102 |  |  |  | -102 |  |  |
|  |  | Conf 3, 6, 9 | -99 |  |  |  | -99 |  |  |
|  | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| Note 3 | dB | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 16 | 16 |  | 16 | 16 | 16 | 16 |
| SS-RSRP Note 3 | dBm/ SCS | Conf 1, 2, 4, 5, 7,8 | -86 | -86 |  | -86 | -86 | -86 | -86 |
|  |  | Conf 3, 6, 9 | -83 | -83 |  | -83 | -83 | -83 | -83 |
| Io Note 3 | dBm/ 9.36 MHz | Conf 1, 2, 4, 5, 7,8 | -57.9 | -57.9 |  | -57.9 | -57.9 | -57.9 | -57.9 |
|  | dBm/ 38.16MHz | Conf 3, 6, 9 | -51.8 | -51.8 |  | -51.8 | -51.8 | -51.8 | -51.8 |
| Propagation Condition |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | AWGN |  |  |  | AWGN |  |  |
| Antenna configuration |  | Conf 1, 2, 3, 4, 5, 6, 7, 8, 9 | 1 x 2 |  |  |  | 1 x 2 |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

In test 1 the UE shall be ready to start transmission on the supplementary uplink carrier on SCell within 20 ms from the start of T2.

In test 1 the UE shall stop the transmission on the supplementary uplink carrier on SCell within 20 ms from the start of T3.

In test 2 the UE shall be ready to start transmission on the NR uplink carrier on SCell within 20 ms from the start of T2.

In test 2 the UE shall stop the transmission on the NR uplink carrier on SCell within 20 ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

### 6.5.5	Link recovery procedures

#### 6.5.5.0	Minimum conformance requirements

##### 6.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB [ms] period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB [ms] period.

The value of TEvaluate_BFD_SSB is defined in Table 6.5.5.0.1-1 for FR1.

For FR1,

-	P=1/(1 – TSSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

Longer evaluation period would be expected if the combination of BFD-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 6.5.5.0.1-1: Evaluation period TEvaluate_BFD_SSB for FR1

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | max([50], ceil(5*P)*TSSB) |
| DRX cycle ≤ 320ms | max([50], ceil(7.5*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(5*P)*TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

When the radio link quality on all the configured RS resources in set  is worse than Qout_LR, Layer 1 of the UE shall send a beam failure instance indication to the higher layers. A Layer 3 filter may be applied to the beam failure instance indications as specified in TS 38.331 [13].

The beam failure instance evaluation for the configured RS resources in set  shall be performed as specified in section 6 in TS 38.213 [8]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval_BFD.

When DRX is not used, TIndication_interval_BFD is max(2ms, TBFD-RS,M), where TBFD-RS,M is the shortest periodicity of all configured RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB  in the set  or CSI-RS resource in the set .

When DRX is used, TIndication_interval_BFD is max(1.5*DRX_cycle_length, 1.5*TBFD-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms.

UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB [ms] period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table B.2.4.1 for a corresponding band.

The value of TEvaluate_CBD_SSB is defined in Table 6.5.5.0.1-2 for FR1.

For FR1,

-	P=1/(1 – TSSB/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

In both FR1 and FR2, if different SCS is used for SSB and CSI-RS, and the UE does not support simultaneousRxDataSSB-DiffNumerology, it is assumed that the SSB configured for candidate beam detection and each CSI-RS resource shall be TDMed transmitted.

Table 6.5.5.0.1-2: Evaluation period TEvaluate_CBD_SSB for FR1

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX | ceil([3]*P) * TSSB |
| DRX cycle ≤ 320ms | ceil([3]*P*1.5) * max(TDRX,TSSB) |
| DRX cycle > 320ms | ceil([3]*P) * TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.5.2.2, 8.5.4 and 8.5.5.2.

##### 6.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures

UE shall be able to evaluate whether the downlink radio link quality on the configured CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS [ms] period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS [ms] period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 8.5.3.2-1 for FR1.

For FR1,

-	P=1/(1 – TCSI-RS/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

Longer evaluation period would be expected if the combination of BFD-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

The values of MBFD used in Table 6.5.5.0.2-1 is defined as

-	MBFD = 10, if the CSI-RS resource configured for BFD is transmitted with Density = 3.

Table 6.5.5.0.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | max([50], [MBFD *P] * TCSI-RS) |
| DRX cycle ≤ 320ms | max([50], [1.5×MBFD *P]*max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD*P] * TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

When the radio link quality on all the configured RS resources in set  is worse than Qout_LR, Layer 1 of the UE shall send a beam failure instance indication to the higher layers. A Layer 3 filter may be applied to the beam failure instance indications as specified in TS 38.331 [13].

The beam failure instance evaluation for the configured RS resources in set  shall be performed as specified in section 6 in TS 38.213 [8]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval_BFD.

When DRX is not used, TIndication_interval_BFD is max(2ms, TBFD-RS,M), where TBFD-RS,M is the shortest periodicity of all configured RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB  in the set  or CSI-RS resource in the set .

When DRX is used, TIndication_interval_BFD is max(1.5*DRX_cycle_length, 1.5*TBFD-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms.

UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table B.2.4.2 for a corresponding band.

The value of TEvaluate_CBD_CSI-RS is defined in Table 6.5.5.0.2-2 for FR1.

For FR1,

-	P=1/(1 – TCSI-RS/MGRP), when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

In both FR1 and FR2, if different SCS is used for SSB and CSI-RS, and the UE does not support simultaneousRxDataSSB-DiffNumerology, it is assumed that the CSI-RS configured for candidate beam detection and each SSB shall be TDMed transmitted.

The values of MCBD used in Table 6.5.5.0.2-2 is defined as

-	MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3.

Table 6.5.5.0.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR1

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max([25], ceil(MCBD *P) * TCSI-RS) |
| DRX cycle ≤ 320ms | ceil(MCBD *P*N) * max(TDRX, TCSI-RS) |
| DRX cycle > 320ms | ceil(MCBD *P) *TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.5.3.2, 8.5.4 and 8.5.6.2.

#### 6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX

6.5.5.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

6.5.5.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.5.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.5.1.

6.5.5.1.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.5.1.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 6.5.5.1.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery

Figure 6.5.5.1.4-1: SNR and L1-RSRP variation for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

6.5.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.5.1.4.1-1.

Table 6.5.5.1.4.1-1: Supported test configurations for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Configuration | Description |
| --- | --- |
| 6.5.5.1-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.1-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.1-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.5.1.4.1-2.

Table 6.5.5.1.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 6.5.5.1.4.1-3.

2. Message contents are defined in clause 6.5.5.1.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.5.5.1.4.1-3: General test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  |  |  | Config 1 |  | FDD |  |
|  |  |  |  | Config 2, 3 |  | TDD |  |
| BWchannel |  |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration |  |  |  | Config 1 |  | Not Applicable |  |
|  |  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  |  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  |  | Config 3 |  | CR.2.1 TDD |  |
| SSB Configuration |  |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  |  | Config 3 |  | SSB.4 FR1 |  |
| SMTC Configuration |  |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  |  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  |  | Config 1, 2 |  | PRACH.2 FR1 |  |
|  |  |  |  | Config 3 |  | PRACH.2 FR1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters |  | DCI format |  |  |  | 1-0 |  |
|  |  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  |  | Aggregation level |  |  | CCE | 8 |  |
|  |  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  |  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  |  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  |  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 |  |
| gapOffset |  |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB | Config 1, 2 |  |  |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  | Config 3 |  |  |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  |  | 0, 1 |  |  |
| T310 Timer |  |  | ms |  | 1000 |  |  |
| N310 |  |  |  |  | 2 |  |  |
| T1 |  |  |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 0.37 |  |
| T3 |  |  |  |  | s | 0.24 |  |
| T4 |  |  |  |  | s | 0 |  |
| T5 |  |  |  |  | s | 0.17 |  |
| D1 |  |  |  |  | s | 0.13 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

6.5.5.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms) in test 1

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. The SS sends an RRCReconfiguration message to the UE to configure inter-frequency measurement.

3. The UE sends an RRCReconfigurationComplete message.

4.	Set the parameters of NR Cell 1 according to T1 in Table 6.5.5.1.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.5.1.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.5.1.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.5.5.1.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.5.5.1.5-1. T5 starts.

9.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect preamble on a beam associated with the candidate beam set q1before time point B

and

c) detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

10.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 6.5.5.1.5-1.

11.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

12.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

13.	Repeat steps 2-12 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.5.5.1.4.3-1: Common Exception messages for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-6 with Condition BFD Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED, GAP_NEEDED Table H.3.1-3 with Condition INTER-FREQ MO (where ssbFrequency is set to the ARFCN value of carrier center of High range) Table H.3.1-4 with A3-offset = 0 Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.5-4 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.5.5.1.4.3-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| duration | 2 |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.5.1.4.3-3: UE-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-200 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 6.5.5.1.4.3-4: Void

6.5.5.1.5	Test requirement

Tables 6.5.5.1.4.1-3 and 6.5.5.1.5-1 define the primary level settings including test tolerances for NR SA FR1 SSB-based beam failure detection and link recovery in DRX.

Table 6.5.5.1.5-1: NR Cell specific test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_ SSB of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1 Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.1.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR without the TT is modified as specified in clause |  |  |  |  |  |  |  |

Table 6.5.5.1.5-2: Void

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 130 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX

6.5.5.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

6.5.5.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.5.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.5.2.

6.5.5.2.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.5.2.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 6.5.5.2.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 6.5.5.2.4-1: SNR and L1-RSRP variation for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

6.5.5.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.5.2.4.1-1.

Table 6.5.5.2.4.1-1: Supported test configurations for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 6.5.5.2-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.2-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.2-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.5.2.4.1-2.

Table 6.5.5.2.4.1-2: Initial conditions for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 6.5.5.2.4.1-3.

2. Message contents are defined in clause 6.5.5.2.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.5.5.2.4.1-3: General test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Parameter |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Test 1 |  |
| Active PSCell |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |
|  |  |  | Config 2, 3 |  | TDD |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |
| DL initial BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration |  |  | Config 1 |  | Not Applicable |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |
| SSB Configuration |  |  | Config 1 |  | SSB.3 FR1 |  |
|  |  |  | Config 2 |  | SSB.3 FR1 |  |
|  |  |  | Config 3 |  | SSB.4 FR1 |  |
| SMTC Configuration |  |  | Config 1, 2 |  | SMTC.1 |  |
|  |  |  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  | 15 KHz |  |
|  |  |  | Config 3 |  | 30 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  | PRACH.2 FR1 |  |
|  |  |  | Config 3 |  | PRACH.2 FR1 |  |
| SSB Index assigned as BFD RS (q0) |  |  |  |  | 0 |  |
| SSB Index assigned as CBD RS (q1) |  |  |  |  | 1 |  |
| OCNG parameters |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  | 2 |  |
|  | Aggregation level |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  | 6 |  |
| DRX |  |  |  |  | DRX.7 | see clause A.3.3.7 of TS 38.133 [6] |
| Gap pattern ID |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1, 2 |  | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 |  | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  |  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  |  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| CSI-RS for tracking |  | Config 1 |  |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  |
| SSB Index assigned as RLM RS |  |  |  | 0, 1 |  |  |
| T310 Timer |  | ms |  | 1000 |  |  |
| N310 |  |  |  | 2 |  |  |
| T1 |  |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  | s | 5.17 |  |
| T3 |  |  |  | s | 3.24 |  |
| T4 |  |  |  | s | 0 |  |
| T5 |  |  |  | s | 1.97 |  |
| D1 |  |  |  | s | 1.93 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |

6.5.5.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 6.5.5.2.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.5.2.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.5.2.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.5.5.2.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.5.5.2.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect preamble on a beam associated with the candidate beam set q1before time point B

and

c) detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 6.5.5.2.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.5.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.5.5.2.4.3-1: Common Exception messages for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-10A Table H.3.5-4 Table H.3.7-1 with Condition DRX.7 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.5.5.2.4.3-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| duration | 2 |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.5.2.4.3-3: UE-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-200 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 6.5.5.2.4.3-4: Void

6.5.5.2.5	Test requirement

Tables 6.5.5.2.4.1-3 and 6.5.5.2.5-1 define the primary level settings including test tolerances for NR SA FR1 SSB-based beam failure detection and link recovery in DRX.

Table 6.5.5.2.5-1: NR Cell specific test parameters for NR SA FR1 SSB-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_ SSB of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_SSB of set q1 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.2.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR without the TT is modified as specified in clause D.4.1. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 1930 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX

6.5.5.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. To test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

6.5.5.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.5.5.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.5.3.

6.5.5.3.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.5.3.4-1 shows the five different time durations and the corresponding variation of the downlink SNR of the CSI-RS in set q0 in the active cell to emulate CSI-RS based beam failure and the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery.

Figure 6.5.5.3.4-1: SNR and L1-RSRP variation for NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX

6.5.5.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.5.3.4.1-1.

Table 6.5.5.3.4.1-1: Supported test configurations for NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 6.5.5.3-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.3-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.3-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.5.3.4.1-2.

Table 6.5.5.3.4.1-2: Initial conditions for NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 6.5.5.3.4.1-3.

2. Message contents are defined in clause 6.5.5.3.4.3.

3. There is one NR carrier and one NR cell specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.5.5.3.4.1-3: General test parameters for NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode | Config 1 |  |  | FDD |  |
|  | Config 2, 3 |  |  | TDD |  |
| TDD Configuration | Config 1 |  |  | Not Applicable |  |
|  | Config 2 |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  | TDDConf.2.1 |  |
| CORESET Reference Channel | Config 1 |  |  | CR.1.1 FDD |  |
|  | Config 2 |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  | SSB.3 FR1 |  |
|  | Config 2 |  |  | SSB.3 FR1 |  |
|  | Config 3 |  |  | SSB.4 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  | SMTC.1 |  |
|  | Config 3 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  | 15 KHz |  |
|  | Config 3 |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2 |  |  | PRACH.4 FR1 |  |
|  | Config 3 |  |  | PRACH.4 FR1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | OFF |  |
| Gap pattern ID |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  | 1 | N |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1, 2 | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], section 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], section 5.17 |
| CSI-RS configuration for q0 and q1 | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| TRS configuration | Config 1 |  |  | TRS.1.1 FDD |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 0.2 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 0.18 |  |
| T3 |  |  | s | 0.14 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 0.08 |  |
| D1 |  |  | s | 0.04 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

6.5.5.3.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is not enabled.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 6.5.5.3.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.5.3.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.5.3.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.5.5.3.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.5.5.3.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect preamble on a beam associated with the candidate beam set q1before time point B

and

c) detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 6.5.5.3.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.5.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.5.3.4.3-1: Common Exception messages for NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with Condition CSI-RS BFD Table H.3.1-10 with Condition CSI-RS Table H.3.1-10A Table H.3.5-4 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.5.5.3.4.3-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| duration | 2 |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.5.3.4.3-3: UE-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-200 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 6.5.5.3.4.3-4: Void

Table 6.5.5.3.4.3-5: NZP-CSI-RS-Resource

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| powerControlOffsetSS | db0 |  |  |
| } |  |  |  |

6.5.5.3.5	Test requirement

Tables 6.5.5.3.4.1-3 and 6.5.5.3.5-1 define the primary level settings including test tolerances for NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX.

Table 6.5.5.3.5-1: NR Cell specific test parameters for NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_CSI-RS of set q1 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.3.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR without the TT is modified as specified in clause D.4.1. |  |  |  |  |  |  |  |

The UE behaviour in each test during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 40 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

6.5.5.4.1	Test purpose

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. To test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in TS 38.133 [6] clause 8.5.

6.5.5.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.5.5.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.5.4.

6.5.5.4.4	Test description

The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 6.5.5.4.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate CSI-RS based beam failure. Figure 6.5.5.4.4-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery.

Figure 6.5.5.4.4-1: SNR and L1-RSRP variation for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

6.5.5.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.5.4.4.1-1.

Table 6.5.5.4.4.1-1: Supported test configurations for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 6.5.5.4-1 | FDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.4-2 | TDD duplex mode, 15 kHz SSB SCS, 10MHz bandwidth |
| 6.5.5.4-3 | TDD duplex mode, 30 kHz SSB SCS, 40MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR1 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.5.4.4.1-2.

Table 6.5.5.4.4.1-2: Initial conditions for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1. The general test parameter settings are set up according to Table 6.5.5.4.4.1-3.

2. Message contents are defined in clause 6.5.5.4.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.5.5.4.4.1-3: General test parameters for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active PCell |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  | 1 |  |
| Duplex mode | Config 1 |  |  | FDD |  |
|  | Config 2, 3 |  |  | TDD |  |
| TDD Configuration | Config 1 |  |  | Not Applicable |  |
|  | Config 2 |  |  | TDDConf.1.1 |  |
|  | Config 3 |  |  | TDDConf.2.1 |  |
| CORESET Reference Channel | Config 1 |  |  | CR.1.1 FDD |  |
|  | Config 2 |  |  | CR.1.1 TDD |  |
|  | Config 3 |  |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  |  | SSB.3 FR1 |  |
|  | Config 2 |  |  | SSB.3 FR1 |  |
|  | Config 3 |  |  | SSB.4 FR1 |  |
| SMTC Configuration | Config 1, 2 |  |  | SMTC.1 |  |
|  | Config 3 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  |  | 15 KHz |  |
|  | Config 3 |  |  | 30 KHz |  |
| PRACH Configuration | Config 1, 2 |  |  | PRACH.4 FR1 |  |
|  | Config 3 |  |  | PRACH.4 FR1 |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  |  | 0 |  |
| OCNG parameters |  |  |  | OP.1 |  |
| CP length |  |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 2x2 Low |  |
| Beam failure detection transmission parameters | DCI format |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  | 2 |  |
|  | Aggregation level |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  | REG bundle size |  |
|  | REG bundle size |  |  | 6 |  |
| DRX |  |  |  | DRX.7 |  |
| Gap pattern ID |  |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1, 2 | dBm/SCS kHz | -98 | Threshold used for Qin_LR_SSB |
|  |  | Config 3 | dBm/SCS kHz | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  | n1 | see TS 38.321 [12], section 5.17 |
| beamFailureDetectionTimer |  |  |  | pbfd4 | see TS 38.321 [12], section 5.17 |
| CSI-RS configuration for q0 and q1 | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  |  | CSI-RS.1.1 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.1 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.1 TDD |  |
| TRS configuration | Config 1 |  |  | TRS.1.1 FDD |  |
|  | Config 2 |  |  | TRS.1.1 TDD |  |
|  | Config 3 |  |  | TRS.1.2 TDD |  |
| CSI-RS-Index assigned as RLM RS | Config 1 |  |  | CSI-RS.1.2 FDD |  |
|  | Config 2 |  |  | CSI-RS.1.2 TDD |  |
|  | Config 3 |  |  | CSI-RS.2.2 TDD |  |
| T310 Timer |  |  | ms | 1000 |  |
| N310 |  |  |  | 2 |  |
| T1 |  |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  |  | s | 8.37 |  |
| T3 |  |  | s | 6.44 |  |
| T4 |  |  | s | 0 |  |
| T5 |  |  | s | 1.97 |  |
| D1 |  |  | s | 1.93 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

6.5.5.4.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 6.5.5.4.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 6.5.5.4.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 6.5.5.4.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 6.5.5.4.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 6.5.5.4.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect preamble on a beam associated with the candidate beam set q1before time point B

and

c) detects preamble on a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 6.5.5.4.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.5.5.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.5.4.4.3-1: Common Exception messages for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with Condition CSI-RS BFD Table H.3.1-10 with Condition CSI-RS Table H.3.1-10A Table H.3.5-4 Table H.3.7-1 with Condition DRX.7Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.5.5.4.4.3-2: PDCCH Search Space

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| duration | 2 |  |  |
| monitoringSymbolsWithinSlot | 10000000000000 | Symbol 0 |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n0 |  |  |
| aggregationLevel4 | n0 |  |  |
| aggregationLevel8 | n1 | AL8 |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-0-And-1-0 | DCI Format 1_0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.5.4.4.3-3: UE-TimersAndConstants

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-200 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE-TimersAndConstants ::= SEQUENCE { |  |  |  |
| n310 | n2 |  |  |
| } |  |  |  |

Table 6.5.5.4.4.3-4: Void

Table 6.5.5.4.4.3-5: NZP-CSI-RS-Resource

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-85 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| powerControlOffsetSS | db0 |  |  |
| } |  |  |  |

6.5.5.4.5	Test requirement

Tables 6.5.5.4.4.1-3 and 6.5.5.4.5-1 define the primary level settings including test tolerances for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX.

Table 6.5.5.4.5-1: NR Cell specific test parameters for NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 2 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
|  | Config 3 |  | 5.8 | -2.2 | -12.8 | -12.8 | -12.8 |
| SNR_CSI-RS of set q1 | Config 1 | dB | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 2 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
|  | Config 3 |  | -10.2 | -10.2 | 10.2 | 10.2 | 10.2 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 2 |  | -108.2 | -108.2 | -87.8 | -87.8 | -87.8 |
|  | Config 3 |  | -105.2 | -105.2 | -84.8 | -84.8 | -84.8 |
|  | Config 1 | dBm/15 kHz | -98 |  |  |  |  |
|  | Config 2 |  | -98 |  |  |  |  |
|  | Config 3 |  | -98 |  |  |  |  |
| Propagation condition |  |  | TDL-C 300ns 100Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 6.5.5.4.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR without the TT is modified as specified in clause D.4.1. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 1930 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

### 6.5.6	Active BWP switch delay

#### 6.5.6.1	DCI-based and time-based active BWP switch

##### 6.5.6.1.0	Minimum conformance requirements

6.5.6.1.0.1	Minimum conformance requirements for DCI-based and time-based active BWP switch

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

The UE is not required to transmit UL signals or receive DL signals until the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n except DCI triggering BWP switch on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this clause when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where slot n is the first slot of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [13] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay after bwp-InactivityTimer [13] expires on the cell where timer-based BWP switch occurs.

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 6.5.6.1.0.1-1.

Table 6.5.6.1.0.1-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Provided the UE does not have the required TCI-state information to receive PDCCH and PDSCH in the new BWP, the UE shall use old TCI-states before the BWP switch until a new MAC CE updating the required TCI-state information for PDCCH and PDSCH is received after the BWP switch.

If UE has the information on the required TCI-state information to receive PDCCH and PDSCH in the new BWP,

-	UE shall be able to receive PDCCH and PDSCH with old TCI-states before the delay as specified in TS 38.133 [6] Clause 8.10 in the new BWP.

-	UE shall be able to receive PDCCH and PDSCH with new TCI-states after the delay as specified in TS 38.133 [6] Clause 8.10 in the new BWP.

If the BWP switch is triggered within DRX active time, and one of the two BWPs in a BWP switching is a dormant BWP [TS 38.321, 12], UE shall be able to complete active BWP switching within

-	TBWPswitchDelay, provided that the BWP switching request is received in any of the first 3 OFDM symbols of a slot corresponding to the serving cell where BWP switching occurs, or

-	TBWPswitchDelay + 1, provided that the BWP switching request is received after the first 3 OFDM symbols of a slot corresponding to the serving cell where BWP switching occurs

When either of the DCI-based, timer-based or RRC-based downlink BWP switch and/or uplink BWP switch occur on multiple CCs simultaneously or over partially overlapping period, the interruption requirements described in this section apply for each BWP switch.

When UE receives a DCI indicating UE to switch its active BWP involving changes in any of the parameters listed in Table 6.5.6.1.0.1-3, the UE is allowed to cause interruption of up to X slot to other active serving cells if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 6.5.6.1.0.1-3 and the UE is capable of per-FR gap the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 6.5.6.1.0.1-2. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in TS 38.133 [6] clause 8.6.2. Interruptions are not allowed during BWP switch involving any other parameter change.

When a BWP timer bwp-InactivityTimer defined in TS 38.331 [13] expires, UE is allowed to cause interruption of up to X slot to other active serving cells due to switching its active BWP involving changes in any of the parameters listed in Table 6.5.6.1.0.1-3 if the UE is not capable of per-FR gap, or if the BWP switching involves SCS changing. When the BWP switch imposes changes in any of the parameters listed in Table 6.5.6.1.0.1-3 and the UE is capable of per-FR gap, the UE is allowed to cause interruption of up to X slot to other active serving cells in the same frequency range wherein the UE is performing BWP switching. X is defined in Table 6.5.6.1.0.1-2. The starting time of interruption is only allowed within the BWP switching delay TBWPswitchDelay as defined in TS 38.133 [6] clause 8.6.2. Interruptions are not allowed during BWP switch involving any other parameter change.

Table 6.5.6.1.0.1-2: Interruption length X

|  | NR Slot length (ms) | interruption length X (slots) |
| --- | --- | --- |
| 0 | 1 | 1 |
| 1 | 0.5 | 1 |
| 2 | 0.25 | 3 |
| 3 | 0.125 | 5 |
| Note1: void |  |  |

Table 6.5.6.1.0.1-3: Parameters which cause interruption other than SCS

| Parameters | Comment |
| --- | --- |
| locationAndBandwidth | From TS 38.331 [13] |
| nrofSRS-Ports |  |
| maxMIMO-Layers-r16 |  |

The normative reference for this requirement is TS 38.133 [6] clauses 8.6.2 and 8.2.1.2.7

##### 6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

6.5.6.1.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6, and interruption requirement on other active serving cell defined in TS 38.133 [6] clause 8.2.2.2.5.

6.5.6.1.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards and support BWP switch and uplink CA.

6.5.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.6.1.1.

6.5.6.1.1.4	Test description

6.5.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.6.1.1.4.1-1.

Table 6.5.6.1.1.4.1-1: Supported test configurations for NR SA FR1-FR1 DCI-based DL active BWP switch with SCell in non-DRX

| Config | Description |
| --- | --- |
| 6.5.6.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – FDD duplex mode |
| 6.5.6.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 6.5.6.1.1-3 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD – FDD duplex mode |
| 6.5.6.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 6.5.6.1.1-5 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.6.1.1.4.1-2.

Table 6.5.6.1.1.4.1-2: Initial conditions for NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.5.6.1.1.4.1-3.

2. Message contents are defined in clause 6.5.6.1.1.4.3.

3. The test scenario comprises of one NR PCell (Cell 1) and one NR SCell (Cell 2). Cell 1 and Cell 2 are configured according to Annex C.1.2 and C.1.3.

Table 6.5.6.1.1.4.1-3: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active SCell |  | Cell 2 | SCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and SCell |
| bwp-InactivityTimer | ms | 200 |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

6.5.6.1.1.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

PDCCHs indicating new transmissions shall be sent continuously on SCell (Cell 2) to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (SCell) on radio channel 2 (SCC).

-	UE is configured with 2 different UE-specific downlink bandwidth parts for PCell, BWP-1 and BWP-2, in Cell 1 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is configured with 1 UE-specific downlink bandwidth parts the same as initial BWP for SCell, BWP-0 in Cell 2 before starting the test.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-0 in SCell.

-	UE is configured with a bwp-InactivityTimer timer value for PCell.

All cells have constant signal levels throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1. Set the parameters according to Tables 6.5.6.1.1.4.1-3 and 6.5.6.1.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	The SS shall send a DCI format 1_1 command for PCell DL BWP switch.

4.	The UE shall receive the DCI format 1_1 command in PCell’s slot # denoted i, then T1 starts and the UE switch its bandwidth part from BWP-1 to BWP-2:

a) If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1)

and

b) If the UE starts to report valid ACK/NACK for SCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+ Interruption length + k1)

and

c) If the number of  consecutive missing ACK/NACK for SCell is no more than 2 for configuation 6.5.6.1.1-1/3 and no more than 1 for configuation 6.5.6.1.1-2/4/5

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test and go to step 7

5.	If the UE sends valid ACK/NACK for the PCell on BWP-2, T2 starts. During T2, the SS shall not transmit DCI format for PDSCH reception on PCell.

6.	T3 starts from the first slot #j of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires. Then, the UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1:

a) If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1)

and

b) If the UE starts to report valid ACK/NACK for SCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+ Interruption length + k1)

and

c) If the number of  consecutive missing ACK/NACK for SCell is no more than 2 for configuation 6.5.6.1.1-1/3 and no more than 1 for configuation 6.5.6.1.1-2/4/5

Then, the number of successful subtests is increased by one. Otherwise, count a fail for the test and go to step 7

7.	If the switch fails, switch off the UE.

Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

and then

The SS shall send a RRCReconfiguration message with updated bandwidth part configuration Active BWP-1 in table 6.5.6.1.1.5-1 for DL BWP switch. Then the UE shall transmit a RRCReconfigurationComplete message.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The SS verifies the DL BWP switch time in PCell by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

The SS verifies that potential interruption to SCell is carried out in the correct time span by monitoring ACK/NACK sent in SCell during BWP switch of PCell, respectively.

Interruption length is defined in TS 38.133 [6] Table 8.2.2.2.5-1-1.

If all subtests pass, the test passes. If one subtest fails, the test fails.

6.5.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.6.1.1.4.3-1: Common Exception messages for NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.5.6.1.1.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | Downlink BWP-1 |  |  |
| BWP-Downlink[2] | Downlink BWP-2 |  |  |
| } |  |  |  |
| bwp-InactivityTimer | ms200 |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | Uplink BWP-1 |  |  |
| BWP-Uplink[2] | Uplink BWP-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-3: BWP-Downlink

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP-Id1 |
|  | 2 |  | BWP-Id2 |
| } |  |  |  |

Table 6.5.6.1.1.4.3-4: BWP-Uplink

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP-Id1 |
|  | 2 |  | BWP-Id2 |
| } |  |  |  |

Table 6.5.6.1.1.4.3-5: BWP

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set the value according to the configuration of active BWP-1/2 |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| cyclicPrefix | Not present |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-6: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 4 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 |  |
| k0 | TBWPswitchDelay | Defined in Table 6.5.6.1.0.1-1 | The DCI indicating BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[4] SEQUENCE { |  | entry 4 |  |
| k0 | 1 |  | First DCI right after DCI-based BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.1.4.3-7: CellGroupConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigCommon | ServingCellConfigCommon |  |  |
| sCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.6.1.1.5	Test requirements

Tables 6.5.6.1.1.4.1-3 and 6.5.6.1.1.5-1 define the primary level settings including test tolerances.

Table 6.5.6.1.1.5-1: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  | Unit | Cell 1 |  | Cell2 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |  | FR1 |
| Duplex mode |  | Config 1 |  | FDD |  | FDD |
|  |  | Config 2,5 |  | TDD |  | TDD |
|  |  | Config 3 |  | TDD |  | FDD |
|  |  | Config 4 |  | FDD |  | TDD |
| TDD configuration |  | Config 1 |  | Not Applicable |  | Not Applicable |
|  |  | Config 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |
|  |  | Config 3 |  | TDDConf.1.1 |  | Not Applicable |
|  |  | Config 4 |  | Not Applicable |  | TDDConf.1.1 |
|  |  | Config 5 |  | TDDConf.2.1 |  | TDDConf.2.1 |
| BWchannel |  | Config 1,2,3,4 |  | 10 MHz: NRB,c = 52 |  | 10 MHz: NRB,c = 52 |
|  |  | Config 5 |  | 40 MHz: NRB,c = 106 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  | 1, 2 |  | 0 |
| Initial DL BWP Configuration |  |  |  | DLBWP.0.2Note4 |  |  |
| Initial UL BWP Configuration |  |  |  | ULBWP.0.2Note4 |  |  |
| Active DL BWP-0 Configuration |  |  |  | NA | DLBWP.0.2Note4 |  |
| Active DL BWP-1 Configuration |  |  |  | DLBWP.1.1Note4 |  | NA |
| Active DL BWP-2 Configuration |  |  |  | DLBWP.1.3Note4 |  | NA |
| Active UL BWP-0 Configuration |  |  |  | N.A. |  | ULBWP.0.2Note4 |
| Active UL BWP-1 Configuration |  |  |  | ULBWP.1.1Note4 |  | N.A. |
| Active UL BWP-2 Configuration |  |  |  | ULBWP.1.3Note4 |  | N.A. |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | SR.1.1 FDD |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |
|  |  | Config 3 |  | SR.1.1 TDD |  | SR.1.1 FDD |
|  |  | Config 4 |  | SR.1.1 FDD |  | SR.1.1 TDD |
|  |  | Config 5 |  | SR.2.1 TDD |  | SR.2.1 TDD |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |  | CR.1.1 FDD |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |
|  |  | Config 3 |  | CR.1.1 TDD |  | CR.1.1 FDD |
|  |  | Config 4 |  | CR.1.1 FDD |  | CR.1.1 TDD |
|  |  | Config 5 |  | CR.2.1 TDD |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD |  | CCR.1.1 FDD |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |
|  |  | Config 3 |  | CCR.1.1 TDD |  | CCR.1.1 FDD |
|  |  | Config 4 |  | CCR.1.1 FDD |  | CCR.1.1 TDD |
|  |  | Config 5 |  | CCR.2.3 TDD |  | CCR.2.3 TDD |
| OCNG Patterns |  |  |  | OP.1 |  |  |
| SSB Configuration |  | Config 1,2,3,4 |  | SSB.1 FR1 |  |  |
|  |  | Config 5 |  | SSB.2 FR1 |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| NocNote 2 | Config 1,2,3,4 |  | dBm/SCS | -104 |  | -104 |
|  | Config 5 |  |  | -101 |  | -101 |
| NocNote 2 |  |  | dBm/15KHz | -104 |  | -104 |
| SS-RSRP Note 3 | Config 1,2,3,4 |  | dBm/SCS | -87 |  | -87 |
|  | Config 5 |  |  | -84 |  | -84 |
| Ês/Iot |  |  | dB | 17 |  | 17 |
| Ês/Noc |  |  | dB | 17 |  | 17 |
| IoNote3 |  | Config 1,2,3,4 | dBm/ 9.36MHz | -58.96 |  | -58.96 |
|  |  | Config 5 | dBm/ 38.16MHz | -52.86 |  | -52.86 |
| Propagation Condition |  |  |  | AWGN |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3 SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |  |

During T1, the UE shall start to send the ACK/NACK for PSCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PSCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in TS 38.133 [6] Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1 and T3, the start time of SCell interruption during PCell active BWP switch shall not happen outside the BWP switch delay.

The interruption of SCell shall not be longer than the interruption duration specified for active BWP switch in TS 38.133 [6] clause 8.2.2.2.5.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch interruption to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after the beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

##### 6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX

6.5.6.1.2.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6.

6.5.6.1.2.2	Test applicability

This test applies to all types of NR UE release 15 onwards and support BWP switch.

6.5.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.6.1.2.

6.5.6.1.2.4	Test description

6.5.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.6.1.2.4.1-1.

Table 6.5.6.1.2.4.1-1: Supported test configurations for NR SA FR1 DCI-based DL active BWP switch in non-DRX

| Config | Description |
| --- | --- |
| 6.5.6.1.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.5.6.1.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.5.6.1.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: A UE which fulfils the requirements in test case A.6.5.6.1.1 can skip the test cases in A.6.5.6.1.2 |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.6.1.2.4.1-2.

Table 6.5.6.1.2.4.1-2: Initial conditions for NR SA FR1 DCI-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.5.6.1.2.4.1-3.

2. Message contents are defined in clause 6.5.6.1.2.4.3.

3. The test scenario comprises of one NR PCell (Cell 1). Cell 1 is configured according to Annex C.1.2 and C.1.3.

Table 6.5.6.1.2.4.1-3: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell1 on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| bwp-InactivityTimer | ms | 200 |  |
| T1 | s | 0.2 |  |
| T2 | s | 0.2 |  |
| T3 | s | 0.2 |  |

6.5.6.1.2.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE is configured with 2 different UE-specific downlink bandwidth parts, BWP-1 and BWP-2 before starting the test. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1.

-	UE is configured with a bwp-InactivityTimer timer value for PCell.

The Cell 1 has constant signal levels throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 6.5.6.1.2.4.1-3 and 6.5.6.1.2.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	The SS shall send a DCI format 1_1 command for Cell 1 DL BWP switch.

4.	The UE shall receive the DCI format 1_1 command in slot # denoted i, then T1 starts and the UE switch its bandwidth part from BWP-1 to BWP-2:

If the UE starts to report valid ACK/NACK for Cell 1 from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1). The number of successful subtest is increased by one. Otherwise, count a fail for the test and go to step 7

5.	If the UE sends valid ACK/NACK for the Cell 1 on BWP-2, T2 starts. During T2, the SS shall not transmit DCI format for PDSCH reception on Cell 1.

6.	T3 starts from the first slot #j of the DL subframe immediately after the slot wherein bwp-InactivityTimer timer expires. Then, the UE shall switch its bandwidth part from BWP-2 back to the default bandwidth part – BWP-1:

If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (j+TBWPswitchDelay+k1). The number of successful subtest is increased by one. Otherwise, count a fail for the test and go to step 7

7.	If the switch fails, switch off the UE.

Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

and then

The SS shall send a RRCReconfiguration message with updated bandwidth part configuration Active BWP-1 in table 6.5.6.1.2.5-1 for DL BWP switch. Then the UE shall transmit a RRCReconfigurationComplete message.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The SS verifies the DL BWP switch time by counting the slots from the time when the BWP switch command is received or bwp-InactivityTimer timer expires till an ACK/NACK is received.

If all subtests pass, the test passes. If one subtest fails, the test fails.

6.5.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.6.1.2.4.3-1: Common Exception messages for NR SA FR1 DCI-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.5.6.1.2.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | Downlink BWP-1 |  |  |
| BWP-Downlink[2] | Downlink BWP-2 |  |  |
| } |  |  |  |
| bwp-InactivityTimer | ms200 |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | Uplink BWP-1 |  |  |
| BWP-Uplink[2] | Uplink BWP-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.6.1.2.4.3-3: BWP-Downlink

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP-Id1 |
|  | 2 |  | BWP-Id2 |
| } |  |  |  |

Table 6.5.6.1.2.4.3-4: BWP-Uplink

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP-Id1 |
|  | 2 |  | BWP-Id2 |
| } |  |  |  |

Table 6.5.6.1.2.4.3-5: BWP

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set the value according to the configuration of active BWP-1/2 |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| cyclicPrefix | Not present |  |  |
| } |  |  |  |

Table 6.5.6.1.2.4.3-6: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 4 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | S=2, L=6 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[3] SEQUENCE { |  | entry 3 |  |
| k0 | TBWPswitchDelay | Defined in Table 6.5.6.1.0.1-1 | The DCI indicating BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[4] SEQUENCE { |  | entry 4 |  |
| k0 | 1 |  | First DCI right after DCI-based BWP switch |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| } |  |  |  |

6.5.6.1.2.5	Test requirements

Tables 6.5.6.1.2.4.1-3 and 6.5.6.1.2.5-1 define the primary level settings including test tolerances.

Table 6.5.6.1.2.5-1: NR Cell specific test parameters for DL BWP switch in SA

| Parameter |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1 |  | FDD |
|  |  |  | Config 2,3 |  | TDD |
| TDD configuration |  |  | Config 1 |  | Not Applicable |
|  |  |  | Config 2 |  | TDDConf.1.1 |
|  |  |  | Config 3 |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  | 1, 2 |
| Initial DL BWP Configuration |  |  | Config 1,2,3 |  | DLBWP.0.2 Note 4 |
| Active DL BWP-1 Configuration |  |  | Config 1,2,3 |  | DLBWP.1.1 Note 4 |
| Active DL BWP-2 Configuration |  |  | Config 1,2,3 |  | DLBWP.1.3 Note 4 |
| Initial UL BWP Configuration |  |  | Config 1,2,3 |  | ULBWP.0.2 Note 4 |
| Active UL BWP-1 Configuration |  |  | Config 1,2,3 |  | ULBWP.1.1 Note 4 |
| Active UL BWP-2 Configuration |  |  | Config 1,2,3 |  | ULBWP.1.3 Note 4 |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD |
|  |  |  | Config 2 |  | SR.1.1 TDD |
|  |  |  | Config 3 |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1 |  | CR.1.1 FDD |
|  |  |  | Config 2 |  | CR.1.1 TDD |
|  |  |  | Config 3 |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1 |  | CCR.1.1 FDD |
|  |  |  | Config 2 |  | CCR.1.1 TDD |
|  |  |  | Config 3 |  | CCR.2.3 TDD |
| OCNG Patterns |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2 |  | SSB.1 FR1 |
|  |  |  | Config 3 |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  | SMTC.1 |
| Correlation Matrix and Antenna Configuration |  |  |  |  | 1x2 Low |
| TRS Configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| NocNote 2 | Config 1,2 |  |  | dBm/SCS | -104 |
|  | Config 3 |  |  |  | -101 |
| NocNote 2 |  |  |  | dBm/15kHz | -104 |
| SS-RSRP Note 3 | Config 1,2 |  |  | dBm/SCS | -87 |
|  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  | dB | 17 |
| Ês/Noc |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2 | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 | dBm/ 38.16MHz | -52.86 |
| Propagation Condition |  |  |  |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |

During T1, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot (i+TBWPswitchDelay+k1).

During T3, the UE shall start to send the ACK/NACK for PCell from the first UL slot that occurs right after the beginning of DL slot (j+TBWPswitchDelay+k1).

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

Depending on UE capability bwp-SwitchingDelay [13], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in TS 38.133 [6] Table 8.6.2-1.

All of the above test requirements shall be fulfilled in order for the observed PCell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	During T1, T3 if there are no uplink resources for reporting the ACK/NACK in the first UL slot that occurs after beginning of DL slot (i+ TBWPswitchDelay+k1), (j+ TBWPswitchDelay+k1), then the UE shall use the next available uplink resource for reporting the corresponding ACK/NACK.

#### 6.5.6.2	RRC-based active BWP switch

##### 6.5.6.2.0	Minimum conformance requirements

6.5.6.2.0.1	Minimum conformance requirements for RRC-based active BWP switch

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n, where

DL slot n is the last slot containing the RRC command, and

TRRCprocessingDelay is the length of the RRC procedure delay in ms as defined in clause 12 in TS 38.331 [13], and

TBWPswitchDelayRRC=6ms is the time used by the UE to perform BWP switch.

The UE is not required to transmit UL signals or receive DL signals during the time defined by TRRCprocessingDelay+TBWPswitchDelayRRC on the cell where RRC-based BWP switch occurs.

The normative reference for this requirement is TS 38.133 [6] clauses 8.6.3.

##### 6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX

6.5.6.2.1.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement for RRC-based BWP switch defined in TS 38.133 [6] clause 8.6.

6.5.6.2.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards and support BWP switch.

6.5.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.6.2.1.

6.5.6.2.1.4	Test description

6.5.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.6.2.1.4.1-1.

Table 6.5.6.2.1.4.1-1: Supported test configurations for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Config | Description |
| --- | --- |
| 6.5.6.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.5.6.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.5.6.2.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.6.2.1.4.1-2.

Table 6.5.6.2.1.4.1-2: Initial conditions for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.5.6.2.1.4.1-3.

2. Message contents are defined in clause 6.5.6.2.1.4.3.

3. The test scenario comprises of one NR Cell (Cell 1). Cell 1 is configured according to Annex C.1.2 and C.1.3.

Table 6.5.6.2.1.4.1-3: General test parameters for DL BWP switch in SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| T1 | s | 0.2 |  |

6.5.6.2.1.4.2	Test procedure

The test consists of 1 time period, with duration of T1.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1  to ensure that the UE will have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 on radio channel 1.

-	UE has bandwidth part BWP-1 in its RRC-configuration for Cell 1.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 of initial condition in Cell 1.

Cell 1 has constant signal level throughout the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 6.5.6.2.1.4.1-3 and 6.5.6.2.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3.	The SS shall send a RRCReconfiguration message with updated bandwidth part configuration for DL BWP switch, change the BWP according to the initial condition of  Active BWP-1 in table 6.5.6.2.1.5-1.

4.	The UE shall transmit a RRCReconfigurationComplete message.

5.	The SS shall send a RRCReconfiguration message with updated bandwidth part configuration for DL BWP switch, change the BWP according to the final condition of  Active BWP-1 in table 6.5.6.2.1.5-1.T1 starts.

6.	The UE shall receive the RRCReconfiguration in PCell’s slot # denoted i and reconfigure its bandwidth part with the updated bandwidth part configuration:

7	If the UE starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot  i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After the SS receive the ACK/NACK in step 5) or when T1 expires, the SS shall transmit RRCRelease message to release the RRC connection.

9.	After the RRC connection release, the SS::

-transmits in NR Cell 1 a Paging message  for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat steps 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

The SS verifies the DL BWP switch time in PCell by counting the slots from the time when the RRC Reconfiguration message including updated BWP configuration is sent till a valid ACK/NACK is received.

6.5.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.5.6.2.1.4.3-1: Common Exception messages for NR SA FR1 RRC-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.5.6.2.1.4.3-2: BWP

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP ::= SEQUENCE { |  |  |  |
| locationAndBandwidth | Set the value according to the configuration of active BWP-1 |  |  |
| subcarrierSpacing | SubcarrierSpacing |  |  |
| cyclicPrefix | Not present |  |  |
| } |  |  |  |

6.5.6.2.1.5	Test requirements

Tables 6.5.6.2.1.4.1-3 and 6.5.6.2.1.5-1 define the primary level settings including test tolerances.

Table 6.5.6.2.1.5-1: NR Cell specific test parameters for DL BWP switch

| Parameter |  |  |  |  | Unit | Cell 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency Range |  |  |  |  |  | FR1 |
| Duplex mode |  |  | Config 1 |  |  | FDD |
|  |  |  | Config 2,3 |  |  | TDD |
| TDD configuration |  |  | Config 1 |  |  | Not Applicable |
|  |  |  | Config 2 |  |  | TDDConf.1.1 |
|  |  |  | Config 3 |  |  | TDDConf.2.1 |
| BWchannel |  |  | Config 1 |  |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 2 |  |  | 10 MHz: NRB,c = 52 |
|  |  |  | Config 3 |  |  | 40 MHz: NRB,c = 106 |
| Active BWP ID |  |  |  |  |  | 1 |
| Initial DL BWP Configuration |  |  | Config 1,2,3 |  |  | DLBWP.0.2 |
| Initial UL BWP Configuration |  |  | Config 1,2,3 |  |  | ULBWP.0.2 |
| Initial Condition | Active DL BWP-1 Configuration |  | Config 1,2,3 |  |  | DLBWP.1.3 |
|  | Active UL BWP-1 Configuration |  | Config 1,2,3 |  |  | ULBWP.1.3 |
| Final Condition | Active DL BWP-1 Configuration |  | Config 1,2,3 |  |  | DLBWP.1.1 |
|  | Active UL BWP-1 Configuration |  | Config 1,2,3 |  |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  | SR.1.1 FDD |
|  |  |  | Config 2 |  |  | SR.1.1 TDD |
|  |  |  | Config 3 |  |  | SR.2.1 TDD |
| RMSI CORESET parameters |  |  | Config 1 |  |  | CR.1.1 FDD |
|  |  |  | Config 2 |  |  | CR.1.1 TDD |
|  |  |  | Config 3 |  |  | CR.2.1 TDD |
| Dedicated CORESET parameters |  |  | Config 1 |  |  | CCR.1.1 FDD |
|  |  |  | Config 2 |  |  | CCR.1.1 TDD |
|  |  |  | Config 3 |  |  | CCR.2.3 TDD |
| OCNG Patterns |  |  |  |  |  | OP.1 |
| SSB Configuration |  |  | Config 1,2 |  |  | SSB.1 FR1 |
|  |  |  | Config 3 |  |  | SSB.2 FR1 |
| SMTC Configuration |  |  |  |  |  | SMTC.1 |
| TRS Configuration |  |  |  | Config 1 |  | TRS.1.1 FDD |
|  |  |  |  | Config 2 |  | TRS.1.1 TDD |
|  |  |  |  | Config 3 |  | TRS.1.2 TDD |
| Antenna Configuration |  |  |  |  |  | 1x2 Low |
| Propagation Condition |  |  |  |  |  | AWGN |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS(Note 1) |  |  |  |  |  |  |
| NocNote 2 |  | Config 1,2 |  |  | dBm/SCS | -104 |
|  |  | Config 3 |  |  |  | -101 |
| SS-RSRP Note 3 |  | Config 1,2 |  |  | dBm/SCS | -87 |
|  |  | Config 3 |  |  |  | -84 |
| Ês/Iot |  |  |  |  | dB | 17 |
| Ês/Noc |  |  |  |  | dB | 17 |
| IoNote3 |  |  | Config 1,2 |  | dBm/ 9.36MHz | -58.96 |
|  |  |  | Config 3 |  | dBm/ 38.16MHz | -52.86 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3 defined in clause 12 of TS 38.213 [8]. |  |  |  |  |  |  |

During T1, the UE shall be ready for the reception of uplink grant for Cell from the first DL slot that occurs right after the beginning of slot i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length and starts to report valid ACK/NACK for PCell from the first UL slot that occurs after the beginning of DL slot  i+TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length+k1.

Where, k1 is the timing between DL data receiving and acknowledgement as specified in [12].

All of the above test requirements shall be fulfilled in order for the observed Cell active BWP switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90%.

## 6.6	Measurement procedures

### 6.6.1	Intra-frequency measurements

#### 6.6.1.0	Minimum conformance requirements

##### 6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap

[TS 38.133, clause 9.2.5.1 and 9.2.5.2]

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCellis always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.1-1.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.1.0.1-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps.

When intra-frequency SMTC is fully non overlapping with measurement gaps or intrafrequency SMTC is fully overlapping with MGs, Kp=1.

When intrafrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP

Table 6.6.1.0.1-1: Time period for PSS/SSS detection (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil([5] x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 6.6.1.0.1-2: Measurement period for intra-frequency measurements without gaps (Frequency Range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max( 200ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 200ms, ceil(1.5x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle > 320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

[TS 38.133, clause 9.2.4.3]

Reported RSRP, RSRQ, and RS-SINR measurements contained in periodically triggered measurement reports shall meet the requirements in TS 38.133 [6] clause 10.1.2.1, 10.1.7.1 and 10.1.12.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra without index defined in TS 38.133 [6] clause 9.2.5.1. When L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period than Tidentify intra without index defined in TS 38.133 [6] clause 9.2.5.1 becomes undetectable for a period and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasurement, Intra provided the timing to that cell has not changed more than  3200 Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

[TS 38.133, clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

References: The conformance requirements covered in the current TC are specified in: TS 38.133 [6], clauses 9.2.2, 9.2.4.3, 9.2.5.1 and 9.2.5.2.

##### 6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra  ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.2-1.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.1.0.2-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in TS 38.133 [6] section 9.1.5.2.2 for measurement within outside measurement gaps.

Table 6.6.1.0.2-1: Time period for PSS/SSS detection (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle > 320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

Table 6.6.1.0.2-2: Measurement period for intra-frequency measurements with gaps (Frequency Range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max( 200ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 200ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] Sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_without_index defined in TS 38.133 [6] section 9.2.6.2. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.6.2 and 9.2.6.3.

##### 6.6.1.0.3	Minimum conformance requirements for event-triggered measurement without gap with SSB index reading

[TS 38.133 [6], clause 9.2.5.1, 9.2.5.2]

UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index.

Tidentify_intra_with_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.3-1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 6.6.1.0.3-2.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.1.0.3-3.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in TS 38.133 [6] section 9.1.5.1 for measurement conducted outside measurement gaps

When intrafrequency SMTC is fully non overlapping with measurement gaps or intrafrequency SMTC is fully overlapping with MGs, Kp=1

When intrafrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP

Table 6.6.1.0.3-1: Time period for PSS/SSS detection (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle ≤ 320ms | max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle > 320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 6.6.1.0.3-2: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period, DRX cycle)] x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 6.6.1.0.3-3: Measurement period for intra-frequency measurements with gaps (Frequency Range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max( 200ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 200ms, ceil(1.5x 5 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] Sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_with_index defined in TS 38.133 [6] section 9.2.5.1. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.5.1 and 9.2.5.2.

##### 6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index.

Tidentify_intra_with_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.4-1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 6.6.1.0.4-2.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.1.0.4-3.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap, i in TS 38.133 section 9.1.5.2.2 for measurement conducted within measurement gaps.

When intrafrequency SMTC is fully non overlapping with measurement gaps or intrafrequency SMTC is fully overlapping with MGs, Kp=1

When intrafrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP.

Table 6.6.1.0.4-1: Time period for PSS/SSS detection (Frequency range FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | [5] x max(MGRP, DRX cycle) x CSSFintra |

Table 6.6.1.0.4-2: Time period for time index detection (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 6.6.1.0.4-3: Measurement period for intra-frequency measurements with gaps (Frequency Range FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max( 200ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 200ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_with_index defined in TS 38.133 [6] section 9.2.6.2. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.6.2 and 9.2.6.3.

##### 6.6.1.0.5	Minimum conformance requirements for event-triggered measurement without gap for UE configured with highSpeedMeasFlag-r16

[TS 38.133, clause 9.2.5.1 and 9.2.5.2]

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCellis always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.1-1.

T SSB_measurement_period_intra: it is specified in Table 6.6.1.0.5-1 when highSpeedMeasFlag-r16 is configured,

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps.

When intra-frequency SMTC is fully non overlapping with measurement gaps or intrafrequency SMTC is fully overlapping with MGs, Kp=1.

When intrafrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP

Table 6.6.1.0.5-1: T SSB_measurement_period_intra When highSpeedMeasFlag-r16 is configured (Frequency range FR1

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX Note 2 | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 160ms | max(200ms, ceil(5 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| 160ms < DRX cycle≤ 320ms | ceil(4 x M2 Note 2 x Kp) x max(SMTC period,DRX cycle) |
| DRX cycle>320ms | ceil( Y Note 3 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1 NOTE 3: Y=3 when SMTC <= 40ms, Y=5 when SMTC > 40ms |  |

[TS 38.133, clause 9.2.4.3]

Reported RSRP, RSRQ, and RS-SINR measurements contained in periodically triggered measurement reports shall meet the requirements in TS 38.133 [6] clause 10.1.2.1, 10.1.7.1 and 10.1.12.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra without index defined in TS 38.133 [6] clause 9.2.5.1. When L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period than Tidentify intra without index defined in TS 38.133 [6] clause 9.2.5.1 becomes undetectable for a period and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than Tmeasurement, Intra provided the timing to that cell has not changed more than  3200 Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

[TS 38.133, clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

References: The conformance requirements covered in the current TC are specified in: TS 38.133 [6], clauses 9.2.2, 9.2.4.3, 9.2.5.1 and 9.2.5.2.

#### 6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX

6.6.1.1.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under non-DRX.

6.6.1.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.1.

6.6.1.1.4	Test description

6.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.1.4.1-1.

Table 6.6.1.1.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.1-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.1-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.1-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.1.1.4.1-2.

Table 6.6.1.1.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.1.4.1-3 below.

2. Message contents are defined in clause 6.6.1.1.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.1.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under non-DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.1.1.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.1.4.1-3 and Table 6.6.1.1.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.1.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.1.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current NR cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
Or
- if the device has been switched off, switcheson the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.1.4.3-1: Common Exception messages SA intra frequency event triggered reporting tests without gap under non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 6.6.1.1-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.1-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM |

6.6.1.1.5	Test requirement

Table 6.6.1.1.4.1-3 and Table 6.6.1.1.5-1 define the primary level settings including test tolerances for NR SA event triggered reporting test without gap under non-DRX.

Table 6.6.1.1.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under non-DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intr= max [ 600 ms, ceil (5 × Kp) × SMTC period ] × CSSFintra = 600 ms

T SSB_measurement_period_intra= max [ 200 ms, ceil( 5 × Kp) × SMTC period ] × CSSFintra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 802 ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX

6.6.1.2.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under DRX.

6.6.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.2.

6.6.1.2.4	Test description

6.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.2.4.1-1.

Table 6.6.1.2.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.2-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.2-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.2-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.2.4.1-2.

Table 6.6.1.2.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.2.4.1-3 below.

2. Message contents are defined in clause 6.6.1.2.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.2.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX | ms | 1, 2, 3 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2 | 3 us |  | Synchronous cells |
|  |  | 3 | 3 us |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

6.6.1.2.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.2.4.1-3 and Table 6.6.1.2.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.2.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.2.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 6402 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current NR cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.1.2.4.1-1 as appropriate.

6.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.2.4.3-1: Common Exception messages for SA intra frequency event triggered reporting tests without gap under DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 6.6.1.2-1 Table H.3.1-3 with Condition INTRA-FREQ MO SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.2-2 Table H.3.1-3 with Condition INTRA-FREQ MO SSB.2 FR1, SMTC .1 and synchronous cells for configuration 6.6.1.2-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.1 for test 1 Table H.3.7-1 with Condition DRX.7 for test 2 |

6.6.1.2.5	Test requirement

Table 6.6.1.2.4.1-3 and Table 6.6.1.2.5-1 define the primary level settings including test tolerances for NR event triggered reporting in synchronous cells when DRX is used test.

Table 6.6.1.2.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to NR Cell 2 on PUSCH.

For both tests:

The overall delays measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delay measured when DRX cycle length is 40 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = max[600ms, ceil(1.5 × 5 x Kp) × max(SMTC period, DRX cycle)] × CSSFintra= 600ms

T SSB_measurement_period_intra = max[200ms, ceil(1.5 × 5 × Kp) x max(SMTC period, DRX cycle)] × CSSFintra= 320ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 40 ms shall be less than a total of 922 ms.

The overall delay measured when DRX cycle length is 640 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

T SSB_measurement_period_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of 6402 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX

6.6.1.3.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under non-DRX within intra-frequency cell search with gaps requirements.

6.6.1.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards. This test applies to UE that support CSI-RS based RLM.

6.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.3.

6.6.1.3.4	Test description

6.6.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.3.4.1-1.

Table 6.6.1.3.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.3-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.3-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.3-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.3.4.1-2.

Table 6.6.1.3.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.3.4.1-3.

2. Message contents are defined in clause 6.6.1.3.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.3.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.1.3.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.3.4.1-3 and Table 6.6.1.3.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.3.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.3.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.3.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 and Asynchronous cells for Configuration 6.6.1.3-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and Synchronous cells for Configuration 6.6.1.3-2 Table H.3.1-3 with ConditionINTRA-FREQ MO, SSB.2 FR1, SMTC.1 and Synchronous cells for Configuration 6.6.1.3-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.3.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.3.5	Test requirement

Table 6.6.1.3.4.1-3 and Table 6.6.1.3.5-1 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in non-DRX test.

Table 6.6.1.3.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.2 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 802 ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX

6.6.1.4.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under DRX within intra-frequency cell search with gaps requirements.

6.6.1.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle.

6.6.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.4.

6.6.1.4.4	Test description

6.6.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.4.4.1-1.

Table 6.6.1.4.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.4-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.4-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.4-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.4.4.1-2.

Table 6.6.1.4.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.4.4.1-3.

2. Message contents are defined in clause 6.6.1.4.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.4.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX | ms | 1, 2, 3 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

6.6.1.4.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.4.4.1-3 and Table 6.6.1.4.4.2-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In Test 1 when DRX cycle = 40 ms is used, UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Timer Alignment Timer to keep the UE uplink time alignment. Furthermore, the UE is allocated with PUSCH resource at every DRX cycle. In Test 2 when DRX = 640 ms is used, the uplink time alignment is not maintained and the UE needs to use RACH to obtain uplink allocation for measurement reporting.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.4.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.4.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 6402 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.1.4.4.1-3 as appropriate.

6.6.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.4.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 6.6.1.4-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and Asynchronous cells for configuration 2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1 and Asynchronous cells for configuration 6.6.1.4-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.7-1 with Condition DRX.1 and Gap for test 1 Table H.3.7-1 with Condition DRX.7 and Gap for test 2 Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.4.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.4.5	Test requirement

Table 6.6.1.4.4.1-3 and Table 6.6.1.4.5-1 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in DRX test.

Table 6.6.1.4.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.2 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report on PUSCH.

For both tests:

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 1: The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2: In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

TPSS/SSS_sync_intra = 600 ms for Test 1, and TPSS/SSS_sync_intra =3200 ms for Test 2

TSSB_measurement_period_intra = 320 ms for Test 1, and TSSB_measurement_period_intra = 3200 ms for Test 2

TTI insertion uncertainty = 2 ms

For Test 1, the overall delays measured shall be less than a total of 922 ms (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For Test 2, the overall delays measured shall be less than a total of 6402 ms (note: this gives a total of 6400 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

6.6.1.5.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event within intra-frequency cell search without gaps requirements.

6.6.1.5.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.6.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.5.

6.6.1.5.4	Test description

6.6.1.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.5.4.1-1.

Table 6.6.1.5.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Configuration | Description |
| --- | --- |
| 6.6.1.5-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.5.4.1-2.

Table 6.6.1.5.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.5.4.1-3.

2. Message contents are defined in clause 6.6.1.5.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.5.4.1-3: General test parameters for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

6.6.1.5.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.5.4.1-3 and Table 6.6.1.5.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.5.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.5.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 clause 4.6 with the following exceptions:

Table 6.6.1.5.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 and Asynchronous cells for Configuration 6.6.1.5-1 Table H.3.1-4 with SSB Index and A3-offset = -4.5 dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ and SSB Index Table H.3.1-8 with Condition SSB RLM |

6.6.1.5.5	Test requirement

Table 6.6.1.5.4.1-3 and Table 6.6.1.5.5-1 define the primary level settings including test tolerances for SA event triggered reporting without gap under non-DRX with SSB index reading test.

Table 6.6.1.5.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_with_index

Tidentify_intra_with_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_time_index_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_time_index_intra = 120 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 922 ms in this test case (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

6.6.1.6.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event within intra-frequency cell search with gaps requirements.

6.6.1.6.2	Test applicability

This test applies to all types of NR UE release 15 onwards. This test applies to UE that support CSI-RS based RLM.

6.6.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.6.

6.6.1.6.4	Test description

6.6.1.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.6.4.1-1.

Table 6.6.1.6.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Configuration | Description |
| --- | --- |
| 6.6.1.6-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.3.4.1-2.

Table 6.6.1.6.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.6.4.1-3.

2. Message contents are defined in clause 6.6.1.6.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.6.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1 | 40 |  |
| Measurement gap length | ms | 1 | 6 |  |
| Measurement gap offset | ms | 1 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

6.6.1.6.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.6.4.1-3 and Table 6.6.1.6.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.6.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.3.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 clause 4.6 with the following exceptions:

Table 6.6.1.6.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 and Asynchronous cells for Configuration 6.6.1.6-1 Table H.3.1-4 with SSB index and A3-offset = -4.5dB. Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ and SSB index Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.6.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.6.5	Test requirement

Table 6.6.1.6.4.1-3 and Table 6.6.1.6.5-1 define the primary level settings including test tolerances for SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading test.

Table 6.6.1.6.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | CSI-RS |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Note 2 | dBm/15 KHz | 1 | -98 |  |  |  |
|  | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS KHz | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_with_index

Tidentify_intra_with_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_time_index_intra = 120 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 922 ms in this test case (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

6.6.1.7.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under DRX for UE configured with highSpeedMeasFlag-r16.

6.6.1.7.2	Test applicability

This test applies to all types of NR UE release 16 onwards that supports measurement enhancements in high speed scenario.

6.6.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.5.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.7.

6.6.1.7.4	Test description

6.6.1.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.7.4.1-1.

Table 6.6.1.7.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 6.6.1.7-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.1.7-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.1.7-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.7.4.1-2.

Table 6.6.1.7.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.7.4.1-3 below.

2. Message contents are defined in clause 6.6.1.7.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.7.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasFlag-r16 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.2 | 640ms DRX cycle |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 6 |  |

6.6.1.7.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.7.4.1-3 and Table 6.6.1.7.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.7.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.7.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5122 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.7.4.3-1: Common Exception messages for SA intra frequency event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasFlag-r16

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.2 |
| Specific message contents exceptions for Test Configuration 6.6.1.7-1 | Table H.3.1-3 with Condition INTRA-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.7-2 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.7-3 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.2 FR1 |

6.6.1.7.5	Test requirement

Table 6.6.1.7.4.1-3 and Table 6.6.1.7.5-1 define the primary level settings including test tolerances for NR event triggered reporting in synchronous cells when DRX is used

Table 6.6.1.7.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |  | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |
| Note 2 | dBm/SCS | 1 | -98+TT |  |  |  |  |
|  |  | 2 | -98+TT |  |  |  |  |
|  |  | 3 | -95+TT |  |  |  |  |
| Note 2 | dBm/15 kHz | 1 | -98+TT |  |  |  |  |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4+TT | -1.46+ TT |  | -Infinity | -1.46+ TT |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
|  | dB | 1 | 4+TT | 4+TT |  | -Infinity | 4+TT |
|  |  | 2 |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94+TT | -94+TT |  | -Infinity | -94+TT |
|  |  | 2 | -94+TT | -94+TT |  | -Infinity | -94+TT |
|  |  | 3 | -91+TT | -91+TT |  | -Infinity | -91+TT |
| Io | dBm/9.36 MHz | 1 | -64.60+ TT | -62.25+ TT |  | -64.60+ TT | -62.25+ TT |
|  | dBm/9.36 MHz | 2 | -64.60+ TT | -62.25+ TT |  | -64.60+ TT | -62.25+ TT |
|  | dBm/38.16 MHz | 3 | -58.50+ TT | -56.16+ TT |  | -58.50+ TT | -56.16+ TT |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN 1944Hz Note 4 |  |  |
|  |  | 3 | AWGN |  | AWGN 3334Hz Note 5 |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.  When DRX cycle length = 640 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to NR Cell 2 on PUSCH.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delay measured when DRX cycle length is 640 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

T SSB_measurement_period_intra = ceil(Y × Kp) × DRX cycle × CSSFintra = 1920ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of 5122 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 6.6.2	Inter-frequency measurements

#### 6.6.2.0	Minimum conformance requirements for Inter-frequency measurements

Same as clause 4.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3.

#### 6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX

6.6.2.1.1	Test purpose

To verify that the UE makes correct reporting of an event in non-DRX within inter-frequency NR cell search requirements without SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

6.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.1.

6.6.2.1.4	Test description

6.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.1.4.1-2. Test environment parameters are given in Table 6.6.2.1.4.1-3.

Table 6.6.2.1.4.1-1: SA FR1-FR1 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.1.4.1-2: SA FR1-FR1 general test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in TS 38.133 clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 | 1 |  |

Table 6.6.2.1.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.1.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.1.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 6.6.2.1.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 6.6.2.1.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.1.4.1-2. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.1.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 920 ms  for Test 1 and 760 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.  Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR:
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.1.4.1-2 as appropriate.

6.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.1.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 Table H.3.1-6 with Conditions gapFR1, Pattern #4 and gap offset = 9 for Test 2 Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.1.5	Test requirement

Table 6.6.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%. with a confidence level of 95%

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 760 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1 and 2 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX

6.6.2.2.1	Test purpose

To verify that the UE makes correct reporting of an event in DRX within inter-frequency NR cell search requirements without SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle. Test 1 and Test 2 are applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

6.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.2.

6.6.2.2.4	Test description

6.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.2.4.1-2. Test environment parameters are given in Table 6.6.2.2.4.1-3.

Table 6.6.2.2.4.1-1: SA FR1-FR1 event triggered reporting tests in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in TS 38.133 clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 11 | 1.1 | 11 |  |

Table 6.6.2.2.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.2.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.2.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 6.6.2.2.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 6.6.2.2.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.2.4.1-2. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.2.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test 1, 10240 ms for Test 2, 1080 ms for Test 3 and 10240 ms for Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.  Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.2.4.1-2 as appropriate.

6.6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.2.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 and Test 2 Table H.3.1-6 with Conditions gapFR1, Pattern #4 and gap offset = 9 for Test 3 and Test 4 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 and Test 3 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-1 | Table H.3.1-3 with Conditions INTER-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.2.5	Test requirement

Table 6.6.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.2.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1, 2, 3 and 4 UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.3	Void

#### 6.6.2.4	Void

#### 6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection

6.6.2.5.1	Test purpose

To verify that the UE makes correct reporting of an event in non-DRX within inter-frequency NR cell search requirements with SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.5.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Test 1 is applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

6.6.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.5.

6.6.2.5.4	Test description

6.6.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.5.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.5.4.1-2. Test environment parameters are given in Table 6.6.2.5.4.1-3.

Table 6.6.2.5.4.1-1: SA FR1-FR1 event triggered reporting tests in non-DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.5-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.5-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.5-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.5.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  | NR cell 2 is on NR RF channel number 2 |
| Gap Pattern Id |  | Config 1,2,3 | 0 | 4 | As specified in TS 38.133 clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 1 |  |

Table 6.6.2.5.4.1-3: Environment test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.5.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.5.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 6.6.2.5.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 6.6.2.5.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.5.4.1-2. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.5.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1040 ms for Test 1 and 880 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.  Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.5.4.1-2 as appropriate.

6.6.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.5.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and Condition SSB Index Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 Table H.3.1-6 with Conditions gapFR1, Pattern #4 and gap offset = 9 for Test 2 Table H.3.1-7 with Conditions INTER-FREQ and SSB Index |
| Specific message contents exceptions for Test Configuration 6.6.2.5-1 | Table H.3.1-3 with Conditions INTER-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.5-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.5-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.5.5	Test requirement

Table 6.6.2.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 |  | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.4 | -58.49 |  | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1040 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 880 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1 and 2 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection

6.6.2.6.1	Test purpose

To verify that the UE makes correct reporting of an event in DRX within inter-frequency NR cell search requirements with SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.6.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle. Test 1 and Test 2 are applicable to UEs not supporting per-FR gap (IndependentGapConfig, as defined in TS 38.306 [11]) and Test 3 and Test 4 are applicable only to UEs supporting per-FR gap and Gap Pattern Id 4.

6.6.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.6.

6.6.2.6.4	Test description

6.6.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.6.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.6.4.1-2. Test environment parameters are given in Table 6.6.2.6.4.1-3.

Table 6.6.2.6.4.1-1: SA FR1-FR1 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.6-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.64-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.6-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.6.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR cell2 |  |  |  | NR cell 2 is on NR RF channel number 2 |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | 4 |  | As specified in TS 38.133 clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 |  | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | DRX.1 | DRX.7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 1.3 | 13.5 | 1.3 | 13.5 |  |

Table 6.6.2.6.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.6.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Table Annex C.1.2.

6.6.2.6.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 6.6.2.6.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #4 as defined in Table 6.6.2.6.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.6.4.1-2. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.6.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1280 ms for Test 1, 13440 ms for Test 2, 1280 ms for Test 3 and 13440 ms for Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.6.4.1-2 as appropriate.

6.6.2.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.6.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and Condition SSB Index Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 for Test 1 and Test 2 Table H.3.1-6 with Conditions gapFR1 and Pattern #4 and gap offset = 9 for Test 3 and Test 4 Table H.3.1-7 with Conditions INTER-FREQ and SSB Index Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 and Test 3 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-1 | Table H.3.1-3 with Conditions INTER-FREQ MO and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.6.5	Test requirement

Table 6.6.2.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.6.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2 with per-UE gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 13440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 3 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 4 with per-FR gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 13440 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1, 2, 3 and 4 UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.7	Void

#### 6.6.2.8	Void

### 6.6.3	Inter-RAT Measurements

#### 6.6.3.0	Minimum conformance requirements

##### 6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD

The requirements are applicable for NR−E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN FDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RSRQ related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RS-SINR related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [23].

6.6.3.0.1.1	Requirements when no DRX is used

When the UE requires measurement gaps to idenitify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, the UE shall be able to identify a new detectable FDD cell within TIdentify, E-UTRAN FDD according to the following expression:

TIdentify,E-UTRAN FDD=TBasicIdentify* 480TInter1*CSSFinterRAT     ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in TS 38.133 [6] section 9.4.1,

CSSFinterRAT = CSSFwithin_gap_i_ is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in TS 38.133 [6] section 9.1.5.2.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN FDD defined in Table 6.6.3.0.1.1-1.

Table 6.6.3.0.1.1-1: Measurement period and measurement bandwidth

| Configuration | Physical Layer Measurement period: TMeasure, E-UTRAN FDD [ms] | Measurement bandwidth [RB] |
| --- | --- | --- |
| 0 | 480 x CSSFinterRAT | 6 |
| 1 (note 1) | 240 x CSSFinterRAT | 50 |
| NOTE 1: This configuration is optional. |  |  |

The UE shall be capable of identifying and performing NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements of at least 4 E-UTRAN FDD cells per E-UTRA FDD carrier frequency layer for up to 7 E-UTRA FDD carrier frequency layers.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.5.

6.6.3.0.1.2	Requirements when DRX is used

When DRX is in use and measurement gaps are configured, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in Table 6.6.3.0.1.2-1. When RRM enhancement for high speed is configured the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in Table 6.6.3.0.1.2-2.

Table 6.6.3.0.1.2-1: Requirement to identify a newly detectable E-UTRAN FDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in Section 6.6.3.0.1.1 apply | Non-DRX requirements in Section 6.6.3.0.1.1 apply |
| 0.256 | 5.12*K (20*CSSFinterRAT) | 7.68*K (30*CSSFinterRAT) |
| 0.32 | 6.4*K (20*CSSFinterRAT) | 7.68*K (24*CSSFinterRAT) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.1.1. |  |  |

Table 6.6.3.0.1.2-2: Requirement to identify a newly detectable E-UTRAN FDD cell for UE configured with RRM enhancement for high speed

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.2.2 apply | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.16<DRx cycle<=0.32 | Note 1(15*CSSFinterRAT) |  |
| 0.32<DRx cycle <= 0.64 | Note 1(10*CSSFinterRAT) |  |
| DRx cycle = 1.024 | Note 1(10*CSSFinterRAT) | Note 1(10*CSSFinterRAT) |
| DRx cycle = 1.28 | Note 1(8*CSSFinterRAT) | Note 1(8*CSSFinterRAT) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN FDD cells per E-UTRA FDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA FDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN FDD specified in Table 6.6.3.0.1.2-3.

Table 6.6.3.0.1.2-3: Requirement to measure E-UTRAN FDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN FDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX requirements in Section 6.6.3.0.1.1 apply |
| 0< DRX-cycle ≤10.24 | Note1 (5* CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.1.1. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.5.

6.6.3.0.1.3	Measurement reporting requirements for Event-Triggered Reporting

The reported NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements contained in event-triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.2.2, 10.2.3, and 10.2.5, respectively.

The UE shall not send any event-triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T Identify, E-UTRAN FDD defined in clauses 6.6.3.0.1.1 and 6.6.3.0.1.2 without DRX and with DRX, respectively. When L3 filtering is used, an additional delay can be expected.

If a cell which has been detectable at least for the time period TIdentify, E-UTRAN FDD becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event as per TS 38.331 [13], the event triggered measurement reporting delay shall be less than TMeasure, E-UTRAN FDD provided the timing to that cell has not changed more than  50 Ts while measurement gap has not been available and the L3 filter has not been used.

The normative reference for this requirement is TS 38.133 [6] clause 9.4.2.

##### 6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD

The requirements are applicable for NR−E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN TDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RSRQ related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RS-SINR related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [23].

6.6.3.0.2.1	Requirements when no DRX is used

When the UE requires measurement gaps to idenitify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, the UE shall be able to identify a new detectable TDD cell within TIdentify, E-UTRAN TDD according to the following expression:

-	When configuration 0 or configuration 1 in Table 6.6.3.0.2-1 is applied,

TIdentify,E-UTRAN TDD=TBasicIdentify* 480TInter1*CSSFinterRAT     ms ,

-	When configuration 2 or configuration 3 in Table 6.6.3.0.2-1 is applied,

TIdentify,E-UTRAN TDD=(TBasicIdentify* 480TInter1+240)*CSSFinterRAT    ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in TS 38.133 [6] section 9.4.1,

CSSFinterRAT = CSSFwithin_gap_i_ is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in TS 38.133 [6] section 9.1.5.2.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN TDD defined in Table 6.6.3.0.2.1-1.

Table 6.6.3.0.2.1-1: TMeasure, E-UTRAN TDD for different configurations

| Configuration | Measurement bandwidth [RB] | Number of UL/DL sub-frames per half frame (5 ms) |  | DwPTS |  | TMeasure, E-UTRAN TDD [ms] |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | Normal CP | Extended CP |  |
| 0 | 6 | 2 | 2 |  |  | 480 x CSSFinterRAT |
| 1 (note 1) | 50 | 2 | 2 |  |  | 240 x CSSFinterRAT |
| 2 | 6 | 1 | 3 |  |  | 720 x CSSFinterRAT |
| 3 (Note 1) | 50 | 1 | 3 |  |  | 480 x CSSFinterRAT |
| NOTE 1: This configuration is optional. NOTE 2: Void |  |  |  |  |  |  |

The UE shall be capable of identifying and performing NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements of at least 4 E-UTRAN TDD cells per E-UTRA TDD carrier frequency layer for up to 7 E-UTRA TDD carrier frequency layers.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN TDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.2. The NR – E-UTRAN TDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.3. The NR – E-UTRAN TDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.5.

6.6.3.0.2.1	Requirements when DRX is used

When DRX is in use and measurement gaps are configured, the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in Table 6.6.3.0.2.1-1. When RRM enhancement for high speed is configured the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in Table 6.6.3.0.2.1-2.

Table 6.6.3.0.2.1-1: Requirement to identify a newly detectable E-UTRAN TDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in Section 6.6.3.0.2.1 apply | Non-DRX requirements in Section 6.6.3.0.2.1 apply |
| 0.256 | 5.12*K (20*CSSFinterRAT) | 7.68*K (30*CSSFinterRAT) |
| 0.32 | 6.4*K (20*CSSFinterRAT) | 7.68*K (24*CSSFinterRAT) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.2.1. |  |  |

Table 6.6.3.0.2.1-2: Requirement to identify a newly detectable E-UTRAN TDD cell for UE configured with RRM enhancement for high speed

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.3.2 apply | Non-DRX requirements in clause 9.4.3.2 apply |
| 0.16<DRx cycle<=0.32 | Note 1(15*CSSFinterRAT) |  |
| 0.32<DRx cycle <= 0.64 | Note 1(10*CSSFinterRAT) |  |
| DRx cycle = 1.024 | Note 1(10*CSSFinterRAT) | Note 1(10*CSSFinterRAT) |
| DRx cycle = 1.28 | Note 1(8*CSSFinterRAT) | Note 1(8*CSSFinterRAT) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.3.2. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN TDD cells per E-UTRA TDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA TDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN TDD specified in Table 6.6.3.0.2.1-3.

Table 6.6.3.0.2.1-3: Requirement to measure E-UTRAN TDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN TDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX Requirements in Section 6.6.3.0.2.1 apply |
| 0.128 | For configuration 2, non-DRX requirements in section 6.6.3.0.2.1 apply, Otherwise: Note1 (5*CSSFinterRAT) |
| 0.128<DRX-cycle≤10.24 | Note1 (5*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.2.1. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN TDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.2. The NR – E-UTRAN TDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.3. The NR – E-UTRAN TDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.5.

6.6.3.0.2.3	Measurement reporting requirements for Event-Triggered Reporting

The reported NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements contained in event-triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.2.2, 10.2.3, and 10.2.5, respectively.

The UE shall not send any event-triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T Identify, E-UTRAN TDD defined in clauses 6.6.3.0.2.1 and 6.6.3.0.2.2 without DRX and with DRX, respectively. When L3 filtering is used, an additional delay can be expected.

If a cell which has been detectable at least for the time period TIdentify, E-UTRAN TDD becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event as per TS 38.331 [13], the event triggered measurement reporting delay shall be less than TMeasure, E-UTRAN TDD provided the timing to that cell has not changed more than  50 Ts while measurement gap has not been available and the L3 filter has not been used.

The normative reference for this requirement is TS 38.133 [6] clause 9.4.3.

#### 6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX

6.6.3.1.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 under the cell search and measurement requirements.

6.6.3.1.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 from Release 15 onwards.

6.6.3.1.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.1.

6.6.3.1.3.1	NR – E-UTRAN FDD requirement

The minimum conformance requirements are specified in clause 6.6.3.0.1.

6.6.3.1.3.2	NR – E-UTRAN TDD requirement

The minimum conformance requirements are specified in clause 6.6.3.0.2.

6.6.3.1.4	Test description

6.6.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.1.4.1-1.

Table 6.6.3.1.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.3.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.1-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.1-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.1-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.1.4.1-2 and Table 6.6.3.1.4.1-3.

Table 6.6.3.1.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.1.5-1 and Table 6.6.3.1.5-2.. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.1.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.3.1.5-1 and 6.6.3.1.5-2. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause TS 38.133 [6] Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in Table 6.6.3.1.5-1 |  |  |  |

1.	Message contents are defined in clause 6.6.3.1.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.6.3.1.4.2	Test procedure

The test consists of two successive time periods, with time durations of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. Gap pattern configuration is configured before T2 begins to enable inter-frequency monitoring.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.3.1.5-1 and 6.6.3.1.5-2. Propagation conditions are set according to Annex C clause C.2.2.T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.3.1.5-1 and 6.6.3.1.5-2.

6.	UE shall transmit a MeasurementReport message triggered by Event B2. If the measurement reporting delay from the beginning of time period T2 is less than 3842ms the number of successful tests is increased by one. If the UE fails to report the event within the measurement reporting delay requirement then the number of failure tests is increased by one.

7.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED according to TS 38.508-1 [14] clause 4.5.4 (if the paging fails, switches off and on the UE and ensures the UE is in the state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5,
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.3.1.4.3-1: Common Exception messages NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3 with Condition SSB.1 FR1 and Asynchronous cells cells for configuration 6.6.3.1-1, 6.6.3.1-2, 6.6.3.1-4, 6.6.3.1-5 Table H.3.1-3 with Condition SSB.1 FR2, and Asynchronous cells cells for configuration 6.6.2.1-3, 6.6.3.1-6 Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT |

6.6.3.1.5	Test requirement

Table 6.6.3.1.5-1 and Table 6.6.3.1.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.3.1.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG pattern note1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| Noc note2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Noc note2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 19.65 | -3.65 |
| Ês/Iot note3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 19.65 | -3.65 |
| SS-RSRP note3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.35 | -109.65 |
|  |  |  |  | 3, 6 | -83.35 | -106.65 |
| SSB_RP note3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.35 | -109.65 |
|  |  |  |  | 3, 6 | -83.35 | -106.65 |
| Io note3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.35 | -76.49 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.25 | -70.39 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.1.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configuration note1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configuration note1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel note2 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel note2 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Patterns note2 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RA note3 |  |  |  |  |
| OCNG_RB note3 |  |  |  |  |
| Noc note4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 20.65 |
| Ês/Iot note5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 20.65 |
| RSRP note5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.35 |
| SCH_RP note5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.35 |
| Io note5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -57.53+10log (NRB,c /50) |
| Propagation Condition not 6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3842ms from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The overall delays measured test requirement is expressed as:

Tidentify,E-UTRAN FDD = TBasicIdentify * 480 / TInter1 * CSSFinterRAT ms

Which:

TBasicIdentify = 480,

TInter1 = 60,

CSSFinterRAT = 1

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 3842 ms in this test case (note: this gives a total of 3840 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

The rate of correct events observed during repeated tests shall be at least 90% with confidence level of 95%.

#### 6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX

6.6.3.2.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used under the cell search and measurement requirements.

6.6.3.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle.

6.6.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 6.6.3.0.1 and 6.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.2.

6.6.3.2.4	Test description

6.6.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.2.4.1-1.

Table 6.6.3.2.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Test Case ID | Description |
| --- | --- |
| 6.6.3.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.2-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.2-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.2-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.2.4.1-2 and Table 6.6.3.2.4.1-3.

Table 6.6.3.2.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.2.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 |  |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.3.2.5-1 and 6.6.3.2.5-2. |  |  |  |
| Active cell |  | Cell 1 |  |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  |  | As specified in Clause TS 38.133 [6] Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | note 1 |  |  | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 |  |  | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |  |  |
| TimeToTrigger | s | 0 |  |  |  |
| Filter coefficient |  | 0 |  |  | L3 filtering is not used |
| DRX |  | DRX.1 |  | DRX.7 | DRX cycle configurations DRX.1 and DRX.7 are defined in Table A.3.3.1-1 and Table A.3.3.2-1 respectively. |
| T1 | s | 5 |  |  |  |
| T2 | s | 5 | 15 |  |  |
| NOTE 1: Values are defined in Table 6.6.3.2.5-1 |  |  |  |  |  |

1.	Message contents are defined in clause 6.6.3.2.4.3.

2.	There are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

6.6.3.2.4.2	Test procedure

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore the UE shall be allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.3.2.5-1 and 6.6.3.2.5-2. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 6.6.3.2.5-1 and 6.6.3.2.5-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 3.48 s for Test 1 or less than 12.8 s for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off .

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.3.2.4.3-1: Common Exception messages NR SA FR1 – E-UTRAN event-triggered reporting in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3 with Condition SSB.1 FR1 and Asynchronous cells cells for configuration 6.6.3.2-1, 6.6.3.2-2, 6.6.3.2-4, 6.6.3.2-5 Table H.3.1-3 with Condition SSB.1 FR2 and Asynchronous cells cells for configuration 6.6.2.2-3, 6.6.3.2-6 Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT Table H.3.7-1 with Condition DRX.1 and Gap for Test 1 Table H.3.7-1 with Condition DRX.7 and Gap for Test 2 |

6.6.3.2.5	Test requirement

Table 6.6.3.2.5-1 and Table 6.6.3.2.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.6.3.2.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 19.65 | -3.65 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 19.65 | -3.65 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.35 | -109.65 |
|  |  |  |  | 3, 6 | -83.35 | -106.65 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.35 | -109.65 |
|  |  |  |  | 3, 6 | -83.35 | -106.65 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.35 | -76.49 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.25 | -70.39 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | TDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.2.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 20.65 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 20.65 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.35 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.35 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -57.53+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211. Note 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 12.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### 6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

-	Cell Configuration table in Annex E is undefined.

-	Antenna diagram is TBD

-	Message content is TBD

-	Test case applicability is TBD

-	Test procedure is TBD

6.6.3.3.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used under the cell search and measurement requirements for UE configured with RRM enhancement for high speed.

6.6.3.3.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 configured with RRM enhancement for high speed from Release 16 onwards.

6.6.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 6.6.3.0.1 and 6.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.3.

6.6.3.3.4	Test description

6.6.3.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.3.4.1-1.

Table 6.6.3.3.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Test Case ID | Description |
| --- | --- |
| 6.6.3.3-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.3-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.3-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.3-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.3-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.3-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.3.4.1-2 and Table 6.6.3.3.4.1-3.

Table 6.6.3.3.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.3.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.3.1-2 and A.6.6.3.3.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 | E-UTRAN RSRP threshold for SS-RSRP measurement on cell1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX.6 | DRX cycle configurations DRX.6 is defined in Table A.3.3.1-6. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in TS 38.133 Table 6.6.3.3.5-1 |  |  |  |

1.	Message contents are defined in clause 6.6.3.3.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

3. UE is configured with highSpeedMeasFlag-r16

6.6.3.3.4.2	Test procedure

TBD

6.6.3.3.4.3	Message contents

TBD

6.6.3.3.5	Test requirement

Table 6.6.3.3.5-1 and Table 6.6.3.3.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.6.3.3.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.3.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -59.16+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN1944 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [25]. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 4.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

### 6.6.4	L1-RSRP measurement for beam reporting

#### 6.6.4.0	Minimum conformance requirements

##### 6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting

Same as clause 4.6.4.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.5.3.1, 9.5.4.1 and 9.5.5.1.

##### 6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting

Same as clause 4.6.4.0.2

The normative reference for this requirement is TS 38.133 [6] clauses 9.5.3.1, 9.5.4.2 and 9.5.5.2.

#### 6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX

6.6.4.1.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

6.6.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.1.

6.6.4.1.4	Test description

6.6.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.1.4.1-2. Test environment parameters are given in Table 6.6.4.1.4.1-3.

Table 6.6.4.1.4.1-1: NR SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.1.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.1.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.1.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.1.4.1-2 and 6.6.4.1.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 4.6.4.1.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.4.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.4.1.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 6.6.4.1.5-1. T2 starts.

5.	If the UE sends L1-RSRP reports  meeting the corresponding absolute accuracy requirements in Table 6.6.4.1.5-2 for for test configurations 1 and 2 the corresponding absolute accuracy requirements in Table 6.6.4.1.5-3 for test configurations 3 and the corresponding relative accuracy requirements in Table 6.6.4.1.5-4 for all test configurations every 80 slots from no later than 720 ms for configuration 1 and 2 and no later than 680 ms for configuration 3 from the beginning of time period T2 until the end of time period T2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.1.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with conditions SSB and PERIODIC |

Table 6.6.4.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.4.1.5	Test requirement

Table 6.6.4.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.1.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.14 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 4.6.4.1.5-2 for for test configurations 1, 2, 4 and 5 and the corresponding absolute accuracy requirements in Table 4.6.4.1.5-3 for test configurations 3 and 6 and the corresponding relative accuracy requirements in Table 4.6.4.1.5-4 for all test configurations.

Table 6.6.4.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 55 |
| Highest reported value (SSB#1) | - | 75 |

Table 6.6.4.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 6.6.4.1.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in  DCCH.

#### 6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX

6.6.4.2.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.2.

6.6.4.2.4	Test description

6.6.4.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.2.4.1-2. Test environment parameters are given in Table 6.6.4.2.4.1-3.

Table 6.6.4.2.4.1-1: SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.2.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.2.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.2.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.2.4.1-2 and 6.6.4.2.5-1. UE is configured to perform RLM and BFD based on the SSBs. DRX is configured as specified in Table 6.6.4.2.4.1-2.

6.6.4.2.4.2	Test procedure

Same test procedure as in subclause 6.6.4.1.4.2 with tables 6.6.4.1.4.1-2 and 6.6.4.1.5-1 replaced by tables 6.6.4.2.4.1-2 and 6.6.4.2.5-1.

6.6.4.2.4.3	Message contents

Same message content as in subclause 6.6.4.1.4.3 with the following exception:

Table 6.6.4.2.4.3-1: Common Exception messages EN-DC SSB based L1-RSRP measurement in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.7-1 with condition DRX.3 |

6.6.4.2.5	Test requirement

Table 6.6.4.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.2.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.14 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
|  | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.2.5-2 for for test configurations 1, 2, 4 and 5 and the corresponding absolute accuracy requirements in Table 6.6.4.2.5-3 for test configurations 3 and 6 and the corresponding relative accuracy requirements in Table 6.6.4.2.5-4 for all test configurations.

Table 6.6.4.2.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 55 |
| Highest reported value (SSB#1) | - | 75 |

Table 6.6.4.2.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 6.6.4.2.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX

6.6.4.3.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

6.6.4.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.3.

6.6.4.3.4	Test description

6.6.4.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.3.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.3.4.1-2. Test environment parameters are given in Table 6.6.4.3.4.1-3.

Table 6.6.4.3.4.1-1:NR SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.3.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.3.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.3.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.3.4.1-2 and 6.6.4.3.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.3.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. UE is also configured to measure L1-RSRP based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in table 6.6.4.3.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.4.3.5-1. T1 starts.

3.	After 80ms from the start of the test the SS transmits the DCI trigger in slot 0 for configuration 1,2 and slot 8 for configuration 3. The corresponding CSI-RS set is transmitted with the offset of 4 slots after the DCI trigger.

4.	The UE shall send L1-RSRP report at slot 8 from the reception of DCI trigger. The report shall contain L1-RSRP of both CSI-RS#0 and CSI-RS#1.

5.	If after T1 expiry no report is received or received report did not contain L1-RSRP of both CSI-RS#0 and CSI-RS#1 or UE sent the L1-RSRP report at different slot than 8 from the reception of DCI trigger, the number of ‘failed’ iterations is increased by one, otherwise, the number of ‘passed’ iterations is increased by one.

6.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.3.4.3-1: Common Exception messages NR SA CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions APERIODIC and CSI-RSRP Table H.3.6-3 with conditions CSI-RS and APERIODIC |

Table 6.6.4.3.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.6.4.3.5	Test requirement

Table 6.6.4.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.3.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.15 |
|  | 3 |  | -91.65 | -88.14 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.49 |
|  | 1~3 | dB | 0 | 3.5 |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.3.5-2 for for test configurations 1, 2, 4 and 5 and the corresponding absolute accuracy requirements in Table 6.6.4.3.5-3 for test configurations 3 and 6 and the corresponding relative accuracy requirements in Table 6.6.4.3.5-4 for all test configurations.

Table 6.6.4.3.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 55 |
| Highest reported value (CSI-RS#1) | 75 |

Table 6.6.4.3.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 58 |
| Highest reported value (CSI-RS#1) | 78 |

Table 6.6.4.3.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF RSRP reported (CSI-RS#0) | 0 |
| Highest DIFF RSRP reported (CSI-RS#0) | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX

6.6.4.4.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.4.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.4.

6.6.4.4.4	Test description

6.6.4.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.4.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.4.4.1-2. Test environment parameters are given in Table 6.6.4.4.4.1-3.

Table 6.6.4.4.4.1-1: NR SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.4.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.4.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.4.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.4.4.1-2 and 6.6.4.4.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.4.4.2	Test procedure

Same test procedure as in subclause 6.6.4.3.4.2 with tables 6.6.4.3.4.1-2 and 6.6.4.3.5-1 replaced by tables 6.6.4.4.4.1-2 and 6.6.4.4.5-1.

6.6.4.4.4.3	Message contents

Same message content as in subclause 6.6.4.3.4.3 with the following exception:

Table 6.6.4.4.4.3-1: Common Exception messages NR SA CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.7-1 with condition DRX.3 |

6.6.4.4.5	Test requirement

Table 6.6.4.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.4.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Note1 | 1~3 | dBm/15kHz | -94.65 |  |
| Note1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
|  | 1~3 | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.15 |
|  | 3 |  | -91.65 | -88.14 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.49 |
|  | 1~3 | dB | 0 | 3.5 |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.4.5-2 for for test configurations 1, 2, 4 and 5 and the corresponding absolute accuracy requirements in Table 6.6.4.4.5-3 for test configurations 3 and 6 and the corresponding relative accuracy requirements in Table 6.6.4.4.5-4 for all test configurations.

Table 6.6.4.4.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 55 |
| Highest reported value (CSI-RS#1) | 75 |

Table 6.6.4.4.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 58 |
| Highest reported value (CSI-RS#1) | 78 |

Table 6.6.4.4.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF RSRP reported (CSI-RS#0) | 0 |
| Highest DIFF RSRP reported (CSI-RS#0) | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

-	Message content is TBD

-	Test case applicability is TBD

-	Test procedure is TBD

-	Test requirement is TBD

6.6.4.5.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements for UE configured with highSpeedMeasFlag-r16 in TS 38.133 clause 9.5.4.1

6.6.4.5.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting 5GS NR SA FR1, measurement enhancements in high speed scenario and long DRX cycle

6.6.4.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.5.

6.6.4.5.4	Test description

6.6.4.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.5.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.5.4.1-2. Test environment parameters are given in Table 6.6.4.5.4.1-3.

Table 6.6.4.5.4.1-1: SA SSB based L1-RSRP measurement supported test configurations for UE configured with highSpeedMeasFlag-r16

| Test Case ID | Description |
| --- | --- |
| 6.6.4.5-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.5-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.5-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.5.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.8 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN 1944 Hz |
|  | 3 |  | AWGN 3334 Hz |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.5.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.4.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n=1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 with n=1 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.5.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in Table 6.6.4.5.4.1-2 and 6.6.4.5.5-1. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. DRX is configured as specified in Table 6.6.4.5.4.1-2.

6.6.4.5.4.2	Test procedure

TBD

6.6.4.5.4.3	Message contents

TBD

6.6.4.5.5	Test requirement

TBD

### 6.6.5

#### 6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX

Editor's note: Following part of this test case are still incomplete:

-	Connect diagram is missing;

-	Message contents is missing;

-	TT analysis is missing

6.6.5.1.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 under the cell search and measurement requirements.

6.6.5.1.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 from Release 16 onwards.

6.6.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.5.1.

6.6.5.1.4	Test description

6.6.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.5.1.4.1-1.

Table 6.6.5.1.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.5.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, UTRAN FDD |
| 6.6.5.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, UTRAN FDD |
| 6.6.5.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, UTRAN FDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.5.1.4.1-2 and Table 6.6.5.1.4.1-3.

Table 6.6.5.1.4.1-2: Initial conditions for SA inter-RAT UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table FFS and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.5.1.5-1 and Table 6.6.5.1.5-2.. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part | FFS |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.5.1.4.1-3: General test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  | 1 | 1 UTRA carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables FFS |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table FFS Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT UTRA measurement quantity |  | CPICH Ec/Io | Measurement quantity for Cell 2 |
| b1-Threshold2UTRA | dB | -16.5 | CPICH Ec/Io threshold for SS-RSRP measurement on cell1 for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.5.1.1-3 |  |  |  |

1.	Message contents are defined in clause 6.6.5.1.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN inter-RAT neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 Annex C.0 and C.1.

6.6.5.1.4.2	Test procedure

The test consists of two successive time periods, with time durations of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of cell 2. Gap pattern configuration is configured before T2 begins to enable inter-frequency monitoring.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.5.1.5-1 and 6.6.5.1.5-2. Propagation conditions are set according to Annex C clause C.2.2.T1 starts.

3.	SS shall transmit an MobilityFromNRCommand message.

4.	The UE shall transmit HANDOVER TO UTRAN COMMAND message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.5.1.5-1 and 6.6.5.1.5-2.

6.	UE shall transmit a MeasurementReport message triggered by Event B2. If the measurement reporting delay from the beginning of time period T2 is less than 3842ms the number of successful tests is increased by one. If the UE fails to report the event within the measurement reporting delay requirement then the number of failure tests is increased by one.

7.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.	Set Cell 2 primary scrambling code = ((current cell 2 primary scrambling code - 50) mod 200 + 100) for next iteration of the test procedure loop.

9.	Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED according to TS 38.508-1 [14] clause 4.5.4 (if the paging fails, switches off and on the UE and ensures the UE is in the state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5,
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.5.1.4.3	Message contents

FFS

6.6.5.1.5	Test requirement

Table 6.6.5.1.5-1 and Table 6.6.5.1.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.5.1.5-1: PCell specific test parameters for SA inter-RAT UTRA FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
| TDD Configuration |  | SCS=15 KHz |  | 2 | TDDConf.1.1 |  |
|  |  | SCS=30 KHz |  | 3 | TDDConf.1.2 |  |
| BWchannel |  |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1 | SR.1.1 FDD |  |
|  |  |  |  | 2 | SR.1.1 TDD |  |
|  |  |  |  | 3 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1 | CR.1.1 FDD |  |
|  |  |  |  | 2 | CR.1.1 TDD |  |
|  |  |  |  | 3 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2 | SSB.1 FR1 |  |
|  |  |  |  | 3 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1 | TRS.1.1 FDD |  |
|  |  |  |  | 2 | TRS.1.1 TDD |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2 | -98 |  |
|  |  |  |  | 3 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 KHz | 1, 2, 3 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2 | -106 |  |
|  |  |  |  | 3 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3 | ETDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.5.1.5-2: UTRAN neighbour cell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 1 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
|  | dB | -Infinity | -1.8 |
|  | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |

The UE shall send one Event B1 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 2.4s from the start of period T2, i.e. when Cell 2 becomes detectable. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

## 6.7	Measurement performance requirements

### 6.7.1	SS-RSRP

#### 6.7.1.0	Minimum conformance requirements

##### 6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.1.

##### 6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.2.

##### 6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.3.

##### 6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.4.

#### 6.7.1.1	Intra-frequency measurements

##### 6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy

6.7.1.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.1.

6.7.1.1.1.4	Test description

6.7.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.1.1.4.1-1.

Table 6.7.1.1.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.1.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.1.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.1.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.1.1.4.1-2.

Table 6.7.1.1.1.4.1-2: Initial conditions for SS-RSRP intra frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 6.7.1.1.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.1.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.1.1.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 6.7.1.1.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.1.1.1.5-1 as appropriate and repeat steps 5-7.

6.7.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.1.1.1.4.3-1: Common Exception messages for NR SA FR1 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-1 | Table H.3.1-3 with Condition SSB.1 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-2 | Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-3 | Table H.3.1-3 with Condition SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.1.1.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRP Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.1.1.1.5	Test requirement

Table 6.7.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.1.1.1.5-3 for test configuration 3.

Table 6.7.1.1.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| Physical cell ID |  |  |  | 489 | 0 |  |  | 489 | 0 |  | 489 |  | 0 |  |
| SSB ARFCN |  |  |  | freq1 |  |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  | - |  | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  |  | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |  |
| SSB configuration |  | Config 1 |  | SSB 1.FR1 |  | SSB.1 FR1 |  | SSB 1.FR1 | SSB.1 FR1 |  | SSB 1.FR1 |  |  | SSB.1 FR1 |
|  |  | Config 2 |  | SSB 1.FR1 |  | SSB.1 FR1 |  | SSB 1.FR1 | SSB.1 FR1 |  | SSB 1.FR1 |  |  | SSB.1 FR1 |
|  |  | Config 3 |  | SSB 2.FR1 |  | SSB.2 FR1 |  | SSB 2.FR1 | SSB.2 FR1 |  | SSB 2.FR1 |  |  | SSB.2 FR1 |
| Time offset with Cell 2 |  | Config 1 | ms | - |  | 3 |  | - | 3 |  | - |  |  | 3 |
|  |  | Config 2,3 | s | - |  | 3 |  | - | 3 |  | - |  |  | 3 |
| SMTC Configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | Depending on band group | dBm/15KhZ | -107.5 |  |  |  | -88 |  |  |  | -116 + ΔBG_offset |  |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  |  |  | -94 |  |  |  | -116 + ΔBG_offset |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -107.4 |  |  |  | -88 |  |  |  | Same as Noc/15kHz |  |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  |  |  | -91 |  |  |  | -113 + ΔBG_offset |  |  |
|  |  |  | dB | 1.88 |  |  | -5.57 | 1.88 |  | -5.57 |  | 0.09 |  | -5.56 |
|  |  |  | dB | 6 |  |  | 1.4 | 6 |  | 1.4 |  | 3 |  | -0.8 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -101.5 |  |  | --106.1 | -82 |  | -86.6 |  | -113 + ΔBG_offset |  | -116.8 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  |  | N/ANote 6 | -85 |  | -89.6 |  | -110+ ΔBG_offset |  | -113.8+ ΔBG_offset |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -71.68 |  |  |  | -52.18 |  |  |  | -82.39+ ΔBG_offset |  |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/ANote 6 |  |  |  | -51.91 |  |  |  | -76.12 + ΔBG_offset |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. Note 6: Subtest 1 is not used when testing with 30kHz SSB SCS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.1.1.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 44 | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 34 |
|  |  |  | Bands NR_FDD_FR1_B | 34 |
|  |  |  | Bands NR_TDD_FR1_C | 35 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 36 |
|  |  |  | Bands NR_FDD_FR1_G | 37 |
|  |  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | 56 | 79 | NR_FDD_FR1_A, NR_TDD_FR1_A | 46 |
|  |  |  | NR_FDD_FR1_B | 46 |
|  |  |  | NR_TDD_FR1_C | 47 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | 47 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | 48 |
|  |  |  | NR_FDD_FR1_G | 49 |
|  |  |  | NR_FDD_FR1_H | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | 40 | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 29 |
|  |  |  | Bands NR_FDD_FR1_B | 30 |
|  |  |  | Bands NR_TDD_FR1_C | 30 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 31 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 31 |
|  |  |  | Bands NR_FDD_FR1_G | 33 |
|  |  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 61 | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | Bands NR_FDD_FR1_B | 51 |
|  |  |  | Bands NR_TDD_FR1_C | 51 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 52 |
|  |  |  | Bands NR_FDD_FR1_G | 53 |
|  |  |  | Bands NR_FDD_FR1_H | 54 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 6.7.1.1.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 37 |
|  |  |  | Bands NR_FDD_FR1_B | 37 |
|  |  |  | Bands NR_TDD_FR1_C | 38 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 38 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 39 |
|  |  |  | Bands NR_FDD_FR1_G | 40 |
|  |  |  | Bands NR_FDD_FR1_H | 40 |
| Highest reported value (Cell 2) | N/A | 76 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 49 |
|  |  |  | Bands NR_FDD_FR1_B | 49 |
|  |  |  | Bands NR_TDD_FR1_C | 50 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 50 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 51 |
|  |  |  | Bands NR_FDD_FR1_G | 52 |
|  |  |  | Bands NR_FDD_FR1_H | 52 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | 54 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 32 |
|  |  |  | Bands NR_FDD_FR1_B | 33 |
|  |  |  | Bands NR_TDD_FR1_C | 33 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 34 |
|  |  |  | Bands NR_FDD_FR1_G | 35 |
|  |  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 2) | N/A | 79 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 53 |
|  |  |  | Bands NR_FDD_FR1_B | 54 |
|  |  |  | Bands NR_TDD_FR1_C | 54 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 55 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 55 |
|  |  |  | Bands NR_FDD_FR1_G | 56 |
|  |  |  | Bands NR_FDD_FR1_H | 57 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy

6.7.1.1.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.1.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.1.

6.7.1.1.2.4	Test description

6.7.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.1.2.4.1-1.

Table 6.7.1.1.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.1.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.1.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.1.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.1.2.4.1-2.

Table 6.7.1.1.2.4.1-2: Initial conditions for SS-RSRP intra frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 6.7.1.1.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.1.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.1.1.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values of Cell 1 and Cell 2 in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 6.7.1.1.2.5-2 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.1.1.2.5-1 as appropriate and repeat steps 5-7.

6.7.1.1.2.4.3	Message contents

Message contents are same as in clause 6.7.1.1.1.4.3.

6.7.1.1.2.5	Test requirement

Table 6.7.1.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.1.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.1.2.5-2.

Table 6.7.1.1.2.5-1: Same as Table 6.7.1.1.1.5-1 with the following exceptions:

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 | Cell 3 |  | Cell 2 | Cell 3 |  | Cell 2 |  | Cell 3 |  |
| Note2 | Config 1,2 | Depending on band group | dBm/15KhZ | -106 |  |  | -88 |  |  |  | -116 + ΔBG_offset |  |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  |  | -94 |  |  |  | -116 + ΔBG_offset |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | Same as Noc/15kHz |  |  | Same as Noc/15kHz |  |  |  | Same as Noc/15kHz |  |  |
|  | Config 3,6 | Depending on band group |  | N/ANote 6 |  |  | -91 |  |  |  | -113 + ΔBG_offset |  |  |
|  |  |  | dB | 1.88 |  | -4.97 | 1.88 |  | -4.97 |  | -0.01 |  | -4.76 |
|  |  |  | dB | 6 |  | 2 | 6 |  | 2 |  | 3 |  | 0 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -100 |  | -104 | -82 |  | -86 |  | -113 + ΔBG_offset |  | -116 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  | N/ANote 6 | -85 |  | -89 |  | -110 + ΔBG_offset |  | -113 + ΔBG_offset |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -70.05 |  |  | -52.05 |  |  |  | -82.20+ ΔBG_offset |  |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/ANote 6 |  |  | -51.77 |  |  |  | -75.93 + ΔBG_offset |  |  |

Table 6.7.1.1.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| RSRP_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.1.2	Inter-frequency measurements

##### 6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy

6.7.1.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.2.

6.7.1.2.1.4	Test description

6.7.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.2.1.4.1-1.

Table 6.7.1.2.1.4.1-1: NR SA FR1-FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.2.1.4.1-2.

Table 6.7.1.2.1.4.1-2: Initial conditions for SS-RSRP inter frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.1.2.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in a different FR1 frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.2.1.4.2	Test procedure

Same as in clause 6.7.1.1.1.4.2 but replacing Table 6.7.1.1.1.5-1 and 6.7.1.1.1.5-2 with 6.7.1.2.1.5-1 and 6.7.1.2.1.5-2, respectively.

6.7.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.1.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.1.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRP Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.1.2.1.5	Test requirement

Table 6.7.1.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.2.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.1.2.1.5-3 for test configuration 3.

Table 6.7.1.2.1.5-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 |  | freq1 | freq2 |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Gap pattern ID |  |  |  | 0 |  |  | 0 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |  |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |  |
|  |  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |  |
|  |  | 3 |  | CR.2.1 TDD | - |  | CR.2.1 TDD | - |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |  |
|  |  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |  |
|  |  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |  |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  | - |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| Time offset between Cell 2 and Cell 3 |  | 1 | ms | 3 |  |  | 3 |  |  |
|  |  | 2,3 | s | 3 |  |  | 3 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |  | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Note2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -94.65 |  | ( for Cell 2 +8dB) + ΔBG_offset | -115+ ΔBG_offset |  |
| Note2 | Depending on band group | 3 | dBm/15kHz | -96 | -96 |  | ( for Cell 2 +8dB) + ΔBG_offset | -115+ ΔBG_offset |  |
| Note2 | Depending on band group | 1,2 | dBm/SSB SCS | -94.65 | -94.65 |  | ( for Cell 2 +8dB) + ΔBG_offset | -115+ ΔBG_offset |  |
|  | Depending on band group | 3 |  | -93 | -93 |  | ( for Cell 2 +8dB) + ΔBG_offset | -112.00+ ΔBG_offset |  |
|  |  | 1~3 | dB | 10 | 10 |  | 13 | -3 |  |
| SS-RSRPNote3 | Depending on band group | 1,2, | dBm/SCS | -84.65 | -84.65 |  | (RSRP for Cell 2 +25dB) + ΔBG_offset | -118.00+ ΔBG_offset |  |
|  | Depending on band group | 3 |  | -83 | -83 |  | (RSRP for Cell 2 +25dB) + ΔBG_offset | -115.00+ ΔBG_offset |  |
| IoNote3 | Depending on band group | 1,2 | dBm/ 9.36MHz | 56.28 | 56.28 |  | (Io for Channel 2 +19.75dB) + ΔBG_offset | -85.28+ ΔBG_offset |  |
|  | Depending on band group | 3 | dBm/ 38.16MHz | -51.53 | -51.53 |  | (Io for Channel 2 +19.75dB) + ΔBG_offset | -79.19+ ΔBG_offset |  |
|  |  | 1~3 | dB | 10 | 10 |  | 13 | -3 |  |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5 The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. Note 6: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 6.7.1.2.1.5-2: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 1, 2, 4 and 5

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 32 |
|  |  | Bands NR_FDD_FR1_B | 33 |
|  |  | Bands NR_TDD_FR1_C | 33 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 34 |
|  |  | Bands NR_FDD_FR1_G | 35 |
|  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 2) | 81 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 45 |
|  |  | Bands NR_FDD_FR1_B | 45 |
|  |  | Bands NR_TDD_FR1_C | 46 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 46 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 47 |
|  |  | Bands NR_FDD_FR1_G | 48 |
|  |  | Bands NR_FDD_FR1_H | 48 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 28 |
|  |  | Bands NR_FDD_FR1_B | 28 |
|  |  | Bands NR_TDD_FR1_C | 29 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 29 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 30 |
|  |  | Bands NR_FDD_FR1_G | 31 |
|  |  | Bands NR_FDD_FR1_H | 31 |
| Highest reported value (Cell 2) | 84 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 49 |
|  |  | Bands NR_FDD_FR1_B | 50 |
|  |  | Bands NR_TDD_FR1_C | 50 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 51 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 51 |
|  |  | Bands NR_FDD_FR1_G | 52 |
|  |  | Bands NR_FDD_FR1_H | 53 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 6.7.1.2.1.5-3: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 3 and 6

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 64 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 35 |
|  |  | Bands NR_FDD_FR1_B | 36 |
|  |  | Bands NR_TDD_FR1_C | 36 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 37 |
|  |  | Bands NR_FDD_FR1_G | 38 |
|  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 2) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 49 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  | Bands NR_FDD_FR1_G | 51 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 32 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 33 |
|  |  | Bands NR_FDD_FR1_G | 34 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 86 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  | Bands NR_FDD_FR1_B | 53 |
|  |  | Bands NR_TDD_FR1_C | 53 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 54 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 54 |
|  |  | Bands NR_FDD_FR1_G | 55 |
|  |  | Bands NR_FDD_FR1_H | 56 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy

6.7.1.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.2.

6.7.1.2.2.4	Test description

6.7.1.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.2.2.4.1-1.

Table 6.7.1.2.2.4.1-1: NR SA FR1-FR1 SS-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.2.2.4.1-2.

Table 6.7.1.2.2.4.1-2: Initial conditions for SS-RSRP inter frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.1.2.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in a different FR1 frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.2.2.4.2	Test procedure

Same as in clause 6.7.1.1.2.4.2 but replacing Table 6.7.1.1.2.5-1 and 6.7.1.1.2.5-2 with 6.7.1.2.2.5-1 and 6.7.1.2.2.5-2, respectively.

6.7.1.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.1.2.1.4.3.

6.7.1.2.2.5	Test requirement

Table 6.7.1.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.2.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.2.2.5-2.

Table 6.7.1.2.2.5-1: same as Table 6.7.1.2.1.5-1

Table 6.7.1.2.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SS-RSRP_x - 7 | SS-RSRP_x - 31 |
| Highest reported value (Cell 2) | SS-RSRP_x + 7 | SS-RSRP_x - 18 |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SS-RSRP_x - 9 | SS-RSRP_x - 33 |
| Highest reported value (Cell 2) | SS-RSRP_x + 9 | SS-RSRP_x - 17 |
| SS-RSRP_x is the reported value of Cell 1 |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.2	SS-RSRQ

#### 6.7.2.0	Minimum conformance requirements

##### 6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements

Same as in clause 4.7.2.0.1.

##### 6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements

Same as in clause 4.7.2.0.2.

##### 6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements

Same as in clause 4.7.2.0.3.

#### 6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy

6.7.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRQ measurement accuracy is within the specified limits for all bands.

6.7.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.1.

6.7.2.1.4	Test description

6.7.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.1.4.1-1.

Table 6.7.2.1.4.1-1: NR SA FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.1.4.1-2.

Table 6.7.2.1.4.1-2: Initial conditions for SS-RSRQ intra frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.3.

6.7.2.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.2.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 6.7.2.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.2.1.5-1 as appropriate and repeat steps 5-7.

6.7.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.2.1.4.3-1: Common Exception messages for NR SA FR1 SS-RSRQ measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-1 | Table H.3.1-3 with Condition SSB.1 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-2 | Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-3 | Table H.3.1-3 with Condition SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRQ Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.2.1.5	Test requirement

Table 6.7.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 6.7.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.1.5-2.

Table 6.7.2.1.5-1: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - |  | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  | - |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 | s | - |  | 3 | - |  | 3 |  | - |  | 3 |
|  |  | Config 1 | ms | - |  | 3 | - |  | 3 |  | - |  | 3 |
| SMTC configuration |  | Config 2, 3 |  | SMTC.1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 |  | 0 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | Depending on band group | dBm/15kHz | -86.5 |  |  | -101 |  |  | -114+ ΔBG_offset |  |  |  |
|  | Config 3 | Depending on band group |  | -92.6 |  |  | - |  |  | -114+ ΔBG_offset |  |  |  |
| Note2 | Config 1,2 | Depending on band group | dBm/SC S | -86.5 |  |  | -101 |  |  | -114+ ΔBG_offset |  |  |  |
|  | Config 3 | Depending on band group |  | -89.6 |  |  | - |  |  | -111+ ΔBG_offset |  |  |  |
|  |  |  | dB | -1.76 |  |  | -4.7 |  |  | -5.46 |  | -5.46 |  |
|  |  |  | dB | 3 | 3 |  | -2.9 | -2.9 |  | -4 |  | -4 |  |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -83.5 | -83.5 |  | -103.9 | -103.9 |  | -118+ΔBG_offset |  | -118+ ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | -86.6 | -86.6 |  | - | - |  | -115+ ΔBG_offset |  | -115+ ΔBG_offset |  |
| SS-RSRQ Note3 |  |  | dB | -14.77 | -14.77 |  | -16.76 | -16.76 |  | -17.34 |  | -17.34 |  |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -51.57 |  |  | -70 |  |  | -83.28+ ΔBG_offset |  |  |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | -51.56 |  |  | - |  |  | -76.67+ ΔBG_offset |  |  |  |
| Propagation condition |  |  | - | AWGN | AWGN |  | AWGN | AWGN |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  |  | 1x2 | 1x2 |  | 1x2 | 1x2 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. Note 6: Subtest 2 is not used when testing with 30kHz SSB SCS. Note 7: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.2.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_52 | SS-RSRQ_46 | SS-RSRQ_44 |
| Highest reported value (Cell 2) | SS-RSRQ_62 | SS-RSRQ_60 | SS-RSRQ_59 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_49 | SS-RSRQ_45 | SS-RSRQ_43 |
| Highest reported value (Cell 2) | SS-RSRQ_65 | SS-RSRQ_61 | SS-RSRQ_60 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy

##### 6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy

6.7.2.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands.

6.7.2.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.2.1.

6.7.2.2.1.4	Test description

6.7.2.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.2.1.4.1-1.

Table 6.7.2.2.1.4.1-1: NR SA FR1-FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.2.1.4.1-2.

Table 6.7.2.2.1.4.1-2: Initial conditions for SS-RSRQ inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.2.2.1.4.2	Test procedure

Same as in clause 6.7.2.1.1.4.2 but replacing Table 6.7.2.1.1.5-1 and 6.7.2.1.1.5-2 with 6.7.2.2.1.5-1 and 6.7.2.2.1.5-2, respectively.

6.7.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.2.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Asynchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.2.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-RSRQ Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.2.2.1.5	Test requirement

Table 6.7.2.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.2.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.2.1.5-2.

Table 6.7.2.2.1.5-1: SS-RSRQ Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  |
| SSB ARFCN |  |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |  |
| Duplex mode |  | Config 1 |  |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 |  | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| Gap pattern ID |  |  |  |  | 0 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |  |
|  |  | Config 2 |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  |  | CR.1.1 FDD | - | R.1.1 FDD | - | CR.1.1 FDD |  |  |
|  |  | Config 2 |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  |
|  |  | Config 2 |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  |
| TRS Configuration |  | Config 1 |  |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | Config 2 |  |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 |  | s | 3 |  |  |  |  |  |  |
|  |  | Config 1 |  | ms | 3 |  |  |  |  |  |  |
| SMTC configuration |  | Config 2, 3 |  |  | SMTC.1 |  |  |  |  |  |  |
|  |  | Config 1 |  |  | SMTC.2 |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  |  | SSB.1 in FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  |  | SSB.2 in FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 |  | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  |  | 30 kHz |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | Depending on band group | dBm/15kHz | -81.68 | -81.68 | -106 | -106 | -116 + ΔBG_offset | -116 + ΔBG_offset |  |
| Note2 | Config 3 |  | Depending on band group | dBm/15kHz | -87.80 | -87.80 | -113 | -113 | -116+ ΔBG_offset | -116+ ΔBG_offset |  |
| Note2 | Config 1,2 |  | Depending on band group | dBm/SCS | -81.68 | -81.68 | -106 | -106 | -116 + ΔBG_offset | -116 + ΔBG_offset |  |
|  | Config 3 |  | Depending on band group |  | -84.8 | -84.8 | -110 | -110 | -113+ ΔBG_offset | -113+ ΔBG_offset |  |
|  |  |  |  | dB | -1.75 | -1.75 | -1.75 | -1.75 | 3 | -1.75 |  |
|  |  |  |  | dB | -1.75 | -1.75 | -1.75 | -1.75 | 3 | -1.75 |  |
| SS-RSRPNote3 | Config 1,2 |  | Depending on band group | dBm/SCS | -83.43 | -83.43 | -107.75 | -107.75 | -113+ ΔBG_offset | -117.75+ ΔBG_offset |  |
|  | Config 3 |  | Depending on band group |  | -86.54 | -86.54 | -111.75 | -111.75 | -110+ ΔBG_offset | -114.75+ ΔBG_offset |  |
| SS-RSRQNote3 |  |  |  | dB | -14.76 | -14.76 | -14.76 | -14.76 | -12.56 | -14.76 |  |
| IoNote3 | Config 1,2 |  | Depending on band group | dBm/Ch BW | -51.51 | -51.51 | -75.83 | -75.83 | -83.28+ ΔBG_offset | -85.83+ ΔBG_offset |  |
|  | Config 3 |  | Depending on band group |  | -51.52 | -51.52 | -76.73 | -76.73 | -77.19+ ΔBG_offset | -79.73+ ΔBG_offset |  |
| Propagation condition |  |  |  | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |  |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.2.2.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_52 | SS-RSRQ_52 | SS-RSRQ_52 |
| Highest reported value (Cell 2) | SS-RSRQ_62 | SS-RSRQ_62 | SS-RSRQ_62 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_49 | SS-RSRQ_49 | SS-RSRQ_49 |
| Highest reported value (Cell 2) | SS-RSRQ_65 | SS-RSRQ_65 | SS-RSRQ_65 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy

6.7.2.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRQ relative measurement accuracy is within the specified limits for all bands.

6.7.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.2.2.

6.7.2.2.2.4	Test description

6.7.2.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.2.2.4.1-1.

Table 6.7.2.2.2.4.1-1: NR SA FR1-FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.2.2.4.1-2.

Table 6.7.2.2.2.4.1-2: Initial conditions for SS-RSRQ inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.2.2.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.2.2.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.2.2.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the SS-RSRQ value of Cell 1 reported by the UE. If the difference between both values is outside the limits in Table 6.7.2.2.2.5-2 or the UE fails to report the measurement value for Cell 2 or Cell 1, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.2.2.2.5-1 as appropriate and repeat steps 5-7.

6.7.2.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.2.2.1.4.3.

6.7.2.2.2.5	Test requirement

Table 6.7.2.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 6.7.2.2.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.2.2.5-2.

Table 6.7.2.2.2.5-1: same as Table 6.7.2.2.1.5-1

Table 6.7.2.2.2.5-2: SS-RSRQ Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x - 7 | SS-RSRQ_x - 7 | SS-RSRQ_x - 11 |
| Highest reported value (Cell 2) | SS-RSRQ_x + 7 | SS-RSRQ_x + 7 | SS-RSRQ_x + 2 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x - 9 | SS-RSRQ_x - 9 | SS-RSRQ_x – 13 |
| Highest reported value (Cell 2) | SS-RSRQ_x + 9 | SS-RSRQ_x + 9 | SS-RSRQ_x + 4 |
| RSRQ_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.3	SS-SINR

#### 6.7.3.0	Minimum conformance requirements

##### 6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.1.

##### 6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.2.

##### 6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.3.

#### 6.7.3.1	NR SA FR1 SS-SINR measurement accuracy

6.7.3.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

6.7.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.1.

6.7.3.1.4	Test description

6.7.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.1.4.1-1.

Table 6.7.3.1.4.1-1: NR SA FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.1.4.1-2.

Table 6.7.3.1.4.1-2: Initial conditions for SS-SINR intra frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.3.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 6.7.3.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.3.1.5-1 as appropriate and repeat steps 5-7.

6.7.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.3.1.4.3-1: Common Exception messages for NR SA FR1 SS-SINR measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-1 | Table H.3.1-3 with Condition Asynchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-2 | Table H.3.1-3 with Condition Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-3 | Table H.3.1-3 with Condition Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.3.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-SINR Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.3.1.5	Test requirements

Table 6.7.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.3.1.5-2

Table 6.7.3.1.5-1: SS-SINR Intra frequency test parameters

| Parameter |  |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  |  | Cell 2 |
| SSB ARFCN |  |  |  |  |  | freq1 |  |  | freq1 |  |  |  |
| Duplex mode |  |  | Config 1 |  |  | FDD |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  |  | TDD |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  |  | Not Applicable |  |  |  |  |  |  |
|  |  |  | Config 2 |  |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | TDDConf.2.1 |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  |  | ms | Not Applicable |  |  |  |  |  |  |
| TRS configuration |  |  | Config 1 |  |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  | TRS.1.1 TDD | - |  | TRS.1.1 TDD |  |  | - |
|  |  |  | Config 3 |  |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  |  | - |
|  |  |  | Config 2 |  |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | SR.2.1 TDD |  |  | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  |  | - |
|  |  |  | Config 2 |  |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  |
| OCNG Patterns |  |  |  |  |  | OP.1 |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  |  | Not Applicable |  |  |  |  |  |  |
| Time offset with Cell 1 |  |  |  | Config 2, 3 | s | - |  | 3 |  | - | 3 |  |
|  |  |  |  | Config 1 | ms | - |  | 3 |  | - | 3 |  |
| SMTC configuration |  |  |  | Config 2, 3 |  | SMTC.1 |  |  |  |  |  |  |
|  |  |  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 |  | kHz | 15 |  |  |  |  |  |  |
|  |  |  | Config 3 |  |  | 30 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  | dB | 0 | 0 |  | 0 |  |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | Depending on band group |  | dBm/15kHz | -93 |  |  | -116+ ΔBG_offset |  |  |  |
| Note2 | Config 1,2 |  |  |  | dBm/SCS | -93.2 |  |  | Same as Noc for 15kHz |  |  |  |
|  | Config 3 |  | Depending on band group |  |  | -90.2 |  |  | -113+ ΔBG_offset |  |  |  |
|  |  |  |  |  | dB | 0 | -3.19 |  | -5.46 |  |  | -5.46 |
|  |  |  |  |  | dB | 4.54 | 2.66 |  | -3.5 |  |  | -3.5 |
| SS-RSRPNote3 | Config 1,2 |  | Depending on band group |  | dBm/SCS | -88.46 | -90.34 |  | -119.5+ ΔBG_offset |  |  | -119.5+ ΔBG_offset |
|  | Config 3 |  | Depending on band group |  |  | -85.65 | -87.53 |  | -116.5+ ΔBG_offset |  |  | -116.5+ ΔBG_offset |
| SS-SINR Note3 |  |  |  |  | dB | 0 | -3.19 |  | -5.1 |  |  | -5.1 |
| IoNote3 |  | Config 1,2 | Depending on band group |  | dBm/ 9.36MHz | -57.5 |  |  | -85.28+ ΔBG_offset |  |  |  |
|  |  | Config 3 | Depending on band group |  | dBm/ 38.16MHz | -51.59 |  |  | -79.17+ ΔBG_offset |  |  |  |
| Propagation condition |  |  |  |  | - | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  |  |  | - | 1x2 |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.3.1.5-3: SS-SINR Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SS-SINR_31 | SS-SINR_28 |
| Highest reported value (Cell 2) | SS-SINR_49 | SS-SINR_45 |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SS-SINR_30 | SS-SINR_27 |
| Highest reported value (Cell 2) | SS-SINR_50 | SS-SINR_46 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.3.2	Inter-Frequency SS-SINR measurement accuracy

##### 6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy

6.7.3.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands.

6.7.3.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.2.1.

6.7.3.2.1.4	Test description

6.7.3.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.2.1.4.1-1.

Table 6.7.3.2.1.4.1-1: NR SA FR1-FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.2.1.4.1-2.

Table 6.7.3.2.1.4.1-2: Initial conditions for SS-SINR inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.2.1.4.2	Test procedure

Same as in clause 6.7.3.1.4.2 but replacing Table 6.7.3.1.5-1 and 6.7.3.1.5-2 with 6.7.3.2.1.5-1 and 6.7.3.2.1.5-2, respectively.

6.7.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.3.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-SINR absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO, and Asynchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, and Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, and Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.3.2.1.4.3-2: ReportConfigNR-DEFAULT(Periodical) for NR SA FR1 SS-SINR Accuracy

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.3.2.1.5	Test requirements

Table 6.7.3.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.3.2.1.5-2.

Table 6.7.3.2.1.5-1: SS-SINR Inter frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  | Test 2 |  |  |  | Test 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  | Cell 1 |  | Cell 2 |  |  |
| SSB ARFCN |  |  |  |  | freq1 | freq2 |  | freq1 |  | freq2 |  | freq1 |  | freq2 |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Gap pattern ID |  |  |  |  | 0 |  | - |  | 0 |  | - |  | 0 |  |  | - |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |  | TRS.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD | - |  | TRS.1.1 TDD |  | - |  | TRS.1.1 TDD |  | - |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |  | SR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD |  | - |  | CR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |  | CCR.1.1 FDD |  | - |  |  |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 |  | s | - |  | 3 |  | - |  | 3 |  | - |  |  | 3 |
|  |  | Config 1 |  | ms | - |  | 3 |  | - |  | 3 |  | - |  |  | 3 |
| SMTC configuration |  | Config 2, 3 |  |  | SMTC.1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Config 1 |  |  | SMTC.2 |  |  |  |  |  |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | Depending on band group | dBm/15kHz | -88 | -88 |  | -108.5 |  | -108.5 |  | -119.5+ ΔBG_offset |  |  | -119.5+ ΔBG_offset |  |
| Note2 | Config 1,2 |  |  | dBm/SCS | -88 | -88 |  | -108.5 |  | -108.5 |  | Same as Noc for 15kHz |  |  | Same as Noc for 15kHz |  |
|  | Config 3 |  | Depending on band group |  | -85 | -85 |  | -105.5 |  | -105.5 |  | -116.5+ ΔBG_offset |  |  | 116.5+ ΔBG_offset |  |
|  |  |  |  | dB | -1.75 | -1.75 |  | 20 |  | 20 |  | -3.2 |  |  | -3.2 |  |
|  |  |  |  | dB | -1.75 | -1.75 |  | 20 |  | 20 |  | -3.2 |  |  | -3.2 |  |
| SS-RSRPNote3 | Config 1,2 |  | Depending on band group | dBm/SCS | -89.75 | -89.75 |  | -88.5 |  | -88.5 |  | -122.7+ ΔBG_offset |  |  | -122.7+ ΔBG_offset |  |
|  | Config 3 |  | Depending on band group |  | -86.75 | -86.75 |  | -85.5 |  | -85.5 |  | -119.7+ ΔBG_offset |  |  | -119.7+ ΔBG_offset |  |
| SS-SINR Note3 |  |  |  | dB | -1.75 | -1.75 |  | -1.75 |  | -1.75 |  | -3.2 |  |  | -3.2 |  |
| IoNote3 | Config 1,2 |  | Depending on band group | dBm/ 9.36MHz | -57.83 | -57.83 |  | -60.5 |  | -60.5 |  | -89.85+ ΔBG_offset |  |  | -89.85+ ΔBG_offset |  |
|  | Config 3 |  | Depending on band group | dBm/ 38.16MHz | -51.73 | -51.73 |  | -54.41 |  | -54.41 |  | -83.75+ ΔBG_offset |  |  | -83.75+ ΔBG_offset |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |  |  |  |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.7.3.2.1.5-2: SS-SINR Inter frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_35 | SS-SINR_79 | SS-SINR_32 |
| Highest reported value (Cell 2) | SS-SINR_51 | SS-SINR_94 | SS-SINR_49 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_33 | SS-SINR_77 | SS-SINR_31 |
| Highest reported value (Cell 2) | SS-SINR_53 | SS-SINR_96 | SS-SINR_50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy

6.7.3.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR relative measurement accuracy is within the specified limits for all bands.

6.7.3.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.2.2.

6.7.3.2.2.4	Test description

6.7.3.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.2.2.4.1-1.

Table 6.7.3.2.2.4.1-1: NR SA FR1-FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.2.2.4.1-2.

Table 6.7.3.2.2.4.1-2: Initial conditions for SS-SINR inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.2.2.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.2.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.3.2.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the SS- SINR value of Cell 1 reported by the UE. If the difference between both values is outside the limits in Table 6.7.3.2.2.5-2 or the UE fails to report the measurement value for Cell 2 or Cell 1, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.3.2.2.5-1 as appropriate and repeat steps 5-7.

6.7.3.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.3.2.1.4.3.

6.7.3.2.2.5	Test requirements

Table 6.7.3.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.2.2.5-1 shall meet the corresponding relative accuracy requirements in Table 6.7.3.2.2.5-2

Table 6.7.3.2.2.2.5-1: same as Table 6.7.3.2.2.1.5-1

Table 6.7.3.2.2.5-2: SS-SINR Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x - 10 | SS-SINR_x - 10 | SS-SINR_x - 11 |
| Highest reported value (Cell 2) | SS-SINR_x + 10 | SS-SINR_x + 10 | SS-SINR_x + 11 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x - 12 | SS-SINR_x - 12 | SS-SINR_x - 12 |
| Highest reported value (Cell 2) | SS-SINR_x + 12 | SS-SINR_x + 12 | SS-SINR_x + 12 |
| RSRQ_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.4	L1-RSRP

#### 6.7.4.0	Minimum conformance requirements

##### 6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.1.

##### 6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.2.

##### 6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.3.

##### 6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.4.

#### 6.7.4.1	SSB based L1-RSRP measurements

##### 6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy

6.7.4.1.1.1	Test purpose

The purpose of this test is to verify that the SSB based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.4.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.1.

6.7.4.1.1.4	Test description

6.7.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.1.1.4.1-1.

Table 6.7.4.1.1.4.1-1: NR SA FR1 SSB based L1-RSRP absolute measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.1.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.1.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.1.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.1.1.4.1-2.

Table 6.7.4.1.1.4.1-2: Initial conditions for SSB based L1-RSRP absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.1.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for SSB-based L1-RSRP measurements. The UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.1.1.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.1.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.1.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.1.1.5-1.

3.	The UE shall start sending L1-RSRP report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of SSB#0 and SSB#1 in the periodic L1-RSRP reports. If the value for both SSBs is within the limits in Table 6.7.4.1.1.5-2 or Table 6.7.4.1.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.1.1.5-1 as appropriate and repeat steps 3-5.

6.7.4.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.4.1.1.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with conditions SSB and PERIODIC Table H.3.5-8 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.4.1.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.4.1.1.5	Test requirement

Table 6.7.4.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.4.1.1.5-3 for test configuration 3.

Table 6.7.4.1.1.5-1: L1-RSRP test parameters

| Parameter |  |  | Config |  | Unit |  | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  |  | 1~3 |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | 1 |  |  |  | FDD |  | FDD |  |
|  |  |  | 2 |  |  |  | TDD |  | TDD |  |
|  |  |  | 3 |  |  |  | TDD |  | TDD |  |
| TDD Configuration |  |  | 1 |  |  |  | N/A |  | N/A |  |
|  |  |  | 2 |  |  |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  |  | 3 |  |  |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  |  | 1 |  | MHz |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  |  | 2 |  |  |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  |  | 3 |  |  |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  |  | 1 |  |  |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | 1 |  |  |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | 1 |  |  |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| SSB configuration |  |  | 1 |  |  |  | SSB.3 FR1 |  | SSB.3 FR1 |  |
|  |  |  | 2 |  |  |  | SSB.3 FR1 |  | SSB.3 FR1 |  |
|  |  |  | 3 |  |  |  | SSB.4 FR1 |  | SSB.4 FR1 |  |
| OCNG Patterns |  |  | 1~3 |  |  |  | OP.1 |  | OP.1 |  |
| TRS configuration |  |  | 1~3 |  |  |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  |  |  | 1 |  |  |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  |  | 2 |  |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration |  |  | 3 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  |  | 1~3 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  |  | 1~3 |  |  |  | SMTC.1 |  | SMTC.1 |  |
| reportConfigType |  |  | 1~3 |  |  |  | periodic |  | periodic |  |
| reportQuantity |  |  | 1~3 |  |  |  | ssb-Index-RSRP |  | ssb-Index-RSRP |  |
| Number of reported RS |  |  | 1~3 |  |  |  | 2 |  | 2 |  |
| L1-RSRP reporting period |  |  | 1~3 |  |  |  | slot80 |  | slot80 |  |
| EPRE ratio of PSS to SSS |  |  | 1~3 |  | dB |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | Depending on band group | 1,2 |  | dBm/15kHz |  | -94.65 |  | -117+ ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -96.00 |  | 117+ ΔBG_offset |  |
| Note2 |  |  | 1,2 |  | dBm/SSB SCS |  | -94.65 |  | -117+ ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -93.00 |  | -114+ ΔBG_offset |  |
|  |  |  | 1~3 |  | dB |  | 10 |  | -2.2 |  |
| SSB RSRP Note3 |  | Depending on band group | 1,2 |  | dBm/SSB SCS |  | -84.65 |  | -120.2 + ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -81.65 |  | -117.2 + ΔBG_offset |  |
| Io Note3 |  | Depending on band group | 1,2 |  | dBm/9.36 MHz |  | -56.28 |  | -87.35 + ΔBG_offset |  |
|  |  |  | 3 |  | dBm/38.16 MHz |  | -51.53 |  | -81.25 + ΔBG_offset |  |
|  |  |  | 1~3 |  | dB |  | 10 |  | -2.2 |  |
| Propagation condition |  |  | 1~3 |  |  |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  | 1~3 |  |  |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |

Table 6.7.4.1.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 32 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 33 |
|  |  | Bands NR_FDD_FR1_G | 34 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 1) | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 44 |
|  |  | Bands NR_FDD_FR1_B | 45 |
|  |  | Bands NR_TDD_FR1_C | 45 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 46 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 46 |
|  |  | Bands NR_FDD_FR1_G | 47 |
|  |  | Bands NR_FDD_FR1_H | 48 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 30 |
|  |  | Bands NR_FDD_FR1_B | 30 |
|  |  | Bands NR_TDD_FR1_C | 31 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 31 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 32 |
|  |  | Bands NR_FDD_FR1_G | 33 |
|  |  | Bands NR_FDD_FR1_H | 33 |
| Highest reported value (Cell 1) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 45 |
|  |  | Bands NR_FDD_FR1_B | 46 |
|  |  | Bands NR_TDD_FR1_C | 46 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 47 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 47 |
|  |  | Bands NR_FDD_FR1_G | 48 |
|  |  | Bands NR_FDD_FR1_H | 49 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 6.7.4.1.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | 63 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 34 |
|  |  | Bands NR_FDD_FR1_B | 34 |
|  |  | Bands NR_TDD_FR1_C | 35 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 36 |
|  |  | Bands NR_FDD_FR1_G | 37 |
|  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 1) | 84 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 47 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 48 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 49 |
|  |  | Bands NR_FDD_FR1_G | 50 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  | Bands NR_FDD_FR1_B | 33 |
|  |  | Bands NR_TDD_FR1_C | 34 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
|  |  | Bands NR_FDD_FR1_G | 36 |
|  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 1) | 85 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  | Bands NR_FDD_FR1_B | 49 |
|  |  | Bands NR_TDD_FR1_C | 49 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 50 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  | Bands NR_FDD_FR1_G | 51 |
|  |  | Bands NR_FDD_FR1_H | 52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy

6.7.4.1.2.1	Test purpose

The purpose of this test is to verify that the SSB based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.4.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.1.

6.7.4.1.2.4	Test description

6.7.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.1.2.4.1-1.

Table 6.7.4.1.2.4.1-1: NR SA FR1 SSB based L1-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.1.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.1.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.1.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.1.2.4.1-2.

Table 6.7.4.1.2.4.1-2: Initial conditions for SSB based L1-RSRP relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.1.2.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for SSB-based L1-RSRP measurements. The UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.1.2.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.1.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.1.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.1.2.5-1.

3.	The UE shall start sending L1-RSRP report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of SSB#0 and SSB#1 in the periodic L1-RSRP reports. The L1-RSRP value for SSB#1 is compared to the L1-RSRP value for SSB#0. If the difference is within the limits in Table 6.7.4.1.2.5-2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.1.2.5-1 as appropriate and repeat steps 3-5.

6.7.4.1.2.4.3	Message contents

Message contents  are same as in Clause 6.7.4.1.1.4.3.

6.7.4.1.2.5	Test requirement

Table 6.7.4.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.1.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.1.2.5-2.

Table 6.7.4.1.2.5-1: Same as Table 6.7.4.1.1.5-1

Table 6.7.4.1.2.5-2: L1-RSRP relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 1 SSB resource 1) | RSRP_x - 3 | RSRP_x - 3 |
| Highest reported value (Cell 1 SSB resource 1) | RSRP_x + 3 | RSRP_x + 3 |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 1 SSB resource 1)) | RSRP_x - 4 | RSRP_x - 4 |
| Highest reported value (Cell 1 SSB resource 1) | RSRP_x + 4 | RSRP_x + 4 |
| RSRP_x is the reported value of Cell 1 SSB resource 0 |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.4.2	CSI-RS based L1-RSRP measurements

##### 6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy

6.7.4.2.1.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.4.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.2.

6.7.4.2.1.4	Test description

6.7.4.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.2.1.4.1-1.

Table 6.7.4.2.1.4.1-1: NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.2.1.4.1-2.

Table 6.7.4.2.1.4.1-2: Initial conditions for CSI-RS based L1-RSRP absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.2.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-RSRP measurements. The UE is configured to perform RLM and BFD measurement based on the SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.2.1.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.2.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.2.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.2.1.5-1.

3.	The UE shall start sending L1-RSRP report including results of both CSI-RS#0 and CSI-RS#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of CSI-RS#0 and CSI-RS#1 in the periodic L1-RSRP reports. If the value for both CSI-RSs is within the limits in Table 6.7.4.2.1.5-2 or Table 6.7.4.2.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.2.1.5-1 as appropriate and repeat steps 3-5.

6.7.4.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.4.2.1.4.3-1: Common Exception messages EN-DC CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and CSI-RSRP Table H.3.6-3 with conditions CSI-RS and PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.4.2.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSB. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.4.2.1.5	Test requirement

Table 6.7.4.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.2.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.4.2.1.5-3 for test configuration 3.

Table 6.7.4.2.1.5-1: L1-RSRP test parameters

| Parameter |  |  | Config |  | Unit |  | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  |  | 1~3 |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | 1 |  |  |  | FDD |  | FDD |  |
|  |  |  | 2 |  |  |  | TDD |  | TDD |  |
|  |  |  | 3 |  |  |  | TDD |  | TDD |  |
| TDD Configuration |  |  | 1 |  |  |  | N/A |  | N/A |  |
|  |  |  | 2 |  |  |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  |  | 3 |  |  |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  |  | 1 |  | MHz |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  |  | 2 |  |  |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  |  | 3 |  |  |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  |  | 1 |  |  |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | 1 |  |  |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | 1 |  |  |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  |  | 2 |  |  |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  | 3 |  |  |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| SSB configuration |  |  | 1 |  |  |  | SSB.3 FR1 |  | SSB.3 FR1 |  |
|  |  |  | 2 |  |  |  | SSB.3 FR1 |  | SSB.3 FR1 |  |
|  |  |  | 3 |  |  |  | SSB.4 FR1 |  | SSB.4 FR1 |  |
| OCNG Patterns |  |  | 1~3 |  |  |  | OP.1 |  | OP.1 |  |
| TRS configuration |  |  | 1~3 |  |  |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  |  |  | 1 |  |  |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  |  | 2 |  |  |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| Initial BWP Configuration |  |  | 3 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  |  | 1~3 |  |  |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  |  | 1~3 |  |  |  | SMTC.1 |  | SMTC.1 |  |
| CSI-RS |  |  | 1,4 |  |  |  | CSI-RS 1.2 FDD |  | CSI-RS 1.2 FDD |  |
|  |  |  | 2,5 |  |  |  | CSI-RS 1.2 TDD |  | CSI-RS 1.2 TDD |  |
|  |  |  | 3,6 |  |  |  | CSI-RS 2.2 TDD |  | CSI-RS 2.2 FDD |  |
| reportConfigType |  |  | 1~3 |  |  |  | periodic |  | periodic |  |
| reportQuantity |  |  | 1~3 |  |  |  | ssb-Index-RSRP |  | ssb-Index-RSRP |  |
| Number of reported RS |  |  | 1~3 |  |  |  | 2 |  | 2 |  |
| L1-RSRP reporting period |  |  | 1~3 |  |  |  | slot80 |  | slot80 |  |
| EPRE ratio of PSS to SSS |  |  | 1~3 |  | dB |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | Depending on band group | 1,2 |  | dBm/15kHz |  | -94.65 |  | -117+ ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -96.00 |  | 117+ ΔBG_offset |  |
| Note2 |  |  | 1,2 |  | dBm/CSI-RS SCS |  | -94.65 |  | -117+ ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -93.00 |  | -114+ ΔBG_offset |  |
|  |  |  | 1~3 |  | dB |  | 10 |  | -2.2 |  |
| CSI-RSRP Note3 |  | Depending on band group | 1,2 |  | dBm/CSI-RS SCS |  | -84.65 |  | -120.2 + ΔBG_offset |  |
|  |  |  | 3 |  |  |  | -81.65 |  | -117.2 + ΔBG_offset |  |
| Io Note3 |  | Depending on band group | 1,2 |  | dBm/9.36 MHz |  | -56.28 |  | -87.35 + ΔBG_offset |  |
|  |  |  | 3 |  | dBm/38.16 MHz |  | -51.53 |  | -81.25 + ΔBG_offset |  |
|  |  |  | 1~3 |  | dB |  | 10 |  | -2.2 |  |
| Propagation condition |  |  | 1~3 |  |  |  | AWGN |  | AWGN |  |
| Antenna configuration |  |  | 1~3 |  |  |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |

Table 6.7.4.2.1.5-2: Same as Table 6.7.4.1.1.5-2

Table 6.7.4.2.1.5-3: Same as Table 6.7.4.1.1.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy

6.7.4.2.2.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.4.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.2.

6.7.4.2.2.4	Test description

6.7.4.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.2.2.4.1-1.

Table 6.7.4.2.2.4.1-1: NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.2.2.4.1-2.

Table 6.7.4.2.2.4.1-2: Initial conditions for CSI-RS based L1-RSRP relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.2.2.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-RSRP measurements. The UE is configured to perform RLM and BFD measurement based on the SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.2.2.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.2.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.2.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.2.2.5-1.

3.	The UE shall start sending L1-RSRP report including results of both CSI-RS#0 and CSI-RS#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of CSI-RS#0 and CSI-RS#1 in the periodic L1-RSRP reports. The L1-RSRP value for CSI-RS #1 is compared to the L1-RSRP value for CSI-RS #0. If the difference is within the limits in Table 6.7.4.2.2.5-2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.2.2.5-1 as appropriate and repeat steps 3-5.

6.7.4.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.4.2.1.4.3.

6.7.4.2.2.5	Test requirement

Table 6.7.4.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.2.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.2.2.5-2.

Table 6.7.4.2.2.5-1: Same as Table 6.7.4.2.1.5-1

Table 6.7.4.2.2.5-2: Same as Table 6.7.4.1.2.5-2

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.5	E-UTRAN RSRP

#### 6.7.5.0	Minimum conformance requirements

##### 6.7.5.0.1	E-UTRAN RSRP absolute accuracy

The measurement period of E-UTRA RSRP in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 of TS 38.133 [6].

The accuracy requirements of E-UTRA RSRP measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRP Accuracy Requirements in clause 9.1.3 of TS 36.133 [23]:

The requirements for absolute accuracy of RSRP in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 6.7.5.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 of TS 36.133 [23] for a corresponding Band.

Table 6.7.5.0.1-1: RSRP Inter frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 4.5 | 9 | -6 dB | FDD_A, TDD_A | -121 | N/A | -70 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | N/A | -70 |
|  |  |  | FDD_C, TDD_C | -120 | N/A | -70 |
|  |  |  | FDD_D | -119.5 | N/A | -70 |
|  |  |  | FDD_E, TDD_E | -119 | N/A | -70 |
|  |  |  | FDD_F | -118.5 | N/A | -70 |
|  |  |  | FDD_G | -118 | N/A | -70 |
|  |  |  | FDD_H | -117.5 | N/A | -70 |
|  |  |  | FDD_N | -114.5 | N/A | -70 |
| 8 | 11 | -6 dB | FDD_A, TDD_A, FDD_B1, FDD_B2, FDD_C, TDD_C, FDD_D, FDD_E, TDD_E, FDD_F, FDD_G, FDD_H, FDD_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. |  |  |  |  |  |  |

The reporting range and mapping specified for RSRP measurements in clause 9.1.4 of TS 36.133 [23] shall apply:

The reporting range of RSRP is defined from -156 dBm to -44 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 6.7.5.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.5.0.1-2: RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRP_-17 | RSRP< -156 | dBm |
| RSRP_-16 | -156 RSRP< -155 | dBm |
| … | … | … |
| RSRP_-03 | -143 RSRP< -142 | dBm |
| RSRP_-02 | -142 RSRP< -141 | dBm |
| RSRP_-01 | -141 RSRP< -140 | dBm |
| RSRP_00 | RSRP -140 | dBm |
| RSRP_01 | -140 RSRP < -139 | dBm |
| RSRP_02 | -139 RSRP < -138 | dBm |
| … | … | … |
| RSRP_95 | -46 RSRP < -45 | dBm |
| RSRP_96 | -45 RSRP < -44 | dBm |
| RSRP_97 | -44 RSRP | dBm |

#### 6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy

6.7.5.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RSRP absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.5.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

6.7.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.5.1.

6.7.5.1.4	Test description

6.7.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.5.1.4.1-1.

Table 6.7.5.1.4.1-1:  test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.5.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.5.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.5.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.5.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.5.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.5.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.5.1.4.1-2.

Table 6.7.5.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.5.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.5.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.5.1.5-1 and Table 6.7.5.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RSRP reported values in the periodic MeasurementReport. The RSRP value of Cell 2 reported by the UE is compared to the expected RSRP. If the value is outside the limits in Table 6.7.5.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.5.1.5-1 as appropriate and repeat steps 5-7.

6.7.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.5.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-RAT Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.5.1.4.3-2: ReportConfigE-UTRA-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-141 with Condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.5.1.5	Test requirement

Table 6.7.5.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.5.1.5-1 and 6.7.5.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.5.1.5-3.

Table 6.7.5.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.1.2 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.5.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 1 |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |
|  | Config 4, 5, 6 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | Depending on band group | dBm/15kHz | -91.65 | -117 + ΔBG_offset |
| Ês/Noc |  | dB | 10 | -3.2 |
| Ês/IotNote5 |  | dB | 10 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -81.65 | -120.2+ ΔBG_offset |
| SCH_RPNote5 | Depending on band group | dBm/15kHz | -81.65 | -120.2+ ΔBG_offset |
| IoNote5 | Depending on band group | dBm/Ch BW | -53.45 + 10log(NRB,c/50) | -87.52+ ΔBG_offset + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |

Table 6.7.5.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 |  |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 48 | FDD_A, TDD_A | 14 |  |
|  |  | FDD_B | 15 |  |
|  |  | TDD_C | 15 |  |
|  |  | FDD_D, TDD_D | 16 |  |
|  |  | FDD_E, TDD_E | 16 |  |
|  |  | FDD_G | 17 |  |
|  |  | FDD_H | 18 |  |
| Highest reported value (Cell 2) | 70 | FDD_A, TDD_A | 27 |  |
|  |  | FDD_B | 27 |  |
|  |  | TDD_C | 28 |  |
|  |  | FDD_D, TDD_D | 28 |  |
|  |  | FDD_E, TDD_E | 29 |  |
|  |  | FDD_G | 30 |  |
|  |  | FDD_H | 30 |  |
| Extreme Conditions | Test 1 All bands | Test 2 |  |  |
| Lowest reported value (Cell 2) | 46 | FDD_A, TDD_A | 11 |  |
|  |  | FDD_B | 11 |  |
|  |  | TDD_C | 12 |  |
|  |  | FDD_D, TDD_D | 12 |  |
|  |  | FDD_E, TDD_E | 13 |  |
|  |  | FDD_G | 14 |  |
|  |  | FDD_H | 14 |  |
| Highest reported value (Cell 2) | 72 | FDD_A, TDD_A | 30 |  |
|  |  | FDD_B | 31 |  |
|  |  | TDD_C | 31 |  |
|  |  | FDD_D, TDD_D | 32 |  |
|  |  | FDD_E, TDD_E | 32 |  |
|  |  | FDD_G | 33 |  |
|  |  | FDD_H | 34 |  |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.6	E-UTRAN RSRQ

#### 6.7.6.0	Minimum conformance requirements

##### 6.7.6.0.1	E-UTRAN RSRQ absolute accuracy

The measurement period of E-UTRA RSRQ in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 on TS 38.133 [6].

The accuracy requirements of E-UTRA RSRQ measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRQ Accuracy Requirements in clause 9.1.6 of TS 36.133 [23]:

The requirements for absolute accuracy of RSRQ in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 6.7.6.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 of TS 36.133 [23] for a corresponding Band

Table 6.7.6.0.1-1: RSRQ Inter frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 2.5 | 4 | -3 dB | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 3.5 | 4 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. |  |  |  |  |  |

The reporting range and mapping specified for RSRQ measurements in clause 9.1.7 of TS 36.133 [23] shall apply:

The reporting range of RSRQ is defined from -34 dB to 2.5 dB with 0.5 dB resolution.

The mapping of measured quantity is defined in table 6.7.6.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.6.0.1-2: RSRQ measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRQ_-30 | RSRQ -34 | dB |
| RSRQ_-29 | -34 RSRQ < -33.5 | dB |
| … | … | … |
| RSRQ_-02 | -20.5 RSRQ < -20 | dB |
| RSRQ_-01 | -20 RSRQ < -19.5 | dB |
| RSRQ_00 | RSRQ -19.5 | dB |
| RSRQ_01 | -19.5 RSRQ < -19 | dB |
| RSRQ_02 | -19 RSRQ < -18.5 | dB |
| … | … | … |
| RSRQ_32 | -4 RSRQ < -3.5 | dB |
| RSRQ_33 | -3.5 RSRQ < -3 | dB |
| RSRQ_34 | -3 RSRQ | dB |
| RSRQ_35 | -3 RSRQ < -2.5 | dB |
| RSRQ_36 | -2.5 RSRQ < -2 | dB |
| … | … | … |
| RSRQ_45 | 2 RSRQ < 2.5 | dB |
| RSRQ_46 | 2.5 RSRQ | dB |

Note: The ranges from RSRQ_-30 to RSRQ_-01 and from RSRQ_35 to RSRQ_46 apply for the UE who can support extended RSRQ range.

#### 6.7.6.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy

6.7.6.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RSRQ absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.6.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

6.7.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.6.1.

6.7.6.1.4	Test description

6.7.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.6.1.4.1-1.

Table 6.7.6.1.4.1-1: test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.6.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.6.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.6.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.6.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.6.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.6.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.6.1.4.1-2.

Table 6.7.6.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.6.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.6.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.6.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.6.1.5-1 and Table 6.7.6.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RSRQ reported values in the periodic MeasurementReport. The RSRQ value of Cell 2 reported by the UE is compared to the expected RSRQ. If the value is outside the limits in Table 6.7.6.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.6.1.5-1 as appropriate and repeat steps 5-7.

6.7.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.6.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-RAT Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.6.1.4.3-2: ReportConfigE-UTRA-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-141 with Condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.6.1.5	Test requirement

Table 6.7.6.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.6.1.5-1 and 6.7.6.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.6.1.5-3.

Table 6.7.6.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.1.2 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.6.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Depending on band group | dBm/15kHz | -83 | -104.70 | -119.5+ ΔBG_offset |
| Ês/Noc |  | dB | -1.75 | -3.2 | -3.2 |
| Ês/IotNote5 |  | dB | -1.75 | -3.2 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -84.75 | -107.90 | -122.7+ ΔBG_offset |
| RSRQNote5 | Depending on band group | dB | -14.76 | -15.69 | -15.69+ ΔBG_offset |
| IoNote5 | Depending on band group | dBm/Ch BW | -53 + 10log(NRB,c /50) | -75.22 + 10log(NRB,c /50) | -90.02+ ΔBG_offset + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table 6.7.6.1.5-3: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 4 | 0 | 0 |
| Highest reported value (Cell 2) | 16 | 16 | 16 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest reported value (Cell 2) | 1 | 0 | 0 |
| Highest reported value (Cell 2) | 19 | 17 | 17 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.7	E-UTRAN RS-SINR

#### 6.7.7.0	Minimum conformance requirements

##### 6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy

The measurement period of E-UTRA RS-SINR in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 of TS 38.133 [6].

The accuracy requirements of E-UTRA RS-SINR measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RS-SINR Accuracy Requirements in clause 9.1.17.3 of TS 36.133 [23]:

The reporting range and mapping for E-UTRA RS-SINR measurements shall be the same as specified for RS-SINR measurements in clause 9.1.17.1 of TS 36.133 [23]:

The requirements for absolute accuracy of intra-frequency RS-SINR in this clause apply to a cell on the same frequency as that of the serving cell.

The accuracy requirements in Table 6.7.7.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.18 of TS 36.133 [23] for a corresponding Band.

Table 6.7.7.0.1-1: Intra-frequency RS-SINR absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 3.0 | 4 | -3 dB Note 5 | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 3.5 | 4 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. NOTE 5: The requirements apply for Ês/Iot ≤ 25 dB. |  |  |  |  |  |

The reporting range of RS-SINR measurement is defined from -23 dB to 40 dB with 0.5 dB resolution.

The mapping of the measured quantity is defined in table 6.7.7.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.7.0.1-2: RS-SINR measurement report mapping

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RS-SINR_000 | RS-SINR -23 | dB |
| RS-SINR_001 | -23 RS-SINR < -22.5 | dB |
| … | … | … |
| RS-SINR_126 | 39.5 RS-SINR < 40 | dB |
| RS-SINR_127 | 40 RS-SINR | dB |

#### 6.7.7.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy

6.7.7.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RS-SINR absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.7.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA and rs-SINR-MeasEUTRA.

6.7.7.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.7.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.7.1.

6.7.7.1.1.4	Test description

6.7.7.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.7.1.4.1-1.

Table 6.7.7.1.4.1-1: test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.7.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.7.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.7.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.7.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.7.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.7.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.7.1.4.1-2.

Table 6.7.7.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.7.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.7.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.7.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.7.1.5-1 and Table 6.7.7.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RS-SINR reported values in the periodic MeasurementReport. The RS-SINR value of Cell 2 reported by the UE is compared to the expected RS-SINR. If the value is outside the limits in Table 6.7.7.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.7.1.5-1 as appropriate and repeat steps 5-7.

6.7.7.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.7.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-RAT Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.7.1.4.3-2: ReportConfigE-UTRA-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-141 with Condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.7.1.5	Test requirement

Table 6.7.7.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.7.1.5-1 and 6.7.7.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.7.1.5-3.

Table 6.7.7.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.1.2 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.7.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| Noc1Note4 | Depending on band group | dBm/15kHz | -88 | -108.50 | -119.5+ ΔBG_offset |
| Noc2Note4a | Depending on band group | dBm/15kHz | -82 | -114.5 | -113.5+ ΔBG_offset |
| CRS Ês/Noc1 |  | dB | -1.75 | 20.0 | -3.2 |
| CRS Ês/IotNote5 |  | dB | -1.75 | 20.0 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -89.75 | -88.50 | -122.7+ ΔBG_offset |
| RS-SINRNote5 |  | dB | -1.75 | 20 | -3.2 |
| IoNote5 | Depending on band group | dBm/Ch BW | -58.00 + 10log(NRB,c /50) | -60.68 + 10log(NRB,c /50) | -90.02+ ΔBG_offset + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over CRS subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 4a: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers other than CRS subcarriers and time and shall be modelled as AWGN of appropriate power for Noc2 to be fulfilled. Note 5: CRS Ês/Iot, RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: For Band 26, the tests shall be performed with the carrier frequency of assigned E-UTRA channel bandwidth within 865-894 MHz. Note 8: Except Band 29. Note 9: Except Band 32, Band 75 and Band 76. Note 10: For Band 74, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 1475.9-1510.9 MHz. |  |  |  |  |  |

Table 6.7.7.1.5-3: RS-SINR Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 35 | 79 | 32 |
| Highest reported value (Cell 2) | 51 | 94 | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest reported value (Cell 2) | 33 | 77 | 31 |
| Highest reported value (Cell 2) | 53 | 96 | 50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

# 7	NR standalone with at least one NR cell in FR2

This section contains test scenarios for NR standalone. This configuration is also known as SA Option 2. At least one NR cell is in Frequency Range 2.

For conformance testing involving FR2 test cases in this specification, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

## 7.1	RRC_IDLE state mobility

### 7.1.1	NR cell re-selection

#### 7.1.1.0	Minimum conformance requirements

##### 7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection

The cell re-selection delay shall be less than Tevaluate NR_Intra + TSI-NR in RRC_IDLE state.

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [30] within Tdetect,NR_Intra as defined in table 4.2.2.3-1 of TS 38.133 [6] when that Treselection= 0. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 of TS 38.133 [6] for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1 of TS 38.133 [6]) for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1 of TS 38.133 [6] provided that the cell has at least [3]dB better ranked.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a non-zero value and non-zeroa-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [30], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.2 and 4.2.2.3.

##### 7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection

The cell re-selection delay shall be less than Tevaluate NR_Intra + TSI-NR in RRC_IDLE state.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 of TS 38.133 [6].

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this subclause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [30] within Kcarrier * Tdetect,NR_Inter  if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least [5] dB for reselections based on ranking or [6]dB for SS-RSRP reselections based on absolute priorities or [4]dB for SS-RSRQ reselections based on absolute priorities. The parameter Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell. An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.3 of TS 38.133 [6] for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier * Tmeasure,NR_Inter (see table 4.2.2.4-1 of TS 38.133 [6]) for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [30] within Kcarrier * Tevaluate,NR_Inter when Treselection = 0 as specified in table 4.2.2.4-1 of TS 38.133 [6] provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and the cell has at least [5]dB better ranked

-	[6]dB for SS-RSRP reselections based on absolute priorities or

-	[4]dB for SS-RSRQ reselections based on absolute priorities.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non-zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in Table 4.2.2.4-1 of TS 38.133 [6] under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where TSMTC_intra and TSMTC_inter are periodicities of the SMTC occasions configured for the intra-frequency carrier and the inter-frequency carrier respectively,

-	SMTC occasions configured for the inter-frequency carrier occur up to TBD ms before the start or up to TBD ms after the end of the SMTC occasions configured for the intra-frequency carrier and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to TBD ms before the start or up to TBD ms after the end of the paging occasion [1].

The normative reference for this requirement is TS 38.133 [6] clause 4.2.2.4.

#### 7.1.1.1	NR SA FR2 cell re-selection

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- TT analysis is missing.

- Connection diagram is TBD.

7.1.1.1.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection requirements specified in TS 38.133 clause 4.2.2.3.

7.1.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.1.

7.1.1.1.4	Test description

7.1.1.1.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.1.4.1-1.

Table 7.1.1.1.4.1-1: Supported test configurations for NR SA FR2 cell re-selection

| Configuration | Description |
| --- | --- |
| 7.1.1.1-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.1-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.1.4.1-2.

Table 7.1.1.1.4.1-2: Initial conditions for NR SA FR2 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.1.4.1-3.

2. Message contents are defined in clause 7.1.1.1.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.1.1.1.4.1-3: General test parameters for NR SA FR2 cell re-selection

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Visited cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 135 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2 | 35 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

7.1.1.1.4.2	Test procedure

Two cells are deployed in the test, which are one FR2 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Set Cell 2 physical cell identity = initial cell 2 physical cell identity.

2.	Set the parameters according to T1 in Table 7.1.1.1.5-1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.1.5-1.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 130 seconds from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 8.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, skip to step 12.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 7.1.1.1.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 27 seconds from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected Cell 1 within T3, after the re-selection or when T3 expires, skip to step 13.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 1, continue with step 12.

12.	Switch off and on the UE and ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 in Cell 1.

13.	Repeat step 2-12 until a test verdict has been achieved.
Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

7.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-1 with Condition SMTC.1 and Synchronous cells Table H.2.1-2 Table H.2.1-3 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.1.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

7.1.1.1.5	Test requirement

Tables 7.1.1.1.4.1-3 and 7.1.1.1.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case.

Table 7.1.1.1.5-1: Cell specific test parameters for NR SA FR2 cell re-selection

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
|  |  | 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
|  |  | 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
|  |  | 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  |  | OP.4 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.9.1 |  |  | Setup 1 defined in A.3.9.1 |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -96 | -91.5 | -infinity | -91.5 | -96 |
|  |  | 2 | -82 | -93 | -88.5 | -infinity | -88.5 | -93 |
| Io | dBm/95.04 MHz | 1 | -59.37 | -63.40 | -62.47 | -64.01 | -62.47 | -63.40 |
|  |  | 2 | -57.18 | -62.86 | -61.67 | -64.01 | -61.67 | -62.86 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The cell re-selection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on Cell 2.

The cell re-selection delay to a newly detectable cell test requirement in this case is expressed as:

Cell re-selection delay to a newly detectable cell = Tdetect,NR_Intra + TSI-NR

Tdetect,NR_Intra = 128 s; as specified in TS 38.133 [6] clause 4.2.2.3.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a newly detectable cell shall be less than a total of 129.28 seconds in this test case (note: this gives a total of 129.28 seconds but the test allows 130 seconds).

The cell re-selection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on Cell 1.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 25.6 s; as specified in TS 38.133 [6] clause 4.2.2.3.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to an already detected cell shall be less than a total of 26.88 seconds in this test case (note: this gives a total of 26.88 seconds but the test allows 27 seconds).

For the test to pass, both events above shall pass.

The statistical pass/ fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

#### 7.1.1.2	NR SA FR2-FR2 cell re-selection

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- TT analysis is missing.

- Connect diagram is TBD

7.1.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection requirements specified in TS 38.133 clause 4.2.2.4.

7.1.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.2.

7.1.1.2.4	Test description

7.1.1.2.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.2.4.1-1.

Table 7.1.1.2.4.1-1: Supported test configurations for NR SA FR2-FR2 cell re-selection

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 7.1.1.2-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.2-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.2.4.1-2.

Table 7.1.1.2.4.1-2: Initial conditions for NR SA FR2-FR2 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.2.4.1-3.

2. Message contents are defined in clause 7.1.1.2.4.3.

3. There is two NR carrier and 2 NR Cells specified in the test. Cell 2 is the PCell and Cell 1 is the neighbour cell in a different carrier than cell 2. Cell 1 and Cell 2 are configured according to Annex C.1.2.

Table 7.1.1.2.4.1-3: General test parameters for NR SA FR2-FR2 cell re-selection

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
| T1 end condition | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 35 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 95 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

7.1.1.2.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.1.1.2.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, Cell 1.

5.	If the UE responds on lower priority cell, Cell 1 during time duration T1 within 27 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”.

6.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 7.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 12.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.2.5-1. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE.

8.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.1.1.2.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, Cell 2.

10.	If the UE responds on higher priority cell, Cell 2 during time duration T3 within 87 seconds from the beginning of time period T3, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”.

11.	If the UE has re-selected Cell 2 within T3, after the re-selection or when T3 expires, skip to step 13.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 2, continue with step 12.

12.	Switch off and on the UE and ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 in Cell 2.

13.	Repeat step 3-12 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

7.1.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions |  |
| Specific message contents exceptions for Test Configuration 7.1.1.2-1 | Table H.2.2-2 with Condition SSB.1 FR2 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 7.1.1.2-2 | Table H.2.2-2 with Condition SSB.2 FR2 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.1.1.2.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

7.1.1.2.5	Test requirement

Tables 7.1.1.2.4.1-3 and 7.1.1.2.5-1 define the primary level settings including test tolerances for inter frequency NR cell re-selection test case.

Table 7.1.1.2.5-1: Cell specific test parameters for NR SA FR2-FR2 cell re-selection

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.9.1 |  |  | Setup 1 defined in A.3.9.1 |  |  |
|  | dB | 1 | 8 | 8 | 8 | -3 | -infinity | 8 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | 8 | 8 | -3 | -infinity | 8 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -85 | -85 | -96 | -infinity | -85 |
|  |  | 2 | -82 | -82 | -82 | -93 | -infinity | -82 |
| Io | dBm/95.04 MHz | 1 | -55.37 | -55.37 | -55.37 | -62.25 | -infinity | -55.37 |
|  |  | 2 | -52.37 | -52.37 | -52.37 | -59.25 | -infinity | -52.37 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2 | 50 |  |  | Not sent |  |  |
| Threshx, highP | dB | 1, 2 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on cell 2.

The cell re-selection delay to a higher priority cell test requirement in this case is expressed as:

Cell re-selection delay to a higher priority cell = Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR

Thigher_priority_search = 60 s, as specified in TS 38.133 [6] clause 4.2.2.7;

Tevaluate, NR_ inter = 25.6 s, as specified in TS 38.133 [6] clause 4.2.2.4;

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a higher priority cell shall be less than a total of 86.88 seconds in this test case (note: this gives a total of 86.88 seconds but this test allows 87 seconds)

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on cell 1.

The cell re-selection delay to a lower priority cell test requirement in this case is expressed as:

Cell re-selection delay to a lower priority cell = Tevaluate, NR_ inter + TSI-NR

Tevaluate, NR_ inter = 25.6 s, as specified in TS 38.133 [6] clause 4.2.2.4;

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a lower priority cell shall be less than a total of 26.88 seconds in this test case (note: this gives a total of 26.88 seconds but this test allows 27 seconds)

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

## 7.2	RRC_INACTIVE state mobility

## 7.3	RRC_CONNECTED state mobility

### 7.3.1	Handover

### 7.3.2	RRC connection mobility control

#### 7.3.2.1	RRC re-establishment

##### 7.3.2.1.0	Minimum conformance requirements

[TS 38.133, clause 6.2.1.2]

In RRC_CONNECTED statethe UE shall be capable of sending RRCReestablishmentRequest message within Tre-establish_delay seconds from the moment it detects a loss in RRC connection. The total RRC connection delay (Tre-establish_delay) shall be less than:

Tre-establish_delay = TUE_re-establish_delay + TUL_grant

TUL_grant: It is the time required to acquire and process uplink grant from the target PCell. The uplink grant is required to transmit RRCReestablishmentRequest message.

The UE re-establishment delay (TUE_re-establish_delay) is the time between the moments when any of the conditions requiring RRC re-establishment as defined in clause 5.3.7 in TS 38.331 [2] is detected by the UE and when the UE sends PRACH to the target PCell. The UE re-establishment delay (TUE_re-establish_delay) requirement shall be less than:

TUE_re-establish_delay = 50ms + Tidentify_intra_NR +i=1Nfreq-1 i=1Nfreq-1Tidentify_inter_NR,i + TSI-NR + TPRACH

The intra-frequency target NR cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in Section 10.1.2 and 10.1.3 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding NR Band.

The inter-frequency target NR cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in Section 10.1.4 and 10.1.5 are fulfilled for a corresponding NR Band for FR1 and FR2, respectively,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding NR Band.

Tidentify_intra_NR: It is the time to identify the target intra-frequency NR cell and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. If the UE is not configured with intra-frequency NR carrier for RRC re-establishment then Tidentify_intra_NR=0; otherwise Tidentify_intra_NR shall not exceed the values defined in table 6.2.1.2.1-1.

Tidentify_inter_NR,i: It is the time to identify the target inter-frequency NR cell on inter-frequency carrier i configured for RRC re-establishment and it depends on whether the target NR cell is known cell or unknown cell and on the frequency range (FR) of the target NR cell. Tidentify_inter_NR,i shall not exceed the values defined in table 6.2.1.2.1-2.

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

TSI-NR = It is the time required for receiving all the relevant system information according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for the target NR cell.

TPRACH = It is the delay uncertainty in acquiring the first available PRACH occasion in the target NR cell. TPRACH can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Nfreq: It is the total number of NR frequencies to be monitored for RRC re-establishment; Nfreq = 1 if the target intra-frequency NR cell is known, else Nfreq = 2 and Tidentify_intra_NR = 0 if the target inter-frequency NR cell is known.

There is no requirement if the target cell does not contain the UE context.

In the requirement defined in the below tables, the target FR1 cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown.

Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | MAX (800 ms, 10 x TSMTC) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 80 x TSMTC)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 3520Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < [-8] dB. |  |  |  |

Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | Frequency range (FR) of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | MAX (800 ms, 13 x TSMTC, i) |
| ≥ -8 | FR2 | N/A | MAX (1000 ms, 104 x TSMTC, i)) |
| < -8 | FR1 | N/A | 800Note1 |
| < -8 | FR2 | N/A | 4000Note1 |
| Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 6.2.1.

##### 7.3.2.1.1	NR SA FR2 RRC re-establishment

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test tolerances and test system uncertainties applicable to this test are undefined.

-	Message content is TBD

7.3.2.1.1.1	Test purpose

The purpose of this test is to verify that the NR intra-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits.

7.3.2.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.3.2.1.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.1.1

7.3.2.1.1.4	Test description

7.3.2.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.3.2.1.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 7.3.2.1.1.4.1-2. Test environment parameters are given in Table 7.3.2.1.1.4.1-3.

Table 7.3.2.1.1.4.1-1: Intra-frequency RRC re-establishment in FR2 supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 7.3.2.1.1.4.1-2: General test parameters for Intra-frequency RRC re-establishment in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
|  |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | PRACH.1 FR2 | As specified in Annex A.7 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | ms | 1 | 1600 | Time for the UE to detect RLF |
| T3 |  | s | 1 | 3 |  |

Table 7.3.2.1.1.4.1-3: Test Environment Intra-frequency RRC re-establishment in FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5.1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2 |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1. Message contents are defined in clause 7.3.2.1.1.4.3.

2. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.1.1 and C.1.2 for this test.

3. The AoA setup for this test is Setup 1 as defined in clause A.9

7.3.2.1.1.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.3.2.1.1.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.2.1.1.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.3.2.1.1.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within [6] s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

7.3.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause FFS with the following exceptions:

Table 7.3.2.1.1.4.3-1: Common Exception messages for NR intra-frequency RRC re-establishment test case in FR2

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | FFS |
| Default RRC messages and information elements contents exceptions | FFS |

7.3.2.1.1.5	Test requirement

Table 7.3.2.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.2.1.1.5-1 : Cell specific test parameters for NR intra-frequency RRC re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
|  |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 |  |  | Setup 1 |  |  |
|  | dB | 1 | -3.07+TT | -infinity | -infinity | -5.07+TT | 2+TT | 2+TT |
| Note2 | dBm/SCS | 1 | -98+TT |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -89+TT |  |  |  |  |  |
|  | dB | 1 | 4+TT | -infinity | -infinity | 2+TT | 2+TT | 2+TT |
| SS-RSRP Note3 | dBm/SCS | 1 | -85+TT | -infinity | -infinity | -87+TT | -87+TT | -87+TT |
| Io | dBm/95.04 MHz | 1 | -52.94+TT | -55.89 | -55.89 | -52.94+TT | -55.89+TT | -55.89+TT |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 in Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell shall be less than 3 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90% with a confidence level of 95%.

##### 7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test tolerances and test system uncertainties applicable to this test are undefined.

-	Message content is TBD

7.3.2.1.2.1	Test purpose

The purpose of this test is to verify that the NR inter-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits.

7.3.2.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.3.2.1.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.1.2

7.3.2.1.2.4	Test description

7.3.2.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.3.2.1.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 7.3.2.1.2.4.1-2. Test environment parameters are given in Table 7.3.2.1.2.4.1-3.

Table 7.3.2.1.2.4.1-1: Inter-frequency RRC re-establishment in FR2 supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 7.3.2.1.2.4.1-2: General test parameters for Inter-frequency RRC re-establishment in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
|  |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC.1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | PRACH.1 FR2 | As specified in Annex A.7 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | ms | 1 | 1600 | Time for the UE to detect RLF |
| T3 |  | s | 1 | 6 |  |

Table 7.3.2.1.2.4.1-3: Test Environment Inter-frequency RRC re-establishment in FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table FFS and TS 38.508-1 [14] clause 4.3.1 and 4.4.2 |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C2.2 |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1. Message contents are defined in clause 7.3.2.1.2.4.3

2. There are two cells on two NR carriers specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.1.1 and C.1.2 for this test.

3. The AoA setup for this test is Setup 1 as defined in clause A.9

7.3.2.1.2.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.3.2.1.2.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.2.1.2.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.3.2.1.2.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within [6] seconds from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

7.3.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause FFS with the following exceptions:

Table 7.3.2.1.2.4.3-1: Common Exception messages for NR intra-frequency RRC re-establishment test case in FR2

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | FFS |
| Default RRC messages and information elements contents exceptions | FFS |

7.3.2.1.2.5	Test requirement

Table 7.3.2.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.3.2.1.2.5-1: Cell specific test parameters for NR intra-frequency RRC re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| AoA setup |  | 1 | Setup 3 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 |  |  | OP.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
|  | dB | 1 | 5+TT | -infinity | -infinity | -infinity | -infinity | 8+TT |
| Note2 | dBm/SCS | 1 | -98+TT |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -89+TT |  |  |  |  |  |
|  | dB | 1 | 5+TT | -infinity | -infinity | -infinity | -infinity | 8+TT |
| SS-RSRP Note3 | dBm/SCS | 1 | -84+TT | -infinity | -infinity | -infinity | -infinity | -81+TT |
| Io | dBm/95.04 MHz | 1 | -53.82+TT | -infinity | -infinity | -infinity | -infinity | -51.37+TT |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than 6 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90% with a confidence level of 95%.

##### 7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test tolerances and test system uncertainties applicable to this test are undefined.

-	Antenna diagram is TBD

-	Message content is TBD

7.3.2.1.3.1	Test purpose

The purpose of this test is to verify that the NR intra-frequency RRC re-establishment delay in FR2 without serving cell timing is within the specified limits, and to verify the requirements in TS 38.133 [6] clause 6.2.1

7.3.2.1.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.3.2.1.3.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.1.0

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.1.3

7.3.2.1.3.4	Test description

7.3.2.1.3.4.1	Initial conditions

The test shall be tested using any of the test configuration in Table 7.3.2.1.3.4.1-1.

Table 7.3.2.1.3.4.1-1: Supported test configurations for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Configure the test requirement and the DUT according to the parameters in Table 7.3.2.1.3.4.1-2.

Table 7.3.2.1.3.4.1-2: Initial conditions for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5.1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2 |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.1.3.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram |  |  |  |

1.	The general test parameter settings are set up according to Table 7.3.2.1.3.4.1-3.

2.	Message contents are defined in clause 7.3.2.1.3.4.3.

There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for registration with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.3.2.1.3.4.1-3: General test parameters for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell1 |  |
|  | Neighbour cells |  | 1 | Cell2 |  |
| Final condition | Active cell |  | 1 | Cell2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
|  |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 6000 | Radio link failure timer configured by RLF-TimersAndConstants |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration index |  |  | 1 | PRACH.1 FR2 |  |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 6 | Time for the UE to detect RLF |
| T3 |  | s | 1 | 6 |  |

7.3.2.1.3.4.2	Test procedure

The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.3.2.1.3.5-1. T1 starts.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.3.2.1.3.5-1. T2 starts

6.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.3.2.1.3.5-1. T3 starts

7.	If the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2 within 5 s from the beginning of time period T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

8.	After T3 expires, cause UE handover back to Cell 1 (if the handover fails, switch off the UE) or switch off the UE. Then ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is the active cell.

9.	Repeat step 2-8 until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved.

7.3.2.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.3.2.1.3.4.3-1: Common Exception messages for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | FFS |
| Default RRC messages and information elements contents exceptions | FFS |

7.3.2.1.3.5	Test requirement

Table 7.3.2.1.3.5-1 defines the primary level settings including test tolerances for NR SA FR2 - FR2 RRC re-establishment without serving cell timing tests.

Table 7.3.2.1.3.5-1 : Cell specific test parameters for NR SA FR2 - FR2 RRC re-establishment without serving cell timing

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
|  |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 FDD |  |  | CR.3.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 FDD |  |  | CCR.3.1 FDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| TCI state |  | 1 | CSI-RS.Config.0 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.2.1 |  |  | OP.1 defined in A.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 defined in A. 9 |  |  | Setup 1 defined in A.9 |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -89 |  |  |  |  |  |
|  | dB | 1 | 5 | -infinity | -infinity | -infinity | -infinity | 5 |
| SS-RSRP Note3 | dBm/SCS | 1 | -93 | -infinity | -infinity | -infinity | -infinity | -93 |
| Io | dBm/95.04 MHz | 1 | -62.82 | -infinity | -infinity | -infinity | -infinity | -62.82 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell without serving cell timing shall be less than 5 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1

Tidentify_intra_NR = 3520 ms

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of 4865 ms, allow 5 s in the test case.

#### 7.3.2.2	Random access

##### 7.3.2.2.0	Minimum conformance requirements

7.3.2.2.0.1	Minimum conformance requirements for Contention based random access

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-2 [3] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-2 [3] Tables 6.3.4.3-1 and 6.3.4.3-2.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to select a Random Access Preamble randomly with equal probability from the Random Access Preambles associated with the selected SSB if the association between Random Access Preambles and SS blocks is configured, as specified in clause 5.1.2 in TS 38.321 [12].

With the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB, UE shall have the capability to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, if the association between PRACH occasions and SSBs is configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [12].

The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

The UE shall send ACK if the Contention Resolution is successful.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires unless the received message includes a UE Contention Resolution Identity MAC control element and the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU transmitted in the uplink message.

The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the backoff time expires if the Contention Resolution Timer expires.

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2.

7.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access

The random access procedure is used when establishing the layer 1 communication between the UE and NG-RAN. The random access is as defined in TS 38.213 [8] clause 7.4 and the control of the RACH transmission is as defined in TS 38.321 [12] clause 5.1.

The UE shall have capability to calculate PRACH transmission power according to the PRACH power formula as defined in TS 38.213 [8] clause 7.4 and apply this power level at the first preamble or additional preambles. The absolute power applied to the first preamble shall have an accuracy as defined in TS 38.101-2 [3] Table 6.3.4.2-1. The relative power applied to additional preambles shall have an accuracy as specified in TS 38.101-2 [3] Tables 6.3.4.3-1 and 6.3.4.3-2.

The UE shall indicate a Random Access problem to upper layers if the maximum number of preamble transmission counter has been reached for the random access procedure on PCell or PSCell as specified in TS 38.321 [12] clause 5.1.4.

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with SSBs is configured, with the UE selected SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs, UE shall have the capability to select the Random Access Preamble corresponding to the selected SSB, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, and PRACH occasion shall be randomly selected with equal probability amongst the selected SSB associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

If the contention-free Random Access Resources and the contention-free PRACH occasions associated with CSI-RSs is configured, with the UE selected CSI-RS with CSI-RSRP above cfra-csirs-DedicatedRACH-Threshold amongst the associated CSI-RSs, UE shall have the capability to select the Random Access Preamble corresponding to the selected CSI-RS, and to transmit Random Access Preamble on the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS, and PRACH occasion shall be randomly selected with equal probability amongst the selected CSI-RS associated PRACH occasions occurring simultaneously but on different subcarriers, as specified in clause 5.1.2 in TS 38.321 [12].

The UE may stop monitoring for Random Access Response(s), if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble, unless the random access procedure is initialized for Other SI request from UE.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [12] for the next available PRACH occasion, and transmit the preamble with the calculated PRACH transmission power, if no Random Access Response is received within the RA Response window configured in RACH-ConfigCommon or if no PDCCH addressed to UE’s C-RNTI is received within the RA Response window configured in BeamFailureRecoveryConfig, as defined in clause 5.1.4 in TS 38.321 [12].

The normative reference for this requirement is TS 38.133 [6] clause 6.2.2.

Non-contention based random access procedure is not initialized for Other SI requested from UE or for beam failure recovery, so the requirements related to those features are omitted.

##### 7.3.2.2.1	NR SA FR2 contention based random access

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	The results of the TT analysis are provisional until the corresponding MU values are agreed

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

7.3.2.2.1.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

7.3.2.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.3.2.2.1.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.2.1.

7.3.2.2.1.4	Test description

7.3.2.2.1.4.1	Initial conditions

This test can be run in the configurations defined in Table 7.3.2.2.1.4.1-1.

Table 7.3.2.2.1.4.1-1: Contention based random access test in FR2 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 7.3.2.2.1-1 | 1 | NR: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 7.3.2.2.1.4.1-2.

Table 7.3.2.2.1.4.1-2: Test Environment for Contention based random access test in FR2 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram | FFS |  |  |

1. Message contents are defined in clause 7.3.2.2.1.4.3.

2. Cell 1 is the NR FR2 serving cell (PCell). The connection setup is done according to the settings in FFS.

7.3.2.2.1.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR2. The System Simulator shall not explicitly assign a random access preamble via dedicated signalling in the downlink.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Tables 7.3.2.2.1.5-1 and 7.3.2.2.1.5-2.

3.	The UE shall establish a connection setup with SS, the random access procedure within the connection setup is used in the test.

4.	Test 1: Correct behaviour when transmitting Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB.

5.	Test 2: Correct behaviour when receiving Random Access Response

5.1. Repeat steps 1-3.

5.2. The UE shall send preambles to the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

5.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

5.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

5.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

5.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.1.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.1.5.

6.	Test 3: Correct behaviour when not receiving Random Access Response

6.1. Repeat steps 1-3.

6.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 2 preambles.

6.3. As no Random Access Response was received within the RA Response window, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.1.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.1.5.

7.	Test 4: Correct behaviour when receiving an UL grant for msg3 retransmission

7.1. Repeat steps 1-3.

7.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

7.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

7.4. The System Simulator shall send PDCCH addressed to the Temporary C-RNTI after receiving the msg3.

7.5. The UE shall re-transmit the msg3.

7.6. The System Simulator shall check if UE re-transmit the msg3.

8.	Test 5: Correct behaviour when receiving an unsuccessful UE Contention Resolution

8.1. Repeat steps 1-3.

8.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

8.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

8.4. The System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element not matching the CCCH SDU transmitted in msg3 uplink message.

8.5. As the UE Contention Resolution Identity included in the MAC control element did not match the CCCH SDU transmitted in the uplink message, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the backoff time expires.

8.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.1.5.

9.	Test 6: Correct behaviour when receiving a successful UE Contention Resolution

9.1. Repeat steps 1-3.

9.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

9.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

9.4. The System Simulator shall send a message addressed to the temporary C-RNTI with a UE Contention Resolution Identity included in the MAC control element matching the CCCH SDU transmitted in msg3 uplink message.

9.5. As the UE Contention Resolution Identity included in the MAC control element matches the CCCH SDU, the Contention Resolution is successful and the UE shall send ACK.

10.	Test 7: Correct behaviour when contention Resolution timer expires

10.1. Repeat steps 1-3.

10.2. The UE shall send a preamble to the System Simulator. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble.

10.3. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE shall transmit the msg3.

10.4. The System Simulator shall not send a response.

10.5. As there was no response, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power when the Contention Resolution Timer expires and then after the backoff timer expires.

10.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.1.5.

7.3.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.2.2.1.4.3-1: FrequencyInfoUL-SIB for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 7.3.2.2.1.4.3-2: RACH-ConfigCommon for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| oneFourth | n48 |  | FR2 |
| } |  |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | sf48 |  |  |
| rsrp-ThresholdSSB | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process |  |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 120 |  |  |
| } |  |  |  |

Table 7.3.2.2.1.4.3-3: RACH-ConfigGeneric for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  | FR2 |
| msg1-FDM | one |  | FR2 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -100 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 7.3.2.2.1.4.3-4: ServingCellConfigCommonSIB for Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | 20 +ΔUL | ΔUL is derived from the uplink calibration process |  |
| } |  |  |  |

7.3.2.2.1.5	Test requirement

Table 7.3.2.2.1.5-2 defines the primary level settings for contention based random access test in FR2 for NR Standalone. Tables 7.3.2.2.1.5-3, 7.3.2.2.1.5-4 and 7.3.2.2.1.5-5 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 7.3.2.2.1.5-1: General test parameters for contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | As defined in A.3.2 |
| Duplex Mode for Cell 1 | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | As defined in A.2.1 |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR.3.1 TDD | As defined in A.1.1 |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | As defined in A.1.2 |
| NR RF Channel Number |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | As defined in clause 6.2.4 of TS 38.101-2 [3] |
| PRACH Configuration |  |  | PRACH.1 FR2 | As defined in A.7.2, with exceptions as defined below |
| rsrp-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | As defined in TS 38.331 [13] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |

Table 7.3.2.2.1.5-2: OTA-related test parameters for contention based random access test in FR2 for NR Standalone

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  |  | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  |  | Rough |  |
| SSB with index 0 | Es Note1 |  | dBm/SCS | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP |  | dBm/SCS | -80.6 |  |
|  | Es/IotBB |  | dB | 21.09 |  |
|  | Io |  | dBm/95.04 MHz | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 |  | Es Note1 | dBm/SCS | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  |  | SSB_RP | dBm/SCS | -95.0 |  |
|  |  | Es/IotBB | dB | 6.69 |  |
|  |  | Io | dBm/95.04 MHz | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Test 1: Correct behaviour when transmitting Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

Test 2: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.1.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.1.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.1.5-5.

Test 3: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.1.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.1.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.1.5-5.

Test 4: Correct behaviour when receiving an UL grant for msg3 retransmission

-	The UE shall re-transmit the msg3 upon the reception of an UL grant for msg3 retransmission.

Test 5: Correct behaviour when receiving an incorrect message over Temporary C-RNTI

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires.

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.1.5-3.

-	The transmit timing of the PRACH transmission shall be within the accuracy specified in Table 7.3.2.2.1.5-5.

Test 6: Correct behaviour when receiving a correct message over Temporary C-RNTI

-	The UE shall send ACK if the contention resolution is successful.

Test 7: Correct behaviour when contention resolution timer expires

-	The UE shall re-select a preamble and transmit with the calculated PRACH transmission power when the back off time expires if the contention resolution timer expires.

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.1.5-3.

-	The transmit timing of the PRACH transmission shall be within the accuracy specified in Table 7.3.2.2.1.5-5.

Table 7.3.2.2.1.5-3 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± FFS dB |

Table 7.3.2.2.1.5-4 Relative power tolerance Test requirements

| Measured power | Power step P (dB) | PRACH (dB) |
| --- | --- | --- |
| Measured power of both PRACHs being compared > (PmaxNote1 – 6dB) | 2 ≤ ΔP < 3 | ± (4+FFS). |
| Measured power of either PRACHs being compared ≤ (PmaxNote1 – 6dB) |  | ± (6+FFS) |
| Note 1: Pmax is the Min peak EIRP defined in clause 6.2.1 of TS 38.101-2 [3], selected according to power class and operating band. |  |  |

Table 7.3.2.2.1.5-5: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 2 | 120 | 120 | 224+[48]*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

##### 7.3.2.2.2	NR SA FR2 non-contention based random access

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The settable window for first preamble uplink power and the uplink calibration process are FFS.

-	The test requirement for absolute uplink power is FFS.

-	The test requirement for relative uplink power is FFS.

-	The uncertainty value and test requirement for PRACH timing are in [ ]

-	The results of the TT analysis are provisional until the corresponding MU values are agreed

-	Antenna diagram and any exceptions are FFS

-	Connection setup in Annex C is FFS

7.3.2.2.2.1	Test purpose

The purpose of this test is to verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits.

7.3.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards. Additionally Test 2 is applicable to UE that supports CSI-RS based Random Access Preamble.

7.3.2.2.2.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.3.2.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.3.2.2.2.

7.3.2.2.2.4	Test description

7.3.2.2.2.4.1	Initial conditions

This test can be run in the configurations defined in Table 7.3.2.2.2.4.1-1.

Table 7.3.2.2.2.4.1-1: Non-Contention based random access test in FR2 for NR standalone supported test configurations

| Test Case ID | Test Config Index | Description |
| --- | --- | --- |
| 7.3.2.2.2-1 | 1 | NR: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

Before the test procedure, Downlink and Uplink calibrations are carried out to derive signalled parameter values. This is necessary to ensure that the test case operates within the specified ranges. The detail of the calibration process is implementation dependent, but shall derive the values of ΔDL and ΔUL according to the following principles:

With the UE configured to report SS-RSRP, the ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value according to TS 38.133 [6] Table 10.1.6.1-1 with -80.6dBm/SCS applied at the Reference point. For a Reported value RSRP_x, x is treated as a positive integer value.

With the UE configured to send a first PRACH preamble, ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied at the Reference point, and with signalled values preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm.

Configure the test equipment and the DUT according to the parameters in Table 7.3.2.2.2.4.1-2.

Table 7.3.2.2.2.4.1-2: Test Environment for Non-Contention based random access test in FR2 for NR standalone

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] subclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.3.2.2.2.4.1-1. |  |  |
| Propagation conditions | No interference |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | FFS | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | FFS |  |
| Exceptions to connection diagram | FFS |  |  |

1. Message contents are defined in clause 7.3.2.2.2.4.3.

2. Cell 1 is the NR FR2 serving cell (PCell). The connection setup is done according to the settings in FFS.

7.3.2.2.2.4.2	Test procedure

The test consists of a single cell, configured as PCell in FR2. The System Simulator shall explicitly assign a random access preamble via dedicated signalling in the downlink. There are two subtests, to test both SSB-based non-contention based random access (subtest 1) and CSI-RS-based non-contention based random access (subtest 2).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.3.2.2.2.5-1 Subtest 1.

3.	The SS shall signal a Random Access Preamble ID via a PDCCH order to the UE and initiate a Non-contention based Random Access procedure.

4.	Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the SSB with index 0, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

5.	Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

5.1. Set the parameters according to Table 7.3.2.2.2.5-1 Subtest 2.

5.2. Repeat steps 1-3

5.3. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble has the Preamble Index associated with the CSI-RS configured, that it arrives on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured, and that the selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

6.	Test 3: Correct behaviour when receiving Random Access Response

6.1. Repeat steps 1-3

6.2. The UE shall send preambles to the System Simulator. In response to the first 2 preambles, the System Simulator shall transmit a Random Access Response containing Random Access Preamble identifiers that do not match the transmitted Random Access Preamble.

6.3. As the received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

6.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

6.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

6.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.2.5.

7.	Test 4: Correct behaviour when not receiving Random Access Response

7.1. Repeat steps 1-3.

7.2. The UE shall send preambles to the System Simulator. The System Simulator shall not respond to the first 2 preambles.

7.3. As no Random Access Response was received within the RA Response window configured in RACH-ConfigCommon, the UE shall perform the Random Access Resource selection procedure specified in clause 5.1.2 in TS 38.321 [12], and transmit with the calculated PRACH transmission power.

7.4. The System Simulator shall transmit a Random Access Response containing a Random Access Preamble identifier matching the transmitted Random Access Preamble after 3 preambles have been received by the System Simulator.

7.5. As the received Random Access Response contains a Random Access Preamble identifier that matches the transmitted Random Access Preamble, the UE may stop monitoring for Random Access Response(s).

7.6. Measure the power and timing of the first preamble and it shall not exceed the values specified in 7.3.2.2.2.5. Measure the relative power and timing applied to additional preambles (last 2 preambles) and it shall not exceed the values specified in 7.3.2.2.2.5.

7.3.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.3.2.2.2.4.3-1: FrequencyInfoUL-SIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| p-Max | 23 | 23 dBm |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-2: RACH-ConfigCommon for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | 48 |  |  |
| groupBconfigured SEQUENCE { |  |  |  |
| numberOfRA-PreamblesGroupA | 48 |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer | Not present |  |  |
| rsrp-ThresholdSSB | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 1 |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| 0 |  |  |  |
| } |  |  |  |
| msg1-SubcarrierSpacing | kHz 120 |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-3: RACH-ConfigDedicated for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| } |  |  |  |
| resources CHOICE { |  |  |  |
| ssb SEQUENCE { |  |  |  |
| ssb-ResourceList SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF { | 2 entries |  |  |
| ssb[1] | 0 |  |  |
| ssb[2] | 1 |  |  |
| ra-PreambleIndex[1] | 50 |  | Subtest 1 |
| } |  |  |  |
| ra-ssb-OccasionMaskIndex | 1 |  | Subtest 1 |
| } |  |  |  |
| csirs SEQUENCE { |  |  |  |
| csirs-ResourceList SEQUENCE (SIZE(1..maxRA- CSIRS -Resources)) OF { |  |  |  |
| ra-OccasionList | 1 |  | Subtest 2 |
| ra-PreambleIndex[1] | 50 |  | Subtest 2 |
| } |  |  |  |
| rsrp-ThresholdCSI-RS | RSRP_69 +ΔDL | ΔDL is derived from the downlink calibration process | Subtest 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-4: RACH-ConfigGeneric for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  | FR2 |
| msg1-FDM | one |  | FR2 |
| zeroCorrelationZoneConfig | 11 |  |  |
| preambleReceivedTargetPower | -100 |  |  |
| preambleTransMax | n6 |  |  |
| powerRampingStep | dB2 |  |  |
| ra-ResponseWindow | sl10 |  |  |
| } |  |  |  |

Table 7.3.2.2.2.4.3-5: ServingCellConfigCommonSIB for Non-Contention Based Random Access

| Derivation Path: TS 38.508-1 [14], table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ‘1100 0000’B |  |  |
| } |  |  |  |
| ss-PBCH-BlockPower | 20 +ΔUL | ΔUL is derived from the uplink calibration process |  |
| } |  |  |  |

7.3.2.2.2.5	Test requirement

Table 7.3.2.2.2.5-2 defines the primary level settings for non-contention based random access test in FR2 for NR Standalone. Tables 7.3.2.2.2.5-3, 7.3.2.2.2.5-4 and 7.3.2.2.2.5-5 define the Absolute power limits, Relative power limits and uplink timing error limits respectively, and all include test tolerances.

Table 7.3.2.2.2.5-1: General test parameters for non-contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration | Config 1 |  | SSB.1 FR2 | SSB.1 FR2 | As defined in A.3.2 |
| CSI-RS Configuration | Config 1 |  | N/A | CSI-RS.3.1 TDD | As defined in A.1.4 |
| Duplex Mode for Cell 2 | Config 1 |  | TDD | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 | TDDConf.3.1 | As defined in A.1.5 |
| BWchannel | Config 1 | MHz | 100: NRB,c = 24 | 100: NRB,c = 24 |  |
| OCNG Pattern Note 1 |  |  | OP.3 | OP.3 | As defined in A.2.1. |
| PDSCH Reference Channel Note 2 | Config 1 |  | SR3.1 TDD | SR3.1 TDD | As defined in A.1.1. |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD | CR.3.1 TDD | As defined in A.1.2 |
| NR RF Channel Number |  |  | 1 | 1 |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  | dB |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  | dB |  |  |  |
| ss-PBCH-BlockPower |  | dBm/ SCS | +20 +ΔUL | +20 +ΔUL | As defined in TS 38.331 [13]. ΔUL is derived from the uplink calibration process Note 3 |
| Configured UE transmitted power () |  | dBm | maximum value configurable for certain power class | maximum value configurable for certain power class | As defined in clause 6.2.4 in TS 38.101-2 [3] |
| PRACH Configuration |  |  | PRACH.2 FR2 | PRACH.3 FR2 | As defined in A.7.2, with exceptions as defined below. |
| rsrp-ThresholdSSB |  | dBm | RSRP_69 +ΔDL | RSRP_69 +ΔDL | RSRP_69 corresponds to -88dBm. ΔDL is derived from the downlink calibration process Note 4 |
| preambleReceivedTargetPower |  | dBm | -100 | -100 | As defined in TS 38.331 [13] |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. Note 3: The ΔUL value is calculated as -ROUND(PPRACH0 -1), where PPRACH0 is the measured first PRACH power with -80.6dBm/SCS applied, preambleReceivedTargetPower = -100dBm and ss-PBCH-BlockPower = 20dBm. These values are used during the uplink calibration process carried out before the test case is run, with the UE configured to send PRACH. Note 4: The ΔDL value is calculated as (RSRP_REP – RSRP_76), where RSRP_REP is the SS-RSRP Reported value in Table 10.1.6.1-1 with -80.6dBm/SCS applied. These values are used during the downlink calibration process carried out before the test case is run, with the UE configured to report SS-RSRP. For a Reported value RSRP_x, x is treated as a positive integer value. |  |  |  |  |  |

Table 7.3.2.2.2.5-2: OTA-related test parameters for non-contention based random access test in FR2 for NR Standalone

| Parameter |  | Unit | Test-1 | Test-2 | Comments |
| --- | --- | --- | --- | --- | --- |
| AoA setup |  |  | Setup 1 | Setup 1 | As defined in A.9.1 |
| Assumption for UE beamsNote 2 |  |  | Rough | Rough |  |
| SSB with index 0 | Es Note1 | dBm/SCS | -80.6 | -80.6 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -80.6 | -80.6 |  |
|  | Es/IotBB | dB | 21.09 | 21.09 |  |
|  | Io | dBm/95.04 MHz | -56.01 | -56.01 | Io in symbols containing SSB index 0 |
| SSB with index 1 | Es Note1 | dBm/SCS | -95.0 | -95.0 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB |
|  | SSB_RP | dBm/SCS | -95.0 | -95.0 |  |
|  | Es/IotBB | dB | 6.69 | 6.69 |  |
|  | Io | dBm/95.04 MHz | -70.41 | -70.41 | Io in symbols containing SSB index 1 |
| Propagation Condition |  | - | AWGN | AWGN |  |
| Note 1: No artificial noise is applied in this test. Note 2: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |

Test 1: Correct behaviour when transmitting SSB-based Random Access Preamble

-	The Random Access Preamble shall be one of the Random Access Preambles associated with SSB index 0.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the SSB with index 0.

-	The selected PRACH occasion shall belong to the PRACH occasions permitted by the restrictions given by the ra-ssb-OccasionMaskIndex.

Test 2: Correct behaviour when transmitting CSI-RS-based Random Access Preamble

-	The Random Access Preamble shall have the Preamble Index associated with the CSI-RS configured.

-	The Random Access Preamble shall arrive on a PRACH occasion which belongs to the PRACH occasions corresponding to the CSI-RS configured.

-	The selected PRACH occasion belongs to the PRACH occasions permitted by the restrictions given by the ra-OccasionList.

Test 3: Correct behaviour when receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.2.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.2.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.2.5-5.

Test 4: Correct behaviour when not receiving Random Access Response

-	The power of the first preamble shall be 0.6 dBm within the accuracy specified in Table 7.3.2.2.2.5-3.

-	The relative power for preamble ramping step shall be 2 dB within the accuracy specified in Table 7.3.2.2.2.5-4.

-	The transmit timing of all PRACH transmissions shall be within the accuracy specified in Table 7.3.2.2.2.5-5.

Table 7.3.2.2.2.5-3 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± FFS dB |

Table 7.3.2.2.2.5-4 Relative power tolerance Test requirements

| Measured power | Power step P (dB) | PRACH (dB) |
| --- | --- | --- |
| Measured power of both PRACHs being compared > (PmaxNote1 – 6dB) | 2 ≤ ΔP < 3 | ± (4+FFS). |
| Measured power of either PRACHs being compared ≤ (PmaxNote1 – 6dB) |  | ± (6+FFS) |
| Note 1: Pmax is the Min peak EIRP defined in clause 6.2.1 of TS 38.101-2 [3], selected according to power class and operating band. |  |  |

Table 7.3.2.2.2.5-5: Te Timing error Test requirements

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals s(KHz) | Te |
| --- | --- | --- | --- |
| 2 | 120 | 120 | 224+[48]*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [7] |  |  |  |

#### 7.3.2.3	RRC connection release with redirection

## 7.4	Timing

### 7.4.1	UE transmit timing

### 7.4.2	UE timer accuracy

### 7.4.3	Timing advance

## 7.5	Signalling characteristics

### 7.5.1	Radio link monitoring

#### 7.5.1.0	Minimum conformance requirements

##### 7.5.1.0.1

##### 7.5.1.0.2

##### 7.5.1.0.3

##### 7.5.1.0.4

##### 7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring

[TS 38.133, clause 8.1.7.3]

The following scheduling restriction applies due to radio link monitoring on an FR2 serving PCell and/or PSCell.

-	If the RLM-RS is CSI-RS which is type-D QCLed with active TCI state for PDCCH or PDSCH, and the CSI-RS is not in a CSI-RS resource set with repetition ON,

-	There are no scheduling restrictions due to radio link monitoring based on the CSI-RS.

-	Otherwise

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring.

For FR2, if following conditions are met,

-	UE has been notified about system information update through paging,

-	The gap between UE’s reception of PDCCH that UE monitors in the Type2-PDCCH CSS set and that notifies system information update, and the PDCCH that UE monitors in the Type0-PDCCH CSS set, is greater than 2 slots,

For the SSB for RLM and CORESET for RMSI scheduling multiplexing patterns 3, UE is expected to receive the PDCCH that UE monitors in the Type0-PDCCH CSS set, and the corresponding PDSCH, on SSB symbols to be measured for RLM; and

For the SSB for RLM and CORESET for RMSI scheduling multiplexing patterns 2, UE is expected to receive PDSCH that corresponds to the PDCCH that UE monitors in the Type0-PDCCH CSS set, on SSB symbols to be measured for RLM.

#### 7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test tolerances and test system uncertainties applicable to this test are undefined.

-	Antenna diagram is TBD

-	Message content is TBD

7.5.1.9.1	Test purpose

The purpose of this test is to verify that the NR UE correctly follows the RLM scheduling restrictions requirements defined in TS 38.133 [6] clause 8.1.7, and to verify that the UE correctly receive the PDCCH scheduled on the symbols right before the RLM SSB symbols without overlap so that it sends ACK/NACK correctly.

7.5.1.9.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

This test is only applicable to UE which supports pdcch-MonitoringAnyOccasions or pdcch-MonitoringAnyOccasionsWithSpanGap.

7.5.1.9.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 7.5.1.0.5.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.1.9.

7.5.1.9.4	Test description

There is one cell (Cell 1), which is the active NR cell, in the test. The test consists of one time period with time duration of T1. The UE is required during time period T1 to transmit ACK/NACK correctly upon scheduling of PDSCH.

7.5.1.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.1.9.4.1-1.

Table 7.5.1.9.4.1-1: Supported test configurations for NR SA FR2 radio link monitoring UE scheduling restrictions

| Configuration | Description |
| --- | --- |
| 7.5.1.9-1 | 120 kHz SSB SCS, 120 kHz RMC SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.5.1.9.4.1-2

Table 7.5.1.9.4.1-2: Initial conditions for NR SA FR2 radio link monitoring UE scheduling restrictions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E.1.2, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.1.9.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The test parameters for PCell are given in Table 7.5.1.9.4.1-3

2.	Message contents are defined in clause 7.5.1.9.4.3.

3.	There is one carrier and one cell specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.1.9.4.1-3: General test parameters for NR SA FR2 radio link monitoring UE scheduling restrictions

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | 1 | 1 |  |
| SSB configuration |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  | 1 | SMTC pattern 1 |  |
| DRX cycle length | s | 1 | OFF |  |
| T1 | s | 1 | 5 | During T1 the UE is required to correctly transmit ACK/NACK |

7.5.1.9.4.2	Test Procedure

There is one cell (Cell 1), which is the active NR cell, in the test. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. During the test PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1) to ensure that the UE would have ACK/NACK sending.

1. Set the parameters according to T1 in Table 7.5.1.9.4.4-1. Propagation conditions are set according to Annex C.2.2. T1 starts.

2. If the SS receives ACK/NACK on each UL transmission occasion scheduled by PDCCH which are not overlapped with SSBs configured for radio link monitoring during T1, the number of successful tests is increased by one. otherwise the number of failed tests is increased by one.

3. The UE is switched off and then on. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [10] clause 4.5.

4. Repeat steps 1-3 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.1.9.4.3	Message Contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.1.9.4.3-1: Common Exception messages for NR SA FR2 radio link monitoring UE scheduling restrictions

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | TBD |
| Default RRC messages and information elements contents exceptions | TBD |

7.5.1.9.5	Test Requirement

Table 7.5.1.9.4.1-3 and 7.5.1.9.5-1 define the primary level settings including test tolerances for NR SA FR2 radio link monitoring UE scheduling restrictions.

Table 7.5.1.9.5-1: Cell specific test parameters for NR SA FR2 radio link monitoring UE scheduling restrictions

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | AoA1 | AoA2 |
| TDD configuration |  | 1 | TDDConf.3.1 |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD | Not sent |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD | Not sent |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.2 TDD | Not sent |
| TRS configuration |  | 1 | TRS.2.1 TDD | TRS.2.2 TDD |
| PDCCH/PDSCH TCI state |  | 1 | TCI.State.2 | N/A |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 | Not sent |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |
| RLM-RS |  | 1 | SSB with index 0 | SSB with index 1 |
| AoA setup |  | 1 | Setup 3 defined in A.3.15.3 |  |
|  | dB | 1 | 3 | N/A |
| Note2 | dBm/SCS | 1 | -84.9 | Not sent |
|  | dB | 1 | 3 | N/A |
| SS-RSRP Note3 | dBm/SCS | 1 | -81.9 | -81.9 |
| Io | dBm/95.04 MHz | 1 | -51.15 | -52.91 |
| Propagation Condition |  | 1 | AWGN |  |

The UE behaviour during time duration T1 follows the requirements defined in TS 38.133 [6] clause 8.1.7.3:

The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on RLM-RS symbols to be measured for radio link monitoring.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 7.5.2	Interruption

### 7.5.3	SCell activation and deactivation delay

#### 7.5.3.0	Minimum conformance requirements

##### 7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot , where:

THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [8]

Tactivation_time is the SCell activation delay in millisecond.

If the SCell is known and belongs to FR1, Tactivation_time is:

-	TFirstSSB+ 5ms, if the SCell measurement cycle is equal to or smaller than 160ms.

-	TFirstSSB_MAX + Trs + 5ms, if the SCell measurement cycle is larger than 160ms.

If the SCell is unknown and belongs to FR1, provided that the side condition Ês/Iot ≥ -2 dB is fulfilled, Tactivation_time is:

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, then Tactivation_time is TFirstSSB+ 5ms provided:

-	The UE is provided with SMTC for the target SCell, and

-	The SSBs in the serving cell(s) and the SSBs in the SCell fulfil the condition defined in TS 38.133 [6] clause 3.6.3,

-	The parameter ssb-PositionsInBurst is same for the serving cell(s) and the SCell.

If the SCell being activated belongs to FR2 and if there is at least one active serving cell on that FR2 band, if the UE is not provided with any SMTC for the target SCell, Tactivation_time is 3 ms, provided

-	the RS (s) of SCell being activated is (are) QCL-TypeD with RS (s) of one active serving cell on that FR2 band.

If the SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1 or in FR2:

If the target SCell is known to UE and semi-persistent CSI-RS is used for CSI reporting, then Tactivation_time is:

-	3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), where Tuncertainty_MAC=0 and Tuncertainty_SP=0 if UE receives the SCell activation command, semi-persistent CSI-RS activation command and TCI state activation command at the same time.

-

If the target SCell is known to UE and periodic CSI-RS is used for CSI reporting, then Tactivation_time is:

-	max (Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), where Tuncertainty_MAC=0 if UE receives the SCell activation command and TCI state activation commands at the same time.

If the PCell/PSCell and the target SCell are in a band pair with independent beam management and the target SCell is unknown to UE and semi-persistent CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs  + TL1-RSRP, measure + TL1-RSRP, report  + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP).

If the PCell/PSCell and the target SCell are in a band pair with independent beam management and the target SCell is unknown to UE and periodic CSI-RS is used for CSI reporting, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, then Tactivation_time is:

-	3ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + {(THARQ + Tuncertainty_MAC + 5ms + TFineTiming), (Tuncertainty_RRC + TRRC_delay)}.

Where,

TSMTC_MAX:

-	In FR1, in case of intra-band SCell activation, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided the cell specific reference signals from the active serving cells and the SCells being activated or released are available in the same slot; in case of inter-band SCell activation, TSMTC_MAX is the SMTC periodicity of SCell being activated.

-	In FR2, TSMTC_MAX is the longer SMTC periodicity between active serving cells and SCell being activated provided that in Rel-15 only support FR2 intra-band CA.

-	TSMTC_MAX is bounded to a minimum value of 10ms.

Trs is the SMTC periodicity of the SCell being activated if the UE has been provided with an SMTC configuration for the SCell in SCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement which involves Trs is applied with Trs = 5ms assuming the SSB transmission periodicity is 5ms. There are no requirements if the SSB transmission periodicity is not 5ms

TFirstSSB: is the time to the end of the first completed SSB burst indicated by the SMTC after slot n + THARQ+3msNR slot length

TFirstSSB_MAX: Is the time to the end of the first completed SSB burst indicated by the SMTC after slot n + THARQ+3msNR slot length, further fulfilling:

-	In FR1, in case of intra-band SCell activation, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst.

-	In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot.

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

TL1-RSRP, measure is L1-RSRP measurement delay TL1-RSRP_Measurement_Period_SSB (ms) or TL1-RSRP_Measurement_Period_CSI-RS based on applicability as defined in TS 38.133 [6] clause 9.5 assuming M=1.

TL1-RSRP, report is delay of acquiring CSI reporting resources.

Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	SCell activation command for known case;

-	First valid L1-RSRP reporting for unknown case.

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [13].

Longer dekays for RRM measurement requirements, and in case of FR2 also SSB based RLM/BFD/CBD/L1-RSRP measurement requirements, can be expected during the cell detection time for unknown SCell activation.

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13].

SCell in FR1 is known if it has been meeting the following conditions:

-	During the period equal to max (5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified in TS 38.133 [6] clause 9.2 and 9.3.

-	the SSB measured during the period equal to max (5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in TS 38.133 [6] clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

-	During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index

-	SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation

-	During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in TS 38.133 [6] clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

If the UE has been provided with higher layer in TS 38.331 [13] signalling of smtc2 prior to the activation command, TSMTC_Scell follows smtc1 or smtc2 according to the physical cell ID of the target cell being activated. TSMTC_MAX follows smtc1 or smtc2 according to the physical cell IDs of the target cells being activated and the active serving cells.

In addition to CSI reporting defined above, UE shall also apply other actions related to the activation command specified in TS 38.331 [13] for a SCell at the first opportunities for the corresponding actions once the SCell is activated.

The interruption on PSCell or any activated SCell in SCG for EN-DC mode specified in TS 38.133 [6] clause 8.2 shall not occur before slot n+1+ and not occur after slot n+1+.

The interruption  on PCell or any activated SCell in MCG for NR standalone mode specified in TS 38.133 [6] clause 8.2 shall not occur before slot n+1+ and not occur after slot n+1+.

Starting from the slot specified in TS 38.213 [8] clause 4.3 (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in TS 38.213 [8] clause 4.3 (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

The normative reference for this requirement is TS 38.133 [6] clause 8.3.2.

##### 7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell

Upon receiving SCell deactivation command or upon expiry of the sCellDeactivationTimer in slot n, the UE shall accomplish the deactivation actions for the SCell being deactivated no later than in slot n+.

The interruption on PSCell or any activated SCell in SCG for EN-DC mode specified in TS 38.133 [6] clause 8.2 shall not occur before slot n+1+[] and not occur after slot n+1+.

The interruption on PCell or any activated SCell in MCG for NR standalone mode specified in TS 38.133 [6] clause 8.2 shall not occur before slot n+1+[] and not occur after slot n+1+.

The normative reference for this requirement is TS 38.133 [6] clause 8.3.3.

#### 7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test procedure

- Connection diagram

- Message contents are not complete.

- Test Requirements (still brackets in core-spec for replacement of Tactivation_time [TSMTC_SCell + 5ms])

- TT analysis is missing.

- Test Applicability in TS38.522

- Annex F

- Cell configuration mapping in Annex E

7.5.3.1.1	Test purpose

The purpose of this test is:

- To verify the requirement for the SCell activation and deactivation times are within the requirements specified in TS 38.133 [6] clause 8.3, when the PCell and SCell are is in FR2 intra-band and SCell is known by the UE at the time of activation.

7.5.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.5.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.3.0.1 and 7.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.3.1.

7.5.3.1.4	Test description

7.5.3.1.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.3.1.4.1-1.

Table 7.5.3.1.4.1-1: Supported test configurations for NR SA FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.5.3.1.4.1-2.

Table 7.5.3.1.4.1-2: Initial conditions for NR SA FR2 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.3.1.4.1-3.

2. Message contents are defined in clause 7.5.3.1.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is SCell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.3.1.4.1-3: General test parameters for NR SA FR2 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels are used for this test, cell 1 and cell2 use RF channel 1 and 2, respectively. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | slot | k | k is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] depends on UE’s capability |
| TCSI_Reporting | ms | 10+5⋅2µDL | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13]. µDL is the subcarrier spacing configuration for DL. |

7.5.3.1.4.2	Test procedure

TBD

7.5.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.3.1.5	Test requirement

TBD

#### 7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test procedure

- Connection diagram

- Message contents are not complete.

- Test Requirements (still brackets in core-spec for replacement of Tactivation_time [TBD])

- TT analysis is missing.

- Test Applicability in TS38.522

- Annex F

- Cell configuration mapping in Annex E

7.5.3.2.1	Test purpose

The purpose of this test is:

- To verify the requirement for the SCell activation and deactivation times are within the requirements specified in TS 38.133 [6] clause 8.3, when the PCell is in FR1 and SCell is in FR2 and SCell is known by the UE at the time of activation.

7.5.3.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.5.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.3.0.1 and 7.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.3.2.

7.5.3.2.4	Test description

7.5.3.2.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.3.2.4.1-1.

Table 7.5.3.2.4.1-1: Supported test configurations for NR SA FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | PCell: 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | PCell: 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target SCell: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.3.2.4.1-2.

Table 7.5.3.2.4.1-2: Initial conditions for NR SA FR2 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.3.2.4.1-3.

2. Message contents are defined in clause 7.5.3.2.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is SCell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.3.2.4.1-3: General test parameters for NR SA FR2 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 7 | During this time the PSCell shall be known and the SCell configured and detected. |
| T2 | s | 1 | During this time the UE shall activate the SCell. |
| T3 | s | 1 | During this time the UE shall deactivate the SCell. |
| THARQ | slot | k | k is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] depends on UE’s capability |
| TCSI_Reporting | ms | 10+5⋅2µDL | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [13]. µDL is the subcarrier spacing configuration for DL. |

7.5.3.2.4.2	Test procedure

TBD

7.5.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.3.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.3.2.5	Test requirement

TBD

### 7.5.4	UE UL carrier RRC reconfiguration delay

### 7.5.5	Link recovery procedures

#### 7.5.5.0	Minimum conformance requirements

##### 7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures

UE shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB [ms] period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB [ms] period.

The value of TEvaluate_BFD_SSB is defined in Table 7.5.5.0.1-1 for FR2 with N=8

For FR2,

-	P=1/(1 – TSSB/TSMTCperiod), when BFD-RS is not overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is Psharing factor, when BFD-RS is not overlapped with measurement gap and BFD-RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P is 1/(1- TSSB/MGRP - TSSB/TSMTCperiod), when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is 1/(1- TSSB /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TSSB = 0.5*TSMTCperiod

-	P is 1/{1- TSSB /min (TSMTCperiod ,MGRP)}, when BFD-RS is partially overlapped with measurement gap (TSSB <MGRP) and BFD-RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TSSB /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor = 3.

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer evaluation period would be expected if the combination of BFD-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 7.5.5.0.1-1: Evaluation period TEvaluate_BFD_out for FR2

| Configuration | TEvaluate_BFD_SSB (ms) |
| --- | --- |
| no DRX | max([50], ceil(5*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max([50], ceil(7.5*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(5*P*N)*TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

When the radio link quality on all the configured RS resources in set  is worse than Qout_LR, Layer 1 of the UE shall send a beam failure instance indication to the higher layers. A Layer 3 filter may be applied to the beam failure instance indications as specified in TS 38.331 [13].

The beam failure instance evaluation for the configured RS resources in set  shall be performed as specified in section 6 in TS 38.213 [8]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval_BFD.

When DRX is not used, TIndication_interval_BFD is max(2ms, TBFD-RS,M), where TBFD-RS,M is the shortest periodicity of all configured RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB in the set  or CSI-RS resource in the set .

When DRX is used, TIndication_interval_BFD is max(1.5*DRX_cycle_length, 1.5*TBFD-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms.

UE shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB [ms] period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table B.2.4.1 for a corresponding band.

The value of TEvaluate_CBD_SSB is defined in Table 7.5.5.0.1-2 for FR2 with N=8.

For FR2,

-	P=1/(1 – TSSB/TSMTCperiod), when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P is 3, when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

-	P is 1/(1- TSSB/MGRP - TSSB/TSMTCperiod), when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TSSB < 0.5*TSMTCperiod

-	P is 1/(1- TSSB /MGRP)*3, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP  and TSSB = 0.5*TSMTCperiod

-	P is 1/{1- TSSB /min (TSMTCperiod ,MGRP)}, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P is 1/(1- TSSB /MGRP)*3, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC occasion (TSSB = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

In both FR1 and FR2, if different SCS is used for SSB and CSI-RS, and the UE does not support simultaneousRxDataSSB-DiffNumerology, it is assumed that the SSB configured for candidate beam detection and each CSI-RS resource shall be TDMed transmitted.

In FR2, it is assumed that the SSB configured for candidate beam detection and each CSI-RS resource shall be TDMed transmitted.

Table 7.5.5.0.1-2: Evaluation period TEvaluate_CBD_out for FR2

| Configuration | TEvaluate_CBD_SSB (ms) |
| --- | --- |
| non-DRX | ceil([3]*P*N) * TSSB |
| DRX cycle ≤ 320ms | ceil([3]*P*N*1.5) * max(TDRX,TSSB) |
| DRX cycle > 320ms | ceil([3]*P*N) * TDRX |
| Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length. |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.5.2.2, 8.5.4 and 8.5.5.2.

##### 7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures

UE shall be able to evaluate whether the downlink radio link quality on the configured CSI-RS resource in set  estimated over the last TEvaluate_BFD_CSI-RS [ms] period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS [ms] period.

The value of TEvaluate_BFD_CSI-RS is defined in Table 7.5.5.0.2-1 for FR2 with N=1

Editor’s Note: It is FFS if a CSI-RS resource in the resource set with repetition “ON” can be configured as a BFD-RS. If CSI-RS for BFD can be in the resource set with repetition “ON”, N=8 may apply.

Editor’s Note: FFS if there are other conditions with N=8.

For FR2,

-	P=1, when BFD-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when BFD-RS is partially overlapped with measurement gap and BFD-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when BFD-RS is not overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor, when BFD-RS is not overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when BFD-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and BFD-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P is 1/(1- TCSI-RS /MGRP)* Psharing factor, when BFD-RS is partially overlapped with measurement gap and BFD-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

-	Psharing factor is 3.

If the high layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

NOTE: The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of BFD-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

The values of MBFD used in Table 7.5.5.0.2-1 is defined as

-	MBFD = 10, if the CSI-RS resource configured for BFD is transmitted with Density = 3.

Table 7.5.5.0.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | max([50], [MBFD *P*N] * TCSI-RS) |
| DRX cycle ≤ 320ms | max([50], [1.5×MBFD*P*N]*max(TDRX, TCSI-RS)) |
| DRX cycle > 320ms | [MBFD *P*N] * TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

When the radio link quality on all the configured RS resources in set  is worse than Qout_LR, Layer 1 of the UE shall send a beam failure instance indication to the higher layers. A Layer 3 filter may be applied to the beam failure instance indications as specified in TS 38.331 [13].

The beam failure instance evaluation for the configured RS resources in set  shall be performed as specified in section 6 in TS 38.213 [8]. Two successive indications from Layer 1 shall be separated by at least TIndication_interval_BFD.

When DRX is not used, TIndication_interval_BFD is max(2ms, TBFD-RS,M), where TBFD-RS,M is the shortest periodicity of all configured RS resources in set  for the accessed cell, corresponding to either the shortest periodicity of the SSB in the set  or CSI-RS resource in the set .

When DRX is used, TIndication_interval_BFD is max(1.5*DRX_cycle_length, 1.5*TBFD-RS,M) if DRX cycle_length is less than or equal to 320ms, and TIndication_interval is DRX_cycle_length if DRX cycle_length is greater than 320ms.

UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table B.2.4.2 for a corresponding band.

The value of TEvaluate_CBD_CSI-RS is defined in Table 7.5.5.0.2-2 for FR2 with N=8.

Editor’s Note: FFS whether N=1 need to be applied for CSI-RS based candidate beam detection in FR2.

For FR2,

-	P=1, when candidate beam detection RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=1/(1 – TCSI-RS/MGRP) , when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=1/(1 – TCSI-RS /TSMTCperiod), when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is 3, when candidate beam detection RS is not overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P is 1/(1- TCSI-RS /MGRP - TCSI-RS /TSMTCperiod), when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P is 1/(1- TCSI-RS /MGRP)* 3, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5*TSMTCperiod

-	P is 1/{1- TCSI-RS /min (TSMTCperiod ,MGRP)}, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap

-	P is 1/(1- TCSI-RS /MGRP)* 3, when candidate beam detection RS is partially overlapped with measurement gap and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP) [Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM/BFD/BM-RS, or other CBD-RS, according to the measurement restrictions defined in section TBD.]

In both FR1 and FR2, if different SCS is used for SSB and CSI-RS, and the UE does not support simultaneousRxDataSSB-DiffNumerology, it iss assumed that the CSI-RS configured for candidate beam detection and each SSB shall be TDMed transmitted.

In FR2, It is assumed that the CSI-RS configured for candidate beam detection with N=1 shall be TDMed with any RS resources configured for RLM/BFD/CBD/L1-RSRP reporting which is not is QCL-Type D with this CSI-RS resource or under the conditions of N>1 as specified in TS 38.133 [6] section 8.1.2.2, 8.1.2.3, 8.5.2.2, 8.5.2.3, 8.5.2.5, 8.5.2.6, 9.5.4.1 and 9,5,4,2.

The values of MCBD used in Table 7.5.5.0.2-2 is defined as

-	MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3.

Table 7.5.5.0.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX | max([25], ceil(MCBD *P*N) * TCSI-RS) |
| DRX cycle ≤ 320ms | ceil(MCBD *P*N*1.5) * max(TDRX, TCSI-RS) |
| DRX cycle > 320ms | ceil(MCBD *P*N) *TDRX |
| Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. |  |

The normative reference for this requirement is TS 38.133 [6] clause 8.5.3.2, 8.5.4 and 8.5.6.2.

#### 7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.1.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1, and to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

7.5.5.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.5.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.1.

7.5.5.1.4	Test description

There is one NR serving cell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.1.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 7.5.5.1.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.1.4-1: SNR and L1-RSRP variation for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

7.5.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.1.4.1-1.

Table 7.5.5.1.4.1-1: Supported test configurations for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |
| 2 | TDD duplex mode, 240 kHz SSB SCS, 100MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.1.4.1-2.

Table 7.5.5.1.4.1-2: Initial conditions for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.5.1.4.1-3. The measurement gap configuration is according to Table 7.5.5.1.4.1-4.

2. Message contents are defined in clause 7.5.5.1.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.1.4.1-3: General test parameters for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  | Config 1, 2 |  |  |  | TDD |  |
| BWchannel |  | Config 1, 2 |  |  |  | 100: NRB,c = 66 |  |
| DL initial BWP configuration |  | Config 1, 2 |  |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | Config 1, 2 |  |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | Config 1, 2 |  |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | Config 1, 2 |  |  |  | ULBWP.1.1 |  |
| TDD Configuration |  | Config 1, 2 |  |  |  | TDDConf.3.1 |  |
| CORESET Reference Channel |  | Config 1, 2 |  |  |  | CR. 3.1 TDD |  |
| SSB Configuration |  | Config 1 |  |  |  | SSB.1 FR2 |  |
|  |  | Config 2 |  |  |  | SSB.2 FR2 |  |
| SMTC Configuration |  | Config 1, 2 |  |  |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 |  |  |  | 120 KHz |  |
| PRACH Configuration |  | Config 1, 2 |  |  |  | PRACH.2 FR2 |  |
| SSB index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| TCI Configuration |  | Config 1, 2 |  |  |  | TBD |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  |  | 2 |  |
|  | Aggregation level |  |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  |  | 6 |  |
| DRX |  |  |  |  |  | OFF |  |
| Gap pattern ID |  |  |  |  |  | gp0 |  |
| gapOffset |  |  |  |  |  | 0 |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0.. |
| rsrp-ThresholdSSB |  |  | Config 1 |  | dBm/SCS kHz | TBD | Threshold used for Qin_LR_SSB |
|  |  |  | Config 2 |  |  | TBD |  |
| powerControlOffsetSS |  |  |  |  |  |  | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  |  |  | Config 1, 2 |  | CSI-RS.3.1 TDD |  |
| TCI states |  |  |  |  |  | TCI.State.0 |  |
| CSI-RS for tracking |  |  |  | Config 1, 2 |  | TRS.2.1 TDD |  |
| SSB index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 2.61 |  |
| T3 |  |  |  |  | s | 1.64 |  |
| T4 |  |  |  |  | S | 0 |  |
| T5 |  |  |  |  | s | 1.01 |  |
| D1 |  |  |  |  | s | 0.97 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table 7.5.5.1.4.1-4: Void

7.5.5.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is not enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.1.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.1.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.1.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.1.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.1.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.1.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 for until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.5.1.4.3-1: Common Exception messages for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED and GAP NEEDED; Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-4 with a3-offset = -6dB; Table H.3.1-6 with Condition RLM. Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-11 |

7.5.5.1.5	Test requirement

Tables 7.5.5.1.4.1-3 and 7.5.5.1.5-1 define the primary level settings including test tolerances for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX.

Table 7.5.5.1.5-1: NR Cell specific test parameters for NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/120 KHz | TBD |  |  |  |  |
|  | Config 2 |  | TBD |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Measurement gap configuration is assigned to the UE prior to the start of time period T1. Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.1.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE hich supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 960+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.2.1	Test purpose

The purpose of this test is to verify that the UE properly detects SSB-based beam failure in the set q0 configured for a serving cell and that the UE performs correct SSB-based link recovery based on beam candidate set q1, and to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the SSB based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

7.5.5.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.5.5.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.2.

7.5.5.2.4	Test description

There are one NR serving cell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.2.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate SSB based beam failure. Figure 7.5.5.2.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.2.4-1: SNR and L1-RSRP variation for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

7.5.5.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.2.4.1-1.

Table 7.5.5.2.4.1-1: Supported test configurations for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |
| 2 | TDD duplex mode, 240 kHz SSB SCS, 100MHz bandwidth |
| Note: The UE is only required to pass in one of the supported test configurations in FR2 |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.2.4.1-2.

Table 7.5.5.2.4.1-2: Initial conditions for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.5.2.4.1-3. The measurement gap configuration is according to Table 7.5.5.2.4.1-4. The DRX configuration is according to Table 7.5.5.2.4.1-3. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

2. Message contents are defined in clause 7.5.5.2.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.2.4.1-3: General test parameters for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter |  |  |  |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test 1 |  |
| Active PCell |  |  |  |  |  | Cell 1 |  |
| RF Channel Number |  |  |  |  |  | 1 |  |
| Duplex mode |  |  | Config 1, 2 |  |  | TDD |  |
| BWchannel |  |  | Config 1, 2 |  |  | 100: NRB,c = 66 |  |
| DL initial BWP configuration |  |  | Config 1, 2 |  |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  |  | Config 1, 2 |  |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  |  | Config 1, 2 |  |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  |  | Config 1, 2 |  |  | ULBWP.1.1 |  |
| TDD Configuration |  |  | Config 1, 2 |  |  | TDDConf.3.1 |  |
| CORESET Reference Channel |  |  | Config 1, 2 |  |  | CR. 3.1 TDD |  |
| SSB Configuration |  |  | Config 1 |  |  | SSB.1 FR2 |  |
|  |  |  | Config 2 |  |  | SSB.2 FR2 |  |
| SMTC Configuration |  |  | Config 1, 2 |  |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1, 2 |  |  | 120 KHz |  |
| PRACH Configuration |  |  | Config 1, 2 |  |  | PRACH.2 FR2 |  |
| SSB index assigned as BFD RS (q0) |  |  |  |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  |  |  |  | 1 |  |
| TCI Configuration |  |  | Config 1, 2 |  |  | TBD |  |
| OCNG parameters |  |  |  |  |  | OP.1 |  |
| CP length |  |  |  |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  |  |  |  | 1-0 |  |
|  | Number of Control OFDM symbols |  |  |  |  | 2 |  |
|  | Aggregation level |  |  |  | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy |  |  |  | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy |  |  |  | dB | 0 |  |
|  | DMRS precoder granularity |  |  |  |  | REG bundle size |  |
|  | REG bundle size |  |  |  |  | 6 |  |
| DRX |  |  |  |  |  | DRX.3 |  |
| Gap pattern ID |  |  |  |  |  | N.A. |  |
| rlmInSyncOutOfSyncThreshold |  |  |  |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | Config 1 |  |  | dBm/SCS kHz | TBD | Threshold used for Qin_LR_SSB |
|  |  | Config 2 |  |  |  | TBD |  |
| powerControlOffsetSS |  |  |  |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  |  |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  |  |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for CSI reporting |  |  |  | Config 1, 2 |  | CSI-RS.3.1 TDD |  |
| TCI states |  |  |  |  |  | TCI.State.0 |  |
| CSI-RS for tracking |  |  |  | Config 1, 2 |  | TRS.2.1 TDD |  |
| SSB index assigned as RLM RS |  |  |  |  |  | 0, 1 |  |
| T310 Timer |  |  |  |  | ms | 1000 |  |
| N310 |  |  |  |  |  | 2 |  |
| T1 |  |  |  |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  |  |  |  | s | 3.37 |  |
| T3 |  |  |  |  | s | 2.8 |  |
| T4 |  |  |  |  | s | 0 |  |
| T5 |  |  |  |  | s | 0.61 |  |
| D1 |  |  |  |  | s | 0.57 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |  |  |

Table 7.5.5.2.4.1-4: Void

7.5.5.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of [2] ms. In the test, DRX configuration is enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.2.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.2.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.2.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.2.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.2.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.2.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.5.2.4.3-1: Common Exception messages for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED and GAP NEEDED; Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-4 with a3-offset = -6dB; Table H.3.1-6 with Condition RLM. Table H.3.1-8 with Condition SSB BFD Table H.3.1-10 with Condition SSB Table H.3.1-11 Table H.3.7-1 with condition DRX.7 |

7.5.5.2.5	Test requirement

Tables 7.5.5.2.4.1-3 and 7.5.5.2.5-1 define the primary level settings including test tolerances for NR SA FR2 SSB-based beam failure detection and link recovery in DRX.

Table 7.5.5.2.5-1: NR Cell specific test parameters for NR SA FR2 SSB-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
|  | Config 2 |  | 5 | -3 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -12 | -12 | -12 | -3 | 10 |
|  | Config 2 |  | -12 | -12 | -12 | -3 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |
|  | Config 2 |  | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/120 KHz | TBD |  |  |  |  |
|  | Config 2 |  | TBD |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.2.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 560+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.3.1	Test purpose

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1, and to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

7.5.5.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.5.5.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.3.

7.5.5.3.4	Test description

There is one NR serving cell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.3.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate CSI-RS based beam failure. Figure 7.5.5.3.4-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.3.4-1: SNR and L1-RSRP variation for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

7.5.5.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.3.4.1-1.

Table 7.5.5.3.4.1-1: Supported test configurations for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.3.4.1-2.

Table 7.5.5.3.4.1-2: Initial conditions for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.5.3.4.1-3. The NZP-CSI-RS configuration is according to Table 7.5.5.3.4.1-3.

2. Message contents are defined in clause 7.5.5.3.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.3.4.1-3: General test parameters for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.3 FR2 |  |
| SMTC Configuration | Config 1 |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120KHz |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | OFF |  |
| Gap pattern ID |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | TBD | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | NA | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for q0 and q1 | Config 1 |  | CSI-RS.3.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |  |
| csi-RS-Index assigned as RLM RS |  |  | 0, 1 |  |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 1.17 |  |
| T3 |  | s | 0.9 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

7.5.5.3.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is not enabled.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.3.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.3.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.3.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.3.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.3.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.3.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.5.3.4.3-1: Common Exception messages for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTRA-FREQ and L3 FILTERING NEEDED; Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-8 with Condition CSI-RS BFD Table H.3.1-10 with Condition CSI-RS Table H.3.1-11 |

7.5.5.3.5	Test requirement

Tables 7.5.5.3.4.1-3 and 7.5.5.3.5-1 define the primary level settings including test tolerances for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX.

Table 7.5.5.3.5-1: NR Cell specific test parameters for NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumptpion for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/15 KHz | TBD |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.3.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 =260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.4.1	Test purpose

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1, and to test the downlink monitoring for beam failure detection within the UEs active DL BWP, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

7.5.5.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.5.5.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.4.

7.5.5.4.4	Test description

There is one NR serving cell configured in this test. This testconsists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.4.4-1 shows the five different time durations and the corresponding variation of the downlink SNR in the active cell to emulate CSI-RS based beam failure. Figure 7.5.5.4.4-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.4.4-1: SNR and L1-RSRP variation for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

7.5.5.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.4.4.1-1.

Table 7.5.5.4.4.1-1: Supported test configurations for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Configuration | Description |
| --- | --- |
| 7.5.5.4-1 | TDD duplex mode, 120 kHz SSB SCS, 100MHz bandwidth |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.4.4.1-2.

Table 7.5.5.4.4.1-2: Initial conditions for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.5.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.5.4.4.1-3. The measurement gap configuration is according to Table 7.5.5.4.4.1-4. The NZP-CSI-RS configuration is according to Table 7.5.5.4.4.1-3. The DRX configuration for is according to Table 7.5.5.4.4.1-3. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

2. Message contents are defined in clause 7.5.5.4.4.3.

3. There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.4.4.1-3: General test parameters for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.3 FR2 |  |
| SMTC Configuration | Config 1 |  | SMTC.3 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| csi-RS-Index assigned as beam failure detection RS in set q0 |  |  | 0 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | CSI-RS.Config.0 |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | DRX.3 |  |
| Gap pattern ID |  |  | N.A. |  |
| csi-RS-Index assigned as candidate beam detection RS in set q1 |  |  | 1 |  |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the value 0. |
| rsrp-ThresholdSSB |  | dBm/SCS kHz | TBD | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see clause 5.17 of TS 38.321 [12] |
| beamFailureDetectionTimer |  |  | pbfd4 | see clause 5.17 of TS 38.321 [12] |
| CSI-RS configuration for q0 and q1 | Config 1 |  | CSI-RS.3.2 TDD |  |
| CSI-RS configuration for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |  |
| csi-RS-Index assigned as RLM RS | Config 1 |  | CSI-RS.3.2 TDD |  |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 5.43 |  |
| T3 |  | s | 5.16 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 0.31 |  |
| D1 |  | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

Table 7.5.5.4.4.1-4: Void

7.5.5.4.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to NR Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 2 ms. In the test, DRX configuration is enabled. The UE is configured to perform inter-frequency measurements using GP ID #0 (40ms).

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.4.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.4.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.4.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.4.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.4.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point A to time point B

and

b) does not detect any uplink power on NR carrier higher than OFF power defined in TS 38.521-1 [17] clause 6.3.2.5 from time point C until T3 expires

and

c) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point F (D1 after the start of T5) until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.4.5-1.

9.	Wait [1s] for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within [1s] continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.5.4.4.3-1: Common Exception messages for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-2 with Condition INTER-FREQ, L3 FILTERING NEEDED and GAP NEEDED; Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR2, SMTC.1 and RLM Table H.3.1-4 with a3-offset = -6dB; Table H.3.1-6 with Condition RLM. Table H.3.1-8 with Condition CSI-RS BFD Table H.3.1-10 with Condition CSI-RS Table H.3.1-11; Table H3.7-1 with Condition DRX.7 |

7.5.5.4.5	Test requirement

Tables 7.5.5.4.4.1-3 and 7.5.5.4.5-1 define the primary level settings including test tolerances for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX.

Table 7.5.5.4.5-1: NR Cell specific test parameters for NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9 |  |  |  |  |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | TBD | TBD | TBD | TBD | TBD |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | TBD | TBD | TBD | TBD | TBD |
|  | Config 1 | dBm/120 KHz | TBD |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.4.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

#### 7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- Connection diagram is TBD.

- TT analysis is missing.

- Test requirements are between brackets.

7.5.5.5.1	Test purpose

The purpose of this test is to test scheduling availability restrictions when the UE is performing beam failure detection or when the UE is performing L1-RSRP measurement for candidate beam detection, when no DRX is used, and to verify the scheduling availability restriction requirements for SSB based beam failure detection and link recovery for an FR2 serving cell in TS 38.133 [6] clause 8.5.7 and 8.5.8.

7.5.5.5.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.5.5.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.5.5.

7.5.5.5.4	Test description

There is one NR serving cell configured in this test. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 7.5.5.5.4-1 shows the variation of the downlink SNR of the SSB in set q0 in the active cell to emulate SSB based beam failure. Figure 7.5.5.5.4-1 additionally shows the variation of the downlink L1-RSRP of the SSB in set q1 of the candidate beam used for link recovery.

Figure 7.5.5.5.4-1: SNR and L1-RSRP variation for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

7.5.5.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.5.5.4.1-1.

Table 7.5.5.5.4.1-1: Supported test configurations for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Configuration | Description |
| --- | --- |
| 7.5.5.5-1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.5.5.4.1-2.

Table 7.5.5.5.4.1-2: Initial conditions for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.5.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 7.5.5.5.4.1-3.

2.	Message contents are defined in clause 7.5.5.5.4.3.

3.	There is one NR carrier and one NR cells specified in the test. Cell 1 is the NR cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.5.5.4.1-3: General test parameters for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 |  |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | TDD |  |
| TDD Configuration | Config 1 |  | TDDConf.3.1 |  |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |  |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |  |
| CORESET Reference Channel | Config 1 |  | CR. 3.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.1 FR2 |  |
| SMTC Configuration | Config 1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1 |  | 120 KHz |  |
| PRACH Configuration | Config 1 |  | PRACH.2 FR2 |  |
| SSB index assigned as BFD RS (q0) |  |  | 0 |  |
| SSB index assigned as CBD RS (q1) |  |  | 1 |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |
| TCI configuration |  |  | TCI.State.0 |  |
| OCNG parameters |  |  | OP.1 |  |
| AoA Setup |  |  | Setup 1 | A.9 |
| CP length |  |  | Normal |  |
| Beam failure detection transmission parameters | DCI format |  | 1-0 |  |
|  | Number of Control OFDM symbols |  | 2 |  |
|  | Aggregation level | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average CSI-RS RE energy | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average CSI-RS RE energy | dB | 0 |  |
|  | DMRS precoder granularity |  | REG bundle size |  |
|  | REG bundle size |  | 6 |  |
| DRX |  |  | OFF | DRX is not in use |
| Gap pattern ID |  |  | N.A. | No measurement gap pattern is configured |
| ssb-Index |  |  | 2 | Number of SSB indexes used for beam failure detection |
| rlmInSyncOutOfSyncThreshold |  |  | absent | When the field is absent, the UE applies the 10% |
| rsrp-ThresholdSSB |  | dBm/SCS KHz | -94.5 | Threshold used for Qin_LR |
| powerControlOffsetSS |  |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI Configuration for reporting | Config 1 |  | CSI-RS.3.1 TDD | A.3.14.2 |
| T310 Timer |  | ms | 1000 |  |
| N310 |  |  | 2 |  |
| T1 |  | s | 1 | During this time the UE shall be fully synchronized to cell 1 |
| T2 |  | s | 2.6 |  |
| T3 |  | s | 1.64 |  |
| T4 |  | s | 0 |  |
| T5 |  | s | 1.01 |  |
| D1 |  | s | 0.97 |  |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |

7.5.5.5.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. This test will focus on the scheduling availability during beam failure detection and candidate beam detection. In the test, DRX configuration is not enabled. During the test the UE is scheduled to transmit continuously in UL.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters of NR Cell 1 according to T1 in Table 7.5.5.5.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

3.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 7.5.5.5.5-1. T2 starts.

4.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 7.5.5.5.5-1. T3 starts.

5.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 7.5.5.5.5-1. T4 starts.

6.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 7.5.5.5.5-1. T5 starts.

7.	If the SS:

a) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) which are not overlapped with SSBs configured for beam failure detection during the period from time point B to time point D

and

b) detects uplink power on NR carrier equal to or higher than minimum output power defined in TS 38.521-1 [17] clause 6.3.1.5 in each slot configured for CQI transmission (according CQI reporting on PUCCH) during the period from time point D until T5 expires,

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

8.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 7.5.5.5.5-1.

9.	Wait 1s for the UE to re-establish the connection or continue directly to step 10. If the UE re-establishes the connection within 1s continue to step 11. Otherwise continue to step 10.

10.	Switch the UE on and off. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat steps 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.5.5.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.5.5.4.3-1: Common Exception messages for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.5.5.5	Test requirement

Table 7.5.5.5.4.1-3 and 7.5.5.5.5-1 define the primary level settings including test tolerances for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX.

Table 7.5.5.5.5-1: NR Cell specific test parameters for NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Assumption for UE beams Note 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_SSB of set q0 | Config 1 | dB | 5 | -3 | -12 | -12 | -12 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
| SNR_SSB of set q1 | Config 1 | dB | -12 | -12 | 5 | 5 | 5 |
|  | Config 1 | dBm/15KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 7.5.5.5.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |  |

The UE behaviour during time duration T3 follows the requirements defined in TS 38.133 [6] clause 8.5.7.3:

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on BFD-RS symbols to be measured for beam failure detection.

The UE behaviour during time durations T4 and T5 follows the requirements defined in TS 38.133 [6] clause 8.5.8.3:

-	The UE is not expected to transmit PUCCH/PUSCH or receive PDCCH/PDSCH on reference symbols to be measured for candidate beam detection.

### 7.5.6	Active BWP switch delay

#### 7.5.6.1	DCI-based and time-based active BWP switch

##### 7.5.6.1.0	Minimum conformance requirements

[TS 38.133, clause 8.6.2]

The requirements in this clause only apply to the case that the BWP switch is performed on a single CC.

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after the beginning of DL slot n+ TBWPswitchDelay.

The UE is not required to transmit UL signals or receive DL signals during time duration TBWPswitchDelay on the cell where DCI-based BWP switch occurs. The UE is not required to follow the requirements defined in this clause when performing a DCI-based BWP switch between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths.

Depending on UE capability bwp-SwitchingDelay [2], UE shall finish BWP switch within the time duration TBWPswitchDelay defined in Table 7.5.6.1.0-1.

Table 7.5.6.1.0-1: BWP switch delay

|  | NR Slot length (ms) | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  |  | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| Note 1: Depends on UE capability. Note 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

Provided the UE does not have the required TCI-state information to receive PDCCH and PDSCH in the new BWP, the UE shall use old TCI-states before the BWP switch until a new MAC CE updating the required TCI-state information for PDCCH and PDSCH is received after the BWP switch.

If UE has the information on the required TCI-state information to receive PDCCH and PDSCH in the new BWP,

-	UE shall be able to receive PDCCH and PDSCH with old TCI-states before the delay as specified in Clause 8.10 in the new BWP.

-	UE shall be able to receive PDCCH and PDSCH with new TCI-states after the delay as specified in Clause 8.10 in the new BWP.

##### 7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test tolerance analysis is missing

- Message contents are TBD

- Cell mapping is TBD

- Test procedure is TBD

- Test applicability needs to be added to TS 38.522

7.5.6.1.1.1	Test purpose

FFS

7.5.6.1.1.2	Test applicability

FFS

7.5.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.6.1.1.

7.5.6.1.1.4	Test description

7.5.6.1.1.4.1	Initial conditions

FFS

7.5.6.1.1.4.2	Test procedure

FFS

7.5.6.1.1.4.3	Message contents

FFS

7.5.6.1.1.5	Test requirements

FFS

##### 7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test tolerance analysis is missing

- Message contents are TBD

- Cell mapping is TBD

- Test procedure is TBD

- Test applicability needs to be added to TS 38.522

7.5.6.1.2.1	Test purpose

FFS

7.5.6.1.2.2	Test applicability

FFS

7.5.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.6.1.2.

7.5.6.1.2.4	Test description

7.5.6.1.2.4.1	Initial conditions

FFS

7.5.6.1.2.4.2	Test procedure

FFS

7.5.6.1.2.4.3	Message contents

FFS

7.5.6.1.2.5	Test requirements

FFS

##### 7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test tolerance analysis is missing

- Message contents are TBD

- Cell mapping is TBD

- Test procedure is TBD

- Test applicability needs to be added to TS 38.522

7.5.6.1.3.1	Test purpose

The purpose of this test is to verify the DL BWP switch delay requirement defined in TS 38.133 [6] clause 8.6

7.5.6.1.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.5.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.6.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.6.1.3.

7.5.6.1.3.4	Test description

There is one cell configured in this test. Cell 1 is PCell. This test consists of three successive time periods, with time duration of T1, T2 and T3 respectively.

7.5.6.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.5.6.1.3.4.1-1.

Table 7.5.6.1.3.4.1-1: Supported test configurations for NR SA FR2 DCI-based DL active BWP switch in non-DRX

| Config | Description |
| --- | --- |
| 7.5.6.1.3-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: A UE which fulfils the requirements in test case 7.5.6.1.1 or 7.5.6.1.2 can skip the test cases in 7.5.6.1.3. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.6.1.3.4.1-2.

Table 7.5.6.1.3.4.1-2: Initial conditions for NR SA FR2 DCI-based DL active BWP switch in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 5.5.5.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 7.5.6.1.3.4.1-3.

2.	Message contents are defined in clause 7.5.6.1.3.4.3.

3.	There are one NR cell specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.5.6.1.3.4.1-3: General test parameters for NR SA FR2 DCI-based DL active BWP switch in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active Cell |  | Cell 1 | Cell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| bwp-InactivityTimer | ms | [200] |  |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |
| T3 | s | [0.2] |  |

7.5.6.1.3.4.2	Test procedure

FFS

7.5.6.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.5.6.1.3.4.3-1: Common Exception messages for NR SA FR2 DCI-based DL active BWP switch in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.6.1.3.5	Test requirements

FFS

#### 7.5.6.2	RRC-based active BWP switch

##### 7.5.6.2.0	Minimum conformance requirements

FFS

##### 7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test tolerance analysis is missing

- Message contents are TBD

- Cell mapping is TBD

- Test procedure is TBD

- Test applicability needs to be added to TS 38.522

7.5.6.2.1.1	Test purpose

FFS

7.5.6.2.1.2	Test applicability

FFS

7.5.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.5.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.6.2.1.

7.5.6.2.1.4	Test description

7.5.6.2.1.4.1	Initial conditions

FFS

7.5.6.2.1.4.2	Test procedure

FFS

7.5.6.2.1.4.3	Message contents

FFS

7.5.6.2.1.5	Test requirements

FFS

### 7.5.7	PSCell addition and release delay

#### 7.5.7.0	Minimum conformance requirements

##### 7.5.7.0.1	Minimum conformance requirements for PSCell addition delay

Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR2 no later than in subframe n + Tconfig_PSCell:

Where:

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [13].

Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 40 ms.

Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 24* Trs ms.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell.

TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [8].

Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

In FR1 and FR2, the PSCell is known if it has been meeting the following conditions:

During the last 5 seconds before the reception of the PSCell configuration command:

-	the UE has sent a valid measurement report for the PSCell being configured and

-	One of the SSBs measured from the PSCell being configured remains detectable according to the cell identification conditions specified in TS 38.133 [6] clause 9.3.

-	One of the SSBs measured from PSCell being configured also remains detectable during the PSCell configuration delay Tconfig_PSCell according to the cell identification conditions specified in TS 38.133 [6] clause 9.3.

otherwise it is unknown.

The PCell interruption specified in TS 38.133 [6] clause 8.2 is allowed only during the RRC reconfiguration procedure in TS 38.331 [13].

The normative reference for this requirement is TS 38.133 [6] clause 8.9.2.

##### 7.5.7.0.2	Minimum conformance requirements for PSCell release delay

The requirements in this clause shall apply for a UE which is configured with PCell and one PSCell.

Upon receiving PSCell release in subframe n, the UE shall accomplish the release actions specified in TS 38.331 [13] no later than in subframe n+ TRRC_delay:

Where

TRRC_delay is the RRC procedure delay as specified in TS 38.331 [13].

The PCell interruption specified in TS 38.133 [6] clause 8.2 is allowed only during the RRC reconfiguration procedure in TS 38.331 [13].

The normative reference for this requirement is TS 38.133 [6] clause 8.9.3.

#### 7.5.7.1	NR SA FR2 addition and release delay of known PSCell

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test procedure

- Connection diagram

- Message contents are not complete.

- Test Requirements (still brackets in core-spec for PRACH preamble time [112] ms and CSI report time [20] ms)

- TT analysis is missing.

- Test Applicability in TS38.522

- Annex F

- Cell configuration mapping in Annex E

7.5.7.1.1	Test purpose

The purpose of this test is:

- To verify the requirement for the PSCell addition and release delay are within the requirements specified in TS 38.133 [6] clause 8.9.2, when the PSCell is known to the UE at the time of addition.

7.5.7.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.5.7.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.7.0.1 and 7.5.7.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.7.1.

7.5.7.1.4	Test description

7.5.7.1.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.7.1.4.1-1.

Table 7.5.7.1.4.1-1: Supported test configurations for NR SA FR2 PSCell

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 2 | FR1 TDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 3 | FR1 TDD SSB SCS 30kHz BW 40MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.7.1.4.1-2.

Table 7.5.7.1.4.1-2: Initial conditions for NR SA FR2 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.7.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.7.1.4.1-3.

2. Message contents are defined in clause 7.5.7.1.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell (PSCell-to-be). Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.7.1.4.1-3: General test parameters for NR SA FR2 PSCell addition and release delay

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| Neighbour cell |  |  | Cell 2 | Neighbour cell (PSCell-to-be) on RF channel number 2 in FR2 |
| A4 | Hysteresis | dB | 0 | Hysteresis for event A4 |
|  | Threshold RSRP | dBm | -97 | Threshold for event A4 |
|  | Time to Trigger | S | 0 | Time to trigger for event A4 |
| DRX |  |  | OFF | For both PCell and PSCell once activated |
| Measurement gap pattern ID |  |  | 0 | Gaps are configured before T2 and released before T3. |
| PRACH configuration in Cell 2 |  |  | FR2 PRACH configuration 2 | PRACH configuration as specified in Clause A.3.8.3.2. |
| CSI reporting periodicity and offset configuration for Cell 2 |  | ms | [2] |  |
| T1 |  | s | 5 | During this time the PCell is known and Cell 2 is unknown. |
| T2 |  | s | 1 | During this time the UE shall identify neighbour cell 2 and report event B1. |
| T3 |  | s | 1 | During this time the UE adds the PSCell. |
| T4 |  | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T5 |  | s | 1 | During this time the UE releases the PSCell. |

7.5.7.1.4.2	Test procedure

TBD

7.5.7.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.7.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.7.1.5	Test requirement

TBD

#### 7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Test procedure

- Connection diagram

- Message contents are not complete

- Test Requirements (still brackets in core-spec for PRACH preamble time [572] ms and CSI report time [20] ms)

- TT analysis is missing

- Test Applicability in TS38.522

- Annex F

- Cell configuration mapping in Annex E

7.5.7.2.1	Test purpose

The purpose of this test is:

- To verify the requirement for the PSCell addition and release delay are within the requirements specified in TS 38.133 [6] clause 8.9.2, when the PSCell is unknown to the UE at the time of addition.

7.5.7.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.5.7.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.7.0.1 and 7.5.7.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.7.2.

7.5.7.2.4	Test description

7.5.7.2.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.5.7.2.4.1-1.

Table 7.5.7.2.4.1-1: Supported test configurations for NR SA FR2 PSCell

| Config | Description |
| --- | --- |
| 1 | FR1 FDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 2 | FR1 TDD SSB SCS 15kHz BW 10MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| 3 | FR1 TDD SSB SCS 30kHz BW 40MHz – FR2 TDD SSB SCS 240kHz BW 100MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.5.73.2.4.1-2.

Table 7.5.7.2.4.1-2: Initial conditions for NR SA FR2 PSCell addition and release delay

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.3.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.5.7.2.4.1-3.

2. Message contents are defined in clause 7.5.7.2.4.3.

3. There are two NR carriers and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell (PSCell-to-be). Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.5.7.2.4.1-3: General test parameters for NR SA FR2 PSCell addition and release delay

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| Neighbour cell |  | Cell 2 | Neighbour cell (PSCell-to-be) on RF channel number 2 in FR2 |
| DRX |  | OFF | For both PCell and PSCell once activated |
| PRACH configuration in Cell 2 |  | FR2 PRACH configuration 2 | PRACH configuration as specified in Clause A.3.8.3.2. |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | [2] |  |
| T1 | s | 5 | During this time the PCell is known and Cell 2 is unknown. |
| T2 | s | 1 | During this time the UE adds the PSCell. |
| T3 | s | 1 | During this time the UE sends CSI reports for PSCell. |
| T4 | s | 1 | During this time the UE releases the PSCell. |

7.5.7.2.4.2	Test procedure

TBD

7.5.7.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.5.7.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | FFS |

7.5.7.2.5	Test requirement

TBD

## 7.6	Measurement procedures

### 7.6.1	Intra-frequency measurements

#### 7.6.1.0	Minimum conformance requirements

##### 7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap

[TS 38.133, clause 9.2.5.1 and 9.2.5.2]

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCellis always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Where:

TPSS/SSS_sync_intr: it is the time period used in PSS/SSS detection given in table 7.6.1.0.1-1.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 7.6.1.0.1-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps.

Mpss/sss_sync_w/o_gaps : For a UE supporting FR2 power class 1, Mpss/sss_sync_w/o_gaps = 40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps = 24. For a UE supporting FR2 power class 3, Mpss/sss_sync_w/o_gaps = 24. For a UE supporting FR2 power class 4, Mpss/sss_sync_w/o_gaps =24

Mmeas_period_w/o_gaps : For a UE supporting power class 1, Mmeas_period_w/o_gaps = 40. For a UE supporting FR2 power class 2, Mmeas_period_w/o_gaps = 24. For a UE supporting power class 3, Mmeas_period_w/o_gaps = 24. For a UE supporting power class 4, Mmeas_period_w/o_gaps = 24.

When intrafrequency SMTC is fully non-overlapping with measurement gaps or intrafrequency SMTC is fully overlapping with MGs, Kp=1.

When intrafrequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP

For FR2 when any of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap is fully overlapping with intra-frequency SMTC, Klayer1_measurement= 1.5, otherwise Klayer1_measurement=1.

Table 7.6.1.0.1-1: Time period for PSS/SSS detection (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mpss/sss_sync_w/o_gaps x Kp x Klayer1_measurement) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 7.6.1.0.1-2: Measurement period for intra-frequency measurements without gaps (Frequency FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

[TS 38.133, clause 9.2.4.3]

Reported RSRP, RSRQ, and RS-SINR measurements contained in periodically triggered measurement reports shall meet the requirements in TS 38.133 [6] clause 10.1.2.1, 10.1.7.1 and 10.1.12.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH.This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify intra without index defined in TS 38.133 [6] clause 9.2.5.1. When L3 filtering is used an additional delay can be expected.

If a cell which has been detectable at least for the time period than Tidentify intra without index defined in TS 38.133 [6] clause 9.2.5.1 becomes undetectable for a period and then the cell becomes detectable again and triggers an event, the event triggered measurement reporting delay shall be less than TMeasurement_Period, Intra provided the timing to that cell has not changed more than  3200 Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

[TS 38.133, clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

References: The conformance requirements covered in the current TC are specified in: TS 38.133 [6], clauses 9.2.2, 9.2.4.3, 9.2.5.1 and 9.2.5.2.

##### 7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 7.6.1.0.2-1.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 7.6.1.0.2-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in TS 38.133 [6] section 9.1.5.2.2 for measurement conducted within measurement gaps.

Mpss/sss_sync_with_gaps : For a UE supporting FR2 power class 1, Mpss/sss_sync with_gaps=40. For a UE supporting FR2 power class 2, Mpss/sss_sync with_gaps =24. For a UE supporting FR2 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting power class 4, Mpss/sss_sync with_gaps =24

Mmeas_period_ with_gaps: For a UE supporting power class 1, Mmeas_period_ with_gaps =40. For a UE supporting power class 2, Mmeas_period_ with_gaps =24. For a UE supporting power class 3, Mmeas_period_ with_gaps =24. For a UE supporting power class 4, Mmeas_period with_gaps =24.

Table 7.6.1.0.2-1: Time period for PSS/SSS detection (Frequency range FR2)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, Mpss/sss_sync_with_gaps x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5x Mpss/sss_sync_with_gaps) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Mpss/sss_sync_with_gaps x max(MGRP, DRX cycle) x CSSFintra |

Table 7.6.1.0.2-2: Measurement period for intra-frequency measurements with gaps (Frequency Range FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, Mmeas_period with_gaps x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5 x Mmeas_period with_gaps) x max(MGRP, SMTC period, DRX cycle)) Note 1 x CSSFintra |
| DRX cycle>320ms | Mmeas_period with_gaps x max(MGRP, DRX cycle) x CSSFintra |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] Sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_without_index defined in TS 38.133 [6] section 9.2.6.2. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.6.2 and 9.2.6.3.

#### 7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-Connection diagrams are missing.

- Some parameters are TBD

- Test tolerance is missing.

7.6.1.1.1	Test purpose

The purpose of this test is to verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under non-DRX. This test will partly verify the TDD intra-frequency cell search requirements defined in TS 38.133 clause 9.2.5.1 and 9.2.5.2

7.6.1.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.1.1.

7.6.1.1.4	Test description

7.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.1.1.4.1-1.

Table 7.6.1.1.4.1-1: Supported test configurations for NR SA FR2 event-triggered reporting without gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 7.6.1.1-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.1-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 7.6.1.1.4.1-2.

Table 7.6.1.1.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting without gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 7.6.1.1.4.1-3 below.

2. Message contents are defined in clause 7.6.1.1.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.6.1.1.4.1-3: General test parameters for NR SA FR2 event-triggered reporting without gap in non-DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -6 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

7.6.1.1.4.2	Test procedure

Two cells are deployed in the test, which are FR2 PCell (NR Cell 1) and a FR2 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 7.6.1.1.4.1-3 and Table 7.6.1.1.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.1.1.5-1. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.1.1.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is

-	2402 ms for UE supporting power class 1,

-	1442 ms for UE supporting power class 2, 3 or 4,

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:
- transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.1.1.4.3-1: Common Exception messages for NR SA FR2 event-triggered reporting without gap in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.1-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.1-2 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ |

7.6.1.1.5	Test requirement

Table 7.6.1.1.4.1-3, Table 7.6.1.1.5-1 and Table 7.6.1.1.5-2 define the primary level settings including test tolerances for NR SA FR2 event-triggered reporting without gap in non-DRX.

Table 7.6.1.1.5-1: NR Cell specific test parameters for NR SA FR2 event-triggered reporting without gap in non-DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.state.2 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.1 FR2 |  | SSB.1 FR2 |  |
|  |  | 2 | SSB.2 FR2 |  | SSB.2 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

Table 7.6.1.1.5-2: NR OTA Cell specific test parameters for NR SA FR2 event-triggered reporting without gap in non-DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.8.15.3 |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  |  | Rough |  |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity |  | 8+TT |
| Note 2 | dBm/15 KHz | 1, 2 | -102+TT |  |  |  |  |
| Note 2 | dBm/SCS | 1 | -93+TT |  |  |  |  |
|  |  | 2 | -90+TT |  |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -89+TT | -89+TT | -Infinity |  | -85+TT |
|  |  | 2 | -86+TT | -86+TT | -Infinity |  | -82+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity |  | 8+TT |
|  | dBm/95.04MHz | 1, 2 | -58.56+TT |  | -55.38+TT |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	2.4s for a UE supporting power class 1,

-	1.44s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-Connection diagrams are missing.

- Test tolerance is missing.

7.6.1.2.1	Test purpose

Ther purpose of this test is to verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under DRX. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 clause 9.2.5.1 and 9.2.5.2.

7.6.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.1.2.

7.6.1.2.4	Test description

7.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.1.2.4.1-1.

Table 7.6.1.2.4.1-1: Supported test configurations for NR SA FR2 event-triggered reporting without gap in DRX

| Test Case ID | Description |
| --- | --- |
| 7.6.1.2-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.2-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.6.1.2.4.1-2.

Table 7.6.1.2.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 7.6.1.2.4.1-3 below.

2. Message contents are defined in clause 7.6.1.2.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.6.1.2.4.1-3: General test parameters for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2 | -6 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX.2 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 10 | 52 |  |

7.6.1.2.4.2	Test procedure

Two cells are deployed in the test, which are FR2 PCell (NR Cell 1) and a FR2 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 7.6.1.2.4.1-3, Table 7.6.1.2.5-1 and Table 7.6.1.2.5-2, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.1.2.5-1 and Table 7.6.1.2.5-2. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.1.2.5-1 and Table 7.6.1.2.5-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms for sub-test 1 or less than Y ms for sub-test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is

-	7202 for UE supporting power class 1,

-	4322 for UE supporting power class 2, 3 or 4.

and Y is

-	51202 for UE supporting power class 1,

-	30722 for UE supporting power class 2, 3 or 4.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:
- transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.1.2.4.1-1 as appropriate.

7.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.1.2.4.3-1: Common Exception messages for NR SA FR2 event-triggered reporting without gap in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.2-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.2-2 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ Table H.3.7-1 with Condition DRX.1 for sub-test 1 Table H.3.7-1 with Condition DRX.2 for sub-test 2 |

7.6.1.2.5	Test requirement

Table 7.6.1.2.4.1-3, Table 7.6.1.2.5-1 and Table 7.6.1.2.5-2 define the primary level settings including test tolerances for NR event triggered reporting in synchronous cells when DRX is used test.

Table 7.6.1.2.5-1: NR Cell specific test parameters for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.state.2 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

Table 7.6.1.2.5-2: NR OTA Cell specific test parameters for NR SA FR2 event-triggered reporting without gap in DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 1 defined in A.9 |  |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
| Note 2 | dBm/15 KHz | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/SCS | 1 | -89+TT |  |  |  |
|  |  | 2 | -86+TT |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -85+TT | -85+TT | -Infinity | -85+TT |
|  |  | 2 | -82+TT | -82+TT | -Infinity | -82+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
|  | dBm/95.04MHz | 1 | -54.53+TT | -52.18+TT | -54.53+TT | -52.18+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to NR Cell 2 on PUSCH.

For both tests:

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The overall delays measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delay measured when DRX cycle length is 40 ms (sub-test 1) is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

For UE supporting power class 1, TPSS/SSS_sync_intra = 3600 ms, T SSB_measurement_period_intra= 3600 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intra = 2160 ms, T SSB_measurement_period_intra= 2160 ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 40 ms shall be less than a total of X ms, where X is

- X = 7202 for UE supporting power class 1,

- X = 4322 for UE supporting power class 2, 3 or 4,

The overall delay measured when DRX cycle length is 640 ms (sub-test 2) is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

For UE supporting power class 1, TPSS/SSS_sync_intra = 25600 ms, T SSB_measurement_period_intra= 25600 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intra = 15360 ms, T SSB_measurement_period_intra= 15360 ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of X ms, where

- X = 51202 for UE supporting power class 1,

- X = 30722 for UE supporting power class 2, 3 or 4,

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-Connection diagrams are missing.

- Some parameters are TBD

- Test tolerance is missing.

7.6.1.3.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under non-DRX within intra-frequency cell search with gaps requirements. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 clause 9.2.5.1 and 9.2.5.2.

7.6.1.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.1.3.

7.6.1.3.4	Test description

7.6.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.1.3.4.1-1.

Table 7.6.1.3.4.1-1: Supported test configurations for NR SA FR2 event-triggered reporting with gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 7.6.1.3-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.3-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.6.1.3.4.1-2.

Table 7.6.1.3.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting with gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.6.1.3.4.1-3.

2. Message contents are defined in clause 7.6.1.3.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.6.1.3.4.1-3: General test parameters for NR SA FR2 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |
| Neighbour cell |  | 1, 2 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | 1, 2 | Per-UE gaps |  |
| Measurement gap repitition periodicity | ms | 1, 2 | 40 |  |
| Measurement gap length | ms | 1, 2 | 6 |  |
| Measurement gap offset | ms | 1, 2 | 39 |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.3.2 TDD |  |
| A3-Offset | dB | 1, 2 | -6 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

7.6.1.3.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 7.6.1.3.4.1-3, Table 7.6.1.3.5-1 and Tavke 7.6.1.3.5-2, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.1.3.5-1 and Table 7.6.1.3.5-2. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.1.3.5-1 and Table 7.6.1.3.5-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is

-	X = 3202 for UE supporting power class 1,

-	X = 1922 ms for UE supporting power class 2, 3 or 4,

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.1.3.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR2, SMTC.1 and Synchronous cells for Configuration 7.6.1.3-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR2, SMTC.1 and Synchronous cells for Configuration 7.6.1.3-2 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ |

Table 7.6.1.3.4.3-: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

7.6.1.3.5	Test requirement

Table 7.6.1.3.4.1-3 and Table 7.6.1.3.5-1 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in non-DRX test.

Table 7.6.1.3.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | CSI-RS |  | CSI-RS |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

7.6.1.3.5-2: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 with per-UE gaps without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.9 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 8+TT |
| Note 2 | dBm/15 KHz | 1, 2 | -102+TT |  |  |  |
| Note 2 | dBm/SCS | 1 | -93+TT |  |  |  |
|  |  | 2 | -90+TT |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -89+TT | -89+TT | -Infinity | -85+TT |
|  |  | 2 | -86+TT | -86+TT | -Infinity | -82+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 8+TT |
|  | dBm/95.04MHz | 1, 2 | -58.56+TT |  | -55.38+TT |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

For UE supporting power class 1, TPSS/SSS_sync_intr = 1600 ms, T SSB_measurement_period_intra= 1600 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intr = 960 ms, T SSB_measurement_period_intra= 960 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of X ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty), where

X = 3202 for UE supporting power class 1,

X = 1922 for UE supporting power class 2, 3 or 4,

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

-Connection diagrams are missing.

- Test tolerance is missing.

7.6.1.4.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under DRX within intra-frequency cell search with gaps requirements. This test will partly verify the TDD intra-frequency cell search requirements in TS 38.133 clause 9.2.5.1 and 9.2.5.2.

7.6.1.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.1.4.

7.6.1.4.4	Test description

7.6.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.1.4.4.1-1.

Table 7.6.1.4.4.1-1: Supported test configurations for NR SA FR2 event-triggered reporting with gap in DRX

| Test Case ID | Description |
| --- | --- |
| 7.6.1.4-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.1.4-2 | 240 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.6.1.4.4.1-2.

Table 7.6.1.4.4.1-2: Initial conditions for NR SA FR2 event-triggered reporting with gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.6.1.4.4.1-3.

2. Message contents are defined in clause 7.6.1.4.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 7.6.1.4.4.1-3: General test parameters for NR SA FR2 event-triggered reporting with gap in DRX

| Parameter | Unit | Config | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2 | PCell (Cell 1) |  |  |
| Neighbour cell |  | 1, 2 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 and Cell 2 |  | One TDD carrier frequency is used for the NR cells. |
| Gap type |  | 1, 2 | Per-UE gaps |  |  |
| Measurement gap repitition periodicity | ms | 1, 2 | 40 |  |  |
| Measurement gap length | ms | 1, 2 | 6 |  |  |
| Measurement gap offset | ms | 1, 2 | 39 |  |  |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1, 2 | CSI-RS.3.2 TDD |  |  |
| A3-Offset | dB | 1, 2 | -6 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| Time To Trigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.1 | DRX.2 |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 10 | 52 |  |

7.6.1.4.4.2	Test procedure

Two cells are deployed in the test, which are FR2 PCell (Cell 1) and a FR2 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 7.6.1.4.4.1-3 and Table 7.6.1.4.4.2-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In Test 1 when DRX cycle = 40 ms is used, UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Timer Alignment Timer to keep the UE uplink time alignment. Furthermore, the UE is allocated with PUSCH resource at every DRX cycle. In Test 2 when DRX = 640 ms is used, the uplink time alignment is not maintained and the UE needs to use RACH to obtain uplink allocation for measurement reporting.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.1.4.5-1 and Table 7.6.1.4.5-2. T1 starts.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.1.4.5-1 and Table 7.6.1.4.5-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than X ms for Test 1 or less than Y ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X and Y is

-	X = 7202, Y = 51202 for UE supporting power class 1,

-	X = 4322, Y = 30722 for UE supporting power class 2, 3 or 4,

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.1.4.4.1-3 as appropriate.

7.6.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.1.4.4.3-1: Common Exception messages for NR SA FR2 event-triggered reporting with gap in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.4-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR2, SMTC.1 and Synchronous cells for configuration 7.6.1.4-2 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.7-1 with Condition DRX.1 for test 1 Table H.3.7-1 with Condition DRX.1 for test 2 |

Table 7.6.1.4.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

7.6.1.4.5	Test requirement

Table 7.6.1.4.4.1-3 , Table 7.6.1.4.5-1 and Table 7.6.1.4.5-2 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in DRX test.

Table 7.6.1.4.5-1: NR Cell specific test parameters for NR SA FR2 event-triggered reporting with gap in DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SCSI-RS |  | SSB |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| TCI state |  | 1, 2 | CSI-RS.Config.0 |  | N/A |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  | OP.1 |  |
| SSB |  | 1 | SSB.3 FR2 |  | SSB.3 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.4 FR2 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |

Table 7.6.1.4.5-2: NR OTA Cell specific test parameters for NR SA FR2 event-triggered reporting with gap in DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  |  |
| Beam AssumptionNote 4 |  | 1,2 | Rough |  |  |  |
|  | dB | 1, 2 | 4+TT | -1.46+TT | -Infinity | -1.46+TT |
| Note 2 | dBm/15 KHz | 1, 2 | -98+TT |  |  |  |
| Note 2 | dBm/SCS | 1 | -89+TT |  |  |  |
|  |  | 2 | -86+TT |  |  |  |
| SS-RSRP | dBm/SCS | 1 | -85+TT | -85+TT | -Infinity | -85+TT |
|  |  | 2 | -82+TT | -82+TT | -Infinity | -82+TT |
|  | dB | 1, 2 | 4+TT | 4+TT | -Infinity | 4+TT |
|  | dBm/95.04MHz | 1 | -54.53+TT | -52.18+TT | -54.53+TT | -52.18+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in TS 38.133 [6] clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms is used, the overall delay measured is defined as the time from the beginning of time period T2, to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report on PUSCH.

For both tests:

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 1: The actual overall delays measured in the test may be up to one DRX cycle higher than the measurement reporting delays above because UE is allowed to delay the initiation of the measurement reporting procedure to the next until the Active Time.

NOTE 2: In order to calculate the rate of correct events the system simulator shall verify that it has received correct Event A3 measurement report.

The overall delay measured when DRX cycle length is 40 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

For UE supporting power class 1, TPSS/SSS_sync_intra = 7200ms, T SSB_measurement_period_intra= 7200 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intra = 2160 ms, T SSB_measurement_period_intra= 2160 ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 40 ms shall be less than a total of X ms, where X is

-	X = 7202 for UE supporting power class 1,

-	X = 4322 for UE supporting power class 2, 3 or 4,

The overall delay measured when DRX cycle length is 640 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

For UE supporting power class 1, TPSS/SSS_sync_intra = 25600 ms, T SSB_measurement_period_intra= 25600 ms,

For UE supporting power class 2, 3 or 4, TPSS/SSS_sync_intra = 15360 ms, T SSB_measurement_period_intra= 15360 ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of X ms, where

-	X = 51202 for UE supporting power class 1,

-	X = 30722 for UE supporting power class 2, 3 or 4,

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 7.6.2	Inter-frequency measurements

#### 7.6.2.0	Minimum conformance requirements for Inter-frequency measurements

Same as clause 5.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3.

#### 7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.1.

7.6.2.1.4	Test description

7.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.1.4.1-1.

Table 7.6.2.1.4.1-1: NR FR2-FR2 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.2.1-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: Void |  |

Table 7.6.2.1.4.1-1: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 5.2 for PC1; 3.5 for other PC |  |

Table 7.6.2.1.4-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.1.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3. The AoA setup for this test is Setup 3 as defined in clause A.9 The UE RX spherical coverage direction has been obtained previously using one of the search procedures as described in Annex I.

7.6.2.1.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 7.6.2.1.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 7.6.2.1.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.1.4.1-2. The TE shall ensure that the NR FR2 cells are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and relative angular offset between active probes are according to Table A.9.3-1. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.1.4.1-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5122 ms for UE supporting power class 1, or 3202 ms for UE supporting other power class for Test 1 and Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. TE shall change the active probes in such way that relative angular offset between active probes differs in the following iteration.

10. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:

- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

12. Repeat step 1-11 for each sub-test in Table 7.6.2.1.4.1-2 as appropriate.

7.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.2.1.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with A3-offset = -11dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #13 Table H.3.1-7 with Condition INTER-FREQ |

Table 7.6.2.1.4.3-2: MeasObjectNR-DEFAULT: SA FR2-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB16 |  |  |
| } |  |  |  |

7.6.2.1.5	Test requirement

Table 7.6.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.9 |  |  |  |  |
|  |  |  |  | AoA1 |  |  | AoA2 |  |
| Assumption for UE beamsNote 7 |  |  | Config 1,2 | Rough |  |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  |  | N/A |  |
| PDSCH/PDCCH TCI state TCI configuration |  |  | Config 1 | CSI-RS.Config.0TCI.State.2 |  |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 |  | -87 | -Infinity | -87 |
| SSB-RP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 |  | -Infinity | -87 |
| BBNote 8 |  | dB | Config 1 | 1.89 | 1.89 |  | -Infinity | 1.89 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 [6] Annex B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 5122 ms in this test for power class UE and 3202 ms for other power classes.

#### 7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

7.6.2.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.2.

7.6.2.2.4	Test description

7.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.2.4.1-1.

Table 7.6.2.2.4.1-1: NR FR2-FR2 event triggered reporting tests in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.2.2-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: Void |  |

Table 7.6.2.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC |  |

Table 7.6.2.2.4-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.2.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3. The AoA setup for this test is Setup 1 as defined in clause A.9

7.6.2.2.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 7.6.2.2.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 7.6.2.2.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.2.4.1-2. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.2.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 7680 ms for UE supporting power class 1, or 4800 ms for UE supporting other power class for Test 1 and Test 3 and 81920 ms for UE supporting power class 1, or 51200 ms for UE supporting other power class for Test 2 and Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.2.4.1-2 as appropriate.

7.6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.2.2.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and and Synchronous cells Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #13 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 for Test 1 and DRX.2 for Test 2 |

7.6.2.2.5	Test requirement

Table 7.6.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Table A.7.6.2.2.1-3: Cell specific test parameters for CA inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.9 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | -104.7 |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

7680 for UE supporting power class 1, or

4800 for UE supporting other power class.

In test 2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

81920 for UE supporting power class 1, or

51200 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.3.

7.6.2.3.4	Test description

7.6.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.3.4.1-1.

Table 7.6.2.3.4.1-1: NR FR2-FR2 event triggered reporting tests in non-DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.2.3-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: Void |  |

Table 7.6.2.3.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | 16 | Applied to NR Cell 2 measurement object |
| A3-Offset | dB | Config 1 | -11 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3s | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 7 for PC1; 4.5 for other PC |  |

Table 7.6.2.3.4-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.3.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3. The AoA setup for this test is Setup 3 as defined in clause A.9. The UE RX spherical coverage direction has been obtained previously using one of the search procedures as described in Annex I.

7.6.2.3.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 7.6.2.3.4.1-2 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 7.6.2.3.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.3.4.1-2. The TE shall ensure that the NR FR2 cells are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and relative angular offset between active probes are according to Table A.9.3-1. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.3.4.1-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 6722 ms for UE supporting power class 1, or 4162 ms for UE supporting other power class for Test 1 and Test 2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. TE shall change the active probes in such way that relative angular offset between active probes differs in the following iteration.10. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

12. Repeat step 1-11 for each sub-test in Table 7.6.2.3.4.1-2 as appropriate.

7.6.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.2.3.4.3-1: Common Exception messages SA inter frequency event triggered reporting with SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with Condition SSB Index and A3-offset = -11dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #13 Table H.3.1-7 with Condition INTER-FREQ |

Table 7.6.2.3.4.3-2: MeasObjectNR-DEFAULT: SA FR2-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB16 |  |  |
| } |  |  |  |

7.6.2.3.5	Test requirement

Table 7.6.2.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.3.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 3 as specified in clause A.9 |  |  |  |  |
|  |  |  |  | AoA1 |  |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  |  | Config 1 | 66 |  |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  |  | N/A |  |
| PDSCH/PDCCH TCI state TCI configuration |  |  | Config 1 | CSI-RS.Config.0TCI.State.2 |  |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -87 |  | -87 | -Infinity | -87 |
| SSB-RP Note 3 |  | dBm/SCS Note5 | Config 1 | -87 | -87 |  | -Infinity | -87 |
| BBNote 8 |  | dB | Config 1 | 1.89 | 1.89 |  | -Infinity | 1.89 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -58.01 | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB-RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 [6] Annex B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [3] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

The UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 6722 ms in this test for power class UE and 4162 ms for other power classes.

#### 7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-		The Test tolerances and Test system uncertainties applicable to this test are undefined.

7.6.2.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.4.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.4.

7.6.2.4.4	Test description

7.6.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.4.4.1-1.

Table 7.6.2.4.4.1-1: NR FR2-FR2 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.2.4-1 | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note 1: Void |  |

Table 7.6.2.4.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.2 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC |  |

Table 7.6.2.4.4-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.4.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3. The AoA setup for this test is Setup 1 as defined in clause A.9

7.6.2.4.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 7.6.2.4.4.1-2 is provided for UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 7.6.2.4.4.1-2 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.4.4.1-2. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.4.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 10080 ms for UE supporting power class 1, or 6240 ms for UE supporting other power class for Test 1 and Test 3 and 107520 ms for UE supporting power class 1, or 66560 ms for UE supporting other power class for Test 2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.4.4.1-2 as appropriate.

7.6.2.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.2.4.4.3-1: Common Exception messages SA inter frequency event triggered reporting with SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with Condition SSB Index and A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #13 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 for Test 1 and DRX.2 for Test 2 |

7.6.2.4.5	Test requirement

Table 7.6.2.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.4.5-1: Cell specific test parameters for CA inter-frequency event triggered reporting with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | Setup 1 as specified in clause A.9 |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  |  |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | M |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1 | -95.7 |  | -95.7 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | Config 1 | -89.7 | -89.7 | -Infinity | -86.7 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
|  |  | dB | Config 1 | 6 | 6 | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note5 | Config 1 | -59.7 | -59.7 | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 6: As observed with 0dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in TS 38.133 Annex B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

10080 for UE supporting power class 1, or

6240 for UE supporting other power class.

In test 2 , the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

107520 for UE supporting power class 1, or

66560 for UE supporting other power class.

In test 1 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

#### 7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.5.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.5.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.5.

7.6.2.5.4	Test description

7.6.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.5.4.1-1.

Table 7.6.2.5.4.1-1: SA FR1-FR2 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.6.2.5-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.2.5-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.6.2.5-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table 7.6.2.5.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | Gap not configured | As specified in TS 38.133 [6]clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 | N/A |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  | As specified in clause A.3.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 5.2 for PC1; 3.5 for other PC | 3 for PC1; 2 for other PC | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 7.6.2.5.4-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.5.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3.	If a UE supports per-FR gap it is only required to pass test 2. Otherwise it is only required to pass test 1.

4. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

7.6.2.5.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 7.6.2.5.4.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 7.6.2.5.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.5.4.1-2. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.5.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5122 ms for UE supporting power class 1, or 3202 ms for UE supporting other power class for Test 1 and 2562 ms for UE supporting power class 1, or 1602 ms for UE supporting other power class for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.5.4.1-2 as appropriate.

7.6.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 7.6.2.5.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ for Test 1 Table H.3.1-2 with Condition INTER-FREQ for Test 2 Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with A4-threshold= -105dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #0 for Test 1 Table H.3.1-7 with Condition INTER-FREQ |

Table 7.6.2.5.4.3-2: MeasObjectNR-DEFAULT: SA FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB6 |  |  |
| } |  |  |  |

7.6.2.5.5	Test requirement

Table 7.6.2.5.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.5.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |  |
| AoA setup |  |  | Config 1,2,3 | N/A |  | Setup 1 as specified in clause A.9 |  |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | CCR.3.1 TDD |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | CCR.3.1 TDD |  |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |  |
|  |  |  | Config 3 | 30 |  | 120 |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1,2,3 |  |  | -Infinity |  | -87 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 |  |  | -Infinity | -87 |  |
|  |  | Note5 | Config 3 |  |  | -Infinity | -87 |  |
| BB Note 8 |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 14.69 |  |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 | A.3.7A |  | -Infinity | -58.01 |  |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

In test 1 with per-UE gap the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

In test 2, without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

2560 for UE supporting power class 1, or

1600 for UE supporting other power class.

In test 1 and 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 5122 ms for power class 1 UE and 3202 ms for other power classes in test 1 and 2562 for power class 1 UE and 1602 ms for other power classes in test 2.

#### 7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.6.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.6.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.6.

7.6.2.6.4	Test description

7.6.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.6.4.1-1.

Table 7.6.2.6.4.1-1: SA FR1-FR2 event triggered reporting tests in DRX supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.6.2.6-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.2.6-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.6.2.6-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table 7.6.2.6.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | Gap not configured |  | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 |  | N/A |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  |  |  | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 |  |  |  | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 |  |  |  | As specified in clause A.3.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  |  |  | As specified in clause A.3.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | DRX is used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC | 8 for PC1; 5 for other PC | 82 for PC1; 52 for other PC | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 7.6.2.6.4-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.6.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3.	If a UE supports per-FR gap it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

4. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.7.6.2.6.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 7.6.2.6.4.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 7.6.2.6.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.6.4.1-2. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.6.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 7682 ms for UE supporting power class 1, or 4802 ms for UE supporting other power class for Test 1 and Test 3 and 81922 ms for UE supporting power class 1, or 51202 ms for UE supporting other power class for Test 2 and Test 4, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.6.4.1-2 as appropriate.

7.6.2.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 7.6.2.6.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ for Test 1 and Test 2 Table H.3.1-2 with Condition INTER-FREQ for Test 3 and Test 4 Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with A4-threshold= -105dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #0 for Test 1 and Test 2 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 for Test 1 and Test 3 and DRX.2 for Test 2 and Test 4 |

Table 7.6.2.6.4.3-2: MeasObjectNR-DEFAULT: SA FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB6 |  |  |
| } |  |  |  |

7.6.2.6.5	Test requirement

Table 7.6.2.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.6.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.9 |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | CCR.3.1 TDD |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
|  |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  | NA Link only, see clause A.3.7A |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1,2 |  |  | -95.7 |  |
|  |  |  | Config 3 |  |  | -95.7 |  |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1,2 |  |  | -Infinity | -86.7 |
|  |  |  | Config 3 |  |  | -Infinity | -86.7 |
|  |  | dB | Config 1,2,3 |  |  | -Infinity | 9 |
|  |  | dB | Config 1,2,3 |  |  | -Infinity | 9 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 |  |  | - | - |
|  |  | dBm/38.16MHz | Config 3 |  |  | - | - |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 |  |  | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone |  |  |  |  |  |  |  |

In test 1 with per-UE gap and in test 3 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

7680 for UE supporting power class 1, or

4800 for UE supporting other power class.

In test 2 with per-UE gap and in test 4 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

81920 for UE supporting power class 1, or

51200 for UE supporting other power class.

In test 1, 2, 3 and 4 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 7682 ms for power class 1 UE and 4802 ms for other power classes in test 1 and test 3 and 81922 for power class 1 UE and 51202 ms for other power classes in test 2 and test 4.

#### 7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.7.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.7.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.2.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.7.

7.6.2.7.4	Test description

7.6.2.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.7.4.1-1.

Table 7.6.2.7.4.1-1: SA FR1-FR2 event triggered reporting tests in non-DRX with SSB time index detection supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.6.2.7-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.2.7-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.6.2.7-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table 7.6.2.7.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two NR carrier frequencies is used |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | Gap not configured | As specified in TS 38.133 [6] clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 39 | N/A |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 |  | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 |  | As specified in clause A.3.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  | As specified in clause A.3.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| a4-Threshold | dBm | Config 1,2,3,4,5,6 | -105 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 7 for PC1; 4.5 for other PC | 3.5 for PC1; 2.5 for other PC | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 7.6.2.7.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.7.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3.	If a UE supports per-FR gap it is only required to pass test 2. Otherwise it is only required to pass test 1.

4. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

7.6.2.7.4.2	Test procedure

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1 measurement gap pattern configuration # 0 as defined in Table 7.6.2.7.4.1-2 is provided for a UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #13 as defined in Table 7.6.2.7.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 7.6.2.7.4.1-2. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction. T1 starts.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.7.4.1-2.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 6722 ms for UE supporting power class 1, or 4162 ms for UE supporting other power class for Test 1 and 3362 ms for UE supporting power class 1, or 2082 ms for UE supporting other power class for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 7.6.2.7.4.1-2 as appropriate.

7.6.2.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 7.6.2.7.4.3-1: Common Exception messages SA inter frequency event triggered reporting with SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ for Test 1 Table H.3.1-2 with Condition INTER-FREQ for Test 2 Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with Condition SSB Index and A4-threshold= -105dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #0 for Test 1 Table H.3.1-7 with Condition INTER-FREQ |

Table 7.6.2.7.4.3-2: MeasObjectNR-DEFAULT: SA FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB6 |  |  |
| } |  |  |  |

7.6.2.7.5	Test requirement

Table 7.6.2.7.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.7.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.9 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | CCR.3.1 TDD |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1,2, 3 |  |  | -Infinity | -87 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1,2 |  |  | -Infinity | -87 |
|  |  | Note5 | Config 3 |  |  | -Infinity | -87 |
| BB Note 8 |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 14.69 |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 | A.3.7A |  | Infinity | -58.01 |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Void Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

In test 1 with per-UE gap , the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6720 for UE supporting power class 1, or

4160 for UE supporting other power class.

In test 2 without the gap, the UE shall send one Event A4 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

3360 for UE supporting power class 1, or

2080 for UE supporting other power class.

In test 1 and 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 6722 ms for power class 1 UE and 4162 ms for other power classes in test 1 and 3362 for power class 1 UE and 2082 ms for other power classes in test 2

#### 7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

7.6.2.8.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event within inter-frequency cell search requirements.

7.6.2.8.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.2.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.2.8.

7.6.2.8.4	Test description

7.6.2.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.2.8.4.1-1.

Table 7.6.2.8.4.1-1: SA FR1-FR2 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.6.2.8-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.6.2.8-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.6.2.8-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |  |

Table 7.6.2.8.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  |  |  | Two NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  |  |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | Gap not configured |  | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 |  | N/A |  |  |
| SMTC-SSB parameters on NR RF Channel 1 |  | Config 1 | SSB.1 FR1 |  |  |  | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 |  |  |  | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 |  |  |  | As specified in clause A.3.1 |
| SMTC-SSB parameters on NR RF Channel 2 |  | Config 1,2,3 | SSB.3 FR2 |  |  |  | As specified in clause A.3.2 |
| offsetMO | dB | Config 1,2,3 | 6 |  |  |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |  |  |
| a4-Threshold | dBm | Config 1,2,3 | -105 |  |  |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.2 | DRX.1 | DRX.2 | DRX is used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  |  |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |  |  |
| T2 | s | Config 1,2,3 | 11 for PC1; 6.5 for other PCTBD | 108 for PC1; 67 for other PCTBD | 11 for PC1; 6.5 for other PCTBD | 108 for PC1; 67 for other PCTBD | PC1 - power class 1 as specified in TS 38.101-2 [3] Table 6.2.1.0 |

Table 7.6.2.8.4-3: Test Environment parameters for SA inter-frequency event triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram |  |  |  |

1.	Message contents are defined in clause 7.6.2.8.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

3.	If a UE supports per-FR, it is only required to pass test 3&4. Otherwise it is only required to pass test 1&2.

4. The UE Rx beam peak direction for Cell 2 has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

7.6.2.8.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 2 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.

In test 1&2 measurement gap pattern configuration # 0 as defined in Table 7.6.2.8.4.1-2 is provided for a UE that does not support per-FR gap and in test 3&4 measurement gap pattern configuration #13 as defined in Table 7.6.2.8.4.1-2 is provided for UE that support per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 3.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.6.2.8.4.1-2. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction. T1 starts.

3.	The SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 7.6.2.8.4.1-2.

6.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 10082 ms for UE supporting power class 1, or 6242 ms for UE supporting other power class for Test 1 and Test 3 and 107522 ms for UE supporting power class 1, or 66562 ms for UE supporting other power class for Test 2 and Test 4 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7.	After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = [(current cell 2 physical cell identity + 1) mod 1008] for next iteration of the test procedure loop.]

9.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11.	Repeat step 1-10 for each sub-test in Table 7.6.2.8.4.1-2 as appropriate.

7.6.2.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause TBD with the following exceptions:

Table 7.6.2.8.4.3-1: Common Exception messages SA inter frequency event triggered reporting with SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ for Test 1 and Test 2 Table H.3.1-2 with Condition INTER-FREQ for Test 3 and Test 4 Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table H.3.1-4 with Condition SSB Index and A4-threshold= -105dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE and Pattern #0 for Test 1 and Test 2 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 for Test 1 and Test 3 and DRX.2 for Test 2 and Test 4 |

Table 7.6.2.8.4.3-2: MeasObjectNR-DEFAULT: SA FR1-FR2 measurement object configuration

| Derivation Path: Table H.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| offsetMO SEQUENCE { |  |  |  |
| rsrpOffsetSSB | dB6 |  |  |
| } |  |  |  |

7.6.2.8.5	Test requirement

Table 7.6.2.8.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.2.8.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1,2,3 | NA |  | Setup 1 as specified in clause A.9 |  |
| Beam AssumptionNote 7 |  |  | Config 1,2,3 | N/A |  | Rough |  |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  | TDD |  |
|  |  |  | Config 2,3 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  |
|  |  |  | Config 2 | TDDConf.1.1 |  | TDDConf.3.1 |  |
|  |  |  | Config 3 | TDDConf.2.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP BW |  | MHz | Config 1 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 2 | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1,2,3 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  |
| measurement channel |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  |
| Channel |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | CCR.3.1 TDD |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | CCR.3.1 TDD |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  |
| in A.3.11.1 and A.3.11.2 |  |  | Config 2,3 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  | 120 |  |
|  |  |  | Config 3 | 30 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz Note5 |  |  |  | -104.7 |  |
| Note2 |  | dBm/SCS Note4 | Config 1,2 |  |  | -95.7 |  |
|  |  |  | Config 3 |  |  | -95.7 |  |
| SSB_RP Note 3 |  | dBm/SCS Note5 | Config 1,2 |  |  | -Infinity | -86.7 |
|  |  |  | Config 3 |  |  | -Infinity | -86.7 |
|  |  | dB | Config 1,2,3 | NA Link only, see clause |  | -Infinity | 9 |
|  |  | dB | Config 1,2,3 | A.3.7A |  | -Infinity | 9 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 |  |  | - | - |
|  |  | dBm/38.16MHz | Config 3 |  |  | - | - |
|  |  | dBm/95.04 MHz Note5 | Config 1,2,3 |  |  | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1,2,3 |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 6: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

In test 1 with per-UE gap and in test 3 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

10080 for UE supporting power class 1, or

6240 for UE supporting other power class.

In test 2 with per-UE gap and in test 4 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

107520 for UE supporting power class 1, or

66560 for UE supporting other power class.

In test 1, 2, 3 and 4 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 10082 ms for power class 1 UE and 6240 ms for other power classes in test 1 and test 3 and 107522 for power class 1 UE and 66562 ms for other power classes in test 2 and test 4.

### 7.6.3	L1-RSRP measurement for beam reporting

#### 7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting

##### 7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting

Same as clause 5.6.3.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.5.3.1, 9.5.4.1 and 9.5.5.1.

##### 7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting

Same as clause 5.6.3.0.2

The normative reference for this requirement is TS 38.133 [6] clauses 9.5.3.1, 9.5.4.2 and 9.5.5.2.

#### 7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-		The Test tolerances and Test system uncertainties applicable to this test are undefined.

-		Antenna diagram is TBD

-		Message content is TBD

- 	Minimum conformance requirements contain [ ] and TBDs (RAN4 Pending)

7.6.3.1.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

7.6.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.1.

7.6.3.1.4	Test description

7.6.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.1.4.1-1.

Table 7.6.3.1.4.1-1: NR SA FR2 SSB-based L1-RSRP measurement in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.3.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.6.3.1-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 7.6.3.1.4.1-2: General test parameters for NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | Off |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 |
| L1-RSRP reporting period | 1~2 | slot | 640 |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 2 |
| Propagation condition | 1~2 |  | AWGN |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.6.3.1.4-3: Test Environment parameters for NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | TBD |  |  |

1.	Message contents are defined in clause 7.6.3.1.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9

7.6.3.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 7.6.3.1.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.4.1.4.1-2.

2.	Set the parameters according to T1 in Table 7.6.3.1.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 640 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 7.6.3.1.5-1. T2 starts.

5.	The UE shall start sending L1-RSRP report including results of both SSB0 and SSB1 every 640 slots, no later than 1680ms for UE supporting power class 1 and 1200 ms for UE supporting power class other than 1, plus 640 slots. If the UE is sending L1-RSRP reports every 640 slots no later than 1760 ms for UE supporting power class 1 in configuration 1 and no later than 1720 ms for UE supporting  power class 1 in configuration 2, no later than 1280 ms for UE supporting  power class other than 1 in configuration 1, no later than 1240 ms for UE supporting  power class other than 1 in configuration 2 from the beginning of time period T2 until the end of time period T2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.6.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.3.1.4.3-1: Common Exception messages NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

7.6.3.1.5	Test requirement

Table 7.6.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.1.5-1: Cell specific test parameters for NR SA FR2 SSB-based L1-RSRP measurement in non-DRX

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.9 |  |  |  |
| Note2 | 1~2 | dBm/15kHz | -105+TT |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96+TT |  |  |  |
|  | 2 |  | -93+TT |  |  |  |
|  | 1~2 | dB | 0+TT | 0+TT | -Infinity | 9+TT |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -96+TT | -96+TT | -Infinity | -87+TT |
|  | 2 |  | -93+TT | -93+TT | -Infinity | -84+TT |
| Io Note3 | 1 | dBm/95.04MHz | -67.5+TT | -67.5+TT | -71.1+TT | -60.7+TT |
|  | 2 |  | -67.5+TT | -67.5+TT | -71.1+TT | -60.7+TT |
|  | 1~2 | dB | 0+TT | 0+TT | -Infinity | 9+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 640 slots. No later than X ms plus 640 slots from the beginning of time period T2, UE shall send L1-RSRP report including the results for both SSB#0 and SSB#1 while meeting the accuracy requirements defined in clause 10.1.20.1, where X is

-	1680 for UE supporting power class 1

-	1200 for UE supporting power class 2,3 or 4.

The reported L1-RSRP value shall include the Rx antenna gain in the range of [-10 ~ +20] dB.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-		The Test tolerances and Test system uncertainties applicable to this test are undefined.

-		Antenna diagram is TBD

-		Message content is TBD

- 	Minimum conformance requirements contain [ ] and TBDs (RAN4 Pending)

7.6.3.2.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

7.6.3.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.2.

7.6.3.2.4	Test description

7.6.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.2.4.1-1.

Table 7.6.3.2.4.1-1: NR SA FR2 SSB-based L1-RSRP measurement in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.3.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.6.3.2-2 | NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 7.6.3.2.4.1-2: General test parameters for NR SA FR2 SSB-based L1-RSRP measurement in DRX

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| DRX configuration | 1~2 |  | DRX.3 |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity | 1~2 |  | ssb-Index-RSRP |
| Number of reported RS | 1~2 |  | 2 |
| L1-RSRP reporting period | 1~2 | slot | 640 |
| T1 | 1~2 | s | 5 |
| T2 | 1~2 | s | 3 |
| Propagation condition | 1~2 |  | AWGN |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~2 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.6.3.2.4-3: Test Environment parameters for NR SA FR2 SSB-based L1-RSRP measurement in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | TBD |  |  |

1. Message contents are defined in clause 7.6.3.2.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9

7.6.3.2.4.2	Test procedure

Same as in 7.6.3.1.4.2 with the following exception:

5.	The UE shall start sending L1-RSRP report including results of both SSB0 and SSB1 every 640 slots, no later than 2880ms for UE supporting power class 1 and 1920ms for UE supporting power class other than 1, plus 640 slots. If the UE is sending L1-RSRP reports every 640 slots no later than 2960 ms for UE supporting power class 1 in configuration 1 and no later than 2920 ms for UE supporting  power class 1 in configuration 2, no later than 2000 ms for UE supporting  power class other than 1 in configuration 1, no later than 1960 ms for UE supporting  power class other than 1 in configuration 2 from the beginning of time period T2 until the end of time period T2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

7.6.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.3.2.4.3-1: Common Exception messages NR SA FR2 SSB-based L1-RSRP measurement in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

7.6.3.2.5	Test requirement

Table 7.6.3.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.2.5-1: Cell specific test parameters for NR SA FR2 SSB-based L1-RSRP measurement in DRX

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  |  | Setup 1 according to A.9 |  |  |  |
| Note2 | 1~2 | dBm/15kHz | -105+TT |  |  |  |
| Note2 | 1 | dBm/SSB SCS | -96+TT |  |  |  |
|  | 2 |  | -93+TT |  |  |  |
|  | 1~2 | dB | 0+TT | 0+TT | -Infinity | 9+TT |
| SSB RSRP Note3 | 1 | dBm/SSB SCS | -96+TT | -96+TT | -Infinity | -87+TT |
|  | 2 |  | -93+TT | -93+TT | -Infinity | -84+TT |
| Io Note3 | 1 | dBm/95.04MHz | -67.5+TT | -67.5+TT | -71.1+TT | -60.7+TT |
|  | 2 |  | -67.5+TT | -67.5+TT | -71.1+TT | -60.7+TT |
|  | 1~2 | dB | 0+TT | 0+TT | -Infinity | 9+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 640 slots. No later than X ms plus 640 slots from the beginning of time period T2, UE shall send L1-RSRP report including the results for both SSB#0 and SSB#1 while meeting the accuracy requirements defined in clause 10.1.20.1, where X is

-	2880 for UE supporting power class 1

-	1920 for UE supporting power class 2,3 or 4.

The reported L1-RSRP value shall include the Rx antenna gain in the range of [-10 ~ +20] dB.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-		The Test tolerances and Test system uncertainties applicable to this test are undefined.

-		Antenna diagram is TBD

-		Message content is TBD

-		Test procedure is TBD

7.6.3.3.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.2.

7.6.3.3.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.6.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.3.

7.6.3.3.4	Test description

7.6.3.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.3.4.1-1.

Table 7.6.3.3.4.1-1: NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.3.4-1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table 7.6.3.3.4.1-2: General test parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | Off |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 26 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.6.3.3.4-3: Test Environment parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | TBD |  |  |

1.	Message contents are defined in clause 7.6.3.3.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9

7.6.3.3.4.2	Test procedure

TBD

7.6.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.3.3.4.3-1: Common Exception messages NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

7.6.3.3.5	Test requirement

Table 7.6.3.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.3.5-1: Cell specific test parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.9 |  |
| Note1 | 1 | dBm/15kHz | -105+TT |  |
| Note1 | 1 | dBm/SSB SCS | -95.97+TT |  |
|  | 1 | dB | 0+TT | 9+TT |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97+TT | -86.97+TT |
| Io Note2 | 1 | dBm/95.04MHz | -63.97+TT | -57.47+TT |
|  | 1 | dB | 0+TT | 9+TT |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 480ms from the beginning of the test, the UE shall send L1-RSRP report at slot 26 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the accuracy requirements defined in clause 10.1.20.1.

For absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1, the UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table 7.6.3.3.5-2.

For relative accuracy of CSI-RS0 compared with CSI-RS1, the UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in TS 38.133 in Table 10.1.20.2.2-1.

Table 7.6.3.3.5-2: L1-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| CSI-RS0 | CSI-RS _RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP0 +δ + Gmax |
| CSI-RS1 | CSI-RS _RP1 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP1 +δ + Gmax |
| Note 1: CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the CSI-RS n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.20.2.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-		The Test tolerances and Test system uncertainties applicable to this test are undefined.

-		Antenna diagram is TBD

-		Message content is TBD

-		Test procedure TBD

7.6.3.4.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.2.

7.6.3.4.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR2 and long DRX cycle.

7.6.3.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.6.3.4.

7.6.3.4.4	Test description

7.6.3.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.6.3.4.4.1-1.

Table 7.6.3.4.4.1-1: NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 7.6.3.4-1 | NR 120 kHz CSI-RS SCS, 100 MHz bandwidth, TDD duplex mode |

Table 7.6.3.4.4.1-2: General test parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD Configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
| CSI-RS configuration | 1 |  | CSI-RS.3.3 TDD |
| OCNG Patterns | 1 |  | OP.1 |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1 |  | TCI.State.2 |
| DRX configuration | 1 |  | DRX.3 |
| reportConfigType | 1 |  | aperiodic |
| reportQuantity | 1 |  | cri-RSRP |
| Number of reported RS | 1 |  | 2 |
| qcl-Info | 1 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1 |  | 26 |
| Propagation condition | 1 |  | AWGN |
| T1 | 1 | s | 5 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 7.6.3.4.4-3: Test Environment parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.3.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | TBD |  |  |

1.	Message contents are defined in clause 7.6.3.4.4.3.

2. The AoA setup for this test is Setup 1 as defined in clause A.9

7.6.3.4.4.2	Test procedure

TBD

7.6.3.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 7.6.3.4.4.3-1: Common Exception messages NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

7.6.3.4.5	Test requirement

Table 7.6.3.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 7.6.3.4.5-1: Cell specific test parameters for NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration | 1 |  | Setup 1 according to A.9 |  |
| Note1 | 1 | dBm/15kHz | -105+TT |  |
| Note1 | 1 | dBm/SSB SCS | -95.97+TT |  |
|  | 1 | dB | 0+TT | 9+TT |
| CSI-RS RSRP Note2 | 1 | dBm/SSB SCS | -95.97+TT | -86.97+TT |
| Io Note2 | 1 | dBm/95.04MHz | -63.97+TT | -57.47+TT |
|  | 1 | dB | 0+TT | 9+TT |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 1440ms from the beginning of the test, the UE shall send L1-RSRP report at slot 26 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1 while meeting the accuracy requirements defined in clause 10.1.20.1.

For absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1, the UE is deemed to meet the requirement if the reported L1-RSRP is in the range shown in Table 7.6.3.4.5-2.

For relative accuracy of CSI-RS0 compared with CSI-RS1, the UE is deemed to meet the requirement if the difference in reported L1-RSRP meets the requirements in Table 10.1.20.2.2-1.

Table 7.6.3.4.5-2: L1-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| CSI-RS0 | CSI-RS _RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP0 +δ + Gmax |
| CSI-RS1 | CSI-RS _RP1 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP1 +δ + Gmax |
| Note 1: CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the CSI-RS n under consideration Note 2: δ is the RSRP absolute accuracy requirement from Table 10.1.20.2.1-1, selected according to the Io used in the test Note 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## 7.7	Measurement performance requirements

### 7.7.1	SS-RSRP

#### 7.7.1.0	Minimum conformance requirements

##### 7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements

Same as in clause 5.7.1.0.1.

##### 7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements

Same as in clause 5.7.1.0.2.

#### 7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- TT analysis is missing.

- Test procedure is FFS

7.7.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP measurement accuracy is within the specified limits for all bands.

7.7.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.1.1.

7.7.1.1.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the NR FR2 neighbour cell.

7.7.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.1.1.4.1-1.

Table 7.7.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.1.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode, LTE FDD |
| 7.7.1.1-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.1.1.4.1-2.

Table 7.7.1.1.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | FFS | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.1.1.4.1-3.

2.	Message contents are defined in clause 7.7.1.1.4.3.

3.	There are two intra-frequency cells specified in the test, where Cell 1 is the NR FR2 serving cell and Cell 2 is the neighbour cell on the same NR FR2 carrier and the target cell for the SS-RSRP measurements.

7.7.1.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.1.1.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	FFS

7.7.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | TBD |
| Default RRC messages and information elements contents exceptions | TBD |

7.7.1.1.5	Test requirement

FFS

#### 7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- TT analysis is missing.

- Test procedure is FFS

7.7.1.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP measurement accuracy is within the specified limits for all bands.

7.7.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.1.2.

7.7.1.2.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the inter-frequency NR FR2 neighbour cell.

7.7.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.1.2.4.1-1.

Table 7.7.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.1.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode, LTE FDD |
| 7.7.1.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.1.2.4.1-2.

Table 7.7.1.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | FFS | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.1.2.4.1-3.

2.	Message contents are defined in clause 7.7.1.2.4.3.

3.	There are two inter-frequency cells specified in the test, where Cell 1 is the serving cell on an NR FR2 carrier and Cell 2 is the neighbour cell on a different NR FR2 carrier and the target cell for the SS-RSRP measurements.

7.7.1.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.1.2.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	FFS

7.7.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | TBD |
| Default RRC messages and information elements contents exceptions | TBD |

7.7.1.2.5	Test requirement

FFS.

#### 7.7.1.3	Inter-frequency measurements between FR1 and FR2

##### 7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy

Editor’s Note:

- Test tolerance analysis is missing.

- Connection diagram is TBD.

- Table 7.7.1.3.1.5-3 of reported value is FFS

- RAN4 dependency: Some test parameters are still TBD.

7.7.1.3.1.1	Test Purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy with FR1 serving cell and FR2 target cell.

7.7.1.3.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

7.7.1.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.1.3.

7.7.1.3.1.4	Test description

7.7.1.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.1.3.1.4.1-1.

Table 7.7.1.3.1.4.1-1: Applicable NR configurations for FR1 inter-frequency SS-RSRP accuracy test

| Config | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 7.7.1.3.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 7.7.1.3.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |  |
| 7.7.1.3.1-3 | NR 30kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.1.3.1.4.1-2.

Table 7.7.1.3.1.4.1-2: Initial conditions for NR SA FR1-FR2 SS-RSRP absolute measurement accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.7.1.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 7.7.1.3.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR FR2 neighbour cell (the target cell for SS-RSRP measurements) on a different frequency than the PCell. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

7.7.1.3.1.4.2	Test procedure

Same as test procedure in clause 6.7.1.1.1.4.2 with the following changes:

-	Table 6.7.1.1.1.5-1 is replaced by Table 7.7.1.3.1.5-1 and 7.7.1.3.1.5-2;

-	Table 6.7.1.2.1.5-2 is replaced by Table 7.7.1.3.1.5-3 for test 1 and Table 7.7.1.3.1.5-4 for test 2.

7.7.1.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.1.3.1.4.3-1: Common Exception messages for NR SA FR1-FR2 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table H.3.1-7 with Condition INTER-FREQ |

Table 7.7.1.3.1.4.3-2: ReportConfigNR-DEFAULT

Derivation Path: TS 38.508-1 [14] Table 4.6.3-142 with Condition PERIODICAL

7.7.1.3.1.5	Test requirement

Table 7.7.1.3.1.5-1 and Table 7.7.1.3.1.5-2 define the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 7.7.1.3.1.5-1 and 7.7.1.3.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 7.7.1.3.1.5-3.

Table 7.7.1.3.1.5-1: SS-RSRP inter-frequency test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 | 100: NRB,c = 66 | 10: NRB,c = 52 | 100: NRB,c = 66 |
|  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| Duplex mode | 1 |  | FDD | TDD | FDD | TDD |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD configuration | 1 |  | N/A | TDDConf.3.1 | N/A | TDDConf.3.1 |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 | SSB.1 FR2 | SSB.1 FR1 | SSB.1 FR2 |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.3 ULBWP.1.3 |  | DLBWP.1.3 ULBWP.1.3 |  |
| TRS Configuration | 1~3 |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| PDCCH/PDSCH TCI Configuration | 1~3 |  | TCI.State.2 |  | TCI.State.2 |  |
| SMTC configuration | 1~3 |  | SMTC.1 |  | SMTC.1 |  |
| Time offset between Cell 1 and Cell 2 | 1~3 | s | 3 |  | 3 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Propagation condition | 1~3 | - | NA Link only, see TS 38.133 [6] clause A.3.7A | AWGN | NA Link only, see TS 38.133 [6] clause A.3.7A | AWGN |
| Antenna configuration | 1~3 | - |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |  |  |  |

Table 7.7.1.3.1.5-2: SS-RSRP inter-frequency OTA related test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 NOTE 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
|  | 1~4 | dBm/15kHz | NA Link only, see 38.133 [6] clause A.3.7A | TBD | NA Link only, see 38.133 [6] clause A.3.7A | NA |
|  | 1,2 | dBm/SSB SCS |  | TBD |  | NA |
|  | 3,4 |  |  | TBD |  | NA |
|  | 1~4 | dB |  | TBD |  | NA |
| SS-RSRPNote1 | 1,2 | dBm/SCS |  | TBD |  | As in 38.133 [6] Table B.2.3-2 |
|  | 3,4 |  |  | TBD |  | As in 38.133 [6] Table B.2.3-2 |
| IoNote1 | 1~4 | dBm/95.04MHz |  | TBD |  | SS-RSRP+28.98 |
|  | 1~4 | dB |  | TBD |  | NA |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: No additional noise is added by the test system in Test 2. |  |  |  |  |  |  |

Table 7.7.1.3.1.5-3: SS-RSRP inter-frequency absolute accuracy requirements for the reported values for Test 1

FFS

Table 7.7.1.3.1.5-4: SS-RSRP inter-frequency absolute accuracy requirements for the reported values for Test 2

FFS

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 7.7.1.3.2	Void

### 7.7.2	SS-RSRQ

#### 7.7.2.0	Minimum conformance requirements

##### 7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements

Same as in clause 5.7.2.0.1.

##### 7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements

Same as in clause 5.7.2.0.2.

#### 7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- TT analysis is missing.

- Test procedure is FFS

7.7.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRQ measurement accuracy is within the specified limits for all bands.

7.7.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.2.1.

7.7.2.1.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the intra-frequency NR FR2 neighbour cell.

7.7.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.2.1.4.1-1.

Table 7.7.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.2.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode, LTE FDD |
| 7.7.2.1-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.2.1.4.1-2.

Table 7.7.2.1.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | FFS | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.2.1.4.1-3.

2.	Message contents are defined in clause 7.7.2.1.4.3.

3.	There are two intra-frequency cells specified in the test, where Cell 1 is the NR FR2 serving cell and Cell 2 is the neighbour cell on the same NR FR2 carrier and the target cell for the SS-RSRQ measurements.

7.7.2.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.2.1.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	FFS

7.7.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | TBD |
| Default RRC messages and information elements contents exceptions | TBD |

7.7.2.1.5	Test requirement

FFS

#### 7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- TT analysis is missing.

- Test procedure is FFS

7.7.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP measurement accuracy is within the specified limits for all bands.

7.7.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.7.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.2.2.

7.7.2.2.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the inter-frequency NR FR2 neighbour cell.

7.7.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.2.2.4.1-1.

Table 7.7.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.2.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode, LTE FDD |
| 7.7.2.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.2.2.4.1-2.

Table 7.7.2.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | FFS | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.2.2.4.1-3.

2.	Message contents are defined in clause 7.7.2.2.4.3.

3.	There are two inter-frequency cells specified in the test, where Cell 1 is the serving cell on an NR FR2 carrier and Cell 2 is the neighbour cell on a different NR FR2 carrier and the target cell for the SS-RSRQ measurements.

7.7.2.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.2.2.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	FFS

7.7.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.2.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | TBD |
| Default RRC messages and information elements contents exceptions | TBD |

7.7.2.2.5	Test requirement

FFS

### 7.7.3	SS-SINR

#### 7.7.3.0	Minimum conformance requirements

##### 7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements

Same as in clause 5.7.3.0.1.

##### 7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements

Same as in clause 5.7.3.0.2.

#### 7.7.3.1	NR SA FR2 SS-SINR measurement accuracy

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- TT analysis is missing.

- Test procedure is FFS

7.7.3.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

7.7.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting ss-SINR-Meas.

7.7.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.3.1.

7.7.3.1.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the intra-frequency NR FR2 neighbour cell.

7.7.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.3.1.4.1-1.

Table 7.7.3.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.3.1-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode, LTE FDD |
| 7.7.3.1-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.3.1.4.1-2.

Table 7.7.3.1.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | FFS | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.3.1.4.1-3.

2.	Message contents are defined in clause 7.7.3.1.4.3.

3.	There are two intra-frequency cells specified in the test, where Cell 1 is the NR FR2 serving cell and Cell 2 is the neighbour cell on the same NR FR2 carrier and the target cell for the SS-SINR measurements.

7.7.3.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.3.1.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	FFS

7.7.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | TBD |
| Default RRC messages and information elements contents exceptions | TBD |

7.7.3.1.5	Test requirement

FFS

#### 7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- Message contents are not complete.

- TT analysis is missing.

- Test procedure is FFS

7.7.3.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

7.7.3.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting ss-SINR-Meas.

7.7.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.7.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.7.3.2.

7.7.3.2.4	Test description

Two cells are configured in this test: Cell 1 is the NR FR2 serving cell and Cell 2 is the inter-frequency NR FR2 neighbour cell.

7.7.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 7.7.3.2.4.1-1.

Table 7.7.3.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 7.7.3.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, FDD duplex mode, LTE FDD |
| 7.7.3.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 7.7.3.2.4.1-2.

Table 7.7.3.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | FFS | As specified in TS 38.508-1 [14] Annex A. |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 7.7.3.2.4.1-3.

2.	Message contents are defined in clause 7.7.3.2.4.3.

3.	There are two inter-frequency cells specified in the test, where Cell 1 is the serving cell on an NR FR2 carrier and Cell 2 is the neighbour cell on a different NR FR2 carrier and the target cell for the SS-SINR measurements.

7.7.3.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Table 7.7.3.2.5-1 as appropriate.

3. 	The SS shall transmit an RRCReconfiguration message on Cell 1.

4. 	The UE shall transmit an RRCReconfigurationComplete message.

5.	FFS

7.7.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.7.3.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | TBD |
| Default RRC messages and information elements contents exceptions | TBD |

7.7.3.2.5	Test requirement

FFS

# 8	E-UTRA – NR inter-RAT with E-UTRA serving cell

This section contains test scenarios for E-UTRA – NR inter-RAT with the serving cell in E-UTRA. The NR cells can be in FR1, FR2 or both.

For conformance testing involving FR2 test cases in this specification, the UE under test shall be pre-configured with UL Tx diversity schemes disabled to account for single polarization System Simulator (SS) in the test environment. The UE under test may transmit with dual polarization.

## 8.1	Void

## 8.2	RRC_IDLE state mobility

### 8.2.1	Inter-RAT cell re-selection

#### 8.2.1.0	Minimum conformance requirements

##### 8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection

[TS 36.133, clause 4.2.1]

When the UE is in either Camped Normally state or Camped on Any Cell state on a cell, the UE shall attempt to detect, synchronise, and monitor intra-frequency, inter-frequency and inter-RAT cells indicated by the serving cell. UE measurement activity is also controlled by measurement rules defined in TS36.304, allowing the UE to limit its measurement activity.

[TS 36.133, clause 4.2.2]

The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds when the UE is not configured with eDRX_IDLE cycle, and at least every Thigher_priority_search = MAX(60 * Nlayers, one eDRX_IDLE cycle) when UE is configured with eDRX_IDLE cycle, where Nlayers is the total number of configured higher priority E-UTRA, UTRA FDD, UTRA TDD, CDMA2000 1x, HRPD and NR carrier frequencies and is additionally increased by one if one or more groups of GSM frequencies is configured as a higher priority.

[TS 36.133, clause 4.2.2.5.6]

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT NR layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in TS 36.133 clause 4.2.2.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT NR layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT NR layers shall be the same as that defined below for lower priority RATs.

The requirements in this section apply for inter-RAT NR measurements. When the measurement rules indicate that inter-RAT NR cells are to be measured, the UE shall measure SS-RSRP and SS-RSRQ of detected NR cells in the neighbour frequency list at the minimum measurement rate specified in this section.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, a carrier is indicated to meet high speed requirement if highSpeedInterRAT-NR-r16 is configured and the carrier to be measured is configured with highSpeedCarrierNR-r16. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE is required to meet non high speed requirements no matter whether highSpeedInterRAT-NR-r16 or highSpeedCarrierNR-r16 is configured or not. The parameter NNR_carrier is the number of configured NR carriers indicated to meet non high speed requirement in the neighbour frequency list. NNR_carrier_HST is the number of configured carriers for reselection indicated to meet high speed requirements. The parameter NNR_carrier for a UE configured with idle mode DC measurements (while T331 is running) is the combined number of configured NR carriers indicated to meet non high speed requirement in the neighbour frequency list, and NR carriers configured for idle mode DC measurements.

Note: combined total number means that if a carrier is indicated in the neighbour frequency list and additionally a carrier configured for idle mode DC measurements, it only counts as one carrier.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured NR cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

The UE shall be able to evaluate whether a newly detectable inter-RAT NR cell meets the reselection criteria defined in TS 36.304 [1] within NNR_carrier_HST * Tdetect, NR_HST + NNR_carrier  * Tdetect, NR

when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 5 dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or 6 dB in FR1 or 7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or 4 dB in FR1 and 4 dB in FR2 for SS-RSRQ reselections based on absolute priorities.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this section shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects on an inter-RAT NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT NR cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

Cells which have been detected shall be measured at least every NNR_carrier_HST * Tmeasure, NR_HST + NNR_carrier  * Tmeasure, NR  when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

For a cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that an already identified inter-RAT NR cell has met reselection criterion defined in TS 36.304 [1] within NNR_carrier_HST * Tevaluate, NR_HST + NNR_carrier  * Tevaluate, NR  when Treselection = 0 as specified in Table 8.2.1.0.1-1 and  table 8.2.1.0.1-2 provided that the reselection criteria is met by a margin of at least 5dB in FR1 or 6.5 dB in FR2 for reselections based on ranking or 6 dB in FR1 or 7.5 dB in FR2 for SS-RSRP reselections based on absolute priorities or 4 dB in FR1 and 4 dB in FR2 for SS-RSRQ reselections based on absolute priorities.

If Treselection timer has a non-zero value and the inter-RAT NR cell is satisfied with the reselection criteria which are defined in TS 36.304 [1], the UE shall evaluate this NR cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 8.2.1.0.1-1: Tdetect,NR, TmeasureNR, and Tevaluate,NR

| DRX cycle length [s] | Scaling Factor (N1) |  | Tdetect,NR [s] (number of DRX cycles) | Tmeasure,NR [s] (number of DRX cycles) | Tevaluate,NR [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |  |  |
| 0.32 | 1 | 8 | 11.52 x 1.5 x N1 (36 x 1.5 x N1) | 1.28 x 1.5 x N1 (4 x 1.5 x N1) | 5.12 x 1.5 x N1 (16 x 1.5 x N1) |
| 0.64 |  | 5 | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 4 | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 3 | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| NOTE 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. |  |  |  |  |  |

Table 8.2.1.0.1-2: Tdetect,NR_HST, TmeasureNR_HST, and Tevaluate,NR_HST for UE configured with highSpeedInterRAT-NR-r16

| DRX cycle length [s] | Tdetect,NR_HST [s] (number of DRX cycles) | Tmeasure,NR_HST [s] (number of DRX cycles) | Tevaluate,NR_HST [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 4.16 x M2 (13 x M2)Note 2 | 0.64 x M3 (2 x M3)Note 2 | 0.96 x M4 (3 x M4) Note 2 |
| 0.64 | 7.68 (12)) | 1.28 (2) | 1.92 (3) |
| 1.28 | 12.8(10) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: FR2 high speed requirements are not specified. Note 2: M2=1.5, M3=2 and M4=2 if SMTC periodicity of measured intra-frequency cell > 40 ms; otherwise M2=1. |  |  |  |

The normative reference for this requirement is TS 36.133 clause 4.2.1, 4.2.2, 4.2.2.5.6.

#### 8.2.1.1	E-UTRA – NR FR1 cell re-selection to higher priority NR target cell

8.2.1.1.1	Test purpose

The purpose of this test is to verify the requirement for the E-UTRAN to NR inter-RAT cell reselection requirements specified in TS 36.133 [23] clause 4.2.2.5.6.

8.2.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

8.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.2.1.1.

8.2.1.1.4	Test description

There are two cells configured in this test, the E-UTRA Cell 1 and NR Cell 2. E-UTRA Cell 1 is the PCell and NR Cell 2 is the neighbour cell. This test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. E-UTRA Cell 1 is already identified by the UE prior to the start of the test. Cell 2 is of higher priority than cell 1.

8.2.1.1.4.1	Initial conditions

This test shall be tested under any of the test configuration in Table 8.2.1.1.4.1-1.

Table 8.2.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.2.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.2.1.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test requirement and the DUT according to the parameters in Table 8.2.1.1.4.1-2.

Table 8.2.1.1.4.1-2: Initial conditions for E-UTRA – NR FR1 cell re-selection to higher priority NR target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.2.1.1.4.1-3.

2.	Message contents are defined in clause 8.2.1.1.4.3.

3.	The test scenario comprises of one NR cell and one E-UTRAN cell. E-UTRA Cell 1 is the PCell and Cell 2 is the neighbour cell. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1 and Cell 2 is configured according to Annex C.1.1 and C.1.2.

Table 8.2.1.1.4.1-3: General test parameters for E-UTRA – NR FR1 cell re-selection to higher priority NR target cell

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE camps on cell 2 in the initial phase |
|  | Neighbour cells |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| T1 end condition | Active cell |  |  | Cell1 | During T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  |  | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3, 4, 5, 6 | Cell2 | The UE shall perform reselection to cell 2 during T3 |
|  | Neighbour cell |  | 1, 2, 3, 4, 5, 6 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3, 4, 5, 6 | 1, 2 | E-UTRAN radio channel (1) and NR radio channel (2) are used for this test |
| Time offset between cells |  |  | 1, 4 | 3 ms | Asynchronous cells |
|  |  |  | 2, 5 | 3 s | Synchronous cells |
|  |  |  | 3, 6 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3, 4, 5, 6 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2, 3, 4, 5, 6 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1, 2, 3, 4, 5, 6 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| T1 |  | s | 1, 2, 3, 4, 5, 6 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3, 4, 5, 6 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3, 4, 5, 6 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

8.2.1.1.4.2	Test procedure

Two cells are deployed in the test, which are one E-UTRA PCell (E-UTRA Cell 1) and a neighbour cell (NR Cell 2). The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. E-UTRA Cell 1 is already identified by the UE prior to the start of the test. NR Cell 2 is of higher priority than E-UTRA Cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a Registration procedure.

1.	Ensure the UE is in State RRC_IDLE with generic procedure parameters Connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 8.2.1.1.5-1 and 8.2.1.1.5-2, T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, E-UTRA Cell 1.

4.	If the UE responds on lower priority cell, E-UTRA Cell 1 during time duration T1 within 8 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”.

5.	If the UE has re-selected E-UTRA Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, skip to step 11

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 8.2.1.1.5-1. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE.

7.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 8.2.1.1.5-1.

8.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, NR Cell 2.

9.	If the UE responds on higher priority cell, NR Cell 2 during time duration T3 within 68 seconds from the beginning of time period T3, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”.

10.	If the UE has re-selected Cell 2 within T3, after the re-selection or when T3 expires, skip to step 12.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 2, continue with step 11.

11.	Switch off and on the UE and ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR according to TS 38.508-1 [14] clause 4.5 in Cell 2.

12.	Repeat step 3-11 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

8.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.2.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions |  |

Table 8.2.1.1.4.3-2: SystemInformationBlockType3

| Derivation Path: TS 36.508 Table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 25 | Actual value = 50 dB |  |
| threshServingLow | 22 | Actual value = 44 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.4.3-3: SystemInformationBlockType24

| Derivation Path: TS 36.508, Table 4.4.3.3-20 with Condition FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType24-r15 ::= SEQUENCE { |  |  |  |
| carrierFreqListNR-r15 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 entry |  |  |
| subcarrierSpacingSSB-r15[1] | kHz15 |  | Configuration 1,2,4,5 |
|  | kHz30 |  | Configuration 3,6 |
| threshX-High-r15[1] | 24 | Actual value = 48 dB |  |
| threshX-Low-r15[1] | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| q-RxLevMin-r15[1] | -70 | Actual value = -140 dBm | Configuration 1,2,4,5 |
|  | -69 | Actual value = -137 dBm, Round down | Configuration 3,6 |
| } |  |  |  |

Table 8.2.1.1.4.3-4: SIB2

| Derivation Path: TS 38.508-1, Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | Actual value = 50 dB |  |
| threshServingLowP | 22 | Actual value = 44 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 8.2.1.1.4.3-5: SIB5

| Derivation Path: TS 38.508-1, Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF SEQUENCE { | 1 entry |  |  |
| threshX-High | 24 | Actual value = 48 dB |  |
| threshX-Low | 25 | Actual value = 50 dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.2.1.1.5	Test requirement

Tables 8.2.1.1.4.1-3, 8.2.1.1.5-1 and 8.2.1.1.5-2 define the primary level settings including test tolerances for higher priority E-UTRA cell re-selection test case.

Table 8.2.1.1.5-1: NR Cell specific test parameters for E-UTRA – NR FR1 cell re-selection to higher priority NR target cell

| Parameter | Unit | Test configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| TDD configuration |  | 1, 4 | N/A |  |  |
|  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  | 3, 6 | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  | 3, 6 | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  | 3, 6 | CR.2.1 TDD |  |  |
| RMC CORESET Reference Channel |  | 1, 4 | CCR.1.1 FDD |  |  |
|  |  | 2, 5 | CCR.1.1 TDD |  |  |
|  |  | 3, 6 | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| SMTC configuration |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  | 1, 4 | SSB.1 FR1 |  |  |
|  |  | 2, 5 | SSB.1 FR1 |  |  |
|  |  | 3, 6 | SSB.2 FR1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3, 4, 5, 6 | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2, 4, 5 | -140 |  |  |
|  |  | 3, 6 | -137 |  |  |
| Pcompensation | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qhysts | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3, 4, 5, 6 | SS-RSRP |  |  |
|  | dB | 1, 4 | -3.65 | -infinity | 13.55 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| Note2 | dBm/SCS | 1, 4 | -99.90 | -98 | -98 |
|  |  | 2, 5 | -99.90 | -98 | -98 |
|  |  | 3, 6 | -96.90 | -95 | -95 |
| Note2 | dBm/15 kHz | 1, 4 | -99.90 | -98 | -98 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
|  | dB | 1, 4 | -3.65 | -infinity | 13.55 |
|  |  | 2, 5 |  |  |  |
|  |  | 3, 6 |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1, 4 | -103.55 | -infinity | -84.45 |
|  |  | 2, 5 | -103.55 | -infinity | -84.45 |
|  |  | 3, 6 | -100.54 | -infinity | -81.44 |
| Io | dBm/9.36 MHz | 1, 4 | -70.39 | -70.05 | -56.31 |
|  | dBm/9.36 MHz | 2, 5 | -70.39 | -70.05 | -56.31 |
|  | dBm/38.16 MHz | 3, 6 | -64.29 | -63.94 | -50.21 |
| Treselection | s | 1, 2, 3, 4, 5, 6 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Threshx, highP | dB | 1, 2, 3, 4, 5, 6 | 48 |  |  |
| Threshserving, lowP | dB | 1, 2, 3, 4, 5, 6 | 44 |  |  |
| Threshx, lowP | dB | 1, 2, 3, 4, 5, 6 | 50 |  |  |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

Table 8.2.1.1.5-2: E-UTRA Cell specific test parameters for for E-UTRA – NR FR1 cell re-selection to higher priority NR target cell

| Parameter | Unit | Cell 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 |
| E-UTRA RF Channel number |  | 1 |  |  |
| BWchannel | MHz | 10 |  |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2, 3; OP.2 FDD for test configuration 4, 5, 6 |  |  |
| PBCH_RA | dB | 0 |  |  |
| PBCH_RB | dB |  |  |  |
| PSS_RA | dB |  |  |  |
| SSS_RA | dB |  |  |  |
| PCFICH_RB | dB |  |  |  |
| PHICH_RA | dB |  |  |  |
| PHICH_RB | dB |  |  |  |
| PDCCH_RA | dB |  |  |  |
| PDCCH_RB | dB |  |  |  |
| PDSCH_RA | dB |  |  |  |
| PDSCH_RB | dB |  |  |  |
| OCNG_RANote 1 | dB |  |  |  |
| OCNG_RBNote 1 | dB |  |  |  |
| Qrxlevmin | dBm | -140 |  |  |
| Note 2 | dBm/15 kHz | -98 |  |  |
| RSRP Note 3 | dBm/15 KHz | -84 | -84 | -82.45 |
|  | dB | 14 | 14 | 15.55 |
|  | dB | 14 | 14 | 15.55 |
| TreselectionEUTRAN | S | 0 |  |  |
| SnonintrasearchP | dB | 50 |  |  |
| Threshx, highP | dB | 48 |  |  |
| Threshserving, lowP | dB | 44 |  |  |
| Threshx, lowP | dB | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

The cell reselection delay to a higher priority NR cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility on cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR + TSI-NR,

Where:

Thigher_priority_search is the higher priority search period, Thigher_priority_search = 60 ms according to TS 36.133 [23], clause 4.2.2;

Tevaluate, NR 	is the evaluation time for NR Cell, Tevaluate, NR = 6400 ms according TS 36.133 [23], Table 4.2.2.5.6-1;

TSI-NR is the maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; TSI-NR = 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority NR cell and 7.68 s for the cell re-selection delay to a lower priority cell in the test case, which we allow 8 s.

#### 8.2.1.2	E-UTRA – NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

-	Cell Configuration table in Annex E is undefined.

-	Antenna diagram is TBD

-	Message content is TBD

-	Test case applicability is TBD

-	Test procedure is TBD

8.2.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the E-UTRAN to NR inter-RAT cell reselection requirements specified in TS 36.133 [23] clause 4.2.2.5.6.

8.2.1.2.2	Test applicability

This test applies to all types of NR UE configured with highSpeedInterRAT-NR-r16 from Release 16 onwards.

8.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.2.1.2.

8.2.1.2.4	Test description

8.2.1.2.4.1	Initial conditions

This test shall be tested under any of the test configuration in Table 8.2.1.2.4.1-1.

Table 8.2.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.2.1.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.2.1.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.2.1.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test requirement and the DUT according to the parameters in Table 8.2.1.2.4.1-2.

Table 8.2.1.2.4.1-2: TBD

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.2.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

TBD

Table 8.2.1.2.4.1-3: TBD

8.2.1.2.4.2	Test procedure

TBD

8.2.1.2.4.3	Message contents

TBD

8.2.1.2.5	Test requirement

TBD

## 8.3	RRC_CONNECTED state mobility

### 8.3.1	Inter-RAT cell handover

#### 8.3.1.0	Minimum conformance requirements

##### 8.3.1.0.1	Minimum conformance requirements for E-UTRA – NR FR1 handover

[TS 36.133, clause 5.3.4.2]

When the UE receives a RRC message implying inter-RAT handover to the UE shall be ready to start the transmission of the uplink PRACH channel in NR within Dhandover seconds from the end of the last TTI containing the RRC command. Dhandover is defined as

Dhandover = TRRC_procedure_delay + Tinterruption

Where:

TRRC_procedure_delay: it is the RRC procedure delay which is [50] ms.

Tinterruption: it is the time between end of the last TTI containing the RRC command on the PDSCH in E-UTRAN and the time the UE starts transmission of the PRACH in NR, excluding TRRC_procedure_delay. Tinterruption is defined in TS 36.133 clause 5.3.4.3.

[TS 36.133, clause 5.3.4.3]

When inter-RAT handover to NR is commanded, the interruption time shall be less than Tinterruption

Tinterruption = Tsearch + TIU + Trs+20 ms

Where:

Tsearch is the time required to search the target cell when the target cell is not already known when the handover command is received by the UE. If the target cell is known, then Tsearch = 0 ms. If the target cell is an unknowncell and target cell Es/Iot ≥ [-2] dB, then Tsearch = 3▪Trs + 2 ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213.

NOTE:	The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

Trs is the SMTC period of the target NR cell if the UE has been provided with an SMTC configuration for the target cell prior to, or in the handover command, otherwise Trs is the target cell SSB transmission period, if such is provided. If the UE is not provided with an SMTC configuration or SSB transmission period, the requirement in this section is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If UE is provided with both SMTC configuration and SSB transmission period the requirement shall be based on SMTC periodicity.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in clause 8.1.2.4.21 and 8.1.2.4.22.

The normative reference for this requirement is TS 38.133 [6] clause 5.3.4.2 and 5.3.4.3.

#### 8.3.1.1	E-UTRA – NR FR1 handover with known target cell

8.3.1.1.1	Test purpose

The purpose of this test is to verify the E-UTRAN to NR FR1 handover requirements as specified in TS 36.133 [23] clause 5.3.4.

8.3.1.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements.

8.3.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.3.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.3.1.1.

8.3.1.1.4	Test description

The test comprises of one E-UTRA carrier and one NR carrier. There are two cells and one cell on each carrier. Cell 1 is the E-UTRAN PCell and Cell 2 is an inter-RAT NR neighbour cell. The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 of TS 36.133 [23] is configured before T2 begins to enable inter-RAT frequency monitoring.

8.3.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.3.1.1.4.1-1.

Table 8.3.1.1.4.1-1: Supported test configurations for E-UTRA – NR FR1 handover with known target cell

| Configuration | Description |
| --- | --- |
| 8.3.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.3.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.3.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.3.1.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.3.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.3.1.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test requirement and the DUT according to the parameters in Table 8.3.1.1.4.1-2.

Table 8.3.1.1.4.1-2: Initial conditions for E-UTRA – NR FR1 handover with known target cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.3.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.3.1.1.4.1-3.

2.	Message contents are defined in clause 8.3.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 8.3.1.1.4.1-3: General test parameters for E-UTRA – NR FR1 handover with known target cell

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  |  | 2 | 1 E-UTRAN carrier frequency is used in the test |
| Initial conditions | Active cell |  | Cell 1 | E-UTRAN cell |
|  | Neighbouring cell |  | Cell 2 | NR cell |
| Final condition | Active cell |  | Cell 2 |  |
| NR measurement quantity |  |  | SS-RSRP |  |
| E-UTRAN measurement quantity |  |  | RSRP |  |
| b2-Threshold1 |  | dBm | -83 | Absolute E-UTRAN RSRP threshold for event B2 |
| b2-Threshold2NR |  | dBm | As specified in Table 8.3.1.1.5-2 | Absolute NR SS-RSRP threshold for event B2 |
| Hysteresis |  | dB | 0 |  |
| TimeToTrigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| DRX |  |  | OFF | Non-DRX test |
| Access Barring Information |  | - | Not sent | No additional delays in random access procedure |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Gap pattern configuration Id |  |  | 0 | As specified in Table 8.1.2.1-1 started before T2 starts [23] |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |
| T3 |  | s | 1 |  |

8.3.1.1.4.2	Test procedure

The test comprises of one E-UTRA carrier and one NR carrier. There are two cells and one cell on each carrier. Cell 1 is the E-UTRAN and Cell 2 is an inter-RAT NR neighbour cell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.3.1.1.4.1-3, 8.3.1.1.5-1 and 8.3.1.1.5-2, respectively.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report. Gap pattern configuration with id #0 as specified in Table 8.1.2.1-1 of TS 36.133 [23] is configured before T2 begins to enable inter-RAT frequency monitoring.

A RRC message implying handover shall be sent to the UE during period T2 after the UE has reported Event B2. The start of T3 is the instant when the last TTI containing the RRC message implying handover is sent to the UE. The handover message shall contain Cell 2 as the target cell.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.3.1.1.5-1 and Table 8.3.1.1.5-2 respectively, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.3.1.1.5-1 and Table 8.3.1.1.5-2 respectively. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event B2.

7.	SS shall transmit a MobilityFromEUTRACommand message implying handover to Cell 2.

8.	The start of T3 is the instant when the last TTI containing the MobilityFromEUTRACommand message implying handover is sent to the UE, at that instant the SS shall switch the power setting from T2 to T3 as specified in Table 8.3.1.1.5-1 and Table 8.3.1.1.5-2.

9.	The UE shall transmit an RRCReconfigurationComplete message on Cell 2.

10.	If the UE transmits the uplink PRACH channel to Cell 2 less than 112 ms from the beginning of time period T3 then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

11.	After T3 expires,

The SS could switch the parameters setting from T3 to T1 as specified in Table 8.3.1.1.5-1 and Table 8.3.1.1.5-2 respectively.

If the UE handover back to E-UTRA Cell 1, then T1 starts, skip to step 3. Otherwise, switch off  and on the UE. Then ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3. E-UTRA Cell 1 is the active cell.

Or, switch off and on the UE Then ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3. E-UTRA Cell 1 is the active cell.

12.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved

8.3.1.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.3.1.1.4.3-1: Common Exception messages for E-UTRA – NR FR1 handover with known target cell

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.3-2; Table H.3.4-1a Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapUE Table H.3.4-5 with Condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 8.3.1.1-1, 8.3.1.1-2, 8.3.1.1-4 and 8.3.1.1-5 | Table H.3.4-6 with Condition SSB.1 FR1, SMTC.1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.3.1.1-3 and 8.3.1.1-6 | Table H.3.4-6 with Condition SSB.2 FR1, SMTC.1 and Asynchronous cells |

8.3.1.1.5	Test requirement

Table 8.3.1.1.4.1-3, 8.3.1.1.5-1 and 8.3.1.1.5-2 define the primary level settings including test tolerances for E-UTRA – NR FR1 handover with known target cell.

Table 8.3.1.1.5-1: E-UTRAN PCell specific test parameters for E-UTRA – NR FR1 handover with known target cell

| Parameter | Unit | Configuration | Cell 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PRACH ConfigurationNote2 |  | 1, 2, 3 | 4 |  |  |
|  |  | 4, 5, 6 | 53 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote3 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote3 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote4 |  |  |  |  |  |
| OCNG_RBNote4 |  |  |  |  |  |
| NocNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 7 | 5.45 | 5.45 |
| Ês/IotNote6 | dB | 1, 2, 3, 4, 5, 6 | 7 | 5.45 | 5.45 |
| RSRPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -91 | -92.55 | -92.55 |
| SCH_RPNote6 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -91 | -92.55 | -92.55 |
| IoNote6 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -62.43 | -63.68 | -63.68 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix Note7 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [23]. Note 2: PRACH configurations are specified in table 5.7.1-2 and table 5.7.1-3 in TS 36.211 [23]. Note 3: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [15] respectively. Note 4: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 5: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 6: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 7: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |  |

Table 8.3.1.1.5-2: NR neighbour cell specific test parameters for E-UTRA – NR FR1 handover with known target cell

| Parameter |  | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 |
| RF channel number |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |
| Duplex mode |  |  | 1, 4 | FDD |  |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |  |
| TDD Configuration |  |  | 2, 5 | TDDConf.1.1 |  |  |
|  |  |  | 3, 6 | TDDConf.1.2 |  |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |
| PDSCH reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |  |
| CORSET reference channel |  |  | 1, 4 | CR.1.1 FDD |  |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |  |
| OCNG patternNote1 |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |
| BWP | Initial DL BWP |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |
| SSB configuration |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |  |
| b2-Threshold2NR |  | dBm | 1, 2, 4, 5 | -106 |  |  |
|  |  |  | 3, 6 | -103 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  | dBm/15 KHz | 1, 2, 3, 4, 5, 6 | -98 |  |  |
| NocNote2 |  | dBm/SCS | 1, 2, 4, 5 | -98 |  |  |
|  |  |  | 3, 6 | -95 |  |  |
| Ês/Noc |  | dB | 1, 2, 3, 4, 5, 6 | -infinity | 1.55 | 1.55 |
| Ês/IotNote3 |  | dB | 1, 2, 3, 4, 5, 6 | -infinity | 1.55 | 1.55 |
| SS-RSRPNote3 |  | dBm/SCS | 1, 2, 4, 5 | -infinity | -96.45 | -96.45 |
|  |  |  | 3, 6 | -infinity | -93.44 | -93.44 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -70.05 | -66.19 | -66.19 |
|  |  | dBm/38.16 MHz | 3, 6 | -63.94 | -60.09 | -60.09 |
| Propagation condition |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall start to transmit the PRACH to Cell 2 less than 260 ms from the beginning of time period T3.

The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

-	RRC procedure delay = 50 ms and is specified in TS36.331 [23].

-	Tinterrupt = 62 ms in the test; Tinterrupt is defined in TS36.133 [23] clause 5.3.4.3.

This gives a total of 112 ms.

The rate of correct handovers observed during repeated tests shall be at least 90%.

## 8.4	Measurement procedures

### 8.4.1	SFTD measurement delay

#### 8.4.1.0	Minimum conformance requirements

##### 8.4.1.0.1	Minimum conformance requirements for E-UTRA – NR FR1 SFTD measurement delay

[TS 36.133, clause 8.1.2.4.25.1]

The UE shall perform inter-RAT SFTD measurement and report SFTD result with/without SS-RSRP after the network requests with reportSFTD-Meas set to neighborCells. The overall delay includes RRC procedure delay to be defined in clause 11.2 in TS 36.331 and SFTD measurement reporting delay in TS 36.133 clause 8.1.2.4.25.3.

[TS 36.133, clause 8.1.2.4.25.2]

The requirements on SFTD measurement delay defined in this section are applicable under the side condition SCH Ês/Iot ≥ -3 dB for the NR cell. Depending on configuration, the SFTD measurement may be carried out with or without the support of configured measurement gaps. In the current release, indication on whether to carry out the SFTD measurement with or without measurement gaps is implicit and depending on whether measurement gaps are configured.

The UE shall be able to detect, identify and measure SFTD of up to 3 of the strongest NR cells on the carrier frequency provided in the SFTD measurement configuration. Further depending on the SFTD measurement configuration, the UE shall additionally report SS-RSRP for the one or more NR cells. The UE may or may not be configured with cellsForWhichToReportSFTD. The UE does not expect cellsForWhichToReportSFTD to change during an ongoing SFTD measurement.

When no measurement gaps are provided, the UE shall be capable of finding the NR cell regardless of its SSB position in the SMTC period. The SFTD measurement shall be conducted with sustained connection to the E-UTRA PCell and activated SCell(s), however, the UE may be allowed to cause a certain amount of interruptions for reconfiguration of the radio receiver, as specified in TS 36.133 clause 7.35.

When measurement gaps are provided, the UE shall be capable of finding the NR cell under the additional condition that the SSB at least occasionally falls within the measurement gap.

When no MCG DRX is used, the UE shall be capable of determining SFTD within a physical layer measurement period of Tmeasure_SFTD1 as follows:

-	For SFTD measurements without measurement gaps, and without additional SS-RSRP reporting:

-	For NR carrier in FR1: Tmeasure_SFTD1 = [14] SMTC periods

-	For SFTD measurements in measurement gaps, and without additional SS-RSRP reporting:

-	For NR carrier in FR1: Tmeasure_SFTD1 = [Nfreq × 8 × max(MGRP, SMTC period)]

-	For SFTD measurements without measurement gaps, and with additional SS-RSRP reporting:

-	For NR carrier in FR1: Tmeasure_SFTD1 = [19] SMTC periods

-	For SFTD measurements in measurement gaps, and with additional SS-RSRP reporting:

-	For NR carrier in FR1: Tmeasure_SFTD1 = [Nfreq × 13 × max(MGRP, SMTC period)]

Where Nfreq is the number of carriers monitored in measurement gaps.

When MCG DRX is used, the same Tmeasure_SFTD1 as for non-DRX applies, but the reporting delay depends on the DRX cycle length in use.

In case an NR PSCell is added, the UE shall terminate the inter-RAT SFTD measurement.

In case PCell is changed due to handover, the UE shall terminate the inter-RAT SFTD measurement.

[TS 36.133, clause 8.1.2.4.25.3]

The SFTD measurement reporting delay is defined as the time between a command that will trigger an SFTD measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When the UE is configured to perform SRS carrier-based switching, an additional delay can be expected.

The SFTD measurement reporting delay shall be less than Tmeasure_SFTD1 defined in TS 36.133, clause 8.1.2.4.25.2.

#### 8.4.1.1	E-UTRA – NR FR1 SFTD measurement delay in non-DRX

8.4.1.1.1	Test purpose

The purpose of this test is topartly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell in FR1 is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [23] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and no DRX is configured.

8.4.1.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements.

8.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.1.1.

8.4.1.1.4	Test description

Two carriers are used in this test: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier with the NR neighbour cell (Cell 2).

This test consists of a single time period of duration T1. Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 2 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report. No measurement gaps are provided and no DRX is configured in this test.

8.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.1.1.4.1-1.

Table 8.4.1.1.4.1-1: Supported test configurations for E-UTRA – NR FR1 SFTD measurement delay in non-DRX

| Config | Description |
| --- | --- |
| 8.4.1.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.1.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.1.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.1.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.1.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.1.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.1.1.4.1-2.

Table 8.4.1.1.4.1-2: Initial conditions for E-UTRA – NR FR1 SFTD measurement delay in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.1.1.4.1-3.

2.	Message contents are defined in clause 8.4.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup for this test. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 8.4.1.1.4.1-3: General test parameters for E-UTRA – NR FR1 SFTD measurement delay in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One NR FR1 carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| SMTC-SSB parameters |  | Config 1,4 | SSB.1 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2,3,4,5,6 | OFF |  | DRX is not used |
| Frame time offset between serving and neighbour cells | ms | Config 1,2,3,4 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | s | Config 5,6 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| T1 | s | Config 1,2,3,4,5,6 | 1 |  |  |

8.4.1.1.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general test parameters are given in Table 8.4.1.1.4.1-3. The test has two subtests with different frame time offset and SFN offset. Cell specific test parameters for the E-UTRA PCell and neighbour cell are given in Table 8.4.1.1.5-1 and 8.4.1.1.5-2, respectively.

In the measurement control information, it is indicated to the UE that inter-RAT SFTD measurement on Cell 2 is used. This test consists of single time period with time duration of T1. Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only.

Configure the frame time offset and SFN offset according to Test 1 in Table 8.4.1.1.4.1-3.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 2 according to T1 in Table 8.4.1.1.5-2, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

6.	UE shall transmit a MeasurementReport message triggered by SFTD measurement reporting. If the overall delays measured from the beginning of time period T1 is less than 297 ms then the number of successful tests is increased by one. If the UE fails to report within the overall delays measured requirement then the number of failure tests is increased by one.

7.	After the SS receive the MeasurementReport message in step 6) or when T1 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

Or

-	switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11	Set the frame time offset and SFN offset for Test 2 according to Table 8.4.1.1.4.1-3 and repeat steps 1 to 10.

8.4.1.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.1.1.4.3-1: Common Exception messages for E-UTRA – NR FR1 SFTD measurement delay in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, SFTD and GAPLESS |
| Specific message contents exceptions for Test Configuration 8.4.1.1-1 and 8.4.1.1-4 | Table H.3.4-6 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.1.1-2 and 8.4.1.1-5 | Table H.3.4-6 with Condition SSB.1 FR1, SMTC.1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.1.1-3 and 8.4.1.1-6 | Table H.3.4-6 with Condition SSB.2 FR1, SMTC.1 and Asynchronous cells |

8.4.1.1.5	Test requirement

Table 8.4.1.1.4.1-3, 8.4.1.1.5-1 and 8.4.1.1.5-2 define the primary level settings including test tolerances for E-UTRA – NR FR1 SFTD measurement delay in non-DRX.

Table 8.4.1.1.5-1: E-UTRA Cell specific test parameters for E-UTRA – NR FR1 SFTD measurement delay in non-DRX

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note5 | dBm/15 kHz | -87 |
| SCH_RP Note5 | dBm/15 kHz | -87 |
| Io Note5 | dBm/Ch BW | -59.13+10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table 8.4.1.1.5-2: NR Cell specific test parameters for E-UTRA – NR FR1 SFTD measurement delay in non-DRX

| Parameter | Unit | Test configuration | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |
| Duplex mode |  | Config 1,4 | FDD |
|  |  | Config 2,3,5,6 | TDD |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |
|  |  | Config 2,5 | 10: NRB,c = 52 |
|  |  | Config 3,6 | 40: NRB,c = 106 |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |
|  |  | Config 3,6 | TDDConf.2.1 |
| OCNG Pattern defined in A.3.2.1.1 |  | Config 1,2,3,4,5,6 | OP.1 |
| SMTC configuration defined in A.3.2.11.1 and A.3.2.11.2 |  | Config 1,4 | SMTC.2 |
|  |  | Config 2,3,5,6 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |
|  |  | Config 3,6 | 30 |
| EPRE ratio of PSS to SSS | dB | Config 1,2,3,4,5,6 | 0 |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 | dB |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 | dB |  |  |
| Noc Note2 | dBm/15kHz |  | -98 |
| Noc Note2 | dBm/SCS | Config 1,2,4,5 | -98 |
|  |  | Config 3,6 | -95 |
| SS-RSRP Note 3, 4 | dBm/SCS | Config 1,2,4,5 | -94 |
|  |  | Config 3,6 | -91 |
| Ês/Iot | dB | Config 1,2,3,4,5,6 | 4 |
| Ês/Noc | dB | Config 1,2,3,4,5,6 | 4 |
| Io Note 3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 |
|  | dBm/38.16MHz | Config 3,6 | -58.49 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T1, to the moment the UE send one SFTD measurement triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = TRRC_procedure_delay + Tmeasure_SFTD1

TRRC_procedure_delay = 15 ms, is the RRC procedure delay for RRC reconfiguration, which is defined in TS 36.331 [2] Clause 11.2

Tmeasure_SFTD1 = 280 ms, is the SFTD measurement report delay defined in TS 36.133 [23] clause 8.1.2.4.25 and 8.1.2.4.26 for FDD and TDD E-UTRA Cell 1, respectively.

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 297 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 8.4.1.2	E-UTRA – NR FR1 SFTD measurement delay in DRX

8.4.1.2.1	Test purpose

The purpose of this test is:

-	To partly verify that measurement reporting delay for SFTD between E-UTRA PCell and inter-RAT NR neighbour cell in FR1 is within the requirements stated in clauses 8.1.2.4.25 and 8.1.2.4.26 of TS 36.133 [23] for E-UTRA FDD and TDD, respectively, when no measurement gaps are provided and DRX is configured.

8.4.1.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle.

8.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.1.2.

8.4.1.2.4	Test description

Two carriers are used in this test: one E-UTRA carrier with the PCell (Cell 1), and one NR carrier with the NR neighbour cell (Cell 2).

This test consists of a single time period of duration T1. Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only. The point in time at which the UE receives, at the UE antenna connector(s), a RRC message containing a measurement configuration for SFTD measurements on RF channel 2 defines the start of time duration T1. Following the start of T1 the UE shall detect Cell 2, determine the SFN and frame time difference of Cell 2 relative to Cell 1, and send a measurement report. No measurement gaps are provided and DRX is configured in this test.

8.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.1.2.4.1-1.

Table 8.4.1.2.4.1-1: Supported test configurations for E-UTRA – NR FR1 SFTD measurement delay in DRX

| Config | Description |
| --- | --- |
| 8.4.1.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.1.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.1.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.1.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.1.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.1.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.1.2.4.1-2.

Table 8.4.1.2.4.1-2: Initial conditions for E-UTRA – NR FR1 SFTD measurement delay in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.1.2.4.1-3.

2.	Message contents are defined in clause 8.4.1.2.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup for this test. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

Table 8.4.1.2.4.1-3: General test parameters for E-UTRA – NR FR1 SFTD measurement delay in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One E-UTRAN carrier frequencies is used. |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |  | One NR FR1 carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4,5,6 | Cell 1 |  | Cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4,5,6 | Cell 2 |  | Cell 2 is on NR RF channel number 1. |
| SMTC-SSB parameters |  | Config 1,4 | SSB.1 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
|  |  | Config 2,5 | SSB.1 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
|  |  | Config 3,6 | SSB.2 FR1 |  | As specified in TS 38.133 [6] clause A.3.10.1 |
| CP length |  | Config 1,2,3,4,5,6 | Normal |  | Applicable to both cells. |
| DRX |  | Config 1,2,3,4,5,6 | DRX.4 |  | DRX configuration as specified in TS 38.133 [6] clause A.3.3.4 |
| Frame time offset between serving and neighbour cells | ms | Config 1,2,3,4 | 3 | 7 | Asynchronous cells. The timing of Cell 2 relative to the timing of Cell 1. |
|  | s | Config 5,6 | 3 |  | Synchronous cells. |
| SFN offset between serving and neighbour cells |  | Config 1,2,3,4,5,6 | 0 | 1 | SFN of Cell 2 relative to SFN of Cell 1. |
| T1 | s | Config 1,2,3,4,5,6 | 1 |  |  |

8.4.1.2.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general test parameters are given in Table 8.4.1.2.4.1-3. The test has two subtests with different frame time offset and SFN offset. Cell specific test parameters for the E-UTRA PCell and neighbour cell are given in Table 8.4.1.2.5-1 and 8.4.1.2.5-2, respectively.

In the measurement control information, it is indicated to the UE that inter-RAT SFTD measurement on Cell 2 is used. This test consists of single time period with time duration of T1. Prior to the start of time duration T1, the UE is connected to Cell 1 and configured to carry out intra-frequency measurements only.

Configure the frame time offset and SFN offset according to Test 1 in Table 8.4.1.2.4.1-3.

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters of Cell 2 according to T1 in Table 8.4.1.2.5-2, T1 starts.

3.	The SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

6.	UE shall transmit a MeasurementReport message triggered by SFTD measurement reporting. If UE sends a measurement before the earliest DRX activity time following upon 297ms after the beginning of time duration T1 then the number of successful tests is increased by one, otherwise the number of failure tests is increased by one.

7.	After the SS receive the MeasurementReport message in step 6) or when T1 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod1008) for next iteration of the test procedure loop.

9.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

Or

-	switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11	Set the frame time offset and SFN offset for Test 2 according to Table 8.4.1.2.4.1-3 and repeat steps 1 to 10.

8.4.1.2.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.1.2.4.3-1: Common Exception messages for E-UTRA – NR FR1 SFTD measurement delay in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, SFTD and GAPLESS Table H.3.7-2 with Condition DRX.4 |
| Specific message contents exceptions for Test Configuration 8.4.1.1-1 and 8.4.1.1-4 | Table H.3.4-6 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.1.1-2 and 8.4.1.1-5 | Table H.3.4-6 with Condition SSB.1 FR1, SMTC.1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.1.1-3 and 8.4.1.1-6 | Table H.3.4-6 with Condition SSB.2 FR1, SMTC.1 and Asynchronous cells |

8.4.1.2.5	Test requirement

Table 8.4.1.2.4.1-3, 8.4.1.2.5-1 and 8.4.1.2.5-2 define the primary level settings including test tolerances for E-UTRA – NR FR1 SFTD measurement delay in DRX.

Table 8.4.1.2.5-1: E-UTRA Cell specific test parameters for E-UTRA – NR FR1 SFTD measurement delay in DRX

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note5 | dBm/15 kHz | -87 |
| SCH_RP Note5 | dBm/15 kHz | -87 |
| Io Note5 | dBm/Ch BW | -59.13+10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table 8.4.1.2.5-2: NR Cell specific test parameters for E-UTRA – NR FR1 SFTD measurement delay in DRX

| Parameter | Unit | Test configuration | Cell 2 |
| --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4,5,6 | 1 |
| Duplex mode |  | Config 1,4 | FDD |
|  |  | Config 2,3,5,6 | TDD |
| BWchannel | MHz | Config 1,4 | 10: NRB,c = 52 |
|  |  | Config 2,5 | 10: NRB,c = 52 |
|  |  | Config 3,6 | 40: NRB,c = 106 |
| TDD configuration |  | Config 2,5 | TDDConf.1.1 |
|  |  | Config 3,6 | TDDConf.2.1 |
| OCNG Pattern defined in A.3.2.1.1 |  | Config 1,2,3,4,5,6 | OP.1 |
| SMTC configuration defined in A.3.2.11.1 and A.3.2.11.2 |  | Config 1,4 | SMTC.2 |
|  |  | Config 2,3,5,6 | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing | kHz | Config 1,2,4,5 | 15 |
|  |  | Config 3,6 | 30 |
| EPRE ratio of PSS to SSS | dB | Config 1,2,3,4,5,6 | 0 |
| EPRE ratio of PBCH DMRS to SSS | dB |  |  |
| EPRE ratio of PBCH to PBCH DMRS | dB |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 | dB |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 | dB |  |  |
| Noc Note2 | dBm/15kHz |  | -98 |
| Noc Note2 | dBm/SCS | Config 1,2,4,5 | -98 |
|  |  | Config 3,6 | -95 |
| SS-RSRP Note 3, 4 | dBm/SCS | Config 1,2,4,5 | -94 |
|  |  | Config 3,6 | -91 |
| Ês/Iot | dB | Config 1,2,3,4,5,6 | 4 |
| Ês/Noc | dB | Config 1,2,3,4,5,6 | 4 |
| Io Note 3 | dBm/9.36MHz | Config 1,2,4,5 | -64.59 |
|  | dBm/38.16MHz | Config 3,6 | -58.49 |
| Propagation Condition |  | Config 1,2,3,4,5,6 | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T1, to the moment the UE send one SFTD measurement triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = TRRC_procedure_delay + Tmeasure_SFTD1

TRRC_procedure_delay = 15 ms, is the RRC procedure delay for RRC reconfiguration, which is defined in TS 36.331 [2] Clause 11.2

Tmeasure_SFTD1 = 280 ms, is the SFTD measurement report delay defined in TS 36.133 [23] clause 8.1.2.4.25 and 8.1.2.4.26 for FDD and TDD E-UTRA Cell 1, respectively. When DRX is used, the same Tmeasure_SFTD1 as for non-DRX applies.

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 297 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 8.4.2	Inter-RAT measurements

#### 8.4.2.0	Minimum conformance requirements

##### 8.4.2.0.1	Minimum conformance requirements for E-UTRA – NR event-triggered measurement

The UE shall be able to identify new inter-RAT E-UTRAN  NR cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified inter-RAT cells if carrier frequency information is provided by the PCell, even if no explicit neighbour list with physical layer cell identities is provided.

When measurement gaps are scheduled, the UE shall be able to identify a new detectable cell within Tidentify_irat_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise, UE shall be able to identify a new detectable inter-RAT frequency cell within Tidentify_irat_with_index. The UE shall be able to identify a new detectable inter-RAT SS block of an already detected cell within Tidentify_irat_without_index.

Tidentify_irat_without_index = (TPSS/SSS_sync_irat + T SSB_measurement_period_irat) ms

Tidentify_irat_with_index = (TPSS/SSS_sync_irat + T SSB_measurement_period_irat + TSSB_time_index_irat) ms

Where:

TPSS/SSS_sync_irat: it is the time period used in PSS/SSS detection given in table 8.4.2.0.1-1 and table 8.4.2.0.1-2.

TSSB_time_index_irat: it is the time period used to acquire the index of the SSB being measured given in table 8.4.2.0.1-3 and table 8.4.2.0.1-4.

T SSB_measurement_period_irat: equal to a measurement period of SSB based measurement given in table 8.4.2.0.1-5 and table 8.4.2.0.1-6.

Mpss/sss_sync_irat: For a UE supporting FR2 power class 1, Mpss/sss_sync_ira=64 samples. For a UE supporting FR2 power class 2 (vehicle mounted), Mpss/sss_sync_irat=40 samples. For a UE supporting FR2 power class 3 (handheld), Mpss/sss_sync_irat=40 samples. For a UE supporting FR2 power class 4, Mpss/sss_syn_iratc=40 samples.

MSSB_index_irat: For a UE supporting power class 1, MSSB_index_irat =40 samples. For a vehicle mounted UE supporting power class 2 (vehicle mounted), Mpss/sss_sync_irat=24 samples.For a UE supporting power class 3 (handheld), MSSB_index_irat =24 samples. For a UE supporting power class 4, Mmeas_period_irat =24 samples.

Mmeas_period_irat: For a UE supporting FR2 power class 1, Mmeas_period_irat =64 samples. For a vehicle mounted UE supporting FR2 power class 2 (vehicle mounted), Mpss/sss_sync_irat=40 samples.For a UE supporting FR2 power class 3 (handheld), Mmeas_period_irat =40 samples. For a UE supporting FR2 power class 4, Mmeas_period_irat =40 samples.

Nfreq is defined in TS 36.133 [23] clause 8.1.2.1.1.

For per-FR measurement gap capable UE, when serving cells are in E-UTRA and measurement objects are only in FR2,

-	UE can perform such measurements without gap, and

-	UE fulfils the requirements for FR2 measurement objects based on effective MGRP = 20 ms.

Table 8.4.2.0.1-1: Time period for PSS/SSS detection, (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | max[600ms, 8 x max(MGRP, SMTC period)] x Nfreq |
| DRX cycle ≤ 320ms | max[600ms, ceil(8x1.5) x max(MGRP, SMTC period, DRX cycle)] x Nfreq |
| DRX cycle > 320ms | 8 x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] section 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 8.4.2.0.1-1A: Time period for PSS/SSS detection for UE configured with highSpeedInterRAT-r16 (Frequency range FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, 8 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle < 320ms | Max(600ms, ceil( 8 × M) × max(MGRP, SMTC period, DRX cycle)) ×Nfreq |
| DRX cycle ≥ 320ms | 8× DRX cycle ×Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [3]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 of TS 38.133 [3] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms |  |

Table 8.4.2.0.1-2: Time period for PSS/SSS detection, (Frequency range FR2)

| Condition NOTE1,2 | TPSS/SSS_sync_irat |
| --- | --- |
| No DRX | Max(600ms, Mpss/sss_sync_irat x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(600ms, (1.5 x Mpss/sss_sync_irat) x max(MGRP, SMTC period, DRX cycle)) x Nfreq |
| DRX cycle > 320ms | Mpss/sss_sync_irat x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] section 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 8.4.2.0.1-3: Time period for time index detection (Frequency range FR1)

| Condition NOTE1,2 | TSSB_time_index_irat |
| --- | --- |
| No DRX | Max(120ms, 3 x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(120ms, ceil(3 x 1.5) x max(MGRP, SMTC period,DRX cycle)) x Nfreq |
| DRX cycle > 320ms | 3 x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] section 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 8.4.2.0.1-3A: Time period for time index detection for UE configured with highSpeedInterRAT-r16 (Frequency range FR1)

| Condition NOTE1,2 | TSSB_time_index_irat |
| --- | --- |
| No DRX | Max(120ms, 3 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle < 320ms | Max(120ms, Ceil(3 M) Max(MGRP, SMTC period, DRX cycle)) Nfreq |
| DRX cycle ≥ 320ms | 3 DRX cycle Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [3]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 of TS 38.133 [3] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms |  |

Table 8.4.2.0.1-4: Time period for time index detection (Frequency range FR2)

| Condition NOTE1,2 | TSSB_time_index_irat |
| --- | --- |
| No DRX | Max(200ms, MSSB_index_irat x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, (1.5 x MSSB_index_irat) x max(MGRP, SMTC period, DRX cycle)) x Nfreq |
| DRX cycle > 320ms | MSSB_index_irat x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] section 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

In the requirements, an NR cell is considered to be detectable when:

-	NR SS-RSRP related conditions in the accuracy requirements in Section 9.11.1 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [6],

-	NR SS-RSRQ related conditions in the accuracy requirements in Section 9.11.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [6],

-	NR SS-SINR related conditions in the accuracy requirements in Section 9.11.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 of TS 38.133 [6].

When measurement gaps are scheduled for NR measurements the UE physical layer shall be capable of reporting NR SS-RSRP, SS-RSRQ, and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 36.133 [23] clause 9.11, with measurement period as shown in table 8.4.2.0.1-5 and table 8.4.2.0.1-6:

Table 8.4.2.0.1-5: Measurement period for inter-RAT frequency measurements (Frequency FR1)

| Condition NOTE1,2 | T SSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(200ms, ceil(8 x 1.5) x max(MGRP, SMTC period, DRX cycle)) x Nfreq |
| DRX cycle > 320ms | 8 x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] section 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 8.4.2.0.1-5A: Measurement period for inter-RAT measurements for UE configured with highSpeedInterRAT-r16 (Frequency range FR1)

| Condition NOTE1,2 | TSSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(200ms, 8 Max(MGRP, SMTC period)) Nfreq |
| DRX cycle < 320ms | Max(200ms, ceil(8 × M) x max(MGRP, SMTC period, DRX cycle))×Nfreq |
| DRX cycle ≥ 320ms | 4× M DRX cycle ×Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in section 3.6.1 of TS 38.133 [50]. NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in section 3.6.1 of TS 38.133 [50] are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. NOTE 3: M = 1 when SMTC < = 40ms, and M = 1.5 when SMTC > 40ms |  |

Table 8.4.2.0.1-6: Measurement period for inter-RAT  frequency measurements (Frequency FR2)

| Condition NOTE1,2 | T SSB_measurement_period_irat |
| --- | --- |
| No DRX | Max(400ms, Mmeas_period_irat x max(MGRP, SMTC period)) x Nfreq |
| DRX cycle ≤ 320ms | Max(400ms, (1.5 x Mmeas_period_irat) x max(MGRP, SMTC period, DRX cycle)) x Nfreq |
| DRX cycle > 320ms | Mmeas_period_irat x DRX cycle x Nfreq |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in TS 36.133 [23] section 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in TS 36.133 [23] section 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

The UE shall be capable of performing NR SS-RSRP, SS-RSRQ, and SS-SINR for up to 7 NR carrier frequencies.

For each RAT E-UTRAN - NR layer on FR1 or FR2, the UE shall be capable of monitoring at least 4 cells.

For each RAT E-UTRAN - NR layer on FR1, during each layer 1 measurement period, the UE shall be capable of monitoring at least 7 SSBs with different SSB index and/or PCI on the RAT E-UTRA -NR.

For each RAT E-UTRA - NR layer on FR2, during each layer 1 measurement period, the UE shall be capable of monitoring at least 10 SSBs with different SSB index and/or PCI on the RAT E-UTRAN - NR layer. The UE shall be capable of monitoring at least one SSB per cell.

The NR SS-RSRP measurement accuracy for all measured cells shall be as specified in TS 36.133 [23] clause 9.11.1. The NR SS-RSRQ measurement accuracy for all measured cells shall be as specified in TS 36.133 [23] clause 9.11.2. The NR SS-SINR measurement accuracy for all measured cells shall be as specified in TS 36.133 [23] clause 9.11.3.

Reported measurements in event triggered measurement reports shall meet the requirements in TS 36.133 [23] clause 9.

The UE shall not send any event triggered measurement reports, as long as the reporting criteria is not fulfilled.

The measurement reporting delay is defined as the time between any events that will trigger a measurement report until the UE starts to transmit the measurement report over the Uu interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is twice the TTI of the uplink DCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_irat_without_index or Tidentify_irat_with_index for the minimum requirements. When L3 filtering is used or IDC autonomous denial or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_irat_without_index or Tidentify_irat_with_index for the minimum requirements and then triggers the measurement report as per TS 36.331 [2], the event triggered measurement reporting delay shall be less than Tmeasurement_NR_FDD provided the timing to that cell has not changed more than 3200 Tc while measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

The normative reference for this requirement is TS 36.133 [23] clause 8.1.2.4.21 and 8.1.2.4.22.

##### 8.4.2.0.2	Void

#### 8.4.2.1	E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX

8.4.2.1.1	Test purpose

The purpose of this test isto verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements for E-UTRA FDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.21 and for E-UTRAN TDD – NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.22.

8.4.2.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements.

8.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.1.

8.4.2.1.4	Test description

The test consists of two sub-tests with two cells configured, the E-UTRA PCell and NR neighbour cell; the difference between the two sub-tests is whether per-FR measurement gap is configured or not. Each sub-test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR neighbour cell. In all sub-tests the UE is not required to report SSB time index.

8.4.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.1.4.1-1.

Table 8.4.2.1.4.1-1: Supported test configurations for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.1.4.1-2.

Table 8.4.2.1.4.1-2: Initial conditions for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 8.4.2.1.4.1-3.

2. Message contents are defined in clause 8.4.2.1.4.3.

3. There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 8.4.2.1.5-1 for this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

Table 8.4.2.1.4.1-3: General test parameters for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  | 1, 2, 3, 4, 5, 6 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Chanel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | 19 | As specified in TS 36.331 [29]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [29] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [29] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 1 | 1 |  |
| Note 1: The value of b2-Threshold1 is defined in Table 8.4.2.1.5-1 Note 2: The value of b2-Threshold2NR is defined in Table 8.4.2.1.5-2 |  |  |  |  |  |

8.4.2.1.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.1.4.1-3, 8.4.2.1.5-1 and 8.4.2.1.5-2, respectively.

In sub-test 1 measurement gap pattern configuration #0 as defined in Table 8.4.2.1.4.1-3 is provided for UE that does not support per-FR gap and in sub-test 2 measurement gap pattern configuration #4 as defined in Table 8.4.2.1.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.1.5-1 and Table 8.4.2.1.5-2 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit an RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.1.5-1 and Table 8.4.2.1.5-2 respectively. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

- 922 for sub-test 1,

- 802 for sub-test 2.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

Or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.1.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.1.4.3-1: Common Exception messages for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |  |
| --- | --- | --- |
| Common contents of system information blocks exceptions |  |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapUE for Test 1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapFR1 for Test 2; Table H.3.4-5 with Condition Pattern #0 for Test 1; Table H.3.4-5 with Condition Pattern #4 for Test 2; |  |
| Specific message contents exceptions for Test Configuration 8.4.2.1-1 and 8.4.2.1-4 |  | Table H.3.4-6 with Conditions SMTC.2, SSB.1 FR1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.1-2 and 8.4.2.1-5 |  | Table H.3.4-6 with Conditions SMTC.1, SSB.1 FR1 and Synchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.1-3 and 8.4.2.1-6 |  | Table H.3.4-6 with Conditions SMTC.1, SSB.2 FR1 and Synchronous cells |

Table 8.4.2.1.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.1-1/2/3 |
| } |  |  |  |
| } |  |  |  |

8.4.2.1.5	Test requirement

Table 8.4.2.1.4.1-3, 8.4.2.1.5-1 and 8.4.2.1.5-2 define the primary level settings including test tolerances for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX.

Table 8.4.2.1.5-1: E-UTRAN PCell specific test parameters for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 15.35 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 15.35 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -88.65 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -88.65 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | 60.74+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 8.4.2.1.5-2: NR neighbour cell specific test parameters for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -89.35 |
|  |  | 3, 6 | -Infinity | -86.34 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8.65 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8.65 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -60.84 |
|  | dBm/38.16MHz | 3, 6 | -63.94 | -54.74 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event B2 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_irat_without_index

Tidentify_irat_without_index = TPSS/SSS_sync_irat + T SSB_measurement_period_irat

TPSS/SSS_sync_intra = 600 ms

T SSB_measurement_period_irat =

- 320 ms for sub-test 1

- 200 ms for sub-test 2

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 922 ms for sub-test 1 and 802 ms for sub-test 2.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 8.4.2.2	E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX

8.4.2.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements given in clause 8.1.2.4.21 of TS 36.133 for E-UTRAN FDD-NR measurements and clause 8.1.2.4.22 of TS 36.133 for E-UTRAN TDD-NR measurements.

8.4.2.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle.

8.4.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.2.

8.4.2.2.4	Test description

The test consists of four sub-tests with two cells configured, the E-UTRA PCell and NR neighbour cell; the difference between the four sub-tests is whether per-FR measurement gap is configured or not and the DRX configuration parameters. Each sub-test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR neighbour cell. In all sub-tests the UE is not required to report SSB time index.

8.4.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.2.4.1-1.

Table 8.4.2.2.4.1-1: Supported test configurations for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.2-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.2-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.2-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.2-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.2-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.2-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.2.4.1-2.

Table 8.4.2.2.4.1-2: Initial conditions for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 8.4.2.2.4.1-3.

2. Message contents are defined in clause 8.4.2.2.4.3.

3. There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 8.4.2.2.5-1 for this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

Table 8.4.2.2.4.1-3: General test parameters for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  | 19 |  | As specified in TS 36.331 [29]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  |  |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [29] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [29] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 | DRX.9 | DRX.12 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 | 11 | 2 | 11 |  |
| Note 1: The value of b2-Threshold1 is defined in Table 8.4.2.2.5-1 Note 2: The value of b2-Threshold2NR is defined in Table 8.4.2.2.5-2 |  |  |  |  |  |  |  |

8.4.2.2.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.2.4.1-3, 8.4.2.2.5-1 and 8.4.2.2.5-2, respectively.

In sub-test 1 and sub-test 2 measurement gap pattern configuration #0 as defined in Table 8.4.2.2.4.1-3 is provided for UE that does not support per-FR gap and in sub-test 3 and sub-test 4 measurement gap pattern configuration #4 as defined in Table 8.4.2.2.4.1-3 is provided for UE that supports per-FR gap.

DRX cycle = 40 ms is used in sub-test 1 and sub-test 3, DRX cycle = 640 ms is used in sub-test 2 and sub-test 4. The UE is allocated with PUSCH resource at every DRX cycle.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.2.5-1 and Table 8.4.2.2.5-2 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit an RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.2.5-1 and Table 8.4.2.2.5-2 respectively. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

- 1082 for sub-test 1 and sub-test 3,

- 10242 for sub-test 2 and sub-test 4.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.2.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.2.4.3-1: Common Exception messages for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapUE for Test 1 and Test 2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapFR1 for Test 3 and Test 4; Table H.3.4-5 with Condition Pattern #0 for Test 1 and Test 2; Table H.3.4-5 with Condition Pattern #4 for Test 3 and Test 4; Table H.3.7-2 with Condition DRX.9 for Test 1 and Test 3 Table H.3.7-2 with Condition DRX.12 for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 8.4.2.2-1 and 8.4.2.2-4 | Table H.3.4-6 with Conditions SMTC.2, SSB.1 FR1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.2-2 and 8.4.2.2-5 | Table H.3.4-6 with Conditions SMTC.1, SSB.1 FR1 and Synchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.2-3 and 8.4.2.2-6 | Table H.3.4-6 with Conditions SMTC.1, SSB.2 FR1 and Synchronous cells |

Table 8.4.2.2.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.2-1/2/3 |
| } |  |  |  |
| } |  |  |  |

8.4.2.2.5	Test requirement

Table 8.4.2.2.4.1-3, 8.4.2.2.5-1 and 8.4.2.2.5-2 define the primary level settings including test tolerances for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX.

Table 8.4.2.2.5-1: E-UTRAN PCell specific test parameters for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 15.35 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 15.35 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -88.65 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -88.65 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -60.74+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 8.4.2.2.5-2: NR neighbour cell specific test parameters for E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -89.35 |
|  |  | 3, 6 | -Infinity | -86.34 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8.65 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8.65 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -60.84 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -54.74 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event B2 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_irat_without_index

Tidentify_irat_without_index = TPSS/SSS_sync_irat + T SSB_measurement_period_irat

TPSS/SSS_sync_intra =

600 ms for sub-test 1 and sub-test 3

5120 ms for sub-test 2 and sub-test 4

T SSB_measurement_period_irat =

480 ms for sub-test 1 and sub-test 3

5120 ms for sub-test 2 and sub-test 4

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 1082 ms for sub-test 1 and sub-test 3, and shall be less than a total of 10242 ms for sub-test 2 and sub-test 4.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 8.4.2.3	E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX

8.4.2.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements for E-UTRA FDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.21 and for E-UTRAN TDD – NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.22.

8.4.2.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

8.4.2.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.3.

8.4.2.3.4	Test description

The test consists of two sub-tests with two cells configured, the E-UTRA PCell and NR neighbour cell; the difference between the two sub-tests is whether per-FR measurement gap is configured or not. Each sub-test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR neighbour cell. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell.

8.4.2.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.3.4.1-1.

Table 8.4.2.3.4.1-1: Supported test configurations for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.3-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.3-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.3-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.3-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.3-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.3-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.3.4.1-2.

Table 8.4.2.3.4.1-2: Initial conditions for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 8.4.2.3.4.1-3.

2. Message contents are defined in clause 8.4.2.3.4.3.

3. There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 8.4.2.3.5-1 for this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

Table 8.4.2.3.4.1-3: General test parameters for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  | 1, 2, 3, 4, 5, 6 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 | 19 | As specified in TS 36.331 [29]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [29] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [29] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 | 1 |  |
| Note 1: The value of b2-Threshold1 is defined in Table 8.4.2.3.5-1 Note 2: The value of b2-Threshold2NR is defined in Table 8.4.2.3.5-2 |  |  |  |  |  |

8.4.2.3.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.3.4.1-3, 8.4.2.3.5-1 and 8.4.2.3.5-2, respectively.

In sub-test 1 measurement gap pattern configuration #0 as defined in Table 8.4.2.3.4.1-3 is provided for UE that does not support per-FR gap and in sub-test 2 measurement gap pattern configuration #4 as defined in Table 8.4.2.3.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.3.5-1 and Table 8.4.2.3.5-2 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit an RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.3.5-1 and Table 8.4.2.3.5-2 respectively. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

- 1042 for sub-test 1,

- 922 for sub-test 2.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

Or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.3.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.3.4.3-1: Common Exception messages for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapUE for Test 1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapFR1 for Test 2; Table H.3.4-5 with Condition Pattern #0 for Test 1; Table H.3.4-5 with Condition Pattern #4 for Test 2; |
| Specific message contents exceptions for Test Configuration 8.4.2.3-1 and 8.4.2.3-4 | Table H.3.4-6 with Conditions SMTC.2, SSB.1 FR1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.3-2 and 8.4.2.3-5 | Table H.3.4-6 with Conditions SMTC.1, SSB.1 FR1 and Synchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.3-3 and 8.4.2.3-6 | Table H.3.4-6 with Conditions SMTC.1, SSB.2 FR1 and Synchronous cells |

Table 8.4.2.3.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.3-1/2/3 |
| } |  |  |  |
| } |  |  |  |

8.4.2.3.5	Test requirement

Table 8.4.2.3.4.1-3, 8.4.2.3.5-1 and 8.4.2.3.5-2 define the primary level settings including test tolerances for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX.

Table 8.4.2.3.5-1: E-UTRAN PCell specific test parameters for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 15.35 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 15.35 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -88.65 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -88.65 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -60.74+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 8.4.2.3.5-2: NR neighbour cell specific test parameters for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -89.35 |
|  |  | 3, 6 | -Infinity | -86.34 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8.65 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8.65 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -60.84 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -54.74 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event B2 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_irat_with_index

Tidentify_irat_without_index = TPSS/SSS_sync_irat + T SSB_measurement_period_irat  + TSSB_time_index_irat

TPSS/SSS_sync_intra = 600 ms

T SSB_measurement_period_irat =

320 ms for sub-test 1

200 ms for sub-test 2

TSSB_time_index_ira = 120 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 1042 ms for sub-test 1 and 922 ms for sub-test 2.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 8.4.2.4	E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX

8.4.2.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements for E-UTRA FDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.21 and for E-UTRAN TDD – NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.22.

8.4.2.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle.

8.4.2.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.4.

8.4.2.4.4	Test description

The test consists of four sub-tests with two cells configured, the E-UTRA PCell and NR neighbour cell; the difference between the four sub-tests is whether per-FR measurement gap is configured or not and the DRX configuration. Each subtest consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR neighbour cell. In measurements configuration UE shall be indicated to report the SSB index of the identified NR cell.

8.4.2.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.4.4.1-1.

Table 8.4.2.4.4.1-1: Supported test configurations for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.4-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.4-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.4-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.4-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.4-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.4-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.4.4.1-2.

Table 8.4.2.4.4.1-2: Initial conditions for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 8.4.2.4.4.1-3.

2. Message contents are defined in clause 8.4.2.4.4.3.

3. There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to Table 8.4.2.4.5-1 for this test. Cell 1 is configured according to Annex C.1.1 and C.1.2.

Table 8.4.2.4.4.1-3: General test parameters for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  | One FR1 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  | 19 |  | As specified in TS 36.331 [29]. |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | Note 1 |  |  |  | E-UTRA RSRP threshold for E-UTRA RSRP measurement on cell 1 for event B2 [29] |
| b2-Threshold2NR | dBm | 1, 2, 3, 4, 5, 6 | Note 2 |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B2 [29] |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  | L3 filtering is not used |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 | DRX.9 | DRX.12 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 2 | 13 | 2 | 13 |  |
| Note 1: The value of b2-Threshold1 is defined in Table 8.4.2.4.5-1 Note 2: The value of b2-Threshold2NR is defined in Table 8.4.2.4.5-2 |  |  |  |  |  |  |  |

8.4.2.4.4.2	Test procedure

Two cells are deployed in the test, which are E-UTRA PCell (Cell 1) on the E-UTRA carrier and a FR1 NR neighbour cell (Cell 2) on NR carrier. The general and cell specific test parameters for PCell and neighbour cell are given in Table 8.4.2.4.4.1-3, 8.4.2.4.5-1 and 8.4.2.4.5-2, respectively.

In sub-test 1 and sub-test 2 measurement gap pattern configuration #0 as defined in Table 8.4.2.4.4.1-3 is provided for UE that does not support per-FR gap and in sub-test 3 and sub-test 4 measurement gap pattern configuration #4 as defined in Table 8.4.2.4.4.1-3 is provided for UE that supports per-FR gap.

DRX cycle = 40 ms is used in sub-test 1 and sub-test 3, DRX cycle = 640 ms is used in sub-test 2 and sub-test 4. The UE is allocated with PUSCH resource at every DRX cycle.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B2 is used. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in State 3A-RF according to TS 36.508 clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to T1 in Table 8.4.2.4.5-1 and Table 8.4.2.4.5-2 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.4.5-1 and Table 8.4.2.4.5-2 respectively. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

- 1262 for sub-test 1 and sub-test 3,

- 12162 for sub-test 2 and sub-test 4.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.4.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.4.4.3-1: Common Exception messages for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapUE for Test 1 and Test 2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B2 and gapFR1 for Test 3 and Test 4; Table H.3.4-5 with Condition Pattern #0 for Test 1 and Test 2; Table H.3.4-5 with Condition Pattern #4 for Test 3 and Test 4; Table H.3.7-2 with Condition DRX.9 for Test 1 and Test 3 Table H.3.7-2 with Condition DRX.12 for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 8.4.2.2-1 and 8.4.2.2-4 | Table H.3.4-6 with Conditions SMTC.2, SSB.1 FR1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.2-2 and 8.4.2.2-5 | Table H.3.4-6 with Conditions SMTC.1, SSB.1 FR1 and Synchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.2-3 and 8.4.2.2-6 | Table H.3.4-6 with Conditions SMTC.1, SSB.2 FR1 and Synchronous cells |

Table 8.4.2.4.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.4-1/2/3 |
| } |  |  |  |
| } |  |  |  |

8.4.2.4.5	Test requirement

Table 8.4.2.4.4.1-3, 8.4.2.4.5-1 and 8.4.2.4.5-2 define the primary level settings including test tolerances for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX.

Table 8.4.2.4.5-1: E-UTRAN PCell specific test parameters for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX

| Parameter | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| b2-Threshold1 | dBm | 1, 2, 3, 4, 5, 6 | -77 |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | 17 | 15.35 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | 17 | 15.35 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -88.65 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -87 | -88.65 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -59.13+10log (NRB,c /50) | -60.74+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

Table 8.4.2.4.5-2: NR neighbour cell specific test parameters for E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| NR RF Channel Number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 4 | FDD |  |
|  |  | 2, 3, 5, 6 | TDD |  |
| TDD configuration |  | 2, 5 | TDDConf.1.1 |  |
|  |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel | MHz | 1, 2, 4, 5 | 10: NRB,c = 52 |  |
|  |  | 3, 6 | 40: NRB,c = 106 |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  | 1, 4 | SMTC.2 |  |
|  |  | 2, 3, 5, 6 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2, 4, 5 | 15 |  |
|  |  | 3, 6 | 30 |  |
| b2-Threshold2NR | dBm/SCS | 1, 2, 4, 5 | -101 |  |
|  |  | 3, 6 | -98 |  |
| EPRE ratio of PSS to SSS |  | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -98 |  |
| Note2 | dBm/SCS | 1, 2, 4, 5 | -98 |  |
|  |  | 3, 6 | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1, 2, 4, 5 | -Infinity | -89.35 |
|  |  | 3, 6 | -Infinity | -86.34 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8.65 |
|  | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 8.65 |
| IoNote3 | dBm/9.36MHz | 1, 2, 4, 5 | -70.05 | -60.84 |
|  | dBm/38.16MHz | 3, 6 | -63.95 | -54.74 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | ETU70 |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event B2 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_irat_with_index

Tidentify_irat_without_index = TPSS/SSS_sync_irat + T SSB_measurement_period_irat + TSSB_time_index_irat

TPSS/SSS_sync_intra =

600 ms for sub-test 1 and sub-test 3

5120 ms for sub-test 2 and sub-test 4

T SSB_measurement_period_irat =

480 ms for sub-test 1 and sub-test 3

5120 ms for sub-test 2 and sub-test 4

TSSB_time_index_irat =

180 ms for sub-test 1 and sub-test 3

1920 ms for sub-test 2 and sub-test 4

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 1292 ms for sub-test 1 and sub-test 3, and shall be less than a total of 12162 ms for sub-test 2 and sub-test 4.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 8.4.2.5	E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in non-DRX

Editor’s notes: This test case is incomplete. The following aspects are either missing or TBD.

-	Connection Diagram is TBD;

-	Test tolerance analysis is missing;

-	Test applicability Table in TS38.522 need to be updated.

8.4.2.5.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event without SSB time index detection in non-DRX within E-UTRA – NR FR2 cell search requirements.

8.4.2.5.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements.

8.4.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.5.

8.4.2.5.4	Test description

8.4.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.5.4.1-1.

Table 8.4.2.5.4.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in non-DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.5-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.4.2.5-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.5.4.1-2.

Table 8.4.2.5.4.1-2: Initial conditions for E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.5.4.1-3.

2.	Message contents are defined in clause 8.4.2.5.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR FR2 neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to clause A.6.1.2 for this test.

Table 8.4.2.5.4.1-3: General test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.6.1.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [29]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [29]. |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used. |
| DRX |  | 1, 2 | OFF |  | DRX is not used. |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 10 |  |  |
| T2 | s | 1, 2 | 6 | 3 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table 8.4.2.5.5-1 |  |  |  |  |  |

8.4.2.5.4.2	Test procedure

In test 1 measurement gap pattern configuration # 0 as defined in Table 8.4.2.5.4.1-3 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 8.4.2.5.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [29] is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have timing information of NR cell 2.

1. Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to Table A.6.1.2-1 and T1 in Table 8.4.2.5.5-1 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit an RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.5.5-1. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B1. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

- 3200 for sub-test 1,

- 1600 for sub-test 2.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

Or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.5.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.5.4.3-1: Common Exception messages for E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and gapUE for Test 1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and GAPLESS for Test 2; Table H.3.4-5 with Condition Pattern #0 for Test 1; Table H.3.4-5 with Condition Pattern #4 for Test 2; |
| Specific message contents exceptions for Test Configuration 8.4.2.5-1 | Table H.3.4-6 with Conditions SMTC.2, SSB.3 FR2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.5-2 | Table H.3.4-6 with Conditions SMTC.1, SSB.3 FR2 and Synchronous cells |

Table 8.4.2.5.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.5-1/2/3 |
| } |  |  |  |
| } |  |  |  |

8.4.2.5.5	Test requirement

Tables 8.4.2.5.4.1-3 and 8.4.2.5.5-1 define the primary level settings including test tolerances for E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in non-DRX.

Table 8.4.2.5.5-1: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.2.1 (OP.1) |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.4 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -108 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| AoA setup defined in A.9 |  |  | 1, 2 | Setup 2a |  |
| Note2 |  | dBm/15kHz | 1, 2 | -111+TT |  |
| Note2 |  | dBm/SCS | 1, 2 | -102+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -88+TT |
|  |  | dB | 1, 2 | -Infinity | 14+TT |
|  |  | dB | 1, 2 | -Infinity | 14+TT |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -Infinity | -58.84+TT |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

Table 8.4.2.5.5-2: Test requirements for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in non-DRX

| Test case | Measurement reporting delay (ms) |  |
| --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms |
| UE power class 3 | 3200 | 1600 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

In test 1 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is not required to report SSB time index.

#### 8.4.2.6	E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in DRX

Editor’s notes: This test case is incomplete. The following aspects are either missing or TBD.

-	Connection Diagram is TBD;

-	Test tolerance analysis is missing;

-	Test applicability Table in TS38.522 need to be updated.

8.4.2.6.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event without SSB time index detection in DRX within E-UTRA – NR FR2 cell search requirements.

8.4.2.6.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements.

8.4.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.6.

8.4.2.6.4	Test description

8.4.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.6.4.1-1.

Table 8.4.2.6.4.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.6-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.4.2.6-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.6.4.1-2.

Table 8.4.2.6.4.1-2: Initial conditions for E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.6.4.1-3.

2.	Message contents are defined in clause 8.4.2.6.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR FR2 neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to clause A.6.1.2 for this test.

Table 8.4.2.6.4.1-3: General test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  | Test 3 | Test 4 |  |  |
| E-UTRA RF Channel Number |  | 1, 2 | 1 |  |  |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Number |  | 1, 2 | 1 |  |  |  |  |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2, 3, 4, 5, 6 | E-UTRA cell 1 (PCell) |  |  |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.6.1.2. |
| Neighbour cell |  | 1, 2, 3, 4, 5, 6 | NR cell 2 |  |  |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2, 3, 4, 5, 6 | 0 |  |  | 4 |  |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2, 3, 4, 5, 6 | 39 |  |  | 19 |  |  | As specified in TS 36.331 [29]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  |  |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [29]. |
| Hysteresis | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| CP length |  | 1, 2, 3, 4, 5, 6 | Normal |  |  |  |  |  |  |
| TimeToTrigger | s | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  |  |
| Filter coefficient |  | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |  |  | L3 filtering is not used. |
| DRX |  | 1, 2, 3, 4, 5, 6 | DRX.9 | DRX.12 |  | DRX.9 | DRX.12 |  | As specified in clause A.3.3. |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  |  |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  |  |  |  |  | Synchronous cells. |
| T1 | s | 1, 2, 3, 4, 5, 6 | 5 |  |  |  |  |  |  |
| T2 | s | 1, 2, 3, 4, 5, 6 | 6 |  | 83 | 6 |  | 83 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table 8.4.2.6.5-1 |  |  |  |  |  |  |  |  |  |

8.4.2.6.4.2	Test procedure

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table 8.4.2.6.4.1-3 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table 8.4.2.6.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [29] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have timing information of NR cell 2.

1. Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to Table A.6.1.2-1 and T1 in Table 8.4.2.6.5-1 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit an RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.6.5-1. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B1. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

- 4800 for sub-test 1 and sub-test 3,

- 51200 for sub-test 2 and sub-test 4.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 [25] clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.6.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.6.4.3-1: Common Exception messages for E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and gapUE for Test 1 and Test 2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and GAPLESS for Test 3 and Test 4; Table H.3.4-5 with Condition Pattern #0 for Test 1 and Test 2; Table H.3.4-5 with Condition Pattern #4 for Test 3 and Test 4; Table H.3.7-2 with Condition DRX.9 for Test 1 and Test 3 Table H.3.7-2 with Condition DRX.12 for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 8.4.2.6-1 | Table H.3.4-6 with Conditions SMTC.2, SSB.3 FR2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.6-2 | Table H.3.4-6 with Conditions SMTC.1, SSB.3 FR2 and Synchronous cells |

Table 8.4.2.6.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.6-1/2/3 |
| } |  |  |  |
| } |  |  |  |

8.4.2.6.5	Test requirement

Tables 8.4.2.6.4.1-3 and 8.4.2.6.5-1 define the primary level settings including test tolerances for E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in DRX.

Table 8.4.2.6.5-1: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.2.1 (OP.1) |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.4 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -96 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| AoA setup defined in A.9 |  |  | 1, 2 | Setup 1 |  |
| Note2 |  | dBm/15kHz | 1, 2 | -111+TT |  |
| Note2 |  | dBm/SCS | 1, 2 | -102+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -88+TT |
|  |  | dB | 1, 2 | -Infinity | 14+TT |
|  |  | dB | 1, 2 | -Infinity | 14+TT |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -Infinity | -58.84+TT |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

Table 8.4.2.6.5-2: Test requirements for NR inter-RAT event triggered reporting for FR2 without SSB time index detection in DRX

| Test case | Measurement reporting delay (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms | Test 3: D3 ms | Test 4: D4 ms |
| UE power class 3 | 4800 | 51200 | 4800 | 51200 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

In test 1 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D3 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B1 triggered measurement report, with a measurement reporting delay less than D4 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is not required to report SSB time index.

#### 8.4.2.7	E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in non-DRX

Editor’s notes: This test case is incomplete. The following aspects are either missing or TBD.

-	Connection Diagram is TBD;

-	Test tolerance analysis is missing;

-	Test applicability Table in TS38.522 need to be updated.

8.4.2.7.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event with SSB time index detection in non-DRX within E-UTRA – NR FR2 cell search requirements.

8.4.2.7.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements.

8.4.2.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.7.

8.4.2.7.4	Test description

8.4.2.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.7.4.1-1.

Table 8.4.2.7.4.1-1: NR inter-RAT event triggered reporting tests with SSB index reading for FR2 in non-DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.7-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.4.2.7-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.7.4.1-2.

Table 8.4.2.7.4.1-2: Initial conditions for E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.7.4.1-3.

2.	Message contents are defined in clause 8.4.2.7.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR FR2 neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to clause A.6.1.2 for this test.

Table 8.4.2.7.4.1-3: General test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| E-UTRA RF Channel Numbers |  | 1, 2 | 1 |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Numbers |  | 1, 2 | 1 |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.6.1.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 | 4 | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2 | 39 | 19 | As specified in TS 36.331 [29]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [29] |
| Hysteresis | dB | 1, 2 | 0 |  |  |
| CP length |  | 1, 2 | Normal |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |
| Filter coefficient |  | 1, 2 | 0 |  | L3 filtering is not used. |
| DRX |  | 1, 2 | OFF |  | DRX is not used. |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |
| T2 | s | 1, 2 | 5 | 3 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table A.8.4.2.7.5-1. |  |  |  |  |  |

8.4.2.7.4.2	Test procedure

In test 1 measurement gap pattern configuration # 0 as defined in Table 8.4.2.7.4.1-3 is provided for UE that does not support per-FR gap and in test 2 measurement gap pattern configuration #4 as defined in Table 8.4.2.7.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [29] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to Table A.6.1.2-1 and T1 in Table 8.4.2.7.5-1 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit an RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.7.5-1. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B1. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

- 4160 for sub-test 1,

- 2080 for sub-test 2.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

Or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.7.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.7.4.3-1: Common Exception messages for E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and gapUE for Test 1; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and GAPLESS for Test 2; Table H.3.4-5 with Condition Pattern #0 for Test 1; Table H.3.4-5 with Condition Pattern #4 for Test 2; |
| Specific message contents exceptions for Test Configuration 8.4.2.7-1 | Table H.3.4-6 with Conditions SMTC.2, SSB.3 FR2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.7-2 | Table H.3.4-6 with Conditions SMTC.1, SSB.3 FR2 and Synchronous cells |

Table 8.4.2.7.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.7-1/2/3 |
| } |  |  |  |
| } |  |  |  |

8.4.2.7.5	Test requirement

Tables 8.4.2.7.4.1-3 and 8.4.2.7.5-1 define the primary level settings including test tolerances for E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in non-DRX.

Table 8.4.2.7.5-1: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.2.1 (OP.1) |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.4 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -96 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| AoA setup defined in A.9 |  |  | 1, 2 | Setup 1 |  |
| Note2 |  | dBm/15kHz | 1, 2 | -111+TT |  |
| Note2 |  | dBm/SCS | 1, 2 | -102+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -88+TT |
|  |  | dB | 1, 2 | -Infinity | 14+TT |
|  |  | dB | 1, 2 | -Infinity | 14+TT |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -Infinity | -58.84+TT |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

Table 8.4.2.7.5-2: Test requirements for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in non-DRX

| Test case | Measurement reporting delay (ms) |  |
| --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms |
| UE power class 3 | 4160 | 2080 |

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 1 and test 2, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 8.4.2.8	E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in DRX

Editor’s notes: This test case is incomplete. The following aspects are either missing or TBD.

-	Connection Diagram is TBD;

-	Test tolerance analysis is missing;

-	Test applicability Table in TS38.522 need to be updated.

8.4.2.8.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event with SSB time index detection in DRX within E-UTRA – NR FR2 cell search requirements.

8.4.2.8.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements.

8.4.2.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.8.

8.4.2.8.4	Test description

8.4.2.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.8.4.1-1.

Table 8.4.2.8.4.1-1: NR inter-RAT event triggered reporting tests without SSB index reading for FR2 in DRX

| Configuration | Description |
| --- | --- |
| 8.4.2.8-1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 8.4.2.8-2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.8.4.1-2.

Table 8.4.2.8.4.1-2: Initial conditions for E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.4.2.8.4.1-3.

2.	Message contents are defined in clause 8.4.2.8.4.3.

3.	There are two carriers and two cells specified in the test, where Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR FR2 neighbour cell on the NR carrier. Cell 1 is the cell used for connection setup with the power level set according to clause A.6.1.2 for this test.

Table 8.4.2.8.4.1-3: General test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 | Test 4 |  |
| E-UTRA RF Channel Numbers |  | 1, 2 | 1 |  |  |  | One E-UTRA carrier frequency is used. |
| NR RF Channel Numbers |  | 1, 2 | 1 |  |  |  | One FR2 NR carrier frequency is used. |
| Active cell |  | 1, 2 | E-UTRA cell 1 (PCell) |  |  |  | E-UTRA cell 1 is on E-UTRA RF channel number 1 as defined in clause A.6.1.2. |
| Neighbour cell |  | 1, 2 | NR cell 2 |  |  |  | NR cell 2 is on NR RF channel number 1. |
| Gap Pattern Id |  | 1, 2 | 0 |  | 4 |  | As specified in clause Table 8.1.2.1-1 of TS 36.133 [23]. |
| Measurement gap offset |  | 1, 2 | 39 |  | 19 |  | As specified in TS 36.331 [29]. |
| b1-ThresholdNR | dBm | 1, 2 | Note 1 |  |  |  | SS-RSRP threshold for SS-RSRP measurement on cell 2 for event B1 [29]. |
| Hysteresis | dB | 1, 2 | 0 |  |  |  |  |
| CP length |  | 1, 2 | Normal |  |  |  |  |
| TimeToTrigger | s | 1, 2 | 0 |  |  |  |  |
| Filter coefficient |  | 1, 2 | 0 |  |  |  | L3 filtering is not used. |
| DRX |  |  | DRX.9 | DRX.12 | DRX.9 | DRX.12 | As specified in clause A.3.3. |
| Time offset between serving and neighbour cells |  | 1 | 3ms |  |  |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3s |  |  |  | Synchronous cells. |
| T1 | s | 1, 2 | 5 |  |  |  |  |
| T2 | s | 1, 2 | 7 | 70 | 7 | 70 |  |
| Note 1: The value of b1-ThresholdNR is defined in Table A.8.4.2.8.5-1 |  |  |  |  |  |  |  |

8.4.2.8.4.2	Test procedure

In tests 1 and 2, measurement gap pattern configuration # 0 as defined in Table 8.4.2.8.4.1-3 is provided for UE that does not support per-FR gap and in tests 3 and 4, measurement gap pattern configuration #4 as defined in Table 8.4.2.8.4.1-3 is provided for UE that supports per-FR gap.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event B1 (Inter RAT neighbour becomes better than threshold) [29] is used. In the measurement configuration the UE shall be indicated to report the SSB index of the identified NR cell. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2. Set the parameters of Cell 1 and Cell 2 according to Table A.6.1.2-1 and T1 in Table 8.4.2.8.5-1 respectively, T1 starts.

3. The SS shall transmit an RRCConnectionReconfiguration message.

4. The UE shall transmit an RRCConnectionReconfigurationComplete message.

5. When T1 expires, the SS shall switch the parameters setting from T1 to T2 as specified in Table 8.4.2.8.5-1. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event B1. If the overall delays measured from the beginning of time period T2 is less than X ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one. Where X is,

- 4800 for sub-test 1 and sub-test 3,

- 51200 for sub-test 2 and sub-test 4.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall transmit an RRCConnectionRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in State 3A according to TS 36.508 clause 4.5.3A (if the paging fails, switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3),

Or

- switches off and on the UE and ensures the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

8.4.2.8.4.3	Message contents

Message contents are according to TS 36.508 [25] clause 7.3 with the following exceptions:

Table 8.4.2.8.4.3-1: Common Exception messages for E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and gapUE for Test 1 and Test 2; Table H.3.4-4 with Condition INTER-RAT NR, EVENT B1 and GAPLESS for Test 3 and Test 4; Table H.3.4-5 with Condition Pattern #0 for Test 1 and Test 2; Table H.3.4-5 with Condition Pattern #4 for Test 3 and Test 4; Table H.3.7-2 with Condition DRX.9 for Test 1 and Test 3 Table H.3.7-2 with Condition DRX.12 for Test 2 and Test 4 |
| Specific message contents exceptions for Test Configuration 8.4.2.8-1 | Table H.3.4-6 with Conditions SMTC.2, SSB.3 FR2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.2.8-2 | Table H.3.4-6 with Conditions SMTC.1, SSB.3 FR2 and Synchronous cells |

Table 8.4.2.8.4.3-2: SchedulingRequest-Config-DEFAULT

| Derivation Path: 36.508 [25] Table 4.6.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 25 |  | Config 8.4.2.8-1/2/3 |
| } |  |  |  |
| } |  |  |  |

8.4.2.8.5	Test requirement

Tables 8.4.2.8.4.1-3 and 8.4.2.8.5-1 define the primary level settings including test tolerances for E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in DRX.

Table 8.4.2.8.5-1: NR neighbour cell specific test parameters for NR inter-RAT event triggered reporting for FR2 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |
| NR RF Channel Number |  |  | 1, 2 | 1 |  |
| Duplex mode |  |  | 1, 2 | TDD |  |
| TDD configuration |  |  | 1, 2 | TDDConf.3.1 |  |
| BWchannel |  | MHz | 1, 2 | 100: NRB,c = 66 |  |
| OCNG patterns defined in A.2.1 (OP.1) |  |  | 1, 2 | OP.1 |  |
| SMTC configuration defined in A.4 |  |  | 1 | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 120 |  |
| b1-ThresholdNR | UE power class 3 | dBm/SCS | 1, 2 | -96 |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| AoA setup defined in A.9 |  |  | 1, 2 | Setup 1 |  |
| Note2 |  | dBm/15kHz | 1, 2 | -111+TT |  |
| Note2 |  | dBm/SCS | 1, 2 | -102+TT |  |
| SS-RSRP Note 3 |  | dBm/SCS | 1, 2 | -Infinity | -88+TT |
|  |  | dB | 1, 2 | -Infinity | 14+TT |
|  |  | dB | 1, 2 | -Infinity | 14+TT |
| IoNote3 |  | dBm/95.04MHz | 1, 2 | -Infinity | -58.84+TT |
| Propagation Condition |  |  | 1, 2 | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |

Table 8.4.2.8.5-2: Test requirements for NR inter-RAT event triggered reporting for FR2 with SSB time index detection in DRX

| Test case | Measurement reporting delay (ms) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1: D1 ms | Test 2: D2 ms | Test 3: D3 ms | Test 4: D4 ms |
| UE power class 3 | 4800 | 51200 | 4800 | 51200 |

In test 1 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D1 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 2 with per-UE gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D2 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 3 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D3 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In test 4 with per-FR gap, the UE shall send one Event B2 triggered measurement report, with a measurement reporting delay less than D4 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

In tests 1, 2, 3 and 4, the UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 8.4.2.9	E-UTRA – NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The Test tolerances and Test system uncertainties applicable to this test are undefined.

-	Cell Configuration table in Annex E is undefined.

-	Antenna diagram is TBD

-	Message content is TBD

-	Test case applicability is TBD

-	Test procedure is TBD

8.4.2.9.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event and to verify partly the NR inter-RAT cell search requirements for E-UTRA FDD - NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.21 and for E-UTRAN TDD – NR FR1 measurements given in TS 36.133 [23] clause 8.1.2.4.22 when UE is configured with highSpeedInterRAT-NR-r16.

8.4.2.9.2	Test applicability

This test applies to all E-UTRA UEs capable of NR measurements and configured with highSpeedInterRAT-NR-r16 from release 16 onwards.

8.4.2.9.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.4.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.4.2.9.

8.4.2.9.4	Test description

8.4.2.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.4.2.9.4.1-1.

Table 8.4.2.9.4.1-1: TBD

| Configuration | Description |
| --- | --- |
| 8.4.2.1-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.1-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 8.4.2.1-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 8.4.2.1-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 8.4.2.9.4.1-2.

Table 8.4.2.9.4.1-2: TBD

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | N/A |  |  |

TBD

8.4.2.9.4.2	Test procedure

TBD

8.4.2.9.4.3	Message contents

TBD

8.4.2.9.5	Test requirement

TBD

## 8.5	Measurement performance requirements

### 8.5.1	SFTD measurement accuracy

#### 8.5.1.0	Minimum conformance requirements

##### 8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements

The SFN and frame timing difference (SFTD) is measured between PCell and NR cell for inter-RAT SFTD. The inter-RAT SFTD measurement can only be configured for E-UTRA - NR band combinations that are supported by the UE.

The accuracy requirements in Table 8.5.1.0.1-3 are applicable under the following conditions:

For PCell SFN and frame timing measurement:

-	Cell specific reference signals are transmitted either from one, two or four antenna ports.

-	Conditions defined in TS 36.101 clause 7.3 for reference sensitivity are fulfilled.

-	No changes to the uplink transmission timing are applied during the measurement period.

-	RSRP|dBm according to TS 36.133 Annex B.3.5 for a corresponding Band.

-	Io range defined in Table 8.5.1.0.1-1.

Table 8.5.1.0.1-1: PCell Io range conditions for SFTD measurement accuracy

| Parameter | Io Note 1 range |  |  |
| --- | --- | --- | --- |
|  | E-UTRA operating band groups Note 4, 5 | Minimum Io | Maximum Io |
|  |  | dBm/15kHz Note 2, 3 | dBm/BWChannel |
| Conditions | FDD_A, TDD_A | -121 | -50 |
|  | FDD_C, TDD_C | -120 | -50 |
|  | FDD_D | -119.5 | -50 |
|  | FDD_E, TDD_E | -119 | -50 |
|  | FDD_F | -118.5 | -50 |
|  | FDD_G | -118 | -50 |
|  | FDD_H | -117.5 | -50 |
|  | FDD_N | -114.5 | -50 |
| NOTE 1: When in dBm/15kHz, the minimum Io condition is expressed as the average Io per RE over all REs in that symbol. Io may be different in different symbols within a subframe. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in clause B.4.2 and B.4.3. NOTE 3: The condition level is increased by MSD as defined in clause 7.3B in TS 38.101-3 [54], if applicable depending on E-UTRA – NR band combination. NOTE 4: E-UTRA operating band groups are as defined in clause 3.5. NOTE 5: Only E-UTRA bands within EN-DC band combinations as specified in clause 5.5B in TS 38.101-3 [54] are applicable. |  |  |  |

For NR PSCell, or NR cell SFN and frame timing measurement in FR1:

-	Conditions defined in TS 38.101-1 clause 7.3 for reference sensitivity are fulfilled.

-	Io range defined in Table 8.5.1.0.1-2.

Table 8.5.1.0.1-2: NR PSCell, or NR cell Io range conditions for SFTD measurement accuracy in FR1

| Parameter | Io Note 1 range |  |  |  |
| --- | --- | --- | --- | --- |
|  | NR operating band groups Note 4, 5 | Minimum Io Note 2, 3 |  | Maximum Io |
|  |  | dBm/ SCSSSB |  | dBm/BWChannel |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -121 | -118 | -50 |
|  | NR_FDD_FR1_B | -120.5 | -117.5 | -50 |
|  | NR_TDD_FR1_C | -120 | -117 | -50 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -50 |
|  | NR_FDD_FR1_G | -118 | -115 | -50 |
|  | NR_FDD_FR1_H | -117.5 | -114.5 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ΔRIB,c as defined in clause 7.3B in TS 38.101-3 [54], depending on E-UTRA – NR band combination. NOTE 3: The condition level is increased by MSD as defined in clause 7.3B in TS 38.101-3 [54], if applicable depending on E-UTRA – NR band combination. NOTE 4: NR operating band groups are as defined in clause 3.5. NOTE 5: Only NR bands within EN-DC band combinations as specified in clause 5.5B in TS 38.101-3 [54] are applicable. |  |  |  |  |

Table 8.5.1.0.1-3: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot | Frequency range |
| Ts Note 1 | dB |  |
| 40 | -3 dB | FR1 |
| NOTE 1: Ts is the basic timing unit defined in TS 36.211 [16]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

#### 8.5.1.1	E-UTRA – NR FR1 SFTD measurement accuracy

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- TT analysis is missing.

8.5.1.1.1	Test purpose

The purpose of this test is to verify that the SFTD measurement accuracy is within the specified limits and to verify the requirements as specified in clause 9.1.27 in TS 36.133 [23] for inter-RAT FR1 SFTD measurements.

8.5.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards and support inter-RAT NR SFTD measurements.

8.5.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.1.1.

8.5.1.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.1.1.4.1-1.

Table 8.5.1.1.4.1-1: Supported test configurations for E-UTRA – NR FR1 SFTD measurement accuracy

| Configuration | Description |
| --- | --- |
| 8.5.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.1.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.1.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.1.1.4.1-2.

Table 8.5.1.1.4.1-2: Initial conditions for E-UTRA – NR FR1 SFTD measurement accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 8.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.5.1.1.4.1-3.

2.	Message contents are defined in clause 8.5.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

8.5.1.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.1.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit a MeasurementReport message triggered by SFTD measurement.

6.	The SS shall check the reported values of SFN offset and frame boundary offset in the MeasurementReport. The SFN offset value and frame boundary offset value between E-UTRA Cell 1 and Cell 2 reported by the UE is compared to the expected SFN offset value and frame boundary offset, respectively. The number of failed iterations is increased by one, if

-	The reported SFN offset value is different with the expected SFN offset value, or

-	The difference between reported frame boundary offset value and expected frame boundary offset value is outside the limits given in Table 8.5.1.1.5-2, or

-	The UE fails to report the measurement value for Cell 2.

Otherwise the number of successful iterations is increased by one.

7.	Repeat steps 1-6 until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.5.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.1.1.4.3-1: Common Exception messages for E-UTRA – NR FR1 SFTD measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-1a; Table H.3.4-4 with Condition INTER-RAT NR, SFTD and GAPLESS |
| Specific message contents exceptions for Test Configuration 8.4.1.1-1 and 8.4.1.1-4 | Table H.3.4-6 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.1.1-2 and 8.4.1.1-5 | Table H.3.4-6 with Condition SSB.1 FR1, SMTC.1 and Asynchronous cells |
| Specific message contents exceptions for Test Configuration 8.4.1.1-3 and 8.4.1.1-6 | Table H.3.4-6 with Condition SSB.2 FR1, SMTC.1 and Asynchronous cells |

8.5.1.1.5	Test requirement

Table 8.5.1.1.5-1 and 8.5.1.1.5-2 defines the primary level settings including test tolerances for E-UTRA – NR FR1 SFTD measurement accuracy.

The SFN offset and frame boundary offset between E-UTRA PCell and the inter-RAT neighbour cell shall be set to one of the configurations in Table 8.5.1.1.5-3. The reported SFTD accuracy shall fulfil the accuracy requirements in Table 8.5.1.1.5-4.

Table 8.5.1.1.5-1: E-UTRA cell specific test parameters for E-UTRA – NR FR1 SFTD measurement accuracy

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel |  | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |
| OCNG PatternsNote2 |  | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| NocNote4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | -3 |
| Ês/Iot | dB | -3 |
| RSRP Note5 | dBm/15 kHz | -107 |
| SCH_RP Note5 | dBm/15 kHz | -107 |
| Io Note5 | dBm/Ch BW | -74.45 +10log (NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |

Table 8.5.1.1.5-2: NR cell specific test parameters for E-UTRA – NR FR1 SFTD measurement accuracy

| Parameter |  | Config | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~6 |  | freq1 |
| Duplex mode |  | 1,4 |  | FDD |
|  |  | 2,5 |  | TDD |
|  |  | 3,6 |  | TDD |
| TDD Configuration |  | 1,4 |  | N/A |
|  |  | 2,5 |  | TDDConf.1.1 |
|  |  | 3,6 |  | TDDConf.2.1 |
| BWchannel |  | 1,4 | MHz | 10: NRB,c = 52 |
|  |  | 2,5 |  | 10: NRB,c = 52 |
|  |  | 3,6 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1,4 |  | SR.1.1 FDD |
|  |  | 2,5 |  | SR.1.1 TDD |
|  |  | 3,6 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1,4 |  | CR.1.1 FDD |
|  |  | 2,5 |  | CR.1.1 TDD |
|  |  | 3,6 |  | CR.2.1 TDD |
| RMC CORESET Reference Channel |  | 1,4 |  | CCR.1.1 FDD |
|  |  | 2,5 |  | CCR.1.1 TDD |
|  |  | 3,6 |  | CCR.2.1 TDD |
| SSB configuration |  | 1,4 |  | SSB.1 FR1 |
|  |  | 2,5 |  | SSB.1 FR1 |
|  |  | 3,6 |  | SSB.2 FR1 |
| SMTC configuration |  | 1~6 |  | SMTC.1 |
| DL BWP configuration |  | 1~6 |  | DLBWP.1.1 |
| UL BWP configuration |  | 1~6 |  | ULBWP.1.1 |
| OCNG Patterns |  | 1~6 |  | OP.1 |
| EPRE ratio of PSS to SSS |  | 1~6 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1~6 | dBm/15kHz | -104 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/SSB SCS | -104 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 |  | -101 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  |  | 1~6 | dB | -3 |
|  |  | 1~6 | dB | -3 |
| SS-RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/SCS | -107 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 |  | -104 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A | 1,2,4,5 | dBm/9.36 MHz | -74.28 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A | 3,6 | dBm/38.16 MHz | -68.18 |
|  | NR_FDD_FR1_B |  |  |  |
|  | NR_TDD_FR1_C |  |  |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |
|  | NR_FDD_FR1_G |  |  |  |
|  | NR_FDD_FR1_H |  |  |  |
| Propagation condition |  | 1~6 |  | AWGN |
| Antenna configuration |  | 1~6 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table 8.5.1.1.5-3: Timing offsets for E-UTRA – NR FR1 SFTD measurement accuracy

| Configuration | SFN offset between PCell and PSCell | Frame boundary offset between PCell and PSCell (Ts) |
| --- | --- | --- |
| 1 | 100 | -122000 |
| 2 | 300 | -60540 |
| 3 | 500 | 1000 |
| 4 | 700 | 62540 |
| 5 | 900 | 124000 |

Table 8.5.1.1.5-4: SFTD measurement accuracy

| Accuracy | Conditions |  |
| --- | --- | --- |
|  | Ês/Iot | Frequency range |
| Ts Note 1 | dB |  |
| 40 | -3 dB | FR1 |
| NOTE 1: Ts is the basic timing unit defined in TS 36.211 [24]. NOTE 2: The parameter Ês/Iot is the minimum Ês/Iot of the pair of cells to which the requirement applies. |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 8.5.2	Inter-RAT measurement accuracy

#### 8.5.2.1	SS-RSRP

##### 8.5.2.1.0	Minimum conformance requirements

The measurement period of NR SS-RSRP measurements in RRC_CONNECTED state is specified in TS 36.133 [23] Section 8.1.2.4.21 for UE with FDD PCell not configured with E-UTRA-NR Dual Connectivity operation. The measurement period of NR SS-RSRP measurements in RRC_CONNECTED state is specified in TS 36.133 [23] Section 8.1.2.4.22 for UE with TDD PCell not configured with E-UTRA-NR Dual Connectivity operation.

The reporting range for SS-RSRP is defined from -156dBm to -31dBm with 1dB resolution. The mapping of the measured quantity to the reported value is defined by Table 4.7.1.0.1-2.

The normative reference for this requirement is TS 36.133 [23] clause 9.11.1.

8.5.2.1.0.1	Inter-RAT E-UTRA – NR FR1 SS-RSRP measurement accuracy requirements

The accuracy requirements of NR SS-RSRP measurements in FR1 and the corresponding side conditions shall be the same as the inter-frequency SS-RSRP absolute accuracy requirements in clause 4.7.1.0.3.

8.5.2.1.0.2	Inter-RAT E-UTRA – NR FR2 SS-RSRP minimum conformance requirements

The accuracy requirements of NR SS-RSRP measurements in FR2 and the corresponding side conditions shall be the same as the inter-frequency SS-RSRP absolute accuracy requirements in clause 5.7.1.0.2.

##### 8.5.2.1.1	SS-RSRP with NR FR1 target cell

###### 8.5.2.1.1.1	E-UTRA – NR FR1 SS-RSRP absolute measurement accuracy

8.5.2.1.1.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRP absolute measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.1.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

8.5.2.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.1.1.1.

8.5.2.1.1.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.1.1.1.4.1-1.

Table 8.5.2.1.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.1.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.1.1.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.1.1.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.1.1.1.4.1-2.

Table 8.5.2.1.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | Without faders |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.1.1.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.1.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

8.5.2.1.1.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.1.1.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in clause 8.5.2.1.1.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.1.1.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.1.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.1.1.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-rsrp | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.1.1.1.5	Test requirement

Table 8.5.2.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 8.5.2.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.1.1.1.5-2 for test configurations 1, 2, 4 and 5, and the corresponding absolute accuracy requirements in Table 8.5.2.1.1.1.5-3 for test configurations 3 and 6.

Table 8.5.2.1.1.1.5-1: SS-RSRP inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -94.65 |  | -117 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/15kHz | -96.00 |  | -117 + ΔBG_offset |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -94.65 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -93.00 |  | -114 + ΔBG_offset |  |
|  |  |  | dB | 10 |  | -3.2 |  |
|  |  |  | dB | 10 |  | -3.2 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -84.65 |  | -120.2 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -83.00 |  | -117.2 + ΔBG_offset |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -56.28 |  | -87.35 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -51.53 |  | -81.25 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

Table 8.5.2.1.1.1.5-2: SS-RSRP Inter RAT absolute accuracy requirements for the reported values for test configurations 1, 2, 4 and 5

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 30 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 31 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 32 |
|  |  | Bands NR_FDD_FR1_F | 33 |
|  |  | Bands NR_FDD_FR1_G | 33 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 43 |
|  |  | Bands NR_FDD_FR1_B | 43 |
|  |  | Bands NR_TDD_FR1_C | 44 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 44 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 45 |
|  |  | Bands NR_FDD_FR1_F | 45 |
|  |  | Bands NR_FDD_FR1_G | 46 |
|  |  | Bands NR_FDD_FR1_H | 46 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 26 |
|  |  | Bands NR_FDD_FR1_B | 26 |
|  |  | Bands NR_TDD_FR1_C | 27 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 27 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 28 |
|  |  | Bands NR_FDD_FR1_F | 28 |
|  |  | Bands NR_FDD_FR1_G | 29 |
|  |  | Bands NR_FDD_FR1_H | 29 |
| Highest reported value (Cell 2) | 85 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 47 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 48 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 49 |
|  |  | Bands NR_FDD_FR1_F | 50 |
|  |  | Bands NR_FDD_FR1_G | 50 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 8.5.2.1.1.1.5-3: SS-RSRP Inter RAT absolute accuracy requirements for the reported values for test configurations 3 and 6

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 64 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  | Bands NR_FDD_FR1_B | 34 |
|  |  | Bands NR_TDD_FR1_C | 34 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
|  |  | Bands NR_FDD_FR1_F | 36 |
|  |  | Bands NR_FDD_FR1_G | 36 |
|  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 46 |
|  |  | Bands NR_FDD_FR1_B | 46 |
|  |  | Bands NR_TDD_FR1_C | 47 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 47 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 48 |
|  |  | Bands NR_FDD_FR1_F | 48 |
|  |  | Bands NR_FDD_FR1_G | 49 |
|  |  | Bands NR_FDD_FR1_H | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 29 |
|  |  | Bands NR_FDD_FR1_B | 29 |
|  |  | Bands NR_TDD_FR1_C | 30 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 30 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 31 |
|  |  | Bands NR_FDD_FR1_F | 31 |
|  |  | Bands NR_FDD_FR1_G | 32 |
|  |  | Bands NR_FDD_FR1_H | 32 |
| Highest reported value (Cell 2) | 86 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  | Bands NR_FDD_FR1_B | 51 |
|  |  | Bands NR_TDD_FR1_C | 51 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 52 |
|  |  | Bands NR_FDD_FR1_F | 53 |
|  |  | Bands NR_FDD_FR1_G | 53 |
|  |  | Bands NR_FDD_FR1_H | 54 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

###### 8.5.2.1.1.2	Void

##### 8.5.2.1.2	E-UTRA – NR FR2 SS-RSRP measurement accuracy

8.5.2.1.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRP measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

8.5.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.1.2.

8.5.2.1.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.1.2.4.1-1.

Table 8.5.2.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.1.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.1.2.4.1-2.

Table 8.5.2.1.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.1.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.1.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.1.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.1.2.5-1 and Table 8.5.2.1.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in clause 8.5.2.1.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.1.2.5-1 and 8.5.2.1.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.1.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| rsrp | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.1.2.5	Test requirement

Table 8.5.2.1.2.5-1 defines the general test parameters and Table 8.5.2.1.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 8.5.2.1.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.1.2.5-3

Table 8.5.2.1.2.5-1: SS-RSRP Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |

Table 8.5.2.1.2.5-2: SS-RSRP Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 |
| Assumption for UE beamsNote 9 |  | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105.1 | N/A |
| Note1 | dBm/SCS Note4 | -96.07 | N/A |
|  | dB | 11 | N/A |
| Es | dBm/SCS Note 4 |  | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +6.65dB) |
| SSB_RPNote2 | dBm/SCS Note4 | -85.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +6.65dB) |
| BB Note 7 | dB | 9.95 | 1.84 |
| IoNote2 | dBm/95.04 MHz Note4 | -55.75 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +35.65dB) |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [3] Table 6.2.1.3-4. Note 8: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table 8.5.2.1.2.5-3: evaluation limits for the reported values

| UE power class 3 |  |  |  |
| --- | --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 48 | n257, n258, n261 | 29 |
|  |  | n260 | 32 |
|  |  | n259 | 33 |
| Highest reported value (Cell 2) | 105 | n257, n258, n261 | 87 |
|  |  | n260 | 89 |
|  |  | n259 | 90 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 45 | n257, n258, n261 | 26 |
|  |  | n260 | 29 |
|  |  | n259 | 30 |
| Highest reported value (Cell 2) | 108 | n257, n258, n261 | 90 |
|  |  | n260 | 92 |
|  |  | n259 | 93 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 8.5.2.2	SS-RSRQ

##### 8.5.2.2.0	Minimum conformance requirements

The measurement period of NR SS-RSRQ measurements is the same as the measurement period of NR SS-RSRP measurements defined in clause 8.5.2.1.0.

The reporting range of SS-RSRQ is defined from -43 dB to 20 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 4.7.2.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

The normative reference for this requirement is TS 36.133 [23] clause 9.11.2.

8.5.2.2.0.1	Inter-RAT E-UTRA – NR FR1 SS-RSRQ minimum conformance requirements

The accuracy requirements of NR SS-RSRQ measurements in FR1 and the corresponding side conditions shall be the same as the inter-frequency SS-RSRQ absolute accuracy requirements in clause 4.7.2.0.2.

8.5.2.2.0.2	Inter-RAT E-UTRA – NR FR2 SS-RSRQ minimum conformance requirements

The accuracy requirements of NR SS-RSRP measurements in FR2 and the corresponding side conditions shall be the same as the inter-frequency SS-RSRP absolute accuracy requirements in clause 5.7.2.0.2.

##### 8.5.2.2.1	E-UTRA – NR FR1 SS-RSRQ measurement accuracy

8.5.2.2.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRQ measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

8.5.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.2.1.

8.5.2.2.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.2.1.4.1-1.

Table 8.5.2.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.2.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.2.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.2.1.4.1-2.

Table 8.5.2.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.2.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.2.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

8.5.2.2.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.2.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in clause 8.5.2.2.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.2.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.2.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-rsrq | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.2.1.5	Test requirement

Table 8.5.2.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.2.1.5-2.

Table 8.5.2.2.1.5-1: SS-RSRQ inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -81.68 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3,6 |  |  | -87.80 |  | -113 |  | Same as Noc for 15kHz for Config 1,2,4,5 |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -81.68 |  | -106 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -84.80 |  | -110 |  | -113 + ΔBG_offset |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -81.93 |  | -107.75 |  | -117.75 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -85.02 |  | -111.75 |  | -114.75 + ΔBG_offset |  |
| SS-RSRQ Note3 |  |  | dB | -14.77 |  | -14.76 |  | -14.76 |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -50 |  | -75.83 |  | -85.83 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -50 |  | -76.73 |  | -79.73 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

Table 8.5.2.2.1.5-2: SS-RSRQ accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_51 | SS-RSRQ_51 | SS-RSRQ_51 |
| Highest reported value (Cell 2) | SS-RSRQ_63 | SS-RSRQ_63 | SS-RSRQ_63 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_48 | SS-RSRQ_48 | SS-RSRQ_48 |
| Highest reported value (Cell 2) | SS-RSRQ_66 | SS-RSRQ_66 | SS-RSRQ_66 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 8.5.2.2.2	E-UTRA – NR FR2 SS-RSRQ measurement accuracy

8.5.2.2.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRQ measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

8.5.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.2.2.

8.5.2.2.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.2.2.4.1-1.

Table 8.5.2.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.2.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.2.2.4.1-2.

Table 8.5.2.2.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.2.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.2.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.2.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.2.2.5-1 and Table 8.5.2.2.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in clause 8.5.2.2.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.2.2.5-1 and 8.5.2.2.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.2.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.2.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| rsrq | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.2.2.5	Test requirement

Table 8.5.2.2.2.5-1 defines the general test parameters and Table 8.5.2.2.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.2.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.2.2.5-3

Table 8.5.2.2.2.5-1: SS-RSRQ Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

Table 8.5.2.2.2.5-2: SS-RSRQ Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 | Setup 1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak -5dB) (Note 7) |
| Note1 | dBm/SCS Note4 | -95.97 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +4dB) (Note 7) |
|  | dB | -0.5 | 3.55 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.47 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +7.55dB) (Note 8) |
| SS-RSRQNote2 | dB | -14.06 | -12.38 |
| Note2 | dB | -0.5 | 3.55 |
| IoNote2 | dBm/95.04 MHz Note4 | -64.21 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +43.16dB) (Note 9) |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, SS-RSRQ, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Void Note 7: Noc for SCS 15kHz is applied at -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Noc for SCS 120kHz is applied at 4dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 8: SSB_RP is applied at 2.25dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 9: Io is applied at 10log10(792)+6.22dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 10: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation. |  |  |  |

Table 8.5.2.2.2.5-3: evaluation limits for the reported values

| Normal Conditions | Test 1 | Test 2 |
| --- | --- | --- |
| Lowest reported value (Cell 2) | 23 | 26 |
| Highest reported value (Cell 2) | 95 | 98 |
| Extreme Conditions | Test 1 | Test 2 |
| Lowest reported value (Cell 2) | 20 | 23 |
| Highest reported value (Cell 2) | 98 | 101 |

#### 8.5.2.3	SS-SINR

##### 8.5.2.3.0	Minimum conformance requirements

The measurement period of NR SS-SINR measurements is the same as the measurement period of NR SS-RSRP measurements defined in clause 8.5.2.1.0.

The reporting range of SS-SINR and CSI-SINR is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 4.7.3.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

The normative reference for this requirement is TS 36.133 [23] clause 9.11.3.

8.5.2.3.0.1	Inter-RAT E-UTRA – NR FR1 SS-SINR minimum conformance requirements

The accuracy requirements of NR SS-SINR measurements in FR1 and the corresponding side conditions shall be the same as the inter-frequency SS-SINR absolute accuracy requirements in clause 4.7.3.0.2.

8.5.2.3.0.2	Inter-RAT E-UTRA – NR FR2 SS-SINR minimum conformance requirements

The accuracy requirements of NR SS-SINR measurements in FR2 and the corresponding side conditions shall be the same as the inter-frequency SS-SINR absolute accuracy requirements in clause 5.7.3.0.2.

##### 8.5.2.3.1	E-UTRA – NR FR1 SS-SINR measurement accuracy

8.5.2.3.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-SINR measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA and ss-SINR-Meas.

8.5.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.3.1.

8.5.2.3.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.3.1.4.1-1.

Table 8.5.2.3.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.3.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.3.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.3.1.4.1-2.

Table 8.5.2.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.3.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.3.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

8.5.2.3.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.3.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the expected SS- SINR. If the value is outside the limits in clause 8.5.2.3.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.3.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.3.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-sinr | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.3.1.5	Test requirement

Table 8.5.2.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.3.1.5-2.

Table 8.5.2.3.1.5-1: SS-SINR inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -88 |  | -108.5 |  | -119.5 + ΔBG_offset |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -88 |  | -108.5 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -85 |  | -105.5 |  | -116.5 + ΔBG_offset |  |
|  |  |  | dB | -1.75 |  | 20 |  | -3.2 |  |
|  |  |  | dB | -1.75 |  | 20 |  | -3.2 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -89.75 |  | -88.5 |  | -122.7 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -86.75 |  | -85.5 |  | -119.7 + ΔBG_offset |  |
| SS-SINR Note3 |  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -57.83 |  | -60.5 |  | -89.85 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -51.73 |  | -54.41 |  | -83.75 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

Table 8.5.2.3.1.5-2: SS-SINR accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_35 | SS-SINR_79 | SS-SINR_32 |
| Highest reported value (Cell 2) | SS-SINR_51 | SS-SINR_94 | SS-SINR_49 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_33 | SS-SINR_77 | SS-SINR_31 |
| Highest reported value (Cell 2) | SS-SINR_53 | SS-SINR_96 | SS-SINR_50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 8.5.2.3.2	E-UTRA – NR FR2 SS-SINR measurement accuracy

8.5.2.3.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-SINR measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.3.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA and ss-SINR-Meas.

8.5.2.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.3.2.

8.5.2.3.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.3.2.4.1-1.

Table 8.5.2.3.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.3.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.3.2.4.1-2.

Table 8.5.2.3.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.3.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.3.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.3.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.3.2.5-1 and Table 8.5.2.3.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the expected SS- SINR. If the value is outside the limits in clause 8.5.2.3.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.3.2.5-1 and 8.5.2.3.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.3.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.3.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| sinr | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.3.2.5	Test requirement

Table 8.5.2.3.2.5-1 defines the general test parameters and Table 8.5.2.3.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.3.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.3.2.5-3.

Table 8.5.2.3.2.5-1: SS-SINR Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 | freq1 |
| Duplex mode |  | TDD | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |
| PDSCH Reference measurement channel |  | - | - | - |
| RMSI CORESET Reference Channel |  | - | - | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table 8.5.2.3.2.5-2: SS-SINR Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 | Setup 1 | Setup 1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105 | -105.1 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak -5dB) (Note 7) |
| Note1 | dBm/SCS Note4 | -95.97 | -96.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +4dB) (Note 7) |
|  | dB | -0.5 | 11 | 3.7 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.47 | -85.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +7.6dB) (Note 8) |
| SS-SINRNote2 | dB | -0.5 | 11 | 3.6 |
| Note2 | dB | -0.5 | 11 | 3.6 |
| IoNote2 | dBm/95.04 MHz Note4 | -64.21 | -55.75 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +43.19dB) (Note 9) |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, SS-SINR, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Void Note 7: Noc for SCS 15kHz is applied at -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Noc for SCS 120kHz is applied at 4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 8: SSB_RP is applied at 3dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 9: Io is applied at level 10log10(792)+6.54dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 10: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation. |  |  |  |  |

Table 8.5.2.3.2.5-3: evaluation limits for the reported values

| Normal Conditions | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 9 | 32 | 17 |
| Highest reported value (Cell 2) | 82 | 105 | 90 |
| Extreme Conditions | Test 1 | Test 2 | Test 3 |
| Lowest reported value (Cell 2) | 7 | 30 | 15 |
| Highest reported value (Cell 2) | 84 | 107 | 92 |

Annex A (normative):
RRM test configurations

# A.1	Reference measurement channels

This section contains the Reference Measurement Channels (RMC) to be used for the RRM test scenarios in Sections 4 to 7 of this document.

## A.1.1	PDSCH

### A.1.1.1	FDD

Table A.1.1.1-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.1.1 FDD |
| Channel bandwidth | MHz | 10 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  | 10 |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 10 |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

### A.1.1.2	TDD

Table A.1.1.2-1: PDSCH Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.1.1 TDD |
| Channel bandwidth | MHz | 10 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  |  |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 4 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

Table A.1.1.2-2: PDSCH Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | SR.2.1 TDD |
| Channel bandwidth | MHz | 40 |
| Number of transmitter antennas |  | 1 |
| Allocated resource blocks for PDSCH Note 1 |  | 24 |
| Allocated slots per Radio Frame |  |  |
| Radio frame containing SSB | slots | Note 5 |
| Radio frame not containing SSB | slots | 10 |
| MCS table |  | 64QAM |
| MCS index |  | 4 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| Number of control symbols |  | 2 |
| PDSCH mapping type |  | Type A |
| Information Bit Payload |  |  |
| For slots with RMSI Note 2 | Bits | 1608 |
| For slots without RMSI | Bits | 1864 |
| Number of Code Blocks per slot |  | 1 |
| Binary Channel Bits Per slot |  |  |
| For slots with RMSI Note 2, 4 | Bits | 5184 |
| For slots without RMSI Note 6 | Bits | 6048 |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. |  |  |

Table A.1.1.2-3: PDSCH Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | SR.3.1 TDD | SR.3.2 TDD | SR.3.3 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 |
| Number of transmitter antennas |  | 1 | 1 | 1 |
| Allocated resource blocks for PDSCH |  | 24Note 1 | 24Note 7 | 48Note 7 |
| Allocated slots per Radio Frame |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 8 | Note 5 |
| Radio frame not containing SSB | slots | 48 | 48 | 48 |
| MCS table |  | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Number of control symbols |  | 2 | 2 | 2 |
| PDSCH mapping type |  | Type A | Type A | Type A |
| Information Bit Payload |  |  |  |  |
| For slots with RMSI | Bits | 1608 | 1608 | 3104 |
| For slots without RMSI | Bits | 1864 | 1864 | 3624 |
| Number of Code Blocks per slot |  | 1 | 1 | 1 |
| Binary Channel Bits Per slot |  |  |  |  |
| For slots with RMSI Note 4 | Bits | 5184 | 5184 | 10368 |
| For slots without RMSI Note 6 | Bits | 6048 | 6048 | 12096 |
| Note 1: Allocated in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: Void Note 3: If necessary, the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in 3GPP TS 38.213 [8]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in section A.3.10. Note 6: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditionalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 1. Note 7: Allocated in the same resource blocks as the CORESET. Note 8: When DRX is configured, PDSCH is scheduled only while drx-onDurationTimer is running, unless otherwise specified in the test case. |  |  |  |  |

## A.1.2	CORESET for RMSI scheduling

### A.1.2.1	FDD

Table A.1.2.1-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.1.1 FDD |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |
| Allocated resource blocks for RMSI CORESETNote 7 |  | 24 |
| Subcarrier spacing for SSB | kHz | 15 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESETNote 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

### A.1.2.2	TDD

Table A.1.2.2-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.1.1 TDD |
| Channel bandwidth | MHz | 10 |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |
| Index of transmitted SSB within an SS-Burst |  | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESET Note 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

Table A.1.2.2-2: RMSI CORESET Reference Measurement Channels for SCS = 30 kHz for TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | CR.2.1 TDD |
| Channel bandwidth | MHz | 40 |
| Subcarrier spacing for RMSI CORESET | kHz | 30 |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 |
| Index of transmitted SSB within an SS-Burst |  | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |
| Number of transmitter antennas |  | 1 |
| Duration of RMSI CORESET Note 7 | symbols | 2 |
| DCI Format Note 1 |  | Note 2 |
| Aggregation level | CCE | 8 |
| DMRS precoder granularity |  | 6 |
| REG bundle size |  | 6 |
| Mapping from REG to CCE |  | Distributed |
| Cell ID |  | Note 5 |
| Payload (without CRC) | Bits | Note 6 |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-6 in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |

Table A.1.2.2-3: RMSI CORESET Reference Measurement Channels for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | CR.3.1 TDD | CR.3.2 TDD |
| Channel bandwidth | MHz | 100 | 100 |
| Subcarrier spacing for RMSI CORESET | kHz | 120 | 120 |
| Allocated resource blocks for RMSI CORESET |  | 24Note 7 | 48Note 9 |
| Subcarrier spacing for SSB | kHz | 120 | 120 |
| Index of transmitted SSB within an SS-Burst |  | #0 | #0 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1Note 7 | Pattern 1Note 9 |
| Offset between SSB and RMSI CORESET Note 3 | RB | 0 (Note 8) Note 7 | 0 (Note 8)Note 9 |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |
| Number of transmitter antennas |  | 1 | 1 |
| Duration of RMSI CORESET | symbols | 2Note 7 | 2Note 9 |
| DCI Format Note 1 |  | Note 2 | Note 2 |
| Aggregation level | CCE | 8 | 8 |
| DMRS precoder granularity |  | 6 | 6 |
| REG bundle size |  | 6 | 6 |
| Mapping from REG to CCE |  | Distributed | Distributed |
| Cell ID |  | Note 5 | Note 5 |
| Payload (without CRC) | Bits | Note 6 | Note 6 |
| Note 1: DCI formats are defined in TS 38.212 [31]. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-8 in TS 38.213 [8]. Note 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC Note 9: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 2 in Table 13-10 in TS 38.213 [8]. |  |  |  |

## A.1.3	CORESET for RMC scheduling

### A.1.3.1	FDD

Table A.1.3.1-1: Control Channel RMC for SCS = 15 kHz for FDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 | 15 | 15 |
| Allocated resource blocks for CORESET |  | 24 | 18 | 24 | 18 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 2 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

### A.1.3.2	TDD

Table A.1.3.2-1: Control Channel RMC for SCS = 15 kHz for TDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 TDD | CCR.1.2 TDD | CCR.1.3 TDD | CCR.1.4 TDD |
| Channel bandwidth | MHz | 10 | 10 | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Allocated resource blocks for CORESET Note3 |  | 24 | 18 | 24 | 18 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 2 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |

Table A.1.3.2-2: Control Channel RMC for SCS = 30 kHz for TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | CCR.2.1 TDD | CCR.2.2 TDD | CCR.2.3 TDD |
| Channel bandwidth | MHz | 40 | 40 | 40 |
| Subcarrier spacing for RMSI CORESET | kHz | 30 | 30 | 30 |
| Allocated resource blocks for CORESET3 |  | 24 | 24 | 18 |
| Number of transmitter antenna |  | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 |
| REG bundle size |  | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration. Note 3: Allocated in the same resource blocks where the associated RMC is scheduled. |  |  |  |  |

Table A.1.3.2-3: Control Channel RMC for SCS = 120 kHz for TDD

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.3.1 TDD | CCR.3.2 TDD | CCR.3.3 TDD | CCR.3.4 TDD | CCR.3.5 TDD | CCR.3.6 TDD | CCR.3.7 TDD |
| Channel bandwidth | MHz | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| Subcarrier spacing for RMSI CORESET | kHz | 120 | 120 | 120 | 120 | 120 | 120 | 120 |
| Allocated resource blocks for CORESETNote 3 |  | 24 | 24 | 24 | 24 | 24 | 24 | 48 |
| Number of transmitter antenna |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| monitoringSlotPeriodicityAndOffsetNote 4 |  | sl160 0 | sl160 0 | sl160 80 | sl160 0 | sl160 0 | sl160 80 | sl160 0 |
| monitoringSymbolsWithinSlot |  | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 | 0011000 0000000 | 1100000 0000000 | 1100000 0000000 |
| Duration of CORESET | symbols | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 4 | 4 | 8 | 8 | 8 | 4 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the same resource blocks where the associated PDSCH RMC is scheduled. Note 4: monitoringSlotPeriodicityAndOffet can be set to other values depending on test purpose of respective test cases, as needed, e.g. monitoringSlotPeriodicityAndOffet is set to “sl1 0” if it is specifically stated that cell(s) configured with one of the control channel RMCs above shall transmit PDCCHs continuously. |  |  |  |  |  |  |  |  |

## A.1.4	CSI-RS

### A.1.4.1	FDD

Table A.1.4.1-1: CSI-RS Reference Measurement Channels for SCS = 15 kHz for FDD

|  | CSI-RS.1.1 FDD | CSI-RS.1.2 FDD | CSI-RS.1.3 FDD | CSI-RS.1.4 FDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 4 | 4 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | N/A | N/A |
| Offset | 1 | 1 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

### A.1.4.2	TDD

Table A.1.4.2-1: CSI-RS Reference Measurement Channels for SCS = 15 kHz for TDD

|  | CSI-RS.1.1 TDD | CSI-RS.1.2 TDD | CSI-RS.1.3 TDD | CSI-RS.1.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 4 | 4 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot5 | slot10 | N/A | N/A |
| Offset | 1 | 1 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 4 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

Table A.1.4.2-2: CSI-RS Reference Measurement Channels for SCS = 30 kHz for TDD

|  | CSI-RS.2.1 TDD | CSI-RS.2.2 TDD | CSI-RS.2.3 TDD | CSI-RS.2.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 4 | 4 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot10 | slot20 | N/A | N/A |
| Offset | 2 | 2 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

Table A.1.4.2-3: CSI-RS Reference Measurement Channels for SCS = 120 kHz for TDD

|  | CSI-RS.3.1 TDD | CSI-RS.3.2 TDD | CSI-RS.3.3 TDD | CSI-RS.3.4 TDD |
| --- | --- | --- | --- | --- |
| Resource Type | periodic | periodic | aperiodic | aperiodic |
| Resource Set Config |  |  |  |  |
| nzp-CSI-ResourceSetId | 0 | 0 | 0 | 0 |
| repetition | N/A | off | off | on |
| aperiodicTriggeringOffset | N/A | N/A | 4 | 4 |
| trs-Info | N/A | N/A | N/A | N/A |
| Resource Config |  |  |  |  |
| nzp-CSI-RS-ResourceId | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 1 for resource #1 | 1 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| powerControlOffset | 0 | 0 | 0 | 0 |
| powerControlOffsetSS | db0 | db0 | db0 | db0 |
| scramblingID | 0 | 0 | 0 | 0 |
| Period (slots) | slot40 | slot80 | N/A | N/A |
| Offset | 8 | 8 | N/A | N/A |
| qcl-InfoPeriodicCSI-RS | TCI.State.0 | TCI.State.0 | N/A | N/A |
|  |  | TCI.State.1 |  |  |
| frequencyDomainAllocation | 000001 | 0001 | 0001 | 0001 |
| nrofPorts | 2 | 1 | 1 | 1 |
| firstOFDMSymbolInTimeDomain | 5 for resource #0 | 6 for resource #0 | 6 for resource #0 | 0 for resource #0 |
|  |  |  |  | 1 for resource #1 |
|  |  |  |  | 2 for resource #2 |
|  |  |  |  | 3 for resource #3 |
|  |  | 10 for resource #1 | 10 for resource #1 | 4 for resource #4 |
|  |  |  |  | 5 for resource #5 |
|  |  |  |  | 6 for resource #6 |
|  |  |  |  | 7 for resource #7 |
| cdm-Type | FD-CDM2 | noCDM | noCDM | noCDM |
| density | 1 | 3 | 3 | 3 |
| startingRB | 0 | 0 | 0 | 0 |
| nrofRBs | 276 | 276 | 276 | 276 |

## A.1.4A	CSI-RS for tracking

### A.1.4A.1	FR1

#### A.1.4A.1.1	FDD

Table A.1.4A.1.1-1: CSI-RS for tracking FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 FDD | TRS.1.2 FDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

#### A.1.4A.1.2	TDD

Table A.1.4A.1.2-1: CSI-RS for TDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.1.1 TDD | TRS.1.2 TDD |
| Bandwidth |  | BW of Active BWP1 | BW of Active BWP1 |
| SCS | kHz | 15 | 30 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 20 for CSI-RS resource 1,2,3,4 | 40 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 | 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 |
| TCI state |  | TCI.State.0 | TCI.State.0 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases |  |  |  |

### A.1.4A.2	FR2

#### A.1.4A.2.1	TDD

Table A.1.4A.2.1-1: CSI-RS for tracking for TDD FR2

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | TRS.2.1 TDD | TRS.2.2 TDD |
| Bandwidth |  | BW of Active BWPNote 1, 2 | BW of Active BWPNote 1,2 |
| SCS | kHz | 120 | 120 |
| First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 | k0=0 for CSI-RS resource 1,2,3,4 |
| First OFDM symbol in the slot used for CSI-RS |  | l0 = 1 for CSI-RS resource 1 and 3 l0 = 5 for CSI-RS resource 2 and 4 | l0 = 2 for CSI-RS resource 1 and 3 l0 = 6 for CSI-RS resource 2 and 4 |
| Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 1,2,3,4 |
| CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 | ‘No CDM’ for CSI-RS resource 1,2,3,4 |
| Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 | 3 for CSI-RS resource 1,2,3,4 |
| CSI-RS periodicity | slots | 80 for CSI-RS resource 1,2,3,4 | 80 for CSI-RS resource 1,2,3,4 |
| CSI-RS offset | slots | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 | 40 for CSI-RS resource 1 and 2 41 for CSI-RS resource 3 and 4 |
| EPRE ratio to SSS | dB | 0 | 0 |
| TCI state |  | TCI.State.0 | TCI.State.1 |
| Note 1: BW of TRS is configured same as the BW size of UE active BWP in the RRM test cases Note 2: If active BWP is larger than 52RBs, BW of TRS is configured as 52RBs. Otherwise, same as active BWP size. |  |  |  |

## A.1.5	TDD UL/DL configuration

Table A.1.5-1: TDD UL/DL configuration for SCS=15kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.1.1 |  |  |
| referenceSubcarrierSpacing | kHz | 15 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DSUU’ S=‘10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 10 |  |  |
| nrofUplinkSlot |  | 2 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘D’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 1 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| Note 1: As specified in TS 38.213 [8] and TS 38.331 [13]. Note 2: For information |  |  |  |  |

Table A.1.5-2: TDD UL/DL configuration for SCS=30kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.2.1 |  |  |
| referenceSubcarrierSpacing | kHz | 30 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘3D1S4U’ S=‘6DL:4GP:4UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 4 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 6 |  |  |
| nrofUplinkSlot |  | 4 |  |  |
| nrofUplinkSymbols |  | 4 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | ‘DD’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 1 |  |  |
| nrofDownlinkSlots |  | 2 |  |  |
| nrofDownlinkSymbols |  | 0 |  |  |
| nrofUplinkSlot |  | 0 |  |  |
| nrofUplinkSymbols |  | 0 |  |  |
| Note 1: As specified in TS 38.213 [8] and TS 38.331 [13]. Note 2: For information |  |  |  |  |

Table A.1.5-3: TDD UL/DL configuration for SCS=120kHz

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | TDDConf.3.1 |  |  |
| referenceSubcarrierSpacing | kHz | 120 |  |  |
| TDD UL/DL pattern 1 Note 2 |  | ‘DDDSU’ S=‘10DL:2GP:2UL’ |  |  |
| dl-UL-TransmissionPeriodicity | ms | 0.625 |  |  |
| nrofDownlinkSlots |  | 3 |  |  |
| nrofDownlinkSymbols |  | 10 |  |  |
| nrofUplinkSlot |  | 1 |  |  |
| nrofUplinkSymbols |  | 2 |  |  |
| TDD UL/DL pattern 2 Note 2 |  | Not configured |  |  |
| dl-UL-TransmissionPeriodicity | ms | Not configured |  |  |
| nrofDownlinkSlots |  | Not configured |  |  |
| nrofDownlinkSymbols |  | Not configured |  |  |
| nrofUplinkSlot |  | Not configured |  |  |
| nrofUplinkSymbols |  | Not configured |  |  |
| Note 1: As specified in TS 38.213 [8] and TS 38.331 [13]. Note 2: For information |  |  |  |  |

## A.1.6	PUSCH

This rule applies to NR cell(s), which the UE is connected to. The UE is in RRC_CONNECTED mode.

When signalling or data payloads are expected to be sent on the PUSCH, the UE may be provided in advance with PUSCH resources by the SS. For sake of simplicity, the PUSCH scheduling may also occur continuously over many consecutive subframes. These options shall not be used if:
1) stated otherwise in the test description, or 
2) the transmission of PUSCH and UL scheduling information affects the test purpose (e.g. DRX, PUCCH reception etc.)

# A.2	Reference OCGN configuration

## A.2.1 Generic OFDMA channel noise generator (OCGN)

The OCGN pattern is used in a test for modelling the allocation of unused resourced in the channel bandwidth to virtual UEs (UEs that are not under test). The OCNG pattern simulates both PDCCH and PDSCH transmissions to the virtual UEs.

Table A.2.1-1: OP.1: Generic OCNG pattern for all unused REs

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. |  |  |

Table A.2.1-2: OP.2: Generic OCNG pattern for all unused REs for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. Note 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

Table A.2.1-3: OP.3: Generic OCNG pattern for unused REs in the same BW as CORESET

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. |  |  |

Table A.2.1-4: OP.4: Generic OCNG pattern for all unused REs outside SSB slot(s)

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell. REs for OCNG shall not be allocated in the slot(s) containing SSB of the respective cell. |  |  |

Table A.2.1-5: OP.5: Generic OCNG pattern for unused REs in the same BW as CORESET for 2AoA setup

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Probe | Transmitting the serving beam |  |
| Resource allocation | Unused REs (Note 1) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. | Unused REs (Note 2) in the symbols where SSB/CSI-RS are not transmitted from both the serving beam probe and non-serving beam probe. |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Subcarrier spacing | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Aggregation level | Same as used in PDCCH RMC | N/A |
| Code rate | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Transmit Power | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| CP length | Same as used in PDCCH RMC | Same as used in PDSCH RMC |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the allocated bandwidth of the CORESET of the serving cell. REs for OCNG shall not be allocated outside the allocated bandwidth of the CORESET of the serving cell. Note 3: No OCNG is transmitted from the probe transmitting non-serving beam. |  |  |

# A.3	Reference SSB configuration

## A.3.1	SSB configuration for FR1

Table A.3.1-1: SSB allocation for FR1

| SSB Parameters | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  | SSB.1 FR1 | SSB.2 FR1 | SSB.3 FR1 |  | SSB.4 FR1 |  | SSB.5 FR1 | SSB.6 FR1 |
| Channel bandwidth | MHz | 10 | 40 | 10 |  | 40 |  | 10 | 40 |
| SSB SCS | kHz | 15 | 30 | 15 |  | 30 |  | 15 | 30 |
| SSB periodicity (TSSB) | ms | 20 | 20 | 20 |  | 20 |  | 20 | 20 |
| Number of SSBs per SS-burst |  | 1 | 1 | 2 |  | 2 |  | 1 | 1 |
| SS/PBCH block index |  | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| Indices of symbols containing SSB |  | 2-5 | 4-7 | 2-5 | 8-11 | 2-5 | 8-11 | 2-5 | 4-7 or 2-5 Note 2 |
| Indices of slots containing SSB |  | 0 | 0 | - |  | - |  | 0 | 0 |
| Indices of SFN containing SSB |  | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |  |  | SFN mod (max(TSSB,10ms)/10ms) = 1 |  |
| RB numbers containing SSB within channel BW |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  |  |  |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. Note 2: Symbols 4-7 are chosen if the SSB pattern Case B should be used for the current band as indicated by Table 5.4.3.3-1 of TS 38.104 [28]; Otherwise, symbols 2-5 are chosen. |  |  |  |  |  |  |  |  |  |

## A.3.2	SSB configuration for FR2

Table A.3.2-1: SSB allocation for FR2

| SSB Parameters | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB Pattern |  | SSB.1 FR2 |  | SSB.2 FR2 |  | SSB.3 FR2 | SSB.4 FR2 | SSB.5 FR2 |  | SSB.6 FR2 |  | SSB.7 FR2 | SSB.8 FR2 |
| Channel bandwidth | MHz | 100 |  | 100 |  | 100 | 100 | 100 |  | 100 |  | 100 | 100 |
| SSB SCS | kHz | 120 |  | 240 |  | 120 | 240 | 120 |  | 240 |  | 120 | 240 |
| SSB periodicity (TSSB) | ms | 20 |  | 20 |  | 20 | 20 | 20 |  | 20 |  | 20 | 20 |
| Number of SSBs per SS-burst |  | 2 |  | 2 |  | 1 | 1 | 2 |  | 2 |  | 1 | 1 |
| SS/PBCH block index |  | 0 | 1 | 0 | 1 | 0 | 0 | 2 | 3 | 2 | 3 | 1 | 1 |
| Indices of symbols containing SSB |  | 4-7 | 8-11 | 8-11 | 12-13, 0-1 | 4-7 | 8-11 | 2-5 | 6-9 | 2-5 | 6-9 | 8-11 | 12-13, 0-1 |
| Indices of slots containing SSB |  | 0 |  | 0 |  | 0 | 0 | 1 |  | 1 |  | 0 | 0 |
| Indices of SFN containing SSB |  | SFN mod (max(TSSB,10ms)/10ms) = 0 |  |  |  |  |  |  |  |  |  |  |  |
| RB numbers containing SSB within channel BW |  | (RBJ, RBJ+1,.…, RBJ+19)Note 1 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: RBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

# A.4	Reference SMTC configuration

Table A.4-1: SMTC configurations

| SMTC Parameters | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SMTC Pattern |  | SMTC.1 | SMTC.2 | SMTC.3 | SMTC.4 | SMTC.5 | SMTC.6 |
| SMTC periodicity | ms | 20 | 20 | 160 | 20 | 20 | 20 |
| SMTC offset | ms | 0 | 0 | 0 | 10 | 10 | 17 |
| SMTC duration | ms | 1 | 5 | 1 | 1 | 5 | 5 |

# A.5	Reference DRX configurations

The reference DRX configurations for the NR serving cell are captured in Table A.5-1. The reference DRX configurations for the E-UTRA serving cell for NSA and inter-RAT test cases are captured in Table A.5-2.

Table A.5-1: DRX configurations for NR serving cell

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRX Configuration |  | DRX.1 | DRX.2 | DRX.3 | DRX.6 | DRX.7 | DRX.8 | DRX.11 |
| drx-onDurationTimer | ms | 1 | 1 | 6 | 1 | 6 | 6 | 6 |
| drx-InactivityTimer | ms | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-RetransmissionTimerDL | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-RetransmissionTimerUL | slot | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| drx-LongCycleStartOffset | ms | 40 | 640 | 40 | 320 | 640 | 320 | 20 |
| shortDRX | - | disabled | disabled | disabled | disabled | disabled | disabled | disabled |
| TimeAlignmentTimer | ms | 500 | 500 | Infinity | 500 | Infinity | Infinity | Infinity |
| Note 1: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |  |  |  |

Table A.5-2: DRX configurations for E-UTRA serving cell

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DRX Configuration |  | DRX.4 | DRX.5 | DRX.9 | DRX.10 |
| drx-onDurationTimer | ms | psf2 | psf6 | psf2 | psf6 |
| drx-InactivityTimer | ms | psf2 | psf1920 | psf100 | psf1920 |
| drx-RetransmissionTimerDL | slot | psf16 | psf16 | psf16 | psf16 |
| drx-LongCycleStartOffset | ms | sf160, 0 | sf320. 0 | sf40, 0 | sf640. 0 |
| shortDRX | - | disabled | disabled | disabled | disabled |
| TimeAlignmentTimer | ms | Infinity | Infinity | 500 | 500 |
| Note 1: The DRX cycle and time alignment timer parameters are specified in clause 6.3.2 in TS 36.331 [29] |  |  |  |  |  |  |

# A.6	EN-DC test setup

The purpose of this Annex is to specify the EN-DC configuration for the test cases in Chapters 4 and 5 of this test specification.

## A.6.1	E-UTRA serving cell parameters

This section defines the cell power levels and other specific cell parameters of the E-UTRA serving cell for EN-DC.

### A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1

Table A.6.1.1-1 defines the E-UTRA serving cell parameters for EN-DC tests with NR FR1 cell(s), defined in Chapter 4 of this test specification. Unless otherwise stated within the test, all measurements in Clauses 4 and 5 are performed only on the NR carrier. The E-UTRA PCell shall configured to not interfere with NR operation and the E-UTRA PCell signal power shall not be critical to the test purpose.

Table A.6.1.1-1: E-UTRAN cell specific test parameters for EN-DC tests with NR FR1

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel Note 6 |  | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2, Note 6 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2, Note 6 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG PatternsNote 2 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote 3 | dB |  |
| OCNG_RBNote 3 | dB |  |
| NocNote 4 | dBm/15 kHz | -104 |
| Ês/Noc | dB | 17 |
| Ês/Iot | dB | 17 |
| RSRP Note 5 | dBm/15 kHz | -87 |
| SCH_RP Note 5 | dBm/15 kHz | -87 |
| Io Note 5 | dBm/Ch BW | -59.13 + 10log(NRB,c /50) |
| Propagation Condition |  | AWGN |
| Antenna Configuration |  | 1x2 |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in sections A.1, A.2 and D.1 of TS 36.521-3 [26]. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Es/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: For E-UTRA anchor configuration, pick 5 MHz as default channel bandwidth setting in the tests as it is supported by all E-UTRA bands. If none of the UE supported EN-DC band combos support 5MHz E-UTRA carrier, pick 20 MHz channel BW or 10 MHz channel BW, in that order, |  |  |

Table A.6.1.1-2: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table A.6.1.1-3:PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| } |  |  |  |  |

Table A.6.1.1-4:MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

### A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2

Table A.6.1.2-1 defines the E-UTRA serving cell parameters for EN-DC tests with NR FR2 cell(s), defined in Chapter 5 of this test specification. Unless otherwise stated within the test, all measurements in Clauses 6 and 7 are performed only on the NR carrier. The E-UTRA PCell shall configured to not interfere with NR operation and the E-UTRA PCell signal power shall not be critical to the test purpose.

Table A.6.1.2-1: E-UTRAN cell specific test parameters for EN-DC tests with NR FR2

| Parameter | Unit | E-UTRAN Cell1 |
| --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 |
| Duplex mode |  | FDD or TDD |
| TDD special subframe configurationNote1 |  | 6 |
| TDD uplink-downlink configurationNote1 |  | 1 |
| BWchannel Note 5 | MHz | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |
| PDSCH parameters: DL Reference Measurement ChannelNote2, Note 5 |  | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2, Note 5 |  | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |
| OCNG PatternsNote2, Note 5 |  | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |
| PBCH_RA | dB | 0 |
| PBCH_RB | dB |  |
| PSS_RA | dB |  |
| SSS_RA | dB |  |
| PCFICH_RB | dB |  |
| PHICH_RA | dB |  |
| PHICH_RB | dB |  |
| PDCCH_RA | dB |  |
| PDCCH_RB | dB |  |
| PDSCH_RA | dB |  |
| PDSCH_RB | dB |  |
| OCNG_RANote3 | dB |  |
| OCNG_RBNote3 | dB |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in sections A 1, A.2 and D.1 of TS 36.521-3 [26]. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: The E-UTRA signal is required only to ensure the E-UTRA link to the DUT in the EN-DC operation. The Test System shall provide a stable and noise-free E-UTRA signal without need of precise propagation modelling, path loss and polarization control. Further details of the E-UTRA signal configuration are not defined as part of the cell specific test parameters, since the E-UTRA link is not under performance verification and is not expected to influence the NR FR2 requirement. Note 5: The For E-UTRA anchor configuration, pick 5 MHz as default channel bandwidth setting in the tests as it is supported by all E-UTRA bands. If none of the UE supported EN-DC band combos support 5MHz E-UTRA carrier, pick 20 MHz channel BW or 10 MHz channel BW, in that order, |  |  |

Table A.6.1.2-2: CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | NOT PRESENT |  |  |
| cqi-ReportPeriodic | NOT PRESENT |  |  |
| } |  |  |  |

Table A.6.1.2-3:PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| } |  |  |  |  |

Table A.6.1.2-4:MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated | Infinity |  |  |

# A.6A	NR FR1-FR2 test setup

Some test cases in clause 7 have NR cells in both FR1 and FR2. Unless otherwise stated within the test, the NR FR1 Cell signal is required only to provide a link to the UE under test. The Test Equipment shall provide a stable and noise-free NR FR1 signal without need of precise propagation modelling, path loss and polarization control. Further details of the NR FR1 signal configuration are not defined as part of the cell specific test parameters, since the NR FR1 link is not under performance verification and is not expected to influence the test purpose.

# A.7	Reference PRACH configurations

This section provides the typical PRACH configurations used for all RRM test cases defined in this test specification. Parameters not listed in this section can be derived from the configuration of each test.

## A.7.1	PRACH configurations for FR1

Table A.7.1-1 defines the PRACH configurations for FR1. Each of the PRACH configurations defined in Table A.7.1-1 have different applicabilities:

-	PRACH.1 FR1 for SSB-based contention based random access in FR1.

-	PRACH.2 FR1 for SSB-based non-contention based random access in FR1.

-	PRACH.3 FR1 for CSI-RS based non-contention based random access in FR1.

-	PRACH.4 FR1 for CSI-RS based non-contention based random access in FR1 to convey BFR.

Table A.7.1-1 Parameters for PRACH Configurations for FR1

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR1 | PRACH.2 FR1 | PRACH.3 FR1 | PRACH.4 FR1 |  |
| prach-ConfigurationIndex | 102 | 102 | 102 | 102 | 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-2 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | 48 | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | 48 | 48 | No group B. |
| prach-RootSequenceIndex | 0 | 0 | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | - | - | - | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | - | oneFourth | oneFourth | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One | One | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | RSRP_51 | N/A | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A | N/A | RSRP_51 | N/A |  |
| ra-ContentionResolutionTimer | sf48 | - | - | - | 48 sub-frames |
| powerRampingStep | dB2 | dB2 | dB2 | dB2 |  |
| preambleReceivedTargetPower | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | n6 | n200 | Max number of RA preamble transmission performed before declaring a failure is 6 |
| ra-ResponseWindow | sl10 | sl10 | sl10 | sl1 | 10 slots |
| zeroCorrelationZoneConfig | 11 | 11 | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE doesn’t use this field if is transmitting CFRA to convey BFR. |
| BFR-SSB-Resource | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0. UE doesn’t use ssb-ResourceList and BFR-SSB-Resource IEs at the same time. UE uses this field only if is transmitting CFRA to convey BFR |
| csirs-ResourceList | N/A | N/A | present | presnt | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A | N/A | 50 | 50 | Associated with CSI-RS configured |
| ra-OccasionList | - | - | 1 | 1 | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - | 1 | N/A | N/A | PRACH occasion index 1 is allowed |
| Note: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |

## A.7.2	PRACH configurations for FR2

Table A.7.2-1 defines the PRACH configurations for FR2. Each of the PRACH configurations defined in Table A.7.2-1 have different applicabilities:

-	PRACH.1 FR2 for SSB-based contention based random access in FR2.

-	PRACH.2 FR2 for SSB-based non-contention based random access in FR2.

-	PRACH.3 FR2 for CSI-RS based non-contention based random access in FR2.

-	PRACH.4 FR2 for CSI-RS based non-contention based random access in FR2 to convey BFR.

Table A.7.2-1 Parameters for PRACH Configurations for FR2

| Field | Value |  |  |  | Comment |
| --- | --- | --- | --- | --- | --- |
| PRACH Configuration | PRACH.1 FR2 | PRACH.2 FR2 | PRACH.3 FR2 | PRACH.4 FR2 |  |
| prach-ConfigurationIndex | 190 | 190 | 190 | 190 | Preamble format C2, 10ms PRACH periodicity and other detailed configuration defined in table 6.3.3.2-4 in TS 38.211 [7]. |
| msg1-SubcarrierSpacing | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS | Same as UL carrier SCS |  |
| totalNumberOfRA-Preambles | 48 | 48 | 48 | 48 | Total number of preambles used for contention based and contention free random access |
| numberOfRA-PreamblesGroupA | 48 | 48 | 48 | 48 | No group B. |
| prach-RootSequenceIndex | 0 | 0 | 0 | 0 | Logic sequence index = 0, resulting in root sequence = 1. |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB | oneFourth, n48 | N/A | N/A | N/A | OneFourth: 1 SSB associated with 4 RACH occasions n48: 48 contention based preambles per SSB |
| ssb-perRACH-Occasion | N/A | oneFourth | oneFourth | oneFourth | OneFourth: 1 SSB associated with 4 RACH occasions |
| msg1-FDM | One | One | One | One | One PRACH transmission occasions FDMed in one time instance. |
| rsrp-ThresholdSSB | RSRP_51 | RSRP_51 | N/A | RSRP_51 | The actual value of the threshold is -105dBm, as defined in TS 38.331 [13]. |
| rsrp-ThresholdCSI-RS | N/A | N/A | RSRP_51 | N/A |  |
| ra-ContentionResolutionTimer | sf48 | N/A | N/A | N/A | 48 sub-frames |
| powerRampingStep | dB2 | dB2 | dB2 | dB2 |  |
| preambleReceivedTargetPower | dBm-120 | dBm-120 | dBm-120 | dBm-120 |  |
| preambleTransMax | n6 | n6 | n6 | n200 | Max number of RA preamble transmission performed before declaring a failure |
| ra-ResponseWindow | sl10 | sl10 | sl10 | sl40 |  |
| zeroCorrelationZoneConfig | 11 | 11 | 11 | 11 | N-CS configuration, NCS = 23 |
| Backoff Parameter Index | 2 | 2 | 2 | 2 | 20ms, as defined in table 7.2-1 in TS 38.321 [12]. |
| ssb-ResourceList | - | present | N/A | N/A | Associated with SSB index 0 |
| ra-PreambleIndex | - | 50 | N/A | N/A | Associated with SSB index 0 |
| csirs-ResourceList | N/A | present | present | present | Associated with CSI-RS configured |
| ra-PreambleIndex | N/A | 50 | 50 | 50 | Associated with CSI-RS configured |
| ra-OccasionList | - | - | 1 | 1 | RA occasions allowed corresponding to CSI-RS |
| ra-ssb-OccasionMaskIndex | - | 1 | N/A | N/A | PRACH occasion index 1 is allowed |
| Note: For further information see Clause 6.3.2 in TS 38.331 [13]. |  |  |  |  |  |

# A.8	Reference BWP configurations

This section provides the typical BWP configurations used for RRM test cases defined in this test specification. For downlink BWP, both initial BWP and dedicated BWP configurations are specified in section A.8.1 and for uplink BWP, dedicated BWP configurations are specified in section A.8.2. Parameters not listed in this section can be derived from the configuration of each test.

## A.8.1	Downlink BWP configurations

Table A.8.1-1 defines the different downlink initial BWP configurations. Table A.8.1-2 defines the different downlink dedicated BWP configurations.

Table A.8.1-1: Downlink BWP patterns for initial BWP configuration

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.0.1 | DLBWP.0.2 |  |
| Starting PRB index |  | 0 | RBc Note 1 |  |
| Bandwidth |  | Same as RF channel defined in each test | same as RMSI CORSET(CORESET #0) defined in each test |  |
| Note 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Section A.1.2. |  |  |  |  |

Table A.8.1-2: Downlink BWP patterns for dedicated BWP configuration

| BWP Parameters | Unit | Values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL BWP |  | DLBWP.1.1 | DLBWP.1.2 |  |  | DLBWP.1.3 |  |  |
| Starting PRB index |  | 0 | RBb Note 1 |  |  | RBa Note 2 |  |  |
| SCS | kHz |  | 15 | 30 | 120 | 15 | 30 | 120 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 25 | 51 | 32 |
| Note 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Section A.3. Note 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Section A.3. |  |  |  |  |  |  |  |  |

## A.8.2	Uplink BWP configurations

Table A.8.2-1 defines the uplink initial BWP configurations. Table A.8.2-2 defines the uplink dedicated BWP configurations.

Table A.8.2-1: Uplink BWP patterns for initial BWP configurations

| BWP Parameters | Values |  |
| --- | --- | --- |
| UL BWP | ULBWP.0.1 | ULBWP.0.2 |
| Staring PRB index | 0 | RBc Note 1 |
| Bandwidth | Same as RF channel defined in each test | same as RMSI CORESET(CORSET #0) defined in each test |
| Note 1: RBc is the lowest PRB index to guarantee the BWP including CORESET #0 which is defined in Section A.1.2. |  |  |

Table A.8.2-2: Uplink BWP patterns for dedicated BWP configurations

| BWP Parameters | Unit | Values |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL BWP |  | ULBWP.1.1 | ULBWP.1.2 |  |  | ULBWP.1.3 |  |  |
| Staring PRB index |  | 0 | RBb Note 1 |  |  | RBa Note 1 |  |  |
| SCS | kHz |  | 15 | 30 | 120 | 15 | 30 | 120 |
| Bandwidth | RB | Same as RF channel defined for the serving cell in each test | 25 | 51 | 32 | 25 | 51 | 32 |
| Note 1: RBb is the lowest PRB index to guarantee the BWP not fully overlapped with SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Section A.3. Note 2: RBa is the lowest PRB index to guarantee the BWP including SSB PRB index (RBJ, RBJ+1,.…, RBJ+19) which is defined in Section A.3. |  |  |  |  |  |  |  |  |

# A.9	Angle of Arrival (AoA) for FR2 RRM test cases

This clause specifies the AoA setups for FR2 RRM test cases in section 5 and 7. The applicable AoA setup is defined in each test case in section 5 and 7.

## A.9.1	Setup 1: Single AoA in Rx beam peak direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, are aligned to the UE Rx beam peak direction (as defined in TS 38.101-2 [3]).

## A.9.2	Setup 2: Single AoA in non Rx beam peak direction

### A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The direction (AoA) of the signals shall not be changed between test iterations.

### A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction

There is only one active probe in the test. The DL signals, and noise if applicable, transmitted from the probe, align to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. For UE power class 3, the direction (AoA) of the signals shall be changed for each test iteration (for UE power classes other than 3, this is FFS). Unless otherwise specified by the test case, the TE shall alternate between a minimum of 33 different AoAs. The minimum separation between AoAs used in consecutive iterations shall be 30° in great-circle distance.

Note: if it is not possible to find at least 33 different test points meeting the required criteria (FFS), the test shall alternate between all the available test points.

## A.9.3	Setup 3: 2 AoAs

There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The relative angular offset between the directions (AoAs) of the 2 active probes, shall be changed for each test iteration. Unless otherwise specified by the test case, the TE shall alternate between a minimum of 33 different test points. A test point comprehends the relative angular offset between the active probes, and the AoA of each of the signals from the UE point of view. The relative angular offset between the active probes must change for consecutive test points.

The applicable set of relative angular offsets between the 2 active probes is given in Table A.9.3-1 for each UE power class.

Table A.9.3-1: Set of relative angular offsets between active probes for each power class

| UE Power class | Relative angular offset between active probes |
| --- | --- |
| 1 | FFS |
| 2 | FFS |
| 3 | 30°, 60°, 90°, 120° and 150° |
| 4 | FFS |

Note: if it is not possible to find at least 33 different test points meeting the required criteria (that is, the two AoAs are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and the angular offset between them corresponds to one of the offsets in Table A.9.3-1 for the corresponding UE power class), the test shall alternate between all the available test points.

## A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak

### A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [3]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The direction (AoA) of the non Rx beam peak signal shall not be changed between test iterations.

### A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction

There are 2 active probes in the test. The DL signals, and noise if applicable, are transmitted from the two active probes. One probe is aligned to the UE Rx beam peak direction as defined in TS 38.101-2 [3]. The second is aligned to a direction (AoA) which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class.

For UE power class 3, the relative angular offset between the directions (AoAs) of the 2 active probes shall be changed for each test iteration, within the probe alignment described above. The applicable set of relative angular offsets between the 2 active probes is given in Table A.9.3-1 for each UE power class. Unless otherwise specified by the test case, the TE shall alternate between a minimum of 5 different test points. A test point comprehends the relative angular offset between the active probes, and the AoA of each of the signals from the UE point of view, one of them being the Rx beam peak direction. The relative angular offset between the active probes must change for consecutive test points.

Note: if it is not possible to find at least 5 different test points meeting the required criteria (that is, the two AoAs are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] and the angular offset between them corresponds to one of the offsets in Table A.9.3-1 for the corresponding UE power class), the test shall alternate between all the available test points.

# A.10	TCI State Configuration

## A.10.1	Introduction

This section provides the configurations for TCI states towards either SSB or CSI-RS. The TCI states defined in this section are configured in each test when applicable to indicate that certain DL signals are quasi-collocated with the referenceSignal configured in the TCI states.

## A.10.2	TCI states

Table A.10.2-1: TCI States

| Parameter | TCI.State.0 | TCI.State.1 | TCI.State.2 | TCI.State.3 |
| --- | --- | --- | --- | --- |
| tci-StateId | Id0 | Id1 | Id2 | Id3 |
| qcl-Type1 | typeC | typeC | typeA | typeA |
| qcl-Type2Note1 | typeD | typeD | typeD | typeD |
| referenceSignal | SSB0 | SSB1 | Resource #4 in TRS resource set 1 Note3 | Resource #4 in TRS resource set 2 Note3 |
| Note 1: qcl-Type2 of typeD only where applicable. For RRM test cases, this will be only in FR2 Note 2: referenceSignal configurations towards which the TCI states are configured are defined in a test-specific manner. Note 3: Reference TRS resource sets are defined in A.3.17, and the applicable TRS resource set(s) are specified in each test case. When a single TRS resource set is configured in a test case, it is considered as resource set 1. |  |  |  |  |

Annex B (normative):
Conditions for RRM requirements applicability for operating bands

# B.1	Conditions for NR RRC_IDLE state mobility

## B.1.1	Introduction

In Annex B.1, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency idle state mobility test cases in clauses 6.1 and 7.1,

-	UE conditions which shall apply for UE inter-frequency idle state mobility test cases in clauses 6.1 and 7.1.

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

Editor’s notes for Table B.1.2-2 (RAN4 dependant):

- The value of Y for Power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for Power classes 1 and 4 respectively

- The value of Z for Power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for Power classes 1 and 4 respectively

This section defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3A.4. |  |  |  |  |

Table B.1.2-2: Conditions for intra-frequency cell re-selection in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105.5 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection

This section defines the following conditions for NR inter-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions defined in Table B.1.2-1 for FR1 NR intra-frequency cell re-selection shall also apply for FR1 NR inter-frequency cells in this section.

The conditions defined in Table B.1.2-2 for FR2 NR intra-frequency cell re-selection shall also apply for FR2 NR inter-frequency cells in this section.

# B.2	Conditions for NR RRC_CONNECTED state

## B.2.1	Introduction

In Annex B.2, the following conditions are specified:

-	UE conditions which shall apply for UE intra-frequency measurement procedures and measurement performance tests in clauses 4.6, 4.7, 5.6, 5.7, 6.6, 6.7, 7.6 and 7.7,

UE conditions which shall apply for UE inter-frequency measurements procedures and requirements in Section 9,

-	UE conditions which shall apply for UE intra-frequency measurements performance requirements in Section 10,

-	UE conditions which shall apply for UE inter-frequency measurements performance requirements in Section 10.

## B.2.2	Conditions for NR intra-frequency measurements

Editor’s notes for Table B.2.2-2 (RAN4 dependant):

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This section defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3A.4. |  |  |  |  |

Table B.2.2-2: Conditions for intra-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-6 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| Note 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. Note 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. Note 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.3	Conditions for NR inter-frequency measurements

Editor’s notes for Table B.2.3-2 (RAN4 dependant):

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1, and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This section defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3A.4. |  |  |  |  |

Table B.2.3-2: Conditions for inter-frequency measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n259 |  |  | -106.5 |  |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.4	Conditions for NR L1-RSRP reporting

### B.2.4.1	Conditions for SSB based L1-RSRP reporting

Editor’s notes for Table B.2.4.1-2 (RAN4 dependant):

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.1-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.1-2 for FR2 NR cells.

Table B.2.4.1-1: Conditions for SSB based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3A.4 |  |  |  |  |

Table B.2.4.1-2: Conditions for SSB based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.3 |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

### B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting

Editor’s notes for Table B.2.4.2-2 (RAN4 dependant):

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This clause defines the following conditions for NR L1-RSRP measurement reporting and corresponding procedures performed based on CSI-RS: CSI-RS_RP and CSI-RS Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.4.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.4.2-2 for FR2 NR cells.

Table B.2.4.2-1: Conditions for CSI-RS based L1-RSRP measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum CSI-RS_RP |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS |  |  | dB |
|  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -118 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 | -117.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 | -117 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 | -116.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 | -116 |  |
|  | NR_FDD_FR1_G | -121 | -118 | -115 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 | -114.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |  |

Table B.2.4.2-2: Conditions for CSI-RS based L1-RSRP measurements in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2, Note 3 |  |  |  |  | CSI-RS Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSCSI-RS |  |  |  |  | dB |
|  |  |  | SCSCSI-RS = 60 kHz |  |  |  | SCSCSI-RS = 120 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  |  | n259 |  |  | -108.5 |  |  |  |
|  |  | n260 | -125.3+Y1 |  | -109.5 | -125.8+Y4 |  |  |
|  |  | n261 | -128.3+Y1 | -113.8 | -112.1 | -127.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 | (Value for SCSCSI-RS = 60 kHz) +3dB | ≥-3 |
|  |  | n258 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
|  |  | n259 |  |  | -95.7 |  |  |  |
|  |  | n260 | -117.3+Z1 |  | -96.9 | -113.8+Z4 |  |  |
|  |  | n261 | -120.3+Z1 | -102.8 | -101.2 | -118.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum CSI-RS Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.5	Conditions for RRC connection release with redirection to NR

Editor’s notes for Table B.2.5.2-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine

This clause defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Table B.2.5-2: Conditions for RRC connection release with redirection to NR in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  |  | n259 |  |  | -106.5 |  |  |  |
|  |  | n260 | -123.3+Y1 |  | -107.5 | -123.8+Y4 |  |  |
|  |  | n261 | -126.3+Y1 | -111.8 | -110.1 | -125.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-4 |
|  |  | n258 | -118.3+Z1 | -100.8 | -99.2 | -116.8+Z4 |  |  |
|  |  | n259 |  |  | -93.7 |  |  |  |
|  |  | n260 | -115.3+Z1 |  | -94.9 | -111.8+Z4 |  |  |
|  |  | n261 | -114.3 | -100.8 | -99.2 | -116.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

## B.2.6	Conditions for UE transmit timing

### B.2.6.1	Conditions for SSB based UE transmit timing

Editor’s notes for Table B.2.6.1-2:

- The value of Y for power classes 1 and 4 is FFS, where Y1 and Y4 are the rough/fine beam gain differences in Rx beam peak direction for power classes 1 and 4 respectively

- The value of Z for power classes 1 and 4 is FFS, where Z1 and Z4 are the rough/fine beam gain differences in spherical coverage directions for power classes 1 and 4 respectively

This clause defines the following conditions for UE transmit timing adjustment performed based on SSBs: SSB_RP and SSB Ês/Iot and applicable for a corresponding operating band.

The conditions are defined in Table B.2.6.1-1 for FR1 SSB.

Table B.2.6.1-1: Conditions for SSB based UE transmit timing in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB=15 kHz | SCSSSB=30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -121 | -3 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

The conditions are defined in Table B.2.6.1-2 for FR2 SSB.

Table B.2.6.1-2: Conditions for SSB based UE transmit timing in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE power class |  |  |  | UE power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 1, 2, 3, 4 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  |  | n259 |  |  | -105.5 |  |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
|  |  | n259 |  |  | -92.7 |  |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOET 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |

# B.3	RRM requirement exceptions

## B.3.1	Introduction

Annex B.3 covers exceptions for the side conditions based on receiver sensitivity for CA, DC, and SUL.

## B.3.2	Receiver sensitivity relaxation for CA

### B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1

For a UE supporting inter-band carrier aggregation configuration with uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1 [18, Section 7.3A.3], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting CA configuration in FR1, the requirement in this section applies for both SC and CA operation.

### B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1

#### B.3.2.2.1	Inter-band carrier aggregation

For a UE configured with inter-band carrier aggregation with active uplink in NR band, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1, Section 7.3A.3 [2], the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

If the relaxation Δ specified in this section applies, then the relaxation specified in Section B.3.2.1 should not be applied.

#### B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA

In this section, requirements exceptions are described for the UE configured with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same CA configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Section 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Section 7.3A.4, when the following conditions are fulfilled,

-	corresponding downlink component carriers on different NR bands are configured with CA and active,

-	the uplink is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in TS 38.101-1 [2], Section 7.3A.4, and

-	the exception requirements specified in TS 38.101-1 [2], Section 7.3A.4 apply.

If the relaxation Δ specified in this section applies, then the relaxation specified in Section B.3.2.1 should not be applied.

#### B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

In this section, requirements exceptions are described for the UE with an inter-band carrier aggregation with uplink assigned to two NR bands.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Section 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Section 7.3A.5, when the following conditions are fulfilled,

-	corresponding downlink component carriers on different bands are configured with CA and active,

-	uplinks are assigned to two NR bands,

-	the exception requirements specified in TS 38.101-1 [2], Section 7.3A.5 apply.

If the relaxation Δ specified in this section applies, then the relaxation specified in Section B.3.2.1 should not be applied.

### B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2

Editor’s note: TBD

### B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2

#### B.3.2.4.1	Intra-band contiguous carrier aggregation

For a UE configured with intra-band contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in TS 38.101-2 [3], Section 7.3A.2.1 depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

#### B.3.2.4.2	Intra-band non-contiguous carrier aggregation

For a UE configured with intra-band non-contiguous carrier aggregation in NR band in FR2, if there is a relaxation of receiver sensitivity ΔRIB>0 dB as defined in TS 38.101-2 [3], Section 7.3A.2.1 depending on the aggregated channel bandwidth, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB defined for the corresponding downlink NR bands.

## B.3.3	Receiver sensitivity relaxation for DC

Editor’s note: TBD

## B.3.4	Receiver sensitivity relaxation for SUL

### B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1

For a UE supporting a SUL configuration in FR1, if there is a relaxation of receiver sensitivity ΔRIB,c>0 dB as defined in TS 38.101-1 [2], Section 7.3C.3, the relevant side conditions specifying received power levels (SSB_RP and Io) shall be increased by the amount Δ=ΔRIB,c defined for the corresponding downlink NR bands.

For a UE supporting a SUL configuration in FR1, the requirement in this section applies for both SC and SUL operation.

### B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1

#### B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL

In this section, requirements exceptions are described for the UE with a band in FR1 when it is impacted by UL harmonic interference from another band in FR1 of the same SUL configuration.

A relevant side condition (SSB_RP and Io) in a requirement shall be increased by the amount Δ=L2-L1, where L1 is the reference sensitivity level specified in TS 38.101-1 [2], Section 7.3.2, and L2 is the reference sensitivity level based on the requirements in TS 38.101-1 [2], Section 7.3C.2, when the following conditions are fulfilled,

-	a downlink component carrier is configured in NR band and is active,

-	the uplink is configured in the NR low operating band and is active,

-	the uplink configuration is as specified in TS 38.101-1 [2], Section 7.3C.2, and

-	the exception requirements specified in TS 38.101-1 [2], Section 7.3C.2 apply.

If the relaxation Δ specified in this section applies, then the relaxation specified in Section B.3.4.1 should not be applied.

Annex C (normative):
Downlink physical channels and propagation conditions

# C.1	Downlink physical channels

The following clauses describe the downlink physical channels that are needed for setting a connection and channels that are needed during a connection.

## C.1.1	General

TBD

## C.1.2	Default downlink signal levels

The downlink power settings in Table C.1.2-1 and Table C.1.2-2 are used unless otherwise specified in a test case for FR1 cell and FR2 cell, respectively. The downlink power settings in Table C.1.2-1 and Table C.1.2-2 are also used for the initial registration for NR SA test cases in clauses 6 and 7. For EN-DC test cases in clauses 4 and 5, the E-UTRA power settings used for initial registration are defined in Annex A.6.

For FR1, if the UE has more than one Rx antenna, the downlink signal is applied to each one. All UE Rx antennas shall be connected.

Table C.1.2-1: Default Downlink power levels for NR (FR1)

| SCS (kHz) | Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| 15 | Number of RBs |  | 25 | 50 | 75 | 100 | 128 | 160 | 215 | 270 | N/A | N/A | N/A | N/A |
|  | Channel BW power | dBm | -60 | -57 | -55 | -54 | -53 | -52 | -51 | -50 | N/A | N/A | N/A | N/A |
| 30 | Number of RBs |  | 10 | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 | 216 | 243 | 270 |
|  | Channel BW power | dBm | -61 | -57 | -55 | -54 | -53 | -52 | -51 | -50 | -49 | -48 | -47 | -47 |
| 60 | Number of RBs |  | N/A | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 | 100 | 120 | 135 |
|  | Channel BW power | dBm | N/A | -58 | -56 | -54 | -53 | -52 | -51 | -50 | -49 | -48 | -47 | -47 |
|  | RS EPRE | dBm/ 15kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 |
| Note 1: The channel bandwidth powers are informative, based on -85dBm/15kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at each UE Rx antenna. Note 3: DL level is applied for any of the Subcarrier Spacing configuration ( ) with the same power spectrum density of -85dBm/15kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table C.1.2-2: Default Downlink power levels for NR (FR2)

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | -70 | -67 | -64 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | -70 | -67 | -64 | -61 |
|  | SS/PBCH SSS EPRE | dBm/60kHz | -99 | -99 | -99 | -99 |
| Note 1: The channel bandwidth powers are informative, based on -99dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of –99dBm/60kHz. |  |  |  |  |  |  |

The default signal level uncertainty for FR1 is +/-3dB at each test port, for any level specified. The default signal level uncertainty for FR2 is +/-6dB at center of quiet zone, for any level specified. If the uncertainty value is critical for the Test purpose, a tighter uncertainty is specified for the related test case in Annex F

## C.1.3	Default connection setup

Table C.1.3-1 describes the downlink physical channels that are required for NR connection setup. For EN-DC test cases in clauses 4 and 5, the required E-UTRA downlink physical channels are defined in TS 36.521-3 [26] Annex C.2.

Table C.1.3-1: Downlink physical channels required for NR connection set-up

| Physical Channel | EPRE Ratio | Note |
| --- | --- | --- |
| PBCH | PBCH_RA = 0 dB |  |
|  | PBCH_RB = 0 dB |  |
| PSS | PSS_RA = 0 dB |  |
| SSS | SSS_RA = 0 dB |  |
| PDCCH | PDCCH_RA = 0 dB |  |
|  | PDCCH_RB = 0 dB |  |
|  | MPDCCH_RB = 0 dB |  |
| PDSCH | PDSCH_RA = 0 dB |  |
|  | PDSCH_RB = 0 dB |  |
| DMRS | TBD |  |
| CSI-RS | TBD |  |
| Note 1: No boosting is applied |  |  |

As common PDSCH and PDCCH configuration parameters the parameters in Table A.1 and C.1.3-2 shall be used to bring up the connection setup for FR1 and FR2 NR cell.

Table C.1.3-2: PDSCH and PDCCH configuration

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes |  | 8 (TDD) 4 (FDD) |
| Maximum number of HARQ transmission |  | 5 |

# C.2	Propagation conditions

The propagation conditions and channel models for various environments are specified. For each environment a propagation model is used to evaluate the propagation pathless due to the distance. Channel models are formed by combining delay profiles with a Doppler spectrum, with the addition of correlation properties in the case of a multi-antenna scenario.

## C.2.1	No interference

The downlink connection between the SS and the UE is without AWGN, and has no fading or multipath effects.

## C.2.2	Static propagation conditions

The downlink connection between the SS and the UE is an AWGN environment (unless otherwise stated) with no fading or multipath effects.

### C.2.2.1	UE receiver with 2Rx antenna connectors

For 1 port transmission to UE receiver with 2Rx the channel matrix is defined in the frequency domain by

.

For 2 port transmission to UE Receiver with 2Rx the channel matrix is defined in the frequency domain by

For 4 port transmission to UE Receiver with 2Rx the channel matrix is defined in the frequency domain by

### C.2.2.2	UE receiver with 4Rx antenna connectors

For 1 port transmission to UE receiver with 4Rx the channel matrix is defined in the frequency domain by

.

For 2 port transmission to UE Receiver with 4Rx the channel matrix is defined in the frequency domain by

.

For 4 port transmission to UE Receiver with 4Rx the channel matrix is defined in the frequency domain by

.

## C.2.3	Multi-path fading propagation conditions

TBD

Annex D (normative):
Deviations from standard test configuration

This annex summarizes the list of test cases which deviate from the standard test configuration.

# D.1	Test cases with different numerologies

TBD

# D.2	EN-DC test cases with different EN-DC configurations

In clauses 4 and 5, EN-DC test cases may be defined for two component carriers (CCs) as well as for more than two CCs to verify the same RRM requirement.

## D.2.1	Principle of testing

If multiple EN-DC test cases are defined for two CCs as well as for more than two CCs to verify the same type of RRM requirement, and this requirement is dependent on the number of CCs, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the EN-DC test cases with the maximum number of CCs in EN-DC supported by the UE. Otherwise if the same type of RRM requirement is independent of the number of CCs then from the UE performance point of view the test coverage can be considered fulfilled by executing only the EN-DC test cases with two CCs in EN-DC supported by the UE.

Editor’s Note: The maximum number of CCs that can be used in FR2 tests in EN-DC would depend on the test equipment capability.

# D.3	Carrier aggregation test cases with different CA configurations

In clauses 6 and 7, carrier aggregation test cases may be defined for two CCs as well as for more than two CCs to verify the same RRM requirement.

## D.3.1	Principle of testing

If multiple carrier aggregation test cases are defined for two CCs as well as for more than two CCs to verify the same RRM requirement, and the test requirement is dependent on the number of CCs, then from the UE performance point of view the test coverage can be considered fulfilled by executing only the CA test cases with the maximum number of CCs in CA supported by the UE. Otherwise if the same type of RRM requirement is independent of the number of CCs then from the UE performance point of view the test coverage can be considered fulfilled by executing only the CA test cases with at least two CCs in CA supported by the UE.

Editor’s Note: The maximum number of CCs that can be used in FR2 tests in CA would depend on the test equipment capability.

# D.4	Antenna connection for 4Rx capable UEs

All the tests in this test specification are defined for UEs supporting 2Rx. This section explains how to apply the 2Rx tests in clauses 4 and 6 to UEs supporting 4Rx antenna ports. No tests are currently specified in clauses 4 or 6 which are applicable only to 4Rx antenna ports, so 4Rx capable UEs are always tested by reusing tests which were originally specified for 2Rx UEs. Please notice that 4Rx is in general not supported for the test cases in clauses 5 and 7.

## D.4.1	Principle of testing

### D.4.1.1	Single carrier tests

For 4Rx capable UEs supporting at least one band where 2Rx is supported and 4Rx is not supported, all single carrier tests specified in clauses 4 and 6, except 4.7 and 6.7 shall be tested with 2Rx on any band where 2Rx is supported and 4Rx is not supported, with the antenna connection defined in D.4.2.1. Single carrier tests specified clauses 4.7 and 6.7 are band dependent and shall be tested in all bands supported by the UE, using 2Rx and the antenna connection defined in D.4.2.1 for the bands where 2Rx is supported and 4Rx is not supported, and the antenna connection defined in D.4.2.2 for the bands where 2Rx is not supported.

For 4Rx capable UEs that do not support any band where 2Rx is supported and 4Rx is not supported, all single carrier tests in clauses 4 and 6 shall be tested with 4Rx using the antenna configuration defined in D.4.2.2.For radio link monitoring tests, the SNR levels are modified according to table D.4.1.1-1 and table D.4.1.1-2

Table D.4.1.1-1: Modified parameters for RLM out of sync testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 | Test 4 |
| 4.5.1.1 | -18 | N/A | N/A | N/A |
| 4.5.1.3 | -18 | N/A | N/A | N/A |
| 4.5.1.5 | -18 | N/A | N/A | N/A |
| 4.5.1.7 | -18 | N/A | N/A | N/A |
| 5.5.1.1 | -18 | N/A | N/A | N/A |
| 5.5.1.3 | -18 | N/A | N/A | N/A |
| 5.5.1.5 | -18 | N/A | N/A | N/A |
| 5.5.1.7 | -18 | N/A | N/A | N/A |
| 6.5.1.1 | -18 | N/A | N/A | N/A |
| 6.5.1.3 | -18 | N/A | N/A | N/A |
| 6.5.1.5 | -18 | N/A | N/A | N/A |
| 6.5.1.7 | -18 | N/A | N/A | N/A |
| 7.5.1.1 | -18 | N/A | N/A | N/A |
| 7.5.1.3 | -18 | N/A | N/A | N/A |
| 7.5.1.5 | -18 | N/A | N/A | N/A |
| 7.5.1.7 | -18 | N/A | N/A | N/A |

Table D.4.1.1-2: Modified parameters for RLM in sync single carrier testing with 4 RX antenna connection

| Test case | SNR during T3 (dB) |  | SNR during T4 (dB) |  |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 1 | Test 2 |
| 4.5.1.2 | -18 | N/A | -8 | N/A |
| 4.5.1.4 | -18 | N/A | -8 | N/A |
| 4.5.1.6 | -18 | N/A | -8 | N/A |
| 4.5.1.8 | -18 | N/A | -8 | N/A |
| 5.5.1.2 | -18 | N/A | -8 | N/A |
| 5.5.1.4 | -18 | N/A | -8 | N/A |
| 5.5.1.6 | -18 | N/A | -8 | N/A |
| 5.5.1.8 | -18 | N/A | -8 | N/A |
| 6.5.1.2 | -18 | N/A | -8 | N/A |
| 6.5.1.4 | -18 | N/A | -8 | N/A |
| 6.5.1.6 | -18 | N/A | -8 | N/A |
| 6.5.1.8 | -18 | N/A | -8 | N/A |
| 7.5.1.2 | -18 | N/A | -8 | N/A |
| 7.5.1.4 | -18 | N/A | -8 | N/A |
| 7.5.1.6 | -18 | N/A | -8 | N/A |
| 7.5.1.8 | -18 | N/A | -8 | N/A |

Table D.4.1.1-3: Modified parameters for Beam Failure Detection and Link Recovery testing with 4 RX antenna connection

| Test case | SNR for RS in set q0 during T3, T4 and T5.(dB) |
| --- | --- |
|  | Test 1 |
| 4.5.5.1 | -15 |
| 4.5.5.2 | -15 |
| 4.5.5.3 | -15 |
| 4.5.5.4 | -15 |
| 5.5.5.1 | -15 |
| 5.5.5.2 | -15 |
| 5.5.5.3 | -15 |
| 5.5.5.4 | -15 |
| 6.5.5.1 | -15 |
| 6.5.5.2 | -15 |
| 6.5.5.3 | -15 |
| 6.5.5.4 | -15 |
| 7.5.5.1 | -15 |
| 7.5.5.2 | -15 |
| 7.5.5.3 | -15 |
| 7.5.5.4 | -15 |

### D.4.1.2	Carrier aggregation tests

For carrier aggregation tests, the antenna connection is selected independently for each cell, the PCell and the SCell(s). If a cell (either PCell or any of the SCell(s)) is on a band where 2Rx is supported and 4Rx is not supported, antenna connection in Section D.4.2.1 shall be used for this cell. If the cell is on a band where 2Rx is not supported, antenna connection in section D.4.2.2 shall be used for this cell.

### D.4.1.3	EN-DC tests

For all EN-DC tests, the antenna connection is selected independently for each cell. For the E-UTRA PCell, the antenna connection specified in D.4.2.3 shall be used if the PCell is on an E-UTRA band where 2Rx is supported and 4Rx is not supported, and the antenna connection specified in D.4.2.4 shall be used if the PCell is on an E-UTRA band not supporting 2Rx.

For the NR PSCell and SCell(s), the principle of testing is the same as in D.4.1.2.

## D.4.2	Antenna connection

### D.4.2.1	Antenna connection for NR bands where 2Rx is supported

For NR bands where 2Rx is supported and 4Rx is not supported, the UE shall decide via manufacturer declaration and antenna port configuration which 2 of the 4 antenna ports shall be connected with the downlink signal from the SS. The remaining 2 antenna ports shall be connected to zero input. The parameters and test requirements remain unmodified.

### D.4.2.2	Antenna connection for NR bands where only 4Rx is supported

For NR bands where only 4Rx is supported, all 4Rx antenna ports shall be connected to the downlink signal from the SS. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring tests specified in D.4.1.1 and D.4.1.2, the parameters and test requirements remain unmodified.

### D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported

For E-UTRAN bands where 2Rx is supported and 4Rx is not supported, the UE shall decide via manufacturer declaration and antenna port configuration which 2 of the 4 antenna ports shall be connected with the downlink signal from the SS. The remaining 2Rx antenna ports shall be connected to zero input. The parameters and test requirements remain unmodified.

### D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported

For E-UTRAN bands where only 4Rx is supported, all 4Rx antenna ports shall be connected to the downlink signal from the SS. The system simulator shall provide independent noise and fading (low correlation) for each antenna port. Except for the modifications to radio link monitoring tests specified in TS36.521-1 [26] clause 3A.4.1.2.1 and 3A.4.1.2.2, the parameters and test requirements remain unmodified.

# D.5	Test Cases with Different Channel Bandwidths

## D.5.1	Test Cases with Different E-UTRA Channel Bandwidths

### D.5.1.1	Introduction

Test cases involving E-UTRA cell(s) may be defined with different E-UTRA channel bandwidths to verify the same type of RRM requirement.

### D.5.1.2	Principle of testing

If multiple test cases involving E-UTRA cell(s) are defined with different E-UTRA channel bandwidths to verify the same type of RRM requirement that is E-UTRA channel bandwidth independent, then the UE needs to be tested with only one channel bandwidth in each E-UTRA cell and with the same bandwidth in all the E-UTRA cells used in the test case.

# D.6	Test Cases for Synchronous and Asynchronous DC Operations

## D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations

### D.6.1.1	Introduction

This clause defines a principle, which is applicable to test cases verifying RRM requirements for EN-DC operation in synchronous and asynchronous scenarios.

Test cases may be defined in both synchronous EN-DC and asynchronous EN-DC scenarios to verify the same type of RRM requirement.

### D.6.1.2	Principle of Testing

If EN-DC test cases are defined in both synchronous and asynchronous EN-DC scenarios to verify the same type of RRM requirement then the UE capable of both synchronous and asynchronous EN-DC operations needs to be tested with one of the tests in either synchronous or asynchronous EN-DC scenarios.

Annex E (normative):
Cell configuration mapping

The cells used in TS 38.533 do not correspond to the cells defined in TS 38.508-1 [14] section 4.4.2. This annex describes the mapping between the test cases in TS 38.533 and the cells defined in TS 38.508-1 [14]. The test case shall apply the RF parameters as defined in TS 38.533 according to the column heading.

NOTE:	For example if the second cell in a test case is an inter-frequency cell then Cell3 from TS 38.508-1 [14] section 4.4.2 is used with the radio parameters as defined for Cell2 in TS 38.533.

# E.1	Test frequency selection

The requirements defined in this test specification comprise EN-DC and NR SA test cases. The test cases are defined with a single NR cell or with multiple NR cells. The multi-cell test cases can be either intra-frequency, i.e. the NR cells defined by the test are overlapping in the frequency domain, or inter-frequency, i.e. the NR cells defined by the test have different centre frequencies, separated from each other by a frequency value bigger than the respective cell bandwidths. This clause describes the general rule on how to select the test frequencies for the NR RRM test cases in this test specification.

## E.1.1	E-UTRA PCell for EN-DC test cases

Unless otherwise stated, the E-UTRA PCell for EN-DC test cases shall be configured using the test frequency “Mid” as defined in TS 36.508 [25] for the corresponding E-UTRA band.

In case that the “Mid” test frequency overlaps with any of the NR test frequencies required by the test case, the E-UTRA PCell shall be shifted to an additional frequency within the E-UTRA same band. If the E-UTRA band channel bandwidth is not sufficient to allocate a non-overlapping E-UTRA PCell, the auxiliary band as defined in TS 36.521-3 [26] clause 3 shall be used.

## E.1.2	Test cases with one NR cell

Unless otherwise stated, for NR test cases with one NR cell, this cell shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding band under test.

## E.1.3	Test cases with more than one NR cell

### E.1.3.1	Intra-frequency test cases

Unless otherwise stated, multi-cell intra-frequency test cases shall be tested using the test frequency “Mid” as defined in TS 38.508-1 [14] for the corresponding NR band under test.

### E.1.3.2	Inter-frequency test cases

For NR SA  and EN-DC multi-cell inter-frequency test cases in FR1, unless otherwise stated, the serving cell shall be configured using the test frequency “Low” as defined in TS 38.508-1 [14] for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using the test frequency “High” as defined in TS 38.508-1 [14] for the corresponding band under test.

For NR SA multi-cell inter-frequency test cases in FR2, unless otherwise stated, the serving cell (and any other neighbour cell in the same frequency carrier) shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14], clause 7.2.3.2 for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using a non-overlapping test frequency adjacent to the serving cell frequency, as defined in TS 38.508-1 [14], clause 7.2.3.2.

For EN-DC multi-cell inter-frequency test cases in FR2, unless otherwise stated, the PSCell (and any other neighbour cell in the same frequency carrier) shall be configured using the test frequency “Mid” as defined in TS 38.508-1 [14] , clause 7.2.3.2 for the corresponding band under test. Any inter-frequency neighbour cell shall be configured using a non-overlapping test frequency adjacent to the PSCell frequency, as defined in TS 38.508-1 [14] , clause 7.2.3.2.

## E.1.4	Carrier aggregation test cases

### E.1.4.1	Inter-band carrier aggregation

For inter-band carrier aggregation test cases, each of the component carriers and their respective neighbour cells shall be configured following the sample principles defined in E.1.2 and E.1.3.

### E.1.4.2	Intra-band contiguous carrier aggregation

For intra-band contiguous carrier aggregation, the test frequency selection shall be done following the same principle as in E.1.3.2 for inter-frequency test cases.

### E.1.4.3	Intra-band non-contiguous carrier aggregation

For intra-band non-contiguous carrier aggregation in FR1, unless otherwise specified, the test frequency selection shall be done following the maximum Wgap principle, i.e. selecting the test frequencies (of the test frequencies defined in TS 38.508-1 [14]) with the widest frequency separation within the band under test.

For intra-band non-contiguous carrier aggregation in FR2, the test frequency selection is TBD.

## E.1.5	E-UTRA – NR inter RAT test cases

Unless otherwise stated, the E-UTRA serving/neighbour cell for E-UTRA – NR inter-RAT test cases shall be configured using the test frequency “Mid” as defined in TS 36.508 [25] for the corresponding E-UTRA band.

In case that the “Mid” test frequency overlaps with any of the NR test frequencies required by the test case, the E-UTRA cell shall be shifted to an additional frequency within the E-UTRA same band. If the E-UTRA band channel bandwidth is not sufficient to allocate a non-overlapping E-UTRA cell, the auxiliary band as defined in TS 36.521-3 [26] clause 3 shall be used.

# E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4

Table E.2-1 defines the cell configuration mapping for EN-DC FR1 test cases in chapter 4 of this test specification.

Table E.2-1: Cell configuration mapping for EN-DC FR1 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 | EN-DC FR1 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.3.2.2.2 | EN-DC FR1 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.1.1 | EN-DC FR1 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.4.3.1 | EN-DC FR1 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.1 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.2 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.3 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.4 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.5 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.6 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.7 | EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.1.8 | EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.1 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.2 | EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.2.3 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.2.4 | EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.2.5 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.2.6 | EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 4.5.3.1 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.3.2 | EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.3.3 | EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.4.1 | EN-DC FR1 UE UL carrier RRC reconfiguration delay | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.5.1 | EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.2 | EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.3 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.5.4 | EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.1.1 | EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.6.1.2 | EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.5.6.2.1 | EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.5.7.1 | EN-DC FR1 addition and release delay of known PSCell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.1.1 | EN-DC FR1 event-triggered reporting without gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.2 | EN-DC FR1 event-triggered reporting without gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.3 | EN-DC FR1 event-triggered reporting with gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.4 | EN-DC FR1 event-triggered reporting with gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.5 | EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.1.6 | EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.6.2.1 | EN-DC FR1-FR1 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.2 | EN-DC FR1-FR1 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.5 | EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.2.6 | EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.6.4.1 | EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.2 | EN-DC FR1 SSB-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.3 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.6.4.4 | EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.1.1.1 | EN-DC FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.1.1.2 | EN-DC FR1 SS-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 4.7.1.2.1 | EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.1.2.2 | EN-DC FR1-FR1 SS-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.2.1 | EN-DC FR1 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.7.2.2.1 | EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.2.2.2 | EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.3.1 | EN-DC FR1 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 4.7.3.2.1 | EN-DC FR1-FR1 SS-SINR absolute measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.3.2.2 | EN-DC FR1-FR1 SS-SINR relative measurement accuracy | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 4.7.4.1.1 | EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.1.2 | EN-DC FR1 SSB based L1-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.2.1 | EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.4.2.2 | EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 4.7.5.1 | EN-DC FR1 SFTD measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |

# E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5

Table E.3-1 defines the cell configuration mapping for EN-DC FR2 test cases in chapter 5 of this test specification.

Table E.3-1: Cell configuration mapping for EN-DC FR2 RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 5.3.2.2.1 | EN-DC FR2 contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.3.2.2.2 | EN-DC FR2 non-contention based random access | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.4.1.1 | EN-DC FR2 UE transmit timing accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.4.3.1 | EN-DC FR2 timing advance adjustment accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.1 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.3 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 6 |  |  |  |
| 5.5.1.5 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.6 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.7 | EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.8 | EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.1.9 | EN-DC FR2 radio link monitoring UE scheduling restrictions | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.1 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.2 | EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.2.3 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.5.2.4 | EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.5.2.5 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.2.6 | EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | LTE Cell 6 | NR Cell 1 | LTE Cell 3 |  |  |
| 5.5.3.1 | EN-DC FR2 SCell activation and deactivation intra-band in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.5.5.1 | EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.2 | EN-DC FR2 SSB-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.3 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.4 | EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.5.5 | EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.8.1 | EN-DC FR2 MAC-CE based active TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.5.8.2 | EN-DC FR2 RRC based active TCI state switch | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.1.1 | EN-DC FR2 event-triggered reporting without gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.2 | EN-DC FR2 event-triggered reporting without gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.3 | EN-DC FR2 event-triggered reporting with gap in non-DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.1.4 | EN-DC FR2 event-triggered reporting with gap in DRX | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.6.2.1 | EN-DC FR2-FR2 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.2 | EN-DC FR2-FR2 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.3 | EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.4 | EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.5 | EN-DC FR1-FR2 event-triggered reporting in non-DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.6 | EN-DC FR1-FR2 event-triggered reporting in DRX | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.7 | EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.2.8 | EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection | LTE Cell 1 | NR Cell 6 | NR Cell 3 |  |  |
| 5.6.3.1 | EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.2 | EN-DC FR2 SSB-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.3 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.6.3.4 | EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 5.7.1.1 | EN-DC FR2 SS-RSRP measurement accuracy | LTE Cell 1 | NR Cell 489 | NR Cell 1 |  |  |
| 5.7.1.2 | EN-DC FR2-FR2 SS-RSRP measurement accuracy | TBD | TBD | TBD |  |  |
| 5.7.2.1 | EN-DC FR2 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |
| 5.7.3.1 | EN-DC FR2 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 | NR Cell 2 |  |  |

# E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for SA FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.1.1.2 | NR SA FR1-FR1 cell re-selection | NR Cell 6 | NR Cell 23 |  |  |  |  |
| 6.1.1.7 | NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 11 |  |  |  |  |
| 6.3.1.1 | NR SA FR1 handover with known target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.2 | NR SA FR1 handover with unknown target cell | NR Cell 1 | NR Cell 2 |  |  |  |
| 6.3.1.3 | NR SA FR1-FR1 Handover with unknown Target Cell | NR Cell 6 | NR Cell 3 |  |  |  |
| 6.3.2.1.1 | NR SA FR1 RRC re-establishment | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.2.1.2 | NR SA FR1 - FR1 RRC re-establishment | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.3.2.1.3 | NR SA FR1 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.3.2.2.1 | NR SA FR1 contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.2.2 | NR SA FR1 non-contention based random access | NR Cell 1 |  |  |  |  |  |
| 6.3.2.3.1 | NR SA FR1 RRC connection release with redirection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.4.3.1 | NR SA FR1 timing advance adjustment accuracy | NR Cell 1 |  |  |  |  |  |
| 6.5.1.1 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.2 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.3 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.4 | NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.5 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.6 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.7 | NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.1.8 | NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode | NR Cell 1 |  |  |  |  |  |
| 6.5.2.1 | NR SA FR1 interruptions during measurements on deactivated NR SCC | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.5.3.1 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.5.3.2 | NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.5.3.3 | NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.5.4.1 | NR SA FR1 UE UL carrier RRC reconfiguration delay | NR Cell 1 | NR Cell 33 |  |  |  |  |
| 6.5.5.1 | NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.2 | NR SA FR1 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.3 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.5.4 | NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.6.1.1 | NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.5.6.1.2 | NR SA FR1 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.5.6.2.1 | NR SA FR1 RRC-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.1.1 | SA event triggered reporting tests without gap under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.2 | SA event triggered reporting tests without gap under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.3 | SA event triggered reporting tests with per-UE gaps under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.4 | SA event triggered reporting tests with per-UE gaps under DRX | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.5 | SA event triggered reporting tests without gap under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.6 | SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.1.7 | NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.6.2.1 | NR SA FR1-FR1 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.2 | NR SA FR1-FR1 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.5 | NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.2.6 | NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.6.4.1 | NR SA FR1 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.2 | NR SA FR1 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.3 | NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.4 | NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |  |
| 6.6.4.5 | NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16 | NR Cell 1 |  |  |  |  |  |
| 6.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.1.3.2 | NR SA FR1-FR2 SS-RSRP relative measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.1.1.1 | NR SA FR1 SS-RSRP absolute measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |
| 6.7.1.1.2 | NR SA FR1 SS-RSRP relative measurement accuracy | NR Cell 489 | NR Cell 1 |  |  |  |  |
| 6.7.1.2.1 | NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.1.2.2 | NR SA FR1-FR1 SS-RSRP relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.2.1 | NR SA FR1 SS-RSRQ measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.2.2.1 | NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.2.2.2 | NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.3.1 | NR SA FR1 SS-SINR measurement accuracy | NR Cell 1 | NR Cell 2 |  |  |  |  |
| 6.7.3.2.1 | NR SA FR1-FR1 SS-SINR absolute measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.3.2.2 | NR SA FR1-FR1 SS-SINR relative measurement accuracy | NR Cell 6 | NR Cell 3 |  |  |  |  |
| 6.7.4.1.1 | NR SA FR1 SSB based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.1.2 | NR SA FR1 SSB based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.2.1 | NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | NR Cell 1 |  |  |  |  |  |
| 6.7.4.2.2 | NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | NR Cell 1 |  |  |  |  |  |

Table E.4-2 defines the cell configuration mapping for SA FR1 – E-UTRA Inter-RAT test cases (serving cell in NR) in chapter 6 of this test specification.

Table E.4-2: Cell configuration mapping for SA FR1 – E-UTRA Inter-RAT RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 LTE Cell2 | 38.533 LTE Cell3 | 38.533 LTE Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.2.1 | NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.1.2.2 | NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.1.4 | NR SA FR1 – E-UTRA handover with known target cell | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.1.5 | NR SA FR1 – E-UTRA handover with unknown target cell | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.3.2.3.2 | NR SA FR1 – E-UTRA RRC connection release with redirection | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.1 | NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.6.3.2 | NR SA FR1 – E-UTRAN event-triggered reporting in DRX | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.5.1 | NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.6.1 | NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |
| 6.7.7.1 | NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | NR Cell 1 | LTE Cell 1 |  |  |  |

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for SA FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 | NR SA FR2 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| 7.1.1.2 | NR SA FR2-FR2 cell re-selection | NR Cell 1 | NR Cell 23 |  |  |  |
| 7.3.2.1.1 | NR SA FR2 RRC re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.2 | NR SA FR2-FR2 re-establishment | NR Cell 1 |  |  |  |  |
| 7.3.2.1.3 | NR SA FR2 RRC re-establishment without serving cell timing | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.3.2.2.1 | NR SA FR2 contention based random access | NR Cell 1 |  |  |  |  |
| 7.3.2.2.2 | NR SA FR2 non-contention based random access | NR Cell 1 |  |  |  |  |
| 7.5.1.9 | NR SA FR2 radio link monitoring UE scheduling restrictions | NR Cell 1 |  |  |  |  |
| 7.5.5.1 | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.2 | NR SA FR2 SSB-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.3 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | NR Cell 1 |  |  |  |  |
| 7.5.5.4 | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | NR Cell 1 |  |  |  |  |
| 7.5.6.1 | NR SA FR2 DCI-based DL active BWP switch in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.1.1 | NR SA FR2 event-triggered reporting without gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.2 | NR SA FR2 event-triggered reporting without gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.3 | NR SA FR2 event-triggered reporting with gap in non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.1.4 | NR SA FR2 event-triggered reporting with gap in DRX | NR Cell 1 | NR Cell 2 |  |  |  |
| 7.6.2.1 | NR SA FR2-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.2 | NR SA FR2-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.3 | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.4 | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.5 | NR SA FR1-FR2 event-triggered reporting in non-DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.6 | NR SA FR1-FR2 event-triggered reporting in DRX | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.7 | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.2.8 | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection | NR Cell 6 | NR Cell 3 |  |  |  |
| 7.6.3.1 | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.2 | NR SA FR2 SSB-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.3 | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | NR Cell 1 |  |  |  |  |
| 7.6.3.4 | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | NR Cell 1 |  |  |  |  |
| 7.7.1.1 | NR SA FR2 SS-RSRP measurement accuracy | TBD | TBD | TBD |  |  |
| 7.7.1.2 | NR SA FR2-FR1 SS-RSRP measurement accuracy | TBD | TBD | TBD |  |  |
| 7.7.1.3.1 | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy | NR Cell 1 | NR Cell 10 |  |  |  |
| 7.7.1.3.2 | Void |  |  |  |  |  |

# E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8

Table E.6-1 defines the cell configuration mapping for E-UTRAN - SA FR1 Inter-RAT test cases (serving cell in LTE) in chapter 8 of this test specification.

Table E.6-1: Cell configuration mapping for E-UTRA – SA FR1 Inter-RAT RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 8.2.1.1 | E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.3.1.1 | E-UTRA – NR FR1 handover with unknown target cell | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.1.1 | E-UTRA – NR FR1 SFTD measurement delay in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.1.2 | E-UTRA – NR FR1 SFTD measurement delay in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.1 | E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.2 | E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.3 | E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.4 | E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.1.1 | E-UTRA – NR FR1 SFTD measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.1.1.1 | E-UTRA – NR FR1 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.2.1 | E-UTRA – NR FR1 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.3.1 | E-UTRA – NR FR1 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |

Table E.6-2 defines the cell configuration mapping for E-UTRAN - SA FR2 Inter-RAT test cases (serving cell in LTE) in chapter 8 of this test specification.

Table E.6-2: Cell configuration mapping for E-UTRA – SA FR2 Inter-RAT RRM testing

| TC | Description | 38.533 LTE Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 8.4.2.5 | E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.6 | E-UTRA – NR FR2 event-triggered reporting without SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.7 | E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in non-DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.4.2.8 | E-UTRA – NR FR2 event-triggered reporting with SSB time index detection in DRX | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.1.2 | E-UTRA – NR FR2 SS-RSRP absolute measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.2.2 | E-UTRA – NR FR2 SS-RSRQ measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |
| 8.5.2.3.2 | E-UTRA – NR FR2 SS-SINR measurement accuracy | LTE Cell 1 | NR Cell 1 |  |  |  |

Annex F (normative):
Measurement uncertainties and test tolerances

The requirements of this clause apply to all tests in the present document.

# F.1	Measurement uncertainties and test tolerances for FR1 and FR2

## F.1.1	Acceptable uncertainty of test system (normative)

See TS 38.521-1 [17] annex F.1.

### F.1.1.1	Measurement of test environments

See TS 38.521-1 [17] Annex F1.1.

### F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

Table F.1.1.2-1: Maximum allowed measurement uncertainty values for the test system for FR1 (up to 6 GHz) and Cell BW ≤ 40 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | ±1.5 |  |
| AWGN absolute power, Noc for Measurement PRB | dB | ±1.5 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 | Same as in LTE |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB105 for CSI-RS-based |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx | Depends on fading profile, can be referenced from TS 38.101-4 [20] |
| AWGN and signal flatness* | dB | ±2.0 | Same as in LTE, can be referenced from TS 38.101-4 [20] |
| Uplink absolute power measurement | dB | ±1.5 | Same as in TS 38.521-1 [17] |
| Uplink relative power measurement | dB | ±0.7 | Same as in TS 38.521-1 [17] |
| Uplink signal transmit timing relative to downlink | Tc | ±112 |  |
| Relative transmit timing accuracy during UE timing adjustment | Tc | ±88 |  |
| Timing Advance Adjustment accuracy | Tc | ±88 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combined these values in the TT analysis in TR 38.903 Note 2:These values apply for cell BW ≤ 40 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Void |  |  |  |

Table F.1.1.2-2: Maximum allowed measurement uncertainty values for the test system for FR2 (up to 40GHz) and Cell BW ≤ 400 MHz

| MU contributor | Unit | Value | Comment |
| --- | --- | --- | --- |
| AWGN absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| AWGN absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Ratio of cell X signal / AWGN, Êsx / Noc averaged over BWConfig | dB | ±0.3 |  |
| Ratio of cell X signal / AWGN, Êsx / Noc for Measurement PRB | dB | ±0.3 | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| ÊsxNote 3 absolute power, Noc averaged over BWConfig | dB | PC3: ±5.654,5 PC1: FFS |  |
| ÊsxNote 3 absolute power, Noc for Measurement PRB | dB | PC3: ±5.654,5 PC1: FFS | Measurement PRB may be for example #RBJ to RBJ+19 for SSB-based, or selected RBs in the range #RB0 to RB274 for CSI-RS-based |
| Relative power between any two Ês levels at the same frequency | dB | ±2.0 |  |
| Fading profile uncertainty | dB | ±0.5 for 1 Tx ±0.7 for 2 Tx |  |
| Uplink absolute power measurement | dB | TBD |  |
| Uplink relative power measurement | dB | TBD |  |
| Uplink signal transmit timing relative to downlink | Tc | ±48 |  |
| Relative transmit timing during UE timing adjustment | Tc | ±40 |  |
| Timing Advance Adjustment | Tc | ±40 |  |
| Note 1: The values in this table are specified per cell. Multi-cell test cases need to combine these values in the TT analysis in TR 38.903 Note 2: These values apply for cell BW ≤ 400 MHz. The maximum allowed measurement uncertainty for higher cell BW is FFS. Note 3: Applies for test cases that use Ês without AWGN Note 4: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in TR 38.903 [22] Table E.3.1.1-2 for PC3 UEs and in Table FFS for PC1 UEs. Note 5: If the TT analysis for a specific test case based on this MU value results in an unsolvable conflict, making the test case untestable, even after the alternative solutions listed in TR 38.903 [22] clause A.4 have been considered for the test case in TS 38.133 [6] Annex A, the TT analysis shall be repeated using a lower MU value, taking into account the values defined in clause E.3.1 TR 38.903 [22]. The test case will be applicable for the subset of the test systems meeting this reduced MU Threshold. |  |  |  |

The maximum test system uncertainty for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.1.2-3.

The maximum test system uncertainty for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.1.2-4.

The maximum test system uncertainty for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.1.2-5.

The maximum test system uncertainty for the NR SA FR2 test cases in chapter 7 is defined in Table F.1.1.2-6.

Table F.1.1.2-3: Maximum test system uncertainty for RRM requirements for EN-DC FR1 test cases

| Subclause |  |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 Contention based random access test in FR1 for PSCell in EN-DC |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC |  |  | Same as 4.3.2.2.1 |  | Same as 4.3.2.2.1 |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  | Same as 4.5.1.1 |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  |  | ± 0.6dB (AWGN conditions) |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  |  | Same as 4.5.2.1 |  | Same as 4.5.2.1 |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  |  | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc2 ±0.3 dB Ês3 / Noc3 ±0.3 dB |  | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  |  | Same as 4.5.2.3 |  | Same as 4.5.2.3 |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  |  | Noc1 ±1.5 dB Noc2 ±1.5 dB Noc3 ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  | Note: Noc1 is the AWGN on E-UTRAN cell 1 frequency Ês1 / Noc1 is the ratio of E-UTRAN cell 1 signal / AWGN Noc2 is the AWGN on NR cell 2 frequency Ês2 / Noc2 is the ratio of NR cell 2 signal / AWGN Noc3 is the AWGN on E-UTRAN cell 3 frequency Ês3 / Noc3 is the ratio of E-UTRAN cell 3 signal / AWGN |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  |  | Same as 4.5.2.5 |  | Same as 4.5.2.5 |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 4.5.3.1 |  | Same as 4.5.3.1 |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  |  |  | Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Noc3 is the AWGN on cell 3 frequency Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  |  |  | Same as 4.5.5.1 |  |  | Same as 4.5.5.1 |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  |  | Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.7 dB |  |  | Note: Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  |  | Same as 4.5.5.3 |  |  | Same as 4.5.5.3 |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  | ±0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  |  |  | Same as 4.5.6.1.1 |  |  | Same as 4.5.6.1.1 |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  |  | ±0.6 dB |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX |  |  | Noc ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB |  | Note: Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |  |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX |  |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX |  |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX |  |  | Same as 4.6.1.1 |  | Same as 4.6.1.1 |  |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection |  | Same as 4.6.1.1 | Same as 4.6.1.1 |  |  |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection |  | Same as 4.6.1.1 | Same as 4.6.1.1 |  |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX |  |  | Average Noc2 ±1.5 dB Average Noc3 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Average Ês3 / Noc3 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Noc3 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Meas PRB Ês3 / Noc3 ±0.3 dB |  | Noc2 is the AWGN on NR freq2 Noc3 is the AWGN on NR freq3 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX |  |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  | Same as 4.6.2.1 |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.1 |  | Same as 4.6.4.1 |  |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.3 |  | Same as 4.6.4.3 |  |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Ês3 / Noc is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.1.1 |  | Same as 4.7.1.1.1 |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês2 / Noc1 ±0.3 dB Average Ês3 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc1 ±0.3 dB Meas PRB Ês3 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês2 / Noc1 is the cell 2 SNR Ês3 / Noc2 is the cell 3 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  | Same as 4.7.1.2.1 |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy |  |  | Same as 4.7.1.1.1 |  | Same as 4.7.1.1.1 |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  | Same as 4.7.1.2.1 |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  | Same as 4.7.1.2.1 |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy |  |  | Same as 4.7.1.1.1 |  | Same as 4.7.1.1.1 |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  |  | Same as 4.7.1.2.1 |  | Same as 4.7.1.2.1 |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  |  | Same as 4.7.1.2.1 |  | Same as 4.7.1.2.1 |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.8 dB |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 2 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.1 |  | Same as 4.7.4.1.1 |  |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Average Noc2 ±1.5 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.8 dB |  | Noc1 is the AWGN on NR freq1 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 Noc2 is the AWGN on E-UTRA freq2 Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RB22 to RB27 |  |  |

Table F.1.1.2-4 Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause |  |  | Maximum Test System Uncertainty1 |  |  |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection |  |  | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  |  |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |  |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection |  |  | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  |  |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  |  | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  |  |  | Note: Noc1 is the AWGN on cell 1 (NR) frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 (E-UTRAN) frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  |  | Same as 6.1.2.1 |  |  |  | Same as 6.1.2.1 |  |  |
| 6.3.1.1 NR SA FR1 handover with known target cell |  |  | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |  |
| 6.3.1.2 NR SA FR1 handover with unknown target cell |  |  | Same as 6.3.1.1 |  |  | Same as 6.3.1.1 |  |  |  |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell |  |  | Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc2 ±1.5 dB Ês2 / Noc2 ±0.3 dB |  |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc2 is the AWGN on cell 2 frequency Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |  |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell |  |  | Same as 6.1.2.1 |  |  |  | Same as 6.1.2.1 |  |  |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell |  |  | Same as 6.1.2.1 |  |  |  | Same as 6.1.2.1 |  |  |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment |  |  | Same as 6.1.1.1 |  |  |  | Same as 6.1.1.1 |  |  |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment |  |  | Same as 6.1.1.2 |  |  |  | Same as 6.1.1.2 |  |  |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing |  |  | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  |  |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 6.3.2.2.1 |  |  | Noc ±1.5 dB Ês / Noc ±0.3 dB Uplink absolute power measurement ±1.5 dB Uplink relative power measurement ±0.7 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 6.3.2.2.2 |  |  | Same as 6.3.2.2.1 |  |  |  | Same as 6.3.2.2.1 |  |  |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  |  | Same as 6.1.1.2 |  |  |  | Same as 6.1.1.2 |  |  |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection |  |  | Same as 6.1.2.1 |  |  |  | Same as 6.1.2.1 |  |  |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy |  |  | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±112Tc Uplink signal transmit timing relative to downlink |  |  |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy |  |  | Noc ±1.5 dB Ês1 / Noc ±0.3 dB ±88Tc Timing Advance Adjustment accuracy |  |  |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  |  | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.3 dB Fading profile ±0.7 dB |  |  |  | Noc is the AWGN on cell 1 frequency Ês / Noc is the SNR Meas PRB are the measurement PRB for SSB-based RLM #RBJ to RBJ+19 |  |  |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  |  | Same as 6.5.1.1 |  |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 6.5.1.1 |  |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 6.5.1.1 |  |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 6.5.1.1 |  |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 6.5.1.1 |  |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 6.5.1.1 |  |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 6.5.1.1 |  |  |  | Same as 6.5.1.1 |  |  |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  |  | Noc1 ±1.5 dB Noc2 ±1.5 dB Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB |  |  |  | Note: Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  |  |  | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  |  | Same as 6.5.3.1 |  |  |  | Same as 6.5.3.1 |  |  |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 6.5.3.1 |  |  |  | Same as 6.5.3.1 |  |  |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  |  |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the SSB Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  |  | Same as 6.5.5.1 |  |  |  | Same as 6.5.5.1 |  |  |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  | Average Noc1 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Fading profile ±0.7 dB |  |  |  | Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the SNR for the CSI-RS Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  | Same as 6.5.5.3 |  |  |  | Same as 6.5.5.3 |  |  |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  |  | ±0.6 dB |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX |  |  | Same as 6.5.6.1.1 |  |  |  | Same as 6.5.6.1.1 |  |  |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX |  |  | ±0.6 dB |  |  |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |  |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX |  |  | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB |  |  |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |  |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX |  |  | Same as 6.6.1.1 |  |  |  | Same as 6.6.1.1 |  |  |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX |  |  | Same as 6.6.1.1 |  |  |  | Same as 6.6.1.1 |  |  |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX |  |  | Same as 6.6.1.1 |  |  |  | Same as 6.6.1.1 |  |  |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading |  |  | Same as 6.6.1.1 |  |  |  | Same as 6.6.1.1 |  |  |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  |  | Same as 6.6.1.1 |  |  |  | Same as 6.6.1.1 |  |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc1 ±1.5 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  |  |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  |  | Same as 6.6.2.1 |  |  |  | Same as 6.6.2.1 |  |  |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 6.6.2.1 |  |  |  | Same as 6.6.2.1 |  |  |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 6.6.2.1 |  |  |  | Same as 6.6.2.1 |  |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  |  | Freq 1 Noc ±1.5 dB Freq 2 Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Fading ±0.5 dB |  |  |  | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |  |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  |  | Same as 6.6.3.1 |  |  |  | Same as 6.6.3.1 |  |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 6.6.4.1 |  |  |  | Same as 6.6.4.1 |  |  |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Average Noc ±1.5 dB Average Ês0 / Noc ±0.3 dB Average Ês1 / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês0 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RSRP #RB0 to RB274 |  |  |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 6.6.4.3 |  |  |  | Same as 6.6.4.3 |  |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB meas PRB Ês1 / Noc ±0.3 dB meas PRB Ês2 / Noc ±0.3 dB |  |  |  | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy |  |  | Same as 6.7.1.1.1 |  |  |  | Same as 6.7.1.1.1 |  |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy |  |  | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB meas PRB Ês1 / Noc1 ±0.3 dB meas PRB Ês2 / Noc2 ±0.3 dB |  |  |  | Noc1 is the AWGN on NR freq1 Noc2 is the AWGN on NR freq2 Ês1 / Noc1 is the cell 2 SNR Ês2 / Noc2 is the cell 3 SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy |  |  | Same as 6.7.1.2.1 |  |  |  | Same as 6.7.1.2.1 |  |  |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy |  |  | Same as 6.7.1.1.1 |  |  |  | Same as 6.7.1.1.1 |  |  |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy |  |  | Same as 6.7.1.2.1 |  |  |  | Same as 6.7.1.2.1 |  |  |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy |  |  | Same as 6.7.1.2.1 |  |  |  | Same as 6.7.1.2.1 |  |  |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy |  |  | Same as 6.7.1.1.1 |  |  |  | Same as 6.7.1.1.1 |  |  |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy |  |  | Same as 6.7.1.2.1 |  |  |  | Same as 6.7.1.2.1 |  |  |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy |  |  | Same as 6.7.1.2.1 |  |  |  | Same as 6.7.1.2.1 |  |  |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.8 dB |  |  |  | Noc is the AWGN on NR freq1 Ês2 / Noc is the cell 1 SNR Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  |  | Same as 6.7.4.1.1 |  |  |  | Same as 6.7.4.1.1 |  |  |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  |  | Same as 6.7.4.1.1 |  |  |  | Same as 6.7.4.1.1 |  |  |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy |  |  | Same as 6.7.4.1.1 |  |  |  | Same as 6.7.4.1.1 |  |  |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy |  |  | Average Noc ±1.5 dB Average Ês / Noc ±0.3 dB Meas PRB Noc ±1.5 dB Meas PRB Ês / Noc ±0.8 dB |  |  |  | Noc is the AWGN on E-UTRA freq1 Ês / Noc is the ratio of cell 1 signal / AWGN Meas PRB are the measurement PRB for RSRP #RB22 to RB27 |  |  |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy |  |  | Same as 6.7.5.1 |  |  |  | Same as 6.7.5.1 |  |  |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy |  |  | Same as 6.7.5.1 |  |  |  | Same as 6.7.5.1 |  |  |

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês2 / Noc is the ratio of cell 2 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | Noc ±5.65 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.5 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used | Average Ês2 ±5.65, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.6.2.6 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 5.6.2.7 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 2 signal Ês2 is NR cell 3 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 2 signal / AWGN Ês2 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Same as 5.7.2.1 | Same as 5.7.2.1 |

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3.2.2.1 NR SA FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | Average Ês2 ±5.65 dB, f <= 40.8 GHz | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.6.2.6, SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB | Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| 7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | Same as 7.6.2.5 | Same as 7.6.2.5 |
| 7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | Same as 7.6.2.6 | Same as 7.6.2.6 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Same as 7.7.2.1 | Same as 7.7.2.1 |

Table F.1.1.2-7: Maximum test system uncertainty for RRM requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX | Average Noc1 ±1.5 dB Average Noc2 ±1.5 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Meas PRB Noc2 ±1.5 dB Meas PRB Ês2 / Noc2 ±0.3 dB Fading profile ±0.5 dB | Note: Noc1 is the AWGN on cell 1 frequency Noc2 is the AWGN on cell 2 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB on cell 2 are the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.4.2.2 E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.5.2.1.1.1 E-UTRA – NR FR1 SS-RSRP absolute measurement accuracy | Average Noc ±1.5 dB Noc over meas PRBs #RBJ-J+19 ±1.5 dB Average Ês / Noc ±0.3 dB meas PRB Ês1 / Noc1 ±0.8 dB | Noc is the AWGN on the NR freq Ês / Noc is the cell SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.1.2 E-UTRA – NR FR2 SS-RSRP absolute measurement accuracy | Test 1 Average Noc ±5.65 dB Noc over meas PRBs ±5.65 dB Average Ês / Noc ±0.3 dB Ês / Noc over meas PRBs ±0.3 dB Test 2 Average Es ±5.65 dB Es over meas PRBs ±5.65 dB | Test 1 Noc is the AWGN on the NR freq Ês / Noc is the cell SNR meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |
| 8.5.2.2.1 E-UTRA – NR FR1 SS-RSRQ measurement accuracy | Same as 8.5.2.1.1.1 | Same as 8.5.2.1.1.1 |
| 8.5.2.2.2 E-UTRA – NR FR2 SS-RSRQ measurement accuracy | Same as 8.5.2.1.2 | Same as 8.5.2.1.2 |
| 8.5.2.3.1 E-UTRA – NR FR1 SS-SINR measurement accuracy | Same as 8.5.2.1.1.1 | Same as 8.5.2.1.1.1 |
| 8.5.2.3.2 E-UTRA – NR FR2 SS-SINR measurement accuracy | Same as 8.5.2.1.2 | Same as 8.5.2.1.2 |

## F.1.2	Interpretation of measurement results (normative)

See TS 38.521-1 [17] Annex F.2.

## F.1.3	Test Tolerance and Derivation of Test Requirements (informative)

See TS 38.521-1 [17] Annex F.3.

### F.1.3.1	Measurement of test environments

See TS 38.521-1 [17] Annex F.3.1.

### F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

The derivation of the test requirements for the EN-DC FR1 test cases in chapter 4 is defined in Table F.1.3.2-1.

The derivation of the test requirements for the NR SA FR1 test cases in chapter 6 is defined in Table F.1.3.2-2.

The derivation of the test requirements for the EN-DC FR2 test cases in chapter 5 is defined in Table F.1.3.2-3.

The derivation of the test requirements for the NR SA FR2 test cases in chapter 7 is defined in Table F.1.3.2-4.

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test |  |  | Minimum requirement in TS 38.133 [6] |  |  | Test tolerance (TT) |  |  |  | Test requirement in TS 38.533 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4.3.2.2.1 |  |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  |  | 2.1dB 0.7dB 112Tc 112Tc |  |  |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |  |  |
| 4.3.2.2.2 |  |  | Same as 4.3.2.2.1 |  |  | Same as 4.3.2.2.1 |  |  |  | Same as 4.3.2.2.1 |  |  |  |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy |  |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB |  |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |  |  |  |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy |  |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  |  |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  |  |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |  |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  |  |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  |  |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |  |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  |  |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  |  |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section D.4.1.1 |  |  |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode |  |  | Same as 4.5.1.2 |  |  |  | Same as 4.5.1.2 |  |  |  | Same as 4.5.1.2 |  |  |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  | Same as 4.5.1.2 |  |  |  | Same as 4.5.1.2 |  |  |  | Same as 4.5.1.2 |  |  |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |  | Same as 4.5.1.1 |  |  |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode |  |  | Same as 4.5.1.2 |  |  |  | Same as 4.5.1.2 |  |  |  | Same as 4.5.1.2 |  |  |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC |  |  | SNRs as specified |  |  |  | 0.6dB |  |  |  | Formula: SNR + TT |  |  |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC |  |  | Same as 4.5.2.1 |  |  |  | Same as 4.5.2.1 |  |  |  | Same as 4.5.2.1 |  |  |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC |  |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |  | During T1: 0dB 0dB 0dB 0dB |  |  |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC |  |  | Same as 4.5.2.3 |  |  |  | Same as 4.5.2.3 |  |  |  | Same as 4.5.2.3 |  |  |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |  | During T1: 0dB 0dB 0dB 0dB |  |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC |  |  | Same as 4.5.2.5 |  |  |  | Same as 4.5.2.5 |  |  |  | Same as 4.5.2.5 |  |  |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |  |  |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  |  | Same as 4.5.3.1 |  |  |  | Same as 4.5.3.1 |  |  |  | Same as 4.5.3.1 |  |  |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  | Same as 4.5.3.1 |  |  |  | Same as 4.5.3.1 |  |  |  | Same as 4.5.3.1 |  |  |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay |  |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |  |  |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  |  |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  |  |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX |  |  | Same as 4.5.5.1 |  |  |  | Same as 4.5.5.1 |  |  |  | Same as 4.5.5.1 |  |  |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB |  |  |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  |  |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |  |  |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  | Same as 4.5.5.3 |  |  |  | Same as 4.5.5.3 |  |  |  | Same as 4.5.5.3 |  |  |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  |  |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC |  |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |  |  |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC |  |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |  | During T1: 0dB 0dB |  |  |  | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |  |  |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell |  |  |  | For Test Configurations 1,2,4,5 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS For Test Configurations 3,6 Noc: -88dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS |  |  |  | -0.6dB 0dB -3dB -0.6dB 0dB -3dB |  |  |  | Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -99 dBm/SCS Noc: -88.6dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |  |  |
| 4.6.1.1 EN-DC FR1 event-triggered reporting without gap in non-DRX |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  |
| 4.6.1.2 EN-DC FR1 event-triggered reporting without gap in DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |  | Same as 4.6.1.1 |  |  |
| 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX |  |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  |  |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  |  | During T1: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: -infinity During T2: Freq 2 Noc: -98dBm/15kHz Freq 3 Noc: -98dBm/15kHz Ês2 / Noc: +4.00dB Ês3 / Noc: +7.00dB |  |  |
| 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |  | Same as 4.6.2.1 |  |  |
| 4.6.4.1 EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX |  |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  |  |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  |  |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |  |
| 4.6.4.2 EN-DC FR1 SSB-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.1 |  |  |  | Same as 4.6.4.1 |  |  |  | Same as 4.6.4.1 |  |  |
| 4.6.4.3 EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  |  |  | 0dB 0dB 0.5dB Via mapping |  |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |  |  |
| 4.6.4.4 EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  | Same as 4.6.4.3 |  |  |  | Same as 4.6.4.3 |  |  |  | Same as 4.6.4.3 |  |  |
| 4.7.1.1.1 EN-DC FR1 SS-RSRP absolute measurement accuracy |  |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc: -106dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  |  |  | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  |  |  | Test 1: Noc: -107.5dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |  |  |
|  |  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc: -110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported RSRP values: ±6dB |  |  |  | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  |  |  | Test 1: Noc: -110.8dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |  |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.1.1.2 EN-DC FR1 SS-RSRP relative measurement accuracy |  |  | Test 1: Noc: Test configuration 1, 2, 4, 5:-106dBm/15kHz Test configuration 3, 6:-110dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported relative SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: -1.0dB Reported relative SS-RSRP values: ±3dB |  |  |  | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  |  | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês2 / Noc: +3.0dB Ês3 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |  |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.1.2.1 EN-DC FR1-FR1 SS-RSRP absolute measurement accuracy |  |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |  |  |
|  |  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  |  |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |  |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 3 SS-RSRP from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.1.2.2 EN-DC FR1-FR1 SS-RSRP relative measurement accuracy |  |  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês2 / Noc1: +10.0dB Ês3 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês2 / Noc1: +13.0dB Ês3 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |  |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 2 and Cell 3 RSRP from Noc, Ês2 / Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.2.1 EN-DC FR1 SS-RSRQ measurement accuracy |  |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc: -85dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc: -86.5dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês2 / Noc: -2.9dB Ês3 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |  |
|  |  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc: -91dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc: -92.6dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês2 / Noc: -4.0dB Ês3 / Noc: -4.0dB RSRQ_44 to RSRQ_59 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.2.2.1 EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy |  |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  |  |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |  |
|  |  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  |  |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.2.2.2 EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy |  |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  |  |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |  |
|  |  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  |  |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  |  |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês2 / Noc1: 3dB Ês3 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 and Cell 3 SS-RSRQ from Noc2 and Ês3 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.3.1 EN-DC FR1 SS-SINR measurement accuracy |  |  | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  |  |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |  |
|  |  |  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc: -93dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -4dB Ês3 / Noc: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  |  |  | Test 1: Noc: -93.2dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês2 / Noc: -3.5dB Ês3 / Noc: -3.5dB SINR_28 to SINR_45 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.3.2.1 EN-DC FR1-FR1 SS-SINR absolute measurement accuracy |  |  | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±3.5dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_32 to SINR_49 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.3.2.2 EN-DC FR1-FR1 SS-SINR relative measurement accuracy |  |  | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -4dB Ês3 / Noc2: -4dB Reported SINR values: ±4dB |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês2 / Noc1: -1.75dB Ês3 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês2 / Noc1: 20dB Ês3 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3.2dB Ês3 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 3 SS- SINR from Noc and Ês3 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4.7.4.1.1 EN-DC FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |  |  |
| Test Configuration 3,6 |  |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |  |  |
| 4.7.4.1.2 EN-DC FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 |  |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |  |
| Test Configuration 3,6 |  |  | Test 1: Noc1: -94.65 dBm/15kHz Ês2 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês2 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  |  | Test 1: Noc1: -96dBm/15kHz Ês2 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês2 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |  |
| 4.7.4.2.1 EN-DC FR1 CSI-RS based L1-RSRP absolute measurement accuracy |  |  | Same as 4.7.4.1.1 |  |  |  | Same as 4.7.4.1.1 |  |  |  | Same as 4.7.4.1.1 |  |  |
| 4.7.4.2.2 EN-DC FR1 CSI-RS based L1-RSRP relative measurement accuracy |  |  | Same as 4.7.4.1.2 |  |  |  | Same as 4.7.4.1.2 |  |  |  | Same as 4.7.4.1.2 |  |  |
| 4.7.5.1 EN-DC FR1 SFTD measurement accuracy |  |  | Noc1: -104dBm/15kHz Ês1 / Noc1: -3.0dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  |  |  | 0dB 0.3dB 0dB 0dB |  |  |  | Noc1: -104dBm/15kHz Ês1 / Noc1: -2.7dB Noc2: -104dBm/15kHz Ês2 / Noc2: 17.0dB |  |  |

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test |  |  |  | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  |  | Test requirement in TS 38.533 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |  |  |
| 6.1.1.2 NR SA FR1-FR1 cell re-selection |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB |  | During T1: 0dB -2dB 1.6dB 0.4dB During T2: 0dB 0dB 1.6dB 0dB During T3: 0dB 0dB 1.6dB 1.6dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB |  |  |
| 6.1.2.1 NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 1.6dB 1.6dB During T3: 0dB -2dB 1.6dB 0.4dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: 13.6dB During T3: Noc1: -98dBm/15kHz Noc2: -100dBm/15kHz Ês1 / Noc1: +15.6dB Ês2 / Noc2: -3.6dB |  |  |
| 6.1.2.2 NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +14dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: +14dB |  | During T1: -2dB 0dB 0.4dB 1.6dB During T2: 0dB 0dB 1.6dB 0dB |  |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -3.6dB Ês2 / Noc2: +15.6dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.6dB Ês2 / Noc2: +14dB |  |  |
| 6.3.1.1 NR SA FR1 handover with known target cell |  |  |  | TEST CONFIGURATION 1, 2 A3-Offset: 0dB |  | -1dB |  |  | A3-Offset: -1dB |  |  |
|  |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |  |  |
|  |  |  |  | TEST CONFIGURATION 3 A3-Offset: 0dB |  | -1dB |  |  | A3-Offset: -1dB |  |  |
|  |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +11dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB -1dB During T3: 0dB 0dB 0dB -1dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +10dB |  |  |
| 6.3.1.2 NR SA FR1 handover with unknown target cell |  |  |  | A3-Offset: 0dB During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +8dB |  | -1dB During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.5dB |  |  | A3-Offset: -1dB During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 8dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +8dB Ês2 / Noc2: +9.5dB |  |  |
| 6.3.1.3 NR SA FR1-FR1 Handover with unknown Target Cell |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 1.7dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB |  |  |
| 6.3.1.4 NR SA FR1 – E-UTRA handover with known target cell |  |  |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +12dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -4dB Ês2 / Noc2: +8dB |  | During T1: 0dB 0dB 1.55dB 0dB During T2: 0dB 0dB -1.55dB 1.55dB During T3: 0dB 0dB -1.55dB 1.55dB |  |  | During T1: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +13.55dB Ês2 / Noc2: -infinity During T2: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB During T3: Noc1: -100dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -5.55dB Ês2 / Noc2: +9.55dB |  |  |
| 6.3.1.5 NR SA FR1 – E-UTRA handover with unknown target cell |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: 0dB Ês2 / Noc2: +7dB |  |  |
| 6.3.2.1.1 NR SA FR1 RRC re-establishment |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +7dB Ês2 / Noc: +4dB During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  |  |
| 6.3.2.1.2 NR SA FR1 - FR1 RRC re-establishment |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: -infinity Ês2 / Noc2: +7dB |  |  |
| 6.3.2.1.3 NR SA FR1 RRC re-establishment without serving cell timing |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: -infinity During T3: Noc: -98dBm/15kHz Ês1 / Noc: -infinity Ês2 / Noc: +4dB |  |  |
| 6.3.2.2.1 |  |  |  | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc |  | 2.1dB 0.7dB 112Tc 112Tc |  |  | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |  |  |
| 6.3.2.2.2 |  |  |  | Same as 6.3.2.2.1 |  | Same as 6.3.2.2.1 |  |  | Same as 6.3.2.2.1 |  |  |
| 6.3.2.3.1 NR SA FR1 RRC connection release with redirection |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  |  |
| 6.3.2.3.2 NR SA FR1 – E-UTRA RRC connection release with redirection |  |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +4dB Ês2 / Noc2: +4dB |  |  |
| 6.4.1.1 EN-DC FR1 UE transmit timing accuracy |  |  |  | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês1 / Noc: +3.00dB |  | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB |  |  | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês1 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês1 / Noc: +3.30dB |  |  |
| 6.4.3.1 NR SA FR1 timing advance adjustment accuracy |  |  |  | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT |  | 0 0 0 +/- 88 Tc +/- 88 Tc |  |  | Noc = -98 dBm/15 kHz (Config 1, 2) Noc = -95 dBm/15 kHz (Config 3) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |  |  |
| 6.5.1.1 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB |  |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |  |  |
| 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode |  |  |  | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB |  | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB |  |  | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in section D.4.1.1 |  |  |
| 6.5.1.3 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode |  |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.4 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode |  |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |  | Same as 6.5.1.2 |  |  |
| 6.5.1.5 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.6 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode |  |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |  | Same as 6.5.1.2 |  |  |
| 6.5.1.7 NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  |  |  | Same as 6.5.1.1 |  | Same as 6.5.1.1 |  |  | Same as 6.5.1.1 |  |  |
| 6.5.1.8 NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode |  |  |  | Same as 6.5.1.2 |  | Same as 6.5.1.2 |  |  | Same as 6.5.1.2 |  |  |
| 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC |  |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |  |
| 6.5.3.1 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle |  |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |  |
| 6.5.3.2 NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 320ms SCell measurement cycle |  |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |  | Same as 6.5.3.1 |  |  |
| 6.5.3.3 NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX |  |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |  | Same as 6.5.3.1 |  |  |
| 6.5.5.1 NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX |  |  |  | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB |  |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 SSB during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |  |  |
| 6.5.5.2 NR SA FR1 SSB-based beam failure detection and link recovery in DRX |  |  |  | Same as 6.5.5.1 |  | Same as 6.5.5.1 |  |  | Same as 6.5.5.1 |  |  |
| 6.5.5.3 NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX |  |  |  | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês1 / Noc1: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês1 / Noc1: +10dB |  | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB |  |  | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 CSI-RS during T1: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T2: Noc1: -98dBm/15kHz Ês1 / Noc1: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc1: -98dBm/15kHz Ês1 / Noc1: +10.2dB |  |  |
| 6.5.5.4 NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX |  |  |  | Same as 6.5.5.3 |  | Same as 6.5.5.3 |  |  | Same as 6.5.5.3 |  |  |
| 6.5.6.1.1 NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX |  |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |  |
| 6.5.6.1.2 NR SA FR1 DCI-based DL active BWP switch in non-DRX |  |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T2: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB During T3: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  |  |
| 6.5.6.2.1 NR SA FR1 RRC-based DL active BWP switch in non-DRX |  |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  | During T1: 0dB 0dB |  |  | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |  |  |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX |  |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB |  |  | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  |  |
| 6.6.1.2 SA event triggered reporting tests without gap under DRX |  |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |  | Same as 6.6.1.1 |  |  |
| 6.6.1.3 SA event triggered reporting tests with per-UE gaps under non-DRX |  |  |  | Same as 64.6.1.1 |  | Same as 6.6.1.1 |  |  | Same as 6.6.1.1 |  |  |
| 6.6.1.4 SA event triggered reporting tests with per-UE gaps under DRX |  |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |  | Same as 6.6.1.1 |  |  |
| 6.6.1.5 SA event triggered reporting tests without gap under non-DRX with SSB index reading |  |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |  | Same as 6.6.1.1 |  |  |
| 6.6.1.6 SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading |  |  |  | Same as 6.6.1.1 |  | Same as 6.6.1.1 |  |  | Same as 6.6.1.1 |  |  |
| 6.5.3.1 |  |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB |  |  | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T2: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB During T3: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB |  |  |
| 6.5.3.2 |  |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |  | Same as 6.5.3.1 |  |  |
| 6.5.3.3 |  |  |  | Same as 6.5.3.1 |  | Same as 6.5.3.1 |  |  | Same as 6.5.3.1 |  |  |
| 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX |  |  |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB |  |  | During T1: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -98dBm/15kHz Freq 2 Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +7.00dB |  |  |
| 6.6.2.2 NR SA FR1-FR1 event-triggered reporting in DRX |  |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |  |
| 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection |  |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |  |
| 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection |  |  |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |  |
| 6.6.3.1 NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX |  |  |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +18.00dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -2dB Ês2 / Noc: +19.00dB |  | During T1: 0dB 0dB +1.65dB 0dB During T2: 0dB 0dB -1.65dB 1.65dB |  |  | During T1: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: +19.65dB Ês2 / Noc: -infinity During T2: Freq 1 Noc: -106dBm/15kHz Freq 2 Noc: -106dBm/15kHz Ês1 / Noc: -3.65dB Ês2 / Noc: +20.65dB |  |  |
| 6.6.3.2 NR SA FR1 – E-UTRAN event-triggered reporting in DRX |  |  |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |  |
| 6.6.4.1 NR SA FR1 SSB-based L1-RSRP measurement in non-DRX |  |  |  | During T1: Noc: -98dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported SSB#0 SS-RSRP values ± 10dB Reported SSB#1 SS-RSRP values ± 10dB Reported Differential SS-RSRP values ± 3dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.5dB Via mapping |  |  | During T1: Noc: -94.65dBm/15kHz Ês0 / Noc: +0.00dB Ês1 / Noc: -infinity During T2: Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 SSB#1: RSRP_55 to RSRP_75 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 SSB#1: RSRP_58 to RSRP_78 SSB#0: DIFFRSRP_0 to DIFFRSRP_3 |  |  |
| 6.6.4.2 NR SA FR1 SSB-based L1-RSRP measurement in DRX |  |  |  | Same as 6.6.4.1 |  | Same as 6.6.4.1 |  |  | Same as 6.6.4.1 |  |  |
| 6.6.4.3 NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX |  |  |  | Noc: -98dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.00dB Reported CSI-RSRP values ± 10dB Reported Differential CSI-RSRP values ± 3dB |  | 0dB 0dB 0.5dB Via mapping |  |  | Noc: -94.65dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +3.50dB For configuration 1,2,4,5 CSI-RS#1: RSRP_55 to RSRP_75 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 For configuration 3,6 CSI-RS#1: RSRP_58 to RSRP_78 CSI-RS#0: DIFFRSRP_0 to DIFFRSRP_3 |  |  |
| 6.6.4.4 NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX |  |  |  | Same as 6.6.4.3 |  | Same as 6.6.4.3 |  |  | Same as 6.6.4.3 |  |  |
| 6.7.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy |  |  |  | TEST CONFIGURATION 1, 2, |  |  |  |  |  |  |  |
|  |  |  |  | Test 1: Noc: -106dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -1.5dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  |  | Test 1: Noc: -107.5dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_44 to RSRP_56 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_60 to RSRP_79 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_34 to RSRP_46 RSRP_34 to RSRP_46 RSRP_35 to RSRP_47 RSRP_35 to RSRP_47 RSRP_36 to RSRP_48 RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |  |  |
|  |  |  |  | TEST CONFIGURATION 3 |  |  |  |  |  |  |  |
|  |  |  |  | Test 1: Noc: -110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±4.5dB Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported RSRP values: ±8dB Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported RSRP values: ±6dB |  | Test 1: -0.8dB 0dB +0.4dB Via mapping Test 2: 0dB 0dB +0.4dB Via mapping Test 3: 0dB 0dB +0.2dB Via mapping |  |  | Test 1: Noc: -110.8dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_41 to RSRP_53 Test 2: Noc: -91dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB RSRP_57 to RSRP_76 Test 3: Noc: -113dBm/30kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -0.8dB RSRP_37 to RSRP_49 RSRP_37 to RSRP_49 RSRP_38 to RSRP_50 RSRP_38 to RSRP_50 RSRP_39 to RSRP_51 RSRP_40 to RSRP_52 RSRP_40 to RSRP_52 depending on operating band |  |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. In all cases the RSRP values are 4.5dB wider at each end for extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy |  |  |  | Test 1: Noc: Test configuration 1, 2:-106dBm/15kHz Test configuration 3:-110dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported rel. SS-RSRP values: ±3dB Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: -1.0dB Reported re. SS-RSRP values: ±3dB |  | Test 1: 0 dB 0 dB 0 dB +1.0dB Via mapping Test 2: 0dB 0dB +1.0dB Via mapping Test 3: 0dB 0dB +1.0dB Via mapping |  |  | Test 1: Noc: -106 dBm/15kHz Noc: -110 dBm/30kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 2: Noc: -88dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +2.0dB RSRP_x-8 to RSRP_x-1 Test 3: Noc: -116dBm/15kHz + ΔBG_offset Ês1 / Noc: +3.0dB Ês2 / Noc: 0dB RSRP_x-7 to RSRP_x+1 |  |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The RSRP values given above are for both normal and extreme conditions. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_62 to RSRP_81 Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc2: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 depending on operating band |  |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported RSRP values: ±8dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported RSRP values: ±4.5dB |  |  |  |  | Test 1: -1.35dB -1.35dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Noc2: -96dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 RSRP_37 to RSRP_50 RSRP_38 to RSRP_51 RSRP_39 to RSRP_51 depending on operating band |  |  |
| The derivation of the RSRP values takes into account the uncertainty in Cell 2 SS-RSRP from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 3.0 dB wider at each end for Test 1, and 4.5 dB wider at each for Test 2. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy | Test 1: Noc1: -94.65 dBm/15kHz Noc2: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB Reported relative RSRP values: ±4.5dB Test 2: Noc1: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB Reported relative RSRP values: ±4.5dB |  |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Noc2: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB Ês2 / Noc2: +10.0dB RSRP_x-7 to RSRP_x+7 Test 2: Noc: -107dBm/15kHz + ΔBG_offset Noc1: -118dBm/15kHz + ΔBG_offset Ês1 / Noc1: +13.0dB Ês2 / Noc2: -3.0dB RSRP_x-31 to RSRP_x-18 |  |  |
| The derivation of the SS-RSRP values takes into account the uncertainty in Cell 1 and Cell 2 RSRP from Noc, Ês1 / Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRP values given above are for normal conditions. For extreme conditions, the RSRP values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.2.1 NR SA FR1 SS-RSRQ measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc: -85dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB Reported RSRQ values: ±3.5dB Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  |  | Test 1: -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc: -86.5dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: Noc: -101dBm/15kHz Ês1 / Noc: -2.9dB Ês2 / Noc: -2.9dB RSRQ_46 to RSRQ_60 Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |  |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc: -91dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported RSRQ values: ±2.5dB Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -4.0dB Ês2 / Noc: -4.0dB Reported RSRQ values: ±3.5dB |  |  |  |  | Test 1: -1.6dB 0dB 0dB Via mapping Test 2: N/A Test 3: 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc: -92.6dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB RSRQ_52 to RSRQ_62 Test 2: N/A Test 3: Noc: -114dBm/15kHz + ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB RSRQ_44 to RSRQ_59 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the SS-RSRQ values are 1.5dB wider at each end for Test 1, and 0.5 dB wider at each for Tests 2 and 3. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  |  |  |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB |  |  |  |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_52 to RSRQ_62 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1.5dB wider at each end. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -80.18 dBm/15kHz Noc2: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  |  |  |  | Test 1: -1.5dB -1.5dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc1: -81.68dBm/15kHz Noc2: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -86.27 dBm/15kHz Noc2: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±2.5dB Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB Reported RSRQ values: ±3dB |  |  |  |  | Test 1: -1.53dB -1.53dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0dB 0dB Via mapping |  |  | Test 1: Noc1: -87.8dBm/15kHz Noc2: -87.8dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -106dBm/15kHz Noc2: -106dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB RSRQ_x-7 to RSRQ_x+7 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Noc2: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: 3dB Ês2 / Noc2: -1.75dB RSRQ_x-11 to RSRQ_x+2 |  |  |
| The derivation of the RSRQ values takes into account the uncertainty in Cell 1 and Cell 2 SS-RSRQ from Noc2 and Ês2 / Noc2, the allowed UE reporting accuracy, and the UE mapping function. The SS-RSRQ values given above are for normal conditions. For extreme conditions, the RSRQ values are 1dB wider at each end. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.3.1 NR SA FR1 SS-SINR measurement accuracy | TEST CONFIGURATION 1, 2, 4, 5 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  |  |  |  | Test 1: 0dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  |  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |  |  |
|  | TEST CONFIGURATION 3, 6 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc: -93dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB Reported SINR values: ±3.5dB Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -4dB Ês2 / Noc: -4dB Reported SINR values: ±3.5dB |  |  |  |  | Test 1: -0.2dB 0dB 0dB Via mapping Test 2: 0dB 0.5dB 0.5dB Via mapping |  |  | Test 1: Noc: -93.2dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +4.54dB SINR_31 to SINR_49 Test 2: Noc: -116dBm/15kHz+ ΔBG_offset Ês1 / Noc: -3.5dB Ês2 / Noc: -3.5dB SINR_28 to SINR_45 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each end. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±3.5dB |  |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_32 to SINR_49 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 1dB wider at each for Tests 1 and 2 and 0.5dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy | TEST CONFIGURATION 1, 2, 3, 4, 5, 6 |  |  |  |  |  |  |  |  |  |  |
|  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported SINR values: ±3.5dB Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB Reported SINR values: ±3.5dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Ês2 / Noc2: -4dB Reported SINR values: ±4dB |  |  |  |  | Test 1: 0dB 0dB 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 0dB Via mapping Test 3: 0dB 0dB 0.8dB 0.8dB Via mapping |  |  | Test 1: Noc1: -88 dBm/15kHz Noc2: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB SINR_x-10 to SINR_x+10 Test 2: Noc1: -108.5dBm/15kHz Noc2: -108.5dBm/15kHz Ês1 / Noc1: 20dB Ês2 / Noc2: 20dB SINR_x-10 to SINR_x+10 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Noc2: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB Ês2 / Noc2: -3.2dB SINR_x-11 to SINR_x+11 |  |  |
| The derivation of the SINR values takes into account the uncertainty in Cell 1 and Cell 2 SS- SINR from Noc and Ês2 / Noc, the allowed UE reporting accuracy, and the UE mapping function. The SS- SINR values given above are for normal conditions. For extreme conditions, the SS- SINR values are 0.5dB wider at each for Tests 1 and 2 and 0dB wider at each end for Test 3. |  |  |  |  |  |  |  |  |  |  |  |
| 6.7.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy |  |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_62 to RSRP_82 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_31 to RSRP_44 RSRP_31 to RSRP_45 RSRP_32 to RSRP_45 RSRP_32 to RSRP_46 RSRP_33 to RSRP_46 RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 depending on operating band |  |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±4.5dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±3dB additionally for extreme conditions) |  |  |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_63 to RSRP_84 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_34 to RSRP_47 RSRP_34 to RSRP_48 RSRP_35 to RSRP_48 RSRP_35 to RSRP_49 RSRP_36 to RSRP_49 RSRP_37 to RSRP_50 RSRP_37 to RSRP_51 depending on operating band |  |  |
| 6.7.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy |  |  |  |  |  |  |  |  |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -119.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |  |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: 10dB Reported L1-RSRP values: ±8.5dB (±1dB additionally for extreme conditions) Test 2: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3dB Reported L1-RSRP values: ±5dB (±1dB additionally for extreme conditions) |  |  |  |  | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc1: -96dBm/15kHz Ês1 / Noc1: 10dB RSRP_x-3 to RSRP_x+3 Test 2: Noc1: -116.2dBm/15kHz + ΔBG_offset Ês1 / Noc1: -2.2dB RSRP_x-3 to RSRP_x+3 |  |  |
| 6.7.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy | Same as 6.7.4.1.1 |  |  |  |  | Same as 6.7.4.1.1 |  |  | Same as 6.7.4.1.1 |  |  |
| 6.7.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy | Same as 6.7.4.1.2 |  |  |  |  | Same as 6.7.4.1.2 |  |  | Same as 6.7.4.1.2 |  |  |
| 6.7.5.1 NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: +10.0dB Reported RSRP values: ±9dB (±2dB additionally for extreme conditions) Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRP values: ±4.5dB (±3.5dB additionally for extreme conditions) |  |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping |  |  | Test 1: Noc: -91.65dBm/15kHz Ês / Noc: 10dB RSRP_48 to RSRP_70 Test 2: Noc: -117dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRP_14 to RSRP_27 RSRP_15 to RSRP_27 RSRP_15 to RSRP_28 RSRP_16 to RSRP_28 RSRP_16 to RSRP_29 RSRP_17 to RSRP_30 RSRP_18 to RSRP_30 Depending on operating band |  |  |
| 6.7.6.1 NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RSRQ values: ±3.5dB (±0.5dB additionally for extreme conditions) |  |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping Test 3: 0dB 0.8dB Via mapping |  |  | Test 1: Noc: -83dBm/15kHz Ês / Noc: -1.75dB RSRQ_4 to RSRQ_16 Test 2: Noc: -104.7dBm/15kHz Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB RSRQ_0 to RSRQ_16 |  |  |
| 6.7.7.1 NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB Reported RS-SINR values: ±3.0dB (±1.0dB additionally for extreme conditions) Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -4.0dB Reported RS-SINR values: ±3.5dB (±0.5dB additionally for extreme conditions) |  |  |  |  | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping |  |  | Test 1: Noc: -88dBm/15kHz Ês / Noc: -1.75dB SINR_35 to SINR_51 Test 2: Noc: -108.5dBm/15kHz Ês / Noc: 20.0dB SINR_79 to SINR_94 Test 3: Noc: -119.5dBm/15kHz+ ΔBG_offset Ês / Noc: -3.2dB SINR_32 to SINR_49 |  |  |

Table F.1.3.2-4: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 5.3.2.2.2 EN-DC FR2 non-contention based random access | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 | Same as 5.3.2.2.1 |
| 5.4.1.1 EN-DC FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês2 / Noc: +4.0dB |
| 5.4.3.1 EN-DC FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 5.6.1.2 EN-DC FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -6.0 | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 5.6.1.4 EN-DC FR2 event-triggered reporting with gap in DRX | Same as 5.6.1.2 | Same as 5.6.1.2 | Same as 5.6.1.2 |
| 5.6.2.1 EN-DC FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: -- infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 5.6.2.3 EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 5.6.2.1 | Same as 5.6.2.1 | Same as 5.6.2.1 |
| 5.6.2.5 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 5.6.2.6 EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 5.6.2.7 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 5.6.2.8 EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 5.7.1.1 EN-DC FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -110.0dBm/120kHz n257 Ês3: -110.0dBm/120kHz n260 Ês2: -107.4dBm/120kHz n260 Ês3: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_50 to RSRP_108 Cell 3: RSRP_46 to RSRP_103 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.3dBm/120kHz n257 Ês3: -102.3dBm/120kHz n260 Ês2: -99.7dBm/120kHz n260 Ês3: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 5.7.2.1 EN-DC FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês2 / Noc: +3.0dB Ês3 / Noc: +3.0dB Cell 3: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Cell 3: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 5.7.3.1 EN-DC FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -3.0dB Ês3 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês2 / Noc: +4.54dB Ês3 / Noc: +2.66dB Cell 3: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês2 / Noc: -2.8dB Ês3 / Noc: -2.8dB Cell 3: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |

Table F.1.3.2-5: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 7.3.2.2.1 NR SA FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 7.6.2.6 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | Same as 5.6.2.5 | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | Same as 5.6.2.6 | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | Test 1: Noc: -91.60dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês1: -110.0dBm/120kHz n257 Ês2: -110.0dBm/120kHz n260 Ês1: -107.4dBm/120kHz n260 Ês2: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB Cell 1: RSRP_50 to RSRP_108 Cell 2: RSRP_46 to RSRP_103 Cell 2: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês1: -102.3dBm/120kHz n257 Ês2: -102.3dBm/120kHz n260 Ês1: -99.7dBm/120kHz n260 Ês2: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Cell 2: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Cell 2: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Cell 2: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -2.8dB Ês2 / Noc: -2.8dB Cell 2: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |

Table F.1.3.2-6: Derivation of test requirements for E-UTRA – NR inter-RAT test cases with E-UTRA serving cell RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 8.2.1.1 E-UTRA – NR FR1 cell re-selection to higher priority NR target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -4dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: +12dB | During T1: 0dB -1.90dB 0dB 0.35dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 1.55dB 1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -99.90dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -3.65dB During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +14dB Ês2 / Noc2: -infinity During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.55dB Ês2 / Noc2: +13.55dB |
| 8.3.1.1 E-UTRA – NR FR1 handover with known target cell | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: 0dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.55dB +1.55dB During T3: 0dB 0dB -1.55dB +1.55dB | During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB During T3: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +5.45dB Ês2 / Noc2: +1.55dB |
| 8.4.1.1 E-UTRA – NR FR1 SFTD measurement delay in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +4dB |
| 8.4.1.2 E-UTRA – NR FR1 SFTD measurement delay in DRX | Same as 8.4.1.1 | Same as 8.4.1.1 | Same as 8.4.1.1 |
| 8.4.2.1 E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in non-DRX | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +7dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB -1.65dB +1.65dB | During T1: Noc1: -104dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Ês1 / Noc1: +15.35dB Ês2 / Noc2: +8.65dB |
| 8.4.2.2 E-UTRA – NR FR1 event-triggered reporting without SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.3 E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in non-DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.4.2.4 E-UTRA – NR FR1 event-triggered reporting with SSB time index detection in DRX | Same as 8.4.2.1 | Same as 8.4.2.1 | Same as 8.4.2.1 |
| 8.5.2.1.1.1 E-UTRA – NR FR1 SS-RSRP absolute measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -94.65dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_62 to RSRP_82 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_30 to RSRP_43 RSRP_31 to RSRP_43 RSRP_31 to RSRP_44 RSRP_32 to RSRP_44 RSRP_32 to RSRP_45 RSRP_33 to RSRP_45 RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 depending on operating band |
| Test Configuration 3,6 | Test 1: Noc1: -94.65 dBm/15kHz Ês1 / Noc1: +10.0dB Reported RSRP values: ±8dB (±3.0dB additionally for extreme conditions) Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4.0dB Reported RSRP values: ±4.5dB (±4.5dB additionally for extreme conditions) | Test 1: -1.35dB 0dB Via mapping Test 2: 0dB 0.8dB Via mapping | Test 1: Noc1: -96.00dBm/15kHz Ês1 / Noc1: +10.0dB RSRP_64 to RSRP_83 Test 2: Noc1: -117dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB RSRP_33 to RSRP_46 RSRP_34 to RSRP_46 RSRP_34 to RSRP_47 RSRP_35 to RSRP_47 RSRP_35 to RSRP_48 RSRP_36 to RSRP_48 RSRP_36 to RSRP_49 RSRP_37 to RSRP_49 depending on operating band |
| 8.5.2.1.2 E-UTRA – NR FR2 SS-RSRP absolute measurement accuracy | Test 1: Noc: -105dBm/15kHz Ês / Noc 11dB Reported RSRP values: ±8dB (±3dB additionally for extreme conditions) Test 2: Ês FR2a PC3: -109.1dBm/SCS Reported RSRP values: ±8dB Ês FR2b PC3: -106.5dBm/SCS Reported RSRP values: ±8dB Ês FR2c PC3: -105.5dBm/SCS Reported RSRP values: ±8dB (±3dB additionally for extreme conditions in all frequency ranges) | Test 1: -0.1dB 0dB Via mapping Test 2: +5.65dB Via mapping +5.65dB Via mapping +5.65dB Via mapping | Test 1: Noc: -105.1dBm/15kHz Ês / Noc: +11.0dB RSRP_48 to RSRP_105 Test 2: Ês: -103.45dBm/SCS RSRP_29 to RSRP_87 Ês: -100.85dBm/SCS RSRP_32 to RSRP_89 Ês: -99.85dBm/SCS RSRP_33 to RSRP_90 |
| 8.5.2.2.1 E-UTRA – NR FR1 SS-RSRQ measurement accuracy |  |  |  |
| Test Configuration 1,2,4,5 | Test 1: Noc1: -80.18 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.5dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -81.68dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -106dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| Test Configuration 3,6 | Test 1: Noc1: -86.27 dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) | Test 1: -1.53dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0dB Via mapping | Test 1: Noc1: -87.80dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 2: Noc1: -113dBm/15kHz Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 Test 3: Noc1: -116dBm/15kHz + ΔBG_offset Ês1 / Noc1: -1.75dB RSRQ_51 to RSRQ_63 |
| 8.5.2.2.2 E-UTRA – NR FR2 SS-RSRQ absolute measurement accuracy | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions) Test 2: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1.75dB Reported RSRQ values: ±2.5dB (±1.5dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB 0dB 5.3dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB RSRQ_23 to RSRQ_95 Test 2: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.55dB RSRQ_26 to RSRQ_98 |
| 8.5.2.3.1 E-UTRA – NR FR1 SS-SINR measurement accuracy | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB Reported SINR values: ±3dB Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB Reported SINR values: ±3dB Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -4dB Reported SINR values: ±3.5dB | Test 1: 0dB 0dB Via mapping Test 2: 0dB 0dB Via mapping Test 3: 0dB 0.8dB Via mapping | Test 1: Noc1: -88 dBm/15kHz Ês1 / Noc1: -1.75dB SINR_35 to SINR_51 Test 2: Noc1: -108.5dBm/15kHz Ês1 / Noc1: 20dB SINR_79 to SINR_94 Test 3: Noc1: -119.5dBm/15kHz + ΔBG_offset Ês1 / Noc1: -3.2dB SINR_32 to SINR_49 |
| 8.5.2.3.2 E-UTRA – NR FR2 SS-SINR absolute measurement accuracy | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 2: Noc1: -105dBm/15kHz Ês1 / Noc1: 11dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions) Test 3: Noc1 FR2a PC3: -115.1dBm/15kHz Noc1 FR2b PC3: -112.5dBm/15kHz Noc1 FR2c PC3: -111.5dBm/15kHz Ês1 / Noc1: -1dB Reported SINR values: ±3dB (±1dB additionally for extreme conditions in all frequency ranges) | Test 1: 0dB 0dB Via mapping Test 2: -0.1dB 0dB Via mapping Test 3: 0dB 0dB 0dB 4.6dB Via mapping | Test 1: Noc1: -105dBm/15kHz Ês1 / Noc1: -0.5dB SINR_9 to SINR_82 Test 2: Noc1: -105.1dBm/15kHz Ês1 / Noc1: 11dB SINR_32 to SINR_105 Test 3: Noc1: -115.1dBm/15kHz Noc1: -112.5dBm/15kHz Noc1: -111.5dBm/SCS Ês1 / Noc1: 3.6dB SINR_17 to SINR_90 |

Annex G (normative):
Statistical testing

# G.1	General

The test requirements are expressed as absolute requirements with a single value stating the requirement or expressed as a success rate. The statistical nature depends on the type of test requirement. Some have large statistical variations, while others are not statistical in nature at all. When testing a parameter with a statistical nature, a confidence level is set. This establishes the probability that a Device Under Test (DUT) passing the test actually meets the test requirement and determines how many times a test have to be repeated and what the pass and fail criteria is. This Annex describes how to set the statistical significance.

# G.2	Statistical testing of delay and UE measurement performance in RRM tests

## G.2.1	General

The RRM tests are either of deterministic or of statistical nature. The pass fail limits in tests of statistical nature are expressed as a limit (e.g. delay limit) and a success ratio applicable for the limit. The success ratio is 90% uniform (the complement is the error ratio ER = 10%).

## G.2.2	Design of the test

The test is defined by the following design principles (see TS 36.521-1 clause G.X, Theory):

1) The early decision concept is applied.

2) A second limit is introduced: bad DUT factor M>1

To decide the test pass:

Supplier risk is applied based on the bad DUT quality

To decide the test fails

Customer risk is applied based on the specified DUT quality

The test is defined by the following parameters:

1) Limit ER = 0.1 (success ratio = 90%)

2) Bad DUT factor M=1.5 (selectivity)

3) Confidence level CL = 95% (for specified DUT and bad DUT-quality)

## G.2.3	Numerical definition of the pass fail limits

Editor’s Note:

-	Further investigate the technical details behind this statistical method to ensure that this is applicable for FR2 radiated test cases.

Table G.2.3-1: pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 33 | NA | 43 | 408 | 283 | 86 | 737 | 644 | 129 | 1056 | 1021 |
| 1 | 46 | NA | 44 | 416 | 291 | 87 | 745 | 653 | 130 | 1064 | 1030 |
| 2 | 58 | NA | 45 | 424 | 299 | 88 | 752 | 661 | 131 | 1071 | 1039 |
| 3 | 69 | NA | 46 | 432 | 307 | 89 | 760 | 670 | 132 | 1078 | 1048 |
| 4 | 79 | NA | 47 | 440 | 315 | 90 | 767 | 679 | 133 | 1086 | 1057 |
| 5 | 89 | NA | 48 | 447 | 324 | 91 | 775 | 687 | 134 | 1093 | 1066 |
| 6 | 99 | NA | 49 | 455 | 332 | 92 | 782 | 696 | 135 | 1100 | 1074 |
| 7 | 109 | NA | 50 | 463 | 340 | 93 | 790 | 705 | 136 | 1108 | 1083 |
| 8 | 118 | NA | 51 | 471 | 348 | 94 | 797 | 713 | 137 | 1115 | 1092 |
| 9 | 127 | NA | 52 | 478 | 356 | 95 | 804 | 722 | 138 | 1122 | 1101 |
| 10 | 136 | 39 | 53 | 486 | 365 | 96 | 812 | 731 | 139 | 1130 | 1110 |
| 11 | 145 | 45 | 54 | 494 | 373 | 97 | 819 | 739 | 140 | 1137 | 1119 |
| 12 | 154 | 51 | 55 | 502 | 381 | 98 | 827 | 748 | 141 | 1144 | 1128 |
| 13 | 163 | 58 | 56 | 509 | 389 | 99 | 834 | 757 | 142 | 1152 | 1137 |
| 14 | 172 | 64 | 57 | 517 | 398 | 100 | 842 | 766 | 143 | 1159 | 1147 |
| 15 | 180 | 71 | 58 | 525 | 406 | 101 | 849 | 774 | 144 | 1166 | 1155 |
| 16 | 189 | 78 | 59 | 532 | 414 | 102 | 857 | 783 | 145 | 1174 | 1164 |
| 17 | 197 | 85 | 60 | 540 | 423 | 103 | 864 | 792 | 146 | 1181 | 1173 |
| 18 | 206 | 92 | 61 | 548 | 431 | 104 | 871 | 801 | 147 | NA | 1182 |
| 19 | 214 | 99 | 62 | 555 | 440 | 105 | 879 | 809 | 148 |  |  |
| 20 | 223 | 106 | 63 | 563 | 448 | 106 | 886 | 818 | 149 |  |  |
| 21 | 231 | 113 | 64 | 571 | 456 | 107 | 894 | 827 | 150 |  |  |
| 22 | 239 | 120 | 65 | 578 | 465 | 108 | 901 | 836 | 151 |  |  |
| 23 | 248 | 128 | 66 | 586 | 473 | 109 | 909 | 844 | 152 |  |  |
| 24 | 256 | 135 | 67 | 594 | 482 | 110 | 916 | 853 | 153 |  |  |
| 25 | 264 | 142 | 68 | 601 | 490 | 111 | 923 | 862 | 154 |  |  |
| 26 | 272 | 150 | 69 | 609 | 499 | 112 | 931 | 871 | 155 |  |  |
| 27 | 281 | 157 | 70 | 616 | 507 | 113 | 938 | 880 | 156 |  |  |
| 28 | 289 | 165 | 71 | 624 | 516 | 114 | 946 | 888 | 157 |  |  |
| 29 | 297 | 173 | 72 | 632 | 524 | 115 | 953 | 897 | 158 |  |  |
| 30 | 305 | 180 | 73 | 639 | 533 | 116 | 960 | 906 | 159 |  |  |
| 31 | 313 | 188 | 74 | 647 | 541 | 117 | 968 | 915 | 160 |  |  |
| 32 | 321 | 196 | 75 | 654 | 550 | 118 | 975 | 924 | 161 |  |  |
| 33 | 329 | 204 | 76 | 662 | 558 | 119 | 983 | 933 | 162 |  |  |
| 34 | 337 | 211 | 77 | 669 | 567 | 120 | 990 | 941 | 163 |  |  |
| 35 | 345 | 219 | 78 | 677 | 575 | 121 | 997 | 950 | 164 |  |  |
| 36 | 353 | 227 | 79 | 684 | 584 | 122 | 1005 | 959 | 165 |  |  |
| 37 | 361 | 235 | 80 | 692 | 592 | 123 | 1012 | 968 | 166 |  |  |
| 38 | 369 | 243 | 81 | 700 | 601 | 124 | 1019 | 977 | 167 |  |  |
| 39 | 377 | 251 | 82 | 707 | 610 | 125 | 1027 | 986 | 168 |  |  |
| 40 | 385 | 259 | 83 | 715 | 618 | 126 | 1034 | 994 | 169 |  |  |
| 41 | 393 | 267 | 84 | 722 | 627 | 127 | 1042 | 1003 |  |  |  |
| 42 | 400 | 275 | 85 | 730 | 635 | 128 | 1049 | 1012 |  |  |  |

The first column is the number of errors (ne = number of exceeded delays or number of wrong reports)

The second column is the number of samples for the pass limit (nsp , ns=Number of samples= number of successes + number of exceedings or number of reports)

The third column is the number of samples for the fail limit (nsf)

## G.2.4	Pass fail decision rules

The pass fail decision rules apply for a single test, comprising one component in the test vector. The over all Pass /Fail conditions are defined in clause G.2.6

Having observed 0 errors, pass the test at 33+ samples, otherwise continue

Having observed 1 error, pass the test at 46+ samples, otherwise continue

Having observed 2 errors, pass the test at 58+ samples, otherwise continue

Having observed 10 errors, pass the test at 136+ samples, fail the test at 39 samples, otherwise continue

Having observed 146 errors, pass the test at 1181+ samples, fail the test at 1173- samples, otherwise continue

Having observed 147 errors, fail the test at 1182- samples,

Where x+ means: x or more, x- means x or less

NOTE 1: an ideal DUT passes after 33 samples. The maximum test time is 1181 samples.

## G.2.5	Void

## G.2.6	Test conditions for delay tests and UE measurement performance

Table G.2.6-1: test conditions

| Test | Statistical independence | Number of components in the test vector, as specified in the test requirements and initial conditions of the applicable test | Over all Pass/Fail condition |
| --- | --- | --- | --- |
| All tests in clauses 4.4.3, 4.5, 4.6, 5.4.3, 5.5, 5.6, 6.1, 6.2, 6.3.1, 6.3.2.1, 6.3.2.3, 6.4.3, 6.5, 6.6, 7.1, 7.2, 7.3.1, 7.3.2.1, 7.3.2.3, 7.4.3, 7.5, 7.6 are delay tests of statistical nature while 4.3.2.2, 4.4.1, 5.3.2.2, 5.4.1, 6.3.2.2, 6.4.1, 7.3.2.2, 7.4.1 are not applicable, since they are deterministic. | Test procedure in all statistical tests ensures independency | 1 per operating band (if tested, see 3A.3) | Full set of environmental conditions (5) per operating band |
| All tests in clauses 4.7, 5.7, 6.7 and 7.7 are UE level reports of statistical nature | Independency is assumed, although Layer 1 filtering is applied to the reported results | Full set of environmental conditions (5) per operating band | Full set of environmental conditions (5) per operating band |

# G.X	Theory to derive the numbers in Table G.2.3-1 (informative)

TS 36.521-1 Annex G.X applies.

Annex H (normative):
Default message contents for RRM

# H.1

# H.2	System information blocks message content exceptions

## H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection

SystemInformationBlockType2: for NR intra-frequency cell re-selection

Table H.2.1-1: SIB2: NR intra frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -65 | -130 is actual value in dBm (-65 * 2 dBm) | SCS15 |
|  | -64 | -128 is actual value in dBm (-64 * 2 dBm) | SCS30 |
|  | -69 | -138 is actual value in dBm (-69 * 2 dBm) | SCS120 |
|  | -68 | -136 is actual value in dBm (-68 * 2 dBm) | SCS240 |
| s-IntraSearchP | 30 | 60 is actual value in dB (30 * 2 dB) |  |
| smtc SEQUENCE { |  |  |  |
| periodicityAndOffset | sf20 : 0 |  | SMTC.1, SMTC.2 |
|  | sf20 : 17 |  | SMTC.6 |
| duration | sf1 |  | SMTC.1 |
|  | sf5 |  | SMTC.2, SMTC.6 |
| } |  |  |  |
| deriveSSB-IndexFromCell | false |  | Asynchronous cells |
|  | true |  | Synchronous cells |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n according to TS 38.133 [6] A.3.11 |
| Synchronous cells | SSB indices of neigibour cells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neigibour cells can not be derived from timing of serving cell |

SystemInformationBlockType3: for NR intra-frequency cell re-selection

Table H.2.1-2: SIB3: NR intra frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB3 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE { |  |  |  |
| IntraFreqNeighCellInfo ::= SEQUENCE{ |  |  |  |
| physCellId | Set according to the neighbour cell PCI |  |  |
| q-OffsetCell | dB0 | 0 is actual value in dB (0 * 2 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

SystemInformationBlockType1: for NR intra frequency cell re-selection

Table H.2.1-3: SIB1: NR intra frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -65 | -130 is actual value in dBm (-65 * 2 dBm) | SCS15 |
|  |  | -64 | -128 is actual value in dBm (-68 * 2 +1 dBm) | SCS30 |
|  |  | -69 | -138 is actual value in dBm (-69 * 2 dBm) | SCS120 |
|  |  | -68 | -136 is actual value in dBm (-68 * 2 dBm) | SCS240 |
| } |  |  |  |  |
| servingCellConfigCommon SEQUENCE { |  |  |  |  |
| highSpeedConfig-r16 SEQUENCE { |  |  |  |  |
| highSpeedMeasFlag-r16 |  | true |  | HighSpeedMeas |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| HighSpeedMeas | highSpeedMeasFlag-r16 is configured. UE shall apply the enhanced measurement requirements to support high speed up to 500 km/h as specified in TS 38.133 [14] |

## H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection

SystemInformationBlockType2: for NR inter-frequency cell re-selection

Table H.2.2-1: SIB2: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| threshServingLowP | 22 | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  | NR Cell 1 |
|  | 5 |  | NR Cell 2 |
| } |  |  |  |
| } |  |  |  |

SystemInformationBlockType4: for NR inter-frequency cell re-selection

Table H.2.2-2: SIB4: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 Entry |  |  |
| dl-CarrierFreq[1] | Downlink NR SSB ARFCN of NR Cell 2 |  |  |
| smtc SEQUENCE { |  |  |  |
| periodicityAndOffset | sf20 : 0 |  | SMTC.1, SMTC.2 |
|  | sf20 : 17 |  | SMTC.6 |
| duration | sf1 |  | SMTC.1, SMTC.2 |
|  | sf5 |  | SMTC.6 |
| deriveSSB-IndexFromCell[1] | false |  | Asynchronous cells |
|  | true |  | Synchronous cells |
| q-RxLevMin[1] | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| threshX-HighP[1] | 24 | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-LowP[1] | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| cellReselectionPriority[1] | 5 |  |  |
| q-OffsetFreq[1] | dB0 | 0 is actual value in dB (0 * 2 dB). |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB.n FRm | SSB pattern n in FRm according to TS 38.133 [6] A.3.10 |
| Synchronous cells | SSB indices of neighbour cells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neighbour cells cannot be derived from timing of serving cell |

SystemInformationBlockType1: for NR inter frequency cell re-selection

Table H.2.2-3: SIB1: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  |  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| } |  |  |  |  |
| } |  |  |  |  |

## H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection

SystemInformationBlockType1: for inter-RAT NR – E-UTRA cell re-selection

Table H.2.3-1: SIB1: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  |  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| } |  |  |  |  |
| } |  |  |  |  |

SystemInformationBlockType2: for inter-RAT NR – E-UTRA cell re-selection

For NR Cell 1

Table H.2.3-2: SIB2: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| threshServingLowP | 22 | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  | higher priority |
|  | 5 |  | lower priority |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| smtc SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  |  |
| } |  |  |  |
| duration | sf1 |  | SMTC.1 |
|  | sf5 |  | SMTC.2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.n | SMTC pattern n according to TS 38.133 [6] A.3.11 |
| higher priority | NR cell re-selection to higher priority E-UTRA |
| lower priority | NR cell re-selection to lower priority E-UTRA |

SystemInformationBlockType5: for inter-RAT NR – E-UTRA cell re-selection

For E-UTRA Cell 1

Table H.2.3-3: SIB5: Inter-RAT NR – E-UTRA cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB5 ::= SEQUENCE { |  |  |  |
| carrierFreqListEUTRA SEQUENCE (SIZE (1..maxEUTRA-Carrier)) OF SEQUENCE { | 1 Entry |  |  |
| carrierFreq[1] | Downlink EUTRA ARFCN of E-UTRA Cell 1 |  |  |
| eutra-FreqNeighCellList[1] SEQUENCE (SIZE (1..maxCellEUTRA)) OF SEQUENCE { |  |  |  |
| physCellId | E-UTRA physical cell identity |  |  |
| dummy | dB0 |  |  |
| } |  |  |  |
| presenceAntennaPort1[1] | FALSE |  |  |
| cellReselectionPriority[1] | 5 |  | higher priority |
|  | 4 |  | lower priority |
| threshX-High | 24 | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-Low | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| q-RxLevMin | -70 | -140 is actual value in dBm (-70 * 2 dBm) |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| higher priority | NR cell re-selection to higher priority E-UTRA |
| lower priority | NR cell re-selection to lower priority E-UTRA |

# H.3	RRC message content exceptions

## H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

RRCReconfiguration

To setup NR Measurement Configuration.

Table H.3.1-1: RRCReconfiguration: NR measurement Configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig-DEFAULT | Measurements configuration |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

MeasConfig-DEFAULT

Configuration for NR measurement.

Table H.3.1-2: MeasConfig-DEFAULT: Configuration of NR measurement

| Derivation path: 38.508-1 [14] table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTRA-FREQ MO |  |  |
| } |  |  |  |
| measObjectId[2] | 2 |  | INTER-FREQ OR INTER-RAT |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTER-FREQ MO |  | INTER-FREQ |
| measObjectNR | MeasObjectNR-DEFAULT with Condition Deactivated SCell |  | Deactivated SCell |
| measObjectEUTRA | MeasObjectEUTRA-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | ReportConfigId |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | MeasId |  |  |
| measObjectId[1] | 1 |  |  |
|  | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| reportConfigId[1] | ReportConfigId |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
|  | QuantityConfig | TS 38.508-1 Table 4.6.3-127 | L3 FILTERING NEEDED |
| measGapConfig | MeasGapConfig-DEFAULT |  | GAP NEEDED |
| } |  |  |  |

| Condition | Explanation |  |
| --- | --- | --- |
| GAP NEEDED | Measurement gap on the NR Cell is needed for measurement |  |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |  |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |  |
| Deactivated SCell |  | Configuration for measurement on deactivated SCell tests |
| L3 FILTERING NEEDED |  | L3 filtering is needed for measurement |

MeasObjectNR-DEFAULT

NR measurement object configuration.

Table H.3.1-3: MeasObjectNR-DEFAULT: NR intra-frequency measurement object configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of the SSB associated to serving cell | frequency of the serving cell | INTRA-FREQ MO |
|  | ARFCN-ValueNR of the SSB associated to SCell |  | Deactivated SCell |
|  | ARFCN-ValueNR of the SSB associated to inter-frequency neighbour cell |  | INTER-FREQ MO |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| deriveSSB-IndexFromCell | false |  | Asynchronous cells |
|  | true |  | Synchronous cells OR RLM |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | 0 | SS-RSRP < -156dB | NOT SS-SINR |
| thresholdSINR | 0 | SS-SINR<-23 | SS-SINR |
| } |  |  |  |
| measCycleSCell-v1530 | sf640 |  | Deactivated SCell |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTRA-FREQ MO | Configuration for NR MO associated to intra-frequency carrier |
| INTER-FREQ MO | Configuration for NR MO associated to inter-frequency carrier |
| Synchronous cells | SSB indices of neighbourcells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neighbourcells cannot be derived from timing of serving cell |
| RLM | Configuration for RLM tests |
| Deactivated SCell | Configuration for SCell activation and deactivation tests |
| SS-SINR | Configuration for SS-SINR tests |

MeasObjectEUTRA-DEFAULT

EUTRA measurement object configuration for NR FR1 to E-UTRAN handover.

Table H.3.1-3A: MeasObjectEUTRA-DEFAULT: InterRAT EUTRA measurement object configuration for FR1 to E-UTRAN handover

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-74 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA ::= SEQUENCE { |  |  |  |
| cellsToAddModListEUTRAN SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF SEQUENCE{ |  |  |  |
| cellIndexEUTRA | 1 |  |  |
| physCellId | Physical Cell ID of the E-UTRA cell | Annex E |  |
| cellIndividualOffset | dB0 |  |  |
| } |  |  |  |
| eutra-PresenceAntennaPort1 | false |  |  |
| } |  |  |  |

ReportConfigNR-DEFAULT

NR Report Configuration

Table H.3.1-4: ReportConfigNR-DEFAULT(A3-offset, a4-threshold): NR report configuration for event A3 or event A4

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition EVENT_A3 or EVENT_A4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | A3-offset*2 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| eventA4 SEQUENCE { |  |  |  |
| a4-Threshold CHOICE { |  |  |  |
| rsrp | a4-threshold+156 | For thresholds, the actual value is (IE value – 156) dBm |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| reportAmount | r2 |  |  |
| reportQuantity-RsIndexes SEQUENCE { |  |  | SSB Index |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| maxNrofRS-IndexesToReport | 2 |  | SSB Index |
| includeBeamMeasurements | false |  | SSB Index |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |

ReportConfigInterRAT-DEFAULT

InterRAT NR Report Configuration for NR FR1 to E-UTRAN handover.

Table H.3.1-4A: ReportConfigInterRAT- DEFAULT (b2-Thres1, b2-Thres2): InterRAT NR report configuration for FR1 to E-UTRAN handover with b2-Threshold1 = b2-Thres1 and b2-Threshold2EUTRA  = b2-Thres2 dBm

| Derivation Path: 38.508-1 [4] Table 4.6.3-141 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | b2-Thres1 | b2-Thres1 – 156 dBm |  |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | b2-Thres2 | b2-Thres2 – 156 dBm |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

QuantityConfig-DEFAULT

NR quantity configuration when L3 filtering is not used.

Table H.3.1-5: QuantityConfig-DEFAULT: NR quantity configuration when L3 filtering is not used

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-127 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig::= SEQUENCE { |  |  |  |
| quantityConfigNR-List SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF SEQUENCE { | 1 entry |  |  |
| quantityConfigCell[1] SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigEUTRA | Not present |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  | INTER-RAT |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| filterCoefficientRS-SINR | fc0 |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |

MeasGapConfig-DEFAULT

Measurement gap configuration.

Table H.3.1-6: MeasGapConfig: measurement gap configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-70 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 CHOICE { |  |  | gapFR2 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #13 |
| mgl | ms5dot5 |  | Pattern #13 |
| mgrp | ms40 |  | Pattern #13 |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapFR1 CHOICE { |  |  | gapFR1 |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #2 |
|  | 19 |  | Pattern #4 |
| mgl | ms3 |  | Pattern #2 |
|  | ms6 |  | Pattern #4 |
| mgrp | ms40 |  | Pattern #2 |
|  | ms20 |  | Pattern #4 |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| gapUE CHOICE { |  |  | gapUE |
| setup SEQUENCE { |  |  |  |
| gapOffset | 39 |  | Pattern #0 OR Pattern #2 OR Pattern #13 |
|  | 0 |  | RLM OR BFD |
| mgl | ms6 |  | Pattern #0 OR RLM OR BFD |
|  | ms3 |  | Pattern #2 |
|  | ms5dot5 |  | Pattern #13 |
| mgrp | ms40 |  | Pattern #0 OR Pattern #2 OR Pattern #13 OR RLM OR BFD |
| mgta | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Pattern #0 | Measurement gap pattern #0 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #2 | Measurement gap pattern #2 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #4 | Measurement gap pattern #4 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| Pattern #13 | Measurement gap pattern #13 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |
| RLM | Measurement gap pattern for RLM tests |
| BFD | Measurement gap pattern for Beam Failure Dectection tests |
| gapFR2 | Indicates measurement gap configuration that applies to FR2 only. gapFR2 cannot be configured together with gapUE. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC |
| gapFR1 | Indicates measurement gap configuration that applies to FR1 only. gapFR1 cannot be configured together with gapUE. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC |
| gapUE | Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. In (NG)EN-DC, gapUE cannot be set up by NR RRC. |

MeasResults-DEFAULT

measurement result for NR measurements.

Table H.3.1-7: MeasResults: measurement result for NR measurements

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 with condition A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |  |
| measId |  | MeasId |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF SEQUENCE { |  | 2 entries |  |  |
| servCellId[1] |  | ServCellIndex of NR SpCell |  |  |
| measResultServingCell[1] SEQUENCE { |  |  |  |  |
| physCellId |  | PhysCellId of NR SpCell |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| servCellId[2] |  | ServCellIndex of NR SCell |  | Deactivated SCell |
| measResultServingCell[2] SEQUENCE { |  |  |  | Deactivated SCell |
| physCellId |  | PhysCellId of NR SCell |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  | INTRA-FREQ OR INTER-FREQ |
| physCellId[1] |  | PhysCellId of NR neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  | n entires of ResultsPerSSB-Index | ResultsPerSSB-IndexList | SSB Index |
| resultsPerSSB-Index SEQUENCE { |  | entry [1] |  |  |
| ssb-Index |  | SSB-Index | an SS-Block within an SS-Burst |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  | INTER-RAT |
| eutra-PhysCellId [1] |  | PhysCellId of E-UTRA neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| rsrp |  | (0..97) |  |  |
| rsrq |  | (0..34) |  |  |
| } |  |  |  |  |
| cgi-Info |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |
| INTRA-FREQ | Configuration for intra-frequency NR measurement tests |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |
| Deactivated SCell | Configuration for measurement on Deactivated SCell tests |

RadioLinkMonitoringConfig -DEFAULT

Default configuration for RLM resources.

Table H.3.1-8: RadioLinkMonitoringConfig-DEFAULT: Default configuration for RLM and BFD resources

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | n entries | n = 1 for FR1, n = 2 for FR2 | SSB RLM OR CSI-RS RLM |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  | FR2 |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId for TRS(4) with condition SECOND_SET | TS 38.508-1[14], table 7.3.1-7C | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF { | 2 entries |  | SSB BFD OR CSI-RS BFD |
| RadioLinkMonitoringRS[1] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 0 |  |  |
| purpose | both |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 | Index of SSB #0 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(0) | TS 38.508-1[14], Table 7.3.1-7D | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| RadioLinkMonitoringRS[2] SEQUENCE { |  |  |  |
| radioLinkMonitoringRS-Id | 1 |  |  |
| purpose | rlf |  |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 1 | Index of SSB #1 | SSB BFD |
| csi-RS-Index | NZP-CSI-RS-ResourceId for BM(1) | TS 38.508-1[14], Table 7.3.1-7D | CSI-RS BFD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| beamFailureInstanceMaxCount | n1 |  | SSB BFD OR CSI-RS BFD |
| beamFailureDetectionTimer | pbfd4 |  | SSB BFD OR CSI-RS BFD |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Used when SSB based RLM is configured in test case |
| CSI-RS RLM | Used when CSI-RS based RLM is configured in test case |
| SSB BFD | Used when SSB based BFD is configured in test case |
| CSI-RS BFD | Used when CSI-RS based BFD is configured in test case |

RLF-TimersAndConstants-DEFAULT

Default parameters for RLM related timers and counters.

Table H.3.1-9: RLF-TimersAndConstants-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-150 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLF-TimersAndConstants ::= SEQUENCE { |  |  |  |
| t310 | ms0 |  |  |
| } |  |  |  |

BeamFailureRecoveryConfig-DEFAULT

Default configuration for CBD and contention-free RACH in link recovery.

Table H.3.1-10: BeamFailureRecoveryConfig-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BeamFailureRecoveryConfig ::= SEQUENCE { |  |  |  |
| rootSequenceIndex-BFR | 0 |  |  |
| rach-ConfigBFR | RACH-ConfigGeneric-DEFAULT |  | SSB |
|  | RACH-ConfigGeneric-DEFAULT with condition CSI-RS |  | CSI-RS |
| rsrp-ThresholdSSB | 51 | -105dBm |  |
| candidateBeamRSList SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF CHOICE { | 1 entry |  |  |
| ssb[1] SEQUENCE { |  |  | SSB |
| ssb | 1 |  |  |
| ra-PreambleIndex | 50 |  |  |
| } |  |  |  |
| csi-RS[1] SEQUENCE { |  |  | CSI-RS |
| csi-RS | NZP-CSI-RS-ResourceId of the CSI-RS resource used for CBD |  |  |
| ra-OccasionList | 1 |  |  |
| ra-PreambleIndex | 50 |  |  |
| } |  |  |  |
| } |  |  |  |
| ssb-perRACH-Occasion | oneFourth |  |  |
| ra-ssb-OccasionMaskIndex | Not present |  |  |
| recoverySearchSpaceId | SearchSpaceId of the search space used for BFD RAR in DL active BWP |  |  |
| ra-Prioritization | Not present |  |  |
| beamFailureRecoveryTimer | Not present |  |  |
| msg1-SubcarrierSpacing-v1530 | kHz120 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CBD |
| CSI-RS | Configuration for CSI-RS based CBD |

BWP-UplinkDedicated

Default BWP configuration for Beam Failure Recovery

Table H.3.1-10A: BWP-UplinkDedicated

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkDedicated ::= SEQUENCE { |  |  |  |
| beamFailureRecoveryConfig | BeamFailureRecoveryConfig-DEFAULT |  |  |
| } |  |  |  |

RACH-ConfigGeneric-DEFAULT

Default generic configuration for contention-free RACH in link recovery.

Table H.3.1-11: RACH-ConfigGeneric-DEFAULT

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| ra-ResponseWindow | sl40 |  | FR2 AND CSI-RS |
|  | sl1 |  | FR1 AND CSI-RS |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSI-RS | Configuration for CSI-RS based CBD |

## H.3.2	RRC messages and information elements contents exceptions for NR cell re-selection and handoverRACH-ConfigGeneric: for NR cell re-selection and handover

Table H.3.2-1: Void

Table H.3.2-2: RRCReconfiguration-HO

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB1 and SRB2 and DRBn and Re-establish_PDCP |  | RBConfig_KeyChange |
|  | RadioBearerConfig with conditions DRBn and Recover_PDCP |  | RBConfig_NoKeyChange |
| secondaryCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with conditions PCell_change | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RBConfig_KeyChange | RadioBearerConfig to perform Intra-NR handover with security key change |
| RBConfig _NoKeyChange | RadioBearerConfig to perform Intra-NR handover without security key change |

## H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover

MobilityFromNRCommand

For Inter-RAT NR handover.

Table H.3.3-1: MobilityFromNRCommand: InterRAT NR handover

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromNRCommand::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| mobilityFromNRCommand SEQUENCE { |  |  |  |  |
| targetRAT-Type |  | eutra |  |  |
| targetRAT-MessageContainer |  | OCTET STRING including the RRCConnectionReconfiguration message according TS 36.508 [2], table 4.6.1-8 with condition HO-TO-EUTRA |  |  |
| nas-SecurityParamFromNR |  | The 4 LSB of the downlink NAS COUNT |  |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

MobilityFromEUTRACommand

Table H.3.3-2: MobilityFromEUTRACommand: inter-RAT handover to NR Cell

| Derivation Path:TS 36.508 [25] Table 4.6.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MobilityFromEUTRACommand ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| mobilityFromEUTRACommand-r9 SEQUENCE { |  |  |  |  |
| purpose CHOICE { |  |  |  |  |
| handover SEQUENCE { |  |  |  |  |
| targetRAT-Type |  | nr |  |  |
| targetRAT-MessageContainer |  | OCTET STRING containing RRCReconfiguration according to TS 38.508-1 [14] with Condition NR |  |  |
| nas-SecurityParamFromEUTRA |  | Not present |  |  |
| systemInformation |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

## H.3.4	E-UTRA RRC messages and information elements contents exceptions for NR measurement configuration

RRCConnectionReconfiguration

Includes the nr-SecondaryCellGroupConfig-r15 to convey NR RRCReconfiguration message as specified in TS 38.331 [13].

Table H.3.4-1: RRCConnectionReconfiguration: NR RRC Reconfiguration in EN-DC

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MCG_and_SCG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | RRCReconfiguration |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table H.3.4-1a: RRCConnectionReconfiguration for measurement configuration

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT | Table H.3.4-4 |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| nonCriticalExtensions::= SEQUENCE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| nr-SecondaryCellGroupConfig-r15 | RRCReconfiguration |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

ULInformationTransferMRDC

uplink transfer of MR DC information for transferring the NR RRC Measurement Report message in EN-DC.

Table H.3.4-2: ULInformationTransferMRDC: uplink transfer of MR DC information

| Derivation Path: 36.508 [25], Table 4.6.1-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULInformationTransferMRDC ::= SEQUENCE { |  |  |  |
| ul-DCCH-MessageNR-r15 | OCTET STRING including the MeasurementReport |  |  |
| } |  |  |  |

RRCConnectionReconfigurationComplete

Includes the scg-ConfigResponseNR to convey NR RRCReconfigurationComplete message as specified in TS 38.331 [13].

Table H.3.4-3: RRCConnectionReconfigurationComplete: NR RRC Reconfiguration Complete in EN-DC

| Derivation Path: 36.508 [25], Table 4.6.1-9: with condition MCG_and_SCG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReconfigurationComplete-r8 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| scg-ConfigResponseNR-r15 | OCTET STRING including the RRCReconfigurationComplete message according TS 38.508-1, table 4.6.1-14. |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

MeasConfig-DEFAULT

Configures measurement gap that applies to FR1 only in EN-DC as specified in TS 38.331 [13].

Table H.3.4-4: MeasConfig-DEFAULT

| Derivation Path: 36.508 [25],Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | INTER-RAT NR |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-GENERIC(Freq) | Freq is the frequency of E-UTRA PCell |  |
| } |  |  |  |
| measObjectId[2] | 2 |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR-r15 | MeasObjectNR | Table 4.6.6-2B |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  | INTER-RAT NR |
| reportConfigId[1] | 1 |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-B2-NR(EUTRA-Thres, NR-Thres) | Set EUTRA-Thres and NR-Thres according to test parameters specified in test cases | EVENT B2 |
|  | ReportConfigInterRAT-B1-NR(EUTRA-Thres, NR-Thres) | Set EUTRA-Thres and NR-Thres according to test parameters specified in test cases | EVENT B1 |
|  | ReportConfigEUTRA-A4(Thres) | Set Thres according to test parameters specified in test cases | EVENT A4 |
|  | ReportConfigInterRAT-SFTD with Condition INTER-RAT |  | SFTD |
|  | ReportConfigInterRAT-PERIODICAL | Table 4.6.6-9 in TS 36.508 [25] | PERIODICAL |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 1 entry |  | INTER-RAT NR |
| measId[1] | 1 |  |  |
| measObjectId[1] | 2 |  |  |
| reportConfigId[1] | 1 |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-FR1 | Table H.3.4-5 |  |
|  | Not present |  | GAPLESS |
| fr1-Gap-r15 | false |  | gapUE |
| fr1-Gap-r15 | true |  | gapFR1 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| gapUE | fr1-Gap-r15 set to false indicates the gap is applicable for measurements on FR1 and FR2. E-UTRAN includes this field only when the UE is configured with (NG)EN-DC. |
| gapFR1 | fr1-Gap-r15 set to true indicates the gap is only applicable for measurements on FR1. E-UTRAN includes this field only when the UE is configured with (NG)EN-DC. |
| INTER-RAT NR | Measurement configuration for inter-RAT NR measurements |
| EVENT B2 | For event B2 triggered measurement reporting test cases |
| EVENT B1 | For event B1 triggered measurement reporting test cases |
| EVENT A4 | For event A4 triggered measurement reporting |
| SFTD | For inter-RAT SFTD measurement test cases |
| GAPLESS | MG is not needed in test |
| PERIODICAL | For periodic measurement reporting test cases |

MeasGapConfig-FR1

Configures FR1 measurement gap by LTE RRC in EN-DC as specified in TS 38.331 [13].

Table H.3.4-5: MeasGapConfig-FR1

| Derivation Path: 36.508 [25],Table 4.6.6-1A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 39 | TGRP = 40 ms | Pattern #0 |
|  | 0 |  | BFD |
| gp4-r15 | 19 | TGRP = 20 ms | Pattern #4 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Pattern #0 | gp0 corresponds to gap offset of Gap Pattern Id "0" with MGRP = 40ms |
| Pattern #4 | gp4-r15 corresponds to gap offset of Gap Pattern Id "4" with MGRP = 40ms (see TS 38.133, Table 9.1.2-1). It can be applied for (NG)EN-DC, see TS 38.133, Table 9.1.2-2. |
| BFD | Measurement gap pattern for Beam Failure Dectection tests |

Table H.3.4-6: MeasObjectNR

| Derivation Path: TS 36.508 [25] Table 4.6.6-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| rs-ConfigSSB-r15 ::= SEQUENCE { |  |  |  |
| measTimingConfig-r15 ::= SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20-r15 | 0 |  | SMTC.1 or SMTC.2 |
| sf20-r15 | 10 |  | SMTC.4 or SMTC.5 |
| Sf160-r15 | 0 |  | SMTC.3 |
| } |  |  |  |
| ssb-Duration-r15 | sf1 |  | SMTC.1, SMTC.3 or SMTC.4 |
|  | sf5 |  | SMTC.2 or SMTC.5 |
| } |  |  |  |
| subcarrierSpacingSSB-r15 | kHz15 |  | SSB.1 FR1, SSB.3 FR1 or SSB.5 FR1 |
|  | kHz30 |  | SSB.2 FR1, SSB.4 FR1 or SSB.6 FR1 |
|  | kHz120 |  | SSB.1 FR2, SSB.3 FR2, SSB.5 FR2 or SSB.7 FR2 |
|  | KHz240 |  | SSB.2 FR2, SSB.4 FR2, SSB.6 FR2 or SSB.8 FR2 |
| } |  |  |  |
| deriveSSB-IndexFromCell-r15 | true |  | Synchronous cells |
|  | false |  | Asynchronous cells |
| } |  |  |  |

Table H.3.4-7: RRCConnectionReconfiguration for iRAT measurement configuration

| Derivation Path: 36.508 [25], Table 4.6.1-8 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 ::= SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT | Table H.3.4-4 with Conditions INTER RAT NR and PERIODICAL |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.5	RRC messages and information elements contents exceptions for NR radio link monitoring (RLM)

CSI-RS information elements contents exception for NR RLM SSB-Based test cases

Table H.3.5-1: Void

Table H.3.5-2: Void

Table H.3.5-3: Void

Table H.3.5-4: CSI-ReportConfig

| Derivation Path: TS 38.508[14] Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  |  |
| reportSlotConfig ::= CHOICE { |  |  |  |
| slots5 | 0 |  | SCS15kHz_FDD |
| slots5 | 2 |  | SCS15kHz_TDD |
| slots10 | 4 |  | SCS30kHz |
| } |  |  |  |
| pucch-CSI-ResourceList SEQUENCE (SIZE (1..maxNrofBWPs)) OF{ |  |  |  |
| PUCCH_CSI_Resource[0] SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id |  |  |
| pucch_Resource | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS15kHz_FDD | SCS 15 kHz test configuration for FDD configuration |
| SCS15kHz_TDD | SCS 15 kHz test configuration for TDD configuration |
| SCS30kHz | SCS 30 kHz test configuration |

Table H.3.5-5: Void

Table H.3.5-6: Void

Table H.3.5-7: Void

BWP-DownlinkDedicated: to setup radioLinkMonitoringConfig Configuration

Table H.3.5-8: Void

RadioLinkMonitoringConfig: Configuration for RLM RS

Table H.3.5-9: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| radioLinkMonitoringRS-Id[1] | 0 |  |  |
| purpose[1] | rlf |  |  |
| detectionResource[1] CHOICE { |  |  |  |
| ssb-Index | 0 |  | SSB RLM |
| csi-RS-Index | NZP-CSI-RS-ResourceId of the RLM-RSspecified in TC |  | CSI-RS RLM |
| } |  |  |  |
| } |  |  |  |
| failureDetectionResourcesToReleaseList | Not present |  |  |
| beamFailureInstanceMaxCount | Not present |  |  |
| beamFailureDetectionTimer | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB RLM | Configuration for SSB based RLM test cases |
| CSI-RS RLM | Configuration for CSI-RS based RLM test cases |

Table H.3.5-10: Void

## H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting

ServingCellConfig: Default generic configuration for enabling CSI measurements and reporting

Table H.3.6-1: Void

CSI-ReportConfig: Default generic configuration for L1-RSRP measurements

Table H.3.6-2: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [14], Table 7,3,1-12G |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| csi-IM-ResourcesForInterference | Not present |  |  |
| nzp-CSI-RS-ResourcesForInterference | Not present |  |  |
| reportConfigType CHOICE { |  |  |  |
| periodic SEQUENCE { |  |  | PERIODIC |
| reportSlotConfig CHOICE { |  |  |  |
| slot80 | 7 | Periodicity 80 slots and offset 7 | SCS30 |
|  | 2 | Periodicity 80 slots and offset 2 | SCS15 |
|  | 4 | Periodicity 80 slots and offset 4 | FR2 |
| pucch-CSI-ResourceList SEQUENCE { | 1 entry |  |  |
| (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| uplinkBandwidthPartId | BWP-Id of active BWP |  |  |
| pucch-Resource | 8 | PUCCH-format2 as configured in TS 38.508-1 [14], Table 4.6.3-112 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity CHOICE { |  |  |  |
| ssb-Index-RSRP | NULL |  | SS-RSRP |
| cri-RSRP | NULL |  | CSI-RSRP |
| } |  |  |  |
| timeRestrictionForChannelMeasurements | configured |  |  |
| groupBasedBeamReporting CHOICE { |  |  |  |
| disabled SEQUENCE { |  |  |  |
| nrofReportedRS | n2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| PERIODIC | Configuration for periodic reporting |
| SS-RSRP | L1-RSRP measurement based on SSB |
| CSI-RSRP | L1-RSRP measurement based on CSI-RS |

CSI-ResourceConfig: Default generic resource configuration for L1-RSRP measurements

Table H.3.6-3: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 7.3.1-12B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| csi-RS-ResourceSetList CHOICE { |  |  |  |
| nzp-CSI-RS-SSB SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceSetList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF { | 1 entry |  | CSI-RS |
| NZP-CSI-RS-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| csi-SSB-ResourceSetList SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF { | 1 entry |  | SSB |
| CSI-SSB-ResourceSetId[0] | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB | Configuration for SSB based CSI |
| CSI-RS | Configuration for CSI-RS based CSI |

Table H.3.6-4: Void

Table H.3.6-5: Void

Table H.3.6-6: Void

Table H.3.6-7: Void

Table H.3.6-8: Void

Table H.3.6-9: Void

## H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used

MAC-CellGroupConfig: DRX configuration for NR serving cell

Table H.3.7-1: MAC-CellGroupConfig: NR intra-frequency cell search when DRX is used

| Derivation Path: TS 38.508-1, Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| drx-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms1 |  | DRX.1 OR DRX.2 OR DRX.6 |
|  | ms6 |  | DRX.3 OR DRX.7 OR DRX.8 OR DRX.11 |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms20 | 0 |  | DRX.11 |
| ms40 | 0 |  | DRX.1 or DRX.3 |
|  | 10 |  | DRX.1 and Gap |
|  | 15 |  | DRX.1 and Gap and INTER-FREQ |
|  | 20 |  | DRX.3 and Gap |
| ms320 | 0 |  | DRX.6 OR DRX.8 |
| ms640 | 0 |  | DRX.2 OR DRX.7 |
|  | 10 |  | (DRX.2 OR DRX.7) and Gap |
|  | 15 |  | (DRX.2 OR DRX.7) and Gap and INTER-FREQ |
| } |  |  |  |
| shortDRX | not present |  |  |
| drx-SlotOffset | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| tag-Config SEQUENCE { |  |  |  |
| tag-ToReleaseList | Not present |  |  |
| tag-ToAddModList SEQUENCE (SIZE (1..maxNrofTAGs)) OF SEQUENCE { | 1 entry |  |  |
| tag-Id[1] | 0 |  |  |
| timeAlignmentTimer[1] | ms500 |  | DRX.1 OR DRX.2 OR DRX.6 |
|  | infinity |  | DRX.3 OR DRX.7 OR DRX.8 OR DRX.11 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX.1 | DRX Configuration 1 according to TS 38.133 [6] A.3.3.1 |
| DRX.2 | DRX Configuration 2 according to TS 38.133 [6] A.3.3.2 |
| DRX.3 | DRX Configuration 3 according to TS 38.133 [6] A.3.3.3 |
| DRX.6 | DRX Configuration 6 according to TS 38.133 [6] A.3.3.6 |
| DRX.7 | DRX Configuration 7 according to TS 38.133 [6] A.3.3.7 |
| DRX.8 | DRX Configuration 8 according to TS 38.133 [6] A.3.3.8 |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| Gap | Configuration for tests with measurement gap |

MAC-MainConfig: DRX configuration for E-UTRAN serving cell

Table H.3.7-2: MAC-MainConfig

| Derivation Path: 36.508, Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig ::= SEQUENCE { |  |  |  |  |
| drx-Config CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf2 |  | DRX.4 OR DRX.9 |
|  |  | psf6 |  | DRX.5 OR DRX.10 |
| drx-InactivityTimer |  | psf2 |  | DRX.4 |
|  |  | psf1920 |  | DRX.5 OR DRX.10 |
|  |  | psf100 |  | DRX.9 |
| longDRX-CycleStartOffset CHOICE { |  |  |  |  |
| sf160 |  | 0 |  | DRX.4 |
| sf320 |  | 0 |  | DRX.5 |
| sf40 |  | 0 |  | DRX.9 |
| sf640 |  | 0 |  | DRX.10 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  | DRX.4 OR DRX.5 |
|  |  | sf500 |  | DRX.9 OR DRX.10 |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| DRX.4 | DRX Configuration 4 according to TS 38.133 [6] A.3.3.4 |
| DRX.5 | DRX Configuration 5 according to TS 38.133 [6] A.3.3.5 |
| DRX.9 | DRX Configuration 9 according to TS 38.133 [6] A.3.3.9 |
| DRX.10 | DRX Configuration 10 according to TS 38.133 [6] A.3.3.10 |

## H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay

ServingCellConfigCommonSIB: information elements content exception for RRC reconfiguration delay test cases to specific both NR uplink and supplementary uplink for SCell.

Table H.3.8-1: ServingCellConfigCommonSIB-Procedure Delay

| Derivation Path: TS 38.508-1, table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB | NR uplink |  |
|  | UplinkConfigCommonSIB with condition SUL_NUL |  | SUL |
| supplementaryUplink | UplinkConfigCommonSIB with condition SUL_SUL |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid; |  |
| groupPresence | Not present |  |  |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | FR1_TDD |
| ss-PBCH-BlockPower | 0 |  |  |
| } |  |  |  |

RRCReconfiguration: information elements contents exception for RRC reconfiguration delay test cases to specific radio bearers and cell groups.

Table H.3.8-2: RRCReconfiguration-Procedure Delay

| Derivation Path: TS 38.508-1, table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions SRB2 and DRB1 |  |  |
| } |  |  |  |
| secondaryCellGroup | CellGroupConfig with condition EN-DC_SCell_add | OCTET STRING (CONTAINING CellGroupConfig) |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig with conditions EN-DC | OCTET STRING (CONTAINING CellGroupConfig) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## H.3.9	RRC messages and information elements contents exceptions for UL timing

Table H.3.9-1: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1, Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| startSymbolAndLength | 41 | Start symbol(S)=0, Length(L)=13 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| startSymbolAndLength | 41 | Start symbol(S)=0, Length(L)=13 |  |
| } |  |  |  |
| } |  |  |  |

Annex I (normative):
RRM OTA procedures

Annex I defines the RRM OTA procedures for the permitted testing methodologies defined in [32].

# I.0	Test applicability per permitted test method

The applicability of each permitted test method for the different RRM Angle of Arrival (AoA) setups as defined in Section A.9 is defined in Table I.0-1.

Table I.0-1 AoA Test Setup applicability per permitted test method

| AoA Test Setup | D > 5cm or No declaration | D ≤ 5cm |
| --- | --- | --- |
| Setup 1 | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 2a | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 2b | IFF, Enhanced IFF | DFF, IFF, Enhanced IFF, IFF+DFF |
| Setup 3 | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| Setup 4a | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| Setup 4b | Enhanced IFF | DFF, Enhanced IFF, IFF+DFF |
| NOTE1: D =The diameter of the smallest sphere that encloses the radiating parts of the phase coherent array antenna(s) active at any one time during the test., declared by UE vendor as per Table 4.3.9-9 in TS 38.508-2 NOTE2: DFF indicates both DFF and DFF simplification in TR 38.810. NOTE3: For DFF and DFF part of IFF+DFF, minimum range length needs to meet the requirement as specified in 38.508-1 Annex B.2.2-4 with the declared D |  |  |

# I.1	Direct far field (DFF)

## I.1.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.1.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.1.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.2	Direct far field (DFF) simplification

## I.2.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.2.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.2.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.3	Indirect far field (IFF)

## I.3.1	RX beam peak direction search

The Rx beam peak direction search can be found using any of the following options:

-	Same measurement procedure as in clause Annex K.3.2 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

## I.3.2	Search for directions corresponding to the EIS spherical coverage percentile

The test directions corresponding to the EIS spherical coverage percentile can be found using any of the following options:

-	EIS spherical coverage scan procedure as described in Annex K.1.6 of TS 38.521-2 [18].

-	Any of the procedures described in Annex I.4.

# I.3A	Enhanced indirect far field (Enhanced IFF)

## I.3A.1	RX beam peak direction search

Same as in clause I.3.1 for IFF.

## I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile

Same as in clause I.3.2 for IFF.

# I.4	Procedures to search test directions for RRM FR2

## I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search

This section provides RSRPB-based scan to find the Rx beam peak of the UE for Setup 1, single AoA cases and one of the test directions for Setup 4. It also provides a method to find test directions fulfilling the EIS Spherical Coverage (TS 38.521-2 [18] clause 7.3.4.3) for Setup 2, 3 and 4. Other approaches are not precluded. RSRPB-based peak beam scan greatly reduces the time to find the peak beam position compared to traditional EIS scan using either constant step size or constant density. The procedure can be used with DFF as well as IFF methodology. The procedure is achieved implementing the following steps:

1.	Enable periodic RSRPB reporting from the UE

2.	Set of grid points for the UE scan can be user defined set or entire sphere.

3.	Set the DL SSB_RP (at the centre of the QZ) to -95 dBm / SCS. For each grid point, record RSRPB first by connecting SS to the DUT through the measurement antenna with PolLink=  polarization to form the Rx beam towards the measurement antenna.

4.	Wait for BEAM_SELECT_WAIT_TIME before recording the RSRPB reports.

4a	For each of the grid points, calculate the RSRPB result for PolLink= as the linear sum of the 2 RSRPB reports (one per receiver branch).

4b	Repeat Steps 3, 4 and 4a with PolLink= .

5.	Once the grid points scan is completed, sort the grid points based on the linear sum of the RSRPB results for PolLink= and PolLink= .

6.	For the top 10 grid points, run the REFSENS throughput test as per the test condition defined in TS 38.521-2 [18] clause 7.3.2

7.	Select the grid point with the best REFSENS result among the 10 points from Step 6 as the Rx beam peak for RRM test cases using AoA Setup 1 or 4.

.

8.	Use the EIS result for the RRM beam peak to offset the RSRPB results to obtain an estimate of the EIS Spherical coverage (SC) map.

9.	From the EIS map in Step 8, select all the points meeting the EIS SC criteria (as defined in TS 38.521-2 [18] Section 7.3.4.3) as potential test directions for AoA Setup 2a, 2b, 3 and 4. In order to confirm that the grid point is suitable, run the REFSENS throughput test as per the test condition defined in TS 38.521-2 [18] clause 7.3.2 and verify that the EIS SC criteria is met. .

If not enough grid points are found using this method, fallback to using the method in Annex I.1, I.2, I.3 or I.3A as applicable.

Annex J (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-08 | RAN5#76 | R5-175205 | - | - | - | Introduction of TS 38.533 | 0.0.1 |
| 2018-08 | RAN5#80 | R5-184115 | - | - | - | Added specification structure including headers up to third level | 0.0.2 |
| 2018-11 | RAN5#81 | R5-186706 | - | - | - | Added references and common sections | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187847 | - | - | - | Added RMCs, OCGN, SMTC and SSB configurations to Annex A | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187996 | - | - | - | Added test cases 6.7.1.1.1 to 6.7.1.2.2 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187997 | - | - | - | Added test cases 4.6.2.1 to 4.6.2.8 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187998 | - | - | - | Added test cases 5.6.2.1 to 5.6.2.4 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-187999 | - | - | - | Added test cases 6.6.2.1 to 6.6.2.8 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188000 | - | - | - | Added test cases 7.6.2.1 to 7.6.2.4 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188001 | - | - | - | Added test case 4.4.1.1 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188002 | - | - | - | Added test cases 4.7.1.1.1 to 4.7.1.2.2 | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188005 | - | - | - | Added Annexes B to H | 0.1.0 |
| 2018-11 | RAN5#81 | R5-188011 | - | - | - | Added test case 4.4.3.1 | 0.1.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190448 | - | - | - | Updating FR1 MU for timing measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190476 | - | - | - | Addition of band group power offsets | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190477 | - | - | - | Update of the annexes | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190478 | - | - | - | Changes to 4.7.1.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190479 | - | - | - | Addition of 4.7.2.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190480 | - | - | - | Addition of 4.7.4.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190481 | - | - | - | Changes to 6.7.1.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190482 | - | - | - | Addition of 6.7.2.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190483 | - | - | - | Addition of 6.7.4.x tests | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190512 | - | - | - | Addition of EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode test case 4.5.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190513 | - | - | - | Addition of EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode test case 4.5.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190514 | - | - | - | Addition of EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode test case 4.5.1.7 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190515 | - | - | - | Addition of EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode test case 4.5.1.8 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190516 | - | - | - | Addition of EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC test case 4.5.2.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190517 | - | - | - | Addition of EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC test case 4.5.2.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190518 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC test case 4.5.2.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190519 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC test case 4.5.2.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190520 | - | - | - | Addition of EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC test case 4.5.2.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190582 | - | - | - | Annex F correction | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190803 | - | - | - | Update Annex G in TS 38.533 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190874 | - | - | - | Addition of NR test case 6.6.1.1-reporting without gap non-DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190875 | - | - | - | Addition of NR test case 6.6.1.2-reporting without gap DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190876 | - | - | - | Addition of NR test case 6.6.1.3-with gap non DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190877 | - | - | - | Addition of NR test case 6.6.1.4-with gap DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190878 | - | - | - | Addition of NR test case 6.6.1.5-without gap non DRX SBI reading | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190879 | - | - | - | Addition of NR test case 6.6.1.6-with gap non DRX SBI reading | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190880 | - | - | - | CR to 38.533 annex for event triggered reporting test cases | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190881 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190882 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190883 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190884 | - | - | - | Addition of EN-DC FR1 event triggered reporting test case 4.6.1.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190885 | - | - | - | Addition of default config for event triggered test cases | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190886 | - | - | - | Introduction of 5G RRM TC 4.5.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190887 | - | - | - | Introduction of 5G RRM TC 4.5.3.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190888 | - | - | - | Introduction of 5G RRM TC 4.5.3.3 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190889 | - | - | - | Introduction of 5G RRM TC 4.6.1.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190890 | - | - | - | Introduction of 5G RRM TC 4.6.1.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190891 | - | - | - | Introduction of 5G RRM TC 5.5.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190892 | - | - | - | Introduction of 5G RRM TC 6.6.3.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190893 | - | - | - | pCR for Addition of TC 6.5.1.3 NR SA FR1 RLM OOS in DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190894 | - | - | - | pCR for Addition of TC 6.4.3.1 NR SA FR1 TAA Accuracy | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190895 | - | - | - | pCR for Modification of TC 4.4.3.1 EN-DC FR1 TAA accuracy | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190896 | - | - | - | pCR for Addition of TC 6.5.1.1 NR SA FR1 RLM OOS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190897 | - | - | - | pCR for Addition of TC 4.5.1.1 EN-DC FR1 RLM OOS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190898 | - | - | - | pCR for Addition of TC 4.5.1.3 EN-DC FR1 RLM OOS in DRX | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190899 | - | - | - | Update to EN-DC FR1 transmit timing accuracy test | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190900 | - | - | - | Addition of EN-DC FR1 RLM IS non-DRX test with SSB-based RLM RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190901 | - | - | - | Addition of EN-DC FR1 RLM IS DRX test with SSB-based RLM RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190902 | - | - | - | Addition of NR SA FR1 RLM IS non-DRX test with SSB-based RLM RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190903 | - | - | - | Addition of NR SA FR1 RLM IS DRX test with SSB-based RLM RS | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190904 | - | - | - | Correction of RRM 5G Test Cases 4.6.2 - EN-DC FR1-FR1 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190905 | - | - | - | Correction of RRM 5G Test Cases 6.6.2 - NR SA FR1-FR1 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190906 | - | - | - | Correction of RRM 5G Test Cases 7.6.2 - NR SA FR2-FR2 Inter-frequency measurements | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190907 | - | - | - | Addition of RRM Test Cases 4.5.2.6: EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190908 | - | - | - | Addition of RRM Test Cases 5.5.2.1: EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190909 | - | - | - | Addition of RRM Test Cases 5.5.2.2: EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190910 | - | - | - | Addition of RRM Test Cases 5.5.2.3: EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190911 | - | - | - | Addition of RRM Test Cases 5.5.2.4: EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190912 | - | - | - | Addition of RRM Test Cases 5.5.2.5: EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190913 | - | - | - | Addition of RRM Test Cases 5.5.2.6: EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190914 | - | - | - | Addition of RRM Test Cases 6.1.1.1: NR SA FR1 cell re-selection | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190915 | - | - | - | Addition of RRM Test Cases 6.1.1.2: NR SA FR1-FR1 cell re-selection | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190916 | - | - | - | Addition of cell re-selection to higher priority E-UTRAN test case 6.1.2.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190917 | - | - | - | Addition of cell re-selection to lower priority E-UTRAN test case 6.1.2.2 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190918 | - | - | - | Addition of SA NR to E-UTRAN handover test case 6.3.1.4 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190919 | - | - | - | Addition of SA NR to E-UTRAN handover test case 6.3.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190920 | - | - | - | Addition of NR SA FR1 UE UL carrier RRC reconfiguration delay test case 6.5.4.1 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190921 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM out-of-sync non-DRX test case 6.5.1.5 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190922 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM in-sync non-DRX test case 6.5.1.6 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190923 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM out-of-sync in DRX test case 6.5.1.7 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190924 | - | - | - | Addition of NR SA FR1 CSI-RS based RLM in-sync in DRX test case 6.5.1.8 | 0.2.0 |
| 2019-01 | RAN5#4 5G-NR AH | R5-190987 | - | - | - | 38.533 Common Section updates to clarify leverage across architecture options | 0.2.0 |
| 2019-03 | RAN5#82 | R5-191484 | - | - | - | Correction Annex G | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191485 | - | - | - | Correction NSA Options | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191486 | - | - | - | Modifications NSA FR1 SS-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191487 | - | - | - | Modifications NSA FR1 SS-RSRQ tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191488 | - | - | - | Modifications NSA FR1 L1-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191489 | - | - | - | Modifications SA FR1 SS-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191490 | - | - | - | Modifications SA FR1 SS-RSRQ tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191491 | - | - | - | Modifications SA FR1 L1-RSRP tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191492 | - | - | - | Addition NSA FR1 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191493 | - | - | - | Addition SA FR1 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191494 | - | - | - | Addition NSA FR2 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191495 | - | - | - | Addition SA FR2 BWP switch tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191720 | - | - | - | addition of cell mapping for BFD and measurement | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191924 | - | - | - | Correction of default message contents for RRM | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191926 | - | - | - | Addition of event-triggered reporting Test Cases to Cell configuration mapping in Annex E | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191930 | - | - | - | Correction of 5G RRM Test Case 4.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191931 | - | - | - | Correction of 5G RRM Test Case 4.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191934 | - | - | - | Correction of 5G RRM Test Case 4.6.2.7 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191935 | - | - | - | Correction of 5G RRM Test Case 4.6.2.8 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191936 | - | - | - | Addition of Minimum conformance requirements 5.6.2.0 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191937 | - | - | - | Correction of 5G RRM Test Case 5.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191938 | - | - | - | Correction of 5G RRM Test Case 5.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191939 | - | - | - | Correction of 5G RRM Test Case 5.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191940 | - | - | - | Correction of 5G RRM Test Case 5.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191945 | - | - | - | Addition of Minimum conformance requirements 7.6.2.0 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191946 | - | - | - | Correction of 5G RRM Test Case 7.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191947 | - | - | - | Correction of 5G RRM Test Case 7.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191948 | - | - | - | Correction of 5G RRM Test Case 7.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191949 | - | - | - | Correction of 5G RRM Test Case 7.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191950 | - | - | - | Correction of 5G RRM Test Case 7.6.2.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191951 | - | - | - | Correction of 5G RRM Test Case 7.6.2.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191952 | - | - | - | Correction of 5G RRM Test Case 7.6.2.7 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-191953 | - | - | - | Correction of 5G RRM Test Case 7.6.2.8 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192062 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192063 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192064 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192221 | - | - | - | Update on RRC_Connected generic procedure within RRM tests | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192477 | - | - | - | Introduction of FR1 EN-DC Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192478 | - | - | - | Introduction of FR1 EN-DC non-Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192479 | - | - | - | Introduction of FR1 standalone Contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192480 | - | - | - | Introduction of FR1 standalone Non-contention based random access Test case | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192481 | - | - | - | Updated to 5G RRM TC 4.6.1.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192482 | - | - | - | Updated to 5G RRM TC 4.6.1.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192483 | - | - | - | Addition of NR test case 6.7.1.3.1-absolute RSRP | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192484 | - | - | - | Addition of NR test case 6.7.1.3.2-relative RSRP | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192485 | - | - | - | Addition of NR test case 6.5.5.3 FR1 CSI-RS BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192486 | - | - | - | Addition of NR test case 6.5.5.4 FR1 CSI-RS BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192487 | - | - | - | Addition of NR test case 7.5.5.1 FR2 SSB BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192488 | - | - | - | Addition of NR test case 7.5.5.2 FR2 SSB BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192489 | - | - | - | Addition of NR test case 7.5.5.3 FR2 CSI-RS BFD nonDRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192490 | - | - | - | Addition of NR test case 7.5.5.4 FR2 CSI-RS BFD DRX | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192492 | - | - | - | Correction of 5G RRM Test Case 4.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192493 | - | - | - | Correction of 5G RRM Test Case 4.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192494 | - | - | - | Correction of 5G RRM Test Case 4.6.2.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192495 | - | - | - | Correction of 5G RRM Test Case 4.6.2.6 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192496 | - | - | - | Correction of 5G RRM Test Case 6.6.2.1 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192497 | - | - | - | Correction of 5G RRM Test Case 6.6.2.2 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192498 | - | - | - | Correction of 5G RRM Test Case 6.6.2.3 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192499 | - | - | - | Correction of 5G RRM Test Case 6.6.2.4 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192500 | - | - | - | Update of EN-DC FR1 event triggered reporting test case 4.6.1.5 | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192503 | - | - | - | Update to EN-DC FR1 transmit timing accuracy test | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192674 | - | - | - | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192675 | - | - | - | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192676 | - | - | - | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192677 | - | - | - | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM RS | 0.3.0 |
| 2019-03 | RAN5#82 | R5-192678 | - | - | - | pCR for modification of TAAA TC for EN-DC FR1 | 0.3.0 |
| 2019-03 | RAN#83 | RP-190161 | - | - | - | Presented to the RAN#83 plenary for 1-step approval | 1.0.0 |
| 2019-03 | RAN#83 | - | - | - | - | Upgraded to Rel-15 with small editorial changes | 15.0.0 |
| 2019-06 | RAN#84 | R5-193578 | 0100 | - | F | Update the test case number of TC 4.3.2.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193755 | 0102 | - | F | Update of Minimum conformance requirements 6.6.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193758 | 0104 | - | F | Update of EN-DC RLM in-sync in non-DRX test case 4.5.1.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193759 | 0105 | - | F | Update of EN-DC RLM out-of-sync in DRX test case 4.5.1.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193760 | 0106 | - | F | Update of EN-DC RLM in-sync in DRX test case 4.5.1.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193761 | 0107 | - | F | Addition of Minimum conformance requirements 6.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193762 | 0108 | - | F | Addition of NR RRM TC 6.3.2.1.1-Intra Freq RRC Re-establishment | 15.1.0 |
| 2019-06 | RAN#84 | R5-193763 | 0109 | - | F | Addition of FR1-FR1 re-establishment test case 6.3.2.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193764 | 0110 | - | F | Addition of FR1 RRC connection release with redirection test case 6.3.2.3.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193765 | 0111 | - | F | Addition of FR1-E-UTRA RRC connection release with redirection test case 6.3.2.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193766 | 0112 | - | F | Update of TC 6.5.1.5 SA FR1 CSI-RS RLM OOS non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193767 | 0113 | - | F | Update of TC 6.5.1.6 SA FR1 CSI-RS RLM IS non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193768 | 0114 | - | F | Update of TC 6.5.1.7 SA FR1 CSI-RS RLM OOS DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193769 | 0115 | - | F | Update of TC 6.5.1.8 SA FR1 CSI-RS RLM IS DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193770 | 0116 | - | F | Addition of Minimum conformance requirements 6.5.5.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193771 | 0117 | - | F | Update of TC 6.5.5.3 SA FR1 CSI-RS BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193772 | 0118 | - | F | Update of TC 6.5.5.4 SA FR1 CSI-RS BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193773 | 0119 | - | F | Addition of Minimum conformance requirements 7.5.5.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193774 | 0120 | - | F | Update of TC 7.5.5.1 SA FR2 SSB BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193775 | 0121 | - | F | Update of TC 7.5.5.2 SA FR2 SSB BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193776 | 0122 | - | F | Update of TC 7.5.5.3 SA FR2 CSI-RS BFD non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193777 | 0123 | - | F | Update of TC 7.5.5.4 SA FR2 CSI-RS BFD DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193778 | 0124 | - | F | Addition of Minimum conformance requirements 7.6.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193779 | 0125 | - | F | Addition of 7.6.1.1 SA FR2 RRM measurement no-gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193780 | 0126 | - | F | Addition of 7.6.1.2 SA FR2 RRM measurement no-gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193781 | 0127 | - | F | Addition of 7.6.1.3 SA FR2 RRM measurement gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193782 | 0128 | - | F | Addition of 7.6.1.4 SA FR2 RRM measurement gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193784 | 0130 | - | F | Update of FR1 cell re-selection test case 6.1.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193785 | 0131 | - | F | Update of FR1-FR1 cell re-selection test case 6.1.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193786 | 0132 | - | F | Update of FR1-EUTRA higher priority cell re-selection test case 6.1.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193787 | 0133 | - | F | Update of FR1-EUTRA lower priority cell re-selection test case 6.1.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193788 | 0134 | - | F | Update of FR1-EUTRA handover known cell test case 6.3.1.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193790 | 0136 | - | F | Update of 6.6.1.1 SA FR1 RRM measurement no-gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193791 | 0137 | - | F | Update of 6.6.1.2 SA FR1 RRM measurement no-gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193792 | 0138 | - | F | Update of 6.6.1.3 SA FR1 RRM measurement gap non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193793 | 0139 | - | F | Update of 6.6.1.4 SA FR1 RRM measurement gap DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193794 | 0140 | - | F | Update of FR1 event-triggered without gap with SSB index test case 6.6.1.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193795 | 0141 | - | F | Update of FR1 event-triggered with gap with SSB index test case 6.6.1.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-193812 | 0145 | - | F | Addition of 6.3.2.2 minimum conformance requirements | 15.1.0 |
| 2019-06 | RAN#84 | R5-193872 | 0146 | - | F | Introduction of TC 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-193949 | 0148 | - | F | Correction of PRACH Configurations | 15.1.0 |
| 2019-06 | RAN#84 | R5-194328 | 0153 | - | F | Additional of new reference used in RRM test spec | 15.1.0 |
| 2019-06 | RAN#84 | R5-194329 | 0154 | - | F | Correction of reference spec number in RRM spec | 15.1.0 |
| 2019-06 | RAN#84 | R5-194494 | 0165 | - | F | Addition missing Editor's note 4.5.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194549 | 0173 | - | F | Correction of 5G RRM Test Case 5.6.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194550 | 0174 | - | F | Correction of 5G RRM Test Case 5.6.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194551 | 0175 | - | F | Correction of 5G RRM Test Case 5.6.2.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194552 | 0176 | - | F | Correction of 5G RRM Test Case 5.6.2.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194553 | 0177 | - | F | Correction of 5G RRM Test Case 5.6.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194555 | 0179 | - | F | Correction of 5G RRM Test Case 5.6.2.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194556 | 0180 | - | F | Correction of 5G RRM Test Case 5.6.2.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194557 | 0181 | - | F | Correction of Minimum conformance requirements 6.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194562 | 0186 | - | F | Correction of Minimum conformance requirements 7.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194564 | 0188 | - | F | Correction of 5G RRM Test Case 7.6.2.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194565 | 0189 | - | F | Correction of 5G RRM Test Case 7.6.2.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194566 | 0190 | - | F | Correction of 5G RRM Test Case 7.6.2.4 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194567 | 0191 | - | F | Correction of 5G RRM Test Case 7.6.2.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194568 | 0192 | - | F | Correction of 5G RRM Test Case 7.6.2.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194569 | 0193 | - | F | Correction of 5G RRM Test Case 7.6.2.7 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194570 | 0194 | - | F | Correction of 5G RRM Test Case 7.6.2.8 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194571 | 0195 | - | F | Correction of Minimum conformance requirements 7.3.2.1.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194572 | 0196 | - | F | Correction of 5G RRM Test Case 7.3.2.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194573 | 0197 | - | F | Correction of 5G RRM Test Case 7.3.2.1.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-194700 | 0208 | - | F | Clean-up in EN-DC FR1 transmit timing accuracy test | 15.1.0 |
| 2019-06 | RAN#84 | R5-194702 | 0210 | - | F | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194703 | 0211 | - | F | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194704 | 0212 | - | F | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-194710 | 0214 | - | F | Update to E-UTRA configuration for RRM EN-DC tests to align with core spec update | 15.1.0 |
| 2019-06 | RAN#84 | R5-194713 | 0216 | - | F | Update to EN-DC FR1 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195013 | 0156 | 1 | F | Updated to EN-DC FR1 RRM tests in clause 4.5.3 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195016 | 0198 | 1 | F | Test tolerance and measurement uncertainty in Annex F for Inter-Freq measurement test cases | 15.1.0 |
| 2019-06 | RAN#84 | R5-195017 | 0129 | 1 | F | Addition of default configuration in Annex H | 15.1.0 |
| 2019-06 | RAN#84 | R5-195018 | 0143 | 1 | F | Update of 6.3.2.2.1 random access | 15.1.0 |
| 2019-06 | RAN#84 | R5-195019 | 0144 | 1 | F | Update of 6.3.2.2.2 non-contention random access | 15.1.0 |
| 2019-06 | RAN#84 | R5-195020 | 0151 | 1 | F | Updated to EN-DC FR1 RRM tests in clause 4.6.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195021 | 0207 | 1 | F | Modification of EN-DC FR1 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195024 | 0199 | 1 | F | Modification of EN-DC FR1 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195025 | 0200 | 1 | F | Modification of EN-DC FR1 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195026 | 0201 | 1 | F | Modification of NR SA FR1 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195027 | 0202 | 1 | F | Modification of SA FR1 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195028 | 0203 | 1 | F | Modification of SA FR1 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195029 | 0204 | 1 | F | Modification of EN-DC FR2 TAAA | 15.1.0 |
| 2019-06 | RAN#84 | R5-195030 | 0205 | 1 | F | Modification of EN-DC FR2 SSB RLM OOS in non-DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195031 | 0206 | 1 | F | Modification of EN-DC FR2 SSB RLM OOS in DRX | 15.1.0 |
| 2019-06 | RAN#84 | R5-195032 | 0219 | 1 | F | Update to EN-DC FR2 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195033 | 0220 | 1 | F | Update to EN-DC FR2 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195034 | 0221 | 1 | F | Addition of EN-DC FR2 Transmit Timing Accuracy tests | 15.1.0 |
| 2019-06 | RAN#84 | R5-195035 | 0209 | 1 | F | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195036 | 0150 | 1 | F | Correction to EN-DC FR1 UE transmit timing accuracy TC 4.4.1.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195037 | 0155 | 1 | F | Updated to cell configuration mapping table for RRM tests | 15.1.0 |
| 2019-06 | RAN#84 | R5-195038 | 0147 | 1 | F | Introduction of TC 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | 15.1.0 |
| 2019-06 | RAN#84 | R5-195039 | 0178 | 1 | F | Correction of 5G RRM Test Case 5.6.2.6 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195040 | 0166 | 1 | F | Correction of default message content for RRM in Annex H | 15.1.0 |
| 2019-06 | RAN#84 | R5-195041 | 0217 | 1 | F | Update to NR SA FR1 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195042 | 0218 | 1 | F | Update to NR SA FR1 RLM IS DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195043 | 0215 | 1 | F | Update to EN-DC FR1 RLM IS non-DRX test with SSB-based RLM RS | 15.1.0 |
| 2019-06 | RAN#84 | R5-195044 | 0187 | 1 | F | Correction of 5G RRM Test Case 7.6.2.1 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195045 | 0103 | 1 | F | Update of EN-DC RLM out-of-sync in non-DRX test case 4.5.1.5 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195099 | 0223 | - | F | RRM implementation of FR2 UL demod OTA tests using single pol Rx TE | 15.1.0 |
| 2019-06 | RAN#84 | R5-195173 | 0167 | 1 | F | Correction of Minimum conformance requirements 4.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195174 | 0168 | 1 | F | Correction of 5G RRM Test Case 4.6.2.1 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195175 | 0169 | 1 | F | Correction of 5G RRM Test Case 4.6.2.2 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195176 | 0170 | 1 | F | Correction of 5G RRM Test Case 4.6.2.3 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195177 | 0171 | 1 | F | Correction of 5G RRM Test Case 4.6.2.4 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195179 | 0172 | 1 | F | Correction of Minimum conformance requirements 5.6.2.0 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195182 | 0142 | 1 | F | Addition of FR1-E-UTRAN event-triggered reporting in DRX test case 6.6.3.2 | 15.1.0 |
| 2019-06 | RAN#84 | R5-195185 | 0158 | 1 | F | Update of FR1 Test tolerance and uncertainties in AnnexF | 15.1.0 |
| 2019-06 | RAN#84 | R5-195186 | 0182 | 1 | F | Correction of 5G RRM Test Case 6.6.2.1 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195187 | 0183 | 1 | F | Correction of 5G RRM Test Case 6.6.2.2 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195188 | 0184 | 1 | F | Correction of 5G RRM Test Case 6.6.2.3 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195189 | 0185 | 1 | F | Correction of 5G RRM Test Case 6.6.2.4 with FR1 Test tolerance | 15.1.0 |
| 2019-06 | RAN#84 | R5-195445 | 0152 | 2 | F | Updated to SA FR1 RRM tests in clause 6.6.3 | 15.1.0 |
| 2019-06 | RAN#84 | - | - | - | - | Administrative release upgrade to match the release of 3GPP TS 38.508-1 and TS 38.521-1 which were upgraded at RAN#84 to Rel-16 due to Rel-16 relevant CR(s) | 16.0.0 |
| 2019-09 | RAN#85 | R5-195563 | 0228 | - | F | Addition of 5.6.1.0 minimum requirements | 16.1.0 |
| 2019-09 | RAN#85 | R5-195568 | 0233 | - | F | Update of 6.1.1.2 inter-freq cell re-selection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195569 | 0234 | - | F | Update of 6.1.2.1 inter-RAT cell re-selection to higher priority | 16.1.0 |
| 2019-09 | RAN#85 | R5-195571 | 0236 | - | F | Update of 6.3.1.4 inter-RAT handover to known cell | 16.1.0 |
| 2019-09 | RAN#85 | R5-195573 | 0238 | - | F | Update Test Tolerance of 6.3.2.1.1 intra-freq RRC re-establishment | 16.1.0 |
| 2019-09 | RAN#85 | R5-195574 | 0239 | - | F | Update Test Tolerance of 6.3.2.1.2 inter-freq RRC re-establishment | 16.1.0 |
| 2019-09 | RAN#85 | R5-195575 | 0240 | - | F | Update Test Tolerance of 6.3.2.3.1 NR RRC redirection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195576 | 0241 | - | F | Update Test Tolerance of 6.3.2.3.2 inter-RAT RRC redirection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195577 | 0242 | - | F | Update of 6.5.1.5 RLM out-of-sync non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195578 | 0243 | - | F | Update of 6.5.1.6 RLM in-sync non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195579 | 0244 | - | F | Update of 6.5.1.7 RLM out-of-sync in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195580 | 0245 | - | F | Update of 6.5.1.8 RLM in-sync in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195581 | 0246 | - | F | Update of 6.5.5.3 CSI-RS-based BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195582 | 0247 | - | F | Update of 6.5.5.4 CSI-RS-based BFD in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195596 | 0248 | - | F | Addition of minimum conformance requirements for FR2 EN-DC CSI-RS based RLM | 16.1.0 |
| 2019-09 | RAN#85 | R5-195601 | 0253 | - | F | Addition of minimum conformance requirements for FR2 EN-DC CSI-RS based BFD | 16.1.0 |
| 2019-09 | RAN#85 | R5-195604 | 0256 | - | F | Addition of minimum conformance requirements for SA FR2 reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195605 | 0257 | - | F | Addition of NR test case 7.1.1.1-intra freq reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195606 | 0258 | - | F | Addition of NR test case 7.1.1.2-inter freq reselection | 16.1.0 |
| 2019-09 | RAN#85 | R5-195608 | 0260 | - | F | Addition of NR test case 8.4.2.1-without SBI non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195609 | 0261 | - | F | Addition of NR test case 8.4.2.2-without SBI DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195610 | 0262 | - | F | Addition of NR test case 8.4.2.3-with SBI non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195611 | 0263 | - | F | Addition of NR test case 8.4.2.4-with SBI DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195612 | 0264 | - | F | Addition of minimum conformance requirements for FR1 EN-DC CSI-RS based RLM | 16.1.0 |
| 2019-09 | RAN#85 | R5-195623 | 0275 | - | F | Correction of NR test case 5.5.2.1-interruption transition DRX sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195624 | 0276 | - | F | Correction of NR test case 5.5.2.2-interruption transition DRX async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195625 | 0277 | - | F | Correction of NR test case 5.5.2.3-interruption NR deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195626 | 0278 | - | F | Correction of NR test case 5.5.2.4-interruption NR deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195627 | 0279 | - | F | Correction of NR test case 5.5.2.5-interruption LTE deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-195628 | 0280 | - | F | Correction of NR test case 5.5.2.6-interruption LTE deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-195629 | 0281 | - | F | Correction of NR test case 7.5.5.1-SSB BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195630 | 0282 | - | F | Correction of NR test case 7.5.5.2-SSB BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195631 | 0283 | - | F | Correction of NR test case 7.5.5.3-CSI-RS BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195632 | 0284 | - | F | Correction of NR test case 7.5.5.4-CSI-RS BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195633 | 0285 | - | F | Correction of NR test case 7.6.1.1-without gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195634 | 0286 | - | F | Correction of NR test case 7.6.1.2-without gap DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-195636 | 0288 | - | F | Correction of NR test case 7.6.1.4-with gap DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-196140 | 0295 | - | F | Update NR operating band groups | 16.1.0 |
| 2019-09 | RAN#85 | R5-196250 | 0296 | - | F | Correction to EN-DC FR1 event-triggered reporting without gap TC 4.6.1.1 and 4.6.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196503 | 0301 | - | F | Updated to EN-DC FR1 RRM tests in clause 4.5.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196504 | 0302 | - | F | Updated to EN-DC FR2 RRM tests in clause 5.5.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196547 | 0304 | - | F | Update of FR1 Test tolerance and uncertainties in AnnexF | 16.1.0 |
| 2019-09 | RAN#85 | R5-196601 | 0306 | - | F | Correction of the reference for test frequencies and test mode - Chapter 4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196602 | 0307 | - | F | Correction of the reference for test frequencies and test mode - Chapter 6 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196603 | 0308 | - | F | Editorial Corrections to section 5.5 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196659 | 0309 | - | F | Editorial corrections of Annex A | 16.1.0 |
| 2019-09 | RAN#85 | R5-196660 | 0310 | - | F | Correction of RRM Test Case 7.3.2.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196661 | 0311 | - | F | Correction of RRM Test Case 7.3.2.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196662 | 0312 | - | F | Correction of cell configuration mapping for RRM Test Cases in Annex E.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196666 | 0316 | - | F | Correction of RRM Test Case 5.6.2.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196667 | 0317 | - | F | Correction of RRM Test Case 5.6.2.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196668 | 0318 | - | F | Correction of RRM Test Case 5.6.2.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196669 | 0319 | - | F | Correction of RRM Test Case 5.6.2.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196670 | 0320 | - | F | Correction of RRM Test Case 5.6.2.5 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196671 | 0321 | - | F | Correction of RRM Test Case 5.6.2.6 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196672 | 0322 | - | F | Correction of RRM Test Case 5.6.2.7 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196673 | 0323 | - | F | Correction of RRM Test Case 5.6.2.8 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196674 | 0324 | - | F | Addition of minimum conformance requirements for L1-RSRP measurement RRM FR1 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196675 | 0325 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 4.6.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196676 | 0326 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 4.6.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196677 | 0327 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 4.6.3.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196679 | 0329 | - | F | Addition of minimum conformance requirements for L1-RSRP measurement RRM FR2 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-196680 | 0330 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 5.6.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196681 | 0331 | - | F | Addition of SSB based L1-RSRP measurement RRM test case 5.6.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196682 | 0332 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 5.6.3.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196735 | 0334 | - | F | Editorial to fix conversion issues with two equations | 16.1.0 |
| 2019-09 | RAN#85 | R5-196736 | 0335 | - | F | Adding Chapter 8 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196737 | 0336 | - | F | New test 4.5.7.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196738 | 0337 | - | F | New test 4.5.5.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196739 | 0338 | - | F | New test 4.5.5.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196740 | 0339 | - | F | New test 4.7.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196741 | 0340 | - | F | New test 4.7.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196742 | 0341 | - | F | New test 6.7.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196743 | 0342 | - | F | New test 6.7.3.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196925 | 0351 | - | F | Modification of EN-DC FR2 TAAA Section 5.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-196943 | 0360 | - | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 5.6.3.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197361 | 0357 | 1 | F | Modification of SA FR1 TAAA Section 6.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197364 | 0290 | 1 | F | Update of NR UE Tx Timing Accuracy Test | 16.1.0 |
| 2019-09 | RAN#85 | R5-197366 | 0235 | 1 | F | Update Test Tolerance of 6.1.2.2 inter-RAT cell re-selection to lower priority | 16.1.0 |
| 2019-09 | RAN#85 | R5-197396 | 0346 | 1 | F | Update NSA Event trigger reporting test case | 16.1.0 |
| 2019-09 | RAN#85 | R5-197397 | 0352 | 1 | F | Modification of EN-DC FR2 SSB-RLM OOS Section 5.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197398 | 0353 | 1 | F | Modification of EN-DC FR2 SSB-RLM OOS in DRX Section 5.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197399 | 0354 | 1 | F | Modification of Synchronous EN-DC FR2 RRC DL BWP Switch in non-DRX Section 5.5.6.2.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197400 | 0355 | 1 | F | Modification of SA FR1 Handover with unknown Target Cell Section 6.3.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197401 | 0356 | 1 | F | Modification of SA FR1-FR1 Handover with unknown Target Cell Section 6.3.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197402 | 0224 | 1 | F | Update of 4.6.1.3 event with gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197403 | 0225 | 1 | F | Update of 4.6.1.4 event with gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197404 | 0226 | 1 | F | Update of 4.6.1.5 event without gap with ssb index | 16.1.0 |
| 2019-09 | RAN#85 | R5-197405 | 0227 | 1 | F | Update of 4.6.1.6 event with gap with ssb index | 16.1.0 |
| 2019-09 | RAN#85 | R5-197406 | 0314 | 1 | F | Correction of 5G RRM Inter Frequency measurements EN-DC test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197407 | 0229 | 1 | F | Addition of 5.6.1.1 event without gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197408 | 0230 | 1 | F | Addition of 5.6.1.2 event without gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197409 | 0231 | 1 | F | Addition of 5.6.1.3 event with gap non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197410 | 0232 | 1 | F | Addition of 5.6.1.4 event with gap in DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197411 | 0249 | 1 | F | Addition of NR test case 5.5.1.5-CSI-RS RLM OOS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197412 | 0250 | 1 | F | Addition of NR test case 5.5.1.6-CSI-RS RLM IS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197413 | 0251 | 1 | F | Addition of NR test case 5.5.1.7-CSI-RS RLM OOS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197414 | 0252 | 1 | F | Addition of NR test case 5.5.1.8-CSI-RS RLM IS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197415 | 0254 | 1 | F | Addition of NR test case 5.5.5.3-CSI-RS BFD non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197416 | 0255 | 1 | F | Addition of NR test case 5.5.5.4-CSI-RS BFD DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197417 | 0259 | 1 | F | Addition of minimum conformance requirements for inter-RAT NR measurements | 16.1.0 |
| 2019-09 | RAN#85 | R5-197418 | 0289 | 1 | F | Correction of default configuration in annex H | 16.1.0 |
| 2019-09 | RAN#85 | R5-197419 | 0303 | 1 | F | Updated to cell configuration mapping table for RRM tests | 16.1.0 |
| 2019-09 | RAN#85 | R5-197420 | 0313 | 1 | F | Correction of default message content for RRM in Annex H | 16.1.0 |
| 2019-09 | RAN#85 | R5-197422 | 0305 | 1 | F | Clarification on NE-DC tests for RRM | 16.1.0 |
| 2019-09 | RAN#85 | R5-197423 | 0343 | 1 | F | Addition RRM FR2 test setups into TS 38.533 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197424 | 0345 | 1 | F | Update TDD UL-DL Config based on TS 38.133 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197425 | 0347 | 1 | F | Modification of EN-DC FR1 UE Transmit Timing Section 4.4.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197426 | 0265 | 1 | F | Correction of NR test case 4.5.1.5-CSI-RS RLM OOS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197427 | 0266 | 1 | F | Correction of NR test case 4.5.1.6-CSI-RS RLM IS non-DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197428 | 0267 | 1 | F | Correction of NR test case 4.5.1.7-CSI-RS RLM OOS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197429 | 0268 | 1 | F | Correction of NR test case 4.5.1.8-CSI-RS RLM IS DRX | 16.1.0 |
| 2019-09 | RAN#85 | R5-197430 | 0269 | 1 | F | Correction of NR test case 4.5.2.1-interruption transition DRX sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197431 | 0270 | 1 | F | Correction of NR test case 4.5.2.2-interruption transition DRX async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197432 | 0271 | 1 | F | Correction of NR test case 4.5.2.3-interruption NR deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197433 | 0272 | 1 | F | Correction of NR test case 4.5.2.4-interruption NR deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197434 | 0273 | 1 | F | Correction of NR test case 4.5.2.5-interruption LTE deactivated SCell sync | 16.1.0 |
| 2019-09 | RAN#85 | R5-197435 | 0274 | 1 | F | Correction of NR test case 4.5.2.6-interruption LTE deactivated SCell async | 16.1.0 |
| 2019-09 | RAN#85 | R5-197436 | 0328 | 1 | F | Addition of CSI-RS based L1-RSRP measurement RRM test case 4.6.3.4 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197437 | 0315 | 1 | F | Correction of 5G RRM Inter Frequency measurements SA test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197570 | 0294 | 1 | F | Annex E and F FR1 Test tolerance update for SS-RSRP | 16.1.0 |
| 2019-09 | RAN#85 | R5-197584 | 0344 | 1 | F | Addition Annex for RRM OTA procedures in TS 38.533 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197607 | 0287 | 1 | F | Correction of NR test case 7.6.1.3-with gap non-DRX. | 16.1.0 |
| 2019-09 | RAN#85 | R5-197611 | 0237 | 1 | F | Update of 6.3.1.5 inter-RAT handover to unknown cell | 16.1.0 |
| 2019-09 | RAN#85 | R5-197612 | 0291 | 1 | F | Update of SSB-based RLM in-sync test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197624 | 0297 | 1 | F | Correction to PRACH configurations for FR1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197651 | 0348 | 2 | F | Modification of EN-DC FR1 TAAA Section 4.4.3.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197652 | 0349 | 2 | F | Modification of EN-DC FR1 SSB-RLM OOS Section 4.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197653 | 0350 | 2 | F | Modification of EN-DC FR1 SSB-RLM OOS DRX Section 4.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197654 | 0358 | 2 | F | Modification of SA FR1 SSB RLM OOS in non-DRX Section 6.5.1.1 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197655 | 0359 | 2 | F | Modification of SA FR1 SSB RLM OOS in DRX Section 6.5.1.3 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197656 | 0292 | 2 | F | FR1 Test tolerance update for SS-RSRP NSA FR1 test cases | 16.1.0 |
| 2019-09 | RAN#85 | R5-197657 | 0300 | 2 | F | Correction to EN-DC FR1 radio link monitoring TC 4.5.1.1 and 4.5.1.2 | 16.1.0 |
| 2019-09 | RAN#85 | R5-197658 | 0293 | 2 | F | FR1 Test tolerance update for SS-RSRP SA FR1 test cases | 16.1.0 |
| 2019-12 | RAN#86 | R5-197794 | 0369 | - | F | Update Test Tolerance of 4.5.2.1 interruptions active and non-active in sync | 16.2.0 |
| 2019-12 | RAN#86 | R5-197795 | 0370 | - | F | Update Test Tolerance of 4.5.2.2 interruptions active and non-active in async | 16.2.0 |
| 2019-12 | RAN#86 | R5-197804 | 0379 | - | F | Update Test Tolerance of 6.6.3.1 inter-RAT measurement non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-197805 | 0380 | - | F | Update Test Tolerance of 6.6.3.2 inter-RAT measurement DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-197813 | 0382 | - | F | Addition of minimum conformance requirements 5.5.5.0.1-SSB based BFD | 16.2.0 |
| 2019-12 | RAN#86 | R5-197814 | 0383 | - | F | Addition of minimum conformance requirements 8.2.1.0-inter-RAT reselection | 16.2.0 |
| 2019-12 | RAN#86 | R5-197815 | 0384 | - | F | Addition of minimum conformance requirements 8.3.1.0-inter-RAT handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-197816 | 0385 | - | F | Addition of minimum conformance requirements 8.4.1.0-inter-RAT SFTD delay | 16.2.0 |
| 2019-12 | RAN#86 | R5-197817 | 0386 | - | F | Addition of minimum conformance requirements 8.5.1.0-inter-RAT SFTD accuracy | 16.2.0 |
| 2019-12 | RAN#86 | R5-197820 | 0389 | - | F | Addition of NR test case 8.2.1.1-high priority NR reselection | 16.2.0 |
| 2019-12 | RAN#86 | R5-198029 | 0394 | - | F | Update NSA SS-RSRP tests for 4Rx connection diagram | 16.2.0 |
| 2019-12 | RAN#86 | R5-198032 | 0397 | - | F | Align Annex D to TS 38.133 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198289 | 0400 | - | F | Correction of PRACH index in 6.1.1.1, 6.3.2.2.1 and 6.3.2.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198415 | 0404 | - | F | Correction to NR SA FR1 timing advance adjustment accuracy TC 6.4.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198416 | 0405 | - | F | Correction to intra-frequency event-triggered reporting TC 6.6.1.1, 6.6.1.2, 6.6.1.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198421 | 0410 | - | F | Correction to FR1 SA random access test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-198472 | 0412 | - | F | Correct message contents for 5.6.2.x tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198477 | 0416 | - | F | Correction to Annex E | 16.2.0 |
| 2019-12 | RAN#86 | R5-198518 | 0418 | - | F | Update PUSCH symbol length as message exception in 6.4.1.1 in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198519 | 0419 | - | F | Update 4.4.3.1 test requirements | 16.2.0 |
| 2019-12 | RAN#86 | R5-198545 | 0420 | - | F | Update 4.7.1.1.1 test requirements in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198549 | 0421 | - | F | Updated to cell configuration mapping table for RRM tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198550 | 0422 | - | F | Minimum conformance requirements updated for 5G RRM Inter-RAT measurements tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198551 | 0423 | - | F | Minimum conformance requirements updated for 5G RRM SCell activation and deactivation delay tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-198558 | 0428 | - | F | 5G RRM Spec 3GPP style correction in clause 4 and 5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198572 | 0429 | - | F | Correction of minimum conformance requirements 4.6.2.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198575 | 0432 | - | F | Correction of minimum conformance requirements 5.6.3.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198577 | 0434 | - | F | Correction of clause 3 definitions, symbols and abbreviations | 16.2.0 |
| 2019-12 | RAN#86 | R5-198578 | 0435 | - | F | Correction of RRM Test Case 4.6.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198581 | 0438 | - | F | Correction of RRM Test Case 4.6.3.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198582 | 0439 | - | F | Correction of RRM Test Case 5.6.2.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198583 | 0440 | - | F | Correction of RRM Test Case 5.6.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198584 | 0441 | - | F | Correction of RRM Test Case 5.6.2.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198585 | 0442 | - | F | Correction of RRM Test Case 5.6.2.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198586 | 0443 | - | F | Correction of RRM Test Case 5.6.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198587 | 0444 | - | F | Correction of RRM Test Case 5.6.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198588 | 0445 | - | F | Correction of RRM Test Case 5.6.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198589 | 0446 | - | F | Correction of RRM Test Case 5.6.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198594 | 0451 | - | F | Correction of RRM Test Case 7.6.2.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198595 | 0452 | - | F | Correction of RRM Test Case 7.6.2.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198596 | 0453 | - | F | Correction of RRM Test Case 7.6.2.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198597 | 0454 | - | F | Correction of RRM Test Case 7.6.2.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198598 | 0455 | - | F | Correction of RRM Test Case 7.6.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198599 | 0456 | - | F | Correction of RRM Test Case 7.6.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198601 | 0458 | - | F | Correction of RRM Test Case 7.6.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198692 | 0471 | - | F | Addition of RRM Test Case 6.6.4.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198693 | 0472 | - | F | Addition of RRM Test Case 6.6.4.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198694 | 0473 | - | F | Addition of RRM Test Case 6.6.4.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198695 | 0474 | - | F | Addition of RRM Test Case 6.6.4.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198696 | 0475 | - | F | Addition of RRM Test Case 7.6.3.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198697 | 0476 | - | F | Addition of RRM Test Case 7.6.3.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198698 | 0477 | - | F | Addition of RRM Test Case 7.6.3.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-198699 | 0478 | - | F | Addition of RRM Test Case 7.6.3.4 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199364 | 0365 | 1 | F | Update Test Tolerance of 4.5.1.5 RLM OOS non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199365 | 0367 | 1 | F | Update Test Tolerance of 4.5.1.7 RLM OOS DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199366 | 0375 | 1 | F | Update Test Tolerance of 6.5.1.5 SA RLM OOS non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199367 | 0377 | 1 | F | Update Test Tolerance of 6.5.1.7 SA RLM OOS DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199368 | 0366 | 1 | F | Update Test Tolerance of 4.5.1.6 RLM in-sync non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199369 | 0368 | 1 | F | Update Test Tolerance of 4.5.1.8 RLM in-sync DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199370 | 0376 | 1 | F | Update Test Tolerance of 6.5.1.6 SA RLM in-sync non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199371 | 0378 | 1 | F | Update Test Tolerance of 6.5.1.8 SA RLM in-sync DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199379 | 0417 | 1 | F | Update 4.4.1.1 test procedure in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199380 | 0479 | 1 | F | Update 4.5.1.1 and 4.5.1.2 in TS 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199381 | 0406 | 1 | F | Correction to NR SA FR1 SS-RSRP absolute and relative measurement accuracy Intra-frequency measurements | 16.2.0 |
| 2019-12 | RAN#86 | R5-199389 | 0402 | 1 | F | Correction to NR SA Cell re-selection tests | 16.2.0 |
| 2019-12 | RAN#86 | R5-199390 | 0436 | 1 | F | Correction of RRM Test Case 4.6.3.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199391 | 0437 | 1 | F | Correction of RRM Test Case 4.6.3.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199392 | 0387 | 1 | F | Addition of NR test case 5.5.5.1-ENDC FR2 SSB BFD no-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199393 | 0388 | 1 | F | Addition of NR test case 5.5.5.2-ENDC FR2 SSB BFD DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199394 | 0431 | 1 | F | Correction of minimum conformance requirements 5.6.2.0 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199395 | 0395 | 1 | F | Update SA SS-RSRP tests for 4Rx connection diagram | 16.2.0 |
| 2019-12 | RAN#86 | R5-199396 | 0403 | 1 | F | Correction to NR SA FR1 UE transmit timing accuracy TC 6.4.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199397 | 0457 | 1 | F | Correction of RRM Test Case 7.6.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199398 | 0361 | 1 | F | Add AoA Setup 4 for FR2 RRM Test cases | 16.2.0 |
| 2019-12 | RAN#86 | R5-199399 | 0390 | 1 | F | Addition of NR test case 8.3.1.1-known handover | 16.2.0 |
| 2019-12 | RAN#86 | R5-199400 | 0391 | 1 | F | Addition of NR test case 8.4.1.1-SFTD delay non-DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199401 | 0392 | 1 | F | Addition of NR test case 8.4.1.2-SFTD delay DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199402 | 0393 | 1 | F | Addition of NR test case 8.5.1.1-SFTD accuracy | 16.2.0 |
| 2019-12 | RAN#86 | R5-199403 | 0396 | 1 | F | Align Annex A to TS 38.133 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199404 | 0413 | 1 | F | Clean up RRM message contents in Annex H | 16.2.0 |
| 2019-12 | RAN#86 | R5-199405 | 0424 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.5 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199406 | 0425 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.6 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199407 | 0426 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.7 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199408 | 0427 | 1 | F | New 5G RRM Inter-RAT measurement TC 8.4.2.8 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199409 | 0433 | 1 | F | Correction of message content in Annex H | 16.2.0 |
| 2019-12 | RAN#86 | R5-199436 | 0470 | 1 | F | Introduction of n29 and n65 to 38.533 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199499 | 0401 | 1 | F | Correction to EN-DC FR1 UE transmit timing accuracy TC 4.4.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199515 | 0381 | 1 | F | Update Test Tolerance in Annex F | 16.2.0 |
| 2019-12 | RAN#86 | R5-199517 | 0481 | 1 | F | Update General parameters in test case 4.5.1.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199533 | 0482 | 1 | F | Update NSA FR2 RLM IS non-DRX test | 16.2.0 |
| 2019-12 | RAN#86 | R5-199534 | 0483 | 1 | F | Update NSA FR2 RLM IS test with DRX | 16.2.0 |
| 2019-12 | RAN#86 | R5-199535 | 0484 | 1 | F | Update NSA FR2 Timing Accuracy Test | 16.2.0 |
| 2019-12 | RAN#86 | R5-199544 | 0399 | 1 | F | Add applicable test methods for RRM FR2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199550 | 0463 | 1 | F | Addition of NR SA FR1 Handover with known Target Cell Section 6.3.1.1 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199551 | 0464 | 1 | F | Modification of NR SA FR1 Handover with unknown Target Cell Section 6.3.1.2 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199552 | 0465 | 1 | F | Modification of NR SA FR1-FR1 Handover with unknown Target Cell Section 6.3.1.3 | 16.2.0 |
| 2019-12 | RAN#86 | R5-199583 | 0409 | 1 | F | Correction to FR1 NSA random access test cases | 16.2.0 |
| 2020-03 | RAN#87 | R5-200822 | 0544 | - | F | Correction to Test Applicability for 4.3.2.2.2 Non-contention based random access test in FR1 for PSCell in EN-DC | 16.3.0 |
| 2020-03 | RAN#87 | R5-200823 | 0545 | - | F | Correction to SRS Configuration for 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | 16.3.0 |
| 2020-03 | RAN#87 | R5-200829 | 0551 | - | F | Correction to Message Exception for 4.6.1.5 EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200833 | 0555 | - | F | Correction to Test Applicability for 6.3.2.2.2 Non-Contention based random access test in FR1 for NR standalone | 16.3.0 |
| 2020-03 | RAN#87 | R5-200834 | 0556 | - | F | Correction to SRS Configuration for 6.4.3.1 NR SA FR1 timing advance adjustment accuracy | 16.3.0 |
| 2020-03 | RAN#87 | R5-200839 | 0561 | - | F | Correction to Message Exception for 6.6.1.5 NR SA FR1 event-triggered reporting without gap in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200277 | 0488 | - | F | Test tolerance update intra-frequency SS-SINR NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200278 | 0489 | - | F | Test tolerance update inter-frequency SS-SINR NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200279 | 0490 | - | F | Test tolerance update inter-frequency SS-RSRP SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200281 | 0492 | - | F | Test tolerance update inter-frequency SS-RSRQ SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200282 | 0493 | - | F | Test tolerance update intra-frequency SS-SINR SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200283 | 0494 | - | F | Test tolerance update inter-frequency SS-SINR SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200306 | 0497 | - | F | Reference SSB configuration correction | 16.3.0 |
| 2020-03 | RAN#87 | R5-200421 | 0508 | - | F | Correction to Active UL BWP for intra-frequency event triggered reporting with gap | 16.3.0 |
| 2020-03 | RAN#87 | R5-200422 | 0509 | - | F | Correction to EN-DC FR1 event-triggered reporting test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200423 | 0510 | - | F | Correction to FR1 NR SA E-UTRA cell re-selection test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200425 | 0512 | - | F | Correction to NR SA FR1 SS-RSRP measurement accuracy tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200427 | 0514 | - | F | Correction to the simulated cell for cell-reselection test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200428 | 0515 | - | F | Correction to UE transmit timing accuracy test case | 16.3.0 |
| 2020-03 | RAN#87 | R5-200564 | 0524 | - | F | Correction to Statistical testing of delay and UE measurement performance in RRM tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200606 | 0526 | - | F | Cell mapping update measurement tests Annex E | 16.3.0 |
| 2020-03 | RAN#87 | R5-200607 | 0527 | - | F | Re-submission R5-197804 not implemented | 16.3.0 |
| 2020-03 | RAN#87 | R5-200608 | 0528 | - | F | Corrections to SS-RSRP meas accuracy NSA tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200610 | 0530 | - | F | Correct message contents measurement tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-200705 | 0533 | - | F | Correction to TC 6.1.1.2 FR1 inter-freq re-selection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200708 | 0536 | - | F | Update of TC 6.3.2.3.1 FR1 RRC redirection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200709 | 0537 | - | F | Update of TC 6.3.2.3.2 inter-freq RRC redirection | 16.3.0 |
| 2020-03 | RAN#87 | R5-200785 | 0539 | - | F | Update of EN-DC SSB-based RLM TC 4.5.1.1, 4.5.1.2, 4.5.1.3 and 4.5.1.4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200786 | 0540 | - | F | Update of EN-DC CSI-RS-based RLM TC 4.5.1.5, 4.5.1.6, 4.5.1.7 and 4.5.1.8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200787 | 0541 | - | F | Update of NR CSI-RS-based RLM TC 6.5.1.5, 6.5.1.6, 6.5.1.7 and 6.5.1.8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200818 | 0543 | - | F | Correction to cell mapping Annex E | 16.3.0 |
| 2020-03 | RAN#87 | R5-200858 | 0569 | - | F | Corrections to Table H.3.1-8 | 16.3.0 |
| 2020-03 | RAN#87 | R5-200916 | 0538 | 1 | F | Update to test applicability per permitted test method | 16.3.0 |
| 2020-03 | RAN#87 | R5-200986 | 0507 | 1 | F | Core alignment for event-triggered reporting test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-200987 | 0534 | 1 | F | Update of TC 6.3.2.1.1 FR1 RRC re-establishment | 16.3.0 |
| 2020-03 | RAN#87 | R5-200988 | 0535 | 1 | F | Update of TC 6.3.2.1.2 inter-freq RRC re-establishment | 16.3.0 |
| 2020-03 | RAN#87 | R5-201030 | 0529 | 1 | F | Corrections to SS-RSRP meas accuracy SA tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-201039 | 0485 | 1 | F | Test tolerance update inter-frequency SS-RSRP NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201040 | 0486 | 1 | F | Test tolerance update intra-frequency SS-RSRQ NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201041 | 0491 | 1 | F | Test tolerance update intra-frequency SS-RSRQ SA | 16.3.0 |
| 2020-03 | RAN#87 | R5-201056 | 0511 | 1 | F | Correction to message contents in 6.6.1.5 and 6.6.1.6 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201057 | 0513 | 1 | F | Correction to test description of RRM TC 4.6.1.3 and 4.6.1.6 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201058 | 0516 | 1 | F | Update of maximum test system uncertainty for FR1 RRM Test | 16.3.0 |
| 2020-03 | RAN#87 | R5-201063 | 0517 | 1 | F | Update FR1 Test Tolerance of 4.5.2.3 interruptions on NR SCC in sync | 16.3.0 |
| 2020-03 | RAN#87 | R5-201064 | 0518 | 1 | F | Update FR1 Test Tolerance of 4.5.2.4 interruptions on NR SCC in async | 16.3.0 |
| 2020-03 | RAN#87 | R5-201071 | 0552 | 1 | F | Correction to Test Applicability for 4.6.2.2 EN-DC FR1-FR1 event-triggered reporting in DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201072 | 0553 | 1 | F | Correction to Test Applicability for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201073 | 0554 | 1 | F | Correction to Test Applicability for 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201078 | 0562 | 1 | F | Correction to Test Applicability for 6.6.2.1 NR SA FR1-FR1 event-triggered reporting in non-DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201080 | 0564 | 1 | F | Correction to Test Applicability for 6.6.2.5 NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201081 | 0565 | 1 | F | Correction to Test Applicability for 6.6.2.6 NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.3.0 |
| 2020-03 | RAN#87 | R5-201082 | 0571 | 1 | F | Correction to Test Parameters for 4.6.1.1 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201083 | 0572 | 1 | F | Correction to Test Parameters for 4.6.1.2 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201160 | 0531 | 1 | F | Correct message contents event triggered measurement tests | 16.3.0 |
| 2020-03 | RAN#87 | R5-201166 | 0567 | 1 | F | Update RLM non DRX test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-201167 | 0568 | 1 | F | Update 4.5.1.4 test case | 16.3.0 |
| 2020-03 | RAN#87 | R5-201168 | 0570 | 1 | F | Clarification 4.4.1.1 test procedure | 16.3.0 |
| 2020-03 | RAN#87 | R5-201169 | 0521 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.3.0 |
| 2020-03 | RAN#87 | R5-201190 | 0542 | 1 | F | Update of NR SSB-based RLM TC 6.5.1.1, 6.5.1.2, 6.5.1.3 and 6.5.1.4 | 16.3.0 |
| 2020-03 | RAN#87 | R5-201240 | 0563 | 2 | F | Correction to Test Applicability for 6.6.2.2 NA SA FR1-FR1 event-triggered reporting in DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-201242 | 0532 | 2 | F | Correction to Test Applicability for 4.6.2.1 EN-DC FR1-FR1 event-triggered reporting in non-DRX | 16.3.0 |
| 2020-03 | RAN#87 | R5-200276 | 0487 | - | F | Test tolerance update inter-frequency SS-RSRQ NSA | 16.3.0 |
| 2020-03 | RAN#87 | R5-200284 | 0495 | - | F | Test tolerance update measurement tests Annex F | 16.3.0 |
| 2020-03 | RAN#87 | R5-201044 | 0496 | - | F | Test Tolerance and Measurement Uncertainty in Annex F for L1-RSRP measurement test cases | 16.3.0 |
| 2020-03 | RAN#87 | R5-201045 | 0499 | 1 | F | Correction of RRM Test Case 4.6.3.1 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201046 | 0500 | 1 | F | Correction of RRM Test Case 4.6.3.2 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201047 | 0501 | 1 | F | Correction of RRM Test Case 4.6.3.3 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201048 | 0502 | 1 | F | Correction of RRM Test Case 4.6.3.4 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201049 | 0503 | 1 | F | Correction of RRM Test Case 6.6.4.1 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201050 | 0504 | 1 | F | Correction of RRM Test Case 6.6.4.2 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201051 | 0505 | 1 | F | Correction of RRM Test Case 6.6.4.3 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201052 | 0506 | 1 | F | Correction of RRM Test Case 6.6.4.4 including FR1 Test Tolerance | 16.3.0 |
| 2020-03 | RAN#87 | R5-201070 | 0548 | 1 | F | Correction to Message Exception for 4.5.1.3 EN-DC FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201074 | 0557 | 1 | F | Correction to Test Parameter for 6.5.1.1 NR SA FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201075 | 0558 | 1 | F | Correction to Test Procedure for 6.5.1.2 NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201076 | 0559 | 1 | F | Correction to Message Exception for 6.5.1.3 NR SA FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201077 | 0560 | 1 | F | Correction to Message Exception for 6.5.1.4 NR SA FR1 radio link monitoring In-sync test for PSCell configured with SSB-based RLM RS in DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201105 | 0546 | 1 | F | Correction to Test Parameters for 4.5.1.1 EN-DC FR1 radio link monitoring Out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | 16.3.0 |
| 2020-03 | RAN#87 | R5-201106 | 0547 | 1 | F | Correction to Test Procedure for 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | 16.3.0 |
| 2020-06 | RAN#88 | R5-201395 | 0576 | - | F | Correction to NR test case 7.7.1.3.2 SS-RSRP relative accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-201396 | 0577 | - | F | Correction to NR test case 8.2.1.1 higher priority NR cell reselection | 16.4.0 |
| 2020-06 | RAN#88 | R5-201398 | 0579 | - | F | Correction to NR test case 8.4.1.1 SFTD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201399 | 0580 | - | F | Correction to NR test case 8.4.1.2 SFTD DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201400 | 0581 | - | F | Correction to NR test case 8.4.2.1 event-triggered without SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201401 | 0582 | - | F | Correction to NR test case 8.4.2.2 event-triggered without SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201402 | 0583 | - | F | Correction to NR test case 8.4.2.3 event-triggered with SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201403 | 0584 | - | F | Correction to NR test case 8.4.2.4 event-triggered with SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201404 | 0585 | - | F | Correction to NR test case 8.5.1.1 SFTD accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-201405 | 0586 | - | F | Addition to minimum conformance requirement 5.5.5.0.3 - scheduling restriction for BFD and CBD | 16.4.0 |
| 2020-06 | RAN#88 | R5-201407 | 0588 | - | F | Addition to minimum conformance requirement 5.5.8.0 - TCI-state switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201408 | 0589 | - | F | Addition to NR test case 5.5.8.1 MAC-CE based TCI switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201409 | 0590 | - | F | Addition to NR test case 5.5.8.2 RRC based TCI switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201412 | 0593 | - | F | Addition to minimum conformance requirements 7.5.1.0.5 - Scheduling restriction for RLM | 16.4.0 |
| 2020-06 | RAN#88 | R5-201414 | 0595 | - | F | Addition to minimum Conformance Requirements 7.5.6.1.0 - DCI-based DL active BWP switch | 16.4.0 |
| 2020-06 | RAN#88 | R5-201415 | 0596 | - | F | Addition to NR test case 7.5.6.1.3 FR2 DCI-based DL active BWP switch non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201619 | 0603 | - | F | Corrections to 6.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-201676 | 0614 | - | F | Correction to inter-freq measurement TCs for SSB configuration | 16.4.0 |
| 2020-06 | RAN#88 | R5-201677 | 0615 | - | F | Correction to TC 6.3.2.3.1 for the test procedure | 16.4.0 |
| 2020-06 | RAN#88 | R5-201679 | 0617 | - | F | Update of TC 5.6.1.1 event-triggered without gap non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201680 | 0618 | - | F | Update of TC 5.6.1.2 event-triggered without gap DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201681 | 0619 | - | F | Update of TC 5.6.1.3 event-triggered with gap non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201682 | 0620 | - | F | Update of TC 5.6.1.4 event-triggered with gap DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201686 | 0624 | - | F | Update of TC 8.4.2.5 FR2 event-triggered without SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201687 | 0625 | - | F | Update of TC 8.4.2.6 FR2 event-triggered without SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201688 | 0626 | - | F | Update of TC 8.4.2.7 FR2 event-triggered with SSB index non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201689 | 0627 | - | F | Update of TC 8.4.2.8 FR2 event-triggered with SSB index DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-201821 | 0634 | - | F | Correction of SS-SINR applicability -NSA | 16.4.0 |
| 2020-06 | RAN#88 | R5-201822 | 0635 | - | F | Correction of SS-SINR applicability -SA | 16.4.0 |
| 2020-06 | RAN#88 | R5-201824 | 0637 | - | F | Corrections to 4.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202054 | 0644 | - | F | Minimum Requirements SS-RSRP iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202055 | 0645 | - | F | Addition 8.5.2.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202056 | 0646 | - | F | Addition 8.5.2.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202057 | 0647 | - | F | Minimum Requirements SS-RSRQ iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202058 | 0648 | - | F | Addition 8.5.2.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202059 | 0649 | - | F | Addition 8.5.2.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202060 | 0650 | - | F | Minimum Requirements SS-SINR iRAT | 16.4.0 |
| 2020-06 | RAN#88 | R5-202061 | 0651 | - | F | Addition 8.5.2.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202062 | 0652 | - | F | Addition 8.5.2.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202063 | 0653 | - | F | Minimum requirements SS-RSRP NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202064 | 0654 | - | F | Addition 5.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202065 | 0655 | - | F | Addition 5.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202066 | 0656 | - | F | Addition 5.7.1.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202067 | 0657 | - | F | Minimum requirements SS-RSRQ NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202068 | 0658 | - | F | Addition 5.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202069 | 0659 | - | F | Addition 5.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202070 | 0660 | - | F | Minimum requirements SS-SINR NSA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202071 | 0661 | - | F | Addition 5.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202072 | 0662 | - | F | Addition 5.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202073 | 0663 | - | F | Minimum requirements SS-RSRP SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202074 | 0664 | - | F | Addition 7.7.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202075 | 0665 | - | F | Addition 7.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202076 | 0666 | - | F | Minimum requirements SS-RSRQ SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202077 | 0667 | - | F | Addition 7.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202078 | 0668 | - | F | Addition 7.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202079 | 0669 | - | F | Minimum requirements SS-SINR SA FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202080 | 0670 | - | F | Addition 7.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202081 | 0671 | - | F | Addition 7.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202091 | 0672 | - | F | Correction of OCNG configurations and CSI-RS for tracking in Annex A | 16.4.0 |
| 2020-06 | RAN#88 | R5-202092 | 0673 | - | F | Cell configuration for SA FR2 event-triggered reporting test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202093 | 0674 | - | F | Correction of MeasGapConfig in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202095 | 0676 | - | F | Correction of AoA configuration for FR2 SA Inter-freq measurement test cases in clause 5.6.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202096 | 0677 | - | F | Correction of RRM Test Case 7.6.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202097 | 0678 | - | F | Correction of RRM Test Case 7.6.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202098 | 0679 | - | F | Correction of RRM Test Case 7.6.2.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202099 | 0680 | - | F | Correction of RRM Test Case 7.6.2.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202100 | 0681 | - | F | Correction of RRM Test Case 7.6.2.5 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202101 | 0682 | - | F | Correction of RRM Test Case 7.6.2.6 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202102 | 0683 | - | F | Correction of RRM Test Case 7.6.2.7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202103 | 0684 | - | F | Correction of RRM Test Case 7.6.2.8 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202263 | 0704 | - | F | Core alignment in 4.6.1.5 and 4.6.1.6 Event Triggered Reporting with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202265 | 0706 | - | F | Correction on EN-DC FR1 radio link monitoring out-of-sync test | 16.4.0 |
| 2020-06 | RAN#88 | R5-202273 | 0714 | - | F | Correction to NR SA FR1 - E-UTRA RRC Connection release with redirection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202277 | 0718 | - | F | Editorial Correction to FR2 Interruption test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202385 | 0720 | - | F | Addition of RRM Cell configuration mapping table for EN-DC FR1 Test Case 4.5.4.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202387 | 0721 | - | F | Addition of RRC message content exceptions for UE UL Carrier Reconfiguration Delay | 16.4.0 |
| 2020-06 | RAN#88 | R5-202389 | 0722 | - | F | Addition of RRM Test Case 4.5.4.1 in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202415 | 0728 | - | F | Alignment 5.4.1.1 test procedure with 4.4.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202418 | 0730 | - | F | Update 6.5.1.1 and CSI-ReportConfig in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202488 | 0736 | - | F | Correction to the simulated cell for cell-reselection test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202496 | 0744 | - | F | Correction of RRM Test Case 6.4.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202497 | 0745 | - | F | Title Correction of Annex D.4.2.4 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202700 | 0723 | 1 | F | Addition of new RRM Test Case 4.5.4.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202701 | 0675 | 1 | F | Editorial correction of RRM TT in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202744 | 0708 | 1 | F | Correction to EN-DC FR1 radio link monitoring tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202745 | 0727 | 1 | F | Clarification cell2 SS-RSRP in 4.7.1.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202746 | 0587 | 1 | F | Addition to NR test case 5.5.5.5 scheduling available restriction | 16.4.0 |
| 2020-06 | RAN#88 | R5-202747 | 0707 | 1 | F | Correction on NR SA FR1 - E-UTRAN event-triggered reporting tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202748 | 0731 | 1 | F | Update 6.5.1.2 message content exception and test procedure | 16.4.0 |
| 2020-06 | RAN#88 | R5-202749 | 0737 | 1 | F | Correction of RRM Test Case 6.1.1.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202750 | 0738 | 1 | F | Correction of RRM Test Case 6.1.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202751 | 0621 | 1 | F | Update of TC 7.1.1.1 intra-freq cell re-selection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202752 | 0622 | 1 | F | Update of TC 7.1.1.2 inter-freq cell re-selection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202753 | 0724 | 1 | F | Addition of new RRM Test Cases in clause 7.5.3 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202754 | 0725 | 1 | F | Addition of new RRM Test Cases in clause 7.5.7 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202812 | 0685 | 1 | F | Correction to Test Applicability for 4.6.1.3 EN-DC FR1 event-triggered reporting with gap in non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202813 | 0686 | 1 | F | Correction to Test Applicability for 4.6.1.4 EN-DC FR1 event-triggered reporting with gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202814 | 0687 | 1 | F | Correction to Test Applicability & Message Exception for 4.6.1.6 EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202815 | 0688 | 1 | F | Correction to Test Applicability for 6.6.1.3 NR SA FR1 event-triggered reporting with gap in non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202816 | 0689 | 1 | F | Correction to Test Applicability for 6.6.1.4 NR SA FR1 event-triggered reporting with gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202817 | 0732 | 1 | F | Correction to Test Requirement for 4.4.3.1 by updating TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202833 | 0591 | 1 | F | Addition to NR test case 6.3.2.1.3 FR1-FR1 RRC re-establishment without serving cell timing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202834 | 0575 | 1 | F | Correction to NR test case 7.7.1.3.1 SA FR2 SS-RSRP absolute accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202835 | 0592 | 1 | F | Addition to NR test case 7.3.2.1.3 FR2-FR2 RRC re-establishment without serving cell timing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202836 | 0594 | 1 | F | Addition to NR test case 7.5.1.9 FR2 RLM scheduling restrictions | 16.4.0 |
| 2020-06 | RAN#88 | R5-202837 | 0578 | 1 | F | Correction to NR test case 8.3.1.1 handover to known cell | 16.4.0 |
| 2020-06 | RAN#88 | R5-202838 | 0597 | 1 | F | Addition of Default Configuration in Annex H | 16.4.0 |
| 2020-06 | RAN#88 | R5-202839 | 0628 | 1 | F | Update of Annex D.4.1.1 for parameters of BFD with 4RX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202840 | 0630 | 1 | F | Update of Cell mapping in Annex E | 16.4.0 |
| 2020-06 | RAN#88 | R5-202841 | 0633 | 1 | F | Add auxiliary bands for RRM inter-frequency SA tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202909 | 0697 | 1 | F | Correction to Test Requirements in 4.4.1.1, Table 4.4.1.1.5-5 to include TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202910 | 0698 | 1 | F | Correction to Test Requirements in 6.4.1.1, Table 6.4.1.1.5-5 to include TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202911 | 0734 | 1 | F | Correction to Test Requirement for 6.4.3.1 by updating TT values from Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-202912 | 0735 | 1 | F | Annex I RRM OTA procedures update to add RSRPB based UE Positioning Method for FR2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202913 | 0703 | 1 | F | Clarification of disabling Tx diversity for FR2 UE for FR2 RRM testing | 16.4.0 |
| 2020-06 | RAN#88 | R5-202914 | 0719 | 1 | F | Updates of FR2 MU and TT in TS 38.533 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202950 | 0693 | 1 | F | Correction to Message Exception for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202951 | 0694 | 1 | F | Correction to Message Exception for 4.6.2.6 EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection | 16.4.0 |
| 2020-06 | RAN#88 | R5-202961 | 0690 | 1 | F | Correction to Test Applicability and Message Exception for 6.6.1.6 NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading | 16.4.0 |
| 2020-06 | RAN#88 | R5-202969 | 0639 | 1 | F | Corrections to 4.7.2.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202970 | 0641 | 1 | F | Corrections to 4.7.3.1 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202971 | 0642 | 1 | F | Corrections to 4.7.3.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202972 | 0598 | 1 | F | Corrections to 6.7.1.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202973 | 0599 | 1 | F | Corrections to 6.7.2.1 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202974 | 0600 | 1 | F | Corrections to 6.7.2.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202975 | 0601 | 1 | F | Corrections to 6.7.3.1 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202976 | 0602 | 1 | F | Corrections to 6.7.3.2 and core spec alignment | 16.4.0 |
| 2020-06 | RAN#88 | R5-202977 | 0638 | 1 | F | Corrections to 4.7.1.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202978 | 0640 | 1 | F | Corrections to 4.7.2.2 | 16.4.0 |
| 2020-06 | RAN#88 | R5-202982 | 0710 | 1 | F | Correction to FR1 UE transmit timing accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202983 | 0709 | 1 | F | Correction to EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous ENDC | 16.4.0 |
| 2020-06 | RAN#88 | R5-202984 | 0711 | 1 | F | Correction to FR2 UE transmit timing accuracy | 16.4.0 |
| 2020-06 | RAN#88 | R5-202985 | 0705 | 1 | F | Core alignment in 6.6.1.2 NR SA FR1 event-triggered reporting without gap in DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-202986 | 0713 | 1 | F | Correction to NR E-UTRA reselection tests | 16.4.0 |
| 2020-06 | RAN#88 | R5-202987 | 0715 | 1 | F | Correction to NR SA FR1 handover test cases | 16.4.0 |
| 2020-06 | RAN#88 | R5-202996 | 0604 | 1 | F | Update FR1 Test Tolerance of 4.5.2.5 E-UTRAN SCC in sync | 16.4.0 |
| 2020-06 | RAN#88 | R5-202997 | 0605 | 1 | F | Update FR1 Test Tolerance of 4.5.2.6 E-UTRAN SCC in async | 16.4.0 |
| 2020-06 | RAN#88 | R5-202998 | 0606 | 1 | F | Update FR1 Test Tolerance of 4.5.3.1 SCell activation 160ms | 16.4.0 |
| 2020-06 | RAN#88 | R5-202999 | 0607 | 1 | F | Update FR1 Test Tolerance of 4.5.3.2 SCell activation 320ms | 16.4.0 |
| 2020-06 | RAN#88 | R5-203000 | 0608 | 1 | F | Update FR1 Test Tolerance of 4.5.3.3 unknown SCell activation | 16.4.0 |
| 2020-06 | RAN#88 | R5-203091 | 0623 | 1 | F | Update of 4.5.3.0 minimum requirements for Scell activation | 16.4.0 |
| 2020-06 | RAN#88 | R5-203092 | 0613 | 1 | F | Update FR1 Test Tolerance of 6.5.4.1 UL carrier RRC reconfiguration delay | 16.4.0 |
| 2020-06 | RAN#88 | R5-203093 | 0629 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.4.0 |
| 2020-06 | RAN#88 | R5-203095 | 0609 | 1 | F | Addition FR1 Test Tolerance of 6.5.5.1 SSB BFD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203096 | 0610 | 1 | F | Addition FR1 Test Tolerance of 6.5.5.2 SSB BFD DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203097 | 0611 | 1 | F | Update FR1 Test Tolerance of 6.5.5.3 CSI-RS BFD non-DRX | 16.4.0 |
| 2020-06 | RAN#88 | R5-203098 | 0612 | 1 | F | Update FR1 Test Tolerance of 6.5.5.4 CSI-RS BFD DRX | 16.4.0 |
| 2020-09 | RAN#89 | R5-203234 | 0747 | - | F | Update TC 8.5.2.1.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-203235 | 0748 | - | F | Update TC 8.5.2.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-203249 | 0758 | - | F | Annex E and F update with TT results for 8.5.2.x.x iRAT periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203250 | 0759 | - | F | Annex E and F update with TT results for 4.7.4.x.x EN-DC L1-RSRP periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203252 | 0761 | - | F | Void unnecessary TC 8.5.2.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203267 | 0765 | - | F | [EDITORIAL] Fix formatting issues throwing DOCX conversion errors in TS 38.533 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203268 | 0766 | - | F | Correction of test applicability for EN-DC event-triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203269 | 0767 | - | F | Correction of test applicability for NR SA event-triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203270 | 0768 | - | F | Adding annex table for iRAT measurement configuration | 16.5.0 |
| 2020-09 | RAN#89 | R5-203271 | 0769 | - | F | Clarification on the usage of auxiliary bands | 16.5.0 |
| 2020-09 | RAN#89 | R5-203272 | 0770 | - | F | Remove IE includeBeamMeasurements from 6.6.1.5 and 6.6.1.6 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203280 | 0772 | - | F | n26 addition to group of bands in 38.533 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203299 | 0773 | - | F | Corrected test description in non-DRX test case 6.5.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203300 | 0774 | - | F | Correction of comment for monitoringSymbolsWithinSlot | 16.5.0 |
| 2020-09 | RAN#89 | R5-203301 | 0775 | - | F | Correction of delta offset application for Cell 2 in test case 4.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203303 | 0777 | - | F | PUSCH allocation correction in test case 6.4.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203304 | 0778 | - | F | Correction to the RRC Reconfiguration message for measurements | 16.5.0 |
| 2020-09 | RAN#89 | R5-203306 | 0779 | - | F | Update extreme conditions limits in TC 4.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203307 | 0780 | - | F | Update extreme conditions limits in TC 6.7.1.2.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-203308 | 0781 | - | F | Update extreme conditions limits in TC 4.7.1.2.1 and TC 6.7.1.2.1 - Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-203309 | 0782 | - | F | Correction power offset for Band Group F | 16.5.0 |
| 2020-09 | RAN#89 | R5-203317 | 0786 | - | F | Removing SearchSpaceId and ControlResourceSetId modification | 16.5.0 |
| 2020-09 | RAN#89 | R5-203319 | 0788 | - | F | Unification of cell frequency for FR1-FR1 test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203732 | 0790 | - | F | Set ss-sinr request to false for EN-DC RRM measurement accuracy tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203733 | 0791 | - | F | Set ss-sinr request to false for NR SA RRM measurement accuracy tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-203832 | 0798 | - | F | Update of 4.5.6.1.0 minimum requirement for DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203835 | 0801 | - | F | Update of 4.5.6.2.0 minimum requirement for RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203837 | 0803 | - | F | Update of FR1 Test Tolerance in 6.3.2.1.3 RRC re-establishment without serving cell timing | 16.5.0 |
| 2020-09 | RAN#89 | R5-203838 | 0804 | - | F | Addition of 6.5.2.0 minimum requirement for interruptions during measurements on NR SCC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203839 | 0805 | - | F | Update of 6.5.2.1 interruptions during measurements on deactivated NR SCC | 16.5.0 |
| 2020-09 | RAN#89 | R5-203840 | 0806 | - | F | Addition of new Test Case 6.5.3.1 SCell activation and deactivation for 160ms cycle | 16.5.0 |
| 2020-09 | RAN#89 | R5-203841 | 0807 | - | F | Addition of new Test Case 6.5.3.2 SCell activation and deactivation for 320ms cycle | 16.5.0 |
| 2020-09 | RAN#89 | R5-203842 | 0808 | - | F | Addition of new Test Case 6.5.3.3 SCell activation and deactivation of unknown SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-203847 | 0813 | - | F | Update of 6.5.6.1.0 minimum requirement for DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203850 | 0816 | - | F | Update of 6.5.6.2.0 minimum requirement for RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-203853 | 0819 | - | F | Correction to FR1 intra-frequency measurement with gap test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-203855 | 0821 | - | F | Update of Cell mapping in Annex E | 16.5.0 |
| 2020-09 | RAN#89 | R5-204034 | 0827 | - | F | CR to 38.533 on RRM Test Cases in clause 7.5.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204036 | 0828 | - | F | CR to 38.533 on RRM Test Cases in clause 7.5.7 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204068 | 0829 | - | F | Correction to FR2 timing advance adjustment accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204069 | 0830 | - | F | Correction to EN-DC FR2 interruptions at transitions between active and non-active during DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204070 | 0831 | - | F | Addition of E-UTRA Test frequency selection rule for IRAT test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204071 | 0832 | - | F | Correction on event-triggered reporting with gap in DRX and DRX configuration settings for NR serving cell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204072 | 0833 | - | F | Correction to EN-DC and SA FR1-FR1 SS-RSRP and SS-RSRQ periodical measurement accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204073 | 0834 | - | F | Correction to SS-RSRQ periodical measurement accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204075 | 0836 | - | F | Correction to message configuration for radio link monitoring tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204076 | 0837 | - | F | Correction to NR SA FR1 E-UTRA RRC connection release with redirection | 16.5.0 |
| 2020-09 | RAN#89 | R5-204077 | 0838 | - | F | Correction to SRS PeriodicityAndOffset for Timing Advance tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204079 | 0840 | - | F | Correction to CSI-RS for tracking for TDD FR2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204116 | 0841 | - | F | Correction of EN-DC FR2 SSB-based L1-RSRP measurement test case 5.6.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204118 | 0843 | - | F | Correction of EN-DC FR2 CSI-RS-based L1-RSRP measurement test case 5.6.3.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204119 | 0844 | - | F | Correction of EN-DC FR2 CSI-RS-based L1-RSRP measurement test case 5.6.3.4 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204122 | 0847 | - | F | Correction of NR SA FR2 SSB-based L1-RSRP measurement test case 7.6.3.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204123 | 0848 | - | F | Correction of NR SA FR2 SSB-based L1-RSRP measurement test case 7.6.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204124 | 0849 | - | F | Correction of NR SA FR2 CSI-RS-based L1-RSRP measurement test case 7.6.3.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204125 | 0850 | - | F | Correction of NR SA FR2 CSI-RS-based L1-RSRP measurement test case 7.6.3.4 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204126 | 0851 | - | F | Correction of minimum conformance requirements 4.6.4.0 for EN-DC FR1 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204127 | 0852 | - | F | Correction of minimum conformance requirements 5.6.3.0 for EN-DC FR2 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204128 | 0853 | - | F | Correction of minimum conformance requirements 7.6.3.0 for SA FR2 L1-RSRP measurement test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204220 | 0854 | - | F | Update missing SMTC and SSB configurations in H.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204221 | 0855 | - | F | Update missing SMTC and SSB configurations in H.3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204222 | 0856 | - | F | Editorial: correct table placement in H.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204285 | 0859 | - | F | Correction to Message Exceptions for 4.6.2.5 EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection | 16.5.0 |
| 2020-09 | RAN#89 | R5-204337 | 0874 | - | F | Addition of 4.5.5.0 minimum requirement for BFR | 16.5.0 |
| 2020-09 | RAN#89 | R5-204730 | 0792 | 1 | F | Update of PDSCH RMC in Annex A.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204733 | 0793 | 1 | F | Correction of RRM Test Case 6.1.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204734 | 0824 | 1 | F | Correction of RRM Test Case 6.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204735 | 0825 | 1 | F | Correction of RRM Test Case 7.1.1.1 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204736 | 0826 | 1 | F | Correction of RRM Test Case 7.1.1.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204775 | 0751 | 1 | F | Update TC 4.7.4.1.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204776 | 0753 | 1 | F | Update TC 4.7.4.2.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204777 | 0771 | 1 | F | Correction of measurement gap configuration for intra-frequency even-triggered tests needing gap | 16.5.0 |
| 2020-09 | RAN#89 | R5-204778 | 0755 | 1 | F | Update TC 6.7.4.1.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204779 | 0757 | 1 | F | Update TC 6.7.4.2.2 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204780 | 0762 | 1 | F | Correction to the test procedure of NR SA event triggered tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204781 | 0763 | 1 | F | Correction to the test procedure of NR SA event triggered tests inter-freq | 16.5.0 |
| 2020-09 | RAN#89 | R5-204782 | 0764 | 1 | F | Correction to the test procedure of NR SA event triggered tests inter-RAT | 16.5.0 |
| 2020-09 | RAN#89 | R5-204783 | 0749 | 1 | F | Update TC 8.5.2.3.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204784 | 0760 | 1 | F | Annex E and F update with TT results for 6.7.4.x.x NR SA L1-RSRP periodic reporting test cases | 16.5.0 |
| 2020-09 | RAN#89 | R5-204785 | 0785 | 1 | F | Correction in A.1.4 CSI-RS for nzp-CSI-RS-ResourceId values | 16.5.0 |
| 2020-09 | RAN#89 | R5-204786 | 0820 | 1 | F | Update of FR1 Test Tolerance in Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-204787 | 0823 | 1 | F | Correction to default configurations in Annex H | 16.5.0 |
| 2020-09 | RAN#89 | R5-204872 | 0857 | 1 | F | Correction to Test Procedure and sync to latest RAN4 in 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204873 | 0858 | 1 | F | Correction to Test Procedure and sync to latest RAN4 in 6.4.1.1 NR SA FR1 UE transmit timing accuracy | 16.5.0 |
| 2020-09 | RAN#89 | R5-204874 | 0866 | 1 | F | Correction to 6.5.1.1 NR SA FR1 RLM OOS test with SSB-based RLM RS in non-DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204875 | 0867 | 1 | F | Correction to 6.5.1.2 NR SA FR1 RLM IS test with SSB-based RLM RS in non-DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204876 | 0868 | 1 | F | Correction to 6.5.1.3 NR SA FR1 RLM OOS test with SSB-based RLM RS in DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204877 | 0869 | 1 | F | Correction to 6.5.1.4 NR SA FR1 RLM IS test with SSB-based RLM RS in DRX mode, sync to latest RAN4 Core Spec | 16.5.0 |
| 2020-09 | RAN#89 | R5-204878 | 0750 | 1 | F | Update TC 4.7.4.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204879 | 0752 | 1 | F | Update TC 4.7.4.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204880 | 0754 | 1 | F | Update TC 6.7.4.1.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204881 | 0756 | 1 | F | Update TC 6.7.4.2.1 with TT results and message contents | 16.5.0 |
| 2020-09 | RAN#89 | R5-204882 | 0776 | 1 | F | Gap Pattern Id correction in section 6.6.2 to align to core spec 38.133 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204883 | 0787 | 1 | F | Test Procedure correction in test case 6.5.1.3 for definition of Point C | 16.5.0 |
| 2020-09 | RAN#89 | R5-204884 | 0789 | 1 | F | Addition of new RRM Test Cases in clause 7.5.5 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204885 | 0783 | 1 | F | CR to update the RRM FR2 MUs in Annex F | 16.5.0 |
| 2020-09 | RAN#89 | R5-204886 | 0784 | 1 | F | Add information on the AoA change during test for RRM FR2 tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204941 | 0835 | 1 | F | Correction to connection diagram for radio link monitoring tests | 16.5.0 |
| 2020-09 | RAN#89 | R5-204942 | 0842 | 1 | F | Correction of EN-DC FR2 SSB-based L1-RSRP measurement test case 5.6.3.2 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204943 | 0845 | 1 | F | Correction of FR2 condition tables in Annex B | 16.5.0 |
| 2020-09 | RAN#89 | R5-204944 | 0846 | 1 | F | Introduction of band n259 in clause 3 | 16.5.0 |
| 2020-09 | RAN#89 | R5-204985 | 0794 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.1 SSB based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204986 | 0795 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.2 SSB based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204987 | 0796 | 1 | F | Addition of new Test Case 4.5.5.3 CSI-RS based BFR non-DRX with FR1 Test Tolerance | 16.5.0 |
| 2020-09 | RAN#89 | R5-204988 | 0797 | 1 | F | Update of FR1 Test Tolerance in 4.5.5.4 CSI-RS based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204989 | 0799 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.1.1 DCI-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204990 | 0800 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.1.2 DCI-based BWP switch with SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204991 | 0802 | 1 | F | Update of FR1 Test Tolerance in 4.5.6.2.1 RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204992 | 0809 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.1 SSB based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204993 | 0810 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.2 SSB based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204994 | 0811 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.3 CSI-RS based BFR in non-DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204995 | 0812 | 1 | F | Update of FR1 Test Tolerance in 6.5.5.4 CSI-RS based BFR in DRX | 16.5.0 |
| 2020-09 | RAN#89 | R5-204996 | 0814 | 1 | F | Update of FR1 Test Tolerance in 6.5.6.1.1 DCI-based BWP switch with SCell | 16.5.0 |
| 2020-09 | RAN#89 | R5-204997 | 0815 | 1 | F | Addition of new Test Case 6.5.6.1.2 DCI-based BWP switch with FR1 Test Tolerance | 16.5.0 |
| 2020-09 | RAN#89 | R5-204998 | 0817 | 1 | F | Update of FR1 Test Tolerance in 6.5.6.2.1 RRC-based BWP switch | 16.5.0 |
| 2020-09 | RAN#89 | R5-204999 | 0818 | 1 | F | Update of test parameters in 6.3.1.4 inter-RAT handover | 16.5.0 |
| 2020-09 | RAN#89 | R5-205000 | 0822 | 1 | F | Update of default configurations for RMC scheduling in Annex A | 16.5.0 |
| 2020-12 | RAN#90 | R5-205051 | 0879 | - | F | Message contents for RRM iRAT | 16.6.0 |
| 2020-12 | RAN#90 | R5-205059 | 0884 | - | F | Message contents for RRM PSCell addition | 16.6.0 |
| 2020-12 | RAN#90 | R5-205063 | 0885 | - | F | Update RRM 8.5.2.1.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-205068 | 0890 | - | F | Correction of the measObjectID, measID and reportConfigID in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205070 | 0892 | - | F | Remove includeBeamMeasurements requirement for TC4.6.1.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205257 | 0907 | - | F | NR Band n53 addition to RRM group of bands | 16.6.0 |
| 2020-12 | RAN#90 | R5-205296 | 0914 | - | F | Add FR2 Fading MU values | 16.6.0 |
| 2020-12 | RAN#90 | R5-205813 | 0920 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205814 | 0921 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205815 | 0922 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205816 | 0923 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205817 | 0924 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205818 | 0925 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.6 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205819 | 0926 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.7 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205820 | 0927 | - | F | Correction of EN-DC FR2 Inter-frequency measurements test case 5.6.2.8 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205821 | 0928 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205822 | 0929 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205823 | 0930 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205824 | 0931 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205826 | 0933 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.6 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205827 | 0934 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.7 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205828 | 0935 | - | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.8 | 16.6.0 |
| 2020-12 | RAN#90 | R5-205834 | 0939 | - | F | Correction of message content exceptions in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205953 | 0946 | - | F | Update of FR1 TT in 8.4.1.1 inter-RAT SFTD in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-205954 | 0947 | - | F | Update of FR1 TT in 8.4.1.2 inter-RAT SFTD in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-205960 | 0953 | - | F | Addition of test case 5.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions | 16.6.0 |
| 2020-12 | RAN#90 | R5-205961 | 0954 | - | F | Update of test case 5.5.3.1 EN-DC FR2 SCell activation and deactivation | 16.6.0 |
| 2020-12 | RAN#90 | R5-205963 | 0956 | - | F | Update of 7.3.2.1.3 SA FR2 RRC re-establishment without serving cell timing | 16.6.0 |
| 2020-12 | RAN#90 | R5-205967 | 0960 | - | F | Update of Annex E for cell mapping | 16.6.0 |
| 2020-12 | RAN#90 | R5-205975 | 0964 | - | F | Correction to default configuration in Annex H | 16.6.0 |
| 2020-12 | RAN#90 | R5-205976 | 0965 | - | F | Correction to SSB reference configuration | 16.6.0 |
| 2020-12 | RAN#90 | R5-206100 | 0968 | - | F | Correction to EN-DC FR1 interruptions at transitions between active and non-active during DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206102 | 0970 | - | F | Correction to FR1 NSA SS-SINR measurement TCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206105 | 0973 | - | F | Correction to message exception for inter-RAT reselection TCs | 16.6.0 |
| 2020-12 | RAN#90 | R5-206106 | 0974 | - | F | Editorial correction of RSRP tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206111 | 0976 | - | F | Update of Annex C.1.3 of default connection setup | 16.6.0 |
| 2020-12 | RAN#90 | R5-206156 | 0977 | - | F | Addition of missing PRACH configuration 4 for FR1 and FR2 applicability | 16.6.0 |
| 2020-12 | RAN#90 | R5-206214 | 0979 | - | F | Alignment test parameter table with TS 38.133 in SA cell reselection test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206215 | 0980 | - | F | Alignment test parameter table with TS 38.133 in SA handover test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206216 | 0981 | - | F | Addition RLM IE into event trigger reporting CSI-RS based RLM | 16.6.0 |
| 2020-12 | RAN#90 | R5-206217 | 0982 | - | F | Addition RLM IE into event trigger reporting SSB based | 16.6.0 |
| 2020-12 | RAN#90 | R5-206219 | 0984 | - | F | Update message content section in SS-SINR test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206221 | 0985 | - | F | Update to Section 5.4.1.1 NSA FR2 Transmit Timing Accuracy | 16.6.0 |
| 2020-12 | RAN#90 | R5-206222 | 0986 | - | F | Update to Section 5.4.3.1 NSA FR2 Timing Advance Accuracy | 16.6.0 |
| 2020-12 | RAN#90 | R5-206606 | 0998 | - | F | Update of CSI-RS RMC configuration in Annex A | 16.6.0 |
| 2020-12 | RAN#90 | R5-206672 | 0918 | 1 | F | Editorial CR to clarify SS-RSRP dBm / 15 kHz for PRACH tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206673 | 0941 | 1 | F | Correction of RRM TC 4.7.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206674 | 0943 | 1 | F | Update of message contents for EN-DC RLM test case | 16.6.0 |
| 2020-12 | RAN#90 | R5-206675 | 0969 | 1 | F | Correction to FR1 Inter frequency Event triggered Reporting test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206677 | 0987 | 1 | F | Update to Message Exceptions for 4.3.2.2.2 EN-DC Non Contention Based Random Access | 16.6.0 |
| 2020-12 | RAN#90 | R5-206678 | 0989 | 1 | F | Update to 4.5.1.1 NSA FR1 OOS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206679 | 0990 | 1 | F | Update to 4.5.1.2 NSA FR1 IS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206680 | 0991 | 1 | F | Update to 4.5.1.3 NSA FR1 OOS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206681 | 0992 | 1 | F | Update to 4.5.1.4 NSA FR1 IS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206682 | 0993 | 1 | F | Update to 6.5.1.1 SA FR1 OOS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206683 | 0994 | 1 | F | Update to 6.5.1.2 SA FR1 IS RLM to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206684 | 0995 | 1 | F | Update to 6.5.1.3 SA FR1 OOS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206685 | 0996 | 1 | F | Update to 6.5.1.4 SA FR1 IS RLM in DRX to reword the Note for proper application of SNR for 4Rx Vs 2Rx | 16.6.0 |
| 2020-12 | RAN#90 | R5-206686 | 0952 | 1 | F | Update of 5.5.1 EN-DC FR2 radio link monitoring | 16.6.0 |
| 2020-12 | RAN#90 | R5-206687 | 0955 | 1 | F | Update of 5.5.5 EN-DC FR2 Beam failure detection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206688 | 0891 | 1 | F | Corrections to re-selection test procedure | 16.6.0 |
| 2020-12 | RAN#90 | R5-206689 | 0896 | 1 | F | Correction of RRM TC 6.1.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206690 | 0897 | 1 | F | Correction of RRM TC 6.1.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206691 | 0899 | 1 | F | Correction of RRM TC 6.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206692 | 0910 | 1 | F | Correction of RRM TC 6.5.5.3 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206693 | 0911 | 1 | F | Correction of RRM TC 6.5.5.4 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206694 | 0919 | 1 | F | Editorial CR to clarify SS-RSRP dBm / 15 kHz for SA PRACH tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206695 | 0942 | 1 | F | Correction of RRM TC 6.7.2.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206696 | 0961 | 1 | F | Update of FR1 TT in 6.1.1.1 intra-freq re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206697 | 0962 | 1 | F | Correction of clause 6.1 cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206698 | 0967 | 1 | F | Correction to NR SA RLM out-of-sync test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206699 | 0983 | 1 | F | Clarification in message exception for SA RLM out of synch test cases in non DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206700 | 0900 | 1 | F | Correction of RRM TC 7.1.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206701 | 0932 | 1 | F | Correction of NR SA FR2 Inter-frequency measurements test case 7.6.2.5 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206702 | 0936 | 1 | F | Correction of NR SA FR2 RRC re-establishment test case 7.3.2.1.1 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206703 | 0937 | 1 | F | Correction of NR SA FR2-FR2 RRC re-establishment test case 7.3.2.1.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206704 | 0957 | 1 | F | Update of clause 7.5.5 SA FR2 Beam failure detection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206705 | 0958 | 1 | F | Update of clause 7.6.1 SA FR2 Intra-frequency measurements | 16.6.0 |
| 2020-12 | RAN#90 | R5-206706 | 0889 | 1 | F | Correction of the message contents for RRM re-selection test cases | 16.6.0 |
| 2020-12 | RAN#90 | R5-206707 | 0917 | 1 | F | Correction to the aggregation level in the CORESET for RMC scheduling | 16.6.0 |
| 2020-12 | RAN#90 | R5-206708 | 0938 | 1 | F | Correction of clause 3.3 Abbreviations | 16.6.0 |
| 2020-12 | RAN#90 | R5-206709 | 0963 | 1 | F | Update of Annex H for cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206797 | 0882 | 1 | F | Complete RRM 4.5.7.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206798 | 0875 | 1 | F | Complete RRM 6.7.5.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206799 | 0916 | 1 | F | Add FR2 PRACH Test cases 7.3.2.2.1 and 7.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206800 | 0944 | 1 | F | Update of FR1 TT in 8.2.1.1 inter-RAT cell re-selection | 16.6.0 |
| 2020-12 | RAN#90 | R5-206801 | 0945 | 1 | F | Update of FR1 TT in 8.3.1.1 inter-RAT handover | 16.6.0 |
| 2020-12 | RAN#90 | R5-206802 | 0948 | 1 | F | Update of FR1 TT in 8.4.2.1 inter-RAT event-triggered reporting without SSB time index in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206803 | 0949 | 1 | F | Update of FR1 TT in 8.4.2.2 inter-RAT event-triggered reporting without SSB time index in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206804 | 0950 | 1 | F | Update of FR1 TT in 8.4.2.3 inter-RAT event-triggered reporting with SSB time index in non-DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206805 | 0951 | 1 | F | Update of FR1 TT in 8.4.2.4 inter-RAT event-triggered reporting with SSB time index in DRX | 16.6.0 |
| 2020-12 | RAN#90 | R5-206806 | 0878 | 1 | F | Annex E and F iRAT measurement accuracy tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206807 | 0888 | 1 | F | Annex E and F iRAT FR2 measurement accuracy tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206808 | 0959 | 1 | F | Update of Annex F for Test Tolerance | 16.6.0 |
| 2020-12 | RAN#90 | R5-206834 | 0915 | 1 | F | Update FR2 Downlink and Es/Noc MU values | 16.6.0 |
| 2020-12 | RAN#90 | R5-206843 | 0876 | 1 | F | Complete RRM 6.7.6.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206844 | 0886 | 1 | F | Update RRM 8.5.2.2.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206871 | 0894 | 1 | F | Correction of RRM TC 4.3.2.2.2 | 16.6.0 |
| 2020-12 | RAN#90 | R5-206872 | 0905 | 1 | F | New DRX configuration for RLM TC | 16.6.0 |
| 2020-12 | RAN#90 | R5-206895 | 0971 | 1 | F | Correction to message configuration for NSA radio link monitoring CSI-RS Based in DRX mode tests | 16.6.0 |
| 2020-12 | RAN#90 | R5-206906 | 0880 | 1 | F | Complete RRM 4.7.5.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206907 | 0877 | 1 | F | Complete RRM 6.7.7.1 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206908 | 0972 | 1 | F | Correction to message configuration for SA radio link monitoring CSI RS Based RLM RS | 16.6.0 |
| 2020-12 | RAN#90 | R5-206909 | 0887 | 1 | F | Update RRM 8.5.2.3.2 including TT analysis results | 16.6.0 |
| 2020-12 | RAN#90 | R5-206910 | 0881 | 1 | F | Annex E and F 4.7.5.1 SFTD measurement accuracy test | 16.6.0 |
| 2021-03 | RAN#91 | R5-210134 | 0999 | - | F | Update of first preamble power for EN-DC TC 4.3.2.2.1 and 4.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210135 | 1000 | - | F | Update of first preamble power for SA TC 6.3.2.2.1 and 6.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210136 | 1001 | - | F | Update of UE initial state for SA RLM TC 6.5.1.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210137 | 1002 | - | F | Update of process delay for SFTD measurement TC 8.4.1.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210138 | 1003 | - | F | Update of cell frequency for TC 6.7.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210171 | 1004 | - | F | Editorial update of SS-RSRP for TC 6.7.1.2.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210173 | 1006 | - | F | Update of RRC message for TC 6.4.3.1 and 6.5.2.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210176 | 1009 | - | F | Update of Scell activation and CSI reporting time for EN-DC TC 5.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210177 | 1010 | - | F | Update of CSI reporting time for SA TC 7.5.3.1 and 7.5.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210178 | 1011 | - | F | Update of PRACH configuration for SA TC 7.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210179 | 1012 | - | F | Update of PRACH configuration for EN-DC TC 4.5.5.3 and 4.5.5.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210180 | 1013 | - | F | Update of PRACH configuration for EN-DC TC 5.5.5.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210181 | 1014 | - | F | Update of PRACH configuration for SA TC 6.5.5.3 and 6.5.5.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210182 | 1015 | - | F | Update of PRACH configuration for SA TC 7.5.5.x | 16.7.0 |
| 2021-03 | RAN#91 | R5-210183 | 1016 | - | F | Update of prach-ConfigurationIndex for FR1 PRACH configuration 4 in A.7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210279 | 1017 | - | F | Addition of minimum conformance requirements for R16 NR HST cell re-selection in 6.1.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210281 | 1019 | - | F | Addition of minimum conformance requirements for R16 NR HST measurement in 6.6.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210283 | 1021 | - | F | Update of Annex H.2.1-3 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-210434 | 1025 | - | F | Correct cell mapping for 5.4.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210437 | 1027 | - | F | Annex E and F 4.5.7.1 PSCell addition test | 16.7.0 |
| 2021-03 | RAN#91 | R5-210440 | 1029 | - | F | Update of TT analysis results in Annex F for FR2 iRAT test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210442 | 1030 | - | F | Update TT results for SS-RSRP measurement accuracy test cases chapter 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210443 | 1031 | - | F | Update TT results for SS-RSRP measurement accuracy test cases chapter 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210444 | 1032 | - | F | Update TT results for SS-RSRP measurement accuracy test cases Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-210446 | 1033 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases chapter 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210447 | 1034 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases chapter 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210448 | 1035 | - | F | Update TT results for SS-RSRQ measurement accuracy test cases Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-210456 | 1039 | - | F | Update Annex I to TS 38.533 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210457 | 1040 | - | F | Update NR frequency band groups for FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210458 | 1041 | - | F | Correction RLM config for event triggered test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210465 | 1046 | - | F | Editorial correction of title of clause 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210466 | 1047 | - | F | Corrections to 6.3.2.2.1 and 6.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210467 | 1048 | - | F | Corrections to 4.3.2.2.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210470 | 1049 | - | F | Clarification on SSB Index to use in the PUxCH-PowerControl for RRM tests with more than one SSB | 16.7.0 |
| 2021-03 | RAN#91 | R5-210471 | 1050 | - | F | Clarification on SSB Index to use in the PUxCH-PowerControl for RRM tests with more than one SSB - EN-DC | 16.7.0 |
| 2021-03 | RAN#91 | R5-210472 | 1051 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - EN-DC FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210473 | 1052 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - SA FR1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210474 | 1053 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - SA FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210475 | 1054 | - | F | Editorial: correct title of PRACH test cases to match RRM work plan - Annexes | 16.7.0 |
| 2021-03 | RAN#91 | R5-210507 | 1056 | - | F | Correction of test applicability for long DRX cycle related test cases in section 4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210508 | 1057 | - | F | Correction of test applicability for long DRX cycle related test cases in section 5 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210509 | 1058 | - | F | Correction of test applicability for long DRX cycle related test cases in section 6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210510 | 1059 | - | F | Correction of test applicability for long DRX cycle related test cases in section 7 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210511 | 1060 | - | F | Correction of test applicability for long DRX cycle related test cases in section 8 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210528 | 1062 | - | F | Correction to test procedure in test case 6.5.1.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210529 | 1063 | - | F | Correction to Interruptions during measurements on deactivated NR SCC test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210530 | 1064 | - | F | Correction to message configuration for NSA CSI-RS-based RLM RS test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210531 | 1065 | - | F | Correction to NR SA RLM out-of-sync test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210532 | 1066 | - | F | Correction to NR SA FR1 RRC Re-establishment test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210533 | 1067 | - | F | Correction to 4.5.1.2 and 6.5.1.2 PDCCH Aggregation Level | 16.7.0 |
| 2021-03 | RAN#91 | R5-210535 | 1069 | - | F | Correction to FR1 NSA SS-SINR measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210536 | 1070 | - | F | Correction to L1-RSRP test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210537 | 1071 | - | F | Correction to the procedure to add a step for establishing SRB2 and DRB | 16.7.0 |
| 2021-03 | RAN#91 | R5-210538 | 1072 | - | F | Update of DRX configuration in FR1 Event-triggered Test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-210601 | 1073 | - | F | Clarification of BWP1 and BWP2 in 6.6.1.3, 6.6.1.4, 6.6.1.6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-210608 | 1077 | - | F | Correction in 4.5.2.1 and 4.5.2.2 test procedure | 16.7.0 |
| 2021-03 | RAN#91 | R5-210613 | 1079 | - | F | Correction in SIB5 for iRAT cell reselection | 16.7.0 |
| 2021-03 | RAN#91 | R5-210614 | 1080 | - | F | Correction in 6.1.2.2 test procedure | 16.7.0 |
| 2021-03 | RAN#91 | R5-210814 | 1085 | - | F | Finalise FR2 Timing MU values | 16.7.0 |
| 2021-03 | RAN#91 | R5-210816 | 1086 | - | F | Update of FR2 Tx Timing Test case 5.4.1.1 Test Tolerances | 16.7.0 |
| 2021-03 | RAN#91 | R5-210851 | 1089 | - | F | Update of FR1 TT for 4.5.5.1 and 4.5.5.2 SSB based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210852 | 1090 | - | F | Update of FR1 TT for 4.5.5.3 and 4.5.5.4 CSI-RS based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210853 | 1091 | - | F | Correction to 4.6.4.3 and 4.6.4.4 L1-RSRP reporting delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-210857 | 1095 | - | F | Update of FR1 TT for 6.5.5.1 and 6.5.5.2 SSB based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210858 | 1096 | - | F | Update of FR1 TT for 6.5.5.3 and 6.5.5.4 CSI-RS based LR | 16.7.0 |
| 2021-03 | RAN#91 | R5-210860 | 1098 | - | F | Correction to 6.6.4.3 and 6.6.4.4 L1-RSRP reporting delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-210863 | 1101 | - | F | Update of 6.5.3.1 SCell activation and deactivation | 16.7.0 |
| 2021-03 | RAN#91 | R5-210864 | 1102 | - | F | Update of D.4 antenna configuration | 16.7.0 |
| 2021-03 | RAN#91 | R5-210866 | 1104 | - | F | Correction to CSI-RS RMC | 16.7.0 |
| 2021-03 | RAN#91 | R5-210867 | 1105 | - | F | Correction to default configuration on L1-RSRP reporting in Annex H | 16.7.0 |
| 2021-03 | RAN#91 | R5-210959 | 1113 | - | F | Update to 6.5.4.1 NR SA FR1 UE UL carrier RRC reconfiguration delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-211049 | 1116 | - | F | Additional of default downlink level for FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211219 | 1124 | - | F | Correction of the minimum conformance requirements for Inter-RAT measurements 6.6.3.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211220 | 1125 | - | F | Addition of new RRM iRAT measurement Inter-RAT measurements test case 6.6.3.3 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211255 | 1131 | - | F | Clarification of BWP1 and BWP2 in 4.6.1.3, 4.6.1.4, 4.6.1.6 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211614 | 1007 | 1 | F | Update of Scell activation and CSI reporting time for EN-DC TC 4.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211615 | 1026 | 1 | F | Complete RRM 4.5.7.1 including TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211616 | 1087 | 1 | F | Update Test Tolerance for FR1 RLM Test Cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211617 | 1092 | 1 | F | Correction to EN-DC radio link monitoring | 16.7.0 |
| 2021-03 | RAN#91 | R5-211618 | 1093 | 1 | F | Update of 4.5.3.1 SCell activation and deactivation | 16.7.0 |
| 2021-03 | RAN#91 | R5-211619 | 1108 | 1 | F | Update to 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | 16.7.0 |
| 2021-03 | RAN#91 | R5-211620 | 1109 | 1 | F | Update to 4.7.5.1 EN-DC FR1 SFTD measurement accuracy | 16.7.0 |
| 2021-03 | RAN#91 | R5-211621 | 1117 | 1 | F | Correction of applied TT for EN-DC FR1 L1-RSRP measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211622 | 1023 | 1 | F | Update 5.4.3.1 with TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211623 | 1119 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.1 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211624 | 1120 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.3 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211625 | 1008 | 1 | F | Update of CSI reporting time for SA TC 6.5.3.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211626 | 1094 | 1 | F | Correction to SA radio link monitoring | 16.7.0 |
| 2021-03 | RAN#91 | R5-211627 | 1110 | 1 | F | Update to 6.3.2.2.1 Contention based random access test in FR1 for NR standalone | 16.7.0 |
| 2021-03 | RAN#91 | R5-211628 | 1111 | 1 | F | Update to 6.3.2.2.2 Non-Contention based random access test in FR1 for NR standalone | 16.7.0 |
| 2021-03 | RAN#91 | R5-211629 | 1112 | 1 | F | Update to 6.5.2.1 NR SA FR1 interruptions during measurements on deactivated NR SCC | 16.7.0 |
| 2021-03 | RAN#91 | R5-211630 | 1114 | 1 | F | Update to 6.6.3.1 NR SA FR1 - E-UTRAN event-triggered reporting in non-DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211631 | 1115 | 1 | F | Update to 6.6.3.2 NR SA FR1 - E-UTRAN event-triggered reporting in DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211632 | 1118 | 1 | F | Correction of applied TT for SA FR1 L1-RSRP measurement test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211633 | 1121 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.1 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211634 | 1122 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.3 including TT | 16.7.0 |
| 2021-03 | RAN#91 | R5-211635 | 1028 | 1 | F | Complete FR2 iRAT measurement accuracy test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211636 | 1024 | 1 | F | Update Annex F for 5.4.3.1 and 7.4.3.1 with TT analysis results | 16.7.0 |
| 2021-03 | RAN#91 | R5-211637 | 1038 | 1 | F | Update DL AWGN MU for RRM FR2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211638 | 1084 | 1 | F | Update FR2 Downlink and Uplink MU values | 16.7.0 |
| 2021-03 | RAN#91 | R5-211639 | 1103 | 1 | F | Update of Annex F for Test Tolerance | 16.7.0 |
| 2021-03 | RAN#91 | R5-211640 | 1123 | 1 | F | Correction of MTSU and applied TT in Annex F | 16.7.0 |
| 2021-03 | RAN#91 | R5-211641 | 1130 | 1 | F | Addition of new RMCs and OCNGs into Annex A | 16.7.0 |
| 2021-03 | RAN#91 | R5-211722 | 1042 | 1 | F | Correction EN-DC FR1 timing tests | 16.7.0 |
| 2021-03 | RAN#91 | R5-211723 | 1043 | 1 | F | Corrections to 5.4.1.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211724 | 1061 | 1 | F | Clarification of SNR for 4RX UE in RLM Test Cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211725 | 1076 | 1 | F | Correction in 4.7.3.2.1 test parameters | 16.7.0 |
| 2021-03 | RAN#91 | R5-211726 | 1005 | 1 | F | Update of RRC message for TC 6.3.1.4, 6.3.1.5 and 6.3.2.3.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211727 | 1045 | 1 | F | Corrections NR SA FR1 timing test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211728 | 1097 | 1 | F | Correction to NR TC 6.6.3.2-Inter-RAT DRX | 16.7.0 |
| 2021-03 | RAN#91 | R5-211729 | 1099 | 1 | F | Update of 6.5.1.2 and 6.5.1.4 SSB based RLM | 16.7.0 |
| 2021-03 | RAN#91 | R5-211730 | 1083 | 1 | F | Clarification of BWP1 and BWP2 in 7.6.1.3 and 7.6.1.4 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211731 | 1044 | 1 | F | Addition of Serving Cell Config for RRM timing test cases | 16.7.0 |
| 2021-03 | RAN#91 | R5-211827 | 1018 | 1 | F | Addition of new test case 6.1.1.7 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211828 | 1020 | 1 | F | Addition of new test case 6.6.1.7 for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211829 | 1022 | 1 | F | Addition of cell configuration mapping in Annex E for R16 NR HST | 16.7.0 |
| 2021-03 | RAN#91 | R5-211830 | 1126 | 1 | F | Correction of the minimum conformance requirements for E-UTRA - NR FR1 Cell reselection 8.2.1.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211831 | 1127 | 1 | F | Addition of new RRM E-UTRA - NR FR1 Cell reselection test case 8.2.1.2 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211832 | 1128 | 1 | F | Correction of the minimum conformance requirements for E-UTRA - NR Inter-RAT event triggered reporting tests 8.4.2.0 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211833 | 1129 | 1 | F | Addition of new RRM E-UTRA - NR Inter-RAT event triggered reporting test case 8.4.2.9 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211889 | 1036 | 1 | F | Update TT results for 6.3.1.1 | 16.7.0 |
| 2021-03 | RAN#91 | R5-211890 | 1037 | 1 | F | Update TT results for 6.3.1.1 Annex F | 16.7.0 |
| 2021-06 | RAN#92 | R5-212024 | 1132 | - | F | Complete 5.7.1.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212025 | 1133 | - | F | Complete 5.7.2.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212026 | 1134 | - | F | Complete 5.7.3.1 with TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-212256 | 1138 | - | F | Core alignment of TCI state of CSI-RS for TRS | 16.8.0 |
| 2021-06 | RAN#92 | R5-212257 | 1139 | - | F | Introducing additional band for n38 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212265 | 1147 | - | F | Updates to DRX configuration in FR1 Event-triggered Test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212266 | 1148 | - | F | Core alignment of BWP configuration parameter in 6.5.2.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212267 | 1149 | - | F | Correction to CSI-RS based L1-RSRP measurement tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-212612 | 1153 | - | F | Addition of new test case 6.6.4.5 for R16 NR HST | 16.8.0 |
| 2021-06 | RAN#92 | R5-212613 | 1154 | - | F | Addition of cell configuration mapping in Annex E for TC 6.6.4.5 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212712 | 1158 | - | F | Remove obsolete message contents tables after Chapter 7 alignment | 16.8.0 |
| 2021-06 | RAN#92 | R5-212715 | 1161 | - | F | Correcting measurement accuracy iRAT test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212716 | 1162 | - | F | Corrections to 5.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212717 | 1163 | - | F | Corrections to 5.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212718 | 1164 | - | F | Corrections to 4.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212719 | 1165 | - | F | Corrections to 4.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212720 | 1166 | - | F | Corrections to 6.4.1.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212721 | 1167 | - | F | Corrections to 6.4.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212726 | 1171 | - | F | Correct applicability of Interruption EN-DC test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212727 | 1172 | - | F | Correct applicability of Scell activation EN-DC test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212728 | 1173 | - | F | Correct applicability of Interruption SA test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212729 | 1174 | - | F | Correct applicability of Scell activation SA test cases with CA | 16.8.0 |
| 2021-06 | RAN#92 | R5-212731 | 1175 | - | F | Addition of RRM FR2 EN-DC PRACH test case 5.3.2.2.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212732 | 1176 | - | F | Addition of RRM FR2 EN-DC PRACH test case 5.3.2.2.2 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212737 | 1177 | - | F | Update to test applicability of RRM TC6.5.4.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-212745 | 1178 | - | F | Update to RRM Antenna connection for 4Rx capable Ues | 16.8.0 |
| 2021-06 | RAN#92 | R5-212895 | 1180 | - | F | Correction to FR1 EN-DC interruption during SCell measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212896 | 1181 | - | F | Correction to FR1 EN-DC beam failure recovery test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212900 | 1185 | - | F | Correction to FR1 NR SA TC 6.5.3.1-SCell activation | 16.8.0 |
| 2021-06 | RAN#92 | R5-212901 | 1186 | - | F | Correction to FR1 NR SA beam failure recovery test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212904 | 1189 | - | F | Correction to RMSI CORESET reference configuration in Annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212905 | 1190 | - | F | Correction to CSI-RS reference configuration in annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212906 | 1191 | - | F | Correction to TRS reference configuration in annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212909 | 1194 | - | F | Correction to BWP reference configuration in Annex A | 16.8.0 |
| 2021-06 | RAN#92 | R5-212910 | 1195 | - | F | Correction of minimum requirements for L1-RSRP reporting test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-212911 | 1196 | - | F | Addition of FR1 EN-DC TC 4.6.4.5-HST L1-RSRP | 16.8.0 |
| 2021-06 | RAN#92 | R5-213183 | 1205 | - | F | Extension of Annex I for EIS spherical coverage points | 16.8.0 |
| 2021-06 | RAN#92 | R5-213227 | 1212 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.5 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213228 | 1213 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.6 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213229 | 1214 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.7 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213230 | 1215 | - | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.8 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213240 | 1219 | - | F | Test frequency selection for Inter-frequency RRM test cases in Annex E | 16.8.0 |
| 2021-06 | RAN#92 | R5-213345 | 1222 | - | F | Update on iRAT measurement 6.6.3.2 test case | 16.8.0 |
| 2021-06 | RAN#92 | R5-213346 | 1223 | - | F | Update on Annex H.3.1 for iRAT measurement | 16.8.0 |
| 2021-06 | RAN#92 | R5-213351 | 1224 | - | F | Correction to Coreset and Search Space RMC for FR2 RRM | 16.8.0 |
| 2021-06 | RAN#92 | R5-213352 | 1225 | - | F | Correction to Message Exceptions for 6.3.2.2.2 Non-contention based random access test in FR1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-213353 | 1226 | - | F | Correction to PDSCH RMC when DRX is configured | 16.8.0 |
| 2021-06 | RAN#92 | R5-213354 | 1227 | - | F | Correction to intra and inter frequency reselection test cases with asynchronous cells | 16.8.0 |
| 2021-06 | RAN#92 | R5-213847 | 1179 | 1 | F | Addition of inter-frequency measurements configuration for FR1 EN-DC BFD and RLM test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213848 | 1183 | 1 | F | Addition of inter-frequency measurements configuration for FR1 NR SA BFD and RLM test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213849 | 1184 | 1 | F | Correction to FR1 NR SA interruption during SCell measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213850 | 1193 | 1 | F | Correction to PRACH reference configuration in Annex H | 16.8.0 |
| 2021-06 | RAN#92 | R5-213921 | 1145 | 1 | F | Correction to test procedure for SSB-based L1-RSRP measurement reporting | 16.8.0 |
| 2021-06 | RAN#92 | R5-213922 | 1159 | 1 | F | Remove obsolete message contents tables after Chapter 7 alignment - EN-DC | 16.8.0 |
| 2021-06 | RAN#92 | R5-213923 | 1168 | 1 | F | Corrections to RLM test cases EN-DC | 16.8.0 |
| 2021-06 | RAN#92 | R5-213924 | 1141 | 1 | F | Correction to FR2 RRM test channel reference for Event triggered reporting test | 16.8.0 |
| 2021-06 | RAN#92 | R5-213925 | 1142 | 1 | F | Correction to FR2 RRM test channel reference for RLM and BFD tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-213926 | 1143 | 1 | F | Correction to FR2 RRM test channel reference for UL timing test | 16.8.0 |
| 2021-06 | RAN#92 | R5-213927 | 1208 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.5 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213928 | 1209 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.6 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213929 | 1210 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.7 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213930 | 1211 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.8 including TT | 16.8.0 |
| 2021-06 | RAN#92 | R5-213931 | 1156 | 1 | F | Correct reference to cell mapping for SA test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-213932 | 1160 | 1 | F | Correcting typos in PRACH test case | 16.8.0 |
| 2021-06 | RAN#92 | R5-213933 | 1169 | 1 | F | Corrections to RLM test cases SA | 16.8.0 |
| 2021-06 | RAN#92 | R5-213934 | 1135 | 1 | F | Annex F for EN-DC and SA FR2 Measurement Accuracy | 16.8.0 |
| 2021-06 | RAN#92 | R5-213935 | 1157 | 1 | F | Align Annex H of TS 38.533 to Clause 7 of TS 38.508-1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-213936 | 1202 | 1 | F | Correction to AoA Test Setup applicability per permitted test method | 16.8.0 |
| 2021-06 | RAN#92 | R5-213937 | 1204 | 1 | F | Adapt Annex F to indicate applicability of the RRM FR2 TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-213938 | 1207 | 1 | F | Correction of MTSU and applied TT in Annex F for FR2 inter-freq measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214009 | 1151 | 1 | F | Addition of 5G-SRVCC RRM TC 6.3.1.6-handover | 16.8.0 |
| 2021-06 | RAN#92 | R5-214010 | 1152 | 1 | F | Addition of 5G-SRVCC RRM TC 6.6.5.1-envent triggered reporting non-DRX | 16.8.0 |
| 2021-06 | RAN#92 | R5-214060 | 1182 | 1 | F | Correction to FR1 EN-DC BWP switching test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214061 | 1198 | 1 | F | Update of CSI reporting time for EN-DC FR1 TC 4.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214062 | 1199 | 1 | F | Update of CSI reporting time for EN-DC FR2 TC 5.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214063 | 1203 | 1 | F | Adapt RRM FR2 test cases to indicate applicability of the TT analysis results | 16.8.0 |
| 2021-06 | RAN#92 | R5-214064 | 1146 | 1 | F | Corrections to NR SA FR1 RRC Re-establishment tests | 16.8.0 |
| 2021-06 | RAN#92 | R5-214065 | 1187 | 1 | F | Correction to FR1 NR SA BWP switch test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214066 | 1200 | 1 | F | Update of CSI reporting time for SA FR1 TC 6.5.3.1 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214067 | 1201 | 1 | F | Update of CSI reporting time for SA FR2 TC 7.5.3.1 and 7.5.3.2 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214068 | 1188 | 1 | F | Correction to NR inter-RAT measurement test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214090 | 1197 | 1 | F | Correction to Annex A.9 on the selection of test direction | 16.8.0 |
| 2021-06 | RAN#92 | R5-214091 | 1206 | 1 | F | Extension of the RSRPB Rx Beak Peak Search method | 16.8.0 |
| 2021-06 | RAN#92 | R5-214092 | 1221 | 1 | F | Update on test frequencies selection for inter-frequency test cases | 16.8.0 |
| 2021-06 | RAN#92 | R5-214108 | 1216 | 1 | F | Correction of EN-DC FR2 inter-freq measurement test case 5.6.2.1 and 5.6.2.3 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214109 | 1217 | 1 | F | Correction of SA FR2 inter-freq measurement test case 7.6.2.1 and 7.6.2.3 | 16.8.0 |
| 2021-06 | RAN#92 | R5-214111 | 1144 | 1 | F | Correction to Interruptions during measurements on deactivated NR SCC | 16.8.0 |
| 2021-06 | RAN#92 | R5-214113 | 1140 | 1 | F | Correction to FR2 event-triggered reporting of intra frequency cell | 16.8.0 |