| 3GPP TS 38.533 V17.7.0 (2023-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	36

1	Scope	37

2	References	37

3	Definitions, symbols and abbreviations	38

3.1	Definitions	38

3.2	Symbols	39

3.3	Abbreviations	40

3A	Requirements for the support of RRM	42

3A.1	General	42

3A.1.0	Overview of RRM requirements	42

3A.1.1	Test coverage across 5G NR connectivity options	43

3A.2	Requirements Classification for Statistical Testing	43

3A.3	Antenna Configuration	43

3A.4	NR band groups	44

3A.4.0	General	44

3A.4.1	NR operating bands in FR1	44

3A.4.2	NR operating bands in FR2	45

3A.5	NR operating band configuration	46

3A.6	UE with Multiband Capability	47

4	EN-DC with all NR cells in FR1	47

4.0	General	47

4.1	Void	48

4.2	Void	48

4.3	RRC_CONNECTED state mobility	48

4.3.1	Void	48

4.3.2	RRC connection mobility control	48

4.3.2.1	Void	48

4.3.2.2	Random access	48

4.3.2.2.1	EN-DC FR1 contention based random access	48

4.3.2.2.2	EN-DC FR1 non-contention based random access	54

4.3.2.2.3	EN-DC FR1 2-step contention based random access	61

4.3.2.2.4	EN-DC FR1 2-step non-contention based random access	66

4.3.2.3	Void	72

4.4	Timing	72

4.4.1	UE transmit timing	72

4.4.1.0	Minimum conformance requirements	72

4.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	72

4.4.1.1	EN-DC FR1 UE transmit timing accuracy	73

4.4.2	UE timer accuracy	79

4.4.3	Timing advance	79

4.4.3.0	Minimum conformance requirements	79

4.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	79

4.4.3.0.2	Minimum conformance requirements for timing advance adjustment delay	79

4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	80

4.5	Signaling characteristics	85

4.5.1	Radio link monitoring	85

4.5.1.00	General	85

4.5.1.0	Minimum conformance requirements	85

4.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	85

4.5.1.0.2	Void	86

4.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	86

4.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	87

4.5.1.0.5	Requirements for UE configured with Relaxed Measurement Criteria	89

4.5.1.1	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	90

4.5.1.2	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	95

4.5.1.3	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	100

4.5.1.4	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	105

4.5.1.5	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	110

4.5.1.6	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	115

4.5.1.7	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	120

4.5.1.9	EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	131

4.5.2	Interruption	137

4.5.2.0	Minimum conformance requirements	137

4.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX.	137

4.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	138

4.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	139

4.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	139

4.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	141

4.5.2.1	EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	142

4.5.2.2	EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	145

4.5.2.3	EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	149

4.5.2.4	EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	156

4.5.2.5	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	162

4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	168

4.5.2.7	172

4.5.2.8	EN-DC FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	173

4.5.2.9	EN-DC FR1 interruptions at E-UTRA SRS carrier based switching	176

4.5.3	SCell activation and deactivation delay	181

4.5.3.0	Minimum conformance requirements	181

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	185

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	198

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX	199

4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	202

4.5.4	UE UL carrier RRC reconfiguration delay	211

4.5.4.1	EN-DC FR1 UE UL carrier RRC reconfiguration delay	211

4.5.5	Link recovery procedures	219

4.5.5.0	Minimum conformance requirements	219

4.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	221

4.5.5.0.4	Requirements for Beam Failure Recovery in SCell	222

4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX	222

4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX	228

4.5.5.3	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	235

4.5.5.4	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX	241

4.5.5.5	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	248

4.5.5.6	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	254

4.5.5.7	EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX	263

4.5.5.8	EN-DC FR1 SCell with TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	274

4.5.6	Active BWP switch delay	284

4.5.6.1	DCI-based and time-based active BWP switch	284

4.5.6.1.0	Minimum conformance requirements	284

4.5.6.1.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	287

4.5.6.1.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	294

4.5.6.2	RRC-based active BWP switch	304

4.5.6.2.0	Minimum conformance requirements	304

4.5.6.2.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	304

4.5.7	PSCell addition and release delay	310

4.5.7.0	Minimum conformance requirements	310

4.5.7.0.1	NR PSCell Addition Delay Requirement	310

4.5.7.0.2	NR PSCell Release Delay Requirement	311

4.5.7.1	EN-DC FR1 addition and release delay of known PSCell	311

4.5.8	UL switching	316

4.5.8.0	Minimum conformance requirements	316

4.5.8.1	EN-DC FR1 interruptions at switching between two uplink carriers	317

4.6	Measurement procedures	327

4.6.1	Intra-frequency measurements	327

4.6.1.0	Minimum conformance requirements	327

4.6.1.0.1	Minimum conformance requirements for event-triggered reporting without gap	327

4.6.1.0.2	Minimum conformance requirements for event-triggered measurements with gap	332

4.6.1.1	EN-DC FR1 event-triggered reporting without gap in non-DRX	334

4.6.1.2	EN-DC FR1 event-triggered reporting without gap in DRX	339

4.6.1.3	EN-DC FR1 event-triggered reporting with gap in non-DRX	342

4.6.1.4	EN-DC FR1 event-triggered reporting with gap in DRX	348

4.6.1.5	EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection	353

4.6.1.6	EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection	357

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	361

4.6.1.8	EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17	366

4.6.2	Inter-frequency measurements	371

4.6.2.0	Minimum conformance requirements for Inter-frequency measurements	371

4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX	374

4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX	379

4.6.2.3	Void	385

4.6.2.4	Void	385

4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	385

4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection	390

4.6.2.7	Void	395

4.6.2.8	Void	395

4.6.2.9	EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	395

4.6.3	Void	401

4.6.4	L1-RSRP measurement for beam reporting	401

4.6.4.0	Minimum conformance requirements	401

4.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	401

4.6.4.1	EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX	405

4.6.4.1.5	Test requirement	408

4.6.4.2	EN-DC FR1 SSB-based L1-RSRP measurement in DRX	409

4.6.4.2.3	Minimum conformance requirements	409

4.6.4.3	EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX	412

4.6.4.4	EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX	416

4.6.4.5	EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	420

4.6.5	CLI measurements	424

4.6.5.0	Minimum conformance requirements	424

4.6.5.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	425

4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX	426

4.6.5.1.3	Minimum conformance requirements	426

4.6.6	434

4.6.7	L1-SINR measurement for beam reporting	434

4.6.7.0	Minimum conformance requirements	434

4.6.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	439

4.6.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	444

4.6.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	450

4.7	Measurement performance requirements	455

4.7.1	SS-RSRP	455

4.7.1.0	Minimum conformance requirements	455

4.7.1.1	Intra-frequency measurements	460

4.7.1.2	Inter-frequency measurements	469

4.7.2	SS-RSRQ	476

4.7.2.0	Minimum conformance requirements	476

4.7.2.1	EN-DC FR1 SS-RSRQ measurement accuracy	478

4.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	482

4.7.2.2.1	EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy	482

4.7.2.2.2	EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy	486

4.7.3	SS-SINR	488

4.7.3.0	Minimum conformance requirements	488

4.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	488

4.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	489

4.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	490

4.7.3.1	EN-DC FR1 SS-SINR measurement accuracy	491

4.7.3.2	Inter-Frequency SS-SINR measurement accuracy	495

4.7.3.2.1	EN-DC FR1-FR1 SS-SINR absolute measurement accuracy	495

4.7.3.2.2	EN-DC FR1-FR1 SS-SINR relative measurement accuracy	498

4.7.4	L1-RSRP	500

4.7.4.0	Minimum conformance requirements	500

4.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	500

4.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	503

4.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	504

4.7.4.1	SSB based L1-RSRP measurements	505

4.7.4.1.1	EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy	505

4.7.4.1.2	EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy	510

4.7.4.2	CSI-RS based L1-RSRP measurements	512

4.7.4.2.1	EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy	512

4.7.4.2.2	EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy	516

4.7.5	SFTD	518

4.7.5.0	Minimum conformance requirements	518

4.7.5.0.1	SFTD Accuracy Requirement	518

4.7.5.1	EN-DC FR1 SFTD measurement accuracy	519

4.7.6	CLI measurements	523

4.7.6.0	Minimum conformance requirements	523

4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy	523

4.7.6.0.1.1	SRS-RSRP report mapping	524

4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	525

4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	525

4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	536

4.7.7	L1-SINR measurement for beam reporting	540

4.7.7.0	Minimum conformance requirements	540

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	540

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	543

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	545

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	549

4.7.7.1.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	549

4.7.7.1.2	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	554

4.7.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	556

4.7.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement	561

4.7.7.3.1	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	561

4.7.7.3.2	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	567

4A	NE-DC with all NR cells in FR1	569

4A.0	General	569

4A.1	Signalling characteristics	569

4A.1.1	E-UTRAN PSCell addition	569

4A.1.1.0	Minimum conformance requirements	569

4A.1.1.0.1	E-UTRA PSCell Addition Delay Requirement	569

4A.1.1.0.2	E-UTRA PSCell Release Delay Requirement	570

4A.1.1.1	NE-DC FR1 addition and release delay of known PSCell	570

4A.1.2	Active BWP switch delay	576

4A.1.2.0	Minimum conformance requirements	576

4A.1.2.1	NE-DC FR1 DCI-based and timer-based DL active BWP switch in non-DRX in synchronous NE-DC	576

4A.2	Measurement performance requirements	581

4A.2.1	SFTD accuracy	581

4A.2.1.0	Minimum conformance requirements	581

4A.1.1.0.1	NE-DC SFTD accuracy Requirement	581

4A.2.1.1	NE-DC FR1 SFTD accuracy	583

5	EN-DC with at least one NR cell in FR2	590

5.0	General	590

5.1	Void	590

5.2	Void	590

5.3	RRC_CONNECTED state mobility	590

5.3.1	Void	590

5.3.2	RRC connection mobility control	590

5.3.2.1	Void	590

5.3.2.2	Random access	590

5.3.2.2.1	EN-DC FR2 contention based random access	590

5.3.2.2.2	EN-DC FR2 non-contention based random access	598

5.3.2.2.3	EN-DC FR2 2-step contention based random access	606

5.3.2.2.4	EN-DC FR2 2-step non-contention based random access	610

5.3.2.3	Void	614

5.4	Timing	614

5.4.1	UE transmit timing	614

5.4.1.0	Minimum Conformance Requirements	614

5.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	614

5.4.1.1	EN-DC FR2 UE transmit timing accuracy	616

5.4.2	UE timer accuracy	622

5.4.3	Timing advance	622

5.4.3.0	Minimum conformance requirements	622

5.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	622

5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	622

5.5	Signaling characteristics	627

5.5.1	Radio link monitoring	627

5.5.1.0	Minimum conformance requirements	627

5.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	627

5.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	628

5.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	629

5.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	632

5.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	635

5.5.1.0.6	Requirements for UE configured with Relaxed Measurement Criteria	636

5.5.1.1	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	636

5.5.1.2	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	643

5.5.1.3	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	648

5.5.1.4	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	654

5.5.1.5	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	659

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	664

5.5.1.7	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	669

5.5.1.9	EN-DC FR2 radio link monitoring UE scheduling restrictions	682

5.5.1.10	EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	687

5.5.2	Interruption	692

5.5.2.0	Minimum conformance requirements	692

5.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX	692

5.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	693

5.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	694

5.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	694

5.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	696

5.5.2.1	EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	696

5.5.2.2	EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	701

5.5.2.3	EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	705

5.5.2.4	EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	711

5.5.2.5	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	716

5.5.2.6	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	720

5.5.2.7	EN-DC FR2 interruptions at E-UTRA SRS carrier based switching	724

5.5.2.8	EN-DC FR2 interruptions at NR SRS carrier based switching	728

5.5.3	SCell activation and deactivation delay	732

5.5.3.1	EN-DC FR2 SCell activation and deactivation intra-band in non-DRX	732

5.5.3.2 to 5.5.3.6	740

5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	740

5.5.4	UE UL carrier RRC reconfiguration delay	744

5.5.5	Link recovery procedures	744

5.5.5.0	Minimum conformance requirements	744

5.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	744

5.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	748

5.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	754

5.5.5.0.4	Requirements for Beam Failure Recovery in SCell	755

5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX	755

5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX	762

5.5.5.3	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	769

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX	775

5.5.5.5	EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX	782

5.5.5.6	EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX	787

5.5.5.7	EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX	794

5.5.5.8	EN-DC FR2 PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode	800

5.5.6	Active BWP switch delay	807

5.5.6.1	DCI-based and time-based active BWP switch	807

5.5.6.1.0	Minimum conformance requirements	807

5.5.6.1.1	EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	807

5.5.6.1.2	EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	808

5.5.6.2	RRC-based active BWP switch	808

5.5.6.2.0	Minimum conformance requirements	808

5.5.6.2.1	EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	809

5.5.7	Void	816

5.5.8	Active TCI state switch delay	816

5.5.8.0	Minimum conformance requirements	816

5.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	816

5.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	817

5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch	818

5.5.8.2	EN-DC FR2 RRC based active TCI state switch	822

5.6	Measurement procedures	825

5.6.1	Intra-frequency measurements	825

5.6.1.0	Minimum conformance requirements	826

5.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	826

5.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	828

5.6.1.1	EN-DC FR2 event-triggered reporting without gap in non-DRX	830

5.6.1.2	EN-DC FR2 event-triggered reporting without gap in DRX	835

5.6.1.3	EN-DC FR2 event-triggered reporting with gap in non-DRX	839

5.6.1.4	EN-DC FR2 event-triggered reporting with gap in DRX	845

5.6.2	Inter-frequency measurements	850

5.6.2.0	Minimum conformance requirements for Inter-frequency measurements	850

5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX	852

5.6.2.2	EN-DC FR2-FR2 event-triggered reporting in DRX	856

5.6.2.3	EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	861

5.6.2.4	EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection	865

5.6.2.5	EN-DC FR1-FR2 event-triggered reporting in non-DRX	870

5.6.2.6	EN-DC FR1-FR2 event-triggered reporting in DRX	875

5.6.2.7	EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	881

5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection	886

5.6.3	L1-RSRP measurement for beam reporting	892

5.6.3.0	Minimum conformance requirements	892

5.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	892

5.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	893

5.6.3.1	EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX	896

5.6.3.2	EN-DC FR2 SSB-based L1-RSRP measurement in DRX	900

5.6.3.3	EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX	904

5.6.3.4	EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX	908

5.6.4	CLI measurements	911

5.6.4.0	Minimum conformance requirements	911

5.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	912

5.6.4.1	EN-DC FR2 SRS-RSRP measurement in non-DRX	913

5.6.4.2	EN-DC FR2 CLI-RSSI measurement in non-DRX	916

5.6.5	920

5.6.6	L1-SINR measurement for beam reporting	920

5.6.6.0	Minimum conformance requirements	920

5.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	920

5.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	923

5.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	925

5.6.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	927

5.6.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	932

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	937

5.7	Measurement performance requirements	942

5.7.1	SS-RSRP	942

5.7.1.0	Minimum conformance requirements	942

5.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	942

5.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	943

5.7.1.1	EN-DC FR2 SS-RSRP measurement accuracy	945

5.7.1.2	EN-DC FR2-FR2 SS-RSRP measurement accuracy	952

5.7.1.3	EN-DC FR1-FR2 SS-RSRP measurement accuracy	959

5.7.2	SS-RSRQ	965

5.7.2.0	Minimum conformance requirements	965

5.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	965

5.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	965

5.7.2.1	EN-DC FR2 SS-RSRQ measurement accuracy	967

5.7.2.2	EN-DC FR2-FR2 SS-RSRQ measurement accuracy	971

5.7.3	SS-SINR	976

5.7.3.0	Minimum conformance requirements	976

5.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	976

5.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	976

5.7.3.1	EN-DC FR2 SS-SINR measurement accuracy	978

5.7.3.2	EN-DC FR2-FR2 SS-SINR measurement accuracy	982

5.7.4	L1-RSRP	987

5.7.4.0	Minimum conformance requirements	987

5.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	988

5.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	988

5.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	989

5.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	990

5.7.4.1	EN-DC FR2 SSB based L1-RSRP measurement accuracy	991

5.7.4.2	EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy	997

5.7.5	SRS-RSRP	1003

5.7.5.0	Minimum conformance requirements	1003

5.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	1003

5.7.5.1	EN-DC FR2 SRS-RSRP measurement accuracy	1005

5.7.5.2	EN-DC FR2 CLI-RSSI measurement accuracy	1010

5.7.5.2.1	Test purpose	1011

5.7.5.2.2	Test applicability	1011

5.7.5.2.3	Minimum conformance requirements	1011

5.7.5.2.4	Test description	1011

5.7.5.2.5	Test requirement	1012

5.7.6	L1-SINR measurement for beam reporting	1015

5.7.6.0	Minimum conformance requirements	1015

5.7.6.0.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1015

5.7.6.0.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1016

5.7.6.0.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1019

5.7.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	1021

5.7.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1026

5.7.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1031

6	NR standalone in FR1	1037

6.0	General	1037

6.1	RRC_IDLE state mobility	1037

6.1.1	NR cell re-selection	1037

6.1.1.0	Minimum conformance requirements	1037

6.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	1037

6.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	1037

6.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with highSpeedMeasFlag-r16	1040

6.1.1.0.4	Minimum conformance requirements for intra-frequency cell re-selection when UE configured with relaxed measurement criterion	1040

6.1.1.0.5	Minimum conformance requirements for inter-frequency cell re-selection when UE configured with relaxed measurement criterion	1042

6.1.1.1	NR SA FR1 cell re-selection	1044

6.1.1.1.5	Test requirement	1048

6.1.1.2	NR SA FR1-FR1 cell re-selection	1050

6.1.1.3	NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1054

6.1.1.4	NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1061

6.1.1.5	NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1067

6.1.1.6	NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1073

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16	1079

6.1.1.8	NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17	1084

6.1.2	NR – E-UTRA cell re-selection	1090

6.1.2.0	Minimum conformance requirements	1090

6.1.2.0.1	Minimum conformance requirements for NR – E-UTRA cell re-selection	1090

6.1.2.0.2	Minimum conformance requirement for inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	1092

6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA	1093

6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA	1099

6.1.2.3	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1105

6.1.2.4	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1111

6.1.2.5	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16	1116

6.2	RRC_INACTIVE state mobility	1122

6.2.0	General	1122

6.2.0.1	Minimum conformance requirements for TA validation using CG-SDT	1122

6.2.1	NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)	1123

6.2.1.1	Test Purpose	1123

6.2.1.2	Test Applicability	1123

6.2.1.3	Minimum Conformance Requirement	1123

6.2.1.4	Test Description	1123

6.2.1.4.1	Initial conditions	1124

6.2.1.4.2	Test Procedure	1125

6.2.1.5	Test Requirement	1128

6.3	RRC_CONNECTED state mobility	1128

6.3.1	Handover	1128

6.3.1.0	Minimum conformance requirements	1128

6.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	1128

6.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	1129

6.3.1.0.3	Minimum conformance requirements for NR – UTRAN handover	1130

6.3.1.0.4	Minimum conformance requirements for NR FR1- NR FR1 DAPS handover	1130

6.3.1.1	NR SA FR1 handover with known target cell	1133

6.3.1.2	NR SA FR1 handover with unknown target cell	1137

6.3.1.3	NR SA FR1-FR1 handover with unknown target cell	1140

6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell	1143

6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell	1150

6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell	1155

6.3.1.7	NR SA FR1 synchronous DAPS handover	1161

6.3.1.8	NR SA FR1 asynchronous DAPS handover	1169

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover	1174

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover	1183

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover	1189

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover	1198

6.3.2	RRC connection mobility control	1206

6.3.2.1	RRC re-establishment	1206

6.3.2.1.0	Minimum conformance requirements	1206

6.3.2.1.1	NR SA FR1 RRC re-establishment	1208

6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment	1212

6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing	1216

6.3.2.2	Random access	1220

6.3.2.2.0	Minimum conformance requirements	1220

6.3.2.2.1	NR SA FR1 contention based random access	1222

6.3.2.2.2	NR SA FR1 non-contention based random access	1228

6.3.2.2.3	NR SA FR1 2-step contention based random access	1235

6.3.2.2.4	NR SA FR1 2-step non-contention based random access	1239

6.3.2.3	RRC connection release with redirection	1243

6.3.2.3.0	Minimum conformance requirements	1243

6.3.2.3.1	NR SA FR1 RRC connection release with redirection	1245

6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection	1248

6.3.3	Conditional handover	1253

6.3.3.0	Minimum conformance requirements	1253

6.3.3.0.1	Minimum conformance requirements for NR FR1 intra-frequency conditional handover	1253

6.3.3.0.2	Minimum conformance requirements for NR FR1 inter-frequency conditional handover	1256

6.3.3.1	NR SA FR1 conditional handover	1259

6.3.3.2	NR SA FR1-FR1 conditional handover	1265

6.4	Timing	1274

6.4.1	UE transmit timing	1274

6.4.1.0	Minimum conformance requirements	1274

6.4.1.1	NR SA FR1 UE transmit timing accuracy	1275

6.4.2	UE timer accuracy	1281

6.4.3	Timing advance	1281

6.4.3.0	Minimum conformance requirement	1281

6.4.3.0.1	Minimum conformance requirement for timing advance adjustment	1281

6.4.3.1	NR SA FR1 timing advance adjustment accuracy	1281

6.5	Signaling characteristics	1286

6.5.1	Radio link monitoring	1286

6.5.1.00	General	1286

6.5.1.0	Minimum conformance requirements	1288

6.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	1288

6.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	1289

6.5.1.0.3	Minimum conformance requirements for out-of-sync and in-sync CSI-RS based RLM	1291

6.5.1.0.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	1292

6.5.1.0.5	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	1292

6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1293

6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1299

6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	1304

6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	1309

6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1315

6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1321

6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1326

6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1332

6.5.1.9	SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	1337

6.5.1.9.1	Test purpose	1337

6.5.1.9.2	Test applicability	1338

6.5.1.9.3	Minimum conformance requirements	1338

6.5.1.9.4	Test description	1338

6.5.1.9.5	Test requirement	1342

6.5.2	Interruption	1344

6.5.2.0	Minimum conformance requirements	1344

6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	1344

6.5.2.0.2	 Interruptions at NR SRS carrier based switching	1345

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC	1347

6.5.2.2	SA FR1 interruptions at NR SRS carrier based switching	1352

6.5.3	SCell activation and deactivation delay	1355

6.5.3.0	Minimum conformance requirements	1355

6.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	1355

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	1356

6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	1367

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX	1368

6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1370

6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1379

6.5.4	UE UL carrier RRC reconfiguration delay	1388

6.5.4.0	Minimum conformance requirements	1388

6.5.4.0.1	Minimum conformance requirements for UL carrier RRC reconfiguration delay	1388

6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay	1389

6.5.5	Link recovery procedures	1396

6.5.5.0	Minimum conformance requirements	1396

6.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	1396

6.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	1396

6.5.5.0.4	Requirements for Beam Failure Recovery in SCell	1396

6.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	1396

6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX	1398

6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX	1405

6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	1411

6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX	1418

6.5.5.5	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	1424

6.5.5.6	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	1432

6.5.5.7	NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	1439

6.5.6	Active BWP switch delay	1447

6.5.6.1	DCI-based and time-based active BWP switch	1447

6.5.6.1.0	Minimum conformance requirements	1447

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX	1448

6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX	1459

6.5.6.2	RRC-based active BWP switch	1467

6.5.6.2.0	Minimum conformance requirements	1467

6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX	1467

6.5.7	DL interruptions at switching between two uplink carriers	1474

6.5.7.0	Minimum conformance requirements	1474

6.5.7.1	NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA	1475

6.5.7.2	NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA	1485

6.5.8	UE specific CBW change	1491

6.5.8.0	Minimum conformance requirements	1491

6.5.8.0.1	Minimum conformance requirements for UE specific CBW change	1491

6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1492

6.6	Measurement procedures	1498

6.6.1	Intra-frequency measurements	1498

6.6.1.0	Minimum conformance requirements	1498

6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1498

6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1498

6.6.1.0.3	Void	1499

6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading	1499

6.6.1.0.5	 Void	1501

6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX	1501

6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX	1505

6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX	1510

6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX	1516

6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading	1521

6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading	1525

6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	1529

6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17	1534

6.6.2	Inter-frequency measurements	1539

6.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1539

6.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps	1539

6.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps	1539

6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX	1542

6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX	1548

6.6.2.3	Void	1554

6.6.2.4	Void	1554

6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	1554

6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection	1561

6.6.2.7	Void	1567

6.6.2.8	Void	1567

6.6.2.9	NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern	1567

6.6.2.10	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used	1572

6.6.2.11	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used	1577

6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	1583

6.6.3	Inter-RAT Measurements	1589

6.6.3.0	Minimum conformance requirements	1589

6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	1589

6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	1591

6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX	1594

6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX	1601

6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16	1607

6.6.4	L1-RSRP measurement for beam reporting	1613

6.6.4.0	Minimum conformance requirements	1613

6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1613

6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1613

6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX	1613

6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX	1617

6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX	1621

6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX	1625

6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	1628

6.6.5	UTRAN inter-RAT measurement	1634

6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX	1634

6.6.6	CLI Measurements	1641

6.6.6.0	Minimum conformance requirements	1641

6.6.6.0.1	Minimum conformance requirements for SRS-RSRP measurement	1641

6.6.6.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	1641

6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX	1641

6.6.7	1648

6.6.8	L1-SINR measurement for beam reporting	1648

6.6.8.0	Minimum conformance requirements	1648

6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1648

6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1648

6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1648

6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1648

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1654

6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1660

6.6.9	Idle Mode CA/DC Measurements	1666

6.6.9.0	Minimum conformance requirements	1666

6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1	1668

6.6.10 to 6.6.14	1678

6.6.15	Idle Mode inter-RAT CA/DC Measurements	1678

6.6.15.0	Minimum conformance requirements	1678

6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1679

6.6.16	1692

6.6.17	NR SA FR1 event triggered reporting tests with Pre-MG	1692

6.6.17.1	NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG	1694

6.6.17.2	NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	1701

6.6.18	SA event triggered reporting tests with concurrent gaps	1708

6.6.18.0	Minimum conformance requirements	1708

6.6.18.0.1	Minimum conformance requirements for Intra-frequency measurement	1708

6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement	1709

6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement	1710

6.6.18.0.4	Minimum conformance requirements for PRS measurement	1711

6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers	1711

6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers	1717

6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	1722

6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	1734

6.6.19	SA event triggered reporting tests with NCSG	1743

6.6.19.0	Minimum conformance requirements	1743

6.6.19.0.1	Minimum conformance requirements for intra-frequency measurement	1743

6.6.19.0.2	Minimum conformance requirements for inter-frequency measurement	1745

6.6.19.0.3	Minimum conformance requirements for inter-RAT measurement	1746

6.6.19.0.4	Minimum conformance requirements for interruptions	1747

6.6.19.1	NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1	1748

6.6.19.2	NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement	1755

6.6.19.3	NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	1762

6.6.19.4	NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	1770

6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	1776

6.6.20.0	Minimum conformance requirements	1776

6.6.20.1	UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA	1779

6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS	1783

6.6.21.0	Minimum conformance requirements	1783

6.6.21.1	UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA	1786

6.7	Measurement performance requirements	1790

6.7.1	SS-RSRP	1790

6.7.1.0	Minimum conformance requirements	1790

6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	1790

6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	1790

6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	1790

6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	1791

6.7.1.1	Intra-frequency measurements	1791

6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy	1791

6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy	1798

6.7.1.2	Inter-frequency measurements	1800

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy	1800

6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy	1807

6.7.2	SS-RSRQ	1809

6.7.2.0	Minimum conformance requirements	1809

6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1809

6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	1809

6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	1809

6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy	1809

6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	1813

6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy	1813

6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy	1818

6.7.3	SS-SINR	1820

6.7.3.0	Minimum conformance requirements	1820

6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1820

6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	1820

6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	1820

6.7.3.1	NR SA FR1 SS-SINR measurement accuracy	1820

6.7.3.2	Inter-Frequency SS-SINR measurement accuracy	1824

6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy	1824

6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy	1828

6.7.4	L1-RSRP	1830

6.7.4.0	Minimum conformance requirements	1830

6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	1830

6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	1830

6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	1830

6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	1830

6.7.4.1	SSB based L1-RSRP measurements	1830

6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy	1830

6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy	1835

6.7.4.2	CSI-RS based L1-RSRP measurements	1837

6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy	1837

6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy	1841

6.7.5	E-UTRAN RSRP	1843

6.7.5.0	Minimum conformance requirements	1843

6.7.5.0.1	E-UTRAN RSRP absolute accuracy	1843

6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy	1844

6.7.6	E-UTRAN RSRQ	1851

6.7.6.0	Minimum conformance requirements	1851

6.7.6.0.1	E-UTRAN RSRQ absolute accuracy	1851

6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy	1852

6.7.7	E-UTRAN RS-SINR	1859

6.7.7.0	Minimum conformance requirements	1859

6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy	1859

6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy	1860

6.7.8	CLI Measurements	1867

6.7.8.0	Minimum conformance requirements	1867

6.7.8.0.1	Minimum conformance requirements for SRS-RSRP accuracy	1867

6.7.8.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	1867

Same as in clause 4.7.6.0.2.6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy	1867

6.7.8.1.1	Test purpose	1867

6.7.8.1.2	Test applicability	1867

6.7.8.1.3	Minimum conformance requirements	1868

6.7.8.1.4	Test description	1868

6.7.8.1.5	Test requirement	1870

6.7.9	L1-SINR measurement for beam reporting	1877

6.7.9.0	Minimum conformance requirements	1877

6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1877

6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	1880

6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	1882

6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	1886

6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	1886

6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	1889

6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1891

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1894

6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1895

6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	1898

7	NR standalone with at least one NR cell in FR2	1901

7.0	General	1901

7.1	RRC_IDLE state mobility	1901

7.1.1	NR cell re-selection	1901

7.1.1.0	Minimum conformance requirements	1901

7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	1901

7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	1901

7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion	1903

7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion	1904

7.1.1.1	NR SA FR2 cell re-selection	1905

7.1.1.2	NR SA FR2-FR2 cell re-selection	1910

7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1918

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1923

7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1928

7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1935

7.2	RRC_INACTIVE state mobility	1941

7.2.1	Small Data Transmission	1941

7.2.1.0	Minimum conformance requirements	1941

7.2.1.0.1	Minimum conformance requirements for CG-SDT TA validation	1941

7.2.1.1	TA Validation for CG-SDT in FR2	1941

7.3	RRC_CONNECTED state mobility	1946

7.3.1	Handover	1946

7.3.1.0	Minimum conformance requirements	1946

7.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	1946

7.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR2 handover	1947

7.3.1.0.3	Minimum conformance requirements for NR FR1 – NR FR2 DAPS handover	1948

7.3.1.1	NR SA FR1-FR2 Inter-frequency handover; unknown target cell	1949

7.3.1.2	NR SA FR2 Intra-frequency handover; unknown target cell	1954

7.3.1.3	NR SA FR2-FR2 Inter-frequency handover; unknown target cell	1958

7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover	1962

7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover	1972

7.3.2	RRC connection mobility control	1979

7.3.2.1	RRC re-establishment	1979

7.3.2.1.0	Minimum conformance requirements	1979

7.3.2.1.1	NR SA FR2 RRC re-establishment	1980

7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment	1984

7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	1987

7.3.2.2	Random access	1992

7.3.2.2.0	Minimum conformance requirements	1992

7.3.2.2.1	NR SA FR2 contention based random access	1993

7.3.2.2.2	NR SA FR2 non-contention based random access	2001

7.3.2.2.3	NR SA FR2 2-step contention based random access	2008

7.3.2.2.4	NR SA FR2 2-step non-contention based random access	2015

7.3.2.3	RRC connection release with redirection	2022

7.3.2.3.0	Minimum conformance requirements	2022

7.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection	2023

7.3.3	Conditional handover	2027

7.3.3.0	Minimum conformance requirements	2027

7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover	2027

7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover	2031

7.3.3.1	NR SA FR2 conditional handover	2035

7.3.3.2	NR SA FR2-FR2 conditional handover	2042

7.4	Timing	2049

7.4.1	UE transmit timing	2049

7.4.1.0	Minimum Conformance Requirements	2049

7.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	2049

7.4.1.1	NR SA FR2 UE transmit timing accuracy	2050

7.4.2	UE timer accuracy	2056

7.4.3	Timing advance	2056

7.4.3.0	Minimum conformance requirements	2056

7.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	2056

7.4.3.1	NR SA FR2 timing advance adjustment accuracy	2057

7.5	Signalling characteristics	2063

7.5.1	Radio link monitoring	2063

7.5.1.0	Minimum conformance requirements	2063

7.5.1.0.1	2063

7.5.1.0.2	2063

7.5.1.0.3	2063

7.5.1.0.4	2063

7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	2063

7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2063

7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2069

7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2076

7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2081

7.5.1.5 to 7.5.1.8	2088

7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	2088

7.5.2	Interruption	2092

7.5.2.0	Minimum conformance requirements	2092

7.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	2092

7.5.2.1	NR SA FR2 interruptions during measurements on deactivated NR SCC	2093

7.5.3	SCell activation and deactivation delay	2098

7.5.3.0	Minimum conformance requirements	2098

7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell	2098

7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell	2101

7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay	2101

7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay	2103

7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	2106

7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	2113

7.5.3.5	Direct SCell activation at handover with known SCell in FR2	2120

7.5.4	Void	2128

7.5.5	Link recovery procedures	2128

7.5.5.0	Minimum conformance requirements	2128

7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	2128

7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	2128

7.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	2128

7.5.5.0.4	Requirements for Beam Failure Recovery in SCell	2128

7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX	2128

7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX	2133

7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	2140

7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX	2145

7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX	2152

7.5.5.6	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX	2157

7.5.5.7	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX	2164

7.5.5.8	FFS	2170

7.5.5.9	NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	2170

7.5.5.10	NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX	2176

7.5.6	Active BWP switch delay	2183

7.5.6.1	DCI-based and time-based active BWP switch	2183

7.5.6.1.0	Minimum conformance requirements	2183

7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2183

7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX	2184

7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2185

7.5.6.2	RRC-based active BWP switch	2186

7.5.6.2.0	Minimum conformance requirements	2186

7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX	2187

7.5.7	PSCell addition and release delay	2189

7.5.7.0	Minimum conformance requirements	2189

7.5.7.0.1	Minimum conformance requirements for PSCell addition delay	2189

7.5.7.0.2	Minimum conformance requirements for PSCell release delay	2190

7.5.7.1	NR SA FR2 addition and release delay of known PSCell	2190

7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell	2192

7.5.8	Active TCI state switch delay	2194

7.5.8.0	Minimum conformance requirements	2194

7.5.8.0.0	Known conditions for TCI state	2194

7.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	2194

7.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	2195

7.5.8.1	NR SA FR2 MAC-CE based active TCI state switch	2196

7.5.8.1.1	NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state	2196

7.5.8.2	NR SA FR2 RRC based active TCI state switch	2202

7.5.8.2.1	NR SA Pcell FR2 RRC based active TCI state switch for a known TCI state	2202

7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	2209

7.5.8.3.0	Minimum conformance requirements	2209

7.5.8.3.1	NR PCell FR2 HST active TCI state switch for a known TCI state	2210

7.5.9 to 7.5.10	2215

7.5.11	UE UL carrier RRC reconfiguration delay	2215

7.5.11.1	UE UL carrier RRC reconfiguration delay	2215

7.6	Measurement procedures	2220

7.6.1	Intra-frequency measurements	2220

7.6.1.0	Minimum conformance requirements	2220

7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	2220

7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	2222

7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX	2223

7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX	2228

7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX	2233

7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	2239

7.6.2	Inter-frequency measurements	2245

7.6.2.0	Minimum conformance requirements for Inter-frequency measurements	2245

7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX	2245

7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX	2250

7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	2256

7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection	2261

7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX	2267

7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX	2273

7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	2279

7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection	2285

7.6.3	L1-RSRP measurement for beam reporting	2291

7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	2291

7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	2291

7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	2291

7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX	2291

7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX	2297

7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX	2301

7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX	2305

7.6.4	CLI measurements	2308

7.6.4.0	Minimum conformance requirements	2308

7.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	2308

7.6.4.1	NR SA FR2 SRS-RSRP measurement in non-DRX	2309

7.6.4.2	NR SA FR2 CLI-RSSI measurement in non-DRX	2313

7.6.4.2.1	Test purpose	2313

7.6.4.2.2	Test applicability	2313

7.6.4.2.3	Minimum conformance requirements	2314

7.6.4.2.4	Test description	2314

7.6.4.2.5	Test requirement	2315

7.6.5	2317

7.6.6	L1-SINR measurement for beam reporting	2317

7.6.6.0	Minimum conformance requirements	2317

7.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	2317

7.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	2317

7.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	2317

7.6.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	2317

7.6.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	2321

7.6.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	2326

7.6.7 to 7.6.12	2332

7.6.13	UE Rx-Tx time difference measurements for PDC	2332

7.6.13.0	Minimum conformance requirements	2332

7.6.13.0.1	PRS based UE Rx-Tx time difference for propagation delay compensation	2332

7.6.13.0.2	TRS based UE Rx-Tx time difference for propagation delay compensation	2334

7.6.13.1	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS	2336

7.6.13.2	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS	2340

7.7	Measurement performance requirements	2344

7.7.1	SS-RSRP	2344

7.7.1.0	Minimum conformance requirements	2344

7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	2344

7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	2344

7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy	2344

7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy	2352

7.7.1.3	Inter-frequency measurements between FR1 and FR2	2360

7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy	2360

7.7.1.3.2	Void	2367

7.7.2	SS-RSRQ	2367

7.7.2.0	Minimum conformance requirements	2367

7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2367

7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	2367

7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy	2367

7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy	2371

7.7.3	SS-SINR	2377

7.7.3.0	Minimum conformance requirements	2377

7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2377

7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	2377

7.7.3.1	NR SA FR2 SS-SINR measurement accuracy	2377

7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy	2382

7.7.4	L1-RSRP	2389

7.7.4.0	Minimum conformance requirements	2389

7.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	2389

7.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	2389

7.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	2389

7.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	2389

7.7.4.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	2389

7.7.4.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy	2394

7.7.5	SRS-RSRP	2400

7.7.5.0	Minimum conformance requirements	2400

7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	2400

7.7.5.1	NR SA FR2 SRS-RSRP measurement accuracy	2402

7.7.5.2	NR SA FR2 CLI-RSSI measurement accuracy	2407

7.7.5.2.1	Test purpose	2408

7.7.5.2.2	Test applicability	2408

7.7.5.2.3	Minimum conformance requirements	2408

7.7.5.2.4	Test description	2408

7.7.5.2.4	Test description	2408

7.7.5.2.5	Test requirement	2409

7.7.6	L1-SINR	2412

7.7.6.0	Minimum conformance requirements	2412

7.7.6.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2412

7.7.6.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR L1-SINR measurement accuracy	2414

7.7.6.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2416

7.7.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	2419

7.7.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy	2424

7.7.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2429

8.0	General	2435

8.1	Void	2435

8.2.1.0	Minimum conformance requirements	2435

8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection	2435

8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell	2437

8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2444

8.2.2.0	Minimum conformance requirements	2451

8.2.2.1	E-UTRA – NR FR1 Early Measurement Reporting	2453

8.2.2.2	E-UTRA – NR FR2 Early Measurement Reporting	2460

8.3.1.0	Minimum conformance requirements	2465

8.3.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 handover	2465

8.3.1.1	E-UTRA - NR FR1 handover with known target cell	2466

8.3.1.1.3	Minimum conformance requirements	2466

8.4.1.0	Minimum conformance requirements	2471

8.4.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 SFTD measurement delay	2471

8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX	2472

8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX	2478

8.4.2.0	Minimum conformance requirements	2483

8.4.2.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	2483

8.4.2.0.2	Void	2487

8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX	2487

8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX	2493

8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX	2499

8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX	2505

8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX	2511

8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX	2516

8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX	2521

8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX	2526

8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2531

8.5.1	SFTD measurement accuracy	2537

8.5.1.0	Minimum conformance requirements	2537

8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	2537

8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy	2538

8.5.2	Inter-RAT measurement accuracy	2544

8.5.2.1	SS-RSRP	2544

8.5.2.1.0	Minimum conformance requirements	2544

8.5.2.1.1	SS-RSRP with NR FR1 target cell	2545

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell	2550

8.5.2.2	SS-RSRQ	2555

8.5.2.2.0	Minimum conformance requirements	2555

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell	2555

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell	2558

8.5.2.3	SS-SINR	2563

8.5.2.3.0	Minimum conformance requirements	2563

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell	2563

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell	2566

9	NR sidelink	2571

9.1	NR sidelink in FR1	2571

9.1.1	UE transmit timing	2571

9.1.1.0	Minimum conformance requirements	2571

9.1.1.0.1	Minimum conformance requirements for GNSS as synchronization reference source	2571

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	2571

9.1.1.0.3	Minimum conformance requirements for FR1 NR Cell as synchronization reference source	2571

9.1.1.1	NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source	2572

9.1.1.2	NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source	2574

9.1.1.3	NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source	2577

9.1.2	Initiation/Cease of S-SSB transmission	2582

9.1.2.0	Minimum conformance requirements	2582

9.1.2.0.1	Minimum conformance requirements for FR1 NR cell as synchronization reference source	2582

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	2583

9.1.2.1	NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source	2584

9.1.2.2	NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source	2589

9.1.3	Synchronization reference selection/reselection	2593

9.1.3.0	Minimum conformance requirements	2593

9.1.3.0.1	Minimum conformance requirements for GNSS configured as the highest priority synchronization reference source	2593

9.1.3.0.2	Minimum conformance requirements for eNB/gNB configured as the highest priority synchronization reference source	2594

9.1.3.1	NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source	2596

9.1.3.2	NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source	2602

9.1.4	L1 SL-RSRP measurements	2607

9.1.4.0	Minimum conformance requirements	2607

9.1.4.0.1	Minimum conformance requirements for resource selection/reselection, re-evaluation and pre-emption	2607

9.1.4.1	NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection	2608

9.1.4.2	NR SA FR1 L1 SL-RSRP measurement for resource pre-emption	2614

9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation	2620

9.1.5	Congestion control measurement	2627

9.1.5.0	Minimum conformance requirements	2627

9.1.5.0.1	Minimum conformance requirements for congestion control measurements	2627

9.1.5.1	NR SA FR1 congestion control measurement for concurrent operation	2628

9.1.5.2	NR SA FR1 congestion control measurement for PC5-only operation	2636

9.1.6	Congestion control measurement	2642

9.1.6.0	Minimum conformance requirements	2642

9.1.6.0.1	Minimum conformance requirements for interruption to WAN due to NR sidelink communication	2642

9.1.6.1	NR SA FR1 interruption to WAN due to NR sidelink communication	2642

10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2646

10.1	RRC_CONNECTED state mobility	2646

10.2	Timing	2646

10.3	Signalling characteristics	2647

10.3.1	Radio link monitoring	2647

10.3.1.0	General	2647

10.3.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring under CCA	2648

10.3.1.1	Requirements for determining UE in-sync or out-of-sync status	2650

10.3.1.2	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	2651

10.3.1.3	EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	2658

10.3.1.4	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell under CCA configured with SSB-based RLM RS in DRX mode	2664

10.3.1.5	EN-DC FR1 radio link monitoring in-sync test for PSCell under CCA configured with SSB-based RLM RS in DRX mode	2664

10.3.2	Interruption	2665

10.3.3	SCell activation and deactivation delay	2665

10.3.4	Beam failure detection and link recovery procedures	2665

10.3.4.0	General	2665

10.3.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	2665

10.3.4.0.3	Scheduling availability of UE during beam failure detection under CCA	2667

10.3.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	2667

10.3.4.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	2668

10.3.4.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	2675

10.4	Measurement procedures	2682

10.4.1	Intra-frequency measurements	2682

10.4.1.0	Minimum conformance requirements	2682

10.4.1.1	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA	2696

10.4.1.2	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA	2699

10.4.1.3	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA	2702

10.4.1.4	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA	2705

10.5	Measurement Performance	2708

10.5.1	SS-RSRP	2708

10.5.1.0	Minimum Conformance Requirements	2708

10.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	2708

10.5.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	2708

10.5.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	2709

10.5.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	2710

10.5.1.1	EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell	2710

10.5.1.2	EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell	2715

11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2720

11.1	RRC_IDLE state mobility	2720

11.2	RRC_CONNECTED state mobility	2721

11.3	Timing	2721

11.4	Signalling characteristics	2721

11.4.1	Radio link monitoring	2721

11.4.1.0	General	2721

11.4.1.0.1	Minimum conformance requirements for SSB based radio link monitoring under CCA	2721

11.4.1.1	Requirements for determining UE in-sync or out-of-sync status	2721

11.4.1.2	NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	2721

11.4.1.3	NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	2728

11.4.2	Interruptions	2733

11.4.3	SCell activation and deactivation delay	2733

11.4.4	Beam failure detection and link recovery procedures	2734

11.4.4.0	General	2734

11.4.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	2734

11.4.4.0.2	Void	2734

11.4.4.0.3	Scheduling availability of UE during beam failure detection under CCA	2734

11.4.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	2734

11.4.4.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA	2734

11.4.4.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA	2742

11.5	2749

11.6	Measurement Performance	2749

11.6.1	SS-RSRP	2749

11.6.1.0	Minimum Conformance Requirements	2749

11.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	2749

11.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	2749

11.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	2749

11.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	2749

11.6.1.1	NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA	2749

11.6.1.2	NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2754

12 to 15	FFS	2759

16	NR standalone tests with all NR cells in FR1 for RedCap	2760

16.0	General	2760

16.1	RRC_IDLE state mobility	2760

16.1.1	NR cell re-selection	2760

16.1.1.0	Minimum conformance requirements	2760

16.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	2760

16.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	2762

16.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	2766

16.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	2769

16.1.1.1	NR SA FR1 Cell reselection for 1 Rx UE	2772

16.1.1.2	NR SA FR1 Cell reselection for 2 Rx UE	2778

16.1.1.3	NR SA FR1-FR1 Cell reselection for 1 Rx UE	2782

16.1.1.4	NR SA FR1-FR1 Cell reselection for 2 Rx UE	2788

16.1.1.5	NR SA FR1  Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	2792

16.1.1.6	NR SA FR1  Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	2798

16.1.1.7	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	2802

16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	2809

16.1.2	NR – E-UTRA cell re-selection	2816

16.1.2.0	Minimum conformance requirements	2816

16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx	2816

16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx	2818

16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection	2818

16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX	2820

16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX	2826

16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX	2830

16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX	2836

16.1.2.5	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	2840

16.1.2.6	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	2846

16.2	2850

16.3	RRC_CONNECTED state mobility for RedCap	2850

16.3.1	Handover	2850

16.3.1.0	Minimum conformance requirements	2850

16.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	2850

16.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	2851

16.3.1.1	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE	2851

16.3.1.2	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE	2855

16.3.1.3	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	2858

16.3.1.4	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	2863

16.3.1.5	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	2869

16.3.1.6	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	2874

16.3.1.7	NR SA FR1 - E-UTRA handover for 1Rx UE	2880

16.3.1.8	NR SA FR1 - E-UTRA handover for 2Rx UE	2885

16.3.1.9	NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE	2890

16.3.1.10	NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE	2894

16.3.2	RRC connection mobility control for RedCap	2898

16.3.2.1	RRC re-establishment for RedCap	2898

16.3.2.1.0	Minimum conformance requirements	2898

16.3.2.1.1	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	2898

16.3.2.1.2	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	2901

16.3.2.1.3	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	2904

16.3.2.1.4	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	2907

16.3.2.1.5	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	2910

16.3.2.1.6	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	2913

16.3.2.2	Random access for RedCap	2916

16.3.2.2.0	Minimum conformance requirements	2916

16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	2917

16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	2920

16.3.2.2.3	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	2923

16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	2926

16.3.2.2.5	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	2929

16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	2932

16.3.2.2.7	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	2935

16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	2938

16.3.2.3	RRC connection release with redirection for RedCap	2941

16.3.2.3.0	Minimum conformance requirements	2941

16.3.2.3.1	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	2941

16.3.2.3.2	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	2945

16.3.2.3.3	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	2948

16.3.2.3.4	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	2953

16.4	Timing for RedCap	2958

16.4.1	UE transmit timing for RedCap	2958

16.4.1.0	Minimum conformance requirements	2958

16.4.1.1	NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	2959

16.4.1.2	NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	2962

16.4.2	2965

16.4.3	Timing advance for RedCap	2965

16.4.3.0	Minimum conformance requirement	2965

16.4.3.1	NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE	2965

16.4.3.2	NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE	2968

16.5	Signalling characteristics for RedCap	2971

16.5.1	Radio link monitoring for RedCap	2971

16.5.1.0	Minimum requirements	2971

16.5.1.0.1	General	2971

16.5.1.0.2	Minimum conformance requirements for SSB-based RLM	2975

16.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	2976

16.5.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	2977

16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	2980

16.5.1.3	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	2983

16.5.1.4	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	2987

16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	2991

16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	2995

16.5.1.7	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	2999

16.5.1.8	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	3003

16.5.1.9	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3007

16.5.1.10	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	3010

16.5.1.11	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3013

16.5.1.12	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	3016

16.5.1.13	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3019

16.5.1.14	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	3022

16.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	3025

16.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	3028

16.5.2	Beam Failure Detection and Link recovery procedures for RedCap	3031

16.5.2.0	Minimum requirements	3031

16.5.2.0.1	General	3031

16.5.2.0.2	Minimum conformance requirements for SSB-based BFD and CBD	3033

16.5.2.0.3	Minimum conformance requirements for CSI-RS based RLM	3035

16.5.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	3037

16.5.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	3041

16.5.2.3	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE	3045

16.5.2.4	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE	3049

16.5.2.5	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	3053

16.5.2.6	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	3057

16.5.2.7	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	3060

16.5.2.8	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	3063

16.5.3	Active BWP switch delay for RedCap	3067

16.5.3.1	DCI-based and time-based active BWP switch for RedCap	3067

16.5.3.1.0	Minimum conformance requirements	3067

16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE	3068

16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE	3072

16.5.3.2	RRC-based active BWP switch for RedCap	3076

16.5.3.2.0	Minimum conformance requirements	3076

16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE	3077

16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE	3080

16.5.4	UE specific CBW change for RedCap	3083

16.5.4.0	Minimum conformance requirements	3083

16.5.4.0.1	Minimum conformance requirements for UE specific CBW change	3083

16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE	3084

16.5.4.2	NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE	3086

16.6	Measurement procedures for RedCap	3088

16.6.1	Intra-frequency measurements for RedCap	3089

16.6.1.0	Minimum conformance requirements	3089

16.6.1.0.1	General	3089

16.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	3090

16.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	3092

16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE	3093

16.6.1.2	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE	3099

16.6.1.3	NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE	3105

16.6.1.4	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE	3107

16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	3110

16.6.1.6	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	3113

16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	3116

16.6.1.8	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	3119

16.6.1.9	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	3122

16.6.1.10	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	3128

16.6.1.11	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	3133

16.6.1.12	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2Rx UE	3136

16.6.2	Inter-frequency measurements for RedCap	3139

16.6.2.0	Minimum conformance requirements	3139

16.6.2.1	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	3140

16.6.2.2	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	3140

16.6.2.3	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	3140

16.6.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	3140

16.6.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	3140

16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	3140

16.6.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	3140

16.6.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	3140

16.6.2.9	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	3140

16.6.2.10	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	3140

16.6.2.11	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	3140

16.6.2.12	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	3140

16.6.3	Inter-RAT measurements for RedCap	3140

16.6.3.0	Minimum conformance requirements	3140

16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE	3141

16.6.3.2	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE	3141

16.6.3.3	NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE	3141

16.6.3.4	NR - E-UTRA event-triggered reporting in DRX in FR1 for 2 Rx UE	3141

16.6.4	L1-RSRP measurement for beam reporting for RedCap	3141

16.6.4.0	Minimum conformance requirements	3141

16.6.4.0.1	General	3141

16.6.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	3145

16.6.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	3149

16.6.4.3	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	3152

16.6.4.4	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	3156

16.6.4.5	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	3159

16.6.4.6	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	3162

16.6.4.7	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	3164

16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	3167

17	NR standalone tests with at least one NR cell in FR2 for RedCap	3169

17.1	RRC_IDLE state mobility	3169

17.1.1	NR cell re-selection	3169

17.1.1.0	Minimum conformance requirements	3169

17.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	3169

17.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	3171

17.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3173

17.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3175

17.1.1.1	NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx	3177

17.1.1.2	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx	3182

17.1.1.3	NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3188

17.1.1.4	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	3193

17.2	3199

17.3	3199

17.3.1	3199

17.3.2	RRC connection mobility control for RedCap	3199

17.3.2.1	RRC re-establishment for RedCap	3199

17.3.2.1.0	Minimum conformance requirements	3199

17.3.2.1.1	NR SA FR2 Intra-frequency RRC Re-establishment in FR2	3199

17.3.2.1.2	NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2	3201

17.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	3203

17.3.2.2	Random access for RedCap	3206

17.3.2.2.0	Minimum conformance requirements	3206

17.3.2.2.0.1	Minimum conformance requirements for Contention based random access	3206

17.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access	3206

17.3.2.2.1	NR SA FR2 4-step RA type contention based random access test in FR2 for NR Standalone	3207

17.3.2.2.2	NR SA FR2 4-step RA type non-contention based random access test in FR2 for NR Standalone	3210

17.3.2.2.3	NR SA FR2 2-step RA type contention based random access test in FR2 for NR Standalone	3213

17.3.2.2.4	NR SA FR2 2-step RA type non-contention based random access test in FR2 for NR Standalone	3216

17.4	3219

17.5	Signalling characteristics for RedCap	3219

17.5.1	Radio link monitoring for RedCap	3219

17.5.1.0	Minimum requirements	3219

17.5.1.0.1	General	3219

17.5.1.0.2	Minimum conformance requirements for SSB-based RLM	3222

17.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	3224

17.5.1.1	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3226

17.5.1.2	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3229

17.5.1.4	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3233

17.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	3236

17.5.2	Beam Failure Detection and Link recovery procedures for RedCap	3238

17.5.2.0	Minimum conformance requirements	3238

17.5.2.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	3238

17.5.2.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	3239

17.5.2.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	3239

17.5.2.1 – 17.5.2.2	3239

17.5.2.3	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3239

17.5.2.4	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3242

17.5.2.5	NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3245

17.6	Measurement procedures for RedCap	3249

17.6.1	Intra-frequency measurements for RedCap	3249

17.6.1.0	Minimum conformance requirements	3249

17.6.1.0.1	General	3249

17.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	3250

17.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	3253

17.6.1.1	NR SA FR2 Event triggered reporting test without gap under non-DRX	3254

17.6.1.2	NR SA FR2 Event triggered reporting test without gap under DRX	3256

17.6.1.3	NR SA FR2 Event triggered reporting test with per-UE gaps under non-DRX	3258

17.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	3265

17.6.2	3268

17.6.3	L1-RSRP measurement for beam reporting for RedCap	3268

17.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	3268

17.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	3268

17.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	3268

17.6.3.1	NR SA FR2 SSB based L1-RSRP measurement when DRX is not used	3268

17.6.3.2	NR SA FR2 SSB based L1-RSRP measurement when DRX is used	3270

17.6.3.3	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is not used	3272

17.6.3.4	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used	3274

18	E-UTRA - NR inter-RAT with E-UTRA serving cell	3277

18.1	RRC_IDLE state mobility	3277

18.1.1	Inter-rat NR cell re-selection	3277

18.1.1.0	Minimum conformance requirements	3277

18.1.1.1	E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1	3277

18.2	RRC_CONNECTED state mobility for RedCap	3281

18.2.1	Handover for RedCap	3281

18.2.1.0	Minimum conformance requirements	3281

18.2.1.1	E-UTRA - NR FR1 handover  for 2Rx UE	3282

18.2.2.0	Minimum conformance requirements	3285

18.2.2.0.1	Redirection from E-UTRA to FR1 RedCap UE	3285

18.2.2.1	Redirection from E-UTRA to NR SA FR1 for redcap UE	3286

18.3	Measurement procedure for RedCap	3293

18.3.1	E-UTRA - NR Measurements for RedCap	3293

18.3.1.0	Minimum conformance requirements	3293

18.3.1.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	3293

18.3.1.1	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3296

18.3.1.2	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3300

18.3.1.3	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3304

18.3.1.4	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3308

18.3.1.5	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3312

18.3.1.6	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3314

18.3.1.7	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3317

18.3.1.8	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3319

Annex A (normative): RRM test configurations	3323

A.1	Reference measurement channels	3323

A.1.0	General	3323

A.1.1	PDSCH	3323

A.1.1.1	FDD	3323

A.1.1.2	TDD	3324

A.1.2	CORESET for RMSI scheduling	3327

A.1.2.1	FDD	3327

A.1.2.2	TDD	3328

A.1.3	CORESET for RMC scheduling	3331

A.1.3.1	FDD	3331

A.1.3.2	TDD	3331

A.1.4	CSI-RS	3333

A.1.4.1	FDD	3333

A.1.4.2	TDD	3335

A.1.4A	CSI-RS for tracking	3340

A.1.4A.1	FR1	3340

A.1.4A.1.1	FDD	3340

A.1.4A.1.2	TDD	3341

A.1.4A.2	FR2	3341

A.1.4A.2.1	TDD	3341

A.1.4B	CSI-IM configurations	3341

A.1.4B.1	FDD	3341

A.1.4B.2	TDD	3342

A.1.5	TDD UL/DL configuration	3344

A.1.6	PUSCH	3345

A.1A	Reference measurement channels under CCA	3345

A.1A.0	General	3345

A.1A.1	PDSCH	3346

A.1A.1.1	FDD	3346

A.1A.2	CORESET for RMSI scheduling	3347

A.1A.2.1	TDD	3347

A.1A.3	CORESET for RMC scheduling	3348

A.1A.3.1	TDD	3348

A.1A.4	TDD UL/DL configuration	3348

A.1A.4.1	TDD	3348

A.1A.5	RMC burst transmission model	3349

A.2	Reference OCGN configuration	3349

A.2.1 Generic OFDMA channel noise generator (OCNG)	3349

A.3	Reference SSB configuration	3352

A.3.1	SSB configuration for FR1	3352

A.3.2	SSB configuration for FR2	3352

A.3A	Reference SSB configuration under CCA	3353

A.3A.1	SSB configuration under CCA for FR1	3353

A.3A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	3353

A.3A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	3353

A.3A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	3354

A.3A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	3354

A.3B	Reference SSB configuration for RedCap	3355

A.3B.1	SSB configuration for FR1	3355

A.3B.2	SSB configuration for FR2	3356

A.4	Reference SMTC configuration	3356

A.4A	Reference SMTC configuration for RedCap	3356

A.5	Reference DRX configurations	3356

A.6	EN-DC test setup	3357

A.6.1	E-UTRA serving cell parameters	3357

A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1	3357

A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2	3359

A.6A	NR FR1-FR2 test setup	3361

A.7	Reference PRACH configurations	3361

A.7.1	PRACH configurations for FR1	3361

A.7.2	PRACH configurations for FR2	3362

A.7A	Reference MsgA configurations	3363

A.7A.1	MsgA configurations for FR1	3363

A.7A.2	MsgA configurations for FR2	3366

A.8	Reference BWP configurations	3368

A.8.1	Downlink BWP configurations	3368

A.8.2	Uplink BWP configurations	3368

A.8A	Reference BWP configurations for RedCap	3369

A.8A.1	Downlink BWP configurations	3369

A.8A.2	Uplink BWP configurations	3369

A.9	Angle of Arrival (AoA) for FR2 RRM test cases	3369

A.9.1	Setup 1: Single AoA in Rx beam peak direction	3370

A.9.2	Setup 2: Single AoA in non Rx beam peak direction	3370

A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	3370

A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	3370

A.9.3	Setup 3: 2 AoAs	3370

A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	3371

A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	3371

A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	3371

A.10	TCI State Configuration	3371

A.10.1	Introduction	3371

A.10.2	TCI states	3371

A.11	NR sidelink communication	3372

A.11.1	Introduction	3372

A.11.2	Reference resource pool configurations for NR Sidelink Communication	3372

A.11.3	Reference measurement channels for NR Sidelink Communication	3375

A.12	Discovery Burst Transmission Window configuration under CCA	3376

A.12.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	3376

A.13	Channel bandwidth (CBW) configurations	3376

A.13.1	DL UE specific CBW	3376

A.13.2	UL UE specific CBW	3376

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3377

B.1	Conditions for NR RRC_IDLE state mobility	3377

B.1.1	Introduction	3377

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	3377

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	3378

B.2	Conditions for NR RRC_CONNECTED state	3380

B.2.1	Introduction	3380

B.2.2	Conditions for NR intra-frequency measurements	3381

B.2.3	Conditions for NR inter-frequency measurements	3381

B.2.4	Conditions for NR L1-RSRP reporting	3382

B.2.4.1	Conditions for SSB based L1-RSRP reporting	3382

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	3383

B.2.5	Conditions for RRC connection release with redirection to NR	3384

B.2.6	Conditions for UE transmit timing	3385

B.2.6.1	Conditions for SSB based UE transmit timing	3385

B.2.7	Conditions for NR intra-frequency measurements for RedCap	3386

B.2.8	Conditions for NR inter-frequency measurements for RedCap	3388

B.3	RRM requirement exceptions	3390

B.3.1	Introduction	3390

B.3.2	Receiver sensitivity relaxation for CA	3390

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	3390

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	3391

B.3.2.2.1	Inter-band carrier aggregation	3391

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	3391

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	3391

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	3391

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	3391

B.3.2.4.1	Intra-band contiguous carrier aggregation	3391

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	3391

B.3.3	Receiver sensitivity relaxation for DC	3392

B.3.4	Receiver sensitivity relaxation for SUL	3392

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	3392

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	3392

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	3392

B.4	Conditions for NR sidelink	3392

B.4.1	Test parameters for GNSS signals	3392

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	3392

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3393

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	3393

Annex C (normative): Downlink physical channels and propagation conditions	3394

C.1	Downlink physical channels	3394

C.1.1	General	3394

C.1.2	Default downlink signal levels	3394

C.1.3	Default connection setup	3395

C.2	Propagation conditions	3395

C.2.0	General	3395

C.2.1	No interference	3396

C.2.2	Static propagation conditions	3396

C.2.2.0	General	3396

C.2.2.1	UE receiver with 2Rx antenna connectors	3396

C.2.2.2	UE receiver with 4Rx antenna connectors	3396

C.2.3	Multi-path fading propagation conditions	3397

Annex D (normative): Deviations from standard test configuration	3398

D.1	Test cases with different numerologies	3398

D.2	EN-DC test cases with different EN-DC configurations	3398

D.2.0	General	3398

D.2.1	Principle of testing	3398

D.3	Carrier aggregation test cases with different CA configurations	3398

D.3.0	General	3398

D.3.1	Principle of testing	3398

D.4	Antenna connection for 4Rx capable UEs	3398

D.4.0	General	3398

D.4.1	Principle of testing	3399

D.4.1.1	Single carrier tests	3399

D.4.1.2	Carrier aggregation tests	3400

D.4.1.3	EN-DC tests	3400

D.4.2	Antenna connection	3400

D.4.2.1	Antenna connection for NR bands where 2Rx is supported	3400

D.4.2.2	Antenna connection for NR bands where only 4Rx is supported	3400

D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported	3400

D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported	3401

D.4A	Antenna configurations with unlicensed bands for 4Rx capable UEs	3401

D.4A.1	Introduction	3401

D.4A.2	Principle of Testing	3401

D.4A.2.1	Single Carrier Tests	3401

D.4A.2.2	Carrier Aggregation Tests	3402

D.4A.2.3	EN-DC Tests	3402

D.4A.2.4	Antenna connection for bands where 2RX is supported	3402

D.4A.2.5	Antenna connection for bands where 4RX is supported	3402

D.4A.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported	3402

D.4A.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported	3402

D.5	Test Cases with Different Channel Bandwidths	3403

D.5.1	Test Cases with Different E-UTRA Channel Bandwidths	3403

D.5.1.1	Introduction	3403

D.5.1.2	Principle of testing	3403

D.6	Test Cases for Synchronous and Asynchronous DC Operations	3403

D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	3403

D.6.1.1	Introduction	3403

D.6.1.2	Principle of Testing	3403

D.7	CCA model	3403

D.7.1	Introduction	3403

D.7.2	CCA model for operation on a carrier frequency with CCA in FR1	3403

D.7.2.1	DL CCA model	3403

D.7.2.2	UL CCA model	3405

D.7.3	Test Cases with at Least One Cell on a Carrier Frequency with CCA	3405

D.8	Test Cases in SA and EN-DC Operations	3405

D.8.0	General	3405

D.8.1	Principle of testing	3406

Annex E (normative): Cell configuration mapping	3407

E.0	General	3407

E.1	Test frequency selection	3407

E.1.0	General	3407

E.1.1	E-UTRA PCell for EN-DC test cases	3407

E.1.2	Test cases with one NR cell	3407

E.1.3	Test cases with more than one NR cell	3407

E.1.3.1	Intra-frequency test cases	3407

E.1.3.2	Inter-frequency test cases	3407

E.1.4	Carrier aggregation test cases	3408

E.1.4.1	Inter-band carrier aggregation	3408

E.1.4.2	Intra-band contiguous carrier aggregation	3408

E.1.4.3	Intra-band non-contiguous carrier aggregation	3408

E.1.5	E-UTRA – NR inter RAT test cases	3408

E.1.6	Intra-band EN-DC test cases	3408

E.1.6.1	Intra-band non-contiguous EN-DC	3408

E.1.6.1.1	Inter frequency neighbour cell	3408

E.1.6.2	Intra-band contiguous EN-DC	3408

E.1.6.1.1	E-UTRA PCell	3408

E.1.7	NR sidelink test cases	3409

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4	3409

E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5	3412

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6	3415

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7	3420

E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8	3423

E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9	3424

E.8 to E.13	3424

E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16	3424

E.15	Cell configuration mapping for SA FR2 test cases for RedCap in Chapter 17	3429

E.16	Cell configuration mapping for E-UTRAN – SA test cases for RedCap in Chapter 18	3430

Annex F (normative): Measurement uncertainties and test tolerances	3431

F.1	Measurement uncertainties and test tolerances for FR1 and FR2	3431

F.1.1	Acceptable uncertainty of test system (normative)	3431

F.1.1.1	Measurement of test environments	3431

F.1.1.2	Measurement of RRM requirements	3431

F.1.2	Interpretation of measurement results (normative)	3473

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)	3473

F.1.3.1	Measurement of test environments	3473

F.1.3.2	Measurement of RRM requirements	3473

Annex G (normative): Statistical testing	3558

G.1	General	3558

G.2	Statistical testing of delay and UE measurement performance in RRM tests	3558

G.2.1	General	3558

G.2.2	Design of the test	3558

G.2.3	Numerical definition of the pass fail limits	3558

G.2.4	Pass fail decision rules	3559

G.2.5	Void	3560

G.2.6	Test conditions for delay tests and UE measurement performance	3560

G.3	Statistical testing of NR sidelink CBR measurement tests	3560

G.3.1	General	3560

G.3.2	Design of the test	3560

G.3.3	Numerical definition of the pass fail limits	3561

G.3.4	Pass fail decision rules	3562

G.4	Theory to derive the numbers in Table G.2.3-1 (informative)	3562

G.5	Theory to derive the numbers in Table G.3.3-1 (informative)	3562

G.5.1	Algorithm to calculate pass and fail probabilities	3562

G.5.2	Method of designing pass/fail limit to approach given risk probability	3566

Annex H (normative): Default message contents for RRM	3570

H.1	Void	3570

H.2	System information blocks message content exceptions	3570

H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection	3570

H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection	3572

H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection	3573

H.3	RRC message content exceptions	3574

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration	3574

H.3.2	RRC messages and information elements contents exceptions for handover	3591

H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover	3592

H.3.4	E-UTRA RRC messages and information elements contents exceptions	3593

H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting	3603

H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting	3606

H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used	3612

H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay	3614

H.3.9	RRC messages and information elements contents exceptions for UL timing	3615

Annex I (normative): RRM OTA procedures	3616

I.0	Test applicability per permitted test method	3616

I.1	Direct far field (DFF)	3616

I.1.1	RX beam peak direction search	3616

I.1.2	Search for directions corresponding to the EIS spherical coverage percentile	3616

I.2	Direct far field (DFF) simplification	3616

I.2.1	RX beam peak direction search	3616

I.2.2	Search for directions corresponding to the EIS spherical coverage percentile	3617

I.3	Indirect far field (IFF)	3617

I.3.1	RX beam peak direction search	3617

I.3.2	Search for directions corresponding to the EIS spherical coverage percentile	3617

I.3A	Enhanced indirect far field (Enhanced IFF)	3617

I.3A.1	RX beam peak direction search	3617

I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile	3617

I.4	Procedures to search test directions for RRM FR2	3617

I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search	3617

Annex J (informative): Change history	3619