| 3GPP TS 38.533 V17.8.0 (2023-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	40

1	Scope	41

2	References	41

3	Definitions, symbols and abbreviations	42

3.1	Definitions	42

3.2	Symbols	43

3.3	Abbreviations	44

3A	Requirements for the support of RRM	46

3A.1	General	46

3A.1.0	Overview of RRM requirements	46

3A.1.1	Test coverage across 5G NR connectivity options	47

3A.2	Requirements Classification for Statistical Testing	47

3A.3	Antenna Configuration	48

3A.4	NR band groups	48

3A.4.0	General	48

3A.4.1	NR operating bands in FR1	48

3A.4.1A	NR operating bands for satellite access in FR1	50

3A.4.2	NR operating bands in FR2	50

3A.5	NR operating band configuration	51

3A.6	UE with Multiband Capability	52

4	EN-DC with all NR cells in FR1	52

4.0	General	52

4.1	Void	53

4.2	Void	53

4.3	RRC_CONNECTED state mobility	53

4.3.1	Void	53

4.3.2	RRC connection mobility control	53

4.3.2.1	Void	53

4.3.2.2	Random access	53

4.3.2.2.1	EN-DC FR1 contention based random access	53

4.3.2.2.2	EN-DC FR1 non-contention based random access	59

4.3.2.2.3	EN-DC FR1 2-step contention based random access	66

4.3.2.2.4	EN-DC FR1 2-step non-contention based random access	71

4.3.2.3	Void	77

4.4	Timing	77

4.4.1	UE transmit timing	77

4.4.1.0	Minimum conformance requirements	77

4.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	77

4.4.1.1	EN-DC FR1 UE transmit timing accuracy	78

4.4.2	UE timer accuracy	84

4.4.3	Timing advance	84

4.4.3.0	Minimum conformance requirements	84

4.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	84

4.4.3.0.2	Minimum conformance requirements for timing advance adjustment delay	85

4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	85

4.5	Signaling characteristics	90

4.5.1	Radio link monitoring	90

4.5.1.00	General	90

4.5.1.0	Minimum conformance requirements	90

4.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	90

4.5.1.0.2	Void	91

4.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	91

4.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	92

4.5.1.0.5	Requirements for UE configured with Relaxed Measurement Criteria	94

4.5.1.1	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	95

4.5.1.2	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	100

4.5.1.3	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	105

4.5.1.4	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	110

4.5.1.5	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	115

4.5.1.6	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	120

4.5.1.7	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	125

4.5.1.9	EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	136

4.5.1.10	141

4.5.1.11	141

4.5.11.1	EN-DC FR1 Conditional PSCell Addition Delay	141

4.5.2	Interruption	146

4.5.2.0	Minimum conformance requirements	146

4.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX.	146

4.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	147

4.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	148

4.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	148

4.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	150

4.5.2.0.6	Minimum conformance requirements for interruptions due to RRM and RLM/BFD measurements on deactivated NR SCG	151

4.5.2.1	EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	151

4.5.2.2	EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	155

4.5.2.3	EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	159

4.5.2.4	EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	165

4.5.2.5	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	171

4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	177

4.5.2.7	181

4.5.2.8	EN-DC FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	182

4.5.2.9	EN-DC FR1 interruptions at E-UTRA SRS carrier based switching	187

4.5.2.10	EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	193

4.5.3	SCell activation and deactivation delay	197

4.5.3.0	Minimum conformance requirements	197

4.5.3.0.2	Fast SCell Activation Delay Requirement for Deactivated SCell	201

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	203

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	214

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX	216

4.5.3.5	EN-DC FR1 direct SCell activation at SCell addition of known SCell	218

4.5.3.6	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	227

4.5.3.7	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	236

4.5.4	UE UL carrier RRC reconfiguration delay	239

4.5.4.1	EN-DC FR1 UE UL carrier RRC reconfiguration delay	239

4.5.5	Link recovery procedures	247

4.5.5.0	Minimum conformance requirements	247

4.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	249

4.5.5.0.4	Requirements for Beam Failure Recovery in SCell	250

4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX	250

4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX	256

4.5.5.3	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	263

4.5.5.4	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX	269

4.5.5.5	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	276

4.5.5.6	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	282

4.5.5.7	EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX	288

4.5.5.8	EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	296

4.5.6	Active BWP switch delay	304

4.5.6.1	DCI-based and time-based active BWP switch	304

4.5.6.1.0	Minimum conformance requirements	304

4.5.6.1.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	307

4.5.6.1.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	314

4.5.6.2	RRC-based active BWP switch	324

4.5.6.2.0	Minimum conformance requirements	324

4.5.6.2.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	324

4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	330

4.5.6.3.0	Minimum conformance requirements	330

4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	332

4.5.6.4	341

4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	341

4.5.6.5.0	Minimum conformance requirements	341

4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	342

4.5.7	PSCell addition and release delay	350

4.5.7.0	Minimum conformance requirements	350

4.5.7.0.1	NR PSCell Addition Delay Requirement	350

4.5.7.0.2	NR PSCell Release Delay Requirement	351

4.5.7.1	EN-DC FR1 addition and release delay of known PSCell	351

4.5.8	UL switching	356

4.5.8.0	Minimum conformance requirements	356

4.5.8.1	EN-DC FR1 interruptions at switching between two uplink carriers	357

4.5.9	366

4.5.10	PSCell activation and deactivation delay	366

4.5.10.0	Minimum conformance requirements	366

4.5.10.1	EN-DC FR1 PSCell activation and deactivation delay	368

4.5.11	PSCell addition and release delay	374

4.5.11.0	Minimum conformance requirements	374

4.5.11.0.1	Measurement time	375

4.5.11.1	EN-DC FR1 Conditional PSCell Addition Delay	375

4.6	Measurement procedures	380

4.6.1	Intra-frequency measurements	380

4.6.1.0	Minimum conformance requirements	380

4.6.1.0.1	Minimum conformance requirements for event-triggered reporting without gap	380

4.6.1.0.2	Minimum conformance requirements for event-triggered measurements with gap	385

4.6.1.1	EN-DC FR1 event-triggered reporting without gap in non-DRX	387

4.6.1.2	EN-DC FR1 event-triggered reporting without gap in DRX	392

4.6.1.3	EN-DC FR1 event-triggered reporting with gap in non-DRX	395

4.6.1.4	EN-DC FR1 event-triggered reporting with gap in DRX	401

4.6.1.5	EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection	406

4.6.1.6	EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection	409

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	414

4.6.1.8	EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17	419

4.6.2	Inter-frequency measurements	425

4.6.2.0	Minimum conformance requirements for Inter-frequency measurements	425

4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX	428

4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX	433

4.6.2.3	Void	438

4.6.2.4	Void	438

4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	438

4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection	443

4.6.2.7	Void	448

4.6.2.8	Void	448

4.6.2.9	EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	448

4.6.3	Void	453

4.6.4	L1-RSRP measurement for beam reporting	453

4.6.4.0	Minimum conformance requirements	453

4.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	453

4.6.4.1	EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX	457

4.6.4.1.5	Test requirement	460

4.6.4.2	EN-DC FR1 SSB-based L1-RSRP measurement in DRX	461

4.6.4.2.3	Minimum conformance requirements	461

4.6.4.3	EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX	464

4.6.4.4	EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX	468

4.6.4.5	EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	472

4.6.5	CLI measurements	476

4.6.5.0	Minimum conformance requirements	476

4.6.5.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	477

4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX	478

4.6.6	486

4.6.7	L1-SINR measurement for beam reporting	486

4.6.7.0	Minimum conformance requirements	486

4.6.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	491

4.6.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	496

4.6.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	501

4.7	Measurement performance requirements	505

4.7.1	SS-RSRP	505

4.7.1.0	Minimum conformance requirements	505

4.7.1.1	Intra-frequency measurements	510

4.7.1.2	Inter-frequency measurements	518

4.7.2	SS-RSRQ	525

4.7.2.0	Minimum conformance requirements	525

4.7.2.1	EN-DC FR1 SS-RSRQ measurement accuracy	527

4.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	532

4.7.2.2.1	EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy	532

4.7.2.2.2	EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy	536

4.7.3	SS-SINR	538

4.7.3.0	Minimum conformance requirements	538

4.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	538

4.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	539

4.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	540

4.7.3.1	EN-DC FR1 SS-SINR measurement accuracy	540

4.7.3.2	Inter-Frequency SS-SINR measurement accuracy	544

4.7.3.2.1	EN-DC FR1-FR1 SS-SINR absolute measurement accuracy	544

4.7.3.2.2	EN-DC FR1-FR1 SS-SINR relative measurement accuracy	548

4.7.4	L1-RSRP	550

4.7.4.0	Minimum conformance requirements	550

4.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	550

4.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	553

4.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	554

4.7.4.1	SSB based L1-RSRP measurements	555

4.7.4.1.1	EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy	555

4.7.4.1.2	EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy	560

4.7.4.2	CSI-RS based L1-RSRP measurements	562

4.7.4.2.1	EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy	562

4.7.4.2.2	EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy	566

4.7.5	SFTD	568

4.7.5.0	Minimum conformance requirements	568

4.7.5.0.1	SFTD Accuracy Requirement	568

4.7.5.1	EN-DC FR1 SFTD measurement accuracy	569

4.7.6	CLI measurements	573

4.7.6.0	Minimum conformance requirements	573

4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy	573

4.7.6.0.1.1	SRS-RSRP report mapping	574

4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	575

4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	575

4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	584

4.7.7	L1-SINR measurement for beam reporting	587

4.7.7.0	Minimum conformance requirements	587

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	587

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	590

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	592

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	596

4.7.7.1.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	596

4.7.7.1.2	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	601

4.7.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	603

4.7.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement	608

4.7.7.3.1	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	608

4.7.7.3.2	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	614

4A	NE-DC with all NR cells in FR1	616

4A.0	General	616

4A.1	Signalling characteristics	616

4A.1.1	E-UTRAN PSCell addition	616

4A.1.1.0	Minimum conformance requirements	616

4A.1.1.0.1	E-UTRA PSCell Addition Delay Requirement	616

4A.1.1.0.2	E-UTRA PSCell Release Delay Requirement	617

4A.1.1.1	NE-DC FR1 addition and release delay of known PSCell	617

4A.1.2	Active BWP switch delay	623

4A.1.2.0	Minimum conformance requirements	623

4A.1.2.1	NE-DC FR1 DCI-based and timer-based DL active BWP switch in non-DRX in synchronous NE-DC	623

4A.2	Measurement performance requirements	627

4A.2.1	SFTD accuracy	627

4A.2.1.0	Minimum conformance requirements	627

4A.1.1.0.1	NE-DC SFTD accuracy Requirement	627

4A.2.1.1	NE-DC FR1 SFTD accuracy	629

5	EN-DC with at least one NR cell in FR2	635

5.0	General	635

5.1	Void	635

5.2	Void	635

5.3	RRC_CONNECTED state mobility	635

5.3.1	Void	635

5.3.2	RRC connection mobility control	635

5.3.2.1	Void	635

5.3.2.2	Random access	635

5.3.2.2.1	EN-DC FR2 contention based random access	635

5.3.2.2.2	EN-DC FR2 non-contention based random access	643

5.3.2.2.3	EN-DC FR2 2-step contention based random access	651

5.3.2.2.4	EN-DC FR2 2-step non-contention based random access	655

5.3.2.3	Void	659

5.4	Timing	659

5.4.1	UE transmit timing	659

5.4.1.0	Minimum Conformance Requirements	659

5.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	659

5.4.1.1	EN-DC FR2 UE transmit timing accuracy	661

5.4.2	UE timer accuracy	667

5.4.3	Timing advance	667

5.4.3.0	Minimum conformance requirements	667

5.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	667

5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	667

5.5	Signaling characteristics	672

5.5.1	Radio link monitoring	672

5.5.1.0	Minimum conformance requirements	672

5.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	672

5.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	673

5.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	674

5.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	677

5.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	680

5.5.1.0.6	Requirements for UE configured with Relaxed Measurement Criteria	681

5.5.1.1	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	681

5.5.1.2	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	688

5.5.1.3	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	694

5.5.1.4	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	700

5.5.1.5	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	705

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	710

5.5.1.7	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	716

5.5.1.9	EN-DC FR2 radio link monitoring UE scheduling restrictions	729

5.5.1.10	EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	734

5.5.2	Interruption	739

5.5.2.0	Minimum conformance requirements	739

5.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX	739

5.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	740

5.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	741

5.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	741

5.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	743

5.5.2.1	EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	744

5.5.2.2	EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	748

5.5.2.3	EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	752

5.5.2.4	EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	758

5.5.2.5	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	763

5.5.2.6	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	767

5.5.2.7	EN-DC FR2 interruptions at E-UTRA SRS carrier based switching	771

5.5.2.8	EN-DC FR2 interruptions at NR SRS carrier based switching	777

5.5.3	SCell activation and deactivation delay	783

5.5.3.1	EN-DC FR2 SCell activation and deactivation intra-band in non-DRX	783

5.5.3.2 to 5.5.3.6	791

5.5.3.7	EN-DC FR2 direct SCell activation at SCell addition of known SCell	791

5.5.3.8	EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band	795

5.5.4	UE UL carrier RRC reconfiguration delay	804

5.5.5	Link recovery procedures	804

5.5.5.0	Minimum conformance requirements	804

5.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	804

5.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	808

5.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	813

5.5.5.0.4	Requirements for Beam Failure Recovery in SCell	814

5.5.5.0.5	Requirements for SSB based beam failure detection for UE fulfilling relaxed measurement criteria	814

5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX	815

5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX	822

5.5.5.3	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	829

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX	835

5.5.5.5	EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX	842

5.5.5.6	EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX	847

5.5.5.7	EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX	854

5.5.5.8	EN-DC FR2 CSI-RS-based PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode	860

5.5.5.9	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion	867

5.5.6	Active BWP switch delay	874

5.5.6.1	DCI-based and time-based active BWP switch	874

5.5.6.1.0	Minimum conformance requirements	874

5.5.6.1.1	EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	874

5.5.6.1.2	EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	875

5.5.6.2	RRC-based active BWP switch	875

5.5.6.2.0	Minimum conformance requirements	875

5.5.6.2.1	EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	876

5.5.7	Void	883

5.5.8	Active TCI state switch delay	883

5.5.8.0	Minimum conformance requirements	883

5.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	883

5.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	884

5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch	885

5.5.8.2	EN-DC FR2 RRC based active TCI state switch	889

5.5.9	Uplink spatial relation switch delay	893

5.5.9.0	Minimum conformance requirements	893

5.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	893

5.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	893

-	TL1-RSRP is defined in clause 8.12.3.5.5.9.1 MAC-CE based uplink spatial relation switch	894

5.5.9.1.1	EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation	894

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.5.5.9.2	RRC based uplink spatial relation switch	898

5.5.9.2.1	EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS	898

5.5.10	FFS	902

5.5.11	Unified TCI state switch delay	902

5.5.11.0	Minimum conformance requirements	902

5.5.11.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	902

5.5.11.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	904

5.5.11.1	EN-DC FR2 MAC-CE based active joint TCI state switch	905

5.5.11.2	EN-DC FR2 MAC-CE based active uplink TCI state switch	909

5.5.11.3	EN-DC FR2 MAC-CE based active downlink TCI state switch	913

5.5.12	PSCell activation and deactivation delay	917

5.5.12.0	Minimum conformance requirements	917

5.5.12.1	EN-DC FR2 PSCell activation and deactivation delay	918

5.5.13	Conditional PSCell addition and release delay	922

5.5.13.1	EN-DC FR2 Addition and Release Delay of NR PSCell	922

5.6	Measurement procedures	927

5.6.1	Intra-frequency measurements	927

5.6.1.0	Minimum conformance requirements	927

5.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	927

5.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	930

5.6.1.1	EN-DC FR2 event-triggered reporting without gap in non-DRX	931

5.6.1.2	EN-DC FR2 event-triggered reporting without gap in DRX	937

5.6.1.3	EN-DC FR2 event-triggered reporting with gap in non-DRX	941

5.6.1.4	EN-DC FR2 event-triggered reporting with gap in DRX	947

5.6.2	Inter-frequency measurements	952

5.6.2.0	Minimum conformance requirements for Inter-frequency measurements	952

5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX	954

5.6.2.2	EN-DC FR2-FR2 event-triggered reporting in DRX	958

5.6.2.3	EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	963

5.6.2.4	EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection	967

5.6.2.5	EN-DC FR1-FR2 event-triggered reporting in non-DRX	972

5.6.2.6	EN-DC FR1-FR2 event-triggered reporting in DRX	977

5.6.2.7	EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	983

5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection	988

5.6.3	L1-RSRP measurement for beam reporting	993

5.6.3.0	Minimum conformance requirements	993

5.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	993

5.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	995

5.6.3.1	EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX	998

5.6.3.2	EN-DC FR2 SSB-based L1-RSRP measurement in DRX	1002

5.6.3.3	EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX	1006

5.6.3.4	EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX	1010

5.6.4	CLI measurements	1013

5.6.4.0	Minimum conformance requirements	1013

5.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	1014

5.6.4.1	EN-DC FR2 SRS-RSRP measurement in non-DRX	1015

5.6.4.2	EN-DC FR2 CLI-RSSI measurement in non-DRX	1018

5.6.5	1022

5.6.6	L1-SINR measurement for beam reporting	1022

5.6.6.0	Minimum conformance requirements	1022

5.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1022

5.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1025

5.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1027

5.6.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1029

5.6.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1034

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1039

5.7	Measurement performance requirements	1044

5.7.1	SS-RSRP	1044

5.7.1.0	Minimum conformance requirements	1044

5.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	1044

5.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	1045

5.7.1.1	EN-DC FR2 SS-RSRP measurement accuracy	1047

5.7.1.2	EN-DC FR2-FR2 SS-RSRP measurement accuracy	1054

5.7.1.3	EN-DC FR1-FR2 SS-RSRP measurement accuracy	1061

5.7.2	SS-RSRQ	1067

5.7.2.0	Minimum conformance requirements	1067

5.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1067

5.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	1067

5.7.2.1	EN-DC FR2 SS-RSRQ measurement accuracy	1069

5.7.2.2	EN-DC FR2-FR2 SS-RSRQ measurement accuracy	1073

5.7.3	SS-SINR	1078

5.7.3.0	Minimum conformance requirements	1078

5.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1078

5.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	1078

5.7.3.1	EN-DC FR2 SS-SINR measurement accuracy	1080

5.7.3.2	EN-DC FR2-FR2 SS-SINR measurement accuracy	1084

5.7.4	L1-RSRP	1089

5.7.4.0	Minimum conformance requirements	1089

5.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	1090

5.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	1090

5.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	1091

5.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	1092

5.7.4.1	EN-DC FR2 SSB based L1-RSRP measurement accuracy	1093

5.7.4.2	EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy	1099

5.7.5	SRS-RSRP	1105

5.7.5.0	Minimum conformance requirements	1105

5.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	1105

5.7.5.1	EN-DC FR2 SRS-RSRP measurement accuracy	1107

5.7.5.2	EN-DC FR2 CLI-RSSI measurement accuracy	1112

5.7.5.2.1	Test purpose	1113

5.7.5.2.2	Test applicability	1113

5.7.5.2.3	Minimum conformance requirements	1113

5.7.5.2.4	Test description	1113

5.7.5.2.5	Test requirement	1114

5.7.6	L1-SINR measurement for beam reporting	1117

5.7.6.0	Minimum conformance requirements	1117

5.7.6.0.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1117

5.7.6.0.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1118

5.7.6.0.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1121

5.7.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	1123

5.7.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1128

5.7.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1133

6	NR standalone in FR1	1139

6.0	General	1139

6.1	RRC_IDLE state mobility	1139

6.1.1	NR cell re-selection	1139

6.1.1.0	Minimum conformance requirements	1139

6.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	1139

6.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	1139

6.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with highSpeedMeasFlag-r16	1142

6.1.1.0.4	Minimum conformance requirements for intra-frequency cell re-selection when UE configured with relaxed measurement criterion	1142

6.1.1.0.5	Minimum conformance requirements for inter-frequency cell re-selection when UE configured with relaxed measurement criterion	1144

6.1.1.1	NR SA FR1 cell re-selection	1146

6.1.1.1.5	Test requirement	1150

6.1.1.2	NR SA FR1-FR1 cell re-selection	1152

6.1.1.3	NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1156

6.1.1.4	NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1163

6.1.1.5	NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1168

6.1.1.6	NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1175

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16	1181

6.1.1.8	NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17	1186

6.1.2	NR – E-UTRA cell re-selection	1192

6.1.2.0	Minimum conformance requirements	1192

6.1.2.0.1	Minimum conformance requirements for NR – E-UTRA cell re-selection	1192

6.1.2.0.2	Minimum conformance requirement for inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	1194

6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA	1195

6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA	1201

6.1.2.3	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1207

6.1.2.4	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1213

6.1.2.5	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16	1218

6.2	RRC_INACTIVE state mobility	1224

6.2.0	General	1224

6.2.0.1	Minimum conformance requirements for TA validation using CG-SDT	1224

6.2.1	NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)	1225

6.2.1.1	Test Purpose	1225

6.2.1.2	Test Applicability	1225

6.2.1.3	Minimum Conformance Requirement	1225

6.2.1.4	Test Description	1225

6.2.1.4.1	Initial conditions	1226

6.2.1.4.2	Test Procedure	1227

6.2.1.5	Test Requirement	1230

6.3	RRC_CONNECTED state mobility	1230

6.3.1	Handover	1230

6.3.1.0	Minimum conformance requirements	1230

6.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	1230

6.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	1231

6.3.1.0.3	Minimum conformance requirements for NR – UTRAN handover	1232

6.3.1.0.4	Minimum conformance requirements for NR FR1- NR FR1 DAPS handover	1232

6.3.1.1	NR SA FR1 handover with known target cell	1235

6.3.1.2	NR SA FR1 handover with unknown target cell	1239

6.3.1.3	NR SA FR1-FR1 handover with unknown target cell	1242

6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell	1245

6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell	1252

6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell	1257

6.3.1.7	NR SA FR1 synchronous DAPS handover	1263

6.3.1.8	NR SA FR1 asynchronous DAPS handover	1271

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover	1276

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover	1285

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover	1291

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover	1300

6.3.2	RRC connection mobility control	1308

6.3.2.1	RRC re-establishment	1308

6.3.2.1.0	Minimum conformance requirements	1308

6.3.2.1.1	NR SA FR1 RRC re-establishment	1310

6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment	1314

6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing	1318

6.3.2.2	Random access	1322

6.3.2.2.0	Minimum conformance requirements	1322

6.3.2.2.1	NR SA FR1 contention based random access	1324

6.3.2.2.2	NR SA FR1 non-contention based random access	1330

6.3.2.2.3	NR SA FR1 2-step contention based random access	1337

6.3.2.2.4	NR SA FR1 2-step non-contention based random access	1341

6.3.2.3	RRC connection release with redirection	1345

6.3.2.3.0	Minimum conformance requirements	1345

6.3.2.3.1	NR SA FR1 RRC connection release with redirection	1347

6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection	1350

6.3.3	Conditional handover	1355

6.3.3.0	Minimum conformance requirements	1355

6.3.3.0.1	Minimum conformance requirements for NR FR1 intra-frequency conditional handover	1355

6.3.3.0.2	Minimum conformance requirements for NR FR1 inter-frequency conditional handover	1358

6.3.3.1	NR SA FR1 conditional handover	1361

6.3.3.2	NR SA FR1-FR1 conditional handover	1367

6.4	Timing	1376

6.4.1	UE transmit timing	1376

6.4.1.0	Minimum conformance requirements	1376

6.4.1.1	NR SA FR1 UE transmit timing accuracy	1377

6.4.2	UE timer accuracy	1383

6.4.3	Timing advance	1383

6.4.3.0	Minimum conformance requirement	1383

6.4.3.0.1	Minimum conformance requirement for timing advance adjustment	1383

6.4.3.1	NR SA FR1 timing advance adjustment accuracy	1383

6.5	Signaling characteristics	1388

6.5.1	Radio link monitoring	1388

6.5.1.00	General	1388

6.5.1.0	Minimum conformance requirements	1390

6.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	1390

6.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	1391

6.5.1.0.3	Minimum conformance requirements for out-of-sync and in-sync CSI-RS based RLM	1393

6.5.1.0.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	1394

6.5.1.0.5	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	1394

6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1395

6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1401

6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	1407

6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	1412

6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1419

6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1426

6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1432

6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1439

6.5.1.9	SA FR1 radio link monitoring out-of-sync Test for PSCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	1445

6.5.1.9.1	Test purpose	1445

6.5.1.9.2	Test applicability	1446

6.5.1.9.3	Minimum conformance requirements	1446

6.5.1.9.4	Test description	1446

6.5.1.9.5	Test requirement	1451

6.5.2	Interruption	1453

6.5.2.0	Minimum conformance requirements	1453

6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	1453

6.5.2.0.2	 Interruptions at NR SRS carrier based switching	1454

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC	1456

6.5.2.2	SA FR1 interruptions at NR SRS carrier based switching	1461

6.5.3	SCell activation and deactivation delay	1466

6.5.3.0	Minimum conformance requirements	1466

6.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	1466

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	1467

6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	1480

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX	1481

6.5.3.4	NR SA FR1 direct SCell activation at SCell addition of known SCell	1483

6.5.3.5	NR SA FR1 direct SCell activation at handover with known SCell	1492

6.5.3.6 to 6.5.3.9	1501

6.5.3.10	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	1501

6.5.3.11	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	1509

6.5.4	UE UL carrier RRC reconfiguration delay	1512

6.5.4.0	Minimum conformance requirements	1512

6.5.4.0.1	Minimum conformance requirements for UL carrier RRC reconfiguration delay	1512

6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay	1512

6.5.5	Link recovery procedures	1520

6.5.5.0	Minimum conformance requirements	1520

6.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	1520

6.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	1520

6.5.5.0.4	Requirements for Beam Failure Recovery in SCell	1520

6.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	1520

6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX	1522

6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX	1530

6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	1537

6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX	1545

6.5.5.5	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	1552

6.5.5.6	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	1559

6.5.5.7	NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	1566

6.5.6	Active BWP switch delay	1574

6.5.6.1	DCI-based and time-based active BWP switch	1574

6.5.6.1.0	Minimum conformance requirements	1574

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX	1575

6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX	1586

6.5.6.2	RRC-based active BWP switch	1595

6.5.6.2.0	Minimum conformance requirements	1595

6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX	1595

6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1603

6.5.6.3.0	Minimum conformance requirements	1603

6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1603

6.5.6.4	1613

6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1613

6.5.6.5.0	Minimum conformance requirements	1613

6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch in non-DRX on multiple CCs	1614

6.5.7	DL interruptions at switching between two uplink carriers	1622

6.5.7.0	Minimum conformance requirements	1622

6.5.7.1	NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA	1622

6.5.7.2	NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA	1634

6.5.7A	NR SA FR1 DL interruptions at switching between two uplink carriers with two transmit antenna connectors	1642

6.5.7A.0	Minimum conformance requirements	1642

6.5.7A.1	Void	1643

6.5.7A.2	NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA	1643

6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	1651

6.5.7B.0	Minimum conformance requirements	1651

6.5.7B.1	Void	1652

6.5.7B.2	NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA	1652

6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	1661

6.5.7C.0	Minimum conformance requirements	1661

6.5.7C.1	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	1662

6.5.7C.1.1	Test purpose	1662

6.5.7C.1.2	Test applicability	1662

6.5.7C.1.3	Minimum conformance requirements	1662

6.5.7C.1.4	Test description	1662

6.5.7C.1.4.1	Initial conditions	1662

6.5.7C.2	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	1673

6.5.7C.2.1	Test purpose	1673

6.5.7C.2.2	Test applicability	1673

6.5.7C.2.3	Minimum conformance requirements	1673

6.5.7C.2.4	Test description	1674

6.5.7C.2.4.1	Initial conditions	1674

6.5.8	UE specific CBW change	1685

6.5.8.0	Minimum conformance requirements	1685

6.5.8.0.1	Minimum conformance requirements for UE specific CBW change	1685

6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1685

6.6	Measurement procedures	1692

6.6.1	Intra-frequency measurements	1692

6.6.1.0	Minimum conformance requirements	1692

6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1692

6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1692

6.6.1.0.3	Void	1693

6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading	1693

6.6.1.0.5	 Void	1695

6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX	1695

6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX	1699

6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX	1704

6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX	1710

6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading	1715

6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading	1719

6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	1723

6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17	1728

6.6.2	Inter-frequency measurements	1733

6.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1733

6.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps	1733

6.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps	1733

6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX	1736

6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX	1742

6.6.2.3	Void	1748

6.6.2.4	Void	1748

6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	1748

6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection	1754

6.6.2.7	Void	1760

6.6.2.8	Void	1760

6.6.2.9	NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern	1760

6.6.2.10	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used	1765

6.6.2.11	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used	1770

6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	1776

6.6.3	Inter-RAT Measurements	1782

6.6.3.0	Minimum conformance requirements	1782

6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	1782

6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	1784

6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX	1787

6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX	1794

6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16	1800

6.6.4	L1-RSRP measurement for beam reporting	1808

6.6.4.0	Minimum conformance requirements	1808

6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1808

6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1808

6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX	1808

6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX	1812

6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX	1816

6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX	1820

6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	1823

6.6.5	UTRAN inter-RAT measurement	1829

6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX	1829

6.6.6	CLI Measurements	1836

6.6.6.0	Minimum conformance requirements	1836

6.6.6.0.1	Minimum conformance requirements for SRS-RSRP measurement	1836

6.6.6.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	1836

6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX	1836

6.6.7	1843

6.6.8	L1-SINR measurement for beam reporting	1843

6.6.8.0	Minimum conformance requirements	1843

6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1843

6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1843

6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1843

6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1843

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1849

6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1855

6.6.9	Idle Mode CA/DC Measurements	1861

6.6.9.0	Minimum conformance requirements	1861

6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1	1863

6.6.10 to 6.6.14	1873

6.6.15	Idle Mode inter-RAT CA/DC Measurements	1873

6.6.15.0	Minimum conformance requirements	1873

6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1874

6.6.16	1887

6.6.17	NR SA FR1 event triggered reporting tests with Pre-MG	1887

6.6.17.1	NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG	1889

6.6.17.2	NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	1895

6.6.18	SA event triggered reporting tests with concurrent gaps	1903

6.6.18.0	Minimum conformance requirements	1903

6.6.18.0.1	Minimum conformance requirements for Intra-frequency measurement	1903

6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement	1904

6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement	1905

6.6.18.0.4	Minimum conformance requirements for PRS measurement	1906

6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers	1906

6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers	1914

6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	1921

6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	1933

6.6.19	SA event triggered reporting tests with NCSG	1942

6.6.19.0	Minimum conformance requirements	1942

6.6.19.0.1	Minimum conformance requirements for intra-frequency measurement	1942

6.6.19.0.2	Minimum conformance requirements for inter-frequency measurement	1944

6.6.19.0.3	Minimum conformance requirements for inter-RAT measurement	1945

6.6.19.0.4	Minimum conformance requirements for interruptions	1946

6.6.19.1	NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1	1947

6.6.19.2	NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement	1953

6.6.19.3	NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	1959

6.6.19.4	NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	1967

6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	1973

6.6.20.0	Minimum conformance requirements	1973

6.6.20.1	NR SA FR1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC	1976

6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS	1980

6.6.21.0	Minimum conformance requirements	1980

6.6.21.1	NR SA FR1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC	1983

6.7	Measurement performance requirements	1987

6.7.1	SS-RSRP	1987

6.7.1.0	Minimum conformance requirements	1987

6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	1987

6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	1987

6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	1987

6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	1988

6.7.1.1	Intra-frequency measurements	1988

6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy	1988

6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy	1995

6.7.1.2	Inter-frequency measurements	1997

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy	1997

6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy	2004

6.7.2	SS-RSRQ	2006

6.7.2.0	Minimum conformance requirements	2006

6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2006

6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	2006

6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	2006

6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy	2006

6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	2010

6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy	2010

6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy	2015

6.7.3	SS-SINR	2017

6.7.3.0	Minimum conformance requirements	2017

6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2017

6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	2017

6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	2017

6.7.3.1	NR SA FR1 SS-SINR measurement accuracy	2017

6.7.3.2	Inter-Frequency SS-SINR measurement accuracy	2021

6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy	2021

6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy	2025

6.7.4	L1-RSRP	2027

6.7.4.0	Minimum conformance requirements	2027

6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	2027

6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	2027

6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	2027

6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	2027

6.7.4.1	SSB based L1-RSRP measurements	2027

6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy	2027

6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy	2032

6.7.4.2	CSI-RS based L1-RSRP measurements	2034

6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy	2034

6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy	2038

6.7.5	E-UTRAN RSRP	2040

6.7.5.0	Minimum conformance requirements	2040

6.7.5.0.1	E-UTRAN RSRP absolute accuracy	2040

6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy	2041

6.7.6	E-UTRAN RSRQ	2048

6.7.6.0	Minimum conformance requirements	2048

6.7.6.0.1	E-UTRAN RSRQ absolute accuracy	2048

6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy	2049

6.7.7	E-UTRAN RS-SINR	2056

6.7.7.0	Minimum conformance requirements	2056

6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy	2056

6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy	2057

6.7.8	CLI Measurements	2064

6.7.8.0	Minimum conformance requirements	2064

6.7.8.0.1	Minimum conformance requirements for SRS-RSRP accuracy	2064

6.7.8.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	2064

6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy	2064

6.7.8.1.1	Test purpose	2064

6.7.8.1.2	Test applicability	2064

6.7.8.1.3	Minimum conformance requirements	2065

6.7.8.1.4	Test description	2065

6.7.8.1.5	Test requirement	2067

6.7.9	L1-SINR measurement for beam reporting	2074

6.7.9.0	Minimum conformance requirements	2074

6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2074

6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	2077

6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	2079

6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	2083

6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	2083

6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	2086

6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2088

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2091

6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2092

6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	2095

7	NR standalone with at least one NR cell in FR2	2098

7.0	General	2098

7.1	RRC_IDLE state mobility	2098

7.1.1	NR cell re-selection	2098

7.1.1.0	Minimum conformance requirements	2098

7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	2098

7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	2101

7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion	2102

7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion	2103

7.1.1.1	NR SA FR2 cell re-selection	2105

7.1.1.2	NR SA FR2-FR2 cell re-selection	2110

7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2118

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2123

7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2128

7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2135

7.2	RRC_INACTIVE state mobility	2147

7.2.1	Small Data Transmission	2147

7.2.1.0	Minimum conformance requirements	2147

7.2.1.0.1	Minimum conformance requirements for CG-SDT TA validation	2147

7.2.1.1	TA Validation for CG-SDT in FR2	2148

7.3	RRC_CONNECTED state mobility	2153

7.3.1	Handover	2153

7.3.1.0	Minimum conformance requirements	2153

7.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	2153

7.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR2 handover	2154

7.3.1.0.3	Minimum conformance requirements for NR FR1 – NR FR2 DAPS handover	2155

7.3.1.1	NR SA FR1-FR2 Inter-frequency handover; unknown target cell	2156

7.3.1.2	NR SA FR2 Intra-frequency handover; unknown target cell	2161

7.3.1.3	NR SA FR2-FR2 Inter-frequency handover; unknown target cell	2165

7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover	2169

7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover	2179

7.3.2	RRC connection mobility control	2186

7.3.2.1	RRC re-establishment	2186

7.3.2.1.0	Minimum conformance requirements	2186

7.3.2.1.1	NR SA FR2 RRC re-establishment	2187

7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment	2191

7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	2194

7.3.2.2	Random access	2199

7.3.2.2.0	Minimum conformance requirements	2199

7.3.2.2.1	NR SA FR2 contention based random access	2200

7.3.2.2.2	NR SA FR2 non-contention based random access	2208

7.3.2.2.3	NR SA FR2 2-step contention based random access	2215

7.3.2.2.4	NR SA FR2 2-step non-contention based random access	2222

7.3.2.3	RRC connection release with redirection	2229

7.3.2.3.0	Minimum conformance requirements	2229

7.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection	2230

7.3.3	Conditional handover	2234

7.3.3.0	Minimum conformance requirements	2234

7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover	2234

7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover	2238

7.3.3.1	NR SA FR2 conditional handover	2242

7.3.3.2	NR SA FR2-FR2 conditional handover	2249

7.4	Timing	2256

7.4.1	UE transmit timing	2256

7.4.1.0	Minimum Conformance Requirements	2256

7.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	2256

7.4.1.1	NR SA FR2 UE transmit timing accuracy	2257

7.4.2	UE timer accuracy	2263

7.4.3	Timing advance	2263

7.4.3.0	Minimum conformance requirements	2263

7.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	2263

7.4.3.1	NR SA FR2 timing advance adjustment accuracy	2264

7.5	Signalling characteristics	2270

7.5.1	Radio link monitoring	2270

7.5.1.0	Minimum conformance requirements	2270

7.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	2270

7.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	2270

7.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	2270

7.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	2270

7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	2270

7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2270

7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2276

7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2284

7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2289

7.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	2296

7.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2301

7.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM RS in DRX mode	2308

7.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2313

7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	2319

7.5.2	Interruption	2324

7.5.2.0	Minimum conformance requirements	2324

7.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	2324

7.5.2.1	NR SA FR2 interruptions during measurements on deactivated NR SCC	2325

7.5.3	SCell activation and deactivation delay	2330

7.5.3.0	Minimum conformance requirements	2330

7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell	2330

7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell	2333

7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay	2333

7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay	2335

7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	2338

7.5.3.4	NR SA FR2 direct SCell activation at SCell addition of known SCell	2346

7.5.3.5	NR SA FR2 direct SCell activation at handover with known SCell	2353

7.5.3.6 to 7.5.3.12	2361

7.5.3.13	NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX	2361

7.5.3.14	NR SA FR2 SCell Activation for known SCell in FR2 inter-band	2368

7.5.4	Void	2375

7.5.5	Link recovery procedures	2375

7.5.5.0	Minimum conformance requirements	2375

7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	2375

7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	2375

7.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	2376

7.5.5.0.4	Requirements for Beam Failure Recovery in SCell	2376

7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX	2376

7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX	2381

7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	2388

7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX	2393

7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX	2400

7.5.5.6	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX	2405

7.5.5.7	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX	2412

7.5.5.8	FFS	2418

7.5.5.9	NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	2418

7.5.5.10	NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX	2424

7.5.6	Active BWP switch delay	2431

7.5.6.1	DCI-based and time-based active BWP switch	2431

7.5.6.1.0	Minimum conformance requirements	2431

7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2431

7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX	2432

7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2433

7.5.6.2	RRC-based active BWP switch	2434

7.5.6.2.0	Minimum conformance requirements	2434

7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX	2435

7.5.7	PSCell addition and release delay	2437

7.5.7.0	Minimum conformance requirements	2437

7.5.7.0.1	Minimum conformance requirements for PSCell addition delay	2437

7.5.7.0.2	Minimum conformance requirements for PSCell release delay	2438

7.5.7.1	NR SA FR2 addition and release delay of known PSCell	2438

7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell	2440

7.5.8	Active TCI state switch delay	2442

7.5.8.0	Minimum conformance requirements	2442

7.5.8.0.0	Known conditions for TCI state	2442

7.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	2442

7.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	2443

7.5.8.0.3 	Minimum conformance requirements for MAC-CE based active TCI state switch in HST FR2	2444

7.5.8.1	NR SA FR2 MAC-CE based active TCI state switch	2444

7.5.8.1.1	NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state	2444

7.5.8.2	NR SA FR2 RRC based active TCI state switch	2451

7.5.8.2.1	NR SA Pcell FR2 RRC based active TCI state switch for a known TCI state	2451

7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	2458

7.5.8.3.1	NR SA FR2 HST active TCI state switch for a known TCI state	2458

7.5.9	Uplink spatial relation switch delay	2464

7.5.9.0	Minimum conformance requirements	2464

7.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	2464

7.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	2464

7.5.9.1	MAC-CE based uplink spatial relation switch	2465

7.5.9.1.1	NR PCell FR2 uplink spatial relation switch associated with known DL-RS	2465

The rate of correct events observed during repeated tests shall be at least [90]%.7.5.9.2	RRC based uplink spatial relation switch	2468

7.5.9.2.1	NR PCell FR2 uplink spatial relation switch associated with known DL-RS	2468

7.5.10	2472

7.5.11	UE UL carrier RRC reconfiguration delay	2472

7.5.11.1	UE UL carrier RRC reconfiguration delay	2472

7.5.12	Conditional PSCell addition and release delay (FR2 SA)	2478

7.5.12.1	NR SA FR2 Addition and Release Delay of PSCell	2478

7.5.13	Unified TCI state switch delay	2482

7.5.13.0	Minimum conformance requirements	2482

7.5.13.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	2482

7.5.13.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	2483

7.5.13.1	NR SA FR2 MAC-CE based active joint TCI state switch	2483

7.5.13.2	NR SA FR2 MAC-CE based active uplink TCI state switch	2486

7.5.13.3	NR SA FR2 MAC-CE based active downlink TCI state switch	2490

7.5.14	2494

7.5.15	2494

7.5.15.1	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	2494

7.6	Measurement procedures	2500

7.6.1	Intra-frequency measurements	2500

7.6.1.0	Minimum conformance requirements	2500

7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	2500

7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	2501

7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX	2503

7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX	2508

7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX	2513

7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	2519

7.6.2	Inter-frequency measurements	2525

7.6.2.0	Minimum conformance requirements for Inter-frequency measurements	2525

7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX	2525

7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX	2530

7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	2536

7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection	2541

7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX	2547

7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX	2553

7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	2559

7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection	2565

7.6.3	L1-RSRP measurement for beam reporting	2571

7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	2571

7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	2571

7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	2571

7.6.3.0.3	Minimum conformance requirements for Inter-cell SSB based L1-RSRP Reporting	2571

7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX	2573

7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX	2579

7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX	2583

7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX	2587

7.6.3.5	2590

7.6.3.6	NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	2590

7.6.4	CLI measurements	2596

7.6.4.0	Minimum conformance requirements	2596

7.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	2596

7.6.4.1	NR SA FR2 SRS-RSRP measurement in non-DRX	2597

7.6.4.2	NR SA FR2 CLI-RSSI measurement in non-DRX	2601

7.6.4.2.1	Test purpose	2601

7.6.4.2.2	Test applicability	2601

7.6.4.2.3	Minimum conformance requirements	2601

7.6.4.2.4	Test description	2601

7.6.4.2.5	Test requirement	2603

7.6.5	2604

7.6.6	L1-SINR measurement for beam reporting	2604

7.6.6.0	Minimum conformance requirements	2604

7.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	2604

7.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	2604

7.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	2604

7.6.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	2604

7.6.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	2608

7.6.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	2613

7.6.7 to 7.6.12	2619

7.6.13	UE Rx-Tx time difference measurements for PDC	2619

7.6.13.0	Minimum conformance requirements	2619

7.6.13.0.1	PRS based UE Rx-Tx time difference for propagation delay compensation	2619

7.6.13.0.2	TRS based UE Rx-Tx time difference for propagation delay compensation	2621

7.6.13.1	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS	2623

7.6.13.2	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS	2627

7.7	Measurement performance requirements	2631

7.7.1	SS-RSRP	2631

7.7.1.0	Minimum conformance requirements	2631

7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	2631

7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	2631

7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy	2631

7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy	2639

7.7.1.3	Inter-frequency measurements between FR1 and FR2	2647

7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy	2647

7.7.1.3.2	Void	2654

7.7.2	SS-RSRQ	2654

7.7.2.0	Minimum conformance requirements	2654

7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2654

7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	2654

7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy	2654

7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy	2658

7.7.3	SS-SINR	2664

7.7.3.0	Minimum conformance requirements	2664

7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2664

7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	2664

7.7.3.1	NR SA FR2 SS-SINR measurement accuracy	2664

7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy	2669

7.7.4	L1-RSRP	2676

7.7.4.0	Minimum conformance requirements	2676

7.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	2676

7.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	2676

7.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	2676

7.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	2676

7.7.4.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	2676

7.7.4.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy	2681

7.7.5	SRS-RSRP	2687

7.7.5.0	Minimum conformance requirements	2687

7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	2687

7.7.5.1	NR SA FR2 SRS-RSRP measurement accuracy	2689

7.7.5.2	NR SA FR2 CLI-RSSI measurement accuracy	2694

7.7.5.2.1	Test purpose	2695

7.7.5.2.2	Test applicability	2695

7.7.5.2.3	Minimum conformance requirements	2695

7.7.5.2.4	Test description	2695

7.7.5.2.4	Test description	2695

7.7.5.2.5	Test requirement	2696

7.7.6	L1-SINR	2699

7.7.6.0	Minimum conformance requirements	2699

7.7.6.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2699

7.7.6.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR L1-SINR measurement accuracy	2701

7.7.6.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2703

7.7.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	2706

7.7.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy	2711

7.7.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2716

8.0	General	2722

8.1	Void	2722

8.2.1.0	Minimum conformance requirements	2722

8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection	2722

8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell	2724

8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2731

8.2.2.0	Minimum conformance requirements	2738

8.2.2.1	E-UTRA – NR FR1 Early Measurement Reporting	2740

8.2.2.2	E-UTRA – NR FR2 Early Measurement Reporting	2748

8.3.1.0	Minimum conformance requirements	2753

8.3.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 handover	2753

8.3.1.1	E-UTRA - NR FR1 handover with known target cell	2754

8.3.1.1.3	Minimum conformance requirements	2754

8.4.1.0	Minimum conformance requirements	2759

8.4.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 SFTD measurement delay	2759

8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX	2760

8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX	2766

8.4.2.0	Minimum conformance requirements	2771

8.4.2.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	2771

8.4.2.0.2	Void	2775

8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX	2775

8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX	2781

8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX	2787

8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX	2793

8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX	2799

8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX	2804

8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX	2809

8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX	2814

8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2819

8.5.1	SFTD measurement accuracy	2825

8.5.1.0	Minimum conformance requirements	2825

8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	2825

8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy	2826

8.5.2	Inter-RAT measurement accuracy	2832

8.5.2.1	SS-RSRP	2832

8.5.2.1.0	Minimum conformance requirements	2832

8.5.2.1.1	SS-RSRP with NR FR1 target cell	2833

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell	2838

8.5.2.2	SS-RSRQ	2843

8.5.2.2.0	Minimum conformance requirements	2843

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell	2843

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell	2846

8.5.2.3	SS-SINR	2851

8.5.2.3.0	Minimum conformance requirements	2851

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell	2851

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell	2854

9	NR sidelink	2859

9.1	NR sidelink in FR1	2859

9.1.1	UE transmit timing	2859

9.1.1.0	Minimum conformance requirements	2859

9.1.1.0.1	Minimum conformance requirements for GNSS as synchronization reference source	2859

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	2859

9.1.1.0.3	Minimum conformance requirements for FR1 NR Cell as synchronization reference source	2859

9.1.1.1	NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source	2860

9.1.1.2	NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source	2862

9.1.1.3	NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source	2865

9.1.2	Initiation/Cease of S-SSB transmission	2870

9.1.2.0	Minimum conformance requirements	2870

9.1.2.0.1	Minimum conformance requirements for FR1 NR cell as synchronization reference source	2870

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	2871

9.1.2.1	NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source	2872

9.1.2.2	NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source	2877

9.1.3	Synchronization reference selection/reselection	2881

9.1.3.0	Minimum conformance requirements	2881

9.1.3.0.1	Minimum conformance requirements for GNSS configured as the highest priority synchronization reference source	2881

9.1.3.0.2	Minimum conformance requirements for eNB/gNB configured as the highest priority synchronization reference source	2882

9.1.3.1	NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source	2884

9.1.3.2	NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source	2890

9.1.4	L1 SL-RSRP measurements	2895

9.1.4.0	Minimum conformance requirements	2895

9.1.4.0.1	Minimum conformance requirements for resource selection/reselection, re-evaluation and pre-emption	2895

9.1.4.1	NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection	2896

9.1.4.2	NR SA FR1 L1 SL-RSRP measurement for resource pre-emption	2902

9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation	2908

9.1.5	Congestion control measurement	2915

9.1.5.0	Minimum conformance requirements	2915

9.1.5.0.1	Minimum conformance requirements for congestion control measurements	2915

9.1.5.1	NR SA FR1 congestion control measurement for concurrent operation	2916

9.1.5.2	NR SA FR1 congestion control measurement for PC5-only operation	2924

9.1.6	Congestion control measurement	2930

9.1.6.0	Minimum conformance requirements	2930

9.1.6.0.1	Minimum conformance requirements for interruption to WAN due to NR sidelink communication	2930

9.1.6.1	NR SA FR1 interruption to WAN due to NR sidelink communication	2930

10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2934

10.1	RRC_CONNECTED state mobility	2934

10.2	Timing	2934

10.3	Signalling characteristics	2935

10.3.1	Radio link monitoring	2935

10.3.1.0	General	2935

10.3.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring under CCA	2936

10.3.1.1	Requirements for determining UE in-sync or out-of-sync status	2938

10.3.1.2	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	2939

10.3.1.3	EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	2947

10.3.1.4	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell under CCA configured with SSB-based RLM RS in DRX mode	2953

10.3.1.5	EN-DC FR1 radio link monitoring in-sync test for PSCell under CCA configured with SSB-based RLM RS in DRX mode	2953

10.3.2	Interruption	2954

10.3.2.0	Minimum Conformance Requirements	2954

10.3.2.1	EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA	2954

10.3.3	SCell activation and deactivation delay	2960

10.3.3.0	Minimum Conformance Requirements	2960

10.3.3.0.1	SCell Activation Delay Requirement for Deactivated SCell under CCA	2960

10.3.3.0.2	SCell Deactivation Delay Requirement for Activated SCell under CCA	2965

10.3.3.1	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	2965

10.3.3.2	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	2972

10.3.3.3	EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	2976

10.3.4	Beam failure detection and link recovery procedures	2980

10.3.4.0	General	2980

10.3.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	2981

10.3.4.0.3	Scheduling availability of UE during beam failure detection under CCA	2983

10.3.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	2983

10.3.4.1	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode	2983

10.3.4.2	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode	2991

10.4	Measurement procedures	2999

10.4.1	Intra-frequency measurements	2999

10.4.1.0	Minimum conformance requirements	2999

10.4.1.1	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA	3013

10.4.1.2	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under DRX and CCA	3015

10.4.1.3	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX and CCA	3018

10.4.1.4	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA	3021

10.5	Measurement Performance	3024

10.5.1	SS-RSRP	3024

10.5.1.0	Minimum Conformance Requirements	3024

10.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3025

10.5.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	3025

10.5.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3026

10.5.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	3026

10.5.1.1	EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell	3027

10.5.1.2	EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell	3032

10.5.2	SS-RSRQ	3038

10.5.2.0	Minimum Conformance Requirements	3038

10.5.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3038

10.5.2.0.2	Void	3039

10.5.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3039

10.5.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3040

10.5.2.1	EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	3040

10.5.2.2	EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	3046

10.5.3	SS-SINR	3053

10.5.3.0	Minimum Conformance Requirements	3053

10.5.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	3053

10.5.3.0.2	Void	3053

10.5.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	3053

10.5.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	3054

10.5.3.1	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA	3055

10.5.3.2	EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA	3060

10.5.3.3	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA	3066

11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	3072

11.1	RRC_IDLE state mobility	3073

11.2	RRC_CONNECTED state mobility	3073

11.3	Timing	3073

11.4	Signalling characteristics	3073

11.4.1	Radio link monitoring	3073

11.4.1.0	General	3073

11.4.1.0.1	Minimum conformance requirements for SSB based radio link monitoring under CCA	3073

11.4.1.1	Requirements for determining UE in-sync or out-of-sync status	3073

11.4.1.2	NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	3073

11.4.1.3	NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	3081

11.4.2	Interruptions	3087

11.4.3	SCell activation and deactivation delay	3087

11.4.4	Beam failure detection and link recovery procedures	3088

11.4.4.0	General	3088

11.4.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	3088

11.4.4.0.2	Void	3088

11.4.4.0.3	Scheduling availability of UE during beam failure detection under CCA	3088

11.4.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	3088

11.4.4.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA	3088

11.4.4.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA	3097

11.5	3105

11.6	Measurement Performance	3105

11.6.1	SS-RSRP	3105

11.6.1.0	Minimum Conformance Requirements	3105

11.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3105

11.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	3105

11.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3105

11.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	3105

11.6.1.1	NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA	3105

11.6.1.2	NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	3111

12 to 13	FFS	3116

14	NR standalone tests for Satellite access	3116

14.0	General	3116

14.0.1	Principle of testing GSO and NGSO scenarios	3117

14.0.2	Principle of testing different RRM requirements	3117

14.0.3	Principle of testing different ephemeris formats	3118

14.0.4	General setup for SIB19	3120

14.1	RRC_IDLE state mobility	3120

14.2	RRC_CONNECTED state mobility	3121

14.3	Timing for Satellite Access	3121

14.4	Signalling characteristics	3121

14.5	Measurement procedure	3121

14.5.1	Intra-frequency Measurements	3121

14.5.1.0	Minimum conformance requirements	3121

14.5.1.1	SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access	3124

14.5.1.2	SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access	3128

14.5.1.3	SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access	3131

15	FFS	3135

16	NR standalone tests with all NR cells in FR1 for RedCap	3136

16.0	General	3136

16.1	RRC_IDLE state mobility	3136

16.1.1	NR cell re-selection	3136

16.1.1.0	Minimum conformance requirements	3136

16.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	3136

16.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	3138

16.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3142

16.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3145

16.1.1.1	NR SA FR1 Cell reselection for 1 Rx UE	3148

16.1.1.2	NR SA FR1 Cell reselection for 2 Rx UE	3155

16.1.1.3	NR SA FR1-FR1 Cell reselection for 1 Rx UE	3159

16.1.1.4	NR SA FR1-FR1 Cell reselection for 2 Rx UE	3164

16.1.1.5	NR SA FR1  Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3168

16.1.1.6	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3176

16.1.1.7	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3181

16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3189

16.1.2	NR – E-UTRA cell re-selection	3197

16.1.2.0	Minimum conformance requirements	3197

16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx	3197

16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx	3199

16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection	3199

16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX	3201

16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX	3207

16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX	3211

16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX	3217

16.1.2.5	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3221

16.1.2.6	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3227

16.2	3231

16.3	RRC_CONNECTED state mobility for RedCap	3231

16.3.1	Handover	3231

16.3.1.0	Minimum conformance requirements	3231

16.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	3231

16.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	3232

16.3.1.1	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE	3232

16.3.1.2	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE	3236

16.3.1.3	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	3240

16.3.1.4	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	3246

16.3.1.5	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	3251

16.3.1.6	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	3257

16.3.1.7	NR SA FR1 - E-UTRA handover for 1Rx UE	3264

16.3.1.8	NR SA FR1 - E-UTRA handover for 2Rx UE	3269

16.3.1.9	NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE	3274

16.3.1.10	NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE	3278

16.3.2	RRC connection mobility control for RedCap	3282

16.3.2.1	RRC re-establishment for RedCap	3282

16.3.2.1.0	Minimum conformance requirements	3282

16.3.2.1.1	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	3282

16.3.2.1.2	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	3285

16.3.2.1.3	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	3288

16.3.2.1.4	NNR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	3291

16.3.2.1.5	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	3294

16.3.2.1.6	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	3297

16.3.2.2	Random access for RedCap	3300

16.3.2.2.0	Minimum conformance requirements	3300

16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	3301

16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	3304

16.3.2.2.3	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	3307

16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	3310

16.3.2.2.5	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	3313

16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	3316

16.3.2.2.7	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	3319

16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	3322

16.3.2.3	RRC connection release with redirection for RedCap	3325

16.3.2.3.0	Minimum conformance requirements	3325

16.3.2.3.1	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	3325

16.3.2.3.2	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	3329

16.3.2.3.3	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	3332

16.3.2.3.4	NR SA FR1-FR1 Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	3337

16.4	Timing for RedCap	3342

16.4.1	UE transmit timing for RedCap	3342

16.4.1.0	Minimum conformance requirements	3342

16.4.1.1	NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	3343

16.4.1.2	NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	3347

16.4.2	3350

16.4.3	Timing advance for RedCap	3350

16.4.3.0	Minimum conformance requirement	3350

16.4.3.1	NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE	3350

16.4.3.2	NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE	3353

16.5	Signalling characteristics for RedCap	3356

16.5.1	Radio link monitoring for RedCap	3356

16.5.1.0	Minimum requirements	3356

16.5.1.0.1	General	3356

16.5.1.0.2	Minimum conformance requirements for SSB-based RLM	3360

16.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	3361

16.5.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	3362

16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	3366

16.5.1.3	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	3370

16.5.1.4	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	3375

16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	3379

16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	3384

16.5.1.7	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	3389

16.5.1.8	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	3394

16.5.1.9	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3398

16.5.1.10	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	3401

16.5.1.11	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3404

16.5.1.12	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	3407

16.5.1.13	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	3411

16.5.1.14	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	3414

16.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	3417

16.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	3420

16.5.2	Beam Failure Detection and Link recovery procedures for RedCap	3423

16.5.2.0	Minimum requirements	3423

16.5.2.0.1	General	3423

16.5.2.0.2	Minimum conformance requirements for SSB-based BFD and CBD	3425

16.5.2.0.3	Minimum conformance requirements for CSI-RS based RLM	3427

16.5.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	3429

16.5.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	3434

16.5.2.3	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 1 Rx UE	3439

16.5.2.4	NR SA FR1 SSB-based beam failure detection and link recovery in DRX mode for 2 Rx UE	3445

16.5.2.5	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	3450

16.5.2.6	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	3455

16.5.2.7	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	3458

16.5.2.8	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	3461

16.5.3	Active BWP switch delay for RedCap	3466

16.5.3.1	DCI-based and time-based active BWP switch for RedCap	3466

16.5.3.1.0	Minimum conformance requirements	3466

16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE	3467

16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE	3471

16.5.3.2	RRC-based active BWP switch for RedCap	3475

16.5.3.2.0	Minimum conformance requirements	3475

16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE	3476

16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE	3480

16.5.4	UE specific CBW change for RedCap	3484

16.5.4.0	Minimum conformance requirements	3484

16.5.4.0.1	Minimum conformance requirements for UE specific CBW change	3484

16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE	3485

16.5.4.2	NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE	3487

16.6	Measurement procedures for RedCap	3489

16.6.1	Intra-frequency measurements for RedCap	3490

16.6.1.0	Minimum conformance requirements	3490

16.6.1.0.1	General	3490

16.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	3491

16.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	3493

16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE	3494

16.6.1.2	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE	3500

16.6.1.3	NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE	3506

16.6.1.4	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE	3508

16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	3511

16.6.1.6	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	3514

16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	3517

16.6.1.8	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	3520

16.6.1.9	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	3523

16.6.1.10	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	3529

16.6.1.11	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	3534

16.6.1.12	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2Rx UE	3537

16.6.2	Inter-frequency measurements for RedCap	3540

16.6.2.0	Minimum conformance requirements	3540

16.6.2.0.1	Minimum conformance requirements for inter-frequency event-triggered measurement with measurement gaps	3540

16.6.2.0.2	Minimum conformance requirements for inter-frequency event-triggered measurements without measurement gaps	3542

16.6.2.1	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	3545

16.6.2.2	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	3551

16.6.2.3	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	3557

16.6.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	3562

16.6.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	3566

16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	3566

16.6.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	3566

16.6.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	3570

16.6.2.9	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	3575

16.6.2.10	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	3580

16.6.2.11	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	3585

16.6.2.12	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	3589

16.6.3	Inter-RAT measurements for RedCap	3595

16.6.3.0	Minimum conformance requirements	3595

16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE	3595

16.6.3.2	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE	3595

16.6.3.3	NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE	3595

16.6.3.4	NR - E-UTRA event-triggered reporting in DRX in FR1 for 2 Rx UE	3595

16.6.4	L1-RSRP measurement for beam reporting for RedCap	3595

16.6.4.0	Minimum conformance requirements	3595

16.6.4.0.1	General	3595

16.6.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	3600

16.6.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	3604

16.6.4.3	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	3607

16.6.4.4	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	3611

16.6.4.5	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	3614

16.6.4.6	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	3617

16.6.4.7	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	3619

16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	3622

16.6.5	3624

16.6.6	3624

16.6.7	NR measurements with autonomous gaps	3624

16.6.7.0	Minimum conformance requirements	3624

16.6.7.0.1	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	3624

16.6.7.0.2	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	3626

16.6.7.1	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	3626

16.6.7.2	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	3629

17	NR standalone tests with at least one NR cell in FR2 for RedCap	3632

17.1	RRC_IDLE state mobility	3632

17.1.1	NR cell re-selection	3632

17.1.1.0	Minimum conformance requirements	3632

17.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	3632

17.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	3634

17.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3637

17.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3638

17.1.1.1	NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx	3640

17.1.1.2	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx	3646

17.1.1.3	NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3652

17.1.1.4	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	3657

17.2	3663

17.3	3663

17.3.1	3663

17.3.2	RRC connection mobility control for RedCap	3663

17.3.2.1	RRC re-establishment for RedCap	3663

17.3.2.1.0	Minimum conformance requirements	3663

17.3.2.1.1	NR SA FR2 Intra-frequency RRC Re-establishment in FR2	3663

17.3.2.1.2	NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2	3665

17.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	3667

17.3.2.2	Random access for RedCap	3670

17.3.2.2.0	Minimum conformance requirements	3670

17.3.2.2.0.1	Minimum conformance requirements for Contention based random access	3670

17.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access	3670

17.3.2.2.1	NR SA FR2 4-step RA type contention based random access test in FR2 for NR Standalone	3671

17.3.2.2.2	NR SA FR2 4-step RA type non-contention based random access test in FR2 for NR Standalone	3674

17.3.2.2.3	NR SA FR2 2-step RA type contention based random access test in FR2 for NR Standalone	3677

17.3.2.2.4	NR SA FR2 2-step RA type non-contention based random access test in FR2 for NR Standalone	3680

17.4	3683

17.5	Signalling characteristics for RedCap	3683

17.5.1	Radio link monitoring for RedCap	3683

17.5.1.0	Minimum requirements	3683

17.5.1.0.1	General	3683

17.5.1.0.2	Minimum conformance requirements for SSB-based RLM	3686

17.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	3688

17.5.1.1	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3690

17.5.1.2	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	3693

17.5.1.4	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	3698

17.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	3701

17.5.2	Beam Failure Detection and Link recovery procedures for RedCap	3703

17.5.2.0	Minimum conformance requirements	3703

17.5.2.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	3703

17.5.2.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	3704

17.5.2.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	3704

17.5.2.1 – 17.5.2.2	3704

17.5.2.3	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	3704

17.5.2.4	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	3708

17.5.2.5	NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	3712

17.6	Measurement procedures for RedCap	3716

17.6.1	Intra-frequency measurements for RedCap	3716

17.6.1.0	Minimum conformance requirements	3716

17.6.1.0.1	General	3716

17.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	3717

17.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	3720

17.6.1.1	NR SA FR2 Event triggered reporting test without gap under non-DRX	3721

17.6.1.2	NR SA FR2 Event triggered reporting test without gap under DRX	3723

17.6.1.3	NR SA FR2 Event triggered reporting test with per-UE gaps under non-DRX	3725

17.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	3732

17.6.2	3735

17.6.3	L1-RSRP measurement for beam reporting for RedCap	3735

17.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	3735

17.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	3735

17.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	3735

17.6.3.1	NR SA FR2 SSB based L1-RSRP measurement when DRX is not used	3735

17.6.3.2	NR SA FR2 SSB based L1-RSRP measurement when DRX is used	3737

17.6.3.3	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is not used	3739

17.6.3.4	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used	3741

18	E-UTRA - NR inter-RAT with E-UTRA serving cell	3744

18.1	RRC_IDLE state mobility	3744

18.1.1	Inter-rat NR cell re-selection	3744

18.1.1.0	Minimum conformance requirements	3744

18.1.1.1	E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1	3744

18.2	RRC_CONNECTED state mobility for RedCap	3748

18.2.1	Handover for RedCap	3748

18.2.1.0	Minimum conformance requirements	3748

18.2.1.1	E-UTRA - NR FR1 handover  for 2Rx UE	3749

18.2.2.0	Minimum conformance requirements	3752

18.2.2.0.1	Redirection from E-UTRA to FR1 RedCap UE	3752

18.2.2.1	Redirection from E-UTRA to NR SA FR1 for redcap UE	3753

18.3	Measurement procedure for RedCap	3760

18.3.1	E-UTRA - NR Measurements for RedCap	3760

18.3.1.0	Minimum conformance requirements	3760

18.3.1.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	3760

18.3.1.1	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3763

18.3.1.2	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3767

18.3.1.3	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3771

18.3.1.4	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3775

18.3.1.5	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	3779

18.3.1.6	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is used	3781

18.3.1.7	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	3784

18.3.1.8	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is used	3786

Annex A (normative): RRM test configurations	3790

A.1	Reference measurement channels	3790

A.1.0	General	3790

A.1.1	PDSCH	3790

A.1.1.1	FDD	3790

A.1.1.2	TDD	3791

A.1.2	CORESET for RMSI scheduling	3794

A.1.2.1	FDD	3794

A.1.2.2	TDD	3795

A.1.3	CORESET for RMC scheduling	3798

A.1.3.1	FDD	3798

A.1.3.2	TDD	3799

A.1.4	CSI-RS	3801

A.1.4.1	FDD	3801

A.1.4.2	TDD	3804

A.1.4A	CSI-RS for tracking	3809

A.1.4A.1	FR1	3809

A.1.4A.1.1	FDD	3809

A.1.4A.1.2	TDD	3810

A.1.4A.2	FR2	3811

A.1.4A.2.1	TDD	3811

A.1.4B	CSI-IM configurations	3812

A.1.4B.1	FDD	3812

A.1.4B.2	TDD	3812

A.1.5	TDD UL/DL configuration	3814

A.1.6	PUSCH	3815

A.1A	Reference measurement channels under CCA	3816

A.1A.0	General	3816

A.1A.1	PDSCH	3816

A.1A.1.1	FDD	3816

A.1A.2	CORESET for RMSI scheduling	3817

A.1A.2.1	TDD	3817

A.1A.3	CORESET for RMC scheduling	3818

A.1A.3.1	TDD	3818

A.1A.4	TDD UL/DL configuration	3819

A.1A.4.1	TDD	3819

A.1A.5	RMC burst transmission model	3819

A.2	Reference OCGN configuration	3819

A.2.1 Generic OFDMA channel noise generator (OCNG)	3819

A.3	Reference SSB configuration	3822

A.3.1	SSB configuration for FR1	3822

A.3.2	SSB configuration for FR2	3822

A.3A	Reference SSB configuration under CCA	3823

A.3A.1	SSB configuration under CCA for FR1	3823

A.3A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	3823

A.3A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	3823

A.3A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	3824

A.3A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	3824

A.3B	Reference SSB configuration for RedCap	3825

A.3B.1	SSB configuration for FR1	3825

A.3B.2	SSB configuration for FR2	3826

A.4	Reference SMTC configuration	3826

A.4A	Reference SMTC configuration for RedCap	3826

A.5	Reference DRX configurations	3826

A.6	EN-DC test setup	3827

A.6.1	E-UTRA serving cell parameters	3827

A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1	3827

A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2	3829

A.6A	NR FR1-FR2 test setup	3831

A.6B	EN-DC test setup with unlicensed bands	3831

A.6B.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	3831

A.7	Reference PRACH configurations	3832

A.7.1	PRACH configurations for FR1	3832

A.7.1A	PRACH configurations for FR1 under CCA	3833

A.7.1A.1	FR1 PRACH configuration 1 under CCA	3833

A.7.1A.2	FR1 PRACH configuration 2 under CCA	3834

A.7.2	PRACH configurations for FR2	3835

A.7A	Reference MsgA configurations	3836

A.7A.1	MsgA configurations for FR1	3836

A.7A.1A	MsgA configurations for FR1 under CCA	3838

A.7A.1A.1	FR1 MsgA configuration 1 under CCA	3838

A.7A.1A.2	FR1 MsgA configuration 2 under CCA	3839

A.7A.2	MsgA configurations for FR2	3841

A.8	Reference BWP configurations	3843

A.8.1	Downlink BWP configurations	3843

A.8.2	Uplink BWP configurations	3843

A.8A	Reference BWP configurations for RedCap	3844

A.8A.1	Downlink BWP configurations	3844

A.8A.2	Uplink BWP configurations	3844

A.9	Angle of Arrival (AoA) for FR2 RRM test cases	3844

A.9.1	Setup 1: Single AoA in Rx beam peak direction	3845

A.9.2	Setup 2: Single AoA in non Rx beam peak direction	3845

A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	3845

A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	3845

A.9.3	Setup 3: 2 AoAs	3845

A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	3846

A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	3846

A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	3846

A.10	TCI State Configuration	3846

A.10.1	Introduction	3846

A.10.2	TCI states	3846

A.10A	Unified TCI State Configuration	3847

A.10A.1	Introduction	3847

A.10A.2	DLorJoint TCI states	3847

A.10A.3	UL TCI states	3847

A.11	NR sidelink communication	3848

A.11.1	Introduction	3848

A.11.2	Reference resource pool configurations for NR Sidelink Communication	3848

A.11.3	Reference measurement channels for NR Sidelink Communication	3851

A.12	Discovery Burst Transmission Window configuration under CCA	3852

A.12.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	3852

A.13	Channel bandwidth (CBW) configurations	3852

A.13.1	DL UE specific CBW	3852

A.13.2	UL UE specific CBW	3852

A.14	Satellite access configuration	3853

A.14.2	Satellite specific configuration for neighbour cell	3853

A.15	SRS Configurations	3854

A.15.1	SRS Configuration for SCS=15kHz	3854

A.15.2	SRS Configuration for SCS=30kHz	3855

A.15.3	SRS Configuration for SCS=30kHz	3856

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3856

B.1	Conditions for NR RRC_IDLE state mobility	3856

B.1.1	Introduction	3856

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	3857

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	3858

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	3860

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	3861

B.2	Conditions for NR RRC_CONNECTED state	3861

B.2.1	Introduction	3861

B.2.2	Conditions for NR intra-frequency measurements	3861

B.2.3	Conditions for NR inter-frequency measurements	3862

B.2.4	Conditions for NR L1-RSRP reporting	3863

B.2.4.1	Conditions for SSB based L1-RSRP reporting	3863

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	3864

B.2.5	Conditions for RRC connection release with redirection to NR	3865

B.2.6	Conditions for UE transmit timing	3866

B.2.6.1	Conditions for SSB based UE transmit timing	3866

B.2.7	Conditions for NR intra-frequency measurements for RedCap	3867

B.2.8	Conditions for NR inter-frequency measurements for RedCap	3869

B.2.9	Conditions for NR intra-frequency measurements under CCA	3871

B.2.10	Conditions for NR inter-frequency measurements under CCA	3872

B.2.11	Conditions for NR L1-RSRP reporting under CCA	3872

B.2.11.1	Conditions for SSB based L1-RSRP reporting	3872

B.2.12 to B.2.16	3872

B.2.17	Conditions for NR intra-frequency measurements for satellite access	3872

B.2.18	Conditions for NR inter-frequency measurements for satellite access	3873

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	3873

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	3873

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	3873

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	3873

B.3	RRM requirement exceptions	3874

B.3.1	Introduction	3874

B.3.2	Receiver sensitivity relaxation for CA	3874

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	3874

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	3874

B.3.2.2.1	Inter-band carrier aggregation	3874

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	3874

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	3874

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	3875

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	3875

B.3.2.4.1	Intra-band contiguous carrier aggregation	3875

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	3875

B.3.3	Receiver sensitivity relaxation for DC	3875

B.3.4	Receiver sensitivity relaxation for SUL	3875

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	3875

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	3875

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	3875

B.4	Conditions for NR sidelink	3876

B.4.1	Test parameters for GNSS signals	3876

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	3876

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3876

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	3877

B.5	High level test procedure for SAN RRM tests	3877

Annex C (normative): Downlink physical channels and propagation conditions	3878

C.1	Downlink physical channels	3878

C.1.1	General	3878

C.1.2	Default downlink signal levels	3878

C.1.3	Default connection setup	3879

C.2	Propagation conditions	3879

C.2.0	General	3879

C.2.1	No interference	3880

C.2.2	Static propagation conditions	3880

C.2.2.0	General	3880

C.2.2.1	UE receiver with 2Rx antenna connectors	3880

C.2.2.2	UE receiver with 4Rx antenna connectors	3880

C.2.3	Multi-path fading propagation conditions	3881

Annex D (normative): Deviations from standard test configuration	3882

D.1	Test cases with different numerologies	3882

D.2	EN-DC test cases with different EN-DC configurations	3882

D.2.0	General	3882

D.2.1	Principle of testing	3882

D.3	Carrier aggregation test cases with different CA configurations	3882

D.3.0	General	3882

D.3.1	Principle of testing	3882

D.4	Antenna connection for 4Rx capable UEs	3882

D.4.0	General	3882

D.4.1	Principle of testing	3883

D.4.1.1	Single carrier tests	3883

D.4.1.2	Carrier aggregation tests	3884

D.4.1.3	EN-DC tests	3884

D.4.2	Antenna connection	3884

D.4.2.1	Antenna connection for NR bands where 2Rx is supported	3884

D.4.2.2	Antenna connection for NR bands where only 4Rx is supported	3884

D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported	3884

D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported	3885

D.4A	Antenna configurations with unlicensed bands for 4Rx capable UEs	3885

D.4A.1	Introduction	3885

D.4A.2	Principle of Testing	3885

D.4A.2.1	Single Carrier Tests	3885

D.4A.2.2	Carrier Aggregation Tests	3886

D.4A.2.3	EN-DC Tests	3886

D.4A.2.4	Antenna connection for bands where 2RX is supported	3886

D.4A.2.5	Antenna connection for bands where 4RX is supported	3886

D.4A.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported	3886

D.4A.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported	3886

D.5	Test Cases with Different Channel Bandwidths	3887

D.5.1	Test Cases with Different E-UTRA Channel Bandwidths	3887

D.5.1.1	Introduction	3887

D.5.1.2	Principle of testing	3887

D.6	Test Cases for Synchronous and Asynchronous DC Operations	3887

D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	3887

D.6.1.1	Introduction	3887

D.6.1.2	Principle of Testing	3887

D.7	CCA model	3887

D.7.1	Introduction	3887

D.7.2	CCA model for operation on a carrier frequency with CCA in FR1	3887

D.7.2.1	DL CCA model	3887

D.7.2.2	UL CCA model	3889

D.7.3	Test Cases with at Least One Cell on a Carrier Frequency with CCA	3889

D.8	Test Cases in SA and EN-DC Operations	3889

D.8.0	General	3889

D.8.1	Principle of testing	3890

Annex E (normative): Cell configuration mapping	3891

E.0	General	3891

E.1	Test frequency selection	3891

E.1.0	General	3891

E.1.1	E-UTRA PCell for EN-DC test cases	3891

E.1.2	Test cases with one NR cell	3891

E.1.3	Test cases with more than one NR cell	3891

E.1.3.1	Intra-frequency test cases	3891

E.1.3.2	Inter-frequency test cases	3891

E.1.4	Carrier aggregation test cases	3892

E.1.4.1	Inter-band carrier aggregation	3892

E.1.4.2	Intra-band contiguous carrier aggregation	3892

E.1.4.3	Intra-band non-contiguous carrier aggregation	3892

E.1.5	E-UTRA – NR inter RAT test cases	3892

E.1.6	Intra-band EN-DC test cases	3892

E.1.6.1	Intra-band non-contiguous EN-DC	3892

E.1.6.1.1	Inter frequency neighbour cell	3892

E.1.6.2	Intra-band contiguous EN-DC	3892

E.1.6.1.1	E-UTRA PCell	3892

E.1.7	NR sidelink test cases	3893

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4	3893

E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5	3896

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6	3899

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7	3905

E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8	3908

E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9	3909

E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10	3910

E.9 to E.13	3912

E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16	3912

E.15	Cell configuration mapping for SA FR2 test cases for RedCap in Chapter 17	3917

E.16	Cell configuration mapping for E-UTRAN – SA test cases for RedCap in Chapter 18	3918

Annex F (normative): Measurement uncertainties and test tolerances	3920

F.1	Measurement uncertainties and test tolerances for FR1 and FR2	3920

F.1.1	Acceptable uncertainty of test system (normative)	3920

F.1.1.1	Measurement of test environments	3920

F.1.1.2	Measurement of RRM requirements	3920

F.1.2	Interpretation of measurement results (normative)	3971

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)	3971

F.1.3.1	Measurement of test environments	3971

F.1.3.2	Measurement of RRM requirements	3971

Annex G (normative): Statistical testing	4086

G.1	General	4086

G.2	Statistical testing of delay and UE measurement performance in RRM tests	4086

G.2.1	General	4086

G.2.2	Design of the test	4086

G.2.3	Numerical definition of the pass fail limits	4086

G.2.4	Pass fail decision rules	4087

G.2.5	Void	4088

G.2.6	Test conditions for delay tests and UE measurement performance	4088

G.3	Statistical testing of NR sidelink CBR measurement tests	4088

G.3.1	General	4088

G.3.2	Design of the test	4088

G.3.3	Numerical definition of the pass fail limits	4089

G.3.4	Pass fail decision rules	4090

G.4	Theory to derive the numbers in Table G.2.3-1 (informative)	4090

G.5	Theory to derive the numbers in Table G.3.3-1 (informative)	4090

G.5.1	Algorithm to calculate pass and fail probabilities	4090

G.5.2	Method of designing pass/fail limit to approach given risk probability	4094

Annex H (normative): Default message contents for RRM	4098

H.1	Void	4098

H.2	System information blocks message content exceptions	4098

H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection	4098

H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection	4100

H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection	4101

H.3	RRC message content exceptions	4102

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration	4102

H.3.2	RRC messages and information elements contents exceptions for handover	4124

H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover	4125

H.3.4	E-UTRA RRC messages and information elements contents exceptions	4126

H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting	4137

H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting	4140

H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used	4146

H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay	4149

H.3.9	RRC messages and information elements contents exceptions for UL timing	4150

H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access	4150

Annex I (normative): RRM OTA procedures	4152

I.0	Test applicability per permitted test method	4152

I.1	Direct far field (DFF)	4152

I.1.1	RX beam peak direction search	4152

I.1.2	Search for directions corresponding to the EIS spherical coverage percentile	4152

I.2	Direct far field (DFF) simplification	4152

I.2.1	RX beam peak direction search	4152

I.2.2	Search for directions corresponding to the EIS spherical coverage percentile	4153

I.3	Indirect far field (IFF)	4153

I.3.1	RX beam peak direction search	4153

I.3.2	Search for directions corresponding to the EIS spherical coverage percentile	4153

I.3A	Enhanced indirect far field (Enhanced IFF)	4153

I.3A.1	RX beam peak direction search	4153

I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile	4153

I.4	Procedures to search test directions for RRM FR2	4153

I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search	4153

Annex J (informative): Change history	4155