| 3GPP TS 38.533 V18.2.0 (2024-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	54

1	Scope	55

2	References	55

3	Definitions, symbols and abbreviations	56

3.1	Definitions	56

3.2	Symbols	57

3.3	Abbreviations	58

3A	Requirements for the support of RRM	60

3A.1	General	60

3A.1.0	Overview of RRM requirements	60

3A.1.1	Test coverage across 5G NR connectivity options	61

3A.2	Requirements Classification for Statistical Testing	61

3A.3	Antenna Configuration	62

3A.4	NR band groups	62

3A.4.0	General	62

3A.4.1	NR operating bands in FR1	62

3A.4.1A	NR operating bands for satellite access in FR1	64

3A.4.2	NR operating bands in FR2	64

3A.5	NR operating band configuration	65

3A.6	UE with Multiband Capability	66

4	EN-DC with all NR cells in FR1	66

4.0	General	66

4.1	Void	67

4.2	Void	67

4.3	RRC_CONNECTED state mobility	67

4.3.1	Void	67

4.3.2	RRC connection mobility control	67

4.3.2.1	Void	67

4.3.2.2	Random access	67

4.3.2.2.1	EN-DC FR1 contention based random access	67

4.3.2.2.2	EN-DC FR1 non-contention based random access	73

4.3.2.2.3	EN-DC FR1 2-step contention based random access	80

4.3.2.2.4	EN-DC FR1 2-step non-contention based random access	85

4.3.2.3	Void	91

4.4	Timing	91

4.4.1	UE transmit timing	91

4.4.1.0	Minimum conformance requirements	91

4.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	91

4.4.1.1	EN-DC FR1 UE transmit timing accuracy	92

4.4.2	UE timer accuracy	98

4.4.3	Timing advance	98

4.4.3.0	Minimum conformance requirements	98

4.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	98

4.4.3.0.2	Minimum conformance requirements for timing advance adjustment delay	99

4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	99

4.5	Signaling characteristics	104

4.5.1	Radio link monitoring	104

4.5.1.00	General	104

4.5.1.0	Minimum conformance requirements	104

4.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	104

4.5.1.0.2	Void	105

4.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	105

4.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	106

4.5.1.0.5	Requirements for UE configured with Relaxed Measurement Criteria	108

4.5.1.1	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	109

4.5.1.2	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	114

4.5.1.3	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	119

4.5.1.4	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	124

4.5.1.5	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	129

4.5.1.6	EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	134

4.5.1.7	EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	139

4.5.1.9	EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	150

4.5.2	Interruption	155

4.5.2.0	Minimum conformance requirements	155

4.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX.	155

4.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	156

4.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	157

4.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	157

4.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	159

4.5.2.0.6	Minimum conformance requirements for interruptions due to RRM and RLM/BFD measurements on deactivated NR SCG	160

4.5.2.1	EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	160

4.5.2.2	EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	164

4.5.2.3	EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	168

4.5.2.4	EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	174

4.5.2.5	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	180

4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	186

4.5.2.7	190

4.5.2.8	EN-DC FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	191

4.5.2.9	EN-DC FR1 interruptions at E-UTRA SRS carrier based switching	196

4.5.2.10	EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell	202

4.5.3	SCell activation and deactivation delay	206

4.5.3.0	Minimum conformance requirements	206

4.5.3.0.1	SCell activation and deactivation delay	206

4.5.3.0.2	Fast SCell Activation Delay Requirement for Deactivated SCell	210

4.5.3.1	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	212

4.5.3.2	EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	223

4.5.3.3	EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX	225

4.5.3.5	EN-DC FR1 direct SCell activation at SCell addition of known SCell	227

4.5.3.6	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	236

4.5.3.7	EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	245

4.5.4	UE UL carrier RRC reconfiguration delay	248

4.5.4.1	EN-DC FR1 UE UL carrier RRC reconfiguration delay	248

4.5.5	Link recovery procedures	256

4.5.5.0	Minimum conformance requirements	256

4.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	258

4.5.5.0.4	Requirements for Beam Failure Recovery in SCell	259

4.5.5.1	EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX	259

4.5.5.2	EN-DC FR1 SSB-based beam failure detection and link recovery in DRX	266

4.5.5.3	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	272

4.5.5.4	EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX	279

4.5.5.5	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	286

4.5.5.6	EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	291

4.5.5.7	EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX	298

4.5.5.8	EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	306

4.5.6	Active BWP switch delay	314

4.5.6.1	DCI-based and time-based active BWP switch	314

4.5.6.1.0	Minimum conformance requirements	314

4.5.6.1.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	317

4.5.6.1.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	324

4.5.6.2	RRC-based active BWP switch	334

4.5.6.2.0	Minimum conformance requirements	334

4.5.6.2.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	334

4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	340

4.5.6.3.0	Minimum conformance requirements	340

4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	342

4.5.6.4	351

4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	351

4.5.6.5.0	Minimum conformance requirements	351

4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	352

4.5.7	PSCell addition and release delay	360

4.5.7.0	Minimum conformance requirements	360

4.5.7.0.1	NR PSCell Addition Delay Requirement	360

4.5.7.0.2	NR PSCell Release Delay Requirement	361

4.5.7.1	EN-DC FR1 addition and release delay of known PSCell	361

4.5.8	UL switching	366

4.5.8.0	Minimum conformance requirements	366

4.5.8.1	EN-DC FR1 interruptions at switching between two uplink carriers	367

4.5.9	UE specific CBW change	376

4.5.9.0	Minimum conformance requirements	376

4.5.9.0.1	Minimum conformance requirements for UE specific CBW change delay	376

4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN-DC	377

4.5.10	PSCell activation and deactivation delay	381

4.5.10.0	Minimum conformance requirements	381

4.5.10.1	EN-DC FR1 PSCell activation and deactivation delay	382

4.5.11	PSCell addition and release delay	387

4.5.11.0	Minimum conformance requirements	387

4.5.11.0.1	Measurement time	388

4.5.11.1	EN-DC FR1 Conditional PSCell Addition Delay	388

4.6	Measurement procedures	394

4.6.1	Intra-frequency measurements	394

4.6.1.0	Minimum conformance requirements	394

4.6.1.0.1	Minimum conformance requirements for event-triggered reporting without gap	395

4.6.1.0.2	Minimum conformance requirements for event-triggered measurements with gap	399

4.6.1.1	EN-DC FR1 event-triggered reporting without gap in non-DRX	401

4.6.1.2	EN-DC FR1 event-triggered reporting without gap in DRX	406

4.6.1.3	EN-DC FR1 event-triggered reporting with gap in non-DRX	409

4.6.1.4	EN-DC FR1 event-triggered reporting with gap in DRX	415

4.6.1.5	EN-DC FR1 event-triggered reporting without gap in non-DRX with SSB time index detection	420

4.6.1.6	EN-DC FR1 event-triggered reporting with gap in non-DRX with SSB time index detection	423

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	428

4.6.1.8	EN-DC FR1 event triggered reporting cell without SSB time index detection in DRX for UE configured with highSpeedMeasCA-Scell-r17	432

4.6.2	Inter-frequency measurements	438

4.6.2.0	Minimum conformance requirements for Inter-frequency measurements	438

4.6.2.1	EN-DC FR1-FR1 event-triggered reporting in non-DRX	441

4.6.2.2	EN-DC FR1-FR1 event-triggered reporting in DRX	446

4.6.2.3	Void	451

4.6.2.4	Void	451

4.6.2.5	EN-DC FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	451

4.6.2.6	EN-DC FR1-FR1 event-triggered reporting in DRX with SSB time index detection	456

4.6.2.7	Void	461

4.6.2.8	Void	461

4.6.2.9	EN-DC FR1-FR1 event triggered reporting without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	461

4.6.3	Void	466

4.6.4	L1-RSRP measurement for beam reporting	466

4.6.4.0	Minimum conformance requirements	466

4.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	466

4.6.4.1	EN-DC FR1 SSB-based L1-RSRP measurement in non-DRX	470

4.6.4.1.5	Test requirement	473

4.6.4.2	EN-DC FR1 SSB-based L1-RSRP measurement in DRX	474

4.6.4.2.3	Minimum conformance requirements	474

4.6.4.3	EN-DC FR1 CSI-RS-based L1-RSRP measurement in non-DRX	477

4.6.4.4	EN-DC FR1 CSI-RS-based L1-RSRP measurement in DRX	481

4.6.4.5	EN-DC FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	485

4.6.5	CLI measurements	489

4.6.5.0	Minimum conformance requirements	489

4.6.5.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	490

4.6.5.1	EN-DC FR1 SRS-RSRP measurement with non-DRX	491

4.6.6	Measurements with autonomous gaps	499

4.6.6.0	Minimum conformance requirements	499

4.6.6.0.1	Minimum conformance requirements for measurements with autonomous gaps	499

4.6.6.0.1.1	Minimum conformance requirements for E-UTRA FDD - NR measurements with autonomous gaps	499

4.6.6.0.1.2	Minimum conformance requirements for E-UTRA TDD - NR measurements with autonomous gaps	500

4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	500

4.6.7	L1-SINR measurement for beam reporting	505

4.6.7.0	Minimum conformance requirements	505

4.6.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	510

4.6.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	516

4.6.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	521

4.7	Measurement performance requirements	525

4.7.1	SS-RSRP	525

4.7.1.0	Minimum conformance requirements	525

4.7.1.1	Intra-frequency measurements	530

4.7.1.2	Inter-frequency measurements	538

4.7.2	SS-RSRQ	545

4.7.2.0	Minimum conformance requirements	545

4.7.2.1	EN-DC FR1 SS-RSRQ measurement accuracy	547

4.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	552

4.7.2.2.1	EN-DC FR1-FR1 SS-RSRQ absolute measurement accuracy	552

4.7.2.2.2	EN-DC FR1-FR1 SS-RSRQ relative measurement accuracy	556

4.7.3	SS-SINR	558

4.7.3.0	Minimum conformance requirements	558

4.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	558

4.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	559

4.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	560

4.7.3.1	EN-DC FR1 SS-SINR measurement accuracy	560

4.7.3.2	Inter-Frequency SS-SINR measurement accuracy	564

4.7.3.2.1	EN-DC FR1-FR1 SS-SINR absolute measurement accuracy	564

4.7.3.2.2	EN-DC FR1-FR1 SS-SINR relative measurement accuracy	568

4.7.4	L1-RSRP	570

4.7.4.0	Minimum conformance requirements	570

4.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	570

4.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	573

4.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	574

4.7.4.1	SSB based L1-RSRP measurements	575

4.7.4.1.1	EN-DC FR1 SSB-based L1-RSRP absolute measurement accuracy	575

4.7.4.1.2	EN-DC FR1 SSB-based L1-RSRP relative measurement accuracy	580

4.7.4.2	CSI-RS based L1-RSRP measurements	582

4.7.4.2.1	EN-DC FR1 CSI-RS-based L1-RSRP absolute measurement accuracy	582

4.7.4.2.2	EN-DC FR1 CSI-RS-based L1-RSRP relative measurement accuracy	586

4.7.5	SFTD	588

4.7.5.0	Minimum conformance requirements	588

4.7.5.0.1	SFTD Accuracy Requirement	588

4.7.5.1	EN-DC FR1 SFTD measurement accuracy	589

4.7.6	CLI measurements	593

4.7.6.0	Minimum conformance requirements	593

4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy	593

4.7.6.0.1.1	SRS-RSRP report mapping	594

4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	595

4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	595

4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	604

4.7.7	L1-SINR measurement for beam reporting	607

4.7.7.0	Minimum conformance requirements	607

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	607

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	610

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	612

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	616

4.7.7.1.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	616

4.7.7.1.2	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	621

4.7.7.2	EN-DC FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	623

4.7.7.3	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement	628

4.7.7.3.1	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	628

4.7.7.3.2	EN-DC FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	634

4.7.8	CSI-RSRP	636

4.7.8.0	Minimum conformance requirements	636

4.7.8.0.1	Intra-frequency absolute CSI-RSRP measurement accuracy	636

4.7.8.0.2	Intra-frequency relative CSI-RSRP measurement accuracy requirements	639

4.7.8.0.3	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	640

4.7.8.0.4	Inter-frequency relative CSI-RSRP measurement accuracy requirements	641

4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	642

4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	648

4.7.9	CSI-RSRQ	653

4.7.9.0	Minimum conformance requirements	653

4.7.9.0.1	Intra-frequency CSI-RSRQ accuracy requirements	653

4.7.9.0.1.1	Absolute CSI-RSRQ accuracy	653

4.7.9.0.2	Inter-frequency CSI-RSRQ accuracy requirements	655

4.7.9.0.2.1	Absolute CSI-RSRQ accuracy	655

4.7.9.0.2.2	Relative CSI-RSRQ accuracy	656

4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	657

4.7.9.1.1	Test purpose	657

4.7.9.1.2	Test applicability	657

4.7.9.1.3	Minimum conformance requirements	657

4.7.9.1.4	Test description	657

4.7.9.1.5	Test Requirements	659

4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	663

4.7.9.2.1	Test purpose	663

4.7.9.2.2	  Test applicability	663

4.7.9.2.3	Minimum conformance requirements	663

4.7.9.2.4	Test description	663

4.7.9.2.5	Test Requirements	664

4.7.10	CSI-SINR	668

4.7.10.0	Minimum conformance requirements	668

4.7.10.0.1	Intra-frequency CSI-SINR accuracy requirements in FR1	668

4.7.10.0.1.1	Absolute CSI-SINR accuracy in FR1	668

4.7.10.0.2	Inter-frequency CSI-SINR accuracy requirements in FR1	669

4.7.10.0.2.1	Absolute accuracy of CSI-SINR in FR1	669

4.7.10.0.2.2	Relative accuracy of CSI-SINR in FR1	671

4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	672

4.7.10.1.1	Test purpose	672

4.7.10.1.2	  Test applicability	672

4.7.10.1.3	Minimum conformance requirements	672

4.7.10.1.4	Test description	672

4.7.10.1.5	Test Requirements	674

4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	677

4.7.10.2.1	Test purpose	677

4.7.10.2.2	Test applicability	677

4.7.10.2.3	Minimum conformance requirements	677

4.7.10.2.4	Test description	678

4.7.10.2.5	Test Requirements	679

4A	NE-DC with all NR cells in FR1	682

4A.0	General	682

4A.1	Signalling characteristics	682

4A.1.1	E-UTRAN PSCell addition	682

4A.1.1.0	Minimum conformance requirements	682

4A.1.1.0.1	E-UTRA PSCell Addition Delay Requirement	682

4A.1.1.0.2	E-UTRA PSCell Release Delay Requirement	683

4A.1.1.1	NE-DC FR1 addition and release delay of known PSCell	683

4A.1.2	Active BWP switch delay	689

4A.1.2.0	Minimum conformance requirements	689

4A.1.2.1	NE-DC FR1 DCI-based and timer-based DL active BWP switch in non-DRX in synchronous NE-DC	689

4A.2	Measurement performance requirements	693

4A.2.1	SFTD accuracy	693

4A.2.1.0	Minimum conformance requirements	693

4A.1.1.0.1	NE-DC SFTD accuracy Requirement	693

4A.2.1.1	NE-DC FR1 SFTD accuracy	695

5	EN-DC with at least one NR cell in FR2	701

5.0	General	701

5.1	Void	701

5.2	Void	701

5.3	RRC_CONNECTED state mobility	701

5.3.1	Void	701

5.3.2	RRC connection mobility control	701

5.3.2.1	Void	701

5.3.2.2	Random access	701

5.3.2.2.1	EN-DC FR2 contention based random access	701

5.3.2.2.2	EN-DC FR2 non-contention based random access	709

5.3.2.2.3	EN-DC FR2 2-step contention based random access	717

5.3.2.2.4	EN-DC FR2 2-step non-contention based random access	721

5.3.2.3	Void	725

5.4	Timing	725

5.4.1	UE transmit timing	725

5.4.1.0	Minimum Conformance Requirements	725

5.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	725

5.4.1.1	EN-DC FR2 UE transmit timing accuracy	727

5.4.2	UE timer accuracy	733

5.4.3	Timing advance	733

5.4.3.0	Minimum conformance requirements	733

5.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	733

5.4.3.1	EN-DC FR2 timing advance adjustment accuracy	733

5.5	Signaling characteristics	738

5.5.1	Radio link monitoring	738

5.5.1.0	Minimum conformance requirements	738

5.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	738

5.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	739

5.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	740

5.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	743

5.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	746

5.5.1.0.6	Requirements for UE configured with Relaxed Measurement Criteria	747

5.5.1.1	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	747

5.5.1.2	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	754

5.5.1.3	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	760

5.5.1.4	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	766

5.5.1.5	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	771

5.5.1.6	EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode	776

5.5.1.7	EN-DC FR2 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode	781

5.5.1.9	EN-DC FR2 radio link monitoring UE scheduling restrictions	794

5.5.1.10	EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion	799

5.5.2	Interruption	804

5.5.2.0	Minimum conformance requirements	804

5.5.2.0.1	Minimum conformance requirements for interruptions at transitions between active and non-active during DRX	804

5.5.2.0.2	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	805

5.5.2.0.3	Minimum conformance requirements for interruptions during measurements on deactivated E-UTRAN SCC	806

5.5.2.0.4	Minimum conformance requirements for interruptions at NR SRS carrier based switching	806

5.5.2.0.5	Minimum conformance requirements for interruptions at E-UTRA SRS carrier based switching	808

5.5.2.1	EN-DC FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	809

5.5.2.2	EN-DC FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	813

5.5.2.3	EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	817

5.5.2.4	EN-DC FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	823

5.5.2.5	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	828

5.5.2.6	EN-DC FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	832

5.5.2.7	EN-DC FR2 interruptions at E-UTRA SRS carrier based switching	836

5.5.2.8	EN-DC FR2 interruptions at NR SRS carrier based switching	842

5.5.3	SCell activation and deactivation delay	848

5.5.3.1	EN-DC FR2 SCell activation and deactivation intra-band in non-DRX	848

5.5.3.2 to 5.5.3.6		856

5.5.3.7	EN-DC FR2 direct SCell activation at SCell addition of known SCell	856

5.5.3.8	EN-DC FR2 fast SCell Activation of SCell in FR2 intra-band	860

5.5.4	UE UL carrier RRC reconfiguration delay	869

5.5.5	Link recovery procedures	869

5.5.5.0	Minimum conformance requirements	869

5.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	869

5.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	873

5.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	878

5.5.5.0.4	Requirements for Beam Failure Recovery in SCell	879

5.5.5.0.5	Requirements for SSB based beam failure detection for UE fulfilling relaxed measurement criteria	879

5.5.5.1	EN-DC FR2 SSB-based beam failure detection and link recovery in non-DRX	880

5.5.5.2	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX	887

5.5.5.3	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	894

5.5.5.4	EN-DC FR2 CSI-RS-based beam failure detection and link recovery in DRX	900

5.5.5.5	EN-DC FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX	907

5.5.5.6	EN-DC FR2 CSI-RS-based BFD and LR for SCell in non-DRX	914

5.5.5.7	EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX	921

5.5.5.8	EN-DC FR2 CSI-RS-based PSCell TRP specific Beam Failure Detection and Link Recovery in DRX mode	927

5.5.5.9	EN-DC FR2 SSB-based beam failure detection and link recovery in DRX mode for UE fulfilling relaxed measurement criterion	934

5.5.6	Active BWP switch delay	941

5.5.6.1	DCI-based and time-based active BWP switch	941

5.5.6.1.0	Minimum conformance requirements	941

5.5.6.1.1	EN-DC FR2 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC	941

5.5.6.1.2	EN-DC FR2 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC	942

5.5.6.2	RRC-based active BWP switch	942

5.5.6.2.0	Minimum conformance requirements	942

5.5.6.2.1	EN-DC FR2 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC	943

5.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	950

5.5.6.3.0	Minimum conformance requirements	950

5.5.6.3.0.1	Minimum conformance requirements for simultaneous DCI-based and timer-based active BWP switch on multiple CCs	950

5.5.6.3.0.1.1	Minimum conformance requirements for simultaneous DCI-based active BWP switch on multiple CCs	950

5.5.6.3.0.1.2	Minimum conformance requirements for simultaneous timer-based active BWP switch on multiple CCs	952

5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	952

5.5.6.4	SCell dormancy switch	956

5.5.6.4.0	Minimum conformance requirements	956

5.5.6.4.0.1	Minimum conformance requirements for interruptions due to SCell dormancy switch	956

5.5.6.4.0.2	Minimum conformance requirements for interruptions due to CQI measurements during SCell dormancy	957

5.5.6.4.0.3	Minimum conformance requirements for interruptions due to RRM measurements during SCell dormancy	957

5.5.6.4.0.4	Minimum conformance requirements for DCI and timer-based BWP switch delay on a single CC	957

5.5.6.4.0.5	Minimum conformance requirements for simultaneous DCI based BWP switch delay on multiple CCs	958

5.5.6.4.0.6	Minimum conformance requirements for interruptions due to NR SCell dormancy switch	960

5.5.6.4.0.7	Minimum conformance requirements for interruptions due to CSI and RRM measurements during SCell dormancy	960

5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 Scell inside active time	960

5.5.7	Void	964

5.5.8	Active TCI state switch delay	964

5.5.8.0	Minimum conformance requirements	964

5.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	964

5.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	965

5.5.8.1	EN-DC FR2 MAC-CE based active TCI state switch	966

5.5.8.2	EN-DC FR2 RRC based active TCI state switch	970

5.5.9	Uplink spatial relation switch delay	974

5.5.9.0	Minimum conformance requirements	974

5.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	974

5.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	974

5.5.9.1	MAC-CE based uplink spatial relation switch	975

5.5.9.1.1	EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation	975

5.5.9.2	RRC based uplink spatial relation switch	979

5.5.9.2.1	EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS	979

5.5.10	UE specific CBW change	983

5.5.10.0	Minimum conformance requirements	983

5.5.10.0.1	Minimum conformance requirements for UE specific CBW change	983

5.5.10.1	UE specific CBW change on FR2 NR PSCell	984

5.5.11	Unified TCI state switch delay	988

5.5.11.0	Minimum conformance requirements	988

5.5.11.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	988

5.5.11.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	990

5.5.11.1	EN-DC FR2 MAC-CE based active joint TCI state switch	991

5.5.11.2	EN-DC FR2 MAC-CE based active uplink TCI state switch	996

5.5.11.3	EN-DC FR2 MAC-CE based active downlink TCI state switch	1000

5.5.12	PSCell activation and deactivation delay	1004

5.5.12.0	Minimum conformance requirements	1004

5.5.12.1	EN-DC FR2 PSCell activation and deactivation delay	1005

5.5.13	Conditional PSCell addition and release delay	1009

5.5.13.1	EN-DC FR2 Addition and Release Delay of NR PSCell	1009

5.6	Measurement procedures	1015

5.6.1	Intra-frequency measurements	1015

5.6.1.0	Minimum conformance requirements	1015

5.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1015

5.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1018

5.6.1.1	EN-DC FR2 event-triggered reporting without gap in non-DRX	1019

5.6.1.2	EN-DC FR2 event-triggered reporting without gap in DRX	1025

5.6.1.3	EN-DC FR2 event-triggered reporting with gap in non-DRX	1029

5.6.1.4	EN-DC FR2 event-triggered reporting with gap in DRX	1035

5.6.2	Inter-frequency measurements	1040

5.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1040

5.6.2.1	EN-DC FR2-FR2 event-triggered reporting in non-DRX	1042

5.6.2.2	EN-DC FR2-FR2 event-triggered reporting in DRX	1046

5.6.2.3	EN-DC FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	1051

5.6.2.4	EN-DC FR2-FR2 event-triggered reporting in DRX with SSB time index detection	1055

5.6.2.5	EN-DC FR1-FR2 event-triggered reporting in non-DRX	1060

5.6.2.6	EN-DC FR1-FR2 event-triggered reporting in DRX	1065

5.6.2.7	EN-DC FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	1071

5.6.2.8	EN-DC FR1-FR2 event-triggered reporting in DRX with SSB time index detection	1076

5.6.3	L1-RSRP measurement for beam reporting	1081

5.6.3.0	Minimum conformance requirements	1081

5.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1081

5.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1083

5.6.3.1	EN-DC FR2 SSB-based L1-RSRP measurement in non-DRX	1086

5.6.3.2	EN-DC FR2 SSB-based L1-RSRP measurement in DRX	1090

5.6.3.3	EN-DC FR2 CSI-RS-based L1-RSRP measurement in non-DRX	1094

5.6.3.4	EN-DC FR2 CSI-RS-based L1-RSRP measurement in DRX	1098

5.6.4	CLI measurements	1101

5.6.4.0	Minimum conformance requirements	1101

5.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	1102

5.6.4.1	EN-DC FR2 SRS-RSRP measurement in non-DRX	1103

5.6.4.2	EN-DC FR2 CLI-RSSI measurement in non-DRX	1106

5.6.5	Measurements with autonomous gaps	1110

5.6.5.0	Minimum conformance requirements	1110

5.6.5.0.1	Minimum conformance requirements for EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1110

5.6.5.0.1.1	Introduction	1110

5.6.5.0.1.2	CGI identification of an NR cell with autonomous gaps	1110

5.6.5.0.1.3	CGI reporting delay	1111

5.6.5.1	EN-DC inter-frequency CGI identification of NR neighbour cell in FR2	1111

5.6.6	L1-SINR measurement for beam reporting	1116

5.6.6.0	Minimum conformance requirements	1116

5.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1116

5.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1119

5.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1121

5.6.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1124

5.6.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1128

5.6.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	1134

5.7	Measurement performance requirements	1139

5.7.1	SS-RSRP	1139

5.7.1.0	Minimum conformance requirements	1139

5.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	1139

5.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	1140

5.7.1.1	EN-DC FR2 SS-RSRP measurement accuracy	1142

5.7.1.2	EN-DC FR2-FR2 SS-RSRP measurement accuracy	1149

5.7.1.3	EN-DC FR1-FR2 SS-RSRP measurement accuracy	1156

5.7.2	SS-RSRQ	1162

5.7.2.0	Minimum conformance requirements	1162

5.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	1162

5.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	1162

5.7.2.1	EN-DC FR2 SS-RSRQ measurement accuracy	1164

5.7.2.2	EN-DC FR2-FR2 SS-RSRQ measurement accuracy	1168

5.7.3	SS-SINR	1173

5.7.3.0	Minimum conformance requirements	1173

5.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	1173

5.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	1173

5.7.3.1	EN-DC FR2 SS-SINR measurement accuracy	1175

5.7.3.2	EN-DC FR2-FR2 SS-SINR measurement accuracy	1179

5.7.4	L1-RSRP	1184

5.7.4.0	Minimum conformance requirements	1184

5.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	1185

5.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	1185

5.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	1186

5.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	1187

5.7.4.1	EN-DC FR2 SSB based L1-RSRP measurement accuracy	1188

5.7.4.2	EN-DC FR2 CSI-RS based L1-RSRP measurement accuracy	1194

5.7.5	SRS-RSRP	1200

5.7.5.0	Minimum conformance requirements	1200

5.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	1200

5.7.5.1	EN-DC FR2 SRS-RSRP measurement accuracy	1202

5.7.5.2	EN-DC FR2 CLI-RSSI measurement accuracy	1207

5.7.5.2.1	Test purpose	1208

5.7.5.2.2	Test applicability	1208

5.7.5.2.3	Minimum conformance requirements	1208

5.7.5.2.4	Test description	1208

5.7.5.2.5	Test requirement	1209

5.7.6	L1-SINR measurement for beam reporting	1212

5.7.6.0	Minimum conformance requirements	1212

5.7.6.0.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	1212

5.7.6.0.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	1213

5.7.6.0.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	1216

5.7.6.1	EN-DC FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	1218

5.7.6.2	EN-DC FR2 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	1223

5.7.6.3	EN-DC FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	1228

5.7.8	CSI-RSRQ	1233

5.7.8.0	Minimum conformance requirements	1233

5.7.8.0.1	Intra-frequency CSI-RSRQ accuracy requirements	1233

5.7.8.0.1.1	Absolute CSI-RSRQ accuracy	1233

5.7.8.0.2	Inter-frequency CSI-RSRQ accuracy requirements	1234

5.7.8.0.2.1	Absolute CSI-RSRQ accuracy	1234

5.7.8.0.2.2	Relative CSI-RSRQ accuracy	1236

5.7.8.1	EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1236

5.7.8.1.1	Test purpose	1236

5.7.8.1.2	Test applicability	1237

5.7.8.1.3	Minimum conformance requirements	1237

5.7.8.1.4	Test description	1237

5.7.8.1.5	Test Requirements	1238

5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1240

5.7.8.2.1	Test purpose	1240

5.7.8.2.2	Test applicability	1240

5.7.8.2.3	Minimum conformance requirements	1240

5.7.8.2.4	Test description	1240

5.7.8.2.5	Test Requirements	1242

5.7.9	CSI-SINR	1243

5.7.9.0	Minimum conformance requirements	1243

5.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements	1243

5.7.9.0.1.1	Absolute CSI-SINR accuracy	1243

5.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements	1244

5.7.9.0.2.1	Absolute CSI-SINR accuracy	1244

5.7.9.0.2.2	Relative CSI-SINR accuracy	1246

5.7.9.1	EN-DC intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1247

5.7.9.1.1	Test purpose	1247

5.7.9.1.2	Test applicability	1247

5.7.9.1.3	Minimum conformance requirements	1247

5.7.9.1.4	Test description	1247

5.7.9.1.5	Test Requirements	1248

5.7.9.2	EN-DC inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1250

5.7.9.2.1	Test purpose	1250

5.7.9.2.2	  Test applicability	1250

5.7.9.2.3	Minimum conformance requirements	1250

5.7.9.2.4	Test description	1251

5.7.9.2.5	Test Requirements	1252

6	NR standalone in FR1	1255

6.0	General	1255

6.1	RRC_IDLE state mobility	1255

6.1.1	NR cell re-selection	1255

6.1.1.0	Minimum conformance requirements	1255

6.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	1255

6.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	1255

6.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with highSpeedMeasFlag-r16	1258

6.1.1.0.4	Minimum conformance requirements for intra-frequency cell re-selection when UE configured with relaxed measurement criterion	1258

6.1.1.0.5	Minimum conformance requirements for inter-frequency cell re-selection when UE configured with relaxed measurement criterion	1260

6.1.1.1	NR SA FR1 cell re-selection	1262

6.1.1.1.5	Test requirement	1266

6.1.1.2	NR SA FR1-FR1 cell re-selection	1268

6.1.1.3	NR SA FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1272

6.1.1.4	NR SA FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1279

6.1.1.5	NR SA FR1-FR1 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	1284

6.1.1.6	NR SA FR1-FR1 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	1291

6.1.1.7	NR SA FR1 cell re-selection for UE configured with highSpeedMeasFlag-r16	1297

6.1.1.8	NR SA FR1-FR1 Cell reselection for UE configured with highSpeedMeasInterFreq-r17	1302

6.1.2	NR – E-UTRA cell re-selection	1308

6.1.2.0	Minimum conformance requirements	1308

6.1.2.0.1	Minimum conformance requirements for NR – E-UTRA cell re-selection	1308

6.1.2.0.2	Minimum conformance requirement for inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	1310

6.1.2.1	NR SA FR1 – E-UTRA cell re-selection to higher priority E-UTRA	1311

6.1.2.2	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA	1317

6.1.2.3	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1323

6.1.2.4	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1329

6.1.2.5	NR SA FR1 – E-UTRA cell re-selection to lower priority E-UTRA for UE configured with highSpeedMeasFlag-r16	1334

6.2	RRC_INACTIVE state mobility	1340

6.2.0	General	1340

6.2.0.1	Minimum conformance requirements for TA validation using CG-SDT	1340

6.2.1	NR SA FR1 Configured Grant based Small Data Transmissions (CG-SDT)	1341

6.2.1.1	Test Purpose	1341

6.2.1.2	Test Applicability	1341

6.2.1.3	Minimum Conformance Requirement	1341

6.2.1.4	Test Description	1341

6.2.1.4.1	Initial conditions	1342

6.2.1.4.2	Test Procedure	1344

6.2.1.5	Test Requirement	1346

6.3	RRC_CONNECTED state mobility	1347

6.3.1	Handover	1347

6.3.1.0	Minimum conformance requirements	1347

6.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	1347

6.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	1347

6.3.1.0.3	Minimum conformance requirements for NR – UTRAN handover	1348

6.3.1.0.4	Minimum conformance requirements for NR FR1- NR FR1 DAPS handover	1349

6.3.1.1	NR SA FR1 handover with known target cell	1351

6.3.1.2	NR SA FR1 handover with unknown target cell	1355

6.3.1.3	NR SA FR1-FR1 handover with unknown target cell	1358

6.3.1.4	NR SA FR1 – E-UTRA handover with known target cell	1361

6.3.1.5	NR SA FR1 – E-UTRA handover with unknown target cell	1369

6.3.1.6	NR SA FR1 – UTRAN FDD handover with known target cell	1374

6.3.1.7	NR SA FR1 synchronous DAPS handover	1380

6.3.1.8	NR SA FR1 asynchronous DAPS handover	1388

6.3.1.9	NR SA FR1 Intra-band inter-frequency synchronous DAPS handover	1393

6.3.1.10	NR SA FR1 Intra-band inter-frequency asynchronous DAPS handover	1402

6.3.1.11	NR SA FR1 Inter-band inter-frequency synchronous DAPS handover	1408

6.3.1.12	NR SA FR1 Inter-band inter-frequency asynchronous DAPS handover	1417

6.3.2	RRC connection mobility control	1425

6.3.2.1	RRC re-establishment	1425

6.3.2.1.0	Minimum conformance requirements	1425

6.3.2.1.1	NR SA FR1 RRC re-establishment	1427

6.3.2.1.2	NR SA FR1 - FR1 RRC re-establishment	1431

6.3.2.1.3	NR SA FR1 RRC re-establishment without serving cell timing	1435

6.3.2.2	Random access	1439

6.3.2.2.0	Minimum conformance requirements	1439

6.3.2.2.1	NR SA FR1 contention based random access	1441

6.3.2.2.2	NR SA FR1 non-contention based random access	1447

6.3.2.2.3	NR SA FR1 2-step contention based random access	1454

6.3.2.2.4	NR SA FR1 2-step non-contention based random access	1458

6.3.2.3	RRC connection release with redirection	1462

6.3.2.3.0	Minimum conformance requirements	1462

6.3.2.3.1	NR SA FR1 RRC connection release with redirection	1464

6.3.2.3.2	NR SA FR1 – E-UTRA RRC connection release with redirection	1467

6.3.3	Conditional handover	1472

6.3.3.0	Minimum conformance requirements	1472

6.3.3.0.1	Minimum conformance requirements for NR FR1 intra-frequency conditional handover	1472

6.3.3.0.2	Minimum conformance requirements for NR FR1 inter-frequency conditional handover	1475

6.3.3.1	NR SA FR1 conditional handover	1478

6.3.3.2	NR SA FR1-FR1 conditional handover	1484

6.4	Timing	1493

6.4.1	UE transmit timing	1493

6.4.1.0	Minimum conformance requirements	1493

6.4.1.1	NR SA FR1 UE transmit timing accuracy	1494

6.4.2	UE timer accuracy	1500

6.4.3	Timing advance	1500

6.4.3.0	Minimum conformance requirement	1500

6.4.3.0.1	Minimum conformance requirement for timing advance adjustment	1500

6.4.3.1	NR SA FR1 timing advance adjustment accuracy	1500

6.5	Signaling characteristics	1505

6.5.1	Radio link monitoring	1505

6.5.1.00	General	1505

6.5.1.0	Minimum conformance requirements	1507

6.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	1507

6.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	1508

6.5.1.0.3	Minimum conformance requirements for out-of-sync and in-sync CSI-RS based RLM	1510

6.5.1.0.4	Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria	1511

6.5.1.0.5	Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria	1511

6.5.1.1	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1512

6.5.1.2	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	1518

6.5.1.3	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	1524

6.5.1.4	NR SA FR1 radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	1529

6.5.1.5	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1536

6.5.1.6	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	1543

6.5.1.7	NR SA FR1 radio link monitoring out-of-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1549

6.5.1.8	NR SA FR1 radio link monitoring in-sync test for PCell configured with CSI-RS-based RLM RS in DRX mode	1556

6.5.1.9	SA FR1 radio link monitoring out-of-sync Test for PCell configured with CSI-RS-based RLM for UE fulfilling relaxed measurement criterion	1562

6.5.1.9.1	Test purpose	1562

6.5.1.9.2	Test applicability	1563

6.5.1.9.3	Minimum conformance requirements	1563

6.5.1.9.4	Test description	1563

6.5.1.9.5	Test requirement	1568

6.5.2	Interruption	1570

6.5.2.0	Minimum conformance requirements	1570

6.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	1570

6.5.2.0.2	 Interruptions at NR SRS carrier based switching	1571

6.5.2.1	NR SA FR1 interruptions during measurements on deactivated NR SCC	1573

6.5.2.2	SA FR1 interruptions at NR SRS carrier based switching	1578

6.5.3	SCell activation and deactivation delay	1583

6.5.3.0	Minimum conformance requirements	1583

6.5.3.0.1	Minimum conformance requirements for SCell activation and deactivation delay	1583

6.5.3.1	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle	1584

6.5.3.2	NR SA FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle	1597

6.5.3.3	NR SA FR1 SCell activation and deactivation of unknown SCell in non-DRX	1598

6.5.3.4	NR SA FR1 direct SCell activation at SCell addition of known SCell	1600

6.5.3.5	NR SA FR1 direct SCell activation at handover with known SCell	1609

6.5.3.6 to 6.5.3.9	1618

6.5.3.10	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle	1618

6.5.3.11	NR SA FR1 fast SCell Activation of known SCell in non-DRX for 640ms SCell measurement cycle	1626

6.5.4	UE UL carrier RRC reconfiguration delay	1629

6.5.4.0	Minimum conformance requirements	1629

6.5.4.0.1	Minimum conformance requirements for UL carrier RRC reconfiguration delay	1629

6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay	1629

6.5.5	Link recovery procedures	1637

6.5.5.0	Minimum conformance requirements	1637

6.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	1637

6.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	1637

6.5.5.0.4	Requirements for Beam Failure Recovery in SCell	1637

6.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	1637

6.5.5.1	NR SA FR1 SSB-based beam failure detection and link recovery in non-DRX	1639

6.5.5.2	NR SA FR1 SSB-based beam failure detection and link recovery in DRX	1647

6.5.5.3	NR SA FR1 CSI-RS-based beam failure detection and link recovery in non-DRX	1655

6.5.5.4	NR SA FR1 CSI-RS-based beam failure detection and link recovery in DRX	1662

6.5.5.5	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX	1669

6.5.5.6	NR SA FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX	1677

6.5.5.7	NR SA FR1 PCell TRP Specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	1684

6.5.6	Active BWP switch delay	1692

6.5.6.1	DCI-based and time-based active BWP switch	1692

6.5.6.1.0	Minimum conformance requirements	1692

6.5.6.1.1	NR SA FR1-FR1 DCI-based DL active BWP switch in non-DRX	1693

6.5.6.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX	1704

6.5.6.2	RRC-based active BWP switch	1713

6.5.6.2.0	Minimum conformance requirements	1713

6.5.6.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX	1713

6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1721

6.5.6.3.0	Minimum conformance requirements	1721

6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1721

6.5.6.4	1731

6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1731

6.5.6.5.0	Minimum conformance requirements	1731

6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch in non-DRX on multiple CCs	1732

6.5.7	DL interruptions at switching between two uplink carriers	1739

6.5.7.0	Minimum conformance requirements	1739

6.5.7.1	NR SA FR1 DL Interruptions at switching between two uplink carriers in FDD-TDD CA	1739

6.5.7.2	NR SA FR1 DL Interruptions at switching between two uplink carriers in TDD-TDD CA	1751

6.5.7A	NR SA FR1 DL interruptions at switching between two uplink carriers with two transmit antenna connectors	1759

6.5.7A.0	Minimum conformance requirements	1759

6.5.7A.1	NR SA FR1 DL interruptions at switching between two uplink carriers in FDD-TDD CA	1760

6.5.7A.2	NR SA FR1 DL interruptions at switching between two uplink carriers in TDD-TDD CA	1768

6.5.7B	DL interruptions at switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors	1776

6.5.7B.0	Minimum conformance requirements	1776

6.5.7B.1	NR SA FR1 DL Interruptions at switching between two uplink bands in FDD-TDD CA	1777

6.5.7B.2	NR SA FR1 DL Interruptions at switching between two uplink bands in TDD-TDD CA	1787

6.5.7C	DL interruptions at switching between two uplink bands with two transmit antenna connectors	1797

6.5.7C.0	Minimum conformance requirements	1797

6.5.7C.1	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA	1798

6.5.7C.1.1	Test purpose	1798

6.5.7C.1.2	Test applicability	1798

6.5.7C.1.3	Minimum conformance requirements	1798

6.5.7C.1.4	Test description	1798

6.5.7C.1.4.1	Initial conditions	1798

6.5.7C.1.5	Test requirements	1806

6.5.7C.2	NR SA FR1 DL interruptions at switching between two uplink bands with two transmit antenna connectors in TDD-TDD CA	1809

6.5.7C.2.1	Test purpose	1809

6.5.7C.2.2	Test applicability	1809

6.5.7C.2.3	Minimum conformance requirements	1809

6.5.7C.2.4	Test description	1810

6.5.7C.2.4.1	Initial conditions	1810

6.5.8	UE specific CBW change	1821

6.5.8.0	Minimum conformance requirements	1821

6.5.8.0.1	Minimum conformance requirements for UE specific CBW change	1821

6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1821

6.6	Measurement procedures	1828

6.6.1	Intra-frequency measurements	1828

6.6.1.0	Minimum conformance requirements	1828

6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	1828

6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	1828

6.6.1.0.3	Void	1829

6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading	1829

6.6.1.0.5	 Void	1831

6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX	1831

6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX	1835

6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX	1840

6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX	1846

6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading	1851

6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading	1855

6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16	1859

6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17	1864

6.6.2	Inter-frequency measurements	1869

6.6.2.0	Minimum conformance requirements for Inter-frequency measurements	1869

6.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps	1869

6.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps	1869

6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX	1872

6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX	1878

6.6.2.3	Void	1884

6.6.2.4	Void	1884

6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection	1884

6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection	1890

6.6.2.7	Void	1896

6.6.2.8	Void	1896

6.6.2.9	NR SA FR1-FR1 event triggered reporting tests with additional mandatory gap pattern	1896

6.6.2.10	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is used	1901

6.6.2.11	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used	1907

6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17	1913

6.6.3	Inter-RAT Measurements	1919

6.6.3.0	Minimum conformance requirements	1919

6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	1919

6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	1921

6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX	1924

6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX	1931

6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16	1937

6.6.4	L1-RSRP measurement for beam reporting	1945

6.6.4.0	Minimum conformance requirements	1945

6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	1945

6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	1945

6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX	1945

6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX	1949

6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX	1953

6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX	1957

6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16	1960

6.6.5	UTRAN inter-RAT measurement	1966

6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX	1966

6.6.6	CLI Measurements	1973

6.6.6.0	Minimum conformance requirements	1973

6.6.6.0.1	Minimum conformance requirements for SRS-RSRP measurement	1973

6.6.6.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX	1973

6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX	1973

6.6.7	NR measurements with autonomous gaps	1980

6.6.7.0	Minimum conformance requirements	1980

6.6.7.0.1	Minimum conformance requirements for SA intra-frequency CGI identification of NR neighbour cell in FR1	1980

6.6.7.0.1.1	Introduction	1980

6.6.7.0.1.2	CGI identification of an NR cell with autonomous gaps	1980

6.6.7.0.1.3	CGI reporting delay	1981

6.6.7.0.2	Minimum conformance requirements for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1981

6.6.7.0.2.1	CGI identification of an E-UTRA cell with autonomous gaps	1981

6.6.7.0.2.2	CGI reporting delay	1982

6.6.7.1	SA intra-frequency CGI identification of NR neighbour cell in FR1	1982

6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1986

6.6.8	L1-SINR measurement for beam reporting	1990

6.6.8.0	Minimum conformance requirements	1990

6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1990

6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	1990

6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1990

6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX	1990

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX	1996

6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX	2002

6.6.9	Idle Mode CA/DC Measurements	2008

6.6.9.0	Minimum conformance requirements	2008

6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1	2010

6.6.10 to 6.6.14	2020

6.6.15	Idle Mode inter-RAT CA/DC Measurements	2020

6.6.15.0	Minimum conformance requirements	2020

6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting	2021

6.6.16	2034

6.6.17	NR SA FR1 event triggered reporting tests with Pre-MG	2034

6.6.17.1	NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG	2036

6.6.17.2	NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2042

6.6.18	SA event triggered reporting tests with concurrent gaps	2050

6.6.18.0	Minimum conformance requirements	2050

6.6.18.0.1	Minimum conformance requirements for Intra-frequency measurement	2050

6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement	2051

6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement	2052

6.6.18.0.4	Minimum conformance requirements for PRS measurement	2053

6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers	2053

6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers	2061

6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1	2068

6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used	2080

6.6.19	SA event triggered reporting tests with NCSG	2089

6.6.19.0	Minimum conformance requirements	2089

6.6.19.0.1	Minimum conformance requirements for intra-frequency measurement	2089

6.6.19.0.2	Minimum conformance requirements for inter-frequency measurement	2091

6.6.19.0.3	Minimum conformance requirements for inter-RAT measurement	2092

6.6.19.0.4	Minimum conformance requirements for interruptions	2093

6.6.19.1	NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1	2094

6.6.19.2	NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement	2100

6.6.19.3	NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG	2106

6.6.19.4	NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX	2114

6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation	2120

6.6.20.0	Minimum conformance requirements	2120

6.6.20.1	NR SA FR1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC	2123

6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS	2127

6.6.21.0	Minimum conformance requirements	2127

6.6.21.1	NR SA FR1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC	2130

6.7	Measurement performance requirements	2134

6.7.1	SS-RSRP	2134

6.7.1.0	Minimum conformance requirements	2134

6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	2134

6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	2134

6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	2134

6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	2135

6.7.1.1	Intra-frequency measurements	2135

6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy	2135

6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy	2142

6.7.1.2	Inter-frequency measurements	2144

6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy	2144

6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy	2151

6.7.2	SS-RSRQ	2153

6.7.2.0	Minimum conformance requirements	2153

6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2153

6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	2153

6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	2153

6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy	2153

6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy	2157

6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy	2157

6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy	2162

6.7.3	SS-SINR	2164

6.7.3.0	Minimum conformance requirements	2164

6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2164

6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements	2164

6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements	2164

6.7.3.1	NR SA FR1 SS-SINR measurement accuracy	2164

6.7.3.2	Inter-Frequency SS-SINR measurement accuracy	2168

6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy	2168

6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy	2172

6.7.4	L1-RSRP	2174

6.7.4.0	Minimum conformance requirements	2174

6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	2174

6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	2174

6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	2174

6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	2174

6.7.4.1	SSB based L1-RSRP measurements	2174

6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy	2174

6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy	2179

6.7.4.2	CSI-RS based L1-RSRP measurements	2181

6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy	2181

6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy	2185

6.7.5	E-UTRAN RSRP	2187

6.7.5.0	Minimum conformance requirements	2187

6.7.5.0.1	E-UTRAN RSRP absolute accuracy	2187

6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy	2188

6.7.6	E-UTRAN RSRQ	2195

6.7.6.0	Minimum conformance requirements	2195

6.7.6.0.1	E-UTRAN RSRQ absolute accuracy	2195

6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy	2196

6.7.7	E-UTRAN RS-SINR	2203

6.7.7.0	Minimum conformance requirements	2203

6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy	2203

6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy	2204

6.7.8	CLI Measurements	2211

6.7.8.0	Minimum conformance requirements	2211

6.7.8.0.1	Minimum conformance requirements for SRS-RSRP accuracy	2211

6.7.8.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell	2211

6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy	2211

6.7.8.1.1	Test purpose	2211

6.7.8.1.2	Test applicability	2211

6.7.8.1.3	Minimum conformance requirements	2212

6.7.8.1.4	Test description	2212

6.7.8.1.5	Test requirement	2214

6.7.9	L1-SINR measurement for beam reporting	2221

6.7.9.0	Minimum conformance requirements	2221

6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2221

6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR	2224

6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR	2226

6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement	2230

6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy	2230

6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy	2233

6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2235

6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2238

6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy	2239

6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy	2242

6.7.11	CSI-RSRQ	2244

6.7.11.0	Minimum conformance requirements	2244

6.7.11.0.1	Intra-frequency CSI-RSRQ accuracy requirements	2244

6.7.11.0.1.1	Absolute CSI-RSRQ accuracy	2244

6.7.11.0.2	Inter-frequency CSI-RSRQ accuracy requirements	2245

6.7.11.0.2.1	Absolute CSI-RSRQ accuracy	2246

6.7.11.0.2.2	Relative CSI-RSRQ accuracy	2247

6.7.11.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2248

6.7.11.1.1	Test purpose	2248

6.7.11.1.2	  Test applicability	2248

6.7.11.1.3	Minimum conformance requirements	2248

6.7.11.1.4	Test description	2248

6.7.11.1.5	Test Requirements	2250

6.7.11.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2255

6.7.11.2.1	Test purpose	2255

6.7.11.2.2	Test applicability	2255

6.7.11.2.3	Minimum conformance requirements	2255

6.7.11.2.4	Test description	2255

6.7.11.2.5	Test Requirements	2256

6.7.12	CSI-SINR	2261

6.7.12.0	Minimum conformance requirements	2261

6.7.12.0.1	Intra-frequency CSI-SINR accuracy requirements	2261

6.7.12.0.1.1	Absolute CSI-SINR accuracy	2261

6.7.12.0.2	Inter-frequency CSI-SINR accuracy requirements	2263

[TS 38.133, Clause 10.1.14.2 and Clause 10.1.16]	2263

6.7.12.0.2.1	Absolute CSI-SINR accuracy	2263

6.7.12.0.2.2	Relative CSI-SINR accuracy	2264

6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2265

6.7.12.1.1	Test purpose	2265

6.7.12.1.2	  Test applicability	2265

6.7.12.1.3	Minimum conformance requirements	2265

6.7.12.1.4	Test description	2265

6.7.12.1.5	Test Requirements	2266

6.7.12.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	2270

6.7.12.2.1	Test purpose	2270

6.7.12.2.2	  Test applicability	2271

6.7.12.2.3	Minimum conformance requirements	2271

6.7.12.2.4	Test description	2271

6.7.12.2.5	Test Requirements	2272

7	NR standalone with at least one NR cell in FR2	2279

7.0	General	2279

7.1	RRC_IDLE state mobility	2279

7.1.1	NR cell re-selection	2279

7.1.1.0	Minimum conformance requirements	2279

7.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection	2279

7.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection	2282

7.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for UE configured with relaxed measurement criterion	2283

7.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for UE configured with relaxed measurement criterion	2284

7.1.1.1	NR SA FR2 cell re-selection	2286

7.1.1.2	NR SA FR2-FR2 cell re-selection	2291

7.1.1.3	NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2298

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2303

7.1.1.5	NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion	2308

7.1.1.6	NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion	2315

7.2	RRC_INACTIVE state mobility	2327

7.2.1	Small Data Transmission	2327

7.2.1.0	Minimum conformance requirements	2327

7.2.1.0.1	Minimum conformance requirements for CG-SDT TA validation	2327

7.2.1.1	TA Validation for CG-SDT in FR2	2328

7.3	RRC_CONNECTED state mobility	2337

7.3.1	Handover	2337

7.3.1.0	Minimum conformance requirements	2337

7.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	2337

7.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR2 handover	2338

7.3.1.0.3	Minimum conformance requirements for NR FR1 – NR FR2 DAPS handover	2339

7.3.1.1	NR SA FR1-FR2 Inter-frequency handover; unknown target cell	2340

7.3.1.2	NR SA FR2 Intra-frequency handover; unknown target cell	2345

7.3.1.3	NR SA FR2-FR2 Inter-frequency handover; unknown target cell	2349

7.3.1.4	NR SA FR1-FR2 synchronous DAPS handover	2353

7.3.1.5	NR SA FR1-FR2 asynchronous DAPS handover	2363

7.3.2	RRC connection mobility control	2370

7.3.2.1	RRC re-establishment	2370

7.3.2.1.0	Minimum conformance requirements	2370

7.3.2.1.1	NR SA FR2 RRC re-establishment	2371

7.3.2.1.2	NR SA FR2 - FR2 RRC re-establishment	2375

7.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	2378

7.3.2.2	Random access	2383

7.3.2.2.0	Minimum conformance requirements	2383

7.3.2.2.1	NR SA FR2 contention based random access	2384

7.3.2.2.2	NR SA FR2 non-contention based random access	2392

7.3.2.2.3	NR SA FR2 2-step contention based random access	2399

7.3.2.2.4	NR SA FR2 2-step non-contention based random access	2406

7.3.2.3	RRC connection release with redirection	2413

7.3.2.3.0	Minimum conformance requirements	2413

7.3.2.3.1	NR SA FR2-FR2 RRC connection release with redirection	2414

7.3.3	Conditional handover	2418

7.3.3.0	Minimum conformance requirements	2418

7.3.3.0.1	Minimum conformance requirements for NR FR2 intra-frequency conditional handover	2418

7.3.3.0.2	Minimum conformance requirements for NR FR2 inter-frequency conditional handover	2422

7.3.3.1	NR SA FR2 conditional handover	2426

7.3.3.2	NR SA FR2-FR2 conditional handover	2433

7.4	Timing	2440

7.4.1	UE transmit timing	2440

7.4.1.0	Minimum Conformance Requirements	2440

7.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	2440

7.4.1.1	NR SA FR2 UE transmit timing accuracy	2441

7.4.2	UE timer accuracy	2447

7.4.3	Timing advance	2447

7.4.3.0	Minimum conformance requirements	2447

7.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	2447

7.4.3.1	NR SA FR2 timing advance adjustment accuracy	2448

7.5	Signalling characteristics	2454

7.5.1	Radio link monitoring	2454

7.5.1.0	Minimum conformance requirements	2454

7.5.1.0.1	Minimum conformance requirements for out-of-sync SSB-based RLM	2454

7.5.1.0.2	Minimum conformance requirements for in-sync SSB-based RLM	2454

7.5.1.0.3	Minimum conformance requirements for out-of-sync CSI-RS based RLM	2454

7.5.1.0.4	Minimum conformance requirements for in-sync CSI-RS based RLM	2454

7.5.1.0.5	Minimum conformance requirements for UE scheduling restrictions during radio link monitoring	2454

7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2454

7.5.1.1.1	Test purpose	2455

7.5.1.1.2	Test applicability	2455

7.5.1.1.3	Minimum conformance requirement	2455

7.5.1.1.4	Test description	2455

7.5.1.1.5	Test Requirement	2459

7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	2460

- Usage of 2 antenna ports in CSI-RS 3.1 TDD implies SS transmitting signal on both polarizations. Potential signal imbalance impact to the test verdict is under investigation7.5.1.2.1	Test purpose	2460

7.5.1.2.2	Test applicability	2460

7.5.1.2.3	Minimum conformance requirement	2460

7.5.1.2.4	Test description	2460

7.5.1.2.5	Test Requirement	2466

7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2467

7.5.1.3.1	Test purpose	2467

7.5.1.3.2	Test applicability	2467

7.5.1.3.3	Minimum conformance requirement	2467

7.5.1.3.4	Test description	2467

7.5.1.3.5	Test Requirement	2471

7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	2472

7.5.1.4.1	Test purpose	2472

7.5.1.4.2	Test applicability	2472

7.5.1.4.3	Minimum conformance requirement	2472

7.5.1.4.4	Test description	2472

7.5.1.4.5	Test Requirement	2478

7.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	2479

7.5.1.5.1	Test purpose	2479

7.5.1.5.2	Test applicability	2479

7.5.1.5.3	Minimum conformance requirement	2479

7.5.1.5.4	Test description	2479

7.5.1.5.5	Test Requirement	2483

7.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	2484

7.5.1.6.1	Test purpose	2485

7.5.1.6.2	Test applicability	2485

7.5.1.6.3	Minimum conformance requirement	2485

7.5.1.6.4	Test description	2485

7.5.1.6.5	Test Requirement	2490

7.5.1.7	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM RS in DRX mode	2491

7.5.1.7.1	Test purpose	2491

7.5.1.7.2	Test applicability	2491

7.5.1.7.3	Minimum conformance requirement	2491

7.5.1.7.4	Test description	2491

7.5.1.7.5	Test Requirement	2495

7.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	2496

7.5.1.8.1	Test purpose	2496

7.5.1.8.2	Test applicability	2496

7.5.1.8.3	Minimum conformance requirement	2496

7.5.1.8.4	Test description	2496

7.5.1.8.5	Test Requirement	2501

7.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	2502

7.5.1.9.1	Test purpose	2503

7.5.1.9.2	Test applicability	2503

7.5.1.9.3	Minimum conformance requirement	2503

7.5.1.9.4	Test description	2503

7.5.1.9.5	Test Requirement	2505

7.5.2	Interruption	2507

7.5.2.0	Minimum conformance requirements	2507

7.5.2.0.1	Minimum conformance requirements for interruptions during measurements on deactivated NR SCC	2507

7.5.2.0.2	Minimum conformance requirements for SA interruptions at NR SRS carrier-based switching	2508

7.5.2.1	NR SA FR2 interruptions during measurements on deactivated NR SCC	2510

7.5.2.1.1	Test purpose	2510

7.5.2.1.2	Test applicability	2510

7.5.2.1.3	Minimum conformance requirements	2510

7.5.2.1.4	Test description	2510

7.5.2.1.5	Test requirement	2512

7.5.2.2	SA interruptions at NR SRS carrier-based switching	2515

7.5.2.2.1	Test purpose	2515

7.5.2.2.2	Test applicability	2515

7.5.2.2.3	Minimum conformance requirements	2515

7.5.2.2.4	Test description	2515

7.5.2.2.5	Test requirement	2516

7.5.3	SCell activation and deactivation delay	2518

7.5.3.0	Minimum conformance requirements	2518

7.5.3.0.1	Minimum conformance requirements for SCell activation delay for deactivated SCell	2518

7.5.3.0.2	Minimum conformance requirements for SCell deactivation delay for activated SCell	2521

7.5.3.1	NR SA FR2-FR2 intra-band SCell activation and deactivation delay	2521

7.5.3.1.1	Test purpose	2522

7.5.3.1.2	Test applicability	2522

7.5.3.1.3	Minimum conformance requirements	2522

7.5.3.1.4	Test description	2522

7.5.3.1.5	Test requirement	2523

7.5.3.2	NR SA FR1-FR2 inter-band SCell activation and deactivation delay	2523

7.5.3.2.1	Test purpose	2524

7.5.3.2.2	Test applicability	2524

7.5.3.2.3	Minimum conformance requirements	2524

7.5.3.2.4	Test description	2524

7.5.3.2.5	Test requirement	2525

7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	2526

7.5.3.3.1	Test purpose	2526

7.5.3.3.2	Test applicability	2526

7.5.3.3.3	Minimum conformance requirements	2526

7.5.3.3.4	Test description	2526

7.5.3.3.5	Test requirement	2531

7.5.3.4	NR SA FR2 direct SCell activation at SCell addition of known SCell	2534

7.5.3.4.1	Test Purpose	2534

7.5.3.4.2	Test applicability	2534

7.5.3.4.3	Minimum conformance requirements	2534

7.5.3.4.4	Test description	2534

7.5.3.4.5	Test requirement	2539

7.5.3.5	NR SA FR2 direct SCell activation at handover with known SCell	2541

7.5.3.5.1	Test Purpose	2541

7.5.3.5.2	Test applicability	2541

7.5.3.5.3	Minimum conformance requirements	2541

7.5.3.5.4	Test description	2541

7.5.3.5.5	Test requirement	2546

7.5.3.6 to 7.5.3.12	2549

7.5.3.13	NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX	2549

7.5.3.13.1	Test Purpose	2549

7.5.3.13.2	Test applicability	2549

7.5.3.13.3	Minimum conformance requirements	2549

7.5.3.13.4	Test description	2549

7.5.3.13.5	Test requirement	2554

7.5.3.14	NR SA FR2 SCell Activation for known SCell in FR2 inter-band	2556

-	This test case is incomplete for UE power class other than PC3.7.5.3.14.1 Test purpose	2557

7.5.3.14.2	Test applicability	2557

7.5.3.14.3	Minimum conformance requirements	2557

7.5.3.14.4	Test description	2557

7.5.3.14.5	Test requirement	2562

7.5.4	Void	2563

7.5.5	Link recovery procedures	2563

7.5.5.0	Minimum conformance requirements	2563

7.5.5.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	2563

7.5.5.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	2563

7.5.5.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	2564

7.5.5.0.4	Requirements for Beam Failure Recovery in SCell	2564

7.5.5.1	NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX	2564

7.5.5.1.1	Test purpose	2564

7.5.5.1.2	Test applicability	2564

7.5.5.1.3	Minimum conformance requirements	2564

7.5.5.1.4	Test description	2564

7.5.5.1.5	Test requirement	2568

7.5.5.2	NR SA FR2 SSB-based beam failure detection and link recovery in DRX	2569

7.5.5.2.1	Test purpose	2570

7.5.5.2.2	Test applicability	2570

7.5.5.2.3	Minimum conformance requirements	2570

7.5.5.2.4	Test description	2570

7.5.5.2.5	Test requirement	2574

7.5.5.3	NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX	2576

7.5.5.3.1	Test purpose	2576

7.5.5.3.2	Test applicability	2576

7.5.5.3.3	Minimum conformance requirements	2576

7.5.5.3.4	Test description	2576

7.5.5.3.5	Test requirement	2580

7.5.5.4	NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX	2581

7.5.5.4.1	Test purpose	2582

7.5.5.4.2	Test applicability	2582

7.5.5.4.3	Minimum conformance requirements	2582

7.5.5.4.4	Test description	2582

7.5.5.4.5	Test requirement	2586

7.5.5.5	NR SA FR2 scheduling availability restriction during SSB-based beam failure detection and link recovery in non-DRX	2588

7.5.5.5.1	Test purpose	2588

7.5.5.5.2	Test applicability	2588

7.5.5.5.3	Minimum conformance requirements	2588

7.5.5.5.4	Test description	2588

7.5.5.5.5	Test requirement	2592

7.5.5.6	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX	2593

7.5.5.6.1	Test purpose	2593

7.5.5.6.2	Test applicability	2594

7.5.5.6.3	Minimum conformance requirements	2594

7.5.5.6.4	Test description	2594

7.5.5.6.5	Test requirement	2598

7.5.5.7	NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX	2600

7.5.5.7.1	Test purpose	2600

7.5.5.7.2	Test applicability	2600

7.5.5.7.3	Minimum conformance requirements	2600

7.5.5.7.4	Test description	2600

7.5.5.7.5	Test requirement	2604

7.5.5.8	FFS	2606

7.5.5.9	NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX	2606

7.5.5.9.1	Test purpose	2606

7.5.5.9.2	Test applicability	2606

7.5.5.9.3	Minimum conformance requirements	2606

7.5.5.9.4	Test description	2606

7.5.5.9.5	Test requirements	2611

7.5.5.10	NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX	2612

7.5.5.10.1	Test purpose	2612

7.5.5.10.2	Test applicability	2612

7.5.5.10.3	Minimum conformance requirements	2612

7.5.5.10.4	Test description	2612

7.5.5.10.5	Test requirement	2617

7.5.6	Active BWP switch delay	2619

7.5.6.1	DCI-based and time-based active BWP switch	2619

7.5.6.1.0	Minimum conformance requirements	2619

7.5.6.1.1	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2619

7.5.6.1.2	NR SA FR1-FR2 DCI-based DL active BWP switch in non-DRX	2620

7.5.6.1.3	NR SA FR2 DCI-based DL active BWP switch in non-DRX	2621

7.5.6.2	RRC-based active BWP switch	2622

7.5.6.2.0	Minimum conformance requirements	2622

7.5.6.2.1	NR SA FR2 RRC-based DL active BWP switch in non-DRX	2623

7.5.6.3	Simultaneous DCI-based and timer-based active BWP switch on multiple CCs	2625

7.5.6.3.0	Minimum conformance requirements	2625

7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	2628

7.5.7	PSCell addition and release delay	2632

7.5.7.0	Minimum conformance requirements	2632

7.5.7.0.1	Minimum conformance requirements for PSCell addition delay	2632

7.5.7.0.2	Minimum conformance requirements for PSCell release delay	2633

7.5.7.1	NR SA FR2 addition and release delay of known PSCell	2633

7.5.7.1.1	Test purpose	2634

7.5.7.1.2	Test applicability	2634

7.5.7.1.3	Minimum conformance requirements	2634

7.5.7.1.4	Test description	2634

7.5.7.1.5	Test requirement	2635

7.5.7.2	NR SA FR2 addition and release delay of unknown PSCell	2635

7.5.7.2.1	Test purpose	2636

7.5.7.2.2	Test applicability	2636

7.5.7.2.3	Minimum conformance requirements	2636

7.5.7.2.4	Test description	2636

7.5.7.2.5	Test requirement	2637

7.5.8	Active TCI state switch delay	2637

7.5.8.0	Minimum conformance requirements	2637

7.5.8.0.0	Known conditions for TCI state	2637

7.5.8.0.1	Minimum conformance requirements for MAC-CE based active TCI state switch	2638

7.5.8.0.2	Minimum conformance requirements for RRC based active TCI state switch	2638

7.5.8.0.3 	Minimum conformance requirements for MAC-CE based active TCI state switch in HST FR2	2639

7.5.8.1	NR SA FR2 MAC-CE based active TCI state switch	2640

7.5.8.1.1	NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state	2640

7.5.8.2	NR SA FR2 RRC based active TCI state switch	2646

7.5.8.2.1	NR SA Pcell FR2 RRC based active TCI state switch for a known TCI state	2646

7.5.8.3	MAC-CE based active TCI state switch for HST FR2 scenario	2653

7.5.8.3.1	NR SA FR2 HST active TCI state switch for a known TCI state	2653

7.5.9	Uplink spatial relation switch delay	2659

7.5.9.0	Minimum conformance requirements	2659

7.5.9.0.1	Minimum conformance requirements for MAC-CE based uplink spatial relation switch delay	2659

7.5.9.0.2	Minimum conformance requirements for RRC based uplink spatial relation switch delay	2659

7.5.9.1	MAC-CE based uplink spatial relation switch	2660

7.5.9.1.1	NR PCell FR2 uplink spatial relation switch associated with known DL-RS	2660

The rate of correct events observed during repeated tests shall be at least [90]%.7.5.9.2	RRC based uplink spatial relation switch	2663

7.5.9.2.1	NR PCell FR2 uplink spatial relation switch associated with known DL-RS	2663

7.5.10	UE specific CBW change	2667

7.5.10.0	Minimum conformance requirements	2667

7.5.10.0.1	Minimum conformance requirements for NR FR2 UE specific CBW change of PCell with non-DRX in SA	2667

7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	2668

7.5.10.1.1	Test purpose	2668

7.5.10.1.2	Test applicability	2668

7.5.10.1.3	Minimum conformance requirements	2668

7.5.10.1.4	Test description	2668

7.5.10.1.5	Test requirement	2669

7.5.11	UE UL carrier RRC reconfiguration delay	2671

7.5.11.1	UE UL carrier RRC reconfiguration delay	2671

7.5.11.1.1	Test purpose	2671

7.5.11.1.2	Test applicability	2672

7.5.11.1.3	Minimum conformance requirements	2672

7.5.11.1.4	Test description	2672

7.5.11.1.5	Test requirement	2674

7.5.12	Conditional PSCell addition and release delay (FR2 SA)	2677

7.5.12.1	NR SA FR2 Addition and Release Delay of PSCell	2677

7.5.13	Unified TCI state switch delay	2682

7.5.13.0	Minimum conformance requirements	2682

7.5.13.0.1	Minimum conformance requirements for MAC-CE based downlink TCI state switch delay for unified TCI	2682

7.5.13.0.2	Minimum conformance requirements for MAC-CE based uplink TCI state switch delay for unified TCI	2683

7.5.13.1	NR SA FR2 MAC-CE based active joint TCI state switch	2683

7.5.13.1.1	Test purpose	2683

7.5.13.1.2	Test applicability	2683

7.5.13.1.3	Minimum conformance requirements	2683

7.5.13.1.4	Test description	2683

7.5.13.1.5	Test requirement	2685

7.5.13.2	NR SA FR2 MAC-CE based active uplink TCI state switch	2686

7.5.13.2.1	Test purpose	2686

7.5.13.2.2	Test applicability	2686

7.5.13.2.3	Minimum conformance requirements	2686

7.5.13.2.4	Test description	2687

7.5.13.2.5	Test requirement	2688

7.5.13.3	NR SA FR2 MAC-CE based active downlink TCI state switch	2690

7.5.13.3.1	Test purpose	2690

7.5.13.3.2	Test applicability	2690

7.5.13.3.3	Minimum conformance requirements	2690

7.5.13.3.4	Test description	2690

7.5.13.3.5	Test requirement	2692

7.5.14	PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2	2694

7.5.14.1	Test purpose	2694

7.5.14.2	Test applicability	2694

7.5.14.3	Minimum conformance requirements	2694

7.5.14.3.1	SCG Activation Delay Requirement	2694

7.5.14.3.2	SCG Deactivation Delay Requirement	2696

7.5.14.4	Test description	2696

7.5.14.4.1	Initial conditions	2696

7.5.14.4.3	Message contents	2698

7.5.14.5	Test requirement	2698

7.6	Measurement procedures	2701

7.6.1	Intra-frequency measurements	2701

7.6.1.0	Minimum conformance requirements	2701

7.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap	2701

7.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap	2702

7.6.1.1	NR SA FR2 event-triggered reporting without gap in non-DRX	2704

7.6.1.2	NR SA FR2 event-triggered reporting without gap in DRX	2709

7.6.1.3	NR SA FR2 event-triggered reporting with gap in non-DRX	2714

7.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	2720

7.6.1.5	NR SA FR2 event triggered reporting test without gap in non-DRX for UE configured with highSpeedMeasFlagFR2-r17	2726

7.6.2	Inter-frequency measurements	2730

7.6.2.0	Minimum conformance requirements for Inter-frequency measurements	2730

7.6.2.1	NR SA FR2-FR2 event-triggered reporting in non-DRX	2730

7.6.2.2	NR SA FR2-FR2 event-triggered reporting in DRX	2736

7.6.2.3	NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection	2742

7.6.2.4	NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection	2747

7.6.2.5	NR SA FR1-FR2 event-triggered reporting in non-DRX	2753

7.6.2.6	NR SA FR1-FR2 event-triggered reporting in DRX	2759

7.6.2.7	NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection	2765

7.6.2.8	NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection	2771

7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	2777

7.6.3	L1-RSRP measurement for beam reporting	2782

7.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	2782

7.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	2782

7.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	2782

7.6.3.0.3	Minimum conformance requirements for Inter-cell SSB based L1-RSRP Reporting	2782

7.6.3.1	NR SA FR2 SSB-based L1-RSRP measurement in non-DRX	2784

7.6.3.2	NR SA FR2 SSB-based L1-RSRP measurement in DRX	2789

7.6.3.3	NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX	2793

7.6.3.4	NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX	2797

7.6.3.5	NR SA FR2 SSB based L1-RSRP measurement test when DRX is used for power class 6 UE configured with highSpeedMeasFlagFR2-r17	2800

7.6.3.6	NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used	2806

7.6.4	CLI measurements	2813

7.6.4.0	Minimum conformance requirements	2813

7.6.4.0.1	Minimum conformance requirements for SRS-RSRP measurement period	2813

7.6.4.1	NR SA FR2 SRS-RSRP measurement in non-DRX	2814

7.6.4.2	NR SA FR2 CLI-RSSI measurement in non-DRX	2818

7.6.4.2.1	Test purpose	2818

7.6.4.2.2	Test applicability	2818

7.6.4.2.3	Minimum conformance requirements	2818

7.6.4.2.4	Test description	2818

7.6.4.2.5	Test requirement	2820

7.6.5	NR measurements with autonomous gaps	2821

7.6.5.0	Minimum conformance requirements	2821

7.6.5.0.1	Minimum conformance requirements for SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	2821

7.6.5.0.1.1	Minimum conformance requirements for interruptions when identifying CGI of an NR cell with autonomous gaps	2821

7.6.5.0.1.2	Minimum conformance requirements for NR measurements with autonomous gaps	2822

7.6.5.0.1.2.1	Introduction	2822

7.6.5.0.1.2.2	CGI identification of an NR cell with autonomous gaps	2822

7.6.5.0.1.2.3	CGI reporting delay	2823

7.6.5.1	SA inter-frequency CGI reporting in autonomous gaps test (PCell in FR2)	2823

7.6.6	L1-SINR measurement for beam reporting	2828

7.6.6.0	Minimum conformance requirements	2828

7.6.6.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	2828

7.6.6.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured	2828

7.6.6.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	2828

7.6.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in non-DRX	2828

7.6.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement in DRX	2832

7.6.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement in DRX	2837

7.6.7 to 7.6.12	2843

7.6.13	UE Rx-Tx time difference measurements for PDC	2843

7.6.13.0	Minimum conformance requirements	2843

7.6.13.0.1	PRS based UE Rx-Tx time difference for propagation delay compensation	2843

7.6.13.0.2	TRS based UE Rx-Tx time difference for propagation delay compensation	2845

7.6.13.1	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS	2847

7.6.13.2	NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS	2851

7.6.14	SA event triggered reporting tests with Pre-MG	2855

7.6.14.0	Minimum conformance requirements	2855

7.6.14.1	NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation	2858

7.6.14.2	Intra-frequency measurement test with SA event triggered reporting tests: with network-controlled activation/deactivation of Pre-MG in FR2	2864

7.6.15	SA event triggered reporting tests with concurrent gaps	2869

7.6.15.0	Minimum conformance requirements	2869

7.6.15.0.1	Minimum conformance requirements for Intra-frequency measurement	2869

7.6.15.0.2	Minimum conformance requirements for Inter-frequency measurement	2870

7.6.15.0.3	Minimum conformance requirements for PRS measurement	2870

7.6.15.1	NR SA FR2 event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements	2871

7.6.15.2	NR SA FR2 event triggered reporting tests For FR2 with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2)	2876

7.6.15.3	NR SA FR2 event triggered reporting tests for FR2 concurrent gap with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement	2885

7.6.16	SA event triggered reporting tests with NCSG	2891

7.6.16.0	Minimum conformance requirements	2891

7.6.16.0.1	Minimum conformance requirements for intra-frequency measurement	2891

7.6.16.0.2	Minimum conformance requirements for inter-frequency measurement	2893

7.6.16.0.3	Minimum conformance requirements for interruptions	2894

7.6.16.1	NR SA FR2 event-triggered reporting test with per-UE NCSG under non-DRX	2895

7.6.16.2	NR SA FR2 event-triggered reporting tests on inter-frequency measurement with NCSG for FR2 when DRX is not used(PCell in FR2)	2901

7.6.16.3	NR SA FR2 event-triggered reporting test on deactivated SCell measurement via NCSG in FR2 in non-DRX	2906

7.7	Measurement performance requirements	2912

7.7.1	SS-RSRP	2912

7.7.1.0	Minimum conformance requirements	2912

7.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	2912

7.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	2912

7.7.1.1	NR SA FR2 SS-RSRP measurement accuracy	2912

7.7.1.2	NR SA FR2-FR2 SS-RSRP measurement accuracy	2920

7.7.1.3	Inter-frequency measurements between FR1 and FR2	2928

7.7.1.3.1	NR SA FR1-FR2 SS-RSRP measurement accuracy	2928

7.7.1.3.2	Void	2935

7.7.2	SS-RSRQ	2935

7.7.2.0	Minimum conformance requirements	2935

7.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	2935

7.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	2935

7.7.2.1	NR SA FR2 SS-RSRQ measurement accuracy	2935

7.7.2.2	NR SA FR2-FR2 SS-RSRQ measurement accuracy	2939

7.7.3	SS-SINR	2945

7.7.3.0	Minimum conformance requirements	2945

7.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	2945

7.7.3.0.2	Inter-frequency SS-SINR measurement accuracy requirements	2945

7.7.3.1	NR SA FR2 SS-SINR measurement accuracy	2945

7.7.3.2	NR SA FR2-FR2 SS-SINR measurement accuracy	2950

7.7.4	L1-RSRP	2957

7.7.4.0	Minimum conformance requirements	2957

7.7.4.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	2957

7.7.4.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	2957

7.7.4.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	2957

7.7.4.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	2957

7.7.4.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	2957

7.7.4.2	NR SA FR2 CSI-RS based L1-RSRP measurement accuracy	2962

7.7.5	SRS-RSRP	2968

7.7.5.0	Minimum conformance requirements	2968

7.7.5.0.1	Minimum conformance requirements for SRS-RSRP measurement accuracy	2968

7.7.5.1	NR SA FR2 SRS-RSRP measurement accuracy	2970

7.7.5.2	NR SA FR2 CLI-RSSI measurement accuracy	2975

7.7.5.2.1	Test purpose	2976

7.7.5.2.2	Test applicability	2976

7.7.5.2.3	Minimum conformance requirements	2976

7.7.5.2.4	Test description	2976

7.7.5.2.4	Test description	2976

7.7.5.2.5	Test requirement	2977

7.7.6	L1-SINR	2980

7.7.6.0	Minimum conformance requirements	2980

7.7.6.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2980

7.7.6.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR L1-SINR measurement accuracy	2982

7.7.6.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2984

7.7.6.1	NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy	2987

7.7.6.2	NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy	2992

7.7.6.3	NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy	2997

7.7.8	CSI-RSRQ	3002

7.7.8.0	Minimum conformance requirements	3002

7.7.8.0.1	Intra-frequency CSI-RSRQ accuracy requirements	3002

7.7.8.0.1.1	Absolute CSI-RSRQ accuracy	3002

7.7.8.0.2	Inter-frequency CSI-RSRQ accuracy requirements	3003

7.7.8.0.2.1	Absolute CSI-RSRQ accuracy	3003

7.7.8.0.2.2	Relative CSI-RSRQ accuracy	3005

7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	3005

7.7.8.1.1	Test purpose	3005

7.7.8.1.2	Test applicability	3006

7.7.8.1.3	Minimum conformance requirements	3006

7.7.8.1.4	Test description	3006

7.7.8.1.5	Test Requirements	3007

7.7.8.2	SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3009

7.7.8.2.1	Test purpose	3009

7.7.8.2.2	Test applicability	3009

7.7.8.2.3	Minimum conformance requirements	3009

7.7.8.2.4	Test description	3009

7.7.8.2.5	Test Requirements	3011

7.7.9	CSI-SINR	3012

7.7.9.0	Minimum conformance requirements	3012

7.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements	3012

7.7.9.0.1.1	Absolute CSI-SINR accuracy	3012

7.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements	3013

7.7.9.0.2.1	Absolute CSI-SINR accuracy	3013

7.7.9.0.2.2	Relative CSI-SINR accuracy	3015

7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	3015

7.7.9.1.1	Test purpose	3016

7.7.9.1.2	  Test applicability	3016

7.7.9.1.3	Minimum conformance requirements	3016

7.7.9.1.4	Test description	3016

7.7.9.1.5	Test Requirements	3017

7.7.9.2	SA inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	3019

7.7.9.2.1	Test purpose	3019

7.7.9.2.2	Test applicability	3019

7.7.9.2.3	Minimum conformance requirements	3019

7.7.9.2.4	Test description	3020

7.7.9.2.5	Test Requirements	3021

8.0	General	3023

8.1	Void	3023

8.2.1.0	Minimum conformance requirements	3023

8.2.1.0.1	Minimum conformance requirements for E-UTRA-NR FR1 inter-RAT cell reselection	3023

8.2.1.1	E-UTRA - NR FR1 cell re-selection to higher priority NR target cell	3025

8.2.1.2	E-UTRA - NR FR1 Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	3032

8.2.2.0	Minimum conformance requirements	3039

8.2.2.1	E-UTRA – NR FR1 Early Measurement Reporting	3041

8.2.2.2	E-UTRA – NR FR2 Early Measurement Reporting	3049

8.3.1.0	Minimum conformance requirements	3054

8.3.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 handover	3054

8.3.1.1	E-UTRA - NR FR1 handover with known target cell	3055

8.3.1.1.3	Minimum conformance requirements	3055

8.4.1.0	Minimum conformance requirements	3060

8.4.1.0.1	Minimum conformance requirements for E-UTRA - NR FR1 SFTD measurement delay	3060

8.4.1.1	E-UTRA - NR FR1 SFTD measurement delay in non-DRX	3061

8.4.1.2	E-UTRA - NR FR1 SFTD measurement delay in DRX	3067

8.4.2.0	Minimum conformance requirements	3072

8.4.2.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	3072

8.4.2.0.2	Void	3076

8.4.2.1	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in non-DRX	3076

8.4.2.2	E-UTRA event-triggered reporting of a NR FR1 neighbour cell without SSB time index detection in DRX	3082

8.4.2.3	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in non-DRX	3088

8.4.2.4	E-UTRA event-triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX	3094

8.4.2.5	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in non-DRX	3100

8.4.2.6	E-UTRA event-triggered reporting of a NR FR2 neighbour cell without SSB time index detection in DRX	3105

8.4.2.7	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in non-DRX	3110

8.4.2.8	E-UTRA event-triggered reporting of a NR FR2 neighbour cell with SSB time index detection in DRX	3115

8.4.2.9	E-UTRA event triggered reporting of a NR FR1 neighbour cell with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	3120

8.5.1	SFTD measurement accuracy	3126

8.5.1.0	Minimum conformance requirements	3126

8.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	3126

8.5.1.1	E-UTRA - NR FR1 SFTD measurement accuracy	3127

8.5.2	Inter-RAT measurement accuracy	3133

8.5.2.1	SS-RSRP	3133

8.5.2.1.0	Minimum conformance requirements	3133

8.5.2.1.1	SS-RSRP with NR FR1 target cell	3134

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell	3139

8.5.2.2	SS-RSRQ	3144

8.5.2.2.0	Minimum conformance requirements	3144

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell	3144

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell	3147

8.5.2.3	SS-SINR	3152

8.5.2.3.0	Minimum conformance requirements	3152

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell	3152

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell	3155

9	NR sidelink	3160

9.1	NR sidelink in FR1	3160

9.1.1	UE transmit timing	3160

9.1.1.0	Minimum conformance requirements	3160

9.1.1.0.1	Minimum conformance requirements for GNSS as synchronization reference source	3160

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	3160

9.1.1.0.3	Minimum conformance requirements for FR1 NR Cell as synchronization reference source	3160

9.1.1.1	NR SA FR1 UE transmit timing accuracy for GNSS as synchronization reference source	3161

9.1.1.2	NR SA FR1 UE transmit timing accuracy for SyncRef UE as synchronization reference source	3163

9.1.1.3	NR SA FR1 UE transmit timing accuracy for FR1 NR cell as synchronization reference source	3166

9.1.2	Initiation/Cease of S-SSB transmission	3171

9.1.2.0	Minimum conformance requirements	3171

9.1.2.0.1	Minimum conformance requirements for FR1 NR cell as synchronization reference source	3171

9.1.1.0.2	Minimum conformance requirements for SyncRef UE as synchronization reference source	3172

9.1.2.1	NR SA FR1 initiation/cease of S-SSB transmission for FR1 NR cell as synchronization reference source	3173

9.1.2.2	NR SA FR1 initiation/cease of S-SSB transmission for SyncRef UE as synchronization reference source	3178

9.1.3	Synchronization reference selection/reselection	3182

9.1.3.0	Minimum conformance requirements	3182

9.1.3.0.1	Minimum conformance requirements for GNSS configured as the highest priority synchronization reference source	3182

9.1.3.0.2	Minimum conformance requirements for eNB/gNB configured as the highest priority synchronization reference source	3183

9.1.3.1	NR SA FR1 synchronization reference selection/reselection for GNSS configured as the highest priority synchronization reference source	3185

9.1.3.2	NR SA FR1 synchronization reference selection/reselection for FR1 NR Cell configured as the highest priority synchronization reference source	3191

9.1.4	L1 SL-RSRP measurements	3196

9.1.4.0	Minimum conformance requirements	3196

9.1.4.0.1	Minimum conformance requirements for resource selection/reselection, re-evaluation and pre-emption	3196

9.1.4.1	NR SA FR1 L1 SL-RSRP measurement for autonomous resource selection/reselection	3197

9.1.4.2	NR SA FR1 L1 SL-RSRP measurement for resource pre-emption	3203

9.1.4.3	NR SA FR1 L1 SL-RSRP measurement for resource re-evaluation	3209

9.1.5	Congestion control measurement	3216

9.1.5.0	Minimum conformance requirements	3216

9.1.5.0.1	Minimum conformance requirements for congestion control measurements	3216

9.1.5.1	NR SA FR1 congestion control measurement for concurrent operation	3217

9.1.5.2	NR SA FR1 congestion control measurement for PC5-only operation	3225

9.1.6	Congestion control measurement	3231

9.1.6.0	Minimum conformance requirements	3231

9.1.6.0.1	Minimum conformance requirements for interruption to WAN due to NR sidelink communication	3231

9.1.6.1	NR SA FR1 interruption to WAN due to NR sidelink communication	3231

10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	3235

10.0	General	3235

10.0.1	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	3235

10.1	RRC_CONNECTED state mobility	3236

10.2	Timing	3236

10.2.1	UE transmit timing	3236

10.2.1.0	Minimum conformance requirements	3236

10.2.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	3236

10.2.1.1	EN-DC FR1 UE Transmit Timing Test with PSCell under DL CCA	3236

10.2.2	UE timing advance	3244

10.2.2.0	Minimum conformance requirements	3244

10.2.2.0.1	Minimum conformance requirements for timing advance adjustment delay	3244

10.2.2.0.2	Minimum conformance requirements for timing advance adjustment accuracy	3244

10.2.2.1	EN-DC FR1 UE Timing Advance Adjustment Accuracy with PSCell under DL CCA	3244

10.3	Signalling characteristics	3250

10.3.1	Radio link monitoring	3250

10.3.1.0	General	3250

10.3.1.0.1	Minimum conformance requirements for SSB based Radio Link Monitoring under CCA	3251

10.3.1.1	Requirements for determining UE in-sync or out-of-sync status	3254

10.3.1.2	EN-DC FR1 Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	3254

10.3.1.3	EN-DC FR1 Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode under CCA	3262

10.3.1.4	Void	3267

10.3.1.5	Void	3267

10.3.2	Interruption	3268

10.3.2.0	Minimum Conformance Requirements	3268

10.3.2.1	EN-DC FR1 E-UTRAN – NR interruptions during SCell operations with CCA	3268

10.3.3	SCell activation and deactivation delay	3273

10.3.3.0	Minimum Conformance Requirements	3273

10.3.3.0.1	SCell Activation Delay Requirement for Deactivated SCell under CCA	3273

10.3.3.0.2	SCell Deactivation Delay Requirement for Activated SCell under CCA	3278

10.3.3.1	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 160 ms SCell measurement cycle	3278

10.3.3.2	EN-DC FR1 SCell Activation and Deactivation of known NR SCell with NR PSCell and NR SCell under CCA, 640 ms SCell measurement cycle	3285

10.3.3.3	EN-DC FR1 SCell Activation and Deactivation of unknown NR SCell with NR PSCell and NR SCell under CCA	3289

10.3.4	Beam failure detection and link recovery procedures	3293

10.3.4.0	General	3293

10.3.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	3294

10.3.4.0.3	Scheduling availability of UE during beam failure detection under CCA	3296

10.3.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	3296

10.3.4.1	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in non-DRX mode	3296

10.3.4.2	EN-DC FR1 Beam Failure Detection and Link Recovery Test for PSCell configured with SSB-based BFD and LR in DRX mode	3304

10.3.5	Active BWP switching	3312

10.3.5.1	EN-DC FR1 UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA	3312

10.3.5.2	DCI-based and Timer-based Active BWP Switch	3319

10.3.5.2.0	Minimum conformance requirements	3319

10.3.5.2.1	EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	3322

10.3.5.2.2	EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC under CCA	3330

10.3.5.3	RRC-based Active BWP Switch	3338

10.3.5.3.0	Minimum conformance requirements	3338

10.3.5.3.1	EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC under CCA	3339

10.4	Measurement procedures	3345

10.4.1	Intra-frequency measurements	3345

10.4.1.0	Minimum conformance requirements	3345

10.4.1.1	EN-DC FR1 Event-triggered reporting tests on PSCC without gaps under non-DRX and CCA	3359

10.4.1.2	Void	3364

10.4.1.3	Void	3364

10.4.1.4	EN-DC FR1 Event-triggered reporting tests on PSCC with per-UE gaps under DRX and CCA	3364

10.4.1.5	3370

10.4.1.6	3370

10.4.1.7	3370

10.4.1.8	3370

10.4.2	Inter-frequency measurements	3370

10.4.2.0	Minimum conformance requirements	3370

10.4.2.1	EN-DC FR1-FR1 RSSI measurement reporting under CCA	3376

10.4.2.2	EN-DC FR1-FR1 Channel occupancy measurement reporting under CCA	3377

10.4.2.3	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is not used	3379

10.4.2.4	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA without SSB time index detection when DRX is used	3385

10.4.2.5	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is not used	3392

10.4.2.6	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with CCA with SSB time index detection when DRX is used	3398

10.4.2.7	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell without SSB time index detection when DRX is not used	3405

10.4.2.8	EN-DC FR1-FR1 Event triggered reporting for FR1 cell without SSB time index detection when DRX is used	3412

10.4.2.9	EN-DC FR1-FR1 Event-triggered reporting for FR1 cell with SSB time index detection when DRX is not used	3420

10.4.2.10	EN-DC FR1-FR1 Event triggered reporting for FR1 cell with SSB time index detection when DRX is used	3428

10.4.3	L1-RSRP measurement for beam reporting under CCA	3436

10.4.3.0	Minimum conformance requirements	3436

10.4.3.1	EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is not used	3438

10.4.3.2	EN-DC FR1 SSB based L1-RSRP measurement on PSCC under CCA when DRX is used	3444

10.4.3.3	EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is not used	3448

10.4.3.4	EN-DC FR1 SSB based L1-RSRP measurement on SCC under CCA when DRX is used	3453

10.4.4	E-UTRAN-NR inter-RAT measurements on NR carrier frequency under CCA	3458

10.4.4.0	Minimum conformance requirements	3458

10.4.4.1	EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3461

10.4.4.2	EN-DC FR1 E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3467

10.4.4.3	EN-DC FR1 E-UTRA-NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3475

10.4.4.4	EN-DC FR1 E-UTRA-NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3481

10.5	Measurement Performance	3489

10.5.1	SS-RSRP	3489

10.5.1.0	Minimum Conformance Requirements	3489

10.5.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3489

10.5.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	3489

10.5.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3490

10.5.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	3490

10.5.1.1	EN-DC FR1 intra-frequency SS-RSRP measurement accuracy on a CCA serving cell	3491

10.5.1.2	EN-DC FR1-FR1 inter-frequency SS-RSRP measurement accuracy with CCA serving cell and CCA target cell	3496

10.5.2	SS-RSRQ	3502

10.5.2.0	Minimum Conformance Requirements	3502

10.5.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3502

10.5.2.0.2	Void	3503

10.5.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3503

10.5.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3504

10.5.2.1	EN-DC FR1 intra-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	3504

10.5.2.2	EN-DC FR1-FR1 inter-frequency SS-RSRQ measurement accuracy with serving cell and target cell under CCA	3510

10.5.3	SS-SINR	3517

10.5.3.0	Minimum Conformance Requirements	3517

10.5.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	3517

10.5.3.0.2	Void	3517

10.5.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	3517

10.5.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	3518

10.5.3.1	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on PSCC under CCA	3519

10.5.3.2	EN-DC FR1-FR1 inter-frequency SS-SINR measurement accuracy on PSCC under CCA	3524

10.5.3.3	EN-DC FR1 intra-frequency SS-SINR measurement accuracy on SCC under CCA	3530

10.5.4	L1-RSRP measurement accuracy for beam reporting with CCA serving cell	3536

10.5.4.0	Minimum conformance requirements	3536

10.5.4.1	EN-DC FR1 SSB based L1-RSRP measurement accuracy with CCA	3538

10.5.5	RSSI	3544

10.5.5.0	Minimum Conformance Requirements	3544

10.5.5.0.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	3544

10.5.5.0.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	3544

10.5.5.1	EN-DC FR1 RSSI measurement accuracy on PSCC with CCA	3544

10.5.5.2	EN-DC FR1 RSSI measurement accuracy on SCC with CCA	3549

10.5.5.3	 EN-DC FR1-FR1 RSSI measurement accuracy on a carrier with CCA	3553

10.5.6	Channel occupancy	3558

10.5.6.0	Minimum Conformance Requirements	3558

10.5.6.0.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	3558

10.5.6.0.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	3558

10.5.6.1	EN-DC FR1 Channel occupancy measurement accuracy on PSCC with CCA	3559

10.5.6.2	EN-DC FR1 Channel occupancy measurement accuracy on SCC with CCA	3563

10.5.6.3	EN-DC FR1-FR1 Channel occupancy measurement accuracy on a carrier with CCA	3568

11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	3573

11.0	General	3573

11.0.1	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	3573

11.1	RRC_IDLE state mobility	3574

11.2	RRC_CONNECTED state mobility	3574

11.2.1	3574

11.2.2	RRC connection mobility control	3574

11.2.2.1	RRC re-establishment	3574

11.2.2.2	Random Access	3574

11.2.2.3	RRC connection release with redirection	3574

11.2.2.3.0	Minimum conformance requirements	3574

11.2.2.3.1	NR SA FR1 Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	3575

11.2.2.3.2	NR SA FR1 Redirection from NR FR1 carrier without CCA to NR FR1 carrier under CCA	3581

11.3	Timing	3586

11.4	Signalling characteristics	3587

11.4.1	Radio link monitoring	3587

11.4.1.0	General	3587

11.4.1.0.1	Minimum conformance requirements for SSB based radio link monitoring under CCA	3587

11.4.1.1	Requirements for determining UE in-sync or out-of-sync status	3587

11.4.1.2	NR SA FR1 Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	3587

11.4.1.3	NR SA FR1 Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode under CCA	3595

11.4.2	Interruptions	3601

11.4.3	SCell activation and deactivation delay	3601

11.4.4	Beam failure detection and link recovery procedures	3602

11.4.4.0	General	3602

11.4.4.0.1	Minimum conformance requirements for SSB based Beam Failure Detection under CCA	3602

11.4.4.0.2	Void	3602

11.4.4.0.3	Scheduling availability of UE during beam failure detection under CCA	3602

11.4.4.0.4	Scheduling availability of UE during candidate beam detection under CCA	3602

11.4.4.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode under CCA	3602

11.4.4.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode under CCA	3611

11.4.5	Active BWP switching	3619

11.4.5.1	NR SA FR1 UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	3619

11.4.5.2	DCI-based and Timer-based Active BWP Switch	3626

11.4.5.2.0	Minimum conformance requirements	3626

11.4.5.2.1	NR SA FR1 DCI-based DL active BWP switch of PCell with non-DRX under CCA	3628

11.4.5.2.2	NR SA FR1 DCI-based DL active BWP switch with non-DRX under CCA	3636

11.4.5.3	RRC-based Active BWP Switch	3644

11.4.5.3.0	Minimum conformance requirements	3644

11.4.5.3.1	NR SA FR1 RRC-based DL active BWP switch of Cell with non-DRX under CCA	3644

11.5	Measurement procedure	3650

11.5.1	Intra-frequency measurements	3650

11.5.1.0	Minimum Conformance Requirements	3650

11.5.1.1	SA FR1 Event-triggered reporting tests on PCC without gaps under non-DRX	3651

11.5.1.2	SA FR1 Event-triggered reporting tests on PCC without gaps under DRX	3655

11.5.1.3	SA FR1 Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	3660

11.5.1.4	SA FR1 Event-triggered reporting tests on PCC with per-UE gaps under DRX	3662

11.5.1.5 to 11.5.1.8	3663

11.5.1.9	SA FR1 RSSI measurement reporting on PCC	3664

11.5.2	Inter-frequency measurements	3665

11.5.2.0	Minimum Conformance Requirements	3665

11.5.2.1	3666

11.5.2.2	3666

11.5.2.3	NR SA FR1-FR1 Event triggered reporting tests with CCA without SSB time index detection when DRX is not used	3666

11.5.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	3671

11.5.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	3677

11.5.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	3682

11.5.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	3687

11.5.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	3692

11.5.2.9	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	3697

11.5.2.10	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	3702

11.6	Measurement Performance	3709

11.6.1	SS-RSRP	3709

11.6.1.0	Minimum Conformance Requirements	3709

11.6.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3709

11.6.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements under CCA	3709

11.6.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements under CCA	3709

11.6.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements under CCA	3709

11.6.1.1	NR SA FR1 intra-frequency SS-RSRP measurement accuracy on a carrier frequency with CCA	3709

11.6.1.2	NR SA FR1 intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	3714

11.6.2	SS-RSRQ	3719

11.6.2.0	Minimum Conformance Requirements	3719

11.6.2.0.1	Intra-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3719

11.6.2.0.2	Intra-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3720

11.6.2.0.3	Inter-frequency absolute SS-RSRQ measurement accuracy requirements under CCA	3720

11.6.2.0.4	Inter-frequency relative SS-RSRQ measurement accuracy requirements under CCA	3720

11.6.2.1	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	3720

11.6.2.2	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA	3726

11.6.2.3	NR SA FR1 intra-frequency SS-RSRQ measurement accuracy on SCC on a carrier frequency with CCA	3731

11.6.2.4	NR SA FR1 inter-frequency SS-RSRQ measurement accuracy on a carrier frequency with CCA from carrier without CCA	3738

11.6.3	SS-SINR	3746

11.6.3.0	Minimum Conformance Requirements	3746

11.6.3.0.1	Intra-frequency absolute SS-SINR measurement accuracy requirements under CCA	3746

11.6.3.0.2	Void	3746

11.6.3.0.3	Inter-frequency absolute SS-SINR measurement accuracy requirements under CCA	3746

11.6.3.0.4	Inter-frequency relative SS-SINR measurement accuracy requirements under CCA	3746

11.6.3.1	NR SA FR1 Intra-frequency SS-SINR measurement accuracy on a carrier frequency with CCA	3746

11.6.3.2	NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA	3752

11.6.3.3	NR SA FR1 Intra-frequency SS-SINR measurement accuracy on SCC on a carrier frequency with CCA	3758

11.6.3.4	NR SA FR1 Inter-frequency SS-SINR measurement accuracy on a carrier frequency with CCA from carrier without CCA	3764

11.6.4	3774

11.6.5	SS-SINR	3774

11.6.5.0	Minimum Conformance Requirements	3774

11.6.5.1	NR SA FR1 Intra-frequency RSSI measurement accuracy on PCC with CCA	3774

11.6.5.2	NR SA FR1 Intra-frequency RSSI measurement accuracy on SCC with CCA	3778

12 to 13	FFS	3782

14	NR standalone tests for Satellite access	3782

14.0	General	3782

14.0.1	Principle of testing GSO and NGSO scenarios	3782

14.0.2	Principle of testing different RRM requirements	3782

14.0.3	Principle of testing different ephemeris formats	3783

14.0.4	General setup for SIB19	3785

14.0.5	Initial test environment conditions for RRM NTN tests	3785

14.1	RRC_IDLE state mobility	3786

14.1.0	Minimum conformance requirements	3786

14.1.0.1	Measurements of intra-frequency NR cells	3786

14.1.0.2	Measurements of inter-frequency NR cells	3788

14.1.1	NR SA FR1 Cell Reselection for Satellite Access	3791

14.1.2	NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access	3791

14.1.3	NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access	3796

14.1.4	NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access	3800

14.2	RRC_CONNECTED state mobility	3804

14.2.1	Handover for SAN	3804

14.2.1.0	Minimum conformance requirements	3804

14.2.1.0.1	Minimum conformance requirements for handover	3804

14.2.1.0.2	Minimum conformance requirements for conditional handover	3805

14.2.1.1	NR SA FR1 Handover for Satellite Access	3807

14.2.1.2	NR SA FR1-FR1 Handover for Satellite Access	3807

14.2.1.3	NR SA FR1 Time-based Conditional Handover for Satellite Access	3807

14.2.1.4	NR SA FR1-FR1 Time-based Conditional Handover for NR Satellite Access	3812

14.3	Timing for Satellite Access	3816

14.3.1	UE transmit timing for Satellite Access	3816

14.3.1.0	Minimum conformance requirements	3816

14.3.1.1	NR SA FR1 UE transmit timing accuracy for Satellite Access	3817

14.4	Signalling characteristics	3824

14.5	Measurement procedure	3824

14.5.1	Intra-frequency Measurements for SAN	3824

14.5.1.0	Minimum conformance requirements	3824

14.5.1.0.1	Minimum conformance requitements for intra-frequency measurements without measurement gaps	3824

14.5.1.0.2	Minimum conformance requitements for intra-frequency measurements with measurement gaps	3827

14.5.1.1	NR SA FR1 Event-triggered reporting tests without gap under non-DRX for NR satellite access	3829

14.5.1.2	NR SA FR1 Event triggered reporting tests without gap under DRX for NR satellite access	3833

14.5.1.3	NR SA FR1 event triggered reporting tests without gap under non-DRX with FDD PCell with SSB index reading for NR satellite access	3836

14.5.1.4	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	3840

14.5.1.5	NR SA FR1 Event-triggered reporting without SSB time index detection when DRX is used with two concurrent fully non-overlapped (FNO) gaps for satellite access	3844

14.5.1.6	NR SA FR1 Event-triggered reporting with SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	3849

14.5.2	Inter-frequency Measurements for SAN	3853

14.5.2.0	Minimum conformance requirements	3853

14.5.2.1	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with single gap for satellite access	3856

14.5.2.2	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is used with single gap for satellite access	3862

14.5.2.3	NR SA FR1-FR1 Event-triggered reporting with SSB time index detection when DRX is not used with single gap for satellite access	3868

14.5.2.4	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent fully non-overlapped (FNO) gaps for satellite access	3874

14.5.2.5	Void	3880

14.5.2.6	NR SA FR1-FR1 Event-triggered reporting without SSB time index detection when DRX is not used with two concurrent partial overlapping (PPO) gaps for satellite access	3880

14.6	Measurement Performance Requirements	3886

14.6.1	3886

14.6.2	3886

14.6.3	SS-SINR Requirements for SAN	3886

14.6.3.0	Minimum conformance requirements	3886

14.6.3.0.1	Intra-Frequency SS-SINR accuracy requirements in FR1	3886

14.6.3.0.2	Inter-Frequency SS-SINR accuracy requirements in FR1	3887

14.6.3.1	NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access	3888

14.6.3.2	NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access	3893

15	FFS	3898

16	NR standalone tests with all NR cells in FR1 for RedCap	3901

16.0	General	3901

16.1	RRC_IDLE state mobility	3901

16.1.1	NR cell re-selection	3901

16.1.1.0	Minimum conformance requirements	3901

16.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	3901

16.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	3903

16.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3907

16.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	3910

16.1.1.1	NR SA FR1 Cell reselection for 1 Rx UE	3913

16.1.1.2	NR SA FR1 Cell reselection for 2 Rx UE	3921

16.1.1.3	NR SA FR1-FR1 Cell reselection for 1 Rx UE	3925

16.1.1.4	NR SA FR1-FR1 Cell reselection for 2 Rx UE	3930

16.1.1.5	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3934

16.1.1.6	NR SA FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3942

16.1.1.7	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3947

16.1.1.8	NR SA FR1-FR1 Cell reselection for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3955

16.1.2	NR – E-UTRA cell re-selection	3963

16.1.2.0	Minimum conformance requirements	3963

16.1.2.0.1	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 1 Rx	3963

16.1.2.0.2	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection for 2 Rx	3965

16.1.2.0.3	Minimum conformance requirements for NR SA FR1 - E-UTRA cell re-selection	3965

16.1.2.1	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 1RX	3967

16.1.2.2	NR SA FR1 - E-UTRA Cell reselection to higher priority E-UTRA for 2RX	3973

16.1.2.3	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 1RX	3977

16.1.2.4	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for 2RX	3983

16.1.2.5	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 1 Rx UE	3987

16.1.2.6	NR SA FR1 - E-UTRA Cell reselection to lower priority E-UTRA for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	3993

16.2	RRC_INACTIVE state mobility for RedCap	3997

16.2.1	Configured Grant based Small Data Transmissions for RedCap	3997

16.2.1.0	Minimum conformance requirements	3997

16.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	3997

16.2.1.1	NR SA FR1 CG-SDT for 1Rx UE	3999

16.2.1.2	NR SA FR1 CG-SDT for 2Rx UE	4003

16.3	RRC_CONNECTED state mobility for RedCap	4006

16.3.1	Handover	4006

16.3.1.0	Minimum conformance requirements	4006

16.3.1.0.1	Minimum conformance requirements for NR – E-UTRAN handover	4006

16.3.1.0.2	Minimum conformance requirements for NR FR1 – NR FR1 handover	4006

16.3.1.1	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 1 Rx UE	4007

16.3.1.2	NR SA FR1 Intra-frequency handover from FR1 to FR1 with known target cell for 2 Rx UE	4010

16.3.1.3	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	4014

16.3.1.4	NR SA FR1 Intra-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	4020

16.3.1.5	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 1 Rx UE	4025

16.3.1.6	NR SA FR1-FR1 Inter-frequency handover from FR1 to FR1 with unknown target cell for 2 Rx UE	4031

16.3.1.7	NR SA FR1 - E-UTRA handover for 1Rx UE	4038

16.3.1.8	NR SA FR1 - E-UTRA handover for 2Rx UE	4043

16.3.1.9	NR SA FR1 - E-UTRA handover with unknown target cell for 1 Rx UE	4048

16.3.1.10	NR SA FR1 - E-UTRA handover with unknown target cell for 2 Rx UE	4052

16.3.2	RRC connection mobility control for RedCap	4056

16.3.2.1	RRC re-establishment for RedCap	4056

16.3.2.1.0	Minimum conformance requirements	4056

16.3.2.1.1	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE	4056

16.3.2.1.2	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE	4059

16.3.2.1.3	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 1 Rx UE	4062

16.3.2.1.4	NR SA FR1-FR1 Inter-frequency RRC Re-establishment in FR1 for 2 Rx UE	4065

16.3.2.1.5	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 1 Rx UE without serving cell timing	4068

16.3.2.1.6	NR SA FR1 Intra-frequency RRC Re-establishment in FR1 for 2 Rx UE without serving cell timing	4071

16.3.2.2	Random access for RedCap	4074

16.3.2.2.0	Minimum conformance requirements	4074

16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	4075

16.3.2.2.2	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	4078

16.3.2.2.3	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 1 Rx UE	4081

16.3.2.2.4	NR SA FR1 4-step RA type non-contention based random access test in FR1 for NR standalone for 2 Rx UE	4084

16.3.2.2.5	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE	4087

16.3.2.2.6	NR SA FR1 2-step RA type contention based random access test in FR1 for NR standalone for 2 Rx UE	4090

16.3.2.2.7	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 1 RX UE	4093

16.3.2.2.8	NR SA FR1 2-step RA type non-contention based test in FR1 for NR standalone for 2 RX UE	4096

16.3.2.3	RRC connection release with redirection for RedCap	4099

16.3.2.3.0	Minimum conformance requirements	4099

16.3.2.3.1	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE	4099

16.3.2.3.2	NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE	4103

16.3.2.3.3	NR SA FR1 - E-UTRA Redirection from NR in FR1 to E-UTRAN for 1 Rx UE	4106

16.3.2.3.4	NR SA FR1 - E-UTRA Redirection from NR in FR1 to E-UTRAN for 2 Rx UE	4111

16.4	Timing for RedCap	4116

16.4.1	UE transmit timing for RedCap	4116

16.4.1.0	Minimum conformance requirements	4116

16.4.1.1	NR SA FR1 NR UE Transmit Timing Test for FR1 for 1Rx RedCap UE	4117

16.4.1.2	NR SA FR1 NR UE Transmit Timing Test for FR1 for 2Rx RedCap UE	4121

16.4.2	4125

16.4.3	Timing advance for RedCap	4125

16.4.3.0	Minimum conformance requirement	4125

16.4.3.1	NR SA FR1 SA FR1 timing advance adjustment accuracy for 1 Rx UE	4125

16.4.3.2	NR SA FR1 SA FR1 timing advance adjustment accuracy for 2 Rx UE	4128

16.5	Signalling characteristics for RedCap	4131

16.5.1	Radio link monitoring for RedCap	4131

16.5.1.0	Minimum requirements	4131

16.5.1.0.1	General	4131

16.5.1.0.2	Minimum conformance requirements for SSB-based RLM	4135

16.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	4136

16.5.1.1	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	4137

16.5.1.2	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	4141

16.5.1.3	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 1 Rx UE	4145

16.5.1.4	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for 2 Rx UE	4150

16.5.1.5	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	4154

16.5.1.6	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	4159

16.5.1.7	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 1 Rx UE	4164

16.5.1.8	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode for 2 Rx UE	4169

16.5.1.9	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	4173

16.5.1.10	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	4176

16.5.1.11	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	4179

16.5.1.12	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 2 Rx UE	4182

16.5.1.13	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode for 1 Rx UE	4186

16.5.1.14	NR SA FR1 Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	4189

16.5.1.15	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 1 Rx UE	4192

16.5.1.16	NR SA FR1 Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode for 2 Rx UE	4195

16.5.2	Beam Failure Detection and Link recovery procedures for RedCap	4198

16.5.2.0	Minimum requirements	4198

16.5.2.0.1	General	4198

16.5.2.0.2	Minimum conformance requirements for SSB-based BFD and CBD	4200

16.5.2.0.3	Minimum conformance requirements for CSI-RS based RLM	4202

16.5.2.1	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 1 Rx UE	4204

16.5.2.2	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode for 2 Rx UE	4209

16.5.2.3	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 1 Rx UE	4214

16.5.2.4	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode for 2 Rx UE	4220

16.5.2.5	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 1 Rx UE	4225

16.5.2.6	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for 2 Rx UE	4230

16.5.2.7	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 1 Rx UE	4233

16.5.2.8	NR SA FR1 Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode for 2 Rx UE	4236

16.5.3	Active BWP switch delay for RedCap	4241

16.5.3.1	DCI-based and time-based active BWP switch for RedCap	4241

16.5.3.1.0	Minimum conformance requirements	4241

16.5.3.1.1	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 1 Rx UE	4242

16.5.3.1.2	NR SA FR1 DCI-based DL active BWP switch in non-DRX for 2 Rx UE	4246

16.5.3.2	RRC-based active BWP switch for RedCap	4251

16.5.3.2.0	Minimum conformance requirements	4251

16.5.3.2.1	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 1 Rx UE	4252

16.5.3.2.2	NR SA FR1 RRC-based DL active BWP switch in non-DRX for 2 Rx UE	4256

16.5.4	UE specific CBW change for RedCap	4261

16.5.4.0	Minimum conformance requirements	4261

16.5.4.0.1	Minimum conformance requirements for UE specific CBW change	4261

16.5.4.1	NR SA FR1 UE specific CBW change on PCell in non-DRX for 1 Rx UE	4262

16.5.4.2	NR SA FR1 UE specific CBW change on PCell in non-DRX for 2 Rx UE	4264

16.6	Measurement procedures for RedCap	4267

16.6.1	Intra-frequency measurements for RedCap	4268

16.6.1.0	Minimum conformance requirements	4268

16.6.1.0.1	General	4268

16.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	4269

16.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	4271

16.6.1.1	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 1 Rx UE	4272

16.6.1.2	NR SA FR1 Event triggered reporting tests without gap under non-DRX for 2 Rx UE	4278

16.6.1.3	NR SA FR1 Event triggered reporting tests without gap under DRX for 1 Rx UE	4284

16.6.1.4	NR SA FR1 Event triggered reporting tests without gap under DRX for 2 Rx UE	4286

16.6.1.5	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 1 Rx UE	4289

16.6.1.6	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX for 2 Rx UE	4292

16.6.1.7	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 1 Rx UE	4295

16.6.1.8	NR SA FR1 Event triggered reporting tests with per-UE gaps under DRX for 2 Rx UE	4298

16.6.1.9	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 1 Rx UE	4301

16.6.1.10	NR SA FR1 Event triggered reporting tests without gap under non-DRX with SSB index reading for 2 Rx UE	4307

16.6.1.11	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 1 Rx UE	4312

16.6.1.12	NR SA FR1 Event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading for 2Rx UE	4315

16.6.2	Inter-frequency measurements for RedCap	4318

16.6.2.0	Minimum conformance requirements	4318

16.6.2.0.1	Minimum conformance requirements for inter-frequency event-triggered measurement with measurement gaps	4318

16.6.2.0.2	Minimum conformance requirements for inter-frequency event-triggered measurements without measurement gaps	4320

16.6.2.1	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 1 Rx UE	4323

16.6.2.2	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used for 2 Rx UE	4328

16.6.2.3	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 1 Rx UE	4334

16.6.2.4	NR SA FR1-FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used for 2 Rx UE	4339

16.6.2.5	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 1 Rx UE	4343

16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE	4347

16.6.2.7	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 1 Rx UE	4351

16.6.2.8	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used for 2 Rx UE	4355

16.6.2.9	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 1 Rx UE	4360

16.6.2.10	NR SA FR1-FR1 Event triggered reporting tests with additional mandatory gap pattern for 2 Rx UE	4366

16.6.2.11	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 1 Rx UE	4371

16.6.2.12	NR SA FR1-FR1 Event triggered reporting tests for FR1 when DRX is used for 2 Rx UE	4376

16.6.3	Inter-RAT measurements for RedCap	4381

16.6.3.0	Minimum conformance requirements	4381

16.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD	4381

16.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD	4384

16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE	4387

16.6.3.1	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 1 Rx UE	4393

16.6.3.2	NR - E-UTRA event-triggered reporting in non-DRX in FR1 for 2 Rx UE	4393

16.6.3.3	NR - E-UTRA event-triggered reporting in DRX in FR1 for 1 Rx UE	4398

16.6.3.4	NR - E-UTRA event-triggered reporting in DRX in FR1 for 2 Rx UE	4403

16.6.4	L1-RSRP measurement for beam reporting for RedCap	4408

16.6.4.0	Minimum conformance requirements	4408

16.6.4.0.1	General	4408

16.6.4.1	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 1 Rx UE	4412

16.6.4.2	NR SA FR1 SSB based L1-RSRP measurement when DRX is not used for 2 Rx UE	4416

16.6.4.3	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 1 Rx UE	4419

16.6.4.4	NR SA FR1 SSB based L1-RSRP measurement when DRX is used for 2 Rx UE	4423

16.6.4.5	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	4426

16.6.4.6	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 2 Rx UE	4429

16.6.4.7	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is not used for 1 Rx UE	4431

16.6.4.8	NR SA FR1 CSI-RS based L1-RSRP measurement when DRX is used for 2 Rx UE	4434

16.6.5	NR measurements with autonomous gaps	4436

16.6.5.0	Minimum conformance requirements	4436

16.6.5.0.1	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	4436

16.6.5.0.2	Minimum conformance requirements for NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	4438

16.6.5.0.3	Minimum conformance requirements for CGI identification of an E-UTRA cell with autonomous gaps	4438

16.6.5.1	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 1 Rx UE	4439

16.6.5.2	NR SA FR1 intra-frequency CGI identification of NR neighbour cell in FR1 for 2 Rx UE	4442

16.6.5.3	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 1 Rx UE	4445

16.6.5.4	NR SA FR1 Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA for 2 Rx UE	4453

16.7	Measurement performance requirements for RedCap	4458

16.7.1	SS-RSRP for RedCap	4458

16.7.1.0	Minimum conformance requirements	4458

16.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements	4458

16.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements	4459

16.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements	4460

16.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements	4461

16.7.1.1	Intra-frequency measurements for RedCap	4462

16.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	4462

16.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	4468

16.7.1.2.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	4470

16.7.1.2.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	4477

16.7.1.2	4479

16.7.1.3	Inter-frequency measurements for RedCap for 1 Rx UE	4479

16.7.1.3.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 1 Rx UE	4479

16.7.1.3.2	NR SA FR1 SS-RSRP relative measurement accuracy for 1 Rx UE	4486

16.7.1.4	Inter-frequency measurements for RedCap for 2 Rx UE	4488

16.7.1.4.1	NR SA FR1 SS-RSRP absolute measurement accuracy for 2 Rx UE	4488

16.7.1.4.2	NR SA FR1 SS-RSRP relative measurement accuracy for 2 Rx UE	4494

16.7.2	SS-RSRQ	4496

16.7.2.0	Minimum conformance requirements	4496

16.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	4496

16.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements	4497

16.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements	4497

16.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy for 1 Rx UE	4498

16.7.2.2	NR SA FR1 SS-RSRQ measurement accuracy for 2 Rx UE	4502

16.7.2.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4506

16.7.2.3.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4506

16.7.2.3.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4511

16.7.2.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4513

16.7.2.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4513

16.7.2.4.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4517

16.7.3	SS-SINR	4519

16.7.3.0	Minimum conformance requirements	4519

16.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements	4519

16.7.3.0.2	Inter-frequency SS-SINR absolute measurement accuracy requirements	4520

16.7.3.0.3	Inter-frequency SS-SINR relative measurement accuracy requirements	4520

16.7.3.1	NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4521

16.7.3.2	NR SA FR1 Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4525

16.7.3.3	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4529

16.7.3.3.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4529

16.7.3.3.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 1 Rx UE	4533

16.7.3.4	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4535

16.7.3.4.1	NR SA FR1-FR1 Inter-frequency absolute measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4535

16.7.3.4.2	NR SA FR1-FR1 Inter-frequency relative measurement accuracy with FR1 serving cell and FR1 target cell for 2 Rx UE	4539

16.7.4	L1-RSRP measurement for beam reporting	4541

16.7.4.0	Minimum conformance requirements	4541

16.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements	4541

16.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements	4541

16.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements	4542

16.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements	4542

16.7.4.1	SSB based L1-RSRP measurements for 1 Rx UE	4542

16.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 1 Rx UE	4542

16.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement for 1 Rx UE	4547

16.7.4.2	SSB based L1-RSRP measurements for 2 Rx UE	4549

16.7.4.2.1	NR SA FR1 SSB based L1-RSRP absolute measurement for 2 Rx UE	4549

16.7.4.2.2	NR SA FR1 SSB based L1-RSRP relative measurement for 2 Rx UE	4553

16.7.4.3	CSI-RS based L1-RSRP measurements for 1 Rx UE	4555

16.7.4.3.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 1 Rx UE	4555

16.7.4.3.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 1 Rx UE	4559

16.7.4.4	CSI-RS based L1-RSRP measurements for 2 Rx UE	4561

16.7.4.4.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement on resource set with repetition off for 2 Rx UE	4561

16.7.4.4.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement on resource set with repetition off for 2 Rx UE	4565

16.7.5	E-UTRAN RSRP	4567

16.7.5.0	Minimum conformance requirements	4567

16.7.5.0.1	E-UTRAN RSRP absolute accuracy	4567

16.7.5.1	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE	4568

16.7.5.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE	4572

16.7.6	E-UTRAN RSRQ	4576

16.7.6.0	Minimum conformance requirements	4576

16.7.6.0.1	E-UTRAN RSRQ absolute accuracy	4576

16.7.6.1	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 1 Rx UE	4577

16.7.6.2	NR SA FR1 - E-UTRA Measurement accuracy with FR1 serving cell for 2 Rx UE	4582

17	NR standalone tests with at least one NR cell in FR2 for RedCap	4587

17.1	RRC_IDLE state mobility	4587

17.1.1	NR cell re-selection	4587

17.1.1.0	Minimum conformance requirements	4587

17.1.1.0.1	Minimum conformance requirements for intra-frequency cell re-selection for RedCap	4587

17.1.1.0.2	Minimum conformance requirements for inter-frequency cell re-selection for RedCap	4589

17.1.1.0.3	Minimum conformance requirements for intra-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	4591

17.1.1.0.4	Minimum conformance requirements for inter-frequency cell re-selection for RedCap UE configured with stationary relaxed measurement criterion	4593

17.1.1.1	NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx	4595

17.1.1.2	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for 2 Rx	4601

17.1.1.3	NR SA FR2 Cell reselection to FR2 intra-frequency NR for UE fulfilling stationary relaxed measurement criterion for 2 Rx UE	4607

17.1.1.4	NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE	4612

17.2	RRC_INACTIVE state mobility for RedCap	4619

17.2.1	Configured Grant based Small Data Transmissions for RedCap	4619

17.2.1.0	Minimum conformance requirements	4619

17.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT	4619

17.2.1.1	NR SA FR2 TA validation for CG-SDT for 2 Rx	4620

17.3	RRC_CONNECTED state mobility for RedCap	4623

17.3.1	Handover for RedCap	4623

17.3.1.0	Minimum conformance requirements	4623

17.3.1.0.1	Minimum conformance requirements for NR FR2 – NR FR2 handover	4623

17.3.1.1	NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	4625

17.3.1.2	NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx	4627

17.3.2	RRC connection mobility control for RedCap	4634

17.3.2.1	RRC re-establishment for RedCap	4634

17.3.2.1.0	Minimum conformance requirements	4634

17.3.2.1.1	NR SA FR2 Intra-frequency RRC Re-establishment in FR2	4634

17.3.2.1.2	NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2	4636

17.3.2.1.3	NR SA FR2 RRC re-establishment without serving cell timing	4638

17.3.2.2	Random access for RedCap	4641

17.3.2.2.0	Minimum conformance requirements	4641

17.3.2.2.0.1	Minimum conformance requirements for Contention based random access	4641

17.3.2.2.0.2	Minimum conformance requirements for Non-Contention based random access	4641

17.3.2.2.1	NR SA FR2 4-step RA type contention based random access test in FR2 for NR Standalone	4642

17.3.2.2.2	NR SA FR2 4-step RA type non-contention based random access test in FR2 for NR Standalone	4645

17.3.2.2.3	NR SA FR2 2-step RA type contention based random access test in FR2 for NR Standalone	4648

17.3.2.2.4	NR SA FR2 2-step RA type non-contention based random access test in FR2 for NR Standalone	4651

17.3.2.3	RRC Connection Release with Redirection for RedCap	4654

17.3.2.3.0	Minimum conformance requirements	4654

17.3.2.3.0.1	Minimum conformance requirements for FR2-FR2 RRC connection release with redirection	4654

17.3.2.3.1	NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2	4654

17.4	Timing for RedCap	4656

17.4.1	UE transmit timing for RedCap	4656

17.4.1.0	Minimum Conformance Requirements	4657

17.4.1.0.1	Minimum conformance requirements for UE transmit timing accuracy	4657

17.4.1.1	NR SA FR2 NR UE Transmit Timing Test for FR2	4658

17.4.2	UE timer accuracy for RedCap	4661

17.4.3	Timing advance for RedCap	4661

17.4.3.0	Minimum conformance requirements	4661

17.4.3.0.1	Minimum conformance requirements for timing advance adjustment accuracy	4661

17.5	Signalling characteristics for RedCap	4665

17.5.1	Radio link monitoring for RedCap	4665

17.5.1.0	Minimum requirements	4665

17.5.1.0.1	General	4665

17.5.1.0.2	Minimum conformance requirements for SSB-based RLM	4668

17.5.1.0.3	Minimum conformance requirements for CSI-RS based RLM	4670

17.5.1.1	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	4672

17.5.1.2	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	4675

17.5.1.3	NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	4680

17.5.1.4	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	4683

17.5.1.5	NR SA FR2 Radio Link Monitoring Out-of-sync Test for PCell configured with CSI-RS-based RLM RS in non-DRX mode	4686

17.5.1.6	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	4689

17.5.1.8	NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	4697

17.5.1.9	NR SA FR2 radio link monitoring UE scheduling restrictions	4701

17.5.2	Beam Failure Detection and Link recovery procedures for RedCap	4704

17.5.2.0	Minimum conformance requirements	4704

17.5.2.0.1	Minimum conformance requirements for SSB-based BFD and link recovery procedures	4704

17.5.2.0.2	Minimum conformance requirements for CSI-RS-based BFD and link recovery procedures	4704

17.5.2.0.3	Scheduling availability of UE during beam failure detection and candidate beam detection	4704

17.5.2.1	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	4704

17.5.2.2	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	4709

17.5.2.3	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	4714

17.5.2.4	NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	4718

17.5.2.5	NR SA FR2 Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	4722

17.5.3	Active BWP switch delay for RedCap	4726

17.5.3.1	DCI-based and time-based active BWP switch for RedCap	4726

17.5.3.1.0	Minimum conformance requirements for DCI-based and time-based active BWP switch	4726

17.5.3.1.1	NR SA FR2 DCI-based and Timer-based DL active BWP switch with non-DRX	4727

17.5.3.2	RRC-based active BWP switch for RedCap	4734

17.5.3.2.0	Minimum conformance requirements for RRC-based active BWP switch	4734

17.5.3.2.1	NR SA FR2 RRC-based DL active BWP switch with non-DRX	4734

17.5.4	Active TCI state switch delay for RedCap	4740

17.5.4.0	Minimum conformance requirements	4740

17.5.4.0.1	Known conditions for TCI state	4740

17.5.4.0.2	Minimum conformance requirements for MAC-CE based active TCI state switch	4740

17.5.4.0.3	Minimum conformance requirements for RRC based active TCI state switch	4740

17.5.4.1	MAC-CE based active TCI state switch for RedCap	4740

17.5.4.2	RRC based active TCI state switch for RedCap	4743

17.5.4.2.1	NR SA FR2 NR PCell FR2 active TCI state switch for a known TCI state	4743

17.5.5	Uplink spatial relation switch delay for RedCap	4746

17.5.5.1	MAC-CE based Spatial Relation switch for RedCap	4746

17.5.5.1.0	Minimum conformance requirements MAC-CE based uplink spatial relation switch delay	4746

17.5.5.1.1	NR SA FR2 PCell MAC-CE based spatial relation switch associated with known DL-RS	4747

17.5.5.2	RRC based spatial relation switch for RedCap	4750

17.5.5.2.0	Minimum conformance requirements RRC based uplink spatial relation switch delay	4750

17.5.5.2.1	NR SA FR2 PCell RRC-based spatial relation switch associated with a known DL-RS	4751

17.5.6	UE specific CBW change for RedCap	4754

17.5.6.0	Minimum conformance requirements	4754

17.5.6.0.1	Minimum conformance requirements for UE specific CBW change	4754

17.5.6.1	NR SA FR2 UE specific CBW change of PCell with non-DRX	4754

17.6	Measurement procedures for RedCap	4760

17.6.1	Intra-frequency measurements for RedCap	4760

17.6.1.0	Minimum conformance requirements	4760

17.6.1.0.1	General	4760

17.6.1.0.2	Minimum conformance requirements for event-triggered measurement without gap	4761

17.6.1.0.3	Minimum conformance requirements for event-triggered measurement with gap	4763

17.6.1.1	NR SA FR2 Event triggered reporting test without gap under non-DRX	4764

17.6.1.2	NR SA FR2 Event triggered reporting test without gap under DRX	4767

17.6.1.3	NR SA FR2 Event triggered reporting test with per-UE gaps under non-DRX	4769

17.6.1.4	NR SA FR2 event-triggered reporting with gap in DRX	4776

17.6.2	Inter-frequency measurements for RedCap	4779

17.6.2.0	Minimum conformance requirements	4779

17.6.2.1	NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used	4781

17.6.2.2	NR SA FR2-FR2 Event triggered reporting tests For FR2 without SSB time index detection when DRX is not used	4785

17.6.2.3	NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used	4790

17.6.2.4	NR SA FR2-FR2 Event triggered reporting tests For FR2 with SSB time index detection when DRX is not used	4795

17.6.3	L1-RSRP measurement for beam reporting for RedCap	4800

17.6.3.0	Minimum conformance requirements for L1-RSRP measurement for beam reporting	4800

17.6.3.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting	4800

17.6.3.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting	4800

17.6.3.1	NR SA FR2 SSB based L1-RSRP measurement when DRX is not used	4800

17.6.3.2	NR SA FR2 SSB based L1-RSRP measurement when DRX is used	4801

17.6.3.3	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is not used	4804

17.6.3.4	NR SA FR2 CSI-RS based L1-RSRP measurement when DRX is used	4806

17.6.4	NR Measurements with autonomous gaps for RedCap	4809

17.6.4.0	Minimum conformance requirements	4809

17.6.4.0.1	Minimum conformance requirements for CGI identification of an NR cell with autonomous gaps	4809

17.6.4.1	NR SA FR2 Interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	4810

17.7	Measurement performance requirements for RedCap	4816

17.7.1	SS-RSRP for RedCap	4816

17.7.1.0	Minimum conformance requirements	4816

17.7.1.0.1	Intra-frequency SS-RSRP measurement accuracy requirements	4816

17.7.1.0.2	Inter-frequency SS-RSRP measurement accuracy requirements	4816

17.7.1.1	Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4816

17.7.1.2	Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4821

17.7.2	SS-RSRQ for RedCap	4827

17.7.2.0	Minimum conformance requirements	4827

17.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements	4827

17.7.2.0.2	Inter-frequency SS-RSRQ measurement accuracy requirements	4827

17.7.2.1	Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4827

17.7.2.2	Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4830

17.7.3	L1-RSRP measurement for beam reporting for RedCap	4833

17.7.3.0	Minimum conformance requirements	4833

17.7.3.0.1	SSB-based L1-RSRP absolute measurement accuracy requirements	4833

17.7.3.0.2	SSB-based L1-RSRP relative measurement accuracy requirements	4833

17.7.3.0.3	CSI-RS-based L1-RSRP absolute measurement accuracy requirements	4833

17.7.3.0.4	CSI-RS-based L1-RSRP relative measurement accuracy requirements	4834

17.7.3.1	NR SA FR2 SSB based L1-RSRP measurement accuracy	4834

17.7.3.2	NR SA FR2 CSI-RS based L1-RSRP measurement on resource set with repetition off	4838

17.7.4	SS-SINR for RedCap	4842

17.7.4.0	Minimum conformance requirements	4842

17.7.4.0.1	Intra-frequency SS-SINR measurement accuracy requirements	4842

17.7.4.0.2	Inter-frequency SS-SINR measurement accuracy requirements	4842

17.7.4.1	Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	4842

18	E-UTRA - NR inter-RAT with E-UTRA serving cell	4845

18.1	RRC_IDLE state mobility	4845

18.1.1	Inter-rat NR cell re-selection	4845

18.1.1.0	Minimum conformance requirements	4845

18.1.1.1	E-UTRA - NR SA FR1 E-UTRA Cell reselection to higher priority NR target Cell in FR1	4846

18.2	RRC_CONNECTED state mobility for RedCap	4850

18.2.1	Handover for RedCap	4850

18.2.1.0	Minimum conformance requirements	4850

18.2.1.1	E-UTRA - NR FR1 handover  for 2Rx UE	4851

18.2.2.0	Minimum conformance requirements	4854

18.2.2.0.1	Redirection from E-UTRA to FR1 RedCap UE	4854

18.2.2.1	Redirection from E-UTRA to NR SA FR1 for redcap UE	4855

18.3	Measurement procedure for RedCap	4862

18.3.1	E-UTRA - NR Measurements for RedCap	4862

18.3.1.0	Minimum conformance requirements	4862

18.3.1.0.1	Minimum conformance requirements for E-UTRA - NR event-triggered measurement	4862

18.3.1.1	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	4865

18.3.1.2	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	4869

18.3.1.3	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	4873

18.3.1.4	E-UTRA - NR SA FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	4877

18.3.1.5	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	4881

18.3.1.6	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 without SSB time index detection when DRX is used	4883

18.3.1.7	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	4886

18.3.1.8	E-UTRA - NR SA FR2 Event triggered reporting tests for FR2 with SSB time index detection when DRX is used	4888

Annex A (normative): RRM test configurations	4892

A.1	Reference measurement channels	4892

A.1.0	General	4892

A.1.1	PDSCH	4892

A.1.1.1	FDD	4892

A.1.1.2	TDD	4893

A.1.2	CORESET for RMSI scheduling	4896

A.1.2.1	FDD	4896

A.1.2.2	TDD	4897

A.1.3	CORESET for RMC scheduling	4900

A.1.3.1	FDD	4900

A.1.3.2	TDD	4901

A.1.4	CSI-RS	4903

A.1.4.1	FDD	4903

A.1.4.2	TDD	4906

A.1.4A	CSI-RS for tracking	4911

A.1.4A.1	FR1	4911

A.1.4A.1.1	FDD	4911

A.1.4A.1.2	TDD	4912

A.1.4A.2	FR2	4913

A.1.4A.2.1	TDD	4913

A.1.4B	CSI-IM configurations	4914

A.1.4B.1	FDD	4914

A.1.4B.2	TDD	4914

A.1.5	TDD UL/DL configuration	4916

A.1.6	PUSCH	4917

A.1A	Reference measurement channels under CCA	4918

A.1A.0	General	4918

A.1A.1	PDSCH	4918

A.1A.1.1	FDD	4918

A.1A.2	CORESET for RMSI scheduling	4919

A.1A.2.1	TDD	4919

A.1A.3	CORESET for RMC scheduling	4920

A.1A.3.1	TDD	4920

A.1A.4	TDD UL/DL configuration	4921

A.1A.4.1	TDD	4921

A.1A.5	RMC burst transmission model	4921

A.2	Reference OCGN configuration	4921

A.2.1 Generic OFDMA channel noise generator (OCNG)	4921

A.3	Reference SSB configuration	4924

A.3.1	SSB configuration for FR1	4924

A.3.2	SSB configuration for FR2	4924

A.3A	Reference SSB configuration under CCA	4925

A.3A.1	SSB configuration under CCA for FR1	4925

A.3A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	4925

A.3A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	4925

A.3A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	4926

A.3A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	4926

A.3B	Reference SSB configuration for RedCap	4927

A.3B.1	SSB configuration for FR1	4927

A.3B.2	SSB configuration for FR2	4928

A.4	Reference SMTC configuration	4928

A.4A	Reference SMTC configuration for RedCap	4928

A.5	Reference DRX configurations	4929

A.6	EN-DC test setup	4929

A.6.1	E-UTRA serving cell parameters	4929

A.6.1.1	E-UTRA serving cell parameters for EN-DC tests with NR FR1	4929

A.6.1.2	E-UTRA serving cell parameters for EN-DC tests with NR FR2	4931

A.6A	NR FR1-FR2 test setup	4933

A.6B	EN-DC test setup with unlicensed bands	4933

A.6B.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	4933

A.7	Reference PRACH configurations	4934

A.7.1	PRACH configurations for FR1	4934

A.7.1A	PRACH configurations for FR1 under CCA	4935

A.7.1A.1	FR1 PRACH configuration 1 under CCA	4935

A.7.1A.2	FR1 PRACH configuration 2 under CCA	4936

A.7.2	PRACH configurations for FR2	4937

A.7A	Reference MsgA configurations	4938

A.7A.1	MsgA configurations for FR1	4938

A.7A.1A	MsgA configurations for FR1 under CCA	4940

A.7A.1A.1	FR1 MsgA configuration 1 under CCA	4940

A.7A.1A.2	FR1 MsgA configuration 2 under CCA	4941

A.7A.2	MsgA configurations for FR2	4943

A.8	Reference BWP configurations	4945

A.8.1	Downlink BWP configurations	4945

A.8.2	Uplink BWP configurations	4945

A.8A	Reference BWP configurations for RedCap	4946

A.8A.1	Downlink BWP configurations	4946

A.8A.2	Uplink BWP configurations	4946

A.9	Angle of Arrival (AoA) for FR2 RRM test cases	4946

A.9.1	Setup 1: Single AoA in Rx beam peak direction	4947

A.9.2	Setup 2: Single AoA in non Rx beam peak direction	4947

A.9.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	4947

A.9.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	4947

A.9.3	Setup 3: 2 AoAs	4947

A.9.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	4948

A.9.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	4948

A.9.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	4948

A.10	TCI State Configuration	4948

A.10.1	Introduction	4948

A.10.2	TCI states	4948

A.10A	Unified TCI State Configuration	4949

A.10A.1	Introduction	4949

A.10A.2	DLorJoint TCI states	4949

A.10A.3	UL TCI states	4949

A.11	NR sidelink communication	4950

A.11.1	Introduction	4950

A.11.2	Reference resource pool configurations for NR Sidelink Communication	4950

A.11.3	Reference measurement channels for NR Sidelink Communication	4953

A.12	Discovery Burst Transmission Window configuration under CCA	4954

A.12.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	4954

A.13	Channel bandwidth (CBW) configurations	4954

A.13.1	DL UE specific CBW	4954

A.13.2	UL UE specific CBW	4954

A.14	Satellite access configuration	4955

A.14.2	Satellite specific configuration for neighbour cell	4955

A.15	SRS Configurations	4956

A.15.1	SRS Configuration for SCS=15kHz	4956

A.15.2	SRS Configuration for SCS=30kHz	4957

A.15.3	SRS Configuration for SCS=30kHz	4958

Annex B (normative): Conditions for RRM requirements applicability for operating bands	4958

B.1	Conditions for NR RRC_IDLE state mobility	4958

B.1.1	Introduction	4958

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	4959

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	4960

B.1.6	Conditions for measurements on NR intra-frequency cells for cell re-selection for satellite access	4962

B.1.7	Conditions for measurements on NR inter-frequency cells for cell re-selection for satellite access	4963

B.2	Conditions for NR RRC_CONNECTED state	4963

B.2.1	Introduction	4963

B.2.2	Conditions for NR intra-frequency measurements	4963

B.2.3	Conditions for NR inter-frequency measurements	4964

B.2.4	Conditions for NR L1-RSRP reporting	4965

B.2.4.1	Conditions for SSB based L1-RSRP reporting	4965

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	4966

B.2.5	Conditions for RRC connection release with redirection to NR	4967

B.2.6	Conditions for UE transmit timing	4968

B.2.6.1	Conditions for SSB based UE transmit timing	4968

B.2.7	Conditions for NR intra-frequency measurements for RedCap	4969

B.2.8	Conditions for NR inter-frequency measurements for RedCap	4971

B.2.9	Conditions for NR intra-frequency measurements under CCA	4973

B.2.10	Conditions for NR inter-frequency measurements under CCA	4974

B.2.11	Conditions for NR L1-RSRP reporting under CCA	4974

B.2.11.1	Conditions for SSB based L1-RSRP reporting	4974

B.2.12 to B.2.16	4974

B.2.17	Conditions for NR intra-frequency measurements for satellite access	4974

B.2.18	Conditions for NR inter-frequency measurements for satellite access	4975

B.2.19	Conditions for NR L1-RSRP reporting for satellite access	4975

B.2.19.1	Conditions for SSB based L1-RSRP reporting for satellite access	4975

B.2.19.2	Conditions for CSI-RS based L1-RSRP reporting for satellite access	4975

B.2.20	Conditions for RRC connection release with redirection to NR for satellite access	4975

B.3	RRM requirement exceptions	4976

B.3.1	Introduction	4976

B.3.2	Receiver sensitivity relaxation for CA	4976

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	4976

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	4976

B.3.2.2.1	Inter-band carrier aggregation	4976

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	4976

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	4976

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	4977

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	4977

B.3.2.4.1	Intra-band contiguous carrier aggregation	4977

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	4977

B.3.3	Receiver sensitivity relaxation for DC	4977

B.3.4	Receiver sensitivity relaxation for SUL	4977

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	4977

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	4977

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	4977

B.4	Conditions for NR sidelink	4978

B.4.1	Test parameters for GNSS signals	4978

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	4978

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4978

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	4979

B.5	High level test procedure for SAN RRM tests	4979

Annex C (normative): Downlink physical channels and propagation conditions	4980

C.1	Downlink physical channels	4980

C.1.1	General	4980

C.1.2	Default downlink signal levels	4980

C.1.3	Default connection setup	4981

C.2	Propagation conditions	4981

C.2.0	General	4981

C.2.1	No interference	4982

C.2.2	Static propagation conditions	4982

C.2.2.0	General	4982

C.2.2.1	UE receiver with 2Rx antenna connectors	4982

C.2.2.2	UE receiver with 4Rx antenna connectors	4982

C.2.3	Multi-path fading propagation conditions	4983

Annex D (normative): Deviations from standard test configuration	4984

D.1	Test cases with different numerologies	4984

D.2	EN-DC test cases with different EN-DC configurations	4984

D.2.0	General	4984

D.2.1	Principle of testing	4984

D.3	Carrier aggregation test cases with different CA configurations	4984

D.3.0	General	4984

D.3.1	Principle of testing	4984

D.4	Antenna connection for 4Rx capable UEs	4984

D.4.0	General	4984

D.4.1	Principle of testing	4985

D.4.1.1	Single carrier tests	4985

D.4.1.2	Carrier aggregation tests	4986

D.4.1.3	EN-DC tests	4986

D.4.2	Antenna connection	4986

D.4.2.1	Antenna connection for NR bands where 2Rx is supported	4986

D.4.2.2	Antenna connection for NR bands where only 4Rx is supported	4986

D.4.2.3	Antenna connection for E-UTRA bands where 2Rx is supported	4986

D.4.2.4	Antenna connection for E-UTRA bands where only 4Rx is supported	4987

D.4A	Antenna configurations with unlicensed bands for 4Rx capable UEs	4987

D.4A.1	Introduction	4987

D.4A.2	Principle of Testing	4987

D.4A.2.1	Single Carrier Tests	4987

D.4A.2.2	Carrier Aggregation Tests	4988

D.4A.2.3	EN-DC Tests	4988

D.4A.2.4	Antenna connection for bands where 2RX is supported	4988

D.4A.2.5	Antenna connection for bands where 4RX is supported	4988

D.4A.2.6	EN-DC LTE Antenna connection for bands where 2RX is supported	4988

D.4A.2.7	EN-DC LTE Antenna connection for bands where 4RX is supported	4988

D.5	Test Cases with Different Channel Bandwidths	4989

D.5.1	Test Cases with Different E-UTRA Channel Bandwidths	4989

D.5.1.1	Introduction	4989

D.5.1.2	Principle of testing	4989

D.6	Test Cases for Synchronous and Asynchronous DC Operations	4989

D.6.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	4989

D.6.1.1	Introduction	4989

D.6.1.2	Principle of Testing	4989

D.7	CCA model	4989

D.7.1	Introduction	4989

D.7.2	CCA model for operation on a carrier frequency with CCA in FR1	4989

D.7.2.1	DL CCA model	4989

D.7.2.2	UL CCA model	4991

D.7.3	Test Cases with at Least One Cell on a Carrier Frequency with CCA	4991

D.8	Test Cases in SA and EN-DC Operations	4991

D.8.0	General	4991

D.8.1	Principle of testing	4992

Annex E (normative): Cell configuration mapping	4993

E.0	General	4993

E.1	Test frequency selection	4993

E.1.0	General	4993

E.1.1	E-UTRA PCell for EN-DC test cases	4993

E.1.2	Test cases with one NR cell	4993

E.1.3	Test cases with more than one NR cell	4993

E.1.3.1	Intra-frequency test cases	4993

E.1.3.2	Inter-frequency test cases	4993

E.1.4	Carrier aggregation test cases	4994

E.1.4.1	Inter-band carrier aggregation	4994

E.1.4.2	Intra-band contiguous carrier aggregation	4994

E.1.4.3	Intra-band non-contiguous carrier aggregation	4994

E.1.5	E-UTRA – NR inter RAT test cases	4994

E.1.6	Intra-band EN-DC test cases	4994

E.1.6.1	Intra-band non-contiguous EN-DC	4994

E.1.6.1.1	Inter frequency neighbour cell	4994

E.1.6.2	Intra-band contiguous EN-DC	4994

E.1.6.1.1	E-UTRA PCell	4994

E.1.7	NR sidelink test cases	4995

E.2	Cell configuration mapping for EN-DC FR1 test cases in Chapter 4	4995

E.3	Cell configuration mapping for EN-DC FR2 test cases in Chapter 5	4998

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6	5001

E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7	5007

E.6	Cell configuration mapping for E-UTRAN – SA test cases in Chapter 8	5011

E.7	Cell configuration mapping for NR sidelink test cases in Chapter 9	5012

E.8	Cell configuration mapping for EN-DC FR1 Shared Spectrum test cases in Chapter 10	5012

E.9	Cell configuration mapping for SA FR1 Shared Spectrum test cases in Chapter 11	5015

E.10 to E.13	5015

E.14	Cell configuration mapping for SA FR1 test cases for RedCap in Chapter 16	5015

E.15	Cell configuration mapping for SA FR2 test cases for RedCap in Chapter 17	5022

E.16	Cell configuration mapping for E-UTRAN – SA test cases for RedCap in Chapter 18	5024

Annex F (normative): Measurement uncertainties and test tolerances	5025

F.1	Measurement uncertainties and test tolerances for FR1 and FR2	5025

F.1.1	Acceptable uncertainty of test system (normative)	5025

F.1.1.1	Measurement of test environments	5025

F.1.1.2	Measurement of RRM requirements	5025

F.1.2	Interpretation of measurement results (normative)	5087

F.1.3	Test Tolerance and Derivation of Test Requirements (informative)	5087

F.1.3.1	Measurement of test environments	5087

F.1.3.2	Measurement of RRM requirements	5087

Annex G (normative): Statistical testing	5226

G.1	General	5226

G.2	Statistical testing of delay and UE measurement performance in RRM tests	5226

G.2.1	General	5226

G.2.2	Design of the test	5226

G.2.3	Numerical definition of the pass fail limits	5226

G.2.4	Pass fail decision rules	5227

G.2.5	Void	5228

G.2.6	Test conditions for delay tests and UE measurement performance	5228

G.3	Statistical testing of NR sidelink CBR measurement tests	5228

G.3.1	General	5228

G.3.2	Design of the test	5228

G.3.3	Numerical definition of the pass fail limits	5229

G.3.4	Pass fail decision rules	5230

G.4	Theory to derive the numbers in Table G.2.3-1 (informative)	5230

G.5	Theory to derive the numbers in Table G.3.3-1 (informative)	5230

G.5.1	Algorithm to calculate pass and fail probabilities	5230

G.5.2	Method of designing pass/fail limit to approach given risk probability	5234

Annex H (normative): Default message contents for RRM	5238

H.1	Void	5238

H.2	System information blocks message content exceptions	5238

H.2.1	System information blocks message contents exceptions for NR intra frequency cell re-selection	5238

H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection	5240

H.2.3	System information blocks message contents exceptions for NR inter-RAT cell re-selection	5241

H.3	RRC message content exceptions	5242

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration	5242

H.3.2	RRC messages and information elements contents exceptions for handover	5265

H.3.3	RRC messages and information elements contents exceptions for NR inter-RAT handover	5266

H.3.4	E-UTRA RRC messages and information elements contents exceptions	5267

H.3.6	RRC messages and IE content exceptions for L1-RSRP measurement for beam reporting	5278

H.3.6A	RRC messages and IE content exceptions for L1-SINR measurement for beam reporting	5281

H.3.7	RRC messages and information elements contents exceptions for NR cell search when DRX is used	5287

H.3.8	RRC messages and information elements contents exceptions for NR RRC reconfiguration delay	5290

H.3.9	RRC messages and information elements contents exceptions for UL timing	5291

H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access	5291

Annex I (normative): RRM OTA procedures	5293

I.0	Test applicability per permitted test method	5293

I.1	Direct far field (DFF)	5293

I.1.1	RX beam peak direction search	5293

I.1.2	Search for directions corresponding to the EIS spherical coverage percentile	5293

I.2	Direct far field (DFF) simplification	5293

I.2.1	RX beam peak direction search	5293

I.2.2	Search for directions corresponding to the EIS spherical coverage percentile	5294

I.3	Indirect far field (IFF)	5294

I.3.1	RX beam peak direction search	5294

I.3.2	Search for directions corresponding to the EIS spherical coverage percentile	5294

I.3A	Enhanced indirect far field (Enhanced IFF)	5294

I.3A.1	RX beam peak direction search	5294

I.3A.2	Search for directions corresponding to the EIS spherical coverage percentile	5294

I.4	Procedures to search test directions for RRM FR2	5294

I.4.1	RSRPB-based scan with fallback option to Rx beam peak direction search	5294

I.5	Re-Positioning Approach for RRM FR2	5295

I.5.1	1 AoA Test Cases	5296

I.5.2	2 AoA Test Cases	5297

Annex J (informative): Change history	5299