## 6.6	Measurement procedures

### 6.6.1	Intra-frequency measurements

#### 6.6.1.0	Minimum conformance requirements

6.6.1.0.1	Minimum conformance requirements for event-triggered measurement without gap

Same as 4.6.1.0.1

6.6.1.0.2	Minimum conformance requirements for event-triggered measurement with gap

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.2-1.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.1.0.2-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in TS 38.133 [6] section 9.1.5.2.2 for measurement within outside measurement gaps.

Table 6.6.1.0.2-1: Time period for PSS/SSS detection (Frequency range 1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle > 320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

Table 6.6.1.0.2-2: Measurement period for intra-frequency measurements with gaps (Frequency Range 1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max( 200ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 200ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] Sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_without_index defined in TS 38.133 [6] section 9.2.6.2. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.6.2 and 9.2.6.3.

6.6.1.0.3	Void

6.6.1.0.4	Minimum conformance requirements for event-triggered measurement with gap with SSB index reading

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_with_index.

Tidentify_intra_with_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.1.0.4-1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured given in table 6.6.1.0.4-2.

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.1.0.4-3.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap, i in TS 38.133 [6] section 9.1.5.2.2 for measurement conducted within measurement gaps.

When intrafrequency SMTC is fully non-overlapping with measurement gaps or intrafrequency SMTC is fully overlapping with MGs, Kp=1

When intrafrequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP.

Table 6.6.1.0.4-1: Time period for PSS/SSS detection (Frequency range 1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

Table 6.6.1.0.4-2: Time period for time index detection (Frequency range 1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 6.6.1.0.4-3: Measurement period for intra-frequency measurements with gaps (Frequency Range 1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max( 200ms, 5 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max( 200ms, ceil(1.5x 5) x max(MGRP, SMTC period, DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(MGRP, DRX cycle) x CSSFintra |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.2]

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_with_index defined in TS 38.133 [6] section 9.2.6.2. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.6.2 and 9.2.6.3.

6.6.1.0.5	Void

6.6.1.0.6	Minimum conformance requirements for event-triggered measurement with MUSIM gap configured

[TS 38.133 [6], clause 9.1.10]

If the UE requires gap patterns for MUSIM purpose, the network may provide one or more per-UE MUSIM gap pattern(s) for concurrent monitoring of all frequency layers for MUSIM via MUSIM-GapConfig [13]. The UE can be configured with no more than three periodic MUSIM gap patterns and/or one aperiodic MUSIM gap pattern for MUSIM via MUSIM-GapConfig [13]. The MUSIM gap patterns specified in Table 6.6.1.0.6-1 are applicable only for MUSIM operation.

The UE is not required to perform cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network that is outside the MUSIM gaps.

The UE is not required to conduct reception or transmission from or to the [source] network during MUSIM gaps that are not dropped due to collisions, as defined in TS 38.133 [6] clauses 9.1.10.4 and 9.1.10.5.

UE supporting MUSIM capability shall support the MUSIM gap patterns listed in Table 6.6.1.0.6-1 based on UE’s capability specified in TS 38.306[11] and the applicability specified in Table 6.6.1.0.6-2.

UE determines MUSIM gap timing based on gap offset configuration from serving cell provided by higher layer signalling as specified in TS 38.331 [13].

Table 6.6.1.0.6-1: MUSIM Gap Pattern Configurations

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 10 | 80 |
| 13 | 20 | 160 |
| 14 | 6 | 320 |
| 15 | 6 | 640 |
| 16 | 6 | 1280 |
| 17 | 6 | 2560 |
| 18 | 10 | 320 |
| 19 | 10 | 640 |
| 20 | 10 | 1280 |
| 21 | 10 | 2560 |
| 22 | 20 | 320 |
| 23 | 20 | 640 |
| 24 | 20 | 1280 |
| 25 | 20 | 2560 |
| 26 | 20 | 5120 |
| 27 | 10 | NA |
| 28 | 20 | NA |
| NOTE 1: Measurement gap pattern #27, #28 are the aperiodic gap pattern without MGRP. |  |  |

Table 6.6.1.0.6-2: Applicability for MUSIM Gap Pattern Configurations supported by the UE with NR standalone operation (with single carrier, NR CA configuration)

| MUSIM gap pattern configuration | Serving cell | Gap Purpose | Applicable MUSIM Gap Pattern Id |
| --- | --- | --- | --- |
| Per-UE | FR1, FR2, or | MUSIM Note1 | 0-13, 14-26, 27, 28 |
| MUSIM gap | FR1 + FR2 |  |  |
| NOTE 1: Inclusion of MUSIM procedures for per-UE MUSIM gaps only in NR single carrier, NR CA mode: MUSIM purpose which includes cell identification and measurement, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell in the target network. |  |  |  |

[TS 38.133 [6], clause 9.1.10.2]

Priority levels are applied for each periodic MUSIM gap. A UE shall request a priority for all requested periodic MUSIM gaps when the UE requests MUSIM gaps via MUSIM-GapConfig-r17 [13]. The UE shall request different priority level for each periodic MUSIM gaps. The network may assign priority to each periodic MUSIM gaps. The allocated priorities may differ from the priorities requested by the UE. The UE MUSIM requirements apply if the configured MUSIM gap priorities retain the same relative priorities among MUSIM gaps as requested by the UE.

The requirements in TS 38.133 [6] clause 9.1.10 apply provided different priority levels are allocated to each periodic MUSIM gaps and different priority levels are allocated to each periodic MUSIM gap and each measurement gap configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17.

An aperiodic MUSIM gap, when configured, is unconditionally kept in case of collisions with any other gap occasions, including MUSIM gaps and measurement gaps.

[TS 38.133 [6], clause 9.1.10.3]

The UE can request use of “keep solution” via musim-GapKeepPreference in [13]. Keep solution is for handling collisions among different MUSIM gaps. If the use of “keep solution” is granted, the UE shall keep all colliding periodic and aperiodic MUSIM gaps irrespectively of the priority of the periodic MUSIM gaps.

[TS 38.133 [6], clause 9.1.10.4]

MUSIM gap occasions are considered colliding if at least one of the following conditions is met:

-	the MUSIM gap occasions are fully overlapping in time domain, or

-	the MUSIM gap occasions partially overlapping in time domain, or

-	the distance between the two MUSIM gap occasions is equal to or smaller than 4ms.

The distance between two MUSIM gap occasions is defined as the time difference between the ending point of the first occasion and the starting point of the second occasion, where the first MUSIM gap occasion occurs earlier in time than the second MUSIM gap occasion.

When “keep solution” in TS 38.133 [6] clause 9.1.10.3 is not used, a periodic MUSIM gap occasion colliding with an aperiodic MUSIM gap is dropped.

When “keep solution” in TS 38.133 [6], clause 9.1.10.3 is not used, collisions between periodic MUSIM gap occasions are resolved based on the assigned MUSIM gap priorities. Collisions are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority. For each collision, the occasion of the MUSIM gap with highest priority shall be kept and the occasion of the MUSIM gap with lower priority shall be dropped.

[TS 38.133 [6], clause 9.1.10.5]

MUSIM gap and measurement gap occasions are considered colliding if at least one of the following conditions is met:

-	the MUSIM gap and measurement gap occasions are fully overlapping in time domain, or

-	the MUSIM gap and measurement gap occasions are partially overlapping in time domain, or

-	the distance between any of the MUSM gap and the measurement gap occasions is equal to or smaller than 4ms.

The distance between two gap occasions is defined as the time difference between the ending point of the first occasion and the starting point of the second occasion, where the first gap occasion occurs earlier in time than the second gap occasion. The gap occasion can be either a MUSIM gap occasion or a measurement gap occasion.

The measurement gap occasion colliding with an aperiodic MUSIM gap is dropped.

Collisions between MUSIM gaps and measurement gaps configured via GapConfig-r17 with assigned priority but without preConfigInd-r17 or ncsgInd-r17 are handled based on their assigned priorities. Collisions are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority. For each collision, the occasion of the MUSIM gap or measurement gap with highest priority shall be kept and the occasion of the MUSIM gap or measurement gap with lower priority shall be dropped.

Collisions between MUSIM gaps and measurement gaps configured via GapConfig or configured via GapConfig-r17 without assigned priority are handled based on MGRP of the colliding gaps. Collisions are resolved sequentially in order of decreasing MGRP, starting with the gap that has the longest MGRP. For each collision, the occasion of the MUSIM gap or measurement gap with longer MGRP shall be kept and the occasion of the MUSIM gap or measurement gap with shorter MGRP shall be dropped. If the colliding MUSIM gap and measurement gap have the same MGRP, the requirements in clause 9 shall not apply.

[TS 38.133 [6], clause 9.1.10.6]

A slot is interrupted by a MUSIM gap if it overlaps with any occasion of the configured MUSIM gap, except for a dropped MUSIM gap occasion.

#### 6.6.1.1	NR SA FR1 event-triggered reporting without gap in non-DRX

6.6.1.1.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under non-DRX.

6.6.1.1.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.6.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.1.

6.6.1.1.4	Test description

6.6.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.1.4.1-1.

Table 6.6.1.1.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.1-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.1-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.1-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.1.1.4.1-2.

Table 6.6.1.1.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.1.4.1-3 below.

2. Message contents are defined in clause 6.6.1.1.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.1.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under non-DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.1.1.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.1.4.1-3 and Table 6.6.1.1.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.1.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.1.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
Or
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.1.4.3-1: Common Exception messages SA intra-frequency event triggered reporting tests without gap under non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for configuration 6.6.1.1-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.1-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM |

6.6.1.1.5	Test requirement

Table 6.6.1.1.4.1-3 and Table 6.6.1.1.5-1 define the primary level settings including test tolerances for NR SA event triggered reporting test without gap under non-DRX.

Table 6.6.1.1.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under non-DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intr= max [ 600 ms, ceil (5 × Kp) × SMTC period ] × CSSFintra = 600 ms

T SSB_measurement_period_intra= max [ 200 ms, ceil( 5 × Kp) × SMTC period ] × CSSFintra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 802 ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.2	NR SA FR1 event-triggered reporting without gap in DRX

6.6.1.2.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under DRX.

6.6.1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle. Test 1 can be skipped if test case 6.6.1.13 is executed.

6.6.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.2.

6.6.1.2.4	Test description

6.6.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.2.4.1-1.

Table 6.6.1.2.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.2-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.2-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.2-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.2.4.1-2.

Table 6.6.1.2.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.2.4.1-3 below.

2. Message contents are defined in clause 6.6.1.2.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.2.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX | ms | 1, 2, 3 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1 |
|  |  | 2 | 3 us |  | Synchronous cells |
|  |  | 3 | 3 us |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

6.6.1.2.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.2.4.1-3 and Table 6.6.1.2.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.2.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.2.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 6402 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.1.2.4.1-1 as appropriate.

6.6.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.2.4.3-1: Common Exception messages for SA intra-frequency event triggered reporting tests without gap under DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO SSB.1 FR1, SMTC.2 for configuration 6.6.1.2-1 Table H.3.1-3 with Condition INTRA-FREQ MO SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.2-2 Table H.3.1-3 with Condition INTRA-FREQ MO SSB.2 FR1, SMTC .1 and synchronous cells for configuration 6.6.1.2-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.1 and Offset for test 1 Table H.3.7-1 with Condition DRX.7 and Offset for test 2 |

6.6.1.2.5	Test requirement

Table 6.6.1.2.4.1-3 and Table 6.6.1.2.5-1 define the primary level settings including test tolerances for NR event triggered reporting in synchronous cells when DRX is used test.

Table 6.6.1.2.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

In Test 1 when DRX cycle length = 40 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report on PUSCH.

In Test 2 when DRX cycle length = 640 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to NR Cell 2 on PUSCH.

For both tests:

The overall delays measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report to NR Cell 2.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delay measured when DRX cycle length is 40 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = max[600ms, ceil(1.5 × 5 x Kp) × max(SMTC period, DRX cycle)] × CSSFintra= 600ms

T SSB_measurement_period_intra = max[200ms, ceil(1.5 × 5 × Kp) x max(SMTC period, DRX cycle)] × CSSFintra= 320ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 40 ms shall be less than a total of 922 ms.

The overall delay measured when DRX cycle length is 640 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

T SSB_measurement_period_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of 6402 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.3	NR SA FR1 event-triggered reporting with gap in non-DRX

6.6.1.3.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under non-DRX within intra-frequency cell search with gaps requirements.

6.6.1.3.2	Test applicability

This test applies to all types of NR UE release 15 onwards supporting 5GS NR SA FR1, CSI-RS-based RLM and BWP operation without bandwidth restriction.

6.6.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.3.

6.6.1.3.4	Test description

6.6.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.3.4.1-1.

Table 6.6.1.3.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in non-DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.3-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.3-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.3-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.3.4.1-2.

Table 6.6.1.3.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.3.4.1-3.

2. Message contents are defined in clause 6.6.1.3.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.3.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.1.3.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.3.4.1-3 and Table 6.6.1.3.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.3.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.3.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.3.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for Configuration 6.6.1.3-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and Synchronous cells for Configuration 6.6.1.3-2 Table H.3.1-3 with ConditionINTRA-FREQ MO, SSB.2 FR1, SMTC.1 and Synchronous cells for Configuration 6.6.1.3-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.3.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.3.5	Test requirement

Table 6.6.1.3.4.1-3 and Table 6.6.1.3.5-1 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in non-DRX test.

Table 6.6.1.3.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.2 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 802 ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.4	NR SA FR1 event-triggered reporting with gap in DRX

6.6.1.4.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event with gaps under DRX within intra-frequency cell search with gaps requirements.

6.6.1.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, CSI-RS-based RLM, BWP operation without bandwidth restriction and long DRX cycle.

6.6.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.4.

6.6.1.4.4	Test description

6.6.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.4.4.1-1.

Table 6.6.1.4.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in DRX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.4-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.4-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.4-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.4.4.1-2.

Table 6.6.1.4.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.4.4.1-3.

2. Message contents are defined in clause 6.6.1.4.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.4.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX | ms | 1, 2, 3 | DRX.1 | DRX.7 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 5 | 10 |  |

6.6.1.4.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.4.4.1-3 and Table 6.6.1.4.4.2-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

In Test 1 when DRX cycle = 40 ms is used, UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Timer Alignment Timer to keep the UE uplink time alignment. Furthermore, the UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.4.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.4.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms for Test 1 or less than 6402 ms for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.1.4.4.1-3 as appropriate.

6.6.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.4.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for configuration 6.6.1.4-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.4-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.4-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.7-1 with Condition DRX.1 and Gap for test 1 Table H.3.7-1 with Condition DRX.7 and Gap for test 2 Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.4.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.4.5	Test requirement

Table 6.6.1.4.4.1-3 and Table 6.6.1.4.5-1 define the primary level settings including test tolerances for NR SA FR1 event-triggered reporting with gap in DRX test.

Table 6.6.1.4.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.2 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

TPSS/SSS_sync_intra = 600 ms for Test 1, and TPSS/SSS_sync_intra =3200 ms for Test 2

TSSB_measurement_period_intra = 320 ms for Test 1, and TSSB_measurement_period_intra = 3200 ms for Test 2

TTI insertion uncertainty = 2 ms

For Test 1, the overall delays measured shall be less than a total of 922 ms (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For Test 2, the overall delays measured shall be less than a total of 6402 ms (note: this gives a total of 6400 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.5	NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

6.6.1.5.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event within intra-frequency cell search without gaps requirements.

6.6.1.5.2	Test applicability

This test applies to all types of NR UE release 15 onwards.

6.6.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.5.

6.6.1.5.4	Test description

6.6.1.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.5.4.1-1.

Table 6.6.1.5.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Configuration | Description |
| --- | --- |
| 6.6.1.5-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.5.4.1-2.

Table 6.6.1.5.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.5.4.1-3.

2. Message contents are defined in clause 6.6.1.5.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.5.4.1-3: General test parameters for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

6.6.1.5.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.5.4.1-3 and Table 6.6.1.5.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.5.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.5.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.5.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for Configuration 6.6.1.5-1 Table H.3.1-4 with SSB Index and A3-offset = -4.5 dB Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-7 with Condition INTRA-FREQ and SSB Index Table H.3.1-8 with Condition SSB RLM |

6.6.1.5.5	Test requirement

Table 6.6.1.5.4.1-3 and Table 6.6.1.5.5-1 define the primary level settings including test tolerances for SA event triggered reporting without gap under non-DRX with SSB index reading test.

Table 6.6.1.5.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting without gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_with_index

Tidentify_intra_with_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_time_index_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_time_index_intra = 120 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 922 ms in this test case (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.6	NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

6.6.1.6.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event within intra-frequency cell search with gaps requirements.

6.6.1.6.2	Test applicability

This test applies to all types of NR UE release 15 onwards supporting 5GS NR SA FR1, CSI-RS-based RLM and BWP operation without bandwidth restriction.

6.6.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.6.

6.6.1.6.4	Test description

6.6.1.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.6.4.1-1.

Table 6.6.1.6.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Configuration | Description |
| --- | --- |
| 6.6.1.6-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.3.4.1-2.

Table 6.6.1.6.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.1.6.4.1-3.

2. Message contents are defined in clause 6.6.1.6.4.3.

3. There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.6.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1 | Cell 1 |  |
| Neighbour cell |  | 1 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1 | 40 |  |
| Measurement gap length | ms | 1 | 6 |  |
| Measurement gap offset | ms | 1 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
| A3-Offset | dB | 1 | -4.5 |  |
| CP length |  | 1 | Normal |  |
| Hysteresis | dB | 1 | 0 |  |
| Time To Trigger | s | 1 | 0 |  |
| Filter coefficient |  | 1 | 0 | L3 filtering is not used |
| DRX | ms | 1 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| T1 | s | 1 | 5 |  |
| T2 | s | 1 | 5 |  |

6.6.1.6.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.6.4.1-3 and Table 6.6.1.6.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.6.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.3.5-1.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.6.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ and GAP NEEDED Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for Configuration 6.6.1.6-1 Table H.3.1-4 with SSB index and A3-offset = -4.5dB. Table H.3.1-5 with Condition INTRA-FREQ Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTRA-FREQ and SSB index Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.1.6.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

6.6.1.6.5	Test requirement

Table 6.6.1.6.4.1-3 and Table 6.6.1.6.5-1 define the primary level settings including test tolerances for SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading test.

Table 6.6.1.6.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with gap in non-DRX with SSB index reading

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
| Initial BWP configuration |  | 1 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1 | CSI-RS |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS | 1 | -94 | -94 | -Infinity | -94 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
| Propagation Condition |  | 1 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_with_index

Tidentify_intra_with_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_time_index_intra = 120 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 922 ms in this test case (note: this gives a total of 920 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.7	NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

6.6.1.7.1	Test purpose

To verify the UE’s ability to make a correct reporting of an event within intra-frequency cell search without gap under DRX for UE configured with highSpeedMeasFlag-r16.

6.6.1.7.2	Test applicability

This test applies to all types of NR UE release 15 onwards that supports measurement enhancements in high speed scenario.

6.6.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.7.

6.6.1.7.4	Test description

6.6.1.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.7.4.1-1.

Table 6.6.1.7.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 6.6.1.7-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.1.7-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.1.7-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.7.4.1-2.

Table 6.6.1.7.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.7.4.1-3 below.

2. Message contents are defined in clause 6.6.1.7.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.7.4.1-3: General test parameters for SA intra-frequency event triggered reporting tests without gap for FR1 under DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasFlag-r16 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.7 | 640ms DRX cycle |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 6 |  |

6.6.1.7.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.7.4.1-3 and Table 6.6.1.7.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.7.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.7.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5122 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.7.4.3-1: Common Exception messages for SA intra-frequency event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasFlag-r16

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.7 and Offset |
| Specific message contents exceptions for Test Configuration 6.6.1.7-1 | Table H.3.1-3 with Condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.7-2 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.7-3 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.2 FR1 |

6.6.1.7.5	Test requirement

Table 6.6.1.7.4.1-3 and Table 6.6.1.7.5-1 define the primary level settings including test tolerances for NR event triggered reporting in synchronous cells when DRX is used

Table 6.6.1.7.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap under DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94+TT | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25T | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2 | AWGN |  | AWGN 1944Hz Note 4 |  |
|  |  | 3 | AWGN |  | AWGN 3334Hz Note 5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2 to the moment the UE send one Event A3 triggered measurement report to NR Cell 2. When DRX cycle length = 640 ms, the overall delay measured is defined as the time from the beginning of time period T2 to the moment the UE starts to send preambles on the PRACH for Scheduling Request (SR) to obtain allocation to send the measurement report to NR Cell 2 on PUSCH.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delay measured when DRX cycle length is 640 ms test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

TPSS/SSS_sync_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200ms

T SSB_measurement_period_intra = ceil(Y × Kp) × DRX cycle × CSSFintra = 1920ms

TTI insertion uncertainty = 2 ms

The overall delay measured when DRX cycle length is 640 ms shall be less than a total of 5122 ms.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.8	NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

6.6.1.8.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasCA-Scell-r17. This test will partly verify the intra-frequency cell search requirements in TS 38.133 [6] clauses 9.2.5.1 and 9.2.5.2.

6.6.1.8.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward supporting enhanced NR CA measurement requirements in high-speed scenario.

6.6.1.8.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.1.0.1

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.8.

6.6.1.8.4	Test description

6.6.1.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.8.4.1-1.

Table 6.6.1.8.4.1-1: Supported PCell test configurations for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Configuration | Description |
| --- | --- |
| 6.6.1.8-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.1.8-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.1.8-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table 6.6.1.8.4.1-1A: Supported SCell test configurations for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| ConfigurationSCell | Description |
| --- | --- |
| 6.6.1.8-1A | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.1.8-2A | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.1.8-3A | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. Note 3: NR Cell 3 has the same SCS, BW and duplex mode as NR Cell 2. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.8.4.1-2.

Table 6.6.1.8.4.1-2: Initial conditions for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.1.8.4.1-3.

2.	Message contents are defined in clause 6.6.1.8.4.3.

3. There is two carriers and three cells specified in the test. NR Cell 1 is the PCell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test. NR Cell 2 is is a FR1 deactivated SCell and NR Cell 3 is a neighbour cell on the same frequency as the SCell.

Table 6.6.1.8.4.1-3: General test parameters for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasCA-Scell-r17 |  | 1,2,3 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 2: Cell 2 and Cell 3 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.6 |  |
| measCycleSCell |  | 1, 2, 3 | 640ms |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

6.6.1.8.4.2	Test procedure

Same test procedure as in 6.6.1.7.4.2 except following step 6:

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 5762 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

6.6.1.8.4.3	Message contents

Table 6.6.1.8.4.3-1: Common Exception messages for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.6 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-1 | Table H.3.1-3 with Condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-2 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.1 FR1 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-3 | Table H.3.1-3 with Condition INTRA-FREQ MO and synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 Table 7.3.1-3a in TS 38.508-1 [14] with condition SSB.2 FR1 |

Table 6.6.1.8.4.3-2: ReportConfigNR-DEFAULT (a6-offset): NR report configuration for event A6

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition EVENT_A6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA6 SEQUENCE { |  |  |  |
| a6-Offset CHOICE { |  |  |  |
| rsrp | -9 | The actual value is field value * 0.5 dB. |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.1.8.4.3-3: ServingCellConfigCommon

| Derivation Path: TS 38.508-1[14], Table 4.6.3-168 with condition R17 HST FR1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| HighSpeedConfig-v1700 SEQUENCE { |  |  |  |
| highSpeedMeasCA-Scell-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.1.8.4.3-4: MeasObjectNR-DEFAULT: measCycleSCell configuration for SCell intra-frequency measurements

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| measCycleSCell-v1530 | sf640 |  |  |
| } |  |  |  |

6.6.1.8.5	Test requirement

Table 6.6.1.8.5-1 defines the primary level settings including test tolerances for EN-DC FR1 interruptions at SRS carrier based switching in asynchronous EN-DC.

Table 6.6.1.8.5-1: NR cell specific test parameters for NR SA FR1 event triggered reporting without gap in DRX for UE configured with highSpeedMeasCA-Scell-r17

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TDD configuration |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| EsIot | dB | 1 | 4 | 4 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1,2 | AWGN |  |  |  | AWGN 1944Hz Note 4 |  |
|  |  | 3 | AWGN |  |  |  | AWGN 3334Hz Note 5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |  |

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than 5760 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.1.9	NR SA FR1 event triggered reporting with MUSIM gap configured

6.6.1.9.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event on an intra-frequency layer based on measurement performed without measurement gaps when the UE is also configured with MUSIM gaps. This test will partly verify the intra-frequency cell search requirements in TS 38.133 [6] clauses 9.2.5.1 and 9.2.5.2.

6.6.1.9.2	Test applicability

This test applies to all types of NR UE release 18 and forward, supporting providing MUSIM assistance information with MUSIM gap preference and related MUSIM gap configuration.

6.6.1.9.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.1.0.1 and 6.6.1.0.6.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.9.

6.6.1.9.4	Test description

6.6.1.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.9.4.1-1.

Table 6.6.1.9.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with MUSIM gap configured

| Configuration | Description |
| --- | --- |
| 6.6.1.9-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.1.9-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.1.9-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.9.4.1-2.

Table 6.6.1.9.4.1-2: Initial conditions for NR SA FR1 event triggered reporting with MUSIM gap configured

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.9.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.1.9.4.1-3.

2.	Message contents are defined in clause 6.6.1.9.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.9.4.1-3: General test parameters for SA intra-frequency event triggered reporting with MUSIM gap configured

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| MUSIM gap Pattern ID |  | 1,2,3 | 0 |  |
| MUSIM gap offset | ms | 1,2,3 | 0 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.1.9.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.9.4.1-3 and Table 6.6.1.9.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.9.5-1.

3. SS shall transmit an RRCReconfiguration message with measConfig and musim-GapConfig-r17.

4. The UE shall transmit an RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.9.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
Or
- if the device has been switched off, switch on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.9.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:Table 6.6.1.9.4.3-1: Common Exception messages SA intra-frequency event triggered reporting with MUSIM gap configured

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM |
| Specific message contents exceptions for Test Configuration 6.6.1.9-1 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.6.1.9-2 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.6.1.9-3 | Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1 |

Table 6.6.1.9.4.3-2: RRCReconfiguration: NR measurement Configuration (Step 3, Test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT specified in Table H.3.1-2 with Condition INTRA-FREQ | Measurement configuration |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-GapConfig-r17 CHOICE { |  |  |  |
| setup | MUSIM-GapConfig | MUSIM gap configuration Table 6.6.9.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.1.9.4.3-3: MUSIMGapConfig (Table 6.6.1.9.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-82BA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | not present |  |  |
| musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17 { | 1 entry |  |  |
| MUSIM-Gap-r17 SEQUENCE { |  | Entry 1 |  |
| musim-GapId-r17 | 0 |  |  |
| musim-GapInfo-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 | not present |  |  |
| musim-GapLength-r17 | ms6 | Pattern #0 |  |
| musim-GapRepetitionAndOffset-r17 CHOICE { |  | Pattern #0 |  |
| ms40 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| musim-AperiodicGap-r17 | not present |  |  |
| } |  |  |  |

6.6.1.9.5	Test requirement

Table 6.6.1.9.4.1-3 and Table 6.6.1.9.5-1 define the primary level settings including test tolerances for NR SA event triggered reporting test with MUSIM gap configured.

Table 6.6.1.9.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting with MUSIM gap configured

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | Specified in Cell 1 columns |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra)ms

TPSS/SSS_sync_intra= max [ 600 ms, ceil (5 × Kp) × SMTC period ] × CSSFintra= 600 ms

TSSB_measurement_period_intra = max [ 200 ms, ceil( 5 × Kp) × SMTC period ] × CSSFintra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 802 ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.10 to 6.6.1.12

#### 6.6.1.12	NR SA FR1 event triggered reporting with SSB index detection without measurement gap in non-DRX mode for UE operating with 12 PRB SSB bandwidth

6.6.1.12.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event when the intra-frequency target cell is transmitting 12 PRB SSB. This test will partly verify the FDD intra-frequency cell search requirements in TS 38.133 [6] clauses 9.2.5.1 and 9.2.5.2.

6.6.1.12.2	Test applicability

This test applies to all types of NR UE Release 18 supporting less than 5 MHz bandwidth.

6.6.1.12.3	Minimum conformance requirements

The minimum conformance requirements are specified in TS 38.133 [6] clauses 9.2.5.1 and 9.2.5.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.12.

6.6.1.12.4	Test description

6.6.1.12.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.12.4.1-1.

Table 6.6.1.12.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting with SSB index reading for 12 PRB bandwidth

| Configuration | Description |
| --- | --- |
| 6.6.1.12-1 | 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.12.4.1-2.

Table 6.6.1.12.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting with SSB index reading for 12 PRB bandwidth

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.12.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.1.5.4.1-3 with test configuration 1, except those specified in Table 6.6.1.12.4.1-3.

2.	Message contents are defined in clause 6.6.1.12.4.3.

3.	There is one NR carrier and two cells specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.1.12.4.1-3: General test parameters for NR SA FR1 event-triggered reporting with SSB index reading for 12 PRB bandwidth

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| SSB configuration |  | 1 | SSB.13 FR1 |  |

6.6.1.12.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.5.4.1-3 config 1 with the exceptions specified in Table 6.6.1.12.4.1-3 and Table 6.6.1.5.5-1 config 1 with the exceptions specified in Table 6.6.1.12.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively.

During time duration T1, the UE shall not have any timing information of Cell 2.

The test steps are same as 6.6.1.5.4.2 except these steps:

2.	Set the parameters according to T1 in Table 6.6.1.5.5-1 with the exceptions specified in Table 6.6.1.12.5-1.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.5.5-1 with the exceptions specified in Table 6.6.1.12.5-1.

6.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 942 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

6.6.1.12.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.12.4.3-1: Common Exception messages for NR SA FR1 event-triggered reporting with SSB index reading for 12 PRB bandwidth

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, [SSB.13 FR1], SMTC.2 Table H.3.1-4 with SSB Index and A3-offset = -4.5 dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ and SSB Index Table H.3.1-8 with Condition SSB RLM |

6.6.1.12.5	Test requirement

Table 6.6.1.5.4.1-3 config 1 with the exceptions specified in Table 6.6.1.12.4.1-3 and Table 6.6.1.5.5-1 config 1 with the exceptions specified in Table 6.6.1.12.5-1 define the primary level settings.

Table 6.6.1.12.5-1: NR Cell specific test parameters for NR SA FR1 event-triggered reporting with SSB index reading for 12 PRB bandwidth

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| BWchannel | MHz | 1 | 3: NRB,c = 15 |  | 3: NRB,c = 15 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.2 FDD |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.3 FDD |  | N/A |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.7 FDD |  | N/A |  |  |
| Io | dBm/2.7 MHz | 1 | -70.00 | -67.65 | -70.00 | -67.65 |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_with_index

Tidentify_intra_with_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra + TSSB_time_index_intra_less_than_5MHz

TPSS/SSS_sync_intra = 600 ms

TSSB_time_index_intra_less_than_5MHz = 140 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

The overall delays measured shall be less than a total of 942 ms in this test case (note: this gives a total of 940 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.1.13	NR SA FR1 event-triggered reporting tests without gap under Cell DTX

6.6.1.13.1	Test purpose

To verify the UE’s ability to make correct reporting of an event within intra-frequency cell search without gap under Cell DTX configuration.

6.6.1.13.2	Test applicability

This test applies to all types of NR UE release 18 and forward and support Cell DTX operation.

6.6.1.13.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.1.13.

6.6.1.13.4	Test description

6.6.1.13.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.1.13.4.1-1.

Table 6.6.1.13.4.1-1: Supported test configurations for NR SA FR1 event-triggered reporting without gap with Cell DTX

| Test Case ID | Description |
| --- | --- |
| 6.6.1.13-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.1.13-2 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.1.13-3 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: The UE fulfils the requirements in this Cell DTX test case can skip the test case in 6.6.1.2 DRX.1 configuration. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.1.13.4.1-2.

Table 6.6.1.13.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap with Cell DTX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.1.13.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.1.13.4.1-3 below.

2. Message contents are defined in clause 6.6.1.13.4.3.

3. There is one carrier and two cells specified in the test. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.1.13.4.1-3: General test parameters for SA intra-frequency event triggered reporting without gap for FR1 with Cell DTX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 | DRX.1 |  |
| Cell DTX |  | 1,2,3 | DTX.1 |  |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.1.13.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the same frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.1.13.4.1-3 and Table 6.6.1.13.5-1, respectively. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2. During time duration T2, NW triggers the Cell DTX.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.1.13.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.1.13.5-1. T2 starts.

6. SS transmit an RRCReconfiguration message to configure and activate Cell DTX. The UE shall transmit RRCReconfigurationComplete message.

7. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1082 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport message in step 7) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

9. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.1.13.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.1.13.4.3-1: Common Exception messages for SA intra-frequency event triggered reporting tests without gap with Cell DTX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 for configuration 6.6.1.13-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.13-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1 and synchronous cells for configuration 6.6.1.13-3 Table H.3.1-4 with A3-offset = -4.5 Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition SSB RLM Table H.3.7-1 with Condition DRX.1 and Offset |

Table 6.6.1.13.4.3-2: RRCReconfiguration (step 6)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.6.1.13.4.3-3: CellGroupConfig (Table 6.6.1.13.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |  |
| spCellConfigDedicated SEQUENCE { |  |  |  |  |
| servingCellConfig SEQUENCE { |  |  |  |  |
| initialDownlinkBWP SEQUENCE { |  |  |  |  |
| pdcch-Config CHOICE { |  |  |  |  |
| setup |  | PDCCH-Config |  |  |
| } |  |  |  |  |
| cellDTX-DRX-Config-r18 CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| cellDTX-DRX-onDurationTimer-r18 CHOICE { |  |  |  |  |
| milliSeconds |  | ms1 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-CycleStartOffset-r18 CHOICE { |  |  |  |  |
| ms160 |  | 0 |  |  |
| } |  |  |  |  |
| cellDTX-DRX-SlotOffset-r18 |  | 0 |  |  |
| cellDTX-DRX-ConfigType-r18 |  | dtx |  |  |
| cellDTX-DRX-ActivationStatus-r18 |  | activated |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| positionInDCI-cellDTRX-r18 |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.6.1.13.5	Test requirement

Table 6.6.1.13.4.1-3 and Table 6.6.1.13.5-1 define the primary level settings including test tolerances for NR event-triggered reporting without gap under Cell DTX.

Table 6.6.1.13.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting tests without gap with Cell DTX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 + 160 = 1080 ms from the beginning of time period T2, while 160ms is the Cell DTX cycle length, which is the delay uncertainty for next available PUSCH for L3 MR reporting during cell DTX. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.2	Inter-frequency measurements

#### 6.6.2.0	Minimum conformance requirements for Inter-frequency measurements

6.6.2.0.1	Minimum conformance requirements for Inter-frequency measurement with measurement gaps

Same as clause 4.6.2.0 when measurement gaps are provided, or the UE supports capability of conducting such measurements without gaps.

6.6.2.0.2	Minimum conformance requirements for Inter-frequency measurement without measurement gaps

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured). Otherwise UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter-frequency SS block of an already detected cell within Tidentify_inter_without_index. It is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2, the following condition is met:

-	SFN and frame boundary across serving cell and inter-frequency neighbour cells is aligned

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 6.6.2.0-1

TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured given in table 6.6.2.0-2.

T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 6.6.2.0-3 and table 6.6.2.0-4.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e. when inter-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps or according to CSSFwithin_gap,i in clause 9.1.5.2 for measurement conducted within measurement gaps, i.e. when inter-frequency SMTC is fully overlapping with measurement gaps.

Mpss/sss_sync_inter: For a UE supporting FR2 power class 1, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2 power class 2, Mpss/sss_sync_inter = 24 samples. For a UE supporting FR2 power class 3, Mpss/sss_sync_inter = 24 samples. For a UE supporting FR2 power class 4, Mpss/sss_sync = 24 samples.

Mmeas_period_inter: For a UE supporting FR2 power class 1, Mmeas_period_inter =40 samples. For a vehicle mounted UE supporting FR2 power class 2, Mmeas_period_inter =24 samples. For a UE supporting FR2 power class 3, Mmeas_period_inter =24 samples. For a UE supporting FR2 power class 4, Mmeas_period_inter = 24 samples.

When inter-frequency SMTC is fully non overlapping with measurement gaps or inter-frequency SMTC is fully overlapping with MGs, Kp=1.

When inter-frequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP.

For FR2,

Klayer1_measurement=1,

-	if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap are not fully overlapped by intra-frequency SMTC occasions, or

-	if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any FR2 serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by SSB-ToMeasure and RSSI symbols are indicated by SS-RSSI-Measurement;

Klayer1_measurement=1.5, otherwise.

If the above-mentioned reference signal configured for L1-RSRP measurement is aperiodic CSI-RS resource, longer cell identification delay would be expected.

Table 6.6.2.0-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |  |

Table 6.6.2.0-2: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |  |

The UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 6.6.2.0-3 and 6.6.2.0-4, if UE supports inter-frequency measurement without measurement gaps:

Table 6.6.2.0-3: Measurement period for inter-frequency measurements without gaps ((FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |  |

Table 6.6.2.0-4: Measurement period for inter-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_inter x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_inter x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mmeas_period_inter xKp x Klayer1_measurement) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. |  |

If UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the Network, UE is required to be capable of measuring without measurement gaps when the SSB is completely contained in the active bandwidth part of the UE. When any of the conditions in the following clauses is met, there are restrictions on the scheduling availability; otherwise, there is no scheduling restriction. Note that the SSB symbols to be measured in the following clauses are the SSB symbols indicated by SSB-ToMeasure [13], if it is configured; otherwise, all L SSB symbols within the SMTC window duration defined in clause 4.1 of TS 38.213 [8] are included.

The scheduling availability requirements when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2 in TS 38.133 [6] clause 9.3.9.3.1~9.3.9.3.3 are valid under the following conditions:

-	SFN and frame boundary across serving cell and inter-frequency neighbour cells is aligned, and

When UE performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

When UE performs inter-frequency measurements without measurement gaps in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

When TDD intra-band carrier aggregation is performed, the scheduling restrictions due to one serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

For UE which do not support simultaneousRxDataSSB-DiffNumerology-Inter-r16 [11] the following restrictions apply due to SS-RSRP/RSRQ/SINR measurement

-	If UE performs inter-frequency measurements without measurement gaps in a TDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

-	If UE performs inter-frequency measurements without measurement gaps in a FDD band, UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration.

When intra-band carrier aggregation is performed, the scheduling restrictions due to a given serving cell should also apply to all other serving cells in the same band on the symbols that fully or partially overlap with aforementioned restricted symbols.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3.

6.6.2.0.3	Minimum conformance requirements for event-triggered measurement with MUSIM gap with lower and higher priority

Same as clause 6.6.1.0.6.

6.6.2.0.4	Minimum conformance requirements for event triggered reporting using MUSIM gap with shorter MGRP

Same as clause 6.6.1.0.6.

#### 6.6.2.1	NR SA FR1-FR1 event-triggered reporting in non-DRX

6.6.2.1.1	Test purpose

To verify that the UE makes correct reporting of an event in non-DRX within inter-frequency NR cell search requirements without SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.6.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.1.

6.6.2.1.4	Test description

6.6.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.1.4.1-2. Test environment parameters are given in Table 6.6.2.1.4.1-3.

Table 6.6.2.1.4.1-1: SA FR1-FR1 event triggered reporting tests in non-DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.1.4.1-2: SA FR1-FR1 general test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

Table 6.6.2.1.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.1.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.1.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.1.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.1.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 920 ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR:
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.1.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.1.5	Test requirement

Table 6.6.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non-DRX

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%. with a confidence level of 95%

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.2	NR SA FR1-FR1 event-triggered reporting in DRX

6.6.2.2.1	Test purpose

To verify that the UE makes correct reporting of an event in DRX within inter-frequency NR cell search requirements without SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.2.

6.6.2.2.4	Test description

6.6.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.2.4.1-2. Test environment parameters are given in Table 6.6.2.2.4.1-3.

Table 6.6.2.2.4.1-1: SA FR1-FR1 event triggered reporting tests in DRX supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.2.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 |  | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | As specified in TS 38.133 [6] clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.1 | 11 |  |

Table 6.6.2.2.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.2.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.2.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

UE needs to be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.2.4.1-2 and 6.6.2.2.5-1.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.2.5-1. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test 1 or 10240 ms for Test 2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.2.4.1-2 as appropriate.

6.6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.2.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-1 | Table H.3.1-3 with Conditions INTER-FREQ MO and Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.2-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.2.5	Test requirement

Table 6.6.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.2.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1 and 2, UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.3	Void

#### 6.6.2.4	Void

#### 6.6.2.5	NR SA FR1-FR1 event-triggered reporting in non-DRX with SSB time index detection

6.6.2.5.1	Test purpose

To verify that the UE makes correct reporting of an event in non-DRX within inter-frequency NR cell search requirements with SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.5.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.6.2.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.5.

6.6.2.5.4	Test description

6.6.2.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.5.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.5.4.1-2. Test environment parameters are given in Table 6.6.2.5.4.1-3.

Table 6.6.2.5.4.1-1: SA FR1-FR1 event triggered reporting tests in non-DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.5-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.5-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.5-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.5.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection in non-DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2 |
| Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in TS 38.133 [6] clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1.1 |  |

Table 6.6.2.5.4.1-3: Environment test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.5.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

6.6.2.5.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.5.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.5.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1040 ms for Test 1 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.2.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.5.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and Condition SSB Index Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 Table H.3.1-7 with Conditions INTER-FREQ and SSB Index |
| Specific message contents exceptions for Test Configuration 6.6.2.5-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.5-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.5-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.5.5	Test requirement

Table 6.6.2.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.5.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| EsIot |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.4 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1040 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.6	NR SA FR1-FR1 event-triggered reporting in DRX with SSB time index detection

6.6.2.6.1	Test purpose

To verify that the UE makes correct reporting of an event in DRX within inter-frequency NR cell search requirements with SSB time index detection in TS 38.133 [6] clause 9.3.4.

6.6.2.6.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.2.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.6.

6.6.2.6.4	Test description

6.6.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.6.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.6.4.1-2. Test environment parameters are given in Table 6.6.2.6.4.1-3.

Table 6.6.2.6.4.1-1: SA FR1-FR1 event triggered reporting tests in DRX with SSB time index detection supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.2.6-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 6.6.2.64-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 6.6.2.6-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.6.4.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 |  | NR Cell 2 is on NR RF channel number 2 |
| Gap Pattern Id |  | Config 1,2,3 | 0 |  | As specified in TS 38.133 [6] clause 9.1.2-1 |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | DRX.7 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.3 | 13.5 |  |

Table 6.6.2.6.4.1-3: Test Environment parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. Message contents are defined in clause 6.6.2.6.4.3.

2. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Table Annex C.1.2.

6.6.2.6.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.6.4.1-2 and Table 6.6.2.6.5-1.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.6.5-1. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1280 ms for Test 1 or 12160 ms for Test 2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.6.4.1-2 as appropriate.

6.6.2.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.6.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB and Condition SSB Index Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 Table H.3.1-7 with Conditions INTER-FREQ and SSB Index Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.6-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.6.5	Test requirement

Table 6.6.2.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.2.6.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR 2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR 2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | - |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 12160 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

In test 1 and 2, UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.7	Void

#### 6.6.2.8	Void

#### 6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern

6.6.2.9.1	Test purpose

To verify that the UE makes correct reporting of an event when mandatory gap pattern with 3ms MGL is configured within inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.4.

6.6.2.9.2	Test applicability

This test applies to all types of NR UE release 16 onwards supporting supportedGapPattern-NRonly-r16 and Gap Pattern 3.

6.6.2.9.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.9.

6.6.2.9.4	Test description

6.6.2.9.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.9.4.1-1.

Table 6.6.2.9.4.1-1: SA FR1 event triggered reporting tests with additional mandatory gap pattern supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.2.9-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 6.6.2.9-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 6.6.2.9-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| NOTE.1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.9.4.1-2.

Table 6.6.2.9.4.1-2: Test Environment parameters for SA FR1 event triggered reporting tests with additional mandatory gap pattern

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.9.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.9.4.1-3.

2.	Message contents are defined in clause 6.6.2.9.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.9.4.1-3: General test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 3 | As specified in clause TS 38.133 [6] 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 12 |  |
|  |  | Config 2,3 | 9 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.2.9.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.9.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.9.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1280 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR:

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.2.9.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.9.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.9-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.9 Table H.3.1-6 with Conditions gapUE, Pattern #3 and gap offset = 12 |
| Specific message contents exceptions for Test Configuration 6.6.2.9-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 Table H.3.1-6 with Conditions gapUE, Pattern #3 and gap offset = 9 |
| Specific message contents exceptions for Test Configuration 6.6.2.9-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 Table H.3.1-6 with Conditions gapUE, Pattern #3 and gap offset = 9 |

6.6.2.9.5	Test requirement

Table 6.6.2.9.5-1 defines the primary level settings including test tolerances for SA inter-frequency event triggered reporting with additional mandatory gap pattern.

Table 6.6.2.9.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting with additional mandatory gap pattern

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.1 |  | SMTC.9 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

UE is not required to report SSB time index.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used

6.6.2.10.1	Test purpose

To verify that the UE makes correct reporting of an event when no gap is configured and DRX is used within inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.9.

6.6.2.10.2	Test applicability

This test applies to all types of NR UE release 16 which supports interFrequencyMeas-Nogap-r16.

6.6.2.10.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.10.

6.6.2.10.4	Test description

6.6.2.10.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.10.4.1-1.

Table 6.6.2.10.4.1-1: SA event triggered reporting tests for FR1 without gap when DRX is used supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.2.10-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.2.10-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.2.10-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.10.4.1-2.

Table 6.6.2.10.4.1-2: Test Environment parameters for SA event triggered reporting tests for FR1 without gap when DRX is used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.10.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.10.4.1-3.

2.	Message contents are defined in clause 6.6.2.10.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.10.4.1-3: General test parameters for SA event triggered reporting tests for FR1 without gap when DRX is used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.1 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.2.10.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.10.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.10.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1080 ms for Test Configuration 1 and 900 ms for Test Configuration 2 and 3then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR:

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.10.4.1-2 as appropriate.

6.6.2.10.4.3	Message contents

Table 6.6.2.10.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.10-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.10-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.10-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.10.5	Test requirement

Table 6.6.2.10.5-1 defines the primary level settings including test tolerances for SA event triggered reporting tests for FR1 without gap when DRX is used.

Table 6.6.2.10.5-1: Cell specific test parameters for SA event triggered reporting tests for FR1 without gap when DRX is used

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Noc Note2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |
| Noc Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test config 1, UE is required to report SSB time index. UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

In test config 2 and 3, UE is not required to report SSB time index. The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 900 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used

6.6.2.11.1	Test purpose

To verify that the UE makes correct reporting of an event when no gap is configured and DRX is not used within inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.9.

6.6.2.11.2	Test applicability

This test applies to all types of NR UE release 16 which supports interFrequencyMeas-Nogap-r16.

6.6.2.11.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.11.

6.6.2.11.4	Test description

6.6.2.11.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.11.4.1-1.

Table 6.6.2.11.4.1-1: SA event triggered reporting tests for FR1 without gap when DRX is not used supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.2.11-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 6.6.2.11-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 6.6.2.11-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| NOTE.1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.11.4.1-2.

Table 6.6.2.11.4.1-2: Test Environment parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.11.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.10.4.1-3.

2.	Message contents are defined in clause 6.6.2.10.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.11.4.1-3: General test parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.2.11.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.11.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.11.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 800 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR:

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.11.4.1-2 as appropriate.

6.6.2.11.4.3	Message contents

Table 6.6.2.11.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.11-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.11-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.11s-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.11.5	Test requirement

Table 6.6.2.11.5-1 defines the primary level settings including test tolerances for SA event triggered tests for FR1 without gap when DRX is not used.

Table 6.6.2.11.5-1: Cell specific test parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Noc Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Noc Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE is not required to read the neighbour cell SSB index in this test.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 6.6.2.12	NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

6.6.2.12.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event when UE is configured with highSpeedMeasInterFreq-r17. This test will partly verify the SA inter-frequency NR cell search requirements in 38.133 [6] clause 9.3.4.

6.6.2.12.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward supporting enhanced inter-frequency NR measurement requirements in high-speed scenario (measurementEnhancementInterFreq-r17, as defined in TS 38.306 [11]).

6.6.2.12.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.12

6.6.2.12.4	Test description

6.6.2.12.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.12.4.1-1.

Table 6.6.2.12.4.1-1: Supported test configurations for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

| Config | Description |
| --- | --- |
| 6.6.2.12-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.2.12-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.2.12-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.12.4.1-2.

Table 6.6.2.12.4.1-2: Initial conditions for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.12.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.12.4.1-3.

2.	Message contents are defined in clause 6.6.2.12.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.12.4.1-3: General test parameters for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 9 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | DRX.14 | As specified in clause A.5 |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 2.3 |  |

6.6.2.12.4.2	Test procedure

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.12.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.12.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 2242 ms, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and proceed with step 1,
OR:
- if the device has been switched off,  proceed with step 1.

10. Repeat step 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.2.12.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.12.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.14 |
| Specific message contents exceptions for Test Configuration 6.6.2.12-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.12-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.12-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

Table 6.6.2.12.4.3-2: SIB4: NR inter-frequency cell re-selection with highSpeedMeasInterFreq-r17

| Derivation Path: Table H.2.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| InterFreqCarrierFreqInfo-v1700 ::= SEQUENCE { |  |  |  |
| highSpeedMeasInterFreq-r17 | TRUE |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.2.12.5	Test requirement

Table 6.6.2.12.5-1 defines the primary level settings including test tolerances for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17.

Table 6.6.2.12.5-1: NR cell specific test parameters for NR SA FR1-FR1 event triggered reporting tests without SSB time index detection in DRX for UE configured with highSpeedMeasInterFreq-r17

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  | Config 1, 2, 3 | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Pattern |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | SR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | SR2.1 TDD |  | NA |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CR.2.1 TDD |  | NA |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  | NA |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz | Config 1,2,3 | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.2 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2 | AWGN |  | AWGN 1944Hz Note 5 |  |
|  |  |  | Config 3 | AWGN |  | AWGN 3334Hz Note 6 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The AWGN 1944 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 6: The AWGN 3334 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 2240 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

UE is not required to report SSB time index.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

NOTE:	The actual overall delays measured in the test is 2ms higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.13	NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority

6.6.2.13.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event on an inter-frequency layer based on measurement performed within measurement gaps when UE is also configured with MUSIM gaps. This test will partly verify the SA inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.4. In addition, the test will verify that UE makes correct transmission/reception during occasions of measurement gaps and MUSIM gaps that are dropped due to collision handling as defined in TS 38.133 [6] clause 9.1.10.

6.6.2.13.2	Test applicability

This test applies to all types of NR UE release 18 and forward, supporting providing MUSIM assistance information with MUSIM gap preference and related MUSIM gap configuration.

6.6.2.13.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.1 and 6.6.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.13.

6.6.2.13.4	Test description

6.6.2.13.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.13.4.1-1.

Table 6.6.2.13.4-1: Supported test configurations for NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority

| Config | Description |
| --- | --- |
| 6.6.2.13-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.2.13-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.2.13-3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.13.4.1-2.

Table 6.6.2.13.4.1-2: Initial conditions for NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.13.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.13.4.1-3.

2.	Message contents are defined in clause 6.6.2.13.4.3.

3. There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.13.4.1-3: General test parameters for NR SA FR1-FR1 event triggered reporting using MUSIM gap with lower and higher priority

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Measurement gap Pattern Id |  | Config 1,2,3 | 0 | As specified in clause TS 38.133 [6] 9.1.2-1 |
| Measurement gap offset | ms | Config 1,2,3 | 9 |  |
| MUSIM gap Pattern Id |  | Config 1,2,3 | 1 |  |
| MUSIM gap offset | ms | Config 1,2,3 | 2 |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |
| T3 | s | Config 1,2,3 | 5 |  |
| T4 | s | Config 1,2,3 | 1.3 |  |

6.6.2.13.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency from the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.2.13.4.1-3 and Table 6.6.2.13.5-1, respectively. In the measurement control information a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of four successive time periods, with time duration of T1, T2, T3 and T4 respectively. During time duration T1 and T3, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.13.5-1.

3. SS shall transmit an RRCReconfiguration message with measConfig and musim-GapConfig-r17. Priority of MISIM gap is lower than the priority of the measurement gap configured by GapConfig-r17.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. The SS starts continuously scheduling PDSCH reception on every DL slot not overlapped with measurement gap occasions.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.13.5-1. T2 starts.

7. If UE

a)	transmits a MeasurementReport message triggered by Event A3 within 922ms from the beginning of time period T2, and

b)	sends HQRQ ACK/NACK for every corresponding PDSCH scheduled in PCell if PDSCH and PUCCH are not overlapped with the measurement gap occasions in time period T1 and T2.

Then the number of successes for the event “low priority MUSIM gap” is increased by one. Otherwise, count a fail for the event “low priority MUSIM gap” and go to step 14.

8. when T2 expires, set the parameters according to T3 in Table 6.6.2.13.5-1. The SS stops scheduling PDSCH reception. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008).

9. SS shall transmit an RRCReconfiguration message to reconfigure MUSIM gap with priority higher than measurement gap configured by GapConfig-r17.

10. The UE shall transmit RRCReconfigurationComplete message. T3 starts.

11. The SS starts continuously scheduling PDSCH reception on every DL slot not overlapped with MUSIM gap occasions and measurement gap occasions that are not colliding with MUSIM gap.

12.When T3 expires, the SS shall switch the power setting from T3 to T4 as specified in Table 6.6.2.13.5-1. T4 starts.

13. If UE

a)	transmits a MeasurementReport message triggered by Event A3 within 1282 ms from the beginning of time period T4, and

b)	sends HQRQ ACK/NACK for every corresponding PDSCH scheduled in PCell if PDSCH and PUCCH are not overlapped with the MUSIM gap occasions and measurement gap occasions that are not colliding with MUSIM gap in time period T3 and T4.

Then the number of successes for the event “high priority MUSIM gap” is increased by one. Otherwise, count a fail for the event “high priority MUSIM gap” and go to step 14.

14. After the SS receive the MeasurementReport message in step 13) or when T4 expires, the SS shall stop scheduling PDSCH reception and:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

or

- switch the UE off.

15. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

16. Depending on the choice in Step 14, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
or
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

17. Repeat step 2-16 until the confidence level according to Tables G.2.3-1 in Annex G.2 is achieved. Each of the events “low priority MUSIM gap” and “high priority MUSIM gap” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. If all events pass, the test passes. If one event fails, the test fails.

6.6.2.13.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.13.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-4 with Condition SSB Index and A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.13-1 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.13-2 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.13-3 | Table H.3.1-3 with Condition INTER-FREQ MO, SSB.2 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

Table 6.6.2.13.4.3-2: RRCReconfiguration: NR measurement Configuration (Step 3 and Step 9, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT specified in Table H.3.1-2 with Condition INTER-FREQ and GAP NEED | Measurement configuration | Step 3 |
|  | Not present |  | Step 9 |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-GapConfig-r17 CHOICE { |  |  |  |
| setup | MUSIM-GapConfig | MUSIM gap configuration Table 6.6.2.13.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.13.4.3-3: MeasGapConfig (Table 6.6.2.13.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe and Pattern #0 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapFR2 | not present |  |  |
| gapFR1 | not present |  |  |
| gapUE | not present |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 1 entry |  |  |
| GapConfig-r17[1] SEQUENCE { |  | Entry 1 |  |
| gapOffset-r17 | 9 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.13.4.3-4: MUSIM-GapConfig (Table 6.6.2.13.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-82BA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | not present |  |  |
| musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17 { | 1 entry |  |  |
| MUSIM-Gap-r17[1] SEQUENCE { |  | Entry 1 |  |
| musim-GapId-r17 | 0 |  |  |
| musim-GapInfo-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 | not present |  |  |
| musim-GapLength-r17 | ms6 | Pattern #1 |  |
| musim-GapRepetitionAndOffset-r17 CHOICE { |  |  |  |
| ms80 | 2 | Pattern #1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| musim-AperiodicGap-r17 | not present |  |  |
| musim-GapToAddModListExt-v1820 SEQUENCE (SIZE (1..3)) OF MUSIM-GapExt-v1820 { | 1 entry |  |  |
| MUSIM-GapExt-v1820 SEQUENCE { |  | Entry 1 |  |
| gapPriority-r18 | 3 |  | Step 3 |
|  | 1 |  | Step 9 |
| } |  |  |  |
| musim-GapKeep-r18 | not present |  |  |
| } |  |  |  |

6.6.2.13.5	Test requirement

Table 6.6.2.13.4.1-3 and Table 6.6.2.13.5-1 define the primary level settings including test tolerances for NR SA event triggered reporting test with MUSIM gap configured.

Table 6.6.2.13.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1, T3 | T2, T4 | T1, T3 | T2, T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Noc Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Noc Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

During T1 and T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the measurement gap occasions.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

During T3 and T4, after UE sends RRC ReconfigurationComplete message for the reconfiguration of MUSIM gap priority, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with

-	MUSIM gap occasions, and

-	measurement gap occasions that are not colliding with MUSIM gap.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T4. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

UE is not required to report SSB time index.

The rate of correct events observed during repeated tests shall be at least 90%.

The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.2.14	NR SA FR1-FR1 event triggered reporting using MUSIM gap with shorter MGRP

Editor’s Note: This test case is incomplete in following aspects:

TT analysis is still missing.

6.6.2.14.1	Test purpose

The purpose of this test is to verify that the MUSIM gap capable UE makes correct reporting of an event when MUSIM gap collides with measurement gaps without assigned priority. This test will partly verify the SA inter-frequency NR cell search requirements in clause TS 38.133 [6] 9.3.4. This test is conducted assuming that the measurement gap is configured as Gap(s) configured via GapConfig without suffix, and the MUSIM gap parameters are configured as requested by UE.

6.6.2.14.2	Test applicability

This test applies to all types of NR UE release 18 and forward, supporting providing MUSIM assistance information with MUSIM gap preference and related MUSIM gap configuration.

6.6.2.14.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.1 and 6.6.2.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.14.

6.6.2.14.4	Test description

6.6.2.14.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.14.4.1-1.

Table 6.6.2.14.4.1-1: Supported test configurations for NR SA FR1-FR1 event-triggered reporting using MUSIM gap with shorter MGRP

| Configuration | Description |
| --- | --- |
| 6.6.2.14-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.2.14-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.2.14-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations. Note2: target NR cell has the same SCS, BW and duplex mode as NR serving cell. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.14.4.1-2.

Table 6.6.2.14.4.1-2: Initial conditions for NR SA FR1-FR1 event-triggered reporting using MUSIM gap with shorter MGRP

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.14.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.14.4.1-3.

2.	Message contents are defined in clause 6.6.2.14.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.14.4.1-3: General test parameters for NR SA FR1-FR1 event-triggered reporting using MUSIM gap with shorter MGRP

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3 | 1 | As specified in clause TS 38.133 [6] 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 4 |  |
| MUSIM Gap Pattern Id |  | Config 1,2,3 | 0 | As specified in clause TS 38.133 [6] 9.1.10-1 |
| MUSIM gap offset |  | Config 1,2,3 | 39 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.2.14.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 6.6.2.14.4.1-3 and Table 6.6.2.14.5-1, respectively. In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

Measurement gap pattern configuration defined in Table 6.6.2.14.4.1-3 is provided for a UE that does not support per-FR gap, and no gap pattern (Gap Pattern Id and Measurement gap offset) is configured for a UE capable of per-FR gap. In test 1 measurement gap pattern configuration # 1 as defined in Table 6.6.2.14.4.1-3 is provided for a UE that does not support per-FR gap and in test 2 without measurement gap pattern configuration as defined in Table 6.6.2.14.4.1-3 is provided for UE that support per-FR gap.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.14.5-1.

3. SS shall transmit an RRCReconfiguration message with measConfig and musim-GapConfig-r17.

4. The UE shall transmit an RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.14.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1282 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
Or
- if the device has been switched off, switche on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.2.14.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.2.14.4.3-1: Common Exception messages SA inter-frequency event triggered reporting using MUSIM gap with shorter MGRP

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.14-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.7 |
| Specific message contents exceptions for Test Configuration 6.6.2.14-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.7 |
| Specific message contents exceptions for Test Configuration 6.6.2.14-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.7 |

Table 6.6.2.14.4.3-2: RRCReconfiguration: NR measurement Configuration (Step 3, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig-DEFAULT specified in Table H. 3.1-2 with Condition INTER-FREQ and GAP NEEDED | Measurement configuration |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| musim-GapConfig-r17 CHOICE { |  |  |  |
| setup | MUSIM-GapConfig | MUSIM gap configuration Table 6.6.2.14.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.14.4.3-3: MeasGapConfig (Table 6.6.2.14.4.3-2)

| Derivation Path: Table H.3.1-6 with condition gapUE |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapUE CHOICE { | 1 entry |  |  |
| setup SEQUENCE { |  | 1entry | test 1 |
| gapOffset | 4 |  |  |
| mgl | ms6 | Pattern#1 |  |
| mgrp | ms80 | Pattern#1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.2.14.4.3-4: MUSIM-GapConfig (Table 6.6.2.14.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-82BA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MUSIM-GapConfig-r17 ::= SEQUENCE { |  |  |  |
| musim-GapToReleaseList-r17 | not present |  |  |
| musim-GapToAddModList-r17 SEQUENCE (SIZE (1..3)) OF MUSIM-Gap-r17 { | 1 entry |  |  |
| MUSIM-Gap-r17[1] SEQUENCE { |  | entry 1 |  |
| musim-GapId-r17 | 0 |  |  |
| musim-GapInfo-r17 SEQUENCE { |  |  |  |
| musim-Starting-SFN-AndSubframe-r17 | not present |  |  |
| musim-GapLength-r17 | ms6 | Pattern #0 |  |
| musim-GapRepetitionAndOffset-r17 CHOICE { |  |  |  |
| ms40 | 39 | Pattern #0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| musim-AperiodicGap-r17 | not present |  |  |
| } |  |  |  |

6.6.2.14.5	Test requirement

Table 6.6.2.14.4.1-3 and Table 6.6.2.14.5-1 define the primary level settings including test tolerances for NR SA FR1-FR1 event-triggered reporting using MUSIM gap with shorter MGRP.

Table 6.6.2.14.5-1: NR Cell specific test parameters for NR SA FR1-FR1 event-triggered reporting using MUSIM gap with shorter MGRP

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.7 |  | SMTC.7 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Noc Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| Noc Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

In test 1 with per-UE gap and in test 2 without the gap, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1280 ms from the beginning of time period T2.

The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured shall be less than a total of 1282 ms in this test case (note: this gives a total of 800 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.2.15	NR SA FR1 event triggered reporting without SSB index detection in DRX mode for UE operating on a cell with less than 5MHz BW

6.6.2.15.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting of an event without SSB time index in DRX mode when the inter-frequency target cell is less than 5MHz bandwidth. This test will partly verify the inter-frequency cell search requirements in TS 38.133 [6] clause 9.3.4.

6.6.2.15.2	Test applicability

This test applies to all types of NR UE Release 18 supporting less than 5 MHz bandwidth.

6.6.2.15.3	Minimum conformance requirements

The minimum conformance requirements are specified in TS 38.133 [6] clause 9.3.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.15.

6.6.2.15.4	Test description

6.6.2.15.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.15.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.2.2.4.1-2 with config 1 apply to this test. Test environment parameters are given in Table 6.6.2.15.4.1-2.

Table 6.6.2.15.4.1-1: SA FR1 event triggered reporting tests in DRX supported test configuration

| Test Case ID | Description |
| --- | --- |
| 6.6.2.15-1 | NR 15 kHz SSB SCS, 3 MHz bandwidth, FDD duplex mode |
| Note 1: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Table 6.6.2.15.4.1-2: Test Environment parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.15.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	Message contents are defined in clause 6.6.2.15.4.3.

2.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2 and C.1.3.

6.6.2.15.4.2	Test procedure

The test procedures are same as 6.6.2.2.4.2 except these steps:

2.	Set the parameters according to T1 in Table 6.6.2.2.4.1-2 config 1 and Table 6.6.2.2.5-1 config 1 with the exceptions specified in Table 6.6.2.15.5-1.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.2.5-1 config 1 with the exceptions specified in Table 6.6.2.15.5-1. T2 Starts.

6.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1082 ms for Test 1 or 10242 ms for Test 2, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

6.6.2.15.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clauses 4.6 and 7.3 with the following exceptions:

Table 6.6.2.15.4.3-1: Common Exception messages SA inter-frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-FREQ Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.y FR1 and Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-6 with Conditions gapUE, Pattern #0 and gap offset = 9 Table H.3.1-7 with Condition INTER-FREQ Table H.3.7-1 with Condition DRX.1 and Gap and INTER-FREQ for Test 1 Table H.3.7-1 with Condition DRX.7 and Gap and INTER-FREQ for Test 2 |

6.6.2.15.5	Test requirement

Table 6.6.2.2.5-1 config 1 with the exceptions specified in Table 6.6.2.15.5-1 defines the primary level settings for all tests.

Table 6.6.2.15.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Duplex mode |  | Config 1 | FDD |  |  |  |
| BWchannel | MHz | Config 1 | 3: NRB,c = 15 |  |  |  |
| BWP BW | MHz | Config 1 | 3: NRB,c = 15 |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 | SR.1.2 FDD |  | NA |  |
| RMSI CORESET Reference Channel |  | Config 1 | CR.1.3 FDD |  | NA |  |
| Dedicated CORESET Reference Channel |  | Config 1 | CCR.1.7 FDD |  | NA |  |
| SSB parameters |  | Config 1 | SSB.13 FR1 |  | SSB.13 FR1 |  |
| IoNote3 | dBm/3MHz | Config 1 | -69.9 | -69.9 | -75.4 | -67.6 |

In test 1 and 2, UE is not required to report SSB time index.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 10240 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The overall delays measured shall be less than a total of 1082 ms in test 1 and 10242 ms for test 2 (note: the total measurement reporting delay plus 2 ms for TTI insertion uncertainty).

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

### 6.6.3	Inter-RAT Measurements

#### 6.6.3.0	Minimum conformance requirements

6.6.3.0.1	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN FDD

The requirements are applicable for NR−E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN FDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RSRQ related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RS-SINR related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [23].

6.6.3.0.1.1	Requirements when no DRX is used

When the UE requires measurement gaps to identify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, the UE shall be able to identify a new detectable FDD cell within TIdentify, E-UTRAN FDD according to the following expression:

TIdentify,E-UTRAN FDD=TBasicIdentify* 480TInter1*CSSFinterRAT     ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in TS 38.133 [6] section 9.4.1,

CSSFinterRAT = CSSFwithin_gap_i is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in TS 38.133 [6] section 9.1.5.2.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN FDD defined in Table 6.6.3.0.1.1-1.

Table 6.6.3.0.1.1-1: Measurement period and measurement bandwidth

| Configuration | Physical Layer Measurement period: TMeasure, E-UTRAN FDD [ms] | Measurement bandwidth [RB] |
| --- | --- | --- |
| 0 | 480 x CSSFinterRAT | 6 |
| 1 (note 1) | 240 x CSSFinterRAT | 50 |
| NOTE 1: This configuration is optional. |  |  |

The UE shall be capable of identifying and performing NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements of at least 4 E-UTRAN FDD cells per E-UTRA FDD carrier frequency layer for up to 7 E-UTRA FDD carrier frequency layers.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.5.

6.6.3.0.1.2	Requirements when DRX is used

When DRX is in use and measurement gaps are configured, the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in Table 6.6.3.0.1.2-1. When RRM enhancement for high speed is configured the UE shall be able to identify a new detectable E-UTRAN FDD cell within TIdentify, E-UTRAN FDD specified in Table 6.6.3.0.1.2-2.

Table 6.6.3.0.1.2-1: Requirement to identify a newly detectable E-UTRAN FDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in Section 6.6.3.0.1.1 apply | Non-DRX requirements in Section 6.6.3.0.1.1 apply |
| 0.256 | 5.12*K (20*CSSFinterRAT) | 7.68*K (30*CSSFinterRAT) |
| 0.32 | 6.4*K (20*CSSFinterRAT) | 7.68*K (24*CSSFinterRAT) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.1.1. |  |  |

Table 6.6.3.0.1.2-2: Requirement to identify a newly detectable E-UTRAN FDD cell for UE configured with RRM enhancement for high speed

| DRX cycle length (s) | TIdentify, E-UTRAN FDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.2.2 apply | Non-DRX requirements in clause 9.4.2.2 apply |
| 0.16<DRx cycle<=0.32 | Note 1(15*CSSFinterRAT) |  |
| 0.32<DRx cycle <= 0.64 | Note 1(10*CSSFinterRAT) |  |
| DRx cycle = 1.024 | Note 1(10*CSSFinterRAT) | Note 1(10*CSSFinterRAT) |
| DRx cycle = 1.28 | Note 1(8*CSSFinterRAT) | Note 1(8*CSSFinterRAT) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.2.2. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN FDD cells per E-UTRA FDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA FDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN FDD specified in Table 6.6.3.0.1.2-3.

Table 6.6.3.0.1.2-3: Requirement to measure E-UTRAN FDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN FDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX requirements in Section 6.6.3.0.1.1 apply |
| 0< DRX-cycle ≤10.24 | Note1 (5* CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.1.1. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN FDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.2. The NR – E-UTRAN FDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.3. The NR – E-UTRAN FDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.5.

6.6.3.0.1.3	Measurement reporting requirements for Event-Triggered Reporting

The reported NR – E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements contained in event-triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.2.2, 10.2.3, and 10.2.5, respectively.

The UE shall not send any event-triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T Identify, E-UTRAN FDD defined in clauses 6.6.3.0.1.1 and 6.6.3.0.1.2 without DRX and with DRX, respectively. When L3 filtering is used, an additional delay can be expected.

If a cell which has been detectable at least for the time period TIdentify, E-UTRAN FDD becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event as per TS 38.331 [13], the event triggered measurement reporting delay shall be less than TMeasure, E-UTRAN FDD provided the timing to that cell has not changed more than  50 Ts while measurement gap has not been available and the L3 filter has not been used.

The normative reference for this requirement is TS 38.133 [6] clause 9.4.2.

6.6.3.0.2	Minimum conformance requirements for inter-RAT event triggered reporting to E-UTRAN TDD

The requirements are applicable for NR−E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN TDD cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RSRQ related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RS-SINR related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [23].

6.6.3.0.2.1	Requirements when no DRX is used

When the UE requires measurement gaps to identify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, the UE shall be able to identify a new detectable TDD cell within TIdentify, E-UTRAN TDD according to the following expression:

-	When configuration 0 or configuration 1 in Table 6.6.3.0.2-1 is applied,

TIdentify,E-UTRAN TDD=TBasicIdentify* 480TInter1*CSSFinterRAT     ms ,

-	When configuration 2 or configuration 3 in Table 6.6.3.0.2-1 is applied,

TIdentify,E-UTRAN TDD=(TBasicIdentify* 480TInter1+240)*CSSFinterRAT    ms,

where:

TBasicIdentify = 480 ms,

TInter1 is defined in TS 38.133 [6] section 9.4.1,

CSSFinterRAT = CSSFwithin_gap_i is the scaling factor for the measured inter-RAT E-UTRA carrier i which is calculated as specified in TS 38.133 [6] section 9.1.5.2.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN TDD defined in Table 6.6.3.0.2.1-1.

Table 6.6.3.0.2.1-1: TMeasure, E-UTRAN TDD for different configurations

| Configuration | Measurement bandwidth [RB] | Number of UL/DL sub-frames per half frame (5 ms) |  | DwPTS |  | TMeasure, E-UTRAN TDD [ms] |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | Normal CP | Extended CP |  |
| 0 | 6 | 2 | 2 |  |  | 480 x CSSFinterRAT |
| 1 (note 1) | 50 | 2 | 2 |  |  | 240 x CSSFinterRAT |
| 2 | 6 | 1 | 3 |  |  | 720 x CSSFinterRAT |
| 3 (Note 1) | 50 | 1 | 3 |  |  | 480 x CSSFinterRAT |
| NOTE 1: This configuration is optional. NOTE 2: Void |  |  |  |  |  |  |

The UE shall be capable of identifying and performing NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements of at least 4 E-UTRAN TDD cells per E-UTRA TDD carrier frequency layer for up to 7 E-UTRA TDD carrier frequency layers.

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN TDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.2. The NR – E-UTRAN TDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.3. The NR – E-UTRAN TDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] section 10.2.5.

6.6.3.0.2.1	Requirements when DRX is used

When DRX is in use and measurement gaps are configured, the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in Table 6.6.3.0.2.1-1. When RRM enhancement for high speed is configured the UE shall be able to identify a new detectable E-UTRAN TDD cell within TIdentify, E-UTRAN TDD specified in Table 6.6.3.0.2.1-2.

Table 6.6.3.0.2.1-1: Requirement to identify a newly detectable E-UTRAN TDD cell

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in Section 6.6.3.0.2.1 apply | Non-DRX requirements in Section 6.6.3.0.2.1 apply |
| 0.256 | 5.12*K (20*CSSFinterRAT) | 7.68*K (30*CSSFinterRAT) |
| 0.32 | 6.4*K (20*CSSFinterRAT) | 7.68*K (24*CSSFinterRAT) |
| 0.32< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.2.1. |  |  |

Table 6.6.3.0.2.1-2: Requirement to identify a newly detectable E-UTRAN TDD cell for UE configured with RRM enhancement for high speed

| DRX cycle length (s) | TIdentify, E-UTRAN TDD (s) (DRX cycles) |  |
| --- | --- | --- |
|  | Gap period = 40 ms, 20 ms | Gap period = 80 ms |
| ≤0.16 | Non-DRX requirements in clause 9.4.3.2 apply | Non-DRX requirements in clause 9.4.3.2 apply |
| 0.16<DRx cycle<=0.32 | Note 1(15*CSSFinterRAT) |  |
| 0.32<DRx cycle <= 0.64 | Note 1(10*CSSFinterRAT) |  |
| DRx cycle = 1.024 | Note 1(10*CSSFinterRAT) | Note 1(10*CSSFinterRAT) |
| DRx cycle = 1.28 | Note 1(8*CSSFinterRAT) | Note 1(8*CSSFinterRAT) |
| 1.28< DRX-cycle ≤10.24 | Note1 (20*CSSFinterRAT) | Note1 (20*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in clause 9.4.3.2. |  |  |

When DRX is in use, the UE shall be capable of performing NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements of at least 4 identified E-UTRAN TDD cells per E-UTRA TDD frequency layer during each layer 1 measurement period, for up to 7 E-UTRA TDD carrier frequency layers, and the UE physical layer shall be capable of reporting NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements to higher layers with the measurement period Tmeasure, E-UTRAN TDD specified in Table 6.6.3.0.2.1-3.

Table 6.6.3.0.2.1-3: Requirement to measure E-UTRAN TDD cells

| DRX cycle length (s) | Tmeasure, E-UTRAN TDD (s) (DRX cycles) |
| --- | --- |
| ≤0.08 | Non-DRX Requirements in Section 6.6.3.0.2.1 apply |
| 0.128 | For configuration 2, non-DRX requirements in section 6.6.3.0.2.1 apply, Otherwise: Note1 (5*CSSFinterRAT) |
| 0.128<DRX-cycle≤10.24 | Note1 (5*CSSFinterRAT) |
| NOTE 1: The time depends on the DRX cycle length. NOTE 2: CSSFinterRAT is as defined in Section 6.6.3.0.2.1. |  |

If higher layer filtering is used, an additional cell identification delay can be expected.

The NR – E-UTRAN TDD RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.2. The NR – E-UTRAN TDD RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.3. The NR – E-UTRAN TDD RS-SINR measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] Section 10.2.5.

6.6.3.0.2.3	Measurement reporting requirements for Event-Triggered Reporting

The reported NR – E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements contained in event-triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.2.2, 10.2.3, and 10.2.5, respectively.

The UE shall not send any event-triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than T Identify, E-UTRAN TDD defined in clauses 6.6.3.0.2.1 and 6.6.3.0.2.2 without DRX and with DRX, respectively. When L3 filtering is used, an additional delay can be expected.

If a cell which has been detectable at least for the time period TIdentify, E-UTRAN TDD becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event as per TS 38.331 [13], the event triggered measurement reporting delay shall be less than TMeasure, E-UTRAN TDD provided the timing to that cell has not changed more than  50 Ts while measurement gap has not been available and the L3 filter has not been used.

The normative reference for this requirement is TS 38.133 [6] clause 9.4.3.

#### 6.6.3.1	NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX

6.6.3.1.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 under the cell search and measurement requirements.

6.6.3.1.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 from Release 15 onwards.

6.6.3.1.3	Minimum conformance requirements

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.1.

6.6.3.1.3.1	NR – E-UTRAN FDD requirement

The minimum conformance requirements are specified in clause 6.6.3.0.1.

6.6.3.1.3.2	NR – E-UTRAN TDD requirement

The minimum conformance requirements are specified in clause 6.6.3.0.2.

6.6.3.1.4	Test description

6.6.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.1.4.1-1.

Table 6.6.3.1.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.3.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.1-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.1-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.1-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.1.4.1-2 and Table 6.6.3.1.4.1-3.

Table 6.6.3.1.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.1.5-1 and Table 6.6.3.1.5-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.1.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.3.1.5-1 and 6.6.3.1.5-2. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause TS 38.133 [6] Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| NOTE 1: Values are defined in Table 6.6.3.1.5-1 |  |  |  |

1.	Message contents are defined in clause 6.6.3.1.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.6.3.1.4.2	Test procedure

The test consists of two successive time periods, with time durations of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. Gap pattern configuration is configured before T2 begins to enable inter-frequency monitoring.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.3.1.5-1 and 6.6.3.1.5-2. Propagation conditions are set according to Annex C clause C.2.2.

3.	SS shall transmit an RRCReconfiguration message.

4.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.3.1.5-1 and 6.6.3.1.5-2.

6.	UE shall transmit a MeasurementReport message triggered by Event B2. If the measurement reporting delay from the beginning of time period T2 is less than 3842ms the number of successful tests is increased by one. If the UE fails to report the event within the measurement reporting delay requirement then the number of failure tests is increased by one.

7.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9.	Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED according to TS 38.508-1 [14] clause 4.5.4 (if the paging fails, switches off and on the UE and ensures the UE is in the state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5,
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.3.1.4.3-1: Common Exception messages NR SA FR1 – E-UTRAN event-triggered reporting in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT |
| Specific message content exceptions for Test Configurations 6.6.3.1-1 and 6.6.3.1-4 | Table H.3.1-3 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message content exceptions for Test Configurations 6.6.3.1-2, 6.6.3.1-3, 6.6.3.1-5 and 6.6.3.1-6 | Table H.3.1-3 with Condition Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

6.6.3.1.5	Test requirement

Table 6.6.3.1.5-1 and Table 6.6.3.1.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.3.1.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 kHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 kHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| CORESET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG pattern note1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| Noc note2 |  |  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Noc note2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| Ês/Iot note3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| SS-RSRP note3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.55 |
| SSB_RP note3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.55 |
| Io note3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.42 | -73.74 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.32 | -67.64 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.1.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configuration note1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configuration note1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement Channel note2 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement Channel note2 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG Patterns note2 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RA note3 |  |  |  |  |
| OCNG_RB note3 |  |  |  |  |
| Noc note4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| Ês/Iot note5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| RSRP note5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| SCH_RP note5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| Io note5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21 | -54.60 |
| Propagation Condition not 6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3842ms from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The overall delays measured test requirement is expressed as:

Tidentify,E-UTRAN FDD = TBasicIdentify * 480 / TInter1 * CSSFinterRAT ms

Which:

TBasicIdentify = 480,

TInter1 = 60,

CSSFinterRAT = 1

TTI insertion uncertainty = TTIDCCH = 1 ms; 2xTTIDCCH = 2 ms

The overall delays measured shall be less than a total of 3842 ms in this test case (note: this gives a total of 3840 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

The rate of correct events observed during repeated tests shall be at least 90% with confidence level of 95%.

#### 6.6.3.2	NR SA FR1 – E-UTRAN event-triggered reporting in DRX

6.6.3.2.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used under the cell search and measurement requirements.

6.6.3.2.2	Test applicability

This test applies to all types of NR UE Release 15 and forward supporting 5GS NR SA FR1, E-UTRAN and long DRX cycle.

6.6.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 6.6.3.0.1 and 6.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.2.

6.6.3.2.4	Test description

6.6.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.2.4.1-1.

Table 6.6.3.2.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Test Case ID | Description |
| --- | --- |
| 6.6.3.2-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.2-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.2-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.2-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.2-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.2-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.2.4.1-2 and Table 6.6.3.2.4.1-3.

Table 6.6.3.2.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.2.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 |  | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 |  | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.3.2.5-1 and 6.6.3.2.5-2. |  |  |
| Active cell |  | Cell 1 |  | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 |  | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 |  | As specified in Clause TS 38.133 [6] Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP |  | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP |  | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | note 1 |  | SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 |  | E-UTRAN RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| Hysteresis | dB | 0 |  |  |
| TimeToTrigger | s | 0 |  |  |
| Filter coefficient |  | 0 |  | L3 filtering is not used |
| DRX |  | DRX.1 | DRX.7 | DRX cycle configurations DRX.1 and DRX.7 are defined in Table A.3.3.1-1 and Table A.3.3.2-1 respectively. |
| T1 | s | 5 |  |  |
| T2 | s | 5 | 15 |  |
| NOTE 1: Values are defined in Table 6.6.3.2.5-1 |  |  |  |  |

1.	Message contents are defined in clause 6.6.3.2.4.3.

2.	There are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

6.6.3.2.4.2	Test procedure

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter-RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore the UE shall be allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.3.2.5-1 and 6.6.3.2.5-2.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 6.6.3.2.5-1 and 6.6.3.2.5-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 3.48 s for Test 1 or less than 12.8 s for Test 2 then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14+2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.3.2.4.3-1: Common Exception messages NR SA FR1 – E-UTRAN event-triggered reporting in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT Table H.3.7-1 with Condition DRX.1 and Gap for Test 1 Table H.3.7-1 with Condition DRX.7 and OFFSET for Test 2 |
| Specific message content exceptions for Test Configurations 6.6.3.2-1 and 6.6.3.2-4 | Table H.3.1-3 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message content exceptions for Test Configurations 6.6.3.2-2, 6.6.3.2-3, 6.6.3.2-5 and 6.6.3.2-6 | Table H.3.1-3 with Condition Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

6.6.3.2.5	Test requirement

Table 6.6.3.2.5-1 and Table 6.6.3.2.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.6.3.2.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 kHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 kHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.55 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.55 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.42 | -73.74 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.32 | -67.64 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.2.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5MHz: NRB,c = 25 10MHz: NRB,c = 50 20MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5MHz: R.7 FDD 10MHz: R.3 FDD 20MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.4 TDD 10MHz: R.0 TDD 20MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5MHz: R.11 FDD 10MHz: R.6 FDD 20MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5MHz: R.11 TDD 10MHz: R.6 TDD 20MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5MHz: OP.20 FDD 10MHz: OP.10 FDD 20MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5MHz: OP.9 TDD 10MHz: OP.1 TDD 20MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21 | -54.60 |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

In test 1, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

In test 2, the UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 12.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

#### 6.6.3.3	NR SA FR1 – E-UTRAN event-triggered reporting in DRX for UE configured with highSpeedMeasFlag-r16

6.6.3.3.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 when DRX is used under the cell search and measurement requirements for UE configured with RRM enhancement for high speed.

6.6.3.3.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 configured with RRM enhancement for high speed from Release 15 onwards.

6.6.3.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 6.6.3.0.1 and 6.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.3.3.

6.6.3.3.4	Test description

6.6.3.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.3.3.4.1-1.

Table 6.6.3.3.4.1-1: Supported test configurations in SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Test Case ID | Description |
| --- | --- |
| 6.6.3.3-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.3.3-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.3-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.3.3-4 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.3.3-5 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.3.3-6 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.3.3.4.1-2 and Table 6.6.3.3.4.1-3.

Table 6.6.3.3.4.1-2: Initial conditions for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.3.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 6.6.3.3.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables A.6.6.3.3.1-2 and A.6.6.3.3.1-3. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table 9.1.2-1. Per-UE gap pattern. |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm | -97 | E-UTRAN RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | DRX.6 | DRX cycle configurations DRX.6 is defined in Table A.3.3.1-6. |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in TS 38.133 [6] Table 6.6.3.3.5-1 |  |  |  |

1.	Message contents are defined in clause 6.6.3.3.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

3. UE is configured with highSpeedMeasFlag-r16

6.6.3.3.4.2	Test procedure

In each test there are two cells: Cell 1 and Cell 2. Cell 1 is the NR PCell and Cell 2 is an inter-RAT E-UTRAN inter-RAT neighbour cell. In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter-RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

In each test the UE shall be provided at least once every 500ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furhtermore the UE shall be allocated with PUSCH resource at every DRX cycle.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.3.3.5-1 and 6.6.3.3.5-2.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 6.6.3.3.5-1 and 6.6.3.3.5-2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 4.8s then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14+2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.3.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks on condition of HighSpeedMeas with exceptions | Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Conditions GAP NEEDED and INTER-RAT Table H.3.1-3 with Condition SSB.1 FR1 for configuration 6.6.3.3-1, 6.6.3.3-2, 6.6.3.3-4, 6.6.3.3-5 Table H.3.1-3 with Condition SSB.2 FR1 for configuration 6.6.3.3-3, 6.6.3.3-6 Table H.3.1-3A Table H.3.1-4A Table H.3.1-5 with Condition INTER-RAT Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-RAT Table H.3.7-1 with Condition DRX.6 and Gap |

6.6.3.3.5	Test requirement

Table 6.6.3.3.5-1 and Table 6.6.3.3.5-2 defines the primary level settings including test tolerances for all tests.

Table 6.6.3.3.5-1: PCell specific test parameters for SA inter-RAT E-UTRA event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 kHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 kHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORESET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORESET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |
|  |  |  |  | 3, 6 | -95 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |
|  |  |  |  | 3, 6 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 | -108 |
|  |  |  |  | 3, 6 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.3.3.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in DRX with PCell in FR1 for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 19 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -78.22+10log (NRB,c /50) | -59.16+10log (NRB,c /50) |
| Propagation Condition Note6 |  | 1, 2, 3, 4, 5, 6 | AWGN1944 |  |
| Antenna Configuration and Correlation Matrix Note6 |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: Propagation condition and correlation matrix are defined in clause B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 4.8s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

### 6.6.4	L1-RSRP measurement for beam reporting

#### 6.6.4.0	Minimum conformance requirements

6.6.4.0.1	Minimum conformance requirements for SSB-based L1-RSRP measurement for beam reporting

Same as clause 4.6.4.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.5.3.1, 9.5.4.1 and 9.5.5.1.

6.6.4.0.2	Minimum conformance requirements for CSI-RS-based L1-RSRP measurement for beam reporting

Same as clause 4.6.4.0.2

The normative reference for this requirement is TS 38.133 [6] clauses 9.5.3.1, 9.5.4.2 and 9.5.5.2.

6.6.4.0.3	Inter-cell SSB based L1-RSRP Reporting

If a cell with PCI different from serving cell is known according 9.13.2 of TS 38.133 [6], the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_CDP. The requirements specified in this clause are only applicable when

-	highSpeedMeasFlag-r16 is not configured, and

-	highSpeedMeasFlagFR2-r17 is not configured, and

-	highSpeedMeasCA-Scell-r17 is not configured, and

-	SSBs of CDP outside SMTCs for L1-RSRP measurement are not overlapped with concurrent gaps.

The value of TL1-RSRP_Measurement_Period_SSB_CDP is defined in Table 6.6.4.0.3-1 for FR1,

For FR1,

-	P=11-TSSB_CDPMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the SSB; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the SSB.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For either an FR1 or FR2 cell with PCI different from serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell with PCI different from serving cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 6.6.4.0.3-1: Inter-cell L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_CDP for known cells with different PCIs in FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB_CDP (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_CDP) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_CDP)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_CDP is the periodicity of the SSB-Index configured for inter-cell L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. |  |

#### 6.6.4.1	NR SA FR1 SSB-based L1-RSRP measurement in non-DRX

6.6.4.1.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

6.6.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.1.

6.6.4.1.4	Test description

6.6.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.1.4.1-2. Test environment parameters are given in Table 6.6.4.1.4.1-3.

Table 6.6.4.1.4.1-1: NR SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.1.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.1.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.1.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.1.4.1-2 and 6.6.4.1.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.6.4.1.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.4.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.4.1.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 6.6.4.1.5-1. T2 starts.

5.	The UE shall start sending valid L1-RSRP reports. The SS shall check following requirements:

- R1:	the UE shall start to transmit valid reports no later than 720 ms for configuration 1, 2 and no later than 680 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement for SSB#1 (Table 6.6.4.1.5-2 for test configurations 1, 2 and Table 6.6.4.1.5-3 for test configuration 3 and the relative L1-RSRP requirement for SSB#0 in Table 6.6.4.1.5-4. If the first valid report is received earlier than the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the UE shall transmit reports every 80 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

- R3:	The L1-RSRP value of SSB#1 reported by the UE is compared to the expected L1-RSRP value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.4.1.5-2 for test configurations 1, 2 and in Table 6.6.4.1.5-3 for test configuration 3 or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4:	The DIFF RSRP value of SSB#0 reported by the UE is compared to the expected DIFF RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.4.1.5-4 or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.1.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with condition SSB Table H.3.6-10 |

Table 6.6.4.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.4.1.5	Test requirement

Table 6.6.4.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.1.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NocNote2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| NocNote2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
| EsIot | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.14 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
| EsNoc | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 4.6.4.1.5-2 for for test configuration 1, 2 and the corresponding absolute accuracy requirements in Table 4.6.4.1.5-3 for test configuration 3 and the corresponding relative accuracy requirements in Table 4.6.4.1.5-4 for all test configurations.

Table 6.6.4.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 55 |
| Highest reported value (SSB#1) | - | 75 |

Table 6.6.4.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 6.6.4.1.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R4) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in  DCCH.

#### 6.6.4.2	NR SA FR1 SSB-based L1-RSRP measurement in DRX

6.6.4.2.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.4.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.2.

6.6.4.2.4	Test description

6.6.4.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.2.4.1-2. Test environment parameters are given in Table 6.6.4.2.4.1-3.

Table 6.6.4.2.4.1-1: SA SSB based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.2.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.2.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.2.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.2.4.1-2 and 6.6.4.2.5-1. UE is configured to perform RLM and BFD based on the SSBs. DRX is configured as specified in Table 6.6.4.2.4.1-2.

6.6.4.2.4.2	Test procedure

Same test procedure as in subclause 6.6.4.1.4.2 with tables 6.6.4.1.4.1-2 and 6.6.4.1.5-1 replaced by tables 6.6.4.2.4.1-2 and 6.6.4.2.5-1.

6.6.4.2.4.3	Message contents

Same message content as in subclause 6.6.4.1.4.3 with the following exception:

Table 6.6.4.2.4.3-1: Common Exception messages EN-DC SSB based L1-RSRP measurement in DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.7-1 with condition DRX.3 |

6.6.4.2.5	Test requirement

Table 6.6.4.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.2.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NocNote2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| NocNote2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
| EsIot | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.14 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
| EsNoc | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.2.5-2 for for test configurations 1, 2 and the corresponding absolute accuracy requirements in Table 6.6.4.2.5-3 for test configuration 3 and the corresponding relative accuracy requirements in Table 6.6.4.2.5-4 for all test configurations.

Table 6.6.4.2.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 55 |
| Highest reported value (SSB#1) | - | 75 |

Table 6.6.4.2.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 6.6.4.2.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.3	NR SA FR1 CSI-RS-based L1-RSRP measurement in non-DRX

6.6.4.3.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in non-DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

6.6.4.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.3.

6.6.4.3.4	Test description

6.6.4.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.3.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.3.4.1-2. Test environment parameters are given in Table 6.6.4.3.4.1-3.

Table 6.6.4.3.4.1-1:NR SA CSI-RS based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.3.4.1-2: General test parameters for NR SA CSI-RS based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.3.4.1-3: Test Environment parameters for NR SA CSI-RS based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.3.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.3.4.1-2 and 6.6.4.3.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.3.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-RSRP on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. UE is also configured to measure L1-RSRP based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in table 6.6.4.3.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.4.3.5-1. T1 starts.

3.	After 80ms from the start of the test the SS transmits the DCI trigger in slot 2 for configuration 1, slot 4 for configuration 2 and slot 8 for configuration 3. The corresponding CSI-RS set is transmitted with the offset of 0 slots after the DCI trigger.

4.	The SS shall check following requirements:

- R1:	the UE shall send L1-RSRP report at slot 8 from the reception of DCI trigger. If the report is received at slot 8 from the reception of DCI trigger, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	The L1-RSRP value of CSI-RS#1 reported by the UE is compared to the expected L1-RSRP value for CSI-RS #1. If the resulting value is outside the limits in Table 6.6.4.3.5-2 for test configurations 1, 2 and in Table 6.6.4.3.5-3 for test configuration 3 or the UE fails to report the measurement value for CSI-RS #1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

-R3:	The DIFF RSRP value of CSI-RS #0 reported by the UE is compared to the expected DIFF RSRP value. If the resulting value is outside the limits in Table 6.6.4.3.5-4 or the UE fails to report the measurement value for CSI-RS #0, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

5.	Void.

6.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.3.4.3-1: Common Exception messages NR SA CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions APERIODIC and CSI-RSRP Table H.3.6-3 with conditions CSI-RS TS 38.508-1 [14] Table 7.3.1-21 with condition APERIODIC |

Table 6.6.4.3.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.4.3.4.3-3: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [14], Table 7.2.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | K1 = 3 if mod(i,10) = 8 where i is slot index per frame; i = {0,…,9} | - | Config1 (NR FDD) |
| PDSCH-to-HARQ_feedback timing indicator | K1 = 6 if mod(i,20) = 11 where i is slot index per frame; i = {0,…,19} | - | Config3 (NR TDD30kHz) |

6.6.4.3.5	Test requirement

Table 6.6.4.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.3.5-1: SSB specific test parameters for NR SA CSI-RS based L1-RSRP measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| NocNote1 | 1~3 | dBm/15kHz | -94.65 |  |
| NocNote1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
| EsIot | 1~3 | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.15 |
|  | 3 |  | -91.65 | -88.14 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.49 |
| EsNoc | 1~3 | dB | 0 | 3.5 |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.3.5-2 for for test configurations 1, 2 and the corresponding absolute accuracy requirements in Table 6.6.4.3.5-3 for test configuration 3 and the corresponding relative accuracy requirements in Table 6.6.4.3.5-4 for all test configurations.

Table 6.6.4.3.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 55 |
| Highest reported value (CSI-RS#1) | 75 |

Table 6.6.4.3.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 58 |
| Highest reported value (CSI-RS#1) | 78 |

Table 6.6.4.3.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF RSRP reported (CSI-RS#0) | 0 |
| Highest DIFF RSRP reported (CSI-RS#0) | 3 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.4	NR SA FR1 CSI-RS-based L1-RSRP measurement in DRX

6.6.4.4.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements in TS 38.133 [6] clause 9.5.4.1.

6.6.4.4.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1 and long DRX cycle.

6.6.4.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.4.

6.6.4.4.4	Test description

6.6.4.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.4.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.4.4.1-2. Test environment parameters are given in Table 6.6.4.4.4.1-3.

Table 6.6.4.4.4.1-1: NR SA CSI-RS based L1-RSRP measurement supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.4.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.4.4.1-2: General test parameters for NR SA CSI-RS based L1-RSRP measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity | 1~3 |  | cri-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 8 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.4.4.1-3: Test Environment parameters for NR SA CSI-RS based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.6.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.4.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.4.4.1-2 and 6.6.4.4.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.4.4.2	Test procedure

Same test procedure as in subclause 6.6.4.3.4.2 with tables 6.6.4.3.4.1-2 and 6.6.4.3.5-1 replaced by tables 6.6.4.4.4.1-2 and 6.6.4.4.5-1.

6.6.4.4.4.3	Message contents

Same message content as in subclause 6.6.4.3.4.3 with the following exception:

Table 6.6.4.4.4.3-1: Common Exception messages NR SA CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.7-1 with condition DRX.3 and Offset |

6.6.4.4.5	Test requirement

Table 6.6.4.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.4.5-1: SSB specific test parameters for NR SA CSI-RS based L1-RSRP measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| NocNote1 | 1~3 | dBm/15kHz | -94.65 |  |
| NocNote1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
| EsIot | 1~3 | dB | 0 | 3.5 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.15 |
|  | 3 |  | -91.65 | -88.14 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.49 |
| EsNoc | 1~3 | dB | 0 | 3.5 |
| Note 1: Void Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-RSRP report at slot 8 from the reception of DCI triggering the L1-RSRP measurement. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1.

Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.4.5-2 for for test configurations 1, 2 and the corresponding absolute accuracy requirements in Table 6.6.4.4.5-3 for test configurations3 and the corresponding relative accuracy requirements in Table 6.6.4.4.5-4 for all test configurations.

Table 6.6.4.4.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 55 |
| Highest reported value (CSI-RS#1) | 75 |

Table 6.6.4.4.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 58 |
| Highest reported value (CSI-RS#1) | 78 |

Table 6.6.4.4.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF RSRP reported (CSI-RS#0) | 0 |
| Highest DIFF RSRP reported (CSI-RS#0) | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.5	NR SA FR1 SSB-based L1-RSRP measurement in DRX for UE configured with highSpeedMeasFlag-r16

Editor's note: This test case is incomplete. The following aspects are either missing or not yet determined:

- The feasibility of configuring different channel models to different SSBs needs further study.

- The TT analysis may need to be redone after concluding if the new test configuration is testable.

6.6.4.5.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements for UE configured with highSpeedMeasFlag-r16 in TS 38.133 [6] clause 9.5.4.1

6.6.4.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5GS NR SA FR1, measurement enhancements in high speed scenario and long DRX cycle

6.6.4.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.5.

6.6.4.5.4	Test description

6.6.4.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.4.5.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.4.5.4.1-2. Test environment parameters are given in Table 6.6.4.5.4.1-3.

Table 6.6.4.5.4.1-1: SA SSB based L1-RSRP measurement supported test configurations for UE configured with highSpeedMeasFlag-r16

| Test Case ID | Description |
| --- | --- |
| 6.6.4.5-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.4.5-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.4.5-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.5.4.1-2: General test parameters for NR SA SSB based L1-RSRP measurement for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 2 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.5.4.1-3: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.4.5.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n=1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 with n=1 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.5.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in Table 6.6.4.5.4.1-2 and 6.6.4.5.5-1. UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. DRX is configured as specified in Table 6.6.4.5.4.1-2.

6.6.4.5.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.6.4.5.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.4.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.4.1.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 6.6.4.1.5-1. T2 starts.

5.	If the UE sends L1-RSRP reports  meeting the corresponding absolute accuracy requirements in Table 6.6.4.5.5-2 for test configurations 1 and 2 the corresponding absolute accuracy requirements in Table 6.6.4.5.5-3 for test configurations 3 and the corresponding relative accuracy requirements in Table 6.6.4.5.5-4 for all test configurations every 80 slots from no later than 640 ms plus 80 slots for all configurations from the beginning of time period T2 until the end of time period T2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.5.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-3 with Condition HighSpeedMeas |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with conditions SSB Table H.3.6-10 Table H.3.7-1 with condition DRX.3 |

Table 6.6.4.5.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.4.5.5	Test requirement

Table 6.6.4.5.5-1: SSB specific test parameters for UE configured with highSpeedMeasFlag-r16

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Noc Note2 | 1~3 | dBm/15kHz | -94.6 |  |  |  |
| Noc Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
| EsIot | 1~3 | dB | 0 | 0 | -Infinity | 3.53 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.15 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.14 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.59 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.49 |
| EsNoc | 1~3 | dB | 0 | 0 | -Infinity | 3.5 |
| Propagation conditions | 1,2 |  | AWGN |  | AWGN 1944 HzNote4 |  |
|  | 3 |  | AWGN |  | AWGN 3334 Hz Note5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 1944 Hz. Note 5: The AWGN 3334 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 3334 Hz. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of both SSB0 and SSB1. Each L1-RSRP measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.4.5.5-2 for for test configurations 1 and 2 and the corresponding absolute accuracy requirements in Table 6.6.4.5.5-3 for test configurations 3 and the corresponding relative accuracy requirements in Table 6.6.4.5.5-4 for all test configurations.

Table 6.6.4.5.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 55 |
| Highest reported value (SSB#1) | - | 75 |

Table 6.6.4.5.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 58 |
| Highest reported value (SSB#1) | - | 78 |

Table 6.6.4.5.5-4: L1-RSRP relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF RSRP reported (SSB#0) | - | 0 |
| Highest DIFF RSRP reported (SSB#0) | - | 3 |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.4.6	NR SA FR1 Inter-cell SSB based L1-RSRP measurements on PCell in DRX

6.6.4.6.1	Test purpose

To verify that the UE makes correct reporting of L1-RSRP measurement in DRX within L1-RSRP measurement requirements for SSB of a cell with additional PCI.

6.6.4.6.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting 5GS NR SA FR1, measurement Inter-cell SSB based L1-RSRP measurements on PCell in DRX.

6.6.4.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.4.6.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.4.6.

6.6.4.6.4	Test description

6.6.4.6.4.1	Initial conditions

This test will partly verify the L1-RSRP measurement requirements in clause 9.5.4.1 of TS 38.133 [6] , with the testing configurations for NR cells in Table 6.6.4.6.1-1.

Table 6.6.4.6.1-1: Applicable NR configurations for FR1 SSB based L1-RSRP test

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.4.6.2-1: General test parameters

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| Active cell | 1~3 |  | Cell 1 |
| A cell with additional PCI | 1~3 |  | Cell 2 |
| RF Channel Number | 1~3 |  | 1: Cell 1 and Cell 2 |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 for Cell 1 and Cell 2 |
|  | 2 |  | SSB.3 FR1 for Cell 1 and Cell 2 |
|  | 3 |  | SSB.4 FR1 for Cell 1 and Cell 2 |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity | 1~3 |  | ssb-Index-RSRP |
| Number of reported RS | 1~3 |  | 2 |
| L1-RSRP reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| CP length | 1~3 |  | Normal |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Time offset between serving and a cell with additional PCI | 1 |  | < CP |
|  | 2 |  | < CP |
|  | 3 |  | < CP |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.4.6.2-2: Test Environment parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.4.6.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.4.6.4.3.

2.	There are two cells in the test, the FR1 PCell (Cell 1) and a cell with additional PCI (Cell 2). The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.4.6.4.1-2 and 6.6.4.6.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.4.6.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.6.4.6.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.4.6.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.4.6.5-1. T1 starts.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in 6.6.4.6.5-1. T2 starts.

5.	The UE shall start sending valid L1-RSRP reports. The SS shall check following requirements:

-	R1:	the UE shall start to transmit valid reports no later than 720 ms for configuration 1, 2 and no later than 680 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement for SSB#1 (Table 6.6.4.6.5-2 for test configurations 1, 2 and Table 6.6.4.6.5-3 for test configuration 3 and the relative L1-RSRP requirement for SSB#0 in Table 6.6.4.6.5-4. If the first valid report is received earlier than the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

-	R2:	the UE shall transmit reports every 80 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

-	R3:	The L1-RSRP value of SSB#1 reported by the UE is compared to the expected L1-RSRP value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.4.6.5-2 for test configurations 1, 2 and in Table 6.6.4.6.5-3 for test configuration 3 or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-	R4:	The DIFF RSRP value of SSB#0 reported by the UE is compared to the expected DIFF RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.4.6.5-4 or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.4.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.4.6.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with condition SSB Table H.3.6-10 |

Table 6.6.4.6.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.4.6.4.3-3: CSI-SSB-ResourceSetConfig

| Derivation Path: TS 38.133 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-SSB-ResourceSet ::= SEQUENCE { |  |  |  |
| csi-SSB-ResourceSetId | 0 |  |  |
| csi-SSB-ResourceList | 0 |  |  |
| servingAdditionalPCIList-r17 | 1 |  |  |
| } |  |  |  |

6.6.4.6.5	Test requirement

Table 6.6.4.6.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.4.6.5-1: SSB specific test parameters for NR SA SSB based L1-RSRP measurement

| Parameter | Config | Unit | SSB#0 of Cell 1 |  | SSB#1 of Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NocNote2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| NocNote2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
| EsIot | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -Infinity | -91.65 |
|  | 3 |  | -91.65 | -91.65 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -57.59 | -60.61 | -55.84 |
| EsNoc | 1~3 | dB | 0 | 0 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. No later than X ms plus 80 slots from the beginning of time period T2, UE shall send L1-RSRP report including results of SSB#1 while meeting the absolute accuracy requirement in clause 10.1.19.1.1 and relative accuracy requirement in clause 10.1.19.1.2, where X is

-	680 for Config 1&2

-	660 for Config 3

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.5	Inter-RAT UTRAN measurements

#### 6.6.5.1	NR SA FR1 – UTRAN event-triggered reporting in non-DRX

6.6.5.1.1	Test purpose

This test is to verify that the UE makes correct event-triggered reporting of inter-RAT UTRAN measurements when operating in standalone (SA) operation with PCell in FR1 under the cell search and measurement requirements.

6.6.5.1.2	Test applicability

This test applies to all types of NR UE supporting SA FR1 from Release 16 onwards and support UTRA FDD.

6.6.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.5.1.

6.6.5.1.4	Test description

6.6.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.5.1.4.1-1.

Table 6.6.5.1.4.1-1: supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.5.1-1 | NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode, UTRAN FDD |
| 6.6.5.1-2 | NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode, UTRAN FDD |
| 6.6.5.1-3 | NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode, UTRAN FDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.5.1.4.1-2 and Table 6.6.5.1.4.1-3.

Table 6.6.5.1.4.1-2: Initial conditions for SA inter-RAT UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-3 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.5.1.5-1 and Table 6.6.5.1.5-2. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | SS LTE in Figure A.3.1.8.2 is replaced by SS UTRA LTE TX/RX in Figure A.3.2.3.2 is replaced by UTRA TX/RX |  |  |

Table 6.6.5.1.4.1-3: General test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| UTRA RF Channel Number |  | 2 | 1 UTRA carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Table 6.6.5.1.5-1 and Table 6.6.5.1.5-2 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in Clause Table FFS Per-UE gap pattern. |
| Inter-RAT UTRA measurement quantity |  | CPICH Ec/Io | Measurement quantity for Cell 2 |
| b1-ThresholdUTRA-FDD | dB | -16.5 | CPICH Ec/Io threshold for SS-RSRP measurement on Cell 1 for event B1 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table A.6.6.5.1.1-3 |  |  |  |

1.	Message contents are defined in clause 6.6.5.1.4.3.

2.	Cell 1 is the NR PCell and Cell 2 is an inter-RAT UTRAN inter-RAT neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.0 and C.1.

6.6.5.1.4.2	Test procedure

The test consists of two successive time periods, with time durations of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. Gap pattern configuration is configured before T2 begins to enable inter-frequency monitoring.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.5.1.5-1 and 6.6.5.1.5-2. Propagation conditions are set according to Annex C clause C.2.2.T1 starts.

3.	SS shall transmit an RRCReconfiguration message to confogure Event B1 measurement reporting.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.5.1.5-1 and 6.6.5.1.5-2.

6.	UE shall transmit a MeasurementReport message triggered by Event B1. If the measurement reporting delay from the beginning of time period T2 is less than 3842ms the number of successful tests is increased by one. If the UE fails to report the event within the measurement reporting delay requirement then the number of failure tests is increased by one.

7.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8.	Set Cell 2 primary scrambling code = ((current Cell 2 primary scrambling code - 50) mod 200 + 100) for next iteration of the test procedure loop.

9.	Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED according to TS 38.508-1 [14] clause 4.5.4 (if the paging fails, switches off and on the UE and ensures the UE is in the state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5,
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10.Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.5.1.4.3	Message contents

Table 6.6.5.1.4.3-1: RRCReconfiguration (Step 3)

| Derivation Path: TS 38.508-1 [14] Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | MeasConfig | Table 6.6.5.1.4.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.5.1.4.3-2: MeasConfig (Table 6.6.5.1.4.3-1)

| Derivation Path: Table H.3.1-2 with condition INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectUTRA-FDD-r16 | MeasObjectUTRA-FDD | Table 6.6.5.1.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT(16) specified in 38.508-1 [14] Table 4.6.3-141 with condition EVENT_B1_UTRA | Actual value of UTRA threshold = (16-49)/2 = -16.5dB |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig specified in Table H.3.1-6 with condition gapUE and Pattern #0 |  |  |
| } |  |  |  |

Table 6.6.5.1.4.3-3: MeasObjectUTRA-FDD (Table 6.6.5.1.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-77A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectUTRA-FDD-r16 ::= SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN-ValueUTRA-FDD-r16 for UTRA Cell 2 |  |  |
| } |  |  |  |

Table 6.6.5.1.4.3-4: MeasurementReport (Step 6)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults | MeasResults | Table 6.6.5.1.4.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.5.1.4.3-5: MeasResults (Table 6.6.5.1.4.3-4)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-79 with condition B1_UTRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListUTRA-FDD-r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultUTRA-FDD-r16 { | 1 entry |  |  |
| MeasResultUTRA-FDD-r16[1] SEQUENCE { |  | entry 1 |  |
| physCellId-r16 | PhysCellIdUTRA-FDD-r16 of UTRA Cell 2 |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| utra-FDD-EcN0-r16 | INTEGER (0..49) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.5.1.5	Test requirement

Table 6.6.5.1.5-1 and Table 6.6.5.1.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.5.1.5-1: PCell specific test parameters for SA inter-RAT UTRA FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
| TDD Configuration |  | SCS=15 kHz |  | 2 | TDDConf.1.1 |  |
|  |  | SCS=30 kHz |  | 3 | TDDConf.1.2 |  |
| BWchannel |  |  | MHz | 1 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1 | SR.1.1 FDD |  |
|  |  |  |  | 2 | SR.1.1 TDD |  |
|  |  |  |  | 3 | SR.2.1 TDD |  |
| CORSET reference channel |  |  |  | 1 | CR.1.1 FDD |  |
|  |  |  |  | 2 | CR.1.1 TDD |  |
|  |  |  |  | 3 | CR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2 | SSB.1 FR1 |  |
|  |  |  |  | 3 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1 | TRS.1.1 FDD |  |
|  |  |  |  | 2 | TRS.1.1 TDD |  |
|  |  |  |  | 3 | TRS.1.2 TDD |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 kHz | 1, 2, 3 | -106 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2 | -106 |  |
|  |  |  |  | 3 | -103 |  |
| Ês/Noc |  |  | dB | 1, 2, 3 | 18 | -2 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3 | 18 | -2 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2 | -88 | -108 |
|  |  |  |  | 3 | -85 | -105 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2 | -59.98 | -75.92 |
|  |  |  | dBm/38.16 MHz | 3 | -53.88 | -69.82 |
| Propagation condition |  |  |  | 1, 2, 3 | ETDLA30 |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3 | 1x2 Low |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.5.1.5-2: UTRAN neighbour cell specific test parameters for SA inter-RAT UTRAN FDD event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| UTRA RF Channel Number |  | 2 |  |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCH_Ec/Ior | dB | -12 |  |
| PICH_Ec/Ior | dB | -15 |  |
| DPCH_Ec/Ior | dB | N/A |  |
| OCNS |  | -0.941 |  |
| IorIoc | dB | -Infinity | -1.8 |
| Ioc | dBm/3.84 MHz | -70 |  |
| CPICH_Ec/Io | dB | -Infinity | -14 |
| Propagation Condition |  | AWGN |  |
| Note 1: The DPCH level is controlled by the power control loop. Note 2: The power of the OCNS channel that is added shall make the total power from the cell to be equal to Ior. |  |  |  |

The UE shall send one Event B1 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 2.4s from the start of period T2, i.e. when Cell 2 becomes detectable. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

### 6.6.6	CLI Measurements

#### 6.6.6.0	Minimum conformance requirements

6.6.6.0.1	Minimum conformance requirements for SRS-RSRP measurement

Same as in clause 4.6.5.0.1.

6.6.6.0.2	Minimum conformance requirements for CLI-RSSI measurement with non-DRX

Same as in clause 4.6.5.0.2.

#### 6.6.6.1	NR SA FR1 SRS-RSRP measurement in non-DRX

6.6.6.1.1	Test purpose

To verify that the UE makes correct reporting of SRS-RSRP measurement. This test will verify the SRS-RSRP measurement requirements in clause 6.6.6.0.

6.6.6.1.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR and CLI-based SRS-RSRP measurements.

6.6.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.6.1.

6.6.6.1.4	Test description

The test scenario comprises of one serving NR FR1 PCell (Cell 1) and one virtual intra-frequency UE transmitting SRS periodically, which are the target of the CLI measurement report evaluated in the test. The test parameters for PCell are given in Table 6.6.6.1.4.1-3 and Table 6.6.6.1.5-1 below. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with event I1 is used. No gap pattern is configured in the test.

During the test, the test system transmits SRS resource for measurement in the DL slot according to the SRS configuration in Table 6.6.6.1.5-3 and the test parameters for the (virtual) neighbour cell UE in Table 6.6.6.1.5-2. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

6.6.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.6.1.4.1-1.

Table 6.6.6.1.4.1-1: Supported test configurations for NR SA FR1 SRS-RSRP measurement in non-DRX

| Configuration | Description |
| --- | --- |
| 6.6.6.1-1 | NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.6.1-2 | NR 30 kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.6.1.4.1-2.

Table 6.6.6.1.4.1-2: Initial conditions for NR SA FR1 SRS-RSRP measurement in non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.6.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.6.1.4.3.

2.	Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3. Virtual UE 1 is the target for SRS-RSRP measurements.

3.	The test parameters are given in Table 6.6.6.1.4.1-3.

Table 6.6.6.1.4.1-3: General test parameters for NR SA FR1 SRS-RSRP measurement in non-DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | Cell 1 |  |
| RF Channel Number |  | 1, 2 | 1: Cell 1 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.1 |  |
|  |  | 2 | SMTC.1 |  |
| SRS configuration |  | 1 | SRSConf.1 | Table 6.6.6.1.5-3 |
|  |  | 2 | SRSConf.2 |  |
| CP length |  | 1, 2 | Normal |  |
| i1-Threshold | dBm | 1 | -97 |  |
|  |  | 2 | -95 |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | DRX.7 |  |
| Time offset between DL from serving cell and SRS from test system | s | 1,2 | 17.67 |  |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

6.6.6.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1 and T2, respectively. During T1 only the serving NR PCell is powered on, but at the start of T2 the virtual UE is powered on and starts transmitting on SRS resources, in addition to the serving NR cell which is kept powered on. The purpose of the test is to evaluate the event i1 reporting delay upon a newly powered up virtual UE, from the start of T2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.6.1.4.1-3, Table 6.6.6.1.5-1 and Table 6.6.6.1.5-2.

3. The SS shall transmit an RRCReconfiguration message configuring a CLI measurement object with a CLI event I1 trigger, as specified in section 6.6.6.1.4.3.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.6.1.4.1-3, Table 6.6.6.1.5-1 and Table 6.6.6.1.5-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by event I1, as specified in section 6.6.6.1.4.3. If the overall delay measured from the beginning of time period T2 is less than 1922 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall requirement delay, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall power off the virtual UE and then transmit an RRCRelease message to release the RRC connection.

8. The SS shall:

- switch off and then on the UE and go to step 9, or

- transmit Paging message (including PagingRecord with ue-Identity) for the UE and go to step 9.

9. Repeat steps 1-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 6.6.6.1.4.3-1: Common Exception messages for NR SA FR1 SRS-RSRP measurement in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 |

Table 6.6.6.1.4.3-2: MeasObjectToAddModList for NR SA FR1 SRS-RSRP measurement in non-DRX

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-77 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectCLI-r16 | MeasObjectCLI-r16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.6.1.4.3-3: MeasObjectCLI-r16 for NR SA FR1 SRS-RSRP measurement in non-DRX

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCLI-r16 ::= SEQUENCE { |  |  |  |
| cli-ResourceConfig-r16 SEQUENCE { |  |  |  |
| srs-ResourceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| SRS-ResourceListConfigCLI-r16 SEQUENCE { | 1 entry |  |  |
| srs-Resource-r16 | SRSConf.1 | entry 1 | Config 1 |
|  | SRSConf.2 | entry 1 | Config 2 |
| srs-SCS-r16 | kHz15 |  | Config 1 |
|  | kHz30 |  | Config 2 |
| refServCellIndex-r16 | 0 | PCell |  |
| refBWP-r16 | 0 | BWP-0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.6.1.4.3-4: ReportConfigNR for NR SA FR1 SRS-RSRP measurement in non-DRX

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| cli-EventTriggered-r16 SEQUENCE { |  |  |  |
| eventId-r16 CHOICE { |  |  |  |
| eventI1-r16 SEQUENCE { |  | Event I1 |  |
| i1-Threshold CHOICE { |  |  |  |
| srs-RSRP-r16 | 43 | 43 = -97-(-140) | Config 1 |
| srs-RSRP-r16 | 45 | 45 = -95-(-140) | Config 2 |
| } |  |  |  |
| reportOnLeave-r16 | False |  |  |
| hysteresis-16 | 0 |  |  |
| timeToTrigger-r16 | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportInterval-r16 | ms120 | Not critical to the test |  |
| reportAmount-r16 | r2 | Similar to other measurement tests |  |
| maxReportCLI-r16 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.6.1.4.3-5: MeasResultCLI-r16 for NR SA FR1 SRS-RSRP measurement in non-DRX

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultCLI-r16 ::= SEQUENCE { |  |  |  |
| measResultsListSRS-r16 CHOICE { |  |  |  |
| srs-ResourceId-r16 | SRS-ResourceId |  |  |
| srs-RSRP-Result-r16 | SRS-RSRP-Range-r16 | INTEGER (0..98) |  |
| } |  |  |  |
| } |  |  |  |

6.6.6.1.5	Test requirement

Table 6.6.6.1.5-1 and Table 6.6.6.1.5-2 define the primary level settings including test tolerances for NR SA FR1 SRS-RSRP measurement in non-DRX. Table 6.6.6.1.5-3 defines the SRS resource configurations.

Table 6.6.6.1.5-1: NR Cell specific test parameters for NR SA FR1 SRS-RSRP measurement in non-DRX

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | TDDConf.1.1 |  |
|  |  | 2 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 TDD |  |
|  |  | 2 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 TDD |  |
|  |  | 2 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 TDD |  |
|  |  | 2 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 TDD |  |
|  |  | 2 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. |  |  |  |  |

Table 6.6.6.1.5-2: Neighbour UE specific test parameters for NR SA FR1 SRS-RSRP measurement in non-DRX

| Parameter | Unit | Test configuration | Neighbour cell UE |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |
|  |  | 2 |  |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -95 |  |
| EsIot | dB | 1 | -infinity | 9.25 8.75 |
|  |  | 2 |  |  |
| EsNoc | dB | 1 | -infinity | 9.25 8.75 |
|  |  | 2 |  |  |
| SRS-RSRP Note 3 | dBm/SCS kHz | 1 | -infinity | -88.75 |
|  |  | 2 | -infinity | -86.25 |
| Io | dBm/9.36 MHz | 1 | -70.05 | -60.62 |
|  | dBm/38.16 MHz | 2 | -63.96 | -57.51 |
| Propagation Condition |  | 1, 2 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 6.6.6.1.5-3: SRS configurations for NR SA FR1 SRS-RSRP measurement in non-DRX

|  | Field | SRSConf.1 | SRSConf.2 | Comments |
| --- | --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |  |
|  | srs-ResourceIdList | 0 | 0 |  |
|  | resourceType | Periodic | Periodic |  |
|  | Usage | Codebook | Codebook |  |
| SRS-Resource | SRS-ResourceId | 0 | 0 |  |
|  | nrofSRS-Ports | Port1 | Port1 |  |
|  | transmissionComb | n2 | n2 |  |
|  | combOffset-n2 | 0 | 0 |  |
|  | cyclicShift-n2 | 0 | 0 |  |
|  | resourceMapping startPosition | 0 | 0 |  |
|  | resourceMapping nrofSymbols | n1 | n1 |  |
|  | resourceMapping repetitionFactor | n1 | n1 |  |
|  | freqDomainPosition | 0 | 0 |  |
|  | freqDomainShift | 0 | 0 |  |
|  | freqHopping c-SRS | 12 | 12 |  |
|  | freqHopping b-SRS | 0 | 0 |  |
|  | freqHopping b-hop | 0 | 0 |  |
|  | groupOrSequenceHopping | Neither | Neither |  |
|  | resourceType | Periodic | Periodic |  |
|  | periodicityAndOffset | sl640, 4 | sl640, 9 |  |
|  | sequenceId | 0 | 0 | Any 10 bit number |

The UE shall send one Event I1 triggered measurement report, with a measurement reporting delay less than 1920 ms from the beginning of time period T2.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.7	NR measurements with autonomous gaps

#### 6.6.7.0	Minimum conformance requirements

6.6.7.0.1	Minimum conformance requirements for SA intra-frequency CGI identification of NR neighbour cell in FR1

[TS 38.133, Clause 9.11]

6.6.7.0.1.1	Introduction

The requirements in this clause are applicable for CGI identification of an intra-frequency and inter-frequency NR target cell.

The requirements in this clause are specified for CGI identification of an NR target cell and are applicable for a UE:

-	in RRC_CONNECTED state, and

-	configured with SA or NR-DC or NE-DC operation mode, or with EN-DC operation mode for CGI identification requested by NR PSCell.

The overall CGI reporting delay is defined in clause 6.6.7.0.1.3.

The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

6.6.7.0.1.2	CGI identification of an NR cell with autonomous gaps

The UE shall identify and report the CGI of a known NR target cell when requested by the network for the purpose of reportCGI. Only one cell is provided to the UE with cellForWhichToReportCGI for identifying the CGI. The UE may make autonomous gaps in both downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3 in TS 38.331 [13]. Note that a UE is not required to use autonomous gap if useAutonomousGaps is set to false. If autonomous gaps are used for measurement with the purpose of reportCGI, regardless of whether DRX is used or not, or whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of NR cell within:

Tidentify_CGI = (TMIB + T SIB1) ms

Where:

TMIB is the time period used to acquire MIB message. TMIB = 6 * TSMTC ms for target cell carrier frequency on FR1 and TMIB = 25 * TSMTC ms for target cell carrier frequency on FR2.

TSIB1 is the time period used to acquire SIB1 message. TSIB1 = 6 * TRMSI-scheduling ms.

Where TSMTC is the SMTC periodicity configured for the target cell measurement, and TRMSI-scheduling is

-	the maximum between the periodicity with which the SIB1 is actually transmitted by the NR target cell and 20ms when SSB and RMSI CORESET multiplexing pattern is 1

-	the maximum between the periodicity with which the SIB1 is actually transmitted by the NR target cell and TSMTC when SSB and RMSI CORESET multiplexing pattern is 2 or 3.

The requirement for identifying the CGI of an NR cell within Tidentify_CGI is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [13] is used.

Within the time Tidentify_CGI, over which the UE identifies the CGI of an NR cell, the UE shall fulfil interruption requirements specified in,

-	Clause 8.2.1.2.16 in TS 38.133 [6] for NR serving cells and Clause 7.32.2.15 in TS 36.133 [23] for E-UTRA serving cells if the UE is configured with EN-DC operation mode,

-	Clause 8.2.2.2.14 in TS 38.133 [6] if the UE is configured with SA operation mode,

-	Clause 8.2.3.2.14 in TS 38.133 [6] for NR serving cells and Clause 7.36.2.14 in TS 36.133 [23] for E-UTRA serving cells if the UE is configured with NE-DC operation mode,

-	Clause 8.2.4.2.11 in TS 38.133 [6] if the UE is configured with NR-DC operation mode.

In the requirement a cell is known if,

-	During the last 5 seconds for FR1 or 3 seconds for FR2 before the reception of the report CGI command:

-	The UE has sent a valid L3-RSRP measurement report with SSB index for the target cell and

-	During MIB decoding at least reported SSBs remains detectable according to the cell identification conditions specified in clauses 9.2 or 9.3 in TS 38.133 [6], and

-	During SIB1 decoding the SSB used for MIB decoding remains detectable according to the cell identification conditions specified in clauses 9.2 or 9.3 in TS 38.133 [6], and

-	During MIB decoding, the SSB for MIB decoding remains detectable with SNR ≥-3dB

-	During SIB1 decoding, the PDSCH for SIB1 decoding remains detectable with SNR ≥-3dB

6.6.7.0.1.3	CGI reporting delay

The CGI reporting delay is defined as the time between a command that will trigger a CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI defined in clause 6.6.7.0.1.2 plus RRC procedure delay defined in clause 12 in TS 38.331 [13], and additional 20ms margin if target cell is on FR2.

6.6.7.0.2	Minimum conformance requirements for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

[TS 38.133, Clause 9.4.7]

6.6.7.0.2.1	CGI identification of an E-UTRA cell with autonomous gaps

The requirements in this clause apply when the UE is configured with standalone NR, NE-DC or NR-DC. The UE shall identify and report the CGI when requested by an NR PCell for the purpose ‘reportCGI’. The UE may make autonomous gaps in downlink reception and uplink transmission for receiving MIB and SIB1 message according to clause 5.5.3.1 in TS 38.331 [13]. If autonomous gaps are used for measurement with the purpose of ‘reportCGI’, regardless of whether DRX is used or not, or regardless of whether SCell(s) are configured or not, the UE shall be able to identify a new CGI of E-UTRA cell within Tidentify_CGI, E-UTRAN= 150 ms. This is the maximum allowed time for the UE to identify a new CGI of an E-UTRA cell, provided that the E-UTRA cell has been already identified by the UE.

A cell shall be considered identifiable following conditions are fulfilled:

-	RSRP related side conditions given in Clause 9.1 in TS 36.133 [23] are fulfilled for a corresponding band,

-	SCH_RP and SCH Ês/Iot according to Annex B.2.2 in TS 36.133 [23] for a corresponding band

The MIB of an E-UTRA cell whose CGI is identified shall be considered decodable by the UE provided the PBCH demodulation requirements are met according to TS 36.101 [27].

The requirement for identifying a new CGI of an E-UTRA cell within Tidentify_CGI, E-UTRAN is applicable when no DRX is used as well as when any of the DRX cycles specified in TS 38.331 [13] is used.

6.6.7.0.2.2	CGI reporting delay

The E-UTRA CGI reporting delay is defined as the time between a command that will trigger an E-UTRA CGI report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty of 2 x TTIDCCH resulting when inserting the measurement report to the TTI of the uplink DCCH. This measurement reporting delay excludes any delay caused by lack of UL resources for UE to send the measurement report.

The CGI reporting delay shall be less than Tidentify_CGI, E-UTRAN plus RRC procedure delay defined in clause 12 in TS 38.331 [13], and an additional 30ms margin.

#### 6.6.7.1	SA intra-frequency CGI identification of NR neighbour cell in FR1

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing

6.6.7.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of intra-frequency CGI identification of an NR neighbour cell in FR1 with autonomous gaps. This test shall partly verify the measurement requirements in Clause 9.11 in TS 38.133 [6].

6.6.7.1.2	Test applicability

This test applies to all types of NR SA UE Rel-16 and forward supporting with PCell in FR1 and supporting acquisition of CGI from neighbour NR cell using autonomous gap, which is controlled by PICS pc_nr_CGI_Reporting.

6.6.7.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.7.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.7.1.

6.6.7.1.4	Test description

The test scenario comprises of one NR FR1 carrier. Two cells are deployed in the test, which are NR FR1 PCell (Cell 1) and NR FR1 neighbour cell (Cell 2) on NR RF channel 1.

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

6.6.7.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.7.1.4.1-1.

Table 6.6.7.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.7.1.4.1-2.

Table 6.6.7.1.4.1-2: Initial conditions for EN-DC PSCell FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.7.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | Figure A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | Figure A.3.2.3.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The test parameters for NR Cells are given in Table 6.6.7.1.4.1-3 below. The test parameters and applicability for the E-UTRAN PCell are defined in Table A.3.7.2.1-1 in TS 38.133 [6]. Cell-specific parameters of NR PSCell are specified in Table 6.6.7.1.5-1.

2.	Message contents are defined in clause 6.6.7.1.4.3.

3.	There are one E-UTRAN cell and two NR cells specified in the test. E-UTRAN Cell 1 is the cell used for connection setup with the power level set according to clause C.1.1 and C.1.2 for this test.

Table 6.6.7.1.4.1-3: General test parameters for SA intra-frequency CGI identification of NR neighbour cell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.7.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.4. UE is connected to Cell 1 (PCell) on NR RF channel 1. NR Cell 2 is also on NR RF channel 1. Set the parameters according to Tables 6.6.7.1.4.1-3 and 6.6.7.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

2.	The SS starts sending PDCCHs indicating new transmissions continuously on Cell 1.

3.	T1 starts. At this moment, the UE does not have any timing information of Cell 2.

4.	After 5 seconds, T2 starts. A measurement object is configured for the frequency of the PCell and it is indicated to the UE that event-triggered reporting with Event A3 is used. The UE is expected to detect and send a measurement report with Event A3. A new RRC message triggering CGI identification shall be sent to the UE during period T2, after the UE has reported Event A3. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to TRUE.

5.	After 5 seconds, T3 starts. The start of T3 is the instant when the last TTI containing the RRC message implying CGI identification is sent to the UE.

6.	If the UE transmits a measurement report containing the cell global identifier of Cell 2 within 252 ms from the start of T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	If it is successful, continue to step 9. Otherwise continue to step 8.

8.	Switch the UE OFF and then ON. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

9.	Repeat steps 2-8 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.7.1.4.3	Message contents

TBD

6.6.7.1.5	Test requirement

Table 6.6.7.1.5-1 defines the cell specific test parameters.

Table 6.6.7.1.5-1: NR Cell specific test parameters for SA intra-frequency CGI identification of NR neighbour cell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 |  | T2 |  | T3 |
| TDD configuration |  | 1 | TN/A |  |  |  | TN/A |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  | N/A |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  |  | CCR.1.1 FDD |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |  |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  |  | OP.1 |  |  |  |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |  |  | N/A |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  | N/A |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  | N/A |  |  |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |  |  | DLBWP.1.1 |  |  |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |  |  | ULBWP.1.1 |  |  |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  |  | SSB |  |  |  |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 |  | -1.46 | -Infinity | -1.46 |  | -1.46 |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 |  | 4 | -Infinity | 4 |  | 4 |  |
|  |  | 2 |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 |  | -94 | -Infinity | -94 |  | -94 |  |
|  |  | 2 | -94 | -94 |  | -94 | -Infinity | -94 |  | -94 |  |
|  |  | 3 | -91 | -91 |  | -91 | -Infinity | -91 |  | -91 |  |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 |  | -62.25 | -64.60 | -62.25 |  | -62.25 |  |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 |  | -62.25 | -64.60 | -62.25 |  | -62.25 |  |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 |  | -56.16 | --58.50 | -56.16 |  | -56.16 |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

The UE shall send a measurement report containing the CGI of Cell 2 within 252 ms from the start of time period T3.

Test requirement 	= RRC Procedure delay + Tidentify_CGI + reporting delay

= 10 + 240 + 2ms from the start of T3

= 252 ms

The UE shall be scheduled continuously throughout the test. From the start of T3 until 252 ms, the interruption on PCell shall not be more than the values specified for SA in clause 8.2.2.2.14 in TS 38.133 [6].

The rate of correct events observed during repeated tests shall be at least 90%.

##### 6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing

6.6.7.2.1	Test purpose

This test is to verify the requirement for identification of a new CGI of E-UTRA cell with autonomous gaps in NR SA in clause 9.4.7 in TS 38.133 [6].

6.6.7.2.2	Test applicability

This test applies to all types of NR SA UE Rel-16 and forward supporting with PCell in FR1 and supporting acquisition of CGI from neighbour E-UTRA cell using autonomous gap, which is controlled by PICS pc_eutra_CGI_Reporting.

6.6.7.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.7.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.7.2.

6.6.7.2.4	Test description

The test scenario comprises of one NR FR1 carrier and one E-UTRA carrier. Two cells are deployed in the test, which are NR FR1 PCell (Cell 1) and E-UTRA neighbour cell (Cell 2).

The test consists of three successive time periods, with time durations of T1, T2 and T3 respectively. At the start of time duration T1, the UE does not have any timing information of Cell 2. Starting T2, Cell 2 becomes detectable and the UE is expected to detect and send a measurement report.

6.6.7.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.7.2.4.1-1.

Table 6.6.7.2.4.1-1: Supported test configurations of inter-RAT E-UTRAN cell using autonomous gap in SA

| Configuration | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.7.2.4.1-2.

Table 6.6.7.2.4.1-2: Initial conditions for EN-DC PSCell FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.7.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | Figure A.3.1.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | Figure A.3.2.3.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameters are given in Table 6.6.7.2.4.1-3 below. Cell-specific parameters of NR and E-UTRA are specified in Table 6.6.7.2.5-1, and 6.6.7.2.5-2, respectively.

2.	Message contents are defined in clause 6.6.7.2.4.3.

3.	There are one NR cell and one E-UTRAN cell specified in the test. NR Cell is the cell used for connection setup with the power level set according to clause C.1.1 and C.1.2 for this test.

Table 6.6.7.2.4.1-3: General test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Active cell |  | Cell 1 |  |
| Neighbour cell |  | Cell 2 | Cell to be identified. |
| LTE Channel Bandwidth | MHz | 10 |  |
| LTE PDSCH/PCFICH/PDCCH/PHICH parameters |  |  | As specified in clause A.3.7.2.1 in TS 38.133 [6] |
| CP length |  | Normal |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF |  |
| useAutonomousGaps |  | TRUE | As specified in clause 5.5.3.1 in TS 38.331 [13]. |
| Time offset between cells | ms | 3 | Asynchronous cells |
| T1 | s | 5 |  |
| T2 | s | 10 |  |
| T3 | s | 5 |  |

6.6.7.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.4. UE is connected to Cell 1 (PCell) on NR RF channel 1. E-UTRA Cell 2 is also on RF channel 1. Set the parameters according to Tables 6.6.7.2.4.1-3 and 6.6.7.2.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

2.	The SS starts sending PDCCHs indicating new transmissions continuously on Cell 1.

3.	T1 starts. At this moment, the UE does not have any timing information of Cell 2.

4.	After 5 seconds, T2 starts. A RRC message implying SI reading shall be sent to the UE during period T2, after the UE has reported Event B2. The RRC message shall create a measurement report configuration with purpose reportCGI and useAutonomousGaps set to TRUE.

5.	The start of T3 is the instant when the last TTI containing the RRC message implying SI reading is sent to the UE.

6.	If the UE transmits a measurement report containing the cell global identifier of Cell 2 within 200 ms from the start of T3, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	If it is successful, continue to step 9. Otherwise continue to step 8.

8.	Switch the UE OFF and then ON. Ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.4.

9.	Repeat steps 2-8 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.7.2.4.3	Message contents

TBD

6.6.7.2.5	Test requirement

Table 6.6.7.2.5-1 and 6.6.7.2.5-2 define the cell specific test parameters.

Table 6.6.7.2.5-1: PCell specific test parameters for identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR

| Parameter |  |  | Unit | Configuration | Cell 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T3 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |  |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |  |  |
|  |  |  |  | 4, 5, 6 | TDD |  |  |  |
| TDD Configuration |  | SCS=15 kHz |  | 2, 5 | TDDConf.1.1 |  |  |  |
|  |  | SCS=30 kHz |  | 3, 6 | TDDConf.2.1 |  |  |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |  |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |  |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |  |  |
| CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |  |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |  |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |  |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |  |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |  |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |  |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |  |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |  |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |  |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |  |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |  |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -98 |  |  |  |
|  |  |  |  | 3, 6 | -95 |  |  |  |
| b2-Threshold2EUTRA |  |  | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -109 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -106 |  |  |  |
|  |  |  |  | 3, 6 | -103 |  |  |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 18 |  | -2 |  |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  |
|  |  |  |  | 3, 6 | -85 |  | -105 |  |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -88 |  | -108 |  |
|  |  |  |  | 3, 6 | -85 |  | -105 |  |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -59.98 |  | -75.92 |  |
|  |  |  | dBm/38.16 MHz | 3, 6 | -53.88 |  | -69.82 |  |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table 6.6.7.2.5-2: Cell specific test parameters for inter-RAT E-UTRAN cell for identification of a new CGI of E-UTRA cell using autonomous gaps

| Parameter | Unit | Configuration | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |  |
|  |  | 4, 5, 6 | TDD |  |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 10 MHz: NRB,c = 50 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.3 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.0 TDD |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 10 MHz: R.6 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: R.6 TDD |  |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 10 MHz: OP.10 FDD |  |  |
|  |  | 4, 5, 6 | 10 MHz: OP.1 TDD |  |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -106 |  |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 7 | 7 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -99 | -99 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 Low |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |

The UE shall transmit a measurement report containing the cell global identifier of Cell 2 within 200 milliseconds from the start of T3.

Test requirement = RRC Procedure delay with additional margin + Tidentify_CGI,E-UTRAN + reporting delay

= 15 + 30 + 150 + 2ms from the start of T3

= 197 ms, allow 200 ms.

-	The UE shall be scheduled continuously throughout the test, and from the start of T3 until 200 ms at least the number of ACK/NACK specified in NOTE 2 shall be detected as being transmitted by the UE.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

NOTE 2:	The overall ACK/NACK number is caused by two parts. Firstly, at least X ACK/NACK shall be sent during identifying the cell global identifier of Cell 2, where X is defined in Table 8.2.2.2.15-1. Secondly, given that continuous DL data allocation, additional 43, 14 and 34 ACK/NACK shall be sent for FDD 15 kHz, TDD 15 kHz and TDD 30 kHz, respectively, from the start of T3 until 200 ms excludes 150 ms for identifying the cell global identifier of Cell 2.

### 6.6.8	L1-SINR measurement for beam reporting

#### 6.6.8.0	Minimum conformance requirements

6.6.8.0.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

Same as clause 4.6.7.0.1

The normative reference for this requirement is TS 38.133 [6] clause 9.8.3, 9.8.4.1 and 9.8.5.2.

6.6.8.0.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured

Same as clause 4.6.7.0.2

The normative reference for this requirement is TS 38.133 [6] clause 9.8.3, 9.8.4.2 and 9.8.5.

6.6.8.0.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

Same as clause 4.6.7.0.3

The normative reference for this requirement is TS 38.133 [6] clause 9.8.3, 9.8.4.3 and 9.8.5.

#### 6.6.8.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR L1-SINR measurement in DRX

6.6.8.1.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.1.

6.6.8.1.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting L1-SINR measurement and long DRX cycle.

6.6.8.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.8.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.8.1.

6.6.8.1.4	Test description

6.6.8.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.8.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.8.1.4.1-2. Test environment parameters are given in Table 6.6.8.1.4.1-3.

Table 6.6.8.1.4.1-1: NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.8.1.4.1-2: General test parameters for NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | DRX.3 |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.8.1.4.1-3: Test Environment parameters for NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.8.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.5 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.8.1.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.8.1.4.1-2 and 6.6.8.1.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.8.1.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. UE is also configured to measure L1-SINR based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in Table 6.6.8.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.8.1.5-1. T1 starts.

3.	After 80ms from the start of the test the SS transmits the DCI trigger in slot 1 for configuration 1, 2 and slot 8 for configuration 3. The corresponding CSI-RS set is transmitted with the offset of 4 slots after the DCI trigger.

4.	The SS shall check following requirements:

- R1:	the UE shall send L1-SINR report at slot 26 from the reception of DCI trigger. If the report is received at slot 26 from the reception of DCI trigger, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	The L1-SINR value of CSI-RS#1 reported by the UE is compared to the expected L1-SINR value for CSI-RS#1. If the resulting value is outside the limits in Table 6.6.8.1.5-2 for all test configurations or the UE fails to report the measurement value for CSI-RS #1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3:	The DIFF SINR value of CSI-RS #0 reported by the UE is compared to the expected DIFF SINR value. If the resulting value is outside the limits in Table 6.6.8.1.5-4 or the UE fails to report the measurement value for CSI-RS#0, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

5.	If after T1 expiry no report is received or received report did not contain L1-SINR of both CSI-RS#0 and CSI-RS#1 or UE sent the L1-SINR report at different slot than 26 from the reception of DCI trigger, the number of ‘failed’ iterations is increased by one, otherwise, the number of ‘passed’ iterations is increased by one.

6.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.8.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.8.1.4.3-1: Common Exception messages NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions APERIODIC and CSI-SINR Table H.3.6A-2 with conditions CSI-RS and APERIODIC Table H.3.7-1 with condition DRX.3 |

Table 6.6.8.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.6.8.1.5	Test requirement

Table 6.6.8.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.8.1.5-1: CSI-RS specific test parameters for NR SA CSI-RS based CMR and no dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| NocNote1 | 1~3 | dBm/15kHz | -94.65 |  |
| NocNote1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
| EsIot | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
| EsNoc | 1~3 | dB | 0 | 3 |
| Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 and CSI-RS#1.

Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.8.1.5-2 for all test configurations and the corresponding relative accuracy requirements in Table 6.6.8.1.5-4 for all test configurations.

Table 6.6.8.1.5-2: L1-SINR absolute accuracy requirements for the reported values for all test configurations

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 41 |
| Highest reported value (CSI-RS#1) | 64 |

Table 6.6.8.1.5-3: Void

Table 6.6.8.1.5-4: L1-SINR relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF SINR reported (CSI-RS#0) | 0 |
| Highest DIFF SINR reported (CSI-RS#0) | 7 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX

6.6.8.2.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in non-DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.2.

6.6.8.2.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting L1-SINR measurement.

6.6.8.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.8.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.8.2.

6.6.8.2.4	Test description

6.6.8.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.8.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.8.2.4.1-2. Test environment parameters are given in Table 6.6.8.2.4.1-3.

Table 6.6.8.2.4.1-1: NR SA SSB based CMR and dedicated IMR L1-SINR measurement supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.8.2.4.1-2: General test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | c |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.1A FDD |
|  | 2 |  | CSI-RS 1.1A TDD |
|  | 3 |  | CSI-RS 2.1A TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity-r16 | 1~3 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| L1-SINR reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.8.2.4.1-3: Test Environment parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.8.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.5 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.8.2.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.8.2.4.1-2, 6.6.8.2.5-1 and 6.6.8.2.5-2. UE is configured to perform RLM and BFD based on the SSBs.

6.6.8.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above Table 6.6.8.2.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-SINR measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.8.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2. T1 starts. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2. T2 starts. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

5.	If the UE sends L1-SINR reports meeting the corresponding absolute accuracy requirements in Table 6.6.8.2.5-3 for test configurations 1 and 2 the corresponding absolute accuracy requirements in Table 6.6.8.2.5-4 for test configurations 3 and the corresponding relative accuracy requirements in Table 6.6.8.2.5-5 for all test configurations every 80 slots from no later than 720 ms for configuration 1 and 2 and no later than 680 ms for configuration 3 from the beginning of time period T2 until the end of time period T2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one. The UE shall start sending valid L1-SINR reports. The SS shall check following requirements:

- R1:	the UE shall start to transmit valid reports no later than 720 ms for configuration 1,2  and no later than 680 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-SINR requirement for SSB#1 (Table 6.6.8.2.5-3 for all configurations) and the relative L1-SINR requirement for SSB#0 in Table 6.6.8.2.5-5. If the first valid report is received earlier than the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	the UE shall transmit reports every 80 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

- R3:	The L1-SINR value of SSB#1 reported by the UE is compared to the expected L1-SINR value for SSB#1. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.8.2.5-3 for all configurations or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

-R4:	The DIFF SINR value of SSB#0 reported by the UE is compared to the expected DIFF SINR value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table6.6.8.2.5-5 for all configurations or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R4 is increased by one. Otherwise, the number of passed iterations for R4 is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.8.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.8.2.4.3-1: Common Exception messages NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and SS-SINR and CSI-RS_IMR Table H.3.6A-2 with conditions SSB and PERIODIC Table H.3.6A-3 with condition PERIODIC |

Table 6.6.8.2.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.8.2.5	Test requirement

Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.8.2.5-1: SSB specific test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Noc Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Noc Note2 | 1,2 | dBm/SSB SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
| EsIot | 1~3 | dB | 0.5 | 0.5 | -Infinity | 3 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.15 | -94.15 | -Infinity | -91.65 |
|  | 3 |  | -91.14 | -91.14 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.43 | -63.43 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.33 | -57.33 | -60.61 | -55.84 |
| EsNoc | 1~3 | dB | 0.5 | 0.5 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.8.2.5-2: CSI-RS specific test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 |  | CSI-RS#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Noc Note2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| Noc Note2 | 1,2 | dBm/CSI-RS SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
| EsIot | 1~3 | dB | 0.5 | 0.5 | -Infinity | 3 |
| CSI-RS RSRP Note3 | 1,2 | dBm/CSI-RS SCS | -94.15 | -94.15 | -Infinity | -91.65 |
|  | 3 |  | -91.14 | -91.14 | -Infinity | -88.65 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.43 | -63.43 | -66.70 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.33 | -57.33 | -60.61 | -55.84 |
| EsNoc | 1~3 | dB | 0.5 | 0.5 | -Infinity | 3 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-SINR report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-SINR report including results of both SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1. Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.8.2.5-3 for all test configurations and the corresponding relative accuracy requirements in Table 6.6.8.2.5-4 for all test configurations.

Table 6.6.8.2.5-3: L1-SINR absolute accuracy requirements for the reported values for all test configurations

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | 44 |
| Highest reported value (SSB#1) | - | 61 |

Table 6.6.8.2.5-4: Void

Table 6.6.8.2.5-5: L1-SINR relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF SINR reported (SSB#0) | - | 0 |
| Highest DIFF SINR reported (SSB#0) | - | 5 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R4) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.8.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement in non-DRX

6.6.8.3.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in non-DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.3.

6.6.8.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting L1-SINR measurement.

6.6.8.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.8.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.8.3.

6.6.8.3.4	Test description

6.6.8.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.8.3.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.8.3.4.1-2. Test environment parameters are given in Table 6.6.8.3.4.1-3.

Table 6.6.8.3.4.1-1: NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.8.3.4.1-2: General test parameters for NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.3 FDD |
|  | 2 |  | CSI-RS 1.3 TDD |
|  | 3 |  | CSI-RS 2.3 TDD |
| CSI-IM configuration | 1 |  | CSI-IM.1.2 FDD |
|  | 2 |  | CSI-IM.1.2 TDD |
|  | 3 |  | CSI-IM.2.2 TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| DRX configuration | 1~3 |  | Off |
| reportConfigType | 1~3 |  | aperiodic |
| reportQuantity-r16 | 1~3 |  | cri-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| qcl-Info | 1~3 |  | SSB#0 for resource#0 |
|  |  |  | SSB#1 for resource#1 |
| reportSlotOffsetList | 1~3 | slots | 26 |
| T1 | 1~3 | s | 5 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.8.3.4.1-3: Test Environment parameters for NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.8.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.5 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.8.3.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.8.3.4.1-2 and 6.6.8.3.5-1. UE is configured to perform RLM and BFD based on the SSBs.

6.6.8.3.4.2	Test procedure

The test consists of a single time period T1, during which the UE is triggered via DCI to report L1-SINR on aperiodic CSI-RS resources. Prior to the start of the time duration T1, the UE shall be fully synchronized to PCell. UE is also configured to measure L1-SINR based on SSB. Upon receiving the DCI trigger, UE provides the report back based on the reporting configuration as defined in table 6.6.8.3.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.8.3.5-1. T1 starts.

3.	After 80ms from the start of the test the SS transmits the DCI trigger in slot 1 for configuration 1, 2 and slot 8 for configuration 3. The corresponding CSI-RS set is transmitted with the offset of 4 slots after the DCI trigger.

4.	The SS shall check following requirements:

- R1:	the UE shall send L1-SINR report at slot 26 from the reception of DCI trigger. If the report is received at slot 26 from the reception of DCI trigger, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

- R2:	The L1-SINR value of CSI-RS#1 reported by the UE is compared to the expected L1-SINR value for CSI-RS#1. If the resulting value is outside the limits in Table 6.6.8.3.5-2 for all test configurations or the UE fails to report the measurement value for CSI-RS #1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

- R3:	The DIFF SINR value of CSI-RS #0 reported by the UE is compared to the expected DIFF SINR value. If the resulting value is outside the limits in Table 6.6.8.3.5-4 or the UE fails to report the measurement value for CSI-RS#0, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.5.	If after T1 expiry no report is received or received report did not contain L1-SINR of both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR or UE sent the L1-SINR report at different slot than 26 from the reception of DCI trigger, the number of ‘failed’ iterations is increased by one, otherwise, the number of ‘passed’ iterations is increased by one.

6.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

7.	After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On  according to TS 38.508-1 [14] clause 4.5.

8.	Repeat steps 2-7 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.8.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.8.3.4.3-1: Common Exception messages NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions APERIODIC and CSI-SINR and CSI-IM_IMR Table H.3.6A-2 with conditions CSI-RS and APERIODIC Table H.3.6A-4 with condition APERIODIC |

Table 6.6.8.3.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.6.8.3.5	Test requirement

Table 6.6.8.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.8.3.5-1: CSI-RS specific test parameters for NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 | CSI-RS#1 |
| --- | --- | --- | --- | --- |
| NocNote1 | 1~3 | dBm/15kHz | -94.65 |  |
| NocNote1 | 1,2 | dBm/SSB SCS | -94.65 |  |
|  | 3 |  | -91.65 |  |
| EsIot | 1~3 | dB | 0 | 3 |
| CSI-RS RSRP Note2 | 1,2 | dBm/SSB SCS | -94.65 | -91.65 |
|  | 3 |  | -91.65 | -88.65 |
| Io Note2 | 1,2 | dBm/9.36 MHz | -63.69 | -61.93 |
|  | 3 | dBm/38.16 MHz | -57.59 | -55.84 |
| EsNoc | 1~3 | dB | 0 | 3 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: CSI-RS RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

After 80ms from the beginning of the test, the UE shall send L1-SINR report at slot 26 from the reception of DCI triggering the L1-SINR measurement. The L1-SINR report shall include the results for both CSI-RS#0 as CMR + CSI-IM#0 as IMR and CSI-RS#1 as CMR + CSI-IM#1 as IMR.

Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.8.3.5-2 for all test configurations and the corresponding relative accuracy requirements in Table 6.6.8.3.5-4 for all test configurations.

Table 6.6.8.3.5-2: L1-SINR absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 |
| --- | --- |
| Lowest reported value (CSI-RS#1) | 43 |
| Highest reported value (CSI-RS#1) | 62 |

Table 6.6.8.3.5-3: Void

Table 6.6.8.3.5-4: L1-SINR relative accuracy requirements for the reported values for all test configurations

|  | T1 |
| --- | --- |
| Lowest DIFF SINR reported (CSI-RS#0) | 0 |
| Highest DIFF SINR reported (CSI-RS#0) | 6 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.9	Idle Mode CA/DC Measurements

#### 6.6.9.0	Minimum conformance requirements

##### 6.6.9.0.1	Idle Mode CA/DC Measurements

For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement reporting provided T331 has not expired, the serving cell is supporting idle mode CA/DC measurement reporting and the serving cell is in the validity area.

This subclause defines the requirements for the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an NE-DC and NR carrier aggregation capable UE which has been configured with one or more of following, one or more SCells, one E-UTRAN PSCell or one or more downlink E-UTRAN SCells during the Connected mode and which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16. The requirements are applicable for SCell(s) and E-UTRAN FDD and TDD PSCell and SCells.

Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signalling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signalling, and

-	The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report, and

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-RAT E-UTRAN cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in TS 36.133 [23] Annex B.1.1 and Annex B.1.2 for a corresponding Band. An inter-frequency cell is considered detectable according to the conditions in TS 38.133 [6] Annex B.1.2 and B.1.3 for a corresponding band. An SSB of an inter-frequency cell is considered detectable according to SSB_RP and SSB Ês/Iot defined in TS 38.133 [6] Annex B.1.2 and B.1.3 for a corresponding Band.

While T331 is running, the UE shall perform measurement on the configured inter-frequency carriers for idle mode CA/DC measurement reporting according to the UE measurement capability.

A UE which supports idleInactiveNR-MeasReport-r16 shall support idle mode CA/DC measurements of:

-	at least 7 inter-frequency carriers which are also configured for inter-frequency mobility measurements, and

-	at least 7 inter-frequency carriers which are not configured for inter-frequency mobility measurements.

The UE shall be capable of monitoring a total of at least 7 inter-frequency carriers for idle mode CA/DC measurements comprising of carriers configured for inter-frequency mobility measurements and carriers not configured for inter-frequency mobility measurements.

For inter-frequency carriers configured for idle mode CA/DC measurements, if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ the inter-frequency measurement requirements in TS 38.133 [6] clause 4.2.2.4 shall apply, where UE shall search for and measure inter-frequency layers configured for idle mode CA/DC measurements in preparation for possible reporting. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements at least every Thigher_priority_search where Thigher_priority_search is described in TS 38.133 [6] clause 4.2.2.7, where UE shall search for and measure inter-frequency layers configured for idle mode CA/DC measurements in preparation for possible reporting.

For UE supporting idleInactiveNR-MeasBeamReport-r16, if the UE is configured with beamMeasConfigIdle-r16 for idle mode CA/DC measurement, the UE shall be capable of performing SS-RSRP, SS-RSRQ for at least

-	7 SSBs with different SSB index and/or PCI on an inter-frequency layer in FR1,

-	10 SSBs with different SSB index and/or PCI on an inter-frequency layer in FR2.

For UE supporting idleInactiveNR-MeasBeamReport-r16, if the UE is configured with beamMeasConfigIdle-r16 for idle mode CA/DC measurement, the UE shall be able to acquire the SSB index for a newly detectable inter-RAT NR cell and perform RSRP/RSRQ measurement within the requirements defined in TS 38.133 [6] clause 4.2.2.4 plus TSSB_index,NR, where TSSB_index,NR is the additional time period used to acquire the index of the SSB being measured as defined in Table 6.6.9.0.1-1.

Table 6.6.9.0.1-1: TSSB_index,NR_Inter

| DRX cycle length [s] | Scaling Factor (N1) |  | TSSB_index,NR_Inter [s] (number of DRX cycles) |
| --- | --- | --- | --- |
|  | FR1 | FR2Note1 |  |
| 0.32 | 1 | 8 | N2 x 1.28 x N1 x 1.5 (N2 x 4 x N1 x 1.5) |
| 0.64 |  | 5 | N2 x 1.28 x N1 (N2 x 2 x N1) |
| 1.28 |  | 4 | N2 x 1.28 x N1 (N2 x 1 x N1) |
| 2.56 |  | 3 | N2 x 2.56 x N1 (N2 x 1 x N1) |
| Note 1: Applies for UE supporting power class 2&3&4. For UE supporting power class 1, N1 = 8 for all DRX cycle length. NOTE 2: N2 = 3 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1, and N2 = 5 if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR2. |  |  |  |

In the absence or expiration of T331, it is up to UE implementation to perform the idle mode CA/DC measurement.

For inter-frequency carriers configured for idle mode CA/DC measurements, the UE shall be capable of performing SS-RSRP and SS-RSRQ measurements of the carriers, and the UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the carriers configured for idle mode CA/DC measurements to higher layers, with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.4B, 10.1.5B, 10.1.9B and 10.1.10B, respectively.

The UE shall be able to report idle mode CA/DC measurements when idle mode CA/DC measurement reporting is requested by the network.

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.133 [6] clause 4.2.2.2 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.2B, 10.1.3B, 10.1.7B and 10.1.8B.

##### 6.6.9.0.2	Measurement report for fast CA/DC setup

The requirements of measurement report for fast CA/DC setup apply in this clause for:

-	UE supporting measValidationReportEMR-r18 and configured with measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 by higher layers.

-	UE supporting measValidationReportReselectionMeasurements-r18 and configured with measReselectionCarrierListNR-r18 by higher layers.

A UE supporting measValidationReportEMR-r18  and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report the results of the idle mode measurement which is specified in clause 6.6.9.0.1, and according to the measurement reporting requirements specified below.

A UE supporting measValidationReportReselectionMeasurements-r18 shall report the results of the idle mode measurement specified in TS 38.133 [6] clause 4.2.2, and according to the measurement reporting requirements specified below.

UE is not required to report for fast CA/DC setup from any measurements during IDLE mode for carriers which are not configured by higher layers.

A UE shall perform validity check and report valid measurement results:

-	if the UE supports measValidationReportEMR-r18 and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16,

-	if the UEsupports measValidationReportReselectionMeasurements-r18 and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18

The measurement results are considered valid if the following conditions are met for the validity check:

-	the measurements are performed before msg1 transmission for RRC resume/setup request within the last:

-	 measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or

-	 measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18,

-	the measurement results satisfy measurement accuracy requirement at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured.

If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

RSRP, RSRQ measurements contained in the measurement reports shall meet the corresponding accuracy requirements at the measurement instance specified in TS 38.133 [6] clauses 10.1.2B, 10.1.3B, 10.1.4B, 10.1.5B, 10.1.7B, 10.1.8B, 10.1.9B, 10.1.10B.

#### 6.6.9.1	NR SA FR1 Idle mode CA/DC measurement for FR1

6.6.9.1.1	Test purpose

The purpose of this test is to verify that the UE performs the required measurements on the serving cell and the configured inter-frequency carrier for idle mode measurement reporting after the UE has entered Idle mode. This test will partly verify the Idle mode CA/DC measurements requirements in TS 38.133 [6] clause 4.4.

6.6.9.1.2	Test applicability

This test applies to all types of NR UE release 16 onwards, supporting 2DL CA and idleInactiveNR-MeasReport-r16.

6.6.9.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.9.0.1.

The normative reference for this requirement is TS 38.133 [6] clause 4.4and A.6.6.9.1

6.6.9.1.4	Test description

6.6.9.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.9.1.4.1-1.

Table 6.6.9.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.9.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.9.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.9.1-3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.6.9.1.4.1-2.

Table 6.6.9.1.4.1-2: Initial conditions for SA Idle mode CA/DC measurement for FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.9.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameter settings are set up according to Table 6.6.9.1.4.1-3.

2. Message contents are defined in clause 6.6.9.1.4.3.

3. The test scenario comprises of two NR Cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.9.1.4.1-3: General test parameters for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 300 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells |
| T1 | s | Config 1,2,3 | 10 |  |
| T2 | s | Config 1,2,3 | 11.52 |  |
| T3 | s | Config 1,2,3 | 10 |  |

6.6.9.1.4.2	Test procedure

Two cells are deployed in the test, one FR1 NR PCell (Cell 1) on RF channel 1 and an FR1 NR neighbour cell (Cell 2) on RF channel 2. The test consists of 3 successive time periods, with time duration of T1, T2, and T3 respectively. During T1, UE is connected to Cell 1 and has not any timing information about Cell 2. UE is configured with early measurement reporting with channel 2. Beam level reporting for early measurements is not configured.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.9.1.5-1. T1 starts.

3. When T1 expires, the SS shall transmit an RRCRelease message including measIdleCarrierListNR-r16 (Table 6.6.9.1.4.3-3) and switch the parameters according to T2 in Table 6.6.9.1.5-2. T2 starts.

4. Ensure the UE is in state RRC_IDLE until T2 expires. T3 starts.

6. The SS transmits in Cell a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE is in State RRC_CONNECTED according to TS 38.508-1 [14]clause 4.5.

7. During the connection setup the UE is reports idleMeasAvailable-r16 in the RRCSetupComplete message.

8. TE shall send the UEInformationRequest message, requesting UE to provide ide mode measurements results (Table 6.6.9.1.4.3-5).

9. If UE responds with UEInformationResponse message containing the idle mode measurement results with SS-RSRP values within the limits in Table 6.6.9.1.5-3 for configuration 1 and 2 and Table 6.6.9.1.5-4 for configuration 3, and SS-RSRQ values within the limits in Table 6.6.9.1.5-5 for configuration 1, 2 and 3, the number of passed iterations is increased by one. Otherwise, the number of failed iterations is increased by one.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.9.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.9.1.4.3-1: Common Exception messages for SA Idle mode CA/DC measurement for FR1

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.6.9.1.4.3-2: SIB1 for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMeasurementsNR-r16 | TRUE | This field indicates that a UE that is configured for NR idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform NR idle/inactive measurements. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.9.1.4.3-3: RRCRelease for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 ::= SEQUENCE { |  | Serving cell not included |  |
| MeasIdleCarrierNR-r161 ::= SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN value of NR channel 2 |  |  |
| ssbSubcarrierSpacing-r16 | kHz15 | Config 1,2 |  |
|  | kHz30 | Config 3 |  |
| frequencyBandList | Not present |  |  |
| measCellListNR-r16 | Not present |  |  |
| reportQuantities-r16 | both |  |  |
| qualityThreshold-r16 | Not present |  |  |
| ssb-MeasConfig-r16 SEQUENCE { |  |  |  |
| nrofSS-BlocksToAverage-r16 | Not present |  |  |
| absThreshSS-BlocksConsolidation-r16 | Not present |  |  |
| smtc-r16 | SSB-MTC | TS 38.508-1 [14], Table 7.3.1-3 |  |
| ssb-ToMeasure-r16 | SSB-ToMeasure | TS 38.508-1 [14], Table 7.3.1-31 |  |
| deriveSSB-IndexFromCell-r16 | false |  |  |
|  | true |  | FR1_TDD, FR2_TDD |
| ss-RSSI-Measurement-r16 | Not present |  |  |
| } |  |  |  |
| beamMeasConfigIdle-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleCarrierListEUTRA-r16 | Not present |  |  |
| measIdleDuration-r16 | sec300 |  |  |
| validityAreaList-r16 ::= SEQUENCE { |  |  |  |
| ValidityArea-r16 ::= SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN value of NR channel 1 |  |  |
| validityCellList-r16 ::= SEQUENCE { |  |  |  |
| PCI-Range | PhysCellId of Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.9.1.4.3-4: RRCSetupComplete for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-22: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| idleMeasAvailable-r16 | TRUE | Indication that the UE has idle/inactive measurement report available. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.9.1.4.3-5: UEInformationRequest

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-32A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r16 | TRUE | UE shall report the idle/inactive measurement information, if available, to the network in the UEInformationResponse message. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.9.1.4.3-6: UEInformationResponse

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleNR-r16 ::= SEQUENCE { |  |  |  |
| rsrp-Result-r16 | INTEGER(0..127) | Integer value for RSRP measurements is according to Table 10.1.6.1-1 in TS 38.133 [6] |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.9.1.5	Test requirements

Tables 6.6.9.1.5-1 and 6.6.9.1.5-2 define the primary level settings including test tolerances and tables 6.6.9.1.5-3, 6.6.9.1.5-4, 6.6.9.1.5-5 and 6.6.9.1.5-6 define accuracy requirements define for SA Idle mode CA/DC measurement for FR1.

Table 6.6.9.1.5-1: Cell specific test parameters for connected mode for SA Idle mode CA/DC measurement for FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  | SR2.1 TDD |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  |  | -98 |  |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  |  | Config 3 | -88 | -88 | -88 | -infinity | -95 | -95 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 7 | 7 | 7 | - infinity | 0 | 0 |
| EsNoc |  | dB | Config 1,2,3 | 7 | 7 | 7 | infinity | 0 | 0 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  |  | dBm/38.16MHz | Config 3 | -56.15 | -56.15 | -56.15 | -63.94 | -60.93 | -60.93 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

Table 6.6.9.1.5-2: Cell specific test parameters for idle mode for SA Idle mode CA/DC measurement for FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1,2,3 | 1 |  |  | 2 |  |  |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| configuration |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| RMC configuration |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
| EsIot | dB | 1 | 14 | 14 | 14 | -infinity | 12 | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Noc Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Noc Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| EsNoc | dB | 1 | 7 | 7 | 7 | -infinity | 0 | 0 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  | 2 | -91 | -91 | -91 | -infinity | -98 | -98 |
|  |  | 3 | -88 | -88 | -88 | -infinity | -95 | -95 |
| Io | dBm/9.36 MHz | 1 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  | dBm/9.36 MHz | 2 | -62.26 | -62.26 | -62.26 | -70.5 | -67.04 | -67.04 |
|  | dBm/38.16 MHz | 3 | -56.15 | -56.15 | -56.15 | -63.94 | -60.93 | -60.93 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2, 3 | Not sent |  |  | Not sent |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table 6.6.9.1.5-3: Absolute accuracy requirements for Cell 1 and Cell 2 SS-RSRP reported values for test configurations 1 and 2

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 1) | 54 |
| Highest reported value (Cell 1) | 77 |
| Lowest reported value (Cell 2) | 47 |
| Highest reported value (Cell 2) | 70 |

Table 6.6.9.1.5-4: Absolute accuracy requirements for Cell 1 and Cell 2 SS-RSRP reported values for test configurations 3

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 1) | 57 |
| Highest reported value (Cell 1) | 80 |
| Lowest reported value (Cell 2) | 50 |
| Highest reported value (Cell 2) | 73 |

Table 6.6.9.1.5-5: Absolute accuracy requirements for Cell 1 and Cell 2 SS-RSRQ reported values for test configurations 1, 2 and 3

| Normal Conditions | T3 |
| --- | --- |
| Lowest reported value (Cell 1) | 55 |
| Highest reported value (Cell 1) | 72 |
| Lowest reported value (Cell 2) | 51 |
| Highest reported value (Cell 2) | 67 |

During the time period T2 the UE is in Idle mode and the signal level of Cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to Section 4.4.

At the start of T3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in Section 10.1.4B for SS-RSRP and 10.1.8B for SS-RSRQ and test ends.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

#### 6.6.9.2	NR SA FR1 Idle mode fast CA/DC eEMR measurement

Editor's Note: This test case is incomplete in following aspects:

Test procedure is missing

Message contents are FFS

Test case applicability is missing in 38.522

Test Tolerance is not applied

6.6.9.2.1	Test purpose

To verify the UE supporting measValidationReportEMR-r18 makes correct reporting. This test will partly verify the he fast CA/DC measurement reporting requirements in TS 38.133 [6] clause clause 4.7.

6.6.9.2.2	Test applicability

This test applies to all types of NR UE supporting measValidationReportEMR-r18 from release 18 and forward.

6.6.9.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.9.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.9.2.

6.6.9.2.4	Test description

6.6.9.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.9.2.4.1-1

Table 6.6.9.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.9.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.9.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.9.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.6.9.2.4.1-2.

Table 6.6.9.2.4.1-2: Initial conditions for NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.14.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

1. 	The test scenario comprises of two NR Cells. NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as inter-frequency neighbour cell in FR1 on NR RF channel 2. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.2 for this test.

2.	The general test parameter settings are in Table 6.6.9.2.4.1-3.

Table 6.6.9.2.4.1-3: General test parameters for TBD

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| T331 | s |  | 30 |  |
| measIdleValidityDuration | s |  | 10 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 10 |  |
| T4 | s | Config 1,2,3 | 1 |  |

6.6.9.2.4.2	Test procedure

TBD

6.6.9.2.4.3	Message contents

TBD

6.6.9.2.5	Test requirements

Table 6.6.9.2.5-1 defines the primary level settings for all tests.

Table 6.6.9.2.5-1: NR Cell specific test parameters for TBD

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98+TT |  |  |  | -98+TT |  |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  |  |  | -98+TT |  |  |  |  |
|  |  |  | Config 3 | -95+TT |  |  |  | -95+TT |  |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91+TT |  |  |  | -infinity | -98+TT | infinity | infinity |
|  |  |  | Config 3 | -88+TT |  |  |  | -infinity | -95+TT | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7+TT |  |  |  | - infinity | 0+TT | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7+TT |  |  |  | infinity | 0+TT | infinity | infinity |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -62.26+TT |  |  |  | -70.05+TT | -67.04+TT | -70.05+TT | -70.05+TT |
|  |  | dBm/38.16 MHz | Config 3 | -56.15+TT |  |  |  | -63.96+TT | -60.9+TT 3 | -63.96+TT | -63.96+TT |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |  |

During the period T2 and T3, the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2.

The UE shall NOT send early measurement report to the PCell in this test.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 6.6.9.3	NR SA FR1 Idle mode fast CA/DC cell reselection measurement without valid reporting

Editor's Note: This test case is incomplete in following aspects:

Test procedure is missing

Message contents are FFS

Test case applicability is missing in 38.522

Test Tolerance is not applied

6.6.9.3.1	Test purpose

To verify the UE supporting measValidationReportReselectionMeasurements-r18 makes correct reporting. This test will partly verify the he fast CA/DC measurement reporting requirements in TS 38.133 [6] clause clause 4.7.

6.6.9.3.2	Test applicability

This test applies to all types of NR UE supporting measValidationReportReselectionMeasurements-r18 from release 18 and forward.

6.6.9.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.9.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.9.3.

6.6.9.3.4	Test description

6.6.9.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.9.3.4.1-1

Table 6.6.9.3.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.9.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.9.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.9.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.6.9.3.4.1-2.

Table 6.6.9.3.4.1-2: Initial conditions for TBD

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.14.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

1. 	The test scenario comprises of two NR Cells. NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as inter-frequency neighbour cell in FR1 on NR RF channel 2. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.2 for this test.

2.	The general test parameter settings are in Table 6.6.9.3.4.1-3.

Table 6.6.9.3.4.1-3: General test parameters for TBD

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| measReselectionValidityDuration-r18 | s |  | 5 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 5 |  |
| T4 | s | Config 1,2,3 | 1 |  |

6.6.9.3.4.2	Test procedure

TBD

6.6.9.3.4.3	Message contents

TBD

6.6.9.3.5	Test requirements

Table 6.6.9.3.5-1 defines the primary level settings for all tests.

Table 6.6.9.3.5-1: NR Cell specific test parameters for TBD

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98+TT |  |  |  | -98+TT |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  |  |  | -98+TT |  |  |  |
|  |  |  | Config 3 | -95+TT |  |  |  | -95+TT |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -91+TT |  |  |  | -infinity | -98+TT | infinity | infinity |
|  |  |  | Config 3 | -88+TT |  |  |  | -infinity | -95+TT | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7+TT |  |  |  | - infinity | 0+TT | infinity | infinity |
|  |  | dB | Config 1,2,3 | 7+TT |  |  |  | infinity | 0+TT | infinity | infinity |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -62.26+TT |  |  |  | -70.05+TT | -67.04+TT | -70.05+TT | -70.05+TT |
|  |  | dBm/38.16 MHz | Config 3 | -56.15 |  |  |  | -63.96+TT | -60.93+TT | -63.96+TT | -63.96+TT |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

During the period T2, the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2.

The UE shall NOT send early measurement report to the PCell in this test.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

#### 6.6.9.4	NR SA FR1 Idle mode fast CA/DC cell reselection measurement with valid reporting

Editor's Note: This test case is incomplete in following aspects:

Test procedure is missing

Message contents are FFS

Test case applicability is missing in 38.522

Test Tolerance is not applied

6.6.9.4.1	Test purpose

To verify the UE supporting measValidationReportReselectionMeasurements-r18 makes correct reporting. This test will partly verify the he fast CA/DC measurement reporting requirements in TS 38.133 [6] clause clause 4.7.

6.6.9.4.2	Test applicability

This test applies to all types of NR UE supporting measValidationReportReselectionMeasurements-r18 from release 18 and forward.

6.6.9.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.9.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.9.4.

6.6.9.4.4	Test description

6.6.9.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.9.4.4.1-1

Table 6.6.9.4.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.9.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.9.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.9.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.6.9.4.4.1-2.

Table 6.6.9.4.4.1-2: Initial conditions for TBD

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.5-1 and TS 38.508-1 [14] clause 7.2.3. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.6.14.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

1. 	The test scenario comprises of two NR Cells. NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as inter-frequency neighbour cell in FR1 on NR RF channel 2. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.2 for this test.

2.	The general test parameter settings are in Table 6.6.9.4.4.1-3.

Table 6.6.9.4.4.1-3: General test parameters for TBD

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2 |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.2 FR1 | As specified in clause A.3.10.1 |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| PRACH configuration index |  | Config 1,2,3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX in connected mode |  | Config 1,2,3 | OFF | DRX is not used |
| DRX in idle mode | s | Config 1,2,3 | 0.32 | The value shall be used for all cells in the test. |
| measReselectionValidityDuration-r18 | s |  | 5 |  |
| Time offset between serving and neighbour cells |  | Config 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3 ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells |
| T1 | s | Config 1,2,3 | 10 |  |
| T2 | s | Config 1,2,3 | 30 |  |
| T3 | s | Config 1,2,3 | 5 |  |
| T4 | s | Config 1,2,3 | 1 |  |

6.6.9.4.4.2	Test procedure

TBD

6.6.9.4.4.3	Message contents

TBD

6.6.9.4.5	Test requirements

Table 6.6.9.4.5-1 defines the primary level settings for all tests.

Table 6.6.9.4.5-1: NR Cell specific test parameters for TBD

| Parameter |  | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  |  | 2 |  |  |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | N.A | N.A |  |  |  |  |  |  |  |
|  |  |  | TDDConf.1.1 | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | TDDConf.2.1 | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  |  | NA |  |  |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  |  | NA |  |  |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  |  | SSB.5 FR1 |  |  |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  |  | SSB.6 FR1 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  |  | SMTC.5 |  |  |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  |  | SMTC.4 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz |  | -98+TT |  |  |  | -98+TT |  |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  |  |  | -98+TT |  |  |  |
|  |  |  | Config 3 | -95+TT |  |  |  | -95+TT |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -88+TT |  |  |  | -infinity | -91+TT | -100+TT | -100+TT |
|  |  |  | Config 3 | -85+TT |  |  |  | -infinity | -88+TT | -97+TT | -97+TT |
|  |  | dB | Config 1,2,3,4, | 10+TT |  |  |  | - infinity | 7+TT | -2+TT | -2+TT |
|  |  | dB | Config 1,2,3 | 10+TT |  |  |  | -infinity | 7+TT | -2+TT | -2+TT |
| IoNote3 |  | dBm/9.36 MHz | Config 1,2 | -59.63+TT |  |  |  | -70.5+TT | -62.26+TT | -67.92+TT | -67.92+TT |
|  |  | dBm/38.16 MHz | Config 3 | -53.54+TT |  |  |  | -63.96+TT | -56.17+TT | -61.83+TT | -61.83+TT |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  |  |  | AWGN |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |  |

During the period T2 and T3 the UE shall not perform reselection.

At the start of T4 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2.

The UE shall send early measurement report to the PCell with valid measurement results.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in section 10.1.4B for SS-RSRP and in section 10.1.8B for SS-RSRQ and test ends. In the test case, the reported measurements are considered valid if they fulfil measurement accuracy requirements according to Cell 2 signal level during T3.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

### 6.6.10 to 6.6.14

### 6.6.15	Idle Mode inter-RAT CA/DC Measurements

#### 6.6.15.0	Minimum conformance requirements

For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement reporting provided T331 has not expired, the serving cell is supporting idle mode CA/DC measurement reporting and the serving cell is in the validity area.

This subclause defines the requirements for the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an NE-DC and NR carrier aggregation capable UE which has been configured with one or more of following, one or more SCells, one E-UTRAN PSCell or one or more downlink E-UTRAN SCells during the Connected mode and which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16. The requirements are applicable for SCell(s) and E-UTRAN FDD and TDD PSCell and SCells.

Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signalling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signalling, and

-	The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report, and

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-RAT E-UTRAN cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in TS 36.133 [23] Annex B.1.1 and Annex B.1.2 for a corresponding Band. An inter-frequency cell is considered detectable according to the conditions in TS 38.133 [6] Annex B.1.2 and B.1.3 for a corresponding band. An SSB of an inter-frequency cell is considered detectable according to SSB_RP and SSB Ês/Iot defined in TS 38.133 [6] Annex B.1.2 and B.1.3 for a corresponding Band.

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in clause 4.2.2.2 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.2B, 10.1.3B, 10.1.7B and 10.1.8B.

While T331 is running, the UE shall perform measurement on the configured inter-RAT carriers for idle mode CA/DC measurement reporting according to the UE measurement capability.

A UE which supports idleInactiveEUTRA-MeasReport-r16 shall support idle mode DC measurements of:

-	at least 7 E-UTRAN inter-RAT carriers which are also configured for E-UTRAN inter-RATmobility measurements, and

-	at least 1 E-UTRAN inter-RAT carrier which is not configured for E-UTRAN inter-RAT mobility measurements.

The UE shall be capable of monitoring a total of at least 7 inter-RAT carriers for idle mode CA/DC measurements comprising of carriers configured for inter-RAT mobility measurements and carriers not configured for inter-RAT mobility measurements.

For inter-RAT carriers configured for idle mode CA/DC measurements, if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ the inter-RAT measurement requirements in TS 38.133 [6] clause 4.2.2.5 shall apply, where UE shall search for and measure inter-RAT layers configured for idle mode CA/DC measurements in preparation for possible reporting. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ the UE shall search for inter-RAT layers configured for idle mode CA/DC measurements at least every Thigher_priority_search where Thigher_priority_search is described in TS 38.133 [6] clause 4.2.2, where UE shall search for and measure inter-RAT layers configured for idle mode CA/DC measurements in preparation for possible reporting.

For overlapping inter-RAT carriers configured for idle mode CA/DC measurements, the UE shall be capable of performing RSRP and RSRQ measurements of the carriers, and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the carriers configured for idle mode CA/DC measurements to higher layers, with measurement accuracy as specified in TS 36.133 [23] clauses in 9.1.3B.3 and 9.1.6B.2, respectively.

The UE shall be able to report idle mode CA measurements when idle mode CA measurement reporting is requested by the network.

#### 6.6.15.1	NR SA FR1 Idle Mode measurements of inter-RAT CA candidate cells for early reporting

6.6.15.1.1	Test purpose

The purpose of this test is to verify that the UE properly retains the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode, when the UE has entered Idle mode. Additionally, test that the UE performs the required measurements on the serving cell and the configured inter-RAT carrier for idle mode measurement reporting. This test will partly verify the Idle mode CA measurements in TS 38.133 [6] clause 4.4. In the test, connected mode DRX configuration is not configured in either PCell or PSCell.

Additionally, the purpose of this test is to verify that the SS-RSRP, SS-RSRQ, RSRP and RSRQ measurement accuracy is within the specified limits. This test will verify the accuracy requirements in TS 38.133 [6] clauses 10.1.2B and 10.1.7B for intra-frequency measurements and clause 10.2.4 and 10.2.5 for the inter-RAT measurements.

6.6.15.1.2	Test applicability

This test applies to all types of NR UE release 16, supporting NE-DC and idleInactiveEUTRA-MeasReport-r16.

6.6.15.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.15.0.

The normative reference for this requirement is TS 38.133 [6] clauses 4.4, 10.1.2B, 10.1.7B, 10.2.4 and 10.2.5 and A.6.6.15

6.6.15.1.4	Test description

6.6.15.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.15.1.4.1-1.

Table 6.6.15.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.15.1-1 | FR1 FDD SSB SCS 15kHz BW 10MHz – LTE FDD 10MHz |
| 6.6.15.1-2 | FR1 FDD SSB SCS 15kHz BW 10MHz – LTE TDD 10MHz |
| 6.6.15.1-3 | FR1 TDD SSB SCS 30kHz BW 40MHz – LTE FDD 10MHz |
| 6.6.15.1-4 | FR1 TDD SSB SCS 30kHz BW 40MHz – LTE TDD 10MHz |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.15.1.4.1-2.

Table 6.6.15.1.4.1-2: Initial conditions for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.15.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.3 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameter settings are set up according to Table 6.6.15.1.4.1-3 and Table 6.6.15.1.4.1-4.

2. Message contents are defined in clause 6.6.15.1.4.3.

3. The test scenario comprises of two NR Cells. (Cell 1 and Cell 2). Cell 1 is configured according to Annex C.1.2 and C.1.3.

Table 6.6.15.1.4.1-3: General test parameters for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two radio channels are used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1 in FR1 |
| PSCell |  | Cell 2 | PSCell on RF channel number 2 in LTE. Cell 2 is of higher priority. |
| DRX |  | OFF | For both PCell and PSCell once configured |
| PRACH configuration in Cell 2 |  | PRACH_2CE | PRACH configuration as specified in Clause A.3.16 in TS 36.133 [23] |
| T331 | s | 300 |  |
| CSI reporting periodicity and offset configuration for Cell 2 | ms | 2 |  |
| T1 | s | 0.5 | During this time the PCell is known and PSCell is configured. |
| T2 | s | 0.5 | PSCell access. |
| T3 + T4 | s | 66 | During this time the UE is configured to perform inter-RAT measurements in idle mode on the PSCell carrier. |
| T5 | s | 0.5 | UE is paged and connection is setup. Network requests measurement report from the UE. |

Table 6.6.15.1.4.1-4: General idle mode test parameters for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Serving cell |  | 1, 2, 3, 4 | Cell 1 | The UE camps on Cell 1 which is the former PCell. |
| Neighbour cell |  | 1, 2, 3, 4 | Cell 2 | The UE shall perform inter-RAT measurements on Cell 2 which is the former PSCell. |
| RF Channel Number |  | 1, 2, 3, 4 | 1, 2 |  |
| Time offset between cells |  | 1, 2, 3, 4 | 3 s | Synchronous cells |
| Access Barring Information | - | 1, 2, 3,4 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  | 1, 2 | SSB.1 FR1 | Serving cell |
|  |  | 3, 4 | SSB.2 FR1 | Serving cell |
| SMTC configuration Serving cell |  | 1, 2, 3, 4 | SMTC.2 |  |
| DRX cycle length | s | 1, 2, 3, 4 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  | 1, 2, 3, 4 | 190 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  | 1, 2, 3, 4 | Not configured |  |
| T3 | s | 1, 2, 3, 4 | 0.5 | T3 needs to be defined so that cell measurement time is taken into account. |
| T4 | s | 1, 2, 3, 4 | 65 | T4 needs to be defined so that cell measurement time is taken into account. |

6.6.15.1.4.2	Test procedure

Two cells are deployed in the test, Cell 1, which is the PCell in connected, and serving cell in idle mode, on radio channel 1 in FR1, and Cell 2, which is the PSCell in connected, and measured LTE inter-RAT cell in idle mode, on radio channel 2 in LTE. The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1 and Cell 2. During T1 Cell 2, the PSCell, shall be configured.

- Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

- The SS shall set the parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate. T1 starts.

- The SS then shall transmit RRCReconfiguration message with condition NE-DC according to TS 38.508-1 [14] Table 4.6.1-13 to add E-UTRA cell (PSCell).

- The UE shall transmit an RRCReconfigurationComplete message.

- The UE shall send a PRACH to PSCell during T1.

- After the UE has transmitted the random-access preamble on the PSCell, the SS shall set T2 parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate. T2 starts.

- After T2, the SS shall set T3 parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate and send the RRCRelease. T3 starts.

- During the time periods T3 and T4 the UE is in Idle mode configured to perform inter-RAT idle mode CA/DC measurements on Cell 2. The UE shall not perform reselection.

- After T3 is expired, the SS shall set T4 parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate. T4 starts.

- After T4 expires the SS shall set T5 parameters according to Table 6.6.15.1.5-1 and Table 6.6.15.1.5-2 as appropriate, T5 starts.

- The SS transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity). During the connection setup the UE is reports idleMeasAvailable-r16 in the RRCSetupComplete message.

- The SS shall send the UEInformationRequest message, requesting UE to report idle mode CA/DC measurements.

- If UE responds with UEInformationResponse message containing the idle mode measurement results with SS-RSRP values within the limits in Table 6.6.15.1.5-3 for configuration 1 and 2 and Table 6.6.15.1.5-4 for configuration 3, and SS-RSRQ values within the limits in Table 6.6.15.1.5-5 for configuration 1, 2 and 3, the number of passed iterations is increased by one. Otherwise, the number of failed iterations is increased by one.

- If paging in step 11 succeeds, go to step 15, otherwise switch off the UE.

- Switch on the UE and ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR with Connected without release On according to TS 38.508-1 [14] clause 4.5.

- Repeat step 2-15 until a test verdict has been achieved.

6.6.15.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.15.1.4.3-1: Common Exception messages for

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |

Table 6.6.15.1.4.3-2: SIB1 for NR SA FR1 Idle mode CA/DC measurement of inter-RAT DC candidate cells for early reporting

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| idleModeMeasurementsEUTRA-r16 | TRUE | This field indicates that a UE that is configured for EUTRA idle/inactive measurements shall perform the measurements while camping in this cell and report availability of these measurements when establishing or resuming a connection in this cell. If absent, a UE is not required to perform EUTRA idle/inactive measurements. |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.15.1.4.3-3: RRCRelease for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcRelease SEQUENCE { |  |  |  |
| MeasIdleConfigDedicated-r16 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListNR-r16 ::= SEQUENCE { | Not present |  |  |
| measIdleCarrierListEUTRA-r16 ::= SEQUENCE { |  |  |  |
| MeasIdleCarrierEUTRA-r16 ::= SEQUENCE { |  |  |  |
| carrierFreqEUTRA-r16 | ARFCN value of Cell 2 |  |  |
| EUTRA-AllowedMeasBandwidth | mbw6 |  |  |
| measCellListEUTRA-r16 | Not present |  |  |
| reportQuantitiesEUTRA-r16 | both |  |  |
| qualityThresholdEUTRA-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdleDuration-r16 | sec300 |  |  |
| validityAreaList-r16 ::= SEQUENCE { |  |  |  |
| ValidityArea-r16 ::= SEQUENCE { |  |  |  |
| carrierFreq-r16 | ARFCN value of NR channel 1 |  |  |
| validityCellList-r16 ::= SEQUENCE { |  |  |  |
| PCI-Range | PhysCellId of Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.15.1.4.3-4: RRCSetupComplete for SA Idle mode CA/DC measurement for FR1

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-22: |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| idleMeasAvailable-r16 | TRUE | Indication that the UE has idle/inactive measurement report available. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.15.1.4.3-5: UEInformationRequest

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-32A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r16 | TRUE | UE shall report the idle/inactive measurement information, if available, to the network in the UEInformationResponse message. |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.15.1.4.3-6: UEInformationResponse

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| measResultIdleEUTRA-r16 ::= SEQUENCE { |  |  |  |
| measResultsPerCarrierListIdleEUTRA-r16 SEQUENCE |  |  |  |
| carrierFreqEUTRA-r16 | ARFCN of Cell 2 |  |  |
| measResultsPerCellListIdleEUTRA-r16 SEQUENCE |  |  |  |
| rsrp-ResultEUTRA-r16 | INTEGER (0..97) |  |  |
| rsrq-ResultEUTRA-r16 | INTEGER (-30..46) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.15.1.5	Test requirements

Tables 6.6.15.1.5-1 and 6.6.15.1.5-2 define the primary level settings including test tolerances and tables 6.6.15.1.5-3, 6.6. 15.1.5-4, 6.6. 15.1.5-5 and 6.6.15.1.5-6 define accuracy requirements define for Idle mode CA/DC measurement.

Table 6.6.15.1.5-1: Cell specific test parameters for NR cell for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  | 1,2,3,4 | N/A |  |  |  |  |
| Assumption for UE beams Note 5 R: Rough |  | 1,2,3,4 | N/A | N/A | R | R | N/A |
| Frequency Range |  | 1,2,3,4 | FR1 |  |  |  |  |
| Duplex mode |  | 1, 2 | FDD |  |  |  |  |
|  |  | 3, 4 | TDD |  |  |  |  |
| TDD Configuration 1: TDDConf.1.1 2: TDDConf.2.1 |  | 1,2 | - |  |  |  |  |
|  |  | 3,4 | 1 | 1 | 2 | 2 | 1 |
| BWchannel 1: 10: NRB,c = 52 2: 40: NRB,c = 106 | MHz | 1, 2 | 1 | 1 | - | - | 1 |
|  |  | 3, 4 | 2 | 2 | - | - | 2 |
| Initial Downlink BWP configuration |  | 1,2,3,4 | DLBWP.0.1 |  |  |  |  |
| Initial Uplink BWP configuration |  | 1,2,3,4 | ULBWP.0.1 |  |  |  |  |
| Dedicated Downlink BWP configuration 1: DLBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - | - | 1 |
| Dedicated Uplink BWP configuration 1: ULBWP.1.1 |  | 1,2,3,4 | 1 | 1 | - | - | 1 |
| PDSCH Reference Measurement Channel 1: SR.1.1 FDD 2: SR.2.1 TDD | FDD | 1,2 | 1 | 1 | 1 | 1 | 1 |
|  | TDD | 3,4 | 2 | 2 | 2 | 2 | 2 |
| TRS configuration |  | 1,2,3,4 | - |  |  |  |  |
| TCI state |  | 1,2,3,4 | - |  |  |  |  |
| RMSI CORESET parameters | FDD | 1,2 | CR.1.1 FDD |  |  |  |  |
|  | TDD | 3,4 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET parameters | FDD | 1,2 | CCR.1.1 FDD |  |  |  |  |
|  | TDD | 3,4 | CCR.2.1 TDD |  |  |  |  |
| OCNG PatternsNote1 |  | 1,2,3,4 | OP.1 defined in A.3.2.1 |  |  |  |  |
| SSB configuration 1: SSB.1 FR1 2: SSB.2 FR1 |  | 1,2 | 1 |  |  |  |  |
|  |  | 3,4 | 2 |  |  |  |  |
| SMTC configuration |  | 1,2,3,4 | SMTC.2 |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |
| EPRE ratio of PSS to SSS | dB | 1,2,3,4 | 0 | 0 | - | - | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PBCH to PBCH DMRS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PDCCH DMRS to SSS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PDSCH DMRS to SSS |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of PDSCH to PDSCH |  |  | 0 | 0 | - | - | 0 |
| EPRE ratio of OCNG DMRS to SSS |  |  | 0 | 0 | - | - | 0 |
| Noc Note2 | dBm/ 15kHz | 1,2 | -98 | -98 | -98 | -98 | -98 |
|  |  | 3,4 |  |  |  |  |  |
| Noc Note2 | dBm/SCS | 1,2 | -98 | -98 | -98 | -98 | -98 |
|  |  | 3,4 | -95 | -95 | -95 | -95 | -95 |
| Ês/Iot | dB | 1,2,3,4 | 5 | 5 | 5 | 5 | 5 |
| Ês/Noc | dB | 1,2,3,4 | 5 | 5 | 5 | 5 | 5 |
| SS-RSRPNote3,4 | dBm/SCS | 1,2 | -93 | -93 | -93 | -93 | -93 |
|  |  | 3,4 | -90 | -90 | -90 | -90 | -90 |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2 | -63.85 | -63.85 | -63.85 | -63.85 | -63.85 |
|  | dBm/ 38.16 MHz | 3,4 | -57.76 | -57.76 | -57.76 | -57.76 | -57.76 |
| Qrxlevmin | dBm/SCS | 1, 2 | - | - | -140 |  | - |
|  | dBm/SCS | 3, 4 | - | - | -137 |  | - |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | SS-RSRP |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  | - |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | Not sent |  | - |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | Not sent |  | - |
| Threshx, high | dB | 1,2,3,4 | - |  | 52 |  | - |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table 6.6.15.1.5-2: Cell specific test parameters for LTE cell for Idle Mode measurements of inter-RAT DC candidate cells for early reporting

| Parameter | Unit | Config | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 |  |  |  |  |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Frequency Range |  | 1,2,3,4 | LTE |  |  |  |  |
| Duplex mode |  | 1, 3 | FDD |  |  |  |  |
|  |  | 2, 4 | TDD |  |  |  |  |
| BWchannel | MHz | 1,2,3,4 | 10 | 10 | - | - | 10 |
| Measurement bandwidth |  | 1,2,3,4 | - | - | 22-27 | 22-27 | - |
| PDSCH Reference Measurement Channel 1: R.1 FDD 2: R.1 TDD | FDD | 1,3 | 1 | 1 | - | - | 1 |
|  | TDD | 2,4 | 2 | 2 | - | - | 2 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.1 and A.3.1.2.2 in 36.133 [23] 1: R.6 FDD 2: R.6 TDD |  | 1,3 | 1 | 1 | - | - | 1 |
|  |  | 2,4 | 2 | 2 | - | - | 2 |
| OCNG Patterns defined in A.3.2.1.1 (OP.2 FDD) and A.3.2.1.2 (OP.2 TDD) in 36.133 [23] 1: OP.2 FDD 2: OP.2 TDD |  | 1,3 | 1 |  |  |  |  |
|  |  | 2,4 | 2 |  |  |  |  |
| Correlation Matrix and Antenna config |  | 1,2,3,4 | 1x2 Low |  |  |  |  |
| PBCH_RA | dB | 1,2,3,4 | N/A | N/A | 0 | 0 | N/A |
| PBCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PSS_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| SSS_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PCFICH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PHICH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PHICH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PDCCH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PDCCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| PDSCH_RA |  |  | N/A | N/A | 0 | 0 | N/A |
| PDSCH_RB |  |  | N/A | N/A | 0 | 0 | N/A |
| OCNG_RANote 1 |  |  | N/A | N/A | 0 | 0 | N/A |
| OCNG_RBNote 1 |  |  | N/A | N/A | 0 | 0 | N/A |
| Noc Note2 | dBm/ 15kHz | 1,2 | -98 | -98 | -98 | -98 | -98 |
|  |  | 3,4 |  |  |  |  |  |
| Ês/Iot | dB | 1,2,3,4 | 5 | 5 | -3 | 8 | 5 |
| Ês/Noc | dB | 1,2,3,4 | 5 | 5 | -3 | 8 | 5 |
| RSRPNote3,4 | dBm/SCS | 1,2,3,4 | -93 | -93 | -101 | -90 | -93 |
| IoNote3,4 | dBm/ 9.36 MHz | 1,2, 3, 4 | -61.01 | -61.01 | -65.44 | -58.57 | -61.01 |
| Qrxlevmin | dBm/SCS | 1,2,3,4 | - | - | -140 |  | - |
| Pcompensation | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qhysts | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Qoffsets, n | dB | 1,2,3,4 | - | - | 0 | 0 | - |
| Cell_selection_and_ reselection_quality_measurement |  | 1,2,3,4 | RSRP and RSRQ |  |  |  |  |
| Treselection | s | 1,2,3,4 | - |  | 0 |  | - |
| SnonintrasearchP | dB | 1,2,3,4 | - |  | Not sent |  | - |
| SnonintrasearchQ | dB | 1,2,3,4 | - |  | Not sent |  | - |
| Propagation Condition | dB | 1,2,3,4 | - |  | AWGN |  | - |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SSB_RP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

Table 6.6.15.1.5-3: Absolute accuracy requirements for the Cell 1 and Cell 2 RSRP reported values for test configurations 1 and 2

| Normal Conditions | T5 |
| --- | --- |
| Lowest reported value (Cell 1) | 52 |
| Highest reported value (Cell 1) | 75 |
| Lowest reported value (Cell 2) | 39 |
| Highest reported value (Cell 2) | 62 |

Table 6.6.15.1.5-4: Absolute accuracy requirements for the Cell 1 and Cell 2 RSRP reported values for test configurations 3

| Normal Conditions | T5 |
| --- | --- |
| Lowest reported value (Cell 1) | 55 |
| Highest reported value (Cell 1) | 78 |
| Lowest reported value (Cell 2) | 39 |
| Highest reported value (Cell 2) | 62 |

Table 6.6.15.1.5-5: Absolute accuracy requirements for the Cell 1 and Cell 2 RSRQ reported values for test configurations 1, 2 and 3.

| Normal Conditions | T5 |
| --- | --- |
| Lowest reported value (Cell 1) | 54 |
| Highest reported value (Cell 1) | 71 |
| Lowest reported value (Cell 2) | 6 |
| Highest reported value (Cell 2) | 27 |

During time durations T1 the UE shall start transmitting preamble on PSCell. During T2 the UE perform intra-frequency measurements on the PCell and the PSCell.

During the time-period T3 the connection is released, and UE enters idle mode. During the time period T3 and T4 the UE is camped in Idle mode and at T4 the signal level of Cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode DC/CA measurement according to Section 4.4.

At the start of T5 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report. The UE shall send early measurement report to the PCell including idle mode CA/DC measurement from Cell 2.

After receiving the requested early measurement report, the test equipment verifies that the accuracy of measurement reported for serving Cell 1 and Cell 2 meets the requirements in Sections 10.1.2B and 10.1.7B and Sections 10.2.4 and 10.2.5, respectively and test ends.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

### 6.6.16

### 6.6.17	NR SA FR1 event triggered reporting tests with Pre-MG

#### 6.6.17.0	Minimum conformance requirements

6.6.17.0.1	Minimum conformance requirements for event-triggered measurement with gap

[TS 38.133 [6], clause 9.2.6.2, 9.2.6.3]

When a measurement gap is provided or an activated Pre-MG is provided without any pre-MG status changed during the measurement period, the UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index. if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.17.0.1-1.

T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.17.0.1-2.

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap,i in TS38.133 [6] clause 9.1.5.2 for measurement conducted within measurement gaps.

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps or not supporting [concurrent measurement gaps]. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

--	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3.

When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0.

The UE shall be able to identify a new detectable intra-frequency cell within the time period specified in TS 38.133 [6].

For a UE that supports Pre-MG, an SMTC occasion is only considered to be overlapped by Pre-MG if the Pre-MG is activated.

The normative reference for this intra-frequency measurements with measurement gaps requirement is in TS 38.133 [6] clauses 9.2.2, 9.2.4.3 and 9.2.6.

Table 6.6.17.0.1-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil( 5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent measurement gaps, if multiple concurrent gaps are configured, the MGRP is the periodicity of the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell. |  |

Table 6.6.17.0.1-2: Measurement period for intra-frequency measurements with gaps(FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil(5 x Kgap )x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(5 x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: For a UE supporting concurrent gaps, if multiple concurrent gaps are configured, the MGRP is the periodicity of the MG pattern associated to the intra-frequency layer. |  |

[TS 38.133 [6], clause 9.2.2]

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] Sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

[TS 38.133 [6], clause 9.2.4.3]

Reported RSRP, RSRQ, and RS-SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1 (RSRP for FR1), 10.1.7.1 (RSRQ for FR1), 10.1.12.1 (RS-SINR for FR1) and 10.1.13.1 (RS-SINR for FR2).

The UE shall not send any event triggered measurement reports as long as no reporting criteria is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in TS 38.133 [6] clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

A cell is detectable only if at least one SSBs measured from the Cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index as defined in TS 38.133 [6] clause 9.2.5.1 or clause 9.2.6.2. If a cell which has been detectable at least for the time period Tidentify intra without index or Tidentify intra with index defined in TS 38.133 [6] clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than  3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [7]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

#### 6.6.17.1	NR SA FR1 event triggered reporting tests with autonomous activation/deactivation Pre-MG

6.6.17.1.1	Test purpose

The purpose of this test is to verify that the UE makes correct measurement and corresponding event reporting. This test will partly verify the intra-frequency cell search requirements in TS 38.133 [6] clause 9.2.6.2 and 9.2.6.3. And this test will also jointly verify pre-configured measurement gap activation/deactivation delay in TS 38.133 [6] clause 8.19.2.

6.6.17.1.2	Test applicability

This test applies to all types of NR UEs supporting 5GS NR SA FR1, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredUE-AutonomousMeasGap-r17 capability from release 17 onwards.

6.6.17.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.17.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.17.1.

6.6.17.1.4	Test description

6.6.17.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.17.1.4-1.

Table 6.6.17.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Test environment parameters are given in Table 6.6.17.1.4.1-2.

Table 6.6.17.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.17.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.17.1.4.1-3.

2. Message contents are defined in clause 6.6.17.1.4.3.

3. There are one NR carrier and two cells specified in the test. Cell 1 (NR Cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.17.1.4.1-3: General test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap Repetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 0.1 |  |
| T2 | s | 1, 2, 3 | 0.2 |  |
| T3 | s | 1,2, 3 | 5 |  |
| bwp-InactivityTimer | ms |  | 500 |  |

6.6.17.1.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

There are two BWPs configured in Cell 1, BWP-1 which contains the cell defining SSB, and BWP-2 which does not contain any SSB of Cell 1.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.17.1.4.1-3 and Table 6.6.17.1.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3. SS shall transmit an RRCReconfiguration message which include with Measurement configuration and 2 different UE-specific bandwidth parts for Cell 1 (PCell), BWP-1 and BWP-2.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

- Pre-MG status is assumed to be ‘deactivated’ when the BWP is active.

- BWP-1 includes bandwidth of the initial DL BWP and SSB.

- BWP-2 does not include bandwidth of the initial DL BWP and SSB.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE would have ACK/NACK sending for those PDSCHs scheduled in the slots overlapped with the pre-MG occasions in T1.

6. When T1 expires, at the start of time duration T2, the serving gNB trigger Pre-MG activation starts via the DCI format 1_1 command for PCell DL BWP switch, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2. And UE is expected to complete the Pre-MG activation within the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5ms in T2 as defined in TS 38.133 [6] clause 8.6.2 and clause 8.19.2.

7. At the start of time duration T3, the SS shall switch the power setting from T2 to T3 as specified in Table 6.6.17.1.5-1.

8. The UE shall NOT be able to receive PDSCH and report corresponding ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

9. The UE shall transmit a MeasurementReport message triggered by Event A3. With the overall delays measured from the beginning of time period T3 is less than 802 ms.

10. After the SS receive the MeasurementReport message in step 9 or when T3 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

11. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod1008) for next iteration of the test procedure loop.

12. Depending on the choice in Step 10, the SS:
- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
Or
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13. Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.17.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.17.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-6 with conditions Pattern #0 and Pre-CFG Table H.3.1-7 with condition INTRA-FREQ Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.17.1.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | Entry 1: BWP-1 |  |
| BWP-Downlink[2] | BWP-Downlink with condition BWP-Id2 | Entry 2: BWP-2 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWWP-1 |  |
| defaultDownlinkBWP-Id | 0 | Initial BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | Entry 1: BWP-1 |  |
| BWP-Uplink[2] | BWP-Uplink with condition BWP-Id2 | Entry 2: BWP-2 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 |  |
| } |  |  |  |
| } |  |  |  |

6.6.17.1.5	Test requirement

Table 6.6.17.1.4.1-3 and Table 6.6.17.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.17.1.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| Initial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| BWP-1 Configuration |  | 1,2,3 | DLBWP.1.3 ULBWP.1.3 |  |  | N/A |  |  |
| BWP-2 Configuration |  | 1,2,3 | DLBWP.1.2 ULBWP.1.2 |  |  | N/A |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.2 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | N/A |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  |  | DLBWP.1.1 |  |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  |  | ULBWP.1.1 |  |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  |  | SSB |  |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| EsIot | dB | 1 | 4 |  | -1.46 | -Infinity |  | -1.46 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| EsNoc | dB | 1 | 4 |  | 4 | -Infinity |  | 4 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 |  | -94 | -Infinity |  | -94 |
|  |  | 2 | -94 |  | -94 | -Infinity |  | -94 |
|  |  | 3 | -91 |  | -91 | -Infinity |  | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 |  | -62.25 | -64.60 |  | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 |  | -62.25 | -64.60 |  | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 |  | -56.16 | -58.50 |  | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

During T1,

The UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

During T2,

The UE is expected to complete the Pre-MG activation within the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5ms in T2 as defined in TS 38.133 [6] clause 8.6.2 and clause 8.19.2.

During T3,

The UE shall NOT be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

The UE shall send one Event A3 triggered measurement report, with a overall measurement reporting delay less than 802ms from the beginning of time period T3.

NOTE: this gives a total of 800 ms for measurement reporting delay plus 2 ms for 2xTTIDCCH insertion uncertainty.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

#### 6.6.17.2	NR SA FR1 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation

6.6.17.2.1	Test purpose

The purpose of this test is to verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG. This test will partly verify the pre-MG activation and deactivation delay requirements in TS 38.133 [6] clause 8.19.2 and the intra-frequency cell search requirements in TS 38.133 [6] clause 9.2.6.2 and 9.3.4.

6.6.17.2.2	Test applicability

This test applies to all types of NR UEs supporting 5GS NR SA FR1, BWP adaptation of at least 2 BWPs, BWP operation without bandwidth restriction, DCI and timer-based active BWP switching delay Type1 or Type2, CSI-RS-based RLM and preconfiguredNW-ControlledMeasGap-r17 capability from release 17 onwards.

6.6.17.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.17.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.17.2.

6.6.17.2.4	Test description

6.6.17.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.17.2.4-1.

Table 6.6.17.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Test environment parameters are given in Table 6.6.17.2.4.1-2.

Table 6.6.17.2.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.17.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.17.2.4.1-3.

2. Message contents are defined in clause 6.6.17.2.4.3.

3. There are one NR carrier and two cells specified in the test. Cell 1 (NR Cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.17.2.4.1-3: General test parameters for SA intra-frequency event triggered reporting with pre-configured measurement gaps and network-controlled activation/deactivation

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE gaps |  |
| Measurement gap repetition periodicity | ms | 1, 2, 3 | 40 |  |
| Measurement gap length | ms | 1, 2, 3 | 6 |  |
| Measurement gap offset | ms | 1, 2, 3 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 0.1 |  |
| T2 | s | 1, 2, 3 | 0.1 |  |
| T3 | s | 1, 2, 3 | 5 |  |

6.6.17.2.4.2	Test procedure

The test consists of 3 successive time periods, with durations of T1, T2, and T3, respectively.

There are two BWPs configured in Cell 1, BWP-1 which contains the cell defining SSB, and BWP-2 which does not contain any SSB of Cell 1.

PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on Cell 1 and the time duration of T2.

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. A pre-MG is configured before the test.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.17.2.4.1-3 and Table 6.6.17.2.5-1. Propagation conditions are set according to Annex C clauses C.2.2.

3. SS shall transmit an RRCReconfiguration message which include with Measurement configuration and 2 different UE-specific bandwidth parts for Cell 1 (PCell), BWP-1 and BWP-2.

- UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in PCell.

- BWP-1 includes bandwidth of the initial DL BWP and SSB with preConfGapStatus for BWP-1 is set to ‘0’ as ‘deactivated’.

- BWP-2 does not include bandwidth of the initial DL BWP and SSB with preConfGapStatus for BWP-2 is set to ‘1’ as ‘activated’.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. PDCCHs indicating new transmissions shall be sent continuously on Cell 1 to ensure that the UE would have ACK/NACK sending for those PDSCHs scheduled in the slots overlapped with the pre-MG occasions in T1.

6. When T1 expires, at the start of time duration T2, the serving gNB trigger Pre-MG activation starts via the DCI format 1_1 command for PCell DL BWP switch, is received at the UE side in PCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2. And UE is expected to complete the Pre-MG activation within the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5ms in T2 as defined in TS 38.133 [6] clause 8.6.2 and clause 8.19.2.

7. At the start of time duration T3, the SS shall switch the power setting from T2 to T3 as specified in Table 6.6.17.2.5-1.

8. The UE shall NOT be able to receive PDSCH and report corresponding ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

9. The UE shall transmit a MeasurementReport message triggered by Event A3. With the overall delays measured from the beginning of time period T3 is less than 802 ms.

10. After the SS receive the MeasurementReport message in step 9 or when T3 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

11. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12. Depending on the choice in Step 10, the SS:

- if the RRC Connection Release has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
Or 
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13. Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.17.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.17.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-6 with conditions Pattern #0 and Pre-CFG Table H.3.1-7 with condition INTRA-FREQ Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.17.2.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | Entry 1: BWP-1 |  |
| BWP-Downlink[2] | BWP-Downlink with condition BWP-Id2 | Entry 2: BWP-2 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWP-1 |  |
| defaultDownlinkBWP-Id | 0 | Initial BWP |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | Entry 1: BWP-1 |  |
| BWP-Uplink[2] | BWP-Uplink with condition BWP-Id2 | Entry 2: BWP-2 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.17.2.4.3-3: BWP-DownlinkDedicated

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkDedicated ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] { | BWP-Downlink with condition BWP-Id0 | DLBWP.1.3 configuration |  |
| EXTENSION { |  |  |  |
| preConfGapStatus-r17 | 00000000 | DLBWP.1.3 configuration |  |
| } |  |  |  |
| } |  |  |  |
| BWP-Downlink[1] { | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| EXTENSION { |  |  |  |
| preConfGapStatus-r17 | 10000000 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.17.2.5	Test requirement

Table 6.6.17.2.4.1-3 and Table 6.6.17.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.17.2.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting with per-UE gaps for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  | N/A |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  | N/A |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  |  |  | N/A |  |  |
|  |  | 2 | CCR.1.2 TDD |  |  |  | N/A |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  | N/A |  |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  | N/A |  |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |  |  | N/A |  |  |
| Active DL BWP configuration for BWP-1 |  | 1, 2, 3 | DLBWP.1.3 |  |  |  | N/A |  |  |
| Active UL BWP configuration for BWP-1 |  | 1, 2, 3 | ULBWP.1.3 |  |  |  | N/A |  |  |
| Active DL BWP configuration for BWP-2 |  | 1, 2, 3 | DLBWP.1.2 |  |  |  | N/A |  |  |
| Active UL BWP configuration for BWP-2 |  | 1, 2, 3 | ULBWP.1.2 |  |  |  | N/A |  |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  |  |  | N/A |  |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |
| EsIot | dB | 1,2,3 | 4 | 4 |  | -1.46 | -Infinity | -Infinity | -1.46 |
| EsNoc | dB | 1,2,3 | 4 | 4 |  | 4 | -Infinity | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1,2 | -94 | -94 |  | -94 | -Infinity | -Infinity | -94 |
|  |  | 3 | -91 | -91 |  | -91 | -Infinity | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1,2 | -64.60 | -64.60 |  | -62.25 | -64.60 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 |  | -56.16 | -58.50 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2 and T4. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

During T1,

The UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

During T2,

The UE is expected to complete the Pre-MG activation within the beginning of PCell’s DL slot (i+TBWPswitchDelay) + 5ms in T2 as defined in TS 38.133 [6] clause 8.6.2 and clause 8.19.2.

During T3,

The UE shall NOT be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasions.

The UE shall send one Event A3 triggered measurement report, with an overall measurement reporting delay less than 802ms from the beginning of time period T3.

NOTE: this gives a total of 800 ms for measurement reporting delay plus 2 ms for 2xTTIDCCH insertion uncertainty.

The overall delays measured test requirement is expressed as:

Overall delays measured = measurement reporting delay + TTI insertion uncertainty

Measurement reporting delay = Tidentify_intra_without_index

Tidentify_intra_without_index = TPSS/SSS_sync_intra + TSSB_measurement_period_intra

TPSS/SSS_sync_intra = 600 ms

TSSB_measurement_period_intra = 200 ms

TTI insertion uncertainty = 2 ms

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 6.6.18	SA event triggered reporting tests with concurrent gaps

#### 6.6.18.0	Minimum conformance requirements

6.6.18.0.1	Minimum conformance requirements for intra-frequency measurement

The UE shall be able to identify a new detectable intra-frequency cell within the time period specified in TS 38.133 [6].

Intra-frequency measurements without measurement gaps:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Kp = 1 when Navailable = 0.

-	Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp =1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with the ML of NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [13]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

If the higher layer signalling in TS38.331 [13] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, time period requirements are not specified to identify a new detectable intra-frequency cell.

Intra-frequency measurements with measurement gaps:

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps or not supporting [concurrent measurement gaps]. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

When concurrent measurement gaps are configured, requirements in this clause do not apply ifNavailable =0.

If the higher layer signalling in TS 38.331 [13] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, time period requirements are not specified for identify a new detectable intra-frequency cell.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.3, 9.2.5 and 9.2.6.

6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement

The UE shall be able to identify a new detectable inter-frequency cell within the time period specified in TS 38.133 [6].

Inter-frequency measurements without measurement gaps:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is a scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the SSB frequency layer, and starting at the beginning of any SMTC occasion:

Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG occasions within the window, and

Navailable is the number of SMTC occasions that are not overlapped with any MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule provided that concurrent measurement gaps are configured.

Kp = 1 when Navailable = 0.

Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When inter-frequency SMTC is fully non overlapping with measurement gaps or inter-frequency SMTC is fully overlapping with MGs, Kp =1.

When inter-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When inter-frequency SMTC is partially overlapping with the VIL of NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP.

Inter-frequency measurements with measurement gaps:

Kgap is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gap(s) and per-FR measurement gap(s) within the same FR, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W, after accounting for collisions between the measurement gaps by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Kgap is only applicable for UE supporting concurrentMeasGap-r17. When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3, 9.3.9.

6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement

The requirements are application for NR−E-UTRAN FDD and NR−E-UTRAN TDD when DRX is used or not.

When the UE requires measurement gaps to identify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are scheduled, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable FDD or TDD cell within the time period specified in TS 38.133 [6].

For a UE supporting and configured with concurrent measurement gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent measurement gaps.

For a window W of duration MGRP_max, where MGRP_max is the maximum MGRP across all configured per-UE measurement gap(s) and per-FR measurement gap(s) for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated gap occasions within the window, including those overlapped with other MG occasions within the window, and

Navailable is the number of non-dropped associated measurement gap occasions after accounting for collisions between the measurement gaps by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Requirements do not apply for UE configured with concurrent measurement gaps, if Navailable=0

The normative reference for this requirement is TS 38.133 [6] clause 9.4.2, 9.4.3.

6.6.18.0.4	Minimum conformance requirements for PRS measurement

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP specified in TS 37.355 [38], the UE shall be able to measure multiple (up to the UE capability specified in TS 38.133 [6] Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [10], during the measurement period defined in TS 38.133 [6] Clause 9.9.2.5

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

-	For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioning frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including those overlapped with other MG occasions within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable =0.

The normative reference for this requirement is TS 38.133 [6] clause 9.9.2.

#### 6.6.18.1	NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers

6.6.18.1.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting for concurrent gaps non-overlap with SSB-based measurement in both inter-frequency layers within inter-frequency cell search requirements in TS 38.133 [6] clause 9.3.4.

6.6.18.1.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap capability from release 17 onwards.

6.6.18.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.1.

6.6.18.1.4	Test description

6.6.18.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.1.4.1-1.

Table 6.6.18.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.18.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.18.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.18.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Test environment parameters are given in Table 6.6.18.1.4.1-2.

Table 6.6.18.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.1.4.1-3.

2. Message contents are defined in clause 6.6.18.1.4.3.

3. There are three NR carriers and three cells specified in the test. Cell 1 (NR Cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.1.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overlapping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR Cell 2 is on NR RF channel number 2. NR Cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 for MeasGapId #1 19 for MeasGapId #2 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour Cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour Cell 2 |  | Config 1, 2, 3 | 5ms |  | Asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

6.6.18.1.4.2	Test procedure

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 (Cell 2) as neighbour Cell 1 in FR1 on NR RF channel 2, and NR Cell 3 (Cell 3) as neighbour Cell 2 in FR1 on NR RF channel 3. The test parameters are given in Table 6.6.18.1.4.1-3 and Table 6.6.18.1.5-1 respectively.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #1 as defined in Table 6.6.18.1.4.1-3. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #2 and #3 respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2 and NR Cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.18.1.5-1.

3. SS shall transmit an RRCReconfiguration message to configure Event A3 triggered measurement report. Two concurrent per-UE measurement gap patterns are configured. MeasGapId #2 for NR Cell 3 is configured with a higher priority than MeasGapId #1 for NR Cell 2.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.18.1.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If Cell 2’s overall delays measured from the beginning of time period T2 is less than 920 ms, and Cell 3’s overall delays measured from the beginning of time period T2 is less than 1280 ms, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with conditions INTER-FREQ MO and Synchronous cells and MGe with MeasGapId-r17=1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Cell 3 | Table H.3.1-3 with condition INTER-FREQ MO and MGe with MeasGapId-r17=2 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

Table 6.6.18.1.4.3-2: MeasConfig (Step3)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod SEQUENCE { | 3 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTRA-FREQ MO | NR Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTER-FREQ MO | NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[3] SEQUENCE { |  | Entry 3 |  |
| measObjectId | 3 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTER-FREQ MO | NR Cell 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod SEQUENCE { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod SEQUENCE { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 | NR Cell 2 |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 3 | NR Cell 3 |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | MeasGapConfig- Concurrent |  |  |
| } |  |  |  |

Table 6.6.18.1.4.3-3: MeasGapConfig-Concurrent

| Derivation path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 2 entry |  |  |
| GapConfig-r17[1] SEQUENCE { |  | Entry 1 |  |
| measGapId-r17 | 1 | NR Cell 2 |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 |  |  |
| mgta-r17 | ms0 |  |  |
| gapPriority-r17 | 2 |  |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE { |  | Entry 2 |  |
| measGapId-r17 | 2 | NR Cell 3 |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 19 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | Ms80 |  |  |
| mgta-r17 | ms0 |  |  |
| gapPriority-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.18.1.5	Test requirement

Table 6.6.18.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.18.1.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overlapping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for each neighbouring cell, with a measurement reporting delay less than 920 ms for Cell 2 and 1280ms for Cell 3 from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.18.2	NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers

6.6.18.2.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting for concurrent gaps partially-overlap with SSB-based measurement in both inter-frequency layers within inter-frequency cell search requirements in TS 38.133 [6] clause 9.3.4.

6.6.18.2.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap capability from release 17 onwards.

6.6.18.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.2.

6.6.18.2.4	Test description

6.6.18.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.2.4.1-1.

Table 6.6.18.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.18.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.18.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.18.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Test environment parameters are given in Table 6.6.18.2.4.1-2.

Table 6.6.18.2.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.2.4.1-3.

2. Message contents are defined in clause 6.6.18.2.4.3.

3. There are three NR carriers and three cells specified in the test. Cell 1 (NR Cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.2.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partial overlapping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR Cell 2 is on NR RF channel number 2. NR Cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset | ms | Config 1,2,3 | 39 for MeasGapId #1 4 for MeasGapId #2 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour Cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour Cell 2 |  | Config 1, 2, 3 | 5ms |  | Asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

6.6.18.2.4.2	Test procedure

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 (Cell 2) as neighbour Cell 1 in FR1 on NR RF channel 2, and NR Cell 3 (Cell 3) as neighbour Cell 2 in FR1 on NR RF channel 3. The test parameters are given in Table 6.6.18.2.4.1-3, and Table 6.6.18.2.5-1 respectively. The TE schedules continuous DL data on PCell throughout the test.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #1 as defined in Table 6.6.18.2.4.1-3. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #2 and #3, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2 and NR Cell 3.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.18.2.5-1.

3. The SS shall transmit an RRCReconfiguration message to configure Event A3 triggered measurement report. Two concurrent per-UE measurement gap patterns are configured. MeasGapId #2 for NR Cell 3 is configured with a higher priority than MeasGapId #1 for NR Cell 2.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission in Cell 1.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.18.2.5-1. T2 starts.

7. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1280 ms for both Cell 2 and Cell 3, then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

8. After the SS receives the MeasurementReport message in step 7 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

9. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:

- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.),
OR:
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with conditions INTER-FREQ MO and Synchronous cells and MGe with MeasGapId-r17=1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Cell 3 | Table H.3.1-3 with condition INTER-FREQ MO and MGe with MeasGapId-r17=2 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.7 |

Table 6.6.18.2.4.3-2: MeasConfig (Step3)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod SEQUENCE { | 3 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTRA-FREQ MO | NR Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTER-FREQ MO | NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[3] SEQUENCE { |  | Entry 3 |  |
| measObjectId | 3 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTER-FREQ MO | NR Cell 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod SEQUENCE { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod SEQUENCE { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 | NR Cell 2 |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 3 | NR Cell 3 |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | MeasGapConfig- Concurrent |  |  |
| } |  |  |  |

Table 6.6.18.2.4.3-3: MeasGapConfig-Concurrent

| Derivation path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 2 entry |  |  |
| GapConfig-r17[1] SEQUENCE { |  | Entry 1 |  |
| measGapId-r17 | 1 | NR Cell 2 |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 |  |  |
| mgta-r17 | ms0 |  |  |
| gapPriority-r17 | 2 |  |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE { |  | Entry 2 |  |
| measGapId-r17 | 2 | NR Cell 3 |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 4 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | Ms80 |  |  |
| mgta-r17 | ms0 |  |  |
| gapPriority-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.18.2.5	Test requirement

Table 6.6.18.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.18.2.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gap with partial-overlapping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.7 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for each neighbouring cell, with a measurement reporting delay less than 1280 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.18.3	NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

6.6.18.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct event-triggered reporting of concurrent inter-RAT E-UTRAN and NR FR1 measurements when operating in standalone (SA) operation with PCell in FR1.

6.6.18.3.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap capability from release 17 onwards.

6.6.18.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2 and 6.6.18.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.3.

6.6.18.3.4	Test description

6.6.18.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.3.4.1-1.

Table 6.6.18.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.18.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.18.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.18.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.18.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.18.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.18.3-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Test environment parameters are given in Table 6.6.18.3.4.1-2.

Table 6.6.18.3.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-4 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.3.4.1-3.

2. Message contents are defined in clause 6.6.18.3.4.3.

3. There are two NR carriers and one E-UTRA cell specified in the test. Cell 1 (NR Cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.3.4.1-3: General test parameters for NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1 - 6 | 1, 2 | 2 NR carrier frequency is used in the test. |
| LTE RF Channel Number |  |  | 3 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | 1 - 6 | As specified in Tables 6.6.18.3.5-1 and 6.6.18.3.5-2. |  |
| Active cell |  | 1 - 6 | NR Cell 1 (PCell in FR1) | Cell 1 is on NR RF channel number 1 |
| Neighbour Cell 1 |  | 1 - 6 | Cell 2 (NR FR1) | Cell 2 is on NR RF channel number 2 |
| Neighbour Cell 2 |  | 1 - 6 | Cell 3 (LTE) | Cell 3 is on LTE RF channel number 3 |
| Gap Pattern Id |  | 1 - 6 | 1 for MeasGapId #1 1 for MeasGapId #2 | As specified in TS 38.133 [6] Table 9.1.2-1. Per-UE gap pattern. |
| Measurement gap offset |  | 1 - 6 | 19 for Gap pattern Id #1 39 for Gap pattern Id #2 |  |
| NR measurement quantity |  |  | SS-RSRP | Measurement quantity for Cell 1 and Cell 2 |
| Inter-RAT E-UTRAN measurement quantity |  |  | RSRP | Measurement quantity for Cell 3 |
| b2-Threshold1 | dBm |  | As specified in Table 6.6.18.3.5-1 | Absolute NR SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm |  | -101 | Absolute E-UTRAN RSRP threshold for SS-RSRP measurement on Cell 3 for event B2 |
| Hysteresis | dB | 1 - 6 | 0 |  |
| TimeToTrigger | s | 1 - 6 | 0 |  |
| Filter coefficient |  | 1 - 6 | 0 | L3 filtering is not used |
| offsetMO | dB | 1 - 6 | 6 | NR Cell 2 |
| a4-Threshold | dBm | 1 - 6 | -105 | NR Cell 2 |
| DRX |  |  | OFF |  |
| Time offset between serving and neighbour cells |  | 1, 4 | 3ms | Asynchronous cells NR cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2, 3, 5, 6 | 3s | Synchronous NR cells. |
| T1 | s | 1 - 6 | 5 | for LTE Cell 3 and NR FR1 Cell 2 |
| T2 | s | 1 - 6 | 5 | for LTE Cell 3 |
|  |  |  | 10.3 for PC1 and PC5; 6.5 for other PC | for NR FR1 Cell 2 |

6.6.18.3.4.2	Test procedure

In this test, there are three cells: Cell 1 is the NR PCell, Cell 2 is an Inter-frequency NR FR1 neighbour cell on NR RF channel 2 and Cell 3 is an inter-RAT E-UTRAN neighbour cell on LTE RF channel 3. The test parameters are given in Table 6.6.18.3.4.1-3 and Tables 6.6.18.3.5-1/2 respectively.

In the measurement control information from the PCell it is indicated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter-RAT neighbour becomes better than threshold2) is to be used for the E-UTRAN cell (Cell 3). In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A4 is used for the NR FR1 cell (Cell 2).

Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2 and Cell 3.

Two concurrent per-UE measurement gap pattern configurations # 0 as defined in Table 6.6.18.3.4.1-3 are provided for a UE. Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #1 as defined in Table 6.6.18.3.4.1-3. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for NR RF channel numbers #2 and LTE RF channel #3, respectively.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Tables 6.6.18.3.5-1/2.

3. SS shall transmit an RRCReconfiguration message to configure Event A4 triggered measurement report for Cell 2, Event B2 triggered measurement report for Cell 3 to the PCell and two concurrent per-UE measurement gap patterns with MeasGapId #2 configured with a higher priority than MeasGapId #1.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Tables 6.6.18.3.5-1/2. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A4 and B2. If Cell 2’s overall delays measured from the beginning of period T2 is less than 10240 ms for UE supporting power class 1, or 6400 ms for UE supporting other power class, and Cell 3’s overall delays measured from the beginning of period T2 is less than 3.84 s, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ, INTER-RAT and GAP NEEDED Table H.3.1-3A with conditions MGe with associatedMeasGap-r17 set to 1 Table H.3.1-4AA Table H.3.1-4A Table H.3.1-5 with condition INTER-RAT Table H.3.1-6 with conditions MGe Table H.3.1-7 with conditions INTER-FREQ and INTER-RAT |
| Specific message content exceptions for Test Configurations 6.6.18.3-1 and 6.6.18.3-4 | Table H.3.1-3 with conditions INTER- FREQ MO and MGe with associatedMeasGapSSB-r17 set to 1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message content exceptions for Test Configurations 6.6.18.3-2, 6.6.18.3-3, 6.6.18.3-5 and 6.6.18.3-6 | Table H.3.1-3 with conditions INTER- FREQ MO and Synchronous cells and MGe with associatedMeasGapSSB-r17 set to 1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.6.18.3.4.3-2: MeasConfig (Step 3)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ, INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod SEQUENCE { | 3 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTRA-FREQ MO | NR Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTER-FREQ MO | NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[3] SEQUENCE { |  | Entry 3 |  |
| measObjectId | 3 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectEUTRA | MeasObjectEUTRA-DEFAULT | E-UTRAN Cell 3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod SEQUENCE { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT with condition EVENT_A4 | NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-DEFAULT with condition EVENT_B2 | E-UTRAN Cell 3 |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod SEQUENCE { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 3 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig |  |  | GAP NEEDED |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE { |  | Entry 1 |  |
| measGapId-r17 | 1 |  | MeasGapId #1 |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 19 |  | MeasGapId #1 |
| mgl-r17 | ms6 |  | MeasGapId #1 |
| mgrp-r17 | ms80 |  | MeasGapId #1 |
| mgta-r17 | ms0 |  | MeasGapId #1 |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE { |  | Entry 2 |  |
| measGapId-r17 | 2 |  | MeasGapId #2 |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  | MeasGapId #2 |
| mgl-r17 | ms6 |  | MeasGapId #2 |
| mgrp-r17 | ms80 |  | MeasGapId #2 |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.18.3.5	Test requirement

Tables 6.6.18.3.5-1/2 define the primary level settings including test tolerances for all tests.

Table 6.6.18.3.5-1: Cell specific test parameters of NR cells for NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 |  | T2 |
| NR RF Channel Number |  |  | 1 - 6 | 1 |  | 2 |  |  |
| Duplex mode |  |  | 1, 4 | FDD |  |  |  |  |
|  |  |  | 2, 3, 5, 6 | TDD |  |  |  |  |
| TDD Configuration |  |  | 1, 4 | NA |  |  |  |  |
|  |  |  | 2, 5 | TDDConf.1.1 |  |  |  |  |
|  |  |  | 3, 6 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |  |  |  |
|  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |  |  |  |
|  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |  |  |  |
| BWP configuration | Initial DL BWP |  | 1 - 6 | DLBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| CSI-RS for tracking parameters on NR RF Channel 1 |  |  | 1, 4 | TRS.1.1 FDD |  |  | - |  |
|  |  |  | 2, 5 | TRS.1.1 TDD |  |  |  |  |
|  |  |  | 3, 6 | TRS.1.2 TDD |  |  |  |  |
| OCNG Patterns |  |  | 1 - 6 | OP.1 |  |  |  |  |
| PDSCH Reference measurement channel |  |  | 1, 4 | SR.1.1 FDD |  |  | - |  |
|  |  |  | 2, 5 | SR.1.1 TDD |  |  |  |  |
|  |  |  | 3, 6 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | 1, 4 | CR.1.1 FDD |  |  | - |  |
|  |  |  | 2, 5 | CR.1.1 TDD |  |  |  |  |
|  |  |  | 3, 6 | CR.2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | 1, 4 | CCR.1.1 FDD |  |  | - |  |
|  |  |  | 2, 5 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | 3, 6 | CCR.2.1 TDD |  |  |  |  |
| SMTC configuration |  |  | 1, 4 | SMTC.2 |  |  | SMTC.2 |  |
|  |  |  | 2, 3, 5, 6 | SMTC.1 |  |  | SMTC.1 |  |
| SSB configuration |  |  | 1, 4 | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  |  |  | 2, 5 | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  |  |  | 3, 6 | SSB.2 FR1 |  |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2, 4 ,5 | 15 |  |  |  |  |
|  |  |  | 3, 6 | 30 |  |  |  |  |
| b2-Threshold1 |  | dBm | 1, 2, 4, 5 | -81 |  | - |  |  |
|  |  |  | 3, 6 | -78 |  | - |  |  |
| EPRE ratio of PSS to SSS |  | dB | 1 - 6 | 0 |  | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Noc note2 |  | dBm/15 kHz | 1-6 | -98 |  |  |  |  |
| Noc note2 |  | dBm/SCS | 1, 2, 4, 5 | -98 |  |  |  |  |
|  |  |  | 3, 6 | -95 |  |  |  |  |
| Ês/Noc |  | dB | 1 - 6 | 4 | 4 | -Infinity |  | 4 |
| Ês/Iot note3 |  | dB | 1 - 6 | 4 | 4 | -Infinity |  | 4 |
| SSB_RP note3 |  | dBm/SCS | 1, 2, 4, 5 | -94 | -94 | -Infinity |  | -94 |
|  |  |  | 3, 6 | -91 | -91 | -Infinity |  | -91 |
| Io note3 |  | dBm/9.36 MHz | 1, 2, 4, 5 | -64.59 | -64.59 | -70.05 |  | -64.59 |
|  |  | dBm/38.16 MHz | 3, 6 | -58.49 | -58.49 | -63.94 |  | -58.49 |
| Propagation condition |  |  | 1 - 6 | AWGN |  |  |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1 - 6 | 1x2 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

Table 6.6.18.3.5-2: Cell specific test parameters LTE cell for NR SA FR1 NR - E-UTRAN and NR FR1 concurrent event-triggered reporting in non-DRX in FR1

| Parameter | Unit | Configuration | Cell 3 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1 - 6 | 3 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1 - 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1 - 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1 - 6 | -104 |  |
| Ês/Noc | dB | 1 - 6 | -Infinity | 17 |
| Ês/IotNote5 | dB | 1 - 6 | -Infinity | 17 |
| RSRPNote5 | dBm/15kHz | 1 - 6 | -Infinity | -87 |
| SCH_RPNote5 | dBm/15kHz | 1 - 6 | -Infinity | -87 |
| IoNote5 | dBm/9MHz | 1 - 6 | -76.22+10log (NRB,c /50) | -59.13+10log (NRB,c /50) |
| Propagation Condition |  | 1 - 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1 - 6 | 1x2 Low |  |
| NOTE 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. NOTE 2: DL RMCs and OCNG patterns are specified in sections A 3.1 and A 3.2 of TS 36.133 [23] respectively. NOTE 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall send one Event A4 triggered measurement report for Cell 2, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

10240 for UE supporting power class 1 and 5, or

6400 for UE supporting other power class.

The UE shall send one Event B2 triggered measurement report for Cell 3 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2.

The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.18.4	NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

6.6.18.4.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event.

6.6.18.4.2	Test applicability

This test applies to all types of NR UEs supporting concurrent gap and independent measurement gap configurations for PRS measurement capability from release 17 onwards.

6.6.18.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.18.0.2 and 6.6.18.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.18.4.

6.6.18.4.4	Test description

6.6.18.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.18.4.4.1-1.

Table 6.6.18.4.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.18.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.18.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.18.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 6.6.18.4.4.1-2.

Table 6.6.18.4.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.18.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.18.4.4.1-3.

2. Message contents are defined in clause 6.6.18.4.4.3.

3. There are three NR carriers specified in the test. Cell 1 (NR Cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.18.4.4.1-3: General test parameters for NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 3 2: Cell 2 | Two FR1 NR carrier frequencies are used for the NR cells. |
| Active cell |  | 1, 2, 3 | NR Cell 1 (Pcell) | Cell 1 is the PCell and the DL-AoD reference cell in the positioning assistance data. |
| Neighbour cell |  | 1, 2, 3 | NR Cell 2, NR Cell 3 | Cell 2 is an inter-frequency neighbour cell Cell 3 is an intra-frequency neighbour cell in the positioning assistance data. |
| Gap Pattern Id |  | 1, 2, 3 | 0 for MeasGapId #1 24 for MeasGapId #2 |  |
| Measurement gap offset | ms | 1, 2, 3 | 7 for MeasGapId #1 11 for MeasGapId #2 |  |
| A3-Offset | dB | 1, 2, 3 | -15 | Value to ensure TT requirement satisified |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| DRX |  | 1, 2, 3 | NA | OFF |
| Time offset between serving and neighbour cells | s | 1, 2, 3 | 3 | Synchronous cells. |
| Expected RSTD | s | 1, 2, 3 | 3 |  |
| Expected RSTD uncertainty | s | 1, 2, 3 | 5 |  |
| T1 | s | 1, 2, 3 | 2 |  |
| T2 | s | 1, 2, 3 | 5 |  |
| NOTE 1: GP#24 is configured if UE supports MG#24, otherwise GP#0 is configured. |  |  |  |  |

6.6.18.4.4.2	Test procedure

In this test, there are three cells: NR Cell 1 as PCell in FR1 on NR RF channel 1, NR Cell 2 as neighbour cell in FR1 on NR RF channel 2 and NR Cell 3 as neighbour cell in FR1 on NR RF channel 1. The test parameters are given in Table 6.6.18.4.4.1-3 and Table 6.6.18.4.5-1 respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2 and NR Cell 3. Cell 1 and Cell 3 transmit PRS during T2.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #24 as defined in Table 6.6.18.4.4.1-3. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #1 and #2, respectively.

The NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message as defined in TS 37.355 [38] shall be provided to the UE during T1. The last slot containing the two messages for the assistance data and location information request is denoted as #n.

The beginning of the time interval T2 shall be aligned with the beginning of the first MG instance of MeasGapId #2 containing the PRS resources that is T after slot #n, where T = 50 ms is the maximum processing time of the assistance data and location information request.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.18.4.5-1.

3. SS shall transmit an RRCReconfiguration message to configure Event A3 triggered measurement report and two concurrent per-UE measurement gap patterns with MeasGapId #2 configured with a higher priority than MeasGapId #1. One of concurrent gaps used for PRS measurement in same frequency layer of serving cells while the other concurrent gap in the different frequency layer of serving cells.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.18.4.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If Cell 2’s overall delays measured from the beginning of period T2 is less than 1840 ms, and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop. And set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.18.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.18.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-4 with A3-offset = -15dB Table H.3.1-5 Table H.3.1-6 with conditions MGeTable H.3.1-7 with condition INTER-FREQ for Cell 2 and condition INTRA-FREQ for Cell 3 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-1 | Table H.3.1-3 with conditions INTER-FREQ MO and MGe with associatedMeasGapSSB-r17 set to 1 for Cell 2 and condition INTRA-FREQ MO and MGe with associatedMeasGapSSB-r17 set to 2 for Cell 3 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.6.1.8-2 and 6.6.1.8-3 | Table H.3.1-3 with conditions INTER-FREQ MO and MGe with associatedMeasGapSSB-r17 set to 1 for Cell 2 and conditions INTRA-FREQ MO and MGe with associatedMeasGapSSB-r17 set to 2 for Cell 3 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.6.18.4.4.3-2: MeasConfig (Step 3)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod SEQUENCE { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTRA-FREQ MO | NR Cell 1 and Cell 3 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | Entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with condition INTER-FREQ MO | NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod SEQUENCE { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT with condition EVENT_A3 | NR Cell 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod SEQUENCE { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | Entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig |  |  | GAP NEEDED |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | Entry 1 | MeasGapId #1 |
| measGapId-r17 | 1 |  | Pattern #0 |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 7 |  | Pattern #0 |
| mgl-r17 | ms6 |  | Pattern #0 |
| mgrp-r17 | ms40 |  | Pattern #0 |
| mgta-r17 | ms0 |  | Pattern #0 |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | Entry 2 | MeasGapId #2 |
| measGapId-r17 | 2 |  | Pattern #24 |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 11 |  | Pattern #24 |
| mgl-r17 | ms10 |  | Pattern #24 |
| mgrp-r17 | ms80 |  | Pattern #24 |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | true |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.18.4.5	Test requirement

Tables 6.6.18.4.5-1 define the primary level settings including test tolerances for all tests.

Table 6.6.18.4.5-1: Cell specific test parameters of NR cells for NR SA FR1 event triggered reporting tests for PRS and SSB measurement in FR1 without SSB time index detection when DRX is not used

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel |  | MHz | 1, 2 | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP BW |  | MHz | 1, 2 | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  |  |  | 3 | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| BWP configuration | Initial DL BWP |  | 1, 2, 3 | DLBWP.0.1 |  | N/A |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  | N/A |  |
| OCNG Patterns |  |  | 1, 2, 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH RMC configuration |  |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| SSB parameters |  |  | 1 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
|  |  |  | 2 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
|  |  |  | 3 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| TRS Configuration |  |  | 1 | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  |  | 2 | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  |  | 3 | TRS.1.2 TDD |  | N/A |  | N/A |  |
| PRS configuration |  |  | 1 | PRS.1.4 FR1 |  | N/A |  | PRS.1.4 FR1 |  |
|  |  |  | 2 | PRS.1.4 FR1 |  | N/A |  | PRS.1.4 FR1 |  |
|  |  |  | 3 | PRS.2.4 FR1 |  | N/A |  | PRS.2.4 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 1, 2 | 15 |  |  |  |  |  |
|  |  |  | 3 | 30 |  |  |  |  |  |
| PRS muting configuration |  |  | 1, 2, 3 | ‘10’ |  | ‘01’ |  | ‘01’ |  |
| EPRE ratio of PSS to SSS |  |  | 1, 2, 3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  | 1, 2, 3 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  | 1, 2, 3 |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz | 1, 2, 3 | -98 |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | 1, 2 | -98 |  | -98 |  | -98 |  |
|  |  |  | 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS Note5 | 1, 2 | -94 | -94 | -Infinity | -94 | N/A | N/A |
|  |  |  | 3 | -91 | -91 | -Infinity | -91 | N/A | N/A |
| PRS-RSRP Note 3 |  | dBm/SCS Note5 | 1, 2 | -Infinity | -101 | N/A | N/A | -Infinity | -108 |
|  |  |  | 3 | -Infinity | -98 | N/A | N/A | -Infinity | -105 |
| PRS EsIot |  | dB | 1, 2, 3 | -Infinity | -3 | N/A | N/A | -Infinity | -10 |
| PRS EsNoc |  | dB | 1, 2, 3 | -Infinity | -3 | N/A | N/A | -Infinity | -10 |
| IoNote3 |  | dBm/9.36 MHz | 1, 2 | -63.50 |  | -70.05 | -64.59 | -63.50 |  |
|  |  | dBm/38.16 MHz | 3 | -57.40 |  | -63.94 | -58.49 | -57.40 |  |
| Propagation Condition |  |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP/PRS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP/PRS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for Cell 2, with a measurement reporting delay less than 1840 ms from the beginning of period T2.

The PRS RSRP measurement time fulfils the requirements specified in TS 38.133 [6] Clause 9.9.3.5. The UE shall perform and report the PRS RSRP measurements for Cell 3 with respect to the reference cell in the DL-AoD assistance data, Cell 1, within the time duration specified in TS 38.133 [6] section 9.9.3.5 starting from the beginning of time interval T2.

The rate of the correct events for the neighbour cell observed during repeated tests shall be at least 90% with a confidence level of 95%, where the reported PRS RSRP measurement for each correct event shall be within the PRS RSRP reporting range specified in TS 38.133 [6] Clause 10.1.24.3, i.e., between PRS RSRP_0 and PRS RSRP_126.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

IUE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.19	SA event triggered reporting tests with NCSG

#### 6.6.19.0	Minimum conformance requirements

6.6.19.0.1	Minimum conformance requirements for intra-frequency measurement

For the UE supporting NCSG, if NCSG is provided, the UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE has been indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise, UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra-frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2.

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra  ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection given in table 6.6.19.0.1-1 and 6.6.19.0.1-2 (deactivated Scell).

TSSB_measurement_period_intra: equal to a measurement period of SSB based measurement given in table 6.6.19.0.1-3 and 6.6.19.0.1-4 (deactivated Scell).

CSSFintra: it is a carrier specific scaling factor and is determined according to CSSFwithin_ncsg,i in TS 38.133 [6] clause 9.1.5.3 for measurement conducted within NCSG.

Table 6.6.19.0.1-1: Time period for PSS/SSS detection with NCSG (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, 5 x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(600ms, ceil(M2Note 1x 5) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(VIRP, DRX cycle) x CSSFintra |
| NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or [intraRAT-MeasurementEnhancement-r16] on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. |  |

Table 6.6.19.0.1-2: Time period for PSS/SSS detection with NCSG (deactivated SCell) (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | 5 x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320ms | 5 x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 5 x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

Table 6.6.19.0.1-3: Measurement period for intra-frequency measurements with NCSG (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x max(VIRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5) x max(VIRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | 5 x max(VIRP, DRX cycle) x CSSFintra |

Table 6.6.19.0.1-4: Measurement period for intra-frequency measurements with NCSG (deactivated SCell) (FR1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | 5 x max(measCycleSCell, VIRP) x CSSFintra |
| DRX cycle≤ 320ms | 5 x max(measCycleSCell, VIRP, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | 5 x max(measCycleSCell, VIRP, DRX cycle) x CSSFintra |

The requirements given above apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] sections 10.1.2 are fulfilled for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] sections 10.1.7 are fulfilled for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] sections 10.1.12 are fulfilled for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.2 for a corresponding Band.

The RSRP measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.2.1.1 and 10.1.2.1.2, the RSRQ measurement accuracy for all measured cells shall be as specified in TS 38.133 [6] clauses 10.1.7.1.1, and the SINR measurement accuracy for all measured cells shall be as specified in the TS 38.133 [6] clause 10.1.12.1.1.

Reported RSRP, RSRQ and SINR measurements contained in event triggered measurement reports shall meet the requirements in TS 38.133 [6] clauses 10.1.2.1.1, 10.1.2.1.2, 10.1.7.1.1 and 10.1.12.1.1, respectively.

The UE shall not send any event triggered measurement reports, as long as no reporting criteria are fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources for UE to send the measurement report.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify_intra_without_index defined in TS 38.133 [6] section 9.2.7.1. When L3 filtering is used an additional delay can be expected.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.2, 9.2.7.1, and 9.2.7.2.

6.6.19.0.2	Minimum conformance requirements for inter-frequency measurement

For the UE supporting NCSG, if NCSG is provided, the UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured. Otherwise, UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter-frequency SS block of an already detected cell within Tidentify_inter_without_index.

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection given in table 6.6.19.0.2-1.

TSSB_measurement_period_inter: equal to a measurement period of SSB based measurement given in table 6.6.19.0.2-2.

Table 6.6.19.0.2-1: Time period for PSS/SSS detection with NCSG (FR1)

| Condition NOTE1,2 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 8 Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(8*1.5) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

Table 6.6.19.0.2-2: Measurement period for inter-frequency measurements with NCSG (FR1)

| Condition NOTE1,2 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(200ms, 8 Max(VIRP, SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(200ms, Ceil(8 1.5) Max(VIRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: In EN-DC operation, the parameters, timers and scheduling requests referred to in clause 3.6.1 are for the secondary cell group. The DRX cycle is the DRX cycle of the secondary cell group. |  |

When NCSG are provided for inter-frequency measurements, the UE physical layer shall be capable of reporting SS-RSRP, SS-RSRQ and SS-SINR measurements to higher layers with measurement accuracy as specified in TS 38.133 [6] clauses 10.1.4, 10.1.5, 10.1.9, 10.1.10, 10.1.14 and 10.1.15, respectively, as shown in table 6.6.19.0.2-2.

For UE supporting nr-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoNR for inter-frequency measurement,

-	An inter-frequency SSB measurement is defined as measurement with NCSG if

-	the UE indicates ‘ncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement, and

-	the SSB is not completely contained in the active BWP of the UE

When network configures NCSG, the delay requirements are specified in TS 38.133 [6] clause 9.3.10.

-	For inter-frequency SSB based measurements with NCSG, UE may cause scheduling restriction as specified in TS 38.133 [6] clause 9.3.10.3.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in TS 38.133 [6] clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in TS 38.133 [6] clauses 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in TS 38.133 [6] clauses 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to TS 38.133 [6] Annex B.2.3 for a corresponding Band.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.6.3, and 9.3.10.

6.6.19.0.3	Minimum conformance requirements for inter-RAT measurement

The requirements are application for NR−E-UTRAN RSRP, RSRQ, and RS-SINR measurements.

In the requirements, an E-UTRAN cell is considered to be detectable when:

-	RSRP related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.2 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RSRQ related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.3 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.3 of TS 36.133 [23],

-	RS-SINR related conditions in the accuracy requirements in TS 38.133 [6] Section 10.2.5 are fulfilled for a corresponding Band, together with the corresponding side conditions in Annex B.2.3 and Annex B.3.19 of TS 36.133 [23].

For UE supporting eutra-NeedForGapNCSG-reporting-r17 and indicating NeedForGapNCSG-InfoEUTRA for inter-RAT measurement,

-	An inter-RAT measurement is defined as measurement with NCSG if

-	the UE indicates ‘ncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement

When network configures measurement gap or NCSG, the delay requirements are specified in TS 38.133 [6] clause 9.4.2 and 9.4.3.

Parameter TInter1 used in inter-RAT requirements in TS 38.133 [6] clause 9.4 is specified in Table 6.6.19.0.3-1 when NCSG is used.

Table 6.6.19.0.3-1: Minimum available time for inter-RAT measurements when NCSG is configured

| NCSG Pattern Id | Measurement Length (ML, ms) | Visible Interruption Repetition Period (VIRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480 ms period (Tinter1, ms) |
| --- | --- | --- | --- |
| 0 | 5 | 40 | 60 |
| 1 | 5 | 80 | 30 |
| 2 | 2 | 40 | 24Note 1 |
| 3 | 2 | 80 | 12Note 1 |
| 4 | 5 | 20 | 120 Note 1 |
| 6 | 3 | 20 | 72 Note 1,3 |
| 7 | 3 | 40 | 36 Note 1,3 |
| 8 | 3 | 80 | 18Note 1,3 |
| 10 | 2 | 20 | 48 Note 1 |
| NOTE 1: When determining UE requirements using Tinter1 for NCSG pattern IDs 2, 3, 4, 6, 7, 8, 10, Tinter1 = 60 for NCSG pattern IDs 2, 4, 6, 7, 10, and Tinter1 = 30 for NCSG pattern IDs 3 and 8 shall be used. NOTE 2: NCSG pattern configurations applicability is as specified in Table 9.1.2C-1. NOTE 3: This NCSG pattern is applicable for E-UTRA inter-frequency measurements only if NCSG based NR measurements are also configured. |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.4.2, 9.4.3.

6.6.19.0.4	Minimum conformance requirements for interruptions

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s),

-	when mgta is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the last each of the interrupted slots after VIL1 and VIL2.

where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if  for the UL transmission is less than the length of one slot; L=2 otherwise.

Note: Network is supposed to take into account the possible difference between the estimated TA at network and actual TA at UE when scheduling UE in the above slot(s).

The interruptions of NCSG in number of slots are listed in Table 6.6.19.0.4-1 on all serving cells when per-UE NCSG is configured or on FR1 serving cells when per-FR FR1 NCSG is configured to ncsg-MeasGapPerFR-r17 capable UE. There are two interruptions in each NCSG occasion, VIL1 before ML and VIL2 after ML, in NR standalone (with single carrier or NR CA). Each of them has number of interrupted slots captured in Table 6.6.19.0.4-1.

Table 6.6.19.0.4-1: Number of interrupted slots on all serving cells for per-UE NCSG or FR1 serving cells for FR1 NCSG during each VIL in NR standalone operation (with single carrier, NR CA)

| NR | Number of interrupted slots on serving cells |
| --- | --- |
| SCS | When MG timing advance of 0ms is applied |
| (kHz) | VIL=1ms |
| 15 | 1 |
| 30 | 2 |
| 60 | 4 |
| 120 | 8 |
| NOTE 1: NR SCS of 120 kHz is only applicable to the case with per-UE NCSG. NOTE 2: Void |  |

#### 6.6.19.1	NR SA FR1 event-triggered reporting tests with NCSG under non-DRX in FR1

6.6.19.1.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.2.7.1 and 9.2.7.2, and also verify the scheduling availability during intra-frequency measurement with NCSG in 38.133 [6] clause 9.2.7.3.

6.6.19.1.2	Test applicability

This test applies to all types of NR UE from release 17 onwards supporting CSI-RS-based RLM, BWP operation without bandwidth restriction and NR NCSG capability. The UE is capable of NCSG and report ‘ncsg’ through NeedForGapNCSG-InfoNR for PCell. Test 1 is applicable to UEs not supporting per-FR NCSG (ncsg-MeasGapPerFR-r17, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR NCSG.

6.6.19.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.19.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.19.1.

6.6.19.1.4	Test description

6.6.19.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.19.1.4.1-1.

Table 6.6.19.1.4.1-1: Supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 6.6.19.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.19.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.19.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Test environment parameters are given in Table 6.6.19.1.4.1-2.

Table 6.6.19.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
|  | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.19.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.19.1.4.1-3.

2. Message contents are defined in clause 6.6.19.1.4.3.

3. Two cells are deployed in the test, which are FR1 PCell(Cell 1) and FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The power levels and settings are according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.19.1.4.1-3: General test parameters for SA intra-frequency event triggered reporting for FR1 with NCSG

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  | 1, 2, 3 | Cell 1 |  |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 |  | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |  |
| NCSG type |  | 1, 2, 3 | Per-UE | Per-FR | Per-FR NCSG pattern is configured if UE support per-FR NCSG; otherwise, per-UE NCSG patter is configured. |
| NCSG pattern |  | 1, 2, 3 | 0 | 2 |  |
| Visible interruption repetition periodicity | ms | 1, 2, 3 | 40 | 40 |  |
| Measurement length | ms | 1, 2, 3 | 5 | 2 |  |
| NCSG offset | ms | 1, 2, 3 | 39 | 39 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |  |
|  |  | 2 | SSB.1 FR1 |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1 | SMTC.2 |  |  |
|  |  | 2 | SMTC.1 |  |  |
|  |  | 3 | SMTC.1 |  |  |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |  |
| CP length |  | 1, 2, 3 | Normal |  |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |  |
| Filter coefficient |  | 1, 2, 3 | 0 |  | L3 filtering is not used |
| DRX | ms | 1, 2, 3 |  |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s |  | Synchronous cells |
|  |  | 3 | 3 s |  | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |  |
| T2 | s | 1, 2, 3 | 1 |  |  |

6.6.19.1.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. The PCell shall continuously scheduled with data in the DL starting from T2 until the UE has sent the measurement report during T2. The test parameters are given in Table 6.6.19.1.4.1-3 and Table 6.6.19.1.5-1 respectively.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.19.1.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.19.1.5-1. T2 start.

6. SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

7. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802ms and DTX in PCell is not observed by the SS for the corresponding PDSCH scheduled in PCell in all the slots expect for the case where PDSCH or PUCCH is overlapped with VIL of NCSG pattern then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or DTX in PCell is observed by the SS except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

9. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.19.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.19.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.2 and MGe with associatedMeasGapSSB-r17=1 for configuration 6.6.19.1-1 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC.1, MGe with associatedMeasGapSSB-r17=1 and synchronous cells for configuration 6.6.1.19-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC.1, MGe with associatedMeasGapSSB-r17=1 and synchronous cells for configuration 6.6.1.19- 3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ Table H.3.1-8 with Condition CSI-RS RLM |

Table 6.6.19.1.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

Table 6.6.19.1.4.3-3: MeasGapConfig(Step 3)

| Derivation Path: Table H.3.1-6 with condition MGe and NCSG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 1 entry |  |  |
| GapConfig-r17 SEQUENCE { |  |  |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  | Test1 |
|  | perFR1 |  | Test2 |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms5 |  | Test1 |
|  | ms2 |  | Test2 |
| mgrp-r17 | ms40 |  |  |
| mgta-r17 | ms0 |  |  |
| ncsgInd-r17 | true |  |  |
| } |  |  |  |

6.6.19.1.5	Test requirement

Table 6.6.19.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.19.1.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting with NCSG for PCell in FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.2 FDD |  | N/A |  |
|  |  | 2 | CCR.1.2 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report to NR Cell 2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured shall be less than a total of 802ms in this test case (note: this gives a total of 800ms for measurement reporting delay plus 2ms for TTI insertion uncertainty)

The UE shall be scheduled on PCell continuously throughout the test. From the start of T2 until the measurement report is received during T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern

For a test to be considered successful requirements on both Event A3 detection and 100% of all expected ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

#### 6.6.19.2	NR SA FR1 event-triggered reporting tests for FR1 with NCSG for inter-frequency measurement

6.6.19.2.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell identification and measurement period requirements in 38.133 [6] clauses 9.3.10.1 and 9.3.10.2, and also verify the scheduling availability during inter-frequency measurement with NCSG in 38.133 [6] clause 9.3.10.3.

6.6.19.2.2	Test applicability

This test applies to all types of NR UEs supporting NR NCSG capability from release 17 onwards. Test 1 is applicable to UEs not supporting per-FR NCSG (ncsg-MeasGapPerFR-r17, as defined in TS 38.306 [11]) and Test 2 is applicable only to UEs supporting per-FR NCSG. The serving frequency and the target frequency should be selected such that UE reports ‘ncsg’ for the target frequency given the serving frequency.

6.6.19.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.19.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.19.2.

6.6.19.2.4	Test description

6.6.19.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.19.2.4.1-1.

Table 6.6.19.2.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.19.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.19.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.19.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Test environment parameters are given in Table 6.6.19.2.4.1-2.

Table 6.6.19.2.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.19.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.19.2.4.1-3.

2. Message contents are defined in clause 6.6.19.2.4.3.

3. There are two NR cells on two carriers specified in the test. Cell 1 (NR Cell 1) is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings are according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.19.2.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 with NCSG

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 |  | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 |  | NR Cell 2 is on NR RF channel number 2. |
| NCSG Pattern Id |  | Config 1,2,3 | 0 | 2 |  |
| NCSG offset |  | Config 1,2,3 | 9 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.1 FR1 |  |  |
|  |  | Config 2 | SSB.1 FR1 |  |  |
|  |  | Config 3 | SSB.2 FR1 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms |  | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s |  | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1 |  |  |

6.6.19.2.4.2	Test procedure

Two cells are deployed in the test: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. NR RF channel 1 and NR RF channel 2 should be selected such the UE reports ‘ncsg’ for the target frequency on NR RF channel 2. The test parameters are given in Table 6.6.19.2.4.1-3 and Table 6.6.19.2.5-1 respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of NR Cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.19.2.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.19.2.5-1. T2 starts.

6. SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

7. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 922ms and DTX in PCell is not observed by the SS for the corresponding PDSCH scheduled in PCell in all the slots expect for the case where PDSCH or PUCCH is overlapped with VIL of NCSG pattern, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or DTX in PCell is observed by the SS except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

9. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.19.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.19.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1, SMTC.5 and MGe with associatedMeasGapSSB-r17=1 for configuration 6.6.19.2-1 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1, SMTC.4, MGe with associatedMeasGapSSB-r17=1 and synchronous cells for configuration 6.6.19.2-2 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.2 FR1, SMTC.4, MGe with associatedMeasGapSSB-r17=1 and synchronous cells for configuration 6.6.19.2-3 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ |

Table 6.6.19.2.4.3-2: MeasGapConfig(Step 3)

| Derivation Path: Table H.3.1-6 with condition MGe and NCSG |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 1 entry |  |  |
| GapConfig-r17[1] SEQUENCE { |  |  |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  | Test1 |
|  | perFR1 |  | Test2 |
| gapOffset-r17 | 9 |  |  |
| mgl-r17 | ms5 |  | Test1 |
|  | ms2 |  | Test2 |
| mgrp-r17 | ms40 |  |  |
| mgta-r17 | ms0 |  |  |
| ncsgInd-r17 | true |  |  |
| } |  |  |  |

6.6.19.2.5	Test requirement

Table 6.6.19.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.19.2.5-1: Cell specific test parameters for SA event triggered reporting for FR1 with NCSG for inter-frequency measurement

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A3 triggered measurement report to NR Cell 2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured shall be less than a total of 922ms in this test case (note: this gives a total of 920ms for measurement reporting delay plus 2ms for TTI insertion uncertainty)

The UE shall be scheduled on PCell continuously throughout the test. From the start of T2 until the measurement report is received during T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern

For a test to be considered successful requirements on both Event A3 detection and 100% of all expected ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%

#### 6.6.19.3	NR SA FR1 NR - E-UTRAN event-triggered reporting in non-DRX in FR1 with NCSG

6.6.19.3.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct event-triggered reporting of inter-RAT E-UTRAN measurements based on NCSG when operating in standalone (SA) operation with PCell in FR1.

6.6.19.3.2	Test applicability

This test applies to all types of NR Release 17 and forward supporting 5GS NR SA FR1, E-UTRAN and NCSG capability for E-UTRAN from release 17 onwards. The serving frequency and target frequency should be selected such that UE reports ‘ncsg’ for the target frequency given the serving frequency.

6.6.19.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.19.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.19.3.

6.6.19.3.4	Test description

6.6.19.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.19.3.4.1-1.

Table 6.6.19.3.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 6.6.19.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.19.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.19.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.19.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.19.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.19.3-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 6.6.19.3.4.1-2.

Table 6.6.19.3.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.19.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 6.6.19.3.4.1-3.

2. Message contents are defined in clause 6.6.19.3.4.3.

3. There are one NR carriers and one E-UTRAN cell specified in the test. Cell 1 (NR Cell 1) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.19.3.4.1-3: General test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell in FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.19.3.5-1 and 6.6.19.3.5-2. |  |
| Active cell |  | Cell 1 | Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on LTE RF channel number 2 |
| NCSG Pattern Id |  | 0 | As specified in TS 38.133 [6] Table 9.1.9.3-1. |
| NCSG offset | ms | 39 |  |
| NCSG mgta | ms | 0 |  |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | As specified in Table 6.6.19.3-5-1 | SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |

6.6.19.3.4.2	Test procedure

The test consists of two successive time periods, with time duration of T1 and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. The test parameters are given in Table 6.6.19.3.4.1-3, Table 6.6.19.3.5-1 and Table 6.6.19.3.5-2 respectively.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.19.3.5-1.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message.

5. T1 starts, SS continuously schedules on PCell with data in the DL starting from T1 and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

6. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.19.3.5-1. T2 starts.

7. UE shall transmit a MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 3842ms and DTX in PCell is not observed by the SS for the corresponding PDSCH scheduled in PCell in all the slots expect for the case where PDSCH or PUCCH is overlapped with VIL of NCSG pattern then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or DTX in PCell is observed by the SS except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

9. Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.19.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.19.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-RAT Table H.3.1-3A with condition MGe with associatedMeasGap-r17=1 Table H.3.1-4A Table H.3.1-3 with condition SMTC.1, SSB.1 FR1 for configuration 6.6.19.3-1 and 6.6.19.3-4 Table H.3.1-3 with condition SMTC.1, SSB.1 FR1 and synchronous cells for configuration 6.6.19.3-2 and 6.6.19.3-5 Table H.3.1-3 with condition SMTC.1, SSB.2 FR1 and synchronous cells for configuration 6.6.19.3-3 and 6.6.19.3-6 Table H.3.1-5 with conditions INTER-RAT Table H.3.1-7 with condition INTER-RAT |

Table 6.6.19.3.4.3-2: MeasGapConfig(Step 3)

| Derivation Path: Table H.3.1-6 with condition MGe and NCSG |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 1 entry |  |
| GapConfig-r17[1] SEQUENCE { |  |  |
| measGapId-r17 | 1 |  |
| gapOffset-r17 | 39 |  |
| mgl-r17 | ms5 |  |
| mgrp-r17 | ms40 |  |
| mgta-r17 | ms0 |  |
| ncsgInd-r17 | true |  |
| } |  |  |

6.6.19.3.5	Test requirement

Table 6.6.19.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.19.3.5-1: PCell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 2, 3 | FDD |  |
|  |  |  |  | 4, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 kHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 kHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote2 |  |  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| Ês/IotNote3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| SS-RSRPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.55 |
| SSB_RPNote3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.55 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.42 | -73.74 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.32 | -67.64 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.19.3.5-2: E-UTRAN neighbour cell specific test parameters for SA inter-RAT E-UTRAN event triggered reporting in non-DRX with PCell

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| Ês/IotNote5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| RSRPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| SCH_RPNote5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| IoNote5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -76.21 | -57.6 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 6: Propagation condition and correlation matrix are defined in section B.2 in TS 36.101 [27]. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3842ms from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria is not fulfilled.

The overall delays measured shall be less than a total of 3842 ms in this test case (note: this gives a total of 3840 ms for measurement reporting delay plus 2 ms for TTI insertion uncertainty).

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

For a test to be considered successful requirements on both Event B2 detection and 100% of all expected ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90% with confidence level of 95%

#### 6.6.19.4	NR SA FR1 Event triggered reporting on SCC with deactivated SCell test with per-UE NCSG under non-DRX

6.6.19.4.1	Test purpose

The purpose of this test is to verify UE’s ability to make a correct reporting for NCSG with SSB-based measurement with deactivated SCELL.

6.6.19.4.2	Test applicability

This test applies to all types of NR UEs supporting NR NCSG capability from release 17 onwards supporting 2DL CA.

6.6.19.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.19.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.19.4.

6.6.19.4.4	Test description

6.6.19.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in this clause. The supported test configurations for NR PCell are shown in Table 6.6.19.4.4.1-1. Supported test configurations for NR SCell are shown in Table 6.6.19.4.4.1-1A. Test configuration for NR PCell and test configuration for NR SCell are chosen independently.

Table 6.6.19.4.4.1-1: Supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 6.6.19.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.19.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.19.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table 6.6.19.4.4.1-1A: Supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 6.6.19.4-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.19.4-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.19.4-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Test environment parameters are given in Table 6.6.19.4.4.1-2.

Table 6.6.19.4.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.19.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.19.4.4.1-3.

2. Message contents are defined in clause 6.6.19.4.4.3.

3. Three cells are deployed in the test, which are FR1 PCell (Cell 1), FR1 SCell (Cell 2) and FR1 neighbour cell (Cell 3) on the same frequency as the SCell. Cell 1 and Cell 2 shall be configured according to Annex C.1.1 and C.1.2.

Table 6.6.19.4.4.1-3: General test parameters for event triggered reporting on SCC with deactivated SCell with per-UE NCSG for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Configured deactivated SCell |  | 1, 2, 3 | Cell 2 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 |  |
| RF Channel Number |  | 1, 2, 3 | 2: Cell 2 and Cell 3 |  |
| Measurement gap type |  | 1, 2, 3 | Per-UE NCSG |  |
| NCSG pattern |  | 1, 2, 3 | ID # 0 |  |
| MGTA | ms | 1, 2, 5 | 0 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| SCell measurement cycle (measCycleSCell) | ms | 1, 2, 3 | 160 |  |
| Cell 2 timing offset to Cell 1 | s | 1, 2, 3 | 0 |  |
| Time alignment error between Cell 2 and Cell 1 | s | 1, 2, 3 | ≤ Time alignment error as specified in TS 38.104 [28] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell 3 timing offset to Cell 1 | s | 1, 2, 3 | 3 | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.19.4.4.2	Test procedure

Three cells are deployed in the test, which are FR1 PCell(Cell 1), FR1 SCell(Cell 2) and FR1 neighbour cell(Cell 3) on the same frequency as the SCell. The SCell is deactivated during the test. In the measurement control information, a measurement object is configured for the frequency of the SCell, and it is indicated to the UE that event-triggered with Event A6 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 3. The PCell shall continuously scheduled with data in the DL starting from T1 until the UE has sent the measurement report during T2. The test parameters are given in Table 6.6.19.4.4.1-3 and Table 6.6.19.4.5-1 respectively.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.19.4.5-1.

3. SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1 and provide measurement configurations.

4. T1 starts, SS continuously schedules on PCell with data in the DL starting from T1 and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.19.4.5-1. T2 starts.

6. UE shall transmit a MeasurementReport message triggered by Event A6. If the overall delays measured from the beginning of time period T2 is less than 1602 ms and DTX in PCell is not observed by the SS for the corresponding PDSCH scheduled in PCell in all the slots expect for the case where PDSCH or PUCCH is overlapped with VIL of NCSG pattern, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or DTX in PCell is observed by the SS except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

8. Set Cell 3 physical cell identity = ((current Cell 3 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.19.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.19.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions Deactivated SCell Table H.3.1-5 Table H.3.1-7 with condition Deactivated SCell |

Table 6.6.19.4.4.3-2: ServingCellConfig (Cell 2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| servingCellMO | 2 | MeasObjectId for SCell in Table H.3.1-2 |  |
| } |  |  |  |

Table 6.6.19.4.4.3-3: MeasObjectNR for SCell

| Derivation Path: Table H.3.1-3 with condition Deactivated SCell and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  | Configuration 6.6.19.4-2 and 6.6.19.4-3 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2 |  | Configuration 6.6.19.4-1 |
| measCycleSCell-v1530 | sf160 |  |  |
| associatedMeasGapSSB-r17 | 1 |  |  |
| } |  |  |  |

Table 6.6.19.4.4.3-4: ReportConfigNR(Step 3)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition EVENT_A6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA6 SEQUENCE { |  |  |  |
| a6-Offset CHOICE { |  |  |  |
| rsrp | -9 | The actual value is field value * 0.5 dB. |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.19.4.4.3-5: MeasGapConfig(Step 3)

| Derivation Path: Table H.3.1-6 with condition MGe and NCSG |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 { | 1 entry |  |
| GapConfig-r17[1] SEQUENCE { |  |  |
| measGapId-r17 | 1 |  |
| gapOffset-r17 | 39 |  |
| mgl-r17 | ms5 |  |
| mgrp-r17 | ms40 |  |
| mgta-r17 | ms0 |  |
| ncsgInd-r17 | true |  |
| } |  |  |

6.6.19.4.5	Test requirement

Table 6.6.19.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.19.4.5-1: NR Cell specific test parameters for event triggered reporting on SCC with deactivated SCell with per-UE NCSG for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TRS.1.1 FDD |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  | N/A |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1, 2, 3 | -98 |  |  |  |  |  |
| EsIot | dB | 1, 2, 3 | 4 | 4 | 4 | -1.46 | -Infinity | -1.46 |
| EsNoc | dB | 1, 2, 3 | 4 | 4 | 4 | 4 | -Infinity | 4 |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -64.60 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -58.50 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The overall delays measured is defined as the time from the beginning of time period T2, to the moment the UE send one Event A6 triggered measurement report to NR Cell 3. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured shall be less than a total of 1602ms in this test case (note: this gives a total of 1600ms for measurement reporting delay plus 2ms for TTI insertion uncertainty)

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern.

For a test to be considered successful requirements on both Event A6 detection and 100% of all expected ACK/NACKs have to be fulfilled simultaneously.

For the test to pass, the total number of successful tests shall be more than 90% of the cases with a confidence level of 95%.

### 6.6.20	UE Rx-Tx time difference measurement for propagation delay compensation

#### 6.6.20.0	Minimum conformance requirements

When UE is configured with prs-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [13] and provided with PRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_PRS, where:

TUERxTx, PRS=NPRSslotN'*Nsample*Klayer3*LCM(TPRS, TDRX)

Where:

NPRSslot is the maximum number of DL PRS resources configured in a slot,

N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumberPRS-ResourceProcessedPerSlot-r17 as specified in TS38.331 [13],

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample= 4,

TPRS is the PRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable, when Navailable>0

Where,

-	For a window W of duration LCM(TPRS, MGRP_max, TSMTC) and starting at the beginning of any PRS resource occasion, where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap, and MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as PCell for a UE supporting concurrentMeasGap-r17 and is configured with concurrent measurement gaps, otherwise MGRP max is the MGRP of configured per-UE measurement gap or per-FR measurement gap within the same FR as PCell:

-	Ntotal is the total number of PRS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Navailable is the number of PRS resource occasions that are not overlapped with:

-	For FR1: any measurement gap occasions within window W. And for a UE supporting concurrentMeasGap-r17 after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3 within the window W

No requirements shall apply if Navailable=0.

UE is only required to perform UE Rx-Tx time difference on PRS within the active DL BWP.

When UE is configured to perform UE Rx-Tx time difference measurement based on PRS, the requirements apply provided that the SCS of the PRS is same as that of the active BWP on PCell.

If PRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the

measurement report mapping requirements specified below.

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024×Tc to 985024×Tc with the resolution step of 2k×Tc.

Table 6.6.20.0-1: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, should be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC DL PRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in Table 6.6.20.0-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 6.6.20.0-2: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± [78++ℇ] | -3 | ≥[24] | 15 | ≥[4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| ± [59++ℇ] |  | ≥[52] |  | ≥[1] | Note 6 | NOTE 6 | NOTE 6 |
| ± [30++ℇ] |  | >[104] |  | ≥[1] | Note 6 | NOTE 6 | NOTE 6 |
| ± [57++ℇ] |  | ≥[24] | 30 | ≥[4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
| ± [30++ℇ] |  | ≥[48] |  | ≥[1] | NOTE 6 | NOTE 6 | NOTE 6 |
| ± [15++ℇ] |  | ≥[132] |  | ≥[1] | NOTE 6 | NOTE 6 | NOTE 6 |
| ± [29++ℇ] |  | ≥[24] | 60 | ≥[4] | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
| ± [15++ℇ] |  | ≥[64] |  | ≥[1] | NOTE 6 | NOTE 6 | NOTE 6 |
| ± [7++ℇ] |  | ≥[132] |  | ≥[1] | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [38]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [7]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 10.1.39.2-3. NOTE 8: ℇ is the margin for reporting quantization error and ℇ=16 Tc. |  |  |  |  |  |  |  |

Table 6.6.20.0-3: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [7]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

#### 6.6.20.1	NR SA FR1 UE Rx-Tx time difference measurement with PRS for RTT-based PDC

6.6.20.1.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx measurement for RTT-based PDC meets the requirements specified in TS 38.133 [6] clause 9.12.4.1 for measurement delay and clause 10.1.39.2 for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

6.6.20.1.2	Test applicability

This test applies to all types of NR UE release 17 onwards supporting rtt-BasedPDC-PRS-r17 or gNB-SideRTT-BasedPDC-r17.

6.6.20.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.20.0.

The normative reference for this requirement is TS 38.133 [6] clauses 9.12.4.1, 9.12.5, 10.1.39.2 and A.6.6.20.

6.6.20.1.4	Test description

6.6.20.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.20.1.4.1-1

Table 6.6.20.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.20-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.20-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.20-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.20.1.4.1-2.

Table 6.6.20.1.4.1-2: Initial conditions for UE Tx-Rx time difference measurement for FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.20.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram |  |  |  |

1. The general test parameter settings are set up according to Table 6.6.20.1.4.1-3.

2. Message contents are defined in clause 6.6.20.1.4.3.

3. The test scenario comprises of 1 NR Cell. The connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.20.1.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 1 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |
|  |  | 2 | 10: NRB,c = 52 |
|  |  | 3 | 40: NRB,c = 106 |
| SSB configuration |  | 1 | SSB.1 FR1 |
|  |  | 2 | SSB.1 FR1 |
|  |  | 3 | SSB.2 FR1 |
| SMTC configuration |  | 1 | SMTC.2 |
|  |  | 2 | SMTC.1 |
|  |  | 3 | SMTC.1 |
| CP length |  | 1, 2, 3 | Normal |
| DRX |  | 1, 2, 3 | OFF |
| T1 | s | 1, 2, 3 | 5 |
| T2 | s | 1, 2, 3 | 10 |

6.6.20.1.4.2	Test procedure

The test is considered with one cell (Cell 1) in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. The Cell 1 mutes PRS transmission during T1 and transmits PRS during T2.

measObject  with measObjectRxTxDiff-r17, and NR-DL-PRS-PDC-Info is defined in as per 6.6.20.1.4.3 and shall be provided to the UE during T1.

The last TTI containing the RRC configuration shall be provided to the UE T ms before the start of T2, where T =  10 ms. The UE is configured to transmit SRS during T2.

The general and cell specific test parameters for PCell is given in Table 6.6.20.1.4.1-3 and Table 6.6.20.1.5-1, respectively. In the measurement control information, a measurement object is configured for the frequency of the PCell, and. The test consists of two successive time periods, with time duration of T1, and T2 respectively.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.20.1.5-1 as appropriate.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.20.1.5-1. T2 starts.

6. The UE shall transmit periodically MeasurementReport messages.

7. After 10s wait from Step 3, the SS shall check the result-k5-r17 reported values in the periodic MeasurementReport. If the result-k5 value is outside the limits in Table 6.6.20.0-1 or the UE fails to report the measurement value for serving cell, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

8. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.6.20.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table 4.6.3-76B in 38.508-1 [14] with condition PRS_REF |
| Specific message contents exceptions for Test Configuration 6.6.20-1 and 6.6.20-2 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS15 |
| Specific message contents exceptions for Test Configuration 6.6.20-3 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS30 |

6.6.20.1.5	Test requirements

Table 6.6.20.1.4.1-3 and Table 6.6.20.1.5-1 define the primary level settings including test tolerances for UE Rx-Tx time difference measurement with PRS for RTT-based PDC in FR1 SA.

Table 6.6.20.1.5-1: Cell specific test parameters

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| PRS configuration |  | 1 | PRS.1.2 FR1 |  |
|  |  | 2 | PRS.1.2 FR1 |  |
|  |  | 3 | PRS.2.2 FR1 |  |
| PRS muting info |  | 1, 2, 3 | ‘10’ |  |
| SRS configuration |  | 1 | PDC-SRS.1 |  |
|  |  | 2 | PDC-SRS.1 |  |
|  |  | 3 | PDC-SRS.2 |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |
|  |  | 2 | -98+TT |  |
|  |  | 3 | -95+TT |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98+TT |  |
|  |  | 2 | -98+TT |  |
|  |  | 3 | -98+TT |  |
| PRS EsIot | dB | 1 | -Infinity | -2.41+TT |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS EsNoc | dB | 1 | -Infinity | -2+TT |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| PRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -100+TT |
|  |  | 2 | -Infinity | -100+TT |
|  |  | 3 | -Infinity | -97+TT |
| Io | dBm/9.36 MHz | 1 | N/A | -67.67+TT |
|  | dBm/9.36 MHz | 2 |  | -67.67+TT |
|  | dBm/38.16 MHz | 3 |  | -61.57+TT |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: The noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: PRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 6.6.20.1.5-2: SRS Configuration for FR1 Propagation Delay Compensation

|  | PDC-SRS.1 | PDC-SRS.2 |  |
| --- | --- | --- | --- |
| Field |  |  | Comment |
| c-SRS | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 |  |
| b-hop | 0 | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | usagePDC-r17 | usagePDC-r17 |  |
| startPosition | 5 | 5 | resourceMapping setting |
| nrofSymbols | 4 | 4 |  |
| repetitionFactor | n.a. | n.a. | without repetition. |
| transmissionComb | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic | Periodic |  |
| periodicityAndOffset-p | sl160, 20 | Sl320, 40 | SRS transmission periodicity |

The UE Rx-Tx time difference measurement time shall fulfils the requirements specified in clause 6.6.20.0. The UE shall perform and report the UE Rx-Tx time difference measurements for Cell 1 within the specified UE Rx-Tx time difference measurement time starting from the beginning of time interval T2.

The reported UE Rx-Tx measurement for each correct event shall be within the UE Rx-Tx reporting range specified in clause  6.6.20.0

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 6.6.20.0 for Cell 1.

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.21	UE Rx-Tx time difference measurement for propagation delay compensation with TRS

#### 6.6.21.0	Minimum conformance requirements

[38.133 clause 9.12.4.2, 9.12.5 and 10.1.25.3.1] for measurement time

When UE is configured with csi-RS-Ref-r17 in MeasObjectRxTxDiff-r17 defined in TS 38.331 [13] and provided with TRS resource configuration, the UE shall be able to measure UE Rx-Tx time difference on PCell within the measurement period TUERx-Tx_TRS, where:

TUERxTx,TRS=Nsample *Klayer3*LCM(TTRS, TDRX)

Where

Nsample is the number of UE Rx-Tx time difference measurement samples and Nsample is [4],

TTRS is the TRS resource periodicity specific for RTT-based propagation delay compensation,

TDRX is the DRX cycle length when DRX is in use, 1ms otherwise.

Klayer3 is

Ntotal / Navailable when Navailable>0

Where,

-	For a window W of duration LCM(TTRS, MGRP_max, TSMTC) and starting at the beginning of any TRS resource occasion, where TSMTC is the periodicity of SMTC for intra-frequency measurement without gap, and MGRP_max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as PCell for a UE supporting concurrentMeasGap-r17and is configured with concurrent measurement gaps, otherwise MGRP_max is the MGRP of configured per-UE measurement gap or per-FR measurement gap within the same FR as PCell:

-	Ntotal is the total number of TRS resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Navailable is the number of TRS resource occasions that are not overlapped with

-	For FR1: any measurement gap occasions within window W. And for a UE supporting concurrentMeasGap-r17 after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3 within the window W

No requirements shall apply if Navailable=0.

UE is only required to perform UE Rx-Tx time difference on TRS within the active DL BWP. When UE is configured to perform UE Rx-Tx time difference measurement based on TRS, the requirements apply provided that the SCS of the TRS is same as that of the active BWP on PCell.

If TRS resources overlap with Type 1A/1B/2 PPW, the UE is allowed longer measurement period to measure UE Rx-Tx time difference on PCell.

The UE Rx-Tx time difference measurement values contained in measurement report shall be based on the measurement report mapping requirements specified in below.

The reporting range for the absolute UE Rx-Tx time difference measurement (TUE Rx-Tx) is defined from -985024×Tc to 985024×Tc with the resolution step of 2k×Tc.

Table 6.6.21.0-1: Absolute UE Rx-Tx time difference measurement report mapping for k=5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_TIME_DIFFERENCE_0000 | TUE Rx-Tx < -985024 | Tc |
| RX-TX_TIME_DIFFERENCE_0001 | -985024  TUE Rx-Tx < -984992 | Tc |
| RX-TX_TIME_DIFFERENCE_0002 | -984992  TUE Rx-Tx < -984960 | Tc |
|  |  | … |
| RX-TX_TIME_DIFFERENCE_30782 | -32  TUE Rx-Tx < 0 | Tc |
| RX-TX_TIME_DIFFERENCE_30783 | 0  TUE Rx-Tx < 32 | Tc |
| … | … | … |
| RX-TX_TIME_DIFFERENCE_61563 | 984960  TUE Rx-Tx < 984992 | Tc |
| RX-TX_TIME_DIFFERENCE_61564 | 984992  TUE Rx-Tx < 985024 | Tc |
| RX-TX_TIME_DIFFERENCE_61565 | 985024  TUE Rx-Tx | Tc |

The error in the reported value of UE Rx-Tx time difference measurement, including both the measurement error and the reporting quantization error, should be within the accuracy requirements specified in this clause.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

-	if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within [-160, 160] msec of at least one PDC TRS resource from the serving cell (PCell).

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in Table 6.6.21.0-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

PRP|dBm according to Annex B.2.13 for a corresponding Band.

AWGN propagation condition.

Table 6.6.21.0-2: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | TRS Ês/Iot | Minimum TRS bandwidth | TRS SCS | NR operating band groupsNote 2 | IoNote 3 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 4 | dB | RB | kHz |  | dBm / SCSTRS | dBm/BW |
| [116++ℇ] | -3 | ≥[24] | 15 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| [60++ℇ] |  | ≥[52] |  | Note 5 | NOTE 5 | NOTE 5 |
| [29++ℇ] |  | >[104] |  | Note 5 | NOTE 5 | NOTE 5 |
| [56++ℇ] |  | ≥[24] | 30 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
| [29++ℇ] |  | ≥[48] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [15++ℇ] |  | ≥[132] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [29++ℇ] |  | ≥[24] | 60 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
| [14++ℇ] |  | ≥[64] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| [7++ℇ] |  | ≥[132] |  | NOTE 5 | NOTE 5 | NOTE 5 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: The Io is defined in TRS slots. The same Io range applies to TRS and non-TRS symbols. Io levels are different in TRS and non-TRS symbols within the same slot. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [7]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the TRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 6: is the margin determined from Table 10.1.39.3-3. NOTE 7: ℇ is the margin for reporting quantitization error and ℇ=16 Tc. |  |  |  |  |  |  |

Table 6.6.21.0-3: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(TRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [7]. NOTE 2: If SRS and TRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

#### 6.6.21.1	NR SA FR1 UE Rx-Tx time difference measurement with TRS for RTT-based PDC

6.6.21.1.1	Test purpose

The purpose of the test is to verify that the UE Rx-Tx measurement with TRS for RTT-based PDC meets the requirements specified in clause 9.12.4.2 for measurement delay and clause 10.1.39.3 for measurement accuracy in AWGN propagation condition in FR1 in standalone scenario.

6.6.21.1.2	Test applicability

This test applies to all types of NR UE release 17 onwards supporting rtt-BasedPDC-CSI-RS-ForTracking-r17 or gNB-SideRTT-BasedPDC-r17.

6.6.21.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.21.0.

The normative reference for this requirement is TS 38.133 [6] clauses 9.12.4.2, 9.12.5, 10.1.39.3 and A.6.6.21.

6.6.21.1.4	Test description

6.6.21.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.21.1.4.1-1

Table 6.6.21.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.21-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.21-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.21-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.21.1.4.1-2.

Table 6.6.21.1.4.1-2: Initial conditions for UE Tx-Rx time difference measurement for FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.21.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |

1. The general test parameter settings are set up according to Table 6.6.21.1.4.1-3.

2. Message contents are defined in clause 6.6.21.1.4.3.

3. The test scenario comprises of 1 NR Cell. The connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.21.1.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value |
| --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |
| RF Channel Number |  | 1, 2, 3 | 1 |
| BWchannel | MHz | 1 | 10: NRB,c = 52 |
|  |  | 2 | 10: NRB,c = 52 |
|  |  | 3 | 40: NRB,c = 106 |
| SSB configuration |  | 1 | SSB.1 FR1 |
|  |  | 2 | SSB.1 FR1 |
|  |  | 3 | SSB.2 FR1 |
| SMTC configuration |  | 1 | SMTC.2 |
|  |  | 2 | SMTC.1 |
|  |  | 3 | SMTC.1 |
| CP length |  | 1, 2, 3 | Normal |
| DRX |  | 1, 2, 3 | OFF |
| T1 | s | 1, 2, 3 | 1 |
| T2 | s | 1, 2, 3 | 1 |

6.6.21.1.4.2	Test procedure

The test is considered with one cell (Cell 1) in FR1.

The test consists of two consecutive time intervals, with duration of T1 and T2. Cell 1 does not have TRS transmission during T1 and transmits TRS during T2.

The measurement control information with MeasObjectRxTxDiff set to ‘csi-RS-Ref’ as defined in table 6.6.21.1.4.3-1, shall be provided to the UE during T1. The last TTI containing the RRC message shall be provided to the UE T ms before the start of T2, where T = 10 ms is the maximum processing time of the measurement request. The UE is configured to transmit SRS during T2.

The general and cell specific test parameters for PCell is given in Table 6.6.21.1.4.1-3 and Table 6.6.21.1.5-1, respectively. The test consists of two sub-tests with different TRS BW.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.21.1.5-1 as appropriate.

3. SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.21.1.5-1. T2 starts.

6. The UE shall transmit periodically MeasurementReport messages.

7. After 10s wait from Step 3, the SS shall check the result-k5-r17 reported values in the periodic MeasurementReport. If the result-k5 value is outside the limits in Table 6.6.21.0-2 or the UE fails to report the measurement value for serving cell, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

8. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.6.21.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table 4.6.3-76B in 38.508-1 [14] with condition CSI-RS_REF |
| Specific message contents exceptions for Test Configuration 6.6.21-1 and 6.6.21-2 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS15 |
| Specific message contents exceptions for Test Configuration 6.6.21-3 | Table 4.6.3-84AC in 38.508-1 [14] with condition SCS30 |

6.6.21.1.5	Test requirements

Table 6.6.21.1.4.1-3 and Table 6.6.21.1.5-1 define the primary level settings including test tolerances for UE Rx-Tx time difference measurement with TRS for RTT-based PDC in FR1 SA.

Table 6.6.21.1.5-1: Cell specific test parameters

| Parameter | Unit | Test configuration | Values |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  |
|  |  | 2 | TRS.1.1 TDD |  |
|  |  | 3 | TRS.1.2 TDD |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.1 |  |
| SRS configuration |  | 1 | PDC-SRS.1 |  |
|  |  | 2 | PDC-SRS.1 |  |
|  |  | 3 | PDC-SRS.2 |  |
| Noc Note 2 | dBm/SCS | 1 | -98+TT |  |
|  |  | 2 | -98+TT |  |
|  |  | 3 | -95+TT |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98+TT |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS EsIot | dB | 1 | -Infinity | -3+TT |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS EsNoc | dB | 1 | -Infinity | -3+TT |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
| TRS-RSRP Note 3 | dBm/SCS kHz | 1 | -Infinity | -101+TT |
|  |  | 2 | -Infinity | -101+TT |
|  |  | 3 | -Infinity | -98+TT |
| Io | dBm/9.36 MHz | 1 | N/A | -68.28+TT |
|  | dBm/9.36 MHz | 2 |  | -68.28+TT |
|  | dBm/38.16 MHz | 3 |  | -62.19+TT |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: TRS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

Table 6.6.21.1.5-2: SRS Configuration for FR1 Propagation Delay Compensation

|  | PDC-SRS.1 | PDC-SRS.2 |  |
| --- | --- | --- | --- |
| Field |  |  | Comment |
| c-SRS | Same as NRB,c in the test case | Same as NRB,c in the test case |  |
| b-SRS | 0 | 0 |  |
| b-hop | 0 | 0 | Frequency hopping is disabled |
| groupOrSequenceHopping | neither | neither | No group or sequence hopping |
| freqDomainPosition | 0 | 0 | Frequency domain position of SRS |
| freqDomainShift | 0 | 0 |  |
| pathlossReferenceRS ssb-Index | 0 | 0 | SSB #0 is used for SRS path loss estimation |
| usage | usagePDC-r17 | usagePDC-r17 |  |
| startPosition | 5 | 5 | resourceMapping setting |
| nrofSymbols | 4 | 4 |  |
| repetitionFactor | n.a. | n.a. | without repetition. |
| transmissionComb | n4 | n4 |  |
| combOffset-n2 | 0 | 0 | transmissionComb setting |
| cyclicShift-n2 | 0 | 0 |  |
| nrofSRS-Ports | port1 | port1 | Number of antenna ports used for SRS transmission |
| resourceType | Periodic | Periodic |  |
| periodicityAndOffset-p | sl160, 20 | Sl320, 40 | SRS transmission periodicity |

The UE Rx-Tx time difference measurement time fulfils the requirements specified in clause 6.6.21.0.

The UE shall perform and report UE Rx-Tx time difference measurement for Cell 1 within the specified UE Rx-Tx time difference measurement time starting from the beginning of T2.

The UE Rx-Tx time difference measurement time fulfils the UE Rx-Tx measurement accuracy requirements specified in clause 6.6.21.0.

For the test to pass, the ratio of successful reported values shall be more than 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.22	SA event triggered reporting tests with concurrent gaps including Pre-MG

#### 6.6.22.0	Minimum conformance requirements

Minimmum conformance requirements given in sub-clause 6.6.1.0.2 and 6.6.2.0.1. In addition, minimum conformance requirements listed below also apply.

When the UE supports concurrentMeasGapsPreMG-r18 capability, network can provide multiple measurement gap patterns with at least one of the measurement gaps is pre-configured measurement gap (Pre-MG) pattern configured by RRC message(s) as specified in TS 38.331 [2]. Requirements in this clause apply when the UE is in SA operation mode.

If the UE requires measurement gaps and/or Pre-MGs to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports concurrentMeasGapsPreMG-r18 but does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3[4], in order for the requirements in the following clauses to apply, the network can provide the UE with no more than two per-UE measurement gap patterns for monitoring all the frequency layers.

When the UE supports concurrentMeasGapsPreMG-r18, the gap association for a frequency layer is configured by the network via associatedMeasGapSSB-r17 or associatedMeasGapCSIRS-r17 in the corresponding MO(s) or via gapAssociationPRS-r17 in GapConfig-r17. In this case the gap association rules in TS 38.133 [6] clause 9.1.8.2 shall also apply to either measurement gap or Pre-MG.

When autonomous mechanism [13] is used for activation/deactivation of Pre-MG pattern, the UE shall autonomously determine the Pre-MG status only based on the measurement objects associated with the concerned Pre-MG. The related Pre-MG autonomous activation/deactivation mechanism is specified in TS 38.133 [6] clause 9.1.7.3.1.

When network-controlled mechanism [13] is used for activation/deactivation, the requirements specified in TS 38.133 [6] clause 9.1.7.3.2 apply.

When UE supports concurrentMeasGapsPreMG-r18, where at least one of the concurrent gaps is Pre-MG, applicable measurement gap patterns are listed in TS 38.133 [6] Table 9.1.2-1, and their applicability based on measurement and serving cell configurations is specified in TS 38.133 [6] Table 9.1.2-3.

The requirements in TS 38.133 [6] clause 9.1.2 are applicable for the UE, capable of concurrentMeasGapsPreMG-r18 and configured with multiple concurrent measurement gap patterns, within each activated Pre-MG occasion.

A collision between occasions of two measurement gaps where the higher priority gap is a Pre-MG and the lower priority gap may or may not be a Pre-MG is called as dynamic collision.

For a UE that supports dynamicCollision-r18:

-	Collisions between a Pre-MG and a measurement gap may occur only when the Pre-MG is activated.

-	Collisions between two Pre-MGs may occur only when both Pre-MGs are activated.

When the collision between a Pre-MG and a measurement gap or two Pre-MGs satisfy the colliding rule defined in TS 38.133 [6] clause 9.1.8.3, the UE shall perform measurements in the occasion of the non-dropped gap except the scenario specified in 38.133 [6] clause 9.1.12.5.

For a UE that does not support dynamicCollision-r18:

-	Collisions between a Pre-MG and a measurement gap or two Pre-MGs may occur when the collision rule defined in 38.133 [6] clause 9.1.8.3 is satisfied, regardless of the Pre-MG activation status.

When the collision between a Pre-MG and a measurement gap or two Pre-MGs satisfy the colliding rule defined in 38.133 [6] clause 9.1.8.3, the UE shall perform measurements in the occasion of the non-dropped gap regardless of whether it collides with the Pre-MG activation or deactivation procedure.

The requirements for the UE capable of concurrentMeasGapsPreMG-r18 apply provided that the two measurement gaps colliding with each other are configured with different priorities.

No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs.

No collisions can occur between per-FR Pre-MGs when they are configured in different FRs.

The requirements in this clause apply for UE that supports dynamicCollision-r18.

A measurement gap occasion and a Pre-MG activation/deactivation procedure collide when the ending point of the Pre-MG activation/deactivation procedure occurs anywhere within a time period starting 4 ms before the starting point of the gap occasion and ending 4 ms after the ending point of the gap occasion. The ending point of the Pre-MG activation/deactivation procedure in this collision case is defined in TS 38.133 [6] clause 8.19.2, 8.19.3 and 8.19.4.

-	When a collision occurs between a measurement gap occasion and a Pre-MG activation procedure, and the Pre-MG is configured with higher priority, the UE shall perform measurements during the measurement gap occasion and the activation of the Pre-MG is delayed until 5 ms after the ending point of the measurement gap occasion.

-	When a collision occurs between a measurement gap occasion and a Pre-MG deactivation procedure, and the Pre-MG is configured with higher priority, the measurement gap occasion shall be dropped.

A slot is interrupted by a Pre-MG if it overlaps with any occasion of the configured Pre-MG, if the Pre-MG is activated and the Pre-MG occasion is not dropped.

The normative reference for this requirement is TS 38.133 [6] clause 9.1.12.

#### 6.6.22.1	NR SA FR1 and FR1-FR1 event-triggered reporting with concurrent Type-2 measurement gap and Pre-MG

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

-	Applicability in TS 38.522 needs to be updated.

6.6.22.1.1	Test purpose

The purpose of this test is to verify that the concurrent gap with Pre-MG capable UE makes correct reporting of events. This test will partly verify the SA intra-frequency and inter-frequency NR cell search requirements in TS 38.133 [6] clauses 9.2.6, 9.3.4 and 9.3.5, respectively. Also, this test will also jointly verify pre-configured measurement gap autonomous activation/deactivation delay in TS 38.133 [6] clause 8.19.2.

6.6.22.1.2	Test applicability

T This test applies to all types of NR UE release 18 and forward supporting concurrentMeasGapsPreMG-r18, BWP adaptation of at least 2 BWPs and BWP operation without bandwidth restriction.

6.6.22.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.22.0.

The normative reference for this requirement is TS 38.133 [6] clauses A.6.6.22.1.

6.6.22.1.4	Test description

6.6.22.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.22.1.4.1-1.

Table 6.6.22.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.22.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.22.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.22.1-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.22.1.4.1-2.

Table 6.6.22.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.22.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.22.1.4.1-3.

2. Message contents are defined in clause 6.6.22.1.4.3.

3. The test scenario comprises of 2 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.22.1.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cells |  | Config 1,2,3 | NR Cells 2 and 3 | NR Cell 2 is on NR RF channel number 1. NR Cell 3 is on the NR RF channel 2. |
| Gap Pattern Id |  | Config 1,2,3 | 1 for MeasGapId #1 (80 ms MGRP) 0 for MeasGapId #2 (40 ms MGRP) | As specified in TS 38.133 [6] clause 9.1.2-1. |
| Measurement gap offset | ms | Config 1,2,3 | 79 for MeasGapId #1 4 for MeasGapId #2 |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cell |  | Config 1,2,3 | 3 s | The timing of Cell 2 is 3s later than the timing of Cell 1. |
|  |  | Config 1,2,3 | 5 ms | The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 2 |  |
| T2 | s | Config 1,2,3 | 2 |  |

6.6.22.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. SS shall transmit an RRCReconfiguration message to provide measurement configuration, BWP configurations for two dedicated BWP (BWP-1 and BWP-2) and firstActiveDownlinkBWP-Id indicating BWP-1 as the first active BWP.

3. The UE shall transmit an RRCReconfigurationComplete message.

4. The SS starts to continuously schedule DL reception on Cell 1.

5. The SS sets the parameters according to T1 in Table 6.6.22.1.5-1. T1 starts.

6. If pc_dynamicCollision_r18 = true,

-	the UE shall transmit an MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T1 is less than 902 ms, and

-	The UE shall report corresponding valid ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the gap occasions associated to MeasGapId#2.

Otherwise,

-	the UE shall transmit a MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T1 is less than 1282 ms, and

-	The UE shall report corresponding valid ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the non-dropped gap occasions associated to MeasGapId#2.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T2 expires, the SS shall set the parameters according to T2 in Table 6.6.22.1.5-1.

8. The SS sends a DCI format 1_1 to trigger BWP switching from BWP-1 to BWP 2. T2 starts.

9. The UE shall,

-	transmit a MeasurementReport message triggered by event A3 for Cell 2 and the overall reporting delay measured from the beginning of time period T2 is less than 1082 ms, and

-	The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the gap occasions associated to MeasGapId #1 or non-dropped gap occasions associated to MeasGapId #2 after MeasGapId #1 is activated, i.e. starting from the first complete gap occasion associated to MeasGapId #1 after the beginning of PCell’s DL slot i+TBWPswitchDelay+5NR slot length, where slot #i is the slot in which the DCI triggering BWP switching is received.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

10. After the SS receive the MeasurementReport message in step 9) or when T2 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

11. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12. Depending on the choice in Step 10, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13. Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.22.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.22.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -4.5dB |

Table 6.6.22.1.4.3-2: MeasConfig (Step 2, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO except that associatedMeasGapSSB-r17 = 1 | MO for intra-frequency carrier is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that associatedMeasGapSSB-r17 = 2 | MO for inter-frequency carrier is associated to MeasGapId #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConPreMG | Table 6.6.22.1.4.3-3 |  |
| } |  |  |  |

Table 6.6.22.1.4.3-3: MeasGapConfig-ConPreMG (Table 6.6.22.1.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 79 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms80 | gap pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | true | Pre-MG |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 4 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 | gap pattern 0 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present | Type-2 MG |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.22.1.4.3-4: ServingCellConfig (Step 2, Test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS, DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] SEQUENCE { | BWP-Downlink specified in TS 38.508-1 [14] Table 4.6.3-9 except that: - bwp-Id = 1, - locationAndBandwidth is set according to DLBWP.1.3 | entry 1 BWP-1 |  |
| BWP-Downlink[2] SEQUENCE { | BWP-Downlink specified in TS 38.508-1 [14] Table 4.6.3-9 except that: - bwp-Id = 2, - locationAndBandwidth is set according to DLBWP.1.2 | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWP-1 is activated at first |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink specified in TS 38.508-1 [14] Table 4.6.3-13 except that: - bwp-Id = 1, - locationAndBandwidth is set according to ULBWP.1.3 | entry 1 BWP-1 |  |
| BWP-Uplink[2] | BWP-Uplink specified in TS 38.508-1 [14] Table 4.6.3-13 except that: - bwp-Id = 2, - locationAndBandwidth is set according to ULBWP.1.2 | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 is activated at first |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.22.1.4.3-5: MeasResults (Step 6 and Step 9, test procedure)

| Derivation Path: Table H.3.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  | Step 6 |
|  | 1 |  | Step 9 |
| measResultServingMOList | Not Checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] { |  | entry 1 |  |
| physCellId | PhysCellId of NR neighbour Cell 3 |  | Step 6 |
|  | PhysCellId of NR neighbour Cell 2 |  | Step 9 |
| measResult SEQUENCE { |  |  |  |
| cellResults | Checked |  |  |
| rsIndexResults | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.22.1.5	Test requirements

Table 6.6.22.1.4.1-3 and Table 6.6.22.1.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.22.1.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Config. | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1,2,3 | 1 |  | 1 |  | 2 |  |
| Duplex mode |  | 1 | FDD |  |  |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  | N/A |  |
| BWP-1 Configuration |  | 1,2,3 | DLBWP.1.3 ULBWP.1.3 |  | N/A |  | N/A |  |
| BWP-2 Configuration |  | 1,2,3 | DLBWP.1.2 ULBWP.1.2 |  | N/A |  | N/A |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  | N/A |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | SR.2.1 TDD |  | N/A |  | N/A |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  | N/A |  |
| Dedicated CORESET RMC |  | 1 | CCR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  | N/A |  |
| SSB parameters |  | 1,2 | SSB.1 FR1 |  |  |  |  |  |
|  |  | 3 | SSB.2 FR1 |  |  |  |  |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.7 |  |
| PDSCH/PDCCH subcarrier | kHz | 1,2 | 15 |  |  |  |  |  |
| spacing |  | 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| NocNote2 | dBm/ 15 kHz |  | -98 |  | -98 |  | -98 |  |
| NocNote2 | dBm/SCS | 1,2 | -98 |  | -98 |  | -98 |  |
|  |  | 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 | dBm/SCS | 1,2 | -94+TT | -94+TT | -Infinity | -94+TT | -91+TT | -Infinity |
|  |  | 3 | -91+TT | -91+TT | -Infinity | -91+TT | -88+TT | -Infinity |
| EsIot | dB | 1,2,3 | 4+TT | -1.46+TT | -Infinity | -1.46+TT | 7+TT | -Infinity |
| EsNoc | dB | 1,2,3 | 4+TT | 4+TT | -Infinity | 4+TT | 7+TT | -Infinity |
| IoNote3 | dBm/9.36 MHz | 1,2 | -64.59 +TT | -62.25 +TT | -64.59 +TT | -62.26 +TT | -62.26 +TT | -70.05 +TT |
|  | dBm/38.16 MHz | 3 | -58.49 +TT | -56.16 +TT | -58.49 +TT | -56.15 +TT | -56.15 +TT | -63.94 +TT |
| Propagation Condition |  | 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

During T1,

-	If pc_dynamicCollision_r18 = true,

-	The UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the non-dropped MeasGapId #2 occasions;

-	The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 902 ms for NR Cell 3 from the beginning of time period T1.

-	If pc_dynamicCollision_r18 = false

-	The UE shall report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the non-dropped MeasGapId #2 occasions.

-	The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1282 ms for NR Cell 3 from the beginning of time period T1.

During T2,

-	The UE shall report ACK/NACK for PDSCHs scheduled in the slots that are not overlapped with the MeasGapId #1 occasions or non-dropped MeasGapId #2 occasions after MeasGapId #1 is activated, i.e. starting from the first complete MeasGapId #1 occasion after the beginning of PCell’s DL slot i+TBWPswitchDelay+5NR slot length

-	The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1082 ms for Cell 2 from the beginning of time period T2. The measurement reporting delay is derived based on the requirements for intra-frequency measurement in TS 38.133 [6] clause 9.2.6 plus 80 ms, considering that the frist MeasGapId #1 occasion in T2 may collide with the pre-configured gap activation delay.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%. UE is not required to report SSB time index.

NOTE 1:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.22.2	NR SA FR1 event-triggered reporting with two concurrent Pre-MGs

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

-	Applicability in TS 38.522 needs to be updated.

6.6.22.2.1	Test purpose

The purpose of this test is to verify that the UE correctly activates and deactivates the pre-MGs and makes correct measurement and reporting of an event with activated and deactivated pre-MG. This test will partly verify the multiple Pre-MG activation/deactivation delay in TS38.133 [6] clause 8.19.5.2, the intra-frequency cell search requirements in TS38.133 [6] clause 9.2.6 and inter-frequency cell search requirements in TS38.133 [6] clause 9.3.4 and 9.3.5.

6.6.22.2.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting concurrentMeasGapsPreMG-r18 and BWP adaptation of at least 2 BWPs.

6.6.22.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.22.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.22.2.

6.6.22.2.4	Test description

6.6.22.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.22.2.4.1-1.

Table 6.6.22.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.22.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.22.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.22.2-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.22.2.4.1-2.

Table 6.6.22.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.22.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.22.2.4.1-3.

2. Message contents are defined in clause 6.6.22.2.4.3.

3. The test scenario comprises of 2 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.22.2.4.1-3: General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2,3 | Cell 1 |  |
| Neighbour cell |  | 1,2,3 | Cell 2, Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1,2,3 | 1: Cell 1 and Cell 2, 2: Cell 3 |  |
| Pre-MG Gap Pattern Id |  | 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  |
| Measurement gap offset |  | 1,2,3 | 39 for MeasGapId #1 19 for MeasGapId #2 |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 | Same configuration for MO1 and MO2 |
|  |  | 2,3 | SMTC.1 | Same configuration for MO1 and MO2 |
| CSI-RS parameters |  | 1 | CSI-RS.1.2 FDD resource #0 |  |
|  |  | 2 | CSI-RS.1.2 TDD resource #0 |  |
|  |  | 3 | CSI-RS.2.2 TDD resource #0 |  |
| A3-Offset | dB | 1,2,3 | -4.5 |  |
| CP length |  | 1,2,3 | Normal |  |
| Hysteresis | dB | 1,2,3 | 0 |  |
| Time To Trigger | s | 1,2,3 | 0 |  |
| Filter coefficient |  | 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 and Cell 3 is 3 ms later than the timing of Cell 1. |
|  |  | 2,3 | 3 s | Synchronous cells |
| T1 | s | 1,2,3 | 5 |  |
| T2 | s | 1,2,3 | 0.1 |  |
| T3 | s | 1,2,3 | 5 |  |

6.6.22.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. SS shall transmit an RRCReconfiguration message to provide measurement configuration, BWP configurations for two dedicated BWP (BWP-1 and BWP-2) and firstActiveDownlinkBWP-Id indicating BWP-1 as the first active BWP.

3. The UE shall transmit an RRCReconfigurationComplete message.

4. The SS starts to continuously schedule DL reception on Cell 1.

5. The SS sets the parameters according to T1 in Table 6.6.22.2.5-1. T1 starts.

6. The UE shall

-	transmit a MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T1 is less than 1282 ms, and

-	The UE shall report corresponding valid ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the Pre-MG occasions associated to MeasGapId#2.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

7. After the SS receive the MeasurementReport message in step 6) or when T1 expires, the SS shall set the parameters according to T2 in Table 6.6.22.2.5-1.

8. The SS sends a DCI format 1_1 to trigger BWP switching from BWP-1 to BWP 2. T2 starts.

9. The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the Pre-MG occasions associated to MeasGapId #1 after MeasGapId #1 is activated, i.e. starting from the first complete gap occasion associated to MeasGapId #1 after the beginning of PCell’s DL slot i+TBWPswitchDelay+7NR slot length, where slot #i is the slot in which the DCI triggering BWP switching in step 8 is received.

Then the number of successful tests is increased by one. If the UE fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

10. When T2 expires, the SS shall set the parameters according to T3 in Table 6.6.22.2.5-1. T3 starts.

11. The UE shall,

-	transmit a MeasurementReport message triggered by event A3 for Cell 2 and the overall reporting delay measured from the beginning of time period T2 is less than 802 ms, and

-	The UE shall report corresponding valid ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the Pre-MG occasions associated to MeasGapId#1.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

12. After the SS receive the MeasurementReport message in step 11) or when T3 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

13. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

14. Depending on the choice in Step 12, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

15. Repeat step 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.22.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.22.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -4.5dB |

Table 6.6.22.2.4.3-2: MeasConfig (Step 2, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO except that associatedMeasGapSSB-r17 = 1 | MO for intra-frequency carrier is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that associatedMeasGapSSB-r17 = 2 | MO for inter-frequency carrier is associated to MeasGapId #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConPreMG | Table 6.6.22.2.4.3-3 |  |
| } |  |  |  |

Table 6.6.22.2.4.3-3: MeasGapConfig-ConPreMG (Table 6.6.22.2.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 | gap pattern 0 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | true | Pre-MG |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 19 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms80 | gap pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | true | Pre-MG |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.22.2.4.3-4: ServingCellConfig (Step 2, Test procedure)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-167 with condition MEAS, DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Downlink[1] SEQUENCE { | BWP-Downlink specified in TS 38.508-1 [14] Table 4.6.3-9 except that: - bwp-Id = 1, - locationAndBandwidth is set according to DLBWP.1.3 - preConfGapStatus = “01” | entry 1 BWP-1 |  |
| BWP-Downlink[2] SEQUENCE { | BWP-Downlink specified in TS 38.508-1 [14] Table 4.6.3-9 except that: - bwp-Id = 2, - locationAndBandwidth is set according to DLBWP.1.2 - preConfGapStatus = “10” | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | BWP-1 is activated at first |  |
| uplinkConfig SEQUENCE { |  |  |  |
| initialUplinkBWP | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 2 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink specified in TS 38.508-1 [14] Table 4.6.3-13 except that: - bwp-Id = 1, - locationAndBandwidth is set according to ULBWP.1.3 | entry 1 BWP-1 |  |
| BWP-Uplink[2] | BWP-Uplink specified in TS 38.508-1 [14] Table 4.6.3-13 except that: - bwp-Id = 2, - locationAndBandwidth is set according to ULBWP.1.2 | entry 2 BWP-2 |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | BWP-1 is activated at first |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.22.2.4.3-5: MeasResults (Step 6 and Step 11, test procedure)

| Derivation Path: Table H.3.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  | Step 6 |
|  | 1 |  | Step 11 |
| measResultServingMOList | Not Checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] { |  | entry 1 |  |
| physCellId | PhysCellId of NR neighbour Cell 3 |  | Step 6 |
|  | PhysCellId of NR neighbour Cell 2 |  | Step 11 |
| measResult SEQUENCE { |  |  |  |
| cellResults | Checked |  |  |
| rsIndexResults | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.22.2.5	Test requirement

Table 6.6.22.2.4.1-3 and Table 6.6.22.2.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.22.2.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | config. | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  |  |  |  |  |  |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |  |  |  |
| RMSI CORESET |  | 1 | CR.1.1 FDD |  |  | N/A |  |  | N/A |  |  |
| RMC |  | 2 | CR.1.1 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | N/A |  |  | N/A |  |  |
| Dedicated |  | 1 | CCR.1.2 FDD |  |  | N/A |  |  | N/A |  |  |
| CORESET RMC |  | 2 | CCR.1.2 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | N/A |  |  | N/A |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  | N/A |  |  | N/A |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  | N/A |  |  | N/A |  |  |
| Initial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  | N/A |  |  | N/A |  |  |
| BWP-1 Configuration |  | 1,2,3 | DLBWP.1.3 ULBWP.1.3 |  |  | N/A |  |  | N/A |  |  |
| BWP-2 Configuration |  | 1,2,3 | DLBWP.1.2 ULBWP.1.2 |  |  | N/A |  |  | N/A |  |  |
| RLM-RS |  | 1,2,3 | CSI-RS |  |  | N/A |  |  | N/A |  |  |
| NocNote 2 | dBm/15 kHz | 1,2,3 | -98+TT |  |  | -98+TT |  |  | -98+TT |  |  |
| NocNote 2 | dBm/SCS | 1 | -98+TT |  |  | -98+TT |  |  | -98+TT |  |  |
|  |  | 2 | -98+TT |  |  | -98+TT |  |  | -98+TT |  |  |
|  |  | 3 | -95+TT |  |  | -95+TT |  |  | -95+TT |  |  |
| EsIot | dB | 1,2,3 | 4 +TT | 4 +TT | -1.46 +TT | -∞ | -∞ | -1.46 +TT | 4 +TT | -∞ | -∞ |
| EsNoc | dB | 1,2,3 | 4 +TT | 4 +TT | 4 +TT | -∞ | -∞ | 4 +TT | 4 +TT | -∞ | -∞ |
| SS-RSRP Note 3 | dBm/SCS | 1,2 | -94 +TT | -94 +TT | -94 +TT | -∞ | -∞ | -94 +TT | -94 +TT | -∞ | -∞ |
|  |  | 3 | -91 +TT | -91 +TT | -91 +TT | -∞ | -∞ | -91 +TT | -91 +TT | -∞ | -∞ |
| Io | dBm/9.36 MHz | 1,2 | -64.60 +TT | -64.60 +TT | -62.25 +TT | -∞ | -∞ | -62.25 +TT | -64.60 +TT | -∞ | -∞ |
|  | dBm/38.16 MHz | 3 | -58.50 +TT | -58.50 +TT | -56.16 +TT | -∞ | -∞ | -56.16 +TT | -58.50 +TT | -∞ | -∞ |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T1 and T3. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |  |

During T1, UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #2; and

The UE shall send one Event A3 triggered measurement report of NR Cell 3 on RF channel 2, with a measurement reporting delay less than 1282 ms from the beginning of time period T1.

During T2 and starting from the first complete Pre-MG occasion after the beginning of PCell’s DL slot i+TBWPswitchDelay+7NR slot length as defined in clause 8.19.5.2, the UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #1.

The UE shall send one Event A3 triggered measurement report of NR Cell 2 on RF channel 1, with a measurement reporting delay less than 802 ms from the beginning of time period T3.

During T3, the UE shall report corresponding HARQ-ACK/NACK for those PDSCHs scheduled in the slots that are not overlapped with the pre-MG occasions of MeasGapId #1.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.23		SA event triggered reporting tests with concurrent gaps including NCSG

#### 6.6.23.0	Minimum conformance requirements

Minimmum conformance requirements given in sub-clause 6.6.1.0.2 and 6.6.2.0.1. In addition, minimum conformance requirements listed below also apply.

When UE supports concurrentMeasGapsNCSG-r18 capability, network can provide multiple measurement gaps with at least one of the measurement gaps is NCSG configured by RRC message(s) as specified in TS 38.331 [13].

If the UE requires concurrent measurement gaps and/or NCSG to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports concurrentMeasGapsNCSG-r18 but does not support independentGapConfig or ncsg-MeasGapPerFR-r17 as specified in [11], in order for the requirements in the following clauses to apply, the network can provide one per-UE concurrent measurement gap and one per-UE NCSG or at most two per-UE NCSGs for monitoring of all frequency layers.

For UE configured in the SA operation mode, when monitoring of multiple inter-RAT E-UTRAN carrier frequency layers, intra-frequency NR carrier frequency layers and/or inter-frequency NR carrier frequency layers as configured by PCell using gaps, each monitored carrier frequency layer, including following measurement types:

-	a measurement object with SSB based measurement,

-	E-UTRA inter-RAT measurement object,

can be associated to either one concurrent measurement gap pattern or one NCSG pattern.

Requirements for UE capable of concurrentMeasGapsNCSG-r18 apply provided that each frequency layer is only associated with one concurrent measurement gap or one NCSG, and at least one of the gaps is NCSG. There can be one or more frequency layers associated with each concurrent measurement gap or each NCSG.

When UE supports concurrentMeasGapsNCSG-r18, where at least one of the concurrent gaps is NCSG, supported concurrent measurement gap patterns are listed in 38.133[6] Table 9.1.2-1 based on the applicability specified in 38.133[6] Table 9.1.2-3, while supported NCSG patterns are listed in 38.133[6] Table 9.1.9.3-1 based on the applicability specified in 38.133[6] Table 9.1.9.3-2.

The requirements in 38.133[6] clause 9.1.2 are also applicable for the UE capable of concurrentMeasGapsNCSG-r18 and configured with multiple measurement gap or NCSG patterns within each concurrent measurement gap pattern. The requirements in 38.133[6] clause 9.1.9 are also applicable for the UE capable of concurrentMeasGapsNCSG-r18 and configured with multiple measurement gap or NCSG patterns within each NCSG pattern.

The normative reference for this requirement is TS 38.133 [6] clause 9.1.13.

#### 6.6.23.1	NR SA FR1-FR1 event-triggered reporting with concurrent Type-2 measurement gap and NCSG

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

6.6.23.1.1	Test purpose

The purpose of this test is to verify that the concurrent gaps with NCSG capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.10.

6.6.23.1.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting concurrentMeasGapsNCSG-r18.

6.6.23.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.23.0.

The normative reference for this requirement is TS 38.133 [6] clauses A.6.6.23.1.

6.6.23.1.4	Test description

6.6.23.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.23.1.4.1-1.

Table 6.6.23.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.23.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.23.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.23.1-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.23.1.4.1-2.

Table 6.6.23.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.23.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.23.1.4.1-3.

2. Message contents are defined in clause 6.6.23.1.4.3.

3. The test scenario comprises of 2 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.23.1.4.1-3: General test parameters

| Parameter | Unit | Test config. | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1,2,3 | 1,2 | Two FR1 NR carrier frequencies are used. |
| Active cell |  | 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | 1,2,3 | NR Cell 2 and 3 | NR Cell 2 is on NR RF channel number 2. NR Cell 3 is on NR RF channel number 3. |
| Measurement Gap Pattern Id |  | 1,2,3 | 0 | As specified in TS 38.133 [6] Table 9.1.2-1. |
| Measurement Gap offset |  | 1,2,3 | 39 |  |
| NCSG Pattern Id |  | 1,2,3 | 1 | As specified in TS 38.133 [6] Table 9.1.9.3-1. |
| NCSG offset |  | 1,2,3 | 4 |  |
| A3-Offset | dB | 1,2,3 | -6 |  |
| Hysteresis | dB | 1,2,3 | 0 |  |
| CP length |  | 1,2,3 | Normal |  |
| TimeToTrigger | s | 1,2,3 | 0 |  |
| Filter coefficient |  | 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | 1,2,3 | OFF | DRX is not used |
| Time offset between Cell 1 and Cell 2 |  | 1,2,3 | 3s | Synchronous. The timing of Cell 2 is 3 s later than the timing of Cell 1. |
| Time offset between Cell 1 and Cell 3 |  | 1,2,3 | 5 ms | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | 1,2,3 | 5 |  |
| T2 | s | 1,2,3 | 1.5 |  |

6.6.23.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. The SS sets the parameters according to T1 in Table 6.6.23.1.5-1. T1 starts.

3. The SS starts to continuously schedule DL reception on Cell 1.

4. SS shall transmit an RRCReconfiguration message to provide measurement configuration.

5. The UE shall transmit an RRCReconfigurationComplete message.

6. The SS shall set the parameters according to T2 in Table 6.6.23.1.5-1 when T1 expires. T2 starts.

7. The UE shall,

-	transmit a MeasurementReport message triggered by event A3 for Cell 2 and the overall reporting delay measured from the beginning of time period T2 is less than 1282 ms, and

-	transmit a MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T2 is less than 1282 ms, and

-	The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the VIL of NCSG occasions associated to MeasGapId #2 or non-dropped gap occasions associated to MeasGapId #1.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport messages in step 7) or when T2 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

9. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.23.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.22.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -6dB |

Table 6.6.23.1.4.3-2: MeasConfig (Step 4, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 3 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO | MO for Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that - ssbFrequency is set as the ARFCN-ValueNR of the SSB associated to Cell 2 - associatedMeasGapSSB-r17 = 1 | MO for Cell 2 is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[3] SEQUENCE { |  | entry 3 |  |
| measObjectId | 3 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that - ssbFrequency is set as the ARFCN-ValueNR of the SSB associated to Cell 3 - associatedMeasGapSSB-r17 = 2 | MO for Cell 3 is associated to MeasGapId #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 3 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConNCSG | Table 6.6.23.1.4.3-3 |  |
| } |  |  |  |

Table 6.6.23.1.4.3-3: MeasGapConfig-ConNCSG (Table 6.6.23.1.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 Type-2 MG |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms40 | gap pattern 0 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 NCSG |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 4 |  |  |
| mgl-r17 | ms5 |  |  |
| mgrp-r17 | ms80 | NCSG pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | true |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.23.1.4.3-4: MeasResults (Step 7, test procedure, Cell 2 and Cell 3)

| Derivation Path: Table H.3.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  | Cell 2 |
|  | 2 |  | Cell 3 |
| measResultServingMOList | Not Checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] { |  | entry 1 |  |
| physCellId | PhysCellId of NR neighbour Cell 2 |  | Cell 2 |
|  | PhysCellId of NR neighbour Cell 3 |  | Cell 3 |
| measResult SEQUENCE { |  |  |  |
| cellResults | Checked |  |  |
| rsIndexResults | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.23.1.5	Test requirements

Table 6.6.23.1.4.1-3 and Table 6.6.23.1.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.23.1.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | config | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1,2,3 | 1 |  | 2 |  | 1 |  |
| Duplex mode |  | 1 | FDD |  |  |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| Initial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  | N/A |  |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  | 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier | kHz | 1,2 | 15 |  |  |  |  |  |
| spacing |  | 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| NocNote2 | dBm/ 15 kHz | 1,2,3 | -98+TT |  | -98+TT |  | -98+TT |  |
| NocNote2 | dBm/SCS | 1,2 | -98+TT |  | -98+TT |  | -98+TT |  |
|  |  | 3 | -95+TT |  | -95+TT |  | -95+TT |  |
| SS-RSRP Note 3 | dBm/SCS | 1,2 | -94+TT | -94+TT | -Infinity | -91+TT | -Infinity | -91+TT |
|  |  | 3 | -91+TT | -91+TT | -Infinity | -88+TT | -Infinity | -88+TT |
| EsIot | dB | 1,2,3 | 4 +TT | 4+TT | -Infinity | 7+TT | -Infinity | 7+TT |
| EsNoc | dB | 1,2,3 | 4+TT | 4+TT | -Infinity | 7+TT | -Infinity | 7+TT |
| IoNote3 | dBm/ 9.36 MHz | 1,2 | -64.59 +TT | -64.59 +TT | -70.05 +TT | -62.26 +TT | -70.05 +TT | -62.26 +TT |
|  | dBm/ 38.16 MHz | 3 | -58.49 +TT | -58.49 +TT | -63.94 +TT | -56.15 +TT | -63.94 +TT | -56.15 +TT |
| Propagation Condition |  | 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1282 ms for Cell 2 and 1282 ms for Cell 3 from the beginning of time period T2.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern and the non-dropped measurement gap occasions after considering the collision between NCSG and measurement gap.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.23.2	NR SA FR1-FR1 event-triggered reporting with two concurrent NCSGs

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

6.6.23.2.1	Test purpose

The purpose of this test is to verify that the concurrent gaps with NCSG capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.10.

6.6.23.2.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting concurrentMeasGapsNCSG-r18.

6.6.23.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.23.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.23.2.

6.6.23.2.4	Test description

6.6.23.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.23.2.4.1-1.

Table 6.6.23.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.23.2-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.23.2-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.23.2-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.23.2.4.1-2.

Table 6.6.23.2.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.23.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.23.2.4.1-3.

2. Message contents are defined in clause 6.6.23.2.4.3.

3. The test scenario comprises of 3 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.23.2.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | 1,2,3 | 1,2,3 | Three FR1 NR carrier frequencies are used. |
| Active cell |  | 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | 1,2,3 | NR Cell 2 and 3 | NR Cell 2 is on NR RF channel number 2. NR Cell 3 is on NR RF channel number 3. |
| NCSG Pattern Id |  | 1,2,3 | 0 for NCSGId #0 1 for NCSGId #1 | As specified in TS 38.133 [6] Table 9.1.9.3-1. |
| NCSG offset |  | 1,2,3 | 39 for NCSGId #0 4 for NCSGId #1 |  |
| A3-Offset | dB | 1,2,3 | -6 |  |
| Hysteresis | dB | 1,2,3 | 0 |  |
| CP length |  | 1,2,3 | Normal |  |
| TimeToTrigger | s | 1,2,3 | 0 |  |
| Filter coefficient |  | 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | 1,2,3 | OFF | DRX is not used |
| Time offset between Cell 1 and Cell 2 |  | 1,2,3 | 3 s | Synchronous. The timing of Cell 2 is 3 s later than the timing of Cell 1. |
| Time offset between Cell 1 and Cell 3 |  | 1,2,3 | 5 ms | Asynchronous. The timing of Cell 3 is 5 ms later than the timing of Cell 1. |
| T1 | s | 1,2,3 | 5 |  |
| T2 | s | 1,2,3 | 1.5 |  |

6.6.23.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. The SS sets the parameters according to T1 in Table 6.6.23.2.5-1. T1 starts.

3. The SS starts to continuously schedule DL reception on Cell 1.

4. SS shall transmit an RRCReconfiguration message to provide measurement configuration.

5. The UE shall transmit an RRCReconfigurationComplete message.

6. The SS shall set the parameters according to T2 in Table 6.6.23.2.5-1 when T1 expires. T2 starts.

7. The UE shall,

-	transmit a MeasurementReport message triggered by event A3 for Cell 2 and the overall reporting delay measured from the beginning of time period T2 is less than 1282 ms, and

-	transmit a MeasurementReport message triggered by event A3 for Cell 3 and the overall reporting delay measured from the beginning of time period T2 is less than 1282 ms, and

-	The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the VIL of NCSG occasions associated to MeasGapId #2 or VIL of the non-dropped NCSG occasions associated to MeasGapId #1.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport messages in step 7) or when T2 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

9. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.23.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.23.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-4 with A3-offset = -6dB |

Table 6.6.23.2.4.3-2: MeasConfig (Step 4, test procedure)

| Derivation path: Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 3 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO | MO for Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that - ssbFrequency is set as the ARFCN-ValueNR of the SSB associated to Cell 2 - associatedMeasGapSSB-r17 = 1 | MO for Cell 2 is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[3] SEQUENCE { |  | entry 3 |  |
| measObjectId | 3 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTER-FREQ MO except that - ssbFrequency is set as the ARFCN-ValueNR of the SSB associated to Cell 3 - associatedMeasGapSSB-r17 = 2 | MO for Cell 3 is associated to MeasGapId #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 3 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConNCSG | Table 6.6.23.2.4.3-3 |  |
| } |  |  |  |

Table 6.6.23.2.4.3-3: MeasGapConfig-ConNCSG (Table 6.6.23.2.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 NCSG |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms5 |  |  |
| mgrp-r17 | ms40 | NCSG pattern 0 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | true |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 NCSG |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 4 |  |  |
| mgl-r17 | ms5 |  |  |
| mgrp-r17 | ms80 | gap pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | true |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.23.2.4.3-4: MeasResults (Step 7, test procedure, Cell 2 and Cell 3)

| Derivation Path: Table H.3.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  | Cell 2 |
|  | 2 |  | Cell 3 |
| measResultServingMOList | Not Checked |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] { |  | entry 1 |  |
| physCellId | PhysCellId of NR neighbour Cell 2 |  | Cell 2 |
|  | PhysCellId of NR neighbour Cell 3 |  | Cell 3 |
| measResult SEQUENCE { |  |  |  |
| cellResults | Checked |  |  |
| rsIndexResults | Not Checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.23.2.5	Test requirement

Table 6.6.23.2.4.1-3 and Table 6.6.23.2.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.23.2.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | config | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  | 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  | 1 | FDD |  |  |  |  |  |
|  |  | 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  | 1 | N/A |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| BWP BW | MHz | 1,2 | 10: NPRB,c = 52 |  |  |  |  |  |
|  |  | 3 | 40: NPRB,c = 106 |  |  |  |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  | N/A |  | N/A |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  |  |  |  |  |
|  |  | 2 | TRS.1.1 TDD |  |  |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |  |  |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET RMC |  | 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  | 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier | kHz | 1,2 | 15 |  |  |  |  |  |
| spacing |  | 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| NocNote2 | dBm/ 15 kHz | 1,2,3 | -98 +TT |  | -98 +TT |  | -98 +TT |  |
| NocNote2 | dBm/ SCS | 1,2 | -98 +TT |  | -98 +TT |  | -98 +TT |  |
|  |  | 3 | -95 +TT |  | -95 +TT |  | -95 +TT |  |
| SS-RSRP Note 3 | dBm/ SCS | 1,2 | -94 +TT | -94 +TT | -Infinity | -91 +TT | -Infinity | -91 +TT |
|  |  | 3 | -91 +TT | -91 +TT | -Infinity | -88 +TT | -Infinity | -88 +TT |
| EsIot | dB | 1,2,3 | 4 +TT | 4 +TT | -Infinity | 7 +TT | -Infinity | 7 +TT |
| EsNoc | dB | 1,2,3 | 4 +TT | 4 +TT | -Infinity | 7 +TT | -Infinity | 7 +TT |
| IoNote3 | dBm/9.36 MHz | 1,2 | -64.59 +TT | -64.59 +TT | -70.05 +TT | -62.26 +TT | -70.05 +TT | -62.26 +TT |
|  | dBm/38.16 MHz | 3 | -58.49 +TT | -58.49 +TT | -63.94 +TT | -56.15 +TT | -63.94 +TT | -56.15 +TT |
| Propagation Condition |  | 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 1282 ms for Cell 2 and 1282 ms for cell 3 from the beginning of time period T2.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of non-dropped NCSG pattern after considering the collision between NCSGs.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.23.3	NR SA FR1 and FR1-FR1 event triggered reporting with concurrent Type-2 measurement gap and NCSG

Editor’s Notes: This test case is incomplete in following aspects:

-	TT analysis is still missing;

6.6.23.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the cell search requirements on SCC with deactivated SCell in TS 38.133 [6] clauses 9.2.7.1 and 9.2.7.2.

6.6.23.3.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting 2CC DL CA and concurrentMeasGapsNCSG-r18.

6.6.23.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.23.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.23.3.

6.6.23.3.4	Test description

6.6.23.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.23.3.4.1-1.

Table 6.6.23.3.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.23.3-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.23.3-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.23.3-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test requirement and the DUT according to the parameters in Table 6.6.23.3.4.1-2.

Table 6.6.23.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E (Note 1), Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.23.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. |  |  |
| Note 1: For UE supporting intra-band contiguous CA, the test frequencies are selected according to E.1.4.2. For UE supporting intra-band non-contiguous CA, the test frequencies are selected according to E.1.4.3. For UE supporting inter-band CA, the the test frequencies are selected according to E.1.4.1 |  |  |  |

1. The test parameters for PCell and neighbour cell are given in Table 6.6.23.3.4.1-3 below.

2. Message contents are defined in clause 6.6.23.3.4.3.

3. The test scenario comprises of 3 NR carriers and 3 cells. NR Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 6.6.23.3.4.1-3: General test parameters

| Parameter | Unit | Test config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2,3 | Cell 1 |  |
| Configured deactivated SCell |  | 1,2,3 | Cell 2 |  |
| Neighbour cell |  | 1,2,3 | Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1,2,3 | 1: Cell 1 |  |
| RF Channel Number |  | 1,2,3 | 2: Cell 2 and Cell 3 |  |
| NCSG type |  | 1,2,3 | Per-UE NCSG |  |
| NCSG pattern Id |  | 1,2,3 | ID # 1 | Defined in TS 38.133 [6] Table 9.1.9.3-1 |
| NCSG offset |  | 1,2,3 | 39 |  |
| Measurement gap type |  | 1,2,3 | Per-UE MG |  |
| MG pattern Id |  | 1,2,3 | ID #1 | Defined in TS 38.133 [6] Table 9.1.2-1 |
| Measurement gap offset |  | 1,2,3 | 19 |  |
| MGTA | ms | 1,2,3 | 0 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A6-Offset | dB | 1,2,3 | -4.5 |  |
| CP length |  | 1,2,3 | Normal |  |
| Hysteresis | dB | 1,2,3 | 0 |  |
| Time To Trigger | s | 1,2,3 | 0 |  |
| Filter coefficient |  | 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | 1,2,3 |  | OFF |
| SCell measurement cycle (measCycleSCell) | ms | 1,2,3 | 640 |  |
| Cell 2 timing offset to Cell 1 | s | 1,2,3 | 0 |  |
| Time alignment error between Cell 2 and Cell 1 | s | 1,2,3 | ≤ Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Cell 3 timing offset to Cell 1 | s | 1,2,3 | 3 | Synchronous cells |
| T1 | s | 1,2,3 | 5 |  |
| T2 | s | 1,2,3 | 5 |  |

6.6.23.3.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. The SS sets the parameters according to T1 in Table 6.6.23.3.5-1. T1 starts.

3. The SS starts to continuously schedule DL reception on Cell 1.

4. SS shall transmit an RRCReconfiguration message to provide measurement configuration and add Cell 2 as a deactivated SCell.

5. The UE shall transmit an RRCReconfigurationComplete message.

6. The SS shall set the parameters according to T2 in Table 6.6.23.3.5-1 when T1 expires. T2 starts.

7. The UE shall,

-	transmit a MeasurementReport message triggered by event A6 and the overall reporting delay measured from the beginning of time period T2 is less than [6402] ms, and

-	The UE shall report ACK/NACK for every PDSCHs scheduled in the slots that are not overlapped with the VIL of NCSG occasions associated to MeasGapId #1 or gap occasions associated to MeasGapId #2.

Then the number of successful tests is increased by one. If the UE fails to report the event within the overall delay requirement or fails to send valid ACK/NACK in the required slots, then the number of failure tests is increased by one.

8. After the SS receive the MeasurementReport messages in step 7) or when T2 expires, the SS shall

-	sends an RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources, or

-	switch the UE off.

9. Set NR Cell 2 physical cell identity = ((current NR Cell 2 physical cell identity + 3) mod 1008) and NR Cell 3 physical cell identity = ((current NR Cell 3 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

10. Depending on the choice in Step 8, the SS:

-	if the RRCRelease message has been sent, transmits in NR Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5), or

-	if the UE has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11. Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.23.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.23.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-7 with condition Deactivated SCell and INTRA-FREQ |

Table 6.6.23.3.4.3-2: MeasConfig (Step 4, test procedure)

| Derivation path: Table H.3.1-2 with condition Deactivated SCell and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition INTRA-FREQ MO | MO for Cell 1 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT specified in Table H.3.1-3 with condition Deactivated SCell except that - associatedMeasGapSSB-r17 = 1 | MO for Cell 2 is associated to MeasGapId #1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-A6 | Table 6.6.23.3.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-ConNCSG | Table 6.6.23.3.4.3-4 |  |
| } |  |  |  |

Table 6.6.23.3.4.3-3: ReportConfigNR (Table 6.6.23.3.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA6 SEQUENCE { |  |  |  |
| a6-Offset CHOICE { |  |  |  |
| rsrp | -9 | Acutal a6-offset = -4.5dB |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | r2 |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.23.3.4.3-4: MeasGapConfig-ConNCSG (Table 6.6.23.3.4.3-2)

| Derivation Path: Table H.3.1-6 with condition MGe |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig ::= SEQUENCE { |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ | 2 entries |  |  |
| GapConfig-r17[1] SEQUENCE{ |  | entry 1 NCSG |  |
| measGapId-r17 | 1 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 39 |  |  |
| mgl-r17 | ms5 |  |  |
| mgrp-r17 | ms80 | NCSG pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | true |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 1 | High priority |  |
| } |  |  |  |
| GapConfig-r17[2] SEQUENCE{ |  | entry 2 Type-2 MG |  |
| measGapId-r17 | 2 |  |  |
| gapType-r17 | perUE |  |  |
| gapOffset-r17 | 19 |  |  |
| mgl-r17 | ms6 |  |  |
| mgrp-r17 | ms80 | gap pattern 1 |  |
| mgta-r17 | ms0 |  |  |
| refServCellIndicator-r17 | Not present |  |  |
| refFR2-ServCellAsyncCA-r17 | Not present |  |  |
| preConfigInd-r17 | Not present |  |  |
| ncsgInd-r17 | Not present |  |  |
| gapAssociationPRS-r17 | Not present |  |  |
| gapSharing-r17 | Not present |  |  |
| gapPriority-r17 | 2 | Low priority |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.23.3.4.3-5: CellGroupConfig (Step 4, test procedure)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition SCell_Add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated SEQUENCE { |  |  |  |
| servingCellMO | 2 | Cell 2 is associated to MO #2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.23.3.5	Test requirement

Table 6.6.23.3.4.1-3 and Table 6.6.23.3.5-1 define the primary level settings including test tolerances for the test.

Table 6.6.23.3.5-1: NR Cell specific test parameters

| Parameter | Unit | Test | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | config | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  |  |  |  |  |
|  |  | 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  | 3 | TDDConf.2.1 |  |  |  |  |  |
| PDSCH RMC |  | 1 | SR.1.1 FDD |  | N/A |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET RMC |  | 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET RMC |  | 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  |  |  |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  |
|  |  | 3 | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  |
| Initial BWP configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Active BWP configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| NocNote2 | dBm/ SCS | 1,2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| NocNote2 | dBm/ 15 kHz | 1,2,3 | -98 |  |  |  |  |  |
| EsIot | dB | 1,2,3 | 4 +TT | 4 +TT | 4 +TT | -1.46 +TT | -Infinity | -1.46 +TT |
| EsNoc | dB | 1,2,3 | 4 +TT | 4 +TT | 4 +TT | 4 +TT | -Infinity | 4 +TT |
| SS-RSRP Note 3 | dBm/ SCS | 1,2 | -94 +TT | -94 +TT | -94 +TT | -94 +TT | -Infinity | -94 +TT |
|  |  | 3 | -91 +TT | -91 +TT | -91 +TT | -91 +TT | -Infinity | -91 +TT |
| Io | dBm/9.36 MHz | 1,2 | -64.60 +TT | -64.60 +TT | -64.60 +TT | -62.25 +TT | -64.60 +TT | -62.25 +TT |
|  | dBm/38.16 MHz | 3 | -58.50 +TT | -58.50 +TT | -58.50 +TT | -56.16 +TT | -58.50 +TT | -56.16 +TT |
| Propagation Condition |  | 1,2,3 | AWGN |  |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The UE shall send one Event A6 triggered measurement report, with a measurement reporting delay less than [6402] ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall be scheduled on PCell continuously throughout the test. From the start of T1 until the measurement report is received during T2, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots except for the case where PDSCH or PUCCH is overlapped with the VIL of NCSG pattern and MGL of MG pattern.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

For a test to be considered successful requirements on both Event A6 detection and percentage of transmitted ACK/NACKs have to be fulfilled simultaneously.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.24	SA event triggered reporting tests with NeedForGap

#### 6.6.24.0	Minimum conformance requirements

6.6.24.0.1	Minimum conformance requirements for intra-frequency event-triggered measurement without gap

[TS 38.133 [6], clause 9.2.5]

The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra-frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz.

Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms

Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms

Where:

TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TPSS/SSS_sync_intra is given in Table 6.6.24.0.1-1 for FR1. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TPSS/SSS_sync_intra is given in Table 6.6.24.0.1-3 for FR1.

TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured.

-	For UE indicatting no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TSSB_time_index_intra is given in Table 6.6.24.0.1-2 for FR1. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TSSB_time_index_intra is given in Table 6.6.24.0.1-4 for FR1.

-	T SSB_measurement_period_intra: equal to a measurement period of SSB based measurement

-	For UE indicating no-gap-no-interruption via NeedForInterruptionInfoNR-r18, TSSB_measurement_period_intra is given in Table 6.6.24.0.1-5 for FR1. For UE indicating no-gap-with-interruption via NeedForInterruptionInfoNR-r18, TSSB_measurement_period_intra is given in Table 6.6.24.0.1-6 for FR1.

-	CSSFintra: it is a carrier specific scaling factor and is determined

-	according to CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e.

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP, or

-	 when intra-frequency SMTC is fully non overlapping with GAP for UE indicating no-gap-with-interruption,  or

-	 when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP for UE indicating no-gap-no-interruption, or

if the high layer in TS 38.331 [13] signalling of smtc2 is configured, the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc2; Otherwise the assumed periodicity of intra-frequency SMTC occasions corresponds to the value of higher layer parameter smtc1.

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  xRP_max), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap.

Kp = 1 when Navailable = 0.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG, also xRP = MGRP for periodic MUSIM gap.

Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W.

-	When the target SSB is completely contained in active BWP of UE or the active downlink BWP is initial BWP, the intra-frequency measurement shall be conducted without gap and without interruption regardless of the NeedForGaps’ status reporting.

Table 6.6.24.0.1-1: Time period for PSS/SSS detection, (Frequency range 1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, ceil( 5 x Kp) x SMTC period )Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2 Note 2x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(5 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. |  |

Table 6.6.24.0.1-2: Time period for time index detection (Frequency range 1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 3 x Kp ) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 Note 2 x 3 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(3 x Kp) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 |  |

Table 6.6.24.0.1-3: Time period for PSS/SSS detection for UE indicating no-gap-with-interruption, (Frequency range 1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max( 600ms, 5 x max (80ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320ms] | max( 600ms, ceil(M2 Note 2x 5) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320ms] | 5 x [DRX cycle x] CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1. NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1. NOTE 5: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 6.6.24.0.1-4: Time period for time index detection for UE indicating no-gap-with-interruption (Frequency range 1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 3 x max (80ms, SMTC period ))Note 1 x CSSFintra |
| [DRX cycle≤ 320ms] | max(120ms, ceil (M2 Note 2 x 3) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| [DRX cycle>320ms] | 3 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms;,otherwise M2=1 NOTE 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured and UE supports measurementEnhancementCA-r17, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2=1 NOTE 5: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates [no gap with interruption]. |  |

Table 6.6.24.0.1-5: Measurement period for intra-frequency measurements without gaps (Frequency range 1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil( 5 x Kp ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 6.6.24.0.1-6: Measurement period for intra-frequency measurements without gaps for UE indicating no-gap-with-interruption (Frequency range 1)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(200ms, 5 x (80ms, SMTC period ))Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x 5) x [max(80ms, SMTC period,DRX cycle)]) x CSSFintra |
| DRX cycle>320ms | 5 x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Requirements only apply when measurement gap is not configured, or measurement gap is fully non-overlapped with SMTC on any carrier on which UE indicates ‘no-gap-with-interruption’ via NeedForInterruptionInfoNR-r18. |  |

6.6.24.0.2	Minimum conformance requirements for inter-frequency event-triggered measurement without gap

[TS 38.133 [6], clause 9.3.9]

UE satisfying the applicability conditions specified in TS 38.133 [6] 9.3.1 on the requirement in this clause shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index (reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured) or deriveSSB-IndexFromCellInter-r17 is configured for the FR1 and FR2-1 target frequency layers and and UE supporting deriveSSB-IndexFromCellInterNon-NCSG-r17. Otherwise UE shall be able to identify a new detectable inter-frequency cell within Tidentify_inter_with_index. The UE shall be able to identify a new detectable inter-frequency SS block of an already detected cell within Tidentify_inter_without_index.

-	For inter-frequency SSB based measurements without measurement gaps in active BWP, it is assumed that when UE performs inter-frequency measurements without measurement gaps in a TDD bands on FR1 and FR2, SFN and frame boundary across serving cell and inter-frequency neighbor cells is aligned

Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms

Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Where:

TPSS/SSS_sync_inter: it is the time period used in PSS/SSS detection

-	For UE indicating no-gap-no-interurption, TPSS/SSS_sync_inter is given in Table 6.6.24.0.2-1 for FR1.

-	For UE indicating no-gap-with-interruption, TPSS/SSS_sync_inter is given in Table 6.6.24.0.2-2 for FR1.

-	TSSB_time_index_inter: it is the time period used to acquire the index of the SSB being measured

-	For UE indicating no-gap-no-interurption, TSSB_time_index_inter is given in Table 6.6.24.0.2-3 for FR1.

-	For UE indicating no-gap-with-interruption, TSSB_time_index_inter is given in Table 6.6.24.0.2-4 for FR1.

-	T SSB_measurement_period_inter: equal to a measurement period of SSB based measurement

-	For UE indicating no-gap-no-interurption, T SSB_measurement_period_inter is given in Table 6.6.24.0.2-5 for FR1.

-	For UE indicating no-gap-with-interruption, T SSB_measurement_period_inter is given in Table 6.6.24.0.2-6 for FR1.

CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1 for measurement conducted outside GAP, i.e. when interfrequency SMTC is fully non overlapping or partially overlapping with GAPs.

-	when inter-frequency SMTC is fully non overlapping or partially overlapping with measurement gaps for UE indicating no-gap-no-interruption or

-	when inter-frequency SMTC is fully non overlapping with measurement gaps for UE indicating n- gap-with-interruption,

If the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR for the inter-frequency measurement or the UE indicates either no-gap-with-interruption or no-gap-no-interruption via NeedForInterruptionInfoNR-r18,

Mpss/sss_sync_inter: For FR1, Mpss/sss_sync_inter = 8.

MSSB_index_inter: For FR1, MSSB_index_inter = 3.

Mmeas_period_inter: For FR1, Mmeas_period_inter = 8.

When UE is not configured with or UE does not support concurrent GAPs and the UE is not configured with MUSIM gaps:

When interfrequency SMTC is fully non overlapping with measurement gaps or NCSG, or interfrequency SMTC is fully overlapping with MGs or NCSG, Kp =1.

When interfrequency SMTC is partially overlapping with measurement gaps, Kp =  1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When inter-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP.

For calculation of Kp, if the high layer signalling (TS 38.331 [13]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

Table 6.6.24.0.2-1: Time period for PSS/SSS detection, (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, ceil(Mpss/sss_sync_inter x Kp) x SMTC period )Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max( 600ms, ceil(M2x Mpss/sss_sync_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil(Mpss/sss_sync_inter x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 6.6.24.0.2-2: Time period for PSS/SSS detection, when UE indicate no-gap-with-interruption (FR1)

| DRX cycle | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | max( 600ms, Mpss/sss_sync_inter x max(80ms, SMTC period) )Note 1 x CSSFinter |
| [DRX cycle≤ 320ms] | max( 600ms, ceil(M2 x Mpss/sss_sync_inter) x max(80ms, SMTC period,DRX cycle)) x CSSFinter |
| [DRX cycle>320ms] | Mpss/sss_sync_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 6.6.24.0.2-3: Time period for time index detection (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil(MSSB_index_inter x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 x MSSB_index_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil(MSSB_index_inter x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 6.6.24.0.2-4: Time period for time index detection, when UE indicate no-gap-with-interruption (FR1)

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, MSSB_index_inter x max(80ms, SMTC period))Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (M2 x MSSB_index_inter) x [max(80ms, SMTC period, DRX cycle)]) x CSSFinter |
| DRX cycle>320ms | MSSB_index_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: Void NOTE 3: When highSpeedMeasInterFreq-r17 is not configured, M2 = 1.5; When highSpeedMeasInterFreq-r17 is configured, M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 |  |

Table 6.6.24.0.2-5: Measurement period for inter-frequency measurements without gaps (FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, ceil(Mmeas_period_inter x Kp) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x Mmeas_period_inter x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | ceil( Mmeas_period_inter x Kp ) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

Table 6.6.24.0.2-6: Measurement period for inter-frequency measurements without gaps when UE indicate [no-gap-with-interruption] (FR1)

| DRX cycle | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | max(200ms, Mmeas_period_inter x max(80ms, SMTC period))Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(200ms, ceil(1.5x Mmeas_period_inter) x max(80ms, SMTC period, DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Mmeas_period_inter x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

6.6.24.0.3	Minimum conformance requirements for measurements without gap carried out by UE supporting NeedForInterruptionInfoNR

[TS 38.133 [6], clause 8.2.2.2.19]

When a UE supports NeedForInterruptionInfoNR-r18 measurements and indicates no-gap-with-interruption on intra-frequency SSB-based or inter-frequency SSB-based measurements, the UE is allowed to cause interruptions while performing measurements on the frequency layers of the bands for which no-gap-with-interruption is indicated. Requirements in this section apply only when the UE is in SA operation mode.

The UE is allowed to cause interruption with interruption ratio no more than the requirements specified below upon UE measurements on a specific frequency layer that corresponds to the configured MO, where Tcycle,i is the interruption cycle on a certain frequency layer i, specified in Table 6.6.24.0.3-1, where CSSFoutside_gap,i is defined in TS 38.133 [6] clause 9.1.5.1 for measurement conducted outside measurement gaps.

Table 6.6.24.0.3-1: Tcycle,i length for inter/intra-frequency measurement target carrier i

| DRX cycle | TCycle,i |
| --- | --- |
| No DRX | max (80ms, SMTC period) x CSSFoutside_gap,i |
| DRX cycle ≤ 320ms | 1.5*max(80ms, SMTC period, DRX cycle) x CSSFoutside_gap,i |
| DRX cycle>320ms | DRX cycle x CSSFoutside_gap,i |

UE is allowed to cause interruption on a certain frequency layer i with the maximum interruption ratio that equals 2LTcycle,i.

The total allowed maximum interruption ratio (D) on each of the active serving cells due to UE measurements without gap applied in this sub-clause is specified as

D= i=1N2LTcycle,i

Where,

-	N is the total number of configured SSB based frequency layers to be measured outside gap including intra-frequency and inter-frequency target carriers where UE indicates that interruption is needed through [no-gap-with-interruption], and

-	L is the maximum interruption length for each interruption occasion specified in the Table 6.6.24.0.3-2.

When no SMTC occasion of the NR measurement objects occurs within the time period which starts from [4ms] before the starting point of the DRX ON duration to [4ms] after the ending point of the DRX ON duration, UE is not allowed to cause interruption during DRX ON duration except the time duration extended due to drx-inactivityTimer.

The interruptions are allowed for all the active serving cells in the same FR as all NR MOs being measured with interruption if UE supports per-FR measurement gaps, and all the active serving cells if UE does not support per-FR measurement gaps.

Table 6.6.24.0.3-2: Interruption length L in FR1

|  | SCS (kHz) of victim cell | NR Slot length (ms) of victim cell | Number of interrupted slots in the victim cell (slots) | Interruption length L (ms) |
| --- | --- | --- | --- | --- |
| 0 | 15 | 1 | [1] | [1] |
| 1 | 30 | 0.5 | [2] | [1] |
| 2 | 60 | 0.25 | [4] | [1] |

#### 6.6.24.1	NR SA FR1 event-triggered reporting without gap with interruptions

6.6.24.1.1	Test purpose

To verify the UE's ability to perform correct reporting of an event within intra-frequency cell search without gap and to verify that the interruption ratio does not exceed the limit for the NR PCell during the measurement without gaps and with interruptions.

6.6.24.1.2	Test applicability

This test applies to all types of NR UE from release 18 onwards supporting CSI-RS-based RLM, BWP operation without bandwidth restriction and NR NeedForGap capability. The UE is capable of nr-NeedForGap-Reporting-r16, nr-NeedForInterruptionReport-r18 and reports ‘no-gap’ in NeedForGapsIntraFreq-r16 and ‘no-gap-with-interruption’ in interruptionIndication-r18 for intra-frequency.

6.6.24.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.24.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.24.1.

6.6.24.1.4	Test description

6.6.24.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.6.24.1.4.1-1.

Table 6.6.24.1.4.1-1: NR SA FR1 event-triggered reporting without gap with interruptions test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.24.1.4.1-2.

Table 6.6.24.1.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap with interruptions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 6.6.24.1.4.1-3.

2.	Message contents are defined in clause 6.6.24.1.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell(Cell 1) and FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The power levels and settings are according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.24.1.4.1-3: General test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 without DRX

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 10 |  |

6.6.24.1.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table 6.6.24.1.4.1-1, 6.6.24.1.4.1-3 and 6.6.24.1.5-1. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. The PCell shall continuously scheduled with data in the DL starting from T1 until the UE has sent the measurement report during T2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

The UE is capable of measurements without gaps with interruption and report ‘no-gap’ through NeedForGapsIntraFreq-r16 and ‘no-gap-with-interruption’ in interruptionIndication-r18 for PCell. UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.24.1.5-1.

3.	SS shall transmit an RRCReconfiguration message with needForGapsConfigNR-r16 and needForInterruptionConfigNR-r18 enabled.

4.	The UE shall transmit RRCReconfigurationComplete message with NeedForGapsIntraFreq-r16 set to ‘no-gap’ and intraFreq-needForInterruption-r18 set to ‘no-gap-with-interruption’.

5.	SS shall transmit an RRCReconfiguration message.

6.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

7.	SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

8.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.24.1.5-1. T2 start.

9.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1002ms and if UE transmits ACK/NACK in PCell with a missed ACK/NACK rate of no more than 2.5% from T2, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or the rate of missed ACK/NACK in PCell exceeds 2.5% from T2, then the number of failure tests is increased by one.

10.	After the SS receive the MeasurementReport message in step 7 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

11.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12.	Depending on the choice in Step 10, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.24.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.24.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions (step 5) | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC pattern 1 and Synchronous cells for 6.6.24.1-1 and 6.6.24.1-2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC pattern 1 and Synchronous cells for 6.6.24.1-3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ |

Table 6.6.24.1.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

Table 6.6.24.1.4.3-3: RRCReconfiguration (step 3)

| Derivation Path: Table H.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| NeedForGapsConfigNR-r16 CHOICE { |  |  |  |
| setup SEQUENCE{ |  |  |  |
| requestedTargetBandFilterNR-r16 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR { | 1 entries |  |  |
| FreqBandIndicatorNR[0] | tested band |  |  |
| } |  |  |  |
| } |  |  |  |
| needForInterruptionConfigNR-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.24.1.4.3-4: RRCReconfigurationComplete (step 4)

| Derivation Path: TS 36.508 [25], Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |  |
| needForGapsInfoNR-r16 SEQUENCE { |  |  |  |  |
| NeedForGapsIntraFreqList-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 { |  |  |  |  |
| NeedForGapsIntraFreq-r16[0] SEQUENCE { |  |  |  |  |
| servCellId-r16 |  | 0 |  |  |
| gapIndicationIntra-r16 |  | no-gap |  |  |
| } |  |  |  |  |
| NeedForGapsBandListNR-r16 SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 { |  |  |  |  |
| NeedForGapsInterFreq-r16[0] SEQUENCE { |  |  |  |  |
| bandNR-r16 |  | Tested band |  |  |
| gapIndicationInter-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| needForInterruptionInfoNR-r18 SEQUENCE { |  |  |  |  |
| NeedForInterruptionIntraFreqList-r18 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | no-gap-with-interruption |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NeedForInterruptionBandListNR-r18 SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.6.24.1.5	Test requirements

Table 6.6.24.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.24.1.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for PCell in FR1 without DRX

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| Initial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.2 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.2 |  |
| RLM-RS |  | 1, 2, 3 | CSI-RS |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25+TT | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16+TT | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall be continuously scheduled on PCell during the entire length of T1 and T2.

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1000 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

During 1000ms from the beginning of time period T2, the UE shall transmit ACK/NACK in PCell and the rate of missed ACK/NACK shall no more than 2.5%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.24.2	NR SA FR1-FR1 event-triggered reporting without gap with interruption with SSB index

6.6.24.2.1	Test purpose

To verify the UE's ability to perform correct reporting of an event within inter-frequency cell search without gap and to verify that the interruption ratio does not exceed the limit for the NR PCell during the measurement without gaps and with interruptions.

6.6.24.2.2	Test applicability

This test applies to all types of NR UE from release 18 onwards supporting NR NeedForGap capability. The UE is capable of nr-NeedForGap-Reporting-r16, nr-NeedForInterruptionReport-r18 and reports ‘no-gap’ in NeedForGapsInterFreq-r16 and ‘no-gap-with-interruption’ in interruptionIndication-r18 for the target frequency given the serving frequency.

6.6.24.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.24.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.24.2.

6.6.24.2.4	Test description

6.6.24.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.6.24.2.4.1-1.

Table 6.6.24.2.4.1-1: NR SA event triggered reporting tests for FR1 for inter-frequency measurement without gap with interruption test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.24.2.4.1-2.

Table 6.6.24.2.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting for inter-frequency measurement without gap with interruptions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 6.6.24.2.4.1-3.

2.	Message contents are defined in clause 6.6.24.2.4.3.

3.	There are two NR carriers and one cell on each carrier in the test. Cell 1 (Pcell) is on NR RF channel 1, and is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter neighbour cell) is on NR RF channel 2.

Table 6.6.24.2.4.1-3: General test parameters for SA event triggered reporting tests for FR1 for inter-frequency measurement without gap with interruption

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies are used |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1 |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 2 |  |

6.6.24.2.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters for PCell are given in Table 6.6.24.2.4.1-1, 6.6.24.2.4.1-3 and 6.6.24.2.5-1. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. The PCell shall continuously scheduled with data in the DL starting from T2 until the UE has sent the measurement report during T2.

NR RF channel 1 and NR RF channel 2 should be selected such that UE reports ‘no-gap’ and ‘no-gap-with-interruption’ for the target frequency on NR RF channel 2

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.24.2.5-1.

3.	SS shall transmit an RRCReconfiguration message with needForGapsConfigNR-r16 and needForInterruptionConfigNR-r18 enabled.

4.	The UE shall transmit RRCReconfigurationComplete message with NeedForGapsInterFreq-r16 set to ‘no-gap’ and interFreq-needForInterruption-r18 set to ‘no-gap-with-interruption’.

5.	SS shall transmit an RRCReconfiguration message.

6.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

7.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.24.2.5-1. T2 start.

8.	SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

9.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 1522ms and if UE transmits ACK/NACK in PCell with a missed ACK/NACK rate of no more than 2.5% from T2, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or the rate of missed ACK/NACK in PCell exceeds 2.5% from T2, then the number of failure tests is increased by one.

10.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

11.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12.	Depending on the choice in Step 10, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.24.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.24.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions (step 5) | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-3 with Condition INTER-FREQ MO, SSB.5 FR1 and SMTC.5 for configuration 6.6.24.2-1 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.5 FR1 and SMTC.4 and synchronous cells for configuration 6.6.24.2-2 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.6 FR1, SMTC.4 and synchronous cells for configuration 6.6.24.2-3 Table H.3.1-4 with A3-offset = -6dB and Condition SSB Index Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ and SSB Index |

Table 6.6.24.2.4.3-2: RRCReconfiguration (step 3)

| Derivation Path: Table H.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| NeedForGapsConfigNR-r16 CHOICE { |  |  |  |
| setup SEQUENCE{ |  |  |  |
| requestedTargetBandFilterNR-r16 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR { | 1 entries |  |  |
| FreqBandIndicatorNR[0] | tested band |  |  |
| } |  |  |  |
| } |  |  |  |
| needForInterruptionConfigNR-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.24.2.4.3-3: RRCReconfigurationComplete (step 4)

| Derivation Path: TS 36.508 [25], Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |  |
| needForGapsInfoNR-r16 SEQUENCE { |  |  |  |  |
| NeedForGapsIntraFreqList-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 { |  |  |  |  |
| NeedForGapsIntraFreq-r16[0] SEQUENCE { |  |  |  |  |
| servCellId-r16 |  | 0 |  |  |
| gapIndicationIntra-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| NeedForGapsBandListNR-r16 SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 { |  |  |  |  |
| NeedForGapsInterFreq-r16[0] SEQUENCE { |  |  |  |  |
| bandNR-r16 |  | Tested band |  |  |
| gapIndicationInter-r16 |  | no-gap |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| needForInterruptionInfoNR-r18 SEQUENCE { |  |  |  |  |
| NeedForInterruptionIntraFreqList-r18 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NeedForInterruptionBandListNR-r18 SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | no-gap-with-interruption |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.6.24.2.5	Test requirements

Table 6.6.24.2.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.24.2.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without gap

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.4 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1520 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

During 1520ms from the beginning of time period T2, the UE shall transmit ACK/NACK in PCell and the rate of missed ACK/NACKs shall no more than 2.5%.

UE is required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.24.3	NR SA FR1-FR1 event-triggered reporting without gap with interruption

6.6.24.3.1	Test purpose

To verify the UE's ability to perform correct reporting of an event within inter-frequency cell search without gap and to verify that the interruption ratio does not exceed the limit for the NR PCell during the measurement without gaps and with interruptions.

6.6.24.3.2	Test applicability

This test applies to all types of NR UE from release 18 onwards supporting NR NeedForGap capability. The UE is capable of nr-NeedForGap-Reporting-r16, nr-NeedForInterruptionReport-r18 and reports ‘no-gap’ in NeedForGapsInterFreq-r16 and ‘no-gap-with-interruption’ in interruptionIndication-r18 for the target frequency given the serving frequency.

6.6.24.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.24.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.24.3.

6.6.24.3.4	Test description

6.6.24.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.6.24.3.4.1-1.

Table 6.6.24.3.4.1-1: NR SA event triggered reporting tests for FR1 for inter-frequency measurement without gap with interruption test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.24.3.4.1-2.

Table 6.6.24.3.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting for inter-frequency measurement without gap with interruptions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 6.6.24.3.4.1-3.

2.	Message contents are defined in clause 6.6.24.3.4.3.

3.	There are two NR carriers and one cell on each carrier in the test. Cell 1 (Pcell) is on NR RF channel 1, and is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter neighbour cell) is on NR RF channel 2.

Table 6.6.24.3.4.1-3: General test parameters for SA event triggered reporting tests for FR1 for inter-frequency measurement without gap with interruption

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| SMTC for NR RF channel number 2 |  | Config 1,2,3 | SMTC.3 | As specified in clause A.4.1 |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.24.3.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters for PCell are given in Table 6.6.24.3.4.1-1, 6.6.24.3.4.1-3 and 6.6.24.3.5-1. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. The PCell shall continuously scheduled with data in the DL starting from T2 until the UE has sent the measurement report during T2.

NR RF channel 1 and NR RF channel 2 should be selected such that UE reports ‘no-gap’ and ‘no-gap-with-interruption’ for the target frequency on NR RF channel 2

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.24.3.5-1.

3.	SS shall transmit an RRCReconfiguration message with needForGapsConfigNR-r16 and needForInterruptionConfigNR-r18 enabled.

4.	The UE shall transmit RRCReconfigurationComplete message with NeedForGapsInterFreq-r16 set to ‘no-gap’ and interFreq-needForInterruption-r18 set to ‘no-gap-with-interruption’.

5.	SS shall transmit an RRCReconfiguration message.

6.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

7.	SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

8.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.24.3.5-1. T2 start.

9.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 2562ms and if UE transmits ACK/NACK in PCell with a missed ACK/NACK rate of no more than 1.25% from T2, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or the rate of missed ACK/NACK in PCell exceeds 1.25% from T2, then the number of failure tests is increased by one.

10.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

11.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12.	Depending on the choice in Step 0, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.24.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.24.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-3 with Condition INTER-FREQ MO, SSB.1 FR1 and SMTC.3 for configuration 6.6.24.3-1 and 6.6.24.3-2 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.2 FR1, SMTC.3 and synchronous cells for configuration 6.6.24.3-3 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ |

Table 6.6.24.3.4.3-2: RRCReconfiguration (step 3)

| Derivation Path: Table H.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| NeedForGapsConfigNR-r16 CHOICE { |  |  |  |
| setup SEQUENCE{ |  |  |  |
| requestedTargetBandFilterNR-r16 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR { | 1 entries |  |  |
| FreqBandIndicatorNR[0] | tested band |  |  |
| } |  |  |  |
| } |  |  |  |
| needForInterruptionConfigNR-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.24.3.4.3-3: RRCReconfigurationComplete (step 4)

| Derivation Path: TS 36.508 [25], Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |  |
| needForGapsInfoNR-r16 SEQUENCE { |  |  |  |  |
| NeedForGapsIntraFreqList-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 { |  |  |  |  |
| NeedForGapsIntraFreq-r16[0] SEQUENCE { |  |  |  |  |
| servCellId-r16 |  | 0 |  |  |
| gapIndicationIntra-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| NeedForGapsBandListNR-r16 SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 { |  |  |  |  |
| NeedForGapsInterFreq-r16[0] SEQUENCE { |  |  |  |  |
| bandNR-r16 |  | Tested band |  |  |
| gapIndicationInter-r16 |  | no-gap |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| needForInterruptionInfoNR-r18 SEQUENCE { |  |  |  |  |
| NeedForInterruptionIntraFreqList-r18 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NeedForInterruptionBandListNR-r18 SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | no-gap-with-interruption |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.6.24.3.5	Test requirements

Table 6.6.24.3.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.24.3.5-1: Cell specific test parameters for inter-frequency event triggered reporting without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.2 FR1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is 2.56 s as derived based on inter-frequency measurement requirements in TS 38.133 [6] clause 9.3.4.

In the test, UE shall send HARQ ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots with an interruption ratio between number of interrupted slot over the total number of slots no larger than 1.25%.

In the test UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.24.4	NR SA FR1-FR1 event-triggered reporting without gap without interruption

- [] still existed in 6.6.24.4.5

6.6.24.4.1	Test purpose

To verify the UE's ability to perform correct reporting of an event within inter-frequency cell search without gap and to verify that UE has no interruption during inter-frequency measurement without gap.

6.6.24.4.2	Test applicability

This test applies to all types of NR UE from release 18 onwards supporting NR NeedForGap capability. The UE is capable of nr-NeedForGap-Reporting-r16, nr-NeedForInterruptionReport-r18 and reports ‘no-gap’ in NeedForGapsInterFreq-r16 and ‘no-gap-no-interruption’ in interruptionIndication-r18 for the target frequency given the serving frequency.

6.6.24.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.24.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.24.4.

6.6.24.4.4	Test description

6.6.24.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.6.24.4.4.1-1.

Table 6.6.24.4.4.1-1: NR SA event triggered reporting tests for FR1 for inter-frequency measurement without gap with interruption test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.24.4.4.1-2.

Table 6.6.24.4.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting for inter-frequency measurement without gap without interruptions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 6.6.24.4.4.1-3.

2.	Message contents are defined in clause 6.6.24.4.4.3.

3.	There are two NR carriers and one cell on each carrier in the test. Cell 1 (Pcell) is on NR RF channel 1, and is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter neighbour cell) is on NR RF channel 2.

Table 6.6.24.4.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 with NeedForInterruption ‘no-gap-no-interruption’

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR Cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR Cell 2 | NR Cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.24.4.4.2	Test procedure

In this test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as neighbour cell in FR1 on NR RF channel 2. The test parameters for PCell are given in Table 6.6.24.4.4.1-1, 6.6.24.4.4.1-3 and 6.6.24.4.5-1. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. The PCell shall continuously scheduled with data in the DL starting from T2 until the UE has sent the measurement report during T2.

NR RF channel 1 and NR RF channel 2 should be selected such that UE reports ‘no-gap’ and ‘no-gap-no-interruption’ for the target frequency on NR RF channel 2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.24.4.5-1.

3.	SS shall transmit an RRCReconfiguration message with needForGapsConfigNR-r16 and needForInterruptionConfigNR-r18 enabled.

4.	The UE shall transmit RRCReconfigurationComplete message with NeedForGapsInterFreq-r16 set to ‘no-gap’ and interFreq-needForInterruption-r18 set to ‘no-gap-no-interruption’.

5.	SS shall transmit an RRCReconfiguration message.

6.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

7.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.24.4.5-1. T2 start.

8.	SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

9.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than [202]ms and if UE transmits ACK/NACK for the corresponding PDSCH scheduled in PCell in all slots from T2, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or DTX in PCell is observed bfrom T2, then the number of failure tests is increased by one.

10.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

11.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12.	Depending on the choice in Step 10, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.24.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.24.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-3 with Condition INTER-FREQ MO, SSB.5 FR1 and SMTC.5 for configuration 6.6.24.4-1 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.5 FR1 and SMTC.4 and synchronous cells for configuration 6.6.24.4-2 Table H.3.1-3 with Condition INTER-FREQ MO, SSB.6 FR1, SMTC.4 and synchronous cells for configuration 6.6.24.4-3 Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ |

Table 6.6.24.4.4.3-2: RRCReconfiguration (step 3)

| Derivation Path: Table H.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| NeedForGapsConfigNR-r16 CHOICE { |  |  |  |
| setup SEQUENCE{ |  |  |  |
| requestedTargetBandFilterNR-r16 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR { | 1 entries |  |  |
| FreqBandIndicatorNR[0] | tested band |  |  |
| } |  |  |  |
| } |  |  |  |
| needForInterruptionConfigNR-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.24.4.4.3-3: RRCReconfigurationComplete (step 4)

| Derivation Path: TS 36.508 [25], Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |  |
| needForGapsInfoNR-r16 SEQUENCE { |  |  |  |  |
| NeedForGapsIntraFreqList-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 { |  |  |  |  |
| NeedForGapsIntraFreq-r16[0] SEQUENCE { |  |  |  |  |
| servCellId-r16 |  | 0 |  |  |
| gapIndicationIntra-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| NeedForGapsBandListNR-r16 SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 { |  |  |  |  |
| NeedForGapsInterFreq-r16[0] SEQUENCE { |  |  |  |  |
| bandNR-r16 |  | Tested band |  |  |
| gapIndicationInter-r16 |  | no-gap |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| needForInterruptionInfoNR-r18 SEQUENCE { |  |  |  |  |
| NeedForInterruptionIntraFreqList-r18 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NeedForInterruptionBandListNR-r18 SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | no-gap-no-interruption |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.6.24.4.5	Test requirements

Table 6.6.24.4.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.24.4.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 with NeedForInterruption ‘no-gap-no-interruption’

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.4 |  |  | Config 1 | SMTC.2 |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | dBm/15kHz |  | -98 |  | -98 |  |
| NocNote2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 |
| EsIot |  | dB | Config 1,2,3,4,5,6 | 4 | 4 | -Infinity | 7 |
| EsNoc |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than [200] ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

During T2, UE shall send HARQ-ACK/NACK for the corresponding PDSCH scheduled in PCell in all the slots.

The rate of correct events observed during repeated tests shall be at least 90%.

The UE is not required to read the neighbour cell SSB index in this test.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.24.5	NR SA FR1 event-triggered reporting without gap without interruption

6.6.24.5.1	Test purpose

To verify the UE's ability to perform correct reporting of an event within intra-frequency cell search and to verify that UE has no interruption during intra-frequency measurement without gap.

6.6.24.5.2	Test applicability

This test applies to all types of NR UE from release 18 onwards supporting CSI-RS-based RLM, BWP operation without bandwidth restriction and NR NeedForGap capability. The UE is capable of nr-NeedForGap-Reporting-r16, nr-NeedForInterruptionReport-r18 and reports ‘no-gap’ in NeedForGapsIntraFreq-r16 and ‘no-gap-no-interruption’ in interruptionIndication-r18.

6.6.24.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.24.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.24.5.

6.6.24.5.4	Test description

6.6.24.5.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

This test shall be tested using any of the test configurations in Table 6.6.24.5.4.1-1.

Table 6.6.24.5.4.1-1: NR SA FR1 event-triggered reporting without gap without interruptions test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.24.5.4.1-2.

Table 6.6.24.5.4.1-2: Initial conditions for NR SA FR1 event-triggered reporting without gap without interruptions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.3.4.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	The general test parameter settings are set up according to Table 6.6.24.5.4.1-3.

2.	Message contents are defined in clause 6.6.24.5.4.3.

3.	Two cells are deployed in the test, which are FR1 PCell(Cell 1) and FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The power levels and settings are according to Annex C.1.2 and C.1.3 for this test.

Table 6.6.24.5.4.1-3: General test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2, 3 | Cell 1 |  |
| Neighbour cell |  | 1, 2, 3 | Cell 2 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3 | 1: Cell 1 and Cell 2 |  |
| SSB configuration |  | 1 | SSB.1 FR1 |  |
|  |  | 2 | SSB.1 FR1 |  |
|  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  | 1 | SMTC.2 |  |
|  |  | 2 | SMTC.1 |  |
|  |  | 3 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3 | -4.5 |  |
| CP length |  | 1, 2, 3 | Normal |  |
| Hysteresis | dB | 1, 2, 3 | 0 |  |
| Time To Trigger | s | 1, 2, 3 | 0 |  |
| Filter coefficient |  | 1, 2, 3 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3 |  | OFF |
| Time offset between serving and neighbour cells |  | 1 | 3 ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | 2 | 3 s | Synchronous cells |
|  |  | 3 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3 | 5 |  |
| T2 | s | 1, 2, 3 | 5 |  |

6.6.24.5.4.2	Test procedure

Two cells are deployed in the test, which are FR1 PCell (Cell 1) and a FR1 neighbour cell (Cell 2) on the same frequency as the PCell. The test parameters for PCell are given in Table 6.6.24.5.4.1-1, 6.6.24.5.4.1-3 and 6.6.24.5.5-1. In the measurement control information, a measurement object is configured for the frequency of the PCell, and it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2. The PCell shall continuously scheduled with data in the DL starting from T1 until the UE has sent the measurement report during T2.

There are two BWPs configured in Cell 1, BWP1 which contains the cell defining SSB, and BWP2 which does not contain any SSB of Cell 1. During the whole test, BWP2 is always scheduled as the active BWP for the UE.

The UE is capable of measurements without gaps with interruption and report ‘no-gap’ through NeedForGapsIntraFreq-r16 and ‘no-gap-no-interruption’ in interruptionIndication-r18 for PCell.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.24.5.5-1.

3.	SS shall transmit an RRCReconfiguration message with needForGapsConfigNR-r16 and needForInterruptionConfigNR-r18 enabled.

4.	The UE shall transmit RRCReconfigurationComplete message with NeedForGapsIntraFreq-r16 set to ‘no-gap’ and intraFreq-needForInterruption-r18 set to ‘no-gap-without-interruption’.

5.	SS shall transmit an RRCReconfiguration message.

6.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

7.	SS schedules on PCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor ACK/NACK/DTX on PCell.

8.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.24.5.5-1. T2 start.

9.	UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 802ms and if UE transmits ACK/NACK for the corresponding PDSCH scheduled in PCell in all slots from T2, then the number of successful tests is increased by one. If the UE fails to report the events within the overall delays measured requirements or DTX in PCell is observed bfrom T2, then the number of failure tests is increased by one.

10.	After the SS receive the MeasurementReport message in step 6 or when T2 expires, the SS shall:

- transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

- switch the UE off.

11.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 3) mod 1008) for next iteration of the test procedure loop.

12.	Depending on the choice in Step 10, the SS:
- if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),
OR
- if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

13.	Repeat step 2-12 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.24.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.24.5.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions (step 5) | Table H.3.1-1 Table H.3.1-2 with Condition INTRA-FREQ Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.1 FR1, SMTC pattern 1 and Synchronous cells for Config 1 and 2 Table H.3.1-3 with Condition INTRA-FREQ MO, SSB.2 FR1, SMTC pattern 1 and Synchronous cells for Config 3 Table H.3.1-4 with A3-offset = -4.5dB Table H.3.1-5 Table H.3.1-7 with Condition INTRA-FREQ |

Table 6.6.24.5.4.3-2: ServingCellConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Downlink[1] | BWP-Downlink with condition BWP-Id1 | DLBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 | Active DL BWP-ID (BWP2) | BWP-Id1 |
| defaultDownlinkBWP-Id | 0 | Initial BWP (BWP1) |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { |  |  |  |
| BWP-Uplink[1] | BWP-Uplink with condition BWP-Id1 | ULBWP.1.2 configuration |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 | Active UL BWP-ID (BWP2) | BWP-Id1 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BWP-Id1 | Active BWP (BWP2) |

Table 6.6.24.5.4.3-3: RRCReconfiguration (step 3)

| Derivation Path: Table H.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| measConfig | Not present |  |  |
| NeedForGapsConfigNR-r16 CHOICE { |  |  |  |
| setup SEQUENCE{ |  |  |  |
| requestedTargetBandFilterNR-r16 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR { | 1 entries |  |  |
| FreqBandIndicatorNR[0] | tested band |  |  |
| } |  |  |  |
| } |  |  |  |
| needForInterruptionConfigNR-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.24.5.4.3-4: RRCReconfigurationComplete (step 4)

| Derivation Path: TS 36.508 [25], Table 4.6.1-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfigurationComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | Not checked |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfigurationComplete SEQUENCE { |  |  |  |  |
| needForGapsInfoNR-r16 SEQUENCE { |  |  |  |  |
| NeedForGapsIntraFreqList-r16 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForGapsIntraFreq-r16 { |  |  |  |  |
| NeedForGapsIntraFreq-r16[0] SEQUENCE { |  |  |  |  |
| servCellId-r16 |  | 0 |  |  |
| gapIndicationIntra-r16 |  | no-gap |  |  |
| } |  |  |  |  |
| NeedForGapsBandListNR-r16 SEQUENCE (SIZE (1..maxBands)) OF NeedForGapsNR-r16 { |  |  |  |  |
| NeedForGapsInterFreq-r16[0] SEQUENCE { |  |  |  |  |
| bandNR-r16 |  | Tested band |  |  |
| gapIndicationInter-r16 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| needForInterruptionInfoNR-r18 SEQUENCE { |  |  |  |  |
| NeedForInterruptionIntraFreqList-r18 SEQUENCE (SIZE (1..maxNrofServingCells)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | no-gap-without-interruption |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| NeedForInterruptionBandListNR-r18 SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18 { |  |  |  |  |
| NeedForInterruptionNR-r18[0] SEQUENCE { |  |  |  |  |
| interruptionIndication-r18 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

6.6.24.5.5	Test requirements

Table 6.6.24.5.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.24.5.5-1: NR Cell specific test parameters for SA intra-frequency event triggered reporting without gap for FR1

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | TN/A |  | TN/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | N/A |  |
|  |  | 2 | CR.1.1 TDD |  | N/A |  |
|  |  | 3 | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | N/A |  |
|  |  | 2 | CCR.1.1 TDD |  | N/A |  |
|  |  | 3 | CCR.2.1 TDD |  | N/A |  |
| OCNG Patterns |  | 1, 2, 3 | OP.1 |  | OP.1 |  |
| TRS Configuration |  | 1 | TRS.1.1 FDD |  | N/A |  |
|  |  | 2 | TRS.1.1 TDD |  | N/A |  |
|  |  | 3 | TRS.1.2 TDD |  | N/A |  |
| IInitial BWP configuration |  | 1, 2, 3 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2, 3 | DLBWP.1.2 |  | DLBWP.1.2 |  |
| Active UL BWP configuration |  | 1, 2, 3 | ULBWP.1.2 |  | ULBWP.1.2 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Noc Note 2 | dBm/15 kHz | 1 | -98 |  |  |  |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsIot | dB | 1 | 4 | -1.46 | -Infinity | -1.46 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| EsNoc | dB | 1 | 4 | 4 | -Infinity | 4 |
|  |  | 2 |  |  |  |  |
|  |  | 3 |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1 | -94 | -94 | -Infinity | -94 |
|  |  | 2 | -94 | -94 | -Infinity | -94 |
|  |  | 3 | -91 | -91 | -Infinity | -91 |
| Io | dBm/9.36 MHz | 1 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/9.36 MHz | 2 | -64.60 | -62.25 | -64.60 | -62.25 |
|  | dBm/38.16 MHz | 3 | -58.50 | -56.16 | -58.50 | -56.16 |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

UE is not allowed to cause interruption during intra-frequency measurement without gap.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.25	SA NR - E-UTRAN event-triggered without measurement gaps

#### 6.6.25.0	Minimum conformance requirements

6.6.25.0.1	Minimum conformance requirements for NR – E-UTRAN FDD measurements when no DRX is used

The requirements are applicable for NR−E-UTRAN FDD RSRP, RSRQ, and RS-SINR measurements when no DRX is used.

The UE shall be able to identify a new detectable FDD cell within TIdentify, E-UTRAN FDD according to the following expression:

TIdentify,  E-UTRAN FDD=TBasicIdentify∙480TInter1∙CSSFinterRAT∙KP    ms

where:

-	TBasicIdentify = [480 ms]

-	TInter1 is defined in TS 38.133 [6] clause 9.4.8.1

-	CSSFinterRAT = [CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1]

-	Kp is the scaling factor due to overlapping between EMW and SMTC, measurement gap, or SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement. Kp =1 if the measurement is performed outside EMW or if EMW is fully non-overlapped with SMTC, SSB/CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement and measurement gap. Otherwise, Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(EMW period, SMTC period, MGRP, TSSB, TCSI-RS), where SMTC period follows smtc1 if high layer in TS 38.331 [13] signaling of smtc2 is not present on the same carrier frequency. Otherwise, SMTC period follows smtc2, TSSB/TCSI-RS is the periodicity of SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement on the same carrier frequency, and starting from the beginning of any SMTC occasion.

-	Ntotal is the total number of EMW occasions within the window W, including those overlapped with SMTC, measurement gap, or SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement, and

-	Navailable is the number of EMW occasions that are not overlapped with any SMTC, measurement gap, or SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement within the window W, after accounting for EMW collisions by applying the EMW collision rule in TS 38.133 [6] section 9.4.8.2.

-	No requirement applies for the measurement when Navailable = 0.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure, E-UTRAN FDD defined in Table 6.6.25.0.1-1.

Table 6.6.25.0.1-1: Measurement period and measurement bandwidth configuration

| Configuration | Physical Layer Measurement period: Tmeasure, E-UTRAN FDD [ms] | Measurement bandwidth [RB] |
| --- | --- | --- |
| 0 | 480 x CSSFinterRAT x Kp | 6 |
| 1 (Note 1) | 240 x CSSFinterRAT x Kp | 50 |
| Note 1: This configuration is optional. |  |  |

The UE physical layer shall be capable of reporting RSRP, RSRQ, and RS-SINR measurements to higher layers with measurement period Tmeasure, E-UTRAN FDD given by Table 6.6.25.0.1-1.

The normative reference for this requirement is TS 38.133 [6] clauses 9.4.8.1, 9.4.8.2 and 9.4.8.3.

6.6.25.0.2	Minimum conformance requirements for NR – E-UTRAN TDD measurements when no DRX is used

The requirements are applicable for NR−E-UTRAN TDD RSRP, RSRQ, and RS-SINR measurements when no DRX is used.

The UE shall be able to identify a new detectable TDD cell within TIdentify, E-UTRAN TDD according to the following expression:

-	When configuration 0 or configuration 1 in Table 6.6.25.0.2-1 is applied,

TIdentify,  E-UTRAN TDD=TBasicIdentify∙480TInter1∙CSSFinterRAT∙KP    ms

-	When configuration 2 or configuration 3 in Table 6.6.25.0.2-1 is applied,

TIdentify,  E-UTRAN TDD=TBasicIdentify∙480TInter1∙CSSFinterRAT∙KP+240∙CSSFinterRAT∙KP    ms

where:

-	TBasicIdentify = [480 ms]

-	TInter1 is defined in TS 38.133 [6] clause 9.4.8.1

-	CSSFinterRAT = [CSSFoutside_gap,i in TS 38.133 [6] clause 9.1.5.1]

-	Kp is the scaling factor due to overlapping between EMW and SMTC, measurement gap, or SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement. Kp =1 if the measurement is performed outside EMW or if EMW is fully non-overlapped with SMTC, SSB/CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement and measurement gap. Otherwise, Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(EMW period, SMTC period,  MGRP, TSSB, TCSI-RS), where SMTC period follows smtc1 if high layer in TS 38.331 [13] signaling of smtc2 is not present on the same carrier frequency. Otherwise, SMTC period follows smtc2, TSSB/TCSI-RS is the periodicity of SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement on the same carrier frequency, and starting from the beginning of any SMTC occasion

-	Ntotal is the total number of EMW occasions within the window, including those overlapped with SMTC, measurement gap, or SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement, and

-	Navailable is the number of EMW occasions that are not overlapped with any SMTC, measurement gap, or SSB/CSI-RS configured for RLM/BFD/CBD/L1-RSRP measurement within the window W, after accounting for EMW collisions by applying the EMW collision rule in TS 38.133 [6] section 9.4.8.2.

-	No requirement applies for the measurement when Navailable = 0.

Identification of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of TMeasure, E-UTRAN TDD defined in Table 6.6.25.0.2-1.

Table 6.6.25.0.2-1: TMeasure, E-UTRAN TDD for different configurations

| Configuration | Measurement bandwidth (RB) | Number of UL/DL sub-frames per half frame (5 ms) |  | DwPTS |  | TMeasure, E-UTRAN TDD (ms) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | DL | UL | Normal CP | Extended CP |  |
| 0 | 6 | 2 | 2 | 19760 Ts | 20480 Ts | [480 x CSSFinterRAT x Kp] |
| 1 (Note 1) | 50 | 2 | 2 | 19760 Ts | 20480 Ts | [240 x CSSFinterRAT x Kp] |
| 2 | 6 | 1 | 3 | 19760 Ts | 20480 Ts | [720 x CSSFinterRAT x Kp] |
| 3 (Note 1) | 50 | 1 | 3 | 19760 Ts | 20480 Ts | [480 x CSSFinterRAT x Kp] |
| Note 1: This configuration is optional. Note 2: Void |  |  |  |  |  |  |

The UE physical layer shall be capable of reporting RSRP, RSRQ, and RS-SINR measurements to higher layers with measurement period Tmeasure, E-UTRAN TDD given by Table 6.6.25.0.2-1.

The normative reference for this requirement is TS 38.133 [6] clauses 9.4.8.1, 9.4.8.2 and 9.4.8.4.

#### 6.6.25.1	NR SA FR1 - E-UTRAN event-triggered reporting without gap without interruption

Editor's Note: This test case is incomplete in following aspects:

-  Test applicability is FFS.

6.6.25.1.1	Test purpose

The purpose of this test is to verify that if UE reports “nogap-noncsg” via NeedForGapNCSG-InfoEUTRA-r17, the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1.

6.6.25.1.2	Test applicability

This test applies to all types of NR UEs supporting “nogap-noncsg” via NeedForGapNCSG-InfoEUTRA-r17 capability from release 18 onwards.

6.6.25.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.25.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.25.1.

6.6.25.1.4	Test description

6.6.25.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.25.1.4.1-1.

Table 6.6.25.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.25.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.25.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.25.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.25.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.25.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.25.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 6.6.25.1.4.1-2.

Table 6.6.25.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.25.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.25.1.4.1-3.

2. Message contents are defined in clause 6.6.25.1.4.3.

3. There are two cells: Cell 1 and Cell 2. Cell 1 (NR PCell) is used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter-RAT E-UTRAN neighbour cell) is configured according to TS 36.521-3 [26] Annex C.0 and C.1.

Table 6.6.25.1.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 1 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in Tables 6.6.25.1.5-1 and 6.6.25.1.5-2. |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Effective Measurement Window (EMW) Id |  | 0 | As specified in TS 38.133 [6] Table 9.4.8.2-1 |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for RSRP measurement on Cell 2 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in Table 6.6.25.1.5-1 |  |  |  |

6.6.25.1.4.2	Test procedure

In the measurement control information from the PCell, it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter-RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.25.1.5-1.

3.	SS shall transmit RRCReconfiguration message to configure Event B2 triggered measurement report.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	T1 starts. SS continuously schedules the UE to receive PDSCH on Pcell, and monitors corresponding ACK/NACK feedbacks sent by the UE.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.25.1.5-1. T2 starts.

7.	UE shall transmit MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 3842ms, and ACK/NACKs are observed by the SS for the corresponding PDSCHs scheduled in PCell in all slots excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.4.5, the number of successful tests is increased by one. If the UE fails to report the events within the overall measured delays requirements, or DTX in PCell is observed by the SS excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.3.4.5, the number of failure tests is increased by one.

8.	After the SS receive the MeasurementReport message in step 7 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

9.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

10.	Depending on the choice in Step 8, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),

OR

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.25.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.25.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-RAT Table H.3.1-4A Table H.3.1-5 with condition INTER-RAT Table H.3.1-7 with condition INTER-RAT |
| Specific message contents exceptions for Cell 1 | Table H.3.1-3 with condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3A |

6.6.25.1.5	Test requirement

Table 6.6.25.1.5-1 and Table 6.6.25.1.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.25.1.5-1: FR1 PCell specific test parameters

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 kHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 kHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote 1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote 2 |  |  | dBm/15 kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote 2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| Ês/IotNote 3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| SS-RSRPNote 3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.54 |
| SSB_RPNote 3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.54 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.42 | -73.74 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.32 | -67.64 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.25.1.5-2: E-UTRAN neighbour cell specific test parameters

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote 1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote 1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote 2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote 2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote 2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 3 |  |  |  |  |
| OCNG_RBNote 3 |  |  |  |  |
| NocNote 4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| Ês/IotNote 5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| RSRPNote 5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| SCH_RPNote 5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| IoNote 5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21 | -54.60 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria are not fulfilled.

During the T1 and T2, UE shall be able to report ACK/NACK for all slots with PDCCH/PDSCH on PCell excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.4.5.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.25.2	NR SA FR1 - E-UTRAN event-triggered reporting without interruption outside gap

Editor's Note: This test case is incomplete in following aspects:

-  Test applicability is FFS.

-  Test configurations are missing.

6.6.25.2.1	Test purpose

The purpose of this test is to verify that the UE makes correct event-triggered reporting of inter-RAT E-UTRAN measurements without gap as there are vacant RF chains available for UE measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in TS 38.133 [6] clause 9.4.8, and also verify the scheduling availability during inter-RAT measurement without gap in TS 38.133 [6] clauses 9.4.8.3.5 and 9.4.8.4.5.

6.6.25.2.2	Test applicability

This test applies to all types of NR UEs supporting “nogap-noncsg” via “NeedForGapNCSG-InfoEUTRA-r17” and “eutra-NoGapMeasurementOutsideBWP-r18” capability from release 18 onwards.

6.6.25.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.25.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.25.2.

6.6.25.2.4	Test description

6.6.25.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in table 6.6.25.2.4.1-1.

Table 6.6.25.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.25.2-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.25.2-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.25.2-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.25.2-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.25.2-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.25.2-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in table 6.6.25.2.4.1-2.

Table 6.6.25.2.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.25.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to Table 6.6.25.2.4.1-3.

2. Message contents are defined in clause 6.6.25.2.4.3.

3. There are two cells: Cell 1 and Cell 2. Cell 1 (NR PCell) is used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter-RAT E-UTRAN neighbour cell) is configured according to TS 36.521-3 [26] Annex C.0 and C.1.

Table 6.6.25.2.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables 6.6.25.2.5-1 and 6.6.25.2.5-2 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| Gap Pattern Id |  | 0 | As specified in TS 38.133 [6] clause table 9.1.2-1 Per-UE gap pattern |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for RSRP measurement on Cell 2 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in table 6.6.25.2.5-1 |  |  |  |

6.6.25.2.4.2	Test procedure

In the measurement control information from the PCell, it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in table 6.6.25.2.5-1.

3.	SS shall transmit RRCReconfiguration message to configure Event B2 triggered measurement report.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	T1 starts. SS continuously schedules the UE to receive PDSCH on PCell, and monitors corresponding ACK/NACK feedbacks sent by the UE.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in table 6.6.25.2.5-1. T2 starts.

7.	UE shall transmit MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 3842ms, and ACK/NACKs are observed by the SS for the corresponding PDSCHs scheduled in PCell in all slots excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.4.5, the number of successful tests is increased by one. If the UE fails to report the events within the overall measured delays requirements, or DTX in PCell is observed by the SS excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.3.4.5, the number of failure tests is increased by one.

8.	After the SS receive the MeasurementReport message in step 7 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

9.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

10.	Depending on the choice in Step 8, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),

OR

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.25.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.25.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-RAT, GAP NEEDED Table H.3.1-4A Table H.3.1-5 with condition INTER-RAT Table H.3.1.6 with condition gapUE Table H.3.1-7 with condition INTER-RAT |
| Specific message contents exceptions for Cell 1 | Table H.3.1-3 with condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3A |

6.6.25.2.5	Test requirement

Table 6.6.25.2.5-1 and table 6.6.25.2.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.25.2.5-1: FR1 PCell specific test parameters

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 kHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 kHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote 1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote 2 |  |  | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote 2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| Ês/IotNote 3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| SS-RSRPNote 3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.54 |
| SSB_RPNote 3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.54 |
| IoNote3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.42 | -73.74 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.32 | -67.64 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.25.2.5-2: E-UTRAN neighbour cell specific test parameters

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote 1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote 1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote 2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote 2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote 2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 3 |  |  |  |  |
| OCNG_RBNote 3 |  |  |  |  |
| NocNote 4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| Ês/IotNote 5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| RSRPNote 5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| SCH_RPNote 5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| IoNote 5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21 | -54.60 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria are not fulfilled.

During the T1 and T2, UE shall be able to report ACK/NACK for all slots with PDCCH/PDSCH on PCell excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.4.5.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

#### 6.6.25.3	NR SA FR1 - E-UTRAN event-triggered reporting without gap when CRS contained within active DL BWP

Editor's Note: This test case is incomplete in following aspects:

-  Test applicability is FFS.

6.6.25.3.1	Test purpose

The purpose of this test is to verify that the UE which supports inter-RAT E-UTRAN measurement without gap when E-UTRAN CRS is contained within UE’s active DL BWP makes correct event-triggered reporting of inter-RAT E-UTRAN measurements when operating in standalone (SA) operation with PCell in FR1. This test shall partly verify the cell search and measurement requirements in TS 38.133 [6] clause 9.4.8, and also verify the scheduling availability during inter-RAT E-UTRAN measurement without gap in TS 38.133 [6] clauses 9.4.8.3.5 and 9.4.8.4.5.

6.6.25.3.2	Test applicability

This test applies to all types of NR UEs supporting “eutra-NoGapMeasurementInsideBWP-r18” and “eutra-MeasEMW-r18” capability from release 18 onwards.

6.6.25.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.25.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.25.3.

6.6.25.3.4	Test description

6.6.25.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in table 6.6.25.3.4.1-1.

Table 6.6.25.3.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.25.3-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 6.6.25.3-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.25.3-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 6.6.25.3-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 6.6.25.3-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 6.6.25.3-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in table 6.6.25.3.4.1-2.

Table 6.6.25.3.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from table 6.6.25.3.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1. The general test parameter settings are set up according to table 6.6.25.3.4.1-3.

2. Message contents are defined in clause 6.6.25.3.4.3.

3. There are two cells: Cell 1 and Cell 2. Cell 1 (NR PCell) is used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter-RAT E-UTRAN neighbour cell) is configured according to TS 36.521-3 [26] Annex C.0 and C.1. The CRS of Cell 2 is completely within UE’s active BWP BW.

Table 6.6.25.3.4.1-3: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | 1 NR carrier frequency is used in the test |
| LTE RF Channel Number |  | 2 | 1 LTE carrier frequency is used in the test |
| Channel Bandwidth | MHz | As specified in tables 6.6.25.3.5-1 and 6.6.25.3.5-2 |  |
| Active cell |  | Cell 1 | Cell 1 is on RF channel number 1 |
| Neighbour cell |  | Cell 2 | Cell 2 is on RF channel number 2 |
| EMW periodicity | ms | 40 |  |
| EMW duration | ms | 5 |  |
| NR measurement quantity |  | SS-RSRP | Measurement quantity for Cell 1 |
| Inter-RAT E-UTRAN measurement quantity |  | RSRP | Measurement quantity for Cell 2 |
| b2-Threshold1 | dBm | Note 1 | SS-RSRP threshold for SS-RSRP measurement on Cell 1 for event B2 |
| b2-Threshold2EUTRA | dBm | -95 | E-UTRAN RSRP threshold for RSRP measurement on Cell 2 for event B2 |
| Hysteresis | dB | 0 |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX |  | OFF | OFF |
| T1 | s | 5 |  |
| T2 | s | 5 |  |
| Note 1: Values are defined in table 6.6.25.3.5-1 |  |  |  |

6.6.25.3.4.2	Test procedure

In the measurement control information from the PCell, it is indictated to the UE that event-triggered reporting with Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2) is to be used. Each test consists of two consecutive time periods, with durations T1 and T2, respectively. Prior to the start of time duration T1, the UE shall be fully synchronized to Cell 1. During T1, the UE shall not have any information on Cell 2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in table 6.6.25.3.5-1.

3.	SS shall transmit RRCReconfiguration message to configure Event B2 triggered measurement report.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	T1 starts. SS continuously schedules the UE to receive PDSCH on PCell, and monitors corresponding ACK/NACK feedbacks sent by the UE.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in table 6.6.25.3.5-1. T2 starts.

7.	UE shall transmit MeasurementReport message triggered by Event B2. If the overall delays measured from the beginning of time period T2 is less than 3842ms, and ACK/NACKs are observed by the SS for the corresponding PDSCHs scheduled in PCell in all slots excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.4.5, the number of successful tests is increased by one. If the UE fails to report the events within the overall measured delays requirements, or DTX in PCell is observed by the SS excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.3.4.5, the number of failure tests is increased by one.

8.	After the SS receive the MeasurementReport message in step 7 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

9.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

10.	Depending on the choice in Step 8, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5),

OR

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.25.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.6.25.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-RAT and EMW_Setup Table H.3.1-4A Table H.3.1-5 with condition INTER-RAT Table H.3.1-7 with condition INTER-RAT Table H.3.1-14 with condition MS40 |
| Specific message contents exceptions for Cell 1 | Table H.3.1-3 with condition INTRA-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3A |

6.6.25.3.5	Test requirement

Table 6.6.25.3.5-1 and table 6.6.25.3.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.25.3.5-1: FR1 PCell specific test parameters

| Parameter |  |  | Unit | Configuration | Cell 1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |
| RF channel number |  |  |  | 1, 2, 3, 4, 5, 6 | 1 |  |
| Duplex mode |  |  |  | 1, 4 | FDD |  |
|  |  |  |  | 2, 3, 5, 6 | TDD |  |
| TDD Configuration |  | SCS=15 kHz |  | 2, 5 | TDDConf.1.1 |  |
|  |  | SCS=30 kHz |  | 3, 6 | TDDConf.2.1 |  |
| BWchannel |  |  | MHz | 1, 4 | 10: NPRB,c = 52 (FDD) |  |
|  |  |  |  | 2, 5 | 10: NPRB,c = 52 (TDD) |  |
|  |  |  |  | 3, 6 | 40: NPRB,c = 106 (TDD) |  |
| PDSCH reference measurement channel |  |  |  | 1, 4 | SR.1.1 FDD |  |
|  |  |  |  | 2, 5 | SR.1.1 TDD |  |
|  |  |  |  | 3, 6 | SR.2.1 TDD |  |
| RMSI CORSET reference channel |  |  |  | 1, 4 | CR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CR.2.1 TDD |  |
| Dedicated CORSET reference channel |  |  |  | 1, 4 | CCR.1.1 FDD |  |
|  |  |  |  | 2, 5 | CCR.1.1 TDD |  |
|  |  |  |  | 3, 6 | CCR.2.1 TDD |  |
| BWP configurations | Initial DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.0.1 |  |
|  | Dedicated DL BWP |  |  | 1, 2, 3, 4, 5, 6 | DLBWP.1.1 |  |
|  | Initial UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.0.1 |  |
|  | Dedicated UL BWP |  |  | 1, 2, 3, 4, 5, 6 | ULBWP.1.1 |  |
| OCNG patternNote 1 |  |  |  | 1, 2, 3, 4, 5, 6 | OP.1 |  |
| SMTC configuration |  |  |  | 1, 2, 3, 4, 5, 6 | SMTC.1 |  |
| SSB configuration |  |  |  | 1, 2, 4, 5 | SSB.1 FR1 |  |
|  |  |  |  | 3, 6 | SSB.2 FR1 |  |
| CSI-RS for tracking |  |  |  | 1, 4 | TRS.1.1 FDD |  |
|  |  |  |  | 2, 5 | TRS.1.1 TDD |  |
|  |  |  |  | 3, 6 | TRS.1.2 TDD |  |
| b2-Threshold1 |  |  | dBm | 1, 2, 4, 5 | -96 |  |
|  |  |  |  | 3, 6 | -93 |  |
| EPRE ratio of PSS to SSS |  |  | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |
| NocNote 2 |  |  | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| NocNote 2 |  |  | dBm/SCS | 1, 2, 4, 5 | -104 |  |
|  |  |  |  | 3, 6 | -101 |  |
| Ês/Noc |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| Ês/IotNote 3 |  |  | dB | 1, 2, 3, 4, 5, 6 | 17.55 | -1.55 |
| SS-RSRPNote 3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.54 |
| SSB_RPNote 3 |  |  | dBm/SCS | 1, 2, 4, 5 | -86.45 | -105.55 |
|  |  |  |  | 3, 6 | -83.44 | -102.54 |
| IoNote 3 |  |  | dBm/9.36 MHz | 1, 2, 4, 5 | -58.42 | -73.74 |
|  |  |  | dBm/38.16 MHz | 3, 6 | -52.32 | -67.64 |
| Propagation condition |  |  |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.25.3.5-2: E-UTRAN neighbour cell specific test parameters

| Parameter | Unit | Configuration | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| RF channel number |  | 1, 2, 3, 4, 5, 6 | 2 |  |
| Duplex mode |  | 1, 2, 3 | FDD |  |
|  |  | 4, 5, 6 | TDD |  |
| TDD special subframe configurationNote 1 |  | 4, 5, 6 | 6 |  |
| TDD uplink-downlink configurationNote 1 |  | 4, 5, 6 | 1 |  |
| BWchannel | MHz | 1, 2, 3, 4, 5, 6 | 5 MHz: NPRB,c = 25 10 MHz: NPRB,c = 50 20 MHz: NPRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote 2 |  | 1, 2, 3 | 5 MHz: R.7 FDD 10 MHz: R.3 FDD 20 MHz: R.6 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.4 TDD 10 MHz: R.0 TDD 20 MHz: R.3 TDD |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote 2 |  | 1, 2, 3 | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote 2 |  | 1, 2, 3 | 5 MHz: OP.20 FDD 10 MHz: OP.10 FDD 20 MHz: OP.17 FDD |  |
|  |  | 4, 5, 6 | 5 MHz: OP.9 TDD 10 MHz: OP.1 TDD 20 MHz: OP.7 TDD |  |
| PBCH_RA | dB | 1, 2, 3, 4, 5, 6 | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote 3 |  |  |  |  |
| OCNG_RBNote 3 |  |  |  |  |
| NocNote 4 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -104 |  |
| Ês/Noc | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| Ês/IotNote 5 | dB | 1, 2, 3, 4, 5, 6 | -Infinity | 18.55 |
| RSRPNote 5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| SCH_RPNote 5 | dBm/15kHz | 1, 2, 3, 4, 5, 6 | -Infinity | -85.45 |
| IoNote 5 | dBm/9MHz | 1, 2, 3, 4, 5, 6 | -73.21 | -54.60 |
| Propagation Condition |  | 1, 2, 3, 4, 5, 6 | AWGN |  |
| Antenna Configuration and Correlation Matrix |  | 1, 2, 3, 4, 5, 6 | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |

The UE shall send one Event B2 triggered measurement report for Cell 2 to the PCell, with a measurement reporting delay less than 3.84s from the start of period T2. The measurement reporting delay is defined as the time from the beginning of time period T2 to the moment when the UE sends the measurement report on PUSCH.

The UE shall not send event-triggered measurement reports as long as the reporting criteria are not fulfilled.

During T2, UE shall be able to report ACK/NACK for all slots with PDCCH/PDSCH on PCell excluding those symbols as defined in TS 38.133 [6] clause 9.4.8.3.5 or clause 9.4.8.4.5.

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.26	LTM Intra-frequency L1-RSRP measurement

#### 6.6.26.0	Minimum conformance requirements

6.6.26.0.1	Minimum conformance requirements for SSB-based intra-frequency L1-RSRP measurement

If a neighbour cell is known according to TS 38.133 [6] clause 9.14.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra, where TSSB_time_index_intra is the time period used to acquire the index of the SSB being measured given in TS 38.133 [6] clause 9.2.5.1.

The value of TL1-RSRP_Measurement_Period_SSB_intra is defined for FR1 in Table 6.6.26.0.1-1 for UE incapable of [capability of measurement with RTD>CP] and in Table 6.6.26.0.1-2 for UE capable of [capability of measurement with RTD>CP], where

-	M = 1

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

For a window W of duration max (TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	TL1 is periodicity of the target SSB.

If the high layer in TS 38.331 [13] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

Longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

Longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 6.6.26.0.1-1: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbour cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: The requirements apply when the max RTD between the serving cell and any candidate cell on which UE is required to perform L1-RSRP measurement on the same frequency layer is not larger than CP length of the cell on the frequency layer. |  |

Table 6.6.26.0.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbour cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. Note 2: K = 1.5. Note 3: UE shall at least measure neighbour cells which are activated for TCI state within UE capability and additional cells to measure is up to UE implementation. |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.14.3.1, 9.14.5, 9.14.6 and 9.14.7.

#### 6.6.26.1	NR SA FR1 Intra-frequency SSB based L1-RSRP measurement

6.6.26.1.1	Test purpose

To verify that the UE makes correct reporting of intra-frequency SSB based L1-RSRP measurement on neighbour cell in FR1 within L1-RSRP measurement requirements specified in TS 38.133 [6] clause 9.14.

6.6.26.1.2	Test applicability

This test applies to all types of NR UEs supporting intra-frequency L1 measurement and reports for LTM from release 18 and forward. Test 2 is applicable only to UEs supporting multiCellL1-measRTD-greaterThan-CP-r18, otherwise Test 1 is applicable.

6.6.26.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.26.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.26.1.

6.6.26.1.4	Test description

6.6.26.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.26.1.4.1-1.

Table 6.6.26.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.26.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.26.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.26.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Test environment parameters are given in Table 6.6.26.1.4.1-2.

Table 6.6.26.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.26.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | -Without LTE link -For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.26.1.4.1-3.

2.	Message contents are defined in clause 6.6.26.1.4.3.

3.	There are one NR carrier and two cells specified in the tests. Cell 1 (Pcell) is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (intra neighbour cell) is on the same frequency as the Cell 1.

Table 6.6.26.1.4.1-3: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbouring cell |  |  | Cell 2 |  | Cell 2 is the candidate cell |
| A3-Offset |  | dB | -6 |  |  |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | ms | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| DRX |  |  | OFF |  | DRX is not used |
| Time offset between cells |  |  | 2s | 20s | The timing of Cell 2 is later than the timing of Cell 1 |
| deriveSSB-IndexFromCell |  |  | Enabled |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 20 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |
| T2 |  | s | 0.5 |  |  |

6.6.26.1.4.2	Test procedure

There are two tests in the test case, test 1 and test 2:

-	In test 1, time offset between cells is within CP length.

-	In test 2, time offset between cells is larger than CP length.

If a UE does not support multiCellL1-measRTD-greaterThan-CP-r18, it is only required to pass test 1. Otherwise, it is only required to pass test 2.

The test consists of two successive time periods, with time durations of T1 and T2 respectively. SSB_RP of Cell 2 in T1 and T2 are different. No gap patterns are configured in the test case.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.26.1.5-1.

3.	SS shall transmit an RRCReconfiguration message to configure Event A3 triggered measurement report.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	The UE shall transmit a MeasurementReport message triggered by Event A3 including Cell 2 measurement report with SSB index.

6.	SS shall transmit an RRCReconfiguration message to configure LTM-Candidate-r18 for Cell 2. UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports for candidate Cell 2 in PUCCH format 2.

7.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

8.	The UE shall transmit L1-RSRP on PUCCH format 2 in Cell 1 with a periodicity of 20 slots.

9.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.26.1.5-1. T2 starts at the beginning of a frame with an odd SFN.

10.	The UE shall continue to send valid L1-RSRP reports. The SS shall check the following requirements:

-R1:	the UE shall start to transmit valid reports no later than 40 ms for configuration 1, 2 and no later than 30 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement (Table 6.6.26.1.5-2 for test configurations 1, 2 and Table 6.6.26.1.5-3 for test configuration 3). For candidate Cell 2, the reported L1-RSRP value in T2 shall be larger than the value reported in T1. If the first valid report is received earlier than the specified time, and includes a larger L1-RSRP value of Cell 2, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

-R2:	the UE shall transmit reports every 20 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

-R3:	the L1-RSRP value of Cell 2 reported by the UE is compared to the expected L1-RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.26.1.5-2 for test configurations 1, 2 and in Table 6.6.26.1.5-3 for test configuration 3, or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

11.	The SS waits until T2 expires.

12.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

14.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),

or:

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

15.	Repeat steps 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.26.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.6.26.1.4.3-1: Common Exception messages (step 3)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 with condition SSB Index Table H.3.1-7 with conditions INTRA-FREQ, SSB Index |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with conditions INTRA-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.6.26.1.4.3-2: ReportConfigNR (a3-Offset) (step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -12 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 1 |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.26.1.4.3-3: Common Exception messages (step 6)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.12-1 with condition L1-RSRP Table H.3.12-2 with condition L1-RSRP Table H.3.12-3 Table H.3.12-4 Table H.3.12-6 Table H.3.12-8 |
| Specific message contents exceptiosn for Test Configuration 1 | Table H.3.12-9 with conditions FR1, SSB#0, FDD |
| Specific message contents exceptiosn for Test Configuration 2 | Table H.3.12-9 with conditions FR1, SSB#0, TDD, SCS15 |
| Specific message contents exceptiosn for Test Configuration 3 | Table H.3.12-9 with conditions FR1, SSB#0, TDD, SCS30 |

Table 6.6.26.1.4.3-4: LTM-CSI-ReportConfig-r18 (step 6)

| Derivation path: Table H.3.12-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-ReportConfig-r18 ::= SEQUENCE { |  |  |  |
| ltm-ReportConfigType-r18 CHOICE { |  |  |  |
| periodic-r18 SEQUENCE { |  |  |  |
| reportSlotConfig-r18 |  |  |  |
| slots20 | 2 | Periodicity 20 slots and offset 2 | SCS15 |
| slots20 | 7 | Periodicity 20 slots and offset 7 | SCS30 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.26.1.5	Test requirement

Table 6.6.26.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.26.1.5-1: Cell specific test parameters

| Parameter |  |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  |  | 1 |  |  |  |
| SSB GSCN |  |  |  |  | freq1 |  |  |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| PDSCH Reference |  |  | Config 1 |  | SR.1.1 FDD |  | N/A |  |
| measurement channel |  |  | Config 2 |  | SR.1.1 TDD |  | N/A |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  | N/A |  |
| CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD |  | N/A |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  | N/A |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  | N/A |  |
| CP length |  |  |  |  | Normal |  |  |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  |  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  |  | Config 3 |  | 30 |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| NocNote2 |  | Config 1,2 |  | dBm/15kHz | -101 | -101 | -101 | -101 |
|  |  | Config 3 |  |  | -101 | -102.2 | -101 | -102.2 |
| NocNote2 |  | Config 1,2 |  | dBm/SCS | -101 | -101 | -101 | -101 |
|  |  | Config 3 |  |  | -98 | -99.2 | -98 | -99.2 |
| EsIot |  |  |  | dB | -1.76 | -0.19 | -1.76 | -0.19 |
| EsNoc |  |  |  | dB | 3 | 13.5 | 3 | 13.5 |
| SSB_RP | Config 1,2 |  |  | dBm/SCS | -98 | -87.5 | -98 | -87.5 |
|  | Config 3 |  |  | dBm/SCS | -94.99 | -85.69 | -94.99 | -85.69 |
| IoNote3 | Config 1,2 |  |  | dBm/ 9.36MHz | -66.07 | -56.44 | -66.07 | -56.44 |
|  | Config 3 |  |  | dBm/ 38.16MHz | -59.96 | -51.54 | -59.96 | -51.54 |
| Propagation condition |  |  |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 20 slots. The UE shall start to report a larger L1-RSRP value of Cell 2 in no later than 20 ms plus 20 slots from the beginning of time period T2. During T2, each L1-RSRP measurement report of Cell 2 shall meet the corresponding absolute accuracy requirement in Table 6.6.26.1.5-2 for test configuration 1, 2 and the corresponding absolute accuracy requirements in Table 6.6.26.1.5-3 for test configuration 3.

Table 6.6.26.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (Cell 2) | 59 |
| Highest reported value (Cell 2) | 79 |

Table 6.6.26.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (Cell 2) | 61 |
| Highest reported value (Cell 2) | 81 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.27	LTM Inter-frequency L1-RSRP measurement with measurement gap

#### 6.6.27.0	Minimum conformance requirements

6.6.27.0.1	Minimum conformance requirements for SSB-based inter-frequency L1-RSRP measurement with gap

If an inter-frequency neighbour cell is known according to TS 38.133 [6] clause 9.15.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_Inter if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter+TSSB_time_index_inter, where TSSB_time_index_inter is the time period used to acquire the index of the SSB being measured given in TS 38.133 [6] clause 9.3.4.

The value of TL1-RSRP_Measurement_Period_SSB_inter is defined in Table 6.6.27.0.1-1 for FR1, where

-	M = 2

-	CSSFinter: it is a carrier specific scaling factor and is determined according to CSSFwithin_gap in TS 38.133 [6] clause 9.1.5.4 for L1-RSRP measurement conducted within measurement gaps.

Table 6.6.27.0.1-1: Inter-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_Inter for known cells in FR1

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil(M * Kgap) Max(MGRP, SSB period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(Treport, Ceil(M 1.5 * Kgap) Max(MGRP, SSB period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(M * Kgap) DRX cycle CSSFinter |
| Note 1: The definition of Kgap is the same as L3 measurement which is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Note 2: If the UE is incapable of [capability of measurement with RTD>CP], the requirements apply for the cells configured by LTM-CSI-ResourceConfig-r18 on which UE is required to perform L1 measurements, when the max RTD among the cells on the same inter-frequency layer is not larger than CP length. |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.15.3.1 and 9.15.5.1.

#### 6.6.27.1	NR SA FR1 Inter-frequency SSB based L1-RSRP measurement with measurement gap

6.6.27.1.1	Test purpose

To verify that the UE makes correct reporting of inter-frequency SSB based L1-RSRP measurement with measurement gap on candidate neighbour cell in FR1 within L1-RSRP measurement requirements specified in TS 38.133 [6] clause 9.15.

6.6.27.1.2	Test applicability

This test applies to all types of NR UE supporting SSB based inter-frequency L1-RSRP measurements with measurement gaps and reports for LTM from release 18 and forward.

6.6.27.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.27.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.27.1.

6.6.27.1.4	Test description

6.6.27.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.27.1.4.1-1.

Table 6.6.27.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.27.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.27.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.27.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.6.27.1.4.1-2.

Table 6.6.27.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.27.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | -Without LTE link -For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.27.1.4.1-3.

2.	Message contents are defined in clause 6.6.27.1.4.3.

3.	There are two NR carriers and one cell on each carrier in the test. Cell 1 (Pcell) is on NR RF channel 1, and is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter neighbour cell) is on NR RF channel 2.

Table 6.6.27.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbouring cell |  |  | Cell 2 | Cell 2 is the candidate cell |
| A3-Offset |  | dB | -7 | Including test tolerance (-1dB) given in Annex F.1.3.2 |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | ms | 160 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| reportQuantityRS-Indexes |  |  | rsrp |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |
| includeBeamMeasurements |  |  | True |  |
| DRX |  |  | OFF | DRX is not used |
| Time offset between cells |  |  | 3 s | The timing of Cell 2 is later than the timing of Cell 1 |
| Gap Pattern Id |  |  | 0 |  |
| Measurement gap offset |  | ms | 39 |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 80 | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |
|  | spCellInclusion |  | N/A |  |
| ltm-ConfigComplete |  |  | True | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |
| T2 |  | s | 0.5 |  |

6.6.27.1.4.2	Test procedure

In CSI measurement configuration, UE is indicated to perform inter-frequency L1-RSRP measurement on the SSBs and report periodically. The test consists of two successive time periods, with time duration of T1 and T2 respectively.

Measurement gap pattern configuration defined in Table 6.6.27.1.4.1-3 is provided.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.27.1.5-1.

3.	SS shall transmit an RRCReconfiguration message to configure Event A3 triggered measurement report.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	The UE shall transmit a MeasurementReport message triggered by Event A3 including Cell 2 measurement report with SSB index.

6.	SS shall transmit an RRCReconfiguration message to configure LTM-Candidate-r18 for Cell 2. UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports for candidate Cell 2 in PUCCH format 2.

7.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

8.	The UE shall transmit L1-RSRP on PUCCH format 2 in Cell 1 with a periodicity of 80 slots.

9.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.27.1.5-1. T2 starts.

10.	The UE shall continue to send valid L1-RSRP reports. The SS shall check the following requirements:

-R1:	the UE shall start to transmit valid reports no later than 160 ms for configuration 1, 2 and no later than 120 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement (Table 6.6.27.1.5-2 for test configurations 1, 2 and Table 6.6.27.1.5-3 for test configuration 3). For candidate Cell 2, the reported L1-RSRP value in T2 shall be larger than the value reported in T1. If the first valid report is received earlier than the specified time and includes a larger L1-RSRP value of Cell 2, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

-R2:	the UE shall transmit reports every 80 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

-R3:	the L1-RSRP value of Cell 2 reported by the UE is compared to the expected L1-RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.27.1.5-2 for test configurations 1, 2 and in Table 6.6.27.1.5-3 for test configuration 3, or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

11.	The SS waits until T2 expires.

12.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

14.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),

or:

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

15.	Repeat steps 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.27.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.6.27.1.4.3-1: Common Exception messages (step 3)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-5 with condition SSB Index Table H.3.1-6 with conditions gapUE and Pattern #0 Table H.3.1-7 with conditions INTER-FREQ, SSB Index |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.6.27.1.4.3-2: ReportConfigNR (a3-Offset) (step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -14 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms160 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 1 |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.27.1.4.3-3: Common Exception messages (step 6)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.12-1 with condition L1-RSRP Table H.3.12-2 with condition L1-RSRP Table H.3.12-3 Table H.3.12-4 Table H.3.12-6 Table H.3.12-8 |
| Specific message contents exceptiosn for Test Configuration 1 | Table H.3.12-5 with condition SCS15 Table H.3.12-9 with conditions FR1, SSB#0, FDD |
| Specific message contents exceptiosn for Test Configuration 2 | Table H.3.12-5 with condition SCS15 Table H.3.12-9 with conditions FR1, SSB#0, TDD, SCS15 |
| Specific message contents exceptiosn for Test Configuration 3 | Table H.3.12-5 with condition SCS30 Table H.3.12-9 with conditions FR1, SSB#0, TDD, SCS30 |

6.6.27.1.5	Test requirement

Table 6.6.27.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.27.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| SSB GSCN | 1~3 |  | freq1 |  | freq2 |  |
| Duplex mode | 1 |  | FDD |  |  |  |
|  | 2 |  | TDD |  |  |  |
|  | 3 |  | TDD |  |  |  |
| TDD Configuration | 1 |  | N/A |  |  |  |
|  | 2 |  | TDDConf.1.1 |  |  |  |
|  | 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | 2 |  | 10: NRB,c = 52 |  |  |  |
|  | 3 |  | 40: NRB,c = 106 |  |  |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |  | N/A |  |
|  | 2 |  | SR.1.1 TDD |  | N/A |  |
|  | 3 |  | SR.2.1 TDD |  | N/A |  |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |  | N/A |  |
|  | 2 |  | CR.1.1 TDD |  | N/A |  |
|  | 3 |  | CR.2.1 TDD |  | N/A |  |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |  | N/A |  |
|  | 2 |  | CCR.1.1 TDD |  | N/A |  |
|  | 3 |  | CCR.2.1 TDD |  | N/A |  |
| SSB configuration | 1 |  | SSB.1 FR1 |  |  |  |
|  | 2 |  | SSB.1 FR1 |  |  |  |
|  | 3 |  | SSB.2 FR1 |  |  |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| SMTC configuration | 1~3 |  | SMTC.1 |  |  |  |
| TRS Configuration | 1 |  | TRS.1.1 FDD |  | N/A |  |
|  | 2 |  | TRS.1.1 TDD |  | N/A |  |
|  | 3 |  | TRS.1.2 TDD |  | N/A |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| NocNote2 | 1~3 | dBm/15kHz | -94.65 |  |  |  |
| NocNote2 | 1,2 | dBm/SCS | -94.65 |  |  |  |
|  | 3 |  | -91.65 |  |  |  |
| EsIot | 1~3 | dB | 0 | 0 | 0 | 8 |
| EsNoc | 1~3 | dB | 0 | 0 | 0 | 8 |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65 | -94.65 | -94.65 | -86.65 |
|  | 3 |  | -91.64 | -91.64 | -91.64 | -83.64 |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69 | -63.69 | -63.69 | -58.06 |
|  | 3 | dBm/38.16 MHz | -57.58 | -57.58 | -57.58 | -51.96 |
| Propagation condition | 1~3 |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 4: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 80 slots. The UE shall start to report a larger L1-RSRP value of Cell 2 in no later than 80 ms plus 80 slots from the beginning of time period T2. During T2, each L1-RSRP measurement report of Cell 2 shall meet the corresponding absolute accuracy requirement in Table 6.6.27.1.5-2 for test configuration 1, 2 and the corresponding absolute accuracy requirements in Table 6.6.27.1.5-3 for test configuration 3.

Table 6.6.27.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (Cell 2) | 60 |
| Highest reported value (Cell 2) | 80 |

Table 6.6.27.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (Cell 2) | 63 |
| Highest reported value (Cell 2) | 83 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R3) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

### 6.6.28	LTM Inter-frequency L1-RSRP measurement without measurement gap

#### 6.6.28.0	Minimum conformance requirements

6.6.28.0.1	Minimum conformance requirements for SSB-based inter-frequency L1-RSRP measurement without gap

When configured by the network and the inter-frequency cell’s SSB is completely contained in the DL active BWP, the UE shall be able to perform L1-RSRP measurements without gaps and without interruptions.

If a neighbour cell is known according TS 38.133 [6] clause 9.15.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap+TSSB_time_index_inter, where TSSB_time_index_inter is the time period used to acquire the index of the SSB being measured given in TS38.133 [6] clause 9.3.9.1.

The value of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap is defined for FR1 in Table 6.6.28.0.1-1, where

-	M = 1

-	P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

For a window W of duration max (TL1, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	TL1 is periodicity of the target SSB.

If the high layer in TS 38.331 [13] signalling of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

Longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

Longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

Table 6.6.28.0.1-1: Measurement delay for inter-frequency L1-RSRP measurement without measurement gaps in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)*NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbour cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NLayer = the number of inter-frequency layers which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] and measured without measurement gaps + the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]. Note 2: K = 1.5. Note 3: Void. |  |

Table 6.6.28.0.1-2: Measurement delay for inter-frequency L1-RSRP measurement without measurement gaps in FR1 for UE incapable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbour cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: The requirements apply for the cells configured by LTM-CSI-ResourceConfig-r18 on which UE is required to perform L1 measurements, when the max RTD between the serving cell and the inter-frequency cell whose SSB are completely contained in the DL active BWP is not larger than CP length. |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.15.3.1, 9.15.6.1, 9.15.6.2 and 9.15.6.3.

#### 6.6.28.1	NR SA FR1 Inter-frequency SSB based L1-RSRP measurement without measurement gap

6.6.28.1.1	Test purpose

To verify that the UE makes correct reporting of inter-frequency SSB based L1-RSRP measurement without measurement gap on candidate neighbour cell in FR1 within L1-RSRP measurement requirements specified in TS 38.133 [6] clause 9.15.

6.6.28.1.2	Test applicability

This test applies to all types of NR UE supporting SSB based inter-frequency L1-RSRP measurements without measurement gaps and reports for LTM from release 18 and forward. Test 2 is applicable only to UEs supporting multiCellL1-measRTD-greaterThan-CP-r18, otherwise Test 1 is applicable.

6.6.28.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.28.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.28.1.

6.6.28.1.4	Test description

6.6.28.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.28.1.4.1-1.

Table 6.6.28.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.6.28.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.6.28.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.6.28.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.6.28.1.4.1-2.

Table 6.6.28.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.28.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | -Without LTE link -For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.28.1.4.1-3.

2.	Message contents are defined in clause 6.6.28.1.4.3.

3.	There are two NR carriers and one cell on each carrier in the test. Cell 1 (Pcell) is on NR RF channel 1, and is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter neighbour cell) is on NR RF channel 2.

Table 6.6.28.1.4.1-3: General test parameters

| Parameter |  | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| Active cell |  |  | Cell 1 |  |  |
| Neighbour cell |  |  | Cell 2 |  | Cell 2 is candidate cell |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 |  |  |
| A3-Offset |  | dB | -7 |  | Including test tolerance (-1dB) given in Annex F.1.3.2 |
| Hysteresis |  | dB | 0 |  |  |
| Time To Trigger |  | s | 0 |  |  |
| Filter coefficient |  |  | 0 |  | L3 filtering is not used |
| reportQuantityRS-Indexes |  |  | rsrp |  |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |  |
| includeBeamMeasurements |  |  | True |  |  |
| Time offset between cells |  | s | 2 | 20 | The timing of Cell 2 is later than the timing of Cell 1 |
| DRX |  |  | OFF |  |  |
| LTM-CSI-ReportConfig | L1-RSRP reporting period | slot | 20 |  | Periodic L1-RSRP reporting configured |
|  | nrOfReportedCells |  | n1 |  | Report candidate cell’s (Cell 2) L1-RSRP measurement results |
|  | nrOfReportedRS-PerCell |  | n1 |  |  |
|  | spCellInclusion |  | N/A |  |  |
| ltm-ConfigComplete |  |  | True |  | Candidate cell’s configuration is complete configuration |
| T1 |  | s | 0.3 |  |  |
| T2 |  | s | 0.5 |  |  |

6.6.28.1.4.2	Test procedure

In CSI measurement configuration, UE is indicated to perform inter-frequency L1-RSRP measurement on the SSBs and report periodically. The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 1 and Cell 2 should be different in frequency location within the cell bandwidth.

There are two tests in the test case, test 1 and test 2:

-	In test 1, time offset between cells is within CP length.

-	In test 2, time offset between cells is larger than CP length.

UE not capable of multiCellL1-measRTD-greaterThan-CP-r18 is only required to pass test 1. Otherwise, it is only required to pass test 2.

The test consists of two successive time periods, with time duration of T1 and T2 respectively. There is no measurement gap configured in the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 6.6.28.1.5-1.

3.	SS shall transmit an RRCReconfiguration message to configure Event A3 triggered measurement report.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	The UE shall transmit a MeasurementReport message triggered by Event A3 including Cell 2 measurement report with SSB index.

6.	SS shall transmit an RRCReconfiguration message to configure LTM-Candidate-r18 for Cell 2. UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports for candidate Cell 2 in PUCCH format 2.

7.	The UE shall transmit RRCReconfigurationComplete message. T1 starts.

8.	The UE shall transmit L1-RSRP on PUCCH format 2 in Cell 1 with a periodicity of 20 slots.

9.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.28.1.5-1. T2 starts at the beginning of a frame with an even SFN.

10.	The UE shall continue to send valid L1-RSRP reports. The SS shall check the following requirements:

-R1:	the UE shall start to transmit valid reports no later than 40 ms for configuration 1, 2 and no later than 30 ms for configuration 3 from the beginning of time period T2. A valid report shall meet the absolute L1-RSRP requirement (Table 6.6.28.1.5-2 for test configurations 1, 2 and Table 6.6.28.1.5-3 for test configuration 3). If the first valid report is received earlier than the specified time, the number of passed iterations for R1 is increased by one. Otherwise, the number of failed iterations for R1 is increased by one.

-R2:	the UE shall transmit reports every 20 slots until the end of time period T2. If the reports are received accordingly, the number of passed iterations for R2 is increased by one. Otherwise, the number of failed iterations for R2 is increased by one.

-R3:	the L1-RSRP value of Cell 2 reported by the UE is compared to the expected L1-RSRP value. In all consecutive reports after the first valid value is received, if the resulting value is outside the limits in Table 6.6.28.1.5-2 for test configurations 1, 2 and in Table 6.6.28.1.5-3 for test configuration 3, or the UE fails to report the measurement value for Cell 2, the number of failed iterations for R3 is increased by one. Otherwise, the number of passed iterations for R3 is increased by one.

11.	The SS waits until T2 expires.

12.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

13.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

14.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),

or:

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

15.	Repeat steps 2-14 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.28.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.6.28.1.4.3-1: Common Exception messages (step 3)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ Table H.3.1-5 with condition SSB Index Table H.3.1-7 with conditions INTER-FREQ, SSB Index |
| Specific message contents exceptions for Cell 1 | Table H.3.6-3 with condition SSB Table H.3.6-10 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 for configuration 6.6.28.1-1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 for configuration 6.6.28.1-2 and configuration 6.6.28.1-3 |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 for configuration 6.6.28.1-1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 for configuration 6.6.28.1-2 and configuration 6.6.28.1-3 |

Table 6.6.28.1.4.3-2: ReportConfigNR (a3-Offset) (step 3)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -14 | The actual value is field value * 0.5 dB |  |
| } |  |  |  |
| Hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| maxReportCells | 1 |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.6.28.1.4.3-3: Common Exception messages (step 6)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.12-1 with condition L1-RSRP Table H.3.12-2 with condition L1-RSRP Table H.3.12-3 Table H.3.12-4 Table H.3.12-6 Table H.3.12-8 |
| Specific message contents exceptiosn for Test Configuration 1 | Table H.3.12-9 with conditions FR1, SSB#0, FDD |
| Specific message contents exceptiosn for Test Configuration 2 | Table H.3.12-9 with conditions FR1, SSB#0, TDD, SCS15 |
| Specific message contents exceptiosn for Test Configuration 3 | Table H.3.12-9 with conditions FR1, SSB#0, TDD, SCS30 |

Table 6.6.28.1.4.3-4: LTM-CSI-ReportConfig-r18 (step 6)

| Derivation path: Table H.3.12-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-CSI-ReportConfig-r18 ::= SEQUENCE { |  |  |  |
| ltm-ReportConfigType-r18 CHOICE { |  |  |  |
| periodic-r18 SEQUENCE { |  |  |  |
| reportSlotConfig-r18 |  |  |  |
| slots20 | 2 | Periodicity 20 slots and offset 2 | SCS15 |
| slots20 | 7 | Periodicity 20 slots and offset 7 | SCS30 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.28.1.5	Test requirement

Table 6.6.28.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.6.28.1.5-1: Cell specific test parameters

| Parameter |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | 1 |  | 2 |  |
| SSB GSCN |  |  | freq1 |  | freq2 |  |
| Duplex mode | Config 1 |  | FDD |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration | Config 1 |  | Not Applicable |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |
| BWchannel | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| BWP BW | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |
|  | Config 2 |  | 10: NRB,c = 52 |  |  |  |
|  | Config 3 |  | 40: NRB,c = 106 |  |  |  |
| PDSCH Reference | Config 1 |  | SR.1.1 FDD |  | N/A |  |
| measurement channel | Config 2 |  | SR.1.1 TDD |  | N/A |  |
|  | Config 3 |  | SR.2.1 TDD |  | N/A |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  | N/A |  |
|  | Config 2 |  | CR.1.1 TDD |  | N/A |  |
|  | Config 3 |  | CR.2.1 TDD |  | N/A |  |
| CP length |  |  | Normal |  |  |  |
| TRS configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |
| SMTC Configuration | Config 1 |  | SMTC.2 |  | SMTC.5 |  |
|  | Config 2,3 |  | SMTC.1 |  | SMTC.4 |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  | SSB.5 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  | SSB.6 FR1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1,2 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| PUCCH/PUSCH subcarrier spacing | Config 1,2 | kHz | 15 |  |  |  |
|  | Config 3 |  | 30 |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  | NA |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  | NA |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  | NA |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  | NA |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |
| NocNote2 | Config 1,2 | dBm/15kHz | -101 | -101 | -101 | -101 |
|  | Config 3 |  | -101 | -102.2 | -101 | -102.2 |
| NocNote2 | Config 1,2 | dBm/SCS | -101 | -101 | -101 | -101 |
|  | Config 3 |  | -98 | -99.2 | -98 | -99.2 |
| EsIot |  | dB | -1.76 | -0.19 | -1.76 | -0.19 |
| EsNoc |  | dB | 3 | 13.5 | 3 | 13.5 |
| SSB_RP | Config 1,2 | dBm/SCS | -98 | -87.5 | -98 | -87.5 |
|  | Config 3 | dBm/SCS | -94.99 | -85.69 | -94.99 | -85.69 |
| IoNote3 | Config 1,2 | dBm/ 9.36MHz | -66.07 | -56.44 | -66.07 | -56.44 |
|  | Config 3 | dBm/ 38.16MHz | -59.96 | -51.54 | -59.96 | -51.54 |
| Propagation condition |  | - | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-RSRP report every 20 slots. The UE shall start to report L1-RSRP value of Cell 2 in no later than 20 ms plus 20 slots from the beginning of time period T2. During T2, each L1-RSRP measurement report of Cell 2 shall meet the corresponding absolute accuracy requirements in Table 6.6.28.1.5-2 for test configuration 1, 2 and the corresponding absolute accuracy requirements in Table 6.6.28.1.5-3 for test configuration 3.

Table 6.6.28.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (Cell 2) | 59 |
| Highest reported value (Cell 2) | 79 |

Table 6.6.28.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T2 |
| --- | --- |
| Lowest reported value (Cell 2) | 61 |
| Highest reported value (Cell 2) | 81 |

For the test to pass, the ratio of successful reported values for each requirement (R1 to R4) shall be more than 90% with a confidence level of 95%. Each requirement is evaluated independently of the others.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

## 6.7	Measurement performance requirements

### 6.7.1	SS-RSRP

#### 6.7.1.0	Minimum conformance requirements

6.7.1.0.1	Intra-frequency absolute SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.1.

6.7.1.0.2	Intra-frequency relative SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.2.

6.7.1.0.3	Inter-frequency absolute SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.3.

6.7.1.0.4	Inter-frequency relative SS-RSRP measurement accuracy requirements

Same as in clause 4.7.1.0.4.

#### 6.7.1.1	Intra-frequency measurements

##### 6.7.1.1.1	NR SA FR1 SS-RSRP absolute measurement accuracy

6.7.1.1.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.1.

6.7.1.1.1.4	Test description

6.7.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.1.1.4.1-1.

Table 6.7.1.1.1.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.1.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.1.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.1.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.1.1.4.1-2.

Table 6.7.1.1.1.4.1-2: Initial conditions for SS-RSRP intra-frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 6.7.1.1.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.1.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.1.1.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in Table 6.7.1.1.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.1.1.1.5-1 as appropriate and repeat steps 5-7.

6.7.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.1.1.1.4.3-1: Common Exception messages for NR SA FR1 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4AB with condition RSRP Table H.3.1-5 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-1 | Table H.3.1-3 with Condition SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-2 | Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.1.1.1-3 | Table H.3.1-3 with Condition SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.1.1.1.4.3-2: Void

6.7.1.1.1.5	Test requirement

Table 6.7.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.1.1.1.5-3 for test configuration 3.

Table 6.7.1.1.1.5-1: NR SA FR1 SS-RSRP measurement accuracy test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Physical cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1 |  | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 | SSB 1.FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB 2.FR1 | SSB.2 FR1 | SSB 2.FR1 | SSB.2 FR1 | SSB 2.FR1 | SSB.2 FR1 |
| Time offset with Cell 2 |  | Config 1 | ms | - | 3 | - | 3 | - | 3 |
|  |  | Config 2,3 | s | - | 3 | - | 3 | - | 3 |
| SMTC Configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 | Config 1,2 | Depending on band group | dBm/15KhZ | -107.5 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  | -94 |  | -115 + ΔBG_offset |  |
| NocNote2 | Config 1,2 |  | dBm/SCS | -107.4 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | Depending on band group |  | N/ANote 6 |  | -91 |  | -112 + ΔBG_offset |  |
| EsIot |  |  | dB | 2.23 | -5.57 | 2.23 | -5.57 | 0.37 | -5.56 |
| EsNoc |  |  | dB | 6 | 1.4 | 6 | 1.4 | 3 | -0.8 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -101.5 | -106.1 | -82 | -86.6 | -111 + ΔBG_offset | -114.8 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/ANote 6 | N/ANote 6 | -85 | -89.6 | -109+ ΔBG_offset | -112.8+ ΔBG_offset |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -71.68 |  | -52.18 |  | -80.39+ ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/ANote 6 |  | -51.91 |  | -75.12 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. Note 6: Subtest 1 is not used when testing with 30kHz SSB SCS. |  |  |  |  |  |  |  |  |  |

Table 6.7.1.1.1.5-2: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 44 | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 36 |
|  |  |  | Bands NR_FDD_FR1_B | 36 |
|  |  |  | Bands NR_TDD_FR1_C | 37 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 38 |
|  |  |  | Bands NR_FDD_FR1_G | 39 |
|  |  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 2) | 56 | 79 | NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  |  | NR_FDD_FR1_B | 48 |
|  |  |  | NR_TDD_FR1_C | 49 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  |  | NR_FDD_FR1_G | 51 |
|  |  |  | NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | 40 | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  |  | Bands NR_FDD_FR1_B | 32 |
|  |  |  | Bands NR_TDD_FR1_C | 32 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 33 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 33 |
|  |  |  | Bands NR_FDD_FR1_G | 34 |
|  |  |  | Bands NR_FDD_FR1_H | 35 |
| Highest reported value (Cell 2) | 61 | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  |  | Bands NR_FDD_FR1_B | 53 |
|  |  |  | Bands NR_TDD_FR1_C | 53 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 54 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 54 |
|  |  |  | Bands NR_FDD_FR1_G | 55 |
|  |  |  | Bands NR_FDD_FR1_H | 56 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

Table 6.7.1.1.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest reported value (Cell 2) | N/A | 57 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 38 |
|  |  |  | Bands NR_FDD_FR1_B | 38 |
|  |  |  | Bands NR_TDD_FR1_C | 39 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 39 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 40 |
|  |  |  | Bands NR_FDD_FR1_G | 41 |
|  |  |  | Bands NR_FDD_FR1_H | 41 |
| Highest reported value (Cell 2) | N/A | 76 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  |  | Bands NR_FDD_FR1_B | 50 |
|  |  |  | Bands NR_TDD_FR1_C | 51 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 51 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 52 |
|  |  |  | Bands NR_FDD_FR1_G | 53 |
|  |  |  | Bands NR_FDD_FR1_H | 53 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| Lowest reported value (Cell 2) | N/A | 54 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  |  | Bands NR_FDD_FR1_B | 34 |
|  |  |  | Bands NR_TDD_FR1_C | 34 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 35 |
|  |  |  | Bands NR_FDD_FR1_G | 36 |
|  |  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | N/A | 79 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 54 |
|  |  |  | Bands NR_FDD_FR1_B | 55 |
|  |  |  | Bands NR_TDD_FR1_C | 55 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 56 |
|  |  |  | Bands NR_FDD_FR1_E, Bands NR_TDD_FR1_E | 56 |
|  |  |  | Bands NR_FDD_FR1_G | 57 |
|  |  |  | Bands NR_FDD_FR1_H | 58 |
| Note 1: NR operating band groups are as defined in Section 3A.4.1. |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.1.1.2	NR SA FR1 SS-RSRP relative measurement accuracy

6.7.1.1.2.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.1.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.1.

6.7.1.1.2.4	Test description

6.7.1.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.1.2.4.1-1.

Table 6.7.1.1.2.4.1-1: NR SA FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.1.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.1.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.1.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.1.2.4.1-2.

Table 6.7.1.1.2.4.1-2: Initial conditions for SS-RSRP intra-frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1. Message contents are defined in clause 6.7.1.1.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in the same frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.1.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.1.1.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRP reported values of Cell 1 and Cell 2 in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the reported SS-RSRP of Cell 1. If the resulting value is outside the limits in Table 6.7.1.1.2.5-2 or the UE fails to report the measurement value for Cell 1 or Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.1.1.2.5-1 as appropriate and repeat steps 5-7.

6.7.1.1.2.4.3	Message contents

Message contents are same as in clause 6.7.1.1.1.4.3.

6.7.1.1.2.5	Test requirement

Table 6.7.1.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.1.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.1.2.5-2.

Table 6.7.1.1.2.5-1: Same as Table 6.7.1.1.1.5-1 with the following exceptions:

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| NocNote2 | Config 1,2 | Depending on band group | dBm/15KhZ | -106 |  | -88 |  | -114 + ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | N/A |  | -94 |  | -114+ ΔBG_offset |  |
| NocNote2 | Config 1,2 |  | dBm/SCS | Same as Noc/15kHz |  | Same as Noc/15kHz |  | Same as Noc/15kHz |  |
|  | Config 3,6 | Depending on band group |  | N/A |  | -91 |  | -111 + ΔBG_offset |  |
| EsIot |  |  | dB | 1.88 | -4.97 | 1.88 | -4.97 | -0.01 | -4.76 |
| EsNoc |  |  | dB | 6 | 2 | 6 | 2 | 3 | 0 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -100 | -104 | -82 | -86 | -111 + ΔBG_offset | - 114 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | N/A | N/A | -85 | -89 | -108 + ΔBG_offset | -111 + ΔBG_offset |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -70.05 |  | -52.05 |  | -80.20+ ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | N/A |  | -51.77 |  | -73.93 + ΔBG_offset |  |

Table 6.7.1.1.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRP_x - 8 | RSRP_x - 8 | RSRP_x - 7 |
| Highest reported value (Cell 2) | RSRP_x - 1 | RSRP_x - 1 | RSRP_x + 1 |
| RSRP_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.1.2	Inter-frequency measurements

##### 6.7.1.2.1	NR SA FR1-FR1 SS-RSRP absolute measurement accuracy

6.7.1.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.1.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.2.

6.7.1.2.1.4	Test description

6.7.1.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.2.1.4.1-1.

Table 6.7.1.2.1.4.1-1: NR SA FR1-FR1 SS-RSRP measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.2.1.4.1-2.

Table 6.7.1.2.1.4.1-2: Initial conditions for SS-RSRP inter-frequency absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] sclause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.1.2.1.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in a different FR1 frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.2.1.4.2	Test procedure

Same as in clause 6.7.1.1.1.4.2 but replacing Table 6.7.1.1.1.5-1 and 6.7.1.1.1.5-2 with 6.7.1.2.1.5-1 and 6.7.1.2.1.5-2, respectively.

6.7.1.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.1.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-RSRP absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-4AB with condition RSRP Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.1.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.1.2.1.4.3-2: Void

6.7.1.2.1.5	Test requirement

Table 6.7.1.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.1.2.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.1.2.1.5-3 for test configuration 3.

Table 6.7.1.2.1.5-1: SS-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  | Cell 1 | Cell 2 |  |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 |  | freq1 | freq2 |  |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |
| Gap pattern ID |  |  |  | 0 |  |  | 0 |  |  |
| Duplex mode |  | 1 |  | FDD |  |  | FDD |  |  |
|  |  | 2 |  | TDD |  |  | TDD |  |  |
|  |  | 3 |  | TDD |  |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - |  | SR.1.1 FDD | - |  |
|  |  | 2 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - |  | CR.1.1 FDD | - |  |
|  |  | 2 |  | CR.1.1 TDD | - |  | CR.1.1 TDD | - |  |
|  |  | 3 |  | CR.2.1 TDD | - |  | CR.2.1 TDD | - |  |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - |  | CCR.1.1 FDD | - |  |
|  |  | 2 |  | CCR.1.1 TDD | - |  | CCR.1.1 TDD | - |  |
|  |  | 3 |  | CCR.2.1 TDD | - |  | CCR.2.1 TDD | - |  |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  | - |
|  |  | 2 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  |  | SMTC.1 |  |  |
| Time offset between Cell 2 and Cell 3 |  | 1 | ms | 3 |  |  | 3 |  |  |
|  |  | 2,3 | s | 3 |  |  | 3 |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |  | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| NocNote2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -94.65 |  | ( for Cell 2 +8dB) | -115+ ΔBG_offset |  |
| NocNote2 | Depending on band group | 3 | dBm/15kHz | -96 | -96 |  | ( for Cell 2 +8dB) | -115+ ΔBG_offset |  |
| NocNote2 | Depending on band group | 1,2 | dBm/SSB SCS | -94.65 | -94.65 |  | ( for Cell 2 +8dB) | -115+ ΔBG_offset |  |
|  | Depending on band group | 3 |  | -93 | -93 |  | ( for Cell 2 +8dB) | -112.00+ ΔBG_offset |  |
| EsIot |  | 1~3 | dB | 10 | 10 |  | 13 | -3 |  |
| SS-RSRPNote3 | Depending on band group | 1,2, | dBm/SCS | -84.65 | -84.65 |  | (RSRP for Cell 2 +24dB) | -118.00+ ΔBG_offset |  |
|  | Depending on band group | 3 |  | -83 | -83 |  | (RSRP for Cell 2 +24dB) | -115.00+ ΔBG_offset |  |
| IoNote3 | Depending on band group | 1,2 | dBm/ 9.36MHz | 56.28 | 56.28 |  | (Io for Channel 2 +19.45dB) | -85.28+ ΔBG_offset |  |
|  | Depending on band group | 3 | dBm/ 38.16MHz | -51.53 | -51.53 |  | (Io for Channel 2 +19.45dB) | -79.19+ ΔBG_offset |  |
| EsNoc |  | 1~3 | dB | 10 | 10 |  | 13 | -3 |  |
| Propagation condition |  | 1~3 | - | AWGN |  |  | AWGN |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5 The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. Note 6: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |  |  |  |

Table 6.7.1.2.1.5-2: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 1, 2, 4 and 5

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 32 |
|  |  | Bands NR_FDD_FR1_B | 33 |
|  |  | Bands NR_TDD_FR1_C | 33 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 34 |
|  |  | Bands NR_FDD_FR1_G | 35 |
|  |  | Bands NR_FDD_FR1_H | 36 |
| Highest reported value (Cell 2) | 81 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 45 |
|  |  | Bands NR_FDD_FR1_B | 45 |
|  |  | Bands NR_TDD_FR1_C | 46 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 46 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 47 |
|  |  | Bands NR_FDD_FR1_G | 48 |
|  |  | Bands NR_FDD_FR1_H | 48 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 28 |
|  |  | Bands NR_FDD_FR1_B | 28 |
|  |  | Bands NR_TDD_FR1_C | 29 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 29 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 30 |
|  |  | Bands NR_FDD_FR1_G | 31 |
|  |  | Bands NR_FDD_FR1_H | 31 |
| Highest reported value (Cell 2) | 84 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 49 |
|  |  | Bands NR_FDD_FR1_B | 50 |
|  |  | Bands NR_TDD_FR1_C | 50 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 51 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 51 |
|  |  | Bands NR_FDD_FR1_G | 52 |
|  |  | Bands NR_FDD_FR1_H | 53 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 6.7.1.2.1.5-3: SS-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 3 and 6

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 64 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 35 |
|  |  | Bands NR_FDD_FR1_B | 36 |
|  |  | Bands NR_TDD_FR1_C | 36 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 37 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 37 |
|  |  | Bands NR_FDD_FR1_G | 38 |
|  |  | Bands NR_FDD_FR1_H | 39 |
| Highest reported value (Cell 2) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 49 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  | Bands NR_FDD_FR1_G | 51 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 32 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 33 |
|  |  | Bands NR_FDD_FR1_G | 34 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 86 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  | Bands NR_FDD_FR1_B | 53 |
|  |  | Bands NR_TDD_FR1_C | 53 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 54 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 54 |
|  |  | Bands NR_FDD_FR1_G | 55 |
|  |  | Bands NR_FDD_FR1_H | 56 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.1.2.2	NR SA FR1-FR1 SS-RSRP relative measurement accuracy

6.7.1.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.1.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.1.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.1.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.1.2.

6.7.1.2.2.4	Test description

6.7.1.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.1.2.2.4.1-1.

Table 6.7.1.2.2.4.1-1: NR SA FR1-FR1 SS-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.1.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.1.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.1.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.1.2.2.4.1-2.

Table 6.7.1.2.2.4.1-2: Initial conditions for SS-RSRP inter-frequency relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.1.2.2.4.3.

2. Cell 1 is the NR FR1 serving cell (PCell) and Cell 2 is the NR neighbour in a different FR1 frequency and the target cell for SS-RSRP measurements. The connection setup is done according to the settings in Annex C.1.1 and C.1.2.

6.7.1.2.2.4.2	Test procedure

Same as in clause 6.7.1.1.2.4.2 but replacing Table 6.7.1.1.2.5-1 and 6.7.1.1.2.5-2 with 6.7.1.2.2.5-1 and 6.7.1.2.2.5-2, respectively.

6.7.1.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.1.2.1.4.3.

6.7.1.2.2.5	Test requirement

Table 6.7.1.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.1.2.2.5-1 shall meet the corresponding relative accuracy requirements in Table 6.7.1.2.2.5-2.

Table 6.7.1.2.2.5-1: same as Table 6.7.1.2.1.5-1

Table 6.7.1.2.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SS-RSRP_x - 7 | SS-RSRP_x - 31 |
| Highest reported value (Cell 2) | SS-RSRP_x + 7 | SS-RSRP_x - 18 |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SS-RSRP_x - 9 | SS-RSRP_x - 33 |
| Highest reported value (Cell 2) | SS-RSRP_x + 9 | SS-RSRP_x - 17 |
| SS-RSRP_x is the reported value of Cell 1 |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.2	SS-RSRQ

#### 6.7.2.0	Minimum conformance requirements

6.7.2.0.1	Intra-frequency SS-RSRQ measurement accuracy requirements

Same as in clause 4.7.2.0.1.

6.7.2.0.2	Inter-frequency SS-RSRQ absolute measurement accuracy requirements

Same as in clause 4.7.2.0.2.

6.7.2.0.3	Inter-frequency SS-RSRQ relative measurement accuracy requirements

Same as in clause 4.7.2.0.3.

#### 6.7.2.1	NR SA FR1 SS-RSRQ measurement accuracy

6.7.2.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-RSRQ measurement accuracy is within the specified limits for all bands.

6.7.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.1.

6.7.2.1.4	Test description

6.7.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.1.4.1-1.

Table 6.7.2.1.4.1-1: NR SA FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.1.4.1-2.

Table 6.7.2.1.4.1-2: Initial conditions for SS-RSRQ intra-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.3.

6.7.2.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.2.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in Table 6.7.2.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.2.1.5-1 as appropriate and repeat steps 5-7.

6.7.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.2.1.4.3-1: Common Exception messages for NR SA FR1 SS-RSRQ measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4AB with condition RSRQ Table H.3.1-5 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-1 | Table H.3.1-3 with Condition SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-2 | Table H.3.1-3 with Condition SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.2.1-3 | Table H.3.1-3 with Condition SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.2.1.4.3-2: Void

6.7.2.1.5	Test requirement

Table 6.7.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 6.7.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.1.5-2.

Table 6.7.2.1.5-1: SS-RSRQ Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Control Channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 | s | - | 3 | - | 3 | - | 3 |
|  |  | Config 1 | ms | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 2, 3 |  | SMTC.1 |  |  |  |  |  |
|  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 | Config 1,2 | Depending on band group | dBm/15kHz | -86.5 |  | -101 |  | -114+ ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | -92.6 |  | - |  | -114+ ΔBG_offset |  |
| NocNote2 | Config 1,2 | Depending on band group | dBm/SC S | -86.5 |  | -101 |  | -114+ ΔBG_offset |  |
|  | Config 3 | Depending on band group |  | -89.6 |  | - |  | -111+ ΔBG_offset |  |
| EsIot |  |  | dB | -1.76 |  | -4.7 |  | -5.46 | -5.46 |
| EsNoc |  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -83.5 | -83.5 | -103.9 | -103.9 | -118+ΔBG_offset | -118+ ΔBG_offset |
|  | Config 3 | Depending on band group |  | -86.6 | -86.6 | - | - | -115+ ΔBG_offset | -115+ ΔBG_offset |
| SS-RSRQ Note3 |  |  | dB | -14.77 | -14.77 | -16.76 | -16.76 | -17.34 | -17.34 |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -51.57 |  | -70 |  | -83.28+ ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | -51.56 |  | - |  | -76.67+ ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2. Note 6: Subtest 2 is not used when testing with 30kHz SSB SCS. Note 7: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

Table 6.7.2.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_52 | SS-RSRQ_46 | SS-RSRQ_44 |
| Highest reported value (Cell 2) | SS-RSRQ_62 | SS-RSRQ_60 | SS-RSRQ_59 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_49 | SS-RSRQ_45 | SS-RSRQ_43 |
| Highest reported value (Cell 2) | SS-RSRQ_65 | SS-RSRQ_61 | SS-RSRQ_60 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.2.2	Inter-Frequency SS-RSRQ measurement accuracy

##### 6.7.2.2.1	NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy

6.7.2.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRQ absolute measurement accuracy is within the specified limits for all bands.

6.7.2.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.2.1.

6.7.2.2.1.4	Test description

6.7.2.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.2.1.4.1-1.

Table 6.7.2.2.1.4.1-1: NR SA FR1-FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.2.1.4.1-2.

Table 6.7.2.2.1.4.1-2: Initial conditions for SS-RSRQ inter-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.2.2.1.4.2	Test procedure

Same as in clause 6.7.2.1.1.4.2 but replacing Table 6.7.2.1.1.5-1 and 6.7.2.1.1.5-2 with 6.7.2.2.1.5-1 and 6.7.2.2.1.5-2, respectively.

6.7.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.2.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-4AB with condition RSRQ Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.1 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.2.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, SSB.2 FR1 and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.2.2.1.4.3-2: Void

6.7.2.2.1.5	Test requirement

Table 6.7.2.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 6.7.2.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.2.1.5-2.

Table 6.7.2.2.1.5-1: SS-RSRQ Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Gap pattern ID |  |  |  | 0 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | R.1.1 FDD | - | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 | s | 3 |  |  |  |  |  |
|  |  | Config 1 | ms | 3 |  |  |  |  |  |
| SMTC configuration |  | Config 2, 3 |  | SMTC.1 |  |  |  |  |  |
|  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 in FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 in FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 | Config 1,2 | Depending on band group | dBm/15kHz | -81.68 | -81.68 | -106 | -106 | -116 + ΔBG_offset | -116 + ΔBG_offset |
| NocNote2 | Config 3 | Depending on band group | dBm/15kHz | -87.80 | -87.80 | -113 | -113 | -116+ ΔBG_offset | -116+ ΔBG_offset |
| NocNote2 | Config 1,2 | Depending on band group | dBm/SCS | -81.68 | -81.68 | -106 | -106 | -116 + ΔBG_offset | -116 + ΔBG_offset |
|  | Config 3 | Depending on band group |  | -84.8 | -84.8 | -110 | -110 | -113+ ΔBG_offset | -113+ ΔBG_offset |
| EsIot |  |  | dB | -1.75 | -1.75 | -1.75 | -1.75 | 3 | -1.75 |
| EsNoc |  |  | dB | -1.75 | -1.75 | -1.75 | -1.75 | 3 | -1.75 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -83.43 | -83.43 | -107.75 | -107.75 | -113+ ΔBG_offset | -117.75+ ΔBG_offset |
|  | Config 3 | Depending on band group |  | -86.54 | -86.54 | -111.75 | -111.75 | -110+ ΔBG_offset | -114.75+ ΔBG_offset |
| SS-RSRQNote3 |  |  | dB | -14.76 | -14.76 | -14.76 | -14.76 | -12.56 | -14.76 |
| IoNote3 | Config 1,2 | Depending on band group | dBm/Ch BW | -51.51 | -51.51 | -75.83 | -75.83 | -83.28+ ΔBG_offset | -85.83+ ΔBG_offset |
|  | Config 3 | Depending on band group |  | -51.52 | -51.52 | -76.73 | -76.73 | -77.19+ ΔBG_offset | -79.73+ ΔBG_offset |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

Table 6.7.2.2.1.5-2: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_52 | SS-RSRQ_52 | SS-RSRQ_52 |
| Highest reported value (Cell 2) | SS-RSRQ_62 | SS-RSRQ_62 | SS-RSRQ_62 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_49 | SS-RSRQ_49 | SS-RSRQ_49 |
| Highest reported value (Cell 2) | SS-RSRQ_65 | SS-RSRQ_65 | SS-RSRQ_65 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.2.2.2	NR SA FR1-FR1 SS-RSRQ relative measurement accuracy

6.7.2.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-RSRQ relative measurement accuracy is within the specified limits for all bands.

6.7.2.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.2.2.2.

6.7.2.2.2.4	Test description

6.7.2.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.2.2.2.4.1-1.

Table 6.7.2.2.2.4.1-1: NR SA FR1-FR1 SS-RSRQ measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.2.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.2.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.2.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.2.2.2.4.1-2.

Table 6.7.2.2.2.4.1-2: Initial conditions for SS-RSRQ inter-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.2.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.2.2.2.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for SS-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.2.2.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.2.2.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the SS-RSRQ value of Cell 1 reported by the UE. If the difference between both values is outside the limits in Table 6.7.2.2.2.5-2 or the UE fails to report the measurement value for Cell 2 or Cell 1, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.2.2.2.5-1 as appropriate and repeat steps 5-7.

6.7.2.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.2.2.1.4.3.

6.7.2.2.2.5	Test requirement

Table 6.7.2.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 6.7.2.2.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.2.2.2.5-2.

Table 6.7.2.2.2.5-1: same as Table 6.7.2.2.1.5-1

Table 6.7.2.2.2.5-2: SS-RSRQ Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x - 7 | SS-RSRQ_x - 7 | SS-RSRQ_x - 11 |
| Highest reported value (Cell 2) | SS-RSRQ_x + 7 | SS-RSRQ_x + 7 | SS-RSRQ_x + 2 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_x - 9 | SS-RSRQ_x - 9 | SS-RSRQ_x – 13 |
| Highest reported value (Cell 2) | SS-RSRQ_x + 9 | SS-RSRQ_x + 9 | SS-RSRQ_x + 4 |
| SS-RSRQ_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.3	SS-SINR

#### 6.7.3.0	Minimum conformance requirements

6.7.3.0.1	Intra-frequency SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.1.

6.7.3.0.2	Inter-frequency absolute SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.2.

6.7.3.0.3	Inter-frequency relative SS-SINR measurement accuracy requirements

Same as in clause 4.7.3.0.3.

#### 6.7.3.1	NR SA FR1 SS-SINR measurement accuracy

6.7.3.1.1	Test purpose

The purpose of this test is to verify that the intra-frequency SS-SINR measurement accuracy is within the specified limits for all bands.

6.7.3.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.1.

6.7.3.1.4	Test description

6.7.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.1.4.1-1.

Table 6.7.3.1.4.1-1: NR SA FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.1.4.1-2.

Table 6.7.3.1.4.1-2: Initial conditions for SS-SINR intra-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.3.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 6.7.3.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.3.1.5-1 as appropriate and repeat steps 5-7.

6.7.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.3.1.4.3-1: Common Exception messages for NR SA FR1 SS-SINR measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-4AB with condition SINR Table H.3.1-5 Table H.3.1-7 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-1 | Table H.3.1-3 with Condition SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-2 | Table H.3.1-3 with Condition Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.3.1-3 | Table H.3.1-3 with Condition Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.3.1.4.3-2: Void

6.7.3.1.5	Test requirements

Table 6.7.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.3.1.5-2

Table 6.7.3.1.5-1: SS-SINR Intra frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD | - | TRS.1.1 TDD | - |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 | s | - | 3 | - | 3 |
|  |  | Config 1 | ms | - | 3 | - | 3 |
| SMTC configuration |  | Config 2, 3 |  | SMTC.1 |  |  |  |
|  |  | Config 1 |  | SMTC.2 |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| NocNote2 |  | Depending on band group | dBm/15kHz | -93 |  | -116+ ΔBG_offset |  |
| NocNote2 | Config 1,2 |  | dBm/SCS | -93.2 |  | Same as Noc for 15kHz |  |
|  | Config 3 | Depending on band group |  | -90.2 |  | -113+ ΔBG_offset |  |
| EsIot |  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
| EsNoc |  |  | dB | 4.54 | 2.66 | -3.5 | -3.5 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -88.46 | -90.34 | -119.5+ ΔBG_offset | -119.5+ ΔBG_offset |
|  | Config 3 | Depending on band group |  | -85.65 | -87.53 | -116.5+ ΔBG_offset | -116.5+ ΔBG_offset |
| SS-SINR Note3 |  |  | dB | 0 | -3.19 | -5.1 | -5.1 |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -57.5 |  | -85.28+ ΔBG_offset |  |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | -51.59 |  | -79.17+ ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |

Table 6.7.3.1.5-2: SS-SINR Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SS-SINR_31 | SS-SINR_28 |
| Highest reported value (Cell 2) | SS-SINR_49 | SS-SINR_45 |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SS-SINR_30 | SS-SINR_27 |
| Highest reported value (Cell 2) | SS-SINR_50 | SS-SINR_46 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.3.2	Inter-Frequency SS-SINR measurement accuracy

##### 6.7.3.2.1	NR SA FR1-FR1 SS-SINR absolute measurement accuracy

6.7.3.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR absolute measurement accuracy is within the specified limits for all bands.

6.7.3.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.2.1.

6.7.3.2.1.4	Test description

6.7.3.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.2.1.4.1-1.

Table 6.7.3.2.1.4.1-1: NR SA FR1-FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.2.1.4.1-2.

Table 6.7.3.2.1.4.1-2: Initial conditions for SS-SINR inter-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.2.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.2.1.4.2	Test procedure

Same as in clause 6.7.3.1.4.2 but replacing Table 6.7.3.1.5-1 and 6.7.3.1.5-2 with 6.7.3.2.1.5-1 and 6.7.3.2.1.5-2, respectively.

6.7.3.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.3.2.1.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-SINR absolute measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-4AB with condition SINR Table H.3.1-5 Table H.3.1-7 with condition INTER-FREQ Table H.3.1-6 with condition Pattern #0 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO, and Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 6.7.3.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO, and Synchronous cells and SS-SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.3.2.1.4.3-2: Void

6.7.3.2.1.5	Test requirements

Table 6.7.3.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.3.2.1.5-2.

Table 6.7.3.2.1.5-1: SS-SINR Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 | freq1 | freq2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| Gap pattern ID |  |  |  | 0 | - | 0 | - | 0 | - |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD | - | TRS.1.1 TDD | - | TRS.1.1 TDD | - |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 2, 3 | s | - | 3 | - | 3 | - | 3 |
|  |  | Config 1 | ms | - | 3 | - | 3 | - | 3 |
| SMTC configuration |  | Config 2, 3 |  | SMTC.1 |  |  |  |  |  |
|  |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 | Config 1,2 | Depending on band group | dBm/15kHz | -88 | -88 | -108.5 | -108.5 | -119.5+ ΔBG_offset | -119.5+ ΔBG_offset |
| NocNote2 | Config 1,2 |  | dBm/SCS | -88 | -88 | -108.5 | -108.5 | Same as Noc for 15kHz | Same as Noc for 15kHz |
|  | Config 3 | Depending on band group |  | -85 | -85 | -105.5 | -105.5 | -116.5+ ΔBG_offset | 116.5+ ΔBG_offset |
| EsIot |  |  | dB | -1.75 | -1.75 | 20 | 20 | -3.2 | -3.2 |
| EsNoc |  |  | dB | -1.75 | -1.75 | 20 | 20 | -3.2 | -3.2 |
| SS-RSRPNote3 | Config 1,2 | Depending on band group | dBm/SCS | -89.75 | -89.75 | -88.5 | -88.5 | -122.7+ ΔBG_offset | -122.7+ ΔBG_offset |
|  | Config 3 | Depending on band group |  | -86.75 | -86.75 | -85.5 | -85.5 | -119.7+ ΔBG_offset | -119.7+ ΔBG_offset |
| SS-SINR Note3 |  |  | dB | -1.75 | -1.75 | 20 | 20 | -3.2 | -3.2 |
| IoNote3 | Config 1,2 | Depending on band group | dBm/ 9.36MHz | -57.83 | -57.83 | -60.5 | -60.5 | -89.85+ ΔBG_offset | -89.85+ ΔBG_offset |
|  | Config 3 | Depending on band group | dBm/ 38.16MHz | -51.73 | -51.73 | -54.41 | -54.41 | -83.75+ ΔBG_offset | -83.75+ ΔBG_offset |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: ΔBG_offset is defined in clause 3A.4, Table 3A.4.1-2 Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |

Table 6.7.3.2.1.5-2: SS-SINR Inter frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_35 | SS-SINR_79 | SS-SINR_32 |
| Highest reported value (Cell 2) | SS-SINR_51 | SS-SINR_94 | SS-SINR_49 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_33 | SS-SINR_77 | SS-SINR_31 |
| Highest reported value (Cell 2) | SS-SINR_53 | SS-SINR_96 | SS-SINR_50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.3.2.2	NR SA FR1-FR1 SS-SINR relative measurement accuracy

6.7.3.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency SS-SINR relative measurement accuracy is within the specified limits for all bands.

6.7.3.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards, which support ss-SINR-Meas.

6.7.3.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.3.2.2.

6.7.3.2.2.4	Test description

6.7.3.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.3.2.2.4.1-1.

Table 6.7.3.2.2.4.1-1: NR SA FR1-FR1 SS-SINR measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.3.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.3.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.3.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.3.2.2.4.1-2.

Table 6.7.3.2.2.4.1-2: Initial conditions for SS-SINR inter-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.3.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in clause 6.7.3.2.2.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for SS-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.3.2.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.3.2.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the SS- SINR value of Cell 1 reported by the UE. If the difference between both values is outside the limits in Table 6.7.3.2.2.5-2 or the UE fails to report the measurement value for Cell 2 or Cell 1, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.3.2.2.5-1 as appropriate and repeat steps 5-7.

6.7.3.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.3.2.1.4.3.

6.7.3.2.2.5	Test requirements

Table 6.7.3.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-SINR measurement report for each of the tests in Table 6.7.3.2.2.5-1 shall meet the corresponding relative accuracy requirements in Table 6.7.3.2.2.5-2

Table 6.7.3.2.2.5-1: same as Table 6.7.3.2.1.5-1

Table 6.7.3.2.2.5-2: SS-SINR Inter frequency relative accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x - 10 | SS-SINR_x - 10 | SS-SINR_x - 11 |
| Highest reported value (Cell 2) | SS-SINR_x + 10 | SS-SINR_x + 10 | SS-SINR_x + 11 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x - 11 | SS-SINR_x - 11 | SS-SINR_x - 11 |
| Highest reported value (Cell 2) | SS-SINR_x + 11 | SS-SINR_x + 11 | SS-SINR_x + 11 |
| SS-SINR_x is the reported value of Cell 1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.4	L1-RSRP

#### 6.7.4.0	Minimum conformance requirements

6.7.4.0.1	SSB based absolute L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.1.

6.7.4.0.2	SSB based relative L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.2.

6.7.4.0.3	CSI-RS based absolute L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.3.

6.7.4.0.4	CSI-RS based relative L1-RSRP measurement accuracy requirements

Same as 4.7.4.0.4.

#### 6.7.4.1	SSB based L1-RSRP measurements

##### 6.7.4.1.1	NR SA FR1 SSB based L1-RSRP absolute measurement accuracy

6.7.4.1.1.1	Test purpose

The purpose of this test is to verify that the SSB based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.4.1.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.1.

6.7.4.1.1.4	Test description

6.7.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.1.1.4.1-1.

Table 6.7.4.1.1.4.1-1: NR SA FR1 SSB based L1-RSRP absolute measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.1.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.1.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.1.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.1.1.4.1-2.

Table 6.7.4.1.1.4.1-2: Initial conditions for SSB based L1-RSRP absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.1.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for SSB-based L1-RSRP measurements. The UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.1.1.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.1.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.1.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.1.1.5-1.

3.	The UE shall start sending L1-RSRP report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of SSB#0 or SSB#1 in the periodic L1-RSRP reports. If the value for the strongest SSB is within the limits in Table 6.7.4.1.1.5-2 or Table 6.7.4.1.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.1.1.5-1 as appropriate and repeat steps 3-5.

6.7.4.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.4.1.1.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and SS-RSRP Table H.3.6-3 with conditions SSB and PERIODIC Table H.3.6-10 Table H.3.5-8 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.4.1.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.4.1.1.5	Test requirement

Table 6.7.4.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.4.1.1.5-3 for test configuration 3.

Table 6.7.4.1.1.5-1: L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | ssb-Index-RSRP | ssb-Index-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -96.00 | 117+ ΔBG_offset |
| NocNote2 |  | 1,2 | dBm/SSB SCS | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
| EsIot |  | 1~3 | dB | 10 | -2.2 |
| SSB RSRP Note3 | Depending on band group | 1,2 | dBm/SSB SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
| EsIot |  | 1~3 | dB | 10 | -2.2 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |

Table 6.7.4.1.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 31 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 32 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 33 |
|  |  | Bands NR_FDD_FR1_F | 33 |
|  |  | Bands NR_FDD_FR1_G | 34 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 1) | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 44 |
|  |  | Bands NR_FDD_FR1_B | 45 |
|  |  | Bands NR_TDD_FR1_C | 45 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 46 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 46 |
|  |  | Bands NR_FDD_FR1_F | 47 |
|  |  | Bands NR_FDD_FR1_G | 47 |
|  |  | Bands NR_FDD_FR1_H | 48 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 26 |
|  |  | Bands NR_FDD_FR1_B | 27 |
|  |  | Bands NR_TDD_FR1_C | 27 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 28 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 28 |
|  |  | Bands NR_FDD_FR1_F | 29 |
|  |  | Bands NR_FDD_FR1_G | 29 |
|  |  | Bands NR_FDD_FR1_H | 30 |
| Highest reported value (Cell 1) | 85 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 49 |
|  |  | Bands NR_FDD_FR1_B | 49 |
|  |  | Bands NR_TDD_FR1_C | 50 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 50 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 51 |
|  |  | Bands NR_FDD_FR1_F | 51 |
|  |  | Bands NR_FDD_FR1_G | 52 |
|  |  | Bands NR_FDD_FR1_H | 52 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 6.7.4.1.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configuration 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 1) | 63 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 34 |
|  |  | Bands NR_FDD_FR1_B | 34 |
|  |  | Bands NR_TDD_FR1_C | 35 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 36 |
|  |  | Bands NR_FDD_FR1_F | 36 |
|  |  | Bands NR_FDD_FR1_G | 37 |
|  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 1) | 84 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 47 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 48 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 49 |
|  |  | Bands NR_FDD_FR1_F | 50 |
|  |  | Bands NR_FDD_FR1_G | 50 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 1) | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 29 |
|  |  | Bands NR_FDD_FR1_B | 30 |
|  |  | Bands NR_TDD_FR1_C | 30 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 31 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 31 |
|  |  | Bands NR_FDD_FR1_F | 32 |
|  |  | Bands NR_FDD_FR1_G | 32 |
|  |  | Bands NR_FDD_FR1_H | 33 |
| Highest reported value (Cell 1) | 87 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 52 |
|  |  | Bands NR_FDD_FR1_B | 52 |
|  |  | Bands NR_TDD_FR1_C | 53 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 53 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 54 |
|  |  | Bands NR_FDD_FR1_F | 54 |
|  |  | Bands NR_FDD_FR1_G | 55 |
|  |  | Bands NR_FDD_FR1_H | 55 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.4.1.2	NR SA FR1 SSB based L1-RSRP relative measurement accuracy

6.7.4.1.2.1	Test purpose

The purpose of this test is to verify that the SSB based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.4.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.1.

6.7.4.1.2.4	Test description

6.7.4.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.1.2.4.1-1.

Table 6.7.4.1.2.4.1-1: NR SA FR1 SSB based L1-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.1.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.1.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.1.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.1.2.4.1-2.

Table 6.7.4.1.2.4.1-2: Initial conditions for SSB based L1-RSRP relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.1.2.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for SSB-based L1-RSRP measurements. The UE is configured to perform RLM, BFD and L1-RSRP measurement based on the SSBs. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.1.2.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.1.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.1.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.1.2.5-1.

3.	The UE shall start sending L1-RSRP report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of SSB#0 and SSB#1 in the periodic L1-RSRP reports. The DIFF-RSRP value of SSB#0 or SSB#1 reported by the UE is compared to the expected DIFF-. If the DIFF-RSRP value is within the limits in Table 6.7.4.1.2.5-2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.1.2.5-1 as appropriate and repeat steps 3-5.

6.7.4.1.2.4.3	Message contents

Message contents  are same as in Clause 6.7.4.1.1.4.3.

6.7.4.1.2.5	Test requirement

Table 6.7.4.1.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.1.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.1.2.5-2.

Table 6.7.4.1.2.5-1: Same as Table 6.7.4.1.1.5-1

Table 6.7.4.1.2.5-2: L1-RSRP relative accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | All bands |
| Normal Conditions |  |  |
| Lowest DIFF RSRP reported value | 0 | 0 |
| Highest DIFF RSRP reported value | 2 | 2 |
| Extreme Conditions |  |  |
| Lowest DIFF RSRP reported value | 0 | 0 |
| Highest DIFF RSRP reported value | 2 | 2 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.4.2	CSI-RS based L1-RSRP measurements

##### 6.7.4.2.1	NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy

6.7.4.2.1.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP absolute measurement accuracy is within the specified limits for all bands.

6.7.4.2.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.2.

6.7.4.2.1.4	Test description

6.7.4.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.2.1.4.1-1.

Table 6.7.4.2.1.4.1-1: NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.2.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.2.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.2.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.2.1.4.1-2.

Table 6.7.4.2.1.4.1-2: Initial conditions for CSI-RS based L1-RSRP absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.2.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-RSRP measurements. The UE is configured to perform RLM and BFD measurement based on the SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.2.1.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.2.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.2.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.2.1.5-1.

3.	The UE shall start sending L1-RSRP report including results of both CSI-RS#0 and CSI-RS#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of CSI-RS#0 or CSI-RS#1 in the periodic L1-RSRP reports. If the value for the strongestCSI-RS is within the limits in Table 6.7.4.2.1.5-2 or Table 6.7.4.2.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.2.1.5-1 as appropriate and repeat steps 3-5.

6.7.4.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.4.2.1.4.3-1: Common Exception messages EN-DC CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6-2 with conditions PERIODIC and CSI-RSRP Table H.3.6-3 with conditions CSI-RS and PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.4.2.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSB. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.4.2.1.5	Test requirement

Table 6.7.4.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.2.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.4.2.1.5-3 for test configuration 3.

Table 6.7.4.2.1.5-1: L1-RSRP test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1,4 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2,5 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3,6 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity |  | 1~3 |  | cri-RSRP | cri-RSRP |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-RSRP reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -96.00 | 117+ ΔBG_offset |
| NocNote2 |  | 1,2 | dBm/CSI-RS SCS | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
| EsIot |  | 1~3 | dB | 10 | -2.2 |
| CSI-RSRP Note3 | Depending on band group | 1,2 | dBm/CSI-RS SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
| EsNoc |  | 1~3 | dB | 10 | -2.2 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |

Table 6.7.4.2.1.5-2: Same as Table 6.7.4.1.1.5-2

Table 6.7.4.2.1.5-3: Same as Table 6.7.4.1.1.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.4.2.2	NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy

6.7.4.2.2.1	Test purpose

The purpose of this test is to verify that the CSI-RS based L1-RSRP relative measurement accuracy is within the specified limits for all bands.

6.7.4.2.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

6.7.4.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.4.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.4.2.

6.7.4.2.2.4	Test description

6.7.4.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.4.2.2.4.1-1.

Table 6.7.4.2.2.4.1-1: NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.4.2.2-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD |
| 6.7.4.2.2-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD |
| 6.7.4.2.2-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.4.2.2.4.1-2.

Table 6.7.4.2.2.4.1-2: Initial conditions for CSI-RS based L1-RSRP relative accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.4.2.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.4.2.2.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-RSRP measurements. The UE is configured to perform RLM and BFD measurement based on the SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.4.2.2.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.4.2.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.4.2.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.4.2.2.5-1.

3.	The UE shall start sending L1-RSRP report including results of both CSI-RS#0 and CSI-RS#1 every 80 slots.

4.	The SS shall check the L1-RSRP reported values of CSI-RS#0 and CSI-RS#1 in the periodic L1-RSRP reports. The DIFF-RSRP value of CSI-RS#0 or CSI-RS#1 reported by the UE is compared to the expected DIFF-. If the DIFF-RSRP value is within the limits in Table 6.7.4.2.2.5-2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-RSRP report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.4.2.2.5-1 as appropriate and repeat steps 3-5.

6.7.4.2.2.4.3	Message contents

Message contents are same as in Clause 6.7.4.2.1.4.3.

6.7.4.2.2.5	Test requirement

Table 6.7.4.2.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.4.2.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.4.2.2.5-2.

Table 6.7.4.2.2.5-1: Same as Table 6.7.4.2.1.5-1

Table 6.7.4.2.2.5-2: Same as Table 6.7.4.1.2.5-2

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.5	E-UTRAN RSRP

#### 6.7.5.0	Minimum conformance requirements

6.7.5.0.1	E-UTRAN RSRP absolute accuracy

The measurement period of E-UTRA RSRP in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 of TS 38.133 [6].

The accuracy requirements of E-UTRA RSRP measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRP Accuracy Requirements in clause 9.1.3 of TS 36.133 [23]:

The requirements for absolute accuracy of RSRP in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 6.7.5.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 of TS 36.133 [23] for a corresponding Band.

Table 6.7.5.0.1-1: RSRP Inter frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | E-UTRA operating band groups Note 3 | Minimum Io |  | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 2 | dBm/BWChannel | dBm/BWChannel |
| 4.5 | 9 | -6 dB | FDD_A, TDD_A | -121 | N/A | -70 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | N/A | -70 |
|  |  |  | FDD_C, TDD_C | -120 | N/A | -70 |
|  |  |  | FDD_D | -119.5 | N/A | -70 |
|  |  |  | FDD_E, TDD_E | -119 | N/A | -70 |
|  |  |  | FDD_F | -118.5 | N/A | -70 |
|  |  |  | FDD_G | -118 | N/A | -70 |
|  |  |  | FDD_H | -117.5 | N/A | -70 |
|  |  |  | FDD_N | -114.5 | N/A | -70 |
| 8 | 11 | -6 dB | FDD_A, TDD_A, FDD_B1, FDD_B2, FDD_C, TDD_C, FDD_D, FDD_E, TDD_E, FDD_F, FDD_G, FDD_H, FDD_N | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 3: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. |  |  |  |  |  |  |

The reporting range and mapping specified for RSRP measurements in clause 9.1.4 of TS 36.133 [23] shall apply:

The reporting range of RSRP is defined from -156 dBm to -44 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 6.7.5.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.5.0.1-2: RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRP_-17 | RSRP< -156 | dBm |
| RSRP_-16 | -156 RSRP< -155 | dBm |
| … | … | … |
| RSRP_-03 | -143 RSRP< -142 | dBm |
| RSRP_-02 | -142 RSRP< -141 | dBm |
| RSRP_-01 | -141 RSRP< -140 | dBm |
| RSRP_00 | RSRP -140 | dBm |
| RSRP_01 | -140 RSRP < -139 | dBm |
| RSRP_02 | -139 RSRP < -138 | dBm |
| … | … | … |
| RSRP_95 | -46 RSRP < -45 | dBm |
| RSRP_96 | -45 RSRP < -44 | dBm |
| RSRP_97 | -44 RSRP | dBm |

#### 6.7.5.1	NR SA FR1 – E-UTRAN RSRP absolute measurement accuracy

6.7.5.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RSRP absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.5.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

6.7.5.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.5.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.5.1.

6.7.5.1.4	Test description

6.7.5.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.5.1.4.1-1.

Table 6.7.5.1.4.1-1:  test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.5.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.5.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.5.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.5.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.5.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.5.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.5.1.4.1-2.

Table 6.7.5.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.5.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.5.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.5.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.5.1.5-1 and Table 6.7.5.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RSRP reported values in the periodic MeasurementReport. The RSRP value of Cell 2 reported by the UE is compared to the expected RSRP. If the value is outside the limits in Table 6.7.5.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.5.1.5-2  as appropriate and repeat steps 5-7.

6.7.5.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.5.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-3 Table H.3.1-3a Table H.3.1-5 with Condition INTER-RAT Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.5.1.4.3-1A: MeasConfig (Test procedure step 3)

| Derivation path: Table H.3.1-2 with condition INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT- DEFAULT specified in Table H.3.1-4B with condition RSRP |  |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-EVENT | Table 6.7.5.1.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.5.1.4.3-1B: ReportConfigInterRAT-EVENT (Table 6.7.5.1.4.3-1A)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-141 with condition EVENT_B1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  |  |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| rsrp | 97 | Set threshold to -44dBm to ensure measId 2 will never be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.5.1.4.3-2: Void

6.7.5.1.5	Test requirement

Table 6.7.5.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.5.1.5-1 and 6.7.5.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.5.1.5-3.

Table 6.7.5.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.5.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRP test parameters

| Parameter |  | Unit | Cell 2 |  |
| --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |
| E-UTRA RF channel number |  |  | 1 |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |
|  | Config 4, 5, 6 |  | TDD |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 6 |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |
|  | Config 4, 5, 6 |  | 1 |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |
| PBCH_RA |  | dB | 0 |  |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PCFICH_RB |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| PDCCH_RA |  |  |  |  |
| PDCCH_RB |  |  |  |  |
| PDSCH_RA |  |  |  |  |
| PDSCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
| NocNote4 | Depending on band group | dBm/15kHz | -91.65 | -117 + ΔBG_offset |
| Ês/Noc |  | dB | 10 | -3.2 |
| Ês/IotNote5 |  | dB | 10 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -81.65 | -120.2+ ΔBG_offset |
| SCH_RPNote5 | Depending on band group | dBm/15kHz | -81.65 | -120.2+ ΔBG_offset |
| IoNote5 | Depending on band group | dBm/Ch BW | -53.45 + 10log(NRB,c/50) | -87.52+ ΔBG_offset + 10log(NRB,c/50) |
| Propagation Condition |  |  | AWGN |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, SCH_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: Void Note 8: Void. Note 9: Void Note 10: Void Note 11: ΔBG_offset for LTE band group is defined in TS 36.521-3 [26] clause 3.5.1, Table 3.5.1-1A. |  |  |  |  |

Table 6.7.5.1.5-3: SS-RSRP Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 49 | FDD_A, TDD_A | 14 |
|  |  | FDD_B | 15 |
|  |  | TDD_C | 15 |
|  |  | FDD_D, TDD_D | 16 |
|  |  | FDD_E, TDD_E | 16 |
|  |  | FDD_G | 17 |
|  |  | FDD_H | 18 |
| Highest reported value (Cell 2) | 69 | FDD_A, TDD_A | 27 |
|  |  | FDD_B | 27 |
|  |  | TDD_C | 28 |
|  |  | FDD_D, TDD_D | 28 |
|  |  | FDD_E, TDD_E | 29 |
|  |  | FDD_G | 30 |
|  |  | FDD_H | 30 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 46 | FDD_A, TDD_A | 10 |
|  |  | FDD_B | 10 |
|  |  | TDD_C | 11 |
|  |  | FDD_D, TDD_D | 11 |
|  |  | FDD_E, TDD_E | 12 |
|  |  | FDD_G | 13 |
|  |  | FDD_H | 13 |
| Highest reported value (Cell 2) | 72 | FDD_A, TDD_A | 31 |
|  |  | FDD_B | 32 |
|  |  | TDD_C | 32 |
|  |  | FDD_D, TDD_D | 33 |
|  |  | FDD_E, TDD_E | 33 |
|  |  | FDD_G | 34 |
|  |  | FDD_H | 35 |
| Note 1: E-UTRA operating band groups are as defined in TS 36.521-3 [26] clause 3.5.1 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.6	E-UTRAN RSRQ

#### 6.7.6.0	Minimum conformance requirements

6.7.6.0.1	E-UTRAN RSRQ absolute accuracy

The measurement period of E-UTRA RSRQ in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 on TS 38.133 [6].

The accuracy requirements of E-UTRA RSRQ measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RSRQ Accuracy Requirements in clause 9.1.6 of TS 36.133 [23]:

The requirements for absolute accuracy of RSRQ in this clause apply to a cell that has different carrier frequency from the serving cell.

The accuracy requirements in Table 6.7.6.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.3 of TS 36.133 [23] for a corresponding Band

Table 6.7.6.0.1-1: RSRQ Inter frequency absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 2.5 | 4 | -3 dB | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 3.5 | 4 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. |  |  |  |  |  |

The reporting range and mapping specified for RSRQ measurements in clause 9.1.7 of TS 36.133 [23] shall apply:

The reporting range of RSRQ is defined from -34 dB to 2.5 dB with 0.5 dB resolution.

The mapping of measured quantity is defined in table 6.7.6.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.6.0.1-2: RSRQ measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRQ_-30 | RSRQ -34 | dB |
| RSRQ_-29 | -34 RSRQ < -33.5 | dB |
| … | … | … |
| RSRQ_-02 | -20.5 RSRQ < -20 | dB |
| RSRQ_-01 | -20 RSRQ < -19.5 | dB |
| RSRQ_00 | RSRQ -19.5 | dB |
| RSRQ_01 | -19.5 RSRQ < -19 | dB |
| RSRQ_02 | -19 RSRQ < -18.5 | dB |
| … | … | … |
| RSRQ_32 | -4 RSRQ < -3.5 | dB |
| RSRQ_33 | -3.5 RSRQ < -3 | dB |
| RSRQ_34 | -3 RSRQ | dB |
| RSRQ_35 | -3 RSRQ < -2.5 | dB |
| RSRQ_36 | -2.5 RSRQ < -2 | dB |
| … | … | … |
| RSRQ_45 | 2 RSRQ < 2.5 | dB |
| RSRQ_46 | 2.5 RSRQ | dB |

Note: The ranges from RSRQ_-30 to RSRQ_-01 and from RSRQ_35 to RSRQ_46 apply for the UE who can support extended RSRQ range.

#### 6.7.6.1	NR SA FR1 – E-UTRAN RSRQ absolute measurement accuracy

6.7.6.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RSRQ absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.6.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA.

6.7.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.6.1.

6.7.6.1.4	Test description

6.7.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.6.1.4.1-1.

Table 6.7.6.1.4.1-1: test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.6.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.6.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.6.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.6.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.6.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.6.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.6.1.4.1-2.

Table 6.7.6.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.6.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.6.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.6.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.6.1.5-1 and Table 6.7.6.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RSRQ reported values in the periodic MeasurementReport. The RSRQ value of Cell 2 reported by the UE is compared to the expected RSRQ. If the value is outside the limits in Table 6.7.6.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.6.1.5-2  as appropriate and repeat steps 5-7.

6.7.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.6.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-3 Table H.3.1-3a Table H.3.1-5 with Condition INTER-RAT Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.6.1.4.3-1A: MeasConfig (Test procedure step 3)

| Derivation path: Table H.3.1-2 with condition INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT- DEFAULT specified in Table H.3.1-4B with condition RSRQ |  |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-EVENT | Table 6.7.6.1.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.6.1.4.3-1B: ReportConfigInterRAT-EVENT (Table 6.7.6.1.4.3-1A)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-141 with condition EVENT_B1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  |  |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| rsrq | 34 | Set threshold to -3dB to ensure measId 2 will never be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.6.1.4.3-2: Void

6.7.6.1.5	Test requirement

Table 6.7.6.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.6.1.5-1 and 6.7.6.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.6.1.5-3.

Table 6.7.6.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.6.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RSRQ test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| NocNote4 | Depending on band group | dBm/15kHz | -83 | -104.70 | -119.5+ ΔBG_offset |
| Ês/Noc |  | dB | -1.75 | -3.2 | -3.2 |
| Ês/IotNote5 |  | dB | -1.75 | -3.2 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -84.75 | -107.90 | -122.7+ ΔBG_offset |
| RSRQNote5 | Depending on band group | dB | -14.76 | -15.69 | -15.69 |
| IoNote5 | Depending on band group | dBm/Ch BW | -53 + 10log(NRB,c /50) | -75.22 + 10log(NRB,c /50) | -90.02+ ΔBG_offset + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 5: Ês/Iot, RSRP, RSRQ and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: Void Note 8: Void Note 9: Void Note 10: Void Note 11: ΔBG_offset for LTE band group is defined in TS 36.521-3 [26] clause 3.5.1, Table 3.5.1-1A. |  |  |  |  |  |

Table 6.7.6.1.5-3: SS-RSRQ Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 4 | 0 | 0 |
| Highest reported value (Cell 2) | 16 | 16 | 16 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest reported value (Cell 2) | 1 | 0 | 0 |
| Highest reported value (Cell 2) | 19 | 17 | 17 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.7	E-UTRAN RS-SINR

#### 6.7.7.0	Minimum conformance requirements

6.7.7.0.1	E-UTRAN RS-SINR absolute accuracy

The measurement period of E-UTRA RS-SINR in RRC_CONNECTED state is specified in clause 9.4.2 and 9.4.3 of TS 38.133 [6].

The accuracy requirements of E-UTRA RS-SINR measurements in RRC_CONNECTED state and the corresponding side conditions shall be the same as the inter-frequency RS-SINR Accuracy Requirements in clause 9.1.17.3 of TS 36.133 [23]:

The reporting range and mapping for E-UTRA RS-SINR measurements shall be the same as specified for RS-SINR measurements in clause 9.1.17.1 of TS 36.133 [23]:

The requirements for absolute accuracy of intra-frequency RS-SINR in this clause apply to a cell on the same frequency as that of the serving cell.

The accuracy requirements in Table 6.7.7.0.1-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in 36.101 [27] Clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex B.3.18 of TS 36.133 [23] for a corresponding Band.

Table 6.7.7.0.1-1: Intra-frequency RS-SINR absolute accuracy

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 3.0 | 4 | -3 dB Note 5 | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 3.5 | 4 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3 of TS 36.133 [23]. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5 of TS 36.133 [23]. NOTE 5: The requirements apply for Ês/Iot ≤ 25 dB. |  |  |  |  |  |

The reporting range of RS-SINR measurement is defined from -23 dB to 40 dB with 0.5 dB resolution.

The mapping of the measured quantity is defined in table 6.7.7.0.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.7.0.1-2: RS-SINR measurement report mapping

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RS-SINR_000 | RS-SINR -23 | dB |
| RS-SINR_001 | -23 RS-SINR < -22.5 | dB |
| … | … | … |
| RS-SINR_126 | 39.5 RS-SINR < 40 | dB |
| RS-SINR_127 | 40 RS-SINR | dB |

#### 6.7.7.1	NR SA FR1 – E-UTRAN RS-SINR absolute measurement accuracy

6.7.7.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT E-UTRAN RS-SINR absolute measurement accuracy is within the specified limits for all bands, when the serving cell is NR FR1 and the target cell is E-UTRA.

6.7.7.1.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards supporting E-UTRA and rs-SINR-MeasEUTRA.

6.7.7.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.7.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.7.1.

6.7.7.1.1.4	Test description

6.7.7.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.7.1.4.1-1.

Table 6.7.7.1.4.1-1: test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.7.1-1 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: FDD |
| 6.7.7.1-2 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.7.1-3 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: FDD |
| 6.7.7.1-4 | NR: 15 kHz SSB SCS, 10MHz bandwidth, FDD, E-UTRAN: TDD |
| 6.7.7.1-5 | NR: 15 kHz SSB SCS, 10MHz bandwidth, TDD, E-UTRAN: TDD |
| 6.7.7.1-6 | NR: 30 kHz SSB SCS, 40MHz bandwidth, TDD, E-UTRAN: TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.7.1.4.1-2.

Table 6.7.7.1.4.1-2: initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-2 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.7.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.7.1.4.3.

2. There are two carriers and two cells specified in the test, where NR Cell 1 is the NR PCell on the NR carrier and Cell 2 is the E-UTRA neighbour cell on the E-UTRA carrier and the target for the measurements.

6.7.7.1.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to Table 6.7.7.1.5-1 and Table 6.7.7.1.5-2 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the RS-SINR reported values in the periodic MeasurementReport. The RS-SINR value of Cell 2 reported by the UE is compared to the expected RS-SINR. If the value is outside the limits in Table 6.7.7.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.7.1.5-2 as appropriate and repeat steps 5-7.

6.7.7.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.7.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-3 Table H.3.1-3a Table H.3.1-5 with Condition INTER-RAT Table H.3.1-7 with condition INTER-RAT Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.7.1.4.3-1A: MeasConfig (Test procedure step 3)

| Derivation path: Table H.3.1-2 with condition INTER-RAT and GAP NEEDED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 2 entries |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT- DEFAULT specified in Table H.3.1-4B with condition RS-SINR |  |  |
| } |  |  |  |
| } |  |  |  |
| ReportConfigToAddMod[2] SEQUENCE { |  | entry 2 |  |
| reportConfigId | 2 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigInterRAT | ReportConfigInterRAT-EVENT | Table 6.7.7.1.4.3-1B |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 2 entries |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| MeasIdToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measId | 2 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.7.1.4.3-1B: ReportConfigInterRAT-EVENT (Table 6.7.7.1.4.3-1A)

| Derivation Path: TS 38.508-1 [14] Table 4.6.3-141 with condition EVENT_B1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB1 SEQUENCE { |  |  |  |
| b1-ThresholdEUTRA CHOICE { |  |  |  |
| sinr | 127 | Set threshold to 40dB to ensure measId 2 will never be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.7.1.4.3-2: Void

6.7.7.1.5	Test requirement

Table 6.7.7.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Tables 6.7.7.1.5-1 and 6.7.7.1.5-2 shall meet the corresponding absolute accuracy requirements in Table 6.7.7.1.5-3.

Table 6.7.7.1.5-1: NR Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 1 |
| --- | --- | --- | --- |
| NR RF channel number |  |  | 1 |
| Duplex mode | Config 1, 4 |  | FDD |
|  | Config 2, 3, 5, 6 |  | TDD |
| TDD Configuration | Config 1, 4 |  | N/A |
|  | Config 2, 5 |  | TDDConf.1.1 |
|  | Config 3, 6 |  | TDDConf.2.1 |
| BWchannel | Config 1, 4 | MHz | 10: NRB,c = 52 (FDD) |
|  | Config 2, 5 |  | 10: NRB,c = 52 (TDD) |
|  | Config 3, 6 |  | 40: NRB,c = 106 (TDD) |
| Gap pattern Id |  |  | 0 |
| PDSCH reference measurement channel | Config 1, 4 |  | SR.1.1 FDD |
|  | Config 2, 5 |  | SR.1.1 TDD |
|  | Config 3, 6 |  | SR.2.1 TDD |
| RMSI CORSET reference channel | Config 1, 4 |  | CR.1.1 FDD |
|  | Config 2, 5 |  | CR.1.1 TDD |
|  | Config 3, 6 |  | CR.2.1 TDD |
| Dedicated CORSET reference channel | Config 1, 4 |  | CCR.1.1 FDD |
|  | Config 2, 5 |  | CCR.1.1 TDD |
|  | Config 3, 6 |  | CCR.2.1 TDD |
| CSI-RS for tracking | Config 1, 4 |  | TRS.1.1 FDD |
|  | Config 2, 5 |  | TRS.1.1 TDD |
|  | Config 3, 6 |  | TRS.1.2 TDD |
| BWP configurations | Initial DL BWP |  | DLBWP.0.1 |
|  | Dedicated DL BWP |  | DLBWP.1.1 |
|  | Initial UL BWP |  | ULBWP.0.1 |
|  | Dedicated UL BWP |  | ULBWP.1.1 |
| OCNG patternNote1 |  |  | OP.1 |
| SMTC configuration |  |  | SMTC.1 |
| SSB configuration | Config 1, 2, 4, 5 |  | SSB.1 FR1 |
|  | Config 3, 6 |  | SSB.2 FR1 |
| EPRE ratio of PSS to SSS |  | dB | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |
| NocNote2 |  | dBm/15 kHz | -104 |
| NocNote2 | Config 1, 2, 4, 5 | dBm/SCS | -104 |
|  | Config 3, 6 |  | -101 |
| Ês/Noc |  | dB | 17 |
| Ês/IotNote3 |  | dB | 17 |
| SS-RSRPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| SSB_RPNote3 | Config 1, 2, 4, 5 | dBm/SCS | -87 |
|  | Config 3, 6 |  | -84 |
| IoNote3 | Config 1, 2, 4, 5 | dBm/9.36 MHz | -58.96 |
|  | Config 3, 6 | dBm/38.16 MHz | -52.87 |
| Propagation condition |  |  | AWGN |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: Ês/Iot, SS-RSRP, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table 6.7.7.1.5-2: E-UTRAN Cell specific test parameters for SA Inter-RAT E-UTRAN RS-SINR test parameters

| Parameter |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF channel number |  |  | 1 |  |  |
| Duplex mode | Config 1, 2, 3 |  | FDD |  |  |
|  | Config 4, 5, 6 |  | TDD |  |  |
| TDD special subframe configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 6 |  |  |
| TDD uplink-downlink configurationNote1 | Config 1, 2, 3 |  | N/A |  |  |
|  | Config 4, 5, 6 |  | 1 |  |  |
| BWchannel |  | MHz | 5 MHz: NRB,c = 25 10 MHz: NRB,c = 50 20 MHz: NRB,c = 100 |  |  |
| PDSCH parameters: DL Reference Measurement ChannelNote2 |  |  | - |  |  |
| PCFICH/PDCCH/PHICH parameters: DL Reference Measurement ChannelNote2 | Config 1, 2, 3 |  | 5 MHz: R.11 FDD 10 MHz: R.6 FDD 20 MHz: R.10 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: R.11 TDD 10 MHz: R.6 TDD 20 MHz: R.10 TDD |  |  |
| OCNG PatternsNote2 | Config 1, 2, 3 |  | 5 MHz: OP.19 FDD 10 MHz: OP.6 FDD 20 MHz: OP.14 FDD |  |  |
|  | Config 4, 5, 6 |  | 5 MHz: OP.10 TDD 10 MHz: OP.2 TDD 20 MHz: OP.8 TDD |  |  |
| PBCH_RA |  | dB | 0 |  |  |
| PBCH_RB |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |  |
| Noc1Note4 | Depending on band group | dBm/15kHz | -88 | -108.50 | -119.5+ ΔBG_offset |
| CRS Ês/Noc1 |  | dB | -1.75 | 20.0 | -3.2 |
| CRS Ês/IotNote5 |  | dB | -1.75 | 20.0 | -3.2 |
| RSRPNote5 | Depending on band group | dBm/15kHz | -89.75 | -88.50 | -122.7+ ΔBG_offset |
| RS-SINRNote5 |  | dB | -1.75 | 20 | -3.2 |
| IoNote5 | Depending on band group | dBm/Ch BW | -58.00 + 10log(NRB,c /50) | -60.68 + 10log(NRB,c /50) | -90.02+ ΔBG_offset + 10log(NRB,c /50) |
| Propagation Condition |  |  | AWGN |  |  |
| Antenna Configuration and Correlation Matrix |  |  | 1x2 |  |  |
| Note 1: Special subframe and uplink-downlink configurations are specified in table 4.2-1 in TS 36.211 [24]. Note 2: DL RMCs and OCNG patterns are specified in clauses A 3.1 and A 3.2 of TS 36.133 [23] respectively. Note 3: OCNG shall be used such that all cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Interference from other cells and noise sources not specified in the test is assumed to be constant over CRS subcarriers and time and shall be modelled as AWGN of appropriate power for Noc1 to be fulfilled. Note 4a: Void Note 5: CRS Ês/Iot, RSRP, RS-SINR and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 6: E-UTRA operating band groups are as defined in clause 3.5 of TS 36.133 [23]. Note 7: Void Note 8: Void Note 9: Void Note 10: Void Note 11: ΔBG_offset for LTE band group is defined in TS 36.521-3 [26] clause 3.5.1, Table 3.5.1-1A. |  |  |  |  |  |

Table 6.7.7.1.5-3: RS-SINR Intra frequency absolute accuracy requirements for the reported values

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 35 | 79 | 32 |
| Highest reported value (Cell 2) | 51 | 94 | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 All bands | Test 3 All bands |
| Lowest reported value (Cell 2) | 33 | 77 | 31 |
| Highest reported value (Cell 2) | 53 | 96 | 50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.8	CLI Measurements

#### 6.7.8.0	Minimum conformance requirements

6.7.8.0.1	Minimum conformance requirements for SRS-RSRP accuracy

Same as in clause 4.7.6.0.1.

6.7.8.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell

Same as in clause 4.7.6.0.2

#### 6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy

6.7.8.1.1	Test purpose

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits. This test will verify the SRS-RSRP measurement requirements in clause 6.7.8.0.

6.7.8.1.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR and CLI-based SRS-RSRP measurements.

6.7.8.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.8.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.8.1.

6.7.8.1.4	Test description

In this set of test cases there is one cell in the test, FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table 6.7.8.1.5-1 below. The test parameter for the (virtual) neighbour cell UE transmitting SRS are given in Table 6.7.8.1.5-2. The SRS resource configuration is given in Table 6.7.8.1.5-3.

6.7.8.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.8.1.4.1-1.

Table 6.7.8.1.4.1-1: Supported test configurations for NR SA FR1 SRS-RSRP measurement accuracy

| Config | Description |
| --- | --- |
| 6.7.8.1-1 | 15kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.7.8.1-2 | 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.8.1.4.1-2.

Table 6.7.8.1.4.1-2: Initial conditions for NR SA FR1 SRS-RSRP measurement accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.8.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 6.7.8.1.4.3.

2.	Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3. Virtual UE 1 is the target for SRS-RSRP measurements.

3.	The test parameters are given in Table 6.7.8.1.4.1-3

6.7.8.1.4.2	Test procedure

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS resource configuration in Table 6.7.8.1.5-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to the corresponding Test in Table 6.7.8.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message configuring a CLI measurement object with periodic reporting, as specified in section 6.7.8.1.4.3.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 5, the SS shall check the SRS-RSRP reported values in the periodic MeasurementReport. The reported SRS-RSRP value of the neighbour virtual UE is compared to the expected SRS-RSRP as specified in Table 6.7.8.1.5-4 and Table 6.7.8.1.5-5 for Test 1 and Test 2, correspondingly. If the reported value is outside the limits specified in such tables, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each Test in Table 6.7.8.1.5-1 as appropriate and repeat steps 2-7.

6.7.8.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 6.7.8.1.4.3-1: Common Exception messages for NR SA FR1 SRS-RSRP measurement accuracy

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 |

Table 6.7.8.1.4.3-2: MeasObjectToAddModList for NR SA FR1 SRS-RSRP measurement accuracy

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-77 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectCLI-r16 | MeasObjectCLI-r16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.8.1.4.3-3: MeasObjectCLI-r16 for NR SA FR1 SRS-RSRP measurement accuracy

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCLI-r16 ::= SEQUENCE { |  |  |  |
| cli-ResourceConfig-r16 SEQUENCE { |  |  |  |
| srs-ResourceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| SRS-ResourceListConfigCLI-r16 SEQUENCE { | 1 entry |  |  |
| srs-Resource-r16 | SRSConf.1 | entry 1 | Config 1 |
|  | SRSConf.2 | entry 1 | Config 2 |
| srs-SCS-r16 | kHz15 |  | Config 1 |
|  | kHz30 |  | Config 2 |
| refServCellIndex-r16 | 0 | PCell |  |
| refBWP-r16 | 0 | BWP-0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.8.1.4.3-4: ReportConfigNR for NR SA FR1 SRS-RSRP measurement accuracy

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| cli-Periodical-r16 SEQUENCE { |  |  |  |
| reportInterval-r16 | ms240 |  |  |
| reportAmount-r16 | infinity |  |  |
| reportQuantityCLI-r16 | srs-rsrp |  |  |
| maxReportCLI-r16 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.8.1.4.3-5: MeasResultCLI-r16 for NR SA FR1 SRS-RSRP measurement accuracy

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultCLI-r16 ::= SEQUENCE { |  |  |  |
| measResultsListSRS-r16 CHOICE { |  |  |  |
| srs-ResourceId-r16 | SRS-ResourceId |  |  |
| srs-RSRP-Result-r16 | SRS-RSRP-Range-r16 | INTEGER (0..98) |  |
| } |  |  |  |
| } |  |  |  |

6.7.8.1.5	Test requirement

Table 6.7.8.1.5-1 and Table 6.7.8.1.5-2 define the primary level settings including test tolerances for NR SA FR1 SRS-RSRP measurement accuracy. Table 6.7.8.1.5-3 defines the SRS resource configurations. Table 6.7.8.1.5-4 and Table 6.7.8.1.5-5 define the absolute accuracy requirements for Tests 1, 2 and 3 for configurations 1 and 2, correspondingly.

Table 6.7.8.1.5-1: NR Cell specific test parameters for NR SA FR1 SRS-RSRP measurement accuracy

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~2 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~2 |  | TDD | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 2 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 2 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~2 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~2 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~2 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~2 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| NocNote2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| NocNote2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 3 | 2 |  | Not applicableNote 4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 4: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table 6.7.8.1.5-2: Neighbor UE specific test parameters for NR SA FR1 SRS-RSRP measurement accuracy

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| NocNote2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| NocNote2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| EsIot on SRS | NR_TDD_FR1_A NOTE 5 | 1 | dB | 1.5 | 1.5 | 1.75 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | 1.5 | 2.25 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -104.5 | -86.5 | -112.25 |
|  | NR_TDD_FR1_C |  |  |  |  | -111.5 |
|  | NR_TDD_FR1_D |  |  |  |  | -111 |
|  | NR_TDD_FR1_E |  |  |  |  | -110.5 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | -86.49 | -108.74 |
|  | NR_TDD_FR1_C |  |  |  |  | -108.49 |
|  | NR_TDD_FR1_D |  |  |  |  | -107.99 |
|  | NR_TDD_FR1_E |  |  |  |  | -107.49 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/9.36 MHz | -74.42 | -56.42 | -82.28 |
|  | NR_TDD_FR1_C |  |  |  |  | -81.42 |
|  | NR_TDD_FR1_D |  |  |  |  | -80.92 |
|  | NR_TDD_FR1_E |  |  |  |  | -80.42 |
|  | NR_TDD_FR1_A NOTE 5 | 2 | dBm/38.16 MHz | Not applicableNote 6 | -54.80 | -77.49 |
|  | NR_TDD_FR1_C |  |  |  |  | -76.80 |
|  | NR_TDD_FR1_D |  |  |  |  | -76.30 |
|  | NR_TDD_FR1_E |  |  |  |  | -75.80 |
| EsNoc on SRS | NR_TDD_FR1_A NOTE 5 | 1 | dB | 1.5 | 1.5 | 1.75 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | 1.5 | 2.25 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
| Propagation condition |  | 1~2 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1~2 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: Test 1 is not used when testing with 30kHz SSB SCS Note 7: For Test 3, values are specified based on NR_TDD_FR1_X but they apply also to the FDD band group counterpart, NR_FDD_FR1_X. Informatively, test parameters are defined based on band group A and then add ΔBG_offset , which is the band group dependent component defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |

Table 6.7.8.1.5-3: SRS configurations for NR SA FR1 SRS-RSRP measurement accuracy

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |
|  | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS-Resource | SRS-ResourceId | 0 | 0 |
|  | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

Table 6.7.8.1.5-4: Absolute accuracy requirements for the reported values for test configuration 1 of NR SA FR1 SRS-RSRP measurement accuracy

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SRS-RSRP reported value | 30 | 45 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 23 |
|  |  |  | Bands NR_TDD_FR1_C | 23 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 24 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 24 |
| Highest SRS-RSRP reported value | 41 | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  |  | Bands NR_TDD_FR1_C | 34 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |  |

Table 6.7.8.1.5-5: Absolute accuracy requirements for the reported values for test configuration 2 of NR SA FR1 SRS-RSRP measurement accuracy

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SRS-RSRP reported value | N/A Note 2 | 44 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 26 |
|  |  |  | Bands NR_TDD_FR1_C | 26 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 26 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 27 |
| Highest SRS-RSRP reported value | N/A Note 2 | 63 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 37 |
|  |  |  | Bands NR_TDD_FR1_C | 37 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 38 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 38 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 Note 2: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.9	L1-SINR measurement for beam reporting

#### 6.7.9.0	Minimum conformance requirements

6.7.9.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off

The UE shall be capable of performing L1-SINR measurements with the CSI-RS configured as CMR and no dedicated resource configured as IMR for L1-SINR computation, and the UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only is defined in Table 6.7.9.0.1-1 for FR1 and in Table 6.7.9.0.1-2 for FR2, where

For the value of M,

-	For periodic and semi-persistent CSI-RS resources as CMR, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise;

-	For aperiodic CSI-RS resources as CMR, M=1.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 of TS 38.133 [6] if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured for all resources in the resource set.

For the value of P in FR1,

-	P=11-TCSI-RSMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For the value of P in FR2,

-	P=1, when CSI-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P=3, when CSI-RS is not overlapped with measurement gap and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P= 11-TCSI-RSMRGP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P=31-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,MGRP), when CSI-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P=31-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

Where:

TSMTCperiod = the configured SMTC1 period or SMTC2 period if configured.

TCSI-RS = the periodicity of CSI-RS configured for L1-SINR measurement

If the high layer in TS 38.331 [13] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note: The overlap between CSI-RS for L1-SINR measurement and SMTC means that CSI-RS for L1-SINR measurement is within the SMTC window duration.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 6.7.9.0.1-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

Table 6.7.9.0.1-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

The accuracy requirements in Table 6.7.9.0.1-3 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.1 for a corresponding Band for each relevant CSI-RS based CMR.

-	The bandwidth of CSI-RS as CMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 6.7.9.0.1-3.

Table 6.7.9.0.1-3: L1-SINR absolute accuracy for CSI-RS based CMR only in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±5.5 | ±6.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.1 and 10.1.27.1.

6.7.9.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR

The UE shall be capable of performing L1-SINR measurements with the SSB configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to SSB configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_SSB_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as SSB configured as CMR.

The value of TL1-SINR_Measurement_Period_SSB_CMR_IMR is defined in Table 6.7.9.0.2-1 for FR1 and in Table 6.7.9.0.2-2 for FR2, where

For the value of M

-	For periodic or semi-persistent NZP CSI-RS or CSI-IM resource as dedicated IMR, M=1 if the higher layer parameters timeRestrictionForChannelMeasurements and/or timeRestrictionForInterferenceMeasurements are configured, and M=3 otherwise;

For the value of N in FR2

-	N = 8.

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	the value of PCMR shall be derived in the same way as the value of P used for SSB based L1-RSRP measurement in clause 9.5.4.1 of TS 38.133 [6], in which the occasions and period of the SSB for CMR shall be used instead.

-	the value of PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2 of TS 38.133 [6], in which the occasions and period of the NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For L1-SINR measurement with SSB as CMR and CSI-RS or CSI-IM as IMR, the requirement shall apply if the CSI-RS is configured as IMR with repetition field as “repetition = OFF” or CSI-IM is configured as IMR.

For L1-SINR measurement with SSB as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if SSB occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap

Table 6.7.9.0.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

Table 6.7.9.0.2-2: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

The accuracy requirements in Tables 6.7.9.0.2-3 and 6.7.9.0.2-4 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 6.7.9.0.2-3 and 6.7.9.0.2-4.

Table 6.7.9.0.2-3: L1-SINR absolute accuracy for SSB based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | NZP-IMR Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±4.0 | ±5.0 | 0 | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

Table 6.7.9.0.2-4: L1-SINR absolute accuracy for SSB based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.2 and 10.1.27.2.

6.7.9.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR

The UE shall be capable of performing L1-SINR measurements with the CSI-RS resource configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to CSI-RS resource configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as CSI-RS resource configured as CMR.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR is defined in Table 6.7.9.0.3-1 for FR1 and in Table 6.7.9.0.3-2 for FR2, where

For the value of M,

-	M=1 shall be applied if

-	aperiodic NZP-CSI-RS as CMR or dedicated IMR, or

-	aperiodic CSI-IMR as dedicated IMR, or

-	periodic and semi-persistent NZP-CSI-RS as CMR or dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured, or

-	periodic and semi-persistent CSI-IM as dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured;

-	M=3 otherwise.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 of TS 38.133 [6] if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured for all resources in the resource set.

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	The value of PCMR and PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2 of TS 38.133 [6], in which the occasions and period of the CSI-RS for CMR and NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead respectively.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS as IMR, the requirement shall apply only if CSI-RS resources as CMR and IMR are configured with the same repetition field and the number of CSI-RS resources in the resource sets for CMR and IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-IM as IMR, the requirement shall apply only if the number of CSI-RS resources in the resource set for CMR and the number of CSI-IM resources in the resource set for IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if CSI-RS occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap.

Table 6.7.9.0.3-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

Table 6.7.9.0.3-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

The accuracy requirements in Tables 6.7.9.0.3-3 and 6.7.9.0.3-4 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.3 for a corresponding Band for each relevant CSI-RS based CMR and IMR.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 6.7.9.0.3-3 and 6.7.9.0.3-4.

Table 6.7.9.0.3-3: L1-SINR absolute accuracy for CSI-RS based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | NZP-IMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.0 | ±5.0 | 0 | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 6.7.9.0.3-4: L1-SINR absolute accuracy for CSI-RS based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.3 and 10.1.27.3.

#### 6.7.9.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement

##### 6.7.9.1.1	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR absolute measurement accuracy

6.7.9.1.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

6.7.9.1.1.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.9.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.1.

6.7.9.1.1.4	Test description

6.7.9.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.1.1.4.1-1.

Table 6.7.9.1.1.4.1-1: Applicable NR configurations for FR1 L1-SINR test with CSI-RS based CMR and no dedicated IMR configured

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.1.1.4.1-2.

Table 6.7.9.1.1.4.1-2: Initial conditions for CSI-RS based L1-SINR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.9.1.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-SINR measurements. The UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.9.1.1.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.1.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.1.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.1.1.5-1.

3.	The UE shall start sending L1-SINR report including results of both CSI-RS#0 and CSI-RS#1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS#0 and CSI-RS#1 in the periodic L1-RSRP reports. If the value for both CSI-RSs is within the limits in Table 6.7.9.1.1.5-2 or Table 6.7.9.1.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.9.1.1.5-1 as appropriate and repeat steps 3-5.

6.7.9.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.9.1.1.4.3-1: Common Exception messages NR SA CSI-RS-based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and CSI-SINR Table H.3.6A-2 with conditions CSI-RS and PERIODIC Table H.3.6A-3 with condition PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.9.1.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSB. |  |
| } |  |  |  |
| } |  |  |  |

6.7.9.1.1.5	Test requirement

Table 6.7.9.1.1.5-1 defines the primary level settings excluding test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.9.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.9.1.1.5-3 for test configuration 3.

Table 6.7.9.1.1.5-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -96.00 | -117+ ΔBG_offset |
| NocNote2 |  | 1,2 | dBm/SSB SCS | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
| EsIot |  | 1~3 | dB | 10 | -2.2 |
| SSB RSRP Note3 | Depending on band group | 1,2 | dBm/SSB SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
| EsNoc |  | 1~3 | dB | 10 | -2.2 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table 6.7.9.1.1.5-2: Same as Table Table 4.7.7.1.1.5-2

Table 6.7.9.1.1.5-3: Same as Table Table 4.7.7.1.1.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.9.1.2	NR SA FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR relative measurement accuracy

6.7.9.1.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR relative measurement accuracy is within the specified limits.

6.7.9.1.2.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.7.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.7.1.

6.7.9.1.2.4	Test description

6.7.9.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.1.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.7.9.1.2.4.1-2.

Table 6.7.9.1.2.4.1-1: Applicable NR configurations for FR1 L1-SINR test with CSI-RS based CMR and no dedicated IMR configured

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 6.7.9.1.2.4.1-2: Initial conditions for CSI-RS based L1-SINR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.7.9.1.2.4.3.

2.	Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based L1-SINR measurements. The UE is configured one CSI-RS resource set with two CSI-RS resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. The connection setup is done according to the settings in Annex C.1.1.

6.7.9.1.2.4.2	Test procedure

The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.1.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.1.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.1.1.5-1.

3.	The UE shall start sending L1-SINR report including results of both CSI-RS#0 and CSI-RS #1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS #0 and CSI-RS #1 in the periodic L1-SINR reports. The L1-SINR value for CSI-RS #1 is compared to the L1-SINR value for CSI-RS #0. If the difference is within the limits in Table 6.7.9.1.2.5-2 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.9.1.1.5-1 as appropriate and repeat steps 3-5.

6.7.9.1.2.4.3	Message contents

Same message contents as described in section 6.7.9.1.1.4.3

6.7.9.1.2.5	Test requirement

Table 6.7.9.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.1.2.5-1 shall meet the corresponding relative accuracy requirements in Table 6.7.9.1.2.5-2.

Table 6.7.9.1.2.5-1: Same as Table 4.7.7.1.2.5-1

Table 6.7.9.1.2.5-2: Same as Table 4.7.7.1.2.5-2

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.9.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR absolute measurement accuracy

6.7.9.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

6.7.9.2.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support for L1-SINR measurements on the NR PSCell.

6.7.9.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.9.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.2.

6.7.9.2.4	Test description

6.7.9.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.2.4.1-1.

Table 6.7.9.2.4.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with SSB based CMR and CSI-RS based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.2.4.1-2.

Table 6.7.9.2.4.1-2: Initial conditions for SSB based and CSI-RS based L1-SINR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 6.7.9.2.4.3.

2.	Cell 1 is the NR FR1 cell. Cell 1 is the target for SSB-based L1-RSRP measurements. The UE is configured one SSB resource set with two SSB resources and one CSI-RS resource set with two CSI-RS resource. UE is configured to perform RLM and BFD measurement based on the SSB resources 0 and 1. UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-RS resources as IMR. The connection setup is done according to the settings in Annex C.1.2 and C.1.3.

6.7.9.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.2.5-1. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

3.	The UE shall start sending L1-SINR report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 in the periodic L1-RSRP reports. If the value for both SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 is within the limits in Table 6.7.9.2.5-2 and Table 6.7.9.2.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.	Set the parameters according to each sub-test in Table 6.7.9.2.5-1 as appropriate and repeat steps 3-5.

6.7.9.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.4.1.1.4.3-1: Common Exception messages NR SA SSB based L1-RSRP measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and SS-SINR and CSI-RS_IMR Table H.3.6A-2 with conditions SSB and PERIODIC Table H.3.6A-3 with condition PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.4.1.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.9.2.5	Test requirement

Table 6.7.9.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-RSRP measurement report for each of the tests in Table 6.7.9.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.9.2.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.9.2.5-3 for test configuration 3.

Table 6.7.9.2.5-1: FR1 SSB based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 2 |  | SSB.3 FR1 | SSB.3 FR1 |
|  |  | 3 |  | SSB.4 FR1 | SSB.4 FR1 |
| CSI-RS configuration |  | 1 |  | CSI-RS 1.1A FDD | CSI-RS 1.1A FDD |
|  |  | 2 |  | CSI-RS 1.1A TDD | CSI-RS 1.1A TDD |
|  |  | 3 |  | CSI-RS 2.1A TDD | CSI-RS 2.1A TDD |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | ssb-Index-SINR-r16 | ssb-Index-SINR-r16 |
| Number of reported RS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -96.00 | -117+ ΔBG_offset |
| NocNote2 |  | 1,2 | dBm/SSB SCS | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
| EsIot |  | 1~3 | dB | 10 | -2.2 |
| SSB RSRP Note3 | Depending on band group | 1,2 | dBm/SSB SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table 6.7.9.2.5-2: Same as Table 4.7.7.2.5-2

Table 6.7.9.2.5-3: Same as Table 4.7.7.2.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.9.3	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy

##### 6.7.9.3.1	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR absolute measurement accuracy

6.7.9.3.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

6.7.9.3.1.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.9.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.3.

6.7.9.3.1.4	Test description

6.7.9.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.3.1.4.1-1.

Table 6.7.9.3.1.4.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.3.1.4.1-2.

Table 6.7.9.3.1.4.1-2: Initial conditions for CSI-RS based CMR and CSI-IM based IMR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.3.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.9.3.1.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based CMR and CSI-IM based IMR measurements. The UE is configured one CSI-RS resource set with two CSI-RS resources and one CSI-IM resource set with two CSI-IM resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. UE is configured to perform L1-SINR measurement based on the configured CSI-RS as CMR and CSI-IM as IMR. The connection setup is done according to the settings in Annex C.1.1.

6.7.9.3.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.3.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.3.1.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.3.1.5-1. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

3.	The UE shall start sending L1-SINR report including results of both SSB#0 and SSB#1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS#0+CSI-IM#0 and CSI-RS#1+CSI-IM#1 in the periodic L1-SINR reports. If the value for both CSI-RSs is within the limits in Table 6.7.9.3.1.5-2 and Table 6.7.9.3.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6.	Set the parameters according to each sub-test in Table 6.7.9.3.1.5-1 as appropriate and repeat steps 3-5.

6.7.9.3.1.4.3	Message contents

Same message content as in subclause 6.7.9.1.4.3 with the following exception:

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.9.3.1.4.3-1: Common Exception messages NR SA CSI-RS based CMR and dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and CSI-SINR and CSI-IM_IMR Table H.3.6A-2 with conditions CSI-RS and PERIODIC Table H.3.6A-3 with condition PERIODIC Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.9.3.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

6.7.9.3.1.5	Test requirement

Table 6.7.9.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.9.3.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.9.3.1.5-3 for test configuration 3.

Table 6.7.9.3.5-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~3 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | FDD | FDD |
|  |  | 2 |  | TDD | TDD |
|  |  | 3 |  | TDD | TDD |
| TDD Configuration |  | 1 |  | N/A | N/A |
|  |  | 2 |  | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 3 |  | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 3 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | SR.1.1 FDD |
|  |  | 2 |  | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 3 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | CR.1.1 FDD |
|  |  | 2 |  | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 3 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | CCR.1.1 FDD |
|  |  | 2 |  | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 3 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 3 |  | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~3 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | TRS.1.1 FDD |
|  |  | 2 |  | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 3 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~3 |  | SMTC.1 | SMTC.1 |
| CSI-RS |  | 1 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |
|  |  | 2 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |
|  |  | 3 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |
| reportConfigType |  | 1~3 |  | periodic | periodic |
| reportQuantity-r16 |  | 1~3 |  | cri-SINR-r16 | cri-SINR-r16 |
| nrofReportedRS |  | 1~3 |  | 2 | 2 |
| L1-SINR reporting period |  | 1~3 |  | slot80 | slot80 |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| NocNote2 | Depending on band group | 1,2 | dBm/15kHz | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -96.00 | -117+ ΔBG_offset |
| NocNote2 |  | 1,2 | dBm/SSB SCS | -94.65 | -117+ ΔBG_offset |
|  |  | 3 |  | -93.00 | -114+ ΔBG_offset |
| EsIot |  | 1~3 | dB | 10 | -2.2 |
| SSB RSRP Note3 | Depending on band group | 1,2 | dBm/SSB SCS | -84.65 | -119.2 + ΔBG_offset |
|  |  | 3 |  | -83.00 | -116.2 + ΔBG_offset |
| Io Note3 | Depending on band group | 1,2 | dBm/9.36 MHz | -56.28 | -87.00 + ΔBG_offset |
|  |  | 3 | dBm/38.16 MHz | -51.53 | -80.90 + ΔBG_offset |
| EsNoc |  | 1~3 | dB | 10 | -2.2 |
| Propagation condition |  | 1~3 |  | AWGN | AWGN |
| Antenna configuration |  | 1~3 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table 6.7.9.3.5-2: Same as Table 4.7.7.3.5-2

Table 6.7.9.3.5-3: Same as Table 4.7.7.3.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.9.3.2	NR SA FR1 CSI-RS based CMR and dedicated IMR L1-SINR relative measurement accuracy

6.7.9.3.2.1	Test purpose

The purpose of this test is to verify that the L1-SINR relative measurement accuracy is within the specified limits.

6.7.9.3.2.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards. Applicability requires support of L1-SINR measurements.

6.7.9.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.7.9.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.9.3.

6.7.9.3.2.4	Test description

6.7.9.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.9.3.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.7.9.3.2.4.1-2.

Table 6.7.9.3.2.4.1-1: Applicable NR configurations for FR1 L1-SINR measurement test with CSI-RS based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.9.3.2.4.1-2.

Table 6.7.9.3.2.4.1-2: Initial conditions for CSI-RS based CMR and CSI-IM based IMR absolute accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.9.3.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. Message contents are defined in clause 6.7.9.3.2.4.3.

2. Cell 1 is the NR FR1 cell. Cell 1 is the target for CSI-RS based CMR and CSI-IM based IMR measurements. The UE is configured one CSI-RS resource set with two CSI-RS resources and one CSI-IM resource set with two CSI-IM resources. UE is configured to perform RLM and BFD based on SSB 0 and 1. CSI-RS is not transmitted in the same OFDM symbols as SSB. UE is configured to perform L1-SINR measurement based on the configured CSI-RS as CMR and CSI-IM as IMR. The connection setup is done according to the settings in Annex C.1.1.

6.7.9.3.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 6.7.9.3.2.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.7.9.3.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.7.9.3.2.5-1.

3.	The UE shall start sending L1-SINR report including results of both CSI-RS#0 and CSI-RS #1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS #0 and CSI-RS #1 in the periodic L1-SINR reports. The L1-SINR value for CSI-RS #1 is compared to the L1-SINR value for CSI-RS #0. If the difference is within the limits in Table 6.7.9.3.2.5-2 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 6.7.9.3.2.5-1 as appropriate and repeat steps 3-5.

6.7.9.3.2.4.3	Message contents

Same message contents as described in section 6.7.9.3.1.4.3

6.7.9.3.2.5	Test requirement

Table 6.7.9.3.2.5-1 defines the primary level settings including test tolerances for all tests.

Each L1-SINR measurement report for each of the tests in Table 6.7.9.3.2.5-1 shall meet the corresponding relative accuracy requirements in Table 6.7.9.3.2.5-2.

Table 6.7.9.3.2.5-1: Same as Table 6.7.9.3.1.5-1

Table 6.7.9.3.2.5-2: Same as Table 4.7.7.3.2.5-2

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.10	CSI-RSRP

#### 6.7.10.0	Minimum conformance requirements

Same as in clause 4.7.8.0.

#### 6.7.10.1	Intra-frequency measurements

##### 6.7.10.1.1	NR SA FR1 CSI-RSRP absolute measurement accuracy

Editor’s Note: This test case is incomplete. The following still is missing:

- Message content is missing

- Test procedure is missing

- Cell mapping is missing

- Antenna connection diagram is missing

- Applicability is missing

- TT is missing

6.7.10.1.1.1	Test purpose

The purpose of this test is to verify that the CSI-RSRP absolute measurement accuracy is within the specified limits for all bands. This test will partly verify the requirements in clause 10.1.2.3.1 in TS 38.133 [6].

6.7.10.1.1.2	Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting CSI-RS based RSRP and RSRQ measurement.

6.7.10.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.7.10.0

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.10.1.

6.7.10.1.1.1.4	Test description

6.7.10.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.10.1.1.4.1-1.

Table 6.7.10.1.1.4.1-1: NR SA FR1 CSI-RSRP absolute measurement accuracy supported test configurations

| Configuration | Description |
| --- | --- |
| 6.7.10.1.1-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.7.10.1.1-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.7.10.1.1-3 | NR 30kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.10.1.1.4.1-2.

Table 6.7.10.1.1.4-3: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 6.7.10.1.1.4.3.

2.	There are two cells on the same frequency. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

6.7.10.1.1.4.2	Test procedure

TBD

6.7.10.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.10.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

6.7.10.1.1.5	Test requirement

Table 6.7.10.1.1.5-1 defines the NR cell specific test parameters, not including test tolerances yet.

Table 6.7.10.1.1.5-1: Cell specific test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - | 4.7 | - | 4.7 | - | 4.7 |
|  |  | Config 2,3 | s | - | 2.35 | - | 2.35 | - | 2.35 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15KhZ | -106 |  | -88 |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 |  | -94 |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
| NocNote2 | Config 1,2 |  | dBm/SCS | -106 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 |  | -91 |  | -111 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -110.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -107.5 |  |
| EsIot |  |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | -0.01 | -4.76 |
| EsNoc |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -100 | -105 | -82 | -87 | -111.00 | -114.00 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -110.50 | -113.50 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -110.00 | -113.00 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -109.50 | -112.50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -109.00 | -112.00 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -108.50 | -111.50 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -108.00 | -111.00 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -107.50 | -110.50 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -90 | -108.00 | -111.00 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -107.50 | -110.50 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -107.00 | -110.00 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -106.50 | -109.50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -106.00 | -109.00 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -105.50 | -108.50 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -105.00 | -108.00 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -104.50 | -107.50 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -70.09 |  | -52.09 |  | -80.03 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -79.53 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -79.03 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -78.53 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -78.03 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -77.53 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -77.03 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -76.53 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | Not applicableNote 5- |  | -51.99 |  | -73.94 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -73.44 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -72.94 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -72.44 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -71.94 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -71.44 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -70.94 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -70.44 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI -RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI -RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |

Table 6.7.10.1.1.5-2: CSI-RSRP Intra frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

TBD

The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90% with the confidence level of 95%,

##### 6.7.10.1.2	NR SA FR1 CSI-RSRP relative measurement accuracy

Editor’s Note: This test case is incomplete. The following still is missing:

- Message content is missing

- Test procedure is missing

- Cell mapping is missing

- Antenna connection diagram is missing

- Applicability is missing

- TT is missing

6.7.10.1.2.1	Test purpose

The purpose of this test is to verify that the CSI-RSRP relative measurement accuracy is within the specified limits for all bands. This test will partly verify the requirements in clause 10.1.2.3.2 in TS 38.133 [6].

6.7.10.1.2.2	Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting CSI-RS based RSRP and RSRQ measurement.

6.7.10.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.7.10.0

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.10.1.

6.7.10.1.2.1.4	Test description

6.7.10.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.10.1.2.4.1-1.

Table 6.7.10.1.2.4.1-1: NR SA FR1 CSI-RSRP absolute measurement accuracy supported test canfigurations

| Configuration | Description |
| --- | --- |
| 6.7.10.1.2-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.7.10.1.2-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.7.10.1.2-3 | NR 30kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.10.1.2.4.1-2.

Table 6.7.10.1.2.4-3: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 6.7.10.1.2.4.3.

2.	There are two cells on the same frequency. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

6.7.10.1.2.4.2	Test procedure

TBD

6.7.10.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.10.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

6.7.10.1.2.5	Test requirement

Table 6.7.10.1.2.5-1 defines the NR cell specific test parameters, not including test tolerances yet.

Table 6.7.10.1.2.5-1: Cell specific test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Cell ID |  |  |  | 489 | 0 | 489 | 0 | 489 | 0 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA | TRS.1.1 FDD | NA |
|  |  | Config 2 |  | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA | TRS.1.1 TDD | NA |
|  |  | Config 3 |  | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA | TRS.1.2 TDD | NA |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR2.1 TDD |  | SR2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  | CR2.1 TDD |  |
| Control channel RMC |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  | CCR2.1 TDD |  |
| SSB configuration |  | Config 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 2 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | Config 3 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| Time offset with Cell 1 |  | Config 1 | ms | - | 4.7 | - | 4.7 | - | 4.7 |
|  |  | Config 2,3 | s | - | 2.35 | - | 2.35 | - | 2.35 |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30kHz |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| NocNote2 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15KhZ | -106 |  | -88 |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 |  | -94 |  | -114 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -111.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
| NocNote2 | Config 1,2 |  | dBm/SCS | -106 |  | -88 |  | Same as Noc/15kHz |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 |  | -91 |  | -111 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -110.5 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -110 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -109.5 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -109 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -108.5 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -108 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -107.5 |  |
| EsIot |  |  | dB | 2.46 | -5.97 | 2.46 | -5.97 | -0.01 | -4.76 |
| EsNoc |  |  | dB | 6 | 1 | 6 | 1 | 3 | 0 |
| CSI -RSRPNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -100 | -105 | -82 | -87 | -111.00 | -114.00 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -110.50 | -113.50 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -110.00 | -113.00 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -109.50 | -112.50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -109.00 | -112.00 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -108.50 | -111.50 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -108.00 | -111.00 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -107.50 | -110.50 |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | Not applicableNote 5 | Not applicableNote 5 | -85 | -90 | -108.00 | -111.00 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -107.50 | -110.50 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -107.00 | -110.00 |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -106.50 | -109.50 |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -106.00 | -109.00 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -105.50 | -108.50 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -105.00 | -108.00 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -104.50 | -107.50 |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -70.09 |  | -52.09 |  | -80.03 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -79.53 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -79.03 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -78.53 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -78.03 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -77.53 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -77.03 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -76.53 |  |
|  | Config 3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | Not applicableNote 5- |  | -51.99 |  | -73.94 |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  | -73.44 |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  | -72.94 |  |
|  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  | -72.44 |  |
|  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  | -71.94 |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  | -71.44 |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  | -70.94 |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  | -70.44 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI -RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI -RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: Subtest 1 is not used when testing with 30kHz SSB SCS. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |

Table 6.7.10.1.2.5-2: SS-RSRP Intra frequency relative accuracy requirements for the reported values for test configurations 1 and 2

TBD

The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90% with the confidence level of 95%,

#### 6.7.10.2	Inter-frequency measurements

##### 6.7.10.2.1	NR SA FR1-FR1 CSI-RSRP absolute measurement accuracy

Editor’s Note: This test case is incomplete. The following still is missing:

- Message content is missing

- Test procedure is missing

- Cell mapping is missing

- Antenna connection diagram is missing

- Applicability is missing

- TT is missing

6.7.10.2.1.1	Test purpose

The purpose of this test is to verify that the inter-frequency CSI-RSRP absolute measurement accuracy is within the specified limits for all bands. This test will partly verify the requirements in clause 10.1.4.3.1 in TS 38.133 [6].

6.7.10.2.1.2	Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting CSI-RS based RSRP and RSRQ measurement.

6.7.10.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.7.10.0

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.10.2.

6.7.10.2.1.1.4	Test description

6.7.10.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.10.2.1.4.1-1.

Table 6.7.10.2.1.4.1-1: NR SA FR1 CSI-RSRP absolute measurement accuracy supported test canfigurations

| Configuration | Description |
| --- | --- |
| 6.7.10.2.1-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.7.10.2.1-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.7.10.2.1-3 | NR 30kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.10.2.1.4.1-2.

Table 6.7.10.2.1.4-3: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 6.7.10.2.1.4.3.

2.	There are two cells on different frequencies. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

6.7.10.2.1.4.2	Test procedure

TBD

6.7.10.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.10.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

6.7.10.2.1.5	Test requirement

Table 6.7.10.2.1.5-1 defines the NR cell specific test parameters, not including test tolerances yet.

Table 6.7.10.2.1.5-1: Cell specific test parameters

| Parameter |  | Config | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  | FDD |  |  |
|  |  | 2 |  | TDD |  | TDD |  |  |
|  |  | 3 |  | TDD |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1,2 | s | - | 4.7 | - | 4.7 |  |
|  |  | 3 | s | - | 2.35 | - | 2.35 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |  |
| CSI-RS configuration for RRM |  | 1 |  | CSI-RS.RRM.FR1.1 FDD |  | CSI-RS.RRM.FR1.1 FDD |  |  |
|  |  | 2 |  | CSI-RS.RRM.FR1.1 TDD |  | CSI-RS.RRM.FR1.1 TDD |  |  |
|  |  | 3 |  | CSI-RS.RRM.FR1.2 TDD |  | CSI-RS.RRM.FR1.2 TDD |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| NocNote2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.5 |
| NocNote2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  | ( for Channel 2 +8dB) |  | -112.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -112.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -110.50 |
| EsIot |  | 1~3 | dB | 10 | 10 | 13 |  | -3 |
| CSI-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SCS | -84.65 |  | (RSRP for Cell 2 +25dB) |  | -118.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -117.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -115.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  | (RSRP for Cell 2 +25dB) |  | -115.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -114.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/ 9.36MHz | -56.28 |  | Io for Channel 2 +19.75dB |  | -85.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -84.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -82.78 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16MHz | -50.19 |  | Io for Channel 2 +19.75dB |  | -79.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -78.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -76.69 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -75.69 |
| EsNoc |  | 1~3 | dB | 10 | 10 | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

Table 6.7.10.2.1.5-2: CSI-RSRP Inter frequency absolute accuracy requirements for the reported values for test configurations 1 and 2

TBD

The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90% with the confidence level of 95%,

##### 6.7.10.2.2	NR SA FR1-FR1 CSI-RSRP relative  measurement accuracy

Editor’s Note: This test case is incomplete. The following still is missing:

- Message content is missing

- Test procedure is missing

- Cell mapping is missing

- Antenna connection diagram is missing

- Applicability is missing

- TT is missing

6.7.10.2.2.1	Test purpose

The purpose of this test is to verify that the inter-frequency CSI-RSRP relative measurement accuracy is within the specified limits for all bands. This test will partly verify the requirements in clause 10.1.4.3.2 in TS 38.133 [6].

6.7.10.2.2.2	Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting CSI-RS based RSRP and RSRQ measurement.

6.7.10.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.7.10.0

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.10.2.

6.7.10.2.2.1.4	Test description

6.7.10.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.10.2.2.4.1-1.

Table 6.7.10.2.2.4.1-1: NR SA FR1 CSI-RSRP relative  measurement accuracy supported test canfigurations

| Configuration | Description |
| --- | --- |
| 6.7.10.2.2-1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.7.10.2.2-2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.7.10.2.2-3 | NR 30kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.10.2.2.4.1-2.

Table 6.7.10.2.2.4-3: Test Environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.1.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 6.7.10.2.2.4.3.

2.	There are two cells on different frequencies. Cell 1 is the PCell, and Cell 2 is the target cell for CSI-RSRP measurements.

6.7.10.2.2.4.2	Test procedure

TBD

6.7.10.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.7.10.2.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

6.7.10.2.2.5	Test requirement

Table 6.7.10.2.2.5-1 defines the NR cell specific test parameters, not including test tolerances yet.

Table 6.7.10.2.2.5-1: Cell specific test parameters

| Parameter |  | Config | Unit | Test 1 |  | Test 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  | Cell 2 |
| SSB ARFCN |  | 1~3 |  | freq1 | freq2 | freq1 |  | freq2 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |  |
|  |  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |  |
|  |  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |  |
| Duplex mode |  | 1 |  | FDD |  | FDD |  |  |
|  |  | 2 |  | TDD |  | TDD |  |  |
|  |  | 3 |  | TDD |  | TDD |  |  |
| TDD configuration |  | 1 |  | N/A |  | N/A |  |  |
|  |  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  | - |
|  |  | 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |
|  |  | 3 |  | SR.2.1 FDD |  | SR.2.1 FDD |  |  |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  | - |
|  |  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD |  | - |
|  |  | 3 |  | CR.2.1 FDD | - | CR.2.1 FDD |  | - |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  | - |
|  |  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD |  | - |
|  |  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD |  | - |
| SSB configuration |  | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
|  |  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |  |
|  |  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |  |
| OCNG Patterns |  | 1~3 |  | OP.1 |  | OP.1 |  |  |
| TRS configuration |  | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |  |
|  |  | 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |
|  |  | 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |
| Initial BWP Configuration |  | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| Time offset with Cell 1 |  | 1,2 | s | - | 4.7 | - | 4.7 |  |
|  |  | 3 | s | - | 2.35 | - | 2.35 |  |
| SMTC configuration |  | 1 |  | SMTC.2 |  | SMTC.2 |  |  |
|  |  | 2,3 |  | SMTC.1 |  | SMTC.1 |  |  |
| CSI-RS configuration for RRM |  | 1 |  | CSI-RS.RRM.FR1.1 FDD |  | CSI-RS.RRM.FR1.1 FDD |  |  |
|  |  | 2 |  | CSI-RS.RRM.FR1.1 TDD |  | CSI-RS.RRM.FR1.1 TDD |  |  |
|  |  | 3 |  | CSI-RS.RRM.FR1.2 TDD |  | CSI-RS.RRM.FR1.2 TDD |  |  |
| EPRE ratio of PSS to SSS |  | 1~3 | dB | 0 | 0 | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| NocNote2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~3 | dBm/15kHz | -94.65 |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.5 |
| NocNote2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SSB SCS | -94.65 |  | ( for Channel 2 +8dB) |  | -115 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -114.5 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -114 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.5 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.5 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -112 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.5 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -91.65 |  | ( for Channel 2 +8dB) |  | -112.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -112.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -111.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -111.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -110.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -110.50 |
| EsIot |  | 1~3 | dB | 10 | 10 | 13 |  | -3 |
| CSI-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/SCS | -84.65 |  | (RSRP for Cell 2 +25dB) |  | -118.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -117.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -117.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -116.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -116.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -115.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -115.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -114.50 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 |  | -81.65 |  | (RSRP for Cell 2 +25dB) |  | -115.00 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -114.50 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -114.00 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -113.50 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -113.00 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -112.50 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -112.00 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -111.50 |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2 | dBm/ 9.36MHz | -56.28 |  | Io for Channel 2 +19.75dB |  | -85.28 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -84.78 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -84.28 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -83.78 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -83.28 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -82.78 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -82.28 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -81.78 |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3 | dBm/ 38.16MHz | -50.19 |  | Io for Channel 2 +19.75dB |  | -79.19 |
|  | NR_FDD_FR1_B |  |  |  |  |  |  | -78.69 |
|  | NR_TDD_FR1_C |  |  |  |  |  |  | -78.19 |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  | -77.69 |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  | -77.19 |
|  | NR_FDD_FR1_F |  |  |  |  |  |  | -76.69 |
|  | NR_FDD_FR1_G |  |  |  |  |  |  | -76.19 |
|  | NR_FDD_FR1_H |  |  |  |  |  |  | -75.69 |
| EsNoc |  | 1~3 | dB | 10 | 10 | 13 |  | -3 |
| Propagation condition |  | 1~3 | - | AWGN |  | AWGN |  |  |
| Antenna configuration |  | 1~3 |  | 1x2 |  | 1x2 |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

Table 6.7.10.2.2.5-2: CSI-RSRP Inter frequency relative accuracy requirements for the reported values for test configurations 1 and 2

TBD

The rate of correct observed PSCell addition delay and PSCell release delay during repeated tests shall be at least 90% with the confidence level of 95%,

### 6.7.11	CSI-RSRQ

#### 6.7.11.0	Minimum conformance requirements

6.7.11.0.1	Intra-frequency CSI-RSRQ accuracy requirements

[TS 38.133, Clause 10.1.7.2 and Clause 10.1.11]

6.7.11.0.1.1	Absolute CSI-RSRQ accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRQ in this clause apply to the intra-frequency measurement defined in Clause 9.10.2.1 in FR1 in TS 38.133 [6].

The accuracy requirements in Table 6.7.11.0.1.1-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 in TS 38.133 [6] for a corresponding Band for associated SSB.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.12 in TS 38.133 [6] for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 6.7.11.0.1.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Note:	The reference measurement timing for one layer for intra-frequency measurement is serving cell timing.

Table 6.7.11.0.1.1-1: CSI-RSRQ Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| 2.5 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in Clause 3.5.2 in TS 38.133 [6]. |  |  |  |  |  |  |  |  |

The reporting range of CSI-RSRQ measurement is defined from -43 dB to 20 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 6.7.11.0.1.1-2. The range in the signaling may be larger than the guaranteed accuracy range.

Table 6.7.11.0.1.1-2: CSI-RSRQ measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CSI-RSRQ_0 | CSI-RSRQ<-43 | dB |
| CSI-RSRQ_1 | -43≤ CSI-RSRQ<-42.5 | dB |
| CSI-RSRQ_2 | -42.5≤ CSI-RSRQ<-42 | dB |
| CSI-RSRQ_3 | -42≤ CSI-RSRQ<-41.5 | dB |
| CSI-RSRQ_4 | -41.5≤ CSI-RSRQ<-41 | dB |
| .. | .. | … |
| CSI-RSRQ_122 | 17.5≤ CSI-RSRQ<18 | dB |
| CSI-RSRQ_123 | 18≤ CSI-RSRQ<18.5 | dB |
| CSI-RSRQ_124 | 18.5≤ CSI-RSRQ<19 | dB |
| CSI-RSRQ_125 | 19≤ CSI-RSRQ<19.5 | dB |
| CSI-RSRQ_126 | 19.5≤ CSI-RSRQ<20 | dB |
| CSI-RSRQ_127 | 20 ≤ CSI-RSRQ | dB |

6.7.11.0.2	Inter-frequency CSI-RSRQ accuracy requirements

[TS 38.133, Clause 10.1.9.2 and Clause 10.1.11]

6.7.11.0.2.1	Absolute CSI-RSRQ accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-RSRQ in this clause apply to the inter-frequency measurement defined in Clause 9.10.3.1 in FR1 in TS 38.133 [6].

The accuracy requirements in Table 6.7.11.0.2.1-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 in TS 38.133 [6] for a corresponding Band for associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.13 in TS 38.133 [6] for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 6.7.11.0.2.1-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Note:	The reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

Table 6.7.11.0.2.1-1: CSI-RSRQ Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| 2.5 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in Clause 3.5.2 in TS 38.133 [6]. |  |  |  |  |  |  |  |  |

The reporting range of CSI-RSRQ measurement is defined from -43 dB to 20 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 6.7.11.0.2.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.11.0.2.1-2: CSI-RSRQ measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CSI-RSRQ_0 | CSI-RSRQ<-43 | dB |
| CSI-RSRQ_1 | -43≤ CSI-RSRQ<-42.5 | dB |
| CSI-RSRQ_2 | -42.5≤ CSI-RSRQ<-42 | dB |
| CSI-RSRQ_3 | -42≤ CSI-RSRQ<-41.5 | dB |
| CSI-RSRQ_4 | -41.5≤ CSI-RSRQ<-41 | dB |
| .. | .. | … |
| CSI-RSRQ_122 | 17.5≤ CSI-RSRQ<18 | dB |
| CSI-RSRQ_123 | 18≤ CSI-RSRQ<18.5 | dB |
| CSI-RSRQ_124 | 18.5≤ CSI-RSRQ<19 | dB |
| CSI-RSRQ_125 | 19≤ CSI-RSRQ<19.5 | dB |
| CSI-RSRQ_126 | 19.5≤ CSI-RSRQ<20 | dB |
| CSI-RSRQ_127 | 20 ≤ CSI-RSRQ | dB |

6.7.11.0.2.2	Relative CSI-RSRQ accuracy

The relative accuracy of CSI-RSRQ is defined as the CSI-RSRQ measured from one cell compared to the CSI-RSRQ measured from another cell with the same center frequency, or between any two CSI-RSRQ levels measured on the same cell in FR1.

The accuracy requirements in Table 6.7.11.0.2.2-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 in TS 38.133 [6] for a corresponding Band for the associated SSB.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.13 in TS 38.133 [6] for a corresponding Band for each relevant CSI-RS.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 6.7.11.0.2.2-1.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

Note:	The reference measurement timing for one layer for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

Table 6.7.11.0.2.2-1: CSI-RSRQ Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  | Note 2 | NR operating band groups Note 4 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| 3 | 4 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -50 |
| 4 | 4 | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in Clause 3.5.2 in TS 38.133 [6]. |  |  |  |  |  |  |  |  |

#### 6.7.11.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing

6.7.11.1.1	Test purpose

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.7.2 in TS 38.133 [6].

6.7.11.1.2			Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting NR SA and CSI-RS based RSRP and RSRQ measurements.

6.7.11.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in Clause 6.7.11.0.1.

The normative reference for this requirement is in TS 38.133 [6] A.6.7.11.1.

6.7.11.1.4	Test description

6.7.11.1.4.1	Initial conditions

Supported test configuration are shown in Table 6.7.11.1.4.1-1.

Table 6.7.11.1.4.1-1: Intra frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.11.1.4.1-2.

Table 6.7.11.1.4.1-2: Initial conditions for CSI-RSRQ intra-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] Clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] Clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.11.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | Without the LTE link |  |  |

1.	Message contents are defined in Clause 6.7.11.2.4.3.

2.	In this test case all cells are on the same carrier frequency. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for CSI-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.3.

6.7.11.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] Clause 4.5.

2.	Set the parameters according to Table 6.7.11.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the CSI-RSRQ reported values in the periodic MeasurementReport. The CSI-RSRQ value of Cell 2 reported by the UE is compared to the expected CSI-RSRQ. If the value is outside the limits (determined by Table 6.7.11.0.1.1-1 and Table 6.7.11.0.1.1-2) or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 6.7.11.1.5-1 as appropriate and repeat steps 5-7.

6.7.11.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] Clause 7.3 with the following exceptions.

Table 6.7.11.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with condition CSI-RS MOBILITY Table H.3.1-5 Table H.3.1-7 with condition CSI-RS MOBILITY and RSRQ |

Table 6.7.11.1.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| rsType | csirs |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.11.1.5	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in Clause 6.7.11.0.1.

Table 6.7.11.1.5-1: CSI-RSRQ Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |  |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 |  |  | Cell 2 | Cell 1 | Cell 2 |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| Gap Pattern ID |  |  |  |  | 0 |  |  |  |  |  |  |  |
| BWP configuration |  |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |  |
|  |  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |  |
|  |  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |  |
|  |  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle |  |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD |  |  | - | SR.1.1 FDD | - |
|  |  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |  | - | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  |  | CR.2.1 TDD |  |
| Control Channel RMC |  |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD |  |  | - | CCR.1.1 FDD | - |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |  | CCR.2.1 TDD |  |
| TRS Configuration |  |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD |  |  | - | TRS.1.1 FDD | - |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  |  | TRS.1.1 TDD |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  |  | TRS.1.2 TDD |  |
| OCNG Patterns |  |  |  |  | OP. 1 |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1 |  | s | - | 4.7 |  | - | 4.7 |  | - | 4.7 |
|  |  | Config 2,3 |  | s | - | 2.35 |  | - | 2.35 |  | - | 2.35 |
| CSI-RS configuration for RRM |  | Config 1 |  |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1 |  |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  |  | Config 3 |  | 30kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 |  |  | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |
| NocNote2 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -85 |  | -101 |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -91 |  | - |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
| NocNote2 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -85 |  | -101 |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -113.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -113 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -112.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -112 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -111.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.5 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.5 |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -88 |  | - |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -110.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -110 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -109.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -109 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -108.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -108 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -107.5 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -104.5 |  |
| EsIot |  |  |  | dB | -1.76 |  | -4.7 |  |  |  | -5..46 | -5.46 |
| EsNoc |  |  |  | dB | 3 | 3 | -2.9 |  |  | -2.9 | -4 | -4 |
| SS-RSRP/ CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -82 | -82 | -103.9 |  |  | -103.9 | -118 | -118 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -117.5 | -117.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -117 | -117 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.5 | -116.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116 | -116 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.5 | -115.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -115 | -115 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85 | -85 | - |  |  | - | -115 | -115 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 | -114.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 | -114 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 | -113.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 | -113 |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 | -112.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 | -112 |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 | -111.5 |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 | -108.5 |
| SS-RSRQ/CSI-RSRQ Note3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -16.76 |  |  | -16.76 | -17.34 | -17.34 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -50 |  | -70 |  |  |  | -83.5 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -83 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -82.5 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -82 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -81.5 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -81 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -80.5 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -80 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -77 |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -50 |  | - |  |  |  | -77.4 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -76.9 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -76.4 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -75.9 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -75.4 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -74.9 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -74.4 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -73.9 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -70.9 |  |
| Propagation condition |  |  |  | - | AWGN | AWGN | AWGN |  |  | AWGN | AWGN | AWGN |
| Antenna configuration |  |  |  |  | 1x2 | 1x2 | 1x2 |  |  | 1x2 | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3.5.2 in TS 38.133 [6]. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.11.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing

6.7.11.2.1	Test purpose

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.2.1 and 10.1.9.2.2 in TS 38.133 [6].

6.7.11.2.2	Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting NR SA and CSI-RS based RSRP and RSRQ measurements.

6.7.11.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in Clause 6.7.11.0.2.

The normative reference for this requirement is in TS 38.133 [6] A.6.7.11.2.

6.7.11.2.4	Test description

6.7.11.2.4.1	Initial conditions

Supported test configurations are shown in Table 6.7.11.2.4.1-1.

Table 6.7.11.2.4.1-1: CSI-RSRQ Inter frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.11.2.4.1-2.

Table 6.7.11.2.4.1-2: Initial conditions for CSI-RSRQ inter-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] Clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] Clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.11.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in Clause 6.7.11.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for CSI-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.11.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] Clause 4.5.

2. Set the parameters according to Table 6.7.11.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the CSI-RSRQ reported values in the periodic MeasurementReport. The CSI-RSRQ value of Cell 2 reported by the UE is compared to the CSI-RSRQ value of Cell 1 reported by the UE. If the difference between both values is outside the limits (determined by Table 6.7.11.0.2.1-1 and Table 6.7.11.0.2.1-2) or the UE fails to report the measurement value for Cell 2 or Cell 1, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.11.2.5-1 as appropriate and repeat steps 5-7.

6.7.11.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] Clause 7.3 with the following exceptions.

Table 6.7.11.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with condition CSI-RS MOBILITY Table H.3.1-5 Table H.3.1-6 with condition gapUE and Pattern #0 Table H.3.1-7 with condition INTER-FREQ, CSI-RS MOBILITY and RSRQ |

Table 6.7.11.2.4.3-2: ReportConfigNR-DEFAULT (Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| rsType | csirs |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.11.2.5	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in Clause 6.7.11.0.2.

Table 6.7.11.2.5-1: CSI-RSRQ Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1,2,3 |  | 0 |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | R.1.1 FDD | - |  | CR.1.1 FDD |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 | s | - | 4.7 | - | 4.7 | - |  | 4.7 |  |
|  |  | Config 3 | s | - | 2.35 | - | 2.35 | - |  | 2.35 |  |
| CSI-RS configuration for RRM |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| NocNote2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  | -106 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
| NocNote2 | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -86.27 |  | -113 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
| NocNote2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  | -106 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  | -110 |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -112 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -111.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -111 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -110 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -109.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -106.5 |  |  |
| EsIot |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
| EsNoc |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
| SS-RSRP/CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 | -81.93 | -107.75 | -107.75 |  | -113 |  | -117.75 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -112.5 |  | -117.25 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -112 |  | -116.75 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -111.5 |  | -116.25 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -111 |  | -115.75 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.5 |  | -115.2 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -110 |  | -114.75 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -109.5 |  | -114.25 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -106.5 |  | -111.25 |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 | -85.02 | -111.75 | -111.75 |  | -110 |  | -114.75 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -109.5 |  | -114.25 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -109 |  | -113.75 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -108.5 |  | -113.25 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -108 |  | -112.75 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -107.5 |  | -112.2 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -107 |  | -111.75 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -106.5 |  | -111.25 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -103.5 |  | -108.25 |
| SS-RSRQ/CSI-RSRQNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -40.59 | -40.59 |  | 12.56T |  | 14.76T |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -50 |  | -75.83 |  |  | -83.28 |  | -85.83 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -82.78 |  | -85.33 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -82.28 |  | -84.83 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -81.78 |  | -84.33 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -81.28 |  | -83.83 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -80.78 |  | -83.33 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -80.28 |  | -82.83 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -79.78 |  | -82.33 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -76.78 |  | -79.33 |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -50 |  | -76.73 |  |  | -77.19 |  | -79.73 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -76.69 |  | -79.23 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -76.19 |  | -78.73 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -75.69 |  | -78.23 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -75.19 |  | -77.73 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -74.69 |  | -77.23 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -74.19 |  | -76.73 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -73.69 |  | -76.53 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -70.69 |  | -73.53 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3.5.2 in TS 38.133 [6]. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.12	CSI-SINR

#### 6.7.12.0	Minimum conformance requirements

6.7.12.0.1	Intra-frequency CSI-SINR accuracy requirements

[TS 38.133, Clause 10.1.12.2 and Clause 10.1.16]

6.7.12.0.1.1	Absolute CSI-SINR accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 6.7.12.0.1.1-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.12 in TS 38.133 [6] for a corresponding Band.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

-	Note: The reference measurement timing for intra-frequency measurement is serving cell timing.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 6.7.12.0.1.1-1.

Table 6.7.12.0.1.1-1: CSI-SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
|  |  |  |  | SCS (kHz) |  |  |  |  |
| 3 | 4 | -3 |  | 15 | 30 | 60 |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in Clause 3.5.2 in TS 38.133 [6]. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |  |  |  |

The reporting range of CSI-SINR for reporting is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 6.7.12.0.1.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 6.7.12.0.1.1-2: CSI-SINR measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CSI-SINR_0 | CSI-SINR <-23 | dB |
| CSI-SINR_1 | -23≤ CSI-SINR <-22.5 | dB |
| CSI-SINR_2 | -22.5≤ CSI-SINR <-22 | dB |
| CSI-SINR_3 | -22≤ CSI-SINR <-21.5 | dB |
| CSI-SINR_4 | -21.5≤ CSI-SINR <-21 | dB |
| .. | .. | … |
| CSI-SINR_123 | 38≤ CSI-SINR <38.5 | dB |
| CSI-SINR_124 | 38.5≤ CSI-SINR <39 | dB |
| CSI-SINR_125 | 39≤ CSI-SINR <39.5 | dB |
| CSI-SINR_126 | 39.5≤ CSI-SINR <40 | dB |
| CSI-SINR_127 | 40≤ CSI-SINR | dB |

6.7.12.0.2	Inter-frequency CSI-SINR accuracy requirements

[TS 38.133, Clause 10.1.14.2 and Clause 10.1.16]

6.7.12.0.2.1	Absolute CSI-SINR accuracy

The requirements for absolute accuracy of CSI-SINR in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 6.7.12.0.2.1-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.13 in TS 38.133 [6] for a corresponding Band.

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

-	Note: The reference measurement timing for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 6.7.12.0.2.1-1.

Table 6.7.12.0.2.1-1: CSI-SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
|  |  |  |  | SCS (kHz) |  |  |  |  |
| 3 | 4 | -3 |  | 15 | 30 | 60 |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 3.5 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: NR operating band groups in FR1 are as defined in Clause 3.5.2 in TS 38.133 [6]. NOTE 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |  |  |  |

The reporting range of CSI-SINR for reporting is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 6.7.12.0.2.1-2. The range in the signaling may be larger than the guaranteed accuracy range.

Table 6.7.12.0.2.1-2: CSI-SINR measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CSI-SINR_0 | CSI-SINR <-23 | dB |
| CSI-SINR_1 | -23≤ CSI-SINR <-22.5 | dB |
| CSI-SINR_2 | -22.5≤ CSI-SINR <-22 | dB |
| CSI-SINR_3 | -22≤ CSI-SINR <-21.5 | dB |
| CSI-SINR_4 | -21.5≤ CSI-SINR <-21 | dB |
| .. | .. | … |
| CSI-SINR_123 | 38≤ CSI-SINR <38.5 | dB |
| CSI-SINR_124 | 38.5≤ CSI-SINR <39 | dB |
| CSI-SINR_125 | 39≤ CSI-SINR <39.5 | dB |
| CSI-SINR_126 | 39.5≤ CSI-SINR <40 | dB |
| CSI-SINR_127 | 40≤ CSI-SINR | dB |

6.7.12.0.2.2	Relative CSI-SINR accuracy

The relative accuracy of CSI-SINR in inter-frequency case is defined as the CSI-SINR measured from one cell on a frequency in FR1 compared to the CSI-SINR measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 6.7.12.0.2.2-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.13 in TS 38.133 [6] for a corresponding Band.

-	|CSI_RP1dBm - CSI_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

-	Note: The reference measurement timing for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

-	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 6.7.12.0.2.2-1.

Table 6.7.12.0.2.2-1: CSI-SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCS |  |  | dBm/BW Channel | dBm/BW Channel |
|  |  |  |  | SCS (kHz) |  |  |  |  |
| 3.5 | 4 | -3 |  | 15 | 30 | 60 |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | -108.5 | N/A | -70 |
| 4 | 4 | -6 | Note 2 | Note 2 | Note 2 | Note 2 | N/A | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The parameter CSI-RS Ês/Iot is the minimum CSI-RS Ês/Iot of the pair of cells to which the requirement applies. NOTE 4: NR operating band groups in FR1 are as defined in Clause 3.5.2 in TS 38.133 [6]. NOTE 5: The requirements apply for CSI-RS Ês/Iot ≤ XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |  |  |  |

#### 6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing

6.7.12.1.1	Test purpose

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.12.2.1 in TS 38.133 [6].

6.7.12.1.2			Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting NR SA, which supports CSI-RS based SINR measurements.

6.7.12.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in Clause 6.7.12.0.1.

The normative reference for this requirement is in TS 38.133 [6] A.6.7.12.1.

6.7.12.1.4	Test description

6.7.12.1.4.1	Initial conditions

Supported test configuration are shown in Table 6.7.12.1.4.1-1.

Table 6.7.12.1.4.1-1: CSI-SINR Intra frequency CSI-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.12.1.4.1-2.

Table 6.7.12.1.4.1-2: Initial conditions for CSI-SINR intra-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] Clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] Clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.12.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1.	Message contents are defined in Clause 6.7.12.1.4.3.

2.	In this test case all cells are on the same carrier frequency. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the same frequency as Cell 1. Cell 2 is the target cell for CSI-SINR measurements. The connection setup is done according to the settings in Annex C.1.3.

6.7.12.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] Clause 4.5.

2.	Set the parameters according to Table 6.7.12.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the CSI-SINR reported values in the periodic MeasurementReport. The CSI-SINR value of Cell 2 reported by the UE is compared to the expected CSI-SINR. If the value is outside the limits (determined by Table 6.7.12.0.1.1-1 and Table 6.7.12.0.1.1-2) or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 6.7.12.1.5-1 as appropriate and repeat steps 5-7.

6.7.12.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] Clause 7.3 with the following exceptions.

Table 6.7.12.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with condition CSI-RS MOBILITY Table H.3.1-5 Table H.3.1-7 with condition CSI-RS MOBILITY and SINR |

Table 6.7.12.1.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| rsType | csirs |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.12.1.5	Test Requirements

The CSI-SINR measurement accuracy shall fulfil the requirements in Clause 6.7.12.0.1.1.

Table 6.7.12.1.5-1: CSI-SINR Intra frequency test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | Config 1 |  | FDD |  |  |  |
|  |  |  | Config 2,3 |  | TDD |  |  |  |
| TDD configuration |  |  | Config 1 |  | Not Applicable |  |  |  |
|  |  |  | Config 2 |  | TDDConf.1.1 |  |  |  |
|  |  |  | Config 3 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  |  | Config 1 |  | TRS.1.1 FDD |  |  |  |
|  |  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  |  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |
|  |  |  | Config 3 |  | SR.2.1 TDD |  | SR2.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | Config 1 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
|  |  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  |  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |
| CSI-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 |  | s | - | 2.35 | - | 2.35 |
|  |  | Config 3 |  | s | - | 1.17 | - | 1.17 |
| SSB configuration |  |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |
|  |  |  | Config 3 |  | SSB.2 FR1 |  |  |  |
| SMTC configuration |  |  | Config 1 |  | SMTC.2 |  |  |  |
|  |  |  | Config 2,3 |  | SMTC.1 |  |  |  |
| CSI-RS for mobility |  |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |
|  |  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |
|  |  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2 | kHz | 15 |  |  |  |
|  |  |  | Config 3 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| NocNote2 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -93 |  | -116 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | -115.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | -115 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -114.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  | -113.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | -113 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -112.5 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -109.5 |  |
| NocNote2 | Config 1,2 |  |  | dBm/SCS | -93 |  | Same as Noc for 15 kHz |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -90 |  | -113 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | -112.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | -112 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -111.5 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  | -110.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | -110 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -109.5 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -106.5 |  |
| EsIot |  |  |  | dB | 0 | -3.19 | -5.46 | -5.46 |
| EsNoc |  |  |  | dB | 4.54 | 2.66 | -4 | -4 |
| CSI-RSRPNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/SCS | -88.46 | -90.34 | -120 | -120 |
|  |  |  | NR_FDD_FR1_B |  |  |  | -119.5 | -119.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  | -119 | -119 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5 | -118.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118 | -118 |
|  |  |  | NR_FDD_FR1_F |  |  |  | -117.5 | -117.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  | -117 | -117 |
|  |  |  | NR_FDD_FR1_H |  |  |  | -116.5 | -116.5 |
|  |  |  | NR_FDD_FR1_N |  |  |  | -113.5 | -113.5 |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 |  | -85.46 | -87.34 | -117 | -117 |
|  |  |  | NR_FDD_FR1_B |  |  |  | -116.5 | -116.5 |
|  |  |  | NR_TDD_FR1_C |  |  |  | -116 | -116 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5 | -115.5 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115 | -115 |
|  |  |  | NR_FDD_FR1_F |  |  |  | -114.5 | -114.5 |
|  |  |  | NR_FDD_FR1_G |  |  |  | -114 | -114 |
|  |  |  | NR_FDD_FR1_H |  |  |  | -113.5 | -113.5 |
|  |  |  | NR_FDD_FR1_N |  |  |  | -113.5 | -113.5 |
| CSI-SINR Note3 |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dB | 0 | -3.19 | -5.46 | -5.46 |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_F |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -110.5 | -110.5 |
| IoNote3 | Config 1,2 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.5 |  | -85.51 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | -85.01 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | -84.51 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -84.01 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -83.51 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  | -83.01 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | -82.51 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -82.01 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -79.01 |  |
|  | Config 3 |  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.41 |  | -79.41 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  | -78.91 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  | -78.41 |  |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -77.91 |  |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -77.41 |  |
|  |  |  | NR_FDD_FR1_F |  |  |  | -76.91 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  | -76.41 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  | -75.91 |  |
|  |  |  | NR_FDD_FR1_N |  |  |  | -72.91 |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-SINR, CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3.5.2 in TS 38.133 [6]. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 6.7.12.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing

6.7.12.2.1	Test purpose

The purpose of this test is to verify that the CSI-SINR measurement accuracy is within the specified limits. This test will verify the requirements in Clauses 10.1.14.2.1 and 10.1.14.2.2 in TS 38.133 [6].

6.7.12.2.2			Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting NR SA, which supports CSI-RS based SINR measurements.

6.7.12.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in Clause 6.7.12.0.2.

The normative reference for this requirement is in TS 38.133 [6] A.6.7.12.2.

6.7.12.2.4	Test description

6.7.12.2.4.1	Initial conditions

Supported test configurations are shown in Table 6.7.12.2.4.1-1.

Table 6.7.12.2.4.1-1: CSI-SINR Inter frequency CSI-SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.12.2.4.1-2.

Table 6.7.12.2.4.1-2: Initial conditions for CSI-SINR inter-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] Clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] Clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.12.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1.	Message contents are defined in Clause 6.7.12.2.4.3.

2.	In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for CSI-SINR measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.12.2.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] Clause 4.5.

2.	Set the parameters according to Table 6.7.12.2.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the CSI-SINR reported values in the periodic MeasurementReport. The CSI-SINR value of Cell 2 reported by the UE is compared to the expected CSI-SINR. If the value is outside the limits (determined by Table 6.7.12.0.2.1-1 and Table 6.7.12.0.2.1-2) or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 6.7.12.2.5-1 as appropriate and repeat steps 5-7.

6.7.12.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] Clause 7.3 with the following exceptions.

Table 6.7.12.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with condition CSI-RS MOBILITY Table H.3.1-5 Table H.3.1-6 with condition gapUE and Pattern #0 Table H.3.1-7 with condition INTER-FREQ, CSI-RS MOBILITY and SINR |

Table 6.7.12.2.4.3-2: ReportConfigNR-DEFAULT(Periodical)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| rsType | csirs |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.7.12.2.5	Test Requirements

The CSI-SINR measurement accuracy shall fulfil the requirements in Clause 6.7.12.0.2.1 and 6.7.12.0.2.2

Table 6.7.12.2.5-1: CSI-SINR Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 |  | freq1 |  | freq2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | Config 2 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | R.1.1 FDD | - |  | CR.1.1 FDD |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |
| CSI-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 | s | - | 2.35 | - | 2.35 | - |  | 2.35 |  |
|  |  | Config 3 | s | - | 1.17 | - | 1.17 | - |  | 1.17 |  |
| SMTC configuration |  | Config 1 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | SMTC.1 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| CSI-RS for mobility |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -88 |  | -108.5 |  |  | -119.5 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -119 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -118.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -118 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -117.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -117 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -116.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -113 |  |  |
| Noc Note2 | Config 1,2 N |  | dBm/15kHz | -88 |  | -108.5 |  |  | Same as Noc for 15kHz T |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85 |  | -105.5 |  |  | -116.5 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -116 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -110 |  |  |
| EsIot |  |  | dB | -1.75 | -1.75 | 15 | 15 |  | -4.0 |  | -4.0 |
| EsNoc |  |  | dB | -1.75 |  | 15 |  |  | -4.0 |  |  |
| CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -89.75 |  | -93.5 |  |  | -123.5 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -123 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -122.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -122 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -121.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -121 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -120.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -120 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -117 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -86.75 |  | -90.5 |  |  | -120.5 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -120 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -119.5 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -119 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -118.5 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -118 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -117.5 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -117 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -114 |  |  |
| CSI-SINRNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -1.75 |  | 15 |  |  | -4.0 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 9.36MHz | -57.83 |  | -65.4 |  |  | -90.09 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -89.59 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -89.09 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -88.59 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -88.09 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -87.59 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -87.09 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -86.59 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -83.59 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/ 38.16MHz | -51.73 |  | -59.3 |  |  | -84 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -83.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -83 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -82.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -82 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -81.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -81 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -80.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -77.5 |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: CSI-SINR, CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-SINR, CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3.5.2 in TS 38.133 [6]. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.13 to 6.7.16

### 6.7.17	LTM L1-RSRP measurement

#### 6.7.17.0	Minimum conformance requirements

6.7.17.0.1	SSB based absolute L1-RSRP measurement accuracy requirements

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-RSRP in this clause apply to all SSBs of candidate neighbour cell(s) on a frequency in FR1 that is on a different frequency than the serving cell.

The accuracy requirements in Table 6.7.17.0.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency L1-RSRP measurements are fulfilled according to TS 38.133 [6] Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 6.7.17.0.1-1: Inter-frequency L1-RSRP absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | IoNote 1 range |  |  |  |  |
|  |  |  | NR operating band groupsNote 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| ±8.5 | ±11.5 | -3 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H, NR_FDD_FR1_N | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in TS 38.133 [6] clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1.19E.1.1 and 10.1.6.

6.7.17.0.2	SSB based relative L1-RSRP measurement accuracy requirements

The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB from one cell on a frequency in FR1 compared to the largest measured value of L1-RSRP among all SSBs measured from another cell on another frequency in FR1.

The accuracy requirements in Table 6.7.17.0.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for L1-RSRP measurements are fulfilled according to TS 38.133 [6] Annex B.2.4.1 for a corresponding Band for each relevant SSB.

Table 6.7.17.0.2-1: Inter-frequency L1-RSRP relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | IoNote 1 range |  |  |  |  |
|  |  |  | NR operating band groupsNote 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| ±5.0 | ±9.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_N | -114.5 | -111.5 | N/A | -70 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in TS 38.133 [6] clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1.19E.1.2 and 10.1.6.

#### 6.7.17.1	Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1

##### 6.7.17.1.1	NR SA FR1 Inter-frequency L1-RSRP absolute accuracy requirements for neighbour cell

6.7.17.1.1.1	Test purpose

To verify that the inter-frequency SSB based L1-RSRP absolute measurement accuracy on neighbour cell is within the limits for all bands specified in TS 38.133 [6] clause 10.1.19E.

6.7.17.1.1.2	Test applicability

This test applies to all types of NR UE supporting SSB based inter-frequency L1-RSRP measurements with measurement gaps and reports for LTM from release 18 and forward.

6.7.17.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.17.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.17.1.

6.7.17.1.1.4	Test description

6.7.17.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.17.1.1.4.1-1.

Table 6.7.17.1.1.4.1-1: Supported test configurations

| Test Case ID | Description |
| --- | --- |
| 6.7.17.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 6.7.17.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.7.17.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations in each supported band Note 2: Target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Test environment parameters are given in Table 6.7.17.1.1.4.1-2.

Table 6.7.17.1.1.4.1-2: Test environment parameters

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.17.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | -Without LTE link -For 4Rx capable UEs without any 2Rx RF bands use A.3.1.8.4 for TE part and A.3.2.5.2 for DUT part. |  |  |

1.	The general test parameter settings are set up according to Table 6.7.17.1.1.4.1-3.

2.	Message contents are defined in clause 6.7.17.1.1.4.3.

3.	There are two NR carriers and one cell on each carrier in the test. Cell 1 (Pcell) is on NR RF channel 1, and is the cell used for connection setup with the power level set according to Annex C.1.2 and C.1.3 for this test. Cell 2 (inter neighbour cell) is on NR RF channel 2.

Table 6.7.17.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  |  | Cell 1 |  |
| Neighbouring cell |  |  | Cell 2 | Cell 2 is the candidate cell |
| RF Channel Number |  |  | 1: Cell 1 2: Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| reportQuantityRS-Indexes |  |  | rsrp |  |
| maxNrofRS-IndexesToReport |  |  | 1 |  |
| includeBeamMeasurements |  |  | True |  |
| DRX |  |  | OFF | DRX is not used |
| Time offset between cells |  |  | 3 s | The timing of Cell 2 is later than the timing of Cell 1 |
| Gap Pattern Id |  |  | 0 |  |
| Measurement gap offset |  | ms | 39 |  |
| LTM-CSI-ReportConfig | reportConfigType |  | periodic | Periodic L1-RSRP reporting configured |
|  | L1-RSRP reporting period | slot | 80 |  |
|  | nrOfReportedCells |  | n1 | Report candidate cell’s (Cell 2) L1-RSRP measurement results. |
|  | nrOfReportedRS-PerCell |  | n1 |  |
|  | spCellInclusion |  | N/A |  |
| ltm-ConfigComplete |  |  | True | Candidate cell’s configuration is complete configuration |

6.7.17.1.1.4.2	Test procedure

In CSI measurement configuration, UE is indicated to perform inter-frequency L1-RSRP measurement on the SSBs and report periodically.

There are two tests in the test case, test 1 and test 2:

-	In test 1, test at the highest power level below maximum Io.

-	In test 2, test at the lowest power level above minimum SSB_RP.

Measurement gap pattern configuration defined in Table 6.7.17.1.1.4.1-3 is provided.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to Test 1 in Table 6.7.17.1.1.5-1.

3.	SS shall transmit an RRCReconfiguration message to configure periodical measurement report.

4.	The UE shall transmit RRCReconfigurationComplete message.

5.	The UE shall transmit a MeasurementReport message periodically including Cell 2 measurement report with SSB index.

6.	SS shall transmit an RRCReconfiguration message to configure LTM-Candidate-r18 for Cell 2. UE is configured with SSB-based L1-RSRP measurements and periodic L1-RSRP measurement reports for candidate Cell 2 in PUCCH format 2.

7.	The UE shall transmit RRCReconfigurationComplete message.

8.	The UE shall transmit L1-RSRP on PUCCH format 2 in Cell 1 with a periodicity of 80 slots.

9.	After 10s wait from Step 6, the SS shall check the values in the periodic measurement report. The L1-RSRP value of Cell 2 reported by the UE is compared to the expected L1-RSRP value. If the value is outside the limits in Table 6.7.17.1.1.5-2 for test configurations 1, 2 and in Table 6.7.17.1.1.5-3 for test configuration 3, or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

10.	The SS shall continue checking the L1-RSRP report transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

11.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

12.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

13.	After the RRC connection release, the SS:

-	transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),

or:

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

14.	Set the parameters according to each test in Table 6.7.17.1.1.5-1 as appropriate and repeat steps 3-13.

6.7.17.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 and clause 7.3 with the following exceptions:

Table 6.7.17.1.1.4.3-1: Common Exception messages (step 3)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with conditions INTER-FREQ and GAP NEEDED Table H.3.1-5 with condition SSB Index Table H.3.1-6 with conditions gapUE and Pattern #0 Table H.3.1-7 with conditions INTER-FREQ, SSB Index |
| Specific message contents exceptions for Cell 2 | Table H.3.1-3 with condition INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 6.7.17.1.1.4.3-2: ReportConfigNR (periodical) (step3)

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| rsType | ssb |  |  |
| reportInterval | ms480 |  |  |
| reportAmount | infinity |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 1 |  |  |
| reportQuantityRS-Indexes SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.17.1.1.4.3-3: Common Exception messages (step 6)

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.12-1 with condition L1-RSRP Table H.3.12-2 with condition L1-RSRP Table H.3.12-3 Table H.3.12-4 Table H.3.12-6 Table H.3.12-8 |
| Specific message contents exceptiosn for Test Configuration 1 | Table H.3.12-5 with condition SCS15 Table H.3.12-9 with conditions FR1, SSB#0, FDD |
| Specific message contents exceptiosn for Test Configuration 2 | Table H.3.12-5 with condition SCS15 Table H.3.12-9 with conditions FR1, SSB#0, TDD, SCS15 |
| Specific message contents exceptiosn for Test Configuration 3 | Table H.3.12-5 with condition SCS30 Table H.3.12-9 with conditions FR1, SSB#0, TDD, SCS30 |

6.7.17.1.1.5	Test requirement

Table 6.7.17.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Table 6.7.17.1.1.5-1: Cell specific test parameters

| Parameter | Config | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB GSCN | 1~3 |  | freq1 | freq2 | freq1 | freq2 |
| Duplex mode | 1 |  | FDD |  | FDD |  |
|  | 2 |  | TDD |  | TDD |  |
|  | 3 |  | TDD |  | TDD |  |
| TDD Configuration | 1 |  | N/A |  | N/A |  |
|  | 2 |  | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  | 3 |  | TDDConf.2.1 |  | TDDConf.2.1 |  |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 2 |  | 10: NRB,c = 52 |  | 10: NRB,c = 52 |  |
|  | 3 |  | 40: NRB,c = 106 |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
|  | 2 |  | SR.1.1 TDD | - | SR.1.1 TDD | - |
|  | 3 |  | SR.2.1 TDD | - | SR.2.1 TDD | - |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |
|  | 2 |  | CR.1.1 TDD | - | CR.1.1 TDD | - |
|  | 3 |  | CR.2.1 TDD | - | CR.2.1 TDD | - |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
|  | 2 |  | CCR.1.1 TDD | - | CCR.1.1 TDD | - |
|  | 3 |  | CCR.2.1 TDD | - | CCR.2.1 TDD | - |
| SSB configuration | 1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 2 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
|  | 3 |  | SSB.2 FR1 |  | SSB.2 FR1 |  |
| OCNG Patterns | 1~3 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| TRS configuration | 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - |
|  | 2 |  | TRS.1.1 TDD | - | TRS.1.1 TDD | - |
|  | 3 |  | TRS.1.2 TDD | - | TRS.1.2 TDD | - |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~3 |  | SMTC.1 |  | SMTC.1 |  |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| NocNote 2; NR_FDD_FR1_A, NR_TDD_FR1_A Note 5; 1,2; dBm/15kHz; -94.65; -94.65; -109; -117. NR_FDD_FR1_B; -108.5; -116.5. NR_TDD_FR1_C; -108; -116. NR_FDD_FR1_D, NR_TDD_FR1_D; -107.5; -115.5. NR_FDD_FR1_E, NR_TDD_FR1_E; -107; -115. NR_FDD_FR1_F; -106.5; -114.5. NR_FDD_FR1_G; -106; -114. NR_FDD_FR1_H; -105.5; -113.5. NR_FDD_FR1_N; -102.5; -110.5. NR_FDD_FR1_A, NR_TDD_FR1_A Note 5; 3; -96; -96; -109; -117. NR_FDD_FR1_B; -108.5; -116.5. NR_TDD_FR1_C; -108; -116. NR_FDD_FR1_D, NR_TDD_FR1_D; -107.5; -115.5. NR_FDD_FR1_E, NR_TDD_FR1_E; -107; -115. NR_FDD_FR1_F; -106.5; -114.5. NR_FDD_FR1_G; -106; -114. NR_FDD_FR1_H; -105.5; -113.5. NR_FDD_FR1_N; -102.5; -110.5. NocNote 2; NR_FDD_FR1_A, NR_TDD_FR1_A Note 5; 1,2; dBm/SSB SCS; -94.65; -94.65; -109; -117. NR_FDD_FR1_B; -108.5; -116.5. NR_TDD_FR1_C; -108; -116. NR_FDD_FR1_D, NR_TDD_FR1_D; -107.5; -115.5. NR_FDD_FR1_E, NR_TDD_FR1_E; -107; -115. NR_FDD_FR1_F; -106.5; -114.5. NR_FDD_FR1_G; -106; -114. NR_FDD_FR1_H; -105.5; -113.5. NR_FDD_FR1_N; -102.5; -110.5. NR_FDD_FR1_A, NR_TDD_FR1_A Note 5; 3; -93; -93; -106; -114. NR_FDD_FR1_B; -105.5; -113.5. NR_TDD_FR1_C; -105; -113. NR_FDD_FR1_D, NR_TDD_FR1_D; -104.5; -112.5. NR_FDD_FR1_E, NR_TDD_FR1_E; -104; -112. NR_FDD_FR1_F; -103.5; -111.5. NR_FDD_FR1_G; -103; -111. NR_FDD_FR1_H; -102.5; -110.5. NR_FDD_FR1_N; -99.5; -107.5. EsIot; 1~3; dB; 10; 10; 13; -2.7. SSB RSRP Note 3; NR_FDD_FR1_A, NR_TDD_FR1_A Note 5; 1,2; dBm/SSB SCS; -84.65; -84.65; -96; -119.7. NR_FDD_FR1_B; -95.5; -119.2. NR_TDD_FR1_C; -95; -118.7. NR_FDD_FR1_D, NR_TDD_FR1_D; -94.5; -118.2. NR_FDD_FR1_E, NR_TDD_FR1_E; -94; -117.7. NR_FDD_FR1_F; -93.5; -117.1. NR_FDD_FR1_G; -93; -116.7. NR_FDD_FR1_H; -92.5; -116.1. NR_FDD_FR1_N; -89.5; -113.2. NR_FDD_FR1_A, NR_TDD_FR1_A Note 5; 3; -82.99; -82.99; -92.99; -116.69. NR_FDD_FR1_B; -92.49; -116.19. NR_TDD_FR1_C; -91.99; -115.69. NR_FDD_FR1_D, NR_TDD_FR1_D; -91.49; -115.19. NR_FDD_FR1_E, NR_TDD_FR1_E; -90.99; -114.69. NR_FDD_FR1_F; -90.49; -114.09. NR_FDD_FR1_G; -89.99; -113.69. NR_FDD_FR1_H; -89.49; -113.09. NR_FDD_FR1_N; -86.49; -110.19. Io Note 3; NR_FDD_FR1_A, NR_TDD_FR1_A Note 5; 1,2; dBm/9.36 MHz; -56.28; -56.28; -67.84; -87.18. NR_FDD_FR1_B; -67.34; -86.68. NR_TDD_FR1_C; -66.84; -86.18. NR_FDD_FR1_D, NR_TDD_FR1_D; -66.34; -85.68. NR_FDD_FR1_E, NR_TDD_FR1_E; -65.84; -85.18. NR_FDD_FR1_F; -65.34; -84.65. NR_FDD_FR1_G; -64.84; -84.18. NR_FDD_FR1_H; -64.34; -83.65. NR_FDD_FR1_N; -61.34; -80.68. NR_FDD_FR1_A, NR_TDD_FR1_A Note 5; 3; dBm/38.16 MHz; -51.53; -51.53; -61.73; -81.08. NR_FDD_FR1_B; -61.23; -80.58. NR_TDD_FR1_C; -60.73; -80.08. NR_FDD_FR1_D, NR_TDD_FR1_D; -60.23; -79.58. NR_FDD_FR1_E, NR_TDD_FR1_E; -59.73; -79.08. NR_FDD_FR1_F; -59.23; -78.54. NR_FDD_FR1_G; -58.73; -78.08. NR_FDD_FR1_H; -58.23; -77.54. NR_FDD_FR1_N; -55.23; -74.58. EsNoc; 1~3; dB; 10; 10; 13; -2.7. |
| Propagation condition | 1~3 |  | AWGN |  | AWGN |  |
| Antenna configuration | 1~3 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51, and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |

Each L1-RSRP measurement report of Cell 2 in each test in Table 6.7.17.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 6.7.17.1.1.5-2 for test configurations 1 and 2, and the corresponding absolute accuracy requirements in Table 6.7.17.1.1.5-3 for test configuration 3.

Table 6.7.17.1.1.5-2: L1-RSRP absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 30 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 31 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 32 |
|  |  | Bands NR_FDD_FR1_F | 33 |
|  |  | Bands NR_FDD_FR1_G | 33 |
|  |  | Bands NR_FDD_FR1_H | 34 |
|  |  | Bands NR_FDD_FR1_N | 37 |
| Highest reported value (Cell 2) | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 43 |
|  |  | Bands NR_FDD_FR1_B | 44 |
|  |  | Bands NR_TDD_FR1_C | 44 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 45 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 45 |
|  |  | Bands NR_FDD_FR1_F | 46 |
|  |  | Bands NR_FDD_FR1_G | 46 |
|  |  | Bands NR_FDD_FR1_H | 47 |
|  |  | Bands NR_FDD_FR1_N | 50 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 26 |
|  |  | Bands NR_FDD_FR1_B | 26 |
|  |  | Bands NR_TDD_FR1_C | 27 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 27 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 28 |
|  |  | Bands NR_FDD_FR1_F | 28 |
|  |  | Bands NR_FDD_FR1_G | 29 |
|  |  | Bands NR_FDD_FR1_H | 29 |
|  |  | Bands NR_FDD_FR1_N | 32 |
| Highest reported value (Cell 2) | 85 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 48 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 49 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 50 |
|  |  | Bands NR_FDD_FR1_F | 50 |
|  |  | Bands NR_FDD_FR1_G | 51 |
|  |  | Bands NR_FDD_FR1_H | 51 |
|  |  | Bands NR_FDD_FR1_N | 54 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1. |  |  |  |

Table 6.7.17.1.1.5-3: L1-RSRP absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 63 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  | Bands NR_FDD_FR1_B | 34 |
|  |  | Bands NR_TDD_FR1_C | 34 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
|  |  | Bands NR_FDD_FR1_F | 36 |
|  |  | Bands NR_FDD_FR1_G | 36 |
|  |  | Bands NR_FDD_FR1_H | 37 |
|  |  | Bands NR_FDD_FR1_N | 40 |
| Highest reported value (Cell 2) | 84 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 46 |
|  |  | Bands NR_FDD_FR1_B | 47 |
|  |  | Bands NR_TDD_FR1_C | 47 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 48 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 48 |
|  |  | Bands NR_FDD_FR1_F | 49 |
|  |  | Bands NR_FDD_FR1_G | 49 |
|  |  | Bands NR_FDD_FR1_H | 50 |
|  |  | Bands NR_FDD_FR1_N | 53 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 60 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 29 |
|  |  | Bands NR_FDD_FR1_B | 29 |
|  |  | Bands NR_TDD_FR1_C | 30 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 30 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 31 |
|  |  | Bands NR_FDD_FR1_F | 31 |
|  |  | Bands NR_FDD_FR1_G | 32 |
|  |  | Bands NR_FDD_FR1_H | 32 |
|  |  | Bands NR_FDD_FR1_N | 35 |
| Highest reported value (Cell 2) | 87 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 51 |
|  |  | Bands NR_FDD_FR1_B | 51 |
|  |  | Bands NR_TDD_FR1_C | 52 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 53 |
|  |  | Bands NR_FDD_FR1_F | 53 |
|  |  | Bands NR_FDD_FR1_G | 54 |
|  |  | Bands NR_FDD_FR1_H | 54 |
|  |  | Bands NR_FDD_FR1_N | 57 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-1. |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

##### 6.7.17.1.2	Void

### 6.7.18	TDCP amplitude measurement accuracy

#### 6.7.18.0	Minimum conformance requirements

6.7.18.0.1	TDCP amplitude measurement accuracy requirements in FR1

[38.133 clause 10.1.42]

The reporting range of TDCP amplitude is defined from 0 to 1. The reporting range of TDCP phase is 0 to 2. The mapping of measured quantity is defined in Tables 6.7.18.0.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table  6.7.18.0.1-1: TDCP phase measurement report mapping

| Reported value | Measured quantity value |
| --- | --- |
| TDCP_PH_0 | 0 ≤ TDCP_PH < 2π16 |
| TDCP_PH_1 | 2π16≤ TDCP_PH < 2π*216 |
| TDCP_PH_2 | 2π*216 ≤ TDCP_PH < 2π*316 |
| TDCP_PH_3 | 2π*316 ≤ TDCP_PH < 2π*416 |
| TDCP_PH_4 | 2π*416 ≤ TDCP_PH < 2π*516 |
| TDCP_PH_5 | 2π*516≤ TDCP_PH < 2π*616 |
| TDCP_PH_6 | 2π*616 ≤ TDCP_PH < 2π*716 |
| TDCP_PH_7 | 2π*716 ≤ TDCP_PH < 2π*816 |
| TDCP_PH_8 | 2π*816 ≤ TDCP_PH < 2π*916 |
| TDCP_PH_9 | 2π*916 ≤ TDCP_PH < 2π*1016 |
| TDCP_PH_10 | 2π*1016 ≤ TDCP_PH < 2π*1116 |
| TDCP_PH_11 | 2π*1116≤ TDCP_PH < 2π*1216 |
| TDCP_PH_12 | 2π*1216 ≤ TDCP_PH < 2π*1316 |
| TDCP_PH_13 | 2π*1316 ≤ TDCP_PH < 2π*1416 |
| TDCP_PH_14 | 2π*1416 ≤ TDCP_PH < 2π*1516 |
| TDCP_PH_15 | 2π*1516 ≤ TDCP_PH ≤ 2π |

#### 6.7.18.1	TDCP amplitude measurement accuracy in EN-DC

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing

- 	test case applicability is missing

6.7.18.1.1	Test purpose

The purpose of this test is to verify that the TRS based TDCP amplitude measurement accuracy is within the specified limits in the test requirements section clause 10.1.42 of TS 38.133 [6]. The cofigurations for the test are specified in Table 6.7.18.1.1-1.

6.7.18.1.2	Test applicability

This test applies to all types of NR UE Rel-18 and forward supporting NR EN-DC, which supportsT based SINR measurements.

6.7.18.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in Clause 6.7.18.0.2.

The normative reference for this requirement is in TS 38.133 [6] A. 6.7.18.

6.7.18.1.4	Test description

6.7.18.1.4.1	Initial conditions

The test consists of two tests, Test 1 and Test 2. Each test further consists of two subtests Test 1A, 1B and Test 2A, 2B.

Test 1A: 10 Hz doppler + 15kHz SCS FDD + 20dB SNR

Test 1B: 10 Hz doppler + 30kHz SCS TDD + 20 dB SNR

Test 2A: 300 Hz doppler + 15kHz SCS FDD + 10 dB SNR

Test 2B: 300 Hz doppler + 30kHz SCS TDD + 10 dB SNR

Relevant parameters for each test are provided in the table 6.7.18.1.2-1. UE needs to pass Test 1A, 1B, 2A, 2B.

Table 6.7.18.1.4-1: Applicable NR configurations for FR1 TRS based TDCP test

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 20 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 50 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.18.1.4.1-2.

Table 6.7.18.1.4.1-2: Initial conditions for CSI-SINR inter-frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] Clause 4.1. |
| Test frequencies | As specified in Annex E, E.1.1, E.1.3.2, Table E.2-1 and TS 38.508-1 [14] Clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.10.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Clause C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

6.7.18.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] Clause 4.5.

2.	Set the parameters according to Table 6.7.18.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the TDCP reported index in the periodic MeasurementReport. The TDCP reported index of Cell 2 reported by the UE is compared to the expected TDCP reported index. If the value is outside the limits (determined by Table 6.7.18.0.2.1-1) or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 6.7.18.1.5-1 as appropriate and repeat steps 5-7.

6.7.18.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] Clause 7.3 with the following exceptions.

6.7.18.1.5	Test Requirements

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2). The test parameters and applicability for Cell 1 are defined in Clause A.3.7.2 of TS 38.133 [6]. The test parameters for the Cell 2 are given in Table 6.7.18.1.5-1. Ampliutude of TDCP is tested by using the parameters in Table 6.7.18.1.5-1.

There is no measurement gap configured in the test. Before the test, UE is configured with 1 TRS set with the TRS resources in the set are configured in adjacent slot. UE is configured to perform L1-RSRP measurement based on the SSB resources 0 and 1.

Table 6.7.18.1.5-1: FR1 TRS based TDCP test parameters

| Parameter | Config | Unit | Test 1 (FDD) |  | Test 2 (TDD) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1A | 1B | 2A | 2B |
| SSB GSCN | 1~4 |  | freq1 |  | freq1 |  |
| Duplex mode | 1,3 |  | FDD |  | N/A |  |
|  | 2,4 |  | N/A |  | TDD |  |
| TDD Configuration | 1,3 |  | N/A |  | N/A |  |
|  | 2,4 |  | N/A |  | TDDConf.1.1 |  |
| BWchannel | 1,3 | MHz | 10: NRB,c = 52 |  | N/A |  |
|  | 2,4 |  | N/A |  | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel | 1,3 |  | SR.1.1 FDD |  | N/A |  |
|  | 2,4 |  | N/A |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | 1,3 |  | CR.1.1 FDD |  | N/A |  |
|  | 2,4 |  | N/A |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | 1,3 |  | CCR.1.1 FDD |  | N/A |  |
|  | 2,4 |  | N/A |  | CCR.2.1 TDD |  |
| SSB configuration | 1,3 |  | SSB.3 FR1 |  | N/A |  |
|  | 2,4 |  | N/A |  | SSB.4 FR1 |  |
| OCNG Patterns | 1~4 |  | OP.1 |  | OP.1 |  |
| Initial BWP Configuration | 1~4 |  | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| TRS configuration | 1,3 |  | TRS.1.5 FDD |  | N/A |  |
|  | 2,4 |  | N/A |  | TRS.1.5 TDD |  |
| Dedicated BWP configuration | 1~4 |  | DLBWP.1.1 ULBWP.1.1 |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration | 1~4 |  | SMTC.1 |  | SMTC.1 |  |
| reportConfigType | 1~4 |  | periodic |  | periodic |  |
| reportQuantity | 1~4 |  | ssb-Index-RSRP |  | ssb-Index-RSRP |  |
| Number of reported RS | 1~4 |  | 2 |  | 2 |  |
| L1-RSRP reporting period | 1~4 |  | slot80 |  | slot80 |  |
| EPRE ratio of PSS to SSS | 1~4 | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Noc | dBm/15kHz | 1 | -98 |  | -98 |  |
|  |  | 2 |  |  |  |  |
| Noc Note 2 | dBm/SCS | 1 | -98 |  | -98 |  |
|  |  | 2 | -95 |  | -95 |  |
| EsIot | 1~4 | dB | TBD |  | TBD |  |
| EsNoc | 1~4 | dB | 20 | 10 | 20 | 10 |
| SSB_RP | 1,4 |  | -78 | -88 | -78 | -88 |
| Propagation condition | 1~4 |  | TDL-A 30ns 10 Hz | TDL-A 30ns 300 Hz | TDL-A 30ns 10Hz | TDL-A 30ns 300 Hz |
| Antenna configuration | 1~4 |  | 1x2 |  | 1x2 |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. |  |  |  |  |  |  |

For Test 1A, the reported TDCP index shall be smaller than or equal to 6 for the 80% of the times over repeated tests

For Test 1B: the reported TDCP index shall be smaller than or equal to 5 for the 80% of the times over repeated tests

For Test 2A: the reported TDCP index shall be larger than 8 for the 80% of the times over repeated tests

For Test 2B: the reported TDCP index shall be larger than 6 for the 80% of the times over repeated tests