| 3GPP TR 38.898 V0.4.0 (2023-05) |  |
| --- | --- |
| Technical Report |  |
| High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Background	10

4.1	TR maintenance	11

5	EN-DC Power Class 2: Specific Band Combination Part	11

5.1	DC_1_n79	11

5.1.1	Configuration for DC	11

5.1.2	Maximum output power for DC	11

5.1.3	REFSENS requirements for DC	11

5.1.4	∆TIB and ∆RIB values	11

5.2	DC_3_n79	11

5.2.1	Configuration for DC	11

5.2.2	Maximum output power for DC	12

5.2.3	REFSENS requirements for DC	12

5.2.4	∆TIB and ∆RIB values	12

5.3	DC_19_n79	12

5.3.1	Configuration for DC	12

5.3.2	Maximum output power for DC	12

5.3.3	REFSENS requirements for DC	12

5.3.4	∆TIB and ∆RIB values	13

5.4	DC_21_n79	13

5.4.1	Configuration for DC	13

5.4.2	Maximum output power for DC	13

5.4.3	REFSENS requirements for DC	13

5.4.4	∆TIB and ∆RIB values	14

5.5	DC_1_n77-n79	15

5.5.1	Configuration for DC	15

5.5.2	Maximum output power for DC	15

5.5.3	REFSENS requirements for DC	15

5.5.4	∆TIB and ∆RIB values	15

5.6	DC_3_n77-n79	15

5.6.1	Configuration for DC	15

5.6.2	Maximum output power for DC	16

5.6.3	REFSENS requirements for DC	16

5.6.4	∆TIB and ∆RIB values	16

5.7	DC_21_n77-n79	16

5.7.1	Configuration for DC	16

5.7.2	Maximum output power for DC	16

5.7.3	REFSENS requirements for DC	16

5.7.4	∆TIB and ∆RIB values	17

5.8	DC_1_n78-n79	17

5.8.1	Configuration for DC	17

5.8.2	Maximum output power for DC	17

5.8.3	REFSENS requirements for DC	17

5.8.4	∆TIB and ∆RIB values	18

5.9	DC_3_n78-n79	18

5.9.1	Configuration for DC	18

5.9.2	Maximum output power for DC	18

5.9.3	REFSENS requirements for DC	18

5.9.4	∆TIB and ∆RIB values	19

5.10	DC_21_n78-n79	19

5.10.1	Configuration for DC	19

5.10.2	Maximum output power for DC	19

5.10.3	REFSENS requirements for DC	19

5.10.4	∆TIB and ∆RIB values	20

5.11	DC_1-21_n77	20

5.11.1	Configuration for DC	20

5.11.2	Maximum output power for DC	20

5.11.3	REFSENS requirements for DC	20

5.11.4	∆TIB and ∆RIB values	21

5.12	DC_1-42_n77	21

5.12.1	Configuration for DC	21

5.12.2	Maximum output power for DC	21

5.12.3	REFSENS requirements for DC	22

5.12.4	∆TIB and ∆RIB values	22

5.13	DC_3-21_n77	22

5.13.1	Configuration for DC	22

5.13.2	Maximum output power for DC	22

5.13.3	REFSENS requirements for DC	22

5.13.4	∆TIB and ∆RIB values	23

5.14	DC_3-42_n77	23

5.14.1	Configuration for DC	23

5.14.2	Maximum output power for DC	23

5.14.3	REFSENS requirements for DC	23

5.14.4	∆TIB and ∆RIB values	24

5.15	DC_21-42_n77	24

5.15.1	Configuration for DC	24

5.15.2	Maximum output power for DC	24

5.15.3	REFSENS requirements for DC	24

5.15.4	∆TIB and ∆RIB values	24

5.16	DC_1_n77	25

5.16.1	Configuration for DC	25

5.16.2	Maximum output power for DC	25

5.16.3	REFSENS requirements for DC	25

5.16.4	∆TIB and ∆RIB values	26

5.17	DC_3_n77	26

5.17.1	Configuration for DC	26

5.17.2	Maximum output power for DC	26

5.17.3	REFSENS requirements for DC	26

5.17.4	∆TIB and ∆RIB values	27

5.18	DC_21_n77	27

5.18.1	Configuration for DC	27

5.18.2	Maximum output power for DC	27

5.18.3	REFSENS requirements for DC	27

5.18.4	∆TIB and ∆RIB values	28

5.19	DC_21_n78	28

5.19.1	Configuration for DC	28

5.19.2	Maximum output power for DC	28

5.19.3	REFSENS requirements for DC	28

5.19.4	∆TIB and ∆RIB values	28

5.20	DC_1-3_n77	29

5.20.1	Configuration for DC	29

5.20.2	Maximum output power for DC	29

5.20.3	REFSENS requirements for DC	29

5.20.4	∆TIB and ∆RIB values	30

5.23	DC_1-3_n78	33

5.23.1	Configuration for DC	33

5.23.2	Maximum output power for DC	33

5.23.3	REFSENS requirements for DC	33

5.23.4	∆TIB and ∆RIB values	33

5.24	DC_1-42_n78	34

5.24.1	Configuration for DC	34

5.24.2	Maximum output power for DC	34

5.24.3	REFSENS requirements for DC	34

5.24.4	∆TIB and ∆RIB values	34

5.25	DC_3-42_n78	35

5.25.1	Configuration for DC	35

5.25.2	Maximum output power for DC	35

5.25.3	REFSENS requirements for DC	35

5.25.4	∆TIB and ∆RIB values	35

5.26	DC_21-42_n78	36

5.26.1	Configuration for DC	36

5.26.2	Maximum output power for DC	36

5.26.3	REFSENS requirements for DC	36

5.26.4	∆TIB and ∆RIB values	36

5.27	DC_1-3_n79	36

5.27.1	Configuration for DC	36

5.27.2	Maximum output power for DC	37

5.27.3	REFSENS requirements for DC	37

5.27.4	∆TIB and ∆RIB values	37

5.28	DC_1-19_n79	37

5.28.1	Configuration for DC	37

5.28.2	Maximum output power for DC	37

5.28.3	REFSENS requirements for DC	38

5.28.4	∆TIB and ∆RIB values	38

5.29	DC_1-21_n79	38

5.29.1	Configuration for DC	38

5.29.2	Maximum output power for DC	38

5.29.3	REFSENS requirements for DC	39

5.29.4	∆TIB and ∆RIB values	39

5.30	DC_1-42_n79	39

5.30.1	Configuration for DC	39

5.30.2	Maximum output power for DC	40

5.30.3	REFSENS requirements for DC	40

5.30.4	∆TIB and ∆RIB values	40

5.31	DC_3-19_n79	40

5.31.1	Configuration for DC	40

5.31.2	Maximum output power for DC	40

5.31.3	REFSENS requirements for DC	41

5.31.4	∆TIB and ∆RIB values	41

5.32	DC_3-21_n79	41

5.32.1	Configuration for DC	41

5.32.2	Maximum output power for DC	41

5.32.3	REFSENS requirements for DC	42

5.32.4	∆TIB and ∆RIB values	42

5.33	DC_3-42_n79	42

5.33.1	Configuration for DC	42

5.33.2	Maximum output power for DC	43

5.33.3	REFSENS requirements for DC	43

5.33.4	∆TIB and ∆RIB values	43

5.34	DC_19-21_n79	43

5.34.1	Configuration for DC	43

5.34.2	Maximum output power for DC	43

5.34.3	REFSENS requirements for DC	44

5.34.4	∆TIB and ∆RIB values	44

5.35	DC_19-42_n79	44

5.35.1	Configuration for DC	44

5.35.2	Maximum output power for DC	45

5.35.3	REFSENS requirements for DC	45

5.35.4	∆TIB and ∆RIB values	45

5.36	DC_21-42_n79	45

5.36.1	Configuration for DC	45

5.36.2	Maximum output power for DC	45

5.36.3	REFSENS requirements for DC	46

5.36.4	∆TIB and ∆RIB values	46

5.39	DC_19_n77	48

5.39.1	Configuration for DC	48

5.39.2	Maximum output power for DC	48

5.39.3	REFSENS requirements for DC	48

5.39.4	∆TIB and ∆RIB values	49

5.40	DC_19_n78	49

5.40.1	Configuration for DC	49

5.40.2	Maximum output power for DC	50

5.40.3	REFSENS requirements for DC	50

5.40.4	∆TIB and ∆RIB values	51

5.41	DC_1-19_n77	51

5.41.1	Configuration for DC	51

5.41.2	Maximum output power for DC	51

5.41.3	REFSENS requirements for DC	51

5.41.4	∆TIB and ∆RIB values	52

5.42	DC_1-19_n78	52

5.42.1	Configuration for DC	52

5.42.2	Maximum output power for DC	52

5.42.3	REFSENS requirements for DC	52

5.42.4	∆TIB and ∆RIB values	53

5.43	DC_3-19_n77	53

5.43.1	Configuration for DC	53

5.43.2	Maximum output power for DC	53

5.43.3	REFSENS requirements for DC	53

5.43.4	∆TIB and ∆RIB values	54

5.44	DC_3-19_n78	54

5.44.1	Configuration for DC	54

5.44.2	Maximum output power for DC	54

5.44.3	REFSENS requirements for DC	54

5.44.4	∆TIB and ∆RIB values	55

5.45	DC_19-21_n77	55

5.45.1	Configuration for DC	55

5.45.2	Maximum output power for DC	55

5.45.3	REFSENS requirements for DC	55

5.45.4	∆TIB and ∆RIB values	56

5.46	DC_19-21_n78	56

5.46.1	Configuration for DC	56

5.46.2	Maximum output power for DC	56

5.46.3	REFSENS requirements for DC	56

5.46.4	∆TIB and ∆RIB values	57

5.47	DC_19-42_n77	57

5.47.1	Configuration for DC	57

5.47.2	Maximum output power for DC	57

5.47.3	REFSENS requirements for DC	58

5.47.4	∆TIB and ∆RIB values	58

5.48	DC_19-42_n78	58

5.48.1	Configuration for DC	58

5.48.2	Maximum output power for DC	58

5.48.3	REFSENS requirements for DC	58

5.48.4	∆TIB and ∆RIB values	58

5.49	DC_19_n77-n79	59

5.49.1	Configuration for DC	59

5.49.2	Maximum output power for DC	59

5.49.3	REFSENS requirements for DC	59

5.49.4	∆TIB and ∆RIB values	59

5.50	DC_19_n78-n79	59

5.50.1	Configuration for DC	59

5.50.2	Maximum output power for DC	59

5.50.3	REFSENS requirements for DC	60

5.50.4	∆TIB and ∆RIB values	60

5.51	DC_1-21_n78	60

5.51.1	Configuration for DC	60

5.51.2	Maximum output power for DC	60

5.51.3	REFSENS requirements for DC	61

5.51.4	∆TIB and ∆RIB values	61

5.52	DC_3-21_n78	62

5.52.1	Configuration for DC	62

5.52.2	Maximum output power for DC	62

5.52.3	REFSENS requirements for DC	62

5.52.4	∆TIB and ∆RIB values	63

5.53	DC_3_n1-n78, DC_3-3_n1-n78	63

5.53.1	Configuration for DC	63

5.53.2	Maximum output power for DC	63

5.53.3	REFSENS requirements for DC	63

5.53.4	∆TIB and ∆RIB values	64

5.54	DC_7_n1-n78, DC_7-7_n1-n78	64

5.54.1	Configuration for DC	64

5.54.2	Maximum output power for DC	64

5.54.3	REFSENS requirements for DC	64

5.54.4	∆TIB and ∆RIB values	65

Annex A - Change history	66

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for High power UE (power class m with 1<m<3) for a single FR1 band in UL of Dual Connectivity (DC) combinations of x bands (x=1,2,3, 4 for y=1 or x=1, 2 for y=2) LTE inter-band CA (xDL/1UL) and y bands NR inter-band CA (yDL/1UL) in the Rel-18 time frame. The purpose is to gather the relevant background information and studies to address relevant requirements for the Rel-18 EN-DC HPUE band combinations requested by proponents and captured in the WID.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-222660, “New WID on High power UE for FR1 for DC_R18_xBLTE_yBNR_zDLnUL”, RAN#97-e

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for EN-DC HPUE band combinations in the Rel-18 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	EN-DC Power Class 2: Specific Band Combination Part

### 5.1	DC_1_n79

#### 5.1.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.1.2	Maximum output power for DC

Table 5.1.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n79A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.1.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_1_n79, the co-existence study is provided in TR 37.863-01-01 [1]. Based on above,

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of band 1.

- the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of band 1.

- the 2nd, 3rd, 4th, and 5th order IMD do not fall into Rx frequencies of band 1 and n79.

Therefore, there is no MSD issue for this DC configuration.

#### 5.1.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.2	DC_3_n79

#### 5.2.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.2.2	Maximum output power for DC

Table 5.2.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_3A_n79A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.2.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_3_n79, the co-existence study is provided in TR 37.863-01-01 [1]. Based on above,

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of band 3.

- the 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of band 3.

- the 2nd, 3rd, 4th, and 5th order IMD do not fall into Rx frequencies of band 3 and n79.

Therefore, there is no MSD issue for this DC configuration.

#### 5.2.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.3	DC_19_n79

#### 5.3.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.3.2	Maximum output power for DC

Table 5.3.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_19A_n79A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.3.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_19_n79, the co-existence study is provided in TR 37.863-01-01. [1] Based on above,

- the 5th order harmonic mixing may fall into Rx frequencies of band 19.

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of band 19.

- the 2nd, 3rd, and 4th order harmonic mixing do not fall into Rx frequencies of band 19.

- the 2nd, 3rd, 4th, and 5th order IMD do not fall into Rx frequencies of band 19 and n79.

For MSD due to 5th harmonic mixing, MSD value of PC2 case will be 3dB higher than that of PC3 case. New MSD values are shown in Table 5.3.3-1 below. Uplink configuration is shown in Table 5.3.3-2 below.

Table 5.3.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n79 | 191 | 32.5 | 29.5 | 27.7 |  |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.3.3-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |

#### 5.3.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.4	DC_21_n79

#### 5.4.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.4.2	Maximum output power for DC

Table 5.4.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_21A_n79A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.4.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_21_n79, the co-existence study is provided in TR 37.863-01-01. [1] Based on above,

- the 3rd order harmonic mixing may fall into Rx frequencies of band 21.

- the 3rd order IMD may fall into Rx frequencies of band 21.

- the 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of band 21.

- the 2nd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of band 21.

- the 2nd, 4th, and 5th order IMD do not fall into Rx frequencies of band 21 and n79.

For MSD due to 3rd harmonic mixing, MSD value of PC2 case will be 3dB higher than that of PC3 case. New MSD values are shown in Table 5.4.3-1 below. Uplink configuration is shown in Table 5.4.3-2 below.

For MSD due to 3rd IMD, the MSD value can be seen as dB related to 1st order proportional of n79 UL power + 2nd order proportional of B21 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Based on these, B21 UL power of PC2 case is 3dB higher than that of PC3 case, and n79 UL power of PC2 case is 9dB higher than that of PC3 case. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case. New MSD values are shown in Table 5.4.3-3 below.

Table 5.4.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n79 | 213 | 42.3 | 39.3 | 37.5 |  |  |  |  |  |  |  |  |
| NOTE 3: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.4.3-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |

Table 5.4.3-3: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n79A | 21 | 1457.5 | 5 | 25 | 1505.5 | 33.4 | IMD3 |
|  | n79 | 4420.5 | 10 | 50 | 4420.5 | N/A | N/A |

#### 5.4.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.5	DC_1_n77-n79

#### 5.5.1	Configuration for DC

Table 5.5.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n77A-n79A14,X | DC_1A_n77A14 DC_1A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE X: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. |  |

#### 5.5.2	Maximum output power for DC

Based on studies of PC2 DC_1_n77 and PC2 DC_1_n79, this section can be omitted.

#### 5.5.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n77 and DC_1_n79 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 3rd, 4th, and 5th order IMD generated by dual uplink of band 1 and band n77 may also fall into own Rx of band n79.

- the 5th order IMD generated by dual uplink of band 1 and band n79 may also fall into own Rx of band n77.

However, IMD will not be an issue because the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. Therefore, there is no MSD issue for this DC configuration.

#### 5.5.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.6	DC_3_n77-n79

#### 5.6.1	Configuration for DC

Table 5.6.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n77A-n79A14,X | DC_3A_n77A14 DC_3A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE X: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. |  |

#### 5.6.2	Maximum output power for DC

Based on studies of PC2 DC_3_n77 and PC2 DC_3_n79, this section can be omitted.

#### 5.6.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n77 and DC_3_n79 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 3rd and 4th order IMD generated by dual uplink of band 3 and band n77 may also fall into own Rx of band n79.

- the 5th order IMD generated by dual uplink of band 3 and band n79 may also fall into own Rx of band n77.

However, IMD will not be an issue because the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. Therefore, there is no MSD issue for this DC configuration.

#### 5.6.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.7	DC_21_n77-n79

#### 5.7.1	Configuration for DC

Table 5.7.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_21A_n77A-n79A14,X | DC_21A_n77A14 DC_21A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE X: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. |  |

#### 5.7.2	Maximum output power for DC

Based on studies of PC2 DC_21_n77 and PC2 DC_21_n79, this section can be omitted.

#### 5.7.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_21_n77 and DC_21_n79 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 2nd and 4th order IMD generated by dual uplink of band 21 and band n77 may also fall into own Rx of band n79.

- the 2nd order IMD generated by dual uplink of band 21 and band n79 may also fall into own Rx of band n77.

However, IMD will not be an issue because the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. Therefore, there is no MSD issue for this DC configuration.

#### 5.7.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.8	DC_1_n78-n79

#### 5.8.1	Configuration for DC

Table 5.8.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n78A-n79A14,X | DC_1A_n78A14 DC_1A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE X: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. |  |

#### 5.8.2	Maximum output power for DC

Based on studies of PC2 DC_1_n78 and PC2 DC_1_n79, this section can be omitted.

#### 5.8.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n78 and DC_1_n79 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 3rd and 5th order IMD generated by dual uplink of band 1 and band n78 may also fall into own Rx of band n79.

- the 5th order IMD generated by dual uplink of band 1 and band n79 may also fall into own Rx of band n78.

For MSD due to 3rd order IMD generated by dual uplink of band 1 and band n78, the MSD value can be seen as dB related to 1st order proportional of band 1 UL power + 2nd order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case. New MSD value is shown in Table 5.8.3-1 below.

Also, For MSD due to 5th order IMD generated by dual uplink of band 1 and band n79, the MSD value can be seen as dB related to 4th order proportional of band 1 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 18dB higher than that of PC3 case. New MSD value is shown in Table 5.8.3-1 below.

Table 5.8.3-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 10 | 50 | 4870 | 24.9 | IMD31 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 22.6 | IMD5 |
|  | n79 | 4670 | 10 | 50 | 4670 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.8.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.9	DC_3_n78-n79

#### 5.9.1	Configuration for DC

Table 5.9.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n78A-n79A14,X | DC_3A_n78A14 DC_3A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE X: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. |  |

#### 5.9.2	Maximum output power for DC

Based on studies of PC2 DC_3_n78 and PC2 DC_3_n79, this section can be omitted.

#### 5.9.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n78 and DC_3_n79 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 3rd order IMD generated by dual uplink of band 3 and band n78 may also fall into own Rx of band n79.

- the 5th order IMD generated by dual uplink of band 3 and band n79 may also fall into own Rx of band n78.

For MSD due to 3rd order IMD generated by dual uplink of band 3 and band n78, the MSD value can be seen as dB related to 1st order proportional of band 3 UL power + 2nd order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case. New MSD value is shown in Table 5.9.3-1 below.

Also, For MSD due to 5th order IMD generated by dual uplink of band 1 and band n79, the MSD value can be seen as dB related to 3rd order proportional of band 1 UL power + 2nd order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 21dB higher than that of PC3 case. New MSD value is shown in Table 5.9.3-1 below.

Table 5.9.3-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | n79 | 4910 | 10 | 50 | 4910 | 25.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 25.2 | IMD5 |
|  | n79 | 4510 | 10 | 50 | 4510 | N/A | N/A |

#### 5.9.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.10	DC_21_n78-n79

#### 5.10.1	Configuration for DC

Table 5.10.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_21A_n78A-n79A14,X | DC_21A_n78A14 DC_21A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE X: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable EN-DC configuration is part of a higher order configuration. |  |

#### 5.10.2	Maximum output power for DC

Based on studies of PC2 DC_21_n78 and PC2 DC_21_n79, this section can be omitted.

#### 5.10.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_21_n78 and DC_21_n79 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 2nd and 4th order IMD generated by dual uplink of band 21 and band n78 may also fall into own Rx of band n79.

- the 2nd order IMD generated by dual uplink of band 21 and band n79 may also fall into own Rx of band n78.

For MSD due to 2nd order IMD generated by dual uplink of band 21 and band n78, the MSD value can be seen as dB related to 1st order proportional of band 21 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case. New MSD value is shown in Table 5.10.3-1 below.

Also, For MSD due to 2nd order IMD generated by dual uplink of band 21 and band n79, the MSD value can be seen as dB related to 1st order proportional of band 21 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case. New MSD value is shown in Table 5.10.3-1 below.

Table 5.10.3-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
|  | n79 | 4873 | 10 | 50 | 4873 | 36.1 | IMD25 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 38.8 | IMD2 |
|  | n79 | 4940 | 10 | 50 | 4940 | N/A | N/A |
| NOTE 5: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.10.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.11	DC_1-21_n77

#### 5.11.1	Configuration for DC

Table 5.11.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-21A_n77A5,14 | DC_1A_n77A14 DC_21A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.11.2	Maximum output power for DC

Based on studies of PC2 DC_1_n77 and PC2 DC_21_n77, this section can be omitted.

#### 5.11.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n77 and DC_21_n77 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 2nd and 5th order IMD generated by dual uplink of band 1 and band n77 may also fall into own Rx of band 21.

- the 2nd and 5th order IMD generated by dual uplink of band 21 and band n77 may also fall into own Rx of band 1.

Considering that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1940-1960/2130-2150 MHz and Band n77 UL/DL = 3600-4200/3600-4200 MHz. Then own Rx impact can be simplified as below.

- the 5th order IMD generated by dual uplink of band 1 and band n77 may also fall into own Rx of band 21.

- the 2nd and 5th order IMD generated by dual uplink of band 21 and band n77 may also fall into own Rx of band 1.

For MSD due to 5th order IMD generated by dual uplink of band 1 and band n77, the MSD value can be seen as dB related to 3rd order proportional of band 1 UL power + 2nd order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case. New MSD value is shown in Table 5.11.3-1 below.

Also, For MSD due to 2nd order IMD generated by dual uplink of band 21 and band n77, the MSD value can be seen as dB related to 1st order proportional of band 21 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case. New MSD value is shown in Table 5.11.3-1 below.

Table 5.11.3-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-21A_n77A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 17.9 | IMD5 |
|  | n77 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | 1964.6 | 5 | 25 | 2154.6 | 36.6 | IMD21 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3605 | 10 | 50 | 3605 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.11.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.12	DC_1-42_n77

#### 5.12.1	Configuration for DC

Table 5.12.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-42A_n77A14,15,16 DC_1A-42C_n77A14,15,16 DC_1A-42D_n77A14,15,16 DC_1A-42E_n77A14,15,16 | DC_1A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.12.2	Maximum output power for DC

Based on studies of PC2 DC_1_n77, this section can be omitted.

#### 5.12.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n77 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 4th and 5th order IMD generated by dual uplink of band 1 and band n77 may also fall into own Rx of band 42.

However, IMD will not be an issue because there is non-simultaneous Rx/Tx operation between band 42 and band n77. Therefore, there is no MSD issue for this DC configuration.

#### 5.12.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.13	DC_3-21_n77

#### 5.13.1	Configuration for DC

Table 5.13.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-21A_n77A5,14 | DC_3A_n77A14 DC_21A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.13.2	Maximum output power for DC

Based on studies of PC2 DC_3_n77 and PC2 DC_21_n77, this section can be omitted.

#### 5.13.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n77 and DC_21_n77 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 4th and 5th order IMD generated by dual uplink of band 3 and band n77 may also fall into own Rx of band 21.

- the 2nd and 5th order IMD generated by dual uplink of band 21 and band n77 may also fall into own Rx of band 3.

Considering that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n77 UL/DL = 3600-4200/3600-4200 MHz. Therefore, own Rx impact can be simplified as below.

- the 4th order IMD generated by dual uplink of band 3 and band n77 may also fall into own Rx of band 21.

- the 5th order IMD generated by dual uplink of band 21 and band n77 may also fall into own Rx of band 3.

For MSD due to 4th order IMD generated by dual uplink of band 3 and band n77, the MSD value can be seen as dB related to 3rd order proportional of band 3 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 12dB higher than that of PC3 case. New MSD value is shown in Table 5.13.3-1 below.

Also, For MSD due to 5th order IMD generated by dual uplink of band 21 and band n77, the MSD value can be seen as dB related to 4th order proportional of band 21 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case. New MSD value is shown in Table 5.13.3-1 below.

Table 5.13.3-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-21A_n77A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 20.8 | IMD4 |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 3 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | 1771.6 | 5 | 25 | 1866.6 | 18.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |

#### 5.13.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.14	DC_3-42_n77

#### 5.14.1	Configuration for DC

Table 5.14.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-42A_n77A14,15,16 DC_3A-42C_n77A14,15,16 DC_3A-42D_n77A14,15,16 DC_3A-42E_n77A14,15,16 | DC_3A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.14.2	Maximum output power for DC

Based on studies of PC2 DC_3_n77, this section can be omitted.

#### 5.14.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n77 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 4th and 5th order IMD generated by dual uplink of band 3 and band n77 may also fall into own Rx of band 42.

However, IMD will not be an issue because there is non-simultaneous Rx/Tx operation between band 42 and band n77. Therefore, there is no MSD issue for this DC configuration.

#### 5.14.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.15	DC_21-42_n77

#### 5.15.1	Configuration for DC

Table 5.15.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_21A-42A_n77A14,15,16 DC_21A-42C_n77A14,15,16 | DC_21A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.15.2	Maximum output power for DC

Based on studies of PC2 DC_21_n77, this section can be omitted.

#### 5.15.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_21_n77 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 5th order IMD generated by dual uplink of band 21 and band n77 may also fall into own Rx of band 42.

However, IMD will not be an issue because there is non-simultaneous Rx/Tx operation between band 42 and band n77. Therefore, there is no MSD issue for this DC configuration.

#### 5.15.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.16	DC_1_n77

#### 5.16.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3 [2], so this section is omitted.

#### 5.16.2	Maximum output power for DC

Table 5.16.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.16.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_1_n77, the co-existence study is provided in TR 37.863-01-01 [3]. Based on above,

- the 2nd, 4th, and 5th order IMD may impact the Rx frequencies of band 1.

- the 4th and 5th order IMD may impact the Rx frequencies of band n77.

- the 2nd, 3rd, 4th, and 5th order harmonic from PC2 UL band n77 do not fall into Rx frequencies of band 1.

- the 2nd, 3rd, 4th, and 5th order harmonic mixing from PC2 UL band n77 do not impact the Rx frequencies of band 1.

- the 3rd order IMD do not impact the Rx frequencies of band 1.

- the 2nd and 3rd order IMD do not impact the Rx frequencies of band n77.

It should be noted that IMD will not be an issue for band n77 (no self-interference for the TDD band) even through the IMD products may impact the concerning band.

For MSD due to 2nd order IMD, the MSD value can be seen as dB related to 1st order proportional of band 1 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case.

For MSD due to 4th order IMD, MSD value for PC2 DC_1A_n77A can reuse the value for PC2 DC_1A_n78A already specified in TS 38.101-3.

New MSD values are shown in Table 5.16.3-1 below.

Table 5.16.3-1: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n77A | 1 | 1950 | 5 | 25 | 2140 | 35.8 | IMD21 |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD41 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.16.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.17	DC_3_n77

#### 5.17.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3 [2], so this section is omitted.

#### 5.17.2	Maximum output power for DC

Table 5.17.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_3A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.17.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_3_n77, the co-existence study is provided in TR 37.863-01-01 [3]. Based on above,

- the 2nd order harmonic mixing from PC2 UL band n77 may impact the Rx frequencies of band 3.

- the 2nd, 4th and 5th order IMD may impact the Rx frequencies of band 3.

- the 4th and 5th order IMD may impact the Rx frequencies of band n77.

- the 2nd, 3rd, 4th, and 5th order harmonic from PC2 UL band n77 do not fall into Rx frequencies of band 3.

- the 3rd, 4th, and 5th order harmonic mixing from PC2 UL band n77 do not impact the Rx frequencies of band 3.

- the 3rd order IMD do not impact the Rx frequencies of band 3.

- the 2nd and 3rd order IMD do not impact the Rx frequencies of band n77.

It should be noted that IMD will not be an issue for band n77 (no self-interference for the TDD band) even through the IMD products may impact the concerning band.

For MSD due to 2nd order harmonic mixing, MSD value of PC2 case will be 3dB higher than that of PC3 case. However, the MSD value is updated to match re-analysed value in R4-2301133 [4] when the channel bandwidth of UL n77 is 5MHz. New MSD values are shown in Table 5.17.3-1 below. Uplink configuration is shown in Table 5.17.3-2 below.

For MSD due to 2nd order IMD and 4th order IMD, MSD values for PC2 DC_3A_n77A can reuse the values for PC2 DC_3A_n78A already specified in TS 38.101-3. MSD value is shown in Table 5.17.3-3 below.

Table 5.17.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 3 | 8.1 | 7.0 | 6.0 | 5.7 |  |  |  |  |  |  |  |

Table 5.17.3-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |

Table 5.17.3-3: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n77A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD21 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | N/A |
|  | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD41 |
|  | n77 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.17.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.18	DC_21_n77

#### 5.18.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3 [2], so this section is omitted.

#### 5.18.2	Maximum output power for DC

Table 5.18.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_21A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.18.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_21_n77, the co-existence study is provided in TR 37.863-01-01 [3]. Based on above,

- the 4th and 5th order IMD may impact the Rx frequencies of band n77.

- the 2nd, 3rd, 4th, and 5th order harmonic from PC2 UL band n77 do not fall into Rx frequencies of band 21.

- the 2nd, 3rd, 4th, and 5th order harmonic mixing from PC2 UL band n77 do not impact the Rx frequencies of band 21.

- the 2nd, 3rd, 4th, and 5th order IMD do not impact the Rx frequencies of band 21.

- the 2nd and 3rd order IMD do not impact the Rx frequencies of band n77.

It should be noted that IMD will not be an issue for band n77 (no self-interference for the TDD band) even through the IMD products may impact the concerning band. Therefore, there is no MSD issue for this DC configuration.

#### 5.18.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.19	DC_21_n78

#### 5.19.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3 [2], so this section is omitted.

#### 5.19.2	Maximum output power for DC

Table 5.19.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_21A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a [HigherPowerLimitCADC] capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.19.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_21_n78, the co-existence study is provided in TR 37.863-01-01 [3]. Based on above,

- the 4th order IMD may impact the Rx frequencies of band n78.

- the 2nd, 3rd, 4th, and 5th order harmonic from PC2 UL band n78 do not fall into Rx frequencies of band 21.

- the 2nd, 3rd, 4th, and 5th order harmonic mixing from PC2 UL band n78 do not impact the Rx frequencies of band 21.

- the 2nd, 3rd, 4th, and 5th order IMD do not impact the Rx frequencies of band 21.

- the 2nd, 3rd, and 5th order IMD do not impact the Rx frequencies of band n78.

It should be noted that IMD will not be an issue for band n78 (no self-interference for the TDD band) even through the IMD products may impact the concerning band. Therefore, there is no MSD issue for this DC configuration.

#### 5.19.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.20	DC_1-3_n77

#### 5.20.1	Configuration for DC

Table 5.20.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n77A5,14 | DC_1A_n77A14 DC_3A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.20.2	Maximum output power for DC

Based on studies of PC2 DC_1_n77 and PC2 DC_3_n77, this section can be omitted.

#### 5.20.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n77 and DC_3_n77 captured in TR 37.863-01-01 [2], own Rx impact of the 3rd band is the followings.

- the 2nd, 4th, and 5th order IMD generated by dual uplink of band 1 and band n77 may also impact the own Rx of band 3.

- the 2nd and 5th order IMD generated by dual uplink of band 3 and band n77 may also impact the own Rx of band 1.

For MSD due to 2nd order IMD generated by dual uplink of band 1 and band n77, the MSD value can be seen as dB related to 1st order proportional of band 1 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case. New MSD value is shown in Table 5.20.3-1 below.

For MSD due to 4th order IMD generated by dual uplink of band 1 and band n77, the MSD value can be seen as dB related to 3rd order proportional of band 1 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 12dB higher than that of PC3 case. New MSD value is shown in Table 5.20.3-1 below.

For MSD due to 2nd order IMD generated by dual uplink of band 3 and band n77, the MSD value can be seen as dB related to 1st order proportional of band 3 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm+20dBm and PC2 DC is assumed to be 23dBm+23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case. New MSD value is shown in Table 5.20.3-1 below.

Table 5.20.3-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 37.5 | IMD21 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 20.5 | IMD41 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 37.0 | IMD21 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.20.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.21	DC_1_n41

#### 5.21.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.21.2	Maximum output power for DC

Table 5.21.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_1A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

#### 5.21.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

- The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n41.

- The 2nd, 3rd, 4th, and 5th order harmonic mixing do not fall into Rx frequencies of band 1.

- The 2nd, 3rd, 4th, and 5th order IMD do not fall into Rx frequencies of band 1 and n41.

- Cross band isolation existing, the interference from band1 UL falls into n41 DL, and the interference from n41 UL also falls into band 1 DL.

The new MSD values are defined in below table.

Table 5.21.3-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n41 |  | 8.5 | 8.5 | 8.5 |  | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 | 8.5 |
| n41 | 1 | 11.8 | 11.8 | 11.8 | 11.8 |  |  |  |  |  |  |  |  |  |

Table 5.21.3-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n41 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n41 | 1 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |  |

#### 5.21.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so there is no additional values.

### 5.22	DC_41_n77

#### 5.22.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.22.2	Maximum output power for DC

Table 5.22.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_41A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

#### 5.22.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

- The 2nd, 3rd, 4th, and 5th order harmonic do not fall into Rx frequencies of n77.

- The UL2/DL3 harmonic mixing falls into Rx frequencies of band 41.

- The UL3/DL2 harmonic mixing falls into RX frequencies of band n77

- The 2nd, 3rd, 4th, and 5th order IMD do not fall into Rx frequencies of band 41 and n77.

- Cross band isolation existing, the interference from band41 UL falls into n77 DL, and the interference from n77 UL also falls into band 41 DL.

New MSDs are defined in the following tables.

Table 5.22.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 41X | 19.4 | 19.4 | 19.4 | 19.4 |  |  |  |  |  |  |  |
| 41 | n77 |  | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 | 17.0 |
| NOTE X: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.22.3-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| 41 | n77 | 15 |  | 50 | 75 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

Table 5.22.3-3: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 41 | n77 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |  |  |  |  |
| n77 | 41X |  | 11 | 11 | 11 | 9.9 | 9.0 | 8.8 | 7.6 | 6.7 | 6.4 | 6.0 | 5.9 | 5.8 |
| NOTE X: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.22.3-4: Uplink configuration for reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 41 | n77 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n77 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |  |

#### 5.22.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.23	DC_1-3_n78

#### 5.23.1	Configuration for DC

Table 5.23.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n78A5,14 | DC_1A_n78A14 DC_3A_n78A14 |
| DC_1A-3A_n78(2A)5,14 | DC_1A_n78A14 DC_3A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.23.2	Maximum output power for DC

Based on studies of PC2 DC_1_n78 and PC2 DC_3_n78, this section can be omitted.

#### 5.23.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n78 and DC_3_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 2nd and 5th order IMD generated by dual uplink of band 1 and band n78 may impact the Rx frequencies of band 3.

- the 5th order IMD generated by dual uplink of band 3 and band n78 may impact the Rx frequencies of band 1.

For MSD due to 2nd order IMD generated by dual uplink of band 1 and band n78, the MSD value can be seen as dB related to 1st order proportional of band 1 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case. New MSD value is shown in Table 5.23.3-1 below.

For MSD due to 5th order IMD generated by dual uplink of band 3 and band n78, the MSD value can be seen as dB related to 3rd order proportional of band 3 UL power + 2nd order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case. New MSD value is shown in Table 5.23.3-1 below.

Table 5.23.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n78A DC_1A-3A_n78(2A) | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 37.2 | IMD21 |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 17.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.23.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.24	DC_1-42_n78

#### 5.24.1	Configuration for DC

Table 5.24.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-42A_n78A14,15,16 DC_1A-42C_n78A14,15,16 DC_1A-42D_n78A14,15,16 DC_1A-42E_n78A14,15,16 | DC_1A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.24.2	Maximum output power for DC

Based on studies of PC2 DC_1_n78, this section can be omitted.

#### 5.24.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 4th order IMD generated by dual uplink of band 1 and band n78 may impact the Rx frequencies of band 42.

However, IMD will not be an issue because there is non-simultaneous Rx/Tx operation between band 42 and band n78. Therefore, there is no MSD issue for this DC configuration.

#### 5.24.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.25	DC_3-42_n78

#### 5.25.1	Configuration for DC

Table 5.25.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-42A_n78A14,15,16 DC_3A-42C_n78A14,15,16 DC_3A-42D_n78A14,15,16 DC_3A-42E_n78A14,15,16 | DC_3A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.25.2	Maximum output power for DC

Based on studies of PC2 DC_3_n78, this section can be omitted.

#### 5.25.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 4th and 5th order IMD generated by dual uplink of band 3 and band n78 may impact the Rx frequencies of band 42.

However, IMD will not be an issue because there is non-simultaneous Rx/Tx operation between band 42 and band n78. Therefore, there is no MSD issue for this DC configuration.

#### 5.25.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.26	DC_21-42_n78

#### 5.26.1	Configuration for DC

Table 5.26.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_21A-42A_n78A14,15,16 DC_21A-42C_n78A14,15,16 | DC_21A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.26.2	Maximum output power for DC

Based on studies of PC2 DC_21_n78, this section can be omitted.

#### 5.26.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_21_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the IMDs up to 5th order generated by dual uplink of band 21 and band n78 do not impact the Rx frequencies of band 42.

Therefore, there is no MSD issue for this DC configuration.

#### 5.26.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.27	DC_1-3_n79

#### 5.27.1	Configuration for DC

Table 5.27.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n79A5,14 | DC_1A_n79A14 DC_3A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.27.2	Maximum output power for DC

Based on studies of PC2 DC_1_n79 and PC2 DC_3_n79, this section can be omitted.

#### 5.27.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n79 and DC_3_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the IMDs up to 5th order generated by dual uplink of band 1 and band n79 do not impact the Rx frequencies of band 3.

- the 5th order IMD generated by dual uplink of band 3 and band n79 may impact the Rx frequencies of band 1.

For MSD due to 5th order IMD generated by dual uplink of band 3 and band n79, the MSD value can be seen as dB related to 4th order proportional of band 3 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 21dB higher than that of PC3 case. New MSD value is shown in Table 5.27.3-1 below.

Table 5.27.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 24.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 10 | 50 | 4860 | N/A | N/A |

#### 5.27.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.28	DC_1-19_n79

#### 5.28.1	Configuration for DC

Table 5.28.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-19A_n79A5,14 | DC_1A_n79A14 DC_19A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.28.2	Maximum output power for DC

Based on studies of PC2 DC_1_n79 and PC2 DC_19_n79, this section can be omitted.

#### 5.28.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n79 and DC_19_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 3rd and 4th order IMD generated by dual uplink of band 1 and band n79 may impact the Rx frequencies of band 19.

- the 4th order IMD generated by dual uplink of band 19 and band n79 may impact the Rx frequencies of band 1.

For MSD due to 3rd order IMD generated by dual uplink of band 1 and band n79, the MSD value can be seen as dB related to 2nd order proportional of band 1 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case. New MSD value is shown in Table 5.28.3-1 below.

For MSD due to 4th order IMD generated by dual uplink of band 19 and band n79, the MSD value can be seen as dB related to 3rd order proportional of band 19 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 18dB higher than that of PC3 case. New MSD value is shown in Table 5.28.3-1 below.

Table 5.28.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 33.3 | IMD35 |
|  | n79 | 4782.5 | 10 | 50 | 4782.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 26.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 10 | 50 | 4652.5 | N/A | N/A |
| NOTE 5: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.28.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.29	DC_1-21_n79

#### 5.29.1	Configuration for DC

Table 5.29.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-21A_n79A5,14 | DC_1A_n79A14 DC_21A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.29.2	Maximum output power for DC

Based on studies of PC2 DC_1_n79 and PC2 DC_21_n79, this section can be omitted.

#### 5.29.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n79 and DC_21_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 4th order IMD generated by dual uplink of band 1 and band n79 may impact the Rx frequencies of band 21.

- the IMDs up to 5th order generated by dual uplink of band 21 and band n79 do not impact the Rx frequencies of band 1.

Considering that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band 1 UL/DL = 1940-1960/2130-2150MHz and Band n79 UL/DL = 4400-4900/4400-4900MHz. This is based on the discussion when PC3 DC with the same configuration was specified [4]. Therefore, own Rx impacts can be simplified as below:

- the IMDs up to 5th order generated by dual uplink of band 1 and band n79 do not impact the Rx frequencies of band 21.

- the IMDs up to 5th order generated by dual uplink of band 21 and band n79 do not impact the Rx frequencies of band 1.

Based on the above, new MSD value is shown in Table 5.29.3-1 below.

Table 5.29.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-21A_n79AX1,X2 | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE X1: The frequency range in band n79 is restricted for this band combination to 4400 - 4900 MHz for both the UL and the DL. NOTE X2: The frequency range in band 1 is restricted for this band combination to 1940 - 1960 MHz for the UL and 2130 - 2150 MHz for the DL. |  |  |  |  |  |  |  |

#### 5.29.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.30	DC_1-42_n79

#### 5.30.1	Configuration for DC

Table 5.30.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-42A_n79A14 DC_1A-42C_n79A14 DC_1A-42D_n79A14 DC_1A-42E_n79A14 | DC_1A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.30.2	Maximum output power for DC

Based on studies of PC2 DC_1_n79, this section can be omitted.

#### 5.30.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 5th order IMD generated by dual uplink of band 1 and band n79 may impact the Rx frequencies of band 42.

For MSD due to 5th order IMD generated by dual uplink of band 3 and band n79, the MSD value can be seen as dB related to 4th order proportional of band 1 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 21dB higher than that of PC3 case. New MSD value is shown in Table 5.30.3-1 below.

Table 5.30.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-42A_n79A DC_1A-42C_n79A DC_1A-42D_n79A DC_1A-42E_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 42 | 3490 | 5 | 25 | 3490 | 25.8 | IMD5 |
|  | n79 | 4420 | 10 | 50 | 4420 | N/A | N/A |

#### 5.30.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.31	DC_3-19_n79

#### 5.31.1	Configuration for DC

Table 5.31.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-19A_n79A5,14 | DC_3A_n79A14 DC_19A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.31.2	Maximum output power for DC

Based on studies of PC2 DC_3_n79 and PC2 DC_19_n79, this section can be omitted.

#### 5.31.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n79 and DC_19_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are follows:

- the 3rd and 4th order IMD generated by dual uplink of band 3 and band n79 may impact the Rx frequencies of band 19.

- the 4th order IMD generated by dual uplink of band 19 and band n79 may impact the Rx frequencies of band 3.

For MSD due to 3rd order IMD generated by dual uplink of band 3 and band n79, the MSD value can be seen as dB related to 2nd order proportional of band 3 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case. New MSD value is shown in Table 5.31.3-1 below.

For MSD due to 4th order IMD generated by dual uplink of band 19 and band n79, the MSD value can be seen as dB related to 3rd order proportional of band 19 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 18dB higher than that of PC3 case. New MSD value is shown in Table 5.31.3-1 below.

Table 5.31.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | 840 | 5 | 25 | 885 | 33.5 | IMD35 |
|  | n79 | 4435 | 10 | 50 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 18.2 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 10 | 50 | 4420 | N/A | N/A |
| NOTE 5: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

#### 5.31.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.32	DC_3-21_n79

#### 5.32.1	Configuration for DC

Table 5.32.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-21A_n79A5,14 | DC_3A_n79A14 DC_21A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.32.2	Maximum output power for DC

Based on studies of PC2 DC_3_n79 and PC2 DC_21_n79, this section can be omitted.

#### 5.32.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n79 and DC_21_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are follows:

- the 3rd order IMD generated by dual uplink of band 3 and band n79 may impact the Rx frequencies of band 21.

- the 3rd order IMD generated by dual uplink of band 21 and band n79 may impact the Rx frequencies of band 3.

Considering that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n79 UL/DL = 4400-4900/4400-4900MHz. This is based on the discussion when PC3 DC with the same configuration was specified [4]. Therefore, own Rx impacts can be simplified as below:

- the IMDs up to 5th order generated by dual uplink of band 3 and band n79 do not impact the Rx frequencies of band 21.

- the 3rd order IMD generated by dual uplink of band 21 and band n79 may impact the Rx frequencies of band 3.

For MSD due to 3rd order IMD generated by dual uplink of band 21 and band n79, the MSD value can be seen as dB related to 2nd order proportional of band 21 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case. New MSD value is shown in Table 5.32.3-1 below.

Table 5.32.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-21A_n79AX | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | 1774.2 | 5 | 25 | 1869.2 | 32.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 10 | 50 | 4770 | N/A | N/A |
| NOTE X: The frequency range in band n79 is restricted for this band combination to 4400 - 4900 MHz for both the UL and the DL. |  |  |  |  |  |  |  |

#### 5.32.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.33	DC_3-42_n79

#### 5.33.1	Configuration for DC

Table 5.33.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-42A_n79A14 DC_3A-42C_n79A14 DC_3A-42D_n79A14 DC_3A-42E_n79A14 | DC_3A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.33.2	Maximum output power for DC

Based on studies of PC2 DC_3_n79, this section can be omitted.

#### 5.33.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 5th order IMD generated by dual uplink of band 3 and band n79 may impact the Rx frequencies of band 42.

Considering actual spectrum holdings, the frequency range can be limited as Band n79 UL/DL = 4500-5000/4500-5000MHz. This is based on the discussion when PC3 DC with the same configuration was specified [4]. Therefore, own Rx impacts can be simplified as below:

- the IMDs up to 5th order generated by dual uplink of band 3 and band n79 do not impact the Rx frequencies of band 42.

Therefore, MSD value is shown in Table 5.33.3-1 below.

Table 5.33.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-42A_n79AX DC_3A-42C_n79AX DC_3A-42D_n79AX DC_3A-42E_n79AX | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE X: The frequency range in band n79 is restricted for this band combination to 4500 - 5000 MHz for both the UL and the DL. |  |  |  |  |  |  |  |

#### 5.33.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.34	DC_19-21_n79

#### 5.34.1	Configuration for DC

Table 5.34.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_19A-21A_n79A5,14 | DC_19A_n79A14 DC_21A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.34.2	Maximum output power for DC

Based on studies of PC2 DC_19_n79 and PC2 DC_21_n79, this section can be omitted.

#### 5.34.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_19_n79 and DC_21_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are follows:

- the 5th order IMD generated by dual uplink of band 19 and band n79 may impact the Rx frequencies of band 21.

- the 5th order IMD generated by dual uplink of band 21 and band n79 may impact the Rx frequencies of band 19.

Considering that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n79 UL/DL = 4400-4900/4400-4900MHz. This is based on the discussion when PC3 DC with the same configuration was specified [4]. Therefore, own Rx impacts can be simplified as below:

- the 5th order IMD generated by dual uplink of band 19 and band n79 may impact the Rx frequencies of band 21.

- the IMDs up to 5th order generated by dual uplink of band 21 and band n79 do not impact the Rx frequencies of band 19.

For MSD due to 5th order IMD generated by dual uplink of band 19 and band n79, the MSD value can be seen as dB related to 4th order proportional of band 19 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. In addition, PSD will be 6dB higher when UL CBW of n79 is changed from 40MHz to 10MHz. Therefore, MSD value of PC2 case will be 21dB higher than that of PC3 case. New MSD value is shown in Table 5.34.3-1 below.

Table 5.34.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A-21A_n79AX | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 24.8 | IMD5 |
|  | n79 | 4850 | 10 | 50 | 4850 | N/A | N/A |
| NOTE X: The frequency range in band n79 is restricted for this band combination to 4400 - 4900 MHz for both the UL and the DL. |  |  |  |  |  |  |  |

#### 5.34.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.35	DC_19-42_n79

#### 5.35.1	Configuration for DC

Table 5.35.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_19A-42A_n79A14 DC_19A-42C_n79A14 | DC_19A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations.. |  |

#### 5.35.2	Maximum output power for DC

Based on studies of PC2 DC_19_n79, this section can be omitted.

#### 5.35.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_19_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 2nd order IMD generated by dual uplink of band 19 and band n79 may impact the Rx frequencies of band 42.

Considering that Band 19 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n79 UL/DL = 4500-4600/4500-4600MHz. This is based on the discussion when PC3 DC with the same configuration was specified [4]. Therefore, own Rx impacts can be simplified as below:

- the IMDs up to 5th order generated by dual uplink of band 19 and band n79 do not impact the Rx frequencies of band 42.

Therefore, MSD value is shown in Table 5.35.3-1 below.

Table 5.35.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A-42A_n79AX DC_19A-42C_n79AX | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE X: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz for both the UL and the DL. |  |  |  |  |  |  |  |

#### 5.35.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.36	DC_21-42_n79

#### 5.36.1	Configuration for DC

Table 5.36.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_21A-42A_n79A14 DC_21A-42C_n79A14 | DC_21A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.36.2	Maximum output power for DC

Based on studies of PC2 DC_21_n79, this section can be omitted.

#### 5.36.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_21_n79 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 2nd order IMD generated by dual uplink of band 21 and band n79 may impact the Rx frequencies of band 42.

Considering that Band 21 is currently operated only by a certain operator in Japan, the frequency range can be limited as Band n79 UL/DL = 4500-4600/4500-4600MHz. This is based on the discussion when PC3 DC with the same configuration was specified [4]. Therefore, own Rx impacts can be simplified as below:

- the IMDs up to 5th order generated by dual uplink of band 21 and band n79 do not impact the Rx frequencies of band 42.

Therefore, MSD value is shown in Table 5.36.3-1 below.

Table 5.36.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_21A-42A_n79AX DC_21A-42C_n79AX | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE X: The frequency range in band n79 is restricted for this band combination to 4500 - 4600 MHz for both the UL and the DL. |  |  |  |  |  |  |  |

#### 5.36.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.37	DC_28_n41

#### 5.37.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.37.2	Maximum output power for DC

Table 5.37.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_28A_n41A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

#### 5.37.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

- No harmonic issue

- No harmonic missing issue

- No IMD of dual UL fall into Rx frequencies of B28.

- No cross band isolation interference.

Thus, no PC2 MSD needs to be defined.

#### 5.37.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

### 5.38	DC_28_n77

#### 5.38.1	Configuration for DC

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

#### 5.38.2	Maximum output power for DC

Table 5.38.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_28A_n77A | 266 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. |  |  |  |  |

#### 5.38.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC.

- The 5th order harmonic of 28 fall into Rx frequencies of n77, but band 28 can only support PC3 so no need to introduce PC2 MSD due to harmonic.

- The 5th harmonic mixing falls into Rx frequencies of band 28.

- The 5th order IMD fall into Rx frequencies of n28.

- No cross band isolation interference.

New PC2 MSDs are defined in the following tables.

Table 5.38.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 281 | 31 | 28 | 26.2 | 25 |  |  |  |  |  |  |  |
| NOTE 1: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Note: The Uplink configuration for reference sensitivity exception table is omitted here which is the same as for PC3.

Table 5.38.3-2: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n77A | 28 | 705.5 | 5 | 25 | 760.5 | 19.2 | IMD5 |
|  | n77 | 3582.5 | 10 | 50 | 3582.5 | N/A | N/A |

#### 5.38.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC.

### 5.39	DC_19_n77

#### 5.39.1	Configuration for DC

For DC_19A_n77A, EN-DC configuration is 2DL2UL without intra-band CA, so it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3 [3], so this section is skipped.

On the other hand, for DC_19A_n77(2A), it is needed to update the inter-band EN-DC configuration table. The table is shown below.

Table 5.39.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_19A_n77(2A)7, 21 | DC_19A_n77A21 | No |  |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 21: For this DC configuration, reference sensitivity exceptions for Power Class 2, if allowed, are specified in Clause 7.3B.2.3. If the uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration, the note is not shown as the reference sensitivity exceptions, if any, have been confirmed. |  |  |  |

#### 5.39.2	Maximum output power for DC

Table 5.39.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_19A_n77A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.39.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_19_n77, the co-existence study is provided in TR 37.863-01-01 [4]. Based on above,

- the 4th order harmonic mixing from PC2 UL band n77 may impact the Rx frequencies of band 19.

- the 4th and 5th order IMD may impact the Rx frequencies of band 19.

- the 2nd and 5th order IMD may impact the Rx frequencies of band n77.

For MSD due to 4th order harmonic mixing, MSD value of PC2 case will be 3dB higher than that of PC3 case. Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD values are shown in Table 5.39.3-1 below. Uplink configuration is shown in Table 5.39.3-2 below.

For MSD due to 4th order IMD generated by dual uplink of band 19 and band n77, the MSD value can be seen as dB related to 3rd order proportional of band 19 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 12dB higher than that of PC3 case. Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD value is shown in Table 5.39.3-3 below.

It should be noted that IMD will not be an issue for band n77 (no self-interference for the TDD band) even through the IMD products may fall into the concerning band.

Table 5.39.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n77 | 19 | 9.8 | 7.2 | 5.8 |  |  |  |  |  |  |  |  |

Table 5.39.3-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n77 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |

Table 5.39.3-3: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A_n77A DC_19A_n77(2A) | 19 | 836.5 | 5 | 25 | 881.5 | 25.3 | IMD4 |
|  | n77 | 3391 | 10 | 50 | 3391 | N/A | N/A |
|  | 19 | 832.5 | 5 | 25 | 877.5 | 8.1 | IMD5 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |

#### 5.39.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.40	DC_19_n78

#### 5.40.1	Configuration for DC

For DC_19A_n78A, EN-DC configuration is 2DL2UL without intra-band CA, so it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3 [3], so this section is skipped.

On the other hand, for DC_19A_n78(2A), it is needed to update the inter-band EN-DC configuration table. The table is shown below.

Table 5.40.1-1: Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_19A_n78(2A)7, 21 | DC_19A_n78A21 | No | No |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. NOTE 14: Applicable when dynamic switching between two uplink carriers is conducted. The DL interruption requirements for NR DL carrier(s) and E-UTRA DL carrier(s) are specified in clause 8.2.1.2.14 of 38.133 [15] and clause 7.32.2.12 of 36.133 [16] respectively. NOTE 21: For this DC configuration, reference sensitivity exceptions for Power Class 2, if allowed, are specified in Clause 7.3B.2.3. If the uplink EN-DC configuration supported in Table 6.2B.1.3-1 is applicable to the same EN-DC configuration, the note is not shown as the reference sensitivity exceptions, if any, have been confirmed. |  |  |  |

#### 5.40.2	Maximum output power for DC

Table 5.40.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_19A_n78A | 266,8 | +2/-3 | 23 | +2/-3 |
| NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 8: The UE that supports PC3 within a TDD or FDD band and supports PC2 within a second TDD band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2B.4.1.3. |  |  |  |  |

#### 5.40.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. For PC3 DC_19_n78, the co-existence study is provided in TR 37.863-01-01 [4]. Based on above,

- the 4th order harmonic mixing from PC2 UL band n78 may impact the Rx frequencies of band 19.

- the 4th order IMD may impact the Rx frequencies of band 19.

For MSD due to 4th order harmonic mixing, MSD value of PC2 case will be 3dB higher than that of PC3 case. Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD values are shown in Table 5.40.3-1 below. Uplink configuration is shown in Table 5.40.3-2 below.

For MSD due to 4th order IMD generated by dual uplink of band 19 and band n78, the MSD value can be seen as dB related to 3rd order proportional of band 19 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 12dB higher than that of PC3 case. Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD value is shown in Table 5.40.3-3 below.

Table 5.40.3-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n78 | 19 | 9.8 | 7.2 | 5.8 |  |  |  |  |  |  |  |  |

Table 5.40.3-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n78 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |

Table 5.40.3-3: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A_n78A DC_19A_n78(2A) | 19 | 836.5 | 5 | 25 | 881.5 | 25.3 | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | N/A |

#### 5.40.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.41	DC_1-19_n77

#### 5.41.1	Configuration for DC

Table 5.41.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-19A_n77A5,14 | DC_1A_n77A14 DC_19A_n77A14 |
| DC_1A-19A_n77(2A)5,14 | DC_1A_n77A14 DC_19A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.41.2	Maximum output power for DC

Based on studies of PC2 DC_1_n77 and PC2 DC_19_n77, this section can be omitted.

#### 5.41.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n77 and DC_19_n77 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 5th order IMD generated by dual uplink of band 1 and band n77 may impact the Rx frequencies of band 19.

- the 3rd order IMD generated by dual uplink of band 19 and band n77 may impact the Rx frequencies of band 1.

For MSD due to 5th order IMD generated by dual uplink of band 1 and band n77, the MSD value can be seen as dB related to 3rd order proportional of band 1 UL power + 2nd order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case.

For MSD due to 3rd order IMD generated by dual uplink of band 19 and band n77, the MSD value can be seen as dB related to 2nd order proportional of band 19 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case.

Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD value is shown in Table 5.41.3-1 below.

Table 5.41.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-19A_n77A DC_1A-19A_n77(2A) | 1 | 1940 | 5 | 25 | 2130 | 26.7 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 19 | 835 | 5 | 25 | 880 | 18.5 | IMD5 |
|  | n77 | 3350 | 10 | 50 | 3350 | N/A | N/A |

#### 5.41.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.42	DC_1-19_n78

#### 5.42.1	Configuration for DC

Table 5.42.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-19A_n78A5,14 | DC_1A_n78A14 DC_19A_n78A14 |
| DC_1A-19A_n78(2A)5,14 | DC_1A_n78A14 DC_19A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.42.2	Maximum output power for DC

Based on studies of PC2 DC_1_n78 and PC2 DC_19_n78, this section can be omitted.

#### 5.42.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n78 and DC_19_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 5th order IMD generated by dual uplink of band 1 and band n78 may impact the Rx frequencies of band 19.

- the 3rd order IMD generated by dual uplink of band 19 and band n78 may impact the Rx frequencies of band 1.

For MSD due to 5th order IMD generated by dual uplink of band 1 and band n78, the MSD value can be seen as dB related to 3rd order proportional of band 1 UL power + 2nd order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case.

For MSD due to 3rd order IMD generated by dual uplink of band 19 and band n78, the MSD value can be seen as dB related to 2nd order proportional of band 19 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case.

Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD value is shown in Table 5.42.3-1 below.

Table 5.42.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-19A_n78A DC_1A-19A_n78(2A) | 1 | 1940 | 5 | 25 | 2130 | 26.7 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 19 | 835 | 5 | 25 | 880 | 18.5 | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |

#### 5.42.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.43	DC_3-19_n77

#### 5.43.1	Configuration for DC

Table 5.43.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-19A_n77A5,14 | DC_3A_n77A14 DC_19A_n77A14 |
| DC_3A-19A_n77(2A)5,14 | DC_3A_n77A14 DC_19A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.43.2	Maximum output power for DC

Based on studies of PC2 DC_3_n77 and PC2 DC_19_n77, this section can be omitted.

#### 5.43.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n77 and DC_19_n77 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 3rd order IMD generated by dual uplink of band 19 and band n77 may impact the Rx frequencies of band 3.

For MSD due to 3rd order IMD generated by dual uplink of band 19 and band n77, the MSD value can be seen as dB related to 2nd order proportional of band 19 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case. Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD value is shown in Table 5.43.3-1 below.

Table 5.43.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-19A_n77A DC_3A-19A_n77(2A) | 3 | 1775 | 5 | 25 | 1850 | 26.3 | IMD3 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n77 | 3520 | 10 | 50 | 3520 | N/A | N/A |

#### 5.43.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.44	DC_3-19_n78

#### 5.44.1	Configuration for DC

Table 5.44.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-19A_n78A5,14 | DC_3A_n78A14 DC_19A_n78A14 |
| DC_3A-19A_n78(2A)5,14 | DC_3A_n78A14 DC_19A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.44.2	Maximum output power for DC

Based on studies of PC2 DC_3_n78 and PC2 DC_19_n78, this section can be omitted.

#### 5.44.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n78 and DC_19_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 3rd order IMD generated by dual uplink of band 19 and band n78 may impact the Rx frequencies of band 3.

For MSD due to 3rd order IMD generated by dual uplink of band 19 and band n78, the MSD value can be seen as dB related to 2nd order proportional of band 19 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case. Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD value is shown in Table 5.44.3-1 below.

Table 5.44.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-19A_n78A DC_3A-19A_n78(2A) | 3 | 1775 | 5 | 25 | 1850 | 26.3 | IMD3 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |

#### 5.44.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.45	DC_19-21_n77

#### 5.45.1	Configuration for DC

Table 5.45.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_19A-21A_n77A5,14 | DC_19A_n77A14 DC_21A_n77A14 |
| DC_19A-21A_n77(2A)5,14 | DC_19A_n77A14 DC_21A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.45.2	Maximum output power for DC

Based on studies of PC2 DC_19_n77 and PC2 DC_21_n77, this section can be omitted.

#### 5.45.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_19_n77 and DC_21_n77 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 4th order IMD generated by dual uplink of band 19 and band n77 may impact the Rx frequencies of band 21.

- the 3rd and 4th order IMD generated by dual uplink of band 21 and band n77 may impact the Rx frequencies of band 19.

For MSD due to 4th order IMD generated by dual uplink of band 19 and band n77, the MSD value can be seen as dB related to 3rd order proportional of band 19 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 12dB higher than that of PC3 case.

For MSD due to 3rd order IMD generated by dual uplink of band 21 and band n77, the MSD value can be seen as dB related to 2nd order proportional of band 21 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case.

For MSD due to 4th order IMD generated by dual uplink of band 21 and band n77, the MSD value can be seen as dB related to 3rd order proportional of band 21 UL power + 1st order proportional of band n77 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 12dB higher than that of PC3 case.

Based on above, new MSD value is shown in Table 5.45.3-1 below.

Table 5.45.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A-21A_n77A DC_19A-21A_n77(2A) | 19 | 837.5 | 5 | 25 | 882.5 | 27.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 25.2 | IMD4 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3468.7 | 10 | 50 | 3468.7 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 21.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |

#### 5.45.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.46	DC_19-21_n78

#### 5.46.1	Configuration for DC

Table 5.46.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_19A-21A_n78A5,14 | DC_19A_n78A14 DC_21A_n78A14 |
| DC_19A-21A_n78(2A)5,14 | DC_19A_n78A14 DC_21A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.46.2	Maximum output power for DC

Based on studies of PC2 DC_19_n78 and PC2 DC_21_n78, this section can be omitted.

#### 5.46.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_19_n78 and DC_21_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 3rd and 4th order IMD generated by dual uplink of band 21 and band n78 may impact the Rx frequencies of band 19.

For MSD due to 3rd order IMD generated by dual uplink of band 21 and band n78, the MSD value can be seen as dB related to 2nd order proportional of band 21 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 9dB higher than that of PC3 case.

For MSD due to 4th order IMD generated by dual uplink of band 21 and band n78, the MSD value can be seen as dB related to 3rd order proportional of band 21 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 12dB higher than that of PC3 case.

Based on above, new MSD value is shown in Table 5.46.3-1 below.

Table 5.46.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A-21A_n78A DC_19A-21A_n78(2A) | 19 | 837.5 | 5 | 25 | 882.5 | 27.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 25.2 | IMD4 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n78 | 3468.7 | 10 | 50 | 3468.7 | N/A | N/A |

#### 5.46.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.47	DC_19-42_n77

#### 5.47.1	Configuration for DC

Table 5.47.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_19A-42A_n77A14,15,16 DC_19A-42C_n77A14,15,16 | DC_19A_n77A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.47.2	Maximum output power for DC

Based on studies of PC2 DC_19_n77, this section can be omitted.

#### 5.47.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_19_n77 captured in TR 37.863-01-01 [2], own Rx impacts of the 3rd band are as follows:

- the 2nd, 3rd, 4th, and 5th order IMD do not impact the Rx frequencies of band n77.

Therefore, there is no MSD issue for this DC configuration.

#### 5.47.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.48	DC_19-42_n78

#### 5.48.1	Configuration for DC

Table 5.48.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_19A-42A_n78A14,15,16 DC_19A-42C_n78A14,15,16 | DC_19A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. NOTE 15: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band non-contiguous EN-DC apply for the Band 42/48 and Band n78/n78 combination and for the Band 2 and Band n25 combinations. For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, when UE capability interBandContiguousMRDC is indicated, the minimum requirements for intra-band-contiguous EN-DC also should be met in addtion to intra-band non-contiguous EN-DC. NOTE 16: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |

#### 5.48.2	Maximum output power for DC

Based on studies of PC2 DC_19_n78, this section can be omitted.

#### 5.48.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_19_n78 captured in TR 37.863-01-01 [2], own Rx impacts of the 3rd band are as follows:

- the 2nd, 3rd, 4th, and 5th order IMD do not impact the Rx frequencies of band n78.

Therefore, there is no MSD issue for this DC configuration.

#### 5.48.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.49	DC_19_n77-n79

#### 5.49.1	Configuration for DC

Table 5.49.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_19A_n77A-n79A14 | DC_19A_n77A14 DC_19A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.49.2	Maximum output power for DC

Based on studies of PC2 DC_19_n77 and PC2 DC_19_n79, this section can be omitted.

#### 5.49.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_19_n77 and DC_19_n79 captured in TR 37.863-01-01 [2], own Rx impacts of the 3rd band are as follows:

- the 2nd, 3rd, 4th, and 5th order IMD generated by dual uplink of band 19 and band n77 may impact the Rx frequencies of band n79.

- the 2nd and 3rd order IMD generated by dual uplink of band 19 and band n79 may impact the Rx frequencies of band n77.

However, IMD will not be an issue because the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. Therefore, there is no MSD issue for this DC configuration.

#### 5.49.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.50	DC_19_n78-n79

#### 5.50.1	Configuration for DC

Table 5.50.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_19A_n78A-n79A14 | DC_19A_n78A14 DC_19A_n79A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.50.2	Maximum output power for DC

Based on studies of PC2 DC_19_n78 and PC2 DC_19_n79, this section can be omitted.

#### 5.50.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_19_n78 and DC_19_n79 captured in TR 37.863-01-01 [2], own Rx impacts of the 3rd band are as follows:

- the 2nd, 3rd, 4th, and 5th order IMD generated by dual uplink of band 19 and band n78 may impact the Rx frequencies of band n79.

- the 2nd and 3rd order IMD generated by dual uplink of band 19 and band n79 may impact the Rx frequencies of band n78.

For MSD due to 2nd order IMD generated by dual uplink of band 19 and band n78, the MSD value can be seen as dB related to 1st order proportional of band 19 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case.

For MSD due to 2nd order IMD generated by dual uplink of band 19 and band n79, the MSD value can be seen as dB related to 1st order proportional of band 19 UL power + 1st order proportional of band n79 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case.

Based on above, new MSD value is shown in Table 5.50.3-1 below.

Table 5.50.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 35.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 34.8 | IMD2 |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |

#### 5.50.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.51	DC_1-21_n78

#### 5.51.1	Configuration for DC

Table 5.51.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-21A_n78A5,14 | DC_1A_n78A14 DC_21A_n78A14 |
| DC_1A-21A_n78(2A)5,14 | DC_1A_n78A14 DC_21A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.51.2	Maximum output power for DC

Based on studies of PC2 DC_1_n78 and PC2 DC_21_n78, this section can be omitted.

#### 5.51.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_1_n78 and DC_21_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 2nd and 5th order IMD generated by dual uplink of band 1 and band n78 may impact the Rx frequencies of band 21.

- the 2nd and 5th order IMD generated by dual uplink of band 21 and band n78 may impact the Rx frequencies of band 1.

For MSD due to 2nd order IMD generated by dual uplink of band 1 and band n78, the MSD value can be seen as dB related to 1st order proportional of band 1 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case.

For MSD due to 5th order IMD generated by dual uplink of band 1 and band n78, the MSD value can be seen as dB related to 3rd order proportional of band 1 UL power + 2nd order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case.

For MSD due to 2nd order IMD generated by dual uplink of band 21 and band n78, the MSD value can be seen as dB related to 1st order proportional of band 21 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case.

For MSD due to 5th order IMD generated by dual uplink of band 21 and band n78, the MSD value can be seen as dB related to 4th order proportional of band 21 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case.

Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD value is shown in Table 5.51.3-1 below.

Table 5.51.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-21A_n78A DC_1A-21A_n78(2A) | 1 | 1964.6 | 5 | 25 | 2154.6 | 36.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | 1964.6 | 5 | 25 | 2154.6 | 16.2 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n78 | 3647 | 10 | 50 | 3647 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 37.5 | IMD2 |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 14.9 | IMD5 |
|  | n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |

#### 5.51.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.52	DC_3-21_n78

#### 5.52.1	Configuration for DC

Table 5.52.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-21A_n78A5,14 | DC_3A_n78A14 DC_21A_n78A14 |
| DC_3A-21A_n78(2A)5,14 | DC_3A_n78A14 DC_21A_n78A14 |
| NOTE 1: Uplink EN-DC configurations are the configurations supported by the present release of specifications. NOTE 5: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

#### 5.52.2	Maximum output power for DC

Based on studies of PC2 DC_3_n78 and PC2 DC_21_n78, this section can be omitted.

#### 5.52.3	REFSENS requirements for DC

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power UL DC. Based on co-existence studies of DC_3_n78 and DC_21_n78 captured in TR 37.863-01-01 [3], own Rx impacts of the 3rd band are as follows:

- the 4th and 5th order IMD generated by dual uplink of band 1 and band n78 may impact the Rx frequencies of band 21.

- the 2nd order IMD generated by dual uplink of band 21 and band n78 may impact the Rx frequencies of band 3.

For MSD due to 4th order IMD generated by dual uplink of band 3 and band n78, the MSD value can be seen as dB related to 3rd order proportional of band 3 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 12dB higher than that of PC3 case.

For MSD due to 5th order IMD generated by dual uplink of band 3 and band n78, the MSD value can be seen as dB related to 3rd order proportional of band 3 UL power + 2nd order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 15dB higher than that of PC3 case.

For MSD due to 2nd order IMD generated by dual uplink of band 21 and band n78, the MSD value can be seen as dB related to 1st order proportional of band 21 UL power + 1st order proportional of band n78 UL power. PC3 DC is assumed to be 20dBm + 20dBm and PC2 DC is assumed to be 23dBm + 23dBm. Therefore, MSD value of PC2 case will be 6dB higher than that of PC3 case.

Using the above simple calculation as a baseline, we reanalyzed the MSD values for PC2. New MSD value is shown in Table 5.52.3-1 below.

Table 5.52.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-21A_n78A DC_3A-21A_n78(2A) | 3 | 1767.5 | 5 | 25 | 1862.5 | 36.6 | IMD2 |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | N/A | N/A |
|  | n78 | 3322 | 10 | 50 | 3322 | N/A | N/A |
|  | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 23.2 | IMD4 |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1455.5 | 5 | 25 | 1503.5 | 9.5 | IMD5 |
|  | n78 | 3403 | 10 | 50 | 3403 | N/A | N/A |

#### 5.52.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.53	DC_3_n1-n78, DC_3-3_n1-n78

#### 5.53.1	Configuration for DC

Table 5.53.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n1A-n78A5, 14 | DC_3A_n1A DC_3A_n78A14 |
| DC_3A-3A_n1A-n78A5, 14 | DC_3A_n1A DC_3A_n78A14 |
| NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

Note that the note 5 already existed in the specification for these combinations.

#### 5.53.2	Maximum output power for DC

Since the maximum output power requirement for PC2 UL DC_3A_n78 is already specified in the specification, this section can be omitted.

#### 5.53.3	REFSENS requirements for DC

Based on co-existence studies of DC_3_n78 captured in TR 37.863-01-01, own Rx impacts of the 3rd band are as follows:

- The 5th order IMD generated by dual uplink of band 3 and band n78 may impact the Rx frequencies of band n1.

The MSD values for IMD5 reused the value for PC2 DC_1A-3A_n78A in section 5.23.

Table 5.53.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n78A DC_3A-3A_n1A-n78A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD5 |
|  | n78 | 3720 | 10 | 50 | 3720 | N/A | N/A |

#### 5.53.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

### 5.y	DC_7_n1-n78, DC_7-7_n1-n78

#### 5.54.1	Configuration for DC

Table 5.54.1-1: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A_n1A-n78A5, 14 | DC_7A_n1A DC_7A_n78A14 |
| DC_7A-7A_n1A-n78A5, 14 | DC_7A_n1A DC_7A_n78A14 |
| NOTE 14: PC3 or PC2 Uplink EN-DC configuration is applicable to EN-DC configurations. |  |

Note that the note 5 already existed in the specification for these combinations.

#### 5.54.2	Maximum output power for DC

Since the maximum output power requirement for PC2 UL DC_7A_n78 is already specified in the specification, this section can be omitted.

#### 5.54.3	REFSENS requirements for DC

Based on co-existence studies of DC_7_n78 captured in TR 37.863-01-01, own Rx impacts of the 3rd band are as follows:

-  The 4th order IMD generated by dual uplink of band 7 and band n78 may impact the Rx frequencies of band n1.

The MSD values for IMD5 reused the value for PC2 DC_1A-7A_n78A in the specifications.

Table 5.54.3-1: MSD test points for SCell due to dual uplink operation for PC2 EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n1A-n78A DC_7A-7A_n1A-n78A | 1 | 1950 | 5 | 25 | 2140 | 19.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |

#### 5.54.4	∆TIB and ∆RIB values

There is no change by comparing to the values for PC3 DC, so this section is omitted.

# Annex A - Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2022-11 | 3GPP RAN4#105 | R4-2218950 | Initial version Implemented TP’s from RAN4 #105: R4-2220466, TP for PC2 DC_1_n79 for TR 38.898, DoCoMo R4-2220467, TP for PC2 DC_3_n79 for TR 38.898, DoCoMo R4-2220468, TP for PC2 DC_19_n79 for TR 38.898, DoCoMo R4-2220469, TP for PC2 DC_21_n79 for TR 38.898, DoCoMo | 0.1.0 |
| 2023-03 | 3GPP RAN4#106 | R4-2301062 | Implemented TP’s from RAN4 #106: R4-2301302, TP for PC2 DC_1_n77-n79 for TR 38.898, NTT DOCOMO, INC. R4-2301303, TP for PC2 DC_3_n77-n79 for TR 38.898, NTT DOCOMO, INC. R4-2301304, TP for PC2 DC_21_n77-n79 for TR 38.898, NTT DOCOMO, INC. R4-2301305, TP for PC2 DC_1_n78-n79 for TR 38.898, NTT DOCOMO, INC. R4-2301306, TP for PC2 DC_3_n78-n79 for TR 38.898, NTT DOCOMO, INC. R4-2301307, TP for PC2 DC_21_n78-n79 for TR 38.898, NTT DOCOMO, INC. R4-2301309, TP for PC2 DC_1-21_n77 for TR 38.898, NTT DOCOMO, INC. R4-2301310, TP for PC2 DC_1-42_n77 for TR 38.898, NTT DOCOMO, INC. R4-2301311, TP for PC2 DC_3-21_n77 for TR 38.898, NTT DOCOMO, INC. R4-2301312, TP for PC2 DC_3-42_n77 for TR 38.898, NTT DOCOMO, INC. R4-2301313, TP for PC2 DC_21-42_n77 for TR 38.898, NTT DOCOMO, INC. R4-2303446, TP for PC2 DC_1_n77 for TR 38.898, NTT DOCOMO, INC. R4-2303447, TP for PC2 DC_3_n77 for TR 38.898, NTT DOCOMO, INC. R4-2303448, TP for PC2 DC_21_n77 for TR 38.898, NTT DOCOMO, INC. R4-2303449, TP for PC2 DC_21_n78 for TR 38.898, NTT DOCOMO, INC. R4-2303450, TP for PC2 DC_1-3_n77 for TR 38.898, NTT DOCOMO, INC. | 0.2.0 |
| 2023-04 | 3GPP RAN4#106bis-e | R4-2304858 | Implemented TP’s from RAN4 #106bis-e: R4-2306534, TP for TR 38.898: DC_1A_n41A, Samsung, KDDI, Qualcomm R4-2306535, TP for TR 38.898 DC_41A_n77A, Samsung, KDDI, Qualcomm R4-2305617, TP for PC2 DC_1-3_n78 for TR 38.898, NTT DOCOMO, INC. R4-2305618, TP for PC2 DC_1-42_n78 for TR 38.898, NTT DOCOMO, INC. R4-2305619, TP for PC2 DC_3-42_n78 for TR 38.898, NTT DOCOMO, INC. R4-2305620, TP for PC2 DC_21-42_n78 for TR 38.898, NTT DOCOMO, INC. R4-2305621, TP for PC2 DC_1-3_n79 for TR 38.898, NTT DOCOMO, INC. R4-2305622, TP for PC2 DC_1-19_n79 for TR 38.898, NTT DOCOMO, INC. R4-2305623, TP for PC2 DC_1-21_n79 for TR 38.898, NTT DOCOMO, INC. R4-2305624, TP for PC2 DC_1-42_n79 for TR 38.898, NTT DOCOMO, INC. R4-2305625, TP for PC2 DC_3-19_n79 for TR 38.898, NTT DOCOMO, INC. R4-2305626, TP for PC2 DC_3-21_n79 for TR 38.898, NTT DOCOMO, INC. R4-2306537, TP for PC2 DC_3-42_n79 for TR 38.898, NTT DOCOMO, INC. R4-2305628, TP for PC2 DC_19-21_n79 for TR 38.898, NTT DOCOMO, INC. R4-2305629, TP for PC2 DC_19-42_n79 for TR 38.898, NTT DOCOMO, INC. R4-2305630, TP for PC2 DC_21-42_n79 for TR 38.898, NTT DOCOMO, INC. | 0.3.0 |
| 2023-05 | 3GPP RAN4#107 | R4-2309388 | Implemented TP’s from RAN4 #107: R4-2308141, TP for TR 38.898 HPUE DC_28A_n41A, Samsung, KDDI R4-2308142, TP for TR 38.898 HPUE DC_28A_n77A, Samsung, KDDI R4-2309332, TP for PC2 DC_19_n77 for TR 38.898, NTT DOCOMO, INC., Qualcomm Inc., MediaTek Inc. R4-2309333, TP for PC2 DC_19_n78 for TR 38.898, NTT DOCOMO, INC., Qualcomm Inc., MediaTek Inc. R4-2309334, TP for PC2 DC_1-19_n77 for TR 38.898, NTT DOCOMO, INC., Qualcomm Inc., MediaTek Inc. R4-2309335, TP for PC2 DC_1-19_n78 for TR 38.898, NTT DOCOMO, INC., Qualcomm Inc., MediaTek Inc. R4-2309336, TP for PC2 DC_3-19_n77 for TR 38.898, NTT DOCOMO, INC., Qualcomm Inc., MediaTek Inc. R4-2309337, TP for PC2 DC_3-19_n78 for TR 38.898, NTT DOCOMO, INC., Qualcomm Inc., MediaTek Inc. R4-2309338, TP for PC2 DC_19-21_n77 for TR 38.898, NTT DOCOMO, INC. R4-2309339, TP for PC2 DC_19-21_n78 for TR 38.898, NTT DOCOMO, INC. R4-2309340, TP for PC2 DC_19-42_n77 for TR 38.898, NTT DOCOMO, INC. R4-2309341, TP for PC2 DC_19-42_n78 for TR 38.898, NTT DOCOMO, INC. R4-2309342, TP for PC2 DC_19_n77-n79 for TR 38.898, NTT DOCOMO, INC. R4-2309343, TP for PC2 DC_19_n78-n79 for TR 38.898, NTT DOCOMO, INC. R4-2309344, TP for PC2 DC_1-21_n78 for TR 38.898, NTT DOCOMO, INC., Qualcomm Inc., MediaTek Inc. R4-2309345, TP for PC2 DC_3-21_n78 for TR 38.898, NTT DOCOMO, INC., Qualcomm Inc., MediaTek Inc. R4-2310240, TP for TR 38.898: PC2 support for DC_3A_n1A-n78A, DC_3A-3A_n1A-n78A, DC_7A_n1A-n78A, DC_7A-7A_n1A-n78A, CHTTL | 0.4.0 |