TSG-SA Working Group 1 (Services) meeting #7	TSG S1 (00) 144
Sophia Antipolis, France 9 - 11 February 2000	Agenda Item: 5.15

| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 22.135 |  |  |  |  |  | CR |  |  |  |  |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  |  |  |  |
| GSM (AA.BB) or 3G (AA.BBB) specification number  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  CR number as allocated by MCC support team |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For submission to: |  |  |  | TSG-SA#7 |  |  |  |  | for approval |  |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  | X | (for SMG |  |  |  |
| list expected approval meeting # here  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  | use only) |  |  |  |
| Form: CR cover sheet, version 2 for 3GPP and SMG The latest version of this form is available from: ftp://ftp.3gpp.org/Information/CR-Form-v2.doc |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  |  |  | ME |  |  |  | X | UTRAN / Radio |  |  |  |  |  |  |  |  | X | Core Network |  |  |  |  |  |  | X |  |
| (at least one should be marked with an X) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source: |  |  | LM Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  | 2000-02-09 |  |  |  |  |  |
| Subject: |  |  | Introduction of SS style in stage 1 and simplifications for R99 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | Multicall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  | Release: |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
| (only one category | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
| shall be marked | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
| with an X) | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | X |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | In order to ensure correct handling of Multicall, it is proposed to write the specifications as for other supplementary services. It is proposed to simplify the requirements on handling of additional terminating calls, restricting the possibility to receive new call on one of the existing bearers. This would eliminate some problems caused with this requirement and make the R99 multicall simpler. It is proposed to define number of bearers as a limit, since that is what multicall is about. Limitation on number of calls does not need to be specified. It is proposed to have a requirement to invoke CW when there is one on-going speech, and the new terminating call is speech. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<--------- double-click here for help and instructions on how to create a CR.

3G TS 22.135 V3.1.0 (1999-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Multicall; Service description;

Stage 1

(3G TS 22.135 version 3.1.0)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTS/TSGSA-0122034U

Keywords

3GPP, SA

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

All rights reserved.

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 1999, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA,TTC).

Contents

Foreword	5

1	Scope	6

2	 References	6

2.1	Normative references	6

2.2	Informative references	6

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

4	Description	7

4.1	Description of multicall	7

4.2	Multicall service scenarios	8

4.2.1	Terminating CS call	9

4.2.2	Originating CS call	10

4.2.3	PS sessions	10

4.2.4 Connectionless traffic	10

4.4	Charging aspects	10

5	Functional requirements	11

5.1	Provision and withdrawal	11

5.1.1	Provision	11

5.1.2	Withdrawal	11

5.2	Limiting the number of multicalls	11

5.7	Busy Definition	11

5.8	Exceptional procedures or unsuccessful outcome	11

6	Interaction with other services	12

6.1	General on Supplementary Services	13

6.2	Line Identification	13

6.2.1	Calling Line Identification Presentation (CLIP)	13

6.2.2	Calling Line Identification Restriction (CLIR)	13

6.2.3	Connected Line Identification Presentation (COLP)	13

6.2.4	Connected Line Identification Restriction (COLR)	13

6.3	Call Forwarding	14

6.3.1	Call Forwarding Unconditional (CFU)	14

6.3.2	Call Forwarding on Busy (CFB)	14

6.3.3	Call Forwarding on No Reply (CFNRy)	14

6.3.4	Call Forwarding on Not Reachable (CFNRc)	14

6.4	Call Completion	14

6.4.1	 Call Hold (CH)	14

6.4.2	Call Waiting (CW)	14

6.5	Multi Party (MPTY)	15

6.6	Closed User Group (CUG)	15

6.7	Advice of Charge (AoC)	15

6.8	Call Barring	15

6.8.1	Barring of all outgoing calls	15

6.8.2	Barring of outgoing international calls	15

6.8.3	Barring of outgoing international calls except those directed to the HPLMN country	15

6.8.4	Barring of all incoming calls	15

6.8.5	Barring of incoming calls when roaming	15

6.9	Explicit Call Transfer (ECT)	15

6.10	Completion of Call to Busy Subscriber (CCBS)	16

6.11	Multiple Subscriber Profile (MSP)	16

6.12	Calling Name Presentation (CNAP)	16

6.13	User-to-User Signalling (UUS)	16

6.14	enhanced Multi-Level Precedence and Pre-emption service (eMLPP)	16

6.15	Call Deflection (CD)	16

6.16	CAMEL	16

6.17	IST	16

7	Cross Phase Compatibility for R99	17

7.1	 Compatibility With Existing Standards	17

7.2	Compatibility With Future Releases	17

7.2.1	Multicall configuration	17

7.2.2	Several simultaneous speech calls / bearers	18

7.2.3	CCBS	18

7.2.4	Registration	18

7.2.5	 Interrogation	18

Annex A (informative) : Change history	19

History	20

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TS, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

≥3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# 1	Scope

The present document describes the Multicall supplementary service for UMTS phase 1 release '99.

The general aspects, including definitions and recommended provision, of the description of the 3GPP Supplementary Services are given in 3GPP TS 22.004.

2		References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

## [1]	TS 22.100 “UMTS Phase 1 Release 99”

GSM 01.04 (ETR 350): " Abbreviations and acronyms".

[a]	GSM 22.004 (ETS 300 918): " General on Supplementary Services".

[a]	GSM 02.30 (ETS 300 907): "Digital cellular telecommunications system (Phase 2+); Man-Machine Interface (MMI) of the Mobile Station (MS)".

[a]	GSM 02.83: "Digital cellular telecommunication system; Call Waiting (CW) and Call Hold (HOLD) Supplementary Services - Stage 1".

[a]	GSM 02.85: "Digital cellular telecommunication system; Closed User Group (CUG) Supplementary Services - Stage 1".

[a]	GSM 02.01 Principles of telecommunication services supported by a GSM Public Land Mobile Network (PLMN)

3	Definitions, symbols and abbreviations

## 3.1	Definitions

CS Call: Circuit switched call. A call routed through CS domain. CS call can be for example a speech call, fax call or data call. One call shall only use one bearer at the time.

Multiparty: Supplementary Service for speech conference service.

Ncs: maximum number of simultaneous CS bearers.

Ncsmax: maximum number of simultaneous CS bearers as defined by the Multicall subscription.

NcsUser: maximum number of simultaneous CS bearers as defined by the user within the limits given by Ncsmax.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CS	circuit switched

# 4	Definition

This Multicall Supplementary Service provides a mobile subscriber with the ability to set up and receive several CS calls, each call utilizing an own dedicated bearer.

# 4	Description

## 4.1	Description

The Multicall supplementary service enables a mobile subscriber to set up several CS calls, each call utilizing an own dedicated bearer.

Maximum number of CS bearers used for speech is 1.

Note:	The protocol architecture in GSM allows several parallel CS calls, the limitation being that there is only one traffic channel, which the different CS calls share. This is facilitated by e.g. the Call Waiting, Call Hold, Call Transfer and Multiparty SSs. Call configurations related to GSM supplementary services are not considered as Multicall. See section 6 for interaction.

It shall be possible for each CS call to utilize a dedicated bearer of independent traffic and performance characteristics, with the restriction that only one speech bearer is allowed at a time. It shall be possible to release each active CS call  independently of any other CS call.

4.2	Applicability to telecommunication services

The applicability of this Supplementary Service is defined in 3GPP TS 22.004.

# 5	Normal procedures with successful outcome

## 5.1	Provision

Multicall is provided by prior arrangement with the home environment.

Multicall shall be provisioned for all Basic Services (BS) subscribed to and to which it is applicable, i.e. not to any subset of these BS.

When the multicall service is provisioned the maximum number of simultaneous CS beares available to the subscriber (Ncsmax) shall be defined as part of the subscription. The value for Ncsmax shall in the range from 2 up to 7.

## 5.2	Withdrawal

It shall be possible to withdraw the multicall service subscription on request of the subscriber or for administrative reasons.

## 5.3	Registration

The user shall be able to modify the maximum number of available CS bearers (NcsUser). The user shall be able to set the value for NcsUser in the range from 1 up to the value defined by Ncsmax. Provision of the Multicall service shall result in an implicit registration of a default value for NcsUser.

## 5.4	Erasure

Service data related to the Multicall service shall be erased by the service provider as a result of withdrawal.

## 5.5	Activation

The Multicall service shall be activated by the service provider as a result of provision.

## 5.6	Deactivation

The Multicall service shall be deactivated by the service provider as a result of withdrawal.

## 5.7	Invocation

The Multicall service shall be invoked by the system when at least one call is in progress and another call with a dedicated bearer is to be set up. The Multicall service applies to mobile originating and terminating calls.

## 5.8	Normal operation with successful outcome

## 5.8.1	Terminating CS call

The indication of a terminating CS call to the mobile terminal will be done until the maximum number of total CS calls (Ncs) has been reached.

If the maximum number of total CS bearers  (Ncs) has been reached an additional terminating call (Ncs+1) will encounter an NDUB condition. For the NDUB condition normal call handling and service invocation applies.

If a terminating call is indicated to the user he may reacted in the following way:

a)	accept the terminating call

. A new bearer is allocated for the terminating call.

b)	reject the terminating call

The call shall be released in the normal way

c)	ignore the incoming call

If the user/user application ignores (i.e. neither accepts nor rejects) the indication of the terminating call the terminating call, the normal call handling shall apply.

### 5.8.2	Originating CS call

If the Ncs  is not reached already and the user/user application wants to establish a new originating CS call

a new bearer shall be allocated for the originating call

5.9	Quality of service

## 5.10	Testing

## 5.11	Interrogation

The user shall be able to request data related to the Multicall service. This data request procedure enables the mobile subscriber to obtain information about the data stored for the Multicall service. After having requested this procedure the system shall return the information about the following data:

-	NcsUser;

- NcsMax;

- Ncs.

## 5.4	Charging requirements

Each of the calls owning a dedicated bearer shall be charged individually.

Allocation of a dedicated bearer shall be visible in the charging data.

6	Exceptional procedures or unsuccessful outcome

## 6.1	Exceptional operation or unsuccessful outcome

The number of simultaneous CS bearers supported by the system may vary in different regions. For instance, after handover the number simultaneous bearers available to the served subscriber may be lower than the number of already established bearers. Such events shall trigger changes to individual calls in any multicall scenario.

Priority setting for CS Calls shall influence the handover process. It shall be possible to handover all calls. If the target cell is not able to accommodate all calls, then the calls to be handed over shall be selected in following order and the calls that cannot be handed over will be released:

- The call of teleservice emergency call.

- If the user has set the priority, the call of highest applied priority

- The call of teleservice telephony

If no single call can be selected according to the above criteria, the treatment of how to continue is a network option.

Note:	Requirements shall be considered in an intra- and inter-system (e.g. UMTS to GSM) handover situations. In the case of intra UMTS handover it shall be possible to handover all calls when resources permit. In the case of UMTS to GSM handover only one call can be handed over

## 6.2	Registration

If the system cannot accept a registration request, the served mobile subscriber shall receive a notification that Multicall registration was not successful. Possible causes are:

-	service not subscribed to;

-	NcsUser > NcsMax;

-	insufficient information;

## 6.3	Erasure

No exceptional operation identified.

## 6.4	Activation

No exceptional operation identified.

## 6.5	Deactivation

No exceptional operation identified.

## 6.6	Invocation

If the served subscriber attempts to set up a new call with a dedicated bearer while the maximum number of bearers available to the subscriber (Ncs) has been reached, the call attempt shall be rejected by normal procedures, indicating that no more bearers are available.

## 6.7	Testing

-

## 6.8	Interrogation

When a mobile subscriber who is not provided witht the Multicall service interrogates the service data of the Multicall service, he shall be notified that he is not subscribed to the service.

## 6.9	Charging requirements

## 6.10	Roaming in non-supporting networks

Roaming into networks not supporting the Multicall service shall be possible. The served subscriber will not have any access to the Multicall service and the system should behave accordingly.

When the served subscriber performs any procedure related to Multicall service as described in the clauses above while roaming in a non-supporting network, he shall be notified that the service is not available.

# 7	Alternate procedures

# 7	Interaction with other supplementary services

## 7.1	Line Identification

### 7.2.1	Calling Line Identification Presentation (CLIP)

No impact, i.e. CLIP shall be provided with all calls.

### 7.2.2	Calling Line Identification Restriction (CLIR)

No impact, i.e. CLIR shall be provided with all calls.

### 7.2.3	Connected Line Identification Presentation (COLP)

No impact, i.e. COLP shall be provided with all calls.

### 7.2.4	Connected Line Identification Restriction (COLR)

No impact, i.e. COLR shall be provided with all calls.

## 7.3	Call Forwarding

### 7.3.1	Call Forwarding Unconditional (CFU)

No impact.

### 7.3.2	Call Forwarding on Busy (CFB)

No impact.

Note: CFB shall be invoked when the network detects a NDUB condition. The maximum number of bearers available to the subscriber (NCS) affects the NDUB condition.

This means that for the case of NCS = 1, the system behaviour shall be the same as for a subscriber who is not provided with the Multicall service.

### 7.3.3	Call Forwarding on No Reply (CFNRy)

No impact.

### 7.3.4	Call Forwarding on Not Reachable (CFNRc)

No impact.

## 7.4	Call Completion

### 7.4.1		Call Hold (CH)

Irrespective of whether the maximum number of CS bearers (Ncs) is reached or not, and a terminating call is indicated to the user, he shall be able to accept the terminating call by applying the Call Hold service to one of the already established calls. By that he makes the bearer of the held call available to the new call.

The Call Hold service shall be implicitly invoked when the served subscriber requests the call set up of a mobile originating call while there is an active speech call in progress and the maximum number of bearers available to the subscriber (NCS) is reached.

Note: This means that for the case of NCS = 1, the system behaviour shall be the same as for a subscriber who is not provided with the Multicall service.

7.4.2	Call Waiting (CW)

When the maximum number of CS bearers (Ncs) is reached and a terminating call is received for the subscriber, and the subscriber has CW active, the Call Waiting service shall be invoked according to the Call Waiting service description in 3GPP TS 22.083 with the following exception: The Call Waiting service is invoked when it is active for at least one of the Basic Services provided by the ongoing CS calls.

When the maximum number of CS bearers used for speech is reached, and a new terminating call is speech, CW shall be invoked if the subscriber has CW active.

This means that for the case of NCS = 1, the system behaviour shall be the same as for a subscriber who is not provided with the Multicall service.

7.5	Multi Party (MPTY)

No Impact.

The number of MPTY member may be limited because of number of existing CS calls.

## 7.6	Closed User Group (CUG)

No impact.

## 7.7	Advice of Charge (AoC)

The subscriber shall receive the AoC indication individually for each CS call.

For the AoCC service, the ACM (Accumulated Call Meter as defined in TS 22.024) shall reckon the sum of the charge generated by all simultaneous CS calls.

7.8	Call Barring

No impact.

### 7.8.1	Barring of all outgoing calls

No impact.

### 7.8.2	Barring of outgoing international calls

No impact.

### 7.8.3	Barring of outgoing international calls except those directed to the HPLMN country

No impact.

### 7.8.4	Barring of all incoming calls

No impact.

### 7.8.5	Barring of incoming calls when roaming

No impact.

## 7.9	Explicit Call Transfer (ECT)

No impact.

## 7.10	Completion of Call to Busy Subscriber (CCBS)

If the maximum number of CS bearers (Ncs) is not reached and a CCBS recall is indicated to the user, he shall be able to accept the CCBS recall and initiate a CCBS call with a dedicated bearer.

Irrespective of whether the maximum number of CS bearers (Ncs) is reached or not, and a CCBS recall is indicated to the user, he shall be able to accept the CCBS recall and initiate a CCBS call by applying the Call Hold service to one of the already established calls. By that he makes the bearer of the held call available to the CCBS call.

7.11	Multiple Subscriber Profile (MSP)

No impact.

## 7.12	Calling Name Presentation (CNAP)

No impact.

## 7.13	User-to-User Signalling (UUS)

No Impact

## 7.14	enhanced Multi-Level Precedence and Pre-emption service (eMLPP)

The eMLPP service shall control which of the ongoing calls survive a handover to a system area where maximum number of CS bearers Ncs is lower than the number of  on-going calls. See section on Handover and section on Roaming in non-supporting networks.

Otherwise, no impact.

7.15	Call Deflection (CD)

No impact.

### 7.16b	Multicall

No impact.

# Interworking considerations

# Dynamic description of service

# 12	Outstanding issues

# 7	Cross Phase Compatibility for R99

This section details the cross phase compatibility requirements relating to the service requirements in this document.

Note: when a change is introduced which affects the 3GPP specifications, it is said to be 'backward compatible' if existing equipment can continue to operate and perform correctly with equipment that conforms to the new implementation.

## 7.1		Compatibility With Existing Standards

Where the service and operational requirements in this document relate to a core network functionality, compatibility is required.

Multicall is not applicable for GSM radio access.

## 7.2	Compatibility With Future Releases

It is envisaged that 3GPP standards will evolve beyond R99, for example with the addition of new service requirements. The standards, which define the technical implementation of R99, should be developed in such a way that it is practical to add the requirements in this section in a backward compatible manner.

Following chapters include requirements that are foreseen for future releases.

### 7.2.1	Multicall configuration

When having one active CS call and one held call on the same bearer, it shall be possible to create a new CS bearer and to move one of the calls to the new bearer, resulting both calls being active within the limits set by the operator/user and within the capability of the terminal. See figure 2: Split of bearer.

When having two calls (multicall) on the separate bearers, it shall be possible to join both calls to one of the two bearers, put the one of the calls to hold and to release unused CS bearer. It shall be possible to select which call to put on hold. See figure 2: Combination of bearers. (Note: there is no clear end-user service requirement for this feature at time being.)

NOTE: Due to that only speech calls can be put on hold, so one of the two active Cs calls has to be a speech call

Figure2: Illustration for split of bearer and combination of bearers.

### 7.2.2	Several simultaneous speech calls / bearers

A key requirements for multicall is to allow several simultaneous CS calls with dedicated bearers. The most important usage scenario is to allow several CS data bearers to be bind at application level resulting to higher than 64kbits/s data rates. Another important feature is to allowing simultaneous speech and data calls.

UMTS release 99 does only support one active speech call at a time. However, Call control should not prohibit a complete set of multiple speech bearer services in future releases and UTRAN shall be designed in a flexible way to support multiple speech bearers. In release 99, Supplementary Services Call Waiting, Multiparty and Call Hold are used to offer simultaneous speech calls to user.

If multiple simultaneous voice calls are supported in the future then the Call Hold service may be used to re-configure the number of bearers supporting voice calls if required during handover, e.g. in the case of handover to GSM where only one voice call can be active at a time. This requirement is dependent on the user subscribing to Call Hold.

### 7.2.3	CCBS

In the future releases the definition of IDLE state of subscriber A and destination B should be modified in away that the IDLE state is reach even if there are active CS calls but the maximum limit of CS calls is not reached.

### Annex A (informative) :
Change history

| Change history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SMG No. / TSG SA# | TDoc. No. | CR. No. | Section affected | New version | Subject/Comments |
| TSG SA#5 | SP99-433 |  |  | Version 3.0.0 | Approved |
| TSGT SA#6 | SP99-523 | 001 | 5.6 | Version 3.1.0 | Transfer of Handover chapter to 22.129 |
| TSGT SA#6 | SP99-523 | 002 | 4.2.1, 5.1.1, 5.4, 5.7, 7.2.2 | Version 3.1.0 | Clarification on handling of multiple bearers |
| TSGT SA#6 | SP99-554 | 003 | 3, 5, 7 | Version 3.1.0 | Registration, Interrogation and Restriction of Packet Domain |

# History

| Document history |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date | Version | Comment |  |  |  |
| 19 Mar. 1999 | 0.0.1 | First draft by the editor (Tommi Kokkola / Nokia) |  |  |  |
| 30 Mar. 1999 | 0.1.0 | Output from S1 Multicall ad hoc |  |  |  |
| 6 Apr. 1999 | 0.1.0 | Minor modifications by the editor. Proposed version 1.0.0 |  |  |  |
| 19 Apr. 1999 | 1.0.0 | For information to 3GPP TSG SA#3 |  |  |  |
| 10. Jun. 1999 | 1.1.0 | Draft from editor for the email meeting. |  |  |  |
| 18. Jun. 1999 | 1.1.1 | Draft during the email meeting. |  |  |  |
| 24. Jun .1999 | 1.2.0 | Result of email discussions. For some issues status still unstable. |  |  |  |
| 1. Jul .1999 | 1.2.1 | Comments from TSGN & TSGS Multimedia and Multicall joint meeting included. (Revisions included from version 1.1.0) |  |  |  |
| 8. Jul. 1999 | 1.3.0 | Results from S1 Plenary. Drafting continues at S1_Multicall mailing lists. Contributions expected on: -Busy, Idle, Active states –Busy definition –Supplementary service interactions incl. related CR to appropriate SS when needed. (Specifically 02.83 need to be studied.) – all topics marked with FFS… |  |  |  |
| 28. Sep. 1999 | 1.4.0 | Output from S1 Multicall ad hoc, Germany. |  |  |  |
| 1.Oct. 1999 | 1.6.0 | Proposed version 2.0.0 |  |  |  |
| 10. Oct 1999 | 2.0.0 | Version 2.0.0 |  |  |  |
| October 99 | 3.0.0 | Stage 1 approved at SA#5, Kyongju, Korea |  |  |  |
| December 199 | 3.1.0 | Inclusion of CRs approved at SA#5, Nice, France |  |  |  |