3GPP TSG-SA WG1 Meeting #109 	S1-250290

Athens, Greece, 17-21 February 2024	(revision of S1-250xxx)

Title:	TR 22.870 pCR Addition of Resilience vs. Robustness

Agenda Item:	8.X.Y (6G study - XXX)

Source:	Samsung

Contact:	Erik Guttman <erik.guttman@samsung.com>

Abstract: Distinguishing between resilience and robustness will help to identify an important difference between 6G and past generations.

Introduction

The term 'resiliency' means more than 'robustness'. Robustness aims at higher service availability through mechanisms that are applied to known problems - for example through retry timers and implicit deregistration, expanding radius paging, the 3GPP system services are robust to intermittent reachability of mobile devices. Additional standardized features to increase service availability come essentially down to two different approaches:

-	additional robustness through standards that alleviate known causes of service interruption

-	additional resiliency through standards that support unknown service failure modes, including e.g. recovery from disasters, energy outages, etc.

Some features considered by SA1 in the context of 5G and before have already begun to address service resiliency, e.g. uniform access control as a means to restore control of operations in networks and SEI requirements for coordinated mobile network and energy system recovery procedures that take into account the interdependence of mobile telecommunications and energy distribution services.

In 6G, higher service availability is a central aspiration. Improvements to robustness against known risks are important as they can increase service availability. This is however different than introducing new functionality to enable insight, recovery and alternative approaches to respond to as yet unknown problems. For this purpose, it is proposed to use the term resilience.

Discussion

Resiliency is often used synonymously with 'robustness' - that is, the system becomes more resilient when it has higher availability. Resiliency can be used this way, but the term has a broader meaning.

Resiliency according to the UN Sustainable Development Group is defined as:

Resilience is the ability of individuals, households, communities, cities, institutions, systems and societies to prevent, resist, absorb, adapt, respond and recover positively, efficiently and effectively when faced with a wide range of risks, while maintaining an acceptable level of functioning without compromising long-term prospects for sustainable development, peace and security, human rights and well-being for all. [1]

There are at least three aspects to resilience which we should consider in 6G:

a)	the capability to respond to risks and incidents which cannot be foreseen. This goes beyond 'hardening', that is, features that respond to common issues faced by operations such as increased demand for capacity.

b)	we need to consider use cases in which incidents that cause lack of availability can be dealt with by service users before, during the incident and after the incident, so as to avoid, get through and adapt to the consequences of  interruptions in availability.

c)	existing capabilities to expose information and coping mechanisms can be reviewed and possibly expanded in 6G.

The 5G system identifies quite a few high availability performance requirements, while the means to achieve this metric is generally left 'to implementation', which is to say, outside of the scope of standardization. Using best effort mobile data service, 99.99% service availability is generally not achieved. There is clearly a gap between aspirations for 99.99% and greater service availability and what is delivered to vertical sector stakeholders.

This observation was made in the FS_5GSEI study, especially in use case 5.7 Remote DSO management of connectivity for Smart Energy and  5.18 Coordination for Energy Recovery Use Case. In the NSOEU feature and some NEF capabilities of information offered by NWDAF, a third party can identify a decline in performance and take proactive steps to reduce reliance on communication services that see at increased risk of outage. Further, in NSOEU, a capability was added to coordinate energy service recovery between energy service utility operators and mobile network operators. These is an example of 'proactive' and 'reactive' response. Additional SEI requirements for 'reporting performance from the customer to the operator' were never implemented, but could be part of standardized support for resiliency in the form of adaptation 'after an incident.'

Other features besides SEI and NSOEU that could be considered as part of a 'toolbox for resilient response to incidents' include network recovery procedures, congestion response (such as UAC, congestion control, etc.) and MINT.

Resilience Engineering is a field that brings together safety experts from fields of healthcare, nuclear power, aviation and other areas, concerned with how to 'reframe an organization's model of how safety is created before accidents occur by developing measures and indicators of contributors to resilience such as the properties of buffers, flexibility, precariousness and tolerance and patters of interactions across scales such as responsibility-authority double binds.' [2] There are limits to the degree to which automated optimization and recovery mechanisms succeed in crises, and diverse means that can be introduced to avoid incidents, identify the conditions of an incident to enable rapid recovery, and to learn from incidents.

The applicability of Resilience Engineering principles to the 6G system should be studied. This pCR introduces the term resilience as well as the general concept. Future input will provide use cases and assessment of existing capabilities of the 3GPP system that support resiliency.

References in discussion section

[1]	United Nations Sustainable Development Group, "Executive Summary, UN Common Guidance on Helping Build Resilient Societies", dhttps://unsdg.un.org/sites/default/files/2021-09/UN-Resilience-Guidance-Exec-Summ-Sept.pdf

[2] Hollnagel, E., Woods, D., Leveson, N., "Resilience Engineering: Concepts and Precepts", CBC Press, 2006.

Proposal

It is proposed to make the change listed below to TR 22.870, 0.1.1.

Begin changes

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

resilience: The capability for a service or system to prevent, resist, absorb, adapt, respond and recover positively, efficiently and effectively when faced with a wide range of risks and adverse conditions both known and as yet unknown, while maintaining an acceptable level of functioning.

robustness: The capability of a service or system to reduce known risks to service interruption, to maintain an acceptable level of functioning over time.

Next change

# 5	System and Operational Aspects

Editor's Note: "System and Operational Aspects" facilitates system and network operation features that underpin overall operation, covering aspects that apply across use cases and services, and those that relate to network operations. These aspects include, for example: migration scenarios, interworking with earlier 3GPP systems, interworking with non-3GPP system, roaming and interconnection, network simplification, network sharing, security, privacy, resilience, robustness, sustainability and energy efficiency, device diversity, support of legacy services

Next change

## 8.4	Use case on resilient positioning in satellite networks

### 8.4.1	Description

Positioning, Navigation and Timing (PNT) is fundamental for a wide range of sectors, such as transportation, critical infrastructure (e.g. 6G system) and emergency services. Nowadays, GNSS is the backbone technology to fulfil these PNT needs. However, the dependency of these sectors on GNSS certainly results in major threats and risks, in case of unavailability or disruption of GNSS, with severe safety, legal and economic impacts. Existing GNSS services are out of the scope of the current 3GPP system, so increasing the robustness of GNSS itself is not possible through 3GPP standardization.

Since so many components of the 6G system require PNT services, it is proposed to standardize a solution within 3GPP. This capability can be designed with robustness in mind. The resilient positioning use case is based on the demand to withstand and recover rapidly from PNT disruptions anywhere - by offering an alternative when other GNSS services are not available.

In addition, the 6G system may assume the need to obtain the UE location with a valid GNSS solution for the operation of 6G NTN solutions. In such situation, the degradation or unavailability of GNSS positioning, such as due to jamming, spoofing or obstructions, may lead to a degradation or loss of the 6G NTN communication capability, as it may currently occur in 5G NTN operations. For 6G NTN, there may be alternative communication approaches, but if these approaches are limited or not successful, a 6G integrated communication and positioning system can provide a GNSS-independent positioning solution for 6G NTN communications.

The satellite network operator of the 6G system is able to provide the resilient positioning service, by ensuring the availability of 3GPP positioning technologies under the service area, and independently of the availability of non-3GPP technologies (e.g. GNSS). This robust positioning service can then be exploited by any kind of 6G user (e.g. aircrafts and drones [25]) that may experience GNSS degraded service.

### 8.4.2	Pre-conditions

The UE has satellite network coverage (e.g. LEO constellation) in the 6G system.

The UE can be unregistered or registered (e.g. in idle or connected mode) to the satellite network.

The UE can be under the coverage of multiple or multi-orbit satellite networks (e.g. LEO and GEO constellations) deployed by one or more operators and have subscription(s) for accessing these satellite networks.

The environment of use is outdoor static or moving with coverage provided by the satellite network(s).

The UE is equipped with a native 6G communication and positioning function. The positioning function relies on 3GPP technologies delivered by the satellite network, to avoid any dependence on the availability of non-3GPP technologies.

### 8.4.3	Service Flows

1. The 3GPP positioning service provided by the satellite network can be initiated by a third party or by the end-user.

2. This 3GPP positioning service can determine the UE location with a certain resilient accuracy level.

3. When triggering the 3GPP positioning service, if the UE fails to obtain the location data within a time limit or the location data is not accurate enough, the UE can select different satellite networks or improve the accuracy of location data combining multiple satellite networks.

4. The UE location is used to provide the resilient positioning service.

5. The UE can report its UE location and positioning accuracy level.

### 8.4.4	Post-conditions

The user successfully obtains the resilient positioning service.

### 8.4.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [14] clause 6.27.1 includes the following description:

5G positioning services aims to support verticals and applications with positioning accuracies better than 10 meters, thus more accurate than the ones of TS 22.071 [24] for LCS. High accuracy positioning is characterized by ambitious system requirements for positioning accuracy in many verticals and applications, including regulatory needs.

3GPP TS 22.261 [14] clause 6.27.2 includes the following positioning requirements:

The 5G system shall provide different 5G positioning services, supported by different single and hybrid positioning methods to supply absolute and relative positioning.

NOTE 2: 	hybrid positioning methods include both the combination of 3GPP positioning technologies and the combination of 3GPP positioning technologies with non-3GPP positioning technologies such as, GNSS (e.g. Beidou, Galileo, GPS, Glonass), Network-based Assisted GNSS and High-Accuracy GNSS, Terrestrial Beacon Systems, dead-reckoning sensors (e.g. IMU, barometer), WLAN/Bluetooth-based positioning.

The 5G system shall support mechanisms to determine the UE's position-related data for period when the UE is outside the coverage of 3GPP RAT-dependent positioning technologies but within the 5G positioning service area (e.g. within the coverage of satellite access).

However, these requirements do not ensure resilience of the 3GPP positioning technologies in satellite networks, independently of the availability of non-3GPP technologies (e.g. GNSS).

3GPP TS 22.261 [14] clause 7.3.2.1 includes the following performance requirements:

The 5G system shall support the combination of 3GPP and non-3GPP positioning technologies to achieve performances of the 5G positioning services better than those achieved using only 3GPP positioning technologies.

NOTE 1:	For instance, the combination of 3GPP positioning technologies with non-3GPP positioning technologies such as GNSS (e.g. Beidou, Galileo, GLONASS, and GPS), Terrestrial Beacon Systems (TBS), sensors (e.g. barometer, IMU), WLAN/Bluetooth-based positioning, can support the improvement of accuracy, positioning service availability, reliability and/or confidence level, the reduction of positioning service latency, the increase of the update rate of the position-related data, increase the coverage (service area).

NOTE 2:	The combination can vary over time to optimise the performances, and can be the combination of multiple positioning technologies at the same epoch and/or the combination of multiple positioning technologies at different epochs.

3GPP TS 22.261 [14] clause 7.3.2.2 includes the following requirements for horizontal and vertical positioning service levels:

The 5G system shall be able to provide positioning services with the performances requirements reported in Table 7.3.2.2-1.

However, these performance requirements are not fully independent of non-3GPP positioning technologies, since a combination of 3GPP and non-3GPP positioning technologies can be used to fulfil these performance requirements.

End of changes