3GPP TSG-SA WG1 Meeting #109 	S1-250843

Athens, Greece, 17-21 February 2025	(revision of S1-250170, 816)

Source:	CATT

pCR Title:	Use Case on Assisted Airspace Management of UAV and UAM Aircrafts

Draft Spec:	3GPP TR 22.870 v0.1.1

Agenda item:	8.1.7

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes a use case about logging the service data such as sensing data, positioning measurement data, sensing results in the 6G network to assist the airspace management of UAV and UAM aircrafts.

1. Introduction

The application of UAVs in the fields related to people's daily and the development of passenger-carrying aerial vehicle (e.g. autonomous eVTOL) raise higher demand on the security and reliability of managing aircrafts, and bring new challenges to the regulatory authorities. Although many countries have enacted associated regulations as well as setup management platform to facilitate the airspace management, there are still practical issues to be solved, such as the retrieval of flight data for the accident analysis. This contribution introduces a use case that the 6G network stores the service data on demand to assist the airspace management of UAV and UAM aircrafts.

2. Reason for Change

For the 6G network that can provide flight assistant service such as sensing services, positioning services, introduce the use case on logging the service data in the 6G network.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.1.1

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1] 	NASA. UTM: Air traffic management for low-altitude drones[EB/OL] (2015-10) (https://www.nasa.gov/sites/default/files/atoms/files/utm-factsheet-11-05-15.pdf)

[x2] 	SESAR. U-space blueprint[EB/OL]. (2017-06-09). (https://www.sesarju.eu/sites/default/files/documents/reports/U-Space%20Blueprint%20brochure%20final.pdf)

[x3] 	AOPA China. Regulations for the operation of light and small UAVs [EB/OL] (2018-04-19). (http://www.aopa.org.cn/usr/1/upload/file/20180419/15241267234030958.pdf)

[x4] 	HIROYUKI U. UTM project in Japan[EB/OL]. (2017-06-26)[2025-01-20]. (https://gutma.org/montreal-2017/wp-ontent/uploads/sites/2/2017/07/UTM-Project-in-Japan_METI.pdf)

* * * Next Change * * * *

11.x	Use case on assisted airspace management of UAV and UAM aircrafts

11.x.1	Description

Over the past decades, uncrewed aerial vehicles (UAVs) have been widely applied in many industries and are expected to be applied into the fields related to people's daily lives, such as last-mile delivery and urban governance. Meanwhile, the passenger-carrying aerial vehicle (e.g. autonomous eVTOL) is rapidly developing and expected to become a critical part of urban air mobility (UAM) in the near future.

The complexity of future low-attitude aerial transportation (including UAV, UAM aircraft) has brought new challenges to the regulatory authorities. For instance, the autonomy of UAVs and UAM aircrafts increase the risk of loss of control, illegal flights and the collision with other aircrafts, which could potentially threaten the aerial and public safety, and thus request effective airspace and traffic management. Moreover, the technical failure, cybersecurity concerns and the need to carry passengers impose higher demands on the security and reliability of managing these aircrafts.

To address these challenges, many countries have enacted regulations and established individual management platform to facilitate the supervision. For example, the FAA (Federal Aviation Administration, USA) has issued a series of acts (e.g. “Safe Drone Act of 2017”, “FAA Reauthorization Act of 2018”) and developed UTM (Unmanned Aircraft System Traffic Management) framework to improve the regulations and airspace integration.[x1] The EASA (European Union Aviation Safety Agency, EU) has issued the UAS regulation(2019/947) and provided U-Space for safety supervision of drones and eVTOL [x2]. The CAAC (Civil Aviation Administration of China) has issued “Civil Aviation Law” and “Interim regulations on the UAV management” including the introduction of UAS Management platform for dynamic supervision and monitoring [x3]. Besides, the Japan’s FIMS (Flight Information Management System) can manage the flight plans, handle the alarms and send commands[x4]. Correspondingly, the UAVs and UAM aircrafts are requested to report the flight data to the remote pilot platform on demand, and also keep the flight data locally or in cloud for backtracking when needed (e.g. recording the access to certain areas). However, the airspace management still faces the issues like

- Incompatible rules: the cross-border flight requires standard rules to resolve the conflict of the regulations of different regions or nations;

- Insufficient compliant check: there are limited resources to monitor rogue operators, illegal flights, and out-of-control UAVs /UAM aircrafts that do not comply with the regulations;

- High cost and security risk of retrieving the flight data: it’s mandatory for the aircraft or its operator to store and report the flight data on demand. However, when an aircraft crashes or is unexpectedly lost, it can be difficult or costly to retrieve the operation data from the moment before the accident. Additionally, the stored data may be tampered with and untrusted.

Therefore, it’s expected that more technical tools will be used to solve the above issues and improve the management efficiency of UAV/UAM aircraft.

The 5G network has already supported some UAV-specific services such as remote identification, monitoring the location and status information, reporting network conditions and QoS along a flight path at specific times as illustrated in TS 22.125[35]. The use cases about UAV intrusion detection and flight trajectory tracing using 3GPP sensing service were discussed in TR 22.837[9], and the associated capabilities of 3GPP sensing and the result reporting (e.g. location, relevant distance between UAVs and other objects) to UTM and UAVs were introduced to TS22.137[6] and TS22.125[35]. The 6G system, besides of the above services, has big potentials to offer more assisted services for the airspace management of UAVs and UAM aircrafts under the coordination of the remote pilot platform. For example, logging the flight data acquired from 3GPP services (e.g. positioning service, sensing service) on demand will help to ensure the data integrity in case that the local data is damaged. Moreover, logging the service intermediate data can support additional data mining when needed e.g. for the issue analysis.

Figure 11.x.1-1 Assisted Airspace Management of UAVs and UAM Aircrafts

11.x.2	Pre-conditions

The 6G network operator “FlyNet” has the agreement with UAV/UAM operator “AirExpress” to provide a series of flight assistant services for its aircrafts, such as:

- Full coverage of communication services along the planned routes of the aircrafts；

- Sensing services for flight trajectory tracing, collision prediction, etc.

- High-accuracy positioning services

- Edge computing services and storage service.

FlyNet’s services comply with regulations and authorized by AirExpress for the data access to the regulatory authority.

AirExpress uses UTM to manage its operated aircrafts and provide remote piloting.

11.x.3	Service Flows

- A UAV/UAM aircraft connects to UTM via the 6G network and exchange the data about the flight tasks.

- UTM requests the 6G network to provide sensing service and positioning service to the authorized aircrafts for a certain period of time and in specific areas for the flight trajectory planning and tracing.

- The 6G core network collects the sensing data and positioning measurement data form the UEs/ the base stations in the specific areas, processes the data into sensing results and the location information, and then transmits the processing results to the aircraft for route planning as well as exposes them to UTM for tracing. In parallel, the 6G network can store the service intermediate data (e.g. sensing data, positioning measurement data) and processing result (e.g. sensing results, and location information) in the network for a period as requested by UTM and/or by the regulatory authority.

- Suddenly, the aircraft loses the connection with UTM and “disappears”. Later, UTM identifies that the aircraft has collided into the power grid infrastructure and crashed into pieces.

- AirExpress requests the 6G network to process the sensing data within the accident period based on the new request (e.g. taking the power grid, environment as the sensing target, based on new trajectory planning model), in order to investigate the root cause of the collision. Also, the regulatory authority retrieves the flight information (e.g. allowed service intermediate data, processing result) and operation data of the aircraft from 6G network and UTM separately to evaluate the responsibilities of the accident.

11.x.4	Post-conditions

Thanks to the in-network data logging service, AirExpress identifies the root cause of the collision (e.g. a fault of the prediction algorithms or operational decision). The regulatory authority justifies the responsibility of the accident (e.g. the aircraft exceeds the speed limit).

11.x.5	Existing features partly or fully covering the use case functionality

TS 22.137 and TS 22.261 have defined the functional and performance requirements for 3GPP sensing services and positioning services. And the information exposure of sensing results and positioning information is included.

TS 22.125 has defined the requirements for remote identification of UAS, UAV traffic and network exposure for UAV services.

11.x.6	Potential New Requirements needed to support the use case

[PR 11.x.6-1] Subject to regulatory requirements and user consent, the 6G network shall be able to aggregate and securely store the service data for a UAV or a UAM aircraft based on the request information (e.g. service type, storage duration, time expiry).

NOTE: The service data may be the processing data for a specific service such as sensing data, positioning measurement data, or the exposed data such as sensing results, positioning information.

[PR 11.x.6-2] Subject to the user consent, the 6G network shall provide secure means for an authorized 3rd party application to derive the information related to a UAV or a UAM aircraft from the stored service data e.g. for regulatory purpose.

* * * End of Changes * * * *