3GPP TSG SA WG 1 Meeting #110 	S1-252016

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Source:	6G Rapporteurs

pCR Title:	Pseudo-CR on Roaming and Interconnect (5.2.1)

Draft Spec:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Xiaonan Shi (shixiaonan@chinamobile.com) and Jean Trakinat (jean.trakinat1@t-mobile.com)

Abstract: Initial roaming and interconnection text.

1. Introduction

6G system roaming and interconnection requirements have not been discussed and agreed.  The Rapporteurs offer this text as a basis to find consensus on this topic.

2. Reason for Change

Requirements in TS 22.261 that address roaming and interconnection are in several clauses.  There are more clauses that refer to roaming and roaming subscribers.  The Rapporteurs used text found in TS 22.261 clauses 5.1.2.1, (shown below) and included aspects and requirements from 6.1.2.1 (Network Slicing), 6.30 (Steering of Roaming), Disaster Roaming (clause 6.31.2.3), Roaming Value Added Services (clause 6.44), Support of Roaming services providers (clause 6.45), and Satellite access Roaming aspects (clause 6.46.4).

Clause 5.1.2.1	Interworking between 5G systems

The 5G system shall support a UE with a 5G subscription roaming into a 5G Visited Mobile Network which has a roaming agreement with the UE's 5G Home Mobile Network.

The 5G system shall enable a Visited Mobile Network to provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

The 5G system shall enable a Visited Mobile Network to provide support for services provided in the home network as well as provide services in the visited network. Whether a service is provided in the visited network or in the home network is determined on a service by service basis.

The 5G system shall provide a mechanism for a network operator to limit access to its services for a roaming UE, (e.g. based on roaming agreement).

The 5G system shall provide a mechanism for a network operator to direct a UE onto a partnership network for routing all or some of the UE user plane and associated control plane traffic over the partnership network, subject to an agreement between the operators.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870v0.2.1.

* * * First Change * * * *

### 5.2.1	Roaming and Interconnection

Editor’s Note:  Text to be provided. This clause to address potential (new) requirements for roaming, interconnection(s), and exceptions (i.e., interworking explicitly not supported).

#### 5.2.1.1	Description

Mobile subscribers will continue to require service not only when they are in their home networks but also when they cannot be directly connected to their home networks.  It is expected that 6G will enable mobile subscribers to access services (e.g., voice, data) using a variety of access technologies and networks.

It is expected that 6G networks will support roaming and interconnection with other 6G networks similar to the roaming and interconnection between 5G networks.

It is expected that 6G mobile network operators will have agreements to provide service to subscribers of other mobile network operators.

#### 5.2.1.2	Potential New Requirements

[PR 5.2.1.2-001] The 6G system shall support a UE with a 6G subscription roaming into a 6G Visited Mobile Network which has a roaming agreement with the UE’s 6G Home Mobile Network.

[PR 5.2.1.2-001a] The 6G system shall enable a Visited Mobile Network to provide support for establishing home network provided data connectivity as well as visited network provided data connectivity.

[PR 5.2.1.2-002] The 6G system shall enable a Visited Mobile Network to provide support for services provided in the home network as well as provide services in the visited network. Whether a service is provided in the visited network or in the home network is determined on a service-by-service basis.

[PR 5.2.1.2-003] The 5G system shall support a UE with a 5G subscription roaming into a 6G Visited Mobile Network, which has a roaming agreement with the UE’s 5G Home Mobile network, to obtain legacy 5G voice and data services.

[PR 5.2.1.2-004] The 6G system shall provide a mechanism for a network operator to limit access to its services for a roaming UE (e.g. based on roaming agreement).

[PR 5.2.1.2-005] The 6G system shall provide a mechanism for a network operator to direct a UE onto a partnership network for routing all or some of the UE user plane and associated control plane traffic over the partnership network, subject to an agreement between the operators.

[PR 5.2.1.2-005a] The 6G system shall support the existing 5G system requirements for network selection and steering of roaming as defined in [60] clause 3 and [14] clause 6.30.

[PR 5.2.1.2-006] The 6G system shall provide end-to-end security and privacy protection for roaming subscribers.

[PR 5.2.1.2-007] Based on regulatory requirements and operator policies, the 6G system shall support Disaster Roaming, as defined in [14] clause 6.31.2.3.

[PR 5.2.1.2-009] The 6G system shall be able to support the following services and capabilities for roaming subscribers that are identified in [14]:

- Network slicing (clause 6.1),

- Positioning services (clause 6.27),

- Ranging based services (clause 6.37),

- Roaming value-added services (RVAS) (clause 6.44),

- Support for roaming services providers (clause 6.45),

- Satellite access related roaming (clause 6.46.4),

- Charging aspects (Clause 9)

Editor’s Note: other services anc capabilities can be added, FFS.

Editor’s Note: the identification of explicit 5G system services and capabilities that the 6G system shall not support are FFS.

* * * * End of Change * * * *