3GPP TSG SA WG 1 Meeting #110 	S1-252048

19-23 May 2025, Fukuoka, Japan

Source:	Huawei

pCR Title:	Update of use case on road digitalization

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.4

Document for:	Approval

Contact:	Fangyuan Zhu< zhufangyuan@huawei.com >

Abstract: This document provides some updates for the Use Case 7.6.

1. Introduction

The aim of this contribution is to update the Use Case 7.6 by addressing the fowllowing Editor’ s Note:

Editor’s Note: the KPI values are FFS.

2. Reason for Change

2. 1  KPI values of sensing target density for road digitalization

This use case proposes some typical urban transport scenarios for object detection and tracking, such as the Crossroad and highway, considering the densities of vehicle could be very diversified.

The sensing area of highway(1000m dual three-lane carriageway(24m) can be described in the Figure 1. Similarly, the crossroad area is approximated by 1000m dual three-lane carriageway(24m), by assuming 250m per direction at the crossroad.

Figure 1: Sensing area of highway

The assumptions of this use case is summarized in the Table 1.

Table 1: Assumptions of the use case

| Sensing target | Size (Length x Width x Height) | Velocity | Distance between vehicles[1] | Sensing area (Note 1) |
| --- | --- | --- | --- | --- |
| Vehicle | 5m x 2m x 1.6m | Up to 5km/h(Static/moving) | 3m | Crossroad (Note 1) |
|  |  | Up to 120km/h | 66m | Highway (Note 2) |
| Note 1: One crossroad area is approximated by 1000m dual three-lane carriageway(24m), by assuming 250m per direction at the crossroad. Note 2: Based on an assumption of dual three-lane carriageway (1000m x 24m). |  |  |  |  |

According to the following definiton of Sensing target density agreed in the SA1#109 meeting, the different values for sensing target density at different sensing area can be desribed in the Table 2.

Sensing target density: total number of objects to be sensed per geographic area. It is a measure of how many objects the 3GPP system can detect, identify and/or track within a target sensing area.

Table 2

| Sensing target density | Sensing target Velocity |
| --- | --- |
| 750 vehicles/Crossroad (Note 1) | Static/moving, up to 5km/h |
| 85 vehicles/Highway (Note 2) | Moving, up to 120km/h |
| Note 1: One crossroad area is approximated by 1000m dual three-lane carriageway(24m), by assuming 250m per direction at the crossroad. Note 2: Based on an assumption of dual three-lane carriageway (1000m x 24m). |  |

The sensing target density for pedestrian at crossroad is suggested to be 100, by assuming 25 pedestrian per direction at the crossroad.

3. Proposal

It is proposed to update the Clause 7.6 of TR 22.870 v0.2.1.

* * * First Change * * * *

7.6	Use Case on road digitalization

7.6.1	Description

Many transportation and urban applications require real-time and citywide traffic flow estimation, which is the basis for transportation planning and traffic control. Estimated traffic flow is generally represented by the number of cyclists/vehicles/pedestrians passing a reference location per unit of time and can be virtualized by a third-party application.

In order to enable real-time navigation for automatic driving or traffic flow monitoring, it is necessary to define the upper limit of sensing object detection/tracking required by a 3GPP sensing service, since any 3GPP based sensing detection/tracking coexists with communication services and is not cost-free for operators by sharing. 3GPP system shall allocate limited but sufficient spectrum resources, power, base stations and/or road side units and etc for sensing operations. Such kind of necessity can be represented by sensing target density.

For example, the number of vehicles that needs to be simultaneously detected and tracked at a crossroad may be up to 1000 cars per [km2], including all stationary and moving vehicles occupying that cross-road temporally.

The assumptions of this use case are described in the Table 7.6.1-1.

Figure 7.6.1-1: Sensing area

Table 7.6.1-1: Assumptions

| Sensing target | Size (Length x Width x Height) | Velocity | Distance between vehicles[96] | Service area (Note 1) |
| --- | --- | --- | --- | --- |
| Vehicle | 5m x 1.75m x 1.5m | Up to 5km/h(Static/moving) | 3m | Crossroad (NOTE 1) |
|  |  | Up to 120km/h | 66m | Highway (NOTE 2) |
| NOTE 1: One crossroad area is approximated by 1000m dual three-lane carriageway(24m), by assuming 250m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000m x 24m). |  |  |  |  |

7.6.2	Pre-conditions

Good partnership and cooperation are established between Traffic Department A and Mobile Operator B. Traffic Department A subscribes to the 3GPP wireless sensing service from Mobile Operator B for the real-time road digitalization.

In order to monitor traffic volume constantly, Mobile Operator B has deployed and activated base stations and road side units around traffic intensive areas, such as crossroads, urban roads and highways, in order to provide a wide sensing coverage and capability to detect and track all moving objects, including vehicles, bicycles and pedestrians effectively.

NOTE:	For the ease of elaboration, the base station is acting as sensing transmitter and/or sensing receiver in this case. Other sensing modes can also be feasible and useful as well.

7.6.3	Service Flows

Figure 7.6.3-1: Target density at crossroad

Figure 7.6.3-2:  Target density at highway

1.	Traffic Department A receives a lot of complaints from local residents about heavy traffic at crossroads and highways due to ongoing city construction or bad route planning.

2.	Traffic Department A requests Mobile Operator B to detect and track at least [100] sensing objects simultaneously including cyclists, pedestrians and vehicles at crossroads shown in Figure 7.6.3-1, and at least [50] sensing objects simultaneously including high-speed vehicles at overpasses shown in Figure 7.6.3-2. According to preferred sensing target density, base stations and road side units deployed by Operator B along these areas constantly monitor the road situation by appropriate sensing operations according to these requests.

3.	Mobile Operator B collects 3GPP sensing data from base stations and road side units, and non-3GPP sensing data from sensors (e.g. radar, cameras) if available. Then Mobile Operator B provides Traffic Department A with real-time traffic flow information, including vehicle volume of different directions within a unit time (e.g. every 10 minutes, every half hour), average queuing time for a vehicle/pedestrian as the crossroad, etc.

4.	For more demanding traffic monitoring, Traffic Department A requests Mobile Operator B to detect and track more objects simultaneously at given crossroads and overpasses during rush hours. Mobile Operator B adapts to the request by updating the configuration parameters of sensing operations.

5.	Based on sensing results exposed by Mobile Operator B for road digitalization, Traffic Department A figures out solutions to improve the utilization of public roads at real-time by finer spatial/temporal traffic control and traffic divergence.

7.6.4	Post-conditions

Thanks to the network-wide coverage, the bird's-eye-view of the base station, road digitalization is enabled by capturing real-time information of the road environment.

7.6.5	Existing features partly or fully covering the use cast functionality

TS 22.137 [6], defines set of key performance requirements under clause 6.1:

-	Accuracy of positioning estimate describes the closeness of the measured sensing result (i.e., position) of the target object to its true position value. It can be further derived into a horizontal sensing accuracy – referring to the sensing result error in a 2D reference or horizontal plane, and into a vertical sensing accuracy – referring to the sensing result error on the vertical axis or altitude.

-	Accuracy of velocity estimate describes the closeness of the measured sensing result (i.e., velocity) of the target object to its true velocity.

-	Confidence level describes the percentage of all the possible measured sensing results that can be expected to include the true sensing result considering the accuracy.

-	Sensing Resolution describes the minimum difference in the measured magnitude of target objects (e.g., range, velocity) to be allowed to detect objects in different magnitude.

-	Missed detection probability describes the conditional probability of not detecting the presence of target object/environment when the target object/environment is present. This probability is denoted by the ratio of the number of events falsely identified as negative, over the total number of events with a positive state. It applies only to binary sensing results.

NOTE 1:	An event with a positive state refers to the presence of the characteristics of a target object or environment, including the event falsely identified as being negative and truly identified as being positive.

-	False alarm probability describes the conditional probability of falsely detecting the presence of target object/environment when the target object/environment is not present. This probability is denoted by the ratio of the number of events falsely identified as being positive, over the total number of events with a negative state. It applies only to binary sensing results.

NOTE 2:	An event with a negative state refers to the non-presence of the characteristics of a target object or environment, including the event falsely identified as being positive and truly identified as being negative.

-	Max sensing service latency: time elapsed between the event triggering the determination of the sensing result and the availability of the sensing result at the sensing system interface.

-	Refreshing rate: rate at which the sensing result is generated by the sensing system. It is the inverse of the time elapsed between two successive sensing results.

TS 22.137 has described object detection and tracking by a number of sensing service categories.  However, there is essential KPI missing from each category at given sensing accuracy and service latency. For example, Category 3 does not elaborate how many vehicles from a sensing service area would like to be detected and/or tracked by 6GS. On the other hand, sensing, data communication and other 6G services enabled for the same service area shall share limited 6G spectrum, power, and entities effectively. Therefore, this use case is proposed to clarify preferred sensing target densities, from sensing service perspective, for object detection and tracking within a few typical urban transport scenarios.

In summary, the capability to provide suitable mechanisms for a trusted third-party to request the sensing target density is not covered.

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |

NOTE 3:	To realize 1m granularity tracking, when the velocity resolution is 1 m/s, the maximum corresponding sensing service latency is 1 s.

7.6.6	Potential New Requirements needed to support the use case

[PR 7.6.6-1]	Subject to regulation and operator's policy, 6G system shall efficiently support sensing target density requested by the third party for a given sensing service.

[PR 7.6.6-2] The 6G system shall be able to support the following KPIs:

Table 7.6.6-1:  	Performance requirements for road digitalization

| Scenario | Service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Sensing target density （NOTE 3） | Sensing target Velocity | Max sensing service latency [s] | Refreshing rate [s] | Missed detection [%] | False alarm [%] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |
| Road digitalization | Outdoor | 95 | 0.5(vehicle) 1（Pedestrian） | N/A | 0.5 | N/A | 0.2 | 0.2 m/s | ≤ 750 Vehicles Per 1000m x 24m ≤100 Pedestrian Per 1000m x 24m NOTE 1 | Static/moving, up to 5km/h | 1~5 | ≤ 0.1 | ≤5 | ≤5 |
|  | Outdoor | 95 | 3 | N/A | 3 | N/A | 1 | 1m/s | ≤85 vehicles Per 1000m x 24m NOTE 2 | Moving, up to 120km/h | 1~5 | ≤ 0.1 | ≤5 | ≤5 |
| NOTE 1: One crossroad area is approximated by 1000m dual three-lane carriageway (24m), by assuming 250m per direction at the crossroad. NOTE 2: Based on an assumption of dual three-lane carriageway (1000m x 24m). NOTE 3: The definition of sensing target density is described in Clause 3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of Change * * * *