3GPP TSG-SA WG1 Meeting #110	S1-252123

19-23 May 2025, Fukuoka, Japan	(revision of S1-250982)

Source:	vivo, OPPO

pCR Title:	Use case on computing service for XR game acceleration

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.9	Other Use Cases

Document for:	Approval

Contact:	Yanchao Kang, kangyanchao@vivo.com

Hank Yu, yuhang.txyjy@vivo.com

Abstract: this paper describes a new use case on 6G computing service, such as network computing offloading for XR gaming rendering, to guarantee service experiences for users holding various 6G devices.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5GAA: Technical report "Evolution of vehicular communication systems beyond 5G".

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation".

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

[15] 	Wang et.al, "Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality", IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057.

[16] 	https://en.wikipedia.org/wiki/Adreno.

[17] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/.

[18] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects).

[19] 	https://phlearn.com/tutorial/how-many-exposures-hdr/.

[20]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw.

[21]	https://www.dotphoton.com/products/jetraw-core.

[22] 	https://www.lambdatest.com/learning-hub/web-performance-testing.

[23] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing.

[24] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[25]	EUSPA: "Report on Aviation and Drones – User Needs and Requirements", 2023.

[26]	3GPP TR 36.763: "Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)".

[27]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[28]	3GPP TS 22.156: "Mobile Metaverse Services".

[29] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[30] 	3GPP TS 23.502: "Procedures for the 5G System".

[31]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[32]	https://www.easa.europa.eu/en/what-is-uam.

[33]	https://www.autoflight.com/en/air/.

[34]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[35]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[36]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/.

[37]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[38]	https://www.flyeye.io/drone-acronym-daa/.

[39]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf).

[40]				3GPP TR 33.841: "Study on the support of 256-bit algorithms for 5G".

[41]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[42]	3GPP TS 35.247: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: implementation test data".

[43]	3GPP TS 35.248: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: design conformance test data".

[44]	3GPP TR 22.883: "Feasibility Study on Energy Efficiency as service criteria Phase 2".

[45]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[46]	3GPP TS 38.211: "NR; Physical channels and modulation".

[47]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[48]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[49]	3GPP TS 22.856: "Study on Localized Mobile Metaverse Services".

[50]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[51]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 22.179: “Mission Critical Push to Talk (MCPTT); Stage 1”.

[54]	3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”..

[55]	3GPP TS 22.281: “Mission Critical (MC) video”.

[56]	3GPP TS 22.282: “Mission Critical (MC) data”.

[57]	3GPP TS 22.262: “Message service within the 5G System (5GS); Stage 1”.

[58]	3GPP TS 22.101: “Service aspects; Service principles”.

[59]	3GPP TS 22.173: “IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1”.

[60]	3GPP TS 22.011: “Service accessibility”.

[61]	3GPP TS 22.071: “Location Services (LCS); Service description; Stage 1”.

[62]	3GPP TS 22.268: “Public Warning System (PWS) requirements”.

[63]	3GPP TS 22.153: “Multimedia priority service”.

[64]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains”.

[65]	3GPP TS 22.185: “Service requirements for V2X services”.

[66]	3GPP TS 22.186: “Service requirements for enhanced V2X scenarios”.

[67]	3GPP TS 22.263: “Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)”.

[68]	3GPP TS 22.115: “Service aspects; Charging and billing”.

[69]	3GPP TS 22.289: “Mobile communication system for railways”.

[70]	3GPP TS 22.468: “Group Communication System Enablers (GCSE)”.

[71]	3GPP TS 22.368: “Service requirements for Machine-Type Communications (MTC); Stage 1”.

[72]	3GPP TR 22.949: “Study on a generalized privacy capability”.

[73]	3GPP TR 33.849: “Study on subscriber privacy impact in 3GPP”.

[74]	3GPP TS 22.278: “Service requirements for the Evolved Packet System (EPS)”.

[75]	ETSI GR SAI 004 (V1.1.1): "Securing Artificial Intelligence (SAI); Problem Statement".

[76]	ETSI GR SAI 005 (V1.1.1): "Securing Artificial Intelligence (SAI); Mitigation Strategy Report".

[77]	3GPP TR 28.915: "Study on management aspects of Network Digital Twin".

[78]	3GPP TS 23.527: “5G System; Restoration procedures”.

[79]	3GPP TR 29.866: “Study on IMS Disaster Prevention and Restoration Enhancement”.

[80]	Enabling Mobile AI Agent in 6G Era: Architecture and Key Technologies; https://dl.acm.org/doi/abs/10.1109/MNET.2024.3422309

[81]	Wang L, Ma C, Feng X, et al. A survey on large language model based autonomous agents[J]. Frontiers of Computer Science, 2024, 18(6): 186345.

[82]	Hong S, Zheng X, Chen J, et al. Metagpt: Meta programming for multi-agent collaborative framework[J]. arXiv preprint arXiv:2308.00352, 2023.[xx]	ISO/IEC 22989: "Information technology — Artificial intelligence — Artificial intelligence concepts and terminology"

[83] 	GSMA Foundry, “Case Study: Delivering Real-Time Translation”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/5g-new-calling/

[84] 	GSMA Foundry, “Case Study: Remote damage assessment ”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/remote-damage-assessment/

[85]	 GSMA Foundry, “Case Study: Visualised Video-Calling”,2023, Reference links for  https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/visualised-voice-calling/

[86] 	GSMA Foundry, “Case Study: Adding New Value for Voice Call”,2024, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/adding-new-value-to-voice-calls/

[87]	United Nations Sustainable Development Goals (https://sdgs.un.org/goals).

[88]	United Nations Global Digital Compact (https://www.un.org/global-digital-compact/sites/default/files/2024-09/Global%20Digital%20Compact%20-%20English_0.pdf).

[89]	Zhang Z, Wang S, Hong Y, et al. Distributed dynamic map fusion via federated learning for intelligent networked vehicles[J]. arXiv preprint arXiv:2103.03786, 2021

[90]	AECC: Operational Behavior of a High Definition Map Application – White Paper Version 1.0.0

[91]	 https://spectrum.ieee.org/boats-should-be-sleek-but-only-up-to-a-point

[92]	https://www.marketsandmarkets.com/Market-Reports/evtol-aircraft-market-28054110.html

[93]	https://www.fortunebusinessinsights.com/autonomous-vehicle-market-109045

[94] 	5G-ACIA White Paper, “Use Cases and Requirements for Integrated Sensing and Communication (ISAC) in Connected Industries and Automation”

[95]	ETSI GR ISC-001 V1.0.0 group report, “Integrated Sensing And Communications (ISAC); Use Cases and Deployment Scenarios”

[96]	3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR.

[97]	3GPP TR 22.865: “Study on satellite access - Phase 3”.

[98]                   	3GPP TR 22.836: "Study on Asset Tracking Use Cases".

[99]	CHINA AIRSPACE CLASSIFICATION AND PILOT UPDATE https://www.icao.int/APAC/Meetings/2024%20ATMSG12/IP09%20China%20Airspace%20Classification%20and%20Pilot%20Update.pdf#search=AIRSPACE%20CLASSIFICATION.

[100]	3GPP TS 38.171: "NR; Requirements for support of Assisted Global Navigation Satellite System (A-GNSS)".

[101] 	https://cloud.tencent.com/developer/article/1654264.

[102] 	ITU G.114: One-way transmission time.

[103]	S4aV200634: "Traces and Configurations for VR2, CG and AR2".

[104]	ITU-T FG NET-2030, "Representative use cases and key network requirements for Network 2030", 2020.

[105]	Ian F. Akyildiz, Hongzhi Guo, "Holographic-type communication: A new challenge for the next decade", ITU Journal on Future and Evolving Technologies, Volume 3 (2022), Issue 2, Pages 421-442.

[106]	X. Xu, Y. Pan, P. P. M. Y. Lwin and X. Liang, "3D holographic display and its data transmission requirement," 2011 International Conference on Information Photonics and Optical Communications, Jurong West, Singapore, 2011, pp. 1-4, doi: 10.1109/IPOC.2011.6122872.

[107]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "A new SNOW stream cipher called SNOW-V", IACR Transactions on Symmetric Cryptology, vol. 2019, no. 3, pp. 1-42, 2019. doi: 10.13154/tosc.v2019.i3.1-42.

[108]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "SNOW-Vi: an extreme performance variant of SNOW-V for lower grade CPUs ", WiSec '21, pp. 261 – 272, 2021. https://doi.org/10.1145/3448300.3467829

[109]	IEC 62056-6-1:2023 Object Identification System (OBIS).

[110]	IEC 62056-6-2:2023 COSEM interface classes.

[111]	IEC 62056-5-3:2023 DLMS/COSEM application layer.

[112]	Hexa-X-II, “Deliverable D1.4, 6G Value, Requirements, and Ecosystem”, April 2025.

[113] 	ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[114]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[115] 	Urban Air Mobility, https://www.airbus.com/en/innovation/low-carbon-aviation/urban-air-mobility, AIRBUS, accessed in May 2024.

[116] 	Urban Air Mobility (UAM) Concept of Operations, https://www.faa.gov/air-taxis/uam_blueprint, Federal Aviation Administration of U.S. Department of Transportation, Aug. 2023.

[117] 	Urban Air Mobility, https://rotorcraft.arc.nasa.gov/Research/Programs/UrbanAirMobility.html, U.S. NASA, 2024.

[118] 	NASA Urban Air Mobility (UAM) Reference Vehicles, https://sacd.larc.nasa.gov/uam-refs/, U.S. NASA, Jan. 2024

[119] 	Future Mobility, https://www.hyundai.com/worldwide/en/newsroom/detail/0000000093.html, Hyundai Motor Company, Jul. 2022.

[120]	LG Uplus, Near-Future Vertical Applications in Korea: K-UAM, https://nextgalliance.org/wp-content/uploads/2024/05/Near-future-Vertical-Applications-in-Korea-K-UAM_6GF_NGA_Joint_Workshop_JKim_final.pdf, Next G Alliance / Korea 6G Forum Joint Workshop

[121]	A. Baltaci, et. al., “A Survey of Wireless Networks for Future Aerial Communications (FACOM),” IEEE Communications Surveys & Tutorial, Vo. 23, No. 4, Q4 2021. https://ieeexplore.ieee.org/document/9508366.

[122]	3GPP TR 22.887: "Feasibility Study on satellite access – Phase 4".

[123]	Next G Alliance Report “6G Applications and Use Cases”: Use Case 4.6 – High-Speed Wireless Connection in Aerial Vehicle for Entertainment Service.

[124]	Topal, O. A., Schupke, D., Björnson, E., & Cavdar, C. ”Optimal Joint Access Point Placement and Resource Allocation for Indoor mmWave Communications”, ICC 2023, Rome, Italy.

[125]	Hofmann, S.; Duhovnikov, S.; Schupke, D., “Massive Data Transfer from and to Aircraft on Ground: Feasibility and Challenges,” IEEE Aerospace and Electronic Systems Magazine, vol. 36, iss. 5, May 2021.

[126] 	NASA. UTM: Air traffic management for low-altitude drones[EB/OL] (2015-10) (https://www.nasa.gov/sites/default/files/atoms/files/utm-factsheet-11-05-15.pdf)

[127] 	SESAR. U-space blueprint[EB/OL]. (2017-06-09). (https://www.sesarju.eu/sites/default/files/documents/reports/U-Space%20Blueprint%20brochure%20final.pdf)

[128] 	AOPA China. Regulations for the operation of light and small UAVs [EB/OL] (2018-04-19). (http://www.aopa.org.cn/usr/1/upload/file/20180419/15241267234030958.pdf)

[129] 	HIROYUKI U. UTM project in Japan[EB/OL]. (2017-06-26)[2025-01-20]. (https://gutma.org/montreal-2017/wp-ontent/uploads/sites/2/2017/07/UTM-Project-in-Japan_METI.pdf).

[130] 	G. Wagner and H. Choset, “A complete multirobot path planning algorithm with performance bounds,” Proc. In IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), San Francisco, CA, USA, Sept. 2011. Online Link.

[131]	B. Hu, S. Xu, and Z. Cao, “Multi-Robot Path Planning for Each Robot with Several Jobs in a Single Trip,” IFAC-PapersOnline, Volume 53, Issue 5, 2020, Pages 279-284. Online Link.

[132]	S. Lin, A. Liu, J. Wang and X. Kong, “A Review of Path-Planning Approaches for Multiple Mobile Robots,” Section of Automation and Control Systems, MDPI Machines, 10(9), 773, Sept. 2022. Online Link.

[133]	A New Approach to Data Sharing - Open data sharing and collaboration for data, analytics and AI, DataBricks eBook 3rd Ed. Online Link.

[134]	K.-D. Lee, “A Smart Network of Service Robots: Technical Challenges and Design Considerations,” IEEE Communications Magazine, pp. 28-34, vol. 59, Iss. 8, Aug. 2021. Online Link.

[135]	Network-Enabled Robotic and Autonomous Systems, Next G Alliance, May 2023. Online Link.

[136]	3GPP TR 22.916: "Study on Network of Service Robots with Ambient Intelligence; Stage 1".

[137]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”.

[138]	3GPP TS 22.228:  “Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1”.

[139]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”.

[140]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[141]	3GPP TS 23.503: “	Policy and charging control framework for the 5G System (5GS); Stage 2”.

[142]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[143] 	Test Report from Utility Broadband Alliance “2024 UBBA Plugfest Testing Report” (https://www.ubba.com/ubba-resources/2024-ubba-plugfest-testing-report/).

[A]	Lai Z, Hu Y C, Cui Y, et al. Furion: Engineering high-quality immersive virtual reality on today's mobile devices[C]//Proceedings of the 23rd Annual International Conference on Mobile Computing and Networking. 2017: 409-421.

[B]	Kelkkanen V, Fiedler M, Lindero D. Synchronous remote rendering for VR[J]. International Journal of Computer Games Technology, 2021, 2021(1): 6676644.

[C]	Stojanovic R, Weinrauch A, Tatzgern W, et al. Efficient Rendering of Participating Media for Multiple Viewpoints[C]//High-Performance Graphics 2023: HPG 2023. Eurographics-European Association for Computer Graphics, 2023: 55-64.

* * * Second Change * * * *(all new text)

W.x	Use case on computing service for XR gaming acceleration

W.x.1	Description

With the rapid advancement of XR technology and the gaming industry, there is an increasing demand from users for more realistic and immersive experiences. Providing such experiences often involves intricate graphics rendering, extensive data processing, and real-time interaction, which necessitates a tremendous amount of computing power.

Rendering technologies, such as local rendering, remote rendering, and split rendering, have been studied to enable high-quality VR content on terminal devices as mentioned in [A]. In split rendering, a client (which can be a lightweight client) renders the most time-sensitive or low-fidelity objects that require less computing resources, while some delay tolerant or high-fidelity objects that demand powerful computing resources are rendered at the server (which can be a powerful device) as mentioned in [B]. The server encodes the rendered data (e.g., surface caches) within an intermediate representation according to [C], and the client then decodes, renders and reconstructs locally.

Figure W.x.1-1: Split rendering

Table W.x.1-1: Example requirements for split rendering

| Split rendering | Rendering on the server | Rendering on the client |
| --- | --- | --- |
| Rendering description | - illumination, spheres or grids; - volumetric clouds; - environment modelling; … | - peripheral pixels; - graph synthesis; … |
| Hardware requirements | Generic CPU/GPU can support. | Generic mobile phone can support. |

Split rendering is also advantageous for reducing end-to-end latency and effectively distributing the rendering load between the server and low-powered device. Thus, in order to enable users with various 6G terminal devices to enjoy high-quality experiences at a fraction of the cost, the implementation of split rendering technology is highly recommended.

When offering rendering services to accelerate XR games for users, operators can consider the following three categories of business models:

- Category 1: Service anchor at a third-party AF. This is a classic business model. In this category, the Application Server (AS) operates outside the 6G network. When UEs requests rendering services through specific applications, the AS negotiates with the 6G network to acquire information about computing resources. The computing process are executed and completed in the cloud or edge. Throughout the process, the service anchor remains with the third-party AF, as the final computing results are reassembled and organized at the AF before being sent to the UE. The primary role of the 6G network in this model is to facilitate the transfer of data and information between the UE and AF.

- Category 2: Service anchor at a third-party AF with partial computing offloading to 6G network. This is a variation of the classic business model. When UEs request rendering services, the AS might offload part of the rendering process to the 6G network based on the SLA/negotiation. When the AS is deployed by operator, then AS negotiates with 6G network and it can be interpreted into the UE interacts/negotiates with 6G network. If the AS is deployed by 3rd party, then AS interacts with 6G network based on SLC. One or multiple computing nodes within the 6G network handle the offloaded work. After processing, the final result is still reassembled and organized at the AS before being sent to the UE. In this model, the 6G network provides services beyond data transfer, which also include computing services.

- Category 3: Service anchor within 6G network. IMS is one instance of this category. In this category, the AS is deployed inside the 6G network. It can be interpreted into the UE interacts/negotiates with 6G network. When UEs request rendering services, the AS invokes one or multiple computing nodes within the network to either individually or collaboratively complete the rendering process. After the process are completed, the results may be reassembled and organized at the AS before being transmitted to the UE. The 6G network provides services beyond data transfer, which includes both communication services and computing services.

Figure W.x.1-2: Business models to provide computing services

The existing mobile network is capable of meeting communication demands, while the convergence of mobile network and computing power can offer a wide range of converged computing and communication services. In the future, 6G networks will provide significant resources, including communication resource, computing resource, data resource, and AI resource, inspiring new solutions for game acceleration services (e.g., VR game rendering).

W.x.2	Pre-conditions

The 6G Operator-C has deployed computing nodes for handling offloaded workload from UE. The 6G Operator-C can provide UE with a variety of converged computing services (e.g., XR rendering, environment reconstruction, image classification, language processing). Meanwhile, they can also expose the information of computing resources deployed in the core network or in the edge compute domain to an authorized 3rd party or a UE to help consumer to select their service appropriately.

One game company Wukong and the 6G operator-C have an SLA to enable the 6G system to provide computing services to accelerate XR game for the players of VR game "Wukong".

The game company Wukong performs some preliminary processing for the game "Wukong" on its own server. Then, the splitting rendering can be utilized to divide the game rendering into: a) most time-sensitive or low-fidelity object parts that require less computing resource, and b) some delay tolerant or high-fidelity objects parts that demand powerful computing resources. The former can be achieved by a generic game device, while the latter can be offloaded to the network of 6G operator-C.

Alice and Bob are big fans of the game "Wukong", they both purchased expensive game devices (e.g., VR headsets) to play the game "Wukong" and subscribe to 6G operator-C’s mobile service to ensure that they can enjoy the game anywhere anytime.

They all enjoyed their game until the game company Wukong releases a new version of game. The game experience has been greatly upgraded, but the hardware requirements for game devices are also getting higher and higher. The old game device is unable to guarantee the upgraded game experience due to hardware limitations. Alice and Bob now face two choices: Option a) to buy the new version of game devices with advanced hardware, and of course, it will cost a lot of money. Option b) to buy the computing service from the 6G operator-C to accelerate the game "Wukong".

W.x.3	Service Flows

Figure W.x.3-1: Computing service for VR game rendering

- Alice and Bob play the online VR game "Wukong" together. To enhance their gaming experience and address game issues caused by low computing power, Alice chooses to buy an expensive new game device with advanced hardware. The business model for Alice can be interpreted as category 1 mentioned in description part. For Bob, replacing the equipment would be costly, so he decides to subscribe to the computing service (e.g., VR rendering) from the 6G operator-C for game "Wukong" to accelerate the game. The business model for Bob can be interpreted as category 2 mentioned in description part.

In the subscribed computing services, the UE may firstly negotiate with the 6G network to interact information about computing service, for example, computing task (e.g., rendering, environment reconstruction, image classification, language processing) supported by network, latency required by the UE, how to report the status of the computing task and so on. 6G Operator-C then evaluates the computing task and schedule some appropriate computing resources for the specific computing service. Besides, the network is capable of providing the network-side rendering for some delay tolerant or high-fidelity objects part of VR game. Subsequently, the rendered data are encoded and transmitted to the game device to carry out the terminal-side rendering for the part of the most time-sensitive or low-fidelity objects.

NOTE:	The algorithm used for split rendering can be pre-configured on the computing node, or uploaded by the UE or downloaded from the authorized 3rd party before the rendering service starts.

- The 6G network receives the compressed data from the game company Wukong, which has been preliminarily processed on the game server.

Upon receiving the compressed data from game company "Wukong" for Bob, the 6G Operator-C identifies that Bob has subscribed to the computing service for game rendering. Consequently, the network allocates communication resource and computing resources to provide appropriate computing services by performing network-side rendering, which helps accelerate the VR game interaction on Bob’s game device.

The network also enforces the policy and QoS control for Bob to guarantee both the communication and computing requirements to render and transmit the game data, such as rendered graphics data. Additionally, the network also manages and provides the computing service related information (e.g., computing status of the task) to the Bob’s UE.

Upon receiving the compressed data from game company "Wukong" for Alice, the 6G Operator-C identifies that Alice has not subscribed to the computing service, and therefore all game data are transmitted to Alice's game devices and the game rendering is locally performed by Alice’s devices.

The network also enforces the policy and QoS control for Alice to guarantee the communication requirements to ensure data transmission.

- In accordance with the SLA between the operator (i.e., 6G Operator-C) and 3rd party (i.e., game company Wukong), the network consistently improves the performance of game Wukong rendering and acceleration. When Bob engages in interactive operations in VR games, such as moving, turning, and triggering actions, the VR game rendering provided by computing service on the network provides superior game experiences, such as smooth scene switching, light and shadow changes, object movements, and more. This ensures Bob can enjoy the same game experience with Alice.

W.x.4	Post-conditions

Alice spends a lot of money to buy the new costly XR devices (e.g., VR headsets) to enjoy high-quality, real-time game experiences.

Bob can enjoy the same experiences with Alice without upgrading his game devices. Instead, Bob only needs to invest a small amount of money in the computing service provided by 6G operator-C.

W.x.5	Existing features partly or fully covering the use case functionality

The existing 3GPP system can only provide communication services, the existing 3GPP system does not provide computing service to subscribers.

W.x.6	Potential New Requirements needed to support the use case

[PR W.x.6-1] Subject to operator’s policy, the 6G network shall be able to provide computing service controlled by the core network of 6G system to a UE.

[PR W.x.6-2] Subject to operator’s policy, the 6G network shall be able to support negotiation on requirements for the computing service for the UE.

[PR W.x.6-3] Subject to operator’s policy, the 6G network shall be able to provide the mechanism to manage network resources, including communication resources and computing resources, to support the computing service.

* * * End of Change * * * *