3GPP TSG-SA WG1 Meeting #110 	S1-252150

Fukuoka, Japan, 19-23 May 2025	(revision of S1-25xxxx)

Source:	Qualcomm Incorporated, Meta

New use case title:	Use Case on Improved User Experience

Draft TS/TR:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.6

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: This contribution proposes a new use case which focuses on improving User Experience (UX) for multimedia services by introducing application awareness in the 6G system.

1. Introduction

2. Reason for Change

The use case introduces requirements for enabling exposing  User Experience information between the application and the 6G system to improve user experience and network resource allocaiton efficiency.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.870 v0.2.1

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

...

[x] Duanmu Z, Liu W, Li Z, Chen D, Wang Z, Wang Y, Gao W. Assessing the quality-of-experience of adaptive bitrate video streaming. arXiv preprint arXiv:2008.08804. 2020 Aug 20.

[y] Amiri A, Paymard P, Andres-Maldonado P, Paris S, Pedersen KI, Kolding T. Application Awareness for Extended Reality Services: 5G-Advanced and Beyond. IEEE Communications Magazine. 2024 Aug 12;62(8):38-44.

* * * Second Change * * * *

### 9.x.1	Description

User eXperience (UX) refers to the overall satisfaction of a user with the service provided by the network, and involves multiple aspects, such as multimedia, power, and network coverage (the rest of the proposal focuses on the multimedia aspect of UX). UX can be measured with different metrics depending on the application. For instance, in multimedia-based applications (e.g., live streaming, cloud gaming, video call and XR Split rendering), the UX may be quantified with frame/image quality and smoothness of video playback.

Typically, in 3GPP, the application presents to the network QoS requirements correlated to expected UX in terms of metrics such as bit rate and latency. However, studies have shown that higher bit rate typically results in a better UX, albeit with a diminishing return[x].

Another factor that impacts the UX is the content complexity; simpler content requires less bitrate to achieve the same UX level as a more complex content. In the context of video transmission, simple content may refer to video frames that have low temporal variation (i.e., mostly static) or spatial variation (i.e., mostly plain). Complex content, on the other hand, refers to videos that are highly dynamic and rich in spatial information. For a given user, the content complexity is dynamic (varies with time), and for the network serving multiple users, the complexity could vary across users in the network also.

In 5G and 5GA, certain application awareness features have been introduced in applications such as XR [y], however, the awareness is quite limited in scope. Hence, most networks still rely on simplified assumptions that higher bitrate indicates higher UX. For a user with highly dynamic content, when its application transitions from simpler content to more complex one and the requested bit rate has become obsolete, this could result in degraded user experience. On the other hand, when a user transitions from a complex content to a simpler one and the bit rate is not appropriately reduced to reflect this, the user may consume unnecessary resources (e.g. energy consumption), this could significantly impact power sensitive application such VR/AR. A figure illustrating these transitions is shown in Figure 1 below.

Figure 1: Example of 2 UEs. UE1 switches from a complex scene to a simple one, while UE2 switches from a simple scene to a more complex one.

From a network’s perspective, inability to set the bit rate appropriately may lead to suboptimal network resource allocation.  For instance, consider the scenario depicted in Figure 1 (left), where 2 devices are connected to the same cell and have similar channel conditions. UE1 is streaming complex content while UE 2 is streaming very simple content. A UX-unaware proportional fair scheduler converges to an operating point where the 2 UEs are allocated similar bitrates, which achieves unnecessarily high UX for UE2 (that may be achievable with a much lower bitrate), while greatly impacting that of UE1, resulting in limiting the UX capacity of the network (UX capacity: the maximum number of users per cell where at least 90% of the users have exceeded a certain UX level).

Figure 2: UX-unaware resource allocation results in reduced UX capacity and/or application coverage.

A UX aware RAN receives the UX information from the application server (or application client). With the awareness of such information, the network can take smarter and more optimized resource allocation decisions. For instance, in the scenario of Figure 1 (a), the network may decide to lower the bitrate of UE2 (without impacting its UX, since it has a simple scene) and allocate the freed-up resources to UE1 with the complex content to meet its own UX target. Additional freed-up resources may even be allocated to a third UE while meeting its UX target as well (as depicted in Figure 1 (b)). The UX awareness increases the UX capacity of the network.

### 9.x.2	Pre-conditions

John and Samantha live in the same apartment complex and use wireless communication services provided by MNO XY. John is a gamer and enjoys playing multiplayer first-person-shooter cloud games on his tablet. These games are typically very dynamic, with intense battle scenes and fast-paced actions, vibrant graphics, and multi-player interaction.

Samantha has a virtual reality (VR) headset which she typically uses in virtual touring of museums using the wireless communication service provided by MNO XY. During the tour, she is typically engaged in a complex environment with many moving people and exhibits, enticing her to be continuously moving her head around. She follows a virtual guide, interacts with exhibits or interactive displays, etc. Sometimes, she stops to look at a specific painting or read an informational plaque.

MNO XY just advertised a new service with improved user experience for multimedia applications such as cloud gaming and VR, so John and Samantha signed up for the service. After service registration, MNO XY configures John’s tablet and Samantha’s VR headset to send application related information to the network. Each application (either application server or application client) sends periodic UX reports with an indication of the current content complexity. An example for the indication of content complexity could be a function mapping the bitrates to video quality, where more complex content requires higher bitrates to achieve a certain target quality.

### 9.x.3	Service Flows

- On Saturday morning, John decides to relax in this apartment and connect with his friends over a multiplayer first-person-shooter game. John’s tablet may send an indication of complex content during intense battle scenes and fast-paced actions, and indication of simple content during loading screens and menu interfaces.

- Around the same time, Samantha decides to take a virtual tour of a museum using her VR headset. Samantha’s VR headset may send an indication of simple content during periods of Samantha’s slow motion or focus on paintings and plaques, and an indication of complex content during periods of Samantha’s fast motion and high interactions.

- The devices or the application servers may send UX reports periodically or when substantial changes happen within the content (e.g., scenes change).

- With the knowledge of content complexity, the network re-distributes its resources in a way that gives fewer resources to users with simpler content (thereby not impacting their UX), and more resources to users experiencing more complex content (thereby greatly improving their UX). The application (server or client) may then be notified of the network’s allocation decisions.

- In a time when Samantha is reading an informational plaque in her virtual museum tour, John’s overall gaming experience in a battlefield is improved (low lag and sharper images) due to the excess network resources he can get after Samantha’s network share is reduced, without affecting Samantha’s experience. Conversely, Samantha may make use of extra network resources that are freed-up from John’s device when John pauses his game (goes to menu screen) or not moving in a safe hide.

### 9.x.4	Post-conditions

With this new service, John notices his cloud gaming application runs more smoothly and hence the delay from when he hits the “shoot” button to when it is displayed on the screen is shorter, improving his scores in the game.  Similarly, Samantha notices a smoother viewing on her device and the battery of her VR headset has lasted longer than it typically would.

In addition, the network’s UX-aware resource allocation decisions improve the overall UX capacity (user satisfaction) in the network.

### 9.x.5	Existing features partly or fully covering the use case functionality

QoE: 3GPP R17/18 has defined QoE measurements configured at a specific UE by the RAN which can be used by the RAN to optimize network and improve user experience. QoE information defined includes RTT, bitrate, frame rate, packet losses for IMS:  and Throughput, buffer level, playout delay for DASH [TS 28.404, TS 28.405,  TS 28.406]. This existing framework does not provide information for tradingoff the user experience vs rate due to lack of informtion on content complexity from the application.

L4S: The L4S (Low Latency, Low Loss and Scalable Throughput) feature in 3GPP R18/19 introduced mechanisms for the network (i.e RAN and/or core network) to notify the application of network congestion and request that the application change its bit rate accordingly [TS 23.501]. Unlike this UX awareness proposal, L4S does not provide user experience information to the network.

### 9.x.6	Potential New Requirements needed to support the use case

[PR 9.x.6-1] Based on operator policy and user consent, the 6G system shall provide means (e.g. API(s)) to receive information about the target user experience (e.g. user experience report with the real-time multimedia user experience as a function of bitrate) from authorized 3rd party applications (such as real-time multimedia services.)

[PR 9.x.6-2] Based on operator policy and/or user consent, the 6G network shall support mechanisms to utilize user experience reports to dynamically adjust QoS parameters to efficiently allocate network resources to achieve target user experiences.

* * * End of  Change * * * *