3GPP TSG SA WG 1 Meeting #110 	S1-252152

19-23 May 2025, Fukuoka, Japan	(revision of S1-25xxxx)

Source:	Apple

Title:	Regulatory Requirements for Satellite Access

Agenda item:	6.1

Document for:	Discussion

Contact:	Mona Mustapha at Apple dot com

Abstract: This document provides a brief overview of regulatory requirements for satellite access, an analysis of current SA1 requirements related to these regulations and proposes a way to address the potential gap.

Introduction

In TR 22.926 "Guidelines for extraterritorial 5G Systems (5GS)", SA1 documented different regulatory scenarios for 3GPP satellite-based systems, including network selection. However, after further review, one aspect of network selection is missing, which is related to UE handling on device switch-on or recovery from lack of coverage.

Regulatory requirements for satellite access

There are regulations in certain countries which prohibit the use of satellite technology (either generally or allow the usage for specific purposes e.g. disaster scenarios), prohibit the use of satellite phones, require a license to use satellite services (without which the use of satellite services is prohibited), etc. A few references are provided below:

https://in.usembassy.gov/travel-alert-for-u-s-citizens-january-10-2025/

https://www.bicma.gov.bt/data/publications/rules-regulations-guidelines/Guidelines_for_use_of_Satellite_Phones_in_Bhutan_2019.pdf

https://rura.rw/fileadmin/Documents/ICT/Laws/Draft_Regulation_governing_Satellites_services_in_Rwanda_18032022.pdf

https://btrc.portal.gov.bd/sites/default/files/files/btrc.portal.gov.bd/page/2f2e55a5_f856_40a4_b0f3_6839b3905318/2022-02-07-07-53-74499b9aa26bfb32f7c6ea30f1c3760e.pdf

https://car.bjmp.gov.ph/wp-content/uploads/2023/07/04.14.2023-BJMP-JSIOC-MC-149-SATELLITE-PHONE-POLICY.pdf

Relevant TR 22.926 content

Most of the clauses in TR 22.926 refer to actions on UE or network side once the UE has selected the network.

Country A

Country B

Country C

MCC A MNC X

Figure 8.2.1-1: Satellite access covering multiple countries (from TR 22.926)

Clause 8.2 deals with "Network selection with extra territoriality". Upon further review, this clause deals with the periodic network selection, where the UE is already on a network. In this scenario, TR 22.926 suggests "Selecting a network in the country where the UE is located is the best way to ensure that national regulatory requirements are fulfilled."

This intention is reflected in TS 22.011 clause 3.2.2.5 Periodic network selection attempts:

"…, the UE shall only select a PLMN […], which are of the same country as the currently registered PLMN.

NOTE: 	In the context of this 3GPP TS, the term country is to be interpreted not as a political entity but as a single Mobile Country Code (MCC). For example, the USA and India have multiple MCC. Such cases are in fact treated as exceptions in the 3GPP specifications. For all other countries, multiple MCCs may be used, however the specifications have not taken this into account and there could be adverse effects such as the UE being unable to detect that multiple MCCs are within the same country."

Gap analysis

Referring to the diagram in the previous clause, copied from TR 22.926, if regulations in country B prohibit the use of satellite access, when the UE is in country B in an area with no terrestrial network coverage, and the UE is under the coverage area of a satellite network with a PLMN ID indicating country A (where satellite access is not prohibited), it is currently unclear what action the UE should take. The UE, in absence of any other reliable information, could inadvertently consider itself to be under terrestrial coverage and try and register to the network, thus violating the regulations of country B.

Note that at the point of selecting the network, the UE is unaware whether the network the UE is trying to select provides terrestrial or satellite coverage, especially if the PLMN ID used by the network is the same for their terrestrial and satellite access.

Regulatory compliance

3GPP has already addressed the case of preventing UEs from accessing satellite cells by making it mandatory for the 5G network to provide an indication that the cell is using satellite access using a combination of broadcast information 'cellBarred' and 'cellBarredNTN-r17' (see TS 38.331 clause 5.2.2.4). The indications are used today to ensure UEs that do not support satellite access avoids selecting a satellite access network, and conversely, for UEs that support satellite access to select it.

It is worth noting that LTE IoT NTN systems also have the indication as in 5G, to ensure that UEs that do not support LTE IoT NTN do not access such coverage. However, for LTE covered by satellite, no such indication exists, which means that the UE is not able to make the same distinction.

Proposal

For regions which do not allow satellite phone usage, the UE is prohibited from transmitting to a satellite. As such it is important for the UE to discover if a particular cell it is trying to camp on and the associated PLMN provides terrestrial coverage or satellite coverage.

For the scenario described in the gap analysis above, to help the UE comply with local regulations, an indication to the UE of whether an LTE cell is terrestrial or satellite will need to be provided similar to 5G NTN and LTE IoT NTN. In addition, it is also required that such indication must be provided to the UE (at least) in the locations where a violation of regulatory requirement is possible due to ambiguity at the UE.

A CR to TS 22.278 is provided in contribution S1-252153 for discussion and to illustrate the potential requirement.