3GPP TSG-SA WG1 Meeting #110 	S1-252159

Fukuoka, Japan, 19-23 May 2025	(revision of S1-250567)

Title:	Network Slice Service continuity between 5G and 6G systems

Agenda Item:	8.1.2

Source:	Nokia, Telefonica, TIM, Verizon, Vodafone, AT&T, MediaTek Inc.

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: Network slicing has been introduced as a fundamental pillar of the 5G system to address the requirements of differentiated services and to enable new business operations and opportunities. In the 6G era, network slicing is expected to continue playing a crucial role. Thus, it is essential to enable 5G and 6G slice service continuity for the network slicing.

---------- Use Case template ----------

## 5.5.x	Network Slice Service continuity between 5G and 6G systems

### 5.5.x.1	Description

3GPP has introduced network slicing as a mandatory feature for 5G system and indeed this remains one of the few features that operators around the globe are trailing and attempting to monetize for differentiated services beyond traditional mobile broadband. While the 5G system and network slicing have had a learning curve, their adoption is accelerating. Therefore, we consider that network slicing will also need to be supported in 6G system to avoid a compatibility issue with 5G system deployments and prevent the removal of the network slicing feature in the 5G system if there is evidence that it will be discontinued in the 6G system. It is therefore assumed network slicing will remain a component of the 3GPP system as we transit from the 5G system to the 6G system. From operator’s point of view, additional flexibility, automation and efficiency are possibly needed in the 6G network slice design and architecture, to deal with the increased dynamics required by verticals and for the new use cases. For example, in the 5G system, a slice creation/termination requires various network function (NF) configurations and optional rules for route selections depending on the UE platforms, resulting in delay and customized efforts to bring up a slice. These issues for deployment and potential solutions for the flexibility refers to enhancements in RAN, Core and UE.

One of the well-known problems of 5G is that even though the 5G system supports network slicing, due to UE mobility to 4G or even 5G non-standalone deployment area, some of the 5G system network slicing features are broken. This creates adoption hurdles for features such as the NSSAA (Network Slice Specific Authentication and Authorization); and also several enhancements in 3GPP have been made over many releases to ensure mobility to the EPS does not break connectivity for the end user. Similarly, seamless mobility between the 5G system and the 6G system is needed to ensure connectivity is preserved when using network slicing features. Facilitating seamless slice service continuity between 5G and 6G for network slicing is vitally important.

Scenarios like the following should be supported:

Scenario A:

- A UE connects to a 5G network and uses some 5G system network slicing features.

- The UE moves to a 6G network

- It is not desirable that the 5G system network slicing features are broken, or service degrades because the network or the UE are unable (due to standards issues) to support the 5G system network slicing features.

- The UE moves again to the 5G network. The UE will access again the initial service.

Scenario B:

- A UE is connected to a 6G network and uses some 6G network slicing features

- The UE moves to a 5G network.

- It is desirable that the UE does not experience service disruption (albeit a downgrade in the 5G system can be understood if the 6G system offers distinct improvement over the 5G system) as it moves to the 5G network because the 5G network or the UE are unable (due to standards issues) to support compatibility with the 6G system network slicing features in the 5G network.

- The UE moves again to the 6G network. The UE will access again the initial service.

To support scenarios like the ones described above, the 6G system needs to support interworking with the 5G system ensuring that a UE can move from a 6G network to a 5G network, receiving the same or similar level of service quality while it is served by a certain network slice. Additionally, the UE should be able to move from the 5G network to the 6G network and be served by a certain network slice, at the same or similar level of service quality as in the previous scenario.

Finally, the 6G and 5G networks should facilitate the UE to switch between the two networks and minimize the impact on the network slice service continuity.

### 5.5.x.2	Potential New Requirements

[PR 5.5.x.2-001] 6G system shall support network slicing requirements as defined in TS 22.261 [14] for the 5G system.

[PR 5.5.x.2-002] Upon UE mobility between a 5G RAT and a 6G RAT, the 6G system shall support service continuity for UE data sessions belonging to network slices with same or similar service quality.

NOTE: the above term does not imply any architectural assumption, e.g. whether 6G CN/RAN is a new or evolved CN/RAN (compared to 5G).

[PR 5.5.x.2-003] 6G system should support potential optimization of network slice management, creation/termination and operation, such as:

- Ability to create slices quickly without much overhead/complexity leveraging automated operations

- Ability to scale and manage the network slices efficiently

- Better interoperability between the UE and the network