3GPP TSG SA WG 1 Meeting #110 	S1-252187

19-23 May 2025, Fukuoka, Japan	(revision of S1-xxxxxx)

Source:	CATT

pCR Title:	Update of 8.8 Use case on low-altitude logistics supported by NTN

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes to update use case on low-altitude logistics supported by NTN to solve the editor’s note.

1. Introduction

In SA1 #109 meeting, the use case on low-altitude logistics supported by NTN is captured in TR22.870 clause 8.8, and the requirements about support operator’s management Service Hosting Environment is marked for future study. The contribution intends to solve the editor’s note left for this requirement.

2. Reason for Change

Rel-18 has already supported edge server on board GEO satellite in 5G system when standardized the feature of satellite backhaul. During the study of Rel-20 FS_5GSAT_Ph4, the use case about providing Service Hosting Environment on multi-orbit satellites for different services was discussed and included in TR 22.887 but has not yet driven the stage2 discussion. 6G system is expected to go further in the coordination of computing and communication for deterministic user experience (e.g. latency), so the impact to satellite access relevant services need to be analyzed and to identify whether any potential service requirements.

PR1 is reworded to reflect the above consideration. The description, gap analysis is also added to align.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.2.1

* * * First Change * * * *

8.8	Use case on low-altitude logistics supported by NTN

8.8.1	Description

For remote regions with inconvenient transportation, complex terrain and rapidly changing weather, traditional logistics delivery methods struggle to meet the needs of local residents. By utilizing unmanned aerial vehicles (UAVs) and other low-altitude aircraft, the low-altitude logistics can overcome limitations posed by terrain and other factors to swiftly deliver the medicine, food, and emergency supplies to the residents live in such remote region, which exhibits its tremendous potential and advantages.

The development and operation of the low-altitude economy heavily rely on robust communication networks to ensure the safety and reliability of UAV systems. However, depending solely on traditional terrestrial networks (such as 4G/5G cellular networks) presents several critical challenges, for example:

- The density of ground base station deployments is limited, particularly in remote areas or challenging terrains such as mountains or forests. This can lead to insufficient or unstable signal coverage, limiting the expansion of the low-altitude economy into these regions.

- UAVs often operate in environments with high mobility and dynamic changes. Ground networks, which are primarily designed to serve relatively stationary or slowly moving objects, will struggle to maintain stable connections and experience higher data transmission delays.

- The coverage height of ground base stations is limited. This can lead to the flight range of medium and large-size UAV being restricted, especially in long-distance, long-haul and heavy-load flight mission. Refer the latest CHINA airspace classification [99], the low altitude area is below 3000m, where the low-altitude UAV could fly.

To address these challenges, the low-altitude economy need reliance on Non-Terrestrial Networks (NTNs), such as Low Earth Orbit (LEO) satellite networks, High-Altitude Platform Stations (HAPS), and other space-based or non-conventional communication infrastructures. NTN can offer broader and reliable coverage, accommodate high-dynamics flight scenarios, and provide enhanced security and regulatory capabilities.

Additionally, in some challenging environment, such as steep mountains, deep valleys, and dense forests, it brings significant difficulties for UAV flight. The rapidly changing weather conditions in remote mountainous regions, such as sudden strong winds, rain, or fog, further complicate UAVs’ operations. An example is showed in Figure 1, some UAVs flight over the area 1 are affected by the heavy rain, while the UAVs flights over the area 2 are affected by the dense fog. Usually, UAVs can be equipped with camera and sensors to monitor the visible environment and process the collected sensor data locally or transmit them back to the application server on the ground for collaborative processing with the data from other UAVs, and/or assisted information about the flight paths from 3rd party (e.g. Meteorological Bureau, which keeps monitoring the weather and forecasting for the whole region based on distributed meteorological monitoring station on the ground and onboard satellites). For the varying environment, the UAV need to adjust the flight paths and associated operations regarding the weather prediction, which raises demands on the round-trip delay of sensing result. Under such scenarios, the 6G networks with satellite access can not only provide satellite-based communication, but also provide satellite onboard computing resources to facilitate non-3GPP sensing data processing (e.g. weather condition, obstacle detection) for UAVs to assist optimizing flight paths and ensure task operation.

Figure 8.8.1-1. Illustration of logistics delivery in remote mountain areas.

Following is an example of service flow to describe the low-altitude logistics delivery via the UAVs supported by the 6G network with satellite access.

8.8.2	Pre-conditions

To improve the delivery efficiency, the Logistics Company A bought a large number of UAVs for the delivery, and it has a dispatch center that is responsible for scheduling and optimizing the flying route of UAVs.  All of the UAVs have already achieved the flight permission from local aviation authority

The UAVs are configured with the camera, sensor and UE.

The Logistics Company A has received many delivery orders toward the remote region A from different customers, and one order is from the customer Alice to deliver the medicine to the Jack home.

The Network Operator B has deployed a 6G network with satellite access to provide the reliable NTN service in this remote region A.

Besides the communication service, the 6G network with satellite access is equipped with the service hosting environment, which can process the high-resolution image data, as well as performing the intelligent analysis of image data directly on the satellite.

The Logistics Company A has subscribed the NTN service from Network Operator B for its UAVs.

8.8.3	Service Flows

- The Logistics Company A inputs all delivery tasks of this remote region into the dispatch center, different delivery tasks have different targeted locations in this region. Then the dispatch center selects some UAVs and distributes a suitable initial route and delivery task for each UAV taking into account environmental conditions. Alice’s order will be delivered by the UAV ‘1-1’.

- The UAVs take their corresponding expresses and begin the delivery according to the planned initial route.

- The 6G network with satellite access provides communication service for the UAVs through satellites or high-altitude platform during UAVs flight to the remote region.

- During the flying, UAVs that are equipped with camera and sensor continuously collect the environmental condition information. Multiple UAVs can share flight status, environmental condition information, and mission progress information through 6G network with satellite access.

- During the flight, the collected environmental information can be transmitted from UAVs to a proper selected Serving Hosting Environment (e.g. edge servers on board satellite) within the 6G network with satellite access. The Service Hosting Environment in the 6G network with satellite access processes the collected environmental data from one or more UAVs, with the associated weather information of this region from 3rd party application and detects that there will be a thunderstorm cloud on the predefined flight path of UAV ’1-1’, which requests UAV ‘1-1’ to adjust the height flight to avoid the cloud.

- The 6G network with satellite access sends an alert to the UAV ‘1-1’, the UAV ‘1-1’ adapts its flight action to avoid the threat.

- Finally, all the UAVs safely arrive at the targeted location as scheduled, and Jack gets the delivered medicine from UAV ‘1-1’.

8.8.4	Post-conditions

The UAVs can communicate with dispatch center, dynamically update their flight routes and successfully finish their own delivery task in such harsh mountainous region through the support of 6G network with satellite access.

The Logistics Company A can finish all delivery orders from all customers securely and efficiently.

8.8.5	Existing features partly or fully covering the use case functionality

SA1 has identified a series of performance requirement related to the UAVs in the clause 7 of [35]. For example, in Table 7.1-1, the maximum altitude is 300m. The flight average speed is 60km/h.

In [14], Table 7.4.2-1, The 5G system with satellite access performance requirements, the data rate for DL0.5Mbps, and data rate for UL 3Mbps for scenario “Vehicle mounted or fixed installation” have been specified.

However, the existing performance requirement cannot support the communication performance requirement for the UAVs in low altitude area (below 3000m) and supported by the satellite access.

In [14], clause 6.5.2 on Service Hosting Environment:

The 5G network shall enable a Service Hosting Environment provided by operator.

Based on operator policy, application needs, or both, the 5G system shall support an efficient user plane path, modifying the path as needed when the UE moves or application changes location, between a UE in an active communication and:

-	an application in a Service Hosting Environment; or

-	an application server located outside the operator’s network; or

-	an application server located in a customer premises network or personal IoT network.

The 5G network shall be able to interact with applications in a Service Hosting Environment for efficient network resource utilization and offloading data traffic to the most suitable Service Hosting Environment, e.g. close to the UE's point of attachment to the access network or based on usage information.

NOTE:	To accomplish offloading data traffic, usage information might be exposed to the Service Hosting Environment.

But the existing requirements don't consider the impact of the characteristics of the satellites (e.g. limited computing resources, the movement of LEO/MEO) for satellite onboard Service Hosting Environment

8.8.6	Potential New Requirements needed to support the use case

[PR.8.8.6-1] Subject to operator’s policy and agreement with 3rd party,  the 6G system with satellite access shall be able to offload the data traffic (e.g. non-3GPP sensing data) of a UAV using only satellite access to the most suitable operator managed Service Hosting Environment on board satellite(e.g. with sufficient computing capabilities, considering joint latency), to process.

[PR.8.8.6-2] The 6G system with satellite access shall be able to support communication service for UAV with variant altitudes (e.g.  from 0 to 3km [99]) using only satellite access.

* * * End of Changes * * * *