3GPP TSG SA WG 1 Meeting #110 	S1-252354

Fukuoka, Japan, 19-23 May 2025

Source:	Philips International B.V.

pCR Title:	New use case on resilient time distribution in satellite networks

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.5

Document for:	Approval

Contact:	Walter Dees (walter.dees@philips.com)

Abstract: This pCR proposes a new use case for clause 8 Ubiquitous connectivity.

1. Introduction

Clause 8 and Clause 5.4 in TR 22.870 contain several use cases related to 6G Ubiquitous connectivity respectively to resilience, indicating potential new requirements. This contribution proposes a new use case related to both topics that is proposed to be included in Clause 8.

2. Reason for Change

In cellular systems rely on tight time synchronization usually distributed via non-3GPP technologies such as GNSS. Similar to use case 8.4, resilient positioning in satellite networks, the degradation or unavailability of GNSStime synchronization, such as due to jamming, spoofing or obstructions, may lead to a degradation or loss of the 6G communication capabilities. This motivates this proposal.

3. Proposal

Include the following use case in Clause 8.

* * * First Change * * * *

## 8.X	Use case on resilient time distribution in satellite networks

### 8.X.1	Description

In cellular systems, accurate time synchronization is essential for various network operations, including the coordination between base stations and efficient spectrum usage. Typically, Global Navigation Satellite Systems (GNSS) like GPS are employed to provide this time synchronization. However, during certain circumstances—such as GNSS signal jamming, natural disasters, or equipment failures—access to GNSS-based time synchronization may be compromised.

In such scenarios, 3GPP satellite networks can serve as a resilient alternative for time distribution. User Equipment (UEs) or terrestrial base stations, can obtain highly accurate time signals directly from the 3GPP satellite networks. This capability ensures the continuity of network operations, even when GNSS signals are unavailable or unreliable. Satellite networks can also support time synchronization across vast geographic regions, making them particularly useful in remote areas or during large-scale disruptions.

For example, in a region where GNSS signals are temporarily unavailable, a local cellular network can use 3GPP satellite networks to maintain accurate timing. This synchronization enables seamless communication between UEs and ensures the proper functioning of time-sensitive applications.

### 8.X.2	Pre-conditions

- The 3GPP satellite network must be operational and capable of providing accurate time synchronization signals.

- User Equipment (UEs) and base stations, must be equipped with capabilities to receive time information from 3GPP satellite networks.

### 8.X.3	Service flows

1. Under normal circumstances, the cellular system uses GNSS to synchronize time for base stations and UEs.

2. When GNSS signals are disrupted (e.g., due to jamming or environmental factors), the system automatically detects the issue and switches to the 3GPP satellite network for time synchronization.

3. The satellite network provides accurate time signals to UEs and base stations, ensuring continuity of operations.

4. The synchronized time allows for uninterrupted communication, coordination between base stations, and proper functioning of network applications.

5. Once GNSS signals are restored, the system may smoothly transition back to GNSS-based synchronization.

### 8.X.4	Post-conditions

- The cellular system continues to function with accurate time synchronization, either via GNSS or 3GPP satellite networks.

- Time-sensitive operations and applications operate without disruptions, maintaining high reliability.

### 8.4.5	Existing features partly or fully covering the use case functionality

Clause 6.36.4 in TS 22.261 includes requirements related to 5G timing resiliency.

6.36 5G Timing Resiliency

6.36.1 Overview

5G systems rely on reference precision timing signals for network synchronization in order to operate. These synchronization references are generated by Primary reference Time Clocks that typically get the timing reference from GNSS receivers and in order to meet the relevant synchronization requirements also during failure conditions, the synchronization network designs typically include means to address potential degradation of the GNSS signal performance. Some deployment of 5G involve applications that themselves can be sensitive to any degradation of the timing signal. In such cases it is beneficial for the 5G system to be enhanced to act as a backup for loss of their GNSS references. In some implementations, timing resiliency enhancements to the 5G system can work in collaboration with different types of time sources (e.g., atomic clock, time service delivered over the fibre) to provide a robust time synchronization. 5G as a consumer of time synchronization benefits from timing resiliency which enables the support of many critical services within the 5G network even during the event of a loss or degradation of the primary GNSS reference timing. Additionally, for time critical services (e.g. financial sector or smart grid), the 5G system can operate in collaboration with or as backup to other timing solutions. A base of clock synchronization requirements when 5G is providing a time signal, if it is deployed in conjunction with an IEEE TSN network or if it is providing support for IEEE 1588 related protocols, is included in [21] clause 5.6. The enhancements in this clause build on this to add timing resiliency to the 5G system enabling its use as a replacement or backup for other timing sources.

6.36.2 General

The 5G system shall support enhanced timing resiliency in collaboration with different types of time sources (e.g., GNSS, TBS/MBS [33] [34], Sync over Fiber [34]) to provide a robust time synchronization.

The 5G system shall be able to maintain accurate time synchronization as appropriate for the supported applications in the event of degradation or loss of the primary timing reference (e.g., GNSS).

6.36.3 Monitoring and Reporting

The 5G system shall be able to support mechanisms to monitor for timing source failure (e.g., GNSS).

The 5G system shall be able to detect when reference timing signals (e.g., from GNSS or other timing source) are no longer viable for network time synchronization.

The 5G system shall support a mechanism to determine if there is degradation of the 5G time synchronization.

The 5G system shall be able to support mechanisms to indicate to devices (e.g., UEs, applications) that there is an alternate time source available for use (e.g., 5G system internal holdover capability, atomic clock, Sync over Fiber, TBS, GNSS), taking into account the holdover capability of the devices.

The 5G system shall be able to detect when a timing source fails or is restored for network time synchronization.

The 5G system shall support mechanisms to monitor different time sources and adopt the most appropriate.

The 5G system shall support a mechanism to report timing errors such as divergence from UTC and time sync degradation to UEs and 3rd party applications.

6.36.4 Service Exposure

The 5G system shall support a mechanism for a 3rd party application to request resilient timing with specific KPIs (e.g., accuracy, interval, coverage area).

However, there is no requirement indicating that the 3GPP satellite network can be used as a synchronization source independently of GNSS.

8.X.6	Potential New Requirements needed to support the use case

[PR 8.6.1-1]	The 6G system with satellite access shall be able to provide time distribution service with 3GPP technologies, independently of non-3GPP technologies (e.g. GNSS).

* * * End of Change * * * *