3GPP TSG SA WG 1 Meeting #110 	S1-252371

19-23 May 2025, Fukuoka, Japan	(revision of S1-252113)

Source:	Charter Communications, Cable Labs, Qualcomm, Comcast, Lenovo, Nokia, LGE, Toyota

pCR Title:	Pseudo-CR on non-3GPP access Support (5.2.X)

Draft Spec:	3GPP TR 22.870v0.2.1

Agenda item:	8.1.2

Document for:	Approval

Contact:	Inma Carrion (inma.carrion@charter.com)

1. Introduction

This contribution proposes to add a specific TR sub-section defining 6G System support for non-3GPP accesses (e.g,WLAN, Wireline).

The support of non-3GPP access will be a key aspect for the 6G system to support the demand of higher data volumes and widespread service offerings. For example, data offload from cellular networks, VoWi-Fi use-cases, and improved indoor coverage (e.g., in indoor scenarios with poor cellular coverage).

2. Reason for Change

Requirements in 5.2.3 are not clear about non-3GPP access and there is currently no agreed text in 5.2.2.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870v0.2.1.

* * * First Change * * * *

### 5.2.X	Interworking with non-3GPP accesses

Interoperability among the various access technologies supported by the 6G system will be imperative. For optimization and resource efficiency, the 6G system will select the most appropriate 3GPP or non-3GPP access technology for a service, potentially allowing multiple access technologies to be used simultaneously for one or more services active on a UE.

The 6G system shall be able to support a user to access network services via 3GPP and/or non-3GPP access (e.g., WLAN or Wireline).

The 6G system shall be able to support 5G system requirements for non-3GPP access as defined in [14].

Subject to operator’s policy, the 6G system shall support mobility between the 6G 3GPP access and non-3GPP access, with minimum impact to the user experience (e.g. QoS, QoE).

* * * End of Change * * * *