3GPP TSG-SA WG1 Meeting #110 	S1-252411

Fukuoka, Japan, May 19 - 23, 2025	(revision of S1-252204)

Source:	Rakuten Mobile, NIST, Intel

pCR Title:	Use case on 6G System Supporting Secure and Privacy-Compliant Data Set Service for AI Training

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Manmeet Bhangu <manmeet.bhangu@rakuten.com>

Awn Muhammad <awn.muhammad@rakuten.com>

Sumanmishra Prabhat <sumanmishra.prabhat@rakuten.com>

Abstract: This contribution proposes new requirements for Use case on 6G System Supporting Secure and Privacy-Compliant Data Set Service for AI Training into TR 22.870 v0.2.1

1. Introduction

This contribution proposes a new use case on Use case on 6G System Supporting Secure and Privacy-Compliant Data Set Service for AI Training.

2. Reason for Change

TR 22.870 v0.2.1 does not yet contain any use case that covers Use case on 6G System Supporting Secure and Privacy-Compliant Data Set Service for AI Training..

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870 v0.2.1

# * * * * First change * * *

## 8.x	Use case on 6G System Supporting Secure and Privacy-Compliant Data Set Service for AI Training

### 8.x.1	Description

Mobile Network Operators (MNOs) in 6G systems will have access to vast amounts of real-time, high-quality data generated by users, devices, and network functions—including mobility patterns, traffic flows, service usage metrics, and QoS/KPI measurements not to mention huge volumes of AI inference data. This data holds immense value for AI/ML model training, particularly for industries like smart cities, autonomous transportation, predictive healthcare, and personalized services. However, accessing such datasets is often challenging for third-party AI developers due to privacy regulations (e.g., GDPR, CCPA), lack of scalable data collection infrastructure, and concerns about data bias or quality. With the advent of new data intensive services such as Sensing etc., we see the Concept of Telco as the Future Data Bank for a Country (refer S1-242023)

This use case enables MNOs to act as trusted intermediaries in a privacy-preserving data marketplace, where:

User Consent & Control:

- Users grant/revoke consent for specific data categories (e.g., location, browsing behaviour)

- Consent is dynamically enforced at the network level via existing or enhanced subscription mechanisms.

Anonymization & Privacy Techniques:

- Raw data is processed using various techniques (e.g., k-anonymity, differential privacy, or synthetic data generation) to prevent re-identification.

- Various forms of learning techniques e.g., Federated learning (FL) may be supported, allowing AI training without raw data leaving the MNO’s domain.

Monetization & Licensing:

- MNOs offer tiered datasets (e.g., Real-time footfall data (premium tier for advertisers) vs historical trends (cheaper, bulk datasets for urban planning)) with usage-based pricing.

- Smart contracts (e.g., via blockchain) automate compliance (e.g., data retention limits, purpose restrictions).

Regulatory Compliance:

- Built-in mechanisms enforce regional rules (e.g., GDPR’s "right to be forgotten," data minimization).

- Audit logs track data access for accountability.

### Example Scenarios where these services can be offered are:

### A smart city planner purchases anonymized foot-traffic data from an MNO to optimize public transport routes.

### An advertising firm uses federated learning to train recommendation engines without accessing raw user profiles.

### 8.x.2	Pre-conditions

The MNO has established legal agreements with third-party AI service providers for data sharing. The 6G system implements:

- Data anonymization/pseudonymization functions.

- Consent management framework (e.g., per-category user preferences).

- Secure data delivery APIs (e.g., encrypted datasets, tokenized access).

Regulatory compliance is ensured (e.g., regional data protection laws).

### 8.x.3	Service Flows

The service flow is as following:

- Third-party request: An AI service provider requests access to specific anonymized datasets (e.g., mobility trends).

The request could be in the form on an intent e.g.,

“I need to train a model for UE mobility for Tokyo West from 6 am to 6 pm for traffic light optimization.

Provide anonymized batch mobility traces dataset for Tokyo West, weekdays 6AM–6PM, at 5-minute granularity and at least 10,000 unique UEs/day to avoid bias for traffic optimization training.”

- Authorization & consent check:

  - The 6G system verifies the third party’s credentials.

  - The system confirms user consent for the requested data categories.

- Data processing:

  - The MNO’s privacy engine applies anonymization (e.g., k-anonymity, synthetic data generation).

  - Optional: Data is pre-processed (e.g., aggregated, noise-injected) for privacy preservation.

- Secure delivery:

  - The dataset is transferred via a secure channel (e.g., encrypted, audit-trailed)

  - Usage restrictions (e.g., retention period) are enforced via smart contracts.

- The MNO charges the third party based on data volume/utility (e.g., per-record, subscription model).

### 8.x.4	Post-conditions

1.	The AI service provider obtains high-quality, compliant datasets for model training.

2.	Users retain control over their data via granular consent settings.

3.	The MNO generates revenue while maintaining regulatory compliance.

### 8.x.5	Existing features partly or fully covering the use case functionality

<tbd>

### 8.x.6	Potential New Requirements needed to support the use case

[6.x.6-1] Based on operator’s policy, user consent and local regulations the 6G network shall support secure mechanisms for dataset discovery (e.g., metadata search, sample access, category of data) for third parties via intent.

Editor’s Note: the meaning of “intent” is FFS

[6.x.6-2] The 6G network shall include configurable (i.e. dynamic) anonymization, consent enforcement, and audit logging.

[6.x.6-3] The 6G system shall support privacy mechanisms for aggregated data sharing.

[6.x.6-4] The 6G network shall be able to charge for these services.

[6.x.6-5] The 6G system shall support model-update messages that are confidentiality- and integrity-protected.

[6.x.6-6] The 6G system shall ensure datasets are deleted or irreversibly anonymised at the end of the agreed retention period.

[6.x.6.7] The 6G system shall support cryptographic metadata so that the data consumer can verify dataset integrity end-to-end.