3GPP TSG-SA WG1 Meeting #110 	S1-252471

Fukuoka, Japan, 19-23 May 2025	(revision of S1-252290)

Source:	InterDigital

New use case title:	Emergency vehicle driving and route management

Draft TS/TR:	3GPP TR 22.870

Agenda item:	8.1.4

Document for:	Approval

Contact:	Catalina Mladin, Catalina.Mladin@Interdigital.com

Abstract: This contribution introduces a 6G use case on traffic and driving management Digital Twin, with focus on management of traffic for emergency vehicle route and traffic impact minimization.

## --------------------- Use Case template – all new text  ---------------------

## 7.x	Use case on Emergency vehicle route planning

### 7.x.1	Description

In a busy city, there are many daily occurrences of emergency response teams such as police, ambulances, fire department, patient transport between health units, civilian emergency driving (e.g., to a hospital), recovery of broken vehicles, etc. These vehicles and the responding workers operate under strict time constrains, and it is of major importance they get to their destination as quickly as possible without any hurdles.

These vehicles have however to sometimes operate in peak traffic hours, where there are many other vehicles on the road, causing bottlenecks to the emergency operation, increasing delays, and ultimately, aggravating the emergency that triggered the emergency vehicle in the first place.

Figure 7.x1-1 shows an example 4km route that could take 30 minutes in peak traffic hours. This example does not intend to be specifically accurate nor describe a definitive timing could take 30 minutes for a route of 4km only, during peak traffic hours.

Figure 7.x.1-1 An example 4 km route that could take 30 minutes in peak traffic hour

The same example route could take potentially 10 minutes under very low traffic conditions. The emergency vehicle route planning aims at the planning of a route for an emergency vehicle once triggered, to efficiently manage all the traffic between the two end points, in such a way that the emergency vehicle route is optimized, and potentially traffic free, while minimizing the impact on the other road vehicles.

### 7.x.2	Pre-conditions

There is a driving and traffic management application that is a Digital Twin (DT) of the city roads, and has relevant information such as road dimensions, can communicate with the mobile network, and has been granted access to the exposure service for sensing results from the mobile network.

Furthermore, the following pre-conditions exist:

- On road vehicles are self-driving and are connected to the DT. Self-driving cars are equipped with wireless communication & sensing device and have the appropriate identification and authorization means to attach to the network

- Cars equipped with wireless communications device are connected to DT and can report on traffic density information, road conditions, pedestrian density, etc.

- Both cars (UEs) and BSs have sensing capabilities and can operate in monostatic, bi-static or multi-static sensing modes in all UE/BS combinations.

- Any non-public information from both cars and BSs in this use case is removed before sensing results are exposed to the DT

### 7.x.3	Service Flows

The action and presence of an emergency vehicle is triggered with the emergency team. The two end points for the trip the emergency vehicle is about to make, i.e., stating point and destination, are known. The traffic DT determines one or more routes for the emergency vehicle. It may consider the best route for the emergency vehicle is, e.g., the fastest route. This route may then be communicated to the road users and emergency vehicle.

Upon the calculation of the route by the DT applications, the DT requests a sensing service from the mobile network with the precise route information. This allows the mobile network to start a sensing service for the purpose of traffic density assessment along and around the provided route(s) by the DT. Sensing traffic along the route allows for a first assessment if that route is suitable for the emergency. This is achieved by exposing the sensing contextual information by the mobile network back to the DT application. An assessment around the route may be necessary as well, because in peak traffic hours, it may be extremely difficult to move traffic to adjacent roads to let the emergency vehicle pass through quicker. This information is fed to the driving and traffic management DT.

As the emergency vehicle moves towards the destination, the traffic ahead is also recurrently assessed or scanned. This is necessary to ensure conditions have not changed in a significant way, i.e., that the current assessment of the best route is maintained. The assessment rate changes accordingly to the initial assessment and may change along the emergency vehicle’s route to the destination. Sensing for the assessment may be carried out by the mobile network’s system, with the support of the connected cars, but may also be carried out by the connected cars, e.g., in case of impairments in the radio sensing process (e.g. when confidence level of vehicle density cannot be accurately determined) from the mobile network’s system. This is realised by the mobile network’s sensing function by either modifying sensing radio resource allocation, switching between sensing modes, or collecting sensing information from other sensing sources such as video or LiDAR.

It is also important to note that sensing of traffic in proximity of the target object (vehicle) is carried out along the vehicle’s route and should be a forward-looking assessment. In the example provided, if the emergency vehicle is close to its destination, the useful area would be around the vehicle and towards the destination, and it would be of little significance for the DT or even inefficient to assess traffic conditions elsewhere, e.g., close to its route’s starting point. Therefore, the mobile network’s assessment needs to consider the provisioning of the best possible resources in the area, e.g., considering latency of communication towards the DT or other requirements. The choice of the best resources will therefore change along the route, and the mobile network needs to ensure smooth transition of system resources allocated for the use case. In addition, along certain routes, connectivity and coverage problems, as well as lower speed zones may be a reality. For coverage and connectivity problems, mechanisms to assess the quality of the connectivity along routes or in certain areas are required, namely as means to anticipate breaks in service or unfulfillment of KPIs, instead of reactively dealing with them when the emergency vehicle is already heading towards its destination. This could be achieved e.g., by keeping historical data of connectivity quality, as well as driving time related history of vehicles that previously used the route.

The connected car can provide useful sensing related information for the use case resorting to either RF, video or LiDAR based sensing. However, both video and LiDAR as sensing sources can also experience impairments. This could be due to weather conditions such as heavy rain that can impair the performance of computer vision algorithms  and cause LiDAR performance degradation due to bright sun, fog, rain, dirt and spray.

Different sensing modes must be considered as well because their resource usage is not equal, and same for their performance. For instance, a BS operating in mono-static mode might not achieve the correct vehicle count resolution, as it may be difficult to distinguish between different reflective paths from different closely located vehicles. Operating in V2V or V2X bi-static mode or multi-static modes could help overcome this issue, but drawbacks include interference that might arise from simultaneous vehicle sensing signal transmissions, as well as additional sensing processing and coordination operations that would be required.

The area that is being sensed along the route will change in requirements. Optimal sensing procedures for this use case need distinction between e.g., empty and heavily loaded roads. For example, a BS with a camera mounted with a clear view to a long stretch of empty road with no pedestrians in sight could prevent excessive resource allocation to RF sensing transmissions for a given RAN node, or could help time sequential activated transmissions for two or more RAN nodes, where the activation would follow the emergency vehicle’s route. The identification of sensing resources that can provide this assessment is therefore required, and needs to be updated as the vehicle moves along the route.

The DT benefits from this information and may change the best route for the vehicle. If this happens, the new route is then communicated to the road users and emergency vehicle. As the emergency vehicle progresses on its route, the DT may send all sort of notifications and/or indications to any of the cars on the road, it being e.g., a warning that an emergency vehicle is approaching, a request for the vehicle to clear the road ahead, or perform a manoeuvre to clear the way, turn left/right to take a different route, etc..

The presented use case may be extended and applicable to different SLA agreements between a customer, e.g., a taxi service user and a service provider of such type. The same technical principles would apply, and the DT could consider traffic prioritization for customers who have e.g., a fast-track subscription service.

### 7.x.4	Post-conditions

The emergency vehicle arrives at its destination with as little delay as possible. The route disturbance of other vehicles is minimized.

### 7.x.5	Existing features partly or fully covering the use case functionality

Positioning feature.

### 7.x.6	Potential New Requirements needed to support the use case

Functional requirements in this use case include:

[PR 7.x.6-1] The 6G System shall be able to adapt the area to be sensed over time to the emergency vehicle’s route.

[PR 7.x..6-2] The 6G System shall ensure smooth transition of the allocated resources as the area being sensed changes .

[PR 7.x.6-3] The 6G System shall be able to adapt the size of the area to be sensed depending on the environment condition, both in location being sensed and physical size of the area being sensed.

[PR 7.x.6-4] The 6G System shall be able to selectively determine Sensing Transmitters and Sensing Receivers  by their location and sensing reachability to the concerned target sensing service area.

[PR 7.x.6-5] The 6G System shall be able to switch between sensing modes amongst BSs and UEs for the detection and tracking of all elements relevant to the use case.

[PR 7.x.6-6] The 6G System shall be able to selectively determine sensing sources by type (e.g. 6GS sensing, LiDaR, Radar, WiFi, video, etc.) depending on diverse weather conditions (e.g., bright sun, fog, rain, dirt and spray).

[PR 7.x.6-7] The 6G System shall be able to fuse sensing information from different sensing sources to produce more accurate sensing results, e.g. under adverse weather conditions.

[PR 7.x.6-8] The 6G Network shall be able to generate charging related information based on sensing information provided to applications (e.g.  DT).

[PR 7.x.6-9] The 6G System shall support sensing services with KPIs as given in Table 7.x.6-1.

Table 7.x.6-1	Performance requirements for emergency vehicle route planning

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service initiation [s] (NOTE 1) | Sensing resources refresh rate [s] (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |  |
| ADAS [long range Radar] | Outdoor | [95] | [≤1.3] (NOTE 2) | ≤0.5 | [≤ 0.12] NOTE 4 | N/A | [0.4] NOTE 5 | [≤ 0.6] NOTE 4 | [50] | [≤ 0.2] | [≤ 10] | [<1] | [<1] | [1<X<7] (NOTE 6 ) |
| ADAS [Short range Radar] | Indoor (parking space)/Outdoor | [95] | [≤2.6] (NOTE 3) | ≤0.5 | [≤ 0.12] (NOTE 4) | N/A | [0.4] (NOTE 5) | [≤ 0.6] (NOTE 4) | [20] | [≤ 0.05] | [≤ 10] | [<1] | [<1] | [1<X<10] (NOTE 6) |
| NOTE 1: Sensing service initiation is the time elapsed from the moment the 6GS receives a request for Sensing from a UE or external Application and the moment the 6GS delivers the requested Sensing result and/or contextual information to the UE or external Application. Sensing resources refresh rate is the rate at which the 6GS should be able to determine the optimal set of resources for the Sensing operations. NOTE 2: Assuming typical max range of 250m, range accuracy of 10cm, azimuth accuracy of ±0.3deg. Positioning accuracy as (Min-Max) within field of view. NOTE 3: Assuming typical max range of 30m, range accuracy of 2cm, azimuth accuracy of ±5deg.Positioning accuracy as (Min-Max) within field of view. NOTE 4: Velocity accuracy and resolution is typically reported for the radial velocity (not absolute H/V velocity). NOTE 5: Range resolution typically reported as 3D ranging distance accuracy. NOTE 6: Depends on vehicle’s speed, values consider a range of speed between 50km/h and 250km/h |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## ----------------------------  end of changes  ---------------------------------------