3GPP TSG SA WG 1 Meeting #110 	S1-252488

19-23 May 2025, Fukuoka, Japan	(revision of S1-252151)

Source:	Qualcomm Technologies

pCR Title:	Update to Use Case on Immersive gaming

Draft Spec:	3GPP TS 22.870 v021

Agenda item:	8.1.6

Document for:	Approval

Contact:	Lola Awoniyi-Oteri <oawoniyi@qti.qualcomm.com>

Abstract: Provide justification for the resolution assumption in Table 9.1.6-1 and remove the EN.

1. Introduction

EN was previously introduced requiring a clarification for the 8K per eye resolution assumption made in the KPI table, Table 9.1.6-1.

2. Reason for Change

We clarify that 8K per eye resolution is indeed required to remove graphics pixelation and provide acceptable XR experience for this use case. The justification and reference is added in NOTE 1 and the EN is removed.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 22.870 v021

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

...

[x] eXtended Reality for NR by By Benoist Sébire, WI Rapporteur RAN2 (Nokia) and Huilin Xu, WI Rapporteur RAN1 (Qualcomm), https://www.3gpp.org/technologies/xr-nr

* * * Next Change * * * *

### 9.1.6	Potential New Requirements needed to support the use case

Table 9.1.6-1: Key Performance Indicator (KPI) for Immersive Gaming

| Characteristic parameter (KPI) |  |  |  | Influence quantity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability | Area Traffic capacity | Message size (byte) | Transfer Interval | Positioning accuracy | UE Speed | Service Area |  |
| Compute flows: [5 ms - 20 ms] Conversational and game state flows: [50 - 100 ms] Streaming flows: [200 - 300 ms] | Player/Cheerleader DL: [640 Mbps] (Note 1) UL: [100 Mbps] (Note 2) Spectator: DL: [320 Mbps] (Note 3) UL: [10 Mbps] (Note 4) | [99.9 – 99.99 %] | N/A | N/A | N/A | [≤10 cm] | Stationary, pedestrian | [38 m x 15 m] (Note 5) |  |
| NOTE 1: This data rate is estimated assuming - 2 eye buffers, 8K video resolution, refresh rate of 120 fps, compression ratio of 300 and 8 bits per color. It is important to note that the data rates may change under different assumptions. Data Rate = Video resolution*(Bits per color*3)*Refresh rate* # of eye buffer/(compression ratio). The frame rate of 120 fps is assumed as it has been shown that such high frame rate help reduce the probability of simulator sickness [15]. A resolution of 8K (8196 x4096) per eye can help to remove graphics pixelation and provide good XR user experience [x]. High refresh rates (eg 120 fps) are very correlated and inter prediction between frames increase compression. Some codecs (eg MV-HEVC) may further drop the bitrate requirement. NOTE 2: Significantly higher compared to 5G to enable sensor sharing for split computation. NOTE 3: DL data rate for spectator is assumed to be 2D 8k video NOTE 4: UL tracking for spectator not as intensive as the player/cheerleader NOTE 5: Average basketball court is 28 m x 15 m (adding 5m on both sides for the spectator seating) to make a 38 m x 15 m basketball gymnasium. |  |  |  |  |  |  |  |  |

* * * End of Change * * * *