3GPP TSG SA WG 1 Meeting #110 	S1-252491

19-23 May 2025, Fukuoka, Japan	(revision of S1-252189)

Source:	CATT, NTT Docomo, OPPO

pCR Title:	Update on 9.5 collaborative service in multi-site involved immersive communication

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes to update use case on collaborative service in multi-site involved immersive communication to address more potential requirements associated with the computing service.

1. Introduction

In SA1 #109 meeting, the use case on collaborative service in multi-site involved immersive communication was agreed and captured in TR22.870 clause 9.5 but leave an editor’s note to clarify the definition of computing service. With more study, additional requirements related to computing service need to be captured to support this use case.

2. Reason for Change

Introduce additional potential requirements identified from the use case.

Change from 2189 to 2491

Reword PR2 with the following points

Use “UE mobility” instead of “the user moves”

Delete the example “e.g. caused by the workload balancing”

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.870 v0.2.1

* * * First Change * * * *

## 9.5	Use case on collaborative service in multi-site involved immersive communication

### 9.5.1	Description

Compared to AR/VR services under existing 5G networks, 6G immersive XR services aim to provide an ultimate user experience through capabilities such as ultra-high resolution, high frame rate, wide color gamut, high dynamic range, wide field of view, and advanced encoding/compression techniques. It supports more natural interaction methods (e.g., voice interaction, gesture interaction, head interaction, eye tracking) to achieve complex perception of users and environments. The 6G system and service platform need to collaborate to complete functions such as rendering, synchronization, encoding, distribution, storage and management of immersive XR services, and support real-time transmission and processing of panoramic video data, FOV video data, etc.

6G immersive XR services present new challenges for platforms, networks and terminals. Based on 3GPP R17 XR project research, current XR service data rates are 30/45Mbps downlink and 10Mbps uplink for AR scenarios; future requirements with increased video resolution and frame rate can reach Gbps levels. While data volumes increase, immersive XR services have business data characteristics of variable packet size, non-integer period, jitter and multiple streams. Concurrent traditional data flows (signaling flows, audio streams, video streams) also exist in the network. In addition to stronger computing capabilities, ensuring real-time transmission of relevant data within the 6G system between various processing nodes is also a key aspect. The timeliness of data transmission will impact the success rate and quality of service, so 6G networks must ensure efficient transmission of large volumes of data to maintain service level agreements

This case demonstrates the 6G network's ability to seamlessly integrate diverse technologies and orchestrate multiple distributed network entities/functionalities with computing resource collaboration for support multi-site involved immersive communications.

### 9.5.2	Pre-conditions

- Alice, Bob, Tom and Lisa are customers of Network Operator A, a 6G service provider.

- On New Year's Eve, a public event will jointly held across two cities - City A and City B. Participants had the option to attend in-person at the event venues for massive gatherings, or experience an immersive viewing through wireless VR devices to feel as if present.

- Alice attended on-site at the City A venue. Bob attended on-site at the City B venue while Tom watched the event in the bus from City B to City A. Lisa experienced the event remotely from her home in City C through a wireless VR device.

- Besides providing communication service to enable this use case, operator A's 6G network also provides   computing resources to fulfill demands on immersive presentation. Rendering, encoder/decoder, and other capabilities are also pre-deployed in the network near the event venues in City A and City B to support third-party application requirements.

### 9.5.3	Service Flows

Sub-scenario 1 (Immersive Program Viewing):

For example, two singers perform the same song on stages located in City A and City B respectively. Through the immersive viewing service, Alice, Bob, Tom and Lisa - attending from different physical locations using different means - perceive the experience as if the two singers were performing on a single stage together.

- Local performances are presented to on-site audiences in City A and City B event venues.

- Meanwhile, multi-dimensional data (such as audio, video, stage view, audience seating view, 3D models, motion, etc.) from the City B venue is being acquired.

- On-site City B multi-dimensional data is being collected, and this massive holographic data needs to go through preprocessing (such as data cleaning, compression and encoding) before rendering operations. Rendering servers deployed nearby on the 6G network perform the complex image rendering computations required. As the core network nodes deployed at the City B venue are also serving traditional communications services for the on-site audience, when the 6G network predicts that the computing resources in local node (e.g. core network nodes with computing resources, edge server) cannot meet the rendering server's demands, it can distribute some of the rendering tasks to other  core network nodes with computing resources or edge server with lighter loads. During the collaborative rendering process, there will be intermediary data interactions between collaborative nodes (core network nodes with computing resources or edge server) to ensure the integrity and consistency of the rendering results. These intermediary data interactions can be between any collaborative nodes (core network nodes with computing resources or edge server) when needed and is different from the PDU session traffic which is on a pre-established connection between starting and destination points, and the 6G network must ensure the effectiveness and correctness of these data interactions

- The rendered data is being transmitted to City A through the 6G network and is then holographically presented to the on-site audience in City A after decoding and a series of other processes.

- In the opposite direction, the holographic data from the City A venue is being processed through the 6G network and delivered to the City B venue for holographic presentation. Similarly, in this process, the 6G network also needs to be able to support orchestrating computing resources across multiple rendering servers or multiple core network nodes to complete the rendering task of the holographic data collected from the City A performance venue. During the collaborative rendering process, there will be intermediary data interactions between collaborative nodes (core network nodes with computing resources or edge server) to ensure the integrity and consistency of the rendering results. These intermediary data interactions can be between any collaborative nodes (core network nodes with computing resources or edge server) when needed and is different from the PDU session traffic which is on a pre-established connection between starting and destination points, and the 6G network must ensure the effectiveness and correctness of these data interactions.

- The 6G network also needs to collaborate with remote audience service VR application servers to render and fuse the data processed from Cities A and B into a global stage scene data and then distribute it to wireless VR devices located in different locations for remote audiences.

- All these data are used to reconstruct through immersive services and presented via Lisa’s wireless VR device.

- During Tom’s bus moves from City B to City A, Tom can keep receiving the render data from 6G network with the requested quality of immersive presentation on VR devices, and enjoy the live show as he is in the event venue.

During above steps, the 6G network needs support efficiently deliver the massive media data for immersive communications as well as the intermediary data for large-scale rendering services to ensure the real time experience for all audience on-site and remote accessing.

Sub-scenario 2 (regular Communication Services Remain Unaffected):

Throughout the duration of the event, audiences' regular communication services such as making phone calls, sending text messages, and IMS multimedia services are unaffected.

- Alice shares her experience at the event venue with friends through a video call, and the video interaction remains smooth without issues like audio-video desynchronization or stuttering.

- Bob successfully receives congratulatory New Year greeting messages sent by his friends from all the country, without delays.

- A phone call from Lisa's friend reaches her successfully, and the voice calling service quality does not degrade.

During this event, there are substantial concurrent up-streams and down-streams of multiple data types (user interaction sensing data, AI algorithm data, AR/VR interaction data) which poses challenges for 6G networks. 6G networks must not only function as a data conduit, but also enable required real-time data processing operations to ensure user experience.

### 9.5.4	Post-conditions

- Alice, Bob, Tom and Lisa have an equivalent viewing and interactive experience during the event.

- Alice, Bob, Tom and Lisa's regular communication services are unaffected during their participation in the event.

### 9.5.5	Existing features partly or fully covering the use case functionality

Currently, communication requirements as described in 22.261 clause 7.1, 7.6.1 and 7.10 are provided. And it is assumed that there are always sufficient computing resources at the selected one node for running rendering server. 5G network lack mechanisms for intermediary data for collaborative tasks in large-scale rendering.

### 9.5.6	Potential New Requirements needed to support the use case

[PR 9.5.6-1] Subject to operator’s policy and regulation, the 6G network shall be able to provide computing service controlled by the core network of 6G system to an authorized third-party for supporting rendering.

Editor’s Note: Computing service definition is FFS.

[PR 9.5.6-2] Subject to operator’s policy, the 6G network shall be able to minimize the service interruption time of a computing service used by a UE for XR rendering offloading during UE mobility and/or when the computing resource changes within core network or Service Hosting Environment.

* * * End of Changes * * * *