3GPP TSG SA WG 1 Meeting #110 	S1-252560

Fukuoka, Japan, 19 - 23 May 2025	(revision of S1-252105)

Title:	Use case on Ubiquitous and Resilient Network

Agenda item:	8.1.5

Source:	Orange

Contact:	Philippe Lottin (philippe.lottin (@) orange.com)

Abstract: This contribution provides an update of the use case on Ubiquitous and Resilient Network taking into account the results of the deliverable D1.4 “6G Value, Requirements and Ecosystem” from the Hexa-X-II project.

1. Introduction

The use case on Ubiquitous and Resilient Network in TR 22.870 corresponds to scenario S1 Connectivity at Remote Locations developed in the Ubiquitous and Resilient Network | Representative Use Case in the Hexa-X-II project deliverable.

This contribution provides an update on the use case for ubiquitous and resilient networks in the Technical Report TR 22.870, taking into account the results of the Hexa-X-II project.

2. Reason for Change

The values in the key performance indicator table was consolidated in the Hexa-X-II project. It is proposed to align the key performance indicator table in the 3GPP TR 22.870 standard accordingly.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

### 8.1.6	Potential New Requirements needed to support the use case

[PR 8.1.6-1]: The 6G system shall be able to support the following KPIs:

Editor's note: KPIs are FFS and need to be further justified.

Table 8.1.6-1: Ubiquitous Network KPIs

| End-to-end latency [ms] | Availability [%] | Reliability [%] | User experienced data rate [Mb/s] | Connection density [devices/m2] | Coverage [%] |
| --- | --- | --- | --- | --- | --- |
| [10-500] | [98.5] | [99.9 –99.999] | [0.1 –25] Downlink/ [2] Uplink | [0.1] | [99.9] of earth area with human activity and integrated network (Note 1) |
| Note 1: Coverage means both extending the range of terrestrial networks and reaching larger cell size, but also the percentage of coverage of a given area, combining the different types of access, TN and NTN. The definition of environment should cover inhabited areas, but with human activity, such as highways or cruise ships. |  |  |  |  |  |

| Mobility [km/h] | Location accuracy [m] | Positioning availability [%] | Positioning latency [s] |
| --- | --- | --- | --- |
| up to [120] | Low (≈ [10]) | [99] | [1] |