3GPP TSG SA WG 1 Meeting #110 	S1-252611

19-23 May 2025, Fukuoka, Japan	(revision of S1-252145)

Title:	pCR on use case: Use case on task driven networking and communication for ship

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda Item:	8.1.8

Source:	CMCC

Document for:	Approval

Contact:	Pengtai Qin, qinpengtai@chinamobile.com

Rong Wang, wangrongyjy@chinamobile.com

Abstract: This document provides a Text Proposal for use case on task driven networking and communication for ship.

1. Introduction

It is proposed to add new use case:  Use case on task driven networking and communication for ship.

.

2. Reason for Change

This contribution proposes the following changes:

- Add new use case and new requirements.

3. Conclusions

With these changes, progress can be made for Clause for Vertical.

4. Proposal

It is proposed to agree the following update in this use case to 3GPP TR 22.870.

* * * First Change * * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AIS	Automatic Identification System

DT	Digital Twin

GPU	Graphics Processing Unit

LLM	Large Language Model

LMM	Large Multimodal Model

- IoT        	Massive Internet of Things

- MMSI			Maritime Mobile Service Identity

NDT	Network Digital Twin

NLP	Natural Language Processing

PII	Personal Identifiable Information

RAG	Retrieval-Augmented Generation

SLM	Small Language Model

UAM	Urban Air Mobility

UAV	Uncrewed Aerial Vehicle

* * * Second Change(all new) * * * *

## 11.x	Use case on task driven networking and communication for ship

### 11.x.1	Description

Maritime and inland waterway transportation are the primary means of cargo transport worldwide. Over two-thirds of global international trade volume and approximately 90% of China's import and export freight are transported by sea and river routes.

As a core maritime communication tool, the MMSI (Maritime Mobile Service Identity) and ship status, such as position (latitude/longitude), speed, course, ship type, etc, is broadcasted by AIS to uniquely identify vessels and Assists maritime authorities in monitoring congestion, optimizing traffic flow, and locating vessels during search and rescue operations. The current communication has following characteristics and limitations:

- Low communication quality assurance: the public Very High Frequency bands (161.975-162.025 MHz) are susceptible to interference in complex marine environments or severe weather, causing signal attenuation or packet loss.

- Low Data Update Frequency: depending on vessel status (moving/stationary), the ship information update frequency ranges from 2 seconds to several minutes. In low-priority modes, static information updates can take up to 6 minutes, limiting real-time monitoring capabilities and posing risks in collision avoidance scenarios.

- Data Collisions in High-Traffic Areas. In high-density maritime environments (e.g., ports or congested waterways), existing ship-to-ship communication protocols can result in packet collisions.

- Limited Data Payload Capacity. Due to the limited payload length of the current communication protocol, it cannot support the high-resolution sensor data or multimedia information required by intelligent shipping systems.

However, as the maritime industry advances toward intelligent shipping systems, vessels require high frequency and high efficient multi-modal data transmission, e.g., the river management department requires for real-time (with a precision of seconds) monitoring of ship dynamics; the law enforcement vessels expect to dynamically form a temporary working groups with other vessels in the surrounding areas, or dynamically retrieve multi-modal data of rescued vessels, such as video streams. In such case, the task driven high frequency and dynamic networking for ships is required to ensure effective ship multimedia information exchange at nearshore areas.

### 11.x.2	Pre-conditions

Network Operator MM provides "task driven ship information exchange" service with 6G system.

Network Operator MM has deployed 6G base stations along the river to monitor ships, collaborating with the river management department of government.

According to the river management department policy, Henry subscribes this "task driven ship information exchange" service from Network Operator MM.

### 11.x.3	Service Flows

Service flow1: location-based ship data transmission task.

1. The river between City A and City B hosts mixed vessel types, including cargo ships (e.g., Henry’s vessel) and tourist boats. Henry frequently operates a ship on it to transport goods.

2. The 6G base stations on the sides of the river, deployed by Network Operator MM, could transmit and receive Henry’s high-frequency data (MMSI, position, speed, heading) alongside multi-modal sensor streams (e.g., 360 video feeds, cargo load sensors, engine diagnostics). Data updates occur at 1 times/sec, leveraging 6G’s high bandwidth and low latency, and due to the constantly moving position of ships and the differentiated QoS requirements of multi-modal sensor streams, the core network needs to support planning different user plane paths for different data streams and updating in real-time for high-speed data streams to ensure optimal path transmission.

3. The river management department aggregates 6G-transmitted data to monitor real-time vessel profiles such as MMSI-authenticated identity, precise location (6G-assisted positioning), ship operational status (e.g., engine health, cargo temperature).

4. To achieve efficient ship data sharing for nearby vessels, the river management department issues the location-based ship data transmission task to broadcast the ship’s high frequency information to all the other ships based on the ship's position and the relative distance (e.g., within 6 kilometres). Then the 6G network locates other ships within its designated area, duplicates the ship data and transmits them to the target ships via the dynamically created user plane path.

Service flow 2: Issuing emergency navigation channel management task.

1. The river management department has temporarily established a forbidden zone for ships on the river and all the vessles approaching this area should have a new navigation channel.

2. The river management department issues an emergency navigation channel management task to update navigation routes for vessels approaching the restricted area within 6 km. Then the 6G network prioritizes these updates as URLLC traffic, ensuring <5 ms end-to-end delivery to all the affected vessels until the task cancelled.

Service flow 3: anomaly detection task.

1. The river management department do the anomaly detection by integrating the sensing data as specified in clause 7.8 and the ship information reported as specified in service flow 1.

2. If discrepancies exceed thresholds (e.g., >5% length/ mismatch or unauthorized MMSI usage), an alerts are triggered for investigation.

3. If the vessels detected by perception systems without valid MMSI or transmitting inconsistent AIS statuses (e.g., "anchored" but moving) are flagged as potential threats.

4. If the reported location from a vessel is not aligned with the corresponding perception data (e.g., vessel claims presence at [X,Y] but no LiDAR/camera confirmation), the river management department issues an 6G positioning verification task. 6G network start the ship positioning to get the real location, with results feedback to the river management department.

Service flow4: temporary ship grouping task.

1. Henry's vessel was assigned as the leader of a high-stakes cargo transport mission. To navigate through a complex waterway, the leader ship initiated a temporary group formation with 5 cargo ships, aiming to enhance operational efficiency and safety.

2. Henry’s vessel, as the captain of this high-stakes cargo transport mission, issues a temporary ship group task to 6G network and ask for priority QoS for leader-to-tail ship communication. Real-time geofencing alert when any vessel exceeds 500m trailing distance.

3. The 6G system received the task and performed multi-dimensional resource orchestration: Created an isolated network with deterministic latency and electronic fence, while continuously adapting networking parameters based on the ships' real-time positions along the waterway, ensuring uninterrupted connectivity for maritime services.

### 11.x.4	Post-conditions

The river management department monitors the real-time vessel navigation status and conducts anomaly detection to identify abnormal behaviors instantly, ensuring timely intervention and maritime safety.

Henry safely completed the high-stakes cargo transport mission. Thanks for the 6G system, which provides ship tracking and task-driven services to both ships and river management department.

### 11.x.5	Existing features partly or fully covering the use case functionality

TS 22.261[14] defines the 5G LAN-type service requirements to provide services with similar functionalities to Local Area Networks (LANs).

TS 22.261[14] defines the positioning service requirements to support verticals and applications with positioning accuracies better than 10 meters.

### 11.x.6	Potential New Requirements needed to support the use case

[PR 11.x.6-1] Subject to regulatory requirements, operator’s policy and user-consent, the 6G system shall be able to support dynamic group communication for authorized UE(s) according to the received location-based data transmission request issued by authorized UE or trust third party.

NOTE: Location-based data transmission requires to identify the target communication UE(s) based on location, relative distance and establish communication for data transmission.

Editor’s Note: It’ s FFS on the KPI of this use case.

* * * End of Change * * * *