3GPP TSG SA WG 1 Meeting #110 	S1-252615

Fukuoka, Japan, 19-23 May 2025	(Revision of S1-250152, S1-252221)

Title:	Use Case on Remote and Automatic Construction

Agenda item:	8.1.8

Source:	NICT, ESA

Contact:	Shiro Mazawa (shiro.mazawa@nict.go.jp)
Motoaki Hara (hara.motoaki@nict.go.jp)

Abstract: This contribution presents a use case for a remote and automatic construction scenario that includes remote installation with/by experienced technical workers and unmanned construction.

Background

In XGMF white paper, several potential applications for Beyond 5G in the construction sector are described.  These include remote installation with/by experienced technical workers and unmanned construction.  To realize these applications, ultra-low latency, ultra-rigid time synchronization, and ultra-precise positioning are required in outdoor location.

This contribution proposes the use case in construction sector to be included in the study on 6G Use Cases and Service Requirements.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1]	XGMF:” Beyond 5G White Paper ~Message to the 2030s~” (https://xgmf.jp/wp-content/uploads/2025/04/Beyond_5G_White_Paper_v4_0.pdf).

* * * Second Change * * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ICT	Information and Communication Technology

RTK	Real-Time Kinematic

* * * Third Change * * * *

Followings are all new texts.

11 Further Use Cases on Industry and Verticals

11.x	Use case on Remote and Automatic Construction

11.x.1	Description

According to [x1], the use of ICT in the construction industry has become a pressing issue. Many of the tasks are performed outdoors and usually involve many workers and organizations in various construction processes. Work planning, management, and safety are also important issues because workers operate machinery such as trucks and heavy construction equipment and handle hazardous materials like chemicals and heavy materials. Extensive use of ICT to monitor workers and construction equipment, remotely control heavy equipment, and automate site operations will enable sustainable and efficient construction operations and site safety.  These ICT-related initiatives are collectively referred to as “Construction 4.0”.

The future construction sector is expected to introduce innovative technologies, such as advanced telecommunication, AI, robots, and cloud computing, into the construction and infrastructure domains, and to take initiatives such as on-site verification of unmanned construction technologies, technologies to support construction site workers, and labour-saving supervision and inspection using video data.  Unmanned construction using 3GPP systems with “high speed and high capacity,” “multiple connectivity,” and “low latency” capabilities will be effective in improving productivity in the construction sector.

The possible applications of remote and automatic construction are as follows:

- Remote installation with cooperation of experienced technical workers – On-site workers use VR technology  to perform installation work with the assistance of experienced technical workers located in remote locations.

- Remote installation by experienced technical workers - Installation by remote operation of construction machinery or robots with using haptics and VR technologies by experienced technical workers.

- Unmanned construction, in which construction machinery and robots carry out the construction and building process;
- Autonomous-driving vehicles carry equipment on behalf of workers, 
- Automatic construction machinery carry out the installation, and 
- Robots perform the instalation automatically.

- Construction harmonized physical space and cyberspace - Design is carried out in cyberspace and reflected to the actual construction.  Actual construction in physical space is proceeded under the virtual construction in cyberspace.  Any issues in the preparation of construction materials and in the site environment are identified while non-3gpp environmental sensors detect temperature, humidity, gas emission, etc. An appropriate measures are taken to those issues.

RTK-GNSS (Real-time kinematic positioning) is widely used in construction sector.  RTK-GNSS can provide cm-level accuracy in open-air environments, but can hardly be used in the non-line-of-sight situations like urban canyons.  There is an afraid of jamming and spoofing.  Also, high cost of RTK-GNSS receiver is an issue.

11.x.2	Pre-conditions

Construction contractor A is contracted to perform construction work at construction site B with stakeholder C. Company A's ICT construction machinery and UAVs are equipped with 3GPP communications devices. Mobile carrier D installs base stations to cover construction site B.

Figure 11.x.2-1: Remote and Automatic Construction

11.x.3	Service Flows

- Surveying – Construction contractor A quickly and accurately assesses the situation at construction site B before construction, by using UAVs equipped with 3GPP communication devices, cameras, laser scanners, etc.  3gpp communication devices are used for communication and measurement of positioning.

- Design – Construction contractor A creates 3D design data based on the survey data of construction site B and makes plans of facilities.

- Construction - Construction contractor A performs construction work at construction site B using the following two types of ICT construction machinery equipped with 3gpp communication devices and non-3gpp sensors, based on real-time measurement and analysis of site conditions and the 3D design data.  3gpp communication devices are used for communication and measurement of positioning.
- Machine Guidance – ICT construction machinery are operated by human operators.
- Machine Control – ICT construction machinery are operated semi-automatically.

- Construction Management – After completion of construction, construction contractor A accurately assesses the situation of construction site B after construction, by using UAVs equipped with 3gpp communication devices, cameras, laser scanners, etc, to confirm whether the construction has been carried out as designed.

- Delivery – Construction contractor A shares the measurement data of construction site B with stakeholder C.

11.x.4	Post-conditions

Construction contractor A completes construction work at construction site B with higher quality, fewer people, and more safety.

11.x.5	Existing features partly or fully covering the use case functionality

RTK-GNSS (Real-time kinematic positioning) is widely used in construction sector.  RTK-GNSS can provide cm-level accuracy in open-air environments, but can hardly be used in the non-line-of-sight situations like urban canyons.  There is an afraid of jamming and spoofing.  Also, high cost of RTK-GNSS receiver is an issue.  In 3GPP, using Positioning Reference Unit, cm-level accuracy is achieved, but it is for indoors.  However, cm-level accuracy in outdoor locations is required in the use case, especially the use case that multiple ICT construction machinery work collaboratively.

11.x.6	Potential New Requirements needed to support the use case

[PR.11.x.6-1] The 6G system shall be able to support the following KPIs utilising the 3GPP and non-3GPP positioning technologies.

Table 11.x.6-1: KPIs for remote and automatic construction

| Scenario | Positioning Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Machine Guidance, Machine Control | [2-10] cm (Note 1) | [10] cm | [99%] | [30]s | Outdoor | [5]km/h | Machinery mounted |
| Note 1: Assuming the similar performance as RTK-GNSS. |  |  |  |  |  |  |  |

.