3GPP TSG-SA WG1 Meeting #110 	S1-252632

Fukuoka, Japan, 19-23 May 2025	(revision of S1-252264, 2617)

Title:	Use case on Network-Requested Execution of Service Functions in Connected Vehicles

Agenda Item:	8.1.8 (Further Use Cases on Industry and Verticals)

Source:	Lenovo

Contact:	Apostolis Salkintzis (y1026c at lenovo.com)

Abstract: This contribution proposes a use case that enables authorized vertical users, such as transport authorities, to request the 6G network to deploy and execute temporary service functions (e.g., tasks) in connected vehicles located in a specific area. These functions/tasks, executed only in vehicles that have indicated support for this capability, can assist in collecting real-time data (e.g., location, sensor information) to enhance situational awareness during events such as road accidents.

NOTE:    	This contribution has been partially supported by the HORIZON-JU-SNS-2023 EU project ENVELOPE (GA Number 101139048).

***********   New Section *********

## 11.x	Use case on Network-Requested Execution of Service Functions in Connected Vehicles

### 11.x.1	Description

In this use case, an authorized vertical user (e.g., road operator, transport authority) requests the 6G system to deploy and execute a service function (e.g. a tasks) in one or more connected vehicles located in a specific area. These service functions are temporary and executed only for the duration and purpose of the request. They can be deployed and executed only in vehicles that have indicated support for this capability.

For example, in the case of a severe car accident, a transport authority may request the 6G network to deploy and execute a service function in vehicles located near the accident. The purpose of this function may be to collect situational data and assist in building real-time awareness of road and traffic conditions.

After the service function is deployed and executed in a vehicle, it may collect data such as:

- GPS location, speed, and heading,

- Lidar or camera sensor metadata,

- Proximity to the accident zone or road obstructions,

- Other relevant environmental or telemetry data.

The service function in a vehicle may periodically transmit this data to the vertical user’s server through the 6G network. This data can be used by the transport authority to reconstruct the event timeline, monitor traffic congestion buildup, identify potential risks, and coordinate emergency response actions more effectively. It may also be used to support other applications such as updating navigation guidance, informing traffic broadcasts, or initiating adaptive traffic control measures in surrounding areas.

### 11.x.2	Pre-conditions

- The connected vehicles are registered with the 6G system and have indicated support for executing network-requested service functions.

- The vertical user is authorized to request service function execution (e.g., through a service exposure interface).

- The 6G network can determine the location of connected vehicles and assess their availability and suitability based on predefined metrics (e.g., capability, battery level, load).

### 11.x.3	Service Flows

1)	A vertical user initiates a request to the 6G network to deploy and execute a service function in vehicles located in a target area.

2)	The 6G network identifies connected vehicles in this area that are eligible and available to execute the requested function.

3)	The 6G network sends a request to the selected vehicles to deploy and execute the service function. This request indicates also where the service function is stored and how it can be downloaded.

4)	Each vehicle that receives (and accepts) the request downloads the service function (e.g. from a network repository) and executes it locally. User consent may be requested before a service function is accepted and executed in a vehicle.

5)	The service function begins collecting sensor data (e.g. GPS location, speed, etc.).

6)	The data is transmitted periodically or on event basis from the service function to a vertical user’s server via the 6G network.

7)	The service function may be terminated in a vehicle either automatically (e.g., after a predefined period) or explicitly by the vertical user or by the 6G system.

### 11.x.4	Post-conditions

- The service function is terminated (and possibly removed) from each vehicle.

- The collected data has been transmitted to the vertical user’s system for further processing or action.

- The 6G network may log the participation of vehicles in the service for auditing or service tracking purposes.

### 11.x.5	Existing features partly or fully covering the use case functionality

Existing specifications (e.g. TS 22.261 [14] and TS 22.186 [66]) support the concept of service exposure to verticals and enhanced V2X services. They also include support for communication between vehicles and edge applications.

However, existing specifications do not fully support the dynamic, network-requested execution of temporary service functions in vehicles.

### 11.x.6	Potential New Requirements needed to support the use case

[PR-11.x.6-1]: Subject to operator policy, regulatory requirements and user consent, the 6G network shall provide suitable APIs to allow authorized third parties to collect data from UEs (e.g., connected vehicles) that are located in a specific area and are capable of collecting data upon network request.

[PR-11.x.6-2]: Subject to operator policy, regulatory requirements and user consent, the 6G network shall enable UEs (e.g., connected vehicles) to indicate whether they can collect data upon network request.