3GPP TSG-SA WG1 Meeting #110	S1-252833

19-23 May 2025, Fukuoka, Japan	(revision of S1-252773, 2121)

Source:	vivo

pCR Title:	Update use case on home robots

Draft Spec:	3GPP TR 22.870 v0.2.1

Agenda item:	8.1.3	Artificial Intelligence

Document for:	Approval

Contact:	Yanchao Kang, kangyanchao@vivo.com

Hank Yu, yuhang.txyjy@vivo.com

Abstract: this contribution updates the definition of AI service and the use case on home robots to add several new PRs.

* * * First Change * * * *

6.9	Use case on home robots

6.9.1	Description

In 6G era home robots will engage in household chores based on preconfigured models, such as sweeping the floor, vacuuming, folding clothes, washing dishes, and organizing rooms. They will also take care of family members by monitoring health data, reminding medication, and dialling emergency calls. Additionally, they will socialize and entertain with humans while interacting with other smart devices to create a more intelligent ecosystem. All of this aims to bring us a more convenient, comfortable, and safe family life. In this manuscript, all or part of the AI inference services are provided by home robots.

Figure 6.9.1-1: Home robot performs specific tasks based on pre-trained models

As shown in Figure 6.9.1-2, the working principle of the robot includes two main components: perceive and action. "Perceive" refers to the process of sensor data from, such as, cameras or radar, for the input to a large model and obtaining perception result. "Action" refers to the process of taking perception result and task as input to a large model and obtaining action parameters to control the joints of the robot to complete the corresponding action.

task

Sensing data

Perception result

Perceive

Action parameters

Action

Figure 6.9.1-2: The perception-action loop of a home robot

In the case of specific tasks in specific scenarios, the perception model can be deployed locally on the robot as its size is not large. However, for general tasks in open environments, the perception model needs to achieve the perception ability similar to an adult human brain towards the world. The number of neurons in an adult human brain is approximately 100 billion, and the current model size for complex logical reasoning and task orchestration in laboratory or limited environments, the model size is also generally on the order of 100B. Due to cost and size limitations, robots are unable to locally implement perception functions for free and open environments.

The 6G network will be equipped with perception models, enabling it to provide perception services to robots in open environments whenever necessary.

This use case aims to provide timely perception and action guidance via the 6G network, assisting robots cope with complex real-world environments and provide users with better intelligent services.

6.9.2	Pre-conditions

The Operator X offers comprehensive 6G services for robots, including communication and AI services. Relevant resources have been deployed in the network to facilitate services like large model inference (e.g., using the AI/ML models are trained by core network entity or provided by 3rd Party), assisting robots in delivering enhanced performance in complex environments.

Lily purchases a robot to assist with various house chores. The robot exhibits a certain degree of local intelligence. By equipping it with one or more advanced models, the robot can better understand the owner’s basic needs by controlling its body joints. For example, it can help with household chores, take care of family members’ well-being, engage in social interactions and entertainment with humans, as well as interact seamlessly with other smart devices.

Lily has subscribed to specific 6G services (communication and AI services) for her robot. Therefore, the robot is able to access to 6G services, such as perception service.

The robot has two perception modes as shown in figure.6.9.2-1:

- Perception mode a) for specific task locally in robot; and

- perception mode b) for general task in open environments via 6G network.

(a) perception for specific task locally in robot

(b) perception for general task in open environments via 6G network

Figure 6.9.2-1: Two perception modes of a home robot: (a) perform local perception for specific task, and (b) perform perception for general task in open environments via 6G network

When a robot is instructed by its owner to start general tasks in an open environment, it will request the perception service for the open environment from the 6G network (e.g., using the AI/ML models trained by core network entity or provided by 3rd Party), and, the robot will interact with 6G more frequently the 6G network, which helps ensure safer behaviour through perception services.

6.9.3	Service Flows

Step 1: One day, while on a shopping trip with Lily and her child, the home robot carried a newly purchased wine glass that Lily had carefully selected. At this time, the robot follows Lily, moves when Lily moves and stops when Lily stops, holding the newly purchased wine glasses. Therefore, it is Perception mode a), and only the local model on the robot is needed to work, as shown in Figure 6.9.3-1(a). However, when an urgent phone call interrupted their outing, and Lily tells the robot that she is going to take a call and asks the robot to watch the child. The robot finds that taking care of the child is a general task in an open environment, so it switches to perception mode b), which the robot cannot support locally, as shown in Figure 6.9.3-1(b).

(b) perception for general task in open environments

(a) perception for specific task locally in robot

I'm going to take a call, please watch the kids for me.

Follow me and help me carry the wine glasses.

Figure 6.9.3-1: The robot switches perception mode from perception node a) to perception mode b) f after lily’s instruction

Step 2: Considering the AI service capability supported by network (e.g., inference for perception service) and AI model(s) supported by network, the home robot requests the perception service for general task in open environments from the 6G network and frequently sends locally collected important information to the 6G network. The request message includes the perception requirements/demand (e.g., latency, inference accuracy) from the home robot. Then the 6G network determines whether the perception requirements can be satisfied taking UE demand in the request message into account and then sends the response (e.g., acknowledgement including promised requirements) to the home robot.

Step 3: (Perception) The 6G network utilizes the sensing data transmitted by the robot, optionally incorporating environmental information from 6G network itself, as input for the model inference (e.g., using the AI/ML models trained by core network entity or provided by 3rd Party). When the 6G network perceives a normal situation, and it will not interrupt robot’s current task, as shown in Figure 6.9.3-2 (a); when the 6G network perceives a dangerous situation, such as, the ball suddenly falls from the child's hand and rolls to the side of the road while the child runs to pick up the ball just before a car approaches, it generates corresponding behaviour guidance to send to the robot, as shown in Figure 6.9.3-2 (b).

2. request perception service for open environments, and send sensing data

2. request perception service for open environments, and send sensing data

task

local perception

(b) perception-action loop when dangerous situation is detected by 6G network

(a) perception-action loop when normal situation is detected by 6G network

1. switch to perception for open environments

5. Action

Perception of dangerous situation

Perception from 6G network

task

Action parameters

Action parameters

4. Perception result

Sensing data

3. Perceive

3. Perceive

4. ACK

5. Action

Normal situation, not to interrupt the robot’s current task

Sensing data

1. switch to perception for open environments

Figure 6.9.3-2: The perception-action loop when a normal situation or dangerous situation is detected by the 6G network, respectively

Step 4: The 6G network delivers ACK or the perception result to the home robot within the latency included in step 2.

Step 5: (Action) The robot uses local perception result or perception result from the 6G network, along with the task, as input to the local large model to output action parameters for action. The actions corresponding to a dangerous situation detected by the 6G network may include, for example:

Action 1: Drop the box containing the cup;

Action 2: Move towards the child while emitting an alarm sound;

Action 3: Pick up the child and move to a safe area;

Action 4: Explain to Lily what happened and why the box containing the cup was dropped.

Step 6: After executing one or more actions suggested in step 5, the robot assesses the action results. It then reports the service experience (e.g., bad, fair, excellent) along with locally collected environmental data to the network. In this way, the 6G network monitors the performance of the perception service and, if necessary, may upgrade AI model (e.g., by retraining, switching) to guarantee the inference accuracy.

6.9.4	Post-conditions

If the robot relies solely on local model inference, it may encounter system crashes in overly complex situations. Alternatively, if it only considers executing the previous command from the female owner, it may pose a danger to the child. However, by providing timely perception and action guidance through the 6G network, the requirements for the robot's local intelligence level can be reduced. Users can purchase a robot at a lower price and obtain high-intelligence robot services that can more easily adapt to complex real-world environments.

6.9.5	Existing features partly or fully covering the use case functionality

Currently, 3GPP systems only provide communication services, and there is no functionality to provide AI as a service.

6.9.6	Potential New Requirements needed to support the use case

[PR 6.9.6-1] Subject to operator’s policy, the 6G network shall be able to provide the AI service (e.g., AI model inference) to a user.

[PR 6.9.6-2] Subject to operator’s policy, the 6G system shall be able to support negotiation the service performance (e.g., latency, inference accuracy), between UE and 6G network, when providing AI service (e.g., AI model inference) to a user.

[PR 6.9.6-3] Subject to operator’s policy, the 6G network shall be able to support mechanism to guarantee the service performance (e.g., latency, inference accuracy) when providing AI service (e.g., AI model inference) to a user.

* * * End of Change * * * *