3GPP TSG-SA1 Meeting #110	S1-252889

19-23 May 2025, Fukuoka, Japan	(revision of S1-252730, S1-252202)

Source:	China Telecom, Telefonica, KDDI, ZTE, Huawei

pCR Title:	Use case on prevention of signalling storm

Draft Spec:	3GPP TR 22.870

Agenda item:	8.1.2

Document for:	Approval

Contact:	Jianzhao Li, lijz@chinatelecom.cn

Yinglin Chen, chenyl37@chinatelecom.cn

Abstract: This use case proposes to consider a 6G capability of prevention of signalling storm to improve network resilience.

1. Introduction

Please read the use case below.

2. Reason for Change

This is a use case.

3. Conclusions

None.

4. Proposal

It is proposed to study the use case in System and Operation Aspects.

* * *Start of Change (All new text)* * * *

5.4.x	Prevention of signalling storm

5.4.x.1	Description

In 5G, when a network element fails, explicit restoration signalling is triggered to its backup element to maintain UE registration status or service sessions. A data-center-level malfunction can generate signalling traffic peaks tens of times higher than normal, potentially causing network congestion or even cascading failures.

For example, a data center malfunction in a commercial 5GC network resulted in an AMF failure. RAN and other functional network elements detected this failure and simultaneously attempted to restore all affected UEs and sessions. This resulted in a 20-fold surge in registration requests and a 69-fold increase in PDU session establishment requests to the backup AMF—far exceeding normal operational levels, as shown in Figure 5.4.x.1-1 below.

To avoid such signalling storms, the failure the network and its backup’s takeover should be transparent to UE, RAN, and other functional network elements, such that no restoration signalling is needed.

Figure 5.4.x.1-1: The number of registration requests and PDU session establishment requests the backup AMF received during a data-center level malfunction.

5.4.x.2	Pre-conditions

The 6GC is deployed in data center A and data center B.

5.4.x.3	Service Flows

1. Data center A malfuctions due to unexpected incidents.

2. Data center B seamlessly takes over data center A. Messages sent to data center A are automatically routed to data center B. From the perspective of UE, RAN, and other functional network elements, network elements in data center A remain functional. No signaling storm of restoration messages generates.

5.4.x.4	Post-conditions

Messages sent to data center A will be switched back to data center A after data center A is recovered.

5.4.x.5	Existing features partly or fully covering the use case functionality

5G supports restoration procedures, which handle network element failures via its backup, as specified in TS 23.527. However, these procedures require network element re-selection and session re-establishment (e.g., alternative NF re-selection, PFCP sessions re-establishment.)

Context transfer procedures specified in clause 5.21.4 of 23.501 and clause 4.26 of 23.502 allow a backup network element to take over the service from the failed network element.

5.4.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 6G network shall provide a mechanism to prevent signalling storm arising from recovery of network failure.

Editor's Note: This requirement is FFS.

* * *End of Change * * * *