3GPP TSG-SA WG1 Meeting #111 	S1-253134

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	China Mobile, FAU

New use case title:	New use case on Predictive Maintenance in Smart Factories

Draft TS/TR:	3GPP TS / TR 22.870

Agenda item:	8.1.8

Document for:	Approval

Contact:	Xiaonan Shi, shixiaonan@chinamobile.com

Shiying Zheng, zhengshiying@chinamobile.com

Mai Alissa, mai.alissa@fau.de

Abstract: In this use case on Predictive Maintenance in Smart Factories, the focus is on predicting equipment failures through more comprehensive sensing and data collection, thus better to predict if it needs a repair to avoid breaking down. This predictive approach is beneficial because it leverages 6G system capabilities (including ISAC and optional massive sensor access) to analyze acoustic noise spectrums and vibrations, enabling early detection of irregularities and providing comprehensive insights into equipment operating status, which brings additional benefits to factory operations and the communication network (6G system).

The use case on Predictive Maintenance in smart factories has been elaborated in the “German Perspective on 6G –Use Cases, Technical Building Blocks and Requirements” White paper,  which has been deeply studied by FAU (Friedrich-Alexander-Universität Erlangen-Nürnberg) , and also has been elaborated in the 5G-ACIA’s work item (WI087 ‘Early considerations of network capability evolution for industries and Automation’).

---------- Use Case template ----------

11.y	Use case on Predictive Maintenance in smart factories

11.y.1	Description

Predictive maintenance is an advanced maintenance method which involves periodic or continuous monitoring of equipment condition to predict if it needs a repair to avoid breaking down. The method allows to minimize unplanned downtime for otherwise reactive equipment repairs, reduce maintenance costs, maximize equipment lifespan and increase revenues as a result. The use case takes advantage of  the deployed indoor 6G system where its RAN nodes and/or UEs are enabled with the capability to collect RF sensing data (ISAC), optional support for massive sensor access capability. For multiple types of manufacturing equipment it is possible to predict impending failures by analyzing their acoustic noise spectrums. This phenomena is confirmed by experienced factory employees who are able to predict certain types of equipment failures based on their perception of such noises. Such noises manifest themselves as vibrations on equipment surface and can be detected by angle- and distance-resolved Doppler measurement of RF sensing system and assigned to respective parts of the monitored equipment.

Predictive maintenance needs to be based on a large amount of historical data and current real-time data. To achieve best performance,  including accuracy and real-time responsiveness, etc. Without modifying the equipment, ISAC can provide supplementary data to achieve a more comprehensive sensing of the equipment's operating status. Based on these more comrehensive information, better predictive maintenance could be realized, such as detecting irregularities in otherwise nominal operation patterns and identifying types and locations of impending failures.

Implementing predictive maintenance using the described method will minimize unplanned downtime, reduce maintenance efforts, maximize equipment lifespan and thus increase benefits, which could provide services to  companies running manufacturing facilities, operators of automated warehouses, operators of 6G-enabled automated construction sites.

The use case is mainly applicable to private spaces, like inside of factories or warehouses, however it can also be useful for scenarios where a relevant type of equipment that requires monitoring is deployed in public spaces or in their immediate proximity (e.g. 6G-enabled automated construction sites, road construction sites).

11.y.2	Pre-conditions

1.The network supports ISAC capability and optionally supports massive communication capability.

2.If predictive maintenance needs to be carried out by combining data such as equipment operating status, the equipment needs to be connected to a private network.

3.Edge servers are responsible for periodically processing data such as acoustic noise spectrums and/or equipment operating status, and provide decision support for predictive maintenance work.

11.y.3	Service Flows

1. Edge servers monitor  the operating parameters of equipment (such as production line robotic arms, warehouse conveyors, etc.). When potential fault risks (such as excessive bearing wear trends, abnormal motor vibration) are detected based on these sensing data, and then generate maintenance subtasks, publish them to the public task board, and label task characteristics (including fault warning level, required tool type, estimated maintenance duration, whether it falls within the scope of automated maintenance tasks, etc.).

2. For tasks within the scope of automated maintenance, the predictive maintenance results will be provided to the antomated maintenance functional entity, and this fuctional entity issues and executes the tasks via the 6G network, such as pausing equipment operation. For tasks outside the scope of automated maintenance, the task information is sent to management staff.

11.y.4	Post-conditions

Equipment failures can be realized with minimal modifications owing to the immediate support from 6G system. In turn, equipment failures can be detected and resolved in advance. Services can continue to operate, reducing the time for production suspension and maintenance.

11.y.5	Existing features partly or fully covering the use case functionality

In TS 22.137 [6], there are requirements specified for 5G system on integration of sensing and communication.

Based on operator’s policies, operator’s control and regulation, the 5G system shall be able to collect 3GPP sensing data from sensing receivers for processing.

Subject to user consent, regulation, and operator’s policy, the 5G system should support the joint processing of the 3GPP sensing data and non-3GPP sensing data to derive a combined sensing result.

Subject to operator’s policy, the 5G network shall provide secure means for a trusted third-party to request 5G wireless sensing service based on specific parameters (e.g., refresh rate, period of time, sensing KPIs, geographical location) and to receive the corresponding sensing results.

In TS 22.137 [6], no specific requirement indicators are proposed. However, the requirement put forward in other use cases in TR 22.870 already cover those of this use case. Therefore, this use case only proposes specific requirement indicators for the predictive maintenance use case. However, the existing requirements don’t support the higher frequency bandwidth monitoring needed in industry and automation.

11.y.6	Potential New Requirements needed to support the use case

[PR 7.y.6-1] The 6G network shall be able to provide the sensing results for predictive maintenance by detecting the status information (e.g.: acoustic noise/oscillation waveform, component’s location) of equipment in factory as described in the following Table 7.y.6-1:

Table 7.y.6-1:	Performance requirements of sensing results for Predictive Maintenance in smart factories

| Scenario | Sensing service area | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refresh rate [s] | Missed detection [%] | False alarm [%] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Predictive maintenance | Indoor/ outdoor | 95 | £1.5 | £1.5 | N/A | N/A | £1.5 Note1 | N/A | 1000 Note2 | 1 | N/A |  | 5 |
| NOTE1: Range resolution of ≤1.5 m is coming from the fact that up to a 100 MHz of BW can be available in FR1 frequency range. Assuming frequency modulated continuous waveform, Δr = c/(2*BW) provides 1.5m of range resolution. Using larger BW in FR2 – FR4 will enable correspondingly better range resolutions. For example, the 400 MHz waveform in FR2 would enable 38 cm of range resolution. NOTE2: Sensing latency requirement of 1000 ms (so as the refresh rate) is not strict. The changes in the equipment are slowly over time. Also the status of manufacturing equipment is not expected to change rapidly. |  |  |  |  |  |  |  |  |  |  |  |  |  |

[PR 7.y.6-2] Subject to operator’s policy and agreement with trusted 3rd party, the 6G network shall support to utilizing the sensing results to provide analysis result for predictive maintenance services.