3GPP TSG SA WG 1 Meeting #111 	S1-253168r1

25-29 August 2025, Goteborg, Sweden	(revision of S1-25xxxx)

Source:	SoftBank, Nokia

pCR Title:	pCR to update figure and to add PR3 in 6.23

Draft Spec:	3GPP TR 22.870v0.3.1

Agenda item:	8.1.3

Document for:	Approval

Contact:	Shunsuke.Kawashima01 at g.softbank.co.jp

Abstract:

1. Introduction

In SA1 #109 meeting (Fukuoka), PR#1 and PR#2 in Clause 6.23 were agreed, while PR#3 was removed due to unclear wording. An editor’s note was also added to correct the figure of this use case. This contribution provides the figure correction and re-introduces PR#3 in an updated form as PR 6.23.6-3.

2. Reason for Change

Following changes are applied based on comments given at SA1 #109 meeting in Fukuoka.

The figure in Clause 6.23 contained Base stations, which have been removed, and the vague term Computational Resources, which have been replaced by Service Hosting Environments.

PR#3 was removed due to unclear wording. Its intent is now updated and re-introduced as PR#3 to clarify the need for considering energy-related data in Service Hosting Environment selection to optimize energy efficiency and maintain the balance between energy consumption and supply.

3. Conclusions

-

4. Proposal

It is proposed to agree the changes to 3GPP TR 22.870 V0.3.1.

* * * First Change * * * *

6.23	Use case on distributed 6G network for AI computing

6.23.1	Description

As outlined in this document, numerous high demand use cases in 6G networks, such as augmented reality and autonomous vehicles, are anticipated to heavily rely on AI, requiring significant computational load and substantial power consumption. Currently, many network configurations centralize these computational loads in urban areas, with power supply also concentrated in these areas. In the context, it is concerned that the balance between power consumption and supply are collapsed, which bring a power outage in the urban area.

Ideally, a balance between power consumption and supply is achieved. The power supply is physically distributed across different areas, each with certain constraints (e.g. an upper limit on power supply). Power generation changes according to the demand within the coverage area. It is expected that computation can be physically distributed, and the use of computations can be adapted to match the power supply. Therefore, mitigating these imbalances to maximize overall computational resources and energy efficiency is highly recommended, especially in the context of AI workloads.

This use case aims to distribute computational workload by appropriately selecting a computing site by routing traffic, particularly considering the available computational resources and surplus power in each area, while also accounting for the specific needs of AI-related tasks. Operators can create optimal computing site selection policies based on typical power usage patterns and the dynamic demands of AI applications to achieve efficient energy consumption. Furthermore, this use case incorporates low latency requirements per traffic flow in the policy and the consequent optimized routing, which is crucial for many AI-powered services.

Figure 6.23.1-1: Distributed 6G Network for AI Computing - The sites A, B, C are the deployed Service Hosting Environment

6.23.2	Pre-conditions

1.	Operators distribute computational resources across sites and use the electric power from the areas, where these sites are located, to provide computational services.

2.	Third-party electric power monitoring systems provide detailed electric power consumption data per computing site via APIs.

3.	Operators inference the balance of power consumption and supply for specific time periods for each computing site.

4.	Operators can pre-define their own appropriate computing site selection policies based on user attributes (users’ locations, communication time, users’ profiles, etc.) and the latency requirements of the users’ communications.

6.23.3	Service Flows

Based on electric power usage patterns, operators use their own optimal selection policies to select the computing sites and then map computation workloads onto the computing sites. These policies are dynamically adjusted according to the specific location, communication time, users’ profiles, and latency requirements of the users’ communications. The selected computing site from these policies must optimize network load and electric power consumption in real-time, achieving better distributed electric power consumption and improved overall energy efficiency.

6.23.4	Post-conditions

The introduction of the computing site selection policies enhances energy efficiency and achieves balancing of power consumption which is especially important during peak electric power consumption periods. This enables operators to achieve sustainable and energy efficient network operations.

6.23.5	Existing features partly or fully covering the use case functionality

Energy efficiency has been considered in clause 6.15, TS 22.261[14]. However, Considerations on a balance between power consumption and supply are still insufficient, especially in achieving significant energy efficiency improvements and distributing computational resources that are aimed at this use case. Thus, new requirements are further needed.

6.23.6	Potential New Requirements needed to support the use case

[PR 6.23.6-1] The 6G network shall be able to collect energy related data of the Service Hosting Environment.

[PR 6.23.6-2] The 6G network shall be capable of providing appropriate Service Hosting Environment in order to accommodate compute and communication (e.g. traffic load) resources to meet service requirements (e.g. bitrate and latency).

[PR 6.23.6-3] The 6G network shall be able to consider energy related data in Service Hosting Environment selection processes to optimize energy efficiency and maintain the balance between energy consumption and supply.

* * * End of the First Change * * * *