3GPP TSG-SA WG1 Meeting #111 	S1-253208r2

25-29 August 2025, Goteborg, Sweden

Source:	SHURE Europe GmbH

New use case title: New use case on Immersive Audio Production in Live Events

Draft TS/TR:	3GPP TR 22.870 v0.3.1

Agenda item:	8.1.6

Document for:	Approval

Contact:	Axel Schmidt, schmida@shure.com

Abstract:

Immersive audio production at live events aims to deliver a multi-dimensional sound experience to the audience, either on-site or participating via a live stream. The sound capturing for the immersive experience can be done using two types of devices: Type A, which is a microphone array consisting of at least four audio transducers capable of providing immersive audio output or raw data from each transducer, and Type B, a single microphone that features localization capabilities, tight synchronization, and outputs a mono audio stream. Depending on the used devices and the size of the production the mixing of the immersive audio stream happens either in the capturing device itself or in a centralized mixing console. In addition to the audio stream for the audience each artist on stage receives an individual audio stream to monitor their own performance.

## x.1	Use Case on Immersive Audio Production in Live Events

### x.1.1	Description

### Definitions

transfer interval: time difference between two consecutive transfers of application data from an application via the service interface to 3GPP system. As stated in TR 22.827, transfer interval refers to the periodicity of the packet transfers. It has to be constant during the whole operation time.

end-to-end latency: (see TS 22.261 [14]) the time that it takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination.

audio processing time: the time that it takes at application to perform all kinds of audio data processing such as, filtering, gain, encoding, decoding.

latency experienced by the user: the total one-way delay between capturing an analog audio signal and replaying it. It is the sum of all internal time delays introduced by all involved entities of the wireless system to transfer an analog signal from a source, e.g., a microphone, to a sink where the original signal is reproduced in an analog manner, e.g., at a loudspeaker. This latency does not refer to a single data packet but to all packets of a consecutive data stream with a constant periodicity, often called as real-time communication.

mouth-to-ear latency: (see TS 22.263) end-to-end maximum latency between the analogue input at the audio source (e.g. wireless microphone) and the analogue output at the audio sink (e.g. IEM). It includes audio application, application interfacing and the time delay introduced by the wireless transmission path.

real-time latency budget: the overall real-time latency budget is composed of different parts depending to which kind of latency it refers. Figure x.1.1-1 shows the components that build up the latency experienced by the user. During the time period of the transfer interval the analog audio signal is converted from analog to digital and split into frames of a constant size in samples. Then the audio data are further processed, e.g. audio encoding, and delivered to the wireless network. The network adds its specific end-2-end latency and after receiving the data, the application decodes the audio data and converted it from digital to analog to generate an analog signal that can be replayed. In case of unsynchronized operation between application and 3GPP network, additional time for buffering at the interfaces between the two must be included. When using IEMs the mouth-to-ear latency is the crucial parameter for the real-time latency budget.

Figure x.1.1-1: Latency experienced by the user

### In a live event or concert, immersive audio production aims to fully engage the audience by delivering three-dimensional soundscape experience. The three-dimensional listening experience can be enjoyed either on-site or remotely via a live audio stream. This is achieved by using audio devices that capture the sound coming from all directions and combine their outputs accordingly to create a three-dimensional audio stream. The production of immersive audio streams can be supported by two different types of devices:

- Type A: microphone array

  - contains at least four audio transducers,

  - delivers either an immersive audio output stream or the raw data of each transducer, and

  - has capabilities for localization.

- Type B: single microphone

  - has capabilities for localization,

  - includes a mechanism for tight synchronization, and

  - outputs a raw mono audio stream.

The type of capturing device used depends on the respective usage scenario.

In addition to the capturing devices, monitor devices are used in audio productions that replay the audio signals on loudspeakers, commonly known as stage monitor wedge, or individual in-ear monitor systems (IEM).

IEMs typically receives a binaural, stereo, or double-mono mix, while the wedge is fed by a mono mix. Stage monitor wedges are usually not wireless, as they generally have a built-in amplifier or are connected to an external close-by amplifier. IEMs are always based on wireless transmission.

## Synchronicity requirements

The devices deployed in live audio production scenarios may require support for time synchronization, in particular to:

- support alignment of user data from different audio sources (wireless microphones),

- minimize buffering when processing multiple isochronous data streams, and

- reduce latency by optimizing interfacing and audio data flow through all network entities, i.e., alignment of the processing of the audio data with the scheduled 3GPP network traffic.

Synchronicity is defined as the maximum allowed time offset between the synchronization leader and any network device synchronized with it specified in seconds.

The requirements for the alignment of the phase locked mono audio streams depend on the targeted output stream (see 5G-MAG report: “Time synchronization services for media production over 5G networks”):

- mono audio signal: 50 - 100µs,

- stereo audio signal: 10 - 50µs, or

- immersive audio signal: 1 - 10µs.

For immersive audio capturing with independent audio devices generating a mono audio stream such as wireless microphones, the phase difference between the different audio streams must be known to compensate for it later when generating the immersive audio stream, or it must be so small and constant that it has no impact on the immersive audio output stream.

If the phase difference is in any order and varies, it might be very difficult or nearly impossible to produce an immersive audio image of the previous captured scenario.

## Latency requirements

In general, latency of a data network depends on the data service, i.e., packet size and transfer interval / periodicity. Typical network performance tests use Ping measurements with 32-byte packets in random sequence without fixed transfer intervals. This might be valid to explore average network performance but does not reflect live audio applications where a streaming latency should be used to specify the overall system latency.

The overall system latency is made up of various components, including transfer interval, end-to-end latency, audio processing time, and sound propagation time in case of not using IEMs.

Latency in live monitoring scenarios can degrade the perceived quality of the audio and even affect the artist’s / user’s ability to perform. Different factors influence the effect of latency, such as the musical abilities of the musician, his critical listening skills, and the type of instrument played. In addition, the monitoring environment also affects the audibility of latency. Therefore, it does not exist a hard limit for the latency requirements but different limit ranges for different live monitoring scenarios and applications. The total tolerated latency varies from 4ms in case of live monitoring with IEM up to 40ms in case of live monitoring with wedge monitor.

### x.1.2	Pre-conditions

- The local operator or the person responsible for the production network provides a network that supports all needed requirements regarding latency, synchronicity, and reliability.

- All wireless microphones and IEMs on-site are switched on, registered, and connected to the 6G system.

- Audio processing units for e.g. creating IEM streams or immersive audio streams are connected to the 6G system.

### x.1.3	Service Flows

A typical immersive audio service flow in a live event is composed of several performers using immersive audio capturing devices (wireless microphones) and an audience experiencing the immersive audio content through loudspeakers installed at the venue or remotely through streaming platforms.

- The event location is equipped with sound capturing devices according to the producer’s instructions.

- Each sound capturing device supplies audio and position information to the 6G system.

- Audio processing units connected to the 6G system combine the incoming audio streams from the different capturing devices and generate required audio output streams, e.g. individual IEM streams, immersive streams for on-site sound reinforcement, or immersive streams for remote consumption.

- The generated audio streams are sent back to the respective sink.

- The audio stream is played back through the respective audio device.

### x.1.4	Post-conditions

The immersive audio production at a live event should operate as needed without any perceivable interruptions/audible impairments during the entire duration of the event.

### x.1.5	Existing features partly or fully covering the use case functionality

In TS 22.263 [67] the AV production use case is described in detail and its associated requirements are explained and defined. Here in clause 4.3, it is pointed out that “the 5G system should enable non-public networks that can be deployed in an agile ad-hoc way”. The related requirements of NPNs are described in TS 22.261 [14], clause 6.25.

Periodic deterministic communication required for audio streaming applications in live scenarios is already described in TS 22.104 [64], clause 5.2, and examples for the required transfer interval can also be found there as well as in TS 22.263 [67]. The QoS model to support low latency real-time communication is defined in TS 23.501 [140], clause 5.7. Standardized 5QI values are also specified that support a packet delay budget of 5ms with a guaranteed flow bit rate.

Synchronicity is a key parameter to combine individual mono audio streams and convert them into an immersive audio stream. In TS 22.263 [67] clock synchronization is already described but it refers to align video and audio streams only. In addition, TS 23.501 [140] describes features that can be used independently or in combination to enable time-sensitive communication, time synchronization and deterministic networking. To interact with the 5G system two types of translators are described: the Device-Side TSN Translator (DS-TT) on the UE side and the Network-Side TSN Translator (NW-TT) on the 5G network side.

### x.1.6	Potential New Requirements needed to support the use case

[PR x.1.6-1]	The 6G system shall provide time synchronization service.

[PR x.1.6-2]	Subject to user consent, the 6G system shall provide precise time information at UE side that can be used for applications running on the UE.

[PR x.1.6-1]	The 6G system shall be able to provide deterministic low-latency with the KPI requirements summarized below:

Table x.1.6-1: Performance requirements for immersive audio production in live events.

| Scenario | # of active UEs (Note 1) | Max. UE speed [km/h] | Max. service area [m²] | Synchro- nicity [µs] (Note 2) | Max. E2E latency [ms] (Note 3) | Positioning accuracy [m] (Note 4) | Min. packet error rate | User data rate UL [Mbit/s] (Note 5) | User data rate DL [Mbit/s] (Note 6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UE device type A: |  |  |  |  |  |  |  |  |  |
| large-scale event | 15 - 80 | 50 | 500 x 500 | 1 - 10 | 0.5 | 0.5 - 1 | 10-6 | 5 - 20 | 0.5 - 2 |
| small-scale event | 1 - 15 | 10 | 50 x 50 | 1 - 10 | 0.5 | 0.5 - 1 | 10-6 | 5 - 20 | 0.5 - 2 |
| UE device type B: |  |  |  |  |  |  |  |  |  |
| large-scale event | 50 - 300 | 50 | 500 x 500 | 1 - 10 | 0.5 | 0.5 - 1 | 10-6 | 1.2 - 2.5 | 0.5 - 2 |
| small-scale event | 4 - 50 | 10 | 50 x 50 | 1 - 10 | 0.5 | 0.5 - 1 | 10-6 | 1.2 - 2.5 | 0.5 - 2 |
| Note 1: the figures are estimated assuming a steady increase in the use of wireless mics based on the development of the last decades Note 2: according to 5G-MAG report on “Time synchronization services for media production over 5G networks” Note 3: more stringent values compared to TS 22.263 [67], Table 6.2.1-1 as immersive audio requires higher processing time for encoding and decoding Note 4: estimated range based on experience with current immersive audio productions Note 5: range from uncompressed audio with 24bit / 48kHz up to 24bit / 96kHz as for immersive audio uncompressed audio is preferred and there is no compression mechanism for immersive audio available at this point in time Note 6: range from compressed audio up to uncompressed audio with 16bit / 48kHz for the use in IEMs |  |  |  |  |  |  |  |  |  |

NOTE:	This table is derived from Table 6.2.1-1 of TS 22.263 [67] but specifies more stringent values for E2E latency and user data rates and specifies the additional KPIs synchronicity and position accuracy as this is required for immersive audio production.

Editor’s NOTE: Values in Table x.1.6-1 are FFS.