3GPP TSG-SA WG1 Meeting #111 	S1-253260r2

Goteborg, Sweden, 25-29 August 2025	(revision of S1-25xxxx)

Title:	ATSSS Enhancements

Agenda Item:	8.1.2 (System and Operation Aspects)

Source:	Lenovo

Contact:	Apostolis Salkintzis (y1026c at lenovo.com)

Abstract: This contribution proposes an enhancement of the multi-access connectivity capabilities for 6G systems. It addresses the limitations of the existing 5G ATSSS and defines new requirements for the 6G system to adopt a more efficient traffic steering strategy.

NOTE:    	This contribution has been partially supported by the HORIZON-JU-SNS-2023 EU project SUNRISE-6G (GA Number 101139257).

## 5.7 	6G enhancements of legacy/existing services and capabilities

Editor’s Note:  Text to be provided.  This clause to address potential (new) requirements for new (6G) aspects of existing (5G and earlier) services or capabilities.

* * * First Change (ALL NEW TEXT) * * *

## 5.7.x	Access Traffic Steering, Switching, and Splitting

### 5.7.x.1	Description

The 5G System supports the Access Traffic Steering, Switching, and Splitting (ATSSS) feature, which enables the UE and the network to establish a multi-access PDU session, using one 3GPP access and one non-3GPP access, and to distribute the traffic across these accesses using one or more steering modes, e.g., "Smallest Delay", "Load Balancing", "Redundant", etc. The multi-access rules created by the network for the multi-access PDU session are used to statically assign a traffic flow to a steering mode. For example:

-	A multi-access rule may assign the traffic of App-X to the "Smallest Delay" steering mode, meaning that this traffic should be sent to the access (3GPP or non-3GPP) with the smallest delay.

-	Another multi-access rule may assign the traffic of App-y to the "Load Balancing" steering mode, meaning that this traffic should be split across the 3GPP and the non-3GPP accesses.

However, this static assignment of a traffic flow to a single steering mode is a limitation for advanced 6G services where the performance requirements of an application can change dynamically during a session. Consider for example the following scenarios:

-	The traffic of an Extended Reality (XR) application can fluctuate dramatically on a small timescale. For example, a user turning their head quickly requires immediate transmission of new head-pose data (low data volume, extreme latency sensitivity) to trigger the rendering of a new viewpoint. The subsequent delivery of the rendered video frames requires high throughput but may have a slightly more relaxed latency budget. In this case, a static steering rule for the traffic of this application is incapable of optimizing for these rapidly changing, multi-modal traffic flows.

-	Similarly, an interactive video conference application may require low latency and low jitter during a conversational phase. However, during the same session, the user may begin sharing a high-resolution, full-motion video, which requires high throughput. Thus, mapping the traffic of this application to a single steering mode cannot provide optimal performance.

From the above, it becomes clear that there is a need for more flexible traffic steering strategies in 6G, which can adapt to the dynamic application requirements and optimize the efficiency of multi-access connectivity using a 3GPP access and a non-3GPP access. A fixed steering mode cannot provide this level of adaptability, leading to bad user experience.

### 5.7.x.2	Potential New Requirements

[PR 5.7.x.2-1] The 6G system shall support multi-access connectivity between a 3GPP access and a non-3GPP access.

[PR 5.7.x.2-2] The 6G system shall be able to identify the different traffic flows of an application and steer each traffic flow to a 3GPP access and/or a non-3GPP access, enabling each traffic flow to be handled according to its specific performance requirements. Only one 3GPP access shall be used to carry one or more traffic flows.

NOTE 1:	For example, an XR application could have a head-tracking traffic flow that requires ultra-low latency and a rendered video traffic flow that requires high throughput. These traffic flows are dynamically created during the application runtime.

* * * End of Change * * *