3GPP TSG-SA WG1 Meeting #112 	S1-254180

17-21 November 2025, Dallas, USA	(revision of S1-xxxx)

Title:	Discussion on CPRs for Computing (Clause Y.1.9)

Agenda Item:	8.1.10

Source:	CATT

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution discusses the possible consolidation of potential requirements relevant to computing for Clause Y.1.9.

1. Introduction

Accrording to the guidance of S1-254020, all the potential requirements related to “computing” will be consolidated into one clause Y.1.9 Computing, that may include around 41 PRs identified from Clause 6 AI, Clause 9 Immersive Communication, Clause 11 Further Use Cases on Industry and Verticals, and Clause W Other Use Cases.

From the service perspective, the potential functional requirements for computing can be categorised into 2 parts:

- 6G computing service relevant, including

  - PR 9.6.6-1/2 from 9.6 use case on collaborative service in multi-site involved immersive communication

  - PR 11.22.6-1/2 from 11.22 use case on spatial computing enabled dynamic material management

  - PR W.1.6-1/2 from W.1 use case on use case on computing service for XR game acceleration

  - PR W.2.6-1/2/3 from W.2 use case on Computing Service enabling personal AI agent

  - PR W.3.6-1/2 from W.3 use case on computing service in Operator managed data network

- Coordinate computing resources to support other network services(e.g. AI services, communication services), including

[Support AI services in AI model training, inference, GenAI, etc]

- PR6.2.6-1/3/4 from 6.2 use case on optimizing 6G infrastructure utilization via resource exposure in 6G

- PR6.3.6-1 from 6.3 use case on end-to-end AI for connected cars

- PR6.5.6-1/2/3 from 6.5 use case on personalized AI for health monitoring

- PR6.10.6-4 from 6.10 use case on home robots

- PR6.14.6-3 from 6.14 use case on Intelligent UAV Swarms

- PR6.15.6-1/2 from 6.15 use case on 6GS assisted target object detection

- PR6.19.6.6-1 from 6.19 use case on AI-based video analysis

- PR6.24.6-2 from 6.24 use case on distributed 6G network for AI computing

- PR6.33.6-1/2 from 6.33 use case on AI text-to-video generation supported by computing

- PR6.34.6-1/2/3 from 6.34 use case on 6G computing support for AI model inference

- PR6.37.6-3 from 6.36 use case on energy efficiency for AI service

- PR6.53.6-1/2 from 6.53 use case on AI-driven smart factory with computing service

[Support communication services in XR rendering]

- PR 9.4.6-1 from 9.4 use case on XR rendering offload support

- PR 9.3.6-2 from 9.3 use case on multi-media services with deterministic experience via collaborative processing among UE-network-cloud

- PR 9.5.6-1/2 from 9.5 use case on seamless immersive reality in education

- PR 9.15.6-1/2/3/4/5/6 from 9.15 use case on coordinating computing and communication for XR rendering

Besides, there are 5 PRs relevant to satellite onboard computing, discussed in S1-254120 now but open to be moved to this clause.

- PR 8.9.6-1/3/4 from 8.9 use case on low-altitude logistics supported by NTN

- PR 8.15.6-1/2 from 8.15 use case on coordinating computing and communication for XR rendering

2. Discussion

Based on the grouping proposal from S1-254020, the PR groups and proposed CPRs have been updated with markups as below.

Table Y.1.9-1 – Computing

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.9-1-1 | The 6G network shall be able to configure, manage and provide computing resource in Service Hosting Environments for 3GPP services (e.g.6G computing service, AI services) | PR 6.3.6-1 PR 11.22.6-1 PR 6.10.6-4 | General, resource |
|  | The 6G network shall be able to provide computing resource in the Service Hosting Environment for AI services. | PR 6.3.6-1 |  |
|  | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall provide a mechanism for operators to configure, adjust and manage the computing resource in Service Hosting Environment based on e.g. UE location, computing service requirements from 3rd party. | PR 11.22.6-1 | Resource management |
|  | Subject to operator policy and user consent, the 6G network shall support mechanisms for scheduling and provisioning network resources and computing resources in Service Host Environment for distributed AI inference service. | PR 6.10.6-4 | Scheduling and provisioning |
| CPR Y.1.9-1-2 | Subject to user consent, the 6G system shall support mechanisms to execute computing tasks in the Service Hosting Environment upon service request from UEs. | PR 6.5.6-1 | General, execute |
| CPR Y.1.9-1-3 | Subject to operator’s policy and user consent, the 6G system shall support a mechanism for providing 6G Computing Service to user (via UE). | PR W.2.6-1 | General, computing service to UE |
| CPR Y.1.9-1-4 | Subject to operator policy, privacy considerations and regulatory requirements, the 6G network shall be able to offload computing tasks to a Service Hosting Environment for third party applications (e.g. rendering, AI tasks, etc.). | PR 9.4.6-1 PR 9.5.6-2 | Proposed merged CPR on General, offloading |
|  | Subject to privacy considerations and regulatory requirements, the 6G network shall be able to enable compute offloading to a Service Hosting Environment for third party applications. | PR 9.4.6-1 | General, offloading |
|  | Subject to operator policy, the 6G network (e.g. core network) shall enable the offloading of computing tasks to the Service Hosting Environment to render 6DoF video and spatial audio to a 3rd party. | PR 9.5.6-2 | General, offloading |
|  |  |  | AI offloading Belong to AI service, Move to Y.1.8 |
| CPR Y.1.9-1-5 | Subject to operator’s policy and regulation, the 6G network shall be able to provide 6G Computing Service controlled by the core network of 6G system to an authorized third-party e.g. for supporting rendering. | PR 9.5.6-1 PR 9.6.6-1 | Proposed merged CPR on General, computing service to 3rd party rendering |
|  | Subject to operator policy, the 6G network (e.g. core network) shall enable the support of computing tasks in the Service Hosting Environment to render 6DoF video and spatial audio. | PR 9.5.6-1 | General, rendering |
|  | Subject to operator’s policy and regulation, the 6G network shall be able to provide 6G Computing Service controlled by the core network of 6G system to an authorized third-party for supporting rendering. | PR 9.6.6-1 | General, rendering |
| CPR Y.1.9-1-6 | Subject to operator policy and user consent, the 6G system shall support mechanisms for a UE to specify requirements for a 6G computing service in the Service Hosting Environment, (e.g. latency, when to execute tasks, etc) | PR 6.5.6-2 PR 6.5.6-3 PR 6.33.6-1 | Proposed merged CPR on Requirements From UE |
|  | Subject to user consent, the 6G system shall support mechanisms that enable a UE to specify when a compute task should be executed (e.g. immediately, periodically, etc.) in the Service Hosting Environment. | PR 6.5.6-2 | Requirements |
|  | Subject to user consent, the 6G system shall support mechanisms that enable a UE to specify requirements for a compute task in the Service Hosting Environment, such as an overall latency indicating how fast the results of the compute task should be provided. | PR 6.5.6-3 | Requirements |
|  | The 6G network shall support the capability to receive the requested computing requirements of 6G Computing Service. | PR 6.33.6-1 | Requirements Split to from UE, from AF, and merge to separate CPR. |
| CPR Y.1.9-1-7 | Subject to operator policy and user consent, the 6G system shall support mechanism to obtain the requirements (e.g. latency) of 6G Computing Service from 3rd party and allocate associated computing and communication resources of the Service Hosting Environment for providing the subscribed 6G Computing Service. | PR W.2.6-3 PR 6.33.6-1 | Requirements From 3rd party |
|  |  |  | Requirements How to schedule AI service Move to Y.1.8 AI |
| CPR Y.1.9-1-8 | The 6G network shall support mechanisms to monitor and report the status (e.g. workload, congestion, available capability, power consumption) of trusted computing resources in the Service Hosting Environment on-demand or periodically. | PR 6.2.6-3 PR 9.15.6-2 | Proposed merged CPR on Status of computing resources |
|  | The 6G network shall support detailed monitoring and reporting of computational resource usage in the Service Hosting Environment. | PR 6.2.6-3 | Monitor and report, Status of computing resources |
|  | The 6G network shall support mechanisms to collect status information (e.g. computing workload, congestion information, available capability, power consumption) of trusted computing resources (i.e. edge server(s) or cloud server(s)) on-demand or periodically. | PR 9.15.6-2 | Collect Status of computing resources |
| CPR Y.1.9-1-9 | The 6G network shall support mechanisms to acquire and maintain the information of trusted computing resources with Service Hosting Environment or 3rd party computing environment for coordinating the usage of computing resource on demand NOTE: The information can be related to computing capabilities (e.g. xPUs, storage), service capabilities (e.g. supported applications, status), and network capabilities (e.g. allowed bandwidth). | PR 9.15.6-1 | computing resources management |
| CPR Y.1.9-1-10 | Subject to operator’s policy, the 6G network shall provide mechanisms to expose to an authorised 3rd party the information (e.g. computing capability, the location, allowed service types, status, power consumption and utilization) related to computing resources in Service Hosting Environment or 6G Computing Service . | PR 6.2.6-1 PR 9.15.6-3 PR W.3.6-2 | Proposed merged CPR on Exposure Exposure CPRs need to be split to two, for NW-AF and NW-UE individually. |
|  | Subject to operator’s policy and user’s consent, 6G network shall provide mechanism to expose information related to 6G Computing Service to an authorised 3rd party (e.g. to track usage patterns of the computing service for offline AI analysis). | PR W.3.6-2 | Exposure |
|  | Based on operator policy, the 6G network shall provide suitable means to allow authorized third parties and/or UEs to retrieve availability information about computational resources (e.g. storage, AI processing units, xPUs information etc.) inside the Service Hosting Environment and to utilize these computational resources for running workloads on demand. | PR 6.2.6-1 | Exposure Missing in S1-254120 |
|  | The 6G network shall provide mechanisms to expose to an authorised 3rd party the information (e.g. computing capability, the location, allowed service types, status, power consumption, utilization) of computing resources (i.e. edge server(s) or cloud server(s)). | PR 9.15.6-3 | Exposure Missing in S1-254120 |
| CPR Y.1.9-1-11 | The 6G network shall be able to inform UE of the information related to 6G Computing Service or computing resources in the Service Hosting Environment (e.g. guaranteed computing capabilities) to assist an UE to utilize a computing service or the computing resources. | PR 6.2.6-1 PR W.1.6-2 | Proposed merged CPR on Exposure Information exchange between NW and UE |
|  | The 6G network shall be able to inform UE of the information related to 6G Computing Service in the Service Hosting Environment (e.g. guaranteed computing capabilities) to assist an UE to invoke a computing service. | PR W.1.6-2 | Inform UE |
| CPR Y.1.9-1-12 | Subject to operator’s policy, 6G network shall support mechanism to allow sharing of information related to 6G Computing Service within the Service Hosting Environment (e.g. to predict overprovisioning and underutilization of computing resources). | PR W.3.6-1 | Info sharing within network |
| CPR Y.1.9-1-13 | Subject to operator’s policy and agreement with 3rd party, the 6G system shall support mechanisms to select computing resources in one or more Service Hosting Environments for 3GPP services(e.g. 6G computing service, AI service) to fulfil the computing requirements from 3rd party or to meet service requirements (e.g. bitrate and latency), and support routing of data traffic between a UE and the selected computing resource(s) based on required communication service QoS. | PR 6.14.6-3 PR 6.15.6-1 PR 6.34.6-2 PR 6.37.6-3 PR 6.53.6-1 PR 11.22.6-2 PR 9.15.6-4 PR 6.24.6-2 | Proposed merged CPR on Selection |
|  | Subject to the operator’s policy, the 6G network shall support the selection of computing resources in the Service Hosting Environment upon request from AI-based applications. | PR 6.14.6-3 | Selection |
|  | Subject to operator policy, 6G network shall be able to support selection of computing resources in Service Hosting Environment for 6G Computing Service. | PR 6.15.6-1 | Selection |
|  | Subject to operator policy, the 6G network shall support selection of computing resources in the Service Hosting Environment for offloading task. | PR 6.34.6-2 | Selection |
|  | Based on operator's policy and agreement with 3rd party, the 6G network shall support a mechanism to assist in selecting computing resources inside Service Hosting Environment for AI service considering requirement of the 3rd party. | PR 6.37.6-3 | Selection |
|  | Subject to operator policy and regulatory requirement, the 6G network shall be able to support the selection of multiple Service Hosting Environments for 3GPP services and 3rd party services. | PR 6.53.6-1 | Selection |
|  | Subject to operator’s policy and agreement with the 3rd party, the 6G system shall support to select one or more Service Hosting Environment to fulfil the computing service requirements from 3rd party, considering the UE location, computing capabilities and traffic load, etc. | PR 11.22.6-2 | Selection |
|  | Subject to operator's policy, application needs or both, the 6G network shall support the selection of computing resource(s) (i.e. edge server(s) or cloud server(s)) based on e.g. requested computing capabilities, and support routing of data traffic between a UE and the selected computing resource(s) based on required communication service QoS. | PR 9.15.6-4 | Selection, Routing between UE-computing resources |
|  | The 6G network shall be capable of providing appropriate Service Hosting Environment in order to accommodate compute and communication (e.g. traffic load) resources to meet service requirements (e.g. bitrate and latency). | PR 6.24.6-2 | SHE selection |
| CPR Y.1.9-1-14 | Subject to operator policy and regulatory requirement, the 6G network shall be able to support the coordination amongst multiple Service Hosting Environments for 3GPP services and 3rd party services. | PR 6.53.6-2 | Coordination of multi-SHE |
| CPR Y.1.9-1-15 | Subject to user consent, the core network of the 6G system shall provide means to coordinate with a UE to make decisions on splitting/offloading rendering tasks dynamically based on the real time network status and the computing resource availability in the Service Hosting Environment, edge or central cloud. NOTE 1: In the case of tethered XR headset connected to a UE, the latency, computing resource consumption and power consumption (incurred by application processing and communication processing) of both the UE and the XR Glasses need to be also considered. NOTE 2: It is assumed that the core network is able to obtain the information of computing resource availability in the Service Hosting Environment, edge or central cloud, which enables the core network to locate appropriate computing resources for the rendering tasks. | PR 9.3.6-2 | Coordination with UE, immersive |
| CPR Y.1.9-1-16 | Subject to operator policy, 6G system shall be able to guarantee the overall E2E latency (i.e. latency of communication service, and latency of 6G Computing Service) requested by the application (e.g. target object detection). NOTE: UE is only considered to contribute to the communication service latency. | PR 6.15.6-2 | Performance for joint computing service and communication service. |
| CPR Y.1.9-1-17 | Subject to operator’s policy, the 6G network shall be able to fulfil the user-requested performance requirement (e.g. latency) for the 6G Computing Service. | PR W.1.6-1 | Performance for computing service |
|  | The 6G network shall be capable of providing appropriate Service Hosting Environment in order to accommodate compute and communication (e.g. traffic load) resources to meet service requirements (e.g. bitrate and latency). | PR 6.24.6-2 | SHE selection merged to Y.1.9-1-13 |
|  | Subject to operator’s policy, the 6G network shall be able to fulfil the user-requested performance requirement (e.g. latency) for the 6G Computing Service. | PR W.1.6-1 | Performance |
| CPR Y.1.9-1-18 | Subject to operator’s policy, the 6G network shall support efficient means to transfer a computing task from one computing resource to another computing resource in the Service Hosting Environment at the same or different location due to, e.g. UE mobility or performance assurance. | PR 6.2.6-4 PR 9.15.6-6 | Proposed merged CPR on Performance, Service continuity |
|  | Based on operator policy, the 6G network shall support efficient ways to transfer a computational task from one computational resource of the Service Hosting Environment at one location to another computational resource of the Service Hosting Environment at another location, based on the mobility of UEs and the desired performance requirements. | PR 6.2.6-4 | Performance, Service continuity |
|  | Subject to operator's policy, the 6G network shall be able to support migration of ongoing computing task from one selected computing resource (e.g. overloaded) to other computing resource(s) in Service Hosting Environments in order to maintain user experience. | PR 9.15.6-6 | Performance, Service continuity Computing data transfer, merged to Y.1.9-1-18 |
| CPR Y.1.9-1-19 | Subject to operator's policy, the 6G network shall support reselection of computing resource(s) in the Service Hosting Environment to maintain service continuity under UE mobility. | PR 6.19.6-1 PR 6.33.6-2 | Proposed merged CPR on reselection, Service continuity |
|  | The 6G network shall support reselection of computing resources in Service Hosting Environment for third party application workload offloading from the UE to Service Hosting Environment (e.g. where AI inference is performed based on video input from UE), considering UE mobility. | PR 6.19.6-1 | reselection, Service continuity |
|  | Subject to operator's policy, application needs or both, the 6G network shall support the reselection of computing resource(s) inside Service Hosting Environment and support the continuity due to, e.g.: UE mobility. | PR 6.33.6-2 | reselection |
| CPR Y.1.9-1-20 | Subject to operator's policy, the 6G network shall be able to maintain user experience (e.g. for immersive communication) with minimum interruption when the selected computing resources changes within the Service Hosting Environment or between the Service Hosting Environment and 3rd party computing environment. | PR 9.6.6-2 PR 9.15.6-5 | Proposed merged CPR on Performance, Service continuity |
|  |  |  | Performance, Service continuity Computing data transfer, merged to Y.1.9-1-18 |
|  | Subject to operator’s policy, the 6G network shall be able to maintain user experience (e.g. for immersive communication) with minimum interruption when the selected computing resources changes within the Service Hosting Environment used for the 6G Computing Service for XR rendering e.g. during UE mobility. | PR 9.6.6-2 | Performance, Service continuity |
|  | Subject to operator’s policy, the 6G network shall be able to maintain user experience (e.g. for immersive communication) with minimum interruption when the selected computing resources switch between those within the Service Hosting Environment and those in 3rd party computing environment. | PR 9.15.6-5 | Performance, Service continuity |
| P CPR Y.1.9-1-21 | Subject to operator policy and user consent, the 6G network shall be able to prioritize the processing of offloading task request(s) from a high-priority user per service. NOTE: the prioritization of processing of computing is with more stringent requirement from high-priority user | PR 6.34.6-3 | Prioritize the processing of computing task |
|  |  |  | Resource management Merge to Y.1.9-1-1, general |
| P CPR Y.1.9-1-22 | Subject to operator’s policy, the 6G system shall support mechanisms to ensure security and privacy when providing 6G Computing Service. | PR W.2.6-2 | Security |
|  |  |  | Energy consumption SHE is logic domain, can provide different services, not limited to computing. Move to Y.1.4 |
| P CPR Y.1.9-1-23 | Subject to operator policy and user consent, the 6G network shall be able to authorize a user to offload task from a 3rd party application (e.g. an AI inference workload) to the Service Hosting Environment. | PR 6.34.6-1 | Security Authorization |

3. Proposal

It’s proposed to agree the following consolidated requirements based on the above discussion.

Y	Consolidated potential functional and performance requirements

Editor’s Note: PRs with associated Editor’s Note(s) will not be considered for consolidation until Editor’s Note(s) associated with the PR are resolved.

Y.1	Consolidated potential Functional Requirements

Y.1.9	Computing

Editor’s Note: This includes (but not limited to) PRs from clause 6, 9, 11, W.

Table Y.1.9-1 – Computing

| CPR # | Consolidated Potential Requirement | Original PR # | Comment |
| --- | --- | --- | --- |
| CPR Y.1.9-1-1 | The 6G network shall be able to configure, manage and provide computing resource in Service Hosting Environments for 3GPP services (e.g.6G computing service, AI services) | PR 6.3.6-1 PR 6.10.6-4 PR 11.22.6-1 | General, resource |
| CPR Y.1.9-1-2 | Subject to user consent, the 6G system shall support mechanisms to execute computing tasks in the Service Hosting Environment upon service request from UEs. | PR 6.5.6-1 | General, execute |
| CPR Y.1.9-1-3 | Subject to operator’s policy and user consent, the 6G system shall support a mechanism for providing 6G Computing Service to user (via UE). | PR W.2.6-1 | General, computing service to UE |
| CPR Y.1.9-1-4 | Subject to operator policy, privacy considerations and regulatory requirements, the 6G network shall be able to offload computing tasks to a Service Hosting Environment for third party applications (e.g. rendering, AI tasks, etc.). | PR 9.4.6-1 PR 9.5.6-2 | General, offloading |
| CPR Y.1.9-1-5 | Subject to operator’s policy and regulation, the 6G network shall be able to provide 6G Computing Service controlled by the core network of 6G system to an authorized third-party e.g. for supporting rendering. | PR 9.5.6-1 PR 9.6.6-1 | General, computing service to 3rd party rendering |
| CPR Y.1.9-1-6 | Subject to operator policy and user consent, the 6G system shall support mechanisms for a UE to specify requirements for a 6G computing service in the Service Hosting Environment, (e.g. latency, when to execute tasks, etc) | PR 6.5.6-2 PR 6.5.6-3 PR 6.33.6-1 | Requirements From UE |
| CPR Y.1.9-1-7 | Subject to operator policy and user consent, the 6G system shall support mechanism to obtain the requirements (e.g. latency) of 6G Computing Service from 3rd party and allocate associated computing and communication resources of the Service Hosting Environment for providing the subscribed 6G Computing Service. | PR W.2.6-3 PR 6.33.6-1 | Requirements From 3rd party |
| CPR Y.1.9-1-8 | The 6G network shall support mechanisms to monitor and report the status (e.g. workload, congestion, available capability, power consumption) of trusted computing resources in the Service Hosting Environment on-demand or periodically. | PR 6.2.6-3 PR 9.15.6-2 | Status of computing resources Monitor, report |
| CPR Y.1.9-1-9 | The 6G network shall support mechanisms to acquire and maintain the information of trusted computing resources with Service Hosting Environment or 3rd party computing environment for coordinating the usage of computing resource on demand NOTE: The information can be related to computing capabilities (e.g. xPUs, storage), service capabilities (e.g. supported applications, status), and network capabilities (e.g. allowed bandwidth). | PR 9.15.6-1 | computing resources management |
| CPR Y.1.9-1-10 | Subject to operator’s policy, the 6G network shall provide mechanisms to expose to an authorised 3rd party the information (e.g. computing capability, the location, allowed service types, status, power consumption and utilization) related to computing resources in Service Hosting Environment or 6G Computing Service . | PR 6.2.6-1 PR 9.15.6-3 PR W.3.6-2 | Exposure NW-AF |
| CPR Y.1.9-1-11 | The 6G network shall be able to inform UE of the information related to 6G Computing Service or computing resources in the Service Hosting Environment (e.g. guaranteed computing capabilities) to assist an UE to utilize a computing service or the computing resources. | PR 6.2.6-1 PR W.1.6-2 | Exposure Information exchange between NW and UE |
| CPR Y.1.9-1-12 | Subject to operator’s policy, 6G network shall support mechanism to allow sharing of information related to 6G Computing Service within the Service Hosting Environment (e.g. to predict overprovisioning and underutilization of computing resources). | PR W.3.6-1 | Info sharing within network |
| CPR Y.1.9-1-13 | Subject to operator’s policy and agreement with 3rd party, the 6G system shall support mechanisms to select computing resources in one or more Service Hosting Environments for 3GPP services(e.g. 6G computing service, AI service) to fulfil the computing requirements from 3rd party or to meet service requirements (e.g. bitrate and latency), and support routing of data traffic between a UE and the selected computing resource(s) based on required communication service QoS. | PR 6.14.6-3 PR 6.15.6-1 PR 6.34.6-2 PR 6.37.6-3 PR 6.53.6-1 PR 11.22.6-2 PR 9.15.6-4 PR 6.24.6-2 | Selection |
| CPR Y.1.9-1-14 | Subject to operator policy and regulatory requirement, the 6G network shall be able to support the coordination amongst multiple Service Hosting Environments for 3GPP services and 3rd party services. | PR 6.53.6-2 | Coordination of multi-SHE |
| CPR Y.1.9-1-15 | Subject to user consent, the core network of the 6G system shall provide means to coordinate with a UE to make decisions on splitting/offloading rendering tasks dynamically based on the real time network status and the computing resource availability in the Service Hosting Environment, edge or central cloud. NOTE 1: In the case of tethered XR headset connected to a UE, the latency, computing resource consumption and power consumption (incurred by application processing and communication processing) of both the UE and the XR Glasses need to be also considered. NOTE 2: It is assumed that the core network is able to obtain the information of computing resource availability in the Service Hosting Environment, edge or central cloud, which enables the core network to locate appropriate computing resources for the rendering tasks. | PR 9.3.6-2 | Coordination with UE, immersive |
| CPR Y.1.9-1-16 | Subject to operator policy, 6G system shall be able to guarantee the overall E2E latency (i.e. latency of communication service, and latency of 6G Computing Service) requested by the application (e.g. target object detection). NOTE: UE is only considered to contribute to the communication service latency. | PR 6.15.6-2 | Performance for joint computing service and communication service. |
| CPR Y.1.9-1-17 | Subject to operator’s policy, the 6G network shall be able to fulfil the user-requested performance requirement (e.g. latency) for the 6G Computing Service. | PR W.1.6-1 | Performance for computing service |
| CPR Y.1.9-1-18 | Subject to operator’s policy, the 6G network shall support efficient means to transfer a computing task from one computing resource to another computing resource in the Service Hosting Environment at the same or different location due to, e.g. UE mobility or performance assurance. | PR 6.2.6-4 PR 9.15.6-6 | Migration, Service continuity |
| CPR Y.1.9-1-19 | Subject to operator's policy, the 6G network shall support reselection of computing resource(s) in the Service Hosting Environment to maintain service continuity under UE mobility. | PR 6.19.6-1 PR 6.33.6-2 | Reselection, Service continuity |
| CPR Y.1.9-1-20 | Subject to operator's policy, the 6G network shall be able to maintain user experience (e.g. for immersive communication) with minimum interruption when the selected computing resources changes within the Service Hosting Environment or between the Service Hosting Environment and 3rd party computing environment. | PR 9.6.6-2 PR 9.15.6-5 | Performance, Service continuity |
| P CPR Y.1.9-1-21 | Subject to operator policy and user consent, the 6G network shall be able to prioritize the processing of offloading task request(s) from a high-priority user per service. NOTE: the prioritization of processing of computing is with more stringent requirement from high-priority user | PR 6.34.6-3 | Prioritize the processing of computing task |
| PCPR Y.1.9-1-22 | Subject to operator’s policy, the 6G system shall support mechanisms to ensure security and privacy when providing 6G Computing Service. | PR W.2.6-2 | Security |
| P CPR Y.1.9-1-23 | Subject to operator policy and user consent, the 6G network shall be able to authorize a user to offload task from a 3rd party application (e.g. an AI inference workload) to the Service Hosting Environment. | PR 6.34.6-1 | Security Authorization |