3GPP TSG-SA WG1 Meeting #112 	S1-254228

17-21 November 2025, Dallas, USA	(revision of S1-252662, S1-252330)

Title:	Use case on 3D indoor positioning

Agenda Item:	8.1.9.2

Source:	Huawei

Contact:	Eddy Hall

edward.hall@huawei.com

Abstract: In this document, we provide a new use case on 3D indoor positioning.

## --- FIRST CHANGE ---

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor's Note: all References numbers to be corrected, missing references to be added

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	QIAN LI et all: "6G Cloud-Native System: Vision, Challenges, Architecture Framework and Enabling Technologies" in IEEE Access, Received 19 August 2022, accepted 1 September 2022, date of publication 8 September 2022, date of current version 19 September 2022.

[3]	GSMA: "Going green: benchmarking the energy efficiency of mobile networks (Second edition)", Feb 2023.

[4] 	3GPP TR 32.972: "Study on system and functional aspects of energy efficiency in 5G networks".

[5]	3GPP TR 23.700-66: "Study on Energy Efficiency and Energy Saving".

[6] 	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

[7]	5GAA: "5GAA MRP input to 3GPP Stage 1 workshop on IMT 2030 Use Cases".

[8]	5GAA: Technical report "Evolution of vehicular communication systems beyond 5G".

[9] 	3GPP TR 22.837: "Feasibility Study on Integrated Sensing and Communication".

[10]	SAE J3016 202104: "Levels of Driving Automation".

[11]	G. Liu, L. Ma, Y. Xue, et al, "SensCAP: A Systematic Sensing Capability Performance Metric for 6G ISAC," IEEE Internet of Things Journal, vol. 11, no. 18, pp. 29438–29454, 2024.

[12]	Igal Bilik, Oren Longman, Shahar Villeval, and Joseph Tabrikian, "The Rise of Radar for Autonomous Vehicles", IEEE signal processing magazine, Volume 36, Issue 5, September 2019.

[13] 	https://knowledge-base.inspire.ec.europa.eu/publications/technical-guidance-implementation-inspire-dataset-and-service-metadata-based-isots-191392007_en.

[14]	3GPP TS 22.261: "Service requirements for the 5G system".

[15] 	Wang et.al, "Effect of Frame Rate on User Experience, Performance, and Simulator Sickness in Virtual Reality", IEEE Transaction on Visualization and Computer Graphics. 2023 Feb 22:PP. doi: 10.1109/TVCG.2023.3247057.

[16] 	https://en.wikipedia.org/wiki/Adreno.

[17] 	https://www.labtekno.com/snapdragon-8-gen-2-vs-apple-a16-bionic/.

[18] 	Algorithms and tools for GPU-accelerated Computer Generated Holography | Ewireless Research Group (https://www.ewireless.eng.ed.ac.uk/projects).

[19] 	https://phlearn.com/tutorial/how-many-exposures-hdr/.

[20]	https://photographylife.com/compressed-vs-uncompressed-vs-lossless-compressed-raw.

[21]	https://www.dotphoton.com/products/jetraw-core.

[22] 	https://www.lambdatest.com/learning-hub/web-performance-testing.

[23] 	Static 3D triangle mesh compression overview, Marcos Aviles, Grupo de Tratamiento de Imágenes, Universidad Politécnica de Madrid, Spain, 15th IEEE International Conference on Image Processing.

[24] 	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[25]	EUSPA: "Report on Aviation and Drones – User Needs and Requirements", 2023.

[26]	3GPP TR 36.763: "Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN)".

[27]	Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond".

[28]	3GPP TS 22.156: "Mobile Metaverse Services".

[29] 	ITU-R Recommendation M.2160-0 (11/2023): "Framework and overall objectives of the future development of IMT for 2030 and beyond" (https://www.itu.int/rec/R-REC-M.2160-0-202311-I/en).

[30] 	3GPP TS 23.502: "Procedures for the 5G System".

[31]	Hexa-X-II, "Deliverable D1.2: 6G Use Cases and Requirements", December 2023.

[32]	https://www.easa.europa.eu/en/what-is-uam.

[33]	https://www.autoflight.com/en/air/.

[34]	"Low altitude economy development white paper (2.0) All digital solutions", International digital economy academy (IDEA), 2023.

[35]	3GPP TS 22.125: "Uncrewed Aerial System (UAS) support in 3GPP; Stage 1".

[36]	https://www.unmannedairspace.info/latest-news-and-information/shenzhen-reaches-record-number-of-drone-operations-with-600000-flights-in-2023/.

[37]	"C-V2X Use Cases and Service Level Requirements Volume II", 5GAA White Paper, 2023.

[38]	https://www.flyeye.io/drone-acronym-daa/.

[39]	Report ITU-R M.2516-0, "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond ". (https://www.itu.int/dms_pub/itu-r/opb/rep/R-REP-M.2516-2022-PDF-E.pdf).

[40]				3GPP TR 33.841: "Study on the support of 256-bit algorithms for 5G".

[41]	3GPP TS 35.246: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 1: algorithm specification".

[42]	3GPP TS 35.247: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 2: implementation test data".

[43]	3GPP TS 35.248: "Specification of the ZUC based 256-bits algorithm set: Specification of the 256-NEA6 encryption, the 256-NIA6 integrity, and the 256-NCA6 authenticated encryption algorithm for 5G; Document 3: design conformance test data".

[44]	3GPP TR 22.883: "Feasibility Study on Energy Efficiency as service criteria Phase 2".

[45]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[46]	3GPP TS 38.211: "NR; Physical channels and modulation".

[47]	3GPP TS 36.305: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[48]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[49]	3GPP TS 22.856: "Study on Localized Mobile Metaverse Services".

[50]	3GPP TR 26.928: "Extended Reality (XR) in 5G".

[51]	3GPP TS 26.565: "Split Rendering Media Service Enabler".

[52]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[53]	3GPP TS 22.179: “Mission Critical Push to Talk (MCPTT); Stage 1”.

[54]	3GPP TS 22.280: “Mission Critical Services Common Requirements (MCCoRe); Stage 1”..

[55]	3GPP TS 22.281: “Mission Critical (MC) video”.

[56]	3GPP TS 22.282: “Mission Critical (MC) data”.

[57]	3GPP TS 22.262: “Message service within the 5G System (5GS); Stage 1”.

[58]	3GPP TS 22.101: “Service aspects; Service principles”.

[59]	3GPP TS 22.173: “IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1”.

[60]	3GPP TS 22.011: “Service accessibility”.

[61]	3GPP TS 22.071: “Location Services (LCS); Service description; Stage 1”.

[62]	3GPP TS 22.268: “Public Warning System (PWS) requirements”.

[63]	3GPP TS 22.153: “Multimedia priority service”.

[64]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains”.

[65]	3GPP TS 22.185: “Service requirements for V2X services”.

[66]	3GPP TS 22.186: “Service requirements for enhanced V2X scenarios”.

[67]	3GPP TS 22.263: “Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)”.

[68]	3GPP TS 22.115: “Service aspects; Charging and billing”.

[69]	3GPP TS 22.289: “Mobile communication system for railways”.

[70]	3GPP TS 22.468: “Group Communication System Enablers (GCSE)”.

[71]	3GPP TS 22.368: “Service requirements for Machine-Type Communications (MTC); Stage 1”.

[72]	3GPP TR 22.949: “Study on a generalized privacy capability”.

[73]	3GPP TR 33.849: “Study on subscriber privacy impact in 3GPP”.

[74]	3GPP TS 22.278: “Service requirements for the Evolved Packet System (EPS)”.

[75]	ETSI GR SAI 004 (V1.1.1): "Securing Artificial Intelligence (SAI); Problem Statement".

[76]	ETSI GR SAI 005 (V1.1.1): "Securing Artificial Intelligence (SAI); Mitigation Strategy Report".

[77]	3GPP TR 28.915: "Study on management aspects of Network Digital Twin".

[78]	3GPP TS 23.527: “5G System; Restoration procedures”.

[79]	3GPP TR 29.866: “Study on IMS Disaster Prevention and Restoration Enhancement”.

[80]	Enabling Mobile AI Agent in 6G Era: Architecture and Key Technologies; https://dl.acm.org/doi/abs/10.1109/MNET.2024.3422309

[81]	Wang L, Ma C, Feng X, et al. A survey on large language model based autonomous agents[J]. Frontiers of Computer Science, 2024, 18(6): 186345.

[82]	Hong S, Zheng X, Chen J, et al. Metagpt: Meta programming for multi-agent collaborative framework[J]. arXiv preprint arXiv:2308.00352, 2023.[xx]	ISO/IEC 22989: "Information technology — Artificial intelligence — Artificial intelligence concepts and terminology"

[83] 	GSMA Foundry, “Case Study: Delivering Real-Time Translation”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/5g-new-calling/

[84] 	GSMA Foundry, “Case Study: Remote damage assessment ”,2023, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/remote-damage-assessment/

[85]	 GSMA Foundry, “Case Study: Visualised Video-Calling”,2023, Reference links for  https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/visualised-voice-calling/

[86] 	GSMA Foundry, “Case Study: Adding New Value for Voice Call”,2024, Reference links for https://www.gsma.com/get-involved/gsma-foundry/gsma_resources/adding-new-value-to-voice-calls/

[87]	United Nations Sustainable Development Goals (https://sdgs.un.org/goals).

[88]	United Nations Global Digital Compact (https://www.un.org/global-digital-compact/sites/default/files/2024-09/Global%20Digital%20Compact%20-%20English_0.pdf).

[89]	Zhang Z, Wang S, Hong Y, et al. Distributed dynamic map fusion via federated learning for intelligent networked vehicles[J]. arXiv preprint arXiv:2103.03786, 2021

[90]	AECC: Operational Behavior of a High Definition Map Application – White Paper Version 1.0.0

[91]	 https://spectrum.ieee.org/boats-should-be-sleek-but-only-up-to-a-point

[92]	https://www.marketsandmarkets.com/Market-Reports/evtol-aircraft-market-28054110.html

[93]	https://www.fortunebusinessinsights.com/autonomous-vehicle-market-109045

[94] 	5G-ACIA White Paper, “Use Cases and Requirements for Integrated Sensing and Communication (ISAC) in Connected Industries and Automation”

[95]	ETSI GR ISC-001 V1.0.0 group report, “Integrated Sensing And Communications (ISAC); Use Cases and Deployment Scenarios”

[96]	3GPP TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR.

[97]	3GPP TR 22.865: “Study on satellite access - Phase 3”.

[98]                   	3GPP TR 22.836: "Study on Asset Tracking Use Cases".

[99]	CHINA AIRSPACE CLASSIFICATION AND PILOT UPDATE https://www.icao.int/APAC/Meetings/2024%20ATMSG12/IP09%20China%20Airspace%20Classification%20and%20Pilot%20Update.pdf#search=AIRSPACE%20CLASSIFICATION.

[100]	3GPP TS 38.171: "NR; Requirements for support of Assisted Global Navigation Satellite System (A-GNSS)".

[101] 	https://cloud.tencent.com/developer/article/1654264.

[102] 	ITU G.114: One-way transmission time.

[103]	S4aV200634: "Traces and Configurations for VR2, CG and AR2".

[104]	ITU-T FG NET-2030, "Representative use cases and key network requirements for Network 2030", 2020.

[105]	Ian F. Akyildiz, Hongzhi Guo, "Holographic-type communication: A new challenge for the next decade", ITU Journal on Future and Evolving Technologies, Volume 3 (2022), Issue 2, Pages 421-442.

[106]	X. Xu, Y. Pan, P. P. M. Y. Lwin and X. Liang, "3D holographic display and its data transmission requirement," 2011 International Conference on Information Photonics and Optical Communications, Jurong West, Singapore, 2011, pp. 1-4, doi: 10.1109/IPOC.2011.6122872.

[107]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "A new SNOW stream cipher called SNOW-V", IACR Transactions on Symmetric Cryptology, vol. 2019, no. 3, pp. 1-42, 2019. doi: 10.13154/tosc.v2019.i3.1-42.

[108]	P. Ekdahl, T. Johansson, A. Maximov, and J. Yang, "SNOW-Vi: an extreme performance variant of SNOW-V for lower grade CPUs ", WiSec '21, pp. 261 – 272, 2021. https://doi.org/10.1145/3448300.3467829

[109]	IEC 62056-6-1:2023 Object Identification System (OBIS).

[110]	IEC 62056-6-2:2023 COSEM interface classes.

[111]	IEC 62056-5-3:2023 DLMS/COSEM application layer.

[112]	Hexa-X-II, “Deliverable D1.4, 6G Value, Requirements, and Ecosystem”, April 2025.

[113] 	ITU-T Recommendation Y.3090 (02/22): "Digital twin network - Requirements and architecture" (https://www.itu.int/rec/T-REC-Y.3090-202202-I).

[114]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[115] 	Urban Air Mobility, https://www.airbus.com/en/innovation/low-carbon-aviation/urban-air-mobility, AIRBUS, accessed in May 2024.

[116] 	Urban Air Mobility (UAM) Concept of Operations, https://www.faa.gov/air-taxis/uam_blueprint, Federal Aviation Administration of U.S. Department of Transportation, Aug. 2023.

[117] 	Urban Air Mobility, https://rotorcraft.arc.nasa.gov/Research/Programs/UrbanAirMobility.html, U.S. NASA, 2024.

[118] 	NASA Urban Air Mobility (UAM) Reference Vehicles, https://sacd.larc.nasa.gov/uam-refs/, U.S. NASA, Jan. 2024

[119] 	Future Mobility, https://www.hyundai.com/worldwide/en/newsroom/detail/0000000093.html, Hyundai Motor Company, Jul. 2022.

[120]	LG Uplus, Near-Future Vertical Applications in Korea: K-UAM, https://nextgalliance.org/wp-content/uploads/2024/05/Near-future-Vertical-Applications-in-Korea-K-UAM_6GF_NGA_Joint_Workshop_JKim_final.pdf, Next G Alliance / Korea 6G Forum Joint Workshop

[121]	A. Baltaci, et. al., “A Survey of Wireless Networks for Future Aerial Communications (FACOM),” IEEE Communications Surveys & Tutorial, Vo. 23, No. 4, Q4 2021. https://ieeexplore.ieee.org/document/9508366.

[122]	3GPP TR 22.887: "Feasibility Study on satellite access – Phase 4".

[123]	Next G Alliance Report “6G Applications and Use Cases”: Use Case 4.6 – High-Speed Wireless Connection in Aerial Vehicle for Entertainment Service.

[124]	Topal, O. A., Schupke, D., Björnson, E., & Cavdar, C. ”Optimal Joint Access Point Placement and Resource Allocation for Indoor mmWave Communications”, ICC 2023, Rome, Italy.

[125]	Hofmann, S.; Duhovnikov, S.; Schupke, D., “Massive Data Transfer from and to Aircraft on Ground: Feasibility and Challenges,” IEEE Aerospace and Electronic Systems Magazine, vol. 36, iss. 5, May 2021.

[126] 	NASA. UTM: Air traffic management for low-altitude drones[EB/OL] (2015-10) (https://www.nasa.gov/sites/default/files/atoms/files/utm-factsheet-11-05-15.pdf)

[127] 	SESAR. U-space blueprint[EB/OL]. (2017-06-09). (https://www.sesarju.eu/sites/default/files/documents/reports/U-Space%20Blueprint%20brochure%20final.pdf)

[128] 	AOPA China. Regulations for the operation of light and small UAVs [EB/OL] (2018-04-19). (http://www.aopa.org.cn/usr/1/upload/file/20180419/15241267234030958.pdf)

[129] 	HIROYUKI U. UTM project in Japan[EB/OL]. (2017-06-26)[2025-01-20]. (https://gutma.org/montreal-2017/wp-ontent/uploads/sites/2/2017/07/UTM-Project-in-Japan_METI.pdf).

[130] 	G. Wagner and H. Choset, “A complete multirobot path planning algorithm with performance bounds,” Proc. In IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS), San Francisco, CA, USA, Sept. 2011. Online Link.

[131]	B. Hu, S. Xu, and Z. Cao, “Multi-Robot Path Planning for Each Robot with Several Jobs in a Single Trip,” IFAC-PapersOnline, Volume 53, Issue 5, 2020, Pages 279-284. Online Link.

[132]	S. Lin, A. Liu, J. Wang and X. Kong, “A Review of Path-Planning Approaches for Multiple Mobile Robots,” Section of Automation and Control Systems, MDPI Machines, 10(9), 773, Sept. 2022. Online Link.

[133]	A New Approach to Data Sharing - Open data sharing and collaboration for data, analytics and AI, DataBricks eBook 3rd Ed. Online Link.

[134]	K.-D. Lee, “A Smart Network of Service Robots: Technical Challenges and Design Considerations,” IEEE Communications Magazine, pp. 28-34, vol. 59, Iss. 8, Aug. 2021. Online Link.

[135]	Network-Enabled Robotic and Autonomous Systems, Next G Alliance, May 2023. Online Link.

[136]	3GPP TR 22.916: "Study on Network of Service Robots with Ambient Intelligence; Stage 1".

[137]	3GPP TS 23.548: “5G System Enhancements for Edge Computing; Stage 2”.

[138]	3GPP TS 22.228:  “Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1”.

[139]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”.

[140]	3GPP TS 23.501: “System architecture for the 5G System (5GS)”.

[141]	3GPP TS 23.503: “	Policy and charging control framework for the 5G System (5GS); Stage 2”.

[142]	3GPP TS 23.228: “IP Multimedia Subsystem (IMS); Stage 2”.

[143] 	Test Report from Utility Broadband Alliance “2024 UBBA Plugfest Testing Report” (https://www.ubba.com/ubba-resources/2024-ubba-plugfest-testing-report/).

[x1] 	3GPP TR 38.901: " Study on channel model for frequencies from 0.5 to 100 GHz ".

[x2] 	https://www.vodafone.co.uk/newscentre/features/trafford-centre-bolstered-vfuk-5g-and-4g/

[x3] 	3GPP TS 38.305: " Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[x4] 	3GPP TR 38.859: " Study on expanded and improved NR positioning".

## --- SECOND CHANGE (new text) ---

## W.x	Use case on 3D indoor positioning

### W.x.1	Description

Positioning is a very important service in human life. When shopping in a mall, we often rely on navigation to find a store or our friends. 3D Indoor positioning play the key role to realize the navigation service. 5G system trys to provide the accurate horizontal and vertical positioning service. However, the high-precision positioning can only be achieved under dense network deployments, which typically only apply for industrial deployment scenarios. For the consumer network scenarios, the network deployments mainly depend on the communication service, which makes it difficult to also provide the positioning service with acceptable accuracy.

For example, for typical indoor positioning evaluation in 5G, Intersite Distance (ISD) is assumed 20m in indoor office scenario [x1]. Hence, UE can receive signals from multiple cells and implement high-precision positioning with multiple base stations. However, the actual network deployments in real commercial scenario is often different. For example, the Trafford Centre in Manchester, UK, is a large indoor shopping centre covering 200,000 square metres, which is divided into 12 separate network sectors (180 radio antennas) [x2]. In such large cell coverage (ISD>100m), UE often receive signals only from a single cell. The cell planning is made primarily based on ensuring coverage for communication services throughout the mall, requiring coverage by only a single cell at any point.. Hence, high-precision positioning relied on multiple base stations is not suitable for this common scenario.

### W.x.2	Pre-conditions

Emily and Alice subscribe the 3D indoor positioning service from MNO A. With this service, they can locate each other with their phones under a crowded mall with large ISD.

### W.x.3	Service Flows

- Emily is shopping in a crowded mall. Her friend, Alice, is coming to meet her.

- Emily, whilst only in range of a single base station inside the mall, uses the 3D indoor positioning service to determine her position and sends her accurate location to Alice.

- Alice whilst only in range of a single base station inside the mall, also uses the 3D indoor positioning service to determine her position and, and navigate to Emily’s location.

- Finally, Alice easily finds Emily in the crowded mall.

### W.x.4	Post-conditions

Thanks to the indoor positioning service, Alice can find Emily in the crowded mall.

### W.x.5	Existing features partly or fully covering the use case functionality

In 5G the following features partly cover the use case functionality but these features do not consider ISD of over 20m:

- Multiple positioning methods are supported in TS 38.305 [x3], but evaluation of indoor positioning mainly considers ISD=20m [x4].

### W.x.6	Potential New Requirements needed to support the use case

[PR W.x.6-1] The 6G system shall be able to provide positioning services when the UE can only communicate with a single base station with the performance requirements reported in Table W.x.6-1.

Table W.x.6-1 Performance requirements for Horizontal and Vertical positioning service levels

| Accuracy (90 % confidence level) |  | Positioning service availability | Positioning service latency | 6G positioning service area |
| --- | --- | --- | --- | --- |
| Horizontal Accuracy | Vertical Accuracy |  |  |  |
| 3 m | 3 m | 95 % | 1 s | Indoor (note) - up to 30 km/h |
| NOTE: Indoor includes location inside buildings such as malls, offices, hospital, industrial buildings with inter-site distances greater than 20m where the UE can only communicate with a single base station. |  |  |  |  |

## --- END OF CHANGES ---