TSG-SA WG 1 Messaging SWG meeting	S1-021233

St. Albans, UK, June 17th – 18th 2002

Title:	Inclusion of MMS into  TR22.940

Source:			Siemens AG

Contact:	Jörg Swetina (joerg.swetina@siemens.com)

Introduction:

The present document proposes several changes to TR22.940 v 0.2.0  to include the MMS functionality in TR 22.940 for the Store-and-forward messaging.

Proposed changes:

The proposed changes affect clauses  6.4, 6.5, 6.7 and 8.

## 6.4	Store-and-forward messaging requirements

The requirements for Store-and-forward messaging are the same as for the Multimedia Messaging Service [2].

In addition to the requirements stated in [2] it is required, that IMS addressing is supported, i.e. it shall be possible to send multimedia messages to a public  identity of a subscriber.

## 6.5	Management requirements

Editors note: This chapter covers the requirements for management of IMS messaging e.g. how users can control send and received messages. Furthermore, the management requirements should also cover service provider related management aspects.

### 6.5.1	Requirements for Store-and-forward messaging

In addition to the requirements stated in [2] the following is required:

-	MMS Control by the operator

The MMS shall be able to support a request from the operator to enable/disable MM delivery and submission from and to each public identity of the user.

-	MMS Control by the user

The MMS shall be able to support a request from the user to enable/disable MM delivery and submission from and to each public identity of the user.

This requirement shall be supported at the application layer in the terminal, and will not be further elaborated.

-	Personalise multimedia messaging

The MMS shall be able to support a request by the user to manage the Service Preferences of his User Service Profile related to this MMS for each public identity of the user

-	Message forwarding

The MMS shall be able to support a request to forward multimedia messages or multimedia message elements without having to first download the MM to the terminal. 
Message forwarding shall be possible for each public identity of the user.

## 6.6	Message delivery requirements

## 6.7	Notification and acknowledgement requirements

Editors note: This chapter covers the requirements that are related to the notifications and acknowledgements informing the user about IMS messaging events.

### 6.7.1	Requirements for Store-and-forward messaging

In addition to the requirements stated in [2] the following is required:

If a subscriber has multiple public identities she shall be able to indicate those public identities, for which she wants to be notified about incoming multimedia messages.

## 6.8	Addressing requirements

# 7	Security

Editors note: This chapter covers the requirements to provide secure IMS messaging (e.g. authentication, privacy, protection against attacks…)

# 8	Charging

### 8.1	Requirements for Store-and-forward messaging

In addition to the requirements stated in [2] the following is required:

If a subscriber has multiple public identities it shall be possible to charge Store-and-forward messaging individually per public identity.