3GPP TSG-SA WG1 Meeting #60 	S1-124245

# Edinburgh, UK, 12th – 16th November 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.088 | CR |  | rev | - | Current version: | 12.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of the BOIC and BOICexHC barring for the SMS service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ORANGE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  |  |  |  |  |  | Date: |  | 2012-10-26 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | The BOICexHC for SMS can be activated by operators to mitigate the risks of SMS roaming frauds that consist of sending free SMS through open SMSC. In the current standard, the BOICexHC for SMS is consistent with the BOICexHC for voice, meaning that subscribers can only send SMS through an SMSC located in the home or the visited country. If roaming fraudsters are using an open SMSC located in the visited country, the current BOICexHC implementation will not bar the fraudulent SMS, allowing organized fraudsters to perpetrate frauds by roaming in countries where an open SMSC is available. The change request consists of updating the BOIC and BOICexHC for SMS in order that it also bars SMS sent by roaming subscribers through SMSC located in the country where the subscriber is presently located. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update BOIC and BOICexHC for SMS in order to forbid the use of SMSC which are located in the country where the roaming subscriber is presently located. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The BOICexHC for SMS will only allow partial mitigatation of SMS roaming frauds despite the fact that it could be an efficient countermeasure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** FIRST CHANGE *****

### 1.2.1	Description

The mobile subscriber may determine by subscription of a set of one or more unique barring programs what kind of outgoing calls shall be barred.

The following barring programs are defined:

1)	Barring of all outgoing calls;

2)	Barring of outgoing international calls;

3)	Barring of outgoing international calls except those directed to the home PLMN country.

The PLMN will ensure that only one of the barring programs is active per basic service group. The activation of one specific barring program will override an already active one (i.e. the old one will be permanently deactivated).

Description of contents of barring programs:

Barring of all outgoing calls:

With this barring program there are no outgoing set-up possibilities, except emergency calls.

In the case of SMS, this barring program shall not allow the mobile subscriber to send any SMS.

Barring of outgoing international calls:

Outgoing call set-up possibilities exist only to subscribers of the PLMN(s) and the fixed network(s) of the country where the mobile subscriber is presently located. So the present PLMN may be the home PLMN or a visited PLMN, respectively the fixed network may be that of the home PLMN country or that of a visited PLMN country.

NOTE 1:	This definition implies that mobile subscribers, subscribed to a PLMN in the country in which the served mobile subscriber roams, can be called irrespective where they roam. On the other hand, mobile subscribers, subscribed to any PLMN in another country than the one in which the served mobile subscriber roams, cannot be called even if they roam in the same PLMN area as the served mobile subscriber.

Barring of all outgoing international calls except those directed to the home PLMN country:

Outgoing call set-up possibilities exist only to subscribers of the PLMN(s) and the fixed network(s) of the country where the mobile subscriber is presently located or to mobile subscribers of the home PLMN country of the served mobile subscriber and to subscribers of the fixed network(s) in the home PLMN country. So the present PLMN may be the home PLMN or a visited PLMN, respectively the fixed network may be that of the home PLMN country or that of a visited PLMN country.

In the case of SMS, this barring program only allows the subscriber to send SMS through an SMSC located in the home country irrespective of whether the mobile subscriber is located in the home country or a foreign country.

NOTE 2:	This definition implies that mobile subscribers, subscribed to a PLMN in the country in which the served mobile subscriber roams or mobile subscribers subscribed to any PLMN in the same country as the served mobile subscriber home PLMN can be called irrespective where they roam. On the other hand, mobile subscribers, subscribed to any PLMN in another country than the one in which the served mobile subscriber roams and not belonging to the same country as the served mobile subscriber, cannot be called even if they roam in the same PLMN area as the served mobile subscriber or if they roam in the home PLMN country of the served mobile subscriber.

Barring of outgoing calls when roaming:

With this barring program there are no outgoing set-up possibilities, except emergency calls when the mobile subscriber is roaming outside its home PLMN.

In the case of SMS, this barring program only allows the subscriber to send SMS when being located in the home country and through an SMSC also located in the home country. Mobile subscribers roaming in a foreign country shall not be allowed to send any SMS irrespective of the location of the SMSC.

## *************** end of change ********************