3GPP TSG-SA WG1 Meeting #72	S1-154243

Anaheim, CA, 16-20 November 2015

Title:	Use case for highly reliable and high latency tolerant services

Agenda Item:	8.1 New SMARTER Use Case

Source:	Samsung

Contact:	Jayshree Bharatia (Jayshree.b@partner.samsung.com)

Abstract: This paper proposes use case relevant to highly reliable and high latency tolerant services.

## 5.x	Use case for Highly Reliable and High Latency Tolerant Services

### 5.x.1	Description

There are many use cases in TR 22.891 categorised under Cric scope for high reliability and low latency. In addition to these use cases, there are use cases which require high reliability and are tolerant to high latency. Some examples of these use cases include billing information aggregation, repository updates, search engine updates etc.

In 5G systems, there may be massive number of devices with diverse data, transmission and bandwidth requirements. The network may not always be capable to handle a massive burst of information and may have to prioritize the traffic. It may not be acceptable for certain data to be lost as the devices may not have the capability to regenerate the data. Therefore, network must be able to transfer this information reliably. In many instances, it is not vital to transmit this information immediately but the information must eventually reach its destination. Such scenarios can benefit from network communication that has high reliability and high latency.

For these services, latency is not an issue but the information must be communicated reliably. This is important for many use cases which rely on the eventual consistency paradigm.

Figure 5.x.2 1 – Example of Billing Information Aggregation

As shown in above example, usage information received from multiple MTC devices are sent to a Billing Aggregator Service through 3GPP 5G network. It is essential that usage information received from these devices is sent reliably. If the network cannot send this information immediately, the communication may be delayed. It is important that the Billing Aggregator Service eventually receives the billing packets sent by the MTC devices so that they can be included in a bill for that customer.

### 5.x.2	Potential Service Requirements

The 3GPP system shall be able to support high reliability for services which are tolerant of high latency.

### 5.x.3	Potential Operational Requirements