3GPP TSG-SA WG1 Meeting #92e	S1-204094

Online, 09 November – 18 November 2020

Title:	 FS_Ranging KPI analysis on Smart Home TV Control Use Case

Agenda Item:	7.10.1

Source:	Xiaomi

Contact:	Xiaowei Jiang (jiangxiaowei@xiaomi.com)

1. Background

In last SA1 meeting, the numbers in the KPI table of Smart Home TV Control were marked in square bracket due to concerns on the value. In this contribution, we try to analyze and justify these values, and suggest to remove the square brackets.

2. Discussion

The KPIs for Smart Home TV Control is defined in Table 5.2.6-1 in 22.855, as cited below:

Table 5.2.6-1 – KPIs for smart home TV control

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart device control | [10cm] up to 2 meter separation | [±2°] horizontal direction accuracy at 0.1 to 2 meter separation and AoA coverage of (-60°) to (+60°); [±2°] Elevation direction accuracy at 0.1 to 2 meter separation and AoA coverage of (-45°) to (+45°) | [99 %] | [10ms] | [10m] | IC/PC/OOC | LOS | Static/ Moving (<[10m/s]) | [50ms] | - | - |

Distance and direction accuracy.

Both distance error and direction error would contribute to the error of estimated point on TV that the remote points at. From user point of view, what matters is the angle between the line from estimated point to the TV remote and the line from the TV remote to the actual point that TV remote points at, which is considered as angle error. The smaller the angle error, the better. The tolerable angle error is related to the direction range of the TV remote, i.e. the angle (call it alpha in the following discussion) between the line from the left side of the TV to the TV remote and the line from the TV remote to the right side of TV. Taking a 50 inch TV (110cm * 62 cm) as an example,  the typical viewing distance is  3 meter.  The angle alpha is about 20 degree. The tolerable angle error is 10% of angle alpha, that is 2 degree, and it corresponds to 10 cm distance error (3 meter * tan(2)).

Availability:

99% availability will ensure a user friendly experience for smart home control.

Relative UE velocity:

Typically, the velocity of remote is smaller than 1m/s.

Service Latency & Ranging Interval:

Due to the fast TV remote movement, distance and direction need to be measured with very low service latency.  For 1m/s TV remote movement, the time for the mouse cursor on TV to move from left side to right side is on average 200ms for the case of 50 inch TV and 3 meter viewing distance. 10ms service latency would correspond to 5cm movement on TV. The distance and direction measurement result would also need to be updated frequently to trace the TV remote movement, 50ms ranging interval will provide a fine granularity for capturing TV remote movement.

Effective ranging distance:

Typically, the distance between TV and the watcher is no more than 10 meter.

3. Proposal

Proposal：Analysis is added to the description part. The square brackets in table 5.2.6-1 in 22.855 are removed. The ranging interval is changed from 50ms to 10ms.

**************** First Change ******************

## 5.2	Smart Home TV control

### 5.2.1	Description

Smart TVs, much like smartphones and smart home devices, offer internet connectivity and support for a range of apps. It typically provides an interactive interface, allowing users to stream on-demand music and videos, browse the internet, access to OTT content and online interactive media, etc.

In smart home scenario, smart TV is one of the most popular devices that people would heavily interact with.  Traditional TV remote is less convenient in control of smart TV, where the interactive interface includes many elements to choose and control.  Ranging capable remote can make the control of smart TV easier. An illustration of smart TV control using remote is shown in Fig 5.2.1-1. In the figure, a1 and b1 are the horizontal direction and elevation direction of remote from TV reference point as an observer. d is the distance between the TV reference point ant remote. a2 and b2 are the horizontal direction and elevation direction of TV reference point from remote as an observer. The TV reference point location in the TV coordinate system is known, the location of remote relative to the TV reference point can be calculated based a1, b1 and d. The position on TV where the remote is pointed can be calculated based on a2, b2 and the location of remote.

Figure 5.2.1-1 Illustration of smart TV control based on TV remote direction

Both distance error and direction error would contribute to the error of estimated point on TV that the remote points at. From user point of view, what matters is the angle between the line from estimated point to the TV remote and the line from the TV remote to the actual point that TV remote points at, which is considered as angle error. The smaller the angle error, the better. The tolerable angle error is related to the direction range of the TV remote, i.e. the angle (call it alpha in the following discussion) between the line from the left side of the TV to the TV remote and the line from the TV remote to the right side of TV. Taking a 50 inch TV (110cm * 62 cm) as an example,  the typical viewing distance is  3 meter.  The angle alpha is about 20 degree. We can consider 10% of angle alpha as tolerable angle error, that is 2 degree, and it corresponds to 10 cm distance error (3 meter * tan(2)).

Due to the fast TV remote movement, distance and direction need to be measured with very low service latency.  For 1m/s movement, the time for the mouse cursor on TV to move from left side to right side is on average 200ms for the case of 50 inch TV and 3 meter viewing distance. 10ms service latency would correspond to 5cm movement on TV. The distance and direction measurement result would also need to be updated frequently to trace the TV remote movement, 50ms ranging interval will provide a fine granularity for capturing TV remote movement.

************* End of First Change ***************

**************** Second Change ******************

Table 5.2.6-1 – KPIs for smart home TV control

| Ranging scenario | Ranging Accuracy (95 % confidence level) |  | Availability | Latency | Effective Ranging distance, Coverage, NLOS/LOS, Relative UE velocity, Ranging interval, Number of concurrent ranging operation for a UE, Number of concurrent ranging operation in an area |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Distance Accuracy | Direction Accuracy |  |  | Effective Ranging distance | Coverage | NLOS/LOS | Relative UE velocity | Ranging interval | Number of concurrent ranging operation for a UE | Number of concurrent ranging operation in an area |
| Distance based smart home TV control | 10cm up to 3 meter separation | ±2° horizontal direction accuracy at 0.1 to 3 meter separation and AoA coverage of (-60°) to (+60°); ±2° Elevation direction accuracy at 0.1 to 3 meter separation and AoA coverage of (-45°) to (+45°) | 99 % | 10ms | 10m | IC/PC/OOC | LOS | Static/ Moving (<1m/s) | [50ms] | - | - |

************* End of Second Change ***************