3GPP TSG-SA1 Meeting #99e	S1-222159r3

Electronic Meeting, 22 August – 1 September 2022	(revision of S1-22xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0018 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Railway staff Emergency Communication related use cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2022-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Railway staff Emergency Communication related use cases are introduced as new use cases. Refer to FRMCS Use cases - Reference MG-7900 (https://uic.org/rail-system/frmcs/). The Railway staff emergency communication behaviour is similar to the Railway Emergency communication with a few exceptions e.g., different priorities are configured, different FRMCS Users and different FRMCS Operator(s) are involved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New use cases related to Railway staff Emergency Communication are introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Railway needs are not fully covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * New change * * *

## 6.25	Railway staff emergency communication

### 6.25.1	Introduction

In this chapter the use cases related to Railway staff emergency communication are defined. The following use cases are defined:

-	Initiation of the Railway staff emergency alert

-	New entry to the Railway staff emergency alert

-	Changing of the Railway staff emergency alert

-	Merging of Railway staff emergency alerts

-	Leaving of the Railway staff emergency alert

-	Termination of the Railway staff emergency alert;

-	Initiation of Railway staff emergency voice communication

-	Termination of Railway staff emergency voice communication

-	Initiation of Data communication during Railway staff emergency Alert

-	Service interworking and service continuation with GSM-R

-	Interface to train borne recorder

### 6.25.2	General overview

The Railway staff emergency communication is very similar to the Railway emergency communication, but both serve two different purposes.

The Railway emergency communication is meant to address staff of all Railway Undertakings in a given area, whereas the Railway staff emergency communication addresses only the users of a specific Railway Undertaking in a given area.

Subsequently, where the Railway emergency communication has the Infrastructure Manager’s Controller a pivotal person in the call, in the Railway staff emergency communication, this is a Railway Undertaking’s Operator - usually someone that has a pivotal function in railways staff safety and has connections to railway police or other Railway Undertaking staff concerning safety and security.

In addition, the priority of the Railway staff emergency communication is lower than the one of the Railway emergency communication.

Furthermore, since the Railway staff emergency communication does not exist in GSM-R, no service interworking is required.

The following sections capture only use cases that show deltas compared to Railway emergency communication. For all other use cases (listed in 6.25.1), description, service flow and requirements would be the same as the corresponding Railway emergency communication use cases, except that they are applicable to Railway staff emergency communication.

### 6.25.3	Use case on Initiation of the railway staff emergency alert

#### 6.25.3.1	Description

An FRMCS User is able to initiate the Railway staff emergency alert. Based on a set of conditions the FRMCS System will determine which FRMCS Users that shall be informed about the emergency.(see 6.25.2 General overview). The targeted FRMCS User can be e.g., railway staff, train staff, a train driver, an external system, a Railway staff emergency operator or maintenance staff.

#### 6.25.3.2	Service flows

In any service flow, a mobile FRMCS User, e.g., a train driver, railway staff, train staff or maintenance staff of a specific organisation is not able to leave or terminate the railway emergency alert.

Operator initiated

Service flows are similar to Railway emergency communication, except that the operator is not an FRMCS Operator (i.e., independent from an administrator of the FRMCS System). Please refer to corresponding section in the current document.

External system initiated

Service flows are similar to Railway emergency communication, except that the external system is not owned and operated by an FRMCS Operator (i.e., independent from an administrator of the FRMCS System). Please refer to corresponding section in the current document.

Mobile FRMCS User initiated

Service flows are similar to Railway emergency communication, except that the Mobile FRMCS User initiator is from a specific organisation and can only communicate with staff from its own organisation. Criteria to determine the FRMCS Users to be included in the Railway staff emergency alert are similar to Railway emergency communication, limited to staff from initiator’s organisation e.g., location and full or subparts of the FRMCS Functional Identity of the FRMCS Users. An example of a full FRMCS Functional Identity of an FRMCS User is a specific driver on a specific train from a specific organisation. An example of a subpart of an FRMCS Functional Identity of an FRMCS User is all train drivers and train staff from the same organisation.

Please refer to corresponding section in the current document for relevant service flows.

#### 6.25.3.3	Potential requirements and gap analysis

Potential requirements are similar to Railway emergency communication, except the followings. Please refer to corresponding section in the current document for relevant potential requirements and gap analysis.

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.25.3-001] ([R-6.4.3-001] not applicable to Railway staff emergency) | Request to initiate a railway staff emergency alert may come from Operators external to FRMCS Operator, External Systems or mobile FRMCS Users. | A | 22.280 | Operators and mobile FRMCS Users covered by R-6.15.5.2-001. Not covered for external systems. |
| [R-6.25.3-002] ([R-6.4.3-002] not applicable to Railway staff emergency) | For requests from Operators, the FRMCS System shall be able to determine which FRMCS Users shall receive the railway staff emergency alert, based on the conditions supplied with the requests (e.g., a list/train number/area/track section/station). | A | 22.280 | Covered by R-6.15.5.2-014 (if MCX Service Ad Hoc Group mechanism can differentiate between railway staff emergency & railway emergency during initiation). |
| [R-6.25.3-003] ([R-6.4.3-003] not applicable to Railway staff emergency) | For requests from mobile FRMCS Users, based on the location information of the initiator and certain other conditions such as the FRMCS Functional Identity or subparts of the FRMCS Functional Identity of the FRMCS Users, track area, accuracy of the location, maximum track speed, station, shunting yard, speed and direction, the FRMCS System shall be able to determine which FRMCS Users shall receive the railway staff emergency alert. | A | 22.280 | Covered by R-6.15.5.2-014 (if MCX Service Ad Hoc Group mechanism can differentiate between railway staff emergency & railway emergency during initiation). |
| [R-6.25.3-004] ([R-6.4.3-007] not applicable to Railway staff emergency) | The FRMCS System shall be able to provide all applicable alerts to an Operator independent of whether involved in another alert or not to allow the Operators to switch between different alerts. | A | 22.280 | [R-5.4.2-007a] |
| [R-6.25.3-005] ([R-6.4.3-010] not applicable to Railway staff emergency) | The Railway emergency alert is able to pre-empt any other ongoing alert including Railway staff emergency alert | A | 22.280 | <Needs analysis of R-6.8.8 (Communication types based on priorities)> |

### 6.25.4	Use case on New entry to the railway staff emergency alert

#### 6.25.4.1	Description

Description is similar to Railway emergency communication, except that additional FRMCS Users can be e.g., railway staff, train staff, a train driver, a Railway staff emergency operator or maintenance staff. Please refer to corresponding section in the current document.

#### 6.25.4.2	Service flows

Service flows are similar to Railway emergency communication, except that the priority is lower than the Railway Emergency Alert. Please refer to corresponding section in the current document.

### 6.25.5	Use case on Changing of the railway staff emergency alert

#### 6.25.5.1	Description

Description is similar to Railway emergency communication, except that the authorised FRMCS User is an operator which is not an FRMCS Operator (i.e., independent from an administrator of the FRMCS System). Please refer to corresponding section in the current document.

#### 6.25.5.2	Service flows

Service flows are similar to Railway emergency communication, except that the priority is lower than the Railway Emergency Alert. Please refer to corresponding section in the current document.

### 6.25.6	Use case on Merging of Railway Staff Emergency Alerts

#### 6.25.6.1	Description

Description is similar to Railway emergency communication, except that the authorised FRMCS User is an operator which is not an FRMCS Operator (i.e., independent from an administrator of the FRMCS System). Please refer to corresponding section in the current document.

#### 6.25.6.2	Service flows

Service flows are similar to Railway emergency communication, except that the priority is lower than the Railway Emergency Alert. Please refer to corresponding section in the current document.

### 6.25.7	Use case on Initiation of railway staff emergency voice communication

#### 6.25.7.1	Service flows

Service flows are similar to Railway emergency voice communication, except that this is applicable to Railway staff emergency alert. Please refer to corresponding section in the current document.

In addition,

- the Railway staff emergency voice communication is not able to pre-empt Railway emergency communication.

- the Railway emergency voice communication is able to pre-empt any other ongoing voice communication including Railway staff emergency communication.

#### 6.25.7.2	Potential requirements and gap analysis

Potential requirements are similar to Railway emergency communication, except the followings. Please refer to corresponding section in the current document for relevant potential requirements and gap analysis.

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.25.7-001] | The Railway emergency voice communication is able to pre-empt any other ongoing voice communication including Railway staff emergency voice communication | A |  | [Needs analysis] |

### 6.25.8	Use case on Service interworking and service continuation with GSM-R

#### 6.25.8.1	Description

Service interworking of Railway staff emergency communication with GSM-R is not required.

* * * End of change * * *