3GPP TSG-SA WG1 Meeting #99e 	S1-222181

Electronic Meeting, 22 Aug. – 1 Sep., 2022

Title:	Definitions related to Ambient IoT

Agenda Item:	7.3

Source:	OPPO

Contact:	Weijie Xu xuweijie@oppo.com;

Yang Xu xuyang@oppo.com

Abstract: This document provides a Text Proposal for the definitions related to Ambient IoT.

# Proposal

It is proposed to add the following reference and use case description in TR 22.840.

************* Start of Changes ***************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	3GPP TS 22.261: "Service requirements for the 5G system".

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Ambient power-enabled Internet of Things (Ambient IoT) device: Ambient power-enabled Internet of Things device is an IoT device powered by energy harvesting, being either battery-less or with limited energy storage capability (e.g.,., using a capacitor). An Ambient IoT device has low complexity, small size and lower capabilities than existing 3GPP IoT devices, e.g., NB-IoT/eMTC devices. Ambient IoT devices can be maintenance free (e.g., without human intervention) and can have long life span (e.g., more than 10 years).

NOTE: The maximum instantaneous power consumption of an Ambient IoT device for communication is lower than device power consumption of NB-IoT/eMTC devices during active transmission/reception.

************* Next Change ***************