TSG-SA Working Group 2 (Architecture )	TSGS2 (99) 214

19 - 22 April 1999

Yokohama

# Source: 	Siemens

# Subject: 	Proposed changes to Draft TR on QoS, Sources of requested bearer parameters

# Purpose: 	for decision

## Reason for change

The Technical Report on QoS shall describe all QoS related issues of UMTS services. This paper identifies sources of UMTS bearer service QoS parameters for bearer setup or change. The description is proposed to be incorporated into the QoS Technical Report.

Change Request:

# Source of UMTS Bearer Service Parameters

UMTS bearer service parameters describe the service provided by the UMTS network to the user of the UMTS bearer service. A set of QoS parameters (QS profile) specifies this service. The UMTS bearer service parameters required at the establishment or modification of a service origin from:

- an explicit QoS parameter request from an application using the bearer service, the parameters are translated from UMTS external description or signalling means,

- a QoS profile in the UMTS subscription, describes also the upper limits for the provided service if the service user requests specific values,

- a QoS profile indicated by the UE configured by the user of the UE (e.g., by AT commands), or

- a default QoS profile for the network pre-configured by operator.

If no profile is requested by the UE or by the application on the UE the network derives requested parameters from the subscribed profile. If there is no unique interpretation of the subscribed profile the network uses a profile pre-configured by the operator.