TSG-SA WG2 		S2-99276

April 19th – 21st 1999

Yokohama, Japan

# Source: 	KPN

# Subject: 	Proposed LS: Response to LS on Service control (CAMEL) support for PS domain services

# Purpose	for decision

#### Liaison Statement

FROM: TSG SA Wg2

TO: TSG CN Wg3

SUBJECT: Response to LS on Service control (CAMEL) support for PS domain services

In its meeting in Yokohama, TSG SA Wg2 has adopted the following Working Assumption for multimedia Call Control and Session Management:

For multimedia services a multimedia CC/SM protocol could be run transparently via a PDP-context or a circuit-switched connection established using GSM SM/CC which would allow transparent handover and roaming between GSM and UMTS provided that GSM supports the QoS requirements..

This architecture decision may have an impact on the service control (CAMEL) support for Packet Switched domain services.

The main foreseen impact is that it is currently unclear how service control (CAMEL) would get information on what service content (e.g. voice, video, data, …) is transported within a PDP context. Without such information, service control within the Packet Switched domain would not have the same kind of scope as service control within the Circuit Switched domain where such information on service content (voice, fax, data) is currently available.

S2 will provide architecture guidance on how service control on top of Packet Switched domain services.