TSG-SA Working Group 2 meeting #5	TSGS2(99)194

Sophia-Antipolis May 25-28 1999

Source: 	Alcatel

Agenda Item: 	7 ( 23.30)

Title: 	Load sharing on Iu for IP domain user plane

Document for:	23.30 Change

# 1	Introduction

This document is a follow-up of Tdoc TSGS2(99)194 on the same subject (and with the same title) that was agreed in Yokohama (S2-99265 shows accepted changes in the way they shall be used for the update of chapter 7.2 of UMTS 23.20.).

Note also that R3 has agreed the same text within 25.214.

As the this feature is very much linked with Iu, it is proposed include the same text in 23.30

# 2	Proposal

Add in sect. 4.2 “UMTS Terrestrial Radio Access Network (UTRAN)” the following text extracted from agreed proposal 4 of Tdoc 265:

22) To ensure the necessary load sharing on the Iu_PS interface,

- When the CN requests the establishment of a Radio Access Bearer (associated with a PDP context) or at SRNS relocation for all Radio Access Bearers (associated with PDP contexts) of an UE, the CN specifies the IP address of the packet processing function allocated to this / each of these PDP context(s) in the CN.

- In the response to the CN request, the RNC specifies the IP address of the packet processing function allocated to this / each of these Radio Access Bearer(s) in the RNC.

When it sends downstream traffic in a RAB, the packet processing function in the CN sends the packet to the RNC IP @ received from the SRNC at RAB establishment or at SRNS relocation.

When it sends upstream traffic in this RAB, the packet processing function in the RNC sends the packet to the CN IP @ received from the CN at RAB establishment or at SRNS relocation.

# 3	References

[1]	UMTS 23.30, version 1.2.0